# 8	Performance requirement

This clause contains performance requirements for the physical channels specified in TS 36.211 [4]. The performance requirements for the UE in this clause are specified for the measurement channels specified in Annex A.3, the propagation conditions in Annex B and the downlink channels in Annex C.3.2.

NOTE:	For the requirements in the following sections, similar Release 8 and 9 requirements apply for time domain measurements restriction under colliding CRS.

## 8.1	General

### 8.1.1	Receiver antenna capability

The performance requirements are based on UE(s) that utilize one or more antenna receivers.

For all test cases, the SNR is defined as

where NRX denotes the number of receiver antenna connectors and the superscript receiver antenna connector j. The above SNR definition assumes that the REs are not precoded. The SNR definition does not account for any gain which can be associated to the precoding operation. The relative power of physical channels transmitted is defined in Table C.3.2-1. The SNR requirement applies for the UE categories and CA capabilities given for each test.

For enhanced performance requirements type A, the SINR is defined as

where NRX denotes the number of reciver antenna connectors and the superscript receiver antenna connector j. The above SINR definition assumes that the REs are not precoded. The SINR definition does not account for any gain which can be associated to the precoding operation. The relative power of physical channels transmitted is defined in Table C.3.2-1. The SINR requirement applies for the UE categories given for each test.

For the performance requirements specified in this clause, it is assumed that NRX=2 unless otherwise stated.

Table 8.1.1-1: Void

#### 8.1.1.1	Simultaneous unicast and MBMS operations

#### 8.1.1.2	Dual-antenna receiver capability in idle mode

### 8.1.2	Applicability of requirements

#### 8.1.2.1	Applicability of requirements for different channel bandwidths

In Clause 8 the test cases may be defined with different channel bandwidth to verify the same target FRC conditions with the same propagation conditions, correlation matrix and antenna configuration.

Test cases defined for 5MHz channel bandwidth that reference this clause are applicable to UEs that support only Band 31, 72, 73, 87 and/or 88.

#### 8.1.2.2	Definition of CA capability

The definition with respect to CA capabilities for 2CCs is given as in Table 8.1.2.2-1. The definition with respect to CA capabilities for 3CCs is given in Table 8.1.2.2-3.

Table 8.1.2.2-1: Definition of CA capability with 2DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA2_C | Intra-band contiguous CA |
| CA2_A2 | Inter-band CA (two bands) |
| CA2_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA2_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 2 DL CCs. CA2_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 2 DL CCs. CA2_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 2 DL CCs. |  |

The supported testable aggregated CA bandwidth combinations for 2CCs for each CA capability are listed in Table 8.1.2.2-2.

Table 8.1.2.2-2: Supported testable aggregated CA bandwidth combinations for different CA capability with 2DL CCs

| CA Capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA2_C | 5+5MHz, 5+10MHz, 5+15MHz, 10+10MHz, 20+20MHz | 20+20MHz, 15+20MHz | NA | NA |
| CA2_A2 | 10+10MHz, 20+5MHz, 10+15MHz, 10+20MHz, 15+20MHz, 20+20MHz | 20+20MHz | 10(FDD)+20(TDD)MHz, 15(FDD)+20(TDD)MHz, 20(FDD)+20(TDD)MHz | 20(FDD)+20(LAA)MHz, 10(FDD)+20(LAA)MHz, 15(FDD)+20(LAA)MHz, 20(TDD)+20(LAA)MHz |
| CA2_N2 | 5+10MHz, 10+10MHz, 10+20MHz, 20+20MHz | 20+20MHz | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

Table 8.1.2.2-3: Definition of CA capability with 3 DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA3_C | Intra-band contiguous CA |
| CA3_A2 | Inter-band CA (two bands) |
| CA3_A3 | Inter-band CA (three bands) |
| CA3_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA3_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 3 DL CCs. CA3_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 3 DL CCs. CA3_A3 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2a for 3 DL CCs. CA3_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 3 DL CCs. |  |

The supported testable largest aggregated CA bandwidth combinations for 3CCs for each CA capability are listed in Table 8.1.2.2-4.

Table 8.1.2.2-4: Supported largest aggregated CA bandwidth combinations for different CA capability with 3 CCs

| CA capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA3_C | NA | 20+20+20MHz | NA | NA |
| CA3_A2 | 5+5+10MHz, 5+10+10MHz, 5+5+20MHz, 10+10+10MHz, 5+10+20MHz, 5+15+20MHz, 10+10+20MHz, 10+20+20MHz, 15+20+20MHz, 20+20+20MHz | 15+20+20MHz, 20+20+20MHz | 10(FDD)+20(TDD)+20(TDD)MHz 15(FDD)+20(TDD)+20(TDD)MHz, 20(FDD)+20(TDD)+20(TDD)MHz | 10(FDD)+ 2×20(LAA)MHz, 15(FDD)+ 2×20(LAA)MHz, 20(FDD)+ 2×20(LAA)MHz, 20(TDD)+ 2×20(LAA)MHz |
| CA3_A3 | 10+10+20MHz, 10+15+15MHz, 10+15+20MHz, 10+20+20MHz, 15+15+20MHz, 15+20+20MHz, 20+20+20MHz | NA | 2×20(FDD)+20(TDD)MHz, 20(FDD)+ 2×20(TDD)MHz, 20(FDD)+15(FDD)+20(TDD)MHz, 20(FDD)+10(FDD)+20(TDD)MHz, 2×15(FDD)+20(TDD)MHz | NA |
| CA3_N2 | NA | 20+20+20MHz | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

Table 8.1.2.2-5: Definition of CA capability with 4 DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA4_C | Intra-band contiguous CA |
| CA4_A2 | Inter-band CA (two bands) |
| CA4_A3 | Inter-band CA (three bands) |
| CA4_A4 | Inter-band CA (four bands) |
| CA4_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA4_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 3 DL CCs. CA4_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 3 DL CCs. CA4_A3 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2a for 3 DL CCs. CA4_A4 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2b for 4 DL CCs CA4_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 3 DL CCs. |  |

The supported testable largest aggregated CA bandwidth combinations for 4CCs for each CA capability are listed in Table 8.1.2.2-6.

Table 8.1.2.2-6: Supported largest aggregated CA bandwidth combinations for different CA capability with 4 CCs

| CA capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA4_C | NA | 20+20+20+20MHz | NA | NA |
| CA4_A2 | 24x20MHz 10+3x20MHz 2x10+2x20MHz 210+5+2x20MHz 2x5+2x20MHz 3x10+20MHz 4x10MHz | 20+20+20+20MHz 15+20+20+20MHz | 20(FDD)+3x20(TDD)MHz 2x20(FDD)+2x20(TDD)MHz 10(FDD)+3x20(TDD)MHz 2x10(FDD)+2x20(TDD)MHz 2x10(FDD)+20+15(TDD)MHz | 10(FDD)+ 3×20(LAA)MHz, 15(FDD)+ 3×20(LAA)MHz, 20(FDD)+ 3×20(LAA)MHz, 20(TDD)+ 3×20(LAA)MHz |
| CA4_A3 | 24x20MHz 15+3x20MHz 210+3x20MHz 2x10+2x20MHz 5+10+2x20MHz 2x5+2x20MHz 3x10+20MHz 25+2x10+20MHz 2x5+10+20MHz | NA | 20(FDD)+3x20(TDD)MHz 2×20(FDD)+2×20(TDD)MHz, 3x20(FDD)+20(TDD)MHz 20(FDD)+15(FDD)+2×20(TDD)MHz, 2×15(FDD)+2x20(TDD)MHz 10(FDD)+20(FDD)+2x20(TDD)MHz 10(FDD)+3x20(TDD)MHz 10(FDD)+15(FDD)+2x20(TDD)MHz 2x10(FDD)+2x20(TDD)MHz | NA |
| CA4_A4 | 24x20MHz 15+3x20MHz 210+3x20MHz 2x15+2x20MHz 10+15+2x20MHz 22x10+2x20MHz | NA | 3x20(FDD)+20(TDD)MHz 2×20(FDD)+15(FDD)+20(TDD)MHz, 2×15(FDD)+20(FDD)+20(TDD)MHz 2×20(FDD)+10(FDD)+20(TDD)MHz 10(FDD)+20(FDD)+2x20(TDD)MHz 10(FDD)+15(FDD)+20(FDD)+20(TDD)MHz | NA |
| CA4_N2 | NA | 20+20+20+20MHz | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

Table 8.1.2.2-7: Definition of CA capability with 5 DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA5_C | Intra-band contiguous CA |
| CA5_A2 | Inter-band CA (two bands) |
| CA5_A3 | Inter-band CA (three bands) |
| CA5_A4 | Inter-band CA (four bands) |
| CA5_A5 | Inter-band CA (five bands) |
| CA5_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA5_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 5 DL CCs. CA5_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 5 DL CCs. CA5_A3 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2a for 5 DL CCs. CA5_A4 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2b for 5 DL CCs CA5_A5 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2c for 5 DL CCs CA5_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 5 DL CCs. |  |

The supported testable largest aggregated CA bandwidth combinations for 5CCs for each CA capability are listed in Table 8.1.2.2-8.

Table 8.1.2.2-8: Supported largest aggregated CA bandwidth combinations for different CA capability with 5 CCs

| CA capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA5_C | NA | 5×20MHz | NA | NA |
| CA5_A2 | 5×20MHz 3x20+2x10MHz 20+4x10MHz | 5×20MHz 15+4X20MHz | 2x20(FDD)+3x20(TDD) 20(FDD)+4x20(TDD)MHz 2x10(FDD)+3x20(TDD)MH | 10(FDD)+ 4×20(LAA)MHz, 15(FDD)+ 4×20(LAA)MHz, 20(FDD)+ 4×20(LAA)MHz, 20(TDD)+ 4×20(LAA)MHz |
| CA5_A3 | 5×20MHz 10+4x20MHz 2x10+3x20MHz 5+10+3x20MHz 3x10+2x20MHz 4x10+20MHz | NA | 3x20(FDD)+2x20(TDD)MHz 20(FDD)+4x20(TDD)MHz 2x20(FDD)+3x20(TDD)MHz 10(FDD)+4x20(TDD)MHz | NA |
| CA5_A4 | 5×20MHz 15+4x20MHz 10+4x20MHz 2x10+3x20MHz 3x10+2x20MHz | NA | 3x20(FDD)+2x20(TDD)MHz 15+2×20(FDD)+2×20(TDD)MHz 0+2×20(FDD)+2×20(TDD)MHz2×15+20(FDD)+2×20(TDD)MHz 10+15+20(FDD)+2x20(TDD)MHz 10+20(FDD)+3x20MHz | NA |
| CA5_A5 | 5×20MHz | NA | 4x20(FDD)+20(TDD)MHz | NA |
| CA5_N2 | NA | 5×20MHz | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

Table 8.1.2.2-9: Definition of CA capability with 6 DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA6_C | Intra-band contiguous CA |
| CA6_A2 | Inter-band CA (two bands) |
| CA6_A3 | Inter-band CA (three bands) |
| CA6_A4 | Inter-band CA (four bands) |
| CA6_A5 | Inter-band CA (five bands) |
| CA6_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA6_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 6 DL CCs. CA5_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 6 DL CCs. CA5_A3 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2a for 6 DL CCs. CA5_A4 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2b for 6 DL CCs CA5_A5 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2c for 6 DL CCs CA5_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 6 DL CCs. |  |

The supported testable largest aggregated CA bandwidth combinations for 6CCs for each CA capability are listed in Table 8.1.2.2-10.

Table 8.1.2.2-10: Supported largest aggregated CA bandwidth combinations for different CA capability with 6 CCs

| CA capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA6_C | NA | NA | NA | NA |
| CA6_A2 | NA | 6×20MHz | 20(FDD)+5x20(TDD)MHz 2x20(FDD)+4x20(TDD)MHz | 10(FDD)+ 5×20(LAA)MHz, 15(FDD)+ 5×20(LAA)MHz, 20(FDD)+ 5×20(LAA)MHz, 20(TDD)+ 5×20(LAA)MHz |
| CA6_A3 | NA | NA | 20(FDD)+5x20(TDD)MHz 2x20(FDD)+4x20(TDD)MHz | NA |
| CA6_A4 | 6×20MHz | NA | 20(FDD)+5x20(TDD)MHz 2x20(FDD)+4x20(TDD)MHz | NA |
| CA6_A5 | NA | NA | NA | NA |
| CA6_N2 | NA | NA | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

Table 8.1.2.2-11: Definition of CA capability with 7 DL CCs

| CA Capability | CA Capability Description |
| --- | --- |
| CA7_C | Intra-band contiguous CA |
| CA7_A2 | Inter-band CA (two bands) |
| CA7_A3 | Inter-band CA (three bands) |
| CA7_A4 | Inter-band CA (four bands) |
| CA7_A5 | Inter-band CA (five bands) |
| CA7_N2 | Intra-band non-contiguous CA (with two sub-blocks) |
| NOTE 1: CA6_C corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-1 for 7 DL CCs. CA5_A2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-2 for 7 DL CCs. CA5_A3 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2a for 7 DL CCs. CA5_A4 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2b for 7 DL CCs CA5_A5 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in and Table 5.6A.1-2c for 7 DL CCs CA5_N2 corresponds to E-UTRA CA configurations and bandwidth combination sets defined in Table 5.6A.1-3 for 7 DL CCs. |  |

The supported testable largest aggregated CA bandwidth combinations for 7CCs for each CA capability are listed in Table 8.1.2.2-12.

Table 8.1.2.2-12: Supported largest aggregated CA bandwidth combinations for different CA capability with 7 CCs

| CA capability | Bandwidth combination for FDD CA | Bandwidth combination for TDD CA | Bandwidth combination for TDD-FDD CA | Bandwidth combination for CA with LAA SCell(s) |
| --- | --- | --- | --- | --- |
| CA7_C | NA | NA | NA | NA |
| CA7_A2 | NA | 7×20MHz | 20(FDD)+6x20(TDD)MHz 2x20(FDD)+5x20(TDD)MHz | 10(FDD)+ 6×20(LAA)MHz, 15(FDD)+ 6×20(LAA)MHz, 20(FDD)+ 6×20(LAA)MHz, 20(TDD)+ 6×20(LAA)MHz |
| CA7_A3 | NA | NA | 20(FDD)+6x20(TDD)MHz 2x20(FDD)+5x20(TDD)MHz | NA |
| CA7_A4 | 7×20MHz | NA | 20(FDD)+6x20(TDD)MHz 2x20(FDD)+5x20(TDD)MHz | NA |
| CA7_A5 | NA | NA | NA | NA |
| CA7_N2 | NA | NA | NA | NA |
| NOTE 1: This table is only for information and applicability and test rules of CA performance requirements are specified in 8.1.2.3 and 9.1.1.2. |  |  |  |  |

For test cases with more than one component carrier, "Fraction of Maximum Throughput" in the performance requirement refers to the ratio of the sum of throughput values of all component carriers to the sum of the nominal maximum throughput values of all component carriers, unless otherwise stated.

#### 8.1.2.2A	Definition of dual connectivity capability

The definition with respect to dual connectivity capabilities for configurations with 2CCs is given as in Table 8.1.2.2A-1. The definition with respect to dual connectivity capabilities for configurations with 3CCs is given as in Table 8.1.2.2A-3.

Table 8.1.2.2A-1: Definition of dual connectivity capability with 2DL CCs

| Dual connectivity Capability | Dual connectivity capability Description |
| --- | --- |
| DC_A_2 | Inter-band dual connecitivty (two bands) |
| NOTE 1: DC_A_2 corresponds to E-UTRA dual connectivity configurations and bandwidth combination sets defined for inter-band dual connecitivty (two bands) as specified in 5.6C . |  |

The supported testable dual connectivity bandwidth combinations for 2CCs for each dual connectivity capability are listed in Table 8.1.2.2A-2.

Table 8.1.2.2A-2: Supported testable dual connectivity bandwidth combinations for different dual connectivitys capability with 2DL CCs

| Dual connectivity capability | Bandwidth combination for FDD dual connectivity | Bandwidth combination for TDD dual connectivity | Bandwidth combination for TDD-FDD dual connectivity |
| --- | --- | --- | --- |
| DC_A_2 | 10+10MHz, 10+20MHz, 15+15MHz,15+20MHz, 20+20MHz,15+5MHz | 20+20MHz | 20(FDD)+20(TDD)MHz |
| NOTE 1: This table is only for information and applicability and test rules of dual connectivity performance requirements are specified in 8.1.2.3A |  |  |  |

Table 8.1.2.2A-3: Definition of dual connectivity capability with 3DL CCs

| Dual connectivity Capability | Dual connectivity capability Description |
| --- | --- |
| DC_A_3 | Inter-band dual connecitivty (three bands) |
| NOTE 1: DC_A_3 corresponds to E-UTRA dual connectivity configurations and bandwidth combination sets defined for inter-band dual connecitivty (three bands) as specified in 5.6C. |  |

The supported testable dual connectivity bandwidth combinations for 3CCs for each dual connectivity capability are listed in Table 8.1.2.2A-4.

Table 8.1.2.2A-4: Supported testable dual connectivity bandwidth combinations for different dual connectivitys capability with 3DL CCs

| Dual connectivity capability | Bandwidth combination for FDD dual connectivity | Bandwidth combination for TDD dual connectivity |
| --- | --- | --- |
| DC_A_3 | 20+20+15MHz, 20+15+15MHz | NA |
| NOTE 1: This table is only for information and applicability and test rules of dual connectivity performance requirements are specified in 8.1.2.3A |  |  |

#### 8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA UE demodulation tests in Clause 8 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL CCs in Table 8.1.2.3-1 and 3 or more DL CCs in Table 8.2.2.3-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 8.1.2.3-1: Applicability and test rules for CA UE demodulation tests with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 8.2.1.1.1, 8.2.1.4.3 | Any one of the supported CA capabilities | Any one of the supported FDD CA configurations | 10+10 MHz, 20+20 MHz, 5+5 MHz, 10MHz+5MHz, 15MHz+5MHz |
| CA tests with 2CCs in Clause 8.2.1.3.1 | Each supported CA capability | Any one of the supported FDD CA configurations in each CA capability | 10+10 MHz, 20+20 MHz, 5+5 MHz, 10MHz+5MHz, other combinations |
| CA tests with 2CCs in Clause 8.2.1.3.1A, 8.7.1, 8.2.1.9.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.1.7.1 | CA_C | Supported FDD intra-band contiguous CA configurations covering the lowest and highest operating bands | Largest aggregated CA bandwidth combinations |
| CA tests with 2CCs in Clause 8.2.2.1.1, 8.2.2.4.3, 8.2.2.9.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.2.3.1 | Each supported CA capability | Any one of the supported TDD CA configurations in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.2.3.1A, 8.7.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in 8.2.2.7.1 | CA_C | Supported TDD intra-band contiguous CA configurations covering the lowest and highest operating bands | Largest aggregated CA bandwidth combinations |
| CA tests with 2CCs in Clause 8.2.1.8.1 | CA_N | CA_3A-3A defined in Table 5.6A.1-3 | 10+10 MHz |
| CA tests with 2CCs in Clause 8.2.2.8.1 | CA2_C | CA_41C defined in Table 5.6A.1-1 | 20+20 MHz |
| CA tests in Clause 8.2.1.10.1 (NOTE 4) | CA2_C | CA_1C and CA_3C defined in Table 5.6A.1-1 | Largest aggregated CA bandwidth combinations |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. NOTE 4: For FDD carriers with minimum channel spacing, UE only supports the carriers in the following scenarios: Inter-band CA with ≥3 CCs, where Band 1 is with 1 CC and normal spacing (PCell) and Band 2 is with 2 intra-band contiguous CCs and minimum channel spacing (SCell), or Band 1 is configured with 1CC and normal spacing, and UE accesses the CC on Band 1, and then handed over to Band 2 with 2 intra-band contiguous CCs and minimum channel spacing. |  |  |  |

Table 8.1.2.3-2: Applicability and test rules for CA UE demodulation tests with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 3 or more CCs in Clause 8.2.1.1.1, 8.2.1.4.3, 8.7.1, 8.2.1.9.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3 or more CCs in Clause 8.2.1.3.1 | Each supported CA capability | Any one of the supported FDD CA configurations in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3 or more CCs in Clause 8.2.2.1.1, 8.2.2.4.3, 8.7.2, 8.2.2.9.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3 or more CCs in Clause 8.2.2.3.1 | Each supported CA capability | Any one of the supported TDD CA configurations in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs in Clause 8.2.2.8.1 | CA3_C | CA_41D defined in Table 5.6A.1-1 | 20+20+20 MHz |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests |  |  |  |

#### 8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set

The performance requirement for dual connectivity UE demodulation tests in Clause 8 are defined independent of dual connectivity configurations and bandwidth combination sets specified in Clause 5.6C.1. For UEs supporting difrerent dual connectivity configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for the configurations with 2CCs in Table 8.1.2.3A-1 and 3 DL CCs in Table 8.1.2.3A-2. For simplicity, dual connectivity configuration below refers to combination of dual connectivity configuration and bandwidth set.

Both CA performance requirements and dual connectivity performance requirements are applied for dual connectivity capable UE.

Table 8.1.2.3A-1: Applicability and test rules for dual connectivity UE demodulation tests with 2DL CCs

| Tests | Dual connectivity capability where the tests apply | Dual connectivity configuration from the selected CA capbility where the tests apply | Dual connectivity Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| Dual connectivity test with 2CCs in Clause 8.2.1.4.3A, 8.7.6 | Any one of the supported dual connectivity capabilities with largest aggregated dual connectivity bandwidth combination | Any one of the supported FDD dual connectvity configurations with the largest aggregated dual connectivity bandwidth combimation | Largest dual connectivity aggregated bandwidth combination |
| Dual connectivity test with 2CCs in Clause 8.2.2.4.3A, 8.7.7 | Any one of the supported dual connectivity capabilities with largest aggregated dual connectivity bandwidth combination | Any one of the supported TDD dual connectvity configurations with the largest aggregated dual connectivity bandwidth combination | Largest dual connectivity aggregated bandwidth combination |
| Dual connectivity test with TDD FDD 2CCs in Clause 8.2.3.4, 8.7.8 | Any one of the supported dual connectivity capabilities with largest aggregated dual connectivity bandwidth combination | Any one of the supported TDD FDD dual connectvity configurations with the largest aggregated dual connectivity bandwidth combination | Largest dual connectivity aggregated bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected DC or CA configuration is 1. |  |  |  |

Table 8.1.2.3A-2: Applicability and test rules for dual connectivity UE demodulation tests with 3DL CCs

| Tests | Dual connectivity capability where the tests apply | Dual connectivity configuration from the selected CA capbility where the tests apply | Dual connectivity Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| Dual connectivity test with 3CCs in Clause 8.2.1.4.3A, 8.7.6 | Any one of the supported dual connectivity capabilities with largest aggregated dual connectivity bandwidth combination | Any one of the supported FDD dual connectvity configurations with the largest aggregated dual connectivity bandwidth combimation | Largest dual connectivity aggregated bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected DC or CA configuration is 1. |  |  |  |

#### 8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets

The performance requirement for TDD-FDD CA UE demodulation tests in Clause 8 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL TDD-FDD CA in Table 8.1.2.3B-1 and in Table 8.1.2.3B-2 for 3 or more DL TDD-FDD CA. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 8.1.2.3B-1: Applicability and test rules for CA UE demodulation tests for TDD-FDD CA with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 8.2.3.1.1, 8.2.3.2.1A, 8.2.3.3.1, 8.7.5.1, 8.2.3.5.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.3.2.1 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with FDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.3.1.2, 8.2.3.2.2A, 8.2.3.3.2, 8.7.5.2, 8.2.3.5.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.3.2.2 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with TDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. |  |  |  |

Table 8.1.2.3B-2: Applicability and test rules for CA UE demodulation tests for TDD-FDD CA with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 3CCs, 4CCs, 5CCs in Clause 8.2.3.1.1, 8.2.3.2.1A, 8.2.3.3.1, 8.7.5.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs in Clause 8.2.3.2.1 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with FDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs in Clause 8.2.3.1.2, 8.2.3.2.2A, 8.2.3.3.2, 8.7.5.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs in Clause 8.2.3.2.2 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with TDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. |  |  |  |

#### 8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs

For FDD single carrier or CA, UE is required to fulfill SDR tests specified in section 8.7.9. For TDD single carrier or CA, UE is required to fulfill SDR tests specified in section 8.7.10. For TDD-FDD CA, UE is required to fulfill SDR test in section 8.7.11.

For FDD DC, UE is required to fulfill SDR tests specified in section 8.7.13. For TDD DC, UE is required to fulfill SDR tests specified in section 8.7.14. For TDD-FDD DC, UE is required to fulfill SDR test in section 8.7.15.

For single carrier or CA SDR tests, CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

-	The procedure applies also for single carrier using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

For DC SDR tests, DC configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one DC bandwidth combination among all supported DC configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all DC bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {DC configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

For UEs with no supported 4Rx RF bands the applicability rule in 8.1.2.3C is not applied for SDR tests.

#### 8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets

The performance requirement for CA with LAA SCell(s) UE demodulation tests in Clause 8 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA with LAA SCell(s) configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL CA with LAA SCell(s) in Table 8.1.2.3D-1 and in Table 8.1.2.3D-2 for 3 or more DL CA with LAA SCell(s). For simplicity, CA configuration below refers to combination of CA with LAA SCell(s) configuration and bandwidth combination set.

Table 8.1.2.3D-1: Applicability and test rules for CA UE demodulation tests for CA with LAA SCell(s) with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 8.2.4.1.1, 8.3.3.1.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported CA with LAA SCell(s) configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.4.1.2, 8.3.3.1.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported CA with LAA SCell(s) configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. |  |  |  |

Table 8.1.2.3D-2: Applicability and test rules for CA UE demodulation tests for CA with LAA SCell(s) with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 8.2.4.1.1, 8.3.3.1.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported CA with LAA SCell(s) configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.4.1.2, 8.3.3.1.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported CA with LAA SCell(s) configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. |  |  |  |

#### 8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs

UE with support of 8Rx RF bands is required to fulfill the specified SDR tests for 8Rx test in section 8.7.17.

For single carrier or CA SDR tests, CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

- Select the set(s) of {CA configuration, bandwidth combination, MIMO layer} among all the supported CA configurations that leads to the largest equivalent aggregated bandwidth which does not cause the transport block bits within a TTI to exceed the capability of the category of UE under test when the defined reference channel applies on each CC. The equivalent aggregated bandwidth is defined as

where  is the number of CCs,  Ri∈2,4, 8and Bi∈5,10,15,20 are MIMO layer and bandwidth of CC , and  for and  for

-	The procedure applies also for single carrier using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

#### 8.1.2.4	Test coverage for different number of component carriers

For FDD tests specified in 8.2.1.1.1, 8.2.1.3.1, 8.2.1.4.3, and 8.7.1, if corresponding CA tests are tested, the test coverage can be considered fulfilled without executing single carrier tests.

For TDD tests specified in 8.2.2.1.1, 8.2.2.3.1, 8.2.2.4.3, and 8.7.2, if corresponding CA tests are tested, the test coverage can be considered fulfilled without executing single carrier tests.

For TDD FDD tests specified in 8.2.3.1, 8.2.3.2, 8.2.3.3, 8.2.3.5, and 8.7.5, if corresponding TDD FDD CA tests are tested, the test coverage can be considered fulfilled without executing both FDD and TDD single carrier tests.

For FDD CA tests specified in 8.2.1.1.1, 8.2.1.4.3, 8.7.1, 8.13.1.1.1 and 8.13.1.2.1, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For FDD CA tests specified in 8.2.1.3.1, for each supported CA capability, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For TDD CA tests specified in 8.2.2.1.1, 8.2.2.4.3, 8.7.2, 8.13.2.1.1 and 8.13.2.2.1, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For TDD CA tests specified in 8.2.2.3.1, for each supported CA capability, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For TDD FDD CA tests specified in 8.2.3.1, 8.2.3.3, 8.7.5, 8.13.3.1 and 8.13.3.2, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the TDD FDD CA tests with less than the largest number of CCs supported by the UE.

For TDD FDD CA tests specified in 8.2.3.2 and 8.2.3.5, for each supported CA capability, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the TDD FDD CA tests with less than the largest number of CCs supported by the UE.

For FDD CA power imbalance tests specified in 8.2.1.7.1, if they are are tested with FDD intra-band contiguous CA configurations with 2 DL CCs, the test coverage can be considered fulfilled with FDD intra-band contiguous CA configurations with 3 or more DL CCs supported by the UE.

For TDD CA power imbalance tests specified in 8.2.2.7.1, if they are are tested with TDD intra-band contiguous CA configurations with 2 DL CCs, the test coverage can be considered fulfilled with TDD intra-band contiguous CA configurations with 3 or more DL CCs supported by the UE.

For FDD DC tests specified in 8.2.1.4.3A, 8.7.6 and 8.13.1.1.2, among all supported DC capabilities, if corresponding DC tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the DC tests with less than the largest number of CCs supported by the UE.

For TDD DC tests specified in 8.2.2.4.3A, 8.7.7 and 8.13.2.1.2, among all supported DC capabilities, if corresponding DC tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the DC tests with less than the largest number of CCs supported by the UE.

For TDD FDD DC tests specified in 8.2.3.4, 8.7.8 and 8.13.3.5, among all supported DC capabilities, if corresponding DC tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the DC tests with less than the largest number of CCs supported by the UE.

For LAA SCell(s) with FDD PCell tests specified in 8.2.4.1.1 and 8.3.3.1.1, for each supported CA capability, if corresponding CA with LAA SCell(s) tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA with LAA SCell(s) tests with less than the largest number of CCs supported by the UE.

For LAA SCell(s) with TDD PCell tests specified in 8.2.4.1.2 and 8.3.3.1.2, for each supported CA capability, if corresponding CA with LAA SCell(s) tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA with LAA SCell(s) tests with less than the largest number of CCs supported by the UE.

#### 8.1.2.5	Applicability of performance requirements for Type B receiver

For TM10 capable UE, if corresponding tests specified in 8.3.1.1F, 8.3.2.1G, 9.3.8.3 are tested, the test coverage can be considered fulfilled without executing the tests specified in 8.3.1.1C, 8.3.2.1D, 9.3.8.2. For a UE which does not have TM10 capability, the tests specified in sections 8.3.1.1C, 8.3.2.1D, 9.3.8.2 should be used.

#### 8.1.2.6	Applicability of performance requirements for 4Rx capable UEs

For 4Rx capable UEs, the 2Rx supported RF bands and 4Rx supported RF bands are up to UE’s declaration.

##### 8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx

For 4Rx capable UEs all single carrier tests specified in 8.2 to 8.8 with 2Rx are tested on any of the 2 Rx supported RF bands by connecting 2 out of the 4Rx with data source from system simulator, and the other 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied.

For 4Rx capable UEs without any 2Rx RF bands, all single carrier tests specified in 8.2 to 8.8 with 2Rx are tested on any of the 4Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. Figure 8.1.2.6.1-1 shows an example of antenna connection for 4Rx UE in any one 4Rx supported RF band to perform a 2Rx performance test with antenna configuration as 2x2 without interference for information. The SNR requirements should be applied with 1.5 dB less than the number specified with 2Rx for test configuration with CRS-based TM and with 1.5 dB less than the number specified with 2Rx for test configuration with DMRS-based TM.

Figure 8.1.2.6.1-1 Antenna connection example for 2Rx tests with antenna configuration as 2x2 without interference (informative)

For 4Rx capable UEs without any 2Rx supported RF bands, for all single carrier tests listed in Table 8.1.2.6.1-0 specified from 8.2 to 8.8 with 2Rx can be skipped.

Table 8.1.2.6.1-0: Requirement lists for 4Rx capable UEs

Requirement lists

Enhanced downlink control channel performance requirements type A

Enhanced downlink control channel performance requirements type B

Enhanced performance requirements type B

Enhanced performance requirements type C

Requirements with demodulation subframe overlaps with aggressor cell ABS

Requirements with demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured

Requirements with CRS assistance information configured

For 4Rx capable UEs, if corresponding tests listed from the 4Rx test lists from Table 8.1.2.6.1-1 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed from the 2Rx test lists from Table 8.1.2.6.1-1.

Table 8.1.2.6.1-1: Applicability rules for single carrier tests with 2Rx

| 4Rx test lists | 2Rx test lists |
| --- | --- |
| 8.10.1.1.1 Test 1 | 8.2.1.2.1 Test 1 |
| 8.10.1.1.2 Test 1 | 8.2.1.3.1 Test 1 |
| 8.10.1.1.3 Test 1 | 8.2.1.4.1B Test 1 |
| 8.10.1.1.4 Test 1 | 8.2.1.4.2 Test 1 |
| 8.10.1.1.4 Test 2 | 8.2.1.4.2 Test 3 |
| 8.10.1.1.5 Test 1 | 8.3.1.1A Test 1 |
| 8.10.1.1.5A Test 1 | 8.3.1.1 Test 2 |
| 8.10.1.1.5B Test 1 | 8.3.1.1H Test 1 |
| 8.10.1.1.6 Test 1 | 8.3.1.2 Test 1 |
| 8.10.1.1.14 Test 1 | 8.2.1.9.2 Test 1 |
| 8.10.1.2.1 Test 1 | 8.2.2.2.1 Test 1 |
| 8.10.1.2.2 Test 1 | 8.2.2.3.1 Test 1 |
| 8.10.1.2.3 Test 1 | 8.2.2.4.1B Test 1 |
| 8.10.1.2.4 Test 1 | 8.2.2.4.2 Test 1 |
| 8.10.1.2.4 Test 2 | 8.2.2.4.2 Test 3 |
| 8.10.1.2.5 Test 1 | 8.3.2.1B Test 1 |
| 8.10.1.2.5A Test 1 | 8.3.2.1A Test 2 |
| 8.10.1.2.5B Test 1 | 8.3.1.1I Test 1 |
| 8.10.1.2.6 Test 1 | 8.3.2.2 Test 2 |
| 8.10.1.2.14 Test 1 | 8.2.2.9.2 Test 1 |
| 8.10.2.1.1 Test 1 | 8.4.1.1 Test 1 |
| 8.10.2.1.2 Test 1 | 8.4.1.2.1 Test 1 |
| 8.10.2.1.3 Test 1 | 8.4.1.2.2 Test 1 |
| 8.10.2.2.1 Test 1 | 8.4.2.1 Test 1 |
| 8.10.2.2.2 Test 1 | 8.4.2.2.1 Test 1 |
| 8.10.2.2.3 Test 1 | 8.4.2.2.2 Test 1 |
| 8.10.3.1.1 Test 1 | 8.5.1.1 Test 1 |
| 8.10.3.1.2 Test 1 | 8.5.1.2.1 Test 1 |
| 8.10.3.1.3 Test 1 | 8.5.1.2.2 Test 1 |
| 8.10.3.2.1 Test 1 | 8.5.2.1 Test 1 |
| 8.10.3.2.2 Test 1 | 8.5.2.2.1 Test 1 |
| 8.10.3.2.3 Test 1 | 8.5.2.2.2 Test 1 |
| 8.10.4.1.1 Test 1 | 8.8.1.1 Test 1 |
| 8.10.4.1.1 Test 2 | 8.8.1.1 Test 2 |
| 8.10.4.1.2 Test 1 | 8.8.1.2 Test 1 |
| 8.10.4.1.2 Test 2 | 8.8.1.2 Test 2 |
| 8.10.4.2.1 Test 1 | 8.8.2.1 Test 1 |
| 8.10.4.2.1 Test 2 | 8.8.2.1 Test 2 |
| 8.10.4.2.2 Test 1 | 8.8.2.2 Test 1 |
| 8.10.4.2.2 Test 2 | 8.8.2.2 Test 2 |

##### 8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx

All tests specified in 8.2 to 8.8 with 2Rx with CA, TDD-FDD CA and DC are tested with 4 Rx capable UEs.

Within the CA/DC configuration if any of the PCell and/or the SCells and/or PSCells is a 2Rx supported RF band, the antenna connection should follow the same method as defined in 8.1.2.6.1 for single carrier tests on any of the 2Rx supported RF bands, with same requirements specified with 2Rx applied. Within the CA configuration if any of the PCell and/or the SCells and/or PSCells is a 4Rx supported RF band, the antenna connection should follow the same as defined in 8.1.2.6.1 for single carrier tests on any of the 4 Rx supported RF bands, with the SNR requirements applied with  1.5 dB less than the number specified with 2Rx.

Same applicability rules defined in 8.1.2.3, 8.1.2.3A, and 8.1.2.3B for CA, TDD-FDD CA and DC applied for different CA and DC configurations and bandwidth combination sets should be applied for 4 Rx capable UEs.

##### 8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx

For 4Rx capable UEs all single carrier tests specified in 8.10 with 4Rx are tested on any of the 4Rx supported RF bands by connecting all 4Rx with data source from system simulator.

##### 8.1.2.6.4	Applicability rule for 256QAM tests

For 256QAM capable UE, if corresponding tests specified in 8.10.1.1.4 Test 2 and 8.10.1.2.4 Test 2 are tested, the test coverage can be considered fulfilled without executing the tests specified in 8.10.1.1.4 Test 1 and 8.10.1.2.4 Test 1. For a UE which does not have 256QAM capability, the test specified in 8.10.1.1.4 Test 1 and 8.10.1.2.4 Test 1 should be used.

##### 8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx

All tests specified in 8.13 with FDD CA/DC, TDD CA/DC and TDD-FDD CA/DC are tested with 4 Rx capable UEs.

Within the CA/DC configuration if any of the PCell and/or the SCells/PSCell is a 2Rx supported RF band, 2 out of the 4Rx should be connected with data source from system simulator, and the other 2Rx are connected with zero input, depending on UE’s declaration and AP configuration. Within the CA/DC configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, all 4Rx should be connected with data source from system simulator.

For 4Rx capable UEs supporting different CA/DC configurations and bandwidth combination sets, the applicability and test rules are defined in Table 8.1.2.6.5-1 for FDD CA/DC, TDD CA/DC and TDD-FDD CA/DC. For simplicity, CA/DC configuration below refers to combination of CA/DC configuration and bandwidth combination set.

Table 8.1.2.6.5-1: Applicability and test rules for CA/DC/TDD-FDD CA UE demodulation tests

| Tests | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA tests in Clause 8.13.1.1.1, 8.13.1.2.1 | Select FDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported FDD CA configurations with any CA capability | Select FDD CA configurations with maximum number of 4Rx CCs among all the selected FDD CA configurations from Step 1 | Select any one of the FDD CA configuration with the largest aggregated CA bandwidth combination among all the selected FDD CA configurations from Step 2 | NA |
| DC tests in Clause 8.13.1.1.2 | Select FDD DC configurations with the maximum number of CCs excluding the 2Rx only DC configuration among all supported FDD DC configurations with any DC capability | Select FDD DC configurations with maximum number of 4Rx CCs among all the selected FDD DC configurations from Step 1 | Select any one of the FDD DC configuration with the largest aggregated DC bandwidth combination among all the selected FDD DC configurations from Step 2 | NA |
| CA tests in Clause 8.13.2.1.1, 8.13.2.2.1 | Select TDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported TDD CA configurations with any CA capability | Select TDD CA configurations with maximum number of 4Rx CCs among all the selected TDD CA configurations from Step 1 | Select any one of the TDD CA configuration with the largest aggregated CA bandwidth combination among all the selected TDD CA configurations from Step 2 | NA |
| DC tests in Clause 8.13.2.1.2 | Select TDD DC configurations with the maximum number of CCs excluding the 2Rx only DC configuration among all supported TDD DC configurations with any DC capability | Select TDD DC configurations with maximum number of 4Rx CCs among all the selected TDD DC configurations from Step 1 | Select any one of the TDD DC configuration with the largest aggregated DC bandwidth combination among all the selected TDD DC configurations from Step 2 | NA |
| CA tests in Clause 8.13.3.1.1, 8.13.3.1.2, 8.13.3.2.1, 8.13.3.2.2 | Select TDD-FDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported TDD-FDD CA configurations with any CA capability | Select TDD-FDD CA configurations with maximum number of 4Rx CCs among all the selected TDD-FDD CA configurations from Step 1 | Select any one of the TDD-FDD CA configuration with the largest aggregated CA bandwidth combination among all the selected TDD-FDD CA configurations from Step 2 | NA |
| DC tests in Clause 8.13.3.5.1, 8.13.3.5.2 | Select TDD-FDD DC configurations with the maximum number of CCs excluding the 2Rx only DC configuration among all supported TDD-FDD DC configurations with any DC capability | Select TDD-FDD DC configurations with maximum number of 4Rx CCs among all the selected TDD-FDD DC configurations from Step 1 | Select any one of the TDD-FDD DC configuration with the largest aggregated DC bandwidth combination among all the selected TDD-FDD DC configurations from Step 2 | NA |
| CA tests in Clause 8.13.1.3.1, 8.13.1.4.1, 8.13.1.1.3 | Select FDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported FDD CA configurations with any CA capability | Select FDD CA configurations with maximum number of 4Rx CCs among all the selected FDD CA configurations from Step 1 | Select any one of the FDD CA configuration with the largest aggregated CA bandwidth combination among all the selected FDD CA configurations from Step 2 | Select 2CCs with maximum number of 4Rx CCs with the largest aggregated CA bandwidth combination among the selected FDD CA configuration from Step 3 |
| CA tests in Clause 8.13.2.3.1, 8.13.2.4.1, 8.13.2.1.3 | Select TDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported TDD CA configurations with any CA capability | Select TDD CA configurations with maximum number of 4Rx CCs among all the selected TDD CA configurations from Step 1 | Select any one of the TDD CA configuration with the largest aggregated CA bandwidth combination among all the selected TDD CA configurations from Step 2 | Select 2CCs with maximum number of 4Rx CCs with the largest aggregated CA bandwidth combination among the selected TDD CA configuration from Step 3 |
| CA tests in Clause 8.13.3.3.1, 8.13.3.3.2, 8.13.3.4.1, 8.13.3.4.2, 8.13.3.6.1, 8.13.3.6.2 | Select TDD-FDD CA configurations with the maximum number of CCs excluding the 2Rx only CA configuration among all supported TDD-FDD CA configurations with any CA capability | Select TDD-FDD CA configurations with maximum number of 4Rx CCs among all the selected TDD-FDD CA configurations from Step 1 | Select any one of the TDD-FDD CA configuration with the largest aggregated CA bandwidth combination among all the selected TDD CA configurations from Step 2 | Select 2CCs with maximum number of 4Rx CCs with the largest aggregated CA bandwidth combination among the selected TDD-FDD CA configuration from Step 3 |
| CA tests in Clause 8.13.1.1.4 | Select FDD CA configurations with the maximum number of CCs supporting 4 layers MIMO among all supported FDD CA configurations with any CA capability | Select any one of the FDD CA configuration with the largest aggregated CA bandwidth combinations on any 2CCs among all the CCs among all the slected FDD CA configuration from Step 1 | NA | NA |
| CA tests in Clause 8.13.2.1.4 | Select TDD CA configurations with the maximum number of CCs supporting 4 layers MIMO among all supported TDD CA configurations with any CA capability | Select any one of the TDD CA configuration with the largest aggregated CA bandwidth combinations on any 2CCs among all the CCs among all the slected TDD CA configuration from Step 1 | NA | NA |
| CA tests in Clause 8.13.3.7.1, 8.13.3.7.2 | Select TDD-FDD CA configurations with the maximum number of CCs supporting 4 layers MIMO among all supported TDD-FDD CA configurations with any CA capability | Select any one of the TDD-FDD CA configuration with the largest aggregated CA bandwidth combinations on any 2CCs among all the CCs among all the slected TDD-FDD CA configuration from Step 1 | NA | NA |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all CA tests and 2 Uplink CCs are configured for all DC tests. |  |  |  |  |

For 4Rx capable UEs, if corresponding tests listed from the 4Rx CA/DC test lists from Table 8.1.2.6.5-2 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed from the 2Rx CA/DC test lists from Table 8.1.2.6.5-2.

Table 8.1.2.6.5-2: Test lists for applicability rules for CA/DC/TDD-FDD CA tests with 4Rx

| 4Rx test lists | 2Rx test lists |
| --- | --- |
| 8.13.1.1.1 CA tests | 8.2.1.4.3 CA tests |
| 8.13.1.1.2 DC tests | 8.2.1.4.3A DC tests |
| 8.13.1.2.1 CA tests | 8.2.1.1.1 CA tests |
| 8.13.2.1.1 CA tests | 8.2.2.4.3 CA tests |
| 8.13.2.1.2 DC tests | 8.2.2.4.3A DC tests |
| 8.13.2.2.1 CA tests | 8.2.2.1.1 CA tests |
| 8.13.3.1.1 CA tests | 8.2.3.3.1 CA tests |
| 8.13.3.1.2 CA tests | 8.2.3.3.2 CA tests |
| 8.13.3.2.1 CA tests | 8.2.3.1.1 CA tests |
| 8.13.3.2.2 CA tests | 8.2.3.1.2 CA tests |
| 8.13.1.1.3 CA tests | 8.2.1.4.2 test 3 and 8.10.1.1.4 test 2 |
| 8.13.1.1.4 CA tests | 8.10.1.1.8 test 1 |
| 8.13.1.3.1 CA tests | 8.2.1.4.1B test 1 and 8.10.1.1.3 test 1 |
| 8.13.1.4.1 CA tests | 8.3.1.1A test 1 and 8.10.1.1.5 test 1 |
| 8.13.2.1.3 CA tests | 8.2.2.4.2 test 3 and 8.10.1.2.4 test 2 |
| 8.13.2.1.4 CA tests | 8.10.1.2.8 test 1 |
| 8.13.2.3.1 CA tests | 8.2.2.4.1B test 1 and 8.10.1.2.3 test 1 |
| 8.13.2.4.1 CA tests | 8.3.2.1B test 1 and 8.10.1.2.5 test 1 |

##### 8.1.2.6.6	Applicability rule for Type C with 4Rx

For Type C with 4Rx capable UEs, if the tests from the 4Rx test lists are tested from Table 8.1.2.6.6-1, the test coverage can be considered fulfilled without executing the corresponding test from 2Rx test lists from Table 8.1.2.6.6-1.

Table 8.1.2.6.6-1: Applicability rules for Type C with 4Rx

| Test category | Type C with 2Rx test lists | Type C with 4Rx test lists |
| --- | --- | --- |
| FDD Tests | 8.2.1.3.1B | 8.2.1.3.1B |
|  | 8.2.1.3.1C | 8.2.1.3.1C |
|  | 8.2.1.4.2A | 8.10.1.1.4A |
|  | 8.3.1.2A | 8.10.1.1.6A |
| TDD Tests | 8.2.2.3.1B | 8.2.2.3.1B |
|  | 8.2.2.3.1C | 8.2.2.3.1C |
|  | 8.2.2.4.2A | 8.10.1.2.4A |
|  | 8.3.2.2A | 8.10.1.2.6A |

##### 8.1.2.6.7	Applicability rule for 1024QAM tests

For 1024QAM capable UE, if corresponding tests specified in 8.10.1.1.4 Test 3, 8.10.1.2.4 Test 3, 8.10.1.1.6B Test 1, and 8.10.1.2.6B Test 1 are tested, the test coverage can be considered fulfilled without executing the tests specified in 8.10.1.1.4 Test 1-2, 8.10.1.2.4 Test 1-2, 8.2.1.4.1A Test2, 8.2.2.4.1A Test 2, 8.3.1.1 Test 5, and 8.3.2.1A Test 5.

#### 8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements

For UE which supports downlink control channel Type A or Type B interference mitigation capabilities the tests from Table 8.1.2.7-1 rows corresponding should be applied in accordance to the supported UE-EUTRA-Capability [7].

Table 8.1.2.7-1: Applicability rules for enhanced downlink control channel performance requirements

| Test set | UE-EUTRA-Capability [7] | Test category |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- | --- |
| 1 | cch-InterfMitigation-RefRecTypeA-r13 (Enhanced downlink control channel interference mitigation Type A receiver for 2 CRS antenna ports for UEs with 2 receiver antenna ports) | FDD | PDCCH/PCFICH | 8.4.1.2.5 Test 1 8.4.1.2.6 Test 1 |  |
|  |  |  | PHICH | 8.5.1.2.5 Test 1 8.5.1.2.6 Test 1 |  |
|  |  |  | EPDCCH | 8.8.4.1 Test 1 8.8.6.1 Test 1 |  |
|  |  | TDD | PDCCH/PCFICH | 8.4.2.2.5 Test 1 8.4.2.2.6 Test 1 |  |
|  |  |  | PHICH | 8.5.2.2.5 Test 1 8.5.2.2.6 Test 1 |  |
|  |  |  | EPDCCH | 8.8.4.2 Test 1 8.8.5.1 Test 1 |  |
| 2 | cch-InterfMitigation-RefRecTypeA-r13 (Enhanced downlink control channel interference mitigation Type A receiver for 4 CRS antenna ports for UEs with 2 receiver antenna ports) | FDD | PDCCH/PCFICH | 8.4.1.2.9 Test 1 | UE may skip the tests 8.4.1.2.6 and 8.4.2.2.6 defined for test set 1 |
|  |  | TDD | PDCCH/PCFICH | 8.4.2.2.9 Test 1 |  |
| 3 | cch-InterfMitigation-RefRecTypeB-r13 | FDD | PDCCH/PCFICH | 8.4.1.2.5 Test 1 8.4.1.2.7 Test 1 8.4.1.2.8 Test 1 | UE may skip the tests defined for test set 1 |
|  |  |  | PHICH | 8.5.1.2.5 Test 1 8.5.1.2.7 Test 1 8.5.1.2.8 Test 1 |  |
|  |  |  | EPDCCH | 8.8.4.1 Test 1 8.8.6.1 Test 1 |  |
|  |  | TDD | PDCCH/PCFICH | 8.4.2.2.7 Test 1 8.4.2.2.8 Test 1 |  |
|  |  |  | PHICH | 8.5.2.2.7 Test 1 8.5.2.2.8 Test 1 |  |
|  |  |  | EPDCCH | 8.8.4.2 Test 1 8.8.5.1 Test 1 |  |
| 4 | MMSE-IRC DL Control Channel interference mitigation receiver for UEs with 4 receiver antenna ports [TS 36.306 Section 6.6.5] | FDD | PDCCH/PCFICH | 8.10.2.1.4 Test 1 |  |
|  |  | TDD | PDCCH/PCFICH | 8.10.2.2.4 Test 1 |  |
| 5 | [Enhanced downlink control channel interference mitigation Type A receiver for 2 CRS antenna ports for UEs with 1 receiver antenna ports and Category 1bis] | FDD | PDCCH/PCFICH | 8.9.4.1.1 Test 1 |  |
|  |  | TDD | PDCCH/PCFICH | [8.9.4.2.1 Test 1] |  |
| 6 | [Enhanced downlink control channel interference mitigation Type A receiver for 4 CRS antenna ports for UEs with 1 receiver antenna ports and Category 1bis] | FDD | PDCCH/PCFICH | 8.9.4.1.2 Test 1 | UE may skip the tests defined for test set 5 |
|  |  | TDD | PDCCH/PCFICH | [8.9.4.2.2 Test 1] |  |
| 7 | [Enhanced downlink control channel interference mitigation Type A receiver for 2 CRS antenna ports for UEs with 1 receiver antenna ports and Category M2] | FDD | MPDCCH | 8.11.2.1.3 Test 1 8.11.2.1.4 Test 1 |  |
|  |  | TDD | MPDCCH | 8.11.2.2.3 Test 1 8.11.2.2.4 Test 1 |  |

#### 8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

For a UE which supports DMRS enhancement (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]), if corresponding tests specified in 8.3.1.1H Test 1 and 8.3.2.1I Test 1 are tested, the test coverage can be considered fulfilled without executing the tests specified in 8.3.1.1 Test 2 and 8.3.2.1A Test 2. For a UE which does not have DMRS enhancement capability, the test specified in in 8.3.1.1 Test 2 and 8.3.2.1A Test 2 should be used.

#### 8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement

The applicability and test rules are defined for the tests for UE supporting coverage enhancement with narrowband transmission in Table 8.1.2.8A-1 and 8.1.2.8A-2.

Table 8.1.2.8A-1: Applicability rules for PDSCH requirements (FDD)

|  | ue-Category or UE-CategoryDL |  |
| --- | --- | --- |
| UE-EUTRA-Capability | M1, Cat-0, 1bis | >=1 (Note 1) |
| ce-ModeA-r13 | 8.11.1.1.1.1 Test 1 8.11.1.1.2.1 Test 1 8.11.1.1.3.1 Test 2 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 |
| ce-ModeB-r13 | 8.11.1.1.1.1 Test 1 8.11.1.1.2.1 Test 1 8.11.1.1.3.1 Test 1 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 8.11.1.1.3.1 Test 3 8.11.1.1.3.1 Test 4 |
| Note 1: Which test case applies to the UE depends on the supported receiver antenna number. |  |  |

Table 8.1.2.8A-2: Applicability rules for PDSCH requirements (TDD)

|  | ue-Category or UE-CategoryDL |  |
| --- | --- | --- |
| UE-EUTRA-Capability | M1, Cat-0, 1bis | >=1 (Note 1) |
| ce-ModeA-r13 | 8.11.1.2.1.1 Test 1 8.11.1.2.2.1 Test 1 8.11.1.2.3.1 Test 2 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 |
| ce-ModeB-r13 | 8.11.1.2.1.1 Test 1 8.11.1.2.2.1 Test 1 8.11.1.2.3.1 Test 1 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 8.11.1.2.3.1 Test 3 8.11.1.2.3.1 Test 4 |
| Note 1: Which test case applies to the UE depends on the supported receiver antenna number. |  |  |

The applicability and test rules are defined for the tests for UE supporting coverage enhancement with wideband transmission in Table 8.1.2.8A-3 and 8.1.2.8A-4.

Table 8.1.2.8A-3: Applicability rules for PDSCH requirements (FDD)

|  | ue-Category or UE-CategoryDL |  |  |
| --- | --- | --- | --- |
| UE-EUTRA-Capability | M2, Cat-0, 1bis | ≥1 (Note 1) |  |
| ce-PDSCH-PUSCH-MaxBandwidth-r14 | 5MHz | 5MHz | 20MHz |
| ce-ModeA-r13 | 8.11.1.1.1.1 Test 1 8.11.1.1.2.1 Test 1 8.11.1.1.3.2 Test 1 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 8.11.1.1.3.2 Test 3 8.11.1.1.3.2 Test 4 |
| ce-ModeB-r13 | 8.11.1.1.1.1 Test 1 8.11.1.1.2.1 Test 1 8.11.1.1.3.2 Test 2 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 8.11.1.1.3.1 Test 3 8.11.1.1.3.1 Test 4 | 8.11.1.1.2.1 Test 2 8.11.1.1.2.1 Test 3 8.11.1.1.3.1 Test 3 8.11.1.1.3.1 Test 4 |
| Note 1: Which test case applies to the UE depends on the supported receiver antenna number. |  |  |  |

Table 8.1.2.8A-4: Applicability rules for PDSCH requirements (TDD)

|  | ue-Category or UE-CategoryDL |  |  |
| --- | --- | --- | --- |
| UE-EUTRA-Capability | M2, Cat-0, 1bis | ≥1 (Note 1) |  |
| ce-PDSCH-PUSCH-MaxBandwidth-r14 | 5MHz | 5MHz | 20MHz |
| ce-ModeA-r13 | 8.11.1.2.1.1 Test 1 8.11.1.2.2.1 Test 1 8.11.1.2.3.2 Test 1 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 8.11.1.2.3.2 Test 3 8.11.1.2.3.2 Test 4 |
| ce-ModeB-r13 | 8.11.1.2.1.1 Test 1 8.11.1.2.2.1 Test 1 8.11.1.2.3.2 Test 2 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 8.11.1.2.3.1 Test 3 8.11.1.2.3.1 Test 4 | 8.11.1.2.2.1 Test 2 8.11.1.2.2.1 Test 3 8.11.1.2.3.1 Test 3 8.11.1.2.3.1 Test 4 |
| Note 1: Which test case applies to the UE depends on the supported receiver antenna number. |  |  |  |

The applicability and test rules are defined for the tests for UE supporting coverage enhancement in Tables 8.1.2.8A-5 and 8.1.2.8A-6.

Table 8.1.2.8A-5: Applicability rules for PDSCH requirements (FDD)

|  | ue-Category or UE-CategoryDL |  |
| --- | --- | --- |
| UE-EUTRA-Capability | M1 | M2 |
| ce-ModeA-r13 | 8.11.1.1.3.1 Test 2a 8.11.1.1.3.1 Test 5 8.11.1.1.3.1 Test 6 | 8.11.1.1.3.1 Test 5 8.11.1.1.3.1 Test 6 |
| ce-ModeB-r13 |  | 8.11.1.1.3.2 Test 2a |

Table 8.1.2.8A-6: Applicability rules for PDSCH requirements (TDD)

|  | ue-Category or UE-CategoryDL |  |
| --- | --- | --- |
| UE-EUTRA-Capability | M1 | M2 |
| ce-ModeA-r13 | 8.11.1.2.3.1 Test 2a 8.11.1.2.3.1 Test 5 8.11.1.2.3.1 Test 6 | 8.11.1.2.3.1 Test 5 8.11.1.2.3.1 Test 6 |
| ce-ModeB-r13 |  | 8.11.1.2.3.2 Test 2a |

#### 8.1.2.9	Applicability of SDR requirements for CA and LAA

For UE supporting both CA and LAA, UE selects the corresponding SDR tests between CA and LAA as per the following applicab1ility rules:

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC following the equation that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are LAA and non-LAA CA configurations with the same largest aggregated bandwidth, select non-LAA CA configuration.

#### 8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission

For a UE which does not supports DMRS enhancement table (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]), if corresponding tests specified in 8.3.1.1I Test 1, 8.3.2.1J Test 1 are tested, the test coverage can be considered fulfilled without executing other tests specified in 8.3.1.1I and 8.3.2.1J.

For a UE which supports DMRS enhancement table (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]) and enahcned performance Type D in TM8/9 or TM10 with assistance information for up to 1 interfering layer, if corresponding tests specified in 8.3.1.1I Test 2, 8.3.2.1J Test 2 are tested, the test coverage can be considered fulfilled without executing other tests specified in 8.3.1.1I and 8.3.2.1J.

For a UE which supports DMRS enhancement table (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]) and enahcned performance Type D in TM8/9 or TM10 with assistance information for up to 3 interfering layer, if corresponding tests specified in 8.3.1.1I Test 3, 8.3.2.1J Test 3 are tested, the test coverage can be considered fulfilled without executing other tests specified in 8.3.1.1I and 8.3.2.1J.

#### 8.1.2.11	Applicability CRS interference mitigation receivers performance requirements

For UE which supports CRS interference mitigation capabilities the tests from Table 8.1.2.11-1 should be applied in accordance to the support UE capabilities (UE-EUTRA-Capability [7]).

Table 8.1.2.11-1: Applicability rules for CRS interference mitigation performance requirements

| Test set | UE-EUTRA-Capability [7] | Test list |  | Notes |
| --- | --- | --- | --- | --- |
|  |  | FDD | TDD |  |
| 1 | crs-InterfMitigationTM1toTM9-r13 (CRS-IM with 2 CRS antenna ports for PDSCH for UEs with 2 receiver antenna ports) | 8.2.1.4.1E Test 1 8.3.1.1G Test 1 | 8.2.2.4.1E Test 1 8.3.2.1H Test 1 |  |
| 2 | crs-InterfMitigationTM10-r13 | 8.3.1.3.4 Test 1 8.3.1.3.5 Test 1 | 8.3.2.3.4 Test 1 8.3.2.3.5 Test 1 |  |
| 3 | crs-InterfMitigationTM1toTM9-r13 (CRS-IM with 4 CRS antenna ports for PDSCH for UEs with 2 receiver antenna ports) | 8.2.1.4.1F Test 1 8.3.1.1G Test 1 | 8.2.2.4.1F Test 1 8.3.2.1H Test 1 | UE may skip the tests defined for test set 1 |
| 4 | crs-InterfMitigationTM1toTM9-r13 (CRS-IM with 2 CRS antenna ports for PDSCH for UEs with 4 receiver antenna ports) | 8.10.1.1.10 Test 1 8.10.1.1.12 Test 1 | 8.10.1.2.10 Test 1 8.10.1.2.12 Test 1 | UE may skip the tests defined for test set 1 |
| 5 | crs-InterfMitigationTM1toTM9-r13 (CRS-IM with 4 CRS antenna ports for PDSCH for UEs with 4 receiver antenna ports) | 8.10.1.1.11 Test 1 8.10.1.1.12 Test 1 | 8.10.1.2.11 Test 1 8.10.1.2.12 Test 1 | UE may skip the tests defined for test sets 1,3 and 4 |
| 6 | [CRS-IM with 2 CRS antenna ports for PDSCH for UEs with 1 receiver antenna port and Category 1bis] | 8.9.1.1.3.2 Test 1 | 8.9.1.2.3.2 Test 1 |  |
| 7 | [CRS-IM with 4 CRS antenna ports for PDSCH for UEs with 1 receiver antenna port and Category 1bis] | 8.9.1.1.2.3 Test 1 | 8.9.1.2.2.3 Test 1 | UE may skip the tests defined for test set 6 |
| 8 | [CRS-IM with 2 CRS antenna ports for PDSCH for UEs with 1 receiver antenna port and Category M2] | 8.11.1.1.1.2 Test 1 | 8.11.1.1.2.2 Test 1 |  |
| 9 | [CRS-IM with 4 CRS antenna ports for PDSCH for UEs with 1 receiver antenna port and Category M2] | 8.11.1.1.1.3 Test 1 | 8.11.1.1.2.3 Test 1 | UE may skip the tests defined for test set 8 |

#### 8.1.2.12	Applicability of performance requirements for 8Rx capable UEs

For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

For any demodulation tests and CSI tests conducted in the 8Rx supported RF band, four receive antenna ports that UE may use for control channel demodulation are clarified via UE declaration. When testing an N-Rx (N = 2,4) demodulation or CSI test on the 8Rx supported band, the fading duplication and antenna mapping should guarantee that the four receive antennas UE declares for the control channel demodulation collectively receives at least 4/N duplicated version of the fading channel seen at each receive antenna of the N-Rx test, i.e., the connection diagrams in Figure 8.1.2.12.1-1 to Figure 8.1.2.12.1-4 are valid under the condition that Rx1, Rx3, Rx5 and Rx7 are the four receive antennas declared by UE for the control channel demodulation.

##### 8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests

8.1.2.12.1.1	Applicability rule and antenna connection for single carrier PDSCH tests with 2Rx and 4Rx

For 8Rx capable UEs, all single carrier tests specified in 8.2 to 8.8 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of the 8Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied.

For 8Rx capable UEs, all single carrier test cases specified in 8.10 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied.

For 8Rx capable UEs without support of any 4Rx RF bands, all single carrier tests specified in 8.10 with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 1.5 dB less than the number specified for 4Rx tests.

For 8Rx capable UEs without support of any 2Rx and 4Rx RF bands, all single carrier tests specified in 8.2 to 8.8 with 2Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 3 dB less than the number specified for 2Rx tests.

For 8Rx capable UEs without support of any 2Rx RF bands but with support of 4Rx RF bands, all single carrier tests specified in 8.2 to 8.8 with 2Rx are tested on any of the 4Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. 4 out of 8 Rx are connected with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. SNR requirements should be applied with 1.5 dB less than the number specified for 2Rx tests.

Figure 8.1.2.12.1-1 ~ Figure 8.1.2.12.1-4 show examples of antenna connection for 8Rx capable UE in any one 8Rx supported RF band to perform a 2Rx or 4Rx performance test with antenna configuration as 2x2 or 4x2 for 2Rx tests and 2x4 or 4x4 for 4Rx tests without interference for information.

Figure 8.1.2.12.1-1: Antenna connection example for 2Rx tests with antenna configuration as 2x2 without interference (informative)

Figure 8.1.2.12.1-2: Antenna connection example for 2Rx tests with antenna configuration as 4x2 without interference (informative)

Figure 8.1.2.12.1-3: Antenna connection example for 4Rx tests with antenna configuration as 2x4 without interference (informative)

Figure 8.1.2.12.1-4: Antenna connection example for 4Rx tests with antenna configuration as 4x4 without interference (informative)

For 8Rx capable UEs without any 2Rx supported RF bands, for all single carrier tests listed in Table 8.1.2.12.1-1 specified from 8.2 to 8.8 with 2Rx can be skipped.

Table 8.1.2.12.1-1: Requirement lists with 2Rx not applicable to 8Rx capable UEs

Requirement lists

Enhanced downlink control channel performance requirements type A

Enhanced downlink control channel performance requirements type B

Enhanced performance requirements type A

Enhanced performance requirements type B

Enhanced performance requirements type C

Requirements with demodulation subframe overlaps with aggressor cell ABS

Requirements with demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured

Requirements with CRS assistance information configured

For 8Rx capable UEs without any 4Rx supported RF bands, for all single carrier tests listed in Table 8.1.2.12.1-2 specified in 8.10 with 4Rx can be skipped.

Table 8.1.2.12.1-2: Requirement lists with 4Rx not applicable to 8Rx capable UEs

Requirement lists

Enhanced downlink control channel performance requirements type A

Enhanced performance requirements type A

Enhanced performance requirements type C

Requirements with CRS assistance information configured

For 8Rx capable UEs, if corresponding tests listed from the 8Rx test lists from Table 8.1.2.12.1-3 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed from botht the 4Rx test lists and the 2Rx test lists from Table 8.1.2.12.1-3.

For 8Rx capable UEs, if corresponding tests listed from the 4Rx test lists from Table 8.1.2.12.1-3 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed from the 2Rx test lists from Table 8.1.2.12.1-3.

Table 8.1.2.12.1-3: Applicability rules for single carrier tests with 2Rx

| 8Rx test lists | 4Rx test lists | 2Rx test lists |
| --- | --- | --- |
|  | 8.10.1.1.1 Test 1 | 8.2.1.2.1 Test 1 |
|  | 8.10.1.1.2 Test 1 | 8.2.1.3.1 Test 1 |
|  | 8.10.1.1.3 Test 1 | 8.2.1.4.1B Test 1 |
|  | 8.10.1.1.4 Test 1 | 8.2.1.4.2 Test 1 |
|  | 8.10.1.1.4 Test 2 | 8.2.1.4.2 Test 3 |
|  | 8.10.1.1.5 Test 1 | 8.3.1.1A Test 1 |
|  | 8.10.1.1.5A Test 1 | 8.3.1.1 Test 2 |
|  | 8.10.1.1.5B Test 1 | 8.3.1.1H Test 1 |
|  | 8.10.1.1.6 Test 1 | 8.3.1.2 Test 1 |
| 8.15.1.2.1 Test 1 | 8.10.1.2.1 Test 1 | 8.2.2.2.1 Test 1 |
| 8.15.1.2.2 Test 1 | 8.10.1.2.2 Test 1 | 8.2.2.3.1 Test 1 |
|  | 8.10.1.2.3 Test 1 | 8.2.2.4.1B Test 1 |
|  | 8.10.1.2.4 Test 1 | 8.2.2.4.2 Test 1 |
|  | 8.10.1.2.4 Test 2 | 8.2.2.4.2 Test 3 |
|  | 8.10.1.2.5 Test 1 | 8.3.2.1B Test 1 |
|  | 8.10.1.2.5A Test 1 | 8.3.2.1A Test 2 |
|  | 8.10.1.2.5B Test 1 | 8.3.1.1I Test 1 |
|  | 8.10.1.2.6 Test 1 | 8.3.2.2 Test 2 |
|  | 8.10.2.1.1 Test 1 | 8.4.1.1 Test 1 |
|  | 8.10.2.1.2 Test 1 | 8.4.1.2.1 Test 1 |
|  | 8.10.2.1.3 Test 1 | 8.4.1.2.2 Test 1 |
|  | 8.10.2.2.1 Test 1 | 8.4.2.1 Test 1 |
|  | 8.10.2.2.2 Test 1 | 8.4.2.2.1 Test 1 |
|  | 8.10.2.2.3 Test 1 | 8.4.2.2.2 Test 1 |
|  | 8.10.3.1.1 Test 1 | 8.5.1.1 Test 1 |
|  | 8.10.3.1.2 Test 1 | 8.5.1.2.1 Test 1 |
|  | 8.10.3.1.3 Test 1 | 8.5.1.2.2 Test 1 |
|  | 8.10.3.2.1 Test 1 | 8.5.2.1 Test 1 |
|  | 8.10.3.2.2 Test 1 | 8.5.2.2.1 Test 1 |
|  | 8.10.3.2.3 Test 1 | 8.5.2.2.2 Test 1 |
|  | 8.10.4.1.1 Test 1 | 8.8.1.1 Test 1 |
|  | 8.10.4.1.1 Test 2 | 8.8.1.1 Test 2 |
|  | 8.10.4.1.2 Test 1 | 8.8.1.2 Test 1 |
|  | 8.10.4.1.2 Test 2 | 8.8.1.2 Test 2 |
|  | 8.10.4.2.1 Test 1 | 8.8.2.1 Test 1 |
|  | 8.10.4.2.1 Test 2 | 8.8.2.1 Test 2 |
|  | 8.10.4.2.2 Test 1 | 8.8.2.2 Test 1 |
|  | 8.10.4.2.2 Test 2 | 8.8.2.2 Test 2 |

8.1.2.12.1.2	Applicability rule and antenna connection for single carrier PDSCH tests with 8Rx

For 8Rx capable UEs all single carrier tests specified in 8.14 with 8Rx are tested on any of the 8Rx supported RF bands by connecting all 8Rx with data source from system simulator.

##### 8.1.2.12.2	Applicability rule and antenna connection for control channel tests

8.1.2.12.2.1	Applicability rule and antenna connection for control channel tests with 2Rx

For 8Rx capable UEs, all single carrier tests specified in 8.2 to 8.8 for control channel with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of the 8Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied.

For 8Rx capable UEs without any 2Rx supported RF bands and with 4Rx supported RF bands, all single carrier tests specified in 8.2 to 8.8 for control channel with 2Rx are tested on any of the 4Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 1.5 dB less than the number specified for 2Rx tests.

For 8Rx capable UEs without any 2Rx and without 4Rx supported RF bands, all single carrier tests specified in 8.2 to 8.8 for control channel with 2Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna, as illustrated in Figure 8.1.2.12.1-1 and Figure 8.1.2.12.1-2. The SNR requirements should be applied with 1.5 dB less than the number specified for 2Rx tests.

8.1.2.12.2.2	Applicability rule and antenna connection for control channel tests with 4Rx

For 8Rx capable UEs, all single carrier tests specified in 8.10 for control channel with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of the 8Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied.

For 8Rx capable UEs without any 4Rx supported RF bands, all single carrier tests specified in 8.10 for control channel with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna, as illustrated in Figure 8.1.2.12.1-3 and Figure 8.1.2.12.1-4. Same requirements specified with 4Rx should be applied.

##### 8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests

8.1.2.12.3.1	Applicability rule and antenna connection for CA and DC tests with 2Rx

All tests specified in 8.2 to 8.8 with 2Rx with CA, TDD-FDD CA and DC are tested with 8 Rx capable UEs.

Within the CA/DC configuration if any of the PCell and/or the SCells and/or PSCells is a 2Rx supported RF band, the antenna connection should follow the same method as defined in 8.1.2.12.1 and 8.1.2.12.2 for single carrier tests on any of the 2Rx supported RF bands, with the same requirements specified with 2Rx applied.

Within the CA configuration if any of the PCell and/or the SCells and/or PSCells is a 4Rx supported RF band, the antenna connection should follow the same as illustrated in Figure 8.1.2.12.1-1 and Figure 8.1.2.12.1-2 for single carrier tests on any of the 4 Rx supported RF bands, with the SNR requirements applied with 1.5dB less than the number specified with 2Rx.

Within the CA configuration if any of the PCell and/or the SCells and/or PSCells is an 8Rx supported RF band, the antenna connection should follow the same as illustrated in Figure 8.1.2.12.1-1 and Figure 8.1.2.12.1-2 for single carrier tests on any of the 8Rx supported RF bands, with the SNR requirements applied with 3.0dB less than the number specified with 2Rx for PDSCH tests and 1.5dB less than the number specified with 2Rx for control channel tests.

Same applicability rules defined in 8.1.2.3, 8.1.2.3A, and 8.1.2.3B for CA, TDD-FDD CA and DC applied for different CA and DC configurations and bandwidth combination sets should be applied for 8 Rx capable UEs.

8.1.2.12.3.2	Applicability rule and antenna connection for CA tests with 4Rx

All tests specified in 8.13 with 4Rx with FDD CA/DC, TDD CA/DC and TDD-FDD CA/DC are tested with 8 Rx capable UEs.

Within the CA/DC configuration if any of the PCell and/or the SCells and/or PSCells is a 4Rx supported RF band, the antenna connection should follow the same method as defined in 8.1.2.12.1 and 8.1.2.12.2 for single carrier tests on any of the 4Rx supported RF bands, with the same requirements specified with 4Rx applied.

Within the CA configuration if any of the PCell and/or the SCells and/or PSCells is an 8Rx supported RF band, the antenna connection should follow the same as illustrated in Figure 8.1.2.12.1-3 and Figure 8.1.2.12.1-4 for single carrier PDSCH tests on any of the 8Rx supported RF bands, with the SNR requirements applied with 1.5dB less than the number specified with 4Rx for PDSCH tests, and with the same SNR requirements as specified with 4Rx applied for control channel test.

Same applicability rules defined in 8.1.2.6.5 for CA, TDD-FDD CA and DC applied for different CA and DC configurations and bandwidth combination sets should be applied for 8 Rx capable UEs.

8.1.2.12.3.3	Applicability rule and antenna connection for CA tests with 8Rx

All tests specified in 8.14.2 with 8Rx with TDD CA are tested with 8 Rx capable UEs.

Within the CA configuration if any of the PCell and/or the SCells and/or PSCells is an 8Rx supported RF band, the antenna connection should follow the same method as defined in 8.1.2.12.1 for single carrier PDSCH tests on any of the 8Rx supported RF bands, with the same requirements specified with 8Rx applied.

For 8Rx capable UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 8.1.2.12.3-1 for TDD CA. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 8.1.2.12.3-1: Applicability and test rules for CA UE demodulation tests

| Tests | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA tests in Clause 8.14.2.2.1 | Select TDD CA configurations with the maximum number of CCs supporting 8 layers MIMO among all supported TDD CA configurations with any CA capability | Select any one of the TDD CA configuration with the largest aggregated CA bandwidth combinations on any 2CCs among all the CCs among all the slected TDD CA configuration from Step 1 | NA | NA |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all CA tests. |  |  |  |  |

### 8.1.3	UE category and UE DL category

UE category and UE DL category refer to ue-Category  and ue-CategoryDL define in 4.1 and 4.1A from [12]. A UE that belongs to either a UE category or a UE DL category indicated in UE performance requirements in subclause 8, 9, 10 shall fulfil the corresponding requirements.

A UE indicating DL category 13 may indicate category 9 or 10 and shall thereby fulfil all requirements in subclause 8, 9, 10  that are indicated for either cat 9 or DL Cat 13 UEs. For SDR tests in section 8.7 both cat 9 and cat 13 test shall be used for this UE while for the other test only Cat 13 tests needs to be done.

## 8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)

### 8.2.1	FDD (Fixed Reference Channel)

The parameters specified in Table 8.2.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.2.1-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Cross carrier scheduling |  | Not configured |

#### 8.2.1.1	Single-antenna port performance

The single-antenna performance in a given multi-path fading environments is determined by the SNR for which a certain relative information bit throughput of the reference measurement channels in Annex A.3.3 is achieved. The purpose of these tests is to verify the single-antenna performance with different channel models and MCS. The QPSK and 64QAM cases are also used to verify the performance for all bandwidths specified in Table 5.6.1-1.

##### 8.2.1.1.1	Minimum Requirement

For single carrier, the requirements are specified in Table 8.2.1.1.1-2, with the addition of the parameters in Table 8.2.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.1.1.1-4, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-6, based on single carrier requirement speicified in Table 8.2.1.1.1-5, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-7, based on single carrier requirement speicified in Table 8.2.1.1.1-5, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-8, based on single carrier requirement speicified in Table 8.2.1.1.1-5, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-9, based on single carrier requirement speicified in Table 8.2.1.1.1-5, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-10, based on single carrier requirement speicified in Table 8.2.1.1.1-5, with the addition of the parameters in Table 8.2.1.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.1.1.1-1: Test Parameters

| Parameter |  | Unit | Test 1- 5 | Test 6- 8 | Test 9- 15 | Test 16- 18 | Test 19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (NOTE 1) | 0 (NOTE 1) | 0 (NOTE 1) | 0 (NOTE 1) | 0 (NOTE 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (NOTE 2) | OCNG (NOTE 2) | OCNG (NOTE 2) | OCNG (NOTE 2) | OCNG (NOTE 2) |
| Modulation |  |  | QPSK | 16QAM | 64QAM | 16QAM | QPSK |
| PDSCH transmission mode |  |  | 1 | 1 | 1 | 1 | 1 |
| NOTE 1: . NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. NOTE 3: Void. NOTE 4: Void. |  |  |  |  |  |  |  |

Table 8.2.1.1.1-2: Minimum performance (FRC)

| Test num. | Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 | ≥1 |
| 2 | 10 MHz | R.2 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | -0.4 | ≥1 |
| 3 | 10 MHz | R.2 FDD | OP.1 FDD | ETU300 | 1x2 Low | 70 | 0.0 | ≥1 |
| 4 | 10 MHz | R.2 FDD | OP.1 FDD | HST | 1x2 | 70 | -2.4 | ≥1 |
| 5 | 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 0.0 | ≥1 |
| 6 | 10 MHz | R.3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | ≥2 |
| 7 | 10 MHz | R.3 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | ≥2 |
| 8 | 10 MHz | R.3 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | ≥2 |
| 9 | 3 MHz | R.5 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥1 |
| 10 | 5 MHz | R.6 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.4 | ≥2 |
|  | 5 MHz | R.6-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.5 | 1 |
| 11 | 10 MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.7 | 1 |
| 12 | 10 MHz | R.7 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | 19.0 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | 18.1 | 1 |
| 13 | 10 MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 High | 70 | 19.1 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | EVA5 | 1x2 High | 70 | 17.8 | 1 |
| 14 | 15 MHz | R.8 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 | ≥2 |
|  | 15 MHz | R.8-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.8 | 1 |
| 15 | 20 MHz | R.9 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥3 |
|  | 20 MHz | R.9-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.3 | 2 |
|  | 20 MHz | R.9-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.7 | 1 |
| 16 | 3 MHz | R.0 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 17 | 10 MHz | R.1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 18 | 20 MHz | R.1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 19 | 10 MHz | R.41 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -5.4 | ≥1 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Void. NOTE 4: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |  |

Table 8.2.1.1.1-3: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (NOTE 2) |
| Modulation |  |  | QPSK |
| PDSCH transmission mode |  |  | 1 |
| NOTE 1: . NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. NOTE 3: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.1.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.1.1-6. NOTE 4: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.1.1-4: Minimum performance (FRC) for CA with 2DL CCs

| Test num. | Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate- gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | 2x10 MHz | R.2 FDD | OP.1 FDD (NOTE 1) | EVA5 | 1x2 Low | 70 | -1.1 | ≥3 (NOTE 2) |
| 2 | 2x20 MHz | R.42 FDD | OP.1 FDD (NOTE 1) | EVA5 | 1x2 Low | 70 | -1.3 | ≥5 |
| 3 | 2x5 MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 | ≥2 |
|  |  |  | OP.1 FDD |  |  | 70 | -1.0 |  |
| 4 | 10MHz+5MHz | R.2 FDD for 10MHz CC | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 | ≥3 |
|  |  | R.42-2 FDD for 5MHz CC | OP.1 FDD |  |  | 70 | -1.0 |  |
| 5 | 15MHz+5MHz | R.42-3 FDD for 15MHz CC | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 | ≥3 |
|  |  | R.42-2 FDD for 5MHz CC | OP.1 FDD |  |  | 70 | -1.0 |  |
| NOTE 1: The OCNG pattern applies for each CC. NOTE 2: 30usec timing difference between two CCs is applied in inter-band CA case. NOTE 3: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

Table 8.2.1.1.1-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.3 |
| 3MHz | R.42-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.1 |
| 5MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |
| 15MHz | R.42-3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |

Table 8.2.1.1.1-6: Minimum performance (FRC) based on single carrier performance for CA with 3DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.1.1.1-7: Minimum performance (FRC) based on single carrier performance for CA with 4DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.1.1.1-8: Minimum performance (FRC) based on single carrier performance for CA with 5DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.1.1.1-9: Minimum performance (FRC) based on single carrier performance for CA with 6DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.1.1.1-10: Minimum performance (FRC) based on single carrier performance for CA with 7DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.1.1.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

##### 8.2.1.1.2	Void

##### 8.2.1.1.3	Void

##### 8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN

The requirements are specified in Table 8.2.1.1.4-2, with the addition of the parameters in Table 8.2.1.1.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the single-antenna performance with a single PRB allocated at the lower band edge in presence of MBSFN.

Table 8.2.1.1.4-1: Test Parameters for Testing 1 PRB allocation

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for MBSFN portion of MBSFN subframes (NOTE 2) |  |  | OCNG (NOTE 3) |
| PDSCH transmission mode |  |  | 1 |
| NOTE 1: NOTE 2: The MBSFN portion of an MBSFN subframe comprises the whole MBSFN subframe except the first two symbols in the first slot. NOTE 3: The MBSFN portion of the MBSFN subframes shall contain QPSK modulated data. Cell-specific reference signals are not inserted in the MBSFN portion of the MBSFN subframes, QPSK modulated MBSFN data is used instead. |  |  |  |

Table 8.2.1.1.4-2: Minimum performance 1PRB (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.29 FDD | OP.3 FDD | ETU70 | 1x2 Low | 30 | 2.0 | ≥1 |

##### 8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA

The requirements are specified in Table 8.2.1.1.4A-2, with the addition of the parameters in Table 8.2.1.1.4A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the single-antenna performance with a single PRB allocated at the lower band edge in presence of FeMBMS Unicast-mixed Cell under CA.

Table 8.2.1.1.4A-1: Test Parameters for Testing 1 PRB allocation under CA

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for MBSFN portion of MBSFN subframes (NOTE 2) |  |  | OCNG (NOTE 3) |
| PDSCH transmission mode in PCell and SCell |  |  | 1 |
| Numerology in the MBSFN subframe in SCell |  |  | 1.25kHz |
| NOTE 1: NOTE 2: The MBSFN portion of an MBSFN subframe comprises the whole MBSFN subframe. NOTE 3: The MBSFN portion of the MBSFN subframes shall contain QPSK modulated data. Cell-specific reference signals are not inserted in the MBSFN portion of the MBSFN subframes, QPSK modulated MBSFN data is used instead. |  |  |  |

Table 8.2.1.1.4A-2: Minimum performance 1PRB (FRC)

| Test number | Cell | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | PCell | 10 MHz | NA | OP.3 FDD | AWGN | 1x2 Low | NA | NA | ≥1 |
|  | SCell | 10MHz | R.29-1 FDD | OP.3A FDD | ETU70 | 1x2 Low | 30 | [2.9] | ≥1 |
| Note 1: If Test 1 in Table 8.2.1.1.4A-2 is tested, the test coverage can be considered fulfilled without executing Test 1 in Table 8.2.1.1.4-2. |  |  |  |  |  |  |  |  |  |

#### 8.2.1.2	Transmit diversity performance

##### 8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port

The requirements are specified in Table 8.2.1.2.1-2, with the addition of the parameters in Table 8.2.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.2.1.2.1-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| NOTE 1: . |  |  |  |

Table 8.2.1.2.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11 FDD | OP.1 FDD | EVA5 | 2x2 Medium | 70 | 6.8 | ≥2 |
|  | 5 MHz | R.11-2 FDD | OP.1 FDD | EVA5 | 2x2 Medium | 70 | 5.9 | 1 |
|  | 5 MHz (NOTE 1) | R.11-2 FDD | OP.1 FDD | EVA5 | 2x2 Medium | 70 | 5.9 | ≥2 |
| 2 | 10 MHz | R.10 FDD | OP.1 FDD | HST | 2x2 | 70 | -2.3 | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |  |

##### 8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port

The requirements are specified in Table 8.2.1.2.2-2, with the addition of the parameters in Table 8.2.1.2.2-1 and the downlink physical channel setup according Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC-FSTD) with 4 transmitter antennas.

Table 8.2.1.2.2-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| NOTE 1: . |  |  |  |

Table 8.2.1.2.2-2: Minimum performance Transmit Diversity (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4 MHz | R.12 FDD | OP.1 FDD | EPA5 | 4x2 Medium | 70 | 0.6 | ≥1 |
| 2 | 10 MHz | R.13 FDD | OP.1 FDD | ETU70 | 4x2 Low | 70 | -0.9 | ≥1 |

##### 8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

The requirements are specified in Table 8.2.1.2.3-2, with the addition of parameters in Table 8.2.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell. In Table 8.2.1.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.2.1.2.3-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (NOTE 1) | -3 |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (NOTE 2) | N/A |
|  |  | dBm/15kHz | -98 (NOTE 3) | N/A |
|  |  | dBm/15kHz | -94.8 (NOTE 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.1.2.3-2 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (NOTE 5) |  |  | N/A | 11000100 11000000 11000000 11000000 11000000 |
| RLM/RRM Measurement Subframe Pattern (NOTE 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A |
| CSI Subframe Sets (NOTE7) | CCSI,0 |  | 11000100 11000000 11000000 11000000 11000000 | N/A |
|  | CCSI,1 |  | 00111011 00111111 00111111 00111111 00111111 | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 2 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| NOTE 1: . NOTE 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. NOTE 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. NOTE 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS NOTE 5: ABS pattern as defined in [9]. NOTE 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] NOTE 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. NOTE 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. NOTE 9: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |

Table 8.2.1.2.3-2: Minimum Performance Transmit Diversity (FRC)

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (NOTE 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) NOTE 5 | SNR (dB) (Note 2) |  |
| 1 | R.11-4 FDD (NOTE 4) | OP.1 FDD | OP.1 FDD | EVA5 | EVA 5 | 2x2 Medium | 70 | 3.4 | ≥2 |
| NOTE 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. NOTE 2: SNR corresponds to of cell 1. NOTE 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. NOTE 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. NOTE 5: The maximum Throughput is calculated from the total Payload in 9 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |

##### 8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.2.1.2.3A-2, with the addition of parameters in Table 8.2.1.2.3A-1. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cells with CRS assistance information. In Table 8.2.1.2.3A-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.1.2.3A-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (NOTE 1) | -3 (NOTE 1) | -3 (NOTE 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (NOTE 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (NOTE 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (NOTE 4) | N/A | N/A |
|  |  | dB | Reference Value in Table8.2.1.2.3A-2 | 12 | 10 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (NOTE 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 | 11000000 11000000 11000000 11000000 11000000 |
| RLM/RRM Measurement Subframe Pattern (NOTE 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |
| CSI Subframe Sets (NOTE 7) | CCSI,0 |  | 11000000 11000000 11000000 11000000 11000000 | N/A | N/A |
|  | CCSI,1 |  | 00111111 00111111 00111111 00111111 00111111 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | NOTE 8 | NOTE 8 |
| PDSCH transmission mode |  |  | 2 | NOTE 9 | NOTE 9 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| NOTE 1: . NOTE 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10, #12, #13 of a subframe overlapping with the aggressor ABS. NOTE 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. NOTE 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS NOTE 5: ABS pattern as defined in [9]. NOTE 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] NOTE 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. NOTE 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. NOTE 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. NOTE 10: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. NOTE 11: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.1.2.3A-2: Minimum Performance Transmit Diversity (FRC)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (NOTE 1) |  |  | Correlation Matrix and Antenna Configuration (NOTE 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) NOTE 5 | SNR (dB) (NOTE 3) |  |
| 1 | R.11-4 FDD NOTE 4 | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 | EVA5 | EVA5 | 2x2 Medium | 70 | 3.4 | ≥2 |
| NOTE 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. NOTE 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. NOTE 3: SNR corresponds to of cell 1. NOTE 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. NOTE 5: The maximum Throughput is calculated from the total Payload in 9 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model

The requirements are specified in Table 8.2.1.2.4-2, with the addition of parameters in Table 8.2.1.2.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two dominant interfering cells applying transmission mode 3 interference model defined in clause B.5.2. In Table 8.2.1.2.4-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.1.2.4-1: Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (NOTE 1) | -3 | -3 |
|  |  | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| DIP (NOTE 2) |  | dB | N/A | -2.23 | -8.06 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 2 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.2 | As specified in clause B.5.2 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Reporting interval |  | ms | 5 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-0 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 5) | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 2 | N/A | N/A |
| NOTE 1: NOTE 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. NOTE 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. NOTE 4: Cell 2 transmission is delayed with respect to Cell 1 by 0.33 ms and Cell 3 transmission is delayed with respect to Cell 1 by 0.67 ms. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 and #0. |  |  |  |  |  |

Table 8.2.1.2.4-2: Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (NOTE 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SINR (dB) (NOTE 2) |  |
| 1 | R.46 FDD | OP.1 FDD | N/A | N/A | EVA70 | EVA70 | EVA70 | 2x2 Low | 70 | -1.1 | ≥1 |
| NOTE 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. NOTE 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. NOTE 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model

The requirements are specified in Table 8.2.1.2.5-2, with the addition of parameters in Table 8.2.1.2.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 2 interference model defined in clause B.6.1. In Table 8.2.1.2.5-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.1.2.5-1: Test Parameters for Transmit Diversity Performance (FRC) with TM2 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 6 | 1 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 2 | 2 | 2 |
| Interference model |  |  |  | N/A | As specified in clause B.6.1 | As specified in clause B.6.1 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.2.1.2.5-2: Minimum Performance for Enhanced Performance Requirement Type B, Transmit Diversity (FRC) with TM2 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-10 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 15.5 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model

The requirements are specified in Table 8.2.1.2.6-2, with the addition of parameters in Table 8.2.1.2.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 9 interference model defined in clause B.6.4. In Table 8.2.1.2.6-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.1.2.6-1: Test Parameters for Transmit Diversity Performance (FRC) with TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | 0 | 0 |
|  |  |  | dB | -3 (Note 1) | 0 | 0 |
|  |  |  | dB | 0 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 3.28 | 0.74 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | Random from set {1,2,3} | Random from set {1,2,3} |
| PDSCH transmission mode |  |  |  | 2 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | N/A | Antenna ports 15,16 | Antenna ports 15,16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 10 / 1 | 10 / 1 |
| CSI reference signal configuration |  |  |  | N/A | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | N/A | 6 / 0100000000000000 | 6 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 5 | -5 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 600 | -600 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.2.1.2.6-2: Minimum Performance for Enhanced Performance Requirement Type B, Transmit Diversity (FRC) with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-9 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 8.4 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.1.2.7-2, with the addition of the parameters in Table 8.2.1.2.7-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance of transmit diversity (SFBC) with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.1.2.7-1: Test Parameters for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| MUSTIdx (Note 2) |  |  | 11 |
| p-a-must-r14 (Note 3) |  |  | N/A |
| Note 1: . Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |

Table 8.2.1.2.7-2: Minimum Performance for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.aa FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | 14.1 | ≥1 |

#### 8.2.1.3	Open-loop spatial multiplexing performance

##### 8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port

For single carrier, the requirements are specified in Table 8.2.1.3.1-2, with the addition of the parameters in Table 8.2.1.3.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CC, the requirements are specified in Table 8.2.1.3.1-4, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.1.3.1-6, based on single carrier requirement specified in Table 8.2.1.3.1-5, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.1.3.1-7, based on single carrier requirement specified in Table 8.2.1.3.1-5, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.1.3.1-8, based on single carrier requirement specified in Table 8.2.1.3.1-5, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are specified in Table 8.2.1.3.1-9, based on single carrier requirement specified in Table 8.2.1.3.1-5, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are specified in Table 8.2.1.3.1-10, based on single carrier requirement specified in Table 8.2.1.3.1-5, with the addition of the parameters in Table 8.2.1.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.1.3.1-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: Void. NOTE 3: Void. |  |  |  |

Table 8.2.1.3.1-2: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 (NOTE 4) | 10 MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.0 | ≥2 |
| 2 (NOTE 3) | 5 MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.7 | ≥2 |
| 3 | 10 MHz | R.35 FDD | OP.1 FDD | EVA200 | 2x2 Low | 70 | 20.2 | ≥2 |
| 4 | 10 MHz | R.35-4 FDD | OP.1 FDD | ETU600 | 2x2 Low | 70 | 20.8 | ≥2 |
| NOTE 1: Void. NOTE 2: Test 1 may not be executed for UE-s for which Test 1 or 2 in Table 8.2.1.3.1-4 is applicable. NOTE 3: Test case applicability is defined in 8.1.2.1. NOTE 4: For UE that supports CRS interference handling, the CRS assistance information defined in [7] is provided. The CRS assistance information includes two aggressor cells with 2 CRS ports and cell ID of agressor cells are 1 and 128. For UE that does not support CRS interference handling, CRS assistance information is not provided. |  |  |  |  |  |  |  |  |

Table 8.2.1.3.1-3: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.3.1-4: Minimum performance Large Delay CDD (FRC) for CA with 2DL CCs

| Test num | Bandwidth | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 (NOTE 2) | 2x10 MHz | R.11 FDD | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.7 | ≥3 |
| 2 (NOTE 2) | 2x20 MHz | R.30 FDD | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.2 | ≥5 |
| 3 | 2x5 MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.7 | ≥2 |
| 4 | 10MHz+5MHz | R.11 FDD for 10MHz CC, | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.0 | ≥3 |
|  |  | R.11-2 FDD for 5MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 12.7 |  |
| 5 | 15MHz+5MHz | R.11-7 FDD for 15MHz CC | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 12.8 | ≥3 |
|  |  | R.11-2 FDD for 5MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 12.7 |  |
| NOTE 1: The OCNG pattern applies for each CC. NOTE 2: Void NOTE 3: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

Table 8.2.1.3.1-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3MHz | R.11-6 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10 MHz | R.11 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP. 1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.2.1.3.1-6: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥5 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.3.1-7: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.3.1-8: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.3.1-9: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.3.1-10: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.1.3.1-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

##### 8.2.1.3.1A	Soft buffer management test

For CA, the requirements are specified in Table 8.2.1.3.1A-2, with the addition of the parameters in Table 8.2.1.3.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the UE performance with proper instantaneous buffer implementation. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.2.1.3.1A-3.

Table 8.2.1.3.1A-1: Test Parameters for soft buffer management test (FRC) for CA

| Parameter |  | Unit | Test 1-7 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: For CA test cases, PUCCH format 1b with channel selection is used to feedback ACK/NACK. NOTE 3: For CA test cases, the same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.3.1A-2: Minimum performance soft buffer management test (FRC) for CA

| Test num | Bandwidth | Reference channel | OCNG pattern | Propa- gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |
| 1 | 2x20 MHz | R.30 FDD | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.2 |
| 2 | 15MHz + 10MHz | R.35-2 FDD for 15MHz CC | OP.1 FDD (NOTE 1) | EVA5 | 2x2 Low | 70 | 15.1 |
|  |  | R.35-3 FDD for 10MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 15.1 |
| 3 | 20MHz + 10MHz | R.30 FDD for 20MHz CC | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.5 |
|  |  | R.11 FDD for 10MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 13.5 |
| 4 | 20MHz + 15MHz | R.30 FDD for 20MHz CC | OP.1 FDD (NOTE 1) | EVA70 | 2x2 Low | 70 | 13.5 |
|  |  | R.30-1 FDD for 15MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 13.5 |
| 5 | 2x20 MHz | R.35-1 FDD | OP.1 FDD (NOTE 1) | EVA5 | 2x2 Low | 70 | 15.8 |
| 6 | 20MHz + 10MHz | R.35-1 FDD for 20MHz CC | OP.1 FDD (NOTE 1) | EVA5 | 2x2 Low | 70 | 15.9 |
|  |  | R.35-3 FDD for 10MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 15.9 |
| 7 | 20MHz + 15MHz | R.35-1 FDD for 20MHz CC | OP.1 FDD (NOTE 1) | EVA5 | 2x2 Low | 70 | 15.9 |
|  |  | R.35-2 FDD for 15MHz CC | OP.1 FDD (NOTE 1) |  |  | 70 | 15.9 |
| NOTE 1: For CA test cases, the OCNG pattern applies for each CC. NOTE 2: For Test 2, 3, 4, 6, 7 the Fraction of maximum Throughput applies to each CC. NOTE 3: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |

Table 8.2.1.3.1A-3: Test points for soft buffer management tests for CA

| UE category | Bandwidth combination with maximum aggregated bandwidth (NOTE 1) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2x20MHz | 15MHz+10MHz | 20MHz+10MHz | 20MHz+15MHz |
| 3 | 1 | 2 | 3 | 4 |
| 4 | 5 | N/A | 6 | 7 |
| NOTE 1: Maximum over all supported CA configurations and bandwidth combination sets according to Table 5.6A.1-1and Table 5.6A.1-2. |  |  |  |  |

##### 8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports

The requirements are specified in Table 8.2.1.3.1B-2, with the addition of the parameters in Table 8.2.1.3.1B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

Table 8.2.1.3.1B-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.1.3.1B-2: Enhanced Performance Requirement Type C for Large Delay CDD (FRC)

| Test num | Bandwidth | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Medium | 70 | 17.8 | ≥2 |

##### 8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference

The requirements are specified in Table 8.2.1.3.1C-2, with the addition of parameters in Table 8.2.1.3.1C-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell with transmission mode 1. In Table 8.2.1.3.1C-1, Cell 1 is the serving cell, and Cell 2 is interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.1.3.1C-1 Test parameters for Larger Delay CDD (FRC) with TM1 interference

| Parameter |  | Unit | Cell 1 |  | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |
| Downlink power allocation |  | dB | -3 | 0 |  |
|  |  |  | -3 (NOTE 1) | 0 |  |
|  |  |  | 0 | 0 |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0 |  |
| Cyclic Prefix |  |  | Normal | Normal |  |
| Cell ID |  |  | 0 | 1 |  |
| Transmission mode |  |  | 3 | NOTE 2 |  |
| at antenna port |  | dBm/15kHz | -98 | N/A |  |
| (NOTE 3) |  | dB | Reference Value in Table 8.2.1.3.1C-2 | 12.95 |  |
| Correlation and antenna configuration |  |  | Medium (2x2) | Medium(1x2) |  |
| Number of OFDM symbols for PDCCH |  |  | 2 | N/A |  |
| Max number of HARQ transmissions |  |  | 4 | N/A |  |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |  |
| NOTE 1: NOTE 2: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.2 applying OCNG pattern OP.5 FDD as defined in Annex A.5.1.5. NOTE 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. NOTE 4: All cells are time-synchronous. NOTE 5: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |  |

Table 8.2.1.3.1C-2 Enhanced Performance Requirement Type C, Larger Delay CDD (FRC) with TM1 interference

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (NOTE 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (NOTE 2) |  |
| 1 | R.11-8 FDD | OP.1 FDD | OP.5 FDD | EVA70 | EVA70 | 70 | 19.9 | ≥2 |
| NOTE 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. NOTE 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

##### 8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port

The requirements are specified in Table 8.2.1.3.2-2, with the addition of the parameters in Table 8.2.1.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 4 transmitter antennas.

Table 8.2.1.3.2-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (NOTE 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: |  |  |  |

Table 8.2.1.3.2-2: Minimum performance Large Delay CDD (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.14 FDD | OP.1 FDD | EVA70 | 4x2 Low | 70 | 14.3 | ≥2 |

##### 8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

The requirements for non-MBSFN ABS are specified in Table 8.2.1.3.3-2, with the addition of parameters in Table 8.2.1.3.3-1 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3.

The requirements for MBSFN ABS are specified in Table 8.2.1.3.3-4, with the addition of parameters in Table 8.2.1.3.3-3 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3.

The purpose is to verify the performance of large delay CDD with 2 transmitter antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell. In Tables 8.2.1.3.3-1 and 8.2.1.3.3-3, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.2.1.3.3-1: Test Parameters for Large Delay CDD (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (NOTE 1) | -3 |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (NOTE 2) | N/A |
|  |  | dBm/15kHz | -98 (NOTE 3) | N/A |
|  |  | dBm/15kHz | -94.8 (NOTE 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.1.3.3-2 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Cell Id |  |  | 0 | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| ABS pattern (NOTE 5) |  |  | N/A | 11000100, 11000000, 11000000, 11000000, 11000000 |
| RLM/RRM Measurement Subframe Pattern(NOTE 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A |
| CSI Subframe Sets (NOTE 7) | CCSI,0 |  | 11000100 11000000 11000000 11000000 11000000 | N/A |
|  | CCSI,1 |  | 00111011 00111111 00111111 00111111 00111111 | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 3 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| NOTE 1: . NOTE 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. NOTE 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. NOTE 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS NOTE 5: ABS pattern as defined in [9]. NOTE 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. NOTE 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. NOTE 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. NOTE 9: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |

Table 8.2.1.3.3-2: Minimum Performance Large Delay CDD (FRC) – Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 2) |  |
| 1 | R.11 FDD Note 4 | OP.1 FDD | OP.1 FDD | EVA 5 | EVA 5 | 2x2 Low | 70 | 13.3 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 9 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |

Table 8.2.1.3.3-3: Test Parameters for Large Delay CDD (FRC) – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -94.8 (Note 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.1.3.3-4 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN |
| Cell Id |  |  | 0 | 126 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| ABS pattern (Note 5) |  |  | N/A | 0001000000 0100000010 0000001000 0000000000 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0001000000 0100000010 0000001000 0000000000 | N/A |
| CSI Subframe Sets (Note 7) | CCSI,0 |  | 0001000000 0100000010 0000001000 0000000000 | N/A |
|  | CCSI,1 |  | 1110111111 1011111101 1111110111 1111111111 | N/A |
| MBSFN Subframe Allocation (Note 10) |  |  | N/A | 001000 100001 000100 000000 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 3 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbol #0 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 5: ABS pattern as defined in [9]. The 4th, 12th, 19th and 27th subframes indicated by ABS pattern are MBSFN ABS subframes. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 9: SIB-1 will not be transmitted in Cell2 in this test. Note 10: MBSFN Subframe Allocation as defined in [7], four frames with 24 bits is chosen for MBSFN subframe allocation. Note 11: The maximum number of uplink HARQ transmission is ≤ 2 so that each PHICH channel transmission is in a subframe protected by MBSFN ABS in this test. |  |  |  |  |

Table 8.2.1.3.3-4: Minimum Performance Large Delay CDD (FRC) – MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 2) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 2) |  |
| 1 | R.11 FDD Note 4 | OP.1 FDD | OP.1 FDD | EVA 5 | EVA 5 | 2x2 Low | 70 | 12.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 4 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |

##### 8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements for non-MBSFN ABS are specified in Table 8.2.1.3.4-2, with the addition of parameters in Table 8.2.1.3.4-1. The purpose is to verify the performance of large delay CDD with 2 transmit antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cells with CRS assistance information. In Table 8.2.1.3.4-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 ad Cell3.

Table 8.2.1.3.4-1: Test Parameters for Large Delay CDD (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.1.3.4-2 | Reference Value in Table 8.2.1.3.4-2 | Reference Value in Table 8.2.1.3.4-2 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 1 | 126 |
| ABS pattern (Note 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 | 11000000 11000000 11000000 11000000 11000000 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 11000000 11000000 11000000 11000000 11000000 | N/A | N/A |
|  | CCSI,1 |  | 00111111 00111111 00111111 00111111 00111111 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| PDSCH transmission mode |  |  | 3 | Note 9 | Note 9 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 11: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.1.3.4-2: Minimum Performance Large Delay CDD (FRC) – Non-MBSFN ABS

| Test Number | Reference Channel |  |  | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 3) |  |
| 1 | R.11 FDD Note 4 | 9 | 7 | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 13.9 | ≥2 |
| 2 | R.35 FDD Note 4 | 9 | 1 | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 22.6 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 9 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.1.3.5-2, with the addition of the parameters in Table 8.2.1.3.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance  of open-loop spatial multiplexing with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.1.3.5-1: Test Parameters for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
|  | PDSCH_RA | dB | -3 |
|  | PDSCH_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| MUSTIdx (Note 2) |  |  | 10 |
| p-a-must-r14 (Note 3) |  |  | -3 |
| Note 1: . Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |

Table 8.2.1.3.5-2: Minimum Performance for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.bb FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | 19.3 | ≥2 |

##### 8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.1.3.6-2, with the addition of parameters in Table 8.2.1.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.1.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.1.3.6-1: Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  |  | -3 (Note 1) | 0 |
|  |  |  | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0,1 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Transmission mode |  |  | 3 | NA (Note 4) |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| (Note 2) |  | dB | Reference Value in Table 8.2.1.3.6-2 | 10 |
| Correlation and antenna configuration |  |  | Low (2x2) | Low (2x2) |
| Max number of HARQ transmissions |  |  | 4 | N/A |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  | Disabled | Enabled |
| CRS transmission |  |  | On | CRS is configured with CRS muting pattern as ‘00000011111111111000’ where every 20 subframes consist of 11 subframes with full system BW CRS and 9 subframes with CRS only on the center 6 PRBs (Note 5) |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: In the muting pattern ‘00000011111111111000’, 1 indicates DL subframes with full system BW CRS configured and 0 indicates DL subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |

Table 8.2.1.3.6-2: Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-13 FDD | OP.1 FDD | NA | EVA5 | EVA5 | 70 | 15.1 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

#### 8.2.1.4	Closed-loop spatial multiplexing performance

##### 8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.2.1.4.1-2, with the addition of the parameters in Table 8.2.1.4.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband and frequency selective precoding.

Table 8.2.1.4.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit |  | Test 1 |  | Test 1A |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  | -3 |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  | -3 (Note 1) |  | -3 (Note 1) |  |
|  |  |  | dB |  | 0 |  | 0 |  | 0 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Precoding granularity |  |  | PRB |  | 6 |  | 4 |  | 50 |  |
| PMI delay (Note 2) |  |  | ms |  | 8 |  | 8 |  | 8 |  |
| Reporting interval |  |  | ms |  | 1 |  | 1 |  | 1 |  |
| Reporting mode |  |  |  |  | PUSCH 1-2 |  | PUSCH 1-2 |  | PUSCH 3-1 |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 001111 |  | 001111 |  | 001111 |  |
| PDSCH transmission mode |  |  |  |  | 4 |  | 4 |  | 4 |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |  |  |  |  |  |

Table 8.2.1.4.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.10 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | -2.5 | ≥1 |
| 1A (Note 1) | 5 MHz | R.10-2 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | -2.9 | ≥1 |
| 2 | 10 MHz | R.10 FDD | OP.1 FDD | EPA5 | 2x2 High | 70 | -2.3 | ≥1 |
| Note 1: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |  |

##### 8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port

The requirements are specified in Table 8.2.1.4.1A-2, with the addition of the parameters in Table 8.2.1.4.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband and frequency selective precoding.

Table 8.2.1.4.1A-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 6 | 50 |
| PMI delay (Note 2) |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 1-2 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000000000000000000001111111111111111 | 0000000000000000000000000000000000000000000000001111111111111111 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |

Table 8.2.1.4.1A-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz | R.13 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | -3.2 | ≥1 | ≥4 |
| 2 | 10MHz 1024QAM | R.101 FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | 25.1 | TBD | 20, ≥ 22 |

##### 8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model

The requirements are specified in Table 8.2.1.4.1B-2, with the addition of the parameters in Table 8.2.1.4.1B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two dominant interfering cells applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.2.1.4.1B-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.1.4.1B-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 | -3 |
|  |  | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 | -8.66 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 6 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Precoding granularity |  | PRB | 50 | 6 | 6 |
| PMI delay (Note 4) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 5 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 1111 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 6) | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 2 | N/A | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 and #0. |  |  |  |  |  |

Table 8.2.1.4.1B-2: Enhanced Performance Requirement Type A, Single-Layer Spatial Multiplexing (FRC) with TM4 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.47 FDD | OP.1 FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 0.8 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.2.1.4.1C-2, with the addition of parameters in Table 8.2.1.4.1C-1. The purpose is to verify the closed loop rank-one performance with wideband precoding if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.2.1.4.1C-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.1.4.1C-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.1.4.1C-2 | 12 | 10 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 | 11000000 11000000 11000000 11000000 11000000 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 11000000 11000000 11000000 11000000 11000000 | N/A | N/A |
|  | CCSI,1 |  | 00111111 00111111 00111111 00111111 00111111 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| PDSCH transmission mode |  |  | 6 | Note 9 | Note 9 |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 10) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 | N/A | N/A |
| Peporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 1111 | N/A | N/A |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 11: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 12: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.1.4.1C-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)– Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 3) |  |
| 1 | R.11 FDD Note 4 | OP.1FDD | OP.1FDD | OP.1FDD | EPA5 | EPA5 | EPA5 | 2x2 High | 70 | 6.1 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 9 subframes, averaged over 40ms. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model

The requirements are specified in Table 8.2.1.4.1D-2, with the addition of the parameters in Table 8.2.1.4.1D-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 4 interference model defined in clause B.6.3. In Table 8.2.1.4.1D-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.1.4.1D-1: Test Parameters for Single-layer Spatial Multiplexing (FRC) with TM4 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 |  | -3 |  |
|  |  |  | dB | -3 (Note 1) | -3 |  | -3 |  |
|  |  |  | dB | 0 | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |  |  |
| Test number (Note 4) |  |  |  |  | Test 1 | Test 2 | Test 1 | Test 2 |
|  |  |  | dB | N/A | 13.91 | 3.28 | 3.34 | 0.74 |
| Cell Id |  |  |  |  | 6 | 1 | 1 | 6 |
| CFI indicated in PCFICH |  |  |  |  | 3 | Random from set {1,2,3} | 3 | Random from set {1,2,3} |
| BWChannel |  |  | MHz | 10 | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  | Normal | Normal |  | Normal |  |
| Number of control OFDM symbols |  |  |  | 3 | 3 |  | 3 |  |
| PDSCH transmission mode |  |  |  | 4 | 4 |  | 4 |  |
| Interference model |  |  |  | N/A | As specified in clause B.6.3 |  | As specified in clause B.6.3 |  |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.3 |  | As specified in clause B.6.3 |  |
| Time offset to cell 1 |  |  | us | N/A | 2 |  | 3 |  |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 |  | 300 |  |
| MBSFN |  |  |  | Not configured | Not configured |  | Not configured |  |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} |  | {dB-6, dB-3, dB0} |  |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} |  | {2,3,4,8,9} |  |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. Note 4: Test 1 and Test 2 are defined in Table 8.2.1.4.1D-2. |  |  |  |  |  |  |  |  |

Table 8.2.1.4.1D-2: Minimum Performance for Enhanced Performance Requirement Type B, Single-layer Spatial Multiplexing (FRC) with TM4 interference model

| Test Num | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-10 FDD | OP.1 FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 85 | 17.0 | ≥1 |
| 2 | R.11-9 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 10.1 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.1.4.1E-2, with the addition of parameters in Table 8.2.1.4.1E-1. The purpose is to verify the closed loop rank-one performance with wideband precoding when CRS assistance information [7] is configured. In Table 8.2.1.4.1E-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.1.4.1E-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  |  | dB | 0 | 0 | 0 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.2.1.4.1E-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset to Cell 1 |  |  | s | N/A | 3 | -1 |
| Frequency shift to Cell 1 |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 4 | N/A | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  |  | ms | 8 | N/A | N/A |
| Reporting interval |  |  | ms | 1 | N/A | N/A |
| Peporting mode |  |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |  |

Table 8.2.1.4.1E-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.10-3 FDD | OP.1FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 low | 70 | 10.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/NocEs/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.1.4.1F-2, with the addition of parameters in Table 8.2.1.4.1F-1. In Table 8.2.1.4.1F-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.2.1.4.1F-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.1.4.1F-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 000000000000FFFF | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |

Table 8.2.1.4.1F-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.36 FDD | OP.1FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 4x2 low | 70 | 14.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.2.1.4.2-2,with the addition of the parameters in Table 8.2.1.4.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

Table 8.2.1.4.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1-2 | Test 2A | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| Precoding granularity |  | PRB | 50 | 25 | 6 |
| PMI delay (Note 2) |  | ms | 8 | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 110000 | 110000 | 110000 |
| PDSCH transmission mode |  |  | 4 | 4 | 4 |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbol | 2 | 3 | 1 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |

Table 8.2.1.4.2-2: Minimum performance Multi-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz | R.35 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 18.9 | ≥2 | ≥6 |
| 2 | 10 MHz | R.11 FDD | OP.1 FDD | ETU70 | 2x2 Low | 70 | 14.3 | ≥2 | ≥6 |
| 2A (Note 1) | 5 MHz | R.11-2 FDD | OP.1 FDD | ETU70 | 2x2 Low | 70 | 14.0 | ≥2 | ≥6 |
| 3 | 10MHz 256QAM | R. 65 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | 25.3 | 11-12 | ≥11 |
| Note 1: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |  |  |

##### 8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports

The requirements are specified in Table 8.2.1.4.2A-2,with the addition of the parameters in Table 8.2.1.4.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband precoding.

Table 8.2.1.4.2A-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 50 |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 110000 |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.2.1.4.2A-2: Enhanced Performance Requirement Type C for Multi-Layer Spatial Multiplexing with TM4 (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11 FDD | OP.1 FDD | ETU70 | 2x2 Medium | 70 | 18.3 | ≥2 |

##### 8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port

For single carrier, the requirements are specified in Table 8.2.1.4.3-2, with the addition of the parameters in Table 8.2.1.4.3-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.1.4.3-4, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.1.4.3-6, based on single carrier requirement specified in Table 8.2.1.4.3-5, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with4 DL CCs, the requirements are specified in Table 8.2.1.4.3-7, based on single carrier requirement specified in Table 8.2.1.4.3-5, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.1.4.3-8, based on single carrier requirement specified in Table 8.2.1.4.3-5, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are specified in Table 8.2.1.4.3-9, based on single carrier requirement specified in Table 8.2.1.4.3-5, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are specified in Table 8.2.1.4.3-10, based on single carrier requirement specified in Table 8.2.1.4.3-5, with the addition of the parameters in Table 8.2.1.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.1.4.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Void. Note 4: Void. Note 5: Void. |  |  |  |

Table 8.2.1.4.3-2: Minimum performance Multi-Layer Spatial Multiplexing (FRC)

| Test num. | Band-width | Referencechannel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate- gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.36 FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | 14.7 | ≥2 |
| Note 1: Void. |  |  |  |  |  |  |  |  |

Table 8.2.1.4.3-3: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.2.1.4.3-4, and with PUCCH format 3 for Tests in Table 8.2.1.4.3-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.4.3-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for CA with 2DL CCs

| Test num. | Band-width | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate- gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | 2x10 MHz | R.14 FDD | OP.1 FDD (Note 1) | EVA5 | 4x2 Low | 70 | 10.8 | ≥3 |
| 2 | 2x20 MHz | R.14-3 FDD | OP.1 FDD (Note 1) | EVA5 | 4x2 Low | 70 | 10.9 | ≥5 |
| 3 | 2x5 MHz | R.14-6 FDD | OP.1 FDD (Note 1) | EVA5 | 4x2 Low | 70 | 9.5 | ≥2 |
|  |  |  | OP.1 FDD (Note 1) |  |  | 70 | 9.5 |  |
| 4 | 10MHz+5MHz | R.14 FDD for 10MHz CC | OP.1 FDD (Note 1) | EVA5 | 4x2 Low | 70 | 10.1 | ≥3 |
|  |  | R.14-6 FDD for 5MHz CC | OP.1 FDD (Note 1) |  |  | 70 | 9.5 |  |
| 5 | 15MHz+5MHz | R.14-7 FDD for 15MHz CC | OP.1 FDD (Note 1) | EVA5 | 4x2 Low | 70 | 10.1 | ≥3 |
|  |  | R.14-6 FDD for 5MHz CC | OP.1 FDD (Note 1) |  |  | 70 | 9.5 |  |
| NOTE 1: The OCNG pattern applies for each CC. NOTE 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

Table 8.2.1.4.3-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.1.4.3-6: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.4.3-7: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.1.4.3-5 per CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.2.1. 4.3-5 per CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.2.1.3.1-5 per CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.4.3-8: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.4.3-9: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.1.4.3-10: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.1.4.3-5 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

##### 8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.2.1.4.3A-3 for 2DL CCs and Table 8.2.1.4.3A-4 for 3DL CCs, based on single carrier requirement specified in Table 8.2.1.4.3A-2, with the addition of the parameters in Table 8.2.1.4.3A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity transmission.

Table 8.2.1.4.3A-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 4) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: The same PDSCH transmission mode is applied to each component carrier. Note 4: As defined in TS36.300 [11]. Note 5: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.2.1.4.3A-2: Single carrier performance for multiple dual connectivity configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.36 |
| 3MHz | R.14-5 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.1.4.3A-3: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity with 2 DL CCs

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 2 | 15+20 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 3 | 10+20MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 4 | 2x15 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 5 | 2x10 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥3 |
| 6 | 15+5 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥3 |
| 7 | 10+15 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

Table 8.2.1.4.3A-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity with 3DL CCs

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 20+20+15MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 2 | 20+15+15MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 3 | 3x20 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 4 | 20+20+10 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 5 | 20+15+10 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 6 | 20+10+10 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| 7 | 15+15+10 MHz | As specified in Table 8.2.1.4.3A-2 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

##### 8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.1.4.4-2, with the addition of the parameters in Table 8.2.1.4.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance of closed-loop spatial multiplexing with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.1.4.4-1: Test Parameters for Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
|  | PDSCH_RA | dB | -3 |
|  | PDSCH_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 4 |
| Precoding |  |  | Random wideband precoding per TTI |
| MUSTIdx for transport block 1 (Note 2) |  |  | 10 |
| MUSTIdx for transport block 2 (Note 2) |  |  | 00 |
| p-a-must-r14 (Note 3) |  |  | -3 |
| Note 1: . Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |

Table 8.2.1.4.4-2: Minimum Performance for Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.bb FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | 17.3 | ≥2 |

#### 8.2.1.5	MU-MIMO

#### 8.2.1.6	[Control channel performance: D-BCH and PCH]

#### 8.2.1.7	Carrier aggregation with power imbalance

For CA, the requirements in this section verify the ability of an intraband adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

##### 8.2.1.7.1	Minimum Requirement

The requirements are specified in Table 8.2.1.7.1-2, with the addition of the parameters in Table 8.2.1.7.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.1.7.1-1: Test Parameters for CA

| Parameter |  |  | Unit | Test 1 | Test 2-3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  |  | dB | 0 | 0 |
| at antenna port |  |  | dBm/15kHz | Off (Note 2) | Off (Note 2) |
| Symbols for unused PRBs |  |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Modulation |  |  |  | 64 QAM | 64 QAM |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 |
| Redundancy version coding sequence |  |  |  | {0} | {0} |
| PDSCH transmission mode of PCell |  |  |  | 1 | 3 |
| PDSCH tramsmission mode of SCell |  |  |  | 3 | 1 |
| OCNG Pattern |  | PCell |  | OP.1 FDD | OP.5 FDD |
|  |  | SCell |  | OP.5 FDD | OP.1 FDD |
| Propagation Conditions |  | PCell |  | Clause B.1 | Clause B.1 |
|  |  | SCell |  | Clause B.1 | Clause B.1 |
| Correlation Matrix and Antenna |  | PCell |  | 1x2 | 2x2 |
|  |  | SCell |  | 2x2 | 1x2 |
| Note 1: for 1x2 and for 2x2 antenna configuration. Note 2: No external noise sources are applied Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated. pseudo random data. Note 4: Void |  |  |  |  |  |

Table 8.2.1.7.1-2: Minimum performance (FRC) for CA

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/15KHz) |  | Reference value Fraction of Maximum Throughput (%) |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |  |
| 1 | 20 | 20 | R.49 FDD | NA | -85 | -79 | 85 | NA | ≥5 |
| 2 | 10 | 10 | NA | R.49-1 FDD | -79 | -85.8 | NA | 85 | ≥5 |
| 3 | 5 | 5 | NA | R.49-2 FDD | -79 | -85.9 | NA | 85 | ≥5 |
| Note 1: The OCNG pattern for PCell is used to fill the control channel. The OCNG pattern for SCell is used to fill the control channel and PDSCH. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |  |

#### 8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset

The requirements in this section verify the ability of an intraband non-contiguous carrier aggregation UE to demodulate the signal transmitted by the PCell and SCell in the presence of timing offset between the cells. Throughput is measured on both cells.

##### 8.2.1.8.1	Minimum Requirement

For CA the requirements are specified in Table 8.2.1.8.1-2, with the addition of the parameters in Table 8.2.1.8.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.1.8.1-1: Test Parameters for CA

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Modulation |  |  | 64 QAM |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,0,1,2} |
| PDSCH transmission mode of PCell |  |  | 3 |
| PDSCH tramsmission mode of SCell |  |  | 3 |
| Note 1: . Note 2: The OCNG pattern is used to fill unused control channel and PDSCH. |  |  |  |

Table 8.2.1.8.1-2: Minimum performance (FRC) for CA

| Test Number | Cell | Band-width | Reference Channel | OCNG Pattern | Propagation Conditions | Correlation Matrix and Antenna | Refence value |  | Timing relative to PCell (µs) | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | PCell | 10MHz | R.35-4 FDD | OP.1 FDD | EPA200 | 2x2 Low | 70 | 21.15 | N/A | ≥3 |
|  | SCell | 10MHz | R.35-3 FDD |  | EPA200 | 2x2 Low | 60 | 15.18 | -30.26 |  |
| Note 1: The EPA200 propagation channels applied to PCell and SCell are statistically independent. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3. |  |  |  |  |  |  |  |  |  |  |

#### 8.2.1.9	HST-SFN performance

##### 8.2.1.9.1	Minimum Requirement

The purpose of this test is to verify UE performance in the HST-SFN scenario defined in B.3A when highSpeedEnhancedDemodulationFlag [7] is received.

For single carrier, the requirements are specified in Table 8.2.1.9.1-2, with the addition of the parameters in Table 8.2.1.9.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CC, the requirements are specified in Table 8.2.1.9.1-5, based on single carrier requirement speicified in Table 8.2.1.9.1-4, with the addition of the parameters in Table 8.2.1.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.1.9.1-6, based on single carrier requirement specified in Table 8.2.1.9.1-4, with the addition of the parameters in Table 8.2.1.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.1.9.1-7, based on single carrier requirement specified in Table 8.2.1.9.1-4, with the addition of the parameters in Table 8.2.1.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.1.9.1-8, based on single carrier requirement specified in Table 8.2.1.9.1-4, with the addition of the parameters in Table 8.2.1.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.1.9.1-1: Test Parameters for UE performance in HST-SFN scenario (FRC)

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.1.9.1-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.87 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | 13.3 | ≥1 |
| NOTE 1: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

Table 8.2.1.9.1-3: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.9.1-4: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.9] |
| 10 MHz | R.87 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.3] |
| 15 MHz | R.87-3 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.9] |
| 20 MHz | R.87-4 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [14.1] |

Table 8.2.1.9.1-5: Minimum performance Large Delay CDD (FRC) for CA with 2DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 2 | 2x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 3 | 2x5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 4 | 10MHz+5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 5 | 15MHz+5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |

Table 8.2.1.9.1-6: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

Table 8.2.1.9.1-7: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.2.1.9.1-4 per CC | ≥8 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

Table 8.2.1.9.1-8: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.2.1.9.1-4 per CC | 8, ≥11 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

##### 8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.2.1.9.2-2, with the addition of the parameters in Table 8.2.1.9.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhDemodFlag2-r16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhDemodFlag2-r16 [7] is not received. HST-500 test is not applicable to UE that has passed HST-SFN-500 test.

Table 8.2.1.9.2-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.1.9.2-2: Minimum performance UE in HST-SFN-500 and HST-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 FDD | OP.1 FDD | HST-SFN-500 | 2x2 | 70 | 9.7 | ≥ 1 |
| 2 | 10 MHz | R.87 FDD | OP.1 FDD | HST-500 | 2x2 | 70 | 11.4 | ≥ 1 |
| NOTE 1: Test case applicability is defined in 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

#### 8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing

The requirements in this section verify the ability of an UE supporting intra-band contiguous carrier aggregation with minimum channel spacing to demodulate the signal transmitted by the PCell and SCell(s). Throughput is measured on each cell. The minimum channel spacing of intra-band contiguous carrier aggregation refers to the possible minimum channel spacing as any multiple of 300 kHz less than the nominal channel spacing defined in 5.7.1A.

#### 8.2.1.10.1	Minimum Requirement

For CA the requirements are specified in Table 8.2.1.10.1-2, with the addition of the parameters in Table 8.2.1.10.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.1.10.1-1: Test Parameters for CA

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | 64QAM |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection |
| PDSCH transmission mode |  |  | 1 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.10.1-2: Single carrier performance for multiple CA configurations with minimum channel spacing

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.6 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.4 |
| 10MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 |
| 15MHz | R.8 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 |
| 20MHz | R.9 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.6 |

Table 8.2.1.10.1-3: Minimum performance (FRC) for intra-band CA with minimum channel spacing for CA with 2DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 20MHz+20MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 3 | 20MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 4 | 20MHz+5MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 5 | 15MHz+15MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 6 | 10MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 7 | 5MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| Note 1: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3. |  |  |  |

### 8.2.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.2.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.2.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 |
| Special subframe configuration (Note 2) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |
| Cross carrier scheduling |  | Not configured |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |

#### 8.2.2.1	Single-antenna port performance

The single-antenna performance in a given multi-path fading environments is determined by the SNR for which a certain relative information bit throughput of the reference measurement channels in Annex A.3.4 is achieved. The purpose of these tests is to verify the single-antenna performance with different channel models and MCS. The QPSK and 64QAM cases are also used to verify the performance for all bandwidths specified in Table 5.6.1-1.

##### 8.2.2.1.1	Minimum Requirement

For single carrier, the requirements are specified in Table 8.2.2.1.1-2, with the addition of the parameters in Table 8.2.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.2.1.1-4, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.2.1.1-7, based on single carrier requirement specified in Table 8.2.2.1.1-5, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.2.1.1-8, based on single carrier requirement specified in Table 8.2.2.1.1-5, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-9, based on single carrier requirement speicified in Table 8.2.2.1.1-5, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-10, based on single carrier requirement speicified in Table 8.2.2.1.1-5, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are speicifed in Table 8.2.1.1.1-11, based on single carrier requirement speicified in Table 8.2.2.1.1-5, with the addition of the parameters in Table 8.2.2.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.2.1.1-1: Test Parameters

| Parameter |  | Unit | Test 1- 5 | Test 6- 8 | Test 9- 15 | Test 16- 18 | Test 19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | OCNG (Note 2) | OCNG (Note 2) | OCNG (Note 2) | OCNG (Note 2) |
| Modulation |  |  | QPSK | 16QAM | 64QAM | 16QAM | QPSK |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing | Multiplexing | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 1 | 1 | 1 | 1 | 1 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: Void Note 4: Void |  |  |  |  |  |  |  |

Table 8.2.2.1.1-2: Minimum performance (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.2 | ≥1 |
| 2 | 10 MHz | R.2 TDD | OP.1 TDD | ETU70 | 1x2 Low | 70 | -0.6 | ≥1 |
| 3 | 10 MHz | R.2 TDD | OP.1 TDD | ETU300 | 1x2 Low | 70 | -0.2 | ≥1 |
| 4 | 10 MHz | R.2 TDD | OP.1 TDD | HST | 1x2 | 70 | -2.6 | ≥1 |
| 5 | 1.4 MHz | R.4 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 0.0 | ≥1 |
| 6 | 10 MHz | R.3 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 6.7 | ≥2 |
|  | 5 MHz | R.3-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 6.7 | 1 |
| 7 | 10 MHz | R.3 TDD | OP.1 TDD | ETU70 | 1x2 Low | 30 | 1.4 | ≥2 |
|  | 5 MHz | R.3-1 TDD | OP.1 TDD | ETU70 | 1x2 Low | 30 | 1.4 | 1 |
| 8 | 10 MHz | R.3 TDD | OP.1 TDD | ETU300 | 1x2 High | 70 | 9.3 | ≥2 |
|  | 5 MHz | R.3-1 TDD | OP.1 TDD | ETU300 | 1x2 High | 70 | 9.3 | 1 |
| 9 | 3 MHz | R.5 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥1 |
| 10 | 5 MHz | R.6 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥2 |
|  | 5 MHz | R.6-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.6 | 1 |
| 11 | 10 MHz | R.7 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥2 |
|  | 10 MHz | R.7-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.6 | 1 |
| 12 | 10 MHz | R.7 TDD | OP.1 TDD | ETU70 | 1x2 Low | 70 | 19.1 | ≥2 |
|  | 10 MHz | R.7-1 TDD | OP.1 TDD | ETU70 | 1x2 Low | 70 | 19.1 | 1 |
| 13 | 10 MHz | R.7 TDD | OP.1 TDD | EVA5 | 1x2 High | 70 | 19.1 | ≥2 |
|  | 10 MHz | R.7-1 TDD | OP.1 TDD | EVA5 | 1x2 High | 70 | 19.1 | 1 |
| 14 | 15 MHz | R.8 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.8 | ≥2 |
|  | 15 MHz | R.8-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.8 | 1 |
| 15 | 20 MHz | R.9 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.7 | ≥3 |
|  | 20 MHz | R.9-2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.7 | 2 |
|  | 20 MHz | R.9-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | 17.7 | 1 |
| 16 | 3 MHz | R.0 TDD | OP.1 TDD | ETU70 | 1x2 Low | 30 | 2.1 | ≥1 |
| 17 | 10 MHz | R.1 TDD | OP.1 TDD | ETU70 | 1x2 Low | 30 | 2.0 | ≥1 |
| 18 | 20 MHz | R.1 TDD | OP.1 TDD | ETU70 | 1x2 Low | 30 | 2.1 | ≥1 |
| 19 | 10 MHz | R.41 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -5.3 | ≥1 |
| Note 1: Void. |  |  |  |  |  |  |  |  |

Table 8.2.2.1.1-3: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | QPSK |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests in Table 8.2.2.1.1-4; PUCCH format 3 for Tests in Table 8.2.2.1.1-7 |
| PDSCH transmission mode |  |  | 1 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.1.1-4: Minimum performance (FRC) for CA with 2DL CCs

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 2x20MHz | R.42 TDD | OP.1 TDD (Note 1) | EVA5 | 1x2 Low | 70 | -1.2 | ≥5 |
| 2 | 20MHz+ 15MHz | R.42 TDD for 20MHz CC | OP.1 TDD (Note 1) | EVA5 | 1x2 Low | 70 | -1.4 | ≥5 |
|  |  | R.42-3 TDD for 15MHz CC | OP.1 TDD (Note 1) |  |  | 70 | -1.4 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. Note 3: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |  |  |  |

Table 8.2.2.1.1-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.4 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.6 |
| 3MHz | R.42-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.8 |
| 5MHz | R.42-2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.2 |
| 10MHz | R.2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 15MHz | R.42-3 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |
| 20MHz | R.42 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |

Table 8.2.2.1.1-6: Void

Table 8.2.2.1.1-7: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.2.1.1-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.2.1.1-5 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.2.1.1-8: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.2.1.1-5 per CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.2.2.1.1-5 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.1.1-9: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.2.1.1-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.2.1.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.1.1-10: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.2.1.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.1.1-11: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.2.1.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

##### 8.2.2.1.2	Void

##### 8.2.2.1.3	Void

##### 8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN

The requirements are specified in Table 8.2.2.1.4-2, with the addition of the parameters in Table 8.2.2.1.1.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the single-antenna performance with a single PRB allocated at the lower band edge in presence of MBSFN.

Table 8.2.2.1.4-1: Test Parameters for Testing 1 PRB allocation

| Parameter | Unit |  | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB | 0 |
|  |  |  |  | dB | 0 (Note 1) |
|  |  |  |  | dB | 0 |
| at antenna port |  |  |  | dBm/15kHz | -98 |
| Symbols for MBSFN portion of MBSFN subframes (Note 2) |  |  |  |  | OCNG (Note 3) |
| ACK/NACK feedback mode |  |  |  |  | Multiplexing |
| PDSCH transmission mode |  |  |  |  | 1 |
| Note 1: . Note 2: The MBSFN portion of an MBSFN subframe comprises the whole MBSFN subframe except the first two symbols in the first slot. Note 3: The MBSFN portion of the MBSFN subframes shall contain QPSK modulated data. Cell-specific reference signals are not inserted in the MBSFN portion of the MBSFN subframes, QPSK modulated MBSFN data is used instead. |  |  |  |  |  |

Table 8.2.2.1.4-2: Minimum performance 1PRB (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.29 TDD | OP.3 TDD | ETU70 | 1x2 Low | 30 | 2.0 | ≥1 |

#### 8.2.2.2	Transmit diversity performance

##### 8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port

The requirements are specified in Table 8.2.2.2.1-2, with the addition of the parameters in Table 8.2.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.2.2.2.1-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter | Unit | Test 1-2 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 2 |
| Note 1: |  |  |  |

Table 8.2.2.2.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11 TDD | OP.1 TDD | EVA5 | 2x2 Medium | 70 | 6.8 | ≥2 |
|  | 5 MHz | R.11-2 TDD | OP.1 TDD | EVA5 | 2x2 Medium | 70 | 6.8 | 1 |
| 2 | 10 MHz | R.10 TDD | OP.1 TDD | HST | 2x2 | 70 | -2.3 | ≥1 |

##### 8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port

The requirements are specified in Table 8.2.2.2.2-2, with the addition of the parameters in Table 8.2.2.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC-FSTD) with 4 transmitter antennas.

Table 8.2.2.2.2-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter | Unit | Test 1-2 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 2 |
| Note 1: |  |  |  |

Table 8.2.2.2.2-2: Minimum performance Transmit Diversity (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4 MHz | R.12 TDD | OP.1 TDD | EPA5 | 4x2 Medium | 70 | 0.2 | ≥1 |
| 2 | 10 MHz | R.13 TDD | OP.1 TDD | ETU70 | 4x2 Low | 70 | -0.5 | ≥1 |

##### 8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

The requirements are specified in Table 8.2.2.2.3-2, with the addition of parameters in Table 8.2.2.2.3-1 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell. In Table 8.2.2.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.2.2.2.3-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -94.8 (Note 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.2.2.3-2 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (Note 5) |  |  | N/A | 0000010001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A |
| CSI Subframe Sets (Note 7) | CCSI,0 |  | 0000010001 0000000001 | N/A |
|  | CCSI,1 |  | 1100101000 1100111000 | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PDSCH transmission mode |  |  | 2 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 9: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |

Table 8.2.2.2.3-2: Minimum Performance Transmit Diversity (FRC)

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 2) |  |
| 1 | R.11-4 TDD Note 4 | OP.1 TDD | OP.1 TDD | EVA5 | EVA5 | 2x2 Medium | 70 | 3.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.2.2.2.3A-2, with the addition of parameters in Table 8.2.2.2.3A-1. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.2.2.2.3A-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.2.2.3A-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.2.2.3A-2 | 12 | 10 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 5) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| PDSCH transmission mode |  |  | 2 | Note 9 | Note 9 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10, #12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 11: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.2.2.3A-2: Minimum Performance Transmit Diversity (FRC)

| Test Number |  | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 3) |  |
| 1 | R.11-4 TDD Note 4 |  | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 | EVA5 | EVA5 | 2x2 Medium | 70 | 3.5 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3.. Note 3: SNR corresponds to of cell 1. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model

The requirements are specified in Table 8.2.2.2.4-2, with the addition of parameters in Table 8.2.2.2.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two dominant interfering cells applying transmission mode 3 interference model defined in clause B.5.2. In Table 8.2.2.2.4-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.2.2.4-1: Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB |  | -3 | -3 | -3 |
|  |  | dB |  | -3 (Note 1) | -3 | -3 |
|  |  | dB |  | 0 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz |  | -98 | N/A | N/A |
| DIP (Note 2) |  | dB |  | N/A | -1.73 | -8.66 |
| BWChannel |  | MHz |  | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 2 |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 2 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.2 | As specified in clause B.5.2 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % |  | N/A | 80 | 80 |
|  | Rank 2 | % |  | N/A | 20 | 20 |
| Reporting interval |  | ms |  | 5 | N/A | N/A |
| Reporting mode |  |  |  | PUCCH 1-0 | N/A | N/A |
| ACK/NACK feedback mode |  |  |  | Multiplexing | N/A | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 5) | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  |  | 4 | N/A | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: All cells are time-synchronous. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |  |

Table 8.2.2.2.4-2: Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.46 TDD | OP.1 TDD | N/A | N/A | EVA70 | EVA70 | EVA70 | 2x2 Low | 70 | -1.4 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)

The requirements are specified in Table 8.2.2.2.5-2 with the addition of the parameters in Table 8.2.2.2.5-1 and the downlink physical channel setup according to Annex C.3.2. The test purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas in case of using eIMTA TDD UL-DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a PCell.

Table 8.2.2.2.5-1: Test Parameters for Transmit diversity Performance (FRC) when 
EIMTA-MainConfigServCell-r12 is configured

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Uplink downlink configuration in SIB1 (Note 2) |  |  | 0 |
| Downlink HARQ reference configuration (eimta-HarqReferenceConfig-r12) (Note 2) |  |  | 5 |
| Set of dynamic TDD UL-DL configurations (NOTES 2,3) |  |  | {0, 1, 2, 3, 4, 5, 6} |
| Periodicity of monitoring the L1 reconfiguration DCI (eimta-CommandPeriodicity-r12) |  | ms | 10 |
| Set of subframes to monitor the L1 reconfiguration DCI (eimta-CommandSubframeSet-r12) (Note 4) |  |  | {0,1,5,6} |
| Number of DL HARQ processes |  | Processes | 15 |
| PDSCH transmission mode |  |  | 2 |
| ACK/NACK feedback mode (Note 5) |  |  | Multiplexing |
| Note 1: Note 2: As specified in Table 4.2-2 in TS 36.211. Note 3: UL/DL configuration in PDCCH with eIMTA-RNTI is randomly selected from the given set on a per-DCI basis with equal probability. Note 4: The set of subframes to monitor PDCCH with eIMTA-RNTI for frame n includes subframes {1,5,6} in frame n-1 and subframe 0 in frame n. Subframes for reconfiguration DCI transmission are chosen in a random way on a per-DCI basis with equal probability. Note 5: PUCCH Format 3 is used for DL HARQ feedback. |  |  |  |

Table 8.2.2.2.5-2: Minimum performance Transmit diversity when EIMTA-MainConfigServCell-r12 is configured

| Test | Reference channel | OCNG Pattern | Propagation Conditions | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | R.67 TDD | OP.1 TDD | EVA5 | 2x2 Medium | 70 | 5.0 | ≥1 |

##### 8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model

The requirements are specified in Table 8.2.2.2.6-2, with the addition of parameters in Table 8.2.2.2.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 2 interference model defined in clause B.6.1. In Table 8.2.2.2.6-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.2.2.6-1: Test Parameters for Transmit Diversity Performance (FRC) with TM2 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 6 | 1 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | 3 | 3 |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 2 | 2 | 2 |
| Interference model |  |  |  | N/A | As specified in clause B.6.1 | As specified in clause B.6.1 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.2.2.2.6-2: Minimum Performance for Enhanced Performance Requirement Type B, Transmit Diversity (FRC) with TM2 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-12 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 15.3 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model

The requirements are specified in Table 8.2.2.2.7-2, with the addition of parameters in Table 8.2.2.2.7-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 9 interference model defined in clause B.6.4. In Table 8.2.2.2.7-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.2.2.7-1: Test Parameters for Transmit Diversity Performance (FRC) with TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | -3 | 0 | 0 |
|  |  |  | dB | -3 (Note 1) | 0 | 0 |
|  |  |  | dB | 0 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 3.28 | 0.74 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | Random from set {1,2,3} | Random from set {1,2,3} |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | Random from set {1,2} | Random from set {1,2} |
| PDSCH transmission mode |  |  |  | 2 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | N/A | Antenna ports 15,16 | Antenna ports 15,16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 10 / 4 | 10 / 4 |
| CSI reference signal configuration |  |  |  | N/A | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | N/A | 9 / 0100000000000000 | 9 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 5 | -5 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 600 | -600 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.2.2.2.7-2: Minimum Performance for Enhanced Performance Requirement Type B, Transmit Diversity (FRC) with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-11 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 8.1 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.2.2.8-2, with the addition of the parameters in Table 8.2.2.2.8-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance of transmit diversity (SFBC) with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.2.2.8-1: Test Parameters for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter | Unit | Test 1-2 |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 2 |
| MUSTIdx (Note 2) |  |  | 11 |
| p-a-must-r14 (Note 3) |  |  | N/A |
| Note 1: Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |

Table 8.2.2.2.8-2: Minimum Performance for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.aa TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | 13.9 | ≥1 |

#### 8.2.2.3	Open-loop spatial multiplexing performance

##### 8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port

For single carrier, the requirements are specified in Table 8.2.2.3.1-2, with the addition of the parameters in Table 8.2.2.3.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.2.3.1-4, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.2.3.1-7, based on single carrier requirement specified in Table 8.2.2.3.1-5, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.2.3.1-8, based on single carrier requirement specified in Table 8.2.2.3.1-5, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.2.3.1-9, based on single carrier requirement specified in Table 8.2.2.3.1-5, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are specified in Table 8.2.2.3.1-10, based on single carrier requirement specified in Table 8.2.2.3.1-5, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are specified in Table 8.2.2.3.1-11, based on single carrier requirement specified in Table 8.2.2.3.1-5, with the addition of the parameters in Table 8.2.2.3.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.2.3.1-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1-3 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Bundling |
| PDSCH transmission mode |  |  | 3 |
| Note 1: Note 2: Void. Note 3: Void. |  |  |  |

Table 8.2.2.3.1-2: Minimum performance Large Delay CDD (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 (Note 2) | 10 MHz | R.11-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.1 | ≥2 |
| 2 | 10 MHz | R.35 TDD | OP.1 TDD | EVA200 | 2x2 Low | 70 | 20.3 | ≥2 |
| 3 | 10 MHz | R.35-2 TDD | OP.1 TDD | ETU600 | 2x2 Low | 70 | 21.1 | ≥2 |
| Note 1: Void. Note 2: For UE that supports CRS interference handling, the CRS assistance information defined in [7] is provided. The CRS assistance information includes two aggressor cells with 2 CRS ports and cell ID of agressor cells are 1 and 128. For UE that does not support CRS interference handling, CRS assistance information is not provided. |  |  |  |  |  |  |  |  |

Table 8.2.2.3.1-3: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests in Table 8.2.2.3.1-4; PUCCH format 3 for Tests in Table 8.2.2.3.1-7 |
| PDSCH transmission mode |  |  | 3 |
| Note 1: Note 2: Void Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.3.1-4: Minimum performance Large Delay CDD (FRC) for CA with 2DL CCs

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 2x20 MHz | R.30-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.7 | ≥5 |
| 2 | 20MHz+15MHz | R.30-1 TDD for 20MHz CC | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.0 | ≥5 |
|  |  | R.11-9 TDD for 15MHz CC | OP.1 TDD (Note 1) | EVA70 |  | 70 | 12.9 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

Table 8.2.2.3.1-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10 MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP. 1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.2.2.3.1-6: Void

Table 8.2.2.3.1-7: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.2.3.1-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.2.3.1-5 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.3.1-8: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.2.3.1-5 per CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.2.2.3.1-5 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.3.1-9: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.2.3.1-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.2.3.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.3.1-10: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.2.3.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.3.1-11: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.2.3.1-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

##### 8.2.2.3.1A	Soft buffer management test

For CA, the requirements are specified in Table 8.2.2.3.1A-2, with the addition of the parameters in Table 8.2.2.3.1A-1 and the downlink physical channel setup according toAnnex C.3.2. The purpose is to verify UE performance with proper instantaneous buffer implementation.

Table 8.2.2.3.1A-1: Test Parameters for soft buffer management test (FRC) for CA

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | - (Note 2) |
| PDSCH transmission mode |  |  | 3 |
| Note 1: Note 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK. Note 3: For CA test cases, the same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.3.1A-2: Minimum performance soft buffer management test (FRC) for CA

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 2x20 MHz | R.30-2 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
| 2 | 2x20 MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA5 | 2x2 Low | 70 | 15.7 | 4 |
| Note 1: For CA test cases, the OCNG pattern applies for each CC. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

##### 8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports

The requirements are specified in Table 8.2.2.3.1B-2, with the addition of the parameters in Table 8.2.2.3.1B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

Table 8.2.2.3.1B-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Bundling |
| PDSCH transmission mode |  |  | 3 |
| Note 1: |  |  |  |

Table 8.2.2.3.1B-2: Enhanced Performance Requirement Type C for Large Delay CDD (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11-1 TDD | OP.1 TDD | EVA70 | 2x2 Medium | 70 | 17.4 | ≥2 |

##### 8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference

The requirements are specified in Table 8.2.2.3.1C-2, with the addition of parameters in Table 8.2.2.3.1C-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell with transmission mode 1. In Table 8.2.2.3.1C-1, Cell 1 is the serving cell, and Cell 2 is interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.2.3.1C-1 Test parameters for Larger Delay CDD (FRC) with TM1 interference

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  |  | -3 (Note 1) | 0 |
|  |  |  | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Transmission mode |  |  | 3 | Note 2 |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| (Note 3) |  | dB | Reference Value in Table 8.2.2.3.1C-2 | 12.95 |
| Correlation and antenna configuration |  |  | Medium (2x2) | Medium(1x2) |
| Number of OFDM symbols for PDCCH |  |  | 2 | N/A |
| Max number of HARQ transmissions |  |  | 4 | N/A |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |
| Note 1: Note 2: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.2 applying OCNG pattern OP.5 TDD as defined in Annex A.5.2.5. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: All cells are time-synchronous. Note 5: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |

Table 8.2.2.3.1C-2 Enhanced Performance Requirement Type C, Larger Delay CDD (FRC) with TM1 interference

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-10 TDD | OP.1 TDD | OP.5 TDD | EVA70 | EVA70 | 70 | 19.6 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

##### 8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port

The requirements are specified in Table 8.2.2.3.2-2, with the addition of the parameters in Table 8.2.2.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 4 transmitter antennas.

Table 8.2.2.3.2-1: Test Parameters for Large Delay CDD (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Bundling |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . |  |  |  |

Table 8.2.2.3.2-2: Minimum performance Large Delay CDD (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.14 TDD | OP.1 TDD | EVA70 | 4x2 Low | 70 | 14.2 | ≥2 |

##### 8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)

The requirements for non-MBSFN ABS are specified in Table 8.2.2.3.3-2, with the addition of parameters in Table 8.2.2.3.3-1 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3.

The requirements for MBSFN ABS are specified in Table 8.2.2.3.3-4, with the addition of parameters in Table 8.2.2.3.3-3 and the downlink physical channel setup according to Annex C.3.2 and Annex C.3.3.

The purpose is to verify the performance of large delay CDD with 2 transmitter antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell. In Tables 8.2.2.3.3-1 and 8.2.2.3.3-3, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.2.2.3.3-1: Test Parameters for Large Delay CDD (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -94.8 (Note 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.2.3.3-2 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Cell Id |  |  | 0 | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| ABS pattern (Note 5) |  |  | N/A | 0000010001, 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001, 0000000001 | N/A |
| CSI Subframe Sets (Note 7) | CCSI,0 |  | 0000010001, 0000000001 | N/A |
|  | CCSI,1 |  | 1100101000 1100111000 | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PDSCH transmission mode |  |  | 3 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 9: SIB-1 will not be transmitted in Cell2 in this test. |  |  |  |  |

Table 8.2.2.3.3-2: Minimum Performance Large Delay CDD (FRC) – Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 2) |  |
| 1 | R.11 TDD Note 4 | OP.1 TDD | OP.1 TDD | EVA 5 | EVA 5 | 2x2 Low | 70 | 14.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |

Table 8.2.2.3.3-3: Test Parameters for Large Delay CDD (FRC) – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A |
| at antenna port |  | dBm/15kHz | -102 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -94.8 (Note 4) | N/A |
|  |  | dB | Reference Value in Table 8.2.2.3.3-4 | 6 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN |
| Cell Id |  |  | 0 | 126 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| ABS pattern (Note 5) |  |  | N/A | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A |
| CSI Subframe Sets (Note 7) | CCSI,0 |  | 0000000001 0000000001 | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A |
| MBSFN Subframe Allocation (Note 10) |  |  | N/A | 000010 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PDSCH transmission mode |  |  | 3 | N/A |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10,#11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbol #0 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 5: ABS pattern as defined in [9]. The 10th and 20th subframes indicated by ABS pattern are MBSFN ABS subframes. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 9: SIB-1 will not be transmitted in Cell2 in this test. Note 10: MBSFN Subframe Allocation as defined in [7], one frame with 6 bits is chosen for MBSFN subframe allocation. |  |  |  |  |

Table 8.2.2.3.3-4: Minimum Performance Large Delay CDD (FRC) – MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 2) |  |
| 1 | R.11 TDD Note 4 | OP.1 TDD | OP.1 TDD | EVA 5 | EVA 5 | 2x2 Low | 70 | 12.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |

##### 8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements for non-MBSFN ABS are specified in Table 8.2.2.3.4-2, with the addition of parameters in Table 8.2.2.3.4-1. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.2.2.3.4-1, Cell 1 is the serving cell, and Cell 2 and Cell3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.2.3.4-1: Test Parameters for Large Delay CDD (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.2.3.4-2 | Reference Value in Table 8.2.2.3.4-2 | Reference Value in Table 8.2.2.3.4-2 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 1 | 126 |
| ABS pattern (Note 5) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| PDSCH transmission mode |  |  | 3 | Note 9 | Note 9 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 11: SIB-1 will not be transmitted in Cell2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.2.3.4-2: Minimum Performance Large Delay CDD (FRC) – Non-MBSFN ABS

| Test Number | Reference Channel |  |  | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 3) |  |
| 1 | R.11 TDD Note 4 | 9 | 7 | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 14.2 | ≥2 |
| 2 | R.35 TDD Note 4 | 9 | 1 | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 22.7 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.2.3.5-2, with the addition of the parameters in Table 8.2.2.3.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance of open-loop spatial multiplexing with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.2.3.5-1: Test Parameters for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter | Unit |  | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB | 0 |
|  |  |  |  | dB | 0 (NOTE 1) |
|  |  |  |  | dB | 0 |
|  |  | PDSCH_RA |  | dB | -3 |
|  |  | PDSCH_RB |  | dB | -3 |
| at antenna port |  |  |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  |  |  | Bundling |
| PDSCH transmission mode |  |  |  |  | 3 |
| MUSTIdx (Note 2) |  |  |  |  | 10 |
| p-a-must-r14 (Note 3) |  |  |  |  | -3 |
| Note 1: . Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |  |  |

Table 8.2.2.3.5-2: Minimum Performance for Minimum Requirement 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.bb TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | 19.3 | ≥2 |

##### 8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.2.3.6-2, with the addition of parameters in Table 8.2.2.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.2.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.2.3.6-1: Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Uplink downlink configuration |  |  | 4 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  |  | -3 (Note 1) | 0 |
|  |  |  | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Transmission mode |  |  | 3 | NA (Note 4) |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| (Note 3) |  | dB | Reference Value in Table 8.2.2.3.6-2 | 10 |
| Correlation and antenna configuration |  |  | Low (2x2) | Low (2x2) |
| Max number of HARQ transmissions |  |  | 4 | N/A |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  | Disabled | Enabled |
| CRS transmission |  |  | On | CRS is configured with CRS muting pattern as ‘10UU11111110UU111000’ where every 20 subframes consist of 11 subframes with full system BW CRS and 9 subframes with CRS only on the center 6 PRBs (Note 5) |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: The muting pattern ‘10UU11111110UU111000’ follows the Uplink downlink configuration 4 where U indicates uplink subframes, 1 indicates DL subframes with full system BW CRS configured and 0 indicateds DL subframes including special subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |

Table 8.2.2.3.6-2: Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-13 TDD | OP.1 TDD | NA | EVA5 | EVA5 | 70 | 15.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

#### 8.2.2.4	Closed-loop spatial multiplexing performance

##### 8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.2.2.4.1-2, with the addition of the parameters in Table 8.2.2.4.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband and frequency selective precoding.

Table 8.2.2.4.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 6 | 50 |
| PMI delay (Note 2) |  | ms | 10 or 11 | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | 001111 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 4 | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |

Table 8.2.2.4.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.10 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | -3.1 | ≥1 |
| 2 | 10 MHz | R.10 TDD | OP.1 TDD | EPA5 | 2x2 High | 70 | -2.8 | ≥1 |

##### 8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port

The requirements are specified in Table 8.2.2.4.1A-2, with the addition of the parameters in Table 8.2.2.4.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband and frequency selective precoding.

Table 8.2.2.4.1A-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 6 | 50 |
| PMI delay (Note 2) |  | ms | 10 or 11 | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000000000000000000001111111111111111 | 0000000000000000000000000000000000000000000000001111111111111111 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 4 | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |

Table 8.2.2.4.1A-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz | R.13 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | -3.5 | ≥1 | ≥4 |
| 2 | 10MHz 1024QAM | R.101 TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | 25.0 | TBD | 20, ≥ 22 |

##### 8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model

The requirements are specified in Table 8.2.2.4.1B-2, with the addition of the parameters in Table 8.2.2.4.1B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two dominant interfering cells applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.2.2.4.1B-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.2.4.1B-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB |  | -3 |  | -3 |  | -3 |  |
|  |  | dB |  | -3 (Note 1) |  | -3 |  | -3 |  |
|  |  | dB |  | 0 |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  | dBm/15kHz |  | -98 |  | N/A |  | N/A |  |
| DIP (Note 2) |  | dB |  | N/A |  | -1.73 |  | -8.66 |  |
| BWChannel |  | MHz |  | 10 |  | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  | Normal |  | Normal |  | Normal |  |
| Cell Id |  |  |  | 0 |  | 1 |  | 2 |  |
| Number of control OFDM symbols |  |  |  | 2 |  | 2 |  | 2 |  |
| PDSCH transmission mode |  |  |  | 6 |  | N/A |  | N/A |  |
| Interference model |  |  |  | N/A |  | As specified in clause B.5.3 |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % |  | N/A |  | 80 |  | 80 |  |
|  | Rank 2 | % |  | N/A |  | 20 |  | 20 |  |
| Precoding granularity |  | PRB |  | 50 |  | 6 |  | 6 |  |
| PMI delay (Note 4) |  | ms |  | 10 or 11 |  | N/A |  | N/A |  |
| Reporting interval |  | ms |  | 5 |  | N/A |  | N/A |  |
| Reporting mode |  |  |  | PUCCH 1-1 |  | N/A |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 1111 |  | N/A |  | N/A |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  | N/A |  | N/A |  |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 6) |  | N/A |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  | 4 |  | N/A |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |  |  |  |  |

Table 8.2.2.4.1B-2: Enhanced Performance Requirement Type A, Single-Layer Spatial Multiplexing (FRC) with TM4 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.47 TDD | OP.1 TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 1.1 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.2.2.4.1C-2, with the addition of parameters in Table 8.2.2.4.1C-1. The purpose is to verify the closed loop rank-one performance with wideband precoding if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.2.2.4.1C-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.2.4.1C-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | N/A | N/A |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.2.4.1C-2 | 12 | 10 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 5) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| ACK/NACK feeback mode |  |  | Multiplexing | N/A | N/A |
| PDSCH transmission mode |  |  | 6 | Note 9 | Note 9 |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 10) |  | ms | 10 or 11 | N/A | N/A |
| Reporting interval |  | ms | 1 or 4 (Note 11) | N/A | N/A |
| Peporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 1111 | N/A | N/A |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 11: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 12: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 13: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.2.2.4.1C-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)– Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) Note 5 | SNR (dB) (Note 3) |  |
| 1 | R.11 TDD Note 4 | OP.1TDD | OP.1FDD | OP.1TDD | EPA5 | EPA5 | EPA5 | 2x2 High | 70 | 6.4 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. Note 4: Cell 1 Reference channel is modified: PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: The maximum Throughput is calculated from the total Payload in 2 subframes, averaged over 20ms. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model

The requirements are specified in Table 8.2.2.4.1D-2, with the addition of the parameters in Table 8.2.2.4.1D-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 4 interference model defined in clause B.6.3. In Table 8.2.2.4.1D-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.2.2.4.1D-1: Test Parameters for Single-layer Spatial Multiplexing (FRC) with TM4 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 |  | 1 |  |
| Special subframe configuration |  |  |  | 4 | 4 |  | 4 |  |
| Downlink power allocation |  |  | dB | -3 | -3 |  | -3 |  |
|  |  |  | dB | -3 (Note 1) | -3 |  | -3 |  |
|  |  |  | dB | 0 | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |  |  |
| Test number (Note 4) |  |  |  |  | Test 1 | Test 2 | Test 1 | Test 2 |
|  |  |  | dB | N/A | 13.91 | 3.28 | 3.34 | 0.74 |
| Cell Id |  |  |  |  | 6 | 1 | 1 | 6 |
| CFI indicated in PCFICH in normal subframes |  |  |  |  | 3 | Random from set {1,2,3} | 3 | Random from set {1,2,3} |
| CFI indicated in PCFICH in special subframes |  |  |  |  | 2 | Random from set {1,2} | 2 | Random from set {1,2} |
| BWChannel |  |  | MHz | 10 | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  | Normal | Normal |  | Normal |  |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 |  | 3 |  |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 |  | 2 |  |
| PDSCH transmission mode |  |  |  | 4 | 4 |  | 4 |  |
| Interference model |  |  |  | N/A | As specified in clause B.6.3 |  | As specified in clause B.6.3 |  |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.3 |  | As specified in clause B.6.3 |  |
| Time offset to cell 1 |  |  | us | N/A | 2 |  | 3 |  |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 |  | 300 |  |
| MBSFN |  |  |  | Not configured | Not configured |  | Not configured |  |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} |  | {dB-6, dB-3, dB0} |  |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} |  | {2,3,4,8,9} |  |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. Note 4: Test 1 and Test 2 are defined in Table 8.2.2.4.1D-2. |  |  |  |  |  |  |  |  |

Table 8.2.2.4.1D-2: Minimum Performance for Enhanced Performance Requirement Type B, Single-layer Spatial Multiplexing (FRC) with TM4 interference model

| Test Num | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-12 TDD | OP.1 TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 85 | 16.1 | ≥1 |
| 2 | R.11-11 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 9.5 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.2.4.1E-2, with the addition of parameters in Table 8.2.2.4.1E-1. The purpose is to verify the closed loop rank-one performance with wideband precoding when CRS assistance information [7] is configured. In Table 8.2.2.4.1E-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.2.4.1E-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  |  | dB | 0 | 0 | 0 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.2.2.4.1E-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset to Cell 1 |  |  | s | N/A | 3 | -1 |
| Frequency shift to Cell 1 |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| ACK/NACK feeback mode |  |  |  | Multiplexing | N/A | N/A |
| PDSCH transmission mode |  |  |  | 4 | N/A | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  |  | ms | 10 or 11 | N/A | N/A |
| Reporting interval |  |  | ms | 1 or 4 (Note 3) | N/A | N/A |
| Peporting mode |  |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |  |

Table 8.2.2.4.1E-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.10-3 TDD | OP.1TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.2.4.1F-2, with the addition of parameters in Table 8.2.2.4.1F-1. In Table 8.2.2.4.1F-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.2.2.4.1F-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.2.2.4.1F-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 or 4 (Note 3) | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 000000000000FFFF | N/A | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |

Table 8.2.2.4.1F-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.36 TDD | OP.1TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 4x2 low | 70 | 14.1 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.2.2.4.2-2, with the addition of the parameters in Table 8.2.2.4.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

Table 8.2.2.4.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1-2 | Test 3 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 50 | 8 |
| PMI delay (Note 2) |  | ms | 10 or 11 | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | Bundling | Bundling |
| CodeBookSubsetRestriction bitmap |  |  | 110000 | 110000 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbol | 2 | 1 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |

Table 8.2.2.4.2-2: Minimum performance Multi-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz | R.35 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 19.5 | ≥2 | ≥6 |
| 2 | 10 MHz | R.11-1 TDD | OP.1 TDD | ETU70 | 2x2 Low | 70 | 13.9 | ≥2 | ≥6 |
| 3 | 20 MHz 256QAM | R. 65 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | 24.9 | 11-12 | ≥11 |

##### 8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.2.2.4.2A-2, with the addition of the parameters in Table 8.2.2.4.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband precoding.

Table 8.2.2.4.2A-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 50 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 |
| ACK/NACK feedback mode |  |  | Bundling |
| CodeBookSubsetRestriction bitmap |  |  | 110000 |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |

Table 8.2.2.4.2A-2: Enhanced Performance Requirement Type C for Multi-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11-1 TDD | OP.1 TDD | ETU70 | 2x2 Medium | 70 | 17.8 | ≥2 |

##### 8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port

For single carrier, the requirements are specified in Table 8.2.2.4.3-2, with the addition of the parameters in Table 8.2.2.4.3-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.2.4.3-4, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.2.4.3-7, based on single carrier requirement specified in Table 8.2.2.4.3-5, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.2.4.3-8, based on single carrier requirement specified in Table 8.2.2.4.3-5, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.2.4.3-9, based on single carrier requirement specified in Table 8.2.2.4.3-5, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are specified in Table 8.2.2.4.3-10, based on single carrier requirement specified in Table 8.2.2.4.3-5, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are specified in Table 8.2.2.4.3-11, based on single carrier requirement specified in Table 8.2.2.4.3-5, with the addition of the parameters in Table 8.2.2.4.3-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.2.4.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | Bundling |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Void. Note 5: Void. Note 6: Void. |  |  |  |

Table 8.2.2.4.3-2: Minimum performance Multi-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.36 TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | 15.7 | ≥2 |
| Note 1: Void |  |  |  |  |  |  |  |  |

Table 8.2.2.4.3-3: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 8 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests in Table 8.2.2.4.3-4; PUCCH format 3 for Tests in Table 8.2.2.4.3-7, Table 8.2.2.4.3-8 and Table 8.2.2.4.3-9. |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.4.3-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for CA with 2DL CCs

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 2x20 MHz | R.43 TDD | OP.1 TDD (Note 1) | EVA5 | 4x2 Low | 70 | 11.1 | ≥5 |
| 2 | 20MHz+15MHz | R.43 TDD for 20MHz CC | OP.1 TDD (Note 1) | EVA5 | 4x2 Low | 70 | 10.7 | ≥5 |
|  |  | R.43-5 TDD for 15MHz CC | OP.1 TDD (Note 1) |  |  |  | 10.6 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

Table 8.2.2.4.3-5: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP. 1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.2.4.3-6: Void

Table 8.2.2.4.3-7: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.2.4.3-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.2.4.3-5 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.4.3-8: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.2.4.3-5 per CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.2.2.4.3-5 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.4.3-9: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.2.4.3-5 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.2.4.3-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.4.3-10: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.2.4.3-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.4.3-11: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.2.4.3-5 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

##### 8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.2.2.4.3A-3, for 2DL CCs, in Table 8.2.2.4.3A-4 for 3DL CCs, and Table 8.2.2.4.3A-5 for 4DL CCs, based on single carrier requirement specified in Table 8.2.2.4.3A-2, with the addition of the parameters in Table 8.2.2.4.3A-1 and the downlink physical channel setup according to Annex C.3.2.The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity.

Table 8.2.2.4.3A-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 5) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The same PDSCH transmission mode is applied to each component carrier. Note 5: As defined in TS36.300 [11]. Note 6: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.2.2.4.3A-2: Single carrier performance for multiple dual connectivity configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP. 1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.2.4.3A-3: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.2.4.3A-2 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

Table 8.2.2.4.3A-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20 MHz | As specified in Table 8.2.2.4.3A-2 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

Table 8.2.2.4.3A-5: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20 MHz | As specified in Table 8.2.2.4.3A-2 per CC | ≥8 |
| 2 | 15+20+20+20MHz | As specified in Table 8.2.2.4.3A-2 per CC | ≥8 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

##### 8.2.2.4.4	Void

##### 8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)

The requirements are specified in Table 8.2.2.4.5-2, with the addition of the parameters in Table 8.2.2.4.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the minimun performance of closed-loop spatial multiplexing with 2 transmitter antennas superposed with simultaneous PDSCH interference.

Table 8.2.2.4.5-1: Test Parameters for Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port - Superposed transmission (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (NOTE 1) |
|  |  | dB | 0 |
|  | PDSCH_RA | dB | -3 |
|  | PDSCH_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 4 |
| Precoding |  |  | Random wideband precoding per TTI |
| MUSTIdx for transport block 1 (Note 2) |  |  | 10 |
| MUSTIdx for transport block 2 (Note 2) |  |  | 00 |
| p-a-must-r14 (Note 3) |  |  | -3 |
| Note 1: . Note 2: MUSTIdx is decribed in subclause 6.3.3 of [4]. Note 3: p-a-must-r14 is decribed in subclause 6.3.2 of [7]. |  |  |  |

Table 8.2.2.4.5-2: Minimum Performance for Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port - Superposed transmission (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.bb TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | 17.2 | ≥2 |

#### 8.2.2.5	MU-MIMO

#### 8.2.2.6	[Control channel performance: D-BCH and PCH]

#### 8.2.2.7	Carrier aggregation with power imbalance

The requirements in this section verify the ability of an intraband adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

##### 8.2.2.7.1	Minimum Requirement

For CA, the requirements are specified in Table 8.2.2.7.1-2, with the addition of the parameters in Table 8.2.2.7.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.2.7.1-1: Test Parameters for CA

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | Off (Note 2) | Off (Note 2) |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Modulation |  |  | 64 QAM | 64 QAM |
| Maximum number of HARQ transmission |  |  | 1 | 1 |
| Redundancy version coding sequence |  |  | {0} | {0} |
| PDSCH transmission mode of PCell |  |  | 1 | 3 |
| PDSCH transmission mode of SCell |  |  | 3 | 1 |
| OCNG Pattern | PCell |  | OP.1 TDD | OP.5 TDD |
|  | SCell |  | OP.5 TDD | OP.1 TDD |
| Propagation Conditions | PCell |  | Clause B.1 | Clause B.1 |
|  | SCell |  | Clause B.1 | Clause B.1 |
| Correlation Matrix and Antenna | PCell |  | 1x2 | 2x2 |
|  | SCell |  | 2x2 | 1x2 |
| Note 1: for 1x2 and for 2x2 antenna configuration. Note 2: No external noise sources are applied. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data. Note 4: Void. |  |  |  |  |

Table 8.2.2.7.1-2: Minimum performance (FRC) for CA

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/15KHz) |  | Reference value Fraction of Maximum Throughput (%) |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |  |
| 1 | 20 | 20 | R.49 TDD | NA | -85 | -79 | 85 | NA | ≥5 |
| 2 | 20 | 15 | NA | R.49-1 TDD | -79 | -85.8 | NA | 85 | ≥5 |
| Note 1: The OCNG pattern for PCell is used to fill the control channel. The OCNG pattern for SCell is used to fill the control channel and PDSCH. Note 2: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |  |

#### 8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing

The requirements in this section verify the ability of an UE supporting intraband contiguous carrier aggregation with minimum channel spacing to demodulate the signal transmitted by the PCell and SCell(s). Throughput is measured on each cell. The minimum channel spacing of intra-band contiguous carrier aggregation refers to the possible minimum channel spacing as any multiple of 300 kHz less than the nominal channel spacing defined in 5.7.1A.

##### 8.2.2.8.1	Minimum Requirement

For CA the requirements are specified in Table 8.2.2.8.1-2, with the addition of the parameters in Table 8.2.2.8.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.2.8.1-1: Test Parameters for CA

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | 64QAM |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Test 1; PUCCH format 3 for Test 2 |
| PDSCH transmission mode |  |  | 1 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.8.1-2: Minimum performance (FRC) for intra-band CA with minimum channel spacing

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 2x20MHz | R.9 TDD | OP.1 TDD (Note 1) | EVA5 | 1x2 Low | 70 | 17.16 | ≥5 |
|  |  | R.9 TDD | OP.1 TDD (Note 1) |  |  | 70 | 17.16 |  |
| 2 | 3x20MHz | R.9 TDD | OP.1 TDD (Note 1) | EVA5 | 1x2 Low | 70 | 17.16 | ≥5 |
|  |  | R.9 TDD | OP.1 TDD (Note 1) |  |  | 70 | 17.16 |  |
|  |  | R.9 TDD | OP.1 TDD (Note 1) |  |  | 70 | 17.16 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3. |  |  |  |  |  |  |  |  |

#### 8.2.2.9	HST-SFN performance

##### 8.2.2.9.1	Minimum Requirement

The purpose of this test is to verify UE performance in the HST-SFN scenario defined in B.3A when highSpeedEnhancedDemodulationFlag [7] is received.

For single carrier, the requirements are specified in Table 8.2.2.9.1-2, with the addition of the parameters in Table 8.2.2.9.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.2.9.1-5, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.2.9.1-6, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.2.9.1-7, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.2.2.9.1-8, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.2.9.1-1: Test Parameters for UE performance in HST-SFN scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.2.9.1-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | 13.2 | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

Table 8.2.2.9.1-3: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests in Table 8.2.2.3.1-4; PUCCH format 3 for Tests in Table 8.2.2.3.1-7 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: NOTE 2: Void NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.2.9.1-4: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.1] |
| 10 MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.2] |
| 15 MHz | R.87-3 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.4] |
| 20 MHz | R.87-4 TDD | OP. 1 TDD | HST-SFN | 2x2 Low | 70 | [13.6] |

Table 8.2.2.9.1-5: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| 2 | 20 MHz + 15 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-6: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| 2 | 20 MHz + 20 MHz + 15 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-7: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥8 |
| 2 | 20 MHz + 20 MHz + 20 MHz + 15 MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥8 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-8: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20 MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| 2 | 15 MHz + 4x20 MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3 |  |  |  |

##### 8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.2.2.9.2-2, with the addition of the parameters in Table 8.2.2.9.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhDemodFlag2-r16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhDemodFlag2-r16 [7] is not received. HST-500 test is not applicable to UE that has passed HST-SFN-500 test.

Table 8.2.2.9.2-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.2.9.2-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 TDD | OP.1 TDD | HST-SFN-500 | 2x2 | 70 | 9.6 | ≥ 1 |
| 2 | 10 MHz | R.87 TDD | OP.1 TDD | HST-500 | 2x2 | 70 | 11.4 | ≥ 1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1 and 8.1.2.6. NOTE 2: The requirement defined is based on the normalized channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

### 8.2.3	TDD FDD CA (Fixed Reference Channel)

The parameters specified in Table 8.2.3-1 are valid for all the TDD FDD CA tests unless otherwise stated.

Table 8.2.3-1: Common Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) for TDD CC only |  |  | 1 |
| Special subframe configuration (Note 2) for TDD CC only |  |  | 4 |
| Inter-TTI Distance |  |  | 1 |
| Maximum number of HARQ processes per component carrier | FDD PCell | Processes | 8 for FDD and TDD CCs |
|  | TDD PCell | Processes | 11 for FDD CC; 7 for TDD CC |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |
| Cyclic Prefix |  |  | Normal |
| Cell_ID |  |  | 0 |
| Cross carrier scheduling |  |  | Not configured |
| ACK/NACK feedback mode |  |  | PUCCH format 3 |
| Downlink HARQ-ACK timing | FDD PCell |  | As specified in Clause 7.3.3 in TS36.213 [6] |
|  | TDD PCell |  | As specified in Clause 7.3.4 in TS36.213 [6] |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

The applicability of ther requirements are specified in Clause 8.1.2.3. The single carrier performance with different bandwidths for multiple CA configurations specified in Clause 8.2.3 cannot be applied for UE single carrier test.

#### 8.2.3.1	Single-antenna port performance

The single-antenna performance in a given multi-path fading environments is determined by the SNR for which a certain relative information bit throughput of the reference measurement channels in Annex A.3.3 is achieved. The purpose of these tests is to verify the single-antenna performance with different channel models and MCS.

##### 8.2.3.1.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.1-4 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.1.1-5 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.1.1-6 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.1.1-7 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.1.1-8 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.1.1-9 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.1-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | QPSK |
| PDSCH transmission mode |  |  | 1 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.3 |
| 3 MHz | R.42-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.1 |
| 5MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |
| 15MHz | R.42-3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |

Table 8.2.3.1.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.6 |
| 3 MHz | R.42-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.8 |
| 5MHz | R.42-2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.2 |
| 10MHz | R.2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 15MHz | R.42-3 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |
| 20MHz | R.42 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |

Table 8.2.3.1.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

##### 8.2.3.1.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.2-4 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 3DL CCs, the requirements are specified in Table 8.2.3.1.2-5 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 4DL CCs, the requirements are specified in Table 8.2.3.1.2-6 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 5DL CCs, the requirements are specified in Table 8.2.3.1.2-7 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 6DL CCs, the requirements are specified in Table 8.2.3.1.2-8 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 7DL CCs, the requirements are specified in Table 8.2.3.1.2-9 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.2-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | QPSK |
| PDSCH transmission mode |  |  | 1 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.3 |
| 3 MHz | R.42-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.1 |
| 5MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |
| 15MHz | R.42-3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |

Table 8.2.3.1.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.6 |
| 3 MHz | R.42-1 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -0.8 |
| 5MHz | R.42-2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.2 |
| 10MHz | R.2 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 15MHz | R.42-3 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |
| 20MHz | R.42 TDD | OP.1 TDD | EVA5 | 1x2 Low | 70 | -1.4 |

Table 8.2.3.1.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

#### 8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port

##### 8.2.3.2.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.2.1-4 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.2.1-5 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.2.1-6 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.2.1-7 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.2.1-8 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.2.1-9 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.2.1-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.2.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3 MHz | R.11-6 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.2.3.2.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3 MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.2.3.2.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.2.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |

Table 8.2.3.2.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |

##### 8.2.3.2.1A	Soft buffer management test for FDD PCell

For TDD-FDD CA, the requirements are specified in Table 8.2.3.2.1A-2, with the addition of the parameters in Table 8.2.3.2.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the UE performance with proper instantaneous buffer implementation for FDD as PCell.

Table 8.2.3.2.1A-1: Test Parameters for CA

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD Carrier | TDD Carrier |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |  |

Table 8.2.3.2.1A-2: Minimum performance (FRC) for CA

| Test num. | Band-width |  | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | PCell | 20MHz | R.30 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 2 | PCell | 20MHz | R.35-1 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.3 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| 3 | PCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 4 | PCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| 5 | PCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 6 | PCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3B. |  |  |  |  |  |  |  |  |  |

##### 8.2.3.2.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.2.2-4 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.2.2-5 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.2.2-6 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.2.2-7 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.2.2-8 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.2.2-9 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.2.2-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.2.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3 MHz | R.11-6 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.2.3.2.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3 MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.2.3.2.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.2.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.2.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

##### 8.2.3.2.2A	Soft buffer management test for TDD PCell

For TDD-FDD CA, the requirements are specified in Table 8.2.3.2.2A-2, with the addition of the parameters in Table 8.2.3.2.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the UE performance with proper instantaneous buffer implementation for TDD as PCell.

Table 8.2.3.2.2A-1: Test Parameters for CA

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD Carrier | TDD Carrier |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |  |

Table 8.2.3.2.2A-2: Minimum performance (FRC) for CA

| Test num. | Band-width |  | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1)) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 20MHz | R.30 FDD | OP.1 FDD (Note 1 |  |  | 70 | 13.2 |  |
| 2 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 20MHz | R.35-1 FDD | OP.1 FDD (Note 1) |  |  | 70 | 16.2 |  |
| 3 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) |  |  | 70 | 16.0 |  |
| 4 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| 5 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| 6 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3B. |  |  |  |  |  |  |  |  |  |

#### 8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port

##### 8.2.3.3.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.3.1-4 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.3.1-5 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.3.1-6 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.3.1-7 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.3.1-8 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.3.1-9 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.3.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.3.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3 MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.3.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3 MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.3.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |

Table 8.2.3.3.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |

##### 8.2.3.3.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.3.2-4 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.3.2-5 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.3.2-6 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.3.2-7 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.3.2-8 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.3.2-9 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.3.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Widelband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.3.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3 MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.3.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3 MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.3.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |  |

Table 8.2.3.3.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.3.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

#### 8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.2.3.4-4, based on single carrier requirement specified in Table 8.2.3.4-2 and Table 8.2.3.4-3, with the addition of the parameters in Table 8.2.3.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity transmission.

Table 8.2.3.4-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for TDD-FDD dual connectivity

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 for FDD CC 10 or 11 for TDD CC |
| Reporting interval |  | ms | 1 for FDD CC 1 or 4 for TDD CC (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 5) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The same PDSCH transmission mode is applied to each component carrier. Note 5: As defined in TS36.300 [11]. Note 6: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.2.3.4-2: FDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.36 |
| 3MHz | R.14-5 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.4-3: TDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP. 1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.4-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Bandwidth combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.3.4-2 and Table 8.2.3.4-3 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

#### 8.2.3.5	HST-SFN performance

##### 8.2.3.5.0	General

The purpose of this test is to verify UE performance in the HST-SFN scenario defined in B.3A when highSpeedEnhancedDemodulationFlag [7] is received.

##### 8.2.3.5.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.5.1-4 based on single carrier requirement specified in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3, with the addition of the parameters in Table 8.2.3.5.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.5.1-5 based on single carrier requirement specified in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3, with the addition of the parameters in Table 8.2.3.5.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.5.1-6 based on single carrier requirement specified in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3, with the addition of the parameters in Table 8.2.3.5.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.5.1-7 based on single carrier requirement specified in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3, with the addition of the parameters in Table 8.2.3.5.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.5.1-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.5.1-2: Single carrier performance for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.9] |
| 10 MHz | R.87 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.3] |
| 15 MHz | R.87-3 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [13.9] |
| 20 MHz | R.87-4 FDD | OP. 1 FDD | HST-SFN | 2x2 Low | 70 | [14.1] |

Table 8.2.3.5.1-3: Single carrier performance for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.1] |
| 10 MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.2] |
| 15 MHz | R.87-3 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.4] |
| 20 MHz | R.87-4 TDD | OP. 1 TDD | HST-SFN | 2x2 Low | 70 | [13.6] |

Table 8.2.3.5.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.5.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.5.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | ≥8 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.5.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.5.1-2 and Table 8.2.3.5.1-3 per CC | 8, ≥11 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

##### 8.2.3.5.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.5.2-4 based on single carrier requirement specified in Table 8.2.5.2.2-2 and Table 8.2.3.5.2-3, with the addition of the parameters in Table 8.2.3.5.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.5.2-5 based on single carrier requirement specified in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3, with the addition of the parameters in Table 8.2.3.5.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.5.2-6 based on single carrier requirement specified in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3, with the addition of the parameters in Table 8.2.3.5.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.5.2-7 based on single carrier requirement specified in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3, with the addition of the parameters in Table 8.2.3.5.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.5.2-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.5.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 FDD | OP.1 FDD | HST-SFN | 2x2 Low | 70 | [13,9] |
| 10 MHz | R.87 FDD | OP.1 FDD | HST-SFN | 2x2 Low | 70 | [13.3] |
| 15 MHz | R.87-3 FDD | OP.1 FDD | HST-SFN | 2x2 Low | 70 | [13.9] |
| 20 MHz | R.87-4 FDD | OP.1 FDD | HST-SFN | 2x2 Low | 70 | [14.1] |

Table 8.2.3.5.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 5 MHz | R.87-2 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.1] |
| 10 MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.2] |
| 15 MHz | R.87-3 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.4] |
| 20 MHz | R.87-4 TDD | OP.1 TDD | HST-SFN | 2x2 Low | 70 | [13.6] |

Table 8.2.3.5.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.5.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥5 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.5.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | ≥8 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.5.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.5.2-2 and Table 8.2.3.5.2-3 per CC | 8, ≥11 |
| NOTE: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in clause 8.1.2.3B. |  |  |  |  |  |

### 8.2.4	LAA

#### 8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port

##### 8.2.4.1.1	FDD PCell (FDD single carrier)

The parameters specified in Table 8.2.4.1.1-1 are valid for FDD CC and LAA SCell(s) unless otherwise stated. And the additional parameters specified in Table 8.2.4.1.1-2 are valid for LAA SCell(s).

Table 8.2.4.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission (Note 1) |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Cross carrier scheduling |  | Not configured |
| Note 1: For retransmission in partial subframes, the TB size should be kept the same as the initial transmission regardless of the initial transmission is performed in full subframes or partial subframes. |  |  |

Table 8.2.4.1.1-2: Addtional Test Parameters for LAA SCell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | 0 |
| Discovery signal occasion duration | subframe | 1 |
| Power allocation of discovery signal |  | Same as power allocation of CRS within a transmission burst in the test |

For CA with LAA SCell(s), the requirements are specified in Table 8.2.4.1.1-4, with the addition of the parameters in Table 8.2.4.1.1-1, Table 8.2.4.1.1-2, Table 8.2.4.1.1-3 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding for CA with LAA SCell(s).

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.4.1.1-3: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  |  | {1,3,5,8} |
| Occupied OFDM symbols set in the last subframe |  |  | {6,9,12,14} |
| Random variable p defined in B.8 |  |  | 0.5 |
| timing error relative of LAA SCell to PCell |  | μs | 0 |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | 200 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.4.1.1-4: Single carrier performance for PCell for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.4.1.1-5: Single carrier performance for LAA SCell(s) for multiple CA configurations

| Band-width | Sub-test (Note 2) | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of norminal maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | 1 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.7 |
|  | 2 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.6 |
|  | 3 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.9 |
|  | 4 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 19 |
| Note 1: Fraction of nominal maximum throughput is calculated based on random transmission occasions of PDSCH. Note 2: An UE is required to fulfill only one test of Sub-test 1-4 depending on UE capabilities of endingDwPTS and secondSlotStartingPosition. For an UE not supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 1; For an UE not supporting endingDwPTS but supporting secondSlotStartingPosition, it is required to fulfill Sub-test 2; For an UE supporting endingDwPTS but not supporting secondSlotStartingPosition, it is required to fulfil Sub-test 3; and For an UE supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 4. |  |  |  |  |  |  |  |

Table 8.2.4.1.1-6: Minimum performance (FRC) based on single carrier performance for CA with one LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+20 | 15 | 20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+20 | 10 | 20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.1-7: Minimum performance (FRC) based on single carrier performance for CA with two LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+2x20 | 15 | 2x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+2x20 | 10 | 2x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in Table 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.1-8: Minimum performance (FRC) based on single carrier performance for CA with three LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+3x20 | 15 | 3x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in Table 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.1-9: Minimum performance (FRC) based on single carrier performance for CA with four LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 5x20 | 20 | 4x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+4x20 | 15 | 4x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in Table 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.1-10: Minimum performance (FRC) based on single carrier performance for CA with five LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 6x20 | 20 | 5x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+5x20 | 15 | 5x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+5x20 | 10 | 5x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in Table 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.1-11: Minimum performance (FRC) based on single carrier performance for CA with six LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 7x20 | 20 | 6x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 2 | 15+6x20 | 15 | 6x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| 3 | 10+6x20 | 10 | 6x20 | As defined in Table 8.2.4.1.1-4 and Table 8.2.4.1.1-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.2.4.1.1-4 for PCell and apply a per-CC requirement defined in Table 8.2.4.1.1-5 for LAA SCell. |  |  |  |  |  |

##### 8.2.4.1.2	TDD PCell (TDD single carrier)

The parameters specified in Table 8.2.4.1.2-1 are valid for TDD CC and LAA SCell(s) unless otherwise stated. And the additional parameters specified in Table 8.2.4.1.2-2 are valid for LAA SCell(s).

Table 8.2.4.1.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 |
| Special subframe configuration (Note 2) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission (Note 3) |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Cross carrier scheduling |  | Not configured |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: For retransmission in partial subframes, the TB size should be kept the same as the initial transmission regardless of the initial transmission is performed in full subframes or partial subframes. |  |  |

Table 8.2.4.1.2-2: Addtional Test Parameters for LAA SCell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | 0 |
| Discovery signal occasion duration | subframe | 1 |
| Power allocation of discovery signal |  | Same as power allocation of CRS within a transmission burst in the test |

For CA with LAA SCell(s), the requirements are specified in Table 8.2.4.1.2-4, with the addition of the parameters in Table 8.2.4.1.2-1, Table 8.2.4.1.2-2, Table 8.2.4.1.2-3 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding for CA with LAA SCell(s).

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.4.1.2-3: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  |  | {1,3,5,8} |
| Occupied OFDM symbols set in the last subframe |  |  | {6,9,12,14} |
| Random variable p defined in B.8 |  |  | 0.5 |
| timing error relative of LAA SCell to PCell |  | μs | 0 |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | 200 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 5: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 6: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.4.1.2-4: Single carrier performance for PCell for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.4.1.2-5: Single carrier performance for LAA SCell for multiple CA configurations

| Bandwidth | Sub-test (Note 2) | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | 1 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.7 |
|  | 2 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.6 |
|  | 3 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 18.9 |
|  | 4 | R.1 FS3 | OP.1 FS3 | EVA5 | 4x2 Low | 70 | 19 |
| Note 1: Fraction of nominal maximum throughput is calculated based on random transmission occasions of PDSCH. Note 2: An UE is required to fulfill only one test of Sub-test 1-4 depending on UE capabilities of endingDwPTS and secondSlotStartingPosition. For an UE not supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 1; For an UE not supporting endingDwPTS but supporting secondSlotStartingPosition, it is required to fulfill Sub-test 2; For an UE supporting endingDwPTS but not supporting secondSlotStartingPosition, it is required to fulfil Sub-test 3; and For an UE supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 4. |  |  |  |  |  |  |  |

Table 8.2.4.1.2-6: Minimum performance (FRC) based on single carrier performance for CA with one LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.2-7: Minimum performance (FRC) based on single carrier performance for CA with two LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.2-8: Minimum performance (FRC) based on single carrier performance for CA with three LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.2-9: Minimum performance (FRC) based on single carrier performance for CA with four LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 5x20 | 20 | 4x20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.2-10: Minimum performance (FRC) based on single carrier performance for CA with five LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 6x20 | 20 | 5x20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

Table 8.2.4.1.2-11: Minimum performance (FRC) based on single carrier performance for CA with six LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 7x20 | 20 | 6x20 | As defined in Table 8.2.4.1.2-4 and Table 8.2.4.1.2-5 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in 8.2.4.1.2-4 for PCell and apply a per-CC requirement defined in 8.2.4.1.2-5 for LAA SCell. |  |  |  |  |  |

## 8.3	Demodulation of PDSCH (User-Specific Reference Symbols)

### 8.3.1	FDD

The parameters specified in Table 8.3.1-1 are valid for FDD unless otherwise stated.

Table 8.3.1-1: Common Test Parameters for User-specific Reference Symbols

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG for Transmission modes 9 and 10 Time domain: 1 ms |  |
| Note 1: Void. Note 2: Void. |  |  |  |  |  |

#### 8.3.1.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.1.1-1 and 8.3.1.1-2, with the addition of the parameters in Table 8.3.1.1-3 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8 with and without a simultaneous transmission on the other antenna port, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.3.1.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| parameter |  | Unit | Test 1, Test 1a | Test 2 | Test 3 | Test 4, Test 4a | Test 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 | -3 | -3 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 | Antenna ports 15, …, 18 | Antenna ports 15,…,18 | Antenna ports 15, …, 18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 | 5 / 2 | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 0 | 3 | 0 | 0 | 0 |
| csi-RS-ConfigZP-ApList |  |  | N/A | N/A | N/A | {0,1} | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 | 3 / 0001000000000000 | 3 / 0001000000000000 | 0100000000000000 or 0001000000000000 (Note 7) | N/A |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 | 50 | 50 | 50 |
| Simultaneous transmission |  |  | No | Yes (Note 3, 5) | No | No | No |
| PDSCH transmission mode |  |  | 9 | 9 | 9 | 9 | 9 |
| Number of MBSFN subframes |  | Subframes | 6 (Note 6) | NA | NA | 6 (Note 6) | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. Note 6: For FDD mode, 6 subframes (#1/2/3/6/7/8) are allocated as MBSFN subframes. Note 7: Through DCI signalling, indicating aperiodic ZP-CSI-RS in sub-frames #3 and #8 per frame, the indicated aperiodic ZP-CSI-RS is random selected from RRC-configured AP ZP CSIRS list. |  |  |  |  |  |  |  |

Table 8.3.1.1-2: Minimum performance for CDM-multiplexed DM RS without simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL Cat-egory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz QPSK 1/3 | R.43-1 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | -1.2 | ≥1 | ≥6 |
| 1a | 10 MHz QPSK 1/3 | R.43-2 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | [-1.3] | ≥1 | ≥6 |
| 3 | 10MHz 256QAM | R. 66 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 24.3 | 11-12 | ≥11 |
| 4 | 10 MHz QPSK 1/3 | R.43-1 FDD | OP.1 FDD | EVA5 | 2x2 Low | 70 | [-1.2] | ≥1 | ≥6 |
| 4a | 10 MHz QPSK 1/3 | R.43-2 FDD | OP.1 FDD | EVA5 | 2x2 Low | [70] | TBD | ≥1 | ≥6 |
| 5 | 10MHz 1024QAM | R.101 FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | 30.1 | TBD | 20, ≥ 22 |
| Note 1: For UE that does not support aperiodic ZP-CSI-RS, test 1a will be run and test 1 will be skipped if the UE indicates support of pdsch-CollisionHandling-r13. Otherwise, test 1 will be run and test 1a will be skipped. For UE that does not support aperiodic ZP-CSI-RS, test 4 and test 4a will be skipped. Note 2: For UE that supports AP ZP CSIRS, test 4a will be run and test 4 will be skipped if the UE indicates support of pdsch-CollisionHandling-r13. Otherwise, test 4 will be run and test 4a will be skipped. For UE that supports AP ZP CSIRS, test 1 and test 1a will be skipped. |  |  |  |  |  |  |  |  |  |

Table 8.3.1.1-3: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 2 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 21.9 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

#### 8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model

The requirements are specified in Table 8.3.1.1A-2, with the addition of the parameters in Table 8.3.1.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.3.1.1A-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.3.1.1A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 126 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 9 | N/A |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  | Ms | 8 | N/A |
| Reporting interval |  | Ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000000000000000000001111111111111111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |

Table 8.3.1.1A-2: Enhanced Performance Requirement Type A, CDM-multiplexed DM RS with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.48 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 4x2 Low | 70 | -1.1 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.3.1.1B-2, with the addition of parameters in Table 8.3.1.1B-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.3.1.1B-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.1.1B-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | -3 | -3 |  |
|  |  | dB | 0 (Note 1) | -3 (Note 1) | -3 (Note 1) |  |
|  |  | dB | -3 | N/A | N/A |  |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |  |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |  |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |  |
|  |  | dB | Reference Value in Table 8.3.1.1B-2 | 12 | 10 |  |
| BWChannel |  | MHz | 10 | 10 | 10 |  |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |  |
| Time Offset between Cells |  | s | N/A | 3 | -1 |  |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |  |
| Cell Id |  |  | 0 | 1 | 126 |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |  |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | N/A | N/A |  |
| ABS pattern (Note 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 | 11000000 11000000 11000000 11000000 11000000 |  |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |  |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 11000000 11000000 11000000 11000000 11000000 | N/A | N/A |  |
|  | CCSI,1 |  | 00111111 00111111 00111111 00111111 00111111 | N/A | N/A |  |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |  |
| PDSCH transmission mode |  |  | TM9-1layer | Note 9 | Note 9 |  |
| Precoding granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |  |
| Beamforming model |  |  | Annex B.4.1 | N/A | N/A |  |
| Cyclic prefix |  |  | Normal | Normal | Normal |  |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 11: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 12: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 13: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |

Table 8.3.1.1B-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports) – Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51 FDD | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 |  |  | 2x2 Low | 70 | 7.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model

The requirements are specified in Table 8.3.1.1C-2, with the addition of the parameters in Table 8.3.1.1C-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7, 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 9 interference model defined in clause B.6.4. In 8.3.1.1C-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.1.1C-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 9 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 1 | 10 / 1 | 10 / 1 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 6 / 1000000000000000 | 6 / 0100000000000000 | 6 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.1.1C-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.5 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model

The requirements are specified in Table 8.3.1.1D-2, with the addition of the parameters in Table 8.3.1.1D-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by the CRS of the interfering cell, applying the CRS interference model defined in clause B.6.5. In 8.3.1.1D-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.1.1D-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with CRS interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 8 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Precoding |  |  |  | Random wideband precoding per TTI | N/A | N/A |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.1.1D-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with CRS interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.71 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 14.3 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model

The requirements are specified in Table 8.3.1.1E-2, with the addition of the parameters in Table 8.3.1.1E-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 3 interference model defined in clause B.6.2. In 8.3.1.1E-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.1.1E-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with TM3 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | -3 | -3 |
|  |  |  | dB | 0 (Note 1) | -3 | -3 |
|  |  |  | dB | -3 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 3.28 | 0.74 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | Random from {1,2,3} | Random from {1,2,3} |
| PDSCH transmission mode |  |  |  | 8 | 3 | 3 |
| Interference model |  |  |  | N/A | As specified in clause B.6.2 | As specified in clause B.6.2 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.2 | As specified in clause B.6.2 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.1.1E-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.70 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 11.5 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model

The requirements are specified in Table 8.3.1.1F-2, with the addition of the parameters in Table 8.3.1.1F-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission configured with TM10 in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.6.3. The NAICS network assistance is provided when the serving cell TM10 is configured with QCL-type A and PCID based DM-RS scrambling. The neighbouring cell has transmission mode TM9 and NeighCellsInfo-r12 for interfering cell indicates presence of TM9. In 8.3.1.1F-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.1.1F-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM10 serving cell configuration and TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  |  | 3 | 3 | 3 |
| PDSCH transmission mode |  |  |  | 10 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 1 | 10 / 1 | 10 / 1 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 6 / 1000000000000000 | 6 / 0100000000000000 | 6 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.1.1F-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM10 serving cell configuration and TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 FDD | OP.1 FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.2 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.1.1G-2, with the addition of parameters in Table 8.3.1.1G-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.1.1G-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.1.1G-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  | dB | Reference Value in Table 8.3.1.1G-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift to Cell 1 |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | TM9-1layer | N/A | N/A |
| Precoding granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  | Annex B.4.1 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 4: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 5: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |

Table 8.3.1.1G-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51-1 FDD | OP.1FDD | N/A | N/A | EVA5 |  |  | 2x2 Low | 70 | 11.6 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/NocEs/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)

For single-layer transmission on antenna port 7, 8, 11 or 13 upon detection of a PDCCH with DCI format 2C, the requirement is specified in Table 8.3.1.1H-2, with the addition of the parameters in Table 8.3.1.1H-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of the test is to verify rank-1 performance on antenna port 11 with a simultaneous transmission on the antenna port 7, 8 or 13 with DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.3.1.1H-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| dmrs-Enhancements-r13 |  |  | Enable |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 11. Note 3: Modulation symbols of an interference signal are random mapped onto one antenna port among antenna port 7, 8 and 13. The upadate granularity for randomized mapping antenna port is 1 PRG in frequency domain and 1ms in time domain. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 with OCC =4. |  |  |  |

Table 8.3.1.1H-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 21.9 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

#### 8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)

For single-layer transmission on a DMRS antenna port upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.1.1I-1, with the addition of the parameters in Table 8.3.1.1I-2 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one DMRS antenna port with a simultaneous transmission on on of the other DMRS antenna port with or without DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.3.1.1I-1: Test Parameters for Minimun Performance Requirement - Single-layer Spatial Multiplexing with assistance information for simultaneous transmition interfering PDSCH (FRC)

| parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 (Note 1) |  |  |
|  |  | dB | -3 |  |  |
| Beamforming model |  |  | Annex B.4.1 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |  |  |
| CSI reference signal configuration |  |  | 3 |  |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |  |  |
| Number of allocated resource blocks |  | PRB | 50 |  |  |
| Simultaneous transmission |  |  | Yes |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |
| Number of MBSFN subframes |  | Subframes | NA |  |  |
| dmrs-Enhancements-r13 |  |  | Disabled | Enable | Enable |
| k-max-r14 (Note 5) |  |  | 1 | 1 | 3 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: In Test 1, the modulation symbols of the signal under test are mapped randomly onto antenna port 7 or 8. Modulation symbols of an interference signal are mapped randomly onto one of the remaining DMRS antenna port. The two UEs’ scrambling identities are set to 0 with OCC =2. Note 4: In Test 2 and Test 3, the modulation symbols of the signal under test are mapped randomly onto antenna port 7, 8, 11 or 13. Modulation symbols of an interference signal are mapped randomly onto one of the remaining DMRS antenna port. The two UEs’ scrambling identities are set to 0 with OCC =4. Note 5: k-max-r14 is decribed in subclause 6.3.2 of [7]. Note 6: The upadate granularity for randomized mapping antenna port is 50 PRGs in frequency domain and 1ms in time domain. |  |  |  |  |  |

Table 8.3.1.1I-2: Minimum performance for Minimun Performance Requirement - Single-layer Spatial Multiplexing with assistance information for simultaneous transmition interfering PDSCH (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.cc FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | 15.2 | ≥2 |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

#### 8.3.1.2	Dual-Layer Spatial Multiplexing

For dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.1.2-2, with the addition of the parameters in Table 8.3.1.2-1 where Cell 1 is the serving cell and Cell 2 is the interfering cell. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power, and to verify that the UE correctly estimate SNR.

Table 8.3.1.2-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| # Parameter |  | # Unit | # Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | # Cell 1 | # Cell 2 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
|  | PDSCH_RA | dB | 4 | NA |
|  | PDSCH_RB | dB | 4 | NA |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 | 126 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  |  | Reference Value in Table 8.3.1.2-2 | 7.25dB |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | Blanked |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.1.2-2: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 FDD | OP.1 FDD | N/A | ETU5 | ETU5 | 2x2 Low | 70 | 14.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing

The requirements are specified in Table 8.3.1.2A-2, with the addition of the parameters in Table 8.3.1.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of this test is to verify rank two performance for full RB allocation upon antenna ports 7 and 8.

Table 8.3.1.2A-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.3.1.2A-2: Enhanced Performance Requirement Type C for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 FDD | OP.1 FDD | EPA5 | 2x2 Medium | 70 | 17.4 | ≥2 |

#### 8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports

##### 8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)

The requirements are specified in Table 8.3.1.3.1-3, with the additional parameters in Table 8.3.1.3.1-1 and Table 8.3.1.3.1-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission point share the same Cell ID. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6], configured according to Table 8.3.1.3.1-2. In Tables 8.3.1.3.1-1 and 8.3.1.3.1-2, transmission point 1 (TP 1) is the serving cell and transmission point 2 (TP 2) transmits PDSCH. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.1.3.1-1: Test Parameters for quasi co-location type B: same Cell ID

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | (Note 2) |
| CSI-RS 0 antenna ports |  |  | NA | Port {15,16} |
| qcl-CSI-RS-ConfigNZPId-r11, CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | NA | 5/2 |
| qcl-CSI-RS-ConfigNZPId-r11, CSI-RS 0 configuration |  |  | NA | 8 |
| csi-RS-ConfigZPId-r11, Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | NA | 2/ 0000010000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  | dB | Reference point in Table 8.3.1.3.1-3 | Reference point in Table 8.3.1.3.1-3 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 0 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | Blanked | 10 |
| Number of allocated PRB |  | PRB | NA | 50 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ |  |
| Time offset between TPs |  | s | NA | Reference point in Table 8.3.1.3.1-3 |
| Frequency error between TPs |  | Hz | NA | 0 |
| Beamforming model |  |  | NA | Port 7 as specified in clause B.4.1 |
| Symbols for unused PRBs |  |  | NA | OCNG (Note 3) |
| Note 1: Noet 2: REs for antenna ports 0 and 1 have zero transmission power. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.1.3.1-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.1.3.1-3: Minimum performance for quasi co-location type B: same Cell ID

| Test Number | Reference Channel | OGCN pattern |  | Time offset between TPs (s) | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 |  | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52 FDD | NA | OP.1 FDD | 2 | EPA5 | EPA5 | 2x2 Low | 70 | 12.1 | ≥2 |
| 2 | R.52 FDD | NA | OP.1 FDD | -0.5 | EPA5 | EPA5 | 2x2 Low | 70 | 12.6 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for TP 1 and TP 2. Note 3: SNR corresponds to of TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)

The requirements are specified in Table 8.3.1.3.2-3, with the additional parameters in Tables 8.3.1.3.2-1 and 8.3.1.3.2-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission point share the same Cell ID. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6]. In Tables 8.3.1.3.2-1 and 8.3.1.3.2-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, and transmission point 2 (TP 2) has same Cell ID as TP 1. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between 2 TPs with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.1.3.2-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.1.3.2-1: Test Parameters for timing offset compensation with DPS transmission

| parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | (Note 2) |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A |
| CSI reference signal 0 configuration |  |  | 0 | N/A |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 2 |
| CSI reference signal 1 configuration |  |  | N/A | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  | Subframes/bitmap | N/A | 2/ 0000010000000000 |
|  |  | dB | Reference Value in Table 8.3.1.3.2-3 | Reference Value in Table 8.3.1.3.2-3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 0 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| Timing offset between TPs |  |  | N/A | Reference Value in Table 8.3.1.3.2-3 |
| Frequency offset between TPs |  | Hz | N/A | 0 |
| Number of allocated resource blocks |  | PRB | 50 | 50 |
| PDSCH transmission mode |  |  | 10 | 10 |
| Probability of occurrence of PDSCH transmission(Note 3) |  | % | 30 | 70 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) |
| Note 1: Note 2: REs for antenna ports 0 and 1 have zero transmission power. Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.1.3.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 3 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.1.3.2-3: Performance Requirements for timing offset compensation with DPS transmission

| Test Number | Timing offset(us) | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | 2 | R.53 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | 2x2 Low | 70 | 12.2 | ≥2 |
| 2 | -0.5 | R.53 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | 2x2 Low | 70 | 12.5 | ≥2 |
| Note 1: The propagation conditions for TP 1and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of both TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)

The requirements are specified in Table 8.3.1.3.3-2, with the additional parameters in Table 8.3.1.3.3-1. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission points have different Cell ID and colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.3.1.3.3-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, and transmission point 2 (TP 2) transmits PDSCH with different Cell ID. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.1.3.3-1: Test Parameters for quasi co-location type B with different Cell ID and Colliding CRS

| parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | N/A | As specified in clause B.4.2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 2 |
| CSI reference signal 0 configuration |  |  | N/A | 0 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 |
|  |  | dB | Reference point in Table 8.3.1.3.3-2 + 4dB | Reference Value in Table 8.3.1.3.3-2 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 126 |
| Number of control OFDM symbols |  |  | 1 | 2 |
| Timing offset between TPs |  | us | N/A | 0 |
| Frequency offset between TPs |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ |  |
| PDSCH transmission mode |  |  | Blank | 10 |
| Number of allocated resource block |  |  | N/A | 50 |
| Symbols for unused PRBs |  |  | N/A | OCNG(Note2) |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.1.3.3-2: Performance Requirements for quasi co-location type B with different Cell ID and Colliding CRS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.54 FDD | N/A | OP.1FDD | EPA5 | ETU5 | 2x2 Low | 70 | 14.4 | ≥2 |
| Note 1: The propagation conditions for TP.1 and TP.2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1 and TP.2. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)

The requirements are specified in Table 8.3.1.3.4-3, with the additional parameters in Table 8.3.1.3.4-1 and Table 8.3.1.3.4-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference and time difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation.  The CRS assistance information [7] includes TP 3. In Table 8.3.1.3.4-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, transmission point 2 (TP 2) transmits PDSCH with different Cell ID, and Transmission point 3 (TP 3) is the aggressor transmission point. The downlink physical channel setup for TP 1 is according to Table C.3.4-1, for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.1.3.4-1: Test Parameters for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| parameter |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  | N/A | Port 7 as specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 0 configuration |  |  | N/A | 0 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 | N/A |
|  |  | dB | 10.45 | Reference Value in Table 8.3.1.3.4-3 | 8.45 |
| at antenna port |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  | 1 | 2 | 2 |
| Timing offset to TP 1 |  | us | N/A | -0.5 | 3 |
| Frequency offset to TP 1 |  | Hz | N/A | 200 | -100 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ |  | N/A |
| PDSCH transmission mode |  |  | Blank | 10 | 9 |
| Number of allocated resource block |  |  | N/A | 50 | N/A |
| Symbols for unused PRBs |  |  | N/A | OCNG(Note2) | N/A |
| Interference model |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | N/A | 80 |
|  | Rank 2 | % | N/A | N/A | 20 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |

Table 8.3.1.3.4-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.1.3.4-3: Performance Requirements for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 FDD | N/A | OP.1FDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 10.8 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)

The requirements are specified in Table 8.3.1.3.5-3, with the additional parameters in Tables 8.3.1.3.5-1 and 8.3.1.3.5-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points  have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference and timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Tables 8.3.1.3.5-1 and 8.3.1.3.5-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, Transmission point 2 (TP 2) has different Cell ID as TP 1, and Transmission point 3 (TP 3) is the aggressor transmission point. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between TP 1 and TP 2 with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.1.3.5-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1,  for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.1.3.5-1: Test Parameters DPS transmission with CRS assistance information

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal 0 configuration |  |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 1 configuration |  |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | 2/ 0010000000000000 | N/A | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  |  | Subframes/bitmap | N/A | 2/ 0000010000000000 | N/A |
| (Note 2) |  |  | dB | Reference Value in Table 8.3.1.3.5-3 | Reference Value in Table 8.3.1.3.5-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Timing offset to TP 1 |  |  |  | N/A | -0.5 | 3 |
| Frequency offset to TP 1 |  |  | Hz | N/A | 200 | -100 |
| Number of allocated resource blocks |  |  | PRB | 50 | 50 | N/A |
| PDSCH transmission mode |  |  |  | 10 | 10 | 9 |
| Probability of occurrence of PDSCH transmission(Note 3) |  |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: of TP1 is set the same as that of TP2. Note 3: PDSCH transmission from TP 1 and TP 2 shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.1.3.5-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.1.3.5-3: Performance Requirements DPS transmission with CRS assistance information

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 FDD | OP.1FDD | OP.1FDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 10.7 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of both TP.1 and TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing

The requirements are specified in Table 8.3.1.3.6-3, with the additional parameters in Table 8.3.1.3.6-1 and Table 8.3.1.3.6-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.3.1.3.6-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP1 and TP2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.3.1.3.6-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 1) | Random beamforming (rank 1) |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 2 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW1 is transmitted from TP 1 and CW2 is transmitted from TP 2) |  |  |  |  |

Table 8.3.1.3.6-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.3.1.3.6-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.97 FDD | R.97 FDD | OP.1FDD | OP.1FDD | EPA5 | EPA5 | 2x2 Low | 70 | 11.1 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.4	Performance Requirements for semiOpenLoop transmission

The requirements are specified in Table 8.3.1.4-2, with the addition of the parameters in Table 8.3.1.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank one and rank two performances for full RB allocation upon antenna ports 7 and 8 with higher layer parameter semiOpenLoop is configured.

Table 8.3.1.4-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) when high layer parameter semiOpenLoop is configured

| parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 | Antenna ports 15,16,17,18 |
| Beamforming model |  |  | B.4.3 (Note 3) | B.4.3 (Note 3) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| Rank Number of PDSCH |  |  | 1 | 2 |
| semiOpenLoop |  |  | True | True |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: For 2 antenna ports {15, 16}, the precoding matrix corresponding to codebook index 0 from Table 6.3.4.2.3-1 in [4] with ; For 4 antenna ports, the precoding matrix random seleted from Table 6.3.4.2.3-2 in [4] with. |  |  |  |  |

Table 8.3.1.4-2: Minimum Performance Requirements for CDM-multiplexed DM RS (FRC) when high layer parameter semiOpenLoop is configured

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.86A FDD | OP.1 FDD | EVA 70Hz | 2x2 Medium | 70 | [0.6] | ≥1 |
| 2 | 10 MHz 16QAM 1/2 | R.45A-1 FDD | OP.1 FDD | EVA 70Hz | 4x2 Low | 70 | [14.9] | ≥2 |

### 8.3.2	TDD

The parameters specified in Table 8.3.2-1 are valid for TDD unless otherwise stated.

Table 8.3.2-1: Common Test Parameters for User-specific Reference Symbols

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRB for Transmission mode 8, 1 PRG for Transmission modes 9 and 10 Time domain: 1 ms |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4] Note 2: as specified in Table 4.2-1 in TS 36.211 [4] |  |  |  |  |  |

#### 8.3.2.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna port 5, the requirements are specified in Table 8.3.2.1-2, with the addition of the parameters in Table 8.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the demodulation performance using user-specific reference signals with full RB or single RB allocation.

Table 8.3.2.1-1: Test Parameters for Testing DRS

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna port 0 |  |  |  |
| Beamforming model |  |  | Annex B.4.1 |  |  |  |
| at antenna port |  | dB/15kHz | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | OCNG (Note 2) | OCNG (Note 2) | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 7 | 7 | 7 | 7 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.2.1-2: Minimum performance DRS (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.25 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | -0.8 | ≥1 |
| 2 | 10 MHz 16QAM 1/2 | R.26 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 7.0 | ≥2 |
|  | 5MHz 16QAM 1/2 | R.26-1 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 7.0 | 1 |
| 3 | 10 MHz 64QAM 3/4 | R.27 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 17.0 | ≥2 |
|  | 10 MHz 64QAM 3/4 | R.27-1 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 17.0 | 1 |
| 4 | 10 MHz 16QAM 1/2 | R.28 TDD | OP.1 TDD | EPA5 | 2x2 Low | 30 | 1.7 | ≥1 |

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 2B, the requirements are specified in Table 8.3.2.1-4 and 8.3.2.1-5, with the addition of the parameters in Table 8.3.2.1-3 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8 with and without a simultaneous transmission on the other antenna port.

Table 8.3.2.1-3: Test Parameters for Testing CDM-multiplexed DM RS (single layer)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna port 0 and antenna port 1 |  |  |  |  |
| Beamforming model |  |  | Annex B.4.1 |  |  |  |  |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) |
| Simultaneous transmission |  |  | No | No | No | Yes (Note 3, 5) | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 8 | 8 | 8 | 8 | 8 |
| Note 1: . Note 2: The modulation symbols of the signal under test is mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |  |  |  |  |

Table 8.3.2.1-4: Minimum performance for CDM-multiplexed DM RS without simultaneous transmission (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.31 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | -1.0 | ≥1 |
| 2 | 10 MHz 16QAM 1/2 | R.32 TDD | OP.1 TDD | EPA5 | 2x2 Medium | 70 | 7.7 | ≥2 |
|  | 5MHz 16QAM 1/2 | R.32-1 TDD | OP.1 TDD | EPA5 | 2x2 Medium | 70 | 7.7 | 1 |
| 3 | 10 MHz 64QAM 3/4 | R.33 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 17.7 | ≥2 |
|  | 10 MHz 64QAM 3/4 | R.33-1 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 17.7 | 1 |

Table 8.3.2.1-5: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 4 | 10 MHz 16QAM 1/2 | R.32 TDD (Note 1) | OP.1 TDD | EPA5 | 2x2 Medium | 70 | 21.9 | ≥2 |
| 5 | 10 MHz 64QAM 1/2 | R.34 TDD (Note 1) | OP.1 TDD | EPA5 | 2x2 Low | 70 | 22.0 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

#### 8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.2.1A-2 and 8.3.2.1A-3, with the addition of the parameters in Table 8.3.2.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8 with and without a simultaneous transmission on the other antenna port, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.3.2.1A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1, Test 1a | Test 2 | Test 3 | Test 4, Test 4a | Test 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 | Antenna ports 15,…,18 | Antenna ports 15,…,18 | Antenna ports 15,…,22 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | 5 / 4 | 5 / 4 | 5 / 4 | 5 / 4 |
| CSI reference signal configuration |  |  | 1 | 3 | 3 | 1 | 3 |
| csi-RS-ConfigZP-ApList |  |  | N/A | NA | NA | {0,1} | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000100000000 | 4 / 0010000000000000 | 4/ 0010000000000000 | 0010000100000000 or 0001000100000000 (Note 7) | N/A |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 | 100 | 50 | 50 |
| Simultaneous transmission |  |  | No | Yes (Note 3, 5) | No | No | No |
| PDSCH transmission mode |  |  | 9 | 9 | 9 | 9 | 9 |
| Number of MBSFN subframes |  | Subframes | 2 (Note 6) | NA | NA | 2 (Note 7) | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. Note 6: For TDD mode, 2 subframes (#4/9) are allocated as MBSFN subframes. Note 7: Through DCI signalling, indicating aperiodic ZP-CSI-RS in sub-frames #4 and #9 per frame, the indicated aperiodic ZP-CSI-RS is random selected from RRC-configured AP ZP CSIRS list. |  |  |  |  |  |  |  |

Table 8.3.2.1A-2: Minimum performance for CDM-multiplexed DM RS without simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | UE DL Cat-egory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz QPSK 1/3 | R.50-1 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | -0.73 | ≥1 | ≥6 |
| 1a | 10 MHz QPSK 1/3 | R.50-2 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | [-0.6] | ≥1 | ≥6 |
| 3 | 20MHz 256QAM | R. 66 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 24.3 | 11-12 | ≥11 |
| 4 | 10 MHz QPSK 1/3 | R.50-1 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | [-0.73] | ≥1 | ≥6 |
| 4a | 10 MHz QPSK 1/3 | R.50-2 TDD | OP.1 TDD | EVA5 | 2x2 Low | [70] | TBD | ≥1 | ≥6 |
| 5 | 10MHz 1024QAM | R.101 TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | 29.0 | TBD | 20, ≥ 22 |
| Note 1: For UE that does not support aperiodic ZP-CSI-RS, test 1a will be run and test 1 will be skipped if the UE indicates support of pdsch-CollisionHandling-r13. Otherwise, test 1 will be run and test 1a will be skipped. For UE that does not support aperiodic ZP-CSI-RS, test 4 and test 4a will be skipped. Note 2: For UE that supports AP ZP CSIRS, test 4a will be run and test 4 will be skipped if the UE indicates support of pdsch-CollisionHandling-r13. Otherwise, test 4 will be run and test 4a will be skipped. For UE that supports AP ZP CSIRS, test 1 and test 1a will be skipped. |  |  |  |  |  |  |  |  |  |

Table 8.3.2.1A-3: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 2 | 10 MHz 64QAM 1/2 | R.44 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 22.1 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

#### 8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model

The requirements are specified in Table 8.3.2.1B-2, with the addition of the parameters in Table 8.3.2.1B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed-loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.3.2.1B-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1and Cell 2, respectively.

Table 8.3.2.1B-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 126 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 9 | N/A |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  | ms | 10 or 11 | N/A |
| Reporting interval |  | ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000000000000000000001111111111111111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |

Table 8.3.2.1B-2: Enhanced Performance Requirement Type A, CDM-multiplexed DM RS with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.48 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 4x2 Low | 70 | -1.0 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

The requirements are specified in Table 8.3.2.1C-2, with the addition of parameters in Table 8.3.2.1C-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell if the PDSCH transmission in the serving cell takes place in subframes that overlap with ABS [9] of the aggressor cell with CRS assistance information. In Table 8.3.2.1C-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.2.1C-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports) – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  | 1 | 1 | 1 |  |
| Special subframe configuration |  |  | 4 | 4 | 4 |  |
| Downlink power allocation |  | dB | 0 | -3 | -3 |  |
|  |  | dB | 0 (Note 1) | -3 (Note 1) | -3 (Note 1) |  |
|  |  | dB | -3 | N/A | N/A |  |
| at antenna port |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |  |
|  |  | dBm/15kHz | -98 (Note 3) | N/A | N/A |  |
|  |  | dBm/15kHz | -93 (Note 4) | N/A | N/A |  |
|  |  | dB | Reference Value in Table 8.3.2.1C-2 | 12 | 10 |  |
| BWChannel |  | MHz | 10 | 10 | 10 |  |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |  |
| Time Offset between Cells |  | s | N/A | 3 | -1 |  |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |  |
| Cell Id |  |  | 0 | 1 | 126 |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A | N/A |  |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 | N/A | N/A |  |
| ABS pattern (Note 5) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |  |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  | 0000000001 0000000001 | N/A | N/A |  |
| CSI Subframe Sets (Note7) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |  |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |  |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |  |
| PDSCH transmission mode |  |  | TM9-1layer | Note 9 | Note 9 |  |
| Precoding granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |  |
| Beamforming model |  |  | Annex B.4.1 | N/A | N/A |  |
| Cyclic prefix |  |  | Normal | Normal | Normal |  |
| Note 1: . Note 2: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 5: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 6: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 7: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5. Note 10: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 11: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 12: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 13: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 14: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |

Table 8.3.2.1C-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports) – Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51 TDD | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 |  |  | 2x2 Low | 70 | 8.5 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference

The requirements are specified in Table 8.3.2.1D-2, with the addition of the parameters in Table 8.3.2.1D-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 9 interference model defined in clause B.6.4. In 8.3.2.1D-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.2.1D-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | 3 | 3 |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 9 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 4 | 10 / 4 | 10 / 4 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 9 / 1000000000000000 | 9 / 0100000000000000 | 9 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.2.1D-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.0 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model

The requirements are specified in Table 8.3.2.1E-2, with the addition of the parameters in Table 8.3.2.1E-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by the CRS of the interfering cell, applying the CRS interference model defined in clause B.6.5. In 8.3.2.1E-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.2.1E-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with CRS interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | 3 | 3 |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 8 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Precoding |  |  |  | Random wideband precoding per TTI | N/A | N/A |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 3) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.2.1E-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with CRS interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.71 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 14.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference

The requirements are specified in Table 8.3.2.1F-2, with the addition of the parameters in Table 8.3.2.1F-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of two interfering cells applying transmission mode 3 interference model defined in clause B.6.2. In 8.3.2.1F-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.2.1F-1: Test Parameters for Testing CDM-multiplexed DM RS (Single-layer) with TM3 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | -3 | -3 |
|  |  |  | dB | 0 (Note 1) | -3 | -3 |
|  |  |  | dB | -3 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 3.28 | 0.74 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | Random from set {1,2,3} | Random from set {1,2,3} |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | Random from set {1,2} | Random from set {1,2} |
| PDSCH transmission mode |  |  |  | 8 | 3 | 3 |
| Interference model |  |  |  | N/A | As specified in clause B.6.2 | As specified in clause B.6.2 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.2 | As specified in clause B.6.2 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.2.1F-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.70 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x2 Low | 85 | 11.3 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SNR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model

The requirements are specified in Table 8.3.2.1G-2, with the addition of the parameters in Table 8.3.2.1G-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission configured with TM10 in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.6.3. The NAICS network assistance is provided when the serving cell TM10 is configured with QCL-type A and PCID based DM-RS scrambling. The neighbouring cell has transmission mode TM9 and NeighCellsInfo-r12 for interfering cell indicates presence of TM9. In 8.3.2.1G-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.3.2.1G-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) Multiplexing with TM10 serving cell configuration and TM9 interference model

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Number of control OFDM symbols in normal subframes |  |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH in normal subframes |  |  |  | 3 | 3 | 3 |
| Number of control OFDM symbols in special subframes |  |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH in special subframes |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 10 | 9 | 9 |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Precoding |  |  |  | Random wideband precoding per TTI | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI reference signals |  |  |  | Antenna ports 15, 16, 17, 18 | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 10 / 4 | 10 / 4 | 10 / 4 |
| CSI reference signal configuration |  |  |  | 5 | 6 | 7 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 9 / 1000000000000000 | 9 / 0100000000000000 | 9 / 0010000000000000 |
| Time offset to cell 1 |  |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| MBSFN |  |  |  | Not configured | Not configured | Not configured |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 3: CSI-RS configurations are according to [4] subclause 6.10.5.2. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 8.3.2.1G-2: Minimum Performance for Enhanced Performance Requirement Type B, CDM-multiplexed DM RS Multiplexing with TM10 serving cell configuration and TM9 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration |  |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.69 TDD | OP.1 TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x2 Low | 2x2 Low | 2x2 Low | 85 | 18.0 | ≥1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.2.1H-2, with the addition of parameters in Table 8.3.2.1H-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.2.1H-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell 2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.2.1H-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  |  | dB | -3 | -3 | -3 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.3.2.1H-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset to Cell 1 |  |  | s | N/A | 3 | -1 |
| Frequency shift to Cell 1 |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 126 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal configuration |  |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 4 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | TM9-1layer | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| Precoding granularity |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  |  | Annex B.4.1 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 5: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 6: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |  |

Table 8.3.2.1H-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51-1 TDD | OP.1TDD | N/A | N/A | EVA5 |  |  | 2x2 Low | 70 | 11.9 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)

For single-layer transmission on antenna port 7, 8, 11 or 13 upon detection of a PDCCH with DCI format 2C, the requirement is specified in Table 8.3.2.1I-2, with the addition of the parameters in Table 8.3.2.1I-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of the test is to verify rank-1 performance on antenna port 11 with a simultaneous transmission on the antenna port 7, 8 or 13 with DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.3.2.1I-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| dmrs-Enhancements-r13 |  |  | Enable |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 11. Note 3: Modulation symbols of an interference signal are random mapped onto one antenna port among antenna port 7, 8 and 13. The upadate granularity for randomized mapping antenna port is 1 PRG in frequency domain and 1ms in time domain. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 with OCC =4. |  |  |  |

Table 8.3.2.1I-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.44 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 22.1 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

#### 8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)

For single-layer transmission on a DMRS antenna port upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.2.1J-1, with the addition of the parameters in Table 8.3.2.1J-2 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one DMRS antenna port with a simultaneous transmission on one of the other DMRS antenna port with or without DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.3.2.1J-1: Test Parameters for Minimun Performance Requirement - Single-layer Spatial Multiplexing with assistance information for simultaneous transmition interfering PDSCH (FRC)

| parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 (Note 1) |  |  |
|  |  | dB | -3 |  |  |
| Beamforming model |  |  | Annex B.4.1 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |  |  |
| CSI reference signal configuration |  |  | 3 |  |  |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |  |  |
| Number of allocated resource blocks |  | PRB | 50 |  |  |
| Simultaneous transmission |  |  | Yes |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |
| Number of MBSFN subframes |  | Subframes | NA |  |  |
| dmrs-Enhancements-r13 |  |  | Disabled | Enable | Enable |
| k-max-r14 (Note 5) |  |  | 1 | 1 | 3 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: In Test 1, the modulation symbols of the signal under test are mapped randomly onto antenna port 7 or 8. Modulation symbols of an interference signal are mapped randomly onto one of the remaining DMRS antenna port. The two UEs’ scrambling identities are set to 0 with OCC =2. Note 4: In Test 2 and Test 3, the modulation symbols of the signal under test are mapped randomly onto antenna port 7, 8, 11 or 13. Modulation symbols of an interference signal are mapped randomly onto one of the remaining DMRS antenna port. The two UEs’ scrambling identities are set to 0 with OCC =4. Note 5: k-max-r14 is decribed in subclause 6.3.2 of [7]. Note 6: The upadate granularity for randomized mapping antenna port is 50 PRGs in frequency domain and 1ms in time domain. |  |  |  |  |  |

Table 8.3.2.1J-2: Minimum performance for Minimun Performance Requirement - Single-layer Spatial Multiplexing with assistance information for simultaneous transmition interfering PDSCH (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.cc TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | 15.2 | ≥2 |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

#### 8.3.2.2	Dual-Layer Spatial Multiplexing

For dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2B, the requirements are specified in Table 8.3.2.2-2, with the addition of the parameters in Table 8.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation.

Table 8.3.2.2-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Cell-specific reference symbols |  |  | Antenna port 0 and antenna port 1 |  |
| Beamforming model |  |  | Annex B.4.2 |  |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | OCNG (Note 2) |
| Number of allocated resource blocks |  | PRB | 50 | 50 |
| PDSCH transmission mode |  |  | 8 | 8 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.2.2-2: Minimum performance for CDM-multiplexed DM RS (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.31 TDD | OP.1 TDD | EVA5 | 2x2 Low | 70 | 4.5 | ≥2 |
| 2 | 10 MHz 16QAM 1/2 | R.32 TDD | OP.1 TDD | EPA5 | 2x2 Medium | 70 | 21.7 | ≥2 |

#### 8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing

The requirements are specified in Table 8.3.2.2A-2, with the addition of the parameters in Table 8.3.2.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation upon antenna ports 7 and 8.

Table 8.3.2.2A-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference symbols |  |  | Antenna port 0 and antenna port 1 |
| Beamforming model |  |  | Annex B.4.2 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks |  | PRB | 50 |
| PDSCH transmission mode |  |  | 8 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.3.2.2A-2: Enhanced Performance Requirement Type C for CDM-multiplexed DM RS (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.32 TDD | OP.1 TDD | EPA5 | 2x2 Medium | 70 | 17.0 | ≥2 |

#### 8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)

For dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.3.2.3-2, with the addition of the parameters in Table 8.3.2.3-1 where Cell 1 is the serving cell and Cell 2 is the interfering cell. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power, and to verify that the UE correctly estimate SNR.

Table 8.3.2.3-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| # Parameter |  | # Unit | # Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | # Cell 1 | # Cell 2 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
|  | PDSCH_RA | dB | 4 | NA |
|  | PDSCH_RB | dB | 4 | NA |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 | 126 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  |  | Reference Value in Table 8.3.2.3-2 | Test specific, 7.25dB |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | Blanked |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.2.3-2: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 TDD | OP.1 TDD | N/A | ETU5 | ETU5 | 2x2 Low | 70 | 14.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports

##### 8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)

The requirements are specified in Table 8.3.2.4.1-3, with the additional parameters in Table 8.3.2.4.1-1 and Table 8.3.2.4.1-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission point share the same Cell ID. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6], configured according to Table 8.3.2.4.1-2. In Tables 8.3.2.4.1-1 and 8.3.2.4.1-2, transmission point 1 (TP 1) is the serving cell and transmission point 2 (TP 2) transmits PDSCH. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.2.4.1-1: Test Parameters for quasi co-location type B: same Cell ID

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | (Note 2) |
| CSI-RS 0 antenna ports |  |  | NA | Port {15,16} |
| qcl-CSI-RS-ConfigNZPId-r11, CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | NA | 5/4 |
| qcl-CSI-RS-ConfigNZPId-r11, CSI-RS 0 configuration |  |  | NA | 8 |
| csi-RS-ConfigZPId-r11, Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | NA | 4/ 0000010000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  | dB | Reference point in Table 8.3.2.4.1-3 | Reference point in Table 8.3.2.4.1-3 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 0 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | Blanked | 10 |
| Number of allocated PRB |  | PRB | NA | 50 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ |  |
| Time offset between TPs |  | s | NA | Reference point in Table 8.3.2.4.1-3 |
| Frequency error between TPs |  | Hz | NA | 0 |
| Beamforming model |  |  | NA | Port 7 as specified in clause B.4.1 |
| Symbols for unused PRBs |  |  | NA | OCNG (Note 3) |
| Note 1: Noet 2: REs for antenna ports 0 and 1 have zero transmission power. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.2.4.1-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.2.4.1-3: Minimum performance for quasi co-location type B: same Cell ID

| Test Number | Reference Channel | OGCN pattern |  | Time offset between TPs (s) | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 |  | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52 TDD | NA | OP.1 TDD | 2 | EPA5 | EPA5 | 2x2 Low | 70 | 12 | ≥2 |
| 2 | R.52 TDD | NA | OP.1 TDD | -0.5 | EPA5 | EPA5 | 2x2 Low | 70 | 12.4 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for TP 1 and TP 2. Note 3: SNR corresponds to of TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)

The requirements are specified in Table 8.3.2.4.2-3, with the additional parameters in Tables 8.3.2.4.2-1 and 8.3.2.4.2-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission point share the same Cell ID. In particular the test verifies that the UE, configured with quasi co-location type B, performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6].  In Tables 8.3.2.4.2-1 and 8.3.2.4.2-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, and transmission point 2 (TP 2) has same Cell ID as TP 1. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between 2 TPs with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.2.4.2-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.2.4.2-1: Test Parameters for timing offset compensation with DPS transmission

| parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | (Note 2) |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A |
| CSI reference signal 0 configuration |  |  | 0 | N/A |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 4 |
| CSI reference signal 1 configuration |  |  | N/A | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 4/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  | Subframes/bitmap | N/A | 4/ 0000010000000000 |
|  |  | dB | Reference Value in Table 8.3.2.4.2-3 | Reference Value in Table 8.3.2.4.2-3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 0 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| Timing offset between TPs |  |  | N/A | Reference Value in Table 8.3.2.4.2-3 |
| Frequency offset between TPs |  | Hz | N/A | 0 |
| Number of allocated resource blocks |  | PRB | 50 | 50 |
| PDSCH transmission mode |  |  | 10 | 10 |
| Probability of occurrence of PDSCH transmission(Note 3) |  | % | 30 | 70 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) |
| Note 1: Note 2: REs for antenna ports 0 and 1 have zero transmission power. Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.2.4.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.2.4.2-3: Performance Requirements for timing offset compensation with DPS transmission

| Test Number | Timing offset(us) | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | 2 | R.53 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | 2x2 Low | 70 | 12.3 | ≥2 |
| 2 | -0.5 | R.53 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | 2x2 Low | 70 | 12.5 | ≥2 |
| Note 1: The propagation conditions for TP 1and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of both TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)

The requirements are specified in Table 8.3.2.4.3-2, with the additional parameters in Table 8.3.2.4.3-1. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where the two transmission points have different Cell ID and colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.3.2.4.3-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, and transmission point 2 (TP 2) transmits PDSCH with different Cell ID. The downlink physical channel setup for TP 1 is according to Table C.3.4-1 and for TP 2 according to Table C.3.4-2.

Table 8.3.2.4.3-1: Test Parameters for quasi co-location type B with different Cell ID and Colliding CRS

| parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | N/A | As specified in clause B.4.2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | N/A | 5 / 4 |
| CSI reference signal 0 configuration |  |  | N/A | 0 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 4/ 0010000000000000 |
|  |  | dB | Reference point in Table 8.3.2.4.3-2 + 4dB | Reference Value in Table 8.3.2.4.3-2 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 126 |
| Number of control OFDM symbols |  |  | 1 | 2 |
| Timing offset between TPs |  | us | N/A | 0 |
| Frequency offset between TPs |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ |  |
| PDSCH transmission mode |  |  | Blank | 10 |
| Number of allocated resource block |  |  | N/A | 50 |
| Symbols for unused PRBs |  |  | N/A | OCNG(Note2) |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.3.2.4.3-2: Performance Requirements for quasi co-location type B with different Cell ID and Colliding CRS

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.54 TDD | N/A | OP.1TDD | EPA5 | ETU5 | 2x2 Low | 70 | 14.7 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)

The requirements are specified in Table 8.3.2.4.4-3, with the additional parameters in Table 8.3.2.4.4-1 and Table 8.3.2.4.4-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference and time difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Table 8.3.2.4.4-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, transmission point 2 (TP 2) transmits PDSCH with different Cell ID, and Transmission point 3 (TP 3) is the aggressor transmission point. The downlink physical channel setup for TP 1 is according to Table C.3.4-1, for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.2.4.4-1: Test Parameters for quasi co-location type B with different Cell ID and non-colliding CRS when CRS assistance information is configured

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | N/A | Port 7 as specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 4 | N/A |
| CSI reference signal 0 configuration |  |  |  | N/A | 0 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | N/A | 4/ 0010000000000000 | N/A |
|  |  |  | dB | 10.45 | Reference Value in Table 8.3.2.4.4-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 1 | 2 | 2 |
| Timing offset to TP 1 |  |  | us | N/A | -0.5 | 3 |
| Frequency offset to TP 1 |  |  | Hz | N/A | 200 | -100 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  |  | Type B, ‘00’ |  | N/A |
| PDSCH transmission mode |  |  |  | Blank | 10 | 9 |
| Number of allocated resource block |  |  |  | N/A | 50 | N/A |
| Symbols for unused PRBs |  |  |  | N/A | OCNG(Note2) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.2.4.4-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.2.4.4-3: Performance Requirements for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 TDD | N/A | OP.1TDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.1 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)

The requirements are specified in Table 8.3.2.4.5-3, with the additional parameters in Tables 8.3.2.4.5-1 and 8.3.2.4.5-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission point have the different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B, performs correct tracking and compensation of the frequency difference and timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Tables 8.3.2.4.5-1 and 8.3.2.4.5-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, Transmission point 2 (TP 2) has different Cell ID as TP 1, and Transmission point 3 (TP3) is the aggressor transmission point. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between TP 1 and TP 2 with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.2.4.5-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1,  for TP 2  is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2

Table 8.3.2.4.5-1: Test Parameters for DPS transmission with CRS assistance information

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal 0 configuration |  |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 4 | N/A |
| CSI reference signal 1 configuration |  |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | 4/ 0010000000000000 | N/A | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  |  | Subframes/bitmap | N/A | 4/ 0000010000000000 | N/A |
| (Note 2) |  |  | dB | Reference Value in Table 8.3.2.4.5-3 | Reference Value in Table 8.3.2.4.5-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Timing offset to TP 1 |  |  |  | N/A | -0.5 | 3 |
| Frequency offset to TP 1 |  |  | Hz | N/A | 200 | -100 |
| Number of allocated resource blocks |  |  | PRB | 50 | 50 | N/A |
| PDSCH transmission mode |  |  |  | 10 | 10 | 9 |
| Probability of occurrence of PDSCH transmission(Note 3) |  |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: of TP 1 is set the same as that of TP 2. Note 3: PDSCH transmission from TP 1 and TP 2 shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.2.4.5-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.2.4.5-3: Performance Requirements for DPS transmission with CRS assistance information

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 TDD | OP.1TDD | OP.1TDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.2 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of both TP.1 and TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing

The requirements are specified in Table 8.3.2.4.6-3, with the additional parameters in Table 8.3.2.4.6-1 and Table 8.3.2.4.6-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.3.2.4.6-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP 1 and TP 2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.3.2.4.6-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 1) | Random beamforming (rank 1) |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 4 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 4/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 4/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW1 is transmitted from TP 1 and CW2 is transmitted from TP 2) |  |  |  |  |

Table 8.3.2.4.6-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.3.2.4.6-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.97 TDD | [R.97 TDD | OP.1TDD | OP.1TDD | EPA5 | EPA5 | 2x2 Low | 70 | 10.7 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

#### 8.3.2.5	Performance Requirements for semiOpenLoop transmission

The requirements are specified in Table 8.3.2.5-2, with the addition of the parameters in Table 8.3.2.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank one and rank two performances for full RB allocation upon antenna ports 7 and 8 with higher layer parameter semiOpenLoop is configured.

Table 8.3.2.5-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) when high layer parameter semiOpenLoop is configured

| parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 | Antenna ports 15,16,17,18 |
| Beamforming model |  |  | B.4.3 (Note 3) | B.4.3 (Note 3) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | 5 / 4 |
| CSI reference signal configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4/ 0010000000000000 | 4/ 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| Rank Number of PDSCH |  |  | 1 | 2 |
| semiOpenLoop |  |  | True | True |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: For 2 antenna ports {15, 16}, the precoding matrix corresponding to codebook index 0 from Table 6.3.4.2.3-1 in [4] with ; For 4 antenna ports, the precoding matrix random seleted from Table 6.3.4.2.3-2 in [4] with |  |  |  |  |

Table 8.3.2.5-2: Minimum Performance Requirements for CDM-multiplexed DM RS (FRC) when high layer parameter semiOpenLoop is configured

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.76A TDD | OP.1 TDD | EVA 70Hz | 2x2 Medium | 70 | [0.9] | ≥1 |
| 2 | 10 MHz 16QAM 1/2 | R.61A TDD | OP.1 TDD | EVA 70Hz | 4x2 Low | 70 | [15.6] | ≥2 |

### 8.3.3	LAA

#### 8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS

##### 8.3.3.1.1	FDD PCell (FDD single carrier)

The parameters specified in Table 8.3.3.1.1-1 are valid for FDD CC and LAA SCell(s) unless otherwise stated. And the additional parameters specified in Table 8.3.3.1.1-2 are valid for LAA SCell(s).

Table 8.3.3.1.1-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission (Note 1) |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms |  |
| Note 1: For retransmission in partial subframes, the TB size should be kept the same as the initial transmission regardless of the initial transmission is performed in full subframes or partial subframes. Note 2: Void. |  |  |  |  |  |

Table 8.3.3.1.1-2: Addtional Test Parameters for LAA SCell(s)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| DMTC Periodicity |  |  | ms | 80 |  |
| dmtc-PeriodOffset-r12 ms80-r12 |  |  |  | 0 |  |
| Discovery signal occasion duration |  |  | subframe | 1 |  |
| Power allocation of discovery signal |  |  |  | Same as power allocation of CRS within a transmission burst in the test |  |

For CA with LAA SCell(s), the requirements for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C are specified in Table 8.3.3.1.1-7, with the addition of the parameters in Table 8.3.3.1.1-3, Table 8.3.3.1.1-4 and Table 8.3.3.1.1-5. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation for CA with LAA SCell(s).

Table 8.3.3.1.1-3: Test Parameters for Large Delay CDD (FRC) for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | TM3 |
| Subframe configuration |  |  | Non-MBSFN |
| NOTE 1: . NOTE 2: PUCCH format 3 is used to feedback ACK/NACK. NOTE 3: Void. |  |  |  |

Table 8.3.3.1.1-4: Test Parameters for CDM-multiplexed DM RS (dual layer) for CA with LAA SCell(s)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5/2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 9 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  |  | {1,3,5,8} |
| Occupied OFDM symbols set in the last subframe |  |  | {6,9,12,14} |
| Random variable p defined in B.8 |  |  | 0.5 |
| timing error relative of LAA SCell to PCell |  | μs | 0 |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | 200 |
| Note 1: Note 2: These physical resource blocks are assigned only within burst transmissions to a LAA UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.3.3.1.1-5: Single carrier performance Large Delay CDD (FRC) for PCell for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Norminal maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3MHz | R.11-6 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.3.3.1.1-6: Single carrier performance for CDM-multiplexed DM RS (dual layer) for LAA SCell for multiple CA configurations

| Band-width | Sub-test (Note 2) | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | 1 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.1 |
|  | 2 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14 |
|  | 3 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.2 |
|  | 4 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.2 |
| Note 1: Fraction of nominal maximum throughput is calculated based on random occasions of LAA PDSCH transmission. Note 2: An UE is required to fulfill only one test of Sub-test 1-4 depending on UE capabilities of endingDwPTS and secondSlotStartingPosition. For an UE not supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 1; For an UE not supporting endingDwPTS but supporting secondSlotStartingPosition, it is required to fulfill Sub-test 2; For an UE supporting endingDwPTS but not supporting secondSlotStartingPosition, it is required to fulfil Sub-test 3; and For an UE supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 4. |  |  |  |  |  |  |  |

Table 8.3.3.1.1-7: Minimum performance (FRC) based on single carrier performance for CA with one LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+20 | 15 | 20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+20 | 10 | 20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.1-8: Minimum performance (FRC) based on single carrier performance for CA with two LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+2x20 | 15 | 2x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+2x20 | 10 | 2x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.1-9: Minimum performance (FRC) based on single carrier performance for CA with three LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+3x20 | 15 | 3x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+3x20 | 10 | 3x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.1-10: Minimum performance (FRC) based on single carrier performance for CA with four LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 5x20 | 20 | 4x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+4x20 | 15 | 4x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+4x20 | 10 | 4x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.1-11: Minimum performance (FRC) based on single carrier performance for CA with five LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 6x20 | 20 | 5x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+5x20 | 15 | 5x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+5x20 | 10 | 5x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.1-12: Minimum performance (FRC) based on single carrier performance for CA with six LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 7x20 | 20 | 6x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 2 | 15+6x20 | 15 | 6x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| 3 | 10+6x20 | 10 | 6x20 | As defined in Table 8.3.3.1.1-5 and Table 8.3.3.1.1-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.1-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.1-6 for LAA SCell. |  |  |  |  |  |

##### 8.3.3.1.2	TDD Pcell (TDD single carrier)

The parameters specified in Table 8.3.3.1.2-1 are valid for TDD CC and LAA SCell(s) unless otherwise stated. And the additional parameters specified in Table 8.3.3.1.2-2 are valid for LAA SCell(s).

Table 8.3.3.1.2-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission (Note 3) |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG for Transmission modes 9 Time domain: 1 ms |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For retransmission in partial subframes, the TB size should be kept the same as the initial transmission regardless of the initial transmission is performed in full subframes or partial subframes. |  |  |  |  |  |

Table 8.3.3.1.2-2: Addtional Test Parameters for LAA SCell(s)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| DMTC Periodicity |  |  | ms | 80 |  |
| dmtc-PeriodOffset-r12 ms80-r12 |  |  |  | 0 |  |
| Discovery signal occasion duration |  |  | subframe | 1 |  |
| Power allocation of discovery signal |  |  |  | Same as power allocation of CRS within a transmission burst in the test |  |

For CA with LAA SCell(s), the requirements for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C are specified in Table 8.3.3.1.2-7, with the addition of the parameters in Table 8.3.3.1.2-3, Table 8.3.3.1.2-4 and Table 8.3.3.1.2-5,  The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation for CA with LAA SCell(s).

Table 8.3.3.1.2-3: Test Parameters for Large Delay CDD (FRC) for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | TM3 |
| Subframe configuration |  |  | Non-MBSFN |
| NOTE 1: . NOTE 2: PUCCH format 3 is used to feedback ACK/NACK. NOTE 3: Void. |  |  |  |

Table 8.3.3.1.2-4: Test Parameters for CDM-multiplexed DM RS (dual layer) for LAA SCell(s)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5/4 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4/ 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 9 |
| DL Burst transmission pattern for LAA SCell |  |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  |  | {1,3,5,8} |
| Occupied OFDM symbols set in the last subframe |  |  | {6,9,12,14} |
| Random variable p defined in B.8 |  |  | 0.5 |
| Timing error relative of LAA SCell to PCell |  | μs | 0 |
| Frequency offset of th i-th LAA SCell relative to PCell |  | Hz | 200 |
| Note 1: Note 2: These physical resource blocks are assigned only within burst transmissions to a LAA UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.3.3.1.2-5: Single carrier performance Large Delay CDD (FRC) for PCell for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.3.3.1.2-6: Single carrier performance for CDM-multiplexed DM RS (dual layer) for LAA SCell(s) for multiple CA configurations

| Band-width | Sub-test (Note2) | Reference channel | OCNG pattern | Propa-gation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Norminal maximum throughput (%) (Note 1) | SNR (dB) |
| 20MHz | 1 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.1 |
|  | 2 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14 |
|  | 3 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.2 |
|  | 4 | R.2 FS3 | OP.1 FS3 | EVA5 | 2x2 Low | 70 | 14.2 |
| Note 1: Fraction of nominal maximum throughput is calculated based on random occasions of LAA PDSCH transmission. Note 2: An UE is required to fulfill only one test of Sub-test 1-4 depending on UE capabilities of endingDwPTS and secondSlotStartingPosition. For an UE not supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 1; For an UE not supporting endingDwPTS but supporting secondSlotStartingPosition, it is required to fulfill Sub-test 2; For an UE supporting endingDwPTS but not supporting secondSlotStartingPosition, it is required to fulfil Sub-test 3; and For an UE supporting both endingDwPTS and secondSlotStartingPosition, it is required to fulfill Sub-test 4. |  |  |  |  |  |  |  |

Table 8.3.3.1.2-7: Minimum performance (FRC) based on single carrier performance for CA with one LAA SCell

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.2-8: Minimum performance (FRC) based on single carrier performance for CA with two LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.2-9: Minimum performance (FRC) based on single carrier performance for CA with three LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.2-10: Minimum performance (FRC) based on single carrier performance for CA with four LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 5x20 | 20 | 4x20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.2-11: Minimum performance (FRC) based on single carrier performance for CA with five LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 6x20 | 20 | 5x20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

Table 8.3.3.1.2-12: Minimum performance (FRC) based on single carrier performance for CA with six LAA SCells

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement (Note 2) | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | PCell | LAA SCell |  |  |
| 1 | 7x20 | 20 | 6x20 | As defined in Table 8.3.3.1.2-5 and Table 8.3.3.1.2-6 | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3D. Note 2: Apply a per-CC requirement defined in Table 8.3.3.1.2-5 for PCell and apply a per-CC requirement defined in Table 8.3.3.1.2-6 for LAA SCell. |  |  |  |  |  |

## 8.4	Demodulation of PDCCH/PCFICH

The receiver characteristics of the PDCCH/PCFICH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). PDCCH and PCFICH are tested jointly, i.e. a miss detection of PCFICH implies a miss detection of PDCCH.

### 8.4.1	FDD

The parameters specified in Table 8.4.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.4.1-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1 | 1 |
| PHICH duration |  |  | Normal | Normal |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | 0 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

#### 8.4.1.1	Single-antenna port performance

For the parameters specified in Table 8.4.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.1.1-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 8 CCE | R.15 FDD | OP.1 FDD | ETU70 | 1x2 Low | 1 | -1.7 |

#### 8.4.1.2	Transmit diversity performance

##### 8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.4.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.1.2.1-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 FDD | OP.1 FDD | EVA70 | 2 x 2 Low | 1 | -0.6 |

##### 8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.4.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.1.2.2-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 5 MHz | 2 CCE | R.17 FDD | OP.1 FDD | EPA5 | 4 x 2 Medium | 1 | 6.3 |

##### 8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

For the parameters for non-MBSFN ABS specified in Table 8.4.1-1 and Table 8.4.1.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.3-2. The downlink physical setup is in accordance with Annex C.3.2 and Annex C.3.3. In Table 8.4.1.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

For the parameters for MBSFN ABS specified in Table 8.4.1-1 and Table 8.4.1.2.3-3, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.3-4. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.4.1.2.3-1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.3-2 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 00000100 00000100 00000100 01000100 00000100 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 00000100 00000100 00000100 00000100 00000100 | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 00000100 00000100 00000100 01000100 00000100 | N/A |
|  | CCSI,1 |  | 11111011 11111011 11111011 10111011 11111011 | N/A |
| Number of control OFDM symbols |  |  | 3 | 3 |
| PHICH Ng (Note 9) |  |  | 1 | N/A |
| PHICH duration |  |  | Extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PDCCH/PCFICH other than that associated with SIB1/Paging are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]; Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]; Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in the test. Note 9: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.4.1.2.3-2: Minimum performance PDCCH/PCFICH – Non-MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 2) |
| 1 | 8 CCE | R15-1 FDD | OP.1 FDD | OP.1 FDD | EVA5 | EVA5 | 2x2 Low | 1 | -3.9 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

Table 8.4.1.2.3-3: Test Parameters for PDCCH/PCFICH – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.3-4 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 126 |
| ABS pattern (Note 4) |  |  | N/A | 0001000000 0100000010 0000001000 0000000000 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0001000000 0100000010 0000001000 0000000000 | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0001000000 0100000010 0000001000 0000000000 | N/A |
|  | CCSI,1 |  | 1110111111 1011111101 1111110111 1111111111 | N/A |
| MBSFN Subframe Allocation (Note 9) |  |  | N/A | 001000 100001 000100 000000 |
| Number of control OFDM symbols |  |  | 3 | 3 |
| PHICH Ng (Note 11) |  |  | 1 | N/A |
| PHICH duration |  |  | extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. The 4th, 12th, 19th and 27th subframes indicated by ABS pattern are MBSFN ABS subframes. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the MBSFN ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in this test. Note 9: MBSFN Subframe Allocation as defined in [7], four frames with 24 bits is chosen for MBSFN subframe allocation. Note 10: The maximum number of uplink HARQ transmission is ≤ 2 so that each PHICH channel transmission is in a subframe protected by MBSFN ABS in this test. Note 11: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.4.1.2.3-4: Minimum performance PDCCH/PCHICH – MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 2) |
| 1 | 8 CCE | R15-1 FDD | OP.1 FDD | OP.1 FDD | EVA5 | EVA5 | 2x2 Low | 1 | -4.2 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

For the parameters for non-MBSFN ABS specified in Table 8.4.1-1 and Table 8.4.1.2.4-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.4-2.

For the parameters for MBSFN ABS specified in Table 8.4.1-1 and Table 8.4.1.2.4-3, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.4-4.

In Tables 8.4.1.2.4-1 and 8.4.1.2.4-3, Cell 1 is the serving cell, and Cell 2 and Cell3are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.4.1.2.4-1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98(Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.4-2 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 00000100 00000100 00000100 00000100 00000100 | 00000100 00000100 00000100 00000100 00000100 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 00000100 00000100 00000100 00000100 00000100 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 00000100 00000100 00000100 00000100 00000100 | N/A | N/A |
|  | CCSI,1 |  | 11111011 11111011 11111011 11111011 11111011 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| PHICH Ng (Note 10) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PDCCH/PCFICH other than that associated with SIB1/Paging are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]; Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]; Note 7: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 8: The number of the CRS ports in Cell1, Cell2 and Cell 3is the same. Note 9: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. Note 10 According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |  |

Table 8.4.1.2.4-2: Minimum performance PDCCH/PCFICH – Non-MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell3 |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 8 CCE | R.15-2 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 1 | -2.2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.4.1.2.4-3: Test Parameters for PDCCH/PCFICH – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98(Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.4-4 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN | MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 0001000000 0100000010 0000001000 0000000000 | 0001000000 0100000010 0000001000 0000000000 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0001000000 0100000010 0000001000 0000000000 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0001000000 0100000010 0000001000 0000000000 | N/A | N/A |
|  | CCSI,1 |  | 1110111111 1011111101 1111110111 1111111111 | N/A | N/A |
| MBSFN Subframe Allocation (Note 7) |  |  | N/A | 001000 100001 000100 000000 | 001000 100001 000100 000000 |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| PHICH Ng (Note 12) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. The 4th, 12th, 19th and 27th subframes indicated by ABS pattern are MBSFN ABS subframes. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the MBSFN ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 7: MBSFN Subframe Allocation as defined in [7], four frames with 24 bits are chosen for MBSFN subframe allocation. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: The maximum number of uplink HARQ transmission is ≤ 2 so that each PHICH channel transmission is in a subframe protected by MBSFN ABS in this test. Note 10: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 11: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 12: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |  |

Table 8.4.1.2.4-4: Minimum performance PDCCH/PCFICH – MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell3 |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 8 CCE | R.15-2 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 1 | -2.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network

The test purpose is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with interference model defined in clause B.5.2. For the parameters specified in Table 8.4.1-1 and Table 8.4.1.2.5-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.5-2 for the Enhanced Downlink Control Channel Performance Requirement Type A. In Table 8.4.1.2.5-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is not provided.

Table 8.4.1.2.5-1: Test Parameters for PDCCH/PCFICH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA PDSCH_RA OCNG_RA |  | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB PDSCH_RB OCNG_RB |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  |  | 3 | 3 | 3 |
| PHICH Ng (Note 1) |  |  |  | 1 | N/A | N/A |
| PHICH duration |  |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  |  | 4 | 3 | 3 |
| Interference model |  |  |  | N/A | As specified in clause B.5.2 | As specified in clause B.5.2 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 80 | 80 |
|  |  | Rank 2 | % | N/A | 20 | 20 |
| Unused RE-s and PRB-s |  |  |  | OCNG | OCNG | OCNG |
| Time offset relative to Cell 1 |  |  | ms | N/A | 0.33 | 0.67 |
| Frequency shift relative to Cell 1 |  |  | Hz | N/A | 0 | 0 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. |  |  |  |  |  |  |

Table 8.4.1.2.5-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.16-1 FDD | OP.1 FDD | EVA70 | EVA70 | EVA70 | 2x2 Low | 1 | 16.5 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.1-1 and Table 8.4.1.2.6-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.6-2. In Table 8.4.1.2.6-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.1.2.6-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 3 | 3 | 3 |
| CFI indicated in PCFICH |  |  | 3 | 3 | 3 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.1.2.6-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-2 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 12.8 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type B for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.1-1 and Table 8.4.1.2.7-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.7-2. In Table 8.4.1.2.7-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.1.2.7-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 6 | 1 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.1.2.7-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.16-3 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 12.7 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type B for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.1-1 and Table 8.4.1.2.8-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.8-2. In Table 8.4.1.2.8-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.1.2.8-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  |  | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.1.2.8-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-4 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 10.3 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.4.1-1 and Table 8.4.1.2.9-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.1.2.9-2. In Table 8.4.1.2.9-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes information on Cell 2 and Cell 3.

Table 8.4.1.2.8-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.4.1.2.9-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH |  |  | 2 | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.1.2.9-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.17-1 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 4x2 Low | 1 | 14.1 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

### 8.4.2	TDD

The parameters specified in Table 8.4.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.4.2-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 0 | 0 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| PHICH Ng (Note 3) |  |  | 1 | 1 |
| PHICH duration |  |  | Normal | Normal |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | 0 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

#### 8.4.2.1	Single-antenna port performance

For the parameters specified in Table 8.4.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.2.1-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 8 CCE | R.15 TDD | OP.1 TDD | ETU70 | 1x2 Low | 1 | -1.6 |

#### 8.4.2.2	Transmit diversity performance

##### 8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.4.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.2.2.1-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 TDD | OP.1 TDD | EVA70 | 2 x 2 Low | 1 | 0.1 |

##### 8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.4.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.2.2.2-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 5 MHz | 2 CCE | R.17 TDD | OP.1 TDD | EPA5 | 4 x 2 Medium | 1 | 6.5 |

##### 8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

For the parameters for non-MBSFN ABS specified in Table 8.4.2-1 and Table 8.4.2.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.3-2. The downlink physical setup is in accordance with Annex C.3.2 and Annex C.3.3.. In Table 8.4.2.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

For the parameters for MBSFN ABS specified in Table 8.4.2-1 and Table 8.4.2.2.3-3, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.3-4. The downlink physical channel setup for Cell 1 is according to Annex C3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.4.2.2.3-1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.4.2.2.3-2 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | μs | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 0000010001 0000000001 |
| RLM/RRM Measurement Subframe Pattern(Note 5) |  |  | 0000000001 0000000001 | N/A |
| CSI Subframe Sets(Note 6) | CCSI,0 |  | 0000010001 0000000001 | N/A |
|  | CCSI,1 |  | 1100101000 1100111000 | N/A |
| Number of control OFDM symbols |  |  | 3 | 3 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PHICH Ng (Note 9) |  |  | 1 | N/A |
| PHICH duration |  |  | extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PDCCH/PCFICH other than that associated with SIB1/Paging are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in the test. Note 9: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.4.2.2.3-2: Minimum performance PDCCH/PCFICH – Non-MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 2) |
| 1 | 8 CCE | R15-1 TDD | OP.1 TDD | OP.1 TDD | EVA5 | EVA5 | 2x2 Low | 1 | -3.9 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

Table 8.4.2.2.3-3: Test Parameters for PDCCH/PCFICH – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.4.2.2.3-4 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN |
| Time Offset between Cells |  | μs | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 126 |
| ABS pattern (Note 4) |  |  | N/A | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern(Note 5) |  |  | 0000000001 0000000001 | N/A |
| CSI Subframe Sets(Note 6) | CCSI,0 |  | 0000000001 0000000001 | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A |
| MBSFN Subframe Allocation (Note 9) |  |  | N/A | 000010 |
| Number of control OFDM symbols |  |  | 3 | 3 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PHICH Ng (Note 10) |  |  | 1 | N/A |
| PHICH duration |  |  | extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. The 10th and 20th subframes indicated by ABS pattern are MBSFN ABS subframes.PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the MBSFN ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in this test. Note 9: MBSFN Subframe Allocation as defined in [7], one frame with 6 bits is chosen for MBSFN subframe allocation. Note 10: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.4.2.2.3-4: Minimum performance PDCCH/PCFICH – MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  | Propagation Conditions(Note 1) |  | Correlation Matrix and Antenna Configuration | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 2) |
| 1 | 8 CCE | R15-1 TDD | OP.1 TDD | OP.1 TDD | EVA5 | EVA5 | 2x2 Low | 1 | -4.1 |
| Note 1: The propagation conditions for Cell 1 and Cell2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

For the parameters for non-MBSFN ABS specified in Table 8.4.2-1 and Table 8.4.2.2.4-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.4-2.

For the parameters for MBSFN ABS specified in Table 8.4.2-1 and Table 8.4.2.2.4-3, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.4-4.

In Tables 8.4.2.2.4-1 and 8.4.2.2.4-3, Cell 1 is the serving cell, and Cell 2 and Cell 3are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.4.2.2.4-1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98(Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.2.2.4-2 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| PHICH Ng (Note 10) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PDCCH/PCFICH other than that associated with SIB1/Paging are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]; Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]; Note 7: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 8: The number of the CRS ports in Cell1, Cell2 and Cell 3is the same. Note 9: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. Note 10: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |  |

Table 8.4.2.2.4-2: Minimum performance PDCCH/PCFICH – Non-MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell3 |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 8 CCE | R.15-2 TDD | OP.1 TDD | OP.1 TDD | OP.1 TDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 1 | -2.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.4.2.2.4-3: Test Parameters for PDCCH/PCFICH – MBSFN ABS

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 (Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.2.2.4-4 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | MBSFN | MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| MBSFN Subframe Allocation (Note 7) |  |  | N/A | 000010 | 000010 |
| Number of control OFDM symbols |  |  | 2 | Note 8 | Note 8 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| PHICH Ng (Note 11) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13 of a subframe overlapping with the aggressor ABS. Note 2: This noise is applied in OFDM symbols #0 of a subframe overlapping with the aggressor ABS. Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. The 10th and 20th subframes indicated by ABS pattern are MBSFN ABS subframes. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the MBSFN ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 7: MBSFN Subframe Allocation as defined in [7], one frame with 6 bits is chosen for MBSFN subframe allocation. Note 8: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 9: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 10: SIB-1 will not be transmitted in Cell2 in this test. Note 11: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |  |

Table 8.4.2.2.4-4: Minimum performance PDCCH/PCFICH – MBSFN ABS

| Test Number | Aggregation Level | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell3 |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 8 CCE | R.15-2 TDD | OP.1 TDD | OP.1 TDD | OP.1 TDD | EVA5 | EVA5 | EVA5 | 2x2 Low | 1 | -1.8 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.2-1 and Table 8.4.2.2.5-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.5-2. In Table 8.4.2.2.5-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.2.2.5-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 6 | 1 |
| UL/DL Configuration |  |  | 0 | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 3 for subframes 0 and 5 2 for subframes 1 and 6 |  |  |
| CFI indicated in PCFICH |  |  | 3 for subframes 0 and 5 2 for subframes 1 and 6 |  |  |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  |  | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.2.2.5-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.16-1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 16.1 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.2-1 and Table 8.4.2.2.6-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.6-2. In Table 8.4.2.2.6-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.2.2.6-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| UL/DL Configuration |  |  | 0 | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 3 for subframes 0 and 5 2 for subframes 1 and 6 |  |  |
| CFI indicated in PCFICH |  |  | 3 for subframes 0 and 5 2 for subframes 1 and 6 |  |  |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  |  | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.2.2.6-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-2 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 13.3 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type B for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.2-1 and Table 8.4.2.2.7-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.7-2. In Table 8.4.2.2.7-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.2.2.7-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 6 | 1 |
| UL/DL Configuration |  |  | 0 | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  |  | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.2.2.7-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.16-3 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 13.7 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.2-1 and Table 8.4.2.2.8-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.8-2. In Table 8.4.2.2.8-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.4.2.2.8-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| UL/DL Configuration |  |  | 0 | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  |  | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.2.2.8-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-4 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 1 | 11.2 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.4.1-1 and Table 8.4.2.2.9-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.2.2.9-2. In Table 8.4.2.2.9-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes information on Cell 2 and Cell 3.

Table 8.4.2.2.9-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.4.1.2.9-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| UL/DL Configuration |  |  | 0 | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 |  |  |
| CFI indicated in PCFICH |  |  | 2 |  |  |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.4.2.2.9-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 2 CCE | R.17-1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 4x2 Low | 1 | 14.4 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

### 8.4.3	LAA

The parameters specified in Table 8.4.3-1 are valid for all LAA PDCCH tests unless otherwise stated.

Table 8.4.3-1: Common test Parameters for PDCCH

| Parameter |  | Unit | Transmit diversity |
| --- | --- | --- | --- |
| Downlink power allocation (Note 1) | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| PHICH Ng (Note 1) |  |  | 1 |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG |
| Cell ID |  |  | 0 |
| Cyclic prefix |  |  | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: In LAA Scell(s), PCFICH_RB, PHICH_RA, and PHICH_RB are not available. Note 2: OCNG is applied only within LAA burst. |  |  |  |

#### 8.4.3.1	Transmit diversity performance

##### 8.4.3.1.1	FDD Pcell (FDD single carrier)

8.4.3.1.1.1	Minimum Requirement 2 Tx Antenna Port

The average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.3.1.1.1-2 for Pcell and in Table 8.4.3.1.1.1-3 for LAA Scell(s), with the addition of the parameters in Table 8.4.3-1, and Table 8.4.3.1.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.3.1.1.1-1: Test Parameters for LAA Scell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | 0 |
| Downlink Burst transmission pattern for LAA SCell |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  | {1,3, 5, 8} |
| Uniform random number (p) in the burst model |  | 0.5 |
| Occupied OFDM symbols set in the last subframe |  | {6, 9, 12,14} |
| timing error relative of LAA SCell to PCell | μs | 15 |
| Frequency offset of th i-th LAA SCell relative to PCell | Hz | 200 |
| Note 1: The same PDSCH transmission mode is applied to each component carrier. Note 2: The OCNG shall be applied for the non-scheduled OFDM symbols within the burst, and which OFDM symbols are scheduled within the burst is according to UE capability. |  |  |

Table 8.4.3.1.1.1-2: Single carrier performance for CCs which are not LAA Scells for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 FDD | OP.1 FDD | EVA5 | 2 x 2 Low | 1 | -0.6 |

Table 8.4.3.1.1.1-3: Single carrier performance for LAA Scell(s) for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 MHz | 4 CCE | R.3 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.2 |
| 2 | 20 MHz | 4 CCE | R.3 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.2 |
| 3 | 20 MHz | 4 CCE | R.3 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.3 |
| 4 | 20 MHz | 4 CCE | R.3 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.1 |
| Note1： UE is required to fulfill only one test among test 1-4 depending on it’s capability for endingDwPTS and secondSlotStartingPosition. For UE don’t support endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 1; For UE don’t support endingDwPTS and support secondSlotStartingPosition, it is required to fulfill test 2; For UE support endingDwPTS and don’t support secondSlotStartingPosition, it is required to fulfil test 3; and For UE support both endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 4. |  |  |  |  |  |  |  |  |

##### 8.4.3.1.2	TDD Pcell (TDD single carrier)

8.4.3.1.2.1	Minimum Requirement 2 Tx Antenna Port

The average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.3.1.2.1-2 for Pcell and in Table 8.4.3.1.2.1-3 for LAA Scell(s), with the additional of the parameters in Table 8.4.3-1, and Table 8.4.3.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.3.1.2.1-1: Test Parameters for LAA Scell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | 0 |
| Downlink Burst transmission pattern for LAA SCell |  | As specified in B.8 |
| The number of subframes set (S1) in a burst |  | {1,3, 5, 8} |
| Uniform random number (p) in the burst model |  | 0.5 |
| subframeStartPosition |  | ‘s07’ |
| Occupied OFDM symbols set in the last subframe |  | {6, 9, 12,14} |
| timing error relative of LAA SCell to PCell | μs | 15 |
| Frequency offset of th i-th LAA SCell relative to PCell | Hz | 200 |
| Note 1: The same PDSCH transmission mode is applied to each component carrier. Note 2: The OCNG shall be applied for the non-scheduled OFDM symbols within the burst, and which OFDM symobls are scheduled within the burst is according to UE capability. |  |  |

Table 8.4.3.1.2.1-2: Single carrier performance for CCs which are not LAA Scells for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 TDD | OP.1 TDD | EVA5 | 2 x 2 Low | 1 | -0.6 |

Table 8.4.3.1.2.1-3: Single carrier performance for LAA Scell(s) for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 MHz | 4 CCE | R.4 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.2 |
| 2 | 20 MHz | 4 CCE | R.4 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.2 |
| 3 | 20 MHz | 4 CCE | R.4 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.3 |
| 4 | 20 MHz | 4 CCE | R.4 FS3 | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | 0.1 |
| Note1： UE is required to fulfill only one test among test 1-4 depending on it’s capability for endingDwPTS and secondSlotStartingPosition. For UE don’t support endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 1; For UE don’t support endingDwPTS and support secondSlotStartingPosition, it is required to fulfill test 2; For UE support endingDwPTS and don’t support secondSlotStartingPosition, it is required to fulfil test 3; and For UE support both endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 4. |  |  |  |  |  |  |  |  |

## 8.5	Demodulation of PHICH

The receiver characteristics of the PHICH are determined by the probability of miss-detecting an ACK for a NACK (Pm-an). It is assumed that there is no bias applied to the detection of ACK and NACK (zero-threshold delection).

### 8.5.1	FDD

The parameters specified in Table 8.5.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.5.1-1: Test Parameters for PHICH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | 0 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | 0 | -3 |
| PHICH duration |  |  | Normal | Normal |
| PHICH Ng (Note 1) |  |  | Ng = 1 | Ng = 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. |  |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: according to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

#### 8.5.1.1	Single-antenna port performance

For the parameters specified in Table 8.5.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.1.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.18 | OP.1 FDD | ETU70 | 1 x 2 Low | 0.1 | 5.5 |
| 2 | 10 MHz | R.24 | OP.1 FDD | ETU70 | 1 x 2 Low | 0.1 | 0.6 |

#### 8.5.1.2	Transmit diversity performance

##### 8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.5.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.1.2.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.19 | OP.1 FDD | EVA70 | 2 x 2 Low | 0.1 | 4.4 |
| 1A | 5MHz (Note 1) | R.19-1 | OP.1 FDD | EVA 70 | 2x2 Low | 0.1 | 4 |
| Note 1: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |

##### 8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.5.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.1.2.2-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 5 MHz | R.20 | OP.1 FDD | EPA5 | 4 x 2 Medium | 0.1 | 6.1 |

##### 8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.3-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.3-2. The downlink physical setup is in accordance with Annex C.3.2 and Annex C.3.3. In Table 8.5.1.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.5.1.2.3-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.5.1.2.3-2 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 00000100 00000100 00000100 01000100 00000100 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 00000100 00000100 00000100 00000100 00000100 | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 00000100 00000100 00000100 01000100 00000100 | N/A |
|  | CCSI,1 |  | 11111011 11111011 11111011 10111011 11111011 | N/A |
| Number of control OFDM symbols |  |  | 3 | 3 |
| PHICH Ng (Note 9) |  |  | 1 | N/A |
| PHICH duration |  |  | extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PHICH is transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell but not in the 26th subframe indicated by the ABS pattern. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in the test. Note 9: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.5.1.2.3-2: Minimum performance PHICH

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Antenna Configuration and Correlation Matrix | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-an (%) | SNR (dB) (Note 2) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | 2x2 Low | 0.1 | 4.6 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |

##### 8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.4-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.4-2. In Table 8.5.1.2.4-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.5.1.2.4-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 (Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.5.1.2.4-2 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. | N/A | N/A |
| ABS pattern (Note 4) |  |  | N/A | 00000100 00000100 00000100 00000100 00000100 | 00000100 00000100 00000100 00000100 00000100 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 00000100 00000100 00000100 00000100 00000100 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 00000100 00000100 00000100 00000100 00000100 | N/A | N/A |
|  | CCSI,1 |  | 11111011 11111011 11111011 11111011 11111011 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| PHICH Ng (Note 10) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PHICH is transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell but not in the 26th subframe indicated by the ABS pattern. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 7: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 8: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell 2 and Cell 3 in the test Note 10: According to Clause 6.9 in TS 36.211 [4]. |  |  |  |  |  |

Table 8.5.1.2.4-2: Minimum performance PHICH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EVA5 | EVA5 | 2x2 Low | 0.1 | 5.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.5-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.5-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells and applying interference model defined in clause B.5.2. In Table 8.5.1.2.5-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is not provided.

Table 8.5.1.2.5-1: Test Parameters for PHICH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA PDSCH_RA OCNG_RA |  | dB | -3 | -3 | -3 |
|  | PCFICH_RB PHICH_RB PDCCH_RB PDSCH_RB OCNG_RB |  | dB | -3 | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  |  | 1 | N/A | N/A |
| PHICH duration |  |  |  | Normal | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.2 | As specified in clause B.5.2 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 80 | 80 |
|  |  | Rank 2 | % | N/A | 20 | 20 |
| Unused RE-s and PRB-s |  |  |  | OCNG | OCNG | OCNG |
| Time offset relative to Cell 1 |  |  | ms | N/A | 0.33 | 0.67 |
| Frequency offset relative to Cell 1 |  |  | Hz | N/A | 0 | 0 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. |  |  |  |  |  |  |

Table 8.5.1.2.5-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EVA70 | EVA70 | EVA70 | 2x2 Low | 0.1 | 17.9 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.6-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.6-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the non-colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.1.2.6-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.1.2.6-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.1.2.6-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 15.8 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.7-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.7-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.1.2.7-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.1.2.7-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 6 | 1 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.1.2.7-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 13.4 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.1-1 and Table 8.5.1.2.8-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.1.2.8-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the non-colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.1.2.8-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.1.2.8-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.1.2.8-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 15.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

### 8.5.2	TDD

The parameters specified in Table 8.5.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.5.2-1: Test Parameters for PHICH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 1 | 1 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | 0 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | 0 | -3 |
| PHICH duration |  |  | Normal | Normal |
| PHICH Ng (Note 3) |  |  | Ng = 1 | Ng = 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. |  |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4] Note 2: as specified in Table 4.2-1 in TS 36.211 [4] Note 3: according to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

#### 8.5.2.1	Single-antenna port performance

For the parameters specified in Table 8.5.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.2.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.18 | OP.1 TDD | ETU70 | 1 x 2 Low | 0.1 | 5.8 |
| 2 | 10 MHz | R.24 | OP.1 TDD | ETU70 | 1 x 2 Low | 0.1 | 1.3 |

#### 8.5.2.2	Transmit diversity performance

##### 8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.5.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.2.2.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.19 | OP.1 TDD | EVA70 | 2 x 2 Low | 0.1 | 4.2 |

##### 8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.5.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.5.2.2.2-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 5 MHz | R.20 | OP.1 TDD | EPA5 | 4 x 2 Medium | 0.1 | 6.2 |

##### 8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.3-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.3-2. The downlink physical setup is in accordance with Annex C.3.2 and Annex C.3.3, In Table 8.5.2.2.3-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 is according to Annex C.3.3, respectively.

Table 8.5.2.2.3-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 |
| at antenna port |  | dBm/15kHz | -100.5 (Note 1) | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -95.3 (Note 3) | N/A |
|  |  | dB | Reference Value in Table 8.5.2.2.3-2 | 1.5 |
| BWChannel |  | MHz | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |
| Cell Id |  |  | 0 | 1 |
| ABS pattern (Note 4) |  |  | N/A | 0000010001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0000000001 0000000001 | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0000010001 0000000001 | N/A |
|  | CCSI,1 |  | 1100101000 1100111000 | N/A |
| Number of control OFDM symbols |  |  | 3 | 3 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| PHICH Ng (Note 9) |  |  | 1 | N/A |
| PHICH duration |  |  | extended | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PHICH is transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell but not in subframe 5 Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 7: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 8: SIB-1 will not be transmitted in Cell2 in the test. Note 9: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |

Table 8.5.2.2.3-2: Minimum performance PHICH

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Antenna Configuration and Correlation Matrix | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Pm-an (%) | SNR (dB) (Note 2) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | 2x2 Low | 0.1 | 4.6 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of cell 1. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |

##### 8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.4-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.4-2. In Table 8.5.2.2.4-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.5.2.2.4-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 (Note 1) | N/A | N/A |
|  |  | dBm/15kHz | -98 (Note 2) | N/A | N/A |
|  |  | dBm/15kHz | -93 (Note 3) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.5.2.2.4-2 | 5 | 3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. | N/A | N/A |
| ABS pattern (Note 4) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 5) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note 6) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| PHICH Ng (Note 10) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS Note 2: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 3: This noise is applied in OFDM symbols of a subframe overlapping with aggressor non-ABS Note 4: ABS pattern as defined in [9]. PHICH is transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell but not in subframe 5 Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 7: The number of control OFDM symbols is not available for ABS and is 2 for the subframe indicated by “0” of ABS pattern. Note 8: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell 2 and Cell 3 in the test. Note 10: According to Clause 6.9 in TS 36.211 [4] |  |  |  |  |  |

Table 8.5.2.2.4-2: Minimum performance PHICH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EVA5 | EVA5 | 2x2 Low | 0.1 | 5.7 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.5-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.5-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.2.2.5-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.2.2.5-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 6 | 1 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.2.2.5-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 16.2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.6-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.6-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the non-colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.2.2.6-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.2.2.6-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.2.2.6-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 16.1 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.7-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.7-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.2.2.7-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.2.2.7-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 6 | 1 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.2.2.7-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 14.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer

For the parameters specified in Table 8.5.2-1 and Table 8.5.2.2.8-1, the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.5.2.2.8-2. The purpose of this test is to verify the PHICH performance with 2 transmit antennas when the serving cell PHICH transmission is interfered by two interfering cells with the dominant interferer having the non-colliding CRS pattern and applying interference model defined in clause B.7.1.  In Table 8.5.2.2.8-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.5.2.2.8-1: Test Parameters for PHICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of control OFDM symbols |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time offset to cell 1 |  | us | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.5.2.2.8-2: Minimum performance PHICH for Enhanced Downlink Control Channel Performance Requirement Type B

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 TDD | OP.1 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x2 Low | 0.1 | 15.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

## 8.6	Demodulation of PBCH

The receiver characteristics of the PBCH are determined by the probability of miss-detection of the PBCH (Pm-bch), which is defined as

Where A is the number of correctly decoded MIB PDUs and B is the Number of transmitted MIB PDUs (Redundancy versions for the same MIB are not counted separately).

### 8.6.1	FDD

Table 8.6.1-1: Test Parameters for PBCH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PBCH_RA | dB | 0 | -3 |
|  | PBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4] Note 2: as specified in Table 4.2-1 in TS 36.211 [4] |  |  |  |  |

#### 8.6.1.1	Single-antenna port performance

For the parameters specified in Table 8.6.1-1 the average probability of a miss-detecting PBCH (Pm-bch) shall be below the specified value in Table 8.6.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.1.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.21 | ETU70 | 1 x 2 Low | 1 | -6.1 |

#### 8.6.1.2	Transmit diversity performance

##### 8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.6.1-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.1.2.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.22 | EPA5 | 2 x 2 Low | 1 | -4.8 |

##### 8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.6.1-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.1.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.1.2.2-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.23 | EVA5 | 4 x 2 Medium | 1 | -3.5 |

##### 8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information

For the parameters specified in Table 8.6.1.2.3-1 and Table 8.6.1.2.3-2, the averaged probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.1.2.3-2. Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, repectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.6.1.2.3-1: Test Parameters for PBCH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PBCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PBCH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  | dB | Reference Value in Table 8.6.1.2.3-2 | 4 | 2 |
| BWChannel |  | MHz | 1.4 | 1.4 | 1.4 |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS Pattern (Note 4) |  |  | N/A | 01000000 01000000 01000000 01000000 01000000 | 01000000 01000000 01000000 01000000 01000000 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 2: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. Note 3: The PBCH transmission from Cell 1, Cell 2 and Cell 3 overlap. The same PBCH transmission redundancy version is used for Cell 1, Cell 2 and Cell 3. Note 4: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. |  |  |  |  |  |

Table 8.6.1.2.3-2: Minimum performance PBCH

| Test Number | Reference Channel | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-bch (%) | SNR (dB) (Note 3) |
| 1 | R.22 | ETU30 | ETU30 | ETU30 | 2x2 Low | 1 | -3.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |

### 8.6.2	TDD

Table 8.6.2-1: Test Parameters for PBCH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 1 | 1 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Downlink power allocation | PBCH_RA | dB | 0 | -3 |
|  | PBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |

#### 8.6.2.1	Single-antenna port performance

For the parameters specified in Table 8.6.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.2.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.21 | ETU70 | 1 x 2 Low | 1 | -6.4 |

#### 8.6.2.2	Transmit diversity performance

##### 8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port

For the parameters specified in Table 8.6.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.2.2.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.22 | EPA5 | 2 x 2 Low | 1 | -4.8 |

##### 8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port

For the parameters specified in Table 8.6.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.6.2.2.2-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 1.4 MHz | R.23 | EVA5 | 4 x 2 Medium | 1 | -4.1 |

##### 8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information

For the parameters specified in Table 8.6.2.2.3-1 and Table 8.6.2.2.3-2, the averaged probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.6.2.2.3-2. Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 8.6.2.2.3-1: Test Parameters for PBCH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PBCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PBCH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  | dB | Reference Value in Table 8.6.2.2.3-2 | 4 | 2 |
| BWChannel |  | MHz | 1.4 | 1.4 | 1.4 |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS Pattern (Note 4) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: The number of the CRS ports in Cell1, Cell2 and Cell 3is the same. Note 2: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. Note 3: The PBCH transmission from Cell 1, Cell 2 and Cell 3 overlap. The same PBCH transmission redundancy version is used for Cell 1, Cell 2 and Cell 3. Note 4: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. |  |  |  |  |  |

Table 8.6.2.2.3-2: Minimum performance PBCH

| Test Number | Reference Channel | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-bch (%) | SNR (dB) (Note 3) |
| 1 | R.22 | ETU30 | ETU30 | ETU30 | 2x2 Low | 1 | -3.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |

## 8.7	Sustained downlink data rate provided by lower layers

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum number of DL-SCH transport block bits received within a TTI for the UE category indicated. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement. The size of the TB per TTI corresponds to the largest possible DL-SCH transport block for each UE category using the maximum number of layers for spatial multiplexing. Transmission modes 1 and 3 are used with radio conditions resembling a scenario where sustained maximum data rates are available.

Test case is selected according to table 8.7-1 depending on UE capability for CA and EPDCCH.

Table 8.7-1: SDR test applicability

|  | # Single carrier UE not supporting EPDCCH | # CA UE not supporting EPDCCH | # Single carrier UE supporting EPDCCH | # CA UE supporting EPDCCH |
| --- | --- | --- | --- | --- |
| # FDD | # 8.7.1 | # 8.7.1 | # 8.7.3 | # 8.7.1, 8.7.3 |
| # TDD | # 8.7.2 | # 8.7.2 | # 8.7.4 | # 8.7.2, 8.7.4 |

### 8.7.1	FDD (single carrier and CA)

The parameters specified in Table 8.7.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.7.1-1: Common Test Parameters (FDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |  |

For UE not supporting 256QAM, the requirements are specified in Table 8.7.1-3, with the addition of the parameters in Table 8.7.1-2 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.1-4. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements are specified in Table 8.7.1-6, with the addition of the parameters in Table 8.7.1-5 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified in Table 8.7.1-7, the TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirement in Table 8.7.1-3 is not applicable.

For UE supporting 256QAM and category 9/10 and category 13, the requirements are specified in both Table 8.7.1-3 and Table 8.7.1-6, with the addition of the parameters in Table 8.7.1-2 and in Table 8.7.1-5 respectivly. The downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.1-4 and in Table 8.7.1-7 for the category 9/10 and category 13, the TB success rate shall be sustained during at least 300 frames.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.1-2: test parameters for sustained downlink data rate (FDD 64QAM)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 1 x 2 | N/A | 0 | 0 | 0 | -85 | OP.6 FDD |
| 1A | 10 | 1 | 1x1 | N/A | 0 | 0 | 0 | -85 | OP.6 FDD |
| 1B | 20 | 1 | 1x1 | N/A | 0 | 0 | 0 | -85 | OP.6 FDD |
| 2 | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3,4,6 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3A | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3B, 4A | 2x10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3C, 4B | 15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6A | 2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6B | 10+15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6C | 10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6D | 15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6E | 2x15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6F | 15+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6G | 20+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7 | 3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7A | 15+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7B | 10+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7C | 15+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7D | 10+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7E | 10+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7F | 10+15+15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7G | 5+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7H | 5+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7I | 5+10+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7J | 5+5+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7K | 3x10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7L | 5+5+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8 | 4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8A | 20+20+20+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8B | 20+20+10+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8C | 20+20+10+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8D | 20+10+10+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8E | 15+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8F | 2x15+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8G | 10+15+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8H | 3x10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8I | 2x5+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8J | 2x5+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8K | 4x10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9A | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9B | 10+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9C | 2x10+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9D | 5+10+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9E | 3x10+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9F | 4x10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 10 | 6x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 11 | 7x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| NOTE 1: For CA test cases, PUCCH format 1b with channel selection is used to feedback ACK/NACK for Test 1-6E, and PUCCH format 3 is used to feedback ACK/NACK for Test 7-7G. |  |  |  |  |  |  |  |  |  |

Table 8.7.1-3: Minimum requirement (FDD 64QAM)

| Test | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 1 | 10296 | R.31-1 FDD | 95 |
| 1A | 10296 | R.31-1 FDD | 95 |
| 1B | 10296 | R.31-1A FDD | 95 |
| 2 | 25456 | R.31-2 FDD | 95 |
| 3 | 51024 | R.31-3 FDD | 95 |
| 3A | 36696 (Note 2) | R.31-3A FDD | 85 |
| 3B | 25456 | R.31-2 FDD | 95 |
| 3C | 51024 | R.31-3C FDD | 85 |
| 4 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 4A | 36696 (Note 2) | R.31-3A FDD | 85 |
| 4B | 55056 (Note 5) | R.31-4B FDD | 85 |
| 6 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 6A | 75376 (Note 3) | R.31-4 FDD | 85 |
| 6B | 36696 (Note 2) for 10MHz CC 55056 for 15MHz CC | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | 85 |
| 6C | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 6D | 55056 for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 6E | 55056 (Note 5) for two 15MHz CCs | R.31-4B FDD for two 15MHz CCs | 85 |
| 6F | 55056 (Note 5) for 15MHz CC 18336 (Note 6) for 5MHz CC | R.31-5 FDD for 15MHz CC R.31-6 FDD for 5MHz CC | 85 |
| 6G | 75376 (Note 3) for 20MHz CC 18336 (Note 6) for 5MHz CC | R.31-4 FDD for 20MHz CC R.31-6 FDD for 5MHz CC | 85 |
| 7 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 7A | 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7B | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7C | 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7D | 36696 (Note 2) for 10MHz CC 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7E | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7F | 36696 (Note 2) for 10MHz CC 55056 (Note 5) for 15MHz CC | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | 85 |
| 7G | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7H | 18336 (Note 6) for 5MHz CC 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7I | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC | 85 |
| 7J | 18336 (Note 6) for 5MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 7K | 36696 (Note 2) for 10MHz CC | R.31-3A FDD for 10MHz CC | 85 |
| 7L | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC | 85 |
| 8E | 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8F | 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8G | 36696 (Note 2) for 10MHz CC 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8H | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8I | 18336 (Note 6) for 5MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8J | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8K | 36696 (Note 2) for 10MHz CC | R.31-3A FDD for 10MHz CC | 85 |
| 8 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 8A | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8B | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8C | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 8D | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 9A | 55056 (Note 5) for 15MHz CC 75376 (Note 3) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9B | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9C | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9D | 18336 (Note 6) for 5MHz CC 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-6 FDD for 5MHz CC R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9E | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 9F | 36696 (Note 2) for 10MHz CC 75376 (Note 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| 10 | 75376 (Note 3) | R.31-4 FDD | 85 |
| 11 | 75376 (Note 3) | R.31-4 FDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 35160 bits for sub-frame 5. Note 3: 71112 bits for sub-frame 5. Note 4: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. Note 5: 52752bits for sub-frame 5. Note 6: 15840bits for sub-frame 0. |  |  |  |

Table 8.7.1-4: Test points for sustained data rate (FRC 64QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 1bis | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | DL Cat. 11,12 |  |
| Single carrier | 10 | 1 | 1A | 2 | 3A | 3A | - | - | - | - |
|  | 15 | - | - | - | 3C | 4B | - | - | - | - |
|  | 20 | - | 1B | - | 3 | 4 | 6 | - | - | - |
| CA with 2CCs | 10+10 | - | - | - | 3B | 4A | 4A | 4A | - | - |
|  | 10+15 | - | - | - | 3B | 4A | 6B | 6B | - | - |
|  | 10+20 | - | - | - | 3B | 4A | 6C | 6C | - | - |
|  | 15+15 | - | - | - | 3B | 4A | 6E | 6E | - | - |
|  | 15+5 |  | - |  | 3B | 4A | 6F | 6F | - | - |
|  | 20+5 | - | - | - | 3 | 4 | 6G | 6G | - | - |
|  | 15+20 | - | - | - | 3B | 4A | 6D | 6D | - | - |
|  | 20+20 | - | - | - | 3B or 3 (Note 4) | 4A or 4 (Note 4) | 6A | 6A | - | - |
| CA with 3CCs | 3x20 | - | - | - | - | - | 6A | 7 | 7 | - |
|  | 15+20+20 | - | - | - | - | - | 6A | 7A | 7A | - |
|  | 10+20+20 | - | - | - | - | - | 6A | 7B | 7B | - |
|  | 15+15+20 |  | - |  |  |  | 6D | 7C | 7C | - |
|  | 10+15+20 | - | - | - | - | - | 6D | 7D | 7D | - |
|  | 10+10+20 | - | - | - | - | - | 7E | 7E | 7E | - |
|  | 10+15+15 | - | - | - | - | - | 7F | 7F | 7F | - |
|  | 5+10+20 | - | - | - | - | - | 7G | 7G | 7G | - |
|  | 5+15+20 | - | - | - | - | - | 7H | 7H | 7H | - |
|  | 5+10+10 | - | - | - | - | - | 7I | 7I | 7I | - |
|  | 5+5+20 |  |  |  |  |  | 7J | 7J | 7J |  |
|  | 3x10 |  |  |  |  |  | 7K | 7K | 7K |  |
|  | 5+5+10 |  |  |  |  |  | 7L | 7L | 7L |  |
| CA with 4CCs | 4x20 | - | - | - | - | - | - | 7 | 8 | 8 |
|  | 20+20+20+10 | - | - | - | - | - | - | 7 | 8A | 8A |
|  | 20+20+10+10 | - | - | - | - | - | - | 8B | 8B | 8B |
|  | 20+20+10+5 | - | - | - | - | - | - | 8C | 8C | 8C |
|  | 20+10+10+5 | - | - | - | - | - | - | 8D | 8D | 8D |
|  | 15+3x20 |  |  |  |  |  |  | 8E | 8E | 8E |
|  | 2x15+2x20 |  |  |  |  |  |  | 8F | 8F | 8F |
|  | 10+15+2x20 |  |  |  |  |  |  | 8G | 8G | 8G |
|  | 3x10+20 |  |  |  |  |  |  | 8H | 8H | 8H |
|  | 2x5+2x20 |  |  |  |  |  |  | 8I | 8I | 8I |
|  | 2x5+10+20 |  |  |  |  |  |  | 8J | 8J | 8J |
|  | 4x10 |  |  |  |  |  |  | 8K | 8K | 8K |
| CA with 5CCs | 5x20 | - | - | - | - | - | - | - | 8 | 9 |
|  | 15+4x20 |  |  |  |  |  |  |  |  | 10 |
|  | 10+4x20 |  |  |  |  |  |  |  |  | 11 |
|  | 2x10+3x20 |  |  |  |  |  |  |  |  | 12 |
|  | 5+10+3x20 |  |  |  |  |  |  |  |  | 13 |
|  | 3x10+2x20 |  |  |  |  |  |  |  |  | 14 |
| CA with 6CCs | 6x20 | - | - | - | - | - | - | - | 10 | 10 |
| CA with 7CCs | 7x20 | - | - | - | - | - | - | - | 11 | 11 |
| Note 1: Void. Note 2: For non-CA UE, test is selected for maximum supported bandwidth. Note 3: Void. Note 4: If the intra-band contiguous CA is the only CA configuration supported by category 3 or 4 UE, the single carrier test is selecte, i.e., Test 3 for UE category 3 and Test 4 for UE category 4. Otherwise, Test 3B applies for category 3 UE and Test 4A applies for category 4 UE. Note 5: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. Note 6: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.1-5: test parameters for sustained downlink data rate (FDD 256QAM)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 2 | 2x15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 2A | 15+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3 | 10+15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 3A | 20+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 4 | 10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 6 | 15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 7 | 2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 8 | 3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 9 | 15+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 10 | 10+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 11 | 15+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 12 | 10+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 13 | 10+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 14 | 10+15+15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15 | 5+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15A | 5+15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15B | 5+10+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15C | 5+5+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15D | 3x10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 15E | 5+5+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 16 | 4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 17 | 20+20+20+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18 | 20+20+10+10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18A | 20+20+10+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18B | 20+10+10+5 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18C | 15+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18D | 2x15+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18E | 10+15+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18F | 3x10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18G | 2x5+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18H | 2x5+10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 18I | 4x10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19A | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19B | 10+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19C | 2x10+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19D | 5+10+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19E | 3x10+2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 19F | 4x10+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 20 | 6x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| 21 | 7x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | OP.1 FDD |
| Note 1: For CA test cases, PUCCH format 3 is used to feedback ACK/NACK. |  |  |  |  |  |  |  |  |  |

Table 8.7.1-6: Minimum requirement (FDD 256QAM)

| Test | Measurement channel | Reference value |
| --- | --- | --- |
|  |  | TB success rate [%] |
| 1 | R.68 FDD | 85 |
| 2 | R.68-1 FDD | 85 |
| 2A | R.68-1 FDD for 15MHz CC R.68-3 FDD for 5MHz CC | 85 |
| 3 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC | 85 |
| 3A | R.68 FDD for 20MHz CC R.68-3 FDD for 5MHz CC | 85 |
| 4 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 6 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 7 | R.68 FDD | 85 |
| 8 | R.68 FDD | 85 |
| 9 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 10 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 11 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 12 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 13 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 14 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC | 85 |
| 15 | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 15A | R.68-3 FDD for 5MHz CC R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 15B | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC | 85 |
| 15C | R.68-3 FDD for 5MHz CC R.68 FDD for 20MHz CC | 85 |
| 15D | R.68-2 FDD for 10MHz CC | 85 |
| 15E | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC | 85 |
| 16 | R.68 FDD | 85 |
| 17 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18A | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18B | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18C | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 18D | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 18E | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 18F | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18G | R.68-3 FDD for 5MHz CC R.68 FDD for 20MHz CC | 85 |
| 18H | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 18I | R.68-2 FDD for 10MHz CC | 85 |
| 19 | R.68 FDD | 85 |
| 19A | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 |
| 19B | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 19C | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 19D | R.68-3 FDD for 5MHz CC R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 19E | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 19F | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 |
| 20 | R.68 FDD | 85 |
| 21 | R.68 FDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |

Table 8.7.1-7: Test points for sustained data rate (FRC 256QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL Cat. 11, 12 |  |  |  |  |  |
| Single carrier | 20 | - | 1 | - | - |  |  |
| CA with 2CCs | 2x15 | 2 | 2 | - | - |  |  |
|  | 15+5 | 2A | 2A | - | - |  |  |
|  | 10+15 | 3 | 3 | - | - |  |  |
|  | 20+5 | 3A | 3A | - | - |  |  |
|  | 10+20 | 4 | 4 | - | - |  |  |
|  | 15+20 | 6 | 6 | - | - |  |  |
|  | 20+20 | 7 | 7 | - | - |  |  |
| CA with 3CCs | 3x20 | 8 | 7 | 8 | - |  |  |
|  | 15+20+20 | 9 | 7 | 9 | - |  |  |
|  | 10+20+20 | 10 | 7 | 10 | - |  |  |
|  | 15+15+20 | 11 | 6 | 11 | - |  |  |
|  | 10+15+20 | 12 | 6 | 12 | - |  |  |
|  | 10+10+20 | 13 | 13 | 13 | - |  |  |
|  | 10+15+15 | 14 | 14 | 14 | - |  |  |
|  | 5+10+20 | 15 | 15 | 15 | - |  |  |
|  | 5+15+20 | 15A | 15A | 15A | - |  |  |
|  | 5+10+10 | 15B | 15B | 15B | - |  |  |
|  | 5+5+20 | 15C | 15C | 15C |  |  |  |
|  | 3x10 | 15D | 15D | 15D |  |  |  |
|  | 5+5+10 | 15E | 15E | 15E |  |  |  |
| CA with 4CCs | 4x20 | 8 | - | 16 | 16 |  |  |
|  | 20+20+20+10 | 8 | - | 17 | 17 |  |  |
|  | 20+20+10+10 | 18 | - | 18 | 18 |  |  |
|  | 20+20+10+5 | 18A | - | 18A | 18A |  |  |
|  | 20+10+10+5 | 18B | - | 18B | 18B |  |  |
|  | 15+3x20 | 18C |  | 18C | 18C |  |  |
|  | 2x15+2x20 | 18D |  | 18D | 18D |  |  |
|  | 10+15+2x20 | 18E |  | 18E | 18E |  |  |
|  | 3x10+20 | 18F |  | 18F | 18F |  |  |
|  | 2x5+2x20 | 18G |  | 18G | 18G |  |  |
|  | 2x5+10+20 | 18H |  | 18H | 18H |  |  |
|  | 4x10 | 18I |  | 18I | 18I |  |  |
| CA with 5CCs | 5x20 | - | - | 16 | 19 |  |  |
|  | 15+4x20 |  |  |  | 20 |  |  |
|  | 10+4x20 |  |  |  | 21 |  |  |
|  | 2x10+3x20 |  |  |  | 22 |  |  |
|  | 5+10+3x20 |  |  |  | 23 |  |  |
|  | 3x10+2x20 |  |  |  | 24 |  |  |
|  | 4x10+20 |  |  |  | 25 |  |  |
| CA with 6CCs | 6x20 | - | - | - | 20 |  |  |
| CA with 7CCs | 7x20 | - | - | - | 21 |  |  |
| NOTE 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |

### 8.7.2	TDD (single carrier and CA)

The parameters specified in Table 8.7.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.7.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Special subframe configuration (Note 1) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| Note 1: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |

For UE not supporting 256QAM, the requirements are specified in Table 8.7.2-3, with the addition of the parameters in Table 8.7.2-2 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.2-4. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements are specified in Table 8.7.2-6, with the addition of the parameters in Table 8.7.2-5 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.2-7. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirement in Table 8.7.2-3 is not applicable.

For UE supporting 256QAM and category 9/10 and category 13, the requirements are specified in both Table 8.7.2-3 and Table 8.7.2-6, with the addition of the parameters in Table 8.7.2-2 and in Table 8.7.2-5 respectivly. The downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.2-4 and in Table 8.7.2-7 for the category 9/10 and category 13, the TB success rate shall be sustained during at least 300 frames.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.2-2: test parameters for sustained downlink data rate (TDD 64QAM)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | ACK/NACK feedback mode | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 1 x 2 | N/A | 0 | 0 | 0 | -85 | Bundling | OP.6 TDD |
| 1A | 10 | 1 | 1x1 | N/A | 0 | 0 | 0 | -85 | Bundling | OP.6 TDD |
| 1B | 20 | 1 | 1x1 | N/A | 0 | 0 | 0 | -85 | Bundling | OP.6 TDD |
| 2 | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | Bundling | OP.1 TDD |
| 3 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | Bundling | OP.1 TDD |
| 3A | 15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | Muliplexing | OP.2 TDD |
| 4,6 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | Multiplexing | OP.1 TDD |
| 6A | 2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | - (Note 1) | OP.1 TDD |
| 6B | 20+15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 7 | 3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 7A | 15+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 8 | 4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 9 | 15+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 10 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 11 | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 12 | 6x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 13 | 7x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| Note 1: PUCCH format 1b with channel selection is used to feedback ACK/NACK. Note 2: PUCCH format 3 is used to feedback ACK/NACK. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.2-3: Minimum requirement (TDD 64QAM)

| Test | Number of bits of a DL-SCH transport block received within a TTI for normal/special sub-frame | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 1 | 10296/0 | R.31-1 TDD | 95 |
| 1A | 10296/0 | R.31-1 TDD | 95 |
| 1B | 10296/0 | R.31-1A TDD | 95 |
| 2 | 25456/0 | R.31-2 TDD | 95 |
| 3 | 51024/0 | R.31-3 TDD | 95 |
| 3A | 51024/0 | R.31-3A TDD | 85 |
| 4 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 6 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 6A | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 6B | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 7 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 7A | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 8 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 9 | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 10 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 11 | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 12 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 13 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 71112 bits for sub-frame 5. Note 3: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |  |

Table 8.7.2-4: Test points for sustained data rate (FRC 64QAM)

| CA config | Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 1bis | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9, 10 | Cat. 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | DL Cat. 11, 12 |  |
| Single carrier | 10 | 1 | 1A | 2 | - | - | - | - | - | - |
|  | 15 | - | - | - | 3A | 3A | - | - | - | - |
|  | 20 | - | 1B | - | 3 | 4 | 6 | - | - | - |
| CA with 2CCs | 20+20 | - | - | -- | 3(Note 4) | 4 (Note 4) | 6A | 6A | - | - |
|  | 15+20 | - | - | - | 3(Note 4) | 4 (Note 4) | 6B | 6B | - | - |
| CA with 3 CCs | 3x20 | - | - | - | - | - | 6A | 7 | 7 | - |
|  | 15+20+20 | - | - | - | - | - | 6A | 7A | 7A | - |
| CA with 4 CCs | 4x20 | - | - | - | - | - | - | 7 | 8 | 8 |
|  | 15+3x20 | - | - | - | - | - | - | 7 | 9 | 9 |
| CA with 5 CCs | 5x20 |  |  |  |  |  |  |  | 10 | 10 |
|  | 15+4x20 |  |  |  |  |  |  |  | 11 | 11 |
| CA with 6 CCs | 6x20 |  |  |  |  |  |  |  | 12 | 12 |
| CA with 7 CCs | 7x20 |  |  |  |  |  |  |  | 13 | 13 |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. Note 2: For non-CA UE, test is selected for maximum supported bandwidth. Note 3: Void. Note 4: If the intra-band contiguous CA is the only CA configuration supported by category 3 or 4 UE, single carrier test is selected. Note 5: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.2-5: test parameters for sustained downlink data rate (TDD 256QAM)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | ACK/NACK feedback mode | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | Bundling | OP.1 TDD |
| 2 | 15+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 3 | 2x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 4 | 3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 5 | 15+20+20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 6 | 4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 7 | 15+3x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 8 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 9 | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 10 | 6x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| 11 | 7x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 1) | OP.1 TDD |
| Note 1: For CA test cases, PUCCH format 3 is used to feedback ACK/NACK. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.2-6: Minimum requirement (TDD 256QAM)

| Test | Measurement channel | Reference value |
| --- | --- | --- |
|  |  | TB success rate [%] |
| 1 | R.68 TDD | 85 |
| 2 | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 3 | R.68 TDD | 85 |
| 4 | R.68 TDD | 85 |
| 5 | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 6 | R.68 TDD | 85 |
| 7 | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 8 | R.68 TDD | 85 |
| 9 | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 10 | R.68 TDD | 85 |
| 11 | R.68 TDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |

Table 8.7.2-7: Test points for sustained data rate (FRC 256QAM)

| CA config | Bandwidth/ Bandwidth combination (MHz) | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL Cat. 11, 12 |  |  |  |  |  |
| Single carrier | 20 | - | 1 | - | - |  |  |
| CA with 2CCs | 15+20 | 2 | 2 | - | - |  |  |
|  | 2x20 | 3 | 3 | - | - |  |  |
| CA with 3 CCs | 3x20 | 4 | 3 | 4 | - |  |  |
|  | 15+20+20 | 5 | 3 | 5 | - |  |  |
| CA with 4 CCs | 4x20 | 4 | - | 6 | 6 |  |  |
|  | 15+3x20 | 5 | - | 7 | 7 |  |  |
| CA with 5 CCs | 5x20 |  |  |  | 8 |  |  |
|  | 15+4x20 |  |  |  | 9 |  |  |
| CA with 6 CCs | 6x20 |  |  |  | 10 |  |  |
| CA with 7 CCs | 7x20 |  |  |  | 11 |  |  |

### 8.7.3	FDD (EPDCCH scheduling)

The parameters specified in Table 8.7.3-1 are valid for all FDD tests unless otherwise stated.

Table 8.7.3-1: Common test parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Number of EPDCCH sets |  | 1 |
| EPDCCH transmission type |  | Localized |
| Number of PRB per EPDCCH set and EPDCCH PRB pair allocation |  | 2 PRB pairs 10MHz BW: Resource blocks nPRB = 48, 49 15MHz BW: Resource blocks nPRB = 70, 71 20MHz BW: Resource blocks nPRB = 98, 99 |
| EPDCCH Starting Symbol |  | Derived from CFI (i.e. default behaviour) |
| ECCE Aggregation Level |  | 2 ECCEs |
| Number of EREGs per ECCE |  | 4 |
| EPDCCH scheduling |  | EPDCCH candidate is randomly assigned in each subframe |
| EPDCCH precoder (Note 1) |  | Fixed PMI 0 |
| EPDCCH monitoring SF pattern |  | 1111111111 0000000000 1111111111 0000000000 |
| Timing advance | μs | 100 |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| Note 1: EPDCCH precoder parameters are defined for tests with 2 x 2 antenna configuration |  |  |

The requirements are specified in Table 8.7.3-3, with the addition of the parameters in Table 8.7.3-2 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category, CA capability and bandwidth combination with maximum aggregated bandwidth as specified in Table 8.7.3-4. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.3-2: Test parameters for SDR test for PDSCH scheduled by EPDCCH (FDD)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  |  | at antenna port (dBm/15kHz) | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | δ |  |  |
| 1 | 10 | 1 | 1 x 2 | N/A | 0 | 0 | 0 | 0 | -85 | OP.6 FDD |
| 2 | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 FDD |
| 3,4,6 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 FDD |
| 3A | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 FDD |
| 3C, 4B | 15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 FDD |

Table 8.7.3-3: Minimum requirement (FDD)

| Test | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 1 | 10296 | R.31E-1 FDD | 95 |
| 2 | 25456 | R.31E-2 FDD | 95 |
| 3 | 51024 | R.31E-3 FDD | 95 |
| 3A | 36696 (Note 2) | R.31E-3A FDD | 85 |
| 3C | 51024 | R.31E-3C FDD | 85 |
| 4 | 75376 (Note 3) | R.31E-4 FDD | 85 |
| 4B | 55056 (Note 5) | R.31E-4B FDD | 85 |
| 6 | 75376 (Note 3) | R.31E-4 FDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 35160 bits for sub-frame 5. Note 3: 71112 bits for sub-frame 5. Note 4: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. Note 5: 52752 bits for sub-frame 5. |  |  |  |

Table 8.7.3-4: Test points for sustained data rate (FRC)

| CA config | Bandwidth (MHz) | Category 1 | Category 2 | Category 3 | Category 4 | Category 6 | Category 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Single carrier | 10 | 1 | 2 | 3A | 3A | - | - |
|  | 15 | - | - | 3C | 4B | - | - |
|  | 20 | - | - | 3 | 4 | 6 | 6 |
| Note 1: The test is selected for maximum supported bandwidth. |  |  |  |  |  |  |  |

### 8.7.4	TDD (EPDCCH scheduling)

The parameters specified in Table 8.7.4-1 are valid for all TDD tests unless otherwise stated.

Table 8.7.4-1: Common test parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Special subframe configuration (Note 1) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Number of EPDCCH sets |  | 1 |
| EPDCCH transmission type |  | Localized |
| Number of PRB per EPDCCH set and EPDCCH PRB pair allocation |  | 2 PRB pairs 10MHz BW: Resource blocks nPRB = 48, 49 15MHz BW: Resource blocks nPRB = 70, 71 20MHz BW: Resource blocks nPRB = 98, 99 |
| EPDCCH Starting Symbol |  | Derived from CFI (i.e. default behaviour) |
| ECCE Aggregation Level |  | 2 ECCEs |
| Number of EREGs per ECCE |  | 4 for normal subframe and for special subframe |
| EPDCCH scheduling |  | EPDCCH candidate is randomly assigned in each subframe |
| EPDCCH precoder (Note 2) |  | Fixed PMI 0 |
| EPDCCH monitoring SF pattern |  | UL-DL configuration 1: 1101111111 0000000000 UL-DL configuration 5: 1100111001 0000000000 |
| Timing advance | μs | 100 |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| Note 1: As specified in Table 4.2-1 in TS 36.211 [4]. Note 2: EPDCCH precoder parameters are defined for tests with 2 x 2 antenna configuration |  |  |

The requirements are specified in Table 8.7.4-3, with the addition of the parameters in Table 8.7.4-2 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category, CA capability and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.4-4. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.4-2: Test parameters for SDR test for PDSCH scheduled by EPDCCH (TDD)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  |  | at antenna port (dBm/15kHz) | Symbols for unused PRBs | ACK/NACK feedback mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | δ |  |  |  |
| 1 | 10 | 1 | 1 x 2 | N/A | 0 | 0 | 0 | 0 | -85 | OP.6 TDD | Bundling |
| 2 | 10 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 TDD | Bundling |
| 3 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 TDD | Bundling |
| 3A | 15 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.2 TDD | Multiplexing |
| 4,6 | 20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | 3 | -85 | OP.1 TDD | Multiplexing |

Table 8.7.4-3: Minimum requirement (TDD)

| Test | Number of bits of a DL-SCH transport block received within a TTI for normal/special sub-frame | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 1 | 10296/0 | R.31E-1 TDD | 95 |
| 2 | 25456/0 | R.31E-2 TDD | 95 |
| 3 | 51024/0 | R.31E-3 TDD | 95 |
| 3A | 51024/0 | R.31E-3A TDD | 85 |
| 4 | 75376/0 (Note 2) | R.31E-4 TDD | 85 |
| 6 | 75376/0 (Note 2) | R.31E-4 TDD | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 71112 bits for sub-frame 5. Note 3: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |  |

Table 8.7.4-4: Test points for sustained data rate (FRC)

| CA config | Bandwidth/ Bandwidth combination (MHz) | Category 1 | Category 2 | Category 3 | Category 4 | Category 6 | Category 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Single carrier | 10 | 1 | 2 | - | - | - | - |
|  | 15 | - | - | 3A | 3A | - | - |
|  | 20 | - | - | 3 | 4 | 6 | 6 |
| Note 1: The test is selected for maximum supported bandwidth. |  |  |  |  |  |  |  |

### 8.7.5	TDD FDD CA

The parameters specified in Table 8.7.5-1 are valid for all TDD FDD CA tests unless otherwise stated.

Table 8.7.5-1: Common Test Parameters (TDD FDD CA)

| Parameter |  | Unit |  | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) for TDD CC |  |  |  |  | 1 |  |
| Special subframe configuration (Note 2) for TDD CC |  |  |  |  | 4 |  |
| Downlink power allocation |  |  |  | dB | -3 |  |
|  |  |  |  | dB | -3 |  |
|  |  |  |  | dB | 0 |  |
| Cyclic prefix |  |  |  |  | Normal |  |
| Cell ID |  |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  |  | 1 |  |
| Maximum number of HARQ processes per component carrier |  | FDD PCell |  | Processes | 8 for FDD and TDD CCs |  |
|  |  | TDD PCell |  | Processes | 11 for FDD CC; 7 for TDD CC |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  |  | {0,0,1,2} for 64QAM, 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  |  | Not configured |  |
| Propagation condition |  |  |  |  | Static propagation condition No external noise sources are applied |  |
| Transmission mode |  |  |  |  | TM3 |  |
| Codebook subset restriction |  |  |  |  | 10 |  |
| Antenna configuration |  |  |  |  | 2 x 2 |  |
| at antenna port (dBm/15kHz) |  |  |  |  | -85 |  |
| Symbols for unused PRBs |  |  |  |  | OP.1 FDD for FDD CC, OP.1 TDD for TDD CC |  |
| ACK/NACK feedback mode |  |  |  |  | PUCCH format 3 |  |
| Downlink HARQ-ACK timing |  | FDD PCell |  |  | As specified in Clause 7.3.3 in TS36.213 [6] |  |
|  |  | TDD PCell |  |  | As specified in Clause 7.3.4 in TS36.213 [6] |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |  |

#### 8.7.5.1	Minimum Requirement FDD PCell

For UE not supporting 256QAM, the requirements for TDD FDD CA with FDD PCell are specified in Table 8.7.5.1-1 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.5.1-2. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements for TDD FDD CA with FDD PCell are specified in Table 8.7.5.1-3 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category or UE DL category, and bandwidth combination with the maximum aggregated bandwidth as specified in Table 8.7.5.1-4. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirement in Table 8.7.5.1-1 is not applicable.

The applicability of the requirements are specified in Clause 8.1.2.3B. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.5.1-1: test parameters for sustained downlink data rate (TDD FDD CA 64QAM)

| Test number | Bandwidth (MHz) |  |  | Number of bits of a DL-SCH transport block received within a TTI (for normal/special subframe for TDD, except for subframe #5) |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 1 | 2x20 | 20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 2 | 10+20 | 10 | 20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 2A | 15+20 | 15 | 20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 3 | 10+10 | 10 | 10 | 36696 | 36696/0 | R.31-3A FDD | R.31-6 TDD | 85 |
| 4 | 3x20 | 20 | 2x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 5 | 15+20+20 | 15 | 2x20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 6 | 10+20+20 | 10 | 2x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 7 | 3x20 | 2x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 8 | 20+20+15 | 20+15 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 9 | 20+20+10 | 20+10 | 20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 9A | 20+10+10 | 2x10 | 20 | 36696 | 75376/0 | R.31-3A | R.31-4 TDD | 85 |
| 9B | 10+15+20 | 10 | 15+20 | 36696 | 75376/0 for 20MHz CC 55056/0 for 15MHz CC | R.31-3A FDD | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 9C | 10+15+20 | 10+15 | 20 | 36696 for 10MHz CC 55056 for 15MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 10 | 4x20 | 20 | 3x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 11 | 4x20 | 2×20 | 2×20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 12 | 3x20+15 | 20+15 | 2×20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 13 | 2×15+2x20 | 2×15 | 2x20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 14 | 3x20+15 | 2×20+15 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15 | 2×15+2x20 | 2x15+20 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15A | 3x20+10 | 2x20+10 | 20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 15B | 4x20 | 3x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 15C | 10+3x20 | 10 | 3x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 15D | 2x10+2x20 | 2x10 | 2x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 15E | 2x10+20+15 | 2x10 | 20+15 | 36696 | 75376/0 for 20MHz CC 55056/0 for 15MHz CC | R.31-3A FDD | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 15F | 10+3x20 | 10+20 | 2x20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 15G | 10+15+2x20 | 10+15 | 2x20 | 36696 for 10MHz CC 55056 for 15MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15H | 10+15+2x20 | 10+15+20 | 20 | 36696 for 10MHz CC 55056 for 15MHz CC 75376 for 20MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | R.31-4 TDD | 85 |
| 16 | 4x20+15 | 2x20+15 | 2x20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 17 | 2x15+3x20 | 2x15+20 | 2x20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 18 | 4x20+20 | 4x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 19 | 3x20+2x20 | 3x20 | 2x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 20 | 2x20+3x20 | 2x20 | 3x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 21 | 20+4x20 | 20 | 4x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 22 | 10+4x20 | 10 | 4x20 | 36696 for 10MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 23 | 10+20+3x20 | 10+20 | 3x20 | 36696 for 10MHz CC 75376 for 20MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC, R.31-4 FDD for 20MHz CC | R.31-4 TDD | 85 |
| 24 | 2x10+3x20 | 2x10 | 3x20 | 36696 for 10MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 25 | 10+2×20+2×20 | 10+2x20 | 2x20 | 36696 for 10MHz CC 75376 for 20MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC, R.31-4 FDD for 20MHz CC | R.31-4 TDD | 85 |
| 26 | 10+15+20+2x20 | 10+15+20 | 2x20 | 36696 for 10MHz CC 55056 for 15MHz CC 75376 for 20MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC, | R.31-4 TDD | 85 |
| 27 | 20+5x20 | 20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 28 | 2x20+4x20 | 2x20 | 4x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 29 | 20+6x20 | 20 | 6x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 30 | 2x20+5x20 | 2x20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |

Table 8.7.5.1-2: Test points for sustained data rate (FRC 64QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |  | DL Cat. 6, 7 | DL Cat. 9, 10 | DL Cat. 11, 12 |  |
| CA with 2CCs | 2x20 | 20 | 20 | - | - | 3 | 3 | 1 | 1 | - | - |
|  | 10+20 | 10 | 20 | - | - | 3 | 3 | 2 | 2 | - | - |
|  | 15+20 | 15 | 20 | - | - | 3 | 3 | 2A | 2A | - | - |
|  | 10+10 | 10 | 10 | - | - | 3 | 3 | 3 | 3 | - | - |
| CA with 3CCs | 3x20 | 20 | 2x20 | - | - | - | - | 1 | 4 | 4 | - |
|  | 15+20+20 | 15 | 2x20 | - | - | - | - | 2A | 5 | 5 | - |
|  | 10+20+20 | 10 | 2x20 | - | - | - | - | 2 | 6 | 6 | - |
|  | 3x20 | 2x20 | 20 | - | - | - | - | 1 | 7 | 7 | - |
|  | 20+20+15 | 20+15 | 20 | - | - | - | - | 1 | 8 | 8 | - |
|  | 20+20+10 | 20+10 | 20 | - | - | - | - | 1 | 9 | 9 | - |
|  | 20+10+10 | 2x10 | 20 | - | - | - | - | 2 | 9A | 9A | - |
|  | 10+15+20 | 10 | 15+20 |  |  |  |  | 2 | 9B | 9B |  |
|  | 10+15+20 | 10+15 | 20 |  |  |  |  | 2A | 9C | 9C |  |
| CA with 4CCs | 4x20 | 20 | 3x20 | - | - | - | - | - | 4 | 10 | 10 |
|  | 4x20 | 2×20 | 2×20 | - | - | - | - | - | 4 or 7 | 11 | 11 |
|  | 3x20+15 | 20+15 | 2×20 | - | - | - | - | - | 4 | 12 | 12 |
|  | 2×15+2x20 | 2×15 | 2x20 | - | - | - | - | - | 5 | 13 | 13 |
|  | 3x20+15 | 2×20+15 | 20 | - | - | - | - | - | 7 | 14 | 14 |
|  | 2×15+2x20 | 2x15+20 | 20 | - | - | - | - | - | 8 | 15 | 15 |
|  | 3x20+10 | 2x20+10 | 20 | - | - | - | - | - | 7 | 15A | 15A |
|  | 4x20 | 2x15+20 | 20 | - | - | - | - | - | 8 | 15B | 15B |
|  | 10+3x20 | 10 | 3x20 |  |  |  |  |  | 6 | 15C | 15C |
|  | 2x10+2x20 | 2x10 | 2x20 |  |  |  |  |  | 6 | 15D | 15D |
|  | 2x10+20+15 | 2x10 | 20+15 |  |  |  |  |  | 9A | 15E | 15E |
|  | 10+3x20 | 10+20 | 2x20 |  |  |  |  |  | 4 | 15F | 15F |
|  | 10+15+2x20 | 10+15 | 2x20 |  |  |  |  |  | 5 | 15G | 15G |
|  | 10+15+2x20 | 10+15+20 | 20 |  |  |  |  |  | 8 | 15H | 15H |
| CA with 5 CCs | 4x20+15 | 2x20+15 | 2x20 | - | - | - | - | - | - | 11 | 16 |
|  | 2x15+3x20 | 2x15+20 | 2x20 | - | - | - | - | - | - | 12 | 17 |
|  | 4x20+20 | 4x20 | 20 |  |  |  |  |  |  |  | 18 |
|  | 3x20+2x20 | 3x20 | 2x20 |  |  |  |  |  |  |  | 19 |
|  | 2x20+3x20 | 2x20 | 3x20 |  |  |  |  |  |  |  | 20 |
|  | 20+4x20 | 20 | 4x20 |  |  |  |  |  |  |  | 21 |
|  | 10+4x20 | 10 | 4x20 |  |  |  |  |  |  |  | 22 |
|  | 10+20+3x20 | 10+20 | 3x20 |  |  |  |  |  |  |  | 23 |
|  | 2x10+3x20 | 2x10 | 3x20 |  |  |  |  |  |  |  | 24 |
|  | 10+2×20+2×20 | 10+2x20 | 2x20 |  |  |  |  |  |  |  | 25 |
|  | 10+15+20+2x20 | 10+15+20 | 2x20 |  |  |  |  |  |  |  | 26 |
| CA with 6 CCs | 20+5x20 | 20 | 5x20 |  |  |  |  |  |  | 27 | 27 |
|  | 2x20+4x20 | 2x20 | 4x20 |  |  |  |  |  |  | 28 | 28 |
| CA with 7 CCs | 20+6x20 | 20 | 6x20 |  |  |  |  |  |  | 29 | 29 |
|  | 2x20+5x20 | 2x20 | 5x20 |  |  |  |  |  |  | 30 | 30 |
| Note 1: Void. Note 2: Void. Note 3: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.7.5.1-3: Minimum requirement (TDD FDD CA 256QAM)

| Test number | Bandwidth (MHz) |  |  | Measurement channel |  |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC |  | TB success rate [%] |
| 1 | 2x20 | 20 | 20 | R.68 FDD |  | R.68 TDD | 85 |
| 2 | 10+20 | 10 | 20 | R.68-2 FDD |  | R.68 TDD | 85 |
| 3 | 15+20 | 15 | 20 | R.68-1 FDD |  | R.68 TDD | 85 |
| 4 | 3x20 | 20 | 2x20 | R.68 FDD |  | R.68 TDD | 85 |
| 5 | 15+20+20 | 15 | 2x20 | R.68-1 FDD |  | R.68 TDD | 85 |
| 6 | 10+20+20 | 10 | 2x20 | R.68-2 FDD |  | R.68TDD | 85 |
| 7 | 3x20 | 2x20 | 20 | R.68 FDD |  | R.68 TDD | 85 |
| 8 | 20+20+15 | 20+15 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 9 | 20+20+10 | 20+10 | 20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC |  | R.68 TDD | 85 |
| 9A | 20+10+10 | 2x10 | 20 | R.68-2 FDD |  | R.68 TDD | 85 |
| 9B | 10+15+20 | 10 | 15+20 | R.68-2 FDD |  | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 9C | 10+15+20 | 10+15 | 20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 10 | 4x20 | 20 | 3x20 | R.68 FDD |  | R.68TDD | 85 |
| 11 | 4x20 | 2×20 | 2×20 | R.68 FDD |  | R.68 TDD | 85 |
| 12 | 3x20+15 | 20+15 | 2×20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 13 | 2×15+2x20 | 2×15 | 2x20 | R.68-1 FDD |  | R.68 TDD | 85 |
| 14 | 3x20+15 | 2×20+15 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 15 | 2×15+2x20 | 2x15+20 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 15A | 3x20+10 | 2x20+10 | 20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC |  | R.68 TDD | 85 |
| 15B | 4x20 | 3x20 | 20 | R.68 FDD |  | R.68TDD | 85 |
| 15C | 10+3x20 | 10 | 3x20 | R.68-2 FDD |  | R.68TDD | 85 |
| 15D | 2x10+2x20 | 2x10 | 2x20 | R.68-2 FDD |  | R.68TDD | 85 |
| 15E | 2x10+20+15 | 2x10 | 20+15 | R.68-2 FDD |  | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 15F | 10+3x20 | 10+20 | 2x20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC |  | R.68 TDD | 85 |
| 15G | 10+15+2x20 | 10+15 | 2x20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 15H | 10+15+2x20 | 10+15+20 | 20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC, R.68 FDD for 20MHz CC |  | R.68 TDD | 85 |
| 16 | 4x20+15 | 2x20+15 | 2x20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 17 | 2x15+3x20 | 2x15+20 | 2x20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC |  | R.68 TDD | 85 |
| 18 | 4x20+20 | 4x20 | 20 | R.68 FDD |  | R.68TDD | 85 |
| 19 | 3x20+2x20 | 3x20 | 2x20 | R.68 FDD |  | R.68 TDD | 85 |
| 20 | 2x20+3x20 | 2x20 | 3x20 | R.68 FDD |  | R.68TDD | 85 |
| 21 | 20+4x20 | 20 | 4x20 | R.68 FDD |  | R.68 TDD | 85 |
| 22 | 10+4x20 | 10 | 4x20 | R.68-2 FDD |  | R.68 TDD | 85 |
| 23 | 10+20+3x20 | 10+20 | 3x20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC |  | R.68 TDD | 85 |
| 24 | 2x10+3x20 | 2x10 | 3x20 | R.68-2 FDD |  | R.68 TDD | 85 |
| 25 | 10+2×20+2×20 | 10+2x20 | 2x20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC |  | R.68 TDD | 85 |
| 26 | 10+15+20+2x20 | 10+15+20 | 2x20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC, R.68 FDD for 20MHz CC |  | R.68 TDD | 85 |
| 27 | 20+5x20 | 20 | 5x20 | R.68 FDD |  | R.68 TDD | 85 |
| 28 | 2x20+4x20 | 2x20 | 4x20 | R.68 FDD |  | R.68 TDD | 85 |
| 29 | 20+6x20 | 20 | 6x20 | R.68 FDD |  | R.68 TDD | 85 |
| 30 | 2x20+5x20 | 2x20 | 5x20 | R.68 FDD |  | R.68 TDD | 85 |

Table 8.7.5.1-4: Test points for sustained data rate (FRC 256QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | DL Cat. 11, 12 |  |  |  |  |  |
| CA with 2CCs | 2x20 | 20 | 20 | 1 | 1 | - | - |  |  |
|  | 10+20 | 10 | 20 | 2 | 2 |  |  |  |  |
|  | 15+20 | 15 | 20 | 3 | 3 | - | - |  |  |
| CA with 3CCs | 3x20 | 20 | 2x20 | 4 | 1 | 4 | - |  |  |
|  | 15+20+20 | 15 | 2x20 | 5 | 3 | 5 | - |  |  |
|  | 10+20+20 | 10 | 2x20 | 6 | 2 | 6 | - |  |  |
|  | 3x20 | 2x20 | 20 | 7 | 1 | 7 | - |  |  |
|  | 20+20+15 | 20+15 | 20 | 8 | 1 | 8 | - |  |  |
|  | 20+20+10 | 20+10 | 20 | 9 | 1 | 9 | - |  |  |
|  | 20+10+10 | 2x10 | 20 | 9A | 2 | 9A |  |  |  |
|  | 10+15+20 | 10 | 15+20 | 9B | 2 | 9B |  |  |  |
|  | 10+15+20 | 10+15 | 20 | 9C | 3 | 9C |  |  |  |
| CA with 4CCs | 4x20 | 20 | 3x20 | 4 | - | 10 | 10 |  |  |
|  | 4x20 | 2×20 | 2×20 | 4 or 7 | - | 11 | 11 |  |  |
|  | 3x20+15 | 20+15 | 2×20 | 8 | - | 12 | 12 |  |  |
|  | 2×15+2x20 | 2×15 | 2x20 | 5 | - | 13 | 13 |  |  |
|  | 3x20+15 | 2×20+15 | 20 | 7 | - | 14 | 14 |  |  |
|  | 2×15+2x20 | 2x15+20 | 20 | 8 | - | 15 | 15 |  |  |
|  | 3x20+10 | 2x20+10 | 20 | 7 | - | 15A | 15A |  |  |
|  | 4x20 | 2x15+20 | 20 | 8 | - | 15B | 15B |  |  |
|  | 10+3x20 | 10 | 3x20 | 6 |  | 15C | 15C |  |  |
|  | 2x10+2x20 | 2x10 | 2x20 | 6 |  | 15D | 15D |  |  |
|  | 2x10+20+15 | 2x10 | 20+15 | 9A |  | 15E | 15E |  |  |
|  | 10+3x20 | 10+20 | 2x20 | 4 |  | 15F | 15F |  |  |
|  | 10+15+2x20 | 10+15 | 2x20 | 5 |  | 15G | 15G |  |  |
|  | 10+15+2x20 | 10+15+20 | 20 | 8 |  | 15H | 15H |  |  |
| CA with 5CCs | 4x20+15 | 2x20+15 | 2x20 | - | - | 14 or 12 | 16 |  |  |
|  | 2x15+3x20 | 2x15+20 | 2x20 | - | - | 15 or 12 | 17 |  |  |
|  | 4x20+20 | 4x20 | 20 |  |  |  | 18 |  |  |
|  | 3x20+2x20 | 3x20 | 2x20 |  |  |  | 19 |  |  |
|  | 2x20+3x20 | 2x20 | 3x20 |  |  |  | 20 |  |  |
|  | 20+4x20 | 20 | 4x20 |  |  |  | 21 |  |  |
|  | 10+4x20 | 10 | 4x20 |  |  |  | 22 |  |  |
|  | 10+20+3x20 | 10+20 | 3x20 |  |  |  | 23 |  |  |
|  | 2x10+3x20 | 2x10 | 3x20 |  |  |  | 24 |  |  |
|  | 10+2×20+2×20 | 10+2x20 | 2x20 |  |  |  | 25 |  |  |
|  | 10+15+20+2x20 | 10+15+20 | 2x20 |  |  |  | 26 |  |  |
| CA with 6CCs | 20+5x20 | 20 | 5x20 |  |  |  | 27 |  |  |
|  | 2x20+4x20 | 2x20 | 4x20 |  |  |  | 28 |  |  |
| CA with 7CCs | 20+6x20 | 20 | 6x20 |  |  |  | 29 |  |  |
|  | 2x20+5x20 | 2x20 | 5x20 |  |  |  | 30 |  |  |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |

#### 8.7.5.2	Minimum Requirement TDD PCell

For UE not supporting 256QAM, the requirements for TDD FDD CA with TDD PCell are specified in Table 8.7.5.2-1 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.5.2-2. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements for TDD FDD CA with FDD PCell are specified in Table 8.7.5.2-3 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category or UE DL category, and bandwidth combination with maximum aggregated bandwidth as specified in Table 8.7.5.2-4. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirements in Table 8.7.5.2-1 is not applicable.

The applicability of ther requirements are specified in Clause 8.1.2.3B. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.5.2-1: test parameters for sustained downlink data rate (TDD FDD CA 64QAM)

| Test number | Bandwidth (MHz) |  |  | Number of bits of a DL-SCH transport block received within a TTI (for normal/special subframe for TDD, except for subframe #5) |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 1 | 2x20 | 20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 2 | 10+20 | 10 | 20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 2A | 15+20 | 15 | 20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 3 | 10+10 | 10 | 10 | 36696 | 36696/0 | R.31-3A FDD | R.31-6 TDD | 85 |
| 4 | 3x20 | 20 | 2x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 5 | 15+20+20 | 15 | 2x20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 6 | 10+20+20 | 10 | 2x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 7 | 3x20 | 2x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 8 | 20+20+15 | 20+15 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 9 | 20+20+10 | 20+10 | 20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 9A | 20+10+10 | 2x10 | 20 | 36696 | 75376/0 | R.31-3A | R.31-4 TDD | 85 |
| 9B | 10+15+20 | 10 | 15+20 | 36696 | 75376/0 for 20MHz CC 55056/0 for 15MHz CC | R.31-3A FDD | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 9C | 10+15+20 | 10+15 | 20 | 36696 for 10MHz CC 55056 for 15MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 10 | 4x20 | 20 | 3x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 11 | 4x20 | 2×20 | 2×20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 12 | 3x20+15 | 20+15 | 2×20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 13 | 2×15+2x20 | 2×15 | 2x20 | 55056 | 75376/0 | R.31-5 FDD | R.31-4 TDD | 85 |
| 14 | 3x20+15 | 2×20+15 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15 | 2×15+2x20 | 2x15+20 | 20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15A | 3x20+10 | 2x20+10 | 20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 15B | 4x20 | 3x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 15C | 10+3x20 | 10 | 3x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 15D | 2x10+2x20 | 2x10 | 2x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 15E | 2x10+20+15 | 2x10 | 20+15 | 36696 | 75376/0 for 20MHz CC 55056/0 for 15MHz CC | R.31-3A FDD | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| 15F | 10+3x20 | 10+20 | 2x20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 15G | 10+15+2x20 | 10+15 | 2x20 | 36696 for 10MHz CC 55056 for 15MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 15H | 10+15+2x20 | 10+15+20 | 20 | 36696 for 10MHz CC 55056 for 15MHz CC 75376 for 20MHz CC | 75376/0 | R.31-3A FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | R.31-4 TDD | 85 |
| 16 | 4x20+15 | 2x20+15 | 2x20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 17 | 2x15+3x20 | 2x15+20 | 2x20 | 75376 for 20MHz CC 55056 for 15MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC | R.31-4 TDD | 85 |
| 18 | 4x20+20 | 4x20 | 20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 19 | 3x20+2x20 | 3x20 | 2x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 20 | 2x20+3x20 | 2x20 | 3x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 21 | 20+4x20 | 20 | 4x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 22 | 10+4x20 | 10 | 4x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 23 | 10+20+3x20 | 10+20 | 3x20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 24 | 2x10+3x20 | 2x10 | 3x20 | 36696 | 75376/0 | R.31-3A FDD | R.31-4 TDD | 85 |
| 25 | 10+2×20+2×20 | 10+2x20 | 2x20 | 75376 for 20MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 26 | 10+15+20+2x20 | 10+15+20 | 2x20 | 75376 for 20MHz CC 55056 for 15MHz CC 36696 for 10MHz CC | 75376/0 | R.31-4 FDD for 20MHz CC, R.31-5 FDD for 15MHz CC, R.31-3A FDD for 10MHz CC | R.31-4 TDD | 85 |
| 27 | 20+5x20 | 20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 28 | 2x20+4x20 | 2x20 | 4x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 29 | 20+6x20 | 20 | 6x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 30 | 2x20+5x20 | 2x20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |

Table 8.7.5.2-2: Test points for sustained data rate (FRC 64QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |  | DL Cat. 6, 7 | DL Cat. 9, 10 | DL Cat. 11, 12 |  |
| CA with 2CCs | 2x20 | 20 | 20 | - | - | 3 | 3 | 1 | 1 | - | - |
|  | 10+20 | 10 | 20 | - | - | 3 | 3 | 2 | 2 | - | - |
|  | 15+20 | 15 | 20 | - | - | 3 | 3 | 2A | 2A | - | - |
|  | 10+10 | 10 | 10 | - | - | 3 | 3 | 3 | 3 | - | - |
| CA with 3CCs | 3x20 | 20 | 2x20 | - | - | - | - | 1 | 4 | 4 | - |
|  | 15+20+20 | 15 | 2x20 | - | - | - | - | 2A | 5 | 5 | - |
|  | 10+20+20 | 10 | 2x20 | - | - | - | - | 2 | 6 | 6 | - |
|  | 3x20 | 2x20 | 20 | - | - | - | - | 1 | 7 | 7 | - |
|  | 20+20+15 | 20+15 | 20 | - | - | - | - | 1 | 8 | 8 | - |
|  | 20+20+10 | 20+10 | 20 | - | - | - | - | 1 | 9 | 9 | - |
|  | 20+10+10 | 2x10 | 20 | - | - | - | - | 2 | 9A | 9A | - |
|  | 10+15+20 | 10 | 15+20 |  |  |  |  | 2 | 9B | 9B |  |
|  | 10+15+20 | 10+15 | 20 |  |  |  |  | 2A | 9C | 9C |  |
| CA with 4CCs | 4x20 | 20 | 3x20 | - | - | - | - | - | 4 | 10 | 10 |
|  | 4x20 | 2×20 | 2×20 | - | - | - | - | - | 4 or 7 | 11 | 11 |
|  | 3x20+15 | 20+15 | 2×20 | - | - | - | - | - | 4 | 12 | 12 |
|  | 2×15+2x20 | 2×15 | 2x20 | - | - | - | - | - | 5 | 13 | 13 |
|  | 3x20+15 | 2×20+15 | 20 | - | - | - | - | - | 7 | 14 | 14 |
|  | 2×15+2x20 | 2x15+20 | 20 | - | - | - | - | - | 8 | 15 | 15 |
|  | 3x20+10 | 2x20+10 | 20 | - | - | - | - | - | 7 | 15A | 15A |
|  | 4x20 | 2x15+20 | 20 | - | - | - | - | - | 8 | 15B | 15B |
|  | 10+3x20 | 10 | 3x20 |  |  |  |  |  | 6 | 15C | 15C |
|  | 2x10+2x20 | 2x10 | 2x20 |  |  |  |  |  | 6 | 15D | 15D |
|  | 2x10+20+15 | 2x10 | 20+15 |  |  |  |  |  | 9A | 15E | 15E |
|  | 10+3x20 | 10+20 | 2x20 |  |  |  |  |  | 4 | 15F | 15F |
|  | 10+15+2x20 | 10+15 | 2x20 |  |  |  |  |  | 5 | 15G | 15G |
|  | 10+15+2x20 | 10+15+20 | 20 |  |  |  |  |  | 8 | 15H | 15H |
| CA with 5 CCs | 4x20+15 | 2x20+15 | 2x20 | - | - | - | - | - | - | 11 | 16 |
|  | 2x15+3x20 | 2x15+20 | 2x20 | - | - | - | - | - | - | 12 | 17 |
|  | 4x20+20 | 4x20 | 20 |  |  |  |  |  |  |  | 18 |
|  | 3x20+2x20 | 3x20 | 2x20 |  |  |  |  |  |  |  | 19 |
|  | 2x20+3x20 | 2x20 | 3x20 |  |  |  |  |  |  |  | 20 |
|  | 20+4x20 | 20 | 4x20 |  |  |  |  |  |  |  | 21 |
|  | 10+4x20 | 10 | 4x20 |  |  |  |  |  |  |  | 22 |
|  | 10+20+3x20 | 10+20 | 3x20 |  |  |  |  |  |  |  | 23 |
|  | 2x10+3x20 | 2x10 | 3x20 |  |  |  |  |  |  |  | 24 |
|  | 10+2×20+2×20 | 10+2x20 | 2x20 |  |  |  |  |  |  |  | 25 |
|  | 10+15+20+2x20 | 10+15+20 | 2x20 |  |  |  |  |  |  |  | 26 |
| CA with 6 CCs | 20+5x20 | 20 | 5x20 |  |  |  |  |  |  | 27 | 27 |
|  | 2x20+4x20 | 2x20 | 4x20 |  |  |  |  |  |  | 28 | 28 |
| CA with 7 CCs | 20+6x20 | 20 | 6x20 |  |  |  |  |  |  | 29 | 29 |
|  | 2x20+5x20 | 2x20 | 5x20 |  |  |  |  |  |  | 30 | 30 |
| Note 1: Void. Note 2: Void. Note 3: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.7.5.2-3: Minimum requirement (TDD FDD CA 256QAM)

| Test number | Bandwidth (MHz) |  |  |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC |  | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 1 | 2x20 |  | 20 | 20 | R.68 FDD | R.68 TDD | 85 |
| 2 | 10+20 |  | 10 | 20 | R.68-2 FDD | R.68 TDD | 85 |
| 3 | 15+20 |  | 15 | 20 | R.68-1 FDD | R.68 TDD | 85 |
| 4 | 3x20 |  | 20 | 2x20 | R.68 FDD | R.68 TDD | 85 |
| 5 | 15+20+20 |  | 15 | 2x20 | R.68-1 FDD | R.68 TDD | 85 |
| 6 | 10+20+20 |  | 10 | 2x20 | R.68-2 FDD | R.68TDD | 85 |
| 7 | 3x20 |  | 2x20 | 20 | R.68 FDD | R.68 TDD | 85 |
| 8 | 20+20+15 |  | 20+15 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 9 | 20+20+10 |  | 20+10 | 20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC | R.68 TDD | 85 |
| 9A | 20+10+10 |  | 2x10 | 20 | R.68-2 FDD | R.68 TDD | 85 |
| 9B | 10+15+20 |  | 10 | 15+20 | R.68-2 FDD | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 9C | 10+15+20 |  | 10+15 | 20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 10 | 4x20 |  | 20 | 3x20 | R.68 FDD | R.68TDD | 85 |
| 11 | 4x20 |  | 2×20 | 2×20 | R.68 FDD | R.68 TDD | 85 |
| 12 | 3x20+15 |  | 20+15 | 2×20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 13 | 2×15+2x20 |  | 2×15 | 2x20 | R.68-1 FDD | R.68 TDD | 85 |
| 14 | 3x20+15 |  | 2×20+15 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 15 | 2×15+2x20 |  | 2x15+20 | 20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 15A | 3x20+10 |  | 2x20+10 | 20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC | R.68 TDD | 85 |
| 15B | 4x20 |  | 3x20 | 20 | R.68 FDD | R.68TDD | 85 |
| 15C | 10+3x20 |  | 10 | 3x20 | R.68-2 FDD | R.68TDD | 85 |
| 15D | 2x10+2x20 |  | 2x10 | 2x20 | R.68-2 FDD | R.68TDD | 85 |
| 15E | 2x10+20+15 |  | 2x10 | 20+15 | R.68-2 FDD | R.68-1 TDD for 15MHz CC R.68 TDD for 20MHz CC | 85 |
| 15F | 10+3x20 |  | 10+20 | 2x20 | R.68 FDD for 20MHz CC, R.68-2 FDD for 10MHz CC | R.68 TDD | 85 |
| 15G | 10+15+2x20 |  | 10+15 | 2x20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 15H | 10+15+2x20 |  | 10+15+20 | 20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC, R.68 FDD for 20MHz CC | R.68 TDD | 85 |
| 16 | 4x20+15 |  | 2x20+15 | 2x20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 17 | 2x15+3x20 |  | 2x15+20 | 2x20 | R.68 FDD for 20MHz CC, R.68-1 FDD for 15MHz CC | R.68 TDD | 85 |
| 18 | 4x20+20 |  | 4x20 | 20 | R.68 FDD | R.68 TDD | 85 |
| 19 | 3x20+2x20 |  | 3x20 | 2x20 | R.68 FDD | R.68 TDD | 85 |
| 20 | 2x20+3x20 |  | 2x20 | 3x20 | R.68 FDD | R.68 TDD | 85 |
| 21 | 20+4x20 |  | 20 | 4x20 | R.68 FDD | R.68 TDD | 85 |
| 22 | 10+4x20 |  | 10 | 4x20 | R.68-2 FDD | R.68 TDD | 85 |
| 23 | 10+20+3x20 |  | 10+20 | 3x20 | R.68-2 FDD for 10MHz CC, R.68 FDD for 20MHz CC | R.68 TDD | 85 |
| 24 | 2x10+3x20 |  | 2x10 | 3x20 | R.68-2 FDD | R.68 TDD | 85 |
| 25 | 10+2×20+2×20 |  | 10+2x20 | 2x20 | R.68-2 FDD for 10MHz CC, R.68 FDD for 20MHz CC | R.68 TDD | 85 |
| 26 | 10+15+20+2x20 |  | 10+15+20 | 2x20 | R.68-2 FDD for 10MHz CC, R.68-1 FDD for 15MHz CC, R.68 FDD for 20MHz CC | R.68 TDD | 85 |
| 27 | 20+5x20 |  | 20 | 5x20 | R.68 FDD | R.68 TDD | 85 |
| 28 | 2x20+4x20 |  | 2x20 | 4x20 | R.68 FDD | R.68 TDD | 85 |
| 29 | 20+6x20 |  | 20 | 6x20 | R.68 FDD | R.68 TDD | 85 |
| 30 | 2x20+5x20 |  | 2x20 | 5x20 | R.68 FDD | R.68 TDD | 85 |

Table 8.7.5.2-4: Test points for sustained data rate (FRC 256QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | DL Cat. 11, 12 |  |  |  |  |  |
| CA with 2CCs | 2x20 | 20 | 20 | 1 | 1 | - | - |  |  |
|  | 10+20 | 10 | 20 | 2 | 2 |  |  |  |  |
|  | 15+20 | 15 | 20 | 3 | 3 | - | - |  |  |
| CA with 3CCs | 3x20 | 20 | 2x20 | 4 | 1 | 4 | - |  |  |
|  | 15+20+20 | 15 | 2x20 | 5 | 3 | 5 | - |  |  |
|  | 10+20+20 | 10 | 2x20 | 6 | 2 | 6 | - |  |  |
|  | 3x20 | 2x20 | 20 | 7 | 1 | 7 | - |  |  |
|  | 20+20+15 | 20+15 | 20 | 8 | 1 | 8 | - |  |  |
|  | 20+20+10 | 20+10 | 20 | 9 | 1 | 9 | - |  |  |
|  | 20+10+10 | 2x10 | 20 | 9A | 2 | 9A |  |  |  |
|  | 10+15+20 | 10 | 15+20 | 9B | 2 | 9B |  |  |  |
|  | 10+15+20 | 10+15 | 20 | 9C | 3 | 9C |  |  |  |
| CA with 4CCs | 4x20 | 20 | 3x20 | 4 | - | 10 | 10 |  |  |
|  | 4x20 | 2×20 | 2×20 | 4 or 7 | - | 11 | 11 |  |  |
|  | 3x20+15 | 20+15 | 2×20 | 8 | - | 12 | 12 |  |  |
|  | 2×15+2x20 | 2×15 | 2x20 | 5 | - | 13 | 13 |  |  |
|  | 3x20+15 | 2×20+15 | 20 | 7 | - | 14 | 14 |  |  |
|  | 2×15+2x20 | 2x15+20 | 20 | 8 | - | 15 | 15 |  |  |
|  | 3x20+10 | 2x20+10 | 20 | 7 | - | 15A | 15A |  |  |
|  | 4x20 | 2x15+20 | 20 | 8 | - | 15B | 15B |  |  |
|  | 10+3x20 | 10 | 3x20 | 6 |  | 15C | 15C |  |  |
|  | 2x10+2x20 | 2x10 | 2x20 | 6 |  | 15D | 15D |  |  |
|  | 2x10+20+15 | 2x10 | 20+15 | 9A |  | 15E | 15E |  |  |
|  | 10+3x20 | 10+20 | 2x20 | 4 |  | 15F | 15F |  |  |
|  | 10+15+2x20 | 10+15 | 2x20 | 5 |  | 15G | 15G |  |  |
|  | 10+15+2x20 | 10+15+20 | 20 | 8 |  | 15H | 15H |  |  |
| CA with 5CCs | 4x20+15 | 2x20+15 | 2x20 | - | - | 14 or 12 | 16 |  |  |
|  | 2x15+3x20 | 2x15+20 | 2x20 | - | - | 15 or 12 | 17 |  |  |
|  | 4x20+20 | 4x20 | 20 |  |  |  | 18 |  |  |
|  | 3x20+2x20 | 3x20 | 2x20 |  |  |  | 19 |  |  |
|  | 2x20+3x20 | 2x20 | 3x20 |  |  |  | 20 |  |  |
|  | 20+4x20 | 20 | 4x20 |  |  |  | 21 |  |  |
|  | 10+4x20 | 10 | 4x20 |  |  |  | 22 |  |  |
|  | 10+20+3x20 | 10+20 | 3x20 |  |  |  | 23 |  |  |
|  | 2x10+3x20 | 2x10 | 3x20 |  |  |  | 24 |  |  |
|  | 10+2×20+2×20 | 10+2x20 | 2x20 |  |  |  | 25 |  |  |
|  | 10+15+20+2x20 | 10+15+20 | 2x20 |  |  |  | 26 |  |  |
| CA with 6CCs | 20+5x20 | 20 | 5x20 |  |  |  | 27 |  |  |
|  | 2x20+4x20 | 2x20 | 4x20 |  |  |  | 28 |  |  |
| CA with 7CCs | 20+6x20 | 20 | 6x20 |  |  |  | 29 |  |  |
|  | 2x20+5x20 | 2x20 | 5x20 |  |  |  | 30 |  |  |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |

### 8.7.6	FDD (DC)

The parameters specified in Table 8.7.6-1 are valid for all FDD DC tests unless otherwise stated.

Table 8.7.6-1: Common Test Parameters (FDD)

| Parameter |  | Unit |  | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  |  | Normal |  |
| Cell ID |  |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  |  | Not configured |  |
| Propagation condition |  |  |  |  | Static propagation condition No external noise sources are applied |  |
| Transmission mode |  |  |  |  | TM3 |  |
| Codebook subset restriction |  |  |  |  | 10 |  |
| Antenna configuration |  |  |  |  | 2x2 |  |
| at antenna port (dBm/15kHz) |  |  |  |  | -85 |  |
| Symbols for unused PRBs |  |  |  |  | OP.1 FDD |  |
| ACK/NACK feedback mode |  |  |  |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |  |
| Time offset between MCG CC and SCG CC |  |  |  | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |  |
| Downlink power allocation |  |  |  | dB | -3 |  |
|  |  |  |  | dB | -3 |  |
|  |  |  |  | dB | 0 |  |
| Note 1: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 2: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |  |  |  |  |

For UE not supporting 256QAM, the requirements are specified in Table 8.7.6-2, with the addition of the parameters in Table 8.7.6-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.6-3. The TB success rate across CGs shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements are specified in Table 8.7.6-4, with the addition of the parameters in Table 8.7.6-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.6-5. The TB success rate across CGs shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirements in Table 8.7.6-2 are not applicable.

The applicability of ther requirements are specified in Clause 8.1.2.3A.

Table 8.7.6-2: Minimum requirement (DC 64QAM)

| Test number | Bandwidth combination (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value TB success rate(%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  |  | MCG | SCG |
| 1 | 2x10 | 25456 | R.31-2 FDD | 95 | 95 | 95 |
| 2 | 2x10 | 36696 (Note 4) | R.31-3A FDD | 85 | 85 | 85 |
| 3 | 10+20 | 36696 (Note 4) for 10MHz CC 75376 (Note 5) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 4 | 2x15 | 55056 (Note 6) | R.31-4B FDD | 85 | 85 | 85 |
| 5 | 15+20 | 55056 for 15MHz CC 75376 (Note 5) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 6 | 2x20 | 75376 (Note 5) | R.31-4 FDD | 85 | 85 | 85 |
| 6A | 10+15 | 36696 (Note 4) for 10MHz CC 55056 (Note 6) for 15MHz CC | R.31-2 FDD for 10MHz CC R.31-5 FDD for 15MHz CC | 85 | 85 | 85 |
| 7 | 15+5 | 55056 for 15MHz CC 18336 for 5MHz CC | R.31-5 FDD for 15MHz CC R.31-6 FDD for 5MHz CC | 85 | 85 | 85 |
| 8 | 15+20+20 | 55056 for 15MHz CC 75376 (Note 5) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 9 | 15+15+20 | 55056for 15MHz CC 75376 (Note 5) for 20MHz CC | R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 10 | 10+10+20 | 36696 (Note 4) for 10MHz CC 75376 (Note 5) for 20MHz CC | R.31-2 FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 11 | 10+15+15 | 36696 (Note 4) for 10MHz CC 55056 (Note 6) for 15MHz CC | R.31-2 FDD for 10MHz CC R.31-5 FDD for 15MHz CC | 85 | 85 | 85 |
| 12 | 10+15+20 | 36696 (Note 4) for 10MHz CC 55056 (Note 6) for 15MHz CC 75376 (Note 5) for 20MHz CC | R.31-2 FDD for 10MHz CC R.31-5 FDD for 15MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 13 | 10+20+20 | 36696 (Note 4) for 10MHz CC 75376 (Note 5) for 20MHz CC | R.31-2 FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 | 85 | 85 |
| 14 | 3x20 | 75376 (Note 5) | R.31-4 FDD | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks , NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. Note 4: 35160 bits for sub-frame 5. Note 5: 71112 bits for sub-frame 5. Note 6: 52752 bits for sub-frame 5. |  |  |  |  |  |  |

Table 8.7.6-3: Test points for sustained data rate (FRC DC 64QAM)

| DC config | Maximum supported Bandwidth combination (MHz) | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9, 10 | Cat. 11, 12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL Cat. 11,12 |  |
| DC with 2CCs | 2x10 | 1 | 2 | 2 | 2 | - |  |
|  | 10+20 | 1 | 2 | 3 | 3 | - |  |
|  | 2x15 | 1 | 2 | 4 | 4 | - |  |
|  | 15+20 | 1 | 2 | 5 | 5 | - |  |
|  | 2x20 | 1 | 2 | 6 | 6 | - |  |
|  | 10+15 | 1 | 2 | 6A | 6A | - |  |
|  | 15+5 | 7 | 7 | 7 | 7 | - |  |
| DC with 3 CCs | 15+20+20 | - | - | 8 | 8 | 8 |  |
|  | 15+15+20 | - | - | 9 | 9 | 9 |  |
|  | 10+10+20 | - | - | 10 | 10 | 10 |  |
|  | 10+15+15 | - | - | 11 | 11 | 11 |  |
|  | 10+15+20 | - | - | 12 | 12 | 12 |  |
|  | 10+20+20 | - | - | 13 | 13 | 13 |  |
|  | 20+20+20 | - | - | 14 | 14 | 14 |  |

Table 8.7.6-4: Minimum requirement (DC 256QAM)

| Test number | Bandwidth combination (MHz) | Measurement channel | Reference value TB success rate (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  | MCG | SCG |
| 1 | 2x10 | R.68-2 FDD | 85 | 85 | 85 |
| 2 | 10+20 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 3 | 2x15 | R.68-1 FDD | 85 | 85 | 85 |
| 4 | 15+20 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 5 | 2x20 | R.68 FDD | 85 | 85 | 85 |
| 6 | 15+5 | R.68-1 FDD for 15MHz CC R.68-3 FDD for 5MHz CC | 85 | 85 | 85 |
| 6A | 10+15 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC | 85 | 85 | 85 |
| 7 | 15+20+20 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 8 | 15+15+20 | R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 9 | 10+10+20 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 10 | 10+15+15 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC | 85 | 85 | 85 |
| 11 | 10+15+20 | R.68-2 FDD for 10MHz CC R.68-1 FDD for 15MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 12 | 10+20+20 | R.68-2 FDD for 10MHz CC R.68 FDD for 20MHz CC | 85 | 85 | 85 |
| 13 | 20+20+20 | R.68 FDD | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks , NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. |  |  |  |  |  |

Table 8.7.6-5: Test points for sustained data rate (FRC DC 256QAM)

| DC config | Maximum supported Bandwidth combination (MHz) | Cat. 11, 12 | DL Cat. 13 | DL Cat.15 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL Cat. 11,12 |  |  |  |  |  |
| DC with 2CCs | 2x10 | 1 | 1 |  |  |  |  |
|  | 10+20 | 2 | 2 |  |  |  |  |
|  | 2x15 | 3 | 3 |  |  |  |  |
|  | 15+20 | 4 | 4 |  |  |  |  |
|  | 2x20 | 5 | 5 |  |  |  |  |
|  | 10+15 | 6A | 6A | - |  |  |  |
|  | 15+5 | 6 | 6 |  |  |  |  |
| DC with 3CCs | 15+20+20 | 7 | 5 |  |  |  |  |
|  | 15+15+20 | 8 | 4 | 8 |  |  |  |
|  | 10+10+20 | 9 | 2 | 9 |  |  |  |
|  | 10+15+15 | 10 | 3 | 10 |  |  |  |
|  | 10+15+20 | 11 | 4 | 11 |  |  |  |
|  | 10+20+20 | 12 | 5 | 12 |  |  |  |
|  | 20+20+20 | 13 | 5 | 13 |  |  |  |

### 8.7.7	TDD (DC)

The parameters specified in Table 8.7.7-1 are valid for all TDD DC tests unless otherwise stated.

Table 8.7.7-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  |  | 2 (Note 2) |  |
| Special subframe configuration |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |  |
| Transmission mode |  |  |  | TM3 |  |
| Codebook subset restriction |  |  |  | 10 |  |
| Antenna configuration |  |  |  | 2x2 |  |
| at antenna port (dBm/15kHz) |  |  |  | -85 |  |
| Symbols for unused PRBs |  |  |  | OP.1 TDD |  |
| ACK/NACK feedback mode |  |  |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |  |
| Time offset between MCG CC and SCG CC |  |  | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |  |
| Downlink power allocation |  |  | dB | -3 |  |
|  |  |  | dB | -3 |  |
|  |  |  | dB | 0 |  |
| Note 1: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 2: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |  |  |  |

For UE not supporting 256QAM, the requirements are specified in Table 8.7.7-2, with the addition of the parameters in Table 8.7.7-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.7-3. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements are specified in Table 8.7.7-4, with the addition of the parameters in Table 8.7.7-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.7-5. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirements in Table 8.7.7-2 are not applicable.

The applicability of ther requirements are specified in Clause 8.1.2.3A.

Table 8.7.7-2: Minimum requirement (DC 64QAM)

| Test number | Bandwidth combination (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value TB success rate across CGs(%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  |  | MCG | SCG |
| 1 | 2x20 | 75376/0 (Note 4) | R.31-4A TDD | 85 | 85 | 85 |
| 2 | 3x20 | 75376/0 (Note 4) | R.31-4A TDD | 85 | 85 | 85 |
| 3 | 4x20 | 75376/0 (Note 4) | R.31-4A TDD | 85 | 85 | 85 |
| 4 | 15+3x20 | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 4) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer,the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. Note 4: 71112 bits for sub-frame 5. |  |  |  |  |  |  |

Table 8.7.7-3: Test points for sustained data rate (FRC DC 64QAM)

| DC config | Maximum supported Bandwidth combination (MHz) | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9, 10 | Cat. 11, 12 | DL Cat.15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL Cat. 11, 12 |  |
| DC with 2CCs | 2x20 | - | - | 1 | 1 | - | - |
| DC with 3CCs | 3x20 | - | - | 1 | 2 | 2 | - |
| DC with 4CCs | 4x20 | - | - | - | 2 | 3 | 3 |
|  | 15+3x20 | - | - | - | 2 | 4 | 4 |

Table 8.7.7-4: Minimum requirement (DC 256QAM)

| Test number | Bandwidth combination (MHz) | Measurement channel | Reference value TB success rate (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  | MCG | SCG |
| 1 | 2x20 | R.68-3 TDD | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. |  |  |  |  |  |

Table 8.7.7-5: Test points for sustained data rate (FRC DC 256QAM)

| DC config | Maximum supported Bandwidth combination (MHz) | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL Cat. 11, 12 |  |  |  |  |  |
| DC with 2CCs | 2x20 | 1 | 1 | - | - |  |  |
| DC with 3CCs | 3x20 | 2 | 1 | 2 | - |  |  |
| DC with 4CCs | 4x20 | 2 | - | 3 | 3 |  |  |
|  | 15+3x20 | 2 | - | 4 | 4 |  |  |

### 8.7.8	TDD FDD (DC)

The parameters specified in Table 8.7.8-1 are valid for all TDD FDD DC tests unless otherwise stated.

Table 8.7.8-1: Common Test Parameters (TDD FDD DC)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration for TDD CC |  |  |  | 2 (Note 2) |  |
| Special subframe configuration for TDD CC |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 for FDD CC; 7 for TDD CC |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |  |
| Transmission mode |  |  |  | TM3 |  |
| Codebook subset restriction |  |  |  | 10 |  |
| Antenna configuration |  |  |  | 2x2 |  |
| at antenna port (dBm/15kHz) |  |  |  | -85 |  |
| Symbols for unused PRBs |  |  |  | OP.1 TDD for TDD CC; OP.1 FDD for FDD CC |  |
| ACK/NACK feedback mode |  |  |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |  |
| Time offset between MCG CC and SCG CC |  |  | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |  |
| Downlink power allocation |  |  | dB | -3 |  |
|  |  |  | dB | -3 |  |
|  |  |  | dB | 0 |  |
| Note 1: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 2: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |  |  |  |

For UE not supporting 256QAM, the requirements are specified in Table 8.7.8-2, with the addition of the parameters in Table 8.7.8-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.8-3. The TB success rate shall be sustained during at least 300 frames.

For UE supporting 256QAM, the requirements are specified in Table 8.7.8-4, with the addition of the parameters in Table 8.7.7-1 and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified inTable 8.7.8-5. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirements in Table 8.7.8-2 are not applicable.

The applicability of ther requirements are specified in Clause 8.1.2.3A.

Table 8.7.8-2: Minimum requirement (TDD FDD DC 64QAM)

| Test number | Bandwidth (MHz) |  |  | Number of bits of a DL-SCH transport block received within a TTI (for normal/special subframe for TDD, except for subframe #5) |  | Measurement channel |  | Reference value TB success rate across CGs(%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  |  |  |  |  |  | MCG | SCG |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | FDD CC | TDD CC |  |  |  |
| 1 | 2x20 | 20 | 20 | 75376 (Note 4) | 75376/0 (Note 4) | R.31-4 FDD | R.31-4A TDD | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer,the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. Note 4: 71112 bits for sub-frame 5. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.8-3: Test points for sustained data rate (FRC TDD FDD DC 64QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 6, 7 | Cat. 9,10 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |
| DC with 2CCs | 2x20 | 20 | 20 | 1 | 1 |  |

Table 8.7.8-4: Minimum requirement (TDD FDD DC 256QAM)

| Test number | Bandwidth (MHz) |  |  | Measurement channel |  | Reference value TB success rate across CGs(%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DRB type of Split bearer (Note 2) | DRB type of SCG bearer (Note 3) |  |
|  |  |  |  |  |  |  | MCG | SCG |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC |  |  |  |
| 1 | 2x20 | 20 | 20 | R.68 FDD | R.68-3 TDD | 85 | 85 | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: For the configuration of DRB type of Split bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes across all the CGs used for DC transmission or reception. Note 3: For the configuration of DRB type of SCG bearer, the TB success rate across CGs is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blockes per CG used for DC transmission or reception, separately. |  |  |  |  |  |  |  |  |

Table 8.7.8-5: Test points for sustained data rate (FRC TDD FDD DC 256QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 11, 12 | DL Cat. 13 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | DL Cat. 11, 12 |  |  |  |  |  |
| DC with 2CCs | 2x20 | 20 | 20 | 1 | 1 |  |  |  |  |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |

### 8.7.9	FDD (4 Rx)

The parameters specified in Table 8.7.9-1 are valid for all FDD tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.9-1: Common Test Parameters (FDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Transmission mode |  |  |  | 3 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |  |
| at antenna port |  |  | dBm/15kHz | -85 |  |
| Antenna configuration |  |  | 2 layer CC | 2x2 or 2x4 |  |
|  |  |  | 4 layer CC | 4x4 |  |
| Codebook subset restriction |  |  | 2 layer CC | 10 |  |
|  |  |  | 4 layer CC | 1000 |  |
| Downlin power allocation |  |  | 2 layer CC | = -3dB, = -3dB, = 0dB |  |
|  |  |  | 4 layer CC | = -6dB, = -6dB, = 3dB |  |
| Symbols for unused PRBs |  |  |  | OP.1 FDD |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.9-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.9-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.9-2: Per-CC FRC for SDR test (FDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD |
|  | 10 | R.31-3A FDD |
|  | 15 | R.31-5 FDD |
|  | 20 | R.31-4 FDD |
| 4 layer | 5 | R.31-10 FDD |
|  | 10 | R.31-7 FDD |
|  | 15 | R.31-8 FDD |
|  | 20 | R.31-9 FDD |

Table 8.7.9-3: Per-CC FRC for SDR test (FDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD |
|  | 10 | R.68-2 FDD |
|  | 15 | R.68-1 FDD |
|  | 20 | R.68 FDD |
| 4 layer | 5 | R.68-7 FDD |
|  | 10 | R.68-4 FDD |
|  | 15 | R.68-5 FDD |
|  | 20 | R.68-6 FDD |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

-	The procedure applies also for single carrier using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

Table 8.7.9-4: Void

Table 8.7.9-5: Void

### 8.7.10	TDD (4 Rx)

The parameters specified in Table 8.7.10-1 are valid for all TDD tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.10-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Transmission mode |  |  |  | 3 |  |
| Special subframe configuration (Note 1) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |  |
| at antenna port |  |  | dBm/15kHz | -85 |  |
| Symbols for unused PRBs |  |  |  | OP.1 TDD |  |
| Antenna configuration |  |  | 2 layer CC | 2x2 or 2x4 |  |
|  |  |  | 4 layer CC | 4x4 |  |
| Codebook subset restriction |  |  | 2 layer CC | 10 |  |
|  |  |  | 4 layer CC | 1000 |  |
| Downlin power allocation |  |  | 2 layer CC | = -3dB, = -3dB, = 0dB |  |
|  |  |  | 4 layer CC | = -6dB, = -6dB, = 3dB |  |
| Symbols for unused PRBs |  |  |  | OP.1 TDD |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.10-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.10-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.10-2:  Per-CC FRC for SDR test(TDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.31-6 TDD |
|  | 15 | R.31-5 TDD |
|  | 20 | R.31-4 TDD |
| 4 layer | 10 | R.31-7 TDD |
|  | 15 | R.31-8 TDD |
|  | 20 | R.31-9 TDD |

Table 8.7.10-3: Per-CC FRC for SDR test (TDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.68-2 TDD |
|  | 15 | R.68-1 TDD |
|  | 20 | R.68 TDD |
| 4 layer | 10 | R.68-5 TDD |
|  | 15 | R.68-6 TDD |
|  | 20 | R.68-7 TDD |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

-	The procedure applies also for single carrier using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

Table 8.7.10-4: Void

Table 8.7.10-5: Void

### 8.7.11	TDD FDD CA (4 Rx)

The parameters specified in Table 8.7.11-1 are valid for all TDD FDD CA tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.11-1: Common Test Parameters (TDD FDD CA)

| Parameter | Unit | FDD CC | TDD CC |
| --- | --- | --- | --- |
| Transmission mode |  | 3 | 3 |
| Uplink downlink configuration (Note 1) |  | N/A | 1 |
| Special subframe configuration (Note 2) |  | N/A | 4 |
| Cyclic prefix |  | Normal | Normal |
| Cell ID |  | 0 | 0 |
| Inter-TTI Distance |  | 1 | 1 |
| Number of HARQ processes per component carrier | Processes | 8 | 8 |
| Maximum number of HARQ transmission |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 | 1 |
| Cross carrier scheduling |  | Not configured | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 | 10 |
|  | 4 layer CC | 1000 | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 FDD | OP.1 TDD |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.11-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.11-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.11-2: Per-CC FRC for SDR test (TDD-FDD 64QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD | N/A |
|  | 10 | R.31-3A FDD | R.31-6 TDD |
|  | 15 | R.31-5 FDD | R.31-5 TDD |
|  | 20 | R.31-4 FDD | R.31-4 TDD |
| 4 layer | 5 | R.31-10 FDD | N/A |
|  | 10 | R.31-7 FDD | R.31-7 TDD |
|  | 15 | R.31-8 FDD | R.31-8 TDD |
|  | 20 | R.31-9 FDD | R.31-9 TDD |

Table 8.7.11-3: Per-CC FRC for SDR test (TDD-FDD 256QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD | N/A |
|  | 10 | R.68-2 FDD | R.68-2 TDD |
|  | 15 | R.68-1 FDD | R.68-1 TDD |
|  | 20 | R.68 FDD | R.68 TDD |
| 4 layer | 5 | R.68-7 FDD | N/A |
|  | 10 | R.68-4 FDD | R.68-5 TDD |
|  | 15 | R.68-5 FDD | R.68-6 TDD |
|  | 20 | R.68-6 FDD | R.68-7 TDD |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

#### 8.7.11.1	Void

Table 8.7.11.1-1: Void

Table 8.7.11.1-2: Void

Table 8.7.11.1-3: Void

Table 8.7.11.1-4:Void

### 8.7.12	LAA

#### 8.7.12.1	FDD CA in licensed bands

The parameters specified in Table 8.7.12.1-1 are valid for all LAA CA SDR tests unless otherwise stated.

Table 8.7.12.1-1: Common Test Parameters

| Parameter | Unit | FDD CC | LAA CC |
| --- | --- | --- | --- |
| Transmission mode |  | 3 |  |
| Cyclic prefix |  | Normal |  |
| Cell ID |  | 0 | 0 |
| Inter-TTI Distance |  | 1 |  |
| Number of HARQ processes per component carrier | Processes | 8 |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |  |
| Cross carrier scheduling |  | Not configured |  |
| Propagation condition |  | Static propagation condition (B.1) No external noise sources are applied |  |
| at antenna port | dBm/15kHz | -85 |  |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |  |
|  | 4 layer CC | 4x4 |  |
| Codebook subset restriction | 2 layer CC | 10 |  |
|  | 4 layer CC | 1000 |  |
| Downlink power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |  |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |  |
| Symbols for unused PRBs |  | OP.1 FDD | OP.1 FS3 |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.1-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.1-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

For LAA SCell, per-CC separate FRCs are defined for different UE capability for endingDwPTS and secondSlotStartingPosition.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.12.1-2: Per-CC FRC for SDR test (64QAM)

| MIMO layer | Bandwidth (MHz) | FDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD | N/A | N/A | N/A |
|  | 10 | R.31-3A FDD | N/A | N/A | N/A |
|  | 15 | R.31-5 FDD | N/A | NA | NA |
|  | 20 | R.31-4 FDD | R.6 FS3 | R.7 FS3 | R.5 FS3 |
| 4 layer | 5 | R.31-10 FDD | N/A | N/A | N/A |
|  | 10 | R.31-7 FDD | N/A | N/A | N/A |
|  | 15 | R.31-8 FDD | N/A | N/A | N/A |
|  | 20 | R.31-9 FDD | R.9 FS3 | R.10 FS3 | R.8 FS3 |

Table 8.7.12.1-3: Per-CC FRC for SDR test (256QAM)

| MIMO layer | Bandwidth (MHz) | FDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD | N/A | N/A | N/A |
|  | 10 | R.68-2 FDD | N/A | N/A | N/A |
|  | 15 | R.68-1 FDD | N/A | NA | NA |
|  | 20 | R.68 FDD | R.12 FS3 | R.13 FS3 | R.11 FS3 |
| 4 layer | 5 | R.68-7 FDD | N/A | N/A | N/A |
|  | 10 | R.68-4 FDD | N/A | N/A | N/A |
|  | 15 | R.68-5 FDD | N/A | N/A | N/A |
|  | 20 | R.68-6 FDD | R.15 FS3 | R.16 FS3 | R.14 FS3 |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC following the equation that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

#### 8.7.12.2	TDD CA in licensed bands

The parameters specified in Table 8.7.12.2-1 are valid for all LAA CA SDR tests unless otherwise stated.

Table 8.7.12.2-1: Common Test Parameters

| Parameter | Unit | TDD CC | LAA CC |
| --- | --- | --- | --- |
| Transmission mode |  | 3 |  |
| Uplink downlink configuration (Note 1) |  | 1 | N/A |
| Special subframe configuration (Note 2) |  | 4 | N/A |
| Cyclic prefix |  | Normal |  |
| Cell ID |  | 0 | 0 |
| Inter-TTI Distance |  | 1 |  |
| Number of HARQ processes per component carrier | Processes | 8 |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |  |
| Cross carrier scheduling |  | Not configured |  |
| Propagation condition |  | Static propagation condition (B.1) No external noise sources are applied |  |
| at antenna port | dBm/15kHz | -85 |  |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |  |
|  | 4 layer CC | 4x4 |  |
| Codebook subset restriction | 2 layer CC | 10 |  |
|  | 4 layer CC | 1000 |  |
| Downlink power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |  |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |  |
| Symbols for unused PRBs |  | OP.1 TDD | OP.1 FS3 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.2-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.2-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

For LAA SCell, per-CC separate FRCs are defined for different UE capability for endingDwPTS and secondSlotStartingPosition.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.12.2-2: Per-CC FRC for SDR test (64QAM)

| MIMO layer | Bandwidth (MHz) | FDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | N/A | N/A | N/A | N/A |
|  | 10 | R.31-6 TDD | N/A | N/A | N/A |
|  | 15 | R.31-5 TDD | N/A | NA | NA |
|  | 20 | R.31-4 TDD | R.6 FS3 | R.7 FS3 | R.5 FS3 |
| 4 layer | 5 | N/A | N/A | N/A | N/A |
|  | 10 | R.31-7 TDD | N/A | N/A | N/A |
|  | 15 | R.31-8 TDD | N/A | N/A | N/A |
|  | 20 | R.31-9 TDD | R.9 FS3 | R.10 FS3 | R.8 FS3 |

Table 8.7.12.2-3: Per-CC FRC for SDR test (256QAM)

| MIMO layer | Bandwidth (MHz) | FDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | N/A | N/A | N/A | N/A |
|  | 10 | R.68-2 TDD | N/A | N/A | N/A |
|  | 15 | R.68-1 TDD | N/A | NA | NA |
|  | 20 | R.68 TDD | R.12 FS3 | R.13 FS3 | R.11 FS3 |
| 4 layer | 5 | N/A | N/A | N/A | N/A |
|  | 10 | R.68-4 TDD | N/A | N/A | N/A |
|  | 15 | R.68-5 TDD | N/A | N/A | N/A |
|  | 20 | R.68-6 TDD | R.15 FS3 | R.16 FS3 | R.14 FS3 |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC following the equation that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

#### 8.7.12.3	TDD-FDD CA in licensed bands

The parameters specified in Table 8.7.12.3-1 are valid for all LAA CA SDR tests unless otherwise stated.

Table 8.7.12.3-1: Common Test Parameters

| Parameter | Unit | FDD CC | TDD CC | LAA CC |
| --- | --- | --- | --- | --- |
| Transmission mode |  | 3 |  |  |
| Uplink downlink configuration (Note 1) |  | N/A | 1 | N/A |
| Special subframe configuration (Note 2) |  | N/A | 4 | N/A |
| Cyclic prefix |  | Normal |  |  |
| Cell ID |  | 0 | 0 | 0 |
| Inter-TTI Distance |  | 1 |  |  |
| Number of HARQ processes per component carrier | Processes | 8 |  |  |
| Maximum number of HARQ transmission |  | 4 |  |  |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |  |  |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |  |  |
| Cross carrier scheduling |  | Not configured |  |  |
| Propagation condition |  | Static propagation condition (B.1) No external noise sources are applied |  |  |
| at antenna port | dBm/15kHz | -85 |  |  |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |  |  |
|  | 4 layer CC | 4x4 |  |  |
| Codebook subset restriction | 2 layer CC | 10 |  |  |
|  | 4 layer CC | 1000 |  |  |
| Downlink power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |  |  |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |  |  |
| Symbols for unused PRBs |  | OP.1 FDD | OP.1 TDD | OP.1 FS3 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.3-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.12.3-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

For LAA SCell, per-CC separate FRCs are defined for different UE capability for endingDwPTS and secondSlotStartingPosition.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.12.3-2: Per-CC FRC for SDR test (64QAM)

| MIMO layer | Bandwidth (MHz) | FDD | TDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD | N/A | N/A | N/A | N/A |
|  | 10 | R.31-3A FDD | R.31-6 TDD | N/A | N/A | N/A |
|  | 15 | R.31-5 FDD | R.31-5 TDD | N/A | NA | NA |
|  | 20 | R.31-4 FDD | R.31-4 TDD | R.6 FS3 | R.7 FS3 | R.5 FS3 |
| 4 layer | 5 | R.31-10 FDD | N/A | N/A | N/A | N/A |
|  | 10 | R.31-7 FDD | R.31-7 TDD | N/A | N/A | N/A |
|  | 15 | R.31-8 FDD | R.31-8 TDD | N/A | N/A | N/A |
|  | 20 | R.31-9 FDD | R.31-9 TDD | R.9 FS3 | R.10 FS3 | R.8 FS3 |

Table 8.7.12.3-3: Per-CC FRC for SDR test (256QAM)

| MIMO layer | Bandwidth (MHz) | FDD | TDD | LAA supporting end partial SF | LAA supporting initial partial SF but not supporting end partial SF | LAA not supporting both initial and end partial SF |
| --- | --- | --- | --- | --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD | N/A | N/A | N/A | N/A |
|  | 10 | R.68-2 FDD | R.68-2 TDD | N/A | N/A | N/A |
|  | 15 | R.68-1 FDD | R.68-1 TDD | N/A | NA | NA |
|  | 20 | R.68 FDD | R.68 TDD | R.12 FS3 | R.13 FS3 | R.11 FS3 |
| 4 layer | 5 | R.68-7 FDD | N/A | N/A | N/A | N/A |
|  | 10 | R.68-4 FDD | R.68-4 TDD | N/A | N/A | N/A |
|  | 15 | R.68-5 FDD | R.68-5 TDD | N/A | N/A | N/A |
|  | 20 | R.68-6 FDD | R.68-6 TDD | R.15 FS3 | R.16 FS3 | R.14 FS3 |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one CA bandwidth combination among all supported CA configurations with bandwidth combination and MIMO layer on each CC following the equation that leads to largest equivalent aggregated bandwidth among all CA bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {CA configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

### 8.7.13	FDD DC (4 Rx)

The parameters specified in Table 8.7.13-1 are valid for all FDD tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.13-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission mode |  | 3 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 |
|  | 4 layer CC | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 FDD |
| ACK/NACK feedback mode |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |
| Time offset between MCG CC and SCG CC | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |
| Note 1: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 2: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.13-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.13-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate across CGs shall be sustained during at least 300 frames.

Table 8.7.13-2: Per-CC FRC for SDR test (FDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD |
|  | 10 | R.31-3A FDD |
|  | 15 | R.31-5 FDD |
|  | 20 | R.31-4 FDD |
| 4 layer | 5 | R.31-10 FDD |
|  | 10 | R.31-7 FDD |
|  | 15 | R.31-8 FDD |
|  | 20 | R.31-9 FDD |

Table 8.7.13-3: Per-CC FRC for SDR test (FDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD |
|  | 10 | R.68-2 FDD |
|  | 15 | R.68-1 FDD |
|  | 20 | R.68 FDD |
| 4 layer | 5 | R.68-7 FDD |
|  | 10 | R.68-4 FDD |
|  | 15 | R.68-5 FDD |
|  | 20 | R.68-6 FDD |

DC configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one DC bandwidth combination among all supported DC configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all DC bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {DC configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

### 8.7.14	TDD DC (4 Rx)

The parameters specified in Table 8.7.14-1 are valid for all TDD tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.14-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission mode |  | 3 |
| Special subframe configuration |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 |
|  | 4 layer CC | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 TDD |
| ACK/NACK feedback mode |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |
| Time offset between MCG CC and SCG CC | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |
| Note 1: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 2: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.14-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.14-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate across CGs shall be sustained during at least 300 frames.

Table 8.7.14-2: Per-CC FRC for SDR test (TDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.31-6 TDD |
|  | 15 | R.31-5 TDD |
|  | 20 | R.31-4 TDD |
| 4 layer | 10 | R.31-7 TDD |
|  | 15 | R.31-8 TDD |
|  | 20 | R.31-9 TDD |

Table 8.7.14-3: Per-CC FRC for SDR test (TDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.68-2 TDD |
|  | 15 | R.68-1 TDD |
|  | 20 | R.68 TDD |
| 4 layer | 10 | R.68-5 TDD |
|  | 15 | R.68-6 TDD |
|  | 20 | R.68-7 TDD |

DC configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one DC bandwidth combination among all supported DC configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all DC bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {DC configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

### 8.7.15	TDD FDD DC (4 Rx)

The parameters specified in Table 8.7.15-1 are valid for all TDD FDD tests for 4Rx capable UEs unless otherwise stated.

Table 8.7.15-1: Common Test Parameters (TDD FDD)

| Parameter | Unit | FDD CC | TDD CC |
| --- | --- | --- | --- |
| Transmission mode |  | 3 | 3 |
| Uplink downlink configuration (Note 1) |  | N/A | 1 |
| Special subframe configuration (Note 2) |  | N/A | 4 |
| Cyclic prefix |  | Normal | Normal |
| Cell ID |  | 0 | 0 |
| Inter-TTI Distance |  | 1 | 1 |
| Number of HARQ processes per component carrier | Processes | 8 | 8 |
| Maximum number of HARQ transmission |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM and 256QAM | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 | 1 |
| Cross carrier scheduling |  | Not configured | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 | 10 |
|  | 4 layer CC | 1000 | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 FDD | OP.1 TDD |
| ACK/NACK feedback mode |  | Separate ACK/NACK feedbacks with PUCCH format 3 on the MCG and SCG |  |
| Time offset between MCG CC and SCG CC | μs | 0 for UE under test supporting synchronous dual connectivity; 500 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 1) |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: Asynchronous and synchrous dual connectivity are defined in TS36.300 [11]. Note 4: If the UE supports both SCG bearer and Split bearer, the Split bearer is configured. |  |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.15-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.15-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate across CGs shall be sustained during at least 300 frames.

Table 8.7.15-2: Per-CC FRC for SDR test (TDD-FDD 64QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD | N/A |
|  | 10 | R.31-3A FDD | R.31-6 TDD |
|  | 15 | R.31-5 FDD | R.31-5 TDD |
|  | 20 | R.31-4 FDD | R.31-4 TDD |
| 4 layer | 5 | R.31-10 FDD | N/A |
|  | 10 | R.31-7 FDD | R.31-7 TDD |
|  | 15 | R.31-8 FDD | R.31-8 TDD |
|  | 20 | R.31-9 FDD | R.31-9 TDD |

Table 8.7.15-3: Per-CC FRC for SDR test (TDD-FDD 256QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD | N/A |
|  | 10 | R.68-2 FDD | R.68-2 TDD |
|  | 15 | R.68-1 FDD | R.68-1 TDD |
|  | 20 | R.68 FDD | R.68 TDD |
| 4 layer | 5 | R.68-7 FDD | N/A |
|  | 10 | R.68-4 FDD | R.68-5 TDD |
|  | 15 | R.68-5 FDD | R.68-6 TDD |
|  | 20 | R.68-6 FDD | R.68-7 TDD |

DC configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

-	Select one DC bandwidth combination among all supported DC configurations with bandwidth combination and MIMO layer on each CC that leads to largest equivalent aggregated bandwidth among all DC bandwidth combinations supported by UE. Equivalent aggregated bandwidth is defined as

where  is number of CCs,  and is MIMO layer  and bandwidth of CC .

-	When there are multiple sets of {DC configuration, bandwidth combination, MIMO layer} with same largest aggregated bandwidth, select one among sets with largest number of 4 layer CCs.

### 8.7.16	FDD (1024QAM and up to 4Rx supported)

The common parameters are specified in Table 8.7.16-1 for UE which is capable of supporting 1024QAM.

Table 8.7.16-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission mode |  | 3 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM, 256QAM and 1024QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 |
|  | 4 layer CC | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 FDD |

The UE capability for 1024QAM is signalled per band or per band combination, hence the SDR tests with the mixed modulation orders and the mixed receiver antenna numbers across CC-s are specified.

For UE not supporting CA and supporting 1024QAM, the TB success rate shall be higher than 85% when PDSCH is scheduled with FRC in Table 8.7.16-4 according the UE capability. The maximum supported channel bandwidth and MIMO layer are configured during the test.

For UE supporting CA and supporting 1024QAM at least on one CC for a supported CA configuration, the SDR requirements are specified as follows:

-	If UE is capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.16-3 for the transmission with 256QAM and Table 8.7.16-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

-	If UE is not capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.16-2 for the transmission with 64QAM and Table 8.7.16-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

For UE supporting 1024 QAM, the SDR requirement with 64QAM and 256QAM only is not applicable.

The CA configuration or band for single carrier, bandwidth combination or bandwidth for single carrier, modulation order on each CC and MIMO layer on each CC are determined by the following procedure.

-	Among all the supported CA configurations which support 1024QAM at least on one CC, select one set of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC}, which leads to the largest equivalent aggregated bandwidth. The equivalent aggregated bandwidth is defined as

Where N is the number of CCs,  represents the MIMO layer,  represents the bandwidths on each CC, [ is the scaling factor according to the supported modulation order on each CC, where Mi = 0.75 is used if the maximum modulation order of CC i is 64QAM, Mi = 1 is used if the maximum modulation order of CC i is 256QAM, and Mi = 1.25 is used if the maximum modulation order of CC i is 1024QAM.]

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth, select one among the sets with the largest number of CCs supporting 1024QAM.

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth with the same number of CCs supporting 1024QAM, select one among the sets with the largest number of CCs supporting 4 layer.

-	The procedure applies also for the single carrier operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

The TB success rate for single carrier or on each CC for CA is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.16-2: Per-CC FRC for SDR test (FDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD |
|  | 10 | R.31-3A FDD |
|  | 15 | R.31-5 FDD |
|  | 20 | R.31-4 FDD |
| 4 layer | 5 | R.31-10 FDD |
|  | 10 | R.31-7 FDD |
|  | 15 | R.31-8 FDD |
|  | 20 | R.31-9 FDD |

Table 8.7.16-3: Per-CC FRC for SDR test (FDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD |
|  | 10 | R.68-2 FDD |
|  | 15 | R.68-1 FDD |
|  | 20 | R.68 FDD |
| 4 layer | 5 | R.68-7 FDD |
|  | 10 | R.68-4 FDD |
|  | 15 | R.68-5 FDD |
|  | 20 | R.68-6 FDD |

Table 8.7.16-4: Per-CC FRC for SDR test (FDD 1024QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 5 | R.105-3 FDD |
|  | 10 | R.105-2 FDD |
|  | 15 | R.105-1 FDD |
|  | 20 | R.105 FDD |
| 4 layer | 5 | R.105-7 FDD |
|  | 10 | R.105-6 FDD |
|  | 15 | R.105-5 FDD |
|  | 20 | R.105-4 FDD |

### 8.7.17	TDD (1024QAM and up to 4 Rx supported)

The common parameters are specified in Table 8.7.17-1 for UE which is capable of supporting 1024QAM.

Table 8.7.17-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission mode |  | 3 |
| Uplink downlink configuration |  | 1 |
| Special subframe configuration (Note 1) |  | 4 |
| Cyclic prefix |  | Normal |
| Cell ID |  | 0 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM, 256QAM, and 1024QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 |
| Symbols for unused PRBs |  | OP.1 TDD |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 |
|  | 4 layer CC | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 TDD |

The UE capability for 1024QAM is signalled per band or per band combination, hence the SDR tests with the mixed modulation orders and the mixed receiver antenna numbers across CC-s are specified.

For UE not supporting CA and supporting 1024QAM, the TB success rate shall be higher than 85% when PDSCH is scheduled with FRC in Table 8.7.17-4 according the UE capability. The maximum supported channel bandwidth and MIMO layer are configured during the test.

For UE supporting CA and supporting 1024QAM at least on one CC for a supported CA configuration, the SDR requirements are specified as follows:

-	If UE is capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.17-3 for the transmission with 256QAM and Table 8.7.17-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

-	If UE is not capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.17-2 for the transmission with 64QAM and Table 8.7.17-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

For UE supporting 1024 QAM, the SDR requirement with 64QAM and 256QAM only is not applicable.

The CA configuration or band for single carrier, bandwidth combination or bandwidth for single carrier, modulation order on each CC and MIMO layer on each CC are determined by the following procedure.

-	Among all the supported CA configurations which support 1024QAM at least on one CC, select one set of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC}, which leads to the largest equivalent aggregated bandwidth. The equivalent aggregated bandwidth is defined as

Where N is the number of CCs,  represents the MIMO layer,  represents the bandwidths on each CC, [ is the scaling factor according to the supported modulation order on each CC, where Mi = 0.75 is used if the maximum modulation order of CC i is 64QAM, Mi = 1 is used if the maximum modulation order of CC i is 256QAM, and Mi = 1.25 is used if the maximum modulation order of CC i is 1024QAM.]

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth, select one among the sets with the largest number of CCs supporting 1024QAM.

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth with the same number of CCs supporting 1024QAM, select one among the sets with the largest number of CCs supporting 4 layer.

-	The procedure applies also for the single carrier operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

The TB success rate for single carrier or on each CC for CA is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.17-2: Per-CC FRC for SDR test(TDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.31-6 TDD |
|  | 15 | R.31-5 TDD |
|  | 20 | R.31-4 TDD |
| 4 layer | 10 | R.31-7 TDD |
|  | 15 | R.31-8 TDD |
|  | 20 | R.31-9 TDD |

Table 8.7.17-3: Per-CC FRC for SDR test (TDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.68-2 TDD |
|  | 15 | R.68-1 TDD |
|  | 20 | R.68 TDD |
| 4 layer | 10 | R.68-5 TDD |
|  | 15 | R.68-6 TDD |
|  | 20 | R.68-7 TDD |

Table 8.7.17-4: Per-CC FRC for SDR test (TDD 1024QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.105-2 TDD |
|  | 15 | R.105-1 TDD |
|  | 20 | R.105 TDD |
| 4 layer | 10 | R.105-5 TDD |
|  | 15 | R.105-4 TDD |
|  | 20 | R.105-3 TDD |

### 8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)

The common parameters are specified in Table 8.7.18-1 for UE which is capable of supporting 1024QAM.

Table 8.7.18-1: Common Test Parameters (TDD FDD CA)

| Parameter | Unit | FDD CC | TDD CC |
| --- | --- | --- | --- |
| Transmission mode |  | 3 | 3 |
| Uplink downlink configuration (Note 1) |  | N/A | 1 |
| Special subframe configuration (Note 2) |  | N/A | 4 |
| Cyclic prefix |  | Normal | Normal |
| Cell ID |  | 0 | 0 |
| Inter-TTI Distance |  | 1 | 1 |
| Number of HARQ processes per component carrier | Processes | 8 | 7 |
| Maximum number of HARQ transmission |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,0,1,2} for 64QAM, 256QAM and 1024QAM | {0,0,1,2} for 64QAM, 256QAM, and 1024QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 | 1 |
| Cross carrier scheduling |  | Not configured | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 | 10 |
|  | 4 layer CC | 1000 | 1000 |
| Downlin power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB | = -6dB, = -6dB, = 3dB |
| Symbols for unused PRBs |  | OP.1 FDD | OP.1 TDD |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

The UE capability for 1024QAM is signalled per band or per band combination, hence the SDR tests with the mixed modulation orders and the mixed receiver antenna numbers across CC-s are specified.

For UE not supporting CA and supporting 1024QAM, the TB success rate shall be higher than 85% when PDSCH is scheduled with FRC in Table 8.7.18-4 according the UE capability. The maximum supported channel bandwidth and MIMO layer are configured during the test.

For UE supporting CA and supporting 1024QAM at least on one CC for a supported CA configuration, the SDR requirements are specified as follows:

-	If UE is capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.18-3 for the transmission with 256QAM and Table 8.7.18-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

-	If UE is not capable of supporting 256QAM, the TB success rate shall be higher than 85% on each CC when PDSCH-s are scheduled with FRC-s in Table 8.7.18-2 for the transmission with 64QAM and Table 8.7.18-4 for the transmission with 1024QAM according to the reported capability of supported modulation order for the determined CA configuration.

For UE supporting 1024 QAM, the SDR requirement with 64QAM and 256QAM only is not applicable.

The CA configuration or band for single carrier, bandwidth combination or bandwidth for single carrier, modulation order on each CC and MIMO layer on each CC are determined by the following procedure.

-	Among all the supported CA configurations which support 1024QAM at least on one CC, select one set of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC}, which leads to the largest equivalent aggregated bandwidth. The equivalent aggregated bandwidth is defined as

Where N is the number of CCs,  represents the MIMO layer,  represents the bandwidths on each FDD CC and  represents the bandwidths on each FDD CC, [ is the scaling factor according to the supported modulation order on each CC, where Mi = 0.75 is used if the maximum modulation order of CC i is 64QAM, Mi = 1 is used if the maximum modulation order of CC i is 256QAM, and Mi = 1.25 is used if the maximum modulation order of CC i is 1024QAM.]

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth, select one among the sets with the largest number of CCs supporting 1024QAM.

-	When there are multiple sets of {CA configuration or a band, bandwidth combination or bandwidth, modulation order on each CC, MIMO layer on each CC} which can reach the same equivalent aggregated bandwidth with the same number of CCs supporting 1024QAM, select one among the sets with the largest number of CCs supporting 4 layer.

-	The procedure applies also for the single carrier operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

The TB success rate for single carrier or on each CC for CA is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.18-2: Per-CC FRC for SDR test (TDD-FDD 64QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.31-6 FDD | N/A |
|  | 10 | R.31-3A FDD | R.31-6 TDD |
|  | 15 | R.31-5 FDD | R.31-5 TDD |
|  | 20 | R.31-4 FDD | R.31-4 TDD |
| 4 layer | 5 | R.31-10 FDD | N/A |
|  | 10 | R.31-7 FDD | R.31-7 TDD |
|  | 15 | R.31-8 FDD | R.31-8 TDD |
|  | 20 | R.31-9 FDD | R.31-9 TDD |

Table 8.7.18-3: Per-CC FRC for SDR test (TDD-FDD 256QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.68-3 FDD | N/A |
|  | 10 | R.68-2 FDD | R.68-2 TDD |
|  | 15 | R.68-1 FDD | R.68-1 TDD |
|  | 20 | R.68 FDD | R.68 TDD |
| 4 layer | 5 | R.68-7 FDD | N/A |
|  | 10 | R.68-4 FDD | R.68-5 TDD |
|  | 15 | R.68-5 FDD | R.68-6 TDD |
|  | 20 | R.68-6 FDD | R.68-7 TDD |

Table 8.7.18-4: Per-CC FRC for SDR test (TDD-FDD 1024QAM)

| MIMO layer | Bandwidth | FDD Reference channel | TDD Reference channel |
| --- | --- | --- | --- |
| 2 layer | 5 | R.105-3 FDD | N/A |
|  | 10 | R.105-2 FDD | R.105-2 TDD |
|  | 15 | R.105-1 FDD | R.105-1 TDD |
|  | 20 | R.105 FDD | R.105 TDD |
| 4 layer | 5 | R.105-7 FDD | N/A |
|  | 10 | R.105-6 FDD | R.105-5 TDD |
|  | 15 | R.105-5 FDD | R.105-4 TDD |
|  | 20 | R.105-4 FDD | R.105-3 TDD |

### 8.7.19	TDD (8 Rx)

The parameters specified in Table 8.7.19-1 are valid for all TDD tests for 8Rx capable UEs unless otherwise stated. For 2/4 layer carrier configurations, please refer to Table 8.7.10-1. 8 layer carier configuration is specified in Table 8.7.19-1.

Table 8.7.19-1: Common Test Parameters for 8 Layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3(Note 3, 4) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OP.1 TDD |
| Cyclic prefix |  |  | Normal |
| Cell ID |  |  | 0 |
| Inter-TTI Distance |  |  | 1 |
| Number of HARQ processes per component carrier |  | Processes | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbols | 1 |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 10 or 11 |
| Reporting interval |  |  | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0010 0000 0000 0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 3: The precoder in clause B.4.3 follows UE recommended PMI. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

For UE not supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.19-2 with the downlink physical channel setup according to Annex C.3.2.

For UE supporting 256QAM, the TB success rate shall be higher than 85% when PDSCH are scheduled with FRC in Table 8.7.19-3 with the downlink physical channel setup according to Annex C.3.2. For UE supporting 256QAM, the requirement with 64QAM is not applicable.

The TB success rate is defined as 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.19-2: Per-CC FRC for SDR test (TDD 64QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.31-6 TDD |
|  | 15 | R.31-5 TDD |
|  | 20 | R.31-4 TDD |
| 4 layer | 10 | R.31-7 TDD |
|  | 15 | R.31-8 TDD |
|  | 20 | R.31-9 TDD |
| 8 layer | 10 | R.31-10 TDD |
|  | 15 | R.31-11 TDD |
|  | 20 | R.31-12 TDD |

Table 8.7.19-3: Per-CC FRC for SDR test (TDD 256QAM)

| MIMO layer | Bandwidth | Reference channel |
| --- | --- | --- |
| 2 layer | 10 | R.68-2 TDD |
|  | 15 | R.68-1 TDD |
|  | 20 | R.68 TDD |
| 4 layer | 10 | R.68-5 TDD |
|  | 15 | R.68-6 TDD |
|  | 20 | R.68-7 TDD |
| 8 layer | 10 | R.68-8 TDD |
|  | 15 | R.68-9 TDD |
|  | 20 | R.68-10 TDD |

CA configuration, bandwidth combination and MIMO layer on each CC is determined by following procedure.

- Select the set(s) of {CA configuration, bandwidth combination, MIMO layer} among all the supported CA configurations that leads to the largest equivalent aggregated bandwidth which does not cause the transport block bits within a TTI to exceed the capability of the category of UE under test when the defined reference channel applies on each CC. The equivalent aggregated bandwidth is defined as

Where  is the number of CCs,  Ri∈2,4, 8and Bi∈5,10,15,20 are MIMO layer and bandwidth of CC . And  for and  for

-	The procedure applies also for single carrier using operating band instead of CA configuration, and bandwidth instead of bandwidth combination.

## 8.8	Demodulation of EPDCCH

The receiver characteristics of the EPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). For the distributed transmission tests in 8.8.1, EPDCCH and PCFICH are tested jointly, i.e. a miss detection of PCFICH implies a miss detection of EPDCCH. For other tests, EPDCCH and PCFICH are not tested jointly.

### 8.8.1	Distributed Transmission

#### 8.8.1.1	FDD

The parameters specified in Table 8.8.1.1-1 are valid for all FDD distributed EPDCCH tests unless otherwise stated.

Table 8.8.1.1-1: Test Parameters for Distributed EPDCCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B. 4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of EPDCCH Sets Configured |  |  | 2 (Note 2) |
| Number of PRB per EPDCCH Set |  |  | 4 (1st Set) 8 (2nd Set) |
| EPDCCH Subframe Monitoring |  |  | NA |
| PDSCH TM |  |  | TM3 |
| DCI Format |  |  | 2A |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: The two sets are distributed EPDCCH sets and non-overlapping with PRB = {3, 17, 31, 45} for the first set and PRB = {0, 7, 14, 21, 28, 35, 42, 49} for the second set. EPDCCH is scheduled in the first set for Test 1 and second set for Test 2, respectively. Both sets are always configured. |  |  |  |

For the parameters specified in Table 8.8.1.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.1.1-2. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.1.1-2: Minimum performance Distributed EPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | 2.60 |
| 2 | 10 MHZ | 16 ECCE | R.56 FDD | OP.7 FDD | EVA70 | 2 x 2 Low | 1 | -3.20 |

##### 8.8.1.1.1	Void

Table 8.8.1.1.1-1: Void

#### 8.8.1.2	TDD

The parameters specified in Table 8.8.1.2-1 are valid for all TDD distributed EPDCCH tests unless otherwise stated.

Table 8.8.1.2-1: Test Parameters for Distributed EPDCCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B. 4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of EPDCCH Sets Configured |  |  | 2 (Note 2) |
| Number of PRB per EPDCCH Set |  |  | 4 (1st Set) 8 (2nd Set) |
| EPDCCH Subframe Monitoring |  |  | NA |
| PDSCH TM |  |  | TM3 |
| DCI Format |  |  | 2A |
| TDD UL/DL Configuration |  |  | 0 |
| TDD Special Subframe |  |  | 1 (Note 3) |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: The two sets are distributed EPDCCH sets and non-overlapping with PRB = {3, 17, 31, 45} for the first set and PRB = {0, 7, 14, 21, 28, 35, 42, 49} for the second set. EPDCCH is scheduled in the first set for Test 1 and second set for Test 2, respectively. Both sets are always configured. Note 3: Demodulation performance is averaged over normal and special subframe. |  |  |  |

For the parameters specified in Table 8.8.1.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.1.2-2. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.1.2-2: Minimum performance Distributed EPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55 TDD | OP.7 TDD | EVA5 | 2 x 2 Low | 1 | 2.80 |
| 2 | 10 MHZ | 16 ECCE | R.56 TDD | OP.7 TDD | EVA70 | 2 x 2 Low | 1 | -3.10 |

##### 8.8.1.2.1	Void

Table 8.8.1.2.1-1: Void

### 8.8.2	Localized Transmission with TM9

#### 8.8.2.1	FDD

The parameters specified in Table 8.8.2.1-1 are valid for all FDD TM9 localized ePDCCH tests unless otherwise stated.

Table 8.8.2.1-1: Test Parameters for Localized EPDCCH with TM9

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1111111110 1111111101 1111111011 1111110111 (Note 3) |
| PDSCH TM |  |  | TM9 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. |  |  |  |

For the parameters specified in Table 8.8.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.2.1-2. EPDCCH subframe monitoring is configured and the subframe monitoring requirement in EPDCCH restricted subframes is statDTX of 99.9%.

The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.2.1-2: Minimum performance Localized EPDCCH with TM9

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | 12.2 |
| 2 | 10 MHZ | 8 ECCE | R.58 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | 2.5 |

##### 8.8.2.1.1	Void

Table 8.8.2.1.1-1: Void

##### 8.8.2.1.2	Void

Table 8.8.2.1.2-1: Void

Table 8.8.2.1.2-2: Void

Table 8.8.2.1.2-3: Void

#### 8.8.2.2	TDD

The parameters specified in Table 8.8.2.2-1 are valid for all TDD TM9 localized ePDCCH tests unless otherwise stated.

Table 8.8.2.2-1: Test Parameters for Localized EPDCCH with TM9

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 0 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 0 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1100011000 1100010000 1100011000 1100001000 1100011000 1000011000 1100011000 (Note 3) |
| PDSCH TM |  |  | TM9 |
| TDD UL/DL Configuration |  |  | 0 |
| TDD Special Subframe |  |  | 1 (Note 4) |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. Note 4: Demodulation performance is averaged over normal and special subframe. |  |  |  |

For the parameters specified in Table 8.8.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.2-2. EPDCCH subframe monitoring is configured and the subframe monitoring requirement in EPDCCH restricted subframes is statDTX of 99.9%.

The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.2.2-2: Minimum performance Localized EPDCCH with TM9

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 TDD | OP.7 TDD | EVA5 | 2 x 2 Low | 1 | 12.8 |
| 2 | 10 MHZ | 8 ECCE | R.58 TDD | OP.7 TDD | EVA5 | 2 x 2 Low | 1 | 2.0 |

##### 8.8.2.2.1	Void

Table 8.8.2.2.1-1: Void

##### 8.8.2.2.2	Void

Table 8.8.2.2.2-1: Void

Table 8.8.2.2.2-2: Void

Table 8.8.2.2.2-3: Void

### 8.8.3	Localized transmission with TM10 Type B quasi co-location type

#### 8.8.3.1	FDD

For the parameters specified in Table 8.8.3.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified values in Table 8.8.3.1-2. In Table 8.8.3.1-1, transmission point 1 (TP 1) is the serving cell. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.3.1-1: Test Parameters for Localized Transmission TM10 Type B quasi co-location type

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |
| PHICH duration |  |  |  | Normal |  |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | -3 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0dB power imbalance is considered between TP 1 and TP 2, | Reference value in Table 8.8.3.1-2 | Reference value in Table 8.8.3.1-2 | Reference value in Table 8.8.3.1-2 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |  |
| Bandwidth |  |  | MHz | 10 | 10 | 10 | 10 |
| Number of configured EPDCCH Sets |  |  |  | 2 (Note 1) |  | 2 (Note1) |  |
| EPDCCH-PRB-Set ID (setConfigId) |  |  |  | 0 | 1 | 0 | 1 |
| Transmission type of EPDCCH-PRB-set |  |  |  | Localized | Localized | Localized | Localized |
| Number of PRB pair per EPDCCH-PRB-set |  |  | PRB | 8 | 8 | 8 | 8 |
| EPDCCH beamforming model |  |  |  | Annex B.4.5 | Annex B.4.5 | Annex B.4.5 | Annex B.4.5 |
| PDSCH transmission mode |  |  |  | TM10 | TM10 | TM10 | TM10 |
| PDSCH transmission scheduling |  |  |  | Blanked in all the subframes | Transmit in all the subframes | Probability of occurrence of PDSCH transmission is 30% (Note 3) | Probability of occurrence of PDSCH transmission is 70% (Note 3) |
| Non-zero power CSI reference signal (NZPId=1) | CSI reference signal configuration |  |  | N/A | 0 | N/A | 0 |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | N/A | 2 | N/A | 2 |
| Non-zero power CSI reference signal (NZPId=2) | CSI reference signal configuration |  |  | N/A | N/A | 10 | N/A |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | N/A | N/A | 2 | N/A |
| Zero power CSI reference signal (ZPId=1) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | 0000010000000000 | N/A | 1000010000000000 |
|  | CSI-RS subframe configuration ICSI-RS |  |  | N/A | 2 | N/A | 2 |
| Zero power CSI reference signal (ZPId=2) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | N/A | 1000010000000000 | N/A |
|  | CSI-RS subframe configuration ICSI-RS |  |  | N/A | N/A | 2 | N/A |
| PQI set 0 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | N/A | 1 | N/A | 1 |
|  | Zero power CSI RS Identity (ZPId) |  |  | N/A | 1 | N/A | 1 |
| PQI set 1 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | N/A | N/A | 2 | N/A |
|  | Zero power CSI RS Identity (ZPId) |  |  | N/A | N/A | 2 | N/A |
| Number of PDCCH symbols |  |  | Symbols | 1 (Note 2) |  |  |  |
| EPDCCH starting position |  |  |  | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) |
| Subframe configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time offset between TPs |  |  | s | N/A | 2 | N/A | 2 |
| Frequency shift between TPs |  |  | Hz | N/A | 200 | N/A | 200 |
| Cell ID |  |  |  | 0 | 126 | 0 | 126 |
| Note 1: Resource blocks nPRB =0, 7, 14, 21, 28, 35, 42, 49 are allocated for both the first set and the second set. Note 2: The starting OFDM symbol for EPDCCH is determined from the higher layer signalling pdsch-Start-r11. And CFI is set to 1. Note 3: The TP from which PDSCH is transmitted shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TP 1 and TP 2 are specified. Note 4: For PQI set 0, PDSCH and EPDCCH are transmitted from TP 2. For PQI set 1, PDSCH and EPDCCH are transmitted from TP1. EPDCCH and PDSCH are transmitted from same TP. |  |  |  |  |  |  |  |

Table 8.8.3.1-2: Minimum Performance

| Test number | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 2 ECCE | R.59 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | 13.4 |
| 2 | 2 ECCE | R.59 FDD | OP.7 FDD | EVA5 | 2 x 2 Low | 1 | 13.4 |

#### 8.8.3.2	TDD

For the parameters specified in Table 8.8.3.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified values in Table 8.8.3.2-2. In Table 8.8.3.2-1, transmission point 1 (TP1) is the serving cell. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.8.3.2-1: Test Parameters for Localized Transmission TM10 Type B quasi co-location type

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |
| PHICH duration |  |  |  | Normal |  |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | -3 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0dB power imbalance is considered between TP 1 and TP 2, | Reference value in Table 8.8.3.2-2 | Reference value in Table 8.8.3.2-2 | Reference value in Table 8.8.3.2-2 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |  |
| Bandwidth |  |  | MHz | 10 | 10 | 10 | 10 |
| Number of EPDCCH Sets |  |  |  | 2 (Note 1) |  | 2 (Note1) |  |
| EPDCCH-PRB-Set ID (setConfigId) |  |  |  | 0 | 1 | 0 | 1 |
| Transmission type of EPDCCH-PRB-set |  |  |  | Localized | Localized | Localized | Localized |
| Number of PRB pair per EPDCCH-PRB-set |  |  | PRB | 8 | 8 | 8 | 8 |
| EPDCCH beamforming model |  |  |  | Annex B.4.5 | Annex B.4.5 | Annex B.4.5 | Annex B.4.5 |
| PDSCH transmission mode |  |  |  | TM10 | TM10 | TM10 | TM10 |
| PDSCH transmission scheduling |  |  |  | Blanked in all the subframes | Transmit in all the subframes | Probability of occurrence of PDSCH transmission is 30% (Note 3) | Probability of occurrence of PDSCH transmission is 70% (Note 3) |
| CSI reference signal configurations |  |  |  | Antenna ports 15,16 | Antenna ports 15,16 | Antenna ports 15,16 | Antenna ports 15,16 |
| Non-zero power CSI reference signal (NZPId=1) | CSI reference signal configuration |  |  | N/A | 0 | N/A | 0 |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | N/A | 0 | N/A | 0 |
| Non-zero power CSI reference signal (NZPId=2) | CSI reference signal configuration |  |  | N/A | N/A | 10 | N/A |
|  | CSI reference signal subframe configuration ICSI-RS |  |  | N/A | N/A | 0 | N/A |
| Zero power CSI reference signal (ZPId=1) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | 0000010000000000 | N/A | 1000010000000000 |
|  | CSI-RS subframe configuration ICSI-RS |  |  | N/A | 0 | N/A | 0 |
| Zero power CSI reference signal (ZPId=2) | CSI-RS Configuration list (ZeroPowerCSI-RS bitmap) |  | Bitmap | N/A | N/A | 1000010000000000 | N/A |
|  | CSI-RS subframe configuration ICSI-RS |  |  | N/A | N/A | 0 | N/A |
| PQI set 0 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | N/A | 1 | N/A | 1 |
|  | Zero power CSI RS Identity (ZPId) |  |  | N/A | 1 | N/A | 1 |
| PQI set 1 (Note 4) | Non-Zero power CSI RS Identity (NZPId) |  |  | N/A | N/A | 2 | N/A |
|  | Zero power CSI RS Identity (ZPId) |  |  | N/A | N/A | 2 | N/A |
| Number of PDCCH symbols |  |  | Symbols | 1 (Note 2) |  |  |  |
| EPDCCH starting position |  |  |  | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) | pdsch-Start-r11=2 (Note 2) |
| Subframe configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time offset between TPs |  |  | s | N/A | 2 | N/A | 2 |
| Frequency shift between TPs |  |  | Hz | N/A | 200 | N/A | 200 |
| Cell ID |  |  |  | 0 | 126 | 0 | 126 |
| TDD UL/DL configuration |  |  |  | 0 |  |  |  |
| TDD special subframe |  |  |  | 1 |  |  |  |
| Note 1: Resource blocks nPRB = 0, 7, 14, 21, 28, 35, 42, 49 are allocated for both the first set and the second set. Note 2: The starting OFDM symbol for EPDCCH is determined from the higher layer signalling pdsch-Start-r11. And CFI is set to 1. Note 3: The TP from which PDSCH is transmitted shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TP 1 and TP 2 are specified. Note 4: For PQI set 0, PDSCH and EPDCCH are transmitted from TP 2. For PQI set 1, PDSCH and EPDCCH are transmitted from TP1. EPDCCH and PDSCH are transmitted from same TP. |  |  |  |  |  |  |  |

Table 8.8.3.2-2: Minimum Performance

| Test number | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 2 ECCE | R.59 TDD | OP.7 TDD | EVA5 | 2 x 2 Low | 1 | 13.6 |
| 2 | 2 ECCE | R.59 TDD | OP.7 TDD | EVA5 | 2 x 2 Low | 1 | 13.6 |

### 8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model

#### 8.8.4.1	FDD

For the parameters specified in Table 8.8.4.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.4.1-2. The purpose of this test is to verify the localized EPDCCH performance, when the EPDCCH transmission in the serving cell is interfered by the CRS of the interfering cells, applying the CRS interference model defined in clause B.6.5. In Table 8.8.4.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical setup is in accordance with Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.8.4.1-1: Test Parameters for EPDCCH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) | 2 | 2 |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) | N/A | N/A |
| PHICH duration |  |  | Normal | Normal | Normal |
| Unused RE-s and PRB-s |  |  | OCNG | N/A | N/A |
| Cell ID |  |  | 0 | 1 | 6 |
| Downlink power allocation |  | dB | 0 | -3 | -3 |
|  |  | dB | 0 | -3 | -3 |
|  |  | dB | -3 | 0 | 0 |
|  | δ | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| EPDCCH Precoder Update Granularity |  | PRB | 1 | N/A | N/A |
|  |  | ms | 1 | N/A | N/A |
| EPDCCH Beamforming Pre-Coder |  |  | Annex B. 4.5 | N/A | N/A |
| CSI-RS Reference Signal |  |  | Port 15 and 16 | N/A | N/A |
| CSI-RS reference signal resource configuration |  |  | 0 | N/A | N/A |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 | N/A | N/A |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 | N/A | N/A |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 | N/A | N/A |
| Number of EPDCCH Sets |  |  | 1 | N/A | N/A |
| EPDCCH Set type |  |  | Localized | N/A | N/A |
| Number of PRB per EPDCCH Set |  |  | 8 | N/A | N/A |
| EPDCCH Set PRBs |  |  | 0, 7, 14, 21, 28, 35, 42, 49 | N/A | N/A |
| PDSCH TM |  |  | TM9 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Time offset to cell 1 |  | s | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. CFI is set to 1. Note 2: EPDCCH is scheduled in every subframe. EPDCCH Subframe Monitoring pattern is not configured. |  |  |  |  |  |

Table 8.8.4.1-2: Minimum performance for EPDCCH for enhanced downlink control channel performance requirements Type A

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 FDD | OP.7 FDD | EPA5 | 2 x 2 Low | 1 | 13.4 |

#### 8.8.4.2	TDD

For the parameters specified in Table 8.8.4.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.4.2-2. The purpose of this test is to verify the localized EPDCCH performance, when the EPDCCH transmission in the serving cell is interfered by the CRS of the interfering cells, applying the CRS interference model defined in clause B.6.5.  In Table 8.8.4.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical setup is in accordance with Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.8.4.2-1: Test Parameters for EPDCCH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) | 2 | 2 |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) | N/A | N/A |
| PHICH duration |  |  | Normal | Normal | Normal |
| Unused RE-s and PRB-s |  |  | OCNG | N/A | N/A |
| Cell ID |  |  | 0 | 1 | 6 |
| Downlink power allocation |  | dB | 0 | -3 | -3 |
|  |  | dB | 0 | -3 | -3 |
|  |  | dB | -3 | 0 | 0 |
|  | δ | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| EPDCCH Precoder Update Granularity |  | PRB | 1 | N/A | N/A |
|  |  | ms | 1 | N/A | N/A |
| EPDCCH Beamforming Pre-Coder |  |  | Annex B. 4.5 | N/A | N/A |
| CSI-RS Reference Signal |  |  | Port 15 and 16 | N/A | N/A |
| CSI-RS reference signal resource configuration |  |  | 0 | N/A | N/A |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 | N/A | N/A |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 | N/A | N/A |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 | N/A | N/A |
| Number of EPDCCH Sets |  |  | 1 | N/A | N/A |
| EPDCCH Set type |  |  | Localized | N/A | N/A |
| Number of PRB per EPDCCH Set |  |  | 8 | N/A | N/A |
| EPDCCH Set PRBs |  |  | 0, 7, 14, 21, 28, 35, 42, 49 | N/A | N/A |
| PDSCH TM |  |  | TM9 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Time offset to cell 1 |  | s | N/A | 2 | 3 |
| Frequency offset to cell 1 |  | Hz | N/A | 200 | 300 |
| TDD UL/DL Configuration |  |  | 0 | 0 | 0 |
| TDD Special Subframe |  |  | 1 (Note 4) | 1 | 1 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. CFI is set to 1. Note 2: EPDCCH is scheduled in every subframe. EPDCCH Subframe Monitoring pattern is not configured. Note 3: Demodulation performance is averaged over normal and special subframes. |  |  |  |  |  |

Table 8.8.4.2-2: Minimum performance for EPDCCH for enhanced downlink control channel performance requirements Type A

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 TDD | OP.7 TDD | EPA5 | 2 x 2 Low | 1 | 14.2 |

### 8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model

#### 8.8.5.1	TDD

For the parameters specified in Table 8.8.5.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.5.1-2. The purpose of this test is to verify the distributed EPDCCH performance when the EPDCCH transmission in the serving cell is interfered by two interfering cells and applying TM9 interference model. In Table 8.8.5.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical setup is in accordance with Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.8.5.1-1: Test Parameters for EPDCCH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  |  | symbols | 2 (Note 1) | 2 | 2 |
| PHICH duration |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 6 | 1 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
|  | δ |  | dB | 3 | 3 | 3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| EPDCCH precoder Update Granularity |  |  | PRB | 1 | N/A | N/A |
|  |  |  | ms | 1 | N/A | N/A |
| Beamforming Pre-Coder |  |  |  | Annex B. 4.4 | N/A | N/A |
| Number of EPDCCH Sets Configured |  |  |  | 1 | N/A | N/A |
| EPDCCH Set type |  |  |  | Distributed | N/A | N/A |
| Number of PRB per EPDCCH Set |  |  |  | 4 | N/A | N/A |
| EPDCCH Set PRBs |  |  |  | 3, 17, 31, 45 | N/A | N/A |
| PDSCH TM |  |  |  | TM9 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 70 | 70 |
|  |  | Rank 2 | % | N/A | 30 | 30 |
| PDSCH precoder update granularity |  |  | PRB | N/A | 50 | 50 |
| Time offset to cell 1 |  |  | s | N/A | 2 | 3 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 200 | 300 |
| TDD UL/DL Configuration |  |  |  | 0 | 0 | 0 |
| TDD Special Subframe |  |  |  | 1 (Note 3) | 1 | 1 |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: EPDCCH is scheduled in every subframe. EPDCCH Subframe Monitoring pattern is not configured. Note 3: Demodulation performance is averaged over normal and special subframes. |  |  |  |  |  |  |

Table 8.8.5.1-2: Minimum performance for EPDCCH for enhanced downlink control channel performance requirements Type A

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55-1 TDD | OP.7 TDD | EPA5 | 2 x 2 Low | 1 | 14.8 |

### 8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model

#### 8.8.6.1	FDD

For the parameters specified in Table 8.8.6.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.8.6.1-2. The purpose of this test is to verify the distributed EPDCCH performance when the serving cell EPDCCH transmission is interfered by two interfering cells applying asynchronous TM3 interference model. In Table 8.8.6.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical setup is in accordance with Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is not provided.

Table 8.8.6.1-1: Test Parameters for EPDCCH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  |  | symbols | 2 (Note 1) | 2 | 2 |
| PHICH duration |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 1 | 6 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
|  | δ |  | dB | 3 | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 13.91 | 3.34 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| EPDCCH Precoder Update Granularity |  |  | PRB | 1 | N/A | N/A |
|  |  |  | ms | 1 | N/A | N/A |
| EPDCCH Beamforming Pre-Coder |  |  |  | Annex B.4.4 | N/A | N/A |
| Number of EPDCCH Sets Configured |  |  |  | 1 | N/A | N/A |
| EPDCCH Set type |  |  |  | Distributed | N/A | N/A |
| Number of PRB per EPDCCH Set |  |  |  | 4 | N/A | N/A |
| EPDCCH Set PRBs |  |  |  | 3, 17, 31, 45 | N/A | N/A |
| PDSCH TM |  |  |  | TM9 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.2 | As specified in clause B.5.2 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 70 | 70 |
|  |  | Rank 2 | % | N/A | 30 | 30 |
| Time offset to cell 1 |  |  | s | N/A | 330 | 667 |
| Frequency offset to cell 1 |  |  | Hz | N/A | 0 | 0 |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: EPDCCH is scheduled in every subframe. EPDCCH Subframe Monitoring pattern is not configured. |  |  |  |  |  |  |

Table 8.8.6.1-2: Minimum performance for EPDCCH for enhanced downlink control channel performance requirements Type A

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55-1 FDD | OP.7 FDD | EVA70 | 2 x 2 Low | 1 | 15.9 |

## 8.9	Demodulation (single receiver antenna)

The SNR deifintion is given in Clause 8.1.1 where the number of receiver antennas NRX assumed for the minimum performance requirement in this clause is 1.

### 8.9.1	PDSCH

#### 8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)

The parameters specified in Table 8.9.1.1-1 are valid for FDD and half-duplex FDD tests unless otherwise stated.

Table 8.9.1.1-1: Common Test Parameters (FDD and half-duplex FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |
| Cyclic Prefix |  | Normal |
| Precoder update granularity |  | Frequency domain: 1 PRG Time domain: 1 ms for Transmission mode 9 |

##### 8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)

8.9.1.1.1.1	Minimum Requirement 2 Tx Antenna Port

The requirements are specified in Table 8.9.1.1.1.1-2, with the addition of the parameters in Table 8.9.1.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.9.1.1.1.1-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| Note 1: . |  |  |  |

Table 8.9.1.1.1.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Band-width and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.62 FDD | OP.1 FDD | EPA5 | 2x1 Low | 70 | 9.0 | 0 |
| 2 | 10 MHz 16QAM 1/2 | R.84 FDD | OP.1 FDD | EPA5 | 2x1 Low | 70 | 9.3 | 1bis |

##### 8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)

8.9.1.1.2.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.9.1.1.2.1-2, with the addition of the parameters in Table 8.9.1.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.9.1.1.2.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  |
|  |  |  | dB |  | 0 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  |
| Precoding granularity |  |  | PRB |  | 6 |  |
| PMI delay (Note 2) |  |  | ms |  | 8 |  |
| Reporting interval |  |  | ms |  | 8 |  |
| Reporting mode |  |  |  |  | PUSCH 1-2 |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 001111 |  |
| PDSCH transmission mode |  |  |  |  | 4 |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |  |

Table 8.9.1.1.2.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Band-width and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.63 FDD | OP.1 FDD | EPA5 | 2x1 Low | 70 | 13.2 | 0 |

8.9.1.1.2.2	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port

The requirements are specified in Table 8.9.1.1.2.2-2, with the addition of the parameters in Table 8.9.1.1.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.9.1.1.2.2-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -6 |  |
|  |  |  | dB |  | -6 (Note 1) |  |
|  |  |  | dB |  | 0 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  |
| Precoding granularity |  |  | PRB |  | 6 |  |
| PMI delay (Note 2) |  |  | ms |  | 8 |  |
| Reporting interval |  |  | ms |  | 8 |  |
| Reporting mode |  |  |  |  | PUSCH 1-2 |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 000000000000FFFF |  |
| PDSCH transmission mode |  |  |  |  | 4 |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |  |

Table 8.9.1.1.2.2-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Band-width and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.85 FDD | OP.1 FDD | EPA5 | 4x1 Low | 70 | 11.3 | 1bis |

8.9.1.1.2.3	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.9.1.1.2.3-2, with the addition of parameters in Table 8.9.1.1.2.3-1. In Table 8.9.1.1.2.3-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.9.1.1.2.3-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 000000000000FFFF | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF #(n+4). Note 3: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.9.1.1.2.3-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.93 FDD | OP.1FDD | N/A | N/A | EPA5 |  |  | 4x1 Low | 70 | 16.7 | 1bis |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)

8.9.1.1.3.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.9.1.1.3.1-2 with the addition of the parameters in Table 8.9.1.1.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.9.1.1.3.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 (Note 1) |  |
|  |  | dB | -3 |  |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 3) |
| Reporting mode |  |  | N/A | PUSCH 3-1 |
| Reporting interval |  |  | N/A | 5 |
| PMI delay (Note 4) |  |  | N/A | 8 |
| CodeBookSubsetRestriction bitmap |  |  | N/A | 001111 |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 | 50 |
| PDSCH transmission mode |  |  | 9 |  |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |

Table 8.9.1.1.3.1-2: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.64 FDD | OP.1 FDD | EPA5 | 2x1 Low | 70 | 4.7 | 0 |
| 2 | 10 MHz QPSK 1/3 | R.86 FDD | OP.1 FDD | EPA5 | 2x1 Low | 70 | 2.5 | 1bis |

8.9.1.1.3.2	Single-layer Spatial Multiplexing with CRS assistance information

The requirements are specified in Table 8.9.1.1.3.2-2, with the addition of parameters in Table 8.9.1.1.3.2-1. In Table 8.9.1.1.3.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the single-layer spatial multiplexing TM9 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.9.1.1.3.2-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | N/A | N/A |
| PDSCH transmission mode |  |  | TM9 | N/A | N/A |
| Precoding granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  | Annex B.4.1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 10 | 10 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.9.1.1.3.2-2: Minimum Performance

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.94 FDD | OP.1FDD | N/A | N/A | EPA5 |  |  | 2x1 Low | 70 | 11.9 | 1bis |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/NocEs/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

#### 8.9.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.9.1.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.9.1.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 1 |
| Special subframe configuration (Note 2) |  |  | 4 |
| Cyclic prefix |  |  | Normal |
| Cell ID |  |  | 0 |
| Inter-TTI Distance |  |  | 1 |
| Number of HARQ processes per component carrier |  | Processes | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |
| Precoder update granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms for Transmission mode 9 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

##### 8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)

8.9.1.2.1.1	Minimum Requirement 2 Tx Antenna Port

The requirements are specified in Table 8.9.1.2.1.1-2, with the addition of the parameters in Table 8.9.1.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.9.1.2.1.1-1: Test Parameters for Transmit diversity Performance (FRC)

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 2 |
| Note 1: . |  |  |  |

Table 8.9.1.2.1.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.62 TDD | OP.1 TDD | EPA5 | 2x1 Low | 70 | 8.8 | 0 |
| 2 | 10 MHz 16QAM 1/2 | R.84 TDD | OP.1 TDD | EPA5 | 2x1 Low | 70 | 9.5 | 1bis |

##### 8.9.1.2.2		Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)

8.9.1.2.2.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.9.1.2.2.1-2, with the addition of the parameters in Table 8.9.1.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.9.1.2.2.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 001111 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |

Table8.9.1.2.2.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.63 TDD | OP.1 TDD | EPA5 | 2x1 Low | 70 | 13.1 | 0 |

8.9.1.2.2.2	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port

The requirements are specified in Table 8.9.1.2.2.2-2, with the addition of the parameters in Table 8.9.1.2.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.9.1.2.2.2-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC(

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 000000000000FFFF |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |

Table 8.9.1.2.2.2-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.85 TDD | OP.1 TDD | EPA5 | 4x1 Low | 70 | 11.6 | 1bis |

8.9.1.2.2.3	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.9.1.2.2.3-2, with the addition of parameters in Table 8.9.1.2.2.3-1. In Table 8.9.1.2.2.3-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.9.1.2.2.3-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 or 4 (Note 4) | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 000000000000FFFF | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 4: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |

Table 8.9.1.2.2.3-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.93 TDD | OP.1TDD | N/A | N/A | EPA5 |  |  | 4x1 Low | 70 | 16.8 | 1bis |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)

8.9.1.2.3.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.9.1.2.3.1-2 with the addition of the parameters in Table 8.9.1.2.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.9.1.2.3.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 (Note 1) |  |
|  |  | dB | -3 |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | 5 / 4 |
| CSI reference signal configuration |  |  | 1 | 1 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000100000000 | 4 / 0000001000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 3) |
| Reporting mode |  |  | N/A | PUSCH 3-1 |
| Reporting interval |  |  | N/A | 5 |
| PMI delay (Note 4) |  |  | N/A | 10 |
| CodeBookSubsetRestriction bitmap |  |  | N/A | 001111 |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 |  |
| Simultaneous transmission |  |  | No |  |
| PDSCH transmission mode |  |  | 9 |  |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |

Table 8.9.1.2.3.1-2: Minimum performance for CDM-multiplexed DM RS without simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.64 TDD | OP.1 TDD | EPA5 | 2x1 Low | 70 | 4.5 | 0 |
| 2 | 10 MHz QPSK 1/3 | R.86 TDD | OP.1 TDD | EPA5 | 2x1 Low | 70 | 2.9 | 1bis |

8.9.1.2.3.2	Single-layer Spatial Multiplexing with CRS assistance information

The requirements are specified in Table 8.9.1.2.3.2-2, with the addition of parameters in Table 8.9.1.2.3.2-1. In Table 8.9.1.2.3.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the single-layer spatial multiplexing TM9 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.9.1.2.3.2-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal configuration |  |  | 1 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | TM9 | N/A | N/A |
| Precoding granularity |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  | Annex B.4.1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 10 | 10 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. |  |  |  |  |  |

Table 8.9.1.2.3.2-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.94 TDD | OP.1TDD | N/A | N/A | EPA5 |  |  | 2x1 Low | 70 | 11.3 | 1bis |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/NocEs/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

### 8.9.2	PHICH

#### 8.9.2.1	FDD and half-duplex FDD

##### 8.9.2.1.1	Transmit diversity performance

For the parameters specified in Table 8.5.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.9.2.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.9.2.1.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |  |
| 1 | 10 MHz | R.19 | OP.1 FDD | EPA5 | 2 x 1 Low | 0.1 | 8.6 | 0, 1bis |

#### 8.9.2.2	TDD

##### 8.9.2.2.1	Transmit diversity performance

For the parameters specified in Table 8.5.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.9.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.9.2.2.1-1: Minimum performance PHICH

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |  |
| 1 | 10 MHz | R.19 | OP.1 TDD | EPA5 | 2 x 1 Low | 0.1 | 8.6 | 0, 1bis |

### 8.9.3	PBCH

#### 8.9.3.1	FDD and half-duplex FDD

##### 8.9.3.1.1	Transmit diversity performance

For the parameters specified in Table 8.6.1-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.9.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.9.3.1.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 1.4 MHz | R.22 | EPA5 | 2 x 1 Low | 1 | -1.3 | 0, 1bis |

#### 8.9.3.2	TDD

##### 8.9.3.2.1	Transmit diversity performance

For the parameters specified in Table 8.6.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 8.9.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.9.3.2.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 1.4 MHz | R.22 | EPA5 | 2 x 1 Low | 1 | -1.7 | 0, 1bis |

### 8.9.4	PDCCH/PCFICH

#### 8.9.4.1	FDD and half-duplex FDD

##### 8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.1-1 and Table 8.9.4.1.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.9.4.1.1-2. In Table 8.9.4.1.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.9.4.1.1-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.9.4.1.1-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.9.4.1.1-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-4 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 16.0 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.4.1-1 and Table 8.9.4.1.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.9.4.1.2-2. In Table 8.9.4.1.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes information on Cell 2 and Cell 3.

Table 8.9.4.1.2-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.9.4.1.2-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH |  |  | 2 | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.9.4.1.2-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.17-3 FDD | OP.1 FDD | EPA5 | EPA5 | EPA5 | 4x1 Low | 1 | 15.4 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

#### 8.9.4.2	TDD

##### 8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 2 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1. For the parameters specified in Table 8.4.2-1 and Table 8.9.4.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.9.4.2.1-2. In Table 8.9.4.2.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the agressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes Cell 2 and Cell 3.

Table 8.9.4.2.1-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.9.4.2.1-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell Id |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 1 | 1 | 1 |
| CFI indicated in PCFICH |  |  | 1 | 1 | 1 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.9.4.2.1-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.16-4 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 16.9 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

##### 8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.4.2-1 and Table 8.9.4.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.9.4.2.2-2. In Table 8.9.4.2.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided and includes information on Cell 2 and Cell 3.

Table 8.9.4.2.2-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | As defined in Table 8.9.4.2.2-2 | 13.91 | 3.34 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 | 2 | 2 |
| CFI indicated in PCFICH |  |  | 2 | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| PDSCH TM |  |  | 4 | N/A | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 and Cell 3 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |  |

Table 8.9.4.2.2-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 CCE | R.17-3 TDD | OP.1 TDD | EPA5 | EPA5 | EPA5 | 4x1 Low | 1 | 15.6 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |

## 8.10	Demodulation (4 receiver antenna ports)

The performance requirements specified in this clause are valid for 4Rx capable UEs.

### 8.10.1	PDSCH

#### 8.10.1.1	FDD (Fixed Reference Channel)

The parameters specified in Table 8.10.1.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.10.1.1-1: Common Test Parameters (FDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG for Transmission modes 9 and 10 Time domain: 1 ms |  |
| Cyclic Prefix |  |  |  | Normal |  |
| Cell_ID |  |  |  | 0 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |

##### 8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.1-2, with the addition of the parameters in Table 8.10.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.10.1.1.1-1: Test Parameters for Transmit diversity Performance (FRC) with 4 RX Antenna Ports

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |
|  |  |  |  | dB |  | -3 (NOTE 1) |
|  |  |  |  | dB |  | 0 |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |
| PDSCH transmission mode |  |  |  |  |  | 2 |
| NOTE 1: . |  |  |  |  |  |  |

Table 8.10.1.1.1-2: Minimum performance Transmit Diversity (FRC) with 4 RX Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11 FDD | OP.1 FDD | EVA5 | 2x4 Medium correlation A, ULA | 70 | 3.9 | ≥2 |

##### 8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model

The requirements are specified in Table 8.10.1.1.1A-2, with the addition of parameters in Table 8.10.1.1.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cells applying transmission mode 3 interference model defined in clause B.5.2. In Table 8.10.1.1.1A-1, Cell 1 is the serving cell, and Cell 2 is an interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.10.1.1.1A-1: Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (NOTE 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (NOTE 2) |  |  |  | dB |  | N/A |  | -2.23 |  |
| BWChannel |  |  |  | MHz |  | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  |  |  | Normal |  | Normal |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| Number of control OFDM symbols |  |  |  |  |  | 2 |  | 2 |  |
| PDSCH transmission mode |  |  |  |  |  | 2 |  | N/A |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.2 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-0 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  | PUSCH(Note 5) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  |  |  | 2 |  | N/A |  |
| NOTE 1: NOTE 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. NOTE 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. NOTE 4: Cell 2 transmission is delayed with respect to Cell 1 by 0.33 ms. NOTE 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 and #0. |  |  |  |  |  |  |  |  |  |

Table 8.10.1.1.1A-2: Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel |  | OCNG Pattern |  |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (NOTE 3) |  | Reference Value |  |  |  |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |  | Fraction of Maximum Throughput (%) |  |  | SINR (dB) (NOTE 2) |  |  |
| 1 | R.46 FDD |  | OP.1 FDD | N/A |  | EVA70 |  | EVA70 |  | 2x4 Low |  |  | 70 |  | -4.4 | ≥1 |  |
| NOTE 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. NOTE 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. NOTE 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.2-2, with the addition of the parameters in Table 8.10.1.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.2-1: Test Parameters for Large Delay CDD (FRC) with 4 RX Antenna Ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.10.1.1.2-2: Minimum performance Large Delay CDD (FRC) with 4 RX Antenna Ports

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11 FDD | OP.1 FDD | EVA70 | 2x4 Low | 70 | 8.0 | ≥2 |

##### 8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.3-2, with the addition of the parameters in Table 8.10.1.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.10.1.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.10.1.1.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model and 4 RX Antenna Ports

| Parameter |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  |  | MHz |  | 10 |  | 10 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | % |  | N/A |  | 80 |  |
|  |  | Rank 2 |  | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 50 |  | 6 |  |
| PMI delay (Note 4) |  |  |  | ms |  | 8 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. |  |  |  |  |  |  |  |  |  |

Table 8.10.1.1.3-2: Enhanced Performance Requirement Type A, Single-Layer Spatial Multiplexing (FRC) with TM4 interference model and 4 RX Antenna Ports

| Test Number | Reference Channel and MCS | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.47 FDD 16QAM | OP.1 FDD | N/A | EVA5 | EVA5 | 2x4 Low | 70 | -2.3 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)

For single carrier, the requirements are specified in Table 8.10.1.1.4-2, with the addition of the parameters in Table 8.10.1.1.4-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.4-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC) with 4 RX Antenna Ports

| Parameter |  | Unit | Test 1-2 | Test 3 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 6 | 50 |
| PMI delay (Note 2) |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 1-2 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |

Table 8.10.1.1.4-2: Minimum performance Dual-Layer Spatial Multiplexing (FRC) with 4 RX Antenna Ports

| Test num. | Bandwidth and MCS | Referencechannel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate- gory | DL UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz | R.36 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 10.1 | ≥2 | ≥6 |
| 2 | 10 MHz 256 QAM | R.72 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 18.0 | 11-12 | ≥11 |
| 3 | 10MHz 1024QAM | R.102 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 27.7 | TBD | 20, ≥ 22 |

##### 8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports

The requirements are specified in Table 8.10.1.1.4A-2, with the addition of the parameters in Table 8.10.1.1.4A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband precoding.

Table 8.10.1.1.4A-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 50 |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 110000 |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.10.1.1.4A-2: Enhanced Performance Requirement Type C for Dual-Layer Spatial Multiplexing with TM4 (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11 FDD | OP.1 FDD | ETU70 | 2x4 Medium | 70 | 16.8 | ≥2 |

##### 8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.5-2, with the addition of the parameters in Table 8.10.1.1.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.10.1.1.5-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.10.1.1.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model and 4 RX Antenna Ports

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cell Id |  |  | 0 | 126 |
| PDSCH transmission mode |  |  | 9 | 9 |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  | Ms | 8 | N/A |
| Reporting interval |  | Ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |

Table 8.10.1.1.5-2: Enhanced Performance Requirement Type A, CDM-multiplexed DM RS with TM9 interference model and 4 RX Antenna Ports

| Test Number | Reference Channel and MCS | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R. 76 FDD QPSK | OP.1 FDD | N/A | EVA5 | EVA5 | 2x4 Low | 70 | -3.0 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.5A-2, with the addition of the parameters in Table 8.10.1.1.5A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 with a simultaneous transmission on the other antenna port in the serving cell, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.10.1.1.5A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 9 |
| Number of MBSFN subframes |  | Subframes | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |

Table 8.10.1.1.5A-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

##### 8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)

For single-layer transmission on antenna port 7, 8, 11 or 13 upon detection of a PDCCH with DCI format 2C, the requirement is specified in Table 8.10.1.1.5B-2, with the addition of the parameters in Table 8.10.1.1.5B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of the test is to verify rank-1 performance on antenna port 11 with a simultaneous transmission on the antenna port 7, 8 or 13 with DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.10.1.1.5B-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| dmrs-Enhancements-r13 |  |  | Enable |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 11. Note 3: Modulation symbols of an interference signal are random mapped onto one antenna port among antenna port 7, 8 and 13. The upadate granularity for randomized mapping antenna port is 1 PRG in frequency domain and 1ms in time domain. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 with OCC =4. |  |  |  |

Table 8.10.1.1.5B-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

##### 8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

For dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.10.1.1.6-2, with the addition of the parameters in Table 8.10.1.1.6-1 where Cell 1 is the serving cell and Cell 2 is the interfering cell. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power, and to verify that the UE correctly estimate SNR.

Table 8.10.1.1.6-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations with 4 RX Antenna Ports

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
|  | PDSCH_RA | dB | 4 | NA |
|  | PDSCH_RB | dB | 4 | NA |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 | 126 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | NA |
|  |  |  | Reference Value in Table 8.10.1.1.6-2 | 7.25dB |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | Blanked |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.10.1.1.6-2: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations and 4 RX Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 FDD | OP.1 FDD | N/A | ETU5 | ETU5 | 2x4 Low | 70 | 9.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing

The requirements are specified in 8.10.1.1.6A-2, with the addition of the parameters in 8.10.1.1.6A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of this test is to verify rank-two performance for full RB allocation upon antenna ports 7 and 8.

8.10.1.1.6A-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| Reporting mode |  |  | PUSCH 3-1 |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

8.10.1.1.6A-2: Enhanced Performance Requirement Type C for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 FDD | OP.1 FDD | EPA5 | 2x4 Medium | 70 | 16.2 | ≥2 |

##### 8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)

The requirements are specified in 8.10.1.1.6B-2, with the addition of the parameters in 8.10.1.1.6B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of this test is to verify rank-two performance for full RB allocation upon antenna ports 7 and 8.

8.10.1.1.6B-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16, 17, 18 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | N/A |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| Reporting mode |  |  | PUSCH 3-1 |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

8.10.1.1.6B-2: Minimum performance Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 1024QAM | R.102 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 29.4 | 20, ≥ 22 |

##### 8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.7-2, with the addition of the parameters in Table 8.10.1.1.7-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.7-1: Test parameters for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| PDSCH rank |  |  | 3 |
| CodeBookSubsetRestriction bitmap |  |  | 0100 |
| Note 1: . |  |  |  |

Table 8.10.1.1.7-2: Minimum performance Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM | R.73 FDD | OP.1 FDD | EVA70 | 4x4 Low | 70 | 15.1 | ≥5 |

##### 8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.7A-2, with the addition of the parameters in Table 8.10.1.1.7A-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.7A-1: Test parameters for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| Cell-specific reference signals |  |  | Antenna ports 1,2,3,4 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| PDSCH rank |  |  | 3 |
| CodeBookSubsetRestriction bitmap |  |  | 0100 |
| Note 1: . |  |  |  |

Table 8.10.1.1.7A-2: Enhanced Performance Requirement Type C for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.73-1 FDD | OP.1 FDD | EVA70 | 4x4 Medium A | 70 | 22.2 | ≥5 |

##### 8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.8-2, with the addition of the parameters in Table 8.10.1.1.8-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.8-1: Test parameters for Closed Loop spatial multiplexing, 4 Layers spatial multiplexing with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Precoding granularity |  | PRB | 50 |
| PMI delay |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| Note 1: . |  |  |  |

Table 8.10.1.1.8-2: Minimum performance for Closed Loop spatial multiplexing, 4 Layers spatial multiplexing with 4 Tx ports and 4 Rx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 | ≥5 |

##### 8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.9-2, with the addition of the parameters in Table 8.10.1.1.9-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.9-1: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 | Test 1a |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | 4 layer precoding based on WB PMI feedback | 4 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 3 | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| Precoding granularity |  |  | 50 | 50 |
| PMI delay |  |  | 8 | 8 |
| Reporting interval |  |  | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 | 0xFFFF000000000000 |
| DMRS OCC length per port |  |  | 2 | 4 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.10.1.1.9-2: Minimum performance for for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.75 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 18.4 | ≥5 |
| 1a | 10MHz 16QAM | R.75A FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | [17.4] | ≥5 |
| NOTE: For a UE supporting TM9 rank3/4 transmission based on DMRS OCC length 4, Test 1a will be run and Test 1 will be skipped. Otherwise, Test 1a is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

##### 8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.9A-2, with the addition of the parameters in Table 8.10.1.1.9A-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.1.9A-1: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3(Note 4, 5) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 8 |
| Reporting interval |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.10.1.1.9A-2: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.75 FDD | OP.1 FDD | EPA5 | 4x4 Medium A Xpol | 70 | 18.2 | ≥5 |

##### 8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.10-2, with the addition of parameters in Table 8.10.1.1.10-1. In Table 8.10.1.1.10-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.10.1.1.10-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.10.1.1.10-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |

Table 8.10.1.1.10-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.35 FDD | OP.1FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x4 low | 70 | 11.1 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.11-2, with the addition of parameters in Table 8.10.1.1.11-1. In Table 8.10.1.1.11-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cell.

Table 8.10.1.1.11-1: Test Parameters

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -6 | -6 |
|  |  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  |  | dB | 3 | 3 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
|  |  |  | dB | Reference Value in Table 8.10.1.1.11-2 | 10.45 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cyclic prefix |  |  |  | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  |  | 0 | 1 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1,2,3 |  |
| Number of control OFDM symbols (CFI) |  |  |  | 2 | 2 |
| PDSCH transmission mode |  |  |  | 4 | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A |
| PMI delay (Note 2) |  |  | ms | 8 | N/A |
| Reporting interval |  |  | ms | 1 | N/A |
| Reporting mode |  |  |  | PUSCH 3-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 000000000000FFFF | N/A |
| Time Offset relative to Cell 1 |  |  | s | N/A | 3 |
| Frequency shift relative to Cell 1 |  |  | Hz | N/A | 300 |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 |
|  | Rank 2 |  | % | N/A | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |

Table 8.10.1.1.11-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.36-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 4x4 low | 70 | 9.3 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.1.12-2, with the addition of parameters in Table 8.10.1.1.12-1. In Table 8.10.1.1.12-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM9 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.10.1.1.12-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.10.1.1.12-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 9 | N/A | N/A |
| Precoding granularity |  | PRB | Frequency domain: 50 PRB Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  | Annex B.4.1 |  |  |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . |  |  |  |  |  |

Table 8.10.1.1.12-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.X FDD | OP.1FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x4 low | 70 | 12.3 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/NocEs/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports

8.10.1.1.13.1	Minimum requirements for QCL Type C and 3 Layers Spatial Multiplexing

The requirements are specified in Table 8.10.1.1.13.1-3, with the additional parameters in Table 8.10.1.1.13.1-1 and Table 8.10.1.1.13.1-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.10.1.1.13.1-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP 1 and TP 2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.10.1.1.13.1-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 2) | Random beamforming (rank 1) |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16,17,18} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16,17,18} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 2 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1-3dB |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW2 is transmitted from TP 1 and CW1 is transmitted from TP 2) |  |  |  |  |

Table 8.10.1.1.13.1-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW2 CSI-RS 1 for CW1 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.10.1.1.13.1-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.98 FDD | R.99 FDD | OP.1FDD | OP.1FDD | EPA5 | EPA5 | 4x4 Low | 70 | 11.0 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

8.10.1.1.13.2	Minimum requirements for QCL Type C and 4 Layers Spatial Multiplexing

The requirements are specified in Table 8.10.1.1.13.2-3, with the additional parameters in Table 8.10.1.1.13.2-1 and Table 8.10.1.1.13.2-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.10.1.1.13.2-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP 1 and TP 2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.10.1.1.13.2-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 2) | Random beamforming (rank 2) |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1,2,3 |
| CSI reference signals 0 |  |  | Antenna ports {15,16,17,18} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16,17,18} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 2 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW1 is transmitted from TP 1 and CW2 is transmitted from TP 2) |  |  |  |  |

Table 8.10.1.1.13.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.10.1.1.13.2-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.100 FDD | R.100 FDD | OP.1FDD | OP.1FDD | EPA5 | EPA5 | 4x4 Low | 70 | 14.8 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.1.14	HST-SFN performance

8.10.1.1.14.1	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.10.1.1.14.1-2, with the addition of the parameters in Table 8.10.1.1.14.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of this test is to verify UE performance in the HST-SFN-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhDemodFlag2-r16 [7] is received.

Table 8.10.1.1.14.1-1: Test Parameters for UE performance in HST-SFN-500 scenario (FRC)

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |
|  |  |  |  | dB |  | -3 (NOTE 1) |
|  |  |  |  | dB |  | 0 |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |
| PDSCH transmission mode |  |  |  |  |  | 3 |
| NOTE 1: . |  |  |  |  |  |  |

Table 8.10.1.1.14.1-2: Minimum performance UE in HST-SFN-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 FDD | OP.1 FDD | HST-SFN-500 | 2x4 | 70 | 6.8 | ≥ 1 |
| NOTE 1: Test case applicability is defined in 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

#### 8.10.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.10.1.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.10.1.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM, 256QAM and 1024QAM |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG for Transmission modes 9 and 10 Time domain: 1 ms |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |

##### 8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8. 10.1.2.1-2, with the addition of the parameters in Table 8. 10.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.10.1.2.1-1: Test Parameters for Transmit diversity Performance (FRC) with 4Rx Antenna Ports

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |
|  |  |  |  | dB |  | -3 (Note 1) |
|  |  |  |  | dB |  | 0 |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |
| PDSCH transmission mode |  |  |  |  |  | 2 |
| Note 1: |  |  |  |  |  |  |

Table 8.10.1.2.1-2: Minimum performance Transmit Diversity (FRC) with 4Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11 TDD | OP.1 TDD | EVA5 | 2x4 Medium correlation A, ULA | 70 | 3.9 | ≥2 |

##### 8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model

The requirements are specified in Table 8.10.1.2.1A-2, with the addition of parameters in Table 8.10.1.2.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cells applying transmission mode 3 interference model defined in clause B.5.2. In Table 8.10.1.2.1A-1, Cell 1 is the serving cell, and Cell 2, 3 are interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively.

Table 8.10.1.2.1A-1: Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  | MHz |  | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  |  | Normal |  | Normal |  |
| Cell Id |  |  |  |  | 0 |  | 1 |  |
| Number of control OFDM symbols |  |  |  |  | 2 |  | 2 |  |
| PDSCH transmission mode |  |  |  |  | 2 |  | N/A |  |
| Interference model |  |  |  |  | N/A |  | As specified in clause B.5.2 |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Reporting interval |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  | PUCCH 1-0 |  | N/A |  |
| ACK/NACK feedback mode |  |  |  |  | Multiplexing |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  | PUSCH(Note 5) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  |  | 4 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: The cells are time-synchronous. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |  |  |  |

Table 8.10.1.2.1A-2: Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel |  | OCNG Pattern |  |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |  |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |  | Fraction of Maximum Throughput (%) |  |  | SINR (dB) (Note 2) |  |  |
| 1 | R.46 TDD |  | OP.1 TDD | N/A |  | EVA70 |  | EVA70 |  | 2x4 Low |  |  | 70 |  | -4.9 | ≥1 |  |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.2-2, with the addition of the parameters in Table 8.10.1.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.2-1: Test Parameters for Large Delay CDD (FRC) with 4Rx Antenna Ports

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 |
|  |  |  | dB | -3 (Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  |  | Bundling |
| PDSCH transmission mode |  |  |  | 3 |
| Note 1: |  |  |  |  |

Table 8.10.1.2.2-2: Minimum performance Large Delay CDD (FRC) with 4Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11-1 TDD | OP.1 TDD | EVA70 | 2x4 Low | 70 | 7.7 | ≥2 |

##### 8.10.1.2.3		Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.3-2, with the addition of the parameters in Table 8.10.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.10.1.2.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.10.1.2.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model and 4Rx Antenna Ports

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  | MHz |  | 10 |  | 10 |  |
| Cell Id |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  | 6 |  | N/A |  |
| Interference model |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  | PRB |  | 50 |  | 6 |  |
| PMI delay (Note 4) |  |  | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 1111 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. |  |  |  |  |  |  |  |  |

Table 8.10.1.2.3-2: Enhanced Performance Requirement Type A, Single-Layer Spatial Multiplexing (FRC) with TM4 interference model and 4Rx Antenna Ports

| Test Number | Reference Channel and MCS | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.47 TDD 16QAM | OP.1 TDD | N/A | EVA5 | EVA5 | 2x4 Low | 70 | -1.9 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.4-2, with the addition of the parameters in Table 8.10.1.2.4-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.4-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC) with 4Rx Antenna Ports

| Parameter |  | Unit | Test 1-2 | Test 3 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  | PRB | 6 | 6 |
| PMI delay (Note 2) |  | ms | 10 or 11 | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 | PUSCH 3-1 |
| ACK/NACK feedback mode |  |  | Bundling | Bundling |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |

Table 8.10.1.2.4-2: Minimum performance Dual-Layer Spatial Multiplexing (FRC) with 4Rx Antenna Ports

|  | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | DL UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz 64 QAM | R.36 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 10.4 | ≥2 | ≥6 |
| 2 | 10 MHz 256QAM | R.72 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 17.5 | 11-12 | ≥11 |
| 3 | 10MHz 1024QAM | R.102 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 28.6 | TBD | 20, ≥ 22 |

##### 8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports

The requirements are specified in Table 8.10.1.2.4A-2, with the addition of the parameters in Table 8.10.1.2.4A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband precoding.

Table 8.10.1.2.4A-1: Test Parameters for Dual-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 |
|  |  |  | dB | -3 (Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -98 |
| Precoding granularity |  |  | PRB | 50 |
| PMI delay (Note 2) |  |  | ms | 10 or 11 |
| Reporting interval |  |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  |  | PUSCH 3-1 |
| ACK/NACK feedback mode |  |  |  | Bundling |
| CodeBookSubsetRestriction bitmap |  |  |  | 110000 |
| PDSCH transmission mode |  |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |

Table 8.10.1.2.4A-2: Enhanced Performance Requirement Type C for Dual-Layer Spatial Multiplexing (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.11-1 TDD | OP.1 TDD | ETU70 | 2x4 Medium | 70 | 16.9 | ≥2 |

##### 8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.5-2, with the addition of the parameters in Table 8.10.1.2.5-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed-loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.10.1.2.5-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1and Cell 2, respectively.

Table 8.10.1.2.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model and 4Rx Antenna Ports

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cell Id |  |  | 0 | 126 |
| PDSCH transmission mode |  |  | 9 | 9 |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  | ms | 10 or 11 | N/A |
| Reporting interval |  | ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |

Table 8.10.1.2.5-2: Enhanced Performance Requirement Type A, CDM-multiplexed DM RS with TM9 interference model and 4Rx Antenna Ports

| Test Number | Reference Channel and MCS | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.76 TDD QPSK | OP.1 TDD | N/A | EVA5 | EVA5 | 2x4 Low | 70 | -3.3 | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)

The requirements are specified in Table 8.10.1.2.5A-2, with the addition of the parameters in Table 8.10.1.2.5A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 with a simultaneous transmission on the other antenna port in the serving cell, and to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power.

Table 8.10.1.2.5A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 9 |
| Number of MBSFN subframes |  | Subframes | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |

Table 8.10.1.2.5A-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.44 TDD | OP.1 TDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

Table 8.10.1.2.5A-3: Void

##### 8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)

For single-layer transmission on antenna port 7, 8, 11 or 13 upon detection of a PDCCH with DCI format 2C, the requirement is specified in Table 8.10.1.2.5B -2, with the addition of the parameters in Table 8.10.1.2.5B -1 and the downlink physical channel setup according to Annex C.3.2. The purpose of the test is to verify rank-1 performance on antenna port 11 with a simultaneous transmission on the antenna port 7, 8 or 13 with DMRS enhancement table and 4 orthogonal DMRS ports (dmrs-Enhancements-r13 UE-EUTRA-Capability [7]).

Table 8.10.1.2.5B-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1A |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| dmrs-Enhancements-r13 |  |  | Enable |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 11. Note 3: Modulation symbols of an interference signal are random mapped onto one antenna port among antenna port 7, 8 and 13. The upadate granularity for randomized mapping antenna port is 1 PRG in frequency domain and 1ms in time domain. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 with OCC =4. |  |  |  |

Table 8.10.1.2.5B-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations with Enhanced DMRS table

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.44 TDD | OP.1 TDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

##### 8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

For dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.10.1.2.6-2, with the addition of the parameters in Table 8.10.1.2.6-1 where Cell 1 is the serving cell and Cell 2 is the interfering cell. The downlink physical channel setup is set according to Annex C.3.2. The purpose of these tests is to verify the rank-2 performance for full RB allocation, to verify rate matching with multiple CSI reference symbol configurations with non-zero and zero transmission power, and to verify that the UE correctly estimate SNR.

Table 8.10.1.2.6-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations and 4Rx Antenna Ports

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
|  | PDSCH_RA | dB | 4 | NA |
|  | PDSCH_RB | dB | 4 | NA |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 | 126 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | NA |
|  |  |  | Reference Value in Table 8.10.1.2.6-2 | 7.25dB |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | Blanked |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.10.1.2.6-2: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.51 TDD | OP.1 TDD | N/A | ETU5 | ETU5 | 2x4 Low | 70 | 9.5 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing

The requirements are specified in Table 8.10.1.2.6A-2, with the addition of the parameters in Table 8.10.1.2.6A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the rank-two performance for full RB allocation upon antenna ports 7 and 8.

Table 8.10.1.2.6A-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference symbols |  |  | Antenna port 0 and antenna port 1 |
| Beamforming model |  |  | Annex B.4.3 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks |  | PRB | 50 |
| Reporting mode |  |  | PUSCH 3-1 |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.10.1.2.6A-2: Enhanced Performance Requirement Type C for CDM-multiplexed DM RS (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.32 TDD | OP.1 TDD | EPA5 | 2x4 Medium | 70 | 15.8 | ≥2 |

##### 8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.6B-2, with the addition of the parameters in Table 8.10.1.2.6B-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the rank-two performance for full RB allocation upon antenna ports 7 and 8.

Table 8.10.1.2.6B-1: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference symbols |  |  | Antenna port 0 and antenna port 1 |
| CSI reference signals |  |  | Antenna ports 15,16, 17, 18 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | N/A |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks |  | PRB | 50 |
| Reporting mode |  |  | PUSCH 3-1 |
| PDSCH transmission mode |  |  | 9 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.10.1.2.6B-2: Minimum performance Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 1024QAM | R.102 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 29.0 | 20, ≥ 22 |

##### 8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.7-2, with the addition of the parameters in Table 8.10.1.2.7-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.7-1: Test parameters for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 3 |
| PDSCH rank |  |  | 3 |
| CodeBookSubsetRestriction bitmap |  |  | 0100 |
| Note 1: . |  |  |  |

Table 8.10.1.2.7-2: Minimum performance Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM | R.73 TDD | OP.1 TDD | EVA70 | 4x4 Low | 70 | 14.9 | ≥5 |

##### 8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.7A-2, with the addition of the parameters in Table 8.10.1.2.7A-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.7A-1: Test parameters for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 3 |
| PDSCH rank |  |  | 3 |
| CodeBookSubsetRestriction bitmap |  |  | 0100 |
| Note 1: . |  |  |  |

Table 8.10.1.2.7A-2: Enhanced Performance Requirement Type C for Open Loop spatial multiplexing, 3 Layers with 4 Tx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.73-1 TDD | OP.1 TDD | EVA70 | 4x4 medium A | 70 | 22.2 | ≥5 |

##### 8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas

The requirements are specified in Table 8.10.1.2.8-2, with the addition of the parameters in Table 8.10.1.2.8-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.8-1: Test parameters for Closed Loop spatial multiplexing, 4 Layers spatial multiplexing with 4 Tx ports and 4 Rx ports

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Precoding granularity |  | PRB | 50 |
| PMI delay |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| Uplink-Downlink Configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Note 1: . |  |  |  |

Table 8.10.1.2.8-2: Minimum performance for Closed Loop spatial multiplexing, 4 Layers spatial multiplexing with 4 Tx ports and 4 Rx ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 | ≥5 |

##### 8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.9-2, with the addition of the parameters in Table 8.10.1.2.9-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.9-1: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 | Test 1a |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | 4 layer precoding based on WB PMI feedback | 4 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | 5 / 4 |
| CSI reference signal configuration |  |  | 3 | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| Precoding granularity |  |  | 50 | 50 |
| PMI delay |  |  | 10 or 11 | 10 or 11 |
| Reporting interval |  |  | 1 or 4 | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 | 0xFFFF000000000000 |
| DMRS OCC length per port |  |  | 2 | 4 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1 and 6. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 8.10.1.2.9-2: Minimum performance for for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth amd MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.75 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 19.0 | ≥5 |
| 1a | 10MHz 16QAM | R.75A TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | [18.0] | ≥5 |
| NOTE: For a UE supporting TM9 rank3/4 transmission based on DMRS OCC length 4, Test 1a will be run and Test 1 will be skipped. Otherwise, Test 1a is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

##### 8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.9A-2, with the addition of the parameters in Table 8.10.1.2.9A-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.10.1.2.9A-1: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 (Note 4, 5) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 10 or 11 |
| Reporting interval |  |  | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1 and 6. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.10.1.2.9A-2: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.75 TDD | OP.1 TDD | EPA5 | 4x4 Medium A Xpol | 70 | 18.1 | ≥5 |

##### 8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.10-2, with the addition of parameters in Table 8.10.1.2.10-1. In Table 8.10.1.2.10-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.10.1.2.10-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.10.1.2.10-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | N/A | N/A |
| Precoding granularity |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 8 | N/A | N/A |
| Reporting interval |  | ms | 1 or 4 (Note 3) | N/A | N/A |
| Reporting mode |  |  | PUSCH 3-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |

Table 8.10.1.2.10-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.35 TDD | OP.1TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x4 low | 70 | 11.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.11-2, with the addition of parameters in Table 8.10.1.2.11-1. In Table 8.10.1.2.11-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2.

The purpose of the test is to verify the closed loop single layer TM4 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cell.

Table 8.10.1.2.11-1: Test Parameters

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  |  | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 |
| Downlink power allocation |  |  | dB | -6 | -6 |
|  |  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  |  | dB | 3 | 3 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
|  |  |  | dB | Reference Value in Table 8.10.1.2.11-2 | 10.45 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cyclic prefix |  |  |  | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  |  | 0 | 1 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1,2,3 |  |
| Number of control OFDM symbols (CFI) |  |  |  | 2 | 2 |
| PDSCH transmission mode |  |  |  | 4 | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A |
| PMI delay (Note 2) |  |  | ms | 8 | N/A |
| Reporting interval |  |  | ms | 1 or 4 (Note 3) | N/A |
| Reporting mode |  |  |  | PUSCH 3-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 000000000000FFFF | N/A |
| ACK/NACK feedback mode |  |  |  | Multiplexing | N/A |
| Time Offset relative to Cell 1 |  |  | s | N/A | 3 |
| Frequency shift relative to Cell 1 |  |  | Hz | N/A | 300 |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 |
|  | Rank 2 |  | % | N/A | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF #n based on PMI estimation at a downlink SF not later than SF #(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |

Table 8.10.1.2.11-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.36-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 4x4 low | 70 | 9.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)

The requirements are specified in Table 8.10.1.2.12-2, with the addition of parameters in Table 8.10.1.2.12-1. In Table 8.10.1.2.12-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single layer TM9 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.10.1.2.12-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.10.1.2.12-2 | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal configuration |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 /0010000000000000 | N/A | N/A |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 9 | N/A | N/A |
| Precoding granularity |  | PRB | Frequency domain: 50 PRB Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  | Annex B.4.1 |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . |  |  |  |  |  |

Table 8.10.1.2.12-2: Minimum Performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.X TDD | OP.1TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x4 low | 70 | 12.0 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports

8.10.1.2.13.1	Minimum requirements for QCL Type C and 3 Layers Spatial Multiplexing

The requirements are specified in Table 8.10.1.2.13.1-3, with the additional parameters in Table 8.10.1.2.13.1-1 and Table 8.10.1.2.13.1-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.10.1.2.13.1-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP 1 and TP 2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.10.1.2.13.1-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 2) | Random beamforming (rank 1) |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  | Antenna ports {15,16,17,18} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16,17,18} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 4 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 4/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 4/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1-3dB |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW2 is transmitted from TP 1 and CW1 is transmitted from TP 2) |  |  |  |  |

Table 8.10.1.2.13.1-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW2 CSI-RS 1 for CW1 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.10.1.2.13.1-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.98 TDD | R.99 TDD | OP.1TDD | OP.1TDD | EPA5 | EPA5 | 4x4 Low | 70 | 11.1 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

8.10.1.2.13.2	Minimum requirements for QCL Type C and 4 Layers Spatial Multiplexing

The requirements are specified in Table 8.10.1.2.13.2-3, with the additional parameters in Table 8.10.1.2.13.2-1 and Table 8.10.1.2.13.2-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario with non-coherent joint transmission from two transmission points. The test verifies that the UE configured with quasi co-location type C performs correct tracking and compensation of the frequency and time difference between two transmission points, channel parameters estimation, channel estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. In Table 8.10.1.2.13.2-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and PDSCH, and transmission point 2 (TP 2) has different Cell ID and transmits PDSCH. In the test the PDSCH is transmitted from TP 1 and TP 2. The downlink physical channel setup for TP 1 is according to Annex C.3.2 and for TP 2 according to Annex C.3.2.

Table 8.10.1.2.13.2-1: Test Parameters

| Parameter |  | Unit | TP 1 | TP 2 |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming (rank 2) | Random beamforming (rank 2) |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1,2,3 |
| CSI reference signals 0 |  |  | Antenna ports {15,16,17,18} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |  |
| CSI reference signal 0 configuration |  |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16,17,18} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 4 |
| CSI reference signal 1 configuration |  |  |  | 8 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 4/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 4/ 0010000000000000 |
|  |  | dB | SNR1 | SNR2 = SNR1 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 |
| Timing offset relative to TP 1 |  | us | N/A | 2 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 200 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type C, ‘00’ |  |
| PDSCH transmission mode (Note 2) |  |  | 10 | 10 |
| Number of allocated resource block |  |  | 50 | 50 |
| Note 1: Note 2: PDSCH transmission is done from both TPs (CW1 is transmitted from TP 1 and CW2 is transmitted from TP 2) |  |  |  |  |

Table 8.10.1.2.13.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 8.10.1.2.13.2-3: Performance Requirements

| Test Number | Reference Channel |  | OCNG Pattern |  | Propagation Conditions (Note1) |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP 1 | TP 2 | TP 1 | TP 2 | TP 1 | TP 2 |  | Fraction of Maximum Throughput (%) | SNR1 (dB) (Note 3) |  |
| 1 | R.100 TDD | R.100 TDD | OP.1TDD | OP.1TDD | EPA5 | EPA5 | 4x4 Low | 70 | 15.4 | ≥2 |
| Note 1: The propagation conditions for TP 1 and TP 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP 1 and TP 2. Note 3: SNR corresponds to of TP 1 and TP 2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |

##### 8.10.1.2.14	HST-SFN performance

8.10.1.2.14.1	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.10.1.2.14-2, with the addition of the parameters in Table 8.10.1.2.14-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of this test is to verify UE performance in the HST-SFN-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhDemodFlag2-r16 [7] is received.

Table 8.10.1.2.14-1: Void

Table 8.10.1.2.14-2: Void

NOTE:	Table 8.10.1.2.14-1 and Table 8.10.1.2.14-2 are moved to subclause 8.10.1.2.14.1 as Table 8.10.1.2.14.1-1 and Table 8.10.1.2.14.1-2.

Table 8.10.1.2.14.1-1: Test Parameters for UE performance in HST-SFN-500 scenario (FRC)

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |
|  |  |  |  | dB |  | -3 (NOTE 1) |
|  |  |  |  | dB |  | 0 |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |
| ACK/NACK feedback mode |  |  |  |  |  | Multiplexing |
| PDSCH transmission mode |  |  |  |  |  | 3 |
| NOTE 1: . |  |  |  |  |  |  |

Table 8.10.1.2.14.1-2: Minimum performance UE in HST-SFN-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 TDD | OP.1 TDD | HST-SFN-500 | 2x4 | 70 | 6.8 | ≥ 1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1 and 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

### 8.10.2	PDCCH/PCFICH

#### 8.10.2.1	FDD

The parameters specified in Table 8.10.2.1-1 are valid for all FDD tests unless otherwise stated.

Table 8.10.2.1-1: Test Parameters for PDCCH/PCFICH with 4 Rx Antenna Ports

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1 | 1 |
| PHICH duration |  |  | Normal | Normal |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | 0 | -3 |
|  |  | dB | 0 | -3 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: according to Clause 6.9 in TS 36.211 [4]. Note 2: PDSCH is mapped as OCNG. |  |  |  |  |

##### 8.10.2.1.1	Single-antenna port performance

For the parameters specified in Table 8.10.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.1.1-1: Minimum performance PDCCH/PCFICH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 8 CCE | R.15 FDD | OP.1 FDD | ETU70 | 1x4 Low | 1 | -5.4 |

##### 8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports

For the parameters specified in Table 8.10.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8. 10.2.1.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.1.2-1: Minimum performance PDCCH/PCFICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 FDD | OP.1 FDD | EVA70 | 2 x 4 Low | 1 | -3.5 |

##### 8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports

For the parameters specified in Table 8.10.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.1.3-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.1.3-1: Minimum performance PDCCH/PCFICH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 5 MHz | 2 CCE | R.17 FDD | OP.1 FDD | EPA5 | 4 x 4 Medium A Xpol | 1 | -0.4 |

##### 8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.10.2.1.4-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.1.4-2. In Table 8.10.2.1.4-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell, respectively.

Table 8.10.2.1.4-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |
| at antenna port |  | dBm/15kHz | -98 |  |
|  |  | dB | As defined in Table 8.10.2.1.4-2 | 13.91 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 | 2 |
| CFI indicated in PCFICH |  |  | 2 | 2 |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A |
| PHICH duration |  |  | Normal | N/A |
| PDSCH TM |  |  | 4 | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |

Table 8.10.2.1.4-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 1 CCE | R.17-2 FDD | OP.1 FDD | EPA5 | EPA5 | 4x4 Low | 1 | 16.5 |
| Note 1: The OCNG pattern applies for Cell 1 and Cell 2. Note 2: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |

#### 8.10.2.2	TDD

Table 8.10.2.2-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 0 | 0 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| PHICH Ng (Note 3) |  |  | 1 | 1 |
| PHICH duration |  |  | Normal | Normal |
| Unused RE-s and PRB-s (Note 4) |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | 0 | -3 |
|  |  | dB | 0 | -3 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: according to Clause 6.9 in TS 36.211 [4]. Note 4: PDSCH is mapped as OCNG. |  |  |  |  |

##### 8.10.2.2.1	Single-antenna port performance

For the parameters specified in Table 8.10.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.2.1-1: Minimum performance PDCCH/PCFICH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 8 CCE | R.15 TDD | OP.1 TDD | ETU70 | 1x4 Low | 1 | -4.7 |

##### 8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports

For the parameters specified in Table 8.10.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.2.2-1: Minimum performance PDCCH/PCFICH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 TDD | OP.1 TDD | EVA70 | 2 x 4 Low | 1 | -3.2 |

##### 8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports

For the parameters specified in Table 8.10.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.2.3-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.2.2.3-1: Minimum performance PDCCH/PCFICH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 5 MHz | 2 CCE | R.17 TDD | OP.1 TDD | EPA5 | 4 x 4 Medium A Xpol | 1 | 0.0 |

##### 8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer

The purpose of this test is to verify the Enhanced Downlink Control Channel Performance Requirement Type A for PDCCH/PCFICH with 4 transmit antennas for the case of dominant interferer with the non-colliding CRS pattern and applying interference model defined in clause B.7.1.

For the parameters specified in Table 8.10.2.2.4-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.2.2.4-2. In Table 8.10.2.2.4-1, Cell 1 is the serving cell, and Cell 2 is the aggressor cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell, respectively.

Table 8.10.2.2.4-1: Test Parameters for PDCCH/PCFICH

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA OCNG_RA | dB | -3 | -3 |
|  | PHICH_RA | dB | -3 | N/A |
|  | PCFICH_RB PDCCH_RB OCNG_RB | dB | -3 | -3 |
|  | PHICH_RB | dB | -3 | N/A |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |
| at antenna port |  | dBm/15kHz | -98 |  |
|  |  | dB | As defined in Table 8.10.2.1.4-2 | 13.91 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| UL/DL Configuration |  |  | 0 | 0 |
| Special Subframe Configuration |  |  | 4 | 4 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Number of DL control region OFDM symbols |  |  | 2 |  |
| CFI indicated in PCFICH |  |  | 2 |  |
| PHICH Ng (Note 1) |  |  | 1/6 | N/A |
| PHICH duration |  |  | Normal | N/A |
| PDSCH TM |  |  | 4 | N/A |
| Interference model |  |  | NA | As specified in clause B.7.1 |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG |
| Time Offset relative to Cell 1 |  | s | N/A | 2 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 |
| Note 1: According to Clause 6.9 in TS 36.211 [4]. Note 2: For Cell 2 unused RE-s and PRB-s do not include control region REs. |  |  |  |  |

Table 8.10.2.1.4-2: Minimum Performance for PDCCH/PCFICH for Enhanced Downlink Control Channel Performance Requirement Type A

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 1 CCE | R.17-2 TDD | OP.1 TDD | EPA5 | EPA5 | 4x4 Low | 1 | 17.9 |
| Note 1: The OCNG pattern applies for Cell 1 and Cell 2. Note 2: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1 and Cell 2. Note 4: SNR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |

### 8.10.3	PHICH

The receiver characteristics of the PHICH are determined by the probability of miss-detecting an ACK for a NACK (Pm-an). It is assumed that there is no bias applied to the detection of ACK and NACK (zero-threshold delection).

#### 8.10.3.1		FDD

The parameters specified in Table 8.10.3.1-1 are valid for all FDD tests with 4Rx unless otherwise stated.

Table 8.10.3.1-1: Test Parameters for PHICH with 4 Rx Antenna Ports

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | -3 |
|  |  | dB | 0 | -3 |
|  |  | dB | 0 | 0 |
| PHICH duration |  |  | Normal | Normal |
| PHICH Ng (Note 1) |  |  | Ng = 1 | Ng = 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. DL Grant: Note 2 |  |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Note 1: according to Clause 6.9 in TS 36.211 [4]. Note 2: Reference measurement channel R.15 FDD for Single Tx Antenna 8.10.3.1.1, R15-2 FDD for Transmit Diversity with 2Tx Antenna Port 8.10.3.1.2, R17 FDD for Transmit diversity with 4Tx Antenna Port 8.10.3.1.3, according to Table A.3.5.1-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 for the serving cell. |  |  |  |  |

##### 8.10.3.1.1	Single Tx Antenna Port performance

For the parameters specified in Table 8.10.3.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.10.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.1.1-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.18 | OP.1 FDD | ETU70 | 1 x 4 Low | 0.1 | 1.6 |

##### 8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports

For the parameters specified in Table 8.10.3.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.10.3.1.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.1.2-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.19 | OP.1 FDD | EVA70 | 2 x 4 Low | 0.1 | 0.6 |

##### 8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports

For the parameters specified in Table 8.10.3.1-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8 .10.3.1.3-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.1.3-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 5 MHz | R.20 | OP.1 FDD | EPA5 | 4 x 4 Medium correlation A, Cross polarized | 0.1 | 0.1 |

#### 8.10.3.2	TDD

The parameters specified in Table 8.10.3.2-1 are valid for all TDD tests with 4 Rx unless otherwise stated.

Table 8.10.3.2-1: Test Parameters for PHICH with 4 Rx Antenna Ports

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 1 | 1 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Downlink power allocation |  | dB | 0 | -3 |
|  |  | dB | 0 | -3 |
|  |  | dB | 0 | 0 |
| PHICH duration |  |  | Normal | Normal |
| PHICH Ng (Note 1) |  |  | Ng = 1 | Ng = 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. DL Grant: Note 4 |  |
| Unused RE-s and PRB-s (Note 4) |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: according to Clause 6.9 in TS 36.211 [4]. Note 4: Reference measurement channel R.15 TDD for Single Tx Antenna 8.10.3.2.1, R15-2 TDD for Transmit Diversity with 2Tx Antenna Port 8.10.3.2.2, R17 TDD for Transmit diversity with 4Tx Antenna Port 8.10.3.2.3, according to Table A.3.5.2-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 for the serving cell |  |  |  |  |

##### 8.10.3.2.1	Single Tx Antenna Port performance

For the parameters specified in Table 8.10.3.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.10.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.2.1-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.18 | OP.1 TDD | ETU70 | 1 x 4 Low | 0.1 | 1.7 |

##### 8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports

For the parameters specified in Table 8.10.3.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.10.3.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.2.2-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 10 MHz | R.19 | OP.1 TDD | EVA70 | 2 x 4 Low | 0.1 | 0.9 |

##### 8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports

For the parameters specified in Table 8.10.3.2-1 the average probability of a miss-detecting ACK for NACK (Pm-an) shall be below the specified value in Table 8.10.3.2.3-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.3.2.3-1: Minimum performance PHICH with 4 Rx Antenna Ports

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-an (%) | SNR (dB) |
| 1 | 5 MHz | R.20 | OP.1 TDD | EPA5 | 4 x 4 Medium cotrrelation A, Cross polarized | 0.1 | 0.3 |

### 8.10.4	ePDCCH

The receiver characteristics of the EPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). For the distributed transmission tests in 8.10.4.1, EPDCCH and PCFICH are tested jointly, i.e. a miss detection of PCFICH implies a miss detection of EPDCCH. For other tests, EPDCCH and PCFICH are not tested jointly.

#### 8.10.4.1	Distributed Transmission with 4Rx

##### 8.10.4.1.1	FDD

The parameters specified in Table 8.10.4.1.1-1 are valid for all FDD distributed EPDCCH test with 4Rx unless otherwise stated.

Table 8.10.4.1.1-1: Test Parameters for Distributed EPDCCH with 4Rx

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of EPDCCH Sets Configured |  |  | 2 (Note 2) |
| Number of PRB per EPDCCH Set |  |  | 4 (1st Set) 8 (2nd Set) |
| EPDCCH Subframe Monitoring |  |  | NA |
| PDSCH TM |  |  | TM3 |
| DCI Format |  |  | 2A |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: The two sets are distributed EPDCCH sets and non-overlapping with PRB = {3, 17, 31, 45} for the first set and PRB = {0, 7, 14, 21, 28, 35, 42, 49} for the second set. EPDCCH is scheduled in the first set for Test 1 and second set for Test 2, respectively. Both sets are always configured. |  |  |  |

For the parameters specified in Table 8.10.4.1.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.4.1.1-2. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.4.1.1-2: Minimum performance Distributed EPDCCH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55 FDD | OP.7 FDD | EVA5 | 2 x 4 Low | 1 | -0.7 |
| 2 | 10 MHZ | 16 ECCE | R.56 FDD | OP.7 FDD | EVA70 | 2 x 4 Low | 1 | -5.8 |

##### 8.10.4.1.2	TDD

The parameters specified in Table 8.10.4.1.2-1 are valid for all TDD distributed EPDCCH tests with 4Rx unless otherwise stated.

Table 8.10.4.1.2-1: Test Parameters for Distributed EPDCCH with 4Rx

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of EPDCCH Sets Configured |  |  | 2 (Note 2) |
| Number of PRB per EPDCCH Set |  |  | 4 (1st Set) 8 (2nd Set) |
| EPDCCH Subframe Monitoring |  |  | NA |
| PDSCH TM |  |  | TM3 |
| DCI Format |  |  | 2A |
| TDD UL/DL Configuration |  |  | 0 |
| TDD Special Subframe |  |  | 1 (Note 3) |
| Note 1: The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11 is not configured. Note 2: The two sets are distributed EPDCCH sets and non-overlapping with PRB = {3, 17, 31, 45} for the first set and PRB = {0, 7, 14, 21, 28, 35, 42, 49} for the second set. EPDCCH is scheduled in the first set for Test 1 and second set for Test 2, respectively. Both sets are always configured. Note 3: Demodulation performance is averaged over normal and special subframe. |  |  |  |

For the parameters specified in Table 8.10.4.1.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.4.1.2-2. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.4.1.2-2: Minimum performance Distributed EPDCCH with 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 ECCE | R.55 TDD | OP.7 TDD | EVA5 | 2 x 4 Low | 1 | -0.7 |
| 2 | 10 MHZ | 16 ECCE | R.56 TDD | OP.7 TDD | EVA70 | 2 x 4 Low | 1 | -5.8 |

#### 8.10.4.2	Localized Transmission with TM9 and 4Rx

##### 8.10.4.2.1	FDD

The parameters specified in Table 8.10.4.2.1-1 are valid for all FDD TM9 localized ePDCCH tests with 4Rx unless otherwise stated.

Table 8.10.4.2.1-1: Test Parameters for Localized EPDCCH with TM9 and 4Rx

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1111111110 1111111101 1111111011 1111110111 (Note 3) |
| PDSCH TM |  |  | TM9 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. |  |  |  |

For the parameters specified in Table 8.10.4.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.4.2.1-2. EPDCCH subframe monitoring is configured and the subframe monitoring requirement in EPDCCH restricted subframes is statDTX of 99.9%.

The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.4.2.1-2: Minimum performance Localized EPDCCH with TM9 and 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 FDD | OP.7 FDD | EVA5 | 2 x 4 Low | 1 | 6.5 |
| 2 | 10 MHZ | 8 ECCE | R.58 FDD | OP.7 FDD | EVA5 | 2 x 4 Low | 1 | -1.5 |

##### 8.10.4.2.2	TDD

The parameters specified in Table 8.10.4.2.2-1 are valid for all TDD TM9 localized ePDCCH tests unless otherwise stated.

Table 8.10.4.2.2-1: Test Parameters for Localized EPDCCH with TM9 and 4Rx

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 0 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 0 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1100011000 1100010000 1100011000 1100001000 1100011000 1000011000 1100011000 (Note 3) |
| PDSCH TM |  |  | TM9 |
| TDD UL/DL Configuration |  |  | 0 |
| TDD Special Subframe |  |  | 1 (Note 4) |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. Note 4: Demodulation performance is averaged over normal and special subframe. |  |  |  |

For the parameters specified in Table 8.10.4.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.10.4.2.2-2. EPDCCH subframe monitoring is configured and the subframe monitoring requirement in EPDCCH restricted subframes is statDTX of 99.9%.

The downlink physical setup is in accordance with Annex C.3.2.

Table 8.10.4.2.2-2: Minimum performance Localized EPDCCH with TM9 and 4Rx Antenna ports

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 2 ECCE | R.57 TDD | OP.7 TDD | EVA5 | 2 x 4 Low | 1 | 6.0 |
| 2 | 10 MHz | 8 ECCE | R.58 TDD | OP.7 TDD | EVA5 | 2 x 4 Low | 1 | -2.1 |

## 8.11	Demodulation (UE supporting coverage enhancement)

The requirements for UE DL Category M1 in this sub-clause are defined based on the simulation results with UE DL Category M1 unless otherwise stated.

The requirements for UE DL Category M2 in this sub-clause are defined based on the simulation results with UE DL Category M2 unless otherwise stated.

The requirements of UE DL Category M1 in this sub-clause are applicable for UE DL Category M2, UE DL Category 1bis and Category 0, as specified in the applicability rule in the sub-clause 8.1.2.8A.

The requirements of UE DL Category M2 in this sub-clause are applicable for UE DL Category 1bis and Category 0, as specified in the applicability rule in the sub-clause 8.1.2.8A.

### 8.11.1	PDSCH

#### 8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)

The parameters specified in Table 8.11.1.1-1 are valid for FDD and half-duplex FDD tests unless otherwise stated.

Table 8.11.1.1-1: Common Test Parameters (FDD and half-duplex FDD)

| Parameter | Unit | CE Mode A | CE Mode B |
| --- | --- | --- | --- |
| Inter-TTI Distance |  | 1 | 1 |
| Number of HARQ processes per component carrier | Processes | 8 or 10 or 14 (Note 2) | 2 |
| Maximum number of HARQ transmission |  | 4 | 4 |
| Redundancy version coding sequence rvidx (Note 1) |  | {0, 2, 3, 1} for QPSK and 16QAM | {0,0,0,0,2,2,2,2,3,3,3,3,1,1,1,1…} for QPSK |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |
| Cyclic Prefix |  | Normal | Normal |
| Beamforming Precoder for MPDCCH |  | Annex B.4.4 | Annex B.4.4 |
| Precoder update granularity for MPDCCH |  | Frequency domain: 1 PRB Time domain: identical during the hopping period (interval-FDD for CE Mode A) | Frequency domain: 1 PRB Time domain: identical during the hopping period (interval-FDD for CE Mode B) |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  | 1111111111 | 1111111111 |
| HARQ bundling(ce-HARQ-AckBundling) |  | For test 7 in clause 8.11.1.1.3.1, enabled,otherwise, disabled | Disabled |
| Note 1: rvidx is defined in TS 36.213 [6] Table 7.1.7.1-2. Note 2: For the tests except test 7 in clause 8.11.1.1.3.1, for UE supporting ce-pdsch-tenProcesses-r13, the number of HARQ processese are set to 10, otherwise, it is set to 8.For the test 7 in clause 8.11.1.1.3.1, the number of HARQ processes is set to 14. |  |  |  |

##### 8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)

8.11.1.1.1.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.11.1.1.1.1-2, with the addition of the parameters in Table 8.11.1.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.11.1.1.1.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  |
|  |  |  | dB |  | 0 |  |
|  | δ |  | dB |  | 3 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  |
| Precoding granularity |  |  | PRB |  | 6 |  |
| PMI delay (Note 2) |  |  | ms |  | 10 |  |
| Reporting interval |  |  | ms |  | 10 |  |
| Reporting mode |  |  |  |  | PUCCH 1-1 |  |
| Physical channel for CQI reporting |  |  |  |  | PUSCH(Note4) |  |
| cqi-pmi-ConfigIndex |  |  |  |  | 12 |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 001111 |  |
| PDSCH transmission mode |  |  |  |  | 6 |  |
| Coverage enhancement mode |  |  |  |  | CE Mode A |  |
| OFDM starting symbol (startSymbolBR) |  |  |  |  | 2 |  |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  |  |  | 1 |  |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  |  |  | Disabled |  |
| MPDCCH transmission duration |  |  | ms |  | 1 |  |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  |  |  | 1 |  |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  |  |  | 1 |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), This reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For each test, DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |

Table 8.11.1.1.1.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70% | 8.6 |

8.11.1.1.1.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information

The requirements are specified in Table 8.11.1.1.1.2-2, with the addition of parameters in Table 8.11.1.1.1.2-1. In Table 8.11.1.1.1.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single-layer spatial multiplexing TM6 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.1.1.1.2-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 6 | N/A | N/A |
| Coverage enhancement mode |  |  | CE Mode A | N/A | N/A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | N/A | N/A |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  | 1 | N/A | N/A |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | N/A | N/A |
| MPDCCH transmission duration |  | ms | 1 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | N/A | N/A |
| Precoding granularity |  | PRB | 6 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 10 | N/A | N/A |
| Reporting interval |  | ms | 10 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note4) |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 10 | 10 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF#(n-4). This reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For each test, DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |  |

Table 8.11.1.1.1.2-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.79 FDD | OP.1FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x1 Low | 70 | 12.5 | M2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

8.11.1.1.1.3	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information

The requirements are specified in Table 8.11.1.1.1.3-2, with the addition of parameters in Table 8.11.1.1.1.3-1. In Table 8.11.1.1.1.3-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single-layer spatial multiplexing TM6 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.11.1.1.1.3-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 6 | N/A | N/A |
| Coverage enhancement mode |  |  | CE Mode A | N/A | N/A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | N/A | N/A |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  | 1 | N/A | N/A |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | N/A | N/A |
| MPDCCH transmission duration |  | ms | 1 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | N/A | N/A |
| Precoding granularity |  | PRB | 6 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 10 | N/A | N/A |
| Reporting interval |  | ms | 10 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note4) |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF#(n-4). This reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For each test, DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |  |

Table 8.11.1.1.1.3-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.95 FDD | OP.1FDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x1 Low | 70 | 11.4 | M2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)

8.11.1.1.2.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna ports 7 or 8 upon detection of a MPDCCH with DCI format 6-1A, the requirements are specified in Table 8.11.1.1.2.1-2 with the addition of the parameters in Table 8.11.1.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8.

Table 8.11.1.1.2.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer)

| parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
|  | δ | dB | 0 | 0 | 0 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 (Note 6) | -98 (Note 6) | -98 (Note 6) |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 | 6 | 6 |
| Simultaneous transmission |  |  | No | No | No |
| PDSCH transmission mode |  |  | 9 | 9 | 9 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions for PDSCH (pdsch-maxNumRepetitionCEmodeA) |  |  | 16 | 16 | Not configured |
| PDSCH repetition number |  |  | 8 | 4 | 2 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | 1 | 1 |
| Frequency hopping interval (interval-FDD) |  | ms | 8 | 2 | 1 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 8 | 2 | 1 |
| MPDCCH repetition number |  |  | 8 | 2 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 4 | 8 | 10 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: For each test, DC subcarrier puncturing shall be considered. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |

Table 8.11.1.1.2.1-2: Minimum performance for CDM-multiplexed DM RS (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/3 | R.80 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | -2.0 | M1 |
| 2 | 10MHz QPSK 1/3 | R.80-1 FDD | OP.2 FDD | EPA5 | 2x2 Low | 70 | -4.0 | ≥1 |
| 3 | 10MHz QPSK 1/3 | R.80-2 FDD | OP.2 FDD | EPA5 | 2x4 Low | 70 | -4.0 | ≥1 |

8.11.1.1.2.2	(Void)

8.11.1.1.2.3	(Void)

##### 8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)

8.11.1.1.3.1	Minimum Requirement 2 Tx Antenna Port supporting narrowband transmission

The requirements are specified in Table 8.11.1.1.3.1-2, with the addition of the parameters in Table 8.11.1.1.3.1-1 and Table 8.11.1.1.3.1-1a, and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.1.3.1-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 (Note 3) | Test 2 (Note 3) | Test 2a (Note 3) | Test 3 (Note 3) | Test 4 (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Coverage enhancement mode |  |  | CE Mode B | CE Mode A | CE Mode A | CE Mode B | CE Mode B |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 64 | 1 | 1 | 32 | 16 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Disabled | Disabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | N/A | N/A | 1 | 1 |
| Frequency hopping interval (interval-FDD) |  | ms | 16 | N/A | N/A | 8 | 4 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 64 | 1 | 1 | 32 | 8 |
| MPDCCH repetition number |  |  | 64 | 1 | 1 | 32 | 8 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 4 | N/A | N/A | 4 | 4 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 2.5 | 1 | 1 | 2.5 | 4 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 0 | 0 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 8 | 8 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 6 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: Test 1, test 3 and test 4 are applicable for UE supporting CE Mode B. Test 2 is applicable for UE not supporting CE Mode B. Test 2a is applicable for UE not supporting CE Mode B and UE capable of ce-CRS-IntfMitig. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |  |

Table 8.11.1.1.3.1-1a: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 5 | Test 6 | Test 7 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode A |
| PDSCH transmission mode |  |  | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 1 | 1 | 1 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | N/A | N/A | N/A |
| Frequency hopping interval (interval-FDD) |  | ms | N/A | N/A | N/A |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 4 | 1 | 1 |
| MPDCCH repetition number |  |  | 4 | 1 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | N/A | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | 1 | 1 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 | 0 |
| MPDCCH aggregation level |  |  | 8 | 8 | 1 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Disabled |
| Activation of 64QAM for non-repeated PDSCH in CE Mode A (ce-PDSCH-64QAM-Config) |  |  | Disabled | Enabled | Enabled |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 6 is applicable for UE capable of ce-PDSCH-64QAM. Note 5: Test 7 is applicable for UE capable of at least one of ce-PDSCH-14HARQProcesses-r17 and ce-PDSCH-14HARQProcesses-Alt2-r17. Test 7 is applicable for UE in half-duplex FDD opreation |  |  |  |  |  |

Table 8.11.1.1.3.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/10 | R.81 FDD | OP.2 FDD | ETU1 | 2x1 Low | 70 | -13.5 | M1 |
| 2 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.4 | M1 |
| 2a | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.6 | M1 |
| 3 | 10MHz QPSK 1/10 | R.81-1 FDD | OP.2 FDD | ETU1 | 2x2 Low | 70 | -13.3 | ≥1 |
| 4 | 10MHz QPSK 1/10 | R.81-2 FDD | OP.2 FDD | ETU1 | 2x4 Low | 70 | -13.3 | ≥1 |
| 5 | 10MHz QPSK 1/3 | R.103 FDD | OP.2 FDD | EPA200 | 2x1 Low | 70 | 3.8 | M1 |
| 6 | 10MHz 64QAM 0.4 | R.104 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 12.9 | M1 |
| 7 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.4 | M1 |

8.11.1.1.3.2	Minimum Requirement 2 Tx Antenna Port supporting wideband transmission

The requirements are specified in Table 8.11.1.1.3.2-2, with the addition of the parameters in Table 8.11.1.1.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.1.3.2-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 | Test 2 | Test 2a (Note 4) | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode B | CE Mode B | CE Mode A | CE Mode A |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 8 | 32 | 32 | 4 | 2 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 5 | 5 | 5 | N/A | N/A |
| Frequency hopping interval (interval-FDD) |  | ms | 4 | 16 | 16 | N/A | N/A |
| Maximum number of MPDCCH repetitions (mpdcch-NumRepetition) |  |  | 16 | 64 | 64 | 8 | 2 |
| MPDCCH transmission duration |  | ms | 16 | 64 | 64 | 8 | 2 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 2 | 2 | 2 | 2.5 | 5 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 | 0 | 0 | 0 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 24 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 2a is applicable for UE supporting CE Mode B and UE capable of ce-CRS-IntfMitig. |  |  |  |  |  |  |  |

Table 8.11.1.1.3.2-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/3 | R.90 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | -5.4 | M2 |
| 2 | 10MHz QPSK 1/10 | R.91 FDD | OP.2 FDD | ETU1 | 2x1 Low | 70 | -13.1 | M2 |
| 2a | 10MHz QPSK 1/10 | R.91 FDD | OP.2 FDD | ETU1 | 2x1 Low | 70 | -12.9 | M2 |
| 3 | 10MHz QPSK 1/2 | R.92-1 FDD | OP.2 FDD | EPA5 | 2x2 Low | 70 | -4.8 | ≥1 |
| 4 | 10MHz QPSK 1/2 | R.92-2 FDD | OP.2 FDD | EPA5 | 2x4 Low | 70 | -4.9 | ≥1 |

#### 8.11.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.11.1.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.11.1.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | CE Mode A |  |  | CE Mode B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  | Normal |  |
| Cell ID |  |  |  | 0 |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  | 2 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  | 4 |  |
| Redundancy version coding sequence rvidx (Note 3) |  |  |  | {0, 2, 3, 1} for QPSK and 16QAM |  | {0,0,0,0,0,0,0,0,0,0,2,2,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,3,3,3,1,1,1,1,1,1,1,1,1,1} for QPSK |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |  | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  | Multiplexing |  |
| Beamforming Precoder for MPDCCH |  |  |  | Annex B.4.4 |  | Annex B.4.4 |  |
| Precoder update granularity for MPDCCH |  |  |  | Frequency domain: 1 PRB Time domain: identical during the hopping period (interval-TDD for CE Mode A) |  | Frequency domain: 1 PRB Time domain: identical during the hopping period (interval-TDD for CE Mode B) |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  |  | 1011110111 |  | 1011110111 |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: rvidx is defined in TS 36.213 [6] Table 7.1.7.1-2. |  |  |  |  |  |  |  |

##### 8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)

8.11.1.2.1.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port

The requirements are specified in Table 8.11.1.2.1.1-2, with the addition of the parameters in Table 8.11.1.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-one performance with frequency selective precoding.

Table 8.11.1.2.1.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  |
|  |  |  | dB |  | -3 (Note 1) |  |
|  |  |  | dB |  | 0 |  |
|  | δ |  | dB |  | 3 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  |
| Precoding granularity |  |  | PRB |  | 6 |  |
| PMI delay (Note 2) |  |  | ms |  | 10 or 11 |  |
| Reporting interval |  |  | ms |  | 5 |  |
| Reporting mode |  |  |  |  | PUCCH 1-1 |  |
| cqi-pmi-ConfigIndex |  |  |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 001111 |  |
| ACK/NACK feedback mode |  |  |  |  | Multiplexing |  |
| Physical channel for CQI reporting |  |  |  |  | PUSCH (Note 3) |  |
| PDSCH transmission mode |  |  |  |  | 6 |  |
| Coverage enhancement mode |  |  |  |  | CE Mode A |  |
| OFDM starting symbol (startSymbolBR) |  |  |  |  | 2 |  |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  |  |  | 1 |  |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  |  |  | Disabled |  |
| MPDCCH transmission duration |  |  | ms |  | 1 |  |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  |  |  | 1 |  |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  |  |  | 1 |  |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 4: For each test, DC subcarrier puncturing shall be considered. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |

Table 8.11.1.2.1.1-2: Minimum performance Single-Layer Spatial Multiplexing (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10MHz 16QAM 1/2 | R.79 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70% | 11.4 |

8.11.1.2.1.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information

The requirements are specified in Table 8.11.1.2.1.2-2, with the addition of parameters in Table 8.11.1.2.1.2-1. In Table 8.11.1.2.1.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single-layer spatial multiplexing TM6 performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.1.2.1.2-1: Test Parameters

| Parameter |  | Unit | Cell 1 (Option 1) | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 6 | N/A | N/A |
| Coverage enhancement mode |  |  | CE Mode A | N/A | N/A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | N/A | N/A |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  | 1 | N/A | N/A |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | N/A | N/A |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | N/A | N/A | N/A |
| Frequency hopping interval (interval-TDD) |  | ms | N/A | N/A | N/A |
| MPDCCH transmission duration |  | ms | 1 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | N/A | N/A |
| Precoding granularity |  | PRB | 6 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 10 or 11 | N/A | N/A |
| Reporting interval |  | ms | 5 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note4) |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 10 | 10 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF #(n-4). This reported PMI cannot be applied at the eNB downlink before SF #(n+4). Note 3: For each test, DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |

Table 8.11.1.2.1.2-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.79 TDD | OP.1TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 2x1 Low | 70 | 11.5 | M2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

8.11.1.2.1.3	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information

The requirements are specified in Table 8.11.1.2.1.3-2, with the addition of parameters in Table 8.11.1.2.1.3-1. In Table 8.11.1.2.1.3-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the closed loop single-layer spatial multiplexing TM6 performance under assumption that UE applies CRS interference mitigation in the scenario with 4 CRS antenna ports in the serving and aggressor cells.

Table 8.11.1.2.1.3-1: Test Parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -6 | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | N/A | 10.45 | 4.6 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell ID |  |  | 0 | 1 | 6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |  |  |
| Number of control OFDM symbols (CFI) |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  | 6 | N/A | N/A |
| Coverage enhancement mode |  |  | CE Mode A | N/A | N/A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | N/A | N/A |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  | 1 | N/A | N/A |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | N/A | N/A |
| MPDCCH transmission duration |  | ms | 1 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | N/A | N/A |
| Precoding granularity |  | PRB | 6 | N/A | N/A |
| PMI delay (Note 2) |  | ms | 10 or 11 | N/A | N/A |
| Reporting interval |  | ms | 5 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note4) |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A | N/A |
| Time Offset relative to Cell 1 |  | s | N/A | 3 | -1 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference model |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 | 80 |
|  | Rank 2 | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF #n based on PMI estimation at a downlink SF not later than SF#(n-4). This reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For each test, DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |

Table 8.11.1.2.1.3-2: Minimum performance for PDSCH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.95 TDD | OP.1TDD | N/A | N/A | EPA5 | EPA5 | EPA5 | 4x1 Low | 70 | 11.1 | M2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)

8.11.1.2.2.1	Single-layer Spatial Multiplexing

For single-layer transmission on antenna ports 7 or 8 upon detection of a PDCCH with DCI format 6-1A, the requirements are specified in Table 8.11.1.2.2.1-2 with the addition of the parameters in Table 8.11.1.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify rank-1 performance on one of the antenna ports 7 or 8.

Table 8.11.1.2.2.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 | -3 |
|  | δ | dB | 0 | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 | 6 | 6 |
| Simultaneous transmission |  |  | No | No | No |
| PDSCH transmission mode |  |  | 9 | 9 | 9 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions for PDSCH (pdsch-maxNumRepetitionCEmodeA) |  |  | Not configured | Not configured | Not configured |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled |
| PDSCH repetition number |  |  | 8 | 4 | 2 |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | 1 | 1 |
| Frequency hopping interval (interval-TDD) |  | ms | 10 | 1 | 1 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 8 | 2 | 1 |
| MPDCCH repetition number |  |  | 8 | 2 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 10 | 20 | 20 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: For each test, DC subcarrier puncturing shall be considered. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |

Table 8.11.1.2.2.1-2: Minimum performance for CDM-multiplexed DM RS without simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/3 | R.80 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | -2.5 | M1 |
| 2 | 10MHz QPSK 1/3 | R.80-1 TDD | OP.2 TDD | EPA5 | 2x2 Low | 70 | -3.6 | ≥1 |
| 3 | 10MHz QPSK 1/3 | R.80-2 TDD | OP.2 TDD | EPA5 | 2x4 Low | 70 | -3.7 | ≥1 |

##### 8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)

8.11.1.2.3.1	Minimum Requirement 2 Tx Antenna Port supporting narrowband transmission

The requirements are specified in Table 8.11.1.2.3.1-2, with the addition of the parameters in Table 8.11.1.2.3.1-1 and Table 8.11.1.2.3.1-1a, and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.2.3.1-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 (Note 3) | Test 2 (Note 3) | Test 2a (Note 3) | Test 3 (Note 3) | Test 4 (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing | Multiplexing | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| Coverage enhancement mode |  |  | CE Mode B | CE Mode A | CE Mode A | CE Mode B | CE Mode B |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 64 | 1 | 1 | 32 | 16 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Disabled | Disabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | N/A | N/A | 1 | 1 |
| Frequency hopping interval (interval-TDD) |  | ms | 20 | N/A | N/A | 20 | 10 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 64 | 1 | 1 | 32 | 8 |
| MPDCCH repetition number |  |  | 64 | 1 | 1 | 32 | 8 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 4 | N/A | N/A | 4 | 4 |
| Starting subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 8 | 1 | 1 | 8 | 10 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 0 | 0 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 8 | 8 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 6 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: Test 1, test 3 and test 4 are applicable for UE supporting CE Mode B. Test 2 is applicable for UE not supporting CE Mode B. Test 2a is applicable for UE not supporting CE Mode B and UE capable of ce-CRS-IntfMitig. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |  |

Table 8.11.1.2.3.1-1a: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 5 | Test 6 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
|  | δ | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 2 | 2 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A |
| PDSCH transmission mode |  |  | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured |
| PDSCH repetition number |  |  | 1 | 1 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | N/A | N/A |
| Frequency hopping interval (interval-TDD) |  | ms | N/A | N/A |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 4 | 1 |
| MPDCCH repetition number |  |  | 4 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 1 | 1 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 |
| MPDCCH aggregation level |  |  | 8 | 8 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled |
| Activation of 64QAM for non-repeated PDSCH in CE Mode A (ce-PDSCH-64QAM-Config) |  |  | Disabled | Enabled |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 6 is applicable for UE capable of ce-PDSCH-64QAM. |  |  |  |  |

Table 8.11.1.2.3.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/10 | R.81 TDD | OP.2 TDD | ETU1 | 2x1 Low | 70 | -14.0 | M1 |
| 2 | 10MHz 16QAM 1/2 | R.79 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 9.6 | M1 |
| 2a | 10MHz 16QAM 1/2 | R.79 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 9.8 | M1 |
| 3 | 10MHz QPSK 1/10 | R.81-1 TDD | OP.2 TDD | ETU1 | 2x2 Low | 70 | -13.3 | ≥1 |
| 4 | 10MHz QPSK 1/10 | R.81-2 TDD | OP.2 TDD | ETU1 | 2x4 Low | 70 | -13.3 | ≥1 |
| 5 | 10MHz QPSK 1/3 | R.103 TDD | OP.2 TDD | EPA200 | 2x1 Low | 70 | 3.5 | M1 |
| 6 | 10MHz 64QAM 0.4 | R.104 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 13.5 | M1 |

8.11.1.2.3.2	Minimum Requirement 2 Tx Antenna Port supporting wideband transmission

The requirements are specified in Table 8.11.1.2.3.2-2, with the addition of the parameters in Table 8.11.1.2.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.2.3.2-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 | Test 2 | Test 2a (Note 4) | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing | Multiplexing | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode B | CE Mode B | CE Mode A | CE Mode A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 8 | 32 | 32 | 4 | 2 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 5 | 5 | 5 | N/A | N/A |
| Frequency hopping interval (interval-TDD) |  | ms | 5 | 20 | 20 | N/A | N/A |
| Maximum number of repetitions (mpdcch-NumRepetition) |  |  | 16 | 32 | 32 | 4 | 2 |
| MPDCCH transmission duration |  | ms | 16 | 32 | 32 | 4 | 2 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 5 | 8 | 8 | 8 | 10 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 | 0 | 0 | 0 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 24 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 2a is applicable for UE supporting CE Mode B and UE capable of ce-CRS-IntfMitig. |  |  |  |  |  |  |  |

Table 8.11.1.2.3.2-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/3 | R.90 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | -5.6 | M2 |
| 2 | 10MHz QPSK 1/10 | R.91 TDD | OP.2 TDD | ETU1 | 2x1 Low | 70 | -13.6 | M2 |
| 2a | 10MHz QPSK 1/10 | R.91 TDD | OP.2 TDD | ETU1 | 2x1 Low | 70 | -13.4 | M2 |
| 3 | 10MHz QPSK 1/2 | R.92-1 TDD | OP.2 TDD | EPA5 | 2x2 Low | 70 | -4.9 | ≥1 |
| 4 | 10MHz QPSK 1/2 | R.92-2 TDD | OP.2 TDD | EPA5 | 2x4 Low | 70 | -5.0 | ≥1 |

### 8.11.2	MPDCCH

The receiver characteristics of the MPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

#### 8.11.2.1	FDD and half-duplex FDD

Table 8.11.2.1-1: Test Parameters for MPDCCH (Category M1)

| Parameter |  | Unit | CE Mode A (Test 1) | CE Mode B (Test 1) |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 |
|  |  | ms | 4 (Note 2) | 16 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Disabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 16 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | 1.5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | 32 | 64 |
| MPDCCH repetition number |  |  | 32 | 64 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 |
| Note1: For each test, DC subcarrier puncturing shall be considered. Note2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval. Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |

Table 8.11.2.1-2: Test Parameters for MPDCCH (Category ≥1)

| Parameter |  | Unit | CE Mode A (Test 2) | CE Mode A (Test 3) | CE Mode B (Test 2) | CE Mode B (Test 3) |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 | 0 | 0 |
|  |  | dB | -3 | 0 | 0 | 0 |
|  |  | dB | 0 | -3 | -3 | -3 |
|  | δ | dB | 3 | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 | 1 | 1 |
|  |  | ms | 4(Note 2) | 4(Note 2) | 16 (Note 2) | 16 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 4 | 2+4 | 2+4 |
| Transmission type |  |  | Distributed | Distributed | Localized | Localized |
| Frequency hopping |  |  | Disabled | Disabled | Enabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | 4 | 4 |
| Frequency hopping offset |  |  | N/A | N/A | 1 | 1 |
| Frequency hopping interval |  | ms | N/A | N/A | 8 | 2 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) Note 3 |  |  | 1.5 | 1.5 | 1.5 | 1.5 |
| Maximum number of repetitions(mPDCCH-NumRepetition) |  |  | 8 | 2 | 32 | 8 |
| MPDCCH repetition number |  |  | 8 | 2 | 32 | 8 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 1 | 7 | 7 |
| PDSCH TM |  |  | TM2 | TM2 | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1A | 6-1B | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 | 1111111111 | 1111111111 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval. Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |

##### 8.11.2.1.1	CE Mode A

For the parameters specified in Table 8.11.2.1-1 and 8.11.2.1-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.1.1-1: Minimum performance CE Mode A MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 16 ECCE | R.82 FDD | OP.2 FDD | EPA5 | 2 x 1 Low | 1 | -4.8 | M1 |
| 2 | 10 MHz | 16 ECCE | R.82 FDD | OP.2 FDD | EPA5 | 2 x 2 Low | 1 | -6.5 | ≥1 |
| 3 | 10 MHz | 16 ECCE | R.82 FDD | OP.2 FDD | EPA5 | 2 x 4 Low | 1 | -6.5 | ≥1 |

##### 8.11.2.1.2	CE Mode B

For the parameters specified in Table 8.11.2.1-1 and 8.11.2.1-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.1.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.1.2-1: Minimum performance CE Mode B MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 24 ECCE | R.83 FDD | OP.2 FDD | ETU1 | 2 x 1 Low | 1 | -12 | M1 |
| 2 | 10 MHz | 24 ECCE | R.83 FDD | OP.2 FDD | ETU1 | 2 x 2 Low | 1 | -13.2 | ≥1 |
| 3 | 10 MHz | 24 ECCE | R.83 FDD | OP.2 FDD | ETU1 | 2 x 4 Low | 1 | -12.8 | ≥1 |

##### 8.11.2.1.3	CE Mode A with TM9 interference model

The requirements are specified in Table 8.11.2.1.3-2, with the addition of parameters in Table 8.11.2.1.3-1. In Table 8.11.2.1.3-2, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the MPDCCH performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.2.1.3-1: Test Parameters for MPDCCH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  |  | symbols | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  |  | OCNG | N/A | N/A |
| Cell ID |  |  |  | 0 | 1 | 6 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
|  | δ |  | dB | 3 | 3 | 3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  |  | PRB | 1 | N/A | N/A |
|  |  |  | ms | 4 (Note 2) | N/A | N/A |
| Beamforming Pre-Coder |  |  |  | Annex B. 4.4 | N/A | N/A |
| Number of PRB per MPDCCH Set |  |  |  | 4 | N/A | N/A |
| Transmission type |  |  |  | Distributed | N/A | N/A |
| Frequency hopping |  |  |  | Disabled | N/A | N/A |
| Number of frequency hopping narrowbands |  |  |  | N/A | N/A | N/A |
| Frequency hopping offset |  |  |  | N/A | N/A | N/A |
| Frequency hopping inverval |  |  | ms | 4 | N/A | N/A |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  |  | 1.5 | N/A | N/A |
| Maximum number of repetitions |  |  |  | 1 | N/A | N/A |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  |  | 1 | N/A | N/A |
| PDSCH TM |  |  |  | TM2 | N/A | N/A |
| DCI Format |  |  |  | 6-1A | N/A | N/A |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  |  | 1111111111 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 10 | 10 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 70 | 70 |
|  |  | Rank 2 | % | N/A | 30 | 30 |
| Time offset relative to Cell 1 |  |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  |  | Hz | N/A | 200 | 300 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |  |

Table 8.11.2.1.3-2: Minimum performance CE Mode A MPDCCH

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 16 ECCE | R.82 FDD | OP.2 FDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 15.4 |
| Note 1: The OCNG pattern applies for Cell 1. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

##### 8.11.2.1.4	CE Mode A with CRS interference model

The requirements are specified in Table 8.11.2.1.4-2, with the addition of parameters in Table 8.11.2.1.4-1. In Table 8.11.2.1.4-2, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the MPDCCH performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.2.1.4-1: Test Parameters for MPDCCH

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | N/A | N/A |
| Cell ID |  |  | 0 | 1 | 6 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 | -3 | -3 |
|  |  | dB | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 15.81 | 10.5 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | N/A | N/A |
|  |  | ms | 4 | N/A | N/A |
| Beamforming Pre-Coder |  |  | Annex B. 4.4 | N/A | N/A |
| Number of PRB per MPDCCH Set |  |  | 4 | N/A | N/A |
| Transmission type |  |  | Distributed | N/A | N/A |
| Frequency hopping |  |  | Disabled | N/A | N/A |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | N/A |
| Frequency hopping offset |  |  | N/A | N/A | N/A |
| Frequency hopping inverval |  | ms | 4 | N/A | N/A |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | N/A | N/A |
| Maximum number of repetitions |  |  | 1 | N/A | N/A |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | N/A | N/A |
| PDSCH TM |  |  | TM2 | N/A | N/A |
| DCI Format |  |  | 6-1A | N/A | N/A |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Time offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |

Table 8.11.2.1.4-2: Minimum performance CE Mode A MPDCCH

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 ECCE | R.96 FDD | OP.2 FDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 15.8 |
| Note 1: The OCNG pattern applies for Cell 1. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

##### 8.11.2.1.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured

For the parameters specified in Table 8.11.2.1.5-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.1.5-2 for CE Mode A and in Table 8.11.2.1.5-3 for CE Mode B. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.1.5-1: Test Parameters for MPDCCH (Category M1)

| Parameter |  | Unit | CE Mode A (Test 1) | CE Mode B (Test 1) |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation | ρA | dB | -3 | 0 |
|  | ρB | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | N/A | Note 2 |
|  |  | ms | N/A | Note 2 |
| Beamforming Pre-Coder |  |  | Note 2 | Note 2 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Disabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 16 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | 1.5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | 16 | 32 |
| MPDCCH repetition number |  |  | 16 | 32 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 |
| mpdcch-crs-config |  |  | Configured | Configured |
| Power offset between CRS and DMRS antenna ports of MPDCCH |  | dB | 0 | 0 |
| mpdcch-crs-localized-mapping-type |  |  | N/A | Not configured |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: TS 36.211 6.8B.5. Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |

Table 8.11.2.1.5-2: Minimum performance CE Mode A MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 16 ECCE | R.82 FDD | OP.2 FDD | EPA5 | 2 x 1 Low | 1 | -3.4 | M1 |

Table 8.11.2.1.5-3: Minimum performance CE Mode B MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 24 ECCE | R.83 FDD | OP.2 FDD | ETU1 | 2 x 1 Low | 1 | -9.5 | M1 |

##### 8.11.2.2.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured

For the parameters specified in Table 8.11.2.2.5-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.2.5-2 for CE Mode A and in Table 8.11.2.2.5-3 for CE Mode B. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.2.5-1: Test Parameters for MPDCCH (Category M1)

| Parameter |  | Unit | CE Mode A | CE Mode B |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation | ρA | dB | -3 | 0 |
|  | ρB | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | N/A | Not e2 |
|  |  | ms | N/A | Note 2 |
| Beamforming Pre-Coder |  |  | Note 2 | Note 2 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Disabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 20 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 4) |  |  | 5 | 5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | 16 | 32 |
| MPDCCH repetition number |  |  | 16 | 32 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| TDD UL/DL Configuration |  |  | 0 | 0 |
| TDD Special Subframe |  |  | 1 | 1 |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 | 1000010000 |
| mpdcch-crs-config |  |  | Configured | Configured |
| Power offset between CRS and DMRS antenna ports of MPDCCH |  | dB | 0 | 0 |
| mpdcch-crs-localized-mapping-type |  |  | N/A | Not configured |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: TS 36.211 6.8B.5 Note 3: The special subframes are not supported by MPDCCH and are assumed as non- BL/CE DL subframes. Note 4: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |

Table 8.11.2.2.5-2: Minimum performance CE Mode A MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 16 ECCE | R.82 TDD | OP.2 TDD | EPA5 | 2 x 1 Low | 1 | -4.1 | M1 |

Table 8.11.2.2.5-3: Minimum performance CE Mode B MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 24 ECCE | R.83 TDD | OP.2 TDD | ETU1 | 2 x 1 Low | 1 | -9.7 | M1 |

#### 8.11.2.2	TDD

Table 8.11.2.2-1: Test Parameters for MPDCCH (Category M1)

| Parameter |  | Unit | CE Mode A | CE Mode B |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 |
|  |  | ms | 5 (Note 2) | 20 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Diabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 20 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 4) |  |  | 5 | 5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | 16 | 32 |
| MPDCCH repetition number |  |  | 16 | 32 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| TDD UL/DL Configuration |  |  | 0 | 0 |
| TDD Special Subframe |  |  | 1 | 1 |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 | 1000010000 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval. Note 3: The special subframes are not supported by MPDCCH, and are assumed as non- BL/CE DL subframes. Note 4: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 5: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |

Table 8.11.2.2-2: Test Parameters for MPDCCH (Category ≥1)

| Parameter |  | Unit | CE Mode A (Test 2) | CE Mode A (Test 3) | CE Mode B (Test 2) | CE Mode B (Test 3) |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 | 0 | 0 |
| Downlink power allocation |  | -3 | -3 | -3 | 0 | 0 |
|  |  | -3 | -3 | -3 | 0 | 0 |
|  |  | 0 | 0 | 0 | -3 | -3 |
|  | δ | 3 | 3 | 3 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 | 1 | 1 |
|  |  | ms | 5 (Note 2) | 5 (Note 2) | 20 (Note 2) | 20 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 4 | 2+4 | 2+4 |
| Transmission type |  |  | Distributed | Distributed | Localized | Localized |
| Frequency hopping |  |  | Disabled | Disabled | Enabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | 4 | 4 |
| Frequency hopping offset |  |  | N/A | N/A | 1 | 1 |
| Frequency hopping interval |  | ms | N/A | N/A | 5 | 5 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) Note 3 |  |  | 5 | 5 | 5 | 5 |
| Maximum number of repetitions(mPDCCH-NumRepetition) |  |  | 4 | 2 | 16 | 8 |
| MPDCCH repetition number |  |  | 4 | 2 | 16 | 8 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 1 | 7 | 7 |
| PDSCH TM |  |  | TM2 | TM2 | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1A | 6-1B | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 | 1000010000 | 1000010000 | 1000010000 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval. Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |

##### 8.11.2.2.1	CE Mode A

For the parameters specified in Table 8.11.2.2-1 and 8.11.2.2-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.2.1-1: Minimum performance CE Mode A MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 16 ECCE | R.82 TDD | OP.2 TDD | EPA5 | 2 x 1 Low | 1 | -5.3 | M1 |
| 2 | 10 MHz | 16 ECCE | R.82 TDD | OP.2 TDD | EPA5 | 2 x 2 Low | 1 | -5.3 | ≥1 |
| 3 | 10 MHz | 16 ECCE | R.82 TDD | OP.2 TDD | EPA5 | 2 x 4 Low | 1 | -6.8 | ≥1 |

##### 8.11.2.2.2	CE Mode B

For the parameters specified in Table 8.11.2.2-1 and 8.11.2.2-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.11.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.2.2.2-1: Minimum performance CE Mode B MPDCCH

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | 10 MHz | 24 ECCE | R.83 TDD | OP.2 TDD | ETU1 | 2 x 1 Low | 1 | -10.1 | M1 |
| 2 | 10 MHz | 24 ECCE | R.83 TDD | OP.2 TDD | ETU1 | 2 x 2 Low | 1 | -12.3 | ≥1 |
| 3 | 10 MHz | 24 ECCE | R.83 TDD | OP.2 TDD | ETU1 | 2 x 4 Low | 1 | -12.8 | ≥1 |

##### 8.11.2.2.3	CE Mode A with TM9 interference model

The requirements are specified in Table 8.11.2.2.3-2, with the addition of parameters in Table 8.11.2.2.3-1. In Table 8.11.2.2.3-2, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the MPDCCH performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.2.2.3-1: Test Parameters for MPDCCH (TM9 interference model)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  |  | symbols | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  |  | OCNG | N/A | N/A |
| Cell ID |  |  |  | 0 | 1 | 6 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | 0 | 0 | 0 |
|  | δ |  | dB | 3 | 3 | 3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 |  |  |
|  |  |  | dB | N/A | 11.75 | 5.69 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  |  | PRB | 1 | N/A | N/A |
|  |  |  | ms | 4 (Note 2) | N/A | N/A |
| Beamforming Pre-Coder |  |  |  | Annex B. 4.4 | N/A | N/A |
| Number of PRB per MPDCCH Set |  |  |  | 4 | N/A | N/A |
| Transmission type |  |  |  | Distributed | N/A | N/A |
| Frequency hopping |  |  |  | Disabled | N/A | N/A |
| Number of frequency hopping narrowbands |  |  |  | N/A | N/A | N/A |
| Frequency hopping offset |  |  |  | N/A | N/A | N/A |
| Frequency hopping inverval |  |  | ms | 4 | N/A | N/A |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  |  | 1.5 | N/A | N/A |
| Maximum number of repetitions |  |  |  | 1 | N/A | N/A |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  |  | 1 | N/A | N/A |
| PDSCH TM |  |  |  | TM2 | N/A | N/A |
| DCI Format |  |  |  | 6-1A | N/A | N/A |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  |  | 1000010000 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 10 | 10 |
| Probability of occurrence of PDSCH transmission rank in interfering cells |  | Rank 1 | % | N/A | 70 | 70 |
|  |  | Rank 2 | % | N/A | 30 | 30 |
| Time offset relative to Cell 1 |  |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  |  | Hz | N/A | 200 | 300 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |  |

Table 8.11.2.2.3-2: Minimum performance CE Mode A MPDCCH

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 16 ECCE | R.82 TDD | OP.2 TDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 15.5 |
| Note 1: The OCNG pattern applies for Cell 1. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

##### 8.11.2.2.4	CE Mode A with CRS interference model

The requirements are specified in Table 8.11.2.2.4-2, with the addition of parameters in Table 8.11.2.2.4-1. In Table 8.11.2.2.4-2, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] is provided to the UE and includes information on Cell 2 and Cell 3.

The purpose of the test is to verify the MPDCCH performance under assumption that UE applies CRS interference mitigation in the scenario with 2 CRS antenna ports in the serving and aggressor cells.

Table 8.11.2.2.4-1: Test Parameters for MPDCCH (CRS interference model)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | N/A | N/A |
| Cell ID |  |  | 0 | 1 | 6 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 | -3 | -3 |
|  |  | dB | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 |  |  |
|  |  | dB | N/A | 15.81 | 10.5 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | N/A | N/A |
|  |  | ms | 4 | N/A | N/A |
| Beamforming Pre-Coder |  |  | Annex B. 4.4 | N/A | N/A |
| Number of PRB per MPDCCH Set |  |  | 4 | N/A | N/A |
| Transmission type |  |  | Distributed | N/A | N/A |
| Frequency hopping |  |  | Disabled | N/A | N/A |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | N/A |
| Frequency hopping offset |  |  | N/A | N/A | N/A |
| Frequency hopping inverval |  | ms | 4 | N/A | N/A |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | N/A | N/A |
| Maximum number of repetitions |  |  | 1 | N/A | N/A |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | N/A | N/A |
| PDSCH TM |  |  | TM2 | N/A | N/A |
| DCI Format |  |  | 6-1A | N/A | N/A |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 | N/A | N/A |
| Interference model |  |  | N/A | As specified in clause B.6.5 | As specified in clause B.6.5 |
| Time offset relative to Cell 1 |  | s | N/A | 2 | 3 |
| Frequency shift relative to Cell 1 |  | Hz | N/A | 200 | 300 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |

Table 8.11.2.2.4-2: Minimum performance CE Mode A MPDCCH

| Test Number | Aggregation level | Reference Channel | OCNG Pattern (Note 1) | Propagation Conditions (Note 2) |  |  | Antenna Configuration and Correlation Matrix (Note 3) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  | Pm-dsg (%) | SNR (dB) (Note 4) |
| 1 | 4 ECCE | R.96 TDD | OP.2 FDD | EPA5 | EPA5 | EPA5 | 2x1 Low | 1 | 16.2 |
| Note 1: The OCNG pattern applies for Cell 1, Cell 2 and Cell 3. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 4: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

### 8.11.3	PBCH

The receiver characteristics of the PBCH are determined by the probability of miss-detection of the PBCH for single decoding interval (Pm-bch-s) and the probability of miss-detection of the PBCH for multiple decoding intervals (Pm-bch-m), which are defined as

The probability of miss-detection of the PBCH for single decoding interval (Pm-bch-s) is calculated under assumption of single PBCH TTI interval decoding. As is the number of correctly decoded MIB PDUs and Bs is the number of transmitted MIB PDUs (redundancy versions for the same MIB are not counted separately).

The probability of miss-detection of the PBCH for multiple decoding intervals (Pm-bch-m) is calculated over multiple PBCH TTI intervals under assumption of independent PBCH decoding over these intervals. Am is the number of PBCH decoding intervals with at least one correctly decoded MIB PDU and Bm is the total number of PBCH decoding intervals. A multiple PBCH decoding interval has 1120 ms duration consisting of continuous PBCH TTIs during the test.

#### 8.11.3.1	FDD and half-duplex FDD

Table 8.11.3.1-1: Test Parameters for PBCH

| Parameter |  | Unit | Transmit diversity |
| --- | --- | --- | --- |
| Downlink power allocation | PBCH_RA | dB | -3 |
|  | PBCH_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Cell ID |  |  | 0 |
| Repetition of the physical broadcast channel (Note 1) |  |  | Enabled |
| Cyclic prefix |  |  | Normal |
| Note 1: as specified in Table 6.6.4-1 in TS 36.211 [4]. |  |  |  |

##### 8.11.3.1.1	Transmit diversity performance

For the parameters specified in Table 8.11.3.1-1 and Table 8.11.3.1.1-1, the averaged probability of a miss-detected PBCH (Pm-bchs and Pm-bch-m) shall be below the specified value in Table 8.11.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.3.1.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value for single PBCH TTI |  | Reference value for multiple PBCH TTI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch-s (%) | SNR (dB) | Pm-bch-m (%) | SNR (dB) |
| 1 | 10 MHz | R.22 | EPA1 | 2 x 1 Low | 1 | -1.9 | 1 | -12.6 |

#### 8.11.3.2	TDD

Table 8.11.3.2-1: Test Parameters for PBCH

| Parameter |  | Unit | Transmit diversity |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  | 1 |
| Special subframe configuration (Note 2) |  |  | 4 |
| Downlink power allocation | PBCH_RA | dB | -3 |
|  | PBCH_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Cell ID |  |  | 0 |
| Repetition of the physical broadcast channel (Note 3) |  |  | Enabled |
| Cyclic prefix |  |  | Normal |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. Note 3: as specified in Table 6.6.4-2 in TS 36.211 [4]. |  |  |  |

##### 8.11.3.2.1	Transmit diversity performance

For the parameters specified in Table 8.11.3.2-1 and Table 8.11.3.2.1-1, the averaged probability of a miss-detected PBCH (Pm-bchs and Pm-bch-m) shall be below the specified value in Table 8.11.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.11.3.2.1-1: Minimum performance PBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value for single PBCH TTI |  | Reference value for multiple PBCH TTI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch-s (%) | SNR (dB) | Pm-bch-m (%) | SNR (dB) |
| 1 | 10 MHz | R.22 | EPA1 | 2 x 1 Low | 1 | -2.8 | 1 | -12.9 |

## 8.12	Demodulation of Narrowband IoT

### 8.12.1	NPDSCH

8.12.1.1	Half-duplex FDDThe parameters specified in Table 8.12.1.1-1 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.12.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier (Note 2) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 3 for In-band and N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 1) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 2: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.12.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | For tests in 8.12.1.1.4: 1 For others: 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 FDD for one NRS antenna port; R.NB.4 FDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

##### 8.12.1.1.1	Minimum Requirements for In-band

The requirements are specified in Table 8.12.1.1.1-2, with the addition of the parameters in Table 8.12.1.1.1 -1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.1-1: Test Parameters for NPDSCH under In-band

| Parameter |  | Unit | Test 1, 2 |
| --- | --- | --- | --- |
| Downlink power allocation of LTE signal |  | dB | -3 |
|  |  | dB | -3(Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -93 (Note 2) |
|  |  | dBm/15kHz | -99 (Note 3) |
| LTE CRS port number (eutra-NumCRS-Ports-r13) |  |  | 4 |
| NPDCCH repetition number |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (nPDCCH-startSF-USS-r13) |  |  | 4 for Test 1; 2 for Test 2; 1.5 for Test 3 |
| Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.1-2: Minimum performance under In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.5 FDD | 1 | EPA5 | 2x1 Low | 70% | 6.9 | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.5 FDD | 32 | EPA5 | 2x1 Low | 70% | -4.8 | NB1, NB2 |
| 3 | 200kHz | Non-anchor | R.NB.5-1 FDD | 256 | ETU1 | 2x1 Low | 70% | -9.8 | NB1, NB2 |

##### 8.12.1.1.2	Minimum Requirements for Standalone/Guard-band

The requirements are specified in Table 8.12.1.1.2-2, with the addition of the parameters in Table 8.12.1.1.2 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.2-1: Test Parameters for NPDSCH under Standalone/Guard-band

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 1x1 | 70% | -3.4 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 1x1 | 70% | -10.2 | NB1, NB2 |

##### 8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2

The requirements are specified in Table 8.12.1.1.3-2, with the addition of the parameters in Table 8.12.1.1.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance.

Table 8.12.1.1.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 FDD | 1 | EPA5 | 1 | 1x1 | 70% | 9.4 | NB2 |

##### 8.12.1.1.4	Minimum Requirements for Standalone for UE with multiple TBs interleaved transmission

The requirements are specified in Table 8.12.1.1.4-2, with the addition of the parameters in Table 8.12.1.1.4-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify NPDSCH performance when multiple TBs with interleaved transmission are scheduled by one DCI as specified in TS 36.213[6].

These requirements are applicable for UE of UE-Category-NB NB2 supporting multiple TBs scheduling with interleaved transmission when multiple TBs are scheduled.

Table 8.12.1.1.4-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 128 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 128 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Two HARQ processes (twoHARQ-ProcessesConfig-r14) |  |  | true |
| Multiple TBs scheduling (multiTB-Config-r16) |  |  | interleaved |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.4-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.8 FDD | 32 | ETU1 | 1 | 1x1 | 70% | -6.0 | NB2 |

##### 8.12.1.1.5	Minimum Requirements for Standalone for UE with 16-QAM

The requirements are specified in Table 8.12.1.1.5-2, with the addition of the parameters in Table 8.12.1.1.5-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM.

Table 8.12.1.1.5-1: Test Parameters for 16-QAM NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 1 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 4 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 2 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.5-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.9 FDD | 1 | EPA5 | 1 | 1x1 | 70% | 19.5 | NB2 |

#### 8.12.1.2	TDD

Table 8.12.1.2-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 4 |
| Special subframe configuration (Note 2) |  | 4 |
| Number of HARQ processes per component carrier (Note 4) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 3) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

##### 8.12.1.2.1	Minimum Requirements for In-band

The requirements are specified in Table 8.12.1.2.1-2, with the addition of the parameters in Table 8.12.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.2.1-1: Test Parameters for NPDSCH under In-band

| Parameter |  | Unit | Test 1, 2 |
| --- | --- | --- | --- |
| Downlink power allocation of LTE signal |  | dB | -3 |
|  |  | dB | -3(Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -93 (Note 2) |
|  |  | dBm/15kHz | -99 (Note 3) |
| LTE CRS port number (eutra-NumCRS-Ports-r13) |  |  | 4 |
| NPDCCH repetition number |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.1-2: Minimum performance under In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.5 TDD | 1 | EPA5 | 2x1 Low | 70% | 6.9 | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.5 TDD | 32 | EPA5 | 2x1 Low | 70% | -4.8 | NB1, NB2 |
| 3 | 200kHz | Non-anchor | R.NB.5-1 TDD | 256 | ETU1 | 2x1 Low | 70% | -9.8 | NB1, NB2 |

##### 8.12.1.2.2	Minimum Requirements for Standalone/Guard-band

The requirements are specified in Table 8.12.1.2.2-2, with the addition of the parameters in Table 8.12.1.2.2-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.2.2-1: Test Parameters for NPDSCH under Standalone/Guard-band

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 32 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 TDD | 32 | EPA5 | 1 | 1x1 | 70% | -3.4 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 TDD | 256 | ETU1 | 1 | 1x1 | 70% | -10.2 | NB1, NB2 |

##### 8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2

The requirements are specified in Table 8.12.1.2.3-2, with the addition of the parameters in Table 8.12.1.2.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance.

Table 8.12.1.2.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 TDD | 1 | EPA5 | 1 | 1x1 | 70% | 9.4 | NB2 |

##### 8.12.1.2.4	Minimum Requirements for Standalone for UE with 16-QAM

The requirements are specified in Table 8.12.1.2.4-2, with the addition of the parameters in Table 8.12.1.2.4-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM.

Table 8.12.1.2.4-1: Test Parameters for 16-QAM NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | Subframe | 1 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | Subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.4-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.8 TDD | 1 | EPA5 | 1 | 1x1 | 70% | 19.5 | NB2 |

### 8.12.2	NPDCCH

The receiver characteristics of the NPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

#### 8.12.2.1	Half-duplex FDD

The parameters specified in Table 8.12.2.1-1 and Table 8.12.2.1-2 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.12.2.1-1: Test Parameters for NPDCCH

| Parameter | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- |
| Narrowband physical layer Cell ID |  | 0 | 0 |
| at antenna port | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  | Normal | Normal |
| Number of CRS ports for in-band deployment mode |  | 4 | 4 |
| NPDCCH starting position (eutraControlRegionSize-r13) (Note 1) |  | 3 | 3 |
| Maximum number of repetitions (npdcch-NumRepetitions-r13) |  | 128 for Test 1; 1024 for Test 2. | 64 for Test 1; 512 for Test 2. |
| NPDCCH start subframe (npdcch-startSF-USS-r13) |  | 2 for test 1, 1.5 for test 2 | 2 for test 1, 1.5 for test 2 |
| NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13) |  | 0 | 0 |
| NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13) |  | Not configured | Not configured |
| NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13) |  | Not configured | Not configured |
| Downlink gap configuration for anchor carrier (dl-Gap-r13) |  | Not configured | Not configured |
| Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13) |  | Not configured | Not configured |
| Unused REs or RBs (Note 1) |  | OCNG | OCNG |
| OCNG pattern |  | NB.OP.1 | NB.OP.1 |
| NOTE 1: Applicable only for in-band deployment mode. |  |  |  |

Table 8.12.2.1-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Scheduling delay field () |  | 0 |
| NPDSCH Repetition number |  | 1 |
| at antenna port for NPDSCH | dBm/15kHz | -98 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDSCH |  | R.NB.6 FDD and R.NB.6-1 FDD for one NRS antenna port; R.NB.5 FDD and R.NB.5-1 FDD for two NRS antenna ports |

##### 8.12.2.1.1	Single-antenna performance

For the parameters specified in Table 8.12.2.1.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.1-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.1.1-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1x1 | 1 | -4.9 | NB1, NB2 |
| 2 | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1x1 | 1 | -11.4 | NB1, NB2 |

##### 8.12.2.1.2	Transmit diversity performance

For the parameters specified in Table 8.12.2.1.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.2-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.1.2-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Antenna Configuration and correlation matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | In-band | 64 | Anchor | R.NB.4 FDD | EPA5 | 2 | 2x1 Low | 1 | -3.9 | NB1, NB2 |
| 2 | In-band | 512 | Non-anchor | R.NB.4 FDD | ETU1 | 2 | 2x1 Low | 1 | -10.0 | NB1, NB2 |

#### 8.12.2.2	TDD

The parameters specified in Table 8.12.2.2-1 and Table 8.12.2.2-2 are valid for all TDD tests unless otherwise stated.

Table 8.12.2.2-1: Test Parameters for NPDCCH

| Parameter | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 4 | 4 |
| Special subframe configuration (Note 2) |  | 4 | 4 |
| Narrowband physical layer Cell ID |  | 0 | 0 |
| at antenna port | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  | Normal | Normal |
| Number of CRS ports for in-band deployment mode |  | 4 | 4 |
| NPDCCH starting position (eutraControlRegionSize-r13) (Note 1) |  | 2 | 2 |
| Maximum number of repetitions (npdcch-NumRepetitions-r13) |  | 128 for Test 1; 1024 for Test 2. | 64 for Test 1; 512 for Test 2. |
| NPDCCH start subframe (npdcch-startSF-USS-r13) |  | 4 | 4 |
| NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13) |  | 0 | 0 |
| NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13) |  | Not configured | Not configured |
| NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13) |  | Not configured | Not configured |
| Downlink gap configuration for anchor carrier (dl-Gap-r13) |  | Not configured | Not configured |
| Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13) |  | Not configured | Not configured |
| Unused REs or RBs (Note 1) |  | OCNG | OCNG |
| OCNG pattern |  | NB.OP.1 | NB.OP.1 |
| NOTE 1: Applicable only for in-band deployment mode. |  |  |  |

Table 8.12.2.2-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Scheduling delay field () |  | 0 |
| NPDSCH Repetition number |  | 1 |
| at antenna port for NPDSCH | dBm/15kHz | -98 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDSCH |  | R.NB.6 TDD and R.NB.6-1 TDD for one NRS antenna port; R.NB.5 TDD and R.NB.5-1 TDD for two NRS antenna ports |

##### 8.12.2.2.1	Single-antenna performance

For the parameters specified in Table 8.12.2.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.1-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.2.1-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 TDD | EPA5 | 1 | 1x1 | 1 | -4.9 | NB1, NB2 |
| 2 | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 TDD | ETU1 | 1 | 1x1 | 1 | -11.4 | NB1, NB2 |

##### 8.12.2.2.2	Transmit diversity performance

For the parameters specified in Table 8.12.2.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.2-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.2.2-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Antenna Configuration and correlation matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | In-band | 64 | Anchor | R.NB.4 TDD | EPA5 | 2 | 2x1 Low | 1 | -3.9 | NB1, NB2 |
| 2 | In-band | 512 | Non-anchor | R.NB.4 TDD | ETU1 | 2 | 2x1 Low | 1 | -10.0 | NB1, NB2 |

### 8.12.3	Demodulation of NPBCH

The receiver characteristics of the NPBCH are determined by the probability of miss-detection of the NPBCH (Pm-bch), which is defined as

For the performance with single a NPBCH TTI decoding, A is the number of correctly decoded MIB-NB PDUs and B is the number of transmitted MIB-NB PDUs.

For the performance with multiple NPBCH TTIs decoding, A is the number of 5120ms durations consisting of contiguous NPBCH TTIs where there is at least one correctly decoded MIB-NB PDU, and B is the number of 5120ms durations consisting of contiguous NPBCH TTIs during the test.

#### 8.12.3.1	HD-FDD

Table 8.12.3.1-1: Test Parameters for NPBCH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | NPBCH_RA | dB | 0 | -3 |
|  | NPBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |

##### 8.12.3.1.1	Single-antenna port performance with single NPBCH TTI

For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.1.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.1 | EPA1 | 1 x 1 | 1 | -2.0 | NB1, NB2 |

##### 8.12.3.1.2	Transmit diversity performance

8.12.3.1.2.1	Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs

For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.1.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.2 | EPA1 | 2 x 1 Low | 1 | -11.5 | NB1, NB2 |

#### 8.12.3.2	TDD

Table 8.12.3.2-1: Test Parameters for NPBCH TDD

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | NPBCH_RA | dB | 0 | -3 |
|  | NPBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |
| Uplink downlink configuration (Note 1) |  |  | 4 | 4 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |

##### 8.12.3.2.1	Single-antenna port performance with single NPBCH TTI

For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.1 | EPA1 | 1 x 1 | 1 | -2.0 | NB1, NB2 |

##### 8.12.3.2.2	Transmit diversity performance

8.12.3.2.2.1	Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs

For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.2.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.2 | EPA1 | 2 x 1 Low | 1 | -11.5 | NB1, NB2 |

## 8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)

### 8.13.1	FDD (CA and DC)

The parameters specified in Table 8.13.1-1 are valid for all FDD CA and DC tests unless otherwise stated.

Table 8.13.1-1: Common Test Parameters (FDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Cyclic Prefix |  |  |  | Normal |  |
| Cell_ID |  |  |  | 0 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |

#### 8.13.1.1	Closed-loop spatial multiplexing performance

##### 8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.1-3, based on single carrier requirement specified in Table 8.13.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.13.1.1.1-4, based on single carrier requirement specified in Table 8.13.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.13.1.1.1-5, based on single carrier requirement specified in Table 8.13.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.13.1.1.1-6, based on single carrier requirement specified in Table 8.13.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.13.1.1.1-3, and with PUCCH format 3 for Tests in Table 8.13.1.1.1-4, Table 8.13.1.1.1-5 and Table 8.13.1.1.1-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.1.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.1-4: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.1-6: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

##### 8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.13.1.1.2-3 for 2DL CCs and Table 8.13.1.1.2-4 for 3DL CCs, based on single carrier requirement specified in Table 8.13.1.1.2-2, with the addition of the parameters in Table 8.13.1.1.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity transmission.

Table 8.13.1.1.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 4) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: The same PDSCH transmission mode is applied to each component carrier. Note 4: As defined in TS36.300 [11]. Note 5: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.13.1.1.2-2: Single carrier performance for multiple DC configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.1.1.2-3: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity with 2 DL CCs

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 15+20 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 10+20MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 2x15 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 2x10 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥3 |
| 6 | 15+5 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥3 |
| 7 | 10+15 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.2-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity with 3DL CCs

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 20+20+15MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20+15+15MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 3x20 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20+20+10 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 20+15+10 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 6 | 20+10+10 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 7 | 15+15+10 MHz | As specified in Table 8.13.1.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

##### 8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding with 256QAM under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.3-3, based on single carrier requirement specified in Table 8.13.1.1.3-2, with the addition of the parameters in Table 8.13.1.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.13.1.1.3-3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.1.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

##### 8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port

The purpose of these tests is to verify the closed loop rank-four performance with wideband precoding with 4Tx and 4Rx under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.4-3, based on single carrier requirement specified in Table 8.13.1.1.4-2, with the addition of the parameters in Table 8.13.1.1.4-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.4-1: Test Parameters for Four-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.4-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.4 |

Table 8.13.1.1.4-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

#### 8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port

For CA with 2 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-3, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-4, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-5, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-6, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  |  |  | Reference Value in Table 8.13.1.2.1-2 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.13.1.2.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.2 | 9.0 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.7 | 8.8 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.6 | 8.7 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.3 | 8.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2. 1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.2.1-4: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.2.1-5: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.2.1-6: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

#### 8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)

The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.13.1.3.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.3.1-3, based on single carrier requirement specified in Table 8.13.1.3.1-2, with the addition of the parameters in Table 8.13.1.3.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.1.3.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 |
|  |  |  | dB | 0 | 0 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| Cell Id |  |  |  | 0 | 1 |
| PDSCH transmission mode |  |  |  | 6 | 4 |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 | % | N/A | 80 |
|  |  | Rank 2 | % | N/A | 20 |
| Precoding granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 4) |  |  | ms | 8 | N/A |
| Reporting interval |  |  | ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 1111 | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH (Note 7) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 2 | N/A |
|  | FDD SCC |  |  | 3 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#2, SF#6 and #7 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 SF#6 SF#0 and SF#1. |  |  |  |  |  |

Table 8.13.1.3.1-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.3.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

#### 8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)

The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.1.4.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.4.1-3, based on single carrier requirement specified in Table 8.13.1.4.1-2, with the addition of the parameters in Table 8.13.1.4.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.1.4.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cell Id |  |  | 0 | 126 |
| PDSCH transmission mode |  |  | 9 | 9 |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) |  | Ms | 8 | N/A |
| Reporting interval |  | Ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  | 4 | N/A |
|  | FDD SCC |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3, SF#4, SF#8 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7, SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |

Table 8.13.1.4.1-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.0 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.4.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

### 8.13.2	TDD (CA and DC)

The parameters specified in Table 8.13.2-1 are valid for all TDD CA and DC tests unless otherwise stated.

Table 8.13.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |

#### 8.13.2.1	Closed-loop spatial multiplexing performance

##### 8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.1-2A, based on single carrier requirement specified in Table 8.13.2.1.1-2, with the addition of the parameters in Table 8.13.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.13.2.1.1-3, based on single carrier requirement specified in Table 8.13.2.1.1-2, with the addition of the parameters in Table 8.13.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.13.2.1.1-4, based on single carrier requirement specified in Table 8.13.2.1.1-2, with the addition of the parameters in Table 8.13.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.13.2.1.1-5, based on single carrier requirement specified in Table 8.13.2.1.1-2, with the addition of the parameters in Table 8.13.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 8 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs; PUCCH format 3 for Tests with more than 2 CCs |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.2.1.1-2A: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.1.1-4: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

##### 8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.13.2.1.2-3, for 2DL CCs, in Table 8.13.2.1.2-4 for 3DL CCs, and Table 8.13.2.1.2-5 for 4DL CCs, based on single carrier requirement specified in Table 8.13.2.1.2-2, with the addition of the parameters in Table 8.13.2.1.2-1 and the downlink physical channel setup according to Annex C.3.2.The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity.

Table 8.13.2.1.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 5) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The same PDSCH transmission mode is applied to each component carrier. Note 5: As defined in TS36.300 [11]. Note 6: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.13.2.1.1-2: Single carrier performance for multiple DC configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.2.1.2-3: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.13.2.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.2-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20 MHz | As specified in Table 8.13.2.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.2-5: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20 MHz | As specified in Table 8.13.2.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 15+20+20+20MHz | As specified in Table 8.13.2.1.2-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectivity configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

##### 8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding with 256QAM under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.3-3, based on single carrier requirement specified in Table 8.13.2.1.3-2, with the addition of the parameters in Table 8.13.2.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.2.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

##### 8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port

The purpose of these tests is to verify the closed loop rank-four performance with wideband precoding with 4Tx and 4Rx under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.4-3, based on single carrier requirement specified in Table 8.13.2.1.4-2, with the addition of the parameters in Table 8.13.2.1.4-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.4-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Uplink-Downlink Configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.4-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.2.1.4-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.4-2 per CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.4-2 per CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

#### 8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port

For CA with 2 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-2A, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-3, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-4, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-5, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  |  |  | Reference Value in Table 8.13.2.2.1-2 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.13.2.2.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.0 | 9.0 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.1 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.2 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.1 | 9.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.1-2A: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.2.1-3: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.2.1-4: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

#### 8.13.2.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.2.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)

The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.13.2.3.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.3.1-3, based on single carrier requirement specified in Table 8.13.2.3.1-2, with the addition of the parameters in Table 8.13.2.3.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.2.3.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  |  | MHz |  | 10 |  | 10 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | N/A |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 50 |  | 6 |  |
| PMI delay (Note 4) |  |  |  | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | TDD PCC |  |  |  | 3 |  | N/A |  |
|  |  | TDD SCC |  |  |  | 4 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 SF#8 SF#2 and SF#3. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

#### 8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)

The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.2.4.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.4.1-3, based on single carrier requirement specified in Table 8.13.2.4.1-2, with the addition of the parameters in Table 8.13.2.4.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.2.4.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cell Id |  |  | 0 | 126 |
| PDSCH transmission mode |  |  | 9 | 9 |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  | ms | 10 or 11 | N/A |
| Reporting interval |  | ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | TDD PCC |  | 3 | N/A |
|  | TDD SCC |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7, SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |

Table 8.13.2.4.1-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.4.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.4.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.4.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

### 8.13.3	TDD-FDD (CA and DC)

The parameters specified in Table 8.13.3-1 are valid for all the TDD FDD CA tests unless otherwise stated.

Table 8.13.3-1: Common Test Parameters

| Parameter |  | Unit |  | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) for TDD CC only |  |  |  |  | 1 |  |
| Special subframe configuration (Note 2) for TDD CC only |  |  |  |  | 4 |  |
| Inter-TTI Distance |  |  |  |  | 1 |  |
| Maximum number of HARQ processes per component carrier |  | FDD PCell |  | Processes | 8 for FDD and TDD CCs |  |
|  |  | TDD PCell |  | Processes | 11 for FDD CC; 7 for TDD CC |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |  |
| Cyclic Prefix |  |  |  |  | Normal |  |
| Cell_ID |  |  |  |  | 0 |  |
| Cross carrier scheduling |  |  |  |  | Not configured |  |
| ACK/NACK feedback mode |  |  |  |  | PUCCH format 3 |  |
| Downlink HARQ-ACK timing |  | FDD PCell |  |  | As specified in Clause 7.3.3 in TS36.213 [6] |  |
|  |  | TDD PCell |  |  | As specified in Clause 7.3.4 in TS36.213 [6] |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |  |

The applicability of ther requirements are specified in Clause 8.1.2.6. The single carrier performance with different bandwidths for multiple CA configurations specified in Clause 8.13.3 cannot be applied for UE single carrier test.

#### 8.13.3.1		Closed-loop spatial multiplexing performance 4Tx Antenna Port

##### 8.13.3.1.1	Minimum Requirement for FDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.1-4 based on single carrier requirement specified in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3, with the addition of the parameters in Table 8.13.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.1-5 based on single carrier requirement specified in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3, with the addition of the parameters in Table 8.13.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.1-6 based on single carrier requirement specified in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3, with the addition of the parameters in Table 8.13.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.1-7 based on single carrier requirement specified in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3, with the addition of the parameters in Table 8.13.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.1.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.1.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.1.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.1.2	Minimum Requirement for TDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.2-4 based on single carrier requirement specified in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3, with the addition of the parameters in Table 8.13.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.2-5 based on single carrier requirement specified in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3, with the addition of the parameters in Table 8.13.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.2-6 based on single carrier requirement specified in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3, with the addition of the parameters in Table 8.13.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.2-7 based on single carrier requirement specified in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3, with the addition of the parameters in Table 8.13.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.1.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Widelband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.1.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.1.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.1.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell

The purpose of these tests is to verify dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.2.1-4 based on single carrier requirement specified in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3, with the addition of the parameters in Table 8.13.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.2.1-5 based on single carrier requirement specified in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3, with the addition of the parameters in Table 8.13.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.2.1-6 based on single carrier requirement specified in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3, with the addition of the parameters in Table 8.13.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.2.1-7 based on single carrier requirement specified in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3, with the addition of the parameters in Table 8.13.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC | Subframes | 5 / 2 |
|  | TDD CC |  | 5 / 4 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | FDD CC | Subframes / bitmap | 3 / 0010000000000000 |
|  | TDD CC |  | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  | FDD CC |  | Reference Value in Table 8.13.3.2.1-2 |
|  | TDD CC |  | Reference Value in Table 8.13.3.2.1-3 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. |  |  |  |

Table 8.13.3.2.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.2 | 9.0 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.7 | 8.8 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.6 | 8.7 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.3 | 8.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.2.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.0 | 9.0 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.1 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.2 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.1 | 9.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.2.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.2.2-4 based on single carrier requirement specified in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3, with the addition of the parameters in Table 8.13.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.2.2-5 based on single carrier requirement specified in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3, with the addition of the parameters in Table 8.13.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.2.2-6 based on single carrier requirement specified in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3, with the addition of the parameters in Table 8.13.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.2.2-7 based on single carrier requirement specified in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3, with the addition of the parameters in Table 8.13.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.2.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC | Subframes | 5 / 2 |
|  | TDD CC |  | 5 / 4 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | FDD CC | Subframes / bitmap | 3 / 0010000000000000 |
|  | TDD CC |  | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  | FDD CC |  | Reference Value in Table 8.13.3.2.2-2 |
|  | TDD CC |  | Reference Value in Table 8.13.3.2.2-3 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. |  |  |  |

Table 8.13.3.2.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.2 | 9.0 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.7 | 8.8 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.6 | 8.7 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.3 | 8.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.2.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.0 | 9.0 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.1 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.2 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.1 | 9.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.2.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.2.1-2 and Table 8.13.3.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.2-2 and Table 8.13.3.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.2.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.2.2-2 and Table 8.13.3.2.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell

The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.13.3.3.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with FDD PCell and with 2 DL CCs, the requirements are specified in Table 8.13.3.3.1-4, based on single carrier requirement specified in Table 8.13.3.3.1-2 and Table 8.13.3.3.1-3, with the addition of the parameters in Table 8.13.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.3.1-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | FDD PCC |  |  |  |  | 4 |  | N/A |  |
|  |  | TDD SCC |  |  |  |  | 5 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD PCell and SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.1-2 and Table 8.13.3.3.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.1-2 and Table 8.13.3.3.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.1-2 and Table 8.13.3.3.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell

The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.13.3.3.2-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with TDD PCell with 2 DL CCs, the requirements are specified in Table 8.13.3.3.2-4, based on single carrier requirement specified in Table 8.13.3.3.2-2 and Table 8.13.3.3.2-3, with the addition of the parameters in Table 8.13.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.3.2-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD PCell and SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.2-2 and Table 8.13.3.3.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.2-2 and Table 8.13.3.3.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.2-2 and Table 8.13.3.3.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |  |  |

#### 8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell

The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.3.4.1-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with FDD PCell with 2 DL CCs, the requirements are specified in Table 8.13.3.4.1-4, based on single carrier requirement specified in Table 8.13.3.4.1-2 and Table 8.13.3.4.1-3, with the addition of the parameters in Table 8.13.3.4.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.4.1-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | FDD CC | Subframes | 5 / 2 | N/A |
|  |  | TDD CC |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC |  | Ms | 8 | N/A |
|  | TDD CC |  |  | 10 or 11 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 4 | N/A |
|  | TDD SCC |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.3.4.1-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.0 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.1-2 and Table 8.13.3.4.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.1-2 and Table 8.13.3.4.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.1-2 and Table 8.13.3.4.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell

The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.3.4.2-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with TDD PCell with 2 DL CCs, the requirements are specified in Table 8.13.3.4.2-4, based on single carrier requirement specified in Table 8.13.3.4.2-2 and Table 8.13.3.4.2-3, with the addition of the parameters in Table 8.13.3.4.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.4.2-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC | Subframes | 5 / 2 | N/A |
|  | TDD CC |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  | 0 | N/A |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| Cell Id |  |  | 0 | 126 |
| PDSCH transmission mode |  |  | 9 | 9 |
| Beamforming model |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | N/A | 70 |
|  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC | Ms | 8 | N/A |
|  | TDD CC |  | 10 or 11 | N/A |
| Reporting interval |  | Ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD CC |  | 3 | N/A |
|  | TDD CC |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |

Table 8.13.3.4.2-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.0 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.2-2 and Table 8.13.3.4.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.2-2 and Table 8.13.3.4.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.2-2 and Table 8.13.3.4.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.5		Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC

##### 8.13.3.5.1	Minimum Requirement for FDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD DC with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.5.1-4 based on single carrier requirement specified in Table 8.13.3.5.1-2 and Table 8.13.3.5.1-3, with the addition of the parameters in Table 8.13.3.5.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.5.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for DC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 6) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. Note 6: As defined in TS36.300 [11]. Note 7: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.13.3.5.1-2: Single carrier performance with different bandwidths for multiple DC configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.5.1-3: Single carrier performance with different bandwidths for multiple DC configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.5.1-4: Minimum performance for multiple DC configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.5.1-2 and Table 8.13.3.5.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.5.2	Minimum Requirement for TDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD DC with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.5.2-4 based on single carrier requirement specified in Table 8.13.3.5.2-2 and Table 8.13.3.5.2-3, with the addition of the parameters in Table 8.13.3.5.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.5.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for DC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 6) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. Note 6: As defined in TS36.300 [11]. Note 7: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.13.3.5.2-2: Single carrier performance with different bandwidths for multiple DC configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.5.2-3: Single carrier performance with different bandwidths for multiple DC configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.5.2-4: Minimum performance for multiple DC configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.5.2-2 and Table 8.13.3.5.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.6		Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM

##### 8.13.3.6.1	Minimum Requirement for FDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.1-4 based on single carrier requirement specified in Table 8.13.3.6.1-2 and Table 8.13.3.6.1-3, with the addition of the parameters in Table 8.13.3.6.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.6.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.6.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.3.6.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.3.6.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.1-2 and Table 8.13.3.6.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.1-2 and Table 8.13.3.6.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.1-2 and Table 8.13.3.6.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.6.2	Minimum Requirement for TDD PCell

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.2-4 based on single carrier requirement specified in Table 8.13.3.6.2-2 and Table 8.13.3.6.2-3, with the addition of the parameters in Table 8.13.3.6.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.6.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Widelband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.6.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.3.6.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.3.6.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.2-2 and Table 8.13.3.6.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.2-2 and Table 8.13.3.6.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.1-2 and Table 8.13.3.6.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

#### 8.13.3.7		Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers

##### 8.13.3.7.1	Minimum Requirement for FDD PCell

The purpose of these tests is to verify the closed loop rank-four performance with wideband precoding with 4Tx and 4Rx under CA.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.7.1-4 based on single carrier requirement specified in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3, with the addition of the parameters in Table 8.13.3.7.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.7.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.7.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.4 |

Table 8.13.3.7.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.3.7.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

##### 8.13.3.7.2	Minimum Requirement for TDD PCell

The purpose of these tests is to verify the closed loop rank-four performance with wideband precoding with 4Tx and 4Rx under CA.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.7.2-4 based on single carrier requirement specified in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3, with the addition of the parameters in Table 8.13.3.7.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.7.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.7.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.4 |

Table 8.13.3.7.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.3.7.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

## 8.14	Demodulation (UE supporting Short TTI)

The performance requirements specified in this clause are valid for UEs capable of short TTI.

### 8.14.1	Slot-PDSCH and Subslot-PDSCH

#### 8.14.1.1	FDD (Fixed Reference Channel)

The parameters specified in Table 8.14.1.1-1 are valid for FDD unless otherwise stated.

Table 8.14.1.1-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Number of HARQ processes |  |  | Processes | 8 for Slot-PDSCH 8 for subslot-PDSCH with proc-Timeline=set1 (Note 1) 12 for subslot-PDSCH with proc-Timeline=set2 (Note 1) |  |
| Minimum Processing Time (Note 2) |  |  |  | Xp=4 (proc-Timeline=set1) Xp=6 (proc-Timeline=set2) |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Note 1: As specified in TS36.211 Table 8.1-2. Note 2: Subslot-PDSCH only. As specified in TS36.213 7.3. |  |  |  |  |  |

##### 8.14.1.1.1	Open-loop spatial multiplexing performance

The requirements are specified in Table 8.14.1.1.1-3, with the addition of the parameters in Table 8.14.1.1.1-1 and Table 8.14.1.1.1-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.1.1-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1,2,3 |
| DL TTI length |  |  | Slot | Subslot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| Used/Unused SPDCCH resource indication |  |  | N/A | ‘10’ |
| PDSCH transmission mode |  |  | TM3 | TM3 |
| DCI format |  |  | 7-1B | 7-1B |

Table 8.14.1.1.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 1 | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Rate Matching mode |  | Mode 1 | Not configured |
| SPDCCH L1 Reuse Indication |  | Not configured | {2, 0} |
| Aggregation level | SCCE | 2 | 2 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1, and PRB = {35, 36, …, 49} for Set 2 |  |  |  |

Table 8.14.1.1.1-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 0.5 | R.sTTI.1 FDD | OP.1 FDD | EVA30 | 4x2 Low | 70 | 13.6 | ≥2 |
| 2 | 10 MHz 16QAM 0.45 | R.sTTI.2 FDD | OP.1 FDD | EVA30 | 4x2 Low | 70 | 11.6 | ≥2 |

##### 8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)

The requirements are specified in Table 8.14.1.1.2-3, with the addition of the parameters in Table 8.14.1.1.2-1 and Table 8.14.1.1.2-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.1.2-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| CSI-RS signals |  |  | Antenna ports 15, 16 | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset |  |  | 5 / 2 | 5 / 2 |
| CSI-RS configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration |  |  | Not configured | Not configured |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | 2 | 2 |
|  |  | ms | 1 | 1 |
| Beamforming precoder |  |  | Annex B.4.2 | Annex B.4.2 |
| Cell specific reference signal |  |  | 0, 1 | 0, 1 |
| DL TTI length |  |  | Slot | Subslot |
| DMRS position indicator |  |  | N/A | 0 for subslots 1,3, and 5 1 for subslots 2 and 4 |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| Used/Unused SPDCCH resource indication |  |  | ’10’ | N/A |
| PDSCH transmission mode |  |  | TM9 | TM9 |
| DCI format |  |  | 7-1F | 7-1F |

Table 8.14.1.1.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence signal |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 2 | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 8 | 8 |
| Rate Matching mode |  | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication |  | {1, 1} | Not configured |
| Aggregation level | SCCE | 4 | 4 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 7} for Set 1, and PRB = {8, 9, …, 15} for Set 2 |  |  |  |

Table 8.14.1.1.2-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.sTTI.3 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 7.1 | ≥2 |
| 2 | 10 MHz QPSK 1/3 | R.sTTI.4 FDD | OP.1 FDD | EPA5 | 2x2 Low | 70 | 8.4 | ≥2 |

#### 8.14.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.14.1.2-1 are valid for TDD unless otherwise stated.

Table 8.14.1.2-1: Common Test Parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Number of HARQ processes |  |  | Processes | 8 (Slot-PDSCH) |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 2 |  |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4] Note 2: as specified in Table 4.2-1 in TS 36.211 [4] |  |  |  |  |  |

##### 8.14.1.2.1	Open-loop spatial multiplexing performance

The requirements are specified in Table 8.14.1.2.1-3, with the addition of the parameters in Table 8.14.1.2.1-1 and Table 8.14.1.2.1-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.2.1-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Subframe configuration |  |  | Non-MBSFN |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 |
| DL TTI length |  |  | Slot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |
| Used/Unused SPDCCH resource indication |  |  | N/A |
| PDSCH transmission mode |  |  | TM3 |
| DCI format |  |  | 7-1B |

Table 8.14.1.2.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence signal |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 1 | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Rate Matching mode |  | Mode 1 | Not configured |
| SPDCCH L1 Reuse Indication |  | Not configured | {2, 0} |
| Aggregation level | SCCE | 2 | 2 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1, and PRB = {35, 36, …, 49} for Set 2 |  |  |  |

Table 8.14.1.2.1-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 0.5 | R.sTTI.1 FDD | OP.1 TDD | EVA30 | 4x2 Low | 70 | 13.5 | ≥2 |

##### 8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)

The requirements are specified in Table 8.14.1.2.2-3, with the addition of the parameters in Table 8.14.1.2.2-1 and Table 8.14.1.2.2-2, and the downlink physical channel setup according to Annex C.3.2.

Table 8.14.1.2.2-1: Test Parameters for PDSCH

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| CSI-RS signals |  |  | Antenna ports 15, 16 |
| CRS-RS periodicity and subframe offset |  |  | 5 / 4 |
| CSI-RS configuration |  |  | 0 |
| Zero-power CRI-RS configuration |  |  | Not configured |
| Subframe configuration |  |  | Non-MBSFN |
| Precoder update granularity |  | SREG | 2 |
|  |  | ms | 1 |
| Beamforming precoder |  |  | Annex B.4.2 |
| Cell specific reference signal |  |  | 0,1 |
| DL TTI length |  |  | Slot |
| Configured SPDCCH-PRB sets |  |  | {Set 1, Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |
| Used/Unused SPDCCH resource indication |  |  | ’10’ |
| PDSCH transmission mode |  |  | TM9 |
| DCI format |  |  | 7-1F |

Table 8.14.1.2.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | CRS |
| Transmission type |  | Localized | Localized |
| Number of OFDM symbols |  | 2 | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 8 | 8 |
| Rate Matching mode |  | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication |  | {1, 1} | Not configured |
| Aggregation level | SCCE | 4 | 4 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 7} for Set 1, and PRB = {8, 9, …, 15} for Set 2 |  |  |  |

Table 8.14.1.2.2-3: Minimum performance Large Delay CDD (FRC)

| Test num | Bandwidth and MCS | Reference channel | OCNG pattern | Propa- gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/3 | R.sTTI.2 TDD | OP.1 TDD | EPA5 | 2x2 Low | 70 | 6.8 | ≥2 |

### 8.14.2	SPDCCH

The receiver characteristics of the SPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

#### 8.14.2.1	FDD

The parameters specified in Table 8.14.2.1-1 and Table 8.14.2.1-2 are valid for all FDD tests unless otherwise stated.

Table 8.14.2.1-1: Test Parameters for SPDCCH

| Parameter |  | Unit | Test 1 Test 3 | Test 2 Test 4 |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -3 |
|  |  | dB | -6 | -3 |
|  |  | dB | 0 | 0 |
|  | δ | dB | N/A | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | N/A | 2 |
|  |  | ms | N/A | 1 |
| Beamforming precoder |  |  | N/A | Annex B.4.4 |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1 |
| Configured SPDCCH-PRB sets |  |  | {Set 1} | {Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| PDSCH transmission mode |  |  | TM4 | TM9 |
| DCI format |  |  | 7-1C | 7-1F |
| Note 1: For slot-based transmission, DCI 7-1C and 7-1F are transmitted only on SPDCCH in slot 1. For subslot-based transmission, DCI 7-1C and 7-1F are transmitted on SPDCCH in all the subslots. |  |  |  |  |

Table 8.14.2.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | DMRS |
| Transmission type |  | Localized | Distributed |
| Number of OFDM symbols |  | 1 | Note 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1 and PRB = {0, 1, 6, 7, 12, 13, 18, 19, 28, 29, 34, 35, 40, 41, 46, 47} for Set 2. Note 2: 2 OFDM symbols for slot-based SPDCCH. 2 OFDM symbols for subslot indices 2, 3, and 4, and 3 OFDM symbols for subslot indices 1 and 5 for subslot-based SPDCCH. |  |  |  |

##### 8.14.2.1.1	Mimimum requirement

For the parameters specified in Table 8.14.2.1-1 and Table 8.14.2.1-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.14.2.1.1-1. The metric Pm-dsg is calculated across all the slots for slot-based SPDCCH and across all the subslots for subslot-based SPDCCH. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.14.2.1.1-1: Minimum performance SPDCCH

| Test number | Bandwidth | DL TTI length | Aggregation Level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | Slot | 2 SCCE | R.sTTI.10 FDD | OP.7 FDD | EVA30 | 4 x 2 Low | 1 | 2.9 |
| 2 | 10 MHz | Slot | 8 SCCE | R.sTTI.11 FDD | OP.7 FDD | EPA5 | 2 x 2 Low | 1 | 1.3 |
| 3 | 10 MHz | Subslot | 2 SCCE | R.sTTI.10 FDD | OP.7 FDD | EVA30 | 4 x 2 Low | 1 | 2.3 |
| 4 | 10 MHz | Subslot | 8 SCCE | R.sTTI.11 FDD | OP.7 FDD | EPA5 | 2 x 2 Low | 1 | 0.9 |

#### 8.14.2.2	TDD

The parameters specified in Table 8.14.2.2-1 and Table 8.14.2.2-2 are valid for all TDD tests unless otherwise stated.

Table 8.14.2.2-1: Test Parameters for SPDCCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -3 |
|  |  | dB | -6 | -3 |
|  |  | dB | 0 | 0 |
|  | δ | dB | N/A | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | N/A | 2 |
|  |  | ms | N/A | 1 |
| Beamforming precoder |  |  | N/A | Annex B.4.4 |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1 |
| Configured SPDCCH-PRB sets |  |  | {Set 1} | {Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| PDSCH transmission mode |  |  | TM4 | TM9 |
| DCI format |  |  | 7-1C | 7-1F |
| TDD UL/DL Configuration |  |  | 0 | 0 |
| TDD Special Subframe |  |  | 1 | 1 |
| Note 1: For slot-based transmission, DCI 7-1C and 7-1F are transmitted only on SPDCCH in slot 1. For subslot-based transmission, DCI 7-1C and 7-1F are transmitted on SPDCCH in all the subslots. |  |  |  |  |

Table 8.14.2.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | DMRS |
| Transmission type |  | Localized | Distributed |
| Number of OFDM symbols |  | 1 | Note 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1 and PRB = {0, 1, 6, 7, 12, 13, 18, 19, 28, 29, 34, 35, 40, 41, 46, 47} for Set 2. Note 2: 2 OFDM symbols for slot-based SPDCCH. 2 OFDM symbols for subslot indices 2, 3, and 4, and 3 OFDM symbols for subslot indices 1 and 5 for subslot-based SPDCCH. |  |  |  |

##### 8.14.2.2.1	Mimimum requirement

For the parameters specified in Table 8.14.2.2-1 and Table 8.14.2.2-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.14.2.2.1-1. The metric Pm-dsg is calculated across all the slots. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.14.2.2.1-1: Minimum performance SPDCCH

| Test number | Bandwidth | DL TTI length | Aggregation Level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | Slot | 2 SCCE | R.sTTI.10TDD | OP.7 TDD | EVA30 | 4 x 2 Low | 1 | 2.7 |
| 2 | 10 MHz | Slot | 8 SCCE | R.sTTI.11 TDD | OP.7 TDD | EPA5 | 2 x 2 Low | 1 | 1.2 |

## 8.15	Demodulation (8 receiver antenna ports)

### 8.15.1	PDSCH

#### 8.15.1.1	Void

#### 8.15.1.2	TDD (Fixed Reference Channel)

The parameters specified in Table 8.15.1.2-1 are valid for all TDD tests unless otherwise stated.

Table 8.15.1.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Precoder update granularity |  |  |  | Frequency domain: 1 PRG for Transmission modes 9 and 10 unless stated otherwise Time domain: 1 ms |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |

##### 8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.1-2, with the addition of the parameters in Table 8.15.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.15.1.2.1-1: Test Parameters for Transmit diversity Performance (FRC) with 8Rx Antenna Ports

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |
|  |  |  |  | dB |  | -3 (Note 1) |
|  |  |  |  | dB |  | 0 |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |
| PDSCH transmission mode |  |  |  |  |  | 2 |
| Note 1: |  |  |  |  |  |  |

Table 8.15.1.2.1-2: Minimum performance Transmit Diversity (FRC) with 8Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11 TDD | OP.1 TDD | EVA5 | 2x8 Medium correlation B, ULA | 70 | 1.4 | ≥2 |

##### 8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.2-2, with the addition of the parameters in Table 8.15.1.2.2-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.1.2.2-1: Test Parameters for Large Delay CDD (FRC) with 8Rx Antenna Ports

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 |
|  |  |  | dB | -3 (Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  |  | Bundling |
| PDSCH transmission mode |  |  |  | 3 |
| Note 1: |  |  |  |  |

Table 8.15.1.2.2-2: Minimum performance Large Delay CDD (FRC) with 8Rx Antenna Ports

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM 1/2 | R.11-1 TDD | OP.1 TDD | EVA70 | 2x8 Low | 70 | 4.1 | ≥2 |

##### 8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

The requirements are specified in Table 8.15.1.2.3-2, with the addition of the parameters in Table 8.15.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.1.2.3-1: Minimum performance for 8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | 8 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 10 or 11 |
| Reporting interval |  |  | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0010 0000 0000 0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1 and 6. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: Number of OFDM symbols for PDCCH: 2 |  |  |  |

Table 8.15.1.2.3-2: Minimum performance for for 8 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category | DL UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |  |
| 1 | 10 MHz 16QAM | R.50-4 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.5 | 8 | 14, 17,18,19,20,22,23,24,25,26 |

### 8.15.2	CA

#### 8.15.2.1	Void

#### 8.15.2.2	TDD

The parameters specified in Table 8.15.2.2-1 are valid for all TDD CA and DC tests unless otherwise stated.

Table 8.15.2.2-1: Common Test Parameters (TDD)

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  |  |  | 1 |  |
| Special subframe configuration (Note 2) |  |  |  | 4 |  |
| Cyclic prefix |  |  |  | Normal |  |
| Cell ID |  |  |  | 0 |  |
| Inter-TTI Distance |  |  |  | 1 |  |
| Number of HARQ processes per component carrier |  |  | Processes | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| Redundancy version coding sequence |  |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths unless otherwise stated |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |  |

##### 8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)

8.15.2.2.1.1	Minimum Requirement Eight-Layer Spatial Multiplexing 8 Tx Antenna Port

The purpose of these tests is to verify the closed loop rank-eight performance with frequency selective precoding with 8Tx and 8Rx under CA.

For CA with 2 DL CCs, the requirements are specified in Table 8.15.2.2.1.1-3, based on single carrier requirement specified in Table 8.15.2.2.1.1-2, with the addition of the parameters in Table 8.15.2.2.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.15.2.2.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| ownlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3(Note 4, 5) |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 8 |
| Reporting interval |  |  | 5 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0010 0000 0000 0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 8.15.2.2.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.50-3 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 17.6 |
| 10MHz | R.50-4 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.5 |
| 15MHz | R.50-5 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.9 |
| 20MHz | R.50-6 TDD | OP.1 TDD | EPA5 | 8x8 Low | 70 | 18.9 |

Table 8.15.2.2.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category | DL UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.15.2.2.1.1-2 per CC | 8 | 14, 17,18,19,20,22,23,24,25,26 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.12.3. |  |  |  |  |

# 9	Reporting of Channel State Information

## 9.1	General

This section includes requirements for the reporting of channel state information (CSI). For all test cases in this section, the definition of SNR and SINR are in accordance with the one given in clause 8.1.1.

For the performance requirements specified in this clause, it is assumed that NRX=2 unless otherwise stated.

Unless otherwise stated, 4-bit CQI Table in Table 7.2.3-1 in TS 36.213 [6], and Modulation and TBS index table in Table 7.1.7.1-1 for PDSCH in TS 36.213 [6] are applied in all the CSI requirements.

### 9.1.1	Applicability of requirements

#### 9.1.1.1	Applicability of requirements for different channel bandwidths

In Clause 9 the test cases may be defined with different channel bandwidth to verify the same CSI requirement.

Test cases defined for 5MHz channel bandwidth that reference this clause are applicable to UEs that support only Band 31, 72, 73, 87 and/or 88.

#### 9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA CQI tests in Clause 9 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL CCs in Table 9.1.1.2-1 and 3 or more DL CCs in Table 9.1.1.2-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 9.1.1.2-1: Applicability and test rules for CA UE CQI tests with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 9.6.1.1 | Any of one of the supported CA capabilities | Any one of the supported FDD CA configurations | 10+10 MHz, 20+20 MHz, 5+5 MHz, 10MHz+5MHz, 15MHz+5MHz |
| CA tests with 2CCs in Clause 9.6.1.2 | Any of one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| Note 1: The applicability and test rules are specified in this table, unless otherwise stated. Note 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. Note 3: A single Uplink CC is configured for all tests |  |  |  |

Table 9.1.1.2-2: Applicability and test rules for CA UE CQI tests with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 3 ore more CCs in Clause 9.6.1.1 | Any of one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3 or more CCs in Clause 9.6.1.2 | Any of one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| Note 1: The applicability and test rules are specified in this table, unless otherwise stated. Note 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. Note 3: A single Uplink CC is configured for all tests |  |  |  |

#### 9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets

The performance requirement for TDD-FDD CA CQI tests in Clause 9 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL TDD-FDD CA in Table 9.1.1.2A-1 and for 3 or more DL TDD-FDD CA in Table 9.1.1.2A-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 9.1.1.2A-1: Applicability and test rules for CA UE CQI tests for TDD-FDD CA with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 9.6.1.3 | Any of one of the supported CA capabilities | Any one of the supported TDD-FDD CA configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 9.6.1.4 | Any of one of the supported CA capabilities | Any one of the supported TDD-FDD CA configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| Note 1: The applicability and test rules are specified in this table, unless otherwise stated. Note 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. Note 3: A single Uplink CC is configured for all tests |  |  |  |

Table 9.1.1.2A-2: Applicability and test rules for CA UE CQI tests for TDD-FDD CA with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 3CCs in Clause 9.6.1.3 | Any of one of the supported CA capabilities | Any one of the supported TDD-FDD CA configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs in Clause 9.6.1.4 | Any of one of the supported CA capabilities | Any one of the supported TDD-FDD CA configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| Note 1: The applicability and test rules are specified in this table, unless otherwise stated. Note 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. Note 3: A single Uplink CC is configured for all tests |  |  |  |

#### 9.1.1.3	Test coverage for different number of componenet carriers

For FDD CA tests specified in 9.6.1.1, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For TDD CA tests specified in 9.6.1.2, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

For TDD FDD CA tests specified in 9.6.1.3 and 9.6.1.4, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the TDD FDD CA tests with less than the largest number of CCs supported by the UE.

#### 9.1.1.4	Applicability of performance requirements for 4Rx capable UEs

##### 9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx

For 4Rx capable UEs all single carrier tests specified in 9.2 to 9.5 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of the 4Rx with data source from system simulator, and the other 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied.

For 4Rx capable UEs without any 2Rx supported RF bands, all single carrier tests specified in 9.2 to 9.5 with 2Rx are tested on any of the 4Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna where applicable. Figure 9.1.1.4.1-1 shows an example of antenna connection for 4Rx UE in any one 4Rx supported RF band to perform a 2Rx performance test with antenna configuration as 2x2 without interference for information. The SNR requirements should be applied with 3 dB less than the number specified with 2Rx, unless there is no SNR requirements specified.

For 4Rx capable UEs without any 2Rx supported RF bands, all single carrier tests specified in 9.3.3 with 2Rx are tested on any of the 4Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent interference for each Rx antenna.

Figure 9.1.1.4.1-1 Antenna connection example for 2Rx tests with antenna configuration as 2x2 without interference (informative)

For 4Rx capable UEs without any 2Rx supported RF bands, for all single carrier tests listed in Table 9.1.1.4.1-1 specified from 9.2 to 9.5 with 2Rx can be skipped.

Table 9.1.1.4.1-1 Requirement lists for 4Rx capable UEs

Requirement lists

Enhanced performance requirements type B

Requirements with demodulation subframe overlaps with aggressor cell ABS

Requirements with demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured

For 4Rx capable UEs, if corresponding tests listed from the 4Rx test lists from Table 9.1.1.4.1-2 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed from the 2Rx test lists from Table 9.1.1.4.1-2.

Table 9.1.1.4.1-2: Applicability rules for single carrier tests with 2Rx

| 4Rx test lists | 2Rx test lists |
| --- | --- |
| 9.9.1.1.1 Test 1 | 9.2.1.1 Test 1 |
| 9.9.1.1.1 Test 2 | 9.2.1.1 Test 2 |
| 9.9.1.1.2 Test 1 | 9.2.1.2 Test 1 |
| 9.9.1.1.2 Test 2 | 9.2.1.2 Test 2 |
| 9.9.1.2.1 Test 1 | 9.2.3.1 Test 1 |
| 9.9.1.2.1 Test 2 | 9.2.3.1 Test 2 |
| 9.9.1.2.2 Test 1 | 9.2.3.2 Test 1 |
| 9.9.1.2.2 Test 2 | 9.2.3.2 Test 2 |
| 9.9.2.1.1 | 9.3.5.1.1 |
| 9.9.2.1.2 | 9.3.5.1.2 |
| 9.9.2.2.1 | 9.3.5.2.1 |
| 9.9.2.2.2 | 9.3.5.2.2 |
| 9.9.3.1.1 Test 1 | 9.4.2.3.2 Test 1 |
| 9.9.4.1.1 Test 1 | 9.5.1.1 Test 1 |
| 9.9.4.1.1 Test 2 | 9.5.1.1 Test 2 |
| 9.9.4.1.1 Test 3 | 9.5.1.1 Test 3 |
| 9.9.4.1.2 Test 1 | 9.5.1.2 Test 1 |
| 9.9.4.1.2 Test 2 | 9.5.1.2 Test 2 |
| 9.9.4.1.2 Test 3 | 9.5.1.2 Test 3 |
| 9.9.4.2.1 Test 1 | 9.5.2.1 Test 1 |
| 9.9.4.2.1 Test 2 | 9.5.2.1 Test 2 |
| 9.9.4.2.1 Test 3 | 9.5.2.1 Test 3 |
| 9.9.4.2.2 Test 1 | 9.5.2.2 Test 1 |
| 9.9.4.2.2 Test 2 | 9.5.2.2 Test 2 |
| 9.9.4.2.2 Test 3 | 9.5.2.2 Test 3 |

##### 9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx

All tests specified in 9.6 with 2Rx with CA and TDD-FDD CA are tested with 4 Rx capable UEs by connecting all 4Rx with data source from system simulator with the following change on the power level in the test configurations listed in Table 9.1.1.4.2-1 and by scheduling the PDSCH for user data based on the Reference measurement channel RC.1 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 for FDD cells and Reference measurement channel RC.1 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 for TDD cells.

Table 9.1.1.4.2-1: Power level for 4Rx capable UE to verify CA tests with 2Rx

| Number of CCs |  | dB[mW/15kHz] |
| --- | --- | --- |
| 2 | PCell | -88 |
|  | SCell | -95 |
| 3,4 | PCell | -85 |
|  | SCell1 | -92 |
|  | SCell2, SCell3 | -99 |
| 5 | PCell | -85 |
|  | SCell1 | -92 |
|  | SCell2, SCell3, SCell4 | -99 |

Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, keep the same power level listed in Table 9.1.1.4.2-1. Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, configure the power level 3 dB smaller than the number listed in Table 9.1.1.4.2-1. Same requirements specified with 2Rx should be applied.

Same applicability rules defined in 9.1.1.2, 9.1.1.2A for CA and TDD-FDD CA applied for different CA configurations and bandwidth combination sets should be applied for 4 Rx capable UEs.

##### 9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx

For 4Rx capable UEs all single carrier tests specified in 9.9 with 4Rx are tested on any of the 4 Rx supported RF bands by connecting all 4Rx with data source from system simulator.

#### 9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement

For 2Rx capable UEs supporting coverage enhancement mode A (ce-ModeA in UE-EUTRA-Capability [7]), all the tests for CE Mode A specified in 9.8.1 and 9.8.2 are tested on any of the 2Rx supported RF bands by connecting all 2Rx with data source from system simulator. The SNR requirements should be applied with 3dB less than the number specified with UE DL Category M1.

For 4Rx capable UEs supporting coverage enhancement mode A (ce-ModeA in UE-EUTRA-Capability [7]), all the tests for CE Mode A specified in 9.8.1 and 9.8.2 are tested on any of the 4Rx supported RF bands by connecting all 4Rx with data source from system simulator. The SNR requirements should be applied with 6dB less than the number specified with UE DL Category M1.

## 9.2		CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 36.213 [6]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

### 9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)

#### 9.2.1.1	FDD

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.1.1-1 and Table 9.2.1.1-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1 FDD / RC.14 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

The applicability of the requirement with 5MHz bandwidth as specificed in Table 9.2.1.1-2 is defined in 9.1.1.1.

Table 9.2.1.1-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.1 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4 FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.2.1.1-2: PUCCH 1-0 static test (FDD 5MHz)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 5 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | [0] | [1] | [6] | [7] |
|  |  | dB[mW/15kHz] | [-98] | [-97] | [-92] | [-91] |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.14 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.15 FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.2.1.2	TDD

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.1.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.2.1.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.1 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1, except for category 1 UE use RC.4 TDD with two sided dynamic OCNG Pattern OP.2 TDD as described in Annex A.5.2.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

#### 9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.1.3-1, and using the downlink physical channels specified in tables C.3.2-1 for Cell 1, C.3.3-1 for Cell 2 and C.3.2-2, the reported CQI value according to RC.2 FDD / RC.6 FDD in Table A.4-1 in subframes overlapping with aggressor cell ABS and non-ABS subframes shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER in non-ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,1 is less than or equal to 0.1, the BLER in non-ABS subframes using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER in non-ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in non-ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1. The value of the median CQI obtained by reports in CSI subframe sets CCSI,0 minus the median CQI obtained by reports in CSI subframe sets CCSI,1 shall be larger than or equal to 2 and less than or equal to 5 in Test 1 and shall be larger than or equal to 0 and less than or equal to 1 in Test 2.

Table 9.2.1.3-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 | Cell 1 |  | Cell 2 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 2 |  | Note 10 | 2 |  | Note 10 |
| Downlink power allocation |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2x2) |  |  | Clause B.1 (2x2) |  |  |
| (Note 1) |  | dB | 4 | 5 | 6 | 4 | 5 | -12 |
| at antenna port |  | dBm/15kHz | -102 (Note 7) |  | N/A | -98(Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98(Note 8) |  | N/A |
|  |  | dBm/15kHz | -94.8 (Note 9) |  | N/A | -98(Note 9) |  | N/A |
|  |  | dB[mW/15kHz] | -94 | -93 | -92 | -94 | -93 | -110 |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | 1 | 0 |  | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |  | 2.5 (synchronous cells) |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 01010101 01010101 01010101 01010101 01010101 | N/A |  | 01010101 01010101 01010101 01010101 01010101 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 00000100 00000100 00000100 00000100 00000100 |  | N/A | 00000100 00000100 00000100 00000100 00000100 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 01010101 01010101 01010101 01010101 01010101 |  | N/A | 01010101 01010101 01010101 01010101 01010101 |  | N/A |
|  | CCSI,1 |  | 10101010 10101010 10101010 10101010 10101010 |  | N/A | 10101010 10101010 10101010 10101010 10101010 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| Physical channel for CCSI,0 CQI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |  |
| Physical channel for CCSI,1 CQI reporting |  |  | PUSCH (Note 12) |  |  | PUSCH (Note 12) |  |  |
| PUCCH Report Type |  |  | 4 |  |  | 4 |  |  |
| Reporting periodicity |  | Ms | Npd = 5 |  |  | Npd = 5 |  |  |
| cqi-pmi-ConfigurationIndex CCSI,0 (Note 13) |  |  | 6 |  | N/A | 6 |  | N/A |
| cqi-pmi-ConfigurationIndex2 CCSI,1 (Note 14) |  |  | 5 |  | N/A | 5 |  | N/A |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.1.5 Note 11: Reference measurement channel in Cell 1 RC.2 FDD according to Table A.4-1 for UE Cateogry 2-8 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, and RC.6 FDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP. 1/2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 12: To avoid collisions between HARQ-ACK and wideband CQI it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 13: cqi-pmi-ConfigurationIndex is applied for CCSI,0. Note 14: cqi-pmi-ConfigurationIndex2 is applied for CCSI,1. |  |  |  |  |  |  |  |  |

#### 9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.1.4-1, and using the downlink physical channels specified in tables C.3.2-1 for Cell 1, C3.3-1 for Cell 2 and C.3.2-2, the reported CQI value according to RC.2 TDD / RC.6 TDD in Table A.4-1 in subframes overlapping with aggressor cell ABS and non-ABS subframes shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER in non-ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,1 is less than or equal to 0.1, the BLER in non-ABS subframes using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER in non-ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in non-ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1. The value of the median CQI obtained by reports in CSI subframe sets CCSI,0 minus the median CQI obtained by reports in CSI subframe sets CCSI,1 shall be larger than or equal to 2 and less than or equal to 5 in Test 1 and shall be larger than or equal to 0 and less than or equal to 1 in Test 2.

Table 9.2.1.4-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 | Cell 1 |  | Cell 2 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 2 |  | Note 10 | 2 |  | Note 10 |
| Uplink downlink configuration |  |  | 1 |  |  | 1 |  |  |
| Special subframe configuration |  |  | 4 |  |  | 4 |  |  |
| Downlink power allocation |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2x2) |  |  | Clause B.1 (2x2) |  |  |
| (Note 1) |  | dB | 4 | 5 | 6 | 4 | 5 | -12 |
| at antenna port |  | dBm/15kHz | -102 (Note 7) |  | N/A | -98 (Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98 (Note 8) |  | N/A |
|  |  | dBm/15kHz | -94.8 (Note 9) |  | N/A | -98 (Note 9) |  | N/A |
|  |  | dB[mW/15kHz] | -94 | -93 | -92 | -94 | -93 | -110 |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | 1 | 0 |  | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |  | 2.5 (synchronous cells) |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 0100010001 0100010001 | N/A |  | 0100010001 0100010001 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 0000000001 0000000001 |  | N/A | 0000000001 0000000001 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 0100010001 0100010001 |  | N/A | 0100010001 0100010001 |  | N.A |
|  | CCSI,1 |  | 1000101000 1000101000 |  | N/A | 1000101000 1000101000 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| Physical channel for CCSI,0 CQI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |  |
| Physical channel for CCSI,1 CQI reporting |  |  | PUSCH (Note 12) |  |  | PUSCH |  |  |
| PUCCH Report Type |  |  | 4 |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  | Npd = 5 |  |  |
| cqi-pmi-ConfigurationIndex CCSI,0 (Note 13) |  |  | 3 |  | N/A | 3 |  | N/A |
| cqi-pmi-ConfigurationIndex2 CCSI,1 (Note 14) |  |  | 4 |  | N/A | 4 |  | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  | Multiplexing |  |  |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell1 and Cell2 is the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.2.5 Note 11: Reference measurement channel in Cell 1 RC.2 TDD according to Table A.4-1 for UE Category ≥2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1, and RC.6 TDD according to Table A.4-1 for Category 1 with one/two sided dynami OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1 and Annex A.5.2.2. Note 12: To avoid collisions between HARQ-ACK and wideband CQI it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 13: cqi-pmi-ConfigurationIndex is applied for CCSI,0. Note 14: cqi-pmi-ConfigurationIndex2 is applied for CCSI,1. |  |  |  |  |  |  |  |  |

#### 9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)

The following requirements apply to UE Category ≥2. For the parameters specified in Table 9.2.1.5-1, and using the downlink physical channels specified in tables C.3.2-1 for Cell 1, C.3.3-2 for Cell 2 and Cell 3, and C.3.2-2, the reported CQI value according to RC.2 FDD in Table A.4-1 in subframes overlapping with aggressor cell ABS and non-ABS subframes shall be in the range of ±1 of the reported median more than 90% of the time.

For test 1 and test 2, if the PDSCH BLER in ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,0 is less than or equal to 0.1, the BLER in ABS subframes using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER in ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

For test 2, if the PDSCH BLER in non-ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,1 is less than or equal to 0.1, the BLER in non-ABS subframes using the transport format indicated by the (median CQI + 2) shall be greater than 0.1. If the PDSCH BLER in non-ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in non-ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.2.1.5-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 and 3 | Cell 1 |  | Cell 2 and 3 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 2 |  | Note 10 | 2 |  | Note 10 |
| Downlink power allocation |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2x2) |  |  | Clause B.1 (2x2) |  |  |
| (Note 1) |  | dB | 4 | 5 | Cell 2: 12 Cell 3: 10 | 13 | 14 | Cell 2: 12 Cell 3: 10 |
| at antenna port |  | dBm/15kHz | -98 (Note 7) |  | N/A | -98 (Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98 (Note 8) |  | N/A |
|  |  | dBm/15kHz | -93 (Note 9) |  | N/A | -93 (Note 9) |  | N/A |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | Cell 2: 6 Cell 3: 1 | 0 |  | Cell 2: 6 Cell 3: 1 |
| Time Offset between Cells |  | s | Cell 2: 3 usec Cell 3: -1usec |  |  | Cell 2: 3 usec Cell 3: -1usec |  |  |
| Frequency Shift between Cells |  | Hz | Cell 2: 300Hz Cell 3: -100Hz |  |  | Cell 2: 300Hz Cell 3: -100Hz |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 01010101 01010101 01010101 01010101 01010101 | N/A |  | 01010101 01010101 01010101 01010101 01010101 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 00000100 00000100 00000100 00000100 00000100 |  | N/A | 00000100 00000100 00000100 00000100 00000100 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 01010101 01010101 01010101 01010101 01010101 |  | N/A | 01010101 01010101 01010101 01010101 01010101 |  | N/A |
|  | CCSI,1 |  | 10101010 10101010 10101010 10101010 10101010 |  | N/A | 10101010 10101010 10101010 10101010 10101010 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| Physical channel for CCSI,0 CQI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |  |
| Physical channel for CCSI,1 CQI reporting |  |  | PUSCH (Note 12) |  |  | PUSCH (Note 12) |  |  |
| PUCCH Report Type |  |  | 4 |  |  | 4 |  |  |
| Reporting periodicity |  | Ms | Npd = 5 |  |  | Npd = 5 |  |  |
| cqi-pmi-ConfigurationIndex CCSI,0 (Note 13) |  |  | 6 |  | N/A | 6 |  | N/A |
| cqi-pmi-ConfigurationIndex2 CCSI,1 (Note 14) |  |  | 5 |  | N/A | 5 |  | N/A |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 and Cell 3 are the aggressor cells. The number of the CRS ports in Cell1, Cell2, and Cell3 are the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.1.5 Note 11: Reference measurement channel in Cell 1 RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 12: To avoid collisions between HARQ-ACK and wideband CQI it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 13: cqi-pmi-ConfigurationIndex is applied for CCSI,0. Note 14: cqi-pmi-ConfigurationIndex2 is applied for CCSI,1. |  |  |  |  |  |  |  |  |

#### 9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)

The following requirements apply to UE Category ≥2. For the parameters specified in Table 9.2.1.6-1, and using the downlink physical channels specified in tables C.3.2-1 for Cell 1, C3.3-2 for Cell 2 and Cell 3, and C.3.2-2, the reported CQI value according to RC.2 TDD in Table A.4-1 in subframes overlapping with aggressor cell ABS and non-ABS subframes shall be in the range of ±1 of the reported median more than 90% of the time.

For test 1 and test 2, if the PDSCH BLER in ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,0 is less than or equal to 0.1, the BLER in ABS subframes using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER in ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

For test 2, if the PDSCH BLER in non-ABS subframes using the transport format indicated by median CQI obtained by reports in CSI subframe sets CCSI,1 is less than or equal to 0.1, the BLER in non-ABS subframes using the transport format indicated by the (median CQI + 2) shall be greater than 0.1. If the PDSCH BLER in non-ABS subframes using the transport format indicated by the median CQI is greater than 0.1, the BLER in non-ABS subframes using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.2.1.6-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 and 3 | Cell 1 |  | Cell 2 and 3 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 2 |  | Note 10 | 2 |  | Note 10 |
| Uplink downlink configuration |  |  | 1 |  |  | 1 |  |  |
| Special subframe configuration |  |  | 4 |  |  | 4 |  |  |
| Downlink power allocation |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | -3 |  |  | -3 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2x2) |  |  | Clause B.1 (2x2) |  |  |
| (Note 1) |  | dB | 4 | 5 | Cell 2: 12 Cell 3: 10 | 13 | 14 | Cell 2: 12 Cell 3: 10 |
| at antenna port |  | dBm/15kHz | -98 (Note 7) |  | N/A | -98 (Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98 (Note 8) |  | N/A |
|  |  | dBm/15kHz | -93 (Note 9) |  | N/A | -93 (Note 9) |  | N/A |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | Cell 2: 6 Cell 3: 1 | 0 |  | Cell 2: 6 Cell 3: 1 |
| Time Offset between Cells |  | s | Cell 2: 3 usec Cell 3: -1usec |  |  | Cell 2: 3 usec Cell 3: -1usec |  |  |
| Frequency shift between Cells |  | Hz | Cell 2: 300Hz Cell 3: -100Hz |  |  | Cell 2: 300Hz Cell 3: -100Hz |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 0100010001 0100010001 | N/A |  | 0100010001 0100010001 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 0000000001 0000000001 |  | N/A | 0000000001 0000000001 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 0100010001 0100010001 |  | N/A | 0100010001 0100010001 |  | N.A |
|  | CCSI,1 |  | 1000101000 1000101000 |  | N/A | 1000101000 1000101000 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| Physical channel for CCSI,0 CQI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |  |
| Physical channel for CCSI,1 CQI reporting |  |  | PUSCH (Note 12) |  |  | PUSCH (Note 12) |  |  |
| PUCCH Report Type |  |  | 4 |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  | Npd = 5 |  |  |
| cqi-pmi-ConfigurationIndex CCSI,0 (Note 13) |  |  | 3 |  | N/A | 3 |  | N/A |
| cqi-pmi-ConfigurationIndex2 CCSI,1 (Note 14) |  |  | 4 |  | N/A | 4 |  | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  | Multiplexing |  |  |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 and Cell 3 are the aggressor cells. The number of the CRS ports in Cell1, Cell2, and Cell3 is the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.2.5 Note 11: Reference measurement channel in Cell 1 RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 12: To avoid collisions between HARQ-ACK and wideband CQI it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 13: cqi-pmi-ConfigurationIndex is applied for CCSI,0. Note 14: cqi-pmi-ConfigurationIndex2 is applied for CCSI,1. |  |  |  |  |  |  |  |  |

#### 9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

The following requirements apply to UE Category 11-12 and DL Category ≥11. For the parameters specified in Table 9.2.1.7-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1A FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1, or the BLER using the transport format indicated by the (median CQI + 1) shall be less than or equal to 0.1 when the highest MCS value of the test case has reached. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

In this test, 4-bit CQI Table 2 in Table 7.2.3-2 in TS 36.213 [6], and Modulation and TBS index table 2 in Table 7.1.7.1-1A for PDSCH in TS 36.213 [6] are applied.

Table 9.2.1.7-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | -1 | 0 | 20 | 21 |
|  |  | dB[mW/15kHz] | -99 | -98 | -78 | -77 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.1A FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

The following requirements apply to UE Category 11-12 and UE DL Category ≥11. For the parameters specified in Table 9.2.1.8-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1A TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1, or the BLER using the transport format indicated by the (median CQI + 1) shall be less than or equal to 0.1 when the highest MCS value of the test case has reached. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

In this test, 4-bit CQI Table 2 in Table 7.2.3-2 in TS 36.213 [6], and Modulation and TBS index table 2 in Table 7.1.7.1-1A for PDSCH in TS 36.213 [6] are applied.

Table 9.2.1.8-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | -1 | 0 | 20 | 21 |
|  |  | dB[mW/15kHz] | -99 | -98 | -78 | -77 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.1A TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

#### 9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)

The following requirements apply to UE DL Category 20 and DL Category ≥22. For the parameters specified in Table 9.2.1.9-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.X FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

In this test, 4-bit CQI Table 4 in Table 7.2.3-4 in TS 36.213 [6], and Modulation and TBS index table 3 in Table 7.1.7.1-1B for PDSCH in TS 36.213 [6] are applied.

Table 9.2.1.9-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |
| SNR (Note 2) |  | dB | 28 | 29 |
|  |  | dB[mW/15kHz] | -70 | -69 |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |
| Note 1: Reference measurement channel RC.30 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |

#### 9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)

The following requirements apply to UE DL Category 20 and DL Category ≥22. For the parameters specified in Table 9.2.1.10-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.X TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

In this test, 4-bit CQI Table 3 in Table 7.2.3-4 in TS 36.213 [6], and Modulation and TBS index table 3 in Table 7.1.7.1-1B for PDSCH in TS 36.213 [6] are applied.

Table 9.2.1.10-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |
| SNR (Note 2) |  | dB | 28 | 29 |
|  |  | dB[mW/15kHz] | -70 | -69 |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: Reference measurement channel RC.30 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |

### 9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

#### 9.2.2.1	FDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.2.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.2.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 010000 |  |  |  |
| SNR (Note 2) |  | dB | 10 | 11 | 16 | 17 |
|  |  | dB[mW/15kHz] | -88 | -87 | -82 | -81 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigIndex |  |  | 1 (Note 3) |  |  |  |
| Note 1: Reference measurement channel RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: It is intended to have UL collisions between RI reports and HARQ-ACK, since the RI reports shall not be used by the eNB in this test. |  |  |  |  |  |  |

#### 9.2.2.2	TDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.2.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.2.2-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 010000 |  |  |  |
| SNR (Note 2) |  | dB | 10 | 11 | 16 | 17 |
|  |  | dB[mW/15kHz] | -88 | -87 | -82 | -81 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 2 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

### 9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

#### 9.2.3.1	FDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.3.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.3.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |  |  |
| SNR (Note 2) |  | dB | 7 | 8 | 13 | 14 |
|  |  | dB[mW/15kHz] | -91 | -90 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| Note 1: Reference measurement channel RC.7 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. |  |  |  |  |  |  |

#### 9.2.3.1A	FDD (With channelMeasRestriction configured)

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.3.1A-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.3.1A-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| e-MIMO Type |  |  | Class B |  |  |  |
| Number of CSI-RS resource (K) |  |  | 1 |  |  |  |
| channelMeasRestriction |  |  | Enable |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |  |  |
| SNR (Note 2) |  | dB | [7] | [8] | [13] | [14] |
|  |  | dB[mW/15kHz] | -91 | -90 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| PDSCH scheduled sub-frames |  |  | 1,2,3,4,7,8,9 |  |  |  |
| NOTE 1: Reference measurement channel RC.7 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. NOTE 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. NOTE 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink #5. NOTE 4: In sub-frame 6, transmission power of CSI-RS REs is 9dB lower than CRS REs, in sub-frame 1, there is no power offset between CSI-RS REs and CRS REs. |  |  |  |  |  |  |

#### 9.2.3.2	TDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.3.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.3.2-1: PUCCH 1-1 submode 1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 |  |  |  |
| SNR (Note 2) |  | dB | 4 | 5 | 10 | 11 |
|  |  | dB[mW/15kHz] | -94 | -93 | -88 | -87 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.7 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

#### 9.2.3.2A	TDD (With channelMeasRestriction configured)

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.3.2A-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.3.2A-1: PUCCH 1-1 submode 1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| e-MIMO Type |  |  | Class B |  |  |  |
| Number of CSI-RS resource (K) |  |  | 1 |  |  |  |
| channelMeasRestriction |  |  | Enable |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 |  |  |  |
| SNR (Note 2) |  | dB | [4] | [5] | ]10] | [11] |
|  |  | dB[mW/15kHz] | -94 | -93 | -88 | -87 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 13 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| PDSCH scheduled sub-frames |  |  | 3,4,9 |  |  |  |
| NOTE 1: Reference measurement channel RC.7 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. NOTE 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. NOTE 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7. NOTE 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. NOTE 5: In sub-frame 8, transmission power of CSI-RS REs is 9dB lower than CRS REs, in sub-frame 3, there is no power offset between CSI-RS REs and CRS REs. |  |  |  |  |  |  |

### 9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

If UE supporting interferenceMeasRestriction, test cases specified in 9.2.4.1A and 9.2.4.2A are applicable for such UE otherwise test cases specified in 9.2.4.1 and 9.2.4.2 are applied.

#### 9.2.4.1	FDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.4.1-1, and using the downlink physical channels specified in Tables C.3.4-1 and C.3.4-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.4.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 |  | TP1 | TP2 |  |
| Bandwidth |  | MHz | 10 |  |  |  |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |  |  |  |
| Downlink power allocation (Note 1) |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  | Pc | dB | -3 | -3 |  | -3 | -3 |  |
|  |  | dB | -3 | N/A |  | -3 | N/A |  |
| Cell ID |  |  | 0 |  |  | 0 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 | (Note 2) |  | Antenna ports 0, 1 | (Note 2) |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 | N/A |  | Antenna ports 15,…,18 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 | N/A |  | 5/1 | N/A |  |
| CSI-RS configuration |  |  | 0 | N/A |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 1 / 0010000000000000 | 1 / 1000000000000000 |  | 1 / 0010000000000000 | 1 / 1000000000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 1 / 0010000000000000 | N/A |  | 1 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 2) | Clause B.1 (2 x 2) |  | Clause B.1 (4 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 | 100000 |  | 0x0000 0000 0100 0000 | 100000 |  |
| SNR (Note 3) |  | dB | 20 | 6 | 7 | 20 | 14 | 15 |
|  |  | dB[mW/15kHz] | -78 | -92 | -91 | -78 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  |  | -98 |  |  |
| Modulation / Information bit payload |  |  | (Note4) | QPSK / 4392 |  | (Note4) | QPSK / 4392 |  |
| Max number of HARQ transmissions |  |  | 1 | N/A |  | 1 | N/A |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note5) | N/A |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 | N/A |  | 2 | N/A |  |
| PUCCH Report Type for RI |  |  | 3 | N/A |  | 3 | N/A |  |
| Reporting periodicity |  | ms | Npd = 5 | N/A |  | Npd = 5 | N/A |  |
| CQI Delay |  | ms | 8 | N/A |  | 8 | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 | N/A |  | 2 | N/A |  |
| ri-ConfigIndex |  |  | 1 | N/A |  | 1 | N/A |  |
| PDSCH scheduled sub-frames |  |  | 1,2,3,4,6,7,8,9 |  |  | 1,2,3,4,6,7,8,9 |  |  |
| Timing offset between TPs |  | us | 0 |  |  | 0 |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  | 0 |  |  |
| Note1: Reference measurement channel RC.10 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: REs for antenna ports 0 and 1 CRS have zero transmission power. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: N/A. Note 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. |  |  |  |  |  |  |  |  |

#### 9.2.4.1A	FDD (With interferenceMeasRestriction configured)

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.4.1A-1, and using the downlink physical channels specified in Tables C.3.4-1 and C.3.4-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.4.1A-1: PUCCH 1-1 static test (FDD)

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TP1 | TP2 |  | TP1 | TP2 |  |
| Bandwidth |  |  | MHz | 10 |  |  |  |  |  |
| PDSCH transmission mode |  |  |  | 10 |  |  |  |  |  |
| Downlink power allocation (Note 1) |  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  | Pc | dB | -3 | -3 |  | -3 | -3 |  |
|  |  |  | dB | -3 | N/A |  | -3 | N/A |  |
| Cell ID |  |  |  | 0 |  |  | 0 |  |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0, 1 | (Note 2) |  | Antenna ports 0, 1 | (Note 2) |  |
| e-MIMO Type |  |  |  | Class B |  |  |  |  |  |
| Number of CSI-RS resource (K) |  |  |  | 1 |  |  |  |  |  |
| interferenceMeasRestriction |  |  |  | Enable |  |  |  |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,…,18 | N/A |  | Antenna ports 15,…,18 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/1 | N/A |  | 5/1 | N/A |  |
| CSI-RS configuration |  |  |  | 0 | N/A |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | 1 / 0010000000000000 | 1 / 1000000000000000 |  | 1 / 0010000000000000 | 1 / 1000000000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | 1 / 0010000000000000 | N/A |  | 1 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (4 x 2) | Clause B.1 (2 x 2) |  | Clause B.1 (4 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 0x0000 0000 0100 0000 | 100000 |  | 0x0000 0000 0100 0000 | 100000 |  |
| SNR (Note 3) | Sub-frame 6 |  | dB | 20 | [15] | [16] | 20 | [23] | [24] |
|  | Other sub-frames |  |  | 20 | 6 | 7 | 20 | 14 | 15 |
|  | Sub-frame 6 |  | dB[mW/15kHz] | -78 | [-83] | [-82] | -78 | [-75] | [-74] |
|  | Other sub-frames |  |  | -78 | -92 | -91 | -78 | -84 | -83 |
|  |  |  | dB[mW/15kHz] | -98 |  |  | -98 |  |  |
| Modulation / Information bit payload |  |  |  | (Note4) | QPSK / 4392 |  | (Note4) | QPSK / 4392 |  |
| Max number of HARQ transmissions |  |  |  | 1 | N/A |  | 1 | N/A |  |
| Physical channel for CQI/PMI reporting |  |  |  | PUSCH (Note5) | N/A |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/PMI |  |  |  | 2 | N/A |  | 2 | N/A |  |
| PUCCH Report Type for RI |  |  |  | 3 | N/A |  | 3 | N/A |  |
| Reporting periodicity |  |  | ms | Npd = 10 | N/A |  | Npd = 10 | N/A |  |
| CQI Delay |  |  | ms | 8 | N/A |  | 8 | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  | 12 | N/A |  | 12 | N/A |  |
| ri-ConfigIndex |  |  |  | 1 | N/A |  | 1 | N/A |  |
| PDSCH scheduled sub-frames |  |  |  | 1,2,3,4,7,8,9 | 1,2,3,4,6,7,8,9 |  | 1,2,3,4,7,8,9 | 1,2,3,4,6,7,8,9 |  |
| Timing offset between TPs |  |  | us | 0 |  |  | 0 |  |  |
| Frequency offset between TPs |  |  | Hz | 0 |  |  | 0 |  |  |
| NOTE1: Reference measurement channel RC.10 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. NOTE 2: REs for antenna ports 0 and 1 CRS have zero transmission power. NOTE 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. NOTE 4: N/A. NOTE 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink #5. |  |  |  |  |  |  |  |  |  |

#### 9.2.4.2	TDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.4.2-1, and using the downlink physical channels specified in Tables C.3.4-1 and C.3.4-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.4.2-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 |  | TP1 | TP2 |  |
| Bandwidth |  | MHz | 10 |  |  |  |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |  |  |
| Downlink power allocation (Note 1) |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  | Pc | dB | -6 | -6 |  | -6 | -6 |  |
|  |  | dB | -3 | N/A |  | -3 | N/A |  |
| Cell ID |  |  | 0 |  |  | 0 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 | (Note 2) |  | Antenna ports 0, 1 | (Note 2) |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 | N/A |  | Antenna ports 15,…,22 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 | N/A |  | 5/3 | N/A |  |
| CSI-RS configuration |  |  | 0 | N/A |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010000000000000 | N/A |  | 3 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  |
| SNR (Note 3) |  | dB | 17 | 6 | 7 | 17 | 14 | 15 |
|  |  | dB[mW/15kHz] | -81 | -92 | -91 | -81 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  |  | -98 |  |  |
| Modulation / Information bit payload |  |  | (Note4) | QPSK / 4392 |  | (Note4) | QPSK / 4392 |  |
| Max number of HARQ transmissions |  |  | 1 | N/A |  | 1 | N/A |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note5) | N/A |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b | N/A |  | 2b | N/A |  |
| Physical channel for RI reporting |  |  | PUSCH | N/A |  | PUSCH | N/A |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 | N/A |  | 5 | N/A |  |
| Reporting periodicity |  | ms | Npd = 5 | N/A |  | Npd = 5 | N/A |  |
| CQI Delay |  | ms | 10 or 11 | N/A |  | 10 or 11 | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 | N/A |  | 3 | N/A |  |
| ri-ConfigIndex |  |  | 805 (Note 6) | N/A |  | 805 (Note 6) | N/A |  |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |  | Multiplexing | N/A |  |
| PDSCH scheduled sub-frames |  |  | 3,4,8,9 |  |  | 3,4,8,9 |  |  |
| Timing offset between TPs |  | us | 0 |  |  | 0 |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  | 0 |  |  |
| Note1: Reference measurement channel RC.10 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: REs for antenna ports 0 and 1 CRS have zero transmission power. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: N/A. Note 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 6: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |  |  |

#### 9.2.4.2A	TDD (With interferenceMeasRestriction configured)

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.2.4.2A-1, and using the downlink physical channels specified in Tables C.3.4-1 and C.3.4-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.2.4.2A-1: PUCCH 1-1 static test (TDD)

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TP1 | TP2 |  | TP1 | TP2 |  |
| Bandwidth |  |  | MHz | 10 |  |  |  |  |  |
| PDSCH transmission mode |  |  |  | 10 |  |  |  |  |  |
| Uplink downlink configuration |  |  |  | 2 |  |  |  |  |  |
| Special subframe configuration |  |  |  | 4 |  |  |  |  |  |
| Downlink power allocation (Note 1) |  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  |  | dB | 0 | 0 |  | 0 | 0 |  |
|  |  | Pc | dB | -6 | -6 |  | -6 | -6 |  |
|  |  |  | dB | -3 | N/A |  | -3 | N/A |  |
| Cell ID |  |  |  | 0 |  |  | 0 |  |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0, 1 | (Note 2) |  | Antenna ports 0, 1 | (Note 2) |  |
| e-MIMO Type |  |  |  | Class B |  |  |  |  |  |
| Number of CSI-RS resource (K) |  |  |  | 1 |  |  |  |  |  |
| interferenceMeasRestriction |  |  |  | Enable |  |  |  |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,…,22 | N/A |  | Antenna ports 15,…,22 | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/3 | N/A |  | 5/3 | N/A |  |
| CSI-RS configuration |  |  |  | 0 | N/A |  | 0 | N/A |  |
| Zero-Power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  | 3 / 0010000000000000 | 3 / 1000010000000000 |  |
| CSI-IM configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | 3 / 0010000000000000 | N/A |  | 3 / 0010000000000000 | N/A |  |
| CSI process configuration Signal/Interference/Reporting mode |  |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  | CSI-RS/CSI-IM/PUCCH 1-1 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  | Clause B.1 (8 x 2) | Clause B.1 (2 x 2) |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  | 0x0000 0000 0020 0000 0000 0001 0000 | 100000 |  |
| SNR (Note 3) | Sub-frame 8 |  | dB | 17 | [15] | [16] | 17 | [23] | [24] |
|  | Other sub-frames |  |  | 17 | 6 | 7 | 17 | 14 | 15 |
|  | Sub-frame 8 |  | dB[mW/15kHz] | -81 | [-83] | [-82] | -81 | [-75] | [-74] |
|  | Other sub-frames |  |  | -81 | -92 | -91 | -81 | -84 | -83 |
|  |  |  | dB[mW/15kHz] | -98 |  |  | -98 |  |  |
| Modulation / Information bit payload |  |  |  | (Note4) | QPSK / 4392 |  | (Note4) | QPSK / 4392 |  |
| Max number of HARQ transmissions |  |  |  | 1 | N/A |  | 1 | N/A |  |
| Physical channel for CQI/PMI reporting |  |  |  | PUSCH (Note5) | N/A |  | PUSCH (Note5) | N/A |  |
| PUCCH Report Type for CQI/second PMI |  |  |  | 2b | N/A |  | 2b | N/A |  |
| Physical channel for RI reporting |  |  |  | PUSCH | N/A |  | PUSCH | N/A |  |
| PUCCH Report Type for RI/ first PMI |  |  |  | 5 | N/A |  | 5 | N/A |  |
| Reporting periodicity |  |  | ms | Npd = 10 | N/A |  | Npd = 10 | N/A |  |
| CQI Delay |  |  | ms | 10 or 11 | N/A |  | 10 or 11 | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  | 13 | N/A |  | 13 | N/A |  |
| ri-ConfigIndex |  |  |  | 805 (Note 6) | N/A |  | 805 (Note 6) | N/A |  |
| ACK/NACK feedback mode |  |  |  | Multiplexing | N/A |  | Multiplexing | N/A |  |
| PDSCH scheduled sub-frames |  |  |  | 3,4,9 | 3,4,8,9 |  | 3,4,9 | 3,4,8,9 |  |
| Timing offset between TPs |  |  | us | 0 |  |  | 0 |  |  |
| Frequency offset between TPs |  |  | Hz | 0 |  |  | 0 |  |  |
| NOTE 1: Reference measurement channel RC.10 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. NOTE 2: REs for antenna ports 0 and 1 CRS have zero transmission power. NOTE 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. NOTE 4: N/A. NOTE 5: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7. NOTE 6: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |  |  |  |

### 9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)

The following requirements apply to UE Category ≥2 which supports eIMTA TDD UL-DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI and Rel-12 CSI subframe sets. For the parameters specified in table 9.2.5-1, and using the downlink physical channels specified in Tables C.3.2-1 and C.3.2-2, for each CSI subframe set, the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time. For each CSI subframe set, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1. The difference of the median CQI obtained by reports in CSI subframe sets CCSI,0 and the median CQI obtained by reports in CSI subframe sets CCSI,1 shall be larger than or equal to 3.

Table 9.2.5-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 9 |  |
| Uplink downlink configuration in SIB1 |  |  | 0 |  |
| Downlink HARQ reference configuration (eimta-HarqReferenceConfig-r12) (Note 4) |  |  | 2 |  |
| Set of dynamic TDD UL-DL configurations (Notes 4,5) |  |  | {0, 2} |  |
| Periodicity of monitoring the L1 reconfiguration DCI (eimta-CommandPeriodicity-r12) |  | ms | 10 |  |
| Set of subframes to monitor the L1 reconfiguration DCI (eimta-CommandSubframeSet-r12) |  |  | SF#5 |  |
| CSI-MeasSubframeSet-r12 |  |  | 0001100011 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | -3 |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/4 |  |
| CSI reference signal configuration |  |  | 4 |  |
| Zero-Power CSI-RS configuration 0 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 0 / 0000010000000000 |  |
| Zero-Power CSI-RS configuration 1 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0100000000000000 |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CodeBookSubsetRestriction bitmap |  |  | '000001' |  |
| SNR in CSI subframe set 0 |  | dB | 0 | 1 |
| SNR in CSI subframe set 1 |  | dB | 10 | 11 |
|  |  | dB[mW/15kHz] | -98 | -97 |
| for CSI subframe set 0 |  | dB[mW/15kHz] | -98 | -98 |
| for CSI subframe set 1 |  | dB[mW/15kHz] | -108 | -108 |
| PDSCH scheduled subframes for CSI subframe set 0 |  |  | 0,5 |  |
| PDSCH scheduled subframes for CSI subframe set 1 |  |  | 3,4,8,9 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 6) |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |
| Physical channel for RI reporting |  |  | PUSCH |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |
| Reporting periodicity |  | ms | Npd = 10 for each Rel-12 CSI subframe set |  |
| CQI delay |  | ms | 14 for CSI subframe set 0 12 for CSI subframe set 1 |  |
| cqi-pmi-ConfigurationIndex |  |  | 8 for set 0 13 for set 1 |  |
| ri-ConfigIndex |  |  | 805 for both set 0 and set 1 (Note 7) |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: Reference measurement channel RC.19 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and dynamic OCNG Pattern with multiple non-contiguous blocks OP.7 TDD as described in Annex A.5.2.1/7 for CSI subframe set 0. Note 2: Reference measurement channel RC.20 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 for CSI subframe set 1. Note 3: In the test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level for each CSI subframe set separately. Note 4: As specified in Table 4.2-2 in TS 36.211. Note 5: UL/DL configuration in PDCCH with eIMTA-RNTI is cyclically selected from the given set on a per-DCI basis. Note 6: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. CQI/PMI reports for CSI subframe set 0 is transmitted in SF#2 and CQI/PMI reports for CSI subframe set 1 is transmitted in SF#7. Note 7: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |

### 9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)

#### 9.2.6.1	Frame structure type 3 with FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.1-1, Table 9.2.6.1-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.6.1-1: Parameters for PUSCH 3-0 static test on FDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 3 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-0 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. One sided dynamic OCNG Pattern OP.1 FDD as described in A.5.1.1 is transmitted on PCell on all RBs, but PDSCH for user data is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 1 and subframe 6 with 5ms periodicity. |  |  |  |

Table 9.2.6.1-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |
|  |  |  | dB | -3 |  |  |
|  |  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |
| SNR in subframes with 6 dB power boost (Note 2) |  |  | dB | 9 |  | 10 |
| SNR in subframes with 0 dB power boost (Note 2) |  |  | dB | 3 |  | 4 |
| in subframes with 6 dB power |  |  | dB[mW/15kHz] | -89 |  | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 |  | -94 |
|  |  |  | dB[mW/15kHz] | -98 |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |
| Cell Id |  |  |  | 0 |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.8 |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3,8} |  |  |
|  | Random variable p defined in B.8 |  |  | 0.5 |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |  |
| Note 1: Reference measurement channel RC.2A FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FS3 as described in Annex A.5.4.1, except for category 1 UE use RC.4A FDD with two sided dynamic OCNG Pattern OP.2 FS3 as described in Annex A.5.4.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.2.6.2	Frame structure type 3 with TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.2-1, Table 9.2.6.2-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.6.2-1: Parameters for PUSCH 3-0 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| PDSCH transmission mode |  |  | 3 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-0 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. One sided dynamic OCNG Pattern OP.1 TDD as described in A.5.2.1 is transmitted on PCell on all RBs, but PDSCH for user data is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 3 and subframe 8 with 5ms periodicity. |  |  |  |

Table 9.2.6.2-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |
|  |  |  | dB | -3 |  |  |
|  |  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |
| SNR in subframes with 6 dB power boost (Note 2) |  |  | dB | 9 |  | 10 |
| SNR in subframes with 0 dB power boost (Note 2) |  |  | dB | 3 |  | 4 |
| in subframes with 6 dB power |  |  | dB[mW/15kHz] | -89 |  | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 |  | -94 |
|  |  |  | dB[mW/15kHz] | -98 |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |
| Cell Id |  |  |  | 0 |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |
|  | The number of subframes set (S1S1) per burst |  |  | {3,8} |  |  |
|  | Random variable p defined in B.8 |  |  | 0.5 |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |  |
| Note 1: Reference measurement channel RC.2A FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FS3 as described in Annex A.5.4.1, except for category 1 UE use RC.4A FDD with two sided dynamic OCNG Pattern OP.2 FS3 as described in Annex A.5.4.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

### 9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

#### 9.2.7.1	Frame structure type 3 wth FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.1-1, Table 9.2.7.1-2and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median for second set of CQI shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.7.1-1: Parameters for PUSCH 3-1 static test on FDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. One sided dynamic OCNG Pattern OP.1 FDD as described in A.5.1.1 is transmitted on PCell on all RBs, but PDSCH for user data is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 1 and subframe 6 with 5ms periodicity. |  |  |  |

Table 9.2.7.1-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |
| Transmission mode |  |  |  | 9 |  |
| Downlink power allocation |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
| SNR in subframes with 6 dB power boost (Note 3) |  |  | dB | 9 | 10 |
| SNR in subframes with 0 dB power boost (Note 3) |  |  |  | 3 | 4 |
| in subframes with 6 dB power |  |  |  | -89 | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 | -94 |
|  |  |  | dB[mW/15kHz] | -98 | -98 |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |
| Cell Id |  |  |  | 0 |  |
| dmtc-Periodicity |  |  | ms | 80 |  |
| dmtc-Offset |  |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 1 |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |
| Number of control OFDM symbols |  |  |  | 3 |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.8 |  |
|  | subframeStartPosition |  |  | s0 |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |
|  | The number of subframes set (S1) per burst |  |  | {3, 8} |  |
|  | Random variable p defined in B.8 |  |  | 0.5 |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.9A FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FS3 as described in Annex A.5.4.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |

#### 9.2.7.2	Frame structure type 3 wth TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.2-1, Table 9.2.7.2-2and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median for second set of CQI shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.7.2-1: Parameters for PUSCH 3-1 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. One sided dynamic OCNG Pattern OP.1 TDD as described in A.5.2.1 is transmitted on PCell on all RBs, but PDSCH for user data is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 3 and subframe 8 with 5ms periodicity. |  |  |  |

Table 9.2.7.2-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |
| Transmission mode |  |  |  | 9 |  |
| Downlink power allocation |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
| SNR in subframes with 6 dB power boost (Note 3) |  |  | dB | 9 | 10 |
| SNR in subframes with 0 dB power boost (Note 3) |  |  |  | 3 | 4 |
| in subframes with 6 dB power |  |  |  | -89 | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 | -94 |
|  |  |  | dB[mW/15kHz] | -98 | -98 |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |
| Cell Id |  |  |  | 0 |  |
| dmtc-Periodicity |  |  | ms | 80 |  |
| dmtc-Offset |  |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 3 |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |
| Number of control OFDM symbols |  |  |  | 3 |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.8 |  |
|  | subframeStartPosition |  |  | s0 |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |
|  | The number of subframes set (S1S1) per burst |  |  | {3, 8} |  |
|  | Random variable p defined in B.8 |  |  | 0.5 |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.9A FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FS3 as described in Annex A.5.4.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |

## 9.3	CQI reporting under fading conditions

### 9.3.1	Frequency-selective scheduling mode

The accuracy of sub-band channel quality indicator (CQI) reporting under frequency selective fading conditions is determined by a double-sided percentile of the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level the corresponding transport format compared to the case for which a fixed format is transmitted on any sub-band in set S of TS 36.213 [6]. The purpose is to verify that preferred sub-bands can be used for frequently-selective scheduling. To account for sensitivity of the input SNR the sub-band CQI reporting under frequency selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)

##### 9.3.1.1.1	FDD

For the parameters specified in Table 9.3.1.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.1.1-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD.

Table 9.3.1.1.1-1 Sub-band test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 9 | 10 | 14 | 15 |
|  |  | dB[mW/15kHz] | -89 | -88 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 2 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.3.1.1.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.1.2	TDD

For the parameters specified in Table 9.3.1.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.1.2-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD.

Table 9.3.1.1.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 9 | 10 | 14 | 15 |
|  |  | dB[mW/15kHz] | -89 | -88 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 2 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.3.1.1.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)

For the parameters specified in Table 9.3.1.1.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.1.3-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput in ABS subframes obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER in ABS subframes for the indicated transport formats shall be greater than or equal to ε.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD.

Table 9.3.1.1.3-1 Sub-band test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 and 3 | Cell 1 |  | Cell 2 and 3 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 1 |  | Note 10 | 1 |  | Note 10 |
| Downlink power allocation |  | dB | 0 |  |  | 0 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition |  |  | Clause B.2.4 with Td = 0.45 us, a = 1, fd = 5 Hz |  | EVA5 Low antenna correlation | Clause B.2.4 with Td = 0.45 us, a = 1, fd = 5 Hz |  | EVA5 Low antenna correlation |
| Antenna configuration |  |  | 1x2 |  |  | 1x2 |  |  |
| (Note 1) |  | dB | 4 | 5 | Cell 2: 12 Cell 3: 10 | 14 | 15 | Cell 2: 12 Cell 3: 10 |
| at antenna port |  | dBm/15kHz | -98 (Note 7) |  | N/A | -98 (Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98 (Note 8) |  | N/A |
|  |  | dBm/15kHz | -93 (Note 9) |  | N/A | -93 (Note 9) |  | N/A |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | Cell 2: 6 Cell 3: 1 | 0 |  | Cell 2: 6 Cell 3: 1 |
| Time Offset between Cells |  | s | Cell 2: 3 usec Cell 3: -1usec |  |  | Cell 2: 3 usec Cell 3: -1usec |  |  |
| Frequency Shift between Cells |  | Hz | Cell 2: 300Hz Cell 3: -100Hz |  |  | Cell 2: 300Hz Cell 3: -100Hz |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 01010101 01010101 01010101 01010101 01010101 | N/A |  | 01010101 01010101 01010101 01010101 01010101 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 00000100 00000100 00000100 00000100 00000100 |  | N/A | 00000100 00000100 00000100 00000100 00000100 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 01010101 01010101 01010101 01010101 01010101 |  | N/A | 01010101 01010101 01010101 01010101 01010101 |  | N/A |
|  | CCSI,1 |  | 10101010 10101010 10101010 10101010 10101010 |  | N/A | 10101010 10101010 10101010 10101010 10101010 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| CQI delay |  | ms | 8 |  |  |  |  |  |
| Reporting interval (Note 13) |  | ms | 10 |  |  |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |  |  |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7] Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 and Cell 3 are the aggressor cells. The number of the CRS ports in Cell1, Cell2, and Cell3 are the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.1.5 Note 11: Reference measurement channel in Cell 1 RC.3 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 12: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 13: The CSI reporting is such that reference subframes belong to Ccsi,0. |  |  |  |  |  |  |  |  |

Table 9.3.1.1.3-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| ε | 0.01 | 0.01 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)

For the parameters specified in Table 9.3.1.1.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.1.4-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput in ABS subframes obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER in ABS subframes for the indicated transport formats shall be greater than or equal to ε.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD.

Table 9.3.1.1.4-1: Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 and 3 | Cell 1 |  | Cell 2 and 3 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |  |
| PDSCH transmission mode |  |  | 1 |  | Note 10 | 1 |  | Note 10 |
| Uplink downlink configuration |  |  | 1 |  |  | 1 |  |  |
| Special subframe configuration |  |  | 4 |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  | 0 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
|  |  | dB | 0 |  |  | 0 |  |  |
| Propagation condition |  |  | Clause B.2.4 with Td = 0.45 us, a = 1, fd = 5 Hz |  | EVA5 Low antenna correlation | Clause B.2.4 with Td = 0.45 us, a = 1, fd = 5 Hz |  | EVA5 Low antenna correlation |
| Antenna configuration |  |  | 1x2 |  |  | 1x2 |  |  |
| (Note 1) |  | dB | 4 | 5 | Cell 2: 12 Cell 3: 10 | 14 | 15 | Cell 2: 12 Cell 3: 10 |
| at antenna port |  | dBm/15kHz | -98 (Note 7) |  | N/A | -98 (Note 7) |  | N/A |
|  |  | dBm/15kHz | -98 (Note 8) |  | N/A | -98 (Note 8) |  | N/A |
|  |  | dBm/15kHz | -93 (Note 9) |  | N/A | -93 (Note 9) |  | N/A |
| Subframe Configuration |  |  | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN |
| Cell Id |  |  | 0 |  | Cell 2: 6 Cell 3: 1 | 0 |  | Cell 2: 6 Cell 3: 1 |
| Time Offset between Cells |  | s | Cell 2: 3 usec Cell 3: -1usec |  |  | Cell 2: 3 usec Cell 3: -1usec |  |  |
| Frequency shift between Cells |  | Hz | Cell 2: 300Hz Cell 3: -100Hz |  |  | Cell 2: 300Hz Cell 3: -100Hz |  |  |
| ABS pattern (Note 2) |  |  | N/A |  | 0100010001 0100010001 | N/A |  | 0100010001 0100010001 |
| RLM/RRM Measurement Subframe Pattern (Note 4) |  |  | 0000000001 0000000001 |  | N/A | 0000000001 0000000001 |  | N/A |
| CSI Subframe Sets (Note 3) | CCSI,0 |  | 0100010001 0100010001 |  | N/A | 0100010001 0100010001 |  | N.A |
|  | CCSI,1 |  | 1000101000 1000101000 |  | N/A | 1000101000 1000101000 |  | N/A |
| Number of control OFDM symbols |  |  | 3 |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  | 1 |  |  |
| CQI delay |  | ms | 10 |  |  |  |  |  |
| Reporting interval (Note 13) |  | ms | 10 |  |  |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  | Multiplexing |  |  |
| Note 1: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 2: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 3: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 4: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 5: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 6: Cell 1 is the serving cell. Cell 2 and Cell 3 are the aggressor cells. The number of the CRS ports in Cell1, Cell2, and Cell3 is the same. Note 7: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 8: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS Note 9: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS. Note 10: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.2.5 Note 11: Reference measurement channel in Cell 1 RC.3 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 12: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 13: The CSI reporting is such that reference subframes belong to Ccsi,0. |  |  |  |  |  |  |  |  |

Table 9.3.1.1.4-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| ε | 0.01 | 0.01 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)

The following requirements apply to UE Category ≥1 which supports Rel-12 CSI subframe sets. For the parameters specified in Table 9.3.1.1.5-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.1.5-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band for each CSI subframe set;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ for each CSI subframe set;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05 and less than  0.60 for each CSI subframe set.

d) the difference of the wide-band median CQI obtained by reports in CSI subframe sets CCSI,0 and the wide-band median CQI obtained by reports in CSI subframe sets CCSI,1 shall be larger than or equal to 3.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.1.1.5-1: Sub-band test for TDD

| Parameter |  | Unit | Test |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Transmission mode |  |  | 2 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| CSI-MeasSubframeSet-r12 |  |  | 0001100000 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| SNR in CSI subframe set 0 |  | dB | 0 | 1 |
| SNR in CSI subframe set 1 |  | dB | 10 | 11 |
|  |  | dB[mW/15kHz] | -98 | -97 |
| for CSI subframe set 0 |  | dB[mW/15kHz] | -98 | -98 |
| for CSI subframe set 1 |  | dB[mW/15kHz] | -108 | -108 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| CRS reference signals |  |  | Antenna port 0 and 1 |  |
| Zero-Power CSI-RS configuration 0 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0000010000000000 |  |
| Zero-Power CSI-RS configuration 1 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0100000000000000 |  |
| PDSCH scheduled subframes for CSI subframe set 0 |  |  | 8,9 |  |
| PDSCH scheduled subframes for CSI subframe set 1 |  |  | 3,4 |  |
| Reporting interval (Note 4) |  | ms | 10 per subframe set |  |
| CQI delay |  | ms | 15 for CSI subframe set 0 15 for CSI subframe set 1 |  |
| Reporting mode |  |  | PUSCH 3-0 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Number of EPDCCH Sets Configured |  |  | 2 (Note 5,6) |  |
| Number of PRB per EPDCCH Set |  |  | 4 |  |
| EPDCCH Subframe Monitoring |  |  | NA |  |
| EPDCCH Aggregation level |  |  | 8ECCE |  |
| EPDCCH beamforming model |  |  | Annex B.4.4 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.17 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: In the test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level for each subframe set separately.. Note 4: For CSI subframe set 0, PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF #7. For CSI subframe set 1, PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2. Note 5: In case UE supports EPDCCH, the PDSCH scheduling grants are transmitted via EPDCCH, otherwise PDCCH is used. Note 6: The two sets are distributed EPDCCH sets and non-overlapping with PRB = {0, 3, 6, 9} for the first set and PRB = {40, 43, 46, 49} for the second set. EPDCCH set is selected after scheduling decision for PDSCH to avoid collision between PDSCH and EPDCCH PRBs, respectively. EPDCCH is only transmitted from one set. The starting symbol for EPDCCH is derived from the PCFICH. RRC signalling epdcch-StartSymbol-r11is not configured |  |  |  |  |

Table 9.3.1.1.5-2: Minimum requirement (TDD)

|  | Test |
| --- | --- |
|  [%] | 2 |
|  [%] | 55 |
|  | 1.1 |
| UE Category | ≥1 |

#### 9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

##### 9.3.1.2.1	FDD

For the parameters specified in Table 9.3.1.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.2.1-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.1.2.1-1 Sub-band test for FDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 4 | 5 | 11 | 12 |
|  |  | dB[mW/15kHz] | -94 | -93 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |  |  |  |
| CSI-RS reference signal configuration |  |  | 4 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |  |  |
| Reporting interval (Note 4) |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#1 and #6 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#0 and #5. |  |  |  |  |  |  |

Table 9.3.1.2.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 40 | 40 |
|  | 1.1 | 1.1 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.2.2	TDD

For the parameters specified in Table 9.3.1.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.2.2-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.1.2.2-1 Sub-band test for TDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 4 | 5 | 11 | 12 |
|  |  | dB[mW/15kHz] | -94 | -93 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 2x2 |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CRS reference signals |  |  | Antenna port 0 |  |  |  |
| CSI reference signals |  |  | Antenna port 15,16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI-RS reference signal configuration |  |  | 4 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |  |  |
| Reporting interval (Note 4) |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 10 |  |  |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 and #8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2 and #7. |  |  |  |  |  |  |

Table 9.3.1.2.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 40 | 40 |
|  | 1.1 | 1.1 |
| UE Category | ≥1 | ≥1 |

##### 9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

For the parameters specified in Table 9.3.1.2.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.2.3-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

In this test, 4-bit CQI Table 2 in Table 7.2.3-2 in TS 36.213 [6], and Modulation and TBS index table 2 in Table 7.1.7.1-1A for PDSCH in TS 36.213 [6] are applied.

Table 9.3.1.2.3-1 Sub-band test for FDD

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 9 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| SNR (Note 3) |  | dB | 16 | 17 |
|  |  | dB[mW/15kHz] | -82 | -81 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  | Antenna ports 0 |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |  |
| CSI-RS reference signal configuration |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |
| Reporting interval (Note 4) |  | ms | 5 |  |
| CQI delay |  | ms | 8 |  |
| Reporting mode |  |  | PUSCH 3-1 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8A FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#1 and #6 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#0 and #5. |  |  |  |  |

Table 9.3.1.2.3-2 Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  [%] | 2 |
|  [%] | 40 |
|  | 1.1 |
| UE Category | 11-12 |
| UE DL Category | ≥11 |

##### 9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

For the parameters specified in Table 9.3.1.2.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.2.4-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

In this test, 4-bit CQI Table 2 in Table 7.2.3-2 in TS 36.213 [6], and Modulation and TBS index table 2 in Table 7.1.7.1-1A for PDSCH in TS 36.213 [6] are applied.

Table 9.3.1.2.4-1 Sub-band test for TDD

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 MHz |  |
| Transmission mode |  |  | 9 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| SNR (Note 3) |  | dB | 16 | 17 |
|  |  | dB[mW/15kHz] | -82 | -81 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  | Antenna port 0 |  |
| CSI reference signals |  |  | Antenna port 15,16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |
| CSI-RS reference signal configuration |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |
| Reporting interval (Note 4) |  | ms | 5 |  |
| CQI delay |  | ms | 10 |  |
| Reporting mode |  |  | PUSCH 3-1 |  |
| Sub-band size |  | RB | 8 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8A TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 and #8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2 and #7. |  |  |  |  |

Table 9.3.1.2.4-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  [%] | 2 |
|  [%] | 40 |
|  | 1.1 |
| UE Category | 11-12 |
| UE DL Category | ≥11 |

##### 9.3.1.2.5	Void

Table 9.3.1.2.5-1: Void

Table 9.3.1.2.5-2: Void

##### 9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)

The following requirements apply to UE Category ≥1 which supports Rel-12 CSI subframe sets and TM10. For the parameters specified in Table 9.3.1.2.6-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.1.2.6-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band for each CSI subframe set;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ for each CSI subframe set;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.01 for each CSI subframe set.

d) The difference of the wide-band median CQI obtained by reports in CSI subframe sets CCSI,0 and the wide-band median CQI obtained by reports in CSI subframe sets CCSI,1 shall be larger than or equal to 3.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.1.2.6-1: Sub-band test for TDD

| Parameter |  | Unit | Test |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Transmission mode |  |  | 10 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| CSI-MeasSubframeSet-r12 |  |  | 0001100000 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | -3 |  |
|  |  | dB | -3 |  |
| SNR in CSI subframe set 0 |  | dB | 0 | 1 |
| SNR in CSI subframe set 1 |  | dB | 10 | 11 |
|  |  | dB[mW/15kHz] | -98 | -97 |
| for CSI subframe set 0 |  | dB[mW/15kHz] | -98 | -98 |
| for CSI subframe set 1 |  | dB[mW/15kHz] | -108 | -108 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  | Antenna port 0 and 1 |  |
| CSI reference signals |  |  | Antenna port 15,16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 0 |  |
| CSI-RS reference signal configuration |  |  | 0 |  |
| Zero-Power CSI-RS configuration 0 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0000010000000000 |  |
| Zero-Power CSI-RS configuration 1 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0100000000000000 |  |
| CSI-IM configuration 0 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0000010000000000 |  |
| CSI-IM configuration 1 ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 4 / 0100000000000000 |  |
| CSI process configuration Signal/Interference/Reporting mode for CSI subframe set 0 |  |  | CSI-RS/CSI-IM 0/PUSCH 3-1 |  |
| CSI process configuration Signal/Interference/Reporting mode for CSI subframe set 1 |  |  | CSI-RS/CSI-IM 1/PUSCH 3-1 |  |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |  |
| Reporting interval (Note 4) |  | ms | 10 per subframe set |  |
| CQI delay |  | ms | 15 for CSI subframe set 0 15 for CSI subframe set 1 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| PDSCH scheduled subframes for CSI subframe set 0 |  |  | 8,9 |  |
| PDSCH scheduled subframes for CSI subframe set 1 |  |  | 3,4 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.18 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level for each subframe set separately. Note 4: For CSI subframe set 0, PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF #7. For CSI subframe set 1, PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#8 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#2. |  |  |  |  |

Table 9.3.1.2.6-2: Minimum requirement (TDD)

|  | Test |
| --- | --- |
|  [%] | 2 |
|  [%] | 55 |
|  | 1.02 |
| UE Category | ≥1 |

### 9.3.2	Frequency non-selective scheduling mode

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective fading conditions is determined by the reporting variance, and the relative increase of the throughput obtained when the transport format transmitted is that indicated by the reported CQI compared to the case for which a fixed transport format configured according to the reported median CQI is transmitted. In addition, the reporting accuracy is determined by a minimum BLER using the transport formats indicated by the reported CQI. The purpose is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for frequently non-selective scheduling. To account for sensitivity of the input SNR the CQI reporting under frequency non-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)

##### 9.3.2.1.1	FDD

For the parameters specified in Table 9.3.2.1.1-1 and Table 9.3.2.1.1-3, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.2.1.1-2 and Table 9.3.2.1.1-4 and by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02

The applicability of the requirement with 5MHz bandwidth as specificed in Table 9.3.2.1.1-3 and Table 9.3.2.1.1-4 is defined in 9.1.1.1.

Table 9.3.2.1.1-1 Fading test for single antenna (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 6 | 7 | 12 | 13 |
|  |  | dB[mW/15kHz] | -92 | -91 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (1 x 2) |  |  |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  |  |
| Reporting periodicity |  | ms | Npd = 2 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 1 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.1 FDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and RC.4 FDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. |  |  |  |  |  |  |

Table 9.3.2.1.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥1 | ≥1 |

Table 9.3.2.1.1-3 Fading test for single antenna (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 5 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 6 | 7 | 12 | 13 |
|  |  | dB[mW/15kHz] | -92 | -91 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (1 x 2) |  |  |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  |  |
| Reporting periodicity |  | ms | Npd = 2 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 1 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.14 FDD according to Table A.4-1 for Category ≥ 2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and RC.15 FDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2.Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. |  |  |  |  |  |  |

Table 9.3.2.1.1-4 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥1 | ≥1 |

##### 9.3.2.1.2	TDD

For the parameters specified in Table 9.3.2.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.2.1.2-2 and by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

Table 9.3.2.1.2-1 Fading test for single antenna (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 6 | 7 | 12 | 13 |
|  |  | dB[mW/15kHz] | -92 | -91 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (1 x 2) |  |  |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.1 TDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and RC.4 TDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

Table 9.3.2.1.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥1 | ≥1 |

#### 9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)

##### 9.3.2.2.1	FDD

For the parameters specified in Table 9.3.2.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.2.2.1-2 and by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

Table 9.3.2.2.1-1 Fading test for FDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| SNR (Note 3) |  | dB | 2 | 3 | 7 | 8 |
|  |  | dB[mW/15kHz] | -96 | -95 | -91 | -90 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | ULA High (4 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI-RS reference signal configuration |  |  | 2 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0001 |  |  |  |
| Reporting mode |  |  | PUCCH 1-1 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Physical channel for CQI/ PMI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH report type for RI |  |  | 3 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.7 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. |  |  |  |  |  |  |

Table 9.3.2.2.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 |

##### 9.3.2.2.2	TDD

For the parameters specified in Table 9.3.2.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.2.2.2-2 and by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

Table 9.3.2.2.2-1 Fading test for TDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| SNR (Note 3) |  | dB | 1 | 2 | 7 | 8 |
|  |  | dB[mW/15kHz] | -97 | -96 | -91 | -90 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | XP High (8 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI-RS reference signal configuration |  |  | 2 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 0020 0000 0000 0001 |  |  |  |
| Reporting mode |  |  | PUCCH 1-1 (Sub-mode: 2) |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 |  |  |  |
| Physical channel for CQI/ PMI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type for CQI/ PMI |  |  | 2c |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH report type for RI |  |  | 3 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 5) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.7 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#2 and #7. Note 5: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. |  |  |  |  |  |  |

Table 9.3.2.2.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 |

### 9.3.3	Frequency-selective interference

The accuracy of sub-band channel quality indicator (CQI) reporting under frequency selective interference conditions is determined by a percentile of the reported differential CQI offset level +2 for a preferred sub-band, and the relative increase of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level the corresponding transport format compared to the case for which a fixed format is transmitted on any sub-band in set S of TS 36.213 [6]. The purpose is to verify that preferred sub-bands are used for frequently-selective scheduling under frequency-selective interference conditions.

#### 9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)

##### 9.3.3.1.1	FDD

For the parameters specified in Table 9.3.3.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.3.1.1-2 and by the following

a)	a sub-band differential CQI offset level of +2 shall be reported at least  % for at least one of the sub-bands of full size at the channel edges;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.3.1.1-1 Sub-band test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz | 10 MHz |
| Transmission mode |  |  | 1 (port 0) | 1 (port 0) |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
| for RB 0…5 |  | dB[mW/15kHz] | -102 | -93 |
| for RB 6…41 |  | dB[mW/15kHz] | -93 | -93 |
| for RB 42…49 |  | dB[mW/15kHz] | -93 | -102 |
|  |  | dB[mW/15kHz] | -94 | -94 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Reporting interval |  | ms | 5 |  |
| Antenna configuration |  |  | 1 x 2 |  |
| CQI delay |  | ms | 8 |  |
| Reporting mode |  |  | PUSCH 3-0 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. |  |  |  |  |

Table 9.3.3.1.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 60 | 60 |
|  | 1.6 | 1.6 |
| UE Category | ≥1 | ≥1 |

##### 9.3.3.1.2	TDD

For the parameters specified in Table 9.3.3.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.3.1.2-2 and by the following

a)	a sub-band differential CQI offset level of +2 shall be reported at least  % for at least one of the sub-bands of full size at the channel edges;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.3.1.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  |  | Unit |  | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz |  | 10 MHz | 10 MHz |  |
| Transmission mode |  |  |  |  | 1 (port 0) | 1 (port 0) |  |
| Downlink power allocation |  |  | dB |  | 0 | 0 |  |
|  |  |  | dB |  | 0 | 0 |  |
|  |  |  | dB |  | 0 | 0 |  |
| Uplink downlink configuration |  |  |  |  | 2 |  |  |
| Special subframe configuration |  |  |  |  | 4 |  |  |
| for RB 0…5 |  |  | dB[mW/15kHz] |  | -102 | -93 |  |
| for RB 6…41 |  |  | dB[mW/15kHz] |  | -93 | -93 |  |
| for RB 42…49 |  |  | dB[mW/15kHz] |  | -93 | -102 |  |
|  |  |  | dB[mW/15kHz] |  | -94 | -94 |  |
| Max number of HARQ transmissions |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |
| Antenna configuration |  |  |  |  | 1 x 2 |  |  |
| Reporting interval |  |  | ms |  | 5 |  |  |
| CQI delay |  |  | ms |  | 10 or 11 |  |  |
| Reporting mode |  |  |  |  | PUSCH 3-0 |  |  |
| Sub-band size |  |  | RB |  | 6 (full size) |  |  |
| ACK/NACK feedback mode |  |  |  |  | Multiplexing |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.3 TDD according to table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. |  |  |  |  |  |  |  |

Table 9.3.3.1.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 60 | 60 |
|  | 1.6 | 1.6 |
| UE Category | ≥1 | ≥1 |

#### 9.3.3.2	Void

##### 9.3.3.2.1	Void

##### 9.3.3.2.2	Void

### 9.3.4	UE-selected subband CQI

The accuracy of UE-selected subband channel quality indicator (CQI) reporting under frequency-selective fading conditions is determined by the relative increase of the throughput obtained when transmitting on the UE-selected subbands with the corresponding transport format compared to the case for which a fixed format is transmitted on any subband in set S of TS 36.213 [6]. The purpose is to verify that correct subbands are accurately reported for frequency-selective scheduling. To account for sensitivity of the input SNR the subband CQI reporting under frequency-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)

##### 9.3.4.1.1	FDD

For the parameters specified in Table 9.3.4.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.4.1.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting on a randomly selected subband among the best M subbands reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected subband in set S shall be ≥ ;

The requirements only apply for subbands of full size and the random scheduling across the subbands is done by selecting a new subband in each TTI for FDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the subband size.

Table 9.3.4.1.1-1 Subband test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 9 | 10 | 14 | 15 |
|  |  | dB[mW/15kHz] | -89 | -88 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Reporting mode |  |  | PUSCH 2-0 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Subband size (k) |  | RBs | 3 (full size) |  |  |  |
| Number of preferred subbands (M) |  |  | 5 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.5 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.3.4.1.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.2 | 1.2 |
| UE Category | ≥1 | ≥1 |

##### 9.3.4.1.2	TDD

For the parameters specified in Table 9.3.4.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.4.1.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting on a randomly selected subband among the best M subbands reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected subband in set S shall be ≥ ;

The requirements only apply for subbands of full size and the random scheduling across the subbands is done by selecting a new subband in each available downlink transmission instance for TDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the subband size.

Table 9.3.4.1.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 9 | 10 | 14 | 15 |
|  |  | dB[mW/15kHz] | -89 | -88 | -84 | -83 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Reporting mode |  |  | PUSCH 2-0 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Subband size (k) |  | RBs | 3 (full size) |  |  |  |
| Number of preferred subbands (M) |  |  | 5 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.5 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.3.4.1.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.2 | 1.2 |
| UE Category | ≥1 | ≥1 |

#### 9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)

##### 9.3.4.2.1	FDD

For the parameters specified in Table 9.3.4.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.4.2.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting on subbands reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected subband in set S shall be ≥ ;

The requirements only apply for subbands of full size and the random scheduling across the subbands is done by selecting a new subband in each TTI for FDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the subband size.

Table 9.3.4.2.1-1 Subband test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Reporting periodicity |  | ms | NP = 2 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type for wideband CQI |  |  | 4 |  |  |  |
| PUCCH Report Type for subband CQI |  |  | 1 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Subband size (k) |  | RBs | 6 (full size) |  |  |  |
| Number of bandwidth parts (J) |  |  | 3 |  |  |  |
| K |  |  | 1 |  |  |  |
| cqi-pmi-ConfigIndex |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. Note 5: CQI reports for the short subband (having 2RBs in the last bandwidth part) are to be disregarded and data scheduling according to the most recent subband CQI report for bandwidth part with j=1. Note 6: In the case where wideband CQI is reported, data is to be scheduled according to the most recently used subband CQI report. |  |  |  |  |  |  |

Table 9.3.4.2.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.15 | 1.15 |
| UE Category | ≥1 | ≥1 |

##### 9.3.4.2.2	TDD

For the parameters specified in Table 9.3.4.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.4.2.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting on subbands reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected subband in set S shall be ≥ ;

The requirements only apply for subbands of full size and the random scheduling across the subbands is done by selecting a new subband in each available downlink transmission instance for TDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the subband size.

Table 9.3.4.2.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Reporting periodicity |  | ms | NP = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |  |  |
| PUCCH Report Type for wideband CQI |  |  | 4 |  |  |  |
| PUCCH Report Type for subband CQI |  |  | 1 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Subband size (k) |  | RBs | 6 (full size) |  |  |  |
| Number of bandwidth parts (J) |  |  | 3 |  |  |  |
| K |  |  | 1 |  |  |  |
| cqi-pmi-ConfigIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.3 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 5: CQI reports for the short subband (having 2RBs in the last bandwidth part) are to be disregarded and data scheduling according to the most recent subband CQI report for bandwidth part with j=1. Note 6: In the case where wideband CQI is reported, data is to be scheduled according to the most recently used subband CQI report. |  |  |  |  |  |  |

Table 9.3.4.2.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.15 | 1.15 |
| UE Category | ≥1 | ≥1 |

### 9.3.5	Additional requirements for enhanced receiver Type A

The purpose of the test is to verify that the reporting of the channel quality is based on the receiver of the enhanced Type A. Performance requirements are specified in terms of the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and a requirement on the minimum BLER of the transmitted transport formats indicated by the reported CQI subject to an interference model.

#### 9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)

##### 9.3.5.1.1	FDD

For the parameters specified in Table 9.3.5.1.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.5.1.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.3.5.1.1-1 Fading test for single antenna (FDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -2 | N/A |
|  | dB[mW/15kHz] | -98 | N/A |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (1 x 2) | (1 x 2) |
| DIP (Note 4) | dB | N/A | -0.41 |
| Reference measurement channel |  | Note 2 | R.2 FDD |
| Reporting mode |  | PUCCH 1-0 | N/A |
| Reporting periodicity | ms | Npd = 2 | N/A |
| CQI delay | ms | 8 | N/A |
| Physical channel for CQI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type |  | 4 | N/A |
| cqi-pmi-ConfigurationIndex |  | 1 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.1 FDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and RC.4 FDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. The number of the CRS ports in both cells is the same. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |

Table 9.3.5.1.1-2 Minimum requirement (FDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥1 |

##### 9.3.5.1.2	TDD

For the parameters specified in Table 9.3.5.1.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.3.5.1.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.3.5.1.2-1 Fading test for single antenna (TDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Uplink downlink configuration |  | 2 |  |
| Special subframe configuration |  | 4 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -2 | N/A |
|  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (1 x 2) | (1 x 2) |
| DIP (Note 4) | dB | N/A | -0.41 |
| Reference measurement channel |  | Note 2 | R.2A TDD |
| Reporting mode |  | PUCCH 1-0 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 10 or 11 | N/A |
| Physical channel for CQI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type |  | 4 | N/A |
| cqi-pmi-ConfigurationIndex |  | 3 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| ACK/NACK feedback mode |  | Multiplexing | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.1 TDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and RC.4 TDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. The number of the CRS ports in both cells is the same. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |

Table 9.3.5.1.2-2 Minimum requirement (TDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥1 |

#### 9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)

##### 9.3.5.2.1	FDD

For the parameters specified in Table 9.3.5.2.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.5.2.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.3.5.2.1-1 Fading test for two antennas (FDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 9 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -2 | N/A |
|  | dB[mW/15kHz] | -98 | N/A |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (2 x 2) | (1 x 2) |
| Beamforming Model |  | As specified in Section B.4.3 (Note 10, 11) | N/A |
| DIP (Note 4) | dB | N/A | -0.41 |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna port 0 |
| CSI reference signals |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset |  | 5/1 | N/A |
| CSI-RS reference signal configuration |  | 2 | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | Subframes / bitmap | N/A | 1 / 0010000000000000 |
| CodeBookSubsetRestriction bitmap |  | 001111 | N/A |
| Reference measurement channel |  | Note 2 | R.2 FDD |
| Reporting mode |  | PUCCH 1-1 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 8 | N/A |
| Physical channel for CQI/PMI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type for CQI/PMI |  | 2 | N/A |
| PUCCH channel for RI reporting |  | PUCCH Format 2 | N/A |
| PUCCH Report Type for RI |  | 3 | N/A |
| cqi-pmi-ConfigurationIndex |  | 2 | N/A |
| ri-ConfigIndex |  | 1 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 9: N/A. Note 10: The precoder in clause B.4.3 follows UE recommended PMI. Note 11: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.3.5.2.1-2 Minimum requirement (FDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥2 |

##### 9.3.5.2.2	TDD

For the parameters specified in Table 9.3.5.2.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.3.5.2.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.3.5.2.2-1 Fading test for single antenna (TDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 9 |  |
| Uplink downlink configuration |  | 2 |  |
| Special subframe configuration |  | 4 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -2 | N/A |
|  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (2 x 2) | (1 x 2) |
| Beamforming Model |  | As specified in Section B.4.3 (Note 11, 12) | N/A |
| DIP (Note 4) | dB | N/A | -0.41 |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna port 0 |
| CSI reference signals |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset |  | 5/3 | N/A |
| CSI-RS reference signal configuration |  | 2 | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | Subframes / bitmap | N/A | 3 / 0010000000000000 |
| CodeBookSubsetRestriction bitmap |  | 001111 | N/A |
| Reference measurement channel |  | Note 2 | R.2A TDD |
| Reporting mode |  | PUCCH 1-1 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 10 | N/A |
| Physical channel for CQI/PMI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type for CQI/PMI |  | 2 | N/A |
| Physical channel for RI reporting |  | PUCCH Format 2 | N/A |
| PUCCH Report Type for RI |  | 3 | N/A |
| cqi-pmi-ConfigurationIndex |  | 3 | N/A |
| ri-ConfigIndex |  | 805 (Note 9) | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| ACK/NACK feedback mode |  | Multiplexing | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#2 and #7. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 9: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. Note 10: N/A. Note 11: The precoder in clause B.4.3 follows UE recommended PMI. Note 12: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.3.5.2.2-2 Minimum requirement (TDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥2 |

### 9.3.6	Minimum requirement (With multiple CSI processes)

The purpose of the test is to verify the reporting accuracy of the CQI and the UE processing capability for multiple CSI processes. Each CSI process is associated with a CSI-RS resource and a CSI-IM resource as shown in Table 9.3.6-1. For UE supports one CSI process, CSI process 2 is configured and the corresponding requirements shall be fulfilled. For UE supports three CSI processes, CSI processes 0, 1 and 2 are configured and the corresponding requirements shall be fulfilled. For UE supports four CSI processes, CSI processes 0, 1, 2 and 3 are configured and the corresponding requirements shall be fulfilled.

Table 9.3.6-1: Configuration of CSI processes

|  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- |
| CSI-RS resource | CSI-RS signal 0 | CSI-RS signal 1 | CSI-RS signal 0 | CSI-RS signal 1 |
| CSI-IM resource | CSI-IM resource 0 | CSI-IM resource 0 | CSI-IM resource 1 | CSI-IM resource 2 |

#### 9.3.6.1	FDD

For the parameters specified in Table 9.3.6.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.6.1-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least % of the time but less than % for each sub-band for CSI process 1, 2, or 3;

b)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least % of the time for CSI process 0;

c)	the difference of the median CQIs of the reported wideband CQI for configurated CSI processes shall be greater or equal to the values as in Table 9.3.6.1-3;

d)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

e)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.6.1-1: Fading test for FDD

| Parameter |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 |  | TP2 |  | TP1 |  | TP2 |  |
| Bandwidth |  | MHz | 10 MHz |  |  |  | 10 MHz |  |  |  |
| Transmission mode |  |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Downlink power allocation |  | dB | 0 |  |  |  | 0 |  |  |  |
|  |  | dB | 0 |  |  |  | 0 |  |  |  |
|  |  | dB | -3 |  | 0 |  | -3 |  | 0 |  |
|  |  | dB | -3 |  |  |  | -3 |  |  |  |
| SNR (Note 7) |  | dB | 10 | 11 | 7 | 8 | 14 | 15 | 9 | 10 |
|  |  | dB[mW/15kHz] | -88 | -87 | -91 | -90 | -84 | -85 | -89 | -88 |
|  |  | dB[mW/15kHz] | -98 |  |  |  | -98 |  |  |  |
| Propagation channel |  |  | EPA 5 Low |  | Clause B.2.4.1 with s, a = 1, Hz |  | EPA 5 Low |  | Clause B.2.4.1 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 4x2 |  | 2x2 |  | 4x2 |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  | As specified in Section B.4.3 |  |  |  |
| Timing offset between TPs |  | us | 0 |  |  |  | 0 |  |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  |  | 0 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  | Antenna ports 0,1 |  |  |  |
| CSI-RS signal 0 |  |  | Antenna ports 15,…,18 |  | N/A |  | Antenna ports 15,…,18 |  | N/A |  |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  | N/A |  | 5/1 |  | N/A |  |
| CSI-RS 0 configuration |  |  | 0 |  | N/A |  | 0 |  | N/A |  |
| CSI-RS signal 1 |  |  | N/A |  | Antenna ports 15,16 |  | N/A |  | Antenna ports 15,16 |  |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A |  | 5/1 |  | N/A |  | 5/1 |  |
| CSI-RS 1 configuration |  |  | N/A |  | 5 |  | N/A |  | 5 |  |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | N/A |  | 1 / 1110000000000000 |  | N/A |  | 1 / 1110000000000000 |  |
| Zero-power CSI-RS 1 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 1 / 0010011000000000 |  | N/A |  | 1 / 0010011000000000 |  | N/A |  |
| CSI-IM 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  | 5/1 |  | 5/1 |  | 5/1 |  |
| CSI-IM 0 configuration |  |  | 2 |  | 2 |  | 2 |  | 2 |  |
| CSI-IM 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  | N/A |  | 5/1 |  | N/A |  |
| CSI-IM 1 configuration |  |  | 6 |  | N/A |  | 6 |  | N/A |  |
| CSI-IM 2 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A |  | 5/1 |  | N/A |  | 5/1 |  |
| CSI-IM 2 configuration |  |  | N/A |  | 1 |  | N/A |  | 1 |  |
| CSI process 0 | CSI-RS |  | CSI-RS 0 |  |  |  | CSI-RS 0 |  |  |  |
|  | CSI-IM |  | CSI-IM 0 |  |  |  | CSI-IM 0 |  |  |  |
|  | Reporting mode |  | PUCCH 1-1 |  |  |  | PUCCH 1-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 0x0000 0000 0000 0001 |  |  |  | 0x0000 0000 0000 0001 |  |  |  |
|  | Reporting periodicity | ms | Npd = 5 |  |  |  | Npd = 5 |  |  |  |
|  | CQI delay | ms | 11 |  |  |  | 11 |  |  |  |
|  | Physical channel for CQI/ PMI reporting |  | PUSCH (Note 6) |  |  |  | PUSCH (Note 6) |  |  |  |
|  | PUCCH Report Type for CQI/PMI |  | 2 |  |  |  | 2 |  |  |  |
|  | PUCCH channel for RI reporting |  | PUCCH Format 2 |  |  |  | PUCCH Format 2 |  |  |  |
|  | PUCCH report type for RI |  | 3 |  |  |  | 3 |  |  |  |
|  | cqi-pmi-ConfigurationIndex |  | 4 |  |  |  | 4 |  |  |  |
|  | ri-ConfigIndex |  | 2 |  |  |  | 2 |  |  |  |
| CSI process 1 | CSI-RS |  | CSI-RS 1 |  |  |  | CSI-RS 1 |  |  |  |
|  | CSI-IM |  | CSI-IM 0 |  |  |  | CSI-IM 0 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 000001 |  |  |  | 000001 |  |  |  |
|  | Reporting interval (Note 10) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 11 |  |  |  | 11 |  |  |  |
|  | Sub-band size | RB | 6 (full size) |  |  |  | 6 (full size) |  |  |  |
| CSI process 2 (For UE configured single process) | CSI-RS |  | CSI-RS 0 |  |  |  | CSI-RS 0 |  |  |  |
|  | CSI-IM |  | CSI-IM 1 |  |  |  | CSI-IM 1 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 0x0000 0000 0000 0001 |  |  |  | 0x0000 0000 0000 0001 |  |  |  |
|  | Reporting interval (Note 8) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 8 |  |  |  | 8 |  |  |  |
|  | Sub-band size | RB | 6 (full size) (Note 9) |  |  |  | 6 (full size) (Note 9) |  |  |  |
| CSI process 3 | CSI-RS |  | CSI-RS 1 |  |  |  | CSI-RS 1 |  |  |  |
|  | CSI-IM |  | CSI-IM 2 |  |  |  | CSI-IM 2 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 000001 |  |  |  | 000001 |  |  |  |
|  | Reporting interval (Note 10) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 11 |  |  |  | 11 |  |  |  |
|  | Sub-band size | RB | 6 (full size) |  |  |  | 6 (full size) |  |  |  |
| CSI process for PDSCH scheduling |  |  | CSI process 2 |  |  |  | CSI process 2 |  |  |  |
| Cell ID |  |  | 0 |  | 6 |  | 0 |  | 6 |  |
| Quasi-co-located CSI-RS |  |  | CSI-RS 0 |  | CSI-RS 1 |  | CSI-RS 0 |  | CSI-RS 1 |  |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 |  | Same Cell ID as Cell 2 |  | Same Cell ID as Cell 1 |  | Same Cell ID as Cell 2 |  |
| PMI for subframe 2, 3, 4, 7, 8 and 9 |  |  | 0x0000 0000 0000 0001 |  | 100000 |  | 0x0000 0000 0000 0001 |  | 100000 |  |
| PMI for subframe 1 and 6 |  |  | 0x0000 0000 0001 0000 |  | 100000 |  | 0x0000 0000 0001 0000 |  | 100000 |  |
| Max number of HARQ transmissions |  |  | 1 |  | N/A |  | 1 |  | N/A |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: 3 symbols allocated to PDCCH Note 3: Reference measurement channel RC.12 FDD according to Table A.4-1. PDSCH transmission is scheduled on subframe 2, 3, 4, 7, 8 and 9 from TP1. Note 4: TM10 OCNG OP.8 FDD as specified in A.5.1.8 is transmitted on subframe 1 and 6 from TP1. Note 5: TM10 OCNG OP.8 FDD as specified in A.5.1.8 is transmitted on subframe 1, 2, 3, 4, 6, 7, 8 and 9 from TP2 Note 6: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#2 and #7. Note 7: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 8: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#1 and #6 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#0 and #5. Note 9: For these sub-bands which are not selected for PDSCH transmission, TM10 OCNG should be transmitted. Note 10: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#2 and #7 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#1 and #6. |  |  |  |  |  |  |  |  |  |  |

Table 9.3.6.1-2: Minimum requirement (FDD)

|  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- |
|  [%] | N/A | 2 | 2 | 2 |
|  [%] | N/A | 40 | 40 | 40 |
|  [%] | 10 | N/A | N/A | N/A |
|  | N/A | N/A | 1.02 | N/A |
| UE Category | ≥1 |  |  |  |

Table 9.3.6.1-3: Minimum median CQI difference between configured CSI processes (FDD)

|  | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- |
| CSI process 0 | N/A | 1 | 3 |
| UE Category | ≥1 |  |  |

#### 9.3.6.2	TDD

For the parameters specified in Table 9.3.6.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.6.2-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least % of the time but less than % for each sub-band for CSI process 1, 2, or 3;

b)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least % of the time for CSI process 0;

c)	the difference of the median CQIs of the reported wideband CQI for configurated CSI processes shall be greater or equal to the values as in Table 9.3.6.2-3;

d)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

e)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD, each available downlink transmission instance for TDD. Sub-bands of a size smaller than full size are excluded from the test.

Table 9.3.6.2-1: Fading test for TDD

| Parameter |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 |  | TP2 |  | TP1 |  | TP2 |  |
| Bandwidth |  | MHz | 10 MHz |  |  |  | 10 MHz |  |  |  |
| Transmission mode |  |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Uplink downlink configuration |  |  | 2 |  | 2 |  | 2 |  | 2 |  |
| Special subframe configuration |  |  | 4 |  | 4 |  | 4 |  | 4 |  |
| Downlink power allocation |  | dB | 0 |  |  |  | 0 |  |  |  |
|  |  | dB | 0 |  |  |  | 0 |  |  |  |
|  |  | dB | -3 |  | 0 |  | -3 |  | 0 |  |
|  |  | dB | -3 |  |  |  | -3 |  |  |  |
| SNR (Note 7) |  | dB | 10 | 11 | 7 | 8 | 14 | 15 | 9 | 10 |
|  |  | dB[mW/15kHz] | -88 | -87 | -91 | -90 | -84 | -85 | -89 | -88 |
|  |  | dB[mW/15kHz] | -98 |  |  |  | -98 |  |  |  |
| Propagation channel |  |  | EPA 5 Low |  | Clause B.2.4.1 with s, a = 1, Hz |  | EPA 5 Low |  | Clause B.2.4.1 with s, a = 1, Hz |  |
| Antenna configuration |  |  | 4x2 |  | 2x2 |  | 4x2 |  | 2x2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  | As specified in Section B.4.3 |  |  |  |
| Timing offset between TPs |  | us | 0 |  |  |  | 0 |  |  |  |
| Frequency offset between TPs |  | Hz | 0 |  |  |  | 0 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  | Antenna ports 0,1 |  |  |  |
| CSI-RS signal 0 |  |  | Antenna ports 15,…, 18 |  | N/A |  | Antenna ports 15,…, 18 |  | N/A |  |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 |  | N/A |  | 5/3 |  | N/A |  |
| CSI-RS 0 configuration |  |  | 0 |  | N/A |  | 0 |  | N/A |  |
| CSI-RS signal 1 |  |  | N/A |  | Antenna ports 15, 16 |  | N/A |  | Antenna ports 15, 16 |  |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A |  | 5/3 |  | N/A |  | 5/3 |  |
| CSI-RS 1 configuration |  |  | N/A |  | 5 |  | N/A |  | 5 |  |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | N/A |  | 3 / 1110000000000000 |  | N/A |  | 3 / 1110000000000000 |  |
| Zero-power CSI-RS 1 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0010011000000000 |  | N/A |  | 3 / 0010011000000000 |  | N/A |  |
| CSI-IM 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 |  | 5/3 |  | 5/3 |  | 5/3 |  |
| CSI-IM 0 configuration |  |  | 2 |  | 2 |  | 2 |  | 2 |  |
| CSI-IM 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 |  | N/A |  | 5/3 |  | N/A |  |
| CSI-IM 1 configuration |  |  | 6 |  | N/A |  | 6 |  | N/A |  |
| CSI-IM 2 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A |  | 5/3 |  | N/A |  | 5/3 |  |
| CSI-IM 2 configuration |  |  | N/A |  | 1 |  | N/A |  | 1 |  |
| CSI process 0 | CSI-RS |  | CSI-RS 0 |  |  |  | CSI-RS 0 |  |  |  |
|  | CSI-IM |  | CSI-IM 0 |  |  |  | CSI-IM 0 |  |  |  |
|  | Reporting mode |  | PUCCH 1-1 |  |  |  | PUCCH 1-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 0x0000 0000 0000 0001 |  |  |  | 0x0000 0000 0000 0001 |  |  |  |
|  | Reporting periodicity | ms | Npd = 5 |  |  |  | Npd = 5 |  |  |  |
|  | CQI delay | ms | 12 |  |  |  | 12 |  |  |  |
|  | Physical channel for CQI/ PMI reporting |  | PUSCH (Note 6) |  |  |  | PUSCH (Note 6) |  |  |  |
|  | PUCCH Report Type for CQI/PMI |  | 2 |  |  |  | 2 |  |  |  |
|  | PUCCH channel for RI reporting |  | PUCCH Format 2 |  |  |  | PUCCH Format 2 |  |  |  |
|  | PUCCH report type for RI |  | 3 |  |  |  | 3 |  |  |  |
|  | cqi-pmi-ConfigurationIndex |  | 3 |  |  |  | 3 |  |  |  |
|  | ri-ConfigIndex |  | 805 (Note 10) |  |  |  | 805 (Note 10) |  |  |  |
| CSI process 1 | CSI-RS |  | CSI-RS 1 |  |  |  | CSI-RS 1 |  |  |  |
|  | CSI-IM |  | CSI-IM 0 |  |  |  | CSI-IM 0 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 000001 |  |  |  | 000001 |  |  |  |
|  | Reporting interval (Note 9) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 12 |  |  |  | 12 |  |  |  |
|  | Sub-band size | RB | 6 (full size) |  |  |  | 6 (full size) |  |  |  |
| CSI process 2 | CSI-RS |  | CSI-RS 0 |  |  |  | CSI-RS 0 |  |  |  |
|  | CSI-IM |  | CSI-IM 1 |  |  |  | CSI-IM 1 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 0x0000 0000 0000 0001 |  |  |  | 0x0000 0000 0000 0001 |  |  |  |
|  | Reporting interval (Note 9) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 12 |  |  |  | 12 |  |  |  |
|  | Sub-band size | RB | 6 (full size) (Note 8) |  |  |  | 6 (full size) (Note 8) |  |  |  |
| CSI process 3 | CSI-RS |  | CSI-RS 1 |  |  |  | CSI-RS 1 |  |  |  |
|  | CSI-IM |  | CSI-IM 2 |  |  |  | CSI-IM 2 |  |  |  |
|  | Reporting mode |  | PUSCH 3-1 |  |  |  | PUSCH 3-1 |  |  |  |
|  | CodeBookSubsetRestriction bitmap |  | 000001 |  |  |  | 000001 |  |  |  |
|  | Reporting interval (Note 9) | ms | 5 |  |  |  | 5 |  |  |  |
|  | CQI delay | ms | 12 |  |  |  | 12 |  |  |  |
|  | Sub-band size | RB | 6 (full size) |  |  |  | 6 (full size) |  |  |  |
| CSI process for PDSCH scheduling |  |  | CSI process 2 |  |  |  | CSI process 2 |  |  |  |
| Cell ID |  |  | 0 |  | 6 |  | 0 |  | 6 |  |
| Quasi-co-located CSI-RS |  |  | CSI-RS 0 |  | CSI-RS 1 |  | CSI-RS 0 |  | CSI-RS 1 |  |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 |  | Same Cell ID as Cell 2 |  | Same Cell ID as Cell 1 |  | Same Cell ID as Cell 2 |  |
| PMI for subframe 4 and 9 |  |  | 0x0000 0000 0000 0001 |  | 100000 |  | 0x0000 0000 0000 0001 |  | 100000 |  |
| PMI for subframe 3 and 8 |  |  | 0x0000 0000 0001 0000 |  | 100000 |  | 0x0000 0000 0001 0000 |  | 100000 |  |
| Max number of HARQ transmissions |  |  | 1 |  | N/A |  | 1 |  | N/A |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  | N/A |  | Multiplexing |  | N/A |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: 3 symbols allocated to PDCCH Note 3: Reference measurement channel RC.12 TDD according to Table A.4-1. PDSCH transmission is scheduled on subframe 4 and 9 from TP1. Note 4: TM10 OCNG OP.8 TDD is transmitted as specified in A.5.2.8 on subframe 3 and 8 from TP1. Note 5: TM10 OCNG OP.8 TDD is transmitted as specified in A.5.2.8 on subframe 3, 4, 8 and 9 from TP2 Note 6: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 7: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 8: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3 and #8 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#7 and #2. Note 9: For these sub-bands which are not selected for PDSCH transmission, TM10 OCNG should be transmitted. Note 10: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. |  |  |  |  |  |  |  |  |  |  |

Table 9.3.6.2-2: Minimum requirement (TDD)

|  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- |
|  [%] | N/A | 2 | 2 | 2 |
|  [%] | N/A | 40 | 40 | 40 |
|  [%] | 10 | N/A | N/A | N/A |
|  | N/A | N/A | 1.02 | N/A |
| UE Category | ≥1 |  |  |  |

Table 9.3.6.2-3: Minimum median CQI difference between configured CSI processes (TDD)

|  | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- |
| CSI process 0 | N/A | 1 | 3 |
| UE Category | ≥1 |  |  |

### 9.3.7	Minimum requirement PUSCH 3-2

#### 9.3.7.1	FDD

For the parameters specified in Table 9.3.7.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.7.1-2 and by the following.

a)	the ratio of the throughput obtained when transmitting based on UE PUSCH 3-2 reported wideband CQI and subband PMI and that obtained when transmitting based on PUSCH 3-1 reported wideband CQI and wideband PMI shall be ≥

b)	The ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS based on UE PUSCH3-2 reported subband CQI and subband PMI and that obtained when transmitting on a randomly selected sub-band in set S based on PUSCH 1-2 reported wideband CQI and subband PMI shall be ≥

The transport block sizes TBS for wideband CQI and subband CQI are selected according to RC.17 FDD for test 1 and according to RC.18 FDD for test 2.

Table 9.3.7.1-1 Sub-band test for FDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10MHz |  |  |  |
| PDSCH resource allocation |  | RB | 50PRB |  | a subband, 6PRB |  |
| Transmission mode |  |  | TM6 |  | TM9 |  |
| Downlink power allocation |  | dB | -6 |  | 0 |  |
|  |  | dB | -6 |  | 0 |  |
|  |  | dB | - |  | -3 |  |
|  |  | dB | 3 |  | -3 |  |
| SNR (Note 3) |  | dB | 0 | 1 | 5 | 6 |
|  |  | dB[mW/15kHz] | -98 | -97 | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
| Propagation channel |  |  | EVA5 |  | EVA5 |  |
| Antenna configuration |  |  | 4x2 ULA low |  | 4x2 XP high (Note 4) |  |
| Beamforming Model |  |  | - |  | B.4.3 |  |
| CRS reference signals |  |  | Antenna ports 0, 1, 2, 3 |  | Antenna ports 0, 1 |  |
| Time offset between TX antenna (Note 5) |  | ns | 65 |  | - |  |
| CSI reference signals |  |  |  |  | Antenna ports 15, 16, 17, 18 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | - |  | 5/ 1 |  |
| CSI-RS reference signal configuration |  |  | - |  | 4 |  |
| alternativeCodebookEnabledFor4TX |  |  | No |  | Yes |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF |  | 0x0000 0000 0000 FFFF 0000 FFFF |  |
| Reporting interval (Note 6) |  | ms | 5 |  | 5 |  |
| CQI delay |  | ms | 8 |  | 8 |  |
| Reporting mode |  |  | PUSCH 3-2, PUSCH 3-1 |  | PUSCH 3-2, PUSCH 1-2 |  |
| Sub-band size |  | RB | 6 (full size) |  | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  | 1 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.17 FDD / RC.18 FDD for Test 1 / 2 according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. Note 5: The values of time offset are [0ns 65ns 0ns 65ns] for antenna port [0, 1, 2, 3] respectively. Note 6: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#1 and #6 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#0 and #5. |  |  |  |  |  |  |

Table 9.3.7.1-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| α | 1.05 | - |
| β | - | 1.15 |
| UE Category | ≥2 | ≥2 |

#### 9.3.7.2	TDD

For the parameters specified in Table 9.3.7.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.3.7.2-2 and by the following.

a)	the ratio of the throughput obtained when transmitting based on UE PUSCH 3-2 reported wideband CQI and subband PMI and that obtained when transmitting based on PUSCH 3-1 reported wideband CQI and wideband PMI shall be ≥

b)	The ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS based on UE PUSCH3-2 reported subband CQI and subband PMI and that obtained when transmitting on a randomly selected sub-band in set S based on PUSCH 1-2 reported wideband CQI and subband PMI shall be ≥

The transport block sizes TBS for wideband CQI and subband CQI are selected according to RC.17 TDD for test 1 and RC.18 TDD for test 2.

Table 9.3.7.2-1 Sub-band test for TDD

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10MHz |  |  |  |
| PDSCH resource allocation |  | RB | 50PRB |  | a subband, 6PRB |  |
| Transmission mode |  |  | TM6 |  | TM9 |  |
| Uplink downlink configuration |  |  | 1 |  | 1 |  |
| Special subframe configuration |  |  | 4 |  | 4 |  |
| Downlink power allocation |  | dB | -6 |  | 0 |  |
|  |  | dB | -6 |  | 0 |  |
|  |  | dB | - |  | -3 |  |
|  |  | dB | 3 |  | -3 |  |
| SNR (Note 3) |  | dB | 0 | 1 | 5 | 6 |
|  |  | dB[mW/15kHz] | -98 | -97 | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
| Propagation channel |  |  | EVA5 |  | EVA5 |  |
| Antenna configuration |  |  | 4x2 ULA low |  | 4x2 XP high (Note 4) |  |
| Beamforming Model |  |  | - |  | B.4.3 |  |
| CRS reference signals |  |  | Antenna ports 0, 1, 2, 3 |  | Antenna ports 0, 1 |  |
| Time offset between TX antenna (Note 5) |  | ns | 65 |  | - |  |
| CSI reference signals |  |  |  |  | Antenna ports 15, 16, 17, 18 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | - |  | 5/ 4 |  |
| CSI-RS reference signal configuration |  |  | - |  | 4 |  |
| alternativeCodebookEnabledFor4TX |  |  | No |  | Yes |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF |  | 0x0000 0000 0000 FFFF 0000 FFFF |  |
| Reporting interval (Note 6) |  | ms | 5 |  | 5 |  |
| CQI delay |  | ms | 8 |  | 8 |  |
| Reporting mode |  |  | PUSCH 3-2, PUSCH 3-1 |  | PUSCH 3-2, PUSCH 1-2 |  |
| Sub-band size |  | RB | 6 (full size) |  | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  | 1 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.17 TDD / RC.18 TDD for Test 1 / 2 according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. Note 5: The values of time offset are [0ns 65ns 0ns 65ns] for antenna port [0, 1, 2, 3] respectively. Note 6: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#3 and #8. |  |  |  |  |  |  |

Table 9.3.7.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| α | 1.05 | - |
| β | - | 1.15 |
| UE Category | ≥2 | ≥2 |

### 9.3.8	Additional requirements for enhanced receiver Type B

The purpose of the test is to verify that the reporting of the channel quality based on the receiver of the enhanced Type B meets a minimum performance. Performance requirements are specified in terms of the relative throughput obtained when the transport format is that indicated by the reported CQI with NeighCellsInfo-r12 configured compared to the case without NeighCellsInfo-r12 configured. Cell 1 is the serving cell, and Cell 2 and Cell 3 are the interference cells.

#### 9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)

##### 9.3.8.1.1	FDD

For the parameters specified in Table 9.3.8.1.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.8.1.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with NeighCellsInfo-r12 configured and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources without NeighCellsInfo-r12 configured shall be ≥ ;

Table 9.3.8.1.1-1 Fading test for FDD

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |
| Transmission mode |  |  |  | 4 |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |
|  |  |  | dB | -3 |  |  |
|  |  |  | dB | 0 |  |  |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 1 | 6 |
| SNR |  |  | dB | 8.34 | N/A | N/A |
|  |  |  |  | N/A | 3.28 | 0.74 |
|  |  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  |  | dB [mW/15kHz] | -98 |  |  |
| Propagation channel |  |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Interference model |  |  |  | N/A | As specified in clause B.6.3 | As specified in clause B.6.3 |
| Reporting periodicity |  |  | ms | Npd = 5 | N/A | N/A |
| Physical channel for CQI/PMI reporting |  |  |  | PUCCH Format 2 | N/A | N/A |
| PUCCH Report Type for CQI/PMI |  |  |  | 2 | N/A | N/A |
| PUCCH Report Type for RI |  |  |  | 3 | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  |  | 6 | N/A | N/A |
| ri-ConfigurationIndex |  |  |  | 1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 | N/A | N/A |
| Max number of HARQ transmissions |  |  |  | 1 | N/A | N/A |
| NeighCellsInfo-r12 (Note 4) | p-aList-r12 |  |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: All cells are time-synchronous. Note 4: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 9.3.8.1.1-2 Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

##### 9.3.8.1.2	TDD

For the parameters specified in Table 9.3.8.1.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.3.8.1.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with NeighCellsInfo-r12 configured and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources without NeighCellsInfo-r12 configured shall be ≥ ;

Table 9.3.8.1.2-1 Fading test for TDD

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| Transmission mode |  |  | 4 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | -3 |  |  |
|  |  | dB | -3 |  |  |
|  |  | dB | 0 |  |  |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| SNR |  | dB | 8.34 | N/A | N/A |
|  |  |  | N/A | 3.28 | 0.74 |
|  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  | dB [mW/15kHz] | -98 |  |  |
| Propagation channel |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Interference model |  |  | N/A | As specified in clause B.6.3 | As specified in clause B.6.3 |
| Reporting periodicity |  | ms | Npd = 5 | N/A | N/A |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) | N/A | N/A |
| PUCCH Report Type |  |  | 2 | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 3 | N/A | N/A |
| ri-ConfigIndex |  |  | 805 (Note 5) | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  | 000001 | N/A | N/A |
| Max number of HARQ transmissions |  |  | 1 | N/A | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| NeighCellsInfo-r12 (Note 6) | p-aList-r12 |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: All cells are time-synchronous. Note 5: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. Note 6: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |

Table 9.3.8.1.2-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

#### 9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)

##### 9.3.8.2.1	FDD

For the parameters specified in Table 9.3.8.2.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.8.2.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with NeighCellsInfo-r12 configured and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources without NeighCellsInfo-r12 configured shall be ≥ ;

Table 9.3.8.2.1-1 Fading test for FDD

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |
| Transmission mode |  |  |  | 9 |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
|  | Pc |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 1 | 6 |
| SNR |  |  | dB | 8.34 | N/A | N/A |
|  |  |  |  | N/A | 3.28 | 0.74 |
|  |  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  |  | dB [mW/15kHz] | -98 |  |  |
| Propagation channel |  |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset |  |  |  | 5/1 | N/A | N/A |
| CSI-RS reference signal configuration |  |  |  | 2 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | N/A | 1 / 0001000000000000 | 1 / 0001000000000000 |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Reporting periodicity |  |  | ms | Npd = 5 | N/A | N/A |
| Physical channel for CQI/PMI reporting |  |  |  | PUSCH (Note 3) | N/A | N/A |
| PUCCH Report Type for CQI/PMI |  |  |  | 2 | N/A | N/A |
| PUCCH channel for RI reporting |  |  |  | PUCCH Format 2 | N/A | N/A |
| PUCCH Report Type for RI |  |  |  | 3 | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  |  | 2 | N/A | N/A |
| ri-ConfigIndex |  |  |  | 1 | N/A | N/A |
| Max number of HARQ transmissions |  |  |  | 1 | N/A | N/A |
| NeighCellsInfo-r12 (Note 5) |  | p-aList-r12 |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  |  | transmissionModeList-r12 |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. Note 4: All cells are time-synchronous. Note 5: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 9.3.8.2.1-2 Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

##### 9.3.8.2.2	TDD

For the parameters specified in Table 9.3.8.2.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.3.8.2.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with NeighCellsInfo-r12 configured and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources without NeighCellsInfo-r12 configured shall be ≥ ;

Table 9.3.8.2.2-1 Fading test for TDD

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |
| Transmission mode |  |  |  | 9 |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
|  | Pc |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
| Uplink downlink configuration |  |  |  | 2 |  |  |
| Special subframe configuration |  |  |  | 4 |  |  |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell ID |  |  |  | 0 | 1 | 6 |
| SNR |  |  | dB | 8.34 | N/A | N/A |
|  |  |  |  | N/A | 3.28 | 0.74 |
|  |  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  |  | dB [mW/15kHz] | -98 |  |  |
| Propagation channel |  |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset |  |  |  | 5/3 | N/A | N/A |
| CSI-RS reference signal configuration |  |  |  | 2 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | N/A | 3 / 0001000000000000 | 3 / 0001000000000000 |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| Reporting periodicity |  |  | ms | Npd = 5 | N/A | N/A |
| Physical channel for CQI/PMI reporting |  |  |  | PUSCH (Note 3) | N/A | N/A |
| PUCCH Report Type for CQI/PMI |  |  |  | 2 | N/A | N/A |
| Physical channel for RI reporting |  |  |  | PUCCH Format 2 | N/A | N/A |
| PUCCH Report Type for RI |  |  |  | 3 | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  |  | 3 | N/A | N/A |
| ri-ConfigIndex |  |  |  | 805 (Note 5) | N/A | N/A |
| Max number of HARQ transmissions |  |  |  | 1 | N/A | N/A |
| ACK/NACK feedback mode |  |  |  | Multiplexing | N/A | N/A |
| NeighCellsInfo-r12 (Note 6) |  | p-aList-r12 |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  |  | transmissionModeList-r12 |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#2 and #7. Note 4: All cells are time-synchronous. Note 5: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. Note 6: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |  |

Table 9.3.8.2.2-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

#### 9.3.8.3	Minimum requirement with CSI process

##### 9.3.8.3.1	FDD

For the parameters specified in Table 9.3.8.3.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.8.3.1-2 and by the following

a)	the ratio of the throughput obtained for the Type B receiver with NAICS assistance information when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with specified   and that obtained for the Type B receiver without NAICS assistance information when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with the same specified   shall be ≥ ;

Table 9.3.8.3.1-1 Fading test for single antenna (FDD)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| Transmission mode |  |  | 10 | 9 | 9 |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
|  | Pc | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| SNR |  | dB | 8.34 | N/A | N/A |
|  |  | dB | N/A | 3.28 | 0.74 |
|  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  | dB[mW/15kHz] | -98 |  |  |
| Propagation channel |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0, 1 | Antenna port 0, 1 |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset |  |  | 5/1 | N/A | N/A |
| CSI-RS reference signal configuration |  |  | 2 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | N/A | 1 / 0001000000000000 | 1 / 0001000000000000 |
| Interference model |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI process | CSI-RS |  | CSI-RS | N/A | N/A |
|  | CSI-IM |  | CSI-IM | N/A | N/A |
|  | Reporting mode |  | PUCCH 1-1 | N/A | N/A |
|  | CodeBookSubsetRestriction bitmap |  | 000001 | N/A | N/A |
|  | Reporting periodicity | ms | Npd = 5 | N/A | N/A |
|  | CQI delay | ms | 8 | N/A | N/A |
|  | Physical channel for CQI/ PMI reporting |  | PUSCH (Note 3) | N/A | N/A |
|  | PUCCH Report Type for CQI/PMI |  | 2 | N/A | N/A |
|  | PUCCH channel for RI reporting |  | PUCCH Format 2 | N/A | N/A |
|  | PUCCH report type for RI |  | 3 | N/A | N/A |
|  | cqi-pmi-ConfigurationIndex |  | 2 | N/A | N/A |
|  | ri-ConfigIndex |  | 1 | N/A | N/A |
| CSI-IM periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 | N/A | N/A |
| CSI-IM configuration |  |  | 6 | N/A | N/A |
| CSI process for PDSCH scheduling |  |  | CSI process | N/A | N/A |
| Quasi-co-located CSI-RS |  |  | CSI-RS | N/A | N/A |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 | N/A | N/A |
| Reference measurement channel |  |  | Note 2 | N/A | N/A |
| Max number of HARQ transmissions |  |  | 1 | N/A | N/A |
| NeighCellsInfo-r12 (Note 5) | p-aList-r12 |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. Note 4: All cells are time-synchronous. Note 5: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |

Table 9.3.8.3.1-2 Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

##### 9.3.8.3.2	TDD

For the parameters specified in Table 9.3.8.3.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.3.8.3.2-2 and by the following

a)	the ratio of the throughput obtained obtained for the Type B receiver with NAICS assistance information when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with specified   and that obtained for the Type B receiver without NAICS assistance information when transmitting the transport format indicated by each reported wideband CQI index subject to interference sources with the same specified  shall be ≥ ;

Table 9.3.8.3.2-1 Fading test for single antenna (TDD)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| Transmission mode |  |  | 10 | 9 | 9 |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
|  | Pc | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Cyclic Prefix |  |  | Normal | Normal | Normal |
| Cell ID |  |  | 0 | 1 | 6 |
| SNR |  | dB | 8.34 | N/A | N/A |
|  |  | dB | N/A | 3.28 | 0.74 |
|  |  | dB [mW/15kHz] | -89.66 | -94.72 | -97.26 |
|  |  | dB[mW/15kHz] | -98 |  |  |
| Propagation channel |  |  | EPA5 | EPA5 | EPA5 |
| Correlation and antenna configuration |  |  | Low 2 x 2 | Low 2 x 2 | Low 2 x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0,1 | Antenna port 0,1 |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset |  |  | 5/3 | N/A | N/A |
| CSI-RS reference signal configuration |  |  | 2 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | N/A | 3 / 0001000000000000 | 3 / 0001000000000000 |
| Interference model |  |  | N/A | As specified in clause B.6.4 | As specified in clause B.6.4 |
| CSI process | CSI-RS |  | CSI-RS | N/A | N/A |
|  | CSI-IM |  | CSI-IM | N/A | N/A |
|  | Reporting mode |  | PUCCH 1-1 | N/A | N/A |
|  | CodeBookSubsetRestriction bitmap |  | 000001 | N/A | N/A |
|  | Reporting periodicity | ms | Npd = 5 | N/A | N/A |
|  | CQI delay | ms | 8 | N/A | N/A |
|  | Physical channel for CQI/ PMI reporting |  | PUSCH (Note 3) | N/A | N/A |
|  | PUCCH Report Type for CQI/PMI |  | 2 | N/A | N/A |
|  | PUCCH channel for RI reporting |  | PUCCH Format 2 | N/A | N/A |
|  | PUCCH report type for RI |  | 3 | N/A | N/A |
|  | cqi-pmi-ConfigurationIndex |  | 3 | N/A | N/A |
|  | ri-ConfigIndex |  | 805 (Note 5) | N/A | N/A |
| CSI-IM periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 | N/A | N/A |
| CSI-IM configuration |  |  | 6 | N/A | N/A |
| CSI process for PDSCH scheduling |  |  | CSI process | N/A | N/A |
| Quasi-co-located CSI-RS |  |  | CSI-RS | N/A | N/A |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 | N/A | N/A |
| Reference measurement channel |  |  | Note 2 | N/A | N/A |
| Max number of HARQ transmissions |  |  | 1 | N/A | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| NeighCellsInfo-r12 (Note 6) | p-aList-r12 |  | N/A | {dB-6, dB-3, dB0} | {dB-6, dB-3, dB0} |
|  | transmissionModeList-r12 |  | N/A | {2,3,4,8,9} | {2,3,4,8,9} |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. Note 4: All cells are time-synchronous. Note 5: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. Note 6: NeighCellsInfo-r12 is described in subclause 6.3.2 of [7]. |  |  |  |  |  |

Table 9.3.8.3.2-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 0.925 |
| UE Category | ≥2 |

## 9.4	Reporting of Precoding Matrix Indicator (PMI)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 6, transmission mode 9 with 4 TX and transmission mode 9 with 8 TX alternativeCodebookEnabledCLASSB_K1=TRUE configured are specified in terms of the ratio

.

In the definition of γ, for PUSCH 3-1 single PMI and PUSCH 1-2 multiple PMI requirements,  is 60% of the maximum throughput obtained at using random precoding, and  the throughput measured at with precoders configured according to the UE reports;

For the PUCCH 2-1 single PMI requirement, is 60% of the maximum throughput obtained at  using random precoding on a randomly selected full-size subband in set S subbands, and the throughput measured at with both the precoder and the preferred full-size subband applied according to the UE reports;

For PUSCH 2-2 multiple PMI requirements, is 60% of the maximum throughput obtained at  using random precoding on a randomly selected full-size subband in set S subbands, and the throughput measured at with both the subband precoder and a randomly selected full-size subband (within the preferred subbands) applied according to the UE reports.

For PUCCH 1-1 single PMI  requirement under transmission mode 9 with  4Tx and 8 TX when alternativeCodebookEnabledCLASSB_K1=TRUE configured,  is 70% of the maximum throughput obtained at  using the precoders configured according to the UE reports, and  is the throughput measured at  with random precoding.

The requirements for transmission mode 9 with 8 TX and transmission mode 9 with 4TX enhanced codebook are specified in terms of the ratio

In the definition of γ, for PUSCH 3-1 single PMI, PUCCH 1-1 single PMI and PUSCH 1-2 multiple PMI requirements, is 70% of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

The requirements for transmission mode 9 with 12 TX, 16 TX, 24TX and 32 TX Class A codebook are specified in terms of the ratio

In the definition of γ, for PUSCH 3-1 single PMI and PUSCH 1-2 multiple PMI requirements,  is 90% of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

The requirements for transmission mode 9 with 16 TX Class A advancedCodebookEnabled=TRUE configured are specified in terms of the ratio

In the definition of γ,  is [70%] of the maximum throughput obtained at  using the precoders configured according to both the PMI and relative power indicator (RPI) reported by the UE, and  is the throughput measured at  using the precoders configured according to the UE reports PMI and fixed RPI equal to 0.

### 9.4.1	Single PMI

#### 9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)

##### 9.4.1.1.1	FDD

For the parameters specified in Table 9.4.1.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.1.1-2.

Table 9.4.1.1.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | Low 2 x 2 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 3-1 |
| Reporting interval |  | ms | 1 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R. 10 FDD |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.4.1.1.1-2 Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.1 |
| UE Category | ≥1 |

##### 9.4.1.1.2	TDD

For the parameters specified in Table 9.4.1.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.1.1.2-2.

Table 9.4.1.1.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | Low 2 x 2 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 3-1 |
| Reporting interval |  | ms | 1 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Measurement channel |  |  | R.10 TDD |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each available downlink transmission instance. Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.4.1.1.2-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.1 |
| UE Category | ≥1 |

#### 9.4.1.2		Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)

##### 9.4.1.2.1	FDD

For the parameters specified in Table 9.4.1.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.2.1-2.

Table 9.4.1.2.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Propagation channel |  |  | EVA5 |
| Correlation and antenna configuration |  |  | Low 4 x 2 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 3 |
|  |  | dB[mW/15kHz] | -98 |
| PMI delay |  | ms | 8 or 9 |
| Reporting mode |  |  | PUCCH 2-1 (Note 6) |
| Reporting periodicity |  | ms | Npd = 2 |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for wideband CQI/PMI |  |  | 2 |
| PUCCH Report Type for subband CQI |  |  | 1 |
| Measurement channel |  |  | R.14-1 FDD |
| OCNG Pattern |  |  | OP.1/2 FDD |
| Precoding granularity |  | PRB | 6 (full size) |
| Number of bandwidth parts (J) |  |  | 3 |
| K |  |  | 1 |
| cqi-pmi-ConfigIndex |  |  | 1 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated every two TTI (2 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between HARQ-ACK and wideband CQI/PMI or subband CQI, it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. Note 4: Reports for the short subband (having 2RBs in the last bandwidth part) are to be disregarded and instead data is to be transmitted on the most recently used subband for bandwidth part with j=1. Note 5: In the case where wideband PMI is reported, data is to be transmitted on the most recently used subband. Note 6: The bit field for PMI confirmation in DCI format 1B shall be mapped to “0” and TPMI information shall indicate the codebook index used in Table 6.3.4.2.3-2 of TS36.211 [4] according to the latest PMI report on PUCCH. |  |  |  |

Table 9.4.1.2.1-2: Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

##### 9.4.1.2.2	TDD

For the parameters specified in Table 9.4.1.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.2.2-2.

Table 9.4.1.2.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Correlation and antenna configuration |  |  | Low 4 x 2 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 3 |
|  |  | dB[mW/15kHz] | -98 |
| PMI delay |  | ms | 10 |
| Reporting mode |  |  | PUCCH 2-1 (Note 6) |
| Reporting periodicity |  | ms | NP = 5 |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for wideband CQI/PMI |  |  | 2 |
| PUCCH Report Type for subband CQI |  |  | 1 |
| Measurement channel |  |  | R.14-1 TDD |
| OCNG Pattern |  |  | OP.1/2 TDD |
| Precoding granularity |  | PRB | 6 (full size) |
| Number of bandwidth parts (J) |  |  | 3 |
| K |  |  | 1 |
| cqi-pmi-ConfigIndex |  |  | 4 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK fedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each available downlink transmission instance. Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between HARQ-ACK and wideband CQI/PMI or subband CQI it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 4: Reports for the short subband (having 2RBs in the last bandwidth part) are to be disregarded and instead data is to be transmitted on the most recently used subband for bandwidth part with j=1. Note 5: In the case where wideband PMI is reported, data is to be transmitted on the most recently used subband. Note 6: The bit field for PMI confirmation in DCI format 1B shall be mapped to “0” and TPMI information shall indicate the codebook index used in Table 6.3.4.2.3-2 of TS36.211 [4] according to the latest PMI report on PUCCH. |  |  |  |

Table 9.4.1.2.2-2: Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

#### 9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

##### 9.4.1.3.1	FDD

For the parameters specified in Table 9.4.1.3.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.1-2.

Table 9.4.1.3.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 | 10 |
| Transmission mode |  |  | 9 | 9 | 9 |
| Propagation channel |  |  | EPA5 | EPA5 | EPA5 |
| Precoding granularity |  | PRB | 50 | 50 | 50 |
| Correlation and antenna configuration |  |  | Low ULA 4 x 2 | Low ULA 4 x 2 | Low ULA 4 x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 | Annex B.4.3 |
| FrequencyDensityBeamformed |  |  | N/A | 1 | 1 |
| NZP-TransmissionCombBeamformed |  |  | N/A | N/A | N/A |
| csi-RS-NZP-mode |  |  | N/A | aperiodic | multiShot |
| NumberActivatedAperiodicCSI-RS-Resources |  |  | N/A | 1 | NA |
| CSI-RS-ConfigNZPAperiodic |  |  | N/A | 6,7 (Note 4) | NA |
| NumberActivatedCSI-RS-Resources |  |  | N/A | NA | 1 |
| eMIMO-Type |  |  | N/A | ClassB | ClassB |
| alternativeCodebookEnabledCLASSB_K1 |  |  | N/A | FALSE | FALSE |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 | N/A | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 6 | N/A | 6,7 (Note 5,6) |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF | 0x0000 0000 0000 FFFF | 0x0000 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 | 0 |  |
|  |  | dB | 0 | 0 |  |
|  | Pc | dB | -3 | -3 |  |
|  |  | dB | -3 | -3 |  |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 | PUSCH 3-1 |
| Reporting interval |  | ms | 5 | 5 | 5 |
| PMI delay (Note 2) |  | ms | 8 | 8 | 8 |
| Measurement channel |  |  | R.44 FDD | R.44 FDD | R.44 FDD |
| OCNG Pattern |  |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Only one of these two aperiodic NZP CSI-RS resources will be randomly seleted and activated during test .The selected aperiodic NZP CSI-RS is transmitted in every subframe #1 and #6. Note 5: Through MAC CE, one of these two periodic NZP CSI-RS resources will be activated and the other one will be deactivated in an alternative way. The activation and deactivation command will be transmitted in sub-frame #0 per [10000] ms. Note 6: When activation and/or deactivation command through MAC CE is transmitted at subframe n, the scheduled PDSCH transmission will be skiped for the subsequent 25 subframes. |  |  |  |  |  |

Table 9.4.1.3.1-2: Minimum requirement (FDD)

| Parameter | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | 1.2 | [1.2] | [1.2] |
| UE Category | ≥1 | ≥2 | ≥2 |
| Note 1: For UE that indicates support of csi-RS-NZP-mode ‘aperiodic’ and/or ‘multiShot’, test 2 and/or test 3 will be run and test 1 will be skipped. Otherwise, test 1 will be run and test 2 and test 3 will be skipped. |  |  |  |

##### 9.4.1.3.2	TDD

For the parameters specified in Table 9.4.1.3.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.2-2.

Table 9.4.1.3.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 | 10 |
| Transmission mode |  |  | 9 | 9 | 9 |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Propagation channel |  |  | EVA5 | EVA5 | EVA5 |
| Precoding granularity |  | PRB | 50 | 50 | 50 |
| Antenna configuration |  |  | 8 x 2 | 8 x 2 | 8 x 2 |
| Correlation modeling |  |  | High, Cross polarized | High, Cross polarized | High, Cross polarized |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 | Antenna ports 15,…,22 | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 | Annex B.4.3 |
| FrequencyDensityBeamformed |  |  | N/A | 1 | 1 |
| NZP-TransmissionCombBeamformed |  |  | N/A | N/A | N/A |
| csi-RS-NZP-mode |  |  | N/A | aperiodic | multiShot |
| NumberActivatedAperiodicCSI-RS-Resources |  |  | N/A | 1 | N/A |
| CSI-RS-ConfigNZPAperiodic |  |  | N/A | 0,1 (Note 5) | N/A |
| NumberActivatedCSI-RS-Resources |  |  | N/A | N/A | 1 |
| eMIMO-Type |  |  | N/A | ClassB | ClassB |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 | N/A | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 0 | N/A | 0,1 (Note 6,7) |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF | 0x0000 0000 001F FFE0 0000 0000 FFFF | 0x0000 0000 001F FFE0 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 | 0 |  |
|  |  | dB | 0 | 0 |  |
|  | Pc | dB | -6 | -6 |  |
|  |  | dB | -3 | -3 |  |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 | PUSCH 3-1 |
| Reporting interval |  | ms | 5 | 5 | 5 |
| PMI delay (Note 2) |  | ms | 10 | 10 | 10 |
| Measurement channel |  |  | R.45-1 TDD for UE Category 1, R.45 TDD for UE Category ≥2 | R.45 TDD | R.45 TDD |
| OCNG Pattern |  |  | OP.7 TDD for UE Category 1, and OP.1 TDD for UE Category ≥2 | OP.1 TDD | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3A.4 Note 5: Only one of these two aperiodic NZP CSI-RS resources will be randomly seleted and activated during test .The selected aperiodic NZP CSI-RS is transmitted in every subframe #4 and #9. Note 6: Through MAC CE, one of these two periodic NZP CSI-RS resources will be activated and the other one will be deactivated in an alternative way. The activation and deactivation command will be transmitted in sub-frame #0 per [10000] ms. Note 7: When activation and/or deactivation command through MAC CE is transmitted at subframe n, the scheduled PDSCH transmission will be skiped for the subsequent 25 subframes. |  |  |  |  |  |

Table 9.4.1.3.2-2: Minimum requirement (TDD)

| Parameter | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | 3 | [3] | [3] |
| UE Category | ≥1 | ≥2 | ≥2 |
| Note 1: For UE that indicates support of csi-RS-NZP-mode ‘aperiodic’ and/or ‘multiShot’, test 2 and/or test 3 will be run and test 1 will be skipped. Otherwise, test 1 will be run and test 2 and test 3 will be skipped. |  |  |  |

##### 9.4.1.3.3	FDD (with Class A 12Tx codebook)

For the parameters specified in Table 9.4.1.3.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.3-2.

Table 9.4.1.3.3-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | High 2D XP 12(2,3,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,26 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 3 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 4 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 0000 1111 Codebook-Config 2,3,4: 0x 00 000000 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -8 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 3-1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.77 FDD |
| Rank Number of PDSCH |  |  | 1 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.1.3.3-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |
| UE Category | ≥2 |

##### 9.4.1.3.4	TDD (with Class A 12Tx codebook)

For the parameters specified in Table 9.4.1.3.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.4-2.

Table 9.4.1.3.4-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | High 2D XP 12(2,3,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,26 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 3 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 4 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 0000 1111 Codebook-Config 2,3,4: 0x 00 000000 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -8 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 3-1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.77 TDD |
| Rank Number of PDSCH |  |  | 1 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.1.3.4-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |
| UE Category | ≥2 |

##### 9.4.1.3.5	FDD (with Class A 24Tx codebook)

For the parameters specified in Table 9.4.1.3.5-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.5-2.

Table 9.4.1.3.5-1: PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1a | Test 1b |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 |
| Transmission mode |  |  | 9 | 9 |
| Propagation channel |  |  | EPA5 | EPA5 |
| Precoding granularity |  | PRB | 50 | 50 |
| Correlation and antenna configuration |  |  | High 2D XP 24(3,4,2) x 2 | High 2D XP 24(3,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,38 | Antenna ports 15,…,38 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| CDM Type |  |  | CDM4 | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 | 5/ 1 |
| FrequencyDensityNonPrecoded |  |  | 1 | 1/3 |
| NZP-TransmissionCombListNonprecoded |  |  | N/A | {0,1,2} |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2} | {0,1,2} |
| eMIMO-Type |  |  | Class A | Class A |
| codebookConfig-N1 |  |  | 3 | 3 |
| codebookConfig-N2 |  |  | 4 | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 4 | 4 |
| Codebook-Config |  |  | Note 5 | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
|  | Pc | dB | -8 | -8 |
|  |  | dB | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| Reporting interval |  | ms | 5 | 5 |
| PMI delay (Note 2) |  | ms | 8 | 8 |
| Measurement channel |  |  | R.88 FDD | R.88A FDD |
| Rank Number of PDSCH |  |  | 2 | 2 |
| OCNG Pattern |  |  | OP.1 FDD | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |  |

Table 9.4.1.3.5-2: Minimum requirement (FDD)

| Parameter | Test 1a | Test 1b |
| --- | --- | --- |
|  | [3.5] | [3.5] |
| UE Category | ≥2 | ≥2 |
| Note1: For UE that indicates support of density reduction for Class A, test 1b will be run and test 1a will be skipped. Otherwise, test 1a will be run and test 1b will be skipped. |  |  |

##### 9.4.1.3.6	TDD (with Class A 24Tx codebook)

For the parameters specified in Table 9.4.1.3.6-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.3.6-2.

Table 9.4.1.3.6-1: PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1a | Test 1b |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 |
| Transmission mode |  |  | 9 | 9 |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Propagation channel |  |  | EPA5 | EPA5 |
| Precoding granularity |  | PRB | 50 | 50 |
| Correlation and antenna configuration |  |  | High 2D XP 24(3,4,2) x 2 | High 2D XP 24(3,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,38 | Antenna ports 15,…,38 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| CDM Type |  |  | CDM4 | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 | 5/ 4 |
| FrequencyDensityNonPrecoded |  |  | 1 | 1/3 |
| NZP-TransmissionCombListNonprecoded |  |  | NA | {0,1,2} |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2} | {0,1,2} |
| eMIMO-Type |  |  | Class A | Class A |
| codebookConfig-N1 |  |  | 3 | 3 |
| codebookConfig-N2 |  |  | 4 | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 4 | 4 |
| Codebook-Config |  |  | Note 5 | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
|  | Pc | dB | -8 | -8 |
|  |  | dB | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| Reporting interval |  | ms | 5 | 5 |
| PMI delay (Note 2) |  | ms | 10 | 10 |
| Measurement channel |  |  | R.88 TDD | R.88A TDD |
| Rank Number of PDSCH |  |  | 2 | 2 |
| OCNG Pattern |  |  | OP.1 TDD | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |  |

Table 9.4.1.3.6-2: Minimum requirement (TDD)

| Parameter | Test 1a | Test 1b |
| --- | --- | --- |
|  | [3.0] | [3.0] |
| UE Category | ≥2 | ≥2 |
| Note1: For UE that indicates support of density reduction for Class A, test 1b will be run and test 1a will be skipped. Otherwise, test 1a will be run and test 1b will be skipped. |  |  |

#### 9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)

##### 9.4.1.4.1	FDD (with 4Tx enhanced codebook)

For the parameters specified in Table 9.4.1.4.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.4.1-2.

Table 9.4.1.4.1-1 PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | High XP 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 6 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF 0000 00FF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Physical channel for RI reporting |  |  | PUSCH |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |
| cqi-pmi-ConfigurationIndex |  |  | 4 |
| ri-ConfigIndex |  |  | 1 |
| Measurement channel |  |  | R.60 FDD |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | True |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4 |  |  |  |

Table 9.4.1.4.1-2 Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.8 |
| UE Category | ≥1 |

##### 9.4.1.4.2	TDD (with 4Tx enhanced codebook)

For the parameters specified in Table 9.4.1.4.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.4.2-2.

Table 9.4.1.4.2-1 PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | High XP 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 6 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF 0000 00FF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 15 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Physical channel for RI reporting |  |  | PUSCH |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |
| cqi-pmi-ConfigurationIndex |  |  | 4 |
| ri-ConfigIndex |  |  | 1 |
| Measurement channel |  |  | R.60 TDD |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | True |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. |  |  |  |

Table 9.4.1.4.2-2 Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.8 |
| UE Category | ≥1 |

##### 9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)

For the parameters specified in Table 9.4.1.4.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.4.3-2.

Table 9.4.1.4.3-1 PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 | Test 1a |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 |
| Transmission mode |  |  | 9 | 9 |
| Propagation channel |  |  | EPA5 | EPA5 |
| Precoding granularity |  | PRB | 50 | 50 |
| Correlation and antenna configuration |  |  | ULA Low 4 x 2 | ULA Low 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| FrequencyDensityBeamformed |  |  | 1 | 1/2 |
| NZP-TransmissionCombBeamformed |  |  | N/A | 0 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 6 | 6 |
| eMIMO-Type |  |  | Class B | Class B |
| alternativeCodebookEnabledCLASSB_K1 |  |  | TRUE | TRUE |
| codebookSubsetRestriction-3 |  |  | 00 0000 0000 0000 1111 1111 | 00 0000 0000 0000 1111 1111 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
|  | Pc | dB | -6 | -6 |
|  |  | dB | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Reporting mode |  |  | PUCCH 1-1 | PUCCH 1-1 |
| Reporting interval |  | ms | 5 | 5 |
| PMI delay (Note 2) |  | ms | 10 | 10 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) | PUSCH (Note 3) |
| PUCCH Report Type for CQI/PMI |  |  | 2 | 2 |
| Physical channel for RI reporting |  |  | PUSCH | PUSCH |
| PUCCH Report Type for RI |  |  | 3 | 3 |
| cqi-pmi-ConfigurationIndex |  |  | 2 | 2 |
| ri-ConfigIndex |  |  | 1 | 1 |
| Measurement channel |  |  | R.45 FDD | R.45A-2 FDD |
| Rank number of PDSCH |  |  | 1 | 1 |
| OCNG Pattern |  |  | OP.1 FDD | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDSCH_RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. |  |  |  |  |

Table 9.4.1.4.3-2 Minimum requirement (FDD)

| Parameter | Test 1 | Test 1a |
| --- | --- | --- |
|  | 1.1 | [1.1] |
| UE Category | ≥2 | ≥2 |
| Note1: For UE that indicates support of density reduction for Class B, test 1a will be run and test 1 will be skipped. Otherwise, test 1 will be run and test 1a will be skipped. |  |  |

##### 9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)

For the parameters specified in Table 9.4.1.4.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.1.4.4-2.

Table 9.4.1.4.4-1 PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 | Test 1a |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 |
| Transmission mode |  |  | 9 | 9 |
| Uplink downlink configuration |  |  | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 |
| Propagation channel |  |  | EPA5 | EPA5 |
| Precoding granularity |  | PRB | 50 | 50 |
| Correlation and antenna configuration |  |  | ULA Low 8 x 2 | ULA Low 8 x 2 |
| Beamforming model |  |  | Annex B.4.3 | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 | Antenna ports 15,…,22 |
| FrequencyDensityBeamformed |  |  | 1 | 1/2 |
| NZP-TransmissionCombBeamformed |  |  | N/A | 0 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 0 | 0 |
| eMIMO-Type |  |  | Class B | Class B |
| alternativeCodebookEnabledCLASSB_K1 |  |  | TRUE | TRUE |
| codebookSubsetRestriction-3 |  |  | 0x 000 0000 0000 FFFF | 0x 000 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 | 0 |
|  | Pc | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Reporting mode |  |  | PUCCH 1-1 | PUCCH 1-1 |
| Reporting interval |  | ms | 5 | 5 |
| PMI delay (Note 2) |  | ms | 10 | 10 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) | PUSCH (Note 3) |
| PUCCH Report Type for CQI/ PMI |  |  | 2 | 2 |
| Physical channel for RI reporting |  |  | PUSCH | PUSCH |
| PUCCH Report Type for RI |  |  | 3 | 3 |
| cqi-pmi-ConfigurationIndex |  |  | 4 | 4 |
| ri-ConfigIndex |  |  | 805 | 805 |
| Measurement channel |  |  | R.45 TDD | R.45A TDD |
| Rank number of PDSCH |  |  | 1 | 1 |
| OCNG Pattern |  |  | OP.1 TDD | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. |  |  |  |  |

Table 9.4.1.4.4-2 Minimum requirement (TDD)

| Parameter | Test 1 | Test 1a |
| --- | --- | --- |
|  | 1.2 | [1.2] |
| UE Category | ≥2 | ≥2 |
| Note1: For UE that indicates support of density reduction for Class B, test 1a will be run and test 1 will be skipped. Otherwise, test 1 will be run and test 1a will be skipped. |  |  |

### 9.4.1a	Void

#### 9.4.1a.1	Void

##### 9.4.1a.1.1	Void

### 9.4.1a.1.2	Void

### 9.4.2	Multiple PMI

#### 9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)

##### 9.4.2.1.1	FDD

For the parameters specified in Table 9.4.2.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.1.1-2.

Table 9.4.2.1.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Low 2 x 2 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 1 |
| PMI delay |  | ms | 8 |
| Measurement channel |  |  | R.11-3 FDD for UE Category 1, R.11 FDD for UE Category ≥2 |
| OCNG Pattern |  |  | OP.1/2 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoders shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: One/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2 shall be used. |  |  |  |

Table 9.4.2.1.1-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

##### 9.4.2.1.2	TDD

For the parameters specified in Table 9.4.2.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.1.2-2.

Table 9.4.2.1.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Low 2 x 2 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 1 |
| PMI delay |  | ms | 10 or 11 |
| Measurement channel |  |  | R.11-3 TDD for UE Category 1 R.11 TDD for UE Category ≥2 |
| OCNG Pattern |  |  | OP.1/2 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoders shall be updated in each available downlink transmission instance. Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: One/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2 shall be used. |  |  |  |

Table 9.4.2.1.2-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

#### 9.4.2.2		Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)

##### 9.4.2.2.1	FDD

For the parameters specified in Table 9.4.2.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.2.1-2.

Table 9.4.2.2.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Propagation channel |  |  | EVA5 |
| Correlation and antenna configuration |  |  | Low 4 x 2 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 3 |
|  |  | dB[mW/15kHz] | -98 |
| PMI delay |  | ms | 8 |
| Reporting mode |  |  | PUSCH 2-2 |
| Reporting interval |  | ms | 1 |
| Measurement channel |  |  | R.14-2 FDD |
| OCNG Pattern |  |  | OP.1/2 FDD |
| Subband size (k) |  | RBs | 3 (full size) |
| Number of preferred subbands (M) |  |  | 5 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) |  |  |  |

Table 9.4.2.2.1-2: Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

##### 9.4.2.2.2	TDD

For the parameters specified in Table 9.4.2.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.2.2-2.

Table 9.4.2.2.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 6 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Correlation and antenna configuration |  |  | Low 4 x 2 |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 |
|  |  | dB | 3 |
|  |  | dB[mW/15kHz] | -98 |
| PMI delay |  | ms | 10 |
| Reporting mode |  |  | PUSCH 2-2 |
| Reporting interval |  | ms | 1 |
| Measurement channel |  |  | R.14-2 TDD |
| OCNG Pattern |  |  | OP.1/2 TDD |
| Subband size (k) |  | RBs | 3 (full size) |
| Number of preferred subbands (M) |  |  | 5 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoders shall be updated in each available downlink transmission instance. Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.4.2.2.2-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 1.15 |
| UE Category | ≥1 |

#### 9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)

##### 9.4.2.3.1	FDD

For the parameters specified in Table 9.4.2.3.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.3.1-2.

Table 9.4.2.3.1-1: PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Low ULA 4 x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 8 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay |  | ms | 8 |
| Measurement channel |  |  | R.45-1 FDD for UE Category 1, R.45 FDD for UE Category ≥2 |
| OCNG Pattern |  |  | OP.7 FDD for UE Category 1 OP.1 FDD for UE Category ≥2 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoders shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Void. Note 4: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. |  |  |  |

Table 9.4.2.3.1-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |
| UE Category | ≥1 |

##### 9.4.2.3.2	TDD

For the parameters specified in Table 9.4.2.3.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.3.2-2.

Table 9.4.2.3.2-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Antenna configuration |  |  | 8 x 2 |
| Correlation modeling |  |  | High, Cross polarized |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | db | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 (Note 4) |
| PMI delay |  | ms | 10 |
| Measurement channel |  |  | R.45-1 TDD for UE Category 1, R.45 TDD for UE Category ≥2 |
| OCNG Pattern |  |  | OP.7 TDD for UE Category 1 OP.1 TDD for UE Category ≥2 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoders shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Void. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. |  |  |  |

Table 9.4.2.3.2-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 3.5 |
| UE Category | ≥1 |

##### 9.4.2.3.3	FDD (with 4Tx enhanced codebook)

For the parameters specified in Table 9.4.2.3.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.3.3-2.

Table 9.4.2.3.3-1 PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | High XP 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 8 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 FFFF 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.45-1 FDD for UE Category 1, R.45 FDD for UE Category ≥2 |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.7 FDD for UE Category 1 OP.1 FDD for UE Category ≥2 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | True |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Void. Note 4: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4 |  |  |  |

Table 9.4.2.3.3-2 Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

##### 9.4.2.3.4	TDD (with 4Tx enhanced codebook)

For the parameters specified in Table 9.4.2.3.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in 9.4.2.3.4-2.

Table 9.4.2.3.4-1 PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | XP High 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  |  | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 FFFF 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.61-1 TDD for UE Category 1, R.61 TDD for UE Category ≥2 |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.7 FDD for UE Category 1 OP.1 FDD for UE Category ≥2 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | True |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note3: Void. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. |  |  |  |

Table 9.4.2.3.4-2 Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 1.2 |
| UE Category | ≥1 |

##### 9.4.2.3.5	FDD (with Class A 16Tx codebook)

For the parameters specified in Table 9.4.2.3.5-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.5-2.

Table 9.4.2.3.5-1: PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | High 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,30 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.78 FDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.5-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |
| UE Category | ≥2 |

##### 9.4.2.3.6	TDD (with Class A 16Tx codebook)

For the parameters specified in Table 9.4.2.3.6-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.6-2.

Table 9.4.2.3.6-1: PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | High 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,26 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.78 TDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4 Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.6-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |
| UE Category | ≥2 |

##### 9.4.2.3.7	FDD (with Class A 32Tx codebook)

For the parameters specified in Table 9.4.2.3.7-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.7-2.

Table 9.4.2.3.7-1: PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | High 2D XP 32(4,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,46 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM8 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| FrequencyDensityNonPrecoded |  |  | 1 |
| NZP-TransmissionCombListNonprecoded |  |  | NA |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2,3} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 4 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 4 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.89 FDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.7-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | [4.5] |
| UE Category | ≥2 |

##### 9.4.2.3.8	TDD (with Class A 32Tx codebook)

For the parameters specified in Table 9.4.2.3.8-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.8-2.

Table 9.4.2.3.8-1: PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | High 2D XP 32(4,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,46 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM8 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| FrequencyDensityNonPrecoded |  |  | 1 |
| NZP-TransmissionCombListNonprecoded |  |  | NA |
| NZP-CSI-RS-Configuration-List |  |  | {0,1,2,3} |
| eMIMO-Type |  |  | Class A |
| codebookConfig-N1 |  |  | 4 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 0000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.89 TDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3B.4 Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.8-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | [4.5] |
| UE Category | ≥2 |

##### 9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)

For the parameters specified in Table 9.4.2.3.9-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.9-2.

Table 9.4.2.3.9-1: PMI test for dual-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Medium 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,30 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| advancedCodebookEnabled |  |  | TRUE |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 8 |
| Measurement channel |  |  | R.78 FDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 4: Randomization of the dual-cluster beam directions shall be used as specified in B.2.3B.4A. The value of relative power ratio (p) shall be fixed as 1 during the test. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.9-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.03] |
| UE Category | ≥2 |

##### 9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)

For the parameters specified in Table 9.4.2.3.10-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.4.2.3.10-2.

Table 9.4.2.3.10-1: PMI test for dual-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 6 |
| Correlation and antenna configuration |  |  | Medium 2D XP 16(2,4,2) x 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,26 |
| Beamforming model |  |  | Annex B.4.3 |
| CDM Type |  |  | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| NZP-CSI-RS-Configuration-List |  |  | {0,1} |
| eMIMO-Type |  |  | Class A |
| advancedCodebookEnabled |  |  | TRUE |
| codebookConfig-N1 |  |  | 2 |
| codebookConfig-N2 |  |  | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  |  | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  |  | 8 |
| Codebook-Config |  |  | Note 5 |
| codebookSubsetRestriction-1 |  |  | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  |  | Codebook-Config 1: 0000 1111 0000 Codebook-Config 2,3,4: 0x 00 000000 FFFF 0000 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 1-2 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.78 TDD |
| Rank Number of PDSCH |  |  | 2 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the dual-cluster beam directions shall be used as specified in B.2.3B.4A. The value of relative power ratio (p) shall be fixed as 1 during the test. Note 5: Value of parameter codebookConfig shall be random selected one value from UE supported codebook configurations. |  |  |  |

Table 9.4.2.3.10-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.03] |
| UE Category | ≥2 |

### 9.4.3	Void

## 9.5	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI (CQI) reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission. Transmission mode 4 is used with the specified CodebookSubSetRestriction in section 9.5.1, transmission mode 9 is used with the specified CodebookSubSetRestriction in section 9.5.2 and transmission mode 3 is used with the specified CodebookSubSetRestriction in section 9.5.3, and transmission mode 10 is used with the specified CodebookSubSetRestriction in section 9.5.5.

For fixed rank 1 transmission in sections 9.5.1, 9.5.2 and 9.5.5, the RI and PMI reporting is restricted to two single-layer precoders, For fixed rank 2 transmission in sections 9.5.1, 9.5.2 and 9.5.5, the RI and PMI reporting is restricted to one two-layer precoder, For follow RI transmission in sections 9.5.1 and 9.5.2, the RI and PMI reporting is restricted to select the union of these precoders. Channels with low and high correlation are used to ensure that RI reporting reflects the channel condition.

For fixed rank 1 transmission in section 9.5.3, the RI reporting is restricted to single-layer, for fixed rank 2 transmission in section 9.5.3, the RI reporting is restricted to two-layers. For follow RI transmission in section 9.5.3, the RI reporting is either one or two layers.

### 9.5.1	Minimum requirement (Cell-Specific Reference Symbols)

#### 9.5.1.1	FDD

The minimum performance requirement in Table 9.5.1.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.5.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.1.1-2.

Table 9.5.1.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |
| Downlink power allocation |  | dB | -3 |  |  |
|  |  | dB | -3 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  |
| Antenna correlation |  |  | Low | Low | High |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI |
| SNR |  | dB | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -98 | -78 | -78 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUCCH 1-1 (Note 4) |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |
| Physical channel for RI reporting |  |  | PUSCH (Note 3) |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |
| Reporting periodicity |  | ms | Npd= 5 |  |  |
| PMI and CQI delay |  | ms | 8 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |
| ri-ConfigurationInd |  |  | 1 (Note 5) |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between RI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic RI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 4: The bit field for precoding information in DCI format 2 shall be mapped as: ● For reported RI = 1 and PMI = 0 >> precoding information bit field index = 1 ● For reported RI = 1 and PMI = 1 >> precoding information bit field index = 2 ● For reported RI = 2 and PMI = 0 >> precoding information bit field index = 0 Note 5: To avoid the ambiguity of TE behaviour when applying CQI and PMI during rank switching, RI reports are to be applied at the TE with one subframe delay in addition to Note 1 to align with CQI and PMI reports. |  |  |  |  |  |

Table 9.5.1.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

#### 9.5.1.2	TDD

The minimum performance requirement in Table 9.5.1.2-2 is defined as

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.5.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.1.2-2.

Table 9.5.1.2-1: RI Test (TDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |
| Downlink power allocation |  | dB | -3 |  |  |
|  |  | dB | -3 |  |  |
|  |  | dB | 0 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  |
| Antenna correlation |  |  | Low | Low | High |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI |
| SNR |  | dB | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -98 | -78 | -78 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUSCH 3-1 (Note 3) |  |  |
| Reporting interval |  | ms | 5 |  |  |
| PMI and CQI delay |  | ms | 10 or 11 |  |  |
| ACK/NACK feedback mode |  |  | Bundling |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: Reported wideband CQI and PMI are used and sub-band CQI is discarded. |  |  |  |  |  |

Table 9.5.1.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

### 9.5.2	Minimum requirement (CSI Reference Symbols)

#### 9.5.2.1	FDD

The minimum performance requirement in Table 9.5.2.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.5.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.2.1-2.

Table 9.5.2.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
|  | Pc | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0 |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |
| CSI reference signal configuration |  |  | 6 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  |
| Antenna correlation |  |  | Low | Low | High |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI |
| SNR |  | dB | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -98 | -78 | -78 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUCCH 1-1 |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |
| PMI and CQI delay |  | ms | 8 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |
| ri-ConfigurationInd |  |  | 1 (Note 4) |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.9 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. Note 4: To avoid the ambiguity of TE behaviour when applying CQI and PMI during rank switching, RI reports are to be applied at the TE with one subframe delay in addition to Note 1 to align with CQI and PMI reports. |  |  |  |  |  |

Table 9.5.2.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

#### 9.5.2.2	TDD

The minimum performance requirement in Table 9.5.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.5.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.2.2-2.

Table 9.5.2.2-1: RI Test (TDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
|  | Pc | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Uplink downlink configuration |  |  | 1 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0 |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |
| CSI reference signal configuration |  |  | 4 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/4 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  |
| Antenna correlation |  |  | Low | Low | High |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI |
| SNR |  | dB | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -98 | -78 | -78 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUCCH 1-1 |  |  |
| Physical channel for CQI/ PMI reporting |  |  | PUSCH (Note 3) |  |  |
| PUCCH report type for CQI/ PMI |  |  | 2 |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |
| PMI and CQI delay |  | ms | 10 |  |  |
| ACK/NACK feedback mode |  |  | Bundling |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 4 |  |  |
| ri-ConfigurationInd |  |  | 1 |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.9 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#3 and #8. |  |  |  |  |  |

Table 9.5.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

### 9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)

#### 9.5.3.1	FDD

The minimum performance requirement in Table 9.5.3.1-2 is defined as

a)	 The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be  1

For the parameters specified in Table 9.5.3.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.3.1-2.

Table 9.5.3.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| Bandwidth |  | MHz | 10 |  |  | 10 |  |
| PDSCH transmission mode |  |  | 3 | Note 10 |  | 3 | Note 10 |
| Downlink power allocation |  | dB | -3 |  |  | -3 |  |
|  |  | dB | -3 |  |  | -3 |  |
|  |  | dB | 0 |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  | 2 x 2 EPA5 |  |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A |
| Antenna correlation |  |  | Low |  |  | Low |  |
| RI configuration |  |  | Fixed RI=1 and follow RI | N/A |  | Fixed RI=1 and follow RI | N/A |
|  |  | dB | 0 | -12 |  | 20 | 6 |
|  |  | dBmW/15kHz | -98 (Note 3) | N/A |  | -102 (Note 3) | N/A |
|  |  |  | -98 (Note 4) | N/A |  | -98 (Note 4) | N/A |
|  |  |  | -98 (Note 5) | N/A |  | -94.8 (Note 5) | N/A |
|  |  | dB[mW/15kHz] | -98 | -110 |  | -78 | -92 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |  | Non-MBSFN | Non-MBSFN |
| Cell Id |  |  | 0 | 1 |  | 0 | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  |  | 2.5 (synchronous cells) |  |
| ABS Pattern (Note 6) |  |  | N/A | 10000000 10000000 10000000 10000000 10000000 |  | N/A | 10000000 10000000 10000000 10000000 10000000 |
| RLM/RRM Measurement Subframe Pattern (Note 7) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  | 10000000 10000000 10000000 10000000 10000000 | N/A |
| CSI Subframe Sets (Note 8) | CCSI,0 |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  | 10000000 10000000 10000000 10000000 10000000 | N/A |
|  | CCSI,1 |  | 01111111 01111111 01111111 01111111 01111111 |  |  | 01111111 01111111 01111111 01111111 01111111 |  |
| Number of control OFDM Symbols |  |  | 3 | 3 |  | 3 | 3 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  | 1 |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  | PUCCH 1-0 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |
| PUCCH Report Type for CQI |  |  | 4 |  |  | 4 |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  | PUCCH Format 2 |  |
| PUCCH Report Type for RI |  |  | 3 |  |  | 3 |  |
| Reporting periodicity |  | ms | Npd= 10 |  |  | Npd= 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 11 |  |  | 11 |  |
| ri-ConfigurationInd |  |  | 5 |  |  | 5 |  |
| cqi-pmi-ConfigurationIndex2 |  |  | 10 |  |  | 10 |  |
| ri-ConfigurationInd2 |  |  | 2 |  |  | 2 |  |
| Cyclic prefix |  |  | Normal |  | Normal | Normal | Normal |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel in Cell 1 RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 5: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 6: ABS pattern as defined in [9]. Note 7: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 8: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 9: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell 1 and Cell 2 is the same. Note 10: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.1.5. |  |  |  |  |  |  |  |

Table 9.5.3.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| 1 | 0.9 | 1.05 |
| UE Category | ≥2 | ≥2 |

#### 9.5.3.2	TDD

The minimum performance requirement in Table 9.5.3.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be  1.

For the parameters specified in Table 9.5.3.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.3.2-2.

Table 9.5.3.2-1: RI Test (TDD)

| Parameter |  | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Bandwidth |  | MHz | 10 |  | 10 |  |
| PDSCH transmission mode |  |  | 3 | Note 11 | 3 | Note 11 |
| Uplink downlink configuration |  |  | 1 |  | 1 |  |
| Special subframe configuration |  |  | 4 |  | 4 |  |
| Downlink power allocation |  | dB | -3 |  | -3 |  |
|  |  | dB | -3 |  | -3 |  |
|  |  | dB | 0 |  | 0 |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  | 2 x 2 EPA5 |  |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A |
| Antenna correlation |  |  | Low |  | Low |  |
| RI configuration |  |  | Fixed RI=1 and follow RI | N/A | Fixed RI=1 and follow RI | N/A |
|  |  | dB | 0 | -12 | 20 | 6 |
|  |  | dB[mW/15kHz] | -98 (Note 4) | N/A | -102 (Note 4) | N/A |
|  |  |  | -98 (Note 5) | N/A | -98 (Note 5) | N/A |
|  |  |  | -98 (Note 6) | N/A | -94.8 (Note 6) | N/A |
|  |  | dB[mW/15kHz] | -98 | -110 | -78 | -92 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Cell Id |  |  | 0 | 1 | 0 | 1 |
| Time Offset between Cells |  | s | 2.5 (synchronous cells) |  | 2.5 (synchronous cells) |  |
| ABS Pattern (Note 7) |  |  | N/A | 0000000001 0000000001 | N/A | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 8) |  |  | 0000000001 0000000001 | N/A | 0000000001 0000000001 | N/A |
| CSI Subframe Sets (Note 9) | CCSI,0 |  | 0000000001 0000000001 | N/A | 0000000001 0000000001 | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 |  | 1100111000 1100111000 |  |
| Number of control OFDM Symbols |  |  | 3 | 3 | 3 | 3 |
| Maximum number of HARQ transmissions |  |  | 1 |  | 1 |  |
| Reporting mode |  |  | PUCCH 1-0 |  | PUCCH 1-0 |  |
| Physical channel for CCSI,0 CQI and RI reporting |  |  | PUCCH Format 2 |  | PUCCH Format 2 |  |
| PUCCH Report Type for CQI |  |  | 4 |  | 4 |  |
| Physical channel for CCSI,1 CQI and RI reporting |  |  | PUSCH (Note 3) |  | PUSCH (Note 3) |  |
| PUCCH Report Type for RI |  |  | 3 |  | 3 |  |
| Reporting periodicity |  | ms | Npd= 10 |  | Npd= 10 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  | Multiplexing |  |
| cqi-pmi-ConfigurationIndex |  |  | 8 |  | 8 |  |
| ri-ConfigurationInd |  |  | 5 |  | 5 |  |
| cqi-pmi-ConfigurationIndex2 |  |  | 9 |  | 9 |  |
| ri-ConfigurationInd2 |  |  | 0 |  | 0 |  |
| Cyclic prefix |  |  | Normal | Normal | Normal | Normal |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel in Cell 1 RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between RI/CQI reports and HARQ-ACK it is necessary to report them on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic RI/CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 4: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS Note 5: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 6: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 7: ABS pattern as defined in [9]. Note 8: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7]. Note 9: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 10: Cell 1 is the serving cell. Cell 2 is the aggressor cell. The number of the CRS ports in Cell 1 and Cell 2 is the same. Note 11: Downlink physical channel setup in Cell 2 in accordance with Annex C.3.3 applying OCNG pattern as defined in Annex A.5.2.5. |  |  |  |  |  |  |

Table 9.5.3.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| 1 | 0.9 | 1.05 |
| UE Category | ≥2 | ≥2 |

### 9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)

#### 9.5.4.1	FDD

For the parameters specified in Table 9.5.4.1-1, the minimum performance requirement in Table 9.5.4.1-2 is defined as

a)	 The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be  1;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

In Table 9.5.4.1-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggresso cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 9.5.4.1-1: RI Test (FDD)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 | 10 |
| PDSCH transmission mode |  |  | 3 | As defined in Note 1 | As defined in Note 1 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 | -3 | -3 |
|  |  | dB | 0 | N/A | N/A |
| Propagation condition and antenna configuration |  |  | 2×2 EPA5 (Note 2) | 2×2 EPA5 (Note 2) | 2×2 EPA5 (Note 2) |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | As defined in Note 1 | As defined in Note 1 |
| at antenna port |  | dB[mW/15kHz] | -98 (Note 3) | N/A | N/A |
|  |  | dB[mW/15kHz] | -98 (Note 4) | N/A | N/A |
|  |  | dB[mW/15kHz] | -93 (Note 5) | N/A | N/A |
|  |  | dB | Reference Value in Table 9.5.4.1-2 for each test | 12 | 10 |
|  |  | dB[mW/15kHz] | Reference Value in Table 9.5.4.1-2 for each test | -86 | -88 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 6) |  |  | N/A | 10000000 10000000 10000000 10000000 10000000 | 10000000 10000000 10000000 10000000 10000000 |
| RLM/RRM Measurement Subframe Pattern (Note 7) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |
| CSI Subframe Sets (Note 8) | CCSI,0 |  | 10000000 10000000 10000000 10000000 10000000 | N/A | N/A |
|  | CCSI,1 |  | 01111111 01111111 01111111 01111111 01111111 | N/A | N/A |
| Number of control OFDM symbols |  |  | 3 | Note 9 | Note 9 |
| Maximum number of HARQ transmissions |  |  | 1 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-0 | N/A | N/A |
| Physical channel for CQI reporting |  |  | PUCCH format 2 | N/A | N/A |
| PUCCH Report Type for CQI |  |  | 4 | N/A | N/A |
| Physical channel for RI reporting |  |  | PUCCH Format 2 | N/A | N/A |
| PUCCH Report Type for RI |  |  | 3 | N/A | N/A |
| Reporting periodicity |  | ms | Npd= 10 | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 11 | N/A | N/A |
| ri-ConfigurationInd |  |  | 5 | N/A | N/A |
| cqi-pmi-ConfigurationIndex2 |  |  | 10 | N/A | N/A |
| ri-ConfigurationInd2 |  |  | 2 | N/A | N/A |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern OP.5 FDD as defined in Annex A.5.1.5. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 5: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 6: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 7: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 8: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 9: The number of control OFDM symbols is not available for ABS and is 3 for the subframe indicated by “0” of ABS pattern. Note 10: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 11: Reference measurement channel in Cell 1 RC.2 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 12: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 13: SIB-1 will not be transmitted in Cell2 and Cell 3 in this test. |  |  |  |  |  |

Table 9.5.4.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| for Cell 1 (dB) | 4 | 20 | 20 |
| for Cell 1 (dB[mW/15kHz]) | -94 | -78 | -78 |
| Antenna correlation | High for Cell 1, low for Cell 2 and Cell 3 | Low for Cell 1, Cell 2 and Cell 3 | High for Cell 1, low for Cell 2 and Cell 3 |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.05 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

#### 9.5.4.2	TDD

For the parameters specified in Table 9.5.4.2-1, the minimum performance requirement in Table 9.5.4.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be  1;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

In Table 9.5.4.2-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggresso cells. The downlink physical channel setup for Cell 1 is according to Annex C.3.2 and for Cell 2 and Cell 3 is according to Annex C.3.3, respectively. The CRS assistance information [7] including Cell 2 and Cell 3 is provided.

Table 9.5.4.2-1: RI Test (TDD)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 10 | 10 |
| PDSCH transmission mode |  |  | 3 | As defined in Note 1 | As defined in Note 1 |
| Uplink downlink configuration |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  | 4 | 4 | 4 |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 | -3 | -3 |
|  |  | dB | 0 | N/A | N/A |
| Propagation condition and antenna configuration |  |  | 2×2 EPA5 (Note 2) | 2×2 EPA5 (Note 2) | 2×2 EPA5 (Note 2) |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | As defined in Note 1 | As defined in Note 1 |
| at antenna port |  | dB[mW/15kHz] | -98 (Note 3) | N/A | N/A |
|  |  | dB[mW/15kHz] | -98 (Note 4) | N/A | N/A |
|  |  | dB[mW/15kHz] | -93 (Note 5) | N/A | N/A |
|  |  | dB | Reference Value in Table 9.5.4.2-2 for each test | 12 | 10 |
|  |  | dB[mW/15kHz] | Reference Value in Table 9.5.4.2-2 for each test | -86 | -88 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 126 | 1 |
| ABS pattern (Note 6) |  |  | N/A | 0000000001 0000000001 | 0000000001 0000000001 |
| RLM/RRM Measurement Subframe Pattern (Note 7) |  |  | 0000000001 0000000001 | N/A | N/A |
| CSI Subframe Sets (Note 8) | CCSI,0 |  | 0000000001 0000000001 | N/A | N/A |
|  | CCSI,1 |  | 1100111000 1100111000 | N/A | N/A |
| Number of control OFDM symbols |  |  | 3 | Note 9 | Note 9 |
| Maximum number of HARQ transmissions |  |  | 1 | N/A | N/A |
| Reporting mode |  |  | PUCCH 1-0 | N/A | N/A |
| Physical channel for CCSI,0 CQI and RI reporting |  |  | PUCCH format 2 | N/A | N/A |
| Physical channel for CCSI,1 CQI and RI reporting |  |  | PUSCH (Note 14) | N/A | N/A |
| PUCCH Report Type for CQI |  |  | 4 | N/A | N/A |
| PUCCH Report Type for RI |  |  | 3 | N/A | N/A |
| Reporting periodicity |  | ms | Npd= 10 | N/A | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 8 | N/A | N/A |
| ri-ConfigurationInd |  |  | 5 | N/A | N/A |
| cqi-pmi-ConfigurationIndex2 |  |  | 9 | N/A | N/A |
| ri-ConfigurationInd2 |  |  | 0 | N/A | N/A |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 1: Downlink physical channel setup in Cell 2 and Cell 3 in accordance with Annex C.3.3 applying OCNG pattern OP.5 TDD as defined in Annex A.5.2.5. Note 2: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 3: This noise is applied in OFDM symbols #1, #2, #3, #5, #6, #8, #9, #10,#12, #13 of a subframe overlapping with the aggressor ABS. Note 4: This noise is applied in OFDM symbols #0, #4, #7, #11 of a subframe overlapping with the aggressor ABS. Note 5: This noise is applied in all OFDM symbols of a subframe overlapping with aggressor non-ABS Note 6: ABS pattern as defined in [9]. PDSCH other than SIB1/paging and its associated PDCCH/PCFICH are transmitted in the serving cell subframe when the subframe is overlapped with the ABS subframe of aggressor cell and the subframe is available in the definition of the reference channel. Note 7: Time-domain measurement resource restriction pattern for PCell measurements as defined in [7] Note 8: As configured according to the time-domain measurement resource restriction pattern for CSI measurements defined in [7]. Note 9: The number of control OFDM symbols is not available for ABS and is 3 for the subframe indicated by “0” of ABS pattern. Note 10: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 11: Reference measurement channel in Cell 1 RC.2 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 12: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 13: SIB-1 will not be transmitted in Cell2 and Cell 3 in this test. Note 14: To avoid collisions between RI/CQI reports and HARQ-ACK it is necessary to report them on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic RI/CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |  |

Table 9.5.4.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| for Cell 1 (dB) | 4 | 20 | 20 |
| for Cell 1 (dB[mW/15kHz]) | -94 | -78 | -78 |
| Antenna correlation | High for Cell 1, low for Cell 2 and Cell 3 | Low for Cell 1, Cell 2 and Cell 3 | High for Cell 1, low for Cell 2 and Cell 3 |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.05 | N/A | N/A |
| UE Category | ≥2 | ≥2 | ≥2 |

### 9.5.5	Minimum requirement (with CSI process)

Each CSI process is associated with a CSI-RS resource and a CSI-IM resource as shown in Table 9.5.5-1.

For UE supports one CSI process, CSI process 0 is configured for Test 1 and Test 2, but CSI process 1 is not configured for Test 2. The corresponding  requirements for Test 1 and Test 2 shall be fulfilled. The requirement on reported RI for CSI process 1 in Test 2 is not applicable.

For UE supports multiple CSI processes, CSI process 0 is configured for Test 1 and CSI processes 0 and 1 are configured for Test 2. The corresponding  requirements for Test 1 and Test 2 shall be fulfilled, and also the requirement on reported RI for CSI process 1 in Test 2.

Table 9.5.5-1: Configuration of CSI processes

|  | CSI process 0 | CSI process 1 |
| --- | --- | --- |
| CSI-RS resource | CSI-RS signal 0 | CSI-RS signal 1 |
| CSI-IM resource | CSI-IM resource 0 | CSI-IM resource 1 |

#### 9.5.5.1	FDD

The minimum performance requirement in Table 9.5.5.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

c)	For Test 2, the RI reported for CSI process 1 shall be the same as the most recent RI reported for CSI process 0 if UE is configured with multiple CSI processes.

For the parameters specified in Table 9.5.5.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.5.1-2.

Table 9.5.5.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 | TP1 | TP2 |
| Bandwidth |  | MHz | 10 MHz |  | 10 MHz |  |
| Transmission mode |  |  | 10 | 10 | 10 | 10 |
| Downlink power allocation |  | dB | 0 |  | 0 |  |
|  |  | dB | 0 |  | 0 |  |
|  |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 |  | 0 |  |
| SNR |  | dB | 0 | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -78 | -78 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA 5 Low | EPA 5 Low | EPA 5 Low | EPA 5 High |
| Antenna configuration |  |  | 2x2 | 2x2 | 2x2 | 2x2 |
| Beamforming Model |  |  | As specified in Section B.4.3 |  | As specified in Section B.4.3 |  |
| Timing offset between TPs |  | us | 0 |  | 0 |  |
| Frequency offset between TPs |  | Hz | 0 |  | 0 |  |
| Cell-specific reference signals |  |  | Antenna ports 0 |  | Antenna ports 0 |  |
| CSI-RS signal 0 |  |  | Antenna ports 15,16 | N/A | Antenna ports 15,16 | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 | N/A | 5/1 | N/A |
| CSI-RS 0 configuration |  |  | 0 | N/A | 0 | N/A |
| CSI-RS signal 1 |  |  | N/A | Antenna ports 15,16 | N/A | Antenna ports 15,16 |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A | 5/1 | N/A | 5/1 |
| CSI-RS 1 configuration |  |  | N/A | 3 | N/A | 3 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | N/A | 1 / 1000001000000000 | N/A | 1] / 1000001000000000 |
| Zero-power CSI-RS 1 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 1 / 0011000000000000 | N/A | 1 / 0011000000000000 | N/A |
| CSI-IM 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 | N/A | 5/1 | N/A |
| CSI-IM 0 configuration |  |  | 2 | N/A | 2 | N/A |
| CSI-IM 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A | 5/1 | N/A | 5/1 |
| CSI-IM 1 configuration |  |  | N/A | 6 | N/A | 6 |
| RI configuration |  |  | Fixed RI=2 and follow RI | N/A | Fixed RI=1 and follow RI | N/A |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 6) | N/A | PUSCH (Note 6) | PUSCH (Note 6) |
| PUCCH Report Type for CQI/PMI |  |  | 2 | N/A | 2 | 2 |
| Physical channel for RI reporting |  |  | PUCCH Format 2 | N/A | PUCCH Format 2 | PUCCH Format 2 |
| PUCCH Report Type for RI |  |  | 3 | N/A | 3 | 3 |
| CSI process 0 (Note 7) | CSI-RS |  | CSI-RS 0 | N/A | CSI-RS 0 | N/A |
|  | CSI-IM |  | CSI-IM 0 | N/A | CSI-IM 0 | N/A |
|  | Reporting mode |  | PUCCH 1-1 | N/A | PUCCH 1-1 | N/A |
|  | Reporting periodicity | ms | Npd = 5 | N/A | Npd = 5 | N/A |
|  | CQI delay | ms | 8 | N/A | 10 | N/A |
|  | cqi-pmi-ConfigurationIndex |  | 6 | N/A | 6 | N/A |
|  | ri-ConfigIndex |  | 1 | N/A | 1 | N/A |
| CSI process 1 (Note 7, Note 9) | CSI-RS |  | N/A | N/A | N/A | CSI-RS 1 |
|  | CSI-IM |  | N/A | N/A | N/A | CSI-IM 1 |
|  | Reporting mode |  | N/A | N/A | N/A | PUCCH 1-1 |
|  | Reporting periodicity | ms | N/A | N/A | N/A | Npd = 5 |
|  | CQI delay | ms | N/A | N/A | N/A | 10 |
|  | cqi-pmi-ConfigurationIndex |  | N/A | N/A | N/A | 4 |
|  | ri-ConfigIndex |  | N/A | N/A | N/A | 1 |
| CSI process for PDSCH scheduling |  |  | CSI process 0 |  | CSI process 0 |  |
| Cell ID |  |  | 0 | 6 | 0 | 6 |
| Quasi-co-located CSI-RS |  |  | CSI-RS 0 | CSI-RS 1 | CSI-RS 0 | CSI-RS 1 |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 | Same Cell ID as Cell 2 | Same Cell ID as Cell 1 | Same Cell ID as Cell 2 |
| PMI for subframe 2, 3, 4, 7, 8 and 9 |  |  | 010000 for fixed RI = 2 010011 for UE reported RI | 100000 | 000011 for fixed RI = 1 010011 for UE reported RI | N/A |
| PMI for subframe 1 and 6 |  |  | 100000 | 100000 | 100000 | N/A |
| Max number of HARQ transmissions |  |  | 1 | N/A | 1 | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: 3 symbols allocated to PDCCH Note 3: Reference measurement channel RC.13 FDD according to Table A.4-1. PDSCH transmission is scheduled on subframe 2, 3, 4, 7, 8 and 9 from TP1. Note 4: TM10 OCNG as specified in A.5.1.8 is transmitted on subframe 1 and 6 from TP1. Note 5: TM10 OCNG as specified in A.5.1.8 is transmitted on subframe 1, 2, 3, 4, 6, 7, 8 and 9 from TP2 for Test 1; TP2 is blanked for Test 2. Note 6: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. Note 7: If UE supports multiple CSI processes, CSI process 0 is configured as ‘RI-reference CSI process’ for CSI process 1. Note 8: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#1 and #6 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#0 and #5. Note 9: If UE supports one CSI process, CSI process 1 is not configured in Test 2. |  |  |  |  |  |  |

Table 9.5.5.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| 1 | N/A | 1.0 |
| 2 | 1.0 | N/A |
| UE Category | ≥2 | ≥2 |

#### 9.5.5.2	TDD

The minimum performance requirement in Table 9.5.5.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

c)	For Test 2, the RI reported for CSI process 1 shall be the same as the most recent RI reported for CSI process 0 if UE is configured with multiple CSI processes.

For the parameters specified in Table 9.5.5.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.5.5.2-2.

Table 9.5.5.2-1: RI Test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP1 | TP2 | TP1 | TP2 |
| Bandwidth |  | MHz | 10 MHz |  | 10 MHz |  |
| Transmission mode |  |  | 10 | 10 | 10 | 10 |
| Downlink power allocation |  | dB | 0 |  | 0 |  |
|  |  | dB | 0 |  | 0 |  |
|  |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 |  | 0 |  |
| Uplink downlink configuration |  |  | 2 | 2 | 2 | 2 |
| Special subframe configuration |  |  | 4 | 4 | 4 | 4 |
| SNR |  | dB | 0 | 0 | 20 | 20 |
|  |  | dB[mW/15kHz] | -98 | -98 | -78 | -78 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA 5 Low | EPA 5 Low | EPA 5 Low | EPA 5 High |
| Antenna configuration |  |  | 2x2 | 2x2 | 2x2 | 2x2 |
| Beamforming Model |  |  | As specified in Section B.4.3 |  | As specified in Section B.4.3 |  |
| Timing offset between TPs |  | us | 0 |  | 0 |  |
| Frequency offset between TPs |  | Hz | 0 |  | 0 |  |
| Cell-specific reference signals |  |  | Antenna ports 0 |  | Antenna ports 0 |  |
| CSI-RS signal 0 |  |  | Antenna ports 15,16 | N/A | Antenna ports 15,16 | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 | N/A | 5/3 | N/A |
| CSI-RS 0 configuration |  |  | 0 | N/A | 0 | N/A |
| CSI-RS signal 1 |  |  | N/A | Antenna ports 15,16 | N/A | Antenna ports 15,16 |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A | 5/3 | N/A | 5/3 |
| CSI-RS 1 configuration |  |  | N/A | 3 | N/A | 3 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | N/A | 3 / 1000001000000000 | N/A | 3 / 1000001000000000 |
| Zero-power CSI-RS 1 configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | 3 / 0011000000000000 | N/A | 3 / 0011000000000000 | N/A |
| CSI-IM 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 | N/A | 5/3 | N/A |
| CSI-IM 0 configuration |  |  | 2 | N/A | 2 | N/A |
| CSI-IM 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | N/A | 5/3 | N/A | 5/3 |
| CSI-IM 1 configuration |  |  | N/A | 6 | N/A | 6 |
| RI configuration |  |  | Fixed RI=2 and follow RI | N/A | Fixed RI=1 and follow RI | N/A |
| CSI process 0 (Note 6, 7) | CSI-RS |  | CSI-RS 0 | N/A | CSI-RS 0 | N/A |
|  | CSI-IM |  | CSI-IM 0 | N/A | CSI-IM 0 | N/A |
|  | Reporting mode |  | PUSCH 3-1 | N/A | PUSCH 3-1 | N/A |
|  | Reporting Interval | ms | 5 | N/A | 5 | N/A |
|  | CQI delay | ms | 11 | N/A | 11 | N/A |
| CSI process 1 (Note 6, 7, 8) | CSI-RS |  | N/A | N/A | N/A | CSI-RS 1 |
|  | CSI-IM |  | N/A | N/A | N/A | CSI-IM 1 |
|  | Reporting mode |  | N/A | N/A | N/A | PUSCH 3-1 |
|  | Reporting Interval | ms | N/A | N/A | N/A | 5 |
|  | CQI delay | ms | N/A | N/A | N/A | 11 |
| CSI process for PDSCH scheduling |  |  | CSI process 0 |  | CSI process 0 |  |
| Cell ID |  |  | 0 | 6 | 0 | 6 |
| Quasi-co-located CSI-RS |  |  | CSI-RS 0 | CSI-RS 1 | CSI-RS 0 | CSI-RS 1 |
| Quasi-co-located CRS |  |  | Same Cell ID as Cell 1 | Same Cell ID as Cell 2 | Same Cell ID as Cell 1 | Same Cell ID as Cell 2 |
| PMI for subframe 4 and 9 |  |  | 010000 for fixed RI = 2 010011 for UE reported RI | 100000 | 000011 for fixed RI = 1 010011 for UE reported RI | N/A |
| PMI for subframe 3 and 8 |  |  | 100000 | 100000 | 100000 | N/A |
| Max number of HARQ transmissions |  |  | 1 | N/A | 1 | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A | Multiplexing | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: 3 symbols allocated to PDCCH Note 3: Reference measurement channel RC.13 TDD according to Table A.4-1. PDSCH transmission is scheduled on subframe 4 and 9 from TP1. Note 4: TM10 OCNG as specified in A.5.2.8 is transmitted on subframe 3 and 8 from TP1. Note 5: TM10 OCNG as specified in A.5.2.8 is transmitted on subframe 3, 4, 8 and 9 from TP2 for Test 1; TP2 is blanked for Test 2. Note 6: Reported wideband CQI and PMI are used and sub-band CQI is discarded. Note 7: If UE supports multiple CSI processes, CSI process 0 is configured as ‘RI-reference CSI process’ for CSI process 1. Note 8: If UE supports one CSI process, CSI process 1 is not configured in Test 2. Note 9: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#3and #8 to allow aperiodic CQI/PMI/RI to be transmitted in uplink SF#7 and #2. |  |  |  |  |  |  |

Table 9.5.5.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
| 1 | N/A | 1.0 |
| 2 | 1.0 | N/A |
| UE Category | ≥2 | ≥2 |

## 9.6		Additional requirements for carrier aggregation

This clause includes requirements for the reporting of channel state information (CSI) with the UE configured for carrier aggregation. The purpose is to verify that the channel state for each cell is correctly reported with multiple cells configured for periodic reporting.

### 9.6.1		Periodic reporting on multiple cells (Cell-Specific Reference Symbols)

#### 9.6.1.1	FDD

The following requirements apply to UE Category ≥3. For CA with 2 DL CC, for the parameters specified in Table 9.6.1.1-1 and Table 9.6.1.1-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of Pcell and Scell reported shall be such that

wideband CQIPcell – wideband CQIScell ≥ 2

for more than 90% of the time.

Table 9.6.1.1-1: Parameters for PUCCH 1-0 static test on multiple cells (FDD, 2 DL CA)

| Parameter |  | Unit | Pcell | Scell |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 10 | 4 |  |
|  |  | dB[mW/15kHz] | -88 | -94 |  |
|  |  | dB[mW/15kHz] | -98 | -98 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 11 |  | 16 (shift of 5 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.1-2: PUCCH 1-0 static test (FDD, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 10MHz for both cells |
| 2 | 20MHz for both cells |
| 3 | 5MHz for both cells |
| 4 | 5MHz for PCell and 10MHz for SCell |
| 5 | 5MHz for PCell and 15MHz for SCell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: Mapping of PCell and Scell to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For CA with 3 DL CC, for the parameters specified in Table 9.6.1.1-3 and Table 9.6.1.1-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Category ≥8. For CA with 4 DL CC, for the parameters specified in Table 9.6.1.1-3 and Table 9.6.1.1-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, and SCell 1 and SCell 3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.1-3: Parameters for PUCCH 1-0 static test on multiple cells (FDD, 3 and 4 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 21 | 26 (shift of 5 ms relative to Pcell) | 31 for Scell2 (shift of 10 ms relative to Pcell) , 36 for Scell3 (shift of 15ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.1-4: PUCCH 1-0 static test (FDD, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 3x20 |
| 2 | 20+20+15 |
| 3 | 20+20+10 |
| 4 | 20+15+15 |
| 5 | 20+15+10 |
| 6 | 20+10+10 |
| 7 | 15+15+10 |
| 8 | 20+10+5 |
| 9 | 20+15+5 |
| 10 | 10+10+5 |
| 11 | 5+5+20 |
| 12 | 3x10 |
| 13 | 5+5+10 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.1-5: PUCCH 1-0 static test (FDD, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 4x20 |
| 2 | 20+20+20+10 |
| 3 | 20+20+10+10 |
| 4 | 20+20+10+5 |
| 5 | 20+10+10+5 |
| 6 | 15+3x20 |
| 7 | 2x15+2x20 |
| 8 | 10+15+2x20 |
| 9 | 3x10+20 |
| 10 | 2x5+2x20 |
| 11 | 2x5+10+20 |
| 12 | 4x10 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For CA with 5 DL CC, for the parameters specified in Table 9.6.1.1-6 and Table 9.6.1.1-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, and SCell 1 and SCell 4 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.1-6: Parameters for PUCCH 1-0 static test on multiple cells (FDD, 5 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 41 | 46 (shift of 5 ms relative to Pcell) | 51 for Scell 2 (shift of 10 ms relative to Pcell), 56 for Scell 3 (shift of 15ms relative to Pcell), 61 for Scell4 (shift of 20ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.1-7: PUCCH 1-0 static test (FDD, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 5x20 |
| 2 | 15+4x20 |
| 3 | 10+4x20 |
| 4 | 2x10+3x20 |
| 5 | 5+10+3x20 |
| 6 | 3x10+2x20 |
| 7 | 4x10+20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For CA with 6 DL CC, for the parameters specified in Table 9.6.1.1-8 and Table 9.6.1.1-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, SCell 1 and SCell 4, and SCell 1 and SCell 5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

for more than 90% of the time.

Table 9.6.1.1-8: Parameters for PUCCH 1-0 static test on multiple cells (FDD, 6 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 41 | 46 (shift of 5 ms relative to Pcell) | 51 for Scell 2 (shift of 10 ms relative to Pcell), 56 for Scell 3 (shift of 15ms relative to Pcell), 61 for Scell4 (shift of 20ms relative to Pcell), 66 for Scell5 (shift of 25ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.1-9: PUCCH 1-0 static test (FDD, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 6x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For CA with 7 DL CC, for the parameters specified in Table 9.6.1.1-10 and Table 9.6.1.1-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, SCell 1 and SCell 4, SCell 1 and SCell 5, and SCell 1 and SCell 6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

wideband CQISCell1 – wideband CQISCell6 ≥ 2

for more than 90% of the time.

Table 9.6.1.1-10: Parameters for PUCCH 1-0 static test on multiple cells (FDD, 7 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5, 6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 41 | 46 (shift of 5 ms relative to Pcell) | 51 for Scell 2 (shift of 10 ms relative to Pcell), 56 for Scell 3 (shift of 15ms relative to Pcell), 61 for Scell4 (shift of 20ms relative to Pcell), 66 for Scell5 (shift of 25ms relative to Pcell), 71 for Scell6 (shift of 30ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.1-11: PUCCH 1-0 static test (FDD, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 7x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

#### 9.6.1.2	TDD

The following requirements apply to UE Category ≥3. For CA with 2 DL CC, for the parameters specified in Table 9.6.1.2-1 and Table 9.6.1.2-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of Pcell and Scell reported shall be such that

wideband CQIPcell – wideband CQIScell ≥ 2

for more than 90% of the time.

Table 9.6.1.2-1: PUCCH 1-0 static test on multiple cells (TDD, 2 DL CA)

| Parameter |  | Unit | Pcell | Scell |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 10 | 4 |  |
|  |  | dB[mW/15kHz] | -88 | -94 |  |
|  |  | dB[mW/15kHz] | -98 | -98 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 8 |  | 13 (shift of 5 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2-2: PUCCH 1-0 static test (TDD, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 20MHz for both cells |
| 2 | 15MHz for PCell and 20MHz for SCell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: Mapping of PCell and Scell to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For CA with 3 DL CC, for the parameters specified in Table 9.6.1.2-3 and Table 9.6.1.2-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Category ≥8. For CA with 4 DL CC, for the parameters specified in Table 9.6.1.2-3 and Table 9.6.1.2-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, and SCell 1 and SCell 3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.2-3: PUCCH 1-0 static test on multiple cells (TDD, 3 and 4 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 18 | 23 (shift of 5 ms relative to Pcell) | 28 (shift of 10 ms relative to Pcell) , 33 for Scell3 (shift of 15ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2-4: PUCCH 1-0 static test (TDD, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 3x20 |
| 2 | 20+20+15 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.2-5: PUCCH 1-0 static test (TDD, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 4x20 |
| 2 | 20+20+20+15 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For CA with 5 DL CC, for the parameters specified in Table 9.6.1.2-6 and Table 9.6.1.2-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3,  and SCell 1 and SCell 4  reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.2-6: Parameters for PUCCH 1-0 static test on multiple cells (TDD, 5 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 (shift of 10 ms relative to Pcell), 53 for Scell3 (shift of 15ms relative to Pcell), 5 8 for Scell4 (shift of 20ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2-7: PUCCH 1-0 static test (TDD, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 5x20 |
| 2 | 15+4x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For CA with 6 DL CC, for the parameters specified in Table 9.6.1.2-8 and Table 9.6.1.2-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, SCell 1 and SCell 4, and SCell 1 and SCell 5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ [2]

wideband CQISCell1 – wideband CQISCell2 ≥ [2]

wideband CQISCell1 – wideband CQISCell3 ≥ [2]

wideband CQISCell1 – wideband CQISCell4 ≥ [2]

wideband CQISCell1 – wideband CQISCell5 ≥ [2]

for more than 90% of the time.

Table 9.6.1.2-8: Parameters for PUCCH 1-0 static test on multiple cells (TDD, 6 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 (shift of 10 ms relative to Pcell), 53 for Scell3 (shift of 15ms relative to Pcell), 58 for Scell4 (shift of 20ms relative to Pcell), 63 for Scell5 (shift of 25ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2-9: PUCCH 1-0 static test (TDD, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 6x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For CA with 7 DL CC, for the parameters specified in Table 9.6.1.2-10 and Table 9.6.1.2-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, SCell 1 and SCell 4, SCell 1 and SCell 5, SCell 1 and SCell 6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ [2]

wideband CQISCell1 – wideband CQISCell2 ≥ [2]

wideband CQISCell1 – wideband CQISCell3 ≥ [2]

wideband CQISCell1 – wideband CQISCell4 ≥ [2]

wideband CQISCell1 – wideband CQISCell5 ≥ [2]

wideband CQISCell1 – wideband CQISCell6 ≥ [2]

for more than 90% of the time.

Table 9.6.1.2-10: Parameters for PUCCH 1-0 static test on multiple cells (TDD, 7 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5, 6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 (shift of 10 ms relative to Pcell), 53 for Scell3 (shift of 15ms relative to Pcell), 58 for Scell4 (shift of 20ms relative to Pcell), 63 for Scell5 (shift of 25ms relative to Pcell), 68 for Scell6 (shift of 30ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2-11: PUCCH 1-0 static test (TDD, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 7x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

#### 9.6.1.3	TDD-FDD CA with FDD PCell

The following requirements apply to UE Category ≥5. For TDD-FDD CA with FDD PCell with 2 DL CC, for the parameters specified in Table 9.6.1.3-1 and Table 9.6.1.3-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell reported shall be such that

wideband CQIPCell – wideband CQISCell ≥ 2

for more than 90% of the time.

Table 9.6.1.3-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 2 DL CA)

| Parameter |  | Unit | PCell | SCell |
| --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |
| Uplink downlink configuration |  |  | N/A | 2 |
| Special subframe configuration |  |  | N/A | 4 |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |
| SNR |  | dB | 10 | 4 |
|  |  | dB[mW/15kHz] | -88 | -94 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 9 | 14 (shift of 5 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |

Table 9.6.1.3-2: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 20MHz for FDD cell and 20MHz for TDD cell |
| 2 | 10MHz for FDD cell and 20MHz for TDD cell |
| 3 | 15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For TDD-FDD CA with FDD PCell with 3 DL CC, for the parameters specified in Table 9.6.1.3-3 and Table 9.6.1.3-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Cateogry ≥8. For TDD-FDD CA with FDD PCell with 4 DL CC, for the parameters specified in Table 9.6.1.3-3 and Table 9.6.1.3-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, and SCell1 and SCell3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-3: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 3 and 4 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 19 | 24 (shift of 5 ms relative to Pcell) | 29 for SCell 2 (shift of 10 ms relative to Pcell) , 34 for SCell 3 (shift of 15ms relative to PCell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-4: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 15MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 10MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x20MHz for FDD cell and 20MHz for TDD cell |
| 5 | 20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 20+10MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 10MHz for FDD cell and 15+20MHz for TDD cell |
| 9 | 10+15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.3-5: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 20MHz for FDD cell and 3x20MHz for TDD cell |
| 2 | 2x20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 20+15MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x15MHz for FDD cell and 2x20MHz for TDD cell |
| 5 | 2x20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 2x15+20MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x20+10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 3x20MHz for FDD cell and 20MHz for TDD cell |
| 9 | 10MHz for FDD cell and 3x20MHz for TDD cell |
| 10 | 2x10MHz for FDD cell and 2x20MHz for TDD cell |
| 11 | 2x10MHz for FDD cell and 20+15MHz for TDD cell |
| 12 | 10+20MHz for FDD cell and 2x20MHz for TDD cell |
| 13 | 10+15MHz for FDD cell and 2x20MHz for TDD cell |
| 14 | 10+15+20MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 5 DL CC, for the parameters specified in Table 9.6.1.3-6 and Table 9.6.1.3-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, and SCell 1 and SCell 4 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-6: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 5 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-7: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 15MHz+2×20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 2×15MHz+20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 4x20(FDD)+20(TDD)MHz |
| 4 | 3x20(FDD)+2x20(TDD)MHz |
| 5 | 2x20(FDD)+3x20(TDD)NA |
| 6 | 20(FDD)+4x20(TDD)MHz |
| 7 | 10(FDD)+4x20(TDD)MHz |
| 8 | 2x10(FDD)+3x20(TDD)MH |
| 9 | 10+20(FDD)+3x20(TDD)MHz |
| 10 | 10+2×20(FDD)+2×20(TDD)MHz |
| 11 | 10+15+20(FDD)+2x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 6 DL CC, for the parameters specified in Table 9.6.1.3-8 and Table 9.6.1.3-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell 1 and SCell 4, and SCell 1 and SCell 5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-8: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 6 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell), 64 for SCell 5 (shift of 25 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-9: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1×20(FDD)+5×20(TDD) |
| 2 | 2×20(FDD)+4×20(TDD) |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 7 DL CC, for the parameters specified in Table 9.6.1.3-10 and Table 9.6.1.3-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell 1 and SCell 4, SCell 1 and SCell 5, SCell 1 and SCell 6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

wideband CQISCell1 – wideband CQISCell6 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-10: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 7 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5, SCell6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell), 64 for SCell 5 (shift of 25 ms relative to Pcell), 69 for SCell 6 (shift of 30 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-11: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1×20(FDD)+6×20(TDD) |
| 2 | 2×20(FDD)+5×20(TDD) |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

#### 9.6.1.4	TDD-FDD CA with TDD PCell

The following requirements apply to UE Category ≥5. For TDD-FDD CA with TDD PCell with 2 DL CC, for the parameters specified in Table 9.6.1.4-1 and Table 9.6.1.4-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell reported shall be such that

wideband CQIPCell – wideband CQISCell ≥ 2

for more than 90% of the time.

Table 9.6.1.4-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 2 DL CA)

| Parameter |  | Unit | PCell | SCell |
| --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |
| Uplink downlink configuration |  |  | 2 | N/A |
| Special subframe configuration |  |  | 4 | N/A |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |
| SNR |  | dB | 10 | 4 |
|  |  | dB[mW/15kHz] | -88 | -94 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 8 | 13 (shift of 5 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |

Table 9.6.1.4-2: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 20MHz for TDD cell and 10MHz for FDD cell |
| 3 | 20MHz for TDD cell and 15MHz for FDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For TDD-FDD CA with TDD PCell with 3 DL CC, for the parameters specified in Table 9.6.1.4-3 and Table 9.6.1.4-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Cateogry ≥8. For TDD-FDD CA with TDD PCell with 4 DL CC, for the parameters specified in Table 9.6.1.4-3 and Table 9.6.1.4-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, and SCell1 and SCell3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-3: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 3 and 4 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 18 | 23 (shift of 5 ms relative to Pcell) | 28 for SCell 2 (shift of 10 ms relative to Pcell) , 33 for SCell 3 (shift of 15ms relative to PCell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-4: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 2x20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 2x20MHz for TDD cell and 15MHz for FDD cell |
| 3 | 2x20MHz for TDD cell and 10MHz for FDD cell |
| 4 | 2x20MHz for FDD cell and 20MHz for TDD cell |
| 5 | 20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 20+10MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 10MHz for FDD cell and 15+20MHz for TDD cell |
| 9 | 10+15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.4-5: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 3x20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 2x20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 20+15MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x15MHz for FDD cell and 2x20MHz for TDD cell |
| 5 | 2x20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 2x15+20MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x20+10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 3x20MHz for FDD cell and 20MHz for TDD cell |
| 9 | 10MHz for FDD cell and 3x20MHz for TDD cell |
| 10 | 2x10MHz for FDD cell and 2x20MHz for TDD cell |
| 11 | 2x10MHz for FDD cell and 20+15MHz for TDD cell |
| 12 | 10+20MHz for FDD cell and 2x20MHz for TDD cell |
| 13 | 10+15MHz for FDD cell and 2x20MHz for TDD cell |
| 14 | 10+15+20MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 5 DL CC, for the parameters specified in Table 9.6.1.4-6 and Table 9.6.1.4-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3 and SCell 1 and SCell 4 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-6: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 5 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-7: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 15MHz+2×20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 2×15MHz+20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 4x20(FDD)+20(TDD)MHz |
| 4 | 3x20(FDD)+2x20(TDD)MHz |
| 5 | 2x20(FDD)+3x20(TDD)NA |
| 6 | 20(FDD)+4x20(TDD)MHz |
| 7 | 10(FDD)+4x20(TDD)MHz |
| 8 | 2x10(FDD)+3x20(TDD)MH |
| 9 | 10+20(FDD)+3x20(TDD)MHz |
| 10 | 10+2×20(FDD)+2×20(TDD)MHz |
| 11 | 10+15+20(FDD)+2x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 6 DL CC, for the parameters specified in Table 9.6.1.4-8 and Table 9.6.1.4-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell1 and SCell4, and SCell1 and SCell5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-8: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 6 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell), 63 for SCell 5 (shift of 25 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-9: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1x20(FDD)+ 5x20(TDD)MHz |
| 2 | 2x20(FDD)+4x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 7 DL CC, for the parameters specified in Table 9.6.1.4-10 and Table 9.6.1.4-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell1 and SCell4, SCell1 and SCell5, and SCell1 and SCell6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

wideband CQISCell1 – wideband CQISCell6 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-10: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 7 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell), 63 for SCell 5 (shift of 25 ms relative to Pcell), 68 for SCell 5 (shift of 30 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-11: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1x20(FDD)+ 6x20(TDD)MHz |
| 2 | 2x20(FDD)+5x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

## 9.7	CSI reporting (Single receiver antenna)

The number of receiver antennas NRX assumed for the minimum performance requirement in this clause is 1.

### 9.7.1	CQI reporting definition under AWGN conditions

#### 9.7.1.1	FDD and half-duplex FDD

The following requirements apply to UE DL Category 0. For the parameters specified in Table 9.7.1.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.16 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.7.1.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 41 |  |  |  |
| Note 1: Reference measurement channel RC.16 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/OP.2 FDD as described in Annex A.5.1.1/A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.7.1.2	TDD

The following requirements apply to UE DL Category 0. For the parameters specified in Table 9.7.1.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.16 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.7.1.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.16 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/OP.2 TDD as described in Annex A.5.2.1/A.5.2.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

#### 9.7.1.3 FDD (Category 1bis UE)

The following requirements apply to UE DL Category 1bis. For the parameters specified in Table 9.7.1.3-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.4 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.7.1.3-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.4 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.7.1.4 TDD (Category 1bis UE)

The following requirements apply to UE DL Category 1bis. For the parameters specified in Table 9.7.1.4-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.4 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.7.1.4-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |  |  |
| SNR (Note 2) |  | dB | 0 | 1 | 6 | 7 |
|  |  | dB[mW/15kHz] | -98 | -97 | -92 | -91 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.4 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/OP.2 TDD as described in Annex A.5.2.1/A.5.2.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

### 9.7.2	CQI reporting under fading conditions

#### 9.7.2.1	FDD and half-duplex FDD

For the parameters specified in Table 9.7.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.7.2.1-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD and in each available downlink transmission instance for half-duplex FDD.

Table 9.7.2.1-1 Sub-band test for single antenna transmission (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 1 |  |  |  |
| Reporting interval |  | ms | 8 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.16 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.7.2.1-2 Minimum requirement (FDD and half-duplex FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| UE DL Category | 0 | 0 |

#### 9.7.2.2	TDD

For the parameters specified in Table 9.7.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.7.2.2-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 9.7.2.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.16 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.7.2.2-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.1 | 1.1 |
| UE DL Category | 0 | 0 |

#### 9.7.2.3 FDD (Category 1bis UE)

The following requirements apply to UE DL Category 1bis. For the parameters specified in Table 9.7.2.3-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.7.2.3-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI.

Table 9.7.2.3-1 Sub-band test for single antenna transmission (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.7.2.3-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.15 | 1.15 |
| UE DL Category | 1bis | 1bis |

#### 9.7.2.4 TDD (Category 1bis UE)

The following requirements apply to UE DL Category 1bis. For the parameters specified in Table 9.7.2.4-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.7.2.4-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 9.7.2.4-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 1 (port 0) |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| SNR (Note 3) |  | dB | 8 | 9 | 13 | 14 |
|  |  | dB[mW/15kHz] | -90 | -89 | -85 | -84 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |  |  |
| Antenna configuration |  |  | 1 x 1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| Reporting mode |  |  | PUSCH 3-0 |  |  |  |
| Sub-band size |  | RB | 6 (full size) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.3 TDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.7.2.4-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  [%] | 55 | 55 |
|  | 1.15 | 1.15 |
| UE DL Category | 1bis | 1bis |

## 9.8	CSI reporting (UE supporting coverage enhancement)

The requirements in this sub-clause are defined based on the simulation results with UE DL Category M1 unless otherwise stated.

The requirements of UE DL Category M1 in 9.8.1 and 9.8.2 are applicable for UE DL Category M2 and UE DL Category 0.

The requirements of UE DL Category M1 in 9.8.3 and 9.8.4 are applicable for UE DL Category M2.

The requirements in 9.8.5 are defined based on the simulation results with Non-BL UEs, and they are applicable for non-BL UEs supporting coverage enhancement.

### 9.8.1	CQI reporting definition under AWGN conditions

#### 9.8.1.1	FDD and half-duplex FDD

The following requirements apply to UE supporting coverage enhancement. For the parameters specified in Table 9.8.1.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.23 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.1.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| CQI delay |  | ms | 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| Note 1: Reference measurement channel RC.23 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD and two sided dynamic OCNG pattern OP.2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |

#### 9.8.1.2	TDD

The following requirements apply to UE supporting coverage enhancement. For the parameters specified in Table 9.7.3.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.23 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.1.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 4 | 5 |
|  |  | dB[mW/15kHz] | -94 | -93 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolLC) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| Note 1: Reference measurement channel RC.23 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and two sided dynamic OCNG pattern OP.2 TDD as described in Annex A.5.2.1 and A.5.2.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: DC subcarrier puncturing shall be considered. |  |  |  |  |

### 9.8.2	UE-selected subband CQI

#### 9.8.2.1	FDD and half-duplex FDD

The following requirements apply to UE supporting coverage enhancement. For the parameters specified in Table 9.8.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.8.2.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting on the best narrowband reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected narrowband within the set of narrowbands in which MPDCCH is monitored shall be ≥ ;

The requirements only apply for narrowbands of full size and the random scheduling across the narrowbands is done by selecting a new narrowband in each TTI for FDD and half-duplex FDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the narrowband size.

Table 9.8.2.1-1 Sub-band test for single antenna transmission (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 1 (port 0) |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| SNR (Note 3) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Reporting interval |  | ms | 10 |  |
| CQI delays for each narrwoband |  | ms | 11, 12, 13, 14 |  |
| Reporting mode |  |  | PUSCH 2-0 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Number of preferred subbands (M) |  |  | 1 |  |
| Number of narrowbands |  |  | 4 |  |
| Frequency hopping |  |  | Enabled |  |
| Frequency hopping offset |  |  | 1 |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| Maximum number of MPDCCH repetitions (mpdcch-NumRepetition) |  |  | 4 |  |
| MPDCCH repetition level (Note 6) |  |  | 1 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 7 (Note 5) |  |
| MPDCCH hopping interval (interval-FDD) |  |  | 1 |  |
| Start subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 2.5 |  |
| Beamforming Precoder for MPDCCH |  |  | No preconding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| RCSI (csi-NumRepetitionCE) |  | Subframe | 1 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported narrowband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.25 FDD according to Table A.4-1 with one sided and dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level Note 4: DC subcarrier puncturing shall be considered Note 5: For RRC parameter setting, mpdcch-narroband is set to 8. Note 6: DCI format 6-1A is scheduled in the subframe k2 according to TS36.213 subclause 9.1.5. |  |  |  |  |

Table 9.8.2.1-2 Minimum requirement (FDD and half-duplex FDD)

|  | Test 1 |
| --- | --- |
|  | 1.3 |
| UE DL Category | M1, ≥0 |

#### 9.8.2.2	TDD

The following requirements apply to UE supporting coverage enhancement. For the parameters specified in Table 9.8.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.8.2.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting on the best narrowband reported by the UE the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected narrowband within the set of narrowbands in which MPDCCH is monitored shall be ≥ ;

The requirements only apply for subbands of full size and the random scheduling across the subbands is done by selecting a new narrowband in each available downlink transmission instance for TDD. The transport block size TBS (wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 of TS 36.213 [6] that corresponds to the narrowband size.

Table 9.8.2.2-1 Sub-band test for single antenna transmission (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 1 (port 0) |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Uplink downlink configuration |  |  | 2 |  |
| Special subframe configuration |  |  | 4 |  |
| SNR (Note 3) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  |  | Clause B.2.4 with s, a = 1, Hz |  |
| Reporting interval |  | ms | 20 |  |
| CQI delay |  | ms | 23, 24, 25, 28 |  |
| Reporting mode |  |  | PUSCH 2-0 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Number of preferred subbands (M) |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| Number of narrowbands |  |  | 4 |  |
| Frequency hopping |  |  | Enabled |  |
| Frequency hopping offset |  |  | 1 |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| Maximum number of MPDCCH repetitions (mpdcch-NumRepetition) |  |  | 4 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 7 (Note 5) |  |
| MPDCCH hopping interval (interval-TDD) |  |  | 1 |  |
| Start subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 5 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| RCSI (csi-NumRepetitionCE) |  | Subframe | 1 |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.25 TDD according to Table A.4-1 with one sided and dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: DC subcarrier puncturing shall be considered Note 5: For RRC parameter setting, mpdcch-narroband is set to 8. Note 6: DCI format 6-1A is scheduled in the subframe k2 according to TS36.213 subclause 9.1.5. Note 7: In the case CQI reports and HARQ-ACK collide, CQI reports and HARQ-ACK will be multiplexed. |  |  |  |  |

Table 9.8.2.2-2 Minimum requirement (TDD)

|  | Test 1 |
| --- | --- |
|  | 1.3 |
| UE Category | M1, ≥0 |

### 9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN

#### 9.8.3.1	FDD and half-duplex FDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-PDSCH-64QAM-r15. For the parameters specified in Table 9.8.3.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.31 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.3.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 11 | 12 |
|  |  | dB[mW/15kHz] | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| CQI delay |  | ms | 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| Note 1: Reference measurement channel RC.31 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD and two sided dynamic OCNG pattern OP.2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |

#### 9.8.3.2	TDD

The following requirements apply to UE supporting supporting ce-ModeA-r13 and ce-PDSCH-64QAM-r15. For the parameters specified in Table 9.8.3.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.31 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.3.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 11 | 12 |
|  |  | dB[mW/15kHz] | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolLC) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| Note 1: Reference measurement channel RC.31 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and two sided dynamic OCNG pattern OP.2 TDD as described in Annex A.5.2.1 and A.5.2.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: DC subcarrier puncturing shall be considered. |  |  |  |  |

### 9.8.4	CQI reporting definition for UE supporting alternative table under AWGN

#### 9.8.4.1	FDD and half-duplex FDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-CQI-AlternativeTable-r15. For the parameters specified in Table 9.8.4.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.32 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.4.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| CQI delay |  | ms | 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| Note 1: Reference measurement channel RC.32 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD and two sided dynamic OCNG pattern OP.2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |

#### 9.8.4.2	TDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-CQI-AlternativeTable-r15. For the parameters specified in Table 9.8.4.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.32 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.4.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolLC) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| Note 1: Reference measurement channel RC.32 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and two sided dynamic OCNG pattern OP.2 TDD as described in Annex A.5.2.1 and A.5.2.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: DC subcarrier puncturing shall be considered. |  |  |  |  |

### 9.8.5	PMI reporting with PUCCH 1-1 (CSI Reference Symbol)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements are specified in terms of the ratio

γ=tuetrnd

In the definition of γ, tue is 70% of the maximum throughput obtained at SNRfollow using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRfollow with random precoding.

#### 9.8.5.1	FDD

For the parameters specified in Table 9.8.5.1-1 and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.8.5.1-2.

Table 9.8.5.1-1 PMI test for single-layer (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 6 |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 |
|  |  | dB | -3 |
|  | δ | dB | 0 |
| NOC(j) |  | dB[mW/15kHz] | -98 |
| Correlation and antenna configuration |  |  | High XP 8 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |
| CSI-RS reference signal configuration |  |  | 0 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| PUCCH Report Type for RI/ first PMI |  |  | 2a |
| cqi-pmi-ConfigurationIndex |  |  | 4 |
| Measurement channel |  |  | R.108 FDD |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ce-csi-rs-feedback-config |  |  | Configured |
| Frequency hopping |  |  | Disabled |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |
| PDSCH repetition level |  |  | 1 |
| MPDCCH repetition level |  |  | 1 |
| Beamforming Precoder for MPDCCH |  |  | No precoding |
| Precoder update granularity for MPDCCH |  |  | N/A |
| BL/CE DL subframe configuration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDSCH _RA = 0dB, PDSCH_RB = 0dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4 |  |  |  |

Table 9.8.5.1-2 Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
| g | 1.2 |
| UE Category | ≥1 |

#### 9.8.5.2	TDD

For the parameters specified in Table 9.8.5.2-1 and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.8.5.2-2.

Table 9.8.5.2-1 PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 6 |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | 0 |
| NOC(j) |  | dB[mW/15kHz] | -98 |
| Correlation and antenna configuration |  |  | High XP 8 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/4 |
| CSI-RS reference signal configuration |  |  | 6 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 FFFF |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 15 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| PUCCH Report Type for RI/ first PMI |  |  | 2a |
| cqi-pmi-ConfigurationIndex |  |  | 4 |
| Measurement channel |  |  | R.108 TDD |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| ce-csi-rs-feedback-config |  |  | Configured |
| Frequency hopping |  |  | Disabled |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |
| PDSCH repetition level |  |  | 1 |
| MPDCCH repetition level |  |  | 1 |
| Beamforming Precoder for MPDCCH |  |  | No precoding |
| Precoder update granularity for MPDCCH |  |  | N/A |
| BL/CE DL subframe configuration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 5: Randomization of the principle beam direction shall be used as specified in B.2.3A.4. |  |  |  |

Table 9.8.5.2-2 Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
| g | 1.2 |
| UE Category | ≥1 |

## 9.9	CSI reporting for 4Rx UE

### 9.9.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 36.213 [6]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)

##### 9.9.1.1.1	FDD

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.9.1.1.1-1, using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1 FDD / RC.4 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.9.1.1.1-1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 4) |  |  |  |
| SNR (Note 2) |  | dB | -2 | -1 | 4 | 5 |
|  |  | dB[mW/15kHz] | -100 | -99 | -94 | -93 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.1 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4 FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

##### 9.9.1.1.2	TDD

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.9.1.1.2-1, using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.1/RC.4 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.9.1.1.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 4) |  |  |  |
| SNR (Note 2) |  | dB | -2 | -1 | 4 | 5 |
|  |  | dB[mW/15kHz] | -100 | -99 | -94 | -93 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.1 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1, except for category 1 UE use RC.4 TDD with two sided dynamic OCNG Pattern OP.2 TDD as described in Annex A.5.2.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. |  |  |  |  |  |  |

#### 9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

##### 9.9.1.2.1	FDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.9.1.2.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.2.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| Note 1: Reference measurement channel RC.7 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. |  |  |  |  |  |  |

##### 9.9.1.2.2	TDD

The following requirements apply to UE Category ≥2. For the parameters specified in table 9.9.1.2.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.2.2-1: PUCCH 1-1 submode 1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,22 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (8 x 4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0020 0000 0000 0001 0000 |  |  |  |
| SNR (Note 2) |  | dB | 2 | 3 | 8 | 9 |
|  |  | dB[mW/15kHz] | -96 | -95 | -90 | -89 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.7 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

#### 9.9.1.3		Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

##### 9.9.1.3.1	FDD

The following requirements apply to UE Category ≥5. For the parameters specified in table 9.9.1.3.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.3.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -6 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 4) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0002 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigIndex |  |  | 1 (Note 3) |  |  |  |
| Note 1: Reference measurement channel RC.21 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: It is intended to have UL collisions between RI reports and HARQ-ACK, since the RI reports shall not be used by the eNB in this test. |  |  |  |  |  |  |

##### 9.9.1.3.2	TDD

The following requirements apply to UE Category ≥5. For the parameters specified in table 9.9.1.3.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.3.2-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -6 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4x4) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0002 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type |  |  | 2 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.21 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

#### 9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

##### 9.9.1.4.1	FDD

The following requirements apply to UE Category ≥5. For the parameters specified in table 9.9.1.4.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.4.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0020 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| Note 1: Reference measurement channel RC.22 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. |  |  |  |  |  |  |

##### 9.9.1.4.2	TDD

The following requirements apply to UE Category ≥5. For the parameters specified in table 9.9.1.4.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.4.2-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0020 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/ PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.22 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

### 9.9.2	CQI reporting definition under fading conditions

#### 9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A

The purpose of the test is to verify that the reporting of the channel quality is based on the receiver of the enhanced Type A. Performance requirements are specified in terms of the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and a requirement on the minimum BLER of the transmitted transport formats indicated by the reported CQI subject to an interference model.

##### 9.9.2.1.1	FDD

For the parameters specified in Table 9.9.2.1.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.9.2.1.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.9.2.1.1-1 Fading test for single antenna (FDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -4 | N/A |
|  | dB[mW/15kHz] | -98 | N/A |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (1 x 4) | (1 x 4) |
| DIP (Note 4) | dB | N/A | -0.41 |
| Reference measurement channel |  | Note 2 | R.2 FDD |
| Reporting mode |  | PUCCH 1-0 | N/A |
| Reporting periodicity | ms | Npd = 2 | N/A |
| CQI delay | ms | 8 | N/A |
| Physical channel for CQI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type |  | 4 | N/A |
| cqi-pmi-ConfigurationIndex |  | 1 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.1 FDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and RC.4 FDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, #3, #7 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5, #7, #1 and #3. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. The number of the CRS ports in both cells is the same. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |

Table 9.9.2.1.1-2 Minimum requirement (FDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥1 |

##### 9.9.2.1.2	TDD

For the parameters specified in Table 9.9.2.1.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.9.2.1.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.9.2.1.2-1 Fading test for single antenna (TDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Uplink downlink configuration |  | 2 |  |
| Special subframe configuration |  | 4 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -4 | N/A |
|  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (1 x 4) | (1 x 4) |
| DIP (Note 4) | dB | N/A | -0.41 |
| Reference measurement channel |  | Note 2 | R.2A TDD |
| Reporting mode |  | PUCCH 1-0 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 10 or 11 | N/A |
| Physical channel for CQI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type |  | 4 | N/A |
| cqi-pmi-ConfigurationIndex |  | 3 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| ACK/NACK feedback mode |  | Multiplexing | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.1 TDD according to Table A.4-1 for Category 2-8 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and RC.4 TDD according to Table A.4-1 for Category 1 with one/two sided dynamic OCNG Pattern OP.1/2 TDD as described in Annex A.5.2.1/2. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. The number of the CRS ports in both cells is the same. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. |  |  |  |

Table 9.3.5.1.2-2 Minimum requirement (TDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥1 |

#### 9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A

The purpose of the test is to verify that the reporting of the channel quality is based on the receiver of the enhanced Type A. Performance requirements are specified in terms of the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and a requirement on the minimum BLER of the transmitted transport formats indicated by the reported CQI subject to an interference model.

##### 9.9.2.2.1	FDD

For the parameters specified in Table 9.9.2.2.1-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in Table 9.9.2.2.1-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.9.2.2.1-1 Fading test for single antenna (FDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 9 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | -4 | N/A |
|  | dB[mW/15kHz] | -98 | N/A |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (2 x 4) | (1 x 4) |
| Beamforming Model |  | As specified in Section B.4.3 (Note 9, 10) | N/A |
| DIP (Note 4) | dB | N/A | -0.41 |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna port 0 |
| CSI reference signals |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset |  | 5/1 | N/A |
| CSI-RS reference signal configuration |  | 2 | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | Subframes / bitmap | N/A | 1 / 0010000000000000 |
| CodeBookSubsetRestriction bitmap |  | 001111 | N/A |
| Reference measurement channel |  | Note 2 | R.2 FDD |
| Reporting mode |  | PUCCH 1-1 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 8 | N/A |
| Physical channel for CQI/PMI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type for CQI/PMI |  | 2 | N/A |
| PUCCH channel for RI reporting |  | PUCCH Format 2 | N/A |
| PUCCH Report Type for RI |  | 3 | N/A |
| cqi-pmi-ConfigurationIndex |  | 2 | N/A |
| ri-ConfigIndex |  | 1 | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#0 and #5. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 9: The precoder in clause B.4.3 follows UE recommended PMI. Note 10: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.9.2.2.1-2 Minimum requirement (FDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥2 |

##### 9.9.2.2.2	TDD

For the parameters specified in Table 9.9.2.2.2-1, and using the downlink physical channels specified in Annex C, the minimum requirements are specified in 9.9.2.2.2-2 and by the following

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

Table 9.9.2.2.2-1 Fading test for single antenna (TDD)

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 9 |  |
| Uplink downlink configuration |  | 2 |  |
| Special subframe configuration |  | 4 |  |
| Cyclic Prefix |  | Normal | Normal |
| Cell ID |  | 0 | 1 |
| SINR (Note 8) | dB | [-4] | N/A |
|  | dB[mW/15kHz] | -98 | -98 |
| Propagation channel |  | EPA5 | Static (Note 7) |
| Correlation and antenna configuration |  | Low (2 x 4) | (1 x 4) |
| Beamforming Model |  | As specified in Section B.4.3 (Note 10, 11) | N/A |
| DIP (Note 4) | dB | N/A | -0.41 |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna port 0 |
| CSI reference signals |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset |  | 5/3 | N/A |
| CSI-RS reference signal configuration |  | 2 | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap | Subframes / bitmap | N/A | 3 / 0010000000000000 |
| CodeBookSubsetRestriction bitmap |  | 001111 | N/A |
| Reference measurement channel |  | Note 2 | R.2A TDD |
| Reporting mode |  | PUCCH 1-1 | N/A |
| Reporting periodicity | ms | Npd = 5 | N/A |
| CQI delay | ms | 10 | N/A |
| Physical channel for CQI/PMI reporting |  | PUSCH (Note 3) | N/A |
| PUCCH Report Type for CQI/PMI |  | 2 | N/A |
| Physical channel for RI reporting |  | PUCCH Format 2 | N/A |
| PUCCH Report Type for RI |  | 3 | N/A |
| cqi-pmi-ConfigurationIndex |  | 3 | N/A |
| ri-ConfigIndex |  | 805 (Note 9) | N/A |
| Max number of HARQ transmissions |  | 1 | N/A |
| ACK/NACK feedback mode |  | Multiplexing | N/A |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink SF not later than SF#(n-4), this reported wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.11 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#2 and #7. Note 4: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 5: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Intefering cell is fully loaded. Note 6: Both cells are time-synchronous. Note 7: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 8: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 9: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification and the reported CQI in subframe SF#7 of the previous frame is applied in downlink subframes until a new CQI (after CQI/PMI dropping) is available. Note 10: The precoder in clause B.4.3 follows UE recommended PMI. Note 11: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |

Table 9.9.2.2.2-2 Minimum requirement (TDD)

|  | 1.8 |
| --- | --- |
| UE Category | ≥2 |

### 9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 9 with 8 TX are specified in terms of the ratio

In the definition of γ, for PUSCH 3-1 single PMI is 70% of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding .

#### 9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

##### 9.9.3.1.1	TDD

For the parameters specified in Table 9.9.3.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.9.3.1.1-2.

Table 9.9.3.1.1-1: PMI test for single-layer (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EVA5 |
| Precoding granularity |  | PRB | 50 |
| Antenna configuration |  |  | 8 x 4 |
| Correlation modeling |  |  | High, Cross polarized |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 0 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -6 |
|  |  | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUSCH 3-1 |
| Reporting interval |  | ms | 5 |
| PMI delay (Note 2) |  | ms | 10 |
| Measurement channel |  |  | R.45-2 TDD |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,0,1,2} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 and #9 to allow aperiodic CQI/PMI/RI to be transmitted on uplink SF#3 and #8. Note 4: Randomization of the principle beam direction shall be used as specified in B.2.3A.4 |  |  |  |

Table 9.9.3.1.1-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |
| UE Category | ≥2 |

### 9.9.4		Reporting of Rank Indicator (RI)

The purpose of this test for 4Rx UEs is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI (CQI) reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission. Transmission mode 4 is used with the specified CodebookSubSetRestriction in section 9.9.4.1, transmission mode 9 is used with the specified CodebookSubSetRestriction in section 9.9.4.2.

For the fixed rank 1 transmission with 2 Tx ports the RI and PMI reporting is restricted to two single-layer precoders, For fixed rank 2 transmission with 2 Tx ports  the RI and PMI reporting is restricted to one two-layer precoder.  For the follow RI transmission for rank 1 and 2 and 2 Tx ports the RI and PMI reporting is restricted to select the union of these precoders.

For the fixed rank 2 transmission with 4 Tx ports the RI and PMI reporting is restricted to any 2 Layer precoder, for the follow RI transmission the RI and PMI reporting is not restricted at all.

Channels with low and high correlation are used to ensure that RI reporting reflects the channel condition.

#### 9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)

##### 9.9.4.1.1	FDD

The minimum performance requirement in Table 9.9.4.1.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.9.4.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.9.4.1.1-2.

Table 9.9.4.1.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  | -6 |
|  |  | dB | -3 |  |  | -6 |
|  |  | dB | 0 |  |  | 3 |
| Propagation condition and antenna configuration |  |  | 2 x 4 EPA5 |  |  | 4 x 4 EPA5 |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  | Antenna ports 0-3 |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  | Note 6 |
| Antenna correlation |  |  | Low | Low | High | Low |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI | Fixed RI=2 and follow RI |
| SNR |  | dB | -4 | 16 | 16 | 25 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -102 | -82 | -82 | -73 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |  |
| Reporting mode |  |  | PUCCH 1-1 (Note 4) |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd= 5 |  |  |  |
| PMI and CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigurationInd |  |  | 1 (Note 5) |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.2 FDD / RC.21 FDD respectively for Test 1-3 / 4 according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between RI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic RI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. Note 4: The bit field for precoding information in DCI format 2 shall be mapped as: ● For reported RI = 1 and PMI = 0 >> precoding information bit field index = 1 ● For reported RI = 1 and PMI = 1 >> precoding information bit field index = 2 ● For reported RI = 2 and PMI = 0 >> precoding information bit field index = 0 Note 5: To avoid the ambiguity of TE behaviour when applying CQI and PMI during rank switching, RI reports are to be applied at the TE with one subframe delay in addition to Note 1 to align with CQI and PMI reports. Note 6: The following precoders are allowed in Test 4: “0x0000 0000 FFFF 0000” for RI=2 “0xFFFF FFFF FFFF FFFF” for UE reported RI |  |  |  |  |  |  |

Table 9.9.4.1.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 1 | N/A | N/A | 1.1 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥5 |

##### 9.9.4.1.2	TDD

The minimum performance requirement in Table 9.9.4.1.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.9.4.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.9.4.1.2-2.

Table 9.9.4.1.2-1: RI Test (TDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  | -6 |
|  |  | dB | -3 |  |  | -6 |
|  |  | dB | 0 |  |  | 3 |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 4 EPA5 |  |  | 4 x 4 EPA5 |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  | Antenna ports 0-3 |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  | Note 4 |
| Antenna correlation |  |  | Low | Low | High | Low |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI= 1 and follow RI | Fixed RI=1 and follow RI | Fixed RI=2 and follow RI |
| SNR |  | dB | -4 | 16 | 16 | 25 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -102 | -82 | -82 | -73 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |  |
| Reporting mode |  |  | PUSCH 3-1 (Note 3) |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| PMI and CQI delay |  | ms | 10 or 11 |  |  |  |
| ACK/NACK feedback mode |  |  | Bundling |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.2 TDD / RC.21 TDD respectively for Test 1-3 / 4 according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: Reported wideband CQI and PMI are used and sub-band CQI is discarded. Note 4: The following precoders are allowed in Test 4: “0x0000 0000 FFFF 0000” for RI=2 “0xFFFF FFFF FFFF FFFF” for UE reported RI |  |  |  |  |  |  |

Table 9.9.4.1.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 1 | N/A | N/A | 1.1 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥5 |

#### 9.9.4.2	Minimum requirement (CSI Reference Symbols)

##### 9.9.4.2.1	FDD

The minimum performance requirement in Table 9.9.4.2.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.9.4.2.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.9.4.2.1-2.

Table 9.9.4.2.1-1: RI Test (FDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  | Pc | dB | 0 |  |  | -3 |
|  |  | dB | 0 |  |  | -3 |
| Propagation condition and antenna configuration |  |  | 2 x 4 EPA5 |  |  | 4 x 4 EPA5 |
| Cell-specific reference signals |  |  | Antenna ports 0 |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  | Antenna ports 15-18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 6 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  | Note 5 |
| Antenna correlation |  |  | Low | Low | High | Low |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI | Fixed RI=2 and follow RI |
| SNR |  | dB | -4 | 16 | 16 | 25 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -102 | -82 | -82 | -73 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |  |
| Reporting mode |  |  | PUCCH 1-1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| PMI and CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigurationInd |  |  | 1 (Note 4) |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.9 FDD / RC.9B FDD respectively for Test 1-3 / 4 according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: To avoid collisions between CQI/ PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/ PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. Note 4: To avoid the ambiguity of TE behaviour when applying CQI and PMI during rank switching, RI reports are to be applied at the TE with one subframe delay in addition to Note 1 to align with CQI and PMI reports. Note 5: The following precoders are allowed in Test 4: “0x0000 0000 FFFF 0000” for RI=2 “0xFFFF FFFF FFFF FFFF” for UE reported RI |  |  |  |  |  |  |

Table 9.9.4.2.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 1 | N/A | N/A | 1.1 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥5 |

##### 9.9.4.2.2	TDD

The minimum performance requirement in Table 9.9.4.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 9.9.4.2.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.9.4.2.2-2.

Table 9.9.4.2.2-1: RI Test (TDD)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  | Pc | dB | 0 |  |  | -3 |
|  |  | dB | 0 |  |  | -3 |
| Uplink downlink configuration |  |  | 1 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 4 EPA5 |  |  | 4 x 4 EPA5 |
| Cell-specific reference signals |  |  | Antenna ports 0 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  | Antenna ports 15-18 |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CSI reference signal configuration |  |  | 4 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/4 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |  |  | Note 4 |
| Antenna correlation |  |  | Low | Low | High | Low |
| RI configuration |  |  | Fixed RI=2 and follow RI | Fixed RI=1 and follow RI | Fixed RI=1 and follow RI | Fixed RI=2 and follow RI |
| SNR |  | dB | -4 | 16 | 16 | 25 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 | -98 |
|  |  | dB[mW/15kHz] | -102 | -82 | -82 | -73 |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |  |
| Reporting mode |  |  | PUCCH 1-1 |  |  |  |
| Physical channel for CQI/ PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH report type for CQI/ PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| PMI and CQI delay |  | ms | 10 |  |  |  |
| ACK/NACK feedback mode |  |  | Bundling |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 4 |  |  |  |
| ri-ConfigurationInd |  |  | 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI and CQI estimation at a downlink subframe not later than SF#(n-4), this reported PMI and wideband CQI cannot be applied at the eNB downlink before SF#(n+4). Note 2: Reference measurement channel RC.9 TDD / RC.9B TDD respectively for Test 1-3 / 4 according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#3 and #8. Note 4: The following precoders are allowed in Test 4: “0x0000 0000 FFFF 0000” for RI=2 “0xFFFF FFFF FFFF FFFF” for UE reported RI |  |  |  |  |  |  |

Table 9.9.4.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 1 | N/A | N/A | 1.1 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥5 |

## 9.10	Reporting of CSI-RS Resource Indicator (CRI)

The purpose of this test is to verify that the reported CSI-RS Resource Indicator is accurate. The accuracy of CRI reporting for Tests in Section 9.10.1 is determined by:

a)	The ratio of the throughput obtained when transmitting based on the reported CRI and fixed precoder with multiple CSI-RS resources configured compared to that obtained when transmitting based on the fixed precoder with one CSI-RS resource configured:

-	 is [70%] of the maximum throughput obtained at  using fixed precoder and power scaling factor according to UE reported CRI value with multiple CSI-RS resources configured

-	is throughput obtained at  using fixed precoder  and power scaling factor according to the one configured CSI-RS resource

-	 is specified based on CRS RE power

b)	Each candidate CRI value among 0, 1,…, K-1 shall be reported at least  % of the time at  with multiple CSI-RS resources configured

-	The number of configured CSI-RS resources K is specific to a test.

The accuracy of CRI reporting for Tests in Section 9.10.2 is determined by:

a)	The ratio of the throughput obtained when transmitting based on the reported CRI and fixed precoder with multiple CSI-RS resources configured compared to that obtained when transmitting based on the fixed precoder with one CSI-RS resource configured:

-	 is [70%] of the maximum throughput obtained at  using the CRI configured according to the CSI UE report

-	is throughput obtained at  under assumption of single TP1 transmission

b)	CRI 2 value among 0, 1 and 2 shall be reported at least  % of the time at  with 2 CSI-RS resources configured

c)	CRI 0 value among 0, 1 and 2 shall be reported at least β % of the time at  with 2 CSI-RS resources configured

-	 corresponds to [70%] of the maximum throughput under assumption of single TP1 transmission

### 9.10.1	Minimum requirement (PUSCH 3-1)

#### 9.10.1.1	FDD

For the parameters specified in Table 9.10.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.10.1.1-2.

Table 9.10.1.1-1: CRI Test (FDD)

| Parameter |  | Unit | Test 1-1 (K,N)=(2,8) | Test 1-2 (K,N)=(2,16) | Test 1-3 (K,N)=(4,32) | Test 1-4 (K,N)=(8,64) |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Beamforming model |  |  | Annex B.4.6 |  |  |  |
| Precoding granularity |  | PRB | 50 |  |  |  |
| Correlation and antenna configuration (Note 1) |  |  | 4x2 XP High | 8x2 XP High | 8x2 XP High | 8x2 XP High |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| eMIMO-Type |  |  | Class B |  |  |  |
| Number of NZP-CSI resources (K) (Note 3) |  |  | 2 for following CRI 1 for fixed CRI | 2 for following CRI 1 for fixed CRI | 4 for following CRI 1 for fixed CRI | 8 for following CRI 1 for fixed CRI |
| NZP-CSI-RS-ID-List |  |  | {0,1} | {0,1} | {0,1,2,3 } | {0,1,2,3,4,5,6,7} |
| legacyCSRList |  |  | {0,0} | {0,0} | {0,0,0,0} | {0,0,0,0,0,0,0,0} |
| CSI reference signal configuration List |  |  | {0,1} | {0,1} | {0,1,2,3 } | {0,1,2,3,0,1,2,3} |
| Number of CSI-RS ports (Nk) |  |  | {4,4} | {8,8} | {8,8,8,8} | {8,8,8,8,8,8,8,8} |
| CSI-RS-SubframeConfig List |  |  | {1,1} | {1,1} | {1,1,1,1} | {1,1,1,1,2,2,2,2} |
| CodeBookSubsetRestriction with ID=0 |  |  | 0x 0000 00000000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | FALSE | N/A | N/A | N/A |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 | 0 | 0 | 0 |
|  | Pc | dB | -3 | -6 | -6 | -6 |
|  |  | dB | -3 | -3 | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 |  |  |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CRI Delay |  | ms | 8 |  |  |  |
| PMI delay |  | ms | 8 |  |  |  |
| Measurement channel |  |  | R.50A-1 FDD | R.50A-2 FDD | R.50A-2 FDD | R.50A-3 FDD |
| OCNG Pattern |  |  | OP.1 FDD |  |  |  |
| Rank Number of PDSCH |  |  | 1 |  |  |  |
| Scheduled PDSCH SFs |  |  | SF 0,2,3,4,7,8,9 | SF 0,2,3,4,7,8,9 | SF 0,2,3,4,7,8,9 | SF 0,3,4,8,9 |
| Max number of HARQ transmissions |  |  | 4 |  |  |  |
| Redundancy version coding sequence |  |  | {0,1,2,3} |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subrame SF#n based on CRI/PMI estimation at a downlink SF not later than SF#(n-4), this reported CRI/PMI cannot be applied at the eNB downlink before SF#(n+4). Note 2: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 3: When one CSI-RS resource configured, the configurations according to NZP-CSI-RS-ID = 0 are configured. |  |  |  |  |  |  |

Table 9.10.1.1-2: Minimum requirement (FDD)

|  | Test 1-1 | Test 1-2 | Test 1-3 | Test 1-4 |
| --- | --- | --- | --- | --- |
|  | 1.2 | 1.2 | 1.3 | 1.35 |
|  | 40 | 40 | 20 | 10 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥2 |
| Note1: According to UE capability configuration list for the maximum number of NZP CSI-RS resource Kmax and the maximum number of total NZP CSI-RS ports N in each K =2,.., Kmax: if UE supports the combination of (K,N) =(8,64), then test 1-4 is applicable ; else if UE supports the combination of (K,N) =(4,32), then test 1-3 is applicable; else if UE supports the combination of (K,N) =(2,16), then test 1-2 is applicable; otherwise test 1-1 is applicable. |  |  |  |  |

#### 9.10.1.2	TDD

For the parameters specified in Table 9.10.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.10.1.2-2.

Table 9.10.1.2-1: CRI Test (TDD)

| Parameter |  | Unit | Test 1-1 (K,N)=(2,8) | Test 1-2 (K,N)=(2,16)) | Test 1-3 (K,N)=(4,32) | Test 1-4 (K,N)=(8,64) |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Beamforming model |  |  | Annex B.4.6 |  |  |  |
| Precoding granularity |  | PRB | 50 |  |  |  |
| Correlation and antenna configuration (Note 1) |  |  | 4x2 XP High | 8x2 XP High | 8x2 XP High | 8x2 XP High |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| eMIMO-Type |  |  | Class B |  |  |  |
| Number of NZP-CSI resources (K) (Note 3) |  |  | 2 for following CRI 1 for fixed CRI | 2 for following CRI 1 for fixed CRI | 4 for following CRI 1 for fixed CRI | 8 for following CRI 1 for fixed CRI |
| NZP-CSI-RS-ID-List |  |  | {0,1} | {0,1} | {0,1,2,3 } | {0,1,2,3,4,5,6,7} |
| legacyCSRList |  |  | {0,0} | {0,0} | {0,0,0,0} | {0,0,0,0,0,0,0,0} |
| CSI reference signal configuration List |  |  | {0,1} | {0,1} | {0,1,2,3 } | {0,1,2,3,0,1,2,3} |
| Number of CSI-RS ports (Nk) |  |  | {4,4} | {8,8} | {8,8,8,8} | {8,8,8,8,8,8,8,8} |
| CSI-RS-SubframeConfig List |  |  | {9,9} | {9,9} | {9,9,9,9} | {8,8,8,8,9,9,9,9} |
| CodeBookSubsetRestriction with ID=0 |  |  | 0x 0000 00000000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 | 0x 0000 0000 0000 0020 0000 0000 0001 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | FALSE | N/A | N/A | N/A |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 | 0 | 0 | 0 |
|  | - Pc | dB | -3 | -6 | -6 | -6 |
|  |  | dB | -3 | -3 | -3 | -3 |
|  |  | dB[mW/15kHz] | -98 |  |  |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Reporting interval |  | ms | 10 |  |  |  |
| CRI Delay |  | ms | 12 |  |  |  |
| PMI delay |  | ms | 12 |  |  |  |
| Measurement channel |  |  | R.44A-1 TDD | R.44A-2 TDD | R.44A-2 TDD | R.44A-3 TDD |
| OCNG Pattern |  |  | OP.1 TDD |  |  |  |
| Rank Number of PDSCH |  |  | 1 |  |  |  |
| Scheduled PDSCH SFs |  |  | SF 0,1,3,6,8,9 | SF 0,1,3,6,8,9 | SF 0,1,3,6,8,9 | SF 0,1,6,8,9 |
| Max number of HARQ transmissions |  |  | 4 |  |  |  |
| Redundancy version coding sequence |  |  | {0,1,2,3} |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 2: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver. Note 3: When one CSI-RS resource configured, the configurations according to NZP-CSI-RS-ID 0 are configured. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in downlink SF#4 to allow aperiodic CRI/CQI/PMI/RI to be transmitted on uplink SF#2. |  |  |  |  |  |  |

Table 9.10.1.2-2: Minimum requirement (TDD)

|  | Test 1-1 | Test 1-2 | Test 1-3 | Test 1-4 |
| --- | --- | --- | --- | --- |
|  | 1.2 | 1.2 | 1.3 | 1.35 |
|  | 40 | 40 | 20 | 10 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥2 |
| Note1: According to UE capability configuration list for the maximum number of NZP CSI-RS resource Kmax and the maximum number of total NZP CSI-RS ports N in each K =2,.., Kmax: if UE supports the combination of (K,N) =(8,64), then test 1-4 is applicable; else if UE supports the combination of (K,N) =(4,32), then test 1-3 is applicable; else if UE supports the combination of (K,N) =(2,16), then test 1-2 is applicable; otherwise test 1-1 is applicable. |  |  |  |  |

### 9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)

#### 9.10.2.1	FDD

The requirements are specified in Table 9.10.2.1-4, with the additional parameters in Table 9.10.2.1-1, Table 9.10.2.1-2 and Table 9.10.2.1-3. In Table 9.10.2.1-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and can transmit PDSCH, and transmission point 2 (TP 2) has different Cell ID and can transmit PDSCH. The downlink physical channel setup for TP 1 is according to Table C.3.2 and for TP 2 according to Table C.3.2.

Table 9.10.2.1-1: Test Parameters

| Parameter |  | Unit | Fixed TP1 case |  | Follow CRI case |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 | -3 | -3 |
|  |  | dB | SNR1 (Note 2) | SNR2 = SNR1 – [9] dB | SNR1 (Note 2) | SNR2 = SNR1 – [9] dB |
| at antenna port |  | dBm/15kHz | -98 |  |  |  |
| BWChannel |  | MHz | 10 |  |  |  |
| Cyclic Prefix |  |  | Normal |  |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |  |
| Number of allocated resource block |  |  | 50 |  |  |  |
| Cell ID |  |  | 0 | 126 | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 | 2 | 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| eMIMO-Type |  |  | Class B |  |  |  |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A | Antenna ports {15,16} | NA |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |  | 5 / 2 |  |
| CSI reference signal 0 configuration |  |  | 0 |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 2 |  | 5 / 2 |
| CSI reference signal 1 configuration |  |  |  | 8 |  | 8 |
| CodeBookSubsetRestriction for CSI-RS 0 |  |  | 001111 | N/A | 001111 | N/A |
| CodeBookSubsetRestriction for CSI-RS 1 |  |  | N/A | 001111 | N/A | 001111 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 2/ 0010000000000000 | N/A | 2/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 2/ 0010000000000000 | N/A | 2/ 0010000000000000 |
| Timing offset relative to TP 1 |  | us | N/A | 0 | N/A | 0 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 0 | N/A | 0 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ Table 9.10.2.1-2 |  | Type C, PQI based on UE reporting Table 9.10.2.1-3 |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CRI Delay |  | ms | 8 |  |  |  |
| PMI delay |  | ms | 8 |  |  |  |
| Measurement channel |  |  | R.97 FDD |  |  |  |
| Correlation and antenna configuration |  |  | 2x2 Low |  |  |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Note 1: Note 2: TP1 SNR based on SNR value derived for exact test metric verification and can be equal to or |  |  |  |  |  |  |

Table 9.10.2.1-2: Configurations of PQI and DL transmission hypothesis for each PQI set (Fixed TP1 case)

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blank |

Table 9.10.2.1-3: Configurations of PQI and DL transmission hypothesis for each PQI set (Follow CRI case)

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blank |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blank | PDSCH |
| PQI set 2 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 9.10.2.1-4: Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 1.2 |
|  | 40 |
|  | 60 |
| UE Category | ≥2 |

#### 9.10.2.2	TDD

The requirements are specified in Table 9.10.2.2-4, with the additional parameters in Table 9.10.2.2-1, Table 9.10.2.2-2 and Table 9.10.2.2-3. In Table 9.10.2.2-1, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals, PBCH and can transmit PDSCH, and transmission point 2 (TP 2) has different Cell ID and can transmit PDSCH. The downlink physical channel setup for TP 1 is according to Table C.3.2 and for TP 2 according to Table C.3.2.

Table 9.10.2.2-1: Test Parameters

| Parameter |  | Unit | Fixed TP1 case |  | Follow CRI case |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TP 1 | TP 2 | TP 1 | TP 2 |
| Downlink power allocation |  | dB | 0 | 0 | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 | -3 | -3 |
|  |  | dB | SNR1 (Note 2) | SNR2 = SNR1 – [9] dB | SNR1 (Note 2) | SNR2 = SNR1 – [9] dB |
| at antenna port |  | dBm/15kHz | -98 |  |  |  |
| Uplink downlink configuration |  |  | 1 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| BWChannel |  | MHz | 10 |  |  |  |
| Cyclic Prefix |  |  | Normal |  |  |  |
| PDSCH transmission mode |  |  | 10 |  |  |  |
| Number of allocated resource block |  |  | 50 |  |  |  |
| Cell ID |  |  | 0 | 126 | 0 | 126 |
| Number of control OFDM symbols / PDSCH start |  |  | 2 | 2 | 2 | 2 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| eMIMO-Type |  |  | Class B |  |  |  |
| CSI reference signals 0 |  |  | Antenna ports {15,16} | N/A | Antenna ports {15,16} | NA |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |  | 5 / 4 |  |
| CSI reference signal 0 configuration |  |  | 0 |  | 0 |  |
| CSI reference signals 1 |  |  | N/A | Antenna ports {15,16} | N/A | Antenna ports {15,16} |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes |  | 5 / 4 |  | 5 / 4 |
| CSI reference signal 1 configuration |  |  |  | 8 |  | 8 |
| CodeBookSubsetRestriction for CSI-RS 0 |  |  | 001111 | N/A | 001111 | N/A |
| CodeBookSubsetRestriction for CSI-RS 1 |  |  | N/A | 001111 | N/A | 001111 |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | 4/ 0010000000000000 | N/A | 4/ 0010000000000000 | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  | Subframes/bitmap | N/A | 4/ 0010000000000000 | N/A | 4/ 0010000000000000 |
| Timing offset relative to TP 1 |  | us | N/A | 0 | N/A | 0 |
| Frequency offset relative to TP 1 |  | Hz | N/A | 0 | N/A | 0 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  | Type B, ‘00’ Table 9.10.2.2-2 |  | Type C, PQI based on UE reporting Table 9.10.2.2-3 |  |
| Reporting mode |  |  | PUSCH 3-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CRI Delay |  | ms | 8 |  |  |  |
| PMI delay |  | ms | 8 |  |  |  |
| Measurement channel |  |  | R.97 TDD |  |  |  |
| Correlation and antenna configuration |  |  | 2x2 Low |  |  |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Note 1: Note 2: TP1 SNR based on SNR value derived for exact test metric verification and can be equal to or |  |  |  |  |  |  |

Table 9.10.2.2-2: Configurations of PQI and DL transmission hypothesis for each PQI set (Fixed TP1 case)

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blank |

Table 9.10.2.2-3: Configurations of PQI and DL transmission hypothesis for each PQI set (Follow CRI case)

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blank |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blank | PDSCH |
| PQI set 2 | CSI-RS 0 for CW1 CSI-RS 1 for CW2 | ZP CSI-RS 0 | PDSCH | PDSCH |

Table 9.10.2.2-4: Minimum requirement (FDD)

|  | Test 1 |
| --- | --- |
|  | 1.2 |
|  | 40 |
|  | 60 |
| UE Category | ≥2 |

## 9.11	Reporting of Hybrid Channel state information

### 9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)

The purpose of this test is to verify that the reported CRI(1) from eMIMO-Type and PMI(2)  from eMIMO-Type2 are accurate.

The accuracy of CRI and PMI reporting are determined by the ratio of the throughput obtained when transmitting based on UE reported CRI(1) and PMI(2)  compared to that obtained when transmitting based on the random precoding and fixed CRI:

-	is [70%] of the maximum throughput obtained at  using following precoder according to UE reported  PMI(2) for eMIMO-Type2 and power scaling factor according to UE reported CRI (1)  for eMIMO-Type

-	is throughput obtained at  using random precoder and power scaling factor according to the one configured CSI-RS resource

-	 is specified based on CRS RE power

#### 9.11.1.1	FDD

For the parameters specified in Table 9.11.1.1-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.11.1.1-2.

Table 9.11.1.1-1: CRI/PMI test for single layer  (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 50 |
| Correlation and antenna configuration |  |  | 4*2 ULA Low |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| Beamforming model |  |  | B.4.6 (Note 4) |
| CSI reference signal configuration List |  |  | {0,1} |
| Number of CSI-RS ports (Nk) |  |  | {4,4} |
| CSI-RS-SubframeConfig List |  |  | {1,1} |
| eMIMO-Type |  |  | Class B |
| Number of NZP-CSI resources (K) |  |  | 2 for following CRI |
| NZP-CSI-RS-ID-List |  |  | {0,1} |
| eMIMO-Type2 |  |  | Class B |
| NZP-CSI-RS-ID |  |  | {0} |
| alternativeCodebookEnabledCLASSB_K1 |  |  | FALSE |
| codebookSubsetRestriction |  |  | 001111 |
| Reporting mode |  |  | PUSCH 3-1 |
| Physical channel for CRI(1) reporting |  |  | PUSCH |
| Reporting interval for CRI(1) reporting |  | ms | 40 (Note 5) |
| Physical channel for RI(2)/CQI(2)/PMI(2) reporting |  |  | PUSCH |
| Reporting Interval for RI(2)/CQI(2)/PMI(2) reporting |  | ms | 5 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
| Rank Number of PDSCH |  |  | 1 |
| Measurement channel |  |  | R.50A-1 FDD |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Scheduled PDSCH SFs |  |  | SF 0,2,3,4,7,8,9 |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on CRI,PMI estimation at a downlink SF not later than SF#(n-4), this reported CRI, PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: The precoder matrix as specified in Table 6.3.4.2.3-1of [4].For following PMI, PMI equals to UE reported PMI(2) for eMIMO-Type2 “Class B”. Note 5: PDCCH DCI format 0 with a trigger for aperiodic CSI of eMIMIO-Type shall be transmitted in downlink SF#1 to allow aperiodic CRI(1) to be transmitted on uplink SF#5. PDCCH DCI format 0 with a trigger for aperiodic CSI of eMIMIO-Type2 shall be transmitted in downlink SF#1,6 to allow aperiodic RI(2)/CQI(2)/PMI(2) to be transmitted on uplink SF#5,0. |  |  |  |

Table 9.11.1.1-2: Minimum requirement (FDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |
| UE Category | ≥2 |

#### 9.11.1.2	TDD

For the parameters specified in Table 9.11.1.2-1, and using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.11.1.2-2.

Table 9.11.1.2-1: CRI/PMI test for single layer  (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity (only for reporting and following PMI) |  | PRB | 50 |
| Correlation and antenna configuration |  |  | 4*2 ULA Low |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| Beamforming model |  |  | B.4.6 (Note 4) |
| CSI reference signal configuration List |  |  | {0,1} |
| Number of CSI-RS ports (Nk) |  |  | {4,4} |
| CSI-RS-SubframeConfig List |  |  | {9,9} |
| eMIMO-Type |  |  | Class B |
| Number of NZP-CSI resources (K) |  |  | 2 for following CRI |
| NZP-CSI-RS-ID-List |  |  | {0,1} |
| eMIMO-Type2 |  |  | Class B |
| NZP-CSI-RS-ID |  |  | {0} |
| alternativeCodebookEnabledCLASSB_K1 |  |  | FALSE |
| codebookSubsetRestriction |  |  | 001111 |
| Reporting mode |  |  | PUSCH 3-1 |
| Physical channel for CRI(1) reporting |  |  | PUSCH |
| Reporting interval for CRI(1) reporting |  | ms | 40 (Note 5) |
| Physical channel for RI(2)/CQI(2)/PMI(2) reporting |  |  | PUSCH |
| Reporting Interval for RI(2)/CQI(2)/PMI(2) reporting |  | ms | 10 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB | -3 |
| Rank Number of PDSCH |  |  | 1 |
| Measurement channel |  |  | R.44A-1 TDD |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Scheduled PDSCH SFs |  |  | SF 0,1,3,6,8,9 |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on CRI,PMI estimation at a downlink SF not later than SF#(n-4), this reported CRI, PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: The precoder matrix as specified in Table 6.3.4.2.3-1of [4].For following PMI, PMI equals to UE reported PMI(2) for eMIMO-Type2 “Class B”. Note 5: PDCCH DCI format 0 with a trigger for aperiodic CSI of eMIMIO-Type shall be transmitted in downlink SF#4 to allow aperiodic CRI(1) to be transmitted on uplink SF#4. PDCCH DCI format 0 with a trigger for aperiodic CSI of eMIMIO-Type2 shall be transmitted in downlink SF#4 to allow aperiodic RI(2)/CQI(2)/PMI(2) to be transmitted on uplink SF#2. |  |  |  |

Table 9.11.2.2-2: Minimum requirement (TDD)

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |
| UE Category | ≥2 |

## 9.12	CSI reporting (UE supporting Short TTI)

The requirements in this sub-clause are valid for UEs capable of short TTI.

### 9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective fading conditions is determined by the reporting variance, and the relative increase of the throughput obtained when the transport format transmitted is that indicated by the reported CQI compared to the case for which a fixed transport format configured according to the reported median CQI is transmitted. In addition, the reporting accuracy is determined by a minimum BLER using the transport formats indicated by the reported CQI. The purpose is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for frequently non-selective scheduling. To account for sensitivity of the input SNR the CQI reporting under frequency non-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.12.1.1	FDD

For the parameters specified in Table 9.12.1.1-1, Table 9.12.1.1-2 and Table 9.12.1.1-3 using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.12.1.1-4 by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

Table 9.12.1.1-1: Fading test for slot-PDSCH (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 4 (ports 0 and 1) |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 1 | 2 | 9 | 10 |
|  |  | dB[mW/15kHz] | -97 | -96 | -89 | -88 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | slots | 8 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Slot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot #(n-4), this reported wideband CQI cannot be applied at the eNB downlink before slot #(n+4). Note 2: Reference measurement channel RC.26 FDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.1.1-2: Fading test for subslot-PDSCH (FDD)

| Parameter |  | Unit | Test 3 |  | Test 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 4 (ports 0 and 1) |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 1 | 2 | 9 | 10 |
|  |  | dB[mW/15kHz] | -97 | -96 | -89 | -88 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | subslots | 8 or 12 (Note 1) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Subslot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subslot #n based on CQI estimation at a downlink slot not later than subslot #(n-k), this reported wideband CQI cannot be applied at the eNB downlink before subslot #(n+k), where k = 4 for UE capability ProcessingTimelineSet=set1 and k=6 for UE capability ProcessingTimelineSet=set2. Note 2: Reference measurement channel RC.28 FDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.1.1-3: SPDCCH parameters (FDD)

| Parameter | Unit | Set 1 |
| --- | --- | --- |
| Refence symbol |  | CRS |
| Number of PRB per SPDCCH-PRB set |  | 16 |
| Transmission type |  | Localized |
| Number of OFDM symbols |  | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 |
| Rate Matching mode |  | Mode 1 |
| SPDCCH L1 Reuse Indication |  | Not configured |
| Aggregatoin level | SCCE | 4 |
| Note 1: PRB = {0, 1, …, 15}. |  |  |

Table 9.12.1.1-4: Minimum requirement for slot/subslot-PDSCH (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
|  [%] | 20 | 20 | 20 | 20 |
|  | 1.05 | 1.05 | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥2 |

#### 9.12.1.2	TDD

For the parameters specified in Table 9.12.1.2-1 and Table 9.12.1.2-2 using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.12.1.2-3 by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02.

Table 9.12.1.2-1: Fading test for slot-PDSCH (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 4 (ports 0 and 1) |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| SNR (Note 3) |  | dB | 1 | 2 | 9 | 10 |
|  |  | dB[mW/15kHz] | -97 | -96 | -89 | -88 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | slots | 8 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Slot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot #(n-4), this reported wideband CQI cannot be applied at the eNB downlink before slot #(n+4). Note 2: Reference measurement channel RC.26 TDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.1.2-2: SPDCCH parameters (FDD)

| Parameter | Unit | Set 1 |
| --- | --- | --- |
| Refence symbol |  | CRS |
| Number of PRB per SPDCCH-PRB set |  | 16 |
| Transmission type |  | Localized |
| Number of OFDM symbols |  | 1 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 |
| Rate Matching mode |  | Mode 1 |
| SPDCCH L1 Reuse Indication |  | Not configured |
| Aggregatoin level | SCCE | 4 |
| Note 1: PRB = {0, 1, …, 15}. |  |  |

Table 9.12.1.2-3: Minimum requirement for slot-PDSCH (FDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 |

### 9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)

#### 9.12.2.1	FDD

For the parameters specified in Table 9.12.2.1-1, Table 9.12.2.1-2, and Table 9.12.2.1-3 using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.12.2.1-4 by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02].

Table 9.12.2.1-1: Fading test for slot-PDSCH (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
| SNR (Note 3) |  | dB | 3 | 4 | 12 | 13 |
|  |  | dB[mW/15kHz] | -95 | -94 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI-RS reference signal configuration |  |  | 0 |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | slots | 8 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Slot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| Note 1: For slot-PDSCH, if the UE reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot #(n-4), this reported wideband CQI cannot be applied at the eNB downlink before slot #(n+4). Note 2: Reference measurement channel RC.27 FDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.2.1-2: Fading test for subslot-PDSCH (FDD)

| Parameter |  | Unit | Test 3 |  | Test 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
| SNR (Note 3) |  | dB | 4 | 5 | 12 | 13 |
|  |  | dB[mW/15kHz] | -94 | -93 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI-RS reference signal configuration |  |  | 0 |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | subslots | 8 or 12 (Note 1) |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Subslot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subslot #n based on CQI estimation at a downlink slot not later than subslot #(n-k), this reported wideband CQI cannot be applied at the eNB downlink before subslot #(n+k), where k = 4 for UE capability ProcessingTimelineSet=set1 and k=6 for UE capability ProcessingTimelineSet=set2. Note 2: Reference measurement channel RC.29 FDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.2.1-3: SPDCCH parameters (FDD)

| Parameter | Unit | Set 1 |
| --- | --- | --- |
| Refence symbol |  | CRS |
| Number of PRB per SPDCCH-PRB set |  | 16 |
| Transmission type |  | Localized |
| Number of OFDM symbols |  | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 |
| Rate Matching mode |  | Mode 1 |
| SPDCCH L1 Reuse Indication |  | Not configured |
| Aggregatoin level | SCCE | 8 |
| Note 1: PRB = {0, 1, …, 15}. |  |  |

Table 9.12.2.1-4: Minimum requirement for slot/subslot-PDSCH (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
|  [%] | 20 | 20 | 20 | 20 |
|  | 1.05 | 1.05 | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 | ≥2 | ≥2 |

#### 9.12.2.2	TDD

For the parameters specified in Table 9.12.2.2-1and Table 9.12.2.2-2 using the downlink physical channels specified in Annex C.3.2, the minimum requirements are specified in Table 9.12.2.2-3 by the following

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02].

Table 9.12.2.2-1: Fading test for slot-PDSCH (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |  |  |
| Transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
| SNR (Note 3) |  | dB | 3 | 4 | 12 | 13 |
|  |  | dB[mW/15kHz] | -95 | -94 | -86 | -85 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Propagation channel |  |  | EPA5 |  |  |  |
| Correlation and antenna configuration |  |  | High (2 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI-RS reference signal configuration |  |  | 0 |  |  |  |
| Reporting mode |  |  | PUSCH 1-1 |  |  |  |
| Reporting interval |  | ms | 5 |  |  |  |
| CQI delay |  | Slots | 8 |  |  |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 00 0001 |  |  |  |
| DL TTI length |  |  | Slot |  |  |  |
| Configured SPDCCH-PRB sets |  |  | {Set 1} |  |  |  |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot #(n-4), this reported wideband CQI cannot be applied at the eNB downlink before slot #(n+4). Note 2: Reference measurement channel RC.27 TDD according to Table A.4-1 for Categories 2 or higher with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.1.1. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 9.12.2.2-2: SPDCCH parameters (TDD)

| Parameter | Unit | Set 1 |
| --- | --- | --- |
| Refence symbol |  | CRS |
| Number of PRB per SPDCCH-PRB set |  | 16 |
| Transmission type |  | Localized |
| Number of OFDM symbols |  | 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 |
| Rate Matching mode |  | Mode 1 |
| SPDCCH L1 Reuse Indication |  | Not configured |
| Aggregatoin level | SCCE | 8 |
| Note 1: PRB = {0, 1, …, 15}. |  |  |

Table 9.12.2.2-3: Minimum requirement for slot-PDSCH (TDD)

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |
| UE Category | ≥2 | ≥2 |

## 9.13	CSI reporting for 8Rx UE

### 9.13.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 36.213 [6]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

#### 9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)

The minimum requirements for dual codeword transmission are defined in terms of a reporting spread of the wideband CQI value for codeword #1, and their BLER performance using the transport format indicated by the reported CQI median of codeword #0 and codeword #1. The precoding used at the transmitter is a fixed precoding matrix specified by the bitmap parameter codebookSubsetRestriction. The propagation condition assumed for the minimum performance requirement is defined in subclause B.1.

##### 9.13.1.2.1	Void

##### 9.13.1.2.2	TDD

The following requirements apply to UE Category 8 and DL Category 14, 17~20, ≥22. For the parameters specified in table 9.13.1.2.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.13.1.2.2-1: PUCCH 1-1 static test 4x8 (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -6 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…, 18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4x8 for Rank4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0001 0000 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 4 | 5 | 11 | 12 |
|  |  | dB[mW/15kHz] | -94 | -93 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.23 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |

## 9.14	CSI reporting of Narrowband IoT

The requirements in this sub-clause are defined based on the simulation results with UE DL Category NB2 unless otherwise stated.

### 9.14.1	CQI reporting definition under AWGN conditions

#### 9.14.1.1	Half-duplex FDD

The following requirements apply to UE Category NB2 capable of npdsch-16QAM-r17. For the parameters specified in Table 9.14.1.1-1, and using the downlink physical channels specified in C.3.6, the reported candidateRep value according to RC.33 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value - 1) shall be less than or equal to 0.1.

Table 9.14.1.1-1: Half-duplex FDD

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Bandwidth | kHz | 200 |  |
| Operation mode |  | Stand-alone |  |
| Carrier type |  | Non-anchor |  |
| Number of NRS ports |  | 1 |  |
| Propagation condition and antenna configuration |  | AWGN (1 x 1) |  |
| SNR (Note 2) | dB | 10 | 11 |
| Ior(j) | dB[mW/15kHz] | -88 | -87 |
| Noc(j) | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  | 1 |  |
| NPDCCH repetition number | subframe | 1 |  |
| Rmax (npdcch-NumRepetitions-r13) | subframe | 4 |  |
| G (nPDCCH-startSF-USS-r13) | subframe | 2 |  |
| Scheduling delay field (IDelay) |  | 1 |  |
| NRepAN (ack-NACK-NumRepetitions-r13) |  | 1 |  |
| ACK/NACK resource field |  | 0 |  |
| NPDCCH reference channel |  | R.NB.3 FDD |  |
| αoffset (npdcch-Offset-USS-r13) |  | 0 |  |
| CQI reporting period | ms | 40 |  |
| NPDSCH repetition number | subframe | 1 |  |
| Resource assignment field (ISF) (Note 4) |  | 3 |  |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |  |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |  |
| OCNG pattern |  | NB.OP.1 |  |
| Note 1: Reference measurement channel RC.33 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern NB.OP.1 FDD as described in Annex A.5.3.1. For the subframes the reference measurement channel is not scheduled, NRS is transmitted with OCNG pattern NB.OP.1 FDD. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: MAC CE Downlink Channel Quality Report Command is triggered every 40 subframes, where NPDCCH scheduling MAC CE (NPDSCH) is transmitted in subframe n. UL grant is transmitted in subframe n+24. Note 4: Set the number of subframes (NSF) for NPDSCH to 4 for NPDSCH transmitting MAC CE Downlink Channel Quality Report Command. |  |  |  |

#### 9.14.1.2	TDD

The following requirements apply to UE Category NB2 capable of npdsch-16QAM-r17. For the parameters specified in Table 9.14.1.2-1, and using the downlink physical channels specified in C.3.6, the reported candidateRep value according to RC.33 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value - 1) shall be less than or equal to 0.1.

Table 9.14.1.2-1: TDD

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Bandwidth | kHz | 200 |  |
| Operation mode |  | Stand-alone |  |
| Uplink downlink configuration (Note 4) |  | 4 |  |
| Special subframe configuration (Note 5) |  | 4 |  |
| Carrier type |  | Non-anchor |  |
| Number of NRS ports |  | 1 |  |
| Propagation condition and antenna configuration |  | AWGN (1 x 1) |  |
| SNR (Note 2) | dB | 10 | 11 |
| Ior(j) | dB[mW/15kHz] | -88 | -87 |
| Noc(j) | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  | 1 |  |
| NPDCCH repetition number | subframe | 1 |  |
| Rmax (npdcch-NumRepetitions-r13) | subframe | 4 |  |
| G (nPDCCH-startSF-USS-r13) | subframe | 2 |  |
| Scheduling delay field (IDelay) |  | 1 |  |
| NRepAN (ack-NACK-NumRepetitions-r13) |  | 1 |  |
| ACK/NACK resource field |  | 0 |  |
| NPDCCH reference channel |  | R.NB.3 TDD |  |
| αoffset (npdcch-Offset-USS-r13) |  | 0 |  |
| CQI reporting period | ms | 80 |  |
| NPDSCH repetition number | subframe | 1 |  |
| Resource assignment field (ISF) (Note 6) |  | 3 |  |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |  |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |  |
| OCNG pattern |  | NB.OP.1 |  |
| Note 1: Reference measurement channel RC.33 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern NB.OP.1 TDD as described in Annex A.5.3.1. For the subframes the reference measurement channel is not scheduled, NRS is transmitted with OCNG pattern NB.OP.1 TDD. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: MAC CE Downlink Channel Quality Report Command is triggered every 80 subframes, where NPDCCH scheduling MAC CE (NPDSCH) is transmitted in subframe n. UL grant is transmitted in subframe n+40. Note 4: As specified in Table 4.2-2 in TS 36.211 [4]. Note 5: As specified in Table 4.2-1 in TS 36.211 [4]. Note 6: Set the number of subframes (NSF) for NPDSCH to 4 for NPDSCH transmitting MAC CE Downlink Channel Quality Report Command. |  |  |  |

# 10	Performance requirement (MBMS)

## 10.1	FDD (Fixed Reference Channel)

The parameters specified in Table 10.1-1 are valid for all FDD tests unless otherwise stated. For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 0 dB as the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

Table 10.1-1: Common Test Parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes | Processes | None |
| Subcarrier spacing | kHz | 15 kHz |
| Allocated subframes per Radio Frame (Note 1) |  | 6 subframes |
| Number of OFDM symbols for PDCCH |  | 2 |
| Cyclic Prefix |  | Extended |
| Note1: For FDD mode, up to 6 subframes (#1/2/3/6/7/8) are available for MBMS, in line with TS 36.331. |  |  |

All tests specified in 10.1 and 10.3 the applicability rules are defined as the following. The requirements in clause 10.3 are applicable to UEs that support FeMBMS. The test case applicability is in according to Table 10.1-2 depending on set of supported UE capabilities.

Table 10.1-2: FeMBMS PMCH tests applicability

| Tests / clause | Applicable if UE indicates at least the following capability |
| --- | --- |
| 10.3.1 | FeMBMS support of fembmsMixedCell and unicast-fembmsMixedSCell with corresponding subcarrier spacing |
| 10.3.2 | FeMBMS support of fembmsMixedCell with corresponding subcarrier spacing |
| 10.3.3 | FeMBMS support of fembmsDedicatedCell with corresponding subcarrier spacing |

For FeMBMS capable UEs, if corresponding tests listed in clause 10.1 or 10.3.1 or 10.3.2 are tested, the test coverage can be considered fulfilled without executing the corresponding tests listed in clause 10.3.3.

### 10.1.1	Minimum requirement

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for MBMS.

For the parameters specified in Table 10.1-1 and Table 10.1.1-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.1.1-2.

Table 10.1.1-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: . |  |  |  |

Table 10.1.1-2: Minimum performance

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | 10 MHz | R.37 FDD | OP.4 FDD | MBSFN channel model (Table B.2.6-1) | 1x2 low | 1 | 4.1 | ≥1 |
| 2 | 10 MHz | R.38 FDD | OP.4 FDD |  |  |  | 11.0 | ≥1 |
| 3 | 10 MHz | R.39 FDD | OP.4 FDD |  |  |  | 20.1 | ≥2 |
|  | 5.0MHz | R.39-1 FDD | OP.4 FDD |  |  |  | 20.5 | 1 |
| 4 | 1.4 MHz | R.40 FDD | OP.4 FDD |  |  |  | 6.6 | ≥1 |

## 10.2	TDD (Fixed Reference Channel)

The parameters specified in Table 10.2-1 are valid for all TDD tests unless otherwise stated. For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 0 dB as the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

Table 10.2-1: Common Test Parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes | Processes | None |
| Subcarrier spacing | kHz | 15 kHz |
| Allocated subframes per Radio Frame (Note 1) |  | 5 subframes |
| Number of OFDM symbols for PDCCH |  | 2 |
| Cyclic Prefix |  | Extended |
| Note1: For TDD mode, in line with TS 36.331, Uplink-Downlink Configuration 5 is proposed, up to 5 subframes (#3/4/7/8/9) are available for MBMS. |  |  |

### 10.2.1	Minimum requirement

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for MBMS.

For the parameters specified in Table 10.2-1 and Table 10.2.1-1 and Annex A.3.8.2, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.2.1-2.

Table 10.2.1-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Note 1: . |  |  |  |

Table 10.2.1-2: Minimum performance

| Test number | Bandwidth | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | 10 MHz | R.37 TDD | OP.4 TDD | MBSFN channel model (Table B.2.6-1) | 1x2 low | 1 | 3.4 | ≥1 |
| 2 | 10 MHz | R.38 TDD | OP.4 TDD |  |  |  | 11.1 | ≥1 |
| 3a | 10 MHz | R.39 TDD | OP.4 TDD |  |  |  | 20.1 | ≥2 |
| 3b | 5MHz | R.39-1 TDD | OP.4 TDD |  |  |  | 20.5 | 1 |
| 4 | 1.4 MHz | R.40 TDD | OP.4 TDD |  |  |  | 5.8 | ≥1 |

## 10.3	FDD (Fixed Reference Channel) with FeMBMS

For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 0 dB for subcarrier spacing as 15kHz, 3dB for for subcarrier spacing as 7.5kHz and 10.8dB for for subcarrier spacing as 1.25kHz because the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

### 10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA

#### 10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for FeMBMS Unicast-mixed Cell under CA.

For the parameters specified in Table 10.3.1.1-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.1.1-2.

Table 10.3.1.1-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell and SCell |  |  | 1 |
| Subcarrier spacing for SCell |  | kHz | 1.25 kHz |
| Allocated subframes per Radio Frame for SCell (Note 2) |  |  | 8 subframes |
| Number of OFDM symbols for PDCCH for SCell |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix for Scell in non-MBSFN subframes |  |  | Extended |
| Note 1: . Note 2: For SCell with FDD mode, up to 8 subframes (#1/2/3/4/6/7/8/9) are available for MBMS, in line with TS 36.331. |  |  |  |

Table 10.3.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | SCell | 10 | R.81-1 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 14.1 | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | SCell | 10 | R.81-2 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 26.0 | ≥2 |

#### 10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for FeMBMS Unicast-mixed Cell under CA.

For the parameters specified in Table 10.3.1.2-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.1.2-2.

Table 10.3.1.2-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell and SCell |  |  | 1 |
| Subcarrier spacing for SCell |  | kHz | 7.5 kHz |
| Allocated subframes per Radio Frame for SCell (Note 2) |  |  | 8 subframes |
| Number of OFDM symbols for PDCCH for SCell |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix for Scell in non-MBSFN subframes |  |  | Extended |
| Note 1: . Note 2: For SCell with FDD mode, up to 8 subframes (#1/2/3/4/6/7/8/9) are available for MBMS, in line with TS 36.331. |  |  |  |

Table 10.3.1.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | SCell | 10 | R.82-1 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 13.9 | ≥2 |

### 10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell

#### 10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for FeMBMS Unicast-mixed Cell as non-serving cell.

For the parameters specified in Table 10.3.2.1-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.2.1-2.

Table 10.3.2.1-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Subcarrier spacing |  | kHz | 1.25 kHz |
| Allocated subframes per Radio Frame (Note 2) |  |  | 8 subframes |
| Number of OFDM symbols for PDCCH |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix in non-MBSFN subframes |  |  | Extended |
| Note 1: . Note 2: For FDD mode, up to 8 subframes (#1/2/3/4/6/7/8/9) are available for MBMS, in line with TS 36.331. |  |  |  |

Table 10.3.2.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | FeMBMS Unicast-mixed Cell as non-serving cell | 10 | R.81-1 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 14.1 | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | FeMBMS Unicast-mixed Cell as non-serving cell | 10 | R.81-2 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 26.0 | ≥2 |

#### 10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for enhanced FeMBMS Unicast-mixed Cell as non-serving cell.

For the parameters specified in Table 10.3.2.2-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.2.2-2.

Table 10.3.2.2-1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Subcarrier spacing |  | kHz | 7.5 kHz |
| Allocated subframes per Radio Frame (Note 2) |  |  | 8 subframes |
| Number of OFDM symbols for PDCCH |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix in non-MBSFN subframes |  |  | Extended |
| Note 1: . Note 2: For FDD mode, up to 8 subframes (#1/2/3/4/6/7/8/9) are available for MBMS, in line with TS 36.331. |  |  |  |

Table 10.3.2.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | FeMBMS Unicast-mixed Cell as non-serving cell | 10 | R.82-1 FDD | OP.4A FDD | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 13.9 | ≥2 |

### 10.3.3	Minimum requirement for MBMS Dedicated cell

#### 10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for MBMS Dedicated Cell.

For the parameters specified in Table 10.3.3.1-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.3.1-2.

Table 10.3.3.1-1: Test Parameters for Testing MBMS Dedicated Cell

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/1.25kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBMS Dedicated Cell |  | kHz | 1.25 kHz |
| Allocated subframes per Radio Frame for MBMS Dedicated Cell (Note 2) |  |  | 10 subframes |
| Number of OFDM symbols for PDCCH for MBMS Dedicated Cell |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix for CAS in MBMS Dedicated Cell |  |  | Extended |
| Note 1: . Note 2: All subframes are MBSFN subframes except CAS |  |  |  |

Table 10.3.3.1-2: Minimum performance for MBMS dedicated Cell

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.83-1 FDD | NA | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 14.1 | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.83-2 FDD | NA | MBSFN channel model (Table B.2.6.2-1) | 1x2 low | 1 | 26.0 | ≥2 |

#### 10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for MBMS Dedicated Cell.

For the parameters specified in Table 10.3.3.2-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.3.2-2.

Table 10.3.3.2-1: Test Parameters for Testing MBMS Dedicated Cell

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/1.25kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBMS Dedicated Cell |  | kHz | 7.5 kHz |
| Allocated subframes per Radio Frame for MBMS Dedicated Cell (Note 2) |  |  | 10 subframes |
| Number of OFDM symbols for PDCCH for MBMS Dedicated Cell |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix for CAS in MBMS Dedicated Cell |  |  | Extended |
| Note 1: . Note 2: All subframes are MBSFN subframes except CAS |  |  |  |

Table 10.3.3.2-2: Minimum performance for MBMS dedicated Cell

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.84-1 FDD | NA | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 13.9 | ≥2 |

#### 10.3.3.3	Minimum requirement with 15kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER. The requirement is valid for all RRC states for which the UE has capabilities for MBMS Dedicated Cell.

For the parameters specified in Table 10.3.3.3-1 and Annex A.3.8.1, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.3.3.3-2.

Table 10.3.3.3-1: Test Parameters for Testing MBMS Dedicated Cell

| Parameter |  | Unit | Test 1-3 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/1.25kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBMS Dedicated Cell |  | kHz | 15 kHz |
| Allocated subframes per Radio Frame for MBMS Dedicated Cell (Note 2) |  |  | 10 subframes |
| Number of OFDM symbols for PDCCH for MBMS Dedicated Cell |  |  | 0 (MBSFN subframes) 2 (non-MBSFN subframes) |
| Cyclic Prefix for MBMS Dedicated Cell |  |  | Extended |
| Note 1: . Note 2: All subframes are MBSFN subframes |  |  |  |

Table 10.3.3.3-2: Minimum performance for MBMS dedicated Cell

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 3 | R.85-1 FDD | NA | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 7.0 | ≥2 |
| 2 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 5 | R.85-2 FDD | NA | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 13.5 | ≥2 |
| 3 | PCell | 10 | NA | OP.1 FDD | AWGN | 1x2 low | NA | NA | NA |
|  | MBMS Dedicated Cell | 10 | R.85-3 FDD | NA | MBSFN channel model (Table B.2.6.1-1) | 1x2 low | 1 | 22.3 | ≥2 |

## 10.4 FDD with 5G terrestrial broadcast

For all tests in section 10.4 the applicability rules are defined in Table 10.4-1 depending on the capabilities of the UE. The requirements in clause 10.4 are applicable to UEs that support 5G terrestrial broadcast.

Table 10.4-1: 5G terrestrial broadcast tests applicability

| Tests | Applicable if UE supports at least the following capability |
| --- | --- |
| Table 10.4.1.1-2 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot4-r16 |
| Table 10.4.1.1-2 test 2 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot2-r16 |
| Table 10.4.1.2-2 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 |
| Table 10.4.2.1-1 test 1 | 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and 5G terrestrial broadcast support of PBCH repetition in CAS |
| Table 10.4.2.1-1 test 2 | 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 and 5G terrestrial broadcast support of PBCH repetition in CAS |

For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 16dB for subcarrier spacing as 0.37kHz, 7.8 dB for subcarrier spacing as 2.5kHz because the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

### 10.4.1	Minimum requirement for PMCH decoding

#### 10.4.1.1	Minimum requirement with 0.37kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 10.4.1.1-1 and Table A.3.8.1-9, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.1-2.

Table 10.4.1.1-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 (Note 1) |
|  |  | dB | 0 |
| Noc at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 0.37 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: PB = 0. Note 2: For both Pcell and Scell. |  |  |  |

Table 10.4.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | MBSFN RS type | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
| 2 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-2 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |

#### 10.4.1.2	Minimum requirement with 2.5kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 10.4.1.2-1 and Table A.3.8.1-10, the average downlink SNR shall be below the specified value for the BLER shown in Table 10.4.1.2-2.

Table 10.4.1.2-1: Test Parameters for Testing

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 (Note 1) |
|  |  | dB | 0 |
| Noc at antenna port |  | dBm/15kHz | Noc at antenna port |
| PDSCH transmission mode in PCell |  |  | 1 |
| Subcarrier spacing for MBSFN cell |  | kHz | 2.5 kHz |
| Bandwidth |  | MHz | 10(Note 2) |
| Note 1: PB = 0. Note 2: For both Pcell and Scell. |  |  |  |

Table 10.4.1.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |

### 10.4.2	Minimum requirement for CAS detection

#### 10.4.2.1	Minimum requirement for PBCH detection

For the parameters specified in Table 8.6.1-1, the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 10.4.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 10.4.2.1-1: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x1 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | AWGN | 1x1 low | 1 | -7.2 | ≥ 2 |
| 2 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | EVA 162Hz | 1x2 low | 1 | -6.7 | ≥ 2 |

# 11	Performance requirement (ProSe Direct Discovery)

This clause contains the performance requirements for the Sidelink physical channels specified for ProSe Direct Discovery.

## 11.1	General

### 11.1.1	Applicability of requirements

The requirements in this clause are applicable to UEs that support ProSe Direct Discovery. The test case applicability is in according to table 11.1.1-1 depending on set of supported UE capabilities.

Table 11.1.1-1: ProSe Direct Discovery test applicability

| FDD/TDD | Tests / clause | Applicable if UE indicates at least the following capability |
| --- | --- | --- |
| FDD | 11.2.1 | ProSe Direct Discovery without support of disc-SLSS-r12 |
|  | 11.2.3 | ProSe Direct Discovery with support of discPeriodicSLSS-r13 and ProSe Direct Communication |
|  | 11.3.1 | ProSe Direct Discovery |
|  | 11.4.1 | ProSe Direct Discovery with support of disc-SLSS-r12 |
|  | 11.5.1 | ProSe Direct Discovery |
| TDD | 11.2.2 | ProSe Direct Discovery |
|  | 11.3.2 | ProSe Direct Discovery |
|  | 11.5.2 | ProSe Direct Discovery |

For maximum Sidelink Processes test specified in clause 11.5, the UE is required to only meet the test for the maximum channel bandwidth over the ProSe operating bands supported by the UE. Test case 11.2.3 for 5MHz channel bandwidth is applicable to UEs that support ProSe Direct Communication on Band 31 only.

### 11.1.2	Reference DRX configuration

Table 11.1.2-1: Reference DRX configuration

| Parameter | Value | Comments |
| --- | --- | --- |
| onDurationTimer | psf1 |  |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf2560, 0 |  |
| shortDRX | disabled |  |
| NOTE 1: For further information see clause 6.3.2 in TS 36.331. |  |  |

## 11.2	Demodulation of PSDCH (single link performance)

The purpose of the requirements in this subclause is to verify the PSDCH demodulation performance with a single active PSDCH link under different operating scenarios and channel conditions.

The active cell(s), when present, are specified in the test parameters specific to the test.

### 11.2.1	FDD (in-coverage)

The minimum requirements are specified in Table 11.2.1-2 with the test parameters specified in Table 11.2.1-1. The receiver UE under test is associated with Cell 1.

Table 11.2.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.1-1 (Configuration #1-FDD) |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 3) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern (NOTE 2) |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSDCH |
|  | PSDCH RB allocation |  |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] in each discovery period. |
|  | Time offset (NOTE 4) |  | s | +1 |
|  | Frequency offset (NOTE 5) |  | Hz | +200 |
|  | Propagation Channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: Applicable to both DL channel and ProSe Direct Discovery Subframes on UL. NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.2.1-2: Minimum performance

| Test num. | Sidelink UE | Band-width | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 1 | 5 MHz | D.1 FDD | 30 | 4.6 |

### 11.2.2	TDD (in-coverage)

The minimum requirements are specified in Table 11.2.2-2 with the test parameters specified in Table 11.2.2-1. The receiver UE under test is associated with Cell 1.

Table 11.2.2-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.2-1 (Configuration #1-TDD) |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 5) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Uplink downlink configuration (NOTE 3) |  |  | 0 |
|  | Special subframe configuration (NOTE 4) |  |  | 4 |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 TDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSDCH |
|  | RB allocation |  |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] in each discovery period. |
|  | Time offset (NOTE 6) |  | s | +1 |
|  | Frequency offset (NOTE 7) |  | Hz | +200 |
|  | Propagation Channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: As specified in Table 4.2-2 in TS 36.211 [4]. NOTE 4: As specified in Table 4.2-1 in TS 36.211 [4]. NOTE 5: Applicable to both DL subframes and UL subframes configured for ProSe Direct Discovery. NOTE 6: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 7: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.2.2-2: Minimum performance

| Test num. | Sidelink UE | Band-width | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 1 | 5 MHz | D.1 TDD | 30 | 4.6 |

### 11.2.3	FDD (out-of-coverage)

The minimum requirements are specified in Table 11.2.3-2 with the test parameters specified in Table 11.2.3-1. The receiver UE under test is out of network coverage.

Table 11.2.3-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Resource pool configuration |  |  | As specified in Table A.7.1.1-4 (Configuration #4-FDD) |
| DRX configuration |  |  | As specified in Table 11.1.2-1 |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UEs 1, 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS + PSBCH |
|  | networkControlledSyncTx |  | ON |
|  | slssid |  | 30 |
|  | inCoverage (in MIB-SL) |  | TRUE |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in Configuration #4-FDD |
|  | Propagation channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -82 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSDCH |
|  | PSDCH RB allocation |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] (for 5MHz) or [0,24] (for 10MHz) in each discovery period. |
|  | Time offset (Note 1) | s | +1 |
|  | Frequency offset (Note 2) | Hz | +200 |
|  | Propagation Channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
| NOTE 1: Time offset of Sidelink UE 2 receive signal timing with respect to Sidelink UE 1 receive signal timing at the tested UE. NOTE 2: Frequency offset of Sidelink UE 2 with respect to Sidelink UE 1 transmit frequency. |  |  |  |

Table 11.2.3-2: Minimum performance

| Test number | Sidelink UE | Band-width | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 1 | 5 MHz | D.1 FDD | 30 | 4.6 |
|  |  | 10 MHz |  |  |  |

## 11.3	Power imbalance performance with two links

The purpose of this test is to check the demodulation performance when receiving PSDCH transmissions from two Sidelink UEs with power imbalance in one subframe.

### 11.3.1	FDD

The minimum requirements are specified in Table 11.3.1-2 with the test parameters specified in Table 11.3.1-1. The receiver UE under test is associated with Cell 1. The Sidelink UE 1 and 2 are synchronized to Cell 1 and transmit PSDCH on adjacent RBs.

Table 11.3.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.1-1 (Configuration #1-FDD) |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 3) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern (NOTE 2) |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSDCH |
|  | PSDCH RB allocation |  |  | PRB pairs {4..5} |
|  | Time offset (NOTE 3) |  | s | 0 |
|  | Frequency offset (NOTE 4) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 Low |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSDCH |
|  | PSDCH RB allocation |  |  | PRB pairs {6..7} |
|  | Time offset (w.r.t. Cell 1 DL) |  | s | 0 |
|  | Frequency offset (w.r.t. Cell 1 UL) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: Applicable to both DL channel and ProSe Direct Discovery Subframes on UL. NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.3.1-2: Minimum performance

| Test num. | Band-width | Sidelink UE | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 5 MHz | 1 | D.1 FDD | (NOTE 1) | 24.3 |
|  |  | 2 | D.1 FDD | 30 | 6.9 |
| NOTE 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |

### 11.3.2	TDD

The minimum requirements are specified in Table 11.3.2-2 with the test parameters specified in Table 11.3.2-1. The receiver UE under test is associated with Cell 1. The Sidelink UE 1 and 2 are synchronized to Cell 1 and transmit PSDCH on adjacent RBs.

Table 11.3.2-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.2-1 (Configuration #1-TDD) |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 5) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Uplink downlink configuration (NOTE 3) |  |  | 0 |
|  | Special subframe configuration (NOTE 4) |  |  | 4 |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 TDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSDCH |
|  | PSDCH RB allocation |  |  | PRB pairs {4..5} |
|  | Time offset (NOTE 6) |  | s | 0 |
|  | Frequency offset (NOTE 7) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 Low |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSDCH |
|  | RB allocation |  |  | PRB pairs {6..7} |
|  | Time offset (NOTE 6) |  | s | 0 |
|  | Frequency offset (NOTE 7) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: As specified in Table 4.2-2 in TS 36.211 [4]. NOTE 4: As specified in Table 4.2-1 in TS 36.211 [4]. NOTE 5: Applicable to both DL subframes and UL subframes configured for ProSe Direct Discovery. NOTE 6: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 7: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.3.2-2: Minimum performance

| Test num. | Band-width | Sidelink UE | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 5 MHz | 1 | D.1 TDD | (NOTE 1) | 24.3 |
|  |  | 2 | D.1 TDD | 30 | 6.9 |
| NOTE 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |

## 11.4	Multiple timing reference test

The purpose of this test is to check the demodulation performance when receiving from two Sidelink UEs that follow different timing references and transmitting on different resources (non-overlapping in time).

### 11.4.1	FDD

The test parameters are specified in Table 11.4.1-1. Sidelink UE 2 and the receiver UE under test are associated with Cell 1. Sidelink UE 1 and 3 are associated with another cell and use a different timing, and UE 1 acts as a synchronization reference. The minimum requirements are specified in Table 11.4.1-2.

Table 11.4.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.1-2 (Configuration #2-FDD) |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 3) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UEs 1, 2, 3 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | SLSS |
|  | networkControlledSyncTx |  |  | ON |
|  | slssid |  |  | 30 |
|  | Time offset (NOTE 4) |  | s | 3511 |
|  | Frequency offset (NOTE 5) |  | Hz | -100 |
|  | Propagation channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
|  | of SLSS at antenna port |  | dBm/15kHz | -82 |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSDCH |
|  | Resource pool used for transmissions |  |  | discRxPool(0) |
|  | RB allocation |  |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] in each discovery period. |
|  | Time offset (NOTE 4) |  | s | +1 |
|  | Frequency offset (NOTE 5) |  | Hz | +200 |
|  | Propagation Channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
| Sidelink UE 3 | Sidelink Transmissions |  |  | PSDCH |
|  | Resource pool used for transmissions |  |  | discRxPool(1) |
|  | RB allocation |  |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] in each discovery period. |
|  | Time offset (NOTE 4) |  | s | 3511 |
|  | Frequency offset (NOTE 5) |  | Hz | +300 |
|  | Propagation Channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: Applicable to both DL channel and ProSe Direct Discovery Subframes on UL. NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.4.1-2: Minimum performance

| Test num. | Band-width | Sidelink UE | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%)NOTE 1 | SNR (dB) |
| 1 | 5 MHz | 2 | D.1 FDD | 30 | 4.6 |
|  |  | 3 | D.1 FDD | 30 | 4.6 |
| NOTE 1: The BLER is measured after 5 D2D Discovery periods (1600 frames) of lead time during which the test UE detects and synchronizes to Sidelink UE 1 SLSS. |  |  |  |  |  |

## 11.5	Maximum Sidelink processes test

The purpose of this test is to verify the maximum number of Sidelink processes supported by the UE as reported using UE capability signalling (discSupportedProc).

The UE is required to meet only the test for the maximum channel bandwidth over the ProSe operating bands supported by the UE.

### 11.5.1	FDD

The test parameters are specified in Table 11.5.1-1. Multiple discovery resource pools are interleaved. Each Sidelink UE transmits in one of the resource pools with 3 retransmissions. The minimum requirements are specified in Table 11.5.1-2.

Table 11.5.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1-7 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.1-3 (Configuration #3-FDD) with parameters BWChannel, NPools = Number of configured resource pools (as specified in Table 11.5.1-2), and N = discSupportedProc |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 FDD |
|  | Propagation channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -85 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE i, i = 0, …, discSupportedProc-1 |
| Sidelink UE i | Sidelink Transmissions |  |  | PSDCH (D.1 FDD) |
|  | Resource pool index (NOTE 3) |  |  |  |
|  | PSDCH RB allocation (NOTE 3) |  |  | PRB pairs {2*(i % NMAX_SF), 2*(i % NMAX_SF)+1} |
|  | Time offset (NOTE 4) |  | s | 0 |
|  | Frequency offset (NOTE 4) |  | Hz | 0 |
|  | Propagation Channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs.. NOTE 3: NMAX_SF represents the maximum number of Sidelink UEs transmitting in one subframe. NMAX_SF = 12 (5 MHz), 25 (10MHz), 37 (15MHz), 50 (10MHz). NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.5.1-2: Minimum performance

| Test num. | Bandwidth | discSupportedProc | Number of configured resource pools | at antenna port (dBm/15kHz) | Reference value for Sidelink UE i=0…discSupportedProc-1 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) |
| 1 | 5 MHz | 50 | 5 | -85 | 95 |
| 2 | 10 MHz | 50 | 2 | -85 | 95 |
| 3 | 15 MHz | 50 | 2 | -85 | 95 |
| 4 | 20 MHz | 50 | 1 | -85 | 95 |
| 5 | 10 MHz | 400 | 16 | -85 | 95 |
| 6 | 15 MHz | 400 | 11 | -85 | 95 |
| 7 | 20 MHz | 400 | 8 | -85 | 95 |

### 11.5.2	TDD

The test parameters are specified in Table 11.5.2-1. Multiple discovery resource pools are interleaved. Each Sidelink UE transmits in one of the resource pools with 3 retransmissions. The minimum requirements are specified in Table 11.5.2-2.

Table 11.5.2-1: Test Parameters

| Parameter |  |  | Unit | Test 1-7 |
| --- | --- | --- | --- | --- |
| Discovery resource pool configuration |  |  |  | As specified in Table A.7.1.2-2 (Configuration #2-TDD) with parameters BWChannel, NPools = Number of configured resource pools (as specified in Table 11.5.2-2), and N = discSupportedProc |
| DRX configuration |  |  |  | As specified in Table 11.1.2-1 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Uplink downlink configuration (NOTE 3) |  |  | 0 |
|  | Special subframe configuration (NOTE 4) |  |  | 4 |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 TDD |
|  | Propagation channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -85 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE i, i = 0, …, discSupportedProc-1 |
| Sidelink UE i | Sidelink Transmissions |  |  | PSDCH (D.1 TDD) |
|  | PSDCH Resource pool (NOTE 5) |  |  |  |
|  | PSDCH RB allocation (NOTE 5) |  |  | PRB pairs {2*(i % NMAX_SF),2*(i % NMAX_SF)+1} |
|  | Time offset (NOTE 6) |  | s | 0 |
|  | Frequency offset (NOTE 7) |  | Hz | 0 |
|  | Propagation Channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: As specified in Table 4.2-2 in TS 36.211 [4]. NOTE 4: As specified in Table 4.2-1 in TS 36.211 [4]. NOTE 5: NMAX_SF represents the maximum number of Sidelink UEs transmitting in one subframe. NMAX_SF = 12 (5 MHz), 25 (10MHz), 37 (15MHz), 50 (10MHz). NOTE 6: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 7: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 11.5.2-2: Minimum performance

| Test num. | Bandwidth | discSupportedProc | Number of configured resource pools | at antenna port (dBm/15kHz | Reference value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) for Sidelink UE i=0…discSupportedProc-1 |
| 1 | 5 MHz | 50 | 5 | -85 | 95 |
| 2 | 10 MHz | 50 | 2 | -85 | 95 |
| 3 | 15 MHz | 50 | 2 | -85 | 95 |
| 4 | 20 MHz | 50 | 1 | -85 | 95 |
| 5 | 10 MHz | 400 | 16 | -85 | 95 |
| 6 | 15 MHz | 400 | 11 | -85 | 95 |
| 7 | 20 MHz | 400 | 8 | -85 | 95 |

# 12	Performance requirement (ProSe Direct Communication)

This clause contains the performance requirements for the Sidelink physical channels specified for ProSe Direct Communication in TS 36.211 [4].

## 12.1	General

### 12.1.1	Applicability of requirements

#### 12.1.1.1	Applicability of requirements for different channel bandwidths

The requirements in this clause are applicable to UEs that support ProSe Direct Communication. Test cases defined for 5MHz channel bandwidth are applicable to UEs that support ProSe Direct Communication on only Band 31.

#### 12.1.1.2	Test coverage for different number of component carriers

For FDD tests specified in 12.8, if corresponding CA tests are tested, the test coverage can be considered fulfilled without executing single carrier tests.

#### 12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA UE demodulation tests with active Sidelink in Clause 12 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 12.1.1.3-1. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 12.1.1.3-1: Applicability and test rules for CA UE demodulation tests with active Sidelink

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 12.8 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |

### 12.1.2	Reference DRX configuration

Table 12.1.2-1: Reference DRX configuration

| Parameter | Value | Comments |
| --- | --- | --- |
| onDurationTimer | psf1 |  |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf2560, 0 |  |
| shortDRX | disabled |  |
| NOTE 1: For further information see clause 6.3.2 in TS 36.331. |  |  |

## 12.2	Demodulation of PSSCH

The purpose of the requirements in this subclause is to verify the PSSCH demodulation performance with a single active PSSCH link.

### 12.2.1	FDD

The minimum requirements are specified in Table 12.2.1-2 with the test parameters specified in Table 12.2.1-1. This test specifies an out-of-coverge scenario where Sidelink UE 1 is the synchronization reference only and Sidelink UE 2 transmits PSCCH and PSSCH.

Table 12.2.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.7.2.1-1 (Configuration #1-FDD) |
| at antenna port (NOTE 1) |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS + PSBCH |
|  | networkControlledSyncTx |  | ON |
|  | slssid |  | 30 |
|  | inCoverage (in MIB-SL) |  | FALSE |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in Configuration #1-FDD |
|  | Propagation channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RMC |  | 5MHz: CC.3 FDD 10 MHz: CC.4 FDD |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with chosen randomly (uniformly) in every sc-period |
|  | PSCCH RB allocation |  |  |
|  | of PSCCH at antenna port | dBm/15kHz | -85 |
|  | PSSCH RMC |  | As specificied in Table 12.2.1-2 |
|  | PSSCH subframe allocation |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  | First transmission: Chosen randomly (uniformly) among the allowed RBs as per TS36.213 HARQ retransmission: As per frequency hopping indicated in PSCCH and specified in TS36.213 |
|  | Time offset (NOTE 2) | s | +1 |
|  | Frequency offset (NOTE 3) | Hz | +200 |
|  | Propagation Channel |  | EVA70 |
|  | Antenna configuration |  | 1x2 Low |
| NOTE 1: Applicable to both DL channel and ProSe Direct Communication Subframes on UL. NOTE 2: Time offset of Sidelink UE 2 receive signal timing with respect to Sidelink UE 1 receive signal timing at the tested UE. NOTE 3: Frequency offset of Sidelink UE 2 with respect to Sidelink UE 1 transmit frequency. |  |  |  |

Table 12.2.1-2: Minimum performance

| Test num. | Sidelink UE | Band-width | PSSCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) (NOTE 1) | SNR (dB) of PSSCH |
| 1 | 2 | 10 MHz | CD.1 FDD | 70 | -3.4 |
|  |  | 5 MHz |  | 70 | -3.3 |
| NOTE 1: The throughput is measured after 40 radio frames of lead time during which the test UE detects and synchronizes to Sidelink UE 1. |  |  |  |  |  |

## 12.3	Demodulation of PSCCH

The purpose of the requirements in this subclause is to verify the PSCCH demodulation performance with a single active PSSCH link.

### 12.3.1	FDD

The minimum requirements are specified in Table 12.3.1-2 with the test parameters specified in Table 12.3.1-1. This test specifies an out-of-coverage scenario where Sidelink UE 1 is the synchronization reference only and Sidelink UE 2 transmits PSCCH and PSSCH.

Table 12.3.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.7.2.1-1 (Configuration #1-FDD) |
| at antenna port (NOTE 1) |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS + PSBCH |
|  | networkControlledSyncTx |  | ON |
|  | slssid |  | 30 |
|  | inCoverage (in MIB-SL) |  | FALSE |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in Configuration #1-FDD |
|  | Propagation channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RMC |  | As specified in Table 12.3.1-2 |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with chosen randomly (uniformly) in every sc-period |
|  | PSCCH RB allocation |  |  |
|  | PSSCH RMC |  | CD.1 FDD |
|  | PSSCH subframe allocation |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  | First transmission: Chosen randomly (uniformly) among the allowed RBs as per TS36.213 HARQ retransmission: As per frequency hopping indicated in PSCCH and specified in TS36.213 |
|  | Time offset (NOTE 2) | s | +1 |
|  | Frequency offset (NOTE 3) | Hz | +200 |
|  | Propagation Channel |  | EVA70 |
|  | Antenna configuration |  | 1x2 Low |
| NOTE 1: Applicable to both DL channel and ProSe Direct Communication Subframes on UL. NOTE 2: Time offset of Sidelink UE 2 receive signal timing with respect to Sidelink UE 1 receive signal timing at the tested UE. NOTE 3: Frequency offset of Sidelink UE 2 with respect to Sidelink UE 1 transmit frequency. |  |  |  |

Table 12.3.1-2: Minimum performance

| Test num. | Sidelink UE | Band-width | PSCCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) (NOTE 1) | SNR (dB) of PSCCH |
| 1 | 2 | 10 MHz | CC.4 FDD | 1 | 4.7 |
|  |  | 5 MHz | CC.3 FDD | 1 | 4.8 |
| NOTE 1: The probability is measured after 40 radio frames of lead time during which the test UE detects and synchronizes to Sidelink UE 1. |  |  |  |  |  |

## 12.4	Demodulation of PSBCH

The purpose of the requirements in this subclause is to verify the PSBCH demodulation performance with a single active link.

### 12.4.1	FDD

The minimum requirements are specified in Table 12.4.1-2 with the test parameters specified in Table 12.4.1-1.

Table 12.4.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.7.2.1-1 (Configuration #1-FDD) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS + PSBCH (CP.1 FDD) |
|  | networkControlledSyncTx |  | ON |
|  | slssid |  | 30 |
|  | inCoverage (in MIB-SL) |  | FALSE |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in Configuration #1-FDD |
|  | Propagation channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |

Table 12.4.1-2: Minimum performance

| Test num. | Sidelink UE | Band-width | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSBCH (%) (NOTE 1) | SNR (dB) |
| 1 | 1 | 10 MHz | PSBCH (CP.1 FDD) | 1 | 4.4 |
|  |  | 5 MHz |  |  |  |
| NOTE 1: The probability is measured after 40 radio frames of lead time during which the test UE detects and synchronizes to Sidelink UE 1. |  |  |  |  |  |

## 12.5	Power imbalance performance with two links

The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two Sidelink UEs with power imbalance in one subframe.

### 12.5.1		FDD

The test parameters in Table 12.5.1-1 specifies an in-coverage scenario where Sidelink UE 1 and 2 are synchronized to Cell 1 and transmit PSSCH on adjacent RBs. The minimum requirements are specified in Table 12.5.1-2.

Table 12.5.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  |  | As specified in Table A.7.2.1-2 (Configuration #2-FDD) |
| DRX configuration |  |  |  | As specified in Table 12.1.2-1 |
| at antenna port (Note 3) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern (Note 2) |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | PSCCH RMC |  |  | 5 MHz: CC.1 FDD 10 MHz: CC.2 FDD |
|  | PSCCH subframe allocation |  |  | (as defined in TS 36.213) |
|  | PSCCH RB allocation |  |  |  |
|  | of PSCCH at antenna port |  | dBm/15kHz | -85 |
|  | PSSCH RMC |  |  | As specified in Table 12.5.1-2 |
|  | PSSCH subframe allocation |  |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  |  | PRB pairs {4, 5} |
|  | Time offset (NOTE 4) |  | s | 0 |
|  | Frequency offset (NOTE 5) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | PSCCH RMC |  |  | 5 MHz: CC.1 FDD 10 MHz: CC.2 FDD |
|  | PSCCH subframe allocation |  |  | (as defined in TS 36.213) |
|  | PSCCH RB allocation |  |  |  |
|  | of PSCCH at antenna port |  | dBm/15kHz | -85 |
|  | PSSCH RMC |  |  | As specified in Table 12.5.1-2 |
|  | PSSCH subframe allocation |  |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  |  | PRB pairs {6, 7} |
|  | Time offset (NOTE 4) |  | s | 0 |
|  | Frequency offset (NOTE 5) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: Applicable to both DL channel and ProSe Direct Communication Subframes on UL. NOTE 4: The power of PSCCH is set high to ensure reliable reception of PSCCH. NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 12.5.1-2: Minimum performance

| Test num. | Band-width | Sidelink UE | PSSCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) of PSSCH |
| 1 | 5 / 10 MHz | 1 | CD.5 FDD | (NOTE 1) | 24.35 |
|  |  | 2 | CD.5 FDD | 70 | 2.4 |
| NOTE 1: There is no throughput requirement for Sidelink UE 1. |  |  |  |  |  |

## 12.6	Multiple timing reference test

The puporse of this test is to check the PSSCH demodulation performance when receiving from two Sidelink UEs that follow different timing references and transmitting on different resources (non-overalapping in time).

### 12.6.1	FDD

The test parameters are specified in Table 12.6.1-1. Sidelink UE 2 and the receiver UE under test are associated with Cell 1. Sidelink UE 1 and Sidelink UE 3 are associated with another cell and use a different timing, and Sidelink UE 1 acts as a synchronization reference only. The minimum requirements are specified in Table 12.6.1-2.

Table 12.6.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  |  | As specified in Table A.7.2.1-3 (Configuration #3-FDD) |
| DRX configuration |  |  |  | As specified in Table 12.1.2-1 |
| at antenna port (Note 3) |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern NOTE 2 |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -92 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2, Sidelink UE 3 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | SLSS + PSBCH |
|  | networkControlledSyncTx |  |  | ON |
|  | slssid |  |  | 30 |
|  | inCoverage (in MIB-SL) |  |  | TRUE |
|  | syncOffsetIndicator |  |  | Set same as syncOffsetIndicator in Configuration #3-FDD |
|  | Time offset (NOTE 5) |  | ms | +12.51 ms |
|  | Frequency offset (NOTE 6) |  | Hz | -100 Hz |
|  | Propagation channel |  |  | EPA5 |
|  | Antenna configuration |  |  | 1x2 Low |
|  | at antenna port |  | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Resource pool |  |  | commRxPool(0) |
|  | PSCCH RMC |  |  | 5MHz: CC.1 FDD 10 MHz: CC.2 FDD (NOTE 5) |
|  | PSCCH subframe allocation |  |  | As defined by TS 36.213 with chosen randomly (uniformly) in every sc-period |
|  | PSCCH RB allocation |  |  |  |
|  | of PSCCH at antenna port |  | dBm/15kHz | -85 |
|  | PSSCH RMC |  |  | As specified in Table 12.6.1-2 |
|  | PSSCH subframe allocation |  |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  |  | First transmission: Chosen randomly (uniformly) among the allowed RBs as per TS36.213 HARQ retransmission: As per frequency hopping indicated in PSCCH and specified in TS36.213 |
|  | Time offset (NOTE 4, 5) |  |  | PSCCH: +1sPSSCH: +1s – 288Ts |
|  | Frequency offset (NOTE 6) |  | Hz | +200 |
|  | Propagation Channel |  |  | EVA70 |
|  | Antenna configuration |  |  | 1x2 Low |
| Sidelink UE 3 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Resource pool |  |  | commRxPool(1) |
|  | PSCCH RMC |  |  | 5MHz: CC.5 FDD 10 MHz: CC.6 FDD |
|  | PSCCH subframe allocation |  |  | As defined by TS 36.213 with chosen randomly (uniformly) in every sc-period |
|  | PSCCH RB allocation |  |  |  |
|  | of PSCCH at antenna port |  | dBm/15kHz | -85 |
|  | PSSCH RMC |  |  | As specified in Table 12.6.1-2 |
|  | PSSCH subframe allocation |  |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  |  | First transmission: Chosen randomly (uniformly) among the allowed RBs as per TS36.213 HARQ retransmission: As per frequency hopping indicated in PSCCH and specified in TS36.213 |
|  | Time offset (NOTE 5) |  | ms | +12.509 |
|  | Frequency offset (NOTE 6) |  | Hz | +300 |
|  | Propagation Channel |  |  | EVA70 |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: Applicable to both DL channel and ProSe Direct Communication Subframes on UL. NOTE 4: Timing advance indication in PSSCH is set as 18 (=288Ts) in this test. PSSCH timing is advanced with respect to PSCCH timing by the quantity (i.e., PSSCH timing shall be +1s – 288Ts in this test). NOTE 5: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 6: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 12.6.1-2: Minimum performance

| Test num. | Band-width | Sidelink UE | PSSCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) (NOTE 1) | SNR (dB) |
| 1 | 10 MHz | 2 | CD.4 FDD | 70 | 3.0 |
|  |  | 3 | CD.2 FDD | 70 | 2.8 |
|  | 5 MHz | 2 | CD.3 FDD | 70 | 2.9 |
|  |  | 3 | CD.2 FDD | 70 | 2.8 |
| NOTE 1: The throughput is measured after 40 radio frames of lead time during which the test UE detects and synchronizes to Sidelink UE 1. |  |  |  |  |  |

## 12.7	Maximum Sidelink processes test

The purpose of this test is to verify the maximum number of Sidelink processes and the maximum number of bits per TTI supported by the UE.

### 12.7.1	FDD

The test parameters are specified in Table 12.7.1-1. Multiple communication resource pools are interleaved. Each active Sidelink UE transmits in one of the resource pools with 3 retransmissions. The minimum requirements are specified in Table 12.7.1-2.

Table 12.7.1-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  |  | As specified in Table A.7.2.1-4 (Configuration #4-FDD) |
| DRX configuration |  |  |  | As specified in Table 12.1.2-1 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (NOTE 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern (Note 2) |  |  | OP.1 FDD |
|  | Propagation channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 |
|  | RSRP |  | dBm/15kHz | -85 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE i, 0 ≤ i ≤ 15 |
| Sidelink UE i, 0 ≤ i ≤ 15 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Resource pool |  |  | commRxPool() |
|  | PSCCH RMC |  |  | 5MHz: CC.1 FDD with ITRP=i%8 (NOTE 3) 10 MHz: CC.2 FDD with ITRP= i%8 (NOTE 3) |
|  | PSCCH subframe allocation |  |  | As defined by TS 36.213 with = i |
|  | PSCCH RB allocation |  |  |  |
|  | PSSCH RMC |  |  | As specified in Table 12.7.1-2 |
|  | PSSCH subframe allocation |  |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  |  | Fully allocated |
|  | Time offset (NOTE 4) |  | s | 0 |
|  | Frequency offset (NOTE 5) |  | Hz | 0 |
|  | Propagation Channel |  |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  |  | 1x2 Low |
| NOTE 1: . NOTE 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. NOTE 3: For NTRP = 8 (FDD) and trpt-Subset = 001, ITRP = 0 corresponds to a time repetition pattern of (1,0,0,0,0,0,0,0), ITRP = 1 corresponds to a time repetition pattern of (0,1,0,0,0,0,0,0), etc. NOTE 4: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 5: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. |  |  |  |  |

Table 12.7.1-2: Minimum performance

| Test num. | Bandwidth | PSCCH Reference channel | at antenna port (dBm/15kHz) | Reference value for Sidelink UE i=0…15 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) |
| 1 | 10 MHz | CD.7 FDD | -85 | 95 |
|  | 5 MHz | CD.6 FDD | -85 | 95 |

## 12.8	Sustained downlink data rate with active Sidelink

The purpose of this test is to verify the downlink data rate is not impacted when Sidelink resource are also configured. The test parameters are in Table 12.8.1-1. Cell 1 is the serving cell and UE 1 and UE 2 are transmitters of Prose Direct Communication. The test UE is expected to receive all PDSCH transmissions, and prioritize the transmission of ACK/NACK over the reception of UE 2’s PSSCH.

The test cases apply to UE categories and bandwidth combinations with maximum aggregated bandwidth as specified in Table 12.8.1-2. The minimum requirements are specified in Table 12.8.1-3. The TB success rate in the cellular link shall be sustained during at least 300 frames.

Table 12.8.1-1: Test parameters for sustained downlink data rate (FDD 64QAM) with active Sidelink

| Parameter |  | Unit | Test 1, 2, 3A, 3B, 4A, 6C |
| --- | --- | --- | --- |
| Communication resource pool configuration Note 5 |  |  | As specified in Table A.7.2.1-5 (Configuration #5-FDD) |
| Active cell(s) |  |  | Cell 1 (PCell) Cell 2 (SCell) for Test 3B, 4A, 6C |
| Cell 1 | Test parameters |  | As specified in clause 8.7.1: Table 8.7.1-1 and Test 1, 2, 3A , 3B, 4A, 6C in Table 8.7.1-2 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RMC |  | 10 MHz: CC.2 FDD with ITRP=0 (NOTE 1) |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with = 0 |
|  | PSCCH RB allocation |  |  |
|  | PSSCH RMC |  | 10 MHz: CD.7 FDD |
|  | PSSCH subframe allocation |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  | Fully allocated |
|  | Time offset (NOTE 3) | s | 0 |
|  | Frequency offset (NOTE 4) | Hz | 0 |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH (NOTE 2) |
|  | PSCCH RMC |  | 10 MHz: CC.2 FDD with ITRP=1 (NOTE 1) |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with = 1 |
|  | PSCCH RB allocation |  |  |
|  | Time offset (NOTE 3) | s | 0 |
|  | Frequency offset (NOTE 4) | Hz | 0 |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| NOTE 1: For NTRP = 8 (FDD) and trpt-Subset = 001, ITRP = 0 corresponds to a time repetition pattern of (1,0,0,0,0,0,0,0), ITRP = 1 corresponds to a time repetition pattern of (0,1,0,0,0,0,0,0). NOTE 2: Sidelink UE 2 transmits PSCCH but not PSSCH. NOTE 3: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 4: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. NOTE 5: Sidelink Communication resources are configured on the primary serving cell. |  |  |  |

Table 12.8.1-2: Test cases for sustained data rate

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DL Cat. 11,12 |  |
| Single carrier | 10 | 1 | 2 | 3A | 3A | 3A | 3A | 3A | 3A |
| CA with 2CCs (Note1) | 10+10 | - | - | 3B | 4A | 4A | 4A | 4A | 4A |
|  | 10+20 | - | - | 3B | 4A | 6C | 6C | 6C | 6C |
| NOTE 1: Sidelink operation is configured on PCC |  |  |  |  |  |  |  |  |  |

Table 12.8.1-3: Minimum requirements (FDD 64QAM) with active Sidelink

| Test | Bandwidth (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | PDSCH TB success rate (%) |
| 1 | 10 | 10296 | R.31-1 FDD (NOTE 2) | 95 |
| 2 | 10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 3A | 10 | 36696 (NOTE 1) | R.31-3A FDD (NOTE 2) | 85 |
| 3B | 10+10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 4A | 10+10 | 75376 (NOTE 3) | R.31-4 FDD (NOTE 2) | 85 |
| 6C | 10+20 | 36696 (NOTE 1) for 10MHz CC 75376 (NOTE 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| NOTE 1: 35160 bits for sub-frame 5. NOTE 2: PDSCH scheduling pattern is changed as per the following bitmap that repeats every 40ms, and applies to all the serving cells.. PDSCH scheduling subframe bitmap = {01110111 11110111 11110111 11110111 11111110}. NOTE 3: 71112 bits for sub-frame 5. |  |  |  |  |

# 13	Void

# 14	Performance requirement (V2X Sidelink Communication)

This clause contains the performance requirements for the sidelink physical channels specified for V2X Sidelink Communication.

## 14.1	General

### 14.1.1	Applicability of requirements

The requirements in this clause are applicable to UEs that support V2X sidelink communication using Band 47.

When GNSS or GNSS-equivalent synchronization source is used, the reference GNSS signal power is as specified in Table B.6.1-1 in TS 36.133.

The test case applicability is in according to Table 14.1.1-1.

Table 14.1.1-1: V2X test applicability

| Tests / clause | Applicable if UE supports the following capability |
| --- | --- |
| 14.2 14.3 14.4 14.8 Test 1 | UE supporting V2X sidelink If the SL-C-RX Category 4 UE passes 14.11 Test 1 or 2, then the test coverage can be considered fulfilled without executing 14.8 Test 1. |
| 14.5 | UE with support of slss-TxRx-r14 [7] |
| 14.6 14.9 | UE supporting concurrent operation band for V2X and E-UTRA |
| 14.7 | UE not supporting concurrent operation band for V2X and E-UTRA |
| 14.8 Test 2 | UE with support of v2x-HighReception-r14 [7]. If the SL-C-RX Category 4 UE passes 14.11 Test 3 or 4, then the test coverage can be considered fulfilled without executing 14.8 Test 2. |
| 14.10 Test 1 14.11 Test 1 | UE with support of CA with 2 SL CCs and without support of CA with 3 SL CCs |
| 14.10 Test 2 14.11 Test 2 | UE with support of CA with 3 SL CCs |
| 14.11 Test 3 | UE with support of v2x-EnhancedHighReception-r15 [7] and CA with 2 SL CCs and without support of CA with 3 SL CCs |
| 14.11 Test 4 | UE with support of v2x-EnhancedHighReception-r15 [7] and CA with 3 SL CCs |

## 14.2	Demodulation of PSSCH

The purpose of the requirements in this subclause is to verify the PSSCH for V2X demodulation performance with a single active PSSCH link.

The minimum requirements are specified in Table 14.2-2 with the test parameters specified in Table 14.2-1. In this test scenario, GNSS or GNSS-equivalent synchronization source is used and Sidelink UE 1 transmits PSCCH and PSSCH.

Table 14.2-1: Test Parameters

| Parameter |  | Unit | Test 1, 2 | Test 3, 4 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-1 (Configuration #1-V2X) |  |
| at antenna port |  | dBm/15kHz | -98 |  |
| Active cell(s) |  |  | None |  |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | N/A | SL-V2X-TxProfile-r15 = rel15 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH+PSSCH |  |
|  | Timing offset (Note 1) |  | CP/2-12Ts |  |
|  | Frequency offset (Note 2) | Hz | +600 |  |
|  | Synchronization |  | GNSS or GNSS-equivalent |  |
|  | Antenna configuration |  | 1x2 Low |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |  |

Table 14.2-2: Minimum performance

| Test num. | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 MHz | CD.8 | EVA180 | 10 | 14.2 |
| 2 | 10 MHz | CD.9 | EVA2700 |  | 5.4 |
| 3 | 20 MHz | CD.13 | EVA180 |  | 12.5 |
| 4 | 10 MHz | CD.14 | EVA2700 |  | 2.8 |

## 14.3	Demodulation of PSCCH

The purpose of the requirements in this subclause is to verify the PSCCH for V2X demodulation performance with a single active PSSCH link.

The minimum requirements are specified in Table 14.3-2 with the test parameters specified in Table 14.3-1. In this test scenario, GNSS or GNSS-equivalent synchronization source is used and Sidelink UE 1 transmits PSCCH and PSSCH.

Table 14.3-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-1 (Configuration #1-V2X) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH+PSSCH |
|  | Timing offset (Note 1) |  | CP/2-12Ts |
|  | Frequency offset (Note 2) | Hz | +600 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
|  | PSSCH RMC |  | CD.9 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |

Table 14.3-2: Minimum performance

| Test number | Bandwidth | PSCCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) | SNR (dB) of PSCCH |
| 1 | 10 MHz | CC.8 | EVA1500 | 1 | 2.6 |

## 14.4	Power imbalance performance with two links

The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two Sidelink UEs with power imbalance in one subframe.

The minimum requirements are specified in Table 14.4-2 with the test parameters specified in Table 14.4-1. The Sidelink UE 1 and 2 are synchronized to GNSS or GNSS-equivalent synchronization reference.

Table 14.4-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-2 (Configuration #2-V2X) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RB allocation |  | PRB pairs {5, 6} |
|  | PSSCH RMC |  | As specified in Table 14.4-2 |
|  | PSSCH RB allocation |  | PRB pairs {7, 8, 9} |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RB allocation |  | PRB pairs {20, 21} |
|  | PSSCH RMC |  | As specified in Table 14.4-2 |
|  | PSSCH RB allocation |  | PRB pairs {22, 23, 24} |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

Table 14.4-2: Minimum performance

| Test number | Bandwidth | Sidelink UE | PSSCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20MHz | 1 | CD.10 | (Note 1) | 30.35 |
|  |  | 2 | CD.10 | 10 | 13.3 |
| Note 1: There is no throughput requirement for Sidelink UE 1. |  |  |  |  |  |

## 14.5	Demodulation of PSBCH

The purpose of the requirements in this subclause is to verify the PSBCH demodulation performance with a single active link.

The minimum requirements are specified in Table 14.5-2 with the test parameters specified in Table 14.5-1. The Sidelink UE 1 transmit PSBCH to tested UE and tested UE is synchronized to SLSS of Sidelink UE 1

Table 14.5-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-1 (Configuration #1-V2X) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS+PSBCH (Note 3) |
|  | slssid |  | 169 |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Antenna configuration |  | 1x2 Low |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: PSBCH transmits together with corresponding SLSS in the same subframe. |  |  |  |

Table 14.5-2: Minimum performance

| Test number | Bandwidth | PSBCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSBCH (%) | SNR (dB) of PSBCH |
| 1 | 20 MHz | CP.2 | EVA180 | 1 | 2.5 |

## 14.6	Demodulation of PSSCH with eNB based synchronization

The purpose of the requirements in this subclause is to verify the PSSCH for V2X demodulation performance with a single active PSSCH link under eNB based synchronization.

The minimum requirement is specified in Table 14.6-2 with the test parameters specified in Table 14.6-1. In this test scenario, eNB based synchronization source is used and Sidelink UE 1 transmits PSCCH and PSSCH.

Table 14.6-1: Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  |  | As specified in Table A.9-3 (Configuration #3-V2X) |
| at antenna port |  |  | dBm/15kHz | -98 |
| Active cell(s) |  |  |  | Cell 1 (Serving cell) |
| Cell 1 | Cyclic prefix |  |  | Normal |
|  | Cell ID |  |  | 0 |
|  | Downlink power allocation |  | dB | 0 |
|  |  |  | dB | 0 (Note 1) |
|  |  |  | dB | 0 |
|  | OCNG Pattern (Note 2) |  |  | OP.1 FDD |
|  | Propagation channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 Low |
|  | RSRP |  | dBm/15kHz | -92 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSCCH+PSSCH |
|  | Timing offset (Note 3) |  |  | CP/2-24Ts |
|  | Frequency offset (Note 4) |  | Hz | +1300 |
|  | Synchronization source |  |  | Cell 1 |
|  | Antenna configuration |  |  | 1x2 Low |
| Note 1: . Note 2: OCNG is used to fully allocate the available resource blocks to virtual UEs. Note 3: Time offset of Sidelink UE receive signal with respect to Cell1 reference timing. Note 4: Frequency offset of Sidelink UE with respect to Cell 1 reference frequency. |  |  |  |  |

Table 14.6-2: Minimum performance

| Test num. | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 MHz | CD.12 | EVA180 | 10 | 4.9 |

## 14.7	Soft buffer test

The purpose of this test is to verify the maximum number of bits per TTI supported by the V2X UE.

The minimum requirement is specified in Table 14.7-2 with the test parameters specified in Table 14.7-1.

Table 14.7-1: Test Parameters

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-4 (Configuration #4-V2X) |  |
| at antenna port |  | dBm/15kHz | -98 |  |
| Active cell(s) |  |  | None |  |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | N/A | SL-V2X-TxProfile-r15 = rel15 |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ 14 |  |
| Sidelink UE i, 0 ≤ i ≤ 14 | Sidelink Transmissions |  | PSCCH + PSSCH |  |
|  | Time gap between initial transmission and retransmission | Subframe | 15 |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Antenna configuration |  | 1x2 |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one circularly for every subframe. |  |  |  |  |

Table 14.7-2: Minimum performance

| Test num. | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  | ue-CategorySL-C-RX |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |  |
| 1 | 20 MHz | CD.11 | AWGN | 5 | 8.0 | SL-C-RX Category 2 |
| 2 | 20 MHz | CD.15 | AWGN | 5 | 12.0 | SL-C-RX Category 3 |

## 14.8	PSCCH/PSSCH decoding capability test

The purpose of this test is to verify the maximum number of Sidelink processes supported by the V2X UE.

The minimum requirements are specified in Table 14.8-3 with the test parameters specified in Table 14.8-1 and Table 14.8-2 according to UE capability.

Table 14.8-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 (Configuration #5-V2X) |
| Active cell(s) |  |  | None |
| Sidelink UE i, 0 ≤ i ≤ 9 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | CD.12 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |

Table 14.8-2: Test Parameters

| Parameter |  | Unit | Test 2 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 (Configuration #5-V2X) |
| Active cell(s) |  |  | None |
| Sidelink UE i, 0 ≤ i ≤ 19 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | CD.10 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |

Table 14.8-3: Minimum performance

| Test number | Bandwidth | PSCCH Reference channel | at antenna port (dBm/15kHz) | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 20 MHz | CC.8 | -85 | 1 |
| 2 | 20 MHz | CC.8 | -85 | 1 |

## 14.9	Sustained downlink data rate with active sidelink

The purpose of this test is to verify the WAN and V2X operation is not impacted with each other when UE is under concurrent operation. This test case applies to UEs support concurrent operation of V2X communication with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5G-2.

The test parameters are in Table 14.9-1, and the test UE is expected to receive all PSSCH and PDSCH transmissions simultaneously.

For PDSCH, the test cases apply to UE categories and bandwidth as specified in Table 14.9-2. The minimum requirements for PSSCH are specified in Table 14.7-2, Table 14.10-3 and Table 14.10-4 with corresponding test applicability and the minimum requirements for PDSCH downlink data rate are specified in Table 14.9-3. Both PDSCH and PSSCH performance will be verified simultaneously. The TB success rate in the cellular link shall be sustained during at least 300 frames.

Table 14.9-1: Test parameters for sustained downlink data rate (FDD 64QAM) and maximum PSSCH processing capability

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-4 (Configuration #4-V2X) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | Cell 1 (PCell) |
| Cell 1 | Test parameters |  | As specified in Table 8.7.1-1 and Table 14.9-2 |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ 14 |
| Sidelink UE i, 0 ≤ i ≤ 14 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Time gap between initial transmission and retransmission | Subframe | 15 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one circularly for every subframe. |  |  |  |

Table 14.9-2: Test cases for sustained data rate

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DL Cat. 11,12 |  |
| Single carrier | 10 | 1 | 2 | 3A | 3A | 3A | 3A | 3A | 3A |
|  | 20 | - | - | 3 | 4 | 6 | 6 | 6 | 6 |

Table 14.9-3: Minimum requirements (FDD 64QAM) for PDSCH

| Test | Bandwidth (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | PDSCH TB success rate (%) |
| 1 | 10 | 10296 | R.31-1 FDD (NOTE 2) | 95 |
| 2 | 10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 3 | 20 | 51024 | R.31-3 FDD | 95 |
| 3A | 10 | 36696 (NOTE 1) | R.31-3A FDD (NOTE 2) | 85 |
| 4 | 20 | 75376 (Note 2) | R.31-4 FDD | 85 |
| 6 | 20 | 75376 (Note 2) | R.31-4 FDD | 85 |
| NOTE 1: 35160 bits for sub-frame 5. NOTE 2: 71112 bits for sub-frame 5. |  |  |  |  |

## 14.10	Soft buffer test (CA)

The purpose of this test is to verify the maximum number of bits per TTI supported by the V2X UE with UE SL-C-RX Category 4 for UEs supporting V2X CA.

For CA with 2 SL CCs, the requirements are specified in Table 14.10-3, based on single carrier requirement specified in Table 14.10-2, with the test parameters specified in Table 14.10-1.

For CA with 3 SL CCs, the requirements are specified in Table 14.10-4, based on single carrier requirement specified in Table 14.10-2, with the test parameters specified in Table 14.10-1.

Table 14.10-1: Test Parameters

| Parameter |  | Unit | Test 1, 2 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-4 (Configuration #4-V2X) |
| at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | SL-V2X-TxProfile-r15 = rel15 |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ 14 |
| Sidelink UE i, 0 ≤ i ≤ 14 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Time gap between initial transmission and retransmission | Subframe | 15 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one circularly for every subframe. |  |  |  |

Table 14.10-2: Single carrier performance with different bandwidths for multiple CA configurations

| Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 10 MHz | CD.16 | AWGN | 5 | 12.0 |
| 20 MHz | CD.15 | AWGN | 5 | 12.0 |

Table 14.10-3: Minimum performance for CA with 2 SL CCs

| Test num. | CA Bandwidth combination | Requirement | ue-CategorySL-C-RX |
| --- | --- | --- | --- |
| 1 | 10+20 MHz | As specified in 14.10-2 per CC | SL-C-RX Category 4 |

Table 14.10-4: Minimum performance for CA with 3 SL CCs

| Test num. | CA Bandwidth combination | Requirement | ue-CategorySL-C-RX |
| --- | --- | --- | --- |
| 2 | 3x10 MHz | As specified in 14.10-2 per CC | SL-C-RX Category 4 |

## 14.11	PSCCH/PSSCH decoding capability test (CA)

The purpose of this test is to verify the maximum number of Sidelink processes and the maximum number of bits per TTI supported by the UE with UE SL-C-RX Category 4 for UEs supporting V2X CA.

For CA with 2 SL CCs, the requirements are specified in Table 14.11-4 based on single carrier requirement specified in Table 14.11-3, with the test parameters specified in Table 14.11-1 and Table 14.11-2 according to UE capability.

For CA with 3 SL CCs, the requirements are specified in Table 14.11-5 based on single carrier requirement specified in Table 14.11-3, with test parameters specified in Table 14.11-1 and Table 14.11-2 according to UE capability.

Table 14.11-1: Test Parameters

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 and A.9-6 (Configuration #5-V2X and #6-V2X) | As specified in Table A.9-6 (Configuration #6-V2X) |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | SL-V2X-TxProfile-r15 = rel15 |  |
| Active cell(s) |  |  | None |  |
| Sidelink UE i, 0 ≤ i ≤ 14 | Sidelink Transmissions |  | PSCCH + PSSCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |
|  | Antenna configuration |  | 1x2 |  |
|  | PSSCH RMC |  | CD.17 and CD.18 | CD.18 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |  |

Table 14.11-2: Test Parameters

| Parameter |  | Unit | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 and A.9-6 (Configuration #5-V2X and #6-V2X) | As specified in Table A.9-6 (Configuration #6-V2X) |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | SL-V2X-TxProfile-r15 = rel15 |  |
| Active cell(s) |  |  | None |  |
| Sidelink UE i, 0 ≤ i ≤ 29 | Sidelink Transmissions |  | PSCCH + PSSCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |
|  | Antenna configuration |  | 1x2 |  |
|  | PSSCH RMC |  | CD.19 and CD.20 | CD.20 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |  |

Table 14.11-3: Single carrier performance with different bandwidths for multiple CA configurations

| Bandwidth | PSCCH Reference channel | at antenna port (dBm/15kHz) | Reference value |
| --- | --- | --- | --- |
|  |  |  | Probability of missed PSCCH (%) |
| 10 MHz | CC.8 | -85 | 1 |
| 20 MHz | CC.8 | -85 | 1 |

Table 14.11-4: Minimum performance for CA configuration with 2 SL CCs

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 1 | 10+20 | As defined in Table 14.11-3 per CC |
| 3 | 10+20 | As defined in Table 14.11-3 per CC |

Table 14.11-5: Minimum performance for CA configuration with 3 SL CCs

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 2 | 3x10 | As defined in Table 14.11-3 per CC |
| 4 | 3x10 | As defined in Table 14.11-3 per CC |