3GPP TS 36.133 V15.3.0 (2018-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	81

1	Scope	82

2	References	82

3	Definitions, symbols and abbreviations	84

3.1	Definitions	84

3.2	Symbols	86

3.3	Abbreviations	87

3.4	Test tolerances	89

3.5	Additional notation	89

3.5.1	Groups of bands	89

3.6	General	91

3.6.1	Applicability of requirements in this specification version	91

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	98

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	99

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	100

4	E-UTRAN RRC_IDLE state mobility	101

4.1	Cell Selection	101

4.2	Cell Re-selection	101

4.2.1	Introduction	101

4.2.2	Requirements	101

4.2.2.1	Measurement and evaluation of serving cell	102

4.2.2.2	Void	103

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	103

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	105

4.2.2.5	Measurements of inter-RAT cells	107

4.2.2.5.1	Measurements of UTRAN FDD cells	107

4.2.2.5.2	Measurements of UTRAN TDD cells	109

4.2.2.5.3	Measurements of GSM cells	110

4.2.2.5.4	Measurements of HRPD cells	111

4.2.2.5.5	Measurements of cdma2000 1X	112

4.2.2.6	Evaluation of cell re-selection criteria	113

4.2.2.7	Maximum interruption in paging reception	114

4.2.2.8	void	114

4.2.2.9	UE measurement capability	114

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	115

4.2.2.10	Reselection to CSG cells	115

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	115

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	116

4.2.2.11	Void	117

4.2.2.12	Void	117

4.2.2.13	Void	117

4.3	Minimization of Drive Tests (MDT)	117

4.3.1	Introduction	118

4.3.2	Measurements	118

4.3.2.1	Requirements	118

4.3.3	Relative Time Stamp Accuracy	118

4.3.3.1	Requirements	118

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	118

4.3.4.1	 Requirements	118

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	119

4.3.5.1	Requirements for timeSinceFailure	119

4.4	MBSFN Measurements	119

4.4.1	Introduction	119

4.4.2	MBSFN RSRP measurements	119

4.4.3	MBSFN RSRQ measurements	119

4.4.4	MCH BLER measurements	120

4.5	Proximity-based Services	120

4.5.1	Introduction	120

4.5.2	Requirements	120

4.5.2.1	Interruptions with ProSe Direct Discovery	120

4.5.2.2	Interruptions with ProSe Direct Communication	120

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	120

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	121

4.6	Cell Selection and Re-selection Requirements for UE category NB1	121

4.6.1	Cell Selection	121

4.6.2	Cell Re-selection	121

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	122

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage when configured with WUS	122

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	123

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	124

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage when configured with WUS	125

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	126

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	127

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	128

4.6.2.7	Maximum interruption in paging reception in normal coverage	130

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	130

4.6.2.8	UE measurement capability	130

4.6.2.9	WUS receptions for NB1	130

4.7	Cell Selection and Re-selection Requirements for UE category M1	131

4.7.1	Cell Selection	131

4.7.2	Cell Re-selection	131

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	131

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	131

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	131

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	132

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	134

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	134

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	135

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	135

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	136

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	137

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	139

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	139

4.8	Idle State Positioning Measurement Requirements for UE category NB1	140

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	140

4.8.1.1	RSTD Measurement Reporting Delay	141

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	142

4.8.2.1	RSTD Measurement Reporting Delay	143

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	143

4.8.3.1	RSTD Measurement Reporting Delay	145

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	145

4.8.4.1	RSTD Measurement Reporting Delay	146

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	147

4.8.5.1	Measurement Reporting Delay	148

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	148

4.8.6.1	Measurement Reporting Delay	149

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	149

4.8.7.1	Measurement Reporting Delay	150

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	151

4.8.8.1	Measurement Reporting Delay	152

4A	E-UTRAN RRC_INACTIVE state mobility	153

4A.1	Cell Re-selection	153

4A.1.1	Introduction	153

4A.1.2	Requirements	153

4A.1.2.1	UE measurement capability	153

4A.1.2.2	Measurement and evaluation of serving cell	153

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	153

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	153

4A.1.2.5	Evaluation of cell re-selection criteria	153

4A.1.2.6	Maximum interruption in paging reception	153

5	E-UTRAN RRC_CONNECTED state mobility	153

5.1	E-UTRAN Handover	154

5.1.1	Introduction	154

5.1.2	Requirements	154

5.1.2.1	E-UTRAN FDD – FDD	154

5.1.2.1.1	Handover delay	154

5.1.2.1.2	Interruption time	154

5.2.2.2	E-UTRAN FDD – TDD	156

5.2.2.2.1	(Void)	156

5.2.2.2.2	(Void)	156

5.2.2.3	E-UTRAN TDD – FDD	156

5.2.2.3.1	(Void)	156

5.2.2.3.2	(Void)	156

5.2.2.4	E-UTRAN TDD – TDD	156

5.2.2.4.1	Handover delay	156

5.2.2.4.2	Interruption time	156

5.2.2.5	E-UTRAN HD–FDD	158

5.2.2.5.1	Handover delay	158

5.2.2.5.2	Interruption time	158

5.3	Handover to other RATs	159

5.3.1	E-UTRAN - UTRAN FDD Handover	159

5.3.1.1	Introduction	159

5.3.1.1.1	Handover delay	160

5.3.1.1.2	Interruption time	160

5.3.2	E-UTRAN - UTRAN TDD Handover	160

5.3.2.1	Introduction	160

5.3.2.2	Requirements	160

5.3.2.2.1	Handover delay	161

5.3.2.2.2	Interruption time	161

5.3.3	E-UTRAN - GSM Handover	161

5.3.3.1	Introduction	161

5.3.3.2	Requirements	161

5.3.3.2.1	Handover delay	161

5.3.3.2.2	Interruption time	162

5.3.4	E-UTRAN - NR FR1 Handover	162

5.3.4.1	Introduction	162

5.3.4.2	Handover delay	162

5.3.4.3	Interruption time	162

5.3.5	E-UTRAN - NR FR2 Handover	163

5.3.5.1	Introduction	163

5.3.5.2	Handover delay	163

5.3.5.3	Interruption time	163

5.4	Handover to Non-3GPP RATs	164

5.4.1	E-UTRAN – HRPD Handover	164

5.4.1.1	Introduction	164

5.4.1.1.1	Handover delay	164

5.4.1.1.2	Interruption time	164

5.4.2	E-UTRAN – cdma2000 1X Handover	164

5.4.2.1	Introduction	164

5.4.2.1.1	Handover delay	164

5.4.2.1.2	Interruption time	165

5.5	E-UTRAN Handover for Cat-M1 UEs	165

5.5.1	Introduction	165

5.5.2	Requirements in CEModeA	165

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	165

5.5.2.1.1	Handover delay	165

5.5.2.1.2	Interruption time	165

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	166

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	166

5.5.2.3.1	Void	166

5.5.2.3.2	Void	166

5.5.3	Requirements in CEModeB	166

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	166

5.5.3.1.1	Handover delay	166

5.5.3.1.2	Interruption time	167

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	167

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	167

5.6	Void	167

6	RRC Connection Mobility Control	167

6.1	RRC Re-establishment	167

6.1.1	Introduction	168

6.1.2	Requirements	168

6.1.2.1	UE Re-establishment delay requirement	168

6.2	Random Access	168

6.2.1	Introduction	168

6.2.2	Requirements	168

6.2.2.1	Contention based random access	169

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	169

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	169

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	169

6.2.2.1.4	Void	169

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	169

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	169

6.2.2.2	Non-Contention based random access	169

6.2.2.2.1	Correct behaviour when receiving Random Access Response	169

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	169

6.2.3	Requirements for Cat-M1 UEs	170

6.3	RRC Connection Release with Redirection	170

6.3.1	Introduction	170

6.3.2	Requirements	170

6.3.2.1	RRC connection release with redirection to UTRAN FDD	170

6.3.2.2	RRC connection release with redirection to GERAN	170

6.3.2.3	RRC connection release with redirection to UTRAN TDD	171

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	171

6.4.1	Introduction	171

6.4.2	Requirements	171

6.5	RRC Re-establishment for NB-IoT UEs	172

6.5.1	Introduction	172

6.5.2	Requirements	172

6.5.2.1	UE Re-establishment delay requirement in normal coverage	172

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	173

6.6	Random Access for UE category NB1	173

6.6.1	Introduction	173

6.6.2	Requirements	173

6.6.2.1	Correct behaviour when receiving Random Access Response reception	173

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	174

6.6.2.3	Correct behaviour when receiving a NACK on msg3	174

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	174

6.6.2.5	Correct behaviour when contention Resolution timer expires	174

6.6.3	Requirements for NPRACH configuration	174

6.7	RRC Re-establishment for Cat-M1 UEs	174

6.7.1	Introduction	174

6.7.2	Requirements	174

6.7.2.1	UE Re-establishment delay requirement for CEModeA	175

6.7.2.2	UE Re-establishment delay requirement for CEModeB	175

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	176

6.8.1	Introduction	176

6.8.2	Requirements	176

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	176

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	176

6.9.1	Introduction	176

6.9.2	Requirements	177

7	Timing and signalling characteristics	177

7.1	UE transmit timing	177

7.1.1	Introduction	177

7.1.2	Requirements	178

7.2	UE timer accuracy	179

7.2.1	Introduction	179

7.2.2	Requirements	179

7.3	Timing Advance	179

7.3.1	Introduction	179

7.3.2	Requirements	179

7.3.2.1	Timing Advance adjustment delay	179

7.3.2.2	Timing Advance adjustment accuracy	180

7.4	Cell phase synchronization accuracy (TDD)	180

7.4.1	Definition	180

7.4.2	Minimum requirements	180

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	181

7.5.1	Introduction	181

7.5.2	eNodeB Synchronization Requirements	181

7.5.2.1	Synchronized E-UTRAN	181

7.5.2.2	Non-Synchronized E-UTRAN	181

7.6	Radio Link Monitoring	181

7.6.1	Introduction	181

7.6.2	Requirements	183

7.6.2.1	Minimum requirement when no DRX is used	183

7.6.2.2	Minimum requirement when DRX is used	184

7.6.2.3	Minimum requirement at transitions	185

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	185

7.6.2.5	Minimum requirement under IDC Interference	186

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	186

7.7.1	Introduction	186

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	186

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	187

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	188

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	189

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	190

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	191

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	191

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	191

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	192

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	193

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	194

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	195

7.8	Interruptions with Carrier Aggregation	195

7.8.1	Introduction	195

7.8.2	Requirements	196

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	196

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	196

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	196

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	196

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	196

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	196

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	197

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	197

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	197

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	198

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	198

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	199

7.8.2.13	Interruptions at SRS carrier based switching	199

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	200

7.9.1	Introduction	200

7.9.2	Minimum Requirements for Interband Carrier Aggregation	200

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	201

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	201

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	201

7.10.1	Introduction	201

7.10.2	Requirements	201

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	201

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	202

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	202

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	202

7.11	Radio Link Monitoring for UE Category 0	203

7.11.1	Introduction	203

7.11.2	Requirements for FD-FDD and TDD	204

7.11.2.1	Minimum requirement when no DRX is used	204

7.11.2.2	Minimum requirement when DRX is used	205

7.11.2.3	Minimum requirement at transitions	205

7.11.3	Requirements for HD-FDD	206

7.11.3.1	Minimum requirement when no DRX is used	206

7.11.3.2	Minimum requirement when DRX is used	206

7.11.3.3	Minimum requirement at transitions	207

7.12	Interruptions with Dual Connectivity	207

7.12.1	Introduction	207

7.12.2	Requirements	207

7.12.2.1	Interruptions at PSCell addition/release	207

7.12.2.2	Interruptions at transitions between active and non-active during DRX	207

7.12.2.3	Interruptions at transitions from non-DRX to DRX	208

7.12.2.4	Interruptions at SCell addition/release	208

7.12.2.5	Interruptions at SCell activation/deactivation	208

7.12.2.6	Interruptions during measurements on SCC	209

7.12.2.7	Interruptions at SRS carrier based switching	209

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	210

7.13.1	Definition	210

7.13.2	Minimum requirements	210

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	210

7.14.1	Introduction	210

7.14.2	PSCell Addition Delay Requirement	210

7.14.3	PSCell Release Delay Requirement	211

7.15	Maximum Receive Timing Difference in Dual Connectivity	211

7.15.1	Introduction	211

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	211

7.16	Proximity-based Services	211

7.16.1	Introduction	211

7.16.2	Requirements	211

7.16.2.1	ProSe UE transmission timing	211

7.16.2.1.1	Serving cell or PCell as timing reference	212

7.16.2.1.2	SCell or non-serving cell as timing reference	212

7.16.3	Interruptions with ProSe	212

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	212

7.16.3.2	Interruptions at ProSe Direct Communication configuration	213

7.16.3.3	Interruptions during ProSe Direct Discovery	213

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	213

7.16.3.5	Interruptions during ProSe Direct Communication	214

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	214

7.16.4.1	Measurement and evaluation of selected cell	214

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	214

7.16.5	Selection / Reselection of ProSe relay UE	215

7.16.6	ProSe operation under deactivated SCell	215

7.17	Maximum Transmission Timing Difference in Dual Connectivity	215

7.17.1	Introduction	215

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	216

7.18.1	Introduction	216

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	216

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	216

7.19	Radio Link Monitoring for UE Category M1	216

7.19.1	Introduction	216

7.19.2	Requirements for FD-FDD and TDD CE mode A	216

7.19.2.1	Minimum requirement when no DRX is used	217

7.19.2.2	Minimum requirement when DRX is used	218

7.19.2.3	Minimum requirement at transitions	219

7.19.3	Requirements for HD-FDD with CE mode A	219

7.19.3.1	Minimum requirement when no DRX is used	220

7.19.3.2	Minimum requirement when DRX is used	220

7.19.3.3	Minimum requirement at transitions	221

7.19.4	Requirements for FD-FDD and TDD with CE mode B	221

7.19.4.1	Minimum requirement when no DRX is used	222

7.19.4.2	Minimum requirement when DRX is used	223

7.19.4.3	Minimum requirement at transitions	224

7.19.5	Requirements for HD-FDD with CE mode B	224

7.19.5.1	Minimum requirement when no DRX is used	225

7.19.5.2	Minimum requirement when DRX is used	225

7.19.5.3	Minimum requirement at transitions	226

7.20	UE transmit timing for NB-IoT	226

7.20.1	Introduction	226

7.20.2	Requirements	226

7.21	 UE timer accuracy for NB-IoT	227

7.21.1	Introduction	227

7.21.2	Requirements	227

7.22	Timing Advance for NB-IoT	227

7.22.1	Introduction	227

7.22.2	Requirements	227

7.22.2.1	Timing Advance adjustment delay	227

7.22.2.2	Timing Advance adjustment accuracy	228

7.23	Radio Link Monitoring for Category NB1 UE	228

7.23.1	Introduction	228

7.23.2	Requirements for Category NB1 UE	228

7.23.2.1	Minimum requirement when no DRX is used	228

7.23.2.2	Minimum requirement when DRX is used	229

7.23.2.3	Minimum requirement at transitions	229

7.24	UE transmit timing for Category M1	230

7.24.1	Introduction	230

7.24.2	Requirements	230

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	231

7.25.1	Definition	231

7.25.2	Minimum requirements	231

7.26	UE transmit timing for Category M2	231

7.26.1	Introduction	231

7.26.2	Requirements	231

7.27	UE timer accuracy for category M1	232

7.27.1	Introduction	232

7.27.2	Requirements	232

7.28	Timing Advance for Category M1	232

7.28.1	Introduction	232

7.28.2	Requirements	232

7.29	Interruptions requirements with FeMBMS	232

7.29.1	Introduction	232

7.29.2	Requirements	232

7.30	Numerology switching delay requirements with FeMBMS	232

7.30.1	Introduction	232

7.30.2	Requirements	232

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	233

7.31.1	Introduction	233

7.31.2	NR PSCell Addition Delay Requirement	233

7.31.3	NR PSCell Release Delay Requirement	234

7.32	Interruptions with EN-DC	234

7.32.1	Introduction	234

7.32.2	Requirements	234

7.32.2.1	Interruptions at PSCell addition/release	234

7.32.2.2	Interruptions at transitions between active and non-active during DRX	234

7.32.2.3	Interruptions at transitions from non-DRX to DRX	235

7.32.2.4	Interruptions at SCell addition/release	235

7.32.2.5	Interruptions at SCell activation/deactivation	235

7.32.2.6	Interruptions during measurements on SCC	235

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	236

7.33.1	Introduction	236

7.33.2	Requirements	236

7.34	Void	236

7.35	Interruptions with SFTD measurements	236

7.35.1	Introduction	236

7.35.2	Requirements	236

8	UE Measurements Procedures in RRC_CONNECTED State	237

8.1	General Measurement Requirements	237

8.1.1	Introduction	237

8.1.2	Requirements	237

8.1.2.1	UE measurement capability	237

8.1.2.1.1	Monitoring of multiple layers using gaps	242

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	244

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	245

8.1.2.1.2	Network controlled small gap	246

8.1.2.2	E-UTRAN intra frequency measurements	248

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	248

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	252

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	256

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	257

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	258

8.1.2.3	E-UTRAN inter frequency measurements	258

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	259

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	264

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	271

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	271

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	271

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	272

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	274

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	275

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	276

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	283

8.1.2.4	Inter RAT measurements	283

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	283

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	288

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	288

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	292

8.1.2.4.5	E-UTRAN FDD – GSM measurements	292

8.1.2.4.6	E-UTRAN TDD – GSM measurements	297

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	297

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	299

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	299

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	299

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	300

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	300

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	300

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	300

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	302

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	302

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	302

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	302

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	303

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	303

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	305

8.1.2.4.21	E-UTRAN FDD – NR measurements	305

8.1.2.4.22	E-UTRAN TDD – NR measurements	307

8.1.2.4.23	Void	307

8.1.2.4.24	Void	307

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	307

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	309

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	309

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	309

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	311

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	312

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	314

8.1.2.6.5	Void	316

8.1.2.6.6	Void	316

8.1.2.6.7	Void	316

8.1.2.6.8	Void	316

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	316

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	316

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	318

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	320

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	321

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	323

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	325

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	326

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	328

8.1.2.7	E-UTRAN E-CID Measurements	330

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	330

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	331

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	333

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	333

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	334

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	334

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	337

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	340

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	343

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	347

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	347

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	347

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	347

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	348

8.1.2.10	Void	348

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	348

8.2.1	Introduction	348

8.2.2	Requirements	349

8.3	Measurements for E-UTRA carrier aggregation	352

8.3.1	Introduction	352

8.3.2	Measurements of the primary component carrier	352

8.3.3	Measurements of a secondary component carrier	352

8.3.3.1	Measurements of a secondary component carrier with active SCell	352

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	353

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	353

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	354

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	355

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	355

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	356

8.4.1	Introduction	356

8.4.2	Measurements on the primary component carrier	356

8.4.3	Measurements on a secondary component carrier	357

8.4.4	Measurements on both primary component carrier and a secondary component carrier	358

8.4.5	Measurements on different secondary component carriers	359

8.5	Measurements for UE category 0	360

8.5.1	Introduction	360

8.5.2 	Requirements	360

8.5.2.1 	E-UTRAN intra frequency measurements	360

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	360

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	363

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	365

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	369

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	370

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	370

8.6	Discovery signal measurements	371

8.6.1	Introduction	371

8.6.2	Requirements for CRS based discovery signal measurements	371

8.6.2.1	E-UTRAN intra frequency measurements	371

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	371

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	374

8.6.2.2	E-UTRAN inter frequency measurements	376

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	377

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	379

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	382

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	382

8.6.3	Requirements for CSI-RS based discovery signal measurements	382

8.6.3.1	E-UTRAN intra frequency measurements	382

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	383

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	385

8.6.3.2	E-UTRAN inter frequency measurements	387

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	388

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	390

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	393

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	393

8.7	Discovery signal measurements for E-UTRA carrier aggregation	393

8.7.1	Introduction	393

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	394

8.7.2.1	Measurements of the primary component carrier	394

8.7.2.2	Measurements of a secondary component carrier	394

8.7.2.3	Measurements of a secondary component carrier with active SCell	394

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	394

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	394

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	395

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	397

8.7.3.1	Measurements of the primary component carrier	397

8.7.3.2	Measurements of a secondary component carrier	397

8.7.3.3	Measurements of a secondary component carrier with active SCell	397

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	397

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	397

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	399

8.8	Measurements for E-UTRA dual connectivity	400

8.8.1	Introduction	400

8.8.2	Intra-frequency measurements requirements on PCell	400

8.8.3	Intra-frequency measurements requirements on PSCell	400

8.8.4	Inter-frequency and inter-RAT measurement requirements	401

8.8.5	Intra-frequency measurements with autonomous gaps	401

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	401

8.8.5.2	ECGI reporting delay	402

8.8.6	Inter-frequency measurements with autonomous gaps	402

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	402

8.8.6.2	ECGI reporting delay	403

8.8.7	SSTD Measurements	403

8.8.7.1	Introduction	403

8.8.7.2	SSTD Measurement requirements	403

8.8.7.3	SSTD Measurement Reporting Delay	404

8.8.8	Intra-frequency measurements requirements on SCell	404

8.9	MBSFN Measurements	404

8.9.1	Introduction	404

8.9.2	MBSFN RSRP Measurements	404

8.9.3	MBSFN RSRQ Measurements	404

8.9.4	MCH BLER Measurements	405

8.10	Proximity-based Services	405

8.10.1	Introduction	405

8.10.2	Requirements	405

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	405

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	406

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	406

8.11.1	Introduction	406

8.11.2	CRS based discovery signal measurements	406

8.11.2.1	E-UTRAN intra-frequency measurements	406

8.11.2.1.1	Requirements	407

8.11.2.1.1.1	Requirements when no DRX is used	407

8.11.2.1.1.1.1	Measurement Reporting Requirements	408

8.11.2.1.1.2	Requirements when DRX is used	409

8.11.2.1.1.2.1	Measurement Reporting Requirements	411

8.11.2.2	E-UTRAN inter-frequency measurements	412

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	412

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	416

8.11.3	CSI-RS based discovery signal measurements	416

8.11.3.1	E-UTRAN intra-frequency measurements	416

8.11.3.1.1	Requirements	416

8.11.3.1.1.1	Requirements when no DRX is used	416

8.11.3.1.1.1.1	Measurement Reporting Requirements	418

8.11.3.1.1.2	Requirements when DRX is used	418

8.11.3.1.1.2.1	Measurement Reporting Requirements	419

8.11.3.2	E-UTRAN inter-frequency measurements	420

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	420

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	424

8.11.4	RSSI measurements	424

8.11.4.1	E-UTRAN intra-frequency measurements	424

8.11.4.2	E-UTRAN inter-frequency measurements	424

8.11.5	Channel occupancy measurements	424

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	424

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	425

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	425

8.12.1	Introduction	425

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	425

8.12.2.1	Introduction	425

8.12.2.2	Measurements of a secondary component carrier	425

8.12.2.3	Measurements of a secondary component carrier with active SCell	425

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	425

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	426

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	428

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	430

8.12.3.1	Introduction	430

8.12.3.2	Measurements of a secondary component carrier	430

8.12.3.3	Measurements of a secondary component carrier with active SCell	430

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	430

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	430

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	432

8.13	Measurements for UE Category M1	434

8.13.1	Introduction	434

8.13.2	Requirements for UE category M1 with CE mode A	434

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	435

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	435

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	439

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	441

8.13.2.2	Void	445

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	445

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	445

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	447

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	450

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	450

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	451

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	453

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	455

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	456

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	456

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	456

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	456

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	457

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	457

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	458

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	458

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	459

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	460

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	460

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	460

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	465

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	467

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	471

8.13.3	Requirements for UE category M1 with CE mode B	471

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	471

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	471

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	476

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	478

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	484

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	485

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	485

8.13.3.2	Void	486

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	486

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	486

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	489

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	491

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	492

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	492

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	492

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	492

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	493

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	493

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	494

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	494

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	494

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	498

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	501

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	505

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	506

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	506

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	508

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	510

8.14	Measurements for UE category NB1	511

8.14.1	Introduction	511

8.14.2	NB-IoT intra frequency measurements under normal coverage	511

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	511

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	511

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	511

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	511

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	511

8.15	Void	512

8.16	Measurements for UE Category M2	512

8.16.1	Introduction	512

8.16.2	Requirements for UE category M2 with CE mode A	512

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	512

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	513

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	513

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	514

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	514

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	514

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	514

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	515

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	517

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	520

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	520

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	520

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	522

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	525

8.16.3	Requirements for UE category M2 with CE mode B	525

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	525

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	525

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	528

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	530

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	531

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	531

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	533

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	535

8.17	Measurements for E-UTRA – NR Dual Connectivity	536

8.17.1	Introduction	536

8.17.2	SFTD Measurements	536

8.17.2.1	Introduction	536

8.17.2.2	SFTD Measurement requirements	536

8.17.2.3	SFTD Measurement Reporting Delay	537

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	537

8.17.3.1	Introduction	537

8.17.3.2	E-UTRAN FDD inter frequency measurements	537

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	537

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	539

8.17.3.3	E-UTRAN TDD inter frequency measurements	541

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	541

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	542

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	544

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	544

8.17.4.1.1	NR Inter-RAT cell identification	545

8.17.4.1.2	NR Inter-RAT measurement	545

8.17.4.1.3	NR Inter-RAT measurement reporting	545

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	545

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	546

8.17.5.1	Introduction	546

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	546

8.17.5.2.1	Identification of a new UTRA FDD cell	546

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	546

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	546

8.17.5.2.4	Periodic Reporting	547

8.17.5.2.5	Event Triggered Reporting	547

8.17.5.2.6	Event-triggered Periodic Reporting	547

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	547

8.17.5.3.1	Periodic Reporting	549

8.17.5.3.2	Event Triggered Reporting	549

8.17.5.3.3	Event-triggered Periodic Reporting	549

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	549

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	549

8.17.7.1	Introduction	549

8.17.7.2	Identification of a new UTRA FDD cell for SON	550

8.17.7.2.1	Requirements when no DRX is used	550

8.17.7.2.2	Requirements when DRX is used	550

8.17.7.2.3	Reporting Delay	551

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	551

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	551

8.17.9.1	Introduction	551

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	552

8.17.9.2.1	Identification of a new UTRA FDD cell	552

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	552

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	552

8.17.9.2.4	Periodic Reporting	553

8.17.9.2.5	Event Triggered Reporting	553

8.17.9.2.6	Event-triggered Periodic Reporting	553

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	553

8.17.9.3.1	Periodic Reporting	554

8.17.9.3.2	Event Triggered Reporting	554

8.17.9.3.3	Event-triggered Periodic Reporting	555

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	555

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	555

8.17.11.1	Introduction	555

8.17.11.2	Identification of a new UTRA TDD cell for SON	555

8.17.11.2.1	Requirements when no DRX is used	555

8.17.11.2.2	Requirements when DRX is used	556

8.17.11.2.3	Reporting Delay	556

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	557

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	557

8.17.13.1	Introduction	557

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	557

8.17.13.2.1	GSM carrier RSSI	557

8.17.13.2.2	BSIC verification	557

8.17.13.2.3	Enhanced BSIC verification	559

8.17.13.2.4	Periodic Reporting	559

8.17.13.2.5	Event Triggered Reporting	559

8.17.13.2.6	Event-triggered Periodic Reporting	560

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	560

8.17.13.3.1	GSM carrier RSSI	560

8.17.13.3.2	BSIC verification	560

8.17.13.3.3	Periodic Reporting	562

8.17.13.3.4	Event Triggered Reporting	562

8.17.13.3.5	Event-triggered Periodic Reporting	562

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	562

9	Measurements performance requirements for UE	562

9.1	E-UTRAN measurements	562

9.1.1	Introduction	562

9.1.2	Intra-frequency RSRP Accuracy Requirements	563

9.1.2.1	Absolute RSRP Accuracy	563

9.1.2.2	Relative Accuracy of RSRP	563

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	564

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	565

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	566

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	567

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	568

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	569

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	570

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	570

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	570

9.1.3	Inter-frequency RSRP Accuracy Requirements	571

9.1.3.1	Absolute RSRP Accuracy	571

9.1.3.2	Relative Accuracy of RSRP	572

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	572

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	573

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	574

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	574

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	574

9.1.4	RSRP Measurement Report Mapping	575

9.1.5	Intra-frequency RSRQ Accuracy Requirements	575

9.1.5.1	Absolute RSRQ Accuracy	575

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	576

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	577

9.1.5.4	Absolute WB-RSRQ Accuracy	578

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	579

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	580

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	580

9.1.6	Inter-frequency RSRQ Accuracy Requirements	580

9.1.6.1	Absolute RSRQ Accuracy	580

9.1.6.2	Relative Accuracy of RSRQ	581

9.1.6.3	Absolute WB-RSRQ Accuracy	582

9.1.6.4	Relative WB-RSRQ Accuracy	582

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	583

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	584

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	584

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	584

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	585

9.1.7	RSRQ Measurement Report Mapping	586

9.1.8	Power Headroom	586

9.1.8.1	Period	586

9.1.8.2	Reporting Delay	586

9.1.8.3	Void	587

9.1.8.4	Report Mapping	587

9.1.9	UE Rx – Tx time difference	587

9.1.9.1	Measurement Requirement	587

9.1.9.2	Measurement Report mapping	588

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	588

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	589

9.1.10	Reference Signal Time Difference (RSTD)	590

9.1.10.1	Intra-Frequency Accuracy Requirement	590

9.1.10.2	Inter-Frequency Accuracy Requirement	592

9.1.10.3	RSTD Measurement Report Mapping	593

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	594

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	594

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	595

9.1.11	Carrier aggregation measurement accuracy	596

9.1.11.1	Primary component carrier accuracy requirement	597

9.1.11.2	Secondary component carrier accuracy requirement	597

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	597

9.1.11.4	Secondary component carrier relative accuracy requirement	597

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	597

9.1.13	Measurement accuracy for UE category 0	598

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	598

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	598

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	599

9.1.14	Accuracy requirements for Discovery Signal Measurements	600

9.1.14.1	Introduction	600

9.1.14.2	RSRP measurements in discovery signal occasions	600

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	600

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	600

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	600

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	601

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	602

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	602

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	603

9.1.14.3.3	CSI-RSRP measurement report mapping	604

9.1.14.4	RSRQ measurements in discovery signal occasions	604

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	604

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	604

9.1.15.1.1	Primary component carrier accuracy requirement	604

9.1.15.1.2	Secondary component carrier accuracy requirement	604

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	605

9.1.15.1.4	Secondary component carrier relative accuracy requirement	605

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	605

9.1.15.2.1	Primary component carrier accuracy requirement	605

9.1.15.2.2	Secondary component carrier accuracy requirement	605

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	605

9.1.15.2.4	Secondary component carrier relative accuracy requirement	605

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	605

9.1.17	RS-SINR Measurements	606

9.1.17.1	Measurement Report Mapping	606

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	606

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	606

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	607

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	607

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	608

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	608

9.1.18.1	Introduction	608

9.1.18.2	RSRP measurements	608

9.1.18.2.1	RSRP measurement report mapping	608

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	608

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	609

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	609

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	610

9.1.18.3	RSRQ measurements	610

9.1.18.3.1	RSRQ measurement report mapping	610

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	611

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	611

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	612

9.1.18.4	CSI-RSRP measurements	612

9.1.18.4.1	CSI-RSRP measurement report mapping	612

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	612

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	613

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	613

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	614

9.1.18.5	RSSI measurements	614

9.1.18.5.1	RSSI measurement report mapping	614

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	615

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	615

9.1.18.6	Channel occupancy measurements	615

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	615

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	616

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	616

9.1.19.1	Introduction	616

9.1.19.2	Accuracy requirements for measurements on SCC	616

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	616

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	617

9.1.20	SFN and Subframe Time Difference (SSTD)	617

9.1.20.1	SSTD Accuracy Requirement	617

9.1.20.2	SSTD Measurement Report Mapping	618

9.1.21	Measurement accuracy for UE category M1	619

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	619

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	620

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	621

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	622

9.1.21.5	RSRP Measurement Report Mapping	623

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	623

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	624

9.1.21.8	RSRQ Measurement Report Mapping	625

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	625

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	626

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	627

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	628

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	630

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	630

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	631

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	632

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	634

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	634

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	635

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	636

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	638

9.1.22	Measurement accuracy for UE Category NB1	640

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	640

9.1.22.2	Void	640

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	640

9.1.22.4	Void	641

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	641

9.1.22.6	Void	641

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	641

9.1.22.8	Void	642

9.1.22.9	NRSRP Measurement Report Mapping	642

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	642

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	643

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	644

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	645

9.1.22.14	NRSRQ Measurement Report Mapping	646

9.1.23	Power Headroom for UE Category NB1	646

9.1.23.1	Period	647

9.1.23.2	Reporting Delay	647

9.1.23.3	Report Mapping for UE Category NB1	647

9.1.23.3.1	Void	647

9.1.23.3.2	Void	647

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	648

9.1.24	Void	648

9.1.25	Measurement accuracy for UE category M2	648

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	648

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	649

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	650

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	651

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	653

9.1.26	Measurement Accuracy for non-BL CE UE	655

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	655

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	656

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	656

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	657

9.1.26.5	RSRP Measurement Report Mapping	658

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	658

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	658

9.1.26.8	RSRQ Measurement Report Mapping	659

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	659

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	660

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	661

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	662

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	663

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	663

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	663

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	664

9.2	UTRAN FDD Measurements	665

9.2.1	UTRAN FDD CPICH RSCP	666

9.2.2	Void	666

9.2.3	UTRAN FDD CPICH Ec/No	666

9.3	UTRAN TDD Measurements	667

9.3.1	UTRAN TDD P-CCPCH RSCP	667

9.3.2	Void	667

9.3.3	Void	667

9.4	GSM Measurements	667

9.4.1	GSM carrier RSSI	667

9.5	CDMA2000 1x RTT Measurements	668

9.5.1	CDMA2000 1x RTT Pilot Strength	668

9.6	PCMAX,c	668

9.6.1	Report Mapping	668

9.6.2 	Estimation Period	668

9.6.3	Reporting Delay	668

9.7	IEEE802.11 Measurements	668

9.7.1	WLAN RSSI	669

9.7.2	WLAN RSSI Measurement Report Mapping	669

9.8	MBSFN Measurements	669

9.8.1	Introduction	669

9.8.2	MBSFN RSRP	669

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	669

9.8.2.2	MBSFN RSRP measurement report mapping	670

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	670

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	671

9.8.3	MBSFN RSRQ	671

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	671

9.8.3.2	MBSFN RSRQ measurement report mapping	672

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	672

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	673

9.8.4	MCH BLER	673

9.8.4.1	Measurement report mapping for MCH BLER	673

9.8.4.2	Measurement report mapping for MCH Block Number	674

9.9	ProSe Measurements	675

9.9.1	Introduction	675

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	675

9.9.2.1	Absolute S-RSRP Accuracy	675

9.9.2.2	Relative Accuracy of S-RSRP	676

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	677

9.9.3.1	Absolute SD-RSRP Accuracy	677

9.9.3.2	Relative Accuracy of SD-RSRP	677

9.10	V2X Measurements	678

9.10.1	Introduction	678

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	678

9.10.2.1	Absolute S-RSRP Accuracy	678

9.10.2.2	Relative Accuracy of S-RSRP	679

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	679

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	679

9.10.4	S-RSSI Measurement Accuracy Requirements	680

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	680

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	680

10	Measurements Performance Requirements for E-UTRAN	681

10.1	Received Interference Power	681

10.1.1	Absolute accuracy requirement	681

10.1.2	Relative accuracy requirement	681

10.1.3	Received Interference Power measurement report mapping	681

10.2	Angle of Arrival (AOA)	682

10.2.1	Range/mapping	682

10.3	Timing Advance (TADV)	682

10.3.1	Report mapping	682

11	ProSe Requirements in Any Cell Selection state	683

11.1	Introduction	683

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	683

11.2.1	Introduction	683

11.2.2	ProSe UE transmission timing	683

11.3	Initiation/Cease of SLSS Transmissions	683

11.3.1	Introduction	683

11.3.2	Requirements	683

11.4	Measurements for ProSe in Any Cell Selection State	684

11.4.1	Introduction	684

11.4.2	Requirements	684

11.4.2.1	E-UTRA FDD	684

11.4.2.2	E-UTRA TDD	684

11.5	Selection / Reselection of ProSe Synchronization Reference	685

11.5.1	Introduction	685

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	685

11.5.2.1	Introduction	685

11.5.2.2	Requirements	685

11.6	Void	685

11.7	Selection / Reselection of ProSe relay UE	685

11.7.1	Introduction	685

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	685

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	686

12.1	Introduction	686

12.2	Transmit Timing	686

12.2.1	GNSS as timing reference	686

12.3	Interruption	686

12.4	Reliability of GNSS signal	687

13	V2X Requirements	687

13.1	Introduction	687

13.2	UE Transmit Timing	687

13.2.1	Introduction	687

13.2.2	GNSS as synchronization reference source	687

13.2.3	Serving cell/PCell as synchronization reference source	688

13.2.4	SyncRef UE as synchronization reference source	688

13.3	Initiation/Cease of SLSS Transmissions	688

13.3.1	Introduction	688

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	688

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	689

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	689

13.4	Selection / Reselection of V2X Synchronization Reference Source	689

13.5	Autonomous Resource Selection/Reselection measurements	690

13.5.1	Introduction	690

13.5.2	PSSCH-RSRP measurements	690

13.5.3	S-RSSI measurements	690

13.6	Congestion Control measurements	691

13.7	Interruption	691

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	691

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	691

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	692

13.8	Reliability of GNSS signal	692

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	692

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	692

Annex A (normative):	Test Cases	694

A.1	Purpose of annex	694

A.2	Requirement classification for statistical testing	694

A.2.1	Types of requirements in TS 36.133	694

A.2.1.1	Time and delay requirements on UE higher layer actions	694

A.2.1.2	Measurements of power levels, relative powers and time	694

A.2.1.3	Implementation requirements	695

A.2.1.4	Physical layer timing requirements	695

A.3	RRM test configurations	696

A.3.1	Reference Measurement Channels	696

A.3.1.1	PDSCH	696

A.3.1.1.1	FDD	696

A.3.1.1.2	TDD	700

A.3.1.1.3	FDD for UE category 0	703

A.3.1.1.4	HD-FDD for UE category 0	704

A.3.1.1.5	TDD for UE category 0	705

A.3.1.1.6	Frame Structure 3	706

A.3.1.2	PCFICH/PDCCH/PHICH	707

A.3.1.2.1	FDD	707

A.3.1.2.2	TDD	707

A.3.1.2.3	HD-FDD for UE category 0	708

A.3.1.2.4	FS 3	708

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	708

A.3.1.3.1	FDD in CEModeA	709

A.3.1.3.2	HD-FDD in CEModeA	709

A.3.1.3.3	TDD in CEModeA	710

A.3.1.3.4	FDD in CEModeB	710

A.3.1.3.5	HD-FDD in CEModeB	711

A.3.1.3.6	TDD in CEModeB	711

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	712

A.3.1.4.1	FDD in CEModeA	712

A.3.1.4.2	HD-FDD in CEModeA	713

A.3.1.4.3	TDD in CEModeA	714

A.3.1.4.4	FDD in CEModeB	715

A.3.1.4.5	 HD-FDD in CEModeB	716

A.3.1.4.6	TDD in CEModeB	717

A.3.1.5	NPDSCH Reference Channel for UE category NB1	718

A.3.1.5.1	HD-FDD in-band operation	718

A.3.1.5.2	Void	719

A.3.1.5.3	HD-FDD standalone operation	719

A.3.1.5.4	Void	720

A.3.1.5.5	HD-FDD guard band operation	720

A.3.1.5.6	Void	720

A.3.1.6	NPDCCH Reference Channel for UE category NB1	720

A.3.1.6.1	HD-FDD in-band operation	720

A.3.1.6.2	Void	721

A.3.1.6.3	HD-FDD standalone operation	721

A.3.1.6.4	Void	721

A.3.1.6.5	HD-FDD guard band operation	721

A.3.1.6.6	Void	722

A.3.2	OFDMA Channel Noise Generator (OCNG)	722

A.3.2.1	OCNG Patterns for FDD	722

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	723

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	723

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	724

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	725

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	725

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	726

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	726

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	726

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	727

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	728

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	728

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	729

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	729

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	730

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	730

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	731

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	731

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	732

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	732

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	733

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	733

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	734

A.3.2.2	OCNG Patterns for TDD	734

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	735

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	735

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	736

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	736

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	737

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	738

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	739

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	740

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	740

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	741

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	742

A.3.2.3	OCNG Patterns for Narrowband IoT	742

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	744

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	745

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	745

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	746

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	747

A.3.2.4	OCNG Patterns for V2X sidelink	748

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	748

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	749

A.3.3	Reference DRX Configurations	749

A.3.4	ABS Transmission Configurations	750

A.3.4.1	Non-MBSFN ABS Transmission Configurations	750

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	750

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	750

A.3.4.2	MBSFN ABS Transmission Configurations	751

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	751

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	752

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	752

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	752

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	753

A.3.6.1	Introduction	753

A.3.7	Test Cases with Different Channel Bandwidths	753

A.3.7.1	Introduction	753

A.3.7.2	Principle of testing	753

A.3.8	Antenna Configuration	753

A.3.8.1	Antenna connection for 4 Rx capable UEs	753

A.3.8.1.1 Introduction	753

A.3.8.1.2 Principle of testing	753

A.3.8.1.2.1 Single carrier tests	753

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	754

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	755

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	755

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	755

A.3.9.1	Introduction	755

A.3.9.2	Principle of testing	755

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	756

A.3.10.1	Introduction	756

A.3.10.2	Principle of testing	756

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	756

A.3.11.1	Introduction	756

A.3.11.2	Principle of Testing	756

A.3.12	Proximity-based Services	756

A.3.12.1	Introduction	756

A.3.12.2	Reference DRX configurations for ProSe tests	756

A.3.12.3	Test Cases with Different Channel Bandwidths	757

A.3.12.3.1	Introduction	757

A.3.12.3.2	Principle of testing	757

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	757

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	763

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	765

A.3.12.6.1	FDD	765

A.3.12.7	Reference measurement channels for ProSe Direct Communication	765

A.3.12.7.1	FDD	765

A.3.12.8	ProSe Receive Traffic Generator	766

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	766

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	766

A.3.13	Time Offset between Cells	767

A.3.13.1	Introduction	767

A.3.13.2	Definition	767

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	767

A.3.14.1	Introduction	767

A.3.14.2	Principle of testing	767

A.3.15	Dual connectivity test cases with different combination of duplex mode	767

A.3.15.1	Introduction	767

A.3.15.2	Principle of testing	767

A.3.16	Reference PRACH Configurations	768

A.3.17	Listen before talk model	768

A.3.17.1	Introduction	768

A.3.17.2	Definition	768

A.3.18	Reference NPRACH Configurations	769

A.3.19	Dual connectivity test cases with different bandwidth combinations	770

A.3.19.1	Introduction	770

A.3.19.2	Principle of testing	770

A.3.20	Category M1 UE Test Cases	770

A.3.20.1	Introduction	770

A.3.20.2	Principle of Cat-M1 UE Testing	770

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	771

A.3.21.1	Introduction	771

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	772

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	773

A.3.22	Category 1bis UE Test Cases	774

A.3.22.1	Introduction	774

A.3.22.2	Principle of Category 1bis UE Testing	774

A.3.23	Category NB2 UE Test Cases	777

A.3.23.1	Introduction	777

A.3.23.2	Principle of Category NB2 UE Testing	777

A.3.24	V2X sidelink communication	777

A.3.24.1	Introduction	777

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	778

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	781

A.3.25	Category M2 UE Test Cases	782

A.3.25.1	Introduction	782

A.3.25.2	Principle of Cat-M2 UE Testing	782

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	783

A.3.26.1	Introduction	783

A.3.26.2	Principle of testing	784

A.4	E-UTRAN RRC_IDLE state	784

A.4.2	Cell Re-Selection	784

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	784

A.4.2.1.1	Test Purpose and Environment	784

A.4.2.1.2	Test Requirements	785

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	786

A.4.2.2.1	Test Purpose and Environment	786

A.4.2.2.2	Test Requirements	787

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	788

A.4.2.3.1	Test Purpose and Environment	788

A.4.2.3.2	Test Requirements	789

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	790

A.4.2.4.1	Test Purpose and Environment	790

A.4.2.4.2	Test Requirements	791

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	792

A.4.2.5.1	Test Purpose and Environment	792

A.4.2.5.2	Test Requirements	793

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	794

A.4.2.6.1	Test Purpose and Environment	794

A.4.2.6.2	Test Requirements	795

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	796

A.4.2.7.1	Test Purpose and Environment	796

A.4.2.7.2	Test Requirements	797

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	798

A.4.2.8.1	Test Purpose and Environment	798

A.4.2.8.2	Test Requirements	799

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	800

A.4.2.9.1	Test Purpose and Environment	800

A.4.2.9.2	Test Requirements	800

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	800

A.4.2.10.1	Test Purpose and Environment	800

A.4.2.10.2	Test Requirements	803

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	803

A.4.2.11.1	Test Purpose and Environment	803

A.4.2.11.2	Test Requirements	806

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	806

A.4.2.12.1	Test Purpose and Environment	806

A.4.2.12.2	Test Requirements	808

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	809

A.4.2.13.1	Test Purpose and Environment	809

A.4.2.13.2	Test Requirements	810

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	811

A.4.2.14.1	Test Purpose and Environment	811

A.4.2.14.2	Test Requirements	812

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	813

A.4.2.15.1	Test Purpose and Environment	813

A.4.2.15.2	Test Requirements	814

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	815

A.4.2.16.1	Test Purpose and Environment	815

A.4.2.16.2	Test Requirements	816

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	817

A.4.2.17.1	Test Purpose and Environment	817

A.4.2.17.2	Test Requirements	818

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	819

A.4.2.18.1	Test Purpose and Environment	819

A.4.2.18.2	Test Requirements	821

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	822

A.4.2.19.1	Test Purpose and Environment	822

A.4.2.19.2	Test Requirements	824

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	825

A.4.2.20.1	Test Purpose and Environment	825

A.4.2.20.2	Test Requirements	826

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	827

A.4.2.21.1	Test Purpose and Environment	827

A.4.2.21.2	Test Requirements	828

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	829

A.4.2.22.1	Test Purpose and Environment	829

A.4.2.22.2	Test Requirements	830

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	831

A.4.2.23.1	Test Purpose and Environment	831

A.4.2.23.2	Test Requirements	832

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	833

A.4.2.24.1	Test Purpose and Environment	833

A.4.2.24.2	Test Requirements	835

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	836

A.4.2.25.1	Test Purpose and Environment	836

A.4.2.25.2	Test Requirements	837

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	838

A.4.2.26.1	Test Purpose and Environment	838

A.4.2.26.2	Test Requirements	839

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	840

A.4.2.27.1	Test Purpose and Environment	840

A.4.2.27.2	Test Requirements	841

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	842

A.4.2.28.1	Test Purpose and Environment	842

A.4.2.28.2	Test Requirements	843

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	844

A.4.2.29.1	Test Purpose and Environment	844

A.4.2.29.2	Test Requirements	845

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	846

A.4.2.30.1	Test Purpose and Environment	846

A.4.2.30.2	Test Requirements	847

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	848

A.4.2.31.1	Test Purpose and Environment	848

A.4.2.31.2	Test Requirements	849

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	850

A.4.2.32.1	Test Purpose and Environment	850

A.4.2.32.2	Test Requirements	851

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	852

A.4.2.33.1	Test Purpose and Environment	852

A.4.2.33.2	Test Requirements	853

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	854

A.4.2.34.1	Test Purpose and Environment	854

A.4.2.34.2	Test Requirements	855

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	856

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	856

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	856

A.4.3.1.1.1	Test Purpose and Environment	856

A.4.3.1.1.2	Test Requirements	858

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	858

A.4.3.1.2.1	Test Purpose and Environment	858

A.4.3.1.2.2	Test Requirements	861

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	861

A.4.3.1.3.1	Test Purpose and Environment	861

A.4.3.1.3.2	Test Requirements	864

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	864

A.4.3.1.4.1	Test Purpose and Environment	864

A.4.3.1.4.2	Test Requirements	865

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	865

A.4.3.1.5.1	Test Purpose and Environment	865

A.4.3.1.5.2	Test Requirements	868

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	869

A.4.3.2.1	Test Purpose and Environment	869

A.4.3.2.1.1	Void	869

A.4.3.2.1.2	1.28Mcps TDD option	869

A.4.3.2.1.3	Void	871

A.4.3.2.2	Test Requirements	871

A.4.3.2.2.1	1.28Mcps TDD option	871

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	871

A.4.3.2A.1	Test Purpose and Environment	871

A.4.3.2A.2	Test Requirements	875

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	875

A.4.3.3.1	Test Purpose and Environment	875

A.4.3.3.2	Test Requirements	878

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	878

A.4.3.3A.1	Test Purpose and Environment	878

A.4.3.3A.2	Test Requirements	882

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	883

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	883

A.4.3.4.1.1	Test Purpose and Environment	883

A.4.3.4.1.2	Test Requirements	885

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	885

A.4.3.4.2.1	Test Purpose and Environment	885

A.4.3.4.2.2	Test Requirements	887

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	887

A.4.3.4.3.1	Test Purpose and Environment	887

A.4.3.4.3.2	Test Requirements	890

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	890

A.4.3.4.4.1	Test Purpose and Environment	890

A.4.3.4.4.2	Test Requirements	894

A.4.4	E-UTRAN to GSM Cell Re-Selection	894

A.4.4.1	E-UTRAN FDD – GSM:	894

A.4.4.1.1	Test Purpose and Environment	894

A.4.4.1.2	Test Requirements	896

A.4.4.2	E-UTRAN TDD – GSM:	896

A.4.4.2.1	Test Purpose and Environment	896

A.4.4.2.2	Test Requirements	898

A.4.5	E-UTRAN to HRPD Cell Re-Selection	899

A.4.5.1	E-UTRAN FDD – HRPD	899

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	899

A.4.5.1.1.1	Test Purpose and Environment	899

A.4.5.1.1.2	Test Requirements	901

A.4.5.2	E-UTRAN TDD – HRPD	901

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	901

A.4.5.2.1.1	Test Purpose and Environment	901

A.4.5.2.1.2	Test Requirements	904

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	904

A.4.6.1	E-UTRAN FDD – cdma2000 1X	904

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	904

A.4.6.1.1.1	Test Purpose and Environment	904

A.4.6.1.1.2	Test Requirements	907

A.4.6.2	E-UTRAN TDD – cdma2000 1X	907

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	907

A.4.6.2.1.1	Test Purpose and Environment	907

A.4.6.2.1.2	Test Requirements	910

A.4.7	Idle State Positioning Measurement for UE category NB1	910

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	910

A.4.7.1.1	Test Purpose and Environment	910

A.4.7.1.2	Test Requirements	914

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	915

A.4.7.2.1	Test Purpose and Environment	915

A.4.7.4.2	Test Requirements	918

A.5	E-UTRAN RRC CONNECTED Mode Mobility	919

A.5.1	E-UTRAN Handover	919

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	919

A.5.1.1.1	Test Purpose and Environment	919

A.5.1.1.2	Test Requirements	921

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	921

A.5.1.2.1	Test Purpose and Environment	921

A.5.1.2.2	Test Requirements	923

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	923

A.5.1.3.1	Test Purpose and Environment	923

A.5.1.3.2	Test Requirements	925

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	925

A.5.1.4.1	Test Purpose and Environment	925

A.5.1.4.2	Test Requirements	927

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	927

A.5.1.5.1	Test Purpose and Environment	927

A.5.1.5.2	Test Requirements	929

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	929

A.5.1.6.1	Test Purpose and Environment	929

A.5.1.6.2	Test Requirements	931

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	931

A.5.1.7.1	Test Purpose and Environment	931

A.5.1.7.2	Test Requirements	934

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	934

A.5.1.8.1	Test Purpose and Environment	934

A.5.1.8.2	Test Requirements	937

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	937

A.5.1.9.1	Test Purpose and Environment	937

A.5.1.9.2	Test Requirements	938

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	938

A.5.1.10.1	Test Purpose and Environment	938

A.5.1.10.2	Test Requirements	940

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	940

A.5.1.11.1	Test Purpose and Environment	940

A.5.1.11.2	Test Requirements	942

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	942

A.5.1.12.1	Test Purpose and Environment	942

A.5.1.12.2	Test Requirements	944

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	944

A.5.1.13.1	Test Purpose and Environment	944

A.5.1.13.2	Test Requirements	946

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	947

A.5.1.14.1	Test Purpose and Environment	947

A.5.1.14.2	Test Requirements	948

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	949

A.5.1.15.1	Test Purpose and Environment	949

A.5.1.15.2	Test Requirements	950

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	951

A.5.1.16.1	Test Purpose and Environment	951

A.5.1.16.2	Test Requirements	952

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	953

A.5.1.17.1	Test Purpose and Environment	953

A.5.1.17.2	Test Requirements	954

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	955

A.5.1.18.1	Test Purpose and Environment	955

A.5.1.18.2	Test Requirements	956

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	957

A.5.1.19.1	Test Purpose and Environment	957

A.5.1.19.2	Test Requirements	958

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	959

A.5.1.20.1	Test Purpose and Environment	959

A.5.1.20.2	Test Requirements	960

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	961

A.5.1.21.1	Test Purpose and Environment	961

A.5.1.21.2	Test Requirements	962

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	962

A.5.1.22.1	Test Purpose and Environment	962

A.5.1.22.2	Test Requirements	964

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	964

A.5.1.23.1	Test Purpose and Environment	964

A.5.1.23.2	Test Requirements	966

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	966

A.5.1.24.1	Test Purpose and Environment	966

A.5.1.24.2	Test Requirements	968

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	968

A.5.1.25.1	Test Purpose and Environment	968

A.5.1.25.2	Test Requirements	970

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	970

A.5.1.26.1	Test Purpose and Environment	970

A.5.1.26.2	Test Requirements	972

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	973

A.5.1.27.1	Test Purpose and Environment	973

A.5.1.27.2	Test Requirements	974

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	975

A.5.1.28.1	Test Purpose and Environment	975

A.5.1.28.2	Test Requirements	976

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	977

A.5.1.29.1	Test Purpose and Environment	977

A.5.1.29.2	Test Requirements	978

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	979

A.5.1.30.1	Test Purpose and Environment	979

A.5.1.30.2	Test Requirements	980

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	981

A.5.1.31.1	Test Purpose and Environment	981

A.5.1.31.2	Test Requirements	982

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	983

A.5.1.32.1	Test Purpose and Environment	983

A.5.1.32.2	Test Requirements	984

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	985

A.5.1.33.1	Test Purpose and Environment	985

A.5.1.13.2	Test Requirements	986

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	987

A.5.1.34.1	Test Purpose and Environment	987

A.5.1.34.2	Test Requirements	988

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	989

A.5.1.35.1	Test Purpose and Environment	989

A.5.1.35.2	Test Requirements	990

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	991

A.5.1.36.1	Test Purpose and Environment	991

A.5.1.36.2	Test Requirements	992

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	993

A.5.1.37.1	Test Purpose and Environment	993

A.5.1.37.2	Test Requirements	994

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	995

A.5.1.38.1	Test Purpose and Environment	995

A.5.1.38.2	Test Requirements	996

A.5.2	E-UTRAN Handover to other RATs	997

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	997

A.5.2.1.1	Test Purpose and Environment	997

A.5.2.1.2	Test Requirements	998

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	999

A.5.2.2.1	Test Purpose and Environment	999

A.5.2.2.2	Test Requirements	1002

A.5.2.3 E-UTRAN FDD- GSM Handover	1002

A.5.2.3.1	Test Purpose and Environment	1002

A.5.2.3.2	Test Requirements	1003

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1004

A.5.2.4.1	Test Purpose and Environment	1004

A.5.2.4.1.1	Void	1004

A.5.2.4.1.2	1.28 Mcps TDD option	1004

A.5.2.4.1.3	Void	1006

A.5.2.4.2	Test Requirements	1006

A.5.2.4.2.1	Void	1006

A.5.2.4.2.2	1.28 Mcps TDD option	1006

A.5.2.4.2.3	Void	1006

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1006

A.5.2.5.1	Test Purpose and Environment	1006

A.5.2.5.1.3	Void	1009

A.5.2.5.2	Test Requirements	1009

A.5.2.5.2.1	Void	1009

A.5.2.5.2.2	1.28 Mcps TDD option	1009

A.5.2.5.2.3	Void	1009

A.5.2.6	E-UTRAN TDD - GSM Handover	1009

A.5.2.6.1	Test Purpose and Environment	1009

A.5.2.6.2	Test Requirements	1011

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1012

A.5.2.7.1	Test Purpose and Environment	1012

A.5.2.7.2	Test Requirements	1014

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1014

A.5.2.8.1	Test Purpose and Environment	1014

A.5.2.8.2	Test Requirements	1015

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1016

A.5.2.9.1	Test Purpose and Environment	1016

A.5.2.9.2	Test Requirements	1017

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1018

A.5.2.10.1	Test Purpose and Environment	1018

A.5.2.10.2	Test Requirements	1020

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1020

A.5.2.10A.1	Test Purpose and Environment	1020

A.5.2.10A.2	Test Requirements	1023

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1023

A.5.2.10B.1	Test Purpose and Environment	1023

A.5.2.10B.2	Test Requirements	1026

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1026

A.5.2.11.1	Test Purpose and Environment	1026

A.5.2.11.2	Test Requirements	1026

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1027

A.5.3.1	E-UTRAN FDD – HRPD Handover	1027

A.5.3.1.1	Test Purpose and Environment	1027

A.5.3.1.2	Test Requirements	1029

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1029

A.5.3.2.1	Test Purpose and Environment	1029

A.5.3.2.2	Test Requirements	1032

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1032

A.5.3.3.1	Test Purpose and Environment	1032

A.5.3.3.2	Test Requirements	1034

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1035

A.5.3.4.1	Test Purpose and Environment	1035

A.5.3.4.2	Test Requirements	1036

A.5.3.5	E-UTRAN TDD – HRPD Handover	1037

A.5.3.5.1	Test Purpose and Environment	1037

A.5.3.5.2	Test Requirements	1039

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1040

A.5.3.6.1	Test Purpose and Environment	1040

A.5.3.6.2	Test Requirements	1043

A.6	RRC Connection Control	1043

A.6.1	RRC Re-establishment	1043

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1043

A.6.1.1.1	Test Purpose and Environment	1043

A.6.1.1.2	Test Requirements	1044

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1045

A.6.1.2.1	Test Purpose and Environment	1045

A.6.1.2.2	Test Requirements	1046

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1047

A.6.1.3.1	Test Purpose and Environment	1047

A.6.1.3.2	Test Requirements	1048

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1049

A.6.1.4.1	Test Purpose and Environment	1049

A.6.1.4.2	Test Requirements	1050

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1051

A.6.1.5.1	Test Purpose and Environment	1051

A.6.1.5.2	Test Requirements	1051

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1051

A.6.1.6.1	Test Purpose and Environment	1051

A.6.1.6.2	Test Requirements	1053

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1054

A.6.1.7.1	Test Purpose and Environment	1054

A.6.1.7.2	Test Requirements	1055

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1056

A.6.1.8.1	Test Purpose and Environment	1056

A.6.1.8.2	Test Requirements	1057

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1058

A.6.1.9.1	Test Purpose and Environment	1058

A.6.1.9.2	Test Requirements	1059

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1060

A.6.1.10.1	Test Purpose and Environment	1060

A.6.1.10.2	Test Requirements	1061

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1062

A.6.1.11.1	Test Purpose and Environment	1062

A.6.1.11.2	Test Requirements	1063

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1064

A.6.1.12.1	Test Purpose and Environment	1064

A.6.1.12.2	Test Requirements	1065

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1066

A.6.1.13.1	Test Purpose and Environment	1066

A.6.1.13.2	Test Requirements	1067

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1068

A.6.1.14.1	Test Purpose and Environment	1068

A.6.1.14.2	Test Requirements	1069

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1070

A.6.1.15.1	Test Purpose and Environment	1070

A.6.1.15.2	Test Requirements	1072

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1073

A.6.1.16.1	Test Purpose and Environment	1073

A.6.1.16.2	Test Requirements	1075

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1076

A.6.1.17.1	Test Purpose and Environment	1076

A.6.1.17.2	Test Requirements	1077

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1078

A.6.1.18.1	Test Purpose and Environment	1078

A.6.1.18.2	Test Requirements	1079

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1080

A.6.1.19.1	Test Purpose and Environment	1080

A.6.1.19.2	Test Requirements	1081

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1082

A.6.1.20.1	Test Purpose and Environment	1082

A.6.1.20.2	Test Requirements	1083

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1084

A.6.1.21.1	Test Purpose and Environment	1084

A.6.1.21.2	Test Requirements	1085

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1086

A.6.1.22.1	Test Purpose and Environment	1086

A.6.1.22.2	Test Requirements	1087

A.6.2	Random Access	1088

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1088

A.6.2.1.1	Test Purpose and Environment	1088

A.6.2.1.2	Test Requirements	1090

A.6.2.1.2.1	Random Access Response Reception	1090

A.6.2.1.2.2	No Random Access Response Reception	1090

A.6.2.1.2.3	Receiving a NACK on msg3	1090

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1090

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1091

A.6.2.1.2.6	Contention Resolution Timer expiry	1091

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1091

A.6.2.2.1	Test Purpose and Environment	1091

A.6.2.2.2	Test Requirements	1092

A.6.2.2.2.1	Random Access Response Reception	1093

A.6.2.2.2.2	No Random Access Response Reception	1093

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1093

A.6.2.3.1	Test Purpose and Environment	1093

A.6.2.3.2	Test Requirements	1095

A.6.2.3.2.1	Random Access Response Reception	1095

A.6.2.3.2.2	No Random Access Response reception	1095

A.6.2.3.2.3	Receiving a NACK on msg3	1095

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1096

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1096

A.6.2.3.2.6	Contention Resolution Timer expiry	1096

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1096

A.6.2.4.1	Test Purpose and Environment	1096

A.6.2.4.2	Test Requirements	1098

A.6.2.4.2.1	Random Access Response Reception	1098

A.6.2.4.2.2	No Random Access Response Reception	1098

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1098

A.6.2.5.1	Test Purpose and Environment	1098

A.6.2.5.2	Test Requirements	1099

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1099

A.6.2.6.1	Test Purpose and Environment	1099

A.6.2.6.2	Test Requirements	1099

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1100

A.6.2.7.1	Test Purpose and Environment	1100

A.6.2.7.2	Test Requirements	1102

A.6.2.7.2.1	Random Access Response Reception	1102

A.6.2.7.2.2	No Random Access Response Reception	1102

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1102

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1103

A.6.2.8.1	Test Purpose and Environment	1103

A.6.2.8.2	Test Requirements	1105

A.6.2.8.2.1	Random Access Response Reception	1105

A.6.2.8.2.2	No Random Access Response Reception	1105

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1105

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1106

A.6.2.9.1	Test Purpose and Environment	1106

A.6.2.9.2	Test Requirements	1108

A.6.2.9.2.1	Random Access Response Reception	1108

A.6.2.9.2.2	No Random Access Response Reception	1109

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1109

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1110

A.6.2.10.1	Test Purpose and Environment	1110

A.6.2.10.2	Test Requirements	1112

A.6.2.10.2.1	Random Access Response Reception	1112

A.6.2.10.2.2	No Random Access Response Reception	1112

A.6.2.10.2.3	Receiving a NACK on msg3	1113

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1113

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1113

A.6.2.10.2.6	Contention Resolution Timer expiry	1113

A.6.2.10.2.7	PRACH Resource Selection	1113

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1113

A.6.2.11.1	Test Purpose and Environment	1113

A.6.2.11.2	Test Requirements	1115

A.6.2.11.2.1	Random Access Response Reception	1115

A.6.2.11.2.2	No Random Access Response Reception	1116

A.6.2.11.2.3	Receiving a NACK on msg3	1116

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1116

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1116

A.6.2.11.2.6	Contention Resolution Timer expiry	1116

A.6.2.11.2.7	PRACH Resource Selection	1117

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1117

A.6.2.12.1	Test Purpose and Environment	1117

A.6.2.12.2	Test Requirements	1119

A.6.2.12.2.1	Random Access Response Reception	1119

A.6.2.12.2.2	No Random Access Response Reception	1120

A.6.2.12.2.3	Receiving a NACK on msg3	1120

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1120

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1120

A.6.2.12.2.6	Contention Resolution Timer expiry	1120

A.6.2.12.2.7	PRACH Resource Selection	1121

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1121

A.6.2.13.1	Test Purpose and Environment	1121

A.6.2.13.2	Test Requirements	1123

A.6.2.13.2.1	Random Access Response Reception	1123

A.6.2.13.2.2	No Random Access Response Reception	1123

A.6.2.13.2.3	Receiving a NACK on msg3	1124

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1124

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1124

A.6.2.13.2.6	Contention Resolution Timer expiry	1124

A.6.2.13.2.7	PRACH Resource Selection	1124

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1124

A.6.2.14.1	Test Purpose and Environment	1124

A.6.2.14.2	Test Requirements	1126

A.6.2.14.2.1	Random Access Response Reception	1126

A.6.2.14.2.2	No Random Access Response Reception	1127

A.6.2.14.2.3	Receiving a NACK on msg3	1127

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1127

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1127

A.6.2.14.2.6	Contention Resolution Timer expiry	1127

A.6.2.14.2.7	PRACH Resource Selection	1128

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1128

A.6.2.15.1	Test Purpose and Environment	1128

A.6.2.15.2	Test Requirements	1130

A.6.2.15.2.1	Random Access Response Reception	1130

A.6.2.15.2.2	No Random Access Response Reception	1131

A.6.2.15.2.3	Receiving a NACK on msg3	1131

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1131

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1131

A.6.2.15.2.6	Contention Resolution Timer expiry	1131

A.6.2.15.2.7	PRACH Resource Selection	1132

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1132

A.6.2.16.1	Test Purpose and Environment	1132

A.6.2.16.2	Test Requirements	1135

A.6.2.16.2.1	Random Access Response Reception	1135

A.6.2.16.2.2	No Random Access Response Reception	1136

A.6.2.16.2.3	Receiving a NACK on msg3	1136

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1136

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1136

A.6.2.16.2.6	Contention Resolution Timer expiry	1136

A.6.2.16.2.7	NPRACH Resource Selection	1136

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1137

A.6.2.17.1	Test Purpose and Environment	1137

A.6.2.17.2	Test Requirements	1140

A.6.2.17.2.1	Random Access Response Reception	1140

A.6.2.17.2.2	No Random Access Response Reception	1141

A.6.2.17.2.3	Receiving a NACK on msg3	1141

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1141

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1141

A.6.2.17.2.6	Contention Resolution Timer expiry	1141

A.6.2.17.2.7	NPRACH Resource Selection	1141

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1142

A.6.2.18.1	Test Purpose and Environment	1142

A.6.2.18.2	Test Requirements	1145

A.6.2.18.2.1	Random Access Response Reception	1145

A.6.2.18.2.2	No Random Access Response Reception	1146

A.6.2.18.2.3	Receiving a NACK on msg3	1146

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1146

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1146

A.6.2.18.2.6	Contention Resolution Timer expiry	1146

A.6.2.18.2.7	NPRACH Resource Selection	1146

A.6.3	RRC Connection Release with Redirection	1147

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1147

A.6.3.1.1	Test Purpose and Environment	1147

A.6.3.1.2	Test Requirements	1149

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1149

A.6.3.2.1	Test Purpose and Environment	1149

A.6.3.2.2	Test Requirements	1151

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1151

A.6.3.3.1	Test Purpose and Environment	1151

A.6.3.3.2	Test Requirements	1152

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1153

A.6.3.4.1	Test Purpose and Environment	1153

A.6.3.4.2	Test Requirements	1154

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1155

A.6.3.5.1	Test Purpose and Environment	1155

A.6.3.5.2	Test Requirements	1157

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1157

A.6.3.6.1	Test Purpose and Environment	1157

A. 6.3.6.2	Test Requirements	1159

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1159

A.6.3.7.1	Test Purpose and Environment	1159

A.6.3.7.2	Test Requirements	1162

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1162

A.6.3.8.1	Test Purpose and Environment	1162

A.6.3.8.2	Test Requirements	1165

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1165

A.6.3.9.1	Test Purpose and Environment	1165

A.6.3.9.2	Test Requirements	1167

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1167

A.6.3.10.1	Test Purpose and Environment	1167

A.6.3.10.2	Test Requirements	1169

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1169

A.6.3.11.1	Test Purpose and Environment	1169

A.6.3.11.2	Test Requirements	1171

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1171

A.6.3.12.1	Test Purpose and Environment	1171

A.6.3.12.2	Test Requirements	1174

A.7	Timing and Signalling Characteristics	1176

A.7.1	UE Transmit Timing	1176

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1176

A.7.1.1.1	Test Purpose and Environment	1176

A.7.1.1.2	Test Requirements	1177

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1178

A.7.1.2.1	Test Purpose and Environment	1178

A.7.1.2.2	Test Requirements	1180

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1181

A.7.1.3.1	Test Purpose and Environment	1181

A.7.1.3.2	Test Requirements	1183

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1184

A.7.1.4.1	Test Purpose and Environment	1184

A.7.1.4.2	Test Requirements	1185

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1186

A.7.1.4A.1	Test Purpose and Environment	1186

A.7.1.4A.2	Test Requirements	1186

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1186

A.7.1.5.1	Test Purpose and Environment	1186

A.7.1.5.2	Test Requirements	1187

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1187

A.7.1.6.1	Test Purpose and Environment	1187

A.7.1.6.2	Test Requirements	1189

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1190

A.7.1.7.1	Test Purpose and Environment	1190

A.7.1.7.2	Test Requirements	1191

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1192

A.7.1.7A.1	Test Purpose and Environment	1192

A.7.1.7A.2	Test Requirements	1192

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1192

A.7.1.7B.1	Test Purpose and Environment	1192

A.7.1.7B.2	Test Requirements	1192

A.7.1.8	Void	1193

A.7.1.8.1	Void	1193

A.7.1.8.2	Void	1193

A.7.1.9	Void	1193

A.7.1.9.1	Void	1193

A.7.1.9.2	Void	1193

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1193

A.7.1.10.1	Test Purpose and Environment	1193

A.7.1.10.2	Test Requirements	1195

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1195

A.7.1.11.1	Test Purpose and Environment	1195

A.7.1.11.2	Test Requirements	1197

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1197

A.7.1.12.1	Test Purpose and Environment	1197

A.7.1.12.2	Test Requirements	1199

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1199

A.7.1.13.1	Test Purpose and Environment	1199

A.7.1.13.2	Test Requirements	1201

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1202

A.7.1.14.1	Test Purpose and Environment	1202

A.7.1.14.2	Test Requirements	1203

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1203

A.7.1.15.1	Test Purpose and Environment	1203

A.7.1.15.2	Test Requirements	1204

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1205

A.7.1.16.1	Test Purpose and Environment	1205

A.7.1.16.2	Test Requirements	1206

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1207

A.7.1.17.1	Test Purpose and Environment	1207

A.7.1.17.2	Test Requirements	1209

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1210

A.7.1.18.1	Test Purpose and Environment	1210

A.7.1.18.2	Test Requirements	1212

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1213

A.7.1.19.1	Test Purpose and Environment	1213

A.7.1.19.2	Test Requirements	1214

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1214

A.7.1.20.1	Test Purpose and Environment	1214

A.7.1.20.2	Test Requirements	1216

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1216

A.7.1.21.1	Test Purpose and Environment	1216

A.7.1.21.2	Test Requirements	1218

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1218

A.7.1.22.1	Test Purpose and Environment	1218

A.7.1.22.2	Test Requirements	1220

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1220

A.7.1.23.1	Test Purpose and Environment	1220

A.7.1.23.2	Test Requirements	1222

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1222

A.7.1.24.1	Test Purpose and Environment	1222

A.7.1.24.2	Test Requirements	1223

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1224

A.7.1.25.1	Test Purpose and Environment	1224

A.7.1.25.2	Test Requirements	1225

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1226

A.7.1.26.1	Test Purpose and Environment	1226

A.7.1.26.2	Test Requirements	1227

A.7.2	UE Timing Advance	1228

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1228

A.7.2.1.1	Test Purpose and Environment	1228

A.7.2.1.2	Test Requirements	1230

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1230

A.7.2.2.1	Test Purpose and Environment	1230

A.7.2.2.2	Test Requirements	1232

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1232

A.7.2.3.1	Test Purpose and Environment	1232

A.7.2.3.2	Test Requirements	1233

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1233

A.7.2.4.1	Test Purpose and Environment	1233

A.7.2.4.2	Test Requirements	1235

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1235

A.7.2.5.1	Test Purpose and Environment	1235

A.7.2.5.2	Test Requirements	1237

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1237

A.7.2.5A.1	Test Purpose and Environment	1237

A.7.2.5A.2	Test Requirements	1238

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1238

A.7.2.5B.1	Test Purpose and Environment	1238

A.7.2.5B.2	Test Requirements	1238

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1238

A.7.2.6.1	Test Purpose and Environment	1238

A.7.2.6.2	Test Requirements	1241

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1241

A.7.2.7.1	Test Purpose and Environment	1241

A.7.2.7.2	Test Requirements	1243

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1243

A.7.2.8.1	Test Purpose and Environment	1243

A.7.2.8.2	Test Requirements	1245

A.7.2.9.2	Test Requirements	1247

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1247

A.7.2.10.1	Test Purpose and Environment	1247

A.7.2.10.2	Test Requirements	1248

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1248

A.7.2.11.1	Test Purpose and Environment	1248

A.7.2.11.2	Test Requirements	1250

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1250

A.7.2.12.1	Test Purpose and Environment	1250

A.7.2.12.2	Test Requirements	1252

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1252

A.7.2.13.1	Test Purpose and Environment	1252

A.7.2.13.2	Test Requirements	1254

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1254

A.7.2.14.1	Test Purpose and Environment	1254

A.7.2.14.2	Test Requirements	1256

A.7.3	Radio Link Monitoring	1256

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1257

A.7.3.1.1	Test Purpose and Environment	1257

A.7.3.1.2	Test Requirements	1261

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1261

A.7.3.2.1	Test Purpose and Environment	1261

A.7.3.2.2	Test Requirements	1264

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1264

A.7.3.3.1	Test Purpose and Environment	1264

A.7.3.3.2	Test Requirements	1268

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1268

A.7.3.4.1	Test Purpose and Environment	1268

A.7.3.4.2	Test Requirements	1271

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1271

A.7.3.5.1	Test Purpose and Environment	1271

A.7.3.5.2	Test Requirements	1275

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1276

A.7.3.6.1	Test Purpose and Environment	1276

A.7.3.6.2	Test Requirements	1279

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1279

A.7.3.7.1	Test Purpose and Environment	1279

A.7.3.7.2	Test Requirements	1282

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1282

A.7.3.8.1	Test Purpose and Environment	1282

A.7.3.8.2	Test Requirements	1285

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1285

A.7.3.9.2	Test Requirements	1288

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1288

A.7.3.10.1	Test Purpose and Environment	1288

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1291

A.7.3.11.1	Test Purpose and Environment	1291

A.7.3.11.2	Test Requirements	1295

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1295

A.7.3.12.1	Test Purpose and Environment	1295

A.7.3.12.2	Test Requirements	1299

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1299

A.7.3.13.1	Test Purpose and Environment	1299

A.7.3.13.2	Test Requirements	1302

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1302

A.7.3.14.1	Test Purpose and Environment	1302

A.7.3.14.2	Test Requirements	1305

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1305

A.7.3.15.1	Test Purpose and Environment	1305

A.7.3.15.2	Test Requirements	1308

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1308

A.7.3.16.1	Test Purpose and Environment	1308

A.7.3.16.2	Test Requirements	1312

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1312

A.7.3.17.1	Test Purpose and Environment	1312

A.7.3.17.2	Test Requirements	1315

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1315

A.7.3.18.1	Test Purpose and Environment	1315

A.7.3.18.2	Test Requirements	1319

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1319

A.7.3.19.1	Test Purpose and Environment	1319

A.7.3.19.2	Test Requirements	1323

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1323

A.7.3.20.1	Test Purpose and Environment	1323

A.7.3.20.2	Test Requirements	1327

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1327

A.7.3.21.1	Test Purpose and Environment	1327

A.7.3.21.2	Test Requirements	1331

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1331

A.7.3.22.1	Test Purpose and Environment	1331

A.7.3.22.2	Test Requirements	1335

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1335

A.7.3.23.1	Test Purpose and Environment	1335

A.7.3.23.2	Test Requirements	1336

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1336

A.7.3.24.1	Test Purpose and Environment	1336

A.7.3.24.2	Test Requirements	1337

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1337

A.7.3.25.1	Test Purpose and Environment	1337

A.7.3.25.2	Test Requirements	1338

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1338

A.7.3.26.1	Test Purpose and Environment	1338

A.7.3.26.2	Test Requirements	1341

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1341

A.7.3.27.1	Test Purpose and Environment	1341

A.7.3.27.2	Test Requirements	1344

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1344

A.7.3.28.1	Test Purpose and Environment	1344

A.7.3.28.2	Test Requirements	1347

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1347

A.7.3.29.1	Test Purpose and Environment	1347

A.7.3.29.2	Test Requirements	1350

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1350

A.7.3.30.1	Test Purpose and Environment	1350

A.7.3.30.2	Test Requirements	1353

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1353

A.7.3.31.1	Test Purpose and Environment	1353

A.7.3.31.2	Test Requirements	1356

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1356

A.7.3.32.1	Test Purpose and Environment	1356

A.7.3.32.2	Test Requirements	1359

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1359

A.7.3.33.1	Test Purpose and Environment	1359

A.7.3.33.2	Test Requirements	1362

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1362

A.7.3.34.1	Test Purpose and Environment	1362

A.7.3.34.2	Test Requirements	1365

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1365

A.7.3.35.1	Test Purpose and Environment	1365

A.7.3.35.2	Test Requirements	1368

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1368

A.7.3.36.1	Test Purpose and Environment	1368

A.7.3.36.2	Test Requirements	1371

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1371

A.7.3.37.1	Test Purpose and Environment	1371

A.7.3.37.2	Test Requirements	1374

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1374

A.7.3.38.1	Test Purpose and Environment	1374

A.7.3.38.2	Test Requirements	1377

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1377

A.7.3.39.1	Test Purpose and Environment	1377

A.7.3.39.2	Test Requirements	1380

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1381

A.7.3.40.1	Test Purpose and Environment	1381

A.7.3.40.2	Test Requirements	1383

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1384

A.7.3.41.1	Test Purpose and Environment	1384

A.7.3.41.2	Test Requirements	1387

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1387

A.7.3.42.1	Test Purpose and Environment	1387

A.7.3.42.2	Test Requirements	1390

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1390

A.7.3.43.1	Test Purpose and Environment	1390

A.7.3.43.2	Test Requirements	1393

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1393

A.7.3.44.1	Test Purpose and Environment	1393

A.7.3.44.2	Test Requirements	1396

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1397

A.7.3.45.1	Test Purpose and Environment	1397

A.7.3.45.2	Test Requirements	1399

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1400

A.7.3.46.1	Test Purpose and Environment	1400

A.7.3.46.2	Test Requirements	1403

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1403

A.7.3.47.1	Test Purpose and Environment	1403

A.7.3.47.2	Test Requirements	1406

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1406

A.7.3.48.1	Test Purpose and Environment	1406

A.7.3.48.2	Test Requirements	1409

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1409

A.7.3.49.1	Test Purpose and Environment	1409

A.7.3.49.2	Test Requirements	1412

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1412

A.7.3.50.1	Test Purpose and Environment	1412

A.7.3.50.2	Test Requirements	1415

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1415

A.7.3.51.1	Test Purpose and Environment	1415

A.7.3.51.2	Test Requirements	1418

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1418

A.7.3.52.1	Test Purpose and Environment	1418

A.7.3.52.2	Test Requirements	1421

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1421

A.7.3.53.1	Test Purpose and Environment	1421

A.7.3.53.2	Test Requirements	1424

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1424

A.7.3.54.1	Test Purpose and Environment	1424

A.7.3.54.2	Test Requirements	1427

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1427

A.7.3.55.1	Test Purpose and Environment	1427

A.7.3.55.2	Test Requirements	1430

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1430

A.7.3.56.1	Test Purpose and Environment	1430

A.7.3.56.2	Test Requirements	1433

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1433

A.7.3.57.1	Test Purpose and Environment	1433

A.7.3.57.2	Test Requirements	1436

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1436

A.7.3.58.1	Test Purpose and Environment	1436

A.7.3.58.2	Test Requirements	1439

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1439

A.7.3.59.1	Test Purpose and Environment	1439

A.7.3.59.2	Test Requirements	1442

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1442

A.7.3.60.1	Test Purpose and Environment	1442

A.7.3.60.2	Test Requirements	1446

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1446

A.7.3.61.1	Test Purpose and Environment	1446

A.7.3.61.2	Test Requirements	1450

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1450

A.7.3.62.1	Test Purpose and Environment	1450

A.7.3.62.2	Test Requirements	1454

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1454

A.7.3.63.1	Test Purpose and Environment	1454

A.7.3.63.2	Test Requirements	1458

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1458

A.7.3.64.1	Test Purpose and Environment	1458

A.7.3.64.2	Test Requirements	1461

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1462

A.7.3.65.1	Test Purpose and Environment	1462

A.7.3.65.2	Test Requirements	1465

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1466

A.7.3.66.1	Test Purpose and Environment	1466

A.7.3.66.2	Test Requirements	1468

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1469

A.7.3.67.1	Test Purpose and Environment	1469

A.7.3.67.2	Test Requirements	1472

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1473

A.7.3.68.1	Test Purpose and Environment	1473

A.7.3.68.2	Test Requirements	1475

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1475

A.7.3.69.1	Test Purpose and Environment	1475

A.7.3.69.2	Test Requirements	1477

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1477

A.7.3.70.1	Test Purpose and Environment	1477

A.7.3.70.2	Test Requirements	1479

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1479

A.7.3.71.1	Test Purpose and Environment	1479

A.7.3.71.2	Test Requirements	1482

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1482

A.7.3.72.1	Test Purpose and Environment	1482

A.7.3.72.2	Test Requirements	1484

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1484

A.7.3.73.1	Test Purpose and Environment	1484

A.7.3.73.2	Test Requirements	1486

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1486

A.7.3.74.1	Test Purpose and Environment	1486

A.7.3.74.2	Test Requirements	1489

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1489

A.7.3.75.1	Test Purpose and Environment	1489

A.7.3.75.2	Test Requirements	1491

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1492

A.7.3.76.1	Test Purpose and Environment	1492

A.7.3.76.2	Test Requirements	1495

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1495

A.7.3.77.1	Test Purpose and Environment	1495

A.7.3.77.2	Test Requirements	1498

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1498

A.7.3.78.1	Test Purpose and Environment	1498

A.7.3.78.2	Test Requirements	1501

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1501

A.7.3.79.1	Test Purpose and Environment	1501

A.7.3.79.2	Test Requirements	1504

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1504

A.7.3.80.1	Test Purpose and Environment	1504

A.7.3.80.2	Test Requirements	1507

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1507

A.7.3.81.1	Test Purpose and Environment	1507

A.7.3.81.2	Test Requirements	1510

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1510

A.7.3.82.1	Test Purpose and Environment	1510

A.7.3.82.2	Test Requirements	1513

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1513

A.7.3.83.1	Test Purpose and Environment	1513

A.7.3.83.2	Test Requirements	1514

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1515

A.7.3.84.1	Test Purpose and Environment	1515

A.7.3.84.2	Test Requirements	1517

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1517

A.7.3.85.1	Test Purpose and Environment	1517

A.7.3.85.2	Test Requirements	1518

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1519

A.7.3.86.1	Test Purpose and Environment	1519

A.7.3.86.2	Test Requirements	1521

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1521

A.7.3.87.1	Test Purpose and Environment	1521

A.7.3.87.2	Test Requirements	1523

A.7.4	Interruption for Dual Connectivity	1524

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1524

A.7.4.1.1	Test Purpose and Environment	1524

A.7.4.1.2	Test Requirements	1526

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1526

A.7.4.2.1	Test Purpose and Environment	1526

A.7.4.2.2	Test Requirements	1528

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1528

A.7.4.3.1	Test Purpose and Environment	1528

A.7.4.3.2	Test Requirements	1530

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1530

A.7.4.4.1	Test Purpose and Environment	1530

A.7.4.4.2	Test Requirements	1533

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1533

A.7.4.5.1	Test Purpose and Environment	1533

A.7.4.5.2	Test Requirements	1535

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1535

A.7.4.6.1	Test Purpose and Environment	1535

A.7.4.6.2	Test Requirements	1537

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1537

A.7.4.7.1	Test Purpose and Environment	1537

A.7.4.7.2	Test Requirements	1540

A.7.5	Proximity-based Services	1540

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1540

A.7.5.1.1	Test Purpose and Environment	1540

A.7.5.1.2	Test Requirements	1541

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1542

A.7.5.2.1	Test Purpose and Environment	1542

A.7.5.1.2	Test Requirements	1542

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1543

A.7.5.3.1	Test Purpose and Environment	1543

A.7.5.3.2	Test Requirements	1544

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1545

A.7.5.4.1	Test Purpose and Environment	1545

A.7.5.4.2	Test Requirements	1546

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1547

A.7.5.5.1	Test Purpose and Environment	1547

A.7.5.5.2	Test Requirements	1549

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1550

A.7.5.6.1	Test Purpose and Environment	1550

A.7.5.6.2	Test Requirements	1551

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1552

A.7.5.7.1	Test Purpose and Environment	1552

A.7.5.7.2	Test Requirements	1554

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1554

A.7.5.8.1	Test Purpose and Environment	1554

A.7.5.8.2	Test Requirements	1556

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1557

A.7.5.9.1	Test Purpose and Environment	1557

A.7.5.9.2	Test Requirements	1559

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1560

A.7.5.10.1	Test Purpose and Environment	1560

A.7.5.10.2	Test Requirements	1562

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1563

A.7.5.11.1	Test Purpose and Environment	1563

A.7.5.11.2	Test Requirements	1565

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1565

A.7.5.12.1	Test Purpose and Environment	1565

A.7.5.12.2	Test Requirements	1569

A.7.6	Interruption for carrier aggregation	1570

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1570

A.7.6.1.1	Test Purpose and Environment	1570

A.7.6.1.2	Test Requirements	1572

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1572

A.7.6.2.1	Test Purpose and Environment	1572

A.7.6.2.2	Test Requirements	1575

A.8	UE Measurements Procedures	1575

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1575

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1575

A.8.1.1.1	Test Purpose and Environment	1575

A.8.1.1.2	Test Requirements	1577

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1577

A.8.1.2.1	Test Purpose and Environment	1577

A.8.1.2.2	Test Requirements	1579

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1579

A.8.1.3.1	Test Purpose and Environment	1579

A.8.1.3.2	Test Requirements	1582

A.8.1.4	Void	1582

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1582

A.8.1.5.1	Test Purpose and Environment	1582

A.8.1.5.2	Test Requirements	1584

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1584

A.8.1.6.1	Test Purpose and Environment	1584

A.8.1.6.2	Test Requirements	1586

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1586

A.8.1.7.1	Test Purpose and Environment	1586

A.8.1.7.2	Test Requirements	1588

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1589

A.8.1.8.1	Test Purpose and Environment	1589

A.8.1.8.2	Test Requirements	1591

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1592

A.8.1.9.1	Test Purpose and Environment	1592

A.8.1.9.2	Test Requirements	1592

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1592

A.8.1.10.1	Test Purpose and Environment	1592

A.8.1.10.2	Test Requirements	1593

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1593

A.8.1.11.1	Test Purpose and Environment	1593

A.8.1.11.2	Test Requirements	1595

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1596

A.8.1.12.1	Test Purpose and Environment	1596

A.8.1.12.2	Test Requirements	1597

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1598

A.8.1.13.1	Test Purpose and Environment	1598

A.8.1.13.2	Test Requirements	1600

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1600

A.8.1.14.1	Test Purpose and Environment	1600

A.8.1.14.2	Test Requirements	1602

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1603

A.8.1.15.1	Test Purpose and Environment	1603

A.8.1.15.2	Test Requirements	1604

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1605

A.8.1.16.1	Test Purpose and Environment	1605

A.8.1.16.2	Test Requirements	1607

A.8.1.17	Void	1607

A.8.1.18	Void	1607

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1607

A.8.1.19.1	Test Purpose and Environment	1607

A.8.1.19.2	Test Requirements	1609

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1610

A.8.1.20.1	Test Purpose and Environment	1610

A.8.1.20.2	Test Requirements	1612

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1612

A.8.1.21.1	Test Purpose and Environment	1612

A.8.1.21.2	Test Requirements	1614

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1615

A.8.1.22.1	Test Purpose and Environment	1615

A.8.1.22.2	Test Requirements	1617

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1617

A.8.1.23.1	Test Purpose and Environment	1617

A.8.1.23.2	Test Requirements	1618

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1619

A.8.1.24.1	Test Purpose and Environment	1619

A.8.1.24.2	Test Requirements	1621

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1621

A.8.1.25.1	Test Purpose and Environment	1621

A.8.1.25.2	Test Requirements	1623

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1623

A.8.1.26.1	Test Purpose and Environment	1623

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1626

A.8.1.27.1	Test Purpose and Environment	1626

A.8.1.27.2	Test Requirements	1628

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1628

A.8.1.28.1	Test Purpose and Environment	1628

A.8.1.28.2	Test Requirements	1630

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1630

A.8.1.29.1	Test Purpose and Environment	1630

A.8.1.29.2	Test Requirements	1632

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1633

A.8.1.30.1	Test Purpose and Environment	1633

A.8.1.30.2	Test Requirements	1635

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1635

A.8.1.31.1	Test Purpose and Environment	1635

A.8.1.31.2	Test Requirements	1637

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1638

A.8.1.32.1	Test Purpose and Environment	1638

A.8.1.32.2	Test Requirements	1639

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1640

A.8.1.33.1	Test Purpose and Environment	1640

A.8.1.33.2	Test Requirements	1641

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1642

A.8.1.34.1	Test Purpose and Environment	1642

A.8.1.34.2	Test Requirements	1643

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1644

A.8.1.35.1	Test Purpose and Environment	1644

A.8.1.35.2	Test Requirements	1645

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1646

A.8.1.36.1	Test Purpose and Environment	1646

A.8.1.36.2	Test Requirements	1647

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1648

A.8.1.37.1	Test Purpose and Environment	1648

A.8.1.37.2	Test Requirements	1650

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1650

A.8.1.38.1	Test Purpose and Environment	1650

A.8.1.38.2	Test Requirements	1651

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1652

A.8.1.39.1	Test Purpose and Environment	1652

A.8.1.39.2	Test Requirements	1654

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1654

A.8.1.40.1	Test Purpose and Environment	1654

A.8.1.40.2	Test Requirements	1656

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1656

A.8.1.41.1	Test Purpose and Environment	1656

A.8.1.41.2	Test Requirement	1658

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1658

A.8.1.42.1	Test Purpose and Environment	1658

A.8.1.42.2	Test Requirement	1659

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1660

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1660

A.8.2.1.1	Test Purpose and Environment	1660

A.8.2.1.2	Test Requirements	1661

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1661

A.8.2.2.1	Test Purpose and Environment	1661

A.8.2.2.2	Test Requirements	1664

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1664

A.8.2.3.1	Test Purpose and Environment	1664

A.8.2.3.2	Test Requirements	1666

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1667

A.8.2.4.1	Test Purpose and Environment	1667

A.8.2.4.2	Test Requirements	1669

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1669

A.8.2.5.1	Test Purpose and Environment	1669

A.8.2.5.2	Test Requirements	1671

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1672

A.8.2.6.1	Test Purpose and Environment	1672

A.8.2.6.2	Test Requirements	1674

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1675

A.8.2.7.1	Test Purpose and Environment	1675

A.8.2.7.2	Test Requirements	1676

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1677

A.8.2.8.1	Test Purpose and Environment	1677

A.8.2.8.2	Test Requirements	1679

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1679

A.8.2.9.1	Test Purpose and Environment	1679

A.8.2.9.2	Test Requirements	1681

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1682

A.8.2.10.1	Test Purpose and Environment	1682

A.8.2.10.2	Test Requirements	1684

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1684

A.8.2.11.1	Test Purpose and Environment	1684

A.8.2.11.2	Test Requirements	1686

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1686

A.8.2.12.1	Test Purpose and Environment	1686

A.8.2.12.2	Test Requirements	1688

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1689

A.8.2.13.1	Test Purpose and Environment	1689

A.8.2.13.2	Test Requirements	1691

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1691

A.8.2.14.1	Test Purpose and Environment	1691

A.8.2.14.2	Test Requirement	1693

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1694

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1694

A.8.3.1.1	Test Purpose and Environment	1694

A.8.3.1.2	Test Requirements	1695

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1695

A.8.3.2.1	Test Purpose and Environment	1695

A.8.3.2.2	Test Requirements	1698

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1698

A.8.3.3.1	Test Purpose and Environment	1698

A.8.3.3.2	Test Requirements	1701

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1701

A.8.3.4.1	Test Purpose and Environment	1701

A.8.3.4.2	Test Requirements	1703

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1703

A.8.3.5.2	Test Requirements	1705

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1705

A.8.3.6.1	Test Purpose and Environment	1705

A.8.3.6.2	Test Requirements	1707

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1708

A.8.3.7.1	Test Purpose and Environment	1708

A.8.3.7.2	Test Requirements	1710

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1711

A.8.3.8.1	Test Purpose and Environment	1711

A.8.3.8.2	Test Requirements	1714

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1714

A.8.3.9.1	Test Purpose and Environment	1714

A.8.3.9.2	Test Requirements	1717

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1718

A.8.3.10.1	Test Purpose and Environment	1718

A.8.3.10.2	Test Requirements	1719

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1719

A.8.3.11.1	Test Purpose and Environment	1719

A.8.3.11.2	Test Requirement	1721

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1722

A.8.3.12.1	Test Purpose and Environment	1722

A.8.3.12.2	Test Requirement	1723

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1724

A.8.3.13.1	Test Purpose and Environment	1724

A.8.3.13.2	Test Requirement	1725

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1726

A.8.3.14.1	Test Purpose and Environment	1726

A.8.3.14.2	Test Requirement	1727

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1728

A.8.3.15.1	Test Purpose and Environment	1728

A.8.3.15.2	Test Requirement	1729

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1730

A.8.3.16.1	Test Purpose and Environment	1730

A.8.3.16.2	Test Requirement	1732

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1732

A.8.3.17.1	Test Purpose and Environment	1732

A.8.3.17.2	Test Requirement	1735

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1735

A.8.3.18.1	Test Purpose and Environment	1735

A.8.3.18.2	Test Requirement	1738

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1738

A.8.3.19.1	Test Purpose and Environment	1738

A.8.3.19.2	Test Requirement	1741

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1741

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1741

A.8.4.1.1	Test Purpose and Environment	1741

A.8.4.1.2	Test Requirements	1743

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1743

A.8.4.2.1	Test Purpose and Environment	1743

A.8.4.2.2	Test Requirements	1746

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1746

A.8.4.3.1	Test Purpose and Environment	1746

A.8.4.3.2	Test Requirements	1748

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1749

A.8.4.4.1	Test Purpose and Environment	1749

A.8.4.4.2	Test Requirements	1751

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1752

A.8.4.5.1	Test Purpose and Environment	1752

A.8.4.5.2	Test Requirements	1754

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1754

A.8.4.6.1	Test Purpose and Environment	1754

A.8.4.6.2	Test Requirements	1755

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1755

A.8.4.7.1	Test Purpose and Environment	1755

A.8.4.7.2	Test Requirements	1759

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1759

A.8.4.8.1	Test Purpose and Environment	1759

A.8.4.8.2	Test Requirements	1762

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1762

A.8.4.9.1	Test Purpose and Environment	1762

A.8.4.9.2	Test Requirements	1766

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1766

A.8.4.10.1	Test Purpose and Environment	1766

A.8.4.10.2	Test Requirements	1768

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1768

A.8.4.11.1	Test Purpose and Environment	1768

A.8.4.11.2	Test Requirement	1770

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1770

A.8.4.12.1	Test Purpose and Environment	1770

A.8.4.12.2	Test Requirement	1772

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1773

A.8.4.13.1	Test Purpose and Environment	1773

A.8.4.13.2	Test Requirement	1774

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1775

A.8.4.14.1	Test Purpose and Environment	1775

A.8.4.14.2	Test Requirement	1777

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1777

A.8.4.15.1	Test Purpose and Environment	1777

A.8.4.15.2	Test Requirement	1780

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1780

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1780

A.8.5.1.1	Test Purpose and Environment	1780

A.8.5.1.2	Test Requirements	1782

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1782

A.8.5.2.1	Test Purpose and Environment	1782

A.8.5.2.2	Test Requirements	1784

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1784

A.8.5.3.1	Test Purpose and Environment	1784

A.8.5.3.2	Test Requirements	1787

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1787

A.8.5.4.1	Test Purpose and Environment	1787

A.8.5.4.2	Test Requirements	1790

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1790

A.8.5.5.1	Test Purpose and Environment	1790

A.8.5.5.2	Test Requirements	1792

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1792

A.8.5.6.1	Test Purpose and Environment	1792

A.8.5.6.2	Test Requirements	1793

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1794

A.8.5.7.1	Test Purpose and Environment	1794

A.8.5.7.2	Test Requirements	1794

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1794

A.8.5.8.1	Test Purpose and Environment	1794

A.8.5.8.2	Test Requirements	1797

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1797

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1797

A.8.6.1.1	Test Purpose and Environment	1797

A.8.6.1.2	Test Requirements	1799

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1800

A.8.6.2.1	Test Purpose and Environment	1800

A.8.6.2.2	Test Requirements	1802

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1802

A.8.6.3.1	Test Purpose and Environment	1802

A.8.6.3.2	Test Requirements	1805

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1805

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1805

A.8.7.1.1	Test Purpose and Environment	1805

A.8.7.1.1.1	Void	1805

A.8.7.1.1.2	1.28 Mcps TDD option	1805

A.8.7.1.1.3	Void	1807

A.8.7.1.2	Test Requirements	1807

A.8.7.1.2.1	Void	1807

A.8.7.1.2.2	1.28 Mcps TDD option	1807

A.8.7.1.2.3	Void	1807

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1807

A.8.7.2.1	Test Purpose and Environment	1807

A.8.7.2.2	Test Requirements	1810

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1811

A.8.7.3.1	Test Purpose and Environment	1811

A.8.7.3.2	Test Parameters	1811

A.8.7.3.3	Test Requirements	1812

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1813

A.8.7.4.1	Test Purpose and Environment	1813

A.8.7.4.2	Test Requirements	1815

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1815

A.8.7.5.1	Test Purpose and Environment	1815

A.8.7.5.2	Test Requirements	1817

A.8.7A	TBD	1817

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1817

A.8.7A.1.1	Test Purpose and Environment	1817

A.8.7A.1.2	Test Requirements	1820

A.8.8	E-UTRAN FDD – GSM Measurements	1820

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1820

A.8.8.1.1	Test Purpose and Environment	1820

A.8.8.1.2	Test Requirements	1822

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1822

A.8.8.2.1	Test Purpose and Environment	1822

A.8.8.2.2	Test Requirements	1824

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1825

A.8.8.3.1	Test Purpose and Environment	1825

A.8.8.3.2	Test Requirements	1826

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1827

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1827

A.8.9.1.1	Test Purpose and Environment	1827

A.8.9.1.2	Test Requirements	1828

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1829

A.8.9.2.1	Test Purpose and Environment	1829

A.8.9.2.2	Test Requirements	1831

A.8.10	E-UTRAN TDD – GSM Measurements	1831

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1831

A.8.10.1.1	Test Purpose and Environment	1831

A.8.10.1.2	Test Requirements	1832

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1833

A.8.10.2.1	Test Purpose and Environment	1833

A.8.10.2.2	Test Requirements	1835

A.8.11	Monitoring of Multiple Layers	1835

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1835

A.8.11.1.1	Test Purpose and Environment	1835

A.8.11.1.2	Test Requirements	1837

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1838

A.8.11.2.1	Test Purpose and Environment	1838

A.8.11.2.2	Test Requirements	1839

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1839

A.8.11.3.1	Test Purpose and Environment	1839

A.8.11.3.2	Test Requirements	1842

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1842

A.8.11.4.1	Test Purpose and Environment	1842

A.8.11.4.2	Test Requirements	1845

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1845

A.8.11.5.1	Test Purpose and Environment	1845

A.8.11.5.2	Test Requirements	1847

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1848

A.8.11.6.1	Test Purpose and Environment	1848

A.8.11.6.2	Test Requirements	1850

A.8.12	RSTD Intra-frequency Measurements	1851

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1851

A.8.12.1.1	Test Purpose and Environment	1851

A.8.12.1.2	Test Requirements	1854

A.8.12.1.2A	Test Requirements for UE Category 1bis	1855

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1855

A.8.12.2.1	Test Purpose and Environment	1855

A.8.12.2.2	Test Requirements	1859

A.8.12.2.2A	Test Requirements for UE Category 1bis	1860

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1860

A.8.12.3.1	Test Purpose and Environment	1860

A.8.12.3.2	Test Requirements	1864

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1865

A.8.12.4.1	Test Purpose and Environment	1865

A.8.12.4.2	Test Requirements	1868

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1869

A.8.12.5.1	Test Purpose and Environment	1869

A.8.12.5.2	Test Requirements	1872

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1873

A.8.12.6.1	Test Purpose and Environment	1873

A.8.12.6.2	Test Requirements	1876

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1877

A.8.12.7.1	Test Purpose and Environment	1877

A.8.12.7.2	Test Requirements	1880

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1881

A.8.12.8.1	Test Purpose and Environment	1881

A.8.12.8.2	Test Requirements	1884

A.8.13	RSTD Inter-frequency Measurements	1885

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1885

A.8.13.1.1	Test Purpose and Environment	1885

A.8.13.1.2	Test Requirements	1888

A.8.13.1.2A	Test Requirements for UE Category 1bis	1889

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1889

A.8.13.2.1	Test Purpose and Environment	1889

A.8.13.2.2	Test Requirements	1893

A.8.13.2.2A	Test Requirements for UE Category 1bis	1894

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1894

A.8.13.3.1	Test Purpose and Environment	1894

A.8.13.3.2	Test Requirements	1898

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1899

A.8.13.4.1	Test Purpose and Environment	1899

A.8.13.4.2	Test Requirements	1902

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1903

A.8.13.5.1	Test Purpose and Environment	1903

A.8.13.5.2	Test Requirements	1907

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1908

A.8.13.6.1	Test Purpose and Environment	1908

A.8.13.6.2	Test Requirements	1911

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1912

A.8.13.7.1	Test Purpose and Environment	1912

A.8.13.7.2	Test Requirements	1915

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1916

A.8.13.8.1	Test Purpose and Environment	1916

A.8.13.8.2	Test Requirements	1920

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1921

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1921

A.8.14.1.1	Test Purpose and Environment	1921

A.8.14.1.2	Test Requirements	1923

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1923

A.8.14.2.1	Test Purpose and Environment	1923

A.8.14.2.2	Test Requirements	1926

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1926

A.8.14.3.1	Test Purpose and Environment	1926

A.8.14.3.2	Test Requirements	1928

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1929

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1929

A.8.15.1.1	Test Purpose and Environment	1929

A.8.15.1.2	Test Requirements	1930

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1930

A.8.15.2.1	Test Purpose and Environment	1930

A.8.15.2.2	Test Requirements	1933

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1933

A.8.15.3.1	Test Purpose and Environment	1933

A.8.15.3.2	Test Requirements	1935

A.8.16	E-UTRAN Carrier Aggregation Measurements	1936

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1936

A.8.16.1.1	Test Purpose and Environment	1936

A.8.16.1.2	Test Requirements	1938

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1938

A.8.16.2.1	Test Purpose and Environment	1938

A.8.16.2.2	Test Requirements	1940

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1940

A.8.16.3.1	Test Purpose and Environment	1940

A.8.16.3.2	Test Requirements	1942

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1943

A.8.16.3A.1	Test Purpose and Environment	1943

A.8.16.3A.2	Test Requirements	1944

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1945

A.8.16.4.1	Test Purpose and Environment	1945

A.8.16.4.2	Test Requirements	1946

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1947

A.8.16.4A.1	Test Purpose and Environment	1947

A.8.16.4A.2	Test Requirements	1949

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1950

A.8.16.5.1	Test Purpose and Environment	1950

A.8.16.5.2	Test Requirements	1950

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1950

A.8.16.6.1	Test Purpose and Environment	1950

A.8.16.6.2	Test Requirements	1951

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1951

A.8.16.7.1	 Test Purpose and Environment	1951

A.8.16.7.2	 Test Requirements	1952

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1952

A.8.16.8.1	 Test Purpose and Environment	1952

A.8.16.8.2	 Test Requirements	1953

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1953

A.8.16.9.1	Test Purpose and Environment	1953

A.8.16.9.2	Test Requirements	1954

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1954

A.8.16.10.1	Test Purpose and Environment	1954

A.8.16.10.2	Test Requirements	1955

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1955

A.8.16.11.1	 Test Purpose and Environment	1955

A.8.16.11.2	 Test Requirements	1956

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1956

A.8.16.12.1	 Test Purpose and Environment	1956

A.8.16.12.2	 Test Requirements	1956

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1956

A.8.16.13.1	Test Purpose and Environment	1956

A.8.16.13.2	Test Requirements	1957

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1957

A.8.16.14.1	Test Purpose and Environment	1957

A.8.16.14.2	Test Requirements	1958

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1958

A.8.16.15.1	 Test Purpose and Environment	1958

A.8.16.7.2	 Test Requirements	1958

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1958

A.8.16.16.1	 Test Purpose and Environment	1958

A.8.16.16.2	 Test Requirements	1959

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1959

A.8.16.17.1	Test Purpose and Environment	1959

A.8.16.17.2	Test Requirements	1961

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1962

A.8.16.17A.1	Test Purpose and Environment	1962

A.8.16.17A.2	Test Requirements	1962

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1962

A.8.16.17B.1	Test Purpose and Environment	1962

A.8.16.17B.2	Test Requirements	1963

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1963

A.8.16.17C.1	Test Purpose and Environment	1963

A.8.16.17C.2	Test Requirements	1963

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1963

A.8.16.18.1	Test Purpose and Environment	1963

A.8.16.18.2	Test Requirements	1965

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1966

A.8.16.18A.1	Test Purpose and Environment	1966

A.8.16.18A.2	Test Requirements	1966

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1966

A.8.16.18B.1	Test Purpose and Environment	1966

A.8.16.18B.2	Test Requirements	1967

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1967

A.8.16.18C.1	Test Purpose and Environment	1967

A.8.16.18C.2	Test Requirements	1967

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1967

A.8.16.18D.1	Test Purpose and Environment	1967

A.8.16.18D.2	Test Requirements	1968

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1968

A.8.16.19.1	Test Purpose and Environment	1968

A.8.16.19.2	Test Requirements	1970

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1970

A.8.16.19A.1	Test Purpose and Environment	1970

A.8.16.19A.2	Test Requirements	1971

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1971

A.8.16.19B.1	Test Purpose and Environment	1971

A.8.16.19B.2	Test Requirements	1971

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1971

A.8.16.19C.1	Test Purpose and Environment	1971

A.8.16.19C.2	Test Requirements	1972

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1972

A.8.16.20.1	Test Purpose and Environment	1972

A.8.16.20.2	Test Requirements	1974

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1975

A.8.16.20A.1	Test Purpose and Environment	1975

A.8.16.20A.2	Test Requirements	1975

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1975

A.8.16.20B.1	Test Purpose and Environment	1975

A.8.16.20B.2	Test Requirements	1976

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1976

A.8.16.20C.1	Test Purpose and Environment	1976

A.8.16.20C.2	Test Requirements	1976

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1976

A.8.16.20D.1	Test Purpose and Environment	1976

A.8.16.20D.2	Test Requirements	1977

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1977

A.8.16.21.1	Test Purpose and Environment	1977

A.8.16.21.2	Test Requirements	1979

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1979

A.8.16.22.1	Test Purpose and Environment	1979

A.8.16.22.2	Test Requirements	1981

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1981

A.8.16.23.1	Test Purpose and Environment	1981

A.8.16.23.2	Test Requirements	1984

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1984

A.8.16.24.1	Test Purpose and Environment	1984

A.8.16.24.2	Test Requirements	1987

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1987

A.8.16.25.1	Test Purpose and Environment	1987

A.8.16.25.2	Test Requirements	1989

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1989

A.8.16.26.1	Test Purpose and Environment	1989

A.8.16.26.2	Test Requirements	1992

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1992

A.8.16.27.1	Test Purpose and Environment	1992

A.8.16.27.2	Test Requirements	1996

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1996

A.8.16.28.1	Test Purpose and Environment	1996

A.8.16.28.2	Test Requirements	2000

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2000

A.8.16.29.1	Test Purpose and Environment	2000

A.8.16.29.2	Test Requirements	2004

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2004

A.8.16.30.1	Test Purpose and Environment	2004

A.8.16.30.2	Test Requirements	2008

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2008

A.8.16.31.1	Test Purpose and Environment	2008

A.8.16.31.2	Test Requirements	2012

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2012

A.8.16.32.1	Test Purpose and Environment	2012

A.8.16.32.2	Test Requirements	2016

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2016

A.8.16.33.1	Test Purpose and Environment	2016

A.8.16.33.2	Test Requirements	2020

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2020

A.8.16.34.1	Test Purpose and Environment	2020

A.8.16.34.2	Test Requirements	2024

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2024

A.8.16.35.1	Test Purpose and Environment	2024

A.8.16.35.2	Test Requirements	2026

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2027

A.8.16.36.1	Test Purpose and Environment	2027

A.8.16.36.2	Test Requirements	2029

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2030

A.8.16.37.1	Test Purpose and Environment	2030

A.8.16.37.2	Test Requirements	2032

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2033

A.8.16.38.1	Test Purpose and Environment	2033

A.8.16.38.2	Test Requirements	2035

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2036

A.8.16.39.1	Test Purpose and Environment	2036

A.8.16.39.2	Test Requirements	2039

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2039

A.8.16.40.1	Test Purpose and Environment	2039

A.8.16.40.2	Test Requirements	2042

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2042

A.8.16.41.1	Test Purpose and Environment	2042

A.8.16.41.2	Test Requirements	2044

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2045

A.8.16.42.1	Test Purpose and Environment	2045

A.8.16.42.2	Test Requirements	2047

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2048

A.8.16.43.1	Test Purpose and Environment	2048

A.8.16.43.2	Test Requirements	2051

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2051

A.8.16.44.1	Test Purpose and Environment	2051

A.8.16.44.2	Test Requirements	2054

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2054

A.8.16.45.1	Test Purpose and Environment	2054

A.8.16.45.2	Test Requirements	2057

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2057

A.8.16.46.1	Test Purpose and Environment	2057

A.8.16.46.2	Test Requirements	2059

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2060

A.8.16.47.1	Test Purpose and Environment	2060

A.8.16.47.2	Test Requirements	2062

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2063

A.8.16.48.1	Test Purpose and Environment	2063

A.8.16.48.2	Test Requirements	2065

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2066

A.8.16.49.1	Test Purpose and Environment	2066

A.8.16.49.2	Test Requirements	2069

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2069

A.8.16.50.1	Test Purpose and Environment	2069

A.8.16.50.2	Test Requirements	2071

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2072

A.8.16.51.1	Test Purpose and Environment	2072

A.8.16.51.2	Test Requirements	2076

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2076

A.8.16.52.1	Test Purpose and Environment	2076

A.8.16.52.2	Test Requirements	2080

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2080

A.8.16.53.1	Test Purpose and Environment	2080

A.8.16.53.2	Test Requirements	2084

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2084

A.8.16.54.1	Test Purpose and Environment	2084

A.8.16.54.2	Test Requirements	2088

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2088

A.8.16.55.1	Test Purpose and Environment	2088

A.8.16.55.2	Test Requirements	2093

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2093

A.8.16.56.1	Test Purpose and Environment	2093

A.8.16.56.2	Test Requirements	2098

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2098

A.8.16.57.1	Test Purpose and Environment	2098

A.8.16.57.2	Test Requirements	2100

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2101

A.8.16.58.1	Test Purpose and Environment	2101

A.8.16.58.2	Test Requirements	2103

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2104

A.8.16.59.1	Test Purpose and Environment	2104

A.8.16.59.2	Test Requirements	2108

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2108

A.8.16.60.1	Test Purpose and Environment	2108

A.8.16.60.2	Test Requirements	2111

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2112

A.8.16.61.1	Test Purpose and Environment	2112

A.8.16.61.2	Test Requirements	2115

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2115

A.8.16.62.1	Test Purpose and Environment	2115

A.8.16.62.2	Test Requirements	2117

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2118

A.8.16.63.1	Test Purpose and Environment	2118

A.8.16.63.2	Test Requirements	2121

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2122

A.8.16.64.1	Test Purpose and Environment	2122

A.8.16.64.2	Test Requirements	2126

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2126

A.8.16.65.1	Test Purpose and Environment	2126

A.8.16.65.2	Test Requirements	2131

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2131

A.8.16.66.1	Test Purpose and Environment	2131

A.8.16.66.2	Test Requirements	2136

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2136

A.8.16.67.1	Test Purpose and Environment	2136

A.8.16.67.2	Test Requirements	2141

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2141

A.8.16.68.1	Test Purpose and Environment	2141

A.8.16.68.2	Test Requirements	2146

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2146

A.8.16.69.1	Test Purpose and Environment	2146

A.8.16.69.2	Test Requirements	2149

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2149

A.8.16.70.1	Test Purpose and Environment	2149

A.8.16.70.2	Test Requirements	2152

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2152

A.8.16.71.1	Test Purpose and Environment	2152

A.8.16.71.2	Test Requirements	2156

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2156

A.8.16.72.1	Test Purpose and Environment	2156

A.8.16.72.2	Test Requirements	2160

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2160

A.8.16.73.1	Test Purpose and Environment	2160

A.8.16.73.2	Test Requirements	2165

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2165

A.8.16.74.1	Test Purpose and Environment	2165

A.8.16.74.2	Test Requirements	2170

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2170

A.8.16.75.1	Test Purpose and Environment	2170

A.8.16.75.2	Test Requirements	2175

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2175

A.8.16.76.1	Test Purpose and Environment	2175

A.8.16.76.2	Test Requirements	2180

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2180

A.8.16.77.1	Test Purpose and Environment	2180

A.8.16.77.2	Test Requirements	2183

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2183

A.8.16.78.1	Test Purpose and Environment	2183

A.8.16.78.2	Test Requirements	2186

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2187

A.8.16.79.1	Test Purpose and Environment	2187

A.8.16.79.2	Test Requirements	2192

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2192

A.8.16.80.1	Test Purpose and Environment	2192

A.8.16.80.2	Test Requirements	2197

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2197

A.8.16.81.1	Test Purpose and Environment	2197

A.8.16.81.2	Test Requirements	2202

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2202

A.8.16.82.1	Test Purpose and Environment	2202

A.8.16.82.2	Test Requirements	2207

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2207

A.8.16.83.1	Test Purpose and Environment	2207

A.8.16.83.2	Test Requirements	2212

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2212

A.8.16.84.1	Test Purpose and Environment	2212

A.8.16.84.2	Test Requirements	2218

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2218

A.8.16.85.1	Test Purpose and Environment	2218

A.8.16.85.2	Test Requirements	2221

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2221

A.8.16.86.1	Test Purpose and Environment	2221

A.8.16.86.2	Test Requirements	2224

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2224

A.8.16.87.1	Test Purpose and Environment	2224

A.8.16.87.2	Test Requirements	2229

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2229

A.8.16.88.1	Test Purpose and Environment	2229

A.8.16.88.2	Test Requirements	2237

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2237

A.8.16.89.1	Test Purpose and Environment	2237

A.8.16.89.2	Test Requirements	2242

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2242

A.8.16.90.1	Test Purpose and Environment	2242

A.8.16.64.2	Test Requirements	2247

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2247

A.8.16.91.1	Test Purpose and Environment	2247

A.8.16.91.2	Test Requirements	2253

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2253

A.8.16.92.1	Test Purpose and Environment	2253

A.8.16.92.2	Test Requirements	2261

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2261

A.8.16.93.1	Test Purpose and Environment	2261

A.8.16.93.2	Test Requirements	2266

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2266

A.8.16.94.1	Test Purpose and Environment	2266

A.8.16.94.2	Test Requirements	2272

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2272

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2272

A.8.17.1.1	Test Purpose and Environment	2272

A.8.17.1.2	Test Requirements	2278

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2278

A.8.17.2.1	Test Purpose and Environment	2278

A.8.17.2.2	Test Requirements	2284

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2284

A.8.17.3.1	Test Purpose and Environment	2284

A.8.17.3.2	Test Requirements	2285

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2285

A.8.17.4.1	Test Purpose and Environment	2285

A.8.17.4.2	Test Requirements	2286

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2286

A.8.17.5.1	Test Purpose and Environment	2286

A.8.17.5.2	Test Requirements	2287

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2287

A.8.17.6.1	Test Purpose and Environment	2287

A.8.17.6.2	Test Requirements	2288

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2289

A.8.17.7.1	Test Purpose and Environment	2289

A.8.17.7.2	Test Requirements	2289

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2290

A.8.17.8.1	Test Purpose and Environment	2290

A.8.17.8.2	Test Requirements	2290

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2291

A.8.17.9.1	Test Purpose and Environment	2291

A.8.17.9.2	Test Requirements	2292

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2292

A.8.17.10.1	Test Purpose and Environment	2292

A.8.17.10.2	Test Requirements	2297

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2298

A.8.17.11.1	Test Purpose and Environment	2298

A.8.17.11.2	Test Requirements	2304

A.8.18	E-UTRAN TDD – HRPD Measurements	2305

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2305

A.8.18.1.1	Test Purpose and Environment	2305

A.8.18.1.2	Test Requirements	2307

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2307

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2307

A.8.19.1.1	Test Purpose and Environment	2307

A.8.19.1.2	Test Requirements	2308

A.8.20	Inter-frequency/RAT Measurements in CA mode	2309

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2309

A.8.20.1.1	Test Purpose and Environment	2309

A.8.20.1.2	Test Requirements	2310

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2310

A.8.20.2.1	Test Purpose and Environment	2311

A.8.20.2.2	Test Requirements	2312

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2313

A.8.20.2A.1	Test Purpose and Environment	2313

A.8.20.2A.2	Test Requirements	2313

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2313

A.8.20.2B.1	Test Purpose and Environment	2313

A.8.20.2B.2	Test Requirements	2316

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2316

A.8.20.3.1	Test Purpose and Environment	2316

A.8.20.3.2	Test Requirements	2318

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2318

A.8.20.4.1	Test Purpose and Environment	2318

A.8.20.4.1.1	1.28 Mcps TDD option	2318

A.8.20.4.2	Test Requirements	2320

A.8.20.4.2.1	1.28 Mcps TDD option	2320

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2320

A.8.20.4A.1	Test Purpose and Environment	2320

A.8.20.4A.1.1	1.28 Mcps TDD option	2320

A.8.20.4A.2	Test Requirements	2320

A.8.20.4A.2.1	1.28 Mcps TDD option	2320

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2321

A.8.20.4B.1	Test Purpose and Environment	2321

A.8.20.4B.1.1	1.28 Mcps TDD option	2321

A.8.20.4B.2	Test Requirements	2323

A.8.20.4B.2.1	1.28 Mcps TDD option	2323

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2323

A.8.21.1	Test Purpose and Environment	2323

A.8.21.2 Test Requirements	2327

A.8.22	E-UTRAN Discovery Signal Measurements	2327

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2327

A.8.22.1.1	Test Purpose and Environment	2327

A.8.22.1.2	Test Requirements	2330

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2330

A.8.22.2.1	Test Purpose and Environment	2330

A.8.22.2.2	Test Requirements	2333

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2333

A.8.22.3.1	Test Purpose and Environment	2333

A.8.22.3.2	Test Requirements	2336

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2336

A.8.22.4.1	Test Purpose and Environment	2336

A.8.22.4.2	Test Requirements	2339

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2339

A.8.22.5.1	Test Purpose and Environment	2339

A.8.22.5.2	Test Requirements	2342

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2342

A.8.22.6.1	Test Purpose and Environment	2342

A.8.22.6.2	Test Requirements	2345

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2345

A.8.22.7.1	Test Purpose and Environment	2345

A.8.22.7.2	Test Requirements	2348

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2348

A.8.22.8.1	Test Purpose and Environment	2348

A.8.22.8.2	Test Requirements	2351

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2351

A.8.22.9.1	Test Purpose and Environment	2351

A.8.22.9.2	Test Requirements	2353

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2354

A.8.22.10.1	Test Purpose and Environment	2354

A.8.22.10.2	Test Requirements	2356

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2357

A.8.22.11.1	Test Purpose and Environment	2357

A.8.22.11.2	Test Requirements	2359

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2359

A.8.22.12.1	Test Purpose and Environment	2359

A.8.22.12.2	Test Requirements	2362

A.8.23	E-UTRAN Dual Connectivity Measurements	2362

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2362

A.8.23.1.1	Test Purpose and Environment	2362

A.8.23.1.2	Test Requirements	2365

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2365

A.8.23.2.1	Test Purpose and Environment	2365

A.8.23.2.2	Test Requirements	2368

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2368

A.8.23.3.1	Test Purpose and Environment	2368

A.8.23.3.2	Test Requirements	2371

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2371

A.8.23.4.1	Test Purpose and Environment	2371

A.8.23.4.2	Test Requirements	2374

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2374

A.8.23.5.1	Test Purpose and Environment	2374

A.8.23.5.2	Test Requirements	2377

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2377

A.8.23.6.1	Test Purpose and Environment	2377

A.8.23.6.2	Test Requirements	2380

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2380

A.8.23.7.1	Test Purpose and Environment	2380

A.8.23.7.2	Test Requirements	2382

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2383

A.8.23.8.1	Test Purpose and Environment	2383

A.8.23.8.2	Test Requirements	2385

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2386

A.8.23.9.1	Test Purpose and Environment	2386

A.8.23.9.2	Test Requirements	2389

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2389

A.8.23.10.1	Test Purpose and Environment	2389

A.8.23.10.2	Test Requirements	2392

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2392

A.8.23.11.1	Test Purpose and Environment	2392

A.8.23.11.2	Test Requirements	2395

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2395

A.8.23.12.1	Test Purpose and Environment	2395

A.8.23.12.2	Test Requirements	2398

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2398

A.8.23.13.1	Test Purpose and Environment	2398

A.8.23.13.2	Test Requirements	2401

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2401

A.8.23.14.1	Test Purpose and Environment	2401

A.8.23.14.2	Test Requirements	2404

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2404

A.8.23.15.1	Test Purpose and Environment	2404

A.8.23.15.2	Test Requirements	2407

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2407

A.8.23.16.1	Test Purpose and Environment	2407

A.8.23.16.2	Test Requirements	2408

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2409

A.8.23.17.1	Test Purpose and Environment	2409

A.8.23.17.2	Test Requirements	2411

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2411

A.8.23.18.1	Test Purpose and Environment	2411

A.8.23.18.2	Test Requirements	2412

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2413

A.8.23.19.1	Test Purpose and Environment	2413

A.8.23.19.2	Test Requirements	2414

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2415

A.8.23.20.1	Test Purpose and Environment	2415

A.8.23.20.2	Test Requirements	2416

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2417

A.8.23.21.1	Test Purpose and Environment	2417

A.8.23.21.2	Test Requirements	2418

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2419

A.8.23.22.1	Test Purpose and Environment	2419

A.8.23.22.2	Test Requirements	2420

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2421

A.8.23.23.1	Test Purpose and Environment	2421

A.8.23.23.2	Test Requirements	2422

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2423

A.8.23.24.1	Test Purpose and Environment	2423

A.8.23.24.2	Test Requirements	2425

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2426

A.8.23.25.1	Test Purpose and Environment	2426

A.8.23.25.2	Test Requirements	2428

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2429

A.8.23.26.1	Test Purpose and Environment	2429

A.8.23.26.2	Test Requirements	2431

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2432

A.8.23.27.1	Test Purpose and Environment	2432

A.8.23.27.2	Test Requirements	2435

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2435

A.8.23.28.1	Test Purpose and Environment	2435

A.8.23.28.2	Test Requirements	2438

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2438

A.8.23.29.1	Test Purpose and Environment	2438

A.8.23.29.2	Test Requirements	2441

A.8.24	Proximity-based Services	2441

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2441

A.8.24.1.1	Test Purpose and Environment	2441

A.8.24.1.2	Test Requirements	2442

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2443

A.8.24.2.1	Test Purpose and Environment	2443

A.8.24.2.2	Test Requirements	2444

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2444

A.8.24.3.1	Test Purpose and Environment	2445

A.8.24.3.2	Test Requirements	2446

A.8.25	E-UTRAN-WLAN Measurements	2447

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2447

A.8.25.1.1	Test Purpose and Environment	2447

A.8.25.1.2	Test Requirements	2449

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2449

A.8.25.2.1	Test Purpose and Environment	2449

A.8.25.2.2	Test Requirements	2451

A.8.26 Frame Structure 3 (FS3)	2451

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2451

A.8.26.1.1	Test Purpose and Environment	2451

A.8.26.1.2	Test Requirements	2453

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2454

A.8.26.2.1	Test Purpose and Environment	2454

A.8.26.2.2	Test Requirements	2456

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2457

A.8.26.3.1	Test Purpose and Environment	2457

A.8.26.3.2	Test Requirements	2460

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2460

A.8.26.3A.1	Test Purpose and Environment	2460

A.8.26.3A.2	Test Requirements	2463

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2463

A.8.26.4.1	Test Purpose and Environment	2463

A.8.26.4.2	Test Requirements	2466

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2466

A.8.26.4A.1	Test Purpose and Environment	2466

A.8.26.4A.2	Test Requirements	2469

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2469

A.8.26.5.1	Test Purpose and Environment	2469

A.8.26.5.2	Test Requirements	2472

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2472

A.8.26.5A.1	Test Purpose and Environment	2472

A.8.26.5A.2	Test Requirements	2475

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2475

A.8.26.6.1	Test Purpose and Environment	2475

A.8.26.6.2	Test Requirements	2478

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2478

A.8.26.6A.1	Test Purpose and Environment	2478

A.8.26.6A.2	Test Requirements	2481

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2481

A.8.26.7.1	Test Purpose and Environment	2481

A.8.26.7.2	Test Requirements	2484

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2484

A.8.26.8.1	Test Purpose and Environment	2484

A.8.26.8.2	Test Requirements	2487

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2487

A.8.26.9.1	Test Purpose and Environment	2487

A.8.26.9.2	Test Requirements	2490

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2490

A.8.26.10.1	Test Purpose and Environment	2490

A.8.26.10.2	Test Requirements	2493

A.9	Measurement Performance Requirements	2494

A.9.1	RSRP	2494

A.9.1.1	FDD Intra frequency case	2494

A.9.1.1.1	Test Purpose and Environment	2494

A.9.1.1.2	Test parameters	2494

A.9.1.1.3	Test Requirements	2496

A.9.1.2	TDD Intra frequency case	2496

A.9.1.2.1	Test Purpose and Environment	2496

A.9.1.2.2	Test parameters	2496

A.9.1.2.3	Test Requirements	2497

A.9.1.3	FDD—FDD Inter frequency case	2498

A.9.1.3.1	Test Purpose and Environment	2498

A.9.1.3.2	Test parameters	2498

A.9.1.3.3	Test Requirements	2500

A.9.1.4	TDD—TDD Inter frequency case	2500

A.9.1.4.1	Test Purpose and Environment	2500

A.9.1.4.2	Test parameters	2500

A.9.1.4.3	Test Requirements	2502

A.9.1.5	FDD—TDD Inter frequency case	2503

A.9.1.5.1	Test Purpose and Environment	2503

A.9.1.5.2	Test parameters	2503

A.9.1.5.3	Test Requirements	2504

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2505

A.9.1.6.1	Test Purpose and Environment	2505

A.9.1.6.2	Test parameters	2505

A.9.1.6.3	Test Requirements	2507

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2507

A.9.1.7.1	Test Purpose and Environment	2507

A.9.1.7.2	Test parameters	2507

A.9.1.7.3	Test Requirements	2509

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2509

A.9.1.8.1	Test Purpose and Environment	2509

A.9.1.8.2	Test parameters	2509

A.9.1.8.3	Test Requirements	2512

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2512

A.9.1.9.1	Test Purpose and Environment	2512

A.9.1.9.2	Test parameters	2512

A.9.1.9.3	Test Requirements	2514

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2515

A.9.1.10.1	Test Purpose and Environment	2515

A.9.1.10.2	Test parameters	2515

A.9.1.10.3	Test Requirements	2517

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2517

A.9.1.11.1	Test Purpose and Environment	2517

A.9.1.11.2	Test parameters	2517

A.9.1.11.3	Test Requirements	2520

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2520

A.9.1.12.1	Test Purpose and Environment	2520

A.9.1.12.2	Test parameters	2520

A.9.1.12.3	Test Requirements	2521

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2521

A.9.1.13.1	Test Purpose and Environment	2521

A.9.1.13.2	Test parameters	2521

A.9.1.13.3	Test Requirements	2522

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2522

A.9.1.14.1	Test Purpose and Environment	2522

A.9.1.14.2	Test parameters	2522

A.9.1.14.3	Test Requirements	2525

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2525

A.9.1.15.1	Test Purpose and Environment	2525

A.9.1.15.2	Test parameters	2525

A.9.1.15.3	Test Requirements	2527

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2528

A.9.1.16.1	Test Purpose and Environment	2528

A.9.1.16.2	Test parameters	2528

A.9.1.16.3	Test Requirements	2529

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2529

A.9.1.17.1	Test Purpose and Environment	2529

A.9.1.17.2	Test parameters	2530

A.9.1.17.3	Test Requirements	2531

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2531

A.9.1.18.1	Test Purpose and Environment	2531

A.9.1.18.2	Test parameters	2531

A.9.1.18.3	Test Requirements	2532

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2533

A.9.1.19.1	Test Purpose and Environment	2533

A.9.1.19.2	Test parameters	2533

A.9.1.19.3	Test Requirements	2533

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2533

A.9.1.20.1	Test Purpose and Environment	2533

A.9.1.20.2	Test parameters	2533

A.9.1.20.3	Test Requirements	2535

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2535

A.9.1.21.1	Test Purpose and Environment	2535

A.9.1.21.2	Test parameters	2535

A.9.1.21.3	Test Requirements	2535

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2535

A.9.1.22.1	Test Purpose and Environment	2535

A.9.1.22.2	Test parameters	2536

A.9.1.22.3	Test Requirements	2539

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2539

A.9.1.23.1	Test Purpose and Environment	2539

A.9.1.23.2	Test parameters	2539

A.9.1.23.3	Test Requirements	2542

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2542

A.9.1.24.1	Test Purpose and Environment	2542

A.9.1.24.2	Test parameters	2542

A.9.1.24.3	Test Requirements	2543

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2543

A.9.1.25.1	Test Purpose and Environment	2543

A.9.1.25.2	Test parameters	2543

A.9.1.25.3	Test Requirements	2545

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2545

A.9.1.26.1	Test Purpose and Environment	2545

A.9.1.26.2	Test parameters	2545

A.9.1.26.3	Test Requirements	2547

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2547

A.9.1.27.1	Test Purpose and Environment	2547

A.9.1.27.2	Test parameters	2547

A.9.1.27.3	Test Requirements	2549

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2549

A.9.1.28.1	Test Purpose and Environment	2549

A.9.1.28.2	Test parameters	2549

A.9.1.28.3	Test Requirements	2551

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2551

A.9.1.29.1	Test Purpose and Environment	2551

A.9.1.29.2	Test parameters	2551

A.9.1.29.3	Test Requirements	2553

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2553

A.9.1.30.1	Test Purpose and Environment	2553

A.9.1.30.2	Test parameters	2554

A.9.1.30.3	Test Requirements	2556

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2556

A.9.1.31.1	Test Purpose and Environment	2556

A.9.1.31.2	Test parameters	2556

A.9.1.31.3	Test Requirements	2558

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2559

A.9.1.32.1	Test Purpose and Environment	2559

A.9.1.32.2	Test parameters	2559

A.9.1.32.3	Test Requirements	2561

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2561

A.9.1.33.1	Test Purpose and Environment	2561

A.9.1.33.2	Test parameters	2561

A.9.1.33.3	Test Requirements	2563

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2563

A.9.1.34.1	Test Purpose and Environment	2563

A.9.1.34.2	Test parameters	2563

A.9.1.34.3	Test Requirements	2565

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2565

A.9.1.35.1	Test Purpose and Environment	2565

A.9.1.35.2	Test parameters	2565

A.9.1.35.3	Test Requirements	2567

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2568

A.9.1.36.1	Test Purpose and Environment	2568

A.9.1.36.2	Test parameters	2568

A.9.1.36.3	Test Requirements	2570

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2570

A.9.1.37.1	Test Purpose and Environment	2570

A.9.1.37.2	Test parameters	2570

A.9.1.37.3	Test Requirements	2572

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2573

A.9.1.38.1	Test Purpose and Environment	2573

A.9.1.38.2	Test parameters	2573

A.9.1.38.3	Test Requirements	2575

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2576

A.9.1.39.1	Test Purpose and Environment	2576

A.9.1.39.2	Test parameters	2576

A.9.1.39.3	Test Requirements	2580

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2580

A.9.1.40.1	Test Purpose and Environment	2580

A.9.1.40.2	Test parameters	2581

A.9.1.40.3	Test Requirements	2585

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2585

A.9.1.41.1	Test Purpose and Environment	2585

A.9.1.41.2	Test parameters	2585

A.9.1.41.3	Test Requirements	2587

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2587

A.9.1.42.1	Test Purpose and Environment	2587

A.9.1.42.2	Test parameters	2587

A.9.1.42.3	Test Requirements	2589

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2589

A.9.1.43.1	Test Purpose and Environment	2589

A.9.1.43.2	Test parameters	2589

A.9.1.43.3	Test Requirements	2590

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2591

A.9.1.44.1	Test Purpose and Environment	2591

A.9.1.44.2	Test parameters	2591

A.9.1.44.3	Test Requirements	2595

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2595

A.9.1.45.1	Test Purpose and Environment	2595

A.9.1.45.2	Test parameters	2595

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2422

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2423

A.9.1.47.1	Test Purpose and Environment	2423

A.9.1.47.2	Test parameters	2423

A.9.1.47.3	Test Requirements	2429

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2429

A.9.1.48.1	Test Purpose and Environment	2429

A.9.1.48.2	Test parameters	2430

A.9.1.48.3	Test Requirements	2437

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2438

A.9.1.49.1	Test Purpose and Environment	2438

A.9.1.49.2	Test parameters	2438

A.9.1.49.3	Test Requirements	2445

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2446

A.9.1.50.1	Test Purpose and Environment	2446

A.9.1.50.2	Test parameters	2446

A.9.1.50.3	Test Requirements	2451

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2451

A.9.1.51.1	Test Purpose and Environment	2451

A.9.1.51.2	Test parameters	2451

A.9.1.51.3	Test Requirements	2455

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2456

A.9.1.52.1	Test Purpose and Environment	2456

A.9.1.52.2	Test parameters	2456

A.9.1.52.3	Test Requirements	2458

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2458

A.9.1.52A.1	Test Purpose and Environment	2458

A.9.1.52A.2	Test parameters	2458

A.9.1.52A.3	Test Requirements	2459

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2459

A.9.1.53.1	Test Purpose and Environment	2459

A.9.1.53.2	Test parameters	2460

A.9.1.53.3	Test Requirements	2462

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2462

A.9.1.53A.1	Test Purpose and Environment	2462

A.9.1.53A.2	Test parameters	2462

A.9.1.53A.3	Test Requirements	2463

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2463

A.9.1.54.1	Test Purpose and Environment	2463

A.9.1.54.2	Test parameters	2464

A.9.1.54.3	Test Requirements	2466

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2466

A.9.1.55.1	Test Purpose and Environment	2466

A.9.1.55.2	Test parameters	2466

A.9.1.55.3	Test Requirements	2469

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2469

A.9.1.56.1	Test Purpose and Environment	2469

A.9.1.56.2	Test parameters	2469

A.9.1.56.3	Test Requirements	2471

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2472

A.9.1.57.1	Test Purpose and Environment	2472

A.9.1.57.2	Test parameters	2472

A.9.1.57.3	Test Requirements	2474

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2474

A.9.1.57A.1	Test Purpose and Environment	2474

A.9.1.57A.2	Test parameters	2474

A.9.1.57A.3	Test Requirements	2475

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2475

A.9.1.58.1	Test Purpose and Environment	2475

A.9.1.58.2	Test parameters	2476

A.9.1.58.3	Test Requirements	2478

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2478

A.9.1.58A.1	Test Purpose and Environment	2478

A.9.1.58A.2	Test parameters	2478

A.9.1.58A.3	Test Requirements	2479

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2479

A.9.1.59.1	Test Purpose and Environment	2479

A.9.1.59.2	Test parameters	2480

A.9.1.59.3	Test Requirements	2482

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2482

A.9.1.60.1	Test Purpose and Environment	2482

A.9.1.60.2	Test parameters	2482

A.9.1.60.3	Test Requirements	2484

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2485

A.9.1.61.1	Test Purpose and Environment	2485

A.9.1.61.2	Test parameters	2485

A.9.1.61.3	Test Requirements	2487

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2487

A.9.1.62.1	Test Purpose and Environment	2487

A.9.1.62.2	Test parameters	2487

A.9.1.62.3	Test Requirements	2489

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2489

A.9.1.53.1	Test Purpose and Environment	2489

A.9.1.53.2	Test parameters	2489

A.9.1.63.3	Test Requirements	2491

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2491

A.9.1.64.1	Test Purpose and Environment	2491

A.9.1.64.2	Test parameters	2491

A.9.1.64.3	Test Requirements	2492

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.65.1	Test Purpose and Environment	2493

A.9.1.65.2	Test parameters	2493

A.9.1.65.3	Test Requirements	2497

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.66.1	Test Purpose and Environment	2497

A.9.1.66.2	Test parameters	2497

A.9.1.66.3	Test Requirements	2501

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2501

A.9.1.67.1	Test Purpose and Environment	2501

A.9.1.67.2	Test parameters	2501

A.9.1.67.3	Test Requirements	2502

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2503

A.9.1.68.1	Test Purpose and Environment	2503

A.9.1.68.2	Test parameters	2503

A.9.1.68.3	Test Requirements	2506

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2506

A.9.1.69.1	Test Purpose and Environment	2506

A.9.1.69.2	Test parameters	2506

A.9.1.69.3	Test Requirements	2509

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2510

A.9.1.70.1	Test Purpose and Environment	2510

A.9.1.70.2	Test parameters	2510

A.9.1.70.3	Test Requirements	2519

A.9.2	RSRQ	2519

A.9.2.1	FDD Intra frequency case	2519

A.9.2.1.1	Test Purpose and Environment	2519

A.9.2.1.2	Test parameters	2520

A.9.2.1.3	Test Requirements	2522

A.9.2.2	TDD Intra frequency case	2522

A.9.2.2.1	Test Purpose and Environment	2522

A.9.2.2.2	Test parameters	2522

A.9.2.2.3	Test Requirements	2524

A.9.2.3	FDD—FDD Inter frequency case	2524

A.9.2.3.1	Test Purpose and Environment	2524

A.9.2.3.2	Test parameters	2524

A.9.2.3.3	Test Requirements	2526

A.9.2.4	TDD—TDD Inter frequency case	2526

A.9.2.4.1	Test Purpose and Environment	2526

A.9.2.4.2	Test parameters	2526

A.9.2.4.3	Test Requirements	2528

A.9.2.4A	FDD—TDD Inter frequency case	2529

A.9.2.4A.1	Test Purpose and Environment	2529

A.9.2.4A.2	Test parameters	2529

A.9.2.4A.3	Test Requirements	2531

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2531

A.9.2.5.1	Test Purpose and Environment	2531

A.9.2.5.2	Test parameters	2532

A.9.2.5.3	Test Requirements	2534

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2534

A.9.2.6.1	Test Purpose and Environment	2534

A.9.2.6.2	Test parameters	2535

A.9.2.6.3	Test Requirements	2537

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2537

A.9.2.7.1	Test Purpose and Environment	2537

A.9.2.7.2	Test parameters	2537

A.9.2.7.3	Test Requirements	2540

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2540

A.9.2.8.1	Test Purpose and Environment	2540

A.9.2.8.2	Test parameters	2540

A.9.2.8.3	Test Requirements	2543

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2543

A.9.2.9.1	Test Purpose and Environment	2543

A.9.2.9.2	Test parameters	2543

A.9.2.9.3	Test Requirements	2546

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2546

A.9.2.10.1	Test Purpose and Environment	2546

A.9.2.10.2	Test parameters	2547

A.9.2.10.3	Test Requirements	2549

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2549

A.9.2.11.1	Test Purpose and Environment	2549

A.9.2.11.2	Test parameters	2549

A.9.2.11.3	Test Requirements	2550

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2550

A.9.2.12.1	Test Purpose and Environment	2550

A.9.2.12.2	Test parameters	2551

A.9.2.12.3	Test Requirements	2551

A.9.2.13	Void	2552

A.9.2.13.1	Void	2552

A.9.2.13.2	Void	2552

A.9.2.13.3	Void	2552

A.9.2.14	Void	2552

A.9.2.14.1	Void	2552

A.9.2.14.2	Void	2552

A.9.2.14.3	Void	2552

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2552

A.9.2.15.1	Test Purpose and Environment	2552

A.9.2.15.2	Test parameters	2552

A.9.2.15.3	Test Requirements	2555

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2555

A.9.2.16.1	Test Purpose and Environment	2555

A.9.2.16.2	Test parameters	2555

A.9.2.16.3	Test Requirements	2559

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2559

A.9.2.17.1	Test Purpose and Environment	2559

A.9.2.17.2	Test parameters	2559

A.9.2.17.3	Test Requirements	2561

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2561

A.9.2.18.1	Test Purpose and Environment	2561

A.9.2.18.2	Test parameters	2561

A.9.2.18.3	Test Requirements	2563

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2563

A.9.2.19.1	Test Purpose and Environment	2563

A.9.2.19.2	Test parameters	2563

A.9.2.19.3	Test Requirements	2565

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2565

A.9.2.20.1	Test Purpose and Environment	2565

A.9.2.20.2	Test parameters	2565

A.9.2.20.3	Test Requirements	2567

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2567

A.9.2.21.1	Test Purpose and Environment	2567

A.9.2.21.2	Test parameters	2567

A.9.2.21.3	Test Requirements	2569

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2569

A.9.2.22.1	Test Purpose and Environment	2570

A.9.2.22.2	Test parameters	2570

A.9.2.22.3	Test Requirements	2570

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2570

A.9.2.23.1	Test Purpose and Environment	2570

A.9.2.23.2	Test parameters	2570

A.9.2.23.3	Test Requirements	2572

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2572

A.9.2.24.1	Test Purpose and Environment	2572

A.9.2.24.2	Test parameters	2572

A.9.2.24.3	Test Requirements	2573

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2573

A.9.2.25.1	Test Purpose and Environment	2573

A.9.2.25.2	Test parameters	2573

A.9.2.25.3	Test Requirements	2577

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2577

A.9.2.26.1	Test Purpose and Environment	2577

A.9.2.26.2	Test parameters	2577

A.9.2.26.3	Test Requirements	2580

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2580

A.9.2.27.1	Test Purpose and Environment	2580

A.9.2.27.2	Test parameters	2580

A.9.2.27.3	Test Requirements	2581

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2581

A.9.2.28.1	Test Purpose and Environment	2581

A.9.2.28.2	Test parameters	2581

A.9.2.28.3	Test Requirements	2583

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2583

A.9.2.29.1	Test Purpose and Environment	2583

A.9.2.29.2	Test parameters	2583

A.9.2.29.3	Test Requirements	2585

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2585

A.9.2.30.1	Test Purpose and Environment	2585

A.9.2.30.2	Test parameters	2585

A.9.2.30.3	Test Requirements	2587

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2587

A.9.2.31.1	Test Purpose and Environment	2587

A.9.2.31.2	Test parameters	2587

A.9.2.31.3	Test Requirements	2589

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2589

A.9.2.32.1	Test Purpose and Environment	2589

A.9.2.32.2	Test parameters	2589

A.9.2.32.3	Test Requirements	2591

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2591

A.9.2.33.1	Test Purpose and Environment	2591

A.9.2.33.2	Test parameters	2591

A.9.2.33.3	Test Requirements	2594

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2594

A.9.2.34.1	Test Purpose and Environment	2594

A.9.2.34.2	Test parameters	2594

A.9.2.34.3	Test Requirements	2596

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2596

A.9.2.35.1	Test Purpose and Environment	2596

A.9.2.35.2	Test parameters	2596

A.9.2.35.3	Test Requirements	2598

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2598

A.9.2.36.1	Test Purpose and Environment	2598

A.9.2.36.2	Test parameters	2598

A.9.2.36.3	Test Requirements	2599

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2600

A.9.2.37.1	Test Purpose and Environment	2600

A.9.2.37.2	Test parameters	2600

A.9.2.37.3	Test Requirements	2601

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2602

A.9.2.38.1	Test Purpose and Environment	2602

A.9.2.38.2	Test parameters	2602

A.9.2.38.3	Test Requirements	2605

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2605

A.9.2.39.1	Test Purpose and Environment	2605

A.9.2.39.2	Test parameters	2605

A.9.2.39.3	Test Requirements	2608

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2608

A.9.2.40.1	Test Purpose and Environment	2608

A.9.2.40.2	Test parameters	2608

A.9.2.40.3	Test Requirements	2610

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2611

A.9.2.41.1	Test Purpose and Environment	2611

A.9.2.41.2	Test parameters	2611

A.9.2.41.3	Test Requirements	2613

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2613

A.9.2.42.1	Test Purpose and Environment	2613

A.9.2.42.2	Test parameters	2613

A.9.2.42.3	Test Requirements	2615

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2615

A.9.2.43.1	Test Purpose and Environment	2615

A.9.2.43.2	Test parameters	2615

A.9.2.43.3	Test Requirements	2617

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2617

A.9.2.44.1	Test Purpose and Environment	2617

A.9.2.44.2	Test parameters	2617

A.9.2.44.3	Test Requirements	2619

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2619

A.9.2.45.1	Test Purpose and Environment	2619

A.9.2.45.2	Test parameters	2619

A.9.2.45.3	Test Requirements	2622

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2623

A.9.2.46.1	Test Purpose and Environment	2623

A.9.2.46.2	Test parameters	2623

A.9.2.46.3	Test Requirements	2626

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2627

A.9.2.47.1	Test Purpose and Environment	2627

A.9.2.47.2	Test parameters	2627

A.9.2.47.3	Test Requirements	2630

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2630

A.9.2.48.1	Test Purpose and Environment	2630

A.9.2.48.2	Test parameters	2630

A.9.2.48.3	Test Requirements	2633

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2633

A.9.2.49.1	Test Purpose and Environment	2633

A.9.2.49.2	Test parameters	2633

A.9.2.49.3	Test Requirements	2637

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2638

A.9.2.50.1	Test Purpose and Environment	2638

A.9.2.50.2	Test parameters	2638

A.9.2.50.3	Test Requirements	2642

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2642

A.9.2.51.1	Test Purpose and Environment	2642

A.9.2.51.2	Test parameters	2643

A.9.2.51.3	Test Requirements	2646

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2646

A.9.2.52.1	Test Purpose and Environment	2646

A.9.2.52.2	Test parameters	2646

A.9.2.52.3	Test Requirements	2648

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2649

A.9.2.53.1	Test Purpose and Environment	2649

A.9.2.53.2	Test parameters	2649

A.9.2.53.3	Test Requirements	2653

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2654

A.9.2.54.1	Test Purpose and Environment	2654

A.9.2.54.2	Test parameters	2654

A.9.2.54.3	Test Requirements	2658

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2658

A.9.2.55.1	Test Purpose and Environment	2658

A.9.2.55.2	Test parameters	2659

A.9.2.55.3	Test Requirements	2662

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2663

A.9.2.56.1	Test Purpose and Environment	2663

A.9.2.56.2	Test parameters	2663

A.9.2.56.3	Test Requirements	2667

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2667

A.9.2.57.1	Test Purpose and Environment	2667

A.9.2.57.2	Test parameters	2667

A.9.2.57.3	Test Requirements	2670

A.9.3	UTRAN FDD CPICH RSCP	2671

A.9.3.1	E-UTRAN FDD	2671

A.9.3.1.1	Test Purpose and Environment	2671

A.9.3.1.2	Parameters	2671

A.9.3.1.3	Test Requirements	2673

A.9.3.2 E-UTRAN TDD	2673

A.9.3.2.1	Test Purpose and Environment	2673

A.9.3.2.2	Parameters	2673

A.9.3.2.3	Test Requirements	2676

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2676

A.9.3.3.1	Test Purpose and Environment	2676

A.9.3.3.2	Parameters	2676

A.9.3.3.3	Test Requirements	2677

A.9.4	UTRAN FDD CPICH Ec/No	2678

A.9.4.1	E-UTRAN FDD	2678

A.9.4.1.1	Test Purpose and Environment	2678

A.9.4.1.2	Parameters	2678

A.9.4.1.3	Test Requirements	2680

A.9.4.2	E-UTRAN TDD	2681

A.9.4.2.1	Test Purpose and Environment	2681

A.9.4.2.2	Parameters	2681

A.9.4.2.3	Test Requirements	2683

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2684

A.9.4.3.1	Test Purpose and Environment	2684

A.9.4.3.2	Parameters	2684

A.9.4.3.3	Test Requirements	2685

A.9.5	UTRAN TDD measurement	2685

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2685

A.9.5.1.1	Test Purpose and Environment	2685

A.9.5.1.2	Test parameters	2685

A.9.5.1.3	Test Requirements	2687

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2687

A.9.5.2.1	Test Purpose and Environment	2687

A.9.5.2.2	Test parameters	2687

A.9.5.2.3	Test Requirements	2689

A.9.6	GSM Carrier RSSI	2690

A.9.6.1	E-UTRAN FDD	2690

A.9.6.1.1	Test Purpose and Environment	2690

A.9.6.1.2	Test Requirements	2691

A.9.6.2	E-UTRAN TDD	2691

A.9.6.2.1	Test Purpose and Environment	2691

A.9.6.2.2	Test Requirements	2694

A.9.7	UE Rx – Tx Time Difference	2694

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2694

A.9.7.1.1	Test Purpose and Environment	2694

A.9.7.1.2	Test parameters	2694

A.9.7.1.3	Test Requirements	2696

A.9.7.2	E-UTRA TDD	2696

A.9.7.2.1	Test Purpose and Environment	2696

A.9.7.2.2	Test parameters	2696

A.9.7.2.3	Test Requirements	2698

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2698

A.9.7.3.1	Test Purpose and Environment	2698

A.9.7.3.2	Test parameters	2698

A.9.7.3.3	Test Requirements	2701

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2701

A.9.7.4.1	Test Purpose and Environment	2701

A.9.7.4.2	Test Parameters	2701

A.9.7.4.3	Test Requirements	2704

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2704

A.9.7.5.1	Test Purpose and Environment	2704

A.9.7.5.2	Test parameters	2704

A.9.7.5.3	Test Requirements	2707

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2707

A.9.7.6.1	Test Purpose and Environment	2707

A.9.7.6.2	Test Parameters	2707

A.9.7.6.3	Test Requirements	2710

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2710

A.9.7.7.1	Test Purpose and Environment	2710

A.9.7.7.2	Test parameters	2710

A.9.7.7.3	Test Requirements	2711

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2712

A.9.7.8.1	Test Purpose and Environment	2712

A.9.7.8.2	Test parameters	2712

A.9.7.8.3	Test Requirements	2713

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2713

A.9.7.9.1	Test Purpose and Environment	2713

A.9.7.9.2	Test parameters	2713

A.9.7.9.3	Test Requirements	2714

A.9.8	RSTD	2715

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2715

A.9.8.1.1	Test Purpose and Environment	2715

A.9.8.1.2	Test Requirements	2717

A.9.8.1.2A	Test Requirements for UE Category 1bis	2717

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2718

A.9.8.2.1	Test Purpose and Environment	2718

A.9.8.2.2	Test Requirements	2720

A.9.8.2.2A	Test Requirements for UE Category 1bis	2720

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2721

A.9.8.3.1	Test Purpose and Environment	2721

A.9.8.3.2	Test Requirements	2723

A.9.8.3.2A	Test Requirements for UE Category 1bis	2723

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2724

A.9.8.4.1	Test Purpose and Environment	2724

A.9.8.4.2	Test Requirements	2726

A.9.8.4.2A	Test Requirements for UE Category 1bis	2726

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2727

A.9.8.5.1	Test Purpose and Environment	2727

A.9.8.5.2	Test Requirements	2729

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2729

A.9.8.6.1	Test Purpose and Environment	2729

A.9.8.6.2	Test Requirements	2732

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2732

A.9.8.7.1	Test Purpose and Environment	2732

A.9.8.7.2	Test Requirements	2733

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2733

A.9.8.8.1	Test Purpose and Environment	2733

A.9.8.8.2	Test Requirements	2734

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2734

A.9.8.9.1	Test Purpose and Environment	2734

A.9.8.9.2	Test Requirements	2735

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2735

A.9.8.10.1	Test Purpose and Environment	2735

A.9.8.10.2	Test Requirements	2735

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2735

A.9.8.11.1	Test Purpose and Environment	2735

A.9.8.11.2	Test Requirements	2736

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2736

A.9.8.12.1	Test Purpose and Environment	2736

A.9.8.12.2	Test Requirements	2737

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2737

A.9.8.13.1	Test Purpose and Environment	2737

A.9.8.13.2	Test Requirements	2738

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2738

A.9.8.14.1	Test Purpose and Environment	2738

A.9.8.14.2	Test Requirements	2743

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2743

A.9.8.15.1	Test Purpose and Environment	2743

A.9.8.15.2	Test Requirements	2747

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2747

A.9.8.16.1	Test Purpose and Environment	2747

A.9.8.16.2	Test Requirements	2750

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2750

A.9.8.17.1	Test Purpose and Environment	2750

A.9.8.17.2	Test Requirements	2753

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2753

A.9.8.18.1	Test Purpose and Environment	2753

A.9.8.18.2	Test Requirements	2756

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2756

A.9.8.19.1	Test Purpose and Environment	2756

A.9.8.19.2	Test Requirements	2759

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2759

A.9.8.20.1	Test Purpose and Environment	2759

A.9.8.20.2	Test Requirements	2761

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2761

A.9.8.21.1	Test Purpose and Environment	2761

A.9.8.21.2	Test Requirements	2764

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2764

A.9.8.22.1	Test Purpose and Environment	2764

A.9.8.22.2	Test Requirements	2767

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2768

A.9.8.23.1	Test Purpose and Environment	2768

A.9.8.23.2	Test Requirements	2770

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2770

A.9.8.24.1	Test Purpose and Environment	2770

A.9.8.24.2	Test Requirements	2773

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2773

A.9.8.25.1	Test Purpose and Environment	2773

A.9.8.25.2	Test Requirements	2776

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2777

A.9.8.26.1	Test Purpose and Environment	2777

A.9.8.26.2	Test Requirements	2779

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2779

A.9.8.27.1	Test Purpose and Environment	2779

A.9.8.27.2	Test Requirements	2782

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2782

A.9.8.28.1	Test Purpose and Environment	2782

A.9.8.28.2	Test Requirements	2785

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2785

A.9.8.29.1	Test Purpose and Environment	2785

A.9.8.29.2	Test Requirements	2788

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2789

A.9.8.30.1	Test Purpose and Environment	2789

A.9.8.30.2	Test Requirements	2791

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2792

A.9.8.31.1	Test Purpose and Environment	2792

A.9.8.31.2	Test Requirements	2795

A.9.9	RSRP and RSRQ on the serving cell	2795

A.9.9.1	FDD Intra frequency serving cell case	2795

A.9.9.1.1	Test Purpose and Environment	2795

A.9.9.1.2	Test parameters	2795

A.9.9.1.3	Test Requirements	2797

A.9.9.2	TDD Intra frequency serving cell case	2797

A.9.9.2.1	Test Purpose and Environment	2797

A.9.9.2.2	Test parameters	2797

A.9.9.2.3	Test Requirements	2799

A.9.10	SSTD	2799

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2799

A.9.10.1.1	Test Purpose and Environment	2799

A.9.10.1.2	Test parameters	2799

A.9.10.1.3	Test Requirements	2801

A.9.10.3	FDD—FDD Inter frequency case	2801

A.9.10.3.1	Test Purpose and Environment	2801

A.9.10.3.2	Test parameters	2801

A.9.10.3.3	Test Requirements	2805

A.9.10.4	TDD—TDD Inter frequency case	2805

A.9.10.4.1	Test Purpose and Environment	2805

A.9.10.4.2	Test parameters	2805

A.9.10.4.3	Test Requirements	2810

A.9.11 RSSI	2810

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2810

A.9.11.1.1	Test Purpose and Environment	2810

A.9.11.1.2	Test parameters	2810

A.9.11.1.3	Test Requirements	2812

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2812

A.9.11.2.1	Test Purpose and Environment	2812

A.9.11.2.2	Test parameters	2812

A.9.12	Channel occupancy	2814

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2814

A.9.12.1.1	Test Purpose and Environment	2814

A.9.12.1.2	Test parameters	2814

A.9.12.1.3	Test Requirements	2816

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2816

A.9.12.2.1	Test Purpose and Environment	2816

A.9.12.2.2	Test parameters	2816

A.9.12.2.3	Test Requirements	2818

A.9.13	RS-SINR	2818

A.9.13.1	FDD Intra-Frequency Case	2818

A.9.13.1.1	Test Purpose and Environment	2818

A.9.13.1.2	Test parameters	2818

A.9.13.1.3	Test Requirements	2820

A.9.13.2	TDD Intra-Frequency Case	2820

A.9.13.2.1	Test Purpose and Environment	2820

A.9.13.2.2	Test parameters	2820

A.9.13.2.3	Test Requirements	2823

A.9.13.5	FDD—TDD Inter frequency case	2823

A.9.13.5.1	Test Purpose and Environment	2823

A.9.13.5.2	Test parameters	2823

A.9.13.5.3	Test Requirements	2827

A.9.13.6	TDD—FDD Inter frequency case	2827

A.9.13.6.1	Test Purpose and Environment	2827

A.9.13.6.2	Test parameters	2827

A.9.13.6.3	Test Requirements	2830

A.10	Proximity-based Services in Any Cell Selection State	2831

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2831

A.10.1.1	Test Purpose and Environment	2831

A.10.1.2	Test Requirements	2832

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2832

A.10.2.1	Test Purpose and Environment	2832

A.10.2.2	Test Requirements	2833

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2834

A.10.3.1	Test Purpose and Environment	2834

A.10.3.2	Test Requirements	2835

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2836

A.10.4.1	Test Purpose and Environment	2836

A.10.4.2	Test Requirements	2838

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2838

A.11.1	V2V UE Transmission Timing Accuracy Test	2838

A.11.1.1	Test Purpose and Environment	2838

A.11.1.2	Test requirements	2839

A.11.2	Interruptions due to V2V sidelink communication	2839

A.11.2.1	Test Purpose and Environment	2839

A.11.2.2	Test Requirements	2841

A.12	2841

A.12.1	V2X UE Transmission Timing Accuracy Test	2841

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2841

A.12.1.1.1	Test Purpose and Environment	2841

A.12.1.1.2	Test requirements	2842

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	2843

A.12.1.2.1	Test Purpose and Environment	2843

A.12.1.2.2	Test Requirements	2843

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2844

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	2844

A.12.2.1.1	Test Purpose and Environment	2844

A.12.2.1.2	Test Requirements	2845

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	2846

A.12.2.2.1	Test Purpose and Environment	2846

A.12.2.1.2	Test Requirements	2847

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	2847

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	2847

A.12.3.1.1	Test Purpose and Environment	2847

A.12.3.1.2	Test Requirements	2849

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	2850

A.12.3.2.1	Test Purpose and Environment	2850

A.12.3.1.2	Test Requirements	2852

A.12.4	Congestion Control Measurement Test for V2X UE	2852

A.12.4.1	Test Purpose and Environment	2852

A.12.4.2	Test Requirements	2854

A.12.5	Interruptions due to V2X Sidelink Communication	2854

A.12.5.1	Test Purpose and Environment	2854

A.12.5.1	Test Requirements	2856

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	2856

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	2856

A.12.6.1.1	Test Purpose and Environment	2856

A.12.6.1.2	Test Requirements	2858

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	2858

A.12.6.2.1	Test Purpose and Environment	2858

A.12.6.1.2	Test Requirements	2860

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	2861

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2861

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2861

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2861

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE supporting coverage enhancement	2861

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2863

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2864

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	2864

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	2864

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	2864

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	2866

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE supporting coverage enhancement	2867

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2867

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2867

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2868

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2868

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2869

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2869

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2869

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2870

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2870

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2871

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2871

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2871

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2871

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2872

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2872

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2872

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2873

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2873

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2873

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2874

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2874

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2875

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	2876

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	2877

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	2878

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	2879

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	2879

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	2879

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	2879

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	2880

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	2880

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	2882

B.3	Conditions for measurements performance requirements for UE	2882

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2882

B.3.2	Void	2882

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2882

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2882

B.3.5	Conditions for UE Rx – Tx time difference	2882

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2883

B.3.7	Conditions for inter-frequency RSTD measurements	2883

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2883

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2883

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2883

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2884

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2884

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2884

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2884

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2884

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2884

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2885

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2885

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2885

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2886

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2886

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2886

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2886

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2886

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2886

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2886

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2886

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2886

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2887

B.3.21.1	Conditions for RSRP measurements	2887

B.3.21.2	Conditions for RSRQ measurements	2887

B.3.21.3	Conditions for CSI-RSRP measurements	2887

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2887

B.3.22.1	Conditions for RSRP measurements	2887

B.3.22.2	Void	2888

B.3.22.3	Conditions for CSI-RSRP measurements	2888

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2888

B.3.23.1	Conditions for RSRP measurements	2888

B.3.23.2	Conditions for RSRQ measurements	2888

B.3.23.3	Conditions for CSI-RSRP measurements	2888

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2889

B.3.24.1	Conditions for RSRP measurements	2889

B.3.24.2	Conditions for RSRQ measurements	2889

B.3.24.3	Conditions for CSI-RSRP measurements	2889

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2889

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2889

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	2889

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	2890

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2890

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2890

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2890

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2890

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2891

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2891

B.4	RRM Requirements Exceptions	2891

B.4.1	General	2891

B.4.2	Receiver sensitivity relaxation for UE supporting CA	2891

B.4.3	Receiver sensitivity relaxation for UE configured with CA	2891

B.4.3.1	Inter-band carrier aggregation	2891

B.4.3.2	Intra-band non-contiguous carrier aggregation	2891

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	2892

B.5	Conditions for Measurement Performance Requirements for ProSe UE	2892

B.5.1	Conditions for S-RSRP Accuracy Requirements	2892

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	2892

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2893

B.5.4	Conditions for SD-RSRP Accuracy Requirements	2893

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	2894

B.6	Conditions for V2X	2894

B.6.1	Test parameters for GNSS signals	2894

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	2894

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	2894

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2895

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	2895

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	2895

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	2895

Annex C (informative):	Change history:	2897