3GPP TS 36.133 V15.5.0 (2018-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	86

1	Scope	87

2	References	87

3	Definitions, symbols and abbreviations	89

3.1	Definitions	89

3.2	Symbols	91

3.3	Abbreviations	92

3.4	Test tolerances	94

3.5	Additional notation	94

3.5.1	Groups of bands	94

3.6	General	96

3.6.1	Applicability of requirements in this specification version	96

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	103

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	105

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	106

4	E-UTRAN RRC_IDLE state mobility	107

4.1	Cell Selection	107

4.2	Cell Re-selection	107

4.2.1	Introduction	107

4.2.2	Requirements	107

4.2.2.1	Measurement and evaluation of serving cell	108

4.2.2.2	Void	109

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	109

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	111

4.2.2.5	Measurements of inter-RAT cells	113

4.2.2.5.1	Measurements of UTRAN FDD cells	113

4.2.2.5.2	Measurements of UTRAN TDD cells	115

4.2.2.5.3	Measurements of GSM cells	116

4.2.2.5.4	Measurements of HRPD cells	117

4.2.2.5.5	Measurements of cdma2000 1X	118

4.2.2.5.6	Measurements of NR cells	119

4.2.2.6	Evaluation of cell re-selection criteria	120

4.2.2.7	Maximum interruption in paging reception	121

4.2.2.8	void	121

4.2.2.9	UE measurement capability	121

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	122

4.2.2.10	Reselection to CSG cells	122

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	122

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	123

4.2.2.11	Void	124

4.2.2.12	Void	124

4.2.2.13	Void	124

4.3	Minimization of Drive Tests (MDT)	124

4.3.1	Introduction	125

4.3.2	Measurements	125

4.3.2.1	Requirements	125

4.3.3	Relative Time Stamp Accuracy	125

4.3.3.1	Requirements	125

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	125

4.3.4.1	 Requirements	125

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	126

4.3.5.1	Requirements for timeSinceFailure	126

4.4	MBSFN Measurements	126

4.4.1	Introduction	126

4.4.2	MBSFN RSRP measurements	126

4.4.3	MBSFN RSRQ measurements	126

4.4.4	MCH BLER measurements	127

4.5	Proximity-based Services	127

4.5.1	Introduction	127

4.5.2	Requirements	127

4.5.2.1	Interruptions with ProSe Direct Discovery	127

4.5.2.2	Interruptions with ProSe Direct Communication	127

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	127

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	128

4.6	Cell Selection and Re-selection Requirements for UE category NB1	128

4.6.1	Cell Selection	128

4.6.2	Cell Re-selection	128

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	129

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	129

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	130

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	132

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	132

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	133

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	135

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.7	Maximum interruption in paging reception in normal coverage	137

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	138

4.6.2.8	UE measurement capability	138

4.6.2.9	WUS receptions for NB1	138

4.7	Cell Selection and Re-selection Requirements for UE category M1	138

4.7.1	Cell Selection	138

4.7.2	Cell Re-selection	138

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	139

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	139

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	139

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	140

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	142

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	142

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	142

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	142

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	143

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	145

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	146

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	147

4.7.2.3	WUS receptions for UE category M1	147

4.8	Idle State Positioning Measurement Requirements for UE category NB1	148

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	148

4.8.1.1	RSTD Measurement Reporting Delay	149

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	149

4.8.2.1	RSTD Measurement Reporting Delay	151

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	151

4.8.3.1	RSTD Measurement Reporting Delay	152

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	153

4.8.4.1	RSTD Measurement Reporting Delay	154

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	155

4.8.5.1	Measurement Reporting Delay	155

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	156

4.8.6.1	Measurement Reporting Delay	157

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	157

4.8.7.1	Measurement Reporting Delay	158

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	159

4.8.8.1	Measurement Reporting Delay	160

4.9	Idle Mode CA Measurement	161

4.9.1	Introduction	161

4.9.2	Requirement	161

4.9.2.1	Detected cell requirement during state transition and Idle mode	161

4.9.2.2	Measurements of inter-frequency CA candidate cells	161

4A	E-UTRAN RRC_INACTIVE state mobility	162

4A.1	Cell Re-selection	162

4A.1.1	Introduction	162

4A.1.2	Requirements	162

4A.1.2.1	UE measurement capability	162

4A.1.2.2	Measurement and evaluation of serving cell	162

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	162

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	162

4A.1.2.5	Evaluation of cell re-selection criteria	162

4A.1.2.6	Maximum interruption in paging reception	162

4A.1.2.7	Measurements of inter-RAT NR cells	162

5	E-UTRAN RRC_CONNECTED state mobility	162

5.1	E-UTRAN Handover	163

5.1.1	Introduction	163

5.1.2	Requirements	163

5.1.2.1	E-UTRAN FDD – FDD	163

5.1.2.1.1	Handover delay	163

5.1.2.1.2	Interruption time	163

5.2.2.2	E-UTRAN FDD – TDD	165

5.2.2.2.1	(Void)	165

5.2.2.2.2	(Void)	165

5.2.2.3	E-UTRAN TDD – FDD	165

5.2.2.3.1	(Void)	165

5.2.2.3.2	(Void)	165

5.2.2.4	E-UTRAN TDD – TDD	165

5.2.2.4.1	Handover delay	165

5.2.2.4.2	Interruption time	166

5.2.2.5	E-UTRAN HD–FDD	167

5.2.2.5.1	Handover delay	167

5.2.2.5.2	Interruption time	167

5.3	Handover to other RATs	169

5.3.1	E-UTRAN - UTRAN FDD Handover	169

5.3.1.1	Introduction	169

5.3.1.1.1	Handover delay	169

5.3.1.1.2	Interruption time	169

5.3.2	E-UTRAN - UTRAN TDD Handover	170

5.3.2.1	Introduction	170

5.3.2.2	Requirements	170

5.3.2.2.1	Handover delay	170

5.3.2.2.2	Interruption time	170

5.3.3	E-UTRAN - GSM Handover	171

5.3.3.1	Introduction	171

5.3.3.2	Requirements	171

5.3.3.2.1	Handover delay	171

5.3.3.2.2	Interruption time	171

5.3.4	E-UTRAN - NR FR1 Handover	171

5.3.4.1	Introduction	171

5.3.4.2	Handover delay	172

5.3.4.3	Interruption time	172

5.3.5	E-UTRAN - NR FR2 Handover	172

5.3.5.1	Introduction	172

5.3.5.2	Handover delay	172

5.3.5.3	Interruption time	173

5.4	Handover to Non-3GPP RATs	173

5.4.1	E-UTRAN – HRPD Handover	173

5.4.1.1	Introduction	173

5.4.1.1.1	Handover delay	173

5.4.1.1.2	Interruption time	173

5.4.2	E-UTRAN – cdma2000 1X Handover	174

5.4.2.1	Introduction	174

5.4.2.1.1	Handover delay	174

5.4.2.1.2	Interruption time	174

5.5	E-UTRAN Handover for Cat-M1 UEs	175

5.5.1	Introduction	175

5.5.2	Requirements in CEModeA	175

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	175

5.5.2.1.1	Handover delay	175

5.5.2.1.2	Interruption time	175

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	176

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	176

5.5.2.3.1	Void	176

5.5.2.3.2	Void	176

5.5.3	Requirements in CEModeB	176

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	176

5.5.3.1.1	Handover delay	176

5.5.3.1.2	Interruption time	176

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	177

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	177

5.6	Void	177

6	RRC Connection Mobility Control	177

6.1	RRC Re-establishment	177

6.1.1	Introduction	177

6.1.2	Requirements	177

6.1.2.1	UE Re-establishment delay requirement	177

6.2	Random Access	178

6.2.1	Introduction	178

6.2.2	Requirements	178

6.2.2.1	Contention based random access	178

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	178

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	178

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	178

6.2.2.1.4	Void	178

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	178

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	179

6.2.2.2	Non-Contention based random access	179

6.2.2.2.1	Correct behaviour when receiving Random Access Response	179

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	179

6.2.3	Requirements for Cat-M1 UEs	179

6.3	RRC Connection Release with Redirection	179

6.3.1	Introduction	179

6.3.2	Requirements	179

6.3.2.1	RRC connection release with redirection to UTRAN FDD	179

6.3.2.2	RRC connection release with redirection to GERAN	180

6.3.2.3	RRC connection release with redirection to UTRAN TDD	180

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	181

6.4.1	Introduction	181

6.4.2	Requirements	181

6.5	RRC Re-establishment for NB-IoT UEs	181

6.5.1	Introduction	181

6.5.2	Requirements	181

6.5.2.1	UE Re-establishment delay requirement in normal coverage	182

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	182

6.6	Random Access for UE category NB1	182

6.6.1	Introduction	182

6.6.2	Requirements	183

6.6.2.1	Correct behaviour when receiving Random Access Response reception	183

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	183

6.6.2.3	Correct behaviour when receiving a NACK on msg3	183

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	183

6.6.2.5	Correct behaviour when contention Resolution timer expires	183

6.6.2.6	MSG3-based channel quality report for UE Category NB1	183

6.6.3	Requirements for NPRACH configuration	184

6.7	RRC Re-establishment for Cat-M1 UEs	184

6.7.1	Introduction	184

6.7.2	Requirements	184

6.7.2.1	UE Re-establishment delay requirement for CEModeA	185

6.7.2.2	UE Re-establishment delay requirement for CEModeB	185

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	186

6.8.1	Introduction	186

6.8.2	Requirements	186

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	186

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	186

6.9.1	Introduction	186

6.9.2	Requirements	187

7	Timing and signalling characteristics	187

7.1	UE transmit timing	187

7.1.1	Introduction	187

7.1.2	Requirements	188

7.2	UE timer accuracy	189

7.2.1	Introduction	189

7.2.2	Requirements	189

7.3	Timing Advance	189

7.3.1	Introduction	189

7.3.2	Requirements	189

7.3.2.1	Timing Advance adjustment delay	189

7.3.2.2	Timing Advance adjustment accuracy	190

7.4	Cell phase synchronization accuracy (TDD)	190

7.4.1	Definition	190

7.4.2	Minimum requirements	190

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	191

7.5.1	Introduction	191

7.5.2	eNodeB Synchronization Requirements	191

7.5.2.1	Synchronized E-UTRAN	191

7.5.2.2	Non-Synchronized E-UTRAN	191

7.6	Radio Link Monitoring	191

7.6.1	Introduction	191

7.6.2	Requirements	193

7.6.2.1	Minimum requirement when no DRX is used	193

7.6.2.2	Minimum requirement when DRX is used	194

7.6.2.3	Minimum requirement at transitions	195

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	195

7.6.2.5	Minimum requirement under IDC Interference	196

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	196

7.7.1	Introduction	196

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	196

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	197

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	198

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	199

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	200

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	201

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	201

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	202

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	202

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	203

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	204

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	205

7.7.14	SCell Activation Delay Requirement for Dormant SCell	205

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	207

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	207

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	209

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	209

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	211

7.8	Interruptions with Carrier Aggregation	213

7.8.1	Introduction	213

7.8.2	Requirements	213

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	213

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	213

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	213

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	213

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	213

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	214

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	214

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	214

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	215

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	216

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	216

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	216

7.8.2.13	Interruptions at SRS carrier based switching	217

7.8.2.14	Interruptions at SCell activation of dormant SCell for intra-band CA	217

7.8.2.15	Interruptions at SCell activation of dormant SCell for inter-band CA	217

7.8.2.16	Interruptions at SCell activation of multiple dormant SCells	218

7.8.2.17	Interruptions during CQI measurement on dormant SCell	218

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	218

7.9.1	Introduction	218

7.9.2	Minimum Requirements for Interband Carrier Aggregation	218

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	219

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	219

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	219

7.10.1	Introduction	219

7.10.2	Requirements	219

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	219

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	220

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	220

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	221

7.11	Radio Link Monitoring for UE Category 0	221

7.11.1	Introduction	221

7.11.2	Requirements for FD-FDD and TDD	222

7.11.2.1	Minimum requirement when no DRX is used	222

7.11.2.2	Minimum requirement when DRX is used	223

7.11.2.3	Minimum requirement at transitions	223

7.11.3	Requirements for HD-FDD	224

7.11.3.1	Minimum requirement when no DRX is used	224

7.11.3.2	Minimum requirement when DRX is used	224

7.11.3.3	Minimum requirement at transitions	225

7.12	Interruptions with Dual Connectivity	225

7.12.1	Introduction	225

7.12.2	Requirements	225

7.12.2.1	Interruptions at PSCell addition/release	225

7.12.2.2	Interruptions at transitions between active and non-active during DRX	225

7.12.2.3	Interruptions at transitions from non-DRX to DRX	226

7.12.2.4	Interruptions at SCell addition/release	226

7.12.2.5	Interruptions at SCell activation/deactivation	226

7.12.2.6	Interruptions during measurements on SCC	227

7.12.2.7	Interruptions at SRS carrier based switching	227

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	228

7.13.1	Definition	228

7.13.2	Minimum requirements	228

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	228

7.14.1	Introduction	228

7.14.2	PSCell Addition Delay Requirement	228

7.14.3	PSCell Release Delay Requirement	229

7.15	Maximum Receive Timing Difference in Dual Connectivity	229

7.15.1	Introduction	229

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	229

7.16	Proximity-based Services	229

7.16.1	Introduction	229

7.16.2	Requirements	229

7.16.2.1	ProSe UE transmission timing	229

7.16.2.1.1	Serving cell or PCell as timing reference	230

7.16.2.1.2	SCell or non-serving cell as timing reference	230

7.16.3	Interruptions with ProSe	230

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	230

7.16.3.2	Interruptions at ProSe Direct Communication configuration	231

7.16.3.3	Interruptions during ProSe Direct Discovery	231

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	231

7.16.3.5	Interruptions during ProSe Direct Communication	232

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	232

7.16.4.1	Measurement and evaluation of selected cell	232

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	232

7.16.5	Selection / Reselection of ProSe relay UE	233

7.16.6	ProSe operation under deactivated SCell	233

7.17	Maximum Transmission Timing Difference in Dual Connectivity	233

7.17.1	Introduction	233

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	234

7.18.1	Introduction	234

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	234

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	234

7.19	Radio Link Monitoring for UE Category M1	234

7.19.1	Introduction	234

7.19.2	Requirements for FD-FDD and TDD CE mode A	235

7.19.2.1	Minimum requirement when no DRX is used	236

7.19.2.2	Minimum requirement when DRX is used	236

7.19.2.3	Minimum requirement at transitions	238

7.19.3	Requirements for HD-FDD with CE mode A	238

7.19.3.1	Minimum requirement when no DRX is used	238

7.19.3.2	Minimum requirement when DRX is used	238

7.19.3.3	Minimum requirement at transitions	239

7.19.4	Requirements for FD-FDD and TDD with CE mode B	239

7.19.4.1	Minimum requirement when no DRX is used	240

7.19.4.2	Minimum requirement when DRX is used	241

7.19.4.3	Minimum requirement at transitions	242

7.19.5	Requirements for HD-FDD with CE mode B	243

7.19.5.1	Minimum requirement when no DRX is used	243

7.19.5.2	Minimum requirement when DRX is used	243

7.19.5.3	Minimum requirement at transitions	244

7.20	UE transmit timing for NB-IoT	244

7.20.1	Introduction	244

7.20.2	Requirements	244

7.21	 UE timer accuracy for NB-IoT	245

7.21.1	Introduction	245

7.21.2	Requirements	245

7.22	Timing Advance for NB-IoT	245

7.22.1	Introduction	245

7.22.2	Requirements	245

7.22.2.1	Timing Advance adjustment delay	245

7.22.2.2	Timing Advance adjustment accuracy	246

7.23	Radio Link Monitoring for Category NB1 UE	246

7.23.1	Introduction	246

7.23.2	Requirements for Category NB1 UE	246

7.23.2.1	Minimum requirement when no DRX is used	246

7.23.2.2	Minimum requirement when DRX is used	247

7.23.2.3	Minimum requirement at transitions	247

7.24	UE transmit timing for Category M1	248

7.24.1	Introduction	248

7.24.2	Requirements	248

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	249

7.25.1	Definition	249

7.25.2	Minimum requirements	249

7.26	UE transmit timing for Category M2	249

7.26.1	Introduction	249

7.26.2	Requirements	249

7.27	UE timer accuracy for category M1	250

7.27.1	Introduction	250

7.27.2	Requirements	250

7.28	Timing Advance for Category M1	250

7.28.1	Introduction	250

7.28.2	Requirements	250

7.29	Interruptions requirements with FeMBMS	250

7.29.1	Introduction	250

7.29.2	Requirements	250

7.30	Numerology switching delay requirements with FeMBMS	250

7.30.1	Introduction	250

7.30.2	Requirements	250

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	251

7.31.1	Introduction	251

7.31.2	NR PSCell Addition Delay Requirement	251

7.31.3	NR PSCell Release Delay Requirement	252

7.32	Interruptions with EN-DC	252

7.32.1	Introduction	252

7.32.2	Requirements	252

7.32.2.1	Interruptions at PSCell addition/release	252

7.32.2.2	Interruptions at transitions between active and non-active during DRX	253

7.32.2.3	Interruptions at transitions from non-DRX to DRX	253

7.32.2.4	Interruptions at SCell addition/release	253

7.32.2.5	Interruptions at SCell activation/deactivation	253

7.32.2.6	Interruptions during measurements on SCC	253

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	253

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	254

7.32.2.7	Interruptions at active BWP switching	254

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	254

7.33.1	Introduction	254

7.33.2	Requirements	255

7.34	Void	255

7.35	Interruptions with SFTD measurements	255

7.35.1	Introduction	255

7.35.2	Requirements	255

8	UE Measurements Procedures in RRC_CONNECTED State	255

8.1	General Measurement Requirements	255

8.1.1	Introduction	255

8.1.2	Requirements	256

8.1.2.1	UE measurement capability	256

8.1.2.1.1	Monitoring of multiple layers using gaps	262

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	264

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	265

8.1.2.1.2	Network controlled small gap	266

8.1.2.2	E-UTRAN intra frequency measurements	268

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	268

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	272

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	276

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	277

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	278

8.1.2.3	E-UTRAN inter frequency measurements	278

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	279

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	284

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	291

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	291

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	291

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	292

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	294

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	295

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	296

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	303

8.1.2.4	Inter RAT measurements	303

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	303

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	308

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	308

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	312

8.1.2.4.5	E-UTRAN FDD – GSM measurements	312

8.1.2.4.6	E-UTRAN TDD – GSM measurements	317

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	317

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	319

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	319

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	319

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	320

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	320

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	320

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	320

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	322

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	322

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	322

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	322

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	323

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	323

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	325

8.1.2.4.21	E-UTRAN FDD – NR measurements	325

8.1.2.4.22	E-UTRAN TDD – NR measurements	328

8.1.2.4.23	Void	328

8.1.2.4.24	Void	328

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	328

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	329

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	329

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	329

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	331

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	333

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	334

8.1.2.6.5	Void	336

8.1.2.6.6	Void	336

8.1.2.6.7	Void	336

8.1.2.6.8	Void	336

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	336

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	336

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	338

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	340

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	341

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	343

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	345

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	347

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	348

8.1.2.7	E-UTRAN E-CID Measurements	350

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	350

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	351

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	353

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	353

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	354

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	354

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	357

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	360

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	363

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	367

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	367

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	367

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	367

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	368

8.1.2.10	Void	368

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	368

8.2.1	Introduction	368

8.2.2	Requirements	369

8.3	Measurements for E-UTRA carrier aggregation	372

8.3.1	Introduction	372

8.3.2	Measurements of the primary component carrier	372

8.3.3	Measurements of a secondary component carrier	372

8.3.3.1	Measurements of a secondary component carrier with active SCell	372

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	373

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	373

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	374

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	375

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	375

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	376

8.4.1	Introduction	376

8.4.2	Measurements on the primary component carrier	376

8.4.3	Measurements on a secondary component carrier	377

8.4.4	Measurements on both primary component carrier and a secondary component carrier	378

8.4.5	Measurements on different secondary component carriers	379

8.5	Measurements for UE category 0	380

8.5.1	Introduction	380

8.5.2 	Requirements	380

8.5.2.1 	E-UTRAN intra frequency measurements	380

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	380

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	383

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	385

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	389

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	390

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	390

8.6	Discovery signal measurements	391

8.6.1	Introduction	391

8.6.2	Requirements for CRS based discovery signal measurements	391

8.6.2.1	E-UTRAN intra frequency measurements	391

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	391

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	394

8.6.2.2	E-UTRAN inter frequency measurements	396

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	397

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	399

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	402

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	402

8.6.3	Requirements for CSI-RS based discovery signal measurements	402

8.6.3.1	E-UTRAN intra frequency measurements	402

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	403

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	405

8.6.3.2	E-UTRAN inter frequency measurements	407

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	408

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	410

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	413

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	413

8.7	Discovery signal measurements for E-UTRA carrier aggregation	413

8.7.1	Introduction	413

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	414

8.7.2.1	Measurements of the primary component carrier	414

8.7.2.2	Measurements of a secondary component carrier	414

8.7.2.3	Measurements of a secondary component carrier with active SCell	414

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	414

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	414

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	415

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	417

8.7.3.1	Measurements of the primary component carrier	417

8.7.3.2	Measurements of a secondary component carrier	417

8.7.3.3	Measurements of a secondary component carrier with active SCell	417

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	417

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	417

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	419

8.8	Measurements for E-UTRA dual connectivity	420

8.8.1	Introduction	420

8.8.2	Intra-frequency measurements requirements on PCell	420

8.8.3	Intra-frequency measurements requirements on PSCell	421

8.8.4	Inter-frequency and inter-RAT measurement requirements	421

8.8.5	Intra-frequency measurements with autonomous gaps	421

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	421

8.8.5.2	ECGI reporting delay	422

8.8.6	Inter-frequency measurements with autonomous gaps	422

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	422

8.8.6.2	ECGI reporting delay	423

8.8.7	SSTD Measurements	423

8.8.7.1	Introduction	423

8.8.7.2	SSTD Measurement requirements	423

8.8.7.3	SSTD Measurement Reporting Delay	424

8.8.8	Intra-frequency measurements requirements on SCell	424

8.9	MBSFN Measurements	424

8.9.1	Introduction	424

8.9.2	MBSFN RSRP Measurements	424

8.9.3	MBSFN RSRQ Measurements	425

8.9.4	MCH BLER Measurements	425

8.10	Proximity-based Services	425

8.10.1	Introduction	425

8.10.2	Requirements	425

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	425

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	426

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	426

8.11.1	Introduction	426

8.11.2	CRS based discovery signal measurements	427

8.11.2.1	E-UTRAN intra-frequency measurements	427

8.11.2.1.1	Requirements	427

8.11.2.1.1.1	Requirements when no DRX is used	427

8.11.2.1.1.1.1	Measurement Reporting Requirements	428

8.11.2.1.1.2	Requirements when DRX is used	429

8.11.2.1.1.2.1	Measurement Reporting Requirements	431

8.11.2.2	E-UTRAN inter-frequency measurements	432

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	432

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	436

8.11.3	CSI-RS based discovery signal measurements	436

8.11.3.1	E-UTRAN intra-frequency measurements	436

8.11.3.1.1	Requirements	436

8.11.3.1.1.1	Requirements when no DRX is used	436

8.11.3.1.1.1.1	Measurement Reporting Requirements	438

8.11.3.1.1.2	Requirements when DRX is used	438

8.11.3.1.1.2.1	Measurement Reporting Requirements	439

8.11.3.2	E-UTRAN inter-frequency measurements	440

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	440

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	444

8.11.4	RSSI measurements	444

8.11.4.1	E-UTRAN intra-frequency measurements	444

8.11.4.2	E-UTRAN inter-frequency measurements	444

8.11.5	Channel occupancy measurements	444

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	444

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	445

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	445

8.12.1	Introduction	445

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	445

8.12.2.1	Introduction	445

8.12.2.2	Measurements of a secondary component carrier	445

8.12.2.3	Measurements of a secondary component carrier with active SCell	445

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	445

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	446

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	448

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	451

8.12.3.1	Introduction	451

8.12.3.2	Measurements of a secondary component carrier	451

8.12.3.3	Measurements of a secondary component carrier with active SCell	451

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	451

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	451

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	453

8.13	Measurements for UE Category M1	455

8.13.1	Introduction	455

8.13.2	Requirements for UE category M1 with CE mode A	455

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	456

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	456

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	460

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	462

8.13.2.2	Void	466

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	466

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	466

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	469

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	471

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	472

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	472

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	474

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	476

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	477

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	477

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	477

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	478

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	478

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	478

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	479

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	479

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	480

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	481

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	481

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	482

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	486

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	488

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	492

8.13.3	Requirements for UE category M1 with CE mode B	492

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	492

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	492

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	497

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	499

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	505

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	506

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	506

8.13.3.2	Void	507

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	507

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	507

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	510

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	512

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	513

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	513

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	513

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	513

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	514

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	514

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	515

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	515

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	515

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	519

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	522

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	526

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	527

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	527

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	529

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	531

8.14	Measurements for UE category NB1	532

8.14.1	Introduction	532

8.14.2	NB-IoT intra frequency measurements under normal coverage	532

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	532

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	532

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	532

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	532

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	532

8.15	Void	533

8.16	Measurements for UE Category M2	533

8.16.1	Introduction	533

8.16.2	Requirements for UE category M2 with CE mode A	533

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	533

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	534

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	534

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	535

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	535

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	535

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	536

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	536

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	538

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	541

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	541

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	541

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	543

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	546

8.16.3	Requirements for UE category M2 with CE mode B	546

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	546

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	546

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	549

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	551

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	552

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	552

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	554

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	556

8.17	Measurements for E-UTRA – NR Dual Connectivity	557

8.17.1	Introduction	557

8.17.1.1	Measurement Gap Sharing	557

8.17.1A	Intrafrequency Measurements	558

8.17.2	SFTD Measurements	558

8.17.2.1	Introduction	558

8.17.2.2	SFTD Measurement requirements	558

8.17.2.3	SFTD Measurement Reporting Delay	558

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	559

8.17.3.1	Introduction	559

8.17.3.2	E-UTRAN FDD inter frequency measurements	559

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	559

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	561

8.17.3.3	E-UTRAN TDD inter frequency measurements	562

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	562

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	564

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	566

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	566

8.17.4.1.1	NR Inter-RAT cell identification	566

8.17.4.1.2	NR Inter-RAT measurement	567

8.17.4.1.3	NR Inter-RAT measurement reporting	567

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	567

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	567

8.17.5.1	Introduction	567

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	568

8.17.5.2.1	Identification of a new UTRA FDD cell	568

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	568

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	568

8.17.5.2.4	Periodic Reporting	569

8.17.5.2.5	Event Triggered Reporting	569

8.17.5.2.6	Event-triggered Periodic Reporting	569

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	569

8.17.5.3.1	Periodic Reporting	571

8.17.5.3.2	Event Triggered Reporting	571

8.17.5.3.3	Event-triggered Periodic Reporting	571

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	571

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	571

8.17.7.1	Introduction	571

8.17.7.2	Identification of a new UTRA FDD cell for SON	572

8.17.7.2.1	Requirements when no DRX is used	572

8.17.7.2.2	Requirements when DRX is used	572

8.17.7.2.3	Reporting Delay	573

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	573

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	573

8.17.9.1	Introduction	573

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	574

8.17.9.2.1	Identification of a new UTRA FDD cell	574

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	574

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	574

8.17.9.2.4	Periodic Reporting	575

8.17.9.2.5	Event Triggered Reporting	575

8.17.9.2.6	Event-triggered Periodic Reporting	575

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	575

8.17.9.3.1	Periodic Reporting	576

8.17.9.3.2	Event Triggered Reporting	576

8.17.9.3.3	Event-triggered Periodic Reporting	577

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	577

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	577

8.17.11.1	Introduction	577

8.17.11.2	Identification of a new UTRA TDD cell for SON	577

8.17.11.2.1	Requirements when no DRX is used	577

8.17.11.2.2	Requirements when DRX is used	578

8.17.11.2.3	Reporting Delay	578

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	579

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	579

8.17.13.1	Introduction	579

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	579

8.17.13.2.1	GSM carrier RSSI	579

8.17.13.2.2	BSIC verification	579

8.17.13.2.3	Enhanced BSIC verification	581

8.17.13.2.4	Periodic Reporting	581

8.17.13.2.5	Event Triggered Reporting	581

8.17.13.2.6	Event-triggered Periodic Reporting	582

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	582

8.17.13.3.1	GSM carrier RSSI	582

8.17.13.3.2	BSIC verification	583

8.17.13.3.3	Periodic Reporting	584

8.17.13.3.4	Event Triggered Reporting	584

8.17.13.3.5	Event-triggered Periodic Reporting	584

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	584

9	Measurements performance requirements for UE	584

9.1	E-UTRAN measurements	585

9.1.1	Introduction	585

9.1.2	Intra-frequency RSRP Accuracy Requirements	585

9.1.2.1	Absolute RSRP Accuracy	585

9.1.2.2	Relative Accuracy of RSRP	586

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	586

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	587

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	588

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	589

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	590

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	591

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	592

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	592

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	592

9.1.3	Inter-frequency RSRP Accuracy Requirements	593

9.1.3.1	Absolute RSRP Accuracy	593

9.1.3.2	Relative Accuracy of RSRP	594

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	594

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	595

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	596

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	596

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	596

9.1.4	RSRP Measurement Report Mapping	597

9.1.5	Intra-frequency RSRQ Accuracy Requirements	597

9.1.5.1	Absolute RSRQ Accuracy	597

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	598

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	599

9.1.5.4	Absolute WB-RSRQ Accuracy	600

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	601

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	602

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	602

9.1.6	Inter-frequency RSRQ Accuracy Requirements	602

9.1.6.1	Absolute RSRQ Accuracy	602

9.1.6.2	Relative Accuracy of RSRQ	603

9.1.6.3	Absolute WB-RSRQ Accuracy	604

9.1.6.4	Relative WB-RSRQ Accuracy	604

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	605

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	606

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	606

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	606

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	607

9.1.7	RSRQ Measurement Report Mapping	608

9.1.8	Power Headroom	608

9.1.8.1	Period	608

9.1.8.2	Reporting Delay	608

9.1.8.3	Void	609

9.1.8.4	Report Mapping	609

9.1.9	UE Rx – Tx time difference	609

9.1.9.1	Measurement Requirement	609

9.1.9.2	Measurement Report mapping	610

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	610

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	611

9.1.10	Reference Signal Time Difference (RSTD)	612

9.1.10.1	Intra-Frequency Accuracy Requirement	612

9.1.10.2	Inter-Frequency Accuracy Requirement	614

9.1.10.3	RSTD Measurement Report Mapping	615

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	616

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	616

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	617

9.1.11	Carrier aggregation measurement accuracy	618

9.1.11.1	Primary component carrier accuracy requirement	619

9.1.11.2	Secondary component carrier accuracy requirement	619

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	619

9.1.11.4	Secondary component carrier relative accuracy requirement	619

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	619

9.1.13	Measurement accuracy for UE category 0	620

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	620

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	620

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	621

9.1.14	Accuracy requirements for Discovery Signal Measurements	622

9.1.14.1	Introduction	622

9.1.14.2	RSRP measurements in discovery signal occasions	622

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	622

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	622

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	622

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	623

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	624

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	624

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	625

9.1.14.3.3	CSI-RSRP measurement report mapping	626

9.1.14.4	RSRQ measurements in discovery signal occasions	626

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	626

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	626

9.1.15.1.1	Primary component carrier accuracy requirement	626

9.1.15.1.2	Secondary component carrier accuracy requirement	626

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	627

9.1.15.1.4	Secondary component carrier relative accuracy requirement	627

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	627

9.1.15.2.1	Primary component carrier accuracy requirement	627

9.1.15.2.2	Secondary component carrier accuracy requirement	627

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	627

9.1.15.2.4	Secondary component carrier relative accuracy requirement	627

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	627

9.1.17	RS-SINR Measurements	628

9.1.17.1	Measurement Report Mapping	628

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	628

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	628

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	629

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	629

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	630

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	630

9.1.18.1	Introduction	630

9.1.18.2	RSRP measurements	630

9.1.18.2.1	RSRP measurement report mapping	630

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	630

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	631

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	631

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	632

9.1.18.3	RSRQ measurements	632

9.1.18.3.1	RSRQ measurement report mapping	632

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	633

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	633

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	634

9.1.18.4	CSI-RSRP measurements	634

9.1.18.4.1	CSI-RSRP measurement report mapping	634

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	634

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	635

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	635

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	636

9.1.18.5	RSSI measurements	636

9.1.18.5.1	RSSI measurement report mapping	636

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	637

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	637

9.1.18.6	Channel occupancy measurements	637

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	637

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	638

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	638

9.1.19.1	Introduction	638

9.1.19.2	Accuracy requirements for measurements on SCC	638

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	638

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	639

9.1.20	SFN and Subframe Time Difference (SSTD)	639

9.1.20.1	SSTD Accuracy Requirement	639

9.1.20.2	SSTD Measurement Report Mapping	640

9.1.21	Measurement accuracy for UE category M1	641

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	641

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	642

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	643

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	644

9.1.21.5	RSRP Measurement Report Mapping	645

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	645

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	646

9.1.21.8	RSRQ Measurement Report Mapping	647

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	647

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	648

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	649

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	650

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	652

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	652

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	653

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	654

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	656

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	656

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	657

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	658

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	660

9.1.22	Measurement accuracy for UE Category NB1	662

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	662

9.1.22.2	Void	663

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	663

9.1.22.4	Void	664

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	664

9.1.22.6	Void	665

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	665

9.1.22.8	Void	666

9.1.22.9	NRSRP Measurement Report Mapping	666

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	666

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	667

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	668

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	669

9.1.22.14	NRSRQ Measurement Report Mapping	670

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	671

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	671

9.1.23	Power Headroom for UE Category NB1	672

9.1.23.1	Period	672

9.1.23.2	Reporting Delay	672

9.1.23.3	Report Mapping for UE Category NB1	672

9.1.23.3.1	Void	674

9.1.23.3.2	Void	674

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	674

9.1.24	Void	675

9.1.25	Measurement accuracy for UE category M2	675

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	675

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	676

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	677

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	678

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	680

9.1.26	Measurement Accuracy for non-BL CE UE	681

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	682

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	682

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	683

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	684

9.1.26.5	RSRP Measurement Report Mapping	685

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	685

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	685

9.1.26.8	RSRQ Measurement Report Mapping	686

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	686

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	687

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	688

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	689

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	690

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	690

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	690

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	691

9.1.27	SFN and frame Timing Difference (SFTD)	692

9.1.27.1	SFTD Accuracy Requirement	692

9.2	UTRAN FDD Measurements	694

9.2.1	UTRAN FDD CPICH RSCP	694

9.2.2	Void	695

9.2.3	UTRAN FDD CPICH Ec/No	695

9.3	UTRAN TDD Measurements	695

9.3.1	UTRAN TDD P-CCPCH RSCP	696

9.3.2	Void	696

9.3.3	Void	696

9.4	GSM Measurements	696

9.4.1	GSM carrier RSSI	696

9.5	CDMA2000 1x RTT Measurements	696

9.5.1	CDMA2000 1x RTT Pilot Strength	696

9.6	PCMAX,c	697

9.6.1	Report Mapping	697

9.6.2 	Estimation Period	697

9.6.3	Reporting Delay	697

9.7	IEEE802.11 Measurements	697

9.7.1	WLAN RSSI	697

9.7.2	WLAN RSSI Measurement Report Mapping	697

9.8	MBSFN Measurements	698

9.8.1	Introduction	698

9.8.2	MBSFN RSRP	698

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	698

9.8.2.2	MBSFN RSRP measurement report mapping	699

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	699

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	700

9.8.3	MBSFN RSRQ	700

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	700

9.8.3.2	MBSFN RSRQ measurement report mapping	701

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	701

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	702

9.8.4	MCH BLER	702

9.8.4.1	Measurement report mapping for MCH BLER	702

9.8.4.2	Measurement report mapping for MCH Block Number	703

9.9	ProSe Measurements	704

9.9.1	Introduction	704

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	704

9.9.2.1	Absolute S-RSRP Accuracy	704

9.9.2.2	Relative Accuracy of S-RSRP	705

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	706

9.9.3.1	Absolute SD-RSRP Accuracy	706

9.9.3.2	Relative Accuracy of SD-RSRP	706

9.10	V2X Measurements	707

9.10.1	Introduction	707

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	707

9.10.2.1	Absolute S-RSRP Accuracy	707

9.10.2.2	Relative Accuracy of S-RSRP	708

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	708

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	708

9.10.4	S-RSSI Measurement Accuracy Requirements	709

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	709

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	709

9.11	NR Measurements	710

9.11.1	NR SS-RSRP Measurements	710

9.11.2	NR SS-RSRQ Measurements	710

9.11.3	NR SS-SINR Measurements	711

10	Measurements Performance Requirements for E-UTRAN	711

10.1	Received Interference Power	711

10.1.1	Absolute accuracy requirement	711

10.1.2	Relative accuracy requirement	712

10.1.3	Received Interference Power measurement report mapping	712

10.2	Angle of Arrival (AOA)	712

10.2.1	Range/mapping	712

10.3	Timing Advance (TADV)	712

10.3.1	Report mapping	712

11	ProSe Requirements in Any Cell Selection state	713

11.1	Introduction	713

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	713

11.2.1	Introduction	713

11.2.2	ProSe UE transmission timing	713

11.3	Initiation/Cease of SLSS Transmissions	714

11.3.1	Introduction	714

11.3.2	Requirements	714

11.4	Measurements for ProSe in Any Cell Selection State	714

11.4.1	Introduction	714

11.4.2	Requirements	714

11.4.2.1	E-UTRA FDD	714

11.4.2.2	E-UTRA TDD	715

11.5	Selection / Reselection of ProSe Synchronization Reference	715

11.5.1	Introduction	715

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	715

11.5.2.1	Introduction	715

11.5.2.2	Requirements	715

11.6	Void	716

11.7	Selection / Reselection of ProSe relay UE	716

11.7.1	Introduction	716

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	716

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	716

12.1	Introduction	716

12.2	Transmit Timing	717

12.2.1	GNSS as timing reference	717

12.3	Interruption	717

12.4	Reliability of GNSS signal	717

13	V2X Requirements	717

13.1	Introduction	717

13.2	UE Transmit Timing	718

13.2.1	Introduction	718

13.2.2	GNSS as synchronization reference source	718

13.2.3	Serving cell/PCell as synchronization reference source	718

13.2.4	SyncRef UE as synchronization reference source	718

13.3	Initiation/Cease of SLSS Transmissions	718

13.3.1	Introduction	718

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	718

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	719

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	719

13.4	Selection / Reselection of V2X Synchronization Reference Source	720

13.5	Autonomous Resource Selection/Reselection measurements	721

13.5.1	Introduction	721

13.5.2	PSSCH-RSRP measurements	721

13.5.3	S-RSSI measurements	721

13.6	Congestion Control measurements	721

13.7	Interruption	721

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	721

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	721

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	722

13.8	Reliability of GNSS signal	722

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	722

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	723

Annex A (normative):	Test Cases	724

A.1	Purpose of annex	724

A.2	Requirement classification for statistical testing	724

A.2.1	Types of requirements in TS 36.133	724

A.2.1.1	Time and delay requirements on UE higher layer actions	724

A.2.1.2	Measurements of power levels, relative powers and time	724

A.2.1.3	Implementation requirements	725

A.2.1.4	Physical layer timing requirements	725

A.3	RRM test configurations	726

A.3.1	Reference Measurement Channels	726

A.3.1.1	PDSCH	726

A.3.1.1.1	FDD	726

A.3.1.1.2	TDD	730

A.3.1.1.3	FDD for UE category 0	733

A.3.1.1.4	HD-FDD for UE category 0	734

A.3.1.1.5	TDD for UE category 0	735

A.3.1.1.6	Frame Structure 3	736

A.3.1.2	PCFICH/PDCCH/PHICH	737

A.3.1.2.1	FDD	737

A.3.1.2.2	TDD	737

A.3.1.2.3	HD-FDD for UE category 0	738

A.3.1.2.4	FS 3	738

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	738

A.3.1.3.1	FDD in CEModeA	739

A.3.1.3.2	HD-FDD in CEModeA	739

A.3.1.3.3	TDD in CEModeA	740

A.3.1.3.4	FDD in CEModeB	740

A.3.1.3.5	HD-FDD in CEModeB	741

A.3.1.3.6	TDD in CEModeB	741

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	742

A.3.1.4.1	FDD in CEModeA	742

A.3.1.4.2	HD-FDD in CEModeA	743

A.3.1.4.3	TDD in CEModeA	744

A.3.1.4.4	FDD in CEModeB	745

A.3.1.4.5	 HD-FDD in CEModeB	746

A.3.1.4.6	TDD in CEModeB	746

A.3.1.5	NPDSCH Reference Channel for UE category NB1	747

A.3.1.5.1	HD-FDD in-band operation	747

A.3.1.5.2	Void	748

A.3.1.5.3	HD-FDD standalone operation	748

A.3.1.5.4	Void	749

A.3.1.5.5	HD-FDD guard band operation	749

A.3.1.5.6	Void	750

A.3.1.5.7	TDD in-band operation	750

A.3.1.5.8	TDD standalone operation	750

A.3.1.5.9	TDD guard band operation	751

A.3.1.6	NPDCCH Reference Channel for UE category NB1	751

A.3.1.6.1	In-band operation	751

A.3.1.6.2	Void	752

A.3.1.6.3	Standalone operation	752

A.3.1.6.4	Void	752

A.3.1.6.5	Guard band operation	752

A.3.1.6.6	Void	753

A.3.2	OFDMA Channel Noise Generator (OCNG)	753

A.3.2.1	OCNG Patterns for FDD	753

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	754

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	754

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	755

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	756

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	756

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	757

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	757

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	757

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	758

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	759

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	759

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	760

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	760

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	761

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	761

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	762

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	762

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	763

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	763

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	764

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	764

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	765

A.3.2.2	OCNG Patterns for TDD	765

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	766

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	766

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	767

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	767

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	768

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	769

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	770

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	771

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	771

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	772

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	773

A.3.2.3	OCNG Patterns for Narrowband IoT	773

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	775

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	776

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	776

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	777

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	778

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	779

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	781

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	781

A.3.2.4	OCNG Patterns for V2X sidelink	782

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	782

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	783

A.3.3	Reference DRX Configurations	783

A.3.4	ABS Transmission Configurations	784

A.3.4.1	Non-MBSFN ABS Transmission Configurations	784

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	784

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	784

A.3.4.2	MBSFN ABS Transmission Configurations	785

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	785

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	786

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	786

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	786

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	787

A.3.6.1	Introduction	787

A.3.7	Test Cases with Different Channel Bandwidths	787

A.3.7.1	Introduction	787

A.3.7.2	Principle of testing	787

A.3.8	Antenna Configuration	787

A.3.8.1	Antenna connection for 4 Rx capable UEs	787

A.3.8.1.1 Introduction	787

A.3.8.1.2 Principle of testing	787

A.3.8.1.2.1 Single carrier tests	787

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	788

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	789

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	789

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	789

A.3.9.1	Introduction	789

A.3.9.2	Principle of testing	789

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	790

A.3.10.1	Introduction	790

A.3.10.2	Principle of testing	790

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	790

A.3.11.1	Introduction	790

A.3.11.2	Principle of Testing	790

A.3.12	Proximity-based Services	790

A.3.12.1	Introduction	790

A.3.12.2	Reference DRX configurations for ProSe tests	790

A.3.12.3	Test Cases with Different Channel Bandwidths	791

A.3.12.3.1	Introduction	791

A.3.12.3.2	Principle of testing	791

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	791

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	797

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	799

A.3.12.6.1	FDD	799

A.3.12.7	Reference measurement channels for ProSe Direct Communication	799

A.3.12.7.1	FDD	799

A.3.12.8	ProSe Receive Traffic Generator	800

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	800

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	800

A.3.13	Time Offset between Cells	801

A.3.13.1	Introduction	801

A.3.13.2	Definition	801

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	801

A.3.14.1	Introduction	801

A.3.14.2	Principle of testing	801

A.3.15	Dual connectivity test cases with different combination of duplex mode	801

A.3.15.1	Introduction	801

A.3.15.2	Principle of testing	801

A.3.16	Reference PRACH Configurations	802

A.3.17	Listen before talk model	802

A.3.17.1	Introduction	802

A.3.17.2	Definition	802

A.3.18	Reference NPRACH Configurations	803

A.3.19	Dual connectivity test cases with different bandwidth combinations	804

A.3.19.1	Introduction	804

A.3.19.2	Principle of testing	804

A.3.20	Category M1 UE Test Cases	805

A.3.20.1	Introduction	805

A.3.20.2	Principle of Cat-M1 UE Testing	805

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	806

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	807

A.3.21.1	Introduction	807

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	807

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	808

A.3.22	Category 1bis UE Test Cases	809

A.3.22.1	Introduction	809

A.3.22.2	Principle of Category 1bis UE Testing	809

A.3.23	Category NB2 UE Test Cases	812

A.3.23.1	Introduction	812

A.3.23.2	Principle of Category NB2 UE Testing	812

A.3.24	V2X sidelink communication	812

A.3.24.1	Introduction	812

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	813

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	816

A.3.25	Category M2 UE Test Cases	817

A.3.25.1	Introduction	817

A.3.25.2	Principle of Cat-M2 UE Testing	817

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	818

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	819

A.3.26.1	Introduction	819

A.3.26.2	Principle of testing	819

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	819

A.3.27.1	Introduction	819

A.3.27.2	Principle of INACTIVE cell re-selection Testing	819

A.4	E-UTRAN RRC_IDLE state	819

A.4.2	Cell Re-Selection	819

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	819

A.4.2.1.1	Test Purpose and Environment	819

A.4.2.1.2	Test Requirements	821

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	822

A.4.2.2.1	Test Purpose and Environment	822

A.4.2.2.2	Test Requirements	823

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	824

A.4.2.3.1	Test Purpose and Environment	824

A.4.2.3.2	Test Requirements	825

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	826

A.4.2.4.1	Test Purpose and Environment	826

A.4.2.4.2	Test Requirements	827

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	828

A.4.2.5.1	Test Purpose and Environment	828

A.4.2.5.2	Test Requirements	829

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	830

A.4.2.6.1	Test Purpose and Environment	830

A.4.2.6.2	Test Requirements	831

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	832

A.4.2.7.1	Test Purpose and Environment	832

A.4.2.7.2	Test Requirements	833

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	834

A.4.2.8.1	Test Purpose and Environment	834

A.4.2.8.2	Test Requirements	835

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	836

A.4.2.9.1	Test Purpose and Environment	836

A.4.2.9.2	Test Requirements	836

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	836

A.4.2.10.1	Test Purpose and Environment	836

A.4.2.10.2	Test Requirements	840

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	840

A.4.2.11.1	Test Purpose and Environment	840

A.4.2.11.2	Test Requirements	844

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	844

A.4.2.12.1	Test Purpose and Environment	844

A.4.2.12.2	Test Requirements	846

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	847

A.4.2.13.1	Test Purpose and Environment	847

A.4.2.13.2	Test Requirements	848

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	849

A.4.2.14.1	Test Purpose and Environment	849

A.4.2.14.2	Test Requirements	850

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	851

A.4.2.15.1	Test Purpose and Environment	851

A.4.2.15.2	Test Requirements	852

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	853

A.4.2.16.1	Test Purpose and Environment	853

A.4.2.16.2	Test Requirements	854

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	855

A.4.2.17.1	Test Purpose and Environment	855

A.4.2.17.2	Test Requirements	856

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	857

A.4.2.18.1	Test Purpose and Environment	857

A.4.2.18.2	Test Requirements	859

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	860

A.4.2.19.1	Test Purpose and Environment	860

A.4.2.19.2	Test Requirements	862

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	863

A.4.2.20.1	Test Purpose and Environment	863

A.4.2.20.2	Test Requirements	864

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	865

A.4.2.21.1	Test Purpose and Environment	865

A.4.2.21.2	Test Requirements	866

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	867

A.4.2.22.1	Test Purpose and Environment	867

A.4.2.22.2	Test Requirements	868

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	869

A.4.2.23.1	Test Purpose and Environment	869

A.4.2.23.2	Test Requirements	870

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	871

A.4.2.24.1	Test Purpose and Environment	871

A.4.2.24.2	Test Requirements	873

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	874

A.4.2.25.1	Test Purpose and Environment	874

A.4.2.25.2	Test Requirements	875

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	876

A.4.2.26.1	Test Purpose and Environment	876

A.4.2.26.2	Test Requirements	877

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	878

A.4.2.27.1	Test Purpose and Environment	878

A.4.2.27.2	Test Requirements	879

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	880

A.4.2.28.1	Test Purpose and Environment	880

A.4.2.28.2	Test Requirements	881

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	882

A.4.2.29.1	Test Purpose and Environment	882

A.4.2.29.2	Test Requirements	883

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	884

A.4.2.30.1	Test Purpose and Environment	884

A.4.2.30.2	Test Requirements	885

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	886

A.4.2.31.1	Test Purpose and Environment	886

A.4.2.31.2	Test Requirements	887

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	888

A.4.2.32.1	Test Purpose and Environment	888

A.4.2.32.2	Test Requirements	889

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	890

A.4.2.33.1	Test Purpose and Environment	890

A.4.2.33.2	Test Requirements	891

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	892

A.4.2.34.1	Test Purpose and Environment	892

A.4.2.34.2	Test Requirements	893

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	894

A.4.2.35.1	Test Purpose and Environment	894

A.4.2.35.2	Test Requirements	896

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	897

A.4.2.36.1	Test Purpose and Environment	897

A.4.2.36.2	Test Requirements	899

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	900

A.4.2.37.1	Test Purpose and Environment	900

A.4.2.37.2	Test Requirements	902

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	903

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	903

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	903

A.4.3.1.1.1	Test Purpose and Environment	903

A.4.3.1.1.2	Test Requirements	905

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	905

A.4.3.1.2.1	Test Purpose and Environment	905

A.4.3.1.2.2	Test Requirements	908

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	908

A.4.3.1.3.1	Test Purpose and Environment	908

A.4.3.1.3.2	Test Requirements	911

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	911

A.4.3.1.4.1	Test Purpose and Environment	911

A.4.3.1.4.2	Test Requirements	912

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	912

A.4.3.1.5.1	Test Purpose and Environment	912

A.4.3.1.5.2	Test Requirements	915

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	916

A.4.3.2.1	Test Purpose and Environment	916

A.4.3.2.1.1	Void	916

A.4.3.2.1.2	1.28Mcps TDD option	916

A.4.3.2.1.3	Void	918

A.4.3.2.2	Test Requirements	918

A.4.3.2.2.1	1.28Mcps TDD option	918

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	918

A.4.3.2A.1	Test Purpose and Environment	918

A.4.3.2A.2	Test Requirements	922

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	922

A.4.3.3.1	Test Purpose and Environment	922

A.4.3.3.2	Test Requirements	925

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	925

A.4.3.3A.1	Test Purpose and Environment	925

A.4.3.3A.2	Test Requirements	929

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	930

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	930

A.4.3.4.1.1	Test Purpose and Environment	930

A.4.3.4.1.2	Test Requirements	932

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	932

A.4.3.4.2.1	Test Purpose and Environment	932

A.4.3.4.2.2	Test Requirements	934

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	934

A.4.3.4.3.1	Test Purpose and Environment	934

A.4.3.4.3.2	Test Requirements	937

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	937

A.4.3.4.4.1	Test Purpose and Environment	937

A.4.3.4.4.2	Test Requirements	941

A.4.4	E-UTRAN to GSM Cell Re-Selection	941

A.4.4.1	E-UTRAN FDD – GSM:	941

A.4.4.1.1	Test Purpose and Environment	941

A.4.4.1.2	Test Requirements	943

A.4.4.2	E-UTRAN TDD – GSM:	943

A.4.4.2.1	Test Purpose and Environment	943

A.4.4.2.2	Test Requirements	945

A.4.5	E-UTRAN to HRPD Cell Re-Selection	946

A.4.5.1	E-UTRAN FDD – HRPD	946

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	946

A.4.5.1.1.1	Test Purpose and Environment	946

A.4.5.1.1.2	Test Requirements	948

A.4.5.2	E-UTRAN TDD – HRPD	948

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	948

A.4.5.2.1.1	Test Purpose and Environment	948

A.4.5.2.1.2	Test Requirements	951

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	951

A.4.6.1	E-UTRAN FDD – cdma2000 1X	951

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	951

A.4.6.1.1.1	Test Purpose and Environment	951

A.4.6.1.1.2	Test Requirements	954

A.4.6.2	E-UTRAN TDD – cdma2000 1X	954

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	954

A.4.6.2.1.1	Test Purpose and Environment	954

A.4.6.2.1.2	Test Requirements	957

A.4.7	Idle State Positioning Measurement for UE category NB1	957

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	957

A.4.7.1.1	Test Purpose and Environment	957

A.4.7.1.2	Test Requirements	961

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	962

A.4.7.2.1	Test Purpose and Environment	962

A.4.7.4.2	Test Requirements	965

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	966

A.4.7.3.1	Test Purpose and Environment	966

A.4.7.3.2	Test Requirements	969

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	970

A.4.7.4.1	Test Purpose and Environment	970

A.4.7.4.2	Test Requirements	973

A.5	E-UTRAN RRC CONNECTED Mode Mobility	974

A.5.1	E-UTRAN Handover	974

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	974

A.5.1.1.1	Test Purpose and Environment	974

A.5.1.1.2	Test Requirements	976

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	976

A.5.1.2.1	Test Purpose and Environment	976

A.5.1.2.2	Test Requirements	978

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	978

A.5.1.3.1	Test Purpose and Environment	978

A.5.1.3.2	Test Requirements	980

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	980

A.5.1.4.1	Test Purpose and Environment	980

A.5.1.4.2	Test Requirements	982

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	982

A.5.1.5.1	Test Purpose and Environment	982

A.5.1.5.2	Test Requirements	984

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	984

A.5.1.6.1	Test Purpose and Environment	984

A.5.1.6.2	Test Requirements	986

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	986

A.5.1.7.1	Test Purpose and Environment	986

A.5.1.7.2	Test Requirements	989

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	989

A.5.1.8.1	Test Purpose and Environment	989

A.5.1.8.2	Test Requirements	992

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	992

A.5.1.9.1	Test Purpose and Environment	992

A.5.1.9.2	Test Requirements	993

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	993

A.5.1.10.1	Test Purpose and Environment	993

A.5.1.10.2	Test Requirements	995

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	995

A.5.1.11.1	Test Purpose and Environment	995

A.5.1.11.2	Test Requirements	997

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	997

A.5.1.12.1	Test Purpose and Environment	997

A.5.1.12.2	Test Requirements	999

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	999

A.5.1.13.1	Test Purpose and Environment	999

A.5.1.13.2	Test Requirements	1001

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1002

A.5.1.14.1	Test Purpose and Environment	1002

A.5.1.14.2	Test Requirements	1003

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1004

A.5.1.15.1	Test Purpose and Environment	1004

A.5.1.15.2	Test Requirements	1005

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1006

A.5.1.16.1	Test Purpose and Environment	1006

A.5.1.16.2	Test Requirements	1007

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1008

A.5.1.17.1	Test Purpose and Environment	1008

A.5.1.17.2	Test Requirements	1009

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1010

A.5.1.18.1	Test Purpose and Environment	1010

A.5.1.18.2	Test Requirements	1011

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1012

A.5.1.19.1	Test Purpose and Environment	1012

A.5.1.19.2	Test Requirements	1013

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1014

A.5.1.20.1	Test Purpose and Environment	1014

A.5.1.20.2	Test Requirements	1015

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1016

A.5.1.21.1	Test Purpose and Environment	1016

A.5.1.21.2	Test Requirements	1017

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1017

A.5.1.22.1	Test Purpose and Environment	1017

A.5.1.22.2	Test Requirements	1019

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1019

A.5.1.23.1	Test Purpose and Environment	1019

A.5.1.23.2	Test Requirements	1021

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1021

A.5.1.24.1	Test Purpose and Environment	1021

A.5.1.24.2	Test Requirements	1023

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1023

A.5.1.25.1	Test Purpose and Environment	1023

A.5.1.25.2	Test Requirements	1025

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1025

A.5.1.26.1	Test Purpose and Environment	1025

A.5.1.26.2	Test Requirements	1027

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1028

A.5.1.27.1	Test Purpose and Environment	1028

A.5.1.27.2	Test Requirements	1029

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1030

A.5.1.28.1	Test Purpose and Environment	1030

A.5.1.28.2	Test Requirements	1031

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1032

A.5.1.29.1	Test Purpose and Environment	1032

A.5.1.29.2	Test Requirements	1033

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1034

A.5.1.30.1	Test Purpose and Environment	1034

A.5.1.30.2	Test Requirements	1035

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1036

A.5.1.31.1	Test Purpose and Environment	1036

A.5.1.31.2	Test Requirements	1037

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1038

A.5.1.32.1	Test Purpose and Environment	1038

A.5.1.32.2	Test Requirements	1039

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1040

A.5.1.33.1	Test Purpose and Environment	1040

A.5.1.13.2	Test Requirements	1041

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1042

A.5.1.34.1	Test Purpose and Environment	1042

A.5.1.34.2	Test Requirements	1043

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1044

A.5.1.35.1	Test Purpose and Environment	1044

A.5.1.35.2	Test Requirements	1045

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1046

A.5.1.36.1	Test Purpose and Environment	1046

A.5.1.36.2	Test Requirements	1047

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1048

A.5.1.37.1	Test Purpose and Environment	1048

A.5.1.37.2	Test Requirements	1049

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1050

A.5.1.38.1	Test Purpose and Environment	1050

A.5.1.38.2	Test Requirements	1051

A.5.2	E-UTRAN Handover to other RATs	1052

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1052

A.5.2.1.1	Test Purpose and Environment	1052

A.5.2.1.2	Test Requirements	1053

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1054

A.5.2.2.1	Test Purpose and Environment	1054

A.5.2.2.2	Test Requirements	1057

A.5.2.3 E-UTRAN FDD- GSM Handover	1057

A.5.2.3.1	Test Purpose and Environment	1057

A.5.2.3.2	Test Requirements	1058

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1059

A.5.2.4.1	Test Purpose and Environment	1059

A.5.2.4.1.1	Void	1059

A.5.2.4.1.2	1.28 Mcps TDD option	1059

A.5.2.4.1.3	Void	1061

A.5.2.4.2	Test Requirements	1061

A.5.2.4.2.1	Void	1061

A.5.2.4.2.2	1.28 Mcps TDD option	1061

A.5.2.4.2.3	Void	1061

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1061

A.5.2.5.1	Test Purpose and Environment	1061

A.5.2.5.1.3	Void	1064

A.5.2.5.2	Test Requirements	1064

A.5.2.5.2.1	Void	1064

A.5.2.5.2.2	1.28 Mcps TDD option	1064

A.5.2.5.2.3	Void	1064

A.5.2.6	E-UTRAN TDD - GSM Handover	1064

A.5.2.6.1	Test Purpose and Environment	1064

A.5.2.6.2	Test Requirements	1066

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1067

A.5.2.7.1	Test Purpose and Environment	1067

A.5.2.7.2	Test Requirements	1069

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1069

A.5.2.8.1	Test Purpose and Environment	1069

A.5.2.8.2	Test Requirements	1070

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1071

A.5.2.9.1	Test Purpose and Environment	1071

A.5.2.9.2	Test Requirements	1072

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1073

A.5.2.10.1	Test Purpose and Environment	1073

A.5.2.10.2	Test Requirements	1075

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1075

A.5.2.10A.1	Test Purpose and Environment	1075

A.5.2.10A.2	Test Requirements	1078

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1078

A.5.2.10B.1	Test Purpose and Environment	1078

A.5.2.10B.2	Test Requirements	1081

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1081

A.5.2.11.1	Test Purpose and Environment	1081

A.5.2.11.2	Test Requirements	1081

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1082

A.5.3.1	E-UTRAN FDD – HRPD Handover	1082

A.5.3.1.1	Test Purpose and Environment	1082

A.5.3.1.2	Test Requirements	1084

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1084

A.5.3.2.1	Test Purpose and Environment	1084

A.5.3.2.2	Test Requirements	1087

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1087

A.5.3.3.1	Test Purpose and Environment	1087

A.5.3.3.2	Test Requirements	1089

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1090

A.5.3.4.1	Test Purpose and Environment	1090

A.5.3.4.2	Test Requirements	1091

A.5.3.5	E-UTRAN TDD – HRPD Handover	1092

A.5.3.5.1	Test Purpose and Environment	1092

A.5.3.5.2	Test Requirements	1094

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1095

A.5.3.6.1	Test Purpose and Environment	1095

A.5.3.6.2	Test Requirements	1098

A.6	RRC Connection Control	1098

A.6.1	RRC Re-establishment	1098

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1098

A.6.1.1.1	Test Purpose and Environment	1098

A.6.1.1.2	Test Requirements	1099

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1100

A.6.1.2.1	Test Purpose and Environment	1100

A.6.1.2.2	Test Requirements	1101

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1102

A.6.1.3.1	Test Purpose and Environment	1102

A.6.1.3.2	Test Requirements	1103

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1104

A.6.1.4.1	Test Purpose and Environment	1104

A.6.1.4.2	Test Requirements	1105

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1106

A.6.1.5.1	Test Purpose and Environment	1106

A.6.1.5.2	Test Requirements	1106

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1106

A.6.1.6.1	Test Purpose and Environment	1106

A.6.1.6.2	Test Requirements	1108

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1109

A.6.1.7.1	Test Purpose and Environment	1109

A.6.1.7.2	Test Requirements	1110

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1111

A.6.1.8.1	Test Purpose and Environment	1111

A.6.1.8.2	Test Requirements	1112

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1113

A.6.1.9.1	Test Purpose and Environment	1113

A.6.1.9.2	Test Requirements	1114

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1115

A.6.1.10.1	Test Purpose and Environment	1115

A.6.1.10.2	Test Requirements	1116

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1117

A.6.1.11.1	Test Purpose and Environment	1117

A.6.1.11.2	Test Requirements	1118

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1119

A.6.1.12.1	Test Purpose and Environment	1119

A.6.1.12.2	Test Requirements	1120

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1121

A.6.1.13.1	Test Purpose and Environment	1121

A.6.1.13.2	Test Requirements	1122

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1123

A.6.1.14.1	Test Purpose and Environment	1123

A.6.1.14.2	Test Requirements	1124

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1125

A.6.1.15.1	Test Purpose and Environment	1125

A.6.1.15.2	Test Requirements	1127

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1128

A.6.1.16.1	Test Purpose and Environment	1128

A.6.1.16.2	Test Requirements	1130

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1131

A.6.1.17.1	Test Purpose and Environment	1131

A.6.1.17.2	Test Requirements	1132

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1133

A.6.1.18.1	Test Purpose and Environment	1133

A.6.1.18.2	Test Requirements	1134

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1135

A.6.1.19.1	Test Purpose and Environment	1135

A.6.1.19.2	Test Requirements	1136

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1137

A.6.1.20.1	Test Purpose and Environment	1137

A.6.1.20.2	Test Requirements	1138

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1139

A.6.1.21.1	Test Purpose and Environment	1139

A.6.1.21.2	Test Requirements	1140

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1141

A.6.1.22.1	Test Purpose and Environment	1141

A.6.1.22.2	Test Requirements	1142

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1143

A.6.1.23.1	Test Purpose and Environment	1143

A.6.1.23.2	Test Requirements	1145

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1146

A.6.1.24.1	Test Purpose and Environment	1146

A.6.1.24.2	Test Requirements	1148

A.6.2	Random Access	1149

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1149

A.6.2.1.1	Test Purpose and Environment	1149

A.6.2.1.2	Test Requirements	1150

A.6.2.1.2.1	Random Access Response Reception	1150

A.6.2.1.2.2	No Random Access Response Reception	1150

A.6.2.1.2.3	Receiving a NACK on msg3	1150

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1151

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1151

A.6.2.1.2.6	Contention Resolution Timer expiry	1151

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1151

A.6.2.2.1	Test Purpose and Environment	1151

A.6.2.2.2	Test Requirements	1152

A.6.2.2.2.1	Random Access Response Reception	1153

A.6.2.2.2.2	No Random Access Response Reception	1153

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1153

A.6.2.3.1	Test Purpose and Environment	1153

A.6.2.3.2	Test Requirements	1155

A.6.2.3.2.1	Random Access Response Reception	1155

A.6.2.3.2.2	No Random Access Response reception	1155

A.6.2.3.2.3	Receiving a NACK on msg3	1155

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1156

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1156

A.6.2.3.2.6	Contention Resolution Timer expiry	1156

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1156

A.6.2.4.1	Test Purpose and Environment	1156

A.6.2.4.2	Test Requirements	1158

A.6.2.4.2.1	Random Access Response Reception	1158

A.6.2.4.2.2	No Random Access Response Reception	1158

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1158

A.6.2.5.1	Test Purpose and Environment	1158

A.6.2.5.2	Test Requirements	1159

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1159

A.6.2.6.1	Test Purpose and Environment	1159

A.6.2.6.2	Test Requirements	1159

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1160

A.6.2.7.1	Test Purpose and Environment	1160

A.6.2.7.2	Test Requirements	1162

A.6.2.7.2.1	Random Access Response Reception	1162

A.6.2.7.2.2	No Random Access Response Reception	1162

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1162

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1163

A.6.2.8.1	Test Purpose and Environment	1163

A.6.2.8.2	Test Requirements	1165

A.6.2.8.2.1	Random Access Response Reception	1165

A.6.2.8.2.2	No Random Access Response Reception	1165

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1165

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1166

A.6.2.9.1	Test Purpose and Environment	1166

A.6.2.9.2	Test Requirements	1168

A.6.2.9.2.1	Random Access Response Reception	1168

A.6.2.9.2.2	No Random Access Response Reception	1169

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1169

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1170

A.6.2.10.1	Test Purpose and Environment	1170

A.6.2.10.2	Test Requirements	1172

A.6.2.10.2.1	Random Access Response Reception	1172

A.6.2.10.2.2	No Random Access Response Reception	1172

A.6.2.10.2.3	Receiving a NACK on msg3	1173

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1173

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1173

A.6.2.10.2.6	Contention Resolution Timer expiry	1173

A.6.2.10.2.7	PRACH Resource Selection	1173

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1173

A.6.2.11.1	Test Purpose and Environment	1173

A.6.2.11.2	Test Requirements	1175

A.6.2.11.2.1	Random Access Response Reception	1175

A.6.2.11.2.2	No Random Access Response Reception	1176

A.6.2.11.2.3	Receiving a NACK on msg3	1176

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1176

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1176

A.6.2.11.2.6	Contention Resolution Timer expiry	1176

A.6.2.11.2.7	PRACH Resource Selection	1177

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1177

A.6.2.12.1	Test Purpose and Environment	1177

A.6.2.12.2	Test Requirements	1179

A.6.2.12.2.1	Random Access Response Reception	1179

A.6.2.12.2.2	No Random Access Response Reception	1180

A.6.2.12.2.3	Receiving a NACK on msg3	1180

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1180

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1180

A.6.2.12.2.6	Contention Resolution Timer expiry	1180

A.6.2.12.2.7	PRACH Resource Selection	1181

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1181

A.6.2.13.1	Test Purpose and Environment	1181

A.6.2.13.2	Test Requirements	1183

A.6.2.13.2.1	Random Access Response Reception	1183

A.6.2.13.2.2	No Random Access Response Reception	1183

A.6.2.13.2.3	Receiving a NACK on msg3	1184

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1184

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1184

A.6.2.13.2.6	Contention Resolution Timer expiry	1184

A.6.2.13.2.7	PRACH Resource Selection	1184

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1184

A.6.2.14.1	Test Purpose and Environment	1184

A.6.2.14.2	Test Requirements	1186

A.6.2.14.2.1	Random Access Response Reception	1186

A.6.2.14.2.2	No Random Access Response Reception	1187

A.6.2.14.2.3	Receiving a NACK on msg3	1187

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1187

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1187

A.6.2.14.2.6	Contention Resolution Timer expiry	1187

A.6.2.14.2.7	PRACH Resource Selection	1188

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1188

A.6.2.15.1	Test Purpose and Environment	1188

A.6.2.15.2	Test Requirements	1190

A.6.2.15.2.1	Random Access Response Reception	1190

A.6.2.15.2.2	No Random Access Response Reception	1191

A.6.2.15.2.3	Receiving a NACK on msg3	1191

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1191

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1191

A.6.2.15.2.6	Contention Resolution Timer expiry	1191

A.6.2.15.2.7	PRACH Resource Selection	1192

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1192

A.6.2.16.1	Test Purpose and Environment	1192

A.6.2.16.2	Test Requirements	1195

A.6.2.16.2.1	Random Access Response Reception	1195

A.6.2.16.2.2	No Random Access Response Reception	1196

A.6.2.16.2.3	Receiving a NACK on msg3	1196

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1196

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1196

A.6.2.16.2.6	Contention Resolution Timer expiry	1196

A.6.2.16.2.7	NPRACH Resource Selection	1196

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1197

A.6.2.17.1	Test Purpose and Environment	1197

A.6.2.17.2	Test Requirements	1200

A.6.2.17.2.1	Random Access Response Reception	1200

A.6.2.17.2.2	No Random Access Response Reception	1201

A.6.2.17.2.3	Receiving a NACK on msg3	1201

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1201

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1201

A.6.2.17.2.6	Contention Resolution Timer expiry	1201

A.6.2.17.2.7	NPRACH Resource Selection	1201

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1202

A.6.2.18.1	Test Purpose and Environment	1202

A.6.2.18.2	Test Requirements	1205

A.6.2.18.2.1	Random Access Response Reception	1205

A.6.2.18.2.2	No Random Access Response Reception	1206

A.6.2.18.2.3	Receiving a NACK on msg3	1206

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1206

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1206

A.6.2.18.2.6	Contention Resolution Timer expiry	1206

A.6.2.18.2.7	NPRACH Resource Selection	1206

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1207

A.6.2.19.1	Test Purpose and Environment	1207

A.6.2.19.2	Test Requirements	1209

A.6.2.19.2.1	Random Access Response Reception	1209

A.6.2.19.2.2	No Random Access Response Reception	1210

A.6.2.19.2.3	Receiving a NACK on msg3	1210

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1210

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1210

A.6.2.19.2.6	Contention Resolution Timer expiry	1210

A.6.2.19.2.7	NPRACH Resource Selection	1210

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1211

A.6.2.20.1	Test Purpose and Environment	1211

A.6.2.20.2	Test Requirements	1213

A.6.2.20.2.1	Random Access Response Reception	1213

A.6.2.20.2.2	No Random Access Response Reception	1214

A.6.2.20.2.3	Receiving a NACK on msg3	1214

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1214

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1214

A.6.2.20.2.6	Contention Resolution Timer expiry	1214

A.6.2.20.2.7	NPRACH Resource Selection	1214

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1215

A.6.2.21.1	Test Purpose and Environment	1215

A.6.2.21.2	Test Requirements	1217

A.6.2.21.2.1	Random Access Response Reception	1217

A.6.2.21.2.2	No Random Access Response Reception	1218

A.6.2.21.2.3	Receiving a NACK on msg3	1218

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1218

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1218

A.6.2.21.2.6	Contention Resolution Timer expiry	1218

A.6.2.21.2.7	NPRACH Resource Selection	1218

A.6.3	RRC Connection Release with Redirection	1219

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1219

A.6.3.1.1	Test Purpose and Environment	1219

A.6.3.1.2	Test Requirements	1221

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1221

A.6.3.2.1	Test Purpose and Environment	1221

A.6.3.2.2	Test Requirements	1223

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1223

A.6.3.3.1	Test Purpose and Environment	1223

A.6.3.3.2	Test Requirements	1224

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1225

A.6.3.4.1	Test Purpose and Environment	1225

A.6.3.4.2	Test Requirements	1226

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1227

A.6.3.5.1	Test Purpose and Environment	1227

A.6.3.5.2	Test Requirements	1229

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1229

A.6.3.6.1	Test Purpose and Environment	1229

A. 6.3.6.2	Test Requirements	1231

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1231

A.6.3.7.1	Test Purpose and Environment	1231

A.6.3.7.2	Test Requirements	1234

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1234

A.6.3.8.1	Test Purpose and Environment	1234

A.6.3.8.2	Test Requirements	1237

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1237

A.6.3.9.1	Test Purpose and Environment	1237

A.6.3.9.2	Test Requirements	1239

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1239

A.6.3.10.1	Test Purpose and Environment	1239

A.6.3.10.2	Test Requirements	1241

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1241

A.6.3.11.1	Test Purpose and Environment	1241

A.6.3.11.2	Test Requirements	1243

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1243

A.6.3.12.1	Test Purpose and Environment	1243

A.6.3.12.2	Test Requirements	1246

A.7	Timing and Signalling Characteristics	1247

A.7.1	UE Transmit Timing	1247

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1247

A.7.1.1.1	Test Purpose and Environment	1247

A.7.1.1.2	Test Requirements	1248

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1249

A.7.1.2.1	Test Purpose and Environment	1249

A.7.1.2.2	Test Requirements	1251

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1252

A.7.1.3.1	Test Purpose and Environment	1252

A.7.1.3.2	Test Requirements	1254

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1255

A.7.1.4.1	Test Purpose and Environment	1255

A.7.1.4.2	Test Requirements	1256

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1257

A.7.1.4A.1	Test Purpose and Environment	1257

A.7.1.4A.2	Test Requirements	1257

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1257

A.7.1.5.1	Test Purpose and Environment	1257

A.7.1.5.2	Test Requirements	1258

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1258

A.7.1.6.1	Test Purpose and Environment	1258

A.7.1.6.2	Test Requirements	1260

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1261

A.7.1.7.1	Test Purpose and Environment	1261

A.7.1.7.2	Test Requirements	1262

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1263

A.7.1.7A.1	Test Purpose and Environment	1263

A.7.1.7A.2	Test Requirements	1263

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1263

A.7.1.7B.1	Test Purpose and Environment	1263

A.7.1.7B.2	Test Requirements	1263

A.7.1.8	Void	1264

A.7.1.8.1	Void	1264

A.7.1.8.2	Void	1264

A.7.1.9	Void	1264

A.7.1.9.1	Void	1264

A.7.1.9.2	Void	1264

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1264

A.7.1.10.1	Test Purpose and Environment	1264

A.7.1.10.2	Test Requirements	1266

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1266

A.7.1.11.1	Test Purpose and Environment	1266

A.7.1.11.2	Test Requirements	1268

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1268

A.7.1.12.1	Test Purpose and Environment	1268

A.7.1.12.2	Test Requirements	1270

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1270

A.7.1.13.1	Test Purpose and Environment	1270

A.7.1.13.2	Test Requirements	1272

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1273

A.7.1.14.1	Test Purpose and Environment	1273

A.7.1.14.2	Test Requirements	1274

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1274

A.7.1.15.1	Test Purpose and Environment	1274

A.7.1.15.2	Test Requirements	1275

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1276

A.7.1.16.1	Test Purpose and Environment	1276

A.7.1.16.2	Test Requirements	1277

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1278

A.7.1.17.1	Test Purpose and Environment	1278

A.7.1.17.2	Test Requirements	1280

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1281

A.7.1.18.1	Test Purpose and Environment	1281

A.7.1.18.2	Test Requirements	1283

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1284

A.7.1.19.1	Test Purpose and Environment	1284

A.7.1.19.2	Test Requirements	1285

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1285

A.7.1.20.1	Test Purpose and Environment	1285

A.7.1.20.2	Test Requirements	1287

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1287

A.7.1.21.1	Test Purpose and Environment	1287

A.7.1.21.2	Test Requirements	1289

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1289

A.7.1.22.1	Test Purpose and Environment	1289

A.7.1.22.2	Test Requirements	1291

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1291

A.7.1.23.1	Test Purpose and Environment	1291

A.7.1.23.2	Test Requirements	1293

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1293

A.7.1.24.1	Test Purpose and Environment	1293

A.7.1.24.2	Test Requirements	1294

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1295

A.7.1.25.1	Test Purpose and Environment	1295

A.7.1.25.2	Test Requirements	1296

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1297

A.7.1.26.1	Test Purpose and Environment	1297

A.7.1.26.2	Test Requirements	1298

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1299

A.7.1.27.1	Test Purpose and Environment	1299

A.7.1.27.2	Test Requirements	1301

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1301

A.7.1.28.1	Test Purpose and Environment	1301

A.7.1.28.2	Test Requirements	1303

A.7.2	UE Timing Advance	1304

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1304

A.7.2.1.1	Test Purpose and Environment	1304

A.7.2.1.2	Test Requirements	1306

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1306

A.7.2.2.1	Test Purpose and Environment	1306

A.7.2.2.2	Test Requirements	1308

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1308

A.7.2.3.1	Test Purpose and Environment	1308

A.7.2.3.2	Test Requirements	1309

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1309

A.7.2.4.1	Test Purpose and Environment	1309

A.7.2.4.2	Test Requirements	1311

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1311

A.7.2.5.1	Test Purpose and Environment	1311

A.7.2.5.2	Test Requirements	1313

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1313

A.7.2.5A.1	Test Purpose and Environment	1313

A.7.2.5A.2	Test Requirements	1314

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1314

A.7.2.5B.1	Test Purpose and Environment	1314

A.7.2.5B.2	Test Requirements	1314

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1314

A.7.2.6.1	Test Purpose and Environment	1314

A.7.2.6.2	Test Requirements	1317

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1317

A.7.2.7.1	Test Purpose and Environment	1317

A.7.2.7.2	Test Requirements	1319

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1319

A.7.2.8.1	Test Purpose and Environment	1319

A.7.2.8.2	Test Requirements	1321

A.7.2.9.2	Test Requirements	1323

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1323

A.7.2.10.1	Test Purpose and Environment	1323

A.7.2.10.2	Test Requirements	1324

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1324

A.7.2.11.1	Test Purpose and Environment	1324

A.7.2.11.2	Test Requirements	1326

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1326

A.7.2.12.1	Test Purpose and Environment	1326

A.7.2.12.2	Test Requirements	1328

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1328

A.7.2.13.1	Test Purpose and Environment	1328

A.7.2.13.2	Test Requirements	1330

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1330

A.7.2.14.1	Test Purpose and Environment	1330

A.7.2.14.2	Test Requirements	1332

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1332

A.7.2.15.1	Test Purpose and Environment	1332

A.7.2.15.2	Test Requirements	1334

A.7.3	Radio Link Monitoring	1334

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1335

A.7.3.1.1	Test Purpose and Environment	1335

A.7.3.1.2	Test Requirements	1339

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1339

A.7.3.2.1	Test Purpose and Environment	1339

A.7.3.2.2	Test Requirements	1342

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1342

A.7.3.3.1	Test Purpose and Environment	1342

A.7.3.3.2	Test Requirements	1346

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1346

A.7.3.4.1	Test Purpose and Environment	1346

A.7.3.4.2	Test Requirements	1349

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1349

A.7.3.5.1	Test Purpose and Environment	1349

A.7.3.5.2	Test Requirements	1353

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1354

A.7.3.6.1	Test Purpose and Environment	1354

A.7.3.6.2	Test Requirements	1357

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1357

A.7.3.7.1	Test Purpose and Environment	1357

A.7.3.7.2	Test Requirements	1360

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1360

A.7.3.8.1	Test Purpose and Environment	1360

A.7.3.8.2	Test Requirements	1363

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1363

A.7.3.9.2	Test Requirements	1366

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1366

A.7.3.10.1	Test Purpose and Environment	1366

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1369

A.7.3.11.1	Test Purpose and Environment	1369

A.7.3.11.2	Test Requirements	1373

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1373

A.7.3.12.1	Test Purpose and Environment	1373

A.7.3.12.2	Test Requirements	1377

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1377

A.7.3.13.1	Test Purpose and Environment	1377

A.7.3.13.2	Test Requirements	1380

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1380

A.7.3.14.1	Test Purpose and Environment	1380

A.7.3.14.2	Test Requirements	1383

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1383

A.7.3.15.1	Test Purpose and Environment	1383

A.7.3.15.2	Test Requirements	1386

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1386

A.7.3.16.1	Test Purpose and Environment	1386

A.7.3.16.2	Test Requirements	1390

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1390

A.7.3.17.1	Test Purpose and Environment	1390

A.7.3.17.2	Test Requirements	1393

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1393

A.7.3.18.1	Test Purpose and Environment	1393

A.7.3.18.2	Test Requirements	1397

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1397

A.7.3.19.1	Test Purpose and Environment	1397

A.7.3.19.2	Test Requirements	1401

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1401

A.7.3.20.1	Test Purpose and Environment	1401

A.7.3.20.2	Test Requirements	1405

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1405

A.7.3.21.1	Test Purpose and Environment	1405

A.7.3.21.2	Test Requirements	1409

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1409

A.7.3.22.1	Test Purpose and Environment	1409

A.7.3.22.2	Test Requirements	1413

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1413

A.7.3.23.1	Test Purpose and Environment	1413

A.7.3.23.2	Test Requirements	1414

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1414

A.7.3.24.1	Test Purpose and Environment	1414

A.7.3.24.2	Test Requirements	1415

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1415

A.7.3.25.1	Test Purpose and Environment	1415

A.7.3.25.2	Test Requirements	1416

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1416

A.7.3.26.1	Test Purpose and Environment	1416

A.7.3.26.2	Test Requirements	1419

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1419

A.7.3.27.1	Test Purpose and Environment	1419

A.7.3.27.2	Test Requirements	1422

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1422

A.7.3.28.1	Test Purpose and Environment	1422

A.7.3.28.2	Test Requirements	1425

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1425

A.7.3.29.1	Test Purpose and Environment	1425

A.7.3.29.2	Test Requirements	1428

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1428

A.7.3.30.1	Test Purpose and Environment	1428

A.7.3.30.2	Test Requirements	1431

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1431

A.7.3.31.1	Test Purpose and Environment	1431

A.7.3.31.2	Test Requirements	1434

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1434

A.7.3.32.1	Test Purpose and Environment	1434

A.7.3.32.2	Test Requirements	1437

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1437

A.7.3.33.1	Test Purpose and Environment	1437

A.7.3.33.2	Test Requirements	1440

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1440

A.7.3.34.1	Test Purpose and Environment	1440

A.7.3.34.2	Test Requirements	1443

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1443

A.7.3.35.1	Test Purpose and Environment	1443

A.7.3.35.2	Test Requirements	1446

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1446

A.7.3.36.1	Test Purpose and Environment	1446

A.7.3.36.2	Test Requirements	1449

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1449

A.7.3.37.1	Test Purpose and Environment	1449

A.7.3.37.2	Test Requirements	1452

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1452

A.7.3.38.1	Test Purpose and Environment	1452

A.7.3.38.2	Test Requirements	1455

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1455

A.7.3.39.1	Test Purpose and Environment	1455

A.7.3.39.2	Test Requirements	1458

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1459

A.7.3.40.1	Test Purpose and Environment	1459

A.7.3.40.2	Test Requirements	1461

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1462

A.7.3.41.1	Test Purpose and Environment	1462

A.7.3.41.2	Test Requirements	1465

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1465

A.7.3.42.1	Test Purpose and Environment	1465

A.7.3.42.2	Test Requirements	1468

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1468

A.7.3.43.1	Test Purpose and Environment	1468

A.7.3.43.2	Test Requirements	1471

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1471

A.7.3.44.1	Test Purpose and Environment	1471

A.7.3.44.2	Test Requirements	1474

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1475

A.7.3.45.1	Test Purpose and Environment	1475

A.7.3.45.2	Test Requirements	1477

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1478

A.7.3.46.1	Test Purpose and Environment	1478

A.7.3.46.2	Test Requirements	1481

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1481

A.7.3.47.1	Test Purpose and Environment	1481

A.7.3.47.2	Test Requirements	1484

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1484

A.7.3.48.1	Test Purpose and Environment	1484

A.7.3.48.2	Test Requirements	1487

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1487

A.7.3.49.1	Test Purpose and Environment	1487

A.7.3.49.2	Test Requirements	1490

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1490

A.7.3.50.1	Test Purpose and Environment	1490

A.7.3.50.2	Test Requirements	1493

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1493

A.7.3.51.1	Test Purpose and Environment	1493

A.7.3.51.2	Test Requirements	1496

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1496

A.7.3.52.1	Test Purpose and Environment	1496

A.7.3.52.2	Test Requirements	1499

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1499

A.7.3.53.1	Test Purpose and Environment	1499

A.7.3.53.2	Test Requirements	1502

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1502

A.7.3.54.1	Test Purpose and Environment	1502

A.7.3.54.2	Test Requirements	1505

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1505

A.7.3.55.1	Test Purpose and Environment	1505

A.7.3.55.2	Test Requirements	1508

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1508

A.7.3.56.1	Test Purpose and Environment	1508

A.7.3.56.2	Test Requirements	1511

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1511

A.7.3.57.1	Test Purpose and Environment	1511

A.7.3.57.2	Test Requirements	1514

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1514

A.7.3.58.1	Test Purpose and Environment	1514

A.7.3.58.2	Test Requirements	1517

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1517

A.7.3.59.1	Test Purpose and Environment	1517

A.7.3.59.2	Test Requirements	1520

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1520

A.7.3.60.1	Test Purpose and Environment	1520

A.7.3.60.2	Test Requirements	1524

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1524

A.7.3.61.1	Test Purpose and Environment	1524

A.7.3.61.2	Test Requirements	1528

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1528

A.7.3.62.1	Test Purpose and Environment	1528

A.7.3.62.2	Test Requirements	1532

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1532

A.7.3.63.1	Test Purpose and Environment	1532

A.7.3.63.2	Test Requirements	1536

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1536

A.7.3.64.1	Test Purpose and Environment	1536

A.7.3.64.2	Test Requirements	1539

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1540

A.7.3.65.1	Test Purpose and Environment	1540

A.7.3.65.2	Test Requirements	1543

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1544

A.7.3.66.1	Test Purpose and Environment	1544

A.7.3.66.2	Test Requirements	1546

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1547

A.7.3.67.1	Test Purpose and Environment	1547

A.7.3.67.2	Test Requirements	1550

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1551

A.7.3.68.1	Test Purpose and Environment	1551

A.7.3.68.2	Test Requirements	1553

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1553

A.7.3.69.1	Test Purpose and Environment	1553

A.7.3.69.2	Test Requirements	1555

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1555

A.7.3.70.1	Test Purpose and Environment	1555

A.7.3.70.2	Test Requirements	1557

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1557

A.7.3.71.1	Test Purpose and Environment	1557

A.7.3.71.2	Test Requirements	1560

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1560

A.7.3.72.1	Test Purpose and Environment	1560

A.7.3.72.2	Test Requirements	1562

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1562

A.7.3.73.1	Test Purpose and Environment	1562

A.7.3.73.2	Test Requirements	1564

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1564

A.7.3.74.1	Test Purpose and Environment	1564

A.7.3.74.2	Test Requirements	1567

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1567

A.7.3.75.1	Test Purpose and Environment	1567

A.7.3.75.2	Test Requirements	1569

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1570

A.7.3.76.1	Test Purpose and Environment	1570

A.7.3.76.2	Test Requirements	1573

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1573

A.7.3.77.1	Test Purpose and Environment	1573

A.7.3.77.2	Test Requirements	1576

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1576

A.7.3.78.1	Test Purpose and Environment	1576

A.7.3.78.2	Test Requirements	1579

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1579

A.7.3.79.1	Test Purpose and Environment	1579

A.7.3.79.2	Test Requirements	1582

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1582

A.7.3.80.1	Test Purpose and Environment	1582

A.7.3.80.2	Test Requirements	1585

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1585

A.7.3.81.1	Test Purpose and Environment	1585

A.7.3.81.2	Test Requirements	1588

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1588

A.7.3.82.1	Test Purpose and Environment	1588

A.7.3.82.2	Test Requirements	1591

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1591

A.7.3.83.1	Test Purpose and Environment	1591

A.7.3.83.2	Test Requirements	1592

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1593

A.7.3.84.1	Test Purpose and Environment	1593

A.7.3.84.2	Test Requirements	1595

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1595

A.7.3.85.1	Test Purpose and Environment	1595

A.7.3.85.2	Test Requirements	1596

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1597

A.7.3.86.1	Test Purpose and Environment	1597

A.7.3.86.2	Test Requirements	1599

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1599

A.7.3.87.1	Test Purpose and Environment	1599

A.7.3.87.2	Test Requirements	1601

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1602

A.7.3.88.1	Test Purpose and Environment	1602

A.7.3.88.2	Test Requirements	1606

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1606

A.7.3.89.1	Test Purpose and Environment	1606

A.7.3.89.2	Test Requirements	1610

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1610

A.7.3.90.1	Test Purpose and Environment	1610

A.7.3.90.2	Test Requirements	1614

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1614

A.7.3.91.1	Test Purpose and Environment	1614

A.7.3.91.2	Test Requirements	1618

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1618

A.7.3.92.1	Test Purpose and Environment	1618

A.7.3.92.2	Test Requirements	1621

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1622

A.7.3.93.1	Test Purpose and Environment	1622

A.7.3.93.2	Test Requirements	1625

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1626

A.7.3.94.1	Test Purpose and Environment	1626

A.7.3.94.2	Test Requirements	1628

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1629

A.7.3.95.1	Test Purpose and Environment	1629

A.7.3.95.2	Test Requirements	1632

A.7.4	Interruption for Dual Connectivity	1633

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1633

A.7.4.1.1	Test Purpose and Environment	1633

A.7.4.1.2	Test Requirements	1635

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1635

A.7.4.2.1	Test Purpose and Environment	1635

A.7.4.2.2	Test Requirements	1637

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1637

A.7.4.3.1	Test Purpose and Environment	1637

A.7.4.3.2	Test Requirements	1639

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1639

A.7.4.4.1	Test Purpose and Environment	1639

A.7.4.4.2	Test Requirements	1642

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1642

A.7.4.5.1	Test Purpose and Environment	1642

A.7.4.5.2	Test Requirements	1644

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1644

A.7.4.6.1	Test Purpose and Environment	1644

A.7.4.6.2	Test Requirements	1646

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1646

A.7.4.7.1	Test Purpose and Environment	1646

A.7.4.7.2	Test Requirements	1649

A.7.5	Proximity-based Services	1649

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1649

A.7.5.1.1	Test Purpose and Environment	1649

A.7.5.1.2	Test Requirements	1650

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1651

A.7.5.2.1	Test Purpose and Environment	1651

A.7.5.1.2	Test Requirements	1651

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1652

A.7.5.3.1	Test Purpose and Environment	1652

A.7.5.3.2	Test Requirements	1653

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1654

A.7.5.4.1	Test Purpose and Environment	1654

A.7.5.4.2	Test Requirements	1655

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1656

A.7.5.5.1	Test Purpose and Environment	1656

A.7.5.5.2	Test Requirements	1658

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1659

A.7.5.6.1	Test Purpose and Environment	1659

A.7.5.6.2	Test Requirements	1660

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1661

A.7.5.7.1	Test Purpose and Environment	1661

A.7.5.7.2	Test Requirements	1663

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1663

A.7.5.8.1	Test Purpose and Environment	1663

A.7.5.8.2	Test Requirements	1665

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1666

A.7.5.9.1	Test Purpose and Environment	1666

A.7.5.9.2	Test Requirements	1668

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1669

A.7.5.10.1	Test Purpose and Environment	1669

A.7.5.10.2	Test Requirements	1671

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1672

A.7.5.11.1	Test Purpose and Environment	1672

A.7.5.11.2	Test Requirements	1674

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1674

A.7.5.12.1	Test Purpose and Environment	1674

A.7.5.12.2	Test Requirements	1678

A.7.6	Interruption for carrier aggregation	1679

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1679

A.7.6.1.1	Test Purpose and Environment	1679

A.7.6.1.2	Test Requirements	1681

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1681

A.7.6.2.1	Test Purpose and Environment	1681

A.7.6.2.2	Test Requirements	1684

A.8	UE Measurements Procedures	1684

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1684

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1684

A.8.1.1.1	Test Purpose and Environment	1684

A.8.1.1.2	Test Requirements	1686

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1687

A.8.1.2.1	Test Purpose and Environment	1687

A.8.1.2.2	Test Requirements	1688

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1689

A.8.1.3.1	Test Purpose and Environment	1689

A.8.1.3.2	Test Requirements	1691

A.8.1.4	Void	1691

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1691

A.8.1.5.1	Test Purpose and Environment	1691

A.8.1.5.2	Test Requirements	1693

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1693

A.8.1.6.1	Test Purpose and Environment	1693

A.8.1.6.2	Test Requirements	1695

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1695

A.8.1.7.1	Test Purpose and Environment	1695

A.8.1.7.2	Test Requirements	1697

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1698

A.8.1.8.1	Test Purpose and Environment	1698

A.8.1.8.2	Test Requirements	1700

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1701

A.8.1.9.1	Test Purpose and Environment	1701

A.8.1.9.2	Test Requirements	1701

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1701

A.8.1.10.1	Test Purpose and Environment	1701

A.8.1.10.2	Test Requirements	1702

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1702

A.8.1.11.1	Test Purpose and Environment	1702

A.8.1.11.2	Test Requirements	1704

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1705

A.8.1.12.1	Test Purpose and Environment	1705

A.8.1.12.2	Test Requirements	1706

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1707

A.8.1.13.1	Test Purpose and Environment	1707

A.8.1.13.2	Test Requirements	1709

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1709

A.8.1.14.1	Test Purpose and Environment	1709

A.8.1.14.2	Test Requirements	1711

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1712

A.8.1.15.1	Test Purpose and Environment	1712

A.8.1.15.2	Test Requirements	1713

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1714

A.8.1.16.1	Test Purpose and Environment	1714

A.8.1.16.2	Test Requirements	1716

A.8.1.17	Void	1716

A.8.1.18	Void	1716

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1716

A.8.1.19.1	Test Purpose and Environment	1716

A.8.1.19.2	Test Requirements	1718

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1719

A.8.1.20.1	Test Purpose and Environment	1719

A.8.1.20.2	Test Requirements	1721

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1721

A.8.1.21.1	Test Purpose and Environment	1721

A.8.1.21.2	Test Requirements	1723

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1724

A.8.1.22.1	Test Purpose and Environment	1724

A.8.1.22.2	Test Requirements	1726

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1726

A.8.1.23.1	Test Purpose and Environment	1726

A.8.1.23.2	Test Requirements	1727

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1728

A.8.1.24.1	Test Purpose and Environment	1728

A.8.1.24.2	Test Requirements	1730

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1730

A.8.1.25.1	Test Purpose and Environment	1730

A.8.1.25.2	Test Requirements	1732

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1732

A.8.1.26.1	Test Purpose and Environment	1732

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1735

A.8.1.27.1	Test Purpose and Environment	1735

A.8.1.27.2	Test Requirements	1737

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1737

A.8.1.28.1	Test Purpose and Environment	1737

A.8.1.28.2	Test Requirements	1739

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1739

A.8.1.29.1	Test Purpose and Environment	1739

A.8.1.29.2	Test Requirements	1741

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1742

A.8.1.30.1	Test Purpose and Environment	1742

A.8.1.30.2	Test Requirements	1744

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1744

A.8.1.31.1	Test Purpose and Environment	1744

A.8.1.31.2	Test Requirements	1746

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1747

A.8.1.32.1	Test Purpose and Environment	1747

A.8.1.32.2	Test Requirements	1748

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1749

A.8.1.33.1	Test Purpose and Environment	1749

A.8.1.33.2	Test Requirements	1750

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1751

A.8.1.34.1	Test Purpose and Environment	1751

A.8.1.34.2	Test Requirements	1752

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1753

A.8.1.35.1	Test Purpose and Environment	1753

A.8.1.35.2	Test Requirements	1754

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1755

A.8.1.36.1	Test Purpose and Environment	1755

A.8.1.36.2	Test Requirements	1756

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1757

A.8.1.37.1	Test Purpose and Environment	1757

A.8.1.37.2	Test Requirements	1759

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1759

A.8.1.38.1	Test Purpose and Environment	1759

A.8.1.38.2	Test Requirements	1760

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1761

A.8.1.39.1	Test Purpose and Environment	1761

A.8.1.39.2	Test Requirements	1763

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1763

A.8.1.40.1	Test Purpose and Environment	1763

A.8.1.40.2	Test Requirements	1765

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1765

A.8.1.41.1	Test Purpose and Environment	1765

A.8.1.41.2	Test Requirement	1767

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1767

A.8.1.42.1	Test Purpose and Environment	1767

A.8.1.42.2	Test Requirement	1768

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1769

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1769

A.8.2.1.1	Test Purpose and Environment	1769

A.8.2.1.2	Test Requirements	1770

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1770

A.8.2.2.1	Test Purpose and Environment	1770

A.8.2.2.2	Test Requirements	1773

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1773

A.8.2.3.1	Test Purpose and Environment	1773

A.8.2.3.2	Test Requirements	1775

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1776

A.8.2.4.1	Test Purpose and Environment	1776

A.8.2.4.2	Test Requirements	1778

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1778

A.8.2.5.1	Test Purpose and Environment	1778

A.8.2.5.2	Test Requirements	1780

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1781

A.8.2.6.1	Test Purpose and Environment	1781

A.8.2.6.2	Test Requirements	1783

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1784

A.8.2.7.1	Test Purpose and Environment	1784

A.8.2.7.2	Test Requirements	1785

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1786

A.8.2.8.1	Test Purpose and Environment	1786

A.8.2.8.2	Test Requirements	1788

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1788

A.8.2.9.1	Test Purpose and Environment	1788

A.8.2.9.2	Test Requirements	1790

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1791

A.8.2.10.1	Test Purpose and Environment	1791

A.8.2.10.2	Test Requirements	1793

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1793

A.8.2.11.1	Test Purpose and Environment	1793

A.8.2.11.2	Test Requirements	1795

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1795

A.8.2.12.1	Test Purpose and Environment	1795

A.8.2.12.2	Test Requirements	1797

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1798

A.8.2.13.1	Test Purpose and Environment	1798

A.8.2.13.2	Test Requirements	1800

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1800

A.8.2.14.1	Test Purpose and Environment	1800

A.8.2.14.2	Test Requirement	1802

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1803

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1803

A.8.3.1.1	Test Purpose and Environment	1803

A.8.3.1.2	Test Requirements	1804

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1804

A.8.3.2.1	Test Purpose and Environment	1804

A.8.3.2.2	Test Requirements	1807

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1807

A.8.3.3.1	Test Purpose and Environment	1807

A.8.3.3.2	Test Requirements	1810

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1810

A.8.3.4.1	Test Purpose and Environment	1810

A.8.3.4.2	Test Requirements	1812

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1812

A.8.3.5.2	Test Requirements	1814

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1814

A.8.3.6.1	Test Purpose and Environment	1814

A.8.3.6.2	Test Requirements	1816

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1817

A.8.3.7.1	Test Purpose and Environment	1817

A.8.3.7.2	Test Requirements	1819

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1820

A.8.3.8.1	Test Purpose and Environment	1820

A.8.3.8.2	Test Requirements	1823

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1823

A.8.3.9.1	Test Purpose and Environment	1823

A.8.3.9.2	Test Requirements	1826

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1827

A.8.3.10.1	Test Purpose and Environment	1827

A.8.3.10.2	Test Requirements	1828

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1828

A.8.3.11.1	Test Purpose and Environment	1828

A.8.3.11.2	Test Requirement	1830

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1831

A.8.3.12.1	Test Purpose and Environment	1831

A.8.3.12.2	Test Requirement	1832

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1833

A.8.3.13.1	Test Purpose and Environment	1833

A.8.3.13.2	Test Requirement	1834

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1835

A.8.3.14.1	Test Purpose and Environment	1835

A.8.3.14.2	Test Requirement	1836

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1837

A.8.3.15.1	Test Purpose and Environment	1837

A.8.3.15.2	Test Requirement	1838

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1839

A.8.3.16.1	Test Purpose and Environment	1839

A.8.3.16.2	Test Requirement	1841

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1841

A.8.3.17.1	Test Purpose and Environment	1841

A.8.3.17.2	Test Requirement	1844

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1844

A.8.3.18.1	Test Purpose and Environment	1844

A.8.3.18.2	Test Requirement	1847

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1847

A.8.3.19.1	Test Purpose and Environment	1847

A.8.3.19.2	Test Requirement	1850

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1850

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1850

A.8.4.1.1	Test Purpose and Environment	1850

A.8.4.1.2	Test Requirements	1852

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1852

A.8.4.2.1	Test Purpose and Environment	1852

A.8.4.2.2	Test Requirements	1855

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1855

A.8.4.3.1	Test Purpose and Environment	1855

A.8.4.3.2	Test Requirements	1857

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1858

A.8.4.4.1	Test Purpose and Environment	1858

A.8.4.4.2	Test Requirements	1860

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1861

A.8.4.5.1	Test Purpose and Environment	1861

A.8.4.5.2	Test Requirements	1863

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1863

A.8.4.6.1	Test Purpose and Environment	1863

A.8.4.6.2	Test Requirements	1864

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1864

A.8.4.7.1	Test Purpose and Environment	1864

A.8.4.7.2	Test Requirements	1868

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1868

A.8.4.8.1	Test Purpose and Environment	1868

A.8.4.8.2	Test Requirements	1871

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1871

A.8.4.9.1	Test Purpose and Environment	1871

A.8.4.9.2	Test Requirements	1875

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1875

A.8.4.10.1	Test Purpose and Environment	1875

A.8.4.10.2	Test Requirements	1877

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1877

A.8.4.11.1	Test Purpose and Environment	1877

A.8.4.11.2	Test Requirement	1879

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1879

A.8.4.12.1	Test Purpose and Environment	1879

A.8.4.12.2	Test Requirement	1881

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1882

A.8.4.13.1	Test Purpose and Environment	1882

A.8.4.13.2	Test Requirement	1883

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1884

A.8.4.14.1	Test Purpose and Environment	1884

A.8.4.14.2	Test Requirement	1886

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1886

A.8.4.15.1	Test Purpose and Environment	1886

A.8.4.15.2	Test Requirement	1889

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1889

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1889

A.8.5.1.1	Test Purpose and Environment	1889

A.8.5.1.2	Test Requirements	1891

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1891

A.8.5.2.1	Test Purpose and Environment	1891

A.8.5.2.2	Test Requirements	1893

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1893

A.8.5.3.1	Test Purpose and Environment	1893

A.8.5.3.2	Test Requirements	1896

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1896

A.8.5.4.1	Test Purpose and Environment	1896

A.8.5.4.2	Test Requirements	1898

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1898

A.8.5.5.1	Test Purpose and Environment	1898

A.8.5.5.2	Test Requirements	1901

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1901

A.8.5.6.1	Test Purpose and Environment	1901

A.8.5.6.2	Test Requirements	1902

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1903

A.8.5.7.1	Test Purpose and Environment	1903

A.8.5.7.2	Test Requirements	1903

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1903

A.8.5.8.1	Test Purpose and Environment	1903

A.8.5.8.2	Test Requirements	1906

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1906

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1906

A.8.6.1.1	Test Purpose and Environment	1906

A.8.6.1.2	Test Requirements	1908

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1908

A.8.6.2.1	Test Purpose and Environment	1908

A.8.6.2.2	Test Requirements	1911

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1911

A.8.6.3.1	Test Purpose and Environment	1911

A.8.6.3.2	Test Requirements	1914

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1914

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1914

A.8.7.1.1	Test Purpose and Environment	1914

A.8.7.1.1.1	Void	1914

A.8.7.1.1.2	1.28 Mcps TDD option	1914

A.8.7.1.1.3	Void	1916

A.8.7.1.2	Test Requirements	1916

A.8.7.1.2.1	Void	1916

A.8.7.1.2.2	1.28 Mcps TDD option	1916

A.8.7.1.2.3	Void	1916

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1916

A.8.7.2.1	Test Purpose and Environment	1916

A.8.7.2.2	Test Requirements	1919

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1920

A.8.7.3.1	Test Purpose and Environment	1920

A.8.7.3.2	Test Parameters	1920

A.8.7.3.3	Test Requirements	1921

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1922

A.8.7.4.1	Test Purpose and Environment	1922

A.8.7.4.2	Test Requirements	1924

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1924

A.8.7.5.1	Test Purpose and Environment	1924

A.8.7.5.2	Test Requirements	1926

A.8.7A	TBD	1926

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1926

A.8.7A.1.1	Test Purpose and Environment	1926

A.8.7A.1.2	Test Requirements	1929

A.8.8	E-UTRAN FDD – GSM Measurements	1929

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1929

A.8.8.1.1	Test Purpose and Environment	1929

A.8.8.1.2	Test Requirements	1931

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1931

A.8.8.2.1	Test Purpose and Environment	1931

A.8.8.2.2	Test Requirements	1933

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1934

A.8.8.3.1	Test Purpose and Environment	1934

A.8.8.3.2	Test Requirements	1935

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1936

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1936

A.8.9.1.1	Test Purpose and Environment	1936

A.8.9.1.2	Test Requirements	1937

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1938

A.8.9.2.1	Test Purpose and Environment	1938

A.8.9.2.2	Test Requirements	1940

A.8.10	E-UTRAN TDD – GSM Measurements	1940

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1940

A.8.10.1.1	Test Purpose and Environment	1940

A.8.10.1.2	Test Requirements	1941

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1942

A.8.10.2.1	Test Purpose and Environment	1942

A.8.10.2.2	Test Requirements	1944

A.8.11	Monitoring of Multiple Layers	1944

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1944

A.8.11.1.1	Test Purpose and Environment	1944

A.8.11.1.2	Test Requirements	1946

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1947

A.8.11.2.1	Test Purpose and Environment	1947

A.8.11.2.2	Test Requirements	1948

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1948

A.8.11.3.1	Test Purpose and Environment	1948

A.8.11.3.2	Test Requirements	1951

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1951

A.8.11.4.1	Test Purpose and Environment	1951

A.8.11.4.2	Test Requirements	1954

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1954

A.8.11.5.1	Test Purpose and Environment	1954

A.8.11.5.2	Test Requirements	1956

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1957

A.8.11.6.1	Test Purpose and Environment	1957

A.8.11.6.2	Test Requirements	1959

A.8.12	RSTD Intra-frequency Measurements	1960

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1960

A.8.12.1.1	Test Purpose and Environment	1960

A.8.12.1.2	Test Requirements	1963

A.8.12.1.2A	Test Requirements for UE Category 1bis	1964

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1964

A.8.12.2.1	Test Purpose and Environment	1964

A.8.12.2.2	Test Requirements	1968

A.8.12.2.2A	Test Requirements for UE Category 1bis	1969

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1969

A.8.12.3.1	Test Purpose and Environment	1969

A.8.12.3.2	Test Requirements	1973

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1974

A.8.12.4.1	Test Purpose and Environment	1974

A.8.12.4.2	Test Requirements	1977

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1978

A.8.12.5.1	Test Purpose and Environment	1978

A.8.12.5.2	Test Requirements	1981

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1982

A.8.12.6.1	Test Purpose and Environment	1982

A.8.12.6.2	Test Requirements	1985

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1986

A.8.12.7.1	Test Purpose and Environment	1986

A.8.12.7.2	Test Requirements	1989

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1990

A.8.12.8.1	Test Purpose and Environment	1990

A.8.12.8.2	Test Requirements	1993

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	1994

A.8.12.9.1	Test Purpose and Environment	1994

A.8.12.9.2	Test Requirements	1997

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	1998

A.8.12.10.1	Test Purpose and Environment	1998

A.8.12.10.2	Test Requirements	2001

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2002

A.8.12.11.1	Test Purpose and Environment	2002

A.8.12.11.2	Test Requirements	2006

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2007

A.8.12.12.1	Test Purpose and Environment	2007

A.8.12.12.2	Test Requirements	2010

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2011

A.8.12.13.1	Test Purpose and Environment	2011

A.8.12.13.2	Test Requirements	2014

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2015

A.8.12.14.1	Test Purpose and Environment	2015

A.8.12.14.2	Test Requirements	2019

A.8.13	RSTD Inter-frequency Measurements	2020

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2020

A.8.13.1.1	Test Purpose and Environment	2020

A.8.13.1.2	Test Requirements	2023

A.8.13.1.2A	Test Requirements for UE Category 1bis	2024

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2024

A.8.13.2.1	Test Purpose and Environment	2024

A.8.13.2.2	Test Requirements	2028

A.8.13.2.2A	Test Requirements for UE Category 1bis	2029

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2029

A.8.13.3.1	Test Purpose and Environment	2029

A.8.13.3.2	Test Requirements	2033

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2034

A.8.13.4.1	Test Purpose and Environment	2034

A.8.13.4.2	Test Requirements	2037

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2038

A.8.13.5.1	Test Purpose and Environment	2038

A.8.13.5.2	Test Requirements	2042

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2043

A.8.13.6.1	Test Purpose and Environment	2043

A.8.13.6.2	Test Requirements	2046

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2047

A.8.13.7.1	Test Purpose and Environment	2047

A.8.13.7.2	Test Requirements	2050

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2051

A.8.13.8.1	Test Purpose and Environment	2051

A.8.13.8.2	Test Requirements	2055

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2056

A.8.13.9.1	Test Purpose and Environment	2056

A.8.13.9.2	Test Requirements	2060

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2061

A.8.13.10.1	Test Purpose and Environment	2061

A.8.13.10.2	Test Requirements	2065

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2066

A.8.13.11.1	Test Purpose and Environment	2066

A.8.13.11.2	Test Requirements	2070

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2071

A.8.13.12.1	Test Purpose and Environment	2071

A.8.13.12.2	Test Requirements	2075

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2076

A.8.13.13.1	Test Purpose and Environment	2076

A.8.13.13.2	Test Requirements	2080

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2081

A.8.13.14.1	Test Purpose and Environment	2081

A.8.13.14.2	Test Requirements	2085

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2086

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2086

A.8.14.1.1	Test Purpose and Environment	2086

A.8.14.1.2	Test Requirements	2088

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2088

A.8.14.2.1	Test Purpose and Environment	2088

A.8.14.2.2	Test Requirements	2091

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2091

A.8.14.3.1	Test Purpose and Environment	2091

A.8.14.3.2	Test Requirements	2093

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2094

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2094

A.8.15.1.1	Test Purpose and Environment	2094

A.8.15.1.2	Test Requirements	2095

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2095

A.8.15.2.1	Test Purpose and Environment	2095

A.8.15.2.2	Test Requirements	2098

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2098

A.8.15.3.1	Test Purpose and Environment	2098

A.8.15.3.2	Test Requirements	2100

A.8.16	E-UTRAN Carrier Aggregation Measurements	2101

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2101

A.8.16.1.1	Test Purpose and Environment	2101

A.8.16.1.2	Test Requirements	2103

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2103

A.8.16.2.1	Test Purpose and Environment	2103

A.8.16.2.2	Test Requirements	2105

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2105

A.8.16.3.1	Test Purpose and Environment	2105

A.8.16.3.2	Test Requirements	2107

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2108

A.8.16.3A.1	Test Purpose and Environment	2108

A.8.16.3A.2	Test Requirements	2109

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2110

A.8.16.4.1	Test Purpose and Environment	2110

A.8.16.4.2	Test Requirements	2111

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2112

A.8.16.4A.1	Test Purpose and Environment	2112

A.8.16.4A.2	Test Requirements	2114

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2115

A.8.16.5.1	Test Purpose and Environment	2115

A.8.16.5.2	Test Requirements	2115

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2115

A.8.16.6.1	Test Purpose and Environment	2115

A.8.16.6.2	Test Requirements	2116

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2116

A.8.16.7.1	 Test Purpose and Environment	2116

A.8.16.7.2	 Test Requirements	2117

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2117

A.8.16.8.1	 Test Purpose and Environment	2117

A.8.16.8.2	 Test Requirements	2118

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2118

A.8.16.9.1	Test Purpose and Environment	2118

A.8.16.9.2	Test Requirements	2119

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2119

A.8.16.10.1	Test Purpose and Environment	2119

A.8.16.10.2	Test Requirements	2120

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2120

A.8.16.11.1	 Test Purpose and Environment	2120

A.8.16.11.2	 Test Requirements	2121

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2121

A.8.16.12.1	 Test Purpose and Environment	2121

A.8.16.12.2	 Test Requirements	2121

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2121

A.8.16.13.1	Test Purpose and Environment	2121

A.8.16.13.2	Test Requirements	2122

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2122

A.8.16.14.1	Test Purpose and Environment	2122

A.8.16.14.2	Test Requirements	2123

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2123

A.8.16.15.1	 Test Purpose and Environment	2123

A.8.16.7.2	 Test Requirements	2123

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2123

A.8.16.16.1	 Test Purpose and Environment	2123

A.8.16.16.2	 Test Requirements	2124

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2124

A.8.16.17.1	Test Purpose and Environment	2124

A.8.16.17.2	Test Requirements	2126

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2127

A.8.16.17A.1	Test Purpose and Environment	2127

A.8.16.17A.2	Test Requirements	2127

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2127

A.8.16.17B.1	Test Purpose and Environment	2127

A.8.16.17B.2	Test Requirements	2128

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2128

A.8.16.17C.1	Test Purpose and Environment	2128

A.8.16.17C.2	Test Requirements	2128

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2128

A.8.16.18.1	Test Purpose and Environment	2128

A.8.16.18.2	Test Requirements	2130

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2131

A.8.16.18A.1	Test Purpose and Environment	2131

A.8.16.18A.2	Test Requirements	2131

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2131

A.8.16.18B.1	Test Purpose and Environment	2131

A.8.16.18B.2	Test Requirements	2132

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2132

A.8.16.18C.1	Test Purpose and Environment	2132

A.8.16.18C.2	Test Requirements	2132

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2132

A.8.16.18D.1	Test Purpose and Environment	2132

A.8.16.18D.2	Test Requirements	2133

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2133

A.8.16.19.1	Test Purpose and Environment	2133

A.8.16.19.2	Test Requirements	2135

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2135

A.8.16.19A.1	Test Purpose and Environment	2135

A.8.16.19A.2	Test Requirements	2136

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2136

A.8.16.19B.1	Test Purpose and Environment	2136

A.8.16.19B.2	Test Requirements	2136

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2136

A.8.16.19C.1	Test Purpose and Environment	2136

A.8.16.19C.2	Test Requirements	2137

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2137

A.8.16.20.1	Test Purpose and Environment	2137

A.8.16.20.2	Test Requirements	2139

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2140

A.8.16.20A.1	Test Purpose and Environment	2140

A.8.16.20A.2	Test Requirements	2140

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2140

A.8.16.20B.1	Test Purpose and Environment	2140

A.8.16.20B.2	Test Requirements	2141

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2141

A.8.16.20C.1	Test Purpose and Environment	2141

A.8.16.20C.2	Test Requirements	2141

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2141

A.8.16.20D.1	Test Purpose and Environment	2141

A.8.16.20D.2	Test Requirements	2142

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2142

A.8.16.21.1	Test Purpose and Environment	2142

A.8.16.21.2	Test Requirements	2144

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2144

A.8.16.22.1	Test Purpose and Environment	2144

A.8.16.22.2	Test Requirements	2146

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2146

A.8.16.23.1	Test Purpose and Environment	2146

A.8.16.23.2	Test Requirements	2149

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2149

A.8.16.24.1	Test Purpose and Environment	2149

A.8.16.24.2	Test Requirements	2152

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2152

A.8.16.25.1	Test Purpose and Environment	2152

A.8.16.25.2	Test Requirements	2154

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2154

A.8.16.26.1	Test Purpose and Environment	2154

A.8.16.26.2	Test Requirements	2157

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2157

A.8.16.27.1	Test Purpose and Environment	2157

A.8.16.27.2	Test Requirements	2161

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2161

A.8.16.28.1	Test Purpose and Environment	2161

A.8.16.28.2	Test Requirements	2165

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2165

A.8.16.29.1	Test Purpose and Environment	2165

A.8.16.29.2	Test Requirements	2169

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2169

A.8.16.30.1	Test Purpose and Environment	2169

A.8.16.30.2	Test Requirements	2173

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2173

A.8.16.31.1	Test Purpose and Environment	2173

A.8.16.31.2	Test Requirements	2177

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2177

A.8.16.32.1	Test Purpose and Environment	2177

A.8.16.32.2	Test Requirements	2181

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2181

A.8.16.33.1	Test Purpose and Environment	2181

A.8.16.33.2	Test Requirements	2185

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2185

A.8.16.34.1	Test Purpose and Environment	2185

A.8.16.34.2	Test Requirements	2189

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2189

A.8.16.35.1	Test Purpose and Environment	2189

A.8.16.35.2	Test Requirements	2191

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2192

A.8.16.36.1	Test Purpose and Environment	2192

A.8.16.36.2	Test Requirements	2194

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2195

A.8.16.37.1	Test Purpose and Environment	2195

A.8.16.37.2	Test Requirements	2197

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2198

A.8.16.38.1	Test Purpose and Environment	2198

A.8.16.38.2	Test Requirements	2200

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2201

A.8.16.39.1	Test Purpose and Environment	2201

A.8.16.39.2	Test Requirements	2204

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2204

A.8.16.40.1	Test Purpose and Environment	2204

A.8.16.40.2	Test Requirements	2207

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2207

A.8.16.41.1	Test Purpose and Environment	2207

A.8.16.41.2	Test Requirements	2209

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2210

A.8.16.42.1	Test Purpose and Environment	2210

A.8.16.42.2	Test Requirements	2212

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2213

A.8.16.43.1	Test Purpose and Environment	2213

A.8.16.43.2	Test Requirements	2216

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2216

A.8.16.44.1	Test Purpose and Environment	2216

A.8.16.44.2	Test Requirements	2219

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2219

A.8.16.45.1	Test Purpose and Environment	2219

A.8.16.45.2	Test Requirements	2222

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2222

A.8.16.46.1	Test Purpose and Environment	2222

A.8.16.46.2	Test Requirements	2224

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2225

A.8.16.47.1	Test Purpose and Environment	2225

A.8.16.47.2	Test Requirements	2227

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2228

A.8.16.48.1	Test Purpose and Environment	2228

A.8.16.48.2	Test Requirements	2230

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2231

A.8.16.49.1	Test Purpose and Environment	2231

A.8.16.49.2	Test Requirements	2234

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2234

A.8.16.50.1	Test Purpose and Environment	2234

A.8.16.50.2	Test Requirements	2236

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2237

A.8.16.51.1	Test Purpose and Environment	2237

A.8.16.51.2	Test Requirements	2241

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2241

A.8.16.52.1	Test Purpose and Environment	2241

A.8.16.52.2	Test Requirements	2245

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2245

A.8.16.53.1	Test Purpose and Environment	2245

A.8.16.53.2	Test Requirements	2249

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2249

A.8.16.54.1	Test Purpose and Environment	2249

A.8.16.54.2	Test Requirements	2253

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2253

A.8.16.55.1	Test Purpose and Environment	2253

A.8.16.55.2	Test Requirements	2258

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2258

A.8.16.56.1	Test Purpose and Environment	2258

A.8.16.56.2	Test Requirements	2263

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2263

A.8.16.57.1	Test Purpose and Environment	2263

A.8.16.57.2	Test Requirements	2265

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2266

A.8.16.58.1	Test Purpose and Environment	2266

A.8.16.58.2	Test Requirements	2268

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2269

A.8.16.59.1	Test Purpose and Environment	2269

A.8.16.59.2	Test Requirements	2272

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2273

A.8.16.60.1	Test Purpose and Environment	2273

A.8.16.60.2	Test Requirements	2276

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2277

A.8.16.61.1	Test Purpose and Environment	2277

A.8.16.61.2	Test Requirements	2280

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2280

A.8.16.62.1	Test Purpose and Environment	2280

A.8.16.62.2	Test Requirements	2282

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2283

A.8.16.63.1	Test Purpose and Environment	2283

A.8.16.63.2	Test Requirements	2286

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2287

A.8.16.64.1	Test Purpose and Environment	2287

A.8.16.64.2	Test Requirements	2291

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2291

A.8.16.65.1	Test Purpose and Environment	2291

A.8.16.65.2	Test Requirements	2296

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2296

A.8.16.66.1	Test Purpose and Environment	2296

A.8.16.66.2	Test Requirements	2301

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2301

A.8.16.67.1	Test Purpose and Environment	2301

A.8.16.67.2	Test Requirements	2306

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2306

A.8.16.68.1	Test Purpose and Environment	2306

A.8.16.68.2	Test Requirements	2311

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2311

A.8.16.69.1	Test Purpose and Environment	2311

A.8.16.69.2	Test Requirements	2314

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2314

A.8.16.70.1	Test Purpose and Environment	2314

A.8.16.70.2	Test Requirements	2317

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2317

A.8.16.71.1	Test Purpose and Environment	2317

A.8.16.71.2	Test Requirements	2321

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2321

A.8.16.72.1	Test Purpose and Environment	2321

A.8.16.72.2	Test Requirements	2325

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2325

A.8.16.73.1	Test Purpose and Environment	2325

A.8.16.73.2	Test Requirements	2330

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2330

A.8.16.74.1	Test Purpose and Environment	2330

A.8.16.74.2	Test Requirements	2335

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2335

A.8.16.75.1	Test Purpose and Environment	2335

A.8.16.75.2	Test Requirements	2340

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2340

A.8.16.76.1	Test Purpose and Environment	2340

A.8.16.76.2	Test Requirements	2345

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2345

A.8.16.77.1	Test Purpose and Environment	2345

A.8.16.77.2	Test Requirements	2348

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2348

A.8.16.78.1	Test Purpose and Environment	2348

A.8.16.78.2	Test Requirements	2351

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2352

A.8.16.79.1	Test Purpose and Environment	2352

A.8.16.79.2	Test Requirements	2357

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2357

A.8.16.80.1	Test Purpose and Environment	2357

A.8.16.80.2	Test Requirements	2362

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2362

A.8.16.81.1	Test Purpose and Environment	2362

A.8.16.81.2	Test Requirements	2367

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2367

A.8.16.82.1	Test Purpose and Environment	2367

A.8.16.82.2	Test Requirements	2372

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2372

A.8.16.83.1	Test Purpose and Environment	2372

A.8.16.83.2	Test Requirements	2377

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2377

A.8.16.84.1	Test Purpose and Environment	2377

A.8.16.84.2	Test Requirements	2383

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2383

A.8.16.85.1	Test Purpose and Environment	2383

A.8.16.85.2	Test Requirements	2386

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2386

A.8.16.86.1	Test Purpose and Environment	2386

A.8.16.86.2	Test Requirements	2389

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2389

A.8.16.87.1	Test Purpose and Environment	2389

A.8.16.87.2	Test Requirements	2394

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2394

A.8.16.88.1	Test Purpose and Environment	2394

A.8.16.88.2	Test Requirements	2402

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2402

A.8.16.89.1	Test Purpose and Environment	2402

A.8.16.89.2	Test Requirements	2407

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2407

A.8.16.90.1	Test Purpose and Environment	2407

A.8.16.90.2	Test Requirements	2412

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2412

A.8.16.91.1	Test Purpose and Environment	2412

A.8.16.91.2	Test Requirements	2418

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2418

A.8.16.92.1	Test Purpose and Environment	2418

A.8.16.92.2	Test Requirements	2426

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2426

A.8.16.93.1	Test Purpose and Environment	2426

A.8.16.93.2	Test Requirements	2431

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2431

A.8.16.94.1	Test Purpose and Environment	2431

A.8.16.94.2	Test Requirements	2436

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2436

A.8.16.95.1	Test Purpose and Environment	2436

A.8.16.95.2	Test Requirements	2444

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2444

A.8.16.96.1	Test Purpose and Environment	2444

A.8.16.96.2	Test Requirements	2452

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2452

A.8.16.97.1	Test Purpose and Environment	2452

A.8.16.97.2	Test Requirements	2457

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2457

A.8.16.98.1	Test Purpose and Environment	2457

A.8.16.98.2	Test Requirements	2463

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2463

A.8.16.99.1	Test Purpose and Environment	2463

A.8.16.99.2	Test Requirements	2470

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2470

A.8.16.100.1	Test Purpose and Environment	2470

A.8.16.100.2	Test Requirements	2478

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2478

A.8.16.101.1	Test Purpose and Environment	2478

A.8.16.101.2	Test Requirements	2485

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2485

A.8.16.102.1	Test Purpose and Environment	2485

A.8.16.102.2	Test Requirements	2493

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2493

A.8.16.103.1	Test Purpose and Environment	2493

A.8.16.103.2	Test Requirements	2497

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2498

A.8.16.104.1	Test Purpose and Environment	2498

A.8.16.104.2	Test Requirements	2501

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2502

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2502

A.8.17.1.1	Test Purpose and Environment	2502

A.8.17.1.2	Test Requirements	2507

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2507

A.8.17.2.1	Test Purpose and Environment	2507

A.8.17.2.2	Test Requirements	2513

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2513

A.8.17.3.1	Test Purpose and Environment	2513

A.8.17.3.2	Test Requirements	2514

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2514

A.8.17.4.1	Test Purpose and Environment	2514

A.8.17.4.2	Test Requirements	2515

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2515

A.8.17.5.1	Test Purpose and Environment	2515

A.8.17.5.2	Test Requirements	2516

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2516

A.8.17.6.1	Test Purpose and Environment	2516

A.8.17.6.2	Test Requirements	2517

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2518

A.8.17.7.1	Test Purpose and Environment	2518

A.8.17.7.2	Test Requirements	2518

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2519

A.8.17.8.1	Test Purpose and Environment	2519

A.8.17.8.2	Test Requirements	2519

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2520

A.8.17.9.1	Test Purpose and Environment	2520

A.8.17.9.2	Test Requirements	2521

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2521

A.8.17.10.1	Test Purpose and Environment	2521

A.8.17.10.2	Test Requirements	2526

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2527

A.8.17.11.1	Test Purpose and Environment	2527

A.8.17.11.2	Test Requirements	2533

A.8.18	E-UTRAN TDD – HRPD Measurements	2534

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2534

A.8.18.1.1	Test Purpose and Environment	2534

A.8.18.1.2	Test Requirements	2536

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2536

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2536

A.8.19.1.1	Test Purpose and Environment	2536

A.8.19.1.2	Test Requirements	2537

A.8.20	Inter-frequency/RAT Measurements in CA mode	2538

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2538

A.8.20.1.1	Test Purpose and Environment	2538

A.8.20.1.2	Test Requirements	2539

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2539

A.8.20.2.1	Test Purpose and Environment	2540

A.8.20.2.2	Test Requirements	2541

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2542

A.8.20.2A.1	Test Purpose and Environment	2542

A.8.20.2A.2	Test Requirements	2542

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2542

A.8.20.2B.1	Test Purpose and Environment	2542

A.8.20.2B.2	Test Requirements	2545

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2545

A.8.20.3.1	Test Purpose and Environment	2545

A.8.20.3.2	Test Requirements	2547

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2547

A.8.20.4.1	Test Purpose and Environment	2547

A.8.20.4.1.1	1.28 Mcps TDD option	2547

A.8.20.4.2	Test Requirements	2549

A.8.20.4.2.1	1.28 Mcps TDD option	2549

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2549

A.8.20.4A.1	Test Purpose and Environment	2549

A.8.20.4A.1.1	1.28 Mcps TDD option	2549

A.8.20.4A.2	Test Requirements	2549

A.8.20.4A.2.1	1.28 Mcps TDD option	2549

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2550

A.8.20.4B.1	Test Purpose and Environment	2550

A.8.20.4B.1.1	1.28 Mcps TDD option	2550

A.8.20.4B.2	Test Requirements	2552

A.8.20.4B.2.1	1.28 Mcps TDD option	2552

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2552

A.8.21.1	Test Purpose and Environment	2552

A.8.21.2 Test Requirements	2556

A.8.22	E-UTRAN Discovery Signal Measurements	2556

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2556

A.8.22.1.1	Test Purpose and Environment	2556

A.8.22.1.2	Test Requirements	2559

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2559

A.8.22.2.1	Test Purpose and Environment	2559

A.8.22.2.2	Test Requirements	2562

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2562

A.8.22.3.1	Test Purpose and Environment	2562

A.8.22.3.2	Test Requirements	2565

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2565

A.8.22.4.1	Test Purpose and Environment	2565

A.8.22.4.2	Test Requirements	2568

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2568

A.8.22.5.1	Test Purpose and Environment	2568

A.8.22.5.2	Test Requirements	2571

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2571

A.8.22.6.1	Test Purpose and Environment	2571

A.8.22.6.2	Test Requirements	2574

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2574

A.8.22.7.1	Test Purpose and Environment	2574

A.8.22.7.2	Test Requirements	2577

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2577

A.8.22.8.1	Test Purpose and Environment	2577

A.8.22.8.2	Test Requirements	2580

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2580

A.8.22.9.1	Test Purpose and Environment	2580

A.8.22.9.2	Test Requirements	2582

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2583

A.8.22.10.1	Test Purpose and Environment	2583

A.8.22.10.2	Test Requirements	2585

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2586

A.8.22.11.1	Test Purpose and Environment	2586

A.8.22.11.2	Test Requirements	2588

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2588

A.8.22.12.1	Test Purpose and Environment	2588

A.8.22.12.2	Test Requirements	2591

A.8.23	E-UTRAN Dual Connectivity Measurements	2591

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2591

A.8.23.1.1	Test Purpose and Environment	2591

A.8.23.1.2	Test Requirements	2594

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2594

A.8.23.2.1	Test Purpose and Environment	2594

A.8.23.2.2	Test Requirements	2597

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2597

A.8.23.3.1	Test Purpose and Environment	2597

A.8.23.3.2	Test Requirements	2600

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2600

A.8.23.4.1	Test Purpose and Environment	2600

A.8.23.4.2	Test Requirements	2603

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2603

A.8.23.5.1	Test Purpose and Environment	2603

A.8.23.5.2	Test Requirements	2606

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2606

A.8.23.6.1	Test Purpose and Environment	2606

A.8.23.6.2	Test Requirements	2609

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2609

A.8.23.7.1	Test Purpose and Environment	2609

A.8.23.7.2	Test Requirements	2611

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2612

A.8.23.8.1	Test Purpose and Environment	2612

A.8.23.8.2	Test Requirements	2614

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2615

A.8.23.9.1	Test Purpose and Environment	2615

A.8.23.9.2	Test Requirements	2618

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2618

A.8.23.10.1	Test Purpose and Environment	2618

A.8.23.10.2	Test Requirements	2621

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2621

A.8.23.11.1	Test Purpose and Environment	2621

A.8.23.11.2	Test Requirements	2624

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2624

A.8.23.12.1	Test Purpose and Environment	2624

A.8.23.12.2	Test Requirements	2627

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2627

A.8.23.13.1	Test Purpose and Environment	2627

A.8.23.13.2	Test Requirements	2630

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2630

A.8.23.14.1	Test Purpose and Environment	2630

A.8.23.14.2	Test Requirements	2633

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2633

A.8.23.15.1	Test Purpose and Environment	2633

A.8.23.15.2	Test Requirements	2636

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2636

A.8.23.16.1	Test Purpose and Environment	2636

A.8.23.16.2	Test Requirements	2637

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2638

A.8.23.17.1	Test Purpose and Environment	2638

A.8.23.17.2	Test Requirements	2640

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2640

A.8.23.18.1	Test Purpose and Environment	2640

A.8.23.18.2	Test Requirements	2641

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2642

A.8.23.19.1	Test Purpose and Environment	2642

A.8.23.19.2	Test Requirements	2643

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2644

A.8.23.20.1	Test Purpose and Environment	2644

A.8.23.20.2	Test Requirements	2645

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2646

A.8.23.21.1	Test Purpose and Environment	2646

A.8.23.21.2	Test Requirements	2647

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2648

A.8.23.22.1	Test Purpose and Environment	2648

A.8.23.22.2	Test Requirements	2649

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2650

A.8.23.23.1	Test Purpose and Environment	2650

A.8.23.23.2	Test Requirements	2651

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2652

A.8.23.24.1	Test Purpose and Environment	2652

A.8.23.24.2	Test Requirements	2654

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2655

A.8.23.25.1	Test Purpose and Environment	2655

A.8.23.25.2	Test Requirements	2657

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2658

A.8.23.26.1	Test Purpose and Environment	2658

A.8.23.26.2	Test Requirements	2660

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2661

A.8.23.27.1	Test Purpose and Environment	2661

A.8.23.27.2	Test Requirements	2664

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2664

A.8.23.28.1	Test Purpose and Environment	2664

A.8.23.28.2	Test Requirements	2667

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2667

A.8.23.29.1	Test Purpose and Environment	2667

A.8.23.29.2	Test Requirements	2670

A.8.24	Proximity-based Services	2670

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2670

A.8.24.1.1	Test Purpose and Environment	2670

A.8.24.1.2	Test Requirements	2671

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2672

A.8.24.2.1	Test Purpose and Environment	2672

A.8.24.2.2	Test Requirements	2673

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2673

A.8.24.3.1	Test Purpose and Environment	2674

A.8.24.3.2	Test Requirements	2675

A.8.25	E-UTRAN-WLAN Measurements	2676

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2676

A.8.25.1.1	Test Purpose and Environment	2676

A.8.25.1.2	Test Requirements	2678

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2678

A.8.25.2.1	Test Purpose and Environment	2678

A.8.25.2.2	Test Requirements	2680

A.8.26 Frame Structure 3 (FS3)	2680

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2680

A.8.26.1.1	Test Purpose and Environment	2680

A.8.26.1.2	Test Requirements	2682

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2683

A.8.26.2.1	Test Purpose and Environment	2683

A.8.26.2.2	Test Requirements	2685

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2686

A.8.26.3.1	Test Purpose and Environment	2686

A.8.26.3.2	Test Requirements	2689

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2689

A.8.26.3A.1	Test Purpose and Environment	2689

A.8.26.3A.2	Test Requirements	2692

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2692

A.8.26.4.1	Test Purpose and Environment	2692

A.8.26.4.2	Test Requirements	2695

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2695

A.8.26.4A.1	Test Purpose and Environment	2695

A.8.26.4A.2	Test Requirements	2698

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2698

A.8.26.5.1	Test Purpose and Environment	2698

A.8.26.5.2	Test Requirements	2701

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2701

A.8.26.5A.1	Test Purpose and Environment	2701

A.8.26.5A.2	Test Requirements	2704

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2704

A.8.26.6.1	Test Purpose and Environment	2704

A.8.26.6.2	Test Requirements	2707

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2707

A.8.26.6A.1	Test Purpose and Environment	2707

A.8.26.6A.2	Test Requirements	2710

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2710

A.8.26.7.1	Test Purpose and Environment	2710

A.8.26.7.2	Test Requirements	2713

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2713

A.8.26.8.1	Test Purpose and Environment	2713

A.8.26.8.2	Test Requirements	2716

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2716

A.8.26.9.1	Test Purpose and Environment	2716

A.8.26.9.2	Test Requirements	2719

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2719

A.8.26.10.1	Test Purpose and Environment	2719

A.8.26.10.2	Test Requirements	2721

A.9	Measurement Performance Requirements	2722

A.9.1	RSRP	2722

A.9.1.1	FDD Intra frequency case	2722

A.9.1.1.1	Test Purpose and Environment	2722

A.9.1.1.2	Test parameters	2722

A.9.1.1.3	Test Requirements	2725

A.9.1.2	TDD Intra frequency case	2725

A.9.1.2.1	Test Purpose and Environment	2725

A.9.1.2.2	Test parameters	2725

A.9.1.2.3	Test Requirements	2726

A.9.1.3	FDD—FDD Inter frequency case	2727

A.9.1.3.1	Test Purpose and Environment	2727

A.9.1.3.2	Test parameters	2727

A.9.1.3.3	Test Requirements	2730

A.9.1.4	TDD—TDD Inter frequency case	2730

A.9.1.4.1	Test Purpose and Environment	2730

A.9.1.4.2	Test parameters	2730

A.9.1.4.3	Test Requirements	2732

A.9.1.5	FDD—TDD Inter frequency case	2733

A.9.1.5.1	Test Purpose and Environment	2733

A.9.1.5.2	Test parameters	2733

A.9.1.5.3	Test Requirements	2734

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2735

A.9.1.6.1	Test Purpose and Environment	2735

A.9.1.6.2	Test parameters	2735

A.9.1.6.3	Test Requirements	2738

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2738

A.9.1.7.1	Test Purpose and Environment	2738

A.9.1.7.2	Test parameters	2738

A.9.1.7.3	Test Requirements	2741

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2741

A.9.1.8.1	Test Purpose and Environment	2741

A.9.1.8.2	Test parameters	2741

A.9.1.8.3	Test Requirements	2745

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2745

A.9.1.9.1	Test Purpose and Environment	2745

A.9.1.9.2	Test parameters	2745

A.9.1.9.3	Test Requirements	2747

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2748

A.9.1.10.1	Test Purpose and Environment	2748

A.9.1.10.2	Test parameters	2748

A.9.1.10.3	Test Requirements	2751

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2751

A.9.1.11.1	Test Purpose and Environment	2751

A.9.1.11.2	Test parameters	2751

A.9.1.11.3	Test Requirements	2755

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2755

A.9.1.12.1	Test Purpose and Environment	2755

A.9.1.12.2	Test parameters	2755

A.9.1.12.3	Test Requirements	2756

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2756

A.9.1.13.1	Test Purpose and Environment	2756

A.9.1.13.2	Test parameters	2756

A.9.1.13.3	Test Requirements	2757

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2757

A.9.1.14.1	Test Purpose and Environment	2757

A.9.1.14.2	Test parameters	2757

A.9.1.14.3	Test Requirements	2761

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2761

A.9.1.15.1	Test Purpose and Environment	2761

A.9.1.15.2	Test parameters	2761

A.9.1.15.3	Test Requirements	2763

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2764

A.9.1.16.1	Test Purpose and Environment	2764

A.9.1.16.2	Test parameters	2764

A.9.1.16.3	Test Requirements	2765

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2765

A.9.1.17.1	Test Purpose and Environment	2765

A.9.1.17.2	Test parameters	2766

A.9.1.17.3	Test Requirements	2767

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2767

A.9.1.18.1	Test Purpose and Environment	2767

A.9.1.18.2	Test parameters	2767

A.9.1.18.3	Test Requirements	2768

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2769

A.9.1.19.1	Test Purpose and Environment	2769

A.9.1.19.2	Test parameters	2769

A.9.1.19.3	Test Requirements	2769

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2769

A.9.1.20.1	Test Purpose and Environment	2769

A.9.1.20.2	Test parameters	2769

A.9.1.20.3	Test Requirements	2771

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2771

A.9.1.21.1	Test Purpose and Environment	2771

A.9.1.21.2	Test parameters	2771

A.9.1.21.3	Test Requirements	2771

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2771

A.9.1.22.1	Test Purpose and Environment	2771

A.9.1.22.2	Test parameters	2772

A.9.1.22.3	Test Requirements	2776

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2776

A.9.1.23.1	Test Purpose and Environment	2776

A.9.1.23.2	Test parameters	2776

A.9.1.23.3	Test Requirements	2780

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2780

A.9.1.24.1	Test Purpose and Environment	2780

A.9.1.24.2	Test parameters	2780

A.9.1.24.3	Test Requirements	2781

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2781

A.9.1.25.1	Test Purpose and Environment	2781

A.9.1.25.2	Test parameters	2781

A.9.1.25.3	Test Requirements	2784

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2784

A.9.1.26.1	Test Purpose and Environment	2784

A.9.1.26.2	Test parameters	2784

A.9.1.26.3	Test Requirements	2787

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2787

A.9.1.27.1	Test Purpose and Environment	2787

A.9.1.27.2	Test parameters	2787

A.9.1.27.3	Test Requirements	2790

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2790

A.9.1.28.1	Test Purpose and Environment	2790

A.9.1.28.2	Test parameters	2790

A.9.1.28.3	Test Requirements	2793

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2793

A.9.1.29.1	Test Purpose and Environment	2793

A.9.1.29.2	Test parameters	2793

A.9.1.29.3	Test Requirements	2796

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2796

A.9.1.30.1	Test Purpose and Environment	2796

A.9.1.30.2	Test parameters	2797

A.9.1.30.3	Test Requirements	2800

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2800

A.9.1.31.1	Test Purpose and Environment	2800

A.9.1.31.2	Test parameters	2800

A.9.1.31.3	Test Requirements	2803

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2803

A.9.1.32.1	Test Purpose and Environment	2803

A.9.1.32.2	Test parameters	2804

A.9.1.32.3	Test Requirements	2807

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2807

A.9.1.33.1	Test Purpose and Environment	2807

A.9.1.33.2	Test parameters	2807

A.9.1.33.3	Test Requirements	2810

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2810

A.9.1.34.1	Test Purpose and Environment	2810

A.9.1.34.2	Test parameters	2810

A.9.1.34.3	Test Requirements	2813

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2813

A.9.1.35.1	Test Purpose and Environment	2813

A.9.1.35.2	Test parameters	2813

A.9.1.35.3	Test Requirements	2816

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2817

A.9.1.36.1	Test Purpose and Environment	2817

A.9.1.36.2	Test parameters	2817

A.9.1.36.3	Test Requirements	2820

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2820

A.9.1.37.1	Test Purpose and Environment	2820

A.9.1.37.2	Test parameters	2820

A.9.1.37.3	Test Requirements	2823

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2824

A.9.1.38.1	Test Purpose and Environment	2824

A.9.1.38.2	Test parameters	2824

A.9.1.38.3	Test Requirements	2827

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2828

A.9.1.39.1	Test Purpose and Environment	2828

A.9.1.39.2	Test parameters	2828

A.9.1.39.3	Test Requirements	2834

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2834

A.9.1.40.1	Test Purpose and Environment	2834

A.9.1.40.2	Test parameters	2835

A.9.1.40.3	Test Requirements	2841

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2841

A.9.1.41.1	Test Purpose and Environment	2841

A.9.1.41.2	Test parameters	2841

A.9.1.41.3	Test Requirements	2844

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2844

A.9.1.42.1	Test Purpose and Environment	2844

A.9.1.42.2	Test parameters	2844

A.9.1.42.3	Test Requirements	2847

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2847

A.9.1.43.1	Test Purpose and Environment	2847

A.9.1.43.2	Test parameters	2847

A.9.1.43.3	Test Requirements	2848

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2849

A.9.1.44.1	Test Purpose and Environment	2849

A.9.1.44.2	Test parameters	2849

A.9.1.44.3	Test Requirements	2854

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2854

A.9.1.45.1	Test Purpose and Environment	2854

A.9.1.45.2	Test parameters	2854

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2599

A.9.2.6.3	Test Requirements	2602

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2602

A.9.2.7.1	Test Purpose and Environment	2602

A.9.2.7.2	Test parameters	2602

A.9.2.7.3	Test Requirements	2606

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2606

A.9.2.8.1	Test Purpose and Environment	2606

A.9.2.8.2	Test parameters	2606

A.9.2.8.3	Test Requirements	2609

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2609

A.9.2.9.1	Test Purpose and Environment	2609

A.9.2.9.2	Test parameters	2609

A.9.2.9.3	Test Requirements	2613

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2613

A.9.2.10.1	Test Purpose and Environment	2613

A.9.2.10.2	Test parameters	2613

A.9.2.10.3	Test Requirements	2617

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2617

A.9.2.11.1	Test Purpose and Environment	2617

A.9.2.11.2	Test parameters	2617

A.9.2.11.3	Test Requirements	2618

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.12.1	Test Purpose and Environment	2618

A.9.2.12.2	Test parameters	2619

A.9.2.12.3	Test Requirements	2619

A.9.2.13	Void	2620

A.9.2.13.1	Void	2620

A.9.2.13.2	Void	2620

A.9.2.13.3	Void	2620

A.9.2.14	Void	2620

A.9.2.14.1	Void	2620

A.9.2.14.2	Void	2620

A.9.2.14.3	Void	2620

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2620

A.9.2.15.1	Test Purpose and Environment	2620

A.9.2.15.2	Test parameters	2620

A.9.2.15.3	Test Requirements	2624

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2624

A.9.2.16.1	Test Purpose and Environment	2624

A.9.2.16.2	Test parameters	2624

A.9.2.16.3	Test Requirements	2629

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2629

A.9.2.17.1	Test Purpose and Environment	2629

A.9.2.17.2	Test parameters	2629

A.9.2.17.3	Test Requirements	2631

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2631

A.9.2.18.1	Test Purpose and Environment	2631

A.9.2.18.2	Test parameters	2631

A.9.2.18.3	Test Requirements	2634

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2634

A.9.2.19.1	Test Purpose and Environment	2634

A.9.2.19.2	Test parameters	2634

A.9.2.19.3	Test Requirements	2636

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2636

A.9.2.20.1	Test Purpose and Environment	2636

A.9.2.20.2	Test parameters	2636

A.9.2.20.3	Test Requirements	2639

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2639

A.9.2.21.1	Test Purpose and Environment	2639

A.9.2.21.2	Test parameters	2639

A.9.2.21.3	Test Requirements	2642

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2642

A.9.2.22.1	Test Purpose and Environment	2643

A.9.2.22.2	Test parameters	2643

A.9.2.22.3	Test Requirements	2643

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2643

A.9.2.23.1	Test Purpose and Environment	2643

A.9.2.23.2	Test parameters	2643

A.9.2.23.3	Test Requirements	2646

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2646

A.9.2.24.1	Test Purpose and Environment	2646

A.9.2.24.2	Test parameters	2646

A.9.2.24.3	Test Requirements	2647

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2647

A.9.2.25.1	Test Purpose and Environment	2647

A.9.2.25.2	Test parameters	2647

A.9.2.25.3	Test Requirements	2652

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2652

A.9.2.26.1	Test Purpose and Environment	2652

A.9.2.26.2	Test parameters	2652

A.9.2.26.3	Test Requirements	2656

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2656

A.9.2.27.1	Test Purpose and Environment	2656

A.9.2.27.2	Test parameters	2656

A.9.2.27.3	Test Requirements	2657

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2657

A.9.2.28.1	Test Purpose and Environment	2657

A.9.2.28.2	Test parameters	2657

A.9.2.28.3	Test Requirements	2660

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2660

A.9.2.29.1	Test Purpose and Environment	2660

A.9.2.29.2	Test parameters	2660

A.9.2.29.3	Test Requirements	2663

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2663

A.9.2.30.1	Test Purpose and Environment	2663

A.9.2.30.2	Test parameters	2663

A.9.2.30.3	Test Requirements	2666

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2666

A.9.2.31.1	Test Purpose and Environment	2666

A.9.2.31.2	Test parameters	2666

A.9.2.31.3	Test Requirements	2669

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2669

A.9.2.32.1	Test Purpose and Environment	2669

A.9.2.32.2	Test parameters	2669

A.9.2.32.3	Test Requirements	2672

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2672

A.9.2.33.1	Test Purpose and Environment	2672

A.9.2.33.2	Test parameters	2672

A.9.2.33.3	Test Requirements	2676

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2676

A.9.2.34.1	Test Purpose and Environment	2676

A.9.2.34.2	Test parameters	2676

A.9.2.34.3	Test Requirements	2679

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2679

A.9.2.35.1	Test Purpose and Environment	2679

A.9.2.35.2	Test parameters	2679

A.9.2.35.3	Test Requirements	2682

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2682

A.9.2.36.1	Test Purpose and Environment	2682

A.9.2.36.2	Test parameters	2682

A.9.2.36.3	Test Requirements	2683

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2684

A.9.2.37.1	Test Purpose and Environment	2684

A.9.2.37.2	Test parameters	2684

A.9.2.37.3	Test Requirements	2685

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2686

A.9.2.38.1	Test Purpose and Environment	2686

A.9.2.38.2	Test parameters	2686

A.9.2.38.3	Test Requirements	2690

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2690

A.9.2.39.1	Test Purpose and Environment	2690

A.9.2.39.2	Test parameters	2690

A.9.2.39.3	Test Requirements	2694

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2694

A.9.2.40.1	Test Purpose and Environment	2694

A.9.2.40.2	Test parameters	2694

A.9.2.40.3	Test Requirements	2697

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2698

A.9.2.41.1	Test Purpose and Environment	2698

A.9.2.41.2	Test parameters	2698

A.9.2.41.3	Test Requirements	2701

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2701

A.9.2.42.1	Test Purpose and Environment	2701

A.9.2.42.2	Test parameters	2701

A.9.2.42.3	Test Requirements	2704

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2704

A.9.2.43.1	Test Purpose and Environment	2704

A.9.2.43.2	Test parameters	2704

A.9.2.43.3	Test Requirements	2707

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2707

A.9.2.44.1	Test Purpose and Environment	2707

A.9.2.44.2	Test parameters	2707

A.9.2.44.3	Test Requirements	2709

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.45.1	Test Purpose and Environment	2709

A.9.2.45.2	Test parameters	2709

A.9.2.45.3	Test Requirements	2713

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2714

A.9.2.46.1	Test Purpose and Environment	2714

A.9.2.46.2	Test parameters	2714

A.9.2.46.3	Test Requirements	2718

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2719

A.9.2.47.1	Test Purpose and Environment	2719

A.9.2.47.2	Test parameters	2719

A.9.2.47.3	Test Requirements	2723

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.48.1	Test Purpose and Environment	2723

A.9.2.48.2	Test parameters	2724

A.9.2.48.3	Test Requirements	2728

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.49.1	Test Purpose and Environment	2729

A.9.2.49.2	Test parameters	2729

A.9.2.49.3	Test Requirements	2735

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.50.1	Test Purpose and Environment	2736

A.9.2.50.2	Test parameters	2736

A.9.2.50.3	Test Requirements	2742

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2742

A.9.2.51.1	Test Purpose and Environment	2742

A.9.2.51.2	Test parameters	2743

A.9.2.51.3	Test Requirements	2747

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2747

A.9.2.52.1	Test Purpose and Environment	2747

A.9.2.52.2	Test parameters	2747

A.9.2.52.3	Test Requirements	2750

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.53.1	Test Purpose and Environment	2751

A.9.2.53.2	Test parameters	2751

A.9.2.53.3	Test Requirements	2757

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2758

A.9.2.54.1	Test Purpose and Environment	2758

A.9.2.54.2	Test parameters	2758

A.9.2.54.3	Test Requirements	2764

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2764

A.9.2.55.1	Test Purpose and Environment	2764

A.9.2.55.2	Test parameters	2765

A.9.2.55.3	Test Requirements	2770

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2770

A.9.2.56.1	Test Purpose and Environment	2770

A.9.2.56.2	Test parameters	2770

A.9.2.56.3	Test Requirements	2775

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2775

A.9.2.57.1	Test Purpose and Environment	2775

A.9.2.57.2	Test parameters	2776

A.9.2.57.3	Test Requirements	2780

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2781

A.9.2.58.1	Test Purpose and Environment	2781

A.9.2.58.2	Test parameters	2781

A.9.2.58.3	Test Requirements	2785

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2786

A.9.2.59.1	Test Purpose and Environment	2786

A.9.2.59.2	Test parameters	2786

A.9.2.59.3	Test Requirements	2791

A.9.3	UTRAN FDD CPICH RSCP	2792

A.9.3.1	E-UTRAN FDD	2792

A.9.3.1.1	Test Purpose and Environment	2792

A.9.3.1.2	Parameters	2792

A.9.3.1.3	Test Requirements	2794

A.9.3.2 E-UTRAN TDD	2794

A.9.3.2.1	Test Purpose and Environment	2794

A.9.3.2.2	Parameters	2794

A.9.3.2.3	Test Requirements	2797

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2797

A.9.3.3.1	Test Purpose and Environment	2797

A.9.3.3.2	Parameters	2797

A.9.3.3.3	Test Requirements	2798

A.9.4	UTRAN FDD CPICH Ec/No	2799

A.9.4.1	E-UTRAN FDD	2799

A.9.4.1.1	Test Purpose and Environment	2799

A.9.4.1.2	Parameters	2799

A.9.4.1.3	Test Requirements	2801

A.9.4.2	E-UTRAN TDD	2802

A.9.4.2.1	Test Purpose and Environment	2802

A.9.4.2.2	Parameters	2802

A.9.4.2.3	Test Requirements	2804

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2805

A.9.4.3.1	Test Purpose and Environment	2805

A.9.4.3.2	Parameters	2805

A.9.4.3.3	Test Requirements	2806

A.9.5	UTRAN TDD measurement	2806

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2806

A.9.5.1.1	Test Purpose and Environment	2806

A.9.5.1.2	Test parameters	2806

A.9.5.1.3	Test Requirements	2808

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2808

A.9.5.2.1	Test Purpose and Environment	2808

A.9.5.2.2	Test parameters	2808

A.9.5.2.3	Test Requirements	2810

A.9.6	GSM Carrier RSSI	2811

A.9.6.1	E-UTRAN FDD	2811

A.9.6.1.1	Test Purpose and Environment	2811

A.9.6.1.2	Test Requirements	2812

A.9.6.2	E-UTRAN TDD	2812

A.9.6.2.1	Test Purpose and Environment	2812

A.9.6.2.2	Test Requirements	2814

A.9.7	UE Rx – Tx Time Difference	2815

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2815

A.9.7.1.1	Test Purpose and Environment	2815

A.9.7.1.2	Test parameters	2815

A.9.7.1.3	Test Requirements	2816

A.9.7.2	E-UTRA TDD	2816

A.9.7.2.1	Test Purpose and Environment	2816

A.9.7.2.2	Test parameters	2816

A.9.7.2.3	Test Requirements	2818

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2818

A.9.7.3.1	Test Purpose and Environment	2818

A.9.7.3.2	Test parameters	2818

A.9.7.3.3	Test Requirements	2821

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2821

A.9.7.4.1	Test Purpose and Environment	2821

A.9.7.4.2	Test Parameters	2821

A.9.7.4.3	Test Requirements	2824

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2824

A.9.7.5.1	Test Purpose and Environment	2824

A.9.7.5.2	Test parameters	2824

A.9.7.5.3	Test Requirements	2827

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2827

A.9.7.6.1	Test Purpose and Environment	2827

A.9.7.6.2	Test Parameters	2827

A.9.7.6.3	Test Requirements	2830

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2830

A.9.7.7.1	Test Purpose and Environment	2830

A.9.7.7.2	Test parameters	2830

A.9.7.7.3	Test Requirements	2831

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.8.1	Test Purpose and Environment	2832

A.9.7.8.2	Test parameters	2832

A.9.7.8.3	Test Requirements	2833

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.9.1	Test Purpose and Environment	2833

A.9.7.9.2	Test parameters	2833

A.9.7.9.3	Test Requirements	2834

A.9.8	RSTD	2835

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2835

A.9.8.1.1	Test Purpose and Environment	2835

A.9.8.1.2	Test Requirements	2837

A.9.8.1.2A	Test Requirements for UE Category 1bis	2837

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2838

A.9.8.2.1	Test Purpose and Environment	2838

A.9.8.2.2	Test Requirements	2841

A.9.8.2.2A	Test Requirements for UE Category 1bis	2841

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2842

A.9.8.3.1	Test Purpose and Environment	2842

A.9.8.3.2	Test Requirements	2844

A.9.8.3.2A	Test Requirements for UE Category 1bis	2844

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2845

A.9.8.4.1	Test Purpose and Environment	2845

A.9.8.4.2	Test Requirements	2847

A.9.8.4.2A	Test Requirements for UE Category 1bis	2847

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2848

A.9.8.5.1	Test Purpose and Environment	2848

A.9.8.5.2	Test Requirements	2850

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.6.1	Test Purpose and Environment	2850

A.9.8.6.2	Test Requirements	2853

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2853

A.9.8.7.1	Test Purpose and Environment	2853

A.9.8.7.2	Test Requirements	2854

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.8.1	Test Purpose and Environment	2854

A.9.8.8.2	Test Requirements	2855

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2855

A.9.8.9.1	Test Purpose and Environment	2855

A.9.8.9.2	Test Requirements	2856

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.10.1	Test Purpose and Environment	2856

A.9.8.10.2	Test Requirements	2857

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2857

A.9.8.11.1	Test Purpose and Environment	2857

A.9.8.11.2	Test Requirements	2858

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2858

A.9.8.12.1	Test Purpose and Environment	2858

A.9.8.12.2	Test Requirements	2859

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2859

A.9.8.13.1	Test Purpose and Environment	2859

A.9.8.13.2	Test Requirements	2860

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2860

A.9.8.14.1	Test Purpose and Environment	2860

A.9.8.14.2	Test Requirements	2867

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2867

A.9.8.15.1	Test Purpose and Environment	2867

A.9.8.15.2	Test Requirements	2873

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2873

A.9.8.16.1	Test Purpose and Environment	2873

A.9.8.16.2	Test Requirements	2876

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2876

A.9.8.17.1	Test Purpose and Environment	2876

A.9.8.17.2	Test Requirements	2879

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2879

A.9.8.18.1	Test Purpose and Environment	2879

A.9.8.18.2	Test Requirements	2882

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2882

A.9.8.19.1	Test Purpose and Environment	2882

A.9.8.19.2	Test Requirements	2885

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2885

A.9.8.20.1	Test Purpose and Environment	2885

A.9.8.20.2	Test Requirements	2888

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2888

A.9.8.21.1	Test Purpose and Environment	2888

A.9.8.21.2	Test Requirements	2892

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2893

A.9.8.22.1	Test Purpose and Environment	2893

A.9.8.22.2	Test Requirements	2896

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2897

A.9.8.23.1	Test Purpose and Environment	2897

A.9.8.23.2	Test Requirements	2900

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2900

A.9.8.24.1	Test Purpose and Environment	2900

A.9.8.24.2	Test Requirements	2904

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2904

A.9.8.25.1	Test Purpose and Environment	2904

A.9.8.25.2	Test Requirements	2908

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2909

A.9.8.26.1	Test Purpose and Environment	2909

A.9.8.26.2	Test Requirements	2911

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.27.1	Test Purpose and Environment	2911

A.9.8.27.2	Test Requirements	2914

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2914

A.9.8.28.1	Test Purpose and Environment	2914

A.9.8.28.2	Test Requirements	2917

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2917

A.9.8.29.1	Test Purpose and Environment	2917

A.9.8.29.2	Test Requirements	2921

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2922

A.9.8.30.1	Test Purpose and Environment	2922

A.9.8.30.2	Test Requirements	2925

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2926

A.9.8.31.1	Test Purpose and Environment	2926

A.9.8.31.2	Test Requirements	2930

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2930

A.9.8.32.1	Test Purpose and Environment	2930

A.9.8.32.2	Test Requirements	2933

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2933

A.9.8.33.1	Test Purpose and Environment	2933

A.9.8.33.2	Test Requirements	2936

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2936

A.9.8.34.1	Test Purpose and Environment	2936

A.9.8.34.2	Test Requirements	2939

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2939

A.9.8.35.1	Test Purpose and Environment	2939

A.9.8.35.2	Test Requirements	2942

A.9.9	RSRP and RSRQ on the serving cell	2942

A.9.9.1	FDD Intra frequency serving cell case	2942

A.9.9.1.1	Test Purpose and Environment	2942

A.9.9.1.2	Test parameters	2942

A.9.9.1.3	Test Requirements	2945

A.9.9.2	TDD Intra frequency serving cell case	2945

A.9.9.2.1	Test Purpose and Environment	2945

A.9.9.2.2	Test parameters	2945

A.9.9.2.3	Test Requirements	2947

A.9.10	SSTD	2947

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2947

A.9.10.1.1	Test Purpose and Environment	2947

A.9.10.1.2	Test parameters	2947

A.9.10.1.3	Test Requirements	2949

A.9.10.3	FDD—FDD Inter frequency case	2949

A.9.10.3.1	Test Purpose and Environment	2949

A.9.10.3.2	Test parameters	2949

A.9.10.3.3	Test Requirements	2955

A.9.10.4	TDD—TDD Inter frequency case	2955

A.9.10.4.1	Test Purpose and Environment	2955

A.9.10.4.2	Test parameters	2955

A.9.10.4.3	Test Requirements	2961

A.9.11 RSSI	2961

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2961

A.9.11.1.1	Test Purpose and Environment	2961

A.9.11.1.2	Test parameters	2961

A.9.11.1.3	Test Requirements	2964

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2964

A.9.11.2.1	Test Purpose and Environment	2964

A.9.11.2.2	Test parameters	2964

A.9.12	Channel occupancy	2967

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2967

A.9.12.1.1	Test Purpose and Environment	2967

A.9.12.1.2	Test parameters	2967

A.9.12.1.3	Test Requirements	2970

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2970

A.9.12.2.1	Test Purpose and Environment	2970

A.9.12.2.2	Test parameters	2970

A.9.12.2.3	Test Requirements	2973

A.9.13	RS-SINR	2973

A.9.13.1	FDD Intra-Frequency Case	2973

A.9.13.1.1	Test Purpose and Environment	2973

A.9.13.1.2	Test parameters	2973

A.9.13.1.3	Test Requirements	2976

A.9.13.2	TDD Intra-Frequency Case	2976

A.9.13.2.1	Test Purpose and Environment	2976

A.9.13.2.2	Test parameters	2976

A.9.13.2.3	Test Requirements	2979

A.9.13.5	FDD—TDD Inter frequency case	2979

A.9.13.5.1	Test Purpose and Environment	2979

A.9.13.5.2	Test parameters	2979

A.9.13.5.3	Test Requirements	2985

A.9.13.6	TDD—FDD Inter frequency case	2985

A.9.13.6.1	Test Purpose and Environment	2985

A.9.13.6.2	Test parameters	2985

A.9.13.6.3	Test Requirements	2989

A.9.14	Channel quality reporting accuracy	2989

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2989

A.9.14.1.1	Test Purpose and Environment	2989

A.9.14.1.2	Test parameters	2990

A.9.14.1.3	Test Requirements	2990

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2991

A.9.14.2.1	Test Purpose and Environment	2991

A.9.14.2.2	Test parameters	2991

A.9.14.2.3	Test Requirements	2992

A.10	Proximity-based Services in Any Cell Selection State	2992

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2992

A.10.1.1	Test Purpose and Environment	2992

A.10.1.2	Test Requirements	2993

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2994

A.10.2.1	Test Purpose and Environment	2994

A.10.2.2	Test Requirements	2995

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2995

A.10.3.1	Test Purpose and Environment	2995

A.10.3.2	Test Requirements	2997

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2998

A.10.4.1	Test Purpose and Environment	2998

A.10.4.2	Test Requirements	3000

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3000

A.11.1	V2V UE Transmission Timing Accuracy Test	3000

A.11.1.1	Test Purpose and Environment	3000

A.11.1.2	Test requirements	3001

A.11.2	Interruptions due to V2V sidelink communication	3001

A.11.2.1	Test Purpose and Environment	3001

A.11.2.2	Test Requirements	3003

A.12	3003

A.12.1	V2X UE Transmission Timing Accuracy Test	3003

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3003

A.12.1.1.1	Test Purpose and Environment	3003

A.12.1.1.2	Test requirements	3004

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3005

A.12.1.2.1	Test Purpose and Environment	3005

A.12.1.2.2	Test Requirements	3005

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3006

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3006

A.12.2.1.1	Test Purpose and Environment	3006

A.12.2.1.2	Test Requirements	3007

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3008

A.12.2.2.1	Test Purpose and Environment	3008

A.12.2.2.2	Test Requirements	3009

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3009

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3009

A.12.3.1.1	Test Purpose and Environment	3009

A.12.3.1.2	Test Requirements	3012

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3013

A.12.3.2.1	Test Purpose and Environment	3013

A.12.3.1.2	Test Requirements	3015

A.12.4	Congestion Control Measurement Test for V2X UE	3015

A.12.4.1	Test Purpose and Environment	3015

A.12.4.2	Test Requirements	3017

A.12.5	Interruptions due to V2X Sidelink Communication	3017

A.12.5.1	Test Purpose and Environment	3017

A.12.5.1	Test Requirements	3019

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3019

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3019

A.12.6.1.1	Test Purpose and Environment	3019

A.12.6.1.2	Test Requirements	3021

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3021

A.12.6.2.1	Test Purpose and Environment	3021

A.12.6.1.2	Test Requirements	3023

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3023

A.12.7.1	Test Purpose and Environment	3023

A.12.7.2	Test Requirements	3026

A.12.8	Interruptions due to V2X Carrier Aggregation	3027

A.12.8.1	Interruptions on a FDD PCell	3027

A.12.8.1.1	Test Purpose and Environment	3027

A.12.8.1.2	Test Requirements	3028

A.12.8.2	Interruptions on a TDD PCell	3028

A.12.8.2.1	Test Purpose and Environment	3028

A.12.8.2.2	Test Requirements	3030

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3031

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3031

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3031

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3031

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3031

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3033

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3034

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3034

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3034

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3034

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3036

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3037

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3037

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3037

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3038

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3038

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3039

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3039

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3039

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3040

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3040

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3041

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3041

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3041

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3041

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3042

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3042

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3042

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3043

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3043

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3043

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3044

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3044

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3045

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3046

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3047

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3048

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3049

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3049

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3049

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3049

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3050

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3050

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3052

B.3	Conditions for measurements performance requirements for UE	3052

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3052

B.3.2	Void	3052

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3052

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3052

B.3.5	Conditions for UE Rx – Tx time difference	3052

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3053

B.3.7	Conditions for inter-frequency RSTD measurements	3053

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3053

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3053

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3053

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3054

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3054

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3054

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3054

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3054

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3054

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3055

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3055

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3055

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3056

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3056

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3056

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3056

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3056

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3056

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3056

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3056

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3056

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3057

B.3.21.1	Conditions for RSRP measurements	3057

B.3.21.2	Conditions for RSRQ measurements	3057

B.3.21.3	Conditions for CSI-RSRP measurements	3057

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3057

B.3.22.1	Conditions for RSRP measurements	3057

B.3.22.2	Void	3058

B.3.22.3	Conditions for CSI-RSRP measurements	3058

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3058

B.3.23.1	Conditions for RSRP measurements	3058

B.3.23.2	Conditions for RSRQ measurements	3058

B.3.23.3	Conditions for CSI-RSRP measurements	3058

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3059

B.3.24.1	Conditions for RSRP measurements	3059

B.3.24.2	Conditions for RSRQ measurements	3059

B.3.24.3	Conditions for CSI-RSRP measurements	3059

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3059

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3059

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3059

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3060

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3060

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3060

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3060

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3060

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3061

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3061

B.4	RRM Requirements Exceptions	3061

B.4.1	General	3061

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3061

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3061

B.4.3.1	Inter-band carrier aggregation	3061

B.4.3.2	Intra-band non-contiguous carrier aggregation	3061

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3062

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3062

B.5.1	Conditions for S-RSRP Accuracy Requirements	3062

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3062

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3063

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3063

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3064

B.6	Conditions for V2X	3064

B.6.1	Test parameters for GNSS signals	3064

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3064

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3064

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3065

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3065

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3065

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3065

Annex C (informative):	Change history:	3067