| 3GPP TS 36.133 V16.18.2 (2023-10) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	133

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	134

4.2.2.9	UE measurement capability	134

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	135

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	136

4.2.2.11	Void	137

4.2.2.12	Void	137

4.2.2.13	Void	137

4.3	Minimization of Drive Tests (MDT)	137

4.3.1	Introduction	138

4.3.2	Measurements	138

4.3.2.1	Requirements	138

4.3.3	Relative Time Stamp Accuracy	138

4.3.3.1	Requirements	138

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	138

4.3.4.1	 Requirements	138

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	139

4.3.5.1	Requirements for timeSinceFailure	139

4.4	MBSFN Measurements	139

4.4.1	Introduction	139

4.4.2	MBSFN RSRP measurements	139

4.4.3	MBSFN RSRQ measurements	139

4.4.4	MCH BLER measurements	140

4.5	Proximity-based Services	140

4.5.1	Introduction	140

4.5.2	Requirements	140

4.5.2.1	Interruptions with ProSe Direct Discovery	140

4.5.2.2	Interruptions with ProSe Direct Communication	140

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	140

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	141

4.6	Cell Selection and Re-selection Requirements for UE category NB1	141

4.6.1	Cell Selection	141

4.6.2	Cell Re-selection	141

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	142

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	143

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	144

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	145

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	147

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	147

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	149

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	150

4.6.2.7	Maximum interruption in paging reception in normal coverage	152

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	152

4.6.2.8	UE measurement capability	152

4.6.2.9	WUS receptions for NB1	152

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	153

4.6.3.1	Introduction	153

4.6.3.2	Requirements on UE synchronization for transmission using PUR	153

4.6.3.3	Requirements on TA validation for transmission using PUR	153

4.7	Cell Selection and Re-selection Requirements for UE category M1	154

4.7.1	Cell Selection	154

4.7.2	Cell Re-selection	154

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	154

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	155

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	156

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	158

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	159

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	160

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	160

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	160

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	161

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	162

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	165

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	166

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	167

4.7.2.3	WUS receptions for UE category M1	167

4.7.3	Channel quality report for UE Category M1 in idle mode	167

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	168

4.7.4.1	Introduction	168

4.7.4.2	Requirements on UE synchronization for transmission using PUR	168

4.7.4.3	Requirements on TA validation for transmission using PUR	169

4.8	Idle State Positioning Measurement Requirements for UE category NB1	170

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	170

4.8.1.1	RSTD Measurement Reporting Delay	171

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	172

4.8.2.1	RSTD Measurement Reporting Delay	173

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	174

4.8.3.1	RSTD Measurement Reporting Delay	175

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	176

4.8.4.1	RSTD Measurement Reporting Delay	177

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	178

4.8.5.1	Measurement Reporting Delay	179

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	179

4.8.6.1	Measurement Reporting Delay	180

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	181

4.8.7.1	Measurement Reporting Delay	182

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	182

4.8.8.1	Measurement Reporting Delay	183

4.9	Idle Mode CA Measurement	184

4.9.1	Introduction	184

4.9.2	Requirement	184

4.9.2.1	Detected cell requirement during state transition and Idle mode	184

4.9.2.2	Measurements of inter-frequency CA candidate cells	185

4.9.2.3	Measurements on serving cell	185

4A	E-UTRAN RRC_INACTIVE state mobility	186

4A.1	Cell Re-selection	186

4A.1.1	Introduction	187

4A.1.2	Requirements	187

4A.1.2.1	UE measurement capability	187

4A.1.2.2	Measurement and evaluation of serving cell	187

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	187

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	187

4A.1.2.5	Evaluation of cell re-selection criteria	187

4A.1.2.6	Maximum interruption in paging reception	187

4A.1.2.7	Measurements of inter-RAT NR cells	187

4A.2	Requirements for UE Category M1	187

4A.2.1	Introduction	187

4A.2.2	Cell Selection	187

4A.2.3	Cell Reselection	187

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	187

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	187

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	188

A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	188

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	189

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	189

4A.2.4	Channel quality report for UE Category M1 in idle mode	190

5	E-UTRAN RRC_CONNECTED state mobility	190

5.1	E-UTRAN Handover	191

5.1.1	Introduction	191

5.1.2	Requirements	191

5.1.2.1	E-UTRAN FDD – FDD	191

5.1.2.1.1	Handover delay	191

5.1.2.1.2	Interruption time	191

5.1.2.2	E-UTRAN FDD – TDD	193

5.1.2.2.1	(Void)	193

5.1.2.2.2	(Void)	193

5.1.2.3	E-UTRAN TDD – FDD	193

5.1.2.3.1	(Void)	193

5.1.2.3.2	(Void)	193

5.1.2.4	E-UTRAN TDD – TDD	193

5.1.2.4.1	Handover delay	193

5.1.2.4.2	Interruption time	193

5.1.2.5	E-UTRAN HD–FDD	195

5.1.2.5.1	Handover delay	195

5.1.2.5.2	Interruption time	195

5.1.2.6	E-UTRAN FDD – FDD conditional handover	197

5.1.2.6.1	Handover delay	197

5.1.2.6.2	Measurement time	197

5.1.2.6.3	Preparation time	197

5.1.2.6.4	Interruption time	197

5.1.2.7	E-UTRAN FDD – TDD conditional handover	198

5.1.2.8	E-UTRAN TDD – FDD conditional handover	198

5.1.2.9	E-UTRAN TDD – TDD conditional handover	198

5.2	Void	198

5.3	Handover to other RATs	198

5.3.1	E-UTRAN - UTRAN FDD Handover	198

5.3.1.1	Introduction	198

5.3.1.1.1	Handover delay	198

5.3.1.1.2	Interruption time	198

5.3.2	E-UTRAN - UTRAN TDD Handover	199

5.3.2.1	Introduction	199

5.3.2.2	Requirements	199

5.3.2.2.1	Handover delay	199

5.3.2.2.2	Interruption time	199

5.3.3	E-UTRAN - GSM Handover	200

5.3.3.1	Introduction	200

5.3.3.2	Requirements	200

5.3.3.2.1	Handover delay	200

5.3.3.2.2	Interruption time	200

5.3.4	E-UTRAN - NR FR1 Handover	201

5.3.4.1	Introduction	201

5.3.4.2	Handover delay	201

5.3.4.3	Interruption time	201

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	202

5.3.4A.1	Introduction	202

5.3.4A.2	Handover delay	202

5.3.4A.3	Interruption time	202

5.3.5	E-UTRAN - NR FR2 Handover	203

5.3.5.1	Introduction	203

5.3.5.2	Handover delay	203

5.3.5.3	Interruption time	203

5.4	Handover to Non-3GPP RATs	204

5.4.1	E-UTRAN – HRPD Handover	204

5.4.1.1	Introduction	204

5.4.1.1.1	Handover delay	204

5.4.1.1.2	Interruption time	204

5.4.2	E-UTRAN – cdma2000 1X Handover	205

5.4.2.1	Introduction	205

5.4.2.1.1	Handover delay	205

5.4.2.1.2	Interruption time	205

5.5	E-UTRAN Handover for Cat-M1 UEs	206

5.5.1	Introduction	206

5.5.2	Requirements in CEModeA	206

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	206

5.5.2.1.1	Handover delay	206

5.5.2.1.2	Interruption time	206

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	207

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	207

5.5.2.3.1	Void	207

5.5.2.3.2	Void	207

5.5.3	Requirements in CEModeB	207

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	207

5.5.3.1.1	Handover delay	207

5.5.3.1.2	Interruption time	207

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	208

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	208

5.6	Void	208

5.7	E-UTRAN DAPS Handover	208

5.7.1	Introduction	208

5.7.2	Requirements	208

5.7.2.1	E-UTRAN FDD – FDD	208

5.7.2.1.1	DAPS Handover delay	209

5.7.2.1.2	Interruption time	209

5.7.2.2	E-UTRAN FDD – TDD	210

5.7.2.3	E-UTRAN TDD – FDD	210

5.7.2.4	E-UTRAN TDD – TDD	210

6	RRC Connection Mobility Control	210

6.1	RRC Re-establishment	210

6.1.1	Introduction	210

6.1.2	Requirements	210

6.1.2.1	UE Re-establishment delay requirement	210

6.2	Random Access	211

6.2.1	Introduction	211

6.2.2	Requirements	211

6.2.2.1	Contention based random access	211

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	211

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	211

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	211

6.2.2.1.4	Void	212

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	212

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	212

6.2.2.2	Non-Contention based random access	212

6.2.2.2.1	Correct behaviour when receiving Random Access Response	212

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	212

6.2.3	Requirements for Cat-M1 UEs	212

6.3	RRC Connection Release with Redirection	212

6.3.1	Introduction	212

6.3.2	Requirements	213

6.3.2.1	RRC connection release with redirection to UTRAN FDD	213

6.3.2.2	RRC connection release with redirection to GERAN	213

6.3.2.3	RRC connection release with redirection to UTRAN TDD	213

6.3.2.4	RRC connection release with redirection to NR	214

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	215

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	216

6.4.1	Introduction	216

6.4.2	Requirements	216

6.5	RRC Re-establishment for NB-IoT UEs	216

6.5.1	Introduction	216

6.5.2	Requirements	216

6.5.2.1	UE Re-establishment delay requirement in normal coverage	217

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	217

6.6	Random Access for UE category NB1	218

6.6.1	Introduction	218

6.6.2	Requirements	218

6.6.2.1	Correct behaviour when receiving Random Access Response reception	218

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	218

6.6.2.3	Correct behaviour when receiving a NACK on msg3	218

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	218

6.6.2.5	Correct behaviour when contention Resolution timer expires	218

6.6.2.6	MSG3-based channel quality report for UE Category NB1	218

6.6.3	Requirements for NPRACH configuration	219

6.7	RRC Re-establishment for Cat-M1 UEs	219

6.7.1	Introduction	219

6.7.2	Requirements	219

6.7.2.1	UE Re-establishment delay requirement for CEModeA	220

6.7.2.2	UE Re-establishment delay requirement for CEModeB	220

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	221

6.8.1	Introduction	221

6.8.2	Requirements	221

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	221

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	222

6.9.1	Introduction	222

6.9.2	Requirements	222

7	Timing and signalling characteristics	222

7.1	UE transmit timing	222

7.1.1	Introduction	222

7.1.2	Requirements	223

7.2	UE timer accuracy	224

7.2.1	Introduction	224

7.2.2	Requirements	224

7.3	Timing Advance	225

7.3.1	Introduction	225

7.3.2	Requirements	225

7.3.2.1	Timing Advance adjustment delay	225

7.3.2.2	Timing Advance adjustment accuracy	225

7.4	Cell phase synchronization accuracy (TDD)	225

7.4.1	Definition	225

7.4.2	Minimum requirements	225

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	226

7.5.1	Introduction	226

7.5.2	eNodeB Synchronization Requirements	226

7.5.2.1	Synchronized E-UTRAN	226

7.5.2.2	Non-Synchronized E-UTRAN	226

7.6	Radio Link Monitoring	227

7.6.1	Introduction	227

7.6.2	Requirements	228

7.6.2.1	Minimum requirement when no DRX is used	228

7.6.2.2	Minimum requirement when DRX is used	229

7.6.2.3	Minimum requirement at transitions	230

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	230

7.6.2.5	Minimum requirement under IDC Interference	231

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	231

7.7.1	Introduction	231

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	231

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	233

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	234

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	235

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	236

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	237

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	237

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	237

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	238

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	239

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	240

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	241

7.7.14	SCell Activation Delay Requirement for Dormant SCell	241

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	242

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	243

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	245

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	245

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	247

7.8	Interruptions with Carrier Aggregation	249

7.8.1	Introduction	249

7.8.2	Requirements	249

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	249

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	249

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	249

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	249

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	250

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	250

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	250

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	250

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	251

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	252

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	252

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	252

7.8.2.13	Interruptions at SRS carrier based switching	253

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	253

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	254

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	254

7.8.2.17	Interruptions during CQI measurement on dormant SCell	254

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	254

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	255

7.8.2.20	Interruptions at SCell hibernation	255

7.8.2.21	Interruptions at direct SCell activation and hibernation	255

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	256

7.9.1	Introduction	256

7.9.2	Minimum Requirements for Interband Carrier Aggregation	256

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	256

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	257

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	257

7.10.1	Introduction	257

7.10.2	Requirements	257

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	257

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	257

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	258

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	258

7.11	Radio Link Monitoring for UE Category 0	258

7.11.1	Introduction	258

7.11.2	Requirements for FD-FDD and TDD	260

7.11.2.1	Minimum requirement when no DRX is used	260

7.11.2.2	Minimum requirement when DRX is used	260

7.11.2.3	Minimum requirement at transitions	261

7.11.3	Requirements for HD-FDD	261

7.11.3.1	Minimum requirement when no DRX is used	261

7.11.3.2	Minimum requirement when DRX is used	261

7.11.3.3	Minimum requirement at transitions	262

7.12	Interruptions with Dual Connectivity	262

7.12.1	Introduction	262

7.12.2	Requirements	262

7.12.2.1	Interruptions at PSCell addition/release	262

7.12.2.2	Interruptions at transitions between active and non-active during DRX	263

7.12.2.3	Interruptions at transitions from non-DRX to DRX	263

7.12.2.4	Interruptions at SCell addition/release	263

7.12.2.5	Interruptions at SCell activation/deactivation	263

7.12.2.6	Interruptions during measurements on SCC	264

7.12.2.7	Interruptions at SRS carrier based switching	264

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	265

7.13.1	Definition	265

7.13.2	Minimum requirements	265

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	265

7.14.1	Introduction	265

7.14.2	PSCell Addition Delay Requirement	265

7.14.3	PSCell Release Delay Requirement	266

7.15	Maximum Receive Timing Difference in Dual Connectivity	266

7.15.1	Introduction	266

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	266

7.16	Proximity-based Services	267

7.16.1	Introduction	267

7.16.2	Requirements	267

7.16.2.1	ProSe UE transmission timing	267

7.16.2.1.1	Serving cell or PCell as timing reference	267

7.16.2.1.2	SCell or non-serving cell as timing reference	267

7.16.3	Interruptions with ProSe	267

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	268

7.16.3.2	Interruptions at ProSe Direct Communication configuration	268

7.16.3.3	Interruptions during ProSe Direct Discovery	268

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	268

7.16.3.5	Interruptions during ProSe Direct Communication	269

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	269

7.16.4.1	Measurement and evaluation of selected cell	269

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	269

7.16.5	Selection / Reselection of ProSe relay UE	270

7.16.6	ProSe operation under deactivated SCell	270

7.17	Maximum Transmission Timing Difference in Dual Connectivity	271

7.17.1	Introduction	271

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	271

7.18.1	Introduction	271

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	271

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	271

7.19	Radio Link Monitoring for UE Category M1	271

7.19.1	Introduction	271

7.19.2	Requirements for FD-FDD and TDD CE mode A	272

7.19.2.1	Minimum requirement when no DRX is used	274

7.19.2.2	Minimum requirement when DRX is used	275

7.19.2.3	Minimum requirement at transitions	276

7.19.3	Requirements for HD-FDD with CE mode A	276

7.19.3.1	Minimum requirement when no DRX is used	276

7.19.3.2	Minimum requirement when DRX is used	276

7.19.3.3	Minimum requirement at transitions	278

7.19.4	Requirements for FD-FDD and TDD with CE mode B	278

7.19.4.1	Minimum requirement when no DRX is used	280

7.19.4.2	Minimum requirement when DRX is used	281

7.19.4.3	Minimum requirement at transitions	282

7.19.5	Requirements for HD-FDD with CE mode B	282

7.19.5.1	Minimum requirement when no DRX is used	282

7.19.5.2	Minimum requirement when DRX is used	282

7.19.5.3	Minimum requirement at transitions	284

7.20	UE transmit timing for NB-IoT	284

7.20.1	Introduction	284

7.20.2	Requirements	284

7.21	 UE timer accuracy for NB-IoT	285

7.21.1	Introduction	285

7.21.2	Requirements	285

7.22	Timing Advance for NB-IoT	285

7.22.1	Introduction	285

7.22.2	Requirements	285

7.22.2.1	Timing Advance adjustment delay	285

7.22.2.2	Timing Advance adjustment accuracy	285

7.23	Radio Link Monitoring for Category NB1 UE	285

7.23.1	Introduction	285

7.23.2	Requirements for Category NB1 UE	285

7.23.2.1	Minimum requirement when no DRX is used	286

7.23.2.2	Minimum requirement when DRX is used	286

7.23.2.3	Minimum requirement at transitions	287

7.24	UE transmit timing for Category M1	287

7.24.1	Introduction	287

7.24.2	Requirements	287

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	288

7.25.1	Definition	288

7.25.2	Minimum requirements	288

7.26	UE transmit timing for Category M2	289

7.26.1	Introduction	289

7.26.2	Requirements	289

7.27	UE timer accuracy for category M1	289

7.27.1	Introduction	289

7.27.2	Requirements	289

7.28	Timing Advance for Category M1	290

7.28.1	Introduction	290

7.28.2	Requirements	290

7.29	Interruptions requirements with FeMBMS	290

7.29.1	Introduction	290

7.29.2	Requirements	290

7.30	Numerology switching delay requirements with FeMBMS	290

7.30.1	Introduction	290

7.30.2	Requirements	290

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	290

7.31.1	Introduction	290

7.31.2	NR PSCell Addition Delay Requirement	291

7.31.3	NR PSCell Release Delay Requirement	291

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	292

7.31A.1	Introduction	292

7.31A.2	NR PSCell Addition Delay Requirement	292

7.31A.3	NR PSCell Release Delay Requirement	293

7.32	Interruptions with EN-DC	293

7.32.1	Introduction	293

7.32.2	Requirements	294

7.32.2.1	Interruptions at PSCell addition/release	294

7.32.2.2	Interruptions at transitions between active and non-active during DRX	294

7.32.2.3	Interruptions at transitions from non-DRX to DRX	294

7.32.2.4	Interruptions at SCell addition/release	294

7.32.2.5	Interruptions at SCell activation/deactivation	294

7.32.2.6	Interruptions during measurements on SCC	295

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	295

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	295

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	296

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	296

7.32.2.7	Interruptions at active BWP switching	296

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	297

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	297

7.32.2.10	Interruptions at SCell hibernation	297

7.32.2.11	Interruptions at direct SCell activation and hibernation	297

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	297

7.32.2.13	Interruptions at NR SRS carrier based switching	298

7.32.2.14	Interruptions at NR SCell dormancy	299

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	299

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	299

7.32.2.15	Interruption during NR measurement with autonomous gaps	299

7.32.2.16	Interruptions at SRS carrier based switching	299

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	300

7.33.1	Introduction	300

7.33.2	Requirements	300

7.34	Void	300

7.35	Interruptions with SFTD measurements	300

7.35.1	Introduction	300

7.35.2	Requirements	300

7.36	Interruptions with NE-DC	301

7.36.1	Introduction	301

7.36.2	Requirements	301

7.36.2.1	Interruptions at transitions between active and non-active during DRX	301

7.36.2.2	Interruptions at transitions from non-DRX to DRX	301

7.36.2.3	Interruptions at SCell addition/release	302

7.36.2.4	Interruptions at SCell activation/deactivation	302

7.36.2.5	Interruptions during measurements on SCC	302

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	302

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	303

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	303

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	303

7.36.2.6	Interruptions at active BWP switching	303

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	304

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	304

7.36.2.9	Interruptions at SCell hibernation	304

7.36.2.10	Interruptions at direct SCell activation and hibernation	304

7.36.2.11	Interruptions at NR SRS carrier based switching	304

7.36.2.12	Interruptions at NR SCell dormancy	305

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	305

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	306

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	306

7.36.2.14	Interruption during NR measurement with autonomous gaps	306

7.36.2.15	Interruptions at SRS carrier based switching	307

7.37	Interruptions during NR measurement with autonomous gaps	307

7.37.1	Introduction	307

7.37.2	Requirements	307

8	UE Measurements Procedures in RRC_CONNECTED State	308

8.1	General Measurement Requirements	308

8.1.1	Introduction	308

8.1.2	Requirements	308

8.1.2.1	UE measurement capability	308

8.1.2.1.1	Monitoring of multiple layers using gaps	316

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	317

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	318

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	320

8.1.2.1.2	Network controlled small gap	321

8.1.2.2	E-UTRAN intra frequency measurements	322

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	322

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	327

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	331

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	332

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	333

8.1.2.3	E-UTRAN inter frequency measurements	334

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	334

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	340

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	346

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	346

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	347

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	348

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	349

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	350

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	351

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	358

8.1.2.4	Inter RAT measurements	358

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	358

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	363

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	363

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	367

8.1.2.4.5	E-UTRAN FDD – GSM measurements	367

8.1.2.4.6	E-UTRAN TDD – GSM measurements	372

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	372

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	374

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	374

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	374

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	375

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	375

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	375

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	375

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	377

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	377

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	377

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	377

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	378

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	378

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	380

8.1.2.4.21	E-UTRAN FDD – NR measurements	380

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	384

8.1.2.4.22	E-UTRAN TDD – NR measurements	388

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	388

8.1.2.4.23	Void	388

8.1.2.4.24	Void	388

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	388

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	390

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	390

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	391

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	391

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	392

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	393

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	395

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	397

8.1.2.6.5	Void	399

8.1.2.6.6	Void	399

8.1.2.6.7	Void	399

8.1.2.6.8	Void	399

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	399

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	399

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	401

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	403

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	405

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	407

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	408

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	410

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	412

8.1.2.7	E-UTRAN E-CID Measurements	414

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	414

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	415

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	416

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	417

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	417

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	417

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	420

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	423

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	427

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	430

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	430

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	431

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	431

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	432

8.1.2.10	Void	432

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	432

8.2.1	Introduction	432

8.2.2	Requirements	432

8.3	Measurements for E-UTRA carrier aggregation	437

8.3.1	Introduction	437

8.3.2	Measurements of the primary component carrier	437

8.3.3	Measurements of a secondary component carrier	437

8.3.3.1	Measurements of a secondary component carrier with active SCell	437

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	438

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	438

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	439

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	441

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	441

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	441

8.4.1	Introduction	441

8.4.2	Measurements on the primary component carrier	442

8.4.3	Measurements on a secondary component carrier	443

8.4.4	Measurements on both primary component carrier and a secondary component carrier	443

8.4.5	Measurements on different secondary component carriers	444

8.5	Measurements for UE category 0	445

8.5.1	Introduction	445

8.5.2 	Requirements	446

8.5.2.1 	E-UTRAN intra frequency measurements	446

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	446

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	449

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	451

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	455

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	456

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	456

8.6	Discovery signal measurements	457

8.6.1	Introduction	457

8.6.2	Requirements for CRS based discovery signal measurements	457

8.6.2.1	E-UTRAN intra frequency measurements	457

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	457

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	460

8.6.2.2	E-UTRAN inter frequency measurements	462

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	463

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	465

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	468

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	468

8.6.3	Requirements for CSI-RS based discovery signal measurements	468

8.6.3.1	E-UTRAN intra frequency measurements	468

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	469

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	471

8.6.3.2	E-UTRAN inter frequency measurements	473

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	474

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	476

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	479

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	479

8.7	Discovery signal measurements for E-UTRA carrier aggregation	479

8.7.1	Introduction	479

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	480

8.7.2.1	Measurements of the primary component carrier	480

8.7.2.2	Measurements of a secondary component carrier	480

8.7.2.3	Measurements of a secondary component carrier with active SCell	480

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	480

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	480

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	482

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	483

8.7.3.1	Measurements of the primary component carrier	483

8.7.3.2	Measurements of a secondary component carrier	483

8.7.3.3	Measurements of a secondary component carrier with active SCell	483

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	483

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	483

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	485

8.8	Measurements for E-UTRA dual connectivity	486

8.8.1	Introduction	486

8.8.2	Intra-frequency measurements requirements on PCell	487

8.8.3	Intra-frequency measurements requirements on PSCell	487

8.8.4	Inter-frequency and inter-RAT measurement requirements	487

8.8.5	Intra-frequency measurements with autonomous gaps	487

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	487

8.8.5.2	ECGI reporting delay	488

8.8.6	Inter-frequency measurements with autonomous gaps	488

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	488

8.8.6.2	ECGI reporting delay	489

8.8.7	SSTD Measurements	489

8.8.7.1	Introduction	489

8.8.7.2	SSTD Measurement requirements	489

8.8.7.3	SSTD Measurement Reporting Delay	490

8.8.8	Intra-frequency measurements requirements on SCell	490

8.9	MBSFN Measurements	490

8.9.1	Introduction	490

8.9.2	MBSFN RSRP Measurements	491

8.9.3	MBSFN RSRQ Measurements	491

8.9.4	MCH BLER Measurements	491

8.10	Proximity-based Services	491

8.10.1	Introduction	491

8.10.2	Requirements	491

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	491

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	492

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	493

8.11.1	Introduction	493

8.11.2	CRS based discovery signal measurements	493

8.11.2.1	E-UTRAN intra-frequency measurements	493

8.11.2.1.1	Requirements	493

8.11.2.1.1.1	Requirements when no DRX is used	493

8.11.2.1.1.1.1	Measurement Reporting Requirements	495

8.11.2.1.1.2	Requirements when DRX is used	496

8.11.2.1.1.2.1	Measurement Reporting Requirements	498

8.11.2.2	E-UTRAN inter-frequency measurements	499

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	499

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	503

8.11.3	CSI-RS based discovery signal measurements	503

8.11.3.1	E-UTRAN intra-frequency measurements	503

8.11.3.1.1	Requirements	503

8.11.3.1.1.1	Requirements when no DRX is used	503

8.11.3.1.1.1.1	Measurement Reporting Requirements	505

8.11.3.1.1.2	Requirements when DRX is used	505

8.11.3.1.1.2.1	Measurement Reporting Requirements	506

8.11.3.2	E-UTRAN inter-frequency measurements	507

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	507

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	511

8.11.4	RSSI measurements	511

8.11.4.1	E-UTRAN intra-frequency measurements	511

8.11.4.2	E-UTRAN inter-frequency measurements	511

8.11.5	Channel occupancy measurements	512

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	512

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	512

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	512

8.12.1	Introduction	512

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	512

8.12.2.1	Introduction	512

8.12.2.2	Measurements of a secondary component carrier	513

8.12.2.3	Measurements of a secondary component carrier with active SCell	513

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	513

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	513

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	516

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	519

8.12.3.1	Introduction	519

8.12.3.2	Measurements of a secondary component carrier	519

8.12.3.3	Measurements of a secondary component carrier with active SCell	519

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	519

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	519

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	521

8.13	Measurements for UE Category M1	523

8.13.1	Introduction	523

8.13.2	Requirements for UE category M1 with CE mode A	523

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	524

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	524

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	529

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	531

8.13.2.2	Void	535

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	535

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	535

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	538

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	540

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	541

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	541

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	543

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	545

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	546

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	546

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	546

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	547

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	547

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	547

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	548

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	548

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	549

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	550

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	550

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	551

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	555

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	557

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	561

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	561

8.13.3	Requirements for UE category M1 with CE mode B	562

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	562

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	563

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	567

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	570

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	575

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	576

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	576

8.13.3.2	Void	577

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	577

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	577

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	580

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	582

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	583

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	583

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	583

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	584

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	584

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	584

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	585

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	585

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	585

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	590

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	592

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	597

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	597

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	597

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	599

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	602

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	602

8.14	Measurements for UE category NB1	603

8.14.1	Introduction	603

8.14.2	NB-IoT intra frequency measurements under normal coverage	603

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	603

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	603

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	603

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	603

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	603

8.14.4	Connected mode channel quality report for UE Category NB1	604

8.15	Void	604

8.16	Measurements for UE Category M2	604

8.16.1	Introduction	604

8.16.2	Requirements for UE category M2 with CE mode A	604

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	604

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	605

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	605

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	606

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	606

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	607

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	607

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	607

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	610

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	612

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	613

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	613

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	615

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	617

8.16.3	Requirements for UE category M2 with CE mode B	618

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	618

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	618

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	621

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	623

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	624

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	624

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	626

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	628

8.17	Measurements for E-UTRA – NR Dual Connectivity	629

8.17.1	Introduction	629

8.17.1.1	Measurement Gap Sharing	629

8.17.1A	Intrafrequency Measurements	630

8.17.2	SFTD Measurements	630

8.17.2.1	Introduction	630

8.17.2.2	SFTD Measurement requirements	630

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	631

8.17.2.3	SFTD Measurement Reporting Delay	632

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	632

8.17.3.1	Introduction	632

8.17.3.2	E-UTRAN FDD inter frequency measurements	632

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	632

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	634

8.17.3.3	E-UTRAN TDD inter frequency measurements	636

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	636

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	638

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	640

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	640

8.17.4.1.1	NR Inter-RAT cell identification	640

8.17.4.1.2	NR Inter-RAT measurement	642

8.17.4.1.3	NR Inter-RAT measurement reporting	642

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	643

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.4A.1.1	NR Inter-RAT cell identification	644

8.17.4A.1.2	NR Inter-RAT measurement	645

8.17.4A.1.3	NR Inter-RAT measurement reporting	646

8.17.4A.1.4	NR inter-RAT RSSI measurements	646

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	647

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	647

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	648

8.17.5.1	Introduction	648

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	648

8.17.5.2.1	Identification of a new UTRA FDD cell	648

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	648

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	648

8.17.5.2.4	Periodic Reporting	649

8.17.5.2.5	Event Triggered Reporting	649

8.17.5.2.6	Event-triggered Periodic Reporting	649

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	650

8.17.5.3.1	Periodic Reporting	651

8.17.5.3.2	Event Triggered Reporting	651

8.17.5.3.3	Event-triggered Periodic Reporting	651

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	652

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	652

8.17.7.1	Introduction	652

8.17.7.2	Identification of a new UTRA FDD cell for SON	652

8.17.7.2.1	Requirements when no DRX is used	652

8.17.7.2.2	Requirements when DRX is used	652

8.17.7.2.3	Reporting Delay	653

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	654

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	654

8.17.9.1	Introduction	654

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	654

8.17.9.2.1	Identification of a new UTRA FDD cell	654

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	654

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	655

8.17.9.2.4	Periodic Reporting	655

8.17.9.2.5	Event Triggered Reporting	655

8.17.9.2.6	Event-triggered Periodic Reporting	656

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	656

8.17.9.3.1	Periodic Reporting	657

8.17.9.3.2	Event Triggered Reporting	657

8.17.9.3.3	Event-triggered Periodic Reporting	657

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	658

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	658

8.17.11.1	Introduction	658

8.17.11.2	Identification of a new UTRA TDD cell for SON	658

8.17.11.2.1	Requirements when no DRX is used	658

8.17.11.2.2	Requirements when DRX is used	658

8.17.11.2.3	Reporting Delay	659

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	659

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	660

8.17.13.1	Introduction	660

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	660

8.17.13.2.1	GSM carrier RSSI	660

8.17.13.2.2	BSIC verification	660

8.17.13.2.3	Enhanced BSIC verification	662

8.17.13.2.4	Periodic Reporting	662

8.17.13.2.5	Event Triggered Reporting	662

8.17.13.2.6	Event-triggered Periodic Reporting	663

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	663

8.17.13.3.1	GSM carrier RSSI	663

8.17.13.3.2	BSIC verification	664

8.17.13.3.3	Periodic Reporting	665

8.17.13.3.4	Event Triggered Reporting	665

8.17.13.3.5	Event-triggered Periodic Reporting	665

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	665

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	665

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	665

8.17.15.1.1	RSTD Measurement Reporting Delay	666

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	666

8.17.15.2.1	RSTD Measurement Reporting Delay	666

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	666

8.17.15.3.1	RSTD Measurement Reporting Delay	667

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	667

8.17.15.4.1	RSTD Measurement Reporting Delay	667

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	668

8.17.16.1	Introduction	668

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	668

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	668

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	668

8.17.17.1	Introduction	668

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	668

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	668

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	668

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	668

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	668

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	668

8.18	Measurements for non-BL/CE UE	668

8.18.1	Introduction	668

8.18.2	Requirements for non-BL/CE UE with CE Mode B	669

8.18.2.1	E-UTRAN intra frequency measurements	669

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	669

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	669

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	670

8.19	Measurements for NR – E-UTRA Dual Connectivity	671

8.19.1	Introduction	671

8.19.2	Intra-frequency Measurements	671

8.19.3	Inter-frequency Measurements	671

8.19.4	Void	671

8.19.5	Intra-frequency E-CID Measurements	671

8.19.6	Intra-frequency measurements with autonomous gaps	672

8.19.6.1	Introduction	672

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	672

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	672

8.19.7	Inter-frequency measurements with autonomous gaps	672

8.19.7.1	Introduction	672

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	672

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	672

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	672

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	672

9	Measurements performance requirements for UE	672

9.1	E-UTRAN measurements	672

9.1.1	Introduction	672

9.1.2	Intra-frequency RSRP Accuracy Requirements	673

9.1.2.1	Absolute RSRP Accuracy	673

9.1.2.2	Relative Accuracy of RSRP	673

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	674

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	675

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	676

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	677

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	678

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	679

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	680

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	680

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	680

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	681

9.1.2B.1	Introduction	681

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	681

9.1.3	Inter-frequency RSRP Accuracy Requirements	682

9.1.3.1	Absolute RSRP Accuracy	682

9.1.3.2	Relative Accuracy of RSRP	683

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	684

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	684

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	685

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	685

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	686

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	687

9.1.3B.1	Introduction	687

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	687

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	687

9.1.4	RSRP Measurement Report Mapping	688

9.1.5	Intra-frequency RSRQ Accuracy Requirements	688

9.1.5.1	Absolute RSRQ Accuracy	688

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	689

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	690

9.1.5.4	Absolute WB-RSRQ Accuracy	691

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	692

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	693

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	693

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	693

9.1.5B.1	Introduction	693

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	693

9.1.6	Inter-frequency RSRQ Accuracy Requirements	694

9.1.6.1	Absolute RSRQ Accuracy	694

9.1.6.2	Relative Accuracy of RSRQ	695

9.1.6.3	Absolute WB-RSRQ Accuracy	696

9.1.6.4	Relative WB-RSRQ Accuracy	696

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	697

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	698

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	698

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	698

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	699

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	700

9.1.6B.1	Introduction	700

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	700

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	700

9.1.7	RSRQ Measurement Report Mapping	701

9.1.8	Power Headroom	701

9.1.8.1	Period	702

9.1.8.2	Reporting Delay	702

9.1.8.3	Void	702

9.1.8.4	Report Mapping	702

9.1.9	UE Rx – Tx time difference	702

9.1.9.1	Measurement Requirement	702

9.1.9.2	Measurement Report mapping	703

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	704

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	705

9.1.10	Reference Signal Time Difference (RSTD)	706

9.1.10.1	Intra-Frequency Accuracy Requirement	706

9.1.10.2	Inter-Frequency Accuracy Requirement	707

9.1.10.3	RSTD Measurement Report Mapping	709

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	709

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	710

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	711

9.1.11	Carrier aggregation measurement accuracy	712

9.1.11.1	Primary component carrier accuracy requirement	713

9.1.11.2	Secondary component carrier accuracy requirement	713

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	713

9.1.11.4	Secondary component carrier relative accuracy requirement	713

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	713

9.1.13	Measurement accuracy for UE category 0	714

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	714

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	714

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	715

9.1.14	Accuracy requirements for Discovery Signal Measurements	716

9.1.14.1	Introduction	716

9.1.14.2	RSRP measurements in discovery signal occasions	716

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	716

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	716

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	716

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	717

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	718

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	718

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	718

9.1.14.3.3	CSI-RSRP measurement report mapping	719

9.1.14.4	RSRQ measurements in discovery signal occasions	719

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	719

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	720

9.1.15.1.1	Primary component carrier accuracy requirement	720

9.1.15.1.2	Secondary component carrier accuracy requirement	720

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	720

9.1.15.1.4	Secondary component carrier relative accuracy requirement	720

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	720

9.1.15.2.1	Primary component carrier accuracy requirement	720

9.1.15.2.2	Secondary component carrier accuracy requirement	720

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	720

9.1.15.2.4	Secondary component carrier relative accuracy requirement	720

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	721

9.1.17	RS-SINR Measurements	721

9.1.17.1	Measurement Report Mapping	721

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	722

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	722

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	722

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	722

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	723

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	724

9.1.18.1	Introduction	724

9.1.18.2	RSRP measurements	724

9.1.18.2.1	RSRP measurement report mapping	724

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	724

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	725

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	725

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	726

9.1.18.3	RSRQ measurements	726

9.1.18.3.1	RSRQ measurement report mapping	726

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	726

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	727

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	727

9.1.18.4	CSI-RSRP measurements	728

9.1.18.4.1	CSI-RSRP measurement report mapping	728

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	728

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	728

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	729

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	729

9.1.18.5	RSSI measurements	730

9.1.18.5.1	RSSI measurement report mapping	730

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	730

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	731

9.1.18.6	Channel occupancy measurements	731

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	731

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	731

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	732

9.1.19.1	Introduction	732

9.1.19.2	Accuracy requirements for measurements on SCC	732

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	732

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	732

9.1.20	SFN and Subframe Time Difference (SSTD)	733

9.1.20.1	SSTD Accuracy Requirement	733

9.1.20.2	SSTD Measurement Report Mapping	733

9.1.21	Measurement accuracy for UE category M1	734

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	734

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	736

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	737

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	739

9.1.21.5	RSRP Measurement Report Mapping	740

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	740

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	741

9.1.21.8	RSRQ Measurement Report Mapping	742

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	742

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	743

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	744

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	746

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	747

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	748

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	749

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	750

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	751

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	752

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	753

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	754

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	757

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	760

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	760

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	763

9.1.22	Measurement accuracy for UE Category NB1	764

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	764

9.1.22.2	Void	765

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	765

9.1.22.4	Void	766

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	766

9.1.22.6	Void	767

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	767

9.1.22.8	Void	768

9.1.22.9	NRSRP Measurement Report Mapping	768

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	768

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	769

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	770

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	771

9.1.22.14	NRSRQ Measurement Report Mapping	772

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	773

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	773

9.1.23	Power Headroom for UE Category NB1	774

9.1.23.1	Period	774

9.1.23.2	Reporting Delay	774

9.1.23.3	Report Mapping for UE Category NB1	774

9.1.23.3.1	Void	776

9.1.23.3.2	Void	776

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	776

9.1.24	Void	777

9.1.25	Measurement accuracy for UE category M2	777

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	777

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	778

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	779

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	780

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	782

9.1.26	Measurement Accuracy for non-BL CE UE	783

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	784

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	786

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	786

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	788

9.1.26.5	RSRP Measurement Report Mapping	789

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	789

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	789

9.1.26.8	RSRQ Measurement Report Mapping	790

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	790

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	791

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	792

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	793

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	794

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	794

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	794

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	795

9.1.27	SFN and frame Timing Difference (SFTD)	796

9.1.27.1	SFTD Accuracy Requirement	796

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	798

9.1.28.1	SFTD Accuracy Requirement under CCA	798

9.2	UTRAN FDD Measurements	799

9.2.1	UTRAN FDD CPICH RSCP	799

9.2.2	Void	800

9.2.3	UTRAN FDD CPICH Ec/No	800

9.3	UTRAN TDD Measurements	800

9.3.1	UTRAN TDD P-CCPCH RSCP	801

9.3.2	Void	801

9.3.3	Void	801

9.4	GSM Measurements	801

9.4.1	GSM carrier RSSI	801

9.5	CDMA2000 1x RTT Measurements	801

9.5.1	CDMA2000 1x RTT Pilot Strength	801

9.6	PCMAX,c	802

9.6.1	Report Mapping	802

9.6.2 	Estimation Period	802

9.6.3	Reporting Delay	802

9.7	IEEE802.11 Measurements	802

9.7.1	WLAN RSSI	802

9.7.2	WLAN RSSI Measurement Report Mapping	802

9.8	MBSFN Measurements	803

9.8.1	Introduction	803

9.8.2	MBSFN RSRP	803

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	803

9.8.2.2	MBSFN RSRP measurement report mapping	804

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	804

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	805

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	805

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	805

9.8.3	MBSFN RSRQ	806

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	806

9.8.3.2	MBSFN RSRQ measurement report mapping	806

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	807

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	807

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	807

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	808

9.8.4	MCH BLER	808

9.8.4.1	Measurement report mapping for MCH BLER	808

9.8.4.2	Measurement report mapping for MCH Block Number	809

9.9	ProSe Measurements	810

9.9.1	Introduction	810

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	810

9.9.2.1	Absolute S-RSRP Accuracy	810

9.9.2.2	Relative Accuracy of S-RSRP	811

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	812

9.9.3.1	Absolute SD-RSRP Accuracy	812

9.9.3.2	Relative Accuracy of SD-RSRP	812

9.10	V2X Measurements	813

9.10.1	Introduction	813

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	813

9.10.2.1	Absolute S-RSRP Accuracy	813

9.10.2.2	Relative Accuracy of S-RSRP	814

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	815

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	815

9.10.4	S-RSSI Measurement Accuracy Requirements	815

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	815

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	816

9.11	NR Measurements	816

9.11.1	NR SS-RSRP Measurements	817

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	817

9.11.2	NR SS-RSRQ Measurements	817

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	817

9.11.3	NR SS-SINR Measurements	818

9.11.4	NR SS-RSRP Measurements under CCA	818

9.11.5	NR SS-RSRQ Measurements under CCA	818

9.11.6	NR SS-SINR Measurements under CCA	819

9.11.7	NR RSSI Measurements under CCA	819

9.11.8	NR Channel Occupancy Measurements under CCA	819

10	Measurements Performance Requirements for E-UTRAN	820

10.1	Received Interference Power	820

10.1.1	Absolute accuracy requirement	820

10.1.2	Relative accuracy requirement	820

10.1.3	Received Interference Power measurement report mapping	820

10.2	Angle of Arrival (AOA)	820

10.2.1	Range/mapping	820

10.3	Timing Advance (TADV)	821

10.3.1	Report mapping	821

11	ProSe Requirements in Any Cell Selection state	821

11.1	Introduction	821

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	822

11.2.1	Introduction	822

11.2.2	ProSe UE transmission timing	822

11.3	Initiation/Cease of SLSS Transmissions	822

11.3.1	Introduction	822

11.3.2	Requirements	822

11.4	Measurements for ProSe in Any Cell Selection State	822

11.4.1	Introduction	822

11.4.2	Requirements	823

11.4.2.1	E-UTRA FDD	823

11.4.2.2	E-UTRA TDD	823

11.5	Selection / Reselection of ProSe Synchronization Reference	823

11.5.1	Introduction	823

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	824

11.5.2.1	Introduction	824

11.5.2.2	Requirements	824

11.6	Void	824

11.7	Selection / Reselection of ProSe relay UE	824

11.7.1	Introduction	824

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	824

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	825

12.1	Introduction	825

12.2	Transmit Timing	825

12.2.1	GNSS as timing reference	825

12.3	Interruption	825

12.4	Reliability of GNSS signal	825

13	V2X Requirements	826

13.1	Introduction	826

13.2	UE Transmit Timing	826

13.2.1	Introduction	826

13.2.2	GNSS as synchronization reference source	826

13.2.3	Serving cell/PCell as synchronization reference source	826

13.2.4	SyncRef UE as synchronization reference source	827

13.3	Initiation/Cease of SLSS Transmissions	827

13.3.1	Introduction	827

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	827

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	827

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	828

13.4	Selection / Reselection of V2X Synchronization Reference Source	828

13.5	Autonomous Resource Selection/Reselection measurements	829

13.5.1	Introduction	829

13.5.2	PSSCH-RSRP measurements	829

13.5.3	S-RSSI measurements	829

13.6	Congestion Control measurements	829

13.7	Interruption	830

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	830

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	830

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	830

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	831

13.8	Reliability of GNSS signal	831

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	831

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	831

Annex A (normative): Test Cases	832

A.1	Purpose of annex	832

A.2	Requirement classification for statistical testing	832

A.2.1	Types of requirements in TS 36.133	832

A.2.1.1	Time and delay requirements on UE higher layer actions	832

A.2.1.2	Measurements of power levels, relative powers and time	832

A.2.1.3	Implementation requirements	833

A.2.1.4	Physical layer timing requirements	833

A.3	RRM test configurations	834

A.3.1	Reference Measurement Channels	834

A.3.1.1	PDSCH	834

A.3.1.1.1	FDD	834

A.3.1.1.2	TDD	839

A.3.1.1.3	FDD for UE category 0	842

A.3.1.1.4	HD-FDD for UE category 0	843

A.3.1.1.5	TDD for UE category 0	844

A.3.1.1.6	Frame Structure 3	845

A.3.1.2	PCFICH/PDCCH/PHICH	846

A.3.1.2.1	FDD	846

A.3.1.2.2	TDD	846

A.3.1.2.3	HD-FDD for UE category 0	847

A.3.1.2.4	FS 3	847

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	847

A.3.1.3.1	FDD in CEModeA	848

A.3.1.3.2	HD-FDD in CEModeA	848

A.3.1.3.3	TDD in CEModeA	849

A.3.1.3.4	FDD in CEModeB	849

A.3.1.3.5	HD-FDD in CEModeB	850

A.3.1.3.6	TDD in CEModeB	850

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	851

A.3.1.4.1	FDD in CEModeA	851

A.3.1.4.2	HD-FDD in CEModeA	852

A.3.1.4.3	TDD in CEModeA	853

A.3.1.4.4	FDD in CEModeB	854

A.3.1.4.5	 HD-FDD in CEModeB	855

A.3.1.4.6	TDD in CEModeB	855

A.3.1.5	NPDSCH Reference Channel for UE category NB1	856

A.3.1.5.1	HD-FDD in-band operation	856

A.3.1.5.2	Void	857

A.3.1.5.3	HD-FDD standalone operation	857

A.3.1.5.4	Void	858

A.3.1.5.5	HD-FDD guard band operation	858

A.3.1.5.6	Void	859

A.3.1.5.7	TDD in-band operation	859

A.3.1.5.8	TDD standalone operation	859

A.3.1.5.9	TDD guard band operation	860

A.3.1.6	NPDCCH Reference Channel for UE category NB1	860

A.3.1.6.1	In-band operation	860

A.3.1.6.2	Void	861

A.3.1.6.3	Standalone operation	861

A.3.1.6.4	Void	861

A.3.1.6.5	Guard band operation	861

A.3.1.6.6	Void	862

A.3.2	OFDMA Channel Noise Generator (OCNG)	862

A.3.2.1	OCNG Patterns for FDD	862

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	863

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	863

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	864

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	865

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	865

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	866

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	866

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	866

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	867

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	868

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	868

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	869

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	869

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	870

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	870

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	871

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	871

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	872

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	872

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	873

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	873

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	874

A.3.2.2	OCNG Patterns for TDD	874

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	875

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	875

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	876

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	876

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	877

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	878

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	879

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	880

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	880

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	881

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	881

A.3.2.3	OCNG Patterns for Narrowband IoT	882

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	883

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	884

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	884

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	885

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	886

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	887

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	888

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	889

A.3.2.4	OCNG Patterns for V2X sidelink	890

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	890

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	891

A.3.3	Reference DRX Configurations	891

A.3.4	ABS Transmission Configurations	892

A.3.4.1	Non-MBSFN ABS Transmission Configurations	892

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	892

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	892

A.3.4.2	MBSFN ABS Transmission Configurations	893

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	893

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	894

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	894

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	894

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	895

A.3.6.1	Introduction	895

A.3.7	Test Cases with Different Channel Bandwidths	895

A.3.7.1	Introduction	895

A.3.7.2	Principle of testing	895

A.3.8	Antenna Configuration	895

A.3.8.1	Antenna connection for 4 Rx capable UEs	895

A.3.8.1.1 Introduction	895

A.3.8.1.2 Principle of testing	895

A.3.8.1.2.1 Single carrier tests	895

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	896

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	897

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	897

A.3.8.2	Antenna connection for 8 Rx capable UEs	897

A.3.8.2.1	Introduction	897

A.3.8.2.2	Principle of testing	897

A.3.8.2.2.1	Single carrier tests	897

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	898

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	898

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	898

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	898

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	898

A.3.9.1	Introduction	898

A.3.9.2	Principle of testing	898

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	899

A.3.10.1	Introduction	899

A.3.10.2	Principle of testing	899

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	899

A.3.11.1	Introduction	899

A.3.11.2	Principle of Testing	899

A.3.12	Proximity-based Services	899

A.3.12.1	Introduction	899

A.3.12.2	Reference DRX configurations for ProSe tests	899

A.3.12.3	Test Cases with Different Channel Bandwidths	900

A.3.12.3.1	Introduction	900

A.3.12.3.2	Principle of testing	900

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	900

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	907

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	910

A.3.12.6.1	FDD	910

A.3.12.7	Reference measurement channels for ProSe Direct Communication	910

A.3.12.7.1	FDD	910

A.3.12.8	ProSe Receive Traffic Generator	911

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	911

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	911

A.3.13	Time Offset between Cells	912

A.3.13.1	Introduction	912

A.3.13.2	Definition	912

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	912

A.3.14.1	Introduction	912

A.3.14.2	Principle of testing	912

A.3.15	Dual connectivity test cases with different combination of duplex mode	912

A.3.15.1	Introduction	912

A.3.15.2	Principle of testing	912

A.3.16	Reference PRACH Configurations	913

A.3.17	Listen before talk model	913

A.3.17.1	Introduction	913

A.3.17.2	Definition	913

A.3.18	Reference NPRACH Configurations	914

A.3.19	Dual connectivity test cases with different bandwidth combinations	915

A.3.19.1	Introduction	915

A.3.19.2	Principle of testing	915

A.3.20	Category M1 UE Test Cases	916

A.3.20.1	Introduction	916

A.3.20.2	Principle of Cat-M1 UE Testing	916

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	917

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	918

A.3.21.1	Introduction	918

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	918

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	919

A.3.22	Category 1bis UE Test Cases	920

A.3.22.1	Introduction	920

A.3.22.2	Principle of Category 1bis UE Testing	920

A.3.23	Category NB2 UE Test Cases	924

A.3.23.1	Introduction	924

A.3.23.2	Principle of Category NB2 UE Testing	924

A.3.24	V2X sidelink communication	927

A.3.24.1	Introduction	927

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	928

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	932

A.3.25	Category M2 UE Test Cases	933

A.3.25.1	Introduction	933

A.3.25.2	Principle of Cat-M2 UE Testing	933

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	934

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	935

A.3.26.1	Introduction	935

A.3.26.2	Principle of testing	935

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	935

A.3.27.1	Introduction	935

A.3.27.2	Principle of INACTIVE cell re-selection Testing	935

A.4	E-UTRAN RRC_IDLE state	935

A.4.2	Cell Re-Selection	935

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	935

A.4.2.1.1	Test Purpose and Environment	935

A.4.2.1.2	Test Requirements	937

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	938

A.4.2.2.1	Test Purpose and Environment	938

A.4.2.2.2	Test Requirements	939

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	940

A.4.2.3.1	Test Purpose and Environment	940

A.4.2.3.2	Test Requirements	941

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	942

A.4.2.4.1	Test Purpose and Environment	942

A.4.2.4.2	Test Requirements	943

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	944

A.4.2.5.1	Test Purpose and Environment	944

A.4.2.5.2	Test Requirements	945

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	946

A.4.2.6.1	Test Purpose and Environment	946

A.4.2.6.2	Test Requirements	947

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	948

A.4.2.7.1	Test Purpose and Environment	948

A.4.2.7.2	Test Requirements	949

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	950

A.4.2.8.1	Test Purpose and Environment	950

A.4.2.8.2	Test Requirements	951

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	952

A.4.2.9.1	Test Purpose and Environment	952

A.4.2.9.2	Test Requirements	952

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	952

A.4.2.10.1	Test Purpose and Environment	952

A.4.2.10.2	Test Requirements	956

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	956

A.4.2.11.1	Test Purpose and Environment	956

A.4.2.11.2	Test Requirements	960

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	960

A.4.2.12.1	Test Purpose and Environment	960

A.4.2.12.2	Test Requirements	962

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	963

A.4.2.13.1	Test Purpose and Environment	963

A.4.2.13.2	Test Requirements	964

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	965

A.4.2.14.1	Test Purpose and Environment	965

A.4.2.14.2	Test Requirements	966

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	967

A.4.2.15.1	Test Purpose and Environment	967

A.4.2.15.2	Test Requirements	968

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	969

A.4.2.16.1	Test Purpose and Environment	969

A.4.2.16.2	Test Requirements	970

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	971

A.4.2.17.1	Test Purpose and Environment	971

A.4.2.17.2	Test Requirements	972

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	973

A.4.2.18.1	Test Purpose and Environment	973

A.4.2.18.2	Test Requirements	975

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	976

A.4.2.19.1	Test Purpose and Environment	976

A.4.2.19.2	Test Requirements	978

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	979

A.4.2.20.1	Test Purpose and Environment	979

A.4.2.20.2	Test Requirements	980

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	981

A.4.2.21.1	Test Purpose and Environment	981

A.4.2.21.2	Test Requirements	982

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	983

A.4.2.22.1	Test Purpose and Environment	983

A.4.2.22.2	Test Requirements	984

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	985

A.4.2.23.1	Test Purpose and Environment	985

A.4.2.23.2	Test Requirements	986

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	987

A.4.2.24.1	Test Purpose and Environment	987

A.4.2.24.2	Test Requirements	990

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	991

A.4.2.25.1	Test Purpose and Environment	991

A.4.2.25.2	Test Requirements	992

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	993

A.4.2.26.1	Test Purpose and Environment	993

A.4.2.26.2	Test Requirements	994

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	995

A.4.2.27.1	Test Purpose and Environment	995

A.4.2.27.2	Test Requirements	996

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	997

A.4.2.28.1	Test Purpose and Environment	997

A.4.2.28.2	Test Requirements	998

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	999

A.4.2.29.1	Test Purpose and Environment	999

A.4.2.29.2	Test Requirements	1000

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1001

A.4.2.30.1	Test Purpose and Environment	1001

A.4.2.30.2	Test Requirements	1002

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1003

A.4.2.31.1	Test Purpose and Environment	1003

A.4.2.31.2	Test Requirements	1004

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1005

A.4.2.32.1	Test Purpose and Environment	1005

A.4.2.32.2	Test Requirements	1006

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1007

A.4.2.33.1	Test Purpose and Environment	1007

A.4.2.33.2	Test Requirements	1008

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1009

A.4.2.34.1	Test Purpose and Environment	1009

A.4.2.34.2	Test Requirements	1010

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1011

A.4.2.35.1	Test Purpose and Environment	1011

A.4.2.35.2	Test Requirements	1013

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1014

A.4.2.36.1	Test Purpose and Environment	1014

A.4.2.36.2	Test Requirements	1016

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1017

A.4.2.37.1	Test Purpose and Environment	1017

A.4.2.37.2	Test Requirements	1019

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1020

A.4.2.38.1	Test Purpose and Environment	1020

A.4.2.38.2	Test Requirements	1022

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1023

A.4.2.39.1	Test Purpose and Environment	1023

A.4.2.39.2	Test Requirements	1024

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1025

A.4.2.40.1	Test Purpose and Environment	1025

A.4.2.40.2	Test Requirements	1026

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1027

A.4.2.41.1	Test Purpose and Environment	1027

A.4.2.41.2	Test Requirements	1030

A.4.2.42.2	Test Requirements	1034

A.4.2.43.2	Test Requirements	1037

A.4.2.44.2	Test Requirements	1041

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1041

A.4.2.45.1	Test Purpose and Environment	1041

A.4.2.45.2	Test Requirements	1044

A.4.2.46.2	Test Requirements	1048

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1048

A.4.2.47.1	Test Purpose and Environment	1048

A.4.2.47.2	Test Requirements	1052

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1052

A.4.2.48.1	Test Purpose and Environment	1052

A.4.2.48.2	Test Requirements	1054

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1055

A.4.2.49.1	Test Purpose and Environment	1055

A.4.2.49.2	Test Requirements	1057

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1058

A.4.2.50.1	Test Purpose and Environment	1058

A.4.2.50.2	Test Requirements	1060

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1061

A.4.2.51.1	Test Purpose and Environment	1061

A.4.2.51.2	Test Requirements	1063

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1064

A.4.2.52.1	Test Purpose and Environment	1064

A.4.2.52.2	Test Requirements	1066

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1067

A.4.2.53.1	Test Purpose and Environment	1067

A.4.2.53.2	Test Requirements	1069

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1070

A.4.2.54.1	Test Purpose and Environment	1070

A.4.2.54.2	Test Requirements	1072

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1073

A.4.2.55.1	Test Purpose and Environment	1073

A.4.2.55.2	Test Requirements	1075

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1076

A.4.2.56.1	Test Purpose and Environment	1076

A.4.2.56.2	Test Requirements	1077

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1078

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1078

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1078

A.4.3.1.1.1	Test Purpose and Environment	1078

A.4.3.1.1.2	Test Requirements	1080

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1080

A.4.3.1.2.1	Test Purpose and Environment	1080

A.4.3.1.2.2	Test Requirements	1083

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1083

A.4.3.1.3.1	Test Purpose and Environment	1083

A.4.3.1.3.2	Test Requirements	1086

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1086

A.4.3.1.4.1	Test Purpose and Environment	1086

A.4.3.1.4.2	Test Requirements	1087

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1087

A.4.3.1.5.1	Test Purpose and Environment	1087

A.4.3.1.5.2	Test Requirements	1090

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1091

A.4.3.2.1	Test Purpose and Environment	1091

A.4.3.2.1.1	Void	1091

A.4.3.2.1.2	1.28Mcps TDD option	1091

A.4.3.2.1.3	Void	1093

A.4.3.2.2	Test Requirements	1093

A.4.3.2.2.1	1.28Mcps TDD option	1093

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1093

A.4.3.2A.1	Test Purpose and Environment	1093

A.4.3.2A.2	Test Requirements	1097

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1097

A.4.3.3.1	Test Purpose and Environment	1097

A.4.3.3.2	Test Requirements	1100

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1100

A.4.3.3A.1	Test Purpose and Environment	1100

A.4.3.3A.2	Test Requirements	1105

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1106

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1106

A.4.3.4.1.1	Test Purpose and Environment	1106

A.4.3.4.1.2	Test Requirements	1108

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1108

A.4.3.4.2.1	Test Purpose and Environment	1108

A.4.3.4.2.2	Test Requirements	1110

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1110

A.4.3.4.3.1	Test Purpose and Environment	1110

A.4.3.4.3.2	Test Requirements	1113

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1113

A.4.3.4.4.1	Test Purpose and Environment	1113

A.4.3.4.4.2	Test Requirements	1117

A.4.4	E-UTRAN to GSM Cell Re-Selection	1117

A.4.4.1	E-UTRAN FDD – GSM:	1117

A.4.4.1.1	Test Purpose and Environment	1117

A.4.4.1.2	Test Requirements	1119

A.4.4.2	E-UTRAN TDD – GSM:	1119

A.4.4.2.1	Test Purpose and Environment	1119

A.4.4.2.2	Test Requirements	1121

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1122

A.4.5.1	E-UTRAN FDD – HRPD	1122

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1122

A.4.5.1.1.1	Test Purpose and Environment	1122

A.4.5.1.1.2	Test Requirements	1124

A.4.5.2	E-UTRAN TDD – HRPD	1124

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1124

A.4.5.2.1.1	Test Purpose and Environment	1124

A.4.5.2.1.2	Test Requirements	1127

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1127

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1127

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1127

A.4.6.1.1.1	Test Purpose and Environment	1127

A.4.6.1.1.2	Test Requirements	1130

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1130

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1130

A.4.6.2.1.1	Test Purpose and Environment	1130

A.4.6.2.1.2	Test Requirements	1133

A.4.7	Idle State Positioning Measurement for UE category NB1	1133

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1133

A.4.7.1.1	Test Purpose and Environment	1133

A.4.7.1.2	Test Requirements	1138

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1139

A.4.7.2.1	Test Purpose and Environment	1139

A.4.7.4.2	Test Requirements	1143

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1144

A.4.7.3.1	Test Purpose and Environment	1144

A.4.7.3.2	Test Requirements	1148

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1149

A.4.7.4.1	Test Purpose and Environment	1149

A.4.7.4.2	Test Requirements	1153

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1154

A.5.1	E-UTRAN Handover	1154

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1154

A.5.1.1.1	Test Purpose and Environment	1154

A.5.1.1.2	Test Requirements	1156

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1156

A.5.1.2.1	Test Purpose and Environment	1156

A.5.1.2.2	Test Requirements	1158

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1158

A.5.1.3.1	Test Purpose and Environment	1158

A.5.1.3.2	Test Requirements	1160

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1160

A.5.1.4.1	Test Purpose and Environment	1160

A.5.1.4.2	Test Requirements	1162

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1162

A.5.1.5.1	Test Purpose and Environment	1162

A.5.1.5.2	Test Requirements	1164

A.5.1.6	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1164

A.5.1.6.1	Test Purpose and Environment	1164

A.5.1.6.2	Test Requirements	1166

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1166

A.5.1.7.1	Test Purpose and Environment	1166

A.5.1.7.2	Test Requirements	1169

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1169

A.5.1.8.1	Test Purpose and Environment	1169

A.5.1.8.2	Test Requirements	1172

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1172

A.5.1.9.1	Test Purpose and Environment	1172

A.5.1.9.2	Test Requirements	1173

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1173

A.5.1.10.1	Test Purpose and Environment	1173

A.5.1.10.2	Test Requirements	1175

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1175

A.5.1.11.1	Test Purpose and Environment	1175

A.5.1.11.2	Test Requirements	1177

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1177

A.5.1.12.1	Test Purpose and Environment	1177

A.5.1.12.2	Test Requirements	1179

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1179

A.5.1.13.1	Test Purpose and Environment	1179

A.5.1.13.2	Test Requirements	1181

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1182

A.5.1.14.1	Test Purpose and Environment	1182

A.5.1.14.2	Test Requirements	1183

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1184

A.5.1.15.1	Test Purpose and Environment	1184

A.5.1.15.2	Test Requirements	1185

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1186

A.5.1.16.1	Test Purpose and Environment	1186

A.5.1.16.2	Test Requirements	1187

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1188

A.5.1.17.1	Test Purpose and Environment	1188

A.5.1.17.2	Test Requirements	1189

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1190

A.5.1.18.1	Test Purpose and Environment	1190

A.5.1.18.2	Test Requirements	1191

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1192

A.5.1.19.1	Test Purpose and Environment	1192

A.5.1.19.2	Test Requirements	1193

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1194

A.5.1.20.1	Test Purpose and Environment	1194

A.5.1.20.2	Test Requirements	1195

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1196

A.5.1.21.1	Test Purpose and Environment	1196

A.5.1.21.2	Test Requirements	1197

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1197

A.5.1.22.1	Test Purpose and Environment	1197

A.5.1.22.2	Test Requirements	1199

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1199

A.5.1.23.1	Test Purpose and Environment	1199

A.5.1.23.2	Test Requirements	1201

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1201

A.5.1.24.1	Test Purpose and Environment	1201

A.5.1.24.2	Test Requirements	1203

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1203

A.5.1.25.1	Test Purpose and Environment	1203

A.5.1.25.2	Test Requirements	1205

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1206

A.5.1.26.1	Test Purpose and Environment	1206

A.5.1.26.2	Test Requirements	1207

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1208

A.5.1.27.1	Test Purpose and Environment	1208

A.5.1.27.2	Test Requirements	1209

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1210

A.5.1.28.1	Test Purpose and Environment	1210

A.5.1.28.2	Test Requirements	1211

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1212

A.5.1.29.1	Test Purpose and Environment	1212

A.5.1.29.2	Test Requirements	1213

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1214

A.5.1.30.1	Test Purpose and Environment	1214

A.5.1.30.2	Test Requirements	1215

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1216

A.5.1.31.1	Test Purpose and Environment	1216

A.5.1.31.2	Test Requirements	1217

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1218

A.5.1.32.1	Test Purpose and Environment	1218

A.5.1.32.2	Test Requirements	1219

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1220

A.5.1.33.1	Test Purpose and Environment	1220

A.5.1.33.2	Test Requirements	1221

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1222

A.5.1.34.1	Test Purpose and Environment	1222

A.5.1.34.2	Test Requirements	1223

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1224

A.5.1.35.1	Test Purpose and Environment	1224

A.5.1.35.2	Test Requirements	1225

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1226

A.5.1.36.1	Test Purpose and Environment	1226

A.5.1.36.2	Test Requirements	1227

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1228

A.5.1.37.1	Test Purpose and Environment	1228

A.5.1.37.2	Test Requirements	1229

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1230

A.5.1.38.1	Test Purpose and Environment	1230

A.5.1.38.2	Test Requirements	1231

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1232

A.5.1.39.1	Test Purpose and Environment	1232

A.5.1.39.2	Test Requirements	1233

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1234

A.5.1.40.1	Test Purpose and Environment	1234

A.5.1.40.2	Test Requirements	1235

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1236

A.5.1.41.1	Test Purpose and Environment	1236

A.5.1.41.2	Test Requirements	1238

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1239

A.5.1.42.1	Test Purpose and Environment	1239

A.5.1.42.2	Test Requirements	1240

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1241

A.5.1.43.1	Test Purpose and Environment	1241

A.5.1.43.2	Test Requirements	1242

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1243

A.5.1.44.1	Test Purpose and Environment	1243

A.5.1.44.2	Test Requirements	1244

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1245

A.5.1.45.1	Test Purpose and Environment	1245

A.5.1.45.1	Test Requirements	1246

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1247

A.5.1.46.1	Test Purpose and Environment	1247

A.5.1.46.2	Test Requirements	1248

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1249

A.5.1.47.1	Test Purpose and Environment	1249

A.5.1.47.2	Test Requirements	1250

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1251

A.5.1.48.1	Test Purpose and Environment	1251

A.5.1.48.2	Test Requirements	1252

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1253

A.5.1.49.1	Test Purpose and Environment	1253

A.5.1.49.2	Test Requirements	1254

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1255

A.5.1.50.1	Test Purpose and Environment	1255

A.5.1.50.2	Test Requirements	1256

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1257

A.5.1.51.1	Test Purpose and Environment	1257

A.5.1.51.2	Test Requirements	1259

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1259

A.5.152.1	Test Purpose and Environment	1259

A.5.1.52.2	Test Requirements	1262

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1263

A.5.1.53.1	Test Purpose and Environment	1263

A.5.1.53.2	Test Requirements	1265

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1266

A.5.1.54.1	Test Purpose and Environment	1266

A.5.1.54.2	Test Requirements	1268

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1269

A.5.1.55.1	Test Purpose and Environment	1269

A.5.1.55.2	Test Requirements	1272

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1273

A.5.1.56.1	Test Purpose and Environment	1273

A.5.1.56.2	Test Requirements	1276

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1277

A.5.1.57.1	Test Purpose and Environment	1277

A.5.1.57.2	Test Requirements	1279

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1279

A.5.1.58.1	Test Purpose and Environment	1279

A.5.1.58.2	Test Requirements	1282

A.5.2	E-UTRAN Handover to other RATs	1282

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1282

A.5.2.1.1	Test Purpose and Environment	1282

A.5.2.1.2	Test Requirements	1284

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1285

A.5.2.2.1	Test Purpose and Environment	1285

A.5.2.2.2	Test Requirements	1288

A.5.2.3 E-UTRAN FDD- GSM Handover	1288

A.5.2.3.1	Test Purpose and Environment	1288

A.5.2.3.2	Test Requirements	1289

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1290

A.5.2.4.1	Test Purpose and Environment	1290

A.5.2.4.1.1	Void	1290

A.5.2.4.1.2	1.28 Mcps TDD option	1290

A.5.2.4.1.3	Void	1292

A.5.2.4.2	Test Requirements	1292

A.5.2.4.2.1	Void	1292

A.5.2.4.2.2	1.28 Mcps TDD option	1292

A.5.2.4.2.3	Void	1292

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1292

A.5.2.5.1	Test Purpose and Environment	1292

A.5.2.5.1.3	Void	1295

A.5.2.5.2	Test Requirements	1295

A.5.2.5.2.1	Void	1295

A.5.2.5.2.2	1.28 Mcps TDD option	1295

A.5.2.5.2.3	Void	1295

A.5.2.6	E-UTRAN TDD - GSM Handover	1295

A.5.2.6.1	Test Purpose and Environment	1295

A.5.2.6.2	Test Requirements	1297

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1298

A.5.2.7.1	Test Purpose and Environment	1298

A.5.2.7.2	Test Requirements	1300

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1300

A.5.2.8.1	Test Purpose and Environment	1300

A.5.2.8.2	Test Requirements	1301

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1302

A.5.2.9.1	Test Purpose and Environment	1302

A.5.2.9.2	Test Requirements	1303

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1304

A.5.2.10.1	Test Purpose and Environment	1304

A.5.2.10.2	Test Requirements	1306

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1306

A.5.2.10A.1	Test Purpose and Environment	1306

A.5.2.10A.2	Test Requirements	1309

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1309

A.5.2.10B.1	Test Purpose and Environment	1309

A.5.2.10B.2	Test Requirements	1312

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1312

A.5.2.11.1	Test Purpose and Environment	1312

A.5.2.11.2	Test Requirements	1312

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1313

A.5.3.1	E-UTRAN FDD – HRPD Handover	1313

A.5.3.1.1	Test Purpose and Environment	1313

A.5.3.1.2	Test Requirements	1315

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1315

A.5.3.2.1	Test Purpose and Environment	1315

A.5.3.2.2	Test Requirements	1318

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1318

A.5.3.3.1	Test Purpose and Environment	1318

A.5.3.3.2	Test Requirements	1320

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1321

A.5.3.4.1	Test Purpose and Environment	1321

A.5.3.4.2	Test Requirements	1322

A.5.3.5	E-UTRAN TDD – HRPD Handover	1323

A.5.3.5.1	Test Purpose and Environment	1323

A.5.3.5.2	Test Requirements	1325

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1326

A.5.3.6.1	Test Purpose and Environment	1326

A.5.3.6.2	Test Requirements	1329

A.6	RRC Connection Control	1329

A.6.1	RRC Re-establishment	1329

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1329

A.6.1.1.1	Test Purpose and Environment	1329

A.6.1.1.2	Test Requirements	1330

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1331

A.6.1.2.1	Test Purpose and Environment	1331

A.6.1.2.2	Test Requirements	1332

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1333

A.6.1.3.1	Test Purpose and Environment	1333

A.6.1.3.2	Test Requirements	1334

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1335

A.6.1.4.1	Test Purpose and Environment	1335

A.6.1.4.2	Test Requirements	1336

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1337

A.6.1.5.1	Test Purpose and Environment	1337

A.6.1.5.2	Test Requirements	1337

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1337

A.6.1.6.1	Test Purpose and Environment	1337

A.6.1.6.2	Test Requirements	1339

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1340

A.6.1.7.1	Test Purpose and Environment	1340

A.6.1.7.2	Test Requirements	1341

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1342

A.6.1.8.1	Test Purpose and Environment	1342

A.6.1.8.2	Test Requirements	1343

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1344

A.6.1.9.1	Test Purpose and Environment	1344

A.6.1.9.2	Test Requirements	1345

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1346

A.6.1.10.1	Test Purpose and Environment	1346

A.6.1.10.2	Test Requirements	1347

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.11.1	Test Purpose and Environment	1348

A.6.1.11.2	Test Requirements	1349

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1350

A.6.1.12.1	Test Purpose and Environment	1350

A.6.1.12.2	Test Requirements	1351

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1352

A.6.1.13.1	Test Purpose and Environment	1352

A.6.1.13.2	Test Requirements	1353

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.14.1	Test Purpose and Environment	1354

A.6.1.14.2	Test Requirements	1355

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1356

A.6.1.15.1	Test Purpose and Environment	1356

A.6.1.15.2	Test Requirements	1358

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1359

A.6.1.16.1	Test Purpose and Environment	1359

A.6.1.16.2	Test Requirements	1361

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1362

A.6.1.17.1	Test Purpose and Environment	1362

A.6.1.17.2	Test Requirements	1363

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1364

A.6.1.18.1	Test Purpose and Environment	1364

A.6.1.18.2	Test Requirements	1365

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1366

A.6.1.19.1	Test Purpose and Environment	1366

A.6.1.19.2	Test Requirements	1367

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1368

A.6.1.20.1	Test Purpose and Environment	1368

A.6.1.20.2	Test Requirements	1369

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1370

A.6.1.21.1	Test Purpose and Environment	1370

A.6.1.21.2	Test Requirements	1371

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1372

A.6.1.22.1	Test Purpose and Environment	1372

A.6.1.22.2	Test Requirements	1373

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1374

A.6.1.23.1	Test Purpose and Environment	1374

A.6.1.23.2	Test Requirements	1376

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1377

A.6.1.24.1	Test Purpose and Environment	1377

A.6.1.24.2	Test Requirements	1379

A.6.2	Random Access	1380

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1380

A.6.2.1.1	Test Purpose and Environment	1380

A.6.2.1.2	Test Requirements	1381

A.6.2.1.2.1	Random Access Response Reception	1381

A.6.2.1.2.2	No Random Access Response Reception	1381

A.6.2.1.2.3	Receiving a NACK on msg3	1381

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1382

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1382

A.6.2.1.2.6	Contention Resolution Timer expiry	1382

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1382

A.6.2.2.1	Test Purpose and Environment	1382

A.6.2.2.2	Test Requirements	1383

A.6.2.2.2.1	Random Access Response Reception	1384

A.6.2.2.2.2	No Random Access Response Reception	1384

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1384

A.6.2.3.1	Test Purpose and Environment	1384

A.6.2.3.2	Test Requirements	1386

A.6.2.3.2.1	Random Access Response Reception	1386

A.6.2.3.2.2	No Random Access Response reception	1386

A.6.2.3.2.3	Receiving a NACK on msg3	1386

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1387

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1387

A.6.2.3.2.6	Contention Resolution Timer expiry	1387

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1387

A.6.2.4.1	Test Purpose and Environment	1387

A.6.2.4.2	Test Requirements	1389

A.6.2.4.2.1	Random Access Response Reception	1389

A.6.2.4.2.2	No Random Access Response Reception	1389

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1389

A.6.2.5.1	Test Purpose and Environment	1389

A.6.2.5.2	Test Requirements	1390

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1390

A.6.2.6.1	Test Purpose and Environment	1390

A.6.2.6.2	Test Requirements	1390

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1391

A.6.2.7.1	Test Purpose and Environment	1391

A.6.2.7.2	Test Requirements	1393

A.6.2.7.2.1	Random Access Response Reception	1393

A.6.2.7.2.2	No Random Access Response Reception	1393

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1393

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1394

A.6.2.8.1	Test Purpose and Environment	1394

A.6.2.8.2	Test Requirements	1396

A.6.2.8.2.1	Random Access Response Reception	1396

A.6.2.8.2.2	No Random Access Response Reception	1396

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1396

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1397

A.6.2.9.1	Test Purpose and Environment	1397

A.6.2.9.2	Test Requirements	1400

A.6.2.9.2.1	Random Access Response Reception	1400

A.6.2.9.2.2	No Random Access Response Reception	1401

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1401

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1402

A.6.2.10.1	Test Purpose and Environment	1402

A.6.2.10.2	Test Requirements	1404

A.6.2.10.2.1	Random Access Response Reception	1404

A.6.2.10.2.2	No Random Access Response Reception	1405

A.6.2.10.2.3	Receiving a NACK on msg3	1405

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1405

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1405

A.6.2.10.2.6	Contention Resolution Timer expiry	1405

A.6.2.10.2.7	PRACH Resource Selection	1406

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1406

A.6.2.11.1	Test Purpose and Environment	1406

A.6.2.11.2	Test Requirements	1408

A.6.2.11.2.1	Random Access Response Reception	1408

A.6.2.11.2.2	No Random Access Response Reception	1409

A.6.2.11.2.3	Receiving a NACK on msg3	1409

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1409

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1409

A.6.2.11.2.6	Contention Resolution Timer expiry	1409

A.6.2.11.2.7	PRACH Resource Selection	1410

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1410

A.6.2.12.1	Test Purpose and Environment	1410

A.6.2.12.2	Test Requirements	1412

A.6.2.12.2.1	Random Access Response Reception	1412

A.6.2.12.2.2	No Random Access Response Reception	1413

A.6.2.12.2.3	Receiving a NACK on msg3	1413

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1413

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1413

A.6.2.12.2.6	Contention Resolution Timer expiry	1413

A.6.2.12.2.7	PRACH Resource Selection	1414

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1414

A.6.2.13.1	Test Purpose and Environment	1414

A.6.2.13.2	Test Requirements	1416

A.6.2.13.2.1	Random Access Response Reception	1416

A.6.2.13.2.2	No Random Access Response Reception	1417

A.6.2.13.2.3	Receiving a NACK on msg3	1417

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.13.2.6	Contention Resolution Timer expiry	1417

A.6.2.13.2.7	PRACH Resource Selection	1418

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1418

A.6.2.14.1	Test Purpose and Environment	1418

A.6.2.14.2	Test Requirements	1420

A.6.2.14.2.1	Random Access Response Reception	1420

A.6.2.14.2.2	No Random Access Response Reception	1421

A.6.2.14.2.3	Receiving a NACK on msg3	1421

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1421

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1421

A.6.2.14.2.6	Contention Resolution Timer expiry	1421

A.6.2.14.2.7	PRACH Resource Selection	1422

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1422

A.6.2.15.1	Test Purpose and Environment	1422

A.6.2.15.2	Test Requirements	1424

A.6.2.15.2.1	Random Access Response Reception	1424

A.6.2.15.2.2	No Random Access Response Reception	1425

A.6.2.15.2.3	Receiving a NACK on msg3	1425

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1425

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1425

A.6.2.15.2.6	Contention Resolution Timer expiry	1425

A.6.2.15.2.7	PRACH Resource Selection	1426

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1426

A.6.2.16.1	Test Purpose and Environment	1426

A.6.2.16.2	Test Requirements	1429

A.6.2.16.2.1	Random Access Response Reception	1429

A.6.2.16.2.2	No Random Access Response Reception	1430

A.6.2.16.2.3	Receiving a NACK on msg3	1430

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.16.2.6	Contention Resolution Timer expiry	1430

A.6.2.16.2.7	NPRACH Resource Selection	1430

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.17.1	Test Purpose and Environment	1431

A.6.2.17.2	Test Requirements	1434

A.6.2.17.2.1	Random Access Response Reception	1434

A.6.2.17.2.2	No Random Access Response Reception	1435

A.6.2.17.2.3	Receiving a NACK on msg3	1435

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1435

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1435

A.6.2.17.2.6	Contention Resolution Timer expiry	1435

A.6.2.17.2.7	NPRACH Resource Selection	1435

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1436

A.6.2.18.1	Test Purpose and Environment	1436

A.6.2.18.2	Test Requirements	1439

A.6.2.18.2.1	Random Access Response Reception	1439

A.6.2.18.2.2	No Random Access Response Reception	1440

A.6.2.18.2.3	Receiving a NACK on msg3	1440

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1440

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1440

A.6.2.18.2.6	Contention Resolution Timer expiry	1440

A.6.2.18.2.7	NPRACH Resource Selection	1440

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1441

A.6.2.19.1	Test Purpose and Environment	1441

A.6.2.19.2	Test Requirements	1443

A.6.2.19.2.1	Random Access Response Reception	1443

A.6.2.19.2.2	No Random Access Response Reception	1444

A.6.2.19.2.3	Receiving a NACK on msg3	1444

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1444

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1444

A.6.2.19.2.6	Contention Resolution Timer expiry	1444

A.6.2.19.2.7	NPRACH Resource Selection	1444

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1445

A.6.2.20.1	Test Purpose and Environment	1445

A.6.2.20.2	Test Requirements	1447

A.6.2.20.2.1	Random Access Response Reception	1447

A.6.2.20.2.2	No Random Access Response Reception	1448

A.6.2.20.2.3	Receiving a NACK on msg3	1448

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1448

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1448

A.6.2.20.2.6	Contention Resolution Timer expiry	1448

A.6.2.20.2.7	NPRACH Resource Selection	1448

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1449

A.6.2.21.1	Test Purpose and Environment	1449

A.6.2.21.2	Test Requirements	1451

A.6.2.21.2.1	Random Access Response Reception	1451

A.6.2.21.2.2	No Random Access Response Reception	1452

A.6.2.21.2.3	Receiving a NACK on msg3	1452

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1452

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1452

A.6.2.21.2.6	Contention Resolution Timer expiry	1452

A.6.2.21.2.7	NPRACH Resource Selection	1452

A.6.3	RRC Connection Release with Redirection	1453

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1453

A.6.3.1.1	Test Purpose and Environment	1453

A.6.3.1.2	Test Requirements	1455

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1455

A.6.3.2.1	Test Purpose and Environment	1455

A.6.3.2.2	Test Requirements	1457

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1457

A.6.3.3.1	Test Purpose and Environment	1457

A.6.3.3.2	Test Requirements	1458

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1459

A.6.3.4.1	Test Purpose and Environment	1459

A.6.3.4.2	Test Requirements	1460

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1461

A.6.3.5.1	Test Purpose and Environment	1461

A.6.3.5.2	Test Requirements	1463

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1463

A.6.3.6.1	Test Purpose and Environment	1463

A. 6.3.6.2	Test Requirements	1466

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1466

A.6.3.7.1	Test Purpose and Environment	1466

A.6.3.7.2	Test Requirements	1469

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1469

A.6.3.8.1	Test Purpose and Environment	1469

A.6.3.8.2	Test Requirements	1472

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1472

A.6.3.9.1	Test Purpose and Environment	1472

A.6.3.9.2	Test Requirements	1474

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1474

A.6.3.10.1	Test Purpose and Environment	1474

A.6.3.10.2	Test Requirements	1476

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1476

A.6.3.11.1	Test Purpose and Environment	1476

A.6.3.11.2	Test Requirements	1478

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1478

A.6.3.12.1	Test Purpose and Environment	1478

A.6.3.12.2	Test Requirements	1481

A.7	Timing and Signalling Characteristics	1482

A.7.1	UE Transmit Timing	1482

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1482

A.7.1.1.1	Test Purpose and Environment	1482

A.7.1.1.2	Test Requirements	1483

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1484

A.7.1.2.1	Test Purpose and Environment	1484

A.7.1.2.2	Test Requirements	1486

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1487

A.7.1.3.1	Test Purpose and Environment	1487

A.7.1.3.2	Test Requirements	1489

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1490

A.7.1.4.1	Test Purpose and Environment	1490

A.7.1.4.2	Test Requirements	1491

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1492

A.7.1.4A.1	Test Purpose and Environment	1492

A.7.1.4A.2	Test Requirements	1492

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1492

A.7.1.5.1	Test Purpose and Environment	1492

A.7.1.5.2	Test Requirements	1493

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1493

A.7.1.6.1	Test Purpose and Environment	1493

A.7.1.6.2	Test Requirements	1495

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1496

A.7.1.7.1	Test Purpose and Environment	1496

A.7.1.7.2	Test Requirements	1497

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1498

A.7.1.7A.1	Test Purpose and Environment	1498

A.7.1.7A.2	Test Requirements	1498

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1498

A.7.1.7B.1	Test Purpose and Environment	1498

A.7.1.7B.2	Test Requirements	1498

A.7.1.8	Void	1499

A.7.1.8.1	Void	1499

A.7.1.8.2	Void	1499

A.7.1.9	Void	1499

A.7.1.9.1	Void	1499

A.7.1.9.2	Void	1499

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1499

A.7.1.10.1	Test Purpose and Environment	1499

A.7.1.10.2	Test Requirements	1501

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1501

A.7.1.11.1	Test Purpose and Environment	1501

A.7.1.11.2	Test Requirements	1503

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1503

A.7.1.12.1	Test Purpose and Environment	1503

A.7.1.12.2	Test Requirements	1505

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1505

A.7.1.13.1	Test Purpose and Environment	1505

A.7.1.13.2	Test Requirements	1507

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1508

A.7.1.14.1	Test Purpose and Environment	1508

A.7.1.14.2	Test Requirements	1509

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1509

A.7.1.15.1	Test Purpose and Environment	1509

A.7.1.15.2	Test Requirements	1510

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1511

A.7.1.16.1	Test Purpose and Environment	1511

A.7.1.16.2	Test Requirements	1512

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1513

A.7.1.17.1	Test Purpose and Environment	1513

A.7.1.17.2	Test Requirements	1515

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1516

A.7.1.18.1	Test Purpose and Environment	1516

A.7.1.18.2	Test Requirements	1518

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1519

A.7.1.19.1	Test Purpose and Environment	1519

A.7.1.19.2	Test Requirements	1520

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1520

A.7.1.20.1	Test Purpose and Environment	1520

A.7.1.20.2	Test Requirements	1522

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1522

A.7.1.21.1	Test Purpose and Environment	1522

A.7.1.21.2	Test Requirements	1524

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1524

A.7.1.22.1	Test Purpose and Environment	1524

A.7.1.22.2	Test Requirements	1526

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1526

A.7.1.23.1	Test Purpose and Environment	1526

A.7.1.23.2	Test Requirements	1528

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1528

A.7.1.24.1	Test Purpose and Environment	1528

A.7.1.24.2	Test Requirements	1529

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1530

A.7.1.25.1	Test Purpose and Environment	1530

A.7.1.25.2	Test Requirements	1531

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1532

A.7.1.26.1	Test Purpose and Environment	1532

A.7.1.26.2	Test Requirements	1533

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1534

A.7.1.27.1	Test Purpose and Environment	1534

A.7.1.27.2	Test Requirements	1536

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1536

A.7.1.28.1	Test Purpose and Environment	1536

A.7.1.28.2	Test Requirements	1538

A.7.2	UE Timing Advance	1539

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1539

A.7.2.1.1	Test Purpose and Environment	1539

A.7.2.1.2	Test Requirements	1540

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1541

A.7.2.2.1	Test Purpose and Environment	1541

A.7.2.2.2	Test Requirements	1543

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1543

A.7.2.3.1	Test Purpose and Environment	1543

A.7.2.3.2	Test Requirements	1543

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1543

A.7.2.4.1	Test Purpose and Environment	1543

A.7.2.4.2	Test Requirements	1546

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1546

A.7.2.5.1	Test Purpose and Environment	1546

A.7.2.5.2	Test Requirements	1548

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1548

A.7.2.5A.1	Test Purpose and Environment	1548

A.7.2.5A.2	Test Requirements	1549

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1549

A.7.2.5B.1	Test Purpose and Environment	1549

A.7.2.5B.2	Test Requirements	1549

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1549

A.7.2.6.1	Test Purpose and Environment	1549

A.7.2.6.2	Test Requirements	1552

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1552

A.7.2.7.1	Test Purpose and Environment	1552

A.7.2.7.2	Test Requirements	1554

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1554

A.7.2.8.1	Test Purpose and Environment	1554

A.7.2.8.2	Test Requirements	1556

A.7.2.9.2	Test Requirements	1557

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1558

A.7.2.10.1	Test Purpose and Environment	1558

A.7.2.10.2	Test Requirements	1559

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1559

A.7.2.11.1	Test Purpose and Environment	1559

A.7.2.11.2	Test Requirements	1561

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1561

A.7.2.12.1	Test Purpose and Environment	1561

A.7.2.12.2	Test Requirements	1563

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1564

A.7.2.13.1	Test Purpose and Environment	1564

A.7.2.13.2	Test Requirements	1565

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1566

A.7.2.14.1	Test Purpose and Environment	1566

A.7.2.14.2	Test Requirements	1568

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1568

A.7.2.15.1	Test Purpose and Environment	1568

A.7.2.15.2	Test Requirements	1570

A.7.3	Radio Link Monitoring	1570

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1570

A.7.3.1.1	Test Purpose and Environment	1570

A.7.3.1.2	Test Requirements	1574

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1574

A.7.3.2.1	Test Purpose and Environment	1574

A.7.3.2.2	Test Requirements	1578

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1578

A.7.3.3.1	Test Purpose and Environment	1578

A.7.3.3.2	Test Requirements	1582

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1582

A.7.3.4.1	Test Purpose and Environment	1582

A.7.3.4.2	Test Requirements	1586

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1586

A.7.3.5.1	Test Purpose and Environment	1586

A.7.3.5.2	Test Requirements	1589

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1590

A.7.3.6.1	Test Purpose and Environment	1590

A.7.3.6.2	Test Requirements	1593

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1593

A.7.3.7.1	Test Purpose and Environment	1593

A.7.3.7.2	Test Requirements	1596

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1596

A.7.3.8.1	Test Purpose and Environment	1596

A.7.3.8.2	Test Requirements	1599

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1599

A.7.3.9.2	Test Requirements	1602

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1602

A.7.3.10.1	Test Purpose and Environment	1602

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1606

A.7.3.11.1	Test Purpose and Environment	1606

A.7.3.11.2	Test Requirements	1611

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1611

A.7.3.12.1	Test Purpose and Environment	1611

A.7.3.12.2	Test Requirements	1616

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1616

A.7.3.13.1	Test Purpose and Environment	1616

A.7.3.13.2	Test Requirements	1619

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1619

A.7.3.14.1	Test Purpose and Environment	1619

A.7.3.14.2	Test Requirements	1622

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1622

A.7.3.15.1	Test Purpose and Environment	1622

A.7.3.15.2	Test Requirements	1626

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1626

A.7.3.16.1	Test Purpose and Environment	1626

A.7.3.16.2	Test Requirements	1631

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1631

A.7.3.17.1	Test Purpose and Environment	1631

A.7.3.17.2	Test Requirements	1635

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1635

A.7.3.18.1	Test Purpose and Environment	1635

A.7.3.18.2	Test Requirements	1639

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1639

A.7.3.19.1	Test Purpose and Environment	1639

A.7.3.19.2	Test Requirements	1644

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1644

A.7.3.20.1	Test Purpose and Environment	1644

A.7.3.20.2	Test Requirements	1649

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1649

A.7.3.21.1	Test Purpose and Environment	1649

A.7.3.21.2	Test Requirements	1654

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1654

A.7.3.22.1	Test Purpose and Environment	1654

A.7.3.22.2	Test Requirements	1659

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1659

A.7.3.23.1	Test Purpose and Environment	1659

A.7.3.23.2	Test Requirements	1660

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1660

A.7.3.24.1	Test Purpose and Environment	1660

A.7.3.24.2	Test Requirements	1661

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1661

A.7.3.25.1	Test Purpose and Environment	1661

A.7.3.25.2	Test Requirements	1662

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1662

A.7.3.26.1	Test Purpose and Environment	1662

A.7.3.26.2	Test Requirements	1665

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1665

A.7.3.27.1	Test Purpose and Environment	1665

A.7.3.27.2	Test Requirements	1668

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1668

A.7.3.28.1	Test Purpose and Environment	1668

A.7.3.28.2	Test Requirements	1671

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1671

A.7.3.29.1	Test Purpose and Environment	1671

A.7.3.29.2	Test Requirements	1674

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1674

A.7.3.30.1	Test Purpose and Environment	1674

A.7.3.30.2	Test Requirements	1677

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1677

A.7.3.31.1	Test Purpose and Environment	1677

A.7.3.31.2	Test Requirements	1680

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1680

A.7.3.32.1	Test Purpose and Environment	1680

A.7.3.32.2	Test Requirements	1683

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1683

A.7.3.33.1	Test Purpose and Environment	1683

A.7.3.33.2	Test Requirements	1686

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1686

A.7.3.34.1	Test Purpose and Environment	1686

A.7.3.34.2	Test Requirements	1689

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1689

A.7.3.35.1	Test Purpose and Environment	1689

A.7.3.35.2	Test Requirements	1692

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1692

A.7.3.36.1	Test Purpose and Environment	1692

A.7.3.36.2	Test Requirements	1695

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1695

A.7.3.37.1	Test Purpose and Environment	1695

A.7.3.37.2	Test Requirements	1698

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1698

A.7.3.38.1	Test Purpose and Environment	1698

A.7.3.38.2	Test Requirements	1701

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1701

A.7.3.39.1	Test Purpose and Environment	1701

A.7.3.39.2	Test Requirements	1704

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1705

A.7.3.40.1	Test Purpose and Environment	1705

A.7.3.40.2	Test Requirements	1707

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1708

A.7.3.41.1	Test Purpose and Environment	1708

A.7.3.41.2	Test Requirements	1711

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1711

A.7.3.42.1	Test Purpose and Environment	1711

A.7.3.42.2	Test Requirements	1714

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1714

A.7.3.43.1	Test Purpose and Environment	1714

A.7.3.43.2	Test Requirements	1717

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1717

A.7.3.44.1	Test Purpose and Environment	1717

A.7.3.44.2	Test Requirements	1720

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1721

A.7.3.45.1	Test Purpose and Environment	1721

A.7.3.45.2	Test Requirements	1723

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1724

A.7.3.46.1	Test Purpose and Environment	1724

A.7.3.46.2	Test Requirements	1727

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1727

A.7.3.47.1	Test Purpose and Environment	1727

A.7.3.47.2	Test Requirements	1730

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1730

A.7.3.48.1	Test Purpose and Environment	1730

A.7.3.48.2	Test Requirements	1733

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1733

A.7.3.49.1	Test Purpose and Environment	1733

A.7.3.49.2	Test Requirements	1736

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1736

A.7.3.50.1	Test Purpose and Environment	1736

A.7.3.50.2	Test Requirements	1739

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1739

A.7.3.51.1	Test Purpose and Environment	1739

A.7.3.51.2	Test Requirements	1742

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1742

A.7.3.52.1	Test Purpose and Environment	1742

A.7.3.52.2	Test Requirements	1745

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1745

A.7.3.53.1	Test Purpose and Environment	1745

A.7.3.53.2	Test Requirements	1748

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1748

A.7.3.54.1	Test Purpose and Environment	1748

A.7.3.54.2	Test Requirements	1751

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1751

A.7.3.55.1	Test Purpose and Environment	1751

A.7.3.55.2	Test Requirements	1754

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1754

A.7.3.56.1	Test Purpose and Environment	1754

A.7.3.56.2	Test Requirements	1757

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1757

A.7.3.57.1	Test Purpose and Environment	1757

A.7.3.57.2	Test Requirements	1760

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1760

A.7.3.58.1	Test Purpose and Environment	1760

A.7.3.58.2	Test Requirements	1763

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1763

A.7.3.59.1	Test Purpose and Environment	1763

A.7.3.59.2	Test Requirements	1766

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1766

A.7.3.60.1	Test Purpose and Environment	1766

A.7.3.60.2	Test Requirements	1770

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1770

A.7.3.61.1	Test Purpose and Environment	1770

A.7.3.61.2	Test Requirements	1774

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1774

A.7.3.62.1	Test Purpose and Environment	1774

A.7.3.62.2	Test Requirements	1778

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1778

A.7.3.63.1	Test Purpose and Environment	1778

A.7.3.63.2	Test Requirements	1782

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1782

A.7.3.64.1	Test Purpose and Environment	1782

A.7.3.64.2	Test Requirements	1785

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1786

A.7.3.65.1	Test Purpose and Environment	1786

A.7.3.65.2	Test Requirements	1789

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1790

A.7.3.66.1	Test Purpose and Environment	1790

A.7.3.66.2	Test Requirements	1792

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1793

A.7.3.67.1	Test Purpose and Environment	1793

A.7.3.67.2	Test Requirements	1796

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1797

A.7.3.68.1	Test Purpose and Environment	1797

A.7.3.68.2	Test Requirements	1799

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1799

A.7.3.69.1	Test Purpose and Environment	1799

A.7.3.69.2	Test Requirements	1801

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1801

A.7.3.70.1	Test Purpose and Environment	1801

A.7.3.70.2	Test Requirements	1803

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1803

A.7.3.71.1	Test Purpose and Environment	1803

A.7.3.71.2	Test Requirements	1806

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1806

A.7.3.72.1	Test Purpose and Environment	1806

A.7.3.72.2	Test Requirements	1808

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1808

A.7.3.73.1	Test Purpose and Environment	1808

A.7.3.73.2	Test Requirements	1810

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1810

A.7.3.74.1	Test Purpose and Environment	1810

A.7.3.74.2	Test Requirements	1813

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1813

A.7.3.75.1	Test Purpose and Environment	1813

A.7.3.75.2	Test Requirements	1815

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1816

A.7.3.76.1	Test Purpose and Environment	1816

A.7.3.76.2	Test Requirements	1819

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1819

A.7.3.77.1	Test Purpose and Environment	1819

A.7.3.77.2	Test Requirements	1822

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1822

A.7.3.78.1	Test Purpose and Environment	1822

A.7.3.78.2	Test Requirements	1825

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1825

A.7.3.79.1	Test Purpose and Environment	1825

A.7.3.79.2	Test Requirements	1828

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1828

A.7.3.80.1	Test Purpose and Environment	1828

A.7.3.80.2	Test Requirements	1831

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1831

A.7.3.81.1	Test Purpose and Environment	1831

A.7.3.81.2	Test Requirements	1834

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1834

A.7.3.82.1	Test Purpose and Environment	1834

A.7.3.82.2	Test Requirements	1837

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1837

A.7.3.83.1	Test Purpose and Environment	1837

A.7.3.83.2	Test Requirements	1838

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1839

A.7.3.84.1	Test Purpose and Environment	1839

A.7.3.84.2	Test Requirements	1841

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1841

A.7.3.85.1	Test Purpose and Environment	1841

A.7.3.85.2	Test Requirements	1842

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1843

A.7.3.86.1	Test Purpose and Environment	1843

A.7.3.86.2	Test Requirements	1845

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1845

A.7.3.87.1	Test Purpose and Environment	1845

A.7.3.87.2	Test Requirements	1847

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1848

A.7.3.88.1	Test Purpose and Environment	1848

A.7.3.88.2	Test Requirements	1852

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1852

A.7.3.89.1	Test Purpose and Environment	1852

A.7.3.89.2	Test Requirements	1856

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1856

A.7.3.90.1	Test Purpose and Environment	1856

A.7.3.90.2	Test Requirements	1860

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1860

A.7.3.91.1	Test Purpose and Environment	1860

A.7.3.91.2	Test Requirements	1864

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1864

A.7.3.92.1	Test Purpose and Environment	1864

A.7.3.92.2	Test Requirements	1867

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1868

A.7.3.93.1	Test Purpose and Environment	1868

A.7.3.93.2	Test Requirements	1871

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1872

A.7.3.94.1	Test Purpose and Environment	1872

A.7.3.94.2	Test Requirements	1874

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1875

A.7.3.95.1	Test Purpose and Environment	1875

A.7.3.95.2	Test Requirements	1878

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1878

A.7.3.96.1	Test Purpose and Environment	1878

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1880

A.7.3.97.1	Test Purpose and Environment	1880

A.7.3.97.2	Test Requirements	1883

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1883

A.7.3.98.1	Test Purpose and Environment	1883

A.7.3.98.2	Test Requirements	1886

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1886

A.7.3.99.1	Test Purpose and Environment	1886

A.7.3.99.2	Test Requirements	1889

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1889

A.7.3.100.1	Test Purpose and Environment	1889

A.7.3.100.2	Test Requirements	1891

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1891

A.7.3.101.1	Test Purpose and Environment	1891

A.7.3.101.2	Test Requirements	1893

A.7.4	Interruption for Dual Connectivity	1893

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1893

A.7.4.1.1	Test Purpose and Environment	1893

A.7.4.1.2	Test Requirements	1896

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1896

A.7.4.2.1	Test Purpose and Environment	1896

A.7.4.2.2	Test Requirements	1898

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1898

A.7.4.3.1	Test Purpose and Environment	1898

A.7.4.3.2	Test Requirements	1900

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1900

A.7.4.4.1	Test Purpose and Environment	1900

A.7.4.4.2	Test Requirements	1903

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1903

A.7.4.5.1	Test Purpose and Environment	1903

A.7.4.5.2	Test Requirements	1905

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1905

A.7.4.6.1	Test Purpose and Environment	1905

A.7.4.6.2	Test Requirements	1907

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1907

A.7.4.7.1	Test Purpose and Environment	1907

A.7.4.7.2	Test Requirements	1910

A.7.5	Proximity-based Services	1910

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1910

A.7.5.1.1	Test Purpose and Environment	1910

A.7.5.1.2	Test Requirements	1911

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1912

A.7.5.2.1	Test Purpose and Environment	1912

A.7.5.1.2	Test Requirements	1912

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1913

A.7.5.3.1	Test Purpose and Environment	1913

A.7.5.3.2	Test Requirements	1914

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1915

A.7.5.4.1	Test Purpose and Environment	1915

A.7.5.4.2	Test Requirements	1916

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1917

A.7.5.5.1	Test Purpose and Environment	1917

A.7.5.5.2	Test Requirements	1919

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1920

A.7.5.6.1	Test Purpose and Environment	1920

A.7.5.6.2	Test Requirements	1921

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1922

A.7.5.7.1	Test Purpose and Environment	1922

A.7.5.7.2	Test Requirements	1924

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1924

A.7.5.8.1	Test Purpose and Environment	1924

A.7.5.8.2	Test Requirements	1926

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1927

A.7.5.9.1	Test Purpose and Environment	1927

A.7.5.9.2	Test Requirements	1929

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1930

A.7.5.10.1	Test Purpose and Environment	1930

A.7.5.10.2	Test Requirements	1932

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1933

A.7.5.11.1	Test Purpose and Environment	1933

A.7.5.11.2	Test Requirements	1935

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1935

A.7.5.12.1	Test Purpose and Environment	1935

A.7.5.12.2	Test Requirements	1939

A.7.6	Interruption for carrier aggregation	1940

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1940

A.7.6.1.1	Test Purpose and Environment	1940

A.7.6.1.2	Test Requirements	1943

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1943

A.7.6.2.1	Test Purpose and Environment	1943

A.7.6.2.2	Test Requirements	1946

A.8	UE Measurements Procedures	1946

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1946

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1946

A.8.1.1.1	Test Purpose and Environment	1946

A.8.1.1.2	Test Requirements	1948

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1948

A.8.1.2.1	Test Purpose and Environment	1948

A.8.1.2.2	Test Requirements	1950

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1950

A.8.1.3.1	Test Purpose and Environment	1950

A.8.1.3.2	Test Requirements	1953

A.8.1.4	Void	1953

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1953

A.8.1.5.1	Test Purpose and Environment	1953

A.8.1.5.2	Test Requirements	1955

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1955

A.8.1.6.1	Test Purpose and Environment	1955

A.8.1.6.2	Test Requirements	1957

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1957

A.8.1.7.1	Test Purpose and Environment	1957

A.8.1.7.2	Test Requirements	1959

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1960

A.8.1.8.1	Test Purpose and Environment	1960

A.8.1.8.2	Test Requirements	1962

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1963

A.8.1.9.1	Test Purpose and Environment	1963

A.8.1.9.2	Test Requirements	1963

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1963

A.8.1.10.1	Test Purpose and Environment	1963

A.8.1.10.2	Test Requirements	1964

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1964

A.8.1.11.1	Test Purpose and Environment	1964

A.8.1.11.2	Test Requirements	1966

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1967

A.8.1.12.1	Test Purpose and Environment	1967

A.8.1.12.2	Test Requirements	1968

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1969

A.8.1.13.1	Test Purpose and Environment	1969

A.8.1.13.2	Test Requirements	1971

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1971

A.8.1.14.1	Test Purpose and Environment	1971

A.8.1.14.2	Test Requirements	1973

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1974

A.8.1.15.1	Test Purpose and Environment	1974

A.8.1.15.2	Test Requirements	1975

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1976

A.8.1.16.1	Test Purpose and Environment	1976

A.8.1.16.2	Test Requirements	1978

A.8.1.17	Void	1978

A.8.1.18	Void	1978

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1978

A.8.1.19.1	Test Purpose and Environment	1978

A.8.1.19.2	Test Requirements	1980

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1981

A.8.1.20.1	Test Purpose and Environment	1981

A.8.1.20.2	Test Requirements	1983

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1983

A.8.1.21.1	Test Purpose and Environment	1983

A.8.1.21.2	Test Requirements	1985

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1986

A.8.1.22.1	Test Purpose and Environment	1986

A.8.1.22.2	Test Requirements	1988

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1988

A.8.1.23.1	Test Purpose and Environment	1988

A.8.1.23.2	Test Requirements	1989

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1990

A.8.1.24.1	Test Purpose and Environment	1990

A.8.1.24.2	Test Requirements	1992

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1992

A.8.1.25.1	Test Purpose and Environment	1992

A.8.1.25.2	Test Requirements	1994

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1994

A.8.1.26.1	Test Purpose and Environment	1994

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1997

A.8.1.27.1	Test Purpose and Environment	1997

A.8.1.27.2	Test Requirements	1999

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1999

A.8.1.28.1	Test Purpose and Environment	1999

A.8.1.28.2	Test Requirements	2001

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2001

A.8.1.29.1	Test Purpose and Environment	2001

A.8.1.29.2	Test Requirements	2003

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2004

A.8.1.30.1	Test Purpose and Environment	2004

A.8.1.30.2	Test Requirements	2006

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2006

A.8.1.31.1	Test Purpose and Environment	2006

A.8.1.31.2	Test Requirements	2008

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2009

A.8.1.32.1	Test Purpose and Environment	2009

A.8.1.32.2	Test Requirements	2010

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2011

A.8.1.33.1	Test Purpose and Environment	2011

A.8.1.33.2	Test Requirements	2012

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2013

A.8.1.34.1	Test Purpose and Environment	2013

A.8.1.34.2	Test Requirements	2014

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2015

A.8.1.35.1	Test Purpose and Environment	2015

A.8.1.35.2	Test Requirements	2016

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2017

A.8.1.36.1	Test Purpose and Environment	2017

A.8.1.36.2	Test Requirements	2018

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2019

A.8.1.37.1	Test Purpose and Environment	2019

A.8.1.37.2	Test Requirements	2021

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2021

A.8.1.38.1	Test Purpose and Environment	2021

A.8.1.38.2	Test Requirements	2022

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2023

A.8.1.39.1	Test Purpose and Environment	2023

A.8.1.39.2	Test Requirements	2025

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2025

A.8.1.40.1	Test Purpose and Environment	2025

A.8.1.40.2	Test Requirements	2027

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2027

A.8.1.41.1	Test Purpose and Environment	2027

A.8.1.41.2	Test Requirement	2029

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2029

A.8.1.42.1	Test Purpose and Environment	2029

A.8.1.42.2	Test Requirement	2030

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2030

A.8.1.43.1	Test Purpose and Environment	2030

A.8.1.43.2	Test Requirements	2033

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2033

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2033

A.8.2.1.1	Test Purpose and Environment	2033

A.8.2.1.2	Test Requirements	2035

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2036

A.8.2.2.1	Test Purpose and Environment	2036

A.8.2.2.2	Test Requirements	2038

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2038

A.8.2.3.1	Test Purpose and Environment	2038

A.8.2.3.2	Test Requirements	2040

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2041

A.8.2.4.1	Test Purpose and Environment	2041

A.8.2.4.2	Test Requirements	2043

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2043

A.8.2.5.1	Test Purpose and Environment	2043

A.8.2.5.2	Test Requirements	2045

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2046

A.8.2.6.1	Test Purpose and Environment	2046

A.8.2.6.2	Test Requirements	2049

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2050

A.8.2.7.1	Test Purpose and Environment	2050

A.8.2.7.2	Test Requirements	2051

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2052

A.8.2.8.1	Test Purpose and Environment	2052

A.8.2.8.2	Test Requirements	2054

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2054

A.8.2.9.1	Test Purpose and Environment	2054

A.8.2.9.2	Test Requirements	2056

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2057

A.8.2.10.1	Test Purpose and Environment	2057

A.8.2.10.2	Test Requirements	2059

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2059

A.8.2.11.1	Test Purpose and Environment	2059

A.8.2.11.2	Test Requirements	2061

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2061

A.8.2.12.1	Test Purpose and Environment	2061

A.8.2.12.2	Test Requirements	2063

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2064

A.8.2.13.1	Test Purpose and Environment	2064

A.8.2.13.2	Test Requirements	2066

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2066

A.8.2.14.1	Test Purpose and Environment	2066

A.8.2.14.2	Test Requirement	2068

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2068

A.8.2.15.1	Test Purpose and Environment	2068

A.8.2.15.2	Test Requirements	2070

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2070

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2070

A.8.3.1.1	Test Purpose and Environment	2070

A.8.3.1.2	Test Requirements	2072

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2072

A.8.3.2.1	Test Purpose and Environment	2072

A.8.3.2.2	Test Requirements	2075

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2075

A.8.3.3.1	Test Purpose and Environment	2075

A.8.3.3.2	Test Requirements	2078

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2078

A.8.3.4.1	Test Purpose and Environment	2078

A.8.3.4.2	Test Requirements	2080

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2080

A.8.3.5.2	Test Requirements	2082

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2082

A.8.3.6.1	Test Purpose and Environment	2082

A.8.3.6.2	Test Requirements	2084

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2085

A.8.3.7.1	Test Purpose and Environment	2085

A.8.3.7.2	Test Requirements	2087

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2088

A.8.3.8.1	Test Purpose and Environment	2088

A.8.3.8.2	Test Requirements	2091

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2091

A.8.3.9.1	Test Purpose and Environment	2091

A.8.3.9.2	Test Requirements	2095

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2096

A.8.3.10.1	Test Purpose and Environment	2096

A.8.3.10.2	Test Requirements	2097

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2097

A.8.3.11.1	Test Purpose and Environment	2097

A.8.3.11.2	Test Requirement	2099

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2100

A.8.3.12.1	Test Purpose and Environment	2100

A.8.3.12.2	Test Requirement	2101

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2102

A.8.3.13.1	Test Purpose and Environment	2102

A.8.3.13.2	Test Requirement	2103

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2104

A.8.3.14.1	Test Purpose and Environment	2104

A.8.3.14.2	Test Requirement	2105

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2106

A.8.3.15.1	Test Purpose and Environment	2106

A.8.3.15.2	Test Requirement	2107

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2108

A.8.3.16.1	Test Purpose and Environment	2108

A.8.3.16.2	Test Requirement	2110

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2110

A.8.3.17.1	Test Purpose and Environment	2110

A.8.3.17.2	Test Requirement	2113

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2113

A.8.3.18.1	Test Purpose and Environment	2113

A.8.3.18.2	Test Requirement	2116

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2116

A.8.3.19.1	Test Purpose and Environment	2116

A.8.3.19.2	Test Requirement	2119

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2119

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2119

A.8.4.1.1	Test Purpose and Environment	2119

A.8.4.1.2	Test Requirements	2121

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2121

A.8.4.2.1	Test Purpose and Environment	2121

A.8.4.2.2	Test Requirements	2124

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2124

A.8.4.3.1	Test Purpose and Environment	2124

A.8.4.3.2	Test Requirements	2126

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2127

A.8.4.4.1	Test Purpose and Environment	2127

A.8.4.4.2	Test Requirements	2129

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2130

A.8.4.5.1	Test Purpose and Environment	2130

A.8.4.5.2	Test Requirements	2132

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2132

A.8.4.6.1	Test Purpose and Environment	2132

A.8.4.6.2	Test Requirements	2133

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2133

A.8.4.7.1	Test Purpose and Environment	2133

A.8.4.7.2	Test Requirements	2137

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2137

A.8.4.8.1	Test Purpose and Environment	2137

A.8.4.8.2	Test Requirements	2140

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2140

A.8.4.9.1	Test Purpose and Environment	2140

A.8.4.9.2	Test Requirements	2145

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2145

A.8.4.10.1	Test Purpose and Environment	2145

A.8.4.10.2	Test Requirements	2147

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2147

A.8.4.11.1	Test Purpose and Environment	2147

A.8.4.11.2	Test Requirement	2149

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2149

A.8.4.12.1	Test Purpose and Environment	2149

A.8.4.12.2	Test Requirement	2151

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2152

A.8.4.13.1	Test Purpose and Environment	2152

A.8.4.13.2	Test Requirement	2153

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2154

A.8.4.14.1	Test Purpose and Environment	2154

A.8.4.14.2	Test Requirement	2156

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2156

A.8.4.15.1	Test Purpose and Environment	2156

A.8.4.15.2	Test Requirement	2159

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2159

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2159

A.8.5.1.1	Test Purpose and Environment	2159

A.8.5.1.2	Test Requirements	2161

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2161

A.8.5.2.1	Test Purpose and Environment	2161

A.8.5.2.2	Test Requirements	2163

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2163

A.8.5.3.1	Test Purpose and Environment	2163

A.8.5.3.2	Test Requirements	2166

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2166

A.8.5.4.1	Test Purpose and Environment	2166

A.8.5.4.2	Test Requirements	2168

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2168

A.8.5.5.1	Test Purpose and Environment	2168

A.8.5.5.2	Test Requirements	2171

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2171

A.8.5.6.1	Test Purpose and Environment	2171

A.8.5.6.2	Test Requirements	2172

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2173

A.8.5.7.1	Test Purpose and Environment	2173

A.8.5.7.2	Test Requirements	2173

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2173

A.8.5.8.1	Test Purpose and Environment	2173

A.8.5.8.2	Test Requirements	2176

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2176

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2176

A.8.6.1.1	Test Purpose and Environment	2176

A.8.6.1.2	Test Requirements	2178

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2178

A.8.6.2.1	Test Purpose and Environment	2178

A.8.6.2.2	Test Requirements	2181

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2181

A.8.6.3.1	Test Purpose and Environment	2181

A.8.6.3.2	Test Requirements	2184

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2184

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2184

A.8.7.1.1	Test Purpose and Environment	2184

A.8.7.1.1.1	Void	2184

A.8.7.1.1.2	1.28 Mcps TDD option	2184

A.8.7.1.1.3	Void	2186

A.8.7.1.2	Test Requirements	2186

A.8.7.1.2.1	Void	2186

A.8.7.1.2.2	1.28 Mcps TDD option	2186

A.8.7.1.2.3	Void	2186

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2186

A.8.7.2.1	Test Purpose and Environment	2186

A.8.7.2.2	Test Requirements	2189

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2190

A.8.7.3.1	Test Purpose and Environment	2190

A.8.7.3.2	Test Parameters	2190

A.8.7.3.3	Test Requirements	2191

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2192

A.8.7.4.1	Test Purpose and Environment	2192

A.8.7.4.2	Test Requirements	2194

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2194

A.8.7.5.1	Test Purpose and Environment	2194

A.8.7.5.2	Test Requirements	2196

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2196

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2196

A.8.7A.1.1	Test Purpose and Environment	2196

A.8.7A.1.2	Test Requirements	2199

A.8.8	E-UTRAN FDD – GSM Measurements	2199

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2199

A.8.8.1.1	Test Purpose and Environment	2199

A.8.8.1.2	Test Requirements	2201

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2201

A.8.8.2.1	Test Purpose and Environment	2201

A.8.8.2.2	Test Requirements	2203

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2204

A.8.8.3.1	Test Purpose and Environment	2204

A.8.8.3.2	Test Requirements	2205

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2206

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2206

A.8.9.1.1	Test Purpose and Environment	2206

A.8.9.1.2	Test Requirements	2207

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2208

A.8.9.2.1	Test Purpose and Environment	2208

A.8.9.2.2	Test Requirements	2210

A.8.10	E-UTRAN TDD – GSM Measurements	2210

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2210

A.8.10.1.1	Test Purpose and Environment	2210

A.8.10.1.2	Test Requirements	2211

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2212

A.8.10.2.1	Test Purpose and Environment	2212

A.8.10.2.2	Test Requirements	2214

A.8.11	Monitoring of Multiple Layers	2214

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2214

A.8.11.1.1	Test Purpose and Environment	2214

A.8.11.1.2	Test Requirements	2216

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2217

A.8.11.2.1	Test Purpose and Environment	2217

A.8.11.2.2	Test Requirements	2218

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2219

A.8.11.3.1	Test Purpose and Environment	2219

A.8.11.3.2	Test Requirements	2221

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2221

A.8.11.4.1	Test Purpose and Environment	2221

A.8.11.4.2	Test Requirements	2224

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2224

A.8.11.5.1	Test Purpose and Environment	2224

A.8.11.5.2	Test Requirements	2226

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2227

A.8.11.6.1	Test Purpose and Environment	2227

A.8.11.6.2	Test Requirements	2229

A.8.12	RSTD Intra-frequency Measurements	2230

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2230

A.8.12.1.1	Test Purpose and Environment	2230

A.8.12.1.2	Test Requirements	2234

A.8.12.1.2A	Test Requirements for UE Category 1bis	2234

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2234

A.8.12.2.1	Test Purpose and Environment	2234

A.8.12.2.2	Test Requirements	2239

A.8.12.2.2A	Test Requirements for UE Category 1bis	2239

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2239

A.8.12.3.1	Test Purpose and Environment	2239

A.8.12.3.2	Test Requirements	2245

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2245

A.8.12.4.1	Test Purpose and Environment	2245

A.8.12.4.2	Test Requirements	2250

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2250

A.8.12.5.1	Test Purpose and Environment	2250

A.8.12.5.2	Test Requirements	2255

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2255

A.8.12.6.1	Test Purpose and Environment	2255

A.8.12.6.2	Test Requirements	2260

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2260

A.8.12.7.1	Test Purpose and Environment	2260

A.8.12.7.2	Test Requirements	2265

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2265

A.8.12.8.1	Test Purpose and Environment	2265

A.8.12.8.2	Test Requirements	2270

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2270

A.8.12.9.1	Test Purpose and Environment	2270

A.8.12.9.2	Test Requirements	2275

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2275

A.8.12.10.1	Test Purpose and Environment	2275

A.8.12.10.2	Test Requirements	2280

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2280

A.8.12.11.1	Test Purpose and Environment	2280

A.8.12.11.2	Test Requirements	2285

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2285

A.8.12.12.1	Test Purpose and Environment	2285

A.8.12.12.2	Test Requirements	2290

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2290

A.8.12.13.1	Test Purpose and Environment	2290

A.8.12.13.2	Test Requirements	2295

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2295

A.8.12.14.1	Test Purpose and Environment	2295

A.8.12.14.2	Test Requirements	2300

A.8.13	RSTD Inter-frequency Measurements	2300

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2300

A.8.13.1.1	Test Purpose and Environment	2300

A.8.13.1.2	Test Requirements	2305

A.8.13.1.2A	Test Requirements for UE Category 1bis	2305

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2305

A.8.13.2.1	Test Purpose and Environment	2305

A.8.13.2.2	Test Requirements	2311

A.8.13.2.2A	Test Requirements for UE Category 1bis	2311

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2311

A.8.13.3.1	Test Purpose and Environment	2311

A.8.13.3.2	Test Requirements	2318

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2318

A.8.13.4.1	Test Purpose and Environment	2318

A.8.13.4.2	Test Requirements	2324

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2324

A.8.13.5.1	Test Purpose and Environment	2324

A.8.13.5.2	Test Requirements	2330

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2330

A.8.13.6.1	Test Purpose and Environment	2330

A.8.13.6.2	Test Requirements	2336

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2336

A.8.13.7.1	Test Purpose and Environment	2336

A.8.13.7.2	Test Requirements	2342

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2342

A.8.13.8.1	Test Purpose and Environment	2342

A.8.13.8.2	Test Requirements	2348

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2348

A.8.13.9.1	Test Purpose and Environment	2348

A.8.13.9.2	Test Requirements	2354

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2354

A.8.13.10.1	Test Purpose and Environment	2354

A.8.13.10.2	Test Requirements	2360

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2360

A.8.13.11.1	Test Purpose and Environment	2360

A.8.13.11.2	Test Requirements	2366

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2366

A.8.13.12.1	Test Purpose and Environment	2366

A.8.13.12.2	Test Requirements	2372

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2372

A.8.13.13.1	Test Purpose and Environment	2372

A.8.13.13.2	Test Requirements	2378

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2378

A.8.13.14.1	Test Purpose and Environment	2378

A.8.13.14.2	Test Requirements	2384

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2384

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2384

A.8.14.1.1	Test Purpose and Environment	2384

A.8.14.1.2	Test Requirements	2386

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2386

A.8.14.2.1	Test Purpose and Environment	2386

A.8.14.2.2	Test Requirements	2389

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2389

A.8.14.3.1	Test Purpose and Environment	2389

A.8.14.3.2	Test Requirements	2391

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2392

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2392

A.8.15.1.1	Test Purpose and Environment	2392

A.8.15.1.2	Test Requirements	2393

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2393

A.8.15.2.1	Test Purpose and Environment	2393

A.8.15.2.2	Test Requirements	2396

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2396

A.8.15.3.1	Test Purpose and Environment	2396

A.8.15.3.2	Test Requirements	2398

A.8.16	E-UTRAN Carrier Aggregation Measurements	2399

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2399

A.8.16.1.1	Test Purpose and Environment	2399

A.8.16.1.2	Test Requirements	2401

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2401

A.8.16.2.1	Test Purpose and Environment	2401

A.8.16.2.2	Test Requirements	2403

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2403

A.8.16.3.1	Test Purpose and Environment	2403

A.8.16.3.2	Test Requirements	2405

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2406

A.8.16.3A.1	Test Purpose and Environment	2406

A.8.16.3A.2	Test Requirements	2407

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2408

A.8.16.4.1	Test Purpose and Environment	2408

A.8.16.4.2	Test Requirements	2409

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2410

A.8.16.4A.1	Test Purpose and Environment	2410

A.8.16.4A.2	Test Requirements	2412

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2413

A.8.16.5.1	Test Purpose and Environment	2413

A.8.16.5.2	Test Requirements	2413

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2413

A.8.16.6.1	Test Purpose and Environment	2413

A.8.16.6.2	Test Requirements	2414

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2414

A.8.16.7.1	 Test Purpose and Environment	2414

A.8.16.7.2	 Test Requirements	2415

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2415

A.8.16.8.1	 Test Purpose and Environment	2415

A.8.16.8.2	 Test Requirements	2416

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2416

A.8.16.9.1	Test Purpose and Environment	2416

A.8.16.9.2	Test Requirements	2417

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2417

A.8.16.10.1	Test Purpose and Environment	2417

A.8.16.10.2	Test Requirements	2418

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2418

A.8.16.11.1	 Test Purpose and Environment	2418

A.8.16.11.2	 Test Requirements	2419

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2419

A.8.16.12.1	 Test Purpose and Environment	2419

A.8.16.12.2	 Test Requirements	2419

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2419

A.8.16.13.1	Test Purpose and Environment	2419

A.8.16.13.2	Test Requirements	2420

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2420

A.8.16.14.1	Test Purpose and Environment	2420

A.8.16.14.2	Test Requirements	2421

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2421

A.8.16.15.1	 Test Purpose and Environment	2421

A.8.16.7.2	 Test Requirements	2421

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2421

A.8.16.16.1	 Test Purpose and Environment	2421

A.8.16.16.2	 Test Requirements	2422

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2422

A.8.16.17.1	Test Purpose and Environment	2422

A.8.16.17.2	Test Requirements	2424

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2425

A.8.16.17A.1	Test Purpose and Environment	2425

A.8.16.17A.2	Test Requirements	2425

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2425

A.8.16.17B.1	Test Purpose and Environment	2425

A.8.16.17B.2	Test Requirements	2426

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2426

A.8.16.17C.1	Test Purpose and Environment	2426

A.8.16.17C.2	Test Requirements	2426

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2426

A.8.16.18.1	Test Purpose and Environment	2426

A.8.16.18.2	Test Requirements	2428

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2429

A.8.16.18A.1	Test Purpose and Environment	2429

A.8.16.18A.2	Test Requirements	2429

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2429

A.8.16.18B.1	Test Purpose and Environment	2429

A.8.16.18B.2	Test Requirements	2430

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2430

A.8.16.18C.1	Test Purpose and Environment	2430

A.8.16.18C.2	Test Requirements	2430

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2430

A.8.16.18D.1	Test Purpose and Environment	2430

A.8.16.18D.2	Test Requirements	2431

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2431

A.8.16.19.1	Test Purpose and Environment	2431

A.8.16.19.2	Test Requirements	2433

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2433

A.8.16.19A.1	Test Purpose and Environment	2433

A.8.16.19A.2	Test Requirements	2434

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2434

A.8.16.19B.1	Test Purpose and Environment	2434

A.8.16.19B.2	Test Requirements	2434

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2434

A.8.16.19C.1	Test Purpose and Environment	2434

A.8.16.19C.2	Test Requirements	2435

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2435

A.8.16.20.1	Test Purpose and Environment	2435

A.8.16.20.2	Test Requirements	2437

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2438

A.8.16.20A.1	Test Purpose and Environment	2438

A.8.16.20A.2	Test Requirements	2438

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2438

A.8.16.20B.1	Test Purpose and Environment	2438

A.8.16.20B.2	Test Requirements	2439

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2439

A.8.16.20C.1	Test Purpose and Environment	2439

A.8.16.20C.2	Test Requirements	2439

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2439

A.8.16.20D.1	Test Purpose and Environment	2439

A.8.16.20D.2	Test Requirements	2440

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2440

A.8.16.21.1	Test Purpose and Environment	2440

A.8.16.21.2	Test Requirements	2442

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2442

A.8.16.22.1	Test Purpose and Environment	2442

A.8.16.22.2	Test Requirements	2444

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2444

A.8.16.23.1	Test Purpose and Environment	2444

A.8.16.23.2	Test Requirements	2447

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2447

A.8.16.24.1	Test Purpose and Environment	2447

A.8.16.24.2	Test Requirements	2450

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2450

A.8.16.25.1	Test Purpose and Environment	2450

A.8.16.25.2	Test Requirements	2453

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2453

A.8.16.26.1	Test Purpose and Environment	2453

A.8.16.26.2	Test Requirements	2456

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2456

A.8.16.27.1	Test Purpose and Environment	2456

A.8.16.27.2	Test Requirements	2461

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2461

A.8.16.28.1	Test Purpose and Environment	2461

A.8.16.28.2	Test Requirements	2466

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2466

A.8.16.29.1	Test Purpose and Environment	2466

A.8.16.29.2	Test Requirements	2471

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2471

A.8.16.30.1	Test Purpose and Environment	2471

A.8.16.30.2	Test Requirements	2476

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2476

A.8.16.31.1	Test Purpose and Environment	2476

A.8.16.31.2	Test Requirements	2481

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2481

A.8.16.32.1	Test Purpose and Environment	2481

A.8.16.32.2	Test Requirements	2486

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2486

A.8.16.33.1	Test Purpose and Environment	2486

A.8.16.33.2	Test Requirements	2491

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2491

A.8.16.34.1	Test Purpose and Environment	2491

A.8.16.34.2	Test Requirements	2496

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2496

A.8.16.35.1	Test Purpose and Environment	2496

A.8.16.35.2	Test Requirements	2498

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2499

A.8.16.36.1	Test Purpose and Environment	2499

A.8.16.36.2	Test Requirements	2501

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2502

A.8.16.37.1	Test Purpose and Environment	2502

A.8.16.37.2	Test Requirements	2504

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2505

A.8.16.38.1	Test Purpose and Environment	2505

A.8.16.38.2	Test Requirements	2507

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2508

A.8.16.39.1	Test Purpose and Environment	2508

A.8.16.39.2	Test Requirements	2511

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2511

A.8.16.40.1	Test Purpose and Environment	2511

A.8.16.40.2	Test Requirements	2514

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2514

A.8.16.41.1	Test Purpose and Environment	2514

A.8.16.41.2	Test Requirements	2516

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2517

A.8.16.42.1	Test Purpose and Environment	2517

A.8.16.42.2	Test Requirements	2519

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2520

A.8.16.43.1	Test Purpose and Environment	2520

A.8.16.43.2	Test Requirements	2523

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2523

A.8.16.44.1	Test Purpose and Environment	2523

A.8.16.44.2	Test Requirements	2526

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2526

A.8.16.45.1	Test Purpose and Environment	2526

A.8.16.45.2	Test Requirements	2529

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2529

A.8.16.46.1	Test Purpose and Environment	2529

A.8.16.46.2	Test Requirements	2531

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2532

A.8.16.47.1	Test Purpose and Environment	2532

A.8.16.47.2	Test Requirements	2534

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2535

A.8.16.48.1	Test Purpose and Environment	2535

A.8.16.48.2	Test Requirements	2537

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2538

A.8.16.49.1	Test Purpose and Environment	2538

A.8.16.49.2	Test Requirements	2541

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2541

A.8.16.50.1	Test Purpose and Environment	2541

A.8.16.50.2	Test Requirements	2543

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2544

A.8.16.51.1	Test Purpose and Environment	2544

A.8.16.51.2	Test Requirements	2549

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2549

A.8.16.52.1	Test Purpose and Environment	2549

A.8.16.52.2	Test Requirements	2554

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2554

A.8.16.53.1	Test Purpose and Environment	2554

A.8.16.53.2	Test Requirements	2558

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2558

A.8.16.54.1	Test Purpose and Environment	2558

A.8.16.54.2	Test Requirements	2562

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2562

A.8.16.55.1	Test Purpose and Environment	2562

A.8.16.55.2	Test Requirements	2568

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2568

A.8.16.56.1	Test Purpose and Environment	2568

A.8.16.56.2	Test Requirements	2574

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2574

A.8.16.57.1	Test Purpose and Environment	2574

A.8.16.57.2	Test Requirements	2576

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2577

A.8.16.58.1	Test Purpose and Environment	2577

A.8.16.58.2	Test Requirements	2579

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2580

A.8.16.59.1	Test Purpose and Environment	2580

A.8.16.59.2	Test Requirements	2584

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2585

A.8.16.60.1	Test Purpose and Environment	2585

A.8.16.60.2	Test Requirements	2588

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2589

A.8.16.61.1	Test Purpose and Environment	2589

A.8.16.61.2	Test Requirements	2592

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2592

A.8.16.62.1	Test Purpose and Environment	2592

A.8.16.62.2	Test Requirements	2594

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2595

A.8.16.63.1	Test Purpose and Environment	2595

A.8.16.63.2	Test Requirements	2598

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2599

A.8.16.64.1	Test Purpose and Environment	2599

A.8.16.64.2	Test Requirements	2603

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2603

A.8.16.65.1	Test Purpose and Environment	2603

A.8.16.65.2	Test Requirements	2608

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2608

A.8.16.66.1	Test Purpose and Environment	2608

A.8.16.66.2	Test Requirements	2613

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2613

A.8.16.67.1	Test Purpose and Environment	2613

A.8.16.67.2	Test Requirements	2618

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2618

A.8.16.68.1	Test Purpose and Environment	2618

A.8.16.68.2	Test Requirements	2623

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2623

A.8.16.69.1	Test Purpose and Environment	2623

A.8.16.69.2	Test Requirements	2627

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2627

A.8.16.70.1	Test Purpose and Environment	2627

A.8.16.70.2	Test Requirements	2631

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2631

A.8.16.71.1	Test Purpose and Environment	2631

A.8.16.71.2	Test Requirements	2636

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2636

A.8.16.72.1	Test Purpose and Environment	2636

A.8.16.72.2	Test Requirements	2641

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2641

A.8.16.73.1	Test Purpose and Environment	2641

A.8.16.73.2	Test Requirements	2648

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2648

A.8.16.74.1	Test Purpose and Environment	2648

A.8.16.74.2	Test Requirements	2655

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2655

A.8.16.75.1	Test Purpose and Environment	2655

A.8.16.75.2	Test Requirements	2661

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2661

A.8.16.76.1	Test Purpose and Environment	2661

A.8.16.76.2	Test Requirements	2667

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2667

A.8.16.77.1	Test Purpose and Environment	2667

A.8.16.77.2	Test Requirements	2671

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2671

A.8.16.78.1	Test Purpose and Environment	2671

A.8.16.78.2	Test Requirements	2673

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2674

A.8.16.79.1	Test Purpose and Environment	2674

A.8.16.79.2	Test Requirements	2681

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2681

A.8.16.80.1	Test Purpose and Environment	2681

A.8.16.80.2	Test Requirements	2688

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2688

A.8.16.81.1	Test Purpose and Environment	2688

A.8.16.81.2	Test Requirements	2694

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2694

A.8.16.82.1	Test Purpose and Environment	2694

A.8.16.82.2	Test Requirements	2700

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2700

A.8.16.83.1	Test Purpose and Environment	2700

A.8.16.83.2	Test Requirements	2705

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2705

A.8.16.84.1	Test Purpose and Environment	2705

A.8.16.84.2	Test Requirements	2711

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2711

A.8.16.85.1	Test Purpose and Environment	2711

A.8.16.85.2	Test Requirements	2715

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2715

A.8.16.86.1	Test Purpose and Environment	2715

A.8.16.86.2	Test Requirements	2718

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2718

A.8.16.87.1	Test Purpose and Environment	2718

A.8.16.87.2	Test Requirements	2724

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2724

A.8.16.88.1	Test Purpose and Environment	2724

A.8.16.88.2	Test Requirements	2732

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2732

A.8.16.89.1	Test Purpose and Environment	2732

A.8.16.89.2	Test Requirements	2739

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2739

A.8.16.90.1	Test Purpose and Environment	2739

A.8.16.90.2	Test Requirements	2746

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2746

A.8.16.91.1	Test Purpose and Environment	2746

A.8.16.91.2	Test Requirements	2752

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2752

A.8.16.92.1	Test Purpose and Environment	2752

A.8.16.92.2	Test Requirements	2760

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2760

A.8.16.93.1	Test Purpose and Environment	2760

A.8.16.93.2	Test Requirements	2767

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2767

A.8.16.94.1	Test Purpose and Environment	2767

A.8.16.94.2	Test Requirements	2774

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2774

A.8.16.95.1	Test Purpose and Environment	2774

A.8.16.95.2	Test Requirements	2782

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2782

A.8.16.96.1	Test Purpose and Environment	2782

A.8.16.96.2	Test Requirements	2791

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2791

A.8.16.97.1	Test Purpose and Environment	2791

A.8.16.97.2	Test Requirements	2798

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2798

A.8.16.98.1	Test Purpose and Environment	2798

A.8.16.98.2	Test Requirements	2806

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2806

A.8.16.99.1	Test Purpose and Environment	2806

A.8.16.99.2	Test Requirements	2816

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2816

A.8.16.100.1	Test Purpose and Environment	2816

A.8.16.100.2	Test Requirements	2826

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2826

A.8.16.101.1	Test Purpose and Environment	2826

A.8.16.101.2	Test Requirements	2836

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2836

A.8.16.102.1	Test Purpose and Environment	2836

A.8.16.102.2	Test Requirements	2847

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2847

A.8.16.103.1	Test Purpose and Environment	2847

A.8.16.103.2	Test Requirements	2852

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2853

A.8.16.104.1	Test Purpose and Environment	2853

A.8.16.104.2	Test Requirements	2857

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2858

A.8.16.105.1	Test Purpose and Environment	2858

A.8.16.105.2	Test Requirements	2862

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2863

A.8.16.106.1	Test Purpose and Environment	2863

A.8.16.106.2	Test Requirements	2867

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2867

A.8.16.107.1	Test Purpose and Environment	2867

A.8.16.107.2	Test Requirements	2869

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2870

A.8.16.108.1	Test Purpose and Environment	2870

A.8.16.108.2	Test Requirements	2872

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2873

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2873

A.8.17.1.1	Test Purpose and Environment	2873

A.8.17.1.2	Test Requirements	2879

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2879

A.8.17.2.1	Test Purpose and Environment	2879

A.8.17.2.2	Test Requirements	2886

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2886

A.8.17.3.1	Test Purpose and Environment	2886

A.8.17.3.2	Test Requirements	2887

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2887

A.8.17.4.1	Test Purpose and Environment	2887

A.8.17.4.2	Test Requirements	2888

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2888

A.8.17.5.1	Test Purpose and Environment	2888

A.8.17.5.2	Test Requirements	2889

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2889

A.8.17.6.1	Test Purpose and Environment	2889

A.8.17.6.2	Test Requirements	2890

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2891

A.8.17.7.1	Test Purpose and Environment	2891

A.8.17.7.2	Test Requirements	2891

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2892

A.8.17.8.1	Test Purpose and Environment	2892

A.8.17.8.2	Test Requirements	2892

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2893

A.8.17.9.1	Test Purpose and Environment	2893

A.8.17.9.2	Test Requirements	2894

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2894

A.8.17.10.1	Test Purpose and Environment	2894

A.8.17.10.2	Test Requirements	2901

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2902

A.8.17.11.1	Test Purpose and Environment	2902

A.8.17.11.2	Test Requirements	2908

A.8.18	E-UTRAN TDD – HRPD Measurements	2909

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2909

A.8.18.1.1	Test Purpose and Environment	2909

A.8.18.1.2	Test Requirements	2911

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2911

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2911

A.8.19.1.1	Test Purpose and Environment	2911

A.8.19.1.2	Test Requirements	2912

A.8.20	Inter-frequency/RAT Measurements in CA mode	2913

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2913

A.8.20.1.1	Test Purpose and Environment	2913

A.8.20.1.2	Test Requirements	2914

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2914

A.8.20.2.1	Test Purpose and Environment	2915

A.8.20.2.2	Test Requirements	2916

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2917

A.8.20.2A.1	Test Purpose and Environment	2917

A.8.20.2A.2	Test Requirements	2917

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2917

A.8.20.2B.1	Test Purpose and Environment	2917

A.8.20.2B.2	Test Requirements	2920

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2920

A.8.20.3.1	Test Purpose and Environment	2920

A.8.20.3.2	Test Requirements	2922

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2922

A.8.20.4.1	Test Purpose and Environment	2922

A.8.20.4.1.1	1.28 Mcps TDD option	2922

A.8.20.4.2	Test Requirements	2924

A.8.20.4.2.1	1.28 Mcps TDD option	2924

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2924

A.8.20.4A.1	Test Purpose and Environment	2924

A.8.20.4A.1.1	1.28 Mcps TDD option	2924

A.8.20.4A.2	Test Requirements	2924

A.8.20.4A.2.1	1.28 Mcps TDD option	2924

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2925

A.8.20.4B.1	Test Purpose and Environment	2925

A.8.20.4B.1.1	1.28 Mcps TDD option	2925

A.8.20.4B.2	Test Requirements	2927

A.8.20.4B.2.1	1.28 Mcps TDD option	2927

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2927

A.8.21.1	Test Purpose and Environment	2927

A.8.21.2 Test Requirements	2931

A.8.22	E-UTRAN Discovery Signal Measurements	2931

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2931

A.8.22.1.1	Test Purpose and Environment	2931

A.8.22.1.2	Test Requirements	2934

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2934

A.8.22.2.1	Test Purpose and Environment	2934

A.8.22.2.2	Test Requirements	2937

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2937

A.8.22.3.1	Test Purpose and Environment	2937

A.8.22.3.2	Test Requirements	2940

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2940

A.8.22.4.1	Test Purpose and Environment	2940

A.8.22.4.2	Test Requirements	2943

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2943

A.8.22.5.1	Test Purpose and Environment	2943

A.8.22.5.2	Test Requirements	2947

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2947

A.8.22.6.1	Test Purpose and Environment	2947

A.8.22.6.2	Test Requirements	2951

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2951

A.8.22.7.1	Test Purpose and Environment	2951

A.8.22.7.2	Test Requirements	2955

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2955

A.8.22.8.1	Test Purpose and Environment	2955

A.8.22.8.2	Test Requirements	2959

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2959

A.8.22.9.1	Test Purpose and Environment	2959

A.8.22.9.2	Test Requirements	2961

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2962

A.8.22.10.1	Test Purpose and Environment	2962

A.8.22.10.2	Test Requirements	2964

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2965

A.8.22.11.1	Test Purpose and Environment	2965

A.8.22.11.2	Test Requirements	2968

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2968

A.8.22.12.1	Test Purpose and Environment	2968

A.8.22.12.2	Test Requirements	2972

A.8.23	E-UTRAN Dual Connectivity Measurements	2972

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2972

A.8.23.1.1	Test Purpose and Environment	2972

A.8.23.1.2	Test Requirements	2975

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2975

A.8.23.2.1	Test Purpose and Environment	2975

A.8.23.2.2	Test Requirements	2978

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2978

A.8.23.3.1	Test Purpose and Environment	2978

A.8.23.3.2	Test Requirements	2981

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2981

A.8.23.4.1	Test Purpose and Environment	2981

A.8.23.4.2	Test Requirements	2984

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2984

A.8.23.5.1	Test Purpose and Environment	2984

A.8.23.5.2	Test Requirements	2987

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2987

A.8.23.6.1	Test Purpose and Environment	2987

A.8.23.6.2	Test Requirements	2990

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2990

A.8.23.7.1	Test Purpose and Environment	2990

A.8.23.7.2	Test Requirements	2992

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2993

A.8.23.8.1	Test Purpose and Environment	2993

A.8.23.8.2	Test Requirements	2995

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2996

A.8.23.9.1	Test Purpose and Environment	2996

A.8.23.9.2	Test Requirements	2999

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2999

A.8.23.10.1	Test Purpose and Environment	2999

A.8.23.10.2	Test Requirements	3002

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3002

A.8.23.11.1	Test Purpose and Environment	3002

A.8.23.11.2	Test Requirements	3005

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3005

A.8.23.12.1	Test Purpose and Environment	3005

A.8.23.12.2	Test Requirements	3008

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3008

A.8.23.13.1	Test Purpose and Environment	3008

A.8.23.13.2	Test Requirements	3011

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3011

A.8.23.14.1	Test Purpose and Environment	3011

A.8.23.14.2	Test Requirements	3014

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3014

A.8.23.15.1	Test Purpose and Environment	3014

A.8.23.15.2	Test Requirements	3017

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3017

A.8.23.16.1	Test Purpose and Environment	3017

A.8.23.16.2	Test Requirements	3018

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3019

A.8.23.17.1	Test Purpose and Environment	3019

A.8.23.17.2	Test Requirements	3021

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3021

A.8.23.18.1	Test Purpose and Environment	3021

A.8.23.18.2	Test Requirements	3022

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3023

A.8.23.19.1	Test Purpose and Environment	3023

A.8.23.19.2	Test Requirements	3024

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3025

A.8.23.20.1	Test Purpose and Environment	3025

A.8.23.20.2	Test Requirements	3026

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3027

A.8.23.21.1	Test Purpose and Environment	3027

A.8.23.21.2	Test Requirements	3028

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3029

A.8.23.22.1	Test Purpose and Environment	3029

A.8.23.22.2	Test Requirements	3030

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3031

A.8.23.23.1	Test Purpose and Environment	3031

A.8.23.23.2	Test Requirements	3032

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3033

A.8.23.24.1	Test Purpose and Environment	3033

A.8.23.24.2	Test Requirements	3035

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3036

A.8.23.25.1	Test Purpose and Environment	3036

A.8.23.25.2	Test Requirements	3038

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3039

A.8.23.26.1	Test Purpose and Environment	3039

A.8.23.26.2	Test Requirements	3041

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3042

A.8.23.27.1	Test Purpose and Environment	3042

A.8.23.27.2	Test Requirements	3045

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3045

A.8.23.28.1	Test Purpose and Environment	3045

A.8.23.28.2	Test Requirements	3049

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3049

A.8.23.29.1	Test Purpose and Environment	3049

A.8.23.29.2	Test Requirements	3053

A.8.24	Proximity-based Services	3053

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3053

A.8.24.1.1	Test Purpose and Environment	3053

A.8.24.1.2	Test Requirements	3054

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3055

A.8.24.2.1	Test Purpose and Environment	3055

A.8.24.2.2	Test Requirements	3056

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3056

A.8.24.3.1	Test Purpose and Environment	3057

A.8.24.3.2	Test Requirements	3058

A.8.25	E-UTRAN-WLAN Measurements	3059

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3059

A.8.25.1.1	Test Purpose and Environment	3059

A.8.25.1.2	Test Requirements	3061

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3061

A.8.25.2.1	Test Purpose and Environment	3061

A.8.25.2.2	Test Requirements	3063

A.8.26 Frame Structure 3 (FS3)	3063

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3063

A.8.26.1.1	Test Purpose and Environment	3063

A.8.26.1.2	Test Requirements	3065

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3066

A.8.26.2.1	Test Purpose and Environment	3066

A.8.26.2.2	Test Requirements	3068

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3069

A.8.26.3.1	Test Purpose and Environment	3069

A.8.26.3.2	Test Requirements	3072

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3072

A.8.26.3A.1	Test Purpose and Environment	3072

A.8.26.3A.2	Test Requirements	3076

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3076

A.8.26.4.1	Test Purpose and Environment	3076

A.8.26.4.2	Test Requirements	3080

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3080

A.8.26.4A.1	Test Purpose and Environment	3080

A.8.26.4A.2	Test Requirements	3084

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3084

A.8.26.5.1	Test Purpose and Environment	3084

A.8.26.5.2	Test Requirements	3087

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3087

A.8.26.5A.1	Test Purpose and Environment	3087

A.8.26.5A.2	Test Requirements	3091

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3091

A.8.26.6.1	Test Purpose and Environment	3091

A.8.26.6.2	Test Requirements	3095

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3095

A.8.26.6A.1	Test Purpose and Environment	3095

A.8.26.6A.2	Test Requirements	3099

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3099

A.8.26.7.1	Test Purpose and Environment	3099

A.8.26.7.2	Test Requirements	3102

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3102

A.8.26.8.1	Test Purpose and Environment	3102

A.8.26.8.2	Test Requirements	3106

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3106

A.8.26.9.1	Test Purpose and Environment	3106

A.8.26.9.2	Test Requirements	3109

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3109

A.8.26.10.1	Test Purpose and Environment	3109

A.8.26.10.2	Test Requirements	3112

A.9	Measurement Performance Requirements	3112

A.9.1	RSRP	3112

A.9.1.1	FDD Intra frequency case	3112

A.9.1.1.1	Test Purpose and Environment	3112

A.9.1.1.2	Test parameters	3112

A.9.1.1.3	Test Requirements	3115

A.9.1.2	TDD Intra frequency case	3115

A.9.1.2.1	Test Purpose and Environment	3115

A.9.1.2.2	Test parameters	3115

A.9.1.2.3	Test Requirements	3116

A.9.1.3	FDD—FDD Inter frequency case	3117

A.9.1.3.1	Test Purpose and Environment	3117

A.9.1.3.2	Test parameters	3117

A.9.1.3.3	Test Requirements	3120

A.9.1.4	TDD—TDD Inter frequency case	3120

A.9.1.4.1	Test Purpose and Environment	3120

A.9.1.4.2	Test parameters	3120

A.9.1.4.3	Test Requirements	3122

A.9.1.5	FDD—TDD Inter frequency case	3123

A.9.1.5.1	Test Purpose and Environment	3123

A.9.1.5.2	Test parameters	3123

A.9.1.5.3	Test Requirements	3124

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3125

A.9.1.6.1	Test Purpose and Environment	3125

A.9.1.6.2	Test parameters	3125

A.9.1.6.3	Test Requirements	3128

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3128

A.9.1.7.1	Test Purpose and Environment	3128

A.9.1.7.2	Test parameters	3128

A.9.1.7.3	Test Requirements	3131

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3131

A.9.1.8.1	Test Purpose and Environment	3131

A.9.1.8.2	Test parameters	3131

A.9.1.8.3	Test Requirements	3135

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3135

A.9.1.9.1	Test Purpose and Environment	3135

A.9.1.9.2	Test parameters	3135

A.9.1.9.3	Test Requirements	3137

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3138

A.9.1.10.1	Test Purpose and Environment	3138

A.9.1.10.2	Test parameters	3138

A.9.1.10.3	Test Requirements	3141

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3141

A.9.1.11.1	Test Purpose and Environment	3141

A.9.1.11.2	Test parameters	3141

A.9.1.11.3	Test Requirements	3145

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3145

A.9.1.12.1	Test Purpose and Environment	3145

A.9.1.12.2	Test parameters	3145

A.9.1.12.3	Test Requirements	3146

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3146

A.9.1.13.1	Test Purpose and Environment	3146

A.9.1.13.2	Test parameters	3146

A.9.1.13.3	Test Requirements	3147

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3147

A.9.1.14.1	Test Purpose and Environment	3147

A.9.1.14.2	Test parameters	3147

A.9.1.14.3	Test Requirements	3151

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3151

A.9.1.15.1	Test Purpose and Environment	3151

A.9.1.15.2	Test parameters	3151

A.9.1.15.3	Test Requirements	3153

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3154

A.9.1.16.1	Test Purpose and Environment	3154

A.9.1.16.2	Test parameters	3154

A.9.1.16.3	Test Requirements	3155

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3155

A.9.1.17.1	Test Purpose and Environment	3155

A.9.1.17.2	Test parameters	3156

A.9.1.17.3	Test Requirements	3157

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3157

A.9.1.18.1	Test Purpose and Environment	3157

A.9.1.18.2	Test parameters	3157

A.9.1.18.3	Test Requirements	3158

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3159

A.9.1.19.1	Test Purpose and Environment	3159

A.9.1.19.2	Test parameters	3159

A.9.1.19.3	Test Requirements	3159

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3159

A.9.1.20.1	Test Purpose and Environment	3159

A.9.1.20.2	Test parameters	3159

A.9.1.20.3	Test Requirements	3161

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3161

A.9.1.21.1	Test Purpose and Environment	3161

A.9.1.21.2	Test parameters	3161

A.9.1.21.3	Test Requirements	3161

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3161

A.9.1.22.1	Test Purpose and Environment	3161

A.9.1.22.2	Test parameters	3162

A.9.1.22.3	Test Requirements	3166

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3166

A.9.1.23.1	Test Purpose and Environment	3166

A.9.1.23.2	Test parameters	3166

A.9.1.23.3	Test Requirements	3170

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3170

A.9.1.24.1	Test Purpose and Environment	3170

A.9.1.24.2	Test parameters	3170

A.9.1.24.3	Test Requirements	3171

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3171

A.9.1.25.1	Test Purpose and Environment	3171

A.9.1.25.2	Test parameters	3171

A.9.1.25.3	Test Requirements	3174

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3174

A.9.1.26.1	Test Purpose and Environment	3174

A.9.1.26.2	Test parameters	3174

A.9.1.26.3	Test Requirements	3177

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3177

A.9.1.27.1	Test Purpose and Environment	3177

A.9.1.27.2	Test parameters	3177

A.9.1.27.3	Test Requirements	3180

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3180

A.9.1.28.1	Test Purpose and Environment	3180

A.9.1.28.2	Test parameters	3180

A.9.1.28.3	Test Requirements	3183

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3183

A.9.1.29.1	Test Purpose and Environment	3183

A.9.1.29.2	Test parameters	3183

A.9.1.29.3	Test Requirements	3186

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3186

A.9.1.30.1	Test Purpose and Environment	3186

A.9.1.30.2	Test parameters	3187

A.9.1.30.3	Test Requirements	3190

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3190

A.9.1.31.1	Test Purpose and Environment	3190

A.9.1.31.2	Test parameters	3190

A.9.1.31.3	Test Requirements	3193

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3194

A.9.1.32.1	Test Purpose and Environment	3194

A.9.1.32.2	Test parameters	3194

A.9.1.32.3	Test Requirements	3197

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3197

A.9.1.33.1	Test Purpose and Environment	3197

A.9.1.33.2	Test parameters	3197

A.9.1.33.3	Test Requirements	3200

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3200

A.9.1.34.1	Test Purpose and Environment	3200

A.9.1.34.2	Test parameters	3200

A.9.1.34.3	Test Requirements	3203

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3203

A.9.1.35.1	Test Purpose and Environment	3203

A.9.1.35.2	Test parameters	3203

A.9.1.35.3	Test Requirements	3206

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3207

A.9.1.36.1	Test Purpose and Environment	3207

A.9.1.36.2	Test parameters	3207

A.9.1.36.3	Test Requirements	3210

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3210

A.9.1.37.1	Test Purpose and Environment	3210

A.9.1.37.2	Test parameters	3210

A.9.1.37.3	Test Requirements	3213

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3214

A.9.1.38.1	Test Purpose and Environment	3214

A.9.1.38.2	Test parameters	3214

A.9.1.38.3	Test Requirements	3217

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3218

A.9.1.39.1	Test Purpose and Environment	3218

A.9.1.39.2	Test parameters	3218

A.9.1.39.3	Test Requirements	3224

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3224

A.9.1.40.1	Test Purpose and Environment	3224

A.9.1.40.2	Test parameters	3225

A.9.1.40.3	Test Requirements	3231

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3231

A.9.1.41.1	Test Purpose and Environment	3231

A.9.1.41.2	Test parameters	3231

A.9.1.41.3	Test Requirements	3234

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3234

A.9.1.42.1	Test Purpose and Environment	3234

A.9.1.42.2	Test parameters	3234

A.9.1.42.3	Test Requirements	3237

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3237

A.9.1.43.1	Test Purpose and Environment	3237

A.9.1.43.2	Test parameters	3237

A.9.1.43.3	Test Requirements	3238

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3239

A.9.1.44.1	Test Purpose and Environment	3239

A.9.1.44.2	Test parameters	3239

A.9.1.44.3	Test Requirements	3244

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3244

A.9.1.45.1	Test Purpose and Environment	3244

A.9.1.45.2	Test parameters	3244

A.9.1.45.3	Test Requirements	3249

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3249

A.9.1.46.1	Test Purpose and Environment	3249

A.9.1.46.2	Test parameters	3249

A.9.1.46.3	Test Requirements	3258

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3259

A.9.1.47.1	Test Purpose and Environment	3259

A.9.1.47.2	Test parameters	3259

A.9.1.47.3	Test Requirements	3268

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3268

A.9.1.48.1	Test Purpose and Environment	3268

A.9.1.48.2	Test parameters	3269

A.9.1.48.3	Test Requirements	3279

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3280

A.9.1.49.1	Test Purpose and Environment	3280

A.9.1.49.2	Test parameters	3280

A.9.1.49.3	Test Requirements	3290

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3291

A.9.1.50.1	Test Purpose and Environment	3291

A.9.1.50.2	Test parameters	3291

A.9.1.50.3	Test Requirements	3298

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3299

A.9.1.51.1	Test Purpose and Environment	3299

A.9.1.51.2	Test parameters	3299

A.9.1.51.3	Test Requirements	3305

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3306

A.9.1.52.1	Test Purpose and Environment	3306

A.9.1.52.2	Test parameters	3306

A.9.1.52.3	Test Requirements	3309

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3309

A.9.1.52A.1	Test Purpose and Environment	3309

A.9.1.52A.2	Test parameters	3309

A.9.1.52A.3	Test Requirements	3310

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3310

A.9.1.53.1	Test Purpose and Environment	3310

A.9.1.53.2	Test parameters	3311

A.9.1.53.3	Test Requirements	3314

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3314

A.9.1.53A.1	Test Purpose and Environment	3314

A.9.1.53A.2	Test parameters	3314

A.9.1.53A.3	Test Requirements	3315

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3316

A.9.1.54.1	Test Purpose and Environment	3316

A.9.1.54.2	Test parameters	3316

A.9.1.54.3	Test Requirements	3319

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3319

A.9.1.55.1	Test Purpose and Environment	3319

A.9.1.55.2	Test parameters	3319

A.9.1.55.3	Test Requirements	3323

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3323

A.9.1.56.1	Test Purpose and Environment	3323

A.9.1.56.2	Test parameters	3323

A.9.1.56.3	Test Requirements	3326

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3327

A.9.1.57.1	Test Purpose and Environment	3327

A.9.1.57.2	Test parameters	3327

A.9.1.57.3	Test Requirements	3330

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3330

A.9.1.57A.1	Test Purpose and Environment	3330

A.9.1.57A.2	Test parameters	3330

A.9.1.57A.3	Test Requirements	3331

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3331

A.9.1.58.1	Test Purpose and Environment	3331

A.9.1.58.2	Test parameters	3332

A.9.1.58.3	Test Requirements	3335

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3335

A.9.1.58A.1	Test Purpose and Environment	3335

A.9.1.58A.2	Test parameters	3335

A.9.1.58A.3	Test Requirements	3336

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3336

A.9.1.59.1	Test Purpose and Environment	3336

A.9.1.59.2	Test parameters	3337

A.9.1.59.3	Test Requirements	3339

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3339

A.9.1.60.1	Test Purpose and Environment	3339

A.9.1.60.2	Test parameters	3339

A.9.1.60.3	Test Requirements	3342

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3343

A.9.1.61.1	Test Purpose and Environment	3343

A.9.1.61.2	Test parameters	3343

A.9.1.61.3	Test Requirements	3346

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3346

A.9.1.62.1	Test Purpose and Environment	3346

A.9.1.62.2	Test parameters	3346

A.9.1.62.3	Test Requirements	3349

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3349

A.9.1.53.1	Test Purpose and Environment	3349

A.9.1.53.2	Test parameters	3349

A.9.1.63.3	Test Requirements	3352

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3352

A.9.1.64.1	Test Purpose and Environment	3352

A.9.1.64.2	Test parameters	3352

A.9.1.64.3	Test Requirements	3353

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3354

A.9.1.65.1	Test Purpose and Environment	3354

A.9.1.65.2	Test parameters	3354

A.9.1.65.3	Test Requirements	3360

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3360

A.9.1.66.1	Test Purpose and Environment	3360

A.9.1.66.2	Test parameters	3360

A.9.1.66.3	Test Requirements	3366

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3366

A.9.1.67.1	Test Purpose and Environment	3366

A.9.1.67.2	Test parameters	3366

A.9.1.67.3	Test Requirements	3367

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3368

A.9.1.68.1	Test Purpose and Environment	3368

A.9.1.68.2	Test parameters	3368

A.9.1.68.3	Test Requirements	3372

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3372

A.9.1.69.1	Test Purpose and Environment	3372

A.9.1.69.2	Test parameters	3372

A.9.1.69.3	Test Requirements	3376

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3377

A.9.1.70.1	Test Purpose and Environment	3377

A.9.1.70.2	Test parameters	3377

A.9.1.70.3	Test Requirements	3389

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3390

A.9.1.71.1	Test Purpose and Environment	3390

A.9.1.71.2	Test parameters	3390

A.9.1.71.3	Test Requirements	3402

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3403

A.9.1.72.1	Test Purpose and Environment	3403

A.9.1.72.2	Test parameters	3403

A.9.1.72.3	Test Requirements	3415

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3416

A.9.1.73.1	Test Purpose and Environment	3416

A.9.1.73.2	Test parameters	3416

A.9.1.73.3	Test Requirements	3419

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3419

A.9.1.74.1	Test Purpose and Environment	3419

A.9.1.74.2	Test parameters	3419

A.9.1.74.3	Test Requirements	3420

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3420

A.9.1.75.1	Test Purpose and Environment	3420

A.9.1.75.2	Test parameters	3421

A.9.1.74.3	Test Requirements	3422

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3422

A.9.1.76.1	Test Purpose and Environment	3422

A.9.1.76.2	Test parameters	3422

A.9.1.76.3	Test Requirements	3425

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3425

A.9.1.77.1	Test Purpose and Environment	3425

A.9.1.77.2	Test parameters	3425

A.9.1.77.3	Test Requirements	3428

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3428

A.9.1.78.1	Test Purpose and Environment	3428

A.9.1.78.2	Test parameters	3428

A.9.1.78.3	Test Requirements	3431

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3431

A.9.1.79.1	Test Purpose and Environment	3431

A.9.1.79.2	Test parameters	3431

A.9.1.79.3	Test Requirements	3434

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3434

A.9.1.80.1	Test Purpose and Environment	3434

A.9.1.80.2	Test parameters	3434

A.9.1.80.3	Test Requirements	3437

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3437

A.9.1.81.1	Test Purpose and Environment	3437

A.9.1.78.2	Test parameters	3437

A.9.1.81.3	Test Requirements	3440

A.9.2	RSRQ	3440

A.9.2.1	FDD Intra frequency case	3440

A.9.2.1.1	Test Purpose and Environment	3440

A.9.2.1.2	Test parameters	3440

A.9.2.1.3	Test Requirements	3443

A.9.2.2	TDD Intra frequency case	3443

A.9.2.2.1	Test Purpose and Environment	3443

A.9.2.2.2	Test parameters	3443

A.9.2.2.3	Test Requirements	3445

A.9.2.3	FDD—FDD Inter frequency case	3445

A.9.2.3.1	Test Purpose and Environment	3445

A.9.2.3.2	Test parameters	3445

A.9.2.3.3	Test Requirements	3448

A.9.2.4	TDD—TDD Inter frequency case	3448

A.9.2.4.1	Test Purpose and Environment	3448

A.9.2.4.2	Test parameters	3448

A.9.2.4.3	Test Requirements	3450

A.9.2.4A	FDD—TDD Inter frequency case	3451

A.9.2.4A.1	Test Purpose and Environment	3451

A.9.2.4A.2	Test parameters	3451

A.9.2.4A.3	Test Requirements	3453

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3453

A.9.2.5.1	Test Purpose and Environment	3453

A.9.2.5.2	Test parameters	3454

A.9.2.5.3	Test Requirements	3457

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3457

A.9.2.6.1	Test Purpose and Environment	3457

A.9.2.6.2	Test parameters	3458

A.9.2.6.3	Test Requirements	3461

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3461

A.9.2.7.1	Test Purpose and Environment	3461

A.9.2.7.2	Test parameters	3461

A.9.2.7.3	Test Requirements	3465

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3465

A.9.2.8.1	Test Purpose and Environment	3465

A.9.2.8.2	Test parameters	3465

A.9.2.8.3	Test Requirements	3468

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3468

A.9.2.9.1	Test Purpose and Environment	3468

A.9.2.9.2	Test parameters	3468

A.9.2.9.3	Test Requirements	3472

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3472

A.9.2.10.1	Test Purpose and Environment	3472

A.9.2.10.2	Test parameters	3472

A.9.2.10.3	Test Requirements	3476

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3476

A.9.2.11.1	Test Purpose and Environment	3476

A.9.2.11.2	Test parameters	3476

A.9.2.11.3	Test Requirements	3477

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3477

A.9.2.12.1	Test Purpose and Environment	3477

A.9.2.12.2	Test parameters	3478

A.9.2.12.3	Test Requirements	3478

A.9.2.13	Void	3479

A.9.2.13.1	Void	3479

A.9.2.13.2	Void	3479

A.9.2.13.3	Void	3479

A.9.2.14	Void	3479

A.9.2.14.1	Void	3479

A.9.2.14.2	Void	3479

A.9.2.14.3	Void	3479

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3479

A.9.2.15.1	Test Purpose and Environment	3479

A.9.2.15.2	Test parameters	3479

A.9.2.15.3	Test Requirements	3483

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3483

A.9.2.16.1	Test Purpose and Environment	3483

A.9.2.16.2	Test parameters	3483

A.9.2.16.3	Test Requirements	3488

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3488

A.9.2.17.1	Test Purpose and Environment	3488

A.9.2.17.2	Test parameters	3488

A.9.2.17.3	Test Requirements	3490

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3490

A.9.2.18.1	Test Purpose and Environment	3490

A.9.2.18.2	Test parameters	3490

A.9.2.18.3	Test Requirements	3493

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3493

A.9.2.19.1	Test Purpose and Environment	3493

A.9.2.19.2	Test parameters	3493

A.9.2.19.3	Test Requirements	3495

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3495

A.9.2.20.1	Test Purpose and Environment	3495

A.9.2.20.2	Test parameters	3495

A.9.2.20.3	Test Requirements	3498

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3498

A.9.2.21.1	Test Purpose and Environment	3498

A.9.2.21.2	Test parameters	3498

A.9.2.21.3	Test Requirements	3501

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3501

A.9.2.22.1	Test Purpose and Environment	3502

A.9.2.22.2	Test parameters	3502

A.9.2.22.3	Test Requirements	3502

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3502

A.9.2.23.1	Test Purpose and Environment	3502

A.9.2.23.2	Test parameters	3502

A.9.2.23.3	Test Requirements	3505

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3505

A.9.2.24.1	Test Purpose and Environment	3505

A.9.2.24.2	Test parameters	3505

A.9.2.24.3	Test Requirements	3506

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3506

A.9.2.25.1	Test Purpose and Environment	3506

A.9.2.25.2	Test parameters	3506

A.9.2.25.3	Test Requirements	3511

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3511

A.9.2.26.1	Test Purpose and Environment	3511

A.9.2.26.2	Test parameters	3511

A.9.2.26.3	Test Requirements	3515

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3515

A.9.2.27.1	Test Purpose and Environment	3515

A.9.2.27.2	Test parameters	3515

A.9.2.27.3	Test Requirements	3516

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3516

A.9.2.28.1	Test Purpose and Environment	3516

A.9.2.28.2	Test parameters	3516

A.9.2.28.3	Test Requirements	3519

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3519

A.9.2.29.1	Test Purpose and Environment	3519

A.9.2.29.2	Test parameters	3519

A.9.2.29.3	Test Requirements	3522

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3522

A.9.2.30.1	Test Purpose and Environment	3522

A.9.2.30.2	Test parameters	3522

A.9.2.30.3	Test Requirements	3525

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3525

A.9.2.31.1	Test Purpose and Environment	3525

A.9.2.31.2	Test parameters	3525

A.9.2.31.3	Test Requirements	3528

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3528

A.9.2.32.1	Test Purpose and Environment	3528

A.9.2.32.2	Test parameters	3528

A.9.2.32.3	Test Requirements	3531

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3531

A.9.2.33.1	Test Purpose and Environment	3531

A.9.2.33.2	Test parameters	3531

A.9.2.33.3	Test Requirements	3535

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3535

A.9.2.34.1	Test Purpose and Environment	3535

A.9.2.34.2	Test parameters	3535

A.9.2.34.3	Test Requirements	3538

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3538

A.9.2.35.1	Test Purpose and Environment	3538

A.9.2.35.2	Test parameters	3538

A.9.2.35.3	Test Requirements	3541

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3541

A.9.2.36.1	Test Purpose and Environment	3541

A.9.2.36.2	Test parameters	3541

A.9.2.36.3	Test Requirements	3542

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3543

A.9.2.37.1	Test Purpose and Environment	3543

A.9.2.37.2	Test parameters	3543

A.9.2.37.3	Test Requirements	3544

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3545

A.9.2.38.1	Test Purpose and Environment	3545

A.9.2.38.2	Test parameters	3545

A.9.2.38.3	Test Requirements	3549

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3549

A.9.2.39.1	Test Purpose and Environment	3549

A.9.2.39.2	Test parameters	3549

A.9.2.39.3	Test Requirements	3553

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3553

A.9.2.40.1	Test Purpose and Environment	3553

A.9.2.40.2	Test parameters	3553

A.9.2.40.3	Test Requirements	3556

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3557

A.9.2.41.1	Test Purpose and Environment	3557

A.9.2.41.2	Test parameters	3557

A.9.2.41.3	Test Requirements	3560

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3560

A.9.2.42.1	Test Purpose and Environment	3560

A.9.2.42.2	Test parameters	3560

A.9.2.42.3	Test Requirements	3563

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3563

A.9.2.43.1	Test Purpose and Environment	3563

A.9.2.43.2	Test parameters	3563

A.9.2.43.3	Test Requirements	3566

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3566

A.9.2.44.1	Test Purpose and Environment	3566

A.9.2.44.2	Test parameters	3566

A.9.2.44.3	Test Requirements	3568

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3568

A.9.2.45.1	Test Purpose and Environment	3568

A.9.2.45.2	Test parameters	3568

A.9.2.45.3	Test Requirements	3572

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3573

A.9.2.46.1	Test Purpose and Environment	3573

A.9.2.46.2	Test parameters	3573

A.9.2.46.3	Test Requirements	3577

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3578

A.9.2.47.1	Test Purpose and Environment	3578

A.9.2.47.2	Test parameters	3578

A.9.2.47.3	Test Requirements	3582

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3582

A.9.2.48.1	Test Purpose and Environment	3582

A.9.2.48.2	Test parameters	3583

A.9.2.48.3	Test Requirements	3587

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3588

A.9.2.49.1	Test Purpose and Environment	3588

A.9.2.49.2	Test parameters	3588

A.9.2.49.3	Test Requirements	3594

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3595

A.9.2.50.1	Test Purpose and Environment	3595

A.9.2.50.2	Test parameters	3595

A.9.2.50.3	Test Requirements	3601

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3601

A.9.2.51.1	Test Purpose and Environment	3601

A.9.2.51.2	Test parameters	3602

A.9.2.51.3	Test Requirements	3606

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3606

A.9.2.52.1	Test Purpose and Environment	3606

A.9.2.52.2	Test parameters	3606

A.9.2.52.3	Test Requirements	3609

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3610

A.9.2.53.1	Test Purpose and Environment	3610

A.9.2.53.2	Test parameters	3610

A.9.2.53.3	Test Requirements	3616

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3617

A.9.2.54.1	Test Purpose and Environment	3617

A.9.2.54.2	Test parameters	3617

A.9.2.54.3	Test Requirements	3623

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3623

A.9.2.55.1	Test Purpose and Environment	3623

A.9.2.55.2	Test parameters	3624

A.9.2.55.3	Test Requirements	3629

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3629

A.9.2.56.1	Test Purpose and Environment	3629

A.9.2.56.2	Test parameters	3629

A.9.2.56.3	Test Requirements	3634

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3634

A.9.2.57.1	Test Purpose and Environment	3634

A.9.2.57.2	Test parameters	3635

A.9.2.57.3	Test Requirements	3639

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3640

A.9.2.58.1	Test Purpose and Environment	3640

A.9.2.58.2	Test parameters	3640

A.9.2.58.3	Test Requirements	3644

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3645

A.9.2.59.1	Test Purpose and Environment	3645

A.9.2.59.2	Test parameters	3645

A.9.2.59.3	Test Requirements	3650

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3651

A.9.2.60.1	Test Purpose and Environment	3651

A.9.2.60.2	Test parameters	3651

A.9.2.60.3	Test Requirements	3654

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3654

A.9.2.61.1	Test Purpose and Environment	3654

A.9.2.61.2	Test parameters	3654

A.9.2.61.3	Test Requirements	3657

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3657

A.9.2.62.1	Test Purpose and Environment	3657

A.9.2.62.2	Test parameters	3657

A.9.2.62.3	Test Requirements	3660

A.9.3	UTRAN FDD CPICH RSCP	3660

A.9.3.1	E-UTRAN FDD	3660

A.9.3.1.1	Test Purpose and Environment	3660

A.9.3.1.2	Parameters	3660

A.9.3.1.3	Test Requirements	3663

A.9.3.2 E-UTRAN TDD	3663

A.9.3.2.1	Test Purpose and Environment	3663

A.9.3.2.2	Parameters	3663

A.9.3.2.3	Test Requirements	3666

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3666

A.9.3.3.1	Test Purpose and Environment	3666

A.9.3.3.2	Parameters	3666

A.9.3.3.3	Test Requirements	3667

A.9.4	UTRAN FDD CPICH Ec/No	3668

A.9.4.1	E-UTRAN FDD	3668

A.9.4.1.1	Test Purpose and Environment	3668

A.9.4.1.2	Parameters	3668

A.9.4.1.3	Test Requirements	3670

A.9.4.2	E-UTRAN TDD	3671

A.9.4.2.1	Test Purpose and Environment	3671

A.9.4.2.2	Parameters	3671

A.9.4.2.3	Test Requirements	3673

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3674

A.9.4.3.1	Test Purpose and Environment	3674

A.9.4.3.2	Parameters	3674

A.9.4.3.3	Test Requirements	3675

A.9.5	UTRAN TDD measurement	3675

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3675

A.9.5.1.1	Test Purpose and Environment	3675

A.9.5.1.2	Test parameters	3675

A.9.5.1.3	Test Requirements	3677

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3678

A.9.5.2.1	Test Purpose and Environment	3678

A.9.5.2.2	Test parameters	3678

A.9.5.2.3	Test Requirements	3679

A.9.6	GSM Carrier RSSI	3680

A.9.6.1	E-UTRAN FDD	3680

A.9.6.1.1	Test Purpose and Environment	3680

A.9.6.1.2	Test Requirements	3681

A.9.6.2	E-UTRAN TDD	3681

A.9.6.2.1	Test Purpose and Environment	3681

A.9.6.2.2	Test Requirements	3683

A.9.7	UE Rx – Tx Time Difference	3684

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3684

A.9.7.1.1	Test Purpose and Environment	3684

A.9.7.1.2	Test parameters	3684

A.9.7.1.3	Test Requirements	3685

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3685

A.9.7.2.1	Test Purpose and Environment	3685

A.9.7.2.2	Test parameters	3685

A.9.7.2.3	Test Requirements	3687

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3687

A.9.7.3.1	Test Purpose and Environment	3687

A.9.7.3.2	Test parameters	3687

A.9.7.3.3	Test Requirements	3690

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3690

A.9.7.4.1	Test Purpose and Environment	3690

A.9.7.4.2	Test Parameters	3690

A.9.7.4.3	Test Requirements	3693

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3693

A.9.7.5.1	Test Purpose and Environment	3693

A.9.7.5.2	Test parameters	3693

A.9.7.5.3	Test Requirements	3696

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3696

A.9.7.6.1	Test Purpose and Environment	3696

A.9.7.6.2	Test Parameters	3696

A.9.7.6.3	Test Requirements	3699

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3699

A.9.7.7.1	Test Purpose and Environment	3699

A.9.7.7.2	Test parameters	3699

A.9.7.7.3	Test Requirements	3700

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3701

A.9.7.8.1	Test Purpose and Environment	3701

A.9.7.8.2	Test parameters	3701

A.9.7.8.3	Test Requirements	3702

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3702

A.9.7.9.1	Test Purpose and Environment	3702

A.9.7.9.2	Test parameters	3702

A.9.7.9.3	Test Requirements	3703

A.9.8	RSTD	3704

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3704

A.9.8.1.1	Test Purpose and Environment	3704

A.9.8.1.2	Test Requirements	3706

A.9.8.1.2A	Test Requirements for UE Category 1bis	3706

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3707

A.9.8.2.1	Test Purpose and Environment	3707

A.9.8.2.2	Test Requirements	3710

A.9.8.2.2A	Test Requirements for UE Category 1bis	3710

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3711

A.9.8.3.1	Test Purpose and Environment	3711

A.9.8.3.2	Test Requirements	3713

A.9.8.3.2A	Test Requirements for UE Category 1bis	3713

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3714

A.9.8.4.1	Test Purpose and Environment	3714

A.9.8.4.2	Test Requirements	3716

A.9.8.4.2A	Test Requirements for UE Category 1bis	3716

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3717

A.9.8.5.1	Test Purpose and Environment	3717

A.9.8.5.2	Test Requirements	3719

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3719

A.9.8.6.1	Test Purpose and Environment	3719

A.9.8.6.2	Test Requirements	3722

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3722

A.9.8.7.1	Test Purpose and Environment	3722

A.9.8.7.2	Test Requirements	3723

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3723

A.9.8.8.1	Test Purpose and Environment	3723

A.9.8.8.2	Test Requirements	3724

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3724

A.9.8.9.1	Test Purpose and Environment	3724

A.9.8.9.2	Test Requirements	3725

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3725

A.9.8.10.1	Test Purpose and Environment	3725

A.9.8.10.2	Test Requirements	3726

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3726

A.9.8.11.1	Test Purpose and Environment	3726

A.9.8.11.2	Test Requirements	3727

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3727

A.9.8.12.1	Test Purpose and Environment	3727

A.9.8.12.2	Test Requirements	3728

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3728

A.9.8.13.1	Test Purpose and Environment	3728

A.9.8.13.2	Test Requirements	3729

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3729

A.9.8.14.1	Test Purpose and Environment	3729

A.9.8.14.2	Test Requirements	3736

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3736

A.9.8.15.1	Test Purpose and Environment	3736

A.9.8.15.2	Test Requirements	3742

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3742

A.9.8.16.1	Test Purpose and Environment	3742

A.9.8.16.2	Test Requirements	3745

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3745

A.9.8.17.1	Test Purpose and Environment	3745

A.9.8.17.2	Test Requirements	3748

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3748

A.9.8.18.1	Test Purpose and Environment	3748

A.9.8.18.2	Test Requirements	3751

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3751

A.9.8.19.1	Test Purpose and Environment	3751

A.9.8.19.2	Test Requirements	3754

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3754

A.9.8.20.1	Test Purpose and Environment	3754

A.9.8.20.2	Test Requirements	3757

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3757

A.9.8.21.1	Test Purpose and Environment	3757

A.9.8.21.2	Test Requirements	3761

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3762

A.9.8.22.1	Test Purpose and Environment	3762

A.9.8.22.2	Test Requirements	3765

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3766

A.9.8.23.1	Test Purpose and Environment	3766

A.9.8.23.2	Test Requirements	3769

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3769

A.9.8.24.1	Test Purpose and Environment	3769

A.9.8.24.2	Test Requirements	3773

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3773

A.9.8.25.1	Test Purpose and Environment	3773

A.9.8.25.2	Test Requirements	3777

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3778

A.9.8.26.1	Test Purpose and Environment	3778

A.9.8.26.2	Test Requirements	3780

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3780

A.9.8.27.1	Test Purpose and Environment	3780

A.9.8.27.2	Test Requirements	3783

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3783

A.9.8.28.1	Test Purpose and Environment	3783

A.9.8.28.2	Test Requirements	3786

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3786

A.9.8.29.1	Test Purpose and Environment	3786

A.9.8.29.2	Test Requirements	3790

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3791

A.9.8.30.1	Test Purpose and Environment	3791

A.9.8.30.2	Test Requirements	3794

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3795

A.9.8.31.1	Test Purpose and Environment	3795

A.9.8.31.2	Test Requirements	3799

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3799

A.9.8.32.1	Test Purpose and Environment	3799

A.9.8.32.2	Test Requirements	3802

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3802

A.9.8.33.1	Test Purpose and Environment	3802

A.9.8.33.2	Test Requirements	3805

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3805

A.9.8.34.1	Test Purpose and Environment	3805

A.9.8.34.2	Test Requirements	3808

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3808

A.9.8.35.1	Test Purpose and Environment	3808

A.9.8.35.2	Test Requirements	3811

A.9.9	RSRP and RSRQ on the serving cell	3811

A.9.9.1	FDD Intra frequency serving cell case	3811

A.9.9.1.1	Test Purpose and Environment	3811

A.9.9.1.2	Test parameters	3811

A.9.9.1.3	Test Requirements	3814

A.9.9.2	TDD Intra frequency serving cell case	3814

A.9.9.2.1	Test Purpose and Environment	3814

A.9.9.2.2	Test parameters	3814

A.9.9.2.3	Test Requirements	3816

A.9.10	SSTD	3816

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3816

A.9.10.1.1	Test Purpose and Environment	3816

A.9.10.1.2	Test parameters	3816

A.9.10.1.3	Test Requirements	3818

A.9.10.2	Void	3818

A.9.10.3	Void	3818

A.9.10.4	Void	3818

A.9.11 RSSI	3818

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	3818

A.9.11.1.1	Test Purpose and Environment	3818

A.9.11.1.2	Test parameters	3818

A.9.11.1.3	Test Requirements	3821

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	3821

A.9.11.2.1	Test Purpose and Environment	3821

A.9.11.2.2	Test parameters	3821

A.9.12	Channel occupancy	3823

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	3823

A.9.12.1.1	Test Purpose and Environment	3823

A.9.12.1.2	Test parameters	3823

A.9.12.1.3	Test Requirements	3826

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	3826

A.9.12.2.1	Test Purpose and Environment	3826

A.9.12.2.2	Test parameters	3826

A.9.12.2.3	Test Requirements	3829

A.9.13	RS-SINR	3829

A.9.13.1	FDD Intra-Frequency Case	3829

A.9.13.1.1	Test Purpose and Environment	3829

A.9.13.1.2	Test parameters	3829

A.9.13.1.3	Test Requirements	3832

A.9.13.2	TDD Intra-Frequency Case	3832

A.9.13.2.1	Test Purpose and Environment	3832

A.9.13.2.2	Test parameters	3832

A.9.13.2.3	Test Requirements	3835

A.9.13.3	FDD—FDD Inter frequency case	3835

A.9.13.3.1	Test Purpose and Environment	3835

A.9.13.3.2	Test parameters	3835

A.9.13.3.3	Test Requirements	3838

A.9.13.4	TDD—TDD Inter frequency case	3838

A.9.13.4.1	Test Purpose and Environment	3838

A.9.13.4.2	Test parameters	3838

A.9.13.4.3	Test Requirements	3842

A.9.13.5	FDD—TDD Inter frequency case	3843

A.9.13.5.1	Test Purpose and Environment	3843

A.9.13.5.2	Test parameters	3843

A.9.13.5.3	Test Requirements	3849

A.9.13.6	TDD—FDD Inter frequency case	3849

A.9.13.6.1	Test Purpose and Environment	3849

A.9.13.6.2	Test parameters	3849

A.9.13.6.3	Test Requirements	3853

A.9.14	Channel quality reporting accuracy	3853

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3853

A.9.14.1.1	Test Purpose and Environment	3853

A.9.14.1.2	Test parameters	3854

A.9.14.1.3	Test Requirements	3854

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3854

A.9.14.2.1	Test Purpose and Environment	3854

A.9.14.2.2	Test parameters	3855

A.9.14.2.3	Test Requirements	3855

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3856

A.9.14.3.1	Test Purpose and Environment	3856

A.9.14.3.2	Test parameters	3856

A.9.14.3.3	Test Requirements	3857

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3857

A.9.14.4.1	Test Purpose and Environment	3857

A.9.14.4.2	Test parameters	3857

A.9.14.4.3	Test Requirements	3858

A.10	Proximity-based Services in Any Cell Selection State	3881

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3881

A.10.1.1	Test Purpose and Environment	3881

A.10.1.2	Test Requirements	3882

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3883

A.10.2.1	Test Purpose and Environment	3883

A.10.2.2	Test Requirements	3884

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3884

A.10.3.1	Test Purpose and Environment	3884

A.10.3.2	Test Requirements	3886

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3887

A.10.4.1	Test Purpose and Environment	3887

A.10.4.2	Test Requirements	3889

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3889

A.11.1	V2V UE Transmission Timing Accuracy Test	3889

A.11.1.1	Test Purpose and Environment	3889

A.11.1.2	Test requirements	3890

A.11.2	Interruptions due to V2V sidelink communication	3890

A.11.2.1	Test Purpose and Environment	3890

A.11.2.2	Test Requirements	3892

A.12	3892

A.12.1	V2X UE Transmission Timing Accuracy Test	3892

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3892

A.12.1.1.1	Test Purpose and Environment	3892

A.12.1.1.2	Test requirements	3893

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3894

A.12.1.2.1	Test Purpose and Environment	3894

A.12.1.2.2	Test Requirements	3894

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3895

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3895

A.12.2.1.1	Test Purpose and Environment	3895

A.12.2.1.2	Test Requirements	3896

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3897

A.12.2.2.1	Test Purpose and Environment	3897

A.12.2.2.2	Test Requirements	3898

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3898

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3898

A.12.3.1.1	Test Purpose and Environment	3898

A.12.3.1.2	Test Requirements	3901

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3902

A.12.3.2.1	Test Purpose and Environment	3902

A.12.3.1.2	Test Requirements	3904

A.12.4	Congestion Control Measurement Test for V2X UE	3904

A.12.4.1	Test Purpose and Environment	3904

A.12.4.2	Test Requirements	3906

A.12.5	Interruptions due to V2X Sidelink Communication	3906

A.12.5.1	Test Purpose and Environment	3906

A.12.5.2	Test Requirements	3908

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3908

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3908

A.12.6.1.1	Test Purpose and Environment	3908

A.12.6.1.2	Test Requirements	3910

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3910

A.12.6.2.1	Test Purpose and Environment	3910

A.12.6.1.2	Test Requirements	3912

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3912

A.12.7.1	Test Purpose and Environment	3912

A.12.7.2	Test Requirements	3915

A.12.8	Interruptions due to V2X Carrier Aggregation	3916

A.12.8.1	Interruptions on a FDD PCell	3916

A.12.8.1.1	Test Purpose and Environment	3916

A.12.8.1.2	Test Requirements	3917

A.12.8.2	Interruptions on a TDD PCell	3917

A.12.8.2.1	Test Purpose and Environment	3917

A.12.8.2.2	Test Requirements	3919

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3920

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3920

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3920

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3920

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3920

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3922

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3923

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3923

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3923

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3923

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3925

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3926

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3926

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3926

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3927

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3927

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3928

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3928

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3928

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3929

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3929

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3930

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3930

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3930

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3930

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3931

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3931

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3931

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3932

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3932

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3932

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3933

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3933

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3934

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3935

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3936

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3937

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3938

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3938

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3938

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3938

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3939

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3939

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3941

B.3	Conditions for measurements performance requirements for UE	3941

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3941

B.3.2	Void	3941

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3941

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3941

B.3.5	Conditions for UE Rx – Tx time difference	3941

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3942

B.3.7	Conditions for inter-frequency RSTD measurements	3942

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3942

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3942

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3942

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3943

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3943

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3943

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3943

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3943

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3943

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3944

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3944

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3944

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3945

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3945

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3945

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3945

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3945

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3945

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3945

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3945

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3945

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3946

B.3.21.1	Conditions for RSRP measurements	3946

B.3.21.2	Conditions for RSRQ measurements	3946

B.3.21.3	Conditions for CSI-RSRP measurements	3946

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3946

B.3.22.1	Conditions for RSRP measurements	3946

B.3.22.2	Void	3947

B.3.22.3	Conditions for CSI-RSRP measurements	3947

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3947

B.3.23.1	Conditions for RSRP measurements	3947

B.3.23.2	Conditions for RSRQ measurements	3947

B.3.23.3	Conditions for CSI-RSRP measurements	3947

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3948

B.3.24.1	Conditions for RSRP measurements	3948

B.3.24.2	Conditions for RSRQ measurements	3948

B.3.24.3	Conditions for CSI-RSRP measurements	3948

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3948

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3948

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3948

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3949

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3949

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3949

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3949

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3949

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3950

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3950

B.4	RRM Requirements Exceptions	3950

B.4.1	General	3950

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3950

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3950

B.4.3.1	Inter-band carrier aggregation	3950

B.4.3.2	Intra-band non-contiguous carrier aggregation	3950

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3951

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3951

B.5.1	Conditions for S-RSRP Accuracy Requirements	3951

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3951

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3952

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3952

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3953

B.6	Conditions for V2X	3953

B.6.1	Test parameters for GNSS signals	3953

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3953

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3953

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3954

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3954

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3955

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3955

Annex C (informative): Change history:	3956