| 3GPP TS 36.133 V17.5.0 (2022-03) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	132

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	133

4.2.2.9	UE measurement capability	133

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	134

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	135

4.2.2.11	Void	136

4.2.2.12	Void	136

4.2.2.13	Void	136

4.3	Minimization of Drive Tests (MDT)	136

4.3.1	Introduction	137

4.3.2	Measurements	137

4.3.2.1	Requirements	137

4.3.3	Relative Time Stamp Accuracy	137

4.3.3.1	Requirements	137

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	137

4.3.4.1	 Requirements	137

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	138

4.3.5.1	Requirements for timeSinceFailure	138

4.4	MBSFN Measurements	138

4.4.1	Introduction	138

4.4.2	MBSFN RSRP measurements	138

4.4.3	MBSFN RSRQ measurements	138

4.4.4	MCH BLER measurements	139

4.5	Proximity-based Services	139

4.5.1	Introduction	139

4.5.2	Requirements	139

4.5.2.1	Interruptions with ProSe Direct Discovery	139

4.5.2.2	Interruptions with ProSe Direct Communication	139

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	139

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	140

4.6	Cell Selection and Re-selection Requirements for UE category NB1	140

4.6.1	Cell Selection	140

4.6.2	Cell Re-selection	140

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	141

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	142

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	143

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	144

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	146

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	148

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	149

4.6.2.7	Maximum interruption in paging reception in normal coverage	151

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	151

4.6.2.8	UE measurement capability	151

4.6.2.9	WUS receptions for NB1	151

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	152

4.6.3.1	Introduction	152

4.6.3.2	Requirements on UE synchronization for transmission using PUR	152

4.6.3.3	Requirements on TA validation for transmission using PUR	152

4.7	Cell Selection and Re-selection Requirements for UE category M1	153

4.7.1	Cell Selection	153

4.7.2	Cell Re-selection	153

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	153

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	153

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	155

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	157

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	158

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	159

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	159

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	159

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	160

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	161

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	164

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	165

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	166

4.7.2.3	WUS receptions for UE category M1	166

4.7.3	Channel quality report for UE Category M1 in idle mode	166

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	167

4.7.4.1	Introduction	167

4.7.4.2	Requirements on UE synchronization for transmission using PUR	167

4.7.4.3	Requirements on TA validation for transmission using PUR	167

4.8	Idle State Positioning Measurement Requirements for UE category NB1	168

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	168

4.8.1.1	RSTD Measurement Reporting Delay	170

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	170

4.8.2.1	RSTD Measurement Reporting Delay	172

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	172

4.8.3.1	RSTD Measurement Reporting Delay	173

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	174

4.8.4.1	RSTD Measurement Reporting Delay	175

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	176

4.8.5.1	Measurement Reporting Delay	177

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	177

4.8.6.1	Measurement Reporting Delay	178

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	179

4.8.7.1	Measurement Reporting Delay	180

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	180

4.8.8.1	Measurement Reporting Delay	181

4.9	Idle Mode CA Measurement	182

4.9.1	Introduction	182

4.9.2	Requirement	182

4.9.2.1	Detected cell requirement during state transition and Idle mode	182

4.9.2.2	Measurements of inter-frequency CA candidate cells	183

4.9.2.3	Measurements on serving cell	183

4A	E-UTRAN RRC_INACTIVE state mobility	184

4A.1	Cell Re-selection	184

4A.1.1	Introduction	184

4A.1.2	Requirements	185

4A.1.2.1	UE measurement capability	185

4A.1.2.2	Measurement and evaluation of serving cell	185

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	185

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	185

4A.1.2.5	Evaluation of cell re-selection criteria	185

4A.1.2.6	Maximum interruption in paging reception	185

4A.1.2.7	Measurements of inter-RAT NR cells	185

4A.2	Requirements for UE Category M1	185

4A.2.1	Introduction	185

4A.2.2	Cell Selection	185

4A.2.3	Cell Reselection	185

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	185

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	185

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	186

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	186

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	186

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	186

4A.2.4	Channel quality report for UE Category M1 in idle mode	188

5	E-UTRAN RRC_CONNECTED state mobility	188

5.1	E-UTRAN Handover	189

5.1.1	Introduction	189

5.1.2	Requirements	189

5.1.2.1	E-UTRAN FDD – FDD	189

5.1.2.1.1	Handover delay	189

5.1.2.1.2	Interruption time	189

5.1.2.2	E-UTRAN FDD – TDD	190

5.1.2.2.1	(Void)	191

5.1.2.2.2	(Void)	191

5.1.2.3	E-UTRAN TDD – FDD	191

5.1.2.3.1	(Void)	191

5.1.2.3.2	(Void)	191

5.1.2.4	E-UTRAN TDD – TDD	191

5.1.2.4.1	Handover delay	191

5.1.2.4.2	Interruption time	191

5.1.2.5	E-UTRAN HD–FDD	193

5.1.2.5.1	Handover delay	193

5.1.2.5.2	Interruption time	193

5.1.2.6	E-UTRAN FDD – FDD conditional handover	194

5.1.2.6.1	Handover delay	194

5.1.2.6.2	Measurement time	195

5.1.2.6.3	Preparation time	195

5.1.2.6.4	Interruption time	195

5.1.2.7	E-UTRAN FDD – TDD conditional handover	195

5.1.2.8	E-UTRAN TDD – FDD conditional handover	196

5.1.2.9	E-UTRAN TDD – TDD conditional handover	196

5.2	Void	196

5.3	Handover to other RATs	196

5.3.1	E-UTRAN - UTRAN FDD Handover	196

5.3.1.1	Introduction	196

5.3.1.1.1	Handover delay	196

5.3.1.1.2	Interruption time	196

5.3.2	E-UTRAN - UTRAN TDD Handover	197

5.3.2.1	Introduction	197

5.3.2.2	Requirements	197

5.3.2.2.1	Handover delay	197

5.3.2.2.2	Interruption time	197

5.3.3	E-UTRAN - GSM Handover	198

5.3.3.1	Introduction	198

5.3.3.2	Requirements	198

5.3.3.2.1	Handover delay	198

5.3.3.2.2	Interruption time	198

5.3.4	E-UTRAN - NR FR1 Handover	198

5.3.4.1	Introduction	198

5.3.4.2	Handover delay	199

5.3.4.3	Interruption time	199

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	199

5.3.4A.1	Introduction	199

5.3.4A.2	Handover delay	200

5.3.4A.3	Interruption time	200

5.3.5	E-UTRAN - NR FR2 Handover	201

5.3.5.1	Introduction	201

5.3.5.2	Handover delay	201

5.3.5.3	Interruption time	201

5.4	Handover to Non-3GPP RATs	202

5.4.1	E-UTRAN – HRPD Handover	202

5.4.1.1	Introduction	202

5.4.1.1.1	Handover delay	202

5.4.1.1.2	Interruption time	202

5.4.2	E-UTRAN – cdma2000 1X Handover	203

5.4.2.1	Introduction	203

5.4.2.1.1	Handover delay	203

5.4.2.1.2	Interruption time	203

5.5	E-UTRAN Handover for Cat-M1 UEs	203

5.5.1	Introduction	203

5.5.2	Requirements in CEModeA	203

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	203

5.5.2.1.1	Handover delay	203

5.5.2.1.2	Interruption time	204

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	204

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	204

5.5.2.3.1	Void	205

5.5.2.3.2	Void	205

5.5.3	Requirements in CEModeB	205

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	205

5.5.3.1.1	Handover delay	205

5.5.3.1.2	Interruption time	205

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	206

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	206

5.6	Void	206

5.7	E-UTRAN DAPS Handover	206

5.7.1	Introduction	206

5.7.2	Requirements	206

5.7.2.1	E-UTRAN FDD – FDD	206

5.7.2.1.1	DAPS Handover delay	206

5.7.2.1.2	Interruption time	207

5.7.2.2	E-UTRAN FDD – TDD	207

5.7.2.3	E-UTRAN TDD – FDD	207

5.7.2.4	E-UTRAN TDD – TDD	208

5.8	EN-DC Handover with PSCell	208

5.8.1	Introduction	208

5.8.1.1	Handover with PSCell Interruption time	208

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	208

6	RRC Connection Mobility Control	209

6.1	RRC Re-establishment	209

6.1.1	Introduction	209

6.1.2	Requirements	209

6.1.2.1	UE Re-establishment delay requirement	209

6.2	Random Access	210

6.2.1	Introduction	210

6.2.2	Requirements	210

6.2.2.1	Contention based random access	210

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	210

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	210

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	210

6.2.2.1.4	Void	210

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	210

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	211

6.2.2.2	Non-Contention based random access	211

6.2.2.2.1	Correct behaviour when receiving Random Access Response	211

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	211

6.2.3	Requirements for Cat-M1 UEs	211

6.3	RRC Connection Release with Redirection	211

6.3.1	Introduction	211

6.3.2	Requirements	211

6.3.2.1	RRC connection release with redirection to UTRAN FDD	211

6.3.2.2	RRC connection release with redirection to GERAN	212

6.3.2.3	RRC connection release with redirection to UTRAN TDD	212

6.3.2.4	RRC connection release with redirection to NR	213

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	214

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	215

6.4.1	Introduction	215

6.4.2	Requirements	215

6.5	RRC Re-establishment for NB-IoT UEs	215

6.5.1	Introduction	215

6.5.2	Requirements	215

6.5.2.1	UE Re-establishment delay requirement in normal coverage	215

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	216

6.6	Random Access for UE category NB1	216

6.6.1	Introduction	216

6.6.2	Requirements	217

6.6.2.1	Correct behaviour when receiving Random Access Response reception	217

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	217

6.6.2.3	Correct behaviour when receiving a NACK on msg3	217

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	217

6.6.2.5	Correct behaviour when contention Resolution timer expires	217

6.6.2.6	MSG3-based channel quality report for UE Category NB1	217

6.6.3	Requirements for NPRACH configuration	218

6.7	RRC Re-establishment for Cat-M1 UEs	218

6.7.1	Introduction	218

6.7.2	Requirements	218

6.7.2.1	UE Re-establishment delay requirement for CEModeA	218

6.7.2.2	UE Re-establishment delay requirement for CEModeB	219

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	219

6.8.1	Introduction	219

6.8.2	Requirements	220

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	220

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	220

6.9.1	Introduction	220

6.9.2	Requirements	220

7	Timing and signalling characteristics	221

7.1	UE transmit timing	221

7.1.1	Introduction	221

7.1.2	Requirements	222

7.2	UE timer accuracy	223

7.2.1	Introduction	223

7.2.2	Requirements	223

7.3	Timing Advance	223

7.3.1	Introduction	223

7.3.2	Requirements	223

7.3.2.1	Timing Advance adjustment delay	223

7.3.2.2	Timing Advance adjustment accuracy	224

7.4	Cell phase synchronization accuracy (TDD)	224

7.4.1	Definition	224

7.4.2	Minimum requirements	224

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	225

7.5.1	Introduction	225

7.5.2	eNodeB Synchronization Requirements	225

7.5.2.1	Synchronized E-UTRAN	225

7.5.2.2	Non-Synchronized E-UTRAN	225

7.6	Radio Link Monitoring	225

7.6.1	Introduction	225

7.6.2	Requirements	227

7.6.2.1	Minimum requirement when no DRX is used	227

7.6.2.2	Minimum requirement when DRX is used	227

7.6.2.3	Minimum requirement at transitions	229

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	229

7.6.2.5	Minimum requirement under IDC Interference	229

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	229

7.7.1	Introduction	229

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	230

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	231

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	232

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	233

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	234

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	235

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	236

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	236

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	236

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	237

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	238

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	239

7.7.14	SCell Activation Delay Requirement for Dormant SCell	239

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	241

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	241

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	243

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	243

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	245

7.8	Interruptions with Carrier Aggregation	247

7.8.1	Introduction	247

7.8.2	Requirements	247

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	247

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	247

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	247

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	248

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	248

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	248

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	248

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	248

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	249

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	250

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	250

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	250

7.8.2.13	Interruptions at SRS carrier based switching	251

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	252

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	252

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	252

7.8.2.17	Interruptions during CQI measurement on dormant SCell	252

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	253

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	253

7.8.2.20	Interruptions at SCell hibernation	253

7.8.2.21	Interruptions at direct SCell activation and hibernation	253

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	254

7.9.1	Introduction	254

7.9.2	Minimum Requirements for Interband Carrier Aggregation	254

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	254

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	255

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	255

7.10.1	Introduction	255

7.10.2	Requirements	255

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	255

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	255

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	256

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	256

7.11	Radio Link Monitoring for UE Category 0	257

7.11.1	Introduction	257

7.11.2	Requirements for FD-FDD and TDD	258

7.11.2.1	Minimum requirement when no DRX is used	258

7.11.2.2	Minimum requirement when DRX is used	259

7.11.2.3	Minimum requirement at transitions	259

7.11.3	Requirements for HD-FDD	260

7.11.3.1	Minimum requirement when no DRX is used	260

7.11.3.2	Minimum requirement when DRX is used	260

7.11.3.3	Minimum requirement at transitions	261

7.12	Interruptions with Dual Connectivity	261

7.12.1	Introduction	261

7.12.2	Requirements	261

7.12.2.1	Interruptions at PSCell addition/release	261

7.12.2.2	Interruptions at transitions between active and non-active during DRX	261

7.12.2.3	Interruptions at transitions from non-DRX to DRX	262

7.12.2.4	Interruptions at SCell addition/release	262

7.12.2.5	Interruptions at SCell activation/deactivation	262

7.12.2.6	Interruptions during measurements on SCC	263

7.12.2.7	Interruptions at SRS carrier based switching	263

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	264

7.13.1	Definition	264

7.13.2	Minimum requirements	264

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	264

7.14.1	Introduction	264

7.14.2	PSCell Addition Delay Requirement	264

7.14.3	PSCell Release Delay Requirement	265

7.15	Maximum Receive Timing Difference in Dual Connectivity	265

7.15.1	Introduction	265

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	265

7.16	Proximity-based Services	265

7.16.1	Introduction	265

7.16.2	Requirements	265

7.16.2.1	ProSe UE transmission timing	265

7.16.2.1.1	Serving cell or PCell as timing reference	266

7.16.2.1.2	SCell or non-serving cell as timing reference	266

7.16.3	Interruptions with ProSe	266

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	266

7.16.3.2	Interruptions at ProSe Direct Communication configuration	267

7.16.3.3	Interruptions during ProSe Direct Discovery	267

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	267

7.16.3.5	Interruptions during ProSe Direct Communication	268

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	268

7.16.4.1	Measurement and evaluation of selected cell	268

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	268

7.16.5	Selection / Reselection of ProSe relay UE	269

7.16.6	ProSe operation under deactivated SCell	269

7.17	Maximum Transmission Timing Difference in Dual Connectivity	269

7.17.1	Introduction	269

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	270

7.18.1	Introduction	270

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	270

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	270

7.19	Radio Link Monitoring for UE Category M1	270

7.19.1	Introduction	270

7.19.2	Requirements for FD-FDD and TDD CE mode A	271

7.19.2.1	Minimum requirement when no DRX is used	272

7.19.2.2	Minimum requirement when DRX is used	273

7.19.2.3	Minimum requirement at transitions	274

7.19.3	Requirements for HD-FDD with CE mode A	274

7.19.3.1	Minimum requirement when no DRX is used	275

7.19.3.2	Minimum requirement when DRX is used	275

7.19.3.3	Minimum requirement at transitions	276

7.19.4	Requirements for FD-FDD and TDD with CE mode B	276

7.19.4.1	Minimum requirement when no DRX is used	278

7.19.4.2	Minimum requirement when DRX is used	279

7.19.4.3	Minimum requirement at transitions	280

7.19.5	Requirements for HD-FDD with CE mode B	280

7.19.5.1	Minimum requirement when no DRX is used	280

7.19.5.2	Minimum requirement when DRX is used	281

7.19.5.3	Minimum requirement at transitions	282

7.20	UE transmit timing for NB-IoT	282

7.20.1	Introduction	282

7.20.2	Requirements	282

7.21	 UE timer accuracy for NB-IoT	283

7.21.1	Introduction	283

7.21.2	Requirements	283

7.22	Timing Advance for NB-IoT	283

7.22.1	Introduction	283

7.22.2	Requirements	283

7.22.2.1	Timing Advance adjustment delay	283

7.22.2.2	Timing Advance adjustment accuracy	283

7.23	Radio Link Monitoring for Category NB1 UE	283

7.23.1	Introduction	283

7.23.2	Requirements for Category NB1 UE	284

7.23.2.1	Minimum requirement when no DRX is used	284

7.23.2.2	Minimum requirement when DRX is used	285

7.23.2.3	Minimum requirement at transitions	285

7.24	UE transmit timing for Category M1	285

7.24.1	Introduction	285

7.24.2	Requirements	286

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	286

7.25.1	Definition	286

7.25.2	Minimum requirements	287

7.26	UE transmit timing for Category M2	287

7.26.1	Introduction	287

7.26.2	Requirements	287

7.27	UE timer accuracy for category M1	287

7.27.1	Introduction	287

7.27.2	Requirements	287

7.28	Timing Advance for Category M1	288

7.28.1	Introduction	288

7.28.2	Requirements	288

7.29	Interruptions requirements with FeMBMS	288

7.29.1	Introduction	288

7.29.2	Requirements	288

7.30	Numerology switching delay requirements with FeMBMS	288

7.30.1	Introduction	288

7.30.2	Requirements	288

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	288

7.31.1	Introduction	288

7.31.2	NR PSCell Addition Delay Requirement	289

7.31.3	NR PSCell Release Delay Requirement	289

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	290

7.31A.1	Introduction	290

7.31A.2	NR PSCell Addition Delay Requirement	290

7.31A.3	NR PSCell Release Delay Requirement	291

7.32	Interruptions with EN-DC	291

7.32.1	Introduction	291

7.32.2	Requirements	292

7.32.2.1	Interruptions at PSCell addition/release	292

7.32.2.2	Interruptions at transitions between active and non-active during DRX	292

7.32.2.3	Interruptions at transitions from non-DRX to DRX	292

7.32.2.4	Interruptions at SCell addition/release	292

7.32.2.5	Interruptions at SCell activation/deactivation	292

7.32.2.6	Interruptions during measurements on SCC	293

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	293

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	293

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	293

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	293

7.32.2.7	Interruptions at active BWP switching	294

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	294

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	295

7.32.2.10	Interruptions at SCell hibernation	295

7.32.2.11	Interruptions at direct SCell activation and hibernation	295

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	295

7.32.2.13	Interruptions at NR SRS carrier based switching	295

7.32.2.14	Interruptions at NR SCell dormancy	296

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	296

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	296

7.32.2.15	Interruption during NR measurement with autonomous gaps	297

7.32.2.16	Interruptions at SRS carrier based switching	297

7.32.2.17	Interruptions at SCG activation/deactivation	297

7.32.2.18	Interruptions due to NR SRS antenna port switching	298

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	298

7.33.1	Introduction	298

7.33.2	Requirements	299

7.34	Void	299

7.35	Interruptions with SFTD measurements	299

7.35.1	Introduction	299

7.35.2	Requirements	299

7.36	Interruptions with NE-DC	299

7.36.1	Introduction	299

7.36.2	Requirements	300

7.36.2.1	Interruptions at transitions between active and non-active during DRX	300

7.36.2.2	Interruptions at transitions from non-DRX to DRX	300

7.36.2.3	Interruptions at SCell addition/release	300

7.36.2.4	Interruptions at SCell activation/deactivation	300

7.36.2.5	Interruptions during measurements on SCC	301

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	301

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	301

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	301

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	301

7.36.2.6	Interruptions at active BWP switching	301

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	302

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	302

7.36.2.9	Interruptions at SCell hibernation	302

7.36.2.10	Interruptions at direct SCell activation and hibernation	302

7.36.2.11	Interruptions at NR SRS carrier based switching	303

7.36.2.12	Interruptions at NR SCell dormancy	303

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	303

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	304

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	304

7.36.2.14	Interruption during NR measurement with autonomous gaps	304

7.36.2.15	Interruptions at SRS carrier based switching	305

7.36.2.16	Interruptions due to NR SRS antenna port switching	305

7.37	Interruptions during NR measurement with autonomous gaps	306

7.37.1	Introduction	306

7.37.2	Requirements	306

7.38 SCG Activation and Deactivation Delay	306

7.38.1	Introduction	306

7.38.2	SCG Activation Delay Requirement	307

7.38.3	SCG Deactivation Delay Requirement	307

8	UE Measurements Procedures in RRC_CONNECTED State	308

8.1	General Measurement Requirements	308

8.1.1	Introduction	308

8.1.2	Requirements	308

8.1.2.1	UE measurement capability	308

8.1.2.1.1	Monitoring of multiple layers using gaps	316

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	317

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	318

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	320

8.1.2.1.2	Network controlled small gap	321

8.1.2.2	E-UTRAN intra frequency measurements	322

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	322

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	327

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	331

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	332

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	333

8.1.2.3	E-UTRAN inter frequency measurements	334

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	334

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	340

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	346

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	346

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	347

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	348

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	349

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	350

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	351

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	358

8.1.2.4	Inter RAT measurements	358

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	358

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	363

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	363

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	367

8.1.2.4.5	E-UTRAN FDD – GSM measurements	367

8.1.2.4.6	E-UTRAN TDD – GSM measurements	372

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	372

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	374

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	374

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	374

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	375

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	375

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	375

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	375

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	377

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	377

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	377

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	377

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	378

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	378

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	380

8.1.2.4.21	E-UTRAN FDD – NR measurements	380

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	384

8.1.2.4.22	E-UTRAN TDD – NR measurements	388

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	388

8.1.2.4.23	Void	388

8.1.2.4.24	Void	388

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	388

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	390

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	390

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	391

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	391

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	391

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	393

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	395

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	397

8.1.2.6.5	Void	398

8.1.2.6.6	Void	398

8.1.2.6.7	Void	398

8.1.2.6.8	Void	398

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	398

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	399

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	400

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	402

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	404

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	406

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	407

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	409

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	411

8.1.2.7	E-UTRAN E-CID Measurements	413

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	413

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	414

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	415

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	416

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	416

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	417

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	419

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	422

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	426

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	429

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	429

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	430

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	430

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	431

8.1.2.10	Void	431

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	431

8.2.1	Introduction	431

8.2.2	Requirements	431

8.3	Measurements for E-UTRA carrier aggregation	436

8.3.1	Introduction	436

8.3.2	Measurements of the primary component carrier	436

8.3.3	Measurements of a secondary component carrier	436

8.3.3.1	Measurements of a secondary component carrier with active SCell	436

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	437

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	437

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	438

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	440

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	440

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	440

8.4.1	Introduction	440

8.4.2	Measurements on the primary component carrier	441

8.4.3	Measurements on a secondary component carrier	442

8.4.4	Measurements on both primary component carrier and a secondary component carrier	442

8.4.5	Measurements on different secondary component carriers	443

8.5	Measurements for UE category 0	444

8.5.1	Introduction	444

8.5.2 	Requirements	445

8.5.2.1 	E-UTRAN intra frequency measurements	445

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	445

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	448

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	450

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	454

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	454

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	455

8.6	Discovery signal measurements	456

8.6.1	Introduction	456

8.6.2	Requirements for CRS based discovery signal measurements	456

8.6.2.1	E-UTRAN intra frequency measurements	456

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	456

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	459

8.6.2.2	E-UTRAN inter frequency measurements	461

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	462

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	464

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	467

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	467

8.6.3	Requirements for CSI-RS based discovery signal measurements	467

8.6.3.1	E-UTRAN intra frequency measurements	467

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	468

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	470

8.6.3.2	E-UTRAN inter frequency measurements	472

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	473

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	475

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	478

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	478

8.7	Discovery signal measurements for E-UTRA carrier aggregation	478

8.7.1	Introduction	478

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	479

8.7.2.1	Measurements of the primary component carrier	479

8.7.2.2	Measurements of a secondary component carrier	479

8.7.2.3	Measurements of a secondary component carrier with active SCell	479

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	479

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	479

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	480

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	482

8.7.3.1	Measurements of the primary component carrier	482

8.7.3.2	Measurements of a secondary component carrier	482

8.7.3.3	Measurements of a secondary component carrier with active SCell	482

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	482

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	482

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	484

8.8	Measurements for E-UTRA dual connectivity	485

8.8.1	Introduction	485

8.8.2	Intra-frequency measurements requirements on PCell	485

8.8.3	Intra-frequency measurements requirements on PSCell	486

8.8.4	Inter-frequency and inter-RAT measurement requirements	486

8.8.5	Intra-frequency measurements with autonomous gaps	486

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	486

8.8.5.2	ECGI reporting delay	487

8.8.6	Inter-frequency measurements with autonomous gaps	487

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	487

8.8.6.2	ECGI reporting delay	488

8.8.7	SSTD Measurements	488

8.8.7.1	Introduction	488

8.8.7.2	SSTD Measurement requirements	488

8.8.7.3	SSTD Measurement Reporting Delay	489

8.8.8	Intra-frequency measurements requirements on SCell	489

8.9	MBSFN Measurements	489

8.9.1	Introduction	489

8.9.2	MBSFN RSRP Measurements	489

8.9.3	MBSFN RSRQ Measurements	490

8.9.4	MCH BLER Measurements	490

8.10	Proximity-based Services	490

8.10.1	Introduction	490

8.10.2	Requirements	490

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	490

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	491

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	491

8.11.1	Introduction	491

8.11.2	CRS based discovery signal measurements	492

8.11.2.1	E-UTRAN intra-frequency measurements	492

8.11.2.1.1	Requirements	492

8.11.2.1.1.1	Requirements when no DRX is used	492

8.11.2.1.1.1.1	Measurement Reporting Requirements	494

8.11.2.1.1.2	Requirements when DRX is used	495

8.11.2.1.1.2.1	Measurement Reporting Requirements	497

8.11.2.2	E-UTRAN inter-frequency measurements	498

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	498

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	502

8.11.3	CSI-RS based discovery signal measurements	502

8.11.3.1	E-UTRAN intra-frequency measurements	502

8.11.3.1.1	Requirements	502

8.11.3.1.1.1	Requirements when no DRX is used	502

8.11.3.1.1.1.1	Measurement Reporting Requirements	504

8.11.3.1.1.2	Requirements when DRX is used	504

8.11.3.1.1.2.1	Measurement Reporting Requirements	505

8.11.3.2	E-UTRAN inter-frequency measurements	506

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	506

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	510

8.11.4	RSSI measurements	510

8.11.4.1	E-UTRAN intra-frequency measurements	510

8.11.4.2	E-UTRAN inter-frequency measurements	510

8.11.5	Channel occupancy measurements	511

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	511

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	511

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	511

8.12.1	Introduction	511

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	511

8.12.2.1	Introduction	511

8.12.2.2	Measurements of a secondary component carrier	511

8.12.2.3	Measurements of a secondary component carrier with active SCell	511

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	512

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	512

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	514

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	517

8.12.3.1	Introduction	517

8.12.3.2	Measurements of a secondary component carrier	517

8.12.3.3	Measurements of a secondary component carrier with active SCell	517

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	517

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	517

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	519

8.13	Measurements for UE Category M1	521

8.13.1	Introduction	521

8.13.2	Requirements for UE category M1 with CE mode A	521

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	522

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	522

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	527

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	529

8.13.2.2	Void	533

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	533

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	533

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	536

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	538

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	539

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	539

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	541

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	543

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	544

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	544

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	544

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	545

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	545

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	545

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	546

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	546

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	547

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	548

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	548

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	549

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	553

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	555

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	559

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	559

8.13.3	Requirements for UE category M1 with CE mode B	560

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	560

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	561

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	565

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	568

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	573

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	574

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	574

8.13.3.2	Void	575

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	575

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	575

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	578

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	580

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	581

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	581

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	581

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	582

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	582

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	582

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	583

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	583

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	583

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	588

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	590

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	594

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	595

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	595

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	597

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	599

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	600

8.14	Measurements for UE category NB1	600

8.14.1	Introduction	600

8.14.2	NB-IoT intra frequency measurements under normal coverage	601

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	601

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	601

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	601

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	601

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	601

8.14.4	Connected mode channel quality report for UE Category NB1	601

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	602

8.14.6	NB-IoT neighbour cell measurements	602

8.14.6.1	Introduction	602

8.14.6.2	Requirements	602

8.14.6.3	Intra-frequency neighbour cell measurements	603

8.14.6.4	Inter-frequency neighbour cell measurements	603

8.15	Void	604

8.16	Measurements for UE Category M2	604

8.16.1	Introduction	604

8.16.2	Requirements for UE category M2 with CE mode A	605

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	605

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	605

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	605

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	606

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	606

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	607

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	607

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	607

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	610

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	612

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	613

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	613

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	615

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	617

8.16.3	Requirements for UE category M2 with CE mode B	618

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	618

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	618

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	621

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	623

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	624

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	624

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	626

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	628

8.17	Measurements for E-UTRA – NR Dual Connectivity	629

8.17.1	Introduction	629

8.17.1.1	Measurement Gap Sharing	629

8.17.1A	Intrafrequency Measurements	630

8.17.2	SFTD Measurements	630

8.17.2.1	Introduction	630

8.17.2.2	SFTD Measurement requirements	630

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	631

8.17.2.3	SFTD Measurement Reporting Delay	632

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	632

8.17.3.1	Introduction	632

8.17.3.2	E-UTRAN FDD inter frequency measurements	632

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	632

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	634

8.17.3.3	E-UTRAN TDD inter frequency measurements	636

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	636

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	638

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	640

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	640

8.17.4.1.1	NR Inter-RAT cell identification	640

8.17.4.1.2	NR Inter-RAT measurement	642

8.17.4.1.3	NR Inter-RAT measurement reporting	642

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	643

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.4A.1.1	NR Inter-RAT cell identification	644

8.17.4A.1.2	NR Inter-RAT measurement	645

8.17.4A.1.3	NR Inter-RAT measurement reporting	646

8.17.4A.1.4	NR inter-RAT RSSI measurements	646

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	647

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	647

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	647

8.17.5.1	Introduction	647

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	648

8.17.5.2.1	Identification of a new UTRA FDD cell	648

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	648

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	648

8.17.5.2.4	Periodic Reporting	649

8.17.5.2.5	Event Triggered Reporting	649

8.17.5.2.6	Event-triggered Periodic Reporting	649

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	649

8.17.5.3.1	Periodic Reporting	651

8.17.5.3.2	Event Triggered Reporting	651

8.17.5.3.3	Event-triggered Periodic Reporting	651

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	651

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	652

8.17.7.1	Introduction	652

8.17.7.2	Identification of a new UTRA FDD cell for SON	652

8.17.7.2.1	Requirements when no DRX is used	652

8.17.7.2.2	Requirements when DRX is used	652

8.17.7.2.3	Reporting Delay	653

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	654

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	654

8.17.9.1	Introduction	654

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	654

8.17.9.2.1	Identification of a new UTRA FDD cell	654

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	654

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	655

8.17.9.2.4	Periodic Reporting	655

8.17.9.2.5	Event Triggered Reporting	655

8.17.9.2.6	Event-triggered Periodic Reporting	656

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	656

8.17.9.3.1	Periodic Reporting	657

8.17.9.3.2	Event Triggered Reporting	657

8.17.9.3.3	Event-triggered Periodic Reporting	657

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	658

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	658

8.17.11.1	Introduction	658

8.17.11.2	Identification of a new UTRA TDD cell for SON	658

8.17.11.2.1	Requirements when no DRX is used	658

8.17.11.2.2	Requirements when DRX is used	658

8.17.11.2.3	Reporting Delay	659

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	659

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	660

8.17.13.1	Introduction	660

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	660

8.17.13.2.1	GSM carrier RSSI	660

8.17.13.2.2	BSIC verification	660

8.17.13.2.3	Enhanced BSIC verification	662

8.17.13.2.4	Periodic Reporting	662

8.17.13.2.5	Event Triggered Reporting	662

8.17.13.2.6	Event-triggered Periodic Reporting	663

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	663

8.17.13.3.1	GSM carrier RSSI	663

8.17.13.3.2	BSIC verification	663

8.17.13.3.3	Periodic Reporting	665

8.17.13.3.4	Event Triggered Reporting	665

8.17.13.3.5	Event-triggered Periodic Reporting	665

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	665

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	665

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	665

8.17.15.1.1	RSTD Measurement Reporting Delay	666

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	666

8.17.15.2.1	RSTD Measurement Reporting Delay	666

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	666

8.17.15.3.1	RSTD Measurement Reporting Delay	667

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	667

8.17.15.4.1	RSTD Measurement Reporting Delay	667

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	668

8.17.16.1	Introduction	668

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	668

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	668

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	668

8.17.17.1	Introduction	668

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	668

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	668

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	668

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	668

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	668

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	668

8.18	Measurements for non-BL/CE UE	668

8.18.1	Introduction	668

8.18.2	Requirements for non-BL/CE UE with CE Mode B	669

8.18.2.1	E-UTRAN intra frequency measurements	669

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	669

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	669

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	670

8.19	Measurements for NR – E-UTRA Dual Connectivity	671

8.19.1	Introduction	671

8.19.2	Intra-frequency Measurements	671

8.19.3	Inter-frequency Measurements	671

8.19.4	Void	671

8.19.5	Intra-frequency E-CID Measurements	671

8.19.6	Intra-frequency measurements with autonomous gaps	672

8.19.6.1	Introduction	672

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	672

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	672

8.19.7	Inter-frequency measurements with autonomous gaps	672

8.19.7.1	Introduction	672

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	672

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	672

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	672

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	672

9	Measurements performance requirements for UE	672

9.1	E-UTRAN measurements	672

9.1.1	Introduction	672

9.1.2	Intra-frequency RSRP Accuracy Requirements	673

9.1.2.1	Absolute RSRP Accuracy	673

9.1.2.2	Relative Accuracy of RSRP	673

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	674

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	675

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	676

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	677

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	678

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	679

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	680

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	680

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	680

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	681

9.1.2B.1	Introduction	681

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	681

9.1.3	Inter-frequency RSRP Accuracy Requirements	682

9.1.3.1	Absolute RSRP Accuracy	682

9.1.3.2	Relative Accuracy of RSRP	683

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	684

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	684

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	685

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	685

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	686

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	687

9.1.3B.1	Introduction	687

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	687

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	687

9.1.4	RSRP Measurement Report Mapping	688

9.1.5	Intra-frequency RSRQ Accuracy Requirements	688

9.1.5.1	Absolute RSRQ Accuracy	688

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	689

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	690

9.1.5.4	Absolute WB-RSRQ Accuracy	691

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	692

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	693

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	693

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	693

9.1.5B.1	Introduction	693

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	693

9.1.6	Inter-frequency RSRQ Accuracy Requirements	694

9.1.6.1	Absolute RSRQ Accuracy	694

9.1.6.2	Relative Accuracy of RSRQ	695

9.1.6.3	Absolute WB-RSRQ Accuracy	696

9.1.6.4	Relative WB-RSRQ Accuracy	696

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	697

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	698

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	698

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	698

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	699

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	700

9.1.6B.1	Introduction	700

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	700

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	700

9.1.7	RSRQ Measurement Report Mapping	701

9.1.8	Power Headroom	701

9.1.8.1	Period	702

9.1.8.2	Reporting Delay	702

9.1.8.3	Void	702

9.1.8.4	Report Mapping	702

9.1.9	UE Rx – Tx time difference	702

9.1.9.1	Measurement Requirement	702

9.1.9.2	Measurement Report mapping	703

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	704

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	705

9.1.10	Reference Signal Time Difference (RSTD)	706

9.1.10.1	Intra-Frequency Accuracy Requirement	706

9.1.10.2	Inter-Frequency Accuracy Requirement	707

9.1.10.3	RSTD Measurement Report Mapping	709

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	709

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	710

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	711

9.1.11	Carrier aggregation measurement accuracy	712

9.1.11.1	Primary component carrier accuracy requirement	713

9.1.11.2	Secondary component carrier accuracy requirement	713

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	713

9.1.11.4	Secondary component carrier relative accuracy requirement	713

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	713

9.1.13	Measurement accuracy for UE category 0	714

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	714

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	714

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	715

9.1.14	Accuracy requirements for Discovery Signal Measurements	716

9.1.14.1	Introduction	716

9.1.14.2	RSRP measurements in discovery signal occasions	716

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	716

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	716

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	716

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	717

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	718

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	718

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	718

9.1.14.3.3	CSI-RSRP measurement report mapping	719

9.1.14.4	RSRQ measurements in discovery signal occasions	719

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	719

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	720

9.1.15.1.1	Primary component carrier accuracy requirement	720

9.1.15.1.2	Secondary component carrier accuracy requirement	720

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	720

9.1.15.1.4	Secondary component carrier relative accuracy requirement	720

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	720

9.1.15.2.1	Primary component carrier accuracy requirement	720

9.1.15.2.2	Secondary component carrier accuracy requirement	720

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	720

9.1.15.2.4	Secondary component carrier relative accuracy requirement	720

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	721

9.1.17	RS-SINR Measurements	721

9.1.17.1	Measurement Report Mapping	721

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	722

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	722

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	722

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	722

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	723

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	724

9.1.18.1	Introduction	724

9.1.18.2	RSRP measurements	724

9.1.18.2.1	RSRP measurement report mapping	724

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	724

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	725

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	725

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	726

9.1.18.3	RSRQ measurements	726

9.1.18.3.1	RSRQ measurement report mapping	726

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	726

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	727

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	727

9.1.18.4	CSI-RSRP measurements	728

9.1.18.4.1	CSI-RSRP measurement report mapping	728

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	728

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	728

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	729

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	729

9.1.18.5	RSSI measurements	730

9.1.18.5.1	RSSI measurement report mapping	730

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	730

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	731

9.1.18.6	Channel occupancy measurements	731

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	731

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	731

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	732

9.1.19.1	Introduction	732

9.1.19.2	Accuracy requirements for measurements on SCC	732

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	732

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	732

9.1.20	SFN and Subframe Time Difference (SSTD)	733

9.1.20.1	SSTD Accuracy Requirement	733

9.1.20.2	SSTD Measurement Report Mapping	733

9.1.21	Measurement accuracy for UE category M1	734

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	734

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	736

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	737

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	739

9.1.21.5	RSRP Measurement Report Mapping	740

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	740

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	741

9.1.21.8	RSRQ Measurement Report Mapping	742

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	742

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	743

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	744

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	745

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	746

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	747

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	748

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	749

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	750

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	751

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	752

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	753

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	756

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	759

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	759

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	762

9.1.22	Measurement accuracy for UE Category NB1	763

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	763

9.1.22.2	Void	764

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	764

9.1.22.4	Void	765

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	765

9.1.22.6	Void	766

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	766

9.1.22.8	Void	767

9.1.22.9	NRSRP Measurement Report Mapping	767

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	767

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	768

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	769

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	770

9.1.22.14	NRSRQ Measurement Report Mapping	771

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	772

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	772

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	773

9.1.23	Power Headroom for UE Category NB1	773

9.1.23.1	Period	774

9.1.23.2	Reporting Delay	774

9.1.23.3	Report Mapping for UE Category NB1	774

9.1.23.3.1	Void	775

9.1.23.3.2	Void	775

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	775

9.1.24	Void	776

9.1.25	Measurement accuracy for UE category M2	776

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	776

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	777

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	778

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	779

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	781

9.1.26	Measurement Accuracy for non-BL CE UE	782

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	783

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	785

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	785

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	787

9.1.26.5	RSRP Measurement Report Mapping	788

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	788

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	788

9.1.26.8	RSRQ Measurement Report Mapping	789

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	789

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	790

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	791

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	792

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	793

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	793

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	793

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	794

9.1.27	SFN and frame Timing Difference (SFTD)	795

9.1.27.1	SFTD Accuracy Requirement	795

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	797

9.1.28.1	SFTD Accuracy Requirement under CCA	797

9.2	UTRAN FDD Measurements	798

9.2.1	UTRAN FDD CPICH RSCP	798

9.2.2	Void	799

9.2.3	UTRAN FDD CPICH Ec/No	799

9.3	UTRAN TDD Measurements	799

9.3.1	UTRAN TDD P-CCPCH RSCP	800

9.3.2	Void	800

9.3.3	Void	800

9.4	GSM Measurements	800

9.4.1	GSM carrier RSSI	800

9.5	CDMA2000 1x RTT Measurements	800

9.5.1	CDMA2000 1x RTT Pilot Strength	800

9.6	PCMAX,c	801

9.6.1	Report Mapping	801

9.6.2 	Estimation Period	801

9.6.3	Reporting Delay	801

9.7	IEEE802.11 Measurements	801

9.7.1	WLAN RSSI	801

9.7.2	WLAN RSSI Measurement Report Mapping	801

9.8	MBSFN Measurements	802

9.8.1	Introduction	802

9.8.2	MBSFN RSRP	802

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	802

9.8.2.2	MBSFN RSRP measurement report mapping	803

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	803

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	804

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	804

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	804

9.8.3	MBSFN RSRQ	805

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	805

9.8.3.2	MBSFN RSRQ measurement report mapping	805

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	806

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	806

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	806

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	807

9.8.4	MCH BLER	807

9.8.4.1	Measurement report mapping for MCH BLER	807

9.8.4.2	Measurement report mapping for MCH Block Number	808

9.9	ProSe Measurements	809

9.9.1	Introduction	809

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	809

9.9.2.1	Absolute S-RSRP Accuracy	809

9.9.2.2	Relative Accuracy of S-RSRP	810

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	811

9.9.3.1	Absolute SD-RSRP Accuracy	811

9.9.3.2	Relative Accuracy of SD-RSRP	811

9.10	V2X Measurements	812

9.10.1	Introduction	812

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	812

9.10.2.1	Absolute S-RSRP Accuracy	812

9.10.2.2	Relative Accuracy of S-RSRP	813

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	813

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	813

9.10.4	S-RSSI Measurement Accuracy Requirements	814

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	814

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	814

9.11	NR Measurements	815

9.11.1	NR SS-RSRP Measurements	815

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	815

9.11.2	NR SS-RSRQ Measurements	816

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	816

9.11.3	NR SS-SINR Measurements	816

9.11.4	NR SS-RSRP Measurements under CCA	817

9.11.5	NR SS-RSRQ Measurements under CCA	817

9.11.6	NR SS-SINR Measurements under CCA	817

9.11.7	NR RSSI Measurements under CCA	817

9.11.8	NR Channel Occupancy Measurements under CCA	818

10	Measurements Performance Requirements for E-UTRAN	818

10.1	Received Interference Power	818

10.1.1	Absolute accuracy requirement	818

10.1.2	Relative accuracy requirement	818

10.1.3	Received Interference Power measurement report mapping	819

10.2	Angle of Arrival (AOA)	819

10.2.1	Range/mapping	819

10.3	Timing Advance (TADV)	819

10.3.1	Report mapping	819

11	ProSe Requirements in Any Cell Selection state	820

11.1	Introduction	820

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	820

11.2.1	Introduction	820

11.2.2	ProSe UE transmission timing	820

11.3	Initiation/Cease of SLSS Transmissions	821

11.3.1	Introduction	821

11.3.2	Requirements	821

11.4	Measurements for ProSe in Any Cell Selection State	821

11.4.1	Introduction	821

11.4.2	Requirements	821

11.4.2.1	E-UTRA FDD	821

11.4.2.2	E-UTRA TDD	822

11.5	Selection / Reselection of ProSe Synchronization Reference	822

11.5.1	Introduction	822

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	822

11.5.2.1	Introduction	822

11.5.2.2	Requirements	822

11.6	Void	823

11.7	Selection / Reselection of ProSe relay UE	823

11.7.1	Introduction	823

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	823

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	823

12.1	Introduction	823

12.2	Transmit Timing	824

12.2.1	GNSS as timing reference	824

12.3	Interruption	824

12.4	Reliability of GNSS signal	824

13	V2X Requirements	824

13.1	Introduction	824

13.2	UE Transmit Timing	825

13.2.1	Introduction	825

13.2.2	GNSS as synchronization reference source	825

13.2.3	Serving cell/PCell as synchronization reference source	825

13.2.4	SyncRef UE as synchronization reference source	825

13.3	Initiation/Cease of SLSS Transmissions	825

13.3.1	Introduction	825

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	825

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	826

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	826

13.4	Selection / Reselection of V2X Synchronization Reference Source	827

13.5	Autonomous Resource Selection/Reselection measurements	828

13.5.1	Introduction	828

13.5.2	PSSCH-RSRP measurements	828

13.5.3	S-RSSI measurements	828

13.6	Congestion Control measurements	828

13.7	Interruption	828

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	828

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	828

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	829

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	829

13.8	Reliability of GNSS signal	829

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	830

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	830

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1017

A.4.2.42.1	Test Purpose and Environment	1017

A.4.2.42.2	Test Requirements	1019

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1020

A.4.2.43.1	Test Purpose and Environment	1020

A.4.2.43.2	Test Requirements	1022

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1023

A.4.2.44.1	Test Purpose and Environment	1023

A.4.2.44.2	Test Requirements	1025

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1026

A.4.2.45.1	Test Purpose and Environment	1026

A.4.2.45.2	Test Requirements	1028

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1029

A.4.2.46.1	Test Purpose and Environment	1029

A.4.2.46.2	Test Requirements	1031

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1032

A.4.2.47.1	Test Purpose and Environment	1032

A.4.2.47.2	Test Requirements	1034

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1035

A.4.2.48.1	Test Purpose and Environment	1035

A.4.2.48.2	Test Requirements	1036

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1037

A.4.2.49.1	Test Purpose and Environment	1037

A.4.2.49.2	Test Requirements	1039

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1040

A.4.2.50.1	Test Purpose and Environment	1040

A.4.2.50.2	Test Requirements	1042

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1043

A.4.2.51.1	Test Purpose and Environment	1043

A.4.2.51.2	Test Requirements	1045

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1046

A.4.2.52.1	Test Purpose and Environment	1046

A.4.2.52.2	Test Requirements	1048

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1049

A.4.2.53.1	Test Purpose and Environment	1049

A.4.2.53.2	Test Requirements	1051

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1052

A.4.2.54.1	Test Purpose and Environment	1052

A.4.2.54.2	Test Requirements	1054

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1055

A.4.2.55.1	Test Purpose and Environment	1055

A.4.2.55.2	Test Requirements	1057

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1058

A.4.2.56.1	Test Purpose and Environment	1058

A.4.2.56.2	Test Requirements	1059

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1060

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1060

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1060

A.4.3.1.1.1	Test Purpose and Environment	1060

A.4.3.1.1.2	Test Requirements	1062

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1062

A.4.3.1.2.1	Test Purpose and Environment	1062

A.4.3.1.2.2	Test Requirements	1065

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1065

A.4.3.1.3.1	Test Purpose and Environment	1065

A.4.3.1.3.2	Test Requirements	1068

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1068

A.4.3.1.4.1	Test Purpose and Environment	1068

A.4.3.1.4.2	Test Requirements	1069

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1069

A.4.3.1.5.1	Test Purpose and Environment	1069

A.4.3.1.5.2	Test Requirements	1072

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1073

A.4.3.2.1	Test Purpose and Environment	1073

A.4.3.2.1.1	Void	1073

A.4.3.2.1.2	1.28Mcps TDD option	1073

A.4.3.2.1.3	Void	1075

A.4.3.2.2	Test Requirements	1075

A.4.3.2.2.1	1.28Mcps TDD option	1075

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1075

A.4.3.2A.1	Test Purpose and Environment	1075

A.4.3.2A.2	Test Requirements	1079

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1079

A.4.3.3.1	Test Purpose and Environment	1079

A.4.3.3.2	Test Requirements	1082

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1082

A.4.3.3A.1	Test Purpose and Environment	1082

A.4.3.3A.2	Test Requirements	1087

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1088

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1088

A.4.3.4.1.1	Test Purpose and Environment	1088

A.4.3.4.1.2	Test Requirements	1090

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1090

A.4.3.4.2.1	Test Purpose and Environment	1090

A.4.3.4.2.2	Test Requirements	1092

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1092

A.4.3.4.3.1	Test Purpose and Environment	1092

A.4.3.4.3.2	Test Requirements	1095

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1095

A.4.3.4.4.1	Test Purpose and Environment	1095

A.4.3.4.4.2	Test Requirements	1099

A.4.4	E-UTRAN to GSM Cell Re-Selection	1099

A.4.4.1	E-UTRAN FDD – GSM:	1099

A.4.4.1.1	Test Purpose and Environment	1099

A.4.4.1.2	Test Requirements	1101

A.4.4.2	E-UTRAN TDD – GSM:	1101

A.4.4.2.1	Test Purpose and Environment	1101

A.4.4.2.2	Test Requirements	1103

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1104

A.4.5.1	E-UTRAN FDD – HRPD	1104

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1104

A.4.5.1.1.1	Test Purpose and Environment	1104

A.4.5.1.1.2	Test Requirements	1106

A.4.5.2	E-UTRAN TDD – HRPD	1106

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1106

A.4.5.2.1.1	Test Purpose and Environment	1106

A.4.5.2.1.2	Test Requirements	1109

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1109

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1109

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1109

A.4.6.1.1.1	Test Purpose and Environment	1109

A.4.6.1.1.2	Test Requirements	1112

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1112

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1112

A.4.6.2.1.1	Test Purpose and Environment	1112

A.4.6.2.1.2	Test Requirements	1115

A.4.7	Idle State Positioning Measurement for UE category NB1	1115

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1115

A.4.7.1.1	Test Purpose and Environment	1115

A.4.7.1.2	Test Requirements	1120

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1121

A.4.7.2.1	Test Purpose and Environment	1121

A.4.7.4.2	Test Requirements	1125

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1126

A.4.7.3.1	Test Purpose and Environment	1126

A.4.7.3.2	Test Requirements	1130

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1131

A.4.7.4.1	Test Purpose and Environment	1131

A.4.7.4.2	Test Requirements	1135

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1136

A.5.1	E-UTRAN Handover	1136

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1136

A.5.1.1.1	Test Purpose and Environment	1136

A.5.1.1.2	Test Requirements	1138

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1138

A.5.1.2.1	Test Purpose and Environment	1138

A.5.1.2.2	Test Requirements	1140

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1140

A.5.1.3.1	Test Purpose and Environment	1140

A.5.1.3.2	Test Requirements	1142

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1142

A.5.1.4.1	Test Purpose and Environment	1142

A.5.1.4.2	Test Requirements	1144

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1144

A.5.1.5.1	Test Purpose and Environment	1144

A.5.1.5.2	Test Requirements	1146

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1146

A.5.1.6.1	Test Purpose and Environment	1146

A.5.1.6.2	Test Requirements	1148

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1148

A.5.1.7.1	Test Purpose and Environment	1148

A.5.1.7.2	Test Requirements	1151

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1151

A.5.1.8.1	Test Purpose and Environment	1151

A.5.1.8.2	Test Requirements	1154

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1154

A.5.1.9.1	Test Purpose and Environment	1154

A.5.1.9.2	Test Requirements	1155

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1155

A.5.1.10.1	Test Purpose and Environment	1155

A.5.1.10.2	Test Requirements	1157

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1157

A.5.1.11.1	Test Purpose and Environment	1157

A.5.1.11.2	Test Requirements	1159

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1159

A.5.1.12.1	Test Purpose and Environment	1159

A.5.1.12.2	Test Requirements	1161

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1161

A.5.1.13.1	Test Purpose and Environment	1161

A.5.1.13.2	Test Requirements	1163

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1164

A.5.1.14.1	Test Purpose and Environment	1164

A.5.1.14.2	Test Requirements	1165

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1166

A.5.1.15.1	Test Purpose and Environment	1166

A.5.1.15.2	Test Requirements	1167

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1168

A.5.1.16.1	Test Purpose and Environment	1168

A.5.1.16.2	Test Requirements	1169

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1170

A.5.1.17.1	Test Purpose and Environment	1170

A.5.1.17.2	Test Requirements	1171

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1172

A.5.1.18.1	Test Purpose and Environment	1172

A.5.1.18.2	Test Requirements	1173

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1174

A.5.1.19.1	Test Purpose and Environment	1174

A.5.1.19.2	Test Requirements	1175

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1176

A.5.1.20.1	Test Purpose and Environment	1176

A.5.1.20.2	Test Requirements	1177

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1178

A.5.1.21.1	Test Purpose and Environment	1178

A.5.1.21.2	Test Requirements	1179

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1179

A.5.1.22.1	Test Purpose and Environment	1179

A.5.1.22.2	Test Requirements	1181

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1181

A.5.1.23.1	Test Purpose and Environment	1181

A.5.1.23.2	Test Requirements	1183

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1183

A.5.1.24.1	Test Purpose and Environment	1183

A.5.1.24.2	Test Requirements	1185

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1185

A.5.1.25.1	Test Purpose and Environment	1185

A.5.1.25.2	Test Requirements	1187

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1188

A.5.1.26.1	Test Purpose and Environment	1188

A.5.1.26.2	Test Requirements	1189

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1190

A.5.1.27.1	Test Purpose and Environment	1190

A.5.1.27.2	Test Requirements	1191

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1192

A.5.1.28.1	Test Purpose and Environment	1192

A.5.1.28.2	Test Requirements	1193

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1194

A.5.1.29.1	Test Purpose and Environment	1194

A.5.1.29.2	Test Requirements	1195

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1196

A.5.1.30.1	Test Purpose and Environment	1196

A.5.1.30.2	Test Requirements	1197

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1198

A.5.1.31.1	Test Purpose and Environment	1198

A.5.1.31.2	Test Requirements	1199

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1200

A.5.1.32.1	Test Purpose and Environment	1200

A.5.1.32.2	Test Requirements	1201

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1202

A.5.1.33.1	Test Purpose and Environment	1202

A.5.1.13.2	Test Requirements	1203

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1204

A.5.1.34.1	Test Purpose and Environment	1204

A.5.1.34.2	Test Requirements	1205

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1206

A.5.1.35.1	Test Purpose and Environment	1206

A.5.1.35.2	Test Requirements	1207

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1208

A.5.1.36.1	Test Purpose and Environment	1208

A.5.1.36.2	Test Requirements	1209

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1210

A.5.1.37.1	Test Purpose and Environment	1210

A.5.1.37.2	Test Requirements	1211

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1212

A.5.1.38.1	Test Purpose and Environment	1212

A.5.1.38.2	Test Requirements	1213

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1214

A.5.1.39.1	Test Purpose and Environment	1214

A.5.1.39.2	Test Requirements	1215

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1216

A.5.1.40.1	Test Purpose and Environment	1216

A.5.1.40.2	Test Requirements	1217

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1218

A.5.1.41.1	Test Purpose and Environment	1218

A.5.1.41.2	Test Requirements	1220

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1221

A.5.1.42.1	Test Purpose and Environment	1221

A.5.1.42.2	Test Requirements	1222

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1223

A.5.1.43.1	Test Purpose and Environment	1223

A.5.1.43.2	Test Requirements	1224

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1225

A.5.1.44.1	Test Purpose and Environment	1225

A.5.1.44.2	Test Requirements	1226

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1227

A.5.1.45.1	Test Purpose and Environment	1227

A.5.1.45.1	Test Requirements	1228

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1229

A.5.1.46.1	Test Purpose and Environment	1229

A.5.1.46.2	Test Requirements	1230

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1231

A.5.1.47.1	Test Purpose and Environment	1231

A.5.1.47.2	Test Requirements	1232

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1233

A.5.1.48.1	Test Purpose and Environment	1233

A.5.1.48.2	Test Requirements	1234

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1235

A.5.1.49.1	Test Purpose and Environment	1235

A.5.1.49.2	Test Requirements	1236

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1237

A.5.1.50.1	Test Purpose and Environment	1237

A.5.1.50.2	Test Requirements	1238

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1239

A.5.1.51.1	Test Purpose and Environment	1239

A.5.1.51.2	Test Requirements	1241

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1241

A.5.152.1	Test Purpose and Environment	1241

A.5.1.52.2	Test Requirements	1244

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1245

A.5.1.53.1	Test Purpose and Environment	1245

A.5.1.53.2	Test Requirements	1247

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1248

A.5.1.54.1	Test Purpose and Environment	1248

A.5.1.54.2	Test Requirements	1250

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1251

A.5.1.55.1	Test Purpose and Environment	1251

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1255

A.5.1.56.1	Test Purpose and Environment	1255

A.5.1.56.2	Test Requirements	1258

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1259

A.5.1.57.1	Test Purpose and Environment	1259

A.5.1.57.2	Test Requirements	1261

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1261

A.5.1.58.1	Test Purpose and Environment	1261

A.5.1.58.2	Test Requirements	1264

A.5.2	E-UTRAN Handover to other RATs	1264

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1264

A.5.2.1.1	Test Purpose and Environment	1264

A.5.2.1.2	Test Requirements	1266

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1267

A.5.2.2.1	Test Purpose and Environment	1267

A.5.2.2.2	Test Requirements	1270

A.5.2.3 E-UTRAN FDD- GSM Handover	1270

A.5.2.3.1	Test Purpose and Environment	1270

A.5.2.3.2	Test Requirements	1271

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1272

A.5.2.4.1	Test Purpose and Environment	1272

A.5.2.4.1.1	Void	1272

A.5.2.4.1.2	1.28 Mcps TDD option	1272

A.5.2.4.1.3	Void	1274

A.5.2.4.2	Test Requirements	1274

A.5.2.4.2.1	Void	1274

A.5.2.4.2.2	1.28 Mcps TDD option	1274

A.5.2.4.2.3	Void	1274

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1274

A.5.2.5.1	Test Purpose and Environment	1274

A.5.2.5.1.3	Void	1277

A.5.2.5.2	Test Requirements	1277

A.5.2.5.2.1	Void	1277

A.5.2.5.2.2	1.28 Mcps TDD option	1277

A.5.2.5.2.3	Void	1277

A.5.2.6	E-UTRAN TDD - GSM Handover	1277

A.5.2.6.1	Test Purpose and Environment	1277

A.5.2.6.2	Test Requirements	1279

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1280

A.5.2.7.1	Test Purpose and Environment	1280

A.5.2.7.2	Test Requirements	1282

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1282

A.5.2.8.1	Test Purpose and Environment	1282

A.5.2.8.2	Test Requirements	1283

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1284

A.5.2.9.1	Test Purpose and Environment	1284

A.5.2.9.2	Test Requirements	1285

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1286

A.5.2.10.1	Test Purpose and Environment	1286

A.5.2.10.2	Test Requirements	1288

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1288

A.5.2.10A.1	Test Purpose and Environment	1288

A.5.2.10A.2	Test Requirements	1291

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1291

A.5.2.10B.1	Test Purpose and Environment	1291

A.5.2.10B.2	Test Requirements	1294

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1294

A.5.2.11.1	Test Purpose and Environment	1294

A.5.2.11.2	Test Requirements	1294

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1295

A.5.3.1	E-UTRAN FDD – HRPD Handover	1295

A.5.3.1.1	Test Purpose and Environment	1295

A.5.3.1.2	Test Requirements	1297

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1297

A.5.3.2.1	Test Purpose and Environment	1297

A.5.3.2.2	Test Requirements	1300

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1300

A.5.3.3.1	Test Purpose and Environment	1300

A.5.3.3.2	Test Requirements	1303

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1303

A.5.3.4.1	Test Purpose and Environment	1303

A.5.3.4.2	Test Requirements	1304

A.5.3.5	E-UTRAN TDD – HRPD Handover	1305

A.5.3.5.1	Test Purpose and Environment	1305

A.5.3.5.2	Test Requirements	1308

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1308

A.5.3.6.1	Test Purpose and Environment	1308

A.5.3.6.2	Test Requirements	1311

A.6	RRC Connection Control	1311

A.6.1	RRC Re-establishment	1311

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1311

A.6.1.1.1	Test Purpose and Environment	1311

A.6.1.1.2	Test Requirements	1312

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1313

A.6.1.2.1	Test Purpose and Environment	1313

A.6.1.2.2	Test Requirements	1314

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1315

A.6.1.3.1	Test Purpose and Environment	1315

A.6.1.3.2	Test Requirements	1316

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1317

A.6.1.4.1	Test Purpose and Environment	1317

A.6.1.4.2	Test Requirements	1318

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1319

A.6.1.5.1	Test Purpose and Environment	1319

A.6.1.5.2	Test Requirements	1319

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1319

A.6.1.6.1	Test Purpose and Environment	1319

A.6.1.6.2	Test Requirements	1321

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1322

A.6.1.7.1	Test Purpose and Environment	1322

A.6.1.7.2	Test Requirements	1323

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1324

A.6.1.8.1	Test Purpose and Environment	1324

A.6.1.8.2	Test Requirements	1325

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1326

A.6.1.9.1	Test Purpose and Environment	1326

A.6.1.9.2	Test Requirements	1327

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1328

A.6.1.10.1	Test Purpose and Environment	1328

A.6.1.10.2	Test Requirements	1329

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1330

A.6.1.11.1	Test Purpose and Environment	1330

A.6.1.11.2	Test Requirements	1331

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1332

A.6.1.12.1	Test Purpose and Environment	1332

A.6.1.12.2	Test Requirements	1333

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1334

A.6.1.13.1	Test Purpose and Environment	1334

A.6.1.13.2	Test Requirements	1335

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1336

A.6.1.14.1	Test Purpose and Environment	1336

A.6.1.14.2	Test Requirements	1337

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1338

A.6.1.15.1	Test Purpose and Environment	1338

A.6.1.15.2	Test Requirements	1340

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1341

A.6.1.16.1	Test Purpose and Environment	1341

A.6.1.16.2	Test Requirements	1343

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1344

A.6.1.17.1	Test Purpose and Environment	1344

A.6.1.17.2	Test Requirements	1345

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1346

A.6.1.18.1	Test Purpose and Environment	1346

A.6.1.18.2	Test Requirements	1347

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.19.1	Test Purpose and Environment	1348

A.6.1.19.2	Test Requirements	1349

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1350

A.6.1.20.1	Test Purpose and Environment	1350

A.6.1.20.2	Test Requirements	1351

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1352

A.6.1.21.1	Test Purpose and Environment	1352

A.6.1.21.2	Test Requirements	1353

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.22.1	Test Purpose and Environment	1354

A.6.1.22.2	Test Requirements	1355

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1356

A.6.1.23.1	Test Purpose and Environment	1356

A.6.1.23.2	Test Requirements	1358

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1359

A.6.1.24.1	Test Purpose and Environment	1359

A.6.1.24.2	Test Requirements	1361

A.6.2	Random Access	1362

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1362

A.6.2.1.1	Test Purpose and Environment	1362

A.6.2.1.2	Test Requirements	1363

A.6.2.1.2.1	Random Access Response Reception	1363

A.6.2.1.2.2	No Random Access Response Reception	1363

A.6.2.1.2.3	Receiving a NACK on msg3	1363

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1364

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1364

A.6.2.1.2.6	Contention Resolution Timer expiry	1364

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1364

A.6.2.2.1	Test Purpose and Environment	1364

A.6.2.2.2	Test Requirements	1365

A.6.2.2.2.1	Random Access Response Reception	1366

A.6.2.2.2.2	No Random Access Response Reception	1366

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1366

A.6.2.3.1	Test Purpose and Environment	1366

A.6.2.3.2	Test Requirements	1368

A.6.2.3.2.1	Random Access Response Reception	1368

A.6.2.3.2.2	No Random Access Response reception	1368

A.6.2.3.2.3	Receiving a NACK on msg3	1368

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1369

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1369

A.6.2.3.2.6	Contention Resolution Timer expiry	1369

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1369

A.6.2.4.1	Test Purpose and Environment	1369

A.6.2.4.2	Test Requirements	1371

A.6.2.4.2.1	Random Access Response Reception	1371

A.6.2.4.2.2	No Random Access Response Reception	1371

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.5.1	Test Purpose and Environment	1371

A.6.2.5.2	Test Requirements	1372

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1372

A.6.2.6.1	Test Purpose and Environment	1372

A.6.2.6.2	Test Requirements	1372

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1373

A.6.2.7.1	Test Purpose and Environment	1373

A.6.2.7.2	Test Requirements	1375

A.6.2.7.2.1	Random Access Response Reception	1375

A.6.2.7.2.2	No Random Access Response Reception	1375

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1375

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1376

A.6.2.8.1	Test Purpose and Environment	1376

A.6.2.8.2	Test Requirements	1378

A.6.2.8.2.1	Random Access Response Reception	1378

A.6.2.8.2.2	No Random Access Response Reception	1378

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1378

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1379

A.6.2.9.1	Test Purpose and Environment	1379

A.6.2.9.2	Test Requirements	1382

A.6.2.9.2.1	Random Access Response Reception	1382

A.6.2.9.2.2	No Random Access Response Reception	1383

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1383

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1384

A.6.2.10.1	Test Purpose and Environment	1384

A.6.2.10.2	Test Requirements	1386

A.6.2.10.2.1	Random Access Response Reception	1386

A.6.2.10.2.2	No Random Access Response Reception	1387

A.6.2.10.2.3	Receiving a NACK on msg3	1387

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1387

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1387

A.6.2.10.2.6	Contention Resolution Timer expiry	1387

A.6.2.10.2.7	PRACH Resource Selection	1388

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1388

A.6.2.11.1	Test Purpose and Environment	1388

A.6.2.11.2	Test Requirements	1390

A.6.2.11.2.1	Random Access Response Reception	1390

A.6.2.11.2.2	No Random Access Response Reception	1391

A.6.2.11.2.3	Receiving a NACK on msg3	1391

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1391

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1391

A.6.2.11.2.6	Contention Resolution Timer expiry	1391

A.6.2.11.2.7	PRACH Resource Selection	1392

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1392

A.6.2.12.1	Test Purpose and Environment	1392

A.6.2.12.2	Test Requirements	1394

A.6.2.12.2.1	Random Access Response Reception	1394

A.6.2.12.2.2	No Random Access Response Reception	1395

A.6.2.12.2.3	Receiving a NACK on msg3	1395

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.12.2.6	Contention Resolution Timer expiry	1395

A.6.2.12.2.7	PRACH Resource Selection	1396

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1396

A.6.2.13.1	Test Purpose and Environment	1396

A.6.2.13.2	Test Requirements	1398

A.6.2.13.2.1	Random Access Response Reception	1398

A.6.2.13.2.2	No Random Access Response Reception	1399

A.6.2.13.2.3	Receiving a NACK on msg3	1399

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.13.2.6	Contention Resolution Timer expiry	1399

A.6.2.13.2.7	PRACH Resource Selection	1400

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1400

A.6.2.14.1	Test Purpose and Environment	1400

A.6.2.14.2	Test Requirements	1402

A.6.2.14.2.1	Random Access Response Reception	1402

A.6.2.14.2.2	No Random Access Response Reception	1403

A.6.2.14.2.3	Receiving a NACK on msg3	1403

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1403

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1403

A.6.2.14.2.6	Contention Resolution Timer expiry	1403

A.6.2.14.2.7	PRACH Resource Selection	1404

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1404

A.6.2.15.1	Test Purpose and Environment	1404

A.6.2.15.2	Test Requirements	1406

A.6.2.15.2.1	Random Access Response Reception	1406

A.6.2.15.2.2	No Random Access Response Reception	1407

A.6.2.15.2.3	Receiving a NACK on msg3	1407

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1407

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1407

A.6.2.15.2.6	Contention Resolution Timer expiry	1407

A.6.2.15.2.7	PRACH Resource Selection	1408

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1408

A.6.2.16.1	Test Purpose and Environment	1408

A.6.2.16.2	Test Requirements	1411

A.6.2.16.2.1	Random Access Response Reception	1411

A.6.2.16.2.2	No Random Access Response Reception	1412

A.6.2.16.2.3	Receiving a NACK on msg3	1412

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1412

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1412

A.6.2.16.2.6	Contention Resolution Timer expiry	1412

A.6.2.16.2.7	NPRACH Resource Selection	1412

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1413

A.6.2.17.1	Test Purpose and Environment	1413

A.6.2.17.2	Test Requirements	1416

A.6.2.17.2.1	Random Access Response Reception	1416

A.6.2.17.2.2	No Random Access Response Reception	1417

A.6.2.17.2.3	Receiving a NACK on msg3	1417

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.17.2.6	Contention Resolution Timer expiry	1417

A.6.2.17.2.7	NPRACH Resource Selection	1417

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1418

A.6.2.18.1	Test Purpose and Environment	1418

A.6.2.18.2	Test Requirements	1421

A.6.2.18.2.1	Random Access Response Reception	1421

A.6.2.18.2.2	No Random Access Response Reception	1422

A.6.2.18.2.3	Receiving a NACK on msg3	1422

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1422

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1422

A.6.2.18.2.6	Contention Resolution Timer expiry	1422

A.6.2.18.2.7	NPRACH Resource Selection	1422

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1423

A.6.2.19.1	Test Purpose and Environment	1423

A.6.2.19.2	Test Requirements	1425

A.6.2.19.2.1	Random Access Response Reception	1425

A.6.2.19.2.2	No Random Access Response Reception	1426

A.6.2.19.2.3	Receiving a NACK on msg3	1426

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.19.2.6	Contention Resolution Timer expiry	1426

A.6.2.19.2.7	NPRACH Resource Selection	1426

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1427

A.6.2.20.1	Test Purpose and Environment	1427

A.6.2.20.2	Test Requirements	1429

A.6.2.20.2.1	Random Access Response Reception	1429

A.6.2.20.2.2	No Random Access Response Reception	1430

A.6.2.20.2.3	Receiving a NACK on msg3	1430

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.20.2.6	Contention Resolution Timer expiry	1430

A.6.2.20.2.7	NPRACH Resource Selection	1430

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.21.1	Test Purpose and Environment	1431

A.6.2.21.2	Test Requirements	1433

A.6.2.21.2.1	Random Access Response Reception	1433

A.6.2.21.2.2	No Random Access Response Reception	1434

A.6.2.21.2.3	Receiving a NACK on msg3	1434

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.21.2.6	Contention Resolution Timer expiry	1434

A.6.2.21.2.7	NPRACH Resource Selection	1434

A.6.3	RRC Connection Release with Redirection	1435

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1435

A.6.3.1.1	Test Purpose and Environment	1435

A.6.3.1.2	Test Requirements	1437

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1437

A.6.3.2.1	Test Purpose and Environment	1437

A.6.3.2.2	Test Requirements	1439

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1439

A.6.3.3.1	Test Purpose and Environment	1439

A.6.3.3.2	Test Requirements	1440

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1441

A.6.3.4.1	Test Purpose and Environment	1441

A.6.3.4.2	Test Requirements	1442

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1443

A.6.3.5.1	Test Purpose and Environment	1443

A.6.3.5.2	Test Requirements	1445

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1445

A.6.3.6.1	Test Purpose and Environment	1445

A. 6.3.6.2	Test Requirements	1448

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1448

A.6.3.7.1	Test Purpose and Environment	1448

A.6.3.7.2	Test Requirements	1451

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1451

A.6.3.8.1	Test Purpose and Environment	1451

A.6.3.8.2	Test Requirements	1454

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1454

A.6.3.9.1	Test Purpose and Environment	1454

A.6.3.9.2	Test Requirements	1456

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1456

A.6.3.10.1	Test Purpose and Environment	1456

A.6.3.10.2	Test Requirements	1458

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1458

A.6.3.11.1	Test Purpose and Environment	1458

A.6.3.11.2	Test Requirements	1460

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1460

A.6.3.12.1	Test Purpose and Environment	1460

A.6.3.12.2	Test Requirements	1463

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1525

A.7.2.13.1	Test Purpose and Environment	1525

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1527

A.7.2.14.1	Test Purpose and Environment	1527

A.7.2.14.2	Test Requirements	1529

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1529

A.7.2.15.1	Test Purpose and Environment	1529

A.7.2.15.2	Test Requirements	1531

A.7.3	Radio Link Monitoring	1531

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1539

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1539

A.7.3.3.1	Test Purpose and Environment	1539

A.7.3.3.2	Test Requirements	1543

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1543

A.7.3.4.1	Test Purpose and Environment	1543

A.7.3.4.2	Test Requirements	1547

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1547

A.7.3.5.1	Test Purpose and Environment	1547

A.7.3.5.2	Test Requirements	1550

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1551

A.7.3.6.1	Test Purpose and Environment	1551

A.7.3.6.2	Test Requirements	1554

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1554

A.7.3.7.1	Test Purpose and Environment	1554

A.7.3.7.2	Test Requirements	1557

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1557

A.7.3.8.1	Test Purpose and Environment	1557

A.7.3.8.2	Test Requirements	1560

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1560

A.7.3.9.2	Test Requirements	1563

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1563

A.7.3.10.1	Test Purpose and Environment	1563

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1567

A.7.3.11.1	Test Purpose and Environment	1567

A.7.3.11.2	Test Requirements	1572

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1572

A.7.3.12.1	Test Purpose and Environment	1572

A.7.3.12.2	Test Requirements	1577

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1577

A.7.3.13.1	Test Purpose and Environment	1577

A.7.3.13.2	Test Requirements	1580

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1580

A.7.3.14.1	Test Purpose and Environment	1580

A.7.3.14.2	Test Requirements	1583

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1583

A.7.3.15.1	Test Purpose and Environment	1583

A.7.3.15.2	Test Requirements	1587

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.16.1	Test Purpose and Environment	1587

A.7.3.16.2	Test Requirements	1592

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1592

A.7.3.17.1	Test Purpose and Environment	1592

A.7.3.17.2	Test Requirements	1596

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.18.1	Test Purpose and Environment	1596

A.7.3.18.2	Test Requirements	1600

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1600

A.7.3.19.1	Test Purpose and Environment	1600

A.7.3.19.2	Test Requirements	1605

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1605

A.7.3.20.1	Test Purpose and Environment	1605

A.7.3.20.2	Test Requirements	1610

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1610

A.7.3.21.1	Test Purpose and Environment	1610

A.7.3.21.2	Test Requirements	1615

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1615

A.7.3.22.1	Test Purpose and Environment	1615

A.7.3.22.2	Test Requirements	1620

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1620

A.7.3.23.1	Test Purpose and Environment	1620

A.7.3.23.2	Test Requirements	1621

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1621

A.7.3.24.1	Test Purpose and Environment	1621

A.7.3.24.2	Test Requirements	1622

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1622

A.7.3.25.1	Test Purpose and Environment	1622

A.7.3.25.2	Test Requirements	1623

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1623

A.7.3.26.1	Test Purpose and Environment	1623

A.7.3.26.2	Test Requirements	1626

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1626

A.7.3.27.1	Test Purpose and Environment	1626

A.7.3.27.2	Test Requirements	1629

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1629

A.7.3.28.1	Test Purpose and Environment	1629

A.7.3.28.2	Test Requirements	1632

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1632

A.7.3.29.1	Test Purpose and Environment	1632

A.7.3.29.2	Test Requirements	1635

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1635

A.7.3.30.1	Test Purpose and Environment	1635

A.7.3.30.2	Test Requirements	1638

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1638

A.7.3.31.1	Test Purpose and Environment	1638

A.7.3.31.2	Test Requirements	1641

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1641

A.7.3.32.1	Test Purpose and Environment	1641

A.7.3.32.2	Test Requirements	1644

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1644

A.7.3.33.1	Test Purpose and Environment	1644

A.7.3.33.2	Test Requirements	1647

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1647

A.7.3.34.1	Test Purpose and Environment	1647

A.7.3.34.2	Test Requirements	1650

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1650

A.7.3.35.1	Test Purpose and Environment	1650

A.7.3.35.2	Test Requirements	1653

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1653

A.7.3.36.1	Test Purpose and Environment	1653

A.7.3.36.2	Test Requirements	1656

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1656

A.7.3.37.1	Test Purpose and Environment	1656

A.7.3.37.2	Test Requirements	1659

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1659

A.7.3.38.1	Test Purpose and Environment	1659

A.7.3.38.2	Test Requirements	1662

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1662

A.7.3.39.1	Test Purpose and Environment	1662

A.7.3.39.2	Test Requirements	1665

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1666

A.7.3.40.1	Test Purpose and Environment	1666

A.7.3.40.2	Test Requirements	1668

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1669

A.7.3.41.1	Test Purpose and Environment	1669

A.7.3.41.2	Test Requirements	1672

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1672

A.7.3.42.1	Test Purpose and Environment	1672

A.7.3.42.2	Test Requirements	1675

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1675

A.7.3.43.1	Test Purpose and Environment	1675

A.7.3.43.2	Test Requirements	1678

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1678

A.7.3.44.1	Test Purpose and Environment	1678

A.7.3.44.2	Test Requirements	1681

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1682

A.7.3.45.1	Test Purpose and Environment	1682

A.7.3.45.2	Test Requirements	1684

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1685

A.7.3.46.1	Test Purpose and Environment	1685

A.7.3.46.2	Test Requirements	1688

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1688

A.7.3.47.1	Test Purpose and Environment	1688

A.7.3.47.2	Test Requirements	1691

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1691

A.7.3.48.1	Test Purpose and Environment	1691

A.7.3.48.2	Test Requirements	1694

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1694

A.7.3.49.1	Test Purpose and Environment	1694

A.7.3.49.2	Test Requirements	1697

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1697

A.7.3.50.1	Test Purpose and Environment	1697

A.7.3.50.2	Test Requirements	1700

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1700

A.7.3.51.1	Test Purpose and Environment	1700

A.7.3.51.2	Test Requirements	1703

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1703

A.7.3.52.1	Test Purpose and Environment	1703

A.7.3.52.2	Test Requirements	1706

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1706

A.7.3.53.1	Test Purpose and Environment	1706

A.7.3.53.2	Test Requirements	1709

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1709

A.7.3.54.1	Test Purpose and Environment	1709

A.7.3.54.2	Test Requirements	1712

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1712

A.7.3.55.1	Test Purpose and Environment	1712

A.7.3.55.2	Test Requirements	1715

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1715

A.7.3.56.1	Test Purpose and Environment	1715

A.7.3.56.2	Test Requirements	1718

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1718

A.7.3.57.1	Test Purpose and Environment	1718

A.7.3.57.2	Test Requirements	1721

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1721

A.7.3.58.1	Test Purpose and Environment	1721

A.7.3.58.2	Test Requirements	1724

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1724

A.7.3.59.1	Test Purpose and Environment	1724

A.7.3.59.2	Test Requirements	1727

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1727

A.7.3.60.1	Test Purpose and Environment	1727

A.7.3.60.2	Test Requirements	1731

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1731

A.7.3.61.1	Test Purpose and Environment	1731

A.7.3.61.2	Test Requirements	1735

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1735

A.7.3.62.1	Test Purpose and Environment	1735

A.7.3.62.2	Test Requirements	1739

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1739

A.7.3.63.1	Test Purpose and Environment	1739

A.7.3.63.2	Test Requirements	1743

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.64.1	Test Purpose and Environment	1743

A.7.3.64.2	Test Requirements	1746

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1747

A.7.3.65.1	Test Purpose and Environment	1747

A.7.3.65.2	Test Requirements	1750

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1751

A.7.3.66.1	Test Purpose and Environment	1751

A.7.3.66.2	Test Requirements	1753

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1754

A.7.3.67.1	Test Purpose and Environment	1754

A.7.3.67.2	Test Requirements	1757

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1758

A.7.3.68.1	Test Purpose and Environment	1758

A.7.3.68.2	Test Requirements	1760

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1760

A.7.3.69.1	Test Purpose and Environment	1760

A.7.3.69.2	Test Requirements	1762

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.70.1	Test Purpose and Environment	1762

A.7.3.70.2	Test Requirements	1764

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1764

A.7.3.71.1	Test Purpose and Environment	1764

A.7.3.71.2	Test Requirements	1767

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1767

A.7.3.72.1	Test Purpose and Environment	1767

A.7.3.72.2	Test Requirements	1769

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1769

A.7.3.73.1	Test Purpose and Environment	1769

A.7.3.73.2	Test Requirements	1771

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1771

A.7.3.74.1	Test Purpose and Environment	1771

A.7.3.74.2	Test Requirements	1774

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1774

A.7.3.75.1	Test Purpose and Environment	1774

A.7.3.75.2	Test Requirements	1776

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1777

A.7.3.76.1	Test Purpose and Environment	1777

A.7.3.76.2	Test Requirements	1780

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1780

A.7.3.77.1	Test Purpose and Environment	1780

A.7.3.77.2	Test Requirements	1783

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1783

A.7.3.78.1	Test Purpose and Environment	1783

A.7.3.78.2	Test Requirements	1786

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1786

A.7.3.79.1	Test Purpose and Environment	1786

A.7.3.79.2	Test Requirements	1789

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1789

A.7.3.80.1	Test Purpose and Environment	1789

A.7.3.80.2	Test Requirements	1792

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1792

A.7.3.81.1	Test Purpose and Environment	1792

A.7.3.81.2	Test Requirements	1795

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.82.1	Test Purpose and Environment	1795

A.7.3.82.2	Test Requirements	1798

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1798

A.7.3.83.1	Test Purpose and Environment	1798

A.7.3.83.2	Test Requirements	1799

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1800

A.7.3.84.1	Test Purpose and Environment	1800

A.7.3.84.2	Test Requirements	1802

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.85.1	Test Purpose and Environment	1802

A.7.3.85.2	Test Requirements	1803

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.86.1	Test Purpose and Environment	1804

A.7.3.86.2	Test Requirements	1806

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.87.1	Test Purpose and Environment	1806

A.7.3.87.2	Test Requirements	1808

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1809

A.7.3.88.1	Test Purpose and Environment	1809

A.7.3.88.2	Test Requirements	1813

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1813

A.7.3.89.1	Test Purpose and Environment	1813

A.7.3.89.2	Test Requirements	1817

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1817

A.7.3.90.1	Test Purpose and Environment	1817

A.7.3.90.2	Test Requirements	1821

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1821

A.7.3.91.1	Test Purpose and Environment	1821

A.7.3.91.2	Test Requirements	1825

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1825

A.7.3.92.1	Test Purpose and Environment	1825

A.7.3.92.2	Test Requirements	1828

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1829

A.7.3.93.1	Test Purpose and Environment	1829

A.7.3.93.2	Test Requirements	1832

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1833

A.7.3.94.1	Test Purpose and Environment	1833

A.7.3.94.2	Test Requirements	1835

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1836

A.7.3.95.1	Test Purpose and Environment	1836

A.7.3.95.2	Test Requirements	1839

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1839

A.7.3.96.1	Test Purpose and Environment	1839

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1841

A.7.3.97.1	Test Purpose and Environment	1841

A.7.3.97.2	Test Requirements	1844

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1844

A.7.3.98.1	Test Purpose and Environment	1844

A.7.3.98.2	Test Requirements	1847

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1847

A.7.3.99.1	Test Purpose and Environment	1847

A.7.3.99.2	Test Requirements	1850

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1850

A.7.3.100.1	Test Purpose and Environment	1850

A.7.3.100.2	Test Requirements	1852

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1852

A.7.3.101.1	Test Purpose and Environment	1852

A.7.3.101.2	Test Requirements	1854

A.7.4	Interruption for Dual Connectivity	1854

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1854

A.7.4.1.1	Test Purpose and Environment	1854

A.7.4.1.2	Test Requirements	1857

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1857

A.7.4.2.1	Test Purpose and Environment	1857

A.7.4.2.2	Test Requirements	1859

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1859

A.7.4.3.1	Test Purpose and Environment	1859

A.7.4.3.2	Test Requirements	1861

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.4.1	Test Purpose and Environment	1861

A.7.4.4.2	Test Requirements	1864

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1864

A.7.4.5.1	Test Purpose and Environment	1864

A.7.4.5.2	Test Requirements	1866

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1866

A.7.4.6.1	Test Purpose and Environment	1866

A.7.4.6.2	Test Requirements	1868

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1868

A.7.4.7.1	Test Purpose and Environment	1868

A.7.4.7.2	Test Requirements	1871

A.7.5	Proximity-based Services	1871

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1871

A.7.5.1.1	Test Purpose and Environment	1871

A.7.5.1.2	Test Requirements	1872

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1873

A.7.5.2.1	Test Purpose and Environment	1873

A.7.5.1.2	Test Requirements	1873

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1874

A.7.5.3.1	Test Purpose and Environment	1874

A.7.5.3.2	Test Requirements	1875

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1876

A.7.5.4.1	Test Purpose and Environment	1876

A.7.5.4.2	Test Requirements	1877

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1878

A.7.5.5.1	Test Purpose and Environment	1878

A.7.5.5.2	Test Requirements	1880

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1881

A.7.5.6.1	Test Purpose and Environment	1881

A.7.5.6.2	Test Requirements	1882

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1883

A.7.5.7.1	Test Purpose and Environment	1883

A.7.5.7.2	Test Requirements	1885

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1885

A.7.5.8.1	Test Purpose and Environment	1885

A.7.5.8.2	Test Requirements	1887

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1888

A.7.5.9.1	Test Purpose and Environment	1888

A.7.5.9.2	Test Requirements	1890

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1891

A.7.5.10.1	Test Purpose and Environment	1891

A.7.5.10.2	Test Requirements	1893

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1894

A.7.5.11.1	Test Purpose and Environment	1894

A.7.5.11.2	Test Requirements	1896

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1896

A.7.5.12.1	Test Purpose and Environment	1896

A.7.5.12.2	Test Requirements	1900

A.7.6	Interruption for carrier aggregation	1901

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1901

A.7.6.1.1	Test Purpose and Environment	1901

A.7.6.1.2	Test Requirements	1904

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1904

A.7.6.2.1	Test Purpose and Environment	1904

A.7.6.2.2	Test Requirements	1907

A.8	UE Measurements Procedures	1907

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1907

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1907

A.8.1.1.1	Test Purpose and Environment	1907

A.8.1.1.2	Test Requirements	1909

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1910

A.8.1.2.1	Test Purpose and Environment	1910

A.8.1.2.2	Test Requirements	1911

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1912

A.8.1.3.1	Test Purpose and Environment	1912

A.8.1.3.2	Test Requirements	1914

A.8.1.4	Void	1914

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1914

A.8.1.5.1	Test Purpose and Environment	1914

A.8.1.5.2	Test Requirements	1916

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1916

A.8.1.6.1	Test Purpose and Environment	1916

A.8.1.6.2	Test Requirements	1918

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1918

A.8.1.7.1	Test Purpose and Environment	1918

A.8.1.7.2	Test Requirements	1920

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1921

A.8.1.8.1	Test Purpose and Environment	1921

A.8.1.8.2	Test Requirements	1923

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1924

A.8.1.9.1	Test Purpose and Environment	1924

A.8.1.9.2	Test Requirements	1924

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1924

A.8.1.10.1	Test Purpose and Environment	1924

A.8.1.10.2	Test Requirements	1925

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1925

A.8.1.11.1	Test Purpose and Environment	1925

A.8.1.11.2	Test Requirements	1927

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1928

A.8.1.12.1	Test Purpose and Environment	1928

A.8.1.12.2	Test Requirements	1929

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1930

A.8.1.13.1	Test Purpose and Environment	1930

A.8.1.13.2	Test Requirements	1932

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1932

A.8.1.14.1	Test Purpose and Environment	1932

A.8.1.14.2	Test Requirements	1934

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1935

A.8.1.15.1	Test Purpose and Environment	1935

A.8.1.15.2	Test Requirements	1936

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1937

A.8.1.16.1	Test Purpose and Environment	1937

A.8.1.16.2	Test Requirements	1939

A.8.1.17	Void	1939

A.8.1.18	Void	1939

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1939

A.8.1.19.1	Test Purpose and Environment	1939

A.8.1.19.2	Test Requirements	1941

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1942

A.8.1.20.1	Test Purpose and Environment	1942

A.8.1.20.2	Test Requirements	1944

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.21.1	Test Purpose and Environment	1944

A.8.1.21.2	Test Requirements	1946

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.22.1	Test Purpose and Environment	1947

A.8.1.22.2	Test Requirements	1949

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1949

A.8.1.23.1	Test Purpose and Environment	1949

A.8.1.23.2	Test Requirements	1950

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1951

A.8.1.24.1	Test Purpose and Environment	1951

A.8.1.24.2	Test Requirements	1953

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.25.1	Test Purpose and Environment	1953

A.8.1.25.2	Test Requirements	1955

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.26.1	Test Purpose and Environment	1955

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1958

A.8.1.27.1	Test Purpose and Environment	1958

A.8.1.27.2	Test Requirements	1960

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1960

A.8.1.28.1	Test Purpose and Environment	1960

A.8.1.28.2	Test Requirements	1962

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.29.1	Test Purpose and Environment	1962

A.8.1.29.2	Test Requirements	1964

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.30.1	Test Purpose and Environment	1965

A.8.1.30.2	Test Requirements	1967

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1967

A.8.1.31.1	Test Purpose and Environment	1967

A.8.1.31.2	Test Requirements	1969

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1970

A.8.1.32.1	Test Purpose and Environment	1970

A.8.1.32.2	Test Requirements	1971

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.33.1	Test Purpose and Environment	1972

A.8.1.33.2	Test Requirements	1973

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.34.1	Test Purpose and Environment	1974

A.8.1.34.2	Test Requirements	1975

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.35.1	Test Purpose and Environment	1976

A.8.1.35.2	Test Requirements	1977

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1978

A.8.1.36.1	Test Purpose and Environment	1978

A.8.1.36.2	Test Requirements	1979

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1980

A.8.1.37.1	Test Purpose and Environment	1980

A.8.1.37.2	Test Requirements	1982

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.38.1	Test Purpose and Environment	1982

A.8.1.38.2	Test Requirements	1983

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.39.1	Test Purpose and Environment	1984

A.8.1.39.2	Test Requirements	1986

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1986

A.8.1.40.1	Test Purpose and Environment	1986

A.8.1.40.2	Test Requirements	1988

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1988

A.8.1.41.1	Test Purpose and Environment	1988

A.8.1.41.2	Test Requirement	1990

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1990

A.8.1.42.1	Test Purpose and Environment	1990

A.8.1.42.2	Test Requirement	1991

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1991

A.8.1.43.1	Test Purpose and Environment	1991

A.8.1.43.2	Test Requirements	1994

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1994

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1994

A.8.2.1.1	Test Purpose and Environment	1994

A.8.2.1.2	Test Requirements	1996

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1997

A.8.2.2.1	Test Purpose and Environment	1997

A.8.2.2.2	Test Requirements	1999

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1999

A.8.2.3.1	Test Purpose and Environment	1999

A.8.2.3.2	Test Requirements	2001

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2002

A.8.2.4.1	Test Purpose and Environment	2002

A.8.2.4.2	Test Requirements	2004

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2004

A.8.2.5.1	Test Purpose and Environment	2004

A.8.2.5.2	Test Requirements	2006

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2007

A.8.2.6.1	Test Purpose and Environment	2007

A.8.2.6.2	Test Requirements	2010

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2011

A.8.2.7.1	Test Purpose and Environment	2011

A.8.2.7.2	Test Requirements	2012

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2013

A.8.2.8.1	Test Purpose and Environment	2013

A.8.2.8.2	Test Requirements	2015

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2015

A.8.2.9.1	Test Purpose and Environment	2015

A.8.2.9.2	Test Requirements	2017

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2018

A.8.2.10.1	Test Purpose and Environment	2018

A.8.2.10.2	Test Requirements	2020

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2020

A.8.2.11.1	Test Purpose and Environment	2020

A.8.2.11.2	Test Requirements	2022

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2022

A.8.2.12.1	Test Purpose and Environment	2022

A.8.2.12.2	Test Requirements	2024

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2025

A.8.2.13.1	Test Purpose and Environment	2025

A.8.2.13.2	Test Requirements	2027

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2027

A.8.2.14.1	Test Purpose and Environment	2027

A.8.2.14.2	Test Requirement	2029

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2029

A.8.2.15.1	Test Purpose and Environment	2029

A.8.2.15.2	Test Requirements	2031

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2031

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2031

A.8.3.1.1	Test Purpose and Environment	2031

A.8.3.1.2	Test Requirements	2033

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2033

A.8.3.2.1	Test Purpose and Environment	2033

A.8.3.2.2	Test Requirements	2036

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2036

A.8.3.3.1	Test Purpose and Environment	2036

A.8.3.3.2	Test Requirements	2039

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2039

A.8.3.4.1	Test Purpose and Environment	2039

A.8.3.4.2	Test Requirements	2041

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2041

A.8.3.5.2	Test Requirements	2043

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2043

A.8.3.6.1	Test Purpose and Environment	2043

A.8.3.6.2	Test Requirements	2045

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2046

A.8.3.7.1	Test Purpose and Environment	2046

A.8.3.7.2	Test Requirements	2048

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2049

A.8.3.8.1	Test Purpose and Environment	2049

A.8.3.8.2	Test Requirements	2052

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2052

A.8.3.9.1	Test Purpose and Environment	2052

A.8.3.9.2	Test Requirements	2056

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2057

A.8.3.10.1	Test Purpose and Environment	2057

A.8.3.10.2	Test Requirements	2058

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2058

A.8.3.11.1	Test Purpose and Environment	2058

A.8.3.11.2	Test Requirement	2060

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2061

A.8.3.12.1	Test Purpose and Environment	2061

A.8.3.12.2	Test Requirement	2062

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2063

A.8.3.13.1	Test Purpose and Environment	2063

A.8.3.13.2	Test Requirement	2064

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2065

A.8.3.14.1	Test Purpose and Environment	2065

A.8.3.14.2	Test Requirement	2066

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2067

A.8.3.15.1	Test Purpose and Environment	2067

A.8.3.15.2	Test Requirement	2068

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2069

A.8.3.16.1	Test Purpose and Environment	2069

A.8.3.16.2	Test Requirement	2071

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2071

A.8.3.17.1	Test Purpose and Environment	2071

A.8.3.17.2	Test Requirement	2074

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2074

A.8.3.18.1	Test Purpose and Environment	2074

A.8.3.18.2	Test Requirement	2077

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2077

A.8.3.19.1	Test Purpose and Environment	2077

A.8.3.19.2	Test Requirement	2080

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2080

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2080

A.8.4.1.1	Test Purpose and Environment	2080

A.8.4.1.2	Test Requirements	2082

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2082

A.8.4.2.1	Test Purpose and Environment	2082

A.8.4.2.2	Test Requirements	2085

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2085

A.8.4.3.1	Test Purpose and Environment	2085

A.8.4.3.2	Test Requirements	2087

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2088

A.8.4.4.1	Test Purpose and Environment	2088

A.8.4.4.2	Test Requirements	2090

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2091

A.8.4.5.1	Test Purpose and Environment	2091

A.8.4.5.2	Test Requirements	2093

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2093

A.8.4.6.1	Test Purpose and Environment	2093

A.8.4.6.2	Test Requirements	2094

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2094

A.8.4.7.1	Test Purpose and Environment	2094

A.8.4.7.2	Test Requirements	2098

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2098

A.8.4.8.1	Test Purpose and Environment	2098

A.8.4.8.2	Test Requirements	2101

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2101

A.8.4.9.1	Test Purpose and Environment	2101

A.8.4.9.2	Test Requirements	2106

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2106

A.8.4.10.1	Test Purpose and Environment	2106

A.8.4.10.2	Test Requirements	2108

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2108

A.8.4.11.1	Test Purpose and Environment	2108

A.8.4.11.2	Test Requirement	2110

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2110

A.8.4.12.1	Test Purpose and Environment	2110

A.8.4.12.2	Test Requirement	2112

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2113

A.8.4.13.1	Test Purpose and Environment	2113

A.8.4.13.2	Test Requirement	2114

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2115

A.8.4.14.1	Test Purpose and Environment	2115

A.8.4.14.2	Test Requirement	2117

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2117

A.8.4.15.1	Test Purpose and Environment	2117

A.8.4.15.2	Test Requirement	2120

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2120

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2120

A.8.5.1.1	Test Purpose and Environment	2120

A.8.5.1.2	Test Requirements	2122

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2122

A.8.5.2.1	Test Purpose and Environment	2122

A.8.5.2.2	Test Requirements	2124

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2124

A.8.5.3.1	Test Purpose and Environment	2124

A.8.5.3.2	Test Requirements	2127

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2127

A.8.5.4.1	Test Purpose and Environment	2127

A.8.5.4.2	Test Requirements	2129

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2129

A.8.5.5.1	Test Purpose and Environment	2129

A.8.5.5.2	Test Requirements	2132

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2132

A.8.5.6.1	Test Purpose and Environment	2132

A.8.5.6.2	Test Requirements	2133

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2134

A.8.5.7.1	Test Purpose and Environment	2134

A.8.5.7.2	Test Requirements	2134

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2134

A.8.5.8.1	Test Purpose and Environment	2134

A.8.5.8.2	Test Requirements	2137

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2137

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2137

A.8.6.1.1	Test Purpose and Environment	2137

A.8.6.1.2	Test Requirements	2139

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2139

A.8.6.2.1	Test Purpose and Environment	2139

A.8.6.2.2	Test Requirements	2142

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2142

A.8.6.3.1	Test Purpose and Environment	2142

A.8.6.3.2	Test Requirements	2145

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2145

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2145

A.8.7.1.1	Test Purpose and Environment	2145

A.8.7.1.1.1	Void	2145

A.8.7.1.1.2	1.28 Mcps TDD option	2145

A.8.7.1.1.3	Void	2147

A.8.7.1.2	Test Requirements	2147

A.8.7.1.2.1	Void	2147

A.8.7.1.2.2	1.28 Mcps TDD option	2147

A.8.7.1.2.3	Void	2147

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2147

A.8.7.2.1	Test Purpose and Environment	2147

A.8.7.2.2	Test Requirements	2150

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2151

A.8.7.3.1	Test Purpose and Environment	2151

A.8.7.3.2	Test Parameters	2151

A.8.7.3.3	Test Requirements	2152

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2153

A.8.7.4.1	Test Purpose and Environment	2153

A.8.7.4.2	Test Requirements	2155

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2155

A.8.7.5.1	Test Purpose and Environment	2155

A.8.7.5.2	Test Requirements	2157

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2157

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2157

A.8.7A.1.1	Test Purpose and Environment	2157

A.8.7A.1.2	Test Requirements	2160

A.8.8	E-UTRAN FDD – GSM Measurements	2160

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2160

A.8.8.1.1	Test Purpose and Environment	2160

A.8.8.1.2	Test Requirements	2162

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2162

A.8.8.2.1	Test Purpose and Environment	2162

A.8.8.2.2	Test Requirements	2164

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2165

A.8.8.3.1	Test Purpose and Environment	2165

A.8.8.3.2	Test Requirements	2166

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2167

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2167

A.8.9.1.1	Test Purpose and Environment	2167

A.8.9.1.2	Test Requirements	2168

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2169

A.8.9.2.1	Test Purpose and Environment	2169

A.8.9.2.2	Test Requirements	2171

A.8.10	E-UTRAN TDD – GSM Measurements	2171

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2171

A.8.10.1.1	Test Purpose and Environment	2171

A.8.10.1.2	Test Requirements	2172

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2173

A.8.10.2.1	Test Purpose and Environment	2173

A.8.10.2.2	Test Requirements	2175

A.8.11	Monitoring of Multiple Layers	2175

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2175

A.8.11.1.1	Test Purpose and Environment	2175

A.8.11.1.2	Test Requirements	2177

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2178

A.8.11.2.1	Test Purpose and Environment	2178

A.8.11.2.2	Test Requirements	2179

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2180

A.8.11.3.1	Test Purpose and Environment	2180

A.8.11.3.2	Test Requirements	2182

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2182

A.8.11.4.1	Test Purpose and Environment	2182

A.8.11.4.2	Test Requirements	2185

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2185

A.8.11.5.1	Test Purpose and Environment	2185

A.8.11.5.2	Test Requirements	2187

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2188

A.8.11.6.1	Test Purpose and Environment	2188

A.8.11.6.2	Test Requirements	2190

A.8.12	RSTD Intra-frequency Measurements	2191

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2191

A.8.12.1.1	Test Purpose and Environment	2191

A.8.12.1.2	Test Requirements	2195

A.8.12.1.2A	Test Requirements for UE Category 1bis	2195

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2195

A.8.12.2.1	Test Purpose and Environment	2195

A.8.12.2.2	Test Requirements	2200

A.8.12.2.2A	Test Requirements for UE Category 1bis	2200

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2200

A.8.12.3.1	Test Purpose and Environment	2200

A.8.12.3.2	Test Requirements	2206

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2206

A.8.12.4.1	Test Purpose and Environment	2206

A.8.12.4.2	Test Requirements	2211

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2211

A.8.12.5.1	Test Purpose and Environment	2211

A.8.12.5.2	Test Requirements	2216

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2216

A.8.12.6.1	Test Purpose and Environment	2216

A.8.12.6.2	Test Requirements	2221

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2221

A.8.12.7.1	Test Purpose and Environment	2221

A.8.12.7.2	Test Requirements	2226

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2226

A.8.12.8.1	Test Purpose and Environment	2226

A.8.12.8.2	Test Requirements	2231

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2231

A.8.12.9.1	Test Purpose and Environment	2231

A.8.12.9.2	Test Requirements	2236

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2236

A.8.12.10.1	Test Purpose and Environment	2236

A.8.12.10.2	Test Requirements	2241

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2241

A.8.12.11.1	Test Purpose and Environment	2241

A.8.12.11.2	Test Requirements	2246

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2246

A.8.12.12.1	Test Purpose and Environment	2246

A.8.12.12.2	Test Requirements	2251

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2251

A.8.12.13.1	Test Purpose and Environment	2251

A.8.12.13.2	Test Requirements	2256

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2256

A.8.12.14.1	Test Purpose and Environment	2256

A.8.12.14.2	Test Requirements	2261

A.8.13	RSTD Inter-frequency Measurements	2261

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2261

A.8.13.1.1	Test Purpose and Environment	2261

A.8.13.1.2	Test Requirements	2266

A.8.13.1.2A	Test Requirements for UE Category 1bis	2266

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2266

A.8.13.2.1	Test Purpose and Environment	2266

A.8.13.2.2	Test Requirements	2272

A.8.13.2.2A	Test Requirements for UE Category 1bis	2272

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2272

A.8.13.3.1	Test Purpose and Environment	2272

A.8.13.3.2	Test Requirements	2279

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2279

A.8.13.4.1	Test Purpose and Environment	2279

A.8.13.4.2	Test Requirements	2285

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2285

A.8.13.5.1	Test Purpose and Environment	2285

A.8.13.5.2	Test Requirements	2291

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2291

A.8.13.6.1	Test Purpose and Environment	2291

A.8.13.6.2	Test Requirements	2297

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2297

A.8.13.7.1	Test Purpose and Environment	2297

A.8.13.7.2	Test Requirements	2303

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2303

A.8.13.8.1	Test Purpose and Environment	2303

A.8.13.8.2	Test Requirements	2309

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2309

A.8.13.9.1	Test Purpose and Environment	2309

A.8.13.9.2	Test Requirements	2315

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2315

A.8.13.10.1	Test Purpose and Environment	2315

A.8.13.10.2	Test Requirements	2321

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2321

A.8.13.11.1	Test Purpose and Environment	2321

A.8.13.11.2	Test Requirements	2327

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2327

A.8.13.12.1	Test Purpose and Environment	2327

A.8.13.12.2	Test Requirements	2333

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2333

A.8.13.13.1	Test Purpose and Environment	2333

A.8.13.13.2	Test Requirements	2339

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2339

A.8.13.14.1	Test Purpose and Environment	2339

A.8.13.14.2	Test Requirements	2345

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2345

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2345

A.8.14.1.1	Test Purpose and Environment	2345

A.8.14.1.2	Test Requirements	2347

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2347

A.8.14.2.1	Test Purpose and Environment	2347

A.8.14.2.2	Test Requirements	2350

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2350

A.8.14.3.1	Test Purpose and Environment	2350

A.8.14.3.2	Test Requirements	2352

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2353

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2353

A.8.15.1.1	Test Purpose and Environment	2353

A.8.15.1.2	Test Requirements	2354

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2354

A.8.15.2.1	Test Purpose and Environment	2354

A.8.15.2.2	Test Requirements	2357

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2357

A.8.15.3.1	Test Purpose and Environment	2357

A.8.15.3.2	Test Requirements	2359

A.8.16	E-UTRAN Carrier Aggregation Measurements	2360

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2360

A.8.16.1.1	Test Purpose and Environment	2360

A.8.16.1.2	Test Requirements	2362

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2362

A.8.16.2.1	Test Purpose and Environment	2362

A.8.16.2.2	Test Requirements	2364

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2364

A.8.16.3.1	Test Purpose and Environment	2364

A.8.16.3.2	Test Requirements	2366

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2367

A.8.16.3A.1	Test Purpose and Environment	2367

A.8.16.3A.2	Test Requirements	2368

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2369

A.8.16.4.1	Test Purpose and Environment	2369

A.8.16.4.2	Test Requirements	2370

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2371

A.8.16.4A.1	Test Purpose and Environment	2371

A.8.16.4A.2	Test Requirements	2373

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2374

A.8.16.5.1	Test Purpose and Environment	2374

A.8.16.5.2	Test Requirements	2374

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2374

A.8.16.6.1	Test Purpose and Environment	2374

A.8.16.6.2	Test Requirements	2375

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2375

A.8.16.7.1	 Test Purpose and Environment	2375

A.8.16.7.2	 Test Requirements	2376

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2376

A.8.16.8.1	 Test Purpose and Environment	2376

A.8.16.8.2	 Test Requirements	2377

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2377

A.8.16.9.1	Test Purpose and Environment	2377

A.8.16.9.2	Test Requirements	2378

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2378

A.8.16.10.1	Test Purpose and Environment	2378

A.8.16.10.2	Test Requirements	2379

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2379

A.8.16.11.1	 Test Purpose and Environment	2379

A.8.16.11.2	 Test Requirements	2380

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2380

A.8.16.12.1	 Test Purpose and Environment	2380

A.8.16.12.2	 Test Requirements	2380

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2380

A.8.16.13.1	Test Purpose and Environment	2380

A.8.16.13.2	Test Requirements	2381

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2381

A.8.16.14.1	Test Purpose and Environment	2381

A.8.16.14.2	Test Requirements	2382

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2382

A.8.16.15.1	 Test Purpose and Environment	2382

A.8.16.7.2	 Test Requirements	2382

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2382

A.8.16.16.1	 Test Purpose and Environment	2382

A.8.16.16.2	 Test Requirements	2383

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2383

A.8.16.17.1	Test Purpose and Environment	2383

A.8.16.17.2	Test Requirements	2385

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2386

A.8.16.17A.1	Test Purpose and Environment	2386

A.8.16.17A.2	Test Requirements	2386

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2386

A.8.16.17B.1	Test Purpose and Environment	2386

A.8.16.17B.2	Test Requirements	2387

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2387

A.8.16.17C.1	Test Purpose and Environment	2387

A.8.16.17C.2	Test Requirements	2387

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2387

A.8.16.18.1	Test Purpose and Environment	2387

A.8.16.18.2	Test Requirements	2389

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2390

A.8.16.18A.1	Test Purpose and Environment	2390

A.8.16.18A.2	Test Requirements	2390

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2390

A.8.16.18B.1	Test Purpose and Environment	2390

A.8.16.18B.2	Test Requirements	2391

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2391

A.8.16.18C.1	Test Purpose and Environment	2391

A.8.16.18C.2	Test Requirements	2391

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2391

A.8.16.18D.1	Test Purpose and Environment	2391

A.8.16.18D.2	Test Requirements	2392

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2392

A.8.16.19.1	Test Purpose and Environment	2392

A.8.16.19.2	Test Requirements	2394

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2394

A.8.16.19A.1	Test Purpose and Environment	2394

A.8.16.19A.2	Test Requirements	2395

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2395

A.8.16.19B.1	Test Purpose and Environment	2395

A.8.16.19B.2	Test Requirements	2395

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2395

A.8.16.19C.1	Test Purpose and Environment	2395

A.8.16.19C.2	Test Requirements	2396

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2396

A.8.16.20.1	Test Purpose and Environment	2396

A.8.16.20.2	Test Requirements	2398

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2399

A.8.16.20A.1	Test Purpose and Environment	2399

A.8.16.20A.2	Test Requirements	2399

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2399

A.8.16.20B.1	Test Purpose and Environment	2399

A.8.16.20B.2	Test Requirements	2400

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2400

A.8.16.20C.1	Test Purpose and Environment	2400

A.8.16.20C.2	Test Requirements	2400

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2400

A.8.16.20D.1	Test Purpose and Environment	2400

A.8.16.20D.2	Test Requirements	2401

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2401

A.8.16.21.1	Test Purpose and Environment	2401

A.8.16.21.2	Test Requirements	2403

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2403

A.8.16.22.1	Test Purpose and Environment	2403

A.8.16.22.2	Test Requirements	2405

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2405

A.8.16.23.1	Test Purpose and Environment	2405

A.8.16.23.2	Test Requirements	2408

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2408

A.8.16.24.1	Test Purpose and Environment	2408

A.8.16.24.2	Test Requirements	2411

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2411

A.8.16.25.1	Test Purpose and Environment	2411

A.8.16.25.2	Test Requirements	2414

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2414

A.8.16.26.1	Test Purpose and Environment	2414

A.8.16.26.2	Test Requirements	2417

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2417

A.8.16.27.1	Test Purpose and Environment	2417

A.8.16.27.2	Test Requirements	2422

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2422

A.8.16.28.1	Test Purpose and Environment	2422

A.8.16.28.2	Test Requirements	2427

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2427

A.8.16.29.1	Test Purpose and Environment	2427

A.8.16.29.2	Test Requirements	2432

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2432

A.8.16.30.1	Test Purpose and Environment	2432

A.8.16.30.2	Test Requirements	2437

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2437

A.8.16.31.1	Test Purpose and Environment	2437

A.8.16.31.2	Test Requirements	2442

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2442

A.8.16.32.1	Test Purpose and Environment	2442

A.8.16.32.2	Test Requirements	2447

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2447

A.8.16.33.1	Test Purpose and Environment	2447

A.8.16.33.2	Test Requirements	2452

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2452

A.8.16.34.1	Test Purpose and Environment	2452

A.8.16.34.2	Test Requirements	2457

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2457

A.8.16.35.1	Test Purpose and Environment	2457

A.8.16.35.2	Test Requirements	2459

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2460

A.8.16.36.1	Test Purpose and Environment	2460

A.8.16.36.2	Test Requirements	2462

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2463

A.8.16.37.1	Test Purpose and Environment	2463

A.8.16.37.2	Test Requirements	2465

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2466

A.8.16.38.1	Test Purpose and Environment	2466

A.8.16.38.2	Test Requirements	2468

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2469

A.8.16.39.1	Test Purpose and Environment	2469

A.8.16.39.2	Test Requirements	2472

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2472

A.8.16.40.1	Test Purpose and Environment	2472

A.8.16.40.2	Test Requirements	2475

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2475

A.8.16.41.1	Test Purpose and Environment	2475

A.8.16.41.2	Test Requirements	2477

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2478

A.8.16.42.1	Test Purpose and Environment	2478

A.8.16.42.2	Test Requirements	2480

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2481

A.8.16.43.1	Test Purpose and Environment	2481

A.8.16.43.2	Test Requirements	2484

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2484

A.8.16.44.1	Test Purpose and Environment	2484

A.8.16.44.2	Test Requirements	2487

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2487

A.8.16.45.1	Test Purpose and Environment	2487

A.8.16.45.2	Test Requirements	2490

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2490

A.8.16.46.1	Test Purpose and Environment	2490

A.8.16.46.2	Test Requirements	2492

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2493

A.8.16.47.1	Test Purpose and Environment	2493

A.8.16.47.2	Test Requirements	2495

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2496

A.8.16.48.1	Test Purpose and Environment	2496

A.8.16.48.2	Test Requirements	2498

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2499

A.8.16.49.1	Test Purpose and Environment	2499

A.8.16.49.2	Test Requirements	2502

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2502

A.8.16.50.1	Test Purpose and Environment	2502

A.8.16.50.2	Test Requirements	2504

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2505

A.8.16.51.1	Test Purpose and Environment	2505

A.8.16.51.2	Test Requirements	2510

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2510

A.8.16.52.1	Test Purpose and Environment	2510

A.8.16.52.2	Test Requirements	2515

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2515

A.8.16.53.1	Test Purpose and Environment	2515

A.8.16.53.2	Test Requirements	2519

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2519

A.8.16.54.1	Test Purpose and Environment	2519

A.8.16.54.2	Test Requirements	2523

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2523

A.8.16.55.1	Test Purpose and Environment	2523

A.8.16.55.2	Test Requirements	2529

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2529

A.8.16.56.1	Test Purpose and Environment	2529

A.8.16.56.2	Test Requirements	2535

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2535

A.8.16.57.1	Test Purpose and Environment	2535

A.8.16.57.2	Test Requirements	2537

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2538

A.8.16.58.1	Test Purpose and Environment	2538

A.8.16.58.2	Test Requirements	2540

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2541

A.8.16.59.1	Test Purpose and Environment	2541

A.8.16.59.2	Test Requirements	2545

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2546

A.8.16.60.1	Test Purpose and Environment	2546

A.8.16.60.2	Test Requirements	2549

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2550

A.8.16.61.1	Test Purpose and Environment	2550

A.8.16.61.2	Test Requirements	2553

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2553

A.8.16.62.1	Test Purpose and Environment	2553

A.8.16.62.2	Test Requirements	2555

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2556

A.8.16.63.1	Test Purpose and Environment	2556

A.8.16.63.2	Test Requirements	2559

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2560

A.8.16.64.1	Test Purpose and Environment	2560

A.8.16.64.2	Test Requirements	2564

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2564

A.8.16.65.1	Test Purpose and Environment	2564

A.8.16.65.2	Test Requirements	2569

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2569

A.8.16.66.1	Test Purpose and Environment	2569

A.8.16.66.2	Test Requirements	2574

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2574

A.8.16.67.1	Test Purpose and Environment	2574

A.8.16.67.2	Test Requirements	2579

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2579

A.8.16.68.1	Test Purpose and Environment	2579

A.8.16.68.2	Test Requirements	2584

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2584

A.8.16.69.1	Test Purpose and Environment	2584

A.8.16.69.2	Test Requirements	2588

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.70.1	Test Purpose and Environment	2588

A.8.16.70.2	Test Requirements	2592

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2592

A.8.16.71.1	Test Purpose and Environment	2592

A.8.16.71.2	Test Requirements	2597

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2597

A.8.16.72.1	Test Purpose and Environment	2597

A.8.16.72.2	Test Requirements	2602

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2602

A.8.16.73.1	Test Purpose and Environment	2602

A.8.16.73.2	Test Requirements	2608

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2608

A.8.16.74.1	Test Purpose and Environment	2608

A.8.16.74.2	Test Requirements	2614

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2614

A.8.16.75.1	Test Purpose and Environment	2614

A.8.16.75.2	Test Requirements	2620

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2620

A.8.16.76.1	Test Purpose and Environment	2620

A.8.16.76.2	Test Requirements	2626

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2626

A.8.16.77.1	Test Purpose and Environment	2626

A.8.16.77.2	Test Requirements	2630

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2630

A.8.16.78.1	Test Purpose and Environment	2630

A.8.16.78.2	Test Requirements	2632

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2633

A.8.16.79.1	Test Purpose and Environment	2633

A.8.16.79.2	Test Requirements	2640

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2640

A.8.16.80.1	Test Purpose and Environment	2640

A.8.16.80.2	Test Requirements	2647

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2647

A.8.16.81.1	Test Purpose and Environment	2647

A.8.16.81.2	Test Requirements	2653

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2653

A.8.16.82.1	Test Purpose and Environment	2653

A.8.16.82.2	Test Requirements	2659

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2659

A.8.16.83.1	Test Purpose and Environment	2659

A.8.16.83.2	Test Requirements	2664

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2664

A.8.16.84.1	Test Purpose and Environment	2664

A.8.16.84.2	Test Requirements	2670

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2670

A.8.16.85.1	Test Purpose and Environment	2670

A.8.16.85.2	Test Requirements	2674

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2674

A.8.16.86.1	Test Purpose and Environment	2674

A.8.16.86.2	Test Requirements	2677

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2677

A.8.16.87.1	Test Purpose and Environment	2677

A.8.16.87.2	Test Requirements	2683

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2683

A.8.16.88.1	Test Purpose and Environment	2683

A.8.16.88.2	Test Requirements	2691

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2691

A.8.16.89.1	Test Purpose and Environment	2691

A.8.16.89.2	Test Requirements	2698

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2698

A.8.16.90.1	Test Purpose and Environment	2698

A.8.16.90.2	Test Requirements	2705

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2705

A.8.16.91.1	Test Purpose and Environment	2705

A.8.16.91.2	Test Requirements	2711

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2711

A.8.16.92.1	Test Purpose and Environment	2711

A.8.16.92.2	Test Requirements	2719

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2719

A.8.16.93.1	Test Purpose and Environment	2719

A.8.16.93.2	Test Requirements	2726

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2726

A.8.16.94.1	Test Purpose and Environment	2726

A.8.16.94.2	Test Requirements	2733

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2733

A.8.16.95.1	Test Purpose and Environment	2733

A.8.16.95.2	Test Requirements	2741

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2741

A.8.16.96.1	Test Purpose and Environment	2741

A.8.16.96.2	Test Requirements	2750

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2750

A.8.16.97.1	Test Purpose and Environment	2750

A.8.16.97.2	Test Requirements	2757

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2757

A.8.16.98.1	Test Purpose and Environment	2757

A.8.16.98.2	Test Requirements	2765

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2765

A.8.16.99.1	Test Purpose and Environment	2765

A.8.16.99.2	Test Requirements	2775

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2775

A.8.16.100.1	Test Purpose and Environment	2775

A.8.16.100.2	Test Requirements	2785

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2785

A.8.16.101.1	Test Purpose and Environment	2785

A.8.16.101.2	Test Requirements	2795

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2795

A.8.16.102.1	Test Purpose and Environment	2795

A.8.16.102.2	Test Requirements	2806

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2806

A.8.16.103.1	Test Purpose and Environment	2806

A.8.16.103.2	Test Requirements	2811

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2812

A.8.16.104.1	Test Purpose and Environment	2812

A.8.16.104.2	Test Requirements	2816

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2817

A.8.16.105.1	Test Purpose and Environment	2817

A.8.16.105.2	Test Requirements	2821

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2822

A.8.16.106.1	Test Purpose and Environment	2822

A.8.16.106.2	Test Requirements	2826

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2826

A.8.16.107.1	Test Purpose and Environment	2826

A.8.16.107.2	Test Requirements	2828

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2829

A.8.16.108.1	Test Purpose and Environment	2829

A.8.16.108.2	Test Requirements	2831

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2832

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2832

A.8.17.1.1	Test Purpose and Environment	2832

A.8.17.1.2	Test Requirements	2838

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2838

A.8.17.2.1	Test Purpose and Environment	2838

A.8.17.2.2	Test Requirements	2845

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2845

A.8.17.3.1	Test Purpose and Environment	2845

A.8.17.3.2	Test Requirements	2846

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2846

A.8.17.4.1	Test Purpose and Environment	2846

A.8.17.4.2	Test Requirements	2847

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2847

A.8.17.5.1	Test Purpose and Environment	2847

A.8.17.5.2	Test Requirements	2848

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2848

A.8.17.6.1	Test Purpose and Environment	2848

A.8.17.6.2	Test Requirements	2849

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2850

A.8.17.7.1	Test Purpose and Environment	2850

A.8.17.7.2	Test Requirements	2850

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2851

A.8.17.8.1	Test Purpose and Environment	2851

A.8.17.8.2	Test Requirements	2851

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2852

A.8.17.9.1	Test Purpose and Environment	2852

A.8.17.9.2	Test Requirements	2853

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2853

A.8.17.10.1	Test Purpose and Environment	2853

A.8.17.10.2	Test Requirements	2860

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2861

A.8.17.11.1	Test Purpose and Environment	2861

A.8.17.11.2	Test Requirements	2867

A.8.18	E-UTRAN TDD – HRPD Measurements	2868

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2868

A.8.18.1.1	Test Purpose and Environment	2868

A.8.18.1.2	Test Requirements	2870

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2870

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2870

A.8.19.1.1	Test Purpose and Environment	2870

A.8.19.1.2	Test Requirements	2871

A.8.20	Inter-frequency/RAT Measurements in CA mode	2872

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2872

A.8.20.1.1	Test Purpose and Environment	2872

A.8.20.1.2	Test Requirements	2873

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2873

A.8.20.2.1	Test Purpose and Environment	2874

A.8.20.2.2	Test Requirements	2875

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2876

A.8.20.2A.1	Test Purpose and Environment	2876

A.8.20.2A.2	Test Requirements	2876

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2876

A.8.20.2B.1	Test Purpose and Environment	2876

A.8.20.2B.2	Test Requirements	2879

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2879

A.8.20.3.1	Test Purpose and Environment	2879

A.8.20.3.2	Test Requirements	2881

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2881

A.8.20.4.1	Test Purpose and Environment	2881

A.8.20.4.1.1	1.28 Mcps TDD option	2881

A.8.20.4.2	Test Requirements	2883

A.8.20.4.2.1	1.28 Mcps TDD option	2883

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2883

A.8.20.4A.1	Test Purpose and Environment	2883

A.8.20.4A.1.1	1.28 Mcps TDD option	2883

A.8.20.4A.2	Test Requirements	2883

A.8.20.4A.2.1	1.28 Mcps TDD option	2883

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2884

A.8.20.4B.1	Test Purpose and Environment	2884

A.8.20.4B.1.1	1.28 Mcps TDD option	2884

A.8.20.4B.2	Test Requirements	2886

A.8.20.4B.2.1	1.28 Mcps TDD option	2886

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2886

A.8.21.1	Test Purpose and Environment	2886

A.8.21.2 Test Requirements	2890

A.8.22	E-UTRAN Discovery Signal Measurements	2890

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2890

A.8.22.1.1	Test Purpose and Environment	2890

A.8.22.1.2	Test Requirements	2893

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2893

A.8.22.2.1	Test Purpose and Environment	2893

A.8.22.2.2	Test Requirements	2896

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2896

A.8.22.3.1	Test Purpose and Environment	2896

A.8.22.3.2	Test Requirements	2899

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2899

A.8.22.4.1	Test Purpose and Environment	2899

A.8.22.4.2	Test Requirements	2902

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2902

A.8.22.5.1	Test Purpose and Environment	2902

A.8.22.5.2	Test Requirements	2906

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2906

A.8.22.6.1	Test Purpose and Environment	2906

A.8.22.6.2	Test Requirements	2910

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2910

A.8.22.7.1	Test Purpose and Environment	2910

A.8.22.7.2	Test Requirements	2914

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2914

A.8.22.8.1	Test Purpose and Environment	2914

A.8.22.8.2	Test Requirements	2918

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2918

A.8.22.9.1	Test Purpose and Environment	2918

A.8.22.9.2	Test Requirements	2920

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2921

A.8.22.10.1	Test Purpose and Environment	2921

A.8.22.10.2	Test Requirements	2923

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2924

A.8.22.11.1	Test Purpose and Environment	2924

A.8.22.11.2	Test Requirements	2927

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2927

A.8.22.12.1	Test Purpose and Environment	2927

A.8.22.12.2	Test Requirements	2931

A.8.23	E-UTRAN Dual Connectivity Measurements	2931

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2931

A.8.23.1.1	Test Purpose and Environment	2931

A.8.23.1.2	Test Requirements	2934

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2934

A.8.23.2.1	Test Purpose and Environment	2934

A.8.23.2.2	Test Requirements	2937

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2937

A.8.23.3.1	Test Purpose and Environment	2937

A.8.23.3.2	Test Requirements	2940

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2940

A.8.23.4.1	Test Purpose and Environment	2940

A.8.23.4.2	Test Requirements	2943

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2943

A.8.23.5.1	Test Purpose and Environment	2943

A.8.23.5.2	Test Requirements	2946

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2946

A.8.23.6.1	Test Purpose and Environment	2946

A.8.23.6.2	Test Requirements	2949

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2949

A.8.23.7.1	Test Purpose and Environment	2949

A.8.23.7.2	Test Requirements	2951

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2952

A.8.23.8.1	Test Purpose and Environment	2952

A.8.23.8.2	Test Requirements	2954

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2955

A.8.23.9.1	Test Purpose and Environment	2955

A.8.23.9.2	Test Requirements	2958

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2958

A.8.23.10.1	Test Purpose and Environment	2958

A.8.23.10.2	Test Requirements	2961

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2961

A.8.23.11.1	Test Purpose and Environment	2961

A.8.23.11.2	Test Requirements	2964

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2964

A.8.23.12.1	Test Purpose and Environment	2964

A.8.23.12.2	Test Requirements	2967

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2967

A.8.23.13.1	Test Purpose and Environment	2967

A.8.23.13.2	Test Requirements	2970

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2970

A.8.23.14.1	Test Purpose and Environment	2970

A.8.23.14.2	Test Requirements	2973

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2973

A.8.23.15.1	Test Purpose and Environment	2973

A.8.23.15.2	Test Requirements	2976

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2976

A.8.23.16.1	Test Purpose and Environment	2976

A.8.23.16.2	Test Requirements	2977

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2978

A.8.23.17.1	Test Purpose and Environment	2978

A.8.23.17.2	Test Requirements	2980

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2980

A.8.23.18.1	Test Purpose and Environment	2980

A.8.23.18.2	Test Requirements	2981

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2982

A.8.23.19.1	Test Purpose and Environment	2982

A.8.23.19.2	Test Requirements	2983

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2984

A.8.23.20.1	Test Purpose and Environment	2984

A.8.23.20.2	Test Requirements	2985

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2986

A.8.23.21.1	Test Purpose and Environment	2986

A.8.23.21.2	Test Requirements	2987

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2988

A.8.23.22.1	Test Purpose and Environment	2988

A.8.23.22.2	Test Requirements	2989

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2990

A.8.23.23.1	Test Purpose and Environment	2990

A.8.23.23.2	Test Requirements	2991

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2992

A.8.23.24.1	Test Purpose and Environment	2992

A.8.23.24.2	Test Requirements	2994

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2995

A.8.23.25.1	Test Purpose and Environment	2995

A.8.23.25.2	Test Requirements	2997

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2998

A.8.23.26.1	Test Purpose and Environment	2998

A.8.23.26.2	Test Requirements	3000

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3001

A.8.23.27.1	Test Purpose and Environment	3001

A.8.23.27.2	Test Requirements	3004

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3004

A.8.23.28.1	Test Purpose and Environment	3004

A.8.23.28.2	Test Requirements	3008

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3008

A.8.23.29.1	Test Purpose and Environment	3008

A.8.23.29.2	Test Requirements	3012

A.8.24	Proximity-based Services	3012

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3012

A.8.24.1.1	Test Purpose and Environment	3012

A.8.24.1.2	Test Requirements	3013

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3014

A.8.24.2.1	Test Purpose and Environment	3014

A.8.24.2.2	Test Requirements	3015

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3015

A.8.24.3.1	Test Purpose and Environment	3016

A.8.24.3.2	Test Requirements	3017

A.8.25	E-UTRAN-WLAN Measurements	3018

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3018

A.8.25.1.1	Test Purpose and Environment	3018

A.8.25.1.2	Test Requirements	3020

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3020

A.8.25.2.1	Test Purpose and Environment	3020

A.8.25.2.2	Test Requirements	3022

A.8.26 Frame Structure 3 (FS3)	3022

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3022

A.8.26.1.1	Test Purpose and Environment	3022

A.8.26.1.2	Test Requirements	3024

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3025

A.8.26.2.1	Test Purpose and Environment	3025

A.8.26.2.2	Test Requirements	3027

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3028

A.8.26.3.1	Test Purpose and Environment	3028

A.8.26.3.2	Test Requirements	3031

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3031

A.8.26.3A.1	Test Purpose and Environment	3031

A.8.26.3A.2	Test Requirements	3034

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3034

A.8.26.4.1	Test Purpose and Environment	3034

A.8.26.4.2	Test Requirements	3038

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3038

A.8.26.4A.1	Test Purpose and Environment	3038

A.8.26.4A.2	Test Requirements	3042

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3042

A.8.26.5.1	Test Purpose and Environment	3042

A.8.26.5.2	Test Requirements	3045

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3045

A.8.26.5A.1	Test Purpose and Environment	3045

A.8.26.5A.2	Test Requirements	3049

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3049

A.8.26.6.1	Test Purpose and Environment	3049

A.8.26.6.2	Test Requirements	3053

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3053

A.8.26.6A.1	Test Purpose and Environment	3053

A.8.26.6A.2	Test Requirements	3057

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3057

A.8.26.7.1	Test Purpose and Environment	3057

A.8.26.7.2	Test Requirements	3060

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3060

A.8.26.8.1	Test Purpose and Environment	3060

A.8.26.8.2	Test Requirements	3064

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3064

A.8.26.9.1	Test Purpose and Environment	3064

A.8.26.9.2	Test Requirements	3067

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3067

A.8.26.10.1	Test Purpose and Environment	3067

A.8.26.10.2	Test Requirements	3070

A.9	Measurement Performance Requirements	2951

A.9.1	RSRP	2951

A.9.1.1	FDD Intra frequency case	2951

A.9.1.1.1	Test Purpose and Environment	2951

A.9.1.1.2	Test parameters	2951

A.9.1.1.3	Test Requirements	2954

A.9.1.2	TDD Intra frequency case	2954

A.9.1.2.1	Test Purpose and Environment	2954

A.9.1.2.2	Test parameters	2954

A.9.1.2.3	Test Requirements	2955

A.9.1.3	FDD—FDD Inter frequency case	2956

A.9.1.3.1	Test Purpose and Environment	2956

A.9.1.3.2	Test parameters	2956

A.9.1.3.3	Test Requirements	2959

A.9.1.4	TDD—TDD Inter frequency case	2959

A.9.1.4.1	Test Purpose and Environment	2959

A.9.1.4.2	Test parameters	2959

A.9.1.4.3	Test Requirements	2961

A.9.1.5	FDD—TDD Inter frequency case	2962

A.9.1.5.1	Test Purpose and Environment	2962

A.9.1.5.2	Test parameters	2962

A.9.1.5.3	Test Requirements	2963

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2964

A.9.1.6.1	Test Purpose and Environment	2964

A.9.1.6.2	Test parameters	2964

A.9.1.6.3	Test Requirements	2967

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2967

A.9.1.7.1	Test Purpose and Environment	2967

A.9.1.7.2	Test parameters	2967

A.9.1.7.3	Test Requirements	2970

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2970

A.9.1.8.1	Test Purpose and Environment	2970

A.9.1.8.2	Test parameters	2970

A.9.1.8.3	Test Requirements	2974

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2974

A.9.1.9.1	Test Purpose and Environment	2974

A.9.1.9.2	Test parameters	2974

A.9.1.9.3	Test Requirements	2976

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2977

A.9.1.10.1	Test Purpose and Environment	2977

A.9.1.10.2	Test parameters	2977

A.9.1.10.3	Test Requirements	2980

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2980

A.9.1.11.1	Test Purpose and Environment	2980

A.9.1.11.2	Test parameters	2980

A.9.1.11.3	Test Requirements	2984

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2984

A.9.1.12.1	Test Purpose and Environment	2984

A.9.1.12.2	Test parameters	2984

A.9.1.12.3	Test Requirements	2985

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2985

A.9.1.13.1	Test Purpose and Environment	2985

A.9.1.13.2	Test parameters	2985

A.9.1.13.3	Test Requirements	2986

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2986

A.9.1.14.1	Test Purpose and Environment	2986

A.9.1.14.2	Test parameters	2986

A.9.1.14.3	Test Requirements	2990

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2990

A.9.1.15.1	Test Purpose and Environment	2990

A.9.1.15.2	Test parameters	2990

A.9.1.15.3	Test Requirements	2992

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2993

A.9.1.16.1	Test Purpose and Environment	2993

A.9.1.16.2	Test parameters	2993

A.9.1.16.3	Test Requirements	2994

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2994

A.9.1.17.1	Test Purpose and Environment	2994

A.9.1.17.2	Test parameters	2995

A.9.1.17.3	Test Requirements	2996

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2996

A.9.1.18.1	Test Purpose and Environment	2996

A.9.1.18.2	Test parameters	2996

A.9.1.18.3	Test Requirements	2997

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2998

A.9.1.19.1	Test Purpose and Environment	2998

A.9.1.19.2	Test parameters	2998

A.9.1.19.3	Test Requirements	2998

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2998

A.9.1.20.1	Test Purpose and Environment	2998

A.9.1.20.2	Test parameters	2998

A.9.1.20.3	Test Requirements	3000

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3000

A.9.1.21.1	Test Purpose and Environment	3000

A.9.1.21.2	Test parameters	3000

A.9.1.21.3	Test Requirements	3000

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3000

A.9.1.22.1	Test Purpose and Environment	3000

A.9.1.22.2	Test parameters	3001

A.9.1.22.3	Test Requirements	3005

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3005

A.9.1.23.1	Test Purpose and Environment	3005

A.9.1.23.2	Test parameters	3005

A.9.1.23.3	Test Requirements	3009

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3009

A.9.1.24.1	Test Purpose and Environment	3009

A.9.1.24.2	Test parameters	3009

A.9.1.24.3	Test Requirements	3010

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3010

A.9.1.25.1	Test Purpose and Environment	3010

A.9.1.25.2	Test parameters	3010

A.9.1.25.3	Test Requirements	3013

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3013

A.9.1.26.1	Test Purpose and Environment	3013

A.9.1.26.2	Test parameters	3013

A.9.1.26.3	Test Requirements	3016

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3016

A.9.1.27.1	Test Purpose and Environment	3016

A.9.1.27.2	Test parameters	3016

A.9.1.27.3	Test Requirements	3019

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3019

A.9.1.28.1	Test Purpose and Environment	3019

A.9.1.28.2	Test parameters	3019

A.9.1.28.3	Test Requirements	3022

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3022

A.9.1.29.1	Test Purpose and Environment	3022

A.9.1.29.2	Test parameters	3022

A.9.1.29.3	Test Requirements	3025

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3025

A.9.1.30.1	Test Purpose and Environment	3025

A.9.1.30.2	Test parameters	3026

A.9.1.30.3	Test Requirements	3029

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3029

A.9.1.31.1	Test Purpose and Environment	3029

A.9.1.31.2	Test parameters	3029

A.9.1.31.3	Test Requirements	3032

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3033

A.9.1.32.1	Test Purpose and Environment	3033

A.9.1.32.2	Test parameters	3033

A.9.1.32.3	Test Requirements	3036

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3036

A.9.1.33.1	Test Purpose and Environment	3036

A.9.1.33.2	Test parameters	3036

A.9.1.33.3	Test Requirements	3039

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3039

A.9.1.34.1	Test Purpose and Environment	3039

A.9.1.34.2	Test parameters	3039

A.9.1.34.3	Test Requirements	3042

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3042

A.9.1.35.1	Test Purpose and Environment	3042

A.9.1.35.2	Test parameters	3042

A.9.1.35.3	Test Requirements	3045

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3046

A.9.1.36.1	Test Purpose and Environment	3046

A.9.1.36.2	Test parameters	3046

A.9.1.36.3	Test Requirements	3049

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3049

A.9.1.37.1	Test Purpose and Environment	3049

A.9.1.37.2	Test parameters	3049

A.9.1.37.3	Test Requirements	3052

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3053

A.9.1.38.1	Test Purpose and Environment	3053

A.9.1.38.2	Test parameters	3053

A.9.1.38.3	Test Requirements	3056

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3057

A.9.1.39.1	Test Purpose and Environment	3057

A.9.1.39.2	Test parameters	3057

A.9.1.39.3	Test Requirements	3063

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3063

A.9.1.40.1	Test Purpose and Environment	3063

A.9.1.40.2	Test parameters	3064

A.9.1.40.3	Test Requirements	3070

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3070

A.9.1.41.1	Test Purpose and Environment	3070

A.9.1.41.2	Test parameters	3070

A.9.1.41.3	Test Requirements	3073

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3073

A.9.1.42.1	Test Purpose and Environment	3073

A.9.1.42.2	Test parameters	3073

A.9.1.42.3	Test Requirements	3076

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3076

A.9.1.43.1	Test Purpose and Environment	3076

A.9.1.43.2	Test parameters	3076

A.9.1.43.3	Test Requirements	3077

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3078

A.9.1.44.1	Test Purpose and Environment	3078

A.9.1.44.2	Test parameters	3078

A.9.1.44.3	Test Requirements	3083

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3083

A.9.1.45.1	Test Purpose and Environment	3083

A.9.1.45.2	Test parameters	3083

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2582

A.9.1.73.1	Test Purpose and Environment	2582

A.9.1.73.2	Test parameters	2582

A.9.1.73.3	Test Requirements	2585

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2585

A.9.1.74.1	Test Purpose and Environment	2585

A.9.1.74.2	Test parameters	2585

A.9.1.74.3	Test Requirements	2586

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2586

A.9.1.75.1	Test Purpose and Environment	2586

A.9.1.75.2	Test parameters	2587

A.9.1.75.3	Test Requirements	2588

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2588

A.9.1.76.1	Test Purpose and Environment	2588

A.9.1.76.2	Test parameters	2588

A.9.1.76.3	Test Requirements	2591

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2591

A.9.1.77.1	Test Purpose and Environment	2591

A.9.1.77.2	Test parameters	2591

A.9.1.77.3	Test Requirements	2594

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2594

A.9.1.78.1	Test Purpose and Environment	2594

A.9.1.78.2	Test parameters	2594

A.9.1.78.3	Test Requirements	2597

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2597

A.9.1.79.1	Test Purpose and Environment	2597

A.9.1.79.2	Test parameters	2597

A.9.1.79.3	Test Requirements	2600

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2600

A.9.1.80.1	Test Purpose and Environment	2600

A.9.1.80.2	Test parameters	2600

A.9.1.80.3	Test Requirements	2603

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2603

A.9.1.81.1	Test Purpose and Environment	2603

A.9.1.78.2	Test parameters	2603

A.9.1.81.3	Test Requirements	2606

A.9.2	RSRQ	2606

A.9.2.1	FDD Intra frequency case	2606

A.9.2.1.1	Test Purpose and Environment	2606

A.9.2.1.2	Test parameters	2606

A.9.2.1.3	Test Requirements	2609

A.9.2.2	TDD Intra frequency case	2609

A.9.2.2.1	Test Purpose and Environment	2609

A.9.2.2.2	Test parameters	2609

A.9.2.2.3	Test Requirements	2611

A.9.2.3	FDD—FDD Inter frequency case	2611

A.9.2.3.1	Test Purpose and Environment	2611

A.9.2.3.2	Test parameters	2611

A.9.2.3.3	Test Requirements	2614

A.9.2.4	TDD—TDD Inter frequency case	2614

A.9.2.4.1	Test Purpose and Environment	2614

A.9.2.4.2	Test parameters	2614

A.9.2.4.3	Test Requirements	2616

A.9.2.4A	FDD—TDD Inter frequency case	2617

A.9.2.4A.1	Test Purpose and Environment	2617

A.9.2.4A.2	Test parameters	2617

A.9.2.4A.3	Test Requirements	2619

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2620

A.9.2.5.3	Test Requirements	2623

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2623

A.9.2.6.1	Test Purpose and Environment	2623

A.9.2.6.2	Test parameters	2624

A.9.2.6.3	Test Requirements	2627

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2627

A.9.2.7.1	Test Purpose and Environment	2627

A.9.2.7.2	Test parameters	2627

A.9.2.7.3	Test Requirements	2631

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2631

A.9.2.8.1	Test Purpose and Environment	2631

A.9.2.8.2	Test parameters	2631

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2830

A.9.3.2 E-UTRAN TDD	2830

A.9.3.2.1	Test Purpose and Environment	2830

A.9.3.2.2	Parameters	2830

A.9.3.2.3	Test Requirements	2833

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2833

A.9.3.3.1	Test Purpose and Environment	2833

A.9.3.3.2	Parameters	2833

A.9.3.3.3	Test Requirements	2834

A.9.4	UTRAN FDD CPICH Ec/No	2835

A.9.4.1	E-UTRAN FDD	2835

A.9.4.1.1	Test Purpose and Environment	2835

A.9.4.1.2	Parameters	2835

A.9.4.1.3	Test Requirements	2837

A.9.4.2	E-UTRAN TDD	2838

A.9.4.2.1	Test Purpose and Environment	2838

A.9.4.2.2	Parameters	2838

A.9.4.2.3	Test Requirements	2840

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2841

A.9.4.3.1	Test Purpose and Environment	2841

A.9.4.3.2	Parameters	2841

A.9.4.3.3	Test Requirements	2842

A.9.5	UTRAN TDD measurement	2842

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2842

A.9.5.1.1	Test Purpose and Environment	2842

A.9.5.1.2	Test parameters	2842

A.9.5.1.3	Test Requirements	2844

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2844

A.9.5.2.1	Test Purpose and Environment	2844

A.9.5.2.2	Test parameters	2844

A.9.5.2.3	Test Requirements	2846

A.9.6	GSM Carrier RSSI	2847

A.9.6.1	E-UTRAN FDD	2847

A.9.6.1.1	Test Purpose and Environment	2847

A.9.6.1.2	Test Requirements	2848

A.9.6.2	E-UTRAN TDD	2848

A.9.6.2.1	Test Purpose and Environment	2848

A.9.6.2.2	Test Requirements	2850

A.9.7	UE Rx – Tx Time Difference	2851

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2851

A.9.7.1.1	Test Purpose and Environment	2851

A.9.7.1.2	Test parameters	2851

A.9.7.1.3	Test Requirements	2852

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2852

A.9.7.2.1	Test Purpose and Environment	2852

A.9.7.2.2	Test parameters	2852

A.9.7.2.3	Test Requirements	2854

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2854

A.9.7.3.1	Test Purpose and Environment	2854

A.9.7.3.2	Test parameters	2854

A.9.7.3.3	Test Requirements	2857

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2857

A.9.7.4.1	Test Purpose and Environment	2857

A.9.7.4.2	Test Parameters	2857

A.9.7.4.3	Test Requirements	2860

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2860

A.9.7.5.1	Test Purpose and Environment	2860

A.9.7.5.2	Test parameters	2860

A.9.7.5.3	Test Requirements	2863

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2863

A.9.7.6.1	Test Purpose and Environment	2863

A.9.7.6.2	Test Parameters	2863

A.9.7.6.3	Test Requirements	2866

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2866

A.9.7.7.1	Test Purpose and Environment	2866

A.9.7.7.2	Test parameters	2866

A.9.7.7.3	Test Requirements	2867

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2868

A.9.7.8.1	Test Purpose and Environment	2868

A.9.7.8.2	Test parameters	2868

A.9.7.8.3	Test Requirements	2869

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.9.1	Test Purpose and Environment	2869

A.9.7.9.2	Test parameters	2869

A.9.7.9.3	Test Requirements	2870

A.9.8	RSTD	2871

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2871

A.9.8.1.1	Test Purpose and Environment	2871

A.9.8.1.2	Test Requirements	2873

A.9.8.1.2A	Test Requirements for UE Category 1bis	2873

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2874

A.9.8.2.1	Test Purpose and Environment	2874

A.9.8.2.2	Test Requirements	2877

A.9.8.2.2A	Test Requirements for UE Category 1bis	2877

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2878

A.9.8.3.1	Test Purpose and Environment	2878

A.9.8.3.2	Test Requirements	2880

A.9.8.3.2A	Test Requirements for UE Category 1bis	2880

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2881

A.9.8.4.1	Test Purpose and Environment	2881

A.9.8.4.2	Test Requirements	2883

A.9.8.4.2A	Test Requirements for UE Category 1bis	2883

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2884

A.9.8.5.1	Test Purpose and Environment	2884

A.9.8.5.2	Test Requirements	2886

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2886

A.9.8.6.1	Test Purpose and Environment	2886

A.9.8.6.2	Test Requirements	2889

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2889

A.9.8.7.1	Test Purpose and Environment	2889

A.9.8.7.2	Test Requirements	2890

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.8.1	Test Purpose and Environment	2890

A.9.8.8.2	Test Requirements	2891

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2891

A.9.8.9.1	Test Purpose and Environment	2891

A.9.8.9.2	Test Requirements	2892

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.10.1	Test Purpose and Environment	2892

A.9.8.10.2	Test Requirements	2893

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2893

A.9.8.11.1	Test Purpose and Environment	2893

A.9.8.11.2	Test Requirements	2894

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2894

A.9.8.12.1	Test Purpose and Environment	2894

A.9.8.12.2	Test Requirements	2895

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2895

A.9.8.13.1	Test Purpose and Environment	2895

A.9.8.13.2	Test Requirements	2896

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2896

A.9.8.14.1	Test Purpose and Environment	2896

A.9.8.14.2	Test Requirements	2903

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2903

A.9.8.15.1	Test Purpose and Environment	2903

A.9.8.15.2	Test Requirements	2909

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2909

A.9.8.16.1	Test Purpose and Environment	2909

A.9.8.16.2	Test Requirements	2912

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2912

A.9.8.17.1	Test Purpose and Environment	2912

A.9.8.17.2	Test Requirements	2915

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2915

A.9.8.18.1	Test Purpose and Environment	2915

A.9.8.18.2	Test Requirements	2918

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2918

A.9.8.19.1	Test Purpose and Environment	2918

A.9.8.19.2	Test Requirements	2921

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2921

A.9.8.20.1	Test Purpose and Environment	2921

A.9.8.20.2	Test Requirements	2924

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2924

A.9.8.21.1	Test Purpose and Environment	2924

A.9.8.21.2	Test Requirements	2928

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2929

A.9.8.22.1	Test Purpose and Environment	2929

A.9.8.22.2	Test Requirements	2932

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2933

A.9.8.23.1	Test Purpose and Environment	2933

A.9.8.23.2	Test Requirements	2936

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2936

A.9.8.24.1	Test Purpose and Environment	2936

A.9.8.24.2	Test Requirements	2940

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2940

A.9.8.25.1	Test Purpose and Environment	2940

A.9.8.25.2	Test Requirements	2944

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2945

A.9.8.26.1	Test Purpose and Environment	2945

A.9.8.26.2	Test Requirements	2947

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2947

A.9.8.27.1	Test Purpose and Environment	2947

A.9.8.27.2	Test Requirements	2950

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2950

A.9.8.28.1	Test Purpose and Environment	2950

A.9.8.28.2	Test Requirements	2953

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2953

A.9.8.29.1	Test Purpose and Environment	2953

A.9.8.29.2	Test Requirements	2957

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2958

A.9.8.30.1	Test Purpose and Environment	2958

A.9.8.30.2	Test Requirements	2961

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2962

A.9.8.31.1	Test Purpose and Environment	2962

A.9.8.31.2	Test Requirements	2966

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2966

A.9.8.32.1	Test Purpose and Environment	2966

A.9.8.32.2	Test Requirements	2969

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2969

A.9.8.33.1	Test Purpose and Environment	2969

A.9.8.33.2	Test Requirements	2972

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2972

A.9.8.34.1	Test Purpose and Environment	2972

A.9.8.34.2	Test Requirements	2975

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2975

A.9.8.35.1	Test Purpose and Environment	2975

A.9.8.35.2	Test Requirements	2978

A.9.9	RSRP and RSRQ on the serving cell	2978

A.9.9.1	FDD Intra frequency serving cell case	2978

A.9.9.1.1	Test Purpose and Environment	2978

A.9.9.1.2	Test parameters	2978

A.9.9.1.3	Test Requirements	2981

A.9.9.2	TDD Intra frequency serving cell case	2981

A.9.9.2.1	Test Purpose and Environment	2981

A.9.9.2.2	Test parameters	2981

A.9.9.2.3	Test Requirements	2983

A.9.10	SSTD	2983

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2983

A.9.10.1.1	Test Purpose and Environment	2983

A.9.10.1.2	Test parameters	2983

A.9.10.1.3	Test Requirements	2985

A.9.10.2	Void	2985

A.9.10.3	Void	2985

A.9.10.4	Void	2985

A.9.11 RSSI	2985

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2985

A.9.11.1.1	Test Purpose and Environment	2985

A.9.11.1.2	Test parameters	2985

A.9.11.1.3	Test Requirements	2988

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2988

A.9.11.2.1	Test Purpose and Environment	2988

A.9.11.2.2	Test parameters	2988

A.9.12	Channel occupancy	2991

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2991

A.9.12.1.1	Test Purpose and Environment	2991

A.9.12.1.2	Test parameters	2991

A.9.12.1.3	Test Requirements	2994

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2994

A.9.12.2.1	Test Purpose and Environment	2994

A.9.12.2.2	Test parameters	2994

A.9.12.2.3	Test Requirements	2997

A.9.13	RS-SINR	2997

A.9.13.1	FDD Intra-Frequency Case	2997

A.9.13.1.1	Test Purpose and Environment	2997

A.9.13.1.2	Test parameters	2997

A.9.13.1.3	Test Requirements	3000

A.9.13.2	TDD Intra-Frequency Case	3000

A.9.13.2.1	Test Purpose and Environment	3000

A.9.13.2.2	Test parameters	3000

A.9.13.2.3	Test Requirements	3003

A.9.13.3	FDD—FDD Inter frequency case	3003

A.9.13.3.1	Test Purpose and Environment	3003

A.9.13.3.2	Test parameters	3003

A.9.13.3.3	Test Requirements	3006

A.9.13.4	TDD—TDD Inter frequency case	3006

A.9.13.4.1	Test Purpose and Environment	3006

A.9.13.4.2	Test parameters	3006

A.9.13.4.3	Test Requirements	3010

A.9.13.5	FDD—TDD Inter frequency case	3011

A.9.13.5.1	Test Purpose and Environment	3011

A.9.13.5.2	Test parameters	3011

A.9.13.5.3	Test Requirements	3017

A.9.13.6	TDD—FDD Inter frequency case	3017

A.9.13.6.1	Test Purpose and Environment	3017

A.9.13.6.2	Test parameters	3017

A.9.13.6.3	Test Requirements	3021

A.9.14	Channel quality reporting accuracy	3021

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3021

A.9.14.1.1	Test Purpose and Environment	3021

A.9.14.1.2	Test parameters	3022

A.9.14.1.3	Test Requirements	3022

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3023

A.9.14.2.1	Test Purpose and Environment	3023

A.9.14.2.2	Test parameters	3023

A.9.14.2.3	Test Requirements	3024

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3024

A.9.14.3.1	Test Purpose and Environment	3024

A.9.14.3.2	Test parameters	3024

A.9.14.3.3	Test Requirements	3025

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3025

A.9.14.4.1	Test Purpose and Environment	3025

A.9.14.4.2	Test parameters	3025

A.9.14.4.3	Test Requirements	3026

A.10	Proximity-based Services in Any Cell Selection State	3049

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3049

A.10.1.1	Test Purpose and Environment	3049

A.10.1.2	Test Requirements	3050

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3051

A.10.2.1	Test Purpose and Environment	3051

A.10.2.2	Test Requirements	3052

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3052

A.10.3.1	Test Purpose and Environment	3052

A.10.3.2	Test Requirements	3054

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3055

A.10.4.1	Test Purpose and Environment	3055

A.10.4.2	Test Requirements	3057

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3057

A.11.1	V2V UE Transmission Timing Accuracy Test	3057

A.11.1.1	Test Purpose and Environment	3057

A.11.1.2	Test requirements	3058

A.11.2	Interruptions due to V2V sidelink communication	3058

A.11.2.1	Test Purpose and Environment	3058

A.11.2.2	Test Requirements	3060

A.12	3060

A.12.1	V2X UE Transmission Timing Accuracy Test	3060

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3060

A.12.1.1.1	Test Purpose and Environment	3060

A.12.1.1.2	Test requirements	3061

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3062

A.12.1.2.1	Test Purpose and Environment	3062

A.12.1.2.2	Test Requirements	3062

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3063

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3063

A.12.2.1.1	Test Purpose and Environment	3063

A.12.2.1.2	Test Requirements	3064

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3065

A.12.2.2.1	Test Purpose and Environment	3065

A.12.2.2.2	Test Requirements	3066

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3066

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3066

A.12.3.1.1	Test Purpose and Environment	3066

A.12.3.1.2	Test Requirements	3069

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3070

A.12.3.2.1	Test Purpose and Environment	3070

A.12.3.1.2	Test Requirements	3072

A.12.4	Congestion Control Measurement Test for V2X UE	3072

A.12.4.1	Test Purpose and Environment	3072

A.12.4.2	Test Requirements	3074

A.12.5	Interruptions due to V2X Sidelink Communication	3074

A.12.5.1	Test Purpose and Environment	3074

A.12.5.2	Test Requirements	3076

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3076

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3076

A.12.6.1.1	Test Purpose and Environment	3076

A.12.6.1.2	Test Requirements	3078

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3078

A.12.6.2.1	Test Purpose and Environment	3078

A.12.6.1.2	Test Requirements	3080

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3080

A.12.7.1	Test Purpose and Environment	3080

A.12.7.2	Test Requirements	3083

A.12.8	Interruptions due to V2X Carrier Aggregation	3084

A.12.8.1	Interruptions on a FDD PCell	3084

A.12.8.1.1	Test Purpose and Environment	3084

A.12.8.1.2	Test Requirements	3085

A.12.8.2	Interruptions on a TDD PCell	3085

A.12.8.2.1	Test Purpose and Environment	3085

A.12.8.2.2	Test Requirements	3087

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3088

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3088

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3088

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3088

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3088

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3090

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3091

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3091

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3091

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3093

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3094

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3094

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3094

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3095

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3095

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3096

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3096

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3096

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3097

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3097

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3098

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3098

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3098

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3098

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3099

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3099

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3099

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3101

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3101

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3102

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3103

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3104

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3105

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3106

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3106

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3106

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3106

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3107

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3107

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3109

B.3	Conditions for measurements performance requirements for UE	3109

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.2	Void	3109

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3109

B.3.5	Conditions for UE Rx – Tx time difference	3109

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3110

B.3.7	Conditions for inter-frequency RSTD measurements	3110

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3110

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3111

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3111

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3111

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3112

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3113

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3113

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.21.1	Conditions for RSRP measurements	3114

B.3.21.2	Conditions for RSRQ measurements	3114

B.3.21.3	Conditions for CSI-RSRP measurements	3114

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.22.1	Conditions for RSRP measurements	3114

B.3.22.2	Void	3115

B.3.22.3	Conditions for CSI-RSRP measurements	3115

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.23.1	Conditions for RSRP measurements	3115

B.3.23.2	Conditions for RSRQ measurements	3115

B.3.23.3	Conditions for CSI-RSRP measurements	3115

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.24.1	Conditions for RSRP measurements	3116

B.3.24.2	Conditions for RSRQ measurements	3116

B.3.24.3	Conditions for CSI-RSRP measurements	3116

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3116

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3117

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3117

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3117

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.4	RRM Requirements Exceptions	3118

B.4.1	General	3118

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3118

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3118

B.4.3.1	Inter-band carrier aggregation	3118

B.4.3.2	Intra-band non-contiguous carrier aggregation	3118

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3119

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3119

B.5.1	Conditions for S-RSRP Accuracy Requirements	3119

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3119

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3120

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3120

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3121

B.6	Conditions for V2X	3121

B.6.1	Test parameters for GNSS signals	3121

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3121

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3121

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3122

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3122

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3123

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3123

Annex C (informative): Change history:	3124