| 3GPP TS 36.133 V17.7.0 (2022-09) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.5.8	Measurements of NR cells for RedCap	133

4.2.2.6	Evaluation of cell re-selection criteria	135

4.2.2.7	Maximum interruption in paging reception	135

4.2.2.8	void	136

4.2.2.9	UE measurement capability	136

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	137

4.2.2.10	Reselection to CSG cells	137

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	137

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	138

4.2.2.11	Void	139

4.2.2.12	Void	139

4.2.2.13	Void	139

4.2.2.14	UE measurement capability for RedCap	139

4.3	Minimization of Drive Tests (MDT)	140

4.3.1	Introduction	140

4.3.2	Measurements	140

4.3.2.1	Requirements	140

4.3.3	Relative Time Stamp Accuracy	140

4.3.3.1	Requirements	140

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	141

4.3.4.1	 Requirements	141

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	141

4.3.5.1	Requirements for timeSinceFailure	141

4.4	MBSFN Measurements	141

4.4.1	Introduction	141

4.4.2	MBSFN RSRP measurements	141

4.4.3	MBSFN RSRQ measurements	142

4.4.4	MCH BLER measurements	142

4.5	Proximity-based Services	142

4.5.1	Introduction	142

4.5.2	Requirements	142

4.5.2.1	Interruptions with ProSe Direct Discovery	142

4.5.2.2	Interruptions with ProSe Direct Communication	142

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	143

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	143

4.6	Cell Selection and Re-selection Requirements for UE category NB1	144

4.6.1	Cell Selection	144

4.6.2	Cell Re-selection	144

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	144

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	145

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	146

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	147

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	149

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	150

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	151

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	152

4.6.2.7	Maximum interruption in paging reception in normal coverage	154

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	154

4.6.2.8	UE measurement capability	154

4.6.2.9	WUS receptions for NB1	154

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	155

4.6.3.1	Introduction	155

4.6.3.2	Requirements on UE synchronization for transmission using PUR	155

4.6.3.3	Requirements on TA validation for transmission using PUR	155

4.7	Cell Selection and Re-selection Requirements for UE category M1	156

4.7.1	Cell Selection	156

4.7.2	Cell Re-selection	156

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	156

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	156

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	157

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	158

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	160

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	161

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	162

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	162

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	162

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	163

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	164

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	167

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	168

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	169

4.7.2.3	WUS receptions for UE category M1	169

4.7.3	Channel quality report for UE Category M1 in idle mode	169

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	170

4.7.4.1	Introduction	170

4.7.4.2	Requirements on UE synchronization for transmission using PUR	170

4.7.4.3	Requirements on TA validation for transmission using PUR	170

4.8	Idle State Positioning Measurement Requirements for UE category NB1	171

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	171

4.8.1.1	RSTD Measurement Reporting Delay	173

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	173

4.8.2.1	RSTD Measurement Reporting Delay	175

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	175

4.8.3.1	RSTD Measurement Reporting Delay	176

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	177

4.8.4.1	RSTD Measurement Reporting Delay	178

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	179

4.8.5.1	Measurement Reporting Delay	180

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	180

4.8.6.1	Measurement Reporting Delay	181

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	182

4.8.7.1	Measurement Reporting Delay	183

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	183

4.8.8.1	Measurement Reporting Delay	184

4.9	Idle Mode CA Measurement	185

4.9.1	Introduction	185

4.9.2	Requirement	185

4.9.2.1	Detected cell requirement during state transition and Idle mode	185

4.9.2.2	Measurements of inter-frequency CA candidate cells	186

4.9.2.3	Measurements on serving cell	186

4A	E-UTRAN RRC_INACTIVE state mobility	187

4A.1	Cell Re-selection	187

4A.1.1	Introduction	188

4A.1.2	Requirements	188

4A.1.2.1	UE measurement capability	188

4A.1.2.2	Measurement and evaluation of serving cell	188

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	188

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	188

4A.1.2.5	Evaluation of cell re-selection criteria	188

4A.1.2.6	Maximum interruption in paging reception	188

4A.1.2.7	Measurements of inter-RAT NR cells	188

4A.1.2.8	UE measurement capability for RedCap	188

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	188

4A.2	Requirements for UE Category M1	189

4A.2.1	Introduction	189

4A.2.2	Cell Selection	189

4A.2.3	Cell Reselection	189

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	189

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	189

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	190

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	190

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	190

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	190

4A.2.4	Channel quality report for UE Category M1 in idle mode	192

5	E-UTRAN RRC_CONNECTED state mobility	192

5.1	E-UTRAN Handover	193

5.1.1	Introduction	193

5.1.2	Requirements	193

5.1.2.1	E-UTRAN FDD – FDD	193

5.1.2.1.1	Handover delay	193

5.1.2.1.2	Interruption time	193

5.1.2.2	E-UTRAN FDD – TDD	194

5.1.2.2.1	(Void)	195

5.1.2.2.2	(Void)	195

5.1.2.3	E-UTRAN TDD – FDD	195

5.1.2.3.1	(Void)	195

5.1.2.3.2	(Void)	195

5.1.2.4	E-UTRAN TDD – TDD	195

5.1.2.4.1	Handover delay	195

5.1.2.4.2	Interruption time	195

5.1.2.5	E-UTRAN HD–FDD	197

5.1.2.5.1	Handover delay	197

5.1.2.5.2	Interruption time	197

5.1.2.6	E-UTRAN FDD – FDD conditional handover	198

5.1.2.6.1	Handover delay	198

5.1.2.6.2	Measurement time	199

5.1.2.6.3	Preparation time	199

5.1.2.6.4	Interruption time	199

5.1.2.7	E-UTRAN FDD – TDD conditional handover	199

5.1.2.8	E-UTRAN TDD – FDD conditional handover	200

5.1.2.9	E-UTRAN TDD – TDD conditional handover	200

5.2	Void	200

5.3	Handover to other RATs	200

5.3.1	E-UTRAN - UTRAN FDD Handover	200

5.3.1.1	Introduction	200

5.3.1.1.1	Handover delay	200

5.3.1.1.2	Interruption time	200

5.3.2	E-UTRAN - UTRAN TDD Handover	201

5.3.2.1	Introduction	201

5.3.2.2	Requirements	201

5.3.2.2.1	Handover delay	201

5.3.2.2.2	Interruption time	201

5.3.3	E-UTRAN - GSM Handover	202

5.3.3.1	Introduction	202

5.3.3.2	Requirements	202

5.3.3.2.1	Handover delay	202

5.3.3.2.2	Interruption time	202

5.3.4	E-UTRAN - NR FR1 Handover	202

5.3.4.1	Introduction	202

5.3.4.2	Handover delay	203

5.3.4.3	Interruption time	203

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	203

5.3.4A.1	Introduction	203

5.3.4A.2	Handover delay	204

5.3.4A.3	Interruption time	204

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	205

5.3.4B.1	Introduction	205

5.3.4B.2	Requirements	205

5.3.5	E-UTRAN - NR FR2 Handover	205

5.3.5.1	Introduction	205

5.3.5.2	Handover delay	205

5.3.5.3	Interruption time	205

5.4	Handover to Non-3GPP RATs	206

5.4.1	E-UTRAN – HRPD Handover	206

5.4.1.1	Introduction	206

5.4.1.1.1	Handover delay	206

5.4.1.1.2	Interruption time	206

5.4.2	E-UTRAN – cdma2000 1X Handover	207

5.4.2.1	Introduction	207

5.4.2.1.1	Handover delay	207

5.4.2.1.2	Interruption time	207

5.5	E-UTRAN Handover for Cat-M1 UEs	207

5.5.1	Introduction	207

5.5.2	Requirements in CEModeA	208

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	208

5.5.2.1.1	Handover delay	208

5.5.2.1.2	Interruption time	208

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	208

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	209

5.5.2.3.1	Void	209

5.5.2.3.2	Void	209

5.5.3	Requirements in CEModeB	209

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	209

5.5.3.1.1	Handover delay	209

5.5.3.1.2	Interruption time	209

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	210

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	210

5.6	Void	210

5.7	E-UTRAN DAPS Handover	210

5.7.1	Introduction	210

5.7.2	Requirements	210

5.7.2.1	E-UTRAN FDD – FDD	210

5.7.2.1.1	DAPS Handover delay	210

5.7.2.1.2	Interruption time	211

5.7.2.2	E-UTRAN FDD – TDD	211

5.7.2.3	E-UTRAN TDD – FDD	212

5.7.2.4	E-UTRAN TDD – TDD	212

5.8	EN-DC Handover with PSCell	212

5.8.1	Introduction	212

5.8.1.1	Handover with PSCell Interruption time	212

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	212

5.9	EN-DC Handover with PSCell using CCA	213

5.9.1	Introduction	213

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	214

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	214

6	RRC Connection Mobility Control	215

6.1	RRC Re-establishment	215

6.1.1	Introduction	215

6.1.2	Requirements	215

6.1.2.1	UE Re-establishment delay requirement	215

6.2	Random Access	216

6.2.1	Introduction	216

6.2.2	Requirements	216

6.2.2.1	Contention based random access	216

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	216

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	216

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	216

6.2.2.1.4	Void	216

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	216

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	217

6.2.2.2	Non-Contention based random access	217

6.2.2.2.1	Correct behaviour when receiving Random Access Response	217

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	217

6.2.3	Requirements for Cat-M1 UEs	217

6.3	RRC Connection Release with Redirection	217

6.3.1	Introduction	217

6.3.2	Requirements	217

6.3.2.1	RRC connection release with redirection to UTRAN FDD	217

6.3.2.2	RRC connection release with redirection to GERAN	218

6.3.2.3	RRC connection release with redirection to UTRAN TDD	218

6.3.2.4	RRC connection release with redirection to NR	219

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	219

6.3.2.6	RRC connection release with redirection to NR Redcap	221

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	222

6.4.1	Introduction	222

6.4.2	Requirements	222

6.5	RRC Re-establishment for NB-IoT UEs	222

6.5.1	Introduction	222

6.5.2	Requirements	222

6.5.2.1	UE Re-establishment delay requirement in normal coverage	222

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	223

6.6	Random Access for UE category NB1	223

6.6.1	Introduction	223

6.6.2	Requirements	224

6.6.2.1	Correct behaviour when receiving Random Access Response reception	224

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	224

6.6.2.3	Correct behaviour when receiving a NACK on msg3	224

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	224

6.6.2.5	Correct behaviour when contention Resolution timer expires	224

6.6.2.6	MSG3-based channel quality report for UE Category NB1	224

6.6.3	Requirements for NPRACH configuration	225

6.7	RRC Re-establishment for Cat-M1 UEs	225

6.7.1	Introduction	225

6.7.2	Requirements	225

6.7.2.1	UE Re-establishment delay requirement for CEModeA	225

6.7.2.2	UE Re-establishment delay requirement for CEModeB	226

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	226

6.8.1	Introduction	226

6.8.2	Requirements	227

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	227

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	227

6.9.1	Introduction	227

6.9.2	Requirements	227

7	Timing and signalling characteristics	228

7.1	UE transmit timing	228

7.1.1	Introduction	228

7.1.2	Requirements	229

7.2	UE timer accuracy	230

7.2.1	Introduction	230

7.2.2	Requirements	230

7.3	Timing Advance	230

7.3.1	Introduction	230

7.3.2	Requirements	230

7.3.2.1	Timing Advance adjustment delay	230

7.3.2.2	Timing Advance adjustment accuracy	231

7.4	Cell phase synchronization accuracy (TDD)	231

7.4.1	Definition	231

7.4.2	Minimum requirements	231

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	232

7.5.1	Introduction	232

7.5.2	eNodeB Synchronization Requirements	232

7.5.2.1	Synchronized E-UTRAN	232

7.5.2.2	Non-Synchronized E-UTRAN	232

7.6	Radio Link Monitoring	232

7.6.1	Introduction	232

7.6.2	Requirements	234

7.6.2.1	Minimum requirement when no DRX is used	234

7.6.2.2	Minimum requirement when DRX is used	234

7.6.2.3	Minimum requirement at transitions	236

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	236

7.6.2.5	Minimum requirement under IDC Interference	236

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	236

7.7.1	Introduction	236

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	237

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	238

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	239

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	240

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	241

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	242

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	243

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	243

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	243

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	244

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	245

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	246

7.7.14	SCell Activation Delay Requirement for Dormant SCell	246

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	248

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	248

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	250

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	250

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	252

7.8	Interruptions with Carrier Aggregation	254

7.8.1	Introduction	254

7.8.2	Requirements	254

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	254

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	254

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	254

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	255

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	255

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	255

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	255

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	255

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	256

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	257

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	257

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	257

7.8.2.13	Interruptions at SRS carrier based switching	258

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	259

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	259

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	259

7.8.2.17	Interruptions during CQI measurement on dormant SCell	259

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	260

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	260

7.8.2.20	Interruptions at SCell hibernation	260

7.8.2.21	Interruptions at direct SCell activation and hibernation	260

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	261

7.9.1	Introduction	261

7.9.2	Minimum Requirements for Interband Carrier Aggregation	261

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	261

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	262

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	262

7.10.1	Introduction	262

7.10.2	Requirements	262

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	262

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	262

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	263

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	263

7.11	Radio Link Monitoring for UE Category 0	264

7.11.1	Introduction	264

7.11.2	Requirements for FD-FDD and TDD	265

7.11.2.1	Minimum requirement when no DRX is used	265

7.11.2.2	Minimum requirement when DRX is used	266

7.11.2.3	Minimum requirement at transitions	266

7.11.3	Requirements for HD-FDD	267

7.11.3.1	Minimum requirement when no DRX is used	267

7.11.3.2	Minimum requirement when DRX is used	267

7.11.3.3	Minimum requirement at transitions	268

7.12	Interruptions with Dual Connectivity	268

7.12.1	Introduction	268

7.12.2	Requirements	268

7.12.2.1	Interruptions at PSCell addition/release	268

7.12.2.2	Interruptions at transitions between active and non-active during DRX	268

7.12.2.3	Interruptions at transitions from non-DRX to DRX	269

7.12.2.4	Interruptions at SCell addition/release	269

7.12.2.5	Interruptions at SCell activation/deactivation	269

7.12.2.6	Interruptions during measurements on SCC	270

7.12.2.7	Interruptions at SRS carrier based switching	270

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	271

7.13.1	Definition	271

7.13.2	Minimum requirements	271

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	271

7.14.1	Introduction	271

7.14.2	PSCell Addition Delay Requirement	271

7.14.3	PSCell Release Delay Requirement	272

7.15	Maximum Receive Timing Difference in Dual Connectivity	272

7.15.1	Introduction	272

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	272

7.16	Proximity-based Services	272

7.16.1	Introduction	272

7.16.2	Requirements	272

7.16.2.1	ProSe UE transmission timing	272

7.16.2.1.1	Serving cell or PCell as timing reference	273

7.16.2.1.2	SCell or non-serving cell as timing reference	273

7.16.3	Interruptions with ProSe	273

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	273

7.16.3.2	Interruptions at ProSe Direct Communication configuration	274

7.16.3.3	Interruptions during ProSe Direct Discovery	274

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	274

7.16.3.5	Interruptions during ProSe Direct Communication	275

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	275

7.16.4.1	Measurement and evaluation of selected cell	275

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	275

7.16.5	Selection / Reselection of ProSe relay UE	276

7.16.6	ProSe operation under deactivated SCell	276

7.17	Maximum Transmission Timing Difference in Dual Connectivity	276

7.17.1	Introduction	276

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	277

7.18.1	Introduction	277

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	277

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	277

7.19	Radio Link Monitoring for UE Category M1	277

7.19.1	Introduction	277

7.19.2	Requirements for FD-FDD and TDD CE mode A	278

7.19.2.1	Minimum requirement when no DRX is used	279

7.19.2.2	Minimum requirement when DRX is used	280

7.19.2.3	Minimum requirement at transitions	281

7.19.3	Requirements for HD-FDD with CE mode A	281

7.19.3.1	Minimum requirement when no DRX is used	282

7.19.3.2	Minimum requirement when DRX is used	282

7.19.3.3	Minimum requirement at transitions	283

7.19.4	Requirements for FD-FDD and TDD with CE mode B	283

7.19.4.1	Minimum requirement when no DRX is used	285

7.19.4.2	Minimum requirement when DRX is used	286

7.19.4.3	Minimum requirement at transitions	287

7.19.5	Requirements for HD-FDD with CE mode B	287

7.19.5.1	Minimum requirement when no DRX is used	287

7.19.5.2	Minimum requirement when DRX is used	288

7.19.5.3	Minimum requirement at transitions	289

7.20	UE transmit timing for NB-IoT	289

7.20.1	Introduction	289

7.20.2	Requirements	289

7.21	 UE timer accuracy for NB-IoT	290

7.21.1	Introduction	290

7.21.2	Requirements	290

7.22	Timing Advance for NB-IoT	290

7.22.1	Introduction	290

7.22.2	Requirements	290

7.22.2.1	Timing Advance adjustment delay	290

7.22.2.2	Timing Advance adjustment accuracy	290

7.23	Radio Link Monitoring for Category NB1 UE	290

7.23.1	Introduction	290

7.23.2	Requirements for Category NB1 UE	291

7.23.2.1	Minimum requirement when no DRX is used	291

7.23.2.2	Minimum requirement when DRX is used	292

7.23.2.3	Minimum requirement at transitions	292

7.24	UE transmit timing for Category M1	292

7.24.1	Introduction	292

7.24.2	Requirements	293

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	293

7.25.1	Definition	293

7.25.2	Minimum requirements	294

7.26	UE transmit timing for Category M2	294

7.26.1	Introduction	294

7.26.2	Requirements	294

7.27	UE timer accuracy for category M1	294

7.27.1	Introduction	294

7.27.2	Requirements	294

7.28	Timing Advance for Category M1	295

7.28.1	Introduction	295

7.28.2	Requirements	295

7.29	Interruptions requirements with FeMBMS	295

7.29.1	Introduction	295

7.29.2	Requirements	295

7.30	Numerology switching delay requirements with FeMBMS	295

7.30.1	Introduction	295

7.30.2	Requirements	295

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	295

7.31.1	Introduction	295

7.31.2	NR PSCell Addition Delay Requirement	296

7.31.3	NR PSCell Release Delay Requirement	296

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	297

7.31A.1	Introduction	297

7.31A.2	NR PSCell Addition Delay Requirement	297

7.31A.3	NR PSCell Release Delay Requirement	298

7.32	Interruptions with EN-DC	298

7.32.1	Introduction	298

7.32.2	Requirements	299

7.32.2.1	Interruptions at PSCell addition/release	299

7.32.2.2	Interruptions at transitions between active and non-active during DRX	299

7.32.2.3	Interruptions at transitions from non-DRX to DRX	299

7.32.2.4	Interruptions at SCell addition/release	299

7.32.2.5	Interruptions at SCell activation/deactivation	299

7.32.2.6	Interruptions during measurements on SCC	300

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	300

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	300

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	300

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	301

7.32.2.7	Interruptions at active BWP switching	301

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	302

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	302

7.32.2.10	Interruptions at SCell hibernation	302

7.32.2.11	Interruptions at direct SCell activation and hibernation	302

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	302

7.32.2.13	Interruptions at NR SRS carrier based switching	302

7.32.2.14	Interruptions at NR SCell dormancy	303

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	303

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	304

7.32.2.15	Interruption during NR measurement with autonomous gaps	304

7.32.2.16	Interruptions at SRS carrier based switching	304

7.32.2.17	Interruptions at SCG activation/deactivation	305

7.32.2.18	Interruptions due to NR SRS antenna port switching	305

7.32.2.19	Interruptions at fast SCell activation/deactivation	305

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	306

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	306

7.33.1	Introduction	306

7.33.2	Requirements	306

7.34	Void	306

7.35	Interruptions with SFTD measurements	306

7.35.1	Introduction	306

7.35.2	Requirements	307

7.36	Interruptions with NE-DC	307

7.36.1	Introduction	307

7.36.2	Requirements	307

7.36.2.1	Interruptions at transitions between active and non-active during DRX	307

7.36.2.2	Interruptions at transitions from non-DRX to DRX	308

7.36.2.3	Interruptions at SCell addition/release	308

7.36.2.4	Interruptions at SCell activation/deactivation	308

7.36.2.5	Interruptions during measurements on SCC	309

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	309

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	309

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	309

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	309

7.36.2.6	Interruptions at active BWP switching	309

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	310

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	310

7.36.2.9	Interruptions at SCell hibernation	310

7.36.2.10	Interruptions at direct SCell activation and hibernation	310

7.36.2.11	Interruptions at NR SRS carrier based switching	311

7.36.2.12	Interruptions at NR SCell dormancy	311

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	311

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	312

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	312

7.36.2.14	Interruption during NR measurement with autonomous gaps	312

7.36.2.15	Interruptions at SRS carrier based switching	313

7.36.2.16	Interruptions due to NR SRS antenna port switching	313

7.37	Interruptions during NR measurement with autonomous gaps	314

7.37.1	Introduction	314

7.37.2	Requirements	314

7.38 SCG Activation and Deactivation Delay	314

7.38.1	Introduction	314

7.38.2	SCG Activation Delay Requirement	314

7.38.3	SCG Deactivation Delay Requirement	315

8	UE Measurements Procedures in RRC_CONNECTED State	316

8.1	General Measurement Requirements	316

8.1.1	Introduction	316

8.1.2	Requirements	316

8.1.2.1	UE measurement capability	316

8.1.2.1.1	Monitoring of multiple layers using gaps	324

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	325

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	326

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	328

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	329

8.1.2.1.2	Network controlled small gap	329

8.1.2.2	E-UTRAN intra frequency measurements	331

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	331

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	336

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	340

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	341

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	342

8.1.2.3	E-UTRAN inter frequency measurements	342

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	343

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	348

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	355

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	355

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	355

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	356

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	358

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	359

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	360

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	367

8.1.2.4	Inter RAT measurements	367

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	367

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	372

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	372

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	376

8.1.2.4.5	E-UTRAN FDD – GSM measurements	376

8.1.2.4.6	E-UTRAN TDD – GSM measurements	381

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	381

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	383

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	383

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	383

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	384

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	384

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	384

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	384

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	386

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	386

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	386

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	386

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	387

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	387

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	389

8.1.2.4.21	E-UTRAN FDD – NR measurements	389

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	393

8.1.2.4.22	E-UTRAN TDD – NR measurements	397

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	397

8.1.2.4.23	Void	397

8.1.2.4.24	Void	397

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	397

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	399

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	399

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	400

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	400

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	400

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	402

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	404

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	406

8.1.2.6.5	Void	407

8.1.2.6.6	Void	407

8.1.2.6.7	Void	407

8.1.2.6.8	Void	407

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	407

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	408

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	409

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	411

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	413

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	415

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	416

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	418

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	420

8.1.2.7	E-UTRAN E-CID Measurements	422

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	422

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	423

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	424

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	425

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	425

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	426

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	428

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	431

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	435

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	438

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	438

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	439

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	439

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	440

8.1.2.10	Void	440

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	440

8.2.1	Introduction	440

8.2.2	Requirements	440

8.3	Measurements for E-UTRA carrier aggregation	445

8.3.1	Introduction	445

8.3.2	Measurements of the primary component carrier	445

8.3.3	Measurements of a secondary component carrier	445

8.3.3.1	Measurements of a secondary component carrier with active SCell	445

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	446

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	446

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	447

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	449

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	449

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	449

8.4.1	Introduction	449

8.4.2	Measurements on the primary component carrier	450

8.4.3	Measurements on a secondary component carrier	451

8.4.4	Measurements on both primary component carrier and a secondary component carrier	451

8.4.5	Measurements on different secondary component carriers	452

8.5	Measurements for UE category 0	453

8.5.1	Introduction	453

8.5.2 	Requirements	454

8.5.2.1 	E-UTRAN intra frequency measurements	454

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	454

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	457

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	459

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	463

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	463

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	464

8.6	Discovery signal measurements	465

8.6.1	Introduction	465

8.6.2	Requirements for CRS based discovery signal measurements	465

8.6.2.1	E-UTRAN intra frequency measurements	465

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	465

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	468

8.6.2.2	E-UTRAN inter frequency measurements	470

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	471

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	473

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	476

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	476

8.6.3	Requirements for CSI-RS based discovery signal measurements	476

8.6.3.1	E-UTRAN intra frequency measurements	476

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	477

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	479

8.6.3.2	E-UTRAN inter frequency measurements	481

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	482

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	484

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	487

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	487

8.7	Discovery signal measurements for E-UTRA carrier aggregation	487

8.7.1	Introduction	487

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	488

8.7.2.1	Measurements of the primary component carrier	488

8.7.2.2	Measurements of a secondary component carrier	488

8.7.2.3	Measurements of a secondary component carrier with active SCell	488

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	488

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	488

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	489

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	491

8.7.3.1	Measurements of the primary component carrier	491

8.7.3.2	Measurements of a secondary component carrier	491

8.7.3.3	Measurements of a secondary component carrier with active SCell	491

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	491

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	491

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	493

8.8	Measurements for E-UTRA dual connectivity	494

8.8.1	Introduction	494

8.8.2	Intra-frequency measurements requirements on PCell	494

8.8.3	Intra-frequency measurements requirements on PSCell	495

8.8.4	Inter-frequency and inter-RAT measurement requirements	495

8.8.5	Intra-frequency measurements with autonomous gaps	495

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	495

8.8.5.2	ECGI reporting delay	496

8.8.6	Inter-frequency measurements with autonomous gaps	496

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	496

8.8.6.2	ECGI reporting delay	497

8.8.7	SSTD Measurements	497

8.8.7.1	Introduction	497

8.8.7.2	SSTD Measurement requirements	497

8.8.7.3	SSTD Measurement Reporting Delay	498

8.8.8	Intra-frequency measurements requirements on SCell	498

8.9	MBSFN Measurements	498

8.9.1	Introduction	498

8.9.2	MBSFN RSRP Measurements	498

8.9.3	MBSFN RSRQ Measurements	499

8.9.4	MCH BLER Measurements	499

8.10	Proximity-based Services	499

8.10.1	Introduction	499

8.10.2	Requirements	499

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	499

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	500

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	500

8.11.1	Introduction	500

8.11.2	CRS based discovery signal measurements	501

8.11.2.1	E-UTRAN intra-frequency measurements	501

8.11.2.1.1	Requirements	501

8.11.2.1.1.1	Requirements when no DRX is used	501

8.11.2.1.1.1.1	Measurement Reporting Requirements	503

8.11.2.1.1.2	Requirements when DRX is used	504

8.11.2.1.1.2.1	Measurement Reporting Requirements	506

8.11.2.2	E-UTRAN inter-frequency measurements	507

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	507

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	511

8.11.3	CSI-RS based discovery signal measurements	511

8.11.3.1	E-UTRAN intra-frequency measurements	511

8.11.3.1.1	Requirements	511

8.11.3.1.1.1	Requirements when no DRX is used	511

8.11.3.1.1.1.1	Measurement Reporting Requirements	513

8.11.3.1.1.2	Requirements when DRX is used	513

8.11.3.1.1.2.1	Measurement Reporting Requirements	514

8.11.3.2	E-UTRAN inter-frequency measurements	515

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	515

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	519

8.11.4	RSSI measurements	519

8.11.4.1	E-UTRAN intra-frequency measurements	519

8.11.4.2	E-UTRAN inter-frequency measurements	519

8.11.5	Channel occupancy measurements	520

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	520

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	520

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	520

8.12.1	Introduction	520

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	520

8.12.2.1	Introduction	520

8.12.2.2	Measurements of a secondary component carrier	520

8.12.2.3	Measurements of a secondary component carrier with active SCell	520

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	521

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	521

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	523

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	526

8.12.3.1	Introduction	526

8.12.3.2	Measurements of a secondary component carrier	526

8.12.3.3	Measurements of a secondary component carrier with active SCell	526

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	526

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	526

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	528

8.13	Measurements for UE Category M1	530

8.13.1	Introduction	530

8.13.2	Requirements for UE category M1 with CE mode A	530

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	531

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	531

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	536

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	538

8.13.2.2	Void	542

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	542

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	542

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	545

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	547

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	548

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	548

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	550

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	552

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	553

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	553

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	553

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	554

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	554

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	554

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	555

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	555

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	556

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	557

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	557

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	558

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	562

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	564

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	568

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	568

8.13.3	Requirements for UE category M1 with CE mode B	569

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	569

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	570

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	574

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	577

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	582

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	583

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	583

8.13.3.2	Void	584

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	584

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	584

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	587

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	589

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	590

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	590

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	590

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	591

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	591

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	591

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	592

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	592

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	592

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	597

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	599

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	603

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	604

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	604

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	606

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	608

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	609

8.14	Measurements for UE category NB1	609

8.14.1	Introduction	609

8.14.2	NB-IoT intra frequency measurements under normal coverage	610

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	610

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	610

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	610

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	610

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	610

8.14.4	Connected mode channel quality report for UE Category NB1	610

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	611

8.14.6	NB-IoT neighbour cell measurements	611

8.14.6.1	Introduction	611

8.14.6.2	Requirements	611

8.14.6.3	Intra-frequency neighbour cell measurements	612

8.14.6.4	Inter-frequency neighbour cell measurements	612

8.14.6.5	 Requirements for monitoring multiple carriers	613

8.15	Void	613

8.16	Measurements for UE Category M2	613

8.16.1	Introduction	613

8.16.2	Requirements for UE category M2 with CE mode A	614

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	614

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	614

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	615

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	615

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	616

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	616

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	616

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	617

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	619

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	622

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	622

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	622

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	624

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	627

8.16.3	Requirements for UE category M2 with CE mode B	627

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	627

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	628

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	630

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	633

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	633

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	633

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	635

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	638

8.17	Measurements for E-UTRA – NR Dual Connectivity	638

8.17.1	Introduction	638

8.17.1.1	Measurement Gap Sharing	638

8.17.1A	Intrafrequency Measurements	639

8.17.2	SFTD Measurements	639

8.17.2.1	Introduction	639

8.17.2.2	SFTD Measurement requirements	639

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	640

8.17.2.3	SFTD Measurement Reporting Delay	641

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	642

8.17.3.1	Introduction	642

8.17.3.2	E-UTRAN FDD inter frequency measurements	642

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	642

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	644

8.17.3.3	E-UTRAN TDD inter frequency measurements	646

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	646

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	647

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	649

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	649

8.17.4.1.1	NR Inter-RAT cell identification	650

8.17.4.1.2	NR Inter-RAT measurement	651

8.17.4.1.3	NR Inter-RAT measurement reporting	652

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	653

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	653

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	653

8.17.4A.1.1	NR Inter-RAT cell identification	654

8.17.4A.1.2	NR Inter-RAT measurement	655

8.17.4A.1.3	NR Inter-RAT measurement reporting	656

8.17.4A.1.4	NR inter-RAT RSSI measurements	656

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	657

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	657

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	658

8.17.5.1	Introduction	658

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	658

8.17.5.2.1	Identification of a new UTRA FDD cell	658

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	658

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	658

8.17.5.2.4	Periodic Reporting	659

8.17.5.2.5	Event Triggered Reporting	659

8.17.5.2.6	Event-triggered Periodic Reporting	659

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	660

8.17.5.3.1	Periodic Reporting	661

8.17.5.3.2	Event Triggered Reporting	661

8.17.5.3.3	Event-triggered Periodic Reporting	661

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	662

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	662

8.17.7.1	Introduction	662

8.17.7.2	Identification of a new UTRA FDD cell for SON	662

8.17.7.2.1	Requirements when no DRX is used	662

8.17.7.2.2	Requirements when DRX is used	662

8.17.7.2.3	Reporting Delay	663

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	664

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	664

8.17.9.1	Introduction	664

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	664

8.17.9.2.1	Identification of a new UTRA FDD cell	664

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	664

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	665

8.17.9.2.4	Periodic Reporting	665

8.17.9.2.5	Event Triggered Reporting	665

8.17.9.2.6	Event-triggered Periodic Reporting	666

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	666

8.17.9.3.1	Periodic Reporting	667

8.17.9.3.2	Event Triggered Reporting	667

8.17.9.3.3	Event-triggered Periodic Reporting	667

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	668

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	668

8.17.11.1	Introduction	668

8.17.11.2	Identification of a new UTRA TDD cell for SON	668

8.17.11.2.1	Requirements when no DRX is used	668

8.17.11.2.2	Requirements when DRX is used	668

8.17.11.2.3	Reporting Delay	669

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	669

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	670

8.17.13.1	Introduction	670

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	670

8.17.13.2.1	GSM carrier RSSI	670

8.17.13.2.2	BSIC verification	670

8.17.13.2.3	Enhanced BSIC verification	672

8.17.13.2.4	Periodic Reporting	672

8.17.13.2.5	Event Triggered Reporting	672

8.17.13.2.6	Event-triggered Periodic Reporting	673

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	673

8.17.13.3.1	GSM carrier RSSI	673

8.17.13.3.2	BSIC verification	673

8.17.13.3.3	Periodic Reporting	675

8.17.13.3.4	Event Triggered Reporting	675

8.17.13.3.5	Event-triggered Periodic Reporting	675

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	675

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	675

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	675

8.17.15.1.1	RSTD Measurement Reporting Delay	676

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	676

8.17.15.2.1	RSTD Measurement Reporting Delay	676

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	676

8.17.15.3.1	RSTD Measurement Reporting Delay	677

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	677

8.17.15.4.1	RSTD Measurement Reporting Delay	677

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	678

8.17.16.1	Introduction	678

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	678

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	678

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	678

8.17.17.1	Introduction	678

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	678

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	678

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	678

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	678

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	678

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	678

8.18	Measurements for non-BL/CE UE	678

8.18.1	Introduction	678

8.18.2	Requirements for non-BL/CE UE with CE Mode B	679

8.18.2.1	E-UTRAN intra frequency measurements	679

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	679

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	679

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	680

8.19	Measurements for NR – E-UTRA Dual Connectivity	681

8.19.1	Introduction	681

8.19.2	Intra-frequency Measurements	681

8.19.3	Inter-frequency Measurements	681

8.19.4	Void	681

8.19.5	Intra-frequency E-CID Measurements	681

8.19.6	Intra-frequency measurements with autonomous gaps	682

8.19.6.1	Introduction	682

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	682

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	682

8.19.7	Inter-frequency measurements with autonomous gaps	682

8.19.7.1	Introduction	682

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	682

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	682

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	682

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	682

8.20	Inter-RAT NR Measurements for RedCap UE	682

8.20.1	Introduction	682

8.20.2	E-UTRAN FDD – NR measurements	682

8.20.2.1	Identification of a new NR cell	682

8.20.2.2	Periodic Reporting	686

8.20.2.3	Event Triggered Reporting	686

8.20.2.4	Event-triggered Periodic Reporting	686

8.20.3	E-UTRAN TDD – NR measurements	686

9	Measurements performance requirements for UE	687

9.1	E-UTRAN measurements	687

9.1.1	Introduction	687

9.1.2	Intra-frequency RSRP Accuracy Requirements	687

9.1.2.1	Absolute RSRP Accuracy	687

9.1.2.2	Relative Accuracy of RSRP	688

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	689

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	689

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	690

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	691

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	692

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	693

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	694

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	694

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	694

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	695

9.1.2B.1	Introduction	695

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	695

9.1.3	Inter-frequency RSRP Accuracy Requirements	696

9.1.3.1	Absolute RSRP Accuracy	696

9.1.3.2	Relative Accuracy of RSRP	697

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	698

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	698

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	699

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	699

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	700

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	701

9.1.3B.1	Introduction	701

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	701

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	701

9.1.4	RSRP Measurement Report Mapping	702

9.1.5	Intra-frequency RSRQ Accuracy Requirements	702

9.1.5.1	Absolute RSRQ Accuracy	702

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	703

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	704

9.1.5.4	Absolute WB-RSRQ Accuracy	705

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	706

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	707

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	707

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	707

9.1.5B.1	Introduction	707

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	707

9.1.6	Inter-frequency RSRQ Accuracy Requirements	708

9.1.6.1	Absolute RSRQ Accuracy	708

9.1.6.2	Relative Accuracy of RSRQ	709

9.1.6.3	Absolute WB-RSRQ Accuracy	710

9.1.6.4	Relative WB-RSRQ Accuracy	710

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	711

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	712

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	712

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	712

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	713

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	714

9.1.6B.1	Introduction	714

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	714

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	714

9.1.7	RSRQ Measurement Report Mapping	715

9.1.8	Power Headroom	715

9.1.8.1	Period	716

9.1.8.2	Reporting Delay	716

9.1.8.3	Void	716

9.1.8.4	Report Mapping	716

9.1.9	UE Rx – Tx time difference	716

9.1.9.1	Measurement Requirement	716

9.1.9.2	Measurement Report mapping	717

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	718

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	719

9.1.10	Reference Signal Time Difference (RSTD)	720

9.1.10.1	Intra-Frequency Accuracy Requirement	720

9.1.10.2	Inter-Frequency Accuracy Requirement	721

9.1.10.3	RSTD Measurement Report Mapping	723

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	723

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	724

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	725

9.1.11	Carrier aggregation measurement accuracy	726

9.1.11.1	Primary component carrier accuracy requirement	727

9.1.11.2	Secondary component carrier accuracy requirement	727

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	727

9.1.11.4	Secondary component carrier relative accuracy requirement	727

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	727

9.1.13	Measurement accuracy for UE category 0	728

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	728

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	728

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	729

9.1.14	Accuracy requirements for Discovery Signal Measurements	730

9.1.14.1	Introduction	730

9.1.14.2	RSRP measurements in discovery signal occasions	730

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	730

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	730

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	730

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	731

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	732

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	732

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	732

9.1.14.3.3	CSI-RSRP measurement report mapping	733

9.1.14.4	RSRQ measurements in discovery signal occasions	733

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	733

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	734

9.1.15.1.1	Primary component carrier accuracy requirement	734

9.1.15.1.2	Secondary component carrier accuracy requirement	734

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	734

9.1.15.1.4	Secondary component carrier relative accuracy requirement	734

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	734

9.1.15.2.1	Primary component carrier accuracy requirement	734

9.1.15.2.2	Secondary component carrier accuracy requirement	734

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	734

9.1.15.2.4	Secondary component carrier relative accuracy requirement	734

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	735

9.1.17	RS-SINR Measurements	735

9.1.17.1	Measurement Report Mapping	735

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	736

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	736

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	736

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	736

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	737

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	738

9.1.18.1	Introduction	738

9.1.18.2	RSRP measurements	738

9.1.18.2.1	RSRP measurement report mapping	738

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	738

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	739

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	739

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	740

9.1.18.3	RSRQ measurements	740

9.1.18.3.1	RSRQ measurement report mapping	740

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	740

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	741

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	741

9.1.18.4	CSI-RSRP measurements	742

9.1.18.4.1	CSI-RSRP measurement report mapping	742

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	742

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	742

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	743

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	743

9.1.18.5	RSSI measurements	744

9.1.18.5.1	RSSI measurement report mapping	744

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	744

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	745

9.1.18.6	Channel occupancy measurements	745

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	745

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	745

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	746

9.1.19.1	Introduction	746

9.1.19.2	Accuracy requirements for measurements on SCC	746

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	746

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	746

9.1.20	SFN and Subframe Time Difference (SSTD)	747

9.1.20.1	SSTD Accuracy Requirement	747

9.1.20.2	SSTD Measurement Report Mapping	747

9.1.21	Measurement accuracy for UE category M1	748

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	748

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	750

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	751

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	753

9.1.21.5	RSRP Measurement Report Mapping	754

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	754

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	755

9.1.21.8	RSRQ Measurement Report Mapping	756

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	756

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	757

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	758

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	759

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	760

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	761

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	762

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	763

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	764

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	765

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	766

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	767

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	770

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	773

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	773

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	776

9.1.22	Measurement accuracy for UE Category NB1	777

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	777

9.1.22.2	Void	778

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	778

9.1.22.4	Void	779

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	779

9.1.22.6	Void	780

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	780

9.1.22.8	Void	781

9.1.22.9	NRSRP Measurement Report Mapping	781

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	781

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	782

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	783

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	784

9.1.22.14	NRSRQ Measurement Report Mapping	785

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	786

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	786

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	787

9.1.23	Power Headroom for UE Category NB1	787

9.1.23.1	Period	788

9.1.23.2	Reporting Delay	788

9.1.23.3	Report Mapping for UE Category NB1	788

9.1.23.3.1	Void	789

9.1.23.3.2	Void	789

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	789

9.1.24	Void	790

9.1.25	Measurement accuracy for UE category M2	790

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	790

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	791

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	792

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	793

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	795

9.1.26	Measurement Accuracy for non-BL CE UE	796

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	797

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	799

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	799

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	801

9.1.26.5	RSRP Measurement Report Mapping	802

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	802

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	802

9.1.26.8	RSRQ Measurement Report Mapping	803

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	803

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	804

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	805

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	806

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	807

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	807

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	807

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	808

9.1.27	SFN and frame Timing Difference (SFTD)	809

9.1.27.1	SFTD Accuracy Requirement	809

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	811

9.1.28.1	SFTD Accuracy Requirement under CCA	811

9.2	UTRAN FDD Measurements	812

9.2.1	UTRAN FDD CPICH RSCP	812

9.2.2	Void	813

9.2.3	UTRAN FDD CPICH Ec/No	813

9.3	UTRAN TDD Measurements	813

9.3.1	UTRAN TDD P-CCPCH RSCP	814

9.3.2	Void	814

9.3.3	Void	814

9.4	GSM Measurements	814

9.4.1	GSM carrier RSSI	814

9.5	CDMA2000 1x RTT Measurements	814

9.5.1	CDMA2000 1x RTT Pilot Strength	814

9.6	PCMAX,c	815

9.6.1	Report Mapping	815

9.6.2 	Estimation Period	815

9.6.3	Reporting Delay	815

9.7	IEEE802.11 Measurements	815

9.7.1	WLAN RSSI	815

9.7.2	WLAN RSSI Measurement Report Mapping	815

9.8	MBSFN Measurements	816

9.8.1	Introduction	816

9.8.2	MBSFN RSRP	816

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	816

9.8.2.2	MBSFN RSRP measurement report mapping	817

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	817

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	818

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	818

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	818

9.8.3	MBSFN RSRQ	819

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	819

9.8.3.2	MBSFN RSRQ measurement report mapping	819

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	820

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	820

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	820

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	821

9.8.4	MCH BLER	821

9.8.4.1	Measurement report mapping for MCH BLER	821

9.8.4.2	Measurement report mapping for MCH Block Number	822

9.9	ProSe Measurements	823

9.9.1	Introduction	823

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	823

9.9.2.1	Absolute S-RSRP Accuracy	823

9.9.2.2	Relative Accuracy of S-RSRP	824

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	825

9.9.3.1	Absolute SD-RSRP Accuracy	825

9.9.3.2	Relative Accuracy of SD-RSRP	825

9.10	V2X Measurements	826

9.10.1	Introduction	826

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	826

9.10.2.1	Absolute S-RSRP Accuracy	826

9.10.2.2	Relative Accuracy of S-RSRP	827

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	827

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	827

9.10.4	S-RSSI Measurement Accuracy Requirements	828

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	828

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	828

9.11	NR Measurements	829

9.11.1	NR SS-RSRP Measurements	829

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	829

9.11.2	NR SS-RSRQ Measurements	830

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	830

9.11.3	NR SS-SINR Measurements	830

9.11.4	NR SS-RSRP Measurements under CCA	831

9.11.5	NR SS-RSRQ Measurements under CCA	831

9.11.6	NR SS-SINR Measurements under CCA	831

9.11.7	NR RSSI Measurements under CCA	831

9.11.8	NR Channel Occupancy Measurements under CCA	832

10	Measurements Performance Requirements for E-UTRAN	832

10.1	Received Interference Power	832

10.1.1	Absolute accuracy requirement	832

10.1.2	Relative accuracy requirement	832

10.1.3	Received Interference Power measurement report mapping	833

10.2	Angle of Arrival (AOA)	833

10.2.1	Range/mapping	833

10.3	Timing Advance (TADV)	833

10.3.1	Report mapping	833

11	ProSe Requirements in Any Cell Selection state	834

11.1	Introduction	834

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	834

11.2.1	Introduction	834

11.2.2	ProSe UE transmission timing	834

11.3	Initiation/Cease of SLSS Transmissions	835

11.3.1	Introduction	835

11.3.2	Requirements	835

11.4	Measurements for ProSe in Any Cell Selection State	835

11.4.1	Introduction	835

11.4.2	Requirements	835

11.4.2.1	E-UTRA FDD	835

11.4.2.2	E-UTRA TDD	836

11.5	Selection / Reselection of ProSe Synchronization Reference	836

11.5.1	Introduction	836

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	836

11.5.2.1	Introduction	836

11.5.2.2	Requirements	836

11.6	Void	837

11.7	Selection / Reselection of ProSe relay UE	837

11.7.1	Introduction	837

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	837

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	837

12.1	Introduction	837

12.2	Transmit Timing	838

12.2.1	GNSS as timing reference	838

12.3	Interruption	838

12.4	Reliability of GNSS signal	838

13	V2X Requirements	838

13.1	Introduction	838

13.2	UE Transmit Timing	839

13.2.1	Introduction	839

13.2.2	GNSS as synchronization reference source	839

13.2.3	Serving cell/PCell as synchronization reference source	839

13.2.4	SyncRef UE as synchronization reference source	839

13.3	Initiation/Cease of SLSS Transmissions	839

13.3.1	Introduction	839

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	839

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	840

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	840

13.4	Selection / Reselection of V2X Synchronization Reference Source	841

13.5	Autonomous Resource Selection/Reselection measurements	842

13.5.1	Introduction	842

13.5.2	PSSCH-RSRP measurements	842

13.5.3	S-RSSI measurements	842

13.6	Congestion Control measurements	842

13.7	Interruption	842

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	842

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	842

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	843

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	843

13.8	Reliability of GNSS signal	843

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	844

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	844

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1017

A.4.2.42.1	Test Purpose and Environment	1017

A.4.2.42.2	Test Requirements	1019

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1020

A.4.2.43.1	Test Purpose and Environment	1020

A.4.2.43.2	Test Requirements	1022

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1023

A.4.2.44.1	Test Purpose and Environment	1023

A.4.2.44.2	Test Requirements	1025

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1026

A.4.2.45.1	Test Purpose and Environment	1026

A.4.2.45.2	Test Requirements	1028

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1029

A.4.2.46.1	Test Purpose and Environment	1029

A.4.2.46.2	Test Requirements	1031

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1032

A.4.2.47.1	Test Purpose and Environment	1032

A.4.2.47.2	Test Requirements	1034

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1035

A.4.2.48.1	Test Purpose and Environment	1035

A.4.2.48.2	Test Requirements	1036

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1037

A.4.2.49.1	Test Purpose and Environment	1037

A.4.2.49.2	Test Requirements	1039

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1040

A.4.2.50.1	Test Purpose and Environment	1040

A.4.2.50.2	Test Requirements	1042

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1043

A.4.2.51.1	Test Purpose and Environment	1043

A.4.2.51.2	Test Requirements	1045

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1046

A.4.2.52.1	Test Purpose and Environment	1046

A.4.2.52.2	Test Requirements	1048

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1049

A.4.2.53.1	Test Purpose and Environment	1049

A.4.2.53.2	Test Requirements	1050

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1051

A.4.2.54.1	Test Purpose and Environment	1051

A.4.2.54.2	Test Requirements	1053

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1054

A.4.2.55.1	Test Purpose and Environment	1054

A.4.2.55.2	Test Requirements	1056

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1057

A.4.2.56.1	Test Purpose and Environment	1057

A.4.2.56.2	Test Requirements	1058

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1059

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1059

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1059

A.4.3.1.1.1	Test Purpose and Environment	1059

A.4.3.1.1.2	Test Requirements	1061

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1061

A.4.3.1.2.1	Test Purpose and Environment	1061

A.4.3.1.2.2	Test Requirements	1064

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1064

A.4.3.1.3.1	Test Purpose and Environment	1064

A.4.3.1.3.2	Test Requirements	1067

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1067

A.4.3.1.4.1	Test Purpose and Environment	1067

A.4.3.1.4.2	Test Requirements	1068

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1068

A.4.3.1.5.1	Test Purpose and Environment	1068

A.4.3.1.5.2	Test Requirements	1071

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1072

A.4.3.2.1	Test Purpose and Environment	1072

A.4.3.2.1.1	Void	1072

A.4.3.2.1.2	1.28Mcps TDD option	1072

A.4.3.2.1.3	Void	1074

A.4.3.2.2	Test Requirements	1074

A.4.3.2.2.1	1.28Mcps TDD option	1074

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1074

A.4.3.2A.1	Test Purpose and Environment	1074

A.4.3.2A.2	Test Requirements	1078

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1078

A.4.3.3.1	Test Purpose and Environment	1078

A.4.3.3.2	Test Requirements	1081

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1081

A.4.3.3A.1	Test Purpose and Environment	1081

A.4.3.3A.2	Test Requirements	1086

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1087

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1087

A.4.3.4.1.1	Test Purpose and Environment	1087

A.4.3.4.1.2	Test Requirements	1089

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1089

A.4.3.4.2.1	Test Purpose and Environment	1089

A.4.3.4.2.2	Test Requirements	1091

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1091

A.4.3.4.3.1	Test Purpose and Environment	1091

A.4.3.4.3.2	Test Requirements	1094

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1094

A.4.3.4.4.1	Test Purpose and Environment	1094

A.4.3.4.4.2	Test Requirements	1098

A.4.4	E-UTRAN to GSM Cell Re-Selection	1098

A.4.4.1	E-UTRAN FDD – GSM:	1098

A.4.4.1.1	Test Purpose and Environment	1098

A.4.4.1.2	Test Requirements	1100

A.4.4.2	E-UTRAN TDD – GSM:	1100

A.4.4.2.1	Test Purpose and Environment	1100

A.4.4.2.2	Test Requirements	1102

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1103

A.4.5.1	E-UTRAN FDD – HRPD	1103

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1103

A.4.5.1.1.1	Test Purpose and Environment	1103

A.4.5.1.1.2	Test Requirements	1105

A.4.5.2	E-UTRAN TDD – HRPD	1105

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1105

A.4.5.2.1.1	Test Purpose and Environment	1105

A.4.5.2.1.2	Test Requirements	1108

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1108

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1108

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1108

A.4.6.1.1.1	Test Purpose and Environment	1108

A.4.6.1.1.2	Test Requirements	1111

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1111

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1111

A.4.6.2.1.1	Test Purpose and Environment	1111

A.4.6.2.1.2	Test Requirements	1114

A.4.7	Idle State Positioning Measurement for UE category NB1	1114

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1114

A.4.7.1.1	Test Purpose and Environment	1114

A.4.7.1.2	Test Requirements	1119

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1120

A.4.7.2.1	Test Purpose and Environment	1120

A.4.7.4.2	Test Requirements	1124

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1125

A.4.7.3.1	Test Purpose and Environment	1125

A.4.7.3.2	Test Requirements	1129

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1130

A.4.7.4.1	Test Purpose and Environment	1130

A.4.7.4.2	Test Requirements	1134

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1135

A.5.1	E-UTRAN Handover	1135

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1135

A.5.1.1.1	Test Purpose and Environment	1135

A.5.1.1.2	Test Requirements	1137

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1137

A.5.1.2.1	Test Purpose and Environment	1137

A.5.1.2.2	Test Requirements	1139

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1139

A.5.1.3.1	Test Purpose and Environment	1139

A.5.1.3.2	Test Requirements	1141

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1141

A.5.1.4.1	Test Purpose and Environment	1141

A.5.1.4.2	Test Requirements	1143

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1143

A.5.1.5.1	Test Purpose and Environment	1143

A.5.1.5.2	Test Requirements	1145

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1145

A.5.1.6.1	Test Purpose and Environment	1145

A.5.1.6.2	Test Requirements	1147

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1147

A.5.1.7.1	Test Purpose and Environment	1147

A.5.1.7.2	Test Requirements	1150

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1150

A.5.1.8.1	Test Purpose and Environment	1150

A.5.1.8.2	Test Requirements	1153

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1153

A.5.1.9.1	Test Purpose and Environment	1153

A.5.1.9.2	Test Requirements	1154

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1154

A.5.1.10.1	Test Purpose and Environment	1154

A.5.1.10.2	Test Requirements	1156

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1156

A.5.1.11.1	Test Purpose and Environment	1156

A.5.1.11.2	Test Requirements	1158

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1158

A.5.1.12.1	Test Purpose and Environment	1158

A.5.1.12.2	Test Requirements	1160

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1160

A.5.1.13.1	Test Purpose and Environment	1160

A.5.1.13.2	Test Requirements	1162

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1163

A.5.1.14.1	Test Purpose and Environment	1163

A.5.1.14.2	Test Requirements	1164

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1165

A.5.1.15.1	Test Purpose and Environment	1165

A.5.1.15.2	Test Requirements	1166

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1167

A.5.1.16.1	Test Purpose and Environment	1167

A.5.1.16.2	Test Requirements	1168

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1169

A.5.1.17.1	Test Purpose and Environment	1169

A.5.1.17.2	Test Requirements	1170

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1171

A.5.1.18.1	Test Purpose and Environment	1171

A.5.1.18.2	Test Requirements	1172

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1173

A.5.1.19.1	Test Purpose and Environment	1173

A.5.1.19.2	Test Requirements	1174

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1175

A.5.1.20.1	Test Purpose and Environment	1175

A.5.1.20.2	Test Requirements	1176

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1177

A.5.1.21.1	Test Purpose and Environment	1177

A.5.1.21.2	Test Requirements	1178

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1178

A.5.1.22.1	Test Purpose and Environment	1178

A.5.1.22.2	Test Requirements	1180

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1180

A.5.1.23.1	Test Purpose and Environment	1180

A.5.1.23.2	Test Requirements	1182

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1182

A.5.1.24.1	Test Purpose and Environment	1182

A.5.1.24.2	Test Requirements	1184

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1184

A.5.1.25.1	Test Purpose and Environment	1184

A.5.1.25.2	Test Requirements	1186

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1187

A.5.1.26.1	Test Purpose and Environment	1187

A.5.1.26.2	Test Requirements	1188

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1189

A.5.1.27.1	Test Purpose and Environment	1189

A.5.1.27.2	Test Requirements	1190

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1191

A.5.1.28.1	Test Purpose and Environment	1191

A.5.1.28.2	Test Requirements	1192

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1193

A.5.1.29.1	Test Purpose and Environment	1193

A.5.1.29.2	Test Requirements	1194

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1195

A.5.1.30.1	Test Purpose and Environment	1195

A.5.1.30.2	Test Requirements	1196

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1197

A.5.1.31.1	Test Purpose and Environment	1197

A.5.1.31.2	Test Requirements	1198

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1199

A.5.1.32.1	Test Purpose and Environment	1199

A.5.1.32.2	Test Requirements	1200

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1201

A.5.1.33.1	Test Purpose and Environment	1201

A.5.1.13.2	Test Requirements	1202

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1203

A.5.1.34.1	Test Purpose and Environment	1203

A.5.1.34.2	Test Requirements	1204

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1205

A.5.1.35.1	Test Purpose and Environment	1205

A.5.1.35.2	Test Requirements	1206

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1207

A.5.1.36.1	Test Purpose and Environment	1207

A.5.1.36.2	Test Requirements	1208

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1209

A.5.1.37.1	Test Purpose and Environment	1209

A.5.1.37.2	Test Requirements	1210

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1211

A.5.1.38.1	Test Purpose and Environment	1211

A.5.1.38.2	Test Requirements	1212

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1213

A.5.1.39.1	Test Purpose and Environment	1213

A.5.1.39.2	Test Requirements	1214

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1215

A.5.1.40.1	Test Purpose and Environment	1215

A.5.1.40.2	Test Requirements	1216

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1217

A.5.1.41.1	Test Purpose and Environment	1217

A.5.1.41.2	Test Requirements	1219

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1220

A.5.1.42.1	Test Purpose and Environment	1220

A.5.1.42.2	Test Requirements	1221

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1222

A.5.1.43.1	Test Purpose and Environment	1222

A.5.1.43.2	Test Requirements	1223

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1224

A.5.1.44.1	Test Purpose and Environment	1224

A.5.1.44.2	Test Requirements	1225

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1226

A.5.1.45.1	Test Purpose and Environment	1226

A.5.1.45.1	Test Requirements	1227

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1228

A.5.1.46.1	Test Purpose and Environment	1228

A.5.1.46.2	Test Requirements	1229

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1230

A.5.1.47.1	Test Purpose and Environment	1230

A.5.1.47.2	Test Requirements	1231

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1232

A.5.1.48.1	Test Purpose and Environment	1232

A.5.1.48.2	Test Requirements	1233

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1234

A.5.1.49.1	Test Purpose and Environment	1234

A.5.1.49.2	Test Requirements	1235

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1236

A.5.1.50.1	Test Purpose and Environment	1236

A.5.1.50.2	Test Requirements	1237

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1238

A.5.1.51.1	Test Purpose and Environment	1238

A.5.1.51.2	Test Requirements	1240

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1240

A.5.152.1	Test Purpose and Environment	1240

A.5.1.52.2	Test Requirements	1243

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1244

A.5.1.53.1	Test Purpose and Environment	1244

A.5.1.53.2	Test Requirements	1246

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1247

A.5.1.54.1	Test Purpose and Environment	1247

A.5.1.54.2	Test Requirements	1249

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1250

A.5.1.55.1	Test Purpose and Environment	1250

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1254

A.5.1.56.1	Test Purpose and Environment	1254

A.5.1.56.2	Test Requirements	1257

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1258

A.5.1.57.1	Test Purpose and Environment	1258

A.5.1.57.2	Test Requirements	1260

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1260

A.5.1.58.1	Test Purpose and Environment	1260

A.5.1.58.2	Test Requirements	1263

A.5.2	E-UTRAN Handover to other RATs	1263

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1263

A.5.2.1.1	Test Purpose and Environment	1263

A.5.2.1.2	Test Requirements	1265

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1266

A.5.2.2.1	Test Purpose and Environment	1266

A.5.2.2.2	Test Requirements	1269

A.5.2.3 E-UTRAN FDD- GSM Handover	1269

A.5.2.3.1	Test Purpose and Environment	1269

A.5.2.3.2	Test Requirements	1270

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1271

A.5.2.4.1	Test Purpose and Environment	1271

A.5.2.4.1.1	Void	1271

A.5.2.4.1.2	1.28 Mcps TDD option	1271

A.5.2.4.1.3	Void	1273

A.5.2.4.2	Test Requirements	1273

A.5.2.4.2.1	Void	1273

A.5.2.4.2.2	1.28 Mcps TDD option	1273

A.5.2.4.2.3	Void	1273

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1273

A.5.2.5.1	Test Purpose and Environment	1273

A.5.2.5.1.3	Void	1276

A.5.2.5.2	Test Requirements	1276

A.5.2.5.2.1	Void	1276

A.5.2.5.2.2	1.28 Mcps TDD option	1276

A.5.2.5.2.3	Void	1276

A.5.2.6	E-UTRAN TDD - GSM Handover	1276

A.5.2.6.1	Test Purpose and Environment	1276

A.5.2.6.2	Test Requirements	1278

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1279

A.5.2.7.1	Test Purpose and Environment	1279

A.5.2.7.2	Test Requirements	1281

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1281

A.5.2.8.1	Test Purpose and Environment	1281

A.5.2.8.2	Test Requirements	1282

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1283

A.5.2.9.1	Test Purpose and Environment	1283

A.5.2.9.2	Test Requirements	1284

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1285

A.5.2.10.1	Test Purpose and Environment	1285

A.5.2.10.2	Test Requirements	1287

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1287

A.5.2.10A.1	Test Purpose and Environment	1287

A.5.2.10A.2	Test Requirements	1290

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1290

A.5.2.10B.1	Test Purpose and Environment	1290

A.5.2.10B.2	Test Requirements	1293

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1293

A.5.2.11.1	Test Purpose and Environment	1293

A.5.2.11.2	Test Requirements	1293

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1294

A.5.3.1	E-UTRAN FDD – HRPD Handover	1294

A.5.3.1.1	Test Purpose and Environment	1294

A.5.3.1.2	Test Requirements	1296

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1296

A.5.3.2.1	Test Purpose and Environment	1296

A.5.3.2.2	Test Requirements	1299

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1299

A.5.3.3.1	Test Purpose and Environment	1299

A.5.3.3.2	Test Requirements	1302

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1302

A.5.3.4.1	Test Purpose and Environment	1302

A.5.3.4.2	Test Requirements	1303

A.5.3.5	E-UTRAN TDD – HRPD Handover	1304

A.5.3.5.1	Test Purpose and Environment	1304

A.5.3.5.2	Test Requirements	1307

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1307

A.5.3.6.1	Test Purpose and Environment	1307

A.5.3.6.2	Test Requirements	1310

A.6	RRC Connection Control	1310

A.6.1	RRC Re-establishment	1310

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1310

A.6.1.1.1	Test Purpose and Environment	1310

A.6.1.1.2	Test Requirements	1311

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1312

A.6.1.2.1	Test Purpose and Environment	1312

A.6.1.2.2	Test Requirements	1313

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1314

A.6.1.3.1	Test Purpose and Environment	1314

A.6.1.3.2	Test Requirements	1315

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1316

A.6.1.4.1	Test Purpose and Environment	1316

A.6.1.4.2	Test Requirements	1317

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1318

A.6.1.5.1	Test Purpose and Environment	1318

A.6.1.5.2	Test Requirements	1318

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1318

A.6.1.6.1	Test Purpose and Environment	1318

A.6.1.6.2	Test Requirements	1320

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1321

A.6.1.7.1	Test Purpose and Environment	1321

A.6.1.7.2	Test Requirements	1322

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1323

A.6.1.8.1	Test Purpose and Environment	1323

A.6.1.8.2	Test Requirements	1324

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1325

A.6.1.9.1	Test Purpose and Environment	1325

A.6.1.9.2	Test Requirements	1326

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1327

A.6.1.10.1	Test Purpose and Environment	1327

A.6.1.10.2	Test Requirements	1328

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1329

A.6.1.11.1	Test Purpose and Environment	1329

A.6.1.11.2	Test Requirements	1330

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1331

A.6.1.12.1	Test Purpose and Environment	1331

A.6.1.12.2	Test Requirements	1332

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1333

A.6.1.13.1	Test Purpose and Environment	1333

A.6.1.13.2	Test Requirements	1334

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1335

A.6.1.14.1	Test Purpose and Environment	1335

A.6.1.14.2	Test Requirements	1336

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1337

A.6.1.15.1	Test Purpose and Environment	1337

A.6.1.15.2	Test Requirements	1339

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1340

A.6.1.16.1	Test Purpose and Environment	1340

A.6.1.16.2	Test Requirements	1342

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1343

A.6.1.17.1	Test Purpose and Environment	1343

A.6.1.17.2	Test Requirements	1344

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1345

A.6.1.18.1	Test Purpose and Environment	1345

A.6.1.18.2	Test Requirements	1346

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1347

A.6.1.19.1	Test Purpose and Environment	1347

A.6.1.19.2	Test Requirements	1348

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1349

A.6.1.20.1	Test Purpose and Environment	1349

A.6.1.20.2	Test Requirements	1350

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1351

A.6.1.21.1	Test Purpose and Environment	1351

A.6.1.21.2	Test Requirements	1352

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1353

A.6.1.22.1	Test Purpose and Environment	1353

A.6.1.22.2	Test Requirements	1354

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1355

A.6.1.23.1	Test Purpose and Environment	1355

A.6.1.23.2	Test Requirements	1357

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1358

A.6.1.24.1	Test Purpose and Environment	1358

A.6.1.24.2	Test Requirements	1360

A.6.2	Random Access	1361

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1361

A.6.2.1.1	Test Purpose and Environment	1361

A.6.2.1.2	Test Requirements	1362

A.6.2.1.2.1	Random Access Response Reception	1362

A.6.2.1.2.2	No Random Access Response Reception	1362

A.6.2.1.2.3	Receiving a NACK on msg3	1362

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1363

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1363

A.6.2.1.2.6	Contention Resolution Timer expiry	1363

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1363

A.6.2.2.1	Test Purpose and Environment	1363

A.6.2.2.2	Test Requirements	1364

A.6.2.2.2.1	Random Access Response Reception	1365

A.6.2.2.2.2	No Random Access Response Reception	1365

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1365

A.6.2.3.1	Test Purpose and Environment	1365

A.6.2.3.2	Test Requirements	1367

A.6.2.3.2.1	Random Access Response Reception	1367

A.6.2.3.2.2	No Random Access Response reception	1367

A.6.2.3.2.3	Receiving a NACK on msg3	1367

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1368

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1368

A.6.2.3.2.6	Contention Resolution Timer expiry	1368

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1368

A.6.2.4.1	Test Purpose and Environment	1368

A.6.2.4.2	Test Requirements	1370

A.6.2.4.2.1	Random Access Response Reception	1370

A.6.2.4.2.2	No Random Access Response Reception	1370

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1370

A.6.2.5.1	Test Purpose and Environment	1370

A.6.2.5.2	Test Requirements	1371

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.6.1	Test Purpose and Environment	1371

A.6.2.6.2	Test Requirements	1371

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1372

A.6.2.7.1	Test Purpose and Environment	1372

A.6.2.7.2	Test Requirements	1374

A.6.2.7.2.1	Random Access Response Reception	1374

A.6.2.7.2.2	No Random Access Response Reception	1374

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1374

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1375

A.6.2.8.1	Test Purpose and Environment	1375

A.6.2.8.2	Test Requirements	1377

A.6.2.8.2.1	Random Access Response Reception	1377

A.6.2.8.2.2	No Random Access Response Reception	1377

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1377

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1378

A.6.2.9.1	Test Purpose and Environment	1378

A.6.2.9.2	Test Requirements	1381

A.6.2.9.2.1	Random Access Response Reception	1381

A.6.2.9.2.2	No Random Access Response Reception	1382

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1382

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1383

A.6.2.10.1	Test Purpose and Environment	1383

A.6.2.10.2	Test Requirements	1385

A.6.2.10.2.1	Random Access Response Reception	1385

A.6.2.10.2.2	No Random Access Response Reception	1386

A.6.2.10.2.3	Receiving a NACK on msg3	1386

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1386

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1386

A.6.2.10.2.6	Contention Resolution Timer expiry	1386

A.6.2.10.2.7	PRACH Resource Selection	1387

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1387

A.6.2.11.1	Test Purpose and Environment	1387

A.6.2.11.2	Test Requirements	1389

A.6.2.11.2.1	Random Access Response Reception	1389

A.6.2.11.2.2	No Random Access Response Reception	1390

A.6.2.11.2.3	Receiving a NACK on msg3	1390

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1390

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1390

A.6.2.11.2.6	Contention Resolution Timer expiry	1390

A.6.2.11.2.7	PRACH Resource Selection	1391

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1391

A.6.2.12.1	Test Purpose and Environment	1391

A.6.2.12.2	Test Requirements	1393

A.6.2.12.2.1	Random Access Response Reception	1393

A.6.2.12.2.2	No Random Access Response Reception	1394

A.6.2.12.2.3	Receiving a NACK on msg3	1394

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1394

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1394

A.6.2.12.2.6	Contention Resolution Timer expiry	1394

A.6.2.12.2.7	PRACH Resource Selection	1395

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1395

A.6.2.13.1	Test Purpose and Environment	1395

A.6.2.13.2	Test Requirements	1397

A.6.2.13.2.1	Random Access Response Reception	1397

A.6.2.13.2.2	No Random Access Response Reception	1398

A.6.2.13.2.3	Receiving a NACK on msg3	1398

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1398

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1398

A.6.2.13.2.6	Contention Resolution Timer expiry	1398

A.6.2.13.2.7	PRACH Resource Selection	1399

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1399

A.6.2.14.1	Test Purpose and Environment	1399

A.6.2.14.2	Test Requirements	1401

A.6.2.14.2.1	Random Access Response Reception	1401

A.6.2.14.2.2	No Random Access Response Reception	1402

A.6.2.14.2.3	Receiving a NACK on msg3	1402

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1402

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1402

A.6.2.14.2.6	Contention Resolution Timer expiry	1402

A.6.2.14.2.7	PRACH Resource Selection	1403

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1403

A.6.2.15.1	Test Purpose and Environment	1403

A.6.2.15.2	Test Requirements	1405

A.6.2.15.2.1	Random Access Response Reception	1405

A.6.2.15.2.2	No Random Access Response Reception	1406

A.6.2.15.2.3	Receiving a NACK on msg3	1406

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1406

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1406

A.6.2.15.2.6	Contention Resolution Timer expiry	1406

A.6.2.15.2.7	PRACH Resource Selection	1407

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1407

A.6.2.16.1	Test Purpose and Environment	1407

A.6.2.16.2	Test Requirements	1410

A.6.2.16.2.1	Random Access Response Reception	1410

A.6.2.16.2.2	No Random Access Response Reception	1411

A.6.2.16.2.3	Receiving a NACK on msg3	1411

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1411

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1411

A.6.2.16.2.6	Contention Resolution Timer expiry	1411

A.6.2.16.2.7	NPRACH Resource Selection	1411

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1412

A.6.2.17.1	Test Purpose and Environment	1412

A.6.2.17.2	Test Requirements	1415

A.6.2.17.2.1	Random Access Response Reception	1415

A.6.2.17.2.2	No Random Access Response Reception	1416

A.6.2.17.2.3	Receiving a NACK on msg3	1416

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1416

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1416

A.6.2.17.2.6	Contention Resolution Timer expiry	1416

A.6.2.17.2.7	NPRACH Resource Selection	1416

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1417

A.6.2.18.1	Test Purpose and Environment	1417

A.6.2.18.2	Test Requirements	1420

A.6.2.18.2.1	Random Access Response Reception	1420

A.6.2.18.2.2	No Random Access Response Reception	1421

A.6.2.18.2.3	Receiving a NACK on msg3	1421

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1421

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1421

A.6.2.18.2.6	Contention Resolution Timer expiry	1421

A.6.2.18.2.7	NPRACH Resource Selection	1421

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1422

A.6.2.19.1	Test Purpose and Environment	1422

A.6.2.19.2	Test Requirements	1424

A.6.2.19.2.1	Random Access Response Reception	1424

A.6.2.19.2.2	No Random Access Response Reception	1425

A.6.2.19.2.3	Receiving a NACK on msg3	1425

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1425

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1425

A.6.2.19.2.6	Contention Resolution Timer expiry	1425

A.6.2.19.2.7	NPRACH Resource Selection	1425

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1426

A.6.2.20.1	Test Purpose and Environment	1426

A.6.2.20.2	Test Requirements	1428

A.6.2.20.2.1	Random Access Response Reception	1428

A.6.2.20.2.2	No Random Access Response Reception	1429

A.6.2.20.2.3	Receiving a NACK on msg3	1429

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1429

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1429

A.6.2.20.2.6	Contention Resolution Timer expiry	1429

A.6.2.20.2.7	NPRACH Resource Selection	1429

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1430

A.6.2.21.1	Test Purpose and Environment	1430

A.6.2.21.2	Test Requirements	1432

A.6.2.21.2.1	Random Access Response Reception	1432

A.6.2.21.2.2	No Random Access Response Reception	1433

A.6.2.21.2.3	Receiving a NACK on msg3	1433

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1433

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1433

A.6.2.21.2.6	Contention Resolution Timer expiry	1433

A.6.2.21.2.7	NPRACH Resource Selection	1433

A.6.3	RRC Connection Release with Redirection	1434

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1434

A.6.3.1.1	Test Purpose and Environment	1434

A.6.3.1.2	Test Requirements	1436

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1436

A.6.3.2.1	Test Purpose and Environment	1436

A.6.3.2.2	Test Requirements	1438

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1438

A.6.3.3.1	Test Purpose and Environment	1438

A.6.3.3.2	Test Requirements	1439

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1440

A.6.3.4.1	Test Purpose and Environment	1440

A.6.3.4.2	Test Requirements	1441

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1442

A.6.3.5.1	Test Purpose and Environment	1442

A.6.3.5.2	Test Requirements	1444

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1444

A.6.3.6.1	Test Purpose and Environment	1444

A. 6.3.6.2	Test Requirements	1447

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1447

A.6.3.7.1	Test Purpose and Environment	1447

A.6.3.7.2	Test Requirements	1450

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1450

A.6.3.8.1	Test Purpose and Environment	1450

A.6.3.8.2	Test Requirements	1453

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1453

A.6.3.9.1	Test Purpose and Environment	1453

A.6.3.9.2	Test Requirements	1455

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1455

A.6.3.10.1	Test Purpose and Environment	1455

A.6.3.10.2	Test Requirements	1457

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1457

A.6.3.11.1	Test Purpose and Environment	1457

A.6.3.11.2	Test Requirements	1459

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1459

A.6.3.12.1	Test Purpose and Environment	1459

A.6.3.12.2	Test Requirements	1462

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1462

A.6.3.13.1	Test Purpose and Environment	1462

A.6.3.13.2	Test Parameters	1462

A.6.3.13.3	Test Requirements	1466

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1525

A.7.2.13.1	Test Purpose and Environment	1525

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1527

A.7.2.14.1	Test Purpose and Environment	1527

A.7.2.14.2	Test Requirements	1529

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1529

A.7.2.15.1	Test Purpose and Environment	1529

A.7.2.15.2	Test Requirements	1531

A.7.3	Radio Link Monitoring	1531

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1539

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1539

A.7.3.3.1	Test Purpose and Environment	1539

A.7.3.3.2	Test Requirements	1543

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1543

A.7.3.4.1	Test Purpose and Environment	1543

A.7.3.4.2	Test Requirements	1547

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1547

A.7.3.5.1	Test Purpose and Environment	1547

A.7.3.5.2	Test Requirements	1550

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1551

A.7.3.6.1	Test Purpose and Environment	1551

A.7.3.6.2	Test Requirements	1554

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1554

A.7.3.7.1	Test Purpose and Environment	1554

A.7.3.7.2	Test Requirements	1557

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1557

A.7.3.8.1	Test Purpose and Environment	1557

A.7.3.8.2	Test Requirements	1560

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1560

A.7.3.9.2	Test Requirements	1563

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1563

A.7.3.10.1	Test Purpose and Environment	1563

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1567

A.7.3.11.1	Test Purpose and Environment	1567

A.7.3.11.2	Test Requirements	1572

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1572

A.7.3.12.1	Test Purpose and Environment	1572

A.7.3.12.2	Test Requirements	1577

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1577

A.7.3.13.1	Test Purpose and Environment	1577

A.7.3.13.2	Test Requirements	1580

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1580

A.7.3.14.1	Test Purpose and Environment	1580

A.7.3.14.2	Test Requirements	1583

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1583

A.7.3.15.1	Test Purpose and Environment	1583

A.7.3.15.2	Test Requirements	1587

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.16.1	Test Purpose and Environment	1587

A.7.3.16.2	Test Requirements	1592

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1592

A.7.3.17.1	Test Purpose and Environment	1592

A.7.3.17.2	Test Requirements	1596

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.18.1	Test Purpose and Environment	1596

A.7.3.18.2	Test Requirements	1600

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1600

A.7.3.19.1	Test Purpose and Environment	1600

A.7.3.19.2	Test Requirements	1605

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1605

A.7.3.20.1	Test Purpose and Environment	1605

A.7.3.20.2	Test Requirements	1610

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1610

A.7.3.21.1	Test Purpose and Environment	1610

A.7.3.21.2	Test Requirements	1615

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1615

A.7.3.22.1	Test Purpose and Environment	1615

A.7.3.22.2	Test Requirements	1620

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1620

A.7.3.23.1	Test Purpose and Environment	1620

A.7.3.23.2	Test Requirements	1621

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1621

A.7.3.24.1	Test Purpose and Environment	1621

A.7.3.24.2	Test Requirements	1622

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1622

A.7.3.25.1	Test Purpose and Environment	1622

A.7.3.25.2	Test Requirements	1623

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1623

A.7.3.26.1	Test Purpose and Environment	1623

A.7.3.26.2	Test Requirements	1626

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1626

A.7.3.27.1	Test Purpose and Environment	1626

A.7.3.27.2	Test Requirements	1629

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1629

A.7.3.28.1	Test Purpose and Environment	1629

A.7.3.28.2	Test Requirements	1632

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1632

A.7.3.29.1	Test Purpose and Environment	1632

A.7.3.29.2	Test Requirements	1635

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1635

A.7.3.30.1	Test Purpose and Environment	1635

A.7.3.30.2	Test Requirements	1638

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1638

A.7.3.31.1	Test Purpose and Environment	1638

A.7.3.31.2	Test Requirements	1641

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1641

A.7.3.32.1	Test Purpose and Environment	1641

A.7.3.32.2	Test Requirements	1644

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1644

A.7.3.33.1	Test Purpose and Environment	1644

A.7.3.33.2	Test Requirements	1647

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1647

A.7.3.34.1	Test Purpose and Environment	1647

A.7.3.34.2	Test Requirements	1650

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1650

A.7.3.35.1	Test Purpose and Environment	1650

A.7.3.35.2	Test Requirements	1653

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1653

A.7.3.36.1	Test Purpose and Environment	1653

A.7.3.36.2	Test Requirements	1656

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1656

A.7.3.37.1	Test Purpose and Environment	1656

A.7.3.37.2	Test Requirements	1659

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1659

A.7.3.38.1	Test Purpose and Environment	1659

A.7.3.38.2	Test Requirements	1662

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1662

A.7.3.39.1	Test Purpose and Environment	1662

A.7.3.39.2	Test Requirements	1665

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1666

A.7.3.40.1	Test Purpose and Environment	1666

A.7.3.40.2	Test Requirements	1668

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1669

A.7.3.41.1	Test Purpose and Environment	1669

A.7.3.41.2	Test Requirements	1672

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1672

A.7.3.42.1	Test Purpose and Environment	1672

A.7.3.42.2	Test Requirements	1675

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1675

A.7.3.43.1	Test Purpose and Environment	1675

A.7.3.43.2	Test Requirements	1678

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1678

A.7.3.44.1	Test Purpose and Environment	1678

A.7.3.44.2	Test Requirements	1681

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1682

A.7.3.45.1	Test Purpose and Environment	1682

A.7.3.45.2	Test Requirements	1684

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1685

A.7.3.46.1	Test Purpose and Environment	1685

A.7.3.46.2	Test Requirements	1688

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1688

A.7.3.47.1	Test Purpose and Environment	1688

A.7.3.47.2	Test Requirements	1691

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1691

A.7.3.48.1	Test Purpose and Environment	1691

A.7.3.48.2	Test Requirements	1694

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1694

A.7.3.49.1	Test Purpose and Environment	1694

A.7.3.49.2	Test Requirements	1697

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1697

A.7.3.50.1	Test Purpose and Environment	1697

A.7.3.50.2	Test Requirements	1700

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1700

A.7.3.51.1	Test Purpose and Environment	1700

A.7.3.51.2	Test Requirements	1703

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1703

A.7.3.52.1	Test Purpose and Environment	1703

A.7.3.52.2	Test Requirements	1706

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1706

A.7.3.53.1	Test Purpose and Environment	1706

A.7.3.53.2	Test Requirements	1709

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1709

A.7.3.54.1	Test Purpose and Environment	1709

A.7.3.54.2	Test Requirements	1712

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1712

A.7.3.55.1	Test Purpose and Environment	1712

A.7.3.55.2	Test Requirements	1715

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1715

A.7.3.56.1	Test Purpose and Environment	1715

A.7.3.56.2	Test Requirements	1718

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1718

A.7.3.57.1	Test Purpose and Environment	1718

A.7.3.57.2	Test Requirements	1721

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1721

A.7.3.58.1	Test Purpose and Environment	1721

A.7.3.58.2	Test Requirements	1724

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1724

A.7.3.59.1	Test Purpose and Environment	1724

A.7.3.59.2	Test Requirements	1727

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1727

A.7.3.60.1	Test Purpose and Environment	1727

A.7.3.60.2	Test Requirements	1731

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1731

A.7.3.61.1	Test Purpose and Environment	1731

A.7.3.61.2	Test Requirements	1735

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1735

A.7.3.62.1	Test Purpose and Environment	1735

A.7.3.62.2	Test Requirements	1739

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1739

A.7.3.63.1	Test Purpose and Environment	1739

A.7.3.63.2	Test Requirements	1743

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.64.1	Test Purpose and Environment	1743

A.7.3.64.2	Test Requirements	1746

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1747

A.7.3.65.1	Test Purpose and Environment	1747

A.7.3.65.2	Test Requirements	1750

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1751

A.7.3.66.1	Test Purpose and Environment	1751

A.7.3.66.2	Test Requirements	1753

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1754

A.7.3.67.1	Test Purpose and Environment	1754

A.7.3.67.2	Test Requirements	1757

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1758

A.7.3.68.1	Test Purpose and Environment	1758

A.7.3.68.2	Test Requirements	1760

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1760

A.7.3.69.1	Test Purpose and Environment	1760

A.7.3.69.2	Test Requirements	1762

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.70.1	Test Purpose and Environment	1762

A.7.3.70.2	Test Requirements	1764

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1764

A.7.3.71.1	Test Purpose and Environment	1764

A.7.3.71.2	Test Requirements	1767

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1767

A.7.3.72.1	Test Purpose and Environment	1767

A.7.3.72.2	Test Requirements	1769

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1769

A.7.3.73.1	Test Purpose and Environment	1769

A.7.3.73.2	Test Requirements	1771

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1771

A.7.3.74.1	Test Purpose and Environment	1771

A.7.3.74.2	Test Requirements	1774

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1774

A.7.3.75.1	Test Purpose and Environment	1774

A.7.3.75.2	Test Requirements	1776

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1777

A.7.3.76.1	Test Purpose and Environment	1777

A.7.3.76.2	Test Requirements	1780

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1780

A.7.3.77.1	Test Purpose and Environment	1780

A.7.3.77.2	Test Requirements	1783

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1783

A.7.3.78.1	Test Purpose and Environment	1783

A.7.3.78.2	Test Requirements	1786

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1786

A.7.3.79.1	Test Purpose and Environment	1786

A.7.3.79.2	Test Requirements	1789

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1789

A.7.3.80.1	Test Purpose and Environment	1789

A.7.3.80.2	Test Requirements	1792

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1792

A.7.3.81.1	Test Purpose and Environment	1792

A.7.3.81.2	Test Requirements	1795

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.82.1	Test Purpose and Environment	1795

A.7.3.82.2	Test Requirements	1798

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1798

A.7.3.83.1	Test Purpose and Environment	1798

A.7.3.83.2	Test Requirements	1799

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1800

A.7.3.84.1	Test Purpose and Environment	1800

A.7.3.84.2	Test Requirements	1802

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.85.1	Test Purpose and Environment	1802

A.7.3.85.2	Test Requirements	1803

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.86.1	Test Purpose and Environment	1804

A.7.3.86.2	Test Requirements	1806

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.87.1	Test Purpose and Environment	1806

A.7.3.87.2	Test Requirements	1808

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1809

A.7.3.88.1	Test Purpose and Environment	1809

A.7.3.88.2	Test Requirements	1813

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1813

A.7.3.89.1	Test Purpose and Environment	1813

A.7.3.89.2	Test Requirements	1817

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1817

A.7.3.90.1	Test Purpose and Environment	1817

A.7.3.90.2	Test Requirements	1821

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1821

A.7.3.91.1	Test Purpose and Environment	1821

A.7.3.91.2	Test Requirements	1825

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1825

A.7.3.92.1	Test Purpose and Environment	1825

A.7.3.92.2	Test Requirements	1828

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1829

A.7.3.93.1	Test Purpose and Environment	1829

A.7.3.93.2	Test Requirements	1832

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1833

A.7.3.94.1	Test Purpose and Environment	1833

A.7.3.94.2	Test Requirements	1835

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1836

A.7.3.95.1	Test Purpose and Environment	1836

A.7.3.95.2	Test Requirements	1839

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1839

A.7.3.96.1	Test Purpose and Environment	1839

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1841

A.7.3.97.1	Test Purpose and Environment	1841

A.7.3.97.2	Test Requirements	1844

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1844

A.7.3.98.1	Test Purpose and Environment	1844

A.7.3.98.2	Test Requirements	1847

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1847

A.7.3.99.1	Test Purpose and Environment	1847

A.7.3.99.2	Test Requirements	1850

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1850

A.7.3.100.1	Test Purpose and Environment	1850

A.7.3.100.2	Test Requirements	1852

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1852

A.7.3.101.1	Test Purpose and Environment	1852

A.7.3.101.2	Test Requirements	1854

A.7.4	Interruption for Dual Connectivity	1854

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1854

A.7.4.1.1	Test Purpose and Environment	1854

A.7.4.1.2	Test Requirements	1857

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1857

A.7.4.2.1	Test Purpose and Environment	1857

A.7.4.2.2	Test Requirements	1859

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1859

A.7.4.3.1	Test Purpose and Environment	1859

A.7.4.3.2	Test Requirements	1861

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.4.1	Test Purpose and Environment	1861

A.7.4.4.2	Test Requirements	1864

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1864

A.7.4.5.1	Test Purpose and Environment	1864

A.7.4.5.2	Test Requirements	1866

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1866

A.7.4.6.1	Test Purpose and Environment	1866

A.7.4.6.2	Test Requirements	1868

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1868

A.7.4.7.1	Test Purpose and Environment	1868

A.7.4.7.2	Test Requirements	1871

A.7.5	Proximity-based Services	1871

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1871

A.7.5.1.1	Test Purpose and Environment	1871

A.7.5.1.2	Test Requirements	1872

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1873

A.7.5.2.1	Test Purpose and Environment	1873

A.7.5.1.2	Test Requirements	1873

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1874

A.7.5.3.1	Test Purpose and Environment	1874

A.7.5.3.2	Test Requirements	1875

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1876

A.7.5.4.1	Test Purpose and Environment	1876

A.7.5.4.2	Test Requirements	1877

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1878

A.7.5.5.1	Test Purpose and Environment	1878

A.7.5.5.2	Test Requirements	1880

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1881

A.7.5.6.1	Test Purpose and Environment	1881

A.7.5.6.2	Test Requirements	1882

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1883

A.7.5.7.1	Test Purpose and Environment	1883

A.7.5.7.2	Test Requirements	1885

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1885

A.7.5.8.1	Test Purpose and Environment	1885

A.7.5.8.2	Test Requirements	1887

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1888

A.7.5.9.1	Test Purpose and Environment	1888

A.7.5.9.2	Test Requirements	1890

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1891

A.7.5.10.1	Test Purpose and Environment	1891

A.7.5.10.2	Test Requirements	1893

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1894

A.7.5.11.1	Test Purpose and Environment	1894

A.7.5.11.2	Test Requirements	1896

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1896

A.7.5.12.1	Test Purpose and Environment	1896

A.7.5.12.2	Test Requirements	1900

A.7.6	Interruption for carrier aggregation	1901

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1901

A.7.6.1.1	Test Purpose and Environment	1901

A.7.6.1.2	Test Requirements	1904

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1904

A.7.6.2.1	Test Purpose and Environment	1904

A.7.6.2.2	Test Requirements	1907

A.8	UE Measurements Procedures	1907

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1907

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1907

A.8.1.1.1	Test Purpose and Environment	1907

A.8.1.1.2	Test Requirements	1909

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1910

A.8.1.2.1	Test Purpose and Environment	1910

A.8.1.2.2	Test Requirements	1911

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1912

A.8.1.3.1	Test Purpose and Environment	1912

A.8.1.3.2	Test Requirements	1914

A.8.1.4	Void	1914

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1914

A.8.1.5.1	Test Purpose and Environment	1914

A.8.1.5.2	Test Requirements	1916

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1916

A.8.1.6.1	Test Purpose and Environment	1916

A.8.1.6.2	Test Requirements	1918

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1918

A.8.1.7.1	Test Purpose and Environment	1918

A.8.1.7.2	Test Requirements	1920

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1921

A.8.1.8.1	Test Purpose and Environment	1921

A.8.1.8.2	Test Requirements	1923

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1924

A.8.1.9.1	Test Purpose and Environment	1924

A.8.1.9.2	Test Requirements	1924

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1924

A.8.1.10.1	Test Purpose and Environment	1924

A.8.1.10.2	Test Requirements	1925

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1925

A.8.1.11.1	Test Purpose and Environment	1925

A.8.1.11.2	Test Requirements	1927

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1928

A.8.1.12.1	Test Purpose and Environment	1928

A.8.1.12.2	Test Requirements	1929

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1930

A.8.1.13.1	Test Purpose and Environment	1930

A.8.1.13.2	Test Requirements	1932

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1932

A.8.1.14.1	Test Purpose and Environment	1932

A.8.1.14.2	Test Requirements	1934

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1935

A.8.1.15.1	Test Purpose and Environment	1935

A.8.1.15.2	Test Requirements	1936

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1937

A.8.1.16.1	Test Purpose and Environment	1937

A.8.1.16.2	Test Requirements	1939

A.8.1.17	Void	1939

A.8.1.18	Void	1939

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1939

A.8.1.19.1	Test Purpose and Environment	1939

A.8.1.19.2	Test Requirements	1941

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1942

A.8.1.20.1	Test Purpose and Environment	1942

A.8.1.20.2	Test Requirements	1944

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.21.1	Test Purpose and Environment	1944

A.8.1.21.2	Test Requirements	1946

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.22.1	Test Purpose and Environment	1947

A.8.1.22.2	Test Requirements	1949

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1949

A.8.1.23.1	Test Purpose and Environment	1949

A.8.1.23.2	Test Requirements	1950

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1951

A.8.1.24.1	Test Purpose and Environment	1951

A.8.1.24.2	Test Requirements	1953

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.25.1	Test Purpose and Environment	1953

A.8.1.25.2	Test Requirements	1955

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.26.1	Test Purpose and Environment	1955

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1958

A.8.1.27.1	Test Purpose and Environment	1958

A.8.1.27.2	Test Requirements	1960

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1960

A.8.1.28.1	Test Purpose and Environment	1960

A.8.1.28.2	Test Requirements	1962

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.29.1	Test Purpose and Environment	1962

A.8.1.29.2	Test Requirements	1964

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.30.1	Test Purpose and Environment	1965

A.8.1.30.2	Test Requirements	1967

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1967

A.8.1.31.1	Test Purpose and Environment	1967

A.8.1.31.2	Test Requirements	1969

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1970

A.8.1.32.1	Test Purpose and Environment	1970

A.8.1.32.2	Test Requirements	1971

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.33.1	Test Purpose and Environment	1972

A.8.1.33.2	Test Requirements	1973

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.34.1	Test Purpose and Environment	1974

A.8.1.34.2	Test Requirements	1975

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.35.1	Test Purpose and Environment	1976

A.8.1.35.2	Test Requirements	1977

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1978

A.8.1.36.1	Test Purpose and Environment	1978

A.8.1.36.2	Test Requirements	1979

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1980

A.8.1.37.1	Test Purpose and Environment	1980

A.8.1.37.2	Test Requirements	1982

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.38.1	Test Purpose and Environment	1982

A.8.1.38.2	Test Requirements	1983

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.39.1	Test Purpose and Environment	1984

A.8.1.39.2	Test Requirements	1986

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1986

A.8.1.40.1	Test Purpose and Environment	1986

A.8.1.40.2	Test Requirements	1988

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1988

A.8.1.41.1	Test Purpose and Environment	1988

A.8.1.41.2	Test Requirement	1990

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1990

A.8.1.42.1	Test Purpose and Environment	1990

A.8.1.42.2	Test Requirement	1991

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1991

A.8.1.43.1	Test Purpose and Environment	1991

A.8.1.43.2	Test Requirements	1994

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	1994

A.8.1.44.1	Test Purpose and Environment	1994

A.8.1.44.2	Test Requirements	1997

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	1998

A.8.1.45.1	Test Purpose and Environment	1998

A.8.1.45.2	Test Requirements	2000

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2001

A.8.1.46.1	Test Purpose and Environment	2001

A.8.1.46.2	Test Requirements	2002

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2003

A.8.1.47.1	Test Purpose and Environment	2003

A.8.1.47.2	Test Requirements	2005

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2006

A.8.1.48.1	Test Purpose and Environment	2006

A.8.1.48.2	Test Requirements	2008

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2009

A.8.1.49.1	Test Purpose and Environment	2009

A.8.1.49.2	Test Requirements	2010

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2011

A.8.1.50.1	Test Purpose and Environment	2011

A.8.1.50.2	Test Requirements	2013

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2014

A.8.1.51.1	Test Purpose and Environment	2014

A.8.1.51.2	Test Requirements	2016

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2017

A.8.1.52.1	Test Purpose and Environment	2017

A.8.1.52.2	Test Requirements	2018

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2019

A.8.1.53.1	Test Purpose and Environment	2019

A.8.1.53.2	Test Requirements	2021

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2022

A.8.1.54.1	Test Purpose and Environment	2022

A.8.1.54.2	Test Requirements	2024

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2025

A.8.1.55.1	Test Purpose and Environment	2025

A.8.1.55.2	Test Requirements	2026

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2027

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2027

A.8.2.1.1	Test Purpose and Environment	2027

A.8.2.1.2	Test Requirements	2028

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2029

A.8.2.2.1	Test Purpose and Environment	2029

A.8.2.2.2	Test Requirements	2031

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2031

A.8.2.3.1	Test Purpose and Environment	2031

A.8.2.3.2	Test Requirements	2033

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2034

A.8.2.4.1	Test Purpose and Environment	2034

A.8.2.4.2	Test Requirements	2036

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2036

A.8.2.5.1	Test Purpose and Environment	2036

A.8.2.5.2	Test Requirements	2038

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2039

A.8.2.6.1	Test Purpose and Environment	2039

A.8.2.6.2	Test Requirements	2042

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2043

A.8.2.7.1	Test Purpose and Environment	2043

A.8.2.7.2	Test Requirements	2044

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2045

A.8.2.8.1	Test Purpose and Environment	2045

A.8.2.8.2	Test Requirements	2047

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2047

A.8.2.9.1	Test Purpose and Environment	2047

A.8.2.9.2	Test Requirements	2049

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2050

A.8.2.10.1	Test Purpose and Environment	2050

A.8.2.10.2	Test Requirements	2052

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2052

A.8.2.11.1	Test Purpose and Environment	2052

A.8.2.11.2	Test Requirements	2054

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2054

A.8.2.12.1	Test Purpose and Environment	2054

A.8.2.12.2	Test Requirements	2056

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2057

A.8.2.13.1	Test Purpose and Environment	2057

A.8.2.13.2	Test Requirements	2059

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2059

A.8.2.14.1	Test Purpose and Environment	2059

A.8.2.14.2	Test Requirement	2061

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2061

A.8.2.15.1	Test Purpose and Environment	2061

A.8.2.15.2	Test Requirements	2063

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2063

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2063

A.8.3.1.1	Test Purpose and Environment	2063

A.8.3.1.2	Test Requirements	2065

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2065

A.8.3.2.1	Test Purpose and Environment	2065

A.8.3.2.2	Test Requirements	2068

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2068

A.8.3.3.1	Test Purpose and Environment	2068

A.8.3.3.2	Test Requirements	2071

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2071

A.8.3.4.1	Test Purpose and Environment	2071

A.8.3.4.2	Test Requirements	2073

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2073

A.8.3.5.2	Test Requirements	2075

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2075

A.8.3.6.1	Test Purpose and Environment	2075

A.8.3.6.2	Test Requirements	2077

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2078

A.8.3.7.1	Test Purpose and Environment	2078

A.8.3.7.2	Test Requirements	2080

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2081

A.8.3.8.1	Test Purpose and Environment	2081

A.8.3.8.2	Test Requirements	2084

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2084

A.8.3.9.1	Test Purpose and Environment	2084

A.8.3.9.2	Test Requirements	2088

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2089

A.8.3.10.1	Test Purpose and Environment	2089

A.8.3.10.2	Test Requirements	2090

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2090

A.8.3.11.1	Test Purpose and Environment	2090

A.8.3.11.2	Test Requirement	2092

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2093

A.8.3.12.1	Test Purpose and Environment	2093

A.8.3.12.2	Test Requirement	2094

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2095

A.8.3.13.1	Test Purpose and Environment	2095

A.8.3.13.2	Test Requirement	2096

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2097

A.8.3.14.1	Test Purpose and Environment	2097

A.8.3.14.2	Test Requirement	2098

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2099

A.8.3.15.1	Test Purpose and Environment	2099

A.8.3.15.2	Test Requirement	2100

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2101

A.8.3.16.1	Test Purpose and Environment	2101

A.8.3.16.2	Test Requirement	2103

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2103

A.8.3.17.1	Test Purpose and Environment	2103

A.8.3.17.2	Test Requirement	2106

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2106

A.8.3.18.1	Test Purpose and Environment	2106

A.8.3.18.2	Test Requirement	2109

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2109

A.8.3.19.1	Test Purpose and Environment	2109

A.8.3.19.2	Test Requirement	2112

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2112

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2112

A.8.4.1.1	Test Purpose and Environment	2112

A.8.4.1.2	Test Requirements	2114

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2114

A.8.4.2.1	Test Purpose and Environment	2114

A.8.4.2.2	Test Requirements	2117

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2117

A.8.4.3.1	Test Purpose and Environment	2117

A.8.4.3.2	Test Requirements	2119

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2120

A.8.4.4.1	Test Purpose and Environment	2120

A.8.4.4.2	Test Requirements	2122

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2123

A.8.4.5.1	Test Purpose and Environment	2123

A.8.4.5.2	Test Requirements	2125

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2125

A.8.4.6.1	Test Purpose and Environment	2125

A.8.4.6.2	Test Requirements	2126

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2126

A.8.4.7.1	Test Purpose and Environment	2126

A.8.4.7.2	Test Requirements	2130

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2130

A.8.4.8.1	Test Purpose and Environment	2130

A.8.4.8.2	Test Requirements	2133

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2133

A.8.4.9.1	Test Purpose and Environment	2133

A.8.4.9.2	Test Requirements	2138

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2138

A.8.4.10.1	Test Purpose and Environment	2138

A.8.4.10.2	Test Requirements	2140

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2140

A.8.4.11.1	Test Purpose and Environment	2140

A.8.4.11.2	Test Requirement	2142

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2142

A.8.4.12.1	Test Purpose and Environment	2142

A.8.4.12.2	Test Requirement	2144

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2145

A.8.4.13.1	Test Purpose and Environment	2145

A.8.4.13.2	Test Requirement	2146

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2147

A.8.4.14.1	Test Purpose and Environment	2147

A.8.4.14.2	Test Requirement	2149

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2149

A.8.4.15.1	Test Purpose and Environment	2149

A.8.4.15.2	Test Requirement	2152

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2152

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2152

A.8.5.1.1	Test Purpose and Environment	2152

A.8.5.1.2	Test Requirements	2154

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2154

A.8.5.2.1	Test Purpose and Environment	2154

A.8.5.2.2	Test Requirements	2156

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2156

A.8.5.3.1	Test Purpose and Environment	2156

A.8.5.3.2	Test Requirements	2159

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2159

A.8.5.4.1	Test Purpose and Environment	2159

A.8.5.4.2	Test Requirements	2161

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2161

A.8.5.5.1	Test Purpose and Environment	2161

A.8.5.5.2	Test Requirements	2164

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2164

A.8.5.6.1	Test Purpose and Environment	2164

A.8.5.6.2	Test Requirements	2165

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2166

A.8.5.7.1	Test Purpose and Environment	2166

A.8.5.7.2	Test Requirements	2166

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2166

A.8.5.8.1	Test Purpose and Environment	2166

A.8.5.8.2	Test Requirements	2169

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2169

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2169

A.8.6.1.1	Test Purpose and Environment	2169

A.8.6.1.2	Test Requirements	2171

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2171

A.8.6.2.1	Test Purpose and Environment	2171

A.8.6.2.2	Test Requirements	2174

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2174

A.8.6.3.1	Test Purpose and Environment	2174

A.8.6.3.2	Test Requirements	2177

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2177

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2177

A.8.7.1.1	Test Purpose and Environment	2177

A.8.7.1.1.1	Void	2177

A.8.7.1.1.2	1.28 Mcps TDD option	2177

A.8.7.1.1.3	Void	2179

A.8.7.1.2	Test Requirements	2179

A.8.7.1.2.1	Void	2179

A.8.7.1.2.2	1.28 Mcps TDD option	2179

A.8.7.1.2.3	Void	2179

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2179

A.8.7.2.1	Test Purpose and Environment	2179

A.8.7.2.2	Test Requirements	2182

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2183

A.8.7.3.1	Test Purpose and Environment	2183

A.8.7.3.2	Test Parameters	2183

A.8.7.3.3	Test Requirements	2184

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2185

A.8.7.4.1	Test Purpose and Environment	2185

A.8.7.4.2	Test Requirements	2187

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2187

A.8.7.5.1	Test Purpose and Environment	2187

A.8.7.5.2	Test Requirements	2189

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2189

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2189

A.8.7A.1.1	Test Purpose and Environment	2189

A.8.7A.1.2	Test Requirements	2192

A.8.8	E-UTRAN FDD – GSM Measurements	2192

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2192

A.8.8.1.1	Test Purpose and Environment	2192

A.8.8.1.2	Test Requirements	2194

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2194

A.8.8.2.1	Test Purpose and Environment	2194

A.8.8.2.2	Test Requirements	2196

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2197

A.8.8.3.1	Test Purpose and Environment	2197

A.8.8.3.2	Test Requirements	2198

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2199

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2199

A.8.9.1.1	Test Purpose and Environment	2199

A.8.9.1.2	Test Requirements	2200

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2201

A.8.9.2.1	Test Purpose and Environment	2201

A.8.9.2.2	Test Requirements	2203

A.8.10	E-UTRAN TDD – GSM Measurements	2203

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2203

A.8.10.1.1	Test Purpose and Environment	2203

A.8.10.1.2	Test Requirements	2204

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2205

A.8.10.2.1	Test Purpose and Environment	2205

A.8.10.2.2	Test Requirements	2207

A.8.11	Monitoring of Multiple Layers	2207

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2207

A.8.11.1.1	Test Purpose and Environment	2207

A.8.11.1.2	Test Requirements	2209

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2210

A.8.11.2.1	Test Purpose and Environment	2210

A.8.11.2.2	Test Requirements	2211

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2212

A.8.11.3.1	Test Purpose and Environment	2212

A.8.11.3.2	Test Requirements	2214

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2214

A.8.11.4.1	Test Purpose and Environment	2214

A.8.11.4.2	Test Requirements	2217

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2217

A.8.11.5.1	Test Purpose and Environment	2217

A.8.11.5.2	Test Requirements	2219

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2220

A.8.11.6.1	Test Purpose and Environment	2220

A.8.11.6.2	Test Requirements	2222

A.8.12	RSTD Intra-frequency Measurements	2223

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2223

A.8.12.1.1	Test Purpose and Environment	2223

A.8.12.1.2	Test Requirements	2227

A.8.12.1.2A	Test Requirements for UE Category 1bis	2227

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2227

A.8.12.2.1	Test Purpose and Environment	2227

A.8.12.2.2	Test Requirements	2232

A.8.12.2.2A	Test Requirements for UE Category 1bis	2232

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2232

A.8.12.3.1	Test Purpose and Environment	2232

A.8.12.3.2	Test Requirements	2238

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2238

A.8.12.4.1	Test Purpose and Environment	2238

A.8.12.4.2	Test Requirements	2243

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2243

A.8.12.5.1	Test Purpose and Environment	2243

A.8.12.5.2	Test Requirements	2248

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2248

A.8.12.6.1	Test Purpose and Environment	2248

A.8.12.6.2	Test Requirements	2253

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2253

A.8.12.7.1	Test Purpose and Environment	2253

A.8.12.7.2	Test Requirements	2258

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2258

A.8.12.8.1	Test Purpose and Environment	2258

A.8.12.8.2	Test Requirements	2263

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2263

A.8.12.9.1	Test Purpose and Environment	2263

A.8.12.9.2	Test Requirements	2268

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2268

A.8.12.10.1	Test Purpose and Environment	2268

A.8.12.10.2	Test Requirements	2273

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2273

A.8.12.11.1	Test Purpose and Environment	2273

A.8.12.11.2	Test Requirements	2278

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2278

A.8.12.12.1	Test Purpose and Environment	2278

A.8.12.12.2	Test Requirements	2283

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2283

A.8.12.13.1	Test Purpose and Environment	2283

A.8.12.13.2	Test Requirements	2288

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2288

A.8.12.14.1	Test Purpose and Environment	2288

A.8.12.14.2	Test Requirements	2293

A.8.13	RSTD Inter-frequency Measurements	2293

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2293

A.8.13.1.1	Test Purpose and Environment	2293

A.8.13.1.2	Test Requirements	2298

A.8.13.1.2A	Test Requirements for UE Category 1bis	2298

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2298

A.8.13.2.1	Test Purpose and Environment	2298

A.8.13.2.2	Test Requirements	2304

A.8.13.2.2A	Test Requirements for UE Category 1bis	2304

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2304

A.8.13.3.1	Test Purpose and Environment	2304

A.8.13.3.2	Test Requirements	2311

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2311

A.8.13.4.1	Test Purpose and Environment	2311

A.8.13.4.2	Test Requirements	2317

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2317

A.8.13.5.1	Test Purpose and Environment	2317

A.8.13.5.2	Test Requirements	2323

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2323

A.8.13.6.1	Test Purpose and Environment	2323

A.8.13.6.2	Test Requirements	2329

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2329

A.8.13.7.1	Test Purpose and Environment	2329

A.8.13.7.2	Test Requirements	2335

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2335

A.8.13.8.1	Test Purpose and Environment	2335

A.8.13.8.2	Test Requirements	2341

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2341

A.8.13.9.1	Test Purpose and Environment	2341

A.8.13.9.2	Test Requirements	2347

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2347

A.8.13.10.1	Test Purpose and Environment	2347

A.8.13.10.2	Test Requirements	2353

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2353

A.8.13.11.1	Test Purpose and Environment	2353

A.8.13.11.2	Test Requirements	2359

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2359

A.8.13.12.1	Test Purpose and Environment	2359

A.8.13.12.2	Test Requirements	2365

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2365

A.8.13.13.1	Test Purpose and Environment	2365

A.8.13.13.2	Test Requirements	2371

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2371

A.8.13.14.1	Test Purpose and Environment	2371

A.8.13.14.2	Test Requirements	2377

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2377

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2377

A.8.14.1.1	Test Purpose and Environment	2377

A.8.14.1.2	Test Requirements	2379

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2379

A.8.14.2.1	Test Purpose and Environment	2379

A.8.14.2.2	Test Requirements	2382

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2382

A.8.14.3.1	Test Purpose and Environment	2382

A.8.14.3.2	Test Requirements	2384

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2385

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2385

A.8.15.1.1	Test Purpose and Environment	2385

A.8.15.1.2	Test Requirements	2386

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2386

A.8.15.2.1	Test Purpose and Environment	2386

A.8.15.2.2	Test Requirements	2389

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2389

A.8.15.3.1	Test Purpose and Environment	2389

A.8.15.3.2	Test Requirements	2391

A.8.16	E-UTRAN Carrier Aggregation Measurements	2392

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2392

A.8.16.1.1	Test Purpose and Environment	2392

A.8.16.1.2	Test Requirements	2394

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2394

A.8.16.2.1	Test Purpose and Environment	2394

A.8.16.2.2	Test Requirements	2396

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2396

A.8.16.3.1	Test Purpose and Environment	2396

A.8.16.3.2	Test Requirements	2398

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2399

A.8.16.3A.1	Test Purpose and Environment	2399

A.8.16.3A.2	Test Requirements	2400

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2401

A.8.16.4.1	Test Purpose and Environment	2401

A.8.16.4.2	Test Requirements	2402

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2403

A.8.16.4A.1	Test Purpose and Environment	2403

A.8.16.4A.2	Test Requirements	2405

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.5.1	Test Purpose and Environment	2406

A.8.16.5.2	Test Requirements	2406

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.6.1	Test Purpose and Environment	2406

A.8.16.6.2	Test Requirements	2407

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2407

A.8.16.7.1	 Test Purpose and Environment	2407

A.8.16.7.2	 Test Requirements	2408

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2408

A.8.16.8.1	 Test Purpose and Environment	2408

A.8.16.8.2	 Test Requirements	2409

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2409

A.8.16.9.1	Test Purpose and Environment	2409

A.8.16.9.2	Test Requirements	2410

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2410

A.8.16.10.1	Test Purpose and Environment	2410

A.8.16.10.2	Test Requirements	2411

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2411

A.8.16.11.1	 Test Purpose and Environment	2411

A.8.16.11.2	 Test Requirements	2412

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2412

A.8.16.12.1	 Test Purpose and Environment	2412

A.8.16.12.2	 Test Requirements	2412

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2412

A.8.16.13.1	Test Purpose and Environment	2412

A.8.16.13.2	Test Requirements	2413

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2413

A.8.16.14.1	Test Purpose and Environment	2413

A.8.16.14.2	Test Requirements	2414

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2414

A.8.16.15.1	 Test Purpose and Environment	2414

A.8.16.7.2	 Test Requirements	2414

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2414

A.8.16.16.1	 Test Purpose and Environment	2414

A.8.16.16.2	 Test Requirements	2415

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2415

A.8.16.17.1	Test Purpose and Environment	2415

A.8.16.17.2	Test Requirements	2417

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2418

A.8.16.17A.1	Test Purpose and Environment	2418

A.8.16.17A.2	Test Requirements	2418

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2418

A.8.16.17B.1	Test Purpose and Environment	2418

A.8.16.17B.2	Test Requirements	2419

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2419

A.8.16.17C.1	Test Purpose and Environment	2419

A.8.16.17C.2	Test Requirements	2419

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2419

A.8.16.18.1	Test Purpose and Environment	2419

A.8.16.18.2	Test Requirements	2421

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2422

A.8.16.18A.1	Test Purpose and Environment	2422

A.8.16.18A.2	Test Requirements	2422

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2422

A.8.16.18B.1	Test Purpose and Environment	2422

A.8.16.18B.2	Test Requirements	2423

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2423

A.8.16.18C.1	Test Purpose and Environment	2423

A.8.16.18C.2	Test Requirements	2423

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2423

A.8.16.18D.1	Test Purpose and Environment	2423

A.8.16.18D.2	Test Requirements	2424

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2424

A.8.16.19.1	Test Purpose and Environment	2424

A.8.16.19.2	Test Requirements	2426

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2426

A.8.16.19A.1	Test Purpose and Environment	2426

A.8.16.19A.2	Test Requirements	2427

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2427

A.8.16.19B.1	Test Purpose and Environment	2427

A.8.16.19B.2	Test Requirements	2427

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2427

A.8.16.19C.1	Test Purpose and Environment	2427

A.8.16.19C.2	Test Requirements	2428

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2428

A.8.16.20.1	Test Purpose and Environment	2428

A.8.16.20.2	Test Requirements	2430

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2431

A.8.16.20A.1	Test Purpose and Environment	2431

A.8.16.20A.2	Test Requirements	2431

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2431

A.8.16.20B.1	Test Purpose and Environment	2431

A.8.16.20B.2	Test Requirements	2432

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2432

A.8.16.20C.1	Test Purpose and Environment	2432

A.8.16.20C.2	Test Requirements	2432

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2432

A.8.16.20D.1	Test Purpose and Environment	2432

A.8.16.20D.2	Test Requirements	2433

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2433

A.8.16.21.1	Test Purpose and Environment	2433

A.8.16.21.2	Test Requirements	2435

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2435

A.8.16.22.1	Test Purpose and Environment	2435

A.8.16.22.2	Test Requirements	2437

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2437

A.8.16.23.1	Test Purpose and Environment	2437

A.8.16.23.2	Test Requirements	2440

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2440

A.8.16.24.1	Test Purpose and Environment	2440

A.8.16.24.2	Test Requirements	2443

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2443

A.8.16.25.1	Test Purpose and Environment	2443

A.8.16.25.2	Test Requirements	2446

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2446

A.8.16.26.1	Test Purpose and Environment	2446

A.8.16.26.2	Test Requirements	2449

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2449

A.8.16.27.1	Test Purpose and Environment	2449

A.8.16.27.2	Test Requirements	2454

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2454

A.8.16.28.1	Test Purpose and Environment	2454

A.8.16.28.2	Test Requirements	2459

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2459

A.8.16.29.1	Test Purpose and Environment	2459

A.8.16.29.2	Test Requirements	2464

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2464

A.8.16.30.1	Test Purpose and Environment	2464

A.8.16.30.2	Test Requirements	2469

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2469

A.8.16.31.1	Test Purpose and Environment	2469

A.8.16.31.2	Test Requirements	2474

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2474

A.8.16.32.1	Test Purpose and Environment	2474

A.8.16.32.2	Test Requirements	2479

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2479

A.8.16.33.1	Test Purpose and Environment	2479

A.8.16.33.2	Test Requirements	2484

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2484

A.8.16.34.1	Test Purpose and Environment	2484

A.8.16.34.2	Test Requirements	2489

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2489

A.8.16.35.1	Test Purpose and Environment	2489

A.8.16.35.2	Test Requirements	2491

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2492

A.8.16.36.1	Test Purpose and Environment	2492

A.8.16.36.2	Test Requirements	2494

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2495

A.8.16.37.1	Test Purpose and Environment	2495

A.8.16.37.2	Test Requirements	2497

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2498

A.8.16.38.1	Test Purpose and Environment	2498

A.8.16.38.2	Test Requirements	2500

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2501

A.8.16.39.1	Test Purpose and Environment	2501

A.8.16.39.2	Test Requirements	2504

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2504

A.8.16.40.1	Test Purpose and Environment	2504

A.8.16.40.2	Test Requirements	2507

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2507

A.8.16.41.1	Test Purpose and Environment	2507

A.8.16.41.2	Test Requirements	2509

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2510

A.8.16.42.1	Test Purpose and Environment	2510

A.8.16.42.2	Test Requirements	2512

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2513

A.8.16.43.1	Test Purpose and Environment	2513

A.8.16.43.2	Test Requirements	2516

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2516

A.8.16.44.1	Test Purpose and Environment	2516

A.8.16.44.2	Test Requirements	2519

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2519

A.8.16.45.1	Test Purpose and Environment	2519

A.8.16.45.2	Test Requirements	2522

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2522

A.8.16.46.1	Test Purpose and Environment	2522

A.8.16.46.2	Test Requirements	2524

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2525

A.8.16.47.1	Test Purpose and Environment	2525

A.8.16.47.2	Test Requirements	2527

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2528

A.8.16.48.1	Test Purpose and Environment	2528

A.8.16.48.2	Test Requirements	2530

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2531

A.8.16.49.1	Test Purpose and Environment	2531

A.8.16.49.2	Test Requirements	2534

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2534

A.8.16.50.1	Test Purpose and Environment	2534

A.8.16.50.2	Test Requirements	2536

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2537

A.8.16.51.1	Test Purpose and Environment	2537

A.8.16.51.2	Test Requirements	2542

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2542

A.8.16.52.1	Test Purpose and Environment	2542

A.8.16.52.2	Test Requirements	2547

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2547

A.8.16.53.1	Test Purpose and Environment	2547

A.8.16.53.2	Test Requirements	2551

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2551

A.8.16.54.1	Test Purpose and Environment	2551

A.8.16.54.2	Test Requirements	2555

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2555

A.8.16.55.1	Test Purpose and Environment	2555

A.8.16.55.2	Test Requirements	2561

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2561

A.8.16.56.1	Test Purpose and Environment	2561

A.8.16.56.2	Test Requirements	2567

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2567

A.8.16.57.1	Test Purpose and Environment	2567

A.8.16.57.2	Test Requirements	2569

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2570

A.8.16.58.1	Test Purpose and Environment	2570

A.8.16.58.2	Test Requirements	2572

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2573

A.8.16.59.1	Test Purpose and Environment	2573

A.8.16.59.2	Test Requirements	2577

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2578

A.8.16.60.1	Test Purpose and Environment	2578

A.8.16.60.2	Test Requirements	2581

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2582

A.8.16.61.1	Test Purpose and Environment	2582

A.8.16.61.2	Test Requirements	2585

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2585

A.8.16.62.1	Test Purpose and Environment	2585

A.8.16.62.2	Test Requirements	2587

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.63.1	Test Purpose and Environment	2588

A.8.16.63.2	Test Requirements	2591

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2592

A.8.16.64.1	Test Purpose and Environment	2592

A.8.16.64.2	Test Requirements	2596

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2596

A.8.16.65.1	Test Purpose and Environment	2596

A.8.16.65.2	Test Requirements	2601

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2601

A.8.16.66.1	Test Purpose and Environment	2601

A.8.16.66.2	Test Requirements	2606

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2606

A.8.16.67.1	Test Purpose and Environment	2606

A.8.16.67.2	Test Requirements	2611

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2611

A.8.16.68.1	Test Purpose and Environment	2611

A.8.16.68.2	Test Requirements	2616

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2616

A.8.16.69.1	Test Purpose and Environment	2616

A.8.16.69.2	Test Requirements	2620

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2620

A.8.16.70.1	Test Purpose and Environment	2620

A.8.16.70.2	Test Requirements	2624

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2624

A.8.16.71.1	Test Purpose and Environment	2624

A.8.16.71.2	Test Requirements	2629

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2629

A.8.16.72.1	Test Purpose and Environment	2629

A.8.16.72.2	Test Requirements	2634

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2634

A.8.16.73.1	Test Purpose and Environment	2634

A.8.16.73.2	Test Requirements	2640

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2640

A.8.16.74.1	Test Purpose and Environment	2640

A.8.16.74.2	Test Requirements	2646

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2646

A.8.16.75.1	Test Purpose and Environment	2646

A.8.16.75.2	Test Requirements	2652

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2652

A.8.16.76.1	Test Purpose and Environment	2652

A.8.16.76.2	Test Requirements	2658

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2658

A.8.16.77.1	Test Purpose and Environment	2658

A.8.16.77.2	Test Requirements	2662

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2662

A.8.16.78.1	Test Purpose and Environment	2662

A.8.16.78.2	Test Requirements	2664

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2665

A.8.16.79.1	Test Purpose and Environment	2665

A.8.16.79.2	Test Requirements	2672

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2672

A.8.16.80.1	Test Purpose and Environment	2672

A.8.16.80.2	Test Requirements	2679

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2679

A.8.16.81.1	Test Purpose and Environment	2679

A.8.16.81.2	Test Requirements	2685

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2685

A.8.16.82.1	Test Purpose and Environment	2685

A.8.16.82.2	Test Requirements	2691

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2691

A.8.16.83.1	Test Purpose and Environment	2691

A.8.16.83.2	Test Requirements	2696

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2696

A.8.16.84.1	Test Purpose and Environment	2696

A.8.16.84.2	Test Requirements	2702

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2702

A.8.16.85.1	Test Purpose and Environment	2702

A.8.16.85.2	Test Requirements	2706

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2706

A.8.16.86.1	Test Purpose and Environment	2706

A.8.16.86.2	Test Requirements	2709

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2709

A.8.16.87.1	Test Purpose and Environment	2709

A.8.16.87.2	Test Requirements	2715

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2715

A.8.16.88.1	Test Purpose and Environment	2715

A.8.16.88.2	Test Requirements	2723

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2723

A.8.16.89.1	Test Purpose and Environment	2723

A.8.16.89.2	Test Requirements	2730

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2730

A.8.16.90.1	Test Purpose and Environment	2730

A.8.16.90.2	Test Requirements	2737

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2737

A.8.16.91.1	Test Purpose and Environment	2737

A.8.16.91.2	Test Requirements	2743

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2743

A.8.16.92.1	Test Purpose and Environment	2743

A.8.16.92.2	Test Requirements	2751

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2751

A.8.16.93.1	Test Purpose and Environment	2751

A.8.16.93.2	Test Requirements	2758

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2758

A.8.16.94.1	Test Purpose and Environment	2758

A.8.16.94.2	Test Requirements	2765

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2765

A.8.16.95.1	Test Purpose and Environment	2765

A.8.16.95.2	Test Requirements	2773

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2773

A.8.16.96.1	Test Purpose and Environment	2773

A.8.16.96.2	Test Requirements	2782

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2782

A.8.16.97.1	Test Purpose and Environment	2782

A.8.16.97.2	Test Requirements	2789

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2789

A.8.16.98.1	Test Purpose and Environment	2789

A.8.16.98.2	Test Requirements	2797

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2797

A.8.16.99.1	Test Purpose and Environment	2797

A.8.16.99.2	Test Requirements	2807

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2807

A.8.16.100.1	Test Purpose and Environment	2807

A.8.16.100.2	Test Requirements	2817

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2817

A.8.16.101.1	Test Purpose and Environment	2817

A.8.16.101.2	Test Requirements	2827

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2827

A.8.16.102.1	Test Purpose and Environment	2827

A.8.16.102.2	Test Requirements	2838

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2838

A.8.16.103.1	Test Purpose and Environment	2838

A.8.16.103.2	Test Requirements	2843

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2844

A.8.16.104.1	Test Purpose and Environment	2844

A.8.16.104.2	Test Requirements	2848

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2849

A.8.16.105.1	Test Purpose and Environment	2849

A.8.16.105.2	Test Requirements	2853

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2854

A.8.16.106.1	Test Purpose and Environment	2854

A.8.16.106.2	Test Requirements	2858

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2858

A.8.16.107.1	Test Purpose and Environment	2858

A.8.16.107.2	Test Requirements	2860

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2861

A.8.16.108.1	Test Purpose and Environment	2861

A.8.16.108.2	Test Requirements	2863

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2864

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2864

A.8.17.1.1	Test Purpose and Environment	2864

A.8.17.1.2	Test Requirements	2870

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2870

A.8.17.2.1	Test Purpose and Environment	2870

A.8.17.2.2	Test Requirements	2877

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2877

A.8.17.3.1	Test Purpose and Environment	2877

A.8.17.3.2	Test Requirements	2878

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2878

A.8.17.4.1	Test Purpose and Environment	2878

A.8.17.4.2	Test Requirements	2879

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2879

A.8.17.5.1	Test Purpose and Environment	2879

A.8.17.5.2	Test Requirements	2880

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2880

A.8.17.6.1	Test Purpose and Environment	2880

A.8.17.6.2	Test Requirements	2881

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2882

A.8.17.7.1	Test Purpose and Environment	2882

A.8.17.7.2	Test Requirements	2882

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2883

A.8.17.8.1	Test Purpose and Environment	2883

A.8.17.8.2	Test Requirements	2883

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2884

A.8.17.9.1	Test Purpose and Environment	2884

A.8.17.9.2	Test Requirements	2885

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2885

A.8.17.10.1	Test Purpose and Environment	2885

A.8.17.10.2	Test Requirements	2892

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2893

A.8.17.11.1	Test Purpose and Environment	2893

A.8.17.11.2	Test Requirements	2899

A.8.18	E-UTRAN TDD – HRPD Measurements	2900

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2900

A.8.18.1.1	Test Purpose and Environment	2900

A.8.18.1.2	Test Requirements	2902

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2902

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2902

A.8.19.1.1	Test Purpose and Environment	2902

A.8.19.1.2	Test Requirements	2903

A.8.20	Inter-frequency/RAT Measurements in CA mode	2904

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2904

A.8.20.1.1	Test Purpose and Environment	2904

A.8.20.1.2	Test Requirements	2905

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2905

A.8.20.2.1	Test Purpose and Environment	2906

A.8.20.2.2	Test Requirements	2907

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2908

A.8.20.2A.1	Test Purpose and Environment	2908

A.8.20.2A.2	Test Requirements	2908

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2908

A.8.20.2B.1	Test Purpose and Environment	2908

A.8.20.2B.2	Test Requirements	2911

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2911

A.8.20.3.1	Test Purpose and Environment	2911

A.8.20.3.2	Test Requirements	2913

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2913

A.8.20.4.1	Test Purpose and Environment	2913

A.8.20.4.1.1	1.28 Mcps TDD option	2913

A.8.20.4.2	Test Requirements	2915

A.8.20.4.2.1	1.28 Mcps TDD option	2915

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2915

A.8.20.4A.1	Test Purpose and Environment	2915

A.8.20.4A.1.1	1.28 Mcps TDD option	2915

A.8.20.4A.2	Test Requirements	2915

A.8.20.4A.2.1	1.28 Mcps TDD option	2915

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2916

A.8.20.4B.1	Test Purpose and Environment	2916

A.8.20.4B.1.1	1.28 Mcps TDD option	2916

A.8.20.4B.2	Test Requirements	2918

A.8.20.4B.2.1	1.28 Mcps TDD option	2918

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2918

A.8.21.1	Test Purpose and Environment	2918

A.8.21.2 Test Requirements	2922

A.8.22	E-UTRAN Discovery Signal Measurements	2922

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2922

A.8.22.1.1	Test Purpose and Environment	2922

A.8.22.1.2	Test Requirements	2925

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2925

A.8.22.2.1	Test Purpose and Environment	2925

A.8.22.2.2	Test Requirements	2928

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2928

A.8.22.3.1	Test Purpose and Environment	2928

A.8.22.3.2	Test Requirements	2931

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2931

A.8.22.4.1	Test Purpose and Environment	2931

A.8.22.4.2	Test Requirements	2934

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2934

A.8.22.5.1	Test Purpose and Environment	2934

A.8.22.5.2	Test Requirements	2938

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2938

A.8.22.6.1	Test Purpose and Environment	2938

A.8.22.6.2	Test Requirements	2942

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2942

A.8.22.7.1	Test Purpose and Environment	2942

A.8.22.7.2	Test Requirements	2946

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2946

A.8.22.8.1	Test Purpose and Environment	2946

A.8.22.8.2	Test Requirements	2950

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2950

A.8.22.9.1	Test Purpose and Environment	2950

A.8.22.9.2	Test Requirements	2952

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2953

A.8.22.10.1	Test Purpose and Environment	2953

A.8.22.10.2	Test Requirements	2955

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2956

A.8.22.11.1	Test Purpose and Environment	2956

A.8.22.11.2	Test Requirements	2959

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2959

A.8.22.12.1	Test Purpose and Environment	2959

A.8.22.12.2	Test Requirements	2963

A.8.23	E-UTRAN Dual Connectivity Measurements	2963

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2963

A.8.23.1.1	Test Purpose and Environment	2963

A.8.23.1.2	Test Requirements	2966

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2966

A.8.23.2.1	Test Purpose and Environment	2966

A.8.23.2.2	Test Requirements	2969

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2969

A.8.23.3.1	Test Purpose and Environment	2969

A.8.23.3.2	Test Requirements	2972

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2972

A.8.23.4.1	Test Purpose and Environment	2972

A.8.23.4.2	Test Requirements	2975

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2975

A.8.23.5.1	Test Purpose and Environment	2975

A.8.23.5.2	Test Requirements	2978

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2978

A.8.23.6.1	Test Purpose and Environment	2978

A.8.23.6.2	Test Requirements	2981

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2981

A.8.23.7.1	Test Purpose and Environment	2981

A.8.23.7.2	Test Requirements	2983

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2984

A.8.23.8.1	Test Purpose and Environment	2984

A.8.23.8.2	Test Requirements	2986

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2987

A.8.23.9.1	Test Purpose and Environment	2987

A.8.23.9.2	Test Requirements	2990

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2990

A.8.23.10.1	Test Purpose and Environment	2990

A.8.23.10.2	Test Requirements	2993

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2993

A.8.23.11.1	Test Purpose and Environment	2993

A.8.23.11.2	Test Requirements	2996

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2996

A.8.23.12.1	Test Purpose and Environment	2996

A.8.23.12.2	Test Requirements	2999

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2999

A.8.23.13.1	Test Purpose and Environment	2999

A.8.23.13.2	Test Requirements	3002

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3002

A.8.23.14.1	Test Purpose and Environment	3002

A.8.23.14.2	Test Requirements	3005

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3005

A.8.23.15.1	Test Purpose and Environment	3005

A.8.23.15.2	Test Requirements	3008

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3008

A.8.23.16.1	Test Purpose and Environment	3008

A.8.23.16.2	Test Requirements	3009

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3010

A.8.23.17.1	Test Purpose and Environment	3010

A.8.23.17.2	Test Requirements	3012

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3012

A.8.23.18.1	Test Purpose and Environment	3012

A.8.23.18.2	Test Requirements	3013

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3014

A.8.23.19.1	Test Purpose and Environment	3014

A.8.23.19.2	Test Requirements	3015

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3016

A.8.23.20.1	Test Purpose and Environment	3016

A.8.23.20.2	Test Requirements	3017

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3018

A.8.23.21.1	Test Purpose and Environment	3018

A.8.23.21.2	Test Requirements	3019

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3020

A.8.23.22.1	Test Purpose and Environment	3020

A.8.23.22.2	Test Requirements	3021

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3022

A.8.23.23.1	Test Purpose and Environment	3022

A.8.23.23.2	Test Requirements	3023

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3024

A.8.23.24.1	Test Purpose and Environment	3024

A.8.23.24.2	Test Requirements	3026

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3027

A.8.23.25.1	Test Purpose and Environment	3027

A.8.23.25.2	Test Requirements	3029

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3030

A.8.23.26.1	Test Purpose and Environment	3030

A.8.23.26.2	Test Requirements	3032

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3033

A.8.23.27.1	Test Purpose and Environment	3033

A.8.23.27.2	Test Requirements	3036

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3036

A.8.23.28.1	Test Purpose and Environment	3036

A.8.23.28.2	Test Requirements	3040

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3040

A.8.23.29.1	Test Purpose and Environment	3040

A.8.23.29.2	Test Requirements	3044

A.8.24	Proximity-based Services	3044

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3044

A.8.24.1.1	Test Purpose and Environment	3044

A.8.24.1.2	Test Requirements	3045

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3046

A.8.24.2.1	Test Purpose and Environment	3046

A.8.24.2.2	Test Requirements	3047

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3047

A.8.24.3.1	Test Purpose and Environment	3048

A.8.24.3.2	Test Requirements	3049

A.8.25	E-UTRAN-WLAN Measurements	3050

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3050

A.8.25.1.1	Test Purpose and Environment	3050

A.8.25.1.2	Test Requirements	3052

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3052

A.8.25.2.1	Test Purpose and Environment	3052

A.8.25.2.2	Test Requirements	3054

A.8.26 Frame Structure 3 (FS3)	3054

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3054

A.8.26.1.1	Test Purpose and Environment	3054

A.8.26.1.2	Test Requirements	3056

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3057

A.8.26.2.1	Test Purpose and Environment	3057

A.8.26.2.2	Test Requirements	3059

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3060

A.8.26.3.1	Test Purpose and Environment	3060

A.8.26.3.2	Test Requirements	3063

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3063

A.8.26.3A.1	Test Purpose and Environment	3063

A.8.26.3A.2	Test Requirements	3066

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3066

A.8.26.4.1	Test Purpose and Environment	3066

A.8.26.4.2	Test Requirements	3070

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3070

A.8.26.4A.1	Test Purpose and Environment	3070

A.8.26.4A.2	Test Requirements	3074

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3074

A.8.26.5.1	Test Purpose and Environment	3074

A.8.26.5.2	Test Requirements	3077

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3077

A.8.26.5A.1	Test Purpose and Environment	3077

A.8.26.5A.2	Test Requirements	3081

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3081

A.8.26.6.1	Test Purpose and Environment	3081

A.8.26.6.2	Test Requirements	3085

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3085

A.8.26.6A.1	Test Purpose and Environment	3085

A.8.26.6A.2	Test Requirements	3089

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3089

A.8.26.7.1	Test Purpose and Environment	3089

A.8.26.7.2	Test Requirements	3092

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3092

A.8.26.8.1	Test Purpose and Environment	3092

A.8.26.8.2	Test Requirements	3096

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3096

A.8.26.9.1	Test Purpose and Environment	3096

A.8.26.9.2	Test Requirements	3099

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3099

A.8.26.10.1	Test Purpose and Environment	3099

A.8.26.10.2	Test Requirements	3102

A.9	Measurement Performance Requirements	2951

A.9.1	RSRP	2951

A.9.1.1	FDD Intra frequency case	2951

A.9.1.1.1	Test Purpose and Environment	2951

A.9.1.1.2	Test parameters	2951

A.9.1.1.3	Test Requirements	2954

A.9.1.2	TDD Intra frequency case	2954

A.9.1.2.1	Test Purpose and Environment	2954

A.9.1.2.2	Test parameters	2954

A.9.1.2.3	Test Requirements	2955

A.9.1.3	FDD—FDD Inter frequency case	2956

A.9.1.3.1	Test Purpose and Environment	2956

A.9.1.3.2	Test parameters	2956

A.9.1.3.3	Test Requirements	2959

A.9.1.4	TDD—TDD Inter frequency case	2959

A.9.1.4.1	Test Purpose and Environment	2959

A.9.1.4.2	Test parameters	2959

A.9.1.4.3	Test Requirements	2961

A.9.1.5	FDD—TDD Inter frequency case	2962

A.9.1.5.1	Test Purpose and Environment	2962

A.9.1.5.2	Test parameters	2962

A.9.1.5.3	Test Requirements	2963

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2964

A.9.1.6.1	Test Purpose and Environment	2964

A.9.1.6.2	Test parameters	2964

A.9.1.6.3	Test Requirements	2967

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2967

A.9.1.7.1	Test Purpose and Environment	2967

A.9.1.7.2	Test parameters	2967

A.9.1.7.3	Test Requirements	2970

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2970

A.9.1.8.1	Test Purpose and Environment	2970

A.9.1.8.2	Test parameters	2970

A.9.1.8.3	Test Requirements	2974

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2974

A.9.1.9.1	Test Purpose and Environment	2974

A.9.1.9.2	Test parameters	2974

A.9.1.9.3	Test Requirements	2976

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2977

A.9.1.10.1	Test Purpose and Environment	2977

A.9.1.10.2	Test parameters	2977

A.9.1.10.3	Test Requirements	2980

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2980

A.9.1.11.1	Test Purpose and Environment	2980

A.9.1.11.2	Test parameters	2980

A.9.1.11.3	Test Requirements	2984

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2984

A.9.1.12.1	Test Purpose and Environment	2984

A.9.1.12.2	Test parameters	2984

A.9.1.12.3	Test Requirements	2985

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2985

A.9.1.13.1	Test Purpose and Environment	2985

A.9.1.13.2	Test parameters	2985

A.9.1.13.3	Test Requirements	2986

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2986

A.9.1.14.1	Test Purpose and Environment	2986

A.9.1.14.2	Test parameters	2986

A.9.1.14.3	Test Requirements	2990

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2990

A.9.1.15.1	Test Purpose and Environment	2990

A.9.1.15.2	Test parameters	2990

A.9.1.15.3	Test Requirements	2992

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2993

A.9.1.16.1	Test Purpose and Environment	2993

A.9.1.16.2	Test parameters	2993

A.9.1.16.3	Test Requirements	2994

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2995

A.9.1.17.1	Test Purpose and Environment	2995

A.9.1.17.2	Test parameters	2995

A.9.1.17.3	Test Requirements	2996

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2997

A.9.1.18.1	Test Purpose and Environment	2997

A.9.1.18.2	Test parameters	2997

A.9.1.18.3	Test Requirements	2998

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2999

A.9.1.19.1	Test Purpose and Environment	2999

A.9.1.19.2	Test parameters	2999

A.9.1.19.3	Test Requirements	2999

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2999

A.9.1.20.1	Test Purpose and Environment	2999

A.9.1.20.2	Test parameters	2999

A.9.1.20.3	Test Requirements	3001

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3001

A.9.1.21.1	Test Purpose and Environment	3001

A.9.1.21.2	Test parameters	3001

A.9.1.21.3	Test Requirements	3001

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3001

A.9.1.22.1	Test Purpose and Environment	3001

A.9.1.22.2	Test parameters	3002

A.9.1.22.3	Test Requirements	3006

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3006

A.9.1.23.1	Test Purpose and Environment	3006

A.9.1.23.2	Test parameters	3006

A.9.1.23.3	Test Requirements	3010

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3010

A.9.1.24.1	Test Purpose and Environment	3010

A.9.1.24.2	Test parameters	3010

A.9.1.24.3	Test Requirements	3011

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3011

A.9.1.25.1	Test Purpose and Environment	3011

A.9.1.25.2	Test parameters	3011

A.9.1.25.3	Test Requirements	3014

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3014

A.9.1.26.1	Test Purpose and Environment	3014

A.9.1.26.2	Test parameters	3014

A.9.1.26.3	Test Requirements	3017

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3017

A.9.1.27.1	Test Purpose and Environment	3017

A.9.1.27.2	Test parameters	3017

A.9.1.27.3	Test Requirements	3020

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3020

A.9.1.28.1	Test Purpose and Environment	3020

A.9.1.28.2	Test parameters	3020

A.9.1.28.3	Test Requirements	3023

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3023

A.9.1.29.1	Test Purpose and Environment	3023

A.9.1.29.2	Test parameters	3023

A.9.1.29.3	Test Requirements	3026

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3026

A.9.1.30.1	Test Purpose and Environment	3026

A.9.1.30.2	Test parameters	3027

A.9.1.30.3	Test Requirements	3030

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3030

A.9.1.31.1	Test Purpose and Environment	3030

A.9.1.31.2	Test parameters	3030

A.9.1.31.3	Test Requirements	3033

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3034

A.9.1.32.1	Test Purpose and Environment	3034

A.9.1.32.2	Test parameters	3034

A.9.1.32.3	Test Requirements	3037

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3037

A.9.1.33.1	Test Purpose and Environment	3037

A.9.1.33.2	Test parameters	3037

A.9.1.33.3	Test Requirements	3040

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3040

A.9.1.34.1	Test Purpose and Environment	3040

A.9.1.34.2	Test parameters	3040

A.9.1.34.3	Test Requirements	3043

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3043

A.9.1.35.1	Test Purpose and Environment	3043

A.9.1.35.2	Test parameters	3043

A.9.1.35.3	Test Requirements	3046

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3047

A.9.1.36.1	Test Purpose and Environment	3047

A.9.1.36.2	Test parameters	3047

A.9.1.36.3	Test Requirements	3050

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3050

A.9.1.37.1	Test Purpose and Environment	3050

A.9.1.37.2	Test parameters	3050

A.9.1.37.3	Test Requirements	3053

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3054

A.9.1.38.1	Test Purpose and Environment	3054

A.9.1.38.2	Test parameters	3054

A.9.1.38.3	Test Requirements	3057

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3058

A.9.1.39.1	Test Purpose and Environment	3058

A.9.1.39.2	Test parameters	3058

A.9.1.39.3	Test Requirements	3064

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3064

A.9.1.40.1	Test Purpose and Environment	3064

A.9.1.40.2	Test parameters	3065

A.9.1.40.3	Test Requirements	3071

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3071

A.9.1.41.1	Test Purpose and Environment	3071

A.9.1.41.2	Test parameters	3071

A.9.1.41.3	Test Requirements	3074

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3074

A.9.1.42.1	Test Purpose and Environment	3074

A.9.1.42.2	Test parameters	3074

A.9.1.42.3	Test Requirements	3077

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3077

A.9.1.43.1	Test Purpose and Environment	3077

A.9.1.43.2	Test parameters	3077

A.9.1.43.3	Test Requirements	3078

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3079

A.9.1.44.1	Test Purpose and Environment	3079

A.9.1.44.2	Test parameters	3079

A.9.1.44.3	Test Requirements	3084

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3084

A.9.1.45.1	Test Purpose and Environment	3084

A.9.1.45.2	Test parameters	3084

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2582

A.9.1.73.1	Test Purpose and Environment	2582

A.9.1.73.2	Test parameters	2582

A.9.1.73.3	Test Requirements	2585

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2585

A.9.1.74.1	Test Purpose and Environment	2585

A.9.1.74.2	Test parameters	2585

A.9.1.74.3	Test Requirements	2586

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2586

A.9.1.75.1	Test Purpose and Environment	2586

A.9.1.75.2	Test parameters	2587

A.9.1.75.3	Test Requirements	2588

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2588

A.9.1.76.1	Test Purpose and Environment	2588

A.9.1.76.2	Test parameters	2588

A.9.1.76.3	Test Requirements	2591

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2591

A.9.1.77.1	Test Purpose and Environment	2591

A.9.1.77.2	Test parameters	2591

A.9.1.77.3	Test Requirements	2594

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2594

A.9.1.78.1	Test Purpose and Environment	2594

A.9.1.78.2	Test parameters	2594

A.9.1.78.3	Test Requirements	2597

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2597

A.9.1.79.1	Test Purpose and Environment	2597

A.9.1.79.2	Test parameters	2597

A.9.1.79.3	Test Requirements	2600

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2600

A.9.1.80.1	Test Purpose and Environment	2600

A.9.1.80.2	Test parameters	2600

A.9.1.80.3	Test Requirements	2603

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2603

A.9.1.81.1	Test Purpose and Environment	2603

A.9.1.78.2	Test parameters	2603

A.9.1.81.3	Test Requirements	2606

A.9.2	RSRQ	2606

A.9.2.1	FDD Intra frequency case	2606

A.9.2.1.1	Test Purpose and Environment	2606

A.9.2.1.2	Test parameters	2606

A.9.2.1.3	Test Requirements	2609

A.9.2.2	TDD Intra frequency case	2609

A.9.2.2.1	Test Purpose and Environment	2609

A.9.2.2.2	Test parameters	2609

A.9.2.2.3	Test Requirements	2611

A.9.2.3	FDD—FDD Inter frequency case	2611

A.9.2.3.1	Test Purpose and Environment	2611

A.9.2.3.2	Test parameters	2611

A.9.2.3.3	Test Requirements	2614

A.9.2.4	TDD—TDD Inter frequency case	2614

A.9.2.4.1	Test Purpose and Environment	2614

A.9.2.4.2	Test parameters	2614

A.9.2.4.3	Test Requirements	2616

A.9.2.4A	FDD—TDD Inter frequency case	2617

A.9.2.4A.1	Test Purpose and Environment	2617

A.9.2.4A.2	Test parameters	2617

A.9.2.4A.3	Test Requirements	2619

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2620

A.9.2.5.3	Test Requirements	2623

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2623

A.9.2.6.1	Test Purpose and Environment	2623

A.9.2.6.2	Test parameters	2624

A.9.2.6.3	Test Requirements	2627

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2627

A.9.2.7.1	Test Purpose and Environment	2627

A.9.2.7.2	Test parameters	2627

A.9.2.7.3	Test Requirements	2631

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2631

A.9.2.8.1	Test Purpose and Environment	2631

A.9.2.8.2	Test parameters	2631

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2829

A.9.3.2 E-UTRAN TDD	2829

A.9.3.2.1	Test Purpose and Environment	2829

A.9.3.2.2	Parameters	2829

A.9.3.2.3	Test Requirements	2832

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2832

A.9.3.3.1	Test Purpose and Environment	2832

A.9.3.3.2	Parameters	2832

A.9.3.3.3	Test Requirements	2833

A.9.4	UTRAN FDD CPICH Ec/No	2834

A.9.4.1	E-UTRAN FDD	2834

A.9.4.1.1	Test Purpose and Environment	2834

A.9.4.1.2	Parameters	2834

A.9.4.1.3	Test Requirements	2836

A.9.4.2	E-UTRAN TDD	2837

A.9.4.2.1	Test Purpose and Environment	2837

A.9.4.2.2	Parameters	2837

A.9.4.2.3	Test Requirements	2839

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2840

A.9.4.3.1	Test Purpose and Environment	2840

A.9.4.3.2	Parameters	2840

A.9.4.3.3	Test Requirements	2841

A.9.5	UTRAN TDD measurement	2841

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2841

A.9.5.1.1	Test Purpose and Environment	2841

A.9.5.1.2	Test parameters	2841

A.9.5.1.3	Test Requirements	2843

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2844

A.9.5.2.1	Test Purpose and Environment	2844

A.9.5.2.2	Test parameters	2844

A.9.5.2.3	Test Requirements	2845

A.9.6	GSM Carrier RSSI	2846

A.9.6.1	E-UTRAN FDD	2846

A.9.6.1.1	Test Purpose and Environment	2846

A.9.6.1.2	Test Requirements	2847

A.9.6.2	E-UTRAN TDD	2847

A.9.6.2.1	Test Purpose and Environment	2847

A.9.6.2.2	Test Requirements	2849

A.9.7	UE Rx – Tx Time Difference	2850

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2850

A.9.7.1.1	Test Purpose and Environment	2850

A.9.7.1.2	Test parameters	2850

A.9.7.1.3	Test Requirements	2852

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2852

A.9.7.2.1	Test Purpose and Environment	2852

A.9.7.2.2	Test parameters	2852

A.9.7.2.3	Test Requirements	2854

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2854

A.9.7.3.1	Test Purpose and Environment	2854

A.9.7.3.2	Test parameters	2854

A.9.7.3.3	Test Requirements	2857

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2857

A.9.7.4.1	Test Purpose and Environment	2857

A.9.7.4.2	Test Parameters	2857

A.9.7.4.3	Test Requirements	2860

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2860

A.9.7.5.1	Test Purpose and Environment	2860

A.9.7.5.2	Test parameters	2860

A.9.7.5.3	Test Requirements	2863

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2863

A.9.7.6.1	Test Purpose and Environment	2863

A.9.7.6.2	Test Parameters	2863

A.9.7.6.3	Test Requirements	2866

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2866

A.9.7.7.1	Test Purpose and Environment	2866

A.9.7.7.2	Test parameters	2866

A.9.7.7.3	Test Requirements	2867

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2868

A.9.7.8.1	Test Purpose and Environment	2868

A.9.7.8.2	Test parameters	2868

A.9.7.8.3	Test Requirements	2869

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.9.1	Test Purpose and Environment	2869

A.9.7.9.2	Test parameters	2869

A.9.7.9.3	Test Requirements	2870

A.9.8	RSTD	2871

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2871

A.9.8.1.1	Test Purpose and Environment	2871

A.9.8.1.2	Test Requirements	2873

A.9.8.1.2A	Test Requirements for UE Category 1bis	2873

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2874

A.9.8.2.1	Test Purpose and Environment	2874

A.9.8.2.2	Test Requirements	2877

A.9.8.2.2A	Test Requirements for UE Category 1bis	2877

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2878

A.9.8.3.1	Test Purpose and Environment	2878

A.9.8.3.2	Test Requirements	2880

A.9.8.3.2A	Test Requirements for UE Category 1bis	2880

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2881

A.9.8.4.1	Test Purpose and Environment	2881

A.9.8.4.2	Test Requirements	2883

A.9.8.4.2A	Test Requirements for UE Category 1bis	2883

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2884

A.9.8.5.1	Test Purpose and Environment	2884

A.9.8.5.2	Test Requirements	2886

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2886

A.9.8.6.1	Test Purpose and Environment	2886

A.9.8.6.2	Test Requirements	2889

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2889

A.9.8.7.1	Test Purpose and Environment	2889

A.9.8.7.2	Test Requirements	2890

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.8.1	Test Purpose and Environment	2890

A.9.8.8.2	Test Requirements	2891

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2891

A.9.8.9.1	Test Purpose and Environment	2891

A.9.8.9.2	Test Requirements	2892

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.10.1	Test Purpose and Environment	2892

A.9.8.10.2	Test Requirements	2893

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2893

A.9.8.11.1	Test Purpose and Environment	2893

A.9.8.11.2	Test Requirements	2894

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2894

A.9.8.12.1	Test Purpose and Environment	2894

A.9.8.12.2	Test Requirements	2895

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2895

A.9.8.13.1	Test Purpose and Environment	2895

A.9.8.13.2	Test Requirements	2896

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2896

A.9.8.14.1	Test Purpose and Environment	2896

A.9.8.14.2	Test Requirements	2903

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2903

A.9.8.15.1	Test Purpose and Environment	2903

A.9.8.15.2	Test Requirements	2909

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2909

A.9.8.16.1	Test Purpose and Environment	2909

A.9.8.16.2	Test Requirements	2912

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2912

A.9.8.17.1	Test Purpose and Environment	2912

A.9.8.17.2	Test Requirements	2915

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2915

A.9.8.18.1	Test Purpose and Environment	2915

A.9.8.18.2	Test Requirements	2918

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2918

A.9.8.19.1	Test Purpose and Environment	2918

A.9.8.19.2	Test Requirements	2921

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2921

A.9.8.20.1	Test Purpose and Environment	2921

A.9.8.20.2	Test Requirements	2924

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2925

A.9.8.21.1	Test Purpose and Environment	2925

A.9.8.21.2	Test Requirements	2928

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2929

A.9.8.22.1	Test Purpose and Environment	2929

A.9.8.22.2	Test Requirements	2932

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2933

A.9.8.23.1	Test Purpose and Environment	2933

A.9.8.23.2	Test Requirements	2936

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2936

A.9.8.24.1	Test Purpose and Environment	2936

A.9.8.24.2	Test Requirements	2940

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2941

A.9.8.25.1	Test Purpose and Environment	2941

A.9.8.25.2	Test Requirements	2944

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2945

A.9.8.26.1	Test Purpose and Environment	2945

A.9.8.26.2	Test Requirements	2947

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2947

A.9.8.27.1	Test Purpose and Environment	2947

A.9.8.27.2	Test Requirements	2950

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2950

A.9.8.28.1	Test Purpose and Environment	2950

A.9.8.28.2	Test Requirements	2953

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2953

A.9.8.29.1	Test Purpose and Environment	2953

A.9.8.29.2	Test Requirements	2957

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2958

A.9.8.30.1	Test Purpose and Environment	2958

A.9.8.30.2	Test Requirements	2961

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2962

A.9.8.31.1	Test Purpose and Environment	2962

A.9.8.31.2	Test Requirements	2966

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2966

A.9.8.32.1	Test Purpose and Environment	2966

A.9.8.32.2	Test Requirements	2969

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2969

A.9.8.33.1	Test Purpose and Environment	2969

A.9.8.33.2	Test Requirements	2972

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2972

A.9.8.34.1	Test Purpose and Environment	2972

A.9.8.34.2	Test Requirements	2975

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2975

A.9.8.35.1	Test Purpose and Environment	2975

A.9.8.35.2	Test Requirements	2978

A.9.9	RSRP and RSRQ on the serving cell	2978

A.9.9.1	FDD Intra frequency serving cell case	2978

A.9.9.1.1	Test Purpose and Environment	2978

A.9.9.1.2	Test parameters	2978

A.9.9.1.3	Test Requirements	2981

A.9.9.2	TDD Intra frequency serving cell case	2981

A.9.9.2.1	Test Purpose and Environment	2981

A.9.9.2.2	Test parameters	2981

A.9.9.2.3	Test Requirements	2983

A.9.10	SSTD	2983

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2983

A.9.10.1.1	Test Purpose and Environment	2983

A.9.10.1.2	Test parameters	2983

A.9.10.1.3	Test Requirements	2985

A.9.10.2	Void	2985

A.9.10.3	Void	2985

A.9.10.4	Void	2985

A.9.11 RSSI	2985

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2985

A.9.11.1.1	Test Purpose and Environment	2985

A.9.11.1.2	Test parameters	2985

A.9.11.1.3	Test Requirements	2988

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2988

A.9.11.2.1	Test Purpose and Environment	2988

A.9.11.2.2	Test parameters	2988

A.9.12	Channel occupancy	2991

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2991

A.9.12.1.1	Test Purpose and Environment	2991

A.9.12.1.2	Test parameters	2991

A.9.12.1.3	Test Requirements	2994

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2994

A.9.12.2.1	Test Purpose and Environment	2994

A.9.12.2.2	Test parameters	2994

A.9.12.2.3	Test Requirements	2997

A.9.13	RS-SINR	2997

A.9.13.1	FDD Intra-Frequency Case	2997

A.9.13.1.1	Test Purpose and Environment	2997

A.9.13.1.2	Test parameters	2997

A.9.13.1.3	Test Requirements	3000

A.9.13.2	TDD Intra-Frequency Case	3000

A.9.13.2.1	Test Purpose and Environment	3000

A.9.13.2.2	Test parameters	3000

A.9.13.2.3	Test Requirements	3003

A.9.13.3	FDD—FDD Inter frequency case	3003

A.9.13.3.1	Test Purpose and Environment	3003

A.9.13.3.2	Test parameters	3003

A.9.13.3.3	Test Requirements	3006

A.9.13.4	TDD—TDD Inter frequency case	3006

A.9.13.4.1	Test Purpose and Environment	3006

A.9.13.4.2	Test parameters	3006

A.9.13.4.3	Test Requirements	3010

A.9.13.5	FDD—TDD Inter frequency case	3011

A.9.13.5.1	Test Purpose and Environment	3011

A.9.13.5.2	Test parameters	3011

A.9.13.5.3	Test Requirements	3017

A.9.13.6	TDD—FDD Inter frequency case	3017

A.9.13.6.1	Test Purpose and Environment	3017

A.9.13.6.2	Test parameters	3017

A.9.13.6.3	Test Requirements	3021

A.9.14	Channel quality reporting accuracy	3021

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3021

A.9.14.1.1	Test Purpose and Environment	3021

A.9.14.1.2	Test parameters	3022

A.9.14.1.3	Test Requirements	3022

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3023

A.9.14.2.1	Test Purpose and Environment	3023

A.9.14.2.2	Test parameters	3023

A.9.14.2.3	Test Requirements	3024

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3024

A.9.14.3.1	Test Purpose and Environment	3024

A.9.14.3.2	Test parameters	3024

A.9.14.3.3	Test Requirements	3025

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3025

A.9.14.4.1	Test Purpose and Environment	3025

A.9.14.4.2	Test parameters	3025

A.9.14.4.3	Test Requirements	3026

A.10	Proximity-based Services in Any Cell Selection State	3049

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3049

A.10.1.1	Test Purpose and Environment	3049

A.10.1.2	Test Requirements	3050

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3051

A.10.2.1	Test Purpose and Environment	3051

A.10.2.2	Test Requirements	3052

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3052

A.10.3.1	Test Purpose and Environment	3052

A.10.3.2	Test Requirements	3054

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3055

A.10.4.1	Test Purpose and Environment	3055

A.10.4.2	Test Requirements	3057

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3057

A.11.1	V2V UE Transmission Timing Accuracy Test	3057

A.11.1.1	Test Purpose and Environment	3057

A.11.1.2	Test requirements	3058

A.11.2	Interruptions due to V2V sidelink communication	3058

A.11.2.1	Test Purpose and Environment	3058

A.11.2.2	Test Requirements	3060

A.12	3060

A.12.1	V2X UE Transmission Timing Accuracy Test	3060

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3060

A.12.1.1.1	Test Purpose and Environment	3060

A.12.1.1.2	Test requirements	3061

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3062

A.12.1.2.1	Test Purpose and Environment	3062

A.12.1.2.2	Test Requirements	3062

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3063

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3063

A.12.2.1.1	Test Purpose and Environment	3063

A.12.2.1.2	Test Requirements	3064

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3065

A.12.2.2.1	Test Purpose and Environment	3065

A.12.2.2.2	Test Requirements	3066

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3066

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3066

A.12.3.1.1	Test Purpose and Environment	3066

A.12.3.1.2	Test Requirements	3069

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3070

A.12.3.2.1	Test Purpose and Environment	3070

A.12.3.1.2	Test Requirements	3072

A.12.4	Congestion Control Measurement Test for V2X UE	3072

A.12.4.1	Test Purpose and Environment	3072

A.12.4.2	Test Requirements	3074

A.12.5	Interruptions due to V2X Sidelink Communication	3074

A.12.5.1	Test Purpose and Environment	3074

A.12.5.2	Test Requirements	3076

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3076

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3076

A.12.6.1.1	Test Purpose and Environment	3076

A.12.6.1.2	Test Requirements	3078

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3078

A.12.6.2.1	Test Purpose and Environment	3078

A.12.6.1.2	Test Requirements	3080

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3080

A.12.7.1	Test Purpose and Environment	3080

A.12.7.2	Test Requirements	3083

A.12.8	Interruptions due to V2X Carrier Aggregation	3084

A.12.8.1	Interruptions on a FDD PCell	3084

A.12.8.1.1	Test Purpose and Environment	3084

A.12.8.1.2	Test Requirements	3085

A.12.8.2	Interruptions on a TDD PCell	3085

A.12.8.2.1	Test Purpose and Environment	3085

A.12.8.2.2	Test Requirements	3087

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3088

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3088

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3088

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3088

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3088

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3090

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3091

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3091

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3091

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3093

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3094

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3094

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3094

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3095

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3095

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3096

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3096

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3096

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3097

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3097

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3098

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3098

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3098

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3098

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3099

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3099

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3099

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3101

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3101

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3102

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3103

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3104

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3105

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3106

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3106

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3106

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3106

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3107

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3107

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3109

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3109

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3109

B.3	Conditions for measurements performance requirements for UE	3109

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.2	Void	3110

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3110

B.3.5	Conditions for UE Rx – Tx time difference	3110

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3110

B.3.7	Conditions for inter-frequency RSTD measurements	3110

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3111

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3112

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3113

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3113

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3113

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3114

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3114

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.21.1	Conditions for RSRP measurements	3114

B.3.21.2	Conditions for RSRQ measurements	3115

B.3.21.3	Conditions for CSI-RSRP measurements	3115

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.22.1	Conditions for RSRP measurements	3115

B.3.22.2	Void	3115

B.3.22.3	Conditions for CSI-RSRP measurements	3115

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.23.1	Conditions for RSRP measurements	3116

B.3.23.2	Conditions for RSRQ measurements	3116

B.3.23.3	Conditions for CSI-RSRP measurements	3116

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.24.1	Conditions for RSRP measurements	3116

B.3.24.2	Conditions for RSRQ measurements	3116

B.3.24.3	Conditions for CSI-RSRP measurements	3116

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3117

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3117

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.4	RRM Requirements Exceptions	3119

B.4.1	General	3119

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3119

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3119

B.4.3.1	Inter-band carrier aggregation	3119

B.4.3.2	Intra-band non-contiguous carrier aggregation	3119

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3119

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3120

B.5.1	Conditions for S-RSRP Accuracy Requirements	3120

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3120

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3120

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3121

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3121

B.6	Conditions for V2X	3122

B.6.1	Test parameters for GNSS signals	3122

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3122

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3122

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3122

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3123

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3123

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3123

Annex C (informative): Change history:	3125