# A.9	Measurement Performance Requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in Clause 9 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in Clause 9 for at least 90% of the reported cases.

-	Cell 1 is the PCell.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

## A.9.1	RSRP

### A.9.1.1	FDD Intra frequency case

#### A.9.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.1 and 9.1.2.2 for FDD intra frequency measurements.

#### A.9.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.1.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.1.2-1: RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -106 | -106 | -88 | -88 | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -100 | -105 | -82 | -87 | -113 | -117 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -110 | -114 |
|  | Bands FDD_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -70.27 | -70.27 | -52.27 | -52.27 | -82.43 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -81.93 |  |
|  | Bands FDD_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -79.43 |  |
|  | Bands FDD_H |  |  |  |  |  | -78.93 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.1.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.2.1 and 9.1.2.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.2.7 and 9.1.2.8.

### A.9.1.2	TDD Intra frequency case

#### A.9.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.1 and 9.1.2.2 for TDD intra frequency measurements.

#### A.9.1.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.2.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.2.2-1: RSRP TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -106 | -106 | -88 | -88 | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -100 | -105 | -82 | -87 | -113 | -117 |
|  | Bands TDD_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -70.27 | -70.27 | -52.27 | -52.27 | -82.43 |  |
|  | Bands TDD_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD_E |  |  |  |  |  | -80.43 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.2.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.2.1 and 9.1.2.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.2.7 and 9.1.2.8.

### A.9.1.3	FDD—FDD Inter frequency case

#### A.9.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3.1 and 9.1.3.2 for FDD—FDD inter frequency measurements. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.2.7 and 9.1.2.8.

#### A.9.1.3.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.3.2-1 In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.1.3.2-1: RSRP FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -117 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -116.5 |
|  | Bands FDD_C |  |  |  |  | -116 |
|  | Bands FDD_D |  |  |  |  | -115.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -115 |
|  | Bands FDD_G Note 7 |  |  |  |  | -114 |
|  | Bands FDD_H |  |  |  |  | -113.5 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -121 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -120.5 |
|  | Bands FDD_C |  |  |  |  | -120 |
|  | Bands FDD_D |  |  |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -119 |
|  | Bands FDD_G Note 7 |  |  |  |  | -118 |
|  | Bands FDD_H |  |  |  |  | -117.5 |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -87.76 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -87.26 |
|  | Bands FDD_C |  |  |  |  | -86.76 |
|  | Bands FDD_D |  |  |  |  | -86.26 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -85.76 |
|  | Bands FDD_G Note 7 |  |  |  |  | -84.76 |
|  | Bands FDD_H |  |  |  |  | -84.26 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.3.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3.1 and 9.1.3.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.3.3 and 9.1.3.4.

### A.9.1.4	TDD—TDD Inter frequency case

#### A.9.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3.1 and 9.1.3.2 for TDD—TDD inter frequency measurements.

#### A.9.1.4.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.4.2-1 for TDD configuration 1 and in Table A.9.1.4.2-2 for TDD configuration 0. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.1.4.2-1: RSRP TDD—TDD Inter frequency test parameters for TDD configuration 1

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  | 1 |  |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -117 |
|  | Bands TDD_C |  |  |  |  | -116 |
|  | Bands TDD_E |  |  |  |  | -115 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -121 |
|  | Bands TDD_C |  |  |  |  | -120 |
|  | Bands TDD_E |  |  |  |  | -119 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -87.76 |
|  | Bands TDD_C |  |  |  |  | -86.76 |
|  | Bands TDD_E |  |  |  |  | -85.76 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table A.9.1.4.2-2: RSRP TDD—TDD Inter frequency test parameters for TDD configuration 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 0 |  | 0 |  |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.5 TDD | - | R.5 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -117 |
|  | Bands TDD_C |  |  |  |  | -116 |
|  | Bands TDD_E |  |  |  |  | -115 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -121 |
|  | Bands TDD_C |  |  |  |  | -120 |
|  | Bands TDD_E |  |  |  |  | -119 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -87.76 |
|  | Bands TDD_C |  |  |  |  | -86.76 |
|  | Bands TDD_E |  |  |  |  | -85.76 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.4.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3.1 and 9.1.3.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.3.3 and 9.1.3.4.

### A.9.1.5	FDD—TDD Inter frequency case

#### A.9.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3.1 and 9.1.3.2 for FDD—TDD inter frequency measurements.

#### A.9.1.5.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.5.2-1 and Table A.9.1.5.2-2.  In all test cases, Cell 1 is the serving cell and Cell 2 the target cell. Cell 1 is FDD cell and Cell 2 is TDD cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.1.5.2-1: RSRP FDD—TDD Inter frequency test parameters (FDD Cell1)

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Gap Pattern Id |  | 0 |  | 0 |  |
| Measurement bandwidth |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD |  | R.0 FDD |  |
| PDSCH allocation |  | 13—36 |  | 13—36 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| Note2 | dBm/15 kHz | -88.65 |  | -104 |  |
|  | dB | 10 |  | 13 |  |
| RSRPNote3 | dBm/15 kHz | -78.65 |  | -91 |  |
| IoNote3 | dBm/9 MHz | -50.45 |  | -63.01 |  |
|  | dB | 10 |  | 13 |  |
| Propagation condition | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table A.9.1.5.2-2: RSRP FDD—TDD Inter frequency test parameters (TDD cell2)

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  | 2 | 2 |
| BWchannel | MHz | 10 | 10 |
| Special subframe configurationNote1 |  | 6 | 6 |
| Uplink-downlink configurationNote1 |  | 1 | 1 |
| Gap Pattern Id |  | - | - |
| Measurement bandwidth |  | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | - | - |
| PDSCH allocation |  | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote2 |  |  |  |
| OCNG_RBNote2 |  |  |  |
| Note3 | dBm/15 kHz | -88.65 | -112 |
|  | dB | 10 | -4 |
| RSRPNote4 | dBm/15 kHz | -78.65 | -116 |
| IoNote4 | dBm/9 MHz | -50.45 | -82.76 |
|  | dB | 10 | -4 |
| Propagation condition | - | AWGN | AWGN |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

#### A.9.1.5.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3.1 and 9.1.3.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.3.3 and 9.1.3.4.

### A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation

#### A.9.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2. The test will also verify the primary and secondary component carrier relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.6.2	Test parameters

In this set of cases cell1 is PCell on the primary component carrier, cell2 is SCell on the secondary component carrier and activated, and cell3 is the neighboring cell on the secondary component carrier.  The test parameters are given in Table A.9.1.6.2-1.

Table A.9.1.6.2-1: RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -87.26 |  |  |
|  | Bands FDD_C |  | -86.76 |  |  |
|  | Bands FDD_D |  | -86.26 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 |  |  |
|  | Bands FDD_G |  | -84.76 |  |  |
|  | Bands FDD_H |  | -84.26 |  |  |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.6.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation

The test case in this clause are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the absolute RSRP accuracy on PCell defined in clause 9.1.11.1, the absolute RSRP accuracy on Scell defined in clause 9.1.11.2, the relative RSRP accuracy between SCell and Cell 3 defined in clause 9.1.11.2, and the relative RSRP accuracy between PCell and SCell defined in clause 9.1.11.3.

#### A.9.1.7.2	Test parameters

In this set of test cases there are three cells on two carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, and Cell 3 is neighbour cell which is also on channel 2. The parameters for the test are listed in Table A.9.1.7.2-1.

Table A.9.1.7.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 |  |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  |  | dB | -4 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands TDD_C |  | -86.76 |  |  |
|  | Bands TDD_E |  | -85.76 |  |  |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### A.9.1.7.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3

### A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.3 and 9.1.2.4 for FDD intra-frequency RSRP measurements under time-domain measurement resource restriction with non-MBSFN ABS configured in the aggressor cell.

#### A.9.1.8.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.8.2-1 and A.9.1.8.2-2.

In the tests there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 2. Cell 2 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 2 are measured for RSRP relative accuracy. Non-MBSFN ABS pattern is configured for Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.1.8.2-1: General test parameters for E-UTRAN FDD RSRP intra frequency test parameters under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The aggressor cell to Cell 2 |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met |
| ABS pattern |  | ‘1000000010000000100000001000000010000000’ | Non-MBSFN ABS. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes in Cell 1. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘1000000010000000100000001000000010000000’ | Configured for Cell 2 measurements by measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘0100000001000000010000000100000001000000’ | Configured for measurements on Cell 1. |

Table A.9.1.8.2-2: Cell-specific test parameters for E-UTRAN FDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and A.3.2.1.6 (OP.6 FDD) |  |  | OP.5 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note2 | Bands FDD_A Note 10 | dBm/15 kHz | -106 |  | -88 |  | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
| CRS |  | dB | 5 | -2 | 5 | -4 | 5 | -4 |
| CRS Note 5 |  | dB | 2.88 | -2 | 3.54 | -4 | 3.54 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -0.46 | -7.54 | -0.46 | -7.54 |
| RSRP Note3,4,5 | Bands FDD_A Note 10 | dBm/15 kHz | -101 | -108 | -83 | -92 | -111 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -110.5 | -119.5 |
|  | Bands FDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands FDD_D |  |  |  |  |  | -109.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -109 | -118 |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -108 | -117 |
|  | Bands FDD_H |  |  |  |  |  | -107.5 | -116.5 |
| Note 3 | Bands FDD_A Note 10 | dBm/9 MHz | -71.41 | -74.88 | -53.63 | -57.37 | -81.63 | -85.37 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -81.13 | -84.87 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -83.87 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -79.63 | -83.37 |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -78.63 | -82.37 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -81.87 |
| Propagation condition |  |  | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: Except Band 29. Note 10: Except Band 32, Band 75, Band 76. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.8.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.3 and 9.1.2.4, respectively.

### A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.1.9.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.3 and 9.1.2.4 for TDD intra-frequency RSRP measurements under time-domain measurement resource restriction with non-MBSFN ABS configured in the aggressor cell.

#### A.9.1.9.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.9.2-1 and A.9.1.9.2-2.

In the tests there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 2. Cell 2 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 2 are measured for RSRP relative accuracy. Non-MBSFN ABS pattern is configured for Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.1.9.2-1: General test parameters for E-UTRAN TDD RSRP intra frequency test parameters under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | Also the aggressor cell. |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| Special subframe configuration |  | 6 | For Cell 1 and Cell 2. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  | 1 | For Cell 1 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are randomly selected so that the condition is met |
| ABS pattern |  | ‘00000000010000000001’ | Non-MBSFN ABS. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are cofigured in the ABS subframes in Cell 1. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘00000000010000000001’ | Configured for Cell 2 measurements by measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘10000000001000000000’ | Configured for Cell 1 measurements. |

Table A.9.1.9.2-2: Cell-specific test parameters for E-UTRAN TDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note 2 | Bands TDD_A | dBm/15 kHz | -106 |  | -88 |  | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
| CRS |  | dB | 5 | -2 | 5 | -4 | 5 | -4 |
| CRS Note 5 |  | dB | 2.88 | -2 | 3.54 | -4 | 5 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -0.46 | -7.54 | -0.46 | -7.54 |
| RSRP Note3,4,5 | Bands TDD_A | dBm/15 kHz | -101 | -108 | -83 | -92 | -111 | -120 |
|  | Bands TDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands TDD_E |  |  |  |  |  | -109 | -118 |
| Note 3 | Bands TDD_A | dBm/9 MHz | -71.41 | -74.88 | -53.63 | -57.37 | -81.6 | -85.4 |
|  | Bands TDD_C |  |  |  |  |  | -80.6 | -84.4 |
|  | Bands TDD_E |  |  |  |  |  | -79.6 | -83.4 |
| Propagation condition |  |  | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.9.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.3 and 9.1.2.4, respectively.

### A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS

#### A.9.1.10.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.3 and 9.1.2.4 for FDD intra-frequency RSRP measurements under time-domain measurement resource restriction with MBSFN ABS configured in the aggressor cell.

#### A.9.1.10.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.10.2-1 and A.9.1.10.2-2.

In the tests there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 2. Cell 2 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 2 are measured for RSRP relative accuracy. MBSFN ABS pattern is configured in Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.1.10.2-1: General test parameters for E-UTRAN FDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The aggressor cell to Cell 2 |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | MBSFN ABS | As defined in Table A.3.4.2.1-1 |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 =0, PCIcell1 not equal to PCIcell2 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met |
| ABS pattern |  | ‘0100000010000000100000000010000001000000’ | MBSFN ABS pattern. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. All ABS subframes are MBSFN subframes. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘0100000010000000100000000010000001000000’ | Configured for Cell 2 measurements by measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘0001000000010000000100000001000000010000’ | Configured for measurements on Cell 1. |

Table A.9.1.10.2-2: Cell-specific test parameters for E-UTRAN FDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.8 (OP.8 FDD) and A.3.2.1.6 (OP.6 FDD) |  |  | OP.8 FDD | OP.6 FDD | OP.8 FDD | OP.6 FDD | OP.8 FDD | OP.6 FDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RB Note1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note 2 | Bands FDD_A Note 11 | dBm/15 kHz | -106 |  | -88 |  | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
| CRS |  | dB | 5 | -2 | 5 | -4 | 5 | -4 |
| CRS Note 5, 7 in the 1st OFDM symbol |  | dB | 2.88 | -8.19 | 3.54 | -10.19 | 3.54 | -10.19 |
| CRS Note 5 in OFDM symbols 4,7,11 |  | dB | 2.88 | -2 | 3.54 | -4 | 3.54 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -0.46 | -7.54 | -0.46 | -7.54 |
| RSRP Note 3,4 | Bands FDD_A Note 11 | dBm/15 kHz | -101 | -108 | -83 | -92 | -111 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -110.5 | -119.5 |
|  | Bands FDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands FDD_D |  |  |  |  |  | -109.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -109 | -118 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -108 | -117 |
|  | Bands FDD_H |  |  |  |  |  | -107.5 | -116.5 |
| Note 3 in the 1st OFDM symbol | Bands FDD_A Note 11 | dBm/9 MHz | -71.41 | -74.88 | -53.63 | -57.37 | -81.63 | -85.37 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -81.13 | -84.87 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -83.87 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -79.63 | -83.37 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -78.63 | -82.37 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -81.87 |
| Note 3 in OFDM symbols other than the 1st one | Bands FDD_A Note 11 | dBm/9 MHz | -71.41 | -76.09 | -53.63 | -58.76 | -81.63 | -86.76 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -81.13 | -86.26 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -85.76 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -85.26 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -79.63 | -84.76 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -78.63 | -83.76 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -83. 26 |
| Propagation condition |  |  | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.1-1. Note 7: In the 1st OFDM symbol, Cell 2 is not expected to meet the Es/Iot side condition in 9.1.2.3 and 9.1.2.4. Note 8: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Except Band 29. Note 11: Except Band 32, Band 75, Band 76. Note 12: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.10.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.3 and 9.1.2.4, respectively.

### A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS

#### A.9.1.11.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.3 and 9.1.2.4 for TDD intra-frequency RSRP measurements under time-domain measurement resource restriction with MBSFN ABS configured in the aggressor cell.

#### A.9.1.11.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.11.2-1 and A.9.1.11.2-2.

In the tests there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 2. Cell 2 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 2 are measured for RSRP relative accuracy. MBSFN ABS pattern is configured in Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.1.11.2-1: General test parameters for E-UTRAN TDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The aggressor cell to Cell 2 |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | MBSFN ABS | As defined in Table A.3.4.2.1-1 |
| Special subframe configuration |  | 6 | For Cell 1 and Cell 2. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  | 1 | For Cell 1 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 =0, PCIcell1 not equal to PCIcell2 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met |
| ABS pattern |  | ‘00001000000000100000’ | MBSFN ABS pattern. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. All ABS subframes are MBSFN subframes. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘00001000000000100000’ | Configured for Cell 2 measurements by measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘10000000001000000000’ | Configured for measurements on Cell 1. |

Table A.9.1.11.2-2: Cell-specific test parameters for E-UTRAN TDD RSRP intra-frequency test parameters under time-domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.5 (OP.5 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.5 TDD | OP.2 TDD | OP.5 TDD | OP.2 TDD | OP.5 TDD | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RB Note1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note 2 | Bands TDD_A | dBm/15 kHz | -106 |  | -88 |  | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
| CRS |  | dB | 5 | -2 | 5 | -4 | 5 | -4 |
| CRS note 5, note 7 in the 1st OFDM symbol |  | dB | 2.88 | -8.19 | 3.54 | -10.19 | 3.54 | -10.19 |
| CRS note 5 in OFDM symbols 4,7,11 |  | dB | 2.88 | -2 | 3.54 | -4 | 3.54 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -0.46 | -7.54 | -0.46 | -7.54 |
| RSRP Note 3,4 | Bands TDD_A | dBm/15 kHz | -101 | -108 | -83 | -92 | -111 | -120 |
|  | Bands TDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands TDD_E |  |  |  |  |  | -109 | -118 |
| Note 3 in the 1st OFDM symbol | Bands TDD_A | dBm/9 MHz | -71.41 | -74.88 | -53.63 | -57.37 | -81.63 | -85.37 |
|  | Bands TDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands TDD_E |  |  |  |  |  | -79.63 | -83.37 |
| Note 3 in OFDM symbols other than the 1st one | Bands TDD_A | dBm/9 MHz | -71.41 | -76.09 | -53.63 | -58.76 | -81.63 | -86.76 |
|  | Bands TDD_C |  |  |  |  |  | -80.63 | -85.76 |
|  | Bands TDD_E |  |  |  |  |  | -79.63 | -84.76 |
| Propagation condition |  |  | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.1-1. Note 7: In the 1st OFDM symbol, Cell 2 is not expected to meet the Es/Iot side condition in 9.1.2.3 and 9.1.2.4. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.11.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.3 and 9.1.2.4, respectively.

### A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.12.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.6.1.

#### A.9.1.12.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.12.2-1 will replace the values of corresponding parameters in Tables A.9.1.6.2-1.

Table A.9.1.12.2-1: RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| BWchannel Note 1 |  | MHz | 20 | 20 | 20 |
| Measurement bandwidth |  |  | 47—52 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.4 FDD | R.4 FDD | N/A |
| PDSCH allocation |  |  | 38—61 | 38—61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.10 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.11 (OP.11 FDD) and A.3.2.1.12 (OP.12 FDD) |  |  | OP.11 FDD | OP.11 FDD | OP.12 FDD |
| IoNote2 | Bands FDD_A Note 5 | dBm/18 MHz | -84.75 | (Io for Channel 1 +5.33dB) |  |
|  | Bands FDD_B1 Note 5, FDD_B2 Note 5, Note 6 |  | -84.25 |  |  |
|  | Bands FDD_C Note 5 |  | -83.75 |  |  |
|  | Bands FDD_D Note 5 |  | -83.25 |  |  |
|  | Bands FDD_E Note 5 |  | -82.75 |  |  |
|  | Bands FDD_G Note 5 |  | -81.75 |  |  |
|  | Bands FDD_H Note 5 |  | -81.25 |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.6.2-1 for the other parameters. Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 20 MHz channel bandwidth. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.12.3	Test Requirements

The test requirements defined in section A.9.1.6.3 shall apply to this test case.

### A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.13.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.7.1.

#### A.9.1.13.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.13.2-1 will replace the values of corresponding parameters in Tables A.9.1.7.2-1.

Table A.9.1.13.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| BWchannel Note 1 |  | MHz | 20 |  |  |
| Measurement bandwidth |  |  | 47—52 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.3 TDD | R.3 TDD | N/A |
| PDSCH allocation |  |  | 38—61 | 38—61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.10 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and A.3.2.2.8 (OP.8 TDD) |  |  | OP.7 TDD | OP.7 TDD | OP.8 TDD |
| IoNote2 | Bands TDD_A Note 5 | dBm/18 MHz | -84.75 | (Io for Channel 1 +5.33dB) |  |
|  | Bands TDD_C Note 5 |  | -83.75 |  |  |
|  | Bands TDD_E Note 5 |  | -82.75 |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.7.2-1 for the other parameters. Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 20 MHz channel bandwidth. |  |  |  |  |  |

#### A.9.1.13.3	Test Requirements

The test requirements defined in section A.9.1.7.3 shall apply to this test case.

### A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.1.14.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.5 and 9.1.2.6 for FDD intra-frequency RSRP measurements under time-domain measurement resource restriction with CRS Assistance Information and non-MBSFN ABS configured in the aggressor cells.

#### A.9.1.14.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.14.2-1 and A.9.1.14.2-2.

In the tests there are three synchronous cells, Cell 1, Cell2 and Cell 3, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 3. Cell 2 is the neighbour aggressor cell without CRS colliding to Cell 3. Cell 3 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 3 are measured for RSRP relative accuracy. Non-MBSFN ABS pattern is configured for Cell 1 and Cell 2 during the test.

The UE is configured by higher layers with a time domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells, namely Cell 3 measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement pattern and the CRS assistance information shall be provided via RRC to the UE before the measurements start.

Note:	It’s up to eNB’s implementation whether the time domain measurement resource restriction pattern for PCell measurements is configured or not.

Table A.9.1.14.2-1: General test parameters for FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell (PCell) |  |  | Cell 1 | The aggressor cell to Cell 3 |
| Neighbour cell |  |  | Cell 2 | The aggressor cell to Cell 3 |
| Neighbour cell |  |  | Cell 3 | Cell to be measured |
| PCell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  |  | Normal | For three cells in the test |
| DRX |  |  |  | OFF |
| Cell 2 time offset with respect to Cell 1 |  |  | 0s | Synchronous cells |
| Cell 3 time offset with respect to Cell 1 |  |  | -2.5 s | Synchronous cells |
| Physical cell ID PCI |  |  | Colliding CRS: (PCIcell1 – PCIcell3)mod6=0, PCIcell1 not equal to PCIcell3 Non-colliding CRS: (PCIcell2 – PCIcell3)mod6 !=0 | Cell PCIs for three cells are selected randomly so that all conditions are met |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | Non-MBSFN ABS. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the Pcell subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are configured in the ABS subframes in Cell 1. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Configured before the measurements start. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  |  | ‘0100000001000000010000000100000001000000’ | Configured for measurements on Cell 1. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.1.14.2-2: Cell-specific test parameters for FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel |  | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  | 22—27 |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | - | R.0 FDD | - | - | R.0 FDD | - | - |
| PDSCH allocation |  |  | 13—36 | - | - | 13—36 | - | - | 13—36 | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  | R.6 FDD |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and A.3.2.1.6 (OP.6 FDD) |  |  | OP.5 FDD | OP.6 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD | OP.6 FDD |
| PBCH_RA |  | dB | Note 6 | Note 6 | 0 | Note 6 | Note 6 | 0 | Note 6 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 10 | dBm/15 kHz | -106 |  |  | -88 |  |  | -116 |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | -115.5 |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  | -115 |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  | -114.5 |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -114 |  |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -113 |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  | -112.5 |  |  |
| CRS |  | dB | 4 | 2 | -1.5 | 4 | 2 | -4 | 4 | 2 | -4 |
| CRS Note 5 |  | dB | -1.18 | -0.32 | -6.96 | -0.75 | 0.54 | -9.46 | -0.75 | 0.54 | -9.46 |
| RSRP Note3,4,5 | Bands FDD_A Note 10 | dBm/15 kHz | -102 | -104 | -107.5 | -84 | -86 | -92 | -112 | -114 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | -111.5 | -113.5 | 119.5 |
|  | Bands FDD_C |  |  |  |  |  |  |  | -111 | -113 | -119 |
|  | Bands FDD_D |  |  |  |  |  |  |  | -110.5 | -112.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -110 | -112 | -118 |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -109 | -111 | -117 |
|  | Bands FDD_H |  |  |  |  |  |  |  | -108.5 | -110.5 | -116.5 |
| Note 3,5 | Bands FDD_A Note 10 | dBm/9 MHz | -70.58 | -74.43 |  | -52.82 | -57.04 |  | -80.82 | -85.04 |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | -80.32 | -84.54 |  |
|  | Bands FDD_C |  |  |  |  |  |  |  | -79.82 | -84.04 |  |
|  | Bands FDD_D |  |  |  |  |  |  |  | -79.32 | -83.54 |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -78.82 | -83.04 |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -77.82 | -82.04 |  |
|  | Bands FDD_H |  |  |  |  |  |  |  | -77.32 | -81.54 |  |
| Propagation condition |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes for only Cell 2 and Cell 3. For Cell 1, the corresponding value is derived from the normal subframes other than the subframes indicated in the time domain measurement resource restriction pattern for intra-frequency measurements. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: Except Band 29. Note 10: Except Band 32, Band 75, Band 76. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.1.14.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.5 and 9.1.2.6, respectively.

### A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.1.15.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.5 and 9.1.2.6 for TDD intra-frequency RSRP measurements under time-domain measurement resource restriction with CRS Assistance Information and non-MBSFN ABS configured in the aggressor cells.

#### A.9.1.15.2	Test parameters

In this set of test cases all cells are on the same carrier frequency as PCell. Both absolute and relative accuracy of RSRP intra-frequency measurements are tested, with test parameters specified in Tables A.9.1.15.2-1 and A.9.1.15.2-2.

In the tests there are three synchronous cells, Cell 1, Cell2 and Cell 3, on the same RF channel. In all test cases, Cell 1 is the serving cell (PCell) and also the aggressor cell to Cell 3. Cell 2 is the neighbour aggressor cell without CRS colliding to Cell 3. Cell 3 is the cell to be measured for RSRP absolute accuracy, whilst both Cell 1 and Cell 3 are measured for RSRP relative accuracy. Non-MBSFN ABS pattern is configured for Cell 1 and Cell 2 during the test.

The UE is configured by higher layers with a time domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells, namely Cell 3 measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement pattern and the CRS assistance information shall be provided via RRC to the UE before the measurements start.

Note:	It’s up to eNB’s implementation whether the time domain measurement resource restriction pattern for PCell measurements is configured or not.

Table A.9.1.15.2-1: General test parameters for TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell (PCell) |  |  | Cell 1 | The aggressor cell to Cell 3 |
| Neighbour cell |  |  | Cell 2 | The aggressor cell to Cell 3 |
| Neighbour cell |  |  | Cell 3 | Cell to be measured |
| PCell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  |  | Normal | For three cells in the test |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| DRX |  |  |  | OFF |
| Cell 2 time offset with respect to Cell 1 |  |  | 0s | Synchronous cells |
| Cell 3 time offset with respect to Cell 1 |  |  | -2.5 s | Synchronous cells |
| Physical cell ID PCI |  |  | Colliding CRS: (PCIcell1 – PCIcell3)mod6=0, PCIcell1 not equal to PCIcell3 Non-colliding CRS: (PCIcell2 – PCIcell3)mod6 !=0 | Cell PCIs for three cells are selected randomly so that all conditions are met |
| ABS pattern |  |  | ‘00000000010000000001’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. Provided to the UE for Cell 1 and Cell 2 before the measurements start. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘00000000010000000001’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Configured before the measurements start. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  |  | ‘10000000001000000000’ | Configured for Cell 1 measurements. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.1.15.2-2: Cell-specific test parameters for TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel |  | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  | 22—27 |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | - | R.0 TDD | - | - | R.0 TDD | - | - |
| PDSCH allocation |  |  | 13—36 | - | - | 13—36 | - | - | 13—36 | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  | R.6 TDD |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2 |  |  | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 |  | 0 | Note 6 |  | 0 | Note 6 |  | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands TDD_A | dBm/15 kHz | -106 |  |  | -88 |  |  | -116 |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |  | -115 |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  | -114 |  |  |
| CRS |  | dB | 4 | 2 | -1.5 | 4 | 2 | -4 | 4 | 2 | -4 |
| CRS Note 5 |  | dB | -1.18 | -0.32 | -6.96 | -0.75 | 0.54 | -9.46 | -0.75 | 0.54 | -9.46 |
| RSRP Note3,4,5 | Bands TDD_A | dBm/15 kHz | -102 | -104 | -107.5 | -84 | -86 | -92 | -112 | -114 | -120 |
|  | Bands TDD_C |  |  |  |  |  |  |  | -111 | -113 | -119 |
|  | Bands TDD_E |  |  |  |  |  |  |  | -110 | -112 | -118 |
| Note 3, 5 | Bands TDD_A | dBm/9 MHz | -70.58 | -74.43 |  | -52.82 | -57.04 |  | -80.82 | -85.04 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  | -79.82 | -84.04 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  | -78.82 | -83.04 |  |
| Propagation condition |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes for only Cell 2 and Cell 3. For Cell 1, the corresponding value is derived from the normal subframes other than the subframes indicated in the time domain measurement resource restriction pattern for intra-frequency measurements. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.1.15.3	Test Requirements

The absolute RSRP measurement accuracy and relative RSRP measurement accuracy shall fulfill the requirements in Sections 9.1.2.5 and 9.1.2.6, respectively.

### A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth

#### A.9.1.16.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.1 and 9.1.2.2 for FDD intra frequency measurements.

#### A.9.1.16.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.16.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.16.2-1: RSRP FDD Intra frequency test parameters for 5MHz Bandwidth

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1-1 |  |  | R.5 FDD | - | R.5 FDD | - | R.5 FDD | - |
| PDSCH allocation |  |  | 7—17 | - | 7-17 | - | 7-17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1-1 |  |  | R.11 FDD |  | R.11 FDD |  | R.11 FDD |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_N | dBm/15 kHz | -103 |  | -83 |  | -109.5 |  |
|  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_N | dBm/15 kHz | -97 | -102 | -77 | -82 | -106.5 | -110.5 |
| IoNote3 | Bands FDD_N | dBm/4.5 MHz | -70.28 |  | -50.28 |  | -78.94 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

#### A.9.1.16.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.2.1 and 9.1.2.2.

### A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth

#### A.9.1.17.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3.1 and 9.1.3.2 for FDD—FDD inter frequency measurements.

#### A.9.1.17.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.17.2-1 In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.1.17.2-1: RSRP FDD—FDD Inter frequency test parameters for 5MHz Bandwidth

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 5 | 5 | 5 | 5 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.5 FDD | - | R.5 FDD | - |
| PDSCH allocation |  |  | 7—17 | - | 7-17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.11 FDD |  | R.11 FDD |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note1 |  |  |  |  |  |  |
| OCNG_RB Note1 |  |  |  |  |  |  |
| Note2 | Cell 2: Bands FDD_N | dBm/15 kHz | -85.65 | -85.65 | -102.5 | -110.5 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | Cell 2: Bands FDD_N | dBm/15 kHz | -75.65 | -75.65 | -89.5 | -114.5 |
| IoNote3 | Cell 2: Bands FDD_N | dBm/4.5 MHz | -50.46 | -50.46 | -64.52 | -84.27 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: This test is only applicable for testing inter-frequency requirements for Bands FDD_N. Cell 2 is on the Band under test, and Cell 1 is on another band supported by the UE. |  |  |  |  |  |  |

Table A.9.1.17.2-1: Void

#### A.9.1.17.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3.1 and 9.1.3.2.

### A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.18.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.6.1.

#### A.9.1.18.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.18.2-1 will replace the values of corresponding parameters in Tables A.9.1.6.2-1.

Table A.9.1.18.2-1: RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| BWchannel Note 1 |  | MHz | 10 | 5 |  |  |
| Measurement bandwidth |  |  | 22-27 | 10-15 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.5 FDD |  | N/A |
| PDSCH allocation |  |  | 13-36 | 7-17 |  | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | R.11 FDD |  |  |
| OCNG Patterns defined in A.3.2.1 (FDD) |  |  | OP.1 FDD | OP.15 FDD | OP.16 FDD |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -116.5 |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -115 |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |
|  | Bands FDD_N |  | N/A |  |  |  |
| RSRPNote2 | Bands FDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) |  | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -120.5 |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -119 |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |
|  | Bands FDD_N |  | N/A |  |  |  |
| IoNote2 | Bands FDD_A | dBm/9 MHz | -87.76 | N/A |  |  |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -87.26 |  |  |  |
|  | Bands FDD_C |  | -86.76 |  |  |  |
|  | Bands FDD_D |  | -86.26 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -85.76 |  |  |  |
|  | Bands FDD_G |  | -84.76 |  |  |  |
|  | Bands FDD_H |  | -84.26 |  |  |  |
|  | Bands FDD_A | dBm/4.5 MHz | N/A | (Io for Channel 1 +2.32dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 4 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
|  | Bands FDD_N |  |  | -80.94 |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.6.2-1 for the other parameters. Note 4: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.18.3	Test Requirements

The test requirements defined in section A.9.1.6.3 shall apply to this test case.

### A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.19.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.7.1.

#### A.9.1.19.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.19.2-1 will replace the values of corresponding parameters in Tables A.9.1.7.2-1.

Table A.9.1.19.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| BWchannel Note 1 |  | MHz | 10 | 5 |  |
| Measurement bandwidth |  |  | 22-27 | 10-15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.4 TDD | N/A |
| PDSCH allocation |  |  | 13-36 | 7-17 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.11 TDD |  |
| OCNG Patterns defined in A.3.2.2 (TDD) |  |  | OP.1 TDD | OP.9 TDD | OP.10 TDD |
| IoNote2 | Bands TDD_A | dBm/9 MHz | -87.76 | N/A |  |
|  | Bands TDD_C |  | -86.76 |  |  |
|  | Bands TDD_E |  | -85.76 |  |  |
|  | Bands TDD_A | dBm/4.5MHz | N/A | (Io for Channel 1 +2.32dB) |  |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.7.2-1 for the other parameters. |  |  |  |  |  |

#### A.9.1.19.3	Test Requirements

The test requirements defined in section A.9.1.7.3 shall apply to this test case.

### A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.20.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.6.1.

#### A.9.1.20.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.20.2-1 will replace the values of corresponding parameters in Tables A.9.1.6.2-1.

Table A.9.1.20.2-1: RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| BWchannel Note 1 |  | MHz | 5 | 5 | 5 |  |
| Measurement bandwidth |  |  | 10-15 | 10-15 | 10-15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.5 FDD | R.5 FDD | N/A |  |
| PDSCH allocation |  |  | 7-17 | 7-17 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.11 FDD | R.11 FDD | R.11 FDD |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.26 (OP.16 FDD) |  |  | OP.15 FDD | OP.15 FDD | OP.16 FDD |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 6 |  | -116.5 |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -115 |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |
|  | Bands FDD_N |  | -110.5 |  |  |  |
| RSRPNote2 | Bands FDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) |  | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 6 |  | -120.5 |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -119 |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |
|  | Bands FDD_N |  | -114.5 |  |  |  |
| IoNote2 | Bands FDD_A Note 5 | dBm/4.5 MHz | -90.76 | (Io for Channel 1 +5.33dB) |  |  |
|  | Bands FDD_B1 Note 5, FDD_B2 Note 5, Note 6 |  | -90.26 |  |  |  |
|  | Bands FDD_C Note 5 |  | -89.76 |  |  |  |
|  | Bands FDD_D Note 5 |  | -89.26 |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -88.76 |  |  |  |
|  | Bands FDD_G Note 5 |  | -87.76 |  |  |  |
|  | Bands FDD_H Note 5 |  | -87.26 |  |  |  |
|  | Bands FDD_N Note 5 |  | -84.26 |  |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.6.2-1 for the other parameters. Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 5MHz + 5MHz channel bandwidth. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.20.3	Test Requirements

The test requirements defined in section A.9.1.6.3 shall apply to this test case.

### A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.21.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.7.1.

#### A.9.1.21.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.21.2-1 will replace the values of corresponding parameters in Tables A.9.1.7.2-1.

Table A.9.1.21.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| BWchannel Note 1 |  | MHz | 5 | 5 | 5 |
| Measurement bandwidth |  |  | 10-15 | 10-15 | 10-15 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.4 TDD | R.4 TDD | N/A |
| PDSCH allocation |  |  | 7-17 | 7-17 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.11 TDD |  |  |
| OCNG Patterns defined in A.3.1.2.9 (OP.9 TDD) and A.3.2.2.10 (OP.10 TDD) |  |  | OP.9 TDD | OP.9 TDD | OP.10 TDD |
| IoNote2 | Bands TDD_A Note 5 | dBm/4.5MHz | -90.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands TDD_C Note 5 |  | -89.76 |  |  |
|  | Bands TDD_E Note 5 |  | -88.76 |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.7.2-1 for the other parameters. Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 5MHz + 5MHz channel bandwidth. |  |  |  |  |  |

#### A.9.1.21.3	Test Requirements

The test requirements defined in section A.9.1.7.3 shall apply to this test case.

### A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD

#### A.9.1.22.1	Test Purpose and Environment

The test case is applicable for TDD-FDD carrier aggregation capable UEs which have been configured with a downlink PCell in FDD and a downlink SCell in TDD.

The purpose of this test is to verify that the RSRP absolute and relative measurements accuracy in TDD-FDD carrier aggregation is within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2, the relative RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2 between the SCell and a neighbour cell, and the relative RSRP accuracy requirements of the PCell compared to the SCell defined in Clause 9.1.11.3.

#### A.9.1.22.2	Test parameters

In this test case, Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured and activated SCell on the TDD secondary component carrier, and Cell 3 is the neighboring cell on the TDD secondary component carrier. The test parameters are given in Table A.9.1.22.2-1.

Table A.9.1.22.2-1: RSRP TDD-FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote9 |  |  | - | 6 | 6 |  |
| Uplink-downlink configurationNote9 |  |  | - | 1 | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 and A.3.1.1.2 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD), A.3.2.2.1 (OP.1 TDD), and A.3.2.2.2 (OP.2 TDD) |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 2 |  |  |  |  |  |  |
| OCNG_RB Note 2 |  |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | - |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 | - |  |  |
|  | Bands FDD_C |  | -116 | - |  |  |
|  | Bands FDD_D |  | -115.5 | - |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 | - |  |  |
|  | Bands FDD_G |  | -114 | - |  |  |
|  | Bands FDD_H |  | -113.5 | - |  |  |
|  | Bands TDD_A |  | - | ( for Channel 1 +1dB) |  |  |
|  | Bands TDD_C |  | - |  |  |  |
|  | Bands TDD_E |  | - |  |  |  |
|  |  | dB | -4 | 3 | -1 |  |
|  |  | dB | -4 | 0.46 | -5.76 |  |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | - | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 | - | - |  |
|  | Bands FDD_C |  | -120 | - | - |  |
|  | Bands FDD_D |  | -119.5 | - | - |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 | - | - |  |
|  | Bands FDD_G |  | -118 | - | - |  |
|  | Bands FDD_H |  | -117.5 | - | - |  |
|  | Bands TDD_A |  | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |  |
|  | Bands TDD_C |  | - |  |  |  |
|  | Bands TDD_E |  | - |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76 + 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26 + 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_C |  | -86.76+ 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_D |  | -86.26+ 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76+ 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_G |  | -84.76+ 10log(NRB,c/50) | - |  |  |
|  | Bands FDD_H |  | -84.26+ 10log(NRB,c/50) | - |  |  |
|  | Bands TDD_A |  | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |  |
|  | Bands TDD_C |  | - |  |  |  |
|  | Bands TDD_E |  | - |  |  |  |
| Propagation condition |  | - | AWGN | AWGN |  | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 |  | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 |  | 3 |
| Time alignment error relative to cell 1 Note 8 |  | - | - | TAE |  | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.22.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD

#### A.9.1.23.1	Test Purpose and Environment

The test case is applicable for TDD-FDD carrier aggregation capable UEs which have been configured with a downlink PCell in TDD and a downlink SCell in FDD.

The purpose of this test is to verify that the RSRP absolute and relative measurements accuracy in TDD-FDD carrier aggregation is within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2, the relative RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.11.2 between the SCell and a neighbour cell, and the relative RSRP accuracy requirements of the PCell compared to the SCell defined in Clause 9.1.11.3.

#### A.9.1.23.2	Test parameters

In this test case, Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured and activated SCell on the FDD secondary component carrier, and Cell 3 is the neighboring cell on the FDD secondary component carrier. The test parameters are given in Table A.9.1.23.2-1.

Table A.9.1.23.2-1: RSRP TDD-FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 | - | - |  |
| Uplink-downlink configurationNote1 |  |  | 1 | - | - |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 and A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD), A.3.2.2.1 (OP.1 TDD), and A.3.2.1.2 (OP.2 FDD) |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 2 |  |  |  |  |  |  |
| OCNG_RB Note 2 |  |  |  |  |  |  |
| Note 3 | Bands FDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | - |  |  |  |
|  | Bands FDD_C |  | - |  |  |  |
|  | Bands FDD_D |  | - |  |  |  |
|  | Bands FDD_E, FDD_F Note 9 |  | - |  |  |  |
|  | Bands FDD_G |  | - |  |  |  |
|  | Bands FDD_H |  | - |  |  |  |
|  | Bands TDD_A |  | -117 | - |  |  |
|  | Bands TDD_C |  | -116 | - |  |  |
|  | Bands TDD_E |  | -115 | - |  |  |
|  |  | dB | -4 | 3 | -1 |  |
|  |  | dB | -4 | 0.46 | -5.76 |  |
| RSRP Note 4 | Bands FDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | - |  |  |  |
|  | Bands FDD_C |  | - |  |  |  |
|  | Bands FDD_D |  | - |  |  |  |
|  | Bands FDD_E, FDD_F Note 9 |  | - |  |  |  |
|  | Bands FDD_G |  | - |  |  |  |
|  | Bands FDD_H |  | - |  |  |  |
|  | Bands TDD_A |  | -121 | - | - |  |
|  | Bands TDD_C |  | -120 |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |
| Io Note 4 | Bands FDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB+10log (NRB channel2 / NRB channel 1)) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | - |  |  |  |
|  | Bands FDD_C |  | - |  |  |  |
|  | Bands FDD_D |  | - |  |  |  |
|  | Bands FDD_E, FDD_F Note 9 |  | - |  |  |  |
|  | Bands FDD_G |  | - |  |  |  |
|  | Bands FDD_H |  | - |  |  |  |
|  | Bands TDD_A |  | -87.76 + 10log(NRB,c/50) | - |  |  |
|  | Bands TDD_C |  | -86.76 + 10log(NRB,c/50) | - |  |  |
|  | Bands TDD_E |  | -85.76 + 10log(NRB,c/50) | - |  |  |
| Propagation condition |  | - | AWGN | AWGN |  | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 |  | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 |  | 3 |
| Time alignment error relative to cell 1 Note 8 |  | - | - | TAE |  | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.23.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.1.24.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.1.7.1.

#### A.9.1.24.2	Test parameters

The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.1.24.2-1 will replace the values of corresponding parameters in Tables A.9.1.7.2-1.

Table A.9.1.24.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Combination | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| BWchannel Note 1 |  | MHz | 20MHz+10MHz | 20MHz: NRB,c = 100 | 10MHz: NRB,c = 50 |  |  |
|  |  |  | 10MHz+20MHz | 10MHz: NRB,c = 50 | 20MHz: NRB,c = 100 |  |  |
| Measurement bandwidth |  |  | 20MHz+10MHz | 47-52 | 22-27 |  |  |
|  |  |  | 10MHz+20MHz | 22-27 | 47-52 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 20MHz+10MHz | R.3 TDD | R.0 TDD | N/A |  |
|  |  |  | 10MHz+20MHz | R.0 TDD | R3.TDD |  |  |
| PDSCH allocation |  |  | 20MHz+10MHz | 38-61 | 13-36 | N/A |  |
|  |  |  | 10MHz+20MHz | 13-36 | 38-61 |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 20MHz+10MHz | R.10 TDD | R.6 TDD |  |  |
|  |  |  | 10MHz+20MHz | R.6 TDD | R.10 TDD |  |  |
| OCNG Patterns defined in A.3.2.2 (TDD) |  |  | 20MHz+10MHz | OP.7 TDD | OP.1 TDD | OP.2 TDD |  |
|  |  |  | 10MHz+20MHz | OP.1 TDD | OP.7 TDD | OP.8 TDD |  |
| IoNote2 | Bands TDD_A | dBm/BWchannel | All | -87.76 + 10log(NRB,c/50) | N/A |  |  |
|  | Bands TDD_C |  |  | -86.76 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_E |  |  | -85.76 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_A | dBm/ BWchannel | All | N/A | (Io for Channel 1 +5.33dB) +10log (NRB channel2 / NRB channel 1)) |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.1.7.2-1 for the other parameters. Note 4: For each parameter, the allowed combinations are shown in separate rows. |  |  |  |  |  |  |  |

#### A.9.1.24.3	Test Requirements

The test requirements defined in section A.9.1.7.3 shall apply to this test case.

### A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal

#### A.9.1.25.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRP absolute and relative measurement accuracies in CRS based discovery signal are within the specified limits. This test will verify the requirements in Sections 9.1.14.2.

#### A.9.1.25.2	Test parameters

In this test case, all cells are on the same carrier frequency. Both absolute and relative accuracies of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.25.2-1. In this test case, Cell 1 is the PCell and Cell 2 is the target cell. The Cell 2 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.25.2-1: RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  |
| BWchannel |  | MHz | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  |
| DTMC period |  | ms | N/A | 160 |
| DTMC period offset |  |  | N/A | 10 |
| Discovery signal occasion duration |  | ms | N/A | 1 |
| Time offset between cells |  | µs | 2.3 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -106 | -106 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 2.5 | -6 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -100 | -105 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -70.27 | -70.27 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 6 | 1 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.25.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.2.

### A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal

#### A.9.1.26.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative measurement accuracies in CRS based discovery signal are within the specified limits. This test will verify the requirements in Sections 9.1.14.2.

#### A.9.1.26.2	Test parameters

In this test case all cells are on the same carrier frequency. Both absolute and relative accuracies of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.26.2-1. In this test case, Cell 1 is the PCell and Cell 2 is the target cell. The Cell 2 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.26.2-1: RSRP TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  |
| BWchannel |  | MHz | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  |
| DTMC period |  | ms | N/A | 160 |
| DTMC period offset |  |  | N/A | 10 |
| Discovery signal occasion duration |  | ms | N/A | 2 |
| Time offset between cells |  | µs | 2.3 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -106 | -106 |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 2.5 | -6 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -100 | -105 |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -70.27 | -70.27 |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 6 | 1 |
| Propagation condition |  | - | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.1.26.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14. 2.

### A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal

#### A.9.1.27.1	Test Purpose and Environment

The purpose of this test is to verify that the CRS RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.2 for FDD—FDD inter frequency measurements.

#### A.9.1.27.2	Test parameters

In this set of test case the cells are on different carrier frequencies. Both absolute and relative accuracy of CRS RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.27.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap and a DMTC configuation.

Table A.9.1.27.2-1: CRS RSRP FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Gap Pattern Id |  |  | 0 | - |  |
| gapOffset |  | ms | 9 |  |  |
| DMTC period |  | ms | - | 160 |  |
| DMTC period offset |  | ms | - | 10 |  |
| Discovery signal occasion duration |  | ms | - | 1 |  |
| Time offset between cells |  | µs | - | 3 |  |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |  |
| PDSCH allocation |  |  | 13—36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD |  |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A Note 9 | dBm/15 kHz | ( for Channel 2 +6dB) | -115 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -114.5 |
|  | Bands FDD_C |  |  | -114 |
|  | Bands FDD_D |  |  | -113.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -113 |
|  | Bands FDD_G Note 7 |  |  | -112 |
|  | Bands FDD_H |  |  | -111.5 |
|  |  | dB | 13 | -6 |  |
| RSRPNote3 | Bands FDD_A Note 9 | dBm/15 kHz | (RSRP for Cell 2 +25dB) | -121 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -119 |
|  | Bands FDD_G Note 7 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| IoNote3 | Bands FDD_A Note 9 | dBm/9 MHz | (Io for Channel 2 +18.24dB) | -86.25 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -85.75 |
|  | Bands FDD_C |  |  | -85.25 |
|  | Bands FDD_D |  |  | -84.75 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -84.25 |
|  | Bands FDD_G Note 7 |  |  | -83.25 |
|  | Bands FDD_H |  |  | -82.75 |
|  |  | dB | 13 | -6 |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 9: Except Band 32, Band 75, Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.27.3	Test Requirements

The CRS RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.2.

### A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal

#### A.9.1.28.1	Test Purpose and Environment

The purpose of this test is to verify that the CRS RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.2 for TDD—TDD inter frequency measurements.

#### A.9.1.28.2	Test parameters

In this set of test case the cells are on different carrier frequencies. Both absolute and relative accuracy of CRS RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.28.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap and a DMTC configuation.

Table A.9.1.28.2-1: CRS RSRP TDD—TDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Gap Pattern Id |  |  | 0 | - |  |
| gapOffset |  | ms | 9 |  |  |
| DMTC period |  | ms | - | 160 |  |
| DMTC period offset |  | ms | - | 10 |  |
| Discovery signal occasion duration |  | ms | - | 2 |  |
| Time offset between cells |  | µs | - | 3 |  |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |  |
| PDSCH allocation |  |  | 13—36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD |  |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | ( for Channel 2 +6dB) | -115 |  |
|  | Bands TDD_C |  |  | -114 |
|  | Bands TDD_E |  |  | -113 |
|  |  | dB | 13 | -6 |  |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | (RSRP for Cell 2 +25dB) | -121 |  |
|  | Bands TDD_C |  |  | -120 |
|  | Bands TDD_E |  |  | -119 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | (Io for Channel 2 +18.24dB) | -86.25 |  |
|  | Bands TDD_C |  |  | -85.25 |
|  | Bands TDD_E |  |  | -84.25 |
|  |  | dB | 13 | -6 |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of test |  |  |  |  |  |

#### A.9.1.28.3	Test Requirements

The CRS RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.2.

### A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal

#### A.9.1.29.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI- RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.3 for FDD intra frequency measurements.

#### A.9.1.29.2	Test parameters

In this set of test case all cells are on the same carrier frequencies. Both absolute and relative accuracy of CSI- RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.29.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The intra frequency measurements are supported by a DMTC configuration.

Table A.9.1.29.2-1: CSI-RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  |
| BWchannel |  | MHz | 10 |  |  |  |
| DMTC period |  | ms | 160 |  |  |  |
| DMTC period offset |  | ms | 10 |  |  |  |
| Discovery signal occasion duration |  | ms | 1 |  |  |  |
| CSI-RS resource configuration |  |  | 2 |  | 4 |  |
| CSI-RS periodicity |  | ms | 10 |  |  |  |
| CSI-RS subframe offset |  | ms | 0 |  |  |  |
| CSI-RS individual offset[2] |  | dB | 0 | 0 |  |  |
| CSI-RS muting |  |  | Enable | Enable |  |  |
| Time offset between cells |  | µs | - | 2.3 |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD |  | - |  |
| PDSCH allocation |  |  | 13—36 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA |  | dB | 0 |  | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| p-C-r10[2] |  | dB | -6 |  | -6 |
| Note2 | Bands FDD_A Note 9 | dBm/15 kHz | -116 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -115.5 |  |  |
|  | Bands FDD_C |  | -115 |  |  |
|  | Bands FDD_D |  | -114.5 |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -114 |  |  |
|  | Bands FDD_G Note 7 |  | -113 |  |  |
|  | Bands FDD_H |  | -112.5 |  |  |
| CRS |  | dB | 0.46 |  | -5.76 |  |
| CSI-RS |  | dB | 6.46 |  | 0.24 |  |
| RSRPNote3 | Bands FDD_A Note 9 | dBm/15 kHz | -113 |  | -117 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -112.5 |  | -116.5 |
|  | Bands FDD_C |  | -112 |  | -116 |
|  | Bands FDD_D |  | -111.5 |  | -115.5 |
|  | Bands FDD_E, FDD_F Note 5 |  | -111 |  | -115 |
|  | Bands FDD_G Note 7 |  | -110 |  | -114 |
|  | Bands FDD_H |  | -109.5 |  | -113.5 |
| CSI-RSRP Note3 | Bands FDD_A Note 9 | dBm/15 kHz | (RSRP for Cell 1 +6dB) |  | (RSRP for Cell 2 +6dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A Note 9 | dBm/9 MHz | -82.43 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -81.93 |  |  |
|  | Bands FDD_C |  | -81.43 |  |  |
|  | Bands FDD_D |  | -80.93 |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -80.43 |  |  |
|  | Bands FDD_G Note 7 |  | -79.43 |  |  |
|  | Bands FDD_H |  | -78.93 |  |  |
| CRS |  | dB | 3 |  | -1 |  |
| CSI-RS |  | dB | 9 |  | 5 |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 9: Except Band 32, Band 75, Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.29.3	Test Requirements

The CSI- RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.3.

### A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal

#### A.9.1.30.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI- RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.3 for TDD intra frequency measurements.

#### A.9.1.30.2	Test parameters

In this set of test case all cells are on the same carrier frequencies. Both absolute and relative accuracy of CSI- RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.30.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The intra frequency measurements are supported by a DMTC configuation.

Table A.9.1.30.2-1: CSI-RSRP TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  |
| BWchannel |  | MHz | 10 |  |  |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |  |
| DMTC period |  | ms | 160 |  |  |  |
| DMTC period offset |  | ms | 10 |  |  |  |
| Discovery signal occasion duration |  | ms | 2 |  |  |  |
| CSI-RS resource configuration |  |  | 2 |  | 4 |  |
| CSI-RS periodicity |  | ms | 10 |  |  |  |
| CSI-RS subframe offset |  | ms | 0 |  |  |  |
| CSI-RS individual offset[2] |  | dB | 0 | 0 |  |  |
| CSI-RS muting |  |  | Enable | Enable |  |  |
| Time offset between cells |  | µs | - | 2.3 |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 TDD |  | - |  |
| PDSCH allocation |  |  | 13—36 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA |  | dB | 0 |  | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| p-C-r10[2] |  | dB | -6 |  | -6 |
| Note3 | Bands TDD_A | dBm/15 kHz | -116 |  |  |
|  | Bands TDD_C |  | -115 |  |  |
|  | Bands TDD_E |  | -114 |  |  |
| CRS |  | dB | 0.46 |  | -5.76 |
| CSI-RS |  | dB | 6.46 |  | 0.24 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -113 |  | -117 |
|  | Bands TDD_C |  | -112 |  | -116 |
|  | Bands TDD_E |  | -111 |  | -115 |
| CSI-RSRP Note4 | Bands TDD_A | dBm/15 kHz | (RSRP for Cell 1 +6dB) |  | (RSRP for Cell 2 +6dB) |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -82.43 |  |  |
|  | Bands TDD_C |  | -81.43 |  |  |
|  | Bands TDD_E |  | -80.43 |  |  |
| CRS |  | dB | 3 |  | -1 |  |
| CSI-RS |  | dB | 9 |  | 5 |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. |  |  |  |  |  |  |

#### A.9.1.30.3	Test Requirements

The CSI- RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.3.

### A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal

#### A.9.1.31.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.3 for FDD—FDD inter frequency measurements.

#### A.9.1.31.2	Test parameters

In this set of test case the cells are on different carrier frequencies. Both absolute and relative accuracy of CSI-RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.31.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap and two DMTC configurations which one is for cell1 and the other is for cell2.

Table A.9.1.31.2-1: CSI-RSRP FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Gap Pattern Id |  |  | 0 | - |  |
| gapOffset |  | ms | 9 |  |  |
| DMTC period |  | ms | 160 | 160 |  |
| DMTC period offset |  | ms | 0 | 10 |  |
| Discovery signal occasion duration |  | ms | 1 | 1 |  |
| CSI-RS resource configuration |  |  | 2 | 4 |  |
| CSI-RS periodicity |  | ms | 10 |  |  |
| CSI-RS subframe offset |  | ms | 0 |  |  |
| CSI-RS individual offset[2] |  | dB | 0 | 0 |  |
| CSI-RS muting |  |  | Enable | Enable |  |
| Time offset between cells |  | µs | - | 3 |  |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |  |
| PDSCH allocation |  |  | 13—36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD |  |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| p-C-r10[2] |  | dB | 0 | -6 |
| Note2 | Bands FDD_A Note 9 | dBm/15 kHz | ( for Channel 2 +6dB) | -115 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -114.5 |
|  | Bands FDD_C |  |  | -114 |
|  | Bands FDD_D |  |  | -113.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -113 |
|  | Bands FDD_G Note 7 |  |  | -112 |
|  | Bands FDD_H |  |  | -111.5 |
| CRS |  | dB | 13 | -6 |  |
| CSI-RS |  | dB | 13 | 0 |  |
| RSRPNote3 | Bands FDD_A Note 9 | dBm/15 kHz | (RSRP for Cell 2 +25dB) | -121 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -119 |
|  | Bands FDD_G Note 7 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| CSI-RSRP Note3 | Bands FDD_A Note 9 | dBm/15 kHz | (RSRP for Cell 1 +0dB) | (RSRP for Cell 2 +6dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A Note 9 | dBm/9 MHz | (Io for Channel 2 +18.24dB) | -86.25 |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -85.75 |
|  | Bands FDD_C |  |  | -85.25 |
|  | Bands FDD_D |  |  | -84.75 |
|  | Bands FDD_E, FDD_F Note 5 |  |  | -84.25 |
|  | Bands FDD_G Note 7 |  |  | -83.25 |
|  | Bands FDD_H |  |  | -82.75 |
| CRS |  | dB | 13 | -6 |  |
| CSI-RS |  | dB | 13 | 0 |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 9: Except Band 32, Band 75, Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.31.3	Test Requirements

The CSI-RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.3.

### A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal

#### A.9.1.32.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.14.3 for TDD—TDD inter frequency measurements.

#### A.9.1.32.2	Test parameters

In this set of test case the cells are on different carrier frequencies. Both absolute and relative accuracy of CSI-RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.32.2-1. In this test case, Cell 1 is the PCell and Cell 2 the target cell. The inter frequency measurements are supported by a measurement gap and two DMTC configurations which one is for cell	1 and the other is for cell2.

Table A.9.1.32.2-1: CSI-RSRP TDD—TDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Gap Pattern Id |  |  | 0 | - |  |
| gapOffset |  | ms | 9 |  |  |
| DMTC period |  | ms | 160 | 160 |  |
| DMTC period offset |  | ms | 0 | 10 |  |
| Discovery signal occasion duration |  | ms | 2 | 2 |  |
| CSI-RS resource configuration |  |  | 2 | 4 |  |
| CSI-RS periodicity |  | ms | 10 |  |  |
| CSI-RS subframe offset |  | ms | 0 |  |  |
| CSI-RS individual offset[2] |  | dB | 0 | 0 |  |
| CSI-RS muting |  |  | Enable | Enable |  |
| Time offset between cells |  | µs | - | 3 |  |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |  |
| PDSCH allocation |  |  | 13—36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD |  |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| p-C-r10[2] |  | dB | 0 | -6 |
| Note3 | Bands TDD_A | dBm/15 kHz | ( for Channel 2 +6dB) | -115 |  |
|  | Bands TDD_C |  |  | -114 |
|  | Bands TDD_E |  |  | -113 |
| CRS |  | dB | 13 | -6 |  |
| CSI-RS |  | dB | 13 | 0 |  |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | (RSRP for Cell 2 +25dB) | -121 |  |
|  | Bands TDD_C |  |  | -120 |
|  | Bands TDD_E |  |  | -119 |
| CSI-RSRP Note4 | Bands TDD_A | dBm/15 kHz | (RSRP for Cell 1 +0dB) | (RSRP for Cell 2 +6dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | (Io for Channel 2 +18.24dB) | -86.25 |  |
|  | Bands TDD_C |  |  | -85.25 |
|  | Bands TDD_E |  |  | -84.25 |
| CRS |  | dB | 13 | -6 |  |
| CSI-RS |  | dB | 13 | 0 |  |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: DMTC is provided to the UE in the measDS-Config (in TS36.331) before the beginning of test. |  |  |  |  |  |

#### A.9.1.32.3	Test Requirements

The CSI-RSRP measurement accuracy shall fulfil the requirements in sections 9.1.14.3.

### A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal

#### A.9.1.33.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRP absolute and relative measurement accuracies in carrier aggregation in CRS based discovery signal are within the specified limits. This test will verify the absolute RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.15.1.2, and the relative RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.15.1.2. The test will also verify the primary and secondary component carrier relative RSRP accuracy requirement defined in Clause 9.1.15.1.3.

#### A.9.1.33.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier and activated, and Cell3 is the neighboring cell on the secondary component carrier. The test parameters are given in Table A.9.1.33.2-1. The Cell 3 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.33.2-1: RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 |
| DMTC period |  | ms | N/A | N/A | 160 |
| DMTC period offset |  |  | N/A | N/A | 10 |
| Discovery signal occasion duration |  | ms | N/A | N/A | 1 |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -87.26 |  |  |
|  | Bands FDD_C |  | -86.76 |  |  |
|  | Bands FDD_D |  | -86.26 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 |  |  |
|  | Bands FDD_G |  | -84.76 |  |  |
|  | Bands FDD_H |  | -84.26 |  |  |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.33.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 3 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.1.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.1.2

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.15.1.3.

### A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal

#### A.9.1.34.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRP absolute and relative measurement accuracies in carrier aggregation in CRS based discovery signal are within the specified limits. This test will verify the absolute RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.15.1.2, and the relative RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.15.1.2. The test will also verify the primary and secondary component carrier relative RSRP accuracy requirement defined in Clause 9.1.15.1.3.

#### A.9.1.34.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier and activated, and Cell3 is the neighboring cell on the secondary component carrier. The test parameters are given in Table A.9.1.34.2-1. The Cell 3 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.34.2-1: Carrier aggregation RSRP test parameters for TDD

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 |
| DMTC period |  | ms | N/A | N/A | 160 |
| DMTC period offset |  |  | N/A | N/A | 10 |
| Discovery signal occasion duration |  | ms | N/A | N/A | 2 |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands TDD_C |  | -86.76 |  |  |
|  | Bands TDD_E |  | -85.76 |  |  |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### A.9.1.34.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following three perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 3 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.1.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.1.2

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.15.1.3.

### A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal

#### A.9.1.35.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD CSI-RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute CSI-RSRP accuracy requirements of the primary component carrier defined in clause 9.1.15.2.1, the absolute CSI-RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.15.2.2, and the relative CSI-RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.15.2.2. The test will also verify the primary and secondary component carrier relative CSI-RSRP accuracy requirement defined in Clause 9.1.15.2.3.

#### A.9.1.35.2	Test parameters

In this set of cases cell1 is PCell on the primary component carrier, cell2 is SCell on the secondary component carrier and activated, and cell3 is the neighboring cell on the secondary component carrier.  The test parameters are given in Table A.9.1.35.2-1. This set is supported by two DMTC configurations which one is for cell1 and the other is for cell2 and cell3.

Table A.9.1.35.2-1: CSI-RSRP FDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |  |
| BWchannel |  | MHz | 10 | 10 | 10 |  |
| Timing offset to cell1 |  | s | - | 0 | 3 |  |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |  |
| DMTC period |  | ms | 160 | 160 |  |  |
| DMTC period offset |  | ms | 0 | 10 |  |  |
| Discovery signal occasion duration |  | ms | 1 | 1 |  |  |
| CSI-RS resource configuration |  |  | 2 | 4 |  | 6 |
| CSI-RS periodicity |  | ms | 10 | 10 |  | 10 |
| CSI-RS subframe offset |  | ms | 0 | 0 |  | 0 |
| CSI-RS individual offset[2] |  | dB | 0 | 0 |  | 0 |
| CSI-RS muting |  |  | Enable | Enable |  | Enable |
| Measurement bandwidth |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.0 FDD | - |  |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.1 FDD | OP.2 FDD |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |  |
| p-C-r10[2] |  | dB | -6 | -6 | -6 |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |
| CRS |  | dB | -4 | 0.46 | -5.76 |  |
| CSI-RS |  | dB | 2 | 6.46 | 0.24 |  |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |
| CSI-RSRPNote3 | Bands FDD_A | dBm/15 kHz | -115 | (CSI-RSRP for Cell 1 +8dB) | (CSI-RSRP for Cell 1 +4dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -114.5 |  |  |  |
|  | Bands FDD_C |  | -114 |  |  |  |
|  | Bands FDD_D |  | -113.5 |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -113 |  |  |  |
|  | Bands FDD_G |  | -112 |  |  |  |
|  | Bands FDD_H |  | -111.5 |  |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26 |  |  |  |
|  | Bands FDD_C |  | -86.76 |  |  |  |
|  | Bands FDD_D |  | -86.26 |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 |  |  |  |
|  | Bands FDD_G |  | -84.76 |  |  |  |
|  | Bands FDD_H |  | -84.26 |  |  |  |
| CRS |  | dB | -4 | 3 | -1 |  |
| CSI-RS |  | dB | 2 | 9 | 5 |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.35.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.15.2.1.

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.2.2.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.2.2.

-	The relative accuracy of inter-frequency CSI-RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.15.2.3.

### A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal

#### A.9.1.36.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD CSI-RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute CSI-RSRP accuracy requirements of the primary component carrier defined in clause 9.1.15.2.1, the absolute CSI-RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.15.2.2, and the relative CSI-RSRP accuracy requirements of the secondary component carrier defined in clause 9.1.15.2.2. The test will also verify the primary and secondary component carrier relative CSI-RSRP accuracy requirement defined in Clause 9.1.15.2.3.

#### A.9.1.36.2	Test parameters

In this set of cases cell1 is PCell on the primary component carrier, cell2 is SCell on the secondary component carrier and activated, and cell3 is the neighboring cell on the secondary component carrier.  The test parameters are given in Table A.9.1.36.2-1. This set is supported by two DMTC configurations which one is for cell1 and the other is for cell2 and cell3.

Table A.9.1.36.2-1: CSI-RSRP TDD carrier aggregation test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 |  |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| DMTC period |  | ms | 160 | 160 |  |
| DMTC period offset |  | ms | 0 | 10 |  |
| Discovery signal occasion duration |  | ms | 2 | 2 |  |
| CSI-RS resource configuration |  |  | 2 | 4 | 6 |
| CSI-RS periodicity |  | ms | 10 | 10 | 10 |
| CSI-RS subframe offset |  | ms | 0 | 0 | 0 |
| CSI-RS individual offset[2] |  | dB | 0 | 0 | 0 |
| CSI-RS muting |  |  | Enable | Enable | Enable |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| p-C-r10[2] |  | dB | -6 | -6 | -6 |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
| CRS |  | dB | -4 | 0.46 | -5.76 |
| CSI-RS |  | dB | 2 | 6.46 | 0.24 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| CSI-RSRPNote4 | Bands TDD_A | dBm/15 kHz | -115 | (CSI-RSRP for Cell 1 +8dB) | (CSI-RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -114 |  |  |
|  | Bands TDD_E |  | -113 |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -87.76 | (Io for Channel 1 +5.33dB) |  |
|  | Bands TDD_C |  | -86.76 |  |  |
|  | Bands TDD_E |  | -85.76 |  |  |
| CRS |  | dB | -4 | 3 | -1 |
| CSI-RS |  | dB | 2 | 9 | 5 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test |  |  |  |  |  |

#### A.9.1.36.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.15.2.1.

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.2.2.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.15.2.2.

-	The relative accuracy of inter-frequency CSI-RSRP measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.15.2.3.

### A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.37.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in FDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.37.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2 and cell 4 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. Cell 3 and cell 5 are neighbouring cells on secondary component carriers SCC1 and SCC2 respectively.  The test parameters are given in Table A.9.1.37.2-1.

Table A.9.1.37.2-1: 3 Downlink PCell in FDD RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | - | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | - | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -117 | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -121 | - | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.1.37.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 7 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.38.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in TDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.38.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2 and cell 4 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. Cell 3 and cell 5 are neighbouring cells on secondary component carriers SCC1 and SCC2 respectively.  The test parameters are given in Table A.9.1.38.2-1.

Table A.9.1.38.2-1: 3 Downlink PCell in TDD RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 | - |  | - |  |
| Uplink/downlink configurationNote1 |  |  | 1 | - |  | - |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note3 | Bands FDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -117 | - |  | - |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands FDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -121 | - | - | - | - |
|  | Bands TDD_C |  | -120 |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |
| IoNote4 | Bands FDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.1.38.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 7 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.39.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.39.2	Test parameters

In this set of test cases there are five cells on three carrier frequencies. Cell 1 is PCell on channel 1, and cell 2 and cell 4 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. Cell 3 and cell 5 are neighbouring cells on secondary component carriers SCC1 and SCC2 respectively.  The parameters for the test are listed in Table A.9.1.39.2-1.

Table A.9.1.39.2-1: 3 DL FDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76 +10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -87.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table A.9.1.39.2-2: 3 DL FDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #4 and cell #5)

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.39.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.40.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.40.2	Test parameters

In this set of test cases there are five cells on three carrier frequencies. Cell 1 is PCell on channel 1, and cell 2 and cell 4 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. Cell 3 and cell 5 are neighbouring cells on secondary component carriers SCC1 and SCC2 respectively.  The parameters for the test are listed in Table A.9.1.40.2-1.

Table A.9.1.40.2-1: 3 DL TDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76 + 10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76 + 10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76 + 10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table A.9.1.40.2-2: 3 DL TDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #4 and cell #5)

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.1.40.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following four perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0

#### A.9.1.41.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.13.1 and 9.1.13.2 for FD-FDD intra frequency RSRP measurements for UE category 0.

#### A.9.1.41.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.41.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.41.2-1: FD-FDD RSRP Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.3 |  |  | R.13 FDD | - | R.13 FDD | - | R.13 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1 |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-0_A Note 7 | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -113 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-0_A Note 7 | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-0_B |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-0_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-0_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-0_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-0_A Note 7 | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: Except Band 29. Note 7: Except Band 32, Band 75, Band 76. |  |  |  |  |  |  |  |  |

#### A.9.1.41.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.13.1 and 9.1.13.2.

### A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0

#### A.9.1.42.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.13.1 and 9.1.13.2 for HD-FDD intra frequency RSRP measurements for UE category 0.

#### A.9.1.42.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.42.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.42.2-1: HD-FDD RSRP Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.4 |  |  | R.1 HD-FDD | - | R.1 HD-FDD | - | R.1 HD-FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.3 |  |  | R.3 HD-FDD |  | R.3 HD-FDD |  | R.3 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1 |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-0_A Note 7 | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-0_E, FDD-0_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -113 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-0_A Note 7 | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-0_B |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-0_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-0_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-0_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-0_A Note 7 | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: Except Band 29. Note 7: Except Band 32, Band 75, Band 76. |  |  |  |  |  |  |  |  |

#### A.9.1.42.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.13.1 and 9.1.13.2.

### A.9.1.43	TDD RSRP Intra frequency case for UE category 0

#### A.9.1.43.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.13.1 and 9.1.13.2 for TDD intra frequency RSRP measurements for UE category 0.

#### A.9.1.43.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.43.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.43.2-1: TDD RSRP Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.5 |  |  | R.12 TDD | - | R.12 TDD | - | R.12 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-0_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands TDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands TDD-0_E |  |  |  |  |  | -114 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note4 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD-0_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands TDD-0_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD-0_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD-0_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands TDD-0_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD-0_E |  |  |  |  |  | -80.43 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.43.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.13.1 and 9.1.13.2.

### A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.44.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD-TDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in FDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.44.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4 and cell 6 are activated SCells on secondary component carriers SCC1, SCC2 and SCC3 respectively. Cell 3, cell 5 and cell 7 are neighbouring cells on secondary component carriers SCC1, SCC2 and SCC3 respectively.  The test parameters are given in Table A.9.1.44.2-1.

Table A.9.1.44.2-1: 4 Downlink PCell in FDD-TDD RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | - | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | - | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  | -117 | - |  | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  | -121 | - | - | - | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel4 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - | TAE | - |
| Time alignment error relative to cell 4Note 8 |  |  | - | - | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |

#### A.9.1.44.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 10 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.45.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in TDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.45.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4 and cell 6 are activated SCells on secondary component carriers SCC1, SCC2 and SCC3 respectively. Cell 3, cell 5 and cell 7 are neighbouring cells on secondary component carriers SCC1, SCC2 and SCC3 respectively.  The test parameters are given in Table A.9.1.45.2-1.

Table A.9.1.45.2-1: 4 Downlink PCell in TDD-FDD RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 | - |  | - |  | - |  |
| Uplink/downlink configurationNote1 |  |  | 1 | - |  | - |  | - |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| Note3 | Bands FDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) | ( for Channel 1 +1dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |
|  | Bands TDD_A |  | -117 | - |  | - |  | - |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands FDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |
|  | Bands TDD_A |  | -121 | - | - | - | - | - | - |
|  | Bands TDD_C |  | -120 |  |  |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |  |  |
| IoNote4 | Bands FDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel4 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |
|  | Bands TDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  | - |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note8 |  |  | - | - | - | TAE | - | TAE | - |
| Time alignment error relative to cell 4Note8 |  |  | - | - | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |

#### A.9.1.45.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.46.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in FDD-FDD carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.46.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4 and cell 6 are activated SCells on secondary component carriers SCC1, SCC2 and SCC3 respectively. Cell 3, cell 5 and cell 7 are neighbouring cells on secondary component carriers SCC1, SCC2 and SCC3 respectively.  The test parameters are given in Table A.9.1.46.2-1, Table A.9.1.46.2-2 and Table A.9.1.46.2-3.

Table A.9.1.46.2-1: 4 DL FDD RSRP carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76 +10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -87.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table A.9.1.46.2-2: 4 DL FDD RSRP carrier aggregation test parameters for cell 4 and cell 5

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table A.9.1.46.2-3: 4 DL FDD RSRP carrier aggregation test parameters for cell 6 and cell 7

| Parameter |  | Unit | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 4Note 7 |  |  | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.46.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 10 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.47.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in TDD-TDD carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.47.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4 and cell 6 are activated SCells on secondary component carriers SCC1, SCC2 and SCC3 respectively. Cell 3, cell 5 and cell 7 are neighbouring cells on secondary component carriers SCC1, SCC2 and SCC3 respectively.  The test parameters are given in Table A.9.1.47.2-1, Table A.9.1.47.2-2 and Table A.9.1.47.2-3.

Table A.9.1.47.2-1: 4 DL TDD RSRP carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76 + 10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76 + 10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76 + 10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table A.9.1.47.2-2: 4 DL TDD RSRP carrier aggregation test parameters for cell 4 and cell 5

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

Table A.9.1.47.2-3: 4 DL TDD RSRP carrier aggregation test parameters for cell 6 and cell 7

| Parameter |  | Unit | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 7 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.1.47.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 10 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.48.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in FDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.48.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell 6 and cell 8 are activated SCells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively. Cell 3, cell 5, cell 7 and cell 9 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively.  The test parameters are given in Table A.9.1.48.2-1, Table A.9.1.48.2-2 and Table A.9.1.48.2-3.

Table A.9.1.48.2-1: 5 Downlink PCell in FDD RSRP carrier aggregation test parameters for cell 1, cell 2, cell 3, cell 4 and cell 5

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | - | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | - | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.7 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -117 | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -121 | - | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

Table A.9.1.48.2-2: 5 Downlink PCell in FDD RSRP carrier aggregation test parameters for cell 6 and cell 7

| Parameter |  | Unit | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 7 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

Table A.9.1.48.2-3: 5 Downlink PCell in FDD RSRP carrier aggregation test parameters for cell 8 and cell 9

| Parameter |  | Unit | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 5 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 7 |  |  | TAE |  |
| Time alignment error relative to cell 6 Note 7 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.1.48.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 13 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.49.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in TDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.49.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell 6 and cell 8 are activated SCells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively. Cell 3, cell 5, cell 7 and cell 9 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively.  The test parameters are given in Table A.9.1.49.2-1, Table A.9.1.49.2-2 and Table A.9.1.49.2-3.

Table A.9.1.49.2-1: 5 Downlink PCell in TDD RSRP carrier aggregation test parameters for cell 1, cell 2, cell 3, cell 4 and cell 5

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 | - |  | - |  |
| Uplink/downlink configurationNote1 |  |  | 1 | - |  | - |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note3 | Bands FDD_A | dBm/15 kHz | - | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -117 | - |  | - |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands FDD_A | dBm/15 kHz | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -121 | - | - | - | - |
|  | Bands TDD_C |  | -120 |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |
| IoNote4 | Bands FDD_A | dBm/ BWchannel | - | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
|  | Bands TDD_A |  | -87.76+10log(NRB,c/50) | - |  | - |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

Table A.9.1.49.2-2: 5 Downlink PCell in TDD RSRP carrier aggregation test parameters for cell 6 and cell 7

| Parameter |  | Unit | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 4Note 7 |  |  | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table A.9.1.49.2-3: 5 Downlink PCell in TDD RSRP carrier aggregation test parameters for cell 8 and cell 9

| Parameter |  | Unit | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 5 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 4Note 7 |  |  | TAE | - |
| Time alignment error relative to cell 6Note 7 |  |  | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.49.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 13 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.50.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD-FDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in FDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.50.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell6 and cell8 are activated SCells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively. Cell 3, cell 5, cell7 and cell9 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively.  The test parameters are given in Table A.9.1.50.2-1.

Table A.9.1.50.2-1: 5 DL FDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76 +10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -87.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | - |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | - |
| Time alignment error relative to cell 4 Note 7 |  |  | - | - | - |
| Time alignment error relative to cell 6 Note 7 |  |  | - | - | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table A.9.1.50.2-2: 5 DL FDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #4 – cell #9)

| Parameter |  | Unit | Cell 4 | Cell 5 | Cell 6 | Cell 7 | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  | 4 |  | 5 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
|  |  | dB | 3 | -1 | 3 | -1 | 3 | -1 |
|  |  | dB | 0.46 | -5.76 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel4 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel5 / NRB channel 1)) |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 4Note 7 |  |  | - | - | TAE | - | TAE | - |
| Time alignment error relative to cell 6Note 7 |  |  | - | - |  | - | TAE | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.50.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation

#### A.9.1.51.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRP absolute and relative accuracy requirements in carrier aggregation with PCell in TDD are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.51.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell6 and cell8 are activated SCells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively. Cell 3, cell 5, cell7 and cell9 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively.  The test parameters are given in Table A.9.1.51.2-1.

Table A.9.1.51.2-1: 5 DL TDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  |  | dB | -4 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76 + 10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76 + 10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76 + 10log(NRB,c/50) |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table A.9.1.51.2-2: 5 DL TDD RSRP test parameters for E-UTRAN Carrier aggregation (cell #4 – cell #9)

| Parameter |  | Unit | Cell 4 | Cell 5 | Cell 6 | Cell 7 | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 3 |  | 4 |  | 5 |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configurationNote1 |  |  | 6 |  |  |  |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  |  |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |
|  |  | dB | 3 | -1 | 3 | -1 | 3 | -1 |
|  |  | dB | 0.46 | -5.76 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP1 +8dB for Cell) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel4 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel5 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1Note 7 |  |  | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 7 |  |  | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 4Note 7 |  |  | - | - | TAE | - | TAE | - |
| Time alignment error relative to cell 6Note 7 |  |  | - | - |  | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.51.3	Test Requirements

In the test, the performance of RSRP measurements is verified form following perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.52.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.52.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.52.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.52.2-1: FD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.20 FDD | - | R.20 FDD | - | R.20 FDD | - |
| PDSCH allocation |  |  | Follows R.20 FDD | - | Follows R.20 FDD | - | Follows R.20 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 FDD |  | R.16 FDD |  | R.16 FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.52.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

#### A.9.1.52A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.52A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.52A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.52A.2-1: FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.28 FDD | - | R.28 FDD | - | R.28 FDD | - |
| PDSCH allocation |  |  | Follows R.28 FDD | - | Follows R.28 FDD | - | Follows R.28 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.24 FDD |  | R.24 FDD |  | R.24 FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -103 |  | -83 |  | -109.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -97 | -102 | -77 | -82 | -106.5 | -110.5 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.28 |  | -50.28 |  | -78.94 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.52A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.53.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.53.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.53.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.53.2-1: HD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.10 HD-FDD | - | R.10 HD-FDD | - | R.10 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.6 HD-FDD |  | R.6 HD-FDD |  | R.6 HD-FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.53.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

#### A.9.1.53A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.53A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.53A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.53A.2-1: HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.18 HD-FDD | - | R.18 HD-FDD | - | R.18 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.18 HD-FDD | - | Follows R.18 HD-FDD | - | Follows R.18 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 HD-FDD |  | R.14 HD-FDD |  | R.14 HD-FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -103 |  | -83 |  | -109.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -97 | -102 | -77 | -82 | -106.5 | -110.5 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.28 |  | -50.27 |  | -78.94 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.53A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.54.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for TDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.54.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.54.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.54.2-1: TDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.16 TDD | - | R.16 TDD | - | R.16 TDD | - |
| PDSCH allocation |  |  | Follows R.16 TDD | - | Follows R.16 TDD | - | Follows R.16 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 TDD |  | R.14 TDD |  | R.14 TDD |  |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -114 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands TDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD-M1_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -80.43 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | s | - | 3 | - | 3 | - | 3 |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.54.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell

#### A.9.1.55.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD intra frequency RSRP absolute and relative measurement accuracies in carrier aggregation with frame structure 3 in the configured DMTC occasion are within the specified limits. This test will verify the absolute RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.19.2, and the relative RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.19.2. The test will also verify the primary and secondary component carrier relative RSRP accuracy requirement defined in Clause 9.1.19.4.

#### A.9.1.55.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier with frame structure 3 and activated, and Cell3 is the neighboring cell on the same secondary component carrier of Cell2. The test parameters are given in Table A.9.1.55.2-1. The DMTC configuration for Cell2 and Cell3 is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.55.2-1: Test parameters for FDD RSRP accuracies of Scell with FS3

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 | 20 |
| DMTC period |  | ms | N/A | 40 | 40 |
| DMTC period offset |  |  | N/A | 10 | 10 |
| Discovery signal occasion duration |  | ms | N/A | 1 | 1 |
| LBT model |  |  | N/A | N/A | [A.3.17] |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 and A.3.1.1.6(R.0 FS3) |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RA Note1 |  |  |  |  |  |
| OCNG_RB Note1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  | Bands FS3_G |  | - | ( for Channel 1 +1dB) |  |
|  |  | dB | -4 | 0.46 Note9 | -5.76 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
|  | Bands FS3_G |  | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
| IoNote3 | Bands FDD_A | 5MHz: dBm/4.5MHz 10MHz: dBm/9MHz 20MHz: dBm/18MHz | -87.76 +10log(NRB,c/50) | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -86.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 +5.33dBNote9 +10log(NRB channel2 / NRB channel 1)) | (Io for Channel 1 +5.33dB +10log(NRB channel2 / NRB channel 1)) |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: The value is corresponding to DRS transmission through LBT operation in Cell3. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.55.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 3 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.19.4.

### A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell

#### A.9.1.56.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD intra frequency RSRP absolute and relative measurement accuracies in carrier aggregation with frame structure 3 in the configured DMTC occasion are within the specified limits. This test will verify the absolute RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.19.2, and the relative RSRP accuracy requirement of the secondary component carrier defined in clause 9.1.19.2. The test will also verify the primary and secondary component carrier relative RSRP accuracy requirement defined in Clause 9.1.19.4.

#### A.9.1.56.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier with frame structure 3 and activated, and Cell3 is the neighboring cell on the same secondary component carrier of Cell2. The test parameters are given in Table A.9.1.56.2-1. The DMTC configuration for Cell2 and Cell3 is provided to the UE in the measDS-Config before the start of the test.

Table A.9.1.56.2-1: Test parameters for TDD RSRP accuracies of Scell with FS3

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 | 20 |
| DMTC period |  | ms | N/A | 40 | 40 |
| DMTC period offset |  |  | N/A | 10 | 10 |
| Discovery signal occasion duration |  | ms | N/A | 1 | 1 |
| LBT model |  |  | N/A | N/A | A.3.17 |
| Special subframe configurationNote1 |  |  | 6 | N/A | N/A |
| Uplink/downlink configurationNote1 |  |  | 1 | N/A | N/A |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RA Note2 |  |  |  |  |  |
| OCNG_RB Note2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | - |  |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  | Bands FS3_G |  | - | ( for Channel 1 +1dB) |  |
|  |  | dB | -4 | 0.46 Note9 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | - |  |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
|  | Bands FS3_G |  | - | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
| IoNote4 | Bands TDD_A | 5MHz: dBm/4.5MHz 10MHz: dBm/9MHz 20MHz: dBm/18MHz | -87.76 +10log(NRB,c/50) | - |  |
|  | Bands TDD_C |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 +5.33dBNote9 +10log(NRB channel2 / NRB channel 1)) | (Io for Channel 1 +5.33dB +10log(NRB channel2 / NRB channel 1)) |
|  |  | dB | -4 | 3 | -1 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: The value is corresponding to DRS transmission through LBT operation in Cell3. |  |  |  |  |  |

#### A.9.1.56.3	Test Requirements

In the test, the performance of RSRP measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 3 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2

-	The relative accuracy of inter-frequency RSRP measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.19.4.

### A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.57.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 and 9.1.21.4 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.57.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.57.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.57.2-1: FD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.22 FDD | - | R.22 FDD | - | R.22 FDD | - |
| PDSCH allocation |  |  | Follows R.22 FDD | - | Follows R.22 FDD | - | Follows R.22 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.18 FDD |  | R.18 FDD |  | R.18 FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| bNote2 | Bands FDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -106.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -105.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -105 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -104 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -103.5 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note3 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -118.5 | -120.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands FDD-M1_D |  |  |  |  |  | -117.5 | -119.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -117 | -119 |
|  | Bands FDD-M1_G |  |  |  |  |  | -116 | -118 |
|  | Bands FDD-M1_H |  |  |  |  |  | -115.5 | -117.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -78.29 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -77.29 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -76.79 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -75.79 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -75.29 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.57.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3 and 9.1.21.4.

### A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB

#### A.9.1.57A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 and 9.1.21.4 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.57A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.57A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.57A.2-1: FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.30 FDD | - | R.30 FDD | - | R.30 FDD | - |
| PDSCH allocation |  |  | Follows R.30 FDD | - | Follows R.30 FDD | - | Follows R.30 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.26 FDD |  | R.26 FDD |  | R.26 FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -96 |  | -79 |  | -97.5 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note3 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -108 | -110 | -88 | -90 | -112.5 | -114.5 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.80 |  | -50.80 |  | -72.30 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.57A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3 and 9.1.21.4.

### A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.58.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 and 9.1.21.4 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.58.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.58.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.58.2-1: HD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.12 HD-FDD | - | R.12 HD-FDD | - | R.12 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.12 HD-FDD | - | Follows R.12 HD-FDD | - | Follows R.12 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.8 HD-FDD |  | R.8 HD-FDD |  | R.8 HD-FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -106.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -105.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -105 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -104 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -103.5 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note3 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands FDD-M1_B |  |  |  |  |  | -118.5 | -120.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands FDD-M1_D |  |  |  |  |  | -117.5 | -119.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -117 | -119 |
|  | Bands FDD-M1_G |  |  |  |  |  | -116 | -118 |
|  | Bands FDD-M1_H |  |  |  |  |  | -115.5 | -117.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -78.29 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -77.29 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -76.79 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -75.79 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -75.29 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.58.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3 and 9.1.21.4.

### A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB

#### A.9.1.58A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 and 9.1.21.4 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.58A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.58A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.58.2-1: HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.20 HD-FDD | - | R.20 HD-FDD | - | R.20 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.20 HD-FDD | - | Follows R.20 HD-FDD | - | Follows R.20 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 HD-FDD |  | R.16 HD-FDD |  | R.16 HD-FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -96 |  | -79 |  | -97.5 |  |
|  |  | dB | -12 | -12 | -14 | -12 | -12 | -14 |
| Note3 |  | dB | -12.17 | -12.17 | -14.27 | -12.17 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -108 | -110 | -88 | -108 | -110 | -88 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.80 |  | -50.80 |  | -72.30 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.58A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3 and 9.1.21.4.

### A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.59.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 and 9.1.21.4 for TDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.59.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.59.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.59.2-1: TDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.18 TDD | - | R.18 TDD | - | R.18 TDD | - |
| PDSCH allocation |  |  | Follows R.18 TDD | - | Follows R.18 TDD | - | Follows R.18 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 TDD |  | R.16 TDD |  | R.16 TDD |  |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -105 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note4 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote4 | Bands TDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands TDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands TDD-M1_E |  |  |  |  |  | -117 | -119 |
| IoNote4 | Bands TDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -76.79 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.59.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3 and 9.1.21.4.

### A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell

#### A.9.1.60.1	Test Purpose and Environment

The purpose of this test is to verify that CSI- RSRP measurement accuracy is within the specified limits. This test will verify the absolute intra-frequency CSI-RSRP accuracy requirements of the SCells defined in Section 9.1.18.4.4 for intra-frequency measurements under FS3, and the relative intra-frequency CSI-RSRP accuracy requirements between SCells defined in Section 9.1.18.4.5.

#### A.9.1.60.2	Test parameters

In this set of cases Cell 1 is PCell on the primary component carrier, Cell 2 using FS3 is SCell on the secondary component carrier and activated, and Cell 3 using FS3 is the neighbouring cell on the secondary component carrier. The test parameters are given in Table A.9.1.60.2-1. Intra-frequency measurements are supported by a DMTC configuration.

A.9.1.60.2-1: CSI-RSRP carrier aggregation test parameters with FDD PCell and FS3 SCells

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| Frame structure |  |  | FDD | FS3 | FS3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Uplink/downlink configuration |  |  | - | Note10 | Note10 |
| Time alignment error relative to cell 1 Note 11 |  |  | - | TAE | - |
| DMTC period |  | ms | - | 40 | 40 |
| DMTC period offset |  | ms | - | 10 | 10 |
| Discovery signal occasion duration |  | ms | - | 1 | 1 |
| CSI-RS resource configuration |  |  | - | 1 | 6 |
| CSI-RS subframe offset |  | ms | - | 0 | 0 |
| CSI-RS individual offset[2] |  | dB | - | 0 | 0 |
| LBT model |  |  | - | - | A.3.17 |
| Measurement bandwidth |  |  | 5 MHz: 10—15 10 MHz: 22—27 20 MHz: 47—52 | 47—52 | 47—52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5 MHz: R.5 FDD 10 MHz: R.0 FDD 20 MHz: R.4 FDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5 MHz: 7-17 10 MHz: 13-36 20 MHz: 38-61 | 38—61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2 |  |  | 5 MHz: OP.15 FDD 10 MHz: OP.1 FDD 20 MHz: OP.11 FDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | - | - |
|  | Bands FDD_B1, FDD_B2 Note 12 |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  | Bands FS3_G |  | - | (for Channel 1 + 1 dB) |  |
| CRS |  | dB | -4 | 0.46 | -5.76 |
| CSI-RSNote3 |  | dB | - | 6.46 | 0.24 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | - | - |
|  | Bands FDD_B1, FDD_B2 Note 12 |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
|  | Bands FS3_G |  | - | RSRP for Cell 1 + 8 dB) | RSRP for Cell 1 + 4 dB) |
| CSI-RSRPNote3 | Bands FS3_G | dBm/15 kHz | - | RSRP for Cell 2 + 6 dB) | RSRP for Cell 3 + 6 dB) |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | - | - |
|  | Bands FDD_B1, FDD_B2 Note 12 |  | -87.26+10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 + 5.33 dB +10log (NRB channel2 / NRB channel 1)) |  |
| CRS |  | dB | -4 | 3 | -1 |
| CSI-RS |  | dB | - | 9 | 5 |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS Es/Iot, RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Void Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 10: Downlink only configuration. Note 11: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 12: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.1.60.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 3 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.5.

### A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell

#### A.9.1.61.1	Test Purpose and Environment

The purpose of this test is to verify that CSI- RSRP measurement accuracy is within the specified limits. This test will verify the absolute intra-frequency CSI-RSRP accuracy requirements of the SCells defined in Section 9.1.18.4.4 for intra-frequency measurements under FS3, and the relative intra-frequency CSI-RSRP accuracy requirements between SCells defined in Section 9.1.18.4.5.

#### A.9.1.61.2	Test parameters

In this set of cases Cell 1 is PCell on the primary component carrier, Cell 2 using FS3 is SCell on the secondary component carrier and activated, and Cell 3 using FS3 is the neighboring cell on the secondary component carrier. The test parameters are given in Table A.9.1.61.2-1. The intra-frequency measurements are supported by a DMTC configuration.

A.9.1.61.2-1: CSI-RSRP carrier aggregation test parameters with TDD PCell and FS3 SCells

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| Frame structure |  |  | TDD | FS3 | FS3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Special subframe configurationNote1 |  |  | 6 | Note10 | Note10 |
| Uplink/downlink configurationNote1 |  |  | 1 | Note10 | Note10 |
| Time alignment error relative to cell 1 Note 11 |  |  | - | TAE | - |
| DMTC period |  | ms | - | 40 | 40 |
| DMTC period offset |  | ms | - | 10 | 10 |
| Discovery signal occasion duration |  | ms | - | 1 | 1 |
| CSI-RS resource configuration |  |  | - | 1 | 6 |
| CSI-RS subframe offset |  | ms | - | 0 | 0 |
| CSI-RS individual offset[2] |  | dB | - | 0 | 0 |
| LBT model |  |  | - | - | A.3.17 |
| Measurement bandwidth |  |  | 5 MHz: 10—15 10 MHz: 22—27 20 MHz: 47—52 | 47—52 | 47—52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5 MHz: 7-17 10 MHz: 13-36 20 MHz: 38-61 | 38—61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2.2 |  |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| Note2 | Bands TDD_A | dBm/15 kHz | -117 | - | - |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  | Bands FS3_G |  | - | (for Channel 1 + 1 dB) |  |
| CRS |  | dB | -4 | 0.46 | -5.76 |
| CSI-RSNote3 |  | dB | - | 6.46 | 0.24 |
| RSRPNote3 | Bands TDD_A | dBm/15 kHz | -121 | - | - |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
|  | Bands FS3_G |  | - | RSRP for Cell 1 + 8 dB) | RSRP for Cell 1 + 4 dB) |
| CSI-RSRPNote3 | Bands FS3_G | dBm/15 kHz | - | RSRP for Cell 2 + 6 dB) | RSRP for Cell 3 + 6 dB) |
| IoNote3 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | - | - |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 + 5.33 dB +10log (NRB channel2 / NRB channel 1)) |  |
| CRS |  | dB | -4 | 3 | -1 |
| CSI-RS |  | dB | - | 9 | 5 |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS Es/Iot, RSRP, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: Void Note 7: Void Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note 10: Downlink only configuration. Note 11: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.9.1.61.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 3 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.5.

### A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA

#### A.9.1.62.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.9 and 9.1.21.10 for FD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.62.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.62.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.62.2-1: FD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.20 FDD | - | R.20 FDD | - |
| PDSCH allocation |  |  | Follows R.20 FDD | - | Follows R.20 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 FDD | R.16 FDD | R.16 FDD | R.16 FDD |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -116 |
|  | Bands FDD-M1_B |  |  |  |  | -115.5 |
|  | Bands FDD-M1_C |  |  |  |  | -115 |
|  | Bands FDD-M1_D |  |  |  |  | -114.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -114 |
|  | Bands FDD-M1_G |  |  |  |  | -113 |
|  | Bands FDD-M1_H |  |  |  |  | -112.5 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -120 |
|  | Bands FDD-M1_B |  |  |  |  | -119.5 |
|  | Bands FDD-M1_C |  |  |  |  | -119 |
|  | Bands FDD-M1_D |  |  |  |  | -118.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -118 |
|  | Bands FDD-M1_G |  |  |  |  | -117 |
|  | Bands FDD-M1_H |  |  |  |  | -116.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.62.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.9 and 9.1.21.10.

### A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA

#### A.9.1.53.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in section 9.1.21.9 and 9.1.21.10 for HD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.53.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.63.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.63.2-1: HD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.10 HD-FDD | - | R.10 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.6 HD-FDD | R.6 HD-FDD | R.6 HD-FDD | R.6 HD-FDD |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -116 |
|  | Bands FDD-M1_B |  |  |  |  | -115.5 |
|  | Bands FDD-M1_C |  |  |  |  | -115 |
|  | Bands FDD-M1_D |  |  |  |  | -114.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -114 |
|  | Bands FDD-M1_G |  |  |  |  | -113 |
|  | Bands FDD-M1_H |  |  |  |  | -112.5 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -120 |
|  | Bands FDD-M1_B |  |  |  |  | -119.5 |
|  | Bands FDD-M1_C |  |  |  |  | -119 |
|  | Bands FDD-M1_D |  |  |  |  | -118.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -118 |
|  | Bands FDD-M1_G |  |  |  |  | -117 |
|  | Bands FDD-M1_H |  |  |  |  | -116.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.63.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA

#### A.9.1.64.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.9 and 9.1.21.10 for TDD inter frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.64.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.64.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.64.2-1: TDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Special subframe configuration |  |  | 6 | 6 | 6 | 6 |
| Uplink/downlink configuration |  |  | 1 | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.16 TDD | - | R.16 TDD | - |
| PDSCH allocation |  |  | Follows R.16 TDD | - | Follows R.16 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 TDD | R.14 TDD | R.14 TDD | R.14 TDD |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands TDD-M1_A | dBm/15 kHz | -88.65 | -88.65 | ( for Channel 2 +8dB) | -116 |
|  | Bands TDD-M1_C |  |  |  |  | -115 |
|  | Bands TDD-M1_E |  |  |  |  | -114 |
|  |  | dB | 10 | 10 | 13 | -4 |
| RSRPNote3 | Bands TDD-M1_A | dBm/15 kHz | -78.65 | -78.65 | (RSRP for Cell 2 +25dB) | -120 |
|  | Bands TDD-M1_C |  |  |  |  | -119 |
|  | Bands TDD-M1_E |  |  |  |  | -118 |
| IoNote3 | Bands TDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | (Io for Channel 2 +19.75dB) | -86.76 |
|  | Bands TDD-M1_C |  |  |  |  | -85.76 |
|  | Bands TDD-M1_E |  |  |  |  | -84.76 |
|  |  | dB | 10 | 10 | 13 | -4 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.64.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.9 and 9.1.21.10.

### A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB

#### A.9.1.65.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.11 and 9.1.21.12 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.65.2	Test parameters

Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.65.2-1 and A.9.1.65.2-2. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.65.2-1: FD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 10MHz cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.22 FDD | - | R.22 FDD | - |
| PDSCH allocation |  |  | Follows R.22 FDD | - | Follows R.22 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.18 FDD | R.18 FDD | R.18 FDD | R.18 FDD |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -78.5 | -78.5 | -98.5 | -108 |
|  | Bands FDD-M1_B |  |  |  |  | -107.5 |
|  | Bands FDD-M1_C |  |  |  |  | -107 |
|  | Bands FDD-M1_D |  |  |  |  | -106.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -106 |
|  | Bands FDD-M1_G |  |  |  |  | -105 |
|  | Bands FDD-M1_H |  |  |  |  | -104.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -90.5 | -90.5 | -110.5 | -120 |
|  | Bands FDD-M1_B |  |  |  |  | -119.5 |
|  | Bands FDD-M1_C |  |  |  |  | -119 |
|  | Bands FDD-M1_D |  |  |  |  | -118.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -118 |
|  | Bands FDD-M1_G |  |  |  |  | -117 |
|  | Bands FDD-M1_H |  |  |  |  | -116.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table A.9.1.65.2-2: FD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 5MHz cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 5 | 5 | 5 | 5 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.30 FDD | - | R.30 FDD z | - |
| PDSCH allocation |  |  | Follow R.30 FDD | - | Follow R.30 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.26 FDD | R.26 FDD | R.26 FDD | R.26 FDD |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -75.5 | -75.5 | -95.5 | -105 |
|  | Bands FDD-M1_B |  |  |  |  | -104.5 |
|  | Bands FDD-M1_C |  |  |  |  | -104 |
|  | Bands FDD-M1_D |  |  |  |  | -103.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -103 |
|  | Bands FDD-M1_G |  |  |  |  | -102 |
|  | Bands FDD-M1_H |  |  |  |  | -101.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -87.5 | -87.5 | -107.5 | -117 |
|  | Bands FDD-M1_B |  |  |  |  | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  | -116 |
|  | Bands FDD-M1_D |  |  |  |  | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -115 |
|  | Bands FDD-M1_G |  |  |  |  | -114 |
|  | Bands FDD-M1_H |  |  |  |  | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/4.5 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.65.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.11 and 9.1.21.12.

### A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB

#### A.9.1.66.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.11 and 9.1.21.12 for HD-FDD inter frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.66.2	Test parameters

Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.66.2-1 and A.9.1.66.2-2. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.66.2-1: HD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 10Mhz Cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.12 HD-FDD | - | R.12 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.12 HD-FDD | - | Follows R.12 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.8 HD-FDD D | R.8 HD-FDD | R.8 HD-FDD | R.8 HD-FDD |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -78.5 | -78.5 | -98.5 | -108 |
|  | Bands FDD-M1_B |  |  |  |  | -107.5 |
|  | Bands FDD-M1_C |  |  |  |  | -107 |
|  | Bands FDD-M1_D |  |  |  |  | -106.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -106 |
|  | Bands FDD-M1_G |  |  |  |  | -105 |
|  | Bands FDD-M1_H |  |  |  |  | -104.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -90.5 | -90.5 | -110.5 | -120 |
|  | Bands FDD-M1_B |  |  |  |  | -119.5 |
|  | Bands FDD-M1_C |  |  |  |  | -119 |
|  | Bands FDD-M1_D |  |  |  |  | -118.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -118 |
|  | Bands FDD-M1_G |  |  |  |  | -117 |
|  | Bands FDD-M1_H |  |  |  |  | -116.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table A.9.1.66.2-2: HD-FDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB for 5MHz Cell BW

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 5 | 5 | 5 | 5 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.13 HD-FDD | - | R.13 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.13 HD-FDD | - | Follows R.13 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.9 HD-FDD D | R.9 HD-FDD | R.9 HD-FDD | R.9 HD-FDD |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -75.5 | -75.5 | -95.5 | -105 |
|  | Bands FDD-M1_B |  |  |  |  | -104.5 |
|  | Bands FDD-M1_C |  |  |  |  | -104 |
|  | Bands FDD-M1_D |  |  |  |  | -103.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -103 |
|  | Bands FDD-M1_G |  |  |  |  | -102 |
|  | Bands FDD-M1_H |  |  |  |  | -101.5 |
|  |  | dB | -12 | -12 | -12 | -12 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -87.5 | -87.5 | -107.5 | -117 |
|  | Bands FDD-M1_B |  |  |  |  | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  | -116 |
|  | Bands FDD-M1_D |  |  |  |  | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -115 |
|  | Bands FDD-M1_G |  |  |  |  | -114 |
|  | Bands FDD-M1_H |  |  |  |  | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/4.5 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | Bands FDD-M1_B |  |  |  |  | -86.26 |
|  | Bands FDD-M1_C |  |  |  |  | -85.76 |
|  | Bands FDD-M1_D |  |  |  |  | -85.26 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  | -84.76 |
|  | Bands FDD-M1_G |  |  |  |  | -83.76 |
|  | Bands FDD-M1_H |  |  |  |  | -83.26 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.66.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.11 and 9.1.21.12.

### A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB

#### A.9.1.67.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.11 and 9.1.21.12 for TDD inter frequency RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.67.2	Test parameters

Both absolute and relative accuracy of RSRP inter frequency measurements are tested by using the parameters in Table A.9.1.67.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

Table A.9.1.67.2-1: TDD RSRP Inter frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - | 0 | - |
| Special subframe configuration |  |  | 6 | 6 | 6 | 6 |
| Uplink/downlink configuration |  |  | 1 | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | R.18 TDD | - | R.18 TDD | - |
| PDSCH allocation |  |  | Follows R.18 TDD | - | Follows R.18 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 TDD | R.16 TDD | R.16 TDD | R.16 TDD |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |
| Note2 | Bands TDD-M1_A | dBm/15 kHz | -78.5 | -78.5 | -98.5 | -108 |
|  | Bands TDD-M1_C |  |  |  |  | -107 |
|  | Bands TDD-M1_E |  |  |  |  | -106 |
|  |  | dB | -12 | 12 | -12 | -12 |
| RSRPNote3 | Bands TDD-M1_A | dBm/15 kHz | -90.5 | -90.5 | -110.5 | -120 |
|  | Bands TDD-M1_C |  |  |  |  | -119 |
|  | Bands TDD-M1_E |  |  |  |  | -118 |
| IoNote3 | Bands TDD-M1_A | dBm/9 MHz | -50.45 | -50.45 | -70.45 | -86.76 |
|  | Bands TDD-M1_C |  |  |  |  | -85.76 |
|  | Bands TDD-M1_E |  |  |  |  | -84.76 |
|  |  | dB | -12 | -12 | -12 | -12 |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.1.67.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.11 and 9.1.21.12.

### A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.1.68.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.1.37, A.9.1.38, A.9.1.39, A.9.1.40, does not need to be tested in the generic duplex-mode test case A.9.1.68.

#### A.9.1.68.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2 and cell 4 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. Cell 3 and cell 5 are neighbouring cells on secondary component carriers SCC1 and SCC2 respectively.  The test parameters are given in Table A.9.1.68.2-1.

Table A.9.1.68.2-1: 3 Downlink RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -117 | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |
|  | Bands FDD_A |  | -117 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |
|  | Bands FDD_A |  | -121 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | (Io for Channel 1+5.33dB+10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1+5.33dB+10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -84.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -84.26+10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.1.68.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 7 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.1.69.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.1.44, A.9.1.45, A.9.1.46, A.9.1.47, does not need to be tested in the generic duplex-mode test case A.9.1.69.

#### A.9.1.69.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4 and cell 6 are activated SCells on secondary component carriers SCC1, SCC2 and SCC3 respectively. Cell 3, cell 5 and cell 7 are neighbouring cells on secondary component carriers SCC1, SCC2 and SCC3 respectively.  The test parameters are given in Table A.9.1.44.2-1.

Table A.9.1.69.2-1: 4 Downlink RSRP carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  | 4 |  |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 |  | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |  |  |
|  | Bands FDD_A |  | -117 |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |  |  |
|  | Bands FDD_A |  | -121 |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel4 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - | TAE | - |
| Time alignment error relative to cell 4Note 8 |  |  | - | - | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |

#### A.9.1.69.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 10 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.1.70.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.1.48, A.9.1.49, A.9.1.50, A.9.1.51, does not need to be tested in the generic duplex-mode test case A.9.1.70.

#### A.9.1.70.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell 6 and cell 8 are activated SCells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively. Cell 3, cell 5, cell 7 and cell 9 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3 and SCC4 respectively.  The test parameters are given in Table A.9.1.70.2-1, Table A.9.1.70.2-2 and Table A.9.1.70.2-3.

Table A.9.1.70.2-1: 5 Downlink RSRP carrier aggregation test parameters for cell 1, cell 2, cell 3, cell 4 and cell 5

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  | Cell 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  | 2 |  |  |  | 3 |  |  |  |
| Duplex mode |  |  |  |  | FDD or TDD |  | FDD or TDD |  |  |  | FDD or TDD |  |  |  |
| BWchannel |  |  | MHz |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| TDD Special subframe configurationNote1 |  |  |  |  | 6 |  | 6 |  |  |  | 6 |  |  |  |
| TDD Uplink/downlink configurationNote1 |  |  |  |  | 1 |  | 1 |  |  |  | 1 |  |  |  |
| Measurement bandwidth |  |  |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  | - |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  | - |  |
| PDSCH allocation |  |  |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  | - |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |
| OCNG Patterns defined in A.3.2. |  |  |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |  |
| PBCH_RA |  |  | dB |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz |  | -117 |  | ( for Channel 1 +1dB) |  |  |  | ( for Channel 1 +1dB) |  |  |  |
|  | Bands TDD_C |  |  |  | -116 |  |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  | -115 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  |  |  | -117 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -116.5 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  | -116 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  | -115.5 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  | -115 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  | -114 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  | -113.5 |  |  |  |  |  |  |  |  |  |
|  |  |  | dB |  | -4 |  | 3 |  | -1 |  | 3 |  | -1 |  |
|  |  |  | dB |  | -4 |  | 0.46 |  | -5.76 |  | 0.46 |  | -5.76 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz |  | -121 |  | (RSRP for Cell 1 +8dB) |  | (RSRP for Cell 1 +4dB) |  | (RSRP for Cell 1 +8dB) |  | (RSRP for Cell 1 +4dB) |  |
|  | Bands TDD_C |  |  |  | -120 |  |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  | -119 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  |  |  | -121 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -120.5 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  | -120 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  | -119.5 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  | -119 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  | -118 |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  | -117.5 |  |  |  |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A |  | dBm/ BWchannel |  | -87.76+10log(NRB,c/50) |  | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  |  |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |  |  |
|  | Bands TDD_C |  |  |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  | -85.76+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  |  |  | -87.76+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -87.26+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  | -86.76+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  | -86.26+10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  | -85.76 +10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  | -84.76 +10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  | -84.26 +10log(NRB,c/50) |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | - |  | AWGN |  | AWGN |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | - |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |  |
| Timing offset to cell 1 |  |  | s |  | - |  | 0 |  | 3 |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 8 |  |  |  |  | - |  | TAE |  | - |  | TAE |  | - |  |
| Time alignment error relative to cell 2Note 8 |  |  |  |  | - |  | - |  | - |  | TAE |  | - |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.9.1.70.2-2: 5 Downlink RSRP carrier aggregation test parameters for cell 6 and cell 7

| Parameter |  | Unit | Cell 6 | Cell 7 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  |
| Duplex mode |  |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table A.9.1.70.2-3: 5 Downlink PCell in FDD RSRP carrier aggregation test parameters for cell 8 and cell 9

| Parameter |  | Unit | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 5 |  |
| Duplex mode |  |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 6 Note 8 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.70.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 13 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.1.71.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.71.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell 6, cell 8 and cell 10 are activated SCells on secondary component carriers SCC1, SCC2, SCC3, SCC4 and SCC5 respectively. Cell 3, cell 5, cell 7, cell 9 and cell 11 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3, SCC4 and SCC5 respectively. For testing the requirement related to a given SCell, the presence of at least the respective intra-frequency neigbouring cell (i.e. on the same secondary component carrier) is required. The test parameters are given in Table A.9.1.71.2-1, Table A.9.1.71.2-2 and Table A.9.1.71.2-3.

Table A.9.1.71.2-1: 6 Downlink RSRP carrier aggregation test parameters for cell 1, cell 2, cell 3, cell 4 and cell 5

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |
|  | Bands FDD_A |  | -117 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |
|  | Bands FDD_A |  | -121 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

Table A.9.1.71.2-2: 6 Downlink RSRP carrier aggregation test parameters for cell 6, cell 7, cell 8 and cell 9

| Parameter |  | Unit | Cell 6 | Cell 7 | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  | 5 |  |
| Duplex mode |  |  | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
|  |  | dB | 3 | -1 | 3 | -1 |
|  |  | dB | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz |  |  |  |  |  |  |

Table A.9.1.71.2-3: 6 Downlink RSRP carrier aggregation test parameters for cell 10 and cell 11

| Parameter |  | Unit | Cell 10 | Cell 11 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 6 |  |
| Duplex mode |  |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
|  |  | dB | 3 | -1 |
|  |  | dB | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 6 Note 8 |  |  | TAE |  |
| Time alignment error relative to cell 8 Note 8 |  |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.1.71.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 13 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 10 on SCC5 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 11 relative to Cell 10 on SCC5 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC5 and the primary component carriers for Cell 10 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.1.72.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP absolute and relative accuracy requirements in carrier aggregation are within the specified limits. This test will verify the absolute RSRP accuracy requirements of the primary component carrier defined in clause 9.1.11.1, the absolute RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2, and the relative RSRP accuracy requirements of the secondary component carriers defined in clause 9.1.11.2. The test will also verify the primary and secondary component carriers’ relative RSRP accuracy requirement defined in Clause 9.1.11.3.

#### A.9.1.72.2	Test parameters

In this set of test cases cell 1 is PCell on the primary component carrier, and cell 2, cell 4, cell 6, cell 8, cell 10 and cell12 are activated SCells on secondary component carriers SCC1, SCC2, SCC3, SCC4, SCC5 and SCC6 respectively. Cell 3, cell 5, cell 7, cell 9, cell 11 and cell 13 are neighbouring cells on secondary component carriers SCC1, SCC2, SCC3, SCC4, SCC5 and SCC6 respectively. For testing the requirement related to a given SCell, the presence of at least the respective intra-frequency neigbouring cell (i.e. on the same secondary component carrier) is required. The test parameters are given in Table A.9.1.72.2-1, Table A.9.1.72.2-2 and Table A.9.1.72.2-3.

Table A.9.1.72.2-1: 7 Downlink RSRP carrier aggregation test parameters for cell 1, cell 2, cell 3, cell 4 and cell 5

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 3 |  |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2. |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -117 | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  | -116 |  |  |  |  |
|  | Bands TDD_E |  | -115 |  |  |  |  |
|  | Bands FDD_A |  | -117 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -116.5 |  |  |  |  |
|  | Bands FDD_C |  | -116 |  |  |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -115 |  |  |  |  |
|  | Bands FDD_G |  | -114 |  |  |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |  |  |
|  |  | dB | -4 | 3 | -1 | 3 | -1 |
|  |  | dB | -4 | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -121 | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  | -120 |  |  |  |  |
|  | Bands TDD_E |  | -119 |  |  |  |  |
|  | Bands FDD_A |  | -121 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -120.5 |  |  |  |  |
|  | Bands FDD_C |  | -120 |  |  |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -119 |  |  |  |  |
|  | Bands FDD_G |  | -118 |  |  |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | (Io for Channel 1 +5.33dB +10log (NRB channel2 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_A |  | -87.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -87.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  | -85.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | - | TAE | - | TAE | - |
| Time alignment error relative to cell 2Note 8 |  |  | - | - | - | TAE | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

Table A.9.1.72.2-2: 7 Downlink RSRP carrier aggregation test parameters for cell 6, cell 7, cell8 and cell 9

| Parameter |  | Unit | Cell 6 | Cell 7 | Cell 8 | Cell 9 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |  | 5 |  |
| Duplex mode |  |  | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
|  |  | dB | 3 | -1 | 3 | -1 |
|  |  | dB | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

Table A.9.1.72.2-3: 7 Downlink RSRP carrier aggregation test parameters for cell 10, cell 11, cell 12 and cell 13

| Parameter |  | Unit | Cell 10 | Cell 11 | Cell 12 | Cell 13 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 6 |  | 7 |  |
| Duplex mode |  |  | FDD or TDD |  | FDD or TDD |  |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| TDD Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | ( for Channel 1 +1dB) |  | ( for Channel 1 +1dB) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
|  |  | dB | 3 | -1 | 3 | -1 |
|  |  | dB | 0.46 | -5.76 | 0.46 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) | (RSRP for Cell 1 +8dB) | (RSRP for Cell 1 +4dB) |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  | (Io for Channel 1 +5.33dB +10log (NRB channel3 / NRB channel 1)) |  |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 3 | 0 | 3 |
| Time alignment error relative to cell 1 Note 8 |  |  | TAE | - | TAE | - |
| Time alignment error relative to cell 2 Note 8 |  |  | TAE |  | TAE |  |
| Time alignment error relative to cell 4 Note 8 |  |  | TAE |  | TAE |  |
| Time alignment error relative to cell 6 Note 8 |  |  | TAE |  | TAE |  |
| Time alignment error relative to cell 8 Note 8 |  |  | TAE |  | TAE |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.72.3	Test Requirements

In the test, the performance of RSRP measurements is verified from the following 13 perspectives:

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 10 on SCC5 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRP measurements for Cell 12 on SCC6 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 3 relative to Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 5 relative to Cell 4 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 7 relative to Cell 6 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 9 relative to Cell 8 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 11 relative to Cell 10 on SCC5 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of intra-frequency RSRP measurements for Cell 13 relative to Cell 12 on SCC6 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRP measurements between SCC1 and the primary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC2 and the primary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC3 and the primary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC4 and the primary component carriers for Cell 8 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC5 and the primary component carriers for Cell 10 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRP measurements between SCC5 and the primary component carriers for Cell 12 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements

#### A.9.1.73.1	Test Purpose and Environment

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2B.2 for FDD intra frequency measurements.

#### A.9.1.73.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Only absolute accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.1.2-1. In all test cases, Cell 1 is the serving cell and Cell 2 the target cell.

Table A.9.1.73.2-1: RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | [-96] |  | [-88] |  | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | [2.21] | [-3.97] | [1.24] | [-3.97] | [1.24] | [-3.97] |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | [-90] | [-93] | [-82] | [-85] | [-110] | [-113] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | [-109.5] | [-112.5] |
|  | Bands FDD_C |  |  |  |  |  | [-109] | [-112] |
|  | Bands FDD_D |  |  |  |  |  | [-108.5] | [-111.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | [-108] | [-111] |
|  | Bands FDD_G Note 7 |  |  |  |  |  | [-107] | [-110] |
|  | Bands FDD_H |  |  |  |  |  | [-106.5] | [-109.5] |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | [-59.78] |  | [-51.78] |  | [-79.78] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | [-79.28] |  |
|  | Bands FDD_C |  |  |  |  |  | [-78.78] |  |
|  | Bands FDD_D |  |  |  |  |  | [-78.28] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | [-77.78] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | [-76.78] |  |
|  | Bands FDD_H |  |  |  |  |  | [-76.28] |  |
|  |  | dB | [6] | [3] | [6] | [3] | [6] | [3] |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.73.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.2B.2.

### A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier

#### A.9.1.74.1	Test Purpose and Environment

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3B.2 for FDD—FDD inter frequency measurements.

#### A.9.1.74.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Absolute accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.74.2-1. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

Table A.9.1.74.2-1: RSRP FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | [-95] | [-95] | ( for Channel 2 +8dB) | [-117] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-116.5] |
|  | Bands FDD_C |  |  |  |  | [-116] |
|  | Bands FDD_D |  |  |  |  | [-115.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-115] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-114] |
|  | Bands FDD_H |  |  |  |  | [-113.5] |
|  |  | dB | [10] | [10] | 13 | [-4] |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | [-85] | [-85] | (RSRP for Cell 2 +25dB) | [-121] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-120.5] |
|  | Bands FDD_C |  |  |  |  | [-120] |
|  | Bands FDD_D |  |  |  |  | [-119.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-119] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-118] |
|  | Bands FDD_H |  |  |  |  | [-117.5] |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | [-56.81] | [-56.81] | (Io for Channel 2 +19.75dB) | [-87.76] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-87.26] |
|  | Bands FDD_C |  |  |  |  | [-86.76] |
|  | Bands FDD_D |  |  |  |  | [-86.26] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-85.76] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-84.76] |
|  | Bands FDD_H |  |  |  |  | [-84.26] |
|  |  | dB | [10] | [10] | 13 | [-4] |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.74.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3B.2.

### A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier

#### A.9.1.75.1	Test Purpose and Environment

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.3B.3 for FDD—FDD inter frequency measurements.

#### A.9.1.75.2	Test parameters

In this set of test cases the cells are on different carrier frequencies. Absolute accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.1.75.2-1. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

Table A.9.1.75.2-1: RSRP FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 | 10 | 10 |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | [-95] | [-95] | ( for Channel 2 +8dB) | [-117] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-116.5] |
|  | Bands FDD_C |  |  |  |  | [-116] |
|  | Bands FDD_D |  |  |  |  | [-115.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-115] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-114] |
|  | Bands FDD_H |  |  |  |  | [-113.5] |
|  |  | dB | [10] | [10] | 13 | [-4] |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | [-85] | [-85] | (RSRP for Cell 2 +25dB) | [-121] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-120.5] |
|  | Bands FDD_C |  |  |  |  | [-120] |
|  | Bands FDD_D |  |  |  |  | [-119.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-119] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-118] |
|  | Bands FDD_H |  |  |  |  | [-117.5] |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | [-56.81] | [-56.81] | (Io for Channel 2 +19.75dB) | [-87.76] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | [-87.26] |
|  | Bands FDD_C |  |  |  |  | [-86.76] |
|  | Bands FDD_D |  |  |  |  | [-86.26] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | [-85.76] |
|  | Bands FDD_G Note 7 |  |  |  |  | [-84.76] |
|  | Bands FDD_H |  |  |  |  | [-84.26] |
|  |  | dB | [10] | [10] | 13 | [-4] |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.1.75.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.3B.3.

### A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.76.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 for FD-FDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.76.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.76.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.76.2-1.

Table A.9.1.76.2-1: FD-FDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.20 FDD | - | R.20 FDD | - | R.20 FDD | - |
| PDSCH allocation |  |  | Follows R.20 FDD | - | Follows R.20 FDD | - | Follows R.20 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 FDD |  | R.16 FDD |  | R.16 FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.76.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1.

### A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.77.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 for HD-FDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.77.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.77.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.77.2-1.

Table A.9.1.77.2-1: HD-FDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.10 HD-FDD | - | R.10 HD-FDD | - | R.10 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.6 HD-FDD |  | R.6 HD-FDD |  | R.6 HD-FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.77.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1.

### A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.78.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 for TDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.78.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.78.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.78.2-1.

Table A.9.1.78.2-1: TDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.16 TDD | - | R.16 TDD | - | R.16 TDD | - |
| PDSCH allocation |  |  | Follows R.16 TDD | - | Follows R.16 TDD | - | Follows R.16 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 TDD |  | R.14 TDD |  | R.14 TDD |  |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -114 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands TDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD-M1_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -80.43 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.78.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1.

### A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.79.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 for FD-FDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.79.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.79.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.79.2-1.

Table A.9.1.79.2-1: FD-FDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.22 FDD | - | R.22 FDD | - | R.22 FDD | - |
| PDSCH allocation |  |  | Follows R.22 FDD | - | Follows R.22 FDD | - | Follows R.22 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.18 FDD |  | R.18 FDD |  | R.18 FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| bNote2 | Bands FDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -106.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -105.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -105 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -104 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -103.5 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note3 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -118.5 | -120.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands FDD-M1_D |  |  |  |  |  | -117.5 | -119.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -117 | -119 |
|  | Bands FDD-M1_G |  |  |  |  |  | -116 | -118 |
|  | Bands FDD-M1_H |  |  |  |  |  | -115.5 | -117.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -78.29 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -77.29 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -76.79 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -75.79 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -75.29 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.79.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3.

### A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.80.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 for HD-FDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.80.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.80.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.80.2-1.

Table A.9.1.80.2-1: HD-FDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.12 HD-FDD | - | R.12 HD-FDD | - | R.12 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.12 HD-FDD | - | Follows R.12 HD-FDD | - | Follows R.12 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.8 HD-FDD |  | R.8 HD-FDD |  | R.8 HD-FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -106.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -105.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -105 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -104 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -103.5 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note3 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands FDD-M1_B |  |  |  |  |  | -118.5 | -120.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands FDD-M1_D |  |  |  |  |  | -117.5 | -119.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -117 | -119 |
|  | Bands FDD-M1_G |  |  |  |  |  | -116 | -118 |
|  | Bands FDD-M1_H |  |  |  |  |  | -115.5 | -117.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -78.29 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -77.29 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -76.79 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -75.79 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -75.29 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.80.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3.

### A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB

#### A.9.1.81.1	Test Purpose and Environment

The purpose of this test is to verify that the RSS based RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.3 for TDD intra frequency RSS based RSRP measurements for Cat-M1 UE in CEModeB.

#### A.9.1.78.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Absolute accuracy of RSS based RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.81.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. All the tests contain MPDCCH for UL grant for reporting RSRP.

RSS measurement is enabled for intra-frequency measurement with MeasRSS-DedicatedConfig setup. RSS are transmitted by Cell 1 and Cell 2 in the same time and frequency resources with rss-ConfigCarrierInfo absent in MeasRSS-DedicatedConfig. Other RSS related parameters for Cell 1 and Cell 2 are defined in Table A.9.1.81.2-1.

Table A.9.1.81.2-1: TDD RSS based RSRP Intra frequency test parameters for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.18 TDD | - | R.18 TDD | - | R.18 TDD | - |
| PDSCH allocation |  |  | Follows R.18 TDD | - | Follows R.18 TDD | - | Follows R.18 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 TDD |  | R.16 TDD |  | R.16 TDD |  |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-M1_A | dBm/15 kHz | -99 |  | -79 |  | -107 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -106 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -105 |  |
|  |  | dB | -12 | -14 | -12 | -14 | -12 | -14 |
| Note4 |  | dB | -12.17 | -14.27 | -12.17 | -14.27 | -12.17 | -14.27 |
| RSRPNote4 | Bands TDD-M1_A | dBm/15 kHz | -111 | -113 | -91 | -93 | -119 | -121 |
|  | Bands TDD-M1_C |  |  |  |  |  | -118 | -120 |
|  | Bands TDD-M1_E |  |  |  |  |  | -117 | -119 |
| IoNote4 | Bands TDD-M1_A | dBm/9 MHz | -70.79 |  | -50.79 |  | -78.79 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -77.79 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -76.79 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| RSS duration |  | ms | 8 |  |  |  |  |  |
| RSS frequency location |  |  | RB index corresponding to the lowest RB of the MPDCCH narrowband |  |  |  |  |  |
| RSS perodicity |  | ms | 160 |  |  |  |  |  |
| RSS power boost |  | dB | 0 |  |  |  |  |  |
| RSS time offset |  | ms | 0 |  |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.81.3	Test Requirements

The RSS based RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.3.

## A.9.2	RSRQ

### A.9.2.1	FDD Intra frequency case

#### A.9.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.5.1.

#### A.9.2.1.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.1.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.1.2-1: RSRQ FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -84.76 | -84.76 | -103.85 | -103.85 | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | -1.76 | -1.76 | -4.7 | -4.7 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -81.76 | -81.76 | -106.75 | -106.75 | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -119.5 | -119.5 |
|  | Bands FDD_C |  |  |  |  |  | -119 | -119 |
|  | Bands FDD_D |  |  |  |  |  | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -118 | -118 |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -117 | -117 |
|  | Bands FDD_H |  |  |  |  |  | -116.5 | -116.5 |
| RSRQNote3 | Bands FDD_A Note 8 | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -50 | -50 | -73 | -73 | -85.67 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | -85.17 |  |
|  | Bands FDD_C |  |  |  |  |  | -84.67 |  |
|  | Bands FDD_D |  |  |  |  |  | -84.17 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -83.67 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -82.67 |  |
|  | Bands FDD_H |  |  |  |  |  | -82.17 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.1.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in section 9.1.5.1. The RSRQ measurement accuracy for UE Category 1bis shall fulfil the requirements in section 9.1.5.5.

### A.9.2.2	TDD Intra frequency case

#### A.9.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.5.1.

#### A.9.2.2.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.2.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.2.2-1: RSRQ TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -84.76 | -84.76 | -103.85 | -103.85 | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
|  |  | dB | -1.76 | -1.76 | -4.7 | -4.7 | -5.46 | -5.46 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -81.76 | -81.76 | -106.75 | -106.75 | -120 | -120 |
|  | Bands TDD_C |  |  |  |  |  | -119 | -119 |
|  | Bands TDD_E |  |  |  |  |  | -118 | -118 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -50 | -50 | -73 | -73 | -85.67 |  |
|  | Bands TDD_C |  |  |  |  |  | -84.67 |  |
|  | Bands TDD_E |  |  |  |  |  | -83.67 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.2.2.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in section 9.1.5.1. The RSRQ measurement accuracy for UE Category 1bis shall fulfil the requirements in section 9.1.5.5.

### A.9.2.3	FDD—FDD Inter frequency case

#### A.9.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6.1 and 9.1.6.2.

#### A.9.2.3.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.3.2-1. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.3.2-1: RSRQ FDD—FDD Inter frequency test parameters

| Parameter |  |  | Unit |  | Test 1 |  |  |  |  | Test 2 |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  | 2 |  |  | 1 |  | 2 |  | 1 |  |  | 2 |  |
| BWchannel |  |  | MHz |  | 10 |  | 10 |  |  | 10 |  | 10 |  | 10 |  |  | 10 |  |
| Gap Pattern Id |  |  |  |  | 0 |  | - |  |  | 0 |  | - |  | 0 |  |  | - |  |
| Measurement bandwidth |  |  |  |  | 22—27 |  |  |  |  | 22—27 |  |  |  | 22—27 |  |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  |  | R.0 FDD |  |  | - |  | R.0 FDD |  | - |  | R.0 FDD |  |  | - |  |
| PDSCH allocation |  |  |  |  | 13—36 |  |  | - |  | 13—36 |  | - |  | 13—36 |  |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  |  | R.6 FDD |  |  |  |  | R.6 FDD |  |  |  | R.6 FDD |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  |  | OP.1 FDD |  | OP.2 FDD |  |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  |  | OP.2 FDD |  |
| PBCH_RA |  |  | dB |  | 0 |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Bands FDD_A Note 8 | dBm/15 kHz |  | -80 |  | -80 |  |  | -104.70 |  | -104.70 |  | -119.5 |  |  | -119.5 |  |
|  |  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | -119 |  |  | -119 |  |
|  |  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | -118.5 |  |  | -118.5 |  |
|  |  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | -118 |  |  | -118 |  |
|  |  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | -117.5 |  |  | -117.5 |  |
|  |  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | -116.5 |  |  | -116.5 |  |
|  |  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | -116 |  |  | -116 |  |
|  |  |  | dB |  | -1.75 |  | -1.75 |  |  | -4.0 |  | -4.0 |  | -4.0 |  |  | -4.0 |  |
| RSRPNote3 |  | Bands FDD_A Note 8 | dBm/15 kHz |  | -81.75 |  | -81.75 |  |  | -108.70 |  | -108.70 |  | -123.5 |  |  | -123.5 |  |
|  |  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | -123 |  |  | -123 |  |
|  |  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | -122.5 |  |  | -122.5 |  |
|  |  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | -122 |  |  | -122 |  |
|  |  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | -121.5 |  |  | -121.5 |  |
|  |  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | -120.5 |  |  | -120.5 |  |
|  |  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | -120 |  |  | -120 |  |
| RSRQNote3 |  | Bands FDD_A Note 8 | dB |  | -14.76 |  | -14.76 |  |  | -16.25 |  | -16.25 |  | -16.25 |  |  | -16.25 |  |
|  |  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 |  | Bands FDD_A Note 8 | dBm/9 MHz |  | -50 |  | -50 |  |  | -75.46 |  | -75.46 |  | -90.26 |  |  | -90.26 |  |
|  |  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | -89.76 |  |  | -89.76 |  |
|  |  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | -89.26 |  |  | -89.26 |  |
|  |  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | -88.76 |  |  | -88.76 |  |
|  |  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | -88.26 |  |  | -88.26 |  |
|  |  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | -87.26 |  |  | -87.26 |  |
|  |  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | -86.76 |  |  | -86.76 |  |
|  |  |  | dB |  | -1.75 |  | -1.75 |  |  | -4.0 |  | -4.0 |  | -4.0 |  |  | -4.0 |  |
| Propagation condition |  |  | - |  | AWGN |  |  |  |  | AWGN |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.3.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in sections 9.1.6.1 and 9.1.6.2. The RSRQ measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.6.5 and 9.1.6.6.

### A.9.2.4	TDD—TDD Inter frequency case

#### A.9.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6.1 and 9.1.6.2.

#### A.9.2.4.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.4.2-1 for TDD configuration 1 and in Table A.9.2.4.2-2 for TDD configuration 0. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A 9.2.4.2-1: RSRQ TDD—TDD Inter frequency test parameters for TDD configuration 1

| Parameter |  | Unit |  | Test 1 |  |  | Test 2 |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 2 |  | 1 |  | 2 |  | 1 |  | 2 |  |
| BWchannel |  | MHz |  | 10 | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Gap Pattern Id |  |  |  | 0 | - |  | 0 |  | - |  | 0 |  | - |  |
| Special subframe configuration Note1 |  |  |  | 6 |  |  | 6 |  |  |  | 6 |  |  |  |
| Uplink-downlink configuration Note1 |  |  |  | 1 |  |  | 1 |  |  |  | 1 |  |  |  |
| Measurement bandwidth |  |  |  | 22—27 |  |  | 22—27 |  |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  |  | R.0 TDD | - |  | R.0 TDD |  | - |  | R.0 TDD |  | - |  |
| PDSCH allocation |  |  |  | 13—36 | - |  | 13—36 |  | - |  | 13—36 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD |  |  | R.6 TDD |  |  |  | R.6 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  |  | OP.1 TDD | OP.2 TDD |  | OP.1 TDD |  | OP.2 TDD |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA |  |  | dB | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -80 |  | -80 |  | -104.70 |  | -104.70 |  | -119.50 |  | -119.50 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -118.50 |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -117.50 |  | -117.50 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -4.0 |  | -4.0 |  | -4.0 |  | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 |  | -81.75 |  | -108.70 |  | -108.70 |  | -123.50 |  | -123.50 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -122.50 |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -121.50 |  | -121.50 |  |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E |  | dB | -14.76 |  | -14.76 |  | -16.25 |  | -16.25 |  | -16.25 |  | -16.25 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50 |  | -50 |  | -75.46 |  | -75.46 |  | -90.26 |  | -90.26 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -89.26 |  | -89.26 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -88.26 |  | -88.26 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -4.0 |  | -4.0 |  | -4.0 |  | -4.0 |  |
| Propagation condition |  |  | - | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A 9.2.4.2-2: RSRQ TDD—TDD Inter frequency test parameters for TDD configuration 0

| Parameter |  | Unit |  | Test 1 |  |  | Test 2 |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 2 |  | 1 |  | 2 |  | 1 |  | 2 |  |
| BWchannel |  | MHz |  | 10 | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Gap Pattern Id |  |  |  | 0 | - |  | 0 |  | - |  | 0 |  | - |  |
| Special subframe configuration Note1 |  |  |  | 6 |  |  | 6 |  |  |  | 6 |  |  |  |
| Uplink-downlink configuration Note1 |  |  |  | 0 |  |  | 0 |  |  |  | 0 |  |  |  |
| Measurement bandwidth |  |  |  | 22—27 |  |  | 22—27 |  |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  |  | R.5 TDD | - |  | R.5 TDD |  | - |  | R.5 TDD |  | - |  |
| PDSCH allocation |  |  |  | 13—36 | - |  | 13—36 |  | - |  | 13—36 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD |  |  | R.6 TDD |  |  |  | R.6 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  |  | OP.1 TDD | OP.2 TDD |  | OP.1 TDD |  | OP.2 TDD |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA |  |  | dB | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -80 |  | -80 |  | -104.70 |  | -104.70 |  | -119.50 |  | -119.50 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -118.50 |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -117.50 |  | -117.50 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -4.0 |  | -4.0 |  | -4.0 |  | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 |  | -81.75 |  | -108.70 |  | -108.70 |  | -123.50 |  | -123.50 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -122.50 |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -121.50 |  | -121.50 |  |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E |  | dB | -14.76 |  | -14.76 |  | -16.25 |  | -16.25 |  | -16.25 |  | -16.25 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50 |  | -50 |  | -75.46 |  | -75.46 |  | -90.26 |  | -90.26 |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |  |  | -89.26 |  | -89.26 |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |  |  | -88.26 |  | -88.26 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -4.0 |  | -4.0 |  | -4.0 |  | -4.0 |  |
| Propagation condition |  |  | - | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.4.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in sections 9.1.6.1 and 9.1.6.2. The RSRQ measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.6.5 and 9.1.6.6.

### A.9.2.4A	FDD—TDD Inter frequency case

#### A.9.2.4A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6.1 and 9.1.6.2 for FDD—TDD inter frequency measurements.

#### A.9.2.4A.2	Test parameters

In this set of test cases the two cells are on different carrier frequencies. Both absolute and relative accuracy of RSRQ inter frequency measurements are tested by using the parameters in Table A.9.2.4A.2-1 and Table A.9.2.4A.2-2.  In all test cases, Cell 1 is the PCell and Cell 2 the target cell. Cell 1 is FDD cell and Cell 2 is TDD cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.2.4A.2-1: RSRQ FDD—TDD Inter frequency test parameters (FDD Cell1)

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| BWchannel | MHz | 10 | 10 | 10 |
| Gap Pattern Id |  | 0 | 0 | 0 |
| Measurement bandwidth |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD | R.0 FDD | R.0 FDD |
| PDSCH allocation |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | dBm/15 kHz | -80 | -104.70 | -114.5 |
|  | dB | -1.75 | -4.0 | -4.0 |
| RSRPNote3 | dBm/15 kHz | -81.75 | -108.70 | -118.5 |
| RSRQNote3 | dB | -14.76 | -16.25 | -16.25 |
| IoNote3 | dBm/9 MHz | -50 | -75.46 | -85.26 |
|  | dB | -1.75 | -4.0 | -4.0 |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9.2.4A.2-2: RSRQ FDD—TDD Inter frequency test parameters (TDD cell2)

| Parameter | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 2 | 2 | 2 |
| BWchannel | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  | - | - | - |
| Special subframe configuration Note1 |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  | - | - | - |
| PDSCH allocation |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.2 (OP.2 TDD) |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 |  | dBm/15 kHz | -80 | -104.70 | -114.50 |  |
|  |  | dB | -1.75 | -4.0 | -4.0 |  |
| RSRPNote4 |  | dBm/15 kHz | -81.75 | -108.70 | -118.50 |  |
| RSRQNote4 |  | dB | -14.76 | -16.25 | -16.25 |  |
| IoNote4 |  | dBm/9 MHz | -50 | -75.46 | -85.26 |  |
|  |  | dB | -1.75 | -4.0 | -4.0 |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

#### A.9.2.4A.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in sections 9.1.6.1 and 9.1.6.2. The RSRQ measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.6.5 and 9.1.6.6.

### A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation

#### A.9.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.5.2	Test parameters

In this test case the PCell and the SCell are on different carrier frequencies. There are three cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carrier are tested by using test parameters specified in Table A.9.2.5.2-1.  In the test, Cell 1 is the PCell, Cell 2 is the SCell on the Secondary Component Carrier (SCC) and Cell 3 is the neighbouring cell on the SCC. The SCC is configured and activated.

Table A.9.2.5.2-1: FDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel_CA |  | MHz | 10 | 10 | 10 |
| Timeing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | R.6FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  |  | dB | -4.0 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -123.5 | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -123 | -119.5 | -119.5 |
|  | Bands FDD_C |  | -122.5 | -119 | -119 |
|  | Bands FDD_D |  | -122 | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -121.5 | -118 | -118 |
|  | Bands FDD_G |  | -120.5 | -117 | -117 |
|  | Bands FDD_H |  | -120 | -116.5 | -116.5 |
| RSRQNote3 | Bands FDD_A | dB | -16.25 | -17.34 | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -90.26 | -85.67 | -85.67 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -89.76 | -85.17 | -85.17 |
|  | Bands FDD_C |  | -89.26 | -84.67 | -84.67 |
|  | Bands FDD_D |  | -88.76 | -84.17 | -84.17 |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.26 | -83.67 | -83.67 |
|  | Bands FDD_G |  | -87.26 | -82.67 | -82.67 |
|  | Bands FDD_H |  | -86.76 | -82.17 | -82.17 |
|  |  | dB | -4.0 | -4.0 | -4.0 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.5.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation

The test case in this clause are applicable to carrier aggregation capable UEs which have been configured with a downlink Scell.

#### A.9.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRQ measurement accuracy in carrier aggregation is within the specified limits in a synchronized network environment with AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier defined in Clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier defined in Clause 9.1.11.2, and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers defined in Clause 9.1.11.3.

#### A.9.2.6.2	Test parameters

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is PCell, Cell 2 is SCell, and Cell 3 is the target cell.  PCell and SCell are in different RF channels. Cell 3 is in the same RF channel as Cell 2. The parameters for the test are listed in Table A.9.2.6.2-1.

Table A.9.2.6.2-1: TDD RSRQ test parameters

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  | 2 |
| BWchannel |  | MHz | 10 |  |  |  |
| Timing offset to cell 1 |  | s | - | 0 |  | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 |  | - |
| Special subframe configurationNote1 |  |  | 6 |  |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.0 TDD |  | - |
| PDSCH allocation |  |  | 13—36 | 13—36 |  | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.6 TDD |  | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.1 TDD |  | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 |  | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | -116 |  |  |
|  | Bands TDD_C |  | -118.5 | -115 |  |  |
|  | Bands TDD_E |  | -117.5 | -114 |  |  |
|  |  | dB | -4.0 | -5.46 |  | -5.46 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -123.50 | -120 |  | -120 |
|  | Bands TDD_C |  | -122.50 | -119 | -119 |  |
|  | Bands TDD_E |  | -121.50 | -118 | -118 |  |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -16.25 | -17.34 |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -90.26 | -85.67 |  |  |
|  | Bands TDD_C |  | -89.26 | -84.67 |  |  |
|  | Bands TDD_E |  | -88.26 | -83.67 |  |  |
|  |  | dB | -4.0 | -4.0 |  | -4.0 |
| Propagation condition |  | - | AWGN |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.2.6.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in section 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction is within the specified limits. This test will verify the requirements in Clause 9.1.5.2 for FDD intra frequency measurements under time domain measurement resource restriction.

#### A.9.2.7.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurements under time domain measurement resource restriction is tested by using the parameters in Table A.9.2.7.2-1 and Table A.9.2.7.2-2 for non-MBSFN ABS with non-colliding CRS. In all test cases, Cell 1 is the serving cell and also the aggressor cell to Cell 2. Cell 2 is the target cell to be measured for RSRQ.

The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.2.7.2-1: General test parameters for E-UTRAN FDD RSRQ intra frequency test parameters under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The aggressor cell to Cell 2 |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met. |
| ABS pattern |  | ‘1000000010000000100000001000000010000000’ | Non-MBSFN ABS. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes in Cell 1. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘1000000010000000100000001000000010000000’ | Configured for Cell 2 measurements by measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘0100000001000000010000000100000001000000’ | Configured for measurements on Cell 1. |

Table A.9.2.7.2-2: Cell-specific test parameters for E-UTRAN FDD RSRQ intra frequency test parameters under time domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and A.3.2.1.6 (OP.6 FDD) |  |  | OP.5 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note2 | Bands FDD_A Note 10 | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
| CRS |  | dB | 5 | -2 | 5 | -2 | 5 | -4 |
| CRS Note 5 |  | dB | 2.88 | -2.00 | 2.88 | -2.00 | 3.54 | -4.00 |
| SCH |  | dB | -1.12 | -5.54 | -1.12 | -5.54 | -0.46 | -7.54 |
| RSRPNote3,4,5 | Bands FDD_A Note 10 | dBm/15 kHz | -79.76 | -86.76 | -98.85 | -105.85 | -111 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -110.5 | -119.5 |
|  | Bands FDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands FDD_D |  |  |  |  |  | -109.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -109 | -118 |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -108 | -117 |
|  | Bands FDD_H |  |  |  |  |  | -107.5 | -116.5 |
| Note3,4,5 | Bands FDD_A Note 10, FDD_B1, FDD_B2 Note 11, FDD_C, FDD_E, FDD_F Note 7, FDD_G Note 9, FDD_H | dB | -12.60 | -15.30 | -12.60 | -15.30 | -12.38 | -16.69 |
| Note3 | Bands FDD_A Note 10 | dBm/9 MHz | -50.17 | -53.64 | -69.26 | -72.73 | -81.63 | -85.37 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  | -81.13 | -84.87 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -83.87 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  | -79.63 | -83.37 |
|  | Bands FDD_G Note 9 |  |  |  |  |  | -78.63 | -82.37 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -81.87 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: Except Band 29. Note 10: Except Band 32, Band 75, Band 76. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.7.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.5.2.

### A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction is within the specified limits. This test will verify the requirements in Clause 9.1.5.2 for TDD intra frequency measurements under time domain measurement resource restriction.

#### A.9.2.8.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurements under time domain measurement resource restriction is tested by using the parameters in Table A.9.2.8.2-1 and Table A.9.2.8.2-2 for non-MBSFN ABS with non-colliding CRS. In all test cases, Cell 1 is the serving cell and also the aggressor cell to Cell 2. Cell 2 is the target cell to be measured for RSRQ.

The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.2.8.2-1: General test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | Also the aggressor cell. |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| Special subframe configuration |  | 6 | For Cell 1 and Cell 2. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  | 1 | For Cell 1 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are randomly selected so that the condition is met |
| ABS pattern |  | ‘00000000010000000001’ | Non-MBSFN ABS. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are cofigured in the ABS subframes in Cell 1. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘00000000010000000001’ | Configured for Cell 2 measurements by measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘10000000001000000000’ | Configured for Cell 1 measurements. |

Table A.9.2.8.2-2: Cell-specific test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note2 | Bands TDD_A | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
| CRS |  | dB | 5 | -2 | 5 | -2 | 5 | -4 |
| CRS Note 5 |  | dB | 2.88 | -2.00 | 2.88 | -2.00 | 3.54 | -4.00 |
| SCH |  | dB | -1.12 | -5.54 | -1.12 | -5.54 | -0.46 | -7.54 |
| RSRPNote3,4,5 | Bands TDD_A | dBm/15 kHz | -79.76 | -86.76 | -98.85 | -105.85 | -111 | -120 |
|  | Bands TDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands TDD_E |  |  |  |  |  | -109 | -118 |
| Note3,4,5 | Bands TDD_A, TDD_C, TDD_E | dB | -12.60 | -15.30 | -12.60 | -15.30 | -12.38 | -16.70 |
| Note3 | Bands TDD_A | dBm/9 MHz | -50.17 | -53.64 | -69.26 | -72.73 | -81.63 | -85.37 |
|  | Bands TDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands TDD_E |  |  |  |  |  | -79.63 | -83.37 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.2.8.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.5.2.

### A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS

#### A.9.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction is within the specified limits under AWGN propagation conditions. This test will verify the absolute FDD RSRQ accuracy under time domain measurement resource restriction specified in Clause 9.1.5.2.

#### A.9.2.9.2	Test parameters

The test parameters are given in Tables A.9.2.9.2-1 and A.9.2.9.2-2 below. In this test case there are two cells on the same frequency used in this test case. In the test, Cell 1 is the serving cell and also the aggressor cell to Cell 2. Cell 2 is the target cell to be measured for RSRQ.

The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured by higher layers with a time domain measurement restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.2.9.2-1: General test parameters for FDD RSRQ under time domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Serving cell (PCell) |  | Cell 1 | Also the aggressor cell on E-UTRA RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell to be identified on E-UTRA RF channel number 1 |
| PCell ABS configuration |  | MBSFN ABS | As defined in Table A.3.4.2.1-1 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 ) mod 6 = 0, PCIcell1 not equal to PCIcell2 | Cell PCIs are selected so that the condition is met (colliding CRS) |
| Cell 1 MBSFN ABS pattern |  | ‘0100000010000000100000000010000001000000’ | ABS subframe is only MBSFN subframe. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. Configured in Cell 1. |
| Time domain measurement resource restriction pattern for PCell (Cell 1) measurements on RF Channel 1 |  | ‘0001000000010000000100000001000000010000’ | Time domain measurement resource restriction pattern for PCell measurement signalled to the UE in measSubframePatternPCell. The IE MeasSubframePattern is used to specify the time domain measurement resource restriction as defined in TS 36.331 [2], clause 6.3.6. Configured for Cell 1 measurements. |
| Time domain measurement resource restriction pattern for neighbour cell (Cell 2) measurements on RF Channel 1 |  | ‘0100000010000000100000000010000001000000’ | Time domain measurement resource restriction pattern for neighbour cell measurement signalled to the UE in measSubframePatternNeigh The IE MeasSubframePattern is used to specify the time domain measurement resource restriction as defined in TS 36.331 [2], clause 6.3.6. Configured for Cell 2 measurements. |

Table A.9.2.9.2-2: Cell specific test parameters for FDD RSRQ under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Patterns defined in A.3.2.1.8 (OP.8 FDD) and A.3.2.1.6 (OP.6 FDD) Note5 |  |  | OP.8 FDD | OP.6 FDD | OP.8 FDD | OP.6 FDD | OP.8 FDD | OP.6 FDD |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note2 | Bands FDD_A Note 11 | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |
| CRS |  | dB | 5 | -2 | 5 | -2 | 5 | -4 |
| CRS Note 5, 7 in the 1st OFDM symbol |  | dB | 2.88 | -8.19 | 2.88 | -8.19 | 3.54 | -10.19 |
| CRS note 5 in OFDM symbols 4,7,11 |  | dB | 2.88 | -2 | 2.88 | -2 | 3.54 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -1.12 | -5.54 | -0.46 | -7.54 |
| RSRP Note 3,4,5 | Bands FDD_A Note 11 | dBm/15 kHz | -79.76 | -86.76 | -98.85 | -105.85 | -111 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -110.5 | -119.5 |
|  | Bands FDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands FDD_D |  |  |  |  |  | -109.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -109 | -118 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -108 | -117 |
|  | Bands FDD_H |  |  |  |  |  | -107.5 | -116.5 |
| (RSRQ) meas Note 3,4,5 | Bands FDD_A Note 11 | dB | -12.60 | -15.02 | -12.60 | -15.02 | -12.38 | -16.36 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  |  |  |
|  | Bands FDD_G Note 10 |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| (Io) meas Note 3 1st OFDM symbol | Bands FDD_A Note 11 | dBm/9 MHz | -50.17 | -53.64 | -69.26 | -72.73 | -81.63 | -85.37 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -81.13 | -84.87 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -83.87 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -79.63 | -83.37 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -78.63 | -82.37 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -81.87 |
| (Io) meas Note 3 OFDM symbols other than the 1st one | Bands FDD_A Note 11 | dBm/9 MHz | -50.17 | -54.85 | -69.26 | -73.94 | -81.63 | -86.76 |
|  | Bands FDD_B1, FDD_B2 Note 12 |  |  |  |  |  | -81.13 | -86.26 |
|  | Bands FDD_C |  |  |  |  |  | -80.63 | -85.76 |
|  | Bands FDD_D |  |  |  |  |  | -80.13 | -85.26 |
|  | Bands FDD_E, FDD_F Note 8 |  |  |  |  |  | -79.63 | -84.76 |
|  | Bands FDD_G Note 10 |  |  |  |  |  | -78.63 | -83.76 |
|  | Bands FDD_H |  |  |  |  |  | -78.13 | -83.26 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.1-1. Note 7: In the 1st OFDM symbol, Cell 2 is not expected to meet the Es/Iot side condition in 9.1.5.2. Note 8: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Except Band 29. Note 11: Except Band 32, Band 75, Band 76. Note 12: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.9.3	Test Requirements

In the test, the RSRQ measurement accuracy under time domain measurement resource restriction shall fulfil the requirements in Clause 9.1.5.2

### A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS

#### A.9.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction is within the specified limits. This test will verify the requirements in Clause 9.1.5.2 for TDD intra frequency measurements under time domain measurement resource restriction.

#### A.9.2.10.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurements under time domain measurement resource restriction is tested by using the parameters in Table A.9.2.10.2-1 and Table A.9.2.10.2-2 for MBSFN ABS with colliding CRS. In all test cases, Cell 1 is the serving cell and also the aggressor cell to Cell 2. Cell 2 is the target cell to be measured for RSRQ.

The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells and provided with a neighbour cell list associated with the pattern, where the cell list includes Cell 2. The UE is also configured with a time-domain measurement resource restriction pattern for the serving cell measurements. The information for both patterns shall be provided to the UE before the measurements start.

Table A.9.2.10.2-1: General test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time-domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The aggressor cell to Cell 2 |
| Neighbour cell |  | Cell 2 | Cell to be measured |
| PCell ABS configuration |  | MBSFN ABS | As defined in Table A.3.4.2.1-1 |
| Special subframe configuration |  | 6 | For Cell 1 and Cell 2. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  | 1 | For Cell 1 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 =0 PCIcell1 not equal to PCIcell2 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met |
| ABS pattern |  | ‘00001000000000100000’ | MBSFN ABS pattern. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. All ABS subframes are MBSFN subframes. |
| Time-domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘00001000000000100000’ | Configured for Cell 2 measurements by measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331 [2], clause 6.3.5. measSubframeCellList contains Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘10000000001000000000’ | Configured for measurements on Cell 1. |

Table A.9.2.10.2-2: Cell-specific test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.5 (OP.5 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.5 TDD | OP.2 TDD | OP.5 TDD | OP.2 TDD | OP.5 TDD | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 | 0 | Note 6 | 0 | Note 6 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| PSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| SSS_RA |  | dB | -4 | 0 | -4 | 0 | -4 | 0 |
| Note2 | Bands TDD_A | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
| CRS |  | dB | 5 | -2 | 5 | -2 | 5 | -4 |
| CRS Note 5, 7 In the 1st OFDM symbol |  | dB | 2.88 | -8.19 | 2.88 | -8.19 | 3.54 | -10.19 |
| CRS Note 5 in OFDM symbols 4,7,11 |  | dB | 2.88 | -2 | 2.88 | -2 | 3.54 | -4 |
| SCH |  | dB | -1.12 | -5.54 | -1.12 | -5.54 | -0.46 | -7.54 |
| RSRP Note 3,4,5 | Bands TDD_A | dBm/15 kHz | -79.76 | -86.76 | -98.85 | -105.85 | -111 | -120 |
|  | Bands TDD_C |  |  |  |  |  | -110 | -119 |
|  | Bands TDD_E |  |  |  |  |  | -109 | -118 |
| Note 3,4,5 | Bands TDD_A, TDD_C, TDD_E | dB | -12.60 | -15.02 | -12.60 | -15.02 | -12.38 | -16.36 |
| Note 3 in the 1st OFDM symbol | Bands TDD_A | dBm/9 MHz | -50.17 | -53.64 | -69.26 | -72.73 | -81.63 | -85.37 |
|  | Bands TDD_C |  |  |  |  |  | -80.63 | -84.37 |
|  | Bands TDD_E |  |  |  |  |  | -79.63 | -83.37 |
| Note 3 in OFDM symbols other than the 1st one | Bands TDD_A | dBm/9 MHz | -50.17 | -54.85 | -69.26 | -73.94 | -81.63 | -86.76 |
|  | Bands TDD_C |  |  |  |  |  | -80.63 | -85.76 |
|  | Bands TDD_E |  |  |  |  |  | -79.63 | -84.76 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes of the respective cell. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.1-1. Note 7: In the 1st OFDM symbol, Cell 2 is not expected to meet the Es/Iot side condition in 9.1.5.2. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.2.10.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in clause 9.1.5.2.

### A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.11.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.5.1.

#### A.9.2.11.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.5.2 except that the values of the parameters in the Table A.9.2.11.2-1 will replace the values of the corresponding parameters in A.9.2.5.2-1.

Table A.9.2.11.2-1: FDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note 1 |  | MHz | 20 | 20 | 20 |
| Measurement bandwidth |  |  | 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.4 FDD | R.4 FDD | - |
| PDSCH allocation |  |  | 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.10 FDD | R.10 FDD | R.10 FDD |
| OCNG Patterns defined in A.3.2.1.11 (OP.11 FDD) and A.3.2.1.12 (OP.12 FDD) |  |  | OP.11 FDD | OP.11 FDD | OP.12 FDD |
| IoNote2 | Bands FDD_A Note 5 | dBm/18 MHz | -87.26 | -82.67 |  |
|  | Bands FDD_B1 Note 5 , FDD_B2 Note 5 Note 6 |  | -86.76 | -82.17 |  |
|  | Bands FDD_C Note 5 |  | -86.26 | -81.67 |  |
|  | Bands FDD_D Note 5 |  | -85.76 | -81.17 |  |
|  | Bands FDD_E Note 5 |  | -85.26 | -80.67 |  |
|  | Bands FDD_G Note 5 |  | -84.26 | -79.67 |  |
|  | Bands FDD_H Note 5 |  | -83.76 | -79.17 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves Note 3: See Table A.9.2.5.2-1 for the other parameters Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 20 MHz channel bandwidth. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.11.3	Test Requirements

The test requirements defined in section A.9.2.5.3 shall apply in this test case.

### A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.12.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.6.1.

#### A.9.2.12.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.6.2 except that the values of the parameters in the Table A.9.2.12.2-1 will replace the values of the corresponding parameters in A.9.2.6.2-1.

Table A.9.2.12.2-1: TDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note1 |  | MHz | 20 | 20 | 20 |
| Measurement bandwidth |  |  | 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.3 TDD | R.3 TDD | - |
| PDSCH allocation |  |  | 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.10 TDD | R.10 TDD | R.10 TDD |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and A.3.2.2.8 (OP.8 TDD) |  |  | OP.7 TDD | OP.7 TDD | OP.8 TDD |
| IoNote2 | Bands TDD_A Note 5 | dBm/18 MHz | -87.26 | -82.67 |  |
|  | Bands TDD_C Note 5 |  | -86.26 | -81.67 |  |
|  | Bands TDD_E Note 5 |  | -85.26 | -80.67 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves Note 3: See Table A.9.2.6.2-1 for the other parameters. Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 20 MHz channel bandwidth. |  |  |  |  |  |

#### A.9.2.12.3	Test Requirements

The test requirements defined in section A.9.2.6.3 shall apply in this test case.

### A.9.2.13	Void

#### A.9.2.13.1	Void

#### A.9.2.13.2	Void

Table A.9.2.13.2-1: Void

#### A.9.2.13.3	Void

### A.9.2.14	Void

#### A.9.2.14.1	Void

#### A.9.2.14.2	Void

Table A.9.2.14.2-1: Void

#### A.9.2.14.3	Void

### A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.2.15.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction with CRS assistance information is  within the specified limits. This test will verify the requirements in Clause 9.1.5.3 for FDD intra frequency measurements under time domain measurement resource restriction with CRS assistance information.

#### A.9.2.15.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurements under time domain measurement resource restriction with CRS assistance information is tested by using the parameters in Table A.9.2.15.2-1 and Table A.9.2.15.2-2 for non-MBSFN ABS with colliding CRS between Cell1 and Cell3 and non-colliding CRS between Cell1 and Cell2. In all test cases, Cell 1 is the serving/aggressor cell, Cell2 is the neighbour/aggressor cell and Cell3 is the target cell to be measured for RSRQ.

The UE is configured by higher layers via Cell 1 with a time domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells, namely Cell 3 measurements with a neighbour cell list, where the cell list includes Cell 3. The UE is also provided via higher layers with the CRS assistance information of Cell 2. The information for both measurement pattern and the CRS assistance information shall be provided to the UE before the measurements start.

Note:	It’s up to eNB’s implementation whether the time domain measurement resource restriction pattern for PCell measurements is configured or not.

Table A.9.2.15.2-1: General test parameters for E-UTRAN FDD RSRQ intra frequency test parameters under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCell |  |  | Cell 1 | Serving/aggressor cell |
| Neighbour cells |  |  | Cell 2 | Neighbour/aggressor cell |
|  |  |  | Cell3 | Cell to be measured |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  |  | Normal | For all cells in the test |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 0 Cell 3 offset with respect to Cell 1: -2.5 | Three synchronous cells |
| Physical cell IDs |  |  | (PCIcell1 - PCIcell3 )mod6 = 0 (PCIcell2 - PCIcell3 )mod6 != 0 PCIcell1 not equal to PCIcell3 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423[28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in Cell 1 and Cell 2. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Configured for Cell 3 measurements. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  |  | ‘0100000001000000010000000100000001000000’ | Configured for measurements on Cell 1. |
| CRS assistance information | physCellId |  | see PCI conditions above | Only the CRS information of cell 2 is provided in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.2.15.2-2: Cell-specific test parameters for E-UTRAN FDD RSRQ intra frequency test parameters under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  |
| E-UTRA RF Channel Number |  |  | 1 |  |  | 1 |  |  | 1 |  |  |  |
| BWchannel |  | MHz | 10 |  |  | 10 |  |  | 10 |  |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  | 22—27 |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |  | R.0 FDD | - |  | R.0 FDD | - |  |  |
| PDSCH allocation |  |  | 13—36 | - |  | 13—36 | - |  | 13—36 | - |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |  | R.6 FDD |  |  | R.6 FDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and A.3.2.1.6 (OP.6 FDD) |  |  | OP.5 FDD | OP.6 FDD | OP.6 FDD | OP.5 FDD | OP.6 FDD | OP.6 FDD | OP.5 FDD | OP.6FDD |  | OP.6 FDD |
| PBCH_RA |  | dB | Note 6 | 0 |  | Note 6 | 0 |  | Note 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 2 | Bands FDD_A Note 10 | dBm/15 kHz | -84.76 |  |  | -103.85 |  |  | -116 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  | -115 |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -114 |  |  |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -113 |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  | -112.5 |  |  |  |
| CRS |  | dB | 4 | 2 | -1.5 | 4 | 2 | -1.5 | 4 | 2 |  | -4 |
| CRS Note 5 |  | dB | -1.18 | -0.32 | -6.96 | -1.18 | -0.32 | -6.96 | -0.75 | 0.54 |  | -9.46 |
| RSRP Note 3,4,5 | Bands FDD_A Note 10 | dBm/15 kHz | -80.76 | -82.76 | -86.26 | -99.85 | -101.85 | -105.35 | -112 | -114 |  | -120 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | 111.5 | 113.5 |  | 119.5 |
|  | Bands FDD_C |  |  |  |  |  |  |  | -111 | -113 |  | -119 |
|  | Bands FDD_D |  |  |  |  |  |  |  | 110.5 | 112.5 |  | 118.5 |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -110 | -112 |  | -118 |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -109 | -111 |  | -117 |
|  | Bands FDD_H |  |  |  |  |  |  |  | 108.5 | 110.5 |  | 116.5 |
| Note 3,4,5 | Bands FDD_A Note 10, FDD_B1, FDD_B2 Note 11, FDD_C, FDD_D, FDD_E, FDD_F Note 7, FDD_G Note 9, FDD_H | dB | -14.43 | -11.59 | -15.09 | -14.43 | -11.59 | -15.09 | -14.19 | -10.81 |  | -16.81 |
| Note 3 | Bands FDD_A Note 10 | dBm/9 MHz | -49.34 | -53.19 |  | -68.43 | -72.28 |  | -80.82 | -85.03 |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |  | -80.32 | -84.54 |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  | -79.82 | -84.04 |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  | -79.32 | -83.54 |  |  |
|  | Bands FDD_E, FDD_F Note 7 |  |  |  |  |  |  |  | -78.82 | -83.04 |  |  |
|  | Bands FDD_G Note 9 |  |  |  |  |  |  |  | -77.82 | -82.04 |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  | -77.32 | -81.54 |  |  |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes for only Cell 2 and Cell 3. For Cell 1, the corresponding value is derived from the normal subframes other than the subframes indicated in the time domain measurement resource restriction pattern for intra-frequency measurements. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: Except Band 29. Note 10: Except Band 32, Band 75, Band 76. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.15.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.5.3.

### A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.2.16.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy under time domain measurement resource restriction with CRS assistance information is  within the specified limits. This test will verify the requirements in Clause 9.1.5.3 for TDD intra frequency measurements under time domain measurement resource restriction with CRS assistance information.

#### A.9.2.16.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurements under time domain measurement resource restriction with CRS assistance information is tested by using the parameters in Table A.9.2.16.2-1 and Table A.9.2.16.2-2 for non-MBSFN ABS with colliding CRS between Cell1 and Cell3 and non-colliding CRS between Cell1 and Cell2. In all test cases, Cell 1 is the serving/aggressor cell, Cell2 is the neighbour/aggressor cell and Cell3 is the target cell to be measured for RSRQ.

The UE is configured by higher layers  via Cell1 with a time domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells, namely Cell 3 measurements with a neighbour cell list, where the cell list includes Cell 3. The UE is also provided via higher layers with the CRS assistance information of Cell 2. The information for both measurement pattern and the CRS assistance information shall be provided to the UE before the measurements start.

Note:	It’s up to eNB’s implementation whether the time domain measurement resource restriction pattern for PCell measurements is configured or not.

Table A.9.2.16.2-1: General test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCell |  |  | Cell 1 | Serving/aggressor cell |
| Neighbour cells |  |  | Cell 2 | Neighbour/aggressor cell |
|  |  |  | Cell3 | Cell to be measured |
| Special subframe configuration |  |  | 6 | For Cell 1, Cell 2 and Cell 3. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  |  | 1 | For Cell 1, Cell 2 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| CP length |  |  | Normal | For all cells in the test |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 0 Cell 3 offset with respect to Cell 1: -2.5 | Three synchronous cells |
| Physical cell IDs |  |  | (PCIcell1 - PCIcell3 )mod6 = 0 (PCIcell2 - PCIcell3 )mod6 != 0 PCIcell1 not equal to PCIcell3 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘00000000010000000001’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. Provided fto the UE for Cell 1 and Cell 2. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘00000000010000000001’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Provided to the UE for Cell 3 measurements. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  |  | ‘10000000001000000000’ | Configured for Cell 1 measurements. |
| CRS assistance information | physCellId |  | see PCI conditions above | Only the CRS assistance information of cell 2 is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.2.16.2-2: Cell-specific test parameters for E-UTRAN TDD RSRQ intra frequency test parameters under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  |
| E-UTRA RF Channel Number |  |  | 1 |  |  | 1 |  |  | 1 |  |  |  |
| BWchannel |  | MHz | 10 |  |  | 10 |  |  | 10 |  |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  | 22—27 |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |  | R.0 TDD | - |  | R.0 TDD | - |  |  |
| PDSCH allocation |  |  | 13—36 | - |  | 13—36 | - |  | 13—36 | - |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |  | R.6 TDD |  |  | R.6 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |  | OP.2 TDD |
| PBCH_RA |  | dB | Note 6 | 0 |  | Note 6 | 0 |  | Note 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands TDD_A | dBm/15 kHz | -84.76 |  |  | -103.85 |  |  | -116 |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |  | -115 |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  | -114 |  |  |  |
| CRS |  | dB | 4 | 2 | -1.5 | 4 | 2 | -1.5 | 4 | 2 |  | -4 |
| CRS Note 5 |  | dB | -1.18 | -0.32 | -6.96 | -1.18 | -0.32 | -6.96 | -0.75 | 0.54 |  | -9.46 |
| RSRPNote3,4,5 | Bands TDD_A | dBm/15 kHz | -80.76 | -82.76 | -86.26 | -99.85 | -101.85 | -105.35 | -112 | -114 |  | -120 |
|  | Bands TDD_C |  |  |  |  |  |  |  | -111 | -113 |  | -119 |
|  | Bands TDD_E |  |  |  |  |  |  |  | -110 | -112 |  | -118 |
| Note3,4,5 | Bands TDD_A, TDD_C, TDD_E | dB | -14.43 | -11.59 | -15.09 | -14.43 | -11.59 | -15.09 | -14.19 | -10.81 |  | -16.81 |
| Note3 | Bands TDD_A | dBm/9 MHz | -49.34 | -53.19 |  | -68.43 | -72.28 |  | -80.82 | -85.03 |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |  | -79.82 | -84.03 |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  | -78.82 | -83.04 |  |  |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement restricted subframes. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Applies to restricted measurement subframes for only Cell 2 and Cell 3. For Cell 1, the corresponding value is derived from the normal subframes other than the subframes indicated in the time domain measurement resource restriction pattern for intra-frequency measurements. Note 6: Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. Note 7: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.16.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.5.3.

### A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth

#### A.9.2.17.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.5.1.

#### A.9.2.17.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.17.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.17.2-1: RSRQ FDD Intra frequency test parameters, 5MHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.5 FDD | - | R.5 FDD | - | R.5 FDD | - |
| PDSCH allocation |  |  | 7—17 | - | 7—17 | - | 7—17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.11 FDD |  | R.11 FDD |  | R.11 FDD |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_N | dBm/15 kHz | -81.76 |  | -100.85 |  | -109.5 |  |
|  |  | dB | -1.76 | -1.76 | -4.70 | -4.70 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD_N | dBm/15 kHz | -78.76 | -78.76 | -103.75 | -103.75 | -113.50 | -113.50 |
| RSRQNote3 | Bands FDD_N | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
| IoNote3 | Bands FDD_N | dBm/4.5 MHz | -50.01 |  | -73.01 |  | -82.19 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

#### A.9.2.17.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.5.1.

### A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth

#### A.9.2.18.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6.1 and 9.1.6.2.

#### A.9.2.18.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.18.2-1. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.18.2-1: RSRQ FDD—FDD Inter frequency test parameters, 5MHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 1 | 2 | 1 | 2 |
| BWchannel |  | MHz | 5 | 5 | 5 | 5 | 5 | 5 |
| Gap Pattern Id |  |  | 0 | - | 0 | - | 0 | - |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.5 FDD | - | R.5 FDD | - | R.6 FDD | - |
| PDSCH allocation |  |  | 7—17 | - | 7—17 | - | 7—17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.11 FDD |  | R.11 FDD |  | R.11 FDD |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD | OP.15 FDD | OP.16 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -77 | -77 | -101.70 | -101.70 | -119.5 | N/A |
|  | Bands FDD_B1, FDD_B2 Note 8 |  |  |  |  |  | -119 | N/A |
|  | Bands FDD_C |  |  |  |  |  | -118.5 | N/A |
|  | Bands FDD_D |  |  |  |  |  | -118 | N/A |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -117.5 | N/A |
|  | Bands FDD_G |  |  |  |  |  | -116.5 | N/A |
|  | Bands FDD_H |  |  |  |  |  | -116 | N/A |
|  | Bands FDD_N |  |  |  |  |  | - N/A | -113 |
|  |  | dB | -1.75 | -1.75 | -4.00 | -4.00 | -4.00 | -4.00 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -78.75 | -78.75 | -105.70 | -105.70 | -123.5 | N/A |
|  | Bands FDD_B1, FDD_B2 Note 8 |  |  |  |  |  | -123 | N/A |
|  | Bands FDD_C |  |  |  |  |  | -122.5 | N/A |
|  | Bands FDD_D |  |  |  |  |  | -122 | N/A |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -121.5 | N/A |
|  | Bands FDD_G |  |  |  |  |  | -120.5 | N/A |
|  | Bands FDD_H |  |  |  |  |  | -120 | N/A |
|  | Bands FDD_N |  |  |  |  |  | N/A | -117 |
| RSRQNote3 | Bands FDD_A | dB | -14.76 | -14.76 | -16.25 | -16.25 | -16.25 | -16.25 |
|  | Bands FDD_B1, FDD_B2 Note 8 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
|  | Bands FDD_N |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/4.5 MHz | -50.01 | -50.01 | -75.47 | -75.47 | -93.27 | N/A |
|  | Bands FDD_B1, FDD_B2 Note 8 |  |  |  |  |  | -92.77 | N/A |
|  | Bands FDD_C |  |  |  |  |  | -92.27 | N/A |
|  | Bands FDD_D |  |  |  |  |  | -91.77 | N/A |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -91.27 | N/A |
|  | Bands FDD_G |  |  |  |  |  | -90.27 | N/A |
|  | Bands FDD_H |  |  |  |  |  | -89.77 | N/A |
|  | Bands FDD_N |  |  |  |  |  | N/A | -86.77 |
|  |  | dB | -1.75 | -1.75 | -4.0 | -4.0 | -4.0 | -4.0 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: This test is only applicable for testing inter-frequency requirements for Bands FDD_N. Cell 2 is on the Band under test, and Cell 1 is on another band supported by the UE. Note 7: E-UTRA operating band groups are as defined in Section 3.5. Note 8: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.18.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Sections 9.1.6.1 and 9.1.6.2.

### A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ

#### A.9.2.19.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits when the measurement configuration message received by the UE contains widebandRSRQ-Meas parameter in TS 36.331 [2]. In the test the UE shall also be configured with the AllowedMeasBandwidth parameter defined in TS 36.331 [2]. The test shall verify the WB-RSRQ inter frequency absolute accuracy requirements defined in Section 9.1.6.3.

#### A.9.2.19.2	Test parameters

In this test case the two cells are on two different carrier frequencies and measurement gaps are provided. The WB-RSRQ inter frequency absolute accuracy requirement is tested by using test parameters in Table A.9.2.19.2-1. In the test, Cell 1 is the PCell and Cell 2 the target cell on which the UE shall be ordered to measure WB-RSRQ.

Table A.9.2.19.2-1: WB-RSRQ FDD-FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |
| Gap Pattern Id |  |  | 0 | - |  |
| PBCH_RA |  | dB | 0 | 0 |  |
| PBCH_RB |  |  |  | 0 |  |
| PSS_RA |  |  |  | 0 |  |
| SSS_RA |  |  |  | 0 |  |
| PCFICH_RB |  |  |  | -∞ |  |
| PHICH_RA |  |  |  | -∞ |  |
| PHICH_RB |  |  |  | -∞ |  |
| PDCCH_RA |  |  |  | -∞ |  |
| PDCCH_RB |  |  |  | -∞ |  |
| PDSCH_RA |  |  |  | -∞ |  |
| PDSCH_RB |  |  |  | -∞ |  |
| OCNG_RANote1 |  |  |  | -∞ |  |
| OCNG_RBNote1 |  |  |  | -∞ |  |
| AllowedMeasBandwidth in TS 36.331 [2] |  | RB | 6 | 50 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |  |
| PDSCH allocation |  |  | 13-36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | - |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  | OP.1 FDD | - |  |
| Note2 | bandwidth |  | 0-49 | 0-21 28-49 | 22-27 |
|  |  | dBm/15 kHz | -94 | -87 | -110 |
|  | bandwidth |  | 0-49 | 0-21 28-49 | 22-27 |
|  |  | dB | -4 | -3 | 20 |
| RSRPNote3 |  | dBm/15 kHz | -98 | -90 |  |
| RSRQNote3 |  | dB | -16.25 | - |  |
| WB-RSRQ0Note3 in subframe 0 |  | dB | - | -13.68 |  |
| WB-RSRQ1Note3 in subframe ≠ 0 |  | dB | - | -13.63 |  |
| IoNote3 |  | dBm/ 9 MHz | -64.76 | - |  |
| IoNote3 in symbol 0, 4, 11 of subframe 0 |  | dBm/ 9 MHz | - | -82.38 |  |
| IoNote3 in symbol 7 of subframe 0 |  | dBm/ 9 MHz | - | -82.20 |  |
| IoNote3 in symbol 0, 4, 7, 11 of subframes ≠ 0 |  | dBm/ 9 MHz | - | -82.38 |  |
| Propagation condition |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that Cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells not specified in the test, assumed to be constant over time and modelled as noise. Note 3: RSRQ, RSRP, WB-RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The stated values represent the weighted average over the allowed measurement bandwidth, and the WB-RSRQ values assume averaging over symbols 0, 4, 7 and 11 of the subframe. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: This test case is applicable to all FDD frequency bands except bands within band group FDD_N. |  |  |  |  |  |

#### A.9.2.19.3	Test Requirements

The WB-RSRQ measurement accuracy for cell 2 shall fulfil the requirements in Section 9.1.6.3, compared with WB-RSRQ0. or WB-RSRQ1.

### A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ

#### A.9.2.20.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits when the measurement configuration message received by the UE contains widebandRSRQ-Meas parameter in TS 36.331 [2]. In the test the UE shall also be configured with the AllowedMeasBandwidth parameter defined in TS 36.331 [2]. The test shall verify the WB-RSRQ inter frequency absolute accuracy requirements defined in Section 9.1.6.3.

#### A.9.2.20.2	Test parameters

In this test case the two cells are on two different carrier frequencies and measurement gaps are provided. The WB-RSRQ inter frequency absolute accuracy requirement is tested by using test parameters in Table A.9.2.20.2-1. In the test, Cell 1 is the PCell and Cell 2 the target cell on which the UE shall be ordered to measure WB-RSRQ.

Table A.9.2.20.2-1: WB-RSRQ TDD-TDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |
| BWchannel |  | MHz | 10 | 10 |  |
| Special subframe configuration Note1 |  |  | 6 | 6 |  |
| Uplink-downlink configuration Note1 |  |  | 1 | 1 |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |
| Gap Pattern Id |  |  | 0 | - |  |
| PBCH_RA |  | dB | 0 | 0 |  |
| PBCH_RB |  |  |  | 0 |  |
| PSS_RA |  |  |  | 0 |  |
| SSS_RA |  |  |  | 0 |  |
| PCFICH_RB |  |  |  | -∞ |  |
| PHICH_RA |  |  |  | -∞ |  |
| PHICH_RB |  |  |  | -∞ |  |
| PDCCH_RA |  |  |  | -∞ |  |
| PDCCH_RB |  |  |  | -∞ |  |
| PDSCH_RA |  |  |  | -∞ |  |
| PDSCH_RB |  |  |  | -∞ |  |
| OCNG_RANote2 |  |  |  | -∞ |  |
| OCNG_RBNote2 |  |  |  | -∞ |  |
| AllowedMeasBandwidth in TS 36.331 [2] |  | RB | 6 | 50 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |  |
| PDSCH allocation |  |  | 13-36 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | - |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  |  | OP.1 TDD | - |  |
| Note3 | bandwidth |  | 0-49 | 0-21 28-49 | 22-27 |
|  |  | dBm/15 kHz | -94 | -87 | -110 |
|  | bandwidth |  | 0—49 | 0-21 28-49 | 22-27 |
|  |  | dB | -4 | -3 | 20 |
| RSRPNote4 |  | dBm/15 kHz | -98 | -90 |  |
| RSRQNote4 |  | dB | -16.25 | - |  |
| WB-RSRQ0Note4 in subframe 0 |  | dB | - | -13.68 |  |
| WB-RSRQ1Note4 in subframe ≠ 0 |  | dB | - | -13.63 |  |
| IoNote4 |  | dBm/ 9 MHz | -64.76 | - |  |
| IoNote4 in symbol 0, 4, 11 of subframe 0 |  | dBm/ 9 MHz | - | -82.38 |  |
| IoNote4 in symbol 7 of subframe 0 |  | dBm/ 9 MHz | - | -82.20 |  |
| IoNote4 in symbol 0, 4, 7, 11 of subframes ≠ 0 |  | dBm/ 9 MHz | - | -82.38 |  |
| Propagation condition |  | - | AWGN | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that Cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells not specified in the test, assumed to be constant over time and modelled as noise. Note 4: RSRQ, RSRP, WB-RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The stated values represent the weighted average over the allowed measurement bandwidth, and the WB-RSRQ values assume averaging over symbols 0, 4, 7 and 11 of the subframe. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

#### A.9.2.20.3	Test Requirements

The WB-RSRQ measurement accuracy for cell 2 shall fulfil the requirements in Section 9.1.6.3, compared with WB-RSRQ0. or WB-RSRQ1.

### A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.21.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.5.1.

#### A.9.2.21.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.5.2 except that the values of the parameters in the Table A.9.2.21.2-1 will replace the values of the corresponding parameters in A.9.2.5.2-1.

Table A.9.2.21.2-1: FDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 |  | Cell 3 |
| BWchannel_CA Note 1 |  | MHz | 10 | 5 |  |  |
| Measurement bandwidth |  |  | 22-27 | 10-15 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.5 FDD | - |  |
| PDSCH allocation |  |  | 13-36 | 7-17 | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | R.11 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD), A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.1 FDD | OP.15 FDD |  | OP.16 FDD |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 |  | -116 |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -119 | -115.5 |  | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 |  | -115 |
|  | Bands FDD_D |  | -118 | -114.5 |  | -114.5 |
|  | Bands FDD_E, FDD_F |  | -117.5 | -114 |  | -114 |
|  | Bands FDD_G |  | -116.5 | -113 |  | -113 |
|  | Bands FDD_H |  | -116 | -112.5 |  | -112.5 |
|  | Bands FDD_N |  | N/A | -109.5 |  | -109.5 |
| RSRPNote2 | Bands FDD_A | dBm/15 kHz | -123.5 | -120 |  | -120 |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -123 | -119.5 |  | -119.5 |
|  | Bands FDD_C |  | -122.5 | -119 |  | -119 |
|  | Bands FDD_D |  | -122 | -118.5 |  | -118.5 |
|  | Bands FDD_E, FDD_F |  | -121.5 | -118 |  | -118 |
|  | Bands FDD_G |  | -120.5 | -117 |  | -117 |
|  | Bands FDD_H |  | -120 | -116.5 |  | -116.5 |
|  | Bands FDD_N |  | N/A | -113.5 |  | -113.5 |
| RSRQNote2 | Bands FDD_A | dB | -16.25 | -17.34 |  | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 4 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
|  | Bands FDD_N |  |  |  |  |  |
| IoNote2 | Bands FDD_A | dBm/9MHz | -90.26 | N/A |  |  |
|  | Bands FDD_B1, FDD_B2 Note 4 |  | -89.76 |  |  |  |
|  | Bands FDD_C |  | -89.26 |  |  |  |
|  | Bands FDD_D |  | -88.76 |  |  |  |
|  | Bands FDD_E, FDD_F |  | -88.26 |  |  |  |
|  | Bands FDD_G |  | -87.26 |  |  |  |
|  | Bands FDD_H |  | -86.76 |  |  |  |
|  | Bands FDD_A | dBm/4.5MHz | N/A | -88.67 |  |  |
|  | Bands FDD_B1, FDD_B2 Note 4 |  |  | -88.17 |  |  |
|  | Bands FDD_C |  |  | -87.67 |  |  |
|  | Bands FDD_D |  |  | -87.17 |  |  |
|  | Bands FDD_E, FDD_F |  |  | -86.67 |  |  |
|  | Bands FDD_G |  |  | -85.67 |  |  |
|  | Bands FDD_H |  |  | -85.17 |  |  |
|  | Bands FDD_N |  |  | -82.17 |  |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.2.5.2-1 for the other parameters Note 4: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.2.21.3	Test Requirements

The test requirements defined in section A.9.2.5.3 shall apply in this test case.

### A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.22.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.6.1.

#### A.9.2.22.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.6.2 except that the values of the parameters in the Table A.9.2.22.2-1 will replace the values of the corresponding parameters in A.9.2.6.2-1.

Table A.9.2.22.2-1: TDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note1 |  | MHz | 10 | 5 |  |
| Measurement bandwidth |  |  | 22-27 | 10-15 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.4TDD | - |
| PDSCH allocation |  |  | 13-36 | 7-17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.11 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD), A.3.2.2.9 (OP.9 TDD) and A.3.2.2.10 (OP.10 TDD) |  |  | OP.1 TDD | OP.9 TDD | OP.10 TDD |
| IoNote2 | Bands TDD_A | dBm/9MHz | -90.26 | N/A |  |
|  | Bands TDD_C |  | -89.26 |  |  |
|  | Bands TDD_E |  | -88.26 |  |  |
|  | Bands TDD_A | dBm/4.5MHz | N/A | -88.67 |  |
|  | Bands TDD_C |  |  | -87.67 |  |
|  | Bands TDD_E |  |  | -86.67 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves Note 3: See Table A.9.2.6.2-1 for the other parameters |  |  |  |  |  |

#### A.9.2.22.3	Test Requirements

The test requirements defined in section A.9.2.6.3 shall apply in this test case.

### A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.23.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.5.1.

#### A.9.2.23.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.5.2 except that the values of the parameters in the Table A.9.2.23.2-1 will replace the values of the corresponding parameters in A.9.2.5.2-1.

Table A.9.2.23.2-1: FDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note 1 |  | MHz | 5 | 5 | 5 |
| Measurement bandwidth |  |  | 10-15 | 10-15 | 10-15 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.5 FDD | R.5 FDD | N/A |
| PDSCH allocation |  |  | 7-17 | 7-17 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.11 FDD | R.11 FDD | R.11 FDD |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  |  | OP.15 FDD | OP.15 FDD | OP.16FDD |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 6 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  | Bands FDD_N |  | -113 | -109.5 | -109.5 |
| RSRPNote2 | Bands FDD_A | dBm/15 kHz | -123.5 | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 6 |  | -123 | -119.5 | -119.5 |
|  | Bands FDD_C |  | -122.5 | -119 | -119 |
|  | Bands FDD_D |  | -122 | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F |  | -121.5 | -118 | -118 |
|  | Bands FDD_G |  | -120.5 | -117 | -117 |
|  | Bands FDD_H |  | -120 | -116.5 | -116.5 |
|  | Bands FDD_N |  | -117 | -113.5 | -113.5 |
| RSRQNote2 | Bands FDD_A | dB | -16.25 | -17.34 | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 6 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
|  | Bands FDD_N |  |  |  |  |
| IoNote2 | Bands FDD_A Note 5 | dBm/4.5MHz | -93.26 | -88.67 |  |
|  | Bands FDD_B1 Note 5,FDD_B2 Note 5 Note 6 |  | -92.76 | -88.17 |  |
|  | Bands FDD_C Note 5 |  | -92.26 | -87.67 |  |
|  | Bands FDD_D |  | -91.76 | -87.17 |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -91.26 | -86.67 |  |
|  | Bands FDD_G Note 5 |  | -90.26 | -85.67 |  |
|  | Bands FDD_H Note 5 |  | -89.76 | -85.17 |  |
|  | Bands FDD_N Note 5 |  | -86.76 | -82.76 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.2.5.2-1 for the other parameters Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 5MHz + 5MHz channel bandwidth. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.23.3	Test Requirements

The test requirements defined in section A.9.2.5.3 shall apply in this test case.

### A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.24.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.6.1.

#### A.9.2.24.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.6.2 except that the values of the parameters in the Table A.9.2.24.2-1 will replace the values of the corresponding parameters in A.9.2.6.2-1.

Table A.9.2.24.2-1: TDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note1 |  | MHz | 10 | 5 | 5 |
| Measurement bandwidth |  |  | 10-15 | 10-15 | 10-15 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 TDD | R.4 TDD | N/A |
| PDSCH allocation |  |  | 13-36 | 7-17 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.11 TDD | R.11 TDD |
| OCNG Patterns defined in A.3.2.2.9 (OP.9 TDD) and A.3.2.2.10 (OP.10 TDD) |  |  | OP.1 TDD | OP.9 TDD | OP.10 TDD |
| IoNote2 | Bands TDD_A Note 5 | dBm4.5MHz | -93.26 | -88.67 |  |
|  | Bands TDD_C Note 5 |  | -92.26 | -87.67 |  |
|  | Bands TDD_E Note 5 |  | -91.26 | -86.67 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.2.6.2-1 for the other parameters Note 4: E-UTRA operating band groups are as defined in Section 3.5. Note 5: The test applies for E-UTRA operating bands in this band group which are supporting 5MHz + 5MHz channel bandwidth. |  |  |  |  |  |

#### A.9.2.24.3	Test Requirements

The test requirements defined in section A.9.2.6.3 shall apply in this test case.

### A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD

The test case in this section are applicable to TDD-FDD carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.25.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of  RSRQ measurements for the primary component carrier defined in clause 9.1.11.1, the absolute accuracy of RSRQ measurements for the secondary component carrier defined in clause 9.1.11.2, and also the relative RSRQ accuracy requirement between primary and secondary component carriers defined in clause 9.1.11.3.

#### A.9.2.25.2	Test parameters

In this test case the PCell is FDD and SCell is TDD. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carrier are tested by using test parameters specified in Table A.9.2.25.2-1.  In the test, Cell 1 is the PCell, Cell 2 is the SCell on the Secondary Component Carrier (SCC). The SCC is configured and activated.

The parameters of this test are given in Table A.9.2.25.2-1.

Table A.9.2.25.2-1: RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | - | 6 |
| Uplink-downlink configurationNote1 |  |  | - | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | - | -116 |
|  | Bands TDD_C |  | - | -115 |
|  | Bands TDD_E |  | - | -114 |
|  | Bands FDD_A |  | -119.5 | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | - |
|  | Bands FDD_C |  | -118.5 | - |
|  | Bands FDD_D |  | -118 | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | - |
|  | Bands FDD_G |  | -116.5 | - |
|  | Bands FDD_H |  | -116 | - |
|  |  | dB | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | - | -122 |
|  | Bands TDD_C |  | - | -121 |
|  | Bands TDD_E |  | - | -120 |
|  | Bands FDD_A |  | -125.5 | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -123 | - |
|  | Bands FDD_C |  | -124.5 | - |
|  | Bands FDD_D |  | -124 | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | - |
|  | Bands FDD_G |  | -122.5 | - |
|  | Bands FDD_H |  | -122 | - |
| RSRQNote4 | Bands TDD_A | dB | - | -17.77 |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  | -17.77 | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote4 | Bands TDD_A | dBm/BWchannel | - | -87.25 + 10log(NRB,c/50) |
|  | Bands TDD_C |  | - | -86.25 + 10log(NRB,c/50) |
|  | Bands TDD_E |  | - | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | 89.76 | - |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | - |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | - |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | - |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | - |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.2.25.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD

The test case in this section are applicable to TDD-FDD carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.26.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD measurement accuracy in carrier aggregation is within the specified limits. This test will verify the absolute accuracy of RSRQ measurements for the primary component carrier defined in Clause 9.1.11.1, the absolute accuracy of RSRQ measurements for the secondary component carrier defined in Clause 9.1.11.2, and also the relative RSRQ accuracy requirement between primary and secondary component carriers defined in Clause 9.1.11.3.

#### A.9.2.26.2	Test parameters

In this test case the PCell is TDD and SCell is FDD. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carrier are tested by using test parameters specified in Table A.9.2.26.2-1.  In the test, Cell 1 is the PCell, Cell 2 is the SCell on the Secondary Component Carrier (SCC). The SCC is configured and activated.

The parameters of this test are given in Table A.9.2.26.2-1.

Table A.9.2.26.2-1: RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 | - |
| Uplink-downlink configurationNote1 |  |  | 1 | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | - |
|  | Bands TDD_C |  | -118.5 | - |
|  | Bands TDD_E |  | -117.5 | - |
|  | Bands FDD_A |  | - | -116 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -115.5 |
|  | Bands FDD_C |  | - | -115 |
|  | Bands FDD_D |  | - | -114.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -114 |
|  | Bands FDD_G |  | - | -113 |
|  | Bands FDD_H |  | - | -112.5 |
|  |  | dB | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -125.50 | - |
|  | Bands TDD_C |  | -124.50 | - |
|  | Bands TDD_E |  | -123.50 | - |
|  | Bands FDD_A |  | - | -122 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -119.5 |
|  | Bands FDD_C |  | - | -121 |
|  | Bands FDD_D |  | - | -120.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -120 |
|  | Bands FDD_G |  | - | -119 |
|  | Bands FDD_H |  | - | -118.5 |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | - |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
|  | Bands FDD_A |  | - | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote4 | Bands TDD_A | dBm/BWchannel | -90.75 + 10log(NRB,c/50) | - |
|  | Bands TDD_C |  | -89.75 + 10log(NRB,c/50) | - |
|  | Bands TDD_E |  | -88.75 + 10log(NRB,c/50) | - |
|  | Bands FDD_A |  | - | -87.25 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  | -85.17 |
|  | Bands FDD_C |  | - | -86.25 + 10log(NRB,c/50) |
|  | Bands FDD_D |  | - | -85.75 + 10log(NRB,c/50) |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_G |  | - | -84.25 + 10log(NRB,c/50) |
|  | Bands FDD_H |  | - | -83.75 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.2.26.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in section 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz

The test case in this section are applicable to carrier aggregation capable UEs which have been configured with a downlink SCell.

#### A.9.2.27.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.9.2.6.1.

#### A.9.2.27.2	Test parameters

The parameters of this test are the same as defined in Subclause A.9.2.6.2 except that the values of the parameters in the Table A.9.2.27.2-1 will replace the values of the corresponding parameters in A.9.2.6.2-1.

Table A.9.2.27.2-1: TDD RSRQ Carrier Aggregation test parameters

| Parameters |  | Units | Combination | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |
| BWchannel_CA Note1 |  | MHz | 20MHz+10MHz | 20MHz: NRB,c = 100 | 10MHz: NRB,c = 50 |  |
|  |  |  | 10MHz+20MHz | 10MHz: NRB,c = 50 | 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  |  | 20MHz+10MHz | 47-52 | 22-27 |  |
|  |  |  | 10MHz+20MHz | 22-27 | 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | 20MHz+10MHz | R.3 TDD | R.0 TDD | N/A |
|  |  |  | 10MHz+20MHz | R.0 TDD | R.3 TDD |  |
| PDSCH allocation |  |  | 20MHz+10MHz | 38-61 | 13-36 | N/A |
|  |  |  | 10MHz+20MHz | 13-36 | 38-61 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | 20MHz+10MHz | R.10 TDD | R.6 TDD | R.6 TDD |
|  |  |  | 10MHz+20MHz | R.6 TDD | R.10 TDD | R.10 TDD |
| OCNG Patterns defined in A.3.2.2 (TDD) |  |  | 20MHz+10MHz | OP.7 TDD | OP.1 TDD | OP.2 TDD |
|  |  |  | 10MHz+20MHz | OP.1 TDD | OP.7 TDD | OP.8 TDD |
| IoNote2 | Bands TDD_A | dBm/BWchannel | All | -90.26 + 10log(NRB,c/50) | N/A |  |
|  | Bands TDD_C |  |  | -89.26 + 10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  |  | -88.26 + 10log(NRB,c/50) |  |  |
|  | Bands TDD_A | dBm/BWchannel | All | N/A | -85.67 + 10log(NRB,c/50) |  |
|  | Bands TDD_C |  |  |  | -84.67 + 10log(NRB,c/50) |  |
|  | Bands TDD_E |  |  |  | -83.67 + 10log(NRB,c/50) |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: See Table A.9.2.6.2-1 for the other parameters. |  |  |  |  |  |  |

#### A.9.2.27.3	Test Requirements

The test requirements defined in section A.9.2.6.3 shall apply in this test case.

### A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal

#### A.9.2.28.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.14.4.

#### A.9.2.28.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement for Cell 2 is tested by using the parameters in Table A.9.2.28.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. Cell 2 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.28.2-1: RSRQ FDD Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 1 |  |
| BWchannel |  |  | MHz | 10 |  |
| Measurement bandwidth |  |  |  | 22—27 |  |
| DMTC period |  |  | ms | N/A | 160 |
| DMTC period offset |  |  | ms | N/A | 10 |
| Discovery signal occasion duration |  |  | ms | N/A | 1 |
| Time offset between cell 1 and cell 2 |  |  | s | 0 | 2.3 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  | R.0 FDD | - |
| PDSCH allocation |  |  |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 |  | Bands FDD_A Note 8 | dBm/15 kHz | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -115.5 |  |
|  | Bands FDD_C |  | -115 |  |
|  | Bands FDD_D |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -114 |  |
|  | Bands FDD_G Note 7 |  | -113 |  |
|  | Bands FDD_H |  | -112.5 |  |
|  |  |  | dB | -5.46 | -5.46 |
| RSRPNote3 |  | Bands FDD_A Note 8 | dBm/15 kHz | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119.5 | -119.5 |
|  | Bands FDD_C |  | -119 | -119 |
|  | Bands FDD_D |  | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 5 |  | -118 | -118 |
|  | Bands FDD_G Note 7 |  | -117 | -117 |
|  | Bands FDD_H |  | -116.5 | -116.5 |
| RSRQNote3 |  | Bands FDD_A Note 8 | dB | -17.34 | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 |  | Bands FDD_A Note 8 | dBm/9 MHz | -85.67 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -85.17 |  |
|  | Bands FDD_C |  | -84.67 |  |
|  | Bands FDD_D |  | -84.17 |  |
|  | Bands FDD_E, FDD_F Note 5 |  | -83.67 |  |
|  | Bands FDD_G Note 7 |  | -82.67 |  |
|  | Bands FDD_H |  | -82.17 |  |
|  |  |  | dB | -4 | -4 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.28.3	Test Requirements

The absolute accuracy of RSRQ intra frequency measurement for Cell 2 shall fulfil the requirements in Clause 9.1.14.4.

### A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal

#### A.9.2.29.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.14.4.

#### A.9.2.29.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement for Cell 2 is tested by using the parameters in Table A.9.2.29.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. Cell 2 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.29.2-1: RSRQ TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  |
| BWchannel |  | MHz | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  |
| DMTC period |  | ms | N/A | 160 |
| DMTC period offset |  | ms | N/A | 10 |
| Discovery signal occasion duration |  | ms | N/A | 2 |
| Time offset between cell 1 and cell 2 |  | s | 0 | 2.3 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands TDD_A | dBm/15 kHz | -116 |  |
|  | Bands TDD_C |  | -115 |  |
|  | Bands TDD_E |  | -114 |  |
|  |  | dB | -5.46 | -5.46 |
| RSRPNote3 | Bands TDD_A | dBm/15 kHz | -120 | -120 |
|  | Bands TDD_C |  | -119 | -119 |
|  | Bands TDD_E |  | -118 | -118 |
| RSRQNote3 | Bands TDD_A | dB | -17.34 | -17.34 |
|  | Bands TDD_C |  |  |  |
|  | Bands TDD_E |  |  |  |
| IoNote3 | Bands TDD_A | dBm/9 MHz | -85.67 |  |
|  | Bands TDD_C |  | -84.67 |  |
|  | Bands TDD_E |  | -83.67 |  |
|  |  | dB | -4 | -4 |
| Propagation condition |  | - | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.2.29.3	Test Requirements

The absolute accuracy of RSRQ intra frequency measurement for Cell 2 shall fulfil the requirements in Clause 9.1.14.4.

### A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal

#### A.9.2.30.1	Test Purpose and Environment

The purpose of this test is to verify that the absolute and relative accuracy of RSRQ measurement in discovery signal occasions is within the specified limits. This test will verify the requirements in Sections 9.1.14.4.

#### A.9.2.30.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.30.2-1. In all tests, Cell 1 is the PCell and Cell 2 the target cell. For measurement of the carrier frequency of Cell 2, DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.30.2-1: RSRQ in discovery signal occasions FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - |
| Gap Offset |  | ms | 9 | - |
| DMTC period |  | ms | - | 160 |
| DMTC period offset |  | ms | - | 10 |
| Discovery signal occasion duration |  | ms | - | 1 |
| Time offset between cell 2 and cell 1 |  | s | 3 |  |
| Measurement bandwidth |  |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |
| PDSCH allocation |  |  | 13-36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -117.5 | -117.5 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -117 | -117 |
|  | Bands FDD_C |  | -116.5 | -116.5 |
|  | Bands FDD_D |  | -116 | -116 |
|  | Bands FDD_E, FDD_F Note 5 |  | -115.5 | -115.5 |
|  | Bands FDD_G Note 7 |  | -114.5 | -114.5 |
|  | Bands FDD_H |  | -114 | -114 |
|  |  | dB | -6 | -6 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -123.5 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -123 | -123 |
|  | Bands FDD_C |  | -122.5 | -122.5 |
|  | Bands FDD_D |  | -122 | -122 |
|  | Bands FDD_E, FDD_F Note 5 |  | -121.5 | -121.5 |
|  | Bands FDD_G Note 7 |  | -120.5 | -120.5 |
|  | Bands FDD_H |  | -120 | -120 |
| RSRQNote3 | Bands FDD_A Note 8 | dB | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/ 9 MHz | -88.75 | -88.75 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -88.25 | -88.25 |
|  | Bands FDD_C |  | -87.75 | -87.75 |
|  | Bands FDD_D |  | -87.25 | -87.25 |
|  | Bands FDD_E, FDD_F Note 5 |  | -86.75 | -86.75 |
|  | Bands FDD_G Note 7 |  | -85.75 | -85.75 |
|  | Bands FDD_H |  | -85.25 | -85.25 |
|  |  | dB | -6 | -6 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.2.30.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Sections 9.1.14.4.

### A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal

#### A.9.2.31.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in discovery signal occasions is within the specified limits. This test will verify the requirements in Sections 9.1.14.4.

#### A.9.2.31.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.31.2-1 for TDD configuration 1. In all tests, Cell 1 is the PCell and Cell 2 the target cell. DMTC configuration for Cell 2 is provided to UE in the measDS-Config before the start of the test.

Table A 9.2.31.2-1: RSRQ TDD—TDD Inter frequency test parameters for TDD configuration 1

| Parameter |  | Unit |  | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  | MHz |  | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | - |
| Gap Offset |  |  |  | 9 | - |
| DMTC period |  | ms |  | - | 160 |
| DMTC period offset |  | ms |  | - | 10 |
| Discovery signal occasion duration |  | ms |  | - | 2 |
| Time offset between cells |  | s |  | 0 | 3 |
| Special subframe configuration Note1 |  |  |  | 6 |  |
| Uplink-downlink configuration Note1 |  |  |  | 1 |  |
| Measurement bandwidth |  |  |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  |  | R.0 TDD | - |
| PDSCH allocation |  |  |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  |  | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -117.50 | -117.50 |
|  | Bands TDD_C |  |  | -116.50 | -116.50 |
|  | Bands TDD_E |  |  | -115.50 | -115.50 |
|  |  |  | dB | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -123.50 | -123.50 |
|  | Bands TDD_C |  |  | -122.50 | -122.50 |
|  | Bands TDD_E |  |  | -121.50 | -121.50 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E |  | dB | -17.77 | -17.77 |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -88.75 | -88.75 |
|  | Bands TDD_C |  |  | -87.75 | -87.75 |
|  | Bands TDD_E |  |  | -86.75 | -86.75 |
|  |  |  | dB | -6.0 | -6.0 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### A.9.2.31.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Sections 9.1.14.4.

### A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal

#### A.9.2.32.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRQ measurement accuracy for carrier aggregation in CRS based discovery signal is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier specified in clause 9.1.15.1.2, and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.15.1.3.

#### A.9.2.32.2	Test parameters

In this test case the PCell and the SCell are on different carrier frequencies. There are three cells used in this test case. RSRQ absolute and relative accuracy requirements of the primary and secondary component carrier are tested by using test parameters specified in Table A.9.2.32.2-1.  In the test, Cell 1 is the PCell, Cell 2 is the SCell on the Secondary Component Carrier (SCC) and Cell 3 is the neighbouring cell on the SCC.  Cell 2 on SCC is configured and activated. Cell 3 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.32.2-1: FDD RSRQ Carrier Aggregation Test Parameters

| Parameters |  | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Units | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel_CA |  | MHz | 10 | 10 | 10 |
| DMTC period |  |  | N/A | N/A | 160 |
| DMTC period offset |  |  | N/A | N/A | 10 |
| Discovery signal occasion duration |  |  | N/A | N/A | 1 |
| Timeing offset to Cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | R.6FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  |  | dB | -4.0 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -123.5 | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -123 | -119.5 | -119.5 |
|  | Bands FDD_C |  | -122.5 | -119 | -119 |
|  | Bands FDD_D |  | -122 | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -121.5 | -118 | -118 |
|  | Bands FDD_G |  | -120.5 | -117 | -117 |
|  | Bands FDD_H |  | -120 | -116.5 | -116.5 |
| RSRQNote3 | Bands FDD_A | dB | -16.25 | -17.34 | -17.34 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -90.26 | -85.67 | -85.67 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -89.76 | -85.17 | -85.17 |
|  | Bands FDD_C |  | -89.26 | -84.67 | -84.67 |
|  | Bands FDD_D |  | -88.76 | -84.17 | -84.17 |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.26 | -83.67 | -83.67 |
|  | Bands FDD_G |  | -87.26 | -82.67 | -82.67 |
|  | Bands FDD_H |  | -86.76 | -82.17 | -82.17 |
|  |  | dB | -4.0 | -4.0 | -4.0 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.32.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.15.1.1, 9.1.15.1.2, and 9.1.15.1.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.15.1.2.

-	The relative accuracy of inter-frequency RSRQ measurements between Cell 1 on primary component carriers and Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.15.1.3.

### A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal

#### A.9.2.33.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRQ measurement accuracy for carrier aggregation in CRS based discovery signal is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier specified in clause 9.1.15.1.2, and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.15.1.3.

#### A.9.2.33.2	Test parameters

In this test case the PCell and the SCell are on different carrier frequencies. There are three cells used in this test case. RSRQ absolute and relative accuracy requirements of the primary and secondary component carrier are tested by using test parameters specified in Table A.9.2.33.2-1.  In the test, Cell 1 is the PCell, Cell 2 is the SCell on the Secondary Component Carrier (SCC) and Cell 3 is the neighbouring cell on the SCC.  Cell 2 on SCC is configured and activated. Cell 3 DMTC configuration is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.33.2-1: TDD RSRQ Carrier Aggregation Test Parameters

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 10 |  |  |
| DMTC period |  |  | N/A | N/A | 160 |
| DMTC period offset |  |  | N/A | N/A | 10 |
| Discovery signal occasion duration |  |  | N/A | N/A | 2 |
| Timing offset to cell 1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1 | - |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 22—27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | -116 |  |
|  | Bands TDD_C |  | -118.5 | -115 |  |
|  | Bands TDD_E |  | -117.5 | -114 |  |
|  |  | dB | -4.0 | -5.46 | -5.46 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -123.50 | -120 | -120 |
|  | Bands TDD_C |  | -122.50 | -119 | -119 |
|  | Bands TDD_E |  | -121.50 | -118 | -118 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -16.25 | -17.34 |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -90.26 | -85.67 |  |
|  | Bands TDD_C |  | -89.26 | -84.67 |  |
|  | Bands TDD_E |  | -88.26 | -83.67 |  |
|  |  | dB | -4.0 | -4.0 | -4.0 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### A.9.2.33.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.15.1.1, 9.1.15.1.2, and 9.1.15.1.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.15.1.2.

-	The relative accuracy of inter-frequency RSRQ measurements between Cell 1 on primary component carriers and Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.15.1.3.

### A.9.2.34	FDD—FDD Inter frequency new RSRQ

#### A.9.2.34.1	Test Purpose and Environment

The purpose of this test is to verify that the absolute accuracy of RSRQ measurement is within the specified limits when measurement configuration message received by the UE contains measRSRQ-OnAllSymbols-r12 parameter in TS 36.331 [2]. This test will verify the requirements in Sections 9.1.16.

#### A.9.2.34.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. The new RSRQ inter frequency absolute accuracy requirement is tested by using test parameters in Table A.9.2.34.2-1. In the test, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.34.2-1: New RSRQ FDD—FDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 |
| Gap Pattern Id |  |  | 0 | - |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Time offset between cell 2 and cell 1 |  | s | 3 |  |
| Measurement bandwidth |  |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - |
| PDSCH allocation |  |  | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | - |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  | OP.1 FDD | - |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  | 0 |
| PSS_RA |  |  |  | 0 |
| SSS_RA |  |  |  | 0 |
| PCFICH_RB |  |  |  | -∞ |
| PHICH_RA |  |  |  | -∞ |
| PHICH_RB |  |  |  | -∞ |
| PDCCH_RA |  |  |  | -∞ |
| PDCCH_RB |  |  |  | -∞ |
| PDSCH_RA |  |  |  | -∞ |
| PDSCH_RB |  |  |  | -∞ |
| OCNG_RANote1 |  |  |  | -∞ |
| OCNG_RBNote1 |  |  |  | -∞ |
| Note2 | Symbols with CRS, PSS, SSS or PBCH | dBm/15 kHz | -103.85 | -103.85 |
|  | All the other symbols |  | -94.75 | -94.75 |
|  |  | dB | -3 | -3 |
| RSRPNote3 |  | dBm/15 kHz | -106.85 | -106.85 |
| RSRQNote3 | Subframe 0 | dB | -14.54 | -14.54 |
|  | Subframes other than 0 |  | -14.14 | -14.14 |
| New RSRQNote3 | Subframe 0 | dB | -19.57 | -19.57 |
|  | Subframe 5 |  | -20.93 | -20.93 |
|  | Subframe other than 0 or 5 |  | -21.66 | -21.66 |
| Io in subframe 0Note3 | Symbol 0/4/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/12/13 |  | -66.97 | -66.97 |
|  | Symbol 5/6/8/9/10 |  | -75.81 | -75.81 |
|  | Symbol 7 |  | -75.52 | -75.52 |
| Io in subframe 5Note3 | Symbol 0/4/7/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/8/9/10/12/13 |  | -66.97 | -66.97 |
|  | Symbol 5/6 |  | -75.81 | -75.81 |
| Io in subframes other than 0 or 5 Note3 | Symbol 0/4/7/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/5/6/8/9/10/12/13 |  | -66.97 | -66.97 |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP, new RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The new RSRQ values assume RSSI averaging over all OFDM symbols of the subframe. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### A.9.2.34.3	Test Requirements

The new RSRQ measurement accuracy for cell 2 shall fulfil the requirements in Section 9.1.16, compared with any nominal new RSRQ value in subframe 0, 5 or others.

### A.9.2.35	TDD—TDD Inter frequency new RSRQ

#### A.9.2.35.1	Test Purpose and Environment

The purpose of this test is to verify that the absolute accuracy of RSRQ measurement is within the specified limits when measurement configuration message received by the UE contains measRSRQ-OnAllSymbols-r12 parameter in TS 36.331 [2]. This test will verify the requirements in Sections 9.1.16.

#### A.9.2.35.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. The new RSRQ inter frequency absolute accuracy requirement is tested by using test parameters in Table A.9.2.35.2-1. In the test, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.35.2-1: New RSRQ TDD—TDD Inter frequency test parameters

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |
| BWchannel |  | MHz | 10 | 10 |
| Special subframe configuration Note1 |  |  | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  | 1 | 1 |
| Gap Pattern Id |  |  | 0 | - |
| Antenna Configuration |  |  | 1x2 | 1x2 |
| Time offset between cell 2 and cell 1 |  | s | 3 |  |
| Measurement bandwidth |  |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - |
| PDSCH allocation |  |  | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | - |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  |  | OP.1 TDD | - |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  | 0 |
| PSS_RA |  |  |  | 0 |
| SSS_RA |  |  |  | 0 |
| PCFICH_RB |  |  |  | -∞ |
| PHICH_RA |  |  |  | -∞ |
| PHICH_RB |  |  |  | -∞ |
| PDCCH_RA |  |  |  | -∞ |
| PDCCH_RB |  |  |  | -∞ |
| PDSCH_RA |  |  |  | -∞ |
| PDSCH_RB |  |  |  | -∞ |
| OCNG_RANote1 |  |  |  | -∞ |
| OCNG_RBNote1 |  |  |  | -∞ |
| Note2 | Symbols with CRS, PSS, SSS or PBCH | dBm/15 kHz | -103.85 | -103.85 |
|  | All the other symbols |  | -94.75 | -94.75 |
|  |  | dB | -3 | -3 |
| RSRPNote3 |  | dBm/15 kHz | -106.85 | -106.85 |
| RSRQNote3 | Subframe 0 | dB | -14.54 | -14.54 |
|  | Subframes other than 0 |  | -14.14 | -14.14 |
| New RSRQNote3 | Subframe 0 | dB | -20.08 | -20.08 |
|  | Subframe 5 |  | -21.31 | -21.31 |
|  | Subframe 1 or 6 |  | -20.82 | -20.82 |
|  | Subframe other than 0, 1, 5 or 6 |  | -21.66 | -21.66 |
| Io in subframe 0Note3 | Symbol 0/4/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/5/6/12 |  | -66.97 | -66.97 |
|  | Symbol 8/9/10/13 |  | -75.81 | -75.81 |
|  | Symbol 7 |  | -75.52 | -75.52 |
| Io in subframe 5Note3 | Symbol 0/4/7/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/5/6/8/9/10/12 |  | -66.97 | -66.97 |
|  | Symbol 13 |  | -75.81 | -75.81 |
| Io in subframe 1 or 6Note3 | Symbol 0/4/7 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/3/5/6/8 |  | -66.97 | -66.97 |
|  | Symbol 2 |  | -75.81 | -75.81 |
| Io in subframes other than 0, 1, 5 or 6 Note3 | Symbol 0/4/7/11 | dBm/ 9 MHz | -75.72 | -75.72 |
|  | Symbol 1/2/3/5/6/8/9/10/12/13 |  | -66.97 | -66.97 |
| Propagation condition |  | - | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP, new RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The new RSRQ values assume RSSI averaging over all OFDM symbols of the subframe. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

#### A.9.2.35.3	Test Requirements

The new RSRQ measurement accuracy for cell 2 shall fulfil the requirements in Section 9.1.16, compared with any nominal new RSRQ value in subframe 0, 5, 1, 6 or others.

### A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols

#### A.9.2.36.1	Test Purpose and Environment

The purpose of this test is to verify that the absolute accuracy of RSRQ measurement is within the specified limits when measurement configuration message received by the UE contains measRSRQ-OnAllSymbols-r12 parameter in TS 36.331 [2]. This test will verify the requirements in Section 9.1.16.

A.9.2.3 is also conducted even if UE is capable of measuring RSRQ on all OFDM symbols.

#### A.9.2.36.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ measured on all OFDM symbols inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.36.2-1. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.36.2-1: FDD—FDD Inter frequency test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10 | 10 |
| Gap Pattern Id |  | 0 | - |
| Antenna Configuration |  | 1x2 | 1x2 |
| Time offset between cell 2 and cell 1 | s | 3 |  |
| Measurement bandwidth |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD | - |
| PDSCH allocation |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| Note2 | dBm/15 kHz | -80 | -80 |
|  | dB | -1.75 | -1.75 |
| RSRPNote3 | dBm/15 kHz | -81.75 | -81.75 |
| RSRQNote3 | dB | -14.76 | -14.76 |
| IoNote3 | dBm/ 9 MHz | -50 | -50 |
|  | dBm/ 9 MHz | -1.75 | -1.75 |
| Propagation condition | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The RSRQ values assume RSSI averaging over all OFDM symbols of the subframe. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

#### A.9.2.36.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Section 9.1.16.

### A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols

#### A.9.2.37.1	Test Purpose and Environment

The purpose of this test is to verify that the absolute accuracy of RSRQ measurement is within the specified limits when measurement configuration message received by the UE contains measRSRQ-OnAllSymbols-r12 parameter in TS 36.331 [2]. This test will verify the requirements in Section 9.1.16.

A.9.2.4 is also conducted even if UE is capable of measuring RSRQ on all OFDM symbols..

#### A.9.2.37.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RSRQ measured on all OFDM symbols inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.37.2-1. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.37.2-1: TDD-TDD Inter frequency test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10 | 10 |
| Special subframe configuration Note1 |  | 6 | 6 |
| Uplink-downlink configuration Note1 |  | 1 | 1 |
| Gap Pattern Id |  | 0 | - |
| Antenna Configuration |  | 1x2 | 1x2 |
| Time offset between cell 2 and cell 1 | s | 3 |  |
| Measurement bandwidth |  | 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | R.0 TDD | - |
| PDSCH allocation |  | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| Note2 | dBm/15 kHz | -80 | -80 |
|  | dB | -1.75 | -1.75 |
| RSRPNote3 | dBm/15 kHz | -81.75 | -81.75 |
| RSRQNote3 | dB | -14.76 | -14.76 |
| IoNote3 | dBm/ 9 MHz | -50 | -50 |
|  | dBm/ 9 MHz | -1.75 | -1.75 |
| Propagation condition | - | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The RSRQ values assume RSSI averaging over all OFDM symbols of the subframe. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

#### A.9.2.37.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Section 9.1.16.

### A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with two downlink SCells.

#### A.9.2.38.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.38.2	Test parameters

In this set of test cases there are three cells on three carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, and Cell 3 is activated SCell on channel 3. The parameters for the test are listed in Table A.9.2.38.2-1.

Table A.9.2.38.2-1: 3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | - | 6 | 6 |
| Uplink/downlink configurationNote1 |  |  | - | 1 | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | - | -116 | -116 |
|  | Bands TDD_C |  | - | -115 | -115 |
|  | Bands TDD_E |  | - | -114 | -114 |
|  | Bands FDD_A |  | -119.5 | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | - | - |
|  | Bands FDD_C |  | -118.5 | - | - |
|  | Bands FDD_D |  | -118 | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | - | - |
|  | Bands FDD_G |  | -116.5 | - | - |
|  | Bands FDD_H |  | -116 | - | - |
|  |  | dB | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | - | -122 | -122 |
|  | Bands TDD_C |  | - | -121 | -121 |
|  | Bands TDD_E |  | - | -120 | -120 |
|  | Bands FDD_A |  | -125.5 | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -125 | - | - |
|  | Bands FDD_C |  | -124.5 | - | - |
|  | Bands FDD_D |  | -124 | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | - | - |
|  | Bands FDD_G |  | -122.5 | - | - |
|  | Bands FDD_H |  | -122 | - | - |
| RSRQNote4 | Bands TDD_A | dB | - | -17.77 | -17.77 |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands TDD_C |  | - | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands TDD_E |  | - | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | - | - |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | TAE | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.38.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with two downlink SCells.

#### A.9.2.39.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD-FDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier defined in Clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier defined in Clause 9.1.11.2, and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers defined in Clause 9.1.11.3.

#### A.9.2.39.2	Test parameters

In this set of cases cell 1 is PCell on the primary component carrier, and cell 2 and cell 3 are activated SCells on secondary component carriers SCC1 and SCC2 respectively. The test parameters for the test are listed in Table A.9.2.39.2-1.

Table A.9.2.39.2-1: 3 Downlink TDD-FDD RSRQ carrier aggregation test parameters with PCell in TDD (cell #1, cell #2 and cell #3)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 | - | - |
| Uplink/downlink configurationNote1 |  |  | 1 | - | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 and A.3.1.1.2 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 and A.3.2.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -119.5 | - | - |
|  | Bands TDD_C |  | -118.5 | - | - |
|  | Bands TDD_E |  | -117.5 | - | - |
|  | Bands FDD_A |  | - | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -115.5 | -115.5 |
|  | Bands FDD_C |  | - | -115 | -115 |
|  | Bands FDD_D |  | - | -114.5 | -114.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -114 | -114 |
|  | Bands FDD_G |  | - | -113 | -113 |
|  | Bands FDD_H |  | - | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.00 | -6.00 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -125.50 | - | - |
|  | Bands TDD_C |  | -124.50 | - | - |
|  | Bands TDD_E |  | -123.50 | - | - |
|  | Bands FDD_A |  | - | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -121.5 | -121.5 |
|  | Bands FDD_C |  | - | -121 | -121 |
|  | Bands FDD_D |  | - | -120.5 | -120.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -120 | -120 |
|  | Bands FDD_G |  | - | -119 | -119 |
|  | Bands FDD_H |  | - | -118.5 | -118.5 |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | - | - |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
|  | Bands FDD_A |  | - | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -90.75 + 10log(NRB,c/50) | - | - |
|  | Bands TDD_C |  | -89.75 + 10log(NRB,c/50) | - | - |
|  | Bands TDD_E |  | -88.75 + 10log(NRB,c/50) | - | - |
|  | Bands FDD_A |  | - | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -86.75 + 10log(NRB,c/50) | -86.75 + 10log(NRB,c/50) |
|  | Bands FDD_C |  | - | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands FDD_D |  | - | -85.75 + 10log(NRB,c/50) | -85.75 + 10log(NRB,c/50) |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_G |  | - | -84.25 + 10log(NRB,c/50) | -84.25 + 10log(NRB,c/50) |
|  | Bands FDD_H |  | - | -83.75 + 10log(NRB,c/50) | -83.75 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | TAE | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.39.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.40.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.40.2	Test parameters

In this test case the PCell and the SCells are on different carrier frequencies. There are three cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carriers are tested by using test parameters specified in Table A.9.2.40.2-1.  In the test, Cell 1 is the PCell, Cell 2 and Cell 3 are the SCells on secondary component carrier SCC1 and SCC2 respectively. The SCC1 and SCC2 are configured and activated.

Table A.9.2.40.2-1: 3 DL FDD RSRQ carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz:NRB,c = 25 10MHz: NRB,c = 50 20MHz:NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Measurement bandwidth |  |  | 5MHz:10-15 10MHz:22-27 20MHz:47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD |
| PDSCH allocation |  |  | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note3 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -125 | -121.5 | -121.5 |
|  | Bands FDD_C |  | -124.5 | -121 | -121 |
|  | Bands FDD_D |  | -124 | -120.5 | -120.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -123.5 | -120 | -120 |
|  | Bands FDD_G |  | -122.5 | -119 | -119 |
|  | Bands FDD_H |  | -122 | -118.5 | -118.5 |
| RSRQNote3 | Bands FDD_A | dB | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -90.25+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) |
|  | Bands FDD_C |  | -89.75+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands FDD_D |  | -89.25+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.75+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
|  | Bands FDD_G |  | -87.75+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) |
|  | Bands FDD_H |  | -87.25+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.40.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.41.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRQ measurement accuracy in carrier aggregation is within the specified limits in a synchronized network environment with AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier defined in Clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers defined in Clause 9.1.11.2, and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers defined in Clause 9.1.11.3.

#### A.9.2.41.2	Test parameters

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is PCell, Cell 2 and Cell 3 are the SCells on secondary component carrier SCC1 and SCC2 respectively.  PCell and SCells are in different RF channels. The parameters for the test are listed in Table A.9.2.41.2-1.

Table A.9.2.41.2-1: 3 DL TDD RSRQ carrier aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands TDD_C |  | -118.5 | -115 | -115 |
|  | Bands TDD_E |  | -117.5 | -114 | -114 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note4 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands TDD_C |  | -124.5 | -121 | -121 |
|  | Bands TDD_E |  | -123.5 | -120 | -120 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -17.77 | -17.77 | -17.77 |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands TDD_C |  | -89.75+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands TDD_E |  | -88.75+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### A.9.2.41.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in section 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0

#### A.9.2.42.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.13.3 for FD-FDD intra frequency RSRQ measurements for UE category 0.

#### A.9.2.42.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.42.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.42.2-1: FD-FDD RSRQ Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.3 |  |  | R.13 FDD | - | R.13 FDD | - | R.13 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-0_A Note 7 | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-0_E, FDD-0_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD-0_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Note3 |  | dB | -1.76 | -1.76 | -4.7 | -4.7 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD-0_A Note 7 | dBm/15 kHz | -81.76 | -81.76 | -106.75 | -106.75 | -120 | -120 |
|  | Bands FDD-0_B |  |  |  |  |  | -119.5 | -119.5 |
|  | Bands FDD-0_C |  |  |  |  |  | -119 | -119 |
|  | Bands FDD-0_D |  |  |  |  |  | -118.5 | -118.5 |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -118 | -118 |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -117 | -117 |
|  | Bands FDD-0_H |  |  |  |  |  | -116.5 | -116.5 |
| RSRQNote3 | Bands FDD-0_A Note 7 | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
|  | Bands FDD-0_B |  |  |  |  |  |  |  |
|  | Bands FDD-0_C |  |  |  |  |  |  |  |
|  | Bands FDD-0_D |  |  |  |  |  |  |  |
|  | Bands FDD-0_E, FDD-0_F Note 45 |  |  |  |  |  |  |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD-0_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD-0_A Note 7 | dBm/9 MHz | -50 |  | -73 |  | -85.67 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -85.17 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -84.67 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -84.17 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -83.67 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -82.67 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -82.17 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: Except Band 29. Note 7: Except Band 32, Band 75, Band 76. |  |  |  |  |  |  |  |  |

#### A.9.2.42.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.13.3.

### A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0

#### A.9.2.43.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.13.3 for HD-FDD intra frequency RSRQ measurements for UE category 0.

#### A.9.2.43.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.43.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.43.2-1: HD-FDD RSRQ Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.4 |  |  | R.1 HD-FDD | - | R.1 HD-FDD | - | R.1 HD-FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.3 |  |  | R.3 HD-FDD |  | R.3 HD-FDD |  | R.3 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-0_A Note 7 | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-0_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Note3 |  | dB | -1.76 | -1.76 | -4.7 | -4.7 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD-0_A Note 7 | dBm/15 kHz | -81.76 | -81.76 | -106.75 | -106.75 | -120 | -120 |
|  | Bands FDD-0_B |  |  |  |  |  | -119.5 | -119.5 |
|  | Bands FDD-0_C |  |  |  |  |  | -119 | -119 |
|  | Bands FDD-0_D |  |  |  |  |  | -118.5 | -118.5 |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -118 | -118 |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -117 | -117 |
|  | Bands FDD-0_H |  |  |  |  |  | -116.5 | -116.5 |
| RSRQNote3 | Bands FDD-0_A Note 7 | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
|  | Bands FDD-0_B |  |  |  |  |  |  |  |
|  | Bands FDD-0_C |  |  |  |  |  |  |  |
|  | Bands FDD-0_D |  |  |  |  |  |  |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  |  |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD-0_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD-0_A Note 7 | dBm/9 MHz | -50 |  | -73 |  | -85.67 |  |
|  | Bands FDD-0_B |  |  |  |  |  | -85.17 |  |
|  | Bands FDD-0_C |  |  |  |  |  | -84.67 |  |
|  | Bands FDD-0_D |  |  |  |  |  | -84.17 |  |
|  | Bands FDD-0_E, FDD-0_F Note 4 |  |  |  |  |  | -83.67 |  |
|  | Bands FDD-0_G Note 6 |  |  |  |  |  | -82.67 |  |
|  | Bands FDD-0_H |  |  |  |  |  | -82.17 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: Except Band 29. Note 7: Except Band 32, Band 75, Band 76. |  |  |  |  |  |  |  |  |

#### A.9.2.43.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.13.3.

### A.9.2.44	TDD RSRQ Intra frequency case for UE category 0

#### A.9.2.44.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.13.3 for TDD intra frequency RSRQ measurements for UE category 0.

#### A.9.2.44.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.44.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.2.44.2-1: TDD RSRQ Intra frequency test parameters for UE category 0

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.5 |  |  | R.12 TDD | - | R.12 TDD | - | R.12 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-0_A | dBm/15 kHz | -84.76 |  | -103.85 |  | -116 |  |
|  | Bands TDD-0_C |  |  |  |  |  | -115 |  |
|  | Bands TDD-0_E |  |  |  |  |  | -114 |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
|  |  | dB | -1.76 | -1.76 | -4.7 | -4.7 | -5.46 | -5.46 |
| RSRPNote4 | Bands TDD-0_A | dBm/15 kHz | -81.76 | -81.76 | -106.75 | -106.75 | -120 | -120 |
|  | Bands TDD-0_C |  |  |  |  |  | -119 | -119 |
|  | Bands TDD-0_E |  |  |  |  |  | -118 | -118 |
| RSRQNote4 | Bands TDD-0_A, TDD-0_C, TDD-0_E | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
| IoNote4 | Bands TDD-0_A | dBm/9 MHz | -50 |  | -73 |  | -85.67 |  |
|  | Bands TDD-0_C |  |  |  |  |  | -84.67 |  |
|  | Bands TDD-0_E |  |  |  |  |  | -83.67 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.2.44.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Clause 9.1.13.3.

### A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with three downlink SCells.

#### A.9.2.45.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.45.2	Test parameters

In this set of test cases there are four cells on four carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, and Cell 4 is activated SCell on channel 4. The parameters for the test are listed in Table A.9.2.45.2-1.

Table A.9.2.45.2-1: 4 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters (cell #1, cell #2, cell #3 and cell #4)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | - | 6 | 6 | 6 |
| Uplink/downlink configurationNote1 |  |  | - | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | - | -116 | -116 | -116 |
|  | Bands TDD_C |  | - | -115 | -115 | -115 |
|  | Bands TDD_E |  | - | -114 | -114 | -114 |
|  | Bands FDD_A |  | -119.5 | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | - | - | - |
|  | Bands FDD_C |  | -118.5 | - | - | - |
|  | Bands FDD_D |  | -118 | - | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | - | - | - |
|  | Bands FDD_G |  | -116.5 | - | - | - |
|  | Bands FDD_H |  | -116 | - | - | - |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | - | -122 | -122 | -122 |
|  | Bands TDD_C |  | - | -121 | -121 | -121 |
|  | Bands TDD_E |  | - | -120 | -120 | -120 |
|  | Bands FDD_A |  | -125.5 | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -125 | - | - | - |
|  | Bands FDD_C |  | -124.5 | - | - | - |
|  | Bands FDD_D |  | -124 | - | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | - | - | - |
|  | Bands FDD_G |  | -122.5 | - | - | - |
|  | Bands FDD_H |  | -122 | - | - | - |
| RSRQNote4 | Bands TDD_A | dB | - | -17.77 | -17.77 | -17.77 |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands TDD_C |  | - | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands TDD_E |  | - | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | - | - | - |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.2.45.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with three downlink SCells.

#### A.9.2.46.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.46.2	Test parameters

In this set of test cases there are four cells on four carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, and Cell 4 is activated SCell on channel 4. The parameters for the test are listed in Table A.9.2.45.2-1.

Table A.9.2.46.2-1: 4 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation test parameters (cell #1, cell #2, cell #3 and cell #4)

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 | - | - | - |
| Uplink/downlink configurationNote1 |  |  | 1 | - | - | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -119.5 | - | - | - |
|  | Bands TDD_C |  | -118.5 | - | - | - |
|  | Bands TDD_E |  | -117.5 | - | - | - |
|  | Bands FDD_A |  | - | -116 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -115.5 | -115.5 | -115.5 |
|  | Bands FDD_C |  | - | -115 | -115 | -115 |
|  | Bands FDD_D |  | - | -114.5 | -114.5 | -114.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -114 | -114 | -114 |
|  | Bands FDD_G |  | - | -113 | -113 | -113 |
|  | Bands FDD_H |  | - | -112.5 | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -125.5 | - | - | - |
|  | Bands TDD_C |  | -124.5 | - | - | - |
|  | Bands TDD_E |  | -123.5 | - | - | - |
|  | Bands FDD_A |  | - | -122 | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -121.5 | -121.5 | -121.5 |
|  | Bands FDD_C |  | - | -121 | -121 | -121 |
|  | Bands FDD_D |  | - | -120.5 | -120.5 | -120.5 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -120 | -120 | -120 |
|  | Bands FDD_G |  | - | -119 | -119 | -119 |
|  | Bands FDD_H |  | - | -118.5 | -118.5 | -118.5 |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | - | - | - |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  | - | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -90.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands TDD_C |  | -89.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands TDD_E |  | -88.75 + 10log(NRB,c/50) | - | - | - |
|  | Bands FDD_A |  | - | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -86.75 + 10log(NRB,c/50) | -86.75 + 10log(NRB,c/50) | -86.75 + 10log(NRB,c/50) |
|  | Bands FDD_C |  | - | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands FDD_D |  | - | -85.75 + 10log(NRB,c/50) | -85.75 + 10log(NRB,c/50) | -85.75 + 10log(NRB,c/50) |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_G |  | - | -84.25 + 10log(NRB,c/50) | -84.25 + 10log(NRB,c/50) | -84.25 + 10log(NRB,c/50) |
|  | Bands FDD_H |  | - | -83.75 + 10log(NRB,c/50) | -83.75 + 10log(NRB,c/50) | -83.75 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.2.46.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with four downlink SCells.

#### A.9.2.47.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.47.2	Test parameters

In this set of test cases there are five cells on five carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, Cell 4 is activated SCell on channel 4, and Cell 5 is activated SCell on channel 5. The parameters for the test are listed in Table A.9.2.45.2-1.

Table A.9.2.47.2-1: 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters (cell #1, cell #2, cell #3, cell #4 and cell#5)

| Parameter |  | Unit | Cell 1 | Cells |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 | 3 | 4 | 5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 | 5 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| Special subframe configurationNote1 |  |  | - | 6 |  |  |  |
| Uplink/downlink configurationNote1 |  |  | - | 1 |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | - | -116 |  |  |  |
|  | Bands TDD_C |  | - | -115 |  |  |  |
|  | Bands TDD_E |  | - | -114 |  |  |  |
|  | Bands FDD_A |  | -119.5 | - |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | - |  |  |  |
|  | Bands FDD_C |  | -118.5 | - |  |  |  |
|  | Bands FDD_D |  | -118 | - |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | - |  |  |  |
|  | Bands FDD_G |  | -116.5 | - |  |  |  |
|  | Bands FDD_H |  | -116 | - |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | - | -122 |  |  |  |
|  | Bands TDD_C |  | - | -121 |  |  |  |
|  | Bands TDD_E |  | - | -120 |  |  |  |
|  | Bands FDD_A |  | -125.5 | - |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -125 | - |  |  |  |
|  | Bands FDD_C |  | -124.5 | - |  |  |  |
|  | Bands FDD_D |  | -124 | - |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | - |  |  |  |
|  | Bands FDD_G |  | -122.5 | - |  |  |  |
|  | Bands FDD_H |  | -122 | - |  |  |  |
| RSRQNote4 | Bands TDD_A | dB | - | -17.77 |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | - |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | - | -87.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_C |  | - | -86.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_E |  | - | -85.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | - |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |
| Timing offset to Cell 1 |  | s | - | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.2.47.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 5 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation

The test case in this clause is applicable to carrier aggregation capable UEs which have been configured with four downlink SCells.

#### A.9.2.48.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.48.2	Test parameters

In this set of test cases there are five cells on five carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, Cell 4 is activated SCell on channel 4, and Cell 5 is activated SCell on channel 5. The parameters for the test are listed in Table A.9.2.45.2-1.

Table A.9.2.48.2-1: 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters (cell #1, cell #2, cell #3, cell #4 and cell#5)

| Parameter |  | Unit | Cell 1 | Cells |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 | 3 | 4 | 5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 | 5 |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| Special subframe configurationNote1 |  |  | 6 | - |  |  |  |
| Uplink/downlink configurationNote1 |  |  | 1 | - |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | - |  |  |  |
|  | Bands TDD_C |  | -115 | - |  |  |  |
|  | Bands TDD_E |  | -114 | - |  |  |  |
|  | Bands FDD_A |  | - | -119.5 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -119 |  |  |  |
|  | Bands FDD_C |  | - | -118.5 |  |  |  |
|  | Bands FDD_D |  | - | -118 |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -117.5 |  |  |  |
|  | Bands FDD_G |  | - | -116.5 |  |  |  |
|  | Bands FDD_H |  | - | -116 |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | - |  |  |  |
|  | Bands TDD_C |  | -121 | - |  |  |  |
|  | Bands TDD_E |  | -120 | - |  |  |  |
|  | Bands FDD_A |  | - | -125.5 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -125 |  |  |  |
|  | Bands FDD_C |  | - | -124.5 |  |  |  |
|  | Bands FDD_D |  | - | -124 |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -123.5 |  |  |  |
|  | Bands FDD_G |  | - | -122.5 |  |  |  |
|  | Bands FDD_H |  | - | -122 |  |  |  |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | - |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | - | -17.77 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | - |  |  |  |
|  | Bands FDD_A |  | - | -90.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | - | -90.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_C |  | - | -89.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_D |  | - | -89.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | - | -88.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_G |  | - | -87.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_H |  | - | -87.25 + 10log(NRB,c/50) |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |
| Timing offset to Cell 1 |  | s | 0 | - |  |  |  |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.2.48.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 5 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.49.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.49.2	Test parameters

In this set of test cases the PCell and the SCells are on different carrier frequencies. There are five cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carriers are tested by using test parameters specified in Table A.9.2.49.2-1 and Table A.9.2.49.2-2.  In the test, Cell 1 is the PCell, Cell 2, Cell 3, Cell 4 and Cell 5 are the SCells on secondary component carrier SCC1, SCC2, SCC3 and SCC4 respectively. The SCC1, SCC2, SCC3 and SCC4 are configured and activated.

Table A.9.2.49.2-1: 5 DL FDD RSRQ carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz:NRB,c = 25 10MHz: NRB,c = 50 20MHz:NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Measurement bandwidth |  |  | 5MHz:10-15 10MHz:22-27 20MHz:47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD |
| PDSCH allocation |  |  | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note3 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -125 | -121.5 | -121.5 |
|  | Bands FDD_C |  | -124.5 | -121 | -121 |
|  | Bands FDD_D |  | -124 | -120.5 | -120.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -123.5 | -120 | -120 |
|  | Bands FDD_G |  | -122.5 | -119 | -119 |
|  | Bands FDD_H |  | -122 | -118.5 | -118.5 |
| RSRQNote3 | Bands FDD_A | dB | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -90.25+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) |
|  | Bands FDD_C |  | -89.75+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands FDD_D |  | -89.25+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.75+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
|  | Bands FDD_G |  | -87.75+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) |
|  | Bands FDD_H |  | -87.25+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table A.9.2.49.2-2: 5 DL FDD RSRQ carrier aggregation test parameters for cell 4 and cell 5

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 2 | 3 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD |
| PDSCH allocation |  |  | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -115.5 | -115.5 |
|  | Bands FDD_C |  | -115 | -115 |
|  | Bands FDD_D |  | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -114 | -114 |
|  | Bands FDD_G |  | -113 | -113 |
|  | Bands FDD_H |  | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 |
| Note3 |  | dB | -6.0 | -6.0 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -121.5 | -121.5 |
|  | Bands FDD_C |  | -121 | -121 |
|  | Bands FDD_D |  | -120.5 | -120.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -120 | -120 |
|  | Bands FDD_G |  | -119 | -119 |
|  | Bands FDD_H |  | -118.5 | -118.5 |
| RSRQNote3 | Bands FDD_A | dB | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |
|  | Bands FDD_C |  |  |  |
|  | Bands FDD_D |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |
|  | Bands FDD_G |  |  |  |
|  | Bands FDD_H |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -86.75+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) |
|  | Bands FDD_C |  | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands FDD_D |  | -85.75+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
|  | Bands FDD_G |  | -84.25+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) |
|  | Bands FDD_H |  | -83.75+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 3 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 4 Note 7 |  |  | - | TAE |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

#### A.9.2.49.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC3 for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC4 for Cell 5 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.50.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.50.2	Test parameters

In this set of test cases the PCell and the SCells are on different carrier frequencies. There are five cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carriers are tested by using test parameters specified in Table A.9.2.50.2-1 and Table A.9.2.50.2-2.  In the test, Cell 1 is the PCell, Cell 2, Cell 3, Cell 4 and Cell 5 are the SCells on secondary component carrier SCC1, SCC2, SCC3 and SCC4 respectively. The SCC1, SCC2, SCC3 and SCC4 are configured and activated.

Table A.9.2.50.2-1: 5 DL TDD RSRQ carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands TDD_C |  | -118.5 | -115 | -115 |
|  | Bands TDD_E |  | -117.5 | -114 | -114 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note4 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands TDD_C |  | -124.5 | -121 | -121 |
|  | Bands TDD_E |  | -123.5 | -120 | -120 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -17.77 | -17.77 | -17.77 |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands TDD_C |  | -89.75+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands TDD_E |  | -88.75+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table A.9.2.50.2-2: 5 DL TDD RSRQ carrier aggregation test parameters for cell 4 and cell 5

| Parameter |  | Unit | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 | 5 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 | 6 |
| Uplink-downlink configurationNote1 |  |  | 1 | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -116 | -116 |
|  | Bands TDD_C |  | -115 | -115 |
|  | Bands TDD_E |  | -114 | -114 |
|  |  | dB | -6.0 | -6.0 |
| Note4 |  | dB | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -122 | -122 |
|  | Bands TDD_C |  | -121 | -121 |
|  | Bands TDD_E |  | -120 | -120 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -17.77 | -17.77 |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands TDD_C |  | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands TDD_E |  | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 3 Note 7 |  |  | TAE | TAE |
| Time alignment error relative to cell 4 Note 7 |  |  | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |

#### A.9.2.50.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on SCC4 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC3 for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC4 for Cell 5 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell

#### A.9.2.51.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD intra frequency RSRQ absolute and relative measurement accuracies in carrier aggregation with frame structure 3 in the configured DMTC occasion are within the specified limits. This test will verify the absolute RSRQ accuracy requirement of the secondary component carrier defined in clause 9.1.19.2. The test will also verify the primary and secondary component carrier relative RSRQ accuracy requirement defined in Clause 9.1.19.4.

#### A.9.2.51.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier with frame structure 3 and activated, and Cell3 is the neighboring cell on the same secondary component carrier of Cell2. The test parameters are given in Table A.9.2.51.2-1. The DMTC configuration for Cell2 and Cell3 is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.51.2-1: Test parameters for FDD RSRQ accuracies of Scell with FS3

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 | 20 |
| DMTC period |  | ms | N/A | 40 | 40 |
| DMTC period offset |  |  | N/A | 10 | 10 |
| Discovery signal occasion duration |  | ms | N/A | 1 | 1 |
| LBT model |  |  | N/A | N/A | A.3.17 |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RA Note1 |  |  |  |  |  |
| OCNG_RB Note1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -119 |  |  |
|  | Bands FDD_C |  | -118.5 |  |  |
|  | Bands FDD_D |  | -118 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 |  |  |
|  | Bands FDD_G |  | -116.5 |  |  |
|  | Bands FDD_H |  | -116 |  |  |
|  | Bands FS3_G |  | - | ( for Channel 1 +3.5dB) |  |
|  |  | dB | -4 | -5.46Note9 | -5.46 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -123.5 | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -123 |  |  |
|  | Bands FDD_C |  | -122.5 |  |  |
|  | Bands FDD_D |  | -122 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -121.5 |  |  |
|  | Bands FDD_G |  | -120.5 |  |  |
|  | Bands FDD_H |  | -120 |  |  |
|  | Bands FS3_G |  | - | (RSRP for Cell 1 +3.5dB) | (RSRP for Cell 1 +3.5dB) |
| RSRQNote3 | Bands FDD_A | dBm/15 kHz | -16.25 | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
|  | Bands FS3_G |  | - | -17.34Note9 | -17.34 |
| IoNote3 | Bands FDD_A | 5MHz: dBm/4.5MHz 10MHz: dBm/9MHz 20MHz: dBm/18MHz | -90.26 +10log(NRB,c/50) | - |  |
|  | Bands FDD_B1, FDD_B2 Note 10 |  | -89.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -89.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -88.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -87.26 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -86.76 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 +4.59dBNote9) | (Io for Channel 1 +4.59dB) |
|  |  | dB | -4 | -4 | -4 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: The value is corresponding to DRS transmission through LBT operation in Cell3. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.51.3	Test Requirements

In the test, the performance of RSRQ measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency RSRQ measurements for Cell 3 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.19.4.

### A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell

#### A.9.2.52.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD intra frequency RSRQ absolute and relative measurement accuracies in carrier aggregation with frame structure 3 in the configured DMTC occasion are within the specified limits. This test will verify the absolute RSRQ accuracy requirement of the secondary component carrier defined in clause 9.1.19.2. The test will also verify the primary and secondary component carrier relative RSRQ accuracy requirement defined in Clause 9.1.19.4.

#### A.9.2.52.2	Test parameters

In this test case, Cell1 is PCell on the primary component carrier, Cell2 is SCell on the secondary component carrier with frame structure 3 and activated, and Cell3 is the neighboring cell on the same secondary component carrier of Cell2. The test parameters are given in Table A.9.2.52.2-1. The DMTC configuration for Cell2 and Cell3 is provided to the UE in the measDS-Config before the start of the test.

Table A.9.2.52.2-1: Test parameters for FDD RSRQ accuracies of Scell with FS3

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 | 20 |
| DMTC period |  | ms | N/A | 40 | 40 |
| DMTC period offset |  |  | N/A | 10 | 10 |
| Discovery signal occasion duration |  | ms | N/A | 1 | 1 |
| LBT model |  |  | N/A | N/A | A.3.17 |
| Special subframe configurationNote1 |  |  | 6 | N/A | N/A |
| Uplink/downlink configurationNote1 |  |  | 1 | N/A | N/A |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 47-52 | 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | R.0 FS3 | - |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 38-61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | OP.13 FDD | OP.14 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RA Note2 |  |  |  |  |  |
| OCNG_RB Note2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | - |  |
|  | Bands TDD_C |  | -118.5 |  |  |
|  | Bands TDD_E |  | -117.5 |  |  |
|  | Bands FS3_G |  | - | ( for Channel 1 +3.5dB) |  |
|  |  | dB | -4 | -5.52Note9 | -5.52 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -123.5 | - |  |
|  | Bands TDD_C |  | -122.5 |  |  |
|  | Bands TDD_E |  | -121.5 |  |  |
|  | Bands FS3_G |  | - | (RSRP for Cell 1 +3.5dB) | (RSRP for Cell 1 +3.5dB) |
| RSRQNote4 | Bands TDD_A | dBm/15 kHz | -16.25 | - |  |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
|  | Bands FS3_G |  | - | -17.34 Note9 | -17.34 |
| IoNote4 | Bands TDD_A | 5MHz: dBm/4.5MHz 10MHz: dBm/9MHz 20MHz: dBm/18MHz | -90.26 +10log(NRB,c/50) | - |  |
|  | Bands TDD_C |  | -89.26 +10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -88.26 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 +4.59dBNote9) | (Io for Channel 1 +4.59dB) |
|  |  | dB | -4 | -4 | -4 |
| Propagation condition |  | - | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: The value is corresponding to DRS transmission through LBT operation in Cell3. |  |  |  |  |  |

#### A.9.2.52.3	Test Requirements

In the test, the performance of RSRQ measurements is verified from following three perspectives:

-	The absolute accuracy of intra-frequency RSRQ measurements for Cell 3 on the secondary component carrier with frame structure 3 shall fulfil the requirements defined in clause 9.1.19.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.19.4.

### A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.53.1	Test Purpose and Environment

The purpose of this test is to verify that the FDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.53.2	Test parameters

In this set of test cases the PCell and the SCells are on different carrier frequencies. There are five cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carriers are tested by using test parameters specified in Table A.9.2.53.2-1 and Table A.9.2.53.2-2.  In the test, Cell 1 is the PCell, Cell 2, Cell 3 and Cell 4 are the SCells on secondary component carrier SCC1, SCC2 and SCC3 respectively. The SCC1, SCC2 and SCC3 are configured and activated.

Table A.9.2.53.2-1: 4 DL FDD RSRQ carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz:NRB,c = 25 10MHz: NRB,c = 50 20MHz:NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Measurement bandwidth |  |  | 5MHz:10-15 10MHz:22-27 20MHz:47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD |
| PDSCH allocation |  |  | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 | 5MHz:7-17 10MHz:13-36 20MHz:38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -119 | -115.5 | -115.5 |
|  | Bands FDD_C |  | -118.5 | -115 | -115 |
|  | Bands FDD_D |  | -118 | -114.5 | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -117.5 | -114 | -114 |
|  | Bands FDD_G |  | -116.5 | -113 | -113 |
|  | Bands FDD_H |  | -116 | -112.5 | -112.5 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note3 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -125 | -121.5 | -121.5 |
|  | Bands FDD_C |  | -124.5 | -121 | -121 |
|  | Bands FDD_D |  | -124 | -120.5 | -120.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -123.5 | -120 | -120 |
|  | Bands FDD_G |  | -122.5 | -119 | -119 |
|  | Bands FDD_H |  | -122 | -118.5 | -118.5 |
| RSRQNote3 | Bands FDD_A | dB | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) | -87.25+ 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -90.25+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) | -86.75+ 10log(NRB,c /50) |
|  | Bands FDD_C |  | -89.75+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) | -86.25+ 10log(NRB,c /50) |
|  | Bands FDD_D |  | -89.25+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) | -85.75+ 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 6 |  | -88.75+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) | -85.25+ 10log(NRB,c /50) |
|  | Bands FDD_G |  | -87.75+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) | -84.25+ 10log(NRB,c /50) |
|  | Bands FDD_H |  | -87.25+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) | -83.75+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table A.9.2.53.2-2: 4 DL FDD RSRQ carrier aggregation test parameters for cell 4

| Parameter |  | Unit | Cell 4 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz:R.5 FDD 10MHz:R.0 FDD 20MHz:R.4 FDD |
| PDSCH allocation |  |  | 5MHz:7-17 10MHz:13-36 20MHz:38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 FDD 10MHz:R.6 FDD 20MHz:R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz:OP.15 FDD 10MHz:OP.1 FDD 20MHz:OP.11 FDD |
| PBCH_RA |  | dB | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -116 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -115.5 |
|  | Bands FDD_C |  | -115 |
|  | Bands FDD_D |  | -114.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -114 |
|  | Bands FDD_G |  | -113 |
|  | Bands FDD_H |  | -112.5 |
|  |  | dB | -6.0 |
| Note3 |  | dB | -6.0 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -121.5 |
|  | Bands FDD_C |  | -121 |
|  | Bands FDD_D |  | -120.5 |
|  | Bands FDD_E, FDD_F Note 6 |  | -120 |
|  | Bands FDD_G |  | -119 |
|  | Bands FDD_H |  | -118.5 |
| RSRQNote3 | Bands FDD_A | dB | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |
|  | Bands FDD_C |  |  |
|  | Bands FDD_D |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  |  |
|  | Bands FDD_G |  |  |
|  | Bands FDD_H |  |  |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.25+ 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -86.75+ 10log(NRB,c /50) |
|  | Bands FDD_C |  | -86.25+ 10log(NRB,c /50) |
|  | Bands FDD_D |  | -85.75+ 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.25+ 10log(NRB,c /50) |
|  | Bands FDD_G |  | -84.25+ 10log(NRB,c /50) |
|  | Bands FDD_H |  | -83.75+ 10log(NRB,c /50) |
| Propagation condition |  | - | AWGN |
| Antenna Configuration |  | - | 1x2 |
| Timing offset to Cell 1 |  | s | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |
| Time alignment error relative to cell 3 Note 7 |  |  | TAE |
| Time alignment error relative to cell 4 Note 7 |  |  | - |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |

#### A.9.2.53.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC3 for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation

#### A.9.2.54.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.54.2	Test parameters

In this set of test cases the PCell and the SCells are on different carrier frequencies. There are five cells used in this test case. Both RSRQ absolute and relative accuracy requirements of the primary and secondary component carriers are tested by using test parameters specified in Table A.9.2.54.2-1 and Table A.9.2.54.2-2.  In the test, Cell 1 is the PCell, Cell 2, Cell 3 and Cell 4 are the SCells on secondary component carrier SCC1, SCC2 and SCC3 respectively. The SCC1, SCC2 and SCC3 are configured and activated.

Table A.9.2.54.2-1: 4 DL TDD RSRQ carrier aggregation test parameters for cell 1, cell 2 and cell 3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 54 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 54 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 54 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -119.5 | -116 | -116 |
|  | Bands TDD_C |  | -118.5 | -115 | -115 |
|  | Bands TDD_E |  | -117.5 | -114 | -114 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| Note4 |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -125.5 | -122 | -122 |
|  | Bands TDD_C |  | -124.5 | -121 | -121 |
|  | Bands TDD_E |  | -123.5 | -120 | -120 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -17.77 | -17.77 | -17.77 |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -90.75+ 10log(NRB,c /54) | -87.25+ 10log(NRB,c /54) | -87.25+ 10log(NRB,c /54) |
|  | Bands TDD_C |  | -89.75+ 10log(NRB,c /54) | -86.25+ 10log(NRB,c /54) | -86.25+ 10log(NRB,c /54) |
|  | Bands TDD_E |  | -88.75+ 10log(NRB,c /54) | -85.25+ 10log(NRB,c /54) | -85.25+ 10log(NRB,c /54) |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Antenna Configuration |  | - | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table A.9.2.54.2-2: 4 DL TDD RSRQ carrier aggregation test parameters for cell 4

| Parameter |  | Unit | Cell 4 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 4 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 54 20MHz: NRB,c = 100 |
| Special subframe configurationNote1 |  |  | 6 |
| Uplink-downlink configurationNote1 |  |  | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | 5MHz:R.11 TDD 10MHz:R.6 TDD 20MHz:R.10 TDD |
| OCNG Patterns defined in A.3.2.1 |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA |  | dB | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote2 |  |  |  |
| OCNG_RBNote2 |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -116 |
|  | Bands TDD_C |  | -115 |
|  | Bands TDD_E |  | -114 |
|  |  | dB | -6.0 |
| Note4 |  | dB | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -122 |
|  | Bands TDD_C |  | -121 |
|  | Bands TDD_E |  | -120 |
| RSRQNote4 | Bands TDD_A, TDD_C, TDD_E | dB | -17.77 |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25+ 10log(NRB,c /54) |
|  | Bands TDD_C |  | -86.25+ 10log(NRB,c /54) |
|  | Bands TDD_E |  | -85.25+ 10log(NRB,c /54) |
| Propagation condition |  | - | AWGN |
| Antenna Configuration |  | - | 1x2 |
| Timing offset to Cell 1 |  | s | 0 |
| Time alignment error relative to cell 1 Note 7 |  |  | TAE |
| Time alignment error relative to cell 2 Note 7 |  |  | TAE |
| Time alignment error relative to cell 3 Note 7 |  |  | TAE |
| Time alignment error relative to cell 4 Note 7 |  |  | - |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 8: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

#### A.9.2.54.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements defined in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on SCC1 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on SCC2 shall fulfil the requirements defined in clause 9.1.11.2.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on SCC3 shall fulfil the requirements defined in clause 9.1.11.2.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC1 for Cell 2 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC2 for Cell 3 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary component carrier and SCC3 for Cell 4 relative to Cell 1 shall fulfil the requirements defined in clause 9.1.11.3.

### A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.2.55.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carrier specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.2.38, A.9.2.39, A.9.2.40, A.9.2.41, does not need to be tested in the generic duplex-mode test case A.9.2.55.

#### A.9.2.55.2	Test parameters

In this set of test cases there are three cells on three carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, and Cell 3 is activated SCell on channel 3. The parameters for the test are listed in Table A.9.2.55.2-1.

Table A.9.2.55.2-1: 3 DL RSRQ for E-UTRAN in Carrier Aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 |
| Duplex mode |  |  | FDD or TDD | FDD or TDD | FDD or TDD |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 | 6 |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | -116 | -116 |
|  | Bands TDD_C |  | -115 | -115 | -115 |
|  | Bands TDD_E |  | -114 | -114 | -114 |
|  | Bands FDD_A |  | -119.5 | -119.5 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | -119 | -119 |
|  | Bands FDD_C |  | -118.5 | -118.5 | -118.5 |
|  | Bands FDD_D |  | -118 | -118 | -118 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | -117.5 | -117.5 |
|  | Bands FDD_G |  | -116.5 | -116.5 | -116.5 |
|  | Bands FDD_H |  | -116 | -116 | -116 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | -122 | -122 |
|  | Bands TDD_C |  | -121 | -121 | -121 |
|  | Bands TDD_E |  | -120 | -120 | -120 |
|  | Bands FDD_A |  | -125.5 | -125.5 | -125.5 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -125 | -125 | -125 |
|  | Bands FDD_C |  | -124.5 | -124.5 | -124.5 |
|  | Bands FDD_D |  | -124 | -124 | -124 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | -123.5 | -123.5 |
|  | Bands FDD_G |  | -122.5 | -122.5 | -122.5 |
|  | Bands FDD_H |  | -122 | -122 | -122 |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | -17.77 | -17.77 |
|  | Bands TDD_C |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) | s |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

#### A.9.2.55.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.2.56.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.2.45, A.9.2.46, A.9.2.53 A.9.2.54, does not need to be tested in the generic duplex-mode test case A.9.2.56.

#### A.9.2.56.2	Test parameters

In this set of test cases there are four cells on four carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, and Cell 4 is activated SCell on channel 4. The parameters for the test are listed in Table A.9.2.56.2-1.

Table A.9.2.56.2-1: 4 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 |
| Duplex mode |  |  | FDD or TDD | FDD or TDD | FDD or TDD | FDD or TDD |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 | 6 | 6 |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 | 1 | 1 |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | -116 | -116 | -116 |
|  | Bands TDD_C |  | -115 | -115 | -115 | -115 |
|  | Bands TDD_E |  | -114 | -114 | -114 | -114 |
|  | Bands FDD_A |  | -119.5 | -119.5 | -119.5 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | -119 | -119 | -119 |
|  | Bands FDD_C |  | -118.5 | -118.5 | -118.5 | -118.5 |
|  | Bands FDD_D |  | -118 | -118 | -118 | -118 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | -117.5 | -117.5 | -117.5 |
|  | Bands FDD_G |  | -116.5 | -116.5 | -116.5 | -116.5 |
|  | Bands FDD_H |  | -116 | -116 | -116 | -116 |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
|  |  | dB | -6.0 | -6.0 | -6.0 | -6.0 |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | -122 | -122 | -122 |
|  | Bands TDD_C |  | -121 | -121 | -121 | -121 |
|  | Bands TDD_E |  | -120 | -120 | -120 | -120 |
|  | Bands FDD_A |  | -125.5 | -125.5 | -125.5 | -125.5 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -125 | -125 | -125 | -125 |
|  | Bands FDD_C |  | -124.5 | -124.5 | -124.5 | -124.5 |
|  | Bands FDD_D |  | -124 | -124 | -124 | -124 |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | -123.5 | -123.5 | -123.5 |
|  | Bands FDD_G |  | -122.5 | -122.5 | -122.5 | -122.5 |
|  | Bands FDD_H |  | -122 | -122 | -122 | -122 |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | -17.77 | -17.77 | -17.77 |
|  | Bands TDD_C |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | -17.77 | -17.77 | -17.77 |
|  | Bands FDD_B1, FDD_B2 Note 11 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Timing offset to Cell 1 |  | s | - | 0 | 0 | 0 |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void. Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.2.56.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.2.57.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.9.2.47, A.9.2.48, A.9.2.49 A.9.2.50, does not need to be tested in the generic duplex-mode test case A.9.2.57.

#### A.9.2.57.2	Test parameters

In this set of test cases there are five cells on five carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, Cell 4 is activated SCell on channel 4, and Cell 5 is activated SCell on channel 5. The parameters for the test are listed in Table A.9.2.57.2-1.

Table A.9.2.57.2-1: 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cells |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 | 3 | 4 | 5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 | 5 |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  |  |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| TDD Special subframe configurationNote1 |  |  | 6 | 6 |  |  |  |
| TDD Uplink/downlink configurationNote1 |  |  | 1 | 1 |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  |  |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  |  |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | -116 |  |  |  |
|  | Bands TDD_C |  | -115 | -115 |  |  |  |
|  | Bands TDD_E |  | -114 | -114 |  |  |  |
|  | Bands FDD_A |  | -119.5 | -119.5 |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | -119 |  |  |  |
|  | Bands FDD_C |  | -118.5 | -118.5 |  |  |  |
|  | Bands FDD_D |  | -118 | -118 |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | -117.5 |  |  |  |
|  | Bands FDD_G |  | -116.5 | -116.5 |  |  |  |
|  | Bands FDD_H |  | -116 | -116 |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | -122 |  |  |  |
|  | Bands TDD_C |  | -121 | -121 |  |  |  |
|  | Bands TDD_E |  | -120 | -120 |  |  |  |
|  | Bands FDD_A |  | -125.5 | -125.5 |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -125 | -125 |  |  |  |
|  | Bands FDD_C |  | -124.5 | -124.5 |  |  |  |
|  | Bands FDD_D |  | -124 | -124 |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | -123.5 |  |  |  |
|  | Bands FDD_G |  | -122.5 | -122.5 |  |  |  |
|  | Bands FDD_H |  | -122 | -122 |  |  |  |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | -17.77 |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | -17.77 |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) |  |  |  |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |
| Timing offset to Cell 1 |  | s | - | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.2.57.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 5 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.2.58.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.58.2	Test parameters

In this set of test cases there are six cells on six carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, Cell 4 is activated SCell on channel 4, Cell 5 is activated SCell on channel 5 and Cell 6 is activated SCell on channel 6. The parameters for the test are listed in Table A.9.2.58.2-1.

Table A.9.2.58.2-1: 6 DL RSRQ for E-UTRAN in Carrier Aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cells |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 | 3 | 4 | 5 | 6 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  |  |  |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |
| TDD Special subframe configurationNote1 |  |  | - | 6 |  |  |  |  |
| TDD uplink/downlink configurationNote1 |  |  | - | 1 |  |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  |  |  |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  |  |  |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | -116 |  |  |  |  |
|  | Bands TDD_C |  | -115 | -115 |  |  |  |  |
|  | Bands TDD_E |  | -114 | -114 |  |  |  |  |
|  | Bands FDD_A |  | -119.5 | -119.5 |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | -119 |  |  |  |  |
|  | Bands FDD_C |  | -118.5 | -118.5 |  |  |  |  |
|  | Bands FDD_D |  | -118 | -118 |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | -117.5 |  |  |  |  |
|  | Bands FDD_G |  | -116.5 | -116.5 |  |  |  |  |
|  | Bands FDD_H |  | -116 | -116 |  |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |  |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | -122 |  |  |  |  |
|  | Bands TDD_C |  | -121 | -121 |  |  |  |  |
|  | Bands TDD_E |  | -120 | -120 |  |  |  |  |
|  | Bands FDD_A |  | -125.5 | -125.5 |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -125 | -125 |  |  |  |  |
|  | Bands FDD_C |  | -124.5 | -124.5 |  |  |  |  |
|  | Bands FDD_D |  | -124 | -124 |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | -123.5 |  |  |  |  |
|  | Bands FDD_G |  | -122.5 | -122.5 |  |  |  |  |
|  | Bands FDD_H |  | -122 | -122 |  |  |  |  |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | -17.77 |  |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | -17.77 |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) |  |  |  |  |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |  |  |  |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |
| Timing offset to Cell 1 |  | s | - | 0 |  |  |  |  |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE | TAE | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | TAE | TAE |
| Time alignment error relative to cell 5 Note 10 |  |  | - | - | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.58.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 6 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 5 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes

#### A.9.2.59.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy in carrier aggregation is within the specified limits under AWGN propagation conditions. This test will verify the absolute accuracy of intra-frequency RSRQ measurements for the primary component carrier specified in clause 9.1.11.1, the absolute accuracy of intra-frequency RSRQ measurements for the secondary component carriers specified in clause 9.1.11.2 and also the relative inter-frequency RSRQ accuracy requirement between primary and secondary component carriers specified in clause 9.1.11.3.

#### A.9.2.59.2	Test parameters

In this set of test cases there are seven cells on seven carrier frequencies. Cell 1 is PCell on channel 1, Cell 2 is activated SCell on channel 2, Cell 3 is activated SCell on channel 3, Cell 4 is activated SCell on channel 4, Cell 5 is activated SCell on channel 5, Cell 6 is activated SCell on channel 6 and Cell 7 is activated SCell on channel 7. The parameters for the test are listed in Table A.9.2.59.2-1.

Table A.9.2.59.2-1: 7 DL RSRQ for E-UTRAN in Carrier Aggregation test parameters

| Parameter |  | Unit | Cell 1 | Cells |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2 | 3 | 4 | 5 | 6 | 7 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Duplex mode |  |  | FDD or TDD | FDD or TDD |  |  |  |  |  |
| BWchannel |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| TDD Special subframe configurationNote1 |  |  | - | 6 |  |  |  |  |  |
| TDD uplink/downlink configurationNote1 |  |  | - | 1 |  |  |  |  |  |
| Measurement bandwidth |  |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  |  |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1 |  |  | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD | 5MHz FDD: R.5 FDD 10MHz FDD: R.0 FDD 20MHz FDD: R.4 FDD 5MHz TDD: R.4 TDD 10MHz TDD: R.0 TDD 20MHz TDD: R.3 TDD |  |  |  |  |  |
| PDSCH allocation |  |  | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 | 5MHz: 7-17 10MHz: 13-36 20MHz: 38-61 |  |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2 |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  |  |  |  |
| OCNG Patterns defined in A.3.2 |  |  | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD | 5MHz FDD: OP.15 FDD 10MHz FDD: OP.1 FDD 20MHz FDD: OP.11 FDD 5MHz TDD: OP.9 TDD 10MHz TDD: OP.1 TDD 20MHz TDD: OP.7 TDD |  |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/ 15kHz | -116 | -116 |  |  |  |  |  |
|  | Bands TDD_C |  | -115 | -115 |  |  |  |  |  |
|  | Bands TDD_E |  | -114 | -114 |  |  |  |  |  |
|  | Bands FDD_A |  | -119.5 | -119.5 |  |  |  |  |  |
|  | Bands FDD_B1, FDD_B2 Note 11 |  | -119 | -119 |  |  |  |  |  |
|  | Bands FDD_C |  | -118.5 | -118.5 |  |  |  |  |  |
|  | Bands FDD_D |  | -118 | -118 |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -117.5 | -117.5 |  |  |  |  |  |
|  | Bands FDD_G |  | -116.5 | -116.5 |  |  |  |  |  |
|  | Bands FDD_H |  | -116 | -116 |  |  |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |  |  |
|  |  | dB | -6.0 | -6.0 |  |  |  |  |  |
| RSRPNote4 | Bands TDD_A | dBm/ 15kHz | -122 | -122 |  |  |  |  |  |
|  | Bands TDD_C |  | -121 | -121 |  |  |  |  |  |
|  | Bands TDD_E |  | -120 | -120 |  |  |  |  |  |
|  | Bands FDD_A |  | -125.5 | -125.5 |  |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -125 | -125 |  |  |  |  |  |
|  | Bands FDD_C |  | -124.5 | -124.5 |  |  |  |  |  |
|  | Bands FDD_D |  | -124 | -124 |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -123.5 | -123.5 |  |  |  |  |  |
|  | Bands FDD_G |  | -122.5 | -122.5 |  |  |  |  |  |
|  | Bands FDD_H |  | -122 | -122 |  |  |  |  |  |
| RSRQNote4 | Bands TDD_A | dB | -17.77 | -17.77 |  |  |  |  |  |
|  | Bands TDD_C |  |  |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |  |  |
|  | Bands FDD_A |  | -17.77 | -17.77 |  |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A | dBm/ BWchannel | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands TDD_C |  | -86.25 + 10log(NRB,c/50) | -86.25 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands TDD_E |  | -85.25 + 10log(NRB,c/50) | -85.25 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_A |  | -90.75 + 10log(NRB,c/50) | -90.75 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_B, FDD_B2 Note 11 |  | -90.25 + 10log(NRB,c/50) | -90.25 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_C |  | -89.75 + 10log(NRB,c/50) | -89.75 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_D |  | -89.25 + 10log(NRB,c/50) | -89.25 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_E, Bands FDD_F Note 6 |  | -88.75 + 10log(NRB,c/50) | -88.75 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_G |  | -87.75 + 10log(NRB,c/50) | -87.75 + 10log(NRB,c/50) |  |  |  |  |  |
|  | Bands FDD_H |  | -87.25 + 10log(NRB,c/50) | -87.25 + 10log(NRB,c/50) |  |  |  |  |  |
| Propagation Condition |  |  | AWGN | AWGN |  |  |  |  |  |
| Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 |  | s | - | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 10 |  |  | - | TAE | TAE | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 2 Note 10 |  |  | - | - | TAE | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 3 Note 10 |  |  | - | - | - | TAE | TAE | TAE | TAE |
| Time alignment error relative to cell 4 Note 10 |  |  | - | - | - | - | TAE | TAE | TAE |
| Time alignment error relative to cell 5 Note 10 |  |  | - | - | - | - | - | TAE | TAE |
| Time alignment error relative to cell 6 Note 10 |  |  | - | - | - | - | - | - | TAE |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 8: Void Note 9: E-UTRA operating band groups are as defined in Section 3.5. Note 10: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 11: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |

#### A.9.2.59.3	Test Requirements

In the test, the RSRQ measurement accuracy in carrier aggregation shall fulfil the requirements in clause 9.1.11.1, 9.1.11.2, and 9.1.11.3.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 1 on the primary component carrier shall fulfil the requirements specified in clause 9.1.11.1.

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 2 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 3 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 4 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 5 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 6 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The absolute accuracy of intra-frequency RSRQ measurements of Cell 7 on the secondary component carrier shall fulfil the requirements specified in clause 9.1.11.2

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 2 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 3 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 4 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 5 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 6 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

-	The relative accuracy of inter-frequency RSRQ measurements between the primary and secondary component carriers for Cell 7 relative to Cell 1 shall fulfil the requirements specified in clause 9.1.11.3.

### A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements

#### A.9.2.60.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.5B.2.

#### A.9.2.60.2	Test parameters

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

In this test case all cells are on the same carrier frequency. The absolute accuracy of RSRQ intra frequency measurement is tested by using the parameters in Table A.9.2.60.2-1. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

Table A.9.2.60.2-1: RSRQ FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | [-84.76] | [-84.76] | [-103.85] | [-103.85] | [-116] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | [-115.5] |  |
|  | Bands FDD_C |  |  |  |  |  | [-115] |  |
|  | Bands FDD_D |  |  |  |  |  | [-114.5] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | [-114] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | [-113] |  |
|  | Bands FDD_H |  |  |  |  |  | [-112.5] |  |
|  |  | dB | [-1.76] | [-1.76] | [-4.7] | [-4.7] | [-5.46] | [-5.46] |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | [-81.76] | [-81.76] | [-106.75] | [-106.75] | [-120] | [-120] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | [-119.5] | [-119.5] |
|  | Bands FDD_C |  |  |  |  |  | [-119] | [-119] |
|  | Bands FDD_D |  |  |  |  |  | [-118.5] | [-118.5] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | [-118] | [-118] |
|  | Bands FDD_G Note 7 |  |  |  |  |  | [-117] | [-117] |
|  | Bands FDD_H |  |  |  |  |  | [-116.5] | [-116.5] |
| RSRQNote3 | Bands FDD_A Note 8 | dB | [-14.77] | [-14.77] | [-16.76] | [-16.76] | [-17.34] | [-17.34] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | [-50] | [-50] | [-73] | [-73] | [-85.67] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  | [-85.17] |  |
|  | Bands FDD_C |  |  |  |  |  | [-84.67] |  |
|  | Bands FDD_D |  |  |  |  |  | [-84.17] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | [-83.67] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | [-82.67] |  |
|  | Bands FDD_H |  |  |  |  |  | [-82.17] |  |
|  |  | dB | [3] | [3] | [-2.9] | [-2.9] | [-4] | [-4] |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.2.60.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in section 9.1.5B.2.

### A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier

#### A.9.2.61.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6B.2.

#### A.9.2.61.2	Test parameters

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

In this test case the two cells are on different carrier frequencies. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.61.2-1. In all tests, Cell 1 is the serving cell and Cell 2 the target cell.

Table A.9.2.61.2-1: RSRQ FDD—FDD Inter frequency test parameters

| Parameter |  |  | Unit |  | Test 1 |  |  |  | Test 2 |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  | 2 |  | 1 |  | 2 |  | 1 |  |  | 2 |
| BWchannel |  |  | MHz |  | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  |  | 10 |
| Measurement bandwidth |  |  |  | 22—27 |  |  |  | 22—27 |  |  |  | 22—27 |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD |  |  |  | R.6 FDD |  |  |  | R.6 FDD |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD |  |  |
| PBCH_RA |  |  | dB |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz |  | [-80] |  | [-80] |  | [-104.70] |  | [-104.70] |  | [-119.5] |  |  | [-119.5] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-119] |  |  | [-119] |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-118.5] |  |  | [-118.5] |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-118] |  |  | [-118] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-117.5] |  |  | [-117.5] |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-116.5] |  |  | [-116.5] |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-116] |  |  | [-116] |
|  |  |  | dB |  | [-1.75] |  | [-1.75] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |  | [-4.0] |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz |  | [-81.75] |  | [-81.75] |  | [-108.70] |  | [-108.70] |  | [-123.5] |  |  | [-123.5] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-123] |  |  | [-123] |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-122.5] |  |  | [-122.5] |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-122] |  |  | [-122] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-121.5] |  |  | [-121.5] |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-120.5] |  |  | [-120.5] |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-120] |  |  | [-120] |
| RSRQNote3 | Bands FDD_A Note 8 |  | dB |  | [-14.76] |  | [-14.76] |  | [-16.25] |  | [-16.25] |  | [-16.25] |  |  | [-16.25] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz |  | [-50] |  | [-50] |  | [-75.46] |  | [-75.46] |  | [-90.26] |  |  | [-90.26] |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-89.76] |  |  | [-89.76] |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-89.26] |  |  | [-89.26] |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-88.76] |  |  | [-88.76] |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-88.26] |  |  | [-88.26] |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-87.26] |  |  | [-87.26] |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-86.76] |  |  | [-86.76] |
|  |  |  | dB |  | [-1.75] |  | [-1.75] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |  | [-4.0] |
| Propagation condition |  |  | - |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.61.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in sections 9.1.6B.2.

### A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier

#### A.9.2.62.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.6B.3.

#### A.9.2.62.2	Test parameters

The requirements in this clause are applicable for a UE:

- in state RRC_IDLE

- that is synchronised to the cell that is measured.

The requirements apply for UE supporting ca-IdleModeMeasurements, when configured with measIdleConfig and while T331 timer is running.

In this test case the two cells are on different carrier frequencies. Both RSRQ inter frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.2.62.2-1. In all tests, Cell 1 is the serving cell and Cell 2 the target cell.

Table A.9.2.62.2-1: RSRQ FDD—FDD Inter frequency test parameters

| Parameter |  |  | Unit |  | Test 1 |  |  |  | Test 2 |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  | 2 |  | 1 |  | 2 |  | 1 |  |  | 2 |  |
| BWchannel |  |  | MHz |  | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  |  | 10 |  |
| Measurement bandwidth |  |  |  | 22—27 |  |  |  | 22—27 |  |  |  | 22—27 |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD |  |  |  | R.6 FDD |  |  |  | R.6 FDD |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD |  |  |
| PBCH_RA |  |  | dB |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz |  | [-80] |  | [-80] |  | [-104.70] |  | [-104.70]8 |  | [-119.5] |  |  | [-119.5] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-119] |  |  | [-119] |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-118.5] |  |  | [-118.5] |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-118] |  |  | [-118] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-117.5] |  |  | [-117.5] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-116.5] |  |  | [-116.5] |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-116] |  |  | [-116] |  |
|  |  |  | dB |  | [-1.75] |  | [-1.75] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |  | [-4.0] |  |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz |  | [-81.75] |  | [-81.75] |  | [-108.70] |  | [-108.70] |  | [-123.5] |  |  | [-123.5] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-123] |  |  | [-123] |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-122.5] |  |  | [-122.5] |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-122] |  |  | [-122] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-121.5] |  |  | [-121.5] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-120.5] |  |  | [-120.5] |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-120] |  |  | [-120] |  |
| RSRQNote3 | Bands FDD_A Note 8 |  | dB |  | [-14.76] |  | [-14.76] |  | [-16.25] |  | [-16.25] |  | [-16.25] |  |  | [-16.25] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz |  | [-50] |  | [-50] |  | [-75.46] |  | [-75.46] |  | [-90.26] |  |  | [-90.26] |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |  |  |  |  |  | [-89.76] |  |  | [-89.76] |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  | [-89.26] |  |  | [-89.26] |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  | [-88.76] |  |  | [-88.76] |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  | [-88.26] |  |  | [-88.26] |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |  |  |  |  |  | [-87.26] |  |  | [-87.26] |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  | [-86.76] |  |  | [-86.76] |  |
|  |  |  | dB |  | [-1.75] |  | [-1.75] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |  | [-4.0] |  |
| Propagation condition |  |  | - |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.62.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in sections 9.1.6B.3.

## A.9.3	UTRAN FDD CPICH RSCP

### A.9.3.1	E-UTRAN FDD

#### A.9.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CPICH RSCP absolute measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.2.1. There are two different test setups with different UTRAN parameters.

#### A.9.3.1.2	Parameters

The test parameters are given in Tables A.9.3.1.2-1, A.9.3.1.2-2 and A.9.3.1.2-3 below.

Table A.9.3.1.2-1: General test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN FDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 1 | One UTRAN FDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRAN cell 1 on RF channel number 1 |
| Neighbor cells |  | Cell 2 | UTRAN cell 2 on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRAN FDD) measurement quantity |  | CPICH RSCP |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CP length |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |

Table A.9.3.1.2-2: E-UTRAN FDD cell specific test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN FDD

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -94 |  |
|  | dB | 4 |  |
| SCH_RP Note 3 | dBm/15 kHz | -94 |  |
|  | dB | 4 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.9.3.1.2-3: UTRAN FDD cell specific test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN FDD

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 |
| CPICH_Ec/Ior |  | dB | -10 | -10 |
| PCCPCH_Ec/Ior |  | dB | -12 | -12 |
| SCH_Ec/Ior |  | dB | -12 | -12 |
| PICH_Ec/Ior |  | dB | -15 | -15 |
| DPCH_Ec/Ior |  | dB | - | - |
| OCNS_Ec/Ior |  | dB | -0.94 | -0.94 |
| Ioc | Band I, IV, VI, X, XI, XIX, XXI | dBm/3.84 MHz | -60.00 | -94.46 |
|  | Band II, V, VII |  |  | -92.46 |
|  | Band XXV, XXVI |  |  | -90.96 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -91.46 |
|  | Band IX (Note 2) |  |  | -93.46 |
| Îor/Ioc |  | dB | 9.54 | -9.54 |
| CPICH RSCP, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm | -60.46 | -114.0 |
|  | Band II, V, VII |  |  | -112.0 |
|  | Band XXV, XXVI |  |  | -110.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -111.0 |
|  | Band IX (Note 2) |  |  | -113.0 |
| Io, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm/3.84 MHz | -50.00 | -94.0 |
|  | Band II, V, VII |  |  | -92.0 |
|  | Band XXV, XXVI |  |  | -90.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -91.0 |
|  | Band IX (Note 2) |  |  | -93.0 |
| Propagation condition |  | - | AWGN | AWGN |
| NOTE 1: CPICH RSCP and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 3: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |
| Tests shall be done sequentially. Test 1 shall be done first. After test 1 has been executed test parameters for test 2 shall be set within 5 seconds so that UE does not loose the Cell 2 in between the tests. |  |  |  |  |

#### A.9.3.1.3	Test Requirements

The CPICH RSCP measurement absolute accuracy shall meet the requirements in Clause 9.2.1.

#### A.9.3.2 E-UTRAN TDD

#### A.9.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CPICH RSCP absolute measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.2.1. There are three different test setups with different UTRAN parameters.

#### A.9.3.2.2	Parameters

The test parameters are given in Tables A.9.3.2.2-1, A.9.3.2.2-2 and A.9.3.2.2-3 below.

Table A.9.3.2.2-1: General test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN TDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 1 | One UTRAN FDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRAN cell 1 on RF channel number 1 |
| Neighbor cells |  | Cell 2 | UTRAN cell 2 on RF channel number 1 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRAN FDD) measurement quantity |  | CPICH RSCP |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CP length |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |

Table A.9.3.2.2-2: E-UTRAN TDD cell specific test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN TDD

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Special subframe configurationNote1 |  | 6 |  |
| Uplink-downlink configurationNote1 |  | 1 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 2 | dB |  |  |
| OCNG_RBNote 2 | dB |  |  |
| Note 3 | dBm/15 kHz | -98 |  |
| RSRP Note 4 | dBm/15 kHz | -94 |  |
|  | dB | 4 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 |  |
|  | dB | 4 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.9.3.2.2-3: UTRAN FDD cell specific test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN TDD

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 |
| CPICH_Ec/Ior |  | dB | -10 | -10 |
| PCCPCH_Ec/Ior |  | dB | -12 | -12 |
| SCH_Ec/Ior |  | dB | -12 | -12 |
| PICH_Ec/Ior |  | dB | -15 | -15 |
| DPCH_Ec/Ior |  | dB | - | - |
| OCNS_Ec/Ior |  | dB | -0.94 | -0.94 |
| Ioc | Band I, IV, VI, X, XI, XIX, XXI | dBm/3.84 MHz | -60.00 | -94.46 |
|  | Band II, V, VII |  |  | -92.46 |
|  | Band XXV, XXVI |  |  | -90.96 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -91.46 |
|  | Band IX (Note 2) |  |  | -93.46 |
| Îor/Ioc |  | dB | 9.54 | -9.54 |
| CPICH RSCP, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm | -60.46 | -114.0 |
|  | Band II, V, VII |  |  | -112.0 |
|  | Band XXV, XXVI |  |  | -110.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -111.0 |
|  | Band IX (Note 2) |  |  | -113.0 |
| Io, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm/3.84 MHz | -50.00 | -94.0 |
|  | Band II, V, VII |  |  | -92.0 |
|  | Band XXV, XXVI |  |  | -90.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  | -91.0 |
|  | Band IX (Note 2) |  |  | -93.0 |
| Propagation condition |  | - | AWGN | AWGN |
| NOTE 1: CPICH RSCP and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 3: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |
| Tests shall be done sequentially. Test 1 shall be done first. After test 1 has been executed test parameters for test 2 shall be set within 5 seconds so that UE does not loose the Cell 2 in between the tests. |  |  |  |  |

#### A.9.3.2.3	Test Requirements

The CPICH RSCP measurement absolute accuracy shall meet the requirements in Clause 9.2.1.

### A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth

#### A.9.3.3.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.3.1.1.

#### A.9.3.3.2	Parameters

The parameters of this test are the same as defined in Subclause A.9.3.1.2 except that the values of the parameters in the Table A.9.3.3.2-1 will replace the values of the corresponding parameters in A.9.3.1.2-1, and the values of E-UTRAN FDD cell specific parameters in the Table A.9.3.3.2-2 shall be adopted, and the values of UTRA FDD cell specific parameters shall be reused as defined in Table A.9.3.1.2-3 of Subclause A.9.3.1.2.

Table A.9.3.3.2-1: General test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN FDD for 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in clause A.3.1.2.1 |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Note 1: See Table A.9.3.1.2-1 for other general test parameters. |  |  |  |

Table A.9.3.3.2-2: E-UTRAN FDD cell specific test parameters for UTRAN FDD CPICH RSCP absolute measurement accuracy test in E-UTRAN FDD for 5MHz bandwidth

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  |  | 1 |  |
| BWchannel |  | MHz | 5 |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) |  |  | OP.15 FDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  | dB |  |  |
| PSS_RA |  | dB |  |  |
| SSS_RA |  | dB |  |  |
| PCFICH_RB |  | dB |  |  |
| PHICH_RA |  | dB |  |  |
| PHICH_RB |  | dB |  |  |
| PDCCH_RA |  | dB |  |  |
| PDCCH_RB |  | dB |  |  |
| PDSCH_RA |  | dB |  |  |
| PDSCH_RB |  | dB |  |  |
| OCNG_RANote 1 |  | dB |  |  |
| OCNG_RBNote 1 |  | dB |  |  |
| Note 2 | FDD_N | dBm/15 kHz | -98 |  |
| RSRP Note 3 | FDD_N | dBm/15 kHz | -94 |  |
|  |  | dB | 4 |  |
| SCH_RP Note 3 | FDD_N | dBm/15 kHz | -94 |  |
|  |  | dB | 4 |  |
| IoNote3 | FDD_N | dBm/4.5 MHz | -67.8 |  |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.9.3.3.3	Test Requirements

The test requirements defined in section A.9.3.1.3 shall apply to this test case.

## A.9.4	UTRAN FDD CPICH Ec/No

### A.9.4.1	E-UTRAN FDD

#### A.9.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CPICH Ec/No absolute measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.2.3. There are three different test setups with different UTRAN parameters.

#### A.9.4.1.2	Parameters

The test parameters are given in Tables A.9.4.1.2-1, A.9.4.1.2-2 and A.9.4.1.2-3 below.

Table A.9.4.1.2-1: General test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN FDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 1 | One UTRAN FDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRAN cell 1 on RF channel number 1 |
| Neighbor cells |  | Cell 2 | UTRAN cell 2 on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRAN FDD) measurement quantity |  | CPICH Ec/N0 |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CP length |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |

Table A.9.4.1.2-2: E-UTRAN FDD cell specific test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN FDD

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 |  |  |
|  | dB | 4 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -94 |  |  |
|  | dB | 4 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.9.4.1.2-3: UTRAN FDD cell specific test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN FDD

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 | Cell 2 |
| CPICH_Ec/Ior |  | dB | -10 | -10 | -10 |
| PCCPCH_Ec/Ior |  | dB | -12 | -12 | -12 |
| SCH_Ec/Ior |  | dB | -12 | -12 | -12 |
| PICH_Ec/Ior |  | dB | -15 | -15 | -15 |
| DPCH_Ec/Ior |  | dB | - | - | - |
| OCNS_Ec/Ior |  | dB | -0.94 | -0.94 | -0.94 |
| Ioc | Band I, IV, VI, X, XI, XIX, XXI | dBm/ 3.84 MHz | -52.22 | -87.27 | -94.46 |
|  | Band II, V, VII |  |  |  | -92.46 |
|  | Band XXV, XXVI |  |  |  | -90.96 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  |  | -91.46 |
|  | Band IX (Note 2) |  |  |  | -93.46 |
| Îor/Ioc |  | dB | -1.75 | -4.7 | -9.54 |
| CPICH Ec/Io, Note 1 |  | dBm | -14.0 | -16.0 | -20.0 |
| Io, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm/ 3.84 MHz | -50 | -86 | -94 |
|  | Band II, V, VII |  |  |  | -92.0 |
|  | Band XXV, XXVI |  |  |  | -90.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  |  | -91.0 |
|  | Band IX (Note 2) |  |  |  | -93 |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| NOTE 1: CPICH Ec/Io and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 3: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |
| Tests shall be done sequentially. Test 1 shall be done first. After test 1 has been executed test parameters for tests 2 and 3 shall be set within 5 seconds so that UE does not loose the Cell 2 in between the tests. |  |  |  |  |  |

#### A.9.4.1.3	Test Requirements

The CPICH Ec/No measurement absolute accuracy shall meet the requirements in Clause 9.2.3.

The effect of assumed thermal noise and noise generated in the receiver (-99 dBm for frequency bands I, IV, VI, X, XI, XIX and XXI; -98 dBm for frequency band IX, -97dBm for frequency bands II, V and VII; -95.5dBm for frequency band XXV and XXVI; and -96dBm for frequency band III) shall be added into the required accuracy. The test requirements for the absolute CPICH_Ec/Io measurement are shown in Table A.9.4.1.3-1.

Table A.9.4.1.3-1: CPICH_Ec/Io absolute accuracy

| Parameter | Unit | Accuracy [dB] |  | Conditions |
| --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | Io [dBm/3,84 MHz] |  |
| CPICH_Ec/Io | dB | -2.7…1.5 for -14 CPICH Ec/Io -3.2…2 for -16 CPICH Ec/Io < -14 -4.2…3 for -20 CPICH Ec/Io < -16 | -4.2…3 | -94...-87(Band I, IV, VI, X, XI, XIX, XXI) -92…-85 (Band II, V, VII) -90.5…-83.5 (Band XXV, XXVI (Note 2)) -91…-84 (Band III, VIII, XII, XIII, XIV, XX, XXII) 93…-86 (Band IX (Note 1)) |
|  |  | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | -87...-50(Band I, IV, VI, X, XI, XIX, XXI) -85…-50 (Band II, V, VII) -83.5…-50 (Band XXV, XXVI (Note 2)) -84…-50 (Band III, VIII, XII, XIII, XIV, XX, XXII) -86…-50 (Band IX (Note 1)) |  |
| NOTE1: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 2: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

### A.9.4.2	E-UTRAN TDD

#### A.9.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CPICH Ec/No absolute measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.2.3. There are three different test setups with different UTRAN parameters.

#### A.9.4.2.2	Parameters

The test parameters are given in Tables A.9.4.2.2-1, A.9.4.2.2-2 and A.9.4.2.2-3 below.

Table A.9.4.2.2-1: General test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN TDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 1 | One UTRAN FDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRAN cell 1 on RF channel number 1 |
| Neighbor cells |  | Cell 2 | UTRAN cell 2 on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRAN FDD) measurement quantity |  | CPICH Ec/N0 |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CP length |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |

Table A.9.4.2.2-2: E-UTRAN TDD cell specific test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN TDD

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configurationNote1 |  | 6 |  |  |
| Uplink-downlink configurationNote1 |  | 1 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 |  |  |
|  | dB | 4 |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 |  |  |
|  | dB | 4 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.9.4.2.2-3: UTRAN FDD cell specific test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN TDD

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 | Cell 2 |
| CPICH_Ec/Ior |  | dB | -10 | -10 | -10 |
| PCCPCH_Ec/Ior |  | dB | -12 | -12 | -12 |
| SCH_Ec/Ior |  | dB | -12 | -12 | -12 |
| PICH_Ec/Ior |  | dB | -15 | -15 | -15 |
| DPCH_Ec/Ior |  | dB | - | - | - |
| OCNS_Ec/Ior |  | dB | -0.94 | -0.94 | -0.94 |
| Ioc | Band I, IV, VI, X, XI, XIX, XXI | dBm/ 3.84 MHz | -52.22 | -87.27 | -94.46 |
|  | Band II, V, VII |  |  |  | -92.46 |
|  | Band XXV, XXVI |  |  |  | -90.96 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  |  | -91.46 |
|  | Band IX (Note 2) |  |  |  | -93.46 |
| Îor/Ioc |  | dB | -1.75 | -4.7 | -9.54 |
| CPICH Ec/Io, Note 1 |  | dBm | -14.0 | -16.0 | -20.0 |
| Io, Note 1 | Band I, IV, VI, X, XI, XIX, XXI | dBm/ 3.84 MHz | -50 | -86 | -94 |
|  | Band II, V, VII |  |  |  | -92.0 |
|  | Band XXV, XXVI |  |  |  | -90.5 (Note 3) |
|  | Band III, VIII, XII, XIII, XIV, XX, XXII |  |  |  | -91.0 |
|  | Band IX (Note 2) |  |  |  | -93 |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| NOTE 1: CPICH Ec/Io and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 3: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |
| Tests shall be done sequentially. Test 1 shall be done first. After test 1 has been executed test parameters for tests 2 and 3 shall be set within 5 seconds so that UE does not loose the Cell 2 in between the tests. |  |  |  |  |  |

#### A.9.4.2.3	Test Requirements

The CPICH Ec/No measurement absolute accuracy shall meet the requirements in Clause 9.2.3.

The effect of assumed thermal noise and noise generated in the receiver (-99 dBm for frequency bands I, IV, VI, X, XI, XIX and XXI; -98 dBm for frequency band IX, -97dBm for frequency bands II, V and VII; -95.5dBm for frequency band XXV and XXVI; and -96dBm for frequency band III) shall be added into the required accuracy. The test requirements for the absolute CPICH_Ec/Io measurement are shown in Table A.9.4.2.3-1.

Table A.9.4.2.3-1: CPICH_Ec/Io absolute accuracy

| Parameter | Unit | Accuracy [dB] |  | Conditions |
| --- | --- | --- | --- | --- |
|  |  | Normal condition | Extreme condition | Io [dBm/3,84 MHz] |  |
| CPICH_Ec/Io | dB | -2.7…1.5 for -14 CPICH Ec/Io -3.2…2 for -16 CPICH Ec/Io < -14 -4.2…3 for -20 CPICH Ec/Io < -16 | -4.2…3 | -94...-87(Band I, IV, VI, X, XI, XIX, XXI) -92…-85 (Band II, V, VII) -90.5…-83.5 (Band XXV, XXVI (Note 2)) -91…-84 (Band III, VIII, XII, XIII, XIV, XX, XXII) 93…-86 (Band IX (Note 1)) |
|  |  | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | -87...-50(Band I, IV, VI, X, XI, XIX, XXI) -85…-50 (Band II, V, VII) -83.5…-50 (Band XXV, XXVI (Note 2)) -84…-50 (Band III, VIII, XII, XIII, XIV, XX, XXII) -86…-50 (Band IX (Note 1)) |  |
| NOTE1: For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE. NOTE 2: The test parameter is modified by -1.5 dB when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

### A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth

#### A.9.4.3.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.4.1.1.

#### A.9.4.3.2	Parameters

The parameters of this test are the same as defined in Subclause A.9.4.1.2 except that the values of the parameters in the Table A.9.4.3.2-1 will replace the values of the corresponding parameters in A.9.4.1.2-1, and the values of E-UTRAN FDD cell specific parameters in the Table A.9.4.3.2-2 shall be adopted, and the values of UTRA FDD cell specific parameters shall be reused as defined in Table A.9.4.1.2-3 of Subclause A.9.4.1.2.

Table A.9.4.3.2-1: General test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN FDD for 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in clause A.3.1.2.1 |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Note 1: See Table A.9.4.1.2-1 for other general test parameters. |  |  |  |

Table A.9.4.3.2-2: E-UTRAN FDD cell specific test parameters for UTRAN FDD CPICH Ec/No absolute measurement accuracy test in E-UTRAN FDD for 5MHz bandwidth

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  |  | 1 |  |  |
| BWchannel |  | MHz | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) |  |  | OP.15 FDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  | dB |  |  |  |
| PSS_RA |  | dB |  |  |  |
| SSS_RA |  | dB |  |  |  |
| PCFICH_RB |  | dB |  |  |  |
| PHICH_RA |  | dB |  |  |  |
| PHICH_RB |  | dB |  |  |  |
| PDCCH_RA |  | dB |  |  |  |
| PDCCH_RB |  | dB |  |  |  |
| PDSCH_RA |  | dB |  |  |  |
| PDSCH_RB |  | dB |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |
| Note 2 | FDD_N | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | FDD_N | dBm/15 kHz | -94 |  |  |
|  |  | dB | 4 |  |  |
| SCH_RP Note 3 | FDD_N | dBm/15 kHz | -94 |  |  |
|  |  | dB | 4 |  |  |
| IoNote3 | FDD_N | dBm/4.5 MHz | -67.8 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.9.4.3.3	Test Requirements

The test requirements defined in section A.9.4.1.3 shall apply to this test case.

## A.9.5	UTRAN TDD measurement

### A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD

#### A.9.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UTRAN TDD P-CCPCH RSCP measurement absolute accuracy is within the specified limits. This test will verify the requirements in clause 9.3.1 and applies to UE supporting this capability.

Gap pattern configuration with id #1 as specified in Table 8.1.2.1-1 is provided. In the measurement control information it is indicated to the UE that periodic reporting of the UTRA TDD P-CCPCH RSRP measurement is used.

#### A.9.5.1.2	Test parameters

In this set of test cases there are two cells. Cell 1 is a E-UTRA FDD cell and cell 2 is a UTRA TDD cell. The absolute accuracy of P-CCPCH RSCP measurements are tested by using test parameters in Table A.9.5.1-1, Table A.9.5.1-2, and Table A.9.5.1-3. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.9.5.1-1: General test parameters for UTRA TDD P-CCPCH RSCP measurement absolute accuracy in E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN FDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 2 | One UTRAN TDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRAN FDD cell 1 on RF channel number 1 |
| Neighbor cells |  | Cell 2 | 1.28Mcps UTRA TDD cell 2 on RF channel number 2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| CP length of cell 1 |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Inter-RAT (UTRAN TDD) measurement quantity |  | P-CCPCH RSRP |  |

Table A.9.5.1-2: UTRA TDD P-CCPCH RSCP measurement tests parameters (cell 1)

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |
|  | dB | 4 |  |  |
| RSRPNote3 | dBm/15 kHz | -94 |  |  |
| IoNote3 | dBm/9 MHz | -64.76 |  |  |
|  | dB | 4 |  |  |
| Propagation condition | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9.5.1-3: UTRA TDD P-CCPCH RSCP measurement tests parameters (cell 2)

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DL timeslot number |  | 0 |  | DwPTS |  | 0 | DwPTS |
| UTRA RF Channel number Note2 |  | Channel 2 |  | Channel 2 |  | Channel 2 |  |
| PCCPCH_Ec/Ior | dB | -3 |  | -3 |  | -3 |  |
| DwPCH_Ec/Ior | dB |  | 0 |  | 0 |  | 0 |
| OCNS_Ec/Ior | dB | -3 |  | -3 |  | -3 |  |
| Ioc | dBm/1.28MHz | -54.1 |  | -75.2 |  | -97 |  |
| Îor/Ioc | dB | 2 |  | 5 |  | 0 |  |
| PCCPCH RSCP Note1 | dBm | -55.1 |  | -73.2 |  | -100 |  |
| Io Note1 | dBm/1.28MHz | -50 |  | -69 |  | -94 |  |
| Propagation condition |  | AWGN |  |  |  |  |  |
| Note 1: PCCPCH RSCP and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. Note 2: In the case of multi-frequency network of 1.28 Mcps TDD, the UTRA RF Channel Number can be set for the primary frequency in this test. |  |  |  |  |  |  |  |

#### A.9.5.1.3	Test Requirements

The UTRA TDD P-CCPCH RSCP measurement accuracy shall meet the requirements in clause 9.3.1.

### A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD

#### A.9.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UTRAN TDD P-CCPCH RSCP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.3.1 and applies to UE supporting this capability.

Gap pattern configuration with id #1 as specified in Table 8.1.2.1-1 is provided. In the measurement control information it is indicated to the UE that periodic reporting of the UTRA TDD P-CCPCH RSRP measurement is used.

#### A.9.5.2.2	Test parameters

In this set of test cases there are two cells. Cell 1 is a E-UTRA TDD cell and cell 2 is a UTRA TDD cell. The absolute accuracy of P-CCPCH RSCP measurements are tested by using test parameters in Table A.9.5.2-1, Table A.9.5.2-2, and Table A.9.5.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.9.5.2-1: General test parameters for UTRA TDD P-CCPCH RSCP measurement

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRAN RF Channel Number |  | 1 | One E-UTRAN TDD carrier frequency is used. |
| UTRAN RF Channel Number |  | 2 | One UTRAN TDD carrier frequency is used. |
| E-UTRAN Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRA TDD cell1 on RF channel number 1 |
| Neighbour cell |  | Cell 2 | 1.28Mcps UTRA TDD Cell2 on RF channel number 2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| CP length of cell 1 |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells | ms | 3 | Asynchronous cells |
| Inter-RAT (UTRAN TDD) measurement quantity |  | P-CCPCH RSCP |  |

Table A.9.5.2-2: UTRA TDD P-CCPCH RSCP measurement tests parameters (cell 1)

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |
|  | dB | 4 |  |  |
| RSRPNote3 | dBm/15 kHz | -94 |  |  |
| IoNote3 | dBm/9 MHz | -64.76 |  |  |
|  | dB | 4 |  |  |
| Propagation condition | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9.5.2-3: UTRA TDD P-CCPCH RSCP measurement tests parameters (cell 2)

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DL timeslot number |  | 0 |  | DwPTS |  | 0 | DwPTS |
| UTRA RF Channel number Note2 |  | Channel 2 |  | Channel 2 |  | Channel 2 |  |
| PCCPCH_Ec/Ior | dB | -3 |  | -3 |  | -3 |  |
| DwPCH_Ec/Ior | dB |  | 0 |  | 0 |  | 0 |
| OCNS_Ec/Ior | dB | -3 |  | -3 |  | -3 |  |
| Ioc | dBm/1.28MHz | -54.1 |  | -75.2 |  | -97 |  |
| Îor/Ioc | dB | 2 |  | 5 |  | 0 |  |
| PCCPCH RSCP Note1 | dBm | -55.1 |  | -73.2 |  | -100 |  |
| Io Note1 | dBm/1.28MHz | -50 |  | -69 |  | -94 |  |
| Propagation condition |  | AWGN |  |  |  |  |  |
| Note 1: PCCPCH RSCP and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. Note 2: In the case of multi-frequency network of 1.28 Mcps TDD, the UTRA RF Channel Number can be set for the primary frequency in this test. |  |  |  |  |  |  |  |

#### A.9.5.2.3	Test Requirements

The UTRA TDD P-CCPCH RSCP measurement accuracy shall meet the requirements in clause 9.3.1.

## A.9.6	GSM Carrier RSSI

### A.9.6.1	E-UTRAN FDD

#### A.9.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the GSM Carrier RSSI measurement accuracy is within the specified limits when the active cell is E-UTRAN FDD. This test will verify the requirements in clause 9.4.1. There are 12 different test setups with different signal levels for the GSM cells.

Measurement gaps are configured to measure on the GSM cells. Table A.9.6.1.1-2 defines the cell specific test parameters for the E-UTRAN FDD cell. In the measurement control information it is indicated to the UE that periodic reporting of the GSM RSSI measurement is used. The limits of the GSM test parameters in terms of GSM BCCH received level at the receiver inputs are defined in Table A.9.6.1.1-3.

Table A.9.6.1.1-1: General GSM Carrier RSSI test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Active cell | - | Cell 1 |  |
| DRX | - | OFF |  |
| Gap pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Filtering coefficient | - | 0 | L3 filtering is not used. |
| Inter-RAT measurement quantity |  | GSM Carrier RSSI |  |
| Monitored cell list size |  | 6 GSM neighbours including ARFCN 1 | Included in the Measurement control information |

Table A.9.6.1.1.-2: E-UTRAN FDD Cell specific test parameters for GSM Carrier RSSI accuracy test in E-UTRAN FDD

| Parameter | Unit | Tests 1-12 |
| --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note 2 | dBm/15 kHz | -98 |
| RSRP Note 3 | dBm/15 kHz | -94 |
|  | dB | 4 |
| SCH_RP Note 3 | dBm/15 kHz | -94 |
|  | dB | 4 |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.9.6.1.1-3: BCCH signal levels at receiver input in dBm

| Step | BCCH1 | BCCH2 | BCCH3 | BCCH4 | BCCH5 | BCCH6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -38.5 | -38.5 | NA | NA | NA | NA |
| 2 | -48.5 | -48.5 | NA | NA | NA | NA |
| 3 | -70.5 | -70.5 | NA | NA | NA | NA |
| 4 | -109.5 | -109.5 | NA | NA | NA | NA |
| 5 | -57.5 | NA | -54.5 | NA | NA | NA |
| 6 | -64.5 | NA | -59.5 | NA | NA | NA |
| 7 | -71.5 | NA | NA | -64.5 | NA | NA |
| 8 | -78.5 | NA | NA | -69.5 | NA | NA |
| 9 | -85.5 | NA | NA | NA | -74.5 | NA |
| 10 | -92.5 | NA | NA | NA | -79.5 | NA |
| 11 | -99.5 | NA | NA | NA | NA | -84.5 |
| 12 | -106.5 | NA | NA | NA | NA | -89.5 |

#### A.9.6.1.2	Test Requirements

The GSM Carrier RSSI measurement accuracy shall meet the requirements in clause 9.4.1.

### A.9.6.2	E-UTRAN TDD

#### A.9.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the GSM Carrier RSSI measurement accuracy is within the specified limits when the active cell is E-UTRAN TDD. This test will verify the requirements in clause 9.4.1. There are 12 different test setups with different signal levels for the GSM cells.

Measurement gaps are configured to measure on the GSM cells. Table A.9.6.2.1-2 defines the cell specific test parameters for the E-UTRAN TDD cell. In the measurement control information it is indicated to the UE that periodic reporting of the GSM RSSI measurement is used. The limits of the GSM test parameters in terms of GSM BCCH received level at the receiver inputs are defined in Table A.9.6.2.1-3.

Table A.9.6.2.1-1: General GSM Carrier RSSI test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |
| Active cell | - | Cell 1 |  |
| DRX | - | OFF |  |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| Gap pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Filtering coefficient | - | 0 | L3 filtering is not used. |
| Inter-RAT measurement quantity |  | GSM Carrier RSSI |  |
| Monitored cell list size |  | 6 GSM neighbours including ARFCN 1 | Included in the Measurement control information |

Table A.9.6.2.1-2: E-UTRAN TDD Cell specific test parameters for GSM Carrier RSSI accuracy test in E-UTRAN TDD

| Parameter | Unit | Tests 1 - 12 |
| --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note 2 | dBm/15 kHz | -98 |
| RSRP Note 3 | dBm/15 kHz | -94 |
|  | dB | 4 |
| SCH_RP Note 3 | dBm/15 kHz | -94 |
|  | dB | 4 |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.9.6.2.1-3: BCCH signal levels at receiver input in dBm

| Step | BCCH1 | BCCH2 | BCCH3 | BCCH4 | BCCH5 | BCCH6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -38.5 | -38.5 | NA | NA | NA | NA |
| 2 | -48.5 | -48.5 | NA | NA | NA | NA |
| 3 | -70.5 | -70.5 | NA | NA | NA | NA |
| 4 | -109.5 | -109.5 | NA | NA | NA | NA |
| 5 | -57.5 | NA | -54.5 | NA | NA | NA |
| 6 | -64.5 | NA | -59.5 | NA | NA | NA |
| 7 | -71.5 | NA | NA | -64.5 | NA | NA |
| 8 | -78.5 | NA | NA | -69.5 | NA | NA |
| 9 | -85.5 | NA | NA | NA | -74.5 | NA |
| 10 | -92.5 | NA | NA | NA | -79.5 | NA |
| 11 | -99.5 | NA | NA | NA | NA | -84.5 |
| 12 | -106.5 | NA | NA | NA | NA | -89.5 |

#### A.9.6.2.2	Test Requirements

The GSM Carrier RSSI measurement accuracy shall meet the requirements in clause 9.4.1.

## A.9.7	UE Rx – Tx Time Difference

### A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case

#### A.9.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN FDD UE Rx – Tx time difference measurement accuracy is within the specified limits in Clause 9.1.9.

There is only one active cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodically, and signaled to report UE Rx – Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE.

#### A.9.7.1.2	Test parameters

The parameters for this test case are defined in Table A.9.7.1.2-1, and the SRS configuration used is defined in Table A.9.7.1.2-2.

Table A.9.7.1.2-1: FDD UE Rx – Tx time difference test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 1.4 | 10 |
| DRX |  | OFF |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.2 FDD | R.0 FDD |
| PDSCH allocation |  | 2—3 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.8 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.3 (OP.3 FDD) and A.3.2.1.1 (OP.1 FDD) |  | OP.3 FDD | OP.1 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 | -98 |
| RSRP Note3 | dBm/15 kHz | -101 | -101 |
|  | dB | -3 | -3 |
| Io Note3 | dBm/1.08 MHz | -77.66 | N/A |
|  | dBm/9 MHz | N/A | -68.45 |
|  | dB | -3 | -3 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the resources in the active cell are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.9.7.1.2-2: Sounding Reference Symbol Configuration to be used in FDD UE Rx – Tx time difference test

| Field | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- |
|  | Value |  |  |
| srsBandwidthConfiguration | bw7 | bw5 |  |
| srsSubframeConfiguration | sc1 |  |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |  |
| srsMaxUpPTS | N/A |  | Not applicable for FDD |
| srsBandwidth | 0 |  | No hopping |
| srsHoppingBandwidth | hbw0 |  |  |
| frequencyDomainPosition | 0 |  |  |
| Duration | TRUE |  | Indefinite duration |
| Srs-ConfigurationIndex | 0 |  | SRS periodicity of 2ms for all Tests. |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  | No cyclic shift |
| SRS-AntennaPort | an1 |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.9.7.1.3	Test Requirements

The UE Rx – Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.9.1.

### A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case

#### A.9.7.2.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN TDD UE Rx-Tx time difference measurement accuracy is within the specified limits in clause 9.1.9.

There is only one cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodcally, and signaled to report UE Rx – Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx – Tx measurement reported by the UE.

#### A.9.7.2.2	Test parameters

The parameters for this test case are defined in Table A.9.7.2.2-1, and the SRS configuration used is defined in Table A.9.7.2.2-2.

Table A.9.7.2.2-1: Cell specific test parameters for UE Rx-Tx time difference measurement

| Parameter | Unit | Tests 1 | Tests 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number | - | 1 | 1 |
| BWchannel | MHz | 1.4 | 10 |
| Uplink-downlink configuration of cell Note1 |  | 1 | 1 |
| Special subframe configuration of cell Note1 |  | 6 | 6 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 | - | R.2 TDD | R.0 TDD |
| PDSCH allocation |  | 2-3 | 13-36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 | - | R.8 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.3 (OP.3 TDD) and A.3.2.2.1 (OP.1 TDD) | - | OP.3 TDD | OP.1 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote2 | dB |  |  |
| OCNG_RBNote2 | dB |  |  |
| Note 3 | dBm/15 kHz | -98 | -98 |
| RSRP Note 4 | dBm/15 kHz | -101 | -101 |
|  | dB | -3 | -3 |
| Io Note 4 | dBm/1.08 MHz | -77.66 | N/A |
|  | dBm/9 MHz | N/A | -68.45 |
|  | dB | -3 | -3 |
| Propagation Condition |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.9.7.2.2-2: Sounding Reference Symbol Configuration to be used in TDD UE Rx – Tx time difference test

| Field | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- |
|  | Value |  |  |
| srsBandwidthConfiguration | bw7 | bw5 |  |
| srsSubframeConfiguration | sc1 |  |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |  |
| srsMaxUpPTS | TRUE |  |  |
| srsBandwidth | 0 |  | No hopping |
| srsHoppingBandwidth | hbw0 |  |  |
| frequencyDomainPosition | 0 |  |  |
| Duration | TRUE |  | Indefinite duration |
| Srs-ConfigurationIndex | 10 |  | SRS periodicity of 10ms for all Tests. |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  | No cyclic shift |
| SRS-AntennaPort | an1 |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.9.7.2.3	Test Requirements

The UE Rx – Tx time difference measurement accuracy shall fulfill the requirements in clause 9.1.9.1.

### A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.7.3.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN FDD UE Rx–Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.9.3 when time-domain measurement resource restriction is configured for PCell measurements via higher-layer signalling [2] and non-MBSFN ABS are configured in the interfering cell.

#### A.9.7.3.2	Test parameters

In this test case, there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. Cell 1 is the PCell on which UE Rx-Tx is measured, and Cell 2 is the interfering cell. Non-MBSFN ABS pattern is configured in Cell 2 during the entire test.

The tested UE is connected to the PCell and configured to transmit SRS signals periodically. The SRS configuration is provided to the UE before the measurement starts. The UE is configured to report UE Rx–Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on PCell. The information for both patterns shall be provided to the UE before the measurement starts.

The general and cell-specific parameters for this test case are defined in Table A.9.7.3.2-1 and Table A.9.7.3.2-2, respectively, and the SRS configuration used is specified in Table A.9.7.3.2-3.

Table A.9.7.3.2-1: General test parameters for FDD UE Rx–Tx time difference measurement under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | The measured cell |
| Neighbour cell |  | Cell 2 | The cell interfering to Cell 1 |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used |
| Downlink Channel Bandwidth (BWchannel) | MHz | 10 | For both cells in the test |
| CP length |  | Normal | For both cells in the test |
| DRX |  |  | OFF |
| Time offset between cells | s | 3 | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met |
| ABS pattern |  | ‘1000000010000000100000001000000010000000’ | Non-MBSFN ABS. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod 40 = 0. No MBSFN subframes are cofigured in Cell 1 or Cell 2 during the ABS subframes of Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘1000000010000000100000001000000010000000’ | Configured for measurements on Cell 1. |

Table A.9.7.3.2-2: Cell-specific test parameters for FDD UE Rx–Tx time difference measurement under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 | 1 |
| Channel bandwidth (BWchannel) | MHz | 10 | 10 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD | N/A |
| PDSCH allocation |  | 13—36 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD | N/A |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.5 FDD | OP.6 FDD |
| PBCH_RA | dB | 0 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note 2 | dBm/15 kHz | -98 | -98 |
| CRS | dB | -3 | 1 |
| CRS Note 3 | dB | -3 | -0.76 |
| CRS Note 3 | dB | -6.54 | -0.76 |
| RSRP Note 4 | dBm/15 kHz | -101 | -97 |
| Note 4 | dBm/9 MHz | -67.89 | -67.89 |
| Note 4 | dBm/9 MHz | -65.81 | -65.81 |
| Propagation condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that the resources in the active cell are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: is calculated in CRS REs in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS REs in the subframes not indicated for PCell measurements by measurement resource restriction paattern. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. is calculated in CRS symbols in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS symbols in the subframes not indicated for PCell measurements by measurement resource restriction pattern. |  |  |  |

Table A.9.7.3.2-3: Sounding Reference Symbol Configuration to be used in FDD UE Rx–Tx time difference test

| Field | Value | Comment |
| --- | --- | --- |
| UL bandwidth | 50 RBs | Same as the DL bandwidth |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| srs-ConfigIndex | 0 | SRS periodicity of 2ms |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| srsAntennaPort | an1 | Number of SRS antenna ports |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.3.3	Test Requirements

The UE Rx–Tx time difference measurement accuracy shall fulfill the requirements in Section 9.1.9.3.

### A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.9.7.4.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.9.3 when time-domain measurement resource restriction is configured for PCell measurements via higher-layer signalling [2] and non-MBSFN ABS are configured in the interfering cell.

#### A.9.7.4.2	Test Parameters

In the test, there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. Cell 1 is the PCell on which UE Rx-Tx is measured, and Cell 2 is the interfering cell. Non-MBSFN ABS pattern is configured in Cell 2 during the entire test.

The tested UE is connected to the PCell and configured to transmit SRS signals periodically. The SRS configuration is provided to the UE before the measurement starts. The UE is configured to report UE Rx–Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD UE Rx-Tx time difference measurements on PCell. The information for both patterns shall be provided to the UE before the measurement starts.

The general and cell-specific parameters for this test case are defined in Table A.9.7.4.2-1 and Table A.9.7.4.2-2, respectively, and the SRS configuration used is defined in Table A.9.7.4.2-3.

Table A.9.7.4.2-1: General test parameters for E-UTRAN TDD UE Rx-Tx time difference measurement under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell (PCell) |  | Cell 1 | Cell to be measured |
| Neighbour cell |  | Cell 2 | The cell interfering to Cell 1 |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used |
| Downlink Channel Bandwidth (BWchannel) | MHz | 10 | For both cells in the test |
| CP length |  | Normal | For both cells in the test |
| Special subframe configuration |  | 6 | For Cell 1 and Cell 2. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  | 1 | For Cell 1 and Cell 2. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| DRX |  |  | OFF |
| Time offset between cells | s | 3 | Synchronous cells |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs for Cell 1 and Cell 2 are selected randomly so that the condition is met. |
| ABS pattern |  | ‘00000000010000000001’ | Non-MBSFN ABS. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the subframe #0 of a radio frame satisfying SFN mod 20 = 0. No MBSFN subframes are cofigured in the ABS subframes in Cell 2. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  | ‘00000000010000000001’ | Configured for measurements on Cell 1. |

Table A.9.7.4.2-2: Cell-specific test parameters for E-UTRAN TDD UE Rx-Tx time difference measurement under time-domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | R.0 TDD | N/A |
| PDSCH allocation |  | 13—36 | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 | -98 |
| CRS | dB | -3 | 1 |
| CRS Note 3 | dB | -3 | -0.76 |
| CRS Note 3 | dB | -6.54 | -0.76 |
| RSRP Note 4 | dBm/15 kHz | -101 | -97 |
| Note 4 | dBm/9 MHz | -67.89 | -67.89 |
| Note 4 | dBm/9 MHz | -65.81 | -65.81 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the resources in the active cell are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: is calculated in CRS REs in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS REs in the subframes not indicated for PCell measurements by measurement resource restriction paattern. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. is calculated in CRS symbols in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS symbols |  |  |  |

Table A.9.7.4.2-3: Sounding Reference Symbol Configuration to be used in TDD UE Rx–Tx time difference test

| Field | Value | Comment |
| --- | --- | --- |
| UL bandwidth | 50 RBs | Same as the DL bandwidth |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | TRUE |  |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 10 | SRS periodicity of 10ms for all Tests. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.4.3	Test Requirements

The UE Rx–Tx time difference measurement accuracy shall fulfill the requirements in Section 9.1.9.3.

### A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.7.5.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN FDD UE Rx–Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.9.4 when the UE is provided with a time-domain measurement resource restriction pattern and CRS assistance information, and when non-MBSFN ABS configured in the interfering cells.

#### A.9.7.5.2	Test parameters

In this test case, there are three synchronous cells, Cell 1, Cell 2 and Cell 3, on the same RF channel. Cell 1 is the PCell on which UE Rx-Tx is measured. Cell 2 and Cell 3 are the interfering cells. A non-MBSFN ABS pattern is configured in each of the Cell 2 and Cell 3 during the entire test.

The tested UE is connected to the PCell and configured to transmit SRS signals periodically. The SRS configuration is provided to the UE before the measurement starts. The UE is configured to report UE Rx–Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on PCell. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement patterns and the CRS assistance information shall be provided via RRC to the UE before the measurement starts.

The general and cell-specific parameters for this test case are defined in Table A.9.7.5.2-1 and Table A.9.7.5.2-2, respectively, and the SRS configuration used is specified in Table A.9.7.5.2-3.

Table A.9.7.5.2-1: General test parameters for FDD UE Rx–Tx time difference measurement under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell (PCell) |  |  | Cell 1 | The measured cell |
| Neighbour cell |  |  | Cell 2 and Cell 3 | Cell 2 is the first interfering cell to Cell 1, whilst Cell 3 is the second interfering cell to Cell 1. |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | One FDD carrier frequency is used |
| Downlink Channel Bandwidth (BWchannel) |  | MHz | 10 | For all cells in the test |
| CP length |  |  | Normal | For all cells in the test |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 3 Cell 3 offset with respect to Cell 1: 2 | Three synchronous cells |
| Physical cell ID PCI |  |  | (PCIcell1 - PCIcell2 )mod6 =0 (PCIcell1 - PCIcell3 )mod6 !=0 | Cell PCIs are selected so that both conditions are met |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | Non-MBSFN ABS. FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in Cell 2 and Cell 3 during the testing. |
| Time-domain measurement resource restriction pattern for PCell measurements |  |  | ‘1000000010000000100000001000000010000000’ | Configured for measurements on Cell 1. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.7.5.2-2: Cell-specific test parameters for FDD UE Rx–Tx time difference measurement under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 | 1 | 1 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD | N/A | N/A |
| PDSCH allocation |  | 13—36 | N/A | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD | N/A | N/A |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.5 FDD | OP.6 FDD | OP.6 FDD |
| PBCH_RA | dB | 0 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
| Note 2 | dBm/15 kHz | -98 | -98 | -98 |
| CRS | dB | -3 | 3 | 1 |
| CRS Note 3 | dB | -7.76 | 1.24 | -0.76 |
| CRS Note 3 | dB | -9.29 | -1.41 | -4.44 |
| RSRP Note 4 | dBm/15 kHz | -101 | -95 | -97 |
| Note 4 | dBm/9 MHz | -67.11 | -67.11 | -67.11 |
| Note 4 | dBm/9 MHz | -63.45 | -63.45 | -63.45 |
| Propagation condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that the resources in the active cell are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 3: is calculated in CRS REs in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS REs in the subframes not indicated for PCell measurements by measurement resource restriction paattern. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. is calculated in CRS symbols in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS symbols in the subframes not indicated for PCell measurements by measurement resource restriction pattern. |  |  |  |  |

Table A.9.7.5.2-3: Sounding Reference Symbol Configuration to be used in FDD UE Rx–Tx time difference test

| Field | Value | Comment |
| --- | --- | --- |
| UL bandwidth | 50 RBs | Same as the DL bandwidth |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| srs-ConfigIndex | 0 | SRS periodicity of 2ms |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| srsAntennaPort | an1 | Number of SRS antenna ports |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.5.3	Test Requirements

The UE Rx–Tx time difference measurement accuracy shall fulfill the requirements in Section 9.1.9.4.

### A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.9.7.6.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN TDD UE Rx–Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.9.4 when the UE is provided with a time-domain measurement resource restriction pattern and CRS assistance information, and when non-MBSFN ABS configured in the interfering cells.

#### A.9.7.6.2	Test Parameters

In this test case, there are three synchronous cells, Cell 1, Cell 2 and Cell 3, on the same RF channel. Cell 1 is the PCell on which UE Rx-Tx is measured. Cell 2 and Cell 3 are the interfering cells. A non-MBSFN ABS pattern is configured in each of the Cell 2 and Cell 3 during the entire test.

The tested UE is connected to the PCell and configured to transmit SRS signals periodically. The SRS configuration is provided to the UE before the measurement starts. The UE is configured to report UE Rx–Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE. The UE is configured by higher layers via Cell 1 with a time-domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on PCell. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement patterns and the CRS assistance information shall be provided via RRC to the UE before the measurement starts.

The general and cell-specific parameters for this test case are defined in Table A.9.7.6.2-1 and Table A.9.7.6.2-2, respectively, and the SRS configuration used is specified in Table A.9.7.6.2-3.

Table A.9.7.6.2-1: General test parameters for E-UTRAN TDD UE Rx-Tx time difference measurement under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell (PCell) |  |  | Cell 1 | Cell to be measured |
| Neighbour cell |  |  | Cell 2 and Cell 3 | Cell 2 is the first interfering cell to Cell 1, whilst Cell 3 is the second interfering cell to Cell 1. |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | One TDD carrier frequency is used |
| Downlink Channel Bandwidth (BWchannel) |  | MHz | 10 | For all cells in the test |
| CP length |  |  | Normal | For all cells in the test |
| Special subframe configuration |  |  | 6 | For all cells in the test. For special subframe configurations see Table 4.2-1 in [16]. |
| Uplink/downlink subframe configuration |  |  | 1 | For all cells in the test. For uplink-downlink subframe configurations see Table 4.2-2 in [16]. |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 3 Cell 3 offset with respect to Cell 1: 2 | Three synchronous cells |
| Physical cell ID PCI |  |  | (PCIcell1 - PCIcell2 )mod6 =0 (PCIcell1 - PCIcell3 )mod6 !=0 | Cell PCIs are selected so that both conditions are met |
| ABS pattern |  |  | ‘00000000010000000001’ | Non-MBSFN ABS. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of a radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in Cell 2 and Cell 3 during the testing. |
| Time-domain measurement resource restriction pattern for serving cell measurements |  |  | ‘00000000010000000001’ | Configured for measurements on Cell 1. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.9.7.6.2-2: Cell-specific test parameters for E-UTRAN TDD UE Rx-Tx time difference measurement under time-domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 | 1 | 1 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | R.0 TDD | N/A | N/A |
| PDSCH allocation |  | 13—36 | N/A | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.6 TDD | N/A | N/A |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
| Note2 | dBm/15 kHz | -98 | -98 | -98 |
| CRS | dB | -3 | 3 | 1 |
| CRS Note 3 | dB | -7.76 | 1.24 | -0.76 |
| CRS Note 3 | dB | -9.29 | -1.41 | -4.44 |
| RSRP Note 4 | dBm/15 kHz | -101 | -95 | -97 |
| Note 4 | dBm/9 MHz | -67.11 | -67.11 | -67.11 |
| Note 4 | dBm/9 MHz | -63.45 | -63.45 | -63.45 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in the active cell are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 3: is calculated in CRS REs in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS REs in the subframes not indicated for PCell measurements by measurement resource restriction paattern. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. is calculated in CRS symbols in the subframes indicated for PCell measurements by measurement resource restriction pattern, whilst is calculated in CRS symbols |  |  |  |  |

Table A.9.7.6.2-3: Sounding Reference Symbol Configuration to be used in TDD UE Rx–Tx time difference test

| Field | Value | Comment |
| --- | --- | --- |
| UL bandwidth | 50 RBs | Same as the DL bandwidth |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | TRUE |  |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 10 | SRS periodicity of 10ms for all Tests. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.6.3	Test Requirements

The UE Rx–Tx time difference measurement accuracy shall fulfill the requirements in Section 9.1.9.4.

### A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA

#### A.9.7.7.1	Test Purpose and Environment

The purpose of this test is to verify that Cat-M1 and Cat-M2 UE can meet the E-UTRAN FDD UE Rx-Tx time difference measurement accuracy requirements. Requirements for Cat-M1 UE is specified in Clause 9.1.21.21 and requirements for Cat-M2 UE is specified in Clause 9.1.25.25.

There is only one active cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodically, and signaled to report UE Rx-Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE.

#### A.9.7.7.2	Test parameters

The parameters for this test case are defined in Table A.9.7.7.2-1, and the SRS configuration used is defined in Table A.9.7.7.2-2.

Table A.9.7.7.2-1: FDD UE Rx-Tx time difference test parameters for Cat-M1/M2 UE in CEModeA

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |
| DRX |  | OFF |
| PRACH Configuration |  | PRACH_4CE As specified in A.3.16 |
| MPDCCH Reference measurement channelNote1 |  | R.16 FDD |
| OCNG PatternNote2 |  | OP.21 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| MPDCCH_RA | dB |  |
| MPDCCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
|  | dBm/15 kHz | -98 |
|  | dB | 3 |
|  | dB | 3 |
| IoNote4 | dBm/9 MHz | -65.5 |
| Propagation Condition |  | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |

Table A.9.7.7.2-2: Sounding Reference Symbol Configuration to be used in FDD UE Rx-Tx time difference test for Cat-M1/M2 UE in CEModeA

| Field | Test 1 | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | SRS periodicity of 2ms for all Tests. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.7.3	Test Requirements

For Cat-M1 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.21.21.

For Cat-M2 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.25.25.

### A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA

#### A.9.7.8.1	Test Purpose and Environment

The purpose of this test is to verify that Cat-M1 and Cat-M2 UE can meet the E-UTRAN HD-FDD UE Rx-Tx time difference measurement accuracy requirements. Requirements for Cat-M1 UE is specified in Clause 9.1.21.21 and requirements for Cat-M2 UE is specified in Clause 9.1.25.25.

There is only one active cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodically, and signaled to report UE Rx-Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE.

#### A.9.7.8.2	Test parameters

The parameters for this test case are defined in Table A.9.7.8.2-1, and the SRS configuration used is defined in Table A.9.7.8.2-2.

Table A.9.7.8.2-1: HD-FDD UE Rx-Tx time difference test parameters for Cat-M1/M2 UE in CEModeA

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |
| DRX |  | OFF |
| PRACH Configuration |  | PRACH_4CE As specified in A.3.16 |
| MPDCCH Reference measurement channelNote1 |  | R.6 HD FDD |
| OCNG PatternNote2 |  | OP.21 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| MPDCCH_RA | dB |  |
| MPDCCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
|  | dBm/15 kHz | -98 |
|  | dB | 3 |
|  | dB | 3 |
| IoNote4 | dBm/9 MHz | -65.5 |
| Propagation Condition |  | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |

Table A.9.7.8.2-2: Sounding Reference Symbol Configuration to be used in HD-FDD UE Rx-Tx time difference test for Cat-M1/M2 UE in CEModeA

| Field | Test 1 | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | SRS periodicity of 2ms for all Tests. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.8.3	Test Requirements

For Cat-M1 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.21.21.

For Cat-M2 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.25.25.

### A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA

#### A.9.7.9.1	Test Purpose and Environment

The purpose of this test is to verify that Cat-M1 and Cat-M2 UE can meet the E-UTRAN TDD UE Rx-Tx time difference measurement accuracy requirements. Requirements for Cat-M1 UE is specified in Clause 9.1.21.21 and requirements for Cat-M2 UE is specified in Clause 9.1.25.25.

There is only one active cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodically, and signaled to report UE Rx-Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE.

#### A.9.7.9.2	Test parameters

The parameters for this test case are defined in Table A.9.7.9.2-1, and the SRS configuration used is defined in Table A.9.7.9.2-2.

Table A.9.7.9.2-1: TDD UE Rx-Tx time difference test parameters for Cat-M1/M2 UE in CEModeA

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRAN RF Channel Number |  | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |
| Special subframe configurationNote1 |  | 6 |
| Uplink-downlink configurationNote2 |  | 1 |
| DRX |  | OFF |
| PRACH Configuration |  | PRACH_4CE As specified in A.3.16 |
| MPDCCH Reference measurement channelNote3 |  | R.14 TDD |
| OCNG PatternNote4 |  | OP.11 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| MPDCCH_RA | dB |  |
| MPDCCH_RB | dB |  |
| OCNG_RANote5 | dB |  |
| OCNG_RBNote5 | dB |  |
|  | dBm/15 kHz | -98 |
|  | dB | 3 |
|  | dB | 3 |
| IoNote6 | dBm/9 MHz | -65.5 |
| Propagation Condition |  | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |

Table A.9.7.9.2-2: Sounding Reference Symbol Configuration to be used in TDD UE Rx-Tx time difference test for Cat-M1/M2 UE in CEModeA

| Field | Test 1 | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | FALSE |  |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | SRS periodicity of 10ms |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.9.7.9.3	Test Requirements

For Cat-M1 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.21.21.

For Cat-M2 UE, the UE Rx-Tx time difference measurement accuracy shall fulfill the requirements in Clause 9.1.25.25.

## A.9.8	RSTD

### A.9.8.1	E-UTRAN FDD RSTD intra frequency case

#### A.9.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSTD intra-frequency measurement accuracy is within the specified limits in clause 9.1.10.1 in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.1.1-1 and A.9.8.1.1-2 during this time.

The test parameters are given in Table A.9.8.1.1-1 and Table A.9.8.1.1-2.

Table A.9.8.1.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| PCFICH/PDCCH/PHICH parameters |  | R.14 FDD |  | R.6 FDD |  | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1 |  | OP.7 FDD |  | OP.6 FDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 1.4 |  | 10 |  |  |
| PRS Bandwidth | RB | 6 |  | 50 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 12 |  | 2 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 1 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | us | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | us | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | us | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 2560 |  |  |  | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.1 |

Table A.9.8.1.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -2.37 | -8.02 | -6 | -13 | -2.37 | -8.02 | -6 | -13 |
| PRS Note3 | dB | -3 | -10 | -6 | -13 | -3 | -10 | -6 | -13 |
| Io Note3 | dBm/1.08 MHz | -78.92 | -78.92 | -79.21 | -79.21 | N/A | N/A | N/A | N/A |
|  | dBm/9 MHz | N/A | N/A | N/A | N/A | -69.72 | -69.72 | -70 | -70 |
| PRP Note3 | dBm/15kHz | -100.37 | -106.02 | -104 | -111 | -100.37 | -106.02 | -104 | -111 |
| Note 3 | dB | -2.37 | -8.02 | -3 | -13 | -2.37 | -8.02 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -100.37 | -106.02 | -101 | -111 | -100.37 | -106.02 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.1.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.1.

#### A.9.8.1.2A	Test Requirements for UE Category 1bis

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.5. The test parameters given in Table A.9.8.1.1-1 and Table A.9.8.1.1-2 shall be applied with the exceptions given in Table A.9.8.1.2A-1.

Table A.9.8.1.2A-1: Specific test parameters for UE Category for 1Bis intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
|  | s | 5.12 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.3 |

### A.9.8.2	E-UTRAN TDD RSTD intra frequency case

#### A.9.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSTD intra-frequency measurement accuracy is within the specified limits in clause 9.1.10.1 in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.2.1-1 and A.9.8.2.1-2 during this time.

The test parameters are given in Table A.9.8.2.1-1 and Table A.9.8.2.1-2.

Table A.9.8.2.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| PCFICH/PDCCH/PHICH parameters |  | R.14 TDD |  | R.6 TDD |  | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2 |  | OP.4 TDD |  | OP.2 TDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 1.4 |  | 10 |  |  |
| PRS Bandwidth | RB | 6 |  | 50 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Special subframe configuration |  | 6 |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 3 |  | 1 |  | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. The same configuration in both cells. |
| PRS configuration Index |  | 9 |  | 14 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 1 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | us | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | us | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | us | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 2560 |  |  |  | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.2 |

Table A.9.8.2.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -2.37 | -8.02 | -6 | -13 | -2.37 | -8.02 | -6 | -13 |
| PRS Note3 | dB | -3 | -10 | -6 | -13 | -3 | -10 | -6 | -13 |
| Io Note3 | dBm/1.08 MHz | -78.92 | -78.92 | -79.21 | -79.21 | N/A | N/A | N/A | N/A |
|  | dBm/9 MHz | N/A | N/A | N/A | N/A | -69.72 | -69.72 | -70 | -70 |
| PRP Note3 | dBm/15kHz | -100.37 | -106.02 | -104 | -111 | -100.37 | -106.02 | -104 | -111 |
| Note 3 | dB | -2.37 | -8.02 | -3 | -13 | -2.37 | -8.02 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -100.37 | -106.02 | -101 | -111 | -100.37 | -106.02 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |  |  |  |  |  |

#### A.9.8.2.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.1.

#### A.9.8.2.2A	Test Requirements for UE Category 1bis

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.5. The test parameters given in Table A.9.8.2.1-1 and Table A.9.8.2.1-2 shall be applied with the exceptions given in Table A.9.8.2.2A-1.

Table A.9.8.2.2A-1: Specific test parameters for UE Category for 1Bis intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
|  | s | 5.12 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.4 |

### A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case

#### A.9.8.3.1	Test Purpose and Environment

The purpose of these tests is to verify that the RSTD inter-frequency measurement accuracy is within the specified limits in clause 9.1.10.2 in AWGN channels.

There are two synchronous cells on different carrier frequencies in the test. In all test cases, Cell 1 is the reference cell as well as the PCell and Cell 2 the neighbor cell. The inter frequency measurements on Cell 2 are supported by measurement gaps. PCIs of the two cells are selected randomly.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.3.1-1 and Table A.9.8.3.1-2 for each of the two cells during this time.

The test parameters are given in Table A.9.8.3.1-1 and Table A.9.8.3.1-2.

Table A.9.8.3.1-1: General Test Parameters for inter frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| PCFICH/PDCCH/PHICH parameters |  | R.14 FDD | R.6 FDD | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1 |  | OP.7 FDD | OP.6 FDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  | Cell 1 on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 on RF channel number 2 |
| E-UTRA RF Channel Number |  | 1,2 |  | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 1.4 | 10 |  |
| PRS Bandwidth | RB | 6 | 50 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 6 | 1 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| expectedRSTD | s | Cell 2:1 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame transmit time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | Synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
|  | ms | 5120 |  | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.6.1 |

Table A.9.8.3.1-2: Cell Specific Test Parameters for inter frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| GapOffset |  | 18 | N/A | 11 | N/A |
| Gap Pattern ID |  | 0 | N/A | 0 | N/A |
| PRS configuration Index |  | 12 | 19 | 2 | 12 |
| PRS subframe offset |  | N/A | 7 | N/A | 10 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | -3 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |  |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRS Note3 | dB | -6 | -13 | -6 | -13 |
| Io Note3 | dBm/1.08 MHz | -79.25 | -79.39 | N/A | N/A |
|  | dBm/9 MHz | N/A | N/A | -70.04 | -70.18 |
| PRP Note3 | dBm/15kHz | -104 | -111 | -104 | -111 |
| Note 3 | dB | -3 | -13 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -101 | -111 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |  |

#### A.9.8.3.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.2.

#### A.9.8.3.2A	Test Requirements for UE Category 1bis

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.6. The test parameters given in Table A.9.8.3.1-1 and Table A.9.8.3.1-2 shall be applied with the exceptions given in Table A.9.8.3.2A-1.

Table A.9.8.3.2A-1: Specific test parameters for UE Category for 1Bis inter frequency RSTD Tests for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
|  | s | 10.24 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.5 |

### A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case

#### A.9.8.4.1	Test Purpose and Environment

The purpose of this test is to verify that the RSTD inter-frequency measurement accuracy is within the specified limits in clause 9.1.10.2 in AWGN channels.

There are two synchronous cells on different carrier frequencies in the test. In all test cases, Cell 1 is the reference cell as well as the PCell and Cell 2 is the neighbour cell. The inter frequency measurements on Cell 2 are supported by a measurement gap. PCIs of the two cells are selected randomly.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.4.1-1 and Table A.9.8.4.1-2 for each of the two cells during this time.

The test parameters are given in Table A.9.8.4.1-1 and Table A.9.8.4.1-2.

Table A.9.8.4.1-1: General Test Parameters for inter frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| PCFICH/PDCCH/PHICH parameters |  | R.14 TDD | R.6 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2 |  | OP.4 TDD | OP.2 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  | Cell 1 on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 on RF channel number 2 |
| E-UTRA RF Channel Number |  | 1,2 |  | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 1.4 | 10 |  |
| PRS Bandwidth | RB | 6 | 50 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 3 | 1 | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. |
| Number of consecutive positioning downlink subframes |  | 6 | 1 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ |  | PRS muting is not used. See clause 6.5.1.2 in TS 36.355 for more information |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
|  | ms | 5120 |  | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.6.3 |

Table A.9.8.4.1-2: Cell Specific Test Parameters for inter frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap pattern ID |  | 0 | N/A | 0 | N/A |
| Gapoffset |  | 34 | N/A | 13 | N/A |
| PRS configuration Index |  | 15 | 35 | 4 | 14 |
| PRS subframe offset |  | N/A | 20 | N/A | 10 |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | -3 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |  |
| PRS | dB | -6 | -13 | -6 | -13 |
| PRS Note3 | dB | -6 | -13 | -6 | -13 |
| Io Note3 | dBm/1.08 MHz | -79.25 | -79.39 | N/A | N/A |
|  | dBm/9 MHz | N/A | N/A | -70.04 | -70.18 |
| PRP Note3 | dBm/15kHz | -104 | -111 | -104 | -111 |
| Note 3 | dB | -3 | -13 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -101 | -111 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |  |

#### A.9.8.4.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.2.

#### A.9.8.4.2A	Test Requirements for UE Category 1bis

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.10.6. The test parameters given in Table A.9.8.4.1-1 and Table A.9.8.4.1-2 shall be applied with the exceptions given in Table A.9.8.4.2A-1.

Table A.9.8.4.2A-1: Specific test parameters for UE Category for 1Bis inter frequency RSTD Tests for E-UTRAN TDD-TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
|  | s | 10.24 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.7 |

### A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation

#### A.9.8.5.1	Test Purpose and Environment

The purpose of these tests is to verify that the E-UTRAN FDD RSTD measurement accuracy in carrier aggregation is within the specified limits in clause 9.1.12.

There are three synchronous cells on two different carrier frequencies in the test. Cell 1 is the PCell on primary component carrier F1 (RF channel number 1), Cell 2 is the SCell and reference cell on sceondary component carrier F2 (RF channel number 2), and Cell 3 is the neighbor cell on F2.

Cell2 and Cell3 are included in the OTDOA assistance data, whilst Cell1 is not included in the OTDOA assistance data. The RSTD measurements are performed between Cell 2 and Cell 3 to verify that when both the reference cell and neighbouring cell belong to the secondary component carrier the RSTD measurement accuracy can meet the intra-frequency RSTD accuracy requirements defined in clause 9.1.10.1.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap is not configured in the test because of UE carrier aggregation capability.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.5.1-1 and Table A.9.8.5.1-2 for each of the three cells during this time.

The test parameters are given in Table A.9.8.5.1-1 and Table A.9.8.5.1-2.

Table A.9.8.5.1-1: General Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.6 FDD | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1 |  | OP.6 FDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Assistance data reference cell |  | Cell 2 | Cell 2 is the SCell on RF channel number 2 |
| PCell |  | Cell 1 | Cell 1 on RF channel number 1 |
| Neighbour cell |  | Cell 3 | Cell 3 on RF channel number 2 |
| E-UTRA RF Channel Number |  | 1,2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Bandwidth | RB | 50 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 1 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ Cell3:’11110000’ | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 2 – Cell ID of cell 3) mod 6 = 3 | PCI of cell 1 is selected randomly. |
| expectedRSTD | s | Cell 3:-2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 2: -1 Cell 3 to Cell 2:1 | PRS are transmitted from synchronous cells |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the assistance-data-reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. All cells provided in OTDOA assistance data are on RF channel 2. |
|  | ms | 2560 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.1 |

Table A.9.8.5.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| PRS configuration Index |  | 2 | 2 | 2 |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| PRS_RA | dB | -3 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |
| PRS | dB | -6 | -6 | -13 |
| PRS | dB | -6 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -70.04 | -70.01 | -70.01 |
| PRP Note3 | dBm/15kHz | -104 | -104 | -111 |
| RSRP Note3 | dBm/15kHz | -101 | -104 | -111 |
| Note3 | dB | -3 | -6 | -13 |
| Propagation condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , RSRP, Io and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |

#### A.9.8.5.2	Test Requirements

The measurement accuracy of RSTD between Cell2 and Cell3 shall fulfill the requirements in clause 9.1.12.

### A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation

#### A.9.8.6.1	Test Purpose and Environment

The purpose of these tests is to verify that the E-UTRAN TDD RSTD measurement accuracy in carrier aggregation is within the specified limits in clause 9.1.12.

There are three synchronous cells on two different carrier frequencies in the test. Cell 1 is the PCell on primary component carrier F1 (RF channel number 1), Cell 2 is the SCell and reference cell on sceondary component carrier F2 (RF channel number 2), and Cell 3 is the neighbor cell on F2.

Cell2 and Cell3 are included in the OTDOA assistance data, whilst Cell1 is not included in the OTDOA assistance data. The RSTD measurements are performed between Cell 2 and Cell 3 to verify that when both the reference cell and neighbouring cell belong to the secondary component carrier the RSTD measurement accuracy can meet the intra-frequency RSTD accuracy requirements defined in clause 9.1.10.1.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap is not configured in the test because of UE carrier aggregation capability.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.6.1-1 and Table A.9.8.6.1-2 for each of the three cells during this time.

The test parameters are given in Table A.9.8.6.1-1 and Table A.9.8.6.1-2.

Table A.9.8.6.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.6 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2 |  | OP.2 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Assistance data reference cell |  | Cell 2 | Cell 2 is the SCell on RF channel number 2 |
| PCell |  | Cell 1 | Cell 1 on RF channel number 1 |
| Neighbour cell |  | Cell 3 | Cell 3 on RF channel number 2 |
| E-UTRA RF Channel Number |  | 1,2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Bandwidth | RB | 50 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 1 | As defined in TS 36.211 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. The same configuration in both cells. |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ Cell3:’11110000’ | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 2 – Cell ID of cell 3) mod 6 = 3 | PCI of cell 1 is selected randomly. |
| expectedRSTD | s | Cell 3:-2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 2: -1 Cell 3 to Cell 2:1 | PRS are transmitted from synchronous cells |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the assistance-data-reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. All cells provided in OTDOA assistance data are on RF channel 2. |
|  | ms | 2560 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.5.2 |

Table A.9.8.6.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| PRS configuration Index |  | 14 | 14 | 14 |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| PRS_RA | dB | -3 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |
| PRS | dB | -6 | -6 | -13 |
| PRS | dB | -6 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -70.04 | -70.01 | -70.01 |
| PRP Note3 | dBm/15kHz | -104 | -104 | -111 |
| RSRP Note3 | dBm/15kHz | -101 | -104 | -111 |
| Note3 | dB | -3 | -6 | -13 |
| Propagation condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , RSRP, Io and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |

#### A.9.8.6.2	Test Requirements

The measurement accuracy of RSTD between Cell2 and Cell3 shall fulfill the requirements in clause 9.1.12.

### A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth

#### A.9.8.7.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.5.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.7.1-1 and A.9.8.7.1-2 will replace the values of corresponding parameters in Tables A.9.8.5.1-1 and A.9.8.5.1-2.

Table A.9.8.7.1-1: General Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.10 FDD | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1.14 |  | OP.14 FDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | 20 |  |
| PRS Bandwidth | RB | 100 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Note 1: See Table A.9.8.5.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.7.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 20MHz bandwidth

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/18 MHz | -67.03 | -67.00 | -67.00 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.5.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.7.2	Test Requirements

The test requirements defined in section A.9.8.5.2 shall apply to this test case.

### A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth

#### A.9.8.8.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.6.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.8.1-1 and A.9.8.8.1-2 will replace the values of corresponding parameters in Tables A.9.8.6.1-1 and A.9.8.6.1-2.

Table A.9.8.8.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.10 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2.8 |  | OP.8 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | 20 |  |
| PRS Bandwidth | RB | 100 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Note 1: See Table A.9.8.6.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.8.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 20MHz bandwidth

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/18 MHz | -67.03 | -67.00 | -67.00 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.6.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.8.2	Test Requirements

The test requirements defined in section A.9.8.6.2 shall apply to this test case.

### A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz

#### A.9.8.9.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.5.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.9.1-1 and A.9.8.9.1-2 will replace the values of corresponding parameters in Tables A.9.8.5.1-1 and A.9.8.5.1-2.

Table A.9.8.9.1-1: General Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 10MHz+5MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | Cell1: R.6 FDD Cell2: R.11 FDD Cell3: R.11 FDD | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1 |  | Cell1: OP.6 FDD Cell2: OP.19 FDD Cell3: OP.19 FDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | Cell1: 10 Cell2: 5 Cell3: 5 |  |
| PRS Bandwidth | RB | Cell1: 50 Cell2: 25 Cell3: 25 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 2 | As defined in TS 36.211 |
| Note 1: See Table A.9.8.5.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.9.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 10MHz+5MHz

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/9 MHz | -70.04 | N/A | N/A |
|  | dBm/4.5 MHz | N/A | -73.02 | -73.02 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.5.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.9.2	Test Requirements

The test requirements defined in section A.9.8.5.2 shall apply to this test case.

### A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz

#### A.9.8.10.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.6.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.10.1-1 and A.9.8.10.1-2 will replace the values of corresponding parameters in Tables A.9.8.6.1-1 and A.9.8.6.1-2.

Table A.9.8.10.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 10MHz+5MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | Cell1: R.6 TDD Cell2: R.11 TDD Cell3: R.11 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2 |  | Cell1: OP.2 TDD Cell2: OP.10 TDD Cell3: OP.10 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | Cell1: 10 Cell2: 5 Cell3: 5 |  |
| PRS Bandwidth | RB | Cell1: 50 Cell2: 25 Cell3: 25 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 2 | As defined in TS 36.211 |
| Note 1: See Table A.9.8.6.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.10.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 10MHz+5MHz

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/9 MHz | -70.04 | N/A | N/A |
|  | dBm/4.5 MHz | N/A | -73.02 | -73.02 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.6.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.10.2	Test Requirements

The test requirements defined in section A.9.8.6.2 shall apply to this test case.

### A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth

#### A.9.8.11.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.5.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.11.1-1 and A.9.8.11.1-2 will replace the values of corresponding parameters in Tables A.9.8.5.1-1 and A.9.8.5.1-2.

Table A.9.8.11.1-1: General Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 5+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.11 FDD | As specified in clause A.3.1.2.1 |
| OCNG Patterns defined in A.3.2.1.19 |  | OP.19 FDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| PRS Bandwidth | RB | 25 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 2 | As defined in TS 36.211 |
| Note 1: See Table A.9.8.5.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.11.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation for 5+5MHz bandwidth

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/4.5 MHz | -73.05 | -73.02 | -73.02 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.5.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.11.2	Test Requirements

The test requirements defined in section A.9.8.5.2 shall apply to this test case.

### A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth

#### A.9.8.12.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.6.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.12.1-1 and A.9.8.12.1-2 will replace the values of corresponding parameters in Tables A.9.8.6.1-1 and A.9.8.6.1-2.

Table A.9.8.12.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 5+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | R.11 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2.10 |  | OP.10 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| PRS Bandwidth | RB | 25 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 2 | As defined in TS 36.211 |
| Note 1: See Table A.9.8.6.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.12.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 5+5MHz bandwidth

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/4.5 MHz | -73.05 | -73.02 | -73.02 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.6.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.12.2	Test Requirements

The test requirements defined in section A.9.8.6.2 shall apply to this test case.

### A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz

#### A.9.8.13.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.9.8.6.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.9.8.13.1-1 and A.9.8.13.1-2 will replace the values of corresponding parameters in Tables A.9.8.6.1-1 and A.9.8.6.1-2.

Table A.9.8.13.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 20MHz+10MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  | Cell1: R.10 TDD Cell2: R.6 TDD Cell3: R.6 TDD | As specified in clause A.3.1.2.2 |
| OCNG Patterns defined in A.3.2.2 |  | Cell1: OP.8 TDD Cell2: OP.2 TDD Cell3: OP.2 TDD | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). There is no PDSCH allocated in the subframe transmitting PRS. |
| Channel Bandwidth (BWchannel) | MHz | Cell1: 20 Cell2: 10 Cell3: 10 |  |
| PRS Bandwidth | RB | Cell1: 100 Cell2: 50 Cell3: 50 | PRS Bandwidth bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Note 1: See Table A.9.8.6.1-1 for other general test parameters. Note 2: N/A |  |  |  |

Table A.9.8.13.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation for 20MHz+10MHz

| Parameter | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Io Note1 | dBm/ 18MHz | -67.03 | N/A | N/A |
|  | dBm/ 9MHz | N/A | -70.01 | -70.01 |
| Note 1: Io level has been derived from other parameters for information purposes. It is not settable parameter itself. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS Note 2: See Table A.9.8.6.1-2 for other cell specific test parameters. |  |  |  |  |

#### A.9.8.13.2	Test Requirements

The test requirements defined in section A.9.8.6.2 shall apply to this test case.

### A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation

#### A.9.8.14.1	Test Purpose and Environment

The purpose of these tests is to verify that the E-UTRAN FDD RSTD measurement accuracy in carrier aggregation is within the specified limits in clause 9.1.12.

There are four synchronous cells on three different carrier frequencies in the test. Cell 1 is the PCell on primary component carrier F1 (RF channel number 1), Cell 2 is an SCell on secondary component carrier F2 (RF channel number 2), Cell 3 is an SCell and reference cell on secondary component carrier F3 (RF channel number 3), and Cell 4 is the neighbor cell on F3.

Cell 1, Cell2, Cell3, and Cell 4 are included in the OTDOA assistance data. The RSTD measurements are performed

-	between Cell 4 and Cell 3 to verify the accuracy of RSTD measurement when the reference cell and neighbouring cell belong to the same secondary component carrier can meet the intra-frequency RSTD accuracy requirements defined in clause 9.1.10.1.

-	between Cell 1 and Cell 3 to verify the accuracy of RSTD measurement between the PCell and an SCell can meet the inter-frequency RSTD accuracy requirements defined in clause 9.1.10.2.

-	between Cell 2 and Cell 3 to verify the accuracy of RSTD measurement between two SCells can meet the inter-frequency RSTD accuracy requirements defined in clause 9.1.10.2.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap is not configured in the test because of UE carrier aggregation capability.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.14.1-1 and Table A.9.8.14.1-2 for each of the three cells during this time.

The test parameters are given in Table A.9.8.14.1-1 and Table A.9.8.14.1-2.

Table A.9.8.14.1-1: General Test Parameters for RSTD Tests for E-UTRAN FDD for 3DL Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCell |  | Cell 1 | Cell 1 on RF channel number 1 |
| SCell 1 |  | Cell 2 | Cell 2 is an SCell on RF channel number 2 |
| SCell 2 (Assistance data reference cell) |  | Cell 3 | Cell 3 is an SCell on RF channel number 3 |
| Neighbour cell |  | Cell 4 | Cell 4 on RF channel number 3 |
| PRS configuration index |  | 171 for all cells on PCC 181 for all cells on SCC1 191 for all cells on SCC2 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ Cell3:’11110000’ Cell4:’11110000’ | See clause 6.5.1.2 in TS 36.355 for more information |
| prs-SubframeOffset |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| Cell ID |  | (Cell ID of cell 4 – Cell ID of cell 3) mod 6 = 3 | PCIs of cell 1 and cell 2 are selected randomly. |
| expectedRSTD | s | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 | PRS are transmitted from synchronous cells |
| Time alignment errors between Cell 1, Cell 2, and Cell 3 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the assistance-data-reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |
|  | ms | 4960 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.6.1 |

Table A.9.8.14.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN FDD for Carrier Aggregation

| Parameter | Unit | Cell1 | Cell2 | Cell3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 3 | 3 |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | 5,10,20 | 5,10,20 | 5,10,20 |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | -3 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |  |
| PRS | dB | -6 | -6 | -6 | -13 |
| PRS | dB | -6 | -6 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -70.04 +10log (NRB,c /50) | -70.04 +10log (NRB,c /50) | -70.01 +10log (NRB,c /50) | -70.01 +10log (NRB,c /50) |
| PRP Note3 | dBm/15kHz | -104 | -104 | -104 | -111 |
| RSRP Note3 | dBm/15kHz | -101 | -104 | -104 | -111 |
| Note3 | dB | -3 | -6 | -6 | -13 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , RSRP, Io and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |  |

#### A.9.8.14.2	Test Requirements

The measurement accuracy of RSTD between Cell1 and Cell3 shall fulfill the requirements in clause 9.1.12.2

The measurement accuracy of RSTD between Cell2 and Cell3 shall fulfill the requirements in clause 9.1.12.2

The measurement accuracy of RSTD between Cell4 and Cell3 shall fulfill the requirements in clause 9.1.12.1.

### A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation

#### A.9.8.15.1	Test Purpose and Environment

The purpose of these tests is to verify that the E-UTRAN TDD RSTD measurement accuracy in carrier aggregation is within the specified limits in clause 9.1.12.

There are four synchronous cells on three different carrier frequencies in the test. Cell 1 is the PCell on primary component carrier F1 (RF channel number 1), Cell 2 is an SCell on secondary component carrier F2 (RF channel number 2), Cell 3 is an SCell and reference cell on secondary component carrier F3 (RF channel number 3), and Cell 4 is the neighbor cell on F3.

Cell 1, Cell2, Cell3, and Cell 4 are included in the OTDOA assistance data. The RSTD measurements are performed

-	between Cell 4 and Cell 3 to verify the accuracy of RSTD measurement when the reference cell and neighbouring cell belong to the same secondary component carrier can meet the intra-frequency RSTD accuracy requirements defined in clause 9.1.10.1.

-	between Cell 1 and Cell 3 to verify the accuracy of RSTD measurement between the PCell and an SCell can meet the inter-frequency RSTD accuracy requirements defined in clause 9.1.10.2.

-	between Cell 2 and Cell 3 to verify the accuracy of RSTD measurement between two SCells can meet the inter-frequency RSTD accuracy requirements defined in clause 9.1.10.2.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap is not configured in the test because of UE carrier aggregation capability.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Table A.9.8.15.1-1 and Table A.9.8.15.1-2 for each of the three cells during this time.

The test parameters are given in Table A.9.8.15.1-1 and Table A.9.8.15.1-2.

Table A.9.8.15.1-1: General Test Parameters for RSTD Tests for E-UTRAN TDD for 3DL Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCell |  | Cell 1 | Cell 1 on RF channel number 1 |
| SCell 1 |  | Cell 2 | Cell 2 is an SCell on RF channel number 2 |
| SCell 2 (Assistance data reference cell) |  | Cell 3 | Cell 3 is an SCell on RF channel number 3 |
| Neighbour cell |  | Cell 4 | Cell 4 on RF channel number 3 |
| E-UTRA RF Channel Number |  | 1,2,3 | Three TDD carrier frequencies are used. |
| PRS configuration index |  | 171 for all cells on PCC 181 for all cells on SCC1 191 for all cells on SCC2 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| prs-MutingInfo |  | Cell1:‘11110000’ Cell2:‘11110000’ Cell3:’11110000’ Cell4:’11110000’ | See clause 6.5.1.2 in TS 36.355 for more information |
| prs-SubframeOffset |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. The same configuration in both cells. |
| Cell ID |  | (Cell ID of cell 4 – Cell ID of cell 3) mod 6 = 3 | PCIs of cell 1 and cell 2 are selected randomly. |
| expectedRSTD | s | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 | PRS are transmitted from synchronous cells |
| Time alignment errors between Cell 1, Cell 2, and Cell 3 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the assistance-data-reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |
|  | ms | 4960 | Derived according to the RSTD measurement requirements specified in Clause 8.1.2.6.3 |

Table A.9.8.15.1-2: Cell Specific Test Parameters for RSTD Tests for E-UTRAN TDD for Carrier Aggregation

| Parameter | Unit | Cell1 | Cell2 | Cell3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 3 | 3 |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | 5,10,20 | 5,10,20 | 5,10,20 |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.2 |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |
| OCNG Patterns defined in A.3.2.2 ( There is no PDSCH allocated in the subframe transmitting PRS) |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 | 5MHz: 25 10MHz: 50 20MHz:100 |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 | 5MHz: 2 10MHz: 1 20MHz:1 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | -3 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 |  |  |  |
| PRS | dB | -6 | -6 | -6 | -13 |
| PRS | dB | -6 | -6 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -70.04 +10log (NRB,c /50) | -70.04 +10log (NRB,c /50) | -70.01 +10log (NRB,c /50) | -70.01 +10log (NRB,c /50) |
| PRP Note3 | dBm/15kHz | -104 | -104 | -104 | -111 |
| RSRP Note3 | dBm/15kHz | -101 | -104 | -104 | -111 |
| Note3 | dB | -3 | -6 | -6 | -13 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , RSRP, Io and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS. |  |  |  |  |  |

#### A.9.8.15.2	Test Requirements

The measurement accuracy of RSTD between Cell1 and Cell3 shall fulfill the requirements in clause 9.1.12.2

The measurement accuracy of RSTD between Cell2 and Cell3 shall fulfill the requirements in clause 9.1.12.2

The measurement accuracy of RSTD between Cell4 and Cell3 shall fulfill the requirements in clause 9.1.12.1.

### A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage

#### A.9.8.16.1	Test Purpose and Environment

The purpose of the tests is to verify that the intra frequency RSTD measurement for HD-FDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.10. Test 1 is applicable for UE supporting NPRS Type 1 and Test 2 is applicable for UE supporting NPRS Type 2.

In the tests there are three synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell12 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355, shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.16.1-1, A.9.8.16.1-2 and A.9.8.16.1-3.

Table A.9.8.16.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 2 and eCell 1 |  |
| NPDCCH parameters |  | R.26 HD-FDD | As defined in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 Test2: (nprsID of Cell 1 – nprsID of Cell2)mod6=0 | As defined in TS36.366 [24] |
| NPRS Type |  | Test1: Type 1 Test2: Type 2 | As defined in TS36.211 |
| nprs-period | ms | 1280 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | as in the following 2 rows: |  |
| subframePattern10 |  | ‘0111001110’ | Correponds to subframePattern10-r14 defined in TS 36.355 [24] |
| nprsSequenceInfo |  | BWchannel 5MHz: 54 BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Measurement period | s | [11.52] | Derived according to the RSTD measurement period in clause 4.8.1 |

Table A.9.8.16.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 17 eCell 2 BWchannel 10MHz: 30 | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 17 eCell 2 BWchannel 10MHz: 30 |
| NPBCH_RA | dB | 0 | 0 | 0 | 0 |
| NPBCH_RB |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |
| NPRS_RA | dB | -7.5 | -9 | 0 | 0 |
| Note 2 | dBm/ 15 kHz | -98 | -98 | -98 | -98 |
| NPRS | dB | -0.5 | -5 | -2.37 | -8.02 |
| NPRS Note 3 | dB | -5.96 | -12.79 | -3.01 | -10.01 |
| Io Note 3 | dBm/ 180kHz | -78.40 | -78.40 | -85.60 | -85.60 |
| NPRP Note 3 | dBm/ 15 kHz | -98.5 | -103 | -100.37 | -106.02 |
| NRSRP Note 3 | dBm/ 15 kHz | -91 | -94 | -100.37 | -106.02 |
| Note 3 | dB | 7 | 4 | -2.37 | -8.02 |
| Propagation Condition |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |

Table A.9.8.16.1-3: eCell 1 and eCell 2 specific test parameters

| Parameter | Unit | Test1 |  |  |  |  |  | Test2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | eCell 1 |  |  | eCell 2 |  |  | eCell 1 |  |  | eCell 2 |  |  |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  |
| NOCNG Pattern defined in clause D.3 | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  | -98 |  |  | -98 |  |  |
| Note2 | dBm | 7 | 7 | 7 | 4 | 4 | 4 | 0 | 0 | 0 | -7 | -7 | -7 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | ms | - |  |  | 3 |  |  | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.8.16.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.10.

### A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage

#### A.9.8.17.1	Test Purpose and Environment

The purpose of the tests is to verify that the intra frequency RSTD measurement for HD-FDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.11. Test 1 is applicable for UE supporting NPRS Type 1 and Test 2 is applicable for UE supporting NPRS Type 2.

In the tests there are three synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell2 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355, shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.17.1-1, A.9.8.17.1-2 and A.9.8.17.1-3.

Table A.9.8.17.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 1 and eCell 2 |  |
| NPDCCH parameters |  | R.26 HD-FDD | As defined in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 Test2: (nprsID of Cell 1 – nprsID of Cell 2)mod6=0 | As defined in TS36.355 [24] |
| NPRS Type |  | Test1: Type 1 Test2: Type 2 | As defined in TS36.211 |
| nprs-period | ms | 1280 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| nprs-slotNumberOffset |  | 0 | As defined in TS36.355 [24] |
| nprs-SubframeOffset |  | 640 | As defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS36.355 [24] |
| NPRS muting info |  | nCell 1: ‘1111111100000000’ nCell 2: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | as in the following 3 rows: |  |
| subframePattern10 |  | ‘0111001110’ | Correponds to subframePattern10-r14 defined in TS 36.355 [24] |
| nprsSequenceInfo nCell1 |  | BWchannel 5MHz: 54 BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| nprsSequenceInfo nCell2 |  | BWchannel 5MHz: 59 BWchannel 10MHz: 135 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Measurement period | s | [20.48] | Derived according to the RSTD measurement period in clause 4.8.3 |

Table A.9.8.17.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 22 eCell 2 BWchannel 10MHz: 35 | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 22 eCell 2 BWchannel 10MHz: 35 |
| NPBCH_RA | dB | 0 | 0 | 0 | 0 |
| NPBCH_RB |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |
| NPRS_RA | dB | -13 | -17 | 0 | 0 |
| Note 2 | dBm/ 15 kHz | -98 | -98 | -98 | -98 |
| NPRS | dB | -6 | -13 | -2.37 | -8.02 |
| NPRS Note 3 | dB | -6 | -13 | -3.01 | -10.01 |
| Io Note 3 | dBm/ 180 kHz | -80.20 | -83.20 | -85.60 | -85.60 |
| NPRP Note 3 | dBm/ 15 kHz | -104 | -111 | -100.37 | -106.02 |
| NRSRP Note 3 | dBm/ 15 kHz | -91 | -94 | -100.37 | -106.02 |
| Note 3 | dB | 7 | 4 | -2.37 | -8.02 |
| Propagation Condition |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |

Table A.9.8.17.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | Test1 |  |  |  |  |  | Test2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | eCell 1 |  |  | eCell 2 |  |  | eCell 1 |  |  | eCell 2 |  |  |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  |
| NOCNG Pattern defined in clause D.3 | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  | -98 |  |  | -98 |  |  |
| Note2 | dBm | 7 | 7 | 7 | 4 | 4 | 4 | 0 | 0 | 0 | -7 | -7 | -7 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | ms | - |  |  | 3 |  |  | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.8.17.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.11.

### A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage

#### A.9.8.18.1	Test Purpose and Environment

The purpose of the tests is to verify that the intra frequency RSTD measurement for HD-FDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.12. Test 1 is applicable for UE supporting NPRS Type 1 and Test 2 is applicable for UE supporting NPRS Type 2.

In the tests there are three synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell2 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355, shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.18.1-1, A.9.8.18.1-2 and A.9.8.18.1-3.

Table A.9.8.18.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 1 and eCell 2 |  |
| NPDCCH parameters |  | R.26 HD-FDD | As defined in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 Test2: (nprsIC of Cell 1 – nprsID of Cell 2)mod6 = 0 | As defined in TS36.355 [24] |
| NPRS Type |  | Test1: Type 1 Test2: Type 2 | As defined in TS36.211 |
| nprs-period | ms | 1280 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | as in the following 2 rows: |  |
| subframePattern10 |  | ‘0111001110’ | Correponds to subframePattern10-r14 defined in TS 36.355 [24] |
| nprsSequenceInfo |  | BWchannel 5MHz: 54 BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Measurement period | s | [11.52] | Derived according to the RSTD measurement period in clause 4.8.2 |

Table A.9.8.18.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 17 eCell 2 BWchannel 10MHz: 30 | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 17 eCell 2 BWchannel 10MHz: 30 |
| NPBCH_RA | dB | 0 | 0 | 0 | 0 |
| NPBCH_RB |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |
| NPRS_RA | dB | -12.7 | -0.7 | 0 | 0 |
| Note 2 | dBm/ 15 kHz | -110 | -110 | -98 | -98 |
| NPRS | dB | -14.7 | -12.7 | -5 | -13 |
| NPRS Note 3 | dB | -14.97 | -14.82 | -5.21 | -14.19 |
| Io Note 3 | dBm/ 180kHz | -100.8 | -100.8 | -87.08 | -87.08 |
| NPRP Note 3 | dBm/ 15 kHz | -124.7 | -122.7 | -103 | -111 |
| NRSRP Note 3 | dBm/ 15 kHz | -112 | -122 | -103 | -111 |
| Note 3 | dB | -2 | -12 | -5 | -13 |
| Propagation Condition |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |

Table A.9.8.18.1-3: eCell 1 and eCell 2 specific test parameters

| Parameter | Unit | Test1 |  |  |  |  |  | Test2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | eCell 1 |  |  | eCell 2 |  |  | eCell 1 |  |  | eCell 2 |  |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  |
| NOCNG Pattern defined in clause D.3 | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -110 |  |  | -110 |  |  | -98 |  |  | -98 |  |  |
| Note2 | dBm | -2 | -2 | -2 | -12 | -12 | -12 | -9 | -9 | -9 | -9 | -9 | -9 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | ms | - |  |  | 3 |  |  | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.8.18.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.12.

### A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage

#### A.9.8.19.1	Test Purpose and Environment

The purpose of the tests is to verify that the intra frequency RSTD measurement for HD-FDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.13. Test 1 is applicable for UE supporting NPRS Type 1 and Test 2 is applicable for UE supporting NPRS Type 2.

In the tests there are three synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell2 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355, shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.19.1-1, A.9.8.19.1-2 and A.9.8.19.1-3.

Table A.9.8.19.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 1 and eCell 2 |  |
| NPDCCH parameters |  | R.26 HD-FDD | As defined in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 Test2: (nprsID of Cell 1 – nprsID of Cell 2)mod6 = 0 | As defined in TS36.355 [24] |
| NPRS Type |  | Test1: Type 1 Test2: Type 2 | As defined in TS36.211 |
| nprs-period | ms | 1280 | As defined in TS36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS36.355 [24] |
| nprs-slotNumberOffset |  | 0 | As defined in TS36.355 [24] |
| nprs-SubframeOffset |  | 640 | As defined in TS36.355 [24] |
| NPRS muting info |  | nCell 1: ‘1111111100000000’ nCell 2: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration |  | as in the following 3 rows: |  |
| subframePattern10 |  | ‘0111001110’ | Correponds to subframePattern10-r14 defined in TS 36.355 [24] |
| nprsSequenceInfo nCell1 |  | BWchannel 5MHz: 54 BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| nprsSequenceInfo nCell2 |  | BWchannel 5MHz: 59 BWchannel 10MHz: 135 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-1 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Measurement period | s | [20.48] | Derived according to the RSTD measurement period in clause 4.8.4 |

Table A.9.8.19.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 22 eCell 2 BWchannel 10MHz: 35 | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 5MHz: 22 eCell 2 BWchannel 10MHz: 35 |
| NPBCH_RA | dB | 0 | 0 | 0 | 0 |
| NPBCH_RB |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |
| NPRS_RA | dB | -13 | -3 | 0 | 0 |
| Note 2 | dBm/ 15 kHz | -110 | -110 | -98 | -98 |
| NPRS | dB | -15 | -15 | -5 | -13 |
| NPRS Note 3 | dB | -15 | -15 | -5.21 | -14.19 |
| Io Note 3 | dBm/ 180 kHz | -101.20 | -111.20 | -87.08 | -87.08 |
| NPRP Note 3 | dBm/ 15 kHz | -125 | -125 | -103 | -111 |
| NRSRP Note 3 | dBm/ 15 kHz | -112 | -122 | -103 | -111 |
| Note 3 | dB | -2 | -12 | -5 | -13 |
| Propagation Condition |  | AWGN | AWGN | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |

Table A.9.8.19.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | Test1 |  |  |  |  |  | Test2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | eCell 1 |  |  | eCell 2 |  |  | eCell 1 |  |  | eCell 2 |  |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  | 5 or 10 |  |  |
| NOCNG Pattern defined in clause D.3 | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -110 |  |  | -110 |  |  | -98 |  |  | -98 |  |  |
| Note2 | dBm | -2 | -2 | -2 | -12 | -12 | -12 | -9 | -9 | -9 | -9 | -9 | -9 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | ms | - |  |  | 3 |  |  | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.8.19.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.13.

### A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A

#### A.9.8.20.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE ModeA that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.20 and 9.1.25.4, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.20.1-1 and A.9.8.20.1-2 during this time.

The test parameters are given in Table A.9.8.20.1-1 and Table A.9.8.20.1-2.

Table A.9.8.20.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.16 FDD |  | R.16 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.21 FDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System channel Bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 151 |  | 151 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 2 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 5120 |  |  |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.3.1 (for Cat-M1) and Clause 8.16.2.3.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.20.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -2.37 | -8.02 | -6 | -13 | -2.37 | -8.02 | -6 | -13 |
| PRS Note3 | dB | -3.01 | -10.01 | -6 | -13 | -3.01 | -10.01 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -69.23 | -69.23 | -70 | -70 | -69.23 | -69.23 | -70 | -70 |
| PRP Note3 | dBm/15kHz | -100.37 | -106.02 | -104 | -111 | -100.37 | -106.02 | -104 | -111 |
| Note 3 | dB | -2.37 | -8.02 | -3 | -13 | -2.37 | -8.02 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -100.37 | -106.02 | -101 | -111 | -100.37 | -106.02 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.20.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.20.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.4.

### A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA

#### A.9.8.21.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CEModeA that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.20 and 9.1.25.4, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.21.1-1 and A.9.8.21.1-2 during this time.

The test parameters are given in Table A.9.8.21.1-1 and Table A.9.8.21.1-2.

Table A.9.8.21.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN HD-FDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.6 HD-FDD |  | R.6 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.21 FDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 151 |  | 151 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 2 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 5120 |  |  |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.3.3 (for Cat-M1) and Clause 8.16.2.3.3 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.21.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN HD-FDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -2.37 | -8.02 | -6 | -13 | -2.37 | -8.02 | -6 | -13 |
| PRS Note3 | dB | -3.01 | -10.01 | -6 | -13 | -3.01 | -10.01 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -69.23 | -69.23 | -70 | -70 | -69.23 | -69.23 | -70 | -70 |
| PRP Note3 | dBm/15kHz | -100.37 | -106.02 | -104 | -111 | -100.37 | -106.02 | -104 | -111 |
| Note 3 | dB | -2.37 | -8.02 | -3 | -13 | -2.37 | -8.02 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -100.37 | -106.02 | -101 | -111 | -100.37 | -106.02 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.21.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.20.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.4.

### A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A

#### A.9.8.22.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.20 and 9.1.25.4, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.22.1-1 and A.9.8.22.1-2 during this time.

The test parameters are given in Table A.9.8.22.1-1 and Table A.9.8.22.1-2.

Table A.9.8.22.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.14 TDD |  | R.14 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.11 TDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| Special subframe configuration |  | 6 |  |  |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 1 |  |  |  | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. The same configuration in both cells. |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 154 |  | 154 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 2 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 5120 |  |  |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.3.2 (for Cat-M1) and Clause 8.16.2.3.2 (Cat-M2) |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.22.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -2.37 | -8.02 | -6 | -13 | -2.37 | -8.02 | -6 | -13 |
| PRS Note3 | dB | -3.01 | -10.01 | -6 | -13 | -3.01 | -10.01 | -6 | -13 |
| Io Note3 | dBm/9 MHz | -69.23 | -69.23 | -70 | -70 | -69.23 | -69.23 | -70 | -70 |
| PRP Note3 | dBm/15kHz | -100.37 | -106.02 | -104 | -111 | -100.37 | -106.02 | -104 | -111 |
| Note 3 | dB | -2.37 | -8.02 | -3 | -13 | -2.37 | -8.02 | -3 | -13 |
| RSRP Note 3 | dBm/15kHz | -100.37 | -106.02 | -101 | -111 | -100.37 | -106.02 | -101 | -111 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.22.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.20.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.4.

### A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B

#### A.9.8.23.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.21 and 9.1.25.5, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.23.1-1 and A.9.8.23.1-2 during this time.

The test parameters are given in Table A.9.8.23.1-1 and Table A.9.8.23.1-2.

Table A.9.8.23.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.18 FDD |  | R.18 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.21 FDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System channel bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 151 |  | 151 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 4 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 12800 |  | 5120 |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.3.3.1 (for Cat-M1) and Clause 8.16.1.1.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.23.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN FDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note 2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -5 | -13 | -15 | -15 | -5 | -13 | -15 | -15 |
| PRS Note 3 | dB | -5.21 | -14.19 | -15 | -15 | -5.21 | -14.19 | -15 | -15 |
| Io Note 3 | dBm/9 MHz | -69.79 | -69.79 | -70.06 | -70.06 | -69.79 | -69.79 | -70.06 | -70.06 |
| PRP Note 3 | dBm/15kHz | -103 | -111 | -113 | -113 | -103 | -111 | -113 | -113 |
| Note 3 | dB | -5 | -13 | -12 | -15 | -5 | -13 | -12 | -15 |
| RSRP Note 3 | dBm/15kHz | -103 | -111 | -110 | -113 | -103 | -111 | -110 | -113 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.23.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.21.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.5.

### A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B

#### A.9.8.24.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.21 and 9.1.25.5, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.24.1-1 and A.9.8.24.1-2 during this time.

The test parameters are given in Table A.9.8.24.1-1 and Table A.9.8.24.1-2.

Table A.9.8.24.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN HD-FDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.8 HD-FDD |  | R.86 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.21 FDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System channel bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 151 |  | 151 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 4 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | ms | 12800 |  | 5120 |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.3.3.3 (for Cat-M1) and Clause 8.16.3.1.3 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.24.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN HD-FDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note 2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -5 | -13 | -15 | -15 | -5 | -13 | -15 | -15 |
| PRS Note 3 | dB | -5.21 | -14.19 | -15 | -15 | -5.21 | -14.19 | -15 | -15 |
| Io Note 3 | dBm/9 MHz | -69.79 | -69.79 | -70.06 | -70.06 | -69.79 | -69.79 | -70.06 | -70.06 |
| PRP Note 3 | dBm/15kHz | -103 | -111 | -113 | -113 | -103 | -111 | -113 | -113 |
| Note 3 | dB | -5 | -13 | -12 | -15 | -5 | -13 | -12 | -15 |
| RSRP Note 3 | dBm/15kHz | -103 | -111 | -110 | -113 | -103 | -111 | -110 | -113 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.24.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.21.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.5.

### A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B

#### A.9.8.25.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD intra-frequency measurement accuracy is within the specified limits in sections 9.1.21.21 and 9.1.25.5, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell on the same frequency. Tests 1 and 2 are applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Tests 3 and 4 are applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.25.1-1 and A.9.8.25.1-2 during this time.

The test parameters are given in Table A.9.8.25.1-1 and Table A.9.8.25.1-2.

Table A.9.8.25.1-1: General Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| M-PDCCH parameters |  | R.16 TDD |  | R.16 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| OCNG Patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.11 TDD |  | OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). |
| Reference cell |  | Cell 1 |  |  |  |  |
| Neighbour cell |  | Cell 2 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  | One FDD carrier frequency is used. |
| System channel bandwidth (BWchannel) | MHz | 10 |  | 10 |  |  |
| Special subframe configuration |  | 6 |  |  |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells. |
| Uplink-downlink configuration |  | 1 |  |  |  | As specified in table 4.2-2 in TS 36.211 and table 8.1.2.5.2-2. The same configuration in both cells. |
| PRS Bandwidth | RB | 50 Note 1 |  | 50 Note 1 |  | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| PRS configuration Index |  | 154 |  | 154 |  | As defined in TS 36.211 |
| Number of consecutive positioning downlink subframes |  | 6 |  | 4 |  | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  |  |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 1 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 3 |  |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 | 5 | 5 |  |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: 3 | Cell 2 to Cell 1: -3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  |  |  | The number of cells includes the reference cell |
|  | Ms | 12800 |  | 5120 |  | Derived according to the RSTD measurement requirements specified in Clause 8.13.3.3.2 (for Cat-M1) and Clause 8.16.3.1.2 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |  |  |

Table A.9.8.25.1-2: Cell Specific Test Parameters for intra frequency RSTD Tests for E-UTRAN TDD

| Parameter | Unit | Test1 |  | Test2 |  | Test3 |  | Test4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | -3 | 0 | 0 | 0 | -3 | 0 |
| Note 2 | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| PRS | dB | -5 | -13 | -15 | -15 | -5 | -13 | -15 | -15 |
| PRS Note 3 | dB | -5.21 | -14.19 | -15 | -15 | -5.21 | -14.19 | -15 | -15 |
| Io Note 3 | dBm/9 MHz | -69.79 | -69.79 | -70.06 | -70.06 | -69.79 | -69.79 | -70.06 | -70.06 |
| PRP Note 3 | dBm/15kHz | -103 | -111 | -113 | -113 | -103 | -111 | -113 | -113 |
| Note 3 | dB | -5 | -13 | -12 | -15 | -5 | -13 | -12 | -15 |
| RSRP Note 3 | dBm/15kHz | -103 | -111 | -110 | -113 | -103 | -111 | -110 | -113 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |  |  |  |  |

#### A.9.8.25.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.21.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.5.

### A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A

#### A.9.8.26.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE ModeA that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.17 and 9.1.25.1, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on FDD RF channel 1 and Cell 2 is on FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.26.1-1 and A.9.8.26.1-2 during this time.

The test parameters are given in Table A.9.8.26.1-1 and Table A.9.8.26.1-2.

Table A.9.8.26.1-1: General Test Parameters for E-UTRAN FDD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.16 FDD | R.16 FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 2 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 15360 | 10240 | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.4.1 (for Cat-M1) and Clause 8.16.2.4.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.26.1-2: Cell Specific Test Parameters for E-UTRAN FDD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PRS configuration Index , as defined in TS 36.211 |  | 142 | 152 | 142 | 152 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -1 | -11 | -1 | -11 |
| PRS Note3 | dB | -1 | -11 | -1 | -11 |
| Io Note3 | dBm/9 MHz | -69.68 | -70.16 | -69.68 | -70.16 |
| PRP Note3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Note 3 | dB | -1 | -11 | -1 | -11 |
| RSRP Note 3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.26.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.17.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.1.

### A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A

#### A.9.8.27.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE ModeA that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.17 and 9.1.25.1, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on FDD RF channel 1 and Cell 2 is on FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.27.1-1 and A.9.8.27.1-2 during this time.

The test parameters are given in Table A.9.8.27.1-1 and Table A.9.8.27.1-2.

Table A.9.8.27.1-1: General Test Parameters for E-UTRAN HD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.6 HD-FDD | R.6 HD-FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 2 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 15360 | 10240 | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.4.1 (for Cat-M1) and Clause 8.16.2.4.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.27.1-2: Cell Specific Test Parameters for E-UTRAN HD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PRS configuration Index , as defined in TS 36.211 |  | 142 | 152 | 142 | 152 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -1 | -11 | -1 | -11 |
| PRS Note3 | dB | -1 | -11 | -1 | -11 |
| Io Note3 | dBm/9 MHz | -69.68 | -70.16 | -69.68 | -70.16 |
| PRP Note3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Note 3 | dB | -1 | -11 | -1 | -11 |
| RSRP Note 3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.27.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.17.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.1.

### A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A

#### A.9.8.28.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE ModeA that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.17 and 9.1.25.1, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on TDD RF channel 1 and Cell 2 is on TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.28.1-1 and A.9.8.28.1-2 during this time.

The test parameters are given in Table A.9.8.28.1-1 and Table A.9.8.28.1-2.

Table A.9.8.28.1-1: General Test Parameters for E-UTRAN TDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.14 TDD | R.14 TDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 2 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 15360 | 10240 | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.4.2 (for Cat-M1) and Clause 8.16.2.4.2 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.28.1-2: Cell Specific Test Parameters for E-UTRAN TDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PRS configuration Index , as defined in TS 36.211 |  | 140 | 150 | 140 | 150 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -1 | -11 | -1 | -11 |
| PRS Note3 | dB | -1 | -11 | -1 | -11 |
| Io Note3 | dBm/9 MHz | -69.68 | -70.16 | -69.68 | -70.16 |
| PRP Note3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Note 3 | dB | -1 | -11 | -1 | -11 |
| RSRP Note 3 | dBm/15kHz | -99 | -109 | -99 | -109 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.28.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.17.

For Cat-M2 UE in CE Mode A, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.1.

### A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B

#### A.9.8.29.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.18 and 9.1.25.2, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on FDD RF channel 1 and Cell 2 is on FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.29.1-1 and A.9.8.29.1-2 during this time.

The test parameters are given in Table A.9.8.29.1-1 and Table A.9.8.29.1-2.

Table A.9.8.29.1-1: General Test Parameters for E-UTRAN FDD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.18 FDD | R.18 FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 4 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 40.96 | 10.24 | Derived according to the RSTD measurement requirements specified in Clause 8.13.3.7.1 (for Cat-M1) and Clause 8.16.3.2.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.29.1-2: Cell Specific Test Parameters for E-UTRAN FDD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PRS configuration Index , as defined in TS 36.211 |  | 142 | 152 | 142 | 152 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -11 | -14 | -11 | -14 |
| PRS Note3 | dB | -11 | -14 | -11 | -14 |
| Io Note3 | dBm/9 MHz | -70.16 | -70.19 | -70.16 | -70.19 |
| PRP Note3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Note 3 | dB | -11 | -14 | -11 | -14 |
| RSRP Note 3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.29.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.18.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.2.

### A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B

#### A.9.8.30.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.18 and 9.1.25.2, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on FDD RF channel 1 and Cell 2 is on FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.30.1-1 and A.9.8.30.1-2 during this time.

The test parameters are given in Table A.9.8.30.1-1 and Table A.9.8.30.1-2.

Table A.9.8.30.1-1: General Test Parameters for E-UTRAN HD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.8 HD-FDD | R.8 HD-FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 4 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 40.96 | 10.24 | Derived according to the RSTD measurement requirements specified in Clause 8.13.2.4.1 (for Cat-M1) and Clause 8.16.2.4.1 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.30.1-2: Cell Specific Test Parameters for E-UTRAN HD-FDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PRS configuration Index , as defined in TS 36.211 |  | 142 | 152 | 142 | 152 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -11 | -14 | -11 | -14 |
| PRS Note3 | dB | -11 | -14 | -11 | -14 |
| Io Note3 | dBm/9 MHz | -70.16 | -70.19 | -70.16 | -70.19 |
| PRP Note3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Note 3 | dB | -11 | -14 | -11 | -14 |
| RSRP Note 3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.30.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.18.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.2.

### A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B

#### A.9.8.31.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD inter-frequency measurement accuracy is within the specified limits in sections 9.1.21.18 and 9.1.25.2, respectively, in AWGN channels.

In the test, there are two synchronous cells, Cell 1 as the reference cell and Cell 2 as the neighbour cell. Cell 1 is on TDD RF channel 1 and Cell 2 is on TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured not to overlap with PRS subframes of Cell 1. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE before the measurement period. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of measurement period, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE before the measurements start.

There is no PDSCH allocated in the subframe transmitting PRS. A time span of  is provided for the measurement period, and PRS are configured according to  in Tables A.9.8.31.1-1 and A.9.8.31.1-2 during this time.

The test parameters are given in Table A.9.8.31.1-1 and Table A.9.8.31.1-2.

Table A.9.8.31.1-1: General Test Parameters for E-UTRAN TDD inter-frequency RSTD Tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| MPDCCH parameters |  | R.16 TDD | R.16 TDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| Reference cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  |  |
| E-UTRA RF Channel Number |  | Cell 1: 1 Cell 2: 2 |  | The two cells are on different frequencies. |
| System Channel Bandwidth (BWchannel) | MHz | 10 | 10 |  |
| PRS Bandwidth | RB | 50 Note 1 | 50 Note 1 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. |
| Number of consecutive positioning downlink subframes |  | 4 | 4 | As defined in TS 36.211 |
| prs-MutingInfo |  | Cell 1: ‘11110000’ Cell 2: ‘11110000’ |  | See clause 6.5.1.2 in TS 36.355 for more information |
| Cell ID |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| prs-SubframeOffset |  | 10 | 10 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| expectedRSTD | s | Cell 2: 1 Other neighbour cells: randomly between -3 and 3 | Cell 2: -1 Other neighbour cells: randomly between -3 and 3 |  |
| expectedRSTDUncertainty for all neighbour cells | s | 5 | 5 |  |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: -3 | Cell 2 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The number of cells includes the reference cell |
|  | ms | 40.96 | 10.24 | Derived according to the RSTD measurement requirements specified in Clause 8.13.3.7.2 (for Cat-M1) and Clause 8.16.3.2.2 (Cat-M2). |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.9.8.31.1-2: Cell Specific Test Parameters for E-UTRAN TDD inter-frequency RSTD Tests

| Parameter | Unit | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell1 | Cell2 | Cell1 | Cell2 |
| E-UTRA RF Channel Number |  | 1 | 2 | 1 | 2 |
| Gap offset |  | 9 | N/A | 9 | N/A |
| Gap pattern |  | #0 | N/A | #0 | N/A |
| OCNG Patterns defined in A.3.2.1 |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PRS configuration Index , as defined in TS 36.211 |  | 140 | 150 | 140 | 150 |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| PRS_RA | dB | 0 | 0 | 0 | 0 |
| Note2 | dBm/15 kHz | -98 | -98 | -98 | -98 |
| PRS | dB | -11 | -14 | -11 | -14 |
| PRS Note3 | dB | -11 | -14 | -11 | -14 |
| Io Note3 | dBm/9 MHz | -70.16 | -70.19 | -70.16 | -70.19 |
| PRP Note3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Note 3 | dB | -11 | -14 | -11 | -14 |
| RSRP Note 3 | dBm/15kHz | -109 | -112 | -109 | -112 |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols (other than those in the PRS subframes). Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , PRS , Io, RSRP and PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io values are derived in the case that there is no PBCH, PSS or SSS in the OFDM symbols carrying PRS |  |  |  |  |  |

#### A.9.8.31.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.21.18.

For Cat-M2 UE in CE Mode B, the RSTD measurement accuracy shall fulfill the requirements in clause 9.1.25.2.

### A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage

#### A.9.8.32.1	Test Purpose and Environment

The purpose of the test is to verify that the intra frequency RSTD measurement for TDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.10.

In the test there are four synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell12 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.32.1-1, A.9.8.32.1-2 and A.9.8.32.1-3.

Table A.9.8.32.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 2 and eCell 1 |  |
| NPDCCH parameters |  | R.26 NB-TDD | Specified in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration: subframePattern10-TDD |  | ‘01100011’ | Correponds to subframePattern10-TDD-r15 defined in TS 36.355 [24] |
| nprsSequenceInfo |  | BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Specified in section A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| Measurement period | s | 11.52 | Derived according to the RSTD measurement period in clause 4.8.1 |

Table A.9.8.32.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 10MHz: 30 |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| OCNG_RA Note 1 |  |  |  |
| OCNG_RB Note 1 |  |  |  |
| NPRS_RA | dB |  |  |
| Note 2 | dBm/ 15 kHz | -98 | -98 |
| NPRS | dB | -6 | -13 |
| NPRS Note 3 | dB | -6 | -13 |
| Io Note 3 | dBm/ 180kHz | -86.89 | -86.89 |
| NPRP Note 3 | dBm/ 15 kHz | -104 | -111 |
| NRSRP Note 3 | dBm/ 15 kHz | -104 | -111 |
| Note 3 | dB | -6 | -13 |
| Propagation Condition |  | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |

Table A.9.8.32.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | eCell 1 |  |  | eCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| NOCNG Pattern | - | NOP.1 TDD |  |  | NOP.1 TDD |  |  |
| PBCH_RA |  | dB | -3 |  |  | -3 |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Note2 | dBm | 0 | 0 | 0 | -7 | -7 | -7 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | us | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |

#### A.9.8.32.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.10.

### A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage

#### A.9.8.33.1	Test Purpose and Environment

The purpose of the test is to verify that the inter frequency RSTD measurement for TDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.11.

In the test there are four synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell2 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.33.1-1, A.9.8.33.1-2 and A.9.8.33.1-3.

Table A.9.8.33.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 1 and eCell 2 |  |
| NPDCCH parameters |  | R.26 NB-TDD | Specified in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| nprs-slotNumberOffset |  | 0 | As defined in TS 36.355 [24] |
| nprs-SubframeOffset |  | 640 | As defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| NPRS muting info |  | nCell 1: ‘1111111100000000’ nCell 2: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration: subframePattern10-TDD |  | ‘01100011’ | Correponds to subframePattern10-TDD-r15 defined in TS 36.355 [24] |
| nprsSequenceInfo nCell1 |  | BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| nprsSequenceInfo nCell2 |  | BWchannel 10MHz: 135 | Correponds to nprsSequenceInfo defined in TS36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Specified in section A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| Measurement period | s | 20.48 | Derived according to the RSTD measurement period in clause 4.8.3 |

Table A.9.8.33.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 10MHz: 35 |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| OCNG_RA Note 1 |  |  |  |
| OCNG_RB Note 1 |  |  |  |
| NPRS_RA |  |  |  |
| Note 2 | dBm/ 15 kHz | -98 | -98 |
| NPRS | dB | -6 | -13 |
| NPRS Note 3 | dB | -6 | -13 |
| Io Note 3 | dBm/ 15kHz | -86.93 | -86.93 |
| NPRP Note 3 | dBm/ 15 kHz | -104 | -111 |
| NRSRP Note 3 | dBm/ 15 kHz | -104 | -111 |
| Note 3 | dB | -6 | -13 |
| Propagation Condition |  | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |

Table A.9.8.33.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | eCell 1 |  |  | eCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| NOCNG Pattern | - | NOP.1 TDD |  |  | NOP.1 TDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Note2 | dBm | 3 | 3 | 3 | 3 | 3 | 3 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | us | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |

#### A.9.8.33.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.11.

### A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage

#### A.9.8.34.1	Test Purpose and Environment

The purpose of the test is to verify that the intra frequency RSTD measurement for TDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.12.

In the test there are four synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell12 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.34.1-1, A.9.8.34.1-2 and A.9.8.34.1-3.

Table A.9.8.34.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 2 and eCell 1 |  |
| NPDCCH parameters |  | R.26 NB-TDD | Specified in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘11110000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration: subframePattern-TDD |  | ‘01100011’ | Correponds to subframePattern10-TDD-r15 defined in TS 36.355 [24] |
| nprsSequenceInfo |  | BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Specified in section A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| Measurement period | s | 11.52 | Derived according to the RSTD measurement period in clause 4.8.1 |

Table A.9.8.34.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 10MHz: 30 |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| OCNG_RA Note 1 |  |  |  |
| OCNG_RB Note 1 |  |  |  |
| NPRS_RA | dB |  |  |
| Note 2 | dBm/ 15 kHz | -98 | -98 |
| NPRS | dB | -15 | -15 |
| NPRS Note 3 | dB | -15 | -15 |
| Io Note 3 | dBm/ 180kHz | -87.14 | -87.14 |
| NPRP Note 3 | dBm/ 15 kHz | -113 | -113 |
| NRSRP Note 3 | dBm/ 15 kHz | -113 | -113 |
| Note 3 | dB | -15 | -15 |
| Propagation Condition |  | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |

Table A.9.8.34.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | eCell 1 |  |  | eCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| NOCNG Pattern | - | NOP.1 TDD |  |  | NOP.1 TDD |  |  |
| PBCH_RA |  | dB | -3 |  |  | -3 |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Note2 | dBm | -9 | -9 | -9 | -9 | -9 | -9 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | us | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |

#### A.9.8.34.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.12.

### A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage

#### A.9.8.35.1	Test Purpose and Environment

The purpose of the test is to verify that the inter frequency RSTD measurement for TDD category NB1 UE meets the accuracy requirements specified in Clause 9.1.22.13.

In the test there are four synchronous cells: nCell 1, nCell 2, eCell1 and eCell 2. nCell 1 is the reference as well as the PCell. nCell 2, eCell1 and eCell12 are the neighbour cells.

The OTDOA assistance data and OTDOA-RequestLocationInformation as defined in TS 36.355 [24], shall be provided to the UE. After the receipt of the OTDOA assistance data and OTDOA-RequestLocationInformation has been successfully acknowledged, the UE is provided with a RRC connection release command. The UE is expected to enter RRC_IDLE before the measurement period.

The test parameters are given in Tables A.9.8.35.1-1, A.9.8.35.1-2 and A.9.8.35.1-3.

Table A.9.8.35.1-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Inband |  |
| Reference cell |  | nCell 1 |  |
| Neighbor cells |  | nCell 2, eCell 2 and eCell 1 |  |
| NPDCCH parameters |  | R.26 NB-TDD | Specified in section A.3.1.6.1 |
| nprsID |  | Test1: (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 | As defined in TS 36.355 [24] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [24] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [24] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [24] |
| NPRS muting info |  | nCell 1: ‘1111111100000000’ nCell 2: ‘1111111100000000’ | Correponds to nprs-MutingInfoB defined in TS 36.355 [24] |
| PartA Configuration : subframePattern10 |  | ‘01100011’ | Correponds to subframePattern10-r14 defined in TS 36.355 [24] |
| nprsSequenceInfo nCell1 |  | BWchannel 10MHz: 130 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| nprsSequenceInfo nCell2 |  | BWchannel 10MHz: 135 | Correponds to nprsSequenceInfo defined in TS 36.355 [24] |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Specified in section A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Expected RSTD | s | nCell 2: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [16] |
| Measurement period | s | 20.48 | Derived according to the RSTD measurement period in clause 4.8.1 |

Table A.9.8.35.1-2: nCell1 and nCell2 specific test parameters

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | nCell 1 | nCell 2 |
| BWchannel | kHz | 180 | 180 |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 | eCell 2 BWchannel 10MHz: 30 |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| OCNG_RA Note 1 |  |  |  |
| OCNG_RB Note 1 |  |  |  |
| NPRS_RA | dB |  |  |
| Note 2 | dBm/ 15 kHz | -98 | -98 |
| NPRS | dB | -15 | -15 |
| NPRS Note 3 | dB | -15 | -15 |
| Io Note 3 | dBm/ 180kHz | -87.14 | -87.14 |
| NPRP Note 3 | dBm/ 15 kHz | -113 | -113 |
| NRSRP Note 3 | dBm/ 15 kHz | -113 | -113 |
| Note 3 | dB | -15 | -15 |
| Propagation Condition |  | AWGN | AWGN |
| Antenna Configuration |  | 1x1 | 1x1 |
| Timing offset to nCell 1 | us | N/A | 3 |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |

Table A.9.8.35.1-3: eCell 1 and eCell2 specific test parameters

| Parameter | Unit | eCell 1 |  |  | eCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| NOCNG Pattern | - | NOP.1 TDD |  |  | NOP.1 TDD |  |  |
| PBCH_RA |  | dB | -3 |  |  | -3 |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Note2 | dBm | -9 | -9 | -9 | -9 | -9 | -9 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to eCell 1 | us | - |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that the Cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |  |  |

#### A.9.8.35.2	Test Requirements

The RSTD measurement accuracy shall fulfill the requirements in clause 9.1.22.13.

## A.9.9	RSRP and RSRQ on the serving cell

### A.9.9.1	FDD Intra frequency serving cell case

#### A.9.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP/ RSRQ absolute measurement accuracy is within the specified limits. This test will verify the requirements in Clause 9.1.2.1 and 9.1. 5.1 for FDD intra frequency measurements.

#### A.9.9.1.2	Test parameters

In this set of test case there is only the serving cell. Absolute accuracy of RSRP/ RSRQ intra frequency measurements for the serving cell is tested by using the parameters in Table A.9.9.1.2-1. In the test case, Cell 1 is the serving cell.

Table A.9.9.1.2-1: RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test |
| --- | --- | --- | --- |
|  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| BWchannel |  | MHz | 10 |
| Measurement bandwidth |  |  | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD |
| PDSCH allocation |  |  | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  | OP.1 FDD |
| PBCH_RA |  | dB | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -122 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -121.5 |
|  | Bands FDD_C |  | -121 |
|  | Bands FDD_D |  | -120.5 |
|  | Bands FDD_E, FDD_F Note 5 |  | -120 |
|  | Bands FDD_G Note 7 |  | -119 |
|  | Bands FDD_H |  | -118.5 |
|  |  | dB | -4 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -126 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -125.5 |
|  | Bands FDD_C |  | -125 |
|  | Bands FDD_D |  | -124.5 |
|  | Bands FDD_E, FDD_F Note 5 |  | -124 |
|  | Bands FDD_G Note 7 |  | -123 |
|  | Bands FDD_H |  | -122.5 |
| RSRQNote3 | Bands FDD_A Note 8 | dB | -16.25 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |
|  | Bands FDD_C |  |  |
|  | Bands FDD_D |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |
|  | Bands FDD_G Note 7 |  |  |
|  | Bands FDD_H |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -92.76 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  | -92.26 |
|  | Bands FDD_C |  | -91.76 |
|  | Bands FDD_D |  | -91.26 |
|  | Bands FDD_E, FDD_F Note 5 |  | -90.76 |
|  | Bands FDD_G Note 7 |  | -89.76 |
|  | Bands FDD_H |  | -89.26 |
|  |  | dB | -4 |
| Propagation condition |  | - | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |

#### A.9.9.1.3	Test Requirements

The absolute RSRP and RSRQ measurement accuracy shall fulfil the requirements in clause 9.1.2.1 and 9.1.5.1 respectively.

### A.9.9.2	TDD Intra frequency serving cell case

#### A.9.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP/ RSRQ absolute measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.2.1 and 9.1.5.1 for TDD intra frequency measurements.

#### A.9.9.2.2	Test parameters

In this set of test case there is only the serving cell. Absolute accuracy of RSRP/ RSRQ intra frequency measurements for the serving cell is tested by using the parameters in Table A.9.9.2.2-1. In the test case, Cell 1 is the serving cell.

Table A.9.9.2.2-1: RSRP TDD Intra frequency test parameters

| Parameter |  | Unit | Test |
| --- | --- | --- | --- |
|  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| BWchannel |  | MHz | 10 |
| Special subframe configurationNote1 |  |  | 6 |
| Uplink/downlink configurationNote1 |  |  | 1 |
| Measurement bandwidth |  |  | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD |
| PDSCH allocation |  |  | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  |  | OP.1 TDD |
| PBCH_RA |  | dB | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote2 |  |  |  |
| OCNG_RBNote2 |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -122 |
|  | Bands TDD_C |  | -121 |
|  | Bands TDD_E |  | -120 |
|  |  | dB | -4 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -126 |
|  | Bands TDD_C |  | -125 |
|  | Bands TDD_E |  | -124 |
| RSRQNote4 | Bands TDD_A | dB | -16.25 |
|  | Bands TDD_C |  |  |
|  | Bands TDD_E |  |  |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -92.76 |
|  | Bands TDD_C |  | -91.76 |
|  | Bands TDD_E |  | -90.76 |
|  |  | dB | -4 |
| Propagation condition |  | - | AWGN |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

#### A.9.9.2.3	Test Requirements

The absolute RSRP and RSRQ measurement accuracy shall fulfil the requirements in section 9.1.2.1 and 9.1.5.1 respectively.

## A.9.10	SSTD

### A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC

#### A.9.10.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SSTD measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.20 for FDD SSTD measurements.

#### A.9.10.1.2	Test parameters

The test parameters are given in Tables A.9.10.1.2-1 and A.9.10.1.2-2. In this test there are 2 cells.  Cell 1 is the PCell and cell 2 is the PScell. Cell 1 and cell 2 are on different frequencies. The SSTD time difference between PCell and PSCell reported by the UE is compared to the actual SSTD. The SSTD time difference between PCell and PSCell shall be set by the test equipment to one of the time differences in table A.9.10.1.2-3.

Table A.9.10.1.2-1: EUTRAN FDD-FDD SSTD accuracy in asynchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.9.10.1.2-2: EUTRAN FDD-FDD SSTD accuracy in asynchronous DC

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | -3 | -3 |
|  | dB | -3 | -3 |
| RSRP Note 3 | dBm/15 KHz | -107 | -107 |
| SCH_RP Note 3 | dBm/15 KHz | -107 | -107 |
| Io Note 3 | dBm/Ch BW | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Receive Time offset to cell1 Note 4 | s | - | Between 22.8 and 42.9 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Offset shall be chosen by the test equipment. Test equipment may also select any SFN and frame offset between cell 1 and cell 2 |  |  |  |

Table A.9.10.1.2-3: EUTRAN FDD-FDD SSTD accuracy in asynchronous DC timing offsets

| Configuration | SFN offset between MeNB and SeNB (ΔX) | Frame boundary offset between MeNB and SeNB (ΔY) | Subframe boundary offset between MeNB and SeNB (ΔZ). |
| --- | --- | --- | --- |
| 1 | 100 | -4 | 800 |
| 2 | 300 | -2 | 900 |
| 3 | 500 | 0 | 1000 |
| 4 | 700 | 3 | 1100 |
| 5 | 900 | 5 | 1200 |

#### A.9.10.1.3	Test Requirements

The SSTD reported by the UE consists of 3 elements,  SFN offset between MeNB and SeNB (ΔX), frame boundary offset between MeNB and SeNB (ΔY)  and subframe boundary offset between MeNB and SeNB (ΔZ).

The reported ΔX, ΔY and ΔZ shall meet the accuracy requirements in section 9.1.20.

### A.9.10.2	Void

### A.9.10.3	Void

### A.9.10.4	Void

## A.9.11 RSSI

### A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)

#### A.9.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the average RSSI measurement accuracy is within the specified limits. This test will partially verify the requirements in Section 9.1.18.5.2.

#### A.9.11.1.2	Test parameters

In all test cases, Cell 1 is the PCell and Cell 2 the FS3 Scell. RSSI is measured on channel number 2.

Table A.9.11.1.2-1: Average RSSI test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5 10 20 | 20 |
| Listen before talk model |  | Not applicable | Not used |
| Measurement bandwidth |  | 6 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | R0.FS3 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | R0.FS3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | OP.14 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -106 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -83 |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | 2.5 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | -Infinity |
| RSRP in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -103.5 | -103.5 |
| RSRP in subframes corresponding to RSSI measurement time configuration (RMTC) |  | -103.5 | -Infinity |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -64.43 |
| Io within measurement bandwidth in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -83 |
| Propagation condition | - | AWGN |  |

Table A.9.11.1.2-2: Average RSSI RMTC and DMTC parameters

| measDuration-r13 | sym14 |
| --- | --- |
| rmtc-Period-r13 | ms40 |
| rmtc-SubframeOffset-r13 | 20 |
| ReportInterval | ms120 |
| dmtc-PeriodOffset-r12 | ms40-r12 value 0 |

#### A.9.11.1.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 9.1.18.5.2. The nominal RSSI used to evaluate the requirement shall be based on Io in subframes corresponding to RSSI measurement time configuration (RMTC).

### A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)

#### A.9.11.2.1	Test Purpose and Environment

The purpose of this test is to verify that the average RSSI measurement accuracy is within the specified limits. This test will partially verify the requirements in Section 9.1.18.5.2.

#### A.9.11.2.2	Test parameters

In all test cases, Cell 1 is the PCell and Cell 2 the FS3 Scell. RSSI is measured on channel number 2.

Table A.9.11.2.2-1: Average RSSI test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5 10 20 | 20 |
| Listen before talk model |  | Not applicable | Not used |
| Measurement bandwidth |  | 6 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | R0.FS3 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | R0.FS3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | OP.14 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -106 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -83 |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | 2.5 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | -Infinity |
| RSRP in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -103.5 | -103.5 |
| RSRP in subframes corresponding to RSSI measurement time configuration (RMTC) |  | -103.5 | -Infinity |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -64.43 |
| Io within measurement bandwidth in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -83 |
| Propagation condition | - | AWGN |  |

Table A.9.11.2.2-2: Average RSSI RMTC and DMTC parameters

| measDuration-r13 | sym14 |
| --- | --- |
| rmtc-Period-r13 | ms40 |
| rmtc-SubframeOffset-r13 | 20 |
| ReportInterval | ms120 |
| dmtc-PeriodOffset-r12 | ms40-r12 value 0 |

#### A.9.11.2.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 9.1.18.5.2. The nominal RSSI used to evaluate the requirement shall be based on Io in subframes corresponding to RSSI measurement time configuration (RMTC).

## A.9.12	Channel occupancy

### A.9.12.1	FS3 channel occupancy test (PCell using FDD)

#### A.9.12.1.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy is within the specified limits. This test will partially verify the requirements in Section 9.1.18.6.1.

#### A.9.12.1.2	Test parameters

In all test cases, Cell 1 is the PCell and Cell 2 the FS3 Scell. Channel occupancy is measured on channel number 2.

Table A.9.12.1.2-1: Channel occupancy test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5 10 20 | 20 |
| Listen before talk model |  | Not applicable | Not used |
| Measurement bandwidth |  | 6 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD | R0.FS3 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | R0.FS3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD | OP.14 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -106 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 = 2 (Note 1) | dBm/15 kHz | -106 | -90.28 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 is not equal to 2 (Note 1) | dBm/15 kHz | -106 | -97.28 |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | 2.5 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) | dB | 2.5 | -Infinity |
| RSRP in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -103.5 | -103.5 |
| RSRP in subframes corresponding to RSSI measurement time configuration (RMTC) |  | -103.5 | -Infinity |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 = 2 (Note 1) | dBm/1.08 MHz | -83 | -71.73 |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 is not equal to 2 (Note 1) | dBm/1.08 MHz | -83 | -78.73 |
| Io within measurement bandwidth in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -83 |
| Propagation condition | - | AWGN |  |
| channelOccupancyThreshold | dBm | -75.2 |  |
| Note 1: Accumulated system frame number is used to avoid a configuration not matching the test purpose at the boundary of hyper frame numbers. |  |  |  |

Table A.9.12.1.2-2: Channel occupancy RMTC and DMTC parameters

| measDuration-r13 | sym14 |
| --- | --- |
| rmtc-Period-r13 | ms40 |
| rmtc-SubframeOffset-r13 | 20 |
| ReportInterval | ms120 |
| dmtc-PeriodOffset-r12 | ms40-r12 value 0 |

#### A.9.12.1.3	Test Requirements

The nominal reported channelOccupancy shall be 33. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.9.12.2	FS3 channel occupancy test (PCell using TDD)

#### A.9.12.2.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy is within the specified limits. This test will partially verify the requirements in Section 9.1.18.6.1.

#### A.9.12.2.2	Test parameters

In all test cases, Cell 1 is the PCell and Cell 2 the FS3 Scell. Channel occupancy is measured on channel number 2.

Table A.9.12.2.2-1: Channel occupancy test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5 10 20 | 20 |
| Listen before talk model |  | Not applicable | Not used |
| Measurement bandwidth |  | 6 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | R0.FS3 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | R0.FS3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | OP.14 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
| in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -106 | -106 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 = 2 (Note 1) | dBm/15 kHz | -106 | -90.28 |
| in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 is not equal to 2 (Note 1) | dBm/15 kHz | -106 | -97.28 |
| in subframes not corresponding to RSSI measurement time configuration (RMT) | dB | 2.5 | 2.5 |
| in subframes corresponding to RSSI measurement time configuration (RMTC | dB | 2.5 | -Infinity |
| RSRP in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/15 kHz | -103.5 | -103.5 |
| RSRP in subframes corresponding to RSSI measurement time configuration (RMTC) |  | -103.5 | -Infinity |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 = 2 (Note 1) | dBm/1.08 MHz | -83 | -71.73 |
| Io within measurement bandwidth in subframes corresponding to RSSI measurement time configuration (RMTC) where system frame number mod 12 is not equal to 2 (Note 1) | dBm/1.08 MHz | -83 | -78.73 |
| Io within measurement bandwidth in subframes not corresponding to RSSI measurement time configuration (RMTC) | dBm/1.08 MHz | -83 | -83 |
| Propagation condition | - | AWGN |  |
| channelOccupancyThreshold | dBm | -75.2 |  |
| Note 1: Accumulated system frame number is used to avoid a configuration not matching the test purpose at the boundary of hyper frame numbers. |  |  |  |

Table A.9.12.2.2-2: Channel occupancy RMTC and DMTC parameters

| measDuration-r13 | sym14 |
| --- | --- |
| rmtc-Period-r13 | ms40 |
| rmtc-SubframeOffset-r13 | 20 |
| ReportInterval | ms120 |
| dmtc-PeriodOffset-r12 | ms40-r12 value 0 |

#### A.9.12.2.3	Test Requirements

The nominal reported channelOccupancy in this test is 33. At least 90% of channel occupancy reports made by the UE shall indicate this value.

## A.9.13	RS-SINR

### A.9.13.1	FDD Intra-Frequency Case

#### A.9.13.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.17.2.1.

#### A.9.13.1.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RS-SINR intra-frequency measurement is tested by using the parameters in Table A.9.13.1.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.13.1.2-1: RS-SINR FDD intra-frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  |
| Cell Ids |  |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 3 ≠ 0 |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| BWchannel |  | MHz | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | - | R.6 FDD | - |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  | OP.1 FDD | - | OP.1 FDD | - |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  | 0 |  | 0 |
| PSS_RA |  |  |  | 0 |  | 0 |
| SSS_RA |  |  |  | 0 |  | 0 |
| PCFICH_RB |  |  |  | -∞ |  | -∞ |
| PHICH_RA |  |  |  | -∞ |  | -∞ |
| PHICH_RB |  |  |  | -∞ |  | -∞ |
| PDCCH_RA |  |  |  | -∞ |  | -∞ |
| PDCCH_RB |  |  |  | -∞ |  | -∞ |
| PDSCH_RA |  |  |  | -∞ |  | -∞ |
| PDSCH_RB |  |  |  | -∞ |  | -∞ |
| OCNG_RANote1 |  |  |  | -∞ |  | -∞ |
| OCNG_RBNote1 |  |  |  | -∞ |  | -∞ |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -90 | -90 | -116 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  | -114 |  |
|  | Bands FDD_G Note 7 |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  | -112.5 |  |
| CRS |  | dB | 5 | -3.19 | -5.46 | -5.46 |
| RSRPNote3 | Bands FDD_A Note 8 | dBm/15 kHz | -85.00 | -87.00 | -120 | -120 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  | -119.5 | -119.5 |
|  | Bands FDD_C |  |  |  | -119 | -119 |
|  | Bands FDD_D |  |  |  | -118.5 | -118.5 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  | -118 | -118 |
|  | Bands FDD_G Note 7 |  |  |  | -117 | -117 |
|  | Bands FDD_H |  |  |  | -116.5 | -116.5 |
| RS-SINRNote3 | Bands FDD_A Note 8 | dB | 5 | -3.19 | -5.46 | -5.46 |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 | dBm/9 MHz | -55.69 | -55.69 | -86.56 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  | -86.06 |  |
|  | Bands FDD_C |  |  |  | -85.56 |  |
|  | Bands FDD_D |  |  |  | -85.06 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  | -84.56 |  |
|  | Bands FDD_G Note 7 |  |  |  | -83.56 |  |
|  | Bands FDD_H |  |  |  | -83.06 |  |
| CRS |  | dB | 5 | 3 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29 Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |

#### A.9.13.1.3	Test Requirements

The RS-SINR measurement accuracy for Cell 2 shall fulfil the requirements in Section 9.1.17.2.1.

### A.9.13.2	TDD Intra-Frequency Case

#### A.9.13.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.17.2.1.

#### A.9.13.2.2	Test parameters

In this test case all cells are on the same carrier frequency. The absolute accuracy of RS-SINR intra-frequency measurement is tested by using the parameters in Table A.9.13.2.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.13.2.2-1: RS-SINR TDD intra-frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  |
| Cell Ids |  |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 3 ≠ 0 |  | (Cell ID of cell 1 – Cell ID of cell 2) mod 6 = 0 |  |
| BWchannel |  | MHz | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote1 |  |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD | - | R.6 TDD | - |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | - | OP.1 TDD | - |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  | 0 |  | 0 |
| PSS_RA |  |  |  | 0 |  | 0 |
| SSS_RA |  |  |  | 0 |  | 0 |
| PCFICH_RB |  |  |  | -∞ |  | -∞ |
| PHICH_RA |  |  |  | -∞ |  | -∞ |
| PHICH_RB |  |  |  | -∞ |  | -∞ |
| PDCCH_RA |  |  |  | -∞ |  | -∞ |
| PDCCH_RB |  |  |  | -∞ |  | -∞ |
| PDSCH_RA |  |  |  | -∞ |  | -∞ |
| PDSCH_RB |  |  |  | -∞ |  | -∞ |
| OCNG_RANote2 |  |  |  | -∞ |  | -∞ |
| OCNG_RBNote2 |  |  |  | -∞ |  | -∞ |
| Note3 | Bands TDD_A | dBm/15 kHz | -90.00 | -90.00 | -116 |  |
|  | Bands TDD_C |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  | -114 |  |
|  |  | dB | 5 | -3.19 | -5.46 | -5.46 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -85 | -87 | -120 | -120 |
|  | Bands TDD_C |  |  |  | -119 | -119 |
|  | Bands TDD_E |  |  |  | -118 | -118 |
| RS-SINRNote4 | Bands TDD_A, TDD_C, TDD_E | dB | 5 | -3.19 | -5.46 | -5.46 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -55.69 | -55.69 | -86.56 |  |
|  | Bands TDD_C |  |  |  | -85.56 |  |
|  | Bands TDD_E |  |  |  | -84.56 |  |
|  |  | dB | 5 | 3 | -4 | -4 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### A.9.13.2.3	Test Requirements

The RS-SINR measurement accuracy for Cell 2 shall fulfil the requirements in Section 9.1.17.2.1.

### A.9.13.3	FDD—FDD Inter frequency case

#### A.9.13.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.17.3.

#### A.9.13.3.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RS-SINR inter-frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.13.3.2-1 and Table A.9.13.3.2-2. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.13.3.2-1: RS-SINR FDD—FDD Inter frequency test parameters (Cell 1)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | 0 | 0 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  | R.0 FDD | R.0 FDD | R.0 FDD |
| PDSCH allocation |  |  |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 |  |  |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz | -80 | -108.50 | -119.5 |  |
|  | Bands FDD_B |  |  |  |  | -119 |  |
|  | Bands FDD_C |  |  |  |  | -118.5 |  |
|  | Bands FDD_D |  |  |  |  | -118 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -117.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -116.5 |  |
|  | Bands FDD_H |  |  |  |  | -116 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz | -81.75 | -88.50 | -123.5 |  |
|  | Bands FDD_B |  |  |  |  | -123 |  |
|  | Bands FDD_C |  |  |  |  | -122.5 |  |
|  | Bands FDD_D |  |  |  |  | -122 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -121.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -120.5 |  |
|  | Bands FDD_H |  |  |  |  | -120 |  |
| RS-SINRNote3 | Bands FDD_A Note 8 |  | dB | -1.75 | 20 | -4.0 |  |
|  | Bands FDD_B |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz | -50 | -60.68 | -90.26 |  |
|  | Bands FDD_B |  |  |  |  | -89.76 |  |
|  | Bands FDD_C |  |  |  |  | -89.26 |  |
|  | Bands FDD_D |  |  |  |  | -88.76 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -88.26 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -87.26 |  |
|  | Bands FDD_H |  |  |  |  | -86.76 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32. |  |  |  |  |  |  |  |

Table A.9.13.3.2-2: RS-SINR FDD—FDD Inter frequency test parameters (Cell 2)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | - | - | - |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  | - | - | - |
| PDSCH allocation |  |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz | -85 | -108.5 | -119.5 |  |
|  | Bands FDD_B |  |  |  |  | -119 |  |
|  | Bands FDD_C |  |  |  |  | -118.5 |  |
|  | Bands FDD_D |  |  |  |  | -118 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -117.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -116.5 |  |
|  | Bands FDD_H |  |  |  |  | -116 |  |
| Note2a | Bands FDD_A Note 8 |  | dBm/15 kHz | -79 | -114.5 | -113.5 |  |
|  | Bands FDD_B |  |  |  |  | -113 |  |
|  | Bands FDD_C |  |  |  |  | -112.5 |  |
|  | Bands FDD_D |  |  |  |  | -112 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -111.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -110.5 |  |
|  | Bands FDD_H |  |  |  |  | -110 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz | -86.75 | -88.50 | -123.5 |  |
|  | Bands FDD_B |  |  |  |  | -123 |  |
|  | Bands FDD_C |  |  |  |  | -122.5 |  |
|  | Bands FDD_D |  |  |  |  | -122 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -121.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -120.5 |  |
|  | Bands FDD_H |  |  |  |  | -120 |  |
| RS-SINRNote3 |  |  | dB | -1.75 | 20 | -4.0 |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz | -50.68 | -60.71 | -85.45 |  |
|  | Bands FDD_B |  |  |  |  | -84.95 |  |
|  | Bands FDD_C |  |  |  |  | -84.45 |  |
|  | Bands FDD_D |  |  |  |  | -83.95 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -83.45 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -82.45 |  |
|  | Bands FDD_H |  |  |  |  | -81.95 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2a: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers other than CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32. |  |  |  |  |  |  |  |

#### A.9.13.3.3	Test Requirements

The RS-SINR measurement accuracy shall fulfil the requirements in Sections 9.1.17.3.

### A.9.13.4	TDD—TDD Inter frequency case

#### A.9.13.4.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.17.3.

#### A.9.13.4.2	Test parameters

In this test case the two cells are on different carrier frequencies and measurement gaps are provided. Both RS-SINR inter-frequency absolute and relative accuracy requirements are tested by using test parameters in Table A.9.13.4.2-1 and Table A.9.13.4.2-2 for TDD configuration 1 and in Table A.9.13.4.2-3 and Table A.9.13.4.2-4 for TDD configuration 0. In all tests, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.13.4.2-1: RS-SINR TDD—TDD Inter frequency test parameters for TDD configuration 1 (Cell 1)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | 0 | 0 |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  |  | R.0 TDD | R.0 TDD | R.0 TDD |
| PDSCH allocation |  |  |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 |  |  |  | OP.1 TDD | OP.1 TDD | OP.1 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -80 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 |  |  | dB | -1.75 | 20 | -4.0 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50 | -60.68 | -90.26 |  |
|  | Bands TDD_C |  |  |  |  | -89.26 |  |
|  | Bands TDD_E |  |  |  |  | -88.26 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

Table A.9.13.4.2-2: RS-SINR TDD—TDD Inter frequency test parameters for TDD configuration 1 (Cell 2)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | - | - | - |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  |  | - | - | - |
| PDSCH allocation |  |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.2 |  |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -85 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| Note3a | Bands TDD_A |  | dBm/15 kHz | -79 | -114.50 | -113.50 |  |
|  | Bands TDD_C |  |  |  |  | -112.50 |  |
|  | Bands TDD_E |  |  |  |  | -111.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 |  |  | dB | -1.75 | 20 | -4.0 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50.68 | -60.71 | -85.45 |  |
|  | Bands TDD_C |  |  |  |  | -84.45 |  |
|  | Bands TDD_E |  |  |  |  | -83.45 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for CRS to be fulfilled. Note 3a: Interference from other cells and noise sources not specified in the test is assumed to be constant over Non-CRS subcarriers and time and shall be modelled as AWGN of appropriate power for Non-CRS to be fulfilled. Note 4: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

Table A.9.13.4.2-3: RS-SINR TDD—TDD Inter frequency test parameters for TDD configuration 0 (Cell 1)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | 0 | 0 |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 0 | 0 | 0 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  |  | R.0 TDD | R.0 TDD | R.0 TDD |
| PDSCH allocation |  |  |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.1 |  |  |  | OP.1 TDD | OP.1 TDD | OP.1 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -80 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 |  |  | dB | -1.75 | 20 | -4.0 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50 | -60.68 | -90.26 |  |
|  | Bands TDD_C |  |  |  |  | -89.26 |  |
|  | Bands TDD_E |  |  |  |  | -88.26 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

Table A.9.13.4.2-4: RS-SINR TDD—TDD Inter frequency test parameters for TDD configuration 0 (Cell 2)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | - | - | - |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 0 | 0 | 0 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  |  | - | - | - |
| PDSCH allocation |  |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.2 |  |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| CRS Note3 | Bands TDD_A |  | dBm/15 kHz | -85 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| Non-CRS Note3a | Bands TDD_A |  | dBm/15 kHz | -79 | -114.50 | -113.50 |  |
|  | Bands TDD_C |  |  |  |  | -112.50 |  |
|  | Bands TDD_E |  |  |  |  | -111.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 |  |  | dB | -1.75 | 20 | -4.0 |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50.68 | -60.71 | -85.45 |  |
|  | Bands TDD_C |  |  |  |  | -84.45 |  |
|  | Bands TDD_E |  |  |  |  | -83.45 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Antenna Configuration |  |  | - | 1x2 | 1x2 | 1x2 |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3a: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers other than CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

#### A.9.13.4.3	Test Requirements

The RS-SINR measurement accuracy shall fulfil the requirements in Sections 9.1.17.3.

### A.9.13.5	FDD—TDD Inter frequency case

#### A.9.13.5.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.17.3.

#### A.9.13.5.2	Test parameters

In this set of test cases the two cells are on different carrier frequencies. Both absolute and relative accuracy of RS-SINR inter frequency measurements are tested by using the parameters in Table A.9.13.5.2-1. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell. Cell 1 is FDD cell and Cell 2 is TDD cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.13.5.2-1: RS-SINR FDD—TDD Inter frequency test parameters (FDD Cell1)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 2 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | 0 | 0 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  | R.0 FDD | R.0 FDD | R.0 FDD |
| PDSCH allocation |  |  |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz | -80 | -108.50 | -119.5 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -119 |  |
|  | Bands FDD_C |  |  |  |  | -118.5 |  |
|  | Bands FDD_D |  |  |  |  | -118 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -117.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -116.5 |  |
|  | Bands FDD_H |  |  |  |  | -116 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz | -81.75 | -88.50 | -123.5 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -123 |  |
|  | Bands FDD_C |  |  |  |  | -122.5 |  |
|  | Bands FDD_D |  |  |  |  | -122 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -121.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -120.5 |  |
|  | Bands FDD_H |  |  |  |  | -120 |  |
| RS-SINRNote3 | Bands FDD_A Note 8 |  | dB | -1.75 | 20 | -4.0 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz | -50 | -60.68 | -90.26 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -89.76 |  |
|  | Bands FDD_C |  |  |  |  | -89.26 |  |
|  | Bands FDD_D |  |  |  |  | -88.76 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -88.26 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -87.26 |  |
|  | Bands FDD_H |  |  |  |  | -86.76 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| Propagation condition |  |  | - |  | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

Table A.9.13.5.2-2: RS-SINR FDD—TDD Inter frequency test parameters (TDD cell2)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | - | - | - |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  |  | - | - | - |
| PDSCH allocation |  |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.2 (OP.2 TDD) |  |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -85 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| Note3a | Bands TDD_A |  | dBm/15 kHz | -79 | -114.50 | -113.50 |  |
|  | Bands TDD_C |  |  |  |  | -112.50 |  |
|  | Bands TDD_E |  |  |  |  | -111.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 | Bands TDD_A |  | dB | -1.75 | 20 | -4.0 |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50.79 | -60.56 | -93.48 |  |
|  | Bands TDD_C |  |  |  |  | -92.48 |  |
|  | Bands TDD_E |  |  |  |  | -91.48 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3a: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers other than CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

#### A.9.13.5.3	Test Requirements

The RS-SINR measurement accuracy shall fulfil the requirements in Sections 9.1.17.3.

### A.9.13.6	TDD—FDD Inter frequency case

#### A.9.13.6.1	Test Purpose and Environment

The purpose of this test is to verify that the RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.17.3.

#### A.9.13.6.2	Test parameters

In this set of test cases the two cells are on different carrier frequencies. Both absolute and relative accuracy of RS-SINR inter frequency measurements are tested by using the parameters in Table A.9.13.6.2-1. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell. Cell 1 is TDD cell and Cell 2 is FDD cell. The inter frequency measurements are supported by a measurement gap.

Table A.9.13.6.2-1: RS-SINR TDD—FDD Inter frequency test parameters (TDD cell1)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | - | - | - |
| Special subframe configuration Note1 |  |  |  | 6 | 6 | 6 |
| Uplink-downlink configuration Note1 |  |  |  | 1 | 1 | 1 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel |  |  |  | - | - | - |
| PDSCH allocation |  |  |  | - | - | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  |  | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.2 (OP.2 TDD) |  |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A |  | dBm/15 kHz | -80 | -108.50 | -119.50 |  |
|  | Bands TDD_C |  |  |  |  | -118.50 |  |
|  | Bands TDD_E |  |  |  |  | -117.50 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote4 | Bands TDD_A |  | dBm/15 kHz | -81.75 | -88.50 | -123.50 |  |
|  | Bands TDD_C |  |  |  |  | -122.50 |  |
|  | Bands TDD_E |  |  |  |  | -121.50 |  |
| RS-SINRNote4 | Bands TDD_A |  | dB | -1.75 | 20 | -4.0 |  |
|  | Bands TDD_C |  |  |  |  |  |  |
|  | Bands TDD_E |  |  |  |  |  |  |
| IoNote4 | Bands TDD_A |  | dBm/9 MHz | -50 | -60.68 | -90.26 |  |
|  | Bands TDD_C |  |  |  |  | -89.26 |  |
|  | Bands TDD_E |  |  |  |  | -88.26 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |

Table A.9.13.6.2-2: RS-SINR TDD—FDD Inter frequency test parameters (FDD Cell2)

| Parameter |  | Unit |  | Test 1 | Test 2 | Test 2 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 2 | 2 | 2 |
| BWchannel |  | MHz |  | 10 | 10 | 10 |
| Gap Pattern Id |  |  |  | 0 | 0 | 0 |
| Measurement bandwidth |  |  |  | 22—27 | 22—27 | 22—27 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  | R.0 FDD | R.0 FDD | R.0 FDD |
| PDSCH allocation |  |  |  | 13—36 | 13—36 | 13—36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  |  |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA |  |  | dB | 0 | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A Note 8 |  | dBm/15 kHz | -85 | -108.5 | -119.5 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -119 |  |
|  | Bands FDD_C |  |  |  |  | -118.5 |  |
|  | Bands FDD_D |  |  |  |  | -118 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -117.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -116.5 |  |
|  | Bands FDD_H |  |  |  |  | -116 |  |
| Note2a | Bands FDD_A Note 8 |  | dBm/15 kHz | -79 | -114.5 | -113.5 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -113 |  |
|  | Bands FDD_C |  |  |  |  | -112.5 |  |
|  | Bands FDD_D |  |  |  |  | -112 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -111.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -110.5 |  |
|  | Bands FDD_H |  |  |  |  | -110 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| RSRPNote3 | Bands FDD_A Note 8 |  | dBm/15 kHz | -86.75 | -88.50 | -123.5 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -123 |  |
|  | Bands FDD_C |  |  |  |  | -122.5 |  |
|  | Bands FDD_D |  |  |  |  | -122 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -121.5 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -120.5 |  |
|  | Bands FDD_H |  |  |  |  | -120 |  |
| RS-SINRNote3 | Bands FDD_A Note 8 |  | dB | -1.75 | 20 | -4.0 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A Note 8 |  | dBm/9 MHz | -50.79 | -60.56 | -93.48 |  |
|  | Bands FDD_B1, FDD_B2 Note 9 |  |  |  |  | -92.98 |  |
|  | Bands FDD_C |  |  |  |  | -92.48 |  |
|  | Bands FDD_D |  |  |  |  | -91.98 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  | -91.48 |  |
|  | Bands FDD_G Note 7 |  |  |  |  | -90.48 |  |
|  | Bands FDD_H |  |  |  |  | -89.98 |  |
| CRS |  |  | dB | -1.75 | 20 | -4.0 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2a: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers other than CRS subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RS-SINR, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RS-SINR minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: Except Band 29. Note 8: Except Band 32, Band 75, Band 76. Note 9: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

#### A.9.13.6.3	Test Requirements

The RS-SINR measurement accuracy shall fulfil the requirements in Sections 9.1.17.3.

## A.9.14	Channel quality reporting accuracy

### A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage

#### A.9.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.1.2-1 and A.9.14.1.2-2.

Table A.9.14.1.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT operational mode |  | Standalone |
| CP Length |  | Normal |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPUSCH repetition level |  | 1 |

Table A.9.14.1.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.31 HD-FDD |
| NPDCCH repetition level for RAR |  | 4 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -6 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

#### A.9.14.1.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage

#### A.9.14.2.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.2.2-1 and A.9.14.2.2-2.

Table A.9.14.2.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT operational mode |  | Standalone |
| CP Length |  | Normal |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPUSCH repetition level |  | 1 |

Table A.9.14.2.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.31 HD-FDD |
| NPDCCH repetition level for RAR |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

#### A.9.14.2.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage

#### A.9.14.3.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy on non-anchor carrier is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.3.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy on non-anchor carrier is tested by using the parameters in Tables A.9.14.3.2-1 and A.9.14.3.2-2.

Table A.9.14.3.2-1: General Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT operational mode |  | Standalone |
| CP Length |  | Normal |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPUSCH repetition level |  | 1 |
| NPDCCH carrier index (npdcch-CarrierIndex-r14) |  | 1 (Note 1) |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]) |  |  |

Table A.9.14.3.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.31 HD-FDD |
| NPDCCH repetition level for RAR on non-anchor |  | 4 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -6 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

#### A.9.14.3.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage

#### A.9.14.4.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy on non-anchor carrier is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.4.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy on non-anchor carrier is tested by using the parameters in Tables A.9.14.4.2-1 and A.9.14.4.2-2.

Table A.9.14.4.2-1: General Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT operational mode |  | Standalone |
| CP Length |  | Normal |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPUSCH repetition level |  | 1 |
| NPDCCH carrier index (npdcch-CarrierIndex-r14) |  | 1 (Note 1) |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]). |  |  |

Table A.9.14.4.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.31 HD-FDD |
| NPDCCH repetition level for RAR on non-anchor |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

#### A.9.14.4.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage

#### A.9.14.5.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.5.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The tests consist of two successive time periods of length T1 and T2, respectively, at different SNR levels. The start of T2 coincides with the start of the channel quality measurement period specified in section 8.14.4. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.5.2-1 and A.9.14.5.2-2.

Table A.9.14.5.2-1: General Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| CP Length |  | Normal |  |
| DRX |  | OFF |  |
| NPUSCH repetition level |  | 1 |  |
| T1 | s | 1 | Initialization period |
| T2 Note 1 | s | - | Evaluation period |
| Note 1: This time period starts at the beginning of the NPDCCH period that carries the uplink grant for the channel quality report (section 8.14.4). |  |  |  |

Table A.9.14.5.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | kHz | 200 |  |
| NPDCCH parameter |  | R.31 HD-FDD |  |
| NPDCCH repetition Note4 |  | 4 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 |  |
| NRS | dB | 3 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality IE Note3 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See TS 36.331 [2]. Note 4: The NPDCCH repetition level shall be adjusted during T2 based on the DL channel quality report so that the requirements in Table 9.1.22.16-1 can be verified. |  |  |  |

#### A.9.14.5.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage

#### A.9.14.6.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.6.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The tests consist of two successive time periods of length T1 and T2, respectively, at different SNR levels. The start of T2 coincides with the start of the channel quality measurement period specified in section 8.14.4. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.6.2-1 and A.9.14.6.2-2.

Table A.9.14.6.2-1: General Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| CP Length |  | Normal |  |
| DRX |  | OFF |  |
| NPUSCH repetition level |  | 1 |  |
| T1 | s | 1 | Initialization period |
| T2 Note 1 | s | - | Evaluation period |
| Note 1: This time period starts at the beginning of the NPDCCH period that carries the uplink grant for the channel quality report (section 8.14.4). |  |  |  |

Table A.9.14.6.2-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | kHz | 200 |  |
| NPDCCH parameter |  | R.31 HD-FDD |  |
| NPDCCH repetition Note4 |  | 16 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 |  |
| NRS | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality IE Note3 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See TS 36.331 [2]. Note 4: The NPDCCH repetition level shall be adjusted during T2 based on the DL channel quality report so that the requirements in Table 9.1.22.16-1 can be verified. |  |  |  |

#### A.9.14.6.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.7	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.7.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.7.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.7.2-1 and A.9.14.7.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.7.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-3. |  |  |  |

Table A.9.14.7.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.7.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.8	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.8.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.8.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.8.2-1 and A.9.14.8.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.8.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-4. |  |  |  |

Table A.9.14.8.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.2 |  | R.7 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.8.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.9	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.9.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.9.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.9.2-1 and A.9.14.9.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.9.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-3. |  |  |  |

Table A.9.14.9.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.3 |  | R.15 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.11 TDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.9.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.10	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.10.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.10.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.10.2-1 and A.9.14.10.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.10.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.10.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.10.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.11	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.11.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.11.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.11.2-1 and A.9.14.11.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.11.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-2. |  |  |  |

Table A.9.14.11.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.11.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.12	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.12.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.12.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.12.2-1 and A.9.14.12.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.12.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.12.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.12.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.13	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.13.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.13.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.13.2-1 and A.9.14.13.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.13.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-1. |  |  |  |

Table A.9.14.13.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.13.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.14	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.14.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.14.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.14.2-1 and A.9.14.14.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.14.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-2. |  |  |  |

Table A.9.14.14.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.14.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.15	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.15.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.15.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.15.2-1 and A.9.14.15.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.15.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-1. |  |  |  |

Table A.9.14.15.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.15.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.16	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.16.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.16.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.16.2-1 and A.9.14.16.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.16.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.16.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.16.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.17	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.17.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.17.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.17.2-1 and A.9.14.17.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.17.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-2. |  |  |  |

Table A.9.14.17.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.17.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.18	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.18.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.18.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.18.2-1 and A.9.14.18.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3.  The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.18.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.18.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.18.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

# A.10	Proximity-based Services in Any Cell Selection State

## A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test

### A.10.1.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for ProSe Direct Communication transmissions in Any Cell Selection state defined in clause 11.2.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A.10.1.1-1 below. There is no serving cell and one active SyncRef UE in this test. The test system shall emulate the SyncRef UE to transmit SLSS and MIB-SL every synchronization period.

The test system will configure the ProSe UE to transmit SLSS in each period (40ms) by configuring syncTxThreshOoC as +infinity in the pre-configured parameters. The ProSe UE is expected to synchronize to the SyncRef UE and transmit its own SLSS and SL-MIB in accordance to the procedure specified in clause 5.10.7.3 of TS 36.331.

The transmit timing is verified using the transmission timing of SLSS transmissions.

Table A.10.1.1-1: Test parameters for ProSe Transmission Timig Accuracy test for E-UTRAN FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Channel Bandwidth (BWchannel) |  | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| ProSe Direct Communication preconfiguration |  |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| syncTxThreshOoC |  |  | 11 (+infinity) |  |
|  |  | dBm/15 kHz | -98 |  |
| SyncRef UE 1 | syncCP-Len |  | Normal |  |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in ProSe Direct Communication preconfiguration |  |
|  | slssid |  | 30 |  |
|  | inCoverage |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  | ON |  |
|  | ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test; Note resource pool is same as Configuration #2 used by ProSe UE. |
|  |  |  | 3 |  |
|  | S-RSRP Note1, Note 2 |  | -95 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |

### A.10.1.2	Test Requirements

For parameters specified in Tables A.10.1.1-1, the timing accuracy for ProSe Direct Communication transmissions shall be within the limits defined in clause 11.2.2. The timing accuracy is verified using SLSS transmissions.

Prior to start of test, test system is required to ensure that the ProSe UE is synchornized to the SyncRef UE 1 and is transmitting SLSS + MIB-SL as derived from the SLSS + MIB-SL of SyncRef UE 1  as per clause 5.10.7.3 of TS 36.331. For the test configuration, the SLSSID used by the ProSe UE shall be 30 with inCoverage IE in MIB-SL set as FALSE.

The following sequence of events shall be used to verify that the requirements are met.

For 5MHz or 10MHz channel bandwith, the test sequence shall be carried out in Any Cell Selection state.

a) 	After the ProSe UE is synchronized to SyncRef UE 1, the test system shall verify that the ProSe UE SLSS transmission timing offset is within ± 24×TS with respect to the first detected path (in time) of the corresponding frame of SyncRef UE 1.

b) 	The test system adjusts the transmit timing of SyncRef UE 1 by +24TS compared to that in (a). The test system shall wait for at least one SLSS period (40ms) before verifying the requirement again in (c).

c) 	The test system shall verify that the UE SLSS transmissiontiming offset stays within ± 24×TS with respect to the first detected path (in time) of the corresponding frame of SyncRef UE 1.

## A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication

### A.10.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the evaluation time allowed to initiate and cease SLSS transmissions in Any Cell Selection state defined in clause 11.3.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A. X.2.1-1 and Table A.10.2.1-2 below. There are no active cells in this test. There is one active SyncRef UE (SyncRef UE 1) in this test. The test system shall emulate SyncRef UE 1 to transmit SLSS and MIB-SL every synchronization period.

Prior to start of test, test system is required to ensure that the ProSe UE is synchornized to the SyncRef UE 1 and is transmitting SLSS + MIB-SL as derived from the SLSS + MIB-SL of SyncRef UE 1  as per clause 5.10.7.3 of TS 36.331. For the test configuration, the SLSSID used by the ProSe UE shall be 30 with inCoverage IE in MIB-SL set as FALSE.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the S-RSRP of SyncRef UE 1 is above syncTxThreshOOC and the UE is not expected to be transmitting SLSS. During T2, the S-RSRP of SyncRef UE 1 is lowered below syncTxThreshOOC and the UE is expected to initiate SLSS transmissions. During T3, the S-RSRP of SyncRef UE 1 is increased back to be above syncTxThreshOOC and the UE is expected to cease SLSS transmissions.

Table A.10.2.1-1: Test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | None |  |
| Active SyncRef UE |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| ProSe Direct Communication preconfiguration |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| syncTxThreshOoC | dBm/15 kHz | -95 |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.10.2.1-2: SyncRef UE specific test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | SyncRef UE 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannelNote 4 | MHz | 5 or 10 |  |  |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) Note resource pool is same as Configuration #2 used by ProSe UE. |  |  |
| syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in ProSe Direct Communication preconfiguration |  |  |
| slssid |  | 30 |  |  |
| inCoverage |  | TRUE |  |  |
| networkControlledSyncTx |  | ON |  |  |
| Note1 | dBm/15 kHz | -96 |  |  |
|  | dB | 5.5 | -3.5 | 5.5 |
| S-RSRP Note2, Note3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |

### A.10.2.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 0.84 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 0.84 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 0.8 sec (clause 11.3.2) for the parameters in this test;

SLSS period	is set as 40ms in this test.

## A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test

### A.10.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection in Any Cell Selection state defined in clause 11.5.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A. X.3.1-1 and Table A.10.3.1-2 below. There are no active cells in this test. There are two active SyncRef UEs (SyncRef UE 1 and SyncRef UE 2) in this test. The test system shall emulate SyncRef UE 1 and SyncRef UE 2 to transmit SLSS and MIB-SL every SLSS period (40ms).

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the ProSe UE for its SLSS+MIB-SL transmissions. When the ProSe UE is not synchronized to any SyncRef UE, then the ProSe UE shall use the SLSS ID pre-configured in the ProSe UE. When the ProSe UE is synchronized to a SyncRef UE, the ProSe UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.10.7.3 of TS 36.331.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, both SyncRef UE 1 and SyncRef UE 2 are powered off and the ProSe UE is expected to transmit SLSS as an independent synchronization source. During T1, SyncRef UE 1 is powered ON and the ProSe UE will select SyncRef UE 1 as the synchronization source. During T2, a higher priority SyncRef UE 2 is additionally powered ON and the ProSe UE will reselect to the higher priority SyncRef UE 2 as the synchronization source.

Table A.10.3.1-1: Test parameters for SyncRef UE selection/reselection test for E-UTRAN FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Independent synchronization source | UE transmits for ProSe Direct Communication and SLSS+MIB-SL with some random SLSS ID and in-coverage set as FALSE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for ProSe Direct Communication and SLSS+MIB-SL with SLSS ID = 168+59 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 2 | UE transmits for ProSe Direct Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as FALSE in MIB-SL. |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Channel Bandwidth (BWchannel) |  | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  |  | None |  |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| Timing offset between SyncRef UE 1 and SyncRef UE 2 |  | ms | 3 | Asynchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| Frequency offset of SyncRef UE 2 |  | ppm | 5 |  |
| ProSe Direct Communication preconfiguration |  |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| syncTxThreshOoC |  |  | 11 (+infinity) |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 24 |  |
| T3 |  | s | 24 |  |

Table A.10.3.1-2: SyncRef UE specific test parameters for SyncRef UE selection/reselection test for E-UTRAN FDD

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |
| BWchannel Note 4 | MHz | 5 or 10 |  |  |  |  |  |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-2 (Configuration #2) |  |  | As specified in Table A.3.12.5-1 (Configuration #1) |  |  |
| networkControlledSyncTx |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | N/A |  |  |
| slssid |  | 59 |  |  | 30 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  | TRUE |  |  |
| syncOffsetIndicator |  | syncOffsetIndicator2 |  |  | syncOffsetIndicator1 |  |  |
| Note1 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | -infinity | 16 | 16 | -infinity | -infinity | 13 |
|  | dB | -infinity | 16 | 2.79 | -infinity | -infinity | -3.11 |
| S-RSRP Note2, Note 3 | dBm/15 kHz | -infinity | -82 | -82 | -infinity | -infinity | -85 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |  |  |  |

### A.10.3.2	Test Requirements

SyncRef UE selection delay is defined as the time from the beginning of T2 to the time UE is synchronized to SyncRef UE 1 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 168+59 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T2.

The SyncRef UE selection delay shall be less than 20.84sec.

SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will be changed o 30 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE reselection delay shall be less than 20.84sec.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

The test system will verify that the ProSe UE does not drop or delay more than 2% of its SLSS transmissions during the duration of T1, T2, and T3.

The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 20sec (as specified in sub-clause 11.5.2.2)

-	Tevaluate,SLSS = 0.8 (as specified in sub-clause 11.3.2)

-	SLSS period = 40ms

This gives a total of 20.84 seconds.

## A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)

### A.10.4.1	Test Purpose and Environment

The purpose of this test is to verify cell identification delay requirement for a newly detectable cell on the downink frequency associated with the pre-configured ProSe carrier frequency in Any Cell Selection state. This test will verify the requirements in clause 11.4 when the UE is transmitting for ProSe.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A. 10.4.1-1, Table A. 10.4.1-2, and Table A.10.4.1-3 below. There is one active cell (Cell 1) and active SyncRef UE (SyncRef UE 1) in this test. The test system shall emulate SyncRef UE 1 to transmit SLSS and MIB-SL every SLSS period (40ms).

The test consists of two successive time periods, with time duration of T1 and T2 respectively. During T1, the cell is powered OFF and the ProSe UE is synchronized to SyncRef UE 1. During T2, the cell is powered ON and the ProSe UE will detect the cell and attempt to camp on the cell.

Prior to start of test, test system is required to ensure that the ProSe UE is synchornized to the SyncRef UE 1 and is transmitting SLSS + MIB-SL as derived from the SLSS + MIB-SL of SyncRef UE 1  as per clause 5.10.7.3 of TS 36.331. For the test configuration, the SLSSID used by the ProSe UE shall be 30 with inCoverage IE in MIB-SL set as FALSE.

Table A.10.4.1-1: Test parameters for cell identification test on on downlink frequency associated with ProSe frequency for E-UTRAN FDD (when UE is transmitting for ProSe)

| Parameter |  |  | Unit | Value | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Sync Ref UE 1 |  |
| Final condition | Active synchronization source |  | Cell1 |  |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Channel Bandwidth (BWchannel) |  | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  |  | Cell1 |  |
| Active SyncRef UEs |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| ProSe Direct Communication preconfiguration |  |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| syncTxThreshOoC |  |  | 11 (+infinity) |  |
| T1 |  | s | 2 |  |
| T2 |  | s | 30 |  |

Table A.10.4.1-2: Cell specific test parameters for cell identification test on on downlink frequency associated with ProSe frequency for E-UTRAN FDD (when UE is transmitting for ProSe)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannelNote 4 | MHz | 5 or 10 |  |
| OCNG Patterns defined in A.3.2.1.2 Note 4 |  | 5 MHz: OP.16 FDD 10 MHz: OP.2 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote 1 |  |  |  |
| OCNG_RBNote 1 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |
|  | dB | -infinity | -3 |
| RSRP Note3 | dBm/15 kHz | -infinity | -101 |
| SCH_RP Note3 | dBm/15 kHz | -infinity | -101 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.10.4.1-3: SyncRef UE specific test parameters for cell identification test on on downlink frequency associated with ProSe frequency for E-UTRAN FDD

| Parameter | Unit | SyncRef UE 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 (Uplink) |  |
| BWchannelNote 4 | MHz | 5 or 10 |  |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) |  |
| networkControlledSyncTx |  | ON |  |
| slssid |  | 30 |  |
| inCoverage (in MIB-SL) |  | TRUE |  |
| syncOffsetIndicator |  | syncOffsetIndicator1 |  |
| Note1 | dBm/15 kHz | -98 |  |
|  | dB | 13 |  |
| S-RSRP Note2, Note3 | dBm/15 kHz | -85 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |

#### A.10.4.2	Test Requirements

The cell selection delay to a newly detectable cell on the downlink associated with the preconfigured ProSe carrier is defined as the time from the beginning of T2 to the time UE camps on the cell and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST.

The cell selection delay to a newly detectable cell on the downlink associated with the preconfigured ProSe carrier shall be less than 7.68 s.

The cell selection delay can be expressed as Tbasic_identify_OoC_ProSe Tx_ON + TSI, where

-	Tbasic_identify_OoC_ProSe Tx_ON = 6.4sec as specified in sub-clause 11.4.2.2

-	TSI = Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case

This gives a total of 7.68 sec.

# A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier

## A.11.1	V2V UE Transmission Timing Accuracy Test

### A.11.1.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2V sidelink transmissions specified in clause 12.2.

For this test, the UE is triggered by the test loop function to transmit for V2V sidelink Communication.

Table A.11.1.1-1 defines test parameters for UE transmit timing accuracy tests for V2V. There is one GNSS based synchronization source during the test. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.6.1.

The transmit timing accuracy is verified by the UE transmitting PSSCH and PSCCH.

UE is not expected to receive any configuration related to V2V sidelink communication from the serving cell.

The test parameters of pre-configuration for V2V sidelink communication is defined in Table A.3.21.2-1.

Table A.11.1.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for V2V

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 10 | Band 47 TDD |
| V2V SL Communication preconfiguration |  | Defined in Table A.3.21.2-1 |  |
| PSCCH Reference Measurement Channel |  | Defined in Table A.3.21.3-1 |  |
| PSSCH Reference Measurement Channel |  | Defined in Table A.3.21.3-2 |  |
| Propagation condition |  | AWGN |  |

### A.11.1.2	Test requirements

For parameters specified in Tables A.11.1.1-1, the timing accuracy for V2V sidelink transmission shall be within the limits defined in clause 12.2.1. The timing accuracy is verified by using PSSCH and PSCCH transmissions.

The following sequence of events shall be used to verify that the requirements are met:

-	After the UE is synchronized to the GNSS synchronization source, the test system shall verify that the UE PSSCH and PSCCH transmission timing offset is within ± 12×TS with respect to the GNSS reference time.

## A.11.2	Interruptions due to V2V sidelink communication

### A.11.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements as defined in clause 12.3, related to interruptions due to V2V sidelink communication under the following additional conditions:

-	the UE is pre-configured with parameters for enabling the UE to acquire timing synchronization

-	the UE has dedicated transmitter chain and dedicated receiver chain for the V2V operation.

This test is applicable for V2V sidelink communication capable UEs that performs independent concurrent E-UTRAN operation in an E-UTRA band and stand-alone V2V sidelink operation in Band 47. If UE supports multiple bands, the UE needs to be tested only with the band with highest frequency.

In the test, the UE under test is configured with PCell on a serving frequency in the E-UTRA band, and is pre-configured with V2V sidelink communication resources for a non-serving frequency in Band 47. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and there is no active cell on RF channel 2. There is no other UE in the test.

UE is not expected to receive any configuration related to V2V sidelink communication from the serving cell. Prior to the start of the test, UE is already synchronized to a GNSS source for V2V sidelink communication. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.6.1.

At the beginning of the test, UE is triggered by the test loop function or the upper layers to receive and transmit V2V sidelink communication. The UE is continuously scheduled with PDSCH traffic on PCell downlink in RF channel 1 for a duration of 1s. The UE is then triggered by the test loop function or the upper layers to stop receiving and transmitting V2V sidelink communication before the end of the test.

The test parameters are given in Table A.11.2.1-1, Table A.11.2.1-2, and Table A.11.2.1-3 below.

Table A.11.2.1-1: Test parameters for interruptions due to V2V sidelink communication

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 10 | On both Band 47 and E-UTRA band |
| Active cell |  | Cell 1 | Serving cell on RF channel number 1 |

Table A.11.2.1-2: Sidelink communication configuration for interruptions due to V2V

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 2 | Band 47 |
| Channel Bandwidth (BWchannel) | MHz | 10 | According to principle defined in clause A.3.21 |
| V2V sidelink communication resource pool configuration |  | As specified in Table A.3.21.2-1 | IE values unless specified otherwise in this test. (Preconfigured) |
| PSCCH Reference Measurement Channel |  | CC.1 | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1 | As specified in Table A.3.21.3-2 |

Table A.11.2.1-3: Cell specific test parameters for interruptions due to V2V sidelink communication

| Parameter | Unit | Cell1 |
| --- | --- | --- |
| RF Channel Number |  | 1 |
| Serving/Non-serving |  | Serving |
| BWchannel Note 5 | MHz | 10 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| drx-Configuration |  | None |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1, Note 5 |  | R.6 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 5 |  | R.3 FDD |
| OCNG Pattern defined in A.3.2.1 |  | OP.10 FDD |
| PCFICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
|  | dB | 16 |
|  | dBm/15 kHz | -98 |
| RSRP Note4 | dBm/15 kHz | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 |
| Propagation condition |  | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.21. |  |  |

### A.11.2.2	Test Requirements

The test system shall verify that no interruption is caused to the ACK/NACKs on the serving cell on RF channel 1 during the test.

# A.12

## A.12.1	V2X UE Transmission Timing Accuracy Test

### A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference

#### A.12.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 13.2.2, when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication.

Table A.12.1.1.1-1 and A.12.1.1.1-2 define test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active cell (PCell) in this test. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.12.1.1.1-1: V2XSidelink Test Parameters for V2X UE Transmit Timing Accuracy Test for eNB as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | Band 47 TDD |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| V2X sidelink communication configuration |  | As specified in Table A.3.22.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  | CC.1A | Defined in Table A.3.22.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1A | Defined in Table A.3.22.3-2 |
| Propagation condition |  | AWGN |  |

Table A.12.1.1.1-2: Cell Test parameters for V2X UE Transmit Timing Accuracy Test for eNB as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | FDD band |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 2 |
| CP length of Cell 1 |  | Normal |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.6 FDD |  |
| OCNG PatternNote2 |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote3 |  |  |  |
| OCNG_RBNote3 |  |  |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3 |  |
| RSRP Note4 | dBm/15 kHz | -95 |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 |  |
| Propagation condition |  | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that Cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.12.1.1.2	Test requirements

For parameters specified in Tables A.12.1.1.1-1 and A.12.1.1.1-2, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 13.2.2. The timing accuracy is verified by using PSSCH transmissions.

### A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference

#### A.12.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 13.2.3, when SyncRef UE is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

Table A.12.1.2.1-1 defines test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active SyncRef UE in this test without either serving cell and or GNSS signals. Before the test starts, the UE has been synchronized to the SyncRef UE. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.12.1.2.1-1: Test parameters for V2X UE Transmit Timing Accuracy Test for SyncRef UE as Timing Reference

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | Band 47 TDD |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| V2X sidelink communication configuration |  |  | As specified in Table A.3.22.2-2 (Configuration #1) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  |  | CC.1A | Defined in Table A.3.22.3-1 |
| PSSCH Reference Measurement Channel |  |  | CD.1A | Defined in Table A.3.22.3-2 |
|  |  | dBm/15 kHz | -98 |  |
| SyncRef UE 1 | syncCP-Len |  | Normal |  |
|  | syncOffsetIndicator |  | 3 |  |
|  | slssid |  | 30 |  |
|  | inCoverage |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  | ON |  |
|  | V2X sidelink communication resource pool configuration |  | As specified in Table A.3.22.2-2 (Configuration #2) | IE values unless specified otherwise in this test; Note resource pool is same as Configuration #1 used by V2X UE. |
|  |  |  | 3 |  |
|  | S-RSRP Note1, Note 2 |  | -95 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |

#### A.12.1.2.2	Test Requirements

For parameters specified in Tables A.12.1.2.1-1, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 13.2.3. The timing accuracy is verified by using PSSCH transmissions.

## A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication

### A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference

#### A.12.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the V2X UE meets the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 13.3.1.1, when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink Communication.

The test parameters are given in Table A.12.2.1.1-1 and Table A.12.2.1.1-2 below. There is one active cell in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS. During T2, the RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions. During T3, the RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

Table A.12.2.1.1-1: Test Parameters for Initiation/Cease of SLSS Transmissions Test for eNB as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 | Serving cell on RF channel number 1 |
| Active SyncRef UE |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on RF channel number 2 (TDD carrier in Band 47) |
| V2X sidelink Communication configuration |  | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/15 kHz | -110 | In SIB21 |
| DRX |  | OFF |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.12.2.1.1-2: Cell Test Parameters for Initiation/Cease of SLSS Transmissions Test for eNB as Timing Reference

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.2 |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -110 |  |  |
|  | dB | 4.5 | -4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -105.5 | -114.5 | -105.5 |
| SCH_RP Note 3 | dBm/15 kHz | -105.5 | -114.5 | -105.5 |
| Io Note 3 | dBm/9 MHz | -76.4 | -80.9 | -76.4 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.12.2.1.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 0.56 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 0.56 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 0.4 sec (clause 13.3.1.1) for the parameters in this test;

SLSS period	is set as 160ms in this test.

### A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference

#### A.12.2.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the evaluation time allowed to initiate and cease SLSS transmissions defined in clause 13.3.1.3, when SyncRef UE is used as timing reference. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink Communication.

The test parameters are given in Table A.12.2.2.1-1 and Table A.12.2.2.1-2 below. There are neither active cells and nor GNSS signals in this test. There is one active SyncRef UE (SyncRef UE 1) in this test. The test system shall emulate SyncRef UE 1 to transmit SLSS and MIB-SL every synchronization period.

Prior to start of test, test system is required to ensure that the V2X UE is synchornized to the SyncRef UE 1 and is transmitting SLSS + MIB-SL as derived from the SLSS + MIB-SL of SyncRef UE 1 as per clause 5.10.7.3 of TS 36.331. For the test configuration, the SLSSID used by the V2X UE shall be 30 with inCoverage IE in MIB-SL set as FALSE. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the S-RSRP of SyncRef UE 1 is above syncTxThreshOOC and the UE is not expected to be transmitting SLSS. During T2, the S-RSRP of SyncRef UE 1 is lowered below syncTxThreshOOC and the UE is expected to initiate SLSS transmissions. During T3, the S-RSRP of SyncRef UE 1 is increased back to be above syncTxThreshOOC and the UE is expected to cease SLSS transmissions.

Table A.12.2.2.1-1: Test Parameters for Initiation/Cease of SLSS Transmissions Test for SyncRef UE as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | TDD carrier in Band 47 is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | None |  |
| Active SyncRef UE |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on RF channel number 1 |
| V2X sidelink Communication preconfiguration |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| syncTxThreshOoC | dBm/15 kHz | -100 |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.12.2.2.1-2: SyncRef UE Specific Test Parameters for Initiation/Cease of SLSS Transmissions Test for SyncRef UE as Timing Reference

| Parameter | Unit | SyncRef UE 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| V2X sidelink Communication resource pool configuration |  | As specified in Table A.3.24.2-1 (Configuration #1) Note resource pool is same as Configuration #1 used by V2X UE. |  |  |
| syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in V2X sidelink Communication preconfiguration |  |  |
| slssid |  | 30 |  |  |
| inCoverage |  | TRUE |  |  |
| networkControlledSyncTx |  | ON |  |  |
| Note1 | dBm/15 kHz | -101 |  |  |
| PSBCH | dB | 5.5 | -3.5 | 5.5 |
| S-RSRP Note2, Note3 | dBm/15 kHz | -95.5 | -104.5 | -95.5 |
| Io | dBm/9MHz | -75.85 | -80.82 | -75.85 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Noc and PSSS Es/Noc are set the same as PSBCH Es/Noc. |  |  |  |  |

#### A.12.2.2.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 0.8 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 0.8 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 0.64 sec (clause 13.3.1.3) for the parameters in this test;

SLSS period	is set as 160ms in this test.

## A.12.3	V2X Synchronization Reference Selection/Reselection Tests

### A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

#### A.12.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 13.4, when GNSS is configured as the highest priority. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.12.3.1.1-1and A.12.3.1.1-2 below. There are no GNSS signals in this test. There are one active cell (PCell) and two active SyncRef UEs (SyncRef UE 1 and SyncRef UE 2) in this test. The test system shall emulate SyncRef UE 1 and SyncRef UE 2 to transmit SLSS and MIB-SL every SLSS period.

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its SLSS+MIB-SL transmissions. When the V2X UE is not synchronized to any SyncRef UE, then the V2X UE shall use the SLSS ID pre-configured in the V2X UE. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.10.7.3 of TS 36.331.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, both SyncRef UE 1 and SyncRef UE 2 are powered off and the V2X UE will select PCell as synchronization source. During T2, SyncRef UE 1 is powered ON and the V2X UE will select SyncRef UE 1 as the synchronization source. During T3, a higher priority SyncRef UE 2 is additionally powered ON and the V2X UE will reselect to the higher priority SyncRef UE 2 as the synchronization source.

Table A.12.3.1.1-1: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Cell 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as TRUE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 168 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 2 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 0 and in-coverage set as FALSE in MIB-SL. |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 | Transmitting SLSS+MIB-SL on RF channel number 1 (TDD carrier in Band 47) |
| Active cell |  |  | Cell 1 | E-UTRA FDD Cell 1 on RF channel number 2 |
| Timing offset between SyncRef UE 1 and SyncRef UE 2 |  | s | 3 | Synchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| Frequency offset of SyncRef UE 2 |  | ppm | 0 |  |
| V2X sidelink Communication configuration |  |  | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| typeTxSync |  |  | gnss |  |
| slssid |  |  | 30 |  |
| syncTxThreshIC |  |  | +infinity |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 3.2 |  |

Table A.12.3.1.1-2: SyncRef UE Specific Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |
| BWchannel Note 4 | MHz | 5 or 10 |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.24.2-1 (Configuration #1) |  |  | As specified in Table A.3.24.2-2 (Configuration #2) |  |  |
| networkControlledSyncTx |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | N/A |  |  |
| slssid |  | 0 |  |  | 0 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  | TRUE |  |  |
| syncOffsetIndicator |  | syncOffsetIndicator2 |  |  | syncOffsetIndicator1 |  |  |
| Note1 | dBm/15 kHz | -95 |  |  |  |  |  |
|  | dB | -infinity | 0 | 0 | -infinity | -infinity | 3 |
|  | dB | -infinity | 0 | 0 | -infinity | -infinity | 3 |
| S-RSRP Note2, Note 3 | dBm/15 kHz | -infinity | -95 | -95 | -infinity | -infinity | -92 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |  |  |  |

Table A.12.3.1.1-3: Cell Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.2 |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | 4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -90.5 | -90.5 | -90.5 |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -90.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.12.3.1.2	Test Requirements

1) During T2, SyncRef UE selection delay is defined as the time from the beginning of T2 to the time UE is synchronized to SyncRef UE 1 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 168 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T2.

The SyncRef UE selection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 13.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 13.3.1.3)

-	SLSS period = 160ms

This gives a total of 8.8seconds.

2) During T3, SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will still be 0 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE reselection delay shall be less than 2.4sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 1.6sec (as specified in sub-clause 13.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 13.3.1.3)

-	SLSS period = 160ms

This gives a total of 2.4seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and SLSS transmissions during the duration of T2, and does not drop or delay more than 30% of its SLSS transmissions during the duration of T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

### A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority

#### A.12.3.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 13.4, when eNB is configured as the highest priority. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.12.3.2.1-1and A.12.3.2.1-2 below. There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.6.1. There are two active SyncRef UEs (SyncRef UE 1 and SyncRef UE 2) in this test. The test system shall emulate SyncRef UE 1 and SyncRef UE 2 to transmit SLSS and MIB-SL every SLSS period.

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its SLSS+MIB-SL transmissions. When the V2X UE is not synchronized to any SyncRef UE, then the V2X UE shall use the SLSS ID pre-configured in the V2X UE. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.10.7.3 of TS 36.331.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, both SyncRef UE 1 and SyncRef UE 2 are powered off and the V2X UE will select GNSS as synchronization source. During T2, SyncRef UE 1 is powered ON and the V2X UE will select SyncRef UE 1 as the synchronization source. During T3, a higher priority SyncRef UE 2 is additionally powered ON and the V2X UE will reselect to the higher priority SyncRef UE 2 as the synchronization source.

Table A.12.3.2.1-1: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | GNSS | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 0 and in-coverage set as TRUE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 168+59 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 2 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as FALSE in MIB-SL. |
| Active cell |  |  | None |  |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 | Transmitting SLSS+MIB-SL on RF channel number 1 |
| Timing offset between SyncRef UE 1 and SyncRef UE 2 |  | ms | 3 | Asynchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| Frequency offset of SyncRef UE 2 |  | ppm | 5 |  |
| V2X sidelink Communication preconfiguration |  |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| syncPriority |  |  | enb |  |
| syncTxThreshOoC |  |  | 11 (+infinity) |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 16 |  |

Table A.12.3.2.1-2: SyncRef UE Specific Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 5 or 10 |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.24.2-1 (Configuration #1) |  |  | As specified in Table A.3.24.2-2 (Configuration #2) |  |  |
| networkControlledSyncTx |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | N/A |  |  |
| slssid |  | 59 |  |  | 30 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  | TRUE |  |  |
| syncOffsetIndicator |  | syncOffsetIndicator2 |  |  | syncOffsetIndicator1 |  |  |
| Note1 | dBm/15 kHz | -95 |  |  |  |  |  |
|  | dB | -infinity | 0 | 0 | -infinity | -infinity | 3 |
|  | dB | -infinity | 0 | 0 | -infinity | -infinity | 3 |
| S-RSRP Note2, Note 3 | dBm/15 kHz | -infinity | -95 | -95 | -infinity | -infinity | -92 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |  |  |  |

#### A.12.3.1.2	Test Requirements

1) During T2, SyncRef UE selection delay is defined as the time from the beginning of T2 to the time UE is synchronized to SyncRef UE 1 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 168+59 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T2.

The SyncRef UE selection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 11.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 13.3.1.3)

-	SLSS period = 160ms

This gives a total of 8.8 seconds.

2) During T3, SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will be changed o 30 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE reselection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 11.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 13.3.1.3)

-	SLSS period = 160ms

This gives a total of 8.8 seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and SLSS transmissions during the duration of T2 and T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

## A.12.4	Congestion Control Measurement Test for V2X UE

### A.12.4.1	Test Purpose and Environment

The purpose of this test is to verify that the V2X UE makes correct reporting of an event. This test will verify the congestion control measurement requirements in section 13.6.

The test parameters are given in Table A.12.4.1-1 and A.12.4.1-2 below. In the measurement control information it is indicated to the V2X UE that event-triggered reporting with Event V1 is used. There are 4 active sidelink UEs in this test. The test system shall emulate the active sidelink UE to transmit PSCCH/PSSCH every 100ms. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During T1, all of active sidelink UEs are powered off. During T2, all of active sidelink UEs are powered on and transmit PSCCH/PSSCH every 100ms.

Table A.12.4.1-1: General test parameters for Congestion Control Measurement Test for V2X UE

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| V2X sidelink communication configuration |  |  | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| sl-Subframe-r14 included in SL-configV2X-TxPoolList |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
| numSubchannel-r14 included in SL-configV2X-TxPoolList |  |  | 1 | ENUMERATED {n1} |
| threshS-RSSI-CBR |  |  | 21 | Corresponding -70dBm as defined in Section 6.3.8 in TS36.331 |
| Number of Active Sidelink UEs every 100ms |  |  | 4 | Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Active Sidelink UEs | V2X sidelink Communication configuration |  | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
|  | sl-Subframe-r14 included in SL-configV2X-TxPoolList |  | 10000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000 00000000000000000000 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
|  | numSubchannel-r14 included in SL-configV2X-TxPoolList |  | 1 | ENUMERATED {n1} |
|  | sl-OffsetIndicator-r14 |  | i | For Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Timing offset between V2X UE and Active Sidelink UEs |  | s | 3 | Synchronous |
| v1-Threshold |  |  | 2 | Corresponding 0.02 as defined in Section 6.3.8 in TS36.331 |
| Hysteresis |  |  | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.12.4.1-2: Active sidelink UE specific test parameters for Congestion Control Measurement Test for V2X UE

| Parameter | Unit | Active Sidelink UE i (i = 0, 1, 2, 3) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PSCCH RMC (defined in A.3.24.3) |  | CC.1A TDD |  |
| PSSCH RMC (defined in A.3.24.3) |  | CD.1A TDD |  |
| Note1 | dBm/15 KHz | -106 |  |
| PSCCH | dB | 7.19 | 13.19 |
| PSSCH | dB | 4.19 | 10.19 |
| PSSCH-RSRP Note 2 | dBm/15 KHz | -101.81 | -95.81 |
| S-RSSI1 Note 2 Note3 | dBm/9 MHz | -72.5 | -67.5 |
| S-RSSI2 Note 2 Note4 | dBm/9 MHz | -78.22 | -78.22 |
| Io1 Note 2 Note4 | dBm/9 MHz | -72.5 | -67.5 |
| Io2 Note 2 Note6 | dBm/9 MHz | -78.22 | -78.22 |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSSCH Es/Noc, PSSCH-RSRP, S-RSSI1, S-RSSI2, Io1 and Io2 levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-RSSI1 is the S-RSSI level measured on the subframe# 0 - 3 with “SFN mod 10 = 0”. Note 4: S-RSSI2 is the S-RSSI level measured on the subframe# 4-9 with “SFN mod 10 = 0” and the subframe# 0-9 with “SFN mod 10 = 1,…, 9”. Note 5: Io1 is the Io level measured on the subframe# 0 - 3 with “SFN mod 10 = 0”. Note 6: Io2 is the Io level measured on the subframe# 4-9 with “SFN mod 10 = 0” and the subframe# 0-9 with “SFN mod 10 = 1,…, 9”. |  |  |  |

### A.12.4.2	Test Requirements

The UE shall not send event V1 triggered measurement reports during T1 and shall send event V1 triggered measurement reports during T2.

The rate of correct events observed during repeated tests shall be at least 98%.

## A.12.5	Interruptions due to V2X Sidelink Communication

### A.12.5.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to V2X sidelink communication defined in clause 13.7.1 under the following additional conditions:

-	The UE is out of coverage on the V2X sidelink carrier and is associated with a serving cell on a non-V2X sidelink carrier

This test is applicable for V2X sidelink communication capable UEs that support concurrent inter-band E-UTRAN and V2X sidelink operation.

For this test, the UE is triggered by the test loop function or the upper layers to monitor V2X sidelink communication.

The test parameters are given in Table A.12.5.1-1, Table A.12.5.1-2, and Table A.12.5.1-3. The test consists of one active cell (PCell) on the serving RF channel 1, and there are no active cells on RF channel 2. On RF channel 2, the test consists of 8 active Sidelink UEs in this test transmitting V2X sidelink communication.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the V2X sidelink communication transmission from other active Sidelink UEs on the V2X sidelink communication resoruces.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating v2x-CommRxInterestedFreqList during T2. On reception of SidelinkUEInformation, the test system shall send RRC reconfiguration message to the UE and wait for the UE to repond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to 2 second, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during V2X sidelink communication (no missed ACK/NACKs are allowed).

Table A.12.5.1-1: Test Parameters for Interruptions due to V2X Sidelink Communication

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number | - | 1, 2 | RF channel 1 is non-V2X sidelink carrier RF channel 2 is V2X sidelink carrier |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell | - | Cell 1 | PCell on RF channel number 1 |
| CP length of Cell 1 | - | Normal |  |
| T1 | s | 5.12 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to 2 second if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 10 |  |

Table A.12.5.1-2: Slidelink Communication Configuration for Interruptions due to V2X Sidelink Communication

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | - | 2 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| V2X sidelink Communication configuration |  | - | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| Number of Active Sidelink UEs per sc-period |  | - | 8 | Sidelink UE i = 0, .., 7 |
| Active Sidelink UEs | V2X sidelink Communication configuration | - | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
|  | PSCCH Reference Measurement Channel | - | CC.1A TDD | As specified in Table A.3.24.3.1-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A TDD | As specified in Table A.3.24.3.1-2 |
|  | numSubchannel-r14 included in v2x-CommTxPoolNormal-r14 | - | 1 | ENUMERATED {n1} |
|  | startRB-Subchannel-r14 included in v2x-CommTxPoolNormal-r14 | - | i | For Sidelink UE i = 0, .., 7 |
|  | RSRP | dBm/15kHz | -98 |  |

Table A.12.5.1-3: Cell specific test parameters for interruptions due to V2X slidelink communication

| Parameter |  | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | - | 1 |  |  |  |
| BWchannel |  | MHz | 10 |  |  |  |
| UE RRC state |  | - | IDLE | CONNECTED |  |  |
| Paging configuration | defaultPagingCycle | - | rf256 | N/A |  |  |
|  | nB | - | T/32 |  |  |  |
| DRX |  | - | N/A | OFF |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 4 |  | - | N/A | None | R.3 FDD |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  | - | R.6 FDD |  |  |  |
| OCNG Pattern |  | - | OP.6 FDD |  |  | OP.10 FDD |
| PBCH_RA |  | dB | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |  |
|  |  | dB | 16 |  |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |  |
| Propagation Condition |  | - | AWGN |  |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 1x2 Low |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

### A.12.5.2	Test Requirements

The UE shall be continuously scheduled on PCell on RF channel 1 during T3. During T3, 100% of all expected ACK/NACKs shall be transmitted by the V2X UE.

## A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test

### A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements

#### A.12.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource selection / reselection for V2X UE in mode 4 defined in clause 13.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.12.6.1.1-1and A.12.6.1.1-2 below. There are 20 active V2X sidelink UEs in this test. Both the UE under test and active V2X sidelink UEs select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink UEs. The test parameters for GNSS signals are defined in B.6.1. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH every 20ms. At the beginning of whole test, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 36.321, in order to make sure that the UE under test needs continuously transmit PSCCH/PSSCH.

The test consists of two duration T1 and T2. During T1, the signal from Test Equipement are configured such that the measured PSSCH-RSRP is above the measurement threshold, and the resource occupied by the active V2X sidelink UEs is expected to be excluded in the resource selection procedure. During T2, the signal from Test Equipement are configured such that the measured PSSCH-RSRP is below the measurement threshold, and the resource occupied by the active V2X sidelink UEs is expected to included in the resource selection procedure.

Table A.12.6.1.1-1: Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList |  |  | 11111111111111111111 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
| numSubchannel-r14 included in SL-PreconfigV2X-TxPoolList |  |  | 5 | Indicates the number of sub-channels for TX resource pool |
| minSubChannel-NumberPSSCH-r14 included in v2x-ResourceSelectionConfig-r14 |  |  | 1 | Indicates the minimum number of sub-channels which may be used for transmissions on PSSCH |
| maxSubchannel-NumberPSSCH-r14 included in v2x-ResourceSelectionConfig-r14 |  |  | 1 | Indicates the maximum number of sub-channels which may be used for transmissions on PSSCH |
| Number of Active Sidelink UEs |  |  | 20 | Active Sidelink UE i = 0, .., 19 |
| SL-ThresPSSCH-RSRP |  |  | 12 | Corresponding -106 dBm as defined in Section 6.3.8 in TS36.331 |
| Active Sidelink UEs | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
|  | sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList |  | 10000000000000000000 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
|  | numSubchannel-r14 included in SL-PreconfigV2X-TxPoolList |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | startRB-Subchannel-r14 included in SL-PreconfigV2X-TxPoolList |  | 5 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  | startRB-PSCCH-Pool-r14 included in SL-PreconfigV2X-TxPoolList |  | 5 | Indicates the lowest RB index of the PSCCH pool. |
|  | sl-OffsetIndicator-r14 |  | i mod 20 | For Active Sidelink UE i, where i = 0, .., 19 |
| Timing offset among Active Sidelink UEs |  | s | 3 | Synchronous |

Table A.12.6.1.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 19) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number | - | 1 |  |
| BWchannel Note 4 | MHz | 10 |  |
| PSCCH RMC (defined in A.3.24.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.24.3) | - | CD.1B HD |  |
| OCNG pattern defined in A.3.2.4 | - | VOP.1 HD |  |
| Note1 | dBm/15 kHz | -103 | -113 |
| PSCCH | dB | 5 |  |
| PSSCH | dB | 2 |  |
| PSCCH Note2 | dB | 5 |  |
| PSSCH Note2 | dB | 2 |  |
| S-RSRP Note 2 | dB | -101 | -111 |
| S-RSSI1 Note 2 Note3 | dBm/0.9 MHz | -80.15 | -90.15 |
| S-RSSI2 Note 2 Note4 | dBm/0.9 MHz | -80.15 | -90.15 |
| S-RSSI3 Note 2 Note5 | dBm/0.9 MHz | -65.18 | -75.18 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, S-RSRP and S-RSSI levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-RSSI1 is the S-RSSI level measured on subchannel #1. Note 4: S-RSSI2 is the S-RSSI level measured on subchannel #3. Note 5: S-RSSI3 is the S-RSSI level measured on subchannel #0/2/4. |  |  |  |

#### A.12.6.1.2	Test Requirements

The test time T1 and T2 should be long enough. The rate of PSSCH transmissions on the resources on subchannel #1 or #3 shall be less than 10% during T1. The rate of PSSCH transmissions on the resources on subchannel #1 or #3 shall be more than 90% during T2.

### A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements

#### A.12.6.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource selection / reselection for V2X UE in mode 4 defined in clause 13.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.12.6.2.1-1and A.12.6.2.1-2 below. There are 20 active V2X sidelink UEs in this test. Both the UE under test and active V2X sidelink UEs select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink UEs. The test parameters for GNSS signals are defined in B.6.1. The test system shall emulate the active sidelink UE to transmit PSCCH/PSSCH every 20ms. At the beginning of whole test, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 36.321, in order to make sure that the UE under test needs continuely transmit PSCCH/PSSCH.

Table A.12.6.2.1-1: Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList |  |  | 11111111111111111111 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
| numSubchannel-r14 included in SL-PreconfigV2X-TxPoolList |  |  | 5 | Indicates the number of sub-channels for TX resource pool |
| minSubChannel-NumberPSSCH-r14 included in v2x-ResourceSelectionConfig-r14 |  |  | 1 | Indicates the minimum number of sub-channels which may be used for transmissions on PSSCH |
| maxSubchannel-NumberPSSCH-r14 included in v2x-ResourceSelectionConfig-r14 |  |  | 1 | Indicates the maximum number of sub-channels which may be used for transmissions on PSSCH |
| Number of Active Sidelink UEs |  |  | 20 | Sidelink UE i = 0, .., 19 |
| SL-ThresPSSCH-RSRP |  |  | 66 | Corresponding infinity dBm as defined in Section 6.3.8 in TS36.331 |
| Active Sidelink UEs | V2X sidelink communication preconfiguration |  | As specified in Table A.3.24.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
|  | sl-Subframe-r14 included in SL-PreconfigV2X-TxPoolList |  | 10000000000000000000 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 36.213 [23]) |
|  | numSubchannel-r14 included in SL-PreconfigV2X-TxPoolList |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | startRB-Subchannel-r14 included in SL-PreconfigV2X-TxPoolList |  | 5 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  | startRB-PSCCH-Pool-r14 included in SL-PreconfigV2X-TxPoolList |  | 5 | Indicates the lowest RB index of the PSCCH pool. |
|  | sl-OffsetIndicator-r14 |  | i mod 20 | For Sidelink UE i, where i = 0, .., 19 |
| Timing offset among Active Sidelink UEs |  | s | 3 | Synchronous |

Table A.12.6.2.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 19) |
| --- | --- | --- |
| E-UTRA RF Channel Number | - | 1 |
| BWchannel Note 4 | MHz | 10 |
| PSCCH RMC (defined in A.3.24.3) | - | CC.1A HD |
| PSSCH RMC (defined in A.3.24.3) | - | CD.1B HD |
| OCNG pattern defined in A.3.2.4 | - | VOP.2 TDD |
| Note1 | dBm/15 kHz | -105 |
| PSCCH | dB | 5 |
| PSSCH | dB | 2 |
| PSCCH Note2 | dB | 5 |
| PSSCH Note2 | dB | 2 |
| S-RSRP Note 2 | dB | -103 |
| S-RSSI1 Note 2 Note3 | dBm/0.9 MHz | -82.15 |
| S-RSSI2 Note 2 Note4 | dBm/0.9 MHz | -76.71 |
| S-RSSI3 Note 2 Note5 | dBm/0.9 MHz | -67.18 |
| Antenna Configuration | - | 1x2 |
| Propagation Condition | - | AWGN |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, S-RSRP and S-RSSI levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: S-RSSI1 is the S-RSSI level measured on subchannel #1. Note 4: S-RSSI2 is the S-RSSI level measured on subchannel #3. Note 5: S-RSSI3 is the S-RSSI level measured on subchannel #0/2/4. |  |  |

#### A.12.6.1.2	Test Requirements

The test shall be run for a long enough amount of time. The rate of PSSCH transmissions on the resources on subchannel #1 shall be more than 80%.

## A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation

### A.12.7.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 13.10, when GNSS is configured as the highest priority.

The test parameters are given in Table A.12.7.1-1and A.12.7.1-2 below. GNSS is configured as the highest priority, and there are no GNSS signals in this test. There are one active cell (PCell) and one active SyncRef UE (SyncRef UE 1) in this test. The test system shall emulate SyncRef UE 1 to transmit SLSS and MIB-SL every SLSS period. SyncRef UE1 is operating on a PC5-based V2X channel (RF channel 1), and no active SyncRef UE is operating on another PC5-based V2X channel (RF channel 2). PCell is operating on an E-UTRAN channel (RF channel 3).

For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication both on RF channel 1 and on RF channel 2. RF channel 1 and on RF channel 2 are both included in syncFreqList.

The test system can verify the selection/reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its SLSS+MIB-SL transmissions. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the selected SyncRef UE as defined in clause 5.10.7.3 of TS 36.331 [2].

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. PCell is only powered on during T1. SyncRef UE 1 is only powered on during T3.

During T1, the V2X UE will select PCell as synchronization source. During T3, the V2X UE will select SyncRef UE 1 as the synchronization source and select RF channel 1 as synchronization carrier.

Table A.12.7.1-1: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Cell 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as TRUE in MIB-SL. |
| T3 end condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 168 and in-coverage set as FALSE in MIB-SL. |
| Active SyncRef UEs |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on RF channel number 1 (TDD carrier in Band 47) |
| Active cell |  |  | Cell 1 | E-UTRA FDD Cell 1 on RF channel number 3 |
| Timing offset between SyncRef UE 1 and Cell 1 |  | s | 3 | Synchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| V2X sidelink Communication configuration |  |  | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| typeTxSync |  |  | gnss |  |
| slssid |  |  | 30 |  |
| syncTxThreshIC |  |  | +infinity |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 24 |  |

Table A.12.7.1-2: SyncRef UE 1 Test Parameters for V2X Synchronization Reference Selection/Reselection Tests

| Parameter | Unit | T1 | T2 | T3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.24.2-1 (Configuration #1) |  |  |
| networkControlledSyncTx |  | N/A |  |  |
| syncTxThreshOoC | dBm/ 15 kHz | +infinity |  |  |
| slssid |  | 0 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  |
| syncOffsetIndicator |  | syncOffsetIndicator2 |  |  |
| Note1 | dBm/ 15kHz | -95 |  |  |
|  | dB | -inf | -inf | 0 |
| Note 3 | dB | -inf | -inf | 0 |
| S-RSRP Note2, | dBm/ 15kHz | -inf | -inf | -95 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |

Table A.12.7.1-3: Cell Test Parameters for V2X Synchronization Reference Selection/Reselection Tests

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 3 |  |  |
| BWchannel | MHz | 10 |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.2 |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | -inf | -inf |
| RSRP Note3 | dBm/15 kHz | -90.5 | -inf | -inf |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -inf | -inf |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

### A.12.7.2	Test Requirements

1) During T3, SyncRef UE selection delay is defined as the time from the beginning of T3 to the time UE is synchronized to SyncRef UE 1 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 168 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE selection delay shall be less than 17.44sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 16sec (as specified in subclause 13.10)

-	Tevaluate,SLSS = 1.28sec (as specified in subclause 13.10)

-	SLSS period = 160ms

This gives a total of 17.44 seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and SLSS transmissions during the duration of T3, and does not drop or delay more than 30% of its SLSS transmissions during the duration of T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

## A.12.8	Interruptions due to V2X Carrier Aggregation

### A.12.8.1	Interruptions on a FDD PCell

#### A.12.8.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to V2X carrier aggregation defined in clause 13.7.3, under the following additional conditions:

-	The UE is out of coverage on the V2X sidelink carriers and is associated with the PCell on a non-V2X sidelink carrier

For this test, the UE is triggered by the test loop function or the upper layers to monitor and transmit V2X sidelink communication. There are three carriers, including two V2X sidelink carriers (CC2 and CC3) and one E-UTRAN carrier (CC1). Before the test starts the UE is connected to PCell on CC1. The UE uses PCell as synchronization reference for V2X sidelik communication and shall be continuously scheduled in the PCell throughout the whole test.

The test consists of two consecutive time periods, with time duration of T1 and T2, respectively.

At the beginning of T1, the UE receives an RRCConnectionReconfiguration message by which the UE is configured only to monitor and transmit V2X sidelik communication on CC2.

An RRCConnectionReconfiguration message including sl-V2X-ConfigDedicated, by which the UE is configured to monitor and transmit V2X sidelik communication on CC2 and CC3, is sent by the test equipment in subframe #n. The point in time at which the sl-V2X-ConfigDedicated message for adding CC3 for V2X sidelik communication is received at the UE antenna connector defines the start of time period T2.

During T2, the test system will count the missed ACK/NACKs during T2 to verify the allowed interruptions due to V2X CC addition/release.

The test parameters are given in Table A.12.8.1.1-1, Table A.12.8.1.1-2, and Table A.12.8.1.1-3.

Table A.12.8.1.1-1: Test Parameters for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number | - | 1, 2, 3 | RF channel 1 is non-V2X sidelink carrier RF channel 2 and 3 are V2X sidelink carrier |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell | - | Cell 1 | PCell on RF channel number 1 |
| CP length of Cell 1 | - | Normal |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.12.8.1.1-2: Slidelink Communication Configuration for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number | - | 2, 3 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| V2X sidelink Communication configuration | - | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |

Table A.12.8.1.1-3: Cell Specific Test Parameters for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number | - | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.3 FDD |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 | - | R.6 FDD |  |
| OCNG Pattern | - | OP.10 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote 1 |  |  |  |
| OCNG_RBNote 1 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |
|  | dB | 16 |  |
| RSRP Note3 | dBm/15 kHz | -82 |  |
| SCH_RP Note 3 | dBm/15 kHz | -82 |  |
| Propagation Condition | - | AWGN |  |
| Correlation Matrix and Antenna Configuration | - | 1x2 Low |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.12.8.1.2	Test Requirements

During T2, an interruption on PCell shall occur no earlier than in subframe (n+5) and no later than in subframe (n + 22), and the total number of missed ACK/NACKs is no more than 4. The ACK/NACK missing shall occur no earlier than in subframe (n+5) and no later than in subframe (n +26).

All of the above test requirements shall be fulfilled in order for the observed CC3 addition delay and PCell interruptions to be counted as correct. The rate of correct observed CC3 addition delay and PCell interruptions during repeated tests shall be at least 90%.

### A.12.8.2	Interruptions on a TDD PCell

#### A.12.8.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to V2X carrier aggregation defined in clause 13.7.3, under the following additional conditions:

-	The UE is out of coverage on the V2X sidelink carriers and is associated with the PCell on a non-V2X sidelink carrier

For this test, the UE is triggered by the test loop function or the upper layers to monitor and transmit V2X sidelink communication. There are three carriers, including two V2X sidelink carriers (CC2 and CC3) and one E-UTRAN carrier (CC1). Before the test starts the UE is connected to PCell on CC1. The UE uses PCell as synchronization reference for V2X sidelik communication and shall be continuously scheduled in the PCell throughout the whole test.

The test consists of two consecutive time periods, with time duration of T1 and T2, respectively.

At the beginning of T1, the UE receives an RRCConnectionReconfiguration message by which the UE is configured only to monitor and transmit V2X sidelik communication on CC2.

An RRCConnectionReconfiguration message including sl-V2X-ConfigDedicated, by which the UE is configured to monitor and transmit V2X sidelik communication on CC2 and CC3, is sent by the test equipment in subframe #n. The point in time at which the sl-V2X-ConfigDedicated message for adding CC3 for V2X sidelik communication is received at the UE antenna connector defines the start of time period T2.

During T2, the test system will count the missed ACK/NACKs during T2 to verify the allowed interruptions due to V2X CC addition/release.

The test parameters are given in Table A.12.8.2.1-1, Table A.12.8.2.1-2, and Table A.12.8.2.1-3.

Table A.12.8.2.1-1: Test Parameters for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number | - | 1, 2, 3 | RF channel 1 is non-V2X sidelink carrier RF channel 2 and 3 are V2X sidelink carrier |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell | - | Cell 1 | PCell on RF channel number 1 |
| CP length of Cell 1 | - | Normal |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.12.8.2.1-2: Slidelink Communication Configuration for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number | - | 2, 3 | TDD carrier in Band 47 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| V2X sidelink Communication configuration | - | As specified in Table A.3.24.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |

Table A.12.8.2.1-3: Cell Specific Test Parameters for Interruptions due to V2X Carrier Aggregation

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number | - | 1 |  |
| BWchannel | MHz | 10 |  |
| Special subframe configurationNote1 |  | 6 |  |
| Uplink/downlink configurationNote1 |  | 1 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | R.0 TDD |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 | - | R.6 TDD |  |
| OCNG Pattern | - | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote 2 |  |  |  |
| OCNG_RBNote 2 |  |  |  |
| Note3 | dBm/15 kHz | -98 |  |
|  | dB | 16 |  |
| RSRP Note4 | dBm/15 kHz | -82 |  |
| SCH_RP Note 4 | dBm/15 kHz | -82 |  |
| Propagation Condition | - | AWGN |  |
| Correlation Matrix and Antenna Configuration | - | 1x2 Low |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. Note 2: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.12.8.2.2	Test Requirements

During T2, an interruption on PCell shall occur no earlier than in subframe (n+5) and no later than in subframe (n + 22), and the total number of missed ACK/NACKs is no more than 2. The ACK/NACK missing shall occur no earlier than in subframe (n+5) and no later than in subframe (n +29).

All of the above test requirements shall be fulfilled in order for the observed CC3 addition delay and PCell interruptions to be counted as correct. The rate of correct observed CC3 addition delay and PCell interruptions during repeated tests shall be at least 90%.

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for E-UTRAN RRC_IDLE state mobility

## B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.1-1.

Table B.1.1-1: Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -124 | -124 | -4 | -4 |
|  | FDD_B1, FDD_B2 | -123.5 | -123.5 |  |  |
|  | FDD_C, TDD_C | -123 | -123 |  |  |
|  | FDD_D | -122.5 | -122.5 |  |  |
|  | FDD_E, TDD_E | -122 | -122 |  |  |
|  | FDD_F | -121.5 Note 2 | -121.5 Note 2 |  |  |
|  | FDD_G | -121 | -121 |  |  |
|  | FDD_H | -120.5 | -120.5 |  |  |
|  | FDD_N | -117.5 | -117.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Section B.4.2. NOTE 2: The condition is -122 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection

This clause defines the E-UTRAN inter-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection defined in Table B.1.1-1 also apply for inter-frequency E-UTRAN cells in this section.

## B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.1.3 is defined in Section 3.1.

The conditions for normal coverage measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.3-1 and for E-UTRAN HD-FDD are defined are defined in Table B.1.3-2.

The conditions for enhanced coverage measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.3-3 and for E-UTRAN HD-FDD are defined are defined in Table B.1.3-4.

Table B.1.3-1: E-UTRAN intra-frequency measurements for FDD and TDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -125 | -125 | -6 | -6 |
|  | FDD-M1_B | -123.5 | -123.5 |  |  |
|  | FDD-M1_D | -123.5 | -123.5 |  |  |
|  | FDD-M1_E, TDD-M1_E | -123 | -123 |  |  |
|  | FDD-M1_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD-M1_G | -122 | -122 |  |  |
|  | FDD-M1_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.3-2: E-UTRAN intra-frequency measurements for HD-FDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD-M1_A | -125 | -125 | -6 | -6 |
|  | FDD-M1_B | -123.5 | -123.5 |  |  |
|  | FDD-M1_D | -123.5 | -123.5 |  |  |
|  | FDD-M1_E | -123 | -123 |  |  |
|  | FDD-M1_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD-M1_G | -122 | -122 |  |  |
|  | FDD-M1_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.3-3: E-UTRAN intra-frequency measurements for FDD and TDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -135 | -135 | -15 | -15 |
|  | FDD-M1_B | -123.5 | -123.5 |  |  |
|  | FDD-M1_D | -133.5 | -133.5 |  |  |
|  | FDD-M1_E TDD-M1_E | -133 | -133 |  |  |
|  | FDD-M1_F | -132.5 Note 2 | -132.5 Note 2 |  |  |
|  | FDD-M1_G | -132 | -132 |  |  |
|  | FDD-M1_N | -128.5 | -128.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.3-4: E-UTRAN intra-frequency measurements for HD-FDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD-M1_A | -135 | -135 | -15 | -15 |
|  | FDD-M1_B | -123.5 | -123.5 |  |  |
|  | FDD-M1_D | -133.5 | -133.5 |  |  |
|  | FDD-M1_E | -133 | -133 |  |  |
|  | FDD-M1_F | -132.5 Note 2 | -132.5 Note 2 |  |  |
|  | FDD-M1_G | -132 | -132 |  |  |
|  | FDD-M1_N | -128.5 | -128.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1

This clause defines the NB-IoT intra-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.1.4 is defined in Section 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection are defined in Table B.1.4-1 and B.1.4-3.

The conditions for measurements of intra-frequency NB-IoT cells in enhanced coverage for cell re-selection are defined in Table B.1.4-2 and B.1.4-4.

Table B.1.4-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_G | -129 | -129 | -6 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.4-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_G | -138 | -138 | -15 | -15 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.4-3: NB-IoT intra-frequency measurements for TDD in normal coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NTDD_G | -129 | -129 | -6 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.4-4: NB-IoT intra-frequency measurements for TDD in enhanced coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NTDD_G | -138 | -138 | -15 | -15 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1

This clause defines the NB-IoT inter-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.1.5 is defined in Section 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection defined in Table B.1.4-1 and B.1.4-3 also apply for inter-frequency NB-IoT cells in normal coverage in this section.

The conditions for measurements of intra-frequency NB-IoT cells in enhanced coverage for cell re-selection defined in Table B.1.4-2 and B.1.4-4 also apply for inter-frequency NB-IoT cells in enhanced coverage in this section.

## B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.6-1.

Table B.1.6-1: Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -124 | -124 | -5 | -5 |
|  | FDD_B1 | -123.5 | -123.5 |  |  |
|  | FDD_C, TDD_C | -123 | -123 |  |  |
|  | FDD_D | -122.5 | -122.5 |  |  |
|  | FDD_E, TDD_E | -122 | -122 |  |  |
|  | FDD_F | -121.5 Note 2 | -121.5 Note 2 |  |  |
|  | FDD_G | -121 | -121 |  |  |
|  | FDD_H | -120.5 | -120.5 |  |  |
|  | FDD_N | -117.5 | -117.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Section B.4.2. NOTE 2: The condition is -122 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2

### B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M2 applicability of the conditions in Appendix B.1.7 is defined in Section 3.1.

The conditions for CE mode A measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.7.1-1 and for E-UTRAN HD-FDD are defined are defined in Table B.1.7.1-2.

The conditions for CE mode B measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.7.1-3 and for E-UTRAN HD-FDD are defined are defined in Table B.1.7.1-4.

Table B.1.7.1-1: E-UTRAN intra-frequency measurements for FDD and TDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A, TDD_M2_A | -125 | -125 | -4 | -4 |
|  | FDD_M2_D | -123.5 | -123.5 |  |  |
|  | FDD_M2_E, TDD_M2_E | -123 | -123 |  |  |
|  | FDD_M2_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_M2_G | -122 | -122 |  |  |
|  | FDD_M2_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.1-2: E-UTRAN intra-frequency measurements for HD-FDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A | -125 | -125 | -4 | -4 |
|  | FDD_M2_D | -123.5 | -123.5 |  |  |
|  | FDD_M2_E | -123 | -123 |  |  |
|  | FDD_M2_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_M2_G | -122 | -122 |  |  |
|  | FDD_M2_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.1-3: E-UTRAN intra-frequency measurements for FDD and TDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A, TDD_M2_A | -136 | -136 | -15 | -15 |
|  | FDD_M2_D | -134.5 | -134.5 |  |  |
|  | FDD_M2_E TDD_M2_E | -134 | -134 |  |  |
|  | FDD_M2_F | -133.5 Note 1 | -133.5 Note 1 |  |  |
|  | FDD_M2_G | -133 | -133 |  |  |
|  | FDD_M2_N | -129.5 | -129.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.1-4: E-UTRAN intra-frequency measurements for HD-FDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A | -136 | -136 | -15 | -15 |
|  | FDD_M2_D | -134.5 | -134.5 |  |  |
|  | FDD_M2_E | -134 | -134 |  |  |
|  | FDD_M2_F | -133.5 Note 1 | -133.5 Note 1 |  |  |
|  | FDD_M2_G | -133 | -133 |  |  |
|  | FDD_M2_N | -129.5 | -129.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

### B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection

This clause defines the E-UTRAN inter-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M2 applicability of the conditions in Appendix B.1.7 is defined in Section 3.1.

The conditions for CE mode A measurements of FDD and TDD inter-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.7.2-1 and for E-UTRAN HD-FDD are defined are defined in Table B.1.7.2-2.

The conditions for CE mode B measurements of FDD and TDD inter-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.7.2-3 and for E-UTRAN HD-FDD are defined are defined in Table B.1.7.2-4.

Table B.1.7.2-1: E-UTRAN inter-frequency measurements for FDD and TDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A, TDD_M2_A | -125 | -125 | -4 | -4 |
|  | FDD_M2_D | -123.5 | -123.5 |  |  |
|  | FDD_M2_E, TDD_M2_E | -123 | -123 |  |  |
|  | FDD_M2_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_M2_G | -122 | -122 |  |  |
|  | FDD_M2_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.2-2: E-UTRAN inter-frequency measurements for HD-FDD for normal coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A | -125 | -124 | -4 | -4 |
|  | FDD_M2_D | -123.5 | -122.5 |  |  |
|  | FDD_M2_E | -123 | -122 |  |  |
|  | FDD_M2_F | -122.5 Note 2 | -121.5 Note 2 |  |  |
|  | FDD_M2_G | -122 | -121 |  |  |
|  | FDD_M2_N | -118.5 | -117.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.2-3: E-UTRAN inter-frequency measurements for FDD and TDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A, TDD_M2_A | -136 | -136 | -15 | -15 |
|  | FDD_M2_D | -134.5 | -134.5 |  |  |
|  | FDD_M2_E TDD_M2_E | -134 | -134 |  |  |
|  | FDD_M2_F | -133.5 Note 1 | -133.5 Note 1 |  |  |
|  | FDD_M2_G | -133 | -133 |  |  |
|  | FDD_M2_N | -129.5 | -129.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.1.7.2-4: E-UTRAN inter-frequency measurements for HD-FDD for enhanced coverage

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_M2_A | -136 | -136 | -15 | -15 |
|  | FDD_M2_D | -134.5 | -134.5 |  |  |
|  | FDD_M2_E | -134 | -134 |  |  |
|  | FDD_M2_F | -133.5 Note 1 | -133.5 Note 1 |  |  |
|  | FDD_M2_G | -133 | -133 |  |  |
|  | FDD_M2_N | -129.5 | -129.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1

This clause defines the E-UTRAN inter-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.1.3 is defined in Section 3.1.

The conditions for normal coverage measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection defined in Table B.1.3-1 and for E-UTRAN HD-FDD defined in Table B.1.3-2 also apply for E-UTRAN FDD, TDD and HD-FDD inter-frequency E-UTRAN cells for cell reselection.

The conditions for enhanced coverage measurements of FDD and TDD intra-frequency E-UTRAN cells for cell re-selection defined in Table B.1.3-3 and for E-UTRAN HD-FDD defined in Table B.1.3-4 also apply for E-UTRAN FDD, TDD, and HD-FDD inter-frequency E-UTRAN cells for re-selection.

# B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State

## B.2.1	Conditions for E-UTRAN intra-frequency measurements

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements are defined in Table B.2.1-1.

Table B.2.1-1: E-UTRAN intra-frequency measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -6 |
|  | FDD_B1, FDD_B2 | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements with autonomous gap are as in Table B.2.1-1.

Table B.2.2-1: Void

## B.2.3	Conditions for E-UTRAN inter-frequency measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, RSRP and RSRP Ês/Iot applicable for a corresponding operating band.

The conditions for inter-frequency E-UTRAN measurements with autonomous gap are defined in Table B.2.3-1.

Table B.2.3-1: E-UTRAN inter-frequency measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -125 | -125 | -4 | -4 |
|  | FDD_B1, FDD_B2 | -124.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -124 | -124 |  |  |
|  | FDD_D | -123.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -123 | -123 |  |  |
|  | FDD_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -122 | -122 |  |  |
|  | FDD_H | -121.5 | -121.5 |  |  |
|  | FDD_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps

This clause defines the E-UTRAN inter-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for inter-frequency E-UTRAN measurements with autonomous gap are defined in Table B.2.4-1.

Table B.2.4-1: E-UTRAN inter-frequency measurements with autonomous gaps

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -125 | -4 |
|  | FDD_B1, FDD_B2 | -124.5 |  |
|  | FDD_C, TDD_C | -124 |  |
|  | FDD_D | -123.5 |  |
|  | FDD_E, TDD_E | -123 |  |
|  | FDD_F | -122.5 Note 2 |  |
|  | FDD_G | -122 |  |
|  | FDD_H | -121.5 |  |
|  | FDD_N | -118.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements

This clause defines the E-UTRAN intra-frequency PRP1,2 applicable for a corresponding operating band

The conditions for E-UTRAN OTDOA intra-frequency RSTD measurements  are defined in Table B.2.5-1

Table B.2.5-1: E-UTRAN OTDOA intra-frequency RSTD measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B1, FDD_B2 | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements

This clause defines the E-UTRAN inter-frequency PRP1,2 applicable for a corresponding operating band.

The conditions for E-UTRAN OTDOA inter-frequency RSTD measurements  are defined in Table B.2.5-1.

## B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell

This clause defines the SCH_RP and SCH Ês/Iot for measurements in the secondary component carrier applicable for a corresponding operating band.

The conditions for measurements of the secondary component carrier with deactivated SCell are defined in Table B.2.7-1.

Table B.2.7-1: Measurements of the secondary component carrier with deactivated SCell

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -6 |
|  | FDD_B1, FDD_B2 | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements under time domain measurement resource restriction are defined in Table B.2.8-1.

Table B.2.8-1: E-UTRAN intra-frequency measurements under time domain measurement resource restriction

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -7.5 |
|  | FDD_B1, FDD_B2 | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency E-UTRAN measurements under time domain measurement resource restriction with CRS assistance information are defined in Table B.2.9-1.

Table B.2.9-1: E-UTRAN intra-frequency measurements under time domain measurement resource restriction with CRS assistance information

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD_A, TDD_A | -127 | -11.07 |
|  | FDD_B1, FDD_B2 | -126.5 |  |
|  | FDD_C, TDD_C | -126 |  |
|  | FDD_D | -125.5 |  |
|  | FDD_E, TDD_E | -125 |  |
|  | FDD_F | -124.5 Note 2 |  |
|  | FDD_G | -124 |  |
|  | FDD_H | -123.5 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements

### B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements

This clause defines the E-UTRAN intra-frequency SCH_RP, SCH Ês/Iot in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements

The conditions for E-UTRAN intra-frequency CRS based discovery signal measurements are as in Table B.2.1-1.

### B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements

This clause defines the E-UTRAN intra-frequency SCH_RP, SCH Ês/Iot, CSI-RSRP, and CSI-RS Ês/Iot in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for E-UTRAN intra-frequency CRI-RS based discovery signal measurements in discovery signal occasions are specified in Table B.2.10.2-1.

Table B.2.10.2-1: E-UTRAN intra-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP Note 1 | Minimum SCH_RP Note 1 | CSI-RS Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -121 | -127 | 0 | -6 |
|  | FDD_B1, FDD_B2 | -120.5 | -126.5 |  |  |
|  | FDD_C, TDD_C | -120 | -126 |  |  |
|  | FDD_D | -119.5 | -125.5 |  |  |
|  | FDD_E, TDD_E | -119 | -125 |  |  |
|  | FDD_F | -118.5 Note 2 | -124.5 Note 2 |  |  |
|  | FDD_G | -118 | -124 |  |  |
|  | FDD_H | -117.5 | -123.5 |  |  |
|  | FDD_N | -114.5 | -120.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements

### B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, RSRP, and Ês/Iot in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for E-UTRAN inter-frequency CRS-based discovery signal measurements in discovery signal occasions are specified in Table B.2.11.1-1.

Table B.2.11.1-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -125 | -125 | -6 | -6 |
|  | FDD_B1, FDD_B2 | -124.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -124 | -124 |  |  |
|  | FDD_D | -123.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -123 | -123 |  |  |
|  | FDD_F | -122.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -122 | -122 |  |  |
|  | FDD_H | -121.5 | -121.5 |  |  |
|  | FDD_N | -118.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

### B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements

This clause defines the E-UTRAN inter-frequency SCH_RP, SCH Ês/Iot, CSI-RSRP, and CSI-RS Ês/Iot in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for E-UTRAN inter-frequency CRS-based discovery signal measurements in discovery signal occasions are specified in Table B.2.11.2-1.

Table B.2.11.2-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP Note 1 | Minimum SCH_RP Note 1 | CSI-RS Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -119 | -125 | 0 | -6 |
|  | FDD_B1, FDD_B2 | -118.5 | -124.5 |  |  |
|  | FDD_C, TDD_C | -118 | -124 |  |  |
|  | FDD_D | -117.5 | -123.5 |  |  |
|  | FDD_E, TDD_E | -117 | -123 |  |  |
|  | FDD_F | -116.5 Note 2 | -122.5 Note 2 |  |  |
|  | FDD_G | -116 | -122 |  |  |
|  | FDD_H | -115.5 | -121.5 |  |  |
|  | FDD_N | -112.5 | -118.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3

This section defines the E-UTRAN intra-frequency SCH_RP in discovery signal occasions [16], applicable for a corresponding operating band for discovery signal measurements under frame structure type 3.

The conditions for E-UTRAN intra-frequency discovery signal measurements are defined in Table B.2.12-1.

Table B.2.12-1: E-UTRAN intra-frequency measurements under operation with frame structure 3

| Parameter | E-UTRA operating band groups Note 2 | Minimum SCH_RP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3

### B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements

This section defines the E-UTRAN inter-frequency SCH_RP in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements under frame structure 3.

The conditions for E-UTRAN inter-frequency CRS-based discovery signal measurements in discovery signal occasions are specified in Table B.2.13.1-1.

Table B.2.13.1-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 2 | Minimum SCH_RP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

### B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements

This section defines the E-UTRAN inter-frequency SCH_RP in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for E-UTRAN inter-frequency CSI-RS based discovery signal measurements in discovery signal occasions under frame structure 3 are specified in Table B.2.13.2-1.

Table B.2.13.2-1: E-UTRAN inter-frequency discovery signal measurements

| Parameter | E-UTRA operating band groups Note 2 | Minimum SCH_RP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.2.14 is defined in Section 3.1.

The conditions for CE mode A intra-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.14-1 and for E-UTRAN HD-FDD measurements are defined in Table B.2.14-2.

The conditions for CE mode B for intra-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.14-3 and for E-UTRAN HD-FDD measurements are defined in Table B.2.14-4.

Table B.2.14-1: E-UTRAN intra-frequency measurements for FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -127 | -6 |
|  | FDD-M1_B | -126.5 |  |
|  | FDD-M1_D | -125.5 |  |
|  | FDD-M1_E, TDD-M1_E | -125 |  |
|  | FDD-M1_F | -124.5 Note 2 |  |
|  | FDD-M1_G | -124 |  |
|  | FDD-M1_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.14-2: E-UTRAN intra-frequency measurements for HD-FDD for CEModeA

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, | -127 | -6 |
|  | FDD-M1_B | -126.5 |  |
|  | FDD-M1_D | -125.5 |  |
|  | FDD-M1_E | -125 |  |
|  | FDD-M1_F | -124.5 Note 2 |  |
|  | FDD-M1_G | -124 |  |
|  | FDD-M1_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.14-3: E-UTRAN intra-frequency measurements for FDD and TDD for CEModeB

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -136 | -15 |
|  | FDD-M1_B | -126.5 |  |
|  | FDD-M1_D | -134.5 |  |
|  | FDD-M1_E, TDD-M1_E | -134 |  |
|  | FDD-M1_F | -133.5 Note 2 |  |
|  | FDD-M1_G | -133 |  |
|  | FDD-M1_N | -129.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.14-4: E-UTRAN intra-frequency measurements for HD-FDD for CE mode B

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, | -136 | -15 |
|  | FDD-M1_B | -126.5 |  |
|  | FDD-M1_D | -134.5 |  |
|  | FDD-M1_E | -134 |  |
|  | FDD-M1_F | -133.5 Note 2 |  |
|  | FDD-M1_G | -133 |  |
|  | FDD-M1_N | -129.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1

This clause defines the NB-IoT intra-frequency NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.2.15 is defined in Section 3.6.

The conditions for intra-frequency measurements in normal coverage are defined in Table B.2.15-1 and B.2.15-3.

The conditions for intra-frequency measurements in denhanced coverage are defined in Table B.2.15-2 and B.2.15-4.

Table B.2.15-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NSCH_RP | NSCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_G | -129 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.15-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NSCH_RP | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_G | -138 | -15 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.15-3: NB-IoT intra-frequency measurements for TDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NSCH_RP | NSCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NTDD_G | -129 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.15-4: NB-IoT intra-frequency measurements for TDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NSCH_RP | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NTDD_G | -138 | -15 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1

This clause defines the NB-IoT intra-frequency PRP1,2 applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.2.16 is defined in Section 3.1.

The conditions for intra-frequency RSTD measurements in normal coverage are defined in Table B.2.16-1 and B.2.16-3.

The conditions for intra-frequency RSTD measurements in enhanced coverage are defined in Table B.2.16-2 and B.2.16-4.

Table B.2.16-1: NB-IoT intra-frequency RSTD measurements for HD-FDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -129 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.16-2: NB-IoT intra-frequency RSTD measurements for HD-FDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -135 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.16-3: NB-IoT intra-frequency RSTD measurements for TDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -129 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.16-4: NB-IoT intra-frequency RSTD measurements for TDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -135 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

## B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1

This clause defines the NB-IoT inter-frequency PRP1,2 applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.2.17 is defined in Section 3.1.

The conditions for intra-frequency RSTD measurements in normal coverage are defined in Table B.2.17-1 and B.2.17-3.

The conditions for intra-frequency RSTD measurements in enhanced coverage are defined in Table B.2.17-2 and B.2.17-4.

Table B.2.17-1: NB-IoT inter-frequency RSTD measurements for HD-FDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -129 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.17-2: NB-IoT inter-frequency RSTD measurements for HD-FDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G | -135 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.17-3: NB-IoT inter-frequency RSTD measurements for TDD in normal coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -129 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

Table B.2.17-4: NB-IoT inter-frequency RSTD measurements for TDD in enhanced coverrage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum PRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NTDD_G | -135 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |

## B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1

This clause defines the E-UTRAN inter-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.2.18 is defined in Section 3.1.

The conditions for CE mode A inter-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.18-1 and for E-UTRAN HD-FDD measurements are defined in Table B.2.18-2.

The conditions for CE mode B for inter-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.18-3 and for E-UTRAN HD-FDD measurements are defined in Table B.2.18-4.

Table B.2.18-1: E-UTRAN inter-frequency measurements for FDD and TDD for CEModeA

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -127 | -6 |
|  | FDD-M1_D | -125.5 |  |
|  | FDD-M1_E, TDD-M1_E | -125 |  |
|  | FDD-M1_F | -124.5 Note 2 |  |
|  | FDD-M1_G | -124 |  |
|  | FDD-M1_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.18-2: E-UTRAN inter-frequency measurements for HD-FDD for CEModeA

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, | -127 | -6 |
|  | FDD-M1_D | -125.5 |  |
|  | FDD-M1_E | -125 |  |
|  | FDD-M1_F | -124.5 Note 2 |  |
|  | FDD-M1_G | -124 |  |
|  | FDD-M1_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.18-3: E-UTRAN inter-frequency measurements for FDD and TDD for CEModeB

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, TDD-M1_A | -136 | -15 |
|  | FDD-M1_D | -134.5 |  |
|  | FDD-M1_E, TDD-M1_E | -134 |  |
|  | FDD-M1_F | -133.5 Note 1 |  |
|  | FDD-M1_G | -133 |  |
|  | FDD-M1_N | -129.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

Table B.2.18-4: E-UTRAN inter-frequency measurements for HD-FDD for CEModeB

| Parameter | E-UTRA operating band groups Note 3 | Minimum SCH_RP Note 1 | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | FDD-M1_A, | -136 | -15 |
|  | FDD-M1_D | -134.5 |  |
|  | FDD-M1_E | -134 |  |
|  | FDD-M1_F | -133.5 Note 1 |  |
|  | FDD-M1_G | -133 |  |
|  | FDD-M1_N | -129.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |

## B.2.19	Conditions for E-UTRAN measurements by UE Category M2

### B.2.19.1	Conditions for E-UTRAN intra-frequency measurements

This clause defines the E-UTRAN intra-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for intra-frequency measurements are defined in sub-section B.2.14.

### B.2.19.2	Conditions for E-UTRAN inter-frequency measurements

This clause defines the E-UTRAN inter-frequency SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for inter-frequency measurements are defined in sub-section B.2.18.

## B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1

This clause defines the E-UTRAN inter-frequency PRP1,2 applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.2.20 is defined in Section 3.1.

The conditions for CE mode A inter-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.20-1 and for E-UTRAN HD-FDD measurements are defined in Table B.2.20-2.

The conditions for CE mode B for inter-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.20-3 and for E-UTRAN HD-FDD measurements are defined in Table B.2.20-4.

Table B.2.20-1: E-UTRAN inter-frequency measurements for FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.20-2: E-UTRAN inter-frequency measurements for HD-FDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.20-3: E-UTRAN inter-frequency measurements for FDD and TDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E, TDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.20-4: E-UTRAN inter-frequency measurements for HD-FDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2

This section defines the inter-frequency PRP applicable for a corresponding operating band for Cat-M2.

The conditions for inter-frequency RSTD measurements are defined in sub-section B.2.20.

## B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1

This clause defines the E-UTRAN intra-frequency PRP1,2 applicable for a corresponding operating band. The UE category M1 applicability of the conditions in Appendix B.2.22 is defined in Section 3.1.

The conditions for CE mode A intra-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.22-1 and for E-UTRAN HD-FDD measurements are defined in Table B.2.22-2.

The conditions for CE mode B for intra-frequency E-UTRAN FDD and TDD measurements are defined in Table B.2.22-3 and for E-UTRAN HD-FDD measurements are defined in Table B.2.22-4.

Table B.2.22-1: E-UTRAN intra-frequency measurements for FDD and TDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.22-2: E-UTRAN intra-frequency measurements for HD-FDD for CE mode A

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, | -127 |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 Note 1 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.22-3: E-UTRAN intra-frequency measurements for FDD and TDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E, TDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

Table B.2.22-4: E-UTRAN intra-frequency measurements for HD-FDD for CE mode B

| Parameter | E-UTRA operating band groups Note 2 | Minimum PRP1,2 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, | -136 |
|  | FDD_D | -134.5 |
|  | FDD_E | -134 |
|  | FDD_F | -133.5 Note 1 |
|  | FDD_G | -133 |
|  | FDD_N | -129.5 |
| NOTE 1: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2

This section defines the intra-frequency PRP applicable for a corresponding operating band for Cat-M2.

The conditions for intra-frequency RSTD measurements are defined in sub-section B.2.22.

## B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1

This clause defines the NB-IoT intra-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.2.24 is defined in Section 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage are defined in Table B.2.24-1 and B.2.24-2.

Table B.2.24-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_G | -129 | -129 | -6 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table B.2.24-2: NB-IoT intra-frequency measurements for TDD in normal coverage

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NTDD_G | -129 | -129 | -6 | -6 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

## B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1

This clause defines the NB-IoT inter-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.2.25 is defined in Section 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection defined in Table B.2.24-1 and B.2.24-2 also apply for inter-frequency NB-IoT cells in normal coverage in this section.

# B.3	Conditions for measurements performance requirements for UE

## B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements are defined in Table B.3.1-1.

Table B.3.1-1: Intra-frequency absolute RSRP and RSRQ Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B1, FDD_B2 | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.2	Void

## B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP applicable for a corresponding operating band.

The conditions for inter-frequency absolute RSRP and RSRQ accuracy requirements are defined in Table B.3.1-1.

## B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for inter-frequency relative RSRP and RSRQ accuracy requirements are defined in Table B.3.8-1.

## B.3.5	Conditions for UE Rx – Tx time difference

This clause defines the E-UTRAN RSRP applicable for a corresponding operating band.

The conditions for UE Rx-Tx time difference are defined in Table B.3.1-1.

## B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements

This sections defines the E-UTRAN intra-frequency PRP  applicable for a corresponding operating band.

The conditions for intra-frequency RSTD measurements  are defined in Table B.2.5-1.

## B.3.7	Conditions for inter-frequency RSTD measurements

This sections defines the E-UTRAN inter-frequency PRP  applicable for a corresponding operating band.

The conditions for inter-frequency RSTD measurements  are defined in Table B.2.5-1.

## B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements are specified in Table B.3.8-1.

Table B.3.8-1: Intra-frequency relative RSRP accuracy requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B1, FDD_B2 | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements under time domain measurement resource restriction are as specified in Table B.3.1-1.

## B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements under time domain measurement resource restriction are defined in Table B.3.8-1.

## B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements under time domain measurement resource restriction with CRS assistance information are as specified in Table B.3.1-1.

## B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements under time domain measurement resource restriction with CRS assistance information are as specified in Table B.3.8-1.

## B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information

This clause defines the E-UTRAN RSRP applicable for a corresponding operating band.

The conditions for UE Rx-Tx time difference measurements, when time domain measurement resource restriction pattern and CRS assistance information are provided, are as defined in Table B.3.1-1.

## B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements

### B.3.14.1	Conditions for Intra-frequency CRS-based measurements

This clause defines the intra-frequency RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table B.3.1-1

### B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements

This clause defines the intra-frequency CSI-RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for intra-frequency absolute CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are specified in Table B.3.14.2-1

Table B.3.14.2-1: Intra-frequency Absolute CSI-RSRP Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B1, FDD_B2 | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements

### B.3.15.1	Conditions for Intra-frequency CRS-based measurements

This clause defines the intra-frequency RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for intra-frequency relative RSRP accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table B.3.8-1

### B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements

This clause defines the intra-frequency CSI-RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for intra-frequency relative CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are specified in Table B.3.15.2-1

Table B.3.15.2-1: Intra-frequency Relative CSI-RSRP Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum CSI-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD_A, TDD_A | -127 |
|  | FDD_B1, FDD_B2 | -126.5 |
|  | FDD_C, TDD_C | -126 |
|  | FDD_D | -125.5 |
|  | FDD_E, TDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_H | -123.5 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements

### B.3.16.1	Conditions for Inter-frequency CRS-based measurements

This clause defines the inter-frequency RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for inter-frequency absolute RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table B.3.1-1

### B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements

This clause defines the inter-frequency CSI-RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for inter-frequency absolute CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are as in Table B.3.14.2-1.

## B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements

### B.3.17.1	Conditions for Inter-frequency CRS-based measurements

This clause defines the inter-frequency RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CRS based discovery signal measurements.

The conditions for inter-frequency relative RSRP and RSRQ accuracy requirements for CRS-based discovery signal measurements in discovery signal occasions are as in Table B.3.8-1

### B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements

This clause defines the inter-frequency CSI-RSRP in discovery signal occasions [16], applicable for a corresponding operating band for CSI-RS based discovery signal measurements.

The conditions for inter-frequency relative CSI-RSRP accuracy requirements for CSI-RS-based discovery signal measurements in discovery signal occasions are as in Table B.3.15.2-1.

## B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute RS-SINR accuracy requirements are the same as defined in Table B.3.1-1.

## B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP applicable for a corresponding operating band.

The conditions for inter-frequency absolute RS-SINR accuracy requirements are the same as defined in Table B.3.1-1.

## B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements

This clause defines the E-UTRAN inter-frequency RSRP1,2 applicable for a corresponding operating band.

The conditions for inter-frequency relative RS-SINR accuracy requirements are the same as defined in Table B.3.8-1.

## B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3

### B.3.21.1	Conditions for RSRP measurements

This clause defines the intra-frequency absolute RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP accuracy requirements are defined in Table B.3.21.1-1.

Table B.3.21.1-1: Intra-frequency absolute RSRP requirements

| Parameter | E-UTRA operating band groups Note 2 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

### B.3.21.2	Conditions for RSRQ measurements

This clause defines the intra-frequency absolute RSRQ during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRQ accuracy requirements are the same as defined in Table B.3.21.1-1.

### B.3.21.3	Conditions for CSI-RSRP measurements

This clause defines the intra-frequency absolute CSI-RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for intra-frequency absolute RSRP accuracy requirements are defined in Table B.3.21.3-1.

Table B.3.21.3-1: Intra-frequency absolute CSI-RSRP requirements

| Parameter | E-UTRA operating band groups Note 2 | Minimum CSI-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3

### B.3.22.1	Conditions for RSRP measurements

This clause defines the intra-frequency relative RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for intra-frequency relative RSRP accuracy requirements are as defined in Table B.3.22.1-1.

Table B.3.22.1-1: Intra-frequency relative RSRP requirements

| Parameter | E-UTRA operating band groups Note 2 | Minimum RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

### B.3.22.2	Void

### B.3.22.3	Conditions for CSI-RSRP measurements

This clause defines the intra-frequency relative CSI-RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for intra-frequency relative CSI-RSRP accuracy requirements are as defined in Table B.3.22.3-1.

Table B.3.22.3-1: Intra-frequency relative CSI-RSRP requirements

| Parameter | E-UTRA operating band groups Note 2 | Minimum CSI-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FS3_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |

## B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3

### B.3.23.1	Conditions for RSRP measurements

This clause defines the inter-frequency absolute RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency absolute RSRP accuracy requirements are the same as defined in Table B.3.21.1-1.

### B.3.23.2	Conditions for RSRQ measurements

This clause defines the inter-frequency absolute RSRQ during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency absolute RSRQ accuracy requirements are the same as defined in Table B.3.21.1-1.

### B.3.23.3	Conditions for CSI-RSRP measurements

This clause defines the inter-frequency absolute CSI-RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency absolute CSI-RSRP accuracy requirements are the same as defined in Table B.3.21.3-1.

## B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3

### B.3.24.1	Conditions for RSRP measurements

This clause defines the inter-frequency relative RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency relative RSRP accuracy requirements are the same as defined in Table B.3.22.1-1.

### B.3.24.2	Conditions for RSRQ measurements

This clause defines the inter-frequency relative RSRQ during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency relative RSRP accuracy requirements are the same as defined in Table B.3.22.1-1.

### B.3.24.3	Conditions for CSI-RSRP measurements

This clause defines the inter-frequency relative CSI-RSRP during the configured DMTC occasion [2] under operation with frame structure 3 [16], applicable for a corresponding operating band.

The conditions for inter-frequency relative CSI-RSRP accuracy requirements are the same as defined in Table B.3.22.3-1.

## B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1

This clause defines the NB-IoT intra-frequency NRSRP applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.3.25 is defined in Section 3.6.

The conditions for intra-frequency absolute NRSRP and NRSRQ accuracy requirements are defined in Table B.3.25-1.

Table B.3.25-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_G, NTDD_G | -139.8 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

## B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1

This clause defines the NB-IoT inter-frequency NRSRP applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.3.26 is defined in Section 3.6.

The conditions for inter-frequency absolute NRSRP and NRSRQ accuracy requirements are defined in Table B.3.25-1.

## B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0

This clause defines the E-UTRAN intra-frequency RSRP applicable for a corresponding operating band. The UE category 0 applicability of the conditions in Appendix B.3.27 is defined in Section 3.1.

The conditions for intra-frequency absolute RSRP and RSRQ accuracy requirements are defined in Table B.3.27-1.

Table B.3.27-1: Intra-frequency absolute RSRP and RSRQ Accuracy Requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD-0_A, TDD-0_A | -127 |
|  | FDD-0_E, TDD-0_E | -125 |
|  | FDD-0_G | -124 |

## B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0

This clause defines the E-UTRAN intra-frequency RSRP1,2 applicable for a corresponding operating band. The UE category 0 applicability of the conditions in Appendix B.3.28 is defined in Section 3.1.

The conditions for intra-frequency relative RSRP accuracy requirements are specified in Table B.3.28-1.

Table B.3.28-1: Intra-frequency relative RSRP accuracy requirements

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | FDD-0_A, TDD-0_A | -127 |
|  | FDD-0_E, TDD-0_E | -125 |
|  | FDD-0_G | -124 |

## B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1

This sections defines the intra-frequency PRP applicable for a corresponding operating band for NB1.

The conditions for intra-frequency RSTD measurements are defined in Table B.2.16-1 and Table B.2.16-2

## B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1

This sections defines the inter-frequency PRP applicable for a corresponding operating band for NB1.

The conditions for inter-frequency RSTD measurements are defined in Table B.2.17-1 and Table B.2.17-2.

## B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1

This sections defines the inter-frequency PRP applicable for a corresponding operating band for Cat-M1.

The conditions for inter-frequency RSTD measurements are defined in sub-section B.2.20.

## B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2

This sections defines the inter-frequency PRP applicable for a corresponding operating band for Cat-M2.

The conditions for inter-frequency RSTD measurements are defined in sub-section B.2.21.

## B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1

This section defines the intra-frequency PRP applicable for a corresponding operating band for Cat-M1.

The conditions for intra-frequency RSTD measurements are defined in sub-section B.2.22.

## B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2

This section defines the intra-frequency PRP applicable for a corresponding operating band for Cat-M2.

The conditions for intra-frequency RSTD measurements are defined in sub-section B.2.23.

# B.4	RRM Requirements Exceptions

## B.4.1	General

## B.4.2	Receiver sensitivity relaxation for UE supporting CA

For a UE supporting inter-band carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 36.101 [5], Table 7.3.1-1A, the relevant side conditions specifying received power levels (E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) shall be increased by the amount Δ=ΔRIB,c defined for each of the downlink E-UTRA bands.

NOTE:	This side condition adjustment applies only for a UE supporting a single inter-band LTE CA band combination. For a UE supporting additional inter-band LTE CA band combinations, the ΔRIB,c for all bands supported by the UE, need to be studied [5].

## B.4.3	Receiver sensitivity relaxation for UE configured with CA

### B.4.3.1	Inter-band carrier aggregation

In this section, requirements exceptions are described for the UE configured with inter-band carrier aggregation with one uplink active in low operating band.

A relevant side condition (e.g., E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in 36.101, Table 7.3.1-1, and L2 is the reference sensitivity level specified in 36.101, Table 7.3.1A-0a, when the following conditions are fulfilled,

-	both downlink component carriers on different bands are configured with CA and active,

-	the single uplink is active in the low operating band,

-	the exception requirements specified in TS36.101, Table 7.3.1A-0a, apply.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.4.2 should not be applied.

### B.4.3.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIBNC>0 as defined in TS 36.101 [5], Table 7.3.1A-3, the relevant side conditions specifying received power levels (E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) shall be increased by the amount Δ=ΔRIBNC defined for the downlink SCC, when the following conditions are fulfilled,

-	both downlink component carriers are configured with CA and active,

-	one uplink carrier is active,

-	the exception requirements specified in TS36.101, Table 7.3.1A-3, apply.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.4.2 should not be applied.

### B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band

In this section, requirements are described for the UE configured with inter-band carrier aggregation involving one operating band without uplink band.

There is no relaxation in relevant side condition (e.g., E-UTRA RSRP, SCH_RP, PRP, CSI-RSRP, and Io) in a requirement, i.e., Δ=0, when the following conditions are fulfilled,

-	both downlink component carriers on different bands are configured with CA and active,

-	the single uplink is active in the high operating band,

-	conditions specified in TS36.101, Table 7.3.1A-0d, apply.

If Δ specified in this section applies, then no other additional relaxation to REFSENS shall be applied.

# B.5	Conditions for Measurement Performance Requirements for ProSe UE

## B.5.1	Conditions for S-RSRP Accuracy Requirements

This clause defines the S-RSRP applicable for a corresponding operating band.

The conditions for absolute S-RSRP accuracy requirements are defined in Table B.5.1-1.

Table B.5.1-1: Absolute S-RSRP Requirements

| Parameter | E-UTRA ProSe operating band groups Note 3 | Minimum S-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA ProSe operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

## B.5.2	Conditions for Relative S-RSRP Accuracy Requirements

This clause defines the S-RSRP1,2 applicable for a corresponding operating band.

The conditions for relative S-RSRP accuracy requirements are specified in Table B.5.2-1.

Table B.5.2-1: Relative S-RSRP accuracy requirements

| Parameter | E-UTRA ProSe operating band groups Note 3 | Minimum S-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 Note 2 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: The condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA ProSe operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

## B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the ProSe SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.5.3-1.

Table B.5.3-1: ProSe synchronization measurements

| Parameter | E-UTRA ProSe operating band groups Note 3 | Minimum ProSe SCH_RP Note 1 | ProSe SCH Ês/Iot Note 4 |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
|  | FDD_D | -125.5 | -4 |
|  | FDD_E | -125 |  |
|  | FDD_F | -124.5 |  |
|  | FDD_G | -124 |  |
|  | FDD_N | -120.5 |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: Void NOTE 3: E-UTRA ProSe operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. NOTE 4: ProSe SCH Ês/Iot for a SyncRef UE is the minimum of the Ês/Iot of PSSS/PSBCH and the Ês/Iot of SSSS |  |  |  |

## B.5.4	Conditions for SD-RSRP Accuracy Requirements

This clause defines the intra-frequency SD-RSRP applicable for a corresponding operating band.

The conditions for intra-frequency absolute SD-RSRP accuracy requirements are defined in Table B.5.4-1.

Table B.5.4-1: Absolute SD-RSRP Requirements

| Parameter | E-UTRA ProSe operating band groups Note 3 | Minimum SD-RSRP Note 1 dBm/15kHz |
| --- | --- | --- |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: Void NOTE 3: E-UTRA ProSe operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

## B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements

This clause defines the intra-frequency SD-RSRP applicable for a corresponding operating band.

The conditions for intra-frequency relative S-RSRP accuracy requirements are specified in Table B.5.5-1.

Table B.5.5-1: Relative S-RSRP accuracy requirements

| Parameter | E-UTRA ProSe operating band groups Note 3 | Minimum SD-RSRP1,2 Note 1 dBm/15kHz |
| --- | --- | --- |
|  | FDD_D | -125.5 |
|  | FDD_E | -125 |
|  | FDD_F | -124.5 |
|  | FDD_G | -124 |
|  | FDD_N | -120.5 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: Void NOTE 3: E-UTRA ProSe operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

# B.6	Conditions for V2X

## B.6.1	Test parameters for GNSS signals

This clause defines the reference signal power levels of generated salellites for a corresponding GNSS, which will be used in V2V and V2X test cases.

Table B.6.1-1: GNSS Referenece Signal Power Parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | 6 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: The DUT UE does not need to support all systems. The DUT UE shall support at least one system and will be test for the supported systems. |  |  |  |

## B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements

This clause defines the S-RSRP applicable for a corresponding operating band.

The conditions for absolute S-RSRP accuracy requirements are defined in Table B.6.2-1.

Table B.6.2-1: Absolute S-RSRP Requirements

| Parameter | E-UTRA V2X operating band groups Note2 | Minimum S-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

## B.6.3	Conditions for Relative S-RSRP Accuracy Requirements

This clause defines the S-RSRP1,2 applicable for a corresponding operating band.

The conditions for relative S-RSRP accuracy requirements are specified in Table B.6.3-1.

Table B.6.3-1: Relative S-RSRP accuracy requirements

| Parameter | E-UTRA V2X operating band groups Note 2 | Minimum S-RSRP1,2 Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

## B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the V2X SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.6.4-1.

Table B.6.4-1: V2X synchronization measurements

| Parameter | E-UTRA V2X operating band groups Note 2 | Minimum V2X SCH_RP Note 1 | V2X SCH Ês/Iot Note 3 |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
|  | TDD_G | 120 | 0 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. NOTE 3: V2X SCH Ês/Iot for a SyncRef UE is the minimum of the Ês/Iot of PSSS/PSBCH and the Ês/Iot of SSSS NOTE 4:: The SyncRef UE transmission frequency shall be accurate to within ±5 PPM compared to the absolute frequency. |  |  |  |

## B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements

This clause defines the PSSCH-RSRP applicable for a corresponding operating band.

The conditions for absolute PSSCH-RSRP accuracy requirements are defined in Table B.6.5-1.

Table B.6.5-1: Absolute PSSCH-RSRP Requirements

| Parameter | E-UTRA V2X operating band groups Note2 | Minimum PSSCH-RSRP Note 1 |
| --- | --- | --- |
|  |  | dBm/15kHz |
|  | TDD_G | -124 |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 2: E-UTRA V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. |  |  |

# B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing

## B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation

This clause defines the condition on the maximum timing difference between the earliest uplink carrier and the latest downlink carrier in carrier aggregation when a UE is configured with at least one serving cell that is configured with dl-STTI-Length-r15 or ShortProcessingTime =TRUE.

The timing difference between the earliest uplink carrier and the latest downlink carrier among all the serving cells configured to the UE is no larger than

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus4set1,

when any of the serving cells is configured with dl-STTI-Length-r15 =subslot and proc-Timeline-r15=nplus6set2,

when any of the serving cells is configured with ShortProcessingTime =TRUE,

when any of the serving cells is configured with dl-STTI-Length-r15=slot,

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set1,

when any of the serving cells is configured with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus8set2.

The values of the parameters for i=1,… ,6 are as specified in Table B.7.2-1.

Table B.7.2-1: Maximum Subframe Timing Boundary Difference Between Earliest Uplink Carrier and Latest Downlink Carrier

| Parameter |  | Value |
| --- | --- | --- |
|  |  | 66.67 μs (Note 1) |
|  |  | 166.67 μs (Note 2) |
|  |  | 200.00 μs (Note 3) |
|  |  | 309.90 μs (Note 4) |
|  |  | 352.08 μs (Note 5) |
|  |  | 452.08 μs (Note 6) |
| Note 1: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus4set1 [16] Note 2: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set2 [16] Note 3: Maximum timing advance with ShortProcessingTime = TRUE [16] Note 4: Maximum timing advance with dl-STTI-Length-r15=slot [16] Note 5: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus6set1 [16] Note 6: Maximum timing advance with dl-STTI-Length-r15=subslot and proc-Timeline-r15=nplus8set2 [16] |  |

Annex C (informative):
Change history:

| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2007-12 | RP#38 | RP-071037 |  |  |  | Approved version in TSG RAN#38 | 8.0.0 |
| 2008-03 | RP#39 | RP-080123 | 2 |  |  | Updates of TS36.133 | 8.1.0 |
| 2008-05 | RP#40 | RP-080325 | 3 |  |  | Updates of TS36.133 | 8.2.0 |
| 2008-09 | RP#41 | RP-080644 | 006 | 1 |  | E-UTRAN TDD intra frequency measurements when DRX is used | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 008 | 1 |  | E-UTRAN TDD - UTRAN TDD measurements | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 012 |  |  | RSRQ reporting Range | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 018 | 1 |  | Interfrequency and UTRA interRAT DRX peformance requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 020 | 1 |  | Additions to UE transmit timing requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 043 |  |  | Received interference power measurement performance requirement | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 044 |  |  | Cell Synchronization requirement for E-UTRA TDD | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 047 |  |  | Power Headroom Requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080644 | 048 |  |  | Event Triggering and Reporting Criteria Capability Requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 004 |  |  | Correction of E-UTRAN to UTRAN TDD handover | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 016 | 1 |  | Definition of Symbols | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 019 | 1 |  | Idle mode requirements updates | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 021 | 1 |  | General updates to 36.133 | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 023 | 1 |  | Handover requirements for E-UTRAN to cdma200 HRPD/1x | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 024 |  |  | Inter-frequency and inter-RAT measurement requirements for multiple layer monitoring | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 025 |  |  | Side conditions for UE measurement procedures and measurement performance requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 026 |  |  | Correction to cell reselection Requirement from E-UTRAN to HRPD/cdma200 1x | 8.3.0 |
| 2008-09 | RP#41 | RP-080642 | 027 |  |  | IRAT Measurement requirements in TS 36.133 | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 022 | 1 |  | Corrections to Handover requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 028 |  |  | Measurement reporting requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 029 | 2 |  | RRC re-establishment requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 032 |  |  | Correction to UE measurement requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 033 |  |  | Correction for the definition of interruption time | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 040 | 1 |  | Correction to idle mode higher priority search requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 045 |  |  | E-UTRAN TDD inter frequency measurement requirements | 8.3.0 |
| 2008-09 | RP#41 | RP-080713 | 046 |  |  | Updates of the Measurement procedures in RRC_Connected state from RAN 4#47bis and RAN 4#48 | 8.3.0 |
| 2008-12 | RP#42 | RP-080919 | 53 |  |  | Introduction of 700MHz Bands 12, 14 and 17 | 8.4.0 |
| 2008-12 | RP#42 | RP-080928 | 88 | 1 |  | CR to 36.133 on Radio Link Failure Monitoring | 8.4.0 |
| 2008-12 | RP#42 | RP-080929 | 51 |  |  | Correction to idle mode requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080929 | 52 |  |  | Definition of out of service area | 8.4.0 |
| 2008-12 | RP#42 | RP-080929 | 54 |  |  | Measurement requirements for UTRAN TDD cells in idle state | 8.4.0 |
| 2008-12 | RP#42 | RP-080929 | 69 | 2 |  | Correction of Inter-RAT UTRA cell reselection requirement | 8.4.0 |
| 2008-12 | RP#42 | RP-080929 | 55 |  |  | Correction of E_UTRAN cell measurement requirements in idle state | 8.4.0 |
| 2008-12 | RP#42 | RP-080930 | 76 |  |  | Correction to HO Requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080931 | 71 |  |  | Random access requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080932 | 85 |  |  | Cell phase synchronization error for large cell | 8.4.0 |
| 2008-12 | RP#42 | RP-080932 | 63 | 4 |  | Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 49 |  |  | E-UTRAN TDD-TDD intra/inter frequency measurement reporting requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 50 |  |  | E-UTRAN FDD – UTRAN FDD Measurement reporting requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 58 |  |  | Measurement requirement for E-UTRAN TDD to UTRAN TDD/FDD when DRX is used | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 60 |  |  | Interfrequency and GSM measurement performance requirements in large DRX | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 62 |  |  | Correction of implementation margin for transmission gap. | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 72 |  |  | Alignement of DRX cycle dependent requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 73 | 1 |  | Alignement of side conditions for mobility measurements | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 66 | 1 |  | Measurement models in RRC_CONNECTED | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 78 | 1 |  | Limitation of maximum number of layers for multiple monitoring | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 83 | 1 |  | GSM Cell identification requirements for parallel monitoring | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 87 |  |  | UE transmit timing requirement | 8.4.0 |
| 2008-12 | RP#42 | RP-080933 | 56 |  |  | Correction of TS 36.133 clause 8.1.2.1.1. | 8.4.0 |
| 2008-12 | RP#42 | RP-080934 | 77 |  |  | Correction to RSRQ Report Mapping | 8.4.0 |
| 2008-12 | RP#42 |  | 86 |  |  | Missing side conditions for RSRP and RSRQ | 8.4.0 |
| 2008-12 | RP#42 | RP-080935 | 81 | 1 |  | Phase I RRM Test Cases | 8.4.0 |
| 2008-12 | RP#42 |  | 80 | 1 |  | Test Configuration for RRM Tests: Measurement Reference Channels and OCNG | 8.4.0 |
| 2008-12 | RP#42 | RP-080936 | 75 |  |  | Cdma200 1xRTT Measurement Requirements | 8.4.0 |
| 2008-12 | RP#42 | RP-080937 | 74 | 1 |  | E-UTRA to UTRA cell search requirements for SON | 8.4.0 |
| 2009-03 | RP#43 | RP-090182 | 101 | 1 |  | Correction of A3-offset parameter in RRM test case | 8.5.0 |
| 2009-03 | RP#43 | RP-090182 | 105 |  |  | Some Editorial Corrections | 8.5.0 |
| 2009-03 | RP#43 | RP-090182 | 145 |  |  | Clarifications for the DRX state | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 89 |  |  | Modification on measurements of UTRAN TDD cells | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 91 |  |  | Clarification of the correct behavior when Treselection is not a multiple of idle mode reselection evaluation period | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 98 |  |  | Clarification of ‘Out of Service Area’ Concept and Definition | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 118 |  |  | Radio link monitoring | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 142 | 1 |  | Update of RRC_IDLE state mobility side conditions | 8.5.0 |
| 2009-03 | RP#43 | RP-090183 | 150 |  |  | UE measurement capability in Idle mode | 8.5.0 |
| 2009-03 | RP#43 | RP-090184 | 133 |  |  | Removal of RRC re-establishment procedure delay | 8.5.0 |
| 2009-03 | RP#43 | RP-090184 | 138 | 1 |  | Correction for the UE Re-establishment delay requirement | 8.5.0 |
| 2009-03 | RP#43 | RP-090185 | 92 | 2 |  | Cell phase synchronization accuracy | 8.5.0 |
| 2009-03 | RP#43 | RP-090185 | 97 |  |  | Radio link monitoring in DRX | 8.5.0 |
| 2009-03 | RP#43 | RP-090185 | 120 |  |  | UE Transmit Timing | 8.5.0 |
| 2009-03 | RP#43 | RP-090185 | 137 | 1 |  | Clarification of the reference point for the UE initial transmission timing control requirement | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 90 |  |  | Correction of clause 8.1.2.2.2.2 in TS36.133 | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 93 | 1 |  | cdma2000 1xRTT and HRPD Measurement Requirements | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 94 |  |  | Event Triggered Periodic Reporting Requirements for IRAT Measurements | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 95 |  |  | Measurement Reporting Requirements for E-UTRAN TDD – UTRAN TDD Measurements | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 99 | 1 |  | Clarification of UE behavior when measurement gap is used | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 100 |  |  | E-UTRA to UTRA cell search requirements in DRX for SON | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 110 | 1 |  | Correction to GSM BSIC Requirements for Parallel Monitoring | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 117 |  |  | Alignment of terminology for GAP | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 134 |  |  | Inter frequency and Inter RAT cell search requirement when DRX is used | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 139 |  |  | Correction of E-UTRAN FDD – UTRAN FDD measurements when no DRX | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 146 |  |  | Addition of the definition of “when DRX is used” | 8.5.0 |
| 2009-03 | RP#43 | RP-090186 | 147 | 1 |  | Corrections to E-UTRAN inter-frequency side conditions | 8.5.0 |
| 2009-03 | RP#43 | RP-090187 | 96 |  |  | Correction to Intra-frequency RSRP Accuracy Requirements | 8.5.0 |
| 2009-03 | RP#43 | RP-090187 | 136 | 1 |  | Power Headroom reporting delay | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 103 | 1 |  | E-UTRAN -GSM Handover Test Case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 104 | 1 |  | E-UTRAN FDD - UTRAN TDD Cell Search Test Cases in Fading | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 106 | 1 |  | E-UTRA FDD to UTRA FDD Handover Test Case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 107 | 1 |  | Correction of E-UTRA FDD-FDD Intra-frequency cell reselection test case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 108 | 1 |  | Correction of E-UTRA FDD-FDD priority based Inter-frequency cell reselection test case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 111 |  |  | E-UTRAN TDD - UTRAN FDD Handover Test Case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 112 | 1 |  | E-UTRAN FDD - GSM Cell Search Test Case in AWGN | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 113 |  |  | E-UTRAN - UTRAN FDD Cell Search Test Cases in Fading | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 114 | 1 |  | E-UTRAN UE Timing Accuracy Related Test Cases | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 115 | 1 |  | Inclusion of MBSFN Configurations for RRM Test Cases | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 116 |  |  | E-UTRAN FDD HRPD Cell Reselection Test Case; HRPD of Low Priority | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 122 | 1 |  | Clarification on Annex A.9: Measurement performance requirements | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 125 |  |  | E-UTRA TDD – UTRA TDD cell reselection: UTRA is of higher priority | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 126 |  |  | E-UTRA TDD – UTRA TDD cell reselection: UTRA is of lower priority | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 127 |  |  | E-UTRA FDD – UTRA TDD cell reselection | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 128 | 1 |  | E-UTRA TDD-UTRA TDD cell search (fading) | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 129 | 1 |  | E-UTRA TDD-UTRA TDD handover | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 132 | 1 |  | Addition of E-UTRA FDD to UTRA FDD reselection test cases | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 141 | 1 |  | Correction and introduction of some test related parameters | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 143 |  |  | Description of Annex A in TS 36.133 | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 148 |  |  | Reselection from E-UTRA to GSM cell test case | 8.5.0 |
| 2009-03 | RP#43 | RP-090370 | 149 |  |  | Radio Link Monitoring Test Cases | 8.5.0 |
| 2009-05 | RP#44 | RP-090546 | 151 |  |  | E-UTRA FDD UTRA TDD HO delay test case | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 153 |  |  | Correction of CQI reporting periodicity for TDD RLM test cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 157 |  |  | Correction to inter RAT reselection requirements to exclude equal priority. (Technically Endorsed CR in R4-50bis - R4-091092) | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 167 |  |  | Clarification of the number of monitoring carriers in idle mode. (Technically Endorsed CR in R4-50bis - R4-091394) | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 180 |  |  | Correction of Core spec references in A.9 Measurements performance test cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 984 |  |  | UTRA FDD-E-UTRA FDD/ TDD handover test cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 184 |  |  | SON ANR UTRAN FDD Cell Search Test Case | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 187 |  |  | E-UTRAN FDD cdma2000 1x RTT Cell Reselection Test Case; Cdma2000 1X of Low Priority | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 188 |  |  | E-UTRAN FDD cdma2000 HO Test cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 190 |  |  | E-UTRAN Random Access Test Cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 191 |  |  | E-UTRAN RRC Re-establishment Test Cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 192 |  |  | E-UTRAN TDD - GSM Cell Search Test Case in AWGN | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 197 |  |  | Correction to E-UTRAN FDD - GSM Handover Test case | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 173 | 1 |  | Correction of cell reselection test cases | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 179 | 1 |  | Test cases of E-UTRA TDD intra-frequency cell search in fading environment when DRX is used | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 152 | 1 |  | E-UTRA TDD GSM handover test case | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 178 | 1 |  | Test cases of E-UTRA FDD intra-frequency cell search in fading environment when DRX is used | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 201 | 1 |  | Test case for E-UTRA FDD E-UTRA FDD inter frequency cell search when DRX is used in fading conditions | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 185 | 1 |  | Correction to Radio Link Monitoring Tests | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 203 |  |  | Correction to E-UTRAN FDD to HRPD Cell Reselection Test Case | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 177 | 1 |  | Introduction of New Reference Channels and OCNG Patterns for 1.4MHz Bandwidth | 8.6.0 |
| 2009-05 | RP#44 | RP-090546 | 200 | 2 |  | Test case for E-UTRA TDD E-UTRA TDD inter frequency cell search when DRX is used in fading conditions | 8.6.0 |
| 2009-05 | RP#44 | RP-090547 | 158 |  |  | Alignment of inter frequency and inter RAT RRM reselection testcases with core requirements. (Technically Endorsed CR in R4-50bis - R4-091094) | 8.6.0 |
| 2009-05 | RP#44 | RP-090547 | 160 |  |  | Correction relating E-UTRAN TDD - UE Transmit Timing Accuracy Tests. (Technically Endorsed CR in R4-50bis - R4-091198) | 8.6.0 |
| 2009-05 | RP#44 | RP-090547 | 165 |  |  | Modifications of T3 and the verification point for in-sync test cases. (Technically Endorsed CR in R4-50bis - R4-091386) | 8.6.0 |
| 2009-05 | RP#44 | RP-090547 | 172 |  |  | E-UTRAN UE Timing Accuracy Related Test Cases. (Technically Endorsed CR in R4-50bis - R4-091517) | 8.6.0 |
| 2009-05 | RP#44 | RP-090547 | 171 | 1 |  | Reference measurement Channels for Radio Link Monitoring Tests with 2 Antennas. (Technically Endorsed CR in R4-50bis - R4-091508) | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 170 |  |  | Misalignment between TS36.133 and TS36.321. (Technically Endorsed CR in R4-50bis - R4-091457) | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 193 |  |  | Correction to Inter-RAT HO Interruption Time Definition | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 195 |  |  | CR c2k RRC delay | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 196 |  |  | CR c2k interruption time | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 162 |  |  | Clarifications to UE UL timing requirements. (Technically Endorsed CR in R4-50bis - R4-091357) | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 176 |  |  | Corrections of Random Access Requirements | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 154 |  |  | Correction of TGRP in clause 8.1.2.1.1 | 8.6.0 |
| 2009-05 | RP#44 | RP-090548 | 168 |  |  | Clarifications for the Relative RSRP and RSRQ measurement requirements. (Technically Endorsed CR in R4-50bis - R4-091407) | 8.6.0 |
| 2009-05 | RP#44 | RP-090549 | 161 |  |  | E-UTRAN UTRAN HO Command Processing Delay. (Technically Endorsed CR in R4-50bis - R4-091291) | 8.6.0 |
| 2009-05 | RP#44 | RP-090549 | 175 |  |  | Corrections of Cell Reselection Requirements in Idle Mode | 8.6.0 |
| 2009-05 | RP#44 | RP-090549 | 181 | 2 |  | Removal of [ ] from ranking criteria in Idle mode cell reselection | 8.6.0 |
| 2009-05 | RP#44 | RP-090550 | 156 |  |  | Correction on the TDD-TDD inter frequency measurements. (Technically Endorsed CR in R4-50bis - R4-091071) | 8.6.0 |
| 2009-05 | RP#44 | RP-090550 | 159 |  |  | Correction to the Referenced Clause Number for Tinter1. (Technically Endorsed CR in R4-50bis - R4-091153) | 8.6.0 |
| 2009-05 | RP#44 | RP-090551 | 166 |  |  | Further clarification of DRX/Non-DRX state. (Technically Endorsed CR in R4-50bis - R4-091389) | 8.6.0 |
| 2009-05 | RP#44 | RP-090551 | 202 |  |  | Correction on reference to 3GPP2 specification | 8.6.0 |
| 2009-05 | RP#44 | RP-090551 | 169 |  |  | OCNG simplification. (Technically Endorsed CR in R4-50bis - R4-091410) | 8.6.0 |
| 2009-05 | RP#44 | RP-090559 | 155 |  |  | Introduction of Extended LTE800 requirements. (Technically Endorsed CR in R4-50bis - R4-091063) | 9.0.0 |
| 2009-05 | RP#45 | RP-090817 | 211 |  |  | Correction to TDD RMC references in RLM test cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 205 |  |  | Introduction of Reference DRX configurations | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 207 |  |  | Addition of DRX configurations into non DRX test cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 225 |  |  | Correction to HO Test Cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 227 |  |  | Correction to E-UTRAN GSM BSIC Identification Requirements with DRX | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 259 |  |  | Corrections of Test Cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 314 |  |  | E-UTRA FDD - E-UTRA FDD and UTRA FDD cell search test cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 315 |  |  | E-UTRAN Radio Link Monitoring Test Cases in DRX | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 316 |  |  | Inter-frequency E-UTRA - E-UTRA HO test cases: unknown target cell | 9.1.0 |
| 2009-05 | RP#45 | RP-090880 | 263 | 2 |  | E-UTRA FDD UTRA FDD Blind Handover test case: unknown target cell | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 321 | 1 |  | Small corrections to Measurements performance tests parameters | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 285 | 1 |  | E-UTRAN GSM Cell Search in DRX Test Cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 267 |  |  | Set 3.2. E-UTRA TDD to UTRA TDD cell search in DRX under fading | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 269 |  |  | Set 3.6. Test case of E-UTRA TDD to E-UTRA TDD and UTRA TDD combined cell search under fading | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 271 |  |  | Set 3.12. E-UTRA TDD to UTRA TDD blind handover test | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 279 |  |  | E-UTRAN FDD - UTRAN FDD Cell Search in DRX Test Cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 281 |  |  | E-UTRAN TDD- E-UTRAN TDD and E-UTRAN TDD Inter-frequency Cell Search Test Case | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 283 |  |  | E-UTRAN GSM Blind Handover Test Cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 287 |  |  | E-UTRAN FDD cdma2000 Blind HO Test cases | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 302 |  |  | RRM Test case for multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions | 9.1.0 |
| 2009-05 | RP#45 | RP-090836 | 304 |  |  | Fading reselection test case between E-UTRA and UTRA (UTRA of lower priority | 9.1.0 |
| 2009-05 | RP#45 | RP-090828 | 233 |  |  | CR SI HRPD correction | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 215 | 1 |  | Corrections to Measurements of HRPD cells and cdma2000 1X | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 231 |  |  | CR reference correction | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 235 | 1 |  | Corrections to Measurements of GSM cells in RRC_IDLE | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 247 |  |  | Range of Idle Mode Es/Iot side conditions | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 249 |  |  | Removal of [ ] from Tdetect, Tmeasure and Tevaluate | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 245 | 1 |  | Clarification to applicability of RSRP side conditions in Idle mode | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 317 |  |  | CR Idle mode IF measurement condition | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 318 |  |  | CR Idle mode IF measurement period | 9.1.0 |
| 2009-05 | RP#45 | RP-090879 | 217 | 2 |  | Corrections to E-UTRAN RRC_IDLE state mobility requirements | 9.1.0 |
| 2009-05 | RP#45 | RP-090814 | 265 | 1 |  | Correction to Random Access | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 221 |  |  | E-UTRAN TDD-TDD inter frequency cell search/measurement requirements when DRX is used | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 223 |  |  | E-UTRAN inter RAT measurement requirements | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 229 |  |  | Correction to Monitoring of Multiple Layers Using Gaps | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 219 | 1 |  | E-UTRAN FDD-FDD inter frequency measurements when DRX is used | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 322 |  |  | CR GSM measurement period | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 323 |  |  | CR cdma2000 1x and HRPD number of carriers | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 213 | 1 |  | Editorial correction on E-UTRAN inter frequency measurements | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 261 | 1 |  | E-UTRAN TDD intra frequency measurements | 9.1.0 |
| 2009-05 | RP#45 | RP-090816 | 319 | 1 |  | Clarification of the number of monitoring cells for intra frequency measurements | 9.1.0 |
| 2009-05 | RP#45 | RP-090815 | 237 |  |  | Correction of timing advance adjustment accuracy test case | 9.1.0 |
| 2009-05 | RP#45 | RP-090815 | 291 |  |  | Correction to UE Transmit Timing Requirements | 9.1.0 |
| 2009-12 | RP-46 | RP-091275 | 329 |  |  | Defining requirements for UTRA TDD measurements for SON (Technically endorsed at RAN 4 52bis in R4-093512) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 332 |  |  | Modification of test case of E-UTRA TDD intra frequency cell reselection (Technically endorsed at RAN 4 52bis in R4-093552) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 333 |  |  | Modification of test case of E-UTRA TDD inter frequency cell reselection (Technically endorsed at RAN 4 52bis in R4-093553) | 9.2.0 |
| 2009-12 | RP-46 | RP-091286 | 334 |  |  | Introduction of Extended LTE1500 requirements for TS36.133 (Technically endorsed at RAN 4 52bis in R4-093636) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 336 |  |  | Addition of E-UTRA TDD to UTRA FDD reselection test cases (Technically endorsed at RAN 4 52bis in R4-093686) | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 338 |  |  | Correction of missing accuracy requirements for UTRAN FDD (Technically endorsed at RAN 4 52bis in R4-093689) | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 340 |  |  | CR cdma2000 HRPD measurement period (Technically endorsed at RAN 4 52bis in R4-093720) | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 342 |  |  | CR cdma2000 1x measurement period (Technically endorsed at RAN 4 52bis in R4-093721) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 344 |  |  | Correction for E-UTRAN FDD - UTRAN FDD Cell Search in DRX Test Cases (Technically endorsed at RAN 4 52bis in R4-093890) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 346 |  |  | Revise geometry factors for Intra freq Reselection Test Cases | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 348 |  |  | Corrections on RRM parameters for Bands 12, 14, 17 | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 351 | 1 |  | Corrections to PDSCH RMC-s | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 353 |  |  | Corrections of TS36.133 | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 356 | 1 |  | UTRA TDD P-CCPCH RSCP absolute accuracy measurement in E-UTRAN | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 358 | 1 |  | E-UTRAN TDD - UTRAN TDD cell search for SON | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 361 |  |  | Cell Search Requirements for Intra-LTE Handover to Unknown Target Cell | 9.2.0 |
| 2009-12 | RP-46 | RP-091273 | 365 |  |  | Combined E-UTRAN interfrequency and GSM cell search test cases (Scenario set 3.2) | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 367 | 1 |  | Correction in UE UTRA TDD P-CCPCH RSCP measurement capability for R9 | 9.2.0 |
| 2009-12 | RP-46 | RP-091273 | 374 |  |  | E-UTRAN GSM RSSI Measurement Accuracy Tests | 9.2.0 |
| 2009-12 | RP-46 | RP-091273 | 375 |  |  | E-UTRAN UTRAN FDD CPICH RSCP Measurement Accuracy Tests | 9.2.0 |
| 2009-12 | RP-46 | RP-091273 | 376 |  |  | E-UTRAN UTRAN FDD CPICH Ec/No Measurement Accuracy Tests | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 378 |  |  | Cell Timing Change Requirements for Event Triggered Reporting | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 380 |  |  | Correction to Power Headroom Requirements | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 382 |  |  | Editorial corrections to 36.133 | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 387 |  |  | Editorial corrections to the time units for RRC Re-establishment test cases | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 389 | 1 |  | Introduction of cell search test case in DRX to verify L3 filtering | 9.2.0 |
| 2009-12 | RP-46 | RP-091271 | 391 |  |  | Correction to ONCG Patterns | 9.2.0 |
| 2009-12 | RP-46 | RP-091275 | 329 |  |  | Defining requirements for UTRA TDD measurements for SON (Technically endorsed at RAN 4 52bis in R4-093512) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 332 |  |  | Modification of test case of E-UTRA TDD intra frequency cell reselection (Technically endorsed at RAN 4 52bis in R4-093552) | 9.2.0 |
| 2009-12 | RP-46 | RP-091272 | 333 |  |  | Modification of test case of E-UTRA TDD inter frequency cell reselection (Technically endorsed at RAN 4 52bis in R4-093553) | 9.2.0 |
| 2010-03 | RP-47 | RP-100254 | 410 |  |  | Idle mode corrections | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 405 | 1 |  | UE measurement capability requirements in Idle and Connected | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 423 |  |  | Correction to UE Measurement Capability Requirements in Idle Mode | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 412 |  |  | Removal of activation time from interRAT handover requirements | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 417 | 1 |  | Correction to UE Transmit Timing Requirements | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 402 |  |  | Correction of E-UTRAN TDD inter frequency measurements_R9 | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 414 | 1 |  | Enhanced GSM Requirements for CSFB | 9.3.0 |
| 2010-03 | RP-47 | RP-100254 | 415 | 1 |  | Enhanced UTRA FDD Requirements for CSFB | 9.3.0 |
| 2010-03 | RP-47 | RP-100255 | 399 |  |  | Correction of RSRP value in E-UTRAN FDDFDD Inter frequency reselection test | 9.3.0 |
| 2010-03 | RP-47 | RP-100255 | 397 |  |  | Addition of missing Es/Noc parameters in RRM test cases | 9.3.0 |
| 2010-03 | RP-47 | RP-100255 | 421 |  |  | Correction to RRC Re-establishment Test Case | 9.3.0 |
| 2010-03 | RP-47 | RP-100255 | 427 | 1 |  | Correction of UE transmit timing test case | 9.3.0 |
| 2010-03 | RP-47 | RP-100255 | 419 | 1 |  | Correction to RLM Test Cases | 9.3.0 |
| 2010-03 | RP-47 | RP-100262 | 407 |  |  | Editorial Corrections in TS36.133(Rel-9) | 9.3.0 |
| 2010-03 | RP-47 | RP-100263 | 413 |  |  | Introduction of LTE in 800 MHz for Europe requirements in TS 36.133 | 9.3.0 |
| 2010-03 | RP-47 | RP-100264 | 395 |  |  | Corrections for Extended UMTS1500 in TS36.133(Rel-9) | 9.3.0 |
| 2010-03 | RP-47 | RP-100269 | 393 |  |  | AOA and TA measurement report mappings | 9.3.0 |
| 2010-03 | RP-47 | RP-100269 | 403 | 2 |  | Mapping of UE RxTx time difference measurement | 9.3.0 |
| 2010-03 | RP-47 | RP-100266 | 425 | 2 |  | Home eNode B synchronization requirement | 9.3.0 |
| 2010-03 | RP-47 | RP-100266 | 424 | 2 |  | Minimum requirements on SI reading for HeNB inbound mobility | 9.3.0 |
| 2010-06 | RP-48 | RP-100622 | 473 |  |  | Clarification on radio link monitoring | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 472 |  |  | Corrections of clause numbering on the test case of E-UTRAN FDD-FDD inter-frequency cell search requirements for L3 fitering | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 466 | 1 |  | Correction to RRM Test Cases | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 464 |  |  | Correction to RRM Requirements | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 462 | 1 |  | Correction to Absolute RSRP/RSRQ Definitions | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 457 |  |  | UE Measurement Capability Requirements for CDMA2000 | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 455 | 1 |  | Correction of E-UTRAN Inter-frequency Cell Re-selection Requirements | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 451 | 1 |  | Correction to idle mode requirements(Rel-9) | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 449 | 1 |  | Editorial corrections to 36.133(Rel-9) | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 447 |  |  | Correction to TDD intrafrequency accuracy test case | 9.4.0 |
| 2010-06 | RP-48 | RP-100622 | 441 | 1 |  | Correction of Io value in E-UTRAN FDD and TDD Inter frequency RSRP tests | 9.4.0 |
| 2010-06 | RP-48 | RP-100627 | 444 | 2 |  | Corrections to CSG SI reading core requirement | 9.4.0 |
| 2010-06 | RP-48 | RP-100627 | 445 | 1 |  | RSRQ idle mode requirements | 9.4.0 |
| 2010-06 | RP-48 | RP-100630 | 470 | 1 |  | Test cases for R9 cell reselection enhancements | 9.4.0 |
| 2010-06 | RP-48 | RP-100630 | 460 |  |  | Missing E-UTRA - UTRA FDD DRX Requirements | 9.4.0 |
| 2010-06 | RP-48 | RP-100631 | 442 | 2 |  | Corrections to enhanced cell identification core requirement | 9.4.0 |
| 2010-06 | RP-48 | RP-100632 | 469 |  |  | Applicability of mobility requirements with inter-frequency RSTD measurements | 9.4.0 |
| 2010-06 | RP-48 | RP-100632 | 439 |  |  | UE Rx-Tx Time Difference Measurement Requirements for E-CID | 9.4.0 |
| 2010-06 | RP-48 | RP-100632 | 438 | 2 |  | CR UE RX-TX time-difference measurement requirement | 9.4.0 |
| 2010-06 | RP-48 | RP-100632 | 433 | 5 |  | RSTD Measurement Requirements for OTDOA | 9.4.0 |
| 2010-06 | RP-48 | RP-100632 | 432 | 5 |  | RSTD Accuracy Requirements for OTDOA | 9.4.0 |
| 2010-09 | RP-49 | RP-100914 | 477 | 1 |  | Cell identity change time in RRM Test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 537 |  |  | A clarification text in the RSTD intra-frequency accuracy requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 506 |  |  | Correction of drx-RetransmissionTimer parameters | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 508 |  |  | Correction of Io value in RSRP FDD and TDD Intra frequency test | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 521 | 1 |  | Editorial corrections to 36.133 (R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 523 |  |  | Alignment of REFSENS between 36.101 and 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 525 | 1 |  | Correction of Time to Trigger unit for 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 505 | 1 |  | Corrections to 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 528 | 1 |  | E-UTRAN FDD Intra Frequency RSTD Measurement Accuracy test case | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 538 | 1 |  | Correction to Enhanced BSIC Verification Requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 539 |  |  | Enhanced CSFB Requirements with DRX | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 540 |  |  | Correction to E-CID Requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 544 | 1 |  | Addition of UTRA and GSM enhanced cell identification test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 547 | 1 |  | E-UTRAN FDD UE Rx – Tx Time Difference Measurement Accuracy test case | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 479 | 1 |  | Scrambling code change time in RRM Test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 549 |  |  | Introduction of CSG cell reselection requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 527 |  |  | correction of redundant Hysteresis(Hys) for 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 488 | 2 |  | Test case for TDD UE Rx-Tx time difference measurement | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 483 |  |  | Clarification of Radio link monitoring test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 485 |  |  | Test case for E-UTRA TDD event triggered reporting when L3 filtering is used in R9 | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 487 |  |  | E-UTRA TDD - UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority in R9 | 9.5.0 |
| 2010-09 | RP-49 | RP-100924 | 492 |  |  | Test case for E-UTRAN TDD in the existence of non-allowed CSG cell | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 494 |  |  | PDCCH Aggregation level for RRM tests | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 503 |  |  | Correction of ES/Iot value in E-UTRAN RSRQ FDD intra frequency test | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 496 |  |  | Corrections to RRM OCNG Patterns | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 498 |  |  | RRC timer accuracy requirement | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 501 |  |  | Correction of OCNG | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 477 | 1 |  | Cell identity change time in RRM Test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 537 |  |  | A clarification text in the RSTD intra-frequency accuracy requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 506 |  |  | Correction of drx-RetransmissionTimer parameters | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 508 |  |  | Correction of Io value in RSRP FDD and TDD Intra frequency test | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 521 | 1 |  | Editorial corrections to 36.133 (R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 523 |  |  | Alignment of REFSENS between 36.101 and 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 525 | 1 |  | Correction of Time to Trigger unit for 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 505 | 1 |  | Corrections to 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 528 | 1 |  | E-UTRAN FDD Intra Frequency RSTD Measurement Accuracy test case | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 538 | 1 |  | Correction to Enhanced BSIC Verification Requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 539 |  |  | Enhanced CSFB Requirements with DRX | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 540 |  |  | Correction to E-CID Requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 544 | 1 |  | Addition of UTRA and GSM enhanced cell identification test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 547 | 1 |  | E-UTRAN FDD UE Rx – Tx Time Difference Measurement Accuracy test case | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 479 | 1 |  | Scrambling code change time in RRM Test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 549 |  |  | Introduction of CSG cell reselection requirements | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 527 |  |  | correction of redundant Hysteresis(Hys) for 36.133(R9) | 9.5.0 |
| 2010-09 | RP-49 | RP-100920 | 488 | 2 |  | Test case for TDD UE Rx-Tx time difference measurement | 9.5.0 |
| 2010-09 | RP-49 | RP-100914 | 483 |  |  | Clarification of Radio link monitoring test cases | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 485 |  |  | Test case for E-UTRA TDD event triggered reporting when L3 filtering is used in R9 | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 487 |  |  | E-UTRA TDD - UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority in R9 | 9.5.0 |
| 2010-09 | RP-49 | RP-100924 | 492 |  |  | Test case for E-UTRAN TDD in the existence of non-allowed CSG cell | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 494 |  |  | PDCCH Aggregation level for RRM tests | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 503 |  |  | Correction of ES/Iot value in E-UTRAN RSRQ FDD intra frequency test | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 496 |  |  | Corrections to RRM OCNG Patterns | 9.5.0 |
| 2010-09 | RP-49 | RP-100919 | 498 |  |  | RRC timer accuracy requirement | 9.5.0 |
| 2010-09 | RP-49 | RP-100915 | 501 |  |  | Correction of OCNG | 9.5.0 |
| 2010-09 | RP-49 | RP-100927 | 497 |  |  | CR LTE_TDD_2600_US spectrum band definition additions to TS 36.133 | 10.0.0 |
| 2010-12 | RP-50 | RP-101331 | 635 |  |  | Corrections to 36.133 performance requirements | 10.1.0 |
| 2010-12 | RP-50 | RP-101331 | 638 |  |  | Correction to intra frequency cell identification time for FDD and TDD | 10.1.0 |
| 2010-12 | RP-50 | RP-101331 | 566 | 1 |  | Corrections and Clarifications to TS36.133 | 10.1.0 |
| 2010-12 | RP-50 | RP-101331 | 592 | 2 |  | Correction to Radio link monitoring test cases | 10.1.0 |
| 2010-12 | RP-50 | RP-101332 | 563 |  |  | PDCCH Aggregation Level for RRM Tests | 10.1.0 |
| 2010-12 | RP-50 | RP-101332 | 571 |  |  | MIMO correlation scenario for RLM test cases | 10.1.0 |
| 2010-12 | RP-50 | RP-101332 | 580 |  |  | Removal of [ ] from PDSCH and PCFICH/PDCCH/PHICH Measurement Channel references in Annex A. | 10.1.0 |
| 2010-12 | RP-50 | RP-101332 | 585 |  |  | Enabling HARQ for RRM Tests | 10.1.0 |
| 2010-12 | RP-50 | RP-101335 | 643 | 1 |  | Completion of CSG cell reselection requirements | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 568 |  |  | Clarification of measurements requirements for HRPD and cdma2000 1x | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 589 |  |  | Addition of Band 18, 19 and 21 into UE Rx - Tx time difference requirements | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 604 |  |  | Correction to Enhanced GSM Cell Identification Requirement | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 632 |  |  | Correction of reselection requirement for UTRAN FDD cells | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 640 |  |  | Correction to Enhanced UTRA FDD Cell Identification Requirements | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 645 |  |  | E-UTRAN TDD Intra Frequency RSTD Measurement Accuracy test case | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 621 | 1 |  | Correction for Measurements of inter-RAT cells | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 598 | 2 |  | E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case | 10.1.0 |
| 2010-12 | RP-50 | RP-101343 | 600 | 2 |  | E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case | 10.1.0 |
| 2010-12 | RP-50 | RP-101356 | 644 |  |  | Band 42 and 43 parameters for UMTS/LTE 3500 (TDD) for TS 36.133 | 10.1.0 |
| 2010-12 | RP-50 | RP-101361 | 552 |  |  | Introduction of L-band in TS36.133 | 10.1.0 |
| 2010-12 | RP-50 | RP-101388 | 648 |  |  | Removal of square brackets from scope of TS36.133 | 10.1.0 |
| 2011-04 | RP-51 | RP-110359 | 0658 | - |  | Addition of UE RRM capabilities for CA | 10.2.0 |
| 2011-04 | RP-51 | RP-110340 | 0663 | - |  | Correction to E-UTRAN TDD in-sync test requirements | 10.2.0 |
| 2011-04 | RP-51 | RP-110348 | 0665 | 1 |  | RSTD requirements, RMC and OCNG patterns | 10.2.0 |
| 2011-04 | RP-51 | RP-110350 | 0669 | - |  | CR to 36.133: Aligning relavant RRM requirements for Band 41 with the reference sensitivity values in 36.101 | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0676 | - |  | Modification on test case of E-UTRA TDD to UTRA TDD cell reselection(R10) | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0681 | 1 |  | Value of MS_TXPWR_MAX_CCH for EUTRA-GSM reselection test cases A.4.4.x | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0687 | 1 |  | Rearrangement of Time periods for EUTRA-UTRA reselection test case A.4.3.1.1 | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0690 | 1 |  | Removal of "Force to Cell 2" during initialisation for EUTRA-UTRA reselection test case A.4.3.1.2 | 10.2.0 |
| 2011-04 | RP-51 | RP-110340 | 0693 | 1 |  | SNR for RRM A.8.x test cases using ETU70 | 10.2.0 |
| 2011-04 | RP-51 | RP-110408 | 0697 | 1 |  | Requirements for Minimaztion of Drive Tests (MDT) in LTE |  |
| 2011-04 | RP-51 | RP-110339 | 0703 | - |  | Correction to test cases of E-UTRA to UTRA cell reselection when UE is in idle state | 10.2.0 |
| 2011-04 | RP-51 | RP-110359 | 0706 | 2 |  | Introduction of measurement requirements for carrier aggregation | 10.2.0 |
| 2011-04 | RP-51 | RP-110347 | 0709 | 1 |  | Addition of test cases for FDD intra-frequency SI reading using autonomous gaps with both non DRX and DRX for Rel-10 | 10.2.0 |
| 2011-04 | RP-51 | RP-110347 | 0711 | 1 |  | Addition of test cases for FDD inter-frequency SI reading using autonomous gaps with both non DRX and DRX for Rel-10 | 10.2.0 |
| 2011-04 | RP-51 | RP-110359 | 0713 | 1 |  | Introduction of core requirements of radio link monitoring in CA | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0719 | 1 |  | Modification on Test Requirements in E-UTRA - UTRA TDD SON Test Case (A.8.7.3) (R10) | 10.2.0 |
| 2011-04 | RP-51 | RP-110348 | 0727 | 2 |  | Requirements for reporting criteria with positioning measurements | 10.2.0 |
| 2011-04 | RP-51 | RP-110340 | 0736 | - |  | Correction of RLM evaluation period in DRX | 10.2.0 |
| 2011-04 | RP-51 | RP-110340 | 0739 | - |  | Correction of inter-frequency measurement accuracy test cases | 10.2.0 |
| 2011-04 | RP-51 | RP-110339 | 0744 | - |  | Modification on Test Requirements in E-UTRA GSM cell reselection Test Case (A.4.4) (R10) | 10.2.0 |
| 2011-04 | RP-51 | RP-110348 | 0747 | 1 |  | Corrections to RSTD measurement for Rel-9 | 10.2.0 |
| 2011-04 | RP-51 | RP-110348 | 0748 | - |  | Correction on FDD Intra Frequency RSTD Measurement Accuracy test case | 10.2.0 |
| 2011-04 | RP-51 | RP-110348 | 0751 | 1 |  | RSTD test case corrections | 10.2.0 |
| 2011-04 | RP-51 | RP-110344 | 0753 | - |  | Correction of serving cell performance requirements for autonomous SI acquisition | 10.2.0 |
| 2011-06 | RP-52 | RP-110753 | 0785 | 1 |  | Simplification of frequency dependent requirements in 36.133 (Table B.2.2-1 contains erroneous values. These wrong values will be corrected in the RAN#53 meeting.) | 10.3.0 |
| 2011-06 | RP-52 | RP-110793 | 754 |  |  | E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency | 10.3.0 |
| 2011-06 | RP-52 | RP-110793 | 755 |  |  | E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 757 |  |  | Core requirements on RRC connection mobility control in CA | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 758 |  |  | Timing core requirements in CA | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 759 |  |  | Introduction of Handover Requirements for Carrier Aggregation | 10.3.0 |
| 2011-06 | RP-52 | RP-110793 | 760 |  |  | E-UTRAN FDD Inter Frequency RSTD Measurement Accuracy test case | 10.3.0 |
| 2011-06 | RP-52 | RP-110793 | 761 |  |  | E-UTRAN TDD Inter Frequency RSTD Measurement Accuracy test case | 10.3.0 |
| 2011-06 | RP-52 | RP-110786 | 765 |  |  | Rearrangement of Time periods for EUTRA-UTRA reselection test case A.4.3.4.1 | 10.3.0 |
| 2011-06 | RP-52 | RP-110786 | 768 |  |  | Removal of "Force to Cell 2" during initialisation for EUTRA -UTRA reselection test cases | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 776 |  |  | Introduction of UE interruption requirements in SCC measurements with de-activated SCell | 10.3.0 |
| 2011-06 | RP-52 | RP-110794 | 797 |  |  | Editorial Correction to Cell Re-selection Requirements | 10.3.0 |
| 2011-06 | RP-52 | RP-110789 | 808 |  |  | Correction to side conditions for TDD inter-frequency CGI identification for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110786 | 814 |  |  | Correction to inter-RAT cell identificiation time in DRX for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110787 | 817 |  |  | Correction to identification time of UTRA FDD cell for SON in DRX for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110787 | 822 |  |  | Correction to requirements of E-UTRAN TDDUTRAN TDD measurements for SON when DRX is used for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 829 |  |  | Corrrection to the side condition for measurements for E-UTRA carrier aggregation | 10.3.0 |
| 2011-06 | RP-52 | RP-110803 | 850 |  |  | CR Timestamp accuracy requirements for MDT | 10.3.0 |
| 2011-06 | RP-52 | RP-110812 | 778 | 1 |  | Add 2GHz S-Band (Band 23) in 36.133 | 10.3.0 |
| 2011-06 | RP-52 | RP-110796 | 787 | 1 |  | Clarification on inter-frequency layers for RSTD | 10.3.0 |
| 2011-06 | RP-52 | RP-110794 | 780 | 1 |  | Correction to RSTD measurement for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 852 | 1 |  | Pcmax,c mapping | 10.3.0 |
| 2011-06 | RP-52 | RP-110787 | 771 | 1 |  | Clarification of Radio link monitoring test requirements (The CR was not implemented as it is not based on the latest version of the specification) | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 793 | 1 |  | E-CID Measurement Requirements under Pcell Switching | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 775 | 1 |  | Removal of undefined intra-freq RSRQ relative accuracy requirements in CA | 10.3.0 |
| 2011-06 | RP-52 | RP-110789 | 856 |  |  | Correction on E-UTRAN FDD RSTD intra frequency case | 10.3.0 |
| 2011-06 | RP-52 | RP-110796 | 800 | 1 |  | Addition of E-UTRAN FDD/TDD cdma2000 1xRTT measurements requirement for SON for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110790 | 804 | 1 |  | Addition of test cases for TDD intra-frequency SI reading using autonomous gaps with both non DRX and DRX for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110790 | 806 | 1 |  | Addition of test cases for TDD inter-frequency SI reading using autonomous gaps with both non DRX and DRX for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110787 | 828 | 1 |  | Addition of missing EsNoc parameters in E-UTRAN TDD UTRAN TDD Measurements test cases for Rel-10 | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 835 | 1 |  | Clarification of UE Rx-Tx time difference measurement requirement for carrier aggregation | 10.3.0 |
| 2011-06 | RP-52 | RP-110804 | 859 |  |  | Expanded 1900 MHz addition to 36.133 | 10.3.0 |
| 2011-06 | RP-52 | RP-110811 | 860 |  |  | Introduction of RLM requirement for eICIC | 10.3.0 |
| 2011-06 | RP-52 | RP-110796 | 794 | 1 |  | E-CID Measurement Requirements under Handover | 10.3.0 |
| 2011-06 | RP-52 | RP-110811 | 762 | 1 |  | CR on RLM requirements for eICIC | 10.3.0 |
| 2011-06 | RP-52 | RP-110811 | 788 | 2 |  | RSRP and RSRQ measurement requirements for eICIC | 10.3.0 |
| 2011-06 | RP-52 | RP-110811 | 851 | 1 |  | CR on RSRP and RSRQ measurement accuracy requirements for eICIC | 10.3.0 |
| 2011-06 | RP-52 | RP-110807 | 802 | 2 |  | Addition of OTDOA measurement requirement for E-UTRAN carrier aggregation | 10.3.0 |
| 2011-09 | RP-53 | RP-111246 | 863 |  |  | Thresholds and margins for reporting of neighbour cells in RRM test A.8.9.1 | 10.4.0 |
| 2011-09 | RP-53 | RP-111246 | 902 |  |  | Thresholds and margins for RRM tests A.5.2.1 and A.5.2.2 | 10.4.0 |
| 2011-09 | RP-53 | RP-111246 | 905 |  |  | Thresholds and margins for RRM tests A.5.2.4 and A.5.2.5 | 10.4.0 |
| 2011-09 | RP-53 | RP-111247 | 889 |  |  | Removing [] in clause 8.1.2.2.2.2 for Rel-10 | 10.4.0 |
| 2011-09 | RP-53 | RP-111247 | 915 |  |  | Adding condition of UTRA TDD measurement report delay requirements applied | 10.4.0 |
| 2011-09 | RP-53 | RP-111247 | 930 |  |  | Clarify time points and time duration for RLM tests A.7.3.x | 10.4.0 |
| 2011-09 | RP-53 | RP-111251 | 926 | 1 |  | Adding enhanced UTRA TDD cell identification requirements for Rel-10 | 10.4.0 |
| 2011-09 | RP-53 | RP-111251 | 969 |  |  | CR for E-UTRAN FDD GSM event triggered reporting in AWGN with enhanced BSIC identification in R10 | 10.4.0 |
| 2011-09 | RP-53 | RP-111252 | 894 |  |  | Requirements for RRC Connection Release with Redirection | 10.4.0 |
| 2011-09 | RP-53 | RP-111252 | 960 |  |  | Missing RSRQ in Intra-frequency measurement requirements | 10.4.0 |
| 2011-09 | RP-53 | RP-111252 | 965 | 1 |  | Requirements for RRC Connection Release with Redirection for TDD in R10 | 10.4.0 |
| 2011-09 | RP-53 | RP-111255 | 946 |  |  | Introduction of Band 22 | 10.4.0 |
| 2011-09 | RP-53 | RP-111255 | 979 | 1 |  | Modifications of Band 42 and 43 | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 879 | 1 |  | Correction to RRC connection mobility control in CA | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 895 | 2 |  | RSTD Measurement Requirements under Handover | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 896 | 2 |  | RSTD Measurement Requirements under Pcell Switching | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 920 | 1 |  | Editorial corrections for 36.133 (Rel-10) | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 924 | 1 |  | Correction to RRC connection mobility control in CA | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 927 |  |  | Modifications on TDD inter frequency measurements with autonomous gaps | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 945 | 1 |  | Frequency band related requirements to 36.133 | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 949 | 1 |  | Correction of references | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 950 |  |  | Alignment of the carrier aggregation terminology | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 951 |  |  | Band simplification for core requirements | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 952 |  |  | Clarification in inter-frequency RSTD accuracy tests | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 953 | 1 |  | Editorial corrections for RRM requirements | 10.4.0 |
| 2011-09 | RP-53 | RP-111263 | 961 |  |  | Missing RSRQ in E-UTRA carrier aggregation measurement requirements | 10.4.0 |
| 2011-09 | RP-53 | RP-111265 | 874 | 1 |  | Clarification of TDD uplink-downlink subframe configurations applicability for RSTD measurement in CA | 10.4.0 |
| 2011-09 | RP-53 | RP-111265 | 875 | 3 |  | CR on UE interruption requirements in SCC measurements with de-activated SCell when common DRX is used | 10.4.0 |
| 2011-09 | RP-53 | RP-111265 | 883 | 1 |  | Alignment of terminology for SCell measurement cycle | 10.4.0 |
| 2011-09 | RP-53 | RP-111265 | 921 | 1 |  | Introduction of Pcmax,c reporting requirements for carrier aggregation | 10.4.0 |
| 2011-09 | RP-53 | RP-111266 | 849 | 3 |  | RSTD Accuracy Requirements for Carrier Aggregation | 10.4.0 |
| 2011-09 | RP-53 | RP-111266 | 898 | 1 |  | Introduction of power headroom reporting requirement for carrier aggregation | 10.4.0 |
| 2011-09 | RP-53 | RP-111308 | 891 | 1 |  | RSRP and RSRQ measurement requirements for eICIC | 10.4.0 |
| 2011-12 | RP-54 | RP-111681 | 982 |  |  | Corrections of inter-frequency measurement accuracy RSRP and RSRQ test cases | 10.5.0 |
| 2011-12 | RP-54 | RP-111682 | 984 |  |  | Removing [] in CSFB requirement for Rel-10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 985 |  |  | Reference channel for RLM testing with eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 987 |  |  | Clarification on RSTD test cases | 10.5.0 |
| 2011-12 | RP-54 | RP-111690 | 988 |  |  | RSRP Measurement performance Io corrections | 10.5.0 |
| 2011-12 | RP-54 | RP-111686 | 989 |  |  | RLM measurement requirements for eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 990 |  |  | PDCCH/PCFICH transmission parameters for RLM | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 992 |  |  | Clarification on PRS bandwidth | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 993 |  |  | Missing RSRQ in intra-frequency measurement requirements for eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111686 | 994 | 1 |  | Test case for TDD RSRQ Accuracy for Carrier Aggregation | 10.5.0 |
| 2011-12 | RP-54 | RP-111686 | 995 |  |  | Cell identification requirements without DRX | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 997 | 1 |  | Test case for cell identification with eICIC in E-UTRAN FDD | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 998 | 1 |  | Test case for cell identification with eICIC in E-UTRAN TDD | 10.5.0 |
| 2011-12 | RP-54 | RP-111691 | 999 | 1 |  | Carrier aggregation RSRP measurement test case for TDD | 10.5.0 |
| 2011-12 | RP-54 | RP-111690 | 1001 |  |  | Test case for enhanced UTRA TDD cell identification for R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111690 | 1003 |  |  | Test case for RRC connection release redirection to UTRA TDD for R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1005 |  |  | Clarification of the Successful Percentage for Measurement Performance Requirements | 10.5.0 |
| 2011-12 | RP-54 | RP-111691 | 1007 | 2 |  | FDD Absolute and Relative RSRQ Accuracy test in CA | 10.5.0 |
| 2011-12 | RP-54 | RP-111691 | 1011 | 1 |  | FDD absolute and relative RSRP accuracies test in CA | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 1014 | 1 |  | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under time domain measurement resource restriction | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1016 |  |  | E-UTRAN FDD - UTRAN TDD enhanced cell identification test under AWGN propagation conditions in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1018 | 1 |  | E-UTRAN FDD RRC connection release with redirection to UTRAN TDD in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1021 | 1 |  | CR for Inter-RAT SI reading | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1022 |  |  | Addition of E-UTRAN FDD - TDD Inter frequency cell reselection test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1023 |  |  | Addtion of E-UTRAN TDD - FDD Inter frequency cell reselection test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1024 |  |  | Addtion of E-UTRAN FDD - TDD Inter frequency handover test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1025 |  |  | Addtion of E-UTRAN TDD - FDD Inter frequency handover test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1026 |  |  | Addtion of E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1027 | 1 |  | Addtion of E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111687 | 1028 |  |  | Addtion of E-UTRAN FDD - TDD inter frequency measurement accuracy test case | 10.5.0 |
| 2011-12 | RP-54 | RP-111681 | 1031 |  |  | Correction for the identification time in DRX for UTRA TDD in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1032 |  |  | Correction the side condition for SCH in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1033 | 1 |  | Correction to event triggered reporting for TS 36.133 in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111681 | 1039 | 1 |  | Correction of E-UTRAN TDD-TDD inter frequency handover test case in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1041 |  |  | Clarification of Expected RSTD and Expected RSTD uncertainty in RSTD test cases in R10 | 10.5.0 |
| 2011-12 | RP-54 | RP-111680 | 1043 |  |  | Thresholds and margins for RRM tests A.8.11.3 and A.8.11.4 | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1046 |  |  | Thresholds and margins for RRM tests A.8.11.5 and A.8.11.6 | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 1047 | 2 |  | RLM Out of Sync Detection Test for eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1049 |  |  | RRC Connection Release with Redirection from E-UTRAN FDD to GERAN | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 1051 |  |  | Colliding CRS in non-MBSFN ABS | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1052 |  |  | RRC Connection Release with Redirection from E-UTRAN TDD to GERAN | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 1053 | 1 |  | RLM In Sync Detection Test for FDD eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111693 | 1054 | 1 |  | RLM In Sync Detection Test for FDD eICIC | 10.5.0 |
| 2011-12 | RP-54 | RP-111691 | 1055 | 1 |  | FDD Event triggered reporting on deactivated Scell in non-DRX | 10.5.0 |
| 2011-12 | RP-54 | RP-111691 | 1056 | 1 |  | TDD Event triggered reporting on deactivated Scell in non-DRX | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1058 |  |  | Adding Band XX | 10.5.0 |
| 2011-12 | RP-54 | RP-111690 | 1061 | 1 |  | Optional faster higher priority reselection | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1064 | 1 |  | Addition of a test case at lower RSRP level for the serving cell measurement accuracy | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1066 |  |  | Test cases for RRC connection release with redirection to UTRAN FDD | 10.5.0 |
| 2011-12 | RP-54 | RP-111735 | 1072 |  |  | CA definition alignment in test cases | 10.5.0 |
| 2011-12 | RP-54 | RP-111683 | 1074 |  |  | Applicable PRS BW for RSTD accuracy requirements | 10.5.0 |
| 2012-03 | RP-55 | RP-120304 | 1077 | 1 |  | RSTD signalling modifications | 10.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1079 | 1 |  | Test case for E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided for R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1081 | 1 |  | Test case for E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided for R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120291 | 1084 |  |  | Thresholds and margins for E-UTRAN to C2K RRM reselection test cases (Rel-10) | 10.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1087 |  |  | Addition of E-UTRAN TDD-HRPD Cell Reselection: HRPD is of Lower Priority test case R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120293 | 1089 |  |  | Addition of E-UTRAN TDD-cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority test case R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120293 | 1091 |  |  | Addition of E-UTRAN TDD-HRPD Handover test case R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1093 |  |  | Addition of E-UTRAN TDD-cdma2000 1X Handover test case R10 | 10.6.010.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1099 |  |  | Addition of E-UTRAN FDD-TDD inter frequency RSRQ measurement accuracy test case R10 | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1112 | 1 |  | RLM test cases with SNRs for OOS and INS for E-UTRAN TDD in eICIC | 10.6.0 |
| 2012-03 | RP-55 | RP-120304 | 1115 |  |  | Io difference band-independent in Inter-frequency RSRP TDD TC A.9.1.4 | 10.6.0 |
| 2012-03 | RP-55 | RP-120292 | 1118 | 1 |  | Thresholds and margins in RRM test case A.8.11.4 | 10.6.0 |
| 2012-03 | RP-55 | RP-120292 | 1121 |  |  | TDD PRACH Test cases value of PRACH Configuration Index and first preamble power | 10.6.0 |
| 2012-03 | RP-55 | RP-120292 | 1124 | 1 |  | PDSCH and OCNG pattern in PRACH Test cases A.6.2.1 and A.6.2.3 | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1134 | 1 |  | Clarification of colliding CRS in MBSFN ABS | 10.6.0 |
| 2012-03 | RP-55 | RP-120304 | 1135 |  |  | Editorial corrections on the test cases of RRC connection release with redirection to UTRAN FDD | 10.6.0 |
| 2012-03 | RP-55 | RP-120304 | 1139 | 1 |  | Corrections on test case of Event triggered reporting on deactivated Scell in non-DRX CR not implemented as it is based on the wrong version of the spec | 10.6.0 |
| 2012-03 | RP-55 | RP-120304 | 1140 |  |  | Core requirements for E-UTRAN TDD inter-RAT UTRAN FDD SI acquisition using autonomous gaps | 10.6.0 |
| 2012-03 | RP-55 | RP-120304 | 1143 | 1 |  | Editorial corrections | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1145 | 1 |  | Side condition clarification for eICIC with MBSFN | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1146 |  |  | Clarification on reported cells with eICIC | 10.6.0 |
| 2012-03 | RP-55 | RP-120294 | 1148 |  |  | Correction of RSTD accuracy test cases for TDD | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1151 | 2 |  | RLM requirements with autonomous gaps | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1152 | 1 |  | SNR levels in out-of-sync RLM test cases for eICIC | 10.6.0 |
| 2012-03 | RP-55 | RP-120303 | 1156 | 1 |  | CR for 36.133: B41 REFSENS and MOP changes to accommodate single filter architecture | 10.6.0 |
| 2012-03 | RP-55 | RP-120300 | 1157 |  |  | eICIC measurement accuracy | 10.6.0 |
| 2012-03 | RP-55 | RP-120307 | 1154 | 1 |  | Introduction of Band 26/XXVI to TS 36.133 | 11.0.0 |
| 2012-06 | RP-56 | RP-120782 | 1162 |  |  | Resolve Band 41 omission between R4-120125 and R4-121106 | 11.1.0 |
| 2012-06 | RP-56 | RP-120770 | 1165 | 1 |  | Corrections to FDD-TDD Inter-freq RSRP measurement accuracy test case parameters | 11.1.0 |
| 2012-06 | RP-56 | RP-120771 | 1168 |  |  | OCNG and PDSCH for FDD-TDD event triggered reporting test cases | 11.1.0 |
| 2012-06 | RP-56 | RP-120771 | 1171 |  |  | RRC Connection Release with Redirection from E-UTRAN FDD to GERAN without System Information | 11.1.0 |
| 2012-06 | RP-56 | RP-120771 | 1174 |  |  | RRC Connection Release with Redirection from E-UTRAN TDD to GERAN without System Information | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1176 |  |  | OCNG Patterns for MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1183 |  |  | Addition of E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1186 |  |  | Addition of E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1189 |  |  | Addition of E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1192 |  |  | Addition of E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120777 | 1195 | 1 |  | Addition of E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1198 |  |  | Addition of E-UTRAN TDD-CDMA2000 1X event triggered reporting under fading propagation conditions test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120770 | 1201 |  |  | E-UTRA TDD RRC connection release redirection to UTRA FDD test without SI provided R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1205 | 1 |  | FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1207 | 1 |  | TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120780 | 1213 |  |  | CR to TS36.133 Corrections on RRC signalling in RLM test cases for eICIC | 11.1.0 |
| 2012-06 | RP-56 | RP-120773 | 1223 |  |  | Test case for event-triggered reporting on deactivated SCell with PCell interruption | 11.1.0 |
| 2012-06 | RP-56 | RP-120770 | 1227 | 1 |  | Finalization of Rel.9 cell reselection enhancement related test cases | 11.1.0 |
| 2012-06 | RP-56 | RP-120770 | 1231 |  |  | E-UTRAN FDD to UTRAN FDD RRC connection release with redirection test case when SI is not provided | 11.1.0 |
| 2012-06 | RP-56 | RP-120781 | 1233 |  |  | No interruptions on PCell at SCell activation/ deactivation when measCycleSCell is smaller than 640 ms | 11.1.0 |
| 2012-06 | RP-56 | RP-120780 | 1235 |  |  | Editorial corrections | 11.1.0 |
| 2012-06 | RP-56 | RP-120782 | 1237 | 1 |  | Reporting criteria requirements for carrier aggregation | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1239 |  |  | Cell identification requirements with DRX | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1241 | 1 |  | Phase II eICIC FDD: absolute and relative RSRP accuracies in non-MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1243 | 1 |  | Phase II eICIC TDD: absolute and relative RSRP accuracies in non-MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1249 |  |  | RLM requirements with autonomous gaps for DRX | 11.1.0 |
| 2012-06 | RP-56 | RP-120779 | 1251 |  |  | CR for 36.133: Aligning RSRQ measurement requirements in TS 36.133 with TS 36.101 regarding the modification of B41 REFSENS | 11.1.0 |
| 2012-06 | RP-56 | RP-120777 | 1260 |  |  | Bands 22, 23, 42 and 43 side conditions for inter-frequency measurements with autonomous gaps | 11.1.0 |
| 2012-06 | RP-56 | RP-120772 | 1261 |  |  | Clarification on UE Rx-Tx with eICIC | 11.1.0 |
| 2012-06 | RP-56 | RP-120767 | 1271 |  |  | sr-ConfigIndex in TDD DRX test cases | 11.1.0 |
| 2012-06 | RP-56 | RP-120782 | 1273 |  |  | Remove [ ] from eICIC RSRP, RSRQ Es/Iot side conditions | 11.1.0 |
| 2012-06 | RP-56 | RP-120764 | 1277 | 1 |  | RRM: Clarifications to the OCNG patterns | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1279 | 2 |  | Intra-Frequency FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1286 | 1 |  | eICIC FDD out-of-sync RLM test case in MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1288 | 1 |  | eICIC TDD out-of-sync RLM test case in MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120781 | 1289 | 1 |  | On UE behavior in the uplink subframe after measurement GAP | 11.1.0 |
| 2012-06 | RP-56 | RP-120773 | 1293 | 1 |  | Clarification on the number of monitoring layers for CA UEs | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1299 | 2 |  | CR on TDD RSRQ test case under Time Domain Measurement Resource Restriction with MBSFN ABS Rel11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1303 | 1 |  | In-Sync RLM test case in MBSFN ABS for E-UTRAN FDD R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1306 | 1 |  | In-Sync RLM test case in MBSFN ABS for E-UTRAN TDD R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120781 | 1310 |  |  | Inter-frequency and Inter-RAT Requirements for Measurements without Measurement Gaps | 11.1.0 |
| 2012-06 | RP-56 | RP-120788 | 1318 | 1 |  | The introduction of Multi-TA timing requirements R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120777 | 1320 | 1 |  | Addition of E-UTRAN FDD RSTD measurement accuracy test case in carrier aggregation R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120777 | 1322 |  |  | Addition of E-UTRAN TDD RSTD measurement accuracy test case in carrier aggregation R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120779 | 1328 |  |  | Correction to RLM requirements in eICIC with Autonomous gaps R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120769 | 1331 | 1 |  | Correction to E-UTRAN FDD/TDD - UTRAN FDD /TDD enhanced cell identification test case R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120770 | 1336 |  |  | Correction to E-UTRAN TDD redirection to UTRAN FDD test configuration R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120780 | 1337 | 1 |  | FDD CA RSTD Measurement Reporting Delay Test Case (Rel-11) | 11.1.0 |
| 2012-06 | RP-56 | RP-120782 | 1338 | 1 |  | TDD CA RSTD Measurement Reporting Delay Test Case (Rel-11) | 11.1.0 |
| 2012-06 | RP-56 | RP-120779 | 1342 |  |  | Correction to RSTD measurement reporting delay requirement in CA R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120795 | 1345 | 1 |  | Add Band 25 Io values R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120777 | 1347 | 1 |  | Clarification for cell identification condition in inter-RAT SI reading requirement R11 | 11.1.0 |
| 2012-06 | RP-56 | RP-120793 | 1349 |  |  | Introduction of Band 28 | 11.1.0 |
| 2012-06 | RP-56 | RP-120794 | 1350 | 1 |  | Introduction of Band 44 | 11.1.0 |
| 2012-06 | RP-56 | RP-120780 | 1355 |  |  | Editorial corrections | 11.1.0 |
| 2012-06 | RP-56 | RP-120766 | 1361 | 2 |  | Correction of a timer period in inter-frequency measurement tests | 11.1.0 |
| 2012-06 | RP-56 | RP-120764 | 1363 | 1 |  | UL Transmit Timing Requirements | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1364 | 2 |  | Phase IIbis eICIC FDD absolute and relative RSRP accuracy with MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1366 | 2 |  | Phase IIbis eICIC TDD absolute and relative RSRP accuracy with MBSFN ABS | 11.1.0 |
| 2012-06 | RP-56 | RP-120784 | 1368 |  |  | OCNG correction in Phase I eICIC test cases | 11.1.0 |
| 2012-06 | RP-56 | RP-120792 | 1379 |  |  | Introduction of e850_LB (Band 27) to TS 36.133 | 11.1.0 |
| 2012-09 | RP-57 | RP-121301 | 1385 |  |  | Identification of Cell 3 in RRM Test cases A.4.2.7 and A.4.2.8 | 11.2.0 |
| 2012-09 | RP-57 | RP-121301 | 1390 |  |  | Making FDD-TDD Inter-freq RSRQ measurement accuracy test case band-agnostic | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1392 |  |  | Thresholds and margins in RRM test cases A.8.16.1 and A.8.16.2 | 11.2.0 |
| 2012-09 | RP-57 | RP-121295 | 1398 | 1 |  | Modification of Handover Delay Requirement and Test Cases from E-UTRAN to cdma2000 1x (Rel-11) | 11.2.0 |
| 2012-09 | RP-57 | RP-121302 | 1400 |  |  | Correction to RSRP/RSRQ measurement accuracy tests in MBSFN R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1403 |  |  | Activation/ deactivation core requirement for carrier aggregation R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121313 | 1405 |  |  | Minor corrections for E-UTRAN â€“ GSM measurements without Measurement Gaps and Rx-Tx measurements when PCell is changed | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1407 | 3 |  | RRM requirements for CA REFSENSE (Rel-11) | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1409 |  |  | Square Bracket Removal for RSTD measurement requirement in Pcell changing and Handover R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1411 |  |  | Correction to the E-UTRAN secondary component carrier measurements when common DRX is used R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1413 |  |  | Requirements for Inter-frequency Measurements without Gaps when DRX is used R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121304 | 1415 |  |  | Clarification on TDD UL-DL subframe configurations in inter-frequency RSTD measurement without gaps R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121301 | 1418 |  |  | Correction for E-UTRA TDD RRC connection release redirection to UTRA TDD test case R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121340 | 1419 |  |  | Addition of E-UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps | 11.2.0 |
| 2012-09 | RP-57 | RP-121340 | 1420 |  |  | Addition of E-UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps | 11.2.0 |
| 2012-09 | RP-57 | RP-121301 | 1423 |  |  | Correction to E-UTRAN TDD-FDD Inter-frequency event triggered reporting test case R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121302 | 1432 |  |  | Alignment for ABS configurations in RRM Tests R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121294 | 1433 | 1 |  | Correction to RSRQ accuracy test cases R11 | 11.2.0 |
| 2012-09 | RP-57 | RP-121297 | 1438 |  |  | Radio conditions for PBCH reading in E-UTRA | 11.2.0 |
| 2012-09 | RP-57 | RP-121305 | 1444 |  |  | Introduction of inter-frequency/ RAT measurements in CA | 11.2.0 |
| 2012-09 | RP-57 | RP-121302 | 1449 |  |  | ABS signal transmission configuration for RRM tests | 11.2.0 |
| 2012-09 | RP-57 | RP-121340 | 1450 | 1 |  | Table format update for adding new bands | 11.2.0 |
| 2012-09 | RP-57 | RP-121301 | 1454 |  |  | Editorial correction RRM | 11.2.0 |
| 2012-12 | RP-58 | RP-121899 | 1458 | - |  | Random Access requirements for SCell | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1459 | - |  | Correction on CA TDD RSTD measurement accuracy test cases R11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121849 | 1461 | - |  | Correction to high priority cell measurement of UTRA TDD R11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1467 | - |  | Clarification of Test Requirements for CA RSRP, RSRQ Test Cases | 11.3.0 |
| 2012-12 | RP-58 | RP-121850 | 1470 | - |  | Remove [ ] from 10% requirement in RRM Test cases A.4.2.7 and A.4.2.8 | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1486 | 1 |  | Clean up for CA | 11.3.0 |
| 2012-12 | RP-58 | RP-121911 | 1487 | - |  | Clarification of CPICH RSCP side conditions | 11.3.0 |
| 2012-12 | RP-58 | RP-121867 | 1489 | - |  | Editorial corrections | 11.3.0 |
| 2012-12 | RP-58 | RP-121867 | 1497 | - |  | Band correction in RRM requirements | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1499 | - |  | Correction to RSTD Measurement Reporting Delay for Carrier Aggregation Test Cases | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1506 | - |  | Band-dependent RRM requirements for CA | 11.3.0 |
| 2012-12 | RP-58 | RP-121872 | 1507 | 1 |  | CR on RLM Requirements for FeICIC | 11.3.0 |
| 2012-12 | RP-58 | RP-121854 | 1516 | - |  | Correction of OCNG Patterns for UE Rx - Tx Time Difference Test Cases | 11.3.0 |
| 2012-12 | RP-58 | RP-121872 | 1517 | 1 |  | Cell identification requirements in FeICIC | 11.3.0 |
| 2012-12 | RP-58 | RP-121851 | 1522 | - |  | Time offset correction in CA test cases R11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121854 | 1529 | 1 |  | Clarification on RSTD measurement requirement under HO and Pcell changing | 11.3.0 |
| 2012-12 | RP-58 | RP-121910 | 1530 | 2 |  | Introduction the IDC requirements in 36.133 Rel-11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121849 | 1537 | - |  | Correction on test cases for handover to UTRAN TDD for Rel-11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121910 | 1542 | - |  | Updating RRM requirements in 36.133 | 11.3.0 |
| 2012-12 | RP-58 | RP-121867 | 1545 | - |  | Editorial corrections RRM | 11.3.0 |
| 2012-12 | RP-58 | RP-121852 | 1549 | - |  | Conditions in CSG reselection requirements | 11.3.0 |
| 2012-12 | RP-58 | RP-121852 | 1553 | - |  | Correcting inconsistency between inter-RAT UTRA measurements and requirements | 11.3.0 |
| 2012-12 | RP-58 | RP-121861 | 1555 | - |  | Refsens requirements for CA capable UE | 11.3.0 |
| 2012-12 | RP-58 | RP-121854 | 1558 | 1 |  | Intra-frequency RSTD accuracy requirements account for serving cell bandwidth | 11.3.0 |
| 2012-12 | RP-58 | RP-121854 | 1559 | 1 |  | Clarification on the total number of cells for RSTD inter-frequency measurement | 11.3.0 |
| 2012-12 | RP-58 | RP-121860 | 1561 | 1 |  | Clarification of the TDM pattern conditions | 11.3.0 |
| 2012-12 | RP-58 | RP-121873 | 1562 | 1 |  | MDT requirements in Rel-11 | 11.3.0 |
| 2012-12 | RP-58 | RP-121901 | 1563 | - |  | Introduction of Band 29 | 11.3.0 |
| 2012-12 |  |  |  |  |  | Editorial Correction | 11.3.1 |
| 2013-03 | RP-59 | RP-130268 | 1477 | 1 |  | Correction to Inter-frequency Measurements in CA mode test case R11 | 11.4.0 |
| 2013-03 | RP-59 | RP-130287 | 1480 | 1 |  | Requirements for RSRP and RSRQ for E-CID Positioning | 11.4.0 |
| 2013-03 | RP-59 | RP-130263 | 1566 |  |  | Secondary Component carrier levels for CA RSRP Test cases A.9.1.6 and A.9.1.7 | 11.4.0 |
| 2013-03 | RP-59 | RP-130263 | 1568 |  |  | Remove intra-frequency relative Requirement for CA RSRQ Test Cases | 11.4.0 |
| 2013-03 | RP-59 | RP-130263 | 1572 |  |  | Cell timing for CA RSRP and RSRQ Test cases | 11.4.0 |
| 2013-03 | RP-59 | RP-130277 | 1573 | 1 |  | Editorial correction for introduction of Band 29 | 11.4.0 |
| 2013-03 | RP-59 | RP-130263 | 1576 |  |  | Clarification of retuning interruption in single carrier operation | 11.4.0 |
| 2013-03 | RP-59 | RP-130260 | 1579 |  |  | RRM: RMC and OCNG pattern for FDD CGI test with autonomous gaps (Rel-11) | 11.4.0 |
| 2013-03 | RP-59 | RP-130268 | 1582 |  |  | Correction to CSG proximity requirement | 11.4.0 |
| 2013-03 | RP-59 | RP-130268 | 1584 |  |  | E-UTRAN FDD Proximity Indication RRM Requirements (Rel-11) | 11.4.0 |
| 2013-03 | RP-59 | RP-130275 | 1589 | 1 |  | Clarification of Cell Identification core requirement in FeICIC | 11.4.0 |
| 2013-03 | RP-59 | RP-130283 | 1591 | 1 |  | RSRP/RSRQ measurement accuracy requirements in FeICIC | 11.4.0 |
| 2013-03 | RP-59 | RP-130263 | 1598 |  |  | UE interruption requirements in SCC RSTD measurements with de-activated Scell R11 | 11.4.0 |
| 2013-03 | RP-59 | RP-130287 | 1602 |  |  | Timing offset correction in CA RSTD test cases | 11.4.0 |
| 2013-03 | RP-59 | RP-130280 | 1616 |  |  | Editorial corrections for IDC | 11.4.0 |
| 2013-03 | RP-59 | RP-130262 | 1618 |  |  | Editorial corrections for eICIC | 11.4.0 |
| 2013-03 | RP-59 | RP-130258 | 1622 |  |  | Editorial corrections RRM | 11.4.0 |
| 2013-03 | RP-59 | RP-130259 | 1627 |  |  | A clarification on measurement gap pattern in RSTD requirements | 11.4.0 |
| 2013-03 | RP-59 | RP-130268 | 1642 | 1 |  | Modification of PRS configuration for RSTD measurement reporting delay test cases(Rel-11) | 11.4.0 |
| 2013-03 | RP-59 | RP-130261 | 1644 |  |  | E-UTRAN FDD Proximity Indication Test Case (Rel-11) | 11.4.0 |
| 2013-06 | RP-60 | RP-130763 | 1648 |  |  | Correction to test parameters for combined E-UTRA - E-UTRA and GSM cell search - Rel 11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1649 |  |  | Remove the Brackets in cell identification of FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1657 |  |  | Clarification on inter-frequency RSTD measurement accuracy requirement R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1659 | 1 |  | RRM test configurations for 20MHz R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1668 | 1 |  | Corrections on RSTD measurement test cases (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1673 |  |  | Remove [ ] from GCI identification Test cases A.8.4.4 and A.8.4.5 | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1677 |  |  | Cell 1 levels for RSRP Test cases A.9.1.3 and A.9.1.4 | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1679 | 1 |  | RSRP, RSRQ RRM eICIC Test case cleanup | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1683 |  |  | Update on the GSM carrier RSSI measurement period when DRX is used | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1692 |  |  | sr-ConfigIndex in TDD-FDD Inter-frequency event triggered DRX Test case A.8.14.2 | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1694 |  |  | Testing of CA tests with multiple BW combinations | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1696 |  |  | Reference measurement channels for 20 MHz Tests | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1702 |  |  | Editorial corrections RRM | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1706 |  |  | Section numbering correction | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1708 | 1 |  | Editorial corrections for FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1709 | 1 |  | Removing an eICIC note on measurements | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1713 |  |  | Clean up for CA | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1716 |  |  | Editorial corrections in RSTD requirements | 11.5.0 |
| 2013-06 | RP-60 | RP-130766 | 1719 |  |  | SCell Activation Delay Requirements in CA | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1721 |  |  | Clarification on supported bandwidth combinations in RSTD requirements with CA | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1723 | 1 |  | Impact of REFSENS requirements on the core specification | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1724 |  |  | Correction of the total number of reporting criteria | 11.5.0 |
| 2013-06 | RP-60 | RP-130769 | 1728 | 1 |  | Condition clarification in MDT requirements | 11.5.0 |
| 2013-06 | RP-60 | RP-130769 | 1732 |  |  | Band 26 test cases corrections | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1739 |  |  | CR on Interruptions for Intra-band Non-contiguous Carrier Aggregation | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1744 |  |  | Time Alignment Timer in Test Case A.8.2.4 | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1745 |  |  | RRM: Adding required measurement gap | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1749 |  |  | TDD PRACH configuration index for Test Cases A.8.7.2, A.8.15.2 | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1752 |  |  | GSM cell list size for Test Cases A.6.3.10, A.6.3.11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1755 |  |  | Neighbour list for Test cases A.8.5.4, A.8.7.4, A.8.9.2 | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1758 |  |  | Additional corrections on intra-frequency RSTD test parameters (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1760 |  |  | Additional corrections on inter-frequency RSTD test parameters (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1762 |  |  | Phase I CA 20 MHz Tests: Event triggered reporting on deactivating Scells in non-DRX | 11.5.0 |
| 2013-06 | RP-60 | RP-130763 | 1767 |  |  | Corrections of E-UTRAN FDD CSG Proximity Indication Test Case (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1770 | 1 |  | In sync detection with CRS assistance information with non-MBSFN ABS in FDD | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1771 | 1 |  | In sync detection with CRS assistance information with non-MBSFN ABS in TDD | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1772 | 1 |  | E-UTRAN FDD RLM Out-of-sync Test of FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1773 | 1 |  | E-UTRAN TDD RLM Out-of-sync Test of FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1776 |  |  | E-UTRAN FDD absolute and relative RSRP accuracies for 20MHz in CA R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1778 |  |  | E-UTRAN TDD absolute and relative RSRP accuracies for 20MHz in CA R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1780 |  |  | Modification of OCNG patterns of RRM test configuration for 20MHz R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1782 |  |  | Clarification of Pcell in 36.133 R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1784 |  |  | FDD Absolute and relative RSRQ accuracies for CA with 20MHz BW (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1786 |  |  | TDD Absolute and relative RSRQ accuracies for CA with 20MHz BW (Rel-11) | 11.5.0 |
| 2013-06 | RP-60 | RP-130761 | 1790 |  |  | Correction on fading propagation condition for CA inter-RAT test cases R11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1791 |  |  | Clean up for band 44 | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1793 | 1 |  | E-UTRAN TDD UE Rx-Tx time difference test case in eICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1799 | 1 |  | Test case for UE Transmit Timing Accuracy for SCell | 11.5.0 |
| 2013-06 | RP-60 | RP-130767 | 1801 |  |  | CR on measurements without gaps | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1804 | 1 |  | Editorial corrections RRM | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1806 | 1 |  | Clarification for UE Rx-Tx with eICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1807 | 2 |  | Capturing RF requirements in the core specification | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1808 | 1 |  | Test case for UE Rx-Tx accuracy with eICIC in FDD | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1812 | 1 |  | RSRP and RSRQ relative accuracy requirements for FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130765 | 1814 | 1 |  | Adding clarification for begin and end of measurement GAP for Rel-11 | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1821 |  |  | Measurement requirements with interruptions due to CA | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1822 |  |  | Clarification on antenna ports in the measured and aggressor cells with FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1825 | 1 |  | UE Rx-Tx accuracy requirements with FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1826 |  |  | UE Rx-Tx measurement requirements with FeICIC | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1827 | 2 |  | Test case for cell identification with FeICIC in FDD | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1828 | 2 |  | Test case for cell identification with FeICIC in TDD | 11.5.0 |
| 2013-06 | RP-60 | RP-130770 | 1829 | 1 |  | Corrections on Wideband RSRQ inter-frequency accuracy requirements | 11.5.0 |
| 2013-06 | RP-60 | RP-130791 | 1769 | 1 |  | Introduction of Band 30 | 12.0.0 |
| 09-2013 | RP-61 | RP-131303 | 1830 | 1 |  | UTRAN FDD CPICH Ec/No measurement accuracy test for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131291 | 1832 |  |  | Correction on the test cases for UE Transmit Timing Accuracy for SCell (Rel-12) | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1836 |  |  | Corrections on RSTD CA test parameters (Rel-12) | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1839 |  |  | FDD: RSTD measurement reporting test cases for CA with 20MHz BW (Rel-12) | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1842 |  |  | TDD: RSTD measurement reporting test cases for CA with 20MHz BW (Rel-12) | 12.1.0 |
| 09-2013 | RP-61 | RP-131285 | 1844 |  |  | Timing and RSRP value corrections in Test cases A.9.2.6 and A.9.2.9 | 12.1.0 |
| 09-2013 | RP-61 | RP-131285 | 1846 |  |  | Corrections to Bands for 20MHz CA Test cases | 12.1.0 |
| 09-2013 | RP-61 | RP-131279 | 1854 |  |  | Cell time offset in TDD Inter-RAT test cases | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1855 |  |  | EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1860 |  |  | Rel-12 CRs on synchronization requirements for E-UTRA to CDMA 2000 handover | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1866 | 1 |  | Correct the SNR values for RLM tests with non-MBSFN ABS in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1869 | 1 |  | E-UTRAN FDD RSRP Measurement Accuracy Test in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1871 | 1 |  | E-UTRAN TDD RSRP Measurement Accuracy Test in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1873 |  |  | E-UTRAN FDD UE Rx-Tx Time difference test in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1875 |  |  | E-UTRAN TDD UE Rx-Tx Time difference test in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1881 |  |  | Clarification on UE Rx-Tx accuracy requirements in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1883 |  |  | Clarification on UE Rx-Tx measurement requirements in FeICIC R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1886 |  |  | Clarification on antenna port for timing and eCID test cases R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1889 | 1 |  | Addition of TDD serving cell measurement accuracy tests R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1890 |  |  | Introduction of Band 31 in 36.133 | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1891 |  |  | Addition of New OCNG Pattern for 5MHz | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1892 |  |  | E-UTRAN FDD intra-frequency RSRP measurement accuracy for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1893 |  |  | E-UTRAN FDD-FDD inter-frequency RSRP measurement accuracy for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1894 |  |  | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1895 |  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1896 |  |  | E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1897 |  |  | E-UTRAN FDD intra-frequency RRC re-establishment for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1898 |  |  | E-UTRAN FDD - Contention Based Random Access Test for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1899 | 1 |  | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1900 |  |  | E-UTRA FDD- UTRA FDD inter-RAT handover case for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1901 | 1 |  | E-UTRA FDD- UTRA FDD CPICH RSCP measurement accuracy issues | 12.1.0 |
| 09-2013 | RP-61 | RP-131285 | 1903 |  |  | Clarification of Refesens in WB-RSRQ sections of 36.133 R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1905 |  |  | Remove the brackets of FeICIC side conditions R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1908 | 1 |  | Test cases of E-UTRAN FDD RSTD Measurement Accuracy for Carrier Aggregation for 20MHz R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1913 | 1 |  | Test cases of E-UTRAN TDD RSTD Measurement Accuracy for Carrier Aggregation for 20MHz R12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1916 |  |  | Correction to SCH Es/Iot side condition for intra-frequency measurements under time domain measurement resource restriction with CRS assistance information | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1919 |  |  | E-UTRAN FDD " Non-contention Based Random Access Test for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1921 |  |  | Modification on the requirement for PCell interruption for Rel-12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1922 |  |  | E-UTRAN FDD " Timing Advance Accuracy Test for 5MHz bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1928 |  |  | Phase II CA 20 MHz Tests: Event triggered reporting on deactivating SCell and and interruption probability without DRX | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1945 | 1 |  | CR on Applicability of 5MHz Test Cases | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1946 | 1 |  | E-UTRAN FDD Radio Link Monitoring Test for In-Sync for 5MHz | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1947 |  |  | E-UTRAN FDD Intra-frequency handover test for 5MHz Channel Bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1948 |  |  | E-UTRAN FDD Intra-frequency RSRQ Accuracy Test for 5MHz Channel Bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131293 | 1952 |  |  | Editorial corrections RRM | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1954 |  |  | E-UTRAN FDD Inter-frequency RSRQ Accuracy Test for 5MHz Channel Bandwidth | 12.1.0 |
| 09-2013 | RP-61 | RP-131293 | 1955 |  |  | Clarification of CGI reading requirements | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1958 | 2 |  | E-UTRAN FDD Radio Link Monitoring Test for In-Sync for 5MHz with DRX | 12.1.0 |
| 09-2013 | RP-61 | RP-131285 | 1961 |  |  | Editorial corrections in capturing RF requirements | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1964 |  |  | Clarification on tests for multiple bandwidths | 12.1.0 |
| 09-2013 | RP-61 | RP-131282 | 1969 |  |  | CR on PCell interrutptions | 12.1.0 |
| 09-2013 | RP-61 | RP-131283 | 1970 |  |  | Time stamp accuracy for RLF and handover failure reporting with eMDT | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1971 |  |  | FDD reference measurement channels for 5 MHz tests | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1972 |  |  | Part II RRM tests: UE intra-frequency measurements with synchronous cells in DRX FDD | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1973 |  |  | Part II RRM tests: E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1978 |  |  | Correction of cell identification test case with FeICIC | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1984 |  |  | RLM requirements correction | 12.1.0 |
| 09-2013 | RP-61 | RP-131284 | 1988 |  |  | Clarification on antenna ports in the measured and aggressor cells for UE Rx-Tx with FeICIC | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1990 |  |  | FeICIC FDD Test for In-sync With MBSFN ABS for Rel. 12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131290 | 1992 |  |  | FeICIC TDD Test for In-sync With MBSFN ABS for Rel. 12 | 12.1.0 |
| 09-2013 | RP-61 | RP-131303 | 1993 |  |  | Correction of the SNR value of Out of sync RLM test for 5MHz | 12.1.0 |
| 12-2013 | RP-62 | RP-131927 | 1996 |  |  | Corrections to CA event triggered tests on deactivated SCell with PCell interruption in non-DRX (Rel-12) | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2003 |  |  | Corrections to CA Interruption Requirements | 12.2.0 |
| 12-2013 | RP-62 | RP-131926 | 2009 |  |  | CRS Es/Iot for eICIC RSRP, RSRQ with MBSFN ABS Test Cases | 12.2.0 |
| 12-2013 | RP-62 | RP-131941 | 2010 |  |  | Correction to RSTD measurement accuracy side condition for Band 31 | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2013 |  |  | Amendment on SCell Activation Delay Requirements for other activation actions | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2016 |  |  | Amendment on SCell Activation Delay Requirements in case no RS for measurement | 12.2.0 |
| 12-2013 | RP-62 | RP-131936 | 2019 |  |  | Correction to the SNR values for RLM tests with MBSFN ABS in FeICIC R12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131936 | 2023 |  |  | Correction for the RSRP/RSRQ test cases in FeICIC R12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2031 | 1 |  | CR on PCell Interruptions For Inter-band CA During Measurements | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2039 |  |  | Introduction of E-UTRAN TDD WB-RSRQ test case R12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131925 | 2044 |  |  | Correction of Proximity Indication Test Case Not implemented as it is not based on the latest version of the spec | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2053 |  |  | Clarifications for intra-band non-contiguous CA R12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2058 |  |  | Inter-frequency WB-RSRQ FDD test case | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2071 |  |  | Clarification on Pcell Interruption shall not occur before SF n+5 | 12.2.0 |
| 12-2013 | RP-62 | RP-131925 | 2078 |  |  | Correction in RSTD requirements | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2080 |  |  | Editorial corrections RRM | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2084 | 1 |  | Band simplification | 12.2.0 |
| 12-2013 | RP-62 | RP-131931 | 2091 |  |  | Requirements clarification under different BWs in FeICIC | 12.2.0 |
| 12-2013 | RP-62 | RP-131931 | 2095 |  |  | Correction in cell search FeICIC test cases | 12.2.0 |
| 12-2013 | RP-62 | RP-131936 | 2097 | 1 |  | Correct ABS pattern for FeICIC for In-sync with MBSFN ABS for Rel. 12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131926 | 2104 |  |  | Correction to Test cases A.9.2.9 and A.9.2.10 | 12.2.0 |
| 12-2013 | RP-62 | RP-131942 | 2106 | 1 |  | Bands applicability in RSRP, RSRQ FDD-FDD Inter frequency tests for 5MHz Bandwidth | 12.2.0 |
| 12-2013 | RP-62 | RP-131925 | 2111 |  |  | Corrections to CGI Reading in Autonomous Gap | 12.2.0 |
| 12-2013 | RP-62 | RP-131936 | 2123 |  |  | Remove the brackets of SNR values in RLM test cases in FeICIC R12 | 12.2.0 |
| 12-2013 | RP-62 | RP-131967 | 2129 |  |  | Correction on RMC pattern for 5MHz UE Transmit Timing Accuracy Tests | 12.2.0 |
| 12-2013 | RP-62 | RP-131928 | 2135 |  |  | CSI Reporting in SCell Activation Requirements | 12.2.0 |
| 12-2013 | RP-62 | RP-131927 | 2143 |  |  | Editorial corrections RRM | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2145 |  |  | Applying band simplification | 12.2.0 |
| 12-2013 | RP-62 | RP-131939 | 2151 |  |  | Correction to MTA requirements | 12.2.0 |
| 12-2013 | RP-62 | RP-131925 | 2155 |  |  | Correction in RSTD test cases | 12.2.0 |
| 12-2013 | RP-62 | RP-131931 | 2157 |  |  | Correction to interference clarification in FeICIC requirements | 12.2.0 |
| 03-2014 | RP-63 | RP-140389 | 2236 |  |  | Band simplification clean up | 12.3.0 |
| 03-2014 | RP-63 | RP-140368 | 2234 |  |  | Missing condition in CGI identification requirements | 12.3.0 |
| 03-2014 | RP-63 | RP-140368 | 2224 |  |  | CSI Reporting in SCell Activation Requirements | 12.3.0 |
| 03-2014 | RP-63 | RP-140368 | 2258 |  |  | Alignment between interruption requirements for RSTD and mobillity measurements for SCell | 12.3.0 |
| 03-2014 | RP-63 | RP-140367 | 2263 |  |  | Correction of Proximity Indication Test Case | 12.3.0 |
| 03-2014 | RP-63 | RP-140380 | 2259 |  |  | Addition of new OCNG pattern for E-UTRA TDD with 5MHz bandwidth | 12.3.0 |
| 03-2014 | RP-63 | RP-140380 | 2260 |  |  | Addition of new RMC for E-UTRA TDD with 5MHz bandwidth | 12.3.0 |
| 03-2014 | RP-63 | RP-140380 | 2261 |  |  | Addition of OCNG pattern for E-UTRA FDD with 5MHz bandwidth without MBSFN | 12.3.0 |
| 03-2014 | RP-63 | RP-140381 | 2169 |  |  | Updates on test case A.9.1.17 FDD—FDD Inter frequency case for 5MHz Bandwidth for R12 | 12.3.0 |
| 03-2014 | RP-63 | RP-140389 | 2170 |  |  | Correction on the SNR values of in-sync RLM test for 5MHz | 12.3.0 |
| 03-2014 | RP-63 | RP-140371 | 2200 | 1 |  | Clarification of BW applicability in Rx-Tx Time Difference measurement R12 | 12.3.0 |
| 03-2014 | RP-63 | RP-140389 | 2182 |  |  | Clarification on FDD reference measurement channels for 5 MHz tests | 12.3.0 |
| 03-2014 | RP-63 | RP-140368 | 2181 |  |  | Correction on PDSCH allocation in PRS subframe r12 | 12.3.0 |
| 03-2014 | RP-63 | RP-140367 | 2192 |  |  | PRS_RA corrections | 12.3.0 |
| 06-2014 | RP-64 | RP-140650 | 2331 | 3 |  | Introduction of test cases for 5MHz +5MHz : absolute and relative RSRQ accuracies in CA for FDD and TDD The CR was not implemented as it contained the wrong content. | 12.4.0 |
| 06-2014 | RP-64 | RP-140743 | 2366 | 1 |  | SCell activation and deactivation delay test case for known SCell | 12.4.0 |
| 06-2014 | RP-64 | RP-140910 | 2312 |  |  | Clarification on UE Transmit Timing Accuracy test cases in DRX mode R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140910 | 2267 |  |  | RRM: Clean-up of time offset between cells in RSTD tests (Rel-12) | 12.4.0 |
| 06-2014 | RP-64 | RP-140910 | 2354 |  |  | RSTD inter-frequency requirements applicability | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2382 |  |  | RRM: Remove square brackets from eICIC RLM test requirement (Rel-12) | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2379 |  |  | Correction to periodicity of ABS pattern in eICIC RRM test cases | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2315 |  |  | Correction for OCNG pattern number in RRM tests R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2302 |  |  | Introduce the CGI reading requirements in CA R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2360 | 1 |  | Test case corrections for eICIC | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2278 |  |  | Removing DPCH for handover from E-UTRAN to UTRA TDD for Rel-12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2422 |  |  | Clean up the correction on PDSCH allocation in PRS subframe R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140911 | 2319 |  |  | Clarification on E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140914 | 2416 |  |  | Correction to PCI configuration conditions in FeICIC tests R12 | 12.4.0 |
| 06-2014 | RP-64 | RP-140914 | 2338 |  |  | CQI feedback periodicity correction for RLM in eICIC/FeICIC test setup | 12.4.0 |
| 06-2014 | RP-64 | RP-140916 | 2307 |  |  | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for Scell in sTAG | 12.4.0 |
| 06-2014 | RP-64 | RP-140916 | 2340 | 1 |  | Test case for RACH on SCell | 12.4.0 |
| 06-2014 | RP-64 | RP-140916 | 2306 |  |  | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for Scell in sTAG | 12.4.0 |
| 06-2014 | RP-64 | RP-140918 | 2357 |  |  | Editorial corrections RRM | 12.4.0 |
| 06-2014 | RP-64 | RP-140918 | 2364 |  |  | Clean up for Band 29 | 12.4.0 |
| 06-2014 | RP-64 | RP-140918 | 2445 |  |  | Removing square brackets in FeICIC test cases | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2387 |  |  | E-UTRAN FDD RSTD measurement reporting in carrier aggregation for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2388 |  |  | E-UTRAN TDD RSTD measurement reporting in carrier aggregation for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2389 |  |  | E-UTRAN FDD RSTD measurement accuracy in CA for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2390 |  |  | E-UTRAN TDD RSTD measurement accuracy in CA for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2290 |  |  | E-UTRAN FDD absolute and relative RSRP accuracies in CA for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140923 | 2291 |  |  | E-UTRAN TDD absolute and relative RSRP accuracies in CA for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140926 | 2339 |  |  | Introduction of Band 32/XXXII | 12.4.0 |
| 06-2014 | RP-64 | RP-140928 | 2394 | 1 |  | Introduce RRM measurement requirements for eIMTA | 12.4.0 |
| 06-2014 | RP-64 | RP-140928 | 2396 | 1 |  | Inter frequency measurements using autonomous gaps | 12.4.0 |
| 06-2014 | RP-64 | RP-140930 | 2374 | 1 |  | RRM requirements for TDD-FDD CA | 12.4.0 |
| 06-2014 | RP-64 | RP-140937 | 2412 | 1 |  | Introduction of test cases for 5MHz +5MHz : RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth | 12.4.0 |
| 06-2014 | RP-64 | RP-140937 | 2330 | 1 |  | Introduction of test cases for 5MHz +5MHz : absolute and relative RSRP accuracies in CA for FDD and TDD | 12.4.0 |
| 06-2014 | RP-64 | RP-140937 | 2410 | 1 |  | Introduction of test cases for 5MHz +5MHz : RSTD Measurement Reporting Test Case | 12.4.0 |
| 06-2014 | RP-64 | RP-140937 | 2332 | 2 |  | Introduction of test cases for 5MHz +5MHz : Event triggered reporting on deactivating Scells in non-DRX FDD and TDD | 12.4.0 |
| 06-2014 | RP-64 | RP-140937 | 2415 | 1 |  | Introduction of test cases for 5MHz +5MHz : E-UTRA event triggered reporting on deactivated SCell with PCell interruption in non-DRX | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2294 |  |  | E-UTRAN TDD absolute and relative RSRQ accuracies in CA for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2385 |  |  | E-UTRAN FDD Event triggered reporting on deactivating Scells and interruption probability (0.5%) without DRX for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2386 |  |  | E-UTRAN TDD Event triggered reporting on deactivating Scells and interruption probability (0.5%) without DRX for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2292 |  |  | E-UTRAN FDD absolute and relative RSRQ accuracies in CA for 5MHz+10MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2289 |  |  | E-UTRAN TDD Event triggered reporting under deactivated Scell in non-DRX for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140939 | 2288 |  |  | E-UTRAN FDD Event triggered reporting under deactivated Scell in non-DRX for 10MHz+5MHz | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2384 |  |  | Correct Correlation Matrix and Antenna Configuration for RRM test cases A.8 | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2346 | 1 |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2383 |  |  | Correct Correlation Matrix and Antenna Configuration for RRM test cases A.4, A.7 | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2336 | 2 |  | New Test Case for UE Transmit Timing Accuracy requirements in DRX | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2268 | 1 |  | UE Behaviour after Measurement Gap | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2345 | 1 |  | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | 12.4.0 |
| 06-2014 | RP-64 | RP-140945 | 2419 |  |  | Editorial correction for band 31 in 36.133 | 12.4.0 |
| 06-2014 | RP-64 | RP-140959 | 2395 | 2 |  | Introduce RSRP/RSRQ measurement accuracy requirements for 3DL CA | 12.4.0 |
| 06-2014 | RP-64 | RP-140959 | 2376 | 3 |  | Introduce the support of 3DL CA to TS 36.133 Section 7.8 "Interruptions with Carrier Aggregation" | 12.4.0 |
| 06-2014 | RP-64 | RP-140959 | 2375 | 2 |  | Introduce the support of 3DL CA to TS 36.133 Section 7.1 "UE transmit timing" | 12.4.0 |
| 06-2014 | RP-64 | RP-140959 | 2373 | 2 |  | SCell activation and deactivation delay requirements for 3 DL CA | 12.4.0 |
| 09-2014 | RP-65 | RP-141526 | 2527 |  |  | Tolerance levels for measurements on UTRAN | 12.5.0 |
| 09-2014 | RP-65 | RP-141530 | 2474 |  |  | Correction to periodicity of ABS pattern in feICIC RRM test cases | 12.5.0 |
| 09-2014 | RP-65 | RP-141531 | 2515 | 1 |  | Maximum transmission timing difference | 12.5.0 |
| 09-2014 | RP-65 | RP-141536 | 2502 |  |  | Introduction of test cases for 5MHz +5MHz : absolute and relative RSRQ accuracies in CA for FDD and TDD | 12.5.0 |
| 09-2014 | RP-65 | RP-141539 | 2481 |  |  | Modification on E-UTRAN event triggered reporting under deactivated SCell for 20 MHz bandwidth | 12.5.0 |
| 09-2014 | RP-65 | RP-141545 | 2523 | 2 |  | Introduction of BeaconRSSI measurements for WLAN/3GPP Radio Interworking | 12.5.0 |
| 09-2014 | RP-65 | RP-141554 | 2492 |  |  | Interruptions on Activated Serving Cells for 3DL CA | 12.5.0 |
| 09-2014 | RP-65 | RP-141554 | 2495 |  |  | Requirements for UE Measurements Procedures in RRC_CONNECTED State for 3DL CA | 12.5.0 |
| 09-2014 | RP-65 | RP-141562 | 2454 | 1 |  | Correction of values in RSTD tests | 12.5.0 |
| 09-2014 | RP-65 | RP-141562 | 2457 |  |  | Clarification to RSTD CA Reporting Delay tests | 12.5.0 |
| 09-2014 | RP-65 | RP-141562 | 2480 | 1 |  | Clarification on UE bahavior considering max transmit timing difference between TAGs R12 | 12.5.0 |
| 09-2014 | RP-65 | RP-141562 | 2496 | 1 |  | Applicability of requirements | 12.5.0 |
| 09-2014 | RP-65 | RP-141562 | 2510 |  |  | Note to clarify that certain requirements do not apply to band 32 | 12.5.0 |
| 09-2014 | RP-65 | RP-141700 | 2471 | 3 |  | Clarification for ACK/NACK feedback of CGI measurement | 12.5.0 |
| 12-2014 | RP-66 | RP-142176 | 2484 | 2 |  | Introducing measurement accuracy requirements for UE category 0 in TS36.133 Clause 9 | 12.6.0 |
| 12-2014 | RP-66 | RP-142176 | 2506 | 3 |  | Measurements requirements for UE category 0 with 1 Rx | 12.6.0 |
| 12-2014 | RP-66 | RP-142143 | 2534 | - |  | Correction of PRS Signal Levels in RSTD Reporting Tests | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2538 | - |  | Correction of Es/Noc values in inter-frequency RSTD tests | 12.6.0 |
| 12-2014 | RP-66 | RP-142174 | 2547 | 1 |  | Introduction of PDSCH FRC for TDD UL-DL configuration 0 | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2553 | 1 |  | Clarification on time to identify the target UTRA TDD cell for blind redirection from E-UTRA to UTRA TDD | 12.6.0 |
| 12-2014 | RP-66 | RP-142174 | 2555 | 1 |  | CR on inter frequency RSRP test case for eIMTA | 12.6.0 |
| 12-2014 | RP-66 | RP-142174 | 2556 | 1 |  | CR on inter frequency RSRQ test case for eIMTA | 12.6.0 |
| 12-2014 | RP-66 | RP-142147 | 2566 | - |  | Correction to ABS pattern and CRS Es/Iot in feICIC RRM test cases | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2569 | - |  | SCell activation and deactivation delay test case for unknown SCell R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142157 | 2573 | 2 |  | Clarification on cell identification for TDD config 0 | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2585 | 1 |  | RSRQ accuracy test case in TDD-FDD CA when Pcell is FDD R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2586 | 1 |  | RSRQ accuracy test case in TDD-FDD CA when Pcell is TDD R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142147 | 2597 | - |  | Correction on Io value in CA 20MHz RSRQ test case R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142163 | 2598 | - |  | Correction on Io value in CA 10MHz+5MHz RSRQ test case R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2599 | - |  | Range increase for RSRQ | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2606 | 1 |  | Clarification of parallel reporting criteria (E-UTRA) | 12.6.0 |
| 12-2014 | RP-66 | RP-142164 | 2611 | 1 |  | Interruptions with RSTD Measurements for 3DL CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142164 | 2614 | - |  | RRM requirements for RSTD in 3 DL CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2619 | 1 |  | RSRP accuracy test cases for TDD-FDD CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142176 | 2630 | - |  | SI reading requirements for UE category 0 with 1 Rx in FDD, TDD and HD-FDD | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2639 | - |  | Changes to RSTD CA Reporting Delay tests | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2640 | - |  | Revision of RSRP absolute accuracy requirements in Rel-12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2644 | - |  | Clarifications to RSTD values | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2656 | - |  | Correction to RSTD Intra Frequency Delay Test Case | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2665 | - |  | Correction on autonomous time adjustment in MTAG case | 12.6.0 |
| 12-2014 | RP-66 | RP-142176 | 2666 | - |  | Introduce RLM requirements for LC-MTC in TS36.133 | 12.6.0 |
| 12-2014 | RP-66 | RP-142174 | 2669 | 1 |  | Introducing test case for TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | 12.6.0 |
| 12-2014 | RP-66 | RP-142179 | 2670 | 1 |  | Introducing requirements for small cell enhancement in TS36.133 | 12.6.0 |
| 12-2014 | RP-66 | RP-142180 | 2671 | 2 |  | Introducing interruption requirements for dual connectivity into TS36.133 | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2674 | - |  | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2675 | - |  | E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2676 | - |  | E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2677 | - |  | TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2678 | 1 |  | TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142162 | 2679 | - |  | E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz | 12.6.0 |
| 12-2014 | RP-66 | RP-142143 | 2682 | 1 |  | Introducing positioning enhancement requirement for UE Rx-Tx accuracy | 12.6.0 |
| 12-2014 | RP-66 | RP-142144 | 2686 | - |  | Correction on CA test cases in R12 | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2687 | - |  | Correction on E-UTRAN TDD – Non-Contention Based Random Access Test For Scell | 12.6.0 |
| 12-2014 | RP-66 | RP-142179 | 2688 | 1 |  | Introduction of RSRP measurement accuracy requirement for DRS based measurement | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2690 | 1 |  | Ecat clarification for iRAT | 12.6.0 |
| 12-2014 | RP-66 | RP-142180 | 2694 | - |  | CR for TS36.133 on Cell phase accuracy for Dual Connectivity | 12.6.0 |
| 12-2014 | RP-66 | RP-142180 | 2695 | 1 |  | Introduction of RRM requirements for Dual Connectivity | 12.6.0 |
| 12-2014 | RP-66 | RP-142180 | 2696 | 1 |  | Introduction of measurement requirements for Dual Connectivity | 12.6.0 |
| 12-2014 | RP-66 | RP-142178 | 2697 | 1 |  | Measurement and reporting of BLER in section 9 | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2698 | 1 |  | Introduction of TDD-FDD CA test cases | 12.6.0 |
| 12-2014 | RP-66 | RP-142178 | 2699 | 1 |  | CR on measurement for MBSFN MDT | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2707 | 1 |  | PCell Interruption in Rel-12 CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142158 | 2708 | 1 |  | UE Behaviour after Measurement Gap in CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2709 | 1 |  | CA RRM Testing for Multiple Duplex Modes | 12.6.0 |
| 12-2014 | RP-66 | RP-142177 | 2710 | 1 |  | CA RRM Testing for Fall back CA Configuration | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2712 | 1 |  | Introduction of High Doppler measurement accuracy requierments | 12.6.0 |
| 12-2014 | RP-66 | RP-142172 | 2714 | 1 |  | Requirements for increased carrier monitoring for idle mode 36.133 | 12.6.0 |
| 12-2014 | RP-66 | RP-142172 | 2715 | 1 |  | Requirements for increased carrier monitoring in RRC connected state 36.133 | 12.6.0 |
| 12-2014 | RP-66 | RP-142161 | 2716 | 1 |  | Different TDD configurations in CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142178 | 2722 | 1 |  | MBMS requirements in section 9 | 12.6.0 |
| 12-2014 | RP-66 | RP-142179 | 2725 | 1 |  | Intra-frequency and inter-frequency measurement accuracy requirements with DMTC | 12.6.0 |
| 12-2014 | RP-66 | RP-142188 | 2727 | - |  | RSTD accuracy requirements for smaller and larger bandwidths | 12.6.0 |
| 12-2014 | RP-66 | RP-142149 | 2736 | - |  | Corrections to E-UTRAN TDD RLM In-sync under Time Domain Measurement Resource Restriction with CRS assistance information | 12.6.0 |
| 12-2014 | RP-66 | RP-142149 | 2738 | - |  | Corrections to E-UTRAN TDD RLM Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information | 12.6.0 |
| 12-2014 | RP-66 | RP-142149 | 2740 | - |  | Test case for inter-RAT HO to multicarrier UTRA | 12.6.0 |
| 12-2014 | RP-66 | RP-142178 | 2741 | - |  | CR on parallel reporting criteria for eMBMS | 12.6.0 |
| 12-2014 | RP-66 | RP-142186 | 2742 | - |  | Introduction of 2UL non-contiguous intra-band CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142021 | 2743 | - |  | Introduction of 2UL inter-band CA | 12.6.0 |
| 12-2014 | RP-66 | RP-142150 | 2745 | - |  | Requirements for multicarrier handover from EUTRA to UTRA | 12.6.0 |
| 03-2015 | RP-77 | RP-150387 | 2747 | - |  | CR to Correct Implementation Error in FDD RSTD Measurement Reporting Delay Test Case and to Update Io Levels for Certain RSTD Test Cases | 12.7.0 |
| 03-2015 | RP-77 | RP-150382 | 2750 | - |  | Remove incorrect note from CA RSTD Accuracy tests | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2751 | - |  | Change Nprs value for 5MHz CA RSTD Accuracy tests | 12.7.0 |
| 03-2015 | RP-77 | RP-150066 | 2754 | 1 |  | Maximum allowed layers for multiple monitoring for CA | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2756 | - |  | DRX correction for interruption with dual connectivity | 12.7.0 |
| 03-2015 | RP-77 | RP-150388 | 2757 | - |  | Correction of Interruptions with RSTD Measurements for 3DL CA | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2761 | 1 |  | RRM requirements for ProSe | 12.7.0 |
| 03-2015 | RP-77 | RP-150396 | 2763 | 1 |  | Updating the requirements applicability for TDD config 0 | 12.7.0 |
| 03-2015 | RP-77 | RP-150394 | 2764 | 1 |  | Cleanup for RSRQ measurement requirement for SCE | 12.7.0 |
| 03-2015 | RP-77 | RP-150394 | 2774 | 1 |  | Clean up the correction on discovery signal measurements | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2775 | 1 |  | Correction on MBSFN measurements | 12.7.0 |
| 03-2015 | RP-77 | RP-150394 | 2776 | - |  | Introduce CA measurement accuracy requirements for SCE | 12.7.0 |
| 03-2015 | RP-77 | RP-150382 | 2777 | - |  | Correction on Io in carrier aggregation test cases | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2783 | - |  | Introducing accuracy requirement for new RSRQ | 12.7.0 |
| 03-2015 | RP-77 | RP-150384 | 2785 | - |  | Time-domain measurement resource restriction pattern for serving cell in feICIC RSRP and RSRQ test cases | 12.7.0 |
| 03-2015 | RP-77 | RP-150384 | 2791 | - |  | CR on typo of referencing section name in CA measurements | 12.7.0 |
| 03-2015 | RP-77 | RP-150393 | 2797 | 1 |  | Clarification including PSCell in Note 1 for Ecat | 12.7.0 |
| 03-2015 | RP-77 | RP-150386 | 2798 | 1 |  | Clarification of IncMon requirements for E-UTRA idle state | 12.7.0 |
| 03-2015 | RP-77 | RP-150386 | 2799 | 1 |  | Clarification of IncMon requirements for E-UTRA connected state | 12.7.0 |
| 03-2015 | RP-77 | RP-150386 | 2800 | 1 |  | Clarification concerning IncMon scaling for non-gap-assisted measurements | 12.7.0 |
| 03-2015 | RP-77 | RP-150382 | 2803 | - |  | Correction of RMC and OCNG pattern in event triggered tests without measurement gap | 12.7.0 |
| 03-2015 | RP-77 | RP-150394 | 2804 | - |  | CR on RSRQ requirements for CRS based discovery signal | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2808 | - |  | Correction to RRM test cases | 12.7.0 |
| 03-2015 | RP-77 | RP-150388 | 2809 | - |  | Correction to CA Testing with Different CA Configurations | 12.7.0 |
| 03-2015 | RP-77 | RP-150393 | 2811 | - |  | Principle to test synchronous and asynchronous DC requirements | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2814 | - |  | Further revision of RSRP requirement for 36.133 release 12 | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2815 | - |  | Additional bandwidths for EUTRAN activation and deactivation of known and unknown SCell in non-DRX | 12.7.0 |
| 03-2015 | RP-77 | RP-150387 | 2816 | 1 |  | High Doppler measurement accuracy requirements | 12.7.0 |
| 03-2015 | RP-77 | RP-150384 | 2817 | 1 |  | 36.133 CR to change CPICH Ec/No to CPICH Ec/Io in EUTRA FDD to UTRA FDD HO test cases | 12.7.0 |
| 03-2015 | RP-77 | RP-150388 | 2822 | - |  | Maximum Transmission Timing Difference in 3DL CA | 12.7.0 |
| 03-2015 | RP-77 | RP-150053 | 2824 | - |  | Correction to the implementation of CR 2471r3 (Clarification for ACK/NACK feedback of CGI measurement) | 12.7.0 |
| 06-2015 | RP-68 | RP-150972 | 2825 |  |  | 3 DL CA Phase I tests # 1-2: Event triggered reporting tests with deactivated SCells in non-DRX for TDD-FDD CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2828 |  |  | RMC for 10 MHz for UE category 0 RRM tests | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2829 |  |  | Correction to measurement scaling factor for incmon | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2832 |  |  | RSRP requierment for SCE | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2833r1 | 1 |  | CR on FDD-FDD inter-frequency absolute and relative CRS RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2834r1 | 1 |  | CR on TDD-TDD inter-frequency absolute and relative CRS RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2835r1 | 1 |  | CR on FDD absolute and relative CSI-RSRP accuracy test case for E-UTRAN Carrier Aggregation | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2836r1 | 1 |  | CR on TDD absolute and relative CSI-RSRP accuracy test case for E-UTRAN Carrier Aggregation | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2837r1 | 1 |  | CR on FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2838r1 | 1 |  | CR on TDD-TDD inter-frequency absolute and relative CSI-RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2839r1 | 1 |  | CR on FDD intra frequency absolute and relative CSI-RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2840r1 | 1 |  | CR on TDD intra frequency absolute and relative CSI-RSRP accuracy test case | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2842r1 | 1 |  | Intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2843r1 | 1 |  | Absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2845r1 | 1 |  | SCE FDD intra-frequency absolute RSRQ accuracy | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2846r1 | 1 |  | SCE TDD intra-frequency absolute RSRQ accuracy | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2847 |  |  | SCE FDD absolute RSRQ accuracy for CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2848 |  |  | SCE TDD absolute RSRQ accuracy for CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2849r1 | 1 |  | Test for CGI acqusition requirements for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2850r1 | 1 |  | Test for cell identification for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2851 |  |  | Test for handover requirements for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2852 |  |  | Test for RRC re-establishment requirements for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2853r1 | 1 |  | HD-FDD handover requirements for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2855r1 | 1 |  | Correction of requirements for ProSe in DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2857r1 | 1 |  | E-UTRAN FDD intra frequency CRS based discovery signal measurements when DRX is used | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2858r1 | 1 |  | E-UTRAN TDD intra frequency CRS based discovery signal measurements when DRX is used | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2859r1 | 1 |  | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2860r1 | 1 |  | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2861r1 | 1 |  | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2862r1 | 1 |  | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2866r1 | 1 |  | RSRP accuracy FD-FDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2867r1 | 1 |  | RSRP accuracy HD-FDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2868r1 | 1 |  | RSRP accuracy TDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2869r1 | 1 |  | RSRQ accuracy FD-FDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2870r1 | 1 |  | RSRQ accuracy HD-FDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2871r1 | 1 |  | RSRQ accuracy TDD Intra frequency case for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2872r1 | 1 |  | Test case for 3DL CA: PCell in FDD: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (TDD-FDD CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2873r1 | 1 |  | Test case for 3DL CA: PCell in TDD: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (TDD-FDD CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2874r1 | 1 |  | Test case for 3DL CA: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (FDD CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150968 | 2875r1 | 1 |  | Test case for 3DL CA: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (TDD 3 DL CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2880 |  |  | OTDOA RSTD Measurements on different secondary component carriers | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 2884 |  |  | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2885 |  |  | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 2886 |  |  | E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2887 |  |  | E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2897 |  |  | Further clarification of MBMSBLER reporting in section 9 | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2903r1 | 1 |  | Test case of FDD-FDD inter-frequency RSRQ measurement accuracy in discovery signal occasions | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2904 |  |  | CR on side conditions for inter-frequency measurement for SCE | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2905 |  |  | CR on test case for RSRQ TDD-TDD inter frequency measurement accuracy requirement for SCE | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 2906r1 | 1 |  | Maximum Rx difference between Pcell and Scell in section 7.9 | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2908r1 | 1 |  | FDD-FDD intra frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2909r1 | 1 |  | TDD-TDD intra frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2910r1 | 1 |  | FDD-FDD inter frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2911r1 | 1 |  | TDD-TDD inter frequency event triggered reporting in DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2912r1 | 1 |  | FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2913r1 | 1 |  | TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2915r2 | 2 |  | CR of DC interruption requirements | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2916r1 | 1 |  | Event triggered reporting on deactivated SCells in non-DRX (FDD CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2917r1 | 1 |  | Event triggered reporting on deactivated SCells in non-DRX (TDD CA) | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2919r1 | 1 |  | Introduction of RRM test case for E-UTRAN TDD-FDD 3 DL CA activation and deactivation of known SCell in non-DRX with PCell in FDD | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2920r1 | 1 |  | Introduction of RRM test case for E-UTRAN TDD-FDD 3 DL CA activation and deactivation of known SCell in non-DRX with PCell in TDD | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2921 |  |  | 3DL CA Phase I tests #11_3DL FDD CA SCell activation and deactivation for known SCells without DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2921a |  |  | Correction of implementation of CR 2644 in Table A.9.8.1.1-1 | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2922 |  |  | 3DL CA Phase I tests #12_3DL TDD CA SCell activation and deactivation for known SCells without DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2922ar2 | 2 |  | Incmon CR for FDD-FDD Interfrequency correct reporting of measurement events without reduced performance group configured, non DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2923r2 | 2 |  | Incmon CR for TDD-TDD Interfrequency correct reporting of measurement events without reduced performance group configured, non DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2928r1 | 1 |  | FDD RLM Test Case for Out-of-sync in DRX for PSCell in asynchronous DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2929r1 | 1 |  | FDD RLM Test Case for In-sync in DRX for PSCell in asynchronous DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150954 | 2932 |  |  | Correction of Cell Time offset in RSTD CA Test cases (Rel-12) | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2933r1 | 1 |  | Introduction of DC intra-frequency event triggered reporting with DRX in synchronous FDD DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2934r1 | 1 |  | Introduction of DC intra-frequency event triggered reporting with DRX in synchronous TDD DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2935r1 | 1 |  | Introduction of DC intra-frequency event triggered reporting with DRX in asynchronous FDD DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2936r1 | 1 |  | Introduction of DC inter-frequency event triggered reporting with DRX in synchronous FDD DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2937r1 | 1 |  | Introduction of DC inter-frequency event triggered reporting with DRX in synchronous TDD DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2938r1 | 1 |  | Testcases for E-UTRA Incmon idle interfrequency reselection | 12.8.0 |
| 06-2015 | RP-68 | RP-150962 | 2940r2 | 2 |  | CR on minimum number of subframes for discovery-based measurements | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2941r1 | 1 |  | E-UTRAN FD-FDD Radio Link Monitoring Tests for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2942r1 | 1 |  | E-UTRAN HD-FDD Radio Link Monitoring Tests for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2943r1 | 1 |  | E-UTRAN TDD Radio Link Monitoring Tests for UE category 0 | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2944r1 | 1 |  | Absolute and relative RSRP accuracies in FDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150968 | 2945r1 | 1 |  | Absolute and relative RSRP accuracies in TDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2946r1 | 1 |  | PCell in FDD: absolute and relative RSRQ accuracies in TDD-FDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2947r1 | 1 |  | PCell in TDD: absolute and relative RSRQ accuracies in TDD-FDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2950 |  |  | FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured for non DRX IncMon | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2951 |  |  | TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured for non DRX IncMon | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2952r1 | 1 |  | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2953r1 | 1 |  | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150961 | 2954r1 | 1 |  | E-UTRAN FDD PCell interruption at transitions between active and non-active when DRX is used in PSCell in asynchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2955 |  |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 2956 |  |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20 MHz +20 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2957 |  |  | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20 MHz +10 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 2958 |  |  | E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2959 |  |  | E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz bandwidth R12 | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2962 |  |  | Addition PDSCH RMC for 5MHz with user data | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 2967r1 | 1 |  | 3 DL CA Phase II tests # 1-2: RSRP measurement accuracies for TDD-FDD CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2971r1 | 1 |  | PSCell Add and Release Delay Tests for Synchronous DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2972r1 | 1 |  | PSCell Add and Release Delay Tests for Asynchronous DC | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2975r1 | 1 |  | Idle mode FDD to UTRA FDD interRAT reselection | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2976r1 | 1 |  | Idle mode TDD to UTRA FDD interRAT reselection | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2977 |  |  | E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2978 |  |  | E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2979r1 | 1 |  | E-UTRAN FDD PCell interruption at transitions between active and non-active when DRX is used in PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2980r1 | 1 |  | E-UTRAN TDD PCell interruption at transitions between active and non-active when DRX is used in PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2981r1 | 1 |  | E-UTRAN FDD inter-frequency event triggered reporting in asynchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 2984r1 | 1 |  | Modification for interruption period for SCell (de-)activation with 3DL | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2987r1 | 1 |  | Test cases of Idle mode E-UTRA to UTRA TDD interRAT cell reselection for IncMon | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 2988r1 | 1 |  | Test cases of Interfrequency correct reporting of measurement events with reduced performance group configured, DRX | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2989r2 | 2 |  | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX for PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150963 | 2990r2 | 2 |  | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for PSCell in synchronous dual connectivity | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 2992 |  |  | CR on interruption during D2D discovery for D2D single RF chain | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2993 |  |  | CR on E-UTRAN TDD-TDD inter frequency measurements when DRX is used | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2998 |  |  | Test case of FDD-FDD inter-frequency new RSRQ measurement accuracy | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 2999 |  |  | Test case of TDD-TDD inter-frequency new RSRQ measurement accuracy | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 3001 |  |  | Correction to feICIC cell configurations in RLM | 12.8.0 |
| 06-2015 | RP-68 | RP-150955 | 3003 |  |  | Correction to A.8.1.8 | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3004r1 | 1 |  | CR on absolute and relative RSRQ accuracies in TDD 3DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3005r1 | 1 |  | CR on absolute and relative RSRQ accuracies in FDD 3DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 3006 |  |  | CR for test case of new RSRQ measurement accuracy in FDD | 12.8.0 |
| 06-2015 | RP-68 | RP-150965 | 3007 |  |  | CR for test case of new RSRQ measurement accuracy in TDD | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3008r1 | 1 |  | RSTD measurement reporting in FDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3009r1 | 1 |  | RSTD measurement reporting in TDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3010r1 | 1 |  | RSTD measurement accuracy in FDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150972 | 3011r1 | 1 |  | RSTD measurement accuracy in TDD 3 DL CA | 12.8.0 |
| 06-2015 | RP-68 | RP-150964 | 3012r2 | 2 |  | Clarification of ProSe requirements in ONC | 12.8.0 |
| 06-2015 | RP-68 | RP-150957 | 3013 |  |  | Correction to Asynchronous Requirements for DC for only FDD-FDD | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 3014 |  |  | E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured non DRX IncMon | 12.8.0 |
| 06-2015 | RP-68 | RP-150959 | 3015 |  |  | E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured non DRX for IncMon | 12.8.0 |
| 06-2015 | RP-68 | RP-150958 | 3016 |  |  | Correction to E-UTRA TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz + 10 MHz | 12.8.0 |
| 06-2015 | RP-68 | RP-150969 | 2893 |  |  | Carrier aggragation test cases for band 31 | 13.0.0 |
| 06-2015 | RP-68 | RP-150974 | 2966 |  |  | 4DL CA RRM requirements for “UE Measurements Procedures in RRC_CONNECTED State” | 13.0.0 |
| 06-2015 | RP-68 | RP-150974 | 2970 | 1 |  | RRM Requirements in Section 7 for 4 DL CA | 13.0.0 |
| 09-2015 | RP-69 | RP-151475 | 3020 | - |  | Correction of Ior/Ioc value in RRM Test case A.4.3.1.1 | 13.1.0 |
| 09-2015 | RP-69 | RP-151479 | 3022 | - |  | Cleanup of 3DL CA RRM Test cases | 13.1.0 |
| 09-2015 | RP-69 | RP-151483 | 3031 | - |  | Time offset between cells | 13.1.0 |
| 09-2015 | RP-69 | RP-151497 | 3032 | - |  | Requirements for DC on ACK/NACK reporting for measurements using autonomous gaps | 13.1.0 |
| 09-2015 | RP-69 | RP-151475 | 3034 | - |  | Interruptions at overlapping addition/release/activation/deactivation of SCells | 13.1.0 |
| 09-2015 | RP-69 | RP-151504 | 3035 | - |  | RRM Requirements for 3 DL/2UL Inter-band CA | 13.1.0 |
| 09-2015 | RP-69 | RP-151483 | 3037 | - |  | CR on editorial corrections in TS36133 in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151478 | 3039 | - |  | CR on item title of table in clause 8.1.2.4.5.1 in TS36133 in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151500 | 3041 | - |  | 3DL CA Phase II tests #15_ SCell activation and deactivation for unknown SCells without DRX (FDD 3 DL CA) in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151500 | 3043 | - |  | 3DL CA Phase II tests #16_SCell activation and deactivation for unknown SCells without DRX (TDD 3 DL CA) in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151475 | 3045 | - |  | Modifying test case of E-UTRAN 2DL TDD CA activation of unknown SCell in non-DRX in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151480 | 3047 | - |  | CR on delete note in table 8.5.2.1.6.1-1 in TS36133 in Rel-13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151479 | 3052 | - |  | Correction of inconsistency in 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX | 13.1.0 |
| 09-2015 | RP-69 | RP-151479 | 3054 | - |  | CR on Interruptions at PSCell Addition/release | 13.1.0 |
| 09-2015 | RP-69 | RP-151483 | 3062 | - |  | Corrections to the RMC configurations in 36.133 R13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151479 | 3064 | - |  | Remove the Brackets in RLM Tests for UE category 0 R13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151474 | 3066 | 1 |  | Adding SNR values to DC RLM test cases R13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151486 | 3068 | - |  | Correction on Band 31 test cases R13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151483 | 3070 | - |  | Correction to UE transmit timing accuracy tests R13 | 13.1.0 |
| 09-2015 | RP-69 | RP-151500 | 3078 | - |  | Introduction of RRM test case for E-UTRAN TDD-FDD 3DL CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD | 13.1.0 |
| 09-2015 | RP-69 | RP-151483 | 3079 | - |  | Modifying test case of E-UTRAN 2DL FDD CA activation of unknown SCell in non-DRX | 13.1.0 |
| 09-2015 | RP-69 | RP-151500 | 3080 | - |  | Introduction of RRM test case for E-UTRAN TDD-FDD 3DL CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD | 13.1.0 |
| 12-2015 | RP-70 | RP-152131 | 3086 | - |  | Correction of RSRQ value in RRM Serving Cell Test cases A.9.9.1, A.9.9.2 | 13.2.0 |
| 12-2015 | RP-70 | RP-152136 | 3088 | - |  | Remove brackets in RSTD measurement accuracy R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3090 | - |  | Remove bracket for CSI-RSRP measurement R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3094 | - |  | Correction to E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3096 | - |  | Correction to E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3098 | - |  | Correction to E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3100 | - |  | Correction to E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3102 | - |  | Correction to RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3104 | - |  | Correction to RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3106 | - |  | Correction to RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3108 | - |  | Correction to RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152136 | 3114 | - |  | Alignment of UE reporting criteria requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152131 | 3116 | - |  | Removal of square brackets for some CA requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3118 | - |  | Cleanup of 3DL CA RRM Test cases | 13.2.0 |
| 12-2015 | RP-70 | RP-152134 | 3121 | - |  | Correction of definition of antenna connection in some RSTD tests | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3129 | - |  | Different TDD configurations for OTDOA in CA in release 12 | 13.2.0 |
| 12-2015 | RP-70 | RP-152157 | 3131 | - |  | Introduction of Band 67 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3136 | - |  | Correction of definition of pTAG and psTAG | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3145 | - |  | Title of new section A.7.4 in TS36.133 for Rel-13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3146 | - |  | SNR levels and Reference channels for DC RLM test cases for Rel-13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152131 | 3150 | - |  | Correction on measurement category for reporting criteria | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3167 | - |  | Alignment of dB values for 2DL CA activation and deactivation Test cases | 13.2.0 |
| 12-2015 | RP-70 | RP-152135 | 3173 | - |  | CR on editorial cleanup for D2D RRM requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152136 | 3178 | - |  | Correction to Trstd values in 3DL RSTD Measurement Accuracy test cases | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3183 | - |  | Update of 2DL CA activation and deactivation of unknown SCell Test cases A.8.16.19+A.8.16.20 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3185 | - |  | Update of 3DL CA activation and deactivation of unknown SCell Test cases A.8.16.41+A.8.16.42 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3187 | - |  | Update to RRM test case for E-UTRAN TDD-FDD 3DL CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3189 | - |  | Update to RRM test case for E-UTRAN TDD-FDD 3DL CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD | 13.2.0 |
| 12-2015 | RP-70 | RP-152136 | 3191 | - |  | Correction to Cells in OTDOA assistance data in 3DL RSTD Measurement Reporting Delay test cases | 13.2.0 |
| 12-2015 | RP-70 | RP-152153 | 3192 | 2 |  | CR on RS-SINR accuracy in 36.133 | 13.2.0 |
| 12-2015 | RP-70 | RP-152136 | 3195 | 1 |  | Correction on RSRQ measurement report mapping R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152173 | 3196 | - |  | Introduction of Band 45 into TS36.133 | 13.2.0 |
| 12-2015 | RP-70 | RP-152131 | 3211 | - |  | Further Correction of Cell Time offset in RSTD CA test cases (Rel-13) | 13.2.0 |
| 12-2015 | RP-70 | RP-152155 | 3213 | 1 |  | CR for clearification of band combination in 36133 | 13.2.0 |
| 12-2015 | RP-70 | RP-152166 | 3214 | 1 |  | CR for RRM requirement up to 3UL CA in 36133 | 13.2.0 |
| 12-2015 | RP-70 | RP-152152 | 3216 | 1 |  | CR on measurement performance requirements for UE reporting SSTD between MeNB and SeNB for dual connectivity enhancements | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3223 | - |  | Adding the title of A.8.22 in TS 36.133 R13 | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3227 | - |  | Correction on A.8.16.17 E-UTRAN FDD activation and deactivation of known SCell in non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3229 | - |  | Correction on A.8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3231 | - |  | Correction on A.8.16.35 3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3233 | - |  | Correction on A.8.16.36 3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3235 | - |  | Correction on A.8.16.37 3DL FDD CA activation and deactivation of known SCell in non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3237 | - |  | Correction on A.8.16.38 3DL TDD CA activation and deactivation of known SCell in non-DRX | 13.2.0 |
| 12-2015 | RP-70 | RP-152152 | 3238 | 1 |  | CR on RRM requirements for SSTD reporting for Dual Connectivity | 13.2.0 |
| 12-2015 | RP-70 | RP-152152 | 3239 | 1 |  | CR on maximum uplink timing difference for Dual Connectivity | 13.2.0 |
| 12-2015 | RP-70 | RP-152152 | 3240 | 1 |  | CR on RRM requirements for 3 DL CC Dual connectivity | 13.2.0 |
| 12-2015 | RP-70 | RP-152152 | 3241 | 1 |  | CR on requirements of interruption for 3 DL CC Dual connectivity | 13.2.0 |
| 12-2015 | RP-70 | RP-152163 | 3243 | 1 |  | CR on RRM requirements for 5DL CC CA | 13.2.0 |
| 12-2015 | RP-70 | RP-152171 | 3250 | - |  | Introduction of Band 65 | 13.2.0 |
| 12-2015 | RP-70 | RP-152172 | 3251 | - |  | Introduction of Band 66 | 13.2.0 |
| 12-2015 | RP-70 | RP-152149 | 3253 | 2 |  | SCell activation/deactivation delay for PUCCH SCell | 13.2.0 |
| 12-2015 | RP-70 | RP-152146 | 3256 | 1 |  | Beacon RSSI Reporting Requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152148 | 3260 | 1 |  | Core RRM requirements for LAA | 13.2.0 |
| 12-2015 | RP-70 | RP-152148 | 3262 | 1 |  | LAA measurement requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152148 | 3263 | 1 |  | LAA measurement accuracy requirements and measurement report mapping | 13.2.0 |
| 12-2015 | RP-70 | RP-152153 | 3265 | - |  | RS-SINR measurement requirements | 13.2.0 |
| 12-2015 | RP-70 | RP-152153 | 3268 | 1 |  | RS-SINR measurement report mapping | 13.2.0 |
| 12-2015 | RP-70 | RP-152154 | 3269 | 1 |  | Extended DRX requirements in RRC_CONNECTED state | 13.2.0 |
| 12-2015 | RP-70 | RP-152135 | 3275 | - |  | CR on ProSe UE transmission timing in Any Cell Selection State | 13.2.0 |
| 12-2015 | RP-70 | RP-152133 | 3279 | - |  | Alignment of time when UE starts CSI reporting for activated SCell | 13.2.0 |
| 03/2016 | RP-71 | RP-160472 | 3317 |  | B | RRM requirements for eMTC in IDLE mode in section 4 | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3355 |  | B | Reference configuration for Rel-13 MTC | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3346 | 1 | B | CR on eMTC positioning | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3369 | 1 | B | Random access requirements for eMTC UEs | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3331 | 1 | F | CR on CGI reading of eMTC | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3354 | 1 | B | Radio link monitoring for Rel-13 MTC UE | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3353 | 1 | B | Measurement accuracy requirements for Rel-13 MTC UE | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3352 | 1 | B | Measurement requirements for Rel-13 MTC UE under normal coverage | 13.3.0 |
| 03/2016 | RP-71 | RP-160472 | 3356 | 1 | B | Measurement requirements for Rel-13 MTC UE under enhanced coverage | 13.3.0 |
| 03/2016 | RP-71 | RP-160473 | 3285 | 1 | B | CR on eD2D RRM requirements: Inter-freq discovery and multicarrier D2D | 13.3.0 |
| 03/2016 | RP-71 | RP-160473 | 3286 | 1 | B | CR on eD2D RRM requirements: UE-NW relays | 13.3.0 |
| 03/2016 | RP-71 | RP-160473 | 3284 | 1 | B | CR on eD2D RRM requirements: OOC Discovery | 13.3.0 |
| 03/2016 | RP-71 | RP-160473 | 3310 | 2 | F | CR of measurement performance on eD2D | 13.3.0 |
| 03/2016 | RP-71 | RP-160475 | 3347 | 1 | B | Correction to SSTD measurement accuracy and reporting range | 13.3.0 |
| 03/2016 | RP-71 | RP-160476 | 3366 | 1 | B | Measurement requirements in RRC CONNECTED state | 13.3.0 |
| 03/2016 | RP-71 | RP-160476 | 3367 | 2 | B | Measurement requirements in RRC IDLE state | 13.3.0 |
| 03/2016 | RP-71 | RP-160477 | 3341 | 1 | F | CR on RSSI measurement | 13.3.0 |
| 03/2016 | RP-71 | RP-160477 | 3351 | 1 | B | RSSI Report Mapping Requirements | 13.3.0 |
| 03/2016 | RP-71 | RP-160478 | 3362 | 1 | F | Corrections in measurement accuracy requirements for LAA | 13.3.0 |
| 03/2016 | RP-71 | RP-160488 | 3321 |  | A | Correction to feICIC TDD RSRP accuracy OCNG in TS 36.133 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3283 |  | A | CR for correction to syncOffsetIndicator parameter in D2D resource pool configuration | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3288 |  | A | Change OGNG for 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions, A.8.16.32+A.8.16.33 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3293 |  | F | Correction of errors in Annex A Activation/Deactivation Test cases | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3323 |  | A | CR on E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD for Rel-13 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3326 |  | A | CR on E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD for Rel-13 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3344 |  | A | Correction on SCE requirements and test cases R13 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3349 |  | A | Correction to antenna configuration principle | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3372 |  | A | CR for IncMon requirements alignment 36.133 Rel-13 | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3373 |  | A | Editorial corrections | 13.3.0 |
| 03/2016 | RP-71 | RP-160489 | 3295 | 1 | F | Modification for MBSFN measurements for R13 | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3290 |  | F | Correction to RS-SINR measurement accuracy requirements | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3297 |  | F | Modification for interruptions with Carrier Aggregation | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3305 |  | F | Clarification on timing of interruption for PUCCH SCell activation/deactivation | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3316 |  | F | Activation and deactivation delay requirements for PUCCH SCell with four downlink SCells | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3339 |  | F | CR on maximum UL transmission time difference for R13 DC | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3357 |  | F | Interruptions on RSTD in CA | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3303 | 1 | F | Clarification of SSTD measurement requirements | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3361 | 1 | F | Corrections for intra-frequency measurement requirements for LAA | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3312 | 1 | F | CR on measurement and measurement accuracy for LAA | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3330 | 2 | F | Correction on LAA measurement conditions | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3363 | 2 | F | Reporting criteria for RS-SINR | 13.3.0 |
| 03/2016 | RP-71 | RP-160490 | 3327 | 2 | F | Modification on the SCell activation delay requirement for deactivated SCell under Frame Structure 3 | 13.3.0 |
| 2016/06 | RP-72 | RP-161128 | 3374 | 1 | B | CR: MPDCCH RMCs for Cat-M1 RRM Tests | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3375 | 1 | B | CR: PDSCH RMCs for Cat-M1 RRM Tests | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3377 | 3 | F | CR: Intra-frequency handover requirements for Cat-M1 UEs in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3379 | 1 | F | CR: UE transmit timing Requirements for Cat-M1 UEs | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3384 |  | F | Removal of conditions for intra-frequency relative RSRQ measurement accuracy requirements under operation with frame structure 3 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3385 |  | F | Radio Link Monitoring Test for In-sync in DRX for PSCell | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3393 |  | F | Introduction of test cases for E-UTRAN FDD-TDD and TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3394 |  | F | CR: Correction to Band 66 notes in E-UTRA band groups | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3395 |  | F | New test cases: E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD/PCell in TDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3396 | 1 | F | New test cases: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (FDD 4 DL CA and TDD 4 DL CA) | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3397 | 1 | F | New test cases: Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (FDD 5 DL CA and TDD 5 DL CA) | 13.4.0 |
| 2016/06 | RP-72 | RP-161134 | 3402 | 1 | F | CR on measurement reference channel and OCNG with FS3 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3404 |  | A | CR on minimum ProSe SCH_RP condition on FDD_F | 13.4.0 |
| 2016/06 | RP-72 | RP-161130 | 3405 |  | F | CR on minimum ProSe SCH_RP condition on FDD_F | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3406 |  | F | CR of typo in interruption requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3408 |  | A | Editorial CR in RSRQ test case for CA in CRS based discovery signal | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3411 |  | F | CR on TDD-FDD DC Radio Link Monitoring Test for Out-of-sync | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3418 |  | F | Editorial correction in RRM requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3421 | 1 | F | Reporting criteria for LAA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3423 |  | F | Channel occupancy measurement requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3428 |  | F | Inter-frequency RSSI measurement requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161134 | 3429 | 1 | B | RSRP and RSRQ accuracy requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3430 |  | F | Editorial correction in LAA requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3432 |  | F | RS-SINR accuracy requirements with CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3433 | 1 | F | RS-SINR measurement requirements with CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3434 |  | F | 4 DL PCell in FDD Event triggered reporting on deactivated SCells in non-DRX # 3 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3435 |  | F | 4 DL PCell in TDD Event triggered reporting on deactivated SCells in non-DRX # 4 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3436 |  | F | 5 DL PCell in FDD Event triggered reporting on deactivated SCells in non-DRX # 1 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3437 |  | F | 5 DL PCell in TDD Event triggered reporting on deactivated SCells in non-DRX # 2 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3439 |  | A | Correction on E-UTRAN TDD-FDD CA activation and deactivation of known/unknown SCell in non-DRX with PCell in FDD for Rel-13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3442 |  | A | CR on E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD for Rel-13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3444 |  | A | CR on E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD for Rel-13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3446 |  | A | Corrections on PDSCH RMC for UE category 0 R13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3450 | 1 | F | 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3451 | 1 | F | 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3452 | 1 | F | 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3453 | 1 | F | 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3455 |  | F | RSTD CA interruption on SCC in release 13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161134 | 3460 | 1 | B | CR on testing principle of Carrier Aggregation under operation with Frame 3 with Different Duplex Modes | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3466 | 1 | F | E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3467 | 1 | F | E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3474 | 2 | F | maintenance on radio link monitoring for Rel-13 MTC UE | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3475 | 2 | F | Maintenance on measurement requirements for Rel-13 MTC UE under normal coverage | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3476 | 2 | F | Maintenance on measurement requirements for Rel-13 MTC UE under CEModeB | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3478 | 1 | F | CR for eMTC RLM | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3479 |  | F | New Test cases: E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD/PCell in TDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3483 |  | B | Measurement accuracy requirements for Rel-13 MTC UE | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3484 | 1 | F | Timing requirements for eMTC | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3487 |  | A | Physical channels undefined in RRM Test cases A.9.1.22, A.9.1.23 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3489 |  | A | Cleanup of Dual Connectivity RRM Test cases | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3494 | - | A | Corrections to values for 3DL RSTD test cases | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3497 | - | A | Removal of duplicated parameter from 3DL RSTD reporting delay test cases | 13.4.0 |
| 2016/06 | RP-72 | RP-161129 | 3498 | 2 | B | 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3502 | 1 | F | Rx-Tx time difference reporting CR not implemented: Not based on the latest version of the spec. | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3506 | - | A | Corrections in A.8.16.12, A.8.16.21, A.8.16.22, A.8.16.30, A.9.1.15 and A.9.1.37 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3510 | - | A | Editorial corrections | 13.4.0 |
| 2016/06 | RP-72 | RP-161134 | 3512 | - | B | CSI-RSRP measurement accuracy requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3515 | - | F | A clarification on LAA band | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3516 | - | F | Editorial corrections for LAA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3519 | 1 | F | Inter-frequency measurement requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3521 | 2 | F | eMTC requirements with eDRX in RRC_CONNECTED | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3526 | - | F | CR on correction for section number of A.7.3.29 | 13.4.0 |
| 2016/06 | RP-72 | RP-161139 | 3528 | - | A | CR on correction for test cases in A.8.16.17x Rel-13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3530 | - | F | Editral correction for title in section A.8 and A.9 Rel-13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3533 | - | F | 3DL/3UL TDD CA - UE Transmit Timing Accuracy Tests for 2SCells | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3534 | 1 | F | Test cases for E-UTRAN DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3535 | 1 | B | UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161168 | 3536 | - | F | E-UTRAN 4DL CA activation and deactivation of know SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161168 | 3537 | - | F | E-UTRAN 5DL CA activation and deactivation of know SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3539 | - | A | PCC and SCC assignment in 20MHz+10MHz test case A.9.1.24 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3541 | 1 | F | Modifications on LAA SCell activation delay requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161133 | 3544 | 1 | B | Antenna connection method for RLM and RRM tests with 4RX | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3548 | 1 | B | Dual connectivity enhancements test case : SSTD accuracy | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3549 | - | B | Dual connectivity enhancements test case : SSTD delay with DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3550 | - | B | Dual connectivity enhancements test case : SSTD delay in non DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3553 | - | F | Absolute and relative RSRP accuracies in FDD 4DL CA# 3 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3554 | - | F | Absolute and relative RSRP accuracies in TDD 4DL CA# 4 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3555 | 1 | F | PCell in FDD: absolute and relative RSRP accuracies in TDD-FDD 5 DL CA # 1 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3556 | 1 | F | PCell in TDD: absolute and relative RSRP accuracies in TDD-FDD 5 DL CA # 2 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3557 | - | F | Absolute and relative RSRQ accuracies in 5 DL FDD CA # 7 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3558 | - | F | Absolute and relative RSRQ accuracies in 5 DL TDD CA # 8 | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3559 | - | F | Test Case on Random Acces for 3 DL/3UL TDD CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161129 | 3560 | 1 | B | SCell activation and deactivation of known PUCCH Scell in TDD CA without valid TA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3562 | - | F | E-UTRAN-WLAN RSSI event triggered reporting in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3563 | 1 | B | CR for Cat-M1 CEMode A RLM test cases: DRX FDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3564 | 1 | B | CR for Cat-M1 CEMode A RLM test cases: DRX HD-FDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3565 | 1 | B | CR for Cat-M1 CEMode A RLM test cases: DRX TDD | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3567 | - | B | CR on activation and deactivation of known SCell for 3DL CC DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3568 | - | B | CR on additional test requirements for Maximum transmission timing difference for DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3569 | - | F | CR on TDD-FDD 4DL CA activation and deactivation of known SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3570 | 1 | F | CR on TDD-FDD 5DL CA activation and deactivation of known SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3571 | - | F | CR on TDD-FDD 4DL CA activation and deactivation of unknown SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3572 | 1 | F | CR on TDD-FDD 5DL CA activation and deactivation of unknown SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3573 | 1 | B | CR on RRM requirements in Section 3 for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3574 | 1 | B | Draft CR on RRM requirements in Annex B for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3578 | 1 | F | Editorial corrections of an incorrect note for Band 32 | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3587 | 1 | F | Modification on intra-frequency discovery signal measurement requirements in LAA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3591 | 2 | F | Modification on CA requirements in LAA | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3596 | - | A | Correction of SCE event trigged reporting test cases for CSI-RS based discovery signal R13 | 13.4.0 |
| 2016/06 | RP-72 | RP-161129 | 3602 | 1 | B | 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3603 | - | B | CR: Introduction of testing principle for different combination of duplex modes DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3605 | - | A | CR of RLM requirement for PSCell in dual connectivity R12 | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3606 | - | B | E-UTRAN FDD - FDD DC intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3607 | - | B | E-UTRAN FDD - FDD DC intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3608 | - | B | E-UTRAN TDD - TDD DC intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3609 | 1 | F | CR on UE transmit timing requirement | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3612 | - | F | PCell in FDD: absolute and relative RSRP accuracies in FDD-TDD 4DL CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3613 | - | F | PCell in TDD: absolute and relative RSRP accuracies in TDD-FDD 4DL CA. | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3614 | - | F | absolute and relative RSRP accuracies in FDD 5 DL CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3615 | - | F | absolute and relative RSRP accuracies in TDD 5 DL CA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3616 | 1 | B | E-UTRAN Intra frequency case for Cat-M1 UE in normal coverage | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3617 | 1 | B | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3618 | 1 | B | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3619 | 1 | B | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3620 | 1 | B | CR on RSRP Intra frequency test cases for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3621 | - | B | PRACH configuration for eMTC | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3622 | 2 | F | CR on eMTC eDRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3629 | 3 | B | FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3630 | 3 | B | FD-FDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3631 | 3 | B | HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA: CR Not implemented as there are no track changes | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3632 | 3 | B | HD-FDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3633 | 3 | B | TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3634 | 3 | B | TDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3636 | 1 | B | E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3637 | 1 | B | E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3638 | 1 | B | Uplink transmit timing adjustments in HD-FDD operation | 13.4.0 |
| 2016/06 | RP-72 | RP-161132 | 3639 | 1 | B | E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3640 | 1 | F | 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3641 | 2 | F | 4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3642 | 2 | F | 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3643 | 2 | F | 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | 13.4.0 |
| 2016/06 | RP-72 | RP-161129 | 3644 | 1 | B | 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3647 | 2 | F | CR: RRC re-establishment requirements for Cat-M1 UEs | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3648 | 1 | B | CR: Cat-M1 PRACH test cases for FDD in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3649 | 1 | B | CR: Cat-M1 PRACH test cases for HD-FDD in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3650 | 1 | B | CR: Cat-M1 PRACH test cases for TDD in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3651 | 1 | B | CR: Cat-M1 Intra-frequency handover test cases for CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3653 | 1 | B | UE transmit timing test for eMTC UEs in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161128 | 3654 | 1 | B | RRC Re-establishment test for eMTC UEs in CEModeA | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3655 | 1 | F | Defining of ProSe periodicity for ProSe inter-frequency and CA operation | 13.4.0 |
| 2016/06 | RP-72 | RP-161141 | 3657 | - | F | Editorial corrections in Rel-12 Cat-0 requirements | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3659 | 2 | B | Draft CR on RRC_IDLE state requirements for NB-IOT | 13.4.0 |
| 2016/06 | RP-72 | RP-161168 | 3664 | - | F | E-UTRAN 4DL CA activation and deactivation for unknown SCells without DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161168 | 3665 | - | F | E-UTRAN 5DL CA activation and deactivation of unknown SCell in non-DRX | 13.4.0 |
| 2016/06 | RP-72 | RP-161142 | 3666 | 1 | F | CR: Intra-frequency handover requirements for Cat-M1 UEs in CEModeB | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3667 | - | B | Modification on RRC re-establishment requirement for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3668 | - | B | Modification for random access requirement for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3669 | - | B | Intra-frequency Absolute NRSRP Accuracy for UE Category NB1 in Normal Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3670 | - | B | Intra-frequency Absolute NRSRP Accuracy for UE Category NB1 in Enhanced Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3671 | - | B | Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3672 | - | B | Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Enhanced Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3673 | - | B | Inter-frequency Absolute NRSRP Accuracy for UE Category NB1 in Normal Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3674 | - | B | Inter-frequency Absolute NRSRP Accuracy for UE Category NB1 in Enhanced Mode | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3675 | 1 | B | CR on measurement requirement in RRC_CONNECTED state for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161126 | 3676 | - | B | CR on Radio Link Monitoring for NB-IoT | 13.4.0 |
| 2016/06 | RP-72 | RP-161125 | 3495 | - | B | Introduction of Band 70 to 36.133 | 14.0.0 |
| 2016/06 | RP-72 | RP-161124 | 3579 | 1 | B | Introduction of 2.6GHz SDL | 14.0.0 |
| 2016/09 | RP-73 | RP-161634 | 3679 | - | A | Duration of T3 in RRM 3DL Test cases A.8.16.31, A.8.16.32, A.8.16.33, A.8.16.34. | 14.1.0 |
| 2016/09 | RP-73 | RP-161640 | 3681 | - | A | Duration of T3 in RRM 4DL/5DL Test cases A.8.16.55, A.8.16.56, A.8.16.73, A.8.16.74. | 14.1.0 |
| 2016/09 | RP-73 | RP-161613 | 3683 | - | A | Square bracket removal for RLM antenna connection for 4 Rx capable UEs | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3685 | - | A | LAA channel occupancy test | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3687 | - | A | LBT model for LAA RRM tests | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3689 | - | A | LAA Average RSSI accuracy test | 14.1.0 |
| 2016/09 | RP-73 | RP-161619 | 3690 | 1 | B | Higher resolution RSTD measurement report mapping | 14.1.0 |
| 2016/09 | RP-73 | RP-161616 | 3691 | 2 | B | eLAA RRM requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3695 | - | A | Intra-frequency FDD test cases for RS-SINR | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3697 | - | A | Intra-frequency TDD test cases for RS-SINR | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3699 | - | A | Applicability of intra-frequency maximum measurement time requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3703 | - | A | Known cell requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3705 | - | A | Channel occupancy accuracy requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161780 | 3707 | - | A | CR: Reference NPRACH Configuration for NB-IoT RRM test cases | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3709 | - | A | CR: Corrections on Section 4.6 “Cell Selection and Re-selection Requirements for UE category NB1 | 14.1.0 |
| 2016/09 | RP-73 | RP-161639 | 3715 | - | A | CR: Correction of E-CID RSRP measurement requirements for Cat-M1 UEs | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3717 | - | A | CR: OCNG patterns for Cat-M1 RRM Tests | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3719 | - | A | CR: Correction of Cat-M1 Intra-frequency handover test cases for CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3721 | - | A | CR: Correction of Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161782 | 3725 | - | A | CR: Cat-M1 PRACH test cases for FDD in Enhanced Coverage | 14.1.0 |
| 2016/09 | RP-73 | RP-161782 | 3727 | - | A | CR: Cat-M1 PRACH test cases for HD-FDD in Enhanced Coverage | 14.1.0 |
| 2016/09 | RP-73 | RP-161782 | 3729 | - | A | CR: Cat-M1 PRACH test cases for TDD in Enhanced Coverage | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3740 | - | A | CR on RS-SINR TDD-FDD inter-frequency accuracy test case | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3741 | - | A | CR on RS-SINR FDD-TDD inter-frequency accuracy test case | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3743 | - | A | CR on RS-SINR measurement accuracy requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161617 | 3744 | 1 | B | CR of core requirements for V2V service | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3751 | - | A | CR of FDD intrafrequency absolute and relative RSRP accuracy test for SCell with FS3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3752 | - | A | CR of TDD intrafrequency absolute and relative RSRP accuracy test for SCell with FS3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3753 | - | A | CR of FDD intrafrequency RSRQ meaurement accuracy test for SCell with FS3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3754 | - | A | CR of TDD intrafrequency RSRQ meaurement accuracy test for SCell with FS3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161627 | 3755 | - | F | CR of Applicability of requirements for xDL2UL CA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3757 | - | A | CR for Cat-M1 CEMode A RLM test cases: DRX FDD | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3759 | - | A | CR for Cat-M1 CEMode A RLM test cases: DRX HD-FDD | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3761 | - | A | CR for Cat-M1 CEMode A RLM test cases: DRX TDD | 14.1.0 |
| 2016/09 | RP-73 | RP-161640 | 3767 | - | A | CR on modification on report mapping of TADV measurement in TS36.133 for Rel-14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161636 | 3769 | - | A | Editing change in TS36.133 for Rel-14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3771 | - | A | LAA SCell activation and deactivation for known SCells without DRX | 14.1.0 |
| 2016/09 | RP-73 | RP-161783 | 3773 | - | A | Event triggered reporting on LAA deactivated SCells and interruption probability (0.5%) without DRX | 14.1.0 |
| 2016/09 | RP-73 | RP-161631 | 3785 | - | A | Correction to RSTD Test Cases for 1.4 MHz | 14.1.0 |
| 2016/09 | RP-73 | RP-161635 | 3787 | - | F | Correction to Band 66 notes in E-UTRA band groups in Rel-14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3791 | - | A | RMCs for NPDSCH and NPDCCH for in-band for NB-IoT test cases | 14.1.0 |
| 2016/09 | RP-73 | RP-161634 | 3804 | - | A | Resolving TBDs in HD-FDD RLM test-cases for Rel-12 category 0 UEs | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3806 | - | A | E-UTRAN UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3811 | - | A | RRC Re-establishment test for eMTC UEs in CEModeB | 14.1.0 |
| 2016/09 | RP-73 | RP-161635 | 3828 | - | A | Correction to DL RMCs for Cell1 in A 8.16.25 | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3832 | - | A | CR on FDD-FDD inter-frequency RS-SINR accuracy test case R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161612 | 3834 | - | A | CR on TDD-TDD inter-frequency RS-SINR accuracy test case R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3838 | - | A | Difinition and abbreviation of frame structure 3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3840 | - | A | Editorial correction on LAA measurement requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3842 | - | A | Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in non-DRX based on CRS under Operation with Frame Structure 3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3844 | - | A | Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS under Operation with Frame Structure 3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3846 | - | A | CR on inter-frequency infinite measurement in LAA R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161638 | 3848 | - | A | Correction on discovery signal conditions in LAA R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161616 | 3849 | 1 | B | CR on RACH in eLAA | 14.1.0 |
| 2016/09 | RP-73 | RP-161785 | 3854 | - | A | Modification on inter-frequency CSI-RS related test cases R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161785 | 3857 | - | A | Modification on CSI-RS related CA test cases R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161785 | 3860 | - | A | Correction on discovery signal conditions for SCE R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161785 | 3863 | - | A | Correction of Band group for TDD SCE test R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161785 | 3866 | - | A | Correction on accuracy test cases for CRS based measurement R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3868 | - | A | E-UTRAN Intra frequency case for Cat-M1 UE in enhanced coverage R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3870 | - | A | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeB R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3872 | - | A | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeB R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3874 | - | A | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in CEModeB R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161610 | 3876 | - | A | RSRP Intra frequency case for Cat-M1 UE in CEModeB R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161782 | 3878 | - | A | CR on PRACH configuration reference R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161639 | 3884 | - | A | CR on eMTC maintenance R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161639 | 3886 | - | A | CGI requirement for eMTC R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161639 | 3890 | - | A | Correction on UE Category M1 measurement requirement R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3892 | - | A | CR for NB-IoT RRC re-establishment R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3900 | - | A | Modification on requirement of measurement in RRC_CONNECTED for NB-IoT R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3904 | - | A | CR for Conditions on NSCH Ês/Iot of identified and of the neighbour cell R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161780 | 3906 | - | A | CR for OCNG pattern for NB-IoT standalone operation R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3908 | 1 | A | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 under normal coverage R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3910 | 1 | A | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 under enhanced coverage R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3914 | - | A | CR: RMCs for NPDCCH RMCs for standalone and guard band NB-IoT test cases R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3915 | - | A | CR: RMCs for NPDCCH RMCs for standalone and guard band NB-IoT test cases R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161780 | 3916 | - | A | CR of OCNG pattern for guard band for NB-IoT test cases R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3918 | - | A | Introduce inter-frequency NRSRQ measurement accuracy requirement R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3920 | - | A | Modification on Conditions for NB-IoT inter-frequency Accuracy Requirements for UE Category NB1 R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161784 | 3923 | - | A | CR of test principle for DC test cases with different bandwidth combinations R14 | 14.1.0 |
| 2016/09 | RP-73 | RP-161780 | 3926 | 1 | A | CR OCNG pattern for in-band RRM tests | 14.1.0 |
| 2016/09 | RP-73 | RP-161634 | 3930 | - | A | CR for correction to some parameters in D2D RRM tests | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3934 | - | A | Inter-frequency event triggered reporting | 14.1.0 |
| 2016/09 | RP-73 | RP-161614 | 3935 | 1 | A | Intra-frequency absolute and relative CSI-RSRP accuracies for SCell with FS3 | 14.1.0 |
| 2016/09 | RP-73 | RP-161616 | 3937 | 1 | B | Clarification for TAGs with FS3 cells | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3945 | - | A | E-UTRAN UE Timing Advance Adjustment Accuracy Test for NB-IoT UE in Enhanced Coverage | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3947 | - | A | HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3858 | - | A | Corrections on FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3960 | - | A | Corrections on FD-FDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3962 | - | A | Corrections on HD-FDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3964 | - | A | Corrections on TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161781 | 3966 | - | A | Corrections on TDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3969 | - | A | Correction CR on UE Measurement Capability for NB-IOT | 14.1.0 |
| 2016/09 | RP-73 | RP-161637 | 3972 | - | A | Correction to Radio Link Monitoring Requirements for NB-IoT | 14.1.0 |
| 2016/09 | RP-73 | RP-161609 | 3973 | - | A | Power headroom reporting requirements | 14.1.0 |
| 2016/09 | RP-73 | RP-161611 | 3974 | - | A | CR on eD2D RRM tests | 14.1.0 |
| 2016/12 | RP-74 | RP-162430 | 3979 | - | A | Correction of RRC re-establishment delay for eMTC | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 3980 | 2 | A | Correction of transmit timing for category M1 | 14.2.0 |
| 2016/12 | RP-74 | RP-162384 | 3990 | - | A | UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeB | 14.2.0 |
| 2016/12 | RP-74 | RP-162423 | 3997 | 1 | A | Corrections to Antenna connection for 4 Rx capable UEs | 14.2.0 |
| 2016/12 | RP-74 | RP-162458 | 4001 | - | A | Corrections on inter-frequency measurement test cases for IncMon in R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162424 | 4006 | - | A | Corrections on DC measurements test cases R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162424 | 4008 | - | A | Corrections on DC test cases for measurements with autonomous gaps R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162385 | 4010 | 1 | A | Corrections on RLM test cases for Cat-M1 UE in R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162433 | 4017 | - | A | CR on RRC re-establishment RRM requirement R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162435 | 4015 | - | A | Introduce test requirements for E-UTRAN TDD - TDD Intra frequency handover for UE category 0 R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162426 | 4027 | - | A | CR on eDRX maintenance R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162431 | 4029 | - | A | CR on TDD CEModeB measurement requirement R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162431 | 4031 | - | A | CR on eMTC maintenance R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162459 | 4043 | - | A | Correction on the test cases of autonomous gaps in R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162387 | 4071 | 1 | B | eLAA RRM requirements | 14.2.0 |
| 2016/12 | RP-74 | RP-162384 | 4083 | - | A | CR: Cat-M1 Editoral Corrections | 14.2.0 |
| 2016/12 | RP-74 | RP-162385 | 4098 | 1 | F | Correction to RRC reestablishment test case in CE mode A | 14.2.0 |
| 2016/12 | RP-74 | RP-162385 | 4102 | 1 | F | Correction to transmit timing accuracy test case in CE mode A | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 4104 | 2 | A | Modification to Handover Delay in CE ModeA | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 4108 | 1 | A | Modification to Handover Delay in CE ModeB | 14.2.0 |
| 2016/12 | RP-74 | RP-162434 | 4112 | 1 | F | Corrections on NPDCCH transmission parameters | 14.2.0 |
| 2016/12 | RP-74 | RP-162429 | 4116 | - | A | CR on finalization of RS-SINR measurement accuracy requirements | 14.2.0 |
| 2016/12 | RP-74 | RP-162428 | 4123 | - | A | Correction on the discovery signal measurements under FS3 R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162428 | 4125 | - | A | Editorial correction on the measurement applicability in LAA R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162416 | 4128 | - | A | Corrections to 3DL CA Event triggered reporting Test cases A.8.16.29, A.8.16.30 | 14.2.0 |
| 2016/12 | RP-74 | RP-162459 | 4134 | - | A | CR on CSI-RS based measurement conditions R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162432 | 4144 | - | A | Correction to levels in LTE-WLAN RRM test | 14.2.0 |
| 2016/12 | RP-74 | RP-162431 | 4148 | - | A | Correction to RRC re-establishment requirements in eMTC | 14.2.0 |
| 2016/12 | RP-74 | RP-162431 | 4150 | - | A | Requirements for RRC Connection Release with Redirection in eMTC | 14.2.0 |
| 2016/12 | RP-74 | RP-162434 | 4152 | - | A | Requirements for redirection to non-amchor carrier | 14.2.0 |
| 2016/12 | RP-74 | RP-162434 | 4158 | - | A | CR on PTW length in cell reselection requirement for NB-IoT | 14.2.0 |
| 2016/12 | RP-74 | RP-162385 | 4160 | - | A | CR for Cat-M1 CEMode A RLM DRX test cases | 14.2.0 |
| 2016/12 | RP-74 | RP-162424 | 4161 | - | A | Modifications on SSTD measurement reporting | 14.2.0 |
| 2016/12 | RP-74 | RP-162384 | 4163 | 1 | A | CR on cell re-selection test case for Cat-M1 in normal coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4167 | 1 | A | CR on RRC re-establishment test case for Cat-M1 in CE Mode A | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4169 | - | A | CR on RRC re-establishment test case for Cat-M1 in CE Mode B | 14.2.0 |
| 2016/12 | RP-74 | RP-162433 | 4170 | - | A | Correction of transmit timing for NB-IoT | 14.2.0 |
| 2016/12 | RP-74 | RP-162415 | 4175 | - | A | PCC and SCC assignment in 20MHz+10MHz test case A.8.20.2B and A.9.2.27 | 14.2.0 |
| 2016/12 | RP-74 | RP-162418 | 4178 | - | A | Remove redundant requirement for Intra-frequency relative CSI-RSRP | 14.2.0 |
| 2016/12 | RP-74 | RP-162420 | 4181 | - | A | Correct InformationBitPayload for Sub-Frame 1, 6 and Max T-put of TDD PDSCH RMC | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4185 | - | A | Correction to Es/Noc, Es/Iot and RSRP values in Idle mode re-selection test in normal coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4187 | - | A | Corrections to rsrp-ThresholdsPrach and test requirement in Random Access Test in enhanced coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162405 | 4194 | - | B | Introduction of Band 48 to 36.133 | 14.2.0 |
| 2016/12 | RP-74 | RP-162435 | 4198 | - | A | 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | 14.2.0 |
| 2016/12 | RP-74 | RP-162435 | 4200 | - | A | 4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation | 14.2.0 |
| 2016/12 | RP-74 | RP-162435 | 4204 | - | A | 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | 14.2.0 |
| 2016/12 | RP-74 | RP-162379 | 4206 | - | A | Clarification to applicability of NRSRQ for UE category NB1 | 14.2.0 |
| 2016/12 | RP-74 | RP-162433 | 4208 | - | A | Introducing agreed measurement period for NB IoT connected mode | 14.2.0 |
| 2016/12 | RP-74 | RP-162456 | 4210 | 2 | A | Introducing agreed measurement accuracy for NB IoT | 14.2.0 |
| 2016/12 | RP-74 | RP-162388 | 4217 | 2 | B | CR for RRM requirements in idle mode for high speed conditions | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4221 | - | A | CR: E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB (Rel-14) | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4223 | - | A | CR: E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB (Rel-14) | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4225 | - | A | CR: E-UTRAN TDD-TDD Intra frequency handover for Cat-M1 UEs in CEModeB (Rel-14) | 14.2.0 |
| 2016/12 | RP-74 | RP-162391 | 4228 | 2 | B | CR to introduce handover requirements for mobility enhancement | 14.2.0 |
| 2016/12 | RP-74 | RP-162409 | 4229 | 1 | F | CR on reliability of GNSS | 14.2.0 |
| 2016/12 | RP-74 | RP-162386 | 4232 | 1 | A | CR for Inter-frequency event triggered reporting | 14.2.0 |
| 2016/12 | RP-74 | RP-162379 | 4237 | - | A | CR on UE Transmit Timing Accuracy Tests for NB-IoT UE | 14.2.0 |
| 2016/12 | RP-74 | RP-162379 | 4239 | - | A | CR for PHR requirement for NB-IoT R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162379 | 4241 | - | A | CR for NPDCCH RMC R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4243 | - | A | CR for NPDSCH RMC R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162456 | 4245 | 1 | F | CR on test parameter for RRC re-establishment R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4246 | - | F | CR on NB-IoT measurement conditions R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4248 | - | A | HD-FDD radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 under normal coverage R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4250 | - | A | HD-FDD radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 under enhanced coverage R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4252 | - | A | Correction on test parameter in RSRP Intra frequency case for Cat-M1 UE in CE mode A R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4254 | - | A | Correction on test parameter in RSRP Intra frequency case for Cat-M1 UE in CE mode B R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162382 | 4256 | - | A | Introduce RSRP accuracy test case for Band 31 for Cat-M1 UE R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4258 | - | A | Introduce 5MHz Bandwidth MPDCCH Reference Channel for Cat-M1 UE R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4260 | - | A | Introduce 5MHz Bandwidth MPDSCH Reference Channel for Cat-M1 UE R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4262 | - | A | Introduce 5MHz Bandwidth OCNG for Cat-M1 UE R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162433 | 4264 | - | A | CR on RRC re-establishment RRM requirement R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162433 | 4266 | - | A | CR on paging interruption for NB-IoT R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 4270 | - | A | CR on CGI reading of eMTC R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 4272 | - | A | CR on handover of eMTC R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4280 | - | A | CR on modification of cell reselection test case R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162420 | 4288 | - | A | Corrections on DC interruption test cases R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162388 | 4291 | 1 | B | CR on the enhanced RRM requirements in connected mode under high speed scenario | 14.2.0 |
| 2016/12 | RP-74 | RP-162392 | 4293 | 3 | B | CR on interruption requirements in SRS switching | 14.2.0 |
| 2016/12 | RP-74 | RP-162418 | 4299 | - | A | Correction on SCE test cases R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162420 | 4302 | - | A | Correction on the test cases of RSTD Mesaurement in R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162420 | 4305 | - | A | Corrections on the test cases of UE measurement procedures and measurement performance requirements in R14 | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4309 | 1 | A | SI reading tests for eMTC UEs in CEModeB | 14.2.0 |
| 2016/12 | RP-74 | RP-162456 | 4315 | 1 | A | RLM in-sync test DRX under enhanced coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162456 | 4316 | 1 | A | RLM in-sync test with DRX under normal coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162456 | 4319 | 1 | A | Intra-frequency cell reselection under normal coverage for NB-IOT | 14.2.0 |
| 2016/12 | RP-74 | RP-162383 | 4321 | 1 | A | Applicability rule for eMTC test cases in CEModeA and CEModeB | 14.2.0 |
| 2016/12 | RP-74 | RP-162387 | 4343 | 1 | F | eLAA requirements corrections | 14.2.0 |
| 2016/12 | RP-74 | RP-162420 | 4348 | - | A | RRM: Correction to TCs A.7.1.7A and A.7.1.7B (Rel-14) | 14.2.0 |
| 201/6/12 | RP-74 | RP-162383 | 4352 | - | A | Correction to the handover test case in CE mode A | 14.2.0 |
| 2016/12 | RP-74 | RP-162414 | 4354 | - | A | PCFICH/PDCCH/PHICH Reference channel in UE Cat 0 new CGI RRM test cases | 14.2.0 |
| 2016/12 | RP-74 | RP-162392 | 4355 | 1 | B | Requirements with SRS carrier based switching | 14.2.0 |
| 2016/12 | RP-74 | RP-162417 | 4358 | - | A | Correction to RRM tests on dual connectivity | 14.2.0 |
| 2016/12 | RP-74 | RP-162430 | 4361 | - | A | Correction of paging interruption for eMTC | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4362 | 1 | A | CR on RLM in-sync test without DRX under normal coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162380 | 4364 | 1 | A | CR on RLM in-sync test without DRX under enhanced coverage | 14.2.0 |
| 2016/12 | RP-74 | RP-162392 | 4365 | 1 | F | Correction on random access requirement for SRS switching | 14.2.0 |
| 2016/12 | RP-74 | RP-162407 | 4366 | - | B | Introduction of new bands for NB-IoT in 36.133 | 14.2.0 |
| 2017/03 | RP-75 | RP-170595 | 4368 |  | A | Correction to transmit timing accuracy test case in Cat-m1 CE mode B | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4372 |  | A | Correction to the intra-frequency reselection requirements in Cat-m1 enhanced coverage | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4374 |  | A | Correction to intra-frequency handover test case in CE mode B | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4376 |  | A | Correction to intra-frequency handover test case in CE mode A | 14.3.0 |
| 2017/03 | RP-75 | RP-170597 | 4378 |  | A | Modification the intra-frequency event triggered reporting test cases in Cat-m1 CE mode B | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4380 |  | A | Correction to the CGI requirement test case in Cat-M1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170597 | 4382 |  | A | Editorial correction to the CGI requirements | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4384 |  | A | Correction to payload sizes in reference channels for Cat-m1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170598 | 4388 | 1 | F | CR Update to Radio Link Monitoring Requirements for NB-IoT | 14.3.0 |
| 2017/03 | RP-75 | RP-170598 | 4390 |  | A | Correction CR on RRC_IDLE state requirements for NB-IOT | 14.3.0 |
| 2017/03 | RP-75 | RP-170563 | 4391 | 2 | B | CR for RRM core requirement for Cat.1 UE with single receiver chain (Part 1) | 14.3.0 |
| 2017/03 | RP-75 | RP-170563 | 4392 | 1 | B | CR for RRM core requirement for Cat.1 UE with single receiver chain (Part 2) | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4395 |  | A | Correction to prach-ConfigIndex for TDD Random Access Test for Cat-M1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4399 |  | A | Corrections to UE Cat M1 Intra-frequency Event triggered Reporting CE Mode A Test Cases | 14.3.0 |
| 2017/03 | RP-75 | RP-170588 | 4401 |  | A | Correct 4DL Act-deact Unknown SCell Test Cases A.8.16.63 and A.8.16.64 | 14.3.0 |
| 2017/03 | RP-75 | RP-170600 | 4403 |  | A | Correct parameters for UE Category NB1 Reselection Test Case A.4.2.18 | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4405 |  | A | Correct Frequency hopping parameters for UE Cat M1 PDSCH Reference channel | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4407 |  | A | Correction to OCNG pattern for CatM1 FD-FDD and HD-FDD re-establishment TCs | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4420 |  | A | Corrections to UE Cat M1 Intra-frequency Event triggered Reporting CE Mode B Test Cases | 14.3.0 |
| 2017/03 | RP-75 | RP-170597 | 4424 |  | A | RRM: Corrections to eMTC PRACH TC A.6.2.10 and A.6.2.11 (Rel-14) | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4428 | 1 | B | CR on Initiation/Cease of SLSS Transmissions | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4430 | 2 | B | CR on Autonomous Resource Selection/Reselection measurements | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4431 | 1 | B | CR on PSSCH-RSRP measurement accuracy requirement | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4432 | 1 | B | CR on Intra-Frequency S-RSRP Measurement Accuracy Requirements | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4433 | 1 | B | CR on Side conditions for V2X measurements | 14.3.0 |
| 2017/03 | RP-75 | RP-170596 | 4437 |  | A | CR on initial PRACH transmit power in RA test for Cat-M1 CEMode B R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170554 | 4438 | 2 | B | CR to introduce test case for V2V interruption | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4439 | 1 | B | CR on feMTC measurement requirements section 4 | 14.3.0 |
| 2017/03 | RP-75 | RP-170579 | 4443 |  | F | CR on HST connected mode measurement requirement | 14.3.0 |
| 2017/03 | RP-75 | RP-170557 | 4444 |  | F | CR to finalize handover requirements for further mobility enhancement | 14.3.0 |
| 2017/03 | RP-75 | RP-170594 | 4448 |  | A | Correction to core requirement of RRC re-establishment in Cat-m1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170594 | 4450 |  | A | Correction to core requirement of Handover in Cat-m1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170597 | 4452 | 1 | A | Removing square brackets from RLM test case in Cat-m1 | 14.3.0 |
| 2017/03 | RP-75 | RP-170554 | 4459 | 1 | B | CR on UE transmission timing accuracy test for V2V | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4460 | 2 | B | CR on interruption requirements for V2X | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4461 | 2 | B | CR on Selection / Reselection of Synchronization Reference for V2X | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4462 | 1 | B | CR on S-RSSI measurement accuracy requirement for V2X | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4463 | 1 | B | CR on Congestion Control Measurements Requirements for V2X | 14.3.0 |
| 2017/03 | RP-75 | RP-170577 | 4464 | 1 | B | CR on Introducing inter-cell synchronization requirements for MBMS R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170600 | 4466 |  | A | CR on RRC re-establishment test case R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170601 | 4472 |  | A | Remove square brackets in NB-IoT performance requirement R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170581 | 4477 |  | A | Correction on test parameter in RSRP Intra frequency case for UE category 0 R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4480 |  | A | Correction on RSRP level in RSRP Intra frequency case for Cat-M1 UE in CEModeB R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4482 |  | A | Correction of 8.13 Measurements for UE Category M1 R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170564 | 4484 | 1 | A | CR on WLAN RSSI R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170594 | 4489 | 1 | A | Not implement CRs for eMTC R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4502 | 1 | B | CR on E-CID RSRP RSRQ measurement requirement for FeMTC | 14.3.0 |
| 2017/03 | RP-75 | RP-170600 | 4508 |  | A | CR for the correction on the testcases of HD-FDD Radio Link Monitoring for UE category NB1 in R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170603 | 4510 | 1 | A | Correction on measurement performance requirements for UE category M1 in R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170585 | 4513 |  | A | CR for the correction on the testcases of Proximity-based Services and measurement performance requirement in R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170555 | 4514 | 1 | B | CR on measurement accuarcy requirements under high speed scenarios | 14.3.0 |
| 2017/03 | RP-75 | RP-170591 | 4518 | 1 | F | Remove the bracket in LAA requirements R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170592 | 4520 |  | A | Correction on LAA test cases R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170592 | 4522 |  | A | Correction on p-C-r10 value for LAA test cases R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170584 | 4525 |  | A | Correction on SCE event triggered reporting for CSI-RS based test cases R14 | 14.3.0 |
| 2017/03 | RP-75 | RP-170559 | 4529 | 1 | B | CR on UE transmission timing requirements for V2X | 14.3.0 |
| 2017/03 | RP-75 | RP-170601 | 4543 | 1 | A | Test on Intra-frequency cell reselection in enhanced coverage | 14.3.0 |
| 2017/03 | RP-75 | RP-170595 | 4545 |  | A | Correction to Cat-M1 UE RRC re-establishment tests | 14.3.0 |
| 2017/03 | RP-75 | RP-170581 | 4550 |  | A | Correction of RMC reference in the cat-0 HD-FDD intra-frequency test case | 14.3.0 |
| 2017/03 | RP-75 | RP-170594 | 4552 |  | A | Clarification on measurement reporting delay for eMTC | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4554 | 3 | B | Introducing intra-frequency measurement requirements for UE category M2 in normal coverage/CEModeA | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4555 | 1 | B | Introducing intra-frequency measurement requirements for UE category M2 in enhanced coverage/CEModeB | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4556 | 2 | B | Introducing inter-frequency measurement requirements for UE category M2 in normal coverage/CEModeA | 14.3.0 |
| 2017/03 | RP-75 | RP-170560 | 4557 | 3 | B | Introducing inter-frequency measurement requirements for UE category M2 in enhanced coverage/CEModeB | 14.3.0 |
| 2017/03 | RP-75 | RP-170598 | 4565 |  | A | Correction to redirection to NB-IoT non-anchor carrier | 14.3.0 |
| 2017/03 | RP-75 | RP-170594 | 4567 |  | A | Correction to RRC release with redirection in eMTC | 14.3.0 |
| 2017/03 | RP-75 | RP-170582 | 4575 | 1 | A | PCC and SCC assignment in 20MHz+10MHz test case A.8.16.21 and A.8.20.4B | 14.3.0 |
| 2017/03 | RP-75 | RP-170603 | 4578 | 1 | A | 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | 14.3.0 |
| 2017/03 | RP-75 | RP-170598 | 4582 | 1 | A | Capturing agreements for NB-IoT | 14.3.0 |
| 2017/03 | RP-75 | RP-170591 | 4598 | 3 | F | LAA SCC requirements with multiple SCCs | 14.3.0 |
| 2017/03 | RP-75 | RP-170553 | 4599 | 1 | F | Correction in UE transmit Timing Requirements with eLAA SCell | 14.3.0 |
| 2017/03 | RP-75 | RP-170592 | 4601 |  | A | Correction of LAA RRM test cases | 14.3.0 |
| 2017/06 | RP-76 | RP-171309 | 4603 |  | A | PDSCH allocation parameters for UE Cat M1 RSRP Test Cases | 14.4.0 |
| 2017/06 | RP-76 | RP-171303 | 4605 |  | F | Cat NB1 RRM Test Case A.4.2.18/19 update for 5MHz Ch BW | 14.4.0 |
| 2017/06 | RP-76 | RP-171278 | 4606 |  | F | CR on applicability of requirements for CA | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4608 | 1 | B | Introduction of non-uniform gap pattern | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4612 |  | A | CR: NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Normal Coverage | 14.4.0 |
| 2017/06 | RP-76 | RP-171307 | 4619 |  | A | LAA RRM: Correction to test case titles (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4621 |  | A | eMTC RRM: Correction to prach test cases (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4623 |  | A | eMTC RRM: Alignment of used OCNG patterns (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4625 |  | A | NB-IoT RRM: Correction to NOCNG definition (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4627 |  | A | CA RRM: 5DL CA test case titles for inter-mode (FDD-TDD) scenarios (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4629 |  | B | RSTD requirements for short MGL | 14.4.0 |
| 2017/06 | RP-76 | RP-171283 | 4633 | 1 | F | Correction to RRC connected state requirements for CA | 14.4.0 |
| 2017/06 | RP-76 | RP-171284 | 4636 | 1 | F | CR on CGI reading requirement with SRS switching | 14.4.0 |
| 2017/06 | RP-76 | RP-171308 | 4650 | 1 | A | CR on measurement condition in B.1.3 R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171279 | 4655 | 1 | B | Introduction of new bands for NB-IoT in 36.133 | 14.4.0 |
| 2017/06 | RP-76 | RP-171296 | 4686 | 1 | A | Band groups for category 0 operation | 14.4.0 |
| 2017/06 | RP-76 | RP-171308 | 4688 |  | A | Band groups for category 0 operation | 14.4.0 |
| 2017/06 | RP-76 | RP-171298 | 4692 | 1 | A | Prose Operation during CA | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4695 | 3 | F | PHR reporting for NB-IOT low-power class UEs | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4698 |  | A | Correction to Cat-M1 UE RRC re-establishment tests | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4701 | 3 | B | Timing requirements for Cat-M2 | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4703 | 1 | B | CR on NCSG configuration | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4704 | 2 | B | CR on per-CC based measurement gap configuration | 14.4.0 |
| 2017/06 | RP-76 | RP-171269 | 4706 | 3 | F | CR for correction for RRM core requirement for Cat.1bis UE | 14.4.0 |
| 2017/06 | RP-76 | RP-171269 | 4707 | 1 | B | CR for RRM measurement requirement for Cat.1bis UE | 14.4.0 |
| 2017/06 | RP-76 | RP-171269 | 4708 | 3 | B | CR for RRM tests for Cat.1 UE with single receiver chain | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4714 |  | A | Correction to event triggered reporting test cases in eMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171299 | 4716 |  | A | Correction to inter-frequency requirements in NB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171288 | 4719 | 1 | F | Clarification on requirements for RSTD based on CRS and PRS | 14.4.0 |
| 2017/06 | RP-76 | RP-171284 | 4721 | 1 | F | SRS switching and SI reading | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4728 | 1 | B | Requirements on NPRACH Transmission in Enhanced NB-IoT | 14.4.0 |
| 2017/06 | RP-76 | RP-171299 | 4760 |  | A | Maintenance of core requirements for UE Cat.NB1 R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4761 | 3 | B | Applicability of requirements for UE Cat.NB2 | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4762 |  | B | RRM tests for UE Cat.NB2 | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4764 |  | A | CR on test parameters for UE transmit timing for UE Cat.NB1 R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4766 |  | A | CR for PHR requirement for NB-IoT R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4770 |  | A | CR on conditions for NRSRP and NRSRQ accuracy R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4772 |  | A | CR on Io range for NB-IoT measurement accuracy R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4776 |  | A | CR for cell reselection test parameter for NB-IoT R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171302 | 4778 | 1 | F | Correction on T2 time duration for RLM in-sync test R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4797 | 3 | B | CR on UE Rx-Tx measurement requirement for FeMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4798 | 1 | B | CR on UE Rx-Tx accuracy requirement for FeMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171262 | 4808 |  | B | Test case for CA interruption at SRS carrier based switching | 14.4.0 |
| 2017/06 | RP-76 | RP-171262 | 4809 | 1 | B | Test case for DC interruption at SRS carrier based switching | 14.4.0 |
| 2017/06 | RP-76 | RP-171257 | 4812 | 1 | B | CR on measurement accuracy requirements under high speed scenarios | 14.4.0 |
| 2017/06 | RP-76 | RP-171257 | 4813 | 5 | B | Test case for cell reselection on idle mode under high speed scenario | 14.4.0 |
| 2017/06 | RP-76 | RP-171257 | 4814 | 3 | B | Test case for measurement reporting on connected mode under high speed scenario | 14.4.0 |
| 2017/06 | RP-76 | RP-171283 | 4815 | 1 | F | Remove bracket in measurement core requirements in high speed | 14.4.0 |
| 2017/06 | RP-76 | RP-171296 | 4816 |  | F | Correction on Io for SCE test case A.8.22.8 | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4819 |  | A | CR on RA test for Cat-M1 normal coverage R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4821 | 3 | F | Refinement of idle mode requirements for feMTC in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4823 | 1 | B | CR on NRSRP mapping table | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4824 |  | B | CR on NRSRQ mapping table | 14.4.0 |
| 2017/06 | RP-76 | RP-171256 | 4829 |  | F | CR on V2V RRM performance requirements corrections | 14.4.0 |
| 2017/06 | RP-76 | RP-171282 | 4830 | 1 | F | CR on V2X RRM core requirements corrections | 14.4.0 |
| 2017/06 | RP-76 | RP-171307 | 4834 |  | A | LAA RRM: Correction to test case A.9.11.x and A.9.12.x (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171309 | 4836 |  | A | eMTC RRM: Correction to MPDCCH and PDSCH Reference Measurement Channels (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171281 | 4837 |  | F | CR on side condition for reliability of GNSS | 14.4.0 |
| 2017/06 | RP-76 | RP-171298 | 4839 | 1 | A | Correction to RRM 4DL FDD CA test cases | 14.4.0 |
| 2017/06 | RP-76 | RP-171307 | 4841 |  | A | Corrections to LAA Event-triggered reporting Test Cases A.8.26 | 14.4.0 |
| 2017/06 | RP-76 | RP-171299 | 4843 |  | A | Rel-14 CR on the applicability of RRC procedures | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4844 | 1 | B | E-UTRAN FDD - FDD Intra frequency RACH-less handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4845 | 1 | B | E-UTRAN FDD - FDD Inter frequency RACH-less handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4846 | 1 | B | E-UTRAN TDD - TDD Intra frequency RACH-less handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4847 | 1 | B | E-UTRAN TDD - TDD Inter frequency RACH-less handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4853 | 1 | B | E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4854 | 1 | B | E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells | 14.4.0 |
| 2017/06 | RP-76 | RP-171298 | 4860 |  | A | CA RRM: 4DL CA test parameter correction for A.8.16.51 and A.8.16.52 (Rel-14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171296 | 4863 |  | A | PCC and SCC assignment in 20MHz+10MHz test case A.8.16.22 | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4864 | 1 | B | Introduction of additional gap patterns and measurement capabilities for measurement gap enhancement | 14.4.0 |
| 2017/06 | RP-76 | RP-171265 | 4867 | 1 | B | CR on reference configurations for V2X RRM | 14.4.0 |
| 2017/06 | RP-76 | RP-171281 | 4868 | 1 | B | Correction on reliability of GNSS singal for V2V | 14.4.0 |
| 2017/06 | RP-76 | RP-171282 | 4869 | 1 | B | CR on reliability of GNSS singal for V2X | 14.4.0 |
| 2017/06 | RP-76 | RP-171301 | 4873 |  | A | CR on T311 timer in RRC re-establishment test case R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171300 | 4875 |  | A | Introduce NPDCCH RMC with DCI format N0 R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171302 | 4877 |  | A | Correction on UE transmit timing test for NB-IoT R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171303 | 4879 |  | A | Correction on timing advance test for NB-IoT R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171299 | 4881 |  | A | Maintenance CR on NB-IoT core requirements R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171302 | 4883 |  | A | Correction on side condition for RRC re-establishment test cases R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4885 |  | B | E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4886 |  | B | E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover | 14.4.0 |
| 2017/06 | RP-76 | RP-171310 | 4888 |  | A | Correction on transmit timing requirement R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4889 | 1 | B | CR on intra frequency RSTD measurement requirement for eNB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4890 | 1 | B | CR on inter frequency RSTD measurement requirement for eNB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4891 | 1 | B | CR on intra frequency RSTD accuracy requirement for eNB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171267 | 4892 | 1 | B | CR on inter frequency RSTD accuracy requirement for eNB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4893 | 1 | B | CR on intra frequency RSTD measurement requirement for FeMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4894 | 1 | B | CR on inter frequency RSTD measurement requirement for FeMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4906 | 1 | B | CR on applicability rule for FeMTC in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4909 | 1 | A | CR on UE timer accuracy for FeMTC in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4911 | 1 | A | 4DL FDD RSRQ for E-UTRAN in Carrier Aggregation in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4913 | 1 | A | 4DL TDD RSRQ for E-UTRAN in Carrier Aggregation in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4915 | 1 | A | Correction on the test cases of RLM for Cat-M1 UE in R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171282 | 4917 | 1 | F | CR on Modification of Selection/Reselection of V2X Synchronization Reference | 14.4.0 |
| 2017/06 | RP-76 | RP-171265 | 4918 |  | B | CR on UE Transmission Timing Accuracy Tests for V2X | 14.4.0 |
| 2017/06 | RP-76 | RP-171285 | 4926 |  | F | Modification on LAA measurement considering Inter-frequency RSSI measurement R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171307 | 4928 |  | A | Corrections on the LAA test cases R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171297 | 4931 |  | A | Update of some SCE test case | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4933 |  | A | Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (FDD-TDD 4DL CA) R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171311 | 4935 |  | A | Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (TDD-FDD 4 DL CA) R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171303 | 4937 |  | A | Inter-frequency cell reselection under enhanced coverage for NB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4938 | 3 | B | CONNECTED mode requirements for feMTC UEs in CEModeA | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4939 | 3 | B | CONNECTED mode requirements for feMTC UEs in CEModeB | 14.4.0 |
| 2017/06 | RP-76 | RP-171308 | 4941 | 1 | A | Timing advance requirements for Cat M1 UEs | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4942 | 1 | B | Introduction of enhanced RLM requirements for feMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171303 | 4953 |  | A | Contention Based Random Access Test for UE category NB1 UEs in Normal Coverage | 14.4.0 |
| 2017/06 | RP-76 | RP-171303 | 4955 |  | A | Contention Based Random Access Test for UE category NB1 UEs in Enhanced Coverage | 14.4.0 |
| 2017/06 | RP-76 | RP-171302 | 4956 |  | A | CR: NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Enhanced Coverage | 14.4.0 |
| 2017/06 | RP-76 | RP-171297 | 4959 | 1 | A | Correction to RSTD test cases for carrier aggregation (R14) | 14.4.0 |
| 2017/06 | RP-76 | RP-171269 | 4960 | 1 | B | CR for RSTD measurement requirement for Cat.1bis UE | 14.4.0 |
| 2017/06 | RP-76 | RP-171302 | 4963 |  | A | Correction to inter-frequency RRC re-establishment test case in NB-IOT | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4968 |  | B | CR for E-UTRAN TDD intra-frequency MBB handover test cases | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4969 |  | B | CR for E-UTRAN FDD intra-frequency MBB handover test cases | 14.4.0 |
| 2017/06 | RP-76 | RP-171259 | 4970 |  | F | CR for correction of handover test cases | 14.4.0 |
| 2017/06 | RP-76 | RP-171286 | 4971 |  | F | CR for correction of handover requirements for mobility enhancement | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4972 | 1 | B | Enhanced Handover Requirement for FeMTC | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4974 |  | F | CR on 36.133: Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1 | 14.4.0 |
| 2017/06 | RP-76 | RP-171285 | 4982 | 1 | F | Requirements applicability for eLAA | 14.4.0 |
| 2017/06 | RP-76 | RP-171307 | 4984 |  | A | Corrections in LAA requirements | 14.4.0 |
| 2017/06 | RP-76 | RP-171261 | 4985 | 4 | B | RRM requirements with eMBMS enhancements | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4986 | 1 | F | Measurement requirements under discontonuous MPDCCH monitoring | 14.4.0 |
| 2017/06 | RP-76 | RP-171266 | 4990 | 1 | F | Introducing gapless measurement requirement for eMTC R14 | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4997 | 1 | B | Introduction of test cases for burst gap | 14.4.0 |
| 2017/06 | RP-76 | RP-171264 | 4998 | 1 | B | Introduction of test cases for NCSG | 14.4.0 |
| 2017/09 | RP-77 | RP-171933 | 5004 | 2 | B | Introduction of event triggered reporting LAA test cases for multiple Scells | 14.5.0 |
| 2017/09 | RP-77 | RP-171967 | 5007 |  | A | Updates to Intra-freq Event-triggered reporting Test cases for UE Cat 0 | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5009 |  | A | Reference Channels in UE Cat M1 RRC Re-establishment test cases | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5011 |  | A | OCNG and RMCs in UE Cat M1 Handover TCs | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5015 |  | A | Es/Iot and RSRP values in UE Cat M1 Re-selection test cases | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5017 |  | A | Reference PRACH Configurations for Cat M1 test cases | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5019 |  | A | PRACH Configurations and parameters for Cat M1 test cases A.6.2.10/11/12 | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5021 |  | A | PRACH Configuration for Cat M1 test case A.6.2.15 This CR was NOT implemented as it didn't use revison marks | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5025 |  | A | Correction to RLM test cases Category NB1. | 14.5.0 |
| 2017/09 | RP-77 | RP-171973 | 5028 |  | A | CA RRM: Correction of Cell 3 Es/Iot for T2 and T4 for TC A.8.16.55 and 56 (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171967 | 5031 |  | A | CA RRM: Correction of PRS Subframe Offset for TC A.8.17.10 and A.8.17.11 (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5033 |  | A | eMTC RRM: Corrections to Timing Advance Adjustment Accuracy test for Cat-M1 UE (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5035 |  | A | NB-IoT RRM: Correction of antenna configuration of LTE and NBIoT cells in Inband RRM scenarios (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5037 |  | A | NB-IoT RRM: Corrections to the NOCNG definition (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171943 | 5042 |  | F | Update applicability of V2V Interruption requriement | 14.5.0 |
| 2017/09 | RP-77 | RP-171943 | 5043 |  | F | Correction to V2X requirements for Initiation/Cease of SLSS transmissions | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5044 | 1 | B | CR on measurement performance scaling with MPDCCH monitoring CEModeA | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5045 | 1 | B | CR on measurement performance scaling with MPDCCH monitoring CEModeB | 14.5.0 |
| 2017/09 | RP-77 | RP-171943 | 5055 | 1 | F | Remove bracket in V2X core requirements | 14.5.0 |
| 2017/09 | RP-77 | RP-171969 | 5058 |  | A | Correction CR on adding band group to RS-SINR measurement conditions | 14.5.0 |
| 2017/09 | RP-77 | RP-171932 | 5061 | 2 | B | RRM requirements with FeMBMS | 14.5.0 |
| 2017/09 | RP-77 | RP-171941 | 5062 | 1 | F | CR for correcting transmit timing requirement of eLAA | 14.5.0 |
| 2017/09 | RP-77 | RP-171943 | 5066 |  | F | Clarification on V2X requirements for V2X UEs in RRC_CONNECTED state | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5068 | 1 | F | Gapsharing due to RSTD measurements for Rel-14 MTC in CEModeA | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5070 | 1 | F | Gap sharing due to RSTD measurements for Rel-14 MTC in CEModeB | 14.5.0 |
| 2017/09 | RP-77 | RP-171935 | 5073 |  | F | CR on evaluation time of GNSS synchronization source reliability | 14.5.0 |
| 2017/09 | RP-77 | RP-171943 | 5076 |  | F | Completing V2V Interruption test case | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5078 |  | A | NB-IoT RRM: Corrections to invalid configuration for In-band Intra-frequency scenairos (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171968 | 5080 |  | A | LAA RRM: Remove Square brackets from TCs A.9.1.60 and A.9.1.61 (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171971 | 5082 |  | A | eMTC RRM: MPDSCH Repetitions in CEModeA test cases with DRX (Rel-14) | 14.5.0 |
| 2017/09 | RP-77 | RP-171932 | 5083 | 1 | B | RRM performance requirements with FeMBMS | 14.5.0 |
| 2017/09 | RP-77 | RP-171940 | 5084 |  | F | CR on BS synchronization requirements for eMBMS enhancements | 14.5.0 |
| 2017/09 | RP-77 | RP-171935 | 5085 |  | B | CR on Initiation/Cease of SLSS Transmissions Tests for V2X | 14.5.0 |
| 2017/09 | RP-77 | RP-171935 | 5086 |  | B | CR on Synchronization Reference Selection / Reselection Tests for V2X | 14.5.0 |
| 2017/09 | RP-77 | RP-171935 | 5088 | 1 | B | CR on Congestion Control Measurement Tests for V2X | 14.5.0 |
| 2017/09 | RP-77 | RP-171935 | 5089 | 1 | B | CR on Interruptions Tests for V2X | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5095 |  | A | Correction on RLM core requirements for NB-IoT | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5097 | 1 | F | Correction on cell reselection test case for NB-IoT | 14.5.0 |
| 2017/09 | RP-77 | RP-171937 | 5099 | 1 | B | Introduce 5MHz NPDSCH RMC pattern for NB-IoT | 14.5.0 |
| 2017/09 | RP-77 | RP-171937 | 5100 |  | B | Introduce 5MHz NPDCCH RMC pattern for NB-IoT | 14.5.0 |
| 2017/09 | RP-77 | RP-171937 | 5101 |  | B | Correction on RRC re-establishment test case for NB-IoT | 14.5.0 |
| 2017/09 | RP-77 | RP-171973 | 5103 |  | A | Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (FDD-TDD 5DL CA) | 14.5.0 |
| 2017/09 | RP-77 | RP-171973 | 5105 |  | A | Event triggered reporting on deactivated SCells and interruption probability (0.5%) without DRX (TDD-FDD 5DL CA) | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5106 |  | F | Correction on the applicability rules for FeMTC in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5107 | 1 | F | Correction on the measurement requirement for FeMTC in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5108 | 1 | B | CR on conditions for E-UTRAN inter-frequency measruements by UE cat M1 in RRC_CONNECTED State in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5109 | 2 | B | CR on conditions for measurements procedures in RRC_CONNECTED State for cat M2 in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5110 | 1 | B | CR on conditions for measurements procedures in RRC_IDLE State for cat M2 in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171940 | 5112 | 1 | F | CR on correction on the band groups in R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171933 | 5116 | 1 | B | FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | 14.5.0 |
| 2017/09 | RP-77 | RP-171933 | 5117 | 1 | B | TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | 14.5.0 |
| 2017/09 | RP-77 | RP-171969 | 5119 |  | A | CR on RSRP range of eMTC Rel-14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5120 | 1 | F | CR on intra frequency RSTD measurement requirement for FeMTC | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5121 | 1 | F | CR on inter frequency RSTD measurement requirement for FeMTC | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5127 | 1 | F | CR on 36.133 Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1 | 14.5.0 |
| 2017/09 | RP-77 | RP-171972 | 5129 | 1 | F | CR for NB-IoT Transmit Timing Test | 14.5.0 |
| 2017/09 | RP-77 | RP-171969 | 5133 | 1 | A | Restructuring of Handover Requirements in FeMTC | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5134 |  | F | Corrections to enhanced RLM core requirements | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5135 |  | F | Corrections to E-CID UE Rx-Tx requirements | 14.5.0 |
| 2017/09 | RP-77 | RP-171973 | 5139 |  | F | Correction to LAA RRM measurement requirement R14 | 14.5.0 |
| 2017/09 | RP-77 | RP-171932 | 5141 | 2 | B | Inter-frequency measurement requirement for FeMBMS unicast mixed cell | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5143 | 2 | B | CR on inter frequency RSTD accuracy requirement for FeMTC | 14.5.0 |
| 2017/09 | RP-77 | RP-171936 | 5145 | 1 | B | CR on UE Rx-Tx accuracy requirement for FeMTC | 14.5.0 |
| 2017/09 | RP-77 | RP-171973 | 5146 |  | A | Correction to LAA RRM test cases | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5147 |  | F | CR on UE Rx - Tx measurement requirement for Cat-M1 | 14.5.0 |
| 2017/09 | RP-77 | RP-171942 | 5148 |  | F | CR on UE Rx - Tx measurement requirement for Cat-M2 | 14.5.0 |
| 2017/09 | RP-77 | RP-171948 | 5056 | 1 | B | Introduction of the FDD L-band (Band 74) into 36.133 | 15.0.0 |
| 2017/09 | RP-77 | RP-171946 | 5059 |  | B | Introduction of Band 72 in TS36.133 | 15.0.0 |
| 2017/09 | RP-77 | RP-171952 | 5072 |  | B | CR to 36.133: Introduction of Band 71 | 15.0.0 |
| 2017/09 | RP-77 | RP-171950 | 5074 | 1 | B | Introduction of Extended 1.5 GHz SDL bands 75 and 76 | 15.0.0 |
| 2017/09 | RP-77 | RP-171949 | 5136 | 1 | B | Introduction of TDD L-band into TS 36.133 | 15.0.0 |
| 2017-12 | RAN#78 | RP-172607 | 5162 |  | A | Correction of test requirement for LAA Test cases A.8.26.x | 15.1.0 |
| 2017-12 | RAN#78 | RP-172593 | 5163 | 1 | B | Introduction of Band 73 into TS 36.133 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5174 |  | A | Introduction of NB-IoT RSTD measurement requirement | 15.1.0 |
| 2017-12 | RAN#78 | RP-172609 | 5175 |  | A | Clarification on the SNR transition in NB-IoT RLM test for in-sync | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5192 |  | A | cell identification test case for FeMTC in CEModeA with discontinuous MPDCCH monitoring | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5194 |  | A | cell identification test case for FeMTC in CEModeA in DRX | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5196 |  | A | cell identiciation test case for serving cell without gap for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5197 |  | A | RSRP accuracy test case for FeMTC in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5210 |  | A | CR for MPDCCH RMCs for Category M2 UE RRM tests (R15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5212 |  | A | CR for Rx-Tx time difference tests for UE category M1/M2 (R15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5214 |  | A | CR for transmit timing accuracy tests for UE category M2 (R15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5222 |  | A | CR for feMTC inter-frequency HO test cases CEModeA R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5224 |  | A | CR for feMTC inter-frequency HO test cases CEModeB R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5226 |  | A | CR for feMTC intra-frequency HO test cases CEModeA without SFN acquisition R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5228 |  | A | CR for feMTC intra-frequency HO test cases CEModeB without SFN acquisition R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172581 | 5230 |  | A | CR on measurement gap enhancement | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5231 |  | A | CR on absolute RSRP accuracy for R15 non-BL/CE UE (The CR was not implemented as it is replaced by CR 5472) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172607 | 5236 | 1 | A | Correction on coverage enhancement level for cat-M1 in IDLE state | 15.1.0 |
| 2017-12 | RAN#78 | RP-172607 | 5238 | 1 | A | Correction of cell reselection margin for Cat-M1 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172607 | 5240 | 1 | A | Removal of squarebrackets from the cat-M2 timing requirements | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5243 | 1 | A | Correction on coverage enhancement level for cat-M1 in IDLE state for Rel-15 MTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5246 | 1 | A | Correction to Rel-15 cat-M1 cell re-selection and RRC re-establishment test cases | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5250 |  | A | FD-FDD Inter-freuency cell re-selection test for cat-M1 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5251 |  | A | HD-FDD Inter-frequency cell re-selection test for cat-M1 UEs in normal coverage | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5252 |  | A | TDD-TDD Inter-freuency cell re-selection test for cat-M1 UEs in normal coverage | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5256 |  | A | FD-FDD Inter-freuency RRC re-establishment est for cat-M1 UEs in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5257 |  | A | HD-FDD Inter-freuency RRC re-establishment est for cat-M1 UEs in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5258 |  | A | TDD-TDD Inter-freuency RRC re-establishment est for cat-M1 UEs in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5265 |  | A | E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5266 |  | A | E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5267 |  | A | E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5268 |  | A | E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5269 |  | A | E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5270 |  | A | E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5295 | 3 | B | Clarification of LTE requirements applicability for NSA NR | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5305 | 2 | B | CR on NR PSCell Addition and Release Delay | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5307 |  | A | CR on 36.133 Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1 CE Mode B | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5308 |  | A | CR to Correct Pool Configuration and FRC for CBR test | 15.1.0 |
| 2017-12 | RAN#78 | RP-172607 | 5313 |  | A | Correction of OCNG for LAA Test cases A.8.26.x, A.9.1.x, A.9.2.x | 15.1.0 |
| 2017-12 | RAN#78 | RP-172609 | 5318 |  | A | Add 5MHz eCell Channel BW scenario in NB-IoT Random Access Test cases | 15.1.0 |
| 2017-12 | RAN#78 | RP-172609 | 5320 |  | A | Add 5MHz eCell Channel BW scenario in NB-IoT Transmit Timing Accuracy Test case | 15.1.0 |
| 2017-12 | RAN#78 | RP-172609 | 5322 |  | A | Add 5MHz eCell Channel BW scenario in NB-IoT RLM In-sync Test cases | 15.1.0 |
| 2017-12 | RAN#78 | RP-172609 | 5325 |  | A | Clarification on the SNR transition in NB-IoT RLM test for out-of-sync | 15.1.0 |
| 2017-12 | RAN#78 | RP-172607 | 5328 |  | A | Correction to Test Parameters for FS3 Channel Occupancy tests | 15.1.0 |
| 2017-12 | RAN#78 | RP-172586 | 5333 |  | A | CR on V2X requirements for asynchronous SyncRef UE Selection / Reselection (Rel-15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5335 | 1 | B | CR on Clarification for MTTD and MTRD for shortened TTI | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5337 |  | A | CR on requirement applicability for R15 non-BL/CE UE | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5339 | 2 | B | CR on TS36.133 on interruptions for NSA LTE NR EN-DC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5340 | 1 | B | Introduction of NR references, definitions and abbreviations in 36.133 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5341 | 1 | B | TX timing requirements for sTTI and reduced processing time | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5342 |  | B | CGI interruption requirements for sTTI and processing time reduction | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5343 |  | B | ProSe interruption requirements for sTTI and processing time reduction | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5344 | 1 | B | Transmit timing adjustment delay | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5345 | 1 | B | STTI definition for 36.133 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172594 | 5346 |  | B | CR to 36.133: Introduction of Band 49 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5348 |  | A | Clarification on UE synchronization behaviour during the evaluation of GNSS reliability | 15.1.0 |
| 2017-12 | RAN#78 | RP-172611 | 5358 |  | A | RRM NB-IoT: Correction to transmit timing accuracy test under enhanced coverage (Rel-15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172613 | 5361 |  | A | RRM 4DL: Correction of cell powers for TC A.9.2.46 (Rel-15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172608 | 5366 |  | A | CR for RLM tests for non-BL/CE UE (R15) | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5367 | 1 | B | Introduction of MRTD/MTTD requirement for sTTI | 15.1.0 |
| 2017-12 | RAN#78 | RP-172608 | 5370 | 1 | A | Introducing PBCH repetition for eMTC RRM test case | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5378 |  | A | Clarification on measurement reporting delay for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5380 |  | A | Inter-freuency cell re-selection test for cat-M1 in enhanced coverage | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5382 |  | B | Inter-freuency RRC re-establishment est for cat-M1 UEs in CEModeB | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5386 |  | B | E-UTRAN FD-FDD Enhanced RLM Tests for Cat-M1 UE in CEModeB | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5387 |  | B | E-UTRAN HD-FDD Enhanced RLM Tests for Cat-M1 UE in CEModeB | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5388 |  | B | E-UTRAN TDD Enhanced RLM Tests for Cat-M1 UE in CEModeB | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5390 |  | A | CR on Autonomous Resource Selection/Reselection Measurement Tests for V2X R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172610 | 5393 |  | A | Correctoin on cell reselectoin test case | 15.1.0 |
| 2017-12 | RAN#78 | RP-172610 | 5396 |  | A | Correctoin on random access test case | 15.1.0 |
| 2017-12 | RAN#78 | RP-172610 | 5399 |  | A | Correction on NPRACH configuration in RRC re-establishment test cases | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5401 |  | A | Test case for random access on non-anchor carrier | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5403 |  | A | Update the maximum DRX cycle length in connected mode | 15.1.0 |
| 2017-12 | RAN#78 | RP-172606 | 5407 |  | A | Correct the core requirements refered in multiple TDD-TDD inter-frequency measurements test cases. | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5432 |  | A | FDD cell identification test case for FeMTC in CEModeB with discontinuous MPDCCH monitoring R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5433 |  | A | HD-FDD cell identification test case for FeMTC in CEModeB with discontinuous MPDCCH monitoring R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5434 |  | A | TDD cell identification test case for FeMTC in CEModeB with discontinuous MPDCCH monitoring R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5435 |  | A | FDD cell identification test case for FeMTC in CEModeB in DRX R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5436 |  | A | HD-FDD cell identification test case for FeMTC in CEModeB in DRX R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5437 |  | A | TDD cell identification test case for FeMTC in CEModeB in DRX R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5438 |  | A | FDD RSRP accuracy test case for FeMTC in CEModeB R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172578 | 5439 |  | A | HD-FDD RSRP accuracy test case for FeMTC in CEModeB R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5440 |  | A | TDD RSRP accuracy test case for FeMTC in CEModeB R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5444 |  | A | CR on E-CID for eNB-IOT normal coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5445 |  | A | CR on E-CID for eNB-IOT enhanced coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5448 |  | A | CR for intra RSTD accuracy test case for eNB-IOT positioning in normal coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5449 |  | A | CR for intra RSTD accuracy test case for eNB-IOT positioning in enhanced coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5450 |  | A | CR for inter RSTD accuracy test case for eNB-IOT positioning in normal coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5451 |  | A | CR for inter RSTD accuracy test case for eNB-IOT positioning in enhanced coverage R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172585 | 5452 |  | A | CR on NB-IOT RSTD requirement R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5455 |  | A | CR on HD-FDD requirement for FeMTC R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5456 |  | B | CR on sTTI related definitions | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5458 | 1 | B | CR on SCell activation and deactivation for sTTI and Spt | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5459 | 1 | B | CR on Measurement reporting delay for shorten TTI and reduced processing time | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5460 |  | B | Power headroom with shorten TTI and reduced processing time | 15.1.0 |
| 2017-12 | RAN#78 | RP-172587 | 5461 |  | B | CR on DRX state for sTTI | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5463 | 2 | B | CR for TS36.133 on inter-RAT measurement requirement for NR | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5467 | 1 | B | CR on measurement gap patterns in TS 36.133 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5468 | 3 | B | CR on event triggering and reporting criteria for NR | 15.1.0 |
| 2017-12 | RAN#78 | RP-172580 | 5470 |  | A | Correction of placement of NRSRQ mapping table | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5472 |  | A | CR on introduction of measurement requirements for non-BL CE UE | 15.1.0 |
| 2017-12 | RAN#78 | RP-172575 | 5474 |  | A | CR on introduction of remaining measurement requirements for non-BL CE UE in CE Mode B | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5475 | 3 | B | Introduction of requirement on EN-DC SSTD measurements | 15.1.0 |
| 2017-12 | RAN#78 | RP-172574 | 5477 |  | A | Applicability of FeMTC RRM Test Cases | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5478 | 1 | B | E-UTRA Inter-frequency Measurement Requirements for NSA Operation | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5479 | 1 | B | LTE and NR cell naming convention in 36.133 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5480 | 1 | B | CR for 36.133 on Number of carriers being monitored | 15.1.0 |
| 2017-12 | RAN#78 | RP-172571 | 5483 | 2 | B | Endorsed CR on NR PSCell Addition and Release Delay with modifications | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5496 |  | A | E-UTRAN FD-FDD intra-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5497 |  | A | E-UTRAN HD-FDD intra-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5498 |  | A | E-UTRAN TDD intra-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5499 |  | A | E-UTRAN FD-FDD intra-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5500 |  | A | E-UTRAN HD-FDD intra-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5501 |  | A | E-UTRAN TDD intra-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5502 |  | A | E-UTRAN FD-FDD inter-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5503 |  | A | E-UTRAN HD-FDD inter-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5504 |  | A | E-UTRAN TDD inter-frequency RSTD measurement period for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172576 | 5505 |  | A | E-UTRAN FD-FDD inter-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5506 |  | A | E-UTRAN HD-FDD inter-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172577 | 5507 |  | A | E-UTRAN TDD inter-frequency RSTD measurement accuracy for FeMTC | 15.1.0 |
| 2017-12 | RAN#78 | RP-172579 | 5510 | 1 | A | Reference correction | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5514 |  | A | CR on inter frequency measurement requirement R15 | 15.1.0 |
| 2017-12 | RAN#78 | RP-172583 | 5516 |  | A | CR on UE Rx-Tx measurement requirement R15 | 15.1.0 |
| 2018-03 | RAN#79 | RP-180264 | 5518 | 4 | F | CR on TS36.133 on interruptions for NSA EN-DC | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5521 | 2 | F | Number of carriers | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5524 | 1 | F | Editorial corrections for NSA NR | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5526 | 1 | F | CR on NR PSCell addition and release delay | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5528 |  | A | Remove [ ] from UE Cat 1bis RRM requirements (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180291 | 5530 |  | A | Correction of test requirement for LAA Test cases A.8.26.x | 15.2.0 |
| 2018-03 | RAN#79 | RP-180293 | 5536 |  | A | Correction to Reference NPRACH Configurations | 15.2.0 |
| 2018-03 | RAN#79 | RP-180293 | 5539 |  | A | Finalising NB-IoT NPRACH Test cases A.6.2.16 and A.6.2.17 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180293 | 5542 |  | A | Correction to NB-IoT UE Transmit Timing Accuracy Test cases | 15.2.0 |
| 2018-03 | RAN#79 | RP-180291 | 5545 | 1 | A | Correction to successful report rate threshold for FS3 channel occupancy tests | 15.2.0 |
| 2018-03 | RAN#79 | RP-180291 | 5548 |  | A | Correction to incorrect reference for A8.26.9 and A8.26.10 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180291 | 5551 |  | A | Correction to OCNG pattern for FS3 RSSI and channel occupancy tests | 15.2.0 |
| 2018-03 | RAN#79 | RP-180297 | 5553 | 1 | A | CR for applibility correction for non-BLCE UE R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180299 | 5555 |  | A | Correction of test requirements for RSTD intra-frequency reporting delay for Cat1Bis (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180299 | 5557 |  | A | Correction of test requirements for RSTD inter-frequency reporting delay for Cat1Bis (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180299 | 5559 |  | A | Correction of test requirements for RSTD measurement performance for Cat1Bis (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180299 | 5561 |  | A | RRM-Cat1Bis: Missing references to the core requirements (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5563 |  | A | Corrections on enhanced RLM tests in CEModeA (R15) The CR was partially implemented as it was not based on the latest version of the spec | 15.2.0 |
| 2018-03 | RAN#79 | RP-180281 | 5566 |  | A | Correction for LAA channel occupancy test (R15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180294 | 5569 |  | A | Correction for LAA RSSI measurement accuracy requirement (R15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5571 | 1 | F | Requirements for LTE incmon with NR | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5573 |  | A | Changes to conditions for V2X GNSS reliability requirements | 15.2.0 |
| 2018-03 | RAN#79 | RP-180279 | 5575 |  | B | CR to 36.133: Introduction of Band 85 (B12-extended) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180297 | 5577 |  | A | Updating HO requirement without SFN acquisition for feMTC R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5579 |  | A | Updating HO test cases without SFN acquisition for feMTC CEModeA R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5581 |  | A | Updating HO test cases without SFN acquisition for feMTC CEModeB R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5582 | 1 | F | Interruption time for MG in EN-DC | 15.2.0 |
| 2018-03 | RAN#79 | RP-180297 | 5591 |  | A | CR on RRM measurement requirements for UE cat M1 R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5593 | 1 | A | CR on intra RSTD accuracy requirement for FeMTC R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180295 | 5595 | 1 | A | CR on non-uniform gap inter frequency requirements when DRX is used R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5596 |  | F | CR on TS36133 for event triggering and reporting criteria | 15.2.0 |
| 2018-03 | RAN#79 | RP-180295 | 5598 |  | A | Correction on interruption test cases for SRS switching in CA R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180295 | 5600 |  | A | Correction on interruption test cases for SRS switching in DC R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5601 |  | F | CR on measurement gap patterns in TS 36.133 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180295 | 5603 | 1 | A | Clean-up for high speed performance enhancement | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5605 |  | A | CR on introducing PSSCH RMC configuration for V2X UE autonomous resource selection/reselection measurement tests R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180293 | 5613 |  | A | Editorial change on RLM requirement | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5618 |  | A | CR for intra frequency RSTD reporting delay test case for eNB-IOT positioning R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5619 |  | A | CR for inter frequency RSTD reporting delay test case for eNB-IOT positioning R15 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5622 | 1 | B | CR on TS36.133 for UE capability in idle mode | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5624 |  | F | CR on TS36.133 for NR RRM measurement in EN-DC | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5625 | 1 | F | CR on TS36.133 for inter-frequency E-UTRAN RRM measurement in EN-DC | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5628 | 2 | B | CR on TS36.133 for inter-RAT handover from E-UTRAN to NR | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5631 | 1 | F | CR on TS36.133 for UE measurement capability related to EN-DC operation | 15.2.0 |
| 2018-03 | RAN#79 | RP-180264 | 5632 | 1 | F | CR on TS36.133 for EN-DC SSTD measurement | 15.2.0 |
| 2018-03 | RAN#79 | RP-180294 | 5642 |  | A | CR for 36.133 introducing band 68 | 15.2.0 |
| 2018-03 | RAN#79 | RP-180293 | 5645 |  | A | Clarification on coverage enhancement level for cat-NB1 in IDLE state | 15.2.0 |
| 2018-03 | RAN#79 | RP-180297 | 5647 |  | A | Clarification on enhanced RLM requirements for FeMTC | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5649 | 1 | A | Editorial correction to test cases on SI reading test in DRX for cat-M1 in CEModeB | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5656 |  | A | Editorial correction in RSTD requirements for UE category 1bis | 15.2.0 |
| 2018-03 | RAN#79 | RP-180296 | 5658 |  | A | OTDOA NB-IoT: Corrections to core and test requirements for NB-IOT Positioning tests (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180298 | 5660 |  | A | OTDOA eMTC: Corrections to core and test requirements for eMTC Positioning tests (Rel-15) | 15.2.0 |
| 2018-03 | RAN#79 | RP-180278 | 5661 |  | B | Introduction of TDD 3.3-3.4GHz band (band 52) | 15.2.0 |
| 2018-06 | RAN#80 | RP-181075 | 5687 |  | B | CR for 36.133 on UE measurement capability for NR SA UE without NR PCell | 15.3.0 |
| 2018-06 | RAN#80 | RP-181075 | 5782 |  | B | On TDD inter-RAT SFTD measurement requirements | 15.3.0 |
| 2018-06 | RAN#80 | RP-181075 | 5831 | 1 | B | CR for 36.133 Clarification on the total number of carrier frequency layers | 15.3.0 |
| 2018-06 | RAN#80 | RP-181075 | 5735 | 4 | B | Introduction of inter-RAT SFTD measurement requirement | 15.3.0 |
| 2018-06 | RAN#80 | RP-181075 | 5783 | 1 | B | On inter-RAT SFTD interruption requirements | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5719 |  | F | CR on TS36.133 for NR RRM measurement in EN-DC | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5666 | 1 | F | Applicability of EN-DC requirements in 36.133 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5761 |  | F | CR on short gap for LTE measurement in TS36.133 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5790 |  | F | CR on inter-frequency measurement in EN-DC for 36.133 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5800 | 1 | F | CR on TS36.133 for NR PSCell addition delay | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5799 | 2 | F | CR on TS36.133 for inter-RAT 3G measurement in EN-DC | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5759 | 2 | F | CR on TS36.133 on interruptions for NSA EN-DC | 15.3.0 |
| 2018-06 | RAN#80 | RP-181076 | 5760 | 2 | F | CR on TS36.133 for inter-RAT handover from E-UTRAN to NR | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5772 | 1 | F | CR on interruption for SCell activation and deactivation with sTTI | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5757 | 1 | D | Editorial corrections to STTI and short processing time requirements | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5758 | 2 | B | ONCG and RMC definitions for slot and subslot RRM testing in LTE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5791 | 2 | B | Test case principle for sTTI | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5792 | 2 | B | E-UTRAN FDD – UE Timing Advance Adjustment Delay Test for sTTI and processing time reduction | 15.3.0 |
| 2018-06 | RAN#80 | RP-181077 | 5833 |  | B | E-UTRAN TDD – UE Timing Advance Adjustment Delay Test for sTTI and processing time reduction | 15.3.0 |
| 2018-06 | RAN#80 | RP-181079 | 5665 | 1 | B | CR on interruption and addition/release requirement for V2X carrier aggregation | 15.3.0 |
| 2018-06 | RAN#80 | RP-181079 | 5788 | 2 | B | CR on Synchronization Reference Source Selection/Reselection requirements for V2X CA | 15.3.0 |
| 2018-06 | RAN#80 | RP-181079 | 5769 | 2 | F | CR on V2X CA requirements corrections | 15.3.0 |
| 2018-06 | RAN#80 | RP-181082 | 5817 | 3 | B | Introducing RRC_INACTIVE mode mobility requirements for 36133 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5754 |  | B | RSTD measurement requirements with new gaps | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5826 |  | B | Introduction of High-velocity support for muting for Rel-15 MTC | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5803 | 1 | F | CR on CGI requirements for CEmodeB | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5805 | 1 | F | CR on new gap impact on intra-frequency RSTD requirements for M1 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5806 | 1 | F | CR on new gap impact on intra-frequency RSTD requirements for M2 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5753 | 2 | B | Introduction of measurement gaps for dense PRS | 15.3.0 |
| 2018-06 | RAN#80 | RP-181083 | 5825 | 3 | B | Introduction of CRS muting requirements for Rel-15 MTC | 15.3.0 |
| 2018-06 | RAN#80 | RP-181084 | 5717 | 1 | B | CR for relaxed monitoring of cell reselection | 15.3.0 |
| 2018-06 | RAN#80 | RP-181084 | 5796 |  | B | CR for TDD NB-IoT RRM requirement | 15.3.0 |
| 2018-06 | RAN#80 | RP-181084 | 5827 |  | B | Side conditions for supporting TDD NB-IOT | 15.3.0 |
| 2018-06 | RAN#80 | RP-181084 | 5797 | 1 | B | CR for serving cell measurement relaxation for WUS-capable UE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181085 | 5765 | 4 | B | Introduction of network-based CRS interference mitigation | 15.3.0 |
| 2018-06 | RAN#80 | RP-181093 | 5736 |  | B | Introduction of 1UL and more than 5DL CA into 36.133 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181100 | 5756 | 1 | F | Introduction of generic duplex modes test cases for 3/4/5DL CA | 15.3.0 |
| 2018-06 | RAN#80 | RP-181100 | 5762 | 1 | F | Introduction of generic duplex modes test cases on RSRP and RSRQ accuracy for 3/4/5DL CA | 15.3.0 |
| 2018-06 | RAN#80 | RP-181100 | 5743 | 1 | F | New generic TC: 4DL CA Activation and Deactivation | 15.3.0 |
| 2018-06 | RAN#80 | RP-181100 | 5744 | 1 | F | New generic TC: 5DL CA Activation and Deactivation | 15.3.0 |
| 2018-06 | RAN#80 | RP-181100 | 5742 | 2 | F | New generic TC: 3DL CA Activation and Deactivation | 15.3.0 |
| 2018-06 | RAN#80 | RP-181110 | 5673 |  | A | Correction of test parameters for LAA Test cases A.9.1.60 and A.9.1.61 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181110 | 5679 |  | A | Specify Measurement BW for LAA Test cases A.8.26.3/4 and A.8.26.9/10 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181110 | 5750 |  | A | Correction to Test Parameters for FS3 Channel Occupancy tests | 15.3.0 |
| 2018-06 | RAN#80 | RP-181111 | 5775 |  | A | Correction to eMTC CGI reading delay requirement R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181112 | 5670 |  | A | Remove [ ] from Physical channels for NB-IoT Test case A.6.1.16 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181112 | 5676 |  | A | Update parameters for NB-IoT Tx Timing Test case A.7.1.18 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181112 | 5778 |  | A | Correction to the delay requirement for RRC connection redirection to non-anchor carrier for NB-IoT R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181113 | 5664 |  | A | Editorial changes to single carrier RLM test case for 4 Rx capable Ues R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181113 | 5747 |  | A | Corrections to CA activation and deactivation test cases (Rel-15) | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5691 |  | A | Intra-frequency RSTD measurement period requirements with gaps for Cat M1 UE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5693 |  | A | Intra-frequency RSTD measurement period requirements with gaps for Cat M2 UE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5695 |  | A | Correction for intra-frequency RSTD accuracy requirements with gaps for Cat M1 UE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5697 |  | A | Correction for intra-frequency RSTD accuracy requirements with gaps for Cat M2 UE | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5709 | 1 | A | CR on intra-frequency RSTD measurement requirements for UE cat M1 in CE mode B | 15.3.0 |
| 2018-06 | RAN#80 | RP-181114 | 5711 | 1 | A | CR on intra-frequency RSTD measurement requirements for UE cat M2 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181115 | 5725 |  | A | OTDOA NB-IoT: Corrections to test requirements for NB-IOT Positioning tests (Rel-15) | 15.3.0 |
| 2018-06 | RAN#80 | RP-181115 | 5699 | 1 | A | Correction in SRS switching requirements | 15.3.0 |
| 2018-06 | RAN#80 | RP-181115 | 5752 |  | A | Remaining square brackets in eNB-IoT RSTD requirements (Rel-15) | 15.3.0 |
| 2018-06 | RAN#80 | RP-181115 | 5787 |  | A | CR on modification of GNSS reliability requirements for V2X R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181115 | 5795 |  | A | CR on NB-IoT test case Random Access on Non-anchor Carrier (A.6.2.18) R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181116 | 5768 | 1 | A | Correction to Category 1bis test case and requirement R15 | 15.3.0 |
| 2018-06 | RAN#80 | RP-181116 | 5785 | 1 | A | CR on modification of interruption requirement for V2V R15 | 15.3.0 |
| 2018-09 | RAN#81 | RP-181897 | 5840 |  | F | Number of frequency layers for inter-frequency RSTD in NSA NR in 36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5843 |  | F | Correcting inactive state requirements in 36133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5842 | 1 | F | Correcting UE measurement capability with same MO configured by MN and SN on TS36.133 (The CR was not implemented as it is replaced by CR 5897r1) | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5837 | 2 | F | CR 36.133 Correction of EN-DC SFTD core requirement in DRX | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5835 | 1 | F | Interuption requirements for EN-DC in 36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5841 | 2 | F | CR on inter-RAT NR measurements iin idle in 36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5848 | 2 | B | Interruption Requirements on LTE Serving Cells due to BWP Switching | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5856 |  | F | Interuption requirements for EN-DC in 36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5866 |  | F | CR to TS36.133 for editorial correction of NR NSA measurement capability | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5868 |  | F | Introduction on Gap sharing for EN-DC in TS36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5886 |  | F | Correction of EN-DC SFTD core requirement | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5885 | 1 | F | Clarification of inter-RAT SFTD core requirement | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5897 | 1 | F | Correcting UE measurement capability with same MO configured by MN and SN on TS36.133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5940 | 2 | F | CR on NR PScell addition | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5857 | 1 | F | CR on TS36.133 for handover from E-UTRAN to NR | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5909 | 1 | B | CR to 36.133 on introduction of SFTD measurement accuracy requirements | 15.4.0 |
| 2018-09 | RAN#81 | RP-181897 | 5867 | 3 | F | CR on TS36.133 for inter-RAT NR measurements in idle | 15.4.0 |
| 2018-09 | RAN#81 | RP-181899 | 5913 |  | F | Indicator for high-speed operation for Rel-15 cat-M1/M2 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181899 | 5879 | 1 | B | Clarification of the requirement for dense PRS gap in eFeMTC UE R15 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181899 | 5891 | 1 | F | CR on RLM requirements for UE configured with dense PRS gaps | 15.4.0 |
| 2018-09 | RAN#81 | RP-181899 | 5936 | 1 | F | Updated requirements for measurement gaps for dense PRS | 15.4.0 |
| 2018-09 | RAN#81 | RP-181899 | 5916 | 1 | F | Remaining work on CRS muting for Cat-M1/M2 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181900 | 5930 | 1 | B | Addition of NSSS-based RRM measurement accuracy requirement | 15.4.0 |
| 2018-09 | RAN#81 | RP-181900 | 5892 | 1 | F | CR on WUS RRM requirements | 15.4.0 |
| 2018-09 | RAN#81 | RP-181903 | 5858 | 1 | F | CR on Interruption and Delay of CC addition/release | 15.4.0 |
| 2018-09 | RAN#81 | RP-181904 | 5895 |  | F | Correcting UE measurement capability in Inactive state for 36133 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181905 | 5876 | 1 | B | Addition of Scell activation/interruption requiremetn for dormant SCell | 15.4.0 |
| 2018-09 | RAN#81 | RP-181905 | 5944 | 2 | B | CR introducing enhanced utilization of CA and direct activation | 15.4.0 |
| 2018-09 | RAN#81 | RP-181905 | 5942 | 2 | B | CR introducing enhanced utilization of CA and idle mode measurements for early reporting | 15.4.0 |
| 2018-09 | RAN#81 | RP-181906 | 5931 | 1 | F | Applicability requirement for network-based CRS-IM | 15.4.0 |
| 2018-09 | RAN#81 | RP-181911 | 5861 |  | A | Correction of test parameters for LAA Test cases A.9.1.60 and A.9.1.61 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181911 | 5872 |  | A | Clarification of the measurement gap offset in LAA inter-frequency measurement test R15 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181912 | 5917 |  | A | Correcton to category M1 PDSCH RMCs | 15.4.0 |
| 2018-09 | RAN#81 | RP-181913 | 5914 |  | A | Correction on gap ID usage for UE category M1/M2 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181913 | 5935 |  | A | Correction in inter-frequency RSTD measurement period requirements in FeMTC | 15.4.0 |
| 2018-09 | RAN#81 | RP-181914 | 5901 |  | A | CR on modification of V2X initiation/cease of SLSS transmission test for eNB as Timing Reference R15 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181914 | 5903 |  | A | CR on modification of V2X initiation/cease of SLSS transmission test for SyncRef UE as Timing Reference R15 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181914 | 5905 |  | A | CR on modification of V2X congestion control measurement test R15 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181915 | 5853 |  | A | Corrections to NB-IoT RSTD test cases (Rel-15) | 15.4.0 |
| 2018-09 | RAN#81 | RP-181915 | 5855 |  | A | Correct 5MHz eCell PRB# for Cat NB1 Test Case A.4.2.18 | 15.4.0 |
| 2018-09 | RAN#81 | RP-181915 | 5874 |  | A | Introduction of MSG3-based channel quality report for NB-IoT | 15.4.0 |
| 2018-09 | RAN#81 | RP-181917 | 5925 |  | B | Introduction of Event Triggered Reporting test cases with generic duplex modes for 6DL/7DL CA | 15.4.0 |
| 2018-09 | RAN#81 | RP-181917 | 5926 |  | B | Introduction of Activation and Deactivation test cases with generic duplex modes for 6DL/7DL CA | 15.4.0 |
| 2018-09 | RAN#81 | RP-181917 | 5927 |  | B | Introduction of generic duplex modes test cases on RSRP accuracy for 6DL/7DL CA | 15.4.0 |
| 2018-09 | RAN#81 | RP-181917 | 5928 |  | B | Introduction of generic duplex modes test cases on RSRQ accuracy for 6DL/7DL CA | 15.4.0 |
| 2018-09 | RAN#81 | RP-181917 | 5851 | 1 | F | Corrections to generic CA RRM test cases (Rel-15) | 15.4.0 |
| 2018-12 | RAN#82 | RP-182360 | 5946 |  | F | Interuption requirements correction for EN-DC in 36.133 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 5948 |  | F | CR on UE measurement capability with MOs configured by MN and SN in TS36.133 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 5954 |  | A | Finalize MSG3-based channel quality report for NB-IoT | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 5955 | 1 | B | RMC for NB-IoT TDD RRM test cases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5956 | 1 | F | Addition of side condition for CGI reading delay for Rel-15 eFeMTC UE | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5959 | 1 | F | Applicability rules for UE supporting new gaps | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5960 | 2 | B | Intra-frequency FDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5961 | 1 | B | Intra-frequency HD-FDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5962 | 1 | B | Intra-frequency TDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5964 | 1 | B | Inter-frequency HD-FDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5965 | 1 | B | Inter-frequency TDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182383 | 5967 | 1 | A | Correction in RSTD measurement requirements | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 5968 | 2 | F | CR to 36.133 on SFTD measurement requirements (Section 8.17.2, 8.1.2.4.25) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 5983 | 3 | F | Requirements for high velocity measurement for efeMTC | 15.5.0 |
| 2018-12 | RAN#82 | RP-182361 | 5987 |  | F | CR on interruption during DRX operation in TS 36.133 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182383 | 6001 |  | A | Correction to inter-frequency handover test case for UE category M1/M2 in CEModeB | 15.5.0 |
| 2018-12 | RAN#82 | RP-182361 | 6017 | 1 | F | CR 36.133 Inter-RAT and EN-DC SFTD measurement accuracy for FR2 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182379 | 6024 |  | A | CR 36.133 Correction of references in OCNG patterns Rel-15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6026 |  | A | CR 36.133 Correction of NB-IoT OCNG patterns Rel-15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6027 | 1 | B | Introduction of OCNG patterns for NB-IoT TDD | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6028 | 1 | F | UE known dormant SCell condition and activation delay requirements for unknown dormant SCell | 15.5.0 |
| 2018-12 | RAN#82 | RP-182384 | 6030 |  | A | Correction to downlink channel quality reporting R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182371 | 6032 | 1 | B | RRM test for dormant SCell | 15.5.0 |
| 2018-12 | RAN#82 | RP-182384 | 6035 |  | A | Correction to RSTD measurement accuracy requirement in NB-IoT R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182362 | 6038 | 4 | B | CR for 36.133 on E-UTRAN measurement to support gap pattern 4, 6,7,8,10 (section 8.1.2.1, 8.1.2.3.1.2, 8.1.2.3.2.2, 8.17.3) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6039 |  | B | Inter-frequency FDD test cases for RSTD measurement period under new measurement gaps for CE Mode A | 15.5.0 |
| 2018-12 | RAN#82 | RP-182361 | 6040 |  | F | CR for correction on measurement gap sharing in EN-DC | 15.5.0 |
| 2018-12 | RAN#82 | RP-182381 | 6043 |  | A | Correction to FDD-FS3 Intra-frequency event triggered reporting in DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182388 | 6044 |  | F | Correction to DL CA Activation and Deactivation Test cases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182384 | 6051 | 1 | A | NB-IoT RSTD accuracy tests using Type 2 NPRS - Rel15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6052 | 2 | F | CR on updating requirement for BWP switching interruption in TS36.133 (Section 7.32.2.7) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182382 | 6058 |  | A | Corrections to feMTC RRM test case A.8.1.28 (Rel-15) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182388 | 6061 |  | F | Corrections to generic CA RRM test cases up to 5 DL CCs (Rel-15) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182388 | 6062 |  | F | Corrections to generic CA RRM test cases for 6 and 7 DL CCs (Rel-15) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6065 |  | B | Introducing TDD NPRACH configuration for NB test cases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6066 | 1 | B | TDD contention based random access test case under normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6067 | 1 | B | TDD contention based random access test case under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6068 | 1 | B | TDD contention based random access test case on non-anchor carriers | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6069 | 1 | B | TDD intra-frequency RSTD measurement accuracy test case under normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6070 | 1 | B | TDD inter-frequency RSTD measurement accuracy test case under normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6071 | 1 | B | TDD intra-frequency RSTD measurement accuracy test case under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6072 | 1 | B | TDD inter-frequency RSTD measurement accuracy test case under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6073 | 2 | B | TDD idle intra-frequency RSTD measurement test case under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6074 | 2 | B | TDD idle inter-frequency RSTD measurement test case under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6075 |  | F | CR on TDD support in conditions for NB-IoT measurements | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6076 | 2 | F | CR on TDD support in measurement accuracy requirements for NB-IoT | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6079 | 1 | A | CR on NRSRP accuracy applicability for UE Category NB1 and NB2 R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6087 |  | A | CR on NB random access test cases R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182370 | 6089 | 1 | B | Introducing principle for LTE UE connected to 5GC test cases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182361 | 6092 | 1 | F | CR on PSCell addition delay in TS36.133 (The CR was not implemented as it is not based on the latest version of the specification) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182361 | 6093 |  | F | Defining MG starting point in 36.133 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6095 |  | A | Introduction of MSG3-based channel quality reporting test | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6097 | 1 | B | CR introducing out-of-sync RLM test for NB-IoT TDD in normal coverage with DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6098 | 1 | B | CR introducing out-of-sync RLM test for NB-IoT TDD in enhanced coverage with DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6099 | 1 | B | CR introducing in-sync RLM test for NB-IoT TDD in normal coverage with DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6100 | 1 | B | CR introducing in-sync RLM test for NB-IoT TDD in enhanced coverage with DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6101 | 1 | B | CR introducing in-sync RLM test for NB-IoT TDD in normal coverage without DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6102 | 1 | B | CR introducing in-sync RLM test for NB-IoT TDD in enhanced coverage without DRX | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6103 | 1 | B | CR introducing Out-of-sync RLM test for NB-IoT TDD in normal coverage without DRX in stand alone mode | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6104 | 1 | B | CR introducing Out-of-sync RLM test for NB-IoT TDD in enhanced coverage without DRX in guard band | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6105 | 1 | F | UE known dormant SCell condition | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6108 | 1 | F | Introducing enhanced utilization of CA and direct activation in HO | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6113 | 1 | B | Cell re-selection test case for TDD Intra frequency case for UE category NB1 in in-band mode in normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6114 | 1 | B | TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6115 | 1 | B | UE Transmit Timing Accuracy Test for category NB1 UE In-Band in Normal coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6118 | 1 | B | UE Transmit Timing Accuracy Test for category NB1 UE In-Band in Enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182367 | 6119 | 1 | B | UE Timing advance adjustment test Test for category NB1 UE standalone under Enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6120 | 1 | B | TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6121 | 1 | B | Cell re-selection test case for TDD Inter- frequency case for UE category NB1 in in-band mode in enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6122 | 1 | B | Cell re-selection test case for TDD Intra frequency case for UE category NB1 in in-band mode in enhanced coverage | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6124 | 2 | F | Correction of references in CONNECTED mode requirements for category M1 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6127 | 1 | B | Introduction of enhanced PHR for category NB1 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182366 | 6128 |  | B | Remaining work on minimum WUS reception requirements for NB-IoT | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6129 |  | B | Introduction of WUS requirements for efeMTC | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6132 | 1 | F | Addition of missing Band 71 in 36.133 NB-IoT bands | 15.5.0 |
| 2018-12 | RAN#82 | RP-182385 | 6134 |  | A | Corrections on V2X core requirements in TS36.133 R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182385 | 6136 |  | A | Corrections on Conditions for Selection/Reselection to Intra-frequency SyncRef UE R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182382 | 6149 |  | A | Clarification on the applicability of side condition for CE UE R15 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182383 | 6153 |  | A | Applicability of non-BL CE requriements for Cat-1bis (The CR was partially implemented as it is not based on the latest version of the specification) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6155 | 1 | F | Correction to CRS muting applicability in efeMTC | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6156 | 1 | F | Introduction of search window for 4ms MGL in LTE interfrequency requirements (The CR was partially implemented as this CR is clashes with CR#6038r4) | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6157 | 2 | F | Corrections to DRX cycle for EN-DC operation | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6158 | 2 | F | Correction of side condition in FR2 handover in section 6.1.1 | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6160 | 2 | F | SIB5 reporting corrections for Idle Mode CA Measurement | 15.5.0 |
| 2018-12 | RAN#82 | RP-182386 | 6167 | 1 | F | Corrections in network-based CRS interference mitigation requirements | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6168 | 2 | B | Intra-frequency FDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6169 | 2 | B | Intra-frequency HD-FDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6170 | 2 | B | Intra-frequency TDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6171 | 1 | B | Inter-frequency FDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6172 | 1 | B | Inter-frequency HD-FDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182365 | 6173 | 2 | B | Inter-frequency TDD test cases for RSTD measurement period under new measurement gaps for CE Mode B | 15.5.0 |
| 2018-12 | RAN#82 | RP-182383 | 6175 |  | A | Correction in FeMTC RSTD test cases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182362 | 6176 |  | B | Inter-RAT NR measurement accuracy requirements | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6177 |  | F | Side conditions for inter-RAT NR measurements in EN-DC case | 15.5.0 |
| 2018-12 | RAN#82 | RP-182360 | 6178 |  | F | Side conditions for inter-RAT NR measurements in non-EN-DC case | 15.5.0 |
| 2018-12 | RAN#82 | RP-182379 | 6186 |  | A | Corrections to incmon testcases | 15.5.0 |
| 2018-12 | RAN#82 | RP-182369 | 6187 |  | B | CR on introducing test cases of Synchronization Reference Source Selection/Reselection requirements for V2X CA | 15.5.0 |
| 2018-12 | RAN#82 | RP-182369 | 6188 |  | B | CR on introducing test cases of interruptions due to V2X CA | 15.5.0 |
| 2018-12 | RAN#82 | RP-182377 | 6015 |  | B | CR adding_B65 for NB1&NB2 | 16.0.0 |
| 2018-12 | RAN#82 | RP-182376 | 6096 | 1 | B | CR to 36.133: Introduction of Band 53 | 16.0.0 |
| 2019-03 | RAN#83 | RP-190414 | 6192 |  | A | Correction to FDD-FS3 Intra-frequency event triggered reporting in DRX | 16.1.0 |
| 2019-03 | RAN#83 | RP-190413 | 6197 |  | A | Clarifications on HD-FDD NB-IoT Out-of-sync RLM test cases | 16.1.0 |
| 2019-03 | RAN#83 | RP-190413 | 6201 |  | A | Corrections to HD-FDD NB-IoT In-Sync RLM tests | 16.1.0 |
| 2019-03 | RAN#83 | RP-190413 | 6205 |  | A | Clarification on test requirements for transmit timing accuracy for HD-FDD Category NB1 UE | 16.1.0 |
| 2019-03 | RAN#83 | RP-190406 | 6207 |  | A | Clarification on test requirements for transmit timing accuracy for TDD Category NB1 UE | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6210 |  | A | Clarifications to HD-FDD Idle State Positioning Measurement test for NB1 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190406 | 6212 |  | A | Clarifications to TDD Idle State Positioning Measurement test for NB1 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6223 |  | A | Corrections on SFTD R16 (8.1.2.4.26.1) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190411 | 6230 |  | A | Correction to Incmon test cases for reselection from E-UTRA FDD/TDD to UTRA | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6234 |  | A | Inter-RAT NR measurements in 36.133 for SA | 16.1.0 |
| 2019-03 | RAN#83 | RP-190413 | 6246 | 1 | A | CR on NB random access test cases R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6249 | 1 | A | CR on MSG3 based channel quality report requirements R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190406 | 6258 |  | A | CR on Cat NB2 UE test cases applicability R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6260 | 1 | A | CR on BWP switch interruptions on LTE PCell in EN-DC R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190421 | 6262 |  | A | Adding section 8.18 in 36.133 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190417 | 6265 |  | A | Correction to the non-BL UE applicability | 16.1.0 |
| 2019-03 | RAN#83 | RP-190417 | 6268 |  | A | Correction of reference number in Cat-M HO requirements | 16.1.0 |
| 2019-03 | RAN#83 | RP-190421 | 6270 |  | A | Finalizing efeMTC CRS muting applicability | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6274 |  | A | Measurement requirements for NE-DC | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6276 |  | A | CR for measurement gap applicability for NE-DC | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6280 |  | A | CR on E-UTRAN E-UTRAN inter-frequency and inter-RAT measurements with EN-DC in TS36.133 R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190402 | 6282 |  | A | CR on corrections on SFTD measurements accuracy in TS36.133 R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6284 |  | A | CR on gap sharing and CSSF definition in TS36.133 R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190408 | 6288 |  | A | Applicability rule for 8Rx RLM R16 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6291 |  | A | Correction of NB-IoT MSG3-based channel quality report test case | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6293 |  | A | CR on E-UTRA SCell (de)activation interruptions for EN-DC (36.133, Rel-16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6295 |  | A | CR on interruption requirements for NE-DC (36.133, Rel-16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6297 |  | A | CR on E-UTRA SCell (de)activation applicability for NE-DC (36.133) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6304 |  | A | Corrections to NB-IOT RSTD reporting delay core requirements (Rel-16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6307 |  | A | Corrections to NB-IOT RSTD reporting accuracy test requirements for normal coverage (Rel-16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190416 | 6310 |  | A | Corrections to NB-IOT RSTD reporting accuracy test requirements for enhanced coverage (Rel-16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190417 | 6316 | 1 | A | Correction to side conditions for cat-M | 16.1.0 |
| 2019-03 | RAN#83 | RP-190421 | 6320 | 1 | A | Maintenace CR on WUS requirements for cat-M Ues | 16.1.0 |
| 2019-03 | RAN#83 | RP-190420 | 6322 | 1 | A | Maintenace CR on WUS requirements for Rel-16 NB-IoT | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6324 |  | A | Requirements for RRC connection release with redirection delay from LTE to NR | 16.1.0 |
| 2019-03 | RAN#83 | RP-190422 | 6326 | 1 | A | IDLE mode measurement requirements for euCA | 16.1.0 |
| 2019-03 | RAN#83 | RP-190407 | 6328 | 1 | A | Introduction of IDLE mode measurement accuracy requirements | 16.1.0 |
| 2019-03 | RAN#83 | RP-190407 | 6330 | 1 | A | CR introducing test case for enhanced utilization of CA and idle mode measurements for early reporting | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6333 | 1 | A | CR on inter-RAT NR measurements (36.133-rel16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6335 | 1 | A | CR on NR PSCell addition delay in EN-DC (36.133-rel16) | 16.1.0 |
| 2019-03 | RAN#83 | RP-190420 | 6337 |  | A | Clean-up in network-based CRS interference mitigation requirements | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6343 | 1 | A | UE measurements capability in RRC_IDLE for EN-DC capable UE | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6345 | 1 | A | Introduction of NE-DC in 36.133 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6347 | 1 | A | Requirements for NGEN-DC | 16.1.0 |
| 2019-03 | RAN#83 | RP-190414 | 6348 |  | A | CR to 36.133 for corrections on section A.9.10 and section A.9.13 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6349 |  | A | Corrections on SFTD mesurement requirements for EN-DC | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6350 |  | A | Corrections on SFTD accuracy requirements | 16.1.0 |
| 2019-03 | RAN#83 | RP-190399 | 6351 | 1 | A | CR for MO merging in 36.133 | 16.1.0 |
| 2019-03 | RAN#83 | RP-190400 | 6352 |  | A | CR for SFTD in 36.133 | 16.1.0 |
| 2019-06 | RAN#84 | RP-191265 | 6360 |  | A | Corrections to TDD in-sync RLM test cases for NB-IoT | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6362 |  | A | Corrections to TDD inter-frequency idle state positioning measurement test in NB1 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6373 |  | A | CR on Cat NB2 UE test cases applicability R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6379 | 1 | A | Maintenance on TDD inter-frequency re-establishment test cases R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6381 |  | A | Maintenance on side conditions for NSSS measurement accuracy requirements R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6383 |  | A | CR on TDD intra frequency idle RSTD accuracy test cases R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191266 | 6385 |  | A | CR on serving cell measurement relaxation for NB-IoT R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6387 |  | A | Maintenance CR on event triggering and reporting criteria R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6397 |  | A | Introduction of UE measurement capability for NE-DC in 36.133 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6404 |  | A | CR for starting point of measurement gap in LTE, ENDC and NEDC in TS 36.133 R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6410 | 1 | A | CR on PSCell addition delay R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6428 |  | A | Interruption Requirement for RRC based BWP Switching on LTE Serving Cells | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6431 | 1 | A | CR on TS36.133 for UE behavior after MG (Section 8.1.2) | 16.2.0 |
| 2019-06 | RAN#84 | RP-191264 | 6438 |  | A | Correction to side conditions for cat-M | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6443 |  | A | Side condition for NR handover | 16.2.0 |
| 2019-06 | RAN#84 | RP-191267 | 6449 |  | A | Correction to event triggered reporting on Deactivated SCell | 16.2.0 |
| 2019-06 | RAN#84 | RP-191258 | 6453 |  | A | Correction to timing advance adjustment accuracy test for Cat-M1 UE | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6458 |  | A | maintenance CR on redirection requirements R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6460 |  | A | CR for NE-DC interruptions due to BWP switch R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191260 | 6465 |  | A | Maintenance on HD-FDD inter-frequency re-establishment test cases R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6467 |  | A | CR on TDD inter frequency idle RSTD accuracy test cases R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191265 | 6469 |  | A | endorsed CR for serving cell RRM measurement relaxation test case R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6475 |  | A | Requirements for RRC connection release with redirection delay from LTE to NR | 16.2.0 |
| 2019-06 | RAN#84 | RP-191264 | 6478 |  | A | CR introducing test cases for direct activation of Scell | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6480 |  | A | PSCell addition delay in FR2 36.133 rel-16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191256 | 6481 |  | B | Introduction of bands 87 and 88 into TS 36.133 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191264 | 6483 |  | A | Corrections to idle mode CA measurement accuracy test | 16.2.0 |
| 2019-06 | RAN#84 | RP-191264 | 6485 |  | A | Clarification on WUS EPRE in requirements for WUS reception for UE category M1 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6487 |  | A | Correction CR for inter-RAT NR measurement before EN-DC in 36.133 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191260 | 6491 |  | A | CR on threshold for FS3 RSSI and channel occupancy tests R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6493 |  | A | CR on reselection criterion in inter-RAT NR measurements (36.133-rel16) | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6495 | 1 | A | CR to 36.133 on SFTD accuracy | 16.2.0 |
| 2019-06 | RAN#84 | RP-191238 | 6505 |  | A | Correction on the Tiu in handover in TS36.133 R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191260 | 6512 |  | A | Corrections on inter-frequency RS-SINR measurement accuracy test in TS36.133 R16 | 16.2.0 |
| 2019-06 | RAN#84 | RP-191261 | 6518 |  | A | CR for eMTC re-establishment test case | 16.2.0 |
| 2019-06 | RAN#84 | RP-191261 | 6521 |  | A | CR for eMTC RSTD test cases | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6532 |  | A | CR 36.133 Correction to SFTD interruption requirements (Rel-16) | 16.2.0 |
| 2019-06 | RAN#84 | RP-191260 | 6536 |  | A | Corrections to NB-IoT PRACH test cases (Rel-16) | 16.2.0 |
| 2019-06 | RAN#84 | RP-191267 | 6538 |  | A | Adding missing bands in the bands grouping | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6540 | 1 | A | RSTD requirements for NE-DC | 16.2.0 |
| 2019-06 | RAN#84 | RP-191260 | 6543 |  | A | Adding missing UE Rx-Tx time difference measurement mapping for TDD | 16.2.0 |
| 2019-06 | RAN#84 | RP-191239 | 6545 | 1 | A | Reporting criteria for NE-DC and EN-DC | 16.2.0 |
| 2019-09 | RAN#85 | RP-192052 | 6552 |  | A | Correction to NB-IoT RSTD delay test case (Rel 16) | 16.3.0 |
| 2019-09 | RAN#85 | RP-192051 | 6562 | 1 | A | Corrections to test case for HD – FDD Inter frequency reselection case for UE Category NB1 In-Band mode in enhanced coverage | 16.3.0 |
| 2019-09 | RAN#85 | RP-192055 | 6565 |  | A | CR on Cat NB2 UE test cases applicability R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192055 | 6567 |  | A | maintenance CR on synchronized TDD NB test cases R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192055 | 6569 |  | A | CR on NB1 timing test cases R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6577 |  | A | CR on interruptions due to RRC based BWP switch for EN-DC R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6579 |  | A | CR on BWP switch interruptions for NE-DC R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192052 | 6584 |  | A | Clarification of Rmax for NB-IoT MSG3-based channel quality report | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6586 |  | A | CR on applicability of gap in LTE, ENDC and NEDC in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6588 |  | A | CR on default gap sharing scheme in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6590 |  | A | CR on handover requirements in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6592 |  | A | CR on PSCell addition in ENDC R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192041 | 6593 | 1 | B | CR on idle mode requirements in HST scenario | 16.3.0 |
| 2019-09 | RAN#85 | RP-192041 | 6594 | 1 | B | CR on Pcell measurement enhancement requirements in HST scenario | 16.3.0 |
| 2019-09 | RAN#85 | RP-192043 | 6598 | 1 | B | CR of adding LTE B42/B43 for UE category NB1/NB2 and CAT M1/M2 in R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192043 | 6599 | 1 | B | CR of adding LTE B7 for UE category NB1/NB2 in R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6601 | 1 | A | Editorial corrections in 36.133 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192055 | 6602 | 1 | F | Applicability of non-BL CE UE requirements in release 16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192052 | 6606 |  | A | Correction of section numbering in handover requirements | 16.3.0 |
| 2019-09 | RAN#85 | RP-192055 | 6610 |  | A | Clarification of normal and enhanced coverage UE requirements in IDLE mode for Rel-16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192041 | 6611 | 1 | B | UE Autonomous Timing Adjustment Requirement under High Speed Scenario | 16.3.0 |
| 2019-09 | RAN#85 | RP-192041 | 6616 | 1 | B | Measurement requirements for LTE further high speed enhancements | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6618 | 1 | A | Correction of interruption requirements for EN-DC in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6622 | 1 | A | Correction of interruption requirements for NE-DC in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192047 | 6624 | 1 | A | Correction to MG interruption due to NE-DC in 36.133 R16 | 16.3.0 |
| 2019-09 | RAN#85 | RP-192040 | 6625 | 1 | B | CR for Tx timing requirements for PUR transmission | 16.3.0 |
| 2019-09 | RAN#85 | RP-192040 | 6626 |  | B | CR for relaxed serving cell monitoring for Rel-16 eMTC | 16.3.0 |
| 2019-12 | RAN#86 | RP-193043 | 6677 |  | A | Correction to misused variables in intra-frequency cell identification requirements | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6680 | 1 | A | CR on handover 36.133 -R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193043 | 6685 |  | A | CR on clarification on RSSI measurement requirement R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193006 | 6686 | 1 | B | CR to 36.133 on interruption requirements for NR V2X | 16.4.0 |
| 2019-12 | RAN#86 | RP-193026 | 6687 | 1 | B | CR: RRM test definition for Rel-16 HST | 16.4.0 |
| 2019-12 | RAN#86 | RP-193024 | 6688 | 2 | B | CR: RRM core requirement for 5G broadcast | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6691 | 1 | A | CR on E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation in TS36.133 (section 8.17.3) | 16.4.0 |
| 2019-12 | RAN#86 | RP-193026 | 6695 |  | F | Adding the IE flag in R16 HST | 16.4.0 |
| 2019-12 | RAN#86 | RP-193025 | 6696 | 2 | B | CR on RRM requirements for DAPS in femobility | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6701 |  | A | Correction RAN4 requirements for a Dormant Scell | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6703 |  | A | Measurements interruption requirements for an SCell in Dormant state | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6705 |  | A | Correction to interruption requirement introduction to include dormant Scell | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6707 |  | A | Removal of brackets from euCA performance requirements | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6709 |  | A | Correction to test case A.8.16.104 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6711 |  | A | Correction to S-measure threshold applicability in IDLE mode measurements | 16.4.0 |
| 2019-12 | RAN#86 | RP-193025 | 6712 | 1 | B | CR Introduction of handover delay requirements for conditional handover (section 5.1) | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6716 |  | A | CR for 36133 editorial for section 7.6 and 7.31 in Rel-16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6717 |  | A | Mirror CR for editorial changes for section 8.1.2.4.21 – 8.1.2.4.26 and 8.17.4 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193049 | 6724 | 1 | A | Correction of feMTC test cases | 16.4.0 |
| 2019-12 | RAN#86 | RP-193046 | 6726 |  | A | Cat-1bis clarification to RSRQ measurements | 16.4.0 |
| 2019-12 | RAN#86 | RP-193046 | 6731 |  | A | CR on maintaining core requirements and tests for V4X synchronization reference source selection/reselection R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193046 | 6734 |  | A | CR on maintaining interruption tests due to V2X sidelink communication R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193024 | 6735 | 1 | B | CR on numerology switching delay and interruptions requirements with FeMBMS for LTE-based 5G broadcast | 16.4.0 |
| 2019-12 | RAN#86 | RP-193023 | 6744 | 1 | B | CR on serving cell measurement configured with WUS in Rel-16 NB IoT | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6746 |  | A | Correction to direct Scell activation requirements R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6748 |  | A | Correction to direct Scell activation during HO requirements R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6750 |  | A | Adding interruption length requirements for direct Scell activation R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6752 |  | A | Update to interruption requirements due to measurement on dormant Scell R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193047 | 6754 |  | A | Adding interruption length requirements for dormant state transition R16 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6756 |  | A | Correction on inter-frequency RSTD measurement requirements (cat-A) | 16.4.0 |
| 2019-12 | RAN#86 | RP-193008 | 6757 |  | B | Introducing euCA related interruption requirements for EN-DC in 36.133 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193008 | 6758 |  | B | Introducing euCA related interruption requirements for NE-DC in 36.133 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193026 | 6769 |  | B | UE Autonomous Timing Adjustment Requirement under High Speed Scenario | 16.4.0 |
| 2019-12 | RAN#86 | RP-193043 | 6773 |  | A | Clarifications on RSSI and Channel Occupancy measurement requirements in LTE LAA | 16.4.0 |
| 2019-12 | RAN#86 | RP-193023 | 6774 |  | B | Connected mode channel quality report for UE category NB1 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193023 | 6775 |  | B | MSG3-based channel quality report in non-anchor carrier for UE category NB1 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193023 | 6776 |  | B | NB-IoT PUR transmit timing requirements | 16.4.0 |
| 2019-12 | RAN#86 | RP-193043 | 6781 |  | A | Correction of reference in subclause 5.3.2 | 16.4.0 |
| 2019-12 | RAN#86 | RP-193038 | 6783 |  | A | Editorial updates (36.133, section 8.19) | 16.4.0 |
| 2019-12 | RAN#86 | RP-193049 | 6791 |  | A | Applicability of relaxed neighbour cell monitoring for category M1/M2 | 16.4.0 |
| 2020-03 | RAN#87-e | RP-200413 | 6794 | 2 | F | CR on correction of measurement delay requirements for LTE HST in TS36.133 | 16.5.0 |
| 2020-03 | RAN#87-e | RP-200405 | 6795 | 2 | B | CR on logged MDT requirements (2, 4.3) | 16.5.0 |
| 2020-03 | RAN#87-e | RP-200413 | 6796 |  | F | CR to TS 36.133: Finalization on RRM requirements for Rel-16 LTE HST | 16.5.0 |
| 2020-03 | RAN#87-e | RP-200418 | 6818 |  | B | Introduction of Rel-16 WUS reception requirements for cat-M1/M2 | 16.5.0 |
| 2020-06 | RAN#88 | RP-200983 | 6826 |  | A | [CR] RRC release with redirection 36.133 R16 Cat A | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6827 | 1 | B | CR to address NR-U in inter-RAT SFTD measurements in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6828 | 1 | B | CR to address NR-U in EN-DC SFTD measurements in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200990 | 6836 |  | A | CR to RRM MPDSCH Repetitions in CE ModeA test case | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6837 | 2 | B | Introduction of addition and release of NR PSCell operating with CCA in EN-DC | 16.6.0 |
| 2020-06 | RAN#88 | RP-200975 | 6840 | 1 | B | 36.133 CR on cell identification in connected mode for | 16.6.0 |
| 2020-06 | RAN#88 | RP-201055 | 6842 | 1 | F | Finalisation of requirements in 36.133 R16 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200992 | 6844 |  | A | Editorial correction of E-UTRAN FDD | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6846 | 1 | B | Updates to general section for NR-U in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200975 | 6848 | 2 | B | Cell re-selection for EUTRAN-NR high speed in TS36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200988 | 6851 |  | A | CR on Number of carriers to monitor for IDLE mode measurements | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6854 | 1 | B | CR to TS 36.133: adding handover to NR-U | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6856 | 1 | B | Introduction of RLM requirements with enhanced MPDCCH | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6857 | 1 | B | Introduction of DL channel quality report for eMTC | 16.6.0 |
| 2020-06 | RAN#88 | RP-200990 | 6861 |  | A | CR to TS 36.133: Change of SR-ConfigIndex in eMTC RLM DRX test cases (Rel-16) | 16.6.0 |
| 2020-06 | RAN#88 | RP-200983 | 6863 |  | F | CR to 36.133 on NR reporting criteria | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6866 |  | B | CR on downlink channel quality measurement requirement for Rel-16 NB IoT | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6867 | 1 | B | CR on non-anchor RRM measurement requirements in enhanced coverage for Rel-16 NB IoT | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6868 | 2 | B | CR on non-anchor RRM measurement requirements in normal coverage for Rel-16 NB IoT | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6869 | 2 | B | CR on measurement requriements for RSRP change based TA validation | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6870 | 1 | B | CR on updating RRM requirement for new introduced UE specific DRX cycles for Rel-16 NB-IoT | 16.6.0 |
| 2020-06 | RAN#88 | RP-200968 | 6874 |  | A | CR on NB-IoT cell reselection margin in enhanced coverage in Rel-16 (Cat A) | 16.6.0 |
| 2020-06 | RAN#88 | RP-201045 | 6875 | 1 | B | CR on DL interruption on LTE carriers at Tx switching between two uplink carriers | 16.6.0 |
| 2020-06 | RAN#88 | RP-201047 | 6876 | 1 | B | CR on impact on LTE RRM measurement due to NR SRS carrier switching | 16.6.0 |
| 2020-06 | RAN#88 | RP-200963 | 6877 | 2 | F | CR on DAPS handover | 16.6.0 |
| 2020-06 | RAN#88 | RP-201047 | 6878 | 2 | B | CR on NR SRS carrier switching interruption in TS 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200983 | 6880 |  | A | Correction on gap pattern applicability in TS 36.133 R16 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200983 | 6882 |  | A | CR to remove RSTD requirements for NE-DC in 36.133 R16 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200966 | 6884 |  | B | CR on interruption requirements for direct SCell activation for 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-201047 | 6885 | 1 | B | CR on Multiple SCell activation/deactivation interruption requirements 36133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-201047 | 6887 |  | B | CR to 36.133 on CGI reading of LTE cell | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6888 |  | B | CR on for quality reporting | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6889 | 1 | B | CR to add additional timing requirements for PUR | 16.6.0 |
| 2020-06 | RAN#88 | RP-200989 | 6893 |  | A | Correction to eMTC inter-frequency reselection margin R16 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6894 | 1 | B | RSS based RSRP measurement to IDLE mode for eMTC in normal coverage | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6895 | 1 | B | RSS based RSRP measurement to IDLE mode for eMTC in enhanced coverage | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6896 | 1 | B | RSS based RSRP measurement to CONNECTED mode for eMTC | 16.6.0 |
| 2020-06 | RAN#88 | RP-200962 | 6902 | 2 | B | Introduction of requirements for preconfigured uplink resource transmission for cat-M1 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200983 | 6906 |  | A | Correction to RRC release with redirection requirements in 36.133 Rel-16 | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6907 |  | B | Interruption due to BWP switching at consistent UL failure in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200969 | 6908 | 1 | B | Applicability of 2-step RA and 4-step RA in RRM requirements in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-201047 | 6909 | 1 | B | CR on 36133 interruption requirements for BWP switching on multiple CCs | 16.6.0 |
| 2020-06 | RAN#88 | RP-200963 | 6910 |  | F | CR on 36133 LTE CHO | 16.6.0 |
| 2020-06 | RAN#88 | RP-201042 | 6911 |  | B | RRC release with redirection requirements in NR-U in 36.133 | 16.6.0 |
| 2020-06 | RAN#88 | RP-200990 | 6912 |  | A | Finalisation of requirements in 36.133 R16 | 16.6.0 |
| 2020-09 | RAN#89 | RP-201512 | 6916 |  | A | Correction to intra-frequency event triggered reporting test case in CEModeA | 16.7.0 |
| 2020-09 | RAN#89 | RP-201500 | 6917 | 1 | B | NR CGI measurements with autonomous gaps for 36.133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201497 | 6922 |  | F | 36.133 CR on cell identification in connected mode for EUTRAN-NR measurement for Rel-16 NR HST | 16.7.0 |
| 2020-09 | RAN#89 | RP-201497 | 6924 |  | F | Correction to cell re-selection for EUTRAN-NR high speed in TS36.133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201500 | 6925 |  | B | Impact of CGI reading on RLM | 16.7.0 |
| 2020-09 | RAN#89 | RP-201512 | 6931 | 1 | B | CR to TS 36.133 to address NR-U inter-RAT measurements | 16.7.0 |
| 2020-09 | RAN#89 | RP-201506 | 6932 |  | F | CR 36.133 (8.17.2.2.a) Clarification of UE behaviour | 16.7.0 |
| 2020-09 | RAN#89 | RP-201509 | 6933 |  | F | CR on serving cell measurement on non-anchor carrier for NB-IoT | 16.7.0 |
| 2020-09 | RAN#89 | RP-201509 | 6934 | 1 | F | CR on PUR requirements for NB-IoT | 16.7.0 |
| 2020-09 | RAN#89 | RP-201512 | 6936 |  | A | CR on NB-IoT Intra frequency with serving cell measurement relaxation test case 4.2.38 R16 (Cat A) | 16.7.0 |
| 2020-09 | RAN#89 | RP-201511 | 6938 |  | F | Draft CR on DAPS handover | 16.7.0 |
| 2020-09 | RAN#89 | RP-201512 | 6941 |  | A | Correction on UE measurement capability in NR idle mode R16 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201512 | 6943 |  | A | CR to measurement capability for NE-DC in 36133 R16 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201494 | 6944 | 2 | B | CR to introduce EMR in 36.133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201494 | 6945 |  | B | CR on interruption for direct activation of multiple SCells 36133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201494 | 6946 | 2 | B | CR on interruption requirements for SCell dormancy 36133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201500 | 6948 | 1 | F | CR to 36.133 for CGI reading | 16.7.0 |
| 2020-09 | RAN#89 | RP-201508 | 6949 | 1 | F | CR on RSS based measurement requriements | 16.7.0 |
| 2020-09 | RAN#89 | RP-201508 | 6950 | 1 | F | CR on RLM requriements based on enhanced MPDCCH | 16.7.0 |
| 2020-09 | RAN#89 | RP-201508 | 6951 | 1 | F | CR on PUR related requirements | 16.7.0 |
| 2020-09 | RAN#89 | RP-201508 | 6953 | 1 | B | Introduction of measurement accuracy requirements for RSS based RSRP measurements for cat-M1/M2 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201508 | 6954 | 1 | F | Correction of eMTC DL channel quality report mapping table and RSS measurement requirements | 16.7.0 |
| 2020-09 | RAN#89 | RP-201512 | 6955 | 1 | B | RRC_IDLE state inter-RAT moblity requirements for NR-U | 16.7.0 |
| 2020-09 | RAN#89 | RP-201506 | 6956 |  | F | Correction to RACH delay in HO delay requirements in NR-U | 16.7.0 |
| 2020-09 | RAN#89 | RP-201506 | 6957 |  | F | Correction to RACH delay in RRC release requirements in NR-U in 36.133 | 16.7.0 |
| 2020-09 | RAN#89 | RP-201500 | 6959 |  | B | CR to 36.133 on CGI reading of E-UTRA cell in NE-DC | 16.7.0 |
| 2020-12 | RAN#90 | RP-202433 | 6962 | 1 | B | CR for TS36.133, Adding requirements for CSI-RS based L3 measurement | 16.8.0 |
| 2020-12 | RAN#90 | RP-202442 | 6963 |  | F | CR to TS 36.133: Add information on the inter-band EN-DC configurations with no DL interruption | 16.8.0 |
| 2020-12 | RAN#90 | RP-202435 | 6964 | 1 | F | CR on IDLE state cell re-selection requirements for HST in 36.133 | 16.8.0 |
| 2020-12 | RAN#90 | RP-202512 | 6967 |  | A | CR on maintaining V2X test cases in TS36.133 R16 | 16.8.0 |
| 2020-12 | RAN#90 | RP-202475 | 6968 |  | B | Test cases for inter-frequency DAPS handover | 16.8.0 |
| 2020-12 | RAN#90 | RP-202479 | 6969 | 1 | F | Correction on the synchronous condition for DAPS handover | 16.8.0 |
| 2020-12 | RAN#90 | RP-202483 | 6970 |  | F | CR on PUR requirements for NB-IoT | 16.8.0 |
| 2020-12 | RAN#90 | RP-202483 | 6971 | 1 | F | CR on RRM requirements for short DRX with eDRX configured for Rel-16 NB-IoT | 16.8.0 |
| 2020-12 | RAN#90 | RP-202444 | 6972 | 1 | F | CR to 36.133: Correction to NR CGI reading interruption requirements | 16.8.0 |
| 2020-12 | RAN#90 | RP-202444 | 6973 | 1 | B | CR to 36.133: Introduce requirements for mandatory gap pattern | 16.8.0 |
| 2020-12 | RAN#90 | RP-202430 | 6976 | 2 | B | CR on EMR requirements in 36.133 | 16.8.0 |
| 2020-12 | RAN#90 | RP-202444 | 6978 | 1 | F | CR on CGI reading requirements 36.133 | 16.8.0 |
| 2020-12 | RAN#90 | RP-202478 | 6979 | 1 | F | CR on RSS measurement requirements | 16.8.0 |
| 2020-12 | RAN#90 | RP-202478 | 6980 | 1 | B | CR to introduce measurement requirements for eMTC in RRC_Inactive | 16.8.0 |
| 2020-12 | RAN#90 | RP-202494 | 6984 | 1 | F | CR on performance requirements tests for euCA. | 16.8.0 |
| 2020-12 | RAN#90 | RP-202491 | 6987 |  | A | CR 36.133 Correction to test cases for SCell Hibernation (Rel-16) | 16.8.0 |
| 2020-12 | RAN#90 | RP-202430 | 6988 | 1 | F | CR 36.133 Removal of brackets for NR SCell Dormancy | 16.8.0 |
| 2020-12 | RAN#90 | RP-202486 | 6990 |  | A | CR 36.133 Removal of brackets for SFTD measurements (Rel-16) | 16.8.0 |
| 2020-12 | RAN#90 | RP-202444 | 6996 | 1 | F | Maintenance CR on NR CGI reading in 36.133 | 16.8.0 |
| 2020-12 | RAN#90 | RP-202479 | 6997 | 1 | F | Correction on LTE conditional handover | 16.8.0 |
| 2020-12 | RAN#90 | RP-202414 | 6999 | 1 | F | Terminology updates for NR-U | 16.8.0 |
| 2020-12 | RAN#90 | RP-202444 | 7000 |  | F | Missing requirements for LTE SRS carrier-based switching | 16.8.0 |
| 2020-12 | RAN#90 | RP-202513 | 7005 |  | A | Correction to test parameters for FDD and TDD intra-frequency RSRP for Cat-M1 UE in CEModeA | 16.8.0 |
| 2020-12 | RAN#90 | RP-202475 | 7008 |  | B | Introduction of intra-frequency sync and async LTE DAPS HO test cases | 16.8.0 |
| 2020-12 | RAN#90 | RP-202478 | 7009 | 1 | F | Correction to RSS based measurement requirements | 16.8.0 |
| 2020-12 | RAN#90 | RP-202473 | 7010 |  | B | Big CR: Introduction of Rel-16 eMTC RRM performance requirements (TS 36.133) | 16.8.0 |
| 2020-12 | RAN#90 | RP-202474 | 7011 |  | B | Big CR: Introduction of Rel-16 Nb-IoT RRM RRM performance requirements (TS 36.133) | 16.8.0 |
| 2020-12 | RAN#90 | RP-202475 | 7012 |  | B | Test cases for LTE conditional handover | 16.8.0 |
| 2020-12 | RAN#90 | RP-202449 | 6994 | 1 | B | Big CR on FR2 new FWA UE RRM requirements in TS 36.133 | 17.0.0 |
| 2021-03 | RAN#91 | RP-210110 | 7016 | 1 | B | CR of adding LTE B24 for UE category NB1/NB2 in R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210120 | 7022 |  | A | CR: Correction of eMTC RLM test cases | 17.1.0 |
| 2021-03 | RAN#91 | RP-210106 | 7024 |  | A | Correction Rel-16 eMTC RRM performance requirements | 17.1.0 |
| 2021-03 | RAN#91 | RP-210071 | 7026 |  | A | CR on interruption requirements of BWP switch on multiple CCs for EN-DC | 17.1.0 |
| 2021-03 | RAN#91 | RP-210108 | 7028 |  | A | CR on maintenance for measurement on non-anchor carrier for NB-IoT | 17.1.0 |
| 2021-03 | RAN#91 | RP-210107 | 7030 |  | A | Clarification on asynchronous DAPS handover | 17.1.0 |
| 2021-03 | RAN#91 | RP-210104 | 7032 |  | A | Test cases for inter-frequency DAPS handover | 17.1.0 |
| 2021-03 | RAN#91 | RP-210169 | 7035 | 1 | F | Inclusive language review in TS 36.133 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210114 | 7037 |  | A | Clarification for performance requirements tests for euCA | 17.1.0 |
| 2021-03 | RAN#91 | RP-210115 | 7040 |  | A | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | 17.1.0 |
| 2021-03 | RAN#91 | RP-210087 | 7042 |  | A | CR 36.133 Removal of brackets for SCell dormancy | 17.1.0 |
| 2021-03 | RAN#91 | RP-210104 | 7044 |  | A | CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs - Cat-A | 17.1.0 |
| 2021-03 | RAN#91 | RP-210084 | 7046 |  | A | Terminology updates for NR-U in 36.133 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210104 | 7048 |  | A | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | 17.1.0 |
| 2021-03 | RAN#91 | RP-210084 | 7050 |  | A | Applicability of RA with CCA on RRM requirements in NR-U in 36.133 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210121 | 7053 |  | A | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | 17.1.0 |
| 2021-03 | RAN#91 | RP-210119 | 7057 |  | A | Correction to requirements for NCSG patterns | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 7060 |  | A | CR to remove intra-frequency ECID requirements for NE-DC 36133 R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 7063 |  | A | CR to idle more requirements in 36133 R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210087 | 7065 |  | A | CR on EMR requirements in 36.133 R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210081 | 7069 |  | A | CR to introduce new measurement gap patterns for positioning in 36.133 R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210071 | 7071 |  | A | CR on CGI reading report delay 36.133 R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210105 | 7073 |  | A | CR on eMTC RRM requirements R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210113 | 7076 |  | A | CR on TC for eMTC RSTD measurement R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210112 | 7079 |  | A | CR on CRS muting for eMTC R17 | 17.1.0 |
| 2021-06 | RAN#92 | RP-211093 | 7083 |  | A | Time synchronization assumption for RSS-based neighbor cell measurements | 17.2.0 |
| 2021-06 | RAN#92 | RP-211095 | 7085 |  | A | CR on core requirements maintenance of IDLE mode inter-RAT measurement for NR-U R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211095 | 7087 |  | A | CR on PSCell Addition requirements for NR-U R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211098 | 7091 |  | A | CR on maintenance of BWP Switch on multiple CCs 36133 R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211107 | 7100 |  | A | CR on requirements of cell reselection for NB-IoT R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211088 | 7107 |  | A | Correction to interruption to LTE serving cells for measurements on deactivated NR SCell_R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211093 | 7109 |  | A | Correction of RLM test parameters for MPDCCH performance improvement | 17.2.0 |
| 2021-06 | RAN#92 | RP-211089 | 7111 |  | A | CR on applicability of requirements for NE-DC operation and SMTC determination 36133 R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211119 | 7115 |  | A | CR on LTE-NR EMR requirements 36133 R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211119 | 7117 |  | A | Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211109 | 7119 |  | A | CR on MG for PRS measurement 36.133 R17 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211095 | 7126 |  | A | Terminology updates for NR-U in 36.133 | 17.2.0 |
| 2021-06 | RAN#92 | RP-211095 | 7128 |  | A | Big CR: Introduction of Rel-16 NR-U RRM performance | 17.2.0 |
| 2021-09 | RAN#93 | RP-211919 | 7129 |  | B | CR on adding B24 for Cat-M1_M2 36.133 | 17.3.0 |
| 2021-09 | RAN#93 | RP-211923 | 7134 |  | A | Big CR to TS 36.133: LTE RRM maintenance (Rel-17) | 17.3.0 |
| 2021-12 | RAN#94 | RP-212847 | 7136 |  | A | Big CR to TS 36.133: LTE RRM maintenance (Rel-17) | 17.4.0 |
| 2022-03 | RAN#95 | RP-220351 | 7137 | 1 | B | Big CR of RRM requirements for Rel-17 NB-IoT and eMTC | 17.5.0 |
| 2022-03 | RAN#95 | RP-220347 | 7141 | 1 | B | CR to TS 36.133: implementation of LTE_upper_700MHz_A band 103 | 17.5.0 |
| 2022-03 | RAN#95 | RP-220368 | 7142 |  | B | Big CR: RRM requirements for Rel-17 NR FeRRM (TS 36.133) | 17.5.0 |
| 2022-03 | RAN#95 | RP-220346 | 7143 |  | B | Big CR: RRM requirements for Rel-17 Further Multi-RAT Dual-Connectivity enhancements (TS 36.133) | 17.5.0 |
| 2022-03 | RAN#95 | RP-220336 | 7147 |  | A | Big CR to TS 36.133: LTE RRM maintenance (Rel-17) | 17.5.0 |
| 2022-06 | RAN#96 | RP-221678 | 7148 | 1 | F | Correction of Configuration Parameters for Test 1 in Test Case A.7.1.11 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221681 | 7149 | 1 | F | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221682 | 7150 | 1 | F | Maintenance CR for SCG activation delay on 36.133 R17 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221681 | 7153 | 1 | F | Correction on HO with PSCell requirements in 36133 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221681 | 7154 | 1 | F | CR on SRS antenna port switching requirements 36.133 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221682 | 7155 | 1 | F | Draft CR on Interruptions at A-TRS based SCell activation/deactivation (36.133) | 17.6.0 |
| 2022-06 | RAN#96 | RP-221678 | 7156 | 1 | F | CR on maintenance for NB-IoT R17 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221676 | 7157 |  | B | Big CR for RedCap for TS 36.133 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221681 | 7158 | 1 | F | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | 17.6.0 |
| 2022-06 | RAN#96 | RP-221669 | 7160 |  | B | CR on adding B48 for M1/M2/NB1/NB2 | 17.6.0 |
| 2022-06 | RAN#96 | RP-221653 | 7162 |  | A | Big CR on TS 36.133 Maintenance (Rel-17) | 17.6.0 |
| 2022-09 | RAN#97-e | RP-222050 | 7169 |  | F | TS36.133 CR on SCG activation/deactivation | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222050 | 7170 |  | B | Interruptions due to RRM measurements on deactivated SCG | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222032 | 7171 |  | F | draftCR on inter-RAT NR measurement for RedCap | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222032 | 7172 | 1 | B | Big CR for RedCap for TS 36.133 | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222029 | 7173 |  | F | CR on maintenance of neighbour cell measurement for NB-IoT | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222032 | 7174 | 1 | F | Changes to RRC_IDLE mode requirements for RedCap for TS 38.133 | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222029 | 7175 |  | B | Big CR for NB_IOTenh4_LTE_eMTC6_RRM (Rel-17) | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222034 | 7176 | 1 | F | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | 17.7.0 |
| 2022-09 | RAN#97-e | RP-222026 | 7180 |  | F | Big CR for 36.133 maintenance (Rel-17) | 17.7.0 |
| 2022-12 | RAN#98-e | RP-223308 | 7182 | 1 | F | Maintenance CR on HO with PSCell where PSCell is on CCA | 17.8.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-12 | RAN#98-e | RP-223315 | 7181 |  | B | CR to TS 36.133: Introduction of LTE TDD band 54 | 18.0.0 |
| 2022-12 | RAN#98-e | RP-223314 | 7183 |  | B | Big CR for IoT NTN core requirements for TS36.133 | 18.0.0 |
| 2023-03 | RAN#99 | RP-230524 | 7184 |  | B | CR related to Introduction of support of NB1/NB2/M1 for LTE TDD Band 54 | 18.1.0 |
| 2023-03 | RAN#99 | RP-230512 | 7186 |  | A | Updates Band grouping for NB-IoT operation in Band 24 to include n24 | 18.1.0 |
| 2023-03 | RAN#99 | RP-230507 | 7190 |  | A | CR on SRS antenna port switching requirements 36.133 R18 | 18.1.0 |
| 2023-03 | RAN#99 | RP-230518 | 7193 |  | A | CR on RedCap Idle mode(36.133) | 18.1.0 |
| 2023-03 | RAN#99 | RP-230530 | 7196 |  | A | Inter-RAT NR cell reselection requiremetns under eDRX for NR-U and NR | 18.1.0 |
| 2023-03 | RAN#99 | RP-230500 | 7199 |  | A | Editorial corrections in TS 36.133 | 18.1.0 |