| 3GPP TS 36.133 V18.2.0 (2023-06) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.5.1A	Groups of bands for satellite access	105

3.6	General	106

3.6.1	Applicability of requirements in this specification version	106

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	112

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	114

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	115

3.6.2	Applicability of requirements for EN-DC operation	116

3.6.3	Applicability of requirements for NE-DC operation	117

3.6.4	Applicability of requirements for NGEN-DC operation	118

4	E-UTRAN RRC_IDLE state mobility	118

4.1	Cell Selection	118

4.2	Cell Re-selection	118

4.2.1	Introduction	118

4.2.2	Requirements	118

4.2.2.1	Measurement and evaluation of serving cell	120

4.2.2.2	Void	121

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	121

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	122

4.2.2.5	Measurements of inter-RAT cells	125

4.2.2.5.1	Measurements of UTRAN FDD cells	125

4.2.2.5.2	Measurements of UTRAN TDD cells	126

4.2.2.5.3	Measurements of GSM cells	128

4.2.2.5.4	Measurements of HRPD cells	129

4.2.2.5.5	Measurements of cdma2000 1X	130

4.2.2.5.6	Measurements of NR cells	131

4.2.2.5.7	Measurements of NR cells subject to CCA	134

4.2.2.5.8	Measurements of NR cells for RedCap	135

4.2.2.6	Evaluation of cell re-selection criteria	137

4.2.2.7	Maximum interruption in paging reception	138

4.2.2.8	void	139

4.2.2.9	UE measurement capability	139

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	139

4.2.2.10	Reselection to CSG cells	140

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	140

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	140

4.2.2.11	Void	141

4.2.2.12	Void	141

4.2.2.13	Void	141

4.2.2.14	UE measurement capability for RedCap	141

4.3	Minimization of Drive Tests (MDT)	142

4.3.1	Introduction	142

4.3.2	Measurements	142

4.3.2.1	Requirements	142

4.3.3	Relative Time Stamp Accuracy	142

4.3.3.1	Requirements	142

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	143

4.3.4.1	 Requirements	143

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	143

4.3.5.1	Requirements for timeSinceFailure	143

4.4	MBSFN Measurements	143

4.4.1	Introduction	143

4.4.2	MBSFN RSRP measurements	143

4.4.3	MBSFN RSRQ measurements	144

4.4.4	MCH BLER measurements	144

4.5	Proximity-based Services	144

4.5.1	Introduction	144

4.5.2	Requirements	144

4.5.2.1	Interruptions with ProSe Direct Discovery	144

4.5.2.2	Interruptions with ProSe Direct Communication	144

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	145

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	145

4.6	Cell Selection and Re-selection Requirements for UE category NB1	146

4.6.1	Cell Selection	146

4.6.2	Cell Re-selection	146

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	146

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	147

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	148

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	149

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	151

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	152

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	153

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	154

4.6.2.7	Maximum interruption in paging reception in normal coverage	156

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	156

4.6.2.8	UE measurement capability	156

4.6.2.9	WUS receptions for NB1	156

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	157

4.6.3.1	Introduction	157

4.6.3.2	Requirements on UE synchronization for transmission using PUR	157

4.6.3.3	Requirements on TA validation for transmission using PUR	157

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	158

4.6A.1	Cell Selection	158

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	158

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	158

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	160

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	161

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	163

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	165

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	165

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	168

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	169

4.6A.2.7	Maximum interruption in paging reception in normal coverage	171

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	172

4.6A.2.8	UE measurement capability	172

4.6A.2.9	WUS receptions for NB1	172

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	173

4.6A.3.1	Introduction	173

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	173

4.6A.3.3	Requirements on TA validation for transmission using PUR	173

4.7	Cell Selection and Re-selection Requirements for UE category M1	173

4.7.1	Cell Selection	173

4.7.2	Cell Re-selection	173

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	174

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	174

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	174

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	175

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	177

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	179

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	179

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	180

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	180

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	181

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	182

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	185

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	186

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	187

4.7.2.3	WUS receptions for UE category M1	187

4.7.3	Channel quality report for UE Category M1 in idle mode	187

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	188

4.7.4.1	Introduction	188

4.7.4.2	Requirements on UE synchronization for transmission using PUR	188

4.7.4.3	Requirements on TA validation for transmission using PUR	188

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	189

4.7A.1	Cell Selection	189

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	189

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	190

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	190

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	192

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	193

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	196

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	197

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	198

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	198

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	198

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	200

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	202

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	204

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	206

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	207

4.7A.2.3	WUS receptions for UE category M1	207

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	207

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	208

4.7A.4.1	Introduction	208

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	208

4.7A.4.3	Requirements on TA validation for transmission using PUR	208

4.8	Idle State Positioning Measurement Requirements for UE category NB1	209

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	209

4.8.1.1	RSTD Measurement Reporting Delay	210

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	211

4.8.2.1	RSTD Measurement Reporting Delay	212

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	213

4.8.3.1	RSTD Measurement Reporting Delay	214

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	215

4.8.4.1	RSTD Measurement Reporting Delay	216

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	217

4.8.5.1	Measurement Reporting Delay	218

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	218

4.8.6.1	Measurement Reporting Delay	219

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	220

4.8.7.1	Measurement Reporting Delay	221

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	221

4.8.8.1	Measurement Reporting Delay	222

4.9	Idle Mode CA Measurement	223

4.9.1	Introduction	223

4.9.2	Requirement	223

4.9.2.1	Detected cell requirement during state transition and Idle mode	223

4.9.2.2	Measurements of inter-frequency CA candidate cells	224

4.9.2.3	Measurements on serving cell	224

4A	E-UTRAN RRC_INACTIVE state mobility	225

4A.1	Cell Re-selection	225

4A.1.1	Introduction	226

4A.1.2	Requirements	226

4A.1.2.1	UE measurement capability	226

4A.1.2.2	Measurement and evaluation of serving cell	226

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	226

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	226

4A.1.2.5	Evaluation of cell re-selection criteria	226

4A.1.2.6	Maximum interruption in paging reception	226

4A.1.2.7	Measurements of inter-RAT NR cells	226

4A.1.2.8	UE measurement capability for RedCap	226

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	226

4A.2	Requirements for UE Category M1	227

4A.2.1	Introduction	227

4A.2.2	Cell Selection	227

4A.2.3	Cell Reselection	227

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	227

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	227

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	228

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	228

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	228

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	228

4A.2.4	Channel quality report for UE Category M1 in idle mode	230

5	E-UTRAN RRC_CONNECTED state mobility	230

5.1	E-UTRAN Handover	231

5.1.1	Introduction	231

5.1.2	Requirements	231

5.1.2.1	E-UTRAN FDD – FDD	231

5.1.2.1.1	Handover delay	231

5.1.2.1.2	Interruption time	231

5.1.2.2	E-UTRAN FDD – TDD	232

5.1.2.2.1	(Void)	233

5.1.2.2.2	(Void)	233

5.1.2.3	E-UTRAN TDD – FDD	233

5.1.2.3.1	(Void)	233

5.1.2.3.2	(Void)	233

5.1.2.4	E-UTRAN TDD – TDD	233

5.1.2.4.1	Handover delay	233

5.1.2.4.2	Interruption time	233

5.1.2.5	E-UTRAN HD–FDD	235

5.1.2.5.1	Handover delay	235

5.1.2.5.2	Interruption time	235

5.1.2.6	E-UTRAN FDD – FDD conditional handover	236

5.1.2.6.1	Handover delay	236

5.1.2.6.2	Measurement time	237

5.1.2.6.3	Preparation time	237

5.1.2.6.4	Interruption time	237

5.1.2.7	E-UTRAN FDD – TDD conditional handover	237

5.1.2.8	E-UTRAN TDD – FDD conditional handover	238

5.1.2.9	E-UTRAN TDD – TDD conditional handover	238

5.2	Void	238

5.3	Handover to other RATs	238

5.3.1	E-UTRAN - UTRAN FDD Handover	238

5.3.1.1	Introduction	238

5.3.1.1.1	Handover delay	238

5.3.1.1.2	Interruption time	238

5.3.2	E-UTRAN - UTRAN TDD Handover	239

5.3.2.1	Introduction	239

5.3.2.2	Requirements	239

5.3.2.2.1	Handover delay	239

5.3.2.2.2	Interruption time	239

5.3.3	E-UTRAN - GSM Handover	240

5.3.3.1	Introduction	240

5.3.3.2	Requirements	240

5.3.3.2.1	Handover delay	240

5.3.3.2.2	Interruption time	240

5.3.4	E-UTRAN - NR FR1 Handover	240

5.3.4.1	Introduction	240

5.3.4.2	Handover delay	241

5.3.4.3	Interruption time	241

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	241

5.3.4A.1	Introduction	241

5.3.4A.2	Handover delay	242

5.3.4A.3	Interruption time	242

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	243

5.3.4B.1	Introduction	243

5.3.4B.2	Requirements	243

5.3.5	E-UTRAN - NR FR2 Handover	243

5.3.5.1	Introduction	243

5.3.5.2	Handover delay	243

5.3.5.3	Interruption time	243

5.4	Handover to Non-3GPP RATs	244

5.4.1	E-UTRAN – HRPD Handover	244

5.4.1.1	Introduction	244

5.4.1.1.1	Handover delay	244

5.4.1.1.2	Interruption time	244

5.4.2	E-UTRAN – cdma2000 1X Handover	245

5.4.2.1	Introduction	245

5.4.2.1.1	Handover delay	245

5.4.2.1.2	Interruption time	245

5.5	E-UTRAN Handover for Cat-M1 UEs	245

5.5.1	Introduction	245

5.5.2	Requirements in CEModeA	246

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	246

5.5.2.1.1	Handover delay	246

5.5.2.1.2	Interruption time	246

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	246

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	247

5.5.2.3.1	Void	247

5.5.2.3.2	Void	247

5.5.3	Requirements in CEModeB	247

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	247

5.5.3.1.1	Handover delay	247

5.5.3.1.2	Interruption time	247

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	248

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	248

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	248

5.5A.1	Introduction	248

5.5A.2	Requirements in CEModeA	248

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	248

5.5A.2.1.1	Handover delay	248

5.5A.2.1.2	Interruption time	248

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	249

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	249

5.5A.2.3.1	Handover delay	249

5.5A.2.3.2	Measurement time	249

5.5A.2.3.3	Preparation time	250

5.5A.2.3.4	Interruption time	250

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	250

5.5A.3	Requirements in CEModeB	251

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	251

5.5A.3.1.1	Handover delay	251

5.5A.3.1.2	Interruption time	251

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	252

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	252

5.5A.3.3.1	Handover delay	252

5.5A.3.3.2	Measurement time	252

5.5A.3.3.3	Preparation time	252

5.5A.3.3.4	Interruption time	253

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	253

5.6	Void	253

5.7	E-UTRAN DAPS Handover	253

5.7.1	Introduction	253

5.7.2	Requirements	254

5.7.2.1	E-UTRAN FDD – FDD	254

5.7.2.1.1	DAPS Handover delay	254

5.7.2.1.2	Interruption time	255

5.7.2.2	E-UTRAN FDD – TDD	255

5.7.2.3	E-UTRAN TDD – FDD	255

5.7.2.4	E-UTRAN TDD – TDD	255

5.8	EN-DC Handover with PSCell	255

5.8.1	Introduction	255

5.8.1.1	Handover with PSCell Interruption time	256

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	256

5.9	EN-DC Handover with PSCell using CCA	257

5.9.1	Introduction	257

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	257

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	258

6	RRC Connection Mobility Control	258

6.1	RRC Re-establishment	258

6.1.1	Introduction	259

6.1.2	Requirements	259

6.1.2.1	UE Re-establishment delay requirement	259

6.2	Random Access	259

6.2.1	Introduction	259

6.2.2	Requirements	259

6.2.2.1	Contention based random access	260

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	260

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	260

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	260

6.2.2.1.4	Void	260

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	260

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	260

6.2.2.2	Non-Contention based random access	260

6.2.2.2.1	Correct behaviour when receiving Random Access Response	260

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	260

6.2.3	Requirements for Cat-M1 UEs	261

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	261

6.3	RRC Connection Release with Redirection	261

6.3.1	Introduction	261

6.3.2	Requirements	261

6.3.2.1	RRC connection release with redirection to UTRAN FDD	261

6.3.2.2	RRC connection release with redirection to GERAN	262

6.3.2.3	RRC connection release with redirection to UTRAN TDD	262

6.3.2.4	RRC connection release with redirection to NR	263

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	263

6.3.2.6	RRC connection release with redirection to NR Redcap	264

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	266

6.4.1	Introduction	266

6.4.2	Requirements	266

6.5	RRC Re-establishment for NB-IoT UEs	266

6.5.1	Introduction	266

6.5.2	Requirements	266

6.5.2.1	UE Re-establishment delay requirement in normal coverage	266

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	267

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	267

6.5A.1	Introduction	267

6.5A.2	Requirements	268

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	268

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	268

6.6	Random Access for UE category NB1	269

6.6.1	Introduction	269

6.6.2	Requirements	269

6.6.2.1	Correct behaviour when receiving Random Access Response reception	269

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	269

6.6.2.3	Correct behaviour when receiving a NACK on msg3	270

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	270

6.6.2.5	Correct behaviour when contention Resolution timer expires	270

6.6.2.6	MSG3-based channel quality report for UE Category NB1	270

6.6.3	Requirements for NPRACH configuration	270

6.6A	Random Access for UE category NB-IoT for Satellite Access	271

6.6A.1	Introduction	271

6.6A.2	Requirements	271

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	271

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	271

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	271

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	271

6.6A.2.5	Correct behaviour when contention Resolution timer expires	272

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	272

6.6A.3	Requirements for NPRACH configuration	272

6.7	RRC Re-establishment for Cat-M1 UEs	273

6.7.1	Introduction	273

6.7.2	Requirements	273

6.7.2.1	UE Re-establishment delay requirement for CEModeA	273

6.7.2.2	UE Re-establishment delay requirement for CEModeB	273

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	274

6.7A.1	Introduction	274

6.7A.2	Requirements	274

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	274

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	275

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	276

6.8.1	Introduction	276

6.8.2	Requirements	276

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	276

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	277

6.8A.1	Introduction	277

6.8A.2	Requirements	277

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	277

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	278

6.9.1	Introduction	278

6.9.2	Requirements	278

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	278

6.9A.1	Introduction	278

7	Timing and signalling characteristics	279

7.1	UE transmit timing	279

7.1.1	Introduction	279

7.1.2	Requirements	280

7.2	UE timer accuracy	281

7.2.1	Introduction	281

7.2.2	Requirements	281

7.3	Timing Advance	281

7.3.1	Introduction	281

7.3.2	Requirements	282

7.3.2.1	Timing Advance adjustment delay	282

7.3.2.2	Timing Advance adjustment accuracy	282

7.4	Cell phase synchronization accuracy (TDD)	282

7.4.1	Definition	282

7.4.2	Minimum requirements	282

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	283

7.5.1	Introduction	283

7.5.2	eNodeB Synchronization Requirements	283

7.5.2.1	Synchronized E-UTRAN	283

7.5.2.2	Non-Synchronized E-UTRAN	283

7.6	Radio Link Monitoring	283

7.6.1	Introduction	283

7.6.2	Requirements	285

7.6.2.1	Minimum requirement when no DRX is used	285

7.6.2.2	Minimum requirement when DRX is used	286

7.6.2.3	Minimum requirement at transitions	287

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	287

7.6.2.5	Minimum requirement under IDC Interference	288

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	288

7.7.1	Introduction	288

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	288

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	290

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	291

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	292

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	293

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	294

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	294

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	294

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	295

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	296

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	297

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	298

7.7.14	SCell Activation Delay Requirement for Dormant SCell	298

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	299

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	300

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	302

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	302

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	304

7.8	Interruptions with Carrier Aggregation	306

7.8.1	Introduction	306

7.8.2	Requirements	306

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	306

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	306

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	306

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	306

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	307

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	307

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	307

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	307

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	308

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	309

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	309

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	309

7.8.2.13	Interruptions at SRS carrier based switching	310

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	310

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	311

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	311

7.8.2.17	Interruptions during CQI measurement on dormant SCell	311

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	311

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	312

7.8.2.20	Interruptions at SCell hibernation	312

7.8.2.21	Interruptions at direct SCell activation and hibernation	312

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	313

7.9.1	Introduction	313

7.9.2	Minimum Requirements for Interband Carrier Aggregation	313

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	313

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	314

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	314

7.10.1	Introduction	314

7.10.2	Requirements	314

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	314

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	314

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	315

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	315

7.11	Radio Link Monitoring for UE Category 0	315

7.11.1	Introduction	315

7.11.2	Requirements for FD-FDD and TDD	317

7.11.2.1	Minimum requirement when no DRX is used	317

7.11.2.2	Minimum requirement when DRX is used	317

7.11.2.3	Minimum requirement at transitions	318

7.11.3	Requirements for HD-FDD	318

7.11.3.1	Minimum requirement when no DRX is used	318

7.11.3.2	Minimum requirement when DRX is used	318

7.11.3.3	Minimum requirement at transitions	319

7.12	Interruptions with Dual Connectivity	319

7.12.1	Introduction	319

7.12.2	Requirements	319

7.12.2.1	Interruptions at PSCell addition/release	319

7.12.2.2	Interruptions at transitions between active and non-active during DRX	320

7.12.2.3	Interruptions at transitions from non-DRX to DRX	320

7.12.2.4	Interruptions at SCell addition/release	320

7.12.2.5	Interruptions at SCell activation/deactivation	320

7.12.2.6	Interruptions during measurements on SCC	321

7.12.2.7	Interruptions at SRS carrier based switching	321

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	322

7.13.1	Definition	322

7.13.2	Minimum requirements	322

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	322

7.14.1	Introduction	322

7.14.2	PSCell Addition Delay Requirement	322

7.14.3	PSCell Release Delay Requirement	323

7.15	Maximum Receive Timing Difference in Dual Connectivity	323

7.15.1	Introduction	323

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	323

7.16	Proximity-based Services	324

7.16.1	Introduction	324

7.16.2	Requirements	324

7.16.2.1	ProSe UE transmission timing	324

7.16.2.1.1	Serving cell or PCell as timing reference	324

7.16.2.1.2	SCell or non-serving cell as timing reference	324

7.16.3	Interruptions with ProSe	324

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	325

7.16.3.2	Interruptions at ProSe Direct Communication configuration	325

7.16.3.3	Interruptions during ProSe Direct Discovery	325

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	325

7.16.3.5	Interruptions during ProSe Direct Communication	326

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	326

7.16.4.1	Measurement and evaluation of selected cell	326

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	326

7.16.5	Selection / Reselection of ProSe relay UE	327

7.16.6	ProSe operation under deactivated SCell	327

7.17	Maximum Transmission Timing Difference in Dual Connectivity	328

7.17.1	Introduction	328

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	328

7.18.1	Introduction	328

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	328

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	328

7.19	Radio Link Monitoring for UE Category M1	328

7.19.1	Introduction	328

7.19.2	Requirements for FD-FDD and TDD CE mode A	329

7.19.2.1	Minimum requirement when no DRX is used	331

7.19.2.2	Minimum requirement when DRX is used	332

7.19.2.3	Minimum requirement at transitions	333

7.19.3	Requirements for HD-FDD with CE mode A	333

7.19.3.1	Minimum requirement when no DRX is used	333

7.19.3.2	Minimum requirement when DRX is used	333

7.19.3.3	Minimum requirement at transitions	335

7.19.4	Requirements for FD-FDD and TDD with CE mode B	335

7.19.4.1	Minimum requirement when no DRX is used	337

7.19.4.2	Minimum requirement when DRX is used	338

7.19.4.3	Minimum requirement at transitions	339

7.19.5	Requirements for HD-FDD with CE mode B	339

7.19.5.1	Minimum requirement when no DRX is used	339

7.19.5.2	Minimum requirement when DRX is used	339

7.19.5.3	Minimum requirement at transitions	341

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	341

7.19A.1	Introduction	341

7.19A.2	Requirements for FD-FDD and CE mode A	341

7.19A.2.1	Minimum requirement when no DRX is used	343

7.19A.2.2	Minimum requirement when DRX is used	344

7.19A.2.3	Minimum requirement at transitions	345

7.19A.3	Requirements for HD-FDD with CE mode A	345

7.19A.3.1	Minimum requirement when no DRX is used	345

7.19A.3.2	Minimum requirement when DRX is used	345

7.19A.3.3	Minimum requirement at transitions	347

7.19A.4	Requirements for HD-FDD with CE mode B	347

7.19A.4.1	Minimum requirement when no DRX is used	347

7.19A.4.2	Minimum requirement when DRX is used	347

7.19A.4.3	Minimum requirement at transitions	348

7.20	UE transmit timing for NB-IoT	348

7.20.1	Introduction	348

7.20.2	Requirements	349

7.20A	UE transmit timing for NB-IoT for Satellite Access	349

7.20A.1	Introduction	349

7.20A.2	Requirements	349

7.21	 UE timer accuracy for NB-IoT	350

7.21.1	Introduction	350

7.21.2	Requirements	350

7.21A	UE timer accuracy for NB-IoT for Satellite Access	351

7.21A.1	Introduction	351

7.21A.2	Requirements	351

7.22	Timing Advance for NB-IoT	351

7.22.1	Introduction	351

7.22.2	Requirements	351

7.22.2.1	Timing Advance adjustment delay	351

7.22.2.2	Timing Advance adjustment accuracy	351

7.22A	Timing Advance for NB-IoT for Satellite Access	352

7.22A.1	Introduction	352

7.22A.2	Requirements	352

7.22A.2.1	Timing Advance adjustment delay	352

7.22A.2.2	Timing Advance adjustment accuracy	352

7.23	Radio Link Monitoring for Category NB1 UE	352

7.23.1	Introduction	352

7.23.2	Requirements for Category NB1 UE	352

7.23.2.1	Minimum requirement when no DRX is used	353

7.23.2.2	Minimum requirement when DRX is used	353

7.23.2.3	Minimum requirement at transitions	354

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	354

7.23A.1	Introduction	354

7.23A.2	Requirements for Category NB1 UE	354

7.23A.2.1	Minimum requirement when no DRX is used	355

7.23A.2.2	Minimum requirement when DRX is used	355

7.23A.2.3	Minimum requirement at transitions	356

7.24	UE transmit timing for Category M1	356

7.24.1	Introduction	356

7.24.2	Requirements	356

7.24A	UE transmit timing for Category M1 for Satellite Access	357

7.24A.1	Introduction	357

7.24A.2	Requirements	357

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	358

7.25.1	Definition	358

7.25.2	Minimum requirements	358

7.26	UE transmit timing for Category M2	359

7.26.1	Introduction	359

7.26.2	Requirements	359

7.27	UE timer accuracy for category M1	359

7.27.1	Introduction	359

7.27.2	Requirements	359

7.27A	UE timer accuracy for category M1 for Satellite Access	359

7.27A.1	Introduction	359

7.27A.2	Requirements	359

7.28	Timing Advance for Category M1	360

7.28.1	Introduction	360

7.28.2	Requirements	360

7.28A	Timing Advance for Category M1 for Satellite Access	360

7.28A.1	Introduction	360

7.28A.2	Requirements	360

7.28A.2.1	Timing Advance adjustment delay	360

7.28A.2.2	Timing Advance adjustment accuracy	360

7.29	Interruptions requirements with FeMBMS	360

7.29.1	Introduction	360

7.29.2	Requirements	360

7.30	Numerology switching delay requirements with FeMBMS	361

7.30.1	Introduction	361

7.30.2	Requirements	361

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	361

7.31.1	Introduction	361

7.31.2	NR PSCell Addition Delay Requirement	361

7.31.3	NR PSCell Release Delay Requirement	362

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	362

7.31A.1	Introduction	362

7.31A.2	NR PSCell Addition Delay Requirement	362

7.31A.3	NR PSCell Release Delay Requirement	363

7.32	Interruptions with EN-DC	364

7.32.1	Introduction	364

7.32.2	Requirements	364

7.32.2.1	Interruptions at PSCell addition/release	364

7.32.2.2	Interruptions at transitions between active and non-active during DRX	364

7.32.2.3	Interruptions at transitions from non-DRX to DRX	365

7.32.2.4	Interruptions at SCell addition/release	365

7.32.2.5	Interruptions at SCell activation/deactivation	365

7.32.2.6	Interruptions during measurements on SCC	366

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	366

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	366

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	366

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	366

7.32.2.7	Interruptions at active BWP switching	366

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	367

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	367

7.32.2.10	Interruptions at SCell hibernation	367

7.32.2.11	Interruptions at direct SCell activation and hibernation	367

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	368

7.32.2.13	Interruptions at NR SRS carrier based switching	368

7.32.2.14	Interruptions at NR SCell dormancy	369

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	369

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	369

7.32.2.15	Interruption during NR measurement with autonomous gaps	369

7.32.2.16	Interruptions at SRS carrier based switching	370

7.32.2.17	Interruptions at SCG activation/deactivation	370

7.32.2.18	Interruptions due to NR SRS antenna port switching	370

7.32.2.19	Interruptions at fast SCell activation/deactivation	371

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	371

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	372

7.33.1	Introduction	372

7.33.2	Requirements	372

7.34	Void	372

7.35	Interruptions with SFTD measurements	372

7.35.1	Introduction	372

7.35.2	Requirements	372

7.36	Interruptions with NE-DC	373

7.36.1	Introduction	373

7.36.2	Requirements	373

7.36.2.1	Interruptions at transitions between active and non-active during DRX	373

7.36.2.2	Interruptions at transitions from non-DRX to DRX	373

7.36.2.3	Interruptions at SCell addition/release	373

7.36.2.4	Interruptions at SCell activation/deactivation	374

7.36.2.5	Interruptions during measurements on SCC	374

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	374

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	374

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	374

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	375

7.36.2.6	Interruptions at active BWP switching	375

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	376

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	376

7.36.2.9	Interruptions at SCell hibernation	376

7.36.2.10	Interruptions at direct SCell activation and hibernation	376

7.36.2.11	Interruptions at NR SRS carrier based switching	376

7.36.2.12	Interruptions at NR SCell dormancy	377

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	377

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	377

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	377

7.36.2.14	Interruption during NR measurement with autonomous gaps	378

7.36.2.15	Interruptions at SRS carrier based switching	378

7.36.2.16	Interruptions due to NR SRS antenna port switching	379

7.37	Interruptions during NR measurement with autonomous gaps	379

7.37.1	Introduction	379

7.37.2	Requirements	380

7.38 SCG Activation and Deactivation Delay	380

7.38.1	Introduction	380

7.38.2	SCG Activation Delay Requirement	380

7.38.3	SCG Deactivation Delay Requirement	381

8	UE Measurements Procedures in RRC_CONNECTED State	381

8.1	General Measurement Requirements	381

8.1.1	Introduction	381

8.1.2	Requirements	382

8.1.2.1	UE measurement capability	382

8.1.2.1.1	Monitoring of multiple layers using gaps	390

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	391

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	392

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	394

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	395

8.1.2.1.2	Network controlled small gap	395

8.1.2.2	E-UTRAN intra frequency measurements	397

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	397

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	402

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	406

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	407

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	408

8.1.2.3	E-UTRAN inter frequency measurements	408

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	409

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	414

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	421

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	421

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	421

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	422

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	424

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	425

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	426

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	433

8.1.2.4	Inter RAT measurements	433

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	433

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	438

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	438

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	442

8.1.2.4.5	E-UTRAN FDD – GSM measurements	442

8.1.2.4.6	E-UTRAN TDD – GSM measurements	447

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	447

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	449

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	449

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	449

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	450

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	450

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	450

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	450

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	452

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	452

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	452

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	452

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	453

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	453

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	455

8.1.2.4.21	E-UTRAN FDD – NR measurements	455

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	459

8.1.2.4.22	E-UTRAN TDD – NR measurements	463

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	463

8.1.2.4.23	Void	463

8.1.2.4.24	Void	463

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	463

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	465

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	465

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	466

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	466

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	466

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	468

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	470

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	472

8.1.2.6.5	Void	473

8.1.2.6.6	Void	473

8.1.2.6.7	Void	473

8.1.2.6.8	Void	473

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	473

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	474

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	475

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	477

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	479

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	481

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	482

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	484

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	486

8.1.2.7	E-UTRAN E-CID Measurements	488

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	488

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	489

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	490

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	491

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	491

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	492

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	494

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	497

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	501

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	504

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	504

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	505

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	505

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	506

8.1.2.10	Void	506

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	506

8.2.1	Introduction	506

8.2.2	Requirements	506

8.3	Measurements for E-UTRA carrier aggregation	511

8.3.1	Introduction	511

8.3.2	Measurements of the primary component carrier	511

8.3.3	Measurements of a secondary component carrier	511

8.3.3.1	Measurements of a secondary component carrier with active SCell	511

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	512

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	512

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	513

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	515

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	515

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	515

8.4.1	Introduction	515

8.4.2	Measurements on the primary component carrier	516

8.4.3	Measurements on a secondary component carrier	517

8.4.4	Measurements on both primary component carrier and a secondary component carrier	517

8.4.5	Measurements on different secondary component carriers	518

8.5	Measurements for UE category 0	519

8.5.1	Introduction	519

8.5.2 	Requirements	520

8.5.2.1 	E-UTRAN intra frequency measurements	520

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	520

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	523

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	525

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	529

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	529

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	530

8.6	Discovery signal measurements	531

8.6.1	Introduction	531

8.6.2	Requirements for CRS based discovery signal measurements	531

8.6.2.1	E-UTRAN intra frequency measurements	531

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	531

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	534

8.6.2.2	E-UTRAN inter frequency measurements	536

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	537

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	539

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	542

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	542

8.6.3	Requirements for CSI-RS based discovery signal measurements	542

8.6.3.1	E-UTRAN intra frequency measurements	542

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	543

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	545

8.6.3.2	E-UTRAN inter frequency measurements	547

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	548

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	550

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	553

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	553

8.7	Discovery signal measurements for E-UTRA carrier aggregation	553

8.7.1	Introduction	553

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	554

8.7.2.1	Measurements of the primary component carrier	554

8.7.2.2	Measurements of a secondary component carrier	554

8.7.2.3	Measurements of a secondary component carrier with active SCell	554

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	554

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	554

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	555

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	557

8.7.3.1	Measurements of the primary component carrier	557

8.7.3.2	Measurements of a secondary component carrier	557

8.7.3.3	Measurements of a secondary component carrier with active SCell	557

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	557

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	557

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	559

8.8	Measurements for E-UTRA dual connectivity	560

8.8.1	Introduction	560

8.8.2	Intra-frequency measurements requirements on PCell	560

8.8.3	Intra-frequency measurements requirements on PSCell	561

8.8.4	Inter-frequency and inter-RAT measurement requirements	561

8.8.5	Intra-frequency measurements with autonomous gaps	561

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	561

8.8.5.2	ECGI reporting delay	562

8.8.6	Inter-frequency measurements with autonomous gaps	562

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	562

8.8.6.2	ECGI reporting delay	563

8.8.7	SSTD Measurements	563

8.8.7.1	Introduction	563

8.8.7.2	SSTD Measurement requirements	563

8.8.7.3	SSTD Measurement Reporting Delay	564

8.8.8	Intra-frequency measurements requirements on SCell	564

8.9	MBSFN Measurements	564

8.9.1	Introduction	564

8.9.2	MBSFN RSRP Measurements	564

8.9.3	MBSFN RSRQ Measurements	565

8.9.4	MCH BLER Measurements	565

8.10	Proximity-based Services	565

8.10.1	Introduction	565

8.10.2	Requirements	565

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	565

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	566

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	566

8.11.1	Introduction	566

8.11.2	CRS based discovery signal measurements	567

8.11.2.1	E-UTRAN intra-frequency measurements	567

8.11.2.1.1	Requirements	567

8.11.2.1.1.1	Requirements when no DRX is used	567

8.11.2.1.1.1.1	Measurement Reporting Requirements	569

8.11.2.1.1.2	Requirements when DRX is used	570

8.11.2.1.1.2.1	Measurement Reporting Requirements	572

8.11.2.2	E-UTRAN inter-frequency measurements	573

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	573

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	577

8.11.3	CSI-RS based discovery signal measurements	577

8.11.3.1	E-UTRAN intra-frequency measurements	577

8.11.3.1.1	Requirements	577

8.11.3.1.1.1	Requirements when no DRX is used	577

8.11.3.1.1.1.1	Measurement Reporting Requirements	579

8.11.3.1.1.2	Requirements when DRX is used	579

8.11.3.1.1.2.1	Measurement Reporting Requirements	580

8.11.3.2	E-UTRAN inter-frequency measurements	581

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	581

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	585

8.11.4	RSSI measurements	585

8.11.4.1	E-UTRAN intra-frequency measurements	585

8.11.4.2	E-UTRAN inter-frequency measurements	585

8.11.5	Channel occupancy measurements	586

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	586

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	586

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	586

8.12.1	Introduction	586

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	586

8.12.2.1	Introduction	586

8.12.2.2	Measurements of a secondary component carrier	586

8.12.2.3	Measurements of a secondary component carrier with active SCell	586

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	587

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	587

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	589

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	592

8.12.3.1	Introduction	592

8.12.3.2	Measurements of a secondary component carrier	592

8.12.3.3	Measurements of a secondary component carrier with active SCell	592

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	592

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	592

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	594

8.13	Measurements for UE Category M1	596

8.13.1	Introduction	596

8.13.2	Requirements for UE category M1 with CE mode A	596

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	597

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	597

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	602

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	604

8.13.2.2	Void	608

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	608

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	608

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	611

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	613

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	614

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	614

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	616

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	618

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	619

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	619

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	619

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	620

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	620

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	620

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	621

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	621

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	622

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	623

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	623

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	624

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	628

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	630

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	634

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	634

8.13.3	Requirements for UE category M1 with CE mode B	635

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	635

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	636

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	640

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	643

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	648

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	649

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	649

8.13.3.2	Void	650

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	650

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	650

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	653

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	655

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	656

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	656

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	656

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	657

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	657

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	657

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	658

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	658

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	658

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	663

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	665

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	669

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	670

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	670

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	672

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	674

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	675

8.13A	Measurements for UE Category M1 for Satellite Access	675

8.13A.1	Introduction	675

8.13A.2	Requirements for UE category M1 with CE mode A	676

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	676

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	677

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	677

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	679

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	680

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	680

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	681

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	683

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	683

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	683

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	685

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	687

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	687

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	687

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	689

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	689

8.13A.3	Requirements for UE category M1 with CE mode B	689

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	690

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	690

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	690

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	692

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	694

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	694

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	695

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	697

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	697

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	697

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	699

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	701

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	701

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	701

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	703

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	703

8.14	Measurements for UE category NB1	703

8.14.1	Introduction	703

8.14.2	NB-IoT intra frequency measurements under normal coverage	704

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	704

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	704

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	704

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	704

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	704

8.14.4	Connected mode channel quality report for UE Category NB1	704

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	705

8.14.6	NB-IoT neighbour cell measurements	705

8.14.6.1	Introduction	705

8.14.6.2	Requirements	705

8.14.6.3	Intra-frequency neighbour cell measurements	706

8.14.6.4	Inter-frequency neighbour cell measurements	706

8.14.6.5	 Requirements for monitoring multiple carriers	707

8.14A	Measurements for UE category NB-IoT for Satellite Access	707

8.14A.1	Introduction	707

8.14A.2	NB-IoT intra frequency measurements under normal coverage	708

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	708

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	708

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	708

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	708

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	708

8.14A.4	Connected mode channel quality report for UE Category NB1	708

8.15	Void	709

8.16	Measurements for UE Category M2	709

8.16.1	Introduction	709

8.16.2	Requirements for UE category M2 with CE mode A	709

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	709

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	710

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	710

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	711

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	711

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	712

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	712

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	712

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	715

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	717

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	718

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	718

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	720

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	722

8.16.3	Requirements for UE category M2 with CE mode B	723

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	723

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	723

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	726

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	728

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	729

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	729

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	731

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	733

8.17	Measurements for E-UTRA – NR Dual Connectivity	734

8.17.1	Introduction	734

8.17.1.1	Measurement Gap Sharing	734

8.17.1A	Intrafrequency Measurements	735

8.17.2	SFTD Measurements	735

8.17.2.1	Introduction	735

8.17.2.2	SFTD Measurement requirements	735

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	736

8.17.2.3	SFTD Measurement Reporting Delay	737

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	737

8.17.3.1	Introduction	737

8.17.3.2	E-UTRAN FDD inter frequency measurements	737

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	737

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	739

8.17.3.3	E-UTRAN TDD inter frequency measurements	741

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	741

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	743

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	745

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	745

8.17.4.1.1	NR Inter-RAT cell identification	745

8.17.4.1.2	NR Inter-RAT measurement	747

8.17.4.1.3	NR Inter-RAT measurement reporting	747

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	748

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	748

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	748

8.17.4A.1.1	NR Inter-RAT cell identification	749

8.17.4A.1.2	NR Inter-RAT measurement	750

8.17.4A.1.3	NR Inter-RAT measurement reporting	751

8.17.4A.1.4	NR inter-RAT RSSI measurements	751

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	752

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	752

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	753

8.17.5.1	Introduction	753

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	753

8.17.5.2.1	Identification of a new UTRA FDD cell	753

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	753

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	753

8.17.5.2.4	Periodic Reporting	754

8.17.5.2.5	Event Triggered Reporting	754

8.17.5.2.6	Event-triggered Periodic Reporting	754

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	755

8.17.5.3.1	Periodic Reporting	756

8.17.5.3.2	Event Triggered Reporting	756

8.17.5.3.3	Event-triggered Periodic Reporting	756

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	757

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	757

8.17.7.1	Introduction	757

8.17.7.2	Identification of a new UTRA FDD cell for SON	757

8.17.7.2.1	Requirements when no DRX is used	757

8.17.7.2.2	Requirements when DRX is used	757

8.17.7.2.3	Reporting Delay	758

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	759

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	759

8.17.9.1	Introduction	759

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	759

8.17.9.2.1	Identification of a new UTRA FDD cell	759

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	759

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	760

8.17.9.2.4	Periodic Reporting	760

8.17.9.2.5	Event Triggered Reporting	760

8.17.9.2.6	Event-triggered Periodic Reporting	761

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	761

8.17.9.3.1	Periodic Reporting	762

8.17.9.3.2	Event Triggered Reporting	762

8.17.9.3.3	Event-triggered Periodic Reporting	762

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	763

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	763

8.17.11.1	Introduction	763

8.17.11.2	Identification of a new UTRA TDD cell for SON	763

8.17.11.2.1	Requirements when no DRX is used	763

8.17.11.2.2	Requirements when DRX is used	763

8.17.11.2.3	Reporting Delay	764

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	764

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	765

8.17.13.1	Introduction	765

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	765

8.17.13.2.1	GSM carrier RSSI	765

8.17.13.2.2	BSIC verification	765

8.17.13.2.3	Enhanced BSIC verification	767

8.17.13.2.4	Periodic Reporting	767

8.17.13.2.5	Event Triggered Reporting	767

8.17.13.2.6	Event-triggered Periodic Reporting	768

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	768

8.17.13.3.1	GSM carrier RSSI	768

8.17.13.3.2	BSIC verification	768

8.17.13.3.3	Periodic Reporting	770

8.17.13.3.4	Event Triggered Reporting	770

8.17.13.3.5	Event-triggered Periodic Reporting	770

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	770

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	770

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	770

8.17.15.1.1	RSTD Measurement Reporting Delay	771

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	771

8.17.15.2.1	RSTD Measurement Reporting Delay	771

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	771

8.17.15.3.1	RSTD Measurement Reporting Delay	772

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	772

8.17.15.4.1	RSTD Measurement Reporting Delay	772

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	773

8.17.16.1	Introduction	773

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	773

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	773

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	773

8.17.17.1	Introduction	773

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	773

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	773

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	773

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	773

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	773

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	773

8.18	Measurements for non-BL/CE UE	773

8.18.1	Introduction	773

8.18.2	Requirements for non-BL/CE UE with CE Mode B	774

8.18.2.1	E-UTRAN intra frequency measurements	774

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	774

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	774

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	775

8.19	Measurements for NR – E-UTRA Dual Connectivity	776

8.19.1	Introduction	776

8.19.2	Intra-frequency Measurements	776

8.19.3	Inter-frequency Measurements	776

8.19.4	Void	776

8.19.5	Intra-frequency E-CID Measurements	776

8.19.6	Intra-frequency measurements with autonomous gaps	777

8.19.6.1	Introduction	777

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	777

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	777

8.19.7	Inter-frequency measurements with autonomous gaps	777

8.19.7.1	Introduction	777

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	777

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	777

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	777

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	777

8.20	Inter-RAT NR Measurements for RedCap UE	777

8.20.1	Introduction	777

8.20.2	E-UTRAN FDD – NR measurements	777

8.20.2.1	Identification of a new NR cell	777

8.20.2.2	Periodic Reporting	781

8.20.2.3	Event Triggered Reporting	781

8.20.2.4	Event-triggered Periodic Reporting	781

8.20.3	E-UTRAN TDD – NR measurements	781

9	Measurements performance requirements for UE	782

9.1	E-UTRAN measurements	782

9.1.1	Introduction	782

9.1.2	Intra-frequency RSRP Accuracy Requirements	782

9.1.2.1	Absolute RSRP Accuracy	782

9.1.2.2	Relative Accuracy of RSRP	783

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	784

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	784

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	785

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	786

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	787

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	788

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	789

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	789

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	789

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	790

9.1.2B.1	Introduction	790

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	790

9.1.3	Inter-frequency RSRP Accuracy Requirements	791

9.1.3.1	Absolute RSRP Accuracy	791

9.1.3.2	Relative Accuracy of RSRP	792

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	793

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	793

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	794

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	794

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	795

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	796

9.1.3B.1	Introduction	796

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	796

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	796

9.1.4	RSRP Measurement Report Mapping	797

9.1.5	Intra-frequency RSRQ Accuracy Requirements	797

9.1.5.1	Absolute RSRQ Accuracy	797

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	798

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	799

9.1.5.4	Absolute WB-RSRQ Accuracy	800

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	801

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	802

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	802

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	802

9.1.5B.1	Introduction	802

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	802

9.1.6	Inter-frequency RSRQ Accuracy Requirements	803

9.1.6.1	Absolute RSRQ Accuracy	803

9.1.6.2	Relative Accuracy of RSRQ	804

9.1.6.3	Absolute WB-RSRQ Accuracy	805

9.1.6.4	Relative WB-RSRQ Accuracy	805

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	806

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	807

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	807

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	807

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	808

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	809

9.1.6B.1	Introduction	809

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	809

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	809

9.1.7	RSRQ Measurement Report Mapping	810

9.1.8	Power Headroom	810

9.1.8.1	Period	811

9.1.8.2	Reporting Delay	811

9.1.8.3	Void	811

9.1.8.4	Report Mapping	811

9.1.8A	Power Headroom for UE category M1 for satellite access	811

9.1.8A.1	Period	812

9.1.8A.2	Reporting Delay	812

9.1.8A.3	Void	812

9.1.8A.4	Report Mapping	812

9.1.9	UE Rx – Tx time difference	812

9.1.9.1	Measurement Requirement	812

9.1.9.2	Measurement Report mapping	813

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	814

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	815

9.1.10	Reference Signal Time Difference (RSTD)	816

9.1.10.1	Intra-Frequency Accuracy Requirement	816

9.1.10.2	Inter-Frequency Accuracy Requirement	817

9.1.10.3	RSTD Measurement Report Mapping	819

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	819

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	820

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	821

9.1.11	Carrier aggregation measurement accuracy	822

9.1.11.1	Primary component carrier accuracy requirement	823

9.1.11.2	Secondary component carrier accuracy requirement	823

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	823

9.1.11.4	Secondary component carrier relative accuracy requirement	823

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	823

9.1.13	Measurement accuracy for UE category 0	824

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	824

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	824

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	825

9.1.14	Accuracy requirements for Discovery Signal Measurements	826

9.1.14.1	Introduction	826

9.1.14.2	RSRP measurements in discovery signal occasions	826

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	826

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	826

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	826

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	827

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	828

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	828

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	828

9.1.14.3.3	CSI-RSRP measurement report mapping	829

9.1.14.4	RSRQ measurements in discovery signal occasions	829

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	829

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	830

9.1.15.1.1	Primary component carrier accuracy requirement	830

9.1.15.1.2	Secondary component carrier accuracy requirement	830

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	830

9.1.15.1.4	Secondary component carrier relative accuracy requirement	830

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	830

9.1.15.2.1	Primary component carrier accuracy requirement	830

9.1.15.2.2	Secondary component carrier accuracy requirement	830

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	830

9.1.15.2.4	Secondary component carrier relative accuracy requirement	830

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	831

9.1.17	RS-SINR Measurements	831

9.1.17.1	Measurement Report Mapping	831

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	832

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	832

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	832

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	832

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	833

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	834

9.1.18.1	Introduction	834

9.1.18.2	RSRP measurements	834

9.1.18.2.1	RSRP measurement report mapping	834

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	834

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	835

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	835

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	836

9.1.18.3	RSRQ measurements	836

9.1.18.3.1	RSRQ measurement report mapping	836

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	836

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	837

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	837

9.1.18.4	CSI-RSRP measurements	838

9.1.18.4.1	CSI-RSRP measurement report mapping	838

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	838

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	838

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	839

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	839

9.1.18.5	RSSI measurements	840

9.1.18.5.1	RSSI measurement report mapping	840

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	840

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	841

9.1.18.6	Channel occupancy measurements	841

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	841

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	841

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	842

9.1.19.1	Introduction	842

9.1.19.2	Accuracy requirements for measurements on SCC	842

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	842

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	842

9.1.20	SFN and Subframe Time Difference (SSTD)	843

9.1.20.1	SSTD Accuracy Requirement	843

9.1.20.2	SSTD Measurement Report Mapping	843

9.1.21	Measurement accuracy for UE category M1	844

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	844

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	846

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	847

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	849

9.1.21.5	RSRP Measurement Report Mapping	850

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	850

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	851

9.1.21.8	RSRQ Measurement Report Mapping	852

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	852

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	853

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	854

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	855

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	856

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	857

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	858

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	859

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	860

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	861

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	862

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	863

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	866

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	869

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	869

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	872

9.1.21A	Measurement accuracy for UE category M1 for satellite access	873

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	874

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	875

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	876

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	878

9.1.21A.5	RSRP Measurement Report Mapping	879

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	879

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	880

9.1.21A.8	RSRQ Measurement Report Mapping	881

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	881

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	882

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	883

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	884

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	885

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	886

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	887

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	888

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	889

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	889

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	891

9.1.22	Measurement accuracy for UE Category NB1	892

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	892

9.1.22.2	Void	893

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	893

9.1.22.4	Void	894

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	894

9.1.22.6	Void	895

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	895

9.1.22.8	Void	896

9.1.22.9	NRSRP Measurement Report Mapping	896

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	896

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	897

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	898

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	899

9.1.22.14	NRSRQ Measurement Report Mapping	900

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	901

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	901

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	902

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	902

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	902

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	903

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	904

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	905

9.1.22A.5	NRSRP Measurement Report Mapping	906

9.1.22A.6	NRSRQ Measurement Report Mapping	906

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	906

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	906

9.1.23	Power Headroom for UE Category NB1	907

9.1.23.1	Period	907

9.1.23.2	Reporting Delay	907

9.1.23.3	Report Mapping for UE Category NB1	907

9.1.23.3.1	Void	909

9.1.23.3.2	Void	909

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	909

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	910

9.1.23A.1	Period	910

9.1.23A.2	Reporting Delay	910

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	911

9.1.23A.3.1	Void	911

9.1.23A.3.2	Void	911

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	911

9.1.24	Void	912

9.1.25	Measurement accuracy for UE category M2	912

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	912

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	913

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	914

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	915

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	917

9.1.26	Measurement Accuracy for non-BL CE UE	918

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	919

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	921

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	921

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	923

9.1.26.5	RSRP Measurement Report Mapping	924

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	924

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	924

9.1.26.8	RSRQ Measurement Report Mapping	925

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	925

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	926

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	927

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	928

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	929

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	929

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	929

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	930

9.1.27	SFN and frame Timing Difference (SFTD)	931

9.1.27.1	SFTD Accuracy Requirement	931

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	933

9.1.28.1	SFTD Accuracy Requirement under CCA	933

9.2	UTRAN FDD Measurements	934

9.2.1	UTRAN FDD CPICH RSCP	934

9.2.2	Void	935

9.2.3	UTRAN FDD CPICH Ec/No	935

9.3	UTRAN TDD Measurements	935

9.3.1	UTRAN TDD P-CCPCH RSCP	936

9.3.2	Void	936

9.3.3	Void	936

9.4	GSM Measurements	936

9.4.1	GSM carrier RSSI	936

9.5	CDMA2000 1x RTT Measurements	936

9.5.1	CDMA2000 1x RTT Pilot Strength	936

9.6	PCMAX,c	937

9.6.1	Report Mapping	937

9.6.2 	Estimation Period	937

9.6.3	Reporting Delay	937

9.7	IEEE802.11 Measurements	937

9.7.1	WLAN RSSI	937

9.7.2	WLAN RSSI Measurement Report Mapping	937

9.8	MBSFN Measurements	938

9.8.1	Introduction	938

9.8.2	MBSFN RSRP	938

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	938

9.8.2.2	MBSFN RSRP measurement report mapping	939

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	939

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	940

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	940

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	940

9.8.3	MBSFN RSRQ	941

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	941

9.8.3.2	MBSFN RSRQ measurement report mapping	941

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	942

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	942

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	942

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	943

9.8.4	MCH BLER	943

9.8.4.1	Measurement report mapping for MCH BLER	943

9.8.4.2	Measurement report mapping for MCH Block Number	944

9.9	ProSe Measurements	945

9.9.1	Introduction	945

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	945

9.9.2.1	Absolute S-RSRP Accuracy	945

9.9.2.2	Relative Accuracy of S-RSRP	946

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	947

9.9.3.1	Absolute SD-RSRP Accuracy	947

9.9.3.2	Relative Accuracy of SD-RSRP	947

9.10	V2X Measurements	948

9.10.1	Introduction	948

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	948

9.10.2.1	Absolute S-RSRP Accuracy	948

9.10.2.2	Relative Accuracy of S-RSRP	949

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	949

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	949

9.10.4	S-RSSI Measurement Accuracy Requirements	950

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	950

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	950

9.11	NR Measurements	951

9.11.1	NR SS-RSRP Measurements	951

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	951

9.11.2	NR SS-RSRQ Measurements	952

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	952

9.11.3	NR SS-SINR Measurements	952

9.11.4	NR SS-RSRP Measurements under CCA	953

9.11.5	NR SS-RSRQ Measurements under CCA	953

9.11.6	NR SS-SINR Measurements under CCA	953

9.11.7	NR RSSI Measurements under CCA	953

9.11.8	NR Channel Occupancy Measurements under CCA	954

10	Measurements Performance Requirements for E-UTRAN	954

10.1	Received Interference Power	954

10.1.1	Absolute accuracy requirement	954

10.1.2	Relative accuracy requirement	954

10.1.3	Received Interference Power measurement report mapping	955

10.2	Angle of Arrival (AOA)	955

10.2.1	Range/mapping	955

10.3	Timing Advance (TADV)	955

10.3.1	Report mapping	955

11	ProSe Requirements in Any Cell Selection state	956

11.1	Introduction	956

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	956

11.2.1	Introduction	956

11.2.2	ProSe UE transmission timing	956

11.3	Initiation/Cease of SLSS Transmissions	957

11.3.1	Introduction	957

11.3.2	Requirements	957

11.4	Measurements for ProSe in Any Cell Selection State	957

11.4.1	Introduction	957

11.4.2	Requirements	957

11.4.2.1	E-UTRA FDD	957

11.4.2.2	E-UTRA TDD	958

11.5	Selection / Reselection of ProSe Synchronization Reference	958

11.5.1	Introduction	958

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	958

11.5.2.1	Introduction	958

11.5.2.2	Requirements	958

11.6	Void	959

11.7	Selection / Reselection of ProSe relay UE	959

11.7.1	Introduction	959

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	959

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	959

12.1	Introduction	959

12.2	Transmit Timing	960

12.2.1	GNSS as timing reference	960

12.3	Interruption	960

12.4	Reliability of GNSS signal	960

13	V2X Requirements	960

13.1	Introduction	960

13.2	UE Transmit Timing	961

13.2.1	Introduction	961

13.2.2	GNSS as synchronization reference source	961

13.2.3	Serving cell/PCell as synchronization reference source	961

13.2.4	SyncRef UE as synchronization reference source	961

13.3	Initiation/Cease of SLSS Transmissions	961

13.3.1	Introduction	961

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	961

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	962

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	962

13.4	Selection / Reselection of V2X Synchronization Reference Source	963

13.5	Autonomous Resource Selection/Reselection measurements	964

13.5.1	Introduction	964

13.5.2	PSSCH-RSRP measurements	964

13.5.3	S-RSSI measurements	964

13.6	Congestion Control measurements	964

13.7	Interruption	964

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	964

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	964

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	965

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	965

13.8	Reliability of GNSS signal	965

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	966

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	966

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	851

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	868

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	871

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	871

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	873

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	875

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	876

A.3.2.4	OCNG Patterns for V2X sidelink	877

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	877

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	878

A.3.3	Reference DRX Configurations	878

A.3.4	ABS Transmission Configurations	879

A.3.4.1	Non-MBSFN ABS Transmission Configurations	879

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	879

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	879

A.3.4.2	MBSFN ABS Transmission Configurations	880

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	880

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	881

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	881

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	881

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	882

A.3.6.1	Introduction	882

A.3.7	Test Cases with Different Channel Bandwidths	882

A.3.7.1	Introduction	882

A.3.7.2	Principle of testing	882

A.3.8	Antenna Configuration	882

A.3.8.1	Antenna connection for 4 Rx capable UEs	882

A.3.8.1.1 Introduction	882

A.3.8.1.2 Principle of testing	882

A.3.8.1.2.1 Single carrier tests	882

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	883

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2	Antenna connection for 8 Rx capable UEs	884

A.3.8.2.1	Introduction	884

A.3.8.2.2	Principle of testing	884

A.3.8.2.2.1	Single carrier tests	884

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	885

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	885

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	885

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	885

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	885

A.3.9.1	Introduction	885

A.3.9.2	Principle of testing	885

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	886

A.3.10.1	Introduction	886

A.3.10.2	Principle of testing	886

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	886

A.3.11.1	Introduction	886

A.3.11.2	Principle of Testing	886

A.3.12	Proximity-based Services	886

A.3.12.1	Introduction	886

A.3.12.2	Reference DRX configurations for ProSe tests	886

A.3.12.3	Test Cases with Different Channel Bandwidths	887

A.3.12.3.1	Introduction	887

A.3.12.3.2	Principle of testing	887

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	887

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	894

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	897

A.3.12.6.1	FDD	897

A.3.12.7	Reference measurement channels for ProSe Direct Communication	897

A.3.12.7.1	FDD	897

A.3.12.8	ProSe Receive Traffic Generator	898

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	898

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	898

A.3.13	Time Offset between Cells	899

A.3.13.1	Introduction	899

A.3.13.2	Definition	899

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	899

A.3.14.1	Introduction	899

A.3.14.2	Principle of testing	899

A.3.15	Dual connectivity test cases with different combination of duplex mode	899

A.3.15.1	Introduction	899

A.3.15.2	Principle of testing	899

A.3.16	Reference PRACH Configurations	900

A.3.17	Listen before talk model	900

A.3.17.1	Introduction	900

A.3.17.2	Definition	900

A.3.18	Reference NPRACH Configurations	901

A.3.19	Dual connectivity test cases with different bandwidth combinations	902

A.3.19.1	Introduction	902

A.3.19.2	Principle of testing	902

A.3.20	Category M1 UE Test Cases	903

A.3.20.1	Introduction	903

A.3.20.2	Principle of Cat-M1 UE Testing	903

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	904

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	905

A.3.21.1	Introduction	905

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	905

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	906

A.3.22	Category 1bis UE Test Cases	907

A.3.22.1	Introduction	907

A.3.22.2	Principle of Category 1bis UE Testing	907

A.3.23	Category NB2 UE Test Cases	911

A.3.23.1	Introduction	911

A.3.23.2	Principle of Category NB2 UE Testing	911

A.3.24	V2X sidelink communication	914

A.3.24.1	Introduction	914

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	915

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	919

A.3.25	Category M2 UE Test Cases	920

A.3.25.1	Introduction	920

A.3.25.2	Principle of Cat-M2 UE Testing	920

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	921

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	922

A.3.26.1	Introduction	922

A.3.26.2	Principle of testing	922

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	922

A.3.27.1	Introduction	922

A.3.27.2	Principle of INACTIVE cell re-selection Testing	922

A.3.28	Reference configurations for satellites	922

A.4	E-UTRAN RRC_IDLE state	923

A.4.2	Cell Re-Selection	923

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	923

A.4.2.1.1	Test Purpose and Environment	923

A.4.2.1.2	Test Requirements	924

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	925

A.4.2.2.1	Test Purpose and Environment	925

A.4.2.2.2	Test Requirements	926

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	927

A.4.2.3.1	Test Purpose and Environment	927

A.4.2.3.2	Test Requirements	928

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	929

A.4.2.4.1	Test Purpose and Environment	929

A.4.2.4.2	Test Requirements	930

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	931

A.4.2.5.1	Test Purpose and Environment	931

A.4.2.5.2	Test Requirements	932

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	933

A.4.2.6.1	Test Purpose and Environment	933

A.4.2.6.2	Test Requirements	934

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	935

A.4.2.7.1	Test Purpose and Environment	935

A.4.2.7.2	Test Requirements	936

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	937

A.4.2.8.1	Test Purpose and Environment	937

A.4.2.8.2	Test Requirements	938

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	939

A.4.2.9.1	Test Purpose and Environment	939

A.4.2.9.2	Test Requirements	939

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	939

A.4.2.10.1	Test Purpose and Environment	939

A.4.2.10.2	Test Requirements	943

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	943

A.4.2.11.1	Test Purpose and Environment	943

A.4.2.11.2	Test Requirements	947

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	947

A.4.2.12.1	Test Purpose and Environment	947

A.4.2.12.2	Test Requirements	949

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	950

A.4.2.13.1	Test Purpose and Environment	950

A.4.2.13.2	Test Requirements	951

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	952

A.4.2.14.1	Test Purpose and Environment	952

A.4.2.14.2	Test Requirements	953

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	954

A.4.2.15.1	Test Purpose and Environment	954

A.4.2.15.2	Test Requirements	955

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	956

A.4.2.16.1	Test Purpose and Environment	956

A.4.2.16.2	Test Requirements	957

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	958

A.4.2.17.1	Test Purpose and Environment	958

A.4.2.17.2	Test Requirements	959

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	960

A.4.2.18.1	Test Purpose and Environment	960

A.4.2.18.2	Test Requirements	962

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	963

A.4.2.19.1	Test Purpose and Environment	963

A.4.2.19.2	Test Requirements	965

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	966

A.4.2.20.1	Test Purpose and Environment	966

A.4.2.20.2	Test Requirements	967

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	968

A.4.2.21.1	Test Purpose and Environment	968

A.4.2.21.2	Test Requirements	969

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	970

A.4.2.22.1	Test Purpose and Environment	970

A.4.2.22.2	Test Requirements	971

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	972

A.4.2.23.1	Test Purpose and Environment	972

A.4.2.23.2	Test Requirements	973

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	974

A.4.2.24.1	Test Purpose and Environment	974

A.4.2.24.2	Test Requirements	977

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	978

A.4.2.25.1	Test Purpose and Environment	978

A.4.2.25.2	Test Requirements	979

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	980

A.4.2.26.1	Test Purpose and Environment	980

A.4.2.26.2	Test Requirements	981

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	982

A.4.2.27.1	Test Purpose and Environment	982

A.4.2.27.2	Test Requirements	983

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	984

A.4.2.28.1	Test Purpose and Environment	984

A.4.2.28.2	Test Requirements	985

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	986

A.4.2.29.1	Test Purpose and Environment	986

A.4.2.29.2	Test Requirements	987

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	988

A.4.2.30.1	Test Purpose and Environment	988

A.4.2.30.2	Test Requirements	989

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	990

A.4.2.31.1	Test Purpose and Environment	990

A.4.2.31.2	Test Requirements	991

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	992

A.4.2.32.1	Test Purpose and Environment	992

A.4.2.32.2	Test Requirements	993

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	994

A.4.2.33.1	Test Purpose and Environment	994

A.4.2.33.2	Test Requirements	995

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	996

A.4.2.34.1	Test Purpose and Environment	996

A.4.2.34.2	Test Requirements	997

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	998

A.4.2.35.1	Test Purpose and Environment	998

A.4.2.35.2	Test Requirements	1000

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1001

A.4.2.36.1	Test Purpose and Environment	1001

A.4.2.36.2	Test Requirements	1003

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1004

A.4.2.37.1	Test Purpose and Environment	1004

A.4.2.37.2	Test Requirements	1006

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1007

A.4.2.38.1	Test Purpose and Environment	1007

A.4.2.38.2	Test Requirements	1009

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1010

A.4.2.39.1	Test Purpose and Environment	1010

A.4.2.39.2	Test Requirements	1011

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1012

A.4.2.40.1	Test Purpose and Environment	1012

A.4.2.40.2	Test Requirements	1013

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1014

A.4.2.41.1	Test Purpose and Environment	1014

A.4.2.41.2	Test Requirements	1017

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1018

A.4.2.42.1	Test Purpose and Environment	1018

A.4.2.42.2	Test Requirements	1020

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1021

A.4.2.43.1	Test Purpose and Environment	1021

A.4.2.43.2	Test Requirements	1023

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1024

A.4.2.44.1	Test Purpose and Environment	1024

A.4.2.44.2	Test Requirements	1026

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1027

A.4.2.45.1	Test Purpose and Environment	1027

A.4.2.45.2	Test Requirements	1029

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1030

A.4.2.46.1	Test Purpose and Environment	1030

A.4.2.46.2	Test Requirements	1032

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1033

A.4.2.47.1	Test Purpose and Environment	1033

A.4.2.47.2	Test Requirements	1035

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1036

A.4.2.48.1	Test Purpose and Environment	1036

A.4.2.48.2	Test Requirements	1037

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1038

A.4.2.49.1	Test Purpose and Environment	1038

A.4.2.49.2	Test Requirements	1040

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1041

A.4.2.50.1	Test Purpose and Environment	1041

A.4.2.50.2	Test Requirements	1043

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1044

A.4.2.51.1	Test Purpose and Environment	1044

A.4.2.51.2	Test Requirements	1046

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1047

A.4.2.52.1	Test Purpose and Environment	1047

A.4.2.52.2	Test Requirements	1049

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1050

A.4.2.53.1	Test Purpose and Environment	1050

A.4.2.53.2	Test Requirements	1051

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1052

A.4.2.54.1	Test Purpose and Environment	1052

A.4.2.54.2	Test Requirements	1054

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1055

A.4.2.55.1	Test Purpose and Environment	1055

A.4.2.55.2	Test Requirements	1057

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1058

A.4.2.56.1	Test Purpose and Environment	1058

A.4.2.56.2	Test Requirements	1059

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1060

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1060

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1060

A.4.3.1.1.1	Test Purpose and Environment	1060

A.4.3.1.1.2	Test Requirements	1062

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1062

A.4.3.1.2.1	Test Purpose and Environment	1062

A.4.3.1.2.2	Test Requirements	1065

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1065

A.4.3.1.3.1	Test Purpose and Environment	1065

A.4.3.1.3.2	Test Requirements	1068

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1068

A.4.3.1.4.1	Test Purpose and Environment	1068

A.4.3.1.4.2	Test Requirements	1069

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1069

A.4.3.1.5.1	Test Purpose and Environment	1069

A.4.3.1.5.2	Test Requirements	1072

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1073

A.4.3.2.1	Test Purpose and Environment	1073

A.4.3.2.1.1	Void	1073

A.4.3.2.1.2	1.28Mcps TDD option	1073

A.4.3.2.1.3	Void	1075

A.4.3.2.2	Test Requirements	1075

A.4.3.2.2.1	1.28Mcps TDD option	1075

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1075

A.4.3.2A.1	Test Purpose and Environment	1075

A.4.3.2A.2	Test Requirements	1079

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1079

A.4.3.3.1	Test Purpose and Environment	1079

A.4.3.3.2	Test Requirements	1082

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1082

A.4.3.3A.1	Test Purpose and Environment	1082

A.4.3.3A.2	Test Requirements	1087

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1088

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1088

A.4.3.4.1.1	Test Purpose and Environment	1088

A.4.3.4.1.2	Test Requirements	1090

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1090

A.4.3.4.2.1	Test Purpose and Environment	1090

A.4.3.4.2.2	Test Requirements	1092

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1092

A.4.3.4.3.1	Test Purpose and Environment	1092

A.4.3.4.3.2	Test Requirements	1095

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1095

A.4.3.4.4.1	Test Purpose and Environment	1095

A.4.3.4.4.2	Test Requirements	1099

A.4.4	E-UTRAN to GSM Cell Re-Selection	1099

A.4.4.1	E-UTRAN FDD – GSM:	1099

A.4.4.1.1	Test Purpose and Environment	1099

A.4.4.1.2	Test Requirements	1101

A.4.4.2	E-UTRAN TDD – GSM:	1101

A.4.4.2.1	Test Purpose and Environment	1101

A.4.4.2.2	Test Requirements	1103

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1104

A.4.5.1	E-UTRAN FDD – HRPD	1104

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1104

A.4.5.1.1.1	Test Purpose and Environment	1104

A.4.5.1.1.2	Test Requirements	1106

A.4.5.2	E-UTRAN TDD – HRPD	1106

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1106

A.4.5.2.1.1	Test Purpose and Environment	1106

A.4.5.2.1.2	Test Requirements	1109

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1109

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1109

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1109

A.4.6.1.1.1	Test Purpose and Environment	1109

A.4.6.1.1.2	Test Requirements	1112

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1112

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1112

A.4.6.2.1.1	Test Purpose and Environment	1112

A.4.6.2.1.2	Test Requirements	1115

A.4.7	Idle State Positioning Measurement for UE category NB1	1115

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1115

A.4.7.1.1	Test Purpose and Environment	1115

A.4.7.1.2	Test Requirements	1120

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1121

A.4.7.2.1	Test Purpose and Environment	1121

A.4.7.4.2	Test Requirements	1125

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1126

A.4.7.3.1	Test Purpose and Environment	1126

A.4.7.3.2	Test Requirements	1130

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1131

A.4.7.4.1	Test Purpose and Environment	1131

A.4.7.4.2	Test Requirements	1135

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1136

A.5.1	E-UTRAN Handover	1136

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1136

A.5.1.1.1	Test Purpose and Environment	1136

A.5.1.1.2	Test Requirements	1138

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1138

A.5.1.2.1	Test Purpose and Environment	1138

A.5.1.2.2	Test Requirements	1140

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1140

A.5.1.3.1	Test Purpose and Environment	1140

A.5.1.3.2	Test Requirements	1142

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1142

A.5.1.4.1	Test Purpose and Environment	1142

A.5.1.4.2	Test Requirements	1144

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1144

A.5.1.5.1	Test Purpose and Environment	1144

A.5.1.5.2	Test Requirements	1146

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1146

A.5.1.6.1	Test Purpose and Environment	1146

A.5.1.6.2	Test Requirements	1148

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1148

A.5.1.7.1	Test Purpose and Environment	1148

A.5.1.7.2	Test Requirements	1151

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1151

A.5.1.8.1	Test Purpose and Environment	1151

A.5.1.8.2	Test Requirements	1154

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1154

A.5.1.9.1	Test Purpose and Environment	1154

A.5.1.9.2	Test Requirements	1155

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1155

A.5.1.10.1	Test Purpose and Environment	1155

A.5.1.10.2	Test Requirements	1157

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1157

A.5.1.11.1	Test Purpose and Environment	1157

A.5.1.11.2	Test Requirements	1159

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1159

A.5.1.12.1	Test Purpose and Environment	1159

A.5.1.12.2	Test Requirements	1161

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1161

A.5.1.13.1	Test Purpose and Environment	1161

A.5.1.13.2	Test Requirements	1163

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1164

A.5.1.14.1	Test Purpose and Environment	1164

A.5.1.14.2	Test Requirements	1165

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1166

A.5.1.15.1	Test Purpose and Environment	1166

A.5.1.15.2	Test Requirements	1167

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1168

A.5.1.16.1	Test Purpose and Environment	1168

A.5.1.16.2	Test Requirements	1169

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1170

A.5.1.17.1	Test Purpose and Environment	1170

A.5.1.17.2	Test Requirements	1171

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1172

A.5.1.18.1	Test Purpose and Environment	1172

A.5.1.18.2	Test Requirements	1173

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1174

A.5.1.19.1	Test Purpose and Environment	1174

A.5.1.19.2	Test Requirements	1175

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1176

A.5.1.20.1	Test Purpose and Environment	1176

A.5.1.20.2	Test Requirements	1177

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1178

A.5.1.21.1	Test Purpose and Environment	1178

A.5.1.21.2	Test Requirements	1179

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1179

A.5.1.22.1	Test Purpose and Environment	1179

A.5.1.22.2	Test Requirements	1181

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1181

A.5.1.23.1	Test Purpose and Environment	1181

A.5.1.23.2	Test Requirements	1183

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1183

A.5.1.24.1	Test Purpose and Environment	1183

A.5.1.24.2	Test Requirements	1185

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1185

A.5.1.25.1	Test Purpose and Environment	1185

A.5.1.25.2	Test Requirements	1187

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1188

A.5.1.26.1	Test Purpose and Environment	1188

A.5.1.26.2	Test Requirements	1189

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1190

A.5.1.27.1	Test Purpose and Environment	1190

A.5.1.27.2	Test Requirements	1191

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1192

A.5.1.28.1	Test Purpose and Environment	1192

A.5.1.28.2	Test Requirements	1193

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1194

A.5.1.29.1	Test Purpose and Environment	1194

A.5.1.29.2	Test Requirements	1195

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1196

A.5.1.30.1	Test Purpose and Environment	1196

A.5.1.30.2	Test Requirements	1197

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1198

A.5.1.31.1	Test Purpose and Environment	1198

A.5.1.31.2	Test Requirements	1199

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1200

A.5.1.32.1	Test Purpose and Environment	1200

A.5.1.32.2	Test Requirements	1201

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1202

A.5.1.33.1	Test Purpose and Environment	1202

A.5.1.13.2	Test Requirements	1203

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1204

A.5.1.34.1	Test Purpose and Environment	1204

A.5.1.34.2	Test Requirements	1205

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1206

A.5.1.35.1	Test Purpose and Environment	1206

A.5.1.35.2	Test Requirements	1207

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1208

A.5.1.36.1	Test Purpose and Environment	1208

A.5.1.36.2	Test Requirements	1209

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1210

A.5.1.37.1	Test Purpose and Environment	1210

A.5.1.37.2	Test Requirements	1211

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1212

A.5.1.38.1	Test Purpose and Environment	1212

A.5.1.38.2	Test Requirements	1213

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1214

A.5.1.39.1	Test Purpose and Environment	1214

A.5.1.39.2	Test Requirements	1215

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1216

A.5.1.40.1	Test Purpose and Environment	1216

A.5.1.40.2	Test Requirements	1217

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1218

A.5.1.41.1	Test Purpose and Environment	1218

A.5.1.41.2	Test Requirements	1220

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1221

A.5.1.42.1	Test Purpose and Environment	1221

A.5.1.42.2	Test Requirements	1222

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1223

A.5.1.43.1	Test Purpose and Environment	1223

A.5.1.43.2	Test Requirements	1224

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1225

A.5.1.44.1	Test Purpose and Environment	1225

A.5.1.44.2	Test Requirements	1226

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1227

A.5.1.45.1	Test Purpose and Environment	1227

A.5.1.45.1	Test Requirements	1228

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1229

A.5.1.46.1	Test Purpose and Environment	1229

A.5.1.46.2	Test Requirements	1230

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1231

A.5.1.47.1	Test Purpose and Environment	1231

A.5.1.47.2	Test Requirements	1232

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1233

A.5.1.48.1	Test Purpose and Environment	1233

A.5.1.48.2	Test Requirements	1234

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1235

A.5.1.49.1	Test Purpose and Environment	1235

A.5.1.49.2	Test Requirements	1236

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1237

A.5.1.50.1	Test Purpose and Environment	1237

A.5.1.50.2	Test Requirements	1238

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1239

A.5.1.51.1	Test Purpose and Environment	1239

A.5.1.51.2	Test Requirements	1241

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1241

A.5.152.1	Test Purpose and Environment	1241

A.5.1.52.2	Test Requirements	1244

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1245

A.5.1.53.1	Test Purpose and Environment	1245

A.5.1.53.2	Test Requirements	1247

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1248

A.5.1.54.1	Test Purpose and Environment	1248

A.5.1.54.2	Test Requirements	1250

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1251

A.5.1.55.1	Test Purpose and Environment	1251

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1255

A.5.1.56.1	Test Purpose and Environment	1255

A.5.1.56.2	Test Requirements	1258

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1259

A.5.1.57.1	Test Purpose and Environment	1259

A.5.1.57.2	Test Requirements	1261

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1261

A.5.1.58.1	Test Purpose and Environment	1261

A.5.1.58.2	Test Requirements	1264

A.5.2	E-UTRAN Handover to other RATs	1264

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1264

A.5.2.1.1	Test Purpose and Environment	1264

A.5.2.1.2	Test Requirements	1266

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1267

A.5.2.2.1	Test Purpose and Environment	1267

A.5.2.2.2	Test Requirements	1270

A.5.2.3 E-UTRAN FDD- GSM Handover	1270

A.5.2.3.1	Test Purpose and Environment	1270

A.5.2.3.2	Test Requirements	1271

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1272

A.5.2.4.1	Test Purpose and Environment	1272

A.5.2.4.1.1	Void	1272

A.5.2.4.1.2	1.28 Mcps TDD option	1272

A.5.2.4.1.3	Void	1274

A.5.2.4.2	Test Requirements	1274

A.5.2.4.2.1	Void	1274

A.5.2.4.2.2	1.28 Mcps TDD option	1274

A.5.2.4.2.3	Void	1274

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1274

A.5.2.5.1	Test Purpose and Environment	1274

A.5.2.5.1.3	Void	1277

A.5.2.5.2	Test Requirements	1277

A.5.2.5.2.1	Void	1277

A.5.2.5.2.2	1.28 Mcps TDD option	1277

A.5.2.5.2.3	Void	1277

A.5.2.6	E-UTRAN TDD - GSM Handover	1277

A.5.2.6.1	Test Purpose and Environment	1277

A.5.2.6.2	Test Requirements	1279

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1280

A.5.2.7.1	Test Purpose and Environment	1280

A.5.2.7.2	Test Requirements	1282

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1282

A.5.2.8.1	Test Purpose and Environment	1282

A.5.2.8.2	Test Requirements	1283

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1284

A.5.2.9.1	Test Purpose and Environment	1284

A.5.2.9.2	Test Requirements	1285

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1286

A.5.2.10.1	Test Purpose and Environment	1286

A.5.2.10.2	Test Requirements	1288

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1288

A.5.2.10A.1	Test Purpose and Environment	1288

A.5.2.10A.2	Test Requirements	1291

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1291

A.5.2.10B.1	Test Purpose and Environment	1291

A.5.2.10B.2	Test Requirements	1294

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1294

A.5.2.11.1	Test Purpose and Environment	1294

A.5.2.11.2	Test Requirements	1294

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1295

A.5.3.1	E-UTRAN FDD – HRPD Handover	1295

A.5.3.1.1	Test Purpose and Environment	1295

A.5.3.1.2	Test Requirements	1297

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1297

A.5.3.2.1	Test Purpose and Environment	1297

A.5.3.2.2	Test Requirements	1300

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1300

A.5.3.3.1	Test Purpose and Environment	1300

A.5.3.3.2	Test Requirements	1303

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1303

A.5.3.4.1	Test Purpose and Environment	1303

A.5.3.4.2	Test Requirements	1304

A.5.3.5	E-UTRAN TDD – HRPD Handover	1305

A.5.3.5.1	Test Purpose and Environment	1305

A.5.3.5.2	Test Requirements	1308

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1308

A.5.3.6.1	Test Purpose and Environment	1308

A.5.3.6.2	Test Requirements	1311

A.6	RRC Connection Control	1311

A.6.1	RRC Re-establishment	1311

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1311

A.6.1.1.1	Test Purpose and Environment	1311

A.6.1.1.2	Test Requirements	1312

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1313

A.6.1.2.1	Test Purpose and Environment	1313

A.6.1.2.2	Test Requirements	1314

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1315

A.6.1.3.1	Test Purpose and Environment	1315

A.6.1.3.2	Test Requirements	1316

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1317

A.6.1.4.1	Test Purpose and Environment	1317

A.6.1.4.2	Test Requirements	1318

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1319

A.6.1.5.1	Test Purpose and Environment	1319

A.6.1.5.2	Test Requirements	1319

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1319

A.6.1.6.1	Test Purpose and Environment	1319

A.6.1.6.2	Test Requirements	1321

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1322

A.6.1.7.1	Test Purpose and Environment	1322

A.6.1.7.2	Test Requirements	1323

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1324

A.6.1.8.1	Test Purpose and Environment	1324

A.6.1.8.2	Test Requirements	1325

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1326

A.6.1.9.1	Test Purpose and Environment	1326

A.6.1.9.2	Test Requirements	1327

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1328

A.6.1.10.1	Test Purpose and Environment	1328

A.6.1.10.2	Test Requirements	1329

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1330

A.6.1.11.1	Test Purpose and Environment	1330

A.6.1.11.2	Test Requirements	1331

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1332

A.6.1.12.1	Test Purpose and Environment	1332

A.6.1.12.2	Test Requirements	1333

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1334

A.6.1.13.1	Test Purpose and Environment	1334

A.6.1.13.2	Test Requirements	1335

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1336

A.6.1.14.1	Test Purpose and Environment	1336

A.6.1.14.2	Test Requirements	1337

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1338

A.6.1.15.1	Test Purpose and Environment	1338

A.6.1.15.2	Test Requirements	1340

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1341

A.6.1.16.1	Test Purpose and Environment	1341

A.6.1.16.2	Test Requirements	1343

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1344

A.6.1.17.1	Test Purpose and Environment	1344

A.6.1.17.2	Test Requirements	1345

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1346

A.6.1.18.1	Test Purpose and Environment	1346

A.6.1.18.2	Test Requirements	1347

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.19.1	Test Purpose and Environment	1348

A.6.1.19.2	Test Requirements	1349

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1350

A.6.1.20.1	Test Purpose and Environment	1350

A.6.1.20.2	Test Requirements	1351

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1352

A.6.1.21.1	Test Purpose and Environment	1352

A.6.1.21.2	Test Requirements	1353

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.22.1	Test Purpose and Environment	1354

A.6.1.22.2	Test Requirements	1355

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1356

A.6.1.23.1	Test Purpose and Environment	1356

A.6.1.23.2	Test Requirements	1358

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1359

A.6.1.24.1	Test Purpose and Environment	1359

A.6.1.24.2	Test Requirements	1361

A.6.2	Random Access	1362

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1362

A.6.2.1.1	Test Purpose and Environment	1362

A.6.2.1.2	Test Requirements	1363

A.6.2.1.2.1	Random Access Response Reception	1363

A.6.2.1.2.2	No Random Access Response Reception	1363

A.6.2.1.2.3	Receiving a NACK on msg3	1363

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1364

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1364

A.6.2.1.2.6	Contention Resolution Timer expiry	1364

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1364

A.6.2.2.1	Test Purpose and Environment	1364

A.6.2.2.2	Test Requirements	1365

A.6.2.2.2.1	Random Access Response Reception	1366

A.6.2.2.2.2	No Random Access Response Reception	1366

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1366

A.6.2.3.1	Test Purpose and Environment	1366

A.6.2.3.2	Test Requirements	1368

A.6.2.3.2.1	Random Access Response Reception	1368

A.6.2.3.2.2	No Random Access Response reception	1368

A.6.2.3.2.3	Receiving a NACK on msg3	1368

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1369

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1369

A.6.2.3.2.6	Contention Resolution Timer expiry	1369

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1369

A.6.2.4.1	Test Purpose and Environment	1369

A.6.2.4.2	Test Requirements	1371

A.6.2.4.2.1	Random Access Response Reception	1371

A.6.2.4.2.2	No Random Access Response Reception	1371

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.5.1	Test Purpose and Environment	1371

A.6.2.5.2	Test Requirements	1372

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1372

A.6.2.6.1	Test Purpose and Environment	1372

A.6.2.6.2	Test Requirements	1372

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1373

A.6.2.7.1	Test Purpose and Environment	1373

A.6.2.7.2	Test Requirements	1375

A.6.2.7.2.1	Random Access Response Reception	1375

A.6.2.7.2.2	No Random Access Response Reception	1375

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1375

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1376

A.6.2.8.1	Test Purpose and Environment	1376

A.6.2.8.2	Test Requirements	1378

A.6.2.8.2.1	Random Access Response Reception	1378

A.6.2.8.2.2	No Random Access Response Reception	1378

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1378

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1379

A.6.2.9.1	Test Purpose and Environment	1379

A.6.2.9.2	Test Requirements	1382

A.6.2.9.2.1	Random Access Response Reception	1382

A.6.2.9.2.2	No Random Access Response Reception	1383

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1383

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1384

A.6.2.10.1	Test Purpose and Environment	1384

A.6.2.10.2	Test Requirements	1386

A.6.2.10.2.1	Random Access Response Reception	1386

A.6.2.10.2.2	No Random Access Response Reception	1387

A.6.2.10.2.3	Receiving a NACK on msg3	1387

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1387

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1387

A.6.2.10.2.6	Contention Resolution Timer expiry	1387

A.6.2.10.2.7	PRACH Resource Selection	1388

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1388

A.6.2.11.1	Test Purpose and Environment	1388

A.6.2.11.2	Test Requirements	1390

A.6.2.11.2.1	Random Access Response Reception	1390

A.6.2.11.2.2	No Random Access Response Reception	1391

A.6.2.11.2.3	Receiving a NACK on msg3	1391

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1391

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1391

A.6.2.11.2.6	Contention Resolution Timer expiry	1391

A.6.2.11.2.7	PRACH Resource Selection	1392

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1392

A.6.2.12.1	Test Purpose and Environment	1392

A.6.2.12.2	Test Requirements	1394

A.6.2.12.2.1	Random Access Response Reception	1394

A.6.2.12.2.2	No Random Access Response Reception	1395

A.6.2.12.2.3	Receiving a NACK on msg3	1395

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.12.2.6	Contention Resolution Timer expiry	1395

A.6.2.12.2.7	PRACH Resource Selection	1396

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1396

A.6.2.13.1	Test Purpose and Environment	1396

A.6.2.13.2	Test Requirements	1398

A.6.2.13.2.1	Random Access Response Reception	1398

A.6.2.13.2.2	No Random Access Response Reception	1399

A.6.2.13.2.3	Receiving a NACK on msg3	1399

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.13.2.6	Contention Resolution Timer expiry	1399

A.6.2.13.2.7	PRACH Resource Selection	1400

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1400

A.6.2.14.1	Test Purpose and Environment	1400

A.6.2.14.2	Test Requirements	1402

A.6.2.14.2.1	Random Access Response Reception	1402

A.6.2.14.2.2	No Random Access Response Reception	1403

A.6.2.14.2.3	Receiving a NACK on msg3	1403

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1403

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1403

A.6.2.14.2.6	Contention Resolution Timer expiry	1403

A.6.2.14.2.7	PRACH Resource Selection	1404

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1404

A.6.2.15.1	Test Purpose and Environment	1404

A.6.2.15.2	Test Requirements	1406

A.6.2.15.2.1	Random Access Response Reception	1406

A.6.2.15.2.2	No Random Access Response Reception	1407

A.6.2.15.2.3	Receiving a NACK on msg3	1407

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1407

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1407

A.6.2.15.2.6	Contention Resolution Timer expiry	1407

A.6.2.15.2.7	PRACH Resource Selection	1408

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1408

A.6.2.16.1	Test Purpose and Environment	1408

A.6.2.16.2	Test Requirements	1411

A.6.2.16.2.1	Random Access Response Reception	1411

A.6.2.16.2.2	No Random Access Response Reception	1412

A.6.2.16.2.3	Receiving a NACK on msg3	1412

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1412

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1412

A.6.2.16.2.6	Contention Resolution Timer expiry	1412

A.6.2.16.2.7	NPRACH Resource Selection	1412

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1413

A.6.2.17.1	Test Purpose and Environment	1413

A.6.2.17.2	Test Requirements	1416

A.6.2.17.2.1	Random Access Response Reception	1416

A.6.2.17.2.2	No Random Access Response Reception	1417

A.6.2.17.2.3	Receiving a NACK on msg3	1417

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.17.2.6	Contention Resolution Timer expiry	1417

A.6.2.17.2.7	NPRACH Resource Selection	1417

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1418

A.6.2.18.1	Test Purpose and Environment	1418

A.6.2.18.2	Test Requirements	1421

A.6.2.18.2.1	Random Access Response Reception	1421

A.6.2.18.2.2	No Random Access Response Reception	1422

A.6.2.18.2.3	Receiving a NACK on msg3	1422

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1422

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1422

A.6.2.18.2.6	Contention Resolution Timer expiry	1422

A.6.2.18.2.7	NPRACH Resource Selection	1422

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1423

A.6.2.19.1	Test Purpose and Environment	1423

A.6.2.19.2	Test Requirements	1425

A.6.2.19.2.1	Random Access Response Reception	1425

A.6.2.19.2.2	No Random Access Response Reception	1426

A.6.2.19.2.3	Receiving a NACK on msg3	1426

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.19.2.6	Contention Resolution Timer expiry	1426

A.6.2.19.2.7	NPRACH Resource Selection	1426

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1427

A.6.2.20.1	Test Purpose and Environment	1427

A.6.2.20.2	Test Requirements	1429

A.6.2.20.2.1	Random Access Response Reception	1429

A.6.2.20.2.2	No Random Access Response Reception	1430

A.6.2.20.2.3	Receiving a NACK on msg3	1430

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.20.2.6	Contention Resolution Timer expiry	1430

A.6.2.20.2.7	NPRACH Resource Selection	1430

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.21.1	Test Purpose and Environment	1431

A.6.2.21.2	Test Requirements	1433

A.6.2.21.2.1	Random Access Response Reception	1433

A.6.2.21.2.2	No Random Access Response Reception	1434

A.6.2.21.2.3	Receiving a NACK on msg3	1434

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.21.2.6	Contention Resolution Timer expiry	1434

A.6.2.21.2.7	NPRACH Resource Selection	1434

A.6.3	RRC Connection Release with Redirection	1435

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1435

A.6.3.1.1	Test Purpose and Environment	1435

A.6.3.1.2	Test Requirements	1437

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1437

A.6.3.2.1	Test Purpose and Environment	1437

A.6.3.2.2	Test Requirements	1439

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1439

A.6.3.3.1	Test Purpose and Environment	1439

A.6.3.3.2	Test Requirements	1440

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1441

A.6.3.4.1	Test Purpose and Environment	1441

A.6.3.4.2	Test Requirements	1442

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1443

A.6.3.5.1	Test Purpose and Environment	1443

A.6.3.5.2	Test Requirements	1445

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1445

A.6.3.6.1	Test Purpose and Environment	1445

A. 6.3.6.2	Test Requirements	1448

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1448

A.6.3.7.1	Test Purpose and Environment	1448

A.6.3.7.2	Test Requirements	1451

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1451

A.6.3.8.1	Test Purpose and Environment	1451

A.6.3.8.2	Test Requirements	1454

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1454

A.6.3.9.1	Test Purpose and Environment	1454

A.6.3.9.2	Test Requirements	1456

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1456

A.6.3.10.1	Test Purpose and Environment	1456

A.6.3.10.2	Test Requirements	1458

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1458

A.6.3.11.1	Test Purpose and Environment	1458

A.6.3.11.2	Test Requirements	1460

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1460

A.6.3.12.1	Test Purpose and Environment	1460

A.6.3.12.2	Test Requirements	1463

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1463

A.6.3.13.1	Test Purpose and Environment	1463

A.6.3.13.2	Test Parameters	1463

A.6.3.13.3	Test Requirements	1467

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1525

A.7.2.13.1	Test Purpose and Environment	1525

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1527

A.7.2.14.1	Test Purpose and Environment	1527

A.7.2.14.2	Test Requirements	1529

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1529

A.7.2.15.1	Test Purpose and Environment	1529

A.7.2.15.2	Test Requirements	1531

A.7.3	Radio Link Monitoring	1531

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1539

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1539

A.7.3.3.1	Test Purpose and Environment	1539

A.7.3.3.2	Test Requirements	1543

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1543

A.7.3.4.1	Test Purpose and Environment	1543

A.7.3.4.2	Test Requirements	1547

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1547

A.7.3.5.1	Test Purpose and Environment	1547

A.7.3.5.2	Test Requirements	1550

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1551

A.7.3.6.1	Test Purpose and Environment	1551

A.7.3.6.2	Test Requirements	1554

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1554

A.7.3.7.1	Test Purpose and Environment	1554

A.7.3.7.2	Test Requirements	1557

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1557

A.7.3.8.1	Test Purpose and Environment	1557

A.7.3.8.2	Test Requirements	1560

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1560

A.7.3.9.2	Test Requirements	1563

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1563

A.7.3.10.1	Test Purpose and Environment	1563

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1567

A.7.3.11.1	Test Purpose and Environment	1567

A.7.3.11.2	Test Requirements	1572

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1572

A.7.3.12.1	Test Purpose and Environment	1572

A.7.3.12.2	Test Requirements	1577

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1577

A.7.3.13.1	Test Purpose and Environment	1577

A.7.3.13.2	Test Requirements	1580

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1580

A.7.3.14.1	Test Purpose and Environment	1580

A.7.3.14.2	Test Requirements	1583

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1583

A.7.3.15.1	Test Purpose and Environment	1583

A.7.3.15.2	Test Requirements	1587

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.16.1	Test Purpose and Environment	1587

A.7.3.16.2	Test Requirements	1592

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1592

A.7.3.17.1	Test Purpose and Environment	1592

A.7.3.17.2	Test Requirements	1596

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.18.1	Test Purpose and Environment	1596

A.7.3.18.2	Test Requirements	1600

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1600

A.7.3.19.1	Test Purpose and Environment	1600

A.7.3.19.2	Test Requirements	1605

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1605

A.7.3.20.1	Test Purpose and Environment	1605

A.7.3.20.2	Test Requirements	1610

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1610

A.7.3.21.1	Test Purpose and Environment	1610

A.7.3.21.2	Test Requirements	1615

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1615

A.7.3.22.1	Test Purpose and Environment	1615

A.7.3.22.2	Test Requirements	1620

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1620

A.7.3.23.1	Test Purpose and Environment	1620

A.7.3.23.2	Test Requirements	1621

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1621

A.7.3.24.1	Test Purpose and Environment	1621

A.7.3.24.2	Test Requirements	1622

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1622

A.7.3.25.1	Test Purpose and Environment	1622

A.7.3.25.2	Test Requirements	1623

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1623

A.7.3.26.1	Test Purpose and Environment	1623

A.7.3.26.2	Test Requirements	1626

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1626

A.7.3.27.1	Test Purpose and Environment	1626

A.7.3.27.2	Test Requirements	1629

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1629

A.7.3.28.1	Test Purpose and Environment	1629

A.7.3.28.2	Test Requirements	1632

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1632

A.7.3.29.1	Test Purpose and Environment	1632

A.7.3.29.2	Test Requirements	1635

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1635

A.7.3.30.1	Test Purpose and Environment	1635

A.7.3.30.2	Test Requirements	1638

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1638

A.7.3.31.1	Test Purpose and Environment	1638

A.7.3.31.2	Test Requirements	1641

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1641

A.7.3.32.1	Test Purpose and Environment	1641

A.7.3.32.2	Test Requirements	1644

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1644

A.7.3.33.1	Test Purpose and Environment	1644

A.7.3.33.2	Test Requirements	1647

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1647

A.7.3.34.1	Test Purpose and Environment	1647

A.7.3.34.2	Test Requirements	1650

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1650

A.7.3.35.1	Test Purpose and Environment	1650

A.7.3.35.2	Test Requirements	1653

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1653

A.7.3.36.1	Test Purpose and Environment	1653

A.7.3.36.2	Test Requirements	1656

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1656

A.7.3.37.1	Test Purpose and Environment	1656

A.7.3.37.2	Test Requirements	1659

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1659

A.7.3.38.1	Test Purpose and Environment	1659

A.7.3.38.2	Test Requirements	1662

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1662

A.7.3.39.1	Test Purpose and Environment	1662

A.7.3.39.2	Test Requirements	1665

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1666

A.7.3.40.1	Test Purpose and Environment	1666

A.7.3.40.2	Test Requirements	1668

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1669

A.7.3.41.1	Test Purpose and Environment	1669

A.7.3.41.2	Test Requirements	1672

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1672

A.7.3.42.1	Test Purpose and Environment	1672

A.7.3.42.2	Test Requirements	1675

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1675

A.7.3.43.1	Test Purpose and Environment	1675

A.7.3.43.2	Test Requirements	1678

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1678

A.7.3.44.1	Test Purpose and Environment	1678

A.7.3.44.2	Test Requirements	1681

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1682

A.7.3.45.1	Test Purpose and Environment	1682

A.7.3.45.2	Test Requirements	1684

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1685

A.7.3.46.1	Test Purpose and Environment	1685

A.7.3.46.2	Test Requirements	1688

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1688

A.7.3.47.1	Test Purpose and Environment	1688

A.7.3.47.2	Test Requirements	1691

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1691

A.7.3.48.1	Test Purpose and Environment	1691

A.7.3.48.2	Test Requirements	1694

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1694

A.7.3.49.1	Test Purpose and Environment	1694

A.7.3.49.2	Test Requirements	1697

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1697

A.7.3.50.1	Test Purpose and Environment	1697

A.7.3.50.2	Test Requirements	1700

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1700

A.7.3.51.1	Test Purpose and Environment	1700

A.7.3.51.2	Test Requirements	1703

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1703

A.7.3.52.1	Test Purpose and Environment	1703

A.7.3.52.2	Test Requirements	1706

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1706

A.7.3.53.1	Test Purpose and Environment	1706

A.7.3.53.2	Test Requirements	1709

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1709

A.7.3.54.1	Test Purpose and Environment	1709

A.7.3.54.2	Test Requirements	1712

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1712

A.7.3.55.1	Test Purpose and Environment	1712

A.7.3.55.2	Test Requirements	1715

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1715

A.7.3.56.1	Test Purpose and Environment	1715

A.7.3.56.2	Test Requirements	1718

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1718

A.7.3.57.1	Test Purpose and Environment	1718

A.7.3.57.2	Test Requirements	1721

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1721

A.7.3.58.1	Test Purpose and Environment	1721

A.7.3.58.2	Test Requirements	1724

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1724

A.7.3.59.1	Test Purpose and Environment	1724

A.7.3.59.2	Test Requirements	1727

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1727

A.7.3.60.1	Test Purpose and Environment	1727

A.7.3.60.2	Test Requirements	1731

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1731

A.7.3.61.1	Test Purpose and Environment	1731

A.7.3.61.2	Test Requirements	1735

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1735

A.7.3.62.1	Test Purpose and Environment	1735

A.7.3.62.2	Test Requirements	1739

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1739

A.7.3.63.1	Test Purpose and Environment	1739

A.7.3.63.2	Test Requirements	1743

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.64.1	Test Purpose and Environment	1743

A.7.3.64.2	Test Requirements	1746

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1747

A.7.3.65.1	Test Purpose and Environment	1747

A.7.3.65.2	Test Requirements	1750

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1751

A.7.3.66.1	Test Purpose and Environment	1751

A.7.3.66.2	Test Requirements	1753

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1754

A.7.3.67.1	Test Purpose and Environment	1754

A.7.3.67.2	Test Requirements	1757

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1758

A.7.3.68.1	Test Purpose and Environment	1758

A.7.3.68.2	Test Requirements	1760

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1760

A.7.3.69.1	Test Purpose and Environment	1760

A.7.3.69.2	Test Requirements	1762

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.70.1	Test Purpose and Environment	1762

A.7.3.70.2	Test Requirements	1764

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1764

A.7.3.71.1	Test Purpose and Environment	1764

A.7.3.71.2	Test Requirements	1767

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1767

A.7.3.72.1	Test Purpose and Environment	1767

A.7.3.72.2	Test Requirements	1769

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1769

A.7.3.73.1	Test Purpose and Environment	1769

A.7.3.73.2	Test Requirements	1771

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1771

A.7.3.74.1	Test Purpose and Environment	1771

A.7.3.74.2	Test Requirements	1774

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1774

A.7.3.75.1	Test Purpose and Environment	1774

A.7.3.75.2	Test Requirements	1776

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1777

A.7.3.76.1	Test Purpose and Environment	1777

A.7.3.76.2	Test Requirements	1780

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1780

A.7.3.77.1	Test Purpose and Environment	1780

A.7.3.77.2	Test Requirements	1783

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1783

A.7.3.78.1	Test Purpose and Environment	1783

A.7.3.78.2	Test Requirements	1786

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1786

A.7.3.79.1	Test Purpose and Environment	1786

A.7.3.79.2	Test Requirements	1789

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1789

A.7.3.80.1	Test Purpose and Environment	1789

A.7.3.80.2	Test Requirements	1792

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1792

A.7.3.81.1	Test Purpose and Environment	1792

A.7.3.81.2	Test Requirements	1795

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.82.1	Test Purpose and Environment	1795

A.7.3.82.2	Test Requirements	1798

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1798

A.7.3.83.1	Test Purpose and Environment	1798

A.7.3.83.2	Test Requirements	1799

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1800

A.7.3.84.1	Test Purpose and Environment	1800

A.7.3.84.2	Test Requirements	1802

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.85.1	Test Purpose and Environment	1802

A.7.3.85.2	Test Requirements	1803

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.86.1	Test Purpose and Environment	1804

A.7.3.86.2	Test Requirements	1806

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.87.1	Test Purpose and Environment	1806

A.7.3.87.2	Test Requirements	1808

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1809

A.7.3.88.1	Test Purpose and Environment	1809

A.7.3.88.2	Test Requirements	1813

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1813

A.7.3.89.1	Test Purpose and Environment	1813

A.7.3.89.2	Test Requirements	1817

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1817

A.7.3.90.1	Test Purpose and Environment	1817

A.7.3.90.2	Test Requirements	1821

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1821

A.7.3.91.1	Test Purpose and Environment	1821

A.7.3.91.2	Test Requirements	1825

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1825

A.7.3.92.1	Test Purpose and Environment	1825

A.7.3.92.2	Test Requirements	1828

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1829

A.7.3.93.1	Test Purpose and Environment	1829

A.7.3.93.2	Test Requirements	1832

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1833

A.7.3.94.1	Test Purpose and Environment	1833

A.7.3.94.2	Test Requirements	1835

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1836

A.7.3.95.1	Test Purpose and Environment	1836

A.7.3.95.2	Test Requirements	1839

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1840

A.7.3.96.1	Test Purpose and Environment	1840

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1841

A.7.3.97.1	Test Purpose and Environment	1841

A.7.3.97.2	Test Requirements	1844

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1844

A.7.3.98.1	Test Purpose and Environment	1844

A.7.3.98.2	Test Requirements	1847

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1847

A.7.3.99.1	Test Purpose and Environment	1847

A.7.3.99.2	Test Requirements	1850

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1850

A.7.3.100.1	Test Purpose and Environment	1850

A.7.3.100.2	Test Requirements	1852

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1852

A.7.3.101.1	Test Purpose and Environment	1852

A.7.3.101.2	Test Requirements	1854

A.7.4	Interruption for Dual Connectivity	1854

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1854

A.7.4.1.1	Test Purpose and Environment	1854

A.7.4.1.2	Test Requirements	1857

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1857

A.7.4.2.1	Test Purpose and Environment	1857

A.7.4.2.2	Test Requirements	1859

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1859

A.7.4.3.1	Test Purpose and Environment	1859

A.7.4.3.2	Test Requirements	1861

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.4.1	Test Purpose and Environment	1861

A.7.4.4.2	Test Requirements	1864

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1864

A.7.4.5.1	Test Purpose and Environment	1864

A.7.4.5.2	Test Requirements	1866

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1866

A.7.4.6.1	Test Purpose and Environment	1866

A.7.4.6.2	Test Requirements	1868

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1868

A.7.4.7.1	Test Purpose and Environment	1868

A.7.4.7.2	Test Requirements	1872

A.7.5	Proximity-based Services	1872

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1872

A.7.5.1.1	Test Purpose and Environment	1872

A.7.5.1.2	Test Requirements	1873

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1874

A.7.5.2.1	Test Purpose and Environment	1874

A.7.5.1.2	Test Requirements	1874

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1875

A.7.5.3.1	Test Purpose and Environment	1875

A.7.5.3.2	Test Requirements	1876

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1877

A.7.5.4.1	Test Purpose and Environment	1877

A.7.5.4.2	Test Requirements	1878

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1879

A.7.5.5.1	Test Purpose and Environment	1879

A.7.5.5.2	Test Requirements	1881

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1882

A.7.5.6.1	Test Purpose and Environment	1882

A.7.5.6.2	Test Requirements	1883

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1884

A.7.5.7.1	Test Purpose and Environment	1884

A.7.5.7.2	Test Requirements	1886

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1886

A.7.5.8.1	Test Purpose and Environment	1886

A.7.5.8.2	Test Requirements	1888

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1889

A.7.5.9.1	Test Purpose and Environment	1889

A.7.5.9.2	Test Requirements	1891

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1892

A.7.5.10.1	Test Purpose and Environment	1892

A.7.5.10.2	Test Requirements	1894

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1895

A.7.5.11.1	Test Purpose and Environment	1895

A.7.5.11.2	Test Requirements	1897

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1897

A.7.5.12.1	Test Purpose and Environment	1897

A.7.5.12.2	Test Requirements	1901

A.7.6	Interruption for carrier aggregation	1902

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1902

A.7.6.1.1	Test Purpose and Environment	1902

A.7.6.1.2	Test Requirements	1905

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1905

A.7.6.2.1	Test Purpose and Environment	1905

A.7.6.2.2	Test Requirements	1908

A.8	UE Measurements Procedures	1908

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1908

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1908

A.8.1.1.1	Test Purpose and Environment	1908

A.8.1.1.2	Test Requirements	1909

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1910

A.8.1.2.1	Test Purpose and Environment	1910

A.8.1.2.2	Test Requirements	1911

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1912

A.8.1.3.1	Test Purpose and Environment	1912

A.8.1.3.2	Test Requirements	1914

A.8.1.4	Void	1914

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1914

A.8.1.5.1	Test Purpose and Environment	1914

A.8.1.5.2	Test Requirements	1916

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1916

A.8.1.6.1	Test Purpose and Environment	1916

A.8.1.6.2	Test Requirements	1918

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1918

A.8.1.7.1	Test Purpose and Environment	1918

A.8.1.7.2	Test Requirements	1920

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1921

A.8.1.8.1	Test Purpose and Environment	1921

A.8.1.8.2	Test Requirements	1923

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1924

A.8.1.9.1	Test Purpose and Environment	1924

A.8.1.9.2	Test Requirements	1924

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1924

A.8.1.10.1	Test Purpose and Environment	1924

A.8.1.10.2	Test Requirements	1925

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1925

A.8.1.11.1	Test Purpose and Environment	1925

A.8.1.11.2	Test Requirements	1927

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1928

A.8.1.12.1	Test Purpose and Environment	1928

A.8.1.12.2	Test Requirements	1929

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1930

A.8.1.13.1	Test Purpose and Environment	1930

A.8.1.13.2	Test Requirements	1932

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1932

A.8.1.14.1	Test Purpose and Environment	1932

A.8.1.14.2	Test Requirements	1934

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1935

A.8.1.15.1	Test Purpose and Environment	1935

A.8.1.15.2	Test Requirements	1936

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1937

A.8.1.16.1	Test Purpose and Environment	1937

A.8.1.16.2	Test Requirements	1939

A.8.1.17	Void	1939

A.8.1.18	Void	1939

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1939

A.8.1.19.1	Test Purpose and Environment	1939

A.8.1.19.2	Test Requirements	1941

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1942

A.8.1.20.1	Test Purpose and Environment	1942

A.8.1.20.2	Test Requirements	1944

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.21.1	Test Purpose and Environment	1944

A.8.1.21.2	Test Requirements	1946

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.22.1	Test Purpose and Environment	1947

A.8.1.22.2	Test Requirements	1949

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1949

A.8.1.23.1	Test Purpose and Environment	1949

A.8.1.23.2	Test Requirements	1950

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1951

A.8.1.24.1	Test Purpose and Environment	1951

A.8.1.24.2	Test Requirements	1953

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.25.1	Test Purpose and Environment	1953

A.8.1.25.2	Test Requirements	1955

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.26.1	Test Purpose and Environment	1955

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1958

A.8.1.27.1	Test Purpose and Environment	1958

A.8.1.27.2	Test Requirements	1960

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1960

A.8.1.28.1	Test Purpose and Environment	1960

A.8.1.28.2	Test Requirements	1962

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.29.1	Test Purpose and Environment	1962

A.8.1.29.2	Test Requirements	1964

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.30.1	Test Purpose and Environment	1965

A.8.1.30.2	Test Requirements	1967

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1967

A.8.1.31.1	Test Purpose and Environment	1967

A.8.1.31.2	Test Requirements	1969

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1970

A.8.1.32.1	Test Purpose and Environment	1970

A.8.1.32.2	Test Requirements	1971

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.33.1	Test Purpose and Environment	1972

A.8.1.33.2	Test Requirements	1973

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.34.1	Test Purpose and Environment	1974

A.8.1.34.2	Test Requirements	1975

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.35.1	Test Purpose and Environment	1976

A.8.1.35.2	Test Requirements	1977

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1978

A.8.1.36.1	Test Purpose and Environment	1978

A.8.1.36.2	Test Requirements	1979

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1980

A.8.1.37.1	Test Purpose and Environment	1980

A.8.1.37.2	Test Requirements	1982

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.38.1	Test Purpose and Environment	1982

A.8.1.38.2	Test Requirements	1983

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.39.1	Test Purpose and Environment	1984

A.8.1.39.2	Test Requirements	1986

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1986

A.8.1.40.1	Test Purpose and Environment	1986

A.8.1.40.2	Test Requirements	1988

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1988

A.8.1.41.1	Test Purpose and Environment	1988

A.8.1.41.2	Test Requirement	1990

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1990

A.8.1.42.1	Test Purpose and Environment	1990

A.8.1.42.2	Test Requirement	1991

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1991

A.8.1.43.1	Test Purpose and Environment	1991

A.8.1.43.2	Test Requirements	1994

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	1994

A.8.1.44.1	Test Purpose and Environment	1994

A.8.1.44.2	Test Requirements	1997

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	1998

A.8.1.45.1	Test Purpose and Environment	1998

A.8.1.45.2	Test Requirements	2000

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2001

A.8.1.46.1	Test Purpose and Environment	2001

A.8.1.46.2	Test Requirements	2002

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2003

A.8.1.47.1	Test Purpose and Environment	2003

A.8.1.47.2	Test Requirements	2005

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2006

A.8.1.48.1	Test Purpose and Environment	2006

A.8.1.48.2	Test Requirements	2008

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2009

A.8.1.49.1	Test Purpose and Environment	2009

A.8.1.49.2	Test Requirements	2010

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2011

A.8.1.50.1	Test Purpose and Environment	2011

A.8.1.50.2	Test Requirements	2013

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2014

A.8.1.51.1	Test Purpose and Environment	2014

A.8.1.51.2	Test Requirements	2016

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2017

A.8.1.52.1	Test Purpose and Environment	2017

A.8.1.52.2	Test Requirements	2018

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2019

A.8.1.53.1	Test Purpose and Environment	2019

A.8.1.53.2	Test Requirements	2021

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2022

A.8.1.54.1	Test Purpose and Environment	2022

A.8.1.54.2	Test Requirements	2024

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2025

A.8.1.55.1	Test Purpose and Environment	2025

A.8.1.55.2	Test Requirements	2026

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2027

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2027

A.8.2.1.1	Test Purpose and Environment	2027

A.8.2.1.2	Test Requirements	2028

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2028

A.8.2.2.1	Test Purpose and Environment	2028

A.8.2.2.2	Test Requirements	2031

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2031

A.8.2.3.1	Test Purpose and Environment	2031

A.8.2.3.2	Test Requirements	2033

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2034

A.8.2.4.1	Test Purpose and Environment	2034

A.8.2.4.2	Test Requirements	2036

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2036

A.8.2.5.1	Test Purpose and Environment	2036

A.8.2.5.2	Test Requirements	2038

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2039

A.8.2.6.1	Test Purpose and Environment	2039

A.8.2.6.2	Test Requirements	2042

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2043

A.8.2.7.1	Test Purpose and Environment	2043

A.8.2.7.2	Test Requirements	2044

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2045

A.8.2.8.1	Test Purpose and Environment	2045

A.8.2.8.2	Test Requirements	2047

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2047

A.8.2.9.1	Test Purpose and Environment	2047

A.8.2.9.2	Test Requirements	2049

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2050

A.8.2.10.1	Test Purpose and Environment	2050

A.8.2.10.2	Test Requirements	2052

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2052

A.8.2.11.1	Test Purpose and Environment	2052

A.8.2.11.2	Test Requirements	2054

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2054

A.8.2.12.1	Test Purpose and Environment	2054

A.8.2.12.2	Test Requirements	2056

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2057

A.8.2.13.1	Test Purpose and Environment	2057

A.8.2.13.2	Test Requirements	2059

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2059

A.8.2.14.1	Test Purpose and Environment	2059

A.8.2.14.2	Test Requirement	2061

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2061

A.8.2.15.1	Test Purpose and Environment	2061

A.8.2.15.2	Test Requirements	2063

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2063

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2063

A.8.3.1.1	Test Purpose and Environment	2063

A.8.3.1.2	Test Requirements	2065

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2065

A.8.3.2.1	Test Purpose and Environment	2065

A.8.3.2.2	Test Requirements	2068

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2068

A.8.3.3.1	Test Purpose and Environment	2068

A.8.3.3.2	Test Requirements	2071

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2071

A.8.3.4.1	Test Purpose and Environment	2071

A.8.3.4.2	Test Requirements	2073

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2073

A.8.3.5.2	Test Requirements	2075

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2075

A.8.3.6.1	Test Purpose and Environment	2075

A.8.3.6.2	Test Requirements	2077

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2078

A.8.3.7.1	Test Purpose and Environment	2078

A.8.3.7.2	Test Requirements	2080

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2081

A.8.3.8.1	Test Purpose and Environment	2081

A.8.3.8.2	Test Requirements	2084

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2084

A.8.3.9.1	Test Purpose and Environment	2084

A.8.3.9.2	Test Requirements	2088

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2089

A.8.3.10.1	Test Purpose and Environment	2089

A.8.3.10.2	Test Requirements	2090

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2090

A.8.3.11.1	Test Purpose and Environment	2090

A.8.3.11.2	Test Requirement	2092

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2093

A.8.3.12.1	Test Purpose and Environment	2093

A.8.3.12.2	Test Requirement	2094

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2095

A.8.3.13.1	Test Purpose and Environment	2095

A.8.3.13.2	Test Requirement	2096

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2097

A.8.3.14.1	Test Purpose and Environment	2097

A.8.3.14.2	Test Requirement	2098

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2099

A.8.3.15.1	Test Purpose and Environment	2099

A.8.3.15.2	Test Requirement	2100

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2101

A.8.3.16.1	Test Purpose and Environment	2101

A.8.3.16.2	Test Requirement	2103

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2103

A.8.3.17.1	Test Purpose and Environment	2103

A.8.3.17.2	Test Requirement	2106

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2106

A.8.3.18.1	Test Purpose and Environment	2106

A.8.3.18.2	Test Requirement	2109

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2109

A.8.3.19.1	Test Purpose and Environment	2109

A.8.3.19.2	Test Requirement	2112

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2112

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2112

A.8.4.1.1	Test Purpose and Environment	2112

A.8.4.1.2	Test Requirements	2114

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2114

A.8.4.2.1	Test Purpose and Environment	2114

A.8.4.2.2	Test Requirements	2117

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2117

A.8.4.3.1	Test Purpose and Environment	2117

A.8.4.3.2	Test Requirements	2119

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2120

A.8.4.4.1	Test Purpose and Environment	2120

A.8.4.4.2	Test Requirements	2122

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2123

A.8.4.5.1	Test Purpose and Environment	2123

A.8.4.5.2	Test Requirements	2125

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2125

A.8.4.6.1	Test Purpose and Environment	2125

A.8.4.6.2	Test Requirements	2126

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2126

A.8.4.7.1	Test Purpose and Environment	2126

A.8.4.7.2	Test Requirements	2130

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2130

A.8.4.8.1	Test Purpose and Environment	2130

A.8.4.8.2	Test Requirements	2133

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2133

A.8.4.9.1	Test Purpose and Environment	2133

A.8.4.9.2	Test Requirements	2138

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2138

A.8.4.10.1	Test Purpose and Environment	2138

A.8.4.10.2	Test Requirements	2140

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2140

A.8.4.11.1	Test Purpose and Environment	2140

A.8.4.11.2	Test Requirement	2142

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2142

A.8.4.12.1	Test Purpose and Environment	2142

A.8.4.12.2	Test Requirement	2144

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2145

A.8.4.13.1	Test Purpose and Environment	2145

A.8.4.13.2	Test Requirement	2146

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2147

A.8.4.14.1	Test Purpose and Environment	2147

A.8.4.14.2	Test Requirement	2149

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2149

A.8.4.15.1	Test Purpose and Environment	2149

A.8.4.15.2	Test Requirement	2152

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2152

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2152

A.8.5.1.1	Test Purpose and Environment	2152

A.8.5.1.2	Test Requirements	2154

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2154

A.8.5.2.1	Test Purpose and Environment	2154

A.8.5.2.2	Test Requirements	2156

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2156

A.8.5.3.1	Test Purpose and Environment	2156

A.8.5.3.2	Test Requirements	2159

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2159

A.8.5.4.1	Test Purpose and Environment	2159

A.8.5.4.2	Test Requirements	2161

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2161

A.8.5.5.1	Test Purpose and Environment	2161

A.8.5.5.2	Test Requirements	2164

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2164

A.8.5.6.1	Test Purpose and Environment	2164

A.8.5.6.2	Test Requirements	2165

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2166

A.8.5.7.1	Test Purpose and Environment	2166

A.8.5.7.2	Test Requirements	2166

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2166

A.8.5.8.1	Test Purpose and Environment	2166

A.8.5.8.2	Test Requirements	2169

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2169

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2169

A.8.6.1.1	Test Purpose and Environment	2169

A.8.6.1.2	Test Requirements	2171

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2171

A.8.6.2.1	Test Purpose and Environment	2171

A.8.6.2.2	Test Requirements	2174

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2174

A.8.6.3.1	Test Purpose and Environment	2174

A.8.6.3.2	Test Requirements	2177

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2177

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2177

A.8.7.1.1	Test Purpose and Environment	2177

A.8.7.1.1.1	Void	2177

A.8.7.1.1.2	1.28 Mcps TDD option	2177

A.8.7.1.1.3	Void	2179

A.8.7.1.2	Test Requirements	2179

A.8.7.1.2.1	Void	2179

A.8.7.1.2.2	1.28 Mcps TDD option	2179

A.8.7.1.2.3	Void	2179

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2179

A.8.7.2.1	Test Purpose and Environment	2179

A.8.7.2.2	Test Requirements	2182

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2183

A.8.7.3.1	Test Purpose and Environment	2183

A.8.7.3.2	Test Parameters	2183

A.8.7.3.3	Test Requirements	2184

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2185

A.8.7.4.1	Test Purpose and Environment	2185

A.8.7.4.2	Test Requirements	2187

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2187

A.8.7.5.1	Test Purpose and Environment	2187

A.8.7.5.2	Test Requirements	2189

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2189

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2189

A.8.7A.1.1	Test Purpose and Environment	2189

A.8.7A.1.2	Test Requirements	2192

A.8.8	E-UTRAN FDD – GSM Measurements	2192

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2192

A.8.8.1.1	Test Purpose and Environment	2192

A.8.8.1.2	Test Requirements	2194

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2194

A.8.8.2.1	Test Purpose and Environment	2194

A.8.8.2.2	Test Requirements	2196

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2197

A.8.8.3.1	Test Purpose and Environment	2197

A.8.8.3.2	Test Requirements	2198

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2199

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2199

A.8.9.1.1	Test Purpose and Environment	2199

A.8.9.1.2	Test Requirements	2200

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2201

A.8.9.2.1	Test Purpose and Environment	2201

A.8.9.2.2	Test Requirements	2203

A.8.10	E-UTRAN TDD – GSM Measurements	2203

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2203

A.8.10.1.1	Test Purpose and Environment	2203

A.8.10.1.2	Test Requirements	2204

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2205

A.8.10.2.1	Test Purpose and Environment	2205

A.8.10.2.2	Test Requirements	2207

A.8.11	Monitoring of Multiple Layers	2207

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2207

A.8.11.1.1	Test Purpose and Environment	2207

A.8.11.1.2	Test Requirements	2209

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2210

A.8.11.2.1	Test Purpose and Environment	2210

A.8.11.2.2	Test Requirements	2211

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2212

A.8.11.3.1	Test Purpose and Environment	2212

A.8.11.3.2	Test Requirements	2214

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2214

A.8.11.4.1	Test Purpose and Environment	2214

A.8.11.4.2	Test Requirements	2217

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2217

A.8.11.5.1	Test Purpose and Environment	2217

A.8.11.5.2	Test Requirements	2219

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2220

A.8.11.6.1	Test Purpose and Environment	2220

A.8.11.6.2	Test Requirements	2222

A.8.12	RSTD Intra-frequency Measurements	2223

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2223

A.8.12.1.1	Test Purpose and Environment	2223

A.8.12.1.2	Test Requirements	2227

A.8.12.1.2A	Test Requirements for UE Category 1bis	2227

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2227

A.8.12.2.1	Test Purpose and Environment	2227

A.8.12.2.2	Test Requirements	2232

A.8.12.2.2A	Test Requirements for UE Category 1bis	2232

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2232

A.8.12.3.1	Test Purpose and Environment	2232

A.8.12.3.2	Test Requirements	2238

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2238

A.8.12.4.1	Test Purpose and Environment	2238

A.8.12.4.2	Test Requirements	2243

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2243

A.8.12.5.1	Test Purpose and Environment	2243

A.8.12.5.2	Test Requirements	2248

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2248

A.8.12.6.1	Test Purpose and Environment	2248

A.8.12.6.2	Test Requirements	2253

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2253

A.8.12.7.1	Test Purpose and Environment	2253

A.8.12.7.2	Test Requirements	2258

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2258

A.8.12.8.1	Test Purpose and Environment	2258

A.8.12.8.2	Test Requirements	2263

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2263

A.8.12.9.1	Test Purpose and Environment	2263

A.8.12.9.2	Test Requirements	2268

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2268

A.8.12.10.1	Test Purpose and Environment	2268

A.8.12.10.2	Test Requirements	2273

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2273

A.8.12.11.1	Test Purpose and Environment	2273

A.8.12.11.2	Test Requirements	2278

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2278

A.8.12.12.1	Test Purpose and Environment	2278

A.8.12.12.2	Test Requirements	2283

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2283

A.8.12.13.1	Test Purpose and Environment	2283

A.8.12.13.2	Test Requirements	2288

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2288

A.8.12.14.1	Test Purpose and Environment	2288

A.8.12.14.2	Test Requirements	2293

A.8.13	RSTD Inter-frequency Measurements	2293

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2293

A.8.13.1.1	Test Purpose and Environment	2293

A.8.13.1.2	Test Requirements	2298

A.8.13.1.2A	Test Requirements for UE Category 1bis	2298

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2298

A.8.13.2.1	Test Purpose and Environment	2298

A.8.13.2.2	Test Requirements	2304

A.8.13.2.2A	Test Requirements for UE Category 1bis	2304

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2304

A.8.13.3.1	Test Purpose and Environment	2304

A.8.13.3.2	Test Requirements	2311

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2311

A.8.13.4.1	Test Purpose and Environment	2311

A.8.13.4.2	Test Requirements	2317

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2317

A.8.13.5.1	Test Purpose and Environment	2317

A.8.13.5.2	Test Requirements	2323

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2323

A.8.13.6.1	Test Purpose and Environment	2323

A.8.13.6.2	Test Requirements	2329

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2329

A.8.13.7.1	Test Purpose and Environment	2329

A.8.13.7.2	Test Requirements	2335

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2335

A.8.13.8.1	Test Purpose and Environment	2335

A.8.13.8.2	Test Requirements	2341

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2341

A.8.13.9.1	Test Purpose and Environment	2341

A.8.13.9.2	Test Requirements	2347

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2347

A.8.13.10.1	Test Purpose and Environment	2347

A.8.13.10.2	Test Requirements	2353

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2353

A.8.13.11.1	Test Purpose and Environment	2353

A.8.13.11.2	Test Requirements	2359

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2359

A.8.13.12.1	Test Purpose and Environment	2359

A.8.13.12.2	Test Requirements	2365

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2365

A.8.13.13.1	Test Purpose and Environment	2365

A.8.13.13.2	Test Requirements	2371

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2371

A.8.13.14.1	Test Purpose and Environment	2371

A.8.13.14.2	Test Requirements	2377

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2377

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2377

A.8.14.1.1	Test Purpose and Environment	2377

A.8.14.1.2	Test Requirements	2379

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2379

A.8.14.2.1	Test Purpose and Environment	2379

A.8.14.2.2	Test Requirements	2382

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2382

A.8.14.3.1	Test Purpose and Environment	2382

A.8.14.3.2	Test Requirements	2384

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2385

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2385

A.8.15.1.1	Test Purpose and Environment	2385

A.8.15.1.2	Test Requirements	2386

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2386

A.8.15.2.1	Test Purpose and Environment	2386

A.8.15.2.2	Test Requirements	2389

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2389

A.8.15.3.1	Test Purpose and Environment	2389

A.8.15.3.2	Test Requirements	2391

A.8.16	E-UTRAN Carrier Aggregation Measurements	2392

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2392

A.8.16.1.1	Test Purpose and Environment	2392

A.8.16.1.2	Test Requirements	2394

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2394

A.8.16.2.1	Test Purpose and Environment	2394

A.8.16.2.2	Test Requirements	2396

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2396

A.8.16.3.1	Test Purpose and Environment	2396

A.8.16.3.2	Test Requirements	2398

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2399

A.8.16.3A.1	Test Purpose and Environment	2399

A.8.16.3A.2	Test Requirements	2400

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2401

A.8.16.4.1	Test Purpose and Environment	2401

A.8.16.4.2	Test Requirements	2402

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2403

A.8.16.4A.1	Test Purpose and Environment	2403

A.8.16.4A.2	Test Requirements	2405

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.5.1	Test Purpose and Environment	2406

A.8.16.5.2	Test Requirements	2406

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.6.1	Test Purpose and Environment	2406

A.8.16.6.2	Test Requirements	2407

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2407

A.8.16.7.1	 Test Purpose and Environment	2407

A.8.16.7.2	 Test Requirements	2408

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2408

A.8.16.8.1	 Test Purpose and Environment	2408

A.8.16.8.2	 Test Requirements	2409

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2409

A.8.16.9.1	Test Purpose and Environment	2409

A.8.16.9.2	Test Requirements	2410

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2410

A.8.16.10.1	Test Purpose and Environment	2410

A.8.16.10.2	Test Requirements	2411

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2411

A.8.16.11.1	 Test Purpose and Environment	2411

A.8.16.11.2	 Test Requirements	2412

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2412

A.8.16.12.1	 Test Purpose and Environment	2412

A.8.16.12.2	 Test Requirements	2412

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2412

A.8.16.13.1	Test Purpose and Environment	2412

A.8.16.13.2	Test Requirements	2413

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2413

A.8.16.14.1	Test Purpose and Environment	2413

A.8.16.14.2	Test Requirements	2414

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2414

A.8.16.15.1	 Test Purpose and Environment	2414

A.8.16.7.2	 Test Requirements	2414

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2414

A.8.16.16.1	 Test Purpose and Environment	2414

A.8.16.16.2	 Test Requirements	2415

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2415

A.8.16.17.1	Test Purpose and Environment	2415

A.8.16.17.2	Test Requirements	2417

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2418

A.8.16.17A.1	Test Purpose and Environment	2418

A.8.16.17A.2	Test Requirements	2418

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2418

A.8.16.17B.1	Test Purpose and Environment	2418

A.8.16.17B.2	Test Requirements	2419

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2419

A.8.16.17C.1	Test Purpose and Environment	2419

A.8.16.17C.2	Test Requirements	2419

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2419

A.8.16.18.1	Test Purpose and Environment	2419

A.8.16.18.2	Test Requirements	2421

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2422

A.8.16.18A.1	Test Purpose and Environment	2422

A.8.16.18A.2	Test Requirements	2422

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2422

A.8.16.18B.1	Test Purpose and Environment	2422

A.8.16.18B.2	Test Requirements	2423

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2423

A.8.16.18C.1	Test Purpose and Environment	2423

A.8.16.18C.2	Test Requirements	2423

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2423

A.8.16.18D.1	Test Purpose and Environment	2423

A.8.16.18D.2	Test Requirements	2424

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2424

A.8.16.19.1	Test Purpose and Environment	2424

A.8.16.19.2	Test Requirements	2426

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2426

A.8.16.19A.1	Test Purpose and Environment	2426

A.8.16.19A.2	Test Requirements	2427

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2427

A.8.16.19B.1	Test Purpose and Environment	2427

A.8.16.19B.2	Test Requirements	2427

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2427

A.8.16.19C.1	Test Purpose and Environment	2427

A.8.16.19C.2	Test Requirements	2428

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2428

A.8.16.20.1	Test Purpose and Environment	2428

A.8.16.20.2	Test Requirements	2430

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2431

A.8.16.20A.1	Test Purpose and Environment	2431

A.8.16.20A.2	Test Requirements	2431

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2431

A.8.16.20B.1	Test Purpose and Environment	2431

A.8.16.20B.2	Test Requirements	2432

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2432

A.8.16.20C.1	Test Purpose and Environment	2432

A.8.16.20C.2	Test Requirements	2432

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2432

A.8.16.20D.1	Test Purpose and Environment	2432

A.8.16.20D.2	Test Requirements	2433

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2433

A.8.16.21.1	Test Purpose and Environment	2433

A.8.16.21.2	Test Requirements	2435

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2435

A.8.16.22.1	Test Purpose and Environment	2435

A.8.16.22.2	Test Requirements	2437

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2437

A.8.16.23.1	Test Purpose and Environment	2437

A.8.16.23.2	Test Requirements	2440

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2440

A.8.16.24.1	Test Purpose and Environment	2440

A.8.16.24.2	Test Requirements	2443

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2443

A.8.16.25.1	Test Purpose and Environment	2443

A.8.16.25.2	Test Requirements	2446

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2446

A.8.16.26.1	Test Purpose and Environment	2446

A.8.16.26.2	Test Requirements	2449

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2449

A.8.16.27.1	Test Purpose and Environment	2449

A.8.16.27.2	Test Requirements	2454

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2454

A.8.16.28.1	Test Purpose and Environment	2454

A.8.16.28.2	Test Requirements	2459

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2459

A.8.16.29.1	Test Purpose and Environment	2459

A.8.16.29.2	Test Requirements	2464

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2464

A.8.16.30.1	Test Purpose and Environment	2464

A.8.16.30.2	Test Requirements	2469

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2469

A.8.16.31.1	Test Purpose and Environment	2469

A.8.16.31.2	Test Requirements	2474

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2474

A.8.16.32.1	Test Purpose and Environment	2474

A.8.16.32.2	Test Requirements	2479

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2479

A.8.16.33.1	Test Purpose and Environment	2479

A.8.16.33.2	Test Requirements	2484

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2484

A.8.16.34.1	Test Purpose and Environment	2484

A.8.16.34.2	Test Requirements	2489

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2489

A.8.16.35.1	Test Purpose and Environment	2489

A.8.16.35.2	Test Requirements	2491

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2492

A.8.16.36.1	Test Purpose and Environment	2492

A.8.16.36.2	Test Requirements	2494

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2495

A.8.16.37.1	Test Purpose and Environment	2495

A.8.16.37.2	Test Requirements	2497

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2498

A.8.16.38.1	Test Purpose and Environment	2498

A.8.16.38.2	Test Requirements	2500

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2501

A.8.16.39.1	Test Purpose and Environment	2501

A.8.16.39.2	Test Requirements	2504

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2504

A.8.16.40.1	Test Purpose and Environment	2504

A.8.16.40.2	Test Requirements	2507

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2507

A.8.16.41.1	Test Purpose and Environment	2507

A.8.16.41.2	Test Requirements	2509

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2510

A.8.16.42.1	Test Purpose and Environment	2510

A.8.16.42.2	Test Requirements	2512

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2513

A.8.16.43.1	Test Purpose and Environment	2513

A.8.16.43.2	Test Requirements	2516

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2516

A.8.16.44.1	Test Purpose and Environment	2516

A.8.16.44.2	Test Requirements	2519

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2519

A.8.16.45.1	Test Purpose and Environment	2519

A.8.16.45.2	Test Requirements	2522

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2522

A.8.16.46.1	Test Purpose and Environment	2522

A.8.16.46.2	Test Requirements	2524

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2525

A.8.16.47.1	Test Purpose and Environment	2525

A.8.16.47.2	Test Requirements	2527

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2528

A.8.16.48.1	Test Purpose and Environment	2528

A.8.16.48.2	Test Requirements	2530

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2531

A.8.16.49.1	Test Purpose and Environment	2531

A.8.16.49.2	Test Requirements	2534

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2534

A.8.16.50.1	Test Purpose and Environment	2534

A.8.16.50.2	Test Requirements	2536

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2537

A.8.16.51.1	Test Purpose and Environment	2537

A.8.16.51.2	Test Requirements	2542

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2542

A.8.16.52.1	Test Purpose and Environment	2542

A.8.16.52.2	Test Requirements	2547

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2547

A.8.16.53.1	Test Purpose and Environment	2547

A.8.16.53.2	Test Requirements	2551

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2551

A.8.16.54.1	Test Purpose and Environment	2551

A.8.16.54.2	Test Requirements	2555

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2555

A.8.16.55.1	Test Purpose and Environment	2555

A.8.16.55.2	Test Requirements	2561

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2561

A.8.16.56.1	Test Purpose and Environment	2561

A.8.16.56.2	Test Requirements	2567

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2567

A.8.16.57.1	Test Purpose and Environment	2567

A.8.16.57.2	Test Requirements	2569

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2570

A.8.16.58.1	Test Purpose and Environment	2570

A.8.16.58.2	Test Requirements	2572

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2573

A.8.16.59.1	Test Purpose and Environment	2573

A.8.16.59.2	Test Requirements	2577

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2578

A.8.16.60.1	Test Purpose and Environment	2578

A.8.16.60.2	Test Requirements	2581

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2582

A.8.16.61.1	Test Purpose and Environment	2582

A.8.16.61.2	Test Requirements	2585

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2585

A.8.16.62.1	Test Purpose and Environment	2585

A.8.16.62.2	Test Requirements	2587

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.63.1	Test Purpose and Environment	2588

A.8.16.63.2	Test Requirements	2591

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2592

A.8.16.64.1	Test Purpose and Environment	2592

A.8.16.64.2	Test Requirements	2596

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2596

A.8.16.65.1	Test Purpose and Environment	2596

A.8.16.65.2	Test Requirements	2601

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2601

A.8.16.66.1	Test Purpose and Environment	2601

A.8.16.66.2	Test Requirements	2606

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2606

A.8.16.67.1	Test Purpose and Environment	2606

A.8.16.67.2	Test Requirements	2611

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2611

A.8.16.68.1	Test Purpose and Environment	2611

A.8.16.68.2	Test Requirements	2616

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2616

A.8.16.69.1	Test Purpose and Environment	2616

A.8.16.69.2	Test Requirements	2620

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2620

A.8.16.70.1	Test Purpose and Environment	2620

A.8.16.70.2	Test Requirements	2624

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2624

A.8.16.71.1	Test Purpose and Environment	2624

A.8.16.71.2	Test Requirements	2629

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2629

A.8.16.72.1	Test Purpose and Environment	2629

A.8.16.72.2	Test Requirements	2634

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2634

A.8.16.73.1	Test Purpose and Environment	2634

A.8.16.73.2	Test Requirements	2640

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2640

A.8.16.74.1	Test Purpose and Environment	2640

A.8.16.74.2	Test Requirements	2646

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2646

A.8.16.75.1	Test Purpose and Environment	2646

A.8.16.75.2	Test Requirements	2652

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2652

A.8.16.76.1	Test Purpose and Environment	2652

A.8.16.76.2	Test Requirements	2658

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2658

A.8.16.77.1	Test Purpose and Environment	2658

A.8.16.77.2	Test Requirements	2662

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2662

A.8.16.78.1	Test Purpose and Environment	2662

A.8.16.78.2	Test Requirements	2664

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2665

A.8.16.79.1	Test Purpose and Environment	2665

A.8.16.79.2	Test Requirements	2672

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2672

A.8.16.80.1	Test Purpose and Environment	2672

A.8.16.80.2	Test Requirements	2679

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2679

A.8.16.81.1	Test Purpose and Environment	2679

A.8.16.81.2	Test Requirements	2685

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2685

A.8.16.82.1	Test Purpose and Environment	2685

A.8.16.82.2	Test Requirements	2691

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2691

A.8.16.83.1	Test Purpose and Environment	2691

A.8.16.83.2	Test Requirements	2696

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2696

A.8.16.84.1	Test Purpose and Environment	2696

A.8.16.84.2	Test Requirements	2702

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2702

A.8.16.85.1	Test Purpose and Environment	2702

A.8.16.85.2	Test Requirements	2706

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2706

A.8.16.86.1	Test Purpose and Environment	2706

A.8.16.86.2	Test Requirements	2709

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2709

A.8.16.87.1	Test Purpose and Environment	2709

A.8.16.87.2	Test Requirements	2715

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2715

A.8.16.88.1	Test Purpose and Environment	2715

A.8.16.88.2	Test Requirements	2723

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2723

A.8.16.89.1	Test Purpose and Environment	2723

A.8.16.89.2	Test Requirements	2730

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2730

A.8.16.90.1	Test Purpose and Environment	2730

A.8.16.90.2	Test Requirements	2737

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2737

A.8.16.91.1	Test Purpose and Environment	2737

A.8.16.91.2	Test Requirements	2743

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2743

A.8.16.92.1	Test Purpose and Environment	2743

A.8.16.92.2	Test Requirements	2751

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2751

A.8.16.93.1	Test Purpose and Environment	2751

A.8.16.93.2	Test Requirements	2758

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2758

A.8.16.94.1	Test Purpose and Environment	2758

A.8.16.94.2	Test Requirements	2765

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2765

A.8.16.95.1	Test Purpose and Environment	2765

A.8.16.95.2	Test Requirements	2773

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2773

A.8.16.96.1	Test Purpose and Environment	2773

A.8.16.96.2	Test Requirements	2782

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2782

A.8.16.97.1	Test Purpose and Environment	2782

A.8.16.97.2	Test Requirements	2789

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2789

A.8.16.98.1	Test Purpose and Environment	2789

A.8.16.98.2	Test Requirements	2797

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2797

A.8.16.99.1	Test Purpose and Environment	2797

A.8.16.99.2	Test Requirements	2807

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2807

A.8.16.100.1	Test Purpose and Environment	2807

A.8.16.100.2	Test Requirements	2817

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2817

A.8.16.101.1	Test Purpose and Environment	2817

A.8.16.101.2	Test Requirements	2827

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2827

A.8.16.102.1	Test Purpose and Environment	2827

A.8.16.102.2	Test Requirements	2838

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2838

A.8.16.103.1	Test Purpose and Environment	2838

A.8.16.103.2	Test Requirements	2843

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2844

A.8.16.104.1	Test Purpose and Environment	2844

A.8.16.104.2	Test Requirements	2848

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2849

A.8.16.105.1	Test Purpose and Environment	2849

A.8.16.105.2	Test Requirements	2853

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2854

A.8.16.106.1	Test Purpose and Environment	2854

A.8.16.106.2	Test Requirements	2858

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2858

A.8.16.107.1	Test Purpose and Environment	2858

A.8.16.107.2	Test Requirements	2860

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2861

A.8.16.108.1	Test Purpose and Environment	2861

A.8.16.108.2	Test Requirements	2863

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2864

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2864

A.8.17.1.1	Test Purpose and Environment	2864

A.8.17.1.2	Test Requirements	2870

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2870

A.8.17.2.1	Test Purpose and Environment	2870

A.8.17.2.2	Test Requirements	2877

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2877

A.8.17.3.1	Test Purpose and Environment	2877

A.8.17.3.2	Test Requirements	2878

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2878

A.8.17.4.1	Test Purpose and Environment	2878

A.8.17.4.2	Test Requirements	2879

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2879

A.8.17.5.1	Test Purpose and Environment	2879

A.8.17.5.2	Test Requirements	2880

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2880

A.8.17.6.1	Test Purpose and Environment	2880

A.8.17.6.2	Test Requirements	2881

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2882

A.8.17.7.1	Test Purpose and Environment	2882

A.8.17.7.2	Test Requirements	2882

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2883

A.8.17.8.1	Test Purpose and Environment	2883

A.8.17.8.2	Test Requirements	2883

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2884

A.8.17.9.1	Test Purpose and Environment	2884

A.8.17.9.2	Test Requirements	2885

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2885

A.8.17.10.1	Test Purpose and Environment	2885

A.8.17.10.2	Test Requirements	2892

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2893

A.8.17.11.1	Test Purpose and Environment	2893

A.8.17.11.2	Test Requirements	2899

A.8.18	E-UTRAN TDD – HRPD Measurements	2900

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2900

A.8.18.1.1	Test Purpose and Environment	2900

A.8.18.1.2	Test Requirements	2902

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2902

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2902

A.8.19.1.1	Test Purpose and Environment	2902

A.8.19.1.2	Test Requirements	2903

A.8.20	Inter-frequency/RAT Measurements in CA mode	2904

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2904

A.8.20.1.1	Test Purpose and Environment	2904

A.8.20.1.2	Test Requirements	2905

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2905

A.8.20.2.1	Test Purpose and Environment	2906

A.8.20.2.2	Test Requirements	2907

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2908

A.8.20.2A.1	Test Purpose and Environment	2908

A.8.20.2A.2	Test Requirements	2908

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2908

A.8.20.2B.1	Test Purpose and Environment	2908

A.8.20.2B.2	Test Requirements	2911

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2911

A.8.20.3.1	Test Purpose and Environment	2911

A.8.20.3.2	Test Requirements	2913

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2913

A.8.20.4.1	Test Purpose and Environment	2913

A.8.20.4.1.1	1.28 Mcps TDD option	2913

A.8.20.4.2	Test Requirements	2915

A.8.20.4.2.1	1.28 Mcps TDD option	2915

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2915

A.8.20.4A.1	Test Purpose and Environment	2915

A.8.20.4A.1.1	1.28 Mcps TDD option	2915

A.8.20.4A.2	Test Requirements	2915

A.8.20.4A.2.1	1.28 Mcps TDD option	2915

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2916

A.8.20.4B.1	Test Purpose and Environment	2916

A.8.20.4B.1.1	1.28 Mcps TDD option	2916

A.8.20.4B.2	Test Requirements	2918

A.8.20.4B.2.1	1.28 Mcps TDD option	2918

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2918

A.8.21.1	Test Purpose and Environment	2918

A.8.21.2 Test Requirements	2922

A.8.22	E-UTRAN Discovery Signal Measurements	2922

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2922

A.8.22.1.1	Test Purpose and Environment	2922

A.8.22.1.2	Test Requirements	2925

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2925

A.8.22.2.1	Test Purpose and Environment	2925

A.8.22.2.2	Test Requirements	2928

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2928

A.8.22.3.1	Test Purpose and Environment	2928

A.8.22.3.2	Test Requirements	2931

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2931

A.8.22.4.1	Test Purpose and Environment	2931

A.8.22.4.2	Test Requirements	2934

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2934

A.8.22.5.1	Test Purpose and Environment	2934

A.8.22.5.2	Test Requirements	2938

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2938

A.8.22.6.1	Test Purpose and Environment	2938

A.8.22.6.2	Test Requirements	2942

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2942

A.8.22.7.1	Test Purpose and Environment	2942

A.8.22.7.2	Test Requirements	2946

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2946

A.8.22.8.1	Test Purpose and Environment	2946

A.8.22.8.2	Test Requirements	2950

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2950

A.8.22.9.1	Test Purpose and Environment	2950

A.8.22.9.2	Test Requirements	2952

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2953

A.8.22.10.1	Test Purpose and Environment	2953

A.8.22.10.2	Test Requirements	2955

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2956

A.8.22.11.1	Test Purpose and Environment	2956

A.8.22.11.2	Test Requirements	2959

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2959

A.8.22.12.1	Test Purpose and Environment	2959

A.8.22.12.2	Test Requirements	2963

A.8.23	E-UTRAN Dual Connectivity Measurements	2963

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2963

A.8.23.1.1	Test Purpose and Environment	2963

A.8.23.1.2	Test Requirements	2966

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2966

A.8.23.2.1	Test Purpose and Environment	2966

A.8.23.2.2	Test Requirements	2969

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2969

A.8.23.3.1	Test Purpose and Environment	2969

A.8.23.3.2	Test Requirements	2972

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2972

A.8.23.4.1	Test Purpose and Environment	2972

A.8.23.4.2	Test Requirements	2975

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2975

A.8.23.5.1	Test Purpose and Environment	2975

A.8.23.5.2	Test Requirements	2978

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2978

A.8.23.6.1	Test Purpose and Environment	2978

A.8.23.6.2	Test Requirements	2981

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2981

A.8.23.7.1	Test Purpose and Environment	2981

A.8.23.7.2	Test Requirements	2983

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2984

A.8.23.8.1	Test Purpose and Environment	2984

A.8.23.8.2	Test Requirements	2986

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2987

A.8.23.9.1	Test Purpose and Environment	2987

A.8.23.9.2	Test Requirements	2990

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2990

A.8.23.10.1	Test Purpose and Environment	2990

A.8.23.10.2	Test Requirements	2993

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2993

A.8.23.11.1	Test Purpose and Environment	2993

A.8.23.11.2	Test Requirements	2996

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2996

A.8.23.12.1	Test Purpose and Environment	2996

A.8.23.12.2	Test Requirements	2999

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2999

A.8.23.13.1	Test Purpose and Environment	2999

A.8.23.13.2	Test Requirements	3002

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3002

A.8.23.14.1	Test Purpose and Environment	3002

A.8.23.14.2	Test Requirements	3005

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3005

A.8.23.15.1	Test Purpose and Environment	3005

A.8.23.15.2	Test Requirements	3008

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3008

A.8.23.16.1	Test Purpose and Environment	3008

A.8.23.16.2	Test Requirements	3009

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3010

A.8.23.17.1	Test Purpose and Environment	3010

A.8.23.17.2	Test Requirements	3012

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3012

A.8.23.18.1	Test Purpose and Environment	3012

A.8.23.18.2	Test Requirements	3013

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3014

A.8.23.19.1	Test Purpose and Environment	3014

A.8.23.19.2	Test Requirements	3015

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3016

A.8.23.20.1	Test Purpose and Environment	3016

A.8.23.20.2	Test Requirements	3017

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3018

A.8.23.21.1	Test Purpose and Environment	3018

A.8.23.21.2	Test Requirements	3019

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3020

A.8.23.22.1	Test Purpose and Environment	3020

A.8.23.22.2	Test Requirements	3021

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3022

A.8.23.23.1	Test Purpose and Environment	3022

A.8.23.23.2	Test Requirements	3023

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3024

A.8.23.24.1	Test Purpose and Environment	3024

A.8.23.24.2	Test Requirements	3026

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3027

A.8.23.25.1	Test Purpose and Environment	3027

A.8.23.25.2	Test Requirements	3029

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3030

A.8.23.26.1	Test Purpose and Environment	3030

A.8.23.26.2	Test Requirements	3032

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3033

A.8.23.27.1	Test Purpose and Environment	3033

A.8.23.27.2	Test Requirements	3036

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3036

A.8.23.28.1	Test Purpose and Environment	3036

A.8.23.28.2	Test Requirements	3040

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3040

A.8.23.29.1	Test Purpose and Environment	3040

A.8.23.29.2	Test Requirements	3044

A.8.24	Proximity-based Services	3044

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3044

A.8.24.1.1	Test Purpose and Environment	3044

A.8.24.1.2	Test Requirements	3045

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3046

A.8.24.2.1	Test Purpose and Environment	3046

A.8.24.2.2	Test Requirements	3047

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3047

A.8.24.3.1	Test Purpose and Environment	3048

A.8.24.3.2	Test Requirements	3049

A.8.25	E-UTRAN-WLAN Measurements	3050

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3050

A.8.25.1.1	Test Purpose and Environment	3050

A.8.25.1.2	Test Requirements	3052

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3052

A.8.25.2.1	Test Purpose and Environment	3052

A.8.25.2.2	Test Requirements	3054

A.8.26 Frame Structure 3 (FS3)	3054

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3054

A.8.26.1.1	Test Purpose and Environment	3054

A.8.26.1.2	Test Requirements	3056

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3057

A.8.26.2.1	Test Purpose and Environment	3057

A.8.26.2.2	Test Requirements	3059

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3060

A.8.26.3.1	Test Purpose and Environment	3060

A.8.26.3.2	Test Requirements	3063

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3063

A.8.26.3A.1	Test Purpose and Environment	3063

A.8.26.3A.2	Test Requirements	3066

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3066

A.8.26.4.1	Test Purpose and Environment	3066

A.8.26.4.2	Test Requirements	3070

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3070

A.8.26.4A.1	Test Purpose and Environment	3070

A.8.26.4A.2	Test Requirements	3074

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3074

A.8.26.5.1	Test Purpose and Environment	3074

A.8.26.5.2	Test Requirements	3077

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3077

A.8.26.5A.1	Test Purpose and Environment	3077

A.8.26.5A.2	Test Requirements	3081

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3081

A.8.26.6.1	Test Purpose and Environment	3081

A.8.26.6.2	Test Requirements	3085

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3085

A.8.26.6A.1	Test Purpose and Environment	3085

A.8.26.6A.2	Test Requirements	3089

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3089

A.8.26.7.1	Test Purpose and Environment	3089

A.8.26.7.2	Test Requirements	3092

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3092

A.8.26.8.1	Test Purpose and Environment	3092

A.8.26.8.2	Test Requirements	3096

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3096

A.8.26.9.1	Test Purpose and Environment	3096

A.8.26.9.2	Test Requirements	3099

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3099

A.8.26.10.1	Test Purpose and Environment	3099

A.8.26.10.2	Test Requirements	3102

A.9	Measurement Performance Requirements	3104

A.9.1	RSRP	3104

A.9.1.1	FDD Intra frequency case	3104

A.9.1.1.1	Test Purpose and Environment	3104

A.9.1.1.2	Test parameters	3104

A.9.1.1.3	Test Requirements	3107

A.9.1.2	TDD Intra frequency case	3107

A.9.1.2.1	Test Purpose and Environment	3107

A.9.1.2.2	Test parameters	3107

A.9.1.2.3	Test Requirements	3108

A.9.1.3	FDD—FDD Inter frequency case	3109

A.9.1.3.1	Test Purpose and Environment	3109

A.9.1.3.2	Test parameters	3109

A.9.1.3.3	Test Requirements	3112

A.9.1.4	TDD—TDD Inter frequency case	3112

A.9.1.4.1	Test Purpose and Environment	3112

A.9.1.4.2	Test parameters	3112

A.9.1.4.3	Test Requirements	3114

A.9.1.5	FDD—TDD Inter frequency case	3115

A.9.1.5.1	Test Purpose and Environment	3115

A.9.1.5.2	Test parameters	3115

A.9.1.5.3	Test Requirements	3116

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3117

A.9.1.6.1	Test Purpose and Environment	3117

A.9.1.6.2	Test parameters	3117

A.9.1.6.3	Test Requirements	3120

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3120

A.9.1.7.1	Test Purpose and Environment	3120

A.9.1.7.2	Test parameters	3120

A.9.1.7.3	Test Requirements	3123

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3123

A.9.1.8.1	Test Purpose and Environment	3123

A.9.1.8.2	Test parameters	3123

A.9.1.8.3	Test Requirements	3127

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3127

A.9.1.9.1	Test Purpose and Environment	3127

A.9.1.9.2	Test parameters	3127

A.9.1.9.3	Test Requirements	3130

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3130

A.9.1.10.1	Test Purpose and Environment	3130

A.9.1.10.2	Test parameters	3130

A.9.1.10.3	Test Requirements	3133

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3133

A.9.1.11.1	Test Purpose and Environment	3133

A.9.1.11.2	Test parameters	3133

A.9.1.11.3	Test Requirements	3137

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3137

A.9.1.12.1	Test Purpose and Environment	3137

A.9.1.12.2	Test parameters	3137

A.9.1.12.3	Test Requirements	3138

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3138

A.9.1.13.1	Test Purpose and Environment	3138

A.9.1.13.2	Test parameters	3138

A.9.1.13.3	Test Requirements	3139

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3139

A.9.1.14.1	Test Purpose and Environment	3139

A.9.1.14.2	Test parameters	3139

A.9.1.14.3	Test Requirements	3143

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3143

A.9.1.15.1	Test Purpose and Environment	3143

A.9.1.15.2	Test parameters	3143

A.9.1.15.3	Test Requirements	3145

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3146

A.9.1.16.1	Test Purpose and Environment	3146

A.9.1.16.2	Test parameters	3146

A.9.1.16.3	Test Requirements	3147

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3147

A.9.1.17.1	Test Purpose and Environment	3147

A.9.1.17.2	Test parameters	3148

A.9.1.17.3	Test Requirements	3149

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3149

A.9.1.18.1	Test Purpose and Environment	3149

A.9.1.18.2	Test parameters	3149

A.9.1.18.3	Test Requirements	3150

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3151

A.9.1.19.1	Test Purpose and Environment	3151

A.9.1.19.2	Test parameters	3151

A.9.1.19.3	Test Requirements	3151

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3151

A.9.1.20.1	Test Purpose and Environment	3151

A.9.1.20.2	Test parameters	3151

A.9.1.20.3	Test Requirements	3153

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3153

A.9.1.21.1	Test Purpose and Environment	3153

A.9.1.21.2	Test parameters	3153

A.9.1.21.3	Test Requirements	3153

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3153

A.9.1.22.1	Test Purpose and Environment	3153

A.9.1.22.2	Test parameters	3154

A.9.1.22.3	Test Requirements	3158

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3158

A.9.1.23.1	Test Purpose and Environment	3158

A.9.1.23.2	Test parameters	3158

A.9.1.23.3	Test Requirements	3162

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3162

A.9.1.24.1	Test Purpose and Environment	3162

A.9.1.24.2	Test parameters	3162

A.9.1.24.3	Test Requirements	3163

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3163

A.9.1.25.1	Test Purpose and Environment	3163

A.9.1.25.2	Test parameters	3163

A.9.1.25.3	Test Requirements	3166

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3166

A.9.1.26.1	Test Purpose and Environment	3166

A.9.1.26.2	Test parameters	3166

A.9.1.26.3	Test Requirements	3169

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3169

A.9.1.27.1	Test Purpose and Environment	3169

A.9.1.27.2	Test parameters	3169

A.9.1.27.3	Test Requirements	3172

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3172

A.9.1.28.1	Test Purpose and Environment	3172

A.9.1.28.2	Test parameters	3172

A.9.1.28.3	Test Requirements	3175

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3175

A.9.1.29.1	Test Purpose and Environment	3175

A.9.1.29.2	Test parameters	3175

A.9.1.29.3	Test Requirements	3178

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3178

A.9.1.30.1	Test Purpose and Environment	3178

A.9.1.30.2	Test parameters	3179

A.9.1.30.3	Test Requirements	3182

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3182

A.9.1.31.1	Test Purpose and Environment	3182

A.9.1.31.2	Test parameters	3182

A.9.1.31.3	Test Requirements	3185

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3186

A.9.1.32.1	Test Purpose and Environment	3186

A.9.1.32.2	Test parameters	3186

A.9.1.32.3	Test Requirements	3189

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3189

A.9.1.33.1	Test Purpose and Environment	3189

A.9.1.33.2	Test parameters	3189

A.9.1.33.3	Test Requirements	3192

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3192

A.9.1.34.1	Test Purpose and Environment	3192

A.9.1.34.2	Test parameters	3192

A.9.1.34.3	Test Requirements	3195

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3195

A.9.1.35.1	Test Purpose and Environment	3195

A.9.1.35.2	Test parameters	3195

A.9.1.35.3	Test Requirements	3198

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3199

A.9.1.36.1	Test Purpose and Environment	3199

A.9.1.36.2	Test parameters	3199

A.9.1.36.3	Test Requirements	3202

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3202

A.9.1.37.1	Test Purpose and Environment	3202

A.9.1.37.2	Test parameters	3202

A.9.1.37.3	Test Requirements	3205

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3206

A.9.1.38.1	Test Purpose and Environment	3206

A.9.1.38.2	Test parameters	3206

A.9.1.38.3	Test Requirements	3209

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3210

A.9.1.39.1	Test Purpose and Environment	3210

A.9.1.39.2	Test parameters	3210

A.9.1.39.3	Test Requirements	3216

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3216

A.9.1.40.1	Test Purpose and Environment	3216

A.9.1.40.2	Test parameters	3217

A.9.1.40.3	Test Requirements	3223

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3223

A.9.1.41.1	Test Purpose and Environment	3223

A.9.1.41.2	Test parameters	3223

A.9.1.41.3	Test Requirements	3226

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3226

A.9.1.42.1	Test Purpose and Environment	3226

A.9.1.42.2	Test parameters	3226

A.9.1.42.3	Test Requirements	3229

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3229

A.9.1.43.1	Test Purpose and Environment	3229

A.9.1.43.2	Test parameters	3229

A.9.1.43.3	Test Requirements	3230

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3231

A.9.1.44.1	Test Purpose and Environment	3231

A.9.1.44.2	Test parameters	3231

A.9.1.44.3	Test Requirements	3236

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3236

A.9.1.45.1	Test Purpose and Environment	3236

A.9.1.45.2	Test parameters	3236

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2582

A.9.1.73.1	Test Purpose and Environment	2582

A.9.1.73.2	Test parameters	2582

A.9.1.73.3	Test Requirements	2585

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2585

A.9.1.74.1	Test Purpose and Environment	2585

A.9.1.74.2	Test parameters	2585

A.9.1.74.3	Test Requirements	2586

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2586

A.9.1.75.1	Test Purpose and Environment	2586

A.9.1.75.2	Test parameters	2587

A.9.1.75.3	Test Requirements	2588

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2588

A.9.1.76.1	Test Purpose and Environment	2588

A.9.1.76.2	Test parameters	2588

A.9.1.76.3	Test Requirements	2591

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2591

A.9.1.77.1	Test Purpose and Environment	2591

A.9.1.77.2	Test parameters	2591

A.9.1.77.3	Test Requirements	2594

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2594

A.9.1.78.1	Test Purpose and Environment	2594

A.9.1.78.2	Test parameters	2594

A.9.1.78.3	Test Requirements	2597

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2597

A.9.1.79.1	Test Purpose and Environment	2597

A.9.1.79.2	Test parameters	2597

A.9.1.79.3	Test Requirements	2600

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2600

A.9.1.80.1	Test Purpose and Environment	2600

A.9.1.80.2	Test parameters	2600

A.9.1.80.3	Test Requirements	2603

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2603

A.9.1.81.1	Test Purpose and Environment	2603

A.9.1.78.2	Test parameters	2603

A.9.1.81.3	Test Requirements	2606

A.9.2	RSRQ	2606

A.9.2.1	FDD Intra frequency case	2606

A.9.2.1.1	Test Purpose and Environment	2606

A.9.2.1.2	Test parameters	2606

A.9.2.1.3	Test Requirements	2609

A.9.2.2	TDD Intra frequency case	2609

A.9.2.2.1	Test Purpose and Environment	2609

A.9.2.2.2	Test parameters	2609

A.9.2.2.3	Test Requirements	2611

A.9.2.3	FDD—FDD Inter frequency case	2611

A.9.2.3.1	Test Purpose and Environment	2611

A.9.2.3.2	Test parameters	2611

A.9.2.3.3	Test Requirements	2614

A.9.2.4	TDD—TDD Inter frequency case	2614

A.9.2.4.1	Test Purpose and Environment	2614

A.9.2.4.2	Test parameters	2614

A.9.2.4.3	Test Requirements	2616

A.9.2.4A	FDD—TDD Inter frequency case	2617

A.9.2.4A.1	Test Purpose and Environment	2617

A.9.2.4A.2	Test parameters	2617

A.9.2.4A.3	Test Requirements	2619

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2620

A.9.2.5.3	Test Requirements	2623

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2623

A.9.2.6.1	Test Purpose and Environment	2623

A.9.2.6.2	Test parameters	2624

A.9.2.6.3	Test Requirements	2627

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2627

A.9.2.7.1	Test Purpose and Environment	2627

A.9.2.7.2	Test parameters	2627

A.9.2.7.3	Test Requirements	2631

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2631

A.9.2.8.1	Test Purpose and Environment	2631

A.9.2.8.2	Test parameters	2631

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2830

A.9.3.2 E-UTRAN TDD	2830

A.9.3.2.1	Test Purpose and Environment	2830

A.9.3.2.2	Parameters	2830

A.9.3.2.3	Test Requirements	2833

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2833

A.9.3.3.1	Test Purpose and Environment	2833

A.9.3.3.2	Parameters	2833

A.9.3.3.3	Test Requirements	2834

A.9.4	UTRAN FDD CPICH Ec/No	2835

A.9.4.1	E-UTRAN FDD	2835

A.9.4.1.1	Test Purpose and Environment	2835

A.9.4.1.2	Parameters	2835

A.9.4.1.3	Test Requirements	2837

A.9.4.2	E-UTRAN TDD	2838

A.9.4.2.1	Test Purpose and Environment	2838

A.9.4.2.2	Parameters	2838

A.9.4.2.3	Test Requirements	2840

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2841

A.9.4.3.1	Test Purpose and Environment	2841

A.9.4.3.2	Parameters	2841

A.9.4.3.3	Test Requirements	2842

A.9.5	UTRAN TDD measurement	2842

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2842

A.9.5.1.1	Test Purpose and Environment	2842

A.9.5.1.2	Test parameters	2842

A.9.5.1.3	Test Requirements	2844

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2845

A.9.5.2.1	Test Purpose and Environment	2845

A.9.5.2.2	Test parameters	2845

A.9.5.2.3	Test Requirements	2846

A.9.6	GSM Carrier RSSI	2847

A.9.6.1	E-UTRAN FDD	2847

A.9.6.1.1	Test Purpose and Environment	2847

A.9.6.1.2	Test Requirements	2848

A.9.6.2	E-UTRAN TDD	2848

A.9.6.2.1	Test Purpose and Environment	2848

A.9.6.2.2	Test Requirements	2850

A.9.7	UE Rx – Tx Time Difference	2851

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2851

A.9.7.1.1	Test Purpose and Environment	2851

A.9.7.1.2	Test parameters	2851

A.9.7.1.3	Test Requirements	2853

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2853

A.9.7.2.1	Test Purpose and Environment	2853

A.9.7.2.2	Test parameters	2853

A.9.7.2.3	Test Requirements	2855

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2855

A.9.7.3.1	Test Purpose and Environment	2855

A.9.7.3.2	Test parameters	2855

A.9.7.3.3	Test Requirements	2858

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2858

A.9.7.4.1	Test Purpose and Environment	2858

A.9.7.4.2	Test Parameters	2858

A.9.7.4.3	Test Requirements	2861

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2861

A.9.7.5.1	Test Purpose and Environment	2861

A.9.7.5.2	Test parameters	2861

A.9.7.5.3	Test Requirements	2864

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2864

A.9.7.6.1	Test Purpose and Environment	2864

A.9.7.6.2	Test Parameters	2864

A.9.7.6.3	Test Requirements	2867

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2867

A.9.7.7.1	Test Purpose and Environment	2867

A.9.7.7.2	Test parameters	2867

A.9.7.7.3	Test Requirements	2868

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.8.1	Test Purpose and Environment	2869

A.9.7.8.2	Test parameters	2869

A.9.7.8.3	Test Requirements	2870

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2870

A.9.7.9.1	Test Purpose and Environment	2870

A.9.7.9.2	Test parameters	2870

A.9.7.9.3	Test Requirements	2871

A.9.8	RSTD	2872

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2872

A.9.8.1.1	Test Purpose and Environment	2872

A.9.8.1.2	Test Requirements	2874

A.9.8.1.2A	Test Requirements for UE Category 1bis	2874

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2875

A.9.8.2.1	Test Purpose and Environment	2875

A.9.8.2.2	Test Requirements	2878

A.9.8.2.2A	Test Requirements for UE Category 1bis	2878

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2879

A.9.8.3.1	Test Purpose and Environment	2879

A.9.8.3.2	Test Requirements	2881

A.9.8.3.2A	Test Requirements for UE Category 1bis	2881

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2882

A.9.8.4.1	Test Purpose and Environment	2882

A.9.8.4.2	Test Requirements	2884

A.9.8.4.2A	Test Requirements for UE Category 1bis	2884

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2885

A.9.8.5.1	Test Purpose and Environment	2885

A.9.8.5.2	Test Requirements	2887

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2887

A.9.8.6.1	Test Purpose and Environment	2887

A.9.8.6.2	Test Requirements	2890

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.7.1	Test Purpose and Environment	2890

A.9.8.7.2	Test Requirements	2891

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2891

A.9.8.8.1	Test Purpose and Environment	2891

A.9.8.8.2	Test Requirements	2892

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.9.1	Test Purpose and Environment	2892

A.9.8.9.2	Test Requirements	2893

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2893

A.9.8.10.1	Test Purpose and Environment	2893

A.9.8.10.2	Test Requirements	2894

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2894

A.9.8.11.1	Test Purpose and Environment	2894

A.9.8.11.2	Test Requirements	2895

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2895

A.9.8.12.1	Test Purpose and Environment	2895

A.9.8.12.2	Test Requirements	2896

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2896

A.9.8.13.1	Test Purpose and Environment	2896

A.9.8.13.2	Test Requirements	2897

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2897

A.9.8.14.1	Test Purpose and Environment	2897

A.9.8.14.2	Test Requirements	2904

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2904

A.9.8.15.1	Test Purpose and Environment	2904

A.9.8.15.2	Test Requirements	2910

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2910

A.9.8.16.1	Test Purpose and Environment	2910

A.9.8.16.2	Test Requirements	2913

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2913

A.9.8.17.1	Test Purpose and Environment	2913

A.9.8.17.2	Test Requirements	2916

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2916

A.9.8.18.1	Test Purpose and Environment	2916

A.9.8.18.2	Test Requirements	2919

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2919

A.9.8.19.1	Test Purpose and Environment	2919

A.9.8.19.2	Test Requirements	2922

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2922

A.9.8.20.1	Test Purpose and Environment	2922

A.9.8.20.2	Test Requirements	2925

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2926

A.9.8.21.1	Test Purpose and Environment	2926

A.9.8.21.2	Test Requirements	2929

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2930

A.9.8.22.1	Test Purpose and Environment	2930

A.9.8.22.2	Test Requirements	2933

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2934

A.9.8.23.1	Test Purpose and Environment	2934

A.9.8.23.2	Test Requirements	2937

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2937

A.9.8.24.1	Test Purpose and Environment	2937

A.9.8.24.2	Test Requirements	2941

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2942

A.9.8.25.1	Test Purpose and Environment	2942

A.9.8.25.2	Test Requirements	2945

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2946

A.9.8.26.1	Test Purpose and Environment	2946

A.9.8.26.2	Test Requirements	2948

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2948

A.9.8.27.1	Test Purpose and Environment	2948

A.9.8.27.2	Test Requirements	2951

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2951

A.9.8.28.1	Test Purpose and Environment	2951

A.9.8.28.2	Test Requirements	2954

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2954

A.9.8.29.1	Test Purpose and Environment	2954

A.9.8.29.2	Test Requirements	2958

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2959

A.9.8.30.1	Test Purpose and Environment	2959

A.9.8.30.2	Test Requirements	2962

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2963

A.9.8.31.1	Test Purpose and Environment	2963

A.9.8.31.2	Test Requirements	2967

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2967

A.9.8.32.1	Test Purpose and Environment	2967

A.9.8.32.2	Test Requirements	2970

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2970

A.9.8.33.1	Test Purpose and Environment	2970

A.9.8.33.2	Test Requirements	2973

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2973

A.9.8.34.1	Test Purpose and Environment	2973

A.9.8.34.2	Test Requirements	2976

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2976

A.9.8.35.1	Test Purpose and Environment	2976

A.9.8.35.2	Test Requirements	2979

A.9.9	RSRP and RSRQ on the serving cell	2979

A.9.9.1	FDD Intra frequency serving cell case	2979

A.9.9.1.1	Test Purpose and Environment	2979

A.9.9.1.2	Test parameters	2979

A.9.9.1.3	Test Requirements	2982

A.9.9.2	TDD Intra frequency serving cell case	2982

A.9.9.2.1	Test Purpose and Environment	2982

A.9.9.2.2	Test parameters	2982

A.9.9.2.3	Test Requirements	2984

A.9.10	SSTD	2984

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2984

A.9.10.1.1	Test Purpose and Environment	2984

A.9.10.1.2	Test parameters	2984

A.9.10.1.3	Test Requirements	2986

A.9.10.2	Void	2986

A.9.10.3	Void	2986

A.9.10.4	Void	2986

A.9.11 RSSI	2986

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2986

A.9.11.1.1	Test Purpose and Environment	2986

A.9.11.1.2	Test parameters	2986

A.9.11.1.3	Test Requirements	2989

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2989

A.9.11.2.1	Test Purpose and Environment	2989

A.9.11.2.2	Test parameters	2989

A.9.12	Channel occupancy	2992

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2992

A.9.12.1.1	Test Purpose and Environment	2992

A.9.12.1.2	Test parameters	2992

A.9.12.1.3	Test Requirements	2995

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2995

A.9.12.2.1	Test Purpose and Environment	2995

A.9.12.2.2	Test parameters	2995

A.9.12.2.3	Test Requirements	2998

A.9.13	RS-SINR	2998

A.9.13.1	FDD Intra-Frequency Case	2998

A.9.13.1.1	Test Purpose and Environment	2998

A.9.13.1.2	Test parameters	2998

A.9.13.1.3	Test Requirements	3001

A.9.13.2	TDD Intra-Frequency Case	3001

A.9.13.2.1	Test Purpose and Environment	3001

A.9.13.2.2	Test parameters	3001

A.9.13.2.3	Test Requirements	3004

A.9.13.3	FDD—FDD Inter frequency case	3004

A.9.13.3.1	Test Purpose and Environment	3004

A.9.13.3.2	Test parameters	3004

A.9.13.3.3	Test Requirements	3007

A.9.13.4	TDD—TDD Inter frequency case	3007

A.9.13.4.1	Test Purpose and Environment	3007

A.9.13.4.2	Test parameters	3007

A.9.13.4.3	Test Requirements	3011

A.9.13.5	FDD—TDD Inter frequency case	3012

A.9.13.5.1	Test Purpose and Environment	3012

A.9.13.5.2	Test parameters	3012

A.9.13.5.3	Test Requirements	3018

A.9.13.6	TDD—FDD Inter frequency case	3018

A.9.13.6.1	Test Purpose and Environment	3018

A.9.13.6.2	Test parameters	3018

A.9.13.6.3	Test Requirements	3022

A.9.14	Channel quality reporting accuracy	3022

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3022

A.9.14.1.1	Test Purpose and Environment	3022

A.9.14.1.2	Test parameters	3023

A.9.14.1.3	Test Requirements	3023

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3024

A.9.14.2.1	Test Purpose and Environment	3024

A.9.14.2.2	Test parameters	3024

A.9.14.2.3	Test Requirements	3025

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3025

A.9.14.3.1	Test Purpose and Environment	3025

A.9.14.3.2	Test parameters	3025

A.9.14.3.3	Test Requirements	3026

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3026

A.9.14.4.1	Test Purpose and Environment	3026

A.9.14.4.2	Test parameters	3026

A.9.14.4.3	Test Requirements	3027

A.10	Proximity-based Services in Any Cell Selection State	3050

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3050

A.10.1.1	Test Purpose and Environment	3050

A.10.1.2	Test Requirements	3051

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3052

A.10.2.1	Test Purpose and Environment	3052

A.10.2.2	Test Requirements	3053

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3053

A.10.3.1	Test Purpose and Environment	3053

A.10.3.2	Test Requirements	3055

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3056

A.10.4.1	Test Purpose and Environment	3056

A.10.4.2	Test Requirements	3058

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3058

A.11.1	V2V UE Transmission Timing Accuracy Test	3058

A.11.1.1	Test Purpose and Environment	3058

A.11.1.2	Test requirements	3059

A.11.2	Interruptions due to V2V sidelink communication	3059

A.11.2.1	Test Purpose and Environment	3059

A.11.2.2	Test Requirements	3061

A.12	3061

A.12.1	V2X UE Transmission Timing Accuracy Test	3061

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3061

A.12.1.1.1	Test Purpose and Environment	3061

A.12.1.1.2	Test requirements	3062

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3063

A.12.1.2.1	Test Purpose and Environment	3063

A.12.1.2.2	Test Requirements	3063

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3064

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3064

A.12.2.1.1	Test Purpose and Environment	3064

A.12.2.1.2	Test Requirements	3065

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3066

A.12.2.2.1	Test Purpose and Environment	3066

A.12.2.2.2	Test Requirements	3067

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3067

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3067

A.12.3.1.1	Test Purpose and Environment	3067

A.12.3.1.2	Test Requirements	3070

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3071

A.12.3.2.1	Test Purpose and Environment	3071

A.12.3.1.2	Test Requirements	3073

A.12.4	Congestion Control Measurement Test for V2X UE	3073

A.12.4.1	Test Purpose and Environment	3073

A.12.4.2	Test Requirements	3075

A.12.5	Interruptions due to V2X Sidelink Communication	3075

A.12.5.1	Test Purpose and Environment	3075

A.12.5.2	Test Requirements	3077

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3077

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3077

A.12.6.1.1	Test Purpose and Environment	3077

A.12.6.1.2	Test Requirements	3079

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3079

A.12.6.2.1	Test Purpose and Environment	3079

A.12.6.1.2	Test Requirements	3081

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3081

A.12.7.1	Test Purpose and Environment	3081

A.12.7.2	Test Requirements	3084

A.12.8	Interruptions due to V2X Carrier Aggregation	3085

A.12.8.1	Interruptions on a FDD PCell	3085

A.12.8.1.1	Test Purpose and Environment	3085

A.12.8.1.2	Test Requirements	3086

A.12.8.2	Interruptions on a TDD PCell	3086

A.12.8.2.1	Test Purpose and Environment	3086

A.12.8.2.2	Test Requirements	3088

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	3089

A.13.1	RRC_IDLE state for satellite access	3089

A.13.1.1	Cell re-selection for satellite access	3089

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	3089

A.13.1.1.1.1	Test Purpose and Environment	3089

A.13.1.1.1.2	Test Requirements	3090

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	3091

A.13.1.1.2.1	Test Purpose and Environment	3091

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	3093

A.13.1.1.3.1	Test Purpose and Environment	3093

A.13.1.1.3.2	Test Requirements	3095

A.13.2	Void	3096

A.13.3	RRC connection mobility control for satellite access	3096

A.13.3.1	RRC re-establishment for satellite access	3096

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	3096

A.13.3.1.1.1	Test Purpose and Environment	3096

A.13.3.1.1.2	Test Requirements	3097

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	3098

A.13.3.1.2.1	Test Purpose and Environment	3098

A.13.3.1.2.2	Test Requirements	3099

A.13.3.2	Random Access for Satellite Access	3100

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	3100

A.13.3.2.1.1	Test Purpose and Environment	3100

A.13.3.2.1.2	Test Requirements	3102

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	3104

A.13.3.2.2.1	Test Purpose and Environment	3104

A.13.3.2.2.2	Test Requirements	3105

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	3107

A.13.3.2.3.1	Test Purpose and Environment	3107

A.13.3.2.3.2	Test Requirements	3108

A.13.4	Timing and signalling characteristics for satellite access	3110

A.13.4.1	UE transmit timing for satellite access	3110

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	3110

A.13.4.1.1.1	Test Purpose and Environment	3110

A.13.4.1.1.2	Test Requirements	3111

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	3112

A.13.4.1.2.1	Test Purpose and Environment	3112

A.13.4.1.2.2	Test Requirements	3113

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	3114

A.13.4.1.3.1	Test Purpose and Environment	3114

A.13.4.1.3.2	Test Requirements	3115

A.13.4.2	UE timing advance for satellite access	3116

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	3116

A.13.4.2.1.1	Test Purpose and Environment	3116

A.13.4.2.1.2	Test Requirements	3118

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	3118

A.13.4.2.2.1	Test Purpose and Environment	3118

A.13.4.2.2.2	Test Requirements	3120

A.13.4.3	Radio Link Monitoring for satellite access	3120

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	3120

A.13.4.3.1.1	Test Purpose and Environment	3120

A.13.4.3.1.2	Test Requirements	3123

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	3123

A.13.4.3.2.1	Test Purpose and Environment	3123

A.13.4.3.2.2	Test Requirements	3126

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	3126

A.13.4.3.3.1	Test Purpose and Environment	3126

A.13.4.3.3.2	Test Requirements	3129

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	3129

A.13.4.3.4.1	Test Purpose and Environment	3129

A.13.4.3.4.2	Test Requirements	3132

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	3132

A.13.4.3.5.1	Test Purpose and Environment	3132

A.13.4.3.5.2	Test Requirements	3135

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	3135

A.13.4.3.6.1	Test Purpose and Environment	3135

A.13.4.3.6.2	Test Requirements	3138

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	3138

A.13.4.3.7.1	Test Purpose and Environment	3138

A.13.4.3.7.2	Test Requirements	3140

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	3141

A.13.4.3.8.1	Test Purpose and Environment	3141

A.13.4.3.8.2	Test Requirements	3143

A.13.5	Void	3144

A.13.6	Measurement performance requirements for UE for satellite access	3144

A.13.6.1	Void	3144

A.13.6.2	Channel quality reporting accuracy for satellite access	3144

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3144

A.13.6.2.1.1	Test Purpose and Environment	3144

A.13.6.2.1.2	Test parameters	3144

A.13.6.2.1.3	Test Requirements	3145

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3145

A.13.6.2.2.1	Test Purpose and Environment	3145

A.13.6.2.2.2	Test parameters	3145

A.13.6.2.2.3	Test Requirements	3146

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3146

A.13.6.2.3.1	Test Purpose and Environment	3146

A.13.6.2.3.2	Test parameters	3147

A.13.6.2.3.3	Test Requirements	3147

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3148

A.13.6.2.4.1	Test Purpose and Environment	3148

A.13.6.2.4.2	Test parameters	3148

A.13.6.2.4.3	Test Requirements	3149

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	3149

A.13.6.2.5.1	Test Purpose and Environment	3149

A.13.6.2.5.2	Test parameters	3149

A.13.6.2.5.3	Test Requirements	3150

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	3151

A.13.6.2.6.1	Test Purpose and Environment	3151

A.13.6.2.6.2	Test parameters	3151

A.13.6.2.6.3	Test Requirements	3152

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	3152

A.14.1	RRC_IDLE state for satellite access	3152

A.14.1.1	Cell re-selection for satellite access	3152

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	3152

A.14.1.1.1.1	Test Purpose and Environment	3152

A.14.1.1.1.2	Test Requirements	3154

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	3155

A.14.1.1.2.1	Test Purpose and Environment	3155

A.14.1.1.2.2	Test Requirements	3157

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	3158

A.14.1.1.3.1	Test Purpose and Environment	3158

A.14.1.1.3.2	Test Requirements	3160

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	3161

A.14.1.1.4.1	Test Purpose and Environment	3161

A.14.1.1.4.2	Test Requirements	3163

A.14.2	RRC_CONNECTED state mobility for satellite access	3164

A.14.2.1	E-UTRAN handover for satellite access	3164

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	3164

A.14.2.1.1.1	Test Purpose and Environment	3164

A.14.2.1.1.2	Test Requirements	3166

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	3167

A.14.2.1.2.1	Test Purpose and Environment	3167

A.14.2.1.2.2	Test Requirements	3168

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	3169

A.14.2.1.3.1	Test Purpose and Environment	3169

A.14.2.1.3.2	Test Requirements	3170

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	3171

A.14.2.1.4.1	Test Purpose and Environment	3171

A.14.2.1.4.2	Test Requirements	3172

A.14.3	RRC connection mobility control for satellite access	3173

A.14.3.1	RRC re-establishment for satellite access	3173

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	3173

A.14.3.1.1.1	Test Purpose and Environment	3173

A.14.3.1.1.2	Test Requirements	3174

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	3175

A.14.3.1.2.1	Test Purpose and Environment	3175

A.14.3.1.2.2	Test Requirements	3176

A.14.3.2	Random access for satellite access	3177

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	3177

A.14.3.2.1.1	Test Purpose and Environment	3177

A.14.3.2.1.2	Test Requirements	3179

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	3181

A.14.3.2.2.1	Test Purpose and Environment	3181

A.14.3.2.2.2	Test Requirements	3183

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	3185

A.14.3.2.3.1	Test Purpose and Environment	3185

A.14.3.2.3.2	Test Requirements	3187

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	3188

A.14.3.2.4.1	Test Purpose and Environment	3188

A.14.3.2.4.2	Test Requirements	3190

A.14.4	Timing and signalling characteristics for satellite access	3192

A.14.4.1	UE transmit timing for satellite access	3192

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	3192

A.14.4.1.1.1	Test Purpose and Environment	3192

A.14.4.1.1.2	Test Requirements	3194

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	3195

A.14.4.1.2.1	Test Purpose and Environment	3195

A.14.4.1.2.2	Test Requirements	3196

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	3197

A.14.4.1.3.1	Test Purpose and Environment	3197

A.14.4.1.3.2	Test Requirements	3198

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	3199

A.14.4.1.4.1	Test Purpose and Environment	3199

A.14.4.1.4.2	Test Requirements	3200

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	3201

A.14.4.1.5.1	Test Purpose and Environment	3201

A.14.4.1.5.2	Test Requirements	3202

A.14.4.2	UE timing advance for satellite access	3203

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	3203

A.14.4.2.1.1	Test Purpose and Environment	3203

A.14.4.2.1.2	Test Requirements	3205

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	3205

A.14.4.2.2.1	Test Purpose and Environment	3205

A.14.4.2.2.2	Test Requirements	3208

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	3208

A.14.4.2.3.1	Test Purpose and Environment	3208

A.14.4.2.3.2	Test Requirements	3209

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	3209

A.14.4.2.4.1	Test Purpose and Environment	3209

A.14.4.2.4.2	Test Requirements	3211

A.14.4.3	Radio Link Monitoring for satellite access	3212

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	3212

A.14.4.3.1.1	Test Purpose and Environment	3212

A.14.4.3.1.2	Test Requirements	3215

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	3215

A.14.4.3.2.1	Test Purpose and Environment	3215

A.14.4.3.2.2	Test Requirements	3218

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	3218

A.14.4.3.3.1	Test Purpose and Environment	3218

A.14.4.3.3.2	Test Requirements	3221

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	3221

A.14.4.3.4.1	Test Purpose and Environment	3221

A.14.4.3.4.2	Test Requirements	3224

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	3224

A.14.4.3.5.1	Test Purpose and Environment	3224

A.14.4.3.5.2	 Test Requirements	3227

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	3227

A.14.4.3.6.1	Test Purpose and Environment	3227

A.14.4.3.6.2	Test Requirements	3230

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	3230

A.14.4.3.7.1	Test Purpose and Environment	3230

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	3233

A.14.4.3.8.1 	Test Purpose and Environment	3233

A.14.4.3.8.2	Test Requirements	3236

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	3236

A.14.5.1 Intra-frequency measurements for satellite access	3236

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	3236

A.14.5.1.1.1	Test Purpose and Environment	3236

A.14.5.1.1.2	Test Requirements	3238

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	3239

A.14.5.1.2.1	Test Purpose and Environment	3239

A.14.5.1.2.2	Test Requirements	3241

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	3241

A.14.5.1.3.1	Test Purpose and Environment	3241

A.14.5.1.3.2	Test Requirements	3243

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	3244

A.14.5.1.4.1	Test Purpose and Environment	3244

A.14.5.1.4.2	Test Requirements	3246

A.14.6	Measurement performance requirements for UE for satellite access	3246

A.14.6.1	RSRP for satellite access	3246

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3246

A.14.6.1.1.1	Test Purpose and Environment	3246

A.14.6.1.1.2	Test parameters	3246

A.14.6.1.1.3	Test Requirements	3249

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3249

A.14.6.1.2.1	Test Purpose and Environment	3249

A.14.6.1.2.2	Test parameters	3249

A.14.6.1.2.3	Test Requirements	3252

A.14.6.2	Channel quality reporting accuracy for satellite access	3252

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	3252

A.14.6.2.1.1	Test Purpose and Environment	3252

A.14.6.2.1.2	Test parameters	3252

A.14.6.2.1.3	Test Requirements	3253

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	3253

A.14.6.2.2.1	Test Purpose and Environment	3253

A.14.6.2.2.2	Test parameters	3254

A.14.6.2.2.3	Test Requirements	3254

A.14.6.2.3.2	Test parameters	3255

A.14.6.2.3.3	Test Requirements	3256

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	3256

A.14.6.2.4.1	Test Purpose and Environment	3256

A.14.6.2.4.2	Test parameters	3256

A.14.6.2.4.3	Test Requirements	3257

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3258

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3258

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3258

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3258

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3258

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3260

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3261

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3261

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3261

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3261

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3263

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3264

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	3264

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	3265

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3266

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3266

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3266

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3267

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3267

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3267

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3268

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3268

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3268

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3269

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3270

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3270

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3270

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3270

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3270

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3271

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3271

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3272

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3272

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3272

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3272

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3274

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3275

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3275

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3276

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3277

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3277

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3278

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3278

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3279

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3279

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3280

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3280

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3281

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	3281

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	3282

B.3	Conditions for measurements performance requirements for UE	3282

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3282

B.3.2	Void	3283

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3283

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3283

B.3.5	Conditions for UE Rx – Tx time difference	3283

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3283

B.3.7	Conditions for inter-frequency RSTD measurements	3283

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3284

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3284

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3284

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3284

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3285

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3285

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3285

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3285

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3285

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3286

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3286

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3286

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3286

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3286

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3286

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3287

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3287

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3287

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3287

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3287

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3287

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3287

B.3.21.1	Conditions for RSRP measurements	3287

B.3.21.2	Conditions for RSRQ measurements	3288

B.3.21.3	Conditions for CSI-RSRP measurements	3288

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3288

B.3.22.1	Conditions for RSRP measurements	3288

B.3.22.2	Void	3288

B.3.22.3	Conditions for CSI-RSRP measurements	3288

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3289

B.3.23.1	Conditions for RSRP measurements	3289

B.3.23.2	Conditions for RSRQ measurements	3289

B.3.23.3	Conditions for CSI-RSRP measurements	3289

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3289

B.3.24.1	Conditions for RSRP measurements	3289

B.3.24.2	Conditions for RSRQ measurements	3289

B.3.24.3	Conditions for CSI-RSRP measurements	3289

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3290

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3290

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3290

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3290

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3291

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3291

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3291

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3291

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3291

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3291

B.4	RRM Requirements Exceptions	3292

B.4.1	General	3292

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3292

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3292

B.4.3.1	Inter-band carrier aggregation	3292

B.4.3.2	Intra-band non-contiguous carrier aggregation	3292

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3292

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3293

B.5.1	Conditions for S-RSRP Accuracy Requirements	3293

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3293

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3293

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3294

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3294

B.6	Conditions for V2X	3295

B.6.1	Test parameters for GNSS signals	3295

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3295

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3295

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3295

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3296

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3296

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3296

Annex C (informative): Change history:	3298