| 3GPP TS 36.133 V18.3.1 (2023-09) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	103

1	Scope	105

2	References	105

3	Definitions, symbols and abbreviations	107

3.1	Definitions	107

3.2	Symbols	109

3.3	Abbreviations	110

3.4	Test tolerances	113

3.5	Additional notation	113

3.5.1	Groups of bands	113

3.5.1A	Groups of bands for satellite access	116

3.6	General	117

3.6.1	Applicability of requirements in this specification version	117

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	123

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	125

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	126

3.6.2	Applicability of requirements for EN-DC operation	127

3.6.3	Applicability of requirements for NE-DC operation	128

3.6.4	Applicability of requirements for NGEN-DC operation	129

4	E-UTRAN RRC_IDLE state mobility	129

4.1	Cell Selection	129

4.2	Cell Re-selection	129

4.2.1	Introduction	129

4.2.2	Requirements	130

4.2.2.1	Measurement and evaluation of serving cell	131

4.2.2.2	Void	132

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	132

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	134

4.2.2.5	Measurements of inter-RAT cells	136

4.2.2.5.1	Measurements of UTRAN FDD cells	136

4.2.2.5.2	Measurements of UTRAN TDD cells	138

4.2.2.5.3	Measurements of GSM cells	139

4.2.2.5.4	Measurements of HRPD cells	140

4.2.2.5.5	Measurements of cdma2000 1X	141

4.2.2.5.6	Measurements of NR cells	142

4.2.2.5.7	Measurements of NR cells subject to CCA	145

4.2.2.5.8	Measurements of NR cells for RedCap	147

4.2.2.6	Evaluation of cell re-selection criteria	149

4.2.2.7	Maximum interruption in paging reception	149

4.2.2.8	void	150

4.2.2.9	UE measurement capability	150

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	150

4.2.2.10	Reselection to CSG cells	151

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	151

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	152

4.2.2.11	Void	153

4.2.2.12	Void	153

4.2.2.13	Void	153

4.2.2.14	UE measurement capability for RedCap	153

4.3	Minimization of Drive Tests (MDT)	154

4.3.1	Introduction	154

4.3.2	Measurements	154

4.3.2.1	Requirements	154

4.3.3	Relative Time Stamp Accuracy	154

4.3.3.1	Requirements	154

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	155

4.3.4.1	 Requirements	155

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	155

4.3.5.1	Requirements for timeSinceFailure	155

4.4	MBSFN Measurements	155

4.4.1	Introduction	155

4.4.2	MBSFN RSRP measurements	155

4.4.3	MBSFN RSRQ measurements	156

4.4.4	MCH BLER measurements	156

4.5	Proximity-based Services	156

4.5.1	Introduction	156

4.5.2	Requirements	156

4.5.2.1	Interruptions with ProSe Direct Discovery	156

4.5.2.2	Interruptions with ProSe Direct Communication	156

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	157

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	157

4.6	Cell Selection and Re-selection Requirements for UE category NB1	158

4.6.1	Cell Selection	158

4.6.2	Cell Re-selection	158

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	158

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	159

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	160

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	161

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	163

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	164

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	165

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	166

4.6.2.7	Maximum interruption in paging reception in normal coverage	168

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	168

4.6.2.8	UE measurement capability	168

4.6.2.9	WUS receptions for NB1	168

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	169

4.6.3.1	Introduction	169

4.6.3.2	Requirements on UE synchronization for transmission using PUR	169

4.6.3.3	Requirements on TA validation for transmission using PUR	169

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	170

4.6A.1	Cell Selection	170

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	170

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	170

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	172

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	173

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	175

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	177

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	177

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	180

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	181

4.6A.2.7	Maximum interruption in paging reception in normal coverage	183

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	184

4.6A.2.8	UE measurement capability	184

4.6A.2.9	WUS receptions for NB1	184

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	185

4.6A.3.1	Introduction	185

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	185

4.6A.3.3	Requirements on TA validation for transmission using PUR	185

4.7	Cell Selection and Re-selection Requirements for UE category M1	185

4.7.1	Cell Selection	185

4.7.2	Cell Re-selection	185

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	186

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	186

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	186

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	187

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	189

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	191

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	191

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	192

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	192

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	193

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	194

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	197

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	198

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	199

4.7.2.3	WUS receptions for UE category M1	199

4.7.3	Channel quality report for UE Category M1 in idle mode	199

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	200

4.7.4.1	Introduction	200

4.7.4.2	Requirements on UE synchronization for transmission using PUR	200

4.7.4.3	Requirements on TA validation for transmission using PUR	201

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	201

4.7A.1	Cell Selection	202

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	202

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	202

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	202

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	204

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	205

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	208

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	210

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	210

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	210

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	210

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	212

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	214

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	216

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	219

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	219

4.7A.2.3	WUS receptions for UE category M1	219

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	219

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	220

4.7A.4.1	Introduction	220

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	220

4.7A.4.3	Requirements on TA validation for transmission using PUR	220

4.8	Idle State Positioning Measurement Requirements for UE category NB1	221

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	221

4.8.1.1	RSTD Measurement Reporting Delay	222

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	223

4.8.2.1	RSTD Measurement Reporting Delay	224

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	225

4.8.3.1	RSTD Measurement Reporting Delay	226

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	227

4.8.4.1	RSTD Measurement Reporting Delay	228

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	229

4.8.5.1	Measurement Reporting Delay	230

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	230

4.8.6.1	Measurement Reporting Delay	231

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	232

4.8.7.1	Measurement Reporting Delay	233

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	233

4.8.8.1	Measurement Reporting Delay	234

4.9	Idle Mode CA Measurement	235

4.9.1	Introduction	235

4.9.2	Requirement	235

4.9.2.1	Detected cell requirement during state transition and Idle mode	235

4.9.2.2	Measurements of inter-frequency CA candidate cells	236

4.9.2.3	Measurements on serving cell	236

4A	E-UTRAN RRC_INACTIVE state mobility	237

4A.1	Cell Re-selection	237

4A.1.1	Introduction	238

4A.1.2	Requirements	238

4A.1.2.1	UE measurement capability	238

4A.1.2.2	Measurement and evaluation of serving cell	238

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	238

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	238

4A.1.2.5	Evaluation of cell re-selection criteria	238

4A.1.2.6	Maximum interruption in paging reception	238

4A.1.2.7	Measurements of inter-RAT NR cells	238

4A.1.2.8	UE measurement capability for RedCap	238

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	238

4A.2	Requirements for UE Category M1	239

4A.2.1	Introduction	239

4A.2.2	Cell Selection	239

4A.2.3	Cell Reselection	239

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	239

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	239

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	240

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	240

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	240

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	240

4A.2.4	Channel quality report for UE Category M1 in idle mode	242

5	E-UTRAN RRC_CONNECTED state mobility	242

5.1	E-UTRAN Handover	243

5.1.1	Introduction	243

5.1.2	Requirements	243

5.1.2.1	E-UTRAN FDD – FDD	243

5.1.2.1.1	Handover delay	243

5.1.2.1.2	Interruption time	243

5.1.2.2	E-UTRAN FDD – TDD	244

5.1.2.2.1	(Void)	245

5.1.2.2.2	(Void)	245

5.1.2.3	E-UTRAN TDD – FDD	245

5.1.2.3.1	(Void)	245

5.1.2.3.2	(Void)	245

5.1.2.4	E-UTRAN TDD – TDD	245

5.1.2.4.1	Handover delay	245

5.1.2.4.2	Interruption time	245

5.1.2.5	E-UTRAN HD–FDD	247

5.1.2.5.1	Handover delay	247

5.1.2.5.2	Interruption time	247

5.1.2.6	E-UTRAN FDD – FDD conditional handover	248

5.1.2.6.1	Handover delay	248

5.1.2.6.2	Measurement time	249

5.1.2.6.3	Preparation time	249

5.1.2.6.4	Interruption time	249

5.1.2.7	E-UTRAN FDD – TDD conditional handover	249

5.1.2.8	E-UTRAN TDD – FDD conditional handover	250

5.1.2.9	E-UTRAN TDD – TDD conditional handover	250

5.2	Void	250

5.3	Handover to other RATs	250

5.3.1	E-UTRAN - UTRAN FDD Handover	250

5.3.1.1	Introduction	250

5.3.1.1.1	Handover delay	250

5.3.1.1.2	Interruption time	250

5.3.2	E-UTRAN - UTRAN TDD Handover	251

5.3.2.1	Introduction	251

5.3.2.2	Requirements	251

5.3.2.2.1	Handover delay	251

5.3.2.2.2	Interruption time	251

5.3.3	E-UTRAN - GSM Handover	252

5.3.3.1	Introduction	252

5.3.3.2	Requirements	252

5.3.3.2.1	Handover delay	252

5.3.3.2.2	Interruption time	252

5.3.4	E-UTRAN - NR FR1 Handover	252

5.3.4.1	Introduction	252

5.3.4.2	Handover delay	253

5.3.4.3	Interruption time	253

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	253

5.3.4A.1	Introduction	253

5.3.4A.2	Handover delay	254

5.3.4A.3	Interruption time	254

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	255

5.3.4B.1	Introduction	255

5.3.4B.2	Requirements	255

5.3.5	E-UTRAN - NR FR2 Handover	255

5.3.5.1	Introduction	255

5.3.5.2	Handover delay	255

5.3.5.3	Interruption time	255

5.4	Handover to Non-3GPP RATs	256

5.4.1	E-UTRAN – HRPD Handover	256

5.4.1.1	Introduction	256

5.4.1.1.1	Handover delay	256

5.4.1.1.2	Interruption time	256

5.4.2	E-UTRAN – cdma2000 1X Handover	257

5.4.2.1	Introduction	257

5.4.2.1.1	Handover delay	257

5.4.2.1.2	Interruption time	257

5.5	E-UTRAN Handover for Cat-M1 UEs	257

5.5.1	Introduction	257

5.5.2	Requirements in CEModeA	258

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	258

5.5.2.1.1	Handover delay	258

5.5.2.1.2	Interruption time	258

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	258

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	259

5.5.2.3.1	Void	259

5.5.2.3.2	Void	259

5.5.3	Requirements in CEModeB	259

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	259

5.5.3.1.1	Handover delay	259

5.5.3.1.2	Interruption time	259

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	260

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	260

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	260

5.5A.1	Introduction	260

5.5A.2	Requirements in CEModeA	260

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	260

5.5A.2.1.1	Handover delay	260

5.5A.2.1.2	Interruption time	260

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	261

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	261

5.5A.2.3.1	Handover delay	261

5.5A.2.3.2	Measurement time	261

5.5A.2.3.3	Preparation time	262

5.5A.2.3.4	Interruption time	262

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	262

5.5A.3	Requirements in CEModeB	263

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	263

5.5A.3.1.1	Handover delay	263

5.5A.3.1.2	Interruption time	263

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	264

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	264

5.5A.3.3.1	Handover delay	264

5.5A.3.3.2	Measurement time	264

5.5A.3.3.3	Preparation time	264

5.5A.3.3.4	Interruption time	265

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	265

5.6	Void	265

5.7	E-UTRAN DAPS Handover	265

5.7.1	Introduction	265

5.7.2	Requirements	266

5.7.2.1	E-UTRAN FDD – FDD	266

5.7.2.1.1	DAPS Handover delay	266

5.7.2.1.2	Interruption time	267

5.7.2.2	E-UTRAN FDD – TDD	267

5.7.2.3	E-UTRAN TDD – FDD	267

5.7.2.4	E-UTRAN TDD – TDD	267

5.8	EN-DC Handover with PSCell	267

5.8.1	Introduction	267

5.8.1.1	Handover with PSCell Interruption time	268

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	268

5.9	EN-DC Handover with PSCell using CCA	269

5.9.1	Introduction	269

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	269

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	270

6	RRC Connection Mobility Control	270

6.1	RRC Re-establishment	270

6.1.1	Introduction	271

6.1.2	Requirements	271

6.1.2.1	UE Re-establishment delay requirement	271

6.2	Random Access	271

6.2.1	Introduction	271

6.2.2	Requirements	271

6.2.2.1	Contention based random access	272

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	272

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	272

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	272

6.2.2.1.4	Void	272

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	272

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	272

6.2.2.2	Non-Contention based random access	272

6.2.2.2.1	Correct behaviour when receiving Random Access Response	272

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	272

6.2.3	Requirements for Cat-M1 UEs	273

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	273

6.3	RRC Connection Release with Redirection	273

6.3.1	Introduction	273

6.3.2	Requirements	273

6.3.2.1	RRC connection release with redirection to UTRAN FDD	273

6.3.2.2	RRC connection release with redirection to GERAN	274

6.3.2.3	RRC connection release with redirection to UTRAN TDD	274

6.3.2.4	RRC connection release with redirection to NR	275

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	275

6.3.2.6	RRC connection release with redirection to NR Redcap	276

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	278

6.4.1	Introduction	278

6.4.2	Requirements	278

6.5	RRC Re-establishment for NB-IoT UEs	278

6.5.1	Introduction	278

6.5.2	Requirements	278

6.5.2.1	UE Re-establishment delay requirement in normal coverage	278

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	279

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	279

6.5A.1	Introduction	279

6.5A.2	Requirements	280

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	280

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	280

6.6	Random Access for UE category NB1	281

6.6.1	Introduction	281

6.6.2	Requirements	281

6.6.2.1	Correct behaviour when receiving Random Access Response reception	281

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	282

6.6.2.3	Correct behaviour when receiving a NACK on msg3	282

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	282

6.6.2.5	Correct behaviour when contention Resolution timer expires	282

6.6.2.6	MSG3-based channel quality report for UE Category NB1	282

6.6.3	Requirements for NPRACH configuration	282

6.6A	Random Access for UE category NB-IoT for Satellite Access	283

6.6A.1	Introduction	283

6.6A.2	Requirements	283

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	283

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	283

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	283

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	283

6.6A.2.5	Correct behaviour when contention Resolution timer expires	284

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	284

6.6A.3	Requirements for NPRACH configuration	284

6.7	RRC Re-establishment for Cat-M1 UEs	285

6.7.1	Introduction	285

6.7.2	Requirements	285

6.7.2.1	UE Re-establishment delay requirement for CEModeA	285

6.7.2.2	UE Re-establishment delay requirement for CEModeB	285

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	286

6.7A.1	Introduction	286

6.7A.2	Requirements	286

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	287

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	287

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	288

6.8.1	Introduction	288

6.8.2	Requirements	288

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	288

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	289

6.8A.1	Introduction	289

6.8A.2	Requirements	289

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	289

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	290

6.9.1	Introduction	290

6.9.2	Requirements	290

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	291

6.9A.1	Introduction	291

7	Timing and signalling characteristics	292

7.1	UE transmit timing	292

7.1.1	Introduction	292

7.1.2	Requirements	292

7.2	UE timer accuracy	293

7.2.1	Introduction	293

7.2.2	Requirements	293

7.3	Timing Advance	294

7.3.1	Introduction	294

7.3.2	Requirements	294

7.3.2.1	Timing Advance adjustment delay	294

7.3.2.2	Timing Advance adjustment accuracy	294

7.4	Cell phase synchronization accuracy (TDD)	294

7.4.1	Definition	294

7.4.2	Minimum requirements	295

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	295

7.5.1	Introduction	295

7.5.2	eNodeB Synchronization Requirements	295

7.5.2.1	Synchronized E-UTRAN	295

7.5.2.2	Non-Synchronized E-UTRAN	296

7.6	Radio Link Monitoring	296

7.6.1	Introduction	296

7.6.2	Requirements	297

7.6.2.1	Minimum requirement when no DRX is used	297

7.6.2.2	Minimum requirement when DRX is used	298

7.6.2.3	Minimum requirement at transitions	299

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	299

7.6.2.5	Minimum requirement under IDC Interference	300

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	300

7.7.1	Introduction	300

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	300

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	302

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	303

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	304

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	305

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	306

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	306

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	306

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	307

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	308

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	309

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	310

7.7.14	SCell Activation Delay Requirement for Dormant SCell	310

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	311

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	312

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	314

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	314

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	316

7.8	Interruptions with Carrier Aggregation	318

7.8.1	Introduction	318

7.8.2	Requirements	318

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	318

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	318

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	318

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	318

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	319

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	319

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	319

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	319

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	320

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	321

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	321

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	321

7.8.2.13	Interruptions at SRS carrier based switching	322

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	322

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	323

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	323

7.8.2.17	Interruptions during CQI measurement on dormant SCell	323

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	323

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	324

7.8.2.20	Interruptions at SCell hibernation	324

7.8.2.21	Interruptions at direct SCell activation and hibernation	324

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	325

7.9.1	Introduction	325

7.9.2	Minimum Requirements for Interband Carrier Aggregation	325

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	325

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	326

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	326

7.10.1	Introduction	326

7.10.2	Requirements	326

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	326

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	326

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	327

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	327

7.11	Radio Link Monitoring for UE Category 0	327

7.11.1	Introduction	327

7.11.2	Requirements for FD-FDD and TDD	329

7.11.2.1	Minimum requirement when no DRX is used	329

7.11.2.2	Minimum requirement when DRX is used	329

7.11.2.3	Minimum requirement at transitions	330

7.11.3	Requirements for HD-FDD	330

7.11.3.1	Minimum requirement when no DRX is used	330

7.11.3.2	Minimum requirement when DRX is used	330

7.11.3.3	Minimum requirement at transitions	331

7.12	Interruptions with Dual Connectivity	331

7.12.1	Introduction	331

7.12.2	Requirements	331

7.12.2.1	Interruptions at PSCell addition/release	331

7.12.2.2	Interruptions at transitions between active and non-active during DRX	332

7.12.2.3	Interruptions at transitions from non-DRX to DRX	332

7.12.2.4	Interruptions at SCell addition/release	332

7.12.2.5	Interruptions at SCell activation/deactivation	332

7.12.2.6	Interruptions during measurements on SCC	333

7.12.2.7	Interruptions at SRS carrier based switching	333

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	334

7.13.1	Definition	334

7.13.2	Minimum requirements	334

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	334

7.14.1	Introduction	334

7.14.2	PSCell Addition Delay Requirement	334

7.14.3	PSCell Release Delay Requirement	335

7.15	Maximum Receive Timing Difference in Dual Connectivity	335

7.15.1	Introduction	335

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	335

7.16	Proximity-based Services	336

7.16.1	Introduction	336

7.16.2	Requirements	336

7.16.2.1	ProSe UE transmission timing	336

7.16.2.1.1	Serving cell or PCell as timing reference	336

7.16.2.1.2	SCell or non-serving cell as timing reference	336

7.16.3	Interruptions with ProSe	336

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	337

7.16.3.2	Interruptions at ProSe Direct Communication configuration	337

7.16.3.3	Interruptions during ProSe Direct Discovery	337

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	337

7.16.3.5	Interruptions during ProSe Direct Communication	338

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	338

7.16.4.1	Measurement and evaluation of selected cell	338

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	338

7.16.5	Selection / Reselection of ProSe relay UE	339

7.16.6	ProSe operation under deactivated SCell	339

7.17	Maximum Transmission Timing Difference in Dual Connectivity	340

7.17.1	Introduction	340

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	340

7.18.1	Introduction	340

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	340

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	340

7.19	Radio Link Monitoring for UE Category M1	340

7.19.1	Introduction	340

7.19.2	Requirements for FD-FDD and TDD CE mode A	341

7.19.2.1	Minimum requirement when no DRX is used	343

7.19.2.2	Minimum requirement when DRX is used	344

7.19.2.3	Minimum requirement at transitions	345

7.19.3	Requirements for HD-FDD with CE mode A	345

7.19.3.1	Minimum requirement when no DRX is used	345

7.19.3.2	Minimum requirement when DRX is used	345

7.19.3.3	Minimum requirement at transitions	347

7.19.4	Requirements for FD-FDD and TDD with CE mode B	347

7.19.4.1	Minimum requirement when no DRX is used	349

7.19.4.2	Minimum requirement when DRX is used	350

7.19.4.3	Minimum requirement at transitions	351

7.19.5	Requirements for HD-FDD with CE mode B	351

7.19.5.1	Minimum requirement when no DRX is used	351

7.19.5.2	Minimum requirement when DRX is used	351

7.19.5.3	Minimum requirement at transitions	353

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	353

7.19A.1	Introduction	353

7.19A.2	Requirements for FD-FDD and CE mode A	353

7.19A.2.1	Minimum requirement when no DRX is used	355

7.19A.2.2	Minimum requirement when DRX is used	356

7.19A.2.3	Minimum requirement at transitions	357

7.19A.3	Requirements for HD-FDD with CE mode A	357

7.19A.3.1	Minimum requirement when no DRX is used	357

7.19A.3.2	Minimum requirement when DRX is used	357

7.19A.3.3	Minimum requirement at transitions	359

7.19A.4	Requirements for HD-FDD with CE mode B	359

7.19A.4.1	Minimum requirement when no DRX is used	359

7.19A.4.2	Minimum requirement when DRX is used	359

7.19A.4.3	Minimum requirement at transitions	360

7.20	UE transmit timing for NB-IoT	360

7.20.1	Introduction	360

7.20.2	Requirements	361

7.20A	UE transmit timing for NB-IoT for Satellite Access	361

7.20A.1	Introduction	361

7.20A.2	Requirements	361

7.21	 UE timer accuracy for NB-IoT	362

7.21.1	Introduction	362

7.21.2	Requirements	362

7.21A	UE timer accuracy for NB-IoT for Satellite Access	363

7.21A.1	Introduction	363

7.21A.2	Requirements	363

7.22	Timing Advance for NB-IoT	363

7.22.1	Introduction	363

7.22.2	Requirements	363

7.22.2.1	Timing Advance adjustment delay	363

7.22.2.2	Timing Advance adjustment accuracy	363

7.22A	Timing Advance for NB-IoT for Satellite Access	364

7.22A.1	Introduction	364

7.22A.2	Requirements	364

7.22A.2.1	Timing Advance adjustment delay	364

7.22A.2.2	Timing Advance adjustment accuracy	364

7.23	Radio Link Monitoring for Category NB1 UE	364

7.23.1	Introduction	364

7.23.2	Requirements for Category NB1 UE	364

7.23.2.1	Minimum requirement when no DRX is used	365

7.23.2.2	Minimum requirement when DRX is used	365

7.23.2.3	Minimum requirement at transitions	366

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	366

7.23A.1	Introduction	366

7.23A.2	Requirements for Category NB1 UE	366

7.23A.2.1	Minimum requirement when no DRX is used	367

7.23A.2.2	Minimum requirement when DRX is used	367

7.23A.2.3	Minimum requirement at transitions	368

7.24	UE transmit timing for Category M1	368

7.24.1	Introduction	368

7.24.2	Requirements	368

7.24A	UE transmit timing for Category M1 for Satellite Access	369

7.24A.1	Introduction	369

7.24A.2	Requirements	369

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	370

7.25.1	Definition	370

7.25.2	Minimum requirements	370

7.26	UE transmit timing for Category M2	371

7.26.1	Introduction	371

7.26.2	Requirements	371

7.27	UE timer accuracy for category M1	371

7.27.1	Introduction	371

7.27.2	Requirements	371

7.27A	UE timer accuracy for category M1 for Satellite Access	371

7.27A.1	Introduction	371

7.27A.2	Requirements	371

7.28	Timing Advance for Category M1	372

7.28.1	Introduction	372

7.28.2	Requirements	372

7.28A	Timing Advance for Category M1 for Satellite Access	372

7.28A.1	Introduction	372

7.28A.2	Requirements	372

7.28A.2.1	Timing Advance adjustment delay	372

7.28A.2.2	Timing Advance adjustment accuracy	372

7.29	Interruptions requirements with FeMBMS	372

7.29.1	Introduction	372

7.29.2	Requirements	372

7.30	Numerology switching delay requirements with FeMBMS	373

7.30.1	Introduction	373

7.30.2	Requirements	373

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	373

7.31.1	Introduction	373

7.31.2	NR PSCell Addition Delay Requirement	373

7.31.3	NR PSCell Release Delay Requirement	374

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	374

7.31A.1	Introduction	374

7.31A.2	NR PSCell Addition Delay Requirement	374

7.31A.3	NR PSCell Release Delay Requirement	375

7.32	Interruptions with EN-DC	376

7.32.1	Introduction	376

7.32.2	Requirements	376

7.32.2.1	Interruptions at PSCell addition/release	376

7.32.2.2	Interruptions at transitions between active and non-active during DRX	376

7.32.2.3	Interruptions at transitions from non-DRX to DRX	377

7.32.2.4	Interruptions at SCell addition/release	377

7.32.2.5	Interruptions at SCell activation/deactivation	377

7.32.2.6	Interruptions during measurements on SCC	378

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	378

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	378

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	378

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	378

7.32.2.7	Interruptions at active BWP switching	378

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	379

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	379

7.32.2.10	Interruptions at SCell hibernation	379

7.32.2.11	Interruptions at direct SCell activation and hibernation	379

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	380

7.32.2.13	Interruptions at NR SRS carrier based switching	380

7.32.2.14	Interruptions at NR SCell dormancy	381

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	381

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	381

7.32.2.15	Interruption during NR measurement with autonomous gaps	381

7.32.2.16	Interruptions at SRS carrier based switching	382

7.32.2.17	Interruptions at SCG activation/deactivation	382

7.32.2.18	Interruptions due to NR SRS antenna port switching	382

7.32.2.19	Interruptions at fast SCell activation/deactivation	383

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	383

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	384

7.33.1	Introduction	384

7.33.2	Requirements	384

7.34	Void	384

7.35	Interruptions with SFTD measurements	384

7.35.1	Introduction	384

7.35.2	Requirements	384

7.36	Interruptions with NE-DC	385

7.36.1	Introduction	385

7.36.2	Requirements	385

7.36.2.1	Interruptions at transitions between active and non-active during DRX	385

7.36.2.2	Interruptions at transitions from non-DRX to DRX	385

7.36.2.3	Interruptions at SCell addition/release	385

7.36.2.4	Interruptions at SCell activation/deactivation	386

7.36.2.5	Interruptions during measurements on SCC	386

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	386

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	386

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	386

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	387

7.36.2.6	Interruptions at active BWP switching	387

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	388

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	388

7.36.2.9	Interruptions at SCell hibernation	388

7.36.2.10	Interruptions at direct SCell activation and hibernation	388

7.36.2.11	Interruptions at NR SRS carrier based switching	388

7.36.2.12	Interruptions at NR SCell dormancy	389

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	389

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	389

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	389

7.36.2.14	Interruption during NR measurement with autonomous gaps	390

7.36.2.15	Interruptions at SRS carrier based switching	390

7.36.2.16	Interruptions due to NR SRS antenna port switching	391

7.37	Interruptions during NR measurement with autonomous gaps	391

7.37.1	Introduction	391

7.37.2	Requirements	392

7.38 SCG Activation and Deactivation Delay	392

7.38.1	Introduction	392

7.38.2	SCG Activation Delay Requirement	392

7.38.3	SCG Deactivation Delay Requirement	393

8	UE Measurements Procedures in RRC_CONNECTED State	393

8.1	General Measurement Requirements	393

8.1.1	Introduction	393

8.1.2	Requirements	394

8.1.2.1	UE measurement capability	394

8.1.2.1.1	Monitoring of multiple layers using gaps	402

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	403

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	404

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	406

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	407

8.1.2.1.2	Network controlled small gap	407

8.1.2.2	E-UTRAN intra frequency measurements	409

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	409

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	414

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	418

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	419

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	420

8.1.2.3	E-UTRAN inter frequency measurements	420

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	421

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	426

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	433

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	433

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	433

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	434

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	436

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	437

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	438

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	445

8.1.2.4	Inter RAT measurements	445

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	445

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	450

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	450

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	454

8.1.2.4.5	E-UTRAN FDD – GSM measurements	454

8.1.2.4.6	E-UTRAN TDD – GSM measurements	459

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	459

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	461

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	461

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	461

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	462

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	462

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	462

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	462

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	464

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	464

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	464

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	464

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	465

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	465

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	467

8.1.2.4.21	E-UTRAN FDD – NR measurements	467

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	471

8.1.2.4.22	E-UTRAN TDD – NR measurements	475

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	475

8.1.2.4.23	Void	475

8.1.2.4.24	Void	475

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	475

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	477

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	477

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	478

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	478

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	479

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	480

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	482

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	484

8.1.2.6.5	Void	486

8.1.2.6.6	Void	486

8.1.2.6.7	Void	486

8.1.2.6.8	Void	486

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	486

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	486

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	488

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	490

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	492

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	493

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	495

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	497

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	499

8.1.2.7	E-UTRAN E-CID Measurements	500

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	500

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	502

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	503

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	504

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	504

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	504

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	507

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	510

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	514

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	517

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	517

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	517

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	518

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	518

8.1.2.10	Void	519

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	519

8.2.1	Introduction	519

8.2.2	Requirements	519

8.3	Measurements for E-UTRA carrier aggregation	523

8.3.1	Introduction	523

8.3.2	Measurements of the primary component carrier	523

8.3.3	Measurements of a secondary component carrier	523

8.3.3.1	Measurements of a secondary component carrier with active SCell	523

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	524

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	524

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	525

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	527

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	527

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	527

8.4.1	Introduction	527

8.4.2	Measurements on the primary component carrier	528

8.4.3	Measurements on a secondary component carrier	529

8.4.4	Measurements on both primary component carrier and a secondary component carrier	529

8.4.5	Measurements on different secondary component carriers	530

8.5	Measurements for UE category 0	531

8.5.1	Introduction	531

8.5.2 	Requirements	532

8.5.2.1 	E-UTRAN intra frequency measurements	532

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	532

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	535

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	537

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	541

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	542

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	542

8.6	Discovery signal measurements	543

8.6.1	Introduction	543

8.6.2	Requirements for CRS based discovery signal measurements	543

8.6.2.1	E-UTRAN intra frequency measurements	543

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	543

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	546

8.6.2.2	E-UTRAN inter frequency measurements	548

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	549

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	551

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	554

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	554

8.6.3	Requirements for CSI-RS based discovery signal measurements	554

8.6.3.1	E-UTRAN intra frequency measurements	554

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	555

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	557

8.6.3.2	E-UTRAN inter frequency measurements	559

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	560

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	562

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	565

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	565

8.7	Discovery signal measurements for E-UTRA carrier aggregation	565

8.7.1	Introduction	565

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	566

8.7.2.1	Measurements of the primary component carrier	566

8.7.2.2	Measurements of a secondary component carrier	566

8.7.2.3	Measurements of a secondary component carrier with active SCell	566

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	566

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	566

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	568

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	569

8.7.3.1	Measurements of the primary component carrier	569

8.7.3.2	Measurements of a secondary component carrier	569

8.7.3.3	Measurements of a secondary component carrier with active SCell	569

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	569

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	569

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	571

8.8	Measurements for E-UTRA dual connectivity	572

8.8.1	Introduction	572

8.8.2	Intra-frequency measurements requirements on PCell	573

8.8.3	Intra-frequency measurements requirements on PSCell	573

8.8.4	Inter-frequency and inter-RAT measurement requirements	573

8.8.5	Intra-frequency measurements with autonomous gaps	573

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	573

8.8.5.2	ECGI reporting delay	574

8.8.6	Inter-frequency measurements with autonomous gaps	574

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	574

8.8.6.2	ECGI reporting delay	575

8.8.7	SSTD Measurements	575

8.8.7.1	Introduction	575

8.8.7.2	SSTD Measurement requirements	575

8.8.7.3	SSTD Measurement Reporting Delay	576

8.8.8	Intra-frequency measurements requirements on SCell	576

8.9	MBSFN Measurements	576

8.9.1	Introduction	576

8.9.2	MBSFN RSRP Measurements	577

8.9.3	MBSFN RSRQ Measurements	577

8.9.4	MCH BLER Measurements	577

8.10	Proximity-based Services	577

8.10.1	Introduction	577

8.10.2	Requirements	577

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	577

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	578

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	579

8.11.1	Introduction	579

8.11.2	CRS based discovery signal measurements	579

8.11.2.1	E-UTRAN intra-frequency measurements	579

8.11.2.1.1	Requirements	579

8.11.2.1.1.1	Requirements when no DRX is used	579

8.11.2.1.1.1.1	Measurement Reporting Requirements	581

8.11.2.1.1.2	Requirements when DRX is used	582

8.11.2.1.1.2.1	Measurement Reporting Requirements	584

8.11.2.2	E-UTRAN inter-frequency measurements	585

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	585

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	589

8.11.3	CSI-RS based discovery signal measurements	589

8.11.3.1	E-UTRAN intra-frequency measurements	589

8.11.3.1.1	Requirements	589

8.11.3.1.1.1	Requirements when no DRX is used	589

8.11.3.1.1.1.1	Measurement Reporting Requirements	591

8.11.3.1.1.2	Requirements when DRX is used	591

8.11.3.1.1.2.1	Measurement Reporting Requirements	592

8.11.3.2	E-UTRAN inter-frequency measurements	593

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	593

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	597

8.11.4	RSSI measurements	597

8.11.4.1	E-UTRAN intra-frequency measurements	597

8.11.4.2	E-UTRAN inter-frequency measurements	597

8.11.5	Channel occupancy measurements	598

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	598

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	598

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	598

8.12.1	Introduction	598

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	598

8.12.2.1	Introduction	598

8.12.2.2	Measurements of a secondary component carrier	599

8.12.2.3	Measurements of a secondary component carrier with active SCell	599

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	599

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	599

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	602

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	605

8.12.3.1	Introduction	605

8.12.3.2	Measurements of a secondary component carrier	605

8.12.3.3	Measurements of a secondary component carrier with active SCell	605

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	605

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	605

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	607

8.13	Measurements for UE Category M1	609

8.13.1	Introduction	609

8.13.2	Requirements for UE category M1 with CE mode A	609

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	610

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	610

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	615

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	617

8.13.2.2	Void	621

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	621

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	621

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	624

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	626

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	627

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	627

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	629

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	631

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	632

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	632

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	632

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	633

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	633

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	633

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	634

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	634

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	635

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	636

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	636

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	637

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	641

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	643

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	647

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	647

8.13.3	Requirements for UE category M1 with CE mode B	648

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	648

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	649

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	653

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	656

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	661

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	662

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	662

8.13.3.2	Void	663

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	663

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	663

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	666

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	668

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	669

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	669

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	669

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	670

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	670

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	670

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	671

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	671

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	671

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	676

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	678

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	683

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	683

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	683

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	685

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	688

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	688

8.13A	Measurements for UE Category M1 for Satellite Access	689

8.13A.1	Introduction	689

8.13A.2	Requirements for UE category M1 with CE mode A	689

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	689

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	690

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	690

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	692

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	694

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	694

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	694

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	696

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	696

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	696

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	698

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	700

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	700

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	700

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	702

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	702

8.13A.3	Requirements for UE category M1 with CE mode B	703

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	703

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	704

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	704

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	706

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	708

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	708

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	708

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	710

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	710

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	710

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	712

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	714

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	714

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	714

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	716

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	716

8.14	Measurements for UE category NB1	717

8.14.1	Introduction	717

8.14.2	NB-IoT intra frequency measurements under normal coverage	717

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	717

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	717

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	718

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	718

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	718

8.14.4	Connected mode channel quality report for UE Category NB1	718

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	718

8.14.6	NB-IoT neighbour cell measurements	719

8.14.6.1	Introduction	719

8.14.6.2	Requirements	719

8.14.6.3	Intra-frequency neighbour cell measurements	719

8.14.6.4	Inter-frequency neighbour cell measurements	720

8.14.6.5	 Requirements for monitoring multiple carriers	721

8.14A	Measurements for UE category NB-IoT for Satellite Access	721

8.14A.1	Introduction	721

8.14A.2	NB-IoT intra frequency measurements under normal coverage	722

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	722

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	722

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	722

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	722

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	722

8.14A.4	Connected mode channel quality report for UE Category NB1	722

8.15	Void	723

8.16	Measurements for UE Category M2	723

8.16.1	Introduction	723

8.16.2	Requirements for UE category M2 with CE mode A	723

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	723

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	724

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	724

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	725

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	725

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	725

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	726

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	726

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	728

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	731

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	731

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	732

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	734

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	736

8.16.3	Requirements for UE category M2 with CE mode B	737

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	737

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	737

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	739

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	742

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	742

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	743

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	745

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	747

8.17	Measurements for E-UTRA – NR Dual Connectivity	748

8.17.1	Introduction	748

8.17.1.1	Measurement Gap Sharing	748

8.17.1A	Intrafrequency Measurements	748

8.17.2	SFTD Measurements	749

8.17.2.1	Introduction	749

8.17.2.2	SFTD Measurement requirements	749

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	749

8.17.2.3	SFTD Measurement Reporting Delay	751

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	751

8.17.3.1	Introduction	751

8.17.3.2	E-UTRAN FDD inter frequency measurements	751

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	751

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	753

8.17.3.3	E-UTRAN TDD inter frequency measurements	755

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	755

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	756

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	758

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	758

8.17.4.1.1	NR Inter-RAT cell identification	759

8.17.4.1.2	NR Inter-RAT measurement	760

8.17.4.1.3	NR Inter-RAT measurement reporting	761

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	762

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	762

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	762

8.17.4A.1.1	NR Inter-RAT cell identification	763

8.17.4A.1.2	NR Inter-RAT measurement	764

8.17.4A.1.3	NR Inter-RAT measurement reporting	765

8.17.4A.1.4	NR inter-RAT RSSI measurements	765

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	766

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	766

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	767

8.17.5.1	Introduction	767

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	767

8.17.5.2.1	Identification of a new UTRA FDD cell	767

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	767

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	767

8.17.5.2.4	Periodic Reporting	768

8.17.5.2.5	Event Triggered Reporting	768

8.17.5.2.6	Event-triggered Periodic Reporting	768

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	769

8.17.5.3.1	Periodic Reporting	770

8.17.5.3.2	Event Triggered Reporting	770

8.17.5.3.3	Event-triggered Periodic Reporting	770

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	771

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	771

8.17.7.1	Introduction	771

8.17.7.2	Identification of a new UTRA FDD cell for SON	771

8.17.7.2.1	Requirements when no DRX is used	771

8.17.7.2.2	Requirements when DRX is used	771

8.17.7.2.3	Reporting Delay	772

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	773

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	773

8.17.9.1	Introduction	773

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	773

8.17.9.2.1	Identification of a new UTRA FDD cell	773

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	773

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	774

8.17.9.2.4	Periodic Reporting	774

8.17.9.2.5	Event Triggered Reporting	774

8.17.9.2.6	Event-triggered Periodic Reporting	775

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	775

8.17.9.3.1	Periodic Reporting	776

8.17.9.3.2	Event Triggered Reporting	776

8.17.9.3.3	Event-triggered Periodic Reporting	776

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	777

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	777

8.17.11.1	Introduction	777

8.17.11.2	Identification of a new UTRA TDD cell for SON	777

8.17.11.2.1	Requirements when no DRX is used	777

8.17.11.2.2	Requirements when DRX is used	777

8.17.11.2.3	Reporting Delay	778

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	778

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	779

8.17.13.1	Introduction	779

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	779

8.17.13.2.1	GSM carrier RSSI	779

8.17.13.2.2	BSIC verification	779

8.17.13.2.3	Enhanced BSIC verification	781

8.17.13.2.4	Periodic Reporting	781

8.17.13.2.5	Event Triggered Reporting	781

8.17.13.2.6	Event-triggered Periodic Reporting	782

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	782

8.17.13.3.1	GSM carrier RSSI	782

8.17.13.3.2	BSIC verification	783

8.17.13.3.3	Periodic Reporting	784

8.17.13.3.4	Event Triggered Reporting	784

8.17.13.3.5	Event-triggered Periodic Reporting	784

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	784

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	784

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	784

8.17.15.1.1	RSTD Measurement Reporting Delay	785

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	785

8.17.15.2.1	RSTD Measurement Reporting Delay	785

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	785

8.17.15.3.1	RSTD Measurement Reporting Delay	786

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	786

8.17.15.4.1	RSTD Measurement Reporting Delay	786

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	787

8.17.16.1	Introduction	787

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	787

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	787

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	787

8.17.17.1	Introduction	787

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	787

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	787

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	787

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	787

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	787

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	787

8.18	Measurements for non-BL/CE UE	787

8.18.1	Introduction	787

8.18.2	Requirements for non-BL/CE UE with CE Mode B	788

8.18.2.1	E-UTRAN intra frequency measurements	788

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	788

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	788

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	789

8.19	Measurements for NR – E-UTRA Dual Connectivity	790

8.19.1	Introduction	790

8.19.2	Intra-frequency Measurements	790

8.19.3	Inter-frequency Measurements	790

8.19.4	Void	790

8.19.5	Intra-frequency E-CID Measurements	790

8.19.6	Intra-frequency measurements with autonomous gaps	791

8.19.6.1	Introduction	791

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	791

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	791

8.19.7	Inter-frequency measurements with autonomous gaps	791

8.19.7.1	Introduction	791

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	791

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	791

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	791

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	791

8.20	Inter-RAT NR Measurements for RedCap UE	791

8.20.1	Introduction	791

8.20.2	E-UTRAN FDD – NR measurements	792

8.20.2.1	Identification of a new NR cell	792

8.20.2.2	Periodic Reporting	795

8.20.2.3	Event Triggered Reporting	795

8.20.2.4	Event-triggered Periodic Reporting	795

8.20.3	E-UTRAN TDD – NR measurements	795

9	Measurements performance requirements for UE	796

9.1	E-UTRAN measurements	796

9.1.1	Introduction	796

9.1.2	Intra-frequency RSRP Accuracy Requirements	796

9.1.2.1	Absolute RSRP Accuracy	796

9.1.2.2	Relative Accuracy of RSRP	797

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	798

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	798

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	799

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	800

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	801

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	802

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	803

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	803

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	803

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	804

9.1.2B.1	Introduction	804

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	804

9.1.3	Inter-frequency RSRP Accuracy Requirements	805

9.1.3.1	Absolute RSRP Accuracy	805

9.1.3.2	Relative Accuracy of RSRP	806

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	807

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	807

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	808

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	808

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	809

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	810

9.1.3B.1	Introduction	810

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	810

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	810

9.1.4	RSRP Measurement Report Mapping	811

9.1.5	Intra-frequency RSRQ Accuracy Requirements	811

9.1.5.1	Absolute RSRQ Accuracy	811

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	812

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	813

9.1.5.4	Absolute WB-RSRQ Accuracy	814

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	815

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	816

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	816

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	816

9.1.5B.1	Introduction	816

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	816

9.1.6	Inter-frequency RSRQ Accuracy Requirements	817

9.1.6.1	Absolute RSRQ Accuracy	817

9.1.6.2	Relative Accuracy of RSRQ	818

9.1.6.3	Absolute WB-RSRQ Accuracy	819

9.1.6.4	Relative WB-RSRQ Accuracy	819

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	820

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	821

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	821

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	821

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	822

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	823

9.1.6B.1	Introduction	823

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	823

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	823

9.1.7	RSRQ Measurement Report Mapping	824

9.1.8	Power Headroom	824

9.1.8.1	Period	825

9.1.8.2	Reporting Delay	825

9.1.8.3	Void	825

9.1.8.4	Report Mapping	825

9.1.8A	Power Headroom for UE category M1 for satellite access	825

9.1.8A.1	Period	826

9.1.8A.2	Reporting Delay	826

9.1.8A.3	Void	826

9.1.8A.4	Report Mapping	826

9.1.9	UE Rx – Tx time difference	826

9.1.9.1	Measurement Requirement	826

9.1.9.2	Measurement Report mapping	827

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	828

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	829

9.1.10	Reference Signal Time Difference (RSTD)	830

9.1.10.1	Intra-Frequency Accuracy Requirement	830

9.1.10.2	Inter-Frequency Accuracy Requirement	831

9.1.10.3	RSTD Measurement Report Mapping	833

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	833

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	834

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	835

9.1.11	Carrier aggregation measurement accuracy	836

9.1.11.1	Primary component carrier accuracy requirement	837

9.1.11.2	Secondary component carrier accuracy requirement	837

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	837

9.1.11.4	Secondary component carrier relative accuracy requirement	837

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	837

9.1.13	Measurement accuracy for UE category 0	838

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	838

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	838

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	839

9.1.14	Accuracy requirements for Discovery Signal Measurements	840

9.1.14.1	Introduction	840

9.1.14.2	RSRP measurements in discovery signal occasions	840

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	840

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	840

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	840

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	841

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	842

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	842

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	842

9.1.14.3.3	CSI-RSRP measurement report mapping	843

9.1.14.4	RSRQ measurements in discovery signal occasions	843

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	843

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	844

9.1.15.1.1	Primary component carrier accuracy requirement	844

9.1.15.1.2	Secondary component carrier accuracy requirement	844

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	844

9.1.15.1.4	Secondary component carrier relative accuracy requirement	844

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	844

9.1.15.2.1	Primary component carrier accuracy requirement	844

9.1.15.2.2	Secondary component carrier accuracy requirement	844

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	844

9.1.15.2.4	Secondary component carrier relative accuracy requirement	844

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	845

9.1.17	RS-SINR Measurements	845

9.1.17.1	Measurement Report Mapping	845

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	846

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	846

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	846

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	846

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	847

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	848

9.1.18.1	Introduction	848

9.1.18.2	RSRP measurements	848

9.1.18.2.1	RSRP measurement report mapping	848

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	848

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	849

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	849

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	850

9.1.18.3	RSRQ measurements	850

9.1.18.3.1	RSRQ measurement report mapping	850

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	850

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	851

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	851

9.1.18.4	CSI-RSRP measurements	852

9.1.18.4.1	CSI-RSRP measurement report mapping	852

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	852

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	852

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	853

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	853

9.1.18.5	RSSI measurements	854

9.1.18.5.1	RSSI measurement report mapping	854

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	854

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	855

9.1.18.6	Channel occupancy measurements	855

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	855

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	855

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	856

9.1.19.1	Introduction	856

9.1.19.2	Accuracy requirements for measurements on SCC	856

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	856

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	856

9.1.20	SFN and Subframe Time Difference (SSTD)	857

9.1.20.1	SSTD Accuracy Requirement	857

9.1.20.2	SSTD Measurement Report Mapping	857

9.1.21	Measurement accuracy for UE category M1	858

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	858

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	860

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	861

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	863

9.1.21.5	RSRP Measurement Report Mapping	864

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	864

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	865

9.1.21.8	RSRQ Measurement Report Mapping	866

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	866

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	867

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	868

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	870

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	871

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	872

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	873

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	874

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	875

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	876

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	877

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	878

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	881

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	884

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	884

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	887

9.1.21A	Measurement accuracy for UE category M1 for satellite access	888

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	889

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	890

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	891

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	893

9.1.21A.5	RSRP Measurement Report Mapping	894

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	894

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	895

9.1.21A.8	RSRQ Measurement Report Mapping	896

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	896

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	897

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	898

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	899

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	900

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	901

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	902

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	903

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	904

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	904

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	906

9.1.22	Measurement accuracy for UE Category NB1	907

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	907

9.1.22.2	Void	908

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	908

9.1.22.4	Void	909

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	909

9.1.22.6	Void	910

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	910

9.1.22.8	Void	911

9.1.22.9	NRSRP Measurement Report Mapping	911

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	911

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	912

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	913

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	914

9.1.22.14	NRSRQ Measurement Report Mapping	915

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	916

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	916

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	917

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	917

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	917

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	918

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	919

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	920

9.1.22A.5	NRSRP Measurement Report Mapping	921

9.1.22A.6	NRSRQ Measurement Report Mapping	921

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	921

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	921

9.1.23	Power Headroom for UE Category NB1	922

9.1.23.1	Period	922

9.1.23.2	Reporting Delay	922

9.1.23.3	Report Mapping for UE Category NB1	922

9.1.23.3.1	Void	924

9.1.23.3.2	Void	924

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	924

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	925

9.1.23A.1	Period	925

9.1.23A.2	Reporting Delay	925

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	926

9.1.23A.3.1	Void	926

9.1.23A.3.2	Void	926

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	926

9.1.24	Void	927

9.1.25	Measurement accuracy for UE category M2	927

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	927

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	928

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	929

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	930

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	932

9.1.26	Measurement Accuracy for non-BL CE UE	933

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	934

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	936

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	936

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	938

9.1.26.5	RSRP Measurement Report Mapping	939

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	939

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	939

9.1.26.8	RSRQ Measurement Report Mapping	940

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	940

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	941

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	942

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	943

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	944

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	944

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	944

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	945

9.1.27	SFN and frame Timing Difference (SFTD)	946

9.1.27.1	SFTD Accuracy Requirement	946

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	948

9.1.28.1	SFTD Accuracy Requirement under CCA	948

9.2	UTRAN FDD Measurements	949

9.2.1	UTRAN FDD CPICH RSCP	949

9.2.2	Void	950

9.2.3	UTRAN FDD CPICH Ec/No	950

9.3	UTRAN TDD Measurements	950

9.3.1	UTRAN TDD P-CCPCH RSCP	951

9.3.2	Void	951

9.3.3	Void	951

9.4	GSM Measurements	951

9.4.1	GSM carrier RSSI	951

9.5	CDMA2000 1x RTT Measurements	951

9.5.1	CDMA2000 1x RTT Pilot Strength	951

9.6	PCMAX,c	952

9.6.1	Report Mapping	952

9.6.2 	Estimation Period	952

9.6.3	Reporting Delay	952

9.7	IEEE802.11 Measurements	952

9.7.1	WLAN RSSI	952

9.7.2	WLAN RSSI Measurement Report Mapping	952

9.8	MBSFN Measurements	953

9.8.1	Introduction	953

9.8.2	MBSFN RSRP	953

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	953

9.8.2.2	MBSFN RSRP measurement report mapping	954

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	954

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	955

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	955

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	955

9.8.3	MBSFN RSRQ	956

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	956

9.8.3.2	MBSFN RSRQ measurement report mapping	956

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	957

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	957

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	957

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	958

9.8.4	MCH BLER	958

9.8.4.1	Measurement report mapping for MCH BLER	958

9.8.4.2	Measurement report mapping for MCH Block Number	959

9.9	ProSe Measurements	960

9.9.1	Introduction	960

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	960

9.9.2.1	Absolute S-RSRP Accuracy	960

9.9.2.2	Relative Accuracy of S-RSRP	961

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	962

9.9.3.1	Absolute SD-RSRP Accuracy	962

9.9.3.2	Relative Accuracy of SD-RSRP	962

9.10	V2X Measurements	963

9.10.1	Introduction	963

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	963

9.10.2.1	Absolute S-RSRP Accuracy	963

9.10.2.2	Relative Accuracy of S-RSRP	964

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	965

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	965

9.10.4	S-RSSI Measurement Accuracy Requirements	965

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	965

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	966

9.11	NR Measurements	966

9.11.1	NR SS-RSRP Measurements	967

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	967

9.11.2	NR SS-RSRQ Measurements	967

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	967

9.11.3	NR SS-SINR Measurements	968

9.11.4	NR SS-RSRP Measurements under CCA	968

9.11.5	NR SS-RSRQ Measurements under CCA	968

9.11.6	NR SS-SINR Measurements under CCA	969

9.11.7	NR RSSI Measurements under CCA	969

9.11.8	NR Channel Occupancy Measurements under CCA	969

10	Measurements Performance Requirements for E-UTRAN	970

10.1	Received Interference Power	970

10.1.1	Absolute accuracy requirement	970

10.1.2	Relative accuracy requirement	970

10.1.3	Received Interference Power measurement report mapping	970

10.2	Angle of Arrival (AOA)	970

10.2.1	Range/mapping	970

10.3	Timing Advance (TADV)	971

10.3.1	Report mapping	971

11	ProSe Requirements in Any Cell Selection state	971

11.1	Introduction	971

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	972

11.2.1	Introduction	972

11.2.2	ProSe UE transmission timing	972

11.3	Initiation/Cease of SLSS Transmissions	972

11.3.1	Introduction	972

11.3.2	Requirements	972

11.4	Measurements for ProSe in Any Cell Selection State	972

11.4.1	Introduction	972

11.4.2	Requirements	973

11.4.2.1	E-UTRA FDD	973

11.4.2.2	E-UTRA TDD	973

11.5	Selection / Reselection of ProSe Synchronization Reference	973

11.5.1	Introduction	973

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	974

11.5.2.1	Introduction	974

11.5.2.2	Requirements	974

11.6	Void	974

11.7	Selection / Reselection of ProSe relay UE	974

11.7.1	Introduction	974

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	974

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	975

12.1	Introduction	975

12.2	Transmit Timing	975

12.2.1	GNSS as timing reference	975

12.3	Interruption	975

12.4	Reliability of GNSS signal	975

13	V2X Requirements	976

13.1	Introduction	976

13.2	UE Transmit Timing	976

13.2.1	Introduction	976

13.2.2	GNSS as synchronization reference source	976

13.2.3	Serving cell/PCell as synchronization reference source	976

13.2.4	SyncRef UE as synchronization reference source	977

13.3	Initiation/Cease of SLSS Transmissions	977

13.3.1	Introduction	977

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	977

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	977

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	978

13.4	Selection / Reselection of V2X Synchronization Reference Source	978

13.5	Autonomous Resource Selection/Reselection measurements	979

13.5.1	Introduction	979

13.5.2	PSSCH-RSRP measurements	979

13.5.3	S-RSSI measurements	979

13.6	Congestion Control measurements	979

13.7	Interruption	980

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	980

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	980

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	980

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	981

13.8	Reliability of GNSS signal	981

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	981

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	981

Annex A (normative): Test Cases	983

A.1	Purpose of annex	983

A.2	Requirement classification for statistical testing	983

A.2.1	Types of requirements in TS 36.133	983

A.2.1.1	Time and delay requirements on UE higher layer actions	983

A.2.1.2	Measurements of power levels, relative powers and time	983

A.2.1.3	Implementation requirements	984

A.2.1.4	Physical layer timing requirements	984

A.3	RRM test configurations	985

A.3.1	Reference Measurement Channels	985

A.3.1.1	PDSCH	985

A.3.1.1.1	FDD	985

A.3.1.1.2	TDD	990

A.3.1.1.3	FDD for UE category 0	993

A.3.1.1.4	HD-FDD for UE category 0	994

A.3.1.1.5	TDD for UE category 0	995

A.3.1.1.6	Frame Structure 3	996

A.3.1.2	PCFICH/PDCCH/PHICH	997

A.3.1.2.1	FDD	997

A.3.1.2.2	TDD	997

A.3.1.2.3	HD-FDD for UE category 0	998

A.3.1.2.4	FS 3	998

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	998

A.3.1.3.1	FDD in CEModeA	999

A.3.1.3.2	HD-FDD in CEModeA	999

A.3.1.3.3	TDD in CEModeA	1000

A.3.1.3.4	FDD in CEModeB	1000

A.3.1.3.5	HD-FDD in CEModeB	1001

A.3.1.3.6	TDD in CEModeB	1001

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1002

A.3.1.4.1	FDD in CEModeA	1002

A.3.1.4.2	HD-FDD in CEModeA	1003

A.3.1.4.3	TDD in CEModeA	1004

A.3.1.4.4	FDD in CEModeB	1005

A.3.1.4.5	HD-FDD in CEModeB	1006

A.3.1.4.6	TDD in CEModeB	1006

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1007

A.3.1.5.1	HD-FDD in-band operation	1007

A.3.1.5.2	Void	1008

A.3.1.5.3	HD-FDD standalone operation	1008

A.3.1.5.4	Void	1009

A.3.1.5.5	HD-FDD guard band operation	1009

A.3.1.5.6	Void	1010

A.3.1.5.7	TDD in-band operation	1010

A.3.1.5.8	TDD standalone operation	1010

A.3.1.5.9	TDD guard band operation	1011

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1011

A.3.1.6.1	In-band operation	1011

A.3.1.6.2	Void	1012

A.3.1.6.3	Standalone operation	1012

A.3.1.6.4	Void	1012

A.3.1.6.5	Guard band operation	1012

A.3.1.6.6	Void	1013

A.3.2	OFDMA Channel Noise Generator (OCNG)	1013

A.3.2.1	OCNG Patterns for FDD	1013

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1014

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1014

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1015

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1016

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1016

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1017

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1017

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1017

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1018

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1019

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1019

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1020

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1020

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1021

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1021

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1022

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1022

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1023

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1023

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1024

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1024

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1025

A.3.2.2	OCNG Patterns for TDD	1025

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1026

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1026

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1027

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1027

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1028

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1029

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1030

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1031

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1031

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1032

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1033

A.3.2.3	OCNG Patterns for Narrowband IoT	1033

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1035

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1036

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1036

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1037

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1038

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1039

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1040

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1041

A.3.2.4	OCNG Patterns for V2X sidelink	1042

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1042

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1043

A.3.3	Reference DRX Configurations	1043

A.3.4	ABS Transmission Configurations	1044

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1044

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1044

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1044

A.3.4.2	MBSFN ABS Transmission Configurations	1045

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1045

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1046

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1046

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1046

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1047

A.3.6.1	Introduction	1047

A.3.7	Test Cases with Different Channel Bandwidths	1047

A.3.7.1	Introduction	1047

A.3.7.2	Principle of testing	1047

A.3.8	Antenna Configuration	1047

A.3.8.1	Antenna connection for 4 Rx capable UEs	1047

A.3.8.1.1 Introduction	1047

A.3.8.1.2 Principle of testing	1047

A.3.8.1.2.1 Single carrier tests	1047

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1048

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1049

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1049

A.3.8.2	Antenna connection for 8 Rx capable UEs	1049

A.3.8.2.1	Introduction	1049

A.3.8.2.2	Principle of testing	1049

A.3.8.2.2.1	Single carrier tests	1049

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1050

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1050

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1050

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1050

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1050

A.3.9.1	Introduction	1050

A.3.9.2	Principle of testing	1050

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1051

A.3.10.1	Introduction	1051

A.3.10.2	Principle of testing	1051

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1051

A.3.11.1	Introduction	1051

A.3.11.2	Principle of Testing	1051

A.3.12	Proximity-based Services	1051

A.3.12.1	Introduction	1051

A.3.12.2	Reference DRX configurations for ProSe tests	1051

A.3.12.3	Test Cases with Different Channel Bandwidths	1052

A.3.12.3.1	Introduction	1052

A.3.12.3.2	Principle of testing	1052

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1052

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1059

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1062

A.3.12.6.1	FDD	1062

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1062

A.3.12.7.1	FDD	1062

A.3.12.8	ProSe Receive Traffic Generator	1063

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1063

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1063

A.3.13	Time Offset between Cells	1064

A.3.13.1	Introduction	1064

A.3.13.2	Definition	1064

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1064

A.3.14.1	Introduction	1064

A.3.14.2	Principle of testing	1064

A.3.15	Dual connectivity test cases with different combination of duplex mode	1064

A.3.15.1	Introduction	1064

A.3.15.2	Principle of testing	1064

A.3.16	Reference PRACH Configurations	1065

A.3.17	Listen before talk model	1065

A.3.17.1	Introduction	1065

A.3.17.2	Definition	1065

A.3.18	Reference NPRACH Configurations	1066

A.3.19	Dual connectivity test cases with different bandwidth combinations	1067

A.3.19.1	Introduction	1067

A.3.19.2	Principle of testing	1067

A.3.20	Category M1 UE Test Cases	1068

A.3.20.1	Introduction	1068

A.3.20.2	Principle of Cat-M1 UE Testing	1068

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1069

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1070

A.3.21.1	Introduction	1070

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1070

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1071

A.3.22	Category 1bis UE Test Cases	1072

A.3.22.1	Introduction	1072

A.3.22.2	Principle of Category 1bis UE Testing	1072

A.3.23	Category NB2 UE Test Cases	1076

A.3.23.1	Introduction	1076

A.3.23.2	Principle of Category NB2 UE Testing	1076

A.3.24	V2X sidelink communication	1079

A.3.24.1	Introduction	1079

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1080

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1084

A.3.25	Category M2 UE Test Cases	1085

A.3.25.1	Introduction	1085

A.3.25.2	Principle of Cat-M2 UE Testing	1085

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1086

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1087

A.3.26.1	Introduction	1087

A.3.26.2	Principle of testing	1087

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1087

A.3.27.1	Introduction	1087

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1087

A.3.28	Reference configurations for satellites	1087

A.4	E-UTRAN RRC_IDLE state	1088

A.4.2	Cell Re-Selection	1088

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1088

A.4.2.1.1	Test Purpose and Environment	1088

A.4.2.1.2	Test Requirements	1089

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1090

A.4.2.2.1	Test Purpose and Environment	1090

A.4.2.2.2	Test Requirements	1091

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1092

A.4.2.3.1	Test Purpose and Environment	1092

A.4.2.3.2	Test Requirements	1093

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1094

A.4.2.4.1	Test Purpose and Environment	1094

A.4.2.4.2	Test Requirements	1095

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1096

A.4.2.5.1	Test Purpose and Environment	1096

A.4.2.5.2	Test Requirements	1097

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1098

A.4.2.6.1	Test Purpose and Environment	1098

A.4.2.6.2	Test Requirements	1099

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1100

A.4.2.7.1	Test Purpose and Environment	1100

A.4.2.7.2	Test Requirements	1101

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1102

A.4.2.8.1	Test Purpose and Environment	1102

A.4.2.8.2	Test Requirements	1103

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1104

A.4.2.9.1	Test Purpose and Environment	1104

A.4.2.9.2	Test Requirements	1104

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1104

A.4.2.10.1	Test Purpose and Environment	1104

A.4.2.10.2	Test Requirements	1108

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1108

A.4.2.11.1	Test Purpose and Environment	1108

A.4.2.11.2	Test Requirements	1112

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1112

A.4.2.12.1	Test Purpose and Environment	1112

A.4.2.12.2	Test Requirements	1114

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1115

A.4.2.13.1	Test Purpose and Environment	1115

A.4.2.13.2	Test Requirements	1116

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1117

A.4.2.14.1	Test Purpose and Environment	1117

A.4.2.14.2	Test Requirements	1118

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1119

A.4.2.15.1	Test Purpose and Environment	1119

A.4.2.15.2	Test Requirements	1120

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1121

A.4.2.16.1	Test Purpose and Environment	1121

A.4.2.16.2	Test Requirements	1122

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1123

A.4.2.17.1	Test Purpose and Environment	1123

A.4.2.17.2	Test Requirements	1124

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1125

A.4.2.18.1	Test Purpose and Environment	1125

A.4.2.18.2	Test Requirements	1127

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1128

A.4.2.19.1	Test Purpose and Environment	1128

A.4.2.19.2	Test Requirements	1130

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1131

A.4.2.20.1	Test Purpose and Environment	1131

A.4.2.20.2	Test Requirements	1132

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1133

A.4.2.21.1	Test Purpose and Environment	1133

A.4.2.21.2	Test Requirements	1134

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1135

A.4.2.22.1	Test Purpose and Environment	1135

A.4.2.22.2	Test Requirements	1136

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1137

A.4.2.23.1	Test Purpose and Environment	1137

A.4.2.23.2	Test Requirements	1138

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1139

A.4.2.24.1	Test Purpose and Environment	1139

A.4.2.24.2	Test Requirements	1142

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1143

A.4.2.25.1	Test Purpose and Environment	1143

A.4.2.25.2	Test Requirements	1144

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1145

A.4.2.26.1	Test Purpose and Environment	1145

A.4.2.26.2	Test Requirements	1146

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1147

A.4.2.27.1	Test Purpose and Environment	1147

A.4.2.27.2	Test Requirements	1148

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1149

A.4.2.28.1	Test Purpose and Environment	1149

A.4.2.28.2	Test Requirements	1150

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1151

A.4.2.29.1	Test Purpose and Environment	1151

A.4.2.29.2	Test Requirements	1152

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1153

A.4.2.30.1	Test Purpose and Environment	1153

A.4.2.30.2	Test Requirements	1154

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1155

A.4.2.31.1	Test Purpose and Environment	1155

A.4.2.31.2	Test Requirements	1156

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1157

A.4.2.32.1	Test Purpose and Environment	1157

A.4.2.32.2	Test Requirements	1158

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1159

A.4.2.33.1	Test Purpose and Environment	1159

A.4.2.33.2	Test Requirements	1160

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1161

A.4.2.34.1	Test Purpose and Environment	1161

A.4.2.34.2	Test Requirements	1162

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1163

A.4.2.35.1	Test Purpose and Environment	1163

A.4.2.35.2	Test Requirements	1165

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1166

A.4.2.36.1	Test Purpose and Environment	1166

A.4.2.36.2	Test Requirements	1168

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1169

A.4.2.37.1	Test Purpose and Environment	1169

A.4.2.37.2	Test Requirements	1171

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1172

A.4.2.38.1	Test Purpose and Environment	1172

A.4.2.38.2	Test Requirements	1174

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1175

A.4.2.39.1	Test Purpose and Environment	1175

A.4.2.39.2	Test Requirements	1176

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1177

A.4.2.40.1	Test Purpose and Environment	1177

A.4.2.40.2	Test Requirements	1178

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1179

A.4.2.41.1	Test Purpose and Environment	1179

A.4.2.41.2	Test Requirements	1182

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1183

A.4.2.42.1	Test Purpose and Environment	1183

A.4.2.42.2	Test Requirements	1185

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1186

A.4.2.43.1	Test Purpose and Environment	1186

A.4.2.43.2	Test Requirements	1188

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1189

A.4.2.44.1	Test Purpose and Environment	1189

A.4.2.44.2	Test Requirements	1191

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1192

A.4.2.45.1	Test Purpose and Environment	1192

A.4.2.45.2	Test Requirements	1194

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1195

A.4.2.46.1	Test Purpose and Environment	1195

A.4.2.46.2	Test Requirements	1197

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1198

A.4.2.47.1	Test Purpose and Environment	1198

A.4.2.47.2	Test Requirements	1200

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1201

A.4.2.48.1	Test Purpose and Environment	1201

A.4.2.48.2	Test Requirements	1202

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1203

A.4.2.49.1	Test Purpose and Environment	1203

A.4.2.49.2	Test Requirements	1205

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1206

A.4.2.50.1	Test Purpose and Environment	1206

A.4.2.50.2	Test Requirements	1208

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1209

A.4.2.51.1	Test Purpose and Environment	1209

A.4.2.51.2	Test Requirements	1211

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1212

A.4.2.52.1	Test Purpose and Environment	1212

A.4.2.52.2	Test Requirements	1214

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1215

A.4.2.53.1	Test Purpose and Environment	1215

A.4.2.53.2	Test Requirements	1216

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1217

A.4.2.54.1	Test Purpose and Environment	1217

A.4.2.54.2	Test Requirements	1219

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1220

A.4.2.55.1	Test Purpose and Environment	1220

A.4.2.55.2	Test Requirements	1222

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1223

A.4.2.56.1	Test Purpose and Environment	1223

A.4.2.56.2	Test Requirements	1224

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1225

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1225

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1225

A.4.3.1.1.1	Test Purpose and Environment	1225

A.4.3.1.1.2	Test Requirements	1227

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1227

A.4.3.1.2.1	Test Purpose and Environment	1227

A.4.3.1.2.2	Test Requirements	1230

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1230

A.4.3.1.3.1	Test Purpose and Environment	1230

A.4.3.1.3.2	Test Requirements	1233

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1233

A.4.3.1.4.1	Test Purpose and Environment	1233

A.4.3.1.4.2	Test Requirements	1234

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1234

A.4.3.1.5.1	Test Purpose and Environment	1234

A.4.3.1.5.2	Test Requirements	1237

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1238

A.4.3.2.1	Test Purpose and Environment	1238

A.4.3.2.1.1	Void	1238

A.4.3.2.1.2	1.28Mcps TDD option	1238

A.4.3.2.1.3	Void	1240

A.4.3.2.2	Test Requirements	1240

A.4.3.2.2.1	1.28Mcps TDD option	1240

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1240

A.4.3.2A.1	Test Purpose and Environment	1240

A.4.3.2A.2	Test Requirements	1244

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1244

A.4.3.3.1	Test Purpose and Environment	1244

A.4.3.3.2	Test Requirements	1247

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1247

A.4.3.3A.1	Test Purpose and Environment	1247

A.4.3.3A.2	Test Requirements	1252

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1253

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1253

A.4.3.4.1.1	Test Purpose and Environment	1253

A.4.3.4.1.2	Test Requirements	1255

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1255

A.4.3.4.2.1	Test Purpose and Environment	1255

A.4.3.4.2.2	Test Requirements	1257

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1257

A.4.3.4.3.1	Test Purpose and Environment	1257

A.4.3.4.3.2	Test Requirements	1260

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1260

A.4.3.4.4.1	Test Purpose and Environment	1260

A.4.3.4.4.2	Test Requirements	1264

A.4.4	E-UTRAN to GSM Cell Re-Selection	1264

A.4.4.1	E-UTRAN FDD – GSM:	1264

A.4.4.1.1	Test Purpose and Environment	1264

A.4.4.1.2	Test Requirements	1266

A.4.4.2	E-UTRAN TDD – GSM:	1266

A.4.4.2.1	Test Purpose and Environment	1266

A.4.4.2.2	Test Requirements	1268

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1269

A.4.5.1	E-UTRAN FDD – HRPD	1269

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1269

A.4.5.1.1.1	Test Purpose and Environment	1269

A.4.5.1.1.2	Test Requirements	1271

A.4.5.2	E-UTRAN TDD – HRPD	1271

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1271

A.4.5.2.1.1	Test Purpose and Environment	1271

A.4.5.2.1.2	Test Requirements	1274

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1274

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1274

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1274

A.4.6.1.1.1	Test Purpose and Environment	1274

A.4.6.1.1.2	Test Requirements	1277

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1277

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1277

A.4.6.2.1.1	Test Purpose and Environment	1277

A.4.6.2.1.2	Test Requirements	1280

A.4.7	Idle State Positioning Measurement for UE category NB1	1280

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1280

A.4.7.1.1	Test Purpose and Environment	1280

A.4.7.1.2	Test Requirements	1285

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1286

A.4.7.2.1	Test Purpose and Environment	1286

A.4.7.4.2	Test Requirements	1290

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1291

A.4.7.3.1	Test Purpose and Environment	1291

A.4.7.3.2	Test Requirements	1295

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1296

A.4.7.4.1	Test Purpose and Environment	1296

A.4.7.4.2	Test Requirements	1300

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1301

A.5.1	E-UTRAN Handover	1301

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1301

A.5.1.1.1	Test Purpose and Environment	1301

A.5.1.1.2	Test Requirements	1303

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1303

A.5.1.2.1	Test Purpose and Environment	1303

A.5.1.2.2	Test Requirements	1305

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1305

A.5.1.3.1	Test Purpose and Environment	1305

A.5.1.3.2	Test Requirements	1307

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1307

A.5.1.4.1	Test Purpose and Environment	1307

A.5.1.4.2	Test Requirements	1309

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1309

A.5.1.5.1	Test Purpose and Environment	1309

A.5.1.5.2	Test Requirements	1311

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1311

A.5.1.6.1	Test Purpose and Environment	1311

A.5.1.6.2	Test Requirements	1313

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1313

A.5.1.7.1	Test Purpose and Environment	1313

A.5.1.7.2	Test Requirements	1316

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1316

A.5.1.8.1	Test Purpose and Environment	1316

A.5.1.8.2	Test Requirements	1319

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1319

A.5.1.9.1	Test Purpose and Environment	1319

A.5.1.9.2	Test Requirements	1320

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1320

A.5.1.10.1	Test Purpose and Environment	1320

A.5.1.10.2	Test Requirements	1322

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1322

A.5.1.11.1	Test Purpose and Environment	1322

A.5.1.11.2	Test Requirements	1324

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1324

A.5.1.12.1	Test Purpose and Environment	1324

A.5.1.12.2	Test Requirements	1326

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1326

A.5.1.13.1	Test Purpose and Environment	1326

A.5.1.13.2	Test Requirements	1328

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1329

A.5.1.14.1	Test Purpose and Environment	1329

A.5.1.14.2	Test Requirements	1330

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1331

A.5.1.15.1	Test Purpose and Environment	1331

A.5.1.15.2	Test Requirements	1332

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1333

A.5.1.16.1	Test Purpose and Environment	1333

A.5.1.16.2	Test Requirements	1334

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1335

A.5.1.17.1	Test Purpose and Environment	1335

A.5.1.17.2	Test Requirements	1336

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1337

A.5.1.18.1	Test Purpose and Environment	1337

A.5.1.18.2	Test Requirements	1338

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1339

A.5.1.19.1	Test Purpose and Environment	1339

A.5.1.19.2	Test Requirements	1340

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1341

A.5.1.20.1	Test Purpose and Environment	1341

A.5.1.20.2	Test Requirements	1342

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1343

A.5.1.21.1	Test Purpose and Environment	1343

A.5.1.21.2	Test Requirements	1344

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1344

A.5.1.22.1	Test Purpose and Environment	1344

A.5.1.22.2	Test Requirements	1346

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1346

A.5.1.23.1	Test Purpose and Environment	1346

A.5.1.23.2	Test Requirements	1348

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1348

A.5.1.24.1	Test Purpose and Environment	1348

A.5.1.24.2	Test Requirements	1350

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1350

A.5.1.25.1	Test Purpose and Environment	1350

A.5.1.25.2	Test Requirements	1352

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1353

A.5.1.26.1	Test Purpose and Environment	1353

A.5.1.26.2	Test Requirements	1354

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1355

A.5.1.27.1	Test Purpose and Environment	1355

A.5.1.27.2	Test Requirements	1356

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1357

A.5.1.28.1	Test Purpose and Environment	1357

A.5.1.28.2	Test Requirements	1358

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1359

A.5.1.29.1	Test Purpose and Environment	1359

A.5.1.29.2	Test Requirements	1360

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1361

A.5.1.30.1	Test Purpose and Environment	1361

A.5.1.30.2	Test Requirements	1362

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1363

A.5.1.31.1	Test Purpose and Environment	1363

A.5.1.31.2	Test Requirements	1364

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1365

A.5.1.32.1	Test Purpose and Environment	1365

A.5.1.32.2	Test Requirements	1366

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1367

A.5.1.33.1	Test Purpose and Environment	1367

A.5.1.13.2	Test Requirements	1368

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1369

A.5.1.34.1	Test Purpose and Environment	1369

A.5.1.34.2	Test Requirements	1370

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1371

A.5.1.35.1	Test Purpose and Environment	1371

A.5.1.35.2	Test Requirements	1372

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1373

A.5.1.36.1	Test Purpose and Environment	1373

A.5.1.36.2	Test Requirements	1374

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1375

A.5.1.37.1	Test Purpose and Environment	1375

A.5.1.37.2	Test Requirements	1376

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1377

A.5.1.38.1	Test Purpose and Environment	1377

A.5.1.38.2	Test Requirements	1378

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1379

A.5.1.39.1	Test Purpose and Environment	1379

A.5.1.39.2	Test Requirements	1380

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1381

A.5.1.40.1	Test Purpose and Environment	1381

A.5.1.40.2	Test Requirements	1382

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1383

A.5.1.41.1	Test Purpose and Environment	1383

A.5.1.41.2	Test Requirements	1385

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1386

A.5.1.42.1	Test Purpose and Environment	1386

A.5.1.42.2	Test Requirements	1387

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1388

A.5.1.43.1	Test Purpose and Environment	1388

A.5.1.43.2	Test Requirements	1389

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1390

A.5.1.44.1	Test Purpose and Environment	1390

A.5.1.44.2	Test Requirements	1391

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1392

A.5.1.45.1	Test Purpose and Environment	1392

A.5.1.45.1	Test Requirements	1393

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1394

A.5.1.46.1	Test Purpose and Environment	1394

A.5.1.46.2	Test Requirements	1395

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1396

A.5.1.47.1	Test Purpose and Environment	1396

A.5.1.47.2	Test Requirements	1397

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1398

A.5.1.48.1	Test Purpose and Environment	1398

A.5.1.48.2	Test Requirements	1399

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1400

A.5.1.49.1	Test Purpose and Environment	1400

A.5.1.49.2	Test Requirements	1401

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1402

A.5.1.50.1	Test Purpose and Environment	1402

A.5.1.50.2	Test Requirements	1403

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1404

A.5.1.51.1	Test Purpose and Environment	1404

A.5.1.51.2	Test Requirements	1406

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1406

A.5.152.1	Test Purpose and Environment	1406

A.5.1.52.2	Test Requirements	1409

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1410

A.5.1.53.1	Test Purpose and Environment	1410

A.5.1.53.2	Test Requirements	1412

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1413

A.5.1.54.1	Test Purpose and Environment	1413

A.5.1.54.2	Test Requirements	1415

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1416

A.5.1.55.1	Test Purpose and Environment	1416

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1420

A.5.1.56.1	Test Purpose and Environment	1420

A.5.1.56.2	Test Requirements	1423

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1424

A.5.1.57.1	Test Purpose and Environment	1424

A.5.1.57.2	Test Requirements	1426

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1426

A.5.1.58.1	Test Purpose and Environment	1426

A.5.1.58.2	Test Requirements	1429

A.5.2	E-UTRAN Handover to other RATs	1429

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1429

A.5.2.1.1	Test Purpose and Environment	1429

A.5.2.1.2	Test Requirements	1431

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1432

A.5.2.2.1	Test Purpose and Environment	1432

A.5.2.2.2	Test Requirements	1435

A.5.2.3 E-UTRAN FDD- GSM Handover	1435

A.5.2.3.1	Test Purpose and Environment	1435

A.5.2.3.2	Test Requirements	1436

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1437

A.5.2.4.1	Test Purpose and Environment	1437

A.5.2.4.1.1	Void	1437

A.5.2.4.1.2	1.28 Mcps TDD option	1437

A.5.2.4.1.3	Void	1439

A.5.2.4.2	Test Requirements	1439

A.5.2.4.2.1	Void	1439

A.5.2.4.2.2	1.28 Mcps TDD option	1439

A.5.2.4.2.3	Void	1439

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1439

A.5.2.5.1	Test Purpose and Environment	1439

A.5.2.5.1.3	Void	1442

A.5.2.5.2	Test Requirements	1442

A.5.2.5.2.1	Void	1442

A.5.2.5.2.2	1.28 Mcps TDD option	1442

A.5.2.5.2.3	Void	1442

A.5.2.6	E-UTRAN TDD - GSM Handover	1442

A.5.2.6.1	Test Purpose and Environment	1442

A.5.2.6.2	Test Requirements	1444

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1445

A.5.2.7.1	Test Purpose and Environment	1445

A.5.2.7.2	Test Requirements	1447

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1447

A.5.2.8.1	Test Purpose and Environment	1447

A.5.2.8.2	Test Requirements	1448

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1449

A.5.2.9.1	Test Purpose and Environment	1449

A.5.2.9.2	Test Requirements	1450

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1451

A.5.2.10.1	Test Purpose and Environment	1451

A.5.2.10.2	Test Requirements	1453

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1453

A.5.2.10A.1	Test Purpose and Environment	1453

A.5.2.10A.2	Test Requirements	1456

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1456

A.5.2.10B.1	Test Purpose and Environment	1456

A.5.2.10B.2	Test Requirements	1459

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1459

A.5.2.11.1	Test Purpose and Environment	1459

A.5.2.11.2	Test Requirements	1459

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1460

A.5.3.1	E-UTRAN FDD – HRPD Handover	1460

A.5.3.1.1	Test Purpose and Environment	1460

A.5.3.1.2	Test Requirements	1462

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1462

A.5.3.2.1	Test Purpose and Environment	1462

A.5.3.2.2	Test Requirements	1465

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1465

A.5.3.3.1	Test Purpose and Environment	1465

A.5.3.3.2	Test Requirements	1468

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1468

A.5.3.4.1	Test Purpose and Environment	1468

A.5.3.4.2	Test Requirements	1469

A.5.3.5	E-UTRAN TDD – HRPD Handover	1470

A.5.3.5.1	Test Purpose and Environment	1470

A.5.3.5.2	Test Requirements	1472

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1473

A.5.3.6.1	Test Purpose and Environment	1473

A.5.3.6.2	Test Requirements	1476

A.6	RRC Connection Control	1476

A.6.1	RRC Re-establishment	1476

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1476

A.6.1.1.1	Test Purpose and Environment	1476

A.6.1.1.2	Test Requirements	1477

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1478

A.6.1.2.1	Test Purpose and Environment	1478

A.6.1.2.2	Test Requirements	1479

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1480

A.6.1.3.1	Test Purpose and Environment	1480

A.6.1.3.2	Test Requirements	1481

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1482

A.6.1.4.1	Test Purpose and Environment	1482

A.6.1.4.2	Test Requirements	1483

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1484

A.6.1.5.1	Test Purpose and Environment	1484

A.6.1.5.2	Test Requirements	1484

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1484

A.6.1.6.1	Test Purpose and Environment	1484

A.6.1.6.2	Test Requirements	1486

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1487

A.6.1.7.1	Test Purpose and Environment	1487

A.6.1.7.2	Test Requirements	1488

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1489

A.6.1.8.1	Test Purpose and Environment	1489

A.6.1.8.2	Test Requirements	1490

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1491

A.6.1.9.1	Test Purpose and Environment	1491

A.6.1.9.2	Test Requirements	1492

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1493

A.6.1.10.1	Test Purpose and Environment	1493

A.6.1.10.2	Test Requirements	1494

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1495

A.6.1.11.1	Test Purpose and Environment	1495

A.6.1.11.2	Test Requirements	1496

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1497

A.6.1.12.1	Test Purpose and Environment	1497

A.6.1.12.2	Test Requirements	1498

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1499

A.6.1.13.1	Test Purpose and Environment	1499

A.6.1.13.2	Test Requirements	1500

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1501

A.6.1.14.1	Test Purpose and Environment	1501

A.6.1.14.2	Test Requirements	1502

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1503

A.6.1.15.1	Test Purpose and Environment	1503

A.6.1.15.2	Test Requirements	1505

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1506

A.6.1.16.1	Test Purpose and Environment	1506

A.6.1.16.2	Test Requirements	1508

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1509

A.6.1.17.1	Test Purpose and Environment	1509

A.6.1.17.2	Test Requirements	1510

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1511

A.6.1.18.1	Test Purpose and Environment	1511

A.6.1.18.2	Test Requirements	1512

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1513

A.6.1.19.1	Test Purpose and Environment	1513

A.6.1.19.2	Test Requirements	1514

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1515

A.6.1.20.1	Test Purpose and Environment	1515

A.6.1.20.2	Test Requirements	1516

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1517

A.6.1.21.1	Test Purpose and Environment	1517

A.6.1.21.2	Test Requirements	1518

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1519

A.6.1.22.1	Test Purpose and Environment	1519

A.6.1.22.2	Test Requirements	1520

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1521

A.6.1.23.1	Test Purpose and Environment	1521

A.6.1.23.2	Test Requirements	1523

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1524

A.6.1.24.1	Test Purpose and Environment	1524

A.6.1.24.2	Test Requirements	1526

A.6.2	Random Access	1527

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1527

A.6.2.1.1	Test Purpose and Environment	1527

A.6.2.1.2	Test Requirements	1528

A.6.2.1.2.1	Random Access Response Reception	1528

A.6.2.1.2.2	No Random Access Response Reception	1528

A.6.2.1.2.3	Receiving a NACK on msg3	1528

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1529

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1529

A.6.2.1.2.6	Contention Resolution Timer expiry	1529

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1529

A.6.2.2.1	Test Purpose and Environment	1529

A.6.2.2.2	Test Requirements	1530

A.6.2.2.2.1	Random Access Response Reception	1531

A.6.2.2.2.2	No Random Access Response Reception	1531

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1531

A.6.2.3.1	Test Purpose and Environment	1531

A.6.2.3.2	Test Requirements	1533

A.6.2.3.2.1	Random Access Response Reception	1533

A.6.2.3.2.2	No Random Access Response reception	1533

A.6.2.3.2.3	Receiving a NACK on msg3	1533

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1534

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1534

A.6.2.3.2.6	Contention Resolution Timer expiry	1534

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1534

A.6.2.4.1	Test Purpose and Environment	1534

A.6.2.4.2	Test Requirements	1536

A.6.2.4.2.1	Random Access Response Reception	1536

A.6.2.4.2.2	No Random Access Response Reception	1536

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1536

A.6.2.5.1	Test Purpose and Environment	1536

A.6.2.5.2	Test Requirements	1537

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1537

A.6.2.6.1	Test Purpose and Environment	1537

A.6.2.6.2	Test Requirements	1537

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1538

A.6.2.7.1	Test Purpose and Environment	1538

A.6.2.7.2	Test Requirements	1540

A.6.2.7.2.1	Random Access Response Reception	1540

A.6.2.7.2.2	No Random Access Response Reception	1540

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1540

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1541

A.6.2.8.1	Test Purpose and Environment	1541

A.6.2.8.2	Test Requirements	1543

A.6.2.8.2.1	Random Access Response Reception	1543

A.6.2.8.2.2	No Random Access Response Reception	1543

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1543

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1544

A.6.2.9.1	Test Purpose and Environment	1544

A.6.2.9.2	Test Requirements	1547

A.6.2.9.2.1	Random Access Response Reception	1547

A.6.2.9.2.2	No Random Access Response Reception	1548

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1548

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1549

A.6.2.10.1	Test Purpose and Environment	1549

A.6.2.10.2	Test Requirements	1551

A.6.2.10.2.1	Random Access Response Reception	1551

A.6.2.10.2.2	No Random Access Response Reception	1552

A.6.2.10.2.3	Receiving a NACK on msg3	1552

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1552

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1552

A.6.2.10.2.6	Contention Resolution Timer expiry	1552

A.6.2.10.2.7	PRACH Resource Selection	1553

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1553

A.6.2.11.1	Test Purpose and Environment	1553

A.6.2.11.2	Test Requirements	1555

A.6.2.11.2.1	Random Access Response Reception	1555

A.6.2.11.2.2	No Random Access Response Reception	1556

A.6.2.11.2.3	Receiving a NACK on msg3	1556

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1556

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1556

A.6.2.11.2.6	Contention Resolution Timer expiry	1556

A.6.2.11.2.7	PRACH Resource Selection	1557

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1557

A.6.2.12.1	Test Purpose and Environment	1557

A.6.2.12.2	Test Requirements	1559

A.6.2.12.2.1	Random Access Response Reception	1559

A.6.2.12.2.2	No Random Access Response Reception	1560

A.6.2.12.2.3	Receiving a NACK on msg3	1560

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1560

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1560

A.6.2.12.2.6	Contention Resolution Timer expiry	1560

A.6.2.12.2.7	PRACH Resource Selection	1561

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1561

A.6.2.13.1	Test Purpose and Environment	1561

A.6.2.13.2	Test Requirements	1563

A.6.2.13.2.1	Random Access Response Reception	1563

A.6.2.13.2.2	No Random Access Response Reception	1564

A.6.2.13.2.3	Receiving a NACK on msg3	1564

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1564

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1564

A.6.2.13.2.6	Contention Resolution Timer expiry	1564

A.6.2.13.2.7	PRACH Resource Selection	1565

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1565

A.6.2.14.1	Test Purpose and Environment	1565

A.6.2.14.2	Test Requirements	1567

A.6.2.14.2.1	Random Access Response Reception	1567

A.6.2.14.2.2	No Random Access Response Reception	1568

A.6.2.14.2.3	Receiving a NACK on msg3	1568

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1568

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1568

A.6.2.14.2.6	Contention Resolution Timer expiry	1568

A.6.2.14.2.7	PRACH Resource Selection	1569

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1569

A.6.2.15.1	Test Purpose and Environment	1569

A.6.2.15.2	Test Requirements	1571

A.6.2.15.2.1	Random Access Response Reception	1571

A.6.2.15.2.2	No Random Access Response Reception	1572

A.6.2.15.2.3	Receiving a NACK on msg3	1572

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1572

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1572

A.6.2.15.2.6	Contention Resolution Timer expiry	1572

A.6.2.15.2.7	PRACH Resource Selection	1573

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1573

A.6.2.16.1	Test Purpose and Environment	1573

A.6.2.16.2	Test Requirements	1576

A.6.2.16.2.1	Random Access Response Reception	1576

A.6.2.16.2.2	No Random Access Response Reception	1577

A.6.2.16.2.3	Receiving a NACK on msg3	1577

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1577

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1577

A.6.2.16.2.6	Contention Resolution Timer expiry	1577

A.6.2.16.2.7	NPRACH Resource Selection	1577

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1578

A.6.2.17.1	Test Purpose and Environment	1578

A.6.2.17.2	Test Requirements	1581

A.6.2.17.2.1	Random Access Response Reception	1581

A.6.2.17.2.2	No Random Access Response Reception	1582

A.6.2.17.2.3	Receiving a NACK on msg3	1582

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1582

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1582

A.6.2.17.2.6	Contention Resolution Timer expiry	1582

A.6.2.17.2.7	NPRACH Resource Selection	1582

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1583

A.6.2.18.1	Test Purpose and Environment	1583

A.6.2.18.2	Test Requirements	1586

A.6.2.18.2.1	Random Access Response Reception	1586

A.6.2.18.2.2	No Random Access Response Reception	1587

A.6.2.18.2.3	Receiving a NACK on msg3	1587

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1587

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1587

A.6.2.18.2.6	Contention Resolution Timer expiry	1587

A.6.2.18.2.7	NPRACH Resource Selection	1587

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1588

A.6.2.19.1	Test Purpose and Environment	1588

A.6.2.19.2	Test Requirements	1590

A.6.2.19.2.1	Random Access Response Reception	1590

A.6.2.19.2.2	No Random Access Response Reception	1591

A.6.2.19.2.3	Receiving a NACK on msg3	1591

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1591

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1591

A.6.2.19.2.6	Contention Resolution Timer expiry	1591

A.6.2.19.2.7	NPRACH Resource Selection	1591

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1592

A.6.2.20.1	Test Purpose and Environment	1592

A.6.2.20.2	Test Requirements	1594

A.6.2.20.2.1	Random Access Response Reception	1594

A.6.2.20.2.2	No Random Access Response Reception	1595

A.6.2.20.2.3	Receiving a NACK on msg3	1595

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1595

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1595

A.6.2.20.2.6	Contention Resolution Timer expiry	1595

A.6.2.20.2.7	NPRACH Resource Selection	1595

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1596

A.6.2.21.1	Test Purpose and Environment	1596

A.6.2.21.2	Test Requirements	1598

A.6.2.21.2.1	Random Access Response Reception	1598

A.6.2.21.2.2	No Random Access Response Reception	1599

A.6.2.21.2.3	Receiving a NACK on msg3	1599

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1599

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1599

A.6.2.21.2.6	Contention Resolution Timer expiry	1599

A.6.2.21.2.7	NPRACH Resource Selection	1599

A.6.3	RRC Connection Release with Redirection	1600

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1600

A.6.3.1.1	Test Purpose and Environment	1600

A.6.3.1.2	Test Requirements	1602

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1602

A.6.3.2.1	Test Purpose and Environment	1602

A.6.3.2.2	Test Requirements	1604

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1604

A.6.3.3.1	Test Purpose and Environment	1604

A.6.3.3.2	Test Requirements	1605

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1606

A.6.3.4.1	Test Purpose and Environment	1606

A.6.3.4.2	Test Requirements	1607

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1608

A.6.3.5.1	Test Purpose and Environment	1608

A.6.3.5.2	Test Requirements	1610

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1610

A.6.3.6.1	Test Purpose and Environment	1610

A. 6.3.6.2	Test Requirements	1613

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1613

A.6.3.7.1	Test Purpose and Environment	1613

A.6.3.7.2	Test Requirements	1616

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1616

A.6.3.8.1	Test Purpose and Environment	1616

A.6.3.8.2	Test Requirements	1619

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1619

A.6.3.9.1	Test Purpose and Environment	1619

A.6.3.9.2	Test Requirements	1621

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1621

A.6.3.10.1	Test Purpose and Environment	1621

A.6.3.10.2	Test Requirements	1623

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1623

A.6.3.11.1	Test Purpose and Environment	1623

A.6.3.11.2	Test Requirements	1625

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1625

A.6.3.12.1	Test Purpose and Environment	1625

A.6.3.12.2	Test Requirements	1628

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1628

A.6.3.13.1	Test Purpose and Environment	1628

A.6.3.13.2	Test Parameters	1628

A.6.3.13.3	Test Requirements	1632

A.7	Timing and Signalling Characteristics	1633

A.7.1	UE Transmit Timing	1633

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1633

A.7.1.1.1	Test Purpose and Environment	1633

A.7.1.1.2	Test Requirements	1634

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1635

A.7.1.2.1	Test Purpose and Environment	1635

A.7.1.2.2	Test Requirements	1637

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1638

A.7.1.3.1	Test Purpose and Environment	1638

A.7.1.3.2	Test Requirements	1640

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1641

A.7.1.4.1	Test Purpose and Environment	1641

A.7.1.4.2	Test Requirements	1642

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1643

A.7.1.4A.1	Test Purpose and Environment	1643

A.7.1.4A.2	Test Requirements	1643

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1643

A.7.1.5.1	Test Purpose and Environment	1643

A.7.1.5.2	Test Requirements	1644

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1644

A.7.1.6.1	Test Purpose and Environment	1644

A.7.1.6.2	Test Requirements	1646

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1647

A.7.1.7.1	Test Purpose and Environment	1647

A.7.1.7.2	Test Requirements	1648

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1649

A.7.1.7A.1	Test Purpose and Environment	1649

A.7.1.7A.2	Test Requirements	1649

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1649

A.7.1.7B.1	Test Purpose and Environment	1649

A.7.1.7B.2	Test Requirements	1649

A.7.1.8	Void	1650

A.7.1.8.1	Void	1650

A.7.1.8.2	Void	1650

A.7.1.9	Void	1650

A.7.1.9.1	Void	1650

A.7.1.9.2	Void	1650

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1650

A.7.1.10.1	Test Purpose and Environment	1650

A.7.1.10.2	Test Requirements	1652

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1652

A.7.1.11.1	Test Purpose and Environment	1652

A.7.1.11.2	Test Requirements	1654

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1654

A.7.1.12.1	Test Purpose and Environment	1654

A.7.1.12.2	Test Requirements	1656

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1656

A.7.1.13.1	Test Purpose and Environment	1656

A.7.1.13.2	Test Requirements	1658

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1659

A.7.1.14.1	Test Purpose and Environment	1659

A.7.1.14.2	Test Requirements	1660

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1660

A.7.1.15.1	Test Purpose and Environment	1660

A.7.1.15.2	Test Requirements	1661

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1662

A.7.1.16.1	Test Purpose and Environment	1662

A.7.1.16.2	Test Requirements	1663

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1664

A.7.1.17.1	Test Purpose and Environment	1664

A.7.1.17.2	Test Requirements	1666

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1667

A.7.1.18.1	Test Purpose and Environment	1667

A.7.1.18.2	Test Requirements	1669

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1670

A.7.1.19.1	Test Purpose and Environment	1670

A.7.1.19.2	Test Requirements	1671

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1671

A.7.1.20.1	Test Purpose and Environment	1671

A.7.1.20.2	Test Requirements	1673

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1673

A.7.1.21.1	Test Purpose and Environment	1673

A.7.1.21.2	Test Requirements	1675

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1675

A.7.1.22.1	Test Purpose and Environment	1675

A.7.1.22.2	Test Requirements	1677

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1677

A.7.1.23.1	Test Purpose and Environment	1677

A.7.1.23.2	Test Requirements	1679

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1679

A.7.1.24.1	Test Purpose and Environment	1679

A.7.1.24.2	Test Requirements	1680

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1681

A.7.1.25.1	Test Purpose and Environment	1681

A.7.1.25.2	Test Requirements	1682

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1683

A.7.1.26.1	Test Purpose and Environment	1683

A.7.1.26.2	Test Requirements	1684

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1685

A.7.1.27.1	Test Purpose and Environment	1685

A.7.1.27.2	Test Requirements	1687

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1687

A.7.1.28.1	Test Purpose and Environment	1687

A.7.1.28.2	Test Requirements	1689

A.7.2	UE Timing Advance	1690

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1690

A.7.2.1.1	Test Purpose and Environment	1690

A.7.2.1.2	Test Requirements	1691

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1692

A.7.2.2.1	Test Purpose and Environment	1692

A.7.2.2.2	Test Requirements	1694

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1694

A.7.2.3.1	Test Purpose and Environment	1694

A.7.2.3.2	Test Requirements	1694

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1694

A.7.2.4.1	Test Purpose and Environment	1694

A.7.2.4.2	Test Requirements	1697

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1697

A.7.2.5.1	Test Purpose and Environment	1697

A.7.2.5.2	Test Requirements	1699

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1699

A.7.2.5A.1	Test Purpose and Environment	1699

A.7.2.5A.2	Test Requirements	1700

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1700

A.7.2.5B.1	Test Purpose and Environment	1700

A.7.2.5B.2	Test Requirements	1700

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1700

A.7.2.6.1	Test Purpose and Environment	1700

A.7.2.6.2	Test Requirements	1703

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1703

A.7.2.7.1	Test Purpose and Environment	1703

A.7.2.7.2	Test Requirements	1705

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1705

A.7.2.8.1	Test Purpose and Environment	1705

A.7.2.8.2	Test Requirements	1707

A.7.2.9.2	Test Requirements	1708

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1709

A.7.2.10.1	Test Purpose and Environment	1709

A.7.2.10.2	Test Requirements	1710

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1710

A.7.2.11.1	Test Purpose and Environment	1710

A.7.2.11.2	Test Requirements	1712

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1712

A.7.2.12.1	Test Purpose and Environment	1712

A.7.2.12.2	Test Requirements	1714

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1715

A.7.2.13.1	Test Purpose and Environment	1715

A.7.2.13.2	Test Requirements	1716

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1717

A.7.2.14.1	Test Purpose and Environment	1717

A.7.2.14.2	Test Requirements	1719

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1719

A.7.2.15.1	Test Purpose and Environment	1719

A.7.2.15.2	Test Requirements	1721

A.7.3	Radio Link Monitoring	1721

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1721

A.7.3.1.1	Test Purpose and Environment	1721

A.7.3.1.2	Test Requirements	1725

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1725

A.7.3.2.1	Test Purpose and Environment	1725

A.7.3.2.2	Test Requirements	1729

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1729

A.7.3.3.1	Test Purpose and Environment	1729

A.7.3.3.2	Test Requirements	1733

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1733

A.7.3.4.1	Test Purpose and Environment	1733

A.7.3.4.2	Test Requirements	1737

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1737

A.7.3.5.1	Test Purpose and Environment	1737

A.7.3.5.2	Test Requirements	1740

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1741

A.7.3.6.1	Test Purpose and Environment	1741

A.7.3.6.2	Test Requirements	1744

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1744

A.7.3.7.1	Test Purpose and Environment	1744

A.7.3.7.2	Test Requirements	1747

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1747

A.7.3.8.1	Test Purpose and Environment	1747

A.7.3.8.2	Test Requirements	1750

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1750

A.7.3.9.2	Test Requirements	1753

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1753

A.7.3.10.1	Test Purpose and Environment	1753

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1757

A.7.3.11.1	Test Purpose and Environment	1757

A.7.3.11.2	Test Requirements	1762

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1762

A.7.3.12.1	Test Purpose and Environment	1762

A.7.3.12.2	Test Requirements	1767

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1767

A.7.3.13.1	Test Purpose and Environment	1767

A.7.3.13.2	Test Requirements	1770

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1770

A.7.3.14.1	Test Purpose and Environment	1770

A.7.3.14.2	Test Requirements	1773

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1773

A.7.3.15.1	Test Purpose and Environment	1773

A.7.3.15.2	Test Requirements	1777

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1777

A.7.3.16.1	Test Purpose and Environment	1777

A.7.3.16.2	Test Requirements	1782

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1782

A.7.3.17.1	Test Purpose and Environment	1782

A.7.3.17.2	Test Requirements	1786

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1786

A.7.3.18.1	Test Purpose and Environment	1786

A.7.3.18.2	Test Requirements	1790

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1790

A.7.3.19.1	Test Purpose and Environment	1790

A.7.3.19.2	Test Requirements	1795

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1795

A.7.3.20.1	Test Purpose and Environment	1795

A.7.3.20.2	Test Requirements	1800

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1800

A.7.3.21.1	Test Purpose and Environment	1800

A.7.3.21.2	Test Requirements	1805

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1805

A.7.3.22.1	Test Purpose and Environment	1805

A.7.3.22.2	Test Requirements	1810

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1810

A.7.3.23.1	Test Purpose and Environment	1810

A.7.3.23.2	Test Requirements	1811

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1811

A.7.3.24.1	Test Purpose and Environment	1811

A.7.3.24.2	Test Requirements	1812

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1812

A.7.3.25.1	Test Purpose and Environment	1812

A.7.3.25.2	Test Requirements	1813

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1813

A.7.3.26.1	Test Purpose and Environment	1813

A.7.3.26.2	Test Requirements	1816

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1816

A.7.3.27.1	Test Purpose and Environment	1816

A.7.3.27.2	Test Requirements	1819

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1819

A.7.3.28.1	Test Purpose and Environment	1819

A.7.3.28.2	Test Requirements	1822

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1822

A.7.3.29.1	Test Purpose and Environment	1822

A.7.3.29.2	Test Requirements	1825

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1825

A.7.3.30.1	Test Purpose and Environment	1825

A.7.3.30.2	Test Requirements	1828

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1828

A.7.3.31.1	Test Purpose and Environment	1828

A.7.3.31.2	Test Requirements	1831

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1831

A.7.3.32.1	Test Purpose and Environment	1831

A.7.3.32.2	Test Requirements	1834

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1834

A.7.3.33.1	Test Purpose and Environment	1834

A.7.3.33.2	Test Requirements	1837

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1837

A.7.3.34.1	Test Purpose and Environment	1837

A.7.3.34.2	Test Requirements	1840

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1840

A.7.3.35.1	Test Purpose and Environment	1840

A.7.3.35.2	Test Requirements	1843

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1843

A.7.3.36.1	Test Purpose and Environment	1843

A.7.3.36.2	Test Requirements	1846

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1846

A.7.3.37.1	Test Purpose and Environment	1846

A.7.3.37.2	Test Requirements	1849

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1849

A.7.3.38.1	Test Purpose and Environment	1849

A.7.3.38.2	Test Requirements	1852

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1852

A.7.3.39.1	Test Purpose and Environment	1852

A.7.3.39.2	Test Requirements	1855

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1856

A.7.3.40.1	Test Purpose and Environment	1856

A.7.3.40.2	Test Requirements	1858

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1859

A.7.3.41.1	Test Purpose and Environment	1859

A.7.3.41.2	Test Requirements	1862

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1862

A.7.3.42.1	Test Purpose and Environment	1862

A.7.3.42.2	Test Requirements	1865

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1865

A.7.3.43.1	Test Purpose and Environment	1865

A.7.3.43.2	Test Requirements	1868

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1868

A.7.3.44.1	Test Purpose and Environment	1868

A.7.3.44.2	Test Requirements	1871

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1872

A.7.3.45.1	Test Purpose and Environment	1872

A.7.3.45.2	Test Requirements	1874

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1875

A.7.3.46.1	Test Purpose and Environment	1875

A.7.3.46.2	Test Requirements	1878

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1878

A.7.3.47.1	Test Purpose and Environment	1878

A.7.3.47.2	Test Requirements	1881

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1881

A.7.3.48.1	Test Purpose and Environment	1881

A.7.3.48.2	Test Requirements	1884

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1884

A.7.3.49.1	Test Purpose and Environment	1884

A.7.3.49.2	Test Requirements	1887

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1887

A.7.3.50.1	Test Purpose and Environment	1887

A.7.3.50.2	Test Requirements	1890

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1890

A.7.3.51.1	Test Purpose and Environment	1890

A.7.3.51.2	Test Requirements	1893

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1893

A.7.3.52.1	Test Purpose and Environment	1893

A.7.3.52.2	Test Requirements	1896

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1896

A.7.3.53.1	Test Purpose and Environment	1896

A.7.3.53.2	Test Requirements	1899

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1899

A.7.3.54.1	Test Purpose and Environment	1899

A.7.3.54.2	Test Requirements	1902

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1902

A.7.3.55.1	Test Purpose and Environment	1902

A.7.3.55.2	Test Requirements	1905

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1905

A.7.3.56.1	Test Purpose and Environment	1905

A.7.3.56.2	Test Requirements	1908

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1908

A.7.3.57.1	Test Purpose and Environment	1908

A.7.3.57.2	Test Requirements	1911

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1911

A.7.3.58.1	Test Purpose and Environment	1911

A.7.3.58.2	Test Requirements	1914

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1914

A.7.3.59.1	Test Purpose and Environment	1914

A.7.3.59.2	Test Requirements	1917

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1917

A.7.3.60.1	Test Purpose and Environment	1917

A.7.3.60.2	Test Requirements	1921

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1921

A.7.3.61.1	Test Purpose and Environment	1921

A.7.3.61.2	Test Requirements	1925

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1925

A.7.3.62.1	Test Purpose and Environment	1925

A.7.3.62.2	Test Requirements	1929

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1929

A.7.3.63.1	Test Purpose and Environment	1929

A.7.3.63.2	Test Requirements	1933

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1933

A.7.3.64.1	Test Purpose and Environment	1933

A.7.3.64.2	Test Requirements	1936

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1937

A.7.3.65.1	Test Purpose and Environment	1937

A.7.3.65.2	Test Requirements	1940

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1941

A.7.3.66.1	Test Purpose and Environment	1941

A.7.3.66.2	Test Requirements	1943

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1944

A.7.3.67.1	Test Purpose and Environment	1944

A.7.3.67.2	Test Requirements	1947

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1948

A.7.3.68.1	Test Purpose and Environment	1948

A.7.3.68.2	Test Requirements	1950

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1950

A.7.3.69.1	Test Purpose and Environment	1950

A.7.3.69.2	Test Requirements	1952

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1952

A.7.3.70.1	Test Purpose and Environment	1952

A.7.3.70.2	Test Requirements	1954

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1954

A.7.3.71.1	Test Purpose and Environment	1954

A.7.3.71.2	Test Requirements	1957

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1957

A.7.3.72.1	Test Purpose and Environment	1957

A.7.3.72.2	Test Requirements	1959

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1959

A.7.3.73.1	Test Purpose and Environment	1959

A.7.3.73.2	Test Requirements	1961

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1961

A.7.3.74.1	Test Purpose and Environment	1961

A.7.3.74.2	Test Requirements	1964

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1964

A.7.3.75.1	Test Purpose and Environment	1964

A.7.3.75.2	Test Requirements	1966

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1967

A.7.3.76.1	Test Purpose and Environment	1967

A.7.3.76.2	Test Requirements	1970

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1970

A.7.3.77.1	Test Purpose and Environment	1970

A.7.3.77.2	Test Requirements	1973

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1973

A.7.3.78.1	Test Purpose and Environment	1973

A.7.3.78.2	Test Requirements	1976

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1976

A.7.3.79.1	Test Purpose and Environment	1976

A.7.3.79.2	Test Requirements	1979

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1979

A.7.3.80.1	Test Purpose and Environment	1979

A.7.3.80.2	Test Requirements	1982

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1982

A.7.3.81.1	Test Purpose and Environment	1982

A.7.3.81.2	Test Requirements	1985

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1985

A.7.3.82.1	Test Purpose and Environment	1985

A.7.3.82.2	Test Requirements	1988

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1988

A.7.3.83.1	Test Purpose and Environment	1988

A.7.3.83.2	Test Requirements	1989

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1990

A.7.3.84.1	Test Purpose and Environment	1990

A.7.3.84.2	Test Requirements	1992

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1992

A.7.3.85.1	Test Purpose and Environment	1992

A.7.3.85.2	Test Requirements	1993

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1994

A.7.3.86.1	Test Purpose and Environment	1994

A.7.3.86.2	Test Requirements	1996

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1996

A.7.3.87.1	Test Purpose and Environment	1996

A.7.3.87.2	Test Requirements	1998

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1999

A.7.3.88.1	Test Purpose and Environment	1999

A.7.3.88.2	Test Requirements	2003

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2003

A.7.3.89.1	Test Purpose and Environment	2003

A.7.3.89.2	Test Requirements	2007

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2007

A.7.3.90.1	Test Purpose and Environment	2007

A.7.3.90.2	Test Requirements	2011

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2011

A.7.3.91.1	Test Purpose and Environment	2011

A.7.3.91.2	Test Requirements	2015

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2015

A.7.3.92.1	Test Purpose and Environment	2015

A.7.3.92.2	Test Requirements	2018

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2019

A.7.3.93.1	Test Purpose and Environment	2019

A.7.3.93.2	Test Requirements	2022

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2023

A.7.3.94.1	Test Purpose and Environment	2023

A.7.3.94.2	Test Requirements	2025

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2026

A.7.3.95.1	Test Purpose and Environment	2026

A.7.3.95.2	Test Requirements	2029

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2030

A.7.3.96.1	Test Purpose and Environment	2030

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2031

A.7.3.97.1	Test Purpose and Environment	2031

A.7.3.97.2	Test Requirements	2034

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2034

A.7.3.98.1	Test Purpose and Environment	2034

A.7.3.98.2	Test Requirements	2037

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2037

A.7.3.99.1	Test Purpose and Environment	2037

A.7.3.99.2	Test Requirements	2040

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2040

A.7.3.100.1	Test Purpose and Environment	2040

A.7.3.100.2	Test Requirements	2042

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2042

A.7.3.101.1	Test Purpose and Environment	2042

A.7.3.101.2	Test Requirements	2044

A.7.4	Interruption for Dual Connectivity	2044

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2044

A.7.4.1.1	Test Purpose and Environment	2044

A.7.4.1.2	Test Requirements	2047

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2047

A.7.4.2.1	Test Purpose and Environment	2047

A.7.4.2.2	Test Requirements	2049

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2049

A.7.4.3.1	Test Purpose and Environment	2049

A.7.4.3.2	Test Requirements	2051

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2051

A.7.4.4.1	Test Purpose and Environment	2051

A.7.4.4.2	Test Requirements	2054

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2054

A.7.4.5.1	Test Purpose and Environment	2054

A.7.4.5.2	Test Requirements	2056

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2056

A.7.4.6.1	Test Purpose and Environment	2056

A.7.4.6.2	Test Requirements	2058

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2058

A.7.4.7.1	Test Purpose and Environment	2058

A.7.4.7.2	Test Requirements	2062

A.7.5	Proximity-based Services	2062

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2062

A.7.5.1.1	Test Purpose and Environment	2062

A.7.5.1.2	Test Requirements	2063

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2064

A.7.5.2.1	Test Purpose and Environment	2064

A.7.5.1.2	Test Requirements	2064

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2065

A.7.5.3.1	Test Purpose and Environment	2065

A.7.5.3.2	Test Requirements	2066

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2067

A.7.5.4.1	Test Purpose and Environment	2067

A.7.5.4.2	Test Requirements	2068

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2069

A.7.5.5.1	Test Purpose and Environment	2069

A.7.5.5.2	Test Requirements	2071

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2072

A.7.5.6.1	Test Purpose and Environment	2072

A.7.5.6.2	Test Requirements	2073

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2074

A.7.5.7.1	Test Purpose and Environment	2074

A.7.5.7.2	Test Requirements	2076

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2076

A.7.5.8.1	Test Purpose and Environment	2076

A.7.5.8.2	Test Requirements	2078

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2079

A.7.5.9.1	Test Purpose and Environment	2079

A.7.5.9.2	Test Requirements	2081

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2082

A.7.5.10.1	Test Purpose and Environment	2082

A.7.5.10.2	Test Requirements	2084

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2085

A.7.5.11.1	Test Purpose and Environment	2085

A.7.5.11.2	Test Requirements	2087

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2087

A.7.5.12.1	Test Purpose and Environment	2087

A.7.5.12.2	Test Requirements	2091

A.7.6	Interruption for carrier aggregation	2092

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2092

A.7.6.1.1	Test Purpose and Environment	2092

A.7.6.1.2	Test Requirements	2095

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2095

A.7.6.2.1	Test Purpose and Environment	2095

A.7.6.2.2	Test Requirements	2098

A.8	UE Measurements Procedures	2099

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2099

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2099

A.8.1.1.1	Test Purpose and Environment	2099

A.8.1.1.2	Test Requirements	2100

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2101

A.8.1.2.1	Test Purpose and Environment	2101

A.8.1.2.2	Test Requirements	2102

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2103

A.8.1.3.1	Test Purpose and Environment	2103

A.8.1.3.2	Test Requirements	2105

A.8.1.4	Void	2105

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2105

A.8.1.5.1	Test Purpose and Environment	2105

A.8.1.5.2	Test Requirements	2107

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2107

A.8.1.6.1	Test Purpose and Environment	2107

A.8.1.6.2	Test Requirements	2109

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2109

A.8.1.7.1	Test Purpose and Environment	2109

A.8.1.7.2	Test Requirements	2111

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2112

A.8.1.8.1	Test Purpose and Environment	2112

A.8.1.8.2	Test Requirements	2114

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2115

A.8.1.9.1	Test Purpose and Environment	2115

A.8.1.9.2	Test Requirements	2115

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2115

A.8.1.10.1	Test Purpose and Environment	2115

A.8.1.10.2	Test Requirements	2116

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2116

A.8.1.11.1	Test Purpose and Environment	2116

A.8.1.11.2	Test Requirements	2118

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2119

A.8.1.12.1	Test Purpose and Environment	2119

A.8.1.12.2	Test Requirements	2120

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2121

A.8.1.13.1	Test Purpose and Environment	2121

A.8.1.13.2	Test Requirements	2123

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2123

A.8.1.14.1	Test Purpose and Environment	2123

A.8.1.14.2	Test Requirements	2125

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2126

A.8.1.15.1	Test Purpose and Environment	2126

A.8.1.15.2	Test Requirements	2127

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2128

A.8.1.16.1	Test Purpose and Environment	2128

A.8.1.16.2	Test Requirements	2130

A.8.1.17	Void	2130

A.8.1.18	Void	2130

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2130

A.8.1.19.1	Test Purpose and Environment	2130

A.8.1.19.2	Test Requirements	2132

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2133

A.8.1.20.1	Test Purpose and Environment	2133

A.8.1.20.2	Test Requirements	2135

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2135

A.8.1.21.1	Test Purpose and Environment	2135

A.8.1.21.2	Test Requirements	2137

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2138

A.8.1.22.1	Test Purpose and Environment	2138

A.8.1.22.2	Test Requirements	2140

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2140

A.8.1.23.1	Test Purpose and Environment	2140

A.8.1.23.2	Test Requirements	2141

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2142

A.8.1.24.1	Test Purpose and Environment	2142

A.8.1.24.2	Test Requirements	2144

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2144

A.8.1.25.1	Test Purpose and Environment	2144

A.8.1.25.2	Test Requirements	2146

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2146

A.8.1.26.1	Test Purpose and Environment	2146

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2149

A.8.1.27.1	Test Purpose and Environment	2149

A.8.1.27.2	Test Requirements	2151

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2151

A.8.1.28.1	Test Purpose and Environment	2151

A.8.1.28.2	Test Requirements	2153

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2153

A.8.1.29.1	Test Purpose and Environment	2153

A.8.1.29.2	Test Requirements	2155

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2156

A.8.1.30.1	Test Purpose and Environment	2156

A.8.1.30.2	Test Requirements	2158

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2158

A.8.1.31.1	Test Purpose and Environment	2158

A.8.1.31.2	Test Requirements	2160

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2161

A.8.1.32.1	Test Purpose and Environment	2161

A.8.1.32.2	Test Requirements	2162

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2163

A.8.1.33.1	Test Purpose and Environment	2163

A.8.1.33.2	Test Requirements	2164

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2165

A.8.1.34.1	Test Purpose and Environment	2165

A.8.1.34.2	Test Requirements	2166

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2167

A.8.1.35.1	Test Purpose and Environment	2167

A.8.1.35.2	Test Requirements	2168

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2169

A.8.1.36.1	Test Purpose and Environment	2169

A.8.1.36.2	Test Requirements	2170

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2171

A.8.1.37.1	Test Purpose and Environment	2171

A.8.1.37.2	Test Requirements	2173

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2173

A.8.1.38.1	Test Purpose and Environment	2173

A.8.1.38.2	Test Requirements	2174

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2175

A.8.1.39.1	Test Purpose and Environment	2175

A.8.1.39.2	Test Requirements	2177

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2177

A.8.1.40.1	Test Purpose and Environment	2177

A.8.1.40.2	Test Requirements	2179

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2179

A.8.1.41.1	Test Purpose and Environment	2179

A.8.1.41.2	Test Requirement	2181

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2181

A.8.1.42.1	Test Purpose and Environment	2181

A.8.1.42.2	Test Requirement	2182

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2182

A.8.1.43.1	Test Purpose and Environment	2182

A.8.1.43.2	Test Requirements	2185

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2185

A.8.1.44.1	Test Purpose and Environment	2185

A.8.1.44.2	Test Requirements	2188

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2189

A.8.1.45.1	Test Purpose and Environment	2189

A.8.1.45.2	Test Requirements	2191

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2192

A.8.1.46.1	Test Purpose and Environment	2192

A.8.1.46.2	Test Requirements	2193

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2194

A.8.1.47.1	Test Purpose and Environment	2194

A.8.1.47.2	Test Requirements	2196

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2197

A.8.1.48.1	Test Purpose and Environment	2197

A.8.1.48.2	Test Requirements	2199

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2200

A.8.1.49.1	Test Purpose and Environment	2200

A.8.1.49.2	Test Requirements	2201

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2202

A.8.1.50.1	Test Purpose and Environment	2202

A.8.1.50.2	Test Requirements	2204

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2205

A.8.1.51.1	Test Purpose and Environment	2205

A.8.1.51.2	Test Requirements	2207

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2208

A.8.1.52.1	Test Purpose and Environment	2208

A.8.1.52.2	Test Requirements	2209

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2210

A.8.1.53.1	Test Purpose and Environment	2210

A.8.1.53.2	Test Requirements	2212

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2213

A.8.1.54.1	Test Purpose and Environment	2213

A.8.1.54.2	Test Requirements	2215

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2216

A.8.1.55.1	Test Purpose and Environment	2216

A.8.1.55.2	Test Requirements	2217

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2218

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2218

A.8.2.1.1	Test Purpose and Environment	2218

A.8.2.1.2	Test Requirements	2219

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2219

A.8.2.2.1	Test Purpose and Environment	2219

A.8.2.2.2	Test Requirements	2222

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2222

A.8.2.3.1	Test Purpose and Environment	2222

A.8.2.3.2	Test Requirements	2224

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2225

A.8.2.4.1	Test Purpose and Environment	2225

A.8.2.4.2	Test Requirements	2227

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2227

A.8.2.5.1	Test Purpose and Environment	2227

A.8.2.5.2	Test Requirements	2229

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2230

A.8.2.6.1	Test Purpose and Environment	2230

A.8.2.6.2	Test Requirements	2233

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2234

A.8.2.7.1	Test Purpose and Environment	2234

A.8.2.7.2	Test Requirements	2235

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2236

A.8.2.8.1	Test Purpose and Environment	2236

A.8.2.8.2	Test Requirements	2238

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2238

A.8.2.9.1	Test Purpose and Environment	2238

A.8.2.9.2	Test Requirements	2240

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2241

A.8.2.10.1	Test Purpose and Environment	2241

A.8.2.10.2	Test Requirements	2243

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2243

A.8.2.11.1	Test Purpose and Environment	2243

A.8.2.11.2	Test Requirements	2245

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2245

A.8.2.12.1	Test Purpose and Environment	2245

A.8.2.12.2	Test Requirements	2247

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2248

A.8.2.13.1	Test Purpose and Environment	2248

A.8.2.13.2	Test Requirements	2250

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2250

A.8.2.14.1	Test Purpose and Environment	2250

A.8.2.14.2	Test Requirement	2252

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2252

A.8.2.15.1	Test Purpose and Environment	2252

A.8.2.15.2	Test Requirements	2254

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2254

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2254

A.8.3.1.1	Test Purpose and Environment	2254

A.8.3.1.2	Test Requirements	2256

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2256

A.8.3.2.1	Test Purpose and Environment	2256

A.8.3.2.2	Test Requirements	2259

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2259

A.8.3.3.1	Test Purpose and Environment	2259

A.8.3.3.2	Test Requirements	2262

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2262

A.8.3.4.1	Test Purpose and Environment	2262

A.8.3.4.2	Test Requirements	2264

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2264

A.8.3.5.2	Test Requirements	2266

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2266

A.8.3.6.1	Test Purpose and Environment	2266

A.8.3.6.2	Test Requirements	2268

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2269

A.8.3.7.1	Test Purpose and Environment	2269

A.8.3.7.2	Test Requirements	2271

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2272

A.8.3.8.1	Test Purpose and Environment	2272

A.8.3.8.2	Test Requirements	2275

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2275

A.8.3.9.1	Test Purpose and Environment	2275

A.8.3.9.2	Test Requirements	2279

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2280

A.8.3.10.1	Test Purpose and Environment	2280

A.8.3.10.2	Test Requirements	2281

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2281

A.8.3.11.1	Test Purpose and Environment	2281

A.8.3.11.2	Test Requirement	2283

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2284

A.8.3.12.1	Test Purpose and Environment	2284

A.8.3.12.2	Test Requirement	2285

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2286

A.8.3.13.1	Test Purpose and Environment	2286

A.8.3.13.2	Test Requirement	2287

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2288

A.8.3.14.1	Test Purpose and Environment	2288

A.8.3.14.2	Test Requirement	2289

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2290

A.8.3.15.1	Test Purpose and Environment	2290

A.8.3.15.2	Test Requirement	2291

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2292

A.8.3.16.1	Test Purpose and Environment	2292

A.8.3.16.2	Test Requirement	2294

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2294

A.8.3.17.1	Test Purpose and Environment	2294

A.8.3.17.2	Test Requirement	2297

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2297

A.8.3.18.1	Test Purpose and Environment	2297

A.8.3.18.2	Test Requirement	2300

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2300

A.8.3.19.1	Test Purpose and Environment	2300

A.8.3.19.2	Test Requirement	2303

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2303

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2303

A.8.4.1.1	Test Purpose and Environment	2303

A.8.4.1.2	Test Requirements	2305

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2305

A.8.4.2.1	Test Purpose and Environment	2305

A.8.4.2.2	Test Requirements	2308

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2308

A.8.4.3.1	Test Purpose and Environment	2308

A.8.4.3.2	Test Requirements	2310

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2311

A.8.4.4.1	Test Purpose and Environment	2311

A.8.4.4.2	Test Requirements	2313

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2314

A.8.4.5.1	Test Purpose and Environment	2314

A.8.4.5.2	Test Requirements	2316

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2316

A.8.4.6.1	Test Purpose and Environment	2316

A.8.4.6.2	Test Requirements	2317

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2317

A.8.4.7.1	Test Purpose and Environment	2317

A.8.4.7.2	Test Requirements	2321

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2321

A.8.4.8.1	Test Purpose and Environment	2321

A.8.4.8.2	Test Requirements	2324

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2324

A.8.4.9.1	Test Purpose and Environment	2324

A.8.4.9.2	Test Requirements	2329

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2329

A.8.4.10.1	Test Purpose and Environment	2329

A.8.4.10.2	Test Requirements	2331

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2331

A.8.4.11.1	Test Purpose and Environment	2331

A.8.4.11.2	Test Requirement	2333

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2333

A.8.4.12.1	Test Purpose and Environment	2333

A.8.4.12.2	Test Requirement	2335

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2336

A.8.4.13.1	Test Purpose and Environment	2336

A.8.4.13.2	Test Requirement	2337

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2338

A.8.4.14.1	Test Purpose and Environment	2338

A.8.4.14.2	Test Requirement	2340

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2340

A.8.4.15.1	Test Purpose and Environment	2340

A.8.4.15.2	Test Requirement	2343

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2343

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2343

A.8.5.1.1	Test Purpose and Environment	2343

A.8.5.1.2	Test Requirements	2345

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2345

A.8.5.2.1	Test Purpose and Environment	2345

A.8.5.2.2	Test Requirements	2347

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2347

A.8.5.3.1	Test Purpose and Environment	2347

A.8.5.3.2	Test Requirements	2350

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2350

A.8.5.4.1	Test Purpose and Environment	2350

A.8.5.4.2	Test Requirements	2352

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2352

A.8.5.5.1	Test Purpose and Environment	2352

A.8.5.5.2	Test Requirements	2355

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2355

A.8.5.6.1	Test Purpose and Environment	2355

A.8.5.6.2	Test Requirements	2356

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2357

A.8.5.7.1	Test Purpose and Environment	2357

A.8.5.7.2	Test Requirements	2357

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2357

A.8.5.8.1	Test Purpose and Environment	2357

A.8.5.8.2	Test Requirements	2360

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2360

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2360

A.8.6.1.1	Test Purpose and Environment	2360

A.8.6.1.2	Test Requirements	2362

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2362

A.8.6.2.1	Test Purpose and Environment	2362

A.8.6.2.2	Test Requirements	2365

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2365

A.8.6.3.1	Test Purpose and Environment	2365

A.8.6.3.2	Test Requirements	2368

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2368

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2368

A.8.7.1.1	Test Purpose and Environment	2368

A.8.7.1.1.1	Void	2368

A.8.7.1.1.2	1.28 Mcps TDD option	2368

A.8.7.1.1.3	Void	2370

A.8.7.1.2	Test Requirements	2370

A.8.7.1.2.1	Void	2370

A.8.7.1.2.2	1.28 Mcps TDD option	2370

A.8.7.1.2.3	Void	2370

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2370

A.8.7.2.1	Test Purpose and Environment	2370

A.8.7.2.2	Test Requirements	2373

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2374

A.8.7.3.1	Test Purpose and Environment	2374

A.8.7.3.2	Test Parameters	2374

A.8.7.3.3	Test Requirements	2375

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2376

A.8.7.4.1	Test Purpose and Environment	2376

A.8.7.4.2	Test Requirements	2378

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2378

A.8.7.5.1	Test Purpose and Environment	2378

A.8.7.5.2	Test Requirements	2380

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2380

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2380

A.8.7A.1.1	Test Purpose and Environment	2380

A.8.7A.1.2	Test Requirements	2383

A.8.8	E-UTRAN FDD – GSM Measurements	2383

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2383

A.8.8.1.1	Test Purpose and Environment	2383

A.8.8.1.2	Test Requirements	2385

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2385

A.8.8.2.1	Test Purpose and Environment	2385

A.8.8.2.2	Test Requirements	2387

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2388

A.8.8.3.1	Test Purpose and Environment	2388

A.8.8.3.2	Test Requirements	2389

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2390

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2390

A.8.9.1.1	Test Purpose and Environment	2390

A.8.9.1.2	Test Requirements	2391

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2392

A.8.9.2.1	Test Purpose and Environment	2392

A.8.9.2.2	Test Requirements	2394

A.8.10	E-UTRAN TDD – GSM Measurements	2394

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2394

A.8.10.1.1	Test Purpose and Environment	2394

A.8.10.1.2	Test Requirements	2395

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2396

A.8.10.2.1	Test Purpose and Environment	2396

A.8.10.2.2	Test Requirements	2398

A.8.11	Monitoring of Multiple Layers	2398

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2398

A.8.11.1.1	Test Purpose and Environment	2398

A.8.11.1.2	Test Requirements	2400

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2401

A.8.11.2.1	Test Purpose and Environment	2401

A.8.11.2.2	Test Requirements	2402

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2403

A.8.11.3.1	Test Purpose and Environment	2403

A.8.11.3.2	Test Requirements	2405

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2405

A.8.11.4.1	Test Purpose and Environment	2405

A.8.11.4.2	Test Requirements	2408

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2408

A.8.11.5.1	Test Purpose and Environment	2408

A.8.11.5.2	Test Requirements	2410

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2411

A.8.11.6.1	Test Purpose and Environment	2411

A.8.11.6.2	Test Requirements	2413

A.8.12	RSTD Intra-frequency Measurements	2414

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2414

A.8.12.1.1	Test Purpose and Environment	2414

A.8.12.1.2	Test Requirements	2418

A.8.12.1.2A	Test Requirements for UE Category 1bis	2418

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2418

A.8.12.2.1	Test Purpose and Environment	2418

A.8.12.2.2	Test Requirements	2423

A.8.12.2.2A	Test Requirements for UE Category 1bis	2423

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2423

A.8.12.3.1	Test Purpose and Environment	2423

A.8.12.3.2	Test Requirements	2429

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2429

A.8.12.4.1	Test Purpose and Environment	2429

A.8.12.4.2	Test Requirements	2434

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2434

A.8.12.5.1	Test Purpose and Environment	2434

A.8.12.5.2	Test Requirements	2439

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2439

A.8.12.6.1	Test Purpose and Environment	2439

A.8.12.6.2	Test Requirements	2444

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2444

A.8.12.7.1	Test Purpose and Environment	2444

A.8.12.7.2	Test Requirements	2449

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2449

A.8.12.8.1	Test Purpose and Environment	2449

A.8.12.8.2	Test Requirements	2454

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2454

A.8.12.9.1	Test Purpose and Environment	2454

A.8.12.9.2	Test Requirements	2459

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2459

A.8.12.10.1	Test Purpose and Environment	2459

A.8.12.10.2	Test Requirements	2464

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2464

A.8.12.11.1	Test Purpose and Environment	2464

A.8.12.11.2	Test Requirements	2469

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2469

A.8.12.12.1	Test Purpose and Environment	2469

A.8.12.12.2	Test Requirements	2474

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2474

A.8.12.13.1	Test Purpose and Environment	2474

A.8.12.13.2	Test Requirements	2479

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2479

A.8.12.14.1	Test Purpose and Environment	2479

A.8.12.14.2	Test Requirements	2484

A.8.13	RSTD Inter-frequency Measurements	2484

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2484

A.8.13.1.1	Test Purpose and Environment	2484

A.8.13.1.2	Test Requirements	2489

A.8.13.1.2A	Test Requirements for UE Category 1bis	2489

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2489

A.8.13.2.1	Test Purpose and Environment	2489

A.8.13.2.2	Test Requirements	2495

A.8.13.2.2A	Test Requirements for UE Category 1bis	2495

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2495

A.8.13.3.1	Test Purpose and Environment	2495

A.8.13.3.2	Test Requirements	2502

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2502

A.8.13.4.1	Test Purpose and Environment	2502

A.8.13.4.2	Test Requirements	2508

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2508

A.8.13.5.1	Test Purpose and Environment	2508

A.8.13.5.2	Test Requirements	2514

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2514

A.8.13.6.1	Test Purpose and Environment	2514

A.8.13.6.2	Test Requirements	2520

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2520

A.8.13.7.1	Test Purpose and Environment	2520

A.8.13.7.2	Test Requirements	2526

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2526

A.8.13.8.1	Test Purpose and Environment	2526

A.8.13.8.2	Test Requirements	2532

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2532

A.8.13.9.1	Test Purpose and Environment	2532

A.8.13.9.2	Test Requirements	2538

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2538

A.8.13.10.1	Test Purpose and Environment	2538

A.8.13.10.2	Test Requirements	2544

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2544

A.8.13.11.1	Test Purpose and Environment	2544

A.8.13.11.2	Test Requirements	2550

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2550

A.8.13.12.1	Test Purpose and Environment	2550

A.8.13.12.2	Test Requirements	2556

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2556

A.8.13.13.1	Test Purpose and Environment	2556

A.8.13.13.2	Test Requirements	2562

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2562

A.8.13.14.1	Test Purpose and Environment	2562

A.8.13.14.2	Test Requirements	2568

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2568

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2568

A.8.14.1.1	Test Purpose and Environment	2568

A.8.14.1.2	Test Requirements	2570

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2570

A.8.14.2.1	Test Purpose and Environment	2570

A.8.14.2.2	Test Requirements	2573

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2573

A.8.14.3.1	Test Purpose and Environment	2573

A.8.14.3.2	Test Requirements	2575

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2576

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2576

A.8.15.1.1	Test Purpose and Environment	2576

A.8.15.1.2	Test Requirements	2577

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2577

A.8.15.2.1	Test Purpose and Environment	2577

A.8.15.2.2	Test Requirements	2580

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2580

A.8.15.3.1	Test Purpose and Environment	2580

A.8.15.3.2	Test Requirements	2582

A.8.16	E-UTRAN Carrier Aggregation Measurements	2583

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2583

A.8.16.1.1	Test Purpose and Environment	2583

A.8.16.1.2	Test Requirements	2585

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2585

A.8.16.2.1	Test Purpose and Environment	2585

A.8.16.2.2	Test Requirements	2587

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2587

A.8.16.3.1	Test Purpose and Environment	2587

A.8.16.3.2	Test Requirements	2589

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2590

A.8.16.3A.1	Test Purpose and Environment	2590

A.8.16.3A.2	Test Requirements	2591

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2592

A.8.16.4.1	Test Purpose and Environment	2592

A.8.16.4.2	Test Requirements	2593

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2594

A.8.16.4A.1	Test Purpose and Environment	2594

A.8.16.4A.2	Test Requirements	2596

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2597

A.8.16.5.1	Test Purpose and Environment	2597

A.8.16.5.2	Test Requirements	2597

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2597

A.8.16.6.1	Test Purpose and Environment	2597

A.8.16.6.2	Test Requirements	2598

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2598

A.8.16.7.1	 Test Purpose and Environment	2598

A.8.16.7.2	 Test Requirements	2599

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2599

A.8.16.8.1	 Test Purpose and Environment	2599

A.8.16.8.2	 Test Requirements	2600

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2600

A.8.16.9.1	Test Purpose and Environment	2600

A.8.16.9.2	Test Requirements	2601

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2601

A.8.16.10.1	Test Purpose and Environment	2601

A.8.16.10.2	Test Requirements	2602

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2602

A.8.16.11.1	 Test Purpose and Environment	2602

A.8.16.11.2	 Test Requirements	2603

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2603

A.8.16.12.1	 Test Purpose and Environment	2603

A.8.16.12.2	 Test Requirements	2603

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2603

A.8.16.13.1	Test Purpose and Environment	2603

A.8.16.13.2	Test Requirements	2604

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2604

A.8.16.14.1	Test Purpose and Environment	2604

A.8.16.14.2	Test Requirements	2605

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2605

A.8.16.15.1	 Test Purpose and Environment	2605

A.8.16.7.2	 Test Requirements	2605

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2605

A.8.16.16.1	 Test Purpose and Environment	2605

A.8.16.16.2	 Test Requirements	2606

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2606

A.8.16.17.1	Test Purpose and Environment	2606

A.8.16.17.2	Test Requirements	2608

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2609

A.8.16.17A.1	Test Purpose and Environment	2609

A.8.16.17A.2	Test Requirements	2609

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2609

A.8.16.17B.1	Test Purpose and Environment	2609

A.8.16.17B.2	Test Requirements	2610

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2610

A.8.16.17C.1	Test Purpose and Environment	2610

A.8.16.17C.2	Test Requirements	2610

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2610

A.8.16.18.1	Test Purpose and Environment	2610

A.8.16.18.2	Test Requirements	2612

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2613

A.8.16.18A.1	Test Purpose and Environment	2613

A.8.16.18A.2	Test Requirements	2613

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2613

A.8.16.18B.1	Test Purpose and Environment	2613

A.8.16.18B.2	Test Requirements	2614

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2614

A.8.16.18C.1	Test Purpose and Environment	2614

A.8.16.18C.2	Test Requirements	2614

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2614

A.8.16.18D.1	Test Purpose and Environment	2614

A.8.16.18D.2	Test Requirements	2615

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2615

A.8.16.19.1	Test Purpose and Environment	2615

A.8.16.19.2	Test Requirements	2617

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2617

A.8.16.19A.1	Test Purpose and Environment	2617

A.8.16.19A.2	Test Requirements	2618

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2618

A.8.16.19B.1	Test Purpose and Environment	2618

A.8.16.19B.2	Test Requirements	2618

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2618

A.8.16.19C.1	Test Purpose and Environment	2618

A.8.16.19C.2	Test Requirements	2619

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2619

A.8.16.20.1	Test Purpose and Environment	2619

A.8.16.20.2	Test Requirements	2621

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2622

A.8.16.20A.1	Test Purpose and Environment	2622

A.8.16.20A.2	Test Requirements	2622

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2622

A.8.16.20B.1	Test Purpose and Environment	2622

A.8.16.20B.2	Test Requirements	2623

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2623

A.8.16.20C.1	Test Purpose and Environment	2623

A.8.16.20C.2	Test Requirements	2623

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2623

A.8.16.20D.1	Test Purpose and Environment	2623

A.8.16.20D.2	Test Requirements	2624

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2624

A.8.16.21.1	Test Purpose and Environment	2624

A.8.16.21.2	Test Requirements	2626

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2626

A.8.16.22.1	Test Purpose and Environment	2626

A.8.16.22.2	Test Requirements	2628

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2628

A.8.16.23.1	Test Purpose and Environment	2628

A.8.16.23.2	Test Requirements	2631

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2631

A.8.16.24.1	Test Purpose and Environment	2631

A.8.16.24.2	Test Requirements	2634

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2634

A.8.16.25.1	Test Purpose and Environment	2634

A.8.16.25.2	Test Requirements	2637

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2637

A.8.16.26.1	Test Purpose and Environment	2637

A.8.16.26.2	Test Requirements	2640

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2640

A.8.16.27.1	Test Purpose and Environment	2640

A.8.16.27.2	Test Requirements	2645

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2645

A.8.16.28.1	Test Purpose and Environment	2645

A.8.16.28.2	Test Requirements	2650

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2650

A.8.16.29.1	Test Purpose and Environment	2650

A.8.16.29.2	Test Requirements	2655

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2655

A.8.16.30.1	Test Purpose and Environment	2655

A.8.16.30.2	Test Requirements	2660

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2660

A.8.16.31.1	Test Purpose and Environment	2660

A.8.16.31.2	Test Requirements	2665

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2665

A.8.16.32.1	Test Purpose and Environment	2665

A.8.16.32.2	Test Requirements	2670

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2670

A.8.16.33.1	Test Purpose and Environment	2670

A.8.16.33.2	Test Requirements	2675

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2675

A.8.16.34.1	Test Purpose and Environment	2675

A.8.16.34.2	Test Requirements	2680

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2680

A.8.16.35.1	Test Purpose and Environment	2680

A.8.16.35.2	Test Requirements	2682

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2683

A.8.16.36.1	Test Purpose and Environment	2683

A.8.16.36.2	Test Requirements	2685

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2686

A.8.16.37.1	Test Purpose and Environment	2686

A.8.16.37.2	Test Requirements	2688

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2689

A.8.16.38.1	Test Purpose and Environment	2689

A.8.16.38.2	Test Requirements	2691

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2692

A.8.16.39.1	Test Purpose and Environment	2692

A.8.16.39.2	Test Requirements	2695

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2695

A.8.16.40.1	Test Purpose and Environment	2695

A.8.16.40.2	Test Requirements	2698

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2698

A.8.16.41.1	Test Purpose and Environment	2698

A.8.16.41.2	Test Requirements	2700

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2701

A.8.16.42.1	Test Purpose and Environment	2701

A.8.16.42.2	Test Requirements	2703

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2704

A.8.16.43.1	Test Purpose and Environment	2704

A.8.16.43.2	Test Requirements	2707

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2707

A.8.16.44.1	Test Purpose and Environment	2707

A.8.16.44.2	Test Requirements	2710

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2710

A.8.16.45.1	Test Purpose and Environment	2710

A.8.16.45.2	Test Requirements	2713

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2713

A.8.16.46.1	Test Purpose and Environment	2713

A.8.16.46.2	Test Requirements	2715

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2716

A.8.16.47.1	Test Purpose and Environment	2716

A.8.16.47.2	Test Requirements	2718

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2719

A.8.16.48.1	Test Purpose and Environment	2719

A.8.16.48.2	Test Requirements	2721

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2722

A.8.16.49.1	Test Purpose and Environment	2722

A.8.16.49.2	Test Requirements	2725

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2725

A.8.16.50.1	Test Purpose and Environment	2725

A.8.16.50.2	Test Requirements	2727

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2728

A.8.16.51.1	Test Purpose and Environment	2728

A.8.16.51.2	Test Requirements	2733

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2733

A.8.16.52.1	Test Purpose and Environment	2733

A.8.16.52.2	Test Requirements	2738

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2738

A.8.16.53.1	Test Purpose and Environment	2738

A.8.16.53.2	Test Requirements	2742

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2742

A.8.16.54.1	Test Purpose and Environment	2742

A.8.16.54.2	Test Requirements	2746

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2746

A.8.16.55.1	Test Purpose and Environment	2746

A.8.16.55.2	Test Requirements	2752

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2752

A.8.16.56.1	Test Purpose and Environment	2752

A.8.16.56.2	Test Requirements	2758

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2758

A.8.16.57.1	Test Purpose and Environment	2758

A.8.16.57.2	Test Requirements	2760

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2761

A.8.16.58.1	Test Purpose and Environment	2761

A.8.16.58.2	Test Requirements	2763

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2764

A.8.16.59.1	Test Purpose and Environment	2764

A.8.16.59.2	Test Requirements	2768

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2769

A.8.16.60.1	Test Purpose and Environment	2769

A.8.16.60.2	Test Requirements	2772

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2773

A.8.16.61.1	Test Purpose and Environment	2773

A.8.16.61.2	Test Requirements	2776

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2776

A.8.16.62.1	Test Purpose and Environment	2776

A.8.16.62.2	Test Requirements	2778

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2779

A.8.16.63.1	Test Purpose and Environment	2779

A.8.16.63.2	Test Requirements	2782

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2783

A.8.16.64.1	Test Purpose and Environment	2783

A.8.16.64.2	Test Requirements	2787

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2787

A.8.16.65.1	Test Purpose and Environment	2787

A.8.16.65.2	Test Requirements	2792

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2792

A.8.16.66.1	Test Purpose and Environment	2792

A.8.16.66.2	Test Requirements	2797

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2797

A.8.16.67.1	Test Purpose and Environment	2797

A.8.16.67.2	Test Requirements	2802

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2802

A.8.16.68.1	Test Purpose and Environment	2802

A.8.16.68.2	Test Requirements	2807

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2807

A.8.16.69.1	Test Purpose and Environment	2807

A.8.16.69.2	Test Requirements	2811

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2811

A.8.16.70.1	Test Purpose and Environment	2811

A.8.16.70.2	Test Requirements	2815

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2815

A.8.16.71.1	Test Purpose and Environment	2815

A.8.16.71.2	Test Requirements	2820

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2820

A.8.16.72.1	Test Purpose and Environment	2820

A.8.16.72.2	Test Requirements	2825

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2825

A.8.16.73.1	Test Purpose and Environment	2825

A.8.16.73.2	Test Requirements	2831

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2831

A.8.16.74.1	Test Purpose and Environment	2831

A.8.16.74.2	Test Requirements	2837

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2837

A.8.16.75.1	Test Purpose and Environment	2837

A.8.16.75.2	Test Requirements	2843

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2843

A.8.16.76.1	Test Purpose and Environment	2843

A.8.16.76.2	Test Requirements	2849

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2849

A.8.16.77.1	Test Purpose and Environment	2849

A.8.16.77.2	Test Requirements	2853

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2853

A.8.16.78.1	Test Purpose and Environment	2853

A.8.16.78.2	Test Requirements	2855

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2856

A.8.16.79.1	Test Purpose and Environment	2856

A.8.16.79.2	Test Requirements	2863

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2863

A.8.16.80.1	Test Purpose and Environment	2863

A.8.16.80.2	Test Requirements	2870

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2870

A.8.16.81.1	Test Purpose and Environment	2870

A.8.16.81.2	Test Requirements	2876

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2876

A.8.16.82.1	Test Purpose and Environment	2876

A.8.16.82.2	Test Requirements	2882

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2882

A.8.16.83.1	Test Purpose and Environment	2882

A.8.16.83.2	Test Requirements	2887

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2887

A.8.16.84.1	Test Purpose and Environment	2887

A.8.16.84.2	Test Requirements	2893

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2893

A.8.16.85.1	Test Purpose and Environment	2893

A.8.16.85.2	Test Requirements	2897

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2897

A.8.16.86.1	Test Purpose and Environment	2897

A.8.16.86.2	Test Requirements	2900

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2900

A.8.16.87.1	Test Purpose and Environment	2900

A.8.16.87.2	Test Requirements	2906

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2906

A.8.16.88.1	Test Purpose and Environment	2906

A.8.16.88.2	Test Requirements	2914

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2914

A.8.16.89.1	Test Purpose and Environment	2914

A.8.16.89.2	Test Requirements	2921

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2921

A.8.16.90.1	Test Purpose and Environment	2921

A.8.16.90.2	Test Requirements	2928

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2928

A.8.16.91.1	Test Purpose and Environment	2928

A.8.16.91.2	Test Requirements	2934

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2934

A.8.16.92.1	Test Purpose and Environment	2934

A.8.16.92.2	Test Requirements	2942

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2942

A.8.16.93.1	Test Purpose and Environment	2942

A.8.16.93.2	Test Requirements	2949

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2949

A.8.16.94.1	Test Purpose and Environment	2949

A.8.16.94.2	Test Requirements	2956

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2956

A.8.16.95.1	Test Purpose and Environment	2956

A.8.16.95.2	Test Requirements	2964

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2964

A.8.16.96.1	Test Purpose and Environment	2964

A.8.16.96.2	Test Requirements	2973

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2973

A.8.16.97.1	Test Purpose and Environment	2973

A.8.16.97.2	Test Requirements	2980

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2980

A.8.16.98.1	Test Purpose and Environment	2980

A.8.16.98.2	Test Requirements	2988

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2988

A.8.16.99.1	Test Purpose and Environment	2988

A.8.16.99.2	Test Requirements	2998

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2998

A.8.16.100.1	Test Purpose and Environment	2998

A.8.16.100.2	Test Requirements	3008

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3008

A.8.16.101.1	Test Purpose and Environment	3008

A.8.16.101.2	Test Requirements	3018

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3018

A.8.16.102.1	Test Purpose and Environment	3018

A.8.16.102.2	Test Requirements	3029

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3029

A.8.16.103.1	Test Purpose and Environment	3029

A.8.16.103.2	Test Requirements	3034

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3035

A.8.16.104.1	Test Purpose and Environment	3035

A.8.16.104.2	Test Requirements	3039

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3040

A.8.16.105.1	Test Purpose and Environment	3040

A.8.16.105.2	Test Requirements	3044

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3045

A.8.16.106.1	Test Purpose and Environment	3045

A.8.16.106.2	Test Requirements	3049

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3049

A.8.16.107.1	Test Purpose and Environment	3049

A.8.16.107.2	Test Requirements	3051

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3052

A.8.16.108.1	Test Purpose and Environment	3052

A.8.16.108.2	Test Requirements	3054

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3055

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3055

A.8.17.1.1	Test Purpose and Environment	3055

A.8.17.1.2	Test Requirements	3061

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3061

A.8.17.2.1	Test Purpose and Environment	3061

A.8.17.2.2	Test Requirements	3068

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3068

A.8.17.3.1	Test Purpose and Environment	3068

A.8.17.3.2	Test Requirements	3069

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3069

A.8.17.4.1	Test Purpose and Environment	3069

A.8.17.4.2	Test Requirements	3070

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3070

A.8.17.5.1	Test Purpose and Environment	3070

A.8.17.5.2	Test Requirements	3071

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3071

A.8.17.6.1	Test Purpose and Environment	3071

A.8.17.6.2	Test Requirements	3072

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3073

A.8.17.7.1	Test Purpose and Environment	3073

A.8.17.7.2	Test Requirements	3073

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3074

A.8.17.8.1	Test Purpose and Environment	3074

A.8.17.8.2	Test Requirements	3074

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3075

A.8.17.9.1	Test Purpose and Environment	3075

A.8.17.9.2	Test Requirements	3076

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3076

A.8.17.10.1	Test Purpose and Environment	3076

A.8.17.10.2	Test Requirements	3083

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3084

A.8.17.11.1	Test Purpose and Environment	3084

A.8.17.11.2	Test Requirements	3090

A.8.18	E-UTRAN TDD – HRPD Measurements	3091

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3091

A.8.18.1.1	Test Purpose and Environment	3091

A.8.18.1.2	Test Requirements	3093

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3093

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3093

A.8.19.1.1	Test Purpose and Environment	3093

A.8.19.1.2	Test Requirements	3094

A.8.20	Inter-frequency/RAT Measurements in CA mode	3095

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3095

A.8.20.1.1	Test Purpose and Environment	3095

A.8.20.1.2	Test Requirements	3096

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3096

A.8.20.2.1	Test Purpose and Environment	3097

A.8.20.2.2	Test Requirements	3098

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3099

A.8.20.2A.1	Test Purpose and Environment	3099

A.8.20.2A.2	Test Requirements	3099

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3099

A.8.20.2B.1	Test Purpose and Environment	3099

A.8.20.2B.2	Test Requirements	3102

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3102

A.8.20.3.1	Test Purpose and Environment	3102

A.8.20.3.2	Test Requirements	3104

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3104

A.8.20.4.1	Test Purpose and Environment	3104

A.8.20.4.1.1	1.28 Mcps TDD option	3104

A.8.20.4.2	Test Requirements	3106

A.8.20.4.2.1	1.28 Mcps TDD option	3106

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3106

A.8.20.4A.1	Test Purpose and Environment	3106

A.8.20.4A.1.1	1.28 Mcps TDD option	3106

A.8.20.4A.2	Test Requirements	3106

A.8.20.4A.2.1	1.28 Mcps TDD option	3106

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3107

A.8.20.4B.1	Test Purpose and Environment	3107

A.8.20.4B.1.1	1.28 Mcps TDD option	3107

A.8.20.4B.2	Test Requirements	3109

A.8.20.4B.2.1	1.28 Mcps TDD option	3109

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3109

A.8.21.1	Test Purpose and Environment	3109

A.8.21.2 Test Requirements	3113

A.8.22	E-UTRAN Discovery Signal Measurements	3113

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3113

A.8.22.1.1	Test Purpose and Environment	3113

A.8.22.1.2	Test Requirements	3116

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3116

A.8.22.2.1	Test Purpose and Environment	3116

A.8.22.2.2	Test Requirements	3119

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3119

A.8.22.3.1	Test Purpose and Environment	3119

A.8.22.3.2	Test Requirements	3122

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3122

A.8.22.4.1	Test Purpose and Environment	3122

A.8.22.4.2	Test Requirements	3125

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3125

A.8.22.5.1	Test Purpose and Environment	3125

A.8.22.5.2	Test Requirements	3129

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3129

A.8.22.6.1	Test Purpose and Environment	3129

A.8.22.6.2	Test Requirements	3133

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3133

A.8.22.7.1	Test Purpose and Environment	3133

A.8.22.7.2	Test Requirements	3137

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3137

A.8.22.8.1	Test Purpose and Environment	3137

A.8.22.8.2	Test Requirements	3141

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3141

A.8.22.9.1	Test Purpose and Environment	3141

A.8.22.9.2	Test Requirements	3143

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3144

A.8.22.10.1	Test Purpose and Environment	3144

A.8.22.10.2	Test Requirements	3146

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3147

A.8.22.11.1	Test Purpose and Environment	3147

A.8.22.11.2	Test Requirements	3150

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3150

A.8.22.12.1	Test Purpose and Environment	3150

A.8.22.12.2	Test Requirements	3154

A.8.23	E-UTRAN Dual Connectivity Measurements	3154

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3154

A.8.23.1.1	Test Purpose and Environment	3154

A.8.23.1.2	Test Requirements	3157

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3157

A.8.23.2.1	Test Purpose and Environment	3157

A.8.23.2.2	Test Requirements	3160

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3160

A.8.23.3.1	Test Purpose and Environment	3160

A.8.23.3.2	Test Requirements	3163

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3163

A.8.23.4.1	Test Purpose and Environment	3163

A.8.23.4.2	Test Requirements	3166

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3166

A.8.23.5.1	Test Purpose and Environment	3166

A.8.23.5.2	Test Requirements	3169

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3169

A.8.23.6.1	Test Purpose and Environment	3169

A.8.23.6.2	Test Requirements	3172

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3172

A.8.23.7.1	Test Purpose and Environment	3172

A.8.23.7.2	Test Requirements	3174

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3175

A.8.23.8.1	Test Purpose and Environment	3175

A.8.23.8.2	Test Requirements	3177

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3178

A.8.23.9.1	Test Purpose and Environment	3178

A.8.23.9.2	Test Requirements	3181

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3181

A.8.23.10.1	Test Purpose and Environment	3181

A.8.23.10.2	Test Requirements	3184

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3184

A.8.23.11.1	Test Purpose and Environment	3184

A.8.23.11.2	Test Requirements	3187

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3187

A.8.23.12.1	Test Purpose and Environment	3187

A.8.23.12.2	Test Requirements	3190

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3190

A.8.23.13.1	Test Purpose and Environment	3190

A.8.23.13.2	Test Requirements	3193

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3193

A.8.23.14.1	Test Purpose and Environment	3193

A.8.23.14.2	Test Requirements	3196

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3196

A.8.23.15.1	Test Purpose and Environment	3196

A.8.23.15.2	Test Requirements	3199

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3199

A.8.23.16.1	Test Purpose and Environment	3199

A.8.23.16.2	Test Requirements	3200

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3201

A.8.23.17.1	Test Purpose and Environment	3201

A.8.23.17.2	Test Requirements	3203

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3203

A.8.23.18.1	Test Purpose and Environment	3203

A.8.23.18.2	Test Requirements	3204

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3205

A.8.23.19.1	Test Purpose and Environment	3205

A.8.23.19.2	Test Requirements	3206

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3207

A.8.23.20.1	Test Purpose and Environment	3207

A.8.23.20.2	Test Requirements	3208

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3209

A.8.23.21.1	Test Purpose and Environment	3209

A.8.23.21.2	Test Requirements	3210

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3211

A.8.23.22.1	Test Purpose and Environment	3211

A.8.23.22.2	Test Requirements	3212

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3213

A.8.23.23.1	Test Purpose and Environment	3213

A.8.23.23.2	Test Requirements	3214

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3215

A.8.23.24.1	Test Purpose and Environment	3215

A.8.23.24.2	Test Requirements	3217

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3218

A.8.23.25.1	Test Purpose and Environment	3218

A.8.23.25.2	Test Requirements	3220

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3221

A.8.23.26.1	Test Purpose and Environment	3221

A.8.23.26.2	Test Requirements	3223

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3224

A.8.23.27.1	Test Purpose and Environment	3224

A.8.23.27.2	Test Requirements	3227

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3227

A.8.23.28.1	Test Purpose and Environment	3227

A.8.23.28.2	Test Requirements	3231

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3231

A.8.23.29.1	Test Purpose and Environment	3231

A.8.23.29.2	Test Requirements	3235

A.8.24	Proximity-based Services	3235

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3235

A.8.24.1.1	Test Purpose and Environment	3235

A.8.24.1.2	Test Requirements	3236

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3237

A.8.24.2.1	Test Purpose and Environment	3237

A.8.24.2.2	Test Requirements	3238

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3238

A.8.24.3.1	Test Purpose and Environment	3239

A.8.24.3.2	Test Requirements	3240

A.8.25	E-UTRAN-WLAN Measurements	3241

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3241

A.8.25.1.1	Test Purpose and Environment	3241

A.8.25.1.2	Test Requirements	3243

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3243

A.8.25.2.1	Test Purpose and Environment	3243

A.8.25.2.2	Test Requirements	3245

A.8.26 Frame Structure 3 (FS3)	3245

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3245

A.8.26.1.1	Test Purpose and Environment	3245

A.8.26.1.2	Test Requirements	3247

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3248

A.8.26.2.1	Test Purpose and Environment	3248

A.8.26.2.2	Test Requirements	3250

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3251

A.8.26.3.1	Test Purpose and Environment	3251

A.8.26.3.2	Test Requirements	3254

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3254

A.8.26.3A.1	Test Purpose and Environment	3254

A.8.26.3A.2	Test Requirements	3257

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3257

A.8.26.4.1	Test Purpose and Environment	3257

A.8.26.4.2	Test Requirements	3261

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3261

A.8.26.4A.1	Test Purpose and Environment	3261

A.8.26.4A.2	Test Requirements	3265

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3265

A.8.26.5.1	Test Purpose and Environment	3265

A.8.26.5.2	Test Requirements	3268

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3268

A.8.26.5A.1	Test Purpose and Environment	3268

A.8.26.5A.2	Test Requirements	3272

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3272

A.8.26.6.1	Test Purpose and Environment	3272

A.8.26.6.2	Test Requirements	3276

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3276

A.8.26.6A.1	Test Purpose and Environment	3276

A.8.26.6A.2	Test Requirements	3280

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3280

A.8.26.7.1	Test Purpose and Environment	3280

A.8.26.7.2	Test Requirements	3283

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3283

A.8.26.8.1	Test Purpose and Environment	3283

A.8.26.8.2	Test Requirements	3287

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3287

A.8.26.9.1	Test Purpose and Environment	3287

A.8.26.9.2	Test Requirements	3290

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3290

A.8.26.10.1	Test Purpose and Environment	3290

A.8.26.10.2	Test Requirements	3293

A.9	Measurement Performance Requirements	3295

A.9.1	RSRP	3295

A.9.1.1	FDD Intra frequency case	3295

A.9.1.1.1	Test Purpose and Environment	3295

A.9.1.1.2	Test parameters	3295

A.9.1.1.3	Test Requirements	3298

A.9.1.2	TDD Intra frequency case	3298

A.9.1.2.1	Test Purpose and Environment	3298

A.9.1.2.2	Test parameters	3298

A.9.1.2.3	Test Requirements	3299

A.9.1.3	FDD—FDD Inter frequency case	3300

A.9.1.3.1	Test Purpose and Environment	3300

A.9.1.3.2	Test parameters	3300

A.9.1.3.3	Test Requirements	3303

A.9.1.4	TDD—TDD Inter frequency case	3303

A.9.1.4.1	Test Purpose and Environment	3303

A.9.1.4.2	Test parameters	3303

A.9.1.4.3	Test Requirements	3305

A.9.1.5	FDD—TDD Inter frequency case	3306

A.9.1.5.1	Test Purpose and Environment	3306

A.9.1.5.2	Test parameters	3306

A.9.1.5.3	Test Requirements	3307

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3308

A.9.1.6.1	Test Purpose and Environment	3308

A.9.1.6.2	Test parameters	3308

A.9.1.6.3	Test Requirements	3311

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3311

A.9.1.7.1	Test Purpose and Environment	3311

A.9.1.7.2	Test parameters	3311

A.9.1.7.3	Test Requirements	3314

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3314

A.9.1.8.1	Test Purpose and Environment	3314

A.9.1.8.2	Test parameters	3314

A.9.1.8.3	Test Requirements	3318

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3318

A.9.1.9.1	Test Purpose and Environment	3318

A.9.1.9.2	Test parameters	3318

A.9.1.9.3	Test Requirements	3321

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3321

A.9.1.10.1	Test Purpose and Environment	3321

A.9.1.10.2	Test parameters	3321

A.9.1.10.3	Test Requirements	3324

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3324

A.9.1.11.1	Test Purpose and Environment	3324

A.9.1.11.2	Test parameters	3324

A.9.1.11.3	Test Requirements	3328

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3328

A.9.1.12.1	Test Purpose and Environment	3328

A.9.1.12.2	Test parameters	3328

A.9.1.12.3	Test Requirements	3329

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3329

A.9.1.13.1	Test Purpose and Environment	3329

A.9.1.13.2	Test parameters	3329

A.9.1.13.3	Test Requirements	3330

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3330

A.9.1.14.1	Test Purpose and Environment	3330

A.9.1.14.2	Test parameters	3330

A.9.1.14.3	Test Requirements	3334

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3334

A.9.1.15.1	Test Purpose and Environment	3334

A.9.1.15.2	Test parameters	3334

A.9.1.15.3	Test Requirements	3336

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3337

A.9.1.16.1	Test Purpose and Environment	3337

A.9.1.16.2	Test parameters	3337

A.9.1.16.3	Test Requirements	3338

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3339

A.9.1.17.1	Test Purpose and Environment	3339

A.9.1.17.2	Test parameters	3339

A.9.1.17.3	Test Requirements	3340

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3341

A.9.1.18.1	Test Purpose and Environment	3341

A.9.1.18.2	Test parameters	3341

A.9.1.18.3	Test Requirements	3342

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3343

A.9.1.19.1	Test Purpose and Environment	3343

A.9.1.19.2	Test parameters	3343

A.9.1.19.3	Test Requirements	3343

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3343

A.9.1.20.1	Test Purpose and Environment	3343

A.9.1.20.2	Test parameters	3343

A.9.1.20.3	Test Requirements	3345

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3345

A.9.1.21.1	Test Purpose and Environment	3345

A.9.1.21.2	Test parameters	3345

A.9.1.21.3	Test Requirements	3345

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3345

A.9.1.22.1	Test Purpose and Environment	3345

A.9.1.22.2	Test parameters	3346

A.9.1.22.3	Test Requirements	3350

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3350

A.9.1.23.1	Test Purpose and Environment	3350

A.9.1.23.2	Test parameters	3350

A.9.1.23.3	Test Requirements	3354

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3354

A.9.1.24.1	Test Purpose and Environment	3354

A.9.1.24.2	Test parameters	3354

A.9.1.24.3	Test Requirements	3355

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3355

A.9.1.25.1	Test Purpose and Environment	3355

A.9.1.25.2	Test parameters	3355

A.9.1.25.3	Test Requirements	3358

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3358

A.9.1.26.1	Test Purpose and Environment	3358

A.9.1.26.2	Test parameters	3358

A.9.1.26.3	Test Requirements	3361

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3361

A.9.1.27.1	Test Purpose and Environment	3361

A.9.1.27.2	Test parameters	3361

A.9.1.27.3	Test Requirements	3364

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3364

A.9.1.28.1	Test Purpose and Environment	3364

A.9.1.28.2	Test parameters	3364

A.9.1.28.3	Test Requirements	3367

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3367

A.9.1.29.1	Test Purpose and Environment	3367

A.9.1.29.2	Test parameters	3367

A.9.1.29.3	Test Requirements	3370

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3370

A.9.1.30.1	Test Purpose and Environment	3370

A.9.1.30.2	Test parameters	3371

A.9.1.30.3	Test Requirements	3374

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3374

A.9.1.31.1	Test Purpose and Environment	3374

A.9.1.31.2	Test parameters	3374

A.9.1.31.3	Test Requirements	3377

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3378

A.9.1.32.1	Test Purpose and Environment	3378

A.9.1.32.2	Test parameters	3378

A.9.1.32.3	Test Requirements	3381

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3381

A.9.1.33.1	Test Purpose and Environment	3381

A.9.1.33.2	Test parameters	3381

A.9.1.33.3	Test Requirements	3384

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3384

A.9.1.34.1	Test Purpose and Environment	3384

A.9.1.34.2	Test parameters	3384

A.9.1.34.3	Test Requirements	3387

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3387

A.9.1.35.1	Test Purpose and Environment	3387

A.9.1.35.2	Test parameters	3387

A.9.1.35.3	Test Requirements	3390

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3391

A.9.1.36.1	Test Purpose and Environment	3391

A.9.1.36.2	Test parameters	3391

A.9.1.36.3	Test Requirements	3394

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3394

A.9.1.37.1	Test Purpose and Environment	3394

A.9.1.37.2	Test parameters	3394

A.9.1.37.3	Test Requirements	3397

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3398

A.9.1.38.1	Test Purpose and Environment	3398

A.9.1.38.2	Test parameters	3398

A.9.1.38.3	Test Requirements	3401

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3402

A.9.1.39.1	Test Purpose and Environment	3402

A.9.1.39.2	Test parameters	3402

A.9.1.39.3	Test Requirements	3408

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3408

A.9.1.40.1	Test Purpose and Environment	3408

A.9.1.40.2	Test parameters	3409

A.9.1.40.3	Test Requirements	3415

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3415

A.9.1.41.1	Test Purpose and Environment	3415

A.9.1.41.2	Test parameters	3415

A.9.1.41.3	Test Requirements	3418

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3418

A.9.1.42.1	Test Purpose and Environment	3418

A.9.1.42.2	Test parameters	3418

A.9.1.42.3	Test Requirements	3421

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3421

A.9.1.43.1	Test Purpose and Environment	3421

A.9.1.43.2	Test parameters	3421

A.9.1.43.3	Test Requirements	3422

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3423

A.9.1.44.1	Test Purpose and Environment	3423

A.9.1.44.2	Test parameters	3423

A.9.1.44.3	Test Requirements	3428

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3428

A.9.1.45.1	Test Purpose and Environment	3428

A.9.1.45.2	Test parameters	3428

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3636

A.9.2.5.1	Test Purpose and Environment	3636

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3640

A.9.2.6.3	Test Requirements	3643

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3643

A.9.2.7.1	Test Purpose and Environment	3643

A.9.2.7.2	Test parameters	3643

A.9.2.7.3	Test Requirements	3647

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3647

A.9.2.8.1	Test Purpose and Environment	3647

A.9.2.8.2	Test parameters	3647

A.9.2.8.3	Test Requirements	3651

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3651

A.9.2.9.1	Test Purpose and Environment	3651

A.9.2.9.2	Test parameters	3651

A.9.2.9.3	Test Requirements	3655

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3655

A.9.2.10.1	Test Purpose and Environment	3655

A.9.2.10.2	Test parameters	3655

A.9.2.10.3	Test Requirements	3659

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.11.1	Test Purpose and Environment	3659

A.9.2.11.2	Test parameters	3659

A.9.2.11.3	Test Requirements	3660

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3660

A.9.2.12.1	Test Purpose and Environment	3660

A.9.2.12.2	Test parameters	3661

A.9.2.12.3	Test Requirements	3661

A.9.2.13	Void	3662

A.9.2.13.1	Void	3662

A.9.2.13.2	Void	3662

A.9.2.13.3	Void	3662

A.9.2.14	Void	3662

A.9.2.14.1	Void	3662

A.9.2.14.2	Void	3662

A.9.2.14.3	Void	3662

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3662

A.9.2.15.1	Test Purpose and Environment	3662

A.9.2.15.2	Test parameters	3662

A.9.2.15.3	Test Requirements	3666

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3666

A.9.2.16.1	Test Purpose and Environment	3666

A.9.2.16.2	Test parameters	3666

A.9.2.16.3	Test Requirements	3671

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3671

A.9.2.17.1	Test Purpose and Environment	3671

A.9.2.17.2	Test parameters	3671

A.9.2.17.3	Test Requirements	3673

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3673

A.9.2.18.1	Test Purpose and Environment	3673

A.9.2.18.2	Test parameters	3673

A.9.2.18.3	Test Requirements	3676

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3676

A.9.2.19.1	Test Purpose and Environment	3676

A.9.2.19.2	Test parameters	3676

A.9.2.19.3	Test Requirements	3678

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3678

A.9.2.20.1	Test Purpose and Environment	3678

A.9.2.20.2	Test parameters	3678

A.9.2.20.3	Test Requirements	3681

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3681

A.9.2.21.1	Test Purpose and Environment	3681

A.9.2.21.2	Test parameters	3681

A.9.2.21.3	Test Requirements	3684

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3684

A.9.2.22.1	Test Purpose and Environment	3685

A.9.2.22.2	Test parameters	3685

A.9.2.22.3	Test Requirements	3685

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3685

A.9.2.23.1	Test Purpose and Environment	3685

A.9.2.23.2	Test parameters	3685

A.9.2.23.3	Test Requirements	3688

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3688

A.9.2.24.1	Test Purpose and Environment	3688

A.9.2.24.2	Test parameters	3688

A.9.2.24.3	Test Requirements	3689

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3689

A.9.2.25.1	Test Purpose and Environment	3689

A.9.2.25.2	Test parameters	3689

A.9.2.25.3	Test Requirements	3694

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3694

A.9.2.26.1	Test Purpose and Environment	3694

A.9.2.26.2	Test parameters	3694

A.9.2.26.3	Test Requirements	3698

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3698

A.9.2.27.1	Test Purpose and Environment	3698

A.9.2.27.2	Test parameters	3698

A.9.2.27.3	Test Requirements	3699

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3699

A.9.2.28.1	Test Purpose and Environment	3699

A.9.2.28.2	Test parameters	3699

A.9.2.28.3	Test Requirements	3702

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3702

A.9.2.29.1	Test Purpose and Environment	3702

A.9.2.29.2	Test parameters	3702

A.9.2.29.3	Test Requirements	3705

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3705

A.9.2.30.1	Test Purpose and Environment	3705

A.9.2.30.2	Test parameters	3705

A.9.2.30.3	Test Requirements	3708

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3708

A.9.2.31.1	Test Purpose and Environment	3708

A.9.2.31.2	Test parameters	3708

A.9.2.31.3	Test Requirements	3711

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3711

A.9.2.32.1	Test Purpose and Environment	3711

A.9.2.32.2	Test parameters	3711

A.9.2.32.3	Test Requirements	3714

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3714

A.9.2.33.1	Test Purpose and Environment	3714

A.9.2.33.2	Test parameters	3714

A.9.2.33.3	Test Requirements	3718

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3718

A.9.2.34.1	Test Purpose and Environment	3718

A.9.2.34.2	Test parameters	3718

A.9.2.34.3	Test Requirements	3721

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3721

A.9.2.35.1	Test Purpose and Environment	3721

A.9.2.35.2	Test parameters	3721

A.9.2.35.3	Test Requirements	3724

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3724

A.9.2.36.1	Test Purpose and Environment	3724

A.9.2.36.2	Test parameters	3724

A.9.2.36.3	Test Requirements	3725

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3726

A.9.2.37.1	Test Purpose and Environment	3726

A.9.2.37.2	Test parameters	3726

A.9.2.37.3	Test Requirements	3727

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3728

A.9.2.38.1	Test Purpose and Environment	3728

A.9.2.38.2	Test parameters	3728

A.9.2.38.3	Test Requirements	3732

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3732

A.9.2.39.1	Test Purpose and Environment	3732

A.9.2.39.2	Test parameters	3732

A.9.2.39.3	Test Requirements	3736

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3736

A.9.2.40.1	Test Purpose and Environment	3736

A.9.2.40.2	Test parameters	3736

A.9.2.40.3	Test Requirements	3739

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3740

A.9.2.41.1	Test Purpose and Environment	3740

A.9.2.41.2	Test parameters	3740

A.9.2.41.3	Test Requirements	3743

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3743

A.9.2.42.1	Test Purpose and Environment	3743

A.9.2.42.2	Test parameters	3743

A.9.2.42.3	Test Requirements	3746

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3746

A.9.2.43.1	Test Purpose and Environment	3746

A.9.2.43.2	Test parameters	3746

A.9.2.43.3	Test Requirements	3749

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3749

A.9.2.44.1	Test Purpose and Environment	3749

A.9.2.44.2	Test parameters	3749

A.9.2.44.3	Test Requirements	3751

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3751

A.9.2.45.1	Test Purpose and Environment	3751

A.9.2.45.2	Test parameters	3751

A.9.2.45.3	Test Requirements	3755

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3756

A.9.2.46.1	Test Purpose and Environment	3756

A.9.2.46.2	Test parameters	3756

A.9.2.46.3	Test Requirements	3760

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3761

A.9.2.47.1	Test Purpose and Environment	3761

A.9.2.47.2	Test parameters	3761

A.9.2.47.3	Test Requirements	3765

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3765

A.9.2.48.1	Test Purpose and Environment	3765

A.9.2.48.2	Test parameters	3766

A.9.2.48.3	Test Requirements	3770

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3771

A.9.2.49.1	Test Purpose and Environment	3771

A.9.2.49.2	Test parameters	3771

A.9.2.49.3	Test Requirements	3777

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3778

A.9.2.50.1	Test Purpose and Environment	3778

A.9.2.50.2	Test parameters	3778

A.9.2.50.3	Test Requirements	3784

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3784

A.9.2.51.1	Test Purpose and Environment	3784

A.9.2.51.2	Test parameters	3785

A.9.2.51.3	Test Requirements	3789

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3789

A.9.2.52.1	Test Purpose and Environment	3789

A.9.2.52.2	Test parameters	3789

A.9.2.52.3	Test Requirements	3792

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3793

A.9.2.53.1	Test Purpose and Environment	3793

A.9.2.53.2	Test parameters	3793

A.9.2.53.3	Test Requirements	3799

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3800

A.9.2.54.1	Test Purpose and Environment	3800

A.9.2.54.2	Test parameters	3800

A.9.2.54.3	Test Requirements	3806

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3806

A.9.2.55.1	Test Purpose and Environment	3806

A.9.2.55.2	Test parameters	3807

A.9.2.55.3	Test Requirements	3812

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3812

A.9.2.56.1	Test Purpose and Environment	3812

A.9.2.56.2	Test parameters	3812

A.9.2.56.3	Test Requirements	3817

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3817

A.9.2.57.1	Test Purpose and Environment	3817

A.9.2.57.2	Test parameters	3818

A.9.2.57.3	Test Requirements	3822

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3823

A.9.2.58.1	Test Purpose and Environment	3823

A.9.2.58.2	Test parameters	3823

A.9.2.58.3	Test Requirements	3827

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3828

A.9.2.59.1	Test Purpose and Environment	3828

A.9.2.59.2	Test parameters	3828

A.9.2.59.3	Test Requirements	3833

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3834

A.9.2.60.1	Test Purpose and Environment	3834

A.9.2.60.2	Test parameters	3834

A.9.2.60.3	Test Requirements	3837

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3837

A.9.2.61.1	Test Purpose and Environment	3837

A.9.2.61.2	Test parameters	3837

A.9.2.61.3	Test Requirements	3840

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3840

A.9.2.62.1	Test Purpose and Environment	3840

A.9.2.62.2	Test parameters	3840

A.9.2.62.3	Test Requirements	3843

A.9.3	UTRAN FDD CPICH RSCP	3843

A.9.3.1	E-UTRAN FDD	3843

A.9.3.1.1	Test Purpose and Environment	3843

A.9.3.1.2	Parameters	3843

A.9.3.1.3	Test Requirements	3846

A.9.3.2 E-UTRAN TDD	3846

A.9.3.2.1	Test Purpose and Environment	3846

A.9.3.2.2	Parameters	3846

A.9.3.2.3	Test Requirements	3849

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3849

A.9.3.3.1	Test Purpose and Environment	3849

A.9.3.3.2	Parameters	3849

A.9.3.3.3	Test Requirements	3850

A.9.4	UTRAN FDD CPICH Ec/No	3851

A.9.4.1	E-UTRAN FDD	3851

A.9.4.1.1	Test Purpose and Environment	3851

A.9.4.1.2	Parameters	3851

A.9.4.1.3	Test Requirements	3853

A.9.4.2	E-UTRAN TDD	3854

A.9.4.2.1	Test Purpose and Environment	3854

A.9.4.2.2	Parameters	3854

A.9.4.2.3	Test Requirements	3856

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3857

A.9.4.3.1	Test Purpose and Environment	3857

A.9.4.3.2	Parameters	3857

A.9.4.3.3	Test Requirements	3858

A.9.5	UTRAN TDD measurement	3858

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3858

A.9.5.1.1	Test Purpose and Environment	3858

A.9.5.1.2	Test parameters	3858

A.9.5.1.3	Test Requirements	3860

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3861

A.9.5.2.1	Test Purpose and Environment	3861

A.9.5.2.2	Test parameters	3861

A.9.5.2.3	Test Requirements	3862

A.9.6	GSM Carrier RSSI	3863

A.9.6.1	E-UTRAN FDD	3863

A.9.6.1.1	Test Purpose and Environment	3863

A.9.6.1.2	Test Requirements	3864

A.9.6.2	E-UTRAN TDD	3864

A.9.6.2.1	Test Purpose and Environment	3864

A.9.6.2.2	Test Requirements	3866

A.9.7	UE Rx – Tx Time Difference	3867

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3867

A.9.7.1.1	Test Purpose and Environment	3867

A.9.7.1.2	Test parameters	3867

A.9.7.1.3	Test Requirements	3869

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3869

A.9.7.2.1	Test Purpose and Environment	3869

A.9.7.2.2	Test parameters	3869

A.9.7.2.3	Test Requirements	3871

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3871

A.9.7.3.1	Test Purpose and Environment	3871

A.9.7.3.2	Test parameters	3871

A.9.7.3.3	Test Requirements	3874

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3874

A.9.7.4.1	Test Purpose and Environment	3874

A.9.7.4.2	Test Parameters	3874

A.9.7.4.3	Test Requirements	3877

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3877

A.9.7.5.1	Test Purpose and Environment	3877

A.9.7.5.2	Test parameters	3877

A.9.7.5.3	Test Requirements	3880

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3880

A.9.7.6.1	Test Purpose and Environment	3880

A.9.7.6.2	Test Parameters	3880

A.9.7.6.3	Test Requirements	3883

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3883

A.9.7.7.1	Test Purpose and Environment	3883

A.9.7.7.2	Test parameters	3883

A.9.7.7.3	Test Requirements	3884

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3885

A.9.7.8.1	Test Purpose and Environment	3885

A.9.7.8.2	Test parameters	3885

A.9.7.8.3	Test Requirements	3886

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3886

A.9.7.9.1	Test Purpose and Environment	3886

A.9.7.9.2	Test parameters	3886

A.9.7.9.3	Test Requirements	3887

A.9.8	RSTD	3888

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3888

A.9.8.1.1	Test Purpose and Environment	3888

A.9.8.1.2	Test Requirements	3890

A.9.8.1.2A	Test Requirements for UE Category 1bis	3890

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3891

A.9.8.2.1	Test Purpose and Environment	3891

A.9.8.2.2	Test Requirements	3894

A.9.8.2.2A	Test Requirements for UE Category 1bis	3894

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3895

A.9.8.3.1	Test Purpose and Environment	3895

A.9.8.3.2	Test Requirements	3897

A.9.8.3.2A	Test Requirements for UE Category 1bis	3897

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3898

A.9.8.4.1	Test Purpose and Environment	3898

A.9.8.4.2	Test Requirements	3900

A.9.8.4.2A	Test Requirements for UE Category 1bis	3900

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3901

A.9.8.5.1	Test Purpose and Environment	3901

A.9.8.5.2	Test Requirements	3903

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3903

A.9.8.6.1	Test Purpose and Environment	3903

A.9.8.6.2	Test Requirements	3906

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3906

A.9.8.7.1	Test Purpose and Environment	3906

A.9.8.7.2	Test Requirements	3907

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3907

A.9.8.8.1	Test Purpose and Environment	3907

A.9.8.8.2	Test Requirements	3908

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3908

A.9.8.9.1	Test Purpose and Environment	3908

A.9.8.9.2	Test Requirements	3909

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3909

A.9.8.10.1	Test Purpose and Environment	3909

A.9.8.10.2	Test Requirements	3910

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3910

A.9.8.11.1	Test Purpose and Environment	3910

A.9.8.11.2	Test Requirements	3911

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3911

A.9.8.12.1	Test Purpose and Environment	3911

A.9.8.12.2	Test Requirements	3912

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3912

A.9.8.13.1	Test Purpose and Environment	3912

A.9.8.13.2	Test Requirements	3913

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3913

A.9.8.14.1	Test Purpose and Environment	3913

A.9.8.14.2	Test Requirements	3920

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3920

A.9.8.15.1	Test Purpose and Environment	3920

A.9.8.15.2	Test Requirements	3926

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3926

A.9.8.16.1	Test Purpose and Environment	3926

A.9.8.16.2	Test Requirements	3929

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3929

A.9.8.17.1	Test Purpose and Environment	3929

A.9.8.17.2	Test Requirements	3932

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3932

A.9.8.18.1	Test Purpose and Environment	3932

A.9.8.18.2	Test Requirements	3935

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3935

A.9.8.19.1	Test Purpose and Environment	3935

A.9.8.19.2	Test Requirements	3938

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3938

A.9.8.20.1	Test Purpose and Environment	3938

A.9.8.20.2	Test Requirements	3941

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3942

A.9.8.21.1	Test Purpose and Environment	3942

A.9.8.21.2	Test Requirements	3945

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3946

A.9.8.22.1	Test Purpose and Environment	3946

A.9.8.22.2	Test Requirements	3949

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3950

A.9.8.23.1	Test Purpose and Environment	3950

A.9.8.23.2	Test Requirements	3953

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3953

A.9.8.24.1	Test Purpose and Environment	3953

A.9.8.24.2	Test Requirements	3957

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3958

A.9.8.25.1	Test Purpose and Environment	3958

A.9.8.25.2	Test Requirements	3961

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3962

A.9.8.26.1	Test Purpose and Environment	3962

A.9.8.26.2	Test Requirements	3964

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3964

A.9.8.27.1	Test Purpose and Environment	3964

A.9.8.27.2	Test Requirements	3967

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3967

A.9.8.28.1	Test Purpose and Environment	3967

A.9.8.28.2	Test Requirements	3970

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3970

A.9.8.29.1	Test Purpose and Environment	3970

A.9.8.29.2	Test Requirements	3974

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3975

A.9.8.30.1	Test Purpose and Environment	3975

A.9.8.30.2	Test Requirements	3978

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3979

A.9.8.31.1	Test Purpose and Environment	3979

A.9.8.31.2	Test Requirements	3983

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3983

A.9.8.32.1	Test Purpose and Environment	3983

A.9.8.32.2	Test Requirements	3986

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3986

A.9.8.33.1	Test Purpose and Environment	3986

A.9.8.33.2	Test Requirements	3989

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3989

A.9.8.34.1	Test Purpose and Environment	3989

A.9.8.34.2	Test Requirements	3992

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3992

A.9.8.35.1	Test Purpose and Environment	3992

A.9.8.35.2	Test Requirements	3995

A.9.9	RSRP and RSRQ on the serving cell	3995

A.9.9.1	FDD Intra frequency serving cell case	3995

A.9.9.1.1	Test Purpose and Environment	3995

A.9.9.1.2	Test parameters	3995

A.9.9.1.3	Test Requirements	3998

A.9.9.2	TDD Intra frequency serving cell case	3998

A.9.9.2.1	Test Purpose and Environment	3998

A.9.9.2.2	Test parameters	3998

A.9.9.2.3	Test Requirements	4000

A.9.10	SSTD	4000

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	4000

A.9.10.1.1	Test Purpose and Environment	4000

A.9.10.1.2	Test parameters	4000

A.9.10.1.3	Test Requirements	4002

A.9.10.2	Void	4002

A.9.10.3	Void	4002

A.9.10.4	Void	4002

A.9.11 RSSI	4002

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4002

A.9.11.1.1	Test Purpose and Environment	4002

A.9.11.1.2	Test parameters	4002

A.9.11.1.3	Test Requirements	4005

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4005

A.9.11.2.1	Test Purpose and Environment	4005

A.9.11.2.2	Test parameters	4005

A.9.12	Channel occupancy	4008

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4008

A.9.12.1.1	Test Purpose and Environment	4008

A.9.12.1.2	Test parameters	4008

A.9.12.1.3	Test Requirements	4011

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4011

A.9.12.2.1	Test Purpose and Environment	4011

A.9.12.2.2	Test parameters	4011

A.9.12.2.3	Test Requirements	4014

A.9.13	RS-SINR	4014

A.9.13.1	FDD Intra-Frequency Case	4014

A.9.13.1.1	Test Purpose and Environment	4014

A.9.13.1.2	Test parameters	4014

A.9.13.1.3	Test Requirements	4017

A.9.13.2	TDD Intra-Frequency Case	4017

A.9.13.2.1	Test Purpose and Environment	4017

A.9.13.2.2	Test parameters	4017

A.9.13.2.3	Test Requirements	4020

A.9.13.3	FDD—FDD Inter frequency case	4020

A.9.13.3.1	Test Purpose and Environment	4020

A.9.13.3.2	Test parameters	4020

A.9.13.3.3	Test Requirements	4023

A.9.13.4	TDD—TDD Inter frequency case	4023

A.9.13.4.1	Test Purpose and Environment	4023

A.9.13.4.2	Test parameters	4023

A.9.13.4.3	Test Requirements	4027

A.9.13.5	FDD—TDD Inter frequency case	4028

A.9.13.5.1	Test Purpose and Environment	4028

A.9.13.5.2	Test parameters	4028

A.9.13.5.3	Test Requirements	4034

A.9.13.6	TDD—FDD Inter frequency case	4034

A.9.13.6.1	Test Purpose and Environment	4034

A.9.13.6.2	Test parameters	4034

A.9.13.6.3	Test Requirements	4038

A.9.14	Channel quality reporting accuracy	4038

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4038

A.9.14.1.1	Test Purpose and Environment	4038

A.9.14.1.2	Test parameters	4039

A.9.14.1.3	Test Requirements	4039

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4040

A.9.14.2.1	Test Purpose and Environment	4040

A.9.14.2.2	Test parameters	4040

A.9.14.2.3	Test Requirements	4041

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4041

A.9.14.3.1	Test Purpose and Environment	4041

A.9.14.3.2	Test parameters	4041

A.9.14.3.3	Test Requirements	4042

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4042

A.9.14.4.1	Test Purpose and Environment	4042

A.9.14.4.2	Test parameters	4042

A.9.14.4.3	Test Requirements	4043

A.10	Proximity-based Services in Any Cell Selection State	4066

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4066

A.10.1.1	Test Purpose and Environment	4066

A.10.1.2	Test Requirements	4067

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4068

A.10.2.1	Test Purpose and Environment	4068

A.10.2.2	Test Requirements	4069

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4069

A.10.3.1	Test Purpose and Environment	4069

A.10.3.2	Test Requirements	4071

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4072

A.10.4.1	Test Purpose and Environment	4072

A.10.4.2	Test Requirements	4074

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4074

A.11.1	V2V UE Transmission Timing Accuracy Test	4074

A.11.1.1	Test Purpose and Environment	4074

A.11.1.2	Test requirements	4075

A.11.2	Interruptions due to V2V sidelink communication	4075

A.11.2.1	Test Purpose and Environment	4075

A.11.2.2	Test Requirements	4077

A.12	4077

A.12.1	V2X UE Transmission Timing Accuracy Test	4077

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4077

A.12.1.1.1	Test Purpose and Environment	4077

A.12.1.1.2	Test requirements	4078

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4079

A.12.1.2.1	Test Purpose and Environment	4079

A.12.1.2.2	Test Requirements	4079

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4080

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4080

A.12.2.1.1	Test Purpose and Environment	4080

A.12.2.1.2	Test Requirements	4081

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4082

A.12.2.2.1	Test Purpose and Environment	4082

A.12.2.2.2	Test Requirements	4083

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4083

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4083

A.12.3.1.1	Test Purpose and Environment	4083

A.12.3.1.2	Test Requirements	4086

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4087

A.12.3.2.1	Test Purpose and Environment	4087

A.12.3.1.2	Test Requirements	4089

A.12.4	Congestion Control Measurement Test for V2X UE	4089

A.12.4.1	Test Purpose and Environment	4089

A.12.4.2	Test Requirements	4091

A.12.5	Interruptions due to V2X Sidelink Communication	4091

A.12.5.1	Test Purpose and Environment	4091

A.12.5.2	Test Requirements	4093

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4093

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4093

A.12.6.1.1	Test Purpose and Environment	4093

A.12.6.1.2	Test Requirements	4095

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4095

A.12.6.2.1	Test Purpose and Environment	4095

A.12.6.1.2	Test Requirements	4097

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4097

A.12.7.1	Test Purpose and Environment	4097

A.12.7.2	Test Requirements	4100

A.12.8	Interruptions due to V2X Carrier Aggregation	4101

A.12.8.1	Interruptions on a FDD PCell	4101

A.12.8.1.1	Test Purpose and Environment	4101

A.12.8.1.2	Test Requirements	4102

A.12.8.2	Interruptions on a TDD PCell	4102

A.12.8.2.1	Test Purpose and Environment	4102

A.12.8.2.2	Test Requirements	4104

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4105

A.13.1	RRC_IDLE state for satellite access	4105

A.13.1.1	Cell re-selection for satellite access	4105

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4105

A.13.1.1.1.1	Test Purpose and Environment	4105

A.13.1.1.1.2	Test Requirements	4107

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4108

A.13.1.1.2.1	Test Purpose and Environment	4108

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4111

A.13.1.1.3.1	Test Purpose and Environment	4111

A.13.1.1.3.2	Test Requirements	4112

A.13.2	Void	4113

A.13.3	RRC connection mobility control for satellite access	4113

A.13.3.1	RRC re-establishment for satellite access	4113

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4113

A.13.3.1.1.1	Test Purpose and Environment	4113

A.13.3.1.1.2	Test Requirements	4114

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4115

A.13.3.1.2.1	Test Purpose and Environment	4115

A.13.3.1.2.2	Test Requirements	4116

A.13.3.2	Random Access for Satellite Access	4117

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4117

A.13.3.2.1.1	Test Purpose and Environment	4117

A.13.3.2.1.2	Test Requirements	4119

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4121

A.13.3.2.2.1	Test Purpose and Environment	4121

A.13.3.2.2.2	Test Requirements	4122

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4124

A.13.3.2.3.1	Test Purpose and Environment	4124

A.13.3.2.3.2	Test Requirements	4125

A.13.4	Timing and signalling characteristics for satellite access	4127

A.13.4.1	UE transmit timing for satellite access	4127

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4127

A.13.4.1.1.1	Test Purpose and Environment	4127

A.13.4.1.1.2	Test Requirements	4128

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4129

A.13.4.1.2.1	Test Purpose and Environment	4129

A.13.4.1.2.2	Test Requirements	4130

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4131

A.13.4.1.3.1	Test Purpose and Environment	4131

A.13.4.1.3.2	Test Requirements	4132

A.13.4.2	UE timing advance for satellite access	4133

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4133

A.13.4.2.1.1	Test Purpose and Environment	4133

A.13.4.2.1.2	Test Requirements	4135

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4135

A.13.4.2.2.1	Test Purpose and Environment	4135

A.13.4.2.2.2	Test Requirements	4137

A.13.4.3	Radio Link Monitoring for satellite access	4137

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4137

A.13.4.3.1.1	Test Purpose and Environment	4137

A.13.4.3.1.2	Test Requirements	4140

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4140

A.13.4.3.2.1	Test Purpose and Environment	4140

A.13.4.3.2.2	Test Requirements	4143

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4143

A.13.4.3.3.1	Test Purpose and Environment	4143

A.13.4.3.3.2	Test Requirements	4146

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4146

A.13.4.3.4.1	Test Purpose and Environment	4146

A.13.4.3.4.2	Test Requirements	4149

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4149

A.13.4.3.5.1	Test Purpose and Environment	4149

A.13.4.3.5.2	Test Requirements	4152

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4152

A.13.4.3.6.1	Test Purpose and Environment	4152

A.13.4.3.6.2	Test Requirements	4155

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4155

A.13.4.3.7.1	Test Purpose and Environment	4155

A.13.4.3.7.2	Test Requirements	4157

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4158

A.13.4.3.8.1	Test Purpose and Environment	4158

A.13.4.3.8.2	Test Requirements	4160

A.13.5	Void	4161

A.13.6	Measurement performance requirements for UE for satellite access	4161

A.13.6.1	Void	4161

A.13.6.2	Channel quality reporting accuracy for satellite access	4161

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4161

A.13.6.2.1.1	Test Purpose and Environment	4161

A.13.6.2.1.2	Test parameters	4161

A.13.6.2.1.3	Test Requirements	4162

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4162

A.13.6.2.2.1	Test Purpose and Environment	4162

A.13.6.2.2.2	Test parameters	4162

A.13.6.2.2.3	Test Requirements	4163

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4163

A.13.6.2.3.1	Test Purpose and Environment	4163

A.13.6.2.3.2	Test parameters	4164

A.13.6.2.3.3	Test Requirements	4164

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4165

A.13.6.2.4.1	Test Purpose and Environment	4165

A.13.6.2.4.2	Test parameters	4165

A.13.6.2.4.3	Test Requirements	4166

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4166

A.13.6.2.5.1	Test Purpose and Environment	4166

A.13.6.2.5.2	Test parameters	4166

A.13.6.2.5.3	Test Requirements	4167

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4168

A.13.6.2.6.1	Test Purpose and Environment	4168

A.13.6.2.6.2	Test parameters	4168

A.13.6.2.6.3	Test Requirements	4169

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4169

A.14.1	RRC_IDLE state for satellite access	4169

A.14.1.1	Cell re-selection for satellite access	4169

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4169

A.14.1.1.1.1	Test Purpose and Environment	4169

A.14.1.1.1.2	Test Requirements	4171

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4172

A.14.1.1.2.1	Test Purpose and Environment	4172

A.14.1.1.2.2	Test Requirements	4174

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4175

A.14.1.1.3.1	Test Purpose and Environment	4175

A.14.1.1.3.2	Test Requirements	4177

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4178

A.14.1.1.4.1	Test Purpose and Environment	4178

A.14.1.1.4.2	Test Requirements	4180

A.14.2	RRC_CONNECTED state mobility for satellite access	4181

A.14.2.1	E-UTRAN handover for satellite access	4181

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4181

A.14.2.1.1.1	Test Purpose and Environment	4181

A.14.2.1.1.2	Test Requirements	4183

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4184

A.14.2.1.2.1	Test Purpose and Environment	4184

A.14.2.1.2.2	Test Requirements	4185

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4186

A.14.2.1.3.1	Test Purpose and Environment	4186

A.14.2.1.3.2	Test Requirements	4187

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4188

A.14.2.1.4.1	Test Purpose and Environment	4188

A.14.2.1.4.2	Test Requirements	4189

A.14.3	RRC connection mobility control for satellite access	4190

A.14.3.1	RRC re-establishment for satellite access	4190

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4190

A.14.3.1.1.1	Test Purpose and Environment	4190

A.14.3.1.1.2	Test Requirements	4191

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4192

A.14.3.1.2.1	Test Purpose and Environment	4192

A.14.3.1.2.2	Test Requirements	4194

A.14.3.2	Random access for satellite access	4195

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4195

A.14.3.2.1.1	Test Purpose and Environment	4195

A.14.3.2.1.2	Test Requirements	4197

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4199

A.14.3.2.2.1	Test Purpose and Environment	4199

A.14.3.2.2.2	Test Requirements	4201

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4203

A.14.3.2.3.1	Test Purpose and Environment	4203

A.14.3.2.3.2	Test Requirements	4205

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4206

A.14.3.2.4.1	Test Purpose and Environment	4206

A.14.3.2.4.2	Test Requirements	4208

A.14.4	Timing and signalling characteristics for satellite access	4210

A.14.4.1	UE transmit timing for satellite access	4210

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4210

A.14.4.1.1.1	Test Purpose and Environment	4210

A.14.4.1.1.2	Test Requirements	4212

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4213

A.14.4.1.2.1	Test Purpose and Environment	4213

A.14.4.1.2.2	Test Requirements	4214

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4215

A.14.4.1.3.1	Test Purpose and Environment	4215

A.14.4.1.3.2	Test Requirements	4216

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4217

A.14.4.1.4.1	Test Purpose and Environment	4217

A.14.4.1.4.2	Test Requirements	4218

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4219

A.14.4.1.5.1	Test Purpose and Environment	4219

A.14.4.1.5.2	Test Requirements	4220

A.14.4.2	UE timing advance for satellite access	4221

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4221

A.14.4.2.1.1	Test Purpose and Environment	4221

A.14.4.2.1.2	Test Requirements	4223

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4223

A.14.4.2.2.1	Test Purpose and Environment	4223

A.14.4.2.2.2	Test Requirements	4226

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4226

A.14.4.2.3.1	Test Purpose and Environment	4226

A.14.4.2.3.2	Test Requirements	4228

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4228

A.14.4.2.4.1	Test Purpose and Environment	4228

A.14.4.2.4.2	Test Requirements	4229

A.14.4.3	Radio Link Monitoring for satellite access	4230

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4230

A.14.4.3.1.1	Test Purpose and Environment	4230

A.14.4.3.1.2	Test Requirements	4233

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4233

A.14.4.3.2.1	Test Purpose and Environment	4233

A.14.4.3.2.2	Test Requirements	4236

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4236

A.14.4.3.3.1	Test Purpose and Environment	4236

A.14.4.3.3.2	Test Requirements	4239

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4239

A.14.4.3.4.1	Test Purpose and Environment	4239

A.14.4.3.4.2	Test Requirements	4242

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4242

A.14.4.3.5.1	Test Purpose and Environment	4242

A.14.4.3.5.2	 Test Requirements	4245

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4245

A.14.4.3.6.1	Test Purpose and Environment	4245

A.14.4.3.6.2	Test Requirements	4248

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4248

A.14.4.3.7.1	Test Purpose and Environment	4248

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4251

A.14.4.3.8.1 	Test Purpose and Environment	4251

A.14.4.3.8.2	Test Requirements	4254

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4254

A.14.5.1 Intra-frequency measurements for satellite access	4254

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	4254

A.14.5.1.1.1	Test Purpose and Environment	4254

A.14.5.1.1.2	Test Requirements	4256

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4257

A.14.5.1.2.1	Test Purpose and Environment	4257

A.14.5.1.2.2	Test Requirements	4259

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	4259

A.14.5.1.3.1	Test Purpose and Environment	4259

A.14.5.1.3.2	Test Requirements	4261

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4262

A.14.5.1.4.1	Test Purpose and Environment	4262

A.14.5.1.4.2	Test Requirements	4264

A.14.6	Measurement performance requirements for UE for satellite access	4264

A.14.6.1	RSRP for satellite access	4264

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4264

A.14.6.1.1.1	Test Purpose and Environment	4264

A.14.6.1.1.2	Test parameters	4264

A.14.6.1.1.3	Test Requirements	4267

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4267

A.14.6.1.2.1	Test Purpose and Environment	4267

A.14.6.1.2.2	Test parameters	4267

A.14.6.1.2.3	Test Requirements	4270

A.14.6.2	Channel quality reporting accuracy for satellite access	4270

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4270

A.14.6.2.1.1	Test Purpose and Environment	4270

A.14.6.2.1.2	Test parameters	4270

A.14.6.2.1.3	Test Requirements	4271

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4271

A.14.6.2.2.1	Test Purpose and Environment	4271

A.14.6.2.2.2	Test parameters	4272

A.14.6.2.2.3	Test Requirements	4272

A.14.6.2.3.2	Test parameters	4273

A.14.6.2.3.3	Test Requirements	4274

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4274

A.14.6.2.4.1	Test Purpose and Environment	4274

A.14.6.2.4.2	Test parameters	4274

A.14.6.2.4.3	Test Requirements	4275

Annex B (normative): Conditions for RRM requirements applicability for operating bands	4276

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4276

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4276

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4276

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4276

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4278

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4279

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4279

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4279

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4279

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4281

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4282

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4282

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4283

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4284

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4284

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4284

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4285

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4285

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4285

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4286

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4286

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4286

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4287

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4288

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4288

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4288

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4288

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4288

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4289

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4289

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4290

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4290

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4290

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4290

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4292

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4293

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4293

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4294

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4295

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4295

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4296

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4296

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4297

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4297

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4298

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4298

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4299

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4299

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4300

B.3	Conditions for measurements performance requirements for UE	4300

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4300

B.3.2	Void	4301

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4301

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4301

B.3.5	Conditions for UE Rx – Tx time difference	4301

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4301

B.3.7	Conditions for inter-frequency RSTD measurements	4301

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4302

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4302

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4302

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4302

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4303

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4303

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4303

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4303

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4303

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4304

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4304

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4304

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4304

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4304

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4304

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4305

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4305

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4305

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4305

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4305

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4305

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4305

B.3.21.1	Conditions for RSRP measurements	4305

B.3.21.2	Conditions for RSRQ measurements	4306

B.3.21.3	Conditions for CSI-RSRP measurements	4306

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4306

B.3.22.1	Conditions for RSRP measurements	4306

B.3.22.2	Void	4306

B.3.22.3	Conditions for CSI-RSRP measurements	4306

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4307

B.3.23.1	Conditions for RSRP measurements	4307

B.3.23.2	Conditions for RSRQ measurements	4307

B.3.23.3	Conditions for CSI-RSRP measurements	4307

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4307

B.3.24.1	Conditions for RSRP measurements	4307

B.3.24.2	Conditions for RSRQ measurements	4307

B.3.24.3	Conditions for CSI-RSRP measurements	4307

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4308

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4308

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4308

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4308

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4309

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4309

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4309

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4309

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4309

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4309

B.4	RRM Requirements Exceptions	4310

B.4.1	General	4310

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4310

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4310

B.4.3.1	Inter-band carrier aggregation	4310

B.4.3.2	Intra-band non-contiguous carrier aggregation	4310

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4310

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4311

B.5.1	Conditions for S-RSRP Accuracy Requirements	4311

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4311

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4311

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4312

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4312

B.6	Conditions for V2X	4313

B.6.1	Test parameters for GNSS signals	4313

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4313

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4313

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4313

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4314

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4314

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4314

B.8	High level test procedure for SAN RRM tests	4316

Annex C (informative): Change history:	4316