| 3GPP TR 23.700-02 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement for satellite access enabled 5G services Phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Key issues	10

4.1	Key Issue #1: AIML model storage and deployed on satellite	10

4.1.1	Description	10

4.1.2	Open Issues	10

4.2	Key Issue #2: Satellite based AIML service maintenance while losing connection with terrestrial network	11

4.2.1	Description	11

4.2.2	Open Issues	11

4.3	Key Issue #3: Support satellite selection in data delivery	11

4.3.1	Description	11

4.3.2	Open Issues	11

4.4	Key issue #4: Application enablement layer enhancement for efficient content delivery over satellite access	12

4.4.1	Description	12

4.4.2	Open Issues	12

4.5	Key issue #5: Location management service via satellite access	13

4.5.1	Description	13

4.5.2	Open issues	13

4.6	Key issue #6: Improve service performance over satellite access utilizing AI capabilities	13

4.6.1	Description	13

4.6.2	Open Issues	13

5	Solutions	14

5.0	Mapping of solutions to key issues	14

5.1	Solution #1: Support of application satellite coverage availability information (ASCAI) analysis	14

5.1.1	Solution description	14

5.1.1.1	Procedure of Satellite related information subscription request	14

5.1.2	Architecture Impacts	16

5.1.3	Corresponding APIs	16

5.1.4	Solution evaluation	16

5.2	Solution #2: Support of QoS analysis for services over satellite access	16

5.2.1	Solution description	16

5.2.1.1	Procedure of QoS analysis for services over satellite access	16

5.2.2	Architecture Impacts	18

5.2.3	Corresponding APIs	18

5.2.4	Solution evaluation	18

5.3	Solution #3: Location management service via satellite access	18

5.3.0	General description	18

5.3.1	Solution description	19

5.3.2	Architecture Impacts	20

5.3.3	Corresponding APIs	20

5.3.4	Solution evaluation	20

5.4	Solution #4: Enhance SEALDD to support satellite selection in data delivery	20

5.4.1	Architecture	20

5.4.2	Solution description	20

5.4.2.1	Procedures	21

5.4.2.1.1	SEALDD enabled transmission with satellite selection support in data delivery	21

5.4.2.2	Information flows	22

9.2.3.1	SEALDD enabled regular transmission request	23

5.4.2	Architecture Impacts	23

5.4.3	Corresponding APIs	23

5.4.4	Solution evaluation	23

5.5	Solution #5: Using SEALDD to enhance application enablement layer for efficient content delivery over satellite access	23

5.5.1	Architecture	23

5.5.2	Solution description	23

5.5.2.1	Procedures	24

5.5.2.1.1	SEALDD enabled transmission for efficient content delivery	24

5.5.2.2	Information flows	26

5.5.2.2.1	SEALDD enabled ECDforSatAccess transmission request	26

5.5.2.2.2	SEALDD enabled ECDforSatAccess transmission response	26

5.5.2.2.3	SEALDD enabled ECDforSatAccess transmission notification	26

5.5.2	Architecture Impacts	27

5.5.3	Corresponding APIs	27

5.5.4	Solution evaluation	27

5.x	Solution #x: <title>	27

5.x.1	Solution description	27

5.x.2	Architecture Impacts	27

5.x.3	Corresponding APIs	27

5.x.4	Solution evaluation	27

6	Deployment scenarios	27

6.1	General	27

7	Business Relationships	27

8	Overall evaluation	27

9	Conclusions	28

9.1	General conclusions	28

9.2	Conclusions of key issue #x	28

Annex A: Change history	29

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document is a technical report capturing the study on enhancement to application enablement for Satellite access enabled 5G Services over 3GPP networks. The aspects of the study include identifying architecture requirements, supporting architecture for satellite access enabled 3GPP services and application enablers, and corresponding solutions.

The study is based on the application enablement requirements as defined in 3GPP TS 22.261 [1]. The study is dependent on 3GPP TS 23.501 [2] (5GC architecture for satellite access for NR), 3GPP TS 23.502 [3] (procedures for NR satellite access). The study would consider the impacts and enhancements to the application enablement to align with SA1 and existing SA6 service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TR 23.700-01: "Study on application enablement for Satellite access enabled 5G Services”.

[6]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[7]	3GPP TR 23.700-82: “Study on application layer support for AI/ML services.”

[8]	3GPP TS 23.482: “Functional architecture and information flows for AIML Enablement Service”.

[9]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

ASCAI: Application Satellite Coverage Availability Information.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key Issue #1: AIML model storage and deployed on satellite

### 4.1.1	Description

In 3GPP's TS 23.482 [8], SA6 defines the architecture of AIML enabling services and related AIML business processes. Existing AIML-enabled services provide different users or third-party entities with services for searching, storing, and calling AIML models by providing a platform. Based on the SA6 process, users can call servers in edge networks or cloud networks, retrieve the required models, and request the server to perform corresponding AIML tasks, such as perception, recognition, reasoning, and federated learning. Although TS 23.700-82 [7] has proposed a variety of ground network use cases for AIML APP, it is worth studying whether the scenario of deploying AIML models on satellites requires enhancements to existing application enablers.

Furthermore, if remote sensing data is collected via satellite and transmitted to AI models on ground networks, the data capacity and speed of transmission will be limited by the satellite's limited bandwidth. Therefore, it is possible to consider deploying AIML models directly on satellites, performing data processing, compression, and inference locally. Ultimately, the compressed remote sensing data and inference results are transmitted to users on the ground via feeder links, thereby reducing data transmission burden and latency.

### 4.1.2	Open Issues

This key issue will study:

1.	Investigate different deployment options for AIMLAPP when AIML models are deployed on satellite and study whether/how to deploy AIMLE on satellite (e.g. ML repository/ADAES on satellite to perform data processing/interference based on collected remote data).

2.	Whether/how to storage AIML models for AIMLE (e.g. Light weight models stored on remote sensing satellite to process collected remote data, Model transmission and sharing between satellites and terrestrial network).

## 4.2	Key Issue #2: Satellite based AIML service maintenance while losing connection with terrestrial network

### 4.2.1	Description

The relative positions of satellites (such as low-orbit and medium-orbit satellites) other than geosynchronous Earth orbit (GEO) satellites to the AIML service platform on the terrestrial network are constantly changing. Considering the scenario where AIML models are deployed on satellites, there are specific time windows during which the AIML model on the satellite can establish data and signalling communications with the AIML service platform. Outside of these time windows, the AIML model on the satellite loses contact with the terrestrial network. If the time required for the onboard AIML model to collect remote sensing data exceeds the satellite-to-ground network communication window, AIML service continuity must be maintained during the extended period (e.g., 1. Waiting for the satellite to reestablish contact with the ground network and obtain the collected remote sensing data; 2. Switching to another AIML satellite providing remote sensing services and taking over the data and processing results from the original AIML satellite; 3. Maintaining data and signalling communication between the ground network and the AIML satellite via an AIMLE server-to-AIMLE server mechanism).

It is worthwhile to explore ways to ensure onboard AIML service continuity by leveraging inter-satellite handovers and predicting/managing the AIML satellite's available time windows, thereby maintaining user services even after the AIML satellite leaves the available time window. Furthermore, it is worthwhile to investigate interconnection solutions between AIMLE platforms in the ground network. By using a roaming AIMLE platform as a proxy for the original AIMLE platform, a data and signalling communication path could be established between the original AIML satellite, the roaming AIMLE platform, and the original AIMLE platform to ensure user service continuity.

### 4.2.2	Open Issues

This key issue will study:

1.	Whether and how to support satellite based AIML service management when losing connection with terrestrial network (e.g. When a satellite is supporting AIML service connection or related data collection, the satellite keeps connection with terrestrial network due to exceeding the available time window).

2.	Whether and how to maintain and reestablish communication between UE and AIMLE layer while collecting/transmitting remote data via satellite (e.g.  predict/manage available time window of satellites related to (based on) AIML data collection and transmission information).

## 4.3	Key Issue #3: Support satellite selection in data delivery

### 4.3.1	Description

When operators deploy some components of the SEALDD architecture on satellites to ensure reliable data transmission via satellite communications, the following scenario may occur: to reduce latency, users initially select low-orbit satellites for satellite data transmission. However, due to the shorter connection time for users, low-orbit satellites may result in incomplete data transmission before the satellite is no longer within the user's connection range. Therefore, when the low-orbit satellite-assisted enabling layer performs data transmission, it is necessary to consider how to ensure uninterrupted data transmission before the satellite leaves users’ connection range. It is worth studying the need to arrange satellite switching and application context transmission in advance before the available time window of the data transmission auxiliary satellite is exceeded.

### 4.3.2	Open Issues

This key issue will study:

1.	When a satellite is handling received message request, whether and how to select proper satellites in the same orbit for data delivery (e.g. selection between satellites to ensure continuity of data transmission to terrestrial network).

2.	When data delivery handling is moved to another satellite in the same orbit to support continuity of data delivery, whether and how to relocate the context between satellites (e.g. enhancement on application context relocation to support data delivery between satellites).

## 4.4	Key issue #4: Application enablement layer enhancement for efficient content delivery over satellite access

### 4.4.1	Description

The 3GPP stage-1 spec. TS 22.261 [2] provides the service requirements for the support of efficient content delivery. Emphasizing the video-based services and personal data storage applications, it brings up that the in-network content caching can be provided by an operator, a third-party or both, to improve user experience, reduce backhaul resource usage and utilize radio resource efficiently. The operation of in-network caching includes the location management of the content cache as well as the efficient delivery of content to and from the appropriate content caching applications. The location requirement includes the support of content caching applications in both the network (e.g., AN/access network and CN/core network) and the UE (e.g., terminal equipment, smartphone, etc.).

When the services are extended to accommodate the satellite access, the stage-1 spec. requires, quoted as:

A 5G system with satellite access optimize the delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service on very large to global coverages.

This extension suggests three aspects: (1) ubiquitous coverage over a large to global scope implies the support of distributed content caching applications; (2) the distributedness necessitates the cache deployment in AN, CN, and even in UE; and (3) efficient content delivery from application servers with the consideration of satellite access characteristics.

The 3GPP SEAL spec. TS 23.434 [4] references the data delivery enabler, i.e., SEALDD, to ease the diversified data delivery demands for VAL applications. SEALDD offers not only the advanced data delivery capabilities, but also the storage capacities. The SEALDD spec. The 3GPP TS 23.433 [3] clauses 4.6 and 9.5 indicate that the SEALDD server supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers (via the SEALDD-E reference point). The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. As we can project, while the content caching service may be leveraged via the storage service as provided by the SEALDD server, the normal deployment locations of SEALDD servers makes the distributedness of content caches less desirable, which posts challenge to the required ubiquitous coverage over a large to global scope for satellite access. In 3GPP TS 23.434 [4] upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on a UE for better distributedness.

3GPP TS 23.434 [4] clause 21, SEAL services over satellite access, implies the satellite footprint or application satellite coverage availability information (ASCAI) can be leveraged for efficient content delivery. It describes a CM (i.e., Configuration Management) server can provide the ASCAI to consumers (e.g., VAL UE, CM client, VAL server, etc.) to guide to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow if the VAL UE is accessing the satellite). The 3GPP TR 23.700-01 [5] indicates that supplying the discontinuous coverage pattern between UEs and AS/AFs to the application layer will help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed caching applications (e.g. UEs supporting caching applications may be given precedence during a connectivity available interval, UEs with large caching demand may be better off utilizing longer connectivity interval, etc.).

### 4.4.2	Open Issues

This Key issue is proposed to study the enhancement of the application enablement architecture for the support of efficient content delivery over the satellite access:

-	Whether and how to enhance the application enablement architecture layer and/or application enabler (e.g., SEALDD) to support the deployment of content caching applications for efficient content delivery.

-	Whether and how to enhance the application enablers (e.g., SEALDD, NRM, etc.) to optimize the content delivery over the satellite connectivity by leveraging the satellite access characteristics (e.g., satellite ephemeris, ASCAI or application satellite coverage availability information, QoS parameters for traffic over the satellite access, etc.).

## 4.5	Key issue #5: Location management service via satellite access

### 4.5.1	Description

3GPP TS 23.434 [4] describes Location Management service that offers the location management related capabilities to one or more vertical applications. The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server. The location management server, with the fused location function, may combine/aggregate location information from multiple sources to provide a more accurate UE location. The location management server also supports to report the value-added location information (e.g. monitoring location deviation events, history location data, periodic verify UE location, prediction related to UE location, etc.) to the VAL server.

In a network with satellite access, the Location management service works as expected when both the service link and feeder link are available, and on par with the location management service with terrestrial access. However, when either of them (service link or feeder link) is not available, the Location management service is expected to have interruption that may have direct impact on the behaviour of the application. Particularly, features such as monitoring location deviation events, history location data, periodic verify UE location does not work when such interruption happens.

### 4.5.2	Open issues

To support the location management service with satellite service, the following aspects need to be studied:

-	How to support history location data, monitoring location deviation events, periodic verify UE location when the UE is connected via satellite access, during the loss of service link or feeder link?

## 4.6	Key issue #6: Improve service performance over satellite access utilizing AI capabilities

### 4.6.1	Description

In Rel-19, the normative work on application enablement for satellite enabled 5G services has been specified which including usage of satellite access characteristics for the application enablement, edge computing on satellite, satellite access with discontinuous coverage, impact on MC service and so on. Especially there is a solution which defines a new analytics for the UE RAT connectivity and utilizing the ADAE server to provide a better service experience as specified in 3GPP TS 23.436 [6].

In fact, besides the UE RAT connectivity analysis, the application enabled layer could obtain more satellite related data analysis to optimize the performance of satellite access and satellite communication with the help of AI enablers/capabilities defined in application enablement layer. For example, compared with defined ASCAI (Application Satellite Coverage Availability Information), satellite related data analysis may provide available satellite information to the target UE in a certain area, the time/predicted time at which the target UE is moved in/out of the satellite coverage, the preferred QoS/QoE when using the services over the satellite access, etc. to indicate the UE client on the application or the application server the related satellite information directly.  Thus it’s necessary to study how to enhance the satellite based data analysis to improve the service performance in the application enabled layer.

### 4.6.2	Open Issues

Based on the above analysis, the following open issues need to be studied:

-	Whether and how to provide/expose the preferred and/or predicated satellite related information (e.g. satellite/time/area) to the consumers when using the satellite, utilizing the analysis from (e.g. ADAES, ALMLE).

-	Whether and how to provide/expose the preferred and/or predicated QoS/QoE for services over satellite access, utilizing analysis from (e.g. ADAES, ALMLE).

# 5	Solutions

## 5.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  | X |
| Sol #2 |  |  |  |  |  | X |
| Sol #3 |  |  |  |  | X |  |
| Sol #4 |  |  | X |  |  |  |
| Sol #5 |  |  |  | X |  |  |

## 5.1	Solution #1: Support of application satellite coverage availability information (ASCAI) analysis

### 5.1.1	Solution description

This solution addresses the KI#6.

The VAL server or the UE may act as consumers to subscribe the analysis related to the application satellite coverage availability information (ASCAI) from ADAES. They may ask to analyse the application satellite coverage availability information of the target UE to assist the UE's satellite access or to minimize the impact on services (e.g. inform the VAL server in time to pause the downlink data when the target UE is unavailable under the satellite coverage).

#### 5.1.1.1	Procedure of Satellite related information subscription request

Figure 5.1.1.1-1 illustrates the high-level procedure of Satellite related information subscription request.

Pre-condition:

-	The following NFs in the application enablement layer (e.g., ADAES, CMS, and LMS) are all deployed on the ground.

Figure 5.1.1.1-1: Procedure of Satellite related information subscription request

1.	Consumers (e.g., the VAL server or UE) initiate a satellite communication analysis subscription request to the ADAES, with the analysis ID (i.e. the application satellite coverage availability information), analysis type (statistics or prediction), target UE ID, service ID, target service area, reporting conditions (e.g., periodic reporting or event-triggered reporting), etc.

2.	The ADAES checks whether the VAL server is authorized to invoke such request.

3.	If the request is authorized, the ADAES enabler sends a satellite communication analysis subscription response to the VAL server.

4a. ADAES may obtain the satellite related information for the target UE from the SEAL servers in the following ways:

-	ADAES may obtain the ASCAI related to the target UE from the configuration management server as specified in clause 21.2.2.1 of 3GPP TS 23.434 [4];

-	ADAES may retrieve the location information and access type of the target UE from the location management server as specified in clause 9.3.5 of 3GPP TS 23.434 [4].

4b. ADAES may obtain the satellite related information for the target UE from the 3GPP CN in the following ways:

-	ADAES may obtain the location information and UE mobility analysis information for the target UE from the 3GPP CN (i.e., NWDAF) as specified in clause 6.7.2 of 3GPP TS 23.288 [9]).

5.	The ADAES stores and analyses the received satellite related information based on the satellite communication analysis subscription service request received in step 1. The ADAES may generate the statistical and predicated ASCAI information for the target UE in a specific service area/time point. E.g., the available satellite information for the target UE in a certain area, the predicted time point when the target UE enters/exits the coverage of a certain satellite, or the predicted time periods during which the target UE is available under the coverage of a certain satellite.

NOTE 1:	Whether the ADAES provides the predictive satellite related information depends on the prediction parameter is included in Step 1or not.

6a.	The ADAES will report the ASCAI analysis (including the statistics and prediction information) to the consumer via the satellite communication analysis notification message. And the consumers may take actions to minimize the service impact if there is. For example, the VAL server may promptly adjust the downlink data polices (e.g., pending the downlink data) based on the time points when the UE is unavailable under the satellite coverage.

6b. The ADAES may generate the corresponding data polices based on the analysed satellite related information and sends these data polices to the consumer directly. These data polices below only apply when the target UE is unavailable under the satellite coverage:

-	Reject forwarding the data for new uplink and downlink data;

-	Pending forwarding the data and cache for new uplink and downlink data;

-	Forwarding the data according to priority for existing uplink and downlink data.

NOTE 2:	The priority for the forwarding polices may be set according to the service QoS/QoE, user levels, operator policies, pre-configurations, etc.

NOTE 3:	It’s up to the ADAES to decide which step (6a or 6b) is performed.

### 5.1.2	Architecture Impacts

This solution has no impact on the existing architecture.

### 5.1.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 5.1.4	Solution evaluation

This solution addresses the KI#6 to provide the application satellite coverage availability information (ASCAI) analytics when the VAL UE is using the satellite access.

The solution has no impact on the existing architecture. The ADAES can be enhanced to obtain the satellite related information from the SEAL servers or 3GPP core network, generate the ASCAI analytics (including the statistics and prediction information) and then report them to the consumers (VAL UE or VAL server).

Besides, the ADAES can generate the corresponding data polices based on the analysed satellite related information to guide the consumers how to operate the traffic data to minimize the service impacts.

## 5.2	Solution #2: Support of QoS analysis for services over satellite access

### 5.2.1	Solution description

This solution addresses the KI#6.

The VAL server or the UE may act as consumers to subscribe the QoS analysis related to the services over satellite access from ADAES. They may ask to provide the preferred or predicted QoS for the requested service over satellite access.

#### 5.2.1.1	Procedure of QoS analysis for services over satellite access

Figure 5.2.1.2-1 illustrates the high-level procedure of QoS analysis for services over satellite access.

Pre-condition:

-	The ADAES, VAL UE, VAL server, 3GPP CN (e.g., NWDAF) are all deployed on the ground.

- 	 The target UE is accessing the satellite.

Figure 5.2.1.1-1: Procedure of QoS analysis for services over satellite access

1.	Consumers (e.g., the VAL server or UE) initiate a satellite communication analysis subscription request to the ADAES, with the analysis ID (i.e. QoS for the satellite communication), analysis type (statistics or prediction), target UE ID, service ID, target service area, reporting conditions (e.g., periodic reporting or event-triggered reporting), service QoS parameters, etc.

NOTE 1:	The definition of QoS can refer to the clause 5.7 of 3GPP TS 23.501 [10].

2.	The ADAES checks whether the VAL server is authorized to invoke such request.

3.	If the request is authorized, the ADAES enabler sends a satellite communication analysis subscription response to the VAL server.

4.	ADAES may obtain the satellite communication related information for the target UE from the 3GPP CN (i.e., NWDAF, via NEF), including the location information, RAT type (i.e. satellite), UE mobility analysis information, the DN performance and QoS analysis information (such as the time delay, the traffic data flow and the uplink/downlink QoS parameters when the UE accesses via satellite access) as specified in clause 6.14 and clause 6.23 of 3GPP TS 23.288 [9]).

5.	Upon receiving the satellite communication related information for the target UE from step 4, the ADAES stores and analyses these information based on the service request received in step 1 to generate the performance or service quality (i.e., QoS) statistics and prediction related to satellite access. E.g., the ADAES may provide the preferred satellite and the preferred satellite type (such as GEO, MEO, LEO) when the UE is using the satellite access to meet the requested QoS requirements, or the recommended/predicated QoS parameters for the specific service when the UE is accessing the dedicated satellite.

6.	The ADAES will report the QoS analysis related to satellite communication, including preferred and prediction information (e.g. the preferred satellite and satellite type, recommended/predicated QoS parameters), to the consumer via satellite communication analysis notification message. And the consumers may take actions to minimize the service impact if there is. For example, when the target UE is accessing the GEO satellite, the VAL server may decrease the service QoS (e.g., latency), etc.

NOTE 2:	The actions from the consumer to minimize the service impact are out of scope of this specification.

### 5.2.2	Architecture Impacts

This solution has no impact on the existing architecture.

### 5.2.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 5.2.4	Solution evaluation

This solution addresses the KI#6 to support the QoS analysis for the services over the satellite access in the application enablement layer.

The solution has no impacts on the existing architecture. The ADAES can be enhanced to obtain the satellite communication related information for the target UE from the 3GPP core network and generate the preferred or predicted QoS analysis (e.g. the preferred satellite and satellite type, recommended/predicated QoS parameters, etc.) for the requested services over the satellite access.

NOTE:	Whether the procedure 5.1.1.1 of Sol#1 and the procedure 5.2.1.1 of Sol#2 can be combined or not will be discussed in normative phase.

## 5.3	Solution #3: Location management service via satellite access

### 5.3.0	General description

Figure 5.3.0-1, illustrates the deployment of location enabler on-board satellite for assisting location management service.

Figure 5.3.0-1: Deployment of location enabler on-board satellite for assisting location management service

Basically, in this architecture option, SEAL location management server is deployed on-board satellite (GEO, MEO, or LEO) to complement and for continuity of the location management service, especially when the feeder link is not available and S&F is supported.

In this deployment option, location management server is deployed both on ground and on-board satellite. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access on-board location management server is deployed on one or more satellites. RAN (e.g. gNB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground and/or on satellite, which is not depicted in the figure for simplicity. UPF is deployed on ground to access the location management server on the ground, and the UE can reach the location management server on ground or via space. The on-board location management server can be mobile depending on the satellite they are deployed on: GEO, MEO, or LEO.

The location management server, with the fused location function, may combine/aggregate location information from multiple sources including the location determined on-board satellite to provide a more accurate UE location. The location management server on-board satellite also assists to report the value-added location information (e.g. monitoring location deviation events, history location data, periodic verify UE location, prediction related to UE location, etc.) to the VAL server via satellite access.

### 5.3.1	Solution description

Figure 5.3.1-1 illustrates the procedure for on-board satellite location management server assisting SEAL location history procedure specified in 3GPP TS 23.434.

Pre-conditions:

1.	The location management server on-board satellite is deployed to assist the value-added location information.

Figure 5.3.1-1: Procedure for on-board satellite LMS assisted SEAL location history service

1.	Location tracing configuration from VAL server to the location management server (on-ground) as described in 3GPP TS 23.434 clause 9.3.21.1.

2.	The location management server (on-ground) sends a location tracing configuration request to the location management server (on-board satellite) to configure the location tracing service while the feeder link is available. The location tracing configuration request may contain the geographical area for history location tracing, events/conditions for triggering the location history report, location QoS, the identities for the UE and service, the time till when the SEAL LM needs to store the location history data and the required positioning method(s) , etc.

Editor's note:	How the target satellite to configure the location tracing service is determined is FFS.

3.	The location management server (on-board satellite) checks whether the location tracing configuration request is authorized and available to serve. If the request is available, the location management server (on-board satellite) stores the configuration received in step 2. If the request includes multiple UEs, the location management server creates and stores the contexts for each UE.

4.	The location management server (on-board satellite) sends the location tracing configuration responses to the location management server (on-ground).

5.	When the feeder link is not available, the location management server (on-board satellite) has obtained and stored the UE location data from multiple sources (e.g. UE, gNB/GMLC onboard, 3rd party), periodically based on the event triggers or conditions received in the request of step 2.

6.	Once the feeder link is available, the location management server (on-board satellite) forwards the stored UE location data towards the location management server (on-ground).

NOTE:	The interactions between the location management server (on-board satellite) and location management server (on-ground) are supported by LM-E reference point as described in 3GPP TS 23.434 clause 9.2.5.6.

### 5.3.2	Architecture Impacts

SEAL location management server is expected to be deployed on-board satellite. And the SEAL-E interface LM-E is required to support interaction between LMS on-ground to LMS on-board satellite.

### 5.3.3	Corresponding APIs

This solution impacts the SEAL-E interface LM-E with new interactions between LMS on-ground to LMS on-board satellite to support SEAL location history service via satellite access.

### 5.3.4	Solution evaluation

This solution provides a proposal to address the open issues listed in key issue#5 by supporting location management service via satellite access. Architecture impacts are identified where location management server is required to be deployed on-board satellite. SEAL-E interface LM-E is required with new interactions between LMS on-ground to LMS on-board satellite to support SEAL location history service via satellite access.

Editor's note:	It is FFS to evaluate further on the impact made by this solution to the use case.

## 5.4	Solution #4: Enhance SEALDD to support satellite selection in data delivery

### 5.4.1	Architecture

This solution is based on the SEALDD architecture as in 3GPP TS 23.433 [3].

### 5.4.2	Solution description

This solution addresses the KI#3, i.e., support satellite selection in data delivery.

Because the KI description states that operators deploy some components of the SEALDD architecture on satellites to ensure reliable data transmission via satellite communications, the solution proposes to deploy the SEALDD servers on-board satellites to achieve the large to global range of service coverage. Low-Earth-Orbit (or LEO) satellites are utilized to reduce the transmission latency. However, thanks to the lower altitude of LEOs, and also depending on the minimum elevation angles, the coverage range of LEO on Earth is limited. This will result in the shorter connection time for on-ground users via LEO-based satellite access.

For example, assuming a LEO satellite orbits at the 500 km altitude, which makes the orbital period be roughly 95 minutes. With a reasonable choice of the minimum elevation angle (e.g., 30 degree), a specific spot on Earth could access the satellite approximately for 7 minutes once within the coverage of the satellite. Accordingly, in some scenarios, incomplete data transmission via a LEO satellite might be experienced before the satellite moves aways and no longer resides within users’ connection range. Therefore, when the LEO satellite-assisted enabling layer performs data transmission, it is necessary to consider how to ensure uninterrupted data transmission and strive for service continuity.

With SEALDD servers on-board LEO satellites, the solution enhances the SEALDD enabled data transmission service with the dynamic discovery and selection of (target) SEALDD server based on the application satellite coverage availability information (i.e., ASCAI). Once the target SEALDD server (on-board another satellite) is selected, the enhanced procedure handles the switching of serving satellites along with the context transfer of the SEALDD servers deployed on-board the satellites.

In the solution, SEALDD clients on VAL UEs are on the ground.

#### 5.4.2.1	Procedures

##### 5.4.2.1.1	SEALDD enabled transmission with satellite selection support in data delivery

Figure 5.4.2.1.1-1 illustrates the procedure for the source SEALDD (or S-SEALDD) server to discover and select a target SEALDD (or T-SEALDD) server based on Application Satellite Coverage Availability Information or ASCAI, and then handle the switching of serving satellites along with the context transfer between the SEALDD servers.

Pre-condition:

-	SEALDD servers are deployed on-board satellites and SEALDD clients are on the ground.

-	The VAL server discovers and selects the S-SEALDD server by CAPIF functions.

-	The S-SEALDD server has gotten the ASCAI by leveraging the procedures like in the TS 23.434 [4] clause 21.2.2.

-	The existence and impact of the Inter-Satellite Links (or ISLs) that provide the transmission connectivity between satellites are out of the scope of the solution.

Figure 5.4.2.1.1-1: SEALDD enabled transmission with satellite selection support in data delivery

- The VAL server sends a Sdd_RegularTransmission request to the SEALDD server#1 (S-SEALDD), as specified in 3GPP TS 23.433 [3] clause 9.2.2.2. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), and optionally, the duration of the data connection. The S-SEALDD server is selected with the consideration of ASCAI.

- Upon receiving the request, the SEALDD server#1 performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server.

- The regular data transmission connection is established according to TS 23.433 [3] clause 9.2.2.2. The S-SEALDD server transmits data received from VAL server to SEALDD client.

- The VAL server uses the ASCAI and the duration of the data connection to determine whether the current connection can maintain the service continuity for data transmission, before the serving satellite (i.e., Sat#1 for SEALDD server#1) moves away from the VAL UE’s connection range. The current S-SEALDD server (on Sat#1) will continue the data connection if no service interruption is expected. If the duration of the data connection will lead to service interruption due to the moving away of the Sat#1, the VAL server triggers the decision process of the service continuity for the data transmission connection.

NOTE 1:	When the ASCAI is utilized to discover a list of the target SEALDD servers, the VAL UE’s information (e.g., location data) may be considered for the discovery. The SEAL location management server can provide the VAL UE information.

NOTE 2:	The VAL server can get the ASCAI by leveraging the procedures like in the TS 23.434 [4] clause 21.2.2.2.

- The S-SEALDD server (SEALDD server#1) performs candidate SEALDD servers discovery by using EEL, as specified in TS 23.433 [3] clause 9.4. The discovery can utilize various discovery filter (or query filters), with the consideration of ASCAI in the query filter.

NOTE 3:	The discovered candidate SEALDD servers must be on different serving satellites from the S-SEALDD server’s serving satellite (i.e., Sat#1), since SEALDD servers on-board the same satellite will have the same ASCAI that cannot address the data connection interruption issue

- Once the available target SEALDD servers are discovered, the S-SEALDD server can select the T-SEALDD server (i.e., SEALDD server #2) based on the coverage range, the coverage duration, or local configuration.

- The SEALDD context transfer procedure is performed between the S-SEALDD server and the T-SEALDD server, as specified in TS 23.433 [3] clause 9.6.2.1. Both SEALDD servers on-board different satellites interact via the reference point SEALDD-E to transfer data content and exchange information for SEALDD service provisioning, control, reporting etc., as specified in TS 23.433 [3] clause 7.4.5.

After the SEALDD context transfer procedure, the SEALDD client on VAL UE connects to the new T-SEALDD server (i.e., SEALDD server#2) to continue the data transmission connection.

#### 5.4.2.2	Information flows

The following information flows are based on from the clause of 9.2.3.1 in the 3GPP TS 23.433 [3]. All new changes are in bold.

### 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of specific UE or VAL user. SEALDD-S connection information; M; Address information (e.g., IP address and/or port, URL) of the VAL server to receive the traffic from the SEALDD server. QoS information; O; QoS information provided by VAL server. Duration of data connection; O; The total time a data transmission connection expects to last. VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users..

### 5.4.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 5.4.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 5.4.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 5.5	Solution #5: Using SEALDD to enhance application enablement layer for efficient content delivery over satellite access

### 5.5.1	Architecture

This solution is based on the SEALDD architecture as in 3GPP TS 23.433 [3].

### 5.5.2	Solution description

According to the 3GPP TS 23.433 [3], the SEALDD server offers the storage capabilities, which supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers. The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. For efficient content delivery with satellite access, while the in-network content caching service may be provided by the SEALDD server, the normal deployment locations of SEALDD servers, i.e., closer to VAL servers, will make the distributed requirement of content cache less achievable. This impacts the desired ubiquitous coverage for satellite access.

In TS 23.434 [4] upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on the VAL UE for better distribution. This part of the solution proposes to enhance the architecture of the SEALDD enabler by providing the data storage capability in SEALDD client(s). The SEALDD client (in VAL UE) is deployed on-ground for satellite access.

The 3GPP TS 23.434 [4] clause 21, SEAL services over satellite access, implies the satellite footprint or application satellite coverage availability information (ASCAI) can be leveraged for efficient content delivery. For example, applications leveraging the discontinuous coverage pattern between UEs and AS/AFs can help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed content cache in VAL UE (e.g. VAL UEs supporting content caching applications may be given precedence during a connectivity available period, UEs with large caching capacity may be better off utilizing longer connectivity interval, etc.). This part of the solution supplements the previous SEALDD layer architecture enhancement proposal by leveraging the satellite access characteristics for efficient content delivery.

#### 5.5.2.1	Procedures

##### 5.5.2.1.1	SEALDD enabled transmission for efficient content delivery

Figure 5.5.2.1.1-1 illustrates the procedure for the SEALDD server to determine the content caching capability of the SEALDD client, establishing the data transmission connection with the SEALDD client for efficient content delivery (or so-named ECD), as well as the SEALDD layer utilizing the satellite services to transmit contents between the VAL client and VAL server for better global service coverage.

Pre-condition:

-	The VAL server discovers and selects the SEALDD server by CAPIF functions.

-	VAL servers, VAL clients and SEALDD servers are deployed on the ground.

Figure 5.5.2.1.1-1: SEALDD enabled data transmission for efficient content delivery via satellite services

1.	The VAL server decides to use SEALDD service to enable efficient content delivery (ECD) for data transmission and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_ECDforSatAccessTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, ECD for satellite access requirement (e.g., indication, max cache capacity, etc.).

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates the SEALDD-S data transmission connection information (e.g., address/port) of the SEALDD server side to receive the application data packets from the VAL server to be delivered to the VAL UE. The SEALDD server responds with a Sdd_ECDforSatAccessTransmission response.

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional, if the SEALDD server uses the downlink pull mode to fetch the application data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the application data to the address provided by VAL server.

3.	If the SEALDD server receives from the VAL server the ECD for satellite access indication, the SEALDD server may invoke the SEAL NRM service to enquire the content caching capability and the data storage capacity from the VAL UE. A procedure like the TS 23.434 [4] clause 21.3.2.2 could be utilized. The NRM client on a VAL UE can report the content caching capability and maximum data storage quota per application layer on the VAL UE for all of services to the NRM server, which then sends to the SEALDD server.

NOTE 2:	While the procedure in TS 23.434 [4] clause 21.3.2.2 is about VAL UE reporting information from NRM client to NRM server, the procedure for NRM server to enquire and collect the VAL UE information from the NRM client makes no significant difference.

4.	If the received ECD indication shows the support of ECD in VAL UE, the SEALDD server and the SEALDD client enable the ECD for satellite access data transmission. Based on the ECD for satellite access requirement received from the VAL server or local configuration, the SEALDD server requires the SEALDD client to set up the content caching service and allocate content cache with provided content storage capacity.

NOTE 3:	Whether a VAL UE can support the ECD and accordingly store received contents is subject to the regulatory requirements and operator’s policy.

5.	The VAL server sends downlink application data to the SEALDD server which delivers it further to the VAL UE with the efficient content delivery or ECD.

NOTE 4:	Based on implementation, the SEALDD server can calculate the available capacity of the content cache in the VAL UE and adjust the volume of the DL transmission data to the SEALDD client accordingly.

NOTE 5:	The SEALDD server may receive the DL application data from the VAL server before the ECD is enabled in SEALDD client, during which the SEALDD server may store the application data locally.

6.	The SEALDD server controls the efficient downlink application data delivery to the SEALDD client based on the application satellite coverage availability information (or ASCAI).

NOTE 6:	The SEALDD server can leverage the procedures like in the 3GPP TS 23.434 [4] clause 21.2.2 to get the application satellite coverage availability information (or ASCAI).

7.	The SEALDD server sends the DL data to the SEALDD client in VAL UE via the 3GPP network.

NOTE 7:	The SEALDD enabled regular data transmission procedure in the 3GPP TS 23.433[3] clause 9.2.2.2 can be leveraged for the data transmission.

8.	The SEALDD client receives the DL data and save it into the content cache. Later, the VAL UE distributes the cached contents to other VAL UEs/VAL clients based on the settings of the content cache applications.

NOTE 8:	When contents are received and cached in the VAL UE, the VAL UE can distribute the contents via the refence point SEALDD-PC5. The 3GPP TS 23.433[3] clause 7.2 illustrates the architecture for SEALDD enabler service to support VAL UE-to-VAL UE communication. However, how a VAL UE/VAL client distributes the cached contents is implementation specific that is out of the scope of the document.

9.	Optionally, the SEALDD server may send Sdd_ECDforSatAcessTransmission notification to the VAL server to influence the DL application data transmission from the VAL server, e.g., if the content cache at the VAL UE is near its capacity, if the VAL UE connection status changes thanks to the variation of satellite connectivity, or if the DL data transmission from the VAL server needs to be adjusted.

#### 5.5.2.2	Information flows

##### 5.5.2.2.1	SEALDD enabled ECDforSatAccess transmission request

Table 5.5.2.2.1-1 describes the information flow from the VAL server to the SEALDD server to enable the efficient content delivery (ECD) for satellite access transmission service.

Table 5.5.2.2.1-1: SEALDD enabled ECDforSatAccess transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | M | Identifier of specific UE or VAL user |
| SEALDD-S endpoint information | M | Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. |
| ECD requirement | O | ECD requirement for satellite access (e.g., support indication, max storage size). |
| DL data delivery status subscription indication | O | Indicates the VAL server expected to receive the DL delivery status notification |

##### 5.5.2.2.2	SEALDD enabled ECDforSatAccess transmission response

Table 5.5.2.2.2-1 describes the information flow from the SEALDD server to the VAL server to respond to the efficient content delivery (ECD) for satellite access transmission service.

Table 5.5.2.2.2-1: SEALDD enabled ECDforSatAccess transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S endpoint information | O | Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g. SEALDD policy mismatch, ECD not support. |
| NOTE: The IE is only present if the Result is failure. |  |  |

##### 5.5.2.2.3	SEALDD enabled ECDforSatAccess transmission notification

Table 5.5.2.2.3-1 describes the information flow from the SEALDD server to the VAL server to notify the events related to the efficient content delivery (ECD) for satellite access transmission service.

Table 5.5.2.2.3-1: SEALDD enabled ECDforSatAccess transmission notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event SEALDD client connection status for DL ECD e.g., reachable, unreachable, cache near capacity. |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | O | Identity of the VAL service. |
| DL data delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL efficient data delivery |
| > Adjust DL data volume | O | Indicates adjusting DL data volume with suggested data volume |
| > Pause DL ECD | O | Indicates pausing DL ECD due to e.g., cache near capacity |
| > Resume DL ECD | O | Indicates enforcing DL ECD due to e.g., more capacity available |
| NOTE: Either Event ID IE or the DL data delivery instruction ID is present. |  |  |

### 5.5.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 5.5.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 5.5.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 5.x	Solution #x: <title>

Provide a suitable title for the solution.

### 5.x.1	Solution description

This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

### 5.x.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 5.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 5.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 6	Deployment scenarios

## 6.1	General

This clause will provide a general description of the deployment scenarios.

# 7	Business Relationships

Provide a description of the involved business relationships.

# 8	Overall evaluation

This clause will provide evaluation of different solutions.

# 9	Conclusions

## 9.1	General conclusions

This clause will provide general conclusions for the study.

## 9.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-09 | SA6#68 | S6-253656 |  |  |  | Skeleton for TR 23.700-02 | 0.0.0 |
| 2025-09 | SA6#68 | S6-253655 |  |  |  | Key Issue on AIML model storage and deployment on satellite | 0.1.0 |
| 2025-09 | SA6#68 | S6-253657 |  |  |  | Key Issue on Satellite based AIML service maintenance while losing connection with terrestrial network | 0.1.0 |
| 2025-09 | SA6#68 | S6-253660 |  |  |  | Key Issue on Support satellite switch selection in data delivery | 0.1.0 |
| 2025-09 | SA6#68 | S6-253722 |  |  |  | New KI on efficient content delivery over satellite access | 0.1.0 |
| 2025-09 | SA6#68 | S6-253732 |  |  |  | Pseudo-CR KI on location service via satellite access | 0.1.0 |
| 2025-09 | SA6#68 | S6-253755 |  |  |  | New KI on improving service performance over satellite access utilizing AI capabilities | 0.1.0 |
| 2025-10 | SA6#69 | S6-254605 |  |  |  | New Solution on support of satellite related information utilizing AI analysis | 0.2.0 |
| 2025-10 | SA6#69 | S6-254606 |  |  |  | New Solution for support of QoS analysis for services over satellite access | 0.2.0 |
| 2025-10 | SA6#69 | S6-254607 |  |  |  | Pseudo-CR Solution for location service via satellite access | 0.2.0 |
| 2025-11 | SA6#70 | S6-255134 |  |  |  | Solution evaluation for Sol#1 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255520 |  |  |  | Solution evaluation for Sol#2 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255521 |  |  |  | KI#3 Solution: Enhance SEALDD to support satellite selection in data delivery | 0.3.0 |
| 2025-11 | SA6#70 | S6-255522 |  |  |  | KI#4 Solution: Application enablement layer enhancement for efficient content delivery over satellite access | 0.3.0 |