Weighted segmental SNR test of Extended AMR-WB encoder

The following document summarizes all the steps needed to validate the weighted segmental signal-to-noise ratio (wSNR) in an implementation of the Extended AMR-WB (AMR-WB+) encoder according to the conformance specification TS 26.274.

There are a total of 18 tests separated in 6 categories. Each of the tests may be called by a single batch file named test_x_y.bat or all at the same time with the batch file named run_all_x_y_tests.bat. In this case we compare wSNR of the floating point encoder (26.304) and the fixed point encoder (26.273).

In order to verify a specific implementation, the executable name at Lines 28 and/or 29 of c_snr.bat need to be changed. The audio files location and names are defined in wseg_snr.bat

A tool named SNR_res.exe outputs the weighted segmental SNR degradation and the percentage of frames with a wSNR degradation higher than 4 dB in a text file. You can set the name and the output path of this file in c_snr.bat. The tool will mark with one star (*) all degradation over 1% and with two stars (**) degradation of more than 2%. Depending on the test, tolerated degradation is 1 or 2%. Also, the tool will mark with a <- when there is more than 2% of the frames having a wSNR degradation higher than 4 dB. However, this is not always a requirement.

The audio files used to execute this test are stored in the folder conformance\wseg_snr\audio. The list is described as follow:

concatener_green_test_amr

c_01_H3_org

c_02_H3_org

m_ch_x_1_org

m_cl_x_1_org

m_ot_x_1_org

m_ot_x_3_org

m_po_x_1_org

m_vo_x_1_org

p_01_H3_org

p_02_H3_org

s_cl_2t_3_org

s_no_2t_1_org

s_no_2t_2_org

s_no_ft_1_org

s_no_ft_2_org

s_no_ft_9_org

sbm_fi_x_9_org

sbm_sm_x_1_org

sbm_sm_x_2_org

si_01_H3_org

sm_01_H3_org

sm_02_H3_org

som_fi_x_1_org

som_fi_x_2_org

som_nt_x_1_org

som_ot_x_1_org

som_ot_x_9_org

sp_01_H3_org

sp_02_H3_org

v_01_H3_org