3GPP TSG-CT WG4 Meeting #127	C4-250564

# Athens, GREECE; 17th – 21st February 2025			revision of C4-250068

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1118 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NEF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NEF service names are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Type ServiceName is extended with missing service names |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Service Name values. Service producers cannot be discovered by consumers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.3.11, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29502_Nsmf_PDUSession.yaml TS29503_Nudm_SDM.yaml TS29503_Nudm_UECM.yaml TS29504_Nudr_DR.yaml TS29507_Npcf_AMPolicyControl.yaml TS29508_Nsmf_EventExposure.yaml TS29510_Nnrf_NFDiscovery.yaml TS29510_Nnrf_NFManagement.yaml TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29525_Npcf_UEPolicyControl.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Cover page updated |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.3.11	Enumeration: ServiceName

Table 6.1.6.3.11-1: Enumeration ServiceName

| Enumeration value | Description |
| --- | --- |
| "nnrf-nfm" | Nnrf_NFManagement Service offered by the NRF |
| "nnrf-disc" | Nnrf_NFDiscovery Service offered by the NRF |
| "nnrf-oauth2" | Nnrf_AccessToken Service offered by the NRF |
| "nudm-sdm" | Nudm_SubscriberDataManagement Service offered by the UDM |
| "nudm-uecm" | Nudm_UEContextManagement Service offered by the UDM |
| "nudm-ueau" | Nudm_UEAuthentication Service offered by the UDM |
| "nudm-ee" | Nudm_EventExposure Service offered by the UDM |
| "nudm-pp" | Nudm_ParameterProvision Service offered by the UDM |
| "nudm-niddau" | Nudm_NIDDAuthorization Service offered by the UDM |
| "nudm-mt" | Nudm_MT Service offered by the UDM |
| "nudm-ssau" | Nudm_ServiceSpecificAuthorization Service offered by the UDM |
| "nudm-rsds" | Nudm_ReportSMDeliveryStatus Service offered by the UDM |
| "nudm-ueid" | Nudm_UEIdentifier Service offered by the UDM |
| "namf-comm" | Namf_Communication Service offered by the AMF |
| "namf-evts" | Namf_EventExposure Service offered by the AMF |
| "namf-mt" | Namf_MT Service offered by the AMF |
| "namf-loc" | Namf_Location Service offered by the AMF |
| "namf-mbs-comm" | Namf_MBSCommunication Service offered by AMF |
| "namf-mbs-bc" | Namf_MBSBroadcast Service offered by AMF |
| "nsmf-pdusession" | Nsmf_PDUSession Service offered by the SMF |
| "nsmf-event-exposure" | Nsmf_EventExposure Service offered by the SMF |
| "nsmf-nidd" | Nsmf_NIDD Service offered by the SMF |
| "nausf-auth" | Nausf_UEAuthentication Service offered by the AUSF |
| "nausf-sorprotection" | Nausf_SoRProtection Service offered by the AUSF |
| "nausf-upuprotection" | Nausf_UPUProtection Service offered by the AUSF |
| "nnef-pfdmanagement" | Nnef_PFDManagement offered by the NEF |
| "nnef-smcontext" | Nnef_SMContext Service offered by the NEF |
| "nnef-smservice" | Nnef_SMService Service offered by the NEF |
| "nnef-eventexposure" | Nnef_EventExposure Service offered by the NEF |
| "nnef-eas-deployment" | Nnef_EASDeployment InfoService offered by the NEF. This is the southbound part of the API (e.g. the service operations used by the SMF) |
| "nnef-dnai-mapping" | Nnef_DNAIMapping Service offered by the NEF |
| "nnef-traffic-influence-data" | Nnef_TrafficInfluenceData Service offered by the NEF |
| "nnef-ecs-addr-cfg-info" | Nnef_ECSAddress Service offered by the NEF |
| "nnef-ueid" | Nnef_UEId Service offered by the NEF |
| "3gpp-cp-parameter-provisioning" | Nnef_ParameterProvision Service offered by the NEF |
| "3gpp-device-triggering" | Nnef_Trigger Service offered by the NEF |
| "3gpp-bdt" | Nnef_BDTPNegotiation Service offered by the NEF |
| "3gpp-traffic-influence" | Nnef_TrafficInfluence Service offered by the NEF |
| "3gpp-chargeable-party" | Nnef_ChargeableParty Service offered by the NEF |
| "3gpp-as-session-with-qos" | Nnef_AFsessionWithQoS Service offered by the NEF |
| "3gpp-msisdn-less-mo-sms" | Nnef_MSISDN-less_MO_SMS Service offered by the NEF |
| "3gpp-service-parameter" | Nnef_ServiceParameter Service offered by the NEF |
| "3gpp-monitoring-event" | Nnef_APISupportCapability Service offered by the NEF |
| "3gpp-nidd-configuration-trigger" | Nnef_NIDDConfiguration Service offered by the NEF |
| "3gpp-nidd" | Nnef_NIDD Service offered by the NEF |
| "3gpp-analyticsexposure" | Nnef_AnalyticsExposure Service offered by the NEF |
| "3gpp-racs-parameter-provisioning" | Nnef_UCMFProvisioning Service offered by the NEF |
| "3gpp-ecr-control" | Nnef_ECRestriction Service offered by the NEF |
| "3gpp-applying-bdt-policy" | Nnef_ApplyPolicy Service offered by the NEF |
| "3gpp-mo-lcs-notify" | Nnef_Location Service offered by the NEF |
| "3gpp-time-sync" | Nnef_TimeSynchronization Service offered by the NEF |
| "3gpp-am-influence" | Nnef_AMInfluence Service offered by the NEF |
| "3gpp-am-policyauthorization" | Nnef_AMPolicyAuthorization |
| "3gpp-akma" | Nnef_AKMA Service offered by the NEF |
| "3gpp-eas-deployment" | Nnef_EASDeployment Service offered by the NEF. This is the northbound part (e.g. the service operations used by the AF). |
| "3gpp-iptvconfiguration" | Nnef_IPTV_configuration Service offered by the NEF |
| "3gpp-mbs-tmgi" | Nnef_MBSTMGI Service offered by the NEF |
| "3gpp-mbs-session" | Nnef_MBSSession Service offered by the NEF |
| "3gpp-authentication" | Nnef_Authentication Service offered by the NEF |
| "3gpp-asti" | Nnef_ASTI Service offered by the NEF |
| "3gpp-pdtq-policy-negotiation" | Nnef_PDTQPolicyNegotiation offered by the NEF |
| "3gpp-musa" | Nnef_MemberUESelectionAssistance offered by the NEF |
| "3gpp-5glan-pp" |  |  | 5G LAN Parameter Provisioning service offered by the NEF |  |
| "3gpp-lpi-pp" |  |  | Location Privacy Indicator Parameter Provision service offered by the NEF |  |
| "3gpp-acs-pp" |  |  | ACS Parameter Provision service offered by the NEF |  |
| "3gpp-ecs-address-provision" |  |  | ECS Parameter Provision service offered by the NEF |  |
| "3gpp-data-reporting" |  |  | Data Reporting service offered by the NEF |  |
| "3gpp-data-reporting-provisioning" |  |  | Data Reporting Provisioning service offered by the NEF |  |
| "3gpp-ueid" |  |  | UE Identifier service offered by the NEF |  |
| "3gpp-mb-us" |  |  | MBS User Service service offered by the NEF |  |
| "3gpp-mb-ud-ingest" |  |  | MBS User Data Ingest Session service offered by the NEF |  |
| "3gpp-ms-event-exposure" |  |  | Media Streaming Event Exposure service offered by the NEF |  |
| "3gpp-mbs-group-msg" |  |  | MBS Group Message Delivery service offered by the NEF |  |
| "3gpp-dnai-mapping" |  |  | DNAI Mapping service offered by the NEF |  |
| "3gpp-grp-pp" |  |  | Group Parameter Provisioning service offered by the NEF |  |
| "3gpp-slice-pp" |  |  | Slice Parameter Provisioning service offered by the NEF |  |
| "3gpp-ue-address" |  |  | UE Address service offered by the NEF |  |
| "3gpp-ecs-address" |  |  | ECS Address service offered by the NEF |  |
| "3gpp-rslppi-pp" |  |  | RSLPPI Parameter Provisioning service offered by the NEF |  |
| "3gpp-group-message-delivery-mb2" | Group Message Delivery via MBMS by MB2 service offered by the NEF |
| "3gpp-group-message-delivery-xmb" | Group Message Delivery via MBMS by xMB service offered by the NEF |
| "3gpp-network-status-reporting" | Network Status Reporting service offered by the NEF |
| "3gpp-pfd-management" | Packet Flow Description service offered by the NEF |
| "3gpp-network-parameter-configuration" | Network Parameter Configuration service offered by the NEF |
| "npcf-am-policy-control" | Npcf_AMPolicyControl Service offered by the PCF |
| "npcf-smpolicycontrol" | Npcf_SMPolicyControl Service offered by the PCF |
| "npcf-policyauthorization" | Npcf_PolicyAuthorization Service offered by the PCF |
| "npcf-bdtpolicycontrol" | Npcf_BDTPolicyControl Service offered by the PCF |
| "npcf-eventexposure" | Npcf_EventExposure Service offered by the PCF |
| "npcf-ue-policy-control" | Npcf_UEPolicyControl Service offered by the PCF |
| "npcf-am-policyauthorization" | Npcf_AM_PolicyAuthorization Service offered by the PCF |
| "npcf-pdtq-policy-control" | Npcf_PDTQPolicyControl Service offered by the PCF |
| "npcf-mbspolicycontrol" | Npcf_MBSPolicyControl Service offered by the PCF |
| "npcf-mbspolicyauth" | Npcf_MBSPolicyAuthorization Service offered by the PCF |
| "nsmsf-sms" | Nsmsf_SMService Service offered by the SMSF |
| "nnssf-nsselection" | Nnssf_NSSelection Service offered by the NSSF |
| "nnssf-nssaiavailability" | Nnssf_NSSAIAvailability Service offered by the NSSF |
| "nudr-dr" | Nudr_DataRepository Service offered by the UDR |
| "nudr-group-id-map" | Nudr_GroupIDmap Service offered by the UDR |
| "nlmf-loc" | Nlmf_Location Service offered by the LMF |
| "nlmf-broadcast" | Nlmf_Broadcast Service offered by the LMF |
| "n5g-eir-eic" | N5g-eir_EquipmentIdentityCheck Service offered by the 5G-EIR |
| "nbsf-management" | Nbsf_Management Service offered by the BSF |
| "nchf-spendinglimitcontrol" | Nchf_SpendingLimitControl Service offered by the CHF |
| "nchf-convergedcharging" | Nchf_Converged_Charging Service offered by the CHF |
| "nchf-offlineonlycharging" | Nchf_OfflineOnlyCharging Service offered by the CHF |
| "nnwdaf-eventssubscription" | Nnwdaf_EventsSubscription Service offered by the NWDAF |
| "nnwdaf-analyticsinfo" | Nnwdaf_AnalyticsInfo Service offered by the NWDAF |
| "nnwdaf-datamanagement" | Nnwdaf_DataManagement Service offered by the NWDAF |
| "nnwdaf-mlmodelprovision" | Nnwdaf_MLModelProvision Service offered by the NWDAF |
| "nnwdaf-mlmodeltraining" | Nnwdaf_MLModelTraining Service offered by the NWDAF |
| "nnwdaf-mlmodelmonitor" | Nnwdaf_MLModelMonitor Service offered by the NWDAF |
| "nnwdaf-roamingdata" | Nnwdaf_RoamingData Service offered by the NWDAF |
| "nnwdaf-roaminganalytics" | Nnwdaf_RoamingAnalytics Service offered by the NWDAF |
| "ngmlc-loc" | Ngmlc_Location Service offered by GMLC |
| "nucmf-provisioning" | Nucmf_Provisioning Service offered by UCMF |
| "nucmf-uecapabilitymanagement" | Nucmf_UECapabilityManagement Service offered by UCMF |
| "nhss-sdm" | Nhss_SubscriberDataManagement Service offered by the HSS |
| "nhss-uecm" | Nhss_UEContextManagement Service offered by the HSS |
| "nhss-ueau" | Nhss_UEAuthentication Service offered by the HSS |
| "nhss-ee" | Nhss_EventExposure Service offered by the HSS |
| "nhss-ims-sdm" | Nhss_imsSubscriberDataManagement Service offered by the HSS |
| "nhss-ims-uecm" | Nhss_imsUEContextManagement Service offered by the HSS |
| "nhss-ims-ueau" | Nhss_imsUEAuthentication Service offered by the HSS |
| "nhss-gba-sdm" | Nhss_gbaSubscriberDataManagement Service offered by the HSS |
| "nhss-gba-ueau" | Nhss_gbaUEAuthentication Service offered by the HSS |
| "nsepp-telescopic" | Nsepp_Telescopic_FQDN_Mapping Service offered by the SEPP |
| "nsoraf-sor" | Nsoraf_SteeringOfRoaming Service offered by the SOR-AF |
| "nspaf-secured-packed" | Nspaf_SecuredPacket Service offered by the SP-AF |
| "nudsf-dr" | Nudsf Data Repository service offered by the UDSF. |
| "nudsf-timer" | Nudsf Timer service offered by the UDSF |
| "nnssaaf-nssaa" | Nnssaaf_NSSAA service offered by the NSSAAF. |
| "nnssaaf-aiw" | Nnssaaf_AIW service offered by the NSSAAF. |
| "naanf-akma" | Naanf_AKMA service offered by the AAnF. |
| "n5gddnmf-discovery" | N5g-ddnmf_Discovery service offered by 5G DDNMF |
| "nmfaf-3dadatamanagement" | Nmfaf 3daDataManagement service offered by the MFAF. |
| "nmfaf-3cadatamanagement" | Nmfaf 3caDataManagement service offered by the MFAF. |
| "nmfaf-contextmanagement" | Nmfaf ContextManagement service offered by the MFAF. |
| "neasdf-dnscontext" | Neasdf_DNSContext service offered by the EASDF |
| "neasdf-baselinednspattern" | Neasdf_BaselineDNSPattern service offered by the EASDF |
| "ndccf-datamanagement" | Ndccf_DataManagement service offered by the DCCF. |
| "ndccf-contextmanagement" | Ndccf_ContextManagement service offered by the DCCF. |
| "nnsacf-nsac" | Nnsacf_NSAC service offered by the NSACF. |
| "nnsacf-slice-ee" | Nnsacf_SliceEventExposure service offered by the NSACF. |
| "nmbsmf-tmgi" | Nmbsmf TMGI service offered by the MB-SMF |
| "nmbsmf-mbssession" | Nmbsmf MBSSession service offered by the MB-SMF |
| "nadrf-datamanagement" | Nadrf_DataManagement service offered by the ADRF. |
| "nadrf-mlmodelmanagement" | Nadrf_MLModelManagement service offered by the ADRF. |
| "nbsp-gba" | Nbsp_GBA service offered by the GBA BSF. |
| "ntsctsf-time-sync" | Ntsctsf_TimeSynchronization service offered by the TSCTSF |
| "ntsctsf-qos-tscai" | Ntsctsf_QoSandTSCAssistance service offered by the TSCTSF |
| "ntsctsf-asti" | Ntsctsf_ASTI service offered by the TSCTSF |
| "npkmf-keyrequest" | Npkmf_PKMFKeyRequest service offered by the PKMF |
| "npkmf-userid" | Npkmf_ResolveRemoteUserId service offered by the PKMF |
| "npkmf-discovery" | Npkmf_Discovery service offered by the PKMF |
| "nmnpf-npstatus" | Nmnpf_NPStatus service offered by the MNPF |
| "niwmsc-smservice" | Niwmsc_SMService service offered by the SMS-IWMSC |
| "nmbsf-mbs-us" | Nmbsf_MBSUserService service offered by the MBSF |
| "nmbsf-mbs-ud-ingest" | Nmbsf_MBSUserDataIngestSession service offered by the MBSF |
| "nmbstf-distsession" | Nmbstf_MBSDistributionSession service offered by the MBSTF |
| "npanf-prosekey" | Npanf_ProseKey service offered by the PAnF |
| "npanf-userid" | Npanf_ResolveRemoteUserId service offered by the PAnF |
| "nupf-ee" | Nupf_EventExposure service offered by the UPF |
| "nupf-gueip" | Nupf_GetUEPrivateIPaddrAndIdentifiers Service offered by the UPF |
| "naf-prose" | Naf_ProSe Service offered by the AF |
| "naf-eventexposure" | Naf_EventExposure Service offered by the AF |
| NOTE: The services defined in this table are those defined by 3GPP NFs in 5GC; however, in order to support custom services offered by standard and custom NFs, the NRF shall also accept the registration of NF Services with other service names. |  |

* * * Next Change * * * *

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

**************text not shown for clarity***********

ServiceName:

description: Service names known to NRF

anyOf:

- type: string

enum:

- nnrf-nfm

- nnrf-disc

- nnrf-oauth2

- nudm-sdm

- nudm-uecm

- nudm-ueau

- nudm-ee

- nudm-pp

- nudm-niddau

- nudm-mt

- nudm-ssau

- nudm-rsds

- nudm-ueid

- namf-comm

- namf-evts

- namf-mt

- namf-loc

- namf-mbs-comm

- namf-mbs-bc

- nsmf-pdusession

- nsmf-event-exposure

- nsmf-nidd

- nausf-auth

- nausf-sorprotection

- nausf-upuprotection

- nnef-pfdmanagement

- nnef-smcontext

- nnef-smservice

- nnef-eventexposure

- nnef-eas-deployment

- nnef-dnai-mapping

- nnef-traffic-influence-data

- nnef-ecs-addr-cfg-info

- nnef-ueid

- 3gpp-cp-parameter-provisioning

- 3gpp-device-triggering

- 3gpp-bdt

- 3gpp-traffic-influence

- 3gpp-chargeable-party

- 3gpp-as-session-with-qos

- 3gpp-msisdn-less-mo-sms

- 3gpp-service-parameter

- 3gpp-monitoring-event

- 3gpp-nidd-configuration-trigger

- 3gpp-nidd

- 3gpp-analyticsexposure

- 3gpp-racs-parameter-provisioning

- 3gpp-ecr-control

- 3gpp-applying-bdt-policy

- 3gpp-mo-lcs-notify

- 3gpp-time-sync

- 3gpp-am-influence

- 3gpp-am-policyauthorization

- 3gpp-akma

- 3gpp-eas-deployment

- 3gpp-iptvconfiguration

- 3gpp-mbs-tmgi

- 3gpp-mbs-session

- 3gpp-authentication

- 3gpp-asti

- 3gpp-pdtq-policy-negotiation

- 3gpp-musa

- 3gpp-5glan-pp

- 3gpp-lpi-pp

- 3gpp-acs-pp

- 3gpp-ecs-address-provision

- 3gpp-data-reporting

- 3gpp-data-reporting-provisioning

- 3gpp-ueid

- 3gpp-mb-us

- 3gpp-mb-ud-ingest

- 3gpp-event-exposure

- 3gpp-mbs-group-msg

- 3gpp-dnai-mapping

- 3gpp-grp-pp

- 3gpp-slice-pp

- 3gpp-ue-address

- 3gpp-ecs-address

- 3gpp-rslppi-pp

- 3gpp-group-message-delivery-mb2

- 3gpp-group-message-delivery-xmb

- 3gpp-network-status-reporting

- 3gpp-pfd-management

- 3gpp-network-parameter-configuration

- npcf-am-policy-control

- npcf-smpolicycontrol

- npcf-policyauthorization

- npcf-bdtpolicycontrol

- npcf-eventexposure

- npcf-ue-policy-control

- npcf-am-policyauthorization

- npcf-pdtq-policy-control

- npcf-mbspolicycontrol

- npcf-mbspolicyauth

- nsmsf-sms

- nnssf-nsselection

- nnssf-nssaiavailability

- nudr-dr

- nudr-group-id-map

- nlmf-loc

- nlmf-broadcast

- n5g-eir-eic

- nbsf-management

- nchf-spendinglimitcontrol

- nchf-convergedcharging

- nchf-offlineonlycharging

- nnwdaf-eventssubscription

- nnwdaf-analyticsinfo

- nnwdaf-datamanagement

- nnwdaf-mlmodelprovision

- nnwdaf-mlmodeltraining

- nnwdaf-mlmodelmonitor

- nnwdaf-roamingdata

- nnwdaf-roaminganalytics

- ngmlc-loc

- nucmf-provisioning

- nucmf-uecapabilitymanagement

- nhss-sdm

- nhss-uecm

- nhss-ueau

- nhss-ee

- nhss-ims-sdm

- nhss-ims-uecm

- nhss-ims-ueau

- nhss-gba-sdm

- nhss-gba-ueau

- nsepp-telescopic

- nsoraf-sor

- nspaf-secured-packet

- nudsf-dr

- nudsf-timer

- nnssaaf-nssaa

- nnssaaf-aiw

- naanf-akma

- n5gddnmf-discovery

- nmfaf-3dadatamanagement

- nmfaf-3cadatamanagement

- nmfaf-contextmanagement

- neasdf-dnscontext

- neasdf-baselinednspattern

- ndccf-datamanagement

- ndccf-contextmanagement

- nnsacf-nsac

- nnsacf-slice-ee

- nmbsmf-tmgi

- nmbsmf-mbssession

- nadrf-datamanagement

- nadrf-mlmodelmanagement

- nbsp-gba

- ntsctsf-time-sync

- ntsctsf-qos-tscai

- ntsctsf-asti

- npkmf-keyreq

- npkmf-userid

- npkmf-discovery

- nmnpf-npstatus

- niwmsc-smservice

- nmbsf-mbs-us

- nmbsf-mbs-ud-ingest

- nmbstf-distsession

- npanf-prosekey

- npanf-userid

- nupf-ee

- nupf-gueip

- naf-prose

- naf-eventexposure

- type: string

**************text not shown for clarity***********

* * * End of Change * * * *