3GPP TSG-CT WG4 Meeting #127	C4-250206

Athens, GREECE; 17th – 21st February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0226 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Encrypted Block Locations in SEPP Service Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the specification, the URI Path and Query Fragments in the Request line may contain sensitive information IEs to be protected, i.e. containing the index to the "DataToIntegrityProtectAndCipher" block. However, the service description of SEPP only indicates the HTTP header and JSON payload IE: #### 5.3.2.3 Message Reformatting … Figure 5.3.2.3-2 Transformation of HTTP Header and Content to Encrypt into CipherText 1. Based on the protection policy exchanged between the SEPPs, the sending SEPP prepares an input for the JWE ciphering and integrity protection as an array of arbitrary types in the "DataToIntegrityProtectAndCipher" block with each entry containing either a HTTP header value or the value of a JSON payload IE of the API message being reformatted. The index value "encBlockIdx" in the contentpart of DataToIntegrityProtectBlock shall point to the index of a header value or IE value in this input array. Also this CR correct the table header of Table 6.2.5.2.13-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Correct the service description that the sensitive information in URI path and query parameters shall also be encrypted. 2/ correct the table header of Table 6.2.5.2.13-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent contents in specifications, leads to different implementation and interworking issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3, 6.2.5.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not require version update on OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.3.2.3	Message Reformatting

A SEPP on the sending side PLMN applies message reformatting in the following cases:

-	When it receives a HTTP/2 request message from an NF service consumer to a an NF service producer in another PLMN;

-	When it receives a response HTTP/2 response message from an NF service producer to an NF service consumer in another PLMN.

-	When it receives a HTTP/2 notification request message from an NF service producer to an NF service consumer in another PLMN;

-	When it receives a HTTP/2 notification response message from an NF service consumer to an NF service producer in another PLMN

The SEPP shall reformat the HTTP/2 message by encapsulating the whole message into the body of a new HTTP POST message. The body of the HTTP POST request / response message shall contain the reformatted original HTTP/2 request/response message respectively. The HTTP POST request/response body shall be encoded as the "N32fReformattedReqMsg"/"N32fReformattedRspMsg" JSON bodies respectively, as specified in clause 6.2.5.

The "N32fReformattedReqMsg"/"N32fReformattedRspMsg" are structured as given below:

Figure 5.3.2.3-1 JSON representation of a reformatted HTTP message

The "cipherText" part of the reformatted message in FlatJweJson shall be prepared as given below

Figure 5.3.2.3-2 Transformation of Sensitive Information Elements to Encrypt into CipherText

1.	Based on the protection policy exchanged between the SEPPs, the sending SEPP prepares an input for the JWE ciphering and integrity protection as an array of arbitrary types in the "DataToIntegrityProtectAndCipher" block with each entry containing either a HTTP header value, the value of a variable in URI path, the value of an URI query parameter or the value of a JSON payload IE of the API message being reformatted. The index value "encBlockIdx" in the contentpart of DataToIntegrityProtectBlock shall point to the index of a header value or IE value in this input array.

2.	The input block is fed into an encryption function along with the other required inputs for JWE as specified in IETF RFC 7516 [14].

3.	The encryption function outputs the cipher text information. This cipher text is then subjected to BASE64URL transformation as specified in IETF RFC 4648 [15] clause 5.

4. The output of the BASE64URL transform is them encoded as the ciphertext part of FlatJweJson IE specified in clause 6.2.5.2.11.

* * * Next Change * * * *

##### 6.2.5.2.13	Type: IndexToEncryptedValue

Table 6.2.5.2.13-1: Definition of type IndexToEncryptedValue

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| encBlockIndex | Uinteger | M | 1 | Index to the value in DataToIntegrityProtectAndCipherBlock |

* * * End of Changes * * * *