3GPP TSG-CT WG4 Meeting #127	C4-250440

Athens, GREECE; 17th – 21st February 2025	was C4-250301

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0935 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Header Handling Control Rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an editor's note related to the Modification of Header Handling Control Rules needs to be resolved. Editor's note: it should be described how the SMF can modify header handling instructions of a PDU session at the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the relevant description for the modifications of Header Handling Control Rules, including the modification of the Header Handling Reporting Control Info and the Header Handling Control information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | stage 2 requirement is not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.41.2, 7.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Improve the description for the modification of the Header Handling Control Rule and move the Header Handling Control ID to the beginning of the message table. |  |  |  |  |  |  |  |  |

# *** First Change ***

### 5.41.2	Provisioning of Header Handling Control Rules

When receiving the Header Handling Control information as part of a PCC rule during a PFCP session establishment or a PFCP session modification procedure, the SMF shall instruct the UPF to perform the handling of payload headers by provisioning a FAR containing one or more Header Handling Control Rule IEs and associate the FAR with the PDR corresponding to the PCC rule.

The SMF shall provision the following IEs in the Header Handling Control Rule IE:

-	the Header Detection Reference IE which sets to a reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions;

-	the Header Detection Support Information IE (if received from the PCF) which contains any dynamic configuration required for the detection of the headers and cannot be preconfigured in the UPF;

-	one or more Header Handling Reporting Control Info IEs (if received from the PCF) to instruct the UPF to report the handling of payload headers information events as further described in clause 5.x.3;

-	one or more Header Handling Control information IEs (when multiple actions are requested on the target traffic, the enforcement of the Header Handling Action shall be performed in the following order: Detect, Remove, Replace, and Insert):

-	the Header Handling Control Reference IE set to a reference to a header handling action related information pre-configured in the UPF. The Header Handling Control Reference IE may be provided alone or together with the IEs below. When the Header Handling Control Reference IE is provided with any other IEs in the Header Handling Control information IE, the data provided by those IEs shall overwrite the related information pre-configured in the UPF.

-	the Header Handling Action IE indicating the header handling action to be performed by the UPF, i.e. detect, remove, replace or insert;

-	the Header Information IE set to a reference to a UPF configuration which defines how to identify or build a specific header field for which to perform the header handling action;

-	the Header Value IE providing the value of the specific header field. Header value shall be provided when Header Handling Action IE is set to replace, and it may be provided for remove, insert and detect actions (see also clause 8.2.257);-	the Header Handling Condition IE indicating whether the action shall be applied under certain events, i.e. first match or every match;

-	the Header Handling Reporting IE indicating whether reporting is requested or not for the performed Header Handling Action and for each of the Reporting Endpoint.

NOTE 1:	Having multiple Header Handling Reporting IE(s) in the Header Handling Control information IE enables different NFs to receive different set of the header handling information, e.g. the SMF can be configured to receive only "detect" header handling information report while the AF/NEF can request to receive all header handling actions information report.

The SMF may modify an existing Header Handling Control Rule by referring to the corresponding Header Handling Control Rule ID with one or more new/modified/deleted IEs in the Header Handling Control Rule IE to request the UPF to update this Header Handling Control Rule in the Update FAR:

-	the SMF may provision a new Header Handling Control information, or may delete an existing Header Handling Control information by including only Header Handling Control ID. The SMF may modify an existing Header Handling Control information by referring the Header Handling Control ID and shall include all IEs applicable for the Header Handling Control information, so the UPF shall replace any earlier received for this Header Handling Control information;

-	Similiar to the handling for Header Handling Control information, the SMF may modify and delete an existing Header Handling Reporting Control Info or add a new Header Handling Reporting Control Info;

-	the SMF may modify an existing IE provisioned earlier (other than the Header Handling Control information and Header Handling Reporting Control Info) with a new value, or delete it with a null length;

-	the SMF may provision a new IE defined in the Header Handling Control Rule IE which has not been provisioned earlier;

-	previously provisioned IE(s) in this Header Handling Control Rule but not included in the modification shall remain unchanged in the UPF.

The SMF may provision a new Header Handling Control Rule including a new Header Handling Control Rule ID in an Update FAR or in a Create FAR.

To delete an existing Header Handling Control Rule, the SMF shall provision the corresponding Header Handling Control Rule IE with only the Header Handling Control Rule ID IE identifying the Header Handling Control Rule IE.

Any existing Header Handling Control Rules provisioned previously shall remain unchanged in the UPF unless it is explicitly modified or deleted as described above.

To delete the complete set of Header Handling Control Rules in the Forwarding Parameters IE, the SMF shall provision an instance of Header Handling Control Rule IE with a null length.

# *** Next Change ***

#### 7.5.2.3	Create FAR IE within PFCP Session Establishment Request

The Create FAR grouped IE shall be encoded as shown in Figure 7.5.2.3-1.

Table 7.5.2.3-1: Create FAR IE within PFCP Session Establishment Request

| Octet 1 and 2 |  | Create FAR IE Type = 3 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| FAR ID | M | This IE shall uniquely identify the FAR among all the FARs configured for that PFCP session. | X | X | X | X | X | FAR ID |
| Apply Action | M | This IE shall indicate the action to apply to the packets, See clauses 5.2.1 and 5.2.3. | X | X | X | X | X | Apply Action |
| Forwarding Parameters | C | This IE shall be present when the Apply Action requests the packets to be forwarded. It may be present otherwise. When present, this IE shall contain the forwarding instructions to be applied by the UP function when the Apply Action requests the packets to be forwarded. See table 7.5.2.3-2. | X | X | X | X | - | Forwarding Parameters |
| Duplicating Parameters | C | This IE shall be present when the Apply Action requests the packets to be duplicated. It may be present otherwise. When present, this IE shall contain the forwarding instructions to be applied by the UP function for the traffic to be duplicated, when the Apply Action requests the packets to be duplicated. Several IEs with the same IE type may be present to represent to duplicate the packets to different destinations. See NOTE 1. See table 7.5.2.3-3. | X | X | - | - | - | Duplicating Parameters |
| BAR ID | O | When present, this IE shall contain the BAR ID of the BAR defining the buffering instructions to be applied by the UP function when the Apply Action requests the packets to be buffered. | X | - | - | X | - | BAR ID |
| Redundant Transmission Forwarding Parameters | C | This IE shall be present when the Apply Action requests the packets to be duplicated for redundant transmission and the Forwarding Parameters IE is included. It may be present otherwise. When present, this IE shall contain the forwarding instructions to be applied by the UP function for the traffic to be duplicated, when the Apply Action requests the packets to be duplicated for redundant transmission. Except for the parameters included in the Redundant Transmission Parameters IE, the duplicated packets shall apply the same parameters as those indicated in the Forwarding Parameters IE. See table 7.5.2.3-4. | - | - | - | - | - | Redundant Transmission Forwarding Parameters |
| MBS Multicast Parameters | C | This IE shall be present when the Apply Action is set to "FSSM". This requests the MB-UPF to forward the MBS session data to a Low Layer SSM. | - | - | - | - | X | MBS Multicast Parameters |
| Add MBS Unicast Parameters | C | This IE shall be present when the Apply Action is set to "MBSU". This requests the MB-UPF to forward the MBS session data to a remote GTP-U peer for unicast transport. Several IEs with the same IE type may be present to request the MB-UPF to forward the MBS session data to multiple remote GTP-U peers. | - | - | - | - | X | Add MBS Unicast Parameters |
| NOTE 1: The same user plane packets may be required, according to operator's policy and configuration, to be duplicated to different SX3LIFs. |  |  |  |  |  |  |  |  |

******Skipped for Clarity********

Table 7.5.2.3-7: Header Handling Control Rule IE in Forwarding Parameters

| Octet 1 and 2 |  | Header Handling Control Rule IE = 361 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Header Handling Control Rule ID | M | This IE shall identify an instance of Header Handling Control Rule IE. | - | - | - | X | - | Header Handling Control Rule ID |
| Header Detection Reference | M | This IE shall contain a reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions. | - | - | - | X | - | Header Detection Reference |
| Header Detection Support Information | O | When present, this IE shall contain dynamic information provided by the NF service consumer. Its format is unspecified and operator specific. | - | - | - | X | - | Header Detection Support Information |
| Header Handling Reporting Control Info | O | When present, this IE shall contain instructions for the UPF to report the handling of payload headers information events. Several IEs with the same IE type may be present to represent multiple Header Handling Reporting Control Infos corresponding to different receivers. | - | - | - | X | - | Header Handling Reporting Control Info |
| Header Handling Control information | M | This IE shall contain a header handling control information. Several IEs with the same IE type may be present to represent multiple Header Handling Control information containing different header handling actions. | - | - | - | X | - | Header Handling Control information |

Table 7.5.2.3-8: Header Handling Reporting Control Info IE in Header Handling Control Rule

| Octet 1 and 2 |  | Header Handling Reporting Control Info IE = 362 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Reporting Endpoint ID | M | This IE shall uniquely identify the Header Handling Reporting Control Info configured for the Header Handling Control Rule. | - | - | - | X | - | Reporting Endpoint ID |
| Minimum Wait Time | C | This IE shall be present if available. When present, it shall indicate the minimum waiting time between two consecutive reports after which the UP function may send a new notification request message to report the handling of payload headers information event. | - | - | - | X | - | Minimum Wait Time |
| Reporting Suggestion Info | C | This IE should be present if the event report can be delayed, i.e. it is delay tolerant. | - | - | - | X | - | Reporting Suggestion Info |
| Event Notification URI | M | This IE shall contain the notification URI to be used for sending the UPF event notifications. | - | - | - | X | - | Event Notification URI |
| Notification Correlation ID | M | This IE shall indicate the notification correlation id to be included in the UPF event notifications. It shall be present if it is available. | - | - | - | X | - | Notification Correlation ID |

Table 7.5.2.3-9: Header Handling Control information IE in Header Handling Control Rule

| Octet 1 and 2 |  | Header Handling Control information IE = 363 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Header Handling Control ID | M | This IE shall identify an instance of Header Handling Control information. | - | - | - | X | - | Header Handling Control ID |
| Header Handling Control Reference | C | This IE shall be present if available. When present, it shall indicate a reference to a header handling action related to information pre-configured in the UPF. | - | - | - | X | - | Header Handling Control Reference |
| Header Handling Action | C | This IE shall be present if the Header Handling Control Reference IE is not present. It may be present otherwise. When present, it shall contain the action to be performed on a specific header field. | - | - | - | X | - | Header Handling Action |
| Header Information | C | This IE shall be present if the Header Handling Control Reference IE is not present. It may be present otherwise. When present, it shall identify a reference to a UPF configuration which defines how to identify or build a specific header field which to perform the header handling action. | - | - | - | X | - | Header Information |
| Header Value | C | This IE shall be present if the Header Handling Control Reference IE is not present. It may be present otherwise. When present, it shall contain the value of the specific header field. | - | - | - | X | - | Header Value |
| Header Handling Condition | C | This IE shall be present if the Header Handling Control Reference IE is not present. It may be present otherwise. When present, it shall represent the condition about how to apply the header handling action. | - | - | - | X | - | Header Handling Condition |
| Header Handling Reporting | C | This IE shall be present if Header Handling Reporting is required. When present, it shall reference a Reporting Endpoint to which the notifications shall be sent. Several IEs with the same IE type may be present to represent multiple Reporting Endpoints. | - | - | - | X | - | Reporting Endpoint ID |
| NOTE 1: If Header Handling Control Reference IE is present, any other attribute included within Header Handling Control information IE shall overwrite the related information pre-configured in the UPF. |  |  |  |  |  |  |  |  |

# *** End of Change ***