3GPP TSG-CT WG4 Meeting #127	C4-250483

# Athens, GREECE; 17th – 21st February 2025									   Revision of C4-250254

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 2118 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slicing and Interworking with EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2025-01-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 23.501 #5866 (approved at SA#106) enables a UE that supports selecting S-NSSAI based on URSP rules while in EPS and that is provisioned with URSP rules to select an S-NSSAI for the APN based on evaluation of the Traffic descriptor and to signal this S-NSSAI in PCO to the PGW. In this case, clause 5.15.7.13 of TS 23.501 specifies: If the PCO received by the SMF+PGW-C contains an S-NSSAI that is not supported by the SMF+PGW-C, then the SMF+PGW-C may determine another SMF+PGW-C that supports the DNN and the S-NSSAI received in the PCO, based on operator policies, and follows the PGW selection in case of PGW mismatch with network slice subscribed by the UE as specified in clause 7.2.1 and clause 7.2.2 of TS 29.274 [101]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing procedures defined in clauses 7.2.1 and 7.2.2 can be used by a PGW to forward or redirect a Create Session Request towards a different PGW supporting the S-NSSAI indicated by the UE in PCO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. Mismatch in S-NSSAI of an established PDN connection and S-NSSAI defined in URSP rules. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: corrects the clause number referenced in the Reason for change |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.2.1	Create Session Request

The direction of this message shall be from MME/S4-SGSN to SGW and from SGW to PGW, and from ePDG/TWAN to the PGW (see Table 6.1-1).

The Create Session Request message shall be sent on the S11 interface by the MME to the SGW, and on the S5/S8 interface by the SGW to the PGW as part of the procedures:

-	E-UTRAN Initial Attach when a PDN connection needs to be established through the SGW and PGW

-	Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN with GTP on S5/S8 interface (see clauses 8.2, 8.6 and 16.11 of 3GPP TS 23.402 [45])

-	UE requested PDN connectivity when a PDN connection needs to be established through the SGW and PGW

-	Addition of a 3GPP access of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

-	Restoration of PDN connections after an PGW-C/SMF change as specified in clause 31 of 3GPP TS 23.007 [17]

The message shall also be sent on S4 interface by the SGSN to the SGW, and on the S5/S8 interface by the SGW to the PGW as part of the procedures:

-	PDP Context Activation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN with GTP on S5/S8 interface (see clauses 8.2, 8.6 and 16.11 of 3GPP TS 23.402 [45])

-	Addition of a 3GPP access of the NBIFOM procedure as specified by 3GPP TS 23.161 [71]

The message shall also be sent on the S11 interface by the MME to the SGW as part of the procedures:

-	Tracking Area Update procedure with Serving GW change

-	S1/X2-based handover with SGW change

-	UTRAN Iu mode to E-UTRAN Inter RAT handover with SGW change

-	GERAN A/Gb mode to E-UTRAN Inter RAT handover with SGW change

-	3G Gn/Gp SGSN to MME combined hard handover and SRNS relocation procedure

-	Gn/Gp SGSN to MME Tracking Area Update procedure

-	Restoration of PDN connections after an SGW failure if the MME and PGW support these procedures as specified in 3GPP TS 23.007 [17]

-	MME triggered Serving GW relocation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to E-UTRAN with PMIP on S5/S8 interface (see clauses 8.2 and 16.11 of 3GPP TS 23.402 [45])

and on the S4 interface by the SGSN to the SGW as part of the procedures:

-	Routing Area Update with MME interaction and with SGW change

-	Gn/Gp SGSN to S4 SGSN Routing Area Update

-	Inter SGSN Routeing Area Update Procedure and Combined Inter SGSN RA / LA Update using S4 with SGW change

-	Iu mode RA Update Procedure using S4 with SGW change

-	E-UTRAN to UTRAN Iu mode Inter RAT handover with SGW change

-	E-UTRAN to GERAN A/Gb mode Inter RAT handover with SGW change

-	Serving RNS relocation using S4 with SGW change

-	Combined hard handover and SRNS relocation using S4 with SGW change

-	Combined Cell / URA update and SRNS relocation using S4 with SGW change

-	Enhanced serving RNS relocation with SGW relocation

-	Restoration of PDN connections after an SGW failure if the SGSN and PGW support these procedures as specified in 3GPP TS 23.007 [17]

-	S4-SGSN triggered Serving GW relocation

-	Handover from Trusted or Untrusted Non-3GPP IP Access to UTRAN/GERAN with PMIP on S5/S8 interface (see clauses 8.2 and 16.11 of 3GPP TS 23.402 [45])

and on the S2b interface by the ePDG to the PGW as part of the procedures:

-	Initial Attach with GTP on S2b

-	UE initiated Connectivity to Additional PDN with GTP on S2b

-	Handover to Untrusted Non-3GPP IP Access with GTP on S2b (See clause 8.6 of 3GPP TS 23.402 [45])

-	Initial Attach for emergency session (GTP on S2b)

-	Addition of an access using S2b of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

and on the S2a interface by the TWAN to the PGW as part of the procedure:

-	Initial Attach in WLAN on GTP S2a

-	Initial Attach in WLAN for Emergency Service on GTP S2a

-	UE initiated Connectivity to Additional PDN with GTP on S2a

-	Handover to TWAN with GTP on S2a (See clause 16.10 of 3GPP TS 23.402 [45])

-	Addition of an access using S2b of NBIFOM procedure as specified by 3GPP TS 23.161 [71]

-	PDN connection establishment over untrusted non-3GPP access as user-plane resource associated with a MA PDU session (see clause 4.22.2.4.2 of 3GPP TS 23.502 [83])

If the new Create Session Request received by the SGW collides with an existing active PDN connection context (the existing PDN connection context is identified with the tuple [IMSI, EPS Bearer ID], where IMSI shall be replaced by TAC and SNR part of ME Identity for emergency or RLOS attached UE without UICC or authenticated IMSI), this Create Session Request shall be treated as a request for a new session. Before creating the new session, the SGW should delete:

-	the existing PDN connection context locally, if the Create Session Request is received with the TEID set to zero in the header, or if it is received with a TEID not set to zero in the header and it collides with the default bearer of an existing PDN connection context;

-	the existing dedicated bearer context locally, if the Create Session Request collides with an existing dedicated bearer context and the message is received with a TEID not set to zero in the header.

In the former case, if the PGW S5/S8 IP address for control plane received in the new Create Session Request is different from the PGW S5/S8 IP address for control plane of the existing PDN connection, the SGW should also delete the existing PDN connection in the corresponding PGW by sending a Delete Session Request message.

NOTE 1:	The SGW can send the Create Session Request and Delete Session Request over S5/S8 asynchronously, e.g. the SGW can send the Delete Session Request and then the Create Session Request without having to wait for the Delete Session Response. It does not matter if the PGW happens to receive the Delete Session Request after the Create Session Request since the PGW assigns a different S5/S8 F-TEID for control plane to the new PDN connection.

In some network deployment, e.g. when 5G Network Slice is deployed and the combined PGW-C/SMFs are connected to the UDM, the PGW may select another PGW supporting the network slice for which the UE has subscription or supporting the S-NSSAI received in the PCO (see clause 5.15.7.13 of 3GPP TS 23.501 [82]) and then forward the Create Session Request to that PGW. When forwarding the Create Session Request message, the PGW shall forward the Create Session Request message as received from the SGW but with the following modifications:

-	the destination IP address of the message shall be set to the selected PGW IP address;

-	the CSRMFI flag shall be set to "1";

-	the source IP address and UDP port of the message shall be set to the IP address and port of the forwarding PGW.

NOTE 2:	The Create Session Response message is sent back to the forwarding PGW that forwards it to the SGW. It is assumed that GTPv2/UDP/IP connectivity between the source PGW/SMF and the target PGW/SMF which are in different slices is allowed in such network deployment; otherwise, if there is no GTPv2/UDP/IP connectivity between the source PGW/SMF and the target PGW/SMF, or if the source PGW/SMF does not support forwarding the request to the target PGW/SMF, the source PGW/SMF can proceed as specified in clause 7.2.2.

If the new Create Session Request received by the PGW collides with an existing PDN connection context (the existing PDN connection context is identified with the triplet [IMSI, EPS Bearer ID, Interface type], where applicable Interface type here is S2a TWAN GTP-C interface or S2b ePDG GTP-C interface or S5/S8 SGW GTP-C interface, and where IMSI shall be replaced by TAC and SNR part of ME Identity for emergency or RLOS attached UE without UICC or authenticated IMSI), this Create Session Request shall be treated as a request for a new session. Before creating the new session, the PGW should delete:

-	the existing PDN connection context, if the Create Session Request collides with the default bearer of an existing PDN connection context;

-	the existing dedicated bearer context, if the Create Session Request collides with a dedicated bearer of an existing PDN connection context.

*** text skipped for clarity ***

* * * Next Change * * * *

### 7.2.2	Create Session Response

The Create Session Response message shall be sent on the S11/S4 interfaces by the SGW to the MME/S4-SGSN, on the S5/S8 interfaces by the PGW to the SGW, on the S2b interface by the PGW to the ePDG, and on the S2a interface by the PGW to the TWAN as part of the procedures listed for the Create Session Request (see clause 7.2.1).

A PGW may receive the Create Session Response message sent from another PGW (see clause 7.2.1), the PGW shall forward the Create Session response message to the SGW as received from another PGW but with the following modifications:

-	the destination IP address and UDP port of the message shall be set to the source IP address and UDP port of the Create Session Request message received from the SGW;

-	the source IP address and UDP port of the message shall be set to the IP address and port of the forwarding PGW.

In some network deployment, e.g. when 5G Network Slice is deployed and the combined PGW-C/SMFs are connected to the UDM, if the MME indicated in the Create Session Request that it supports PGW redirection due to mismatch with network slice subscribed by UE, the source PGW/SMF may select another PGW/SMF supporting the network slice for which the UE has a subscription or supporting the S-NSSAI received in the PCO (see clause 5.15.7.13 of 3GPP TS 23.501 [82]) and then reject the Create Session Request with the cause set to "PGW mismatch with network slice subscribed by the UE" and with the FQDN or IP address of the other PGW that the MME should use for establishing the PDN connection.

NOTE:	This can be used e.g. if there is no GTPv2/UDP/IP connectivity between PGW/SMFs pertaining to different network slices, or if the source PGW/SMF does not support forwarding the request to the target PGW/SMF as specified in clause 7.2.1.

*** text skipped for clarity ***

* * * End of Changes * * * *