3GPP TSG-CT WG4 Meeting #127	C4-250251

# Athens, GREECE; 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1141 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of unsupported Access Token Request parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 29.510 requires the NRF to ignore unsupported query parameters received in an NF Discovery request: "When certain query parameters in the discovery request are not supported by the NRF, the NRF shall ignore the unsupported query parameters and continue processing the request with the rest of the query parameters." However it does not specify how the NRF should handle unsupported parameters received in an Access Token Request. An NRF's consumer may include parameters that are not supported by the NRF (in both intra and inter-PLMN access token requests), e.g. if the NRF's consumer has implemented a more recent version of the 3GPP specification than the NRF, or if the NRF has selectively implemented some but not all parameters. In this case, the question arises on whether the NRF shall ignore the unsupported parameters (with the consequence of possibly granting an access token with a wider permission than what was actually requested by the NRF's consumer), or if the NRF should reject the request. TS 29.500 specifies in clause 6.6 (Extensibility Mechanisms): "Unknown attributes and values shall be ignored by the receiving entity. Unsupported query parameters shall be handled as specified in clause 5.2.9." RFC 6749 (OAuth 2.0 Authorization Framework) specifies in clause 3.2: "The authorization server MUST ignore unrecognized request parameters." Rejecting an access token request due to unsupported request parameters would cause backward & forward compatibility issues (note that for Inter-PLMN Access Token Requests, the operator of the authorization server in the target PLMN does not have any control on the request parameters that the requester's PLMN may include) and big operational burden (to ensure that NRF's consumers do never include parameters that are not supported by the NRF). It is understood that if the NRF does not implement certain request parameters, corresponding functionalities are not (desired to be) used by the operator in the NRF's PLMN. In other words, the operator shall upgrade its NRF (Nnrf_AccessToken Service) first to support the Access Token Request parameters corresponding to functionalities used by the operator’s PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the NRF shall ignore unsupported parameters received in an Access Token Request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification risking to cause interoperability problems and failures to authorize service requests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.4.2.2	Get (Access Token Request)

##### 5.4.2.2.1	General

This service operation is used by an NF Service Consumer to request an OAuth 2.0 access token from the authorization server (NRF).

Figure 5.4.2.2.1-1: Access Token Request

1.	The NF Service Consumer shall send a POST request to the "Token Endpoint", as described in IETF RFC 6749 [16], clause 3.2. The "Token Endpoint" URI shall be:

{nrfApiRoot}/oauth2/token

where {nrfApiRoot} represents the concatenation of the "scheme" and "authority" components of the NRF, as defined in IETF RFC 3986 [17].

The OAuth 2.0 Access Token Request included in the body of the HTTP POST request shall contain:

-	An OAuth 2.0 grant type set to "client_credentials";

-	The "scope" parameter indicating the names of the NF Services that the NF Service Consumer is trying to access (i.e., the expected NF service names);

-	The NF Instance Id of the the NF Service Consumer requesting the OAuth 2.0 access token;

-	NF type of the NF Service Consumer, if this is an access token request not for a specific NF Service Producer;

-	NF type of the expected NF Service Producer, if this is an access token request not for a specific NF Service Producer;

-	The NF Instance Id of the expected NF Service Producer, if this is an access token request for a specific NF Service Producer;

-	Home and Serving PLMN IDs, if this is an access token request for use in roaming scenarios (see clause 13.4.1.2 of 3GPP TS 33.501 [15]).

The request may additionally contain:

-	the NF Set ID of the expected NF service producer instances, if this is an access token request not for a specific NF Service Producer.

-	the NF Instance Id of the source NF (the NF that requests data), if this is an access token request from the DCCF as NF Service Consumer request data from NF Service Producers on behalf of the source NF.

The NF Service Consumer shall use TLS for mutual authentication with the NRF in order to access this endpoint, if the PLMN uses protection at the transport layer. Otherwise, the NF Service Consumer shall use NDS or physical security to mutually authenticate with the NRF as specified in clause 13.3.1 of 3GPP TS 33.501 [15].

When certain parameters in the Access Token Request are not supported by the NRF, the NRF shall ignore the unsupported parameters and continue processing the request with the rest of the parameters.

The NRF may verify that the input attributes (e.g. NF type) in the access token request match with the corresponding ones in the public key certificate of the NF service consumer. If the verification is successful, other authorization check shall be performed, otherwise, the request shall be rejected immediately with "400 Bad Request" status code, and "error" attribute set to "invalid_client".

2a.	On success, "200 OK" shall be returned, the content of the POST response shall contain the requested access token and the token type set to value "Bearer". The response in addition:

-	should contain the expiration time for the token as indicated in IETF RFC 6749 [16] unless the expiration time of the token is made available by other means (e.g. deployment-specific documentation); and

-	shall contain the NF service name(s) of the requested NF service producer(s), if it is different from the scope included in the access token request (see IETF RFC 6749 [16]).

The access token shall be a JSON Web Token (JWT) as specified in IETF RFC 7519 [25]. The access token returned by the NRF shall include the claims encoded as a JSON object as specified in clause 6.3.5.2.4 and then digitally signed using JWS as specified in IETF RFC 7515 [24] and in clause 13.4.1 of 3GPP TS 33.501 [15].

The digitally signed access token shall be converted to the JWS Compact Serialization encoding as a string as specified in clause 7.1 of IETF RFC 7515 [24].

2b.	On failure or redirection:

-	If the access token request fails at the NRF, the NRF shall return "400 Bad Request" status code, including in the response content a JSON object that provides details about the specific error that occurred.

-	In the case of redirection, the NRF shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NRF service instance.

-	If based on operator policy the required information used to authorize the access token request, e.g. requesterSnssaiList, is not included, the NRF may return "403 Forbidden" status code with the ProblemDetails IE indicating that the missing information shall be provided.

* * * End of Changes * * * *