3GPP TSG-CT6 Meeting #121	C6-250098

# Athens, Greece, 18th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.111 | CR | 0845 | rev | 2 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify TR handling when both mapping and not mapping S-NSSAI data objects are unavailable - Rel 18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple AB Denmark |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, the clause in 3GPP TS 31.111 6.8.7 , specifies how to handle (i) PLI - served + allowed slice information (with/without mapping) (ii) PLI - rejected slice information (with/without mapping) ONLY when either the mapping data object TLV or not-mapping data object TLV is present. But it doesn’t specify what UE must provide when both the mapping and not mapping data objects are not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.8.7 is updated to specify how to handle such a sceanrio. - If both the (Allowed) S-NSSAI with mapping and without mapping data objects are unavailable (i.e., have a length of zero), then the S-NSSAI without mapping data object TLV with a length of zero shall be included. - If both the rejected slices S-NSSAI with mapping and without mapping data objects are unavailable (i.e., have a length of zero), then the rejected S-NSSAI without mapping data object TLV with a length of zero shall be included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to different UE implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | was C6-250089 was C6-250031 |  |  |  |  |  |  |  |  |

### ####first change####

### 6.8.7	Local information

For Local Information values defined in clause 8.6 then ETSI TS 102 223 [32] clause 6.8.7 applies, with the addition of the following procedures:

-	Where the UICC has requested the Network Measurement Results, the TERMINAL RESPONSE shall contain

-	for GERAN: The NMR data object and the BCCH channel list data object

-	for UTRAN: The Network Measurement Results are coded as the MEASUREMENT REPORT message as defined in TS 25.331 [38].

-	for E-UTRAN and Satellite E-UTRAN: The Network Measurement Results are coded as the MeasurementReport message defined in TS 36.331 [49]

-	for NG-RAN or Satellite NG-RAN: The Network Measurement Results are coded as the MeasurementReport message defined in TS 38.331 [71]

-	Where the UICC has requested the Network Measurement Results for multiple access technologies, TERMINAL RESPONSE shall contain the Access Technology data object listing all current access technologies, followed by one NMR data object and one BCCH channel list data object for each current access technology in the same sequence. The BCCH channel list data object shall immediately follow the NMR data object, even if not supported by a network access technology. If no NMR data or no BCCH channel list is available for an access technology, the respective data object shall have length zero.

-	Where the UICC has requested the Timing Advance, the TERMINAL RESPONSE shall contain the Timing Advance data object if supported by the network access technology.

-	Where the UICC has requested the WLAN Specific Identifier, the TERMINAL RESPONSE shall contain the WSID of the current I-WLAN connection.

-	Where the UICC has requested the WLAN Identifier, the TERMINAL RESPONSE shall contain the SSID, the BSSID when available, and the HESSID when available, of the current WLAN connection.

-	Where the UICC has requested the CSG ID list Identifier, the TERMINAL RESPONSE shall contain the CSG ID list and the corresponding HNB name (if available in the broadcasted information to the ME) of the detected CSG or Hybrid cells in the Allowed CSG list or the Operator CSG list. (if class "q" is supported)

-	Where the UICC has requested the H(e)NB IP address, the TERMINAL RESPONSE shall contain the list of all IP addresses available on the H(e)NB-network interface, as a sequence of "Other Address" Data Objects in the TERMINAL RESPONSE. (if class "v" is supported)

-	Where the UICC has requested the list of surrounding macrocells, the TERMINAL RESPONSE shall contain, for all supported access technologies, the Access Technology data object listing all current access technologies, followed by one location information data object for each current access technology in the same sequence, up to the limit of the TERMINAL RESPONSE APDU command size. If no location information is available for an access technology, the respective data object shall have length zero. (if class "w" is supported).

-	Where the UICC has requested the list of slice(s) information, the TERMINAL RESPONSE shall contain the Serving PLMN served S-NSSAIs list, along with (i) either the slices information data object listing all slice(s) information without S-NSSAI mapping data object (if S-NSSAI mapping information is not available) or the list of slice(s) information with S-NSSAI mapping data object listing all available mapping information, (ii) if the S-NSSAI is supported only on a subset of Tracking Area(s) of the current Registration Area, the tracking area identity list where the S-NSSAI is supported shall be available. If ME has more data in the buffer to be sent to the UICC and complete data cannot be included within a single response, the TERMINAL RESPONSE shall contain the 'More data' data object. If 'More data' data object is part of this request, then TERMINAL RESPONSE shall contain only the associated data object (e.g., 'partial NSSAI' data object) with remaining S-NSSAI information, up to the limit of the TERMINAL RESPONSE APDU (see Annex V). If no slice information is available, the respective data object shall have length zero. If both the S-NSSAI with mapping and without mapping data objects are unavailable (i.e., have a length of zero), then the S-NSSAI without mapping data object TLV with a length of zero shall be included.

-	Where the UICC has requested the CAG information list, the TERMINAL RESPONSE shall contain the CAG information list and the corresponding CAG Human-readable network name per CAG ID (if available in the broadcasted information to the ME) of the detected CAG cells. (if class "ag" is supported).

-	Where the UICC has requested the list of rejected slice(s), then TERMINAL RESPONSE shall contain (i) either the list of rejected slice(s) information with S-NSSAI mapping information (if S-NSSAI mapping information is available) or rejected slice(s) information without S-NSSAI mapping information (if S-NSSAI mapping information is not available), (ii) if the S-NSSAI is rejected only on a subset of Tracking Area(s) of the current Registration Area, the tracking area identity list where the S-NSSAI is rejected shall be available. If ME has more data in the buffer to be sent to the UICC and complete data cannot be included within a single response, the TERMINAL RESPONSE shall contain the 'More data' data object. If 'More data' data object is part of this request, then TERMINAL RESPONSE shall contain only the associated data object (e.g., 'partial NSSAI' data object) with remaining S-NSSAI information, up to the limit of the TERMINAL RESPONSE APDU (see Annex V). If no rejected slice information is available, the respective data object shall have length zero. If both the rejected slices S-NSSAI with mapping and without mapping data objects are unavailable (i.e., have a length of zero), then the rejected S-NSSAI without mapping data object TLV with a length of zero shall be included.

### ####end of changes####