3GPP TSG-CT3 Meeting #139	C3-250646

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025								revision of C3-250242

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0387 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of update API invoker API list status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, SNAAPP |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per stage-2 requirement the notification of the update result with the reason for partial or complete failure while updating API invoker API list is mandatory.The below cl 8.26.2.2 and 8.26.3 of TS 23.222 are for reference: |  |  |  |  |  |  |  |  |
| Summary of change: |  | This correction proposes to define the failure reason IE to be sent in the OnboardingNotification data type when update result is unsuccessful. Also introduce the Editor’s NOTE to sending failure reason in case the update result is partial failure is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not fulfilement of stage-2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.5.2, 8.4.4.2.7, 8.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

##### 5.5.2.5.2	API Invoker updating its details on CAPIF using Update_API_Invoker_Details service operation

To update the API Invoker's onboarding details at the CCF, the service consumer (e.g., API Invoker) shall send a HTTP PUT request message to the CCF targeting the corresponding "Individual On-boarded API Invoker" resource, with the request body including the APIInvokerEnrolmentDetails data structure as specified in clause 8.4.2.3.3.2. The "apiInvokerId" and "onboardingInformation" attributes shall remain unchanged from the previously provided values.. Otherwise, if the "PatchUpdate" feature defined in clause 8.4.6 is supported, the service consumer (e.g., API Invoker) may send instead an HTTP PATCH request message to the CCF targeting the corresponding "Individual On-boarded API Invoker" resource with the request body including the APIInvokerEnrolmentDetailsPatch data structure as specified in clause 8.4.2.3.3.3.

Upon reception of the above described HTTP PUT or PATCH request message:

1.	if the CCF decides to update the onboarding details of the API Invoker without validation by the CAPIF administrator, then the CCF:

a.	shall determine if the API Invoker is authorized or not;

b.	verify that the "apiInvokerId" and "onboardingInformation" attributes are not changed;

c.	if the API Invoker's request is authorized and the "apiInvokerId" and "onboardingInformation" attributes are unchanged, the CCF shall:

i.	if the request contains an API list:

-	create a list of APIs that the API Invoker is allowed to access; and

-	update the corresponding "Individual On-boarded API Invoker" resource with the updated information in the request and the created API list;

ii.	if the request does not contain an API list, update the corresponding "Individual On-boarded API Invoker" resource with the updated information in the request; and

iii.	return either:

-	an HTTP "200 OK" status code with the response body including the updated representation of the corresponding "Individual On-boarded API Invoker" resource containing the updated API Invoker's onboarding details within the APIInvokerEnrolmentDetails data structure; or

-	an HTTP "204 No Content" status code.

2.	otherwise, the CCF shall:

a.	acknowledge the reception of the request by returning an HTTP "202 Accepted" status code;

b.	verify that the "apiInvokerId" and "onboardingInformation" properties are not changed;

c.	if the "apiInvokerId" and "onboardingInformation" are unchanged, then request the CAPIF administrator or the API management to validate the request by sharing the API Invoker's identity information and the updated information received in the HTTP PUT/PATCH request message;

d.	upon reception of the confirmation of successful validation of the request from the CAPIF administrator or the API management:

i.	update the corresponding "Individual On-boarded API Invoker" resource with the validated information; and

ii.	send to the API Invoker a notification, using the procedure defined in clause 5.5.2.4, including the updated "Individual On-boarded API Invoker" resource representation containing the updated API Invoker's onboarding details and additional related information, and if the EnhancedUpdateStatus feature is supported, the failure reason in case the update operation is unsuccessful within the OnboardingNotification data structure;

and

3.	if errors occur when processing the request, the CCF shall respond with an appropriate error status code as defined in clause 8.4.5.

NOTE 1:	How the CCF determines that the CCF can process the request and update the API list of the API Invoker automatically is out-of-scope of this specification.

NOTE 2:	Interactions between the CCF and the CAPIF administrator or the API management is out-of-scope of this specification.

NOTE 3:	After the operation is completed the API Invoker no longer needs to maintain the Notification Destination URI and may delete it.

# * * * * 2nd Change * * * *

##### 8.4.4.2.7	Type: OnboardingNotification

Table 8.4.4.2.7-1: Definition of type OnboardingNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | boolean | M | 1 | Contains the onboarding result. - Set to "true" to indicate successful on-boarding. - Set to "false" to indicate unsuccessful onboarding. - The default value is "false" if omitted. |  |
| resourceLocation | Uri | C | 0..1 | Contains the URI pointing to the new CAPIF resource created as a result of successful on-boarding, according to the structure: {apiRoot}/api-invoker-management/<apiVersion>/onboardedInvokers/{onboardingId} This attribute shall be present only if the "result" attribute is set to "true". Otherwise it shall not be present. |  |
| apiInvokerEnrolmentDetails | APIInvokerEnrolmentDetails | C | 0..1 | Contains the enrolment details of the API Invoker which are verified by the CAPIF administrator or API management. This attribute shall be present only if the "result" attribute is set to "true". Otherwise it shall not be present. |  |
| apiList | APIList | O | 0..1 | Contains the list of APIs that the API Invoker is allowed to access. This attribute may be present only if the "result" attribute is set to "true". Otherwise it shall not be present. |  |
| failReason | string | C | 0..1 | Contains the failure reason information when the update result is unsuccessful. | EnhancedUpdateStatus |

Editor’s NOTE: Whether sending failure reason in case the update result is partial failure is FFS.

# * * * * 4th Change * * * *

### 8.4.6	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.4.6-1 lists the supported features for CAPIF_API_Invoker_Management_API.

Table 8.4.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Indicates the support of the testing of notification connection i according to clause 7.6. |
| 2 | Notification_websocket | Indicates the support of the delivery of notifications over Websocket according to clause 7.6. This feature requires the support of the "Notification_test_event" feature. |
| 3 | PatchUpdate | Indicates the support of the PATCH method for updating an "Individual On-boarded API Invoker" resource. |
| 4 | ExpirationTime | Indicates the support of expiration time for the API invoker onboarding functionality as part of the support of network slice capability exposure application layer framework. This feature enables the following functionalities: - provisioning/updating/deleting the expiration time of an onboarding. |
| 5 | EnhancedUpdateStatus | Indicates the support of sending the failure reason for unsuccessful update result. |

# * * * * End of Changes * * * *