3GPP TSG CT Meeting #107	CP-250257

# Incheon , KR, 12th – 13th March, 2025							                    was C3-250634

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1472 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Altitude reporting information in the Nnef_UAVFlightAssistance API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, CATT, LG Electronics, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, CATT, LG Electronics, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage-2 (SA2) agreed S2-2501300, S2-2501304 (TS 23.256 CR#0167, CR#0171) clarifies the support of NEF providing Altitude reporting information at the USS, which includes Altitude threshold and periodicity, and USS notifies the altitude along with the time stamp to the NEF during altitude reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Nnef_UAVFlightAssistance API is updated to reflect above clarification from stage-2. Also change includes one EN resolution and minor correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage-3 implementation and stage-2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.43.3, 4.4.43.5, 5.39.2.3.3.1, 5.39.5.1, 5.39.5.2.2, 5.39.5.2.5, 5.39.5.2.7, 5.39.5.2.8, A.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.256 CR 0167, 0171 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces new backward compatible feature to the following API: TS29522_UAVFlightAssistance.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: Added Huawei as co-signing company Rev 3: Correction to bad indentation (2 additional spaces) in the definition of the altReportReqs attribute within the UAVFlightAssistNotif data type in OpenAPI. |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 4.4.43.3	Update of an existing individual UAV Flight Assistance Configuration

In order to update an existing UAV Flight Assistance Configuration and provide UAV altitude reporting, the USS shall send an HTTP PUT request message and targeting the correponding "Individual UAV Flight Assistance Configuration" resource with the request body including the PlannedFlightPaths data structure as defined in clause 5.39.2.3.3.2.

On successful update the subscription, the NEF shall respond to the USS with a "200 OK" status code with the PlannedFlightPaths data structure as response body containing the representation of the modified "Individual UAV Flight Assistance Configuration", or an HTTP "204 No Content" response.

On failure, the NEF shall take proper error handling actions, as specified in clause 5.39.7, and respond to the USS with an appropriate error status code.

# *** Next Changes ***

#### 4.4.43.5	Notification for UAV Flight Assistance information

This procedure is used by the NEF to notify the previously subscribed USS(es) about UAV status changes and provide UAV altitude reporting information.

When the NEF (UAS NF) receives the notification of the UAV Flight Assistance information on UAV status changes, the NEF shall provide a notification by sending an HTTP POST request message to the USS with the request body including the UAVFlightAssistNotif data structure and targeting the notification URI provided by the USS during the corresponding UAV Flight Assistance information subscription as defined in clause 5.39.4.

Upon reception of this notification request, the USS shall acknowledge its successful reception by sending a HTTP POST response message with an HTTP "204 No Content" status code.

On failure, the USS shall take proper error handling actions, as specified in clause 5.39.7, and respond to the NEF with an appropriate error status code.

# *** Next Changes ***

###### 5.39.2.3.3.1	PUT

This method enables an AF to update an existing Individual UAV Flight Assistance Configuration resource at the NEF.

This method shall support the URI query parameters specified in table 5.39.2.3.3.1-1.

Table 5.39.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.39.2.3.3.1-2 and the response data structures and response codes specified in table 5.39.2.3.3.1-3.

Table 5.39.2.3.3.1-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PlannedFlightPaths | M | 1 | Parameters to update the Individual UAV Flight Assistance Configuration resource. |

Table 5.39.2.3.3.1-3: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PlannedFlightPaths | M | 1 | 200 OK | The Individual UAV Flight Assistance Configuration resource was updated successfully and a representation of the created resource is returned in the response body. |
| n/a |  |  | 204 No Content | The Individual UAV Flight Assistance Configuration resource was successfully updated and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.39.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.39.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

# *** Next Changes ***

#### 5.39.5.1	General

This clause specifies the application data model supported by the UAVFlightAssistance API. Table 5.39.5.1-1 specifies the data types defined for the UAVFlightAssistance API.

Table 5.39.5.1-1: UAVFlightAssistance API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AltitudeReport | 5.39.5.2.8 | Represents the UAV altitude report. |  |
| AltRange | 5.39.5.2.9 | Represents the UAV altitude threshold range. |  |
| AltReportEvent | 5.39.5.3.5 | Represents the UAV altitude reporting events. |  |
| AltReportReqs | 5.39.5.2.7 | Represents the UAV altitude reporting requirements. |  |
| FlightPathInfo | 5.39.5.2.4 | Represents the information of each segment that constitutes the single flight path. |  |
| NotifyResponse | 5.39.5.2.6 | Represents the response to the notification. |  |
| PlannedFlightPaths | 5.39.5.2.2 | Represents the planned flight path(s). |  |
| RequestPurpose | 5.39.5.3.4 | Represents the purpose of the request. |  |
| SingleFlightPath | 5.39.5.2.3 | Represents the single flight path. |  |
| UAVFlightAssistNotif | 5.39.5.2.5 | Represents the UAV status notification. |  |
| UavStatus | 5.39.5.3.3 | Represents the status of the UAV. |  |

Table 5.39.5.1-2 specifies data types re-used by the UAVFlightAssistance API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UAVFlightAssistance API.

Table 5.39.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Altitude | 3GPP TS 29.572 [34] | Represents an altitude value. |  |
| DateTime | 3GPP TS 29.122 [4] | Represents a date and a time. |  |
| DurationSec | 3GPP TS 29.122 [4] | Represents a duration in seconds. |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |  |
| LocationArea5G | 3GPP TS 29.122 [4] | Represents location area information. |  |
| TimeWindow | 3GPP TS 29.122 [4] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [8] | Represents an unsigned integer. |  |

# *** Next Changes ***

##### 5.39.5.2.2	Type: PlannedFlightPaths

Table 5.39.5.2.2-1: Definition of type PlannedFlightPaths

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uavId | Gpsi | M | 1 | Identifies the UAV. |  |
| reqPurpose | RequestPurpose | M | 1 | Contains the purpose of the request. |  |
| notifUri | Uri | M | 1 | Notification URI for USS changing reporting. |  |
| notifId | string | O | 0..1 | Indicates the Notification Correlation ID. |  |
| flightPaths | array(SingleFlightPath) | O | 1..N | Indicates the list of flight paths. |  |
| altReports | array(AltitudeReport) | O | 1..N | Indicates the altitude reporting. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features used as defined in clause 5.39.6. This attribute shall be present only when feature negotiation needs to take place. |  |

# *** Next Changes ***

##### 5.39.5.2.5	Type: UAVFlightAssistNotif

Table 5.39.5.2.5-1: Definition of type UAVFlightAssistNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | O | 0..1 | Indicates the Notification Correlation ID. |  |
| uavStatus | UavStatus | M | 1 | Indicates the status of the UAV. |  |
| borderPoint | LocationArea5G | O | 0..1 | Indicates the border-crossing point. |  |
| altReportReqs | AltReportReqs | O | 0..1 | Indicates the Altitude reporting requirements. |  |

# *** Next Changes ***

##### 5.39.5.2.7	Type: AltReportReqs

Table 5.39.5.2.7-1: Definition of type AltReportReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| altThreshold | AltRange | C | 0..1 | Contains the altitude threshold range for threshold based reporting. |  |
| altReportPeriod | DurationSec | C | 0..1 | Contains the periodicity information for USS to provide UAV altitude reporting. (NOTE). |  |
| altReportEvent | AltReportEvent | C | 0..1 | Contains the event-based altitude reporting requirement. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Changes ***

##### 5.39.5.2.8	Type: AltitudeReport

Table 5.39.5.2.8-1: Definition of type AltitudeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| altitude | Altitude | M | 1 | Indicates the altitude of the UAV |  |
| timeStamp | DateTime | M | 1 | Time at which the altitude of the UAV indicated by the "altitude" attribute is measured. |  |

# *** Next Changes ***

##### 5.39.5.2.9	Type: AltRange

Table 5.39.5.2.9-1: Definition of type AltRange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lowerBound | Uinteger | C | 0..1 | Lower bound of the altitude threshold range. |  |
| upperBound | Uinteger | C | 0..1 | Upper bound of the altitude threshold range. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Changes ***

##### 5.39.5.3.5	Enumeration: AltReportEvent

Table 5.39.5.3.5-1: Enumeration AltReportEvent

| Enumeration value | Description |
| --- | --- |
| TA_CHANGE | Indicates the change of UAV's serving TA. |
| RAN_NODE_CHANGE | Indicates the change of UAV's serving RAN node. |

# *** Next Changes ***

# A.37	UAVFlightAssistance API

openapi: 3.0.0

info:

title: UAVFlightAssistance

version: 1.0.0-alpha.1

description: |

API for UAV Flight Assistance Configuration.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.1.0; 5G System; Network Exposure Function Northbound APIs.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/

servers:

- url: '{apiRoot}/3gpp-uav-flight-assistance/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{afId}/configurations:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF

required: true

schema:

type: string

post:

summary: Request the creation of a new UAV Flight Assistance Configuration.

operationId: CreateUavFlightAssistConfig

tags:

- UAV Flight Assistance Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlannedFlightPaths'

responses:

'201':

description: >

Created. The UAV Flight Assistance Configuration is successfully created and a

representation of the created Individual UAV Flight Assistance Configuration resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PlannedFlightPaths'

headers:

Location:

description: >

Contains the URI of the created Individual UAV Flight Assistance Configuration

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UAVFlightAssistNotif'

responses:

'200':

description: The receipt of the notification is acknowledged.

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyResponse'

'204':

description: >

No Content. The notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/configurations/{configurationId}:

parameters:

- name: afId

description: Represents the identifier of the AF.

in: path

required: true

schema:

type: string

- name: configurationId

description: String identifying a UAV Flight Assistance Configuration.

in: path

required: true

schema:

type: string

delete:

summary: Delete an existing Individual UAV Flight Assistance Configuration.

operationId: DeleteUavFlightAssistConfig

tags:

- Individual UAV Flight Assistance Configuration (Document)

responses:

'204':

description: >

No Content. The Individual UAV Flight Assistance Configuration resource matching the

configurationId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual UAV Flight Assistance Configuration.

operationId: UpdateUavFlightAssistConfig

tags:

- Individual UAV Flight Assistance Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PlannedFlightPaths'

responses:

'200':

description: >

The Individual UAV Flight Assistance Configuration resource was modified successfully

and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PlannedFlightPaths'

'204':

description: >

The Individual UAV Flight Assistance Configuration resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PlannedFlightPaths:

description: Represents the planned flight path(s).

type: object

properties:

uavId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

reqPurpose:

$ref: '#/components/schemas/RequestPurpose'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

flightPaths:

type: array

items:

$ref: '#/components/schemas/SingleFlightPath'

minItems: 1

description: Indicates the list of flight paths.

altReports:

type: array

items:

$ref: '#/components/schemas/AltitudeReport'

minItems: 1

description: Indicates the UAV altitude report.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- uavId

- notifUri

- reqPurpose

SingleFlightPath:

description: Contains the single flight path information.

type: object

properties:

pathInfo:

type: array

items:

$ref: '#/components/schemas/FlightPathInfo'

minItems: 1

description: >

Each element contains the flight path information for each segment that constitutes the

single flight path.

required:

- pathInfo

FlightPathInfo:

description: Contains the information of the flight path.

type: object

properties:

areaInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

uavSpeed:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

height:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeSchedule:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- areaInfo

UAVFlightAssistNotif:

description: Notify the UAV status.

type: object

properties:

notifId:

type: string

uavStatus:

$ref: '#/components/schemas/UavStatus'

borderPoint:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

altReportReqs:

$ref: '#/components/schemas/AltReportReqs'

required:

- uavStatus

NotifyResponse:

description: Represents the response to the notification.

type: object

properties:

chgCompleteInd:

type: boolean

description: >

This attribute is used to indicate whether the USS changeover is completed successfully.

When present, it shall be set as follows: true: USS changeover is completed

successfully; false (default): USS changeover failed.

required:

- chgCompleteInd

AltReportReqs:

description: Represents the UAV altitude reporting requirements.

type: object

properties:

altThreshold:

$ref: '#/components/schemas/AltRange'

altReportPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

altReportEvent:

$ref: '#/components/schemas/AltReportEvent'

AltitudeReport:

description: Represents the UAV altitude report.

type: object

properties:

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AltRange:

description: Represents the UAV altitude range for reporting.

type: object

properties:

lowerBound:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

upperBound:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

#

# ENUMERATIONS

#

UavStatus:

anyOf:

- type: string

enum:

- LEAVING

- ENTERING

- DEVIATING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the status of the UAV..

Possible values are:

- LEAVING: The status of the UAV is leaving an area (e.g. TA/Cell) which is part of the

planned flight path.

- ENTERING: The status of the UAV is entering an area (e.g. TA/Cell) which is part of the

planned flight path.

- DEVIATING: The status of the UAV is deviating an area (e.g. TA/Cell) which is part of the

planned flight path.

RequestPurpose:

anyOf:

- type: string

enum:

- USS_CHANGEOVER

- PRE_FLIGHT_PLANNING

- IN_FLIGHT_MONITORING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the filter criterion event.

Possible values are:

- USS_CHANGEOVER: Indicates the purpose of the request is for pre-flight planning.

- PRE_FLIGHT_PLANNING: Indicates the purpose of the request is for USS changeover.

- IN_FLIGHT_MONITORING:Indicates the purpose of the request is for in flight monitoring.

AltReportEvent:

anyOf:

- type: string

enum:

- TA_CHANGE

- RAN_NODE_CHANGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the Altitude reporting events.

Possible values are:

- TA_CHANGE: Indicates the change of UAV’s serving TA.

- RAN_NODE_CHANGE: Indicates the change of UAV’s serving RAN node.

# *** End of Changes ***