3GPP TSG-CT WG1 Meeting #155	C1-253136

# Bratislava, Slovakia, 19-23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.250 | CR | 0027 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference to obsolete IETF RFC4122 and RFC7049 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains two IETF RFCs which are obsolete: (1) RFC4122 is obsoleted by RFC9562. Hence, the obsolete IETF HTTP RFC needs to be updated in this specification. Note that RFC9562 incorporates the five versions of UUIDs defined by RFC4122. Changes between the obsoleted RFC and new RFC9562 are the introduction of three new UUID versions, clarified ambiguities in the earlier specification, and provided updates to ensure better compatibility and security. Those changes do not impact the current text in this specification. TS 24.250 provides support to provide a UUID in the MANAGE_PORT parameters within the "Application Id" used in the MANAGE_PORT command and response frames. The specification would benefit of having the possibility of providing UUID following also any of the three new versions provided by RFC9562; and (2) RFC7049 is obsoleted by RFC8949. Hence, the obsolete IETF HTTP RFC needs to be updated in this specification. Note that RFC8949 is fully compatible with RFC7049. RFC8949 introduces editorial improvements, new details, and errata fixes, but it does not create a new version of the format. Instead, it obsoletes RFC7049 while keeping the interchange format unchanged. The use of RFC8949 does not impact the current text in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections as per the reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would refer to obsolete IETF RFCs, this will not be aligned with IETF and would then be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.2.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[3]	IETF RFC 9562: "Universally Unique IDentifiers (UUIDs).

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	W3C Recommendation:  "Extensible Markup Language (XML) 1.0 (Fifth Edition)", 26 November 2008

[6]	IETF RFC 8529: "The JavaScript Object Notation (JSON) Data Interchange Format".

[7]	IETF RFC 8949: "Concise Binary Object Representation (CBOR)".

* * * Next Change * * * *

##### 5.4.2.6.1	General

The originator and receiver may support the handling specified in subclause 5.4.2.6.

The MANAGE_PORT command and response is used to manage association of applications with source and destination port numbers between originator and receiver and negotiate the serialization format that will be used by the application in both acknowledged and unacknowledged mode of transfer. The MANAGE_PORT command and response can be used to:

-	reserve a combination of source and destination port numbers for use with a specific application. Applications can also additionally reserve the serialization format to be used;

-	release a combination of source and destination port numbers that are reserved;

-	query the list of port numbers that are reserved for use with a specific application. Applications can also additionally query the serialization format; and

-	notify the list of port numbers that are reserved for use with a specific application. The serialization format used by the applications can also be additionally notified.

Port number 0 shall not be reserved at the originator or receiver. If an application at the originator communicates with multiple applications at the receiver, then the application does not reserve a port number at destination and shall set the Destination Port to 0. The ADS bit in the U frame header of MANAGE_PORT command and response is set to 0. Table 5.4.2.6.1-1 lists the parameters used in MANAGE_PORT command and response frames.

Table 5.4.2.6-1: MANAGE_PORT parameters

| Action (Bits 1 to 4, octet 1) This field indicates the operation that the originator or receiver performs as part of MANAGE_PORT command or response and can have the following values |  |
| --- | --- |
| Bits 4 3 2 1 0 0 0 1 Reserve port 0 0 1 0 Release port 0 0 1 1 Query port 0 1 0 0 Notify port 0 1 0 1 Reserve port and serialization format 0 1 1 0 Query port and serialization format 0 1 1 1 Notify port and serialization format All other values are reserved. Application Id This field shall be encoded as a sequence of a sixteen octet OS Id field, a one octet OS App Id length field, and an OS App Id field. The OS Id is the operating system Identifier and it contains a UUID as specified in IETF RFC 9562 [3]. The OS App Id field contains an OS specific application identifier of variable length octets. |  |
| Serialization Format |  |  |
| This field is used to indicate the serialization format used by the application and shall be encoded as a bitmap of one octet as indicated below: |  |  |
| Status This field is used only in the response frame in the direction from the receiver to the originator. It specifies the status of the operation and can have the following values: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Success 0 0 0 0 0 0 0 1 Port not free 0 0 0 0 0 0 1 0 Port not associated with specified application 0 0 0 0 0 0 1 1 Serialization format not supported All other values are reserved. Requested port numbers This field indicates the destination port numbers that the originator wants to query and shall be encoded as a bitmap of two octets as indicated below. Port numbers not available This field indicates the port numbers that are reserved and for which information is not included in the command or response frame. This field shall be encoded as a bitmap of two octets as indicated below. |  |  |

a)	Requested port numbers

The port numbers that are requested are coded in the first and second octet of the Requested port numbers bitmap as follows:

| bit 8 | 7 | 6 | 5 | 4 | 3 | 2 | bit 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 7 | Port 6 | Port 5 | Port 4 | Port 3 | Port 2 | Port 1 | (reserved) | Octet 1 |

| bit 16 | 15 | 14 | 13 | 12 | 11 | 10 | bit 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 15 | Port 14 | Port 13 | Port 12 | Port 11 | Port 10 | Port 9 | Port 8 | Octet 2 |

A port number is requested, if the corresponding bit is set to "1". All reserved bits shall be set to "0".

b)	Port numbers not available

The port numbers that are not available are coded in the first and second octet of the Port numbers not available bitmap as follows:

| bit 8 | 7 | 6 | 5 | 4 | 3 | 2 | bit 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 7 | Port 6 | Port 5 | Port 4 | Port 3 | Port 2 | Port 1 | (reserved) | Octet 1 |

| bit 16 | 15 | 14 | 13 | 12 | 11 | 10 | bit 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port 15 | Port 14 | Port 13 | Port 12 | Port 11 | Port 10 | Port 9 | Port 8 | Octet 2 |

A port number is not available, if the corresponding bit is set to "1". All reserved bits shall be set to "0".

c)	Serialization format

The serialization formats that are supported are coded in the first octet of the Serialization Format bitmap as follows:

| bit 8 | 7 | 6 | 5 | 4 | 3 | 2 | bit 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (reserved) | (reserved) | (reserved) | (reserved) | (reserved) | XML [5] | JSON [6] | CBOR [7] | Octet 1 |

A serialization format is supported, if the corresponding bit is set to "1". All reserved bits shall be set to "0".

* * * End of Changes * * * *