3GPP TSG-CT WG1 Meeting #155	C1-253362

# Bratislava, Slovakia, 19-23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 6907 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LP-WUSPS paging subgroup ID values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core, 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN1 agreed (see Final_Minutes_report_RAN1#120_v100): ---------------- R1-2500794 Summary #4 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) From Thursday session Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32. ---------------- Thus, only 31 LP-WUSPS paging subgroup IDs need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 31 LP-WUSPS paging subgroup IDs (LP-WUSPS paging subgroup ID 0 to LP-WUSPS paging subgroup ID 30) are defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.111 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 9.11.3.111	LP-WUSPS assistance information

The purpose of the LP-WUSPS assistance information IE is to transfer the required assistance information for LP-WUSPS.

The coding of the information element allows combining different types of the LP-WUSPS assistance information.

The LP-WUSPS assistance information, information element is coded as shown in figure 9.11.3.111.1, figure 9.11.3.111.2, figure 9.11.3.111.3 and table 9.11.3.111.1.

The LP-WUSPS assistance information is a type 4 information element, with a minimum length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUSPS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LP-WUSPS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| LP-WUSPS assistance information type 1 |  |  |  |  |  |  |  | octet 3* octet i* |
| LP-WUSPS assistance information type 2 |  |  |  |  |  |  |  | octet (i+1)* octet l* |
| … |  |  |  |  |  |  |  | octet (l+1)* octet m* |
| LP-WUSPS assistance information type p |  |  |  |  |  |  |  | octet (m+1)* octet n* |

Figure 9.11.3.111.1: LP-WUSPS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | LP-WUSPS paging subgroup ID value |  |  |  |  | octet 1 |

Figure 9.11.3.111.2: LP-WUSPS assistance information type – type of information= "000"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 1 |

Figure 9.11.3.111.3: LP-WUSPS assistance information type – type of information= "001"

Table 9.11.3.111.1: LP-WUSPS assistance information information element

| Value part of the LP-WUSPS assistance information information element (octets 3 to n) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| The value part of the LP-WUSPS assistance information information element consists of one or several types of the LP-WUSPS assistance information. |  |  |  |  |  |  |
| LP-WUSPS assistance information type: |  |  |  |  |  |  |
| Type of information (octet 1, bits 6 to 8) |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |  |
| 0 | 0 | 0 | LP-WUSPS paging subgroup ID |  |  |  |
| 0 | 0 | 1 | UE paging probability information |  |  |  |
| All other values are reserved. |  |  |  |  |  |  |
| LP-WUSPS paging subgroup ID value: (octet 1, bits 1 to 5) |  |  |  |  |  |  |
| This field contains one out of 31 (decimal) LP-WUSPS paging subgroup IDs, that is assigned by the AMF for paging the UE. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  | LP-WUSPS paging subgroup ID value |
| 0 | 0 | 0 | 0 | 0 |  | LP-WUSPS paging subgroup ID 0 (decimal) |
|  |  | to |  |  |  |  |
| 1 | 1 | 1 | 1 | 0 |  | LP-WUSPS paging subgroup ID 30 (decimal) |
| 1 | 1 | 1 | 1 | 1 |  | reserved |
| UE paging probability information value: (octet 1, bits 1 to 5) |  |  |  |  |  |  |
| This field contains the value of UE paging probability information provided by the UE to the AMF. It represents the probability of the UE receiving the paging. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  | UE paging probability information value |
| 0 | 0 | 0 | 0 | 0 |  | p00 (UE calculated paging probability is 0%) |
| 0 | 0 | 0 | 0 | 1 |  | p05 (UE calculated paging probability > 0% and <= 5%) |
| 0 | 0 | 0 | 1 | 0 |  | p10 (UE calculated paging probability > 5% and <= 10%) |
| 0 | 0 | 0 | 1 | 1 |  | p15 (UE calculated paging probability > 10% and <= 15%) |
| 0 | 0 | 1 | 0 | 0 |  | p20 (UE calculated paging probability > 15% and <= 20%) |
| 0 | 0 | 1 | 0 | 1 |  | p25 (UE calculated paging probability > 20% and <= 25%) |
| 0 | 0 | 1 | 1 | 0 |  | p30 (UE calculated paging probability > 25% and <= 30%) |
| 0 | 0 | 1 | 1 | 1 |  | p35 (UE calculated paging probability > 30% and <= 35%) |
| 0 | 1 | 0 | 0 | 0 |  | p40 (UE calculated paging probability > 35% and <= 40%) |
| 0 | 1 | 0 | 0 | 1 |  | p45 (UE calculated paging probability > 40% and <= 45%) |
| 0 | 1 | 0 | 1 | 0 |  | p50 (UE calculated paging probability > 45% and <= 50%) |
| 0 | 1 | 0 | 1 | 1 |  | p55 (UE calculated paging probability > 50% and <= 55%) |
| 0 | 1 | 1 | 0 | 0 |  | p60 (UE calculated paging probability > 55% and <= 60%) |
| 0 | 1 | 1 | 0 | 1 |  | p65 (UE calculated paging probability > 60% and <= 65%) |
| 0 | 1 | 1 | 1 | 0 |  | p70 (UE calculated paging probability > 65% and <= 70%) |
| 0 | 1 | 1 | 1 | 1 |  | p75 (UE calculated paging probability > 70% and <= 75%) |
| 1 | 0 | 0 | 0 | 0 |  | p80 (UE calculated paging probability > 75% and <= 80%) |
| 1 | 0 | 0 | 0 | 1 |  | p85 (UE calculated paging probability > 80% and <= 85%) |
| 1 | 0 | 0 | 1 | 0 |  | p90 (UE calculated paging probability > 85% and <= 90%) |
| 1 | 0 | 0 | 1 | 1 |  | p95 (UE calculated paging probability > 90% and <= 95%) |
| 1 | 0 | 1 | 0 | 0 |  | p100 (UE calculated paging probability > 95% and <= 100%) |
| All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |  |