3GPP TSG-CT WG1 Meeting #155	C1-253238

Bratislava, SK, 19-23 May 2025	was C1-25xxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4377 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the WUS assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAES19 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In WUS assistance information IE coding, the Type of information field is in bits 6 - 8 of octet 3. And the UE paging probability information value field is in bits 1 - 5 of octet 3. Additionally, the editorial fix in 5.3.21 is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Correction to the octet and bits of Type of information field, and add the bits for UE paging probability information value field; 2) Editorial fix in 5.3.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong octet and bits for Type of information field remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.21, 9.9.3.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.3.21	Wake-up signal assistance

A UE supporting wake-up signal (WUS) assistance can indicate its WUS assistance capability during attach or tracking area updating procedure (see 3GPP TS 23.401 [10]). The UE supporting WUS assistance may include its UE paging probability information in the Requested WUS assistance information IE during an attach or tracking area updating procedure (see 3GPP TS 23.401 [10]). The UE shall not include its UE paging probability information during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The UE and the network may negotiate the UE paging probability information during an attach or tracking area updating procedure when the UE is not attached for emergency bearer services. The UE paging probability information is an assistance information used to determine the WUS group for paging UE (see 3GPP TS 23.401 [10], 3GPP TS 36.300 [20]).

NOTE:	The determination of UE paging probability information is up to UE implementation.

If the UE is not attached for emergency bearer services and the network accepts the use of the WUS assistance for the UE, the network determines the negotiated UE paging probability information for the UE based on the requested UE paging probability information, if any, local configuration or previous statistical information for the UE, and then indicates the negotiated UE paging probability information in the Negotiated WUS assistance information IE to the UE when accepting the attach or the tracking area updating procedure. The network shall store the negotiated UE paging probability information in the EMM context of the UE for paging.

The UE shall use WUS assistance only if it received the Negotiated WUS assistance information IE during the last attach or tracking area updating procedure. If the UE did not receive the Negotiated WUS assistance information IE during the last attach or tracking area updating procedure, the UE shall delete any existing WUS assistance information received from the network.

If the network did not accept the request to use WUS assistance, the network shall delete the stored negotiated UE paging probability information for the UE, if available.

When a PDN connection for emergency bearer service is successfully established after the UE received the Negotiated WUS assistance information IE during the last attach or tracking area updating procedure, the UE and the network shall not use WUS assistance information until:

-	the successful completion of the PDN disconnect procedure of the PDN connection for emergency bearer services or EPS bearer context deactivation procedure of the EPS bearer context for emergency, or

-	the UE receives WUS assistance information during a tracking area updating procedure with EPS bearer context synchronization or upon successful completion of a service request procedure, if the UE or the network locally releases the PDN connection for emergency bearer service.

* * * Next Change * * * *

#### 9.9.3.62	WUS assistance information

The purpose of the WUS assistance information information element is to transfer the required assistance information to determine the WUS group used when paging the UE.

The coding of the information element allows combining different types of WUS assistance information.

The WUS assistance information information element is coded as shown in figure 9.9.3.62.1, figure 9.9.3.62.2 and table 9.9.3.62.1.

The WUS assistance information is a type 4 information element, with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WUS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of WUS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| WUS assistance information type 1 |  |  |  |  |  |  |  | octet 3 octet i |
| WUS assistance information type 2 |  |  |  |  |  |  |  | octet i+1* octet l* |
| … |  |  |  |  |  |  |  | octet l+1* octet m* |
| WUS assistance information type p |  |  |  |  |  |  |  | octet m+1* octet n* |

Figure 9.9.3.62.1: WUS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 3 |

Figure 9.9.3.62.2: WUS assistance information type –type of information= "000"

Table 9.9.3.62.1: WUS assistance information information element

| Value part of the WUS assistance information information element (octets 3 to n) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the WUS assistance information information element consists of one or several types of WUS assistance information. |  |  |  |  |  |
| WUS assistance information type: |  |  |  |  |  |
| Type of information (octet 3, bits 6 to 8) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |
| 0 | 0 | 0 | UE paging probability information |  |  |
| All other values are reserved. |  |  |  |  |  |
| UE paging probability information value (octet 3, bits 1 to 5) |  |  |  |  |  |
| This field contains the value of UE paging probability information requested by the UE or negotiated by the network. It represents the probability of the UE receiving the paging. bit |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 | UE paging probability information value |
| 0 | 0 | 0 | 0 | 0 | p00 (UE calculated paging probability is 0%) |
| 0 | 0 | 0 | 0 | 1 | p05 (UE calculated paging probability > 0% and <= 5%) |
| 0 | 0 | 0 | 1 | 0 | p10 (UE calculated paging probability > 5% and <= 10%) |
| 0 | 0 | 0 | 1 | 1 | p15 (UE calculated paging probability > 10% and <= 15%) |
| 0 | 0 | 1 | 0 | 0 | p20 (UE calculated paging probability > 15% and <= 20%) |
| 0 | 0 | 1 | 0 | 1 | p25 (UE calculated paging probability > 20% and <= 25%) |
| 0 | 0 | 1 | 1 | 0 | p30 (UE calculated paging probability > 25% and <= 30%) |
| 0 | 0 | 1 | 1 | 1 | p35 (UE calculated paging probability > 30% and <= 35%) |
| 0 | 1 | 0 | 0 | 0 | p40 (UE calculated paging probability > 35% and <= 40%) |
| 0 | 1 | 0 | 0 | 1 | p45 (UE calculated paging probability > 40% and <= 45%) |
| 0 | 1 | 0 | 1 | 0 | p50 (UE calculated paging probability > 45% and <= 50%) |
| 0 | 1 | 0 | 1 | 1 | p55 (UE calculated paging probability > 50% and <= 55%) |
| 0 | 1 | 1 | 0 | 0 | p60 (UE calculated paging probability > 55% and <= 60%) |
| 0 | 1 | 1 | 0 | 1 | p65 (UE calculated paging probability > 60% and <= 65%) |
| 0 | 1 | 1 | 1 | 0 | p70 (UE calculated paging probability > 65% and <= 70%) |
| 0 | 1 | 1 | 1 | 1 | p75 (UE calculated paging probability > 70% and <= 75%) |
| 1 | 0 | 0 | 0 | 0 | p80 (UE calculated paging probability > 75% and <= 80%) |
| 1 | 0 | 0 | 0 | 1 | p85 (UE calculated paging probability > 80% and <= 85%) |
| 1 | 0 | 0 | 1 | 0 | p90 (UE calculated paging probability > 85% and <= 90%) |
| 1 | 0 | 0 | 1 | 1 | p95 (UE calculated paging probability > 90% and <= 95%) |
| 1 | 0 | 1 | 0 | 0 | p100 (UE calculated paging probability > 95% and <= 100%) |
| All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |

* * * End of Changes * * * *