3GPP TSG-CT Meeting #108	CP-251184

# Prague, Czech Republic; 09th – 10th June 2025	(was C1-253932)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.545 | CR | 0149 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the event-triggered location reporting procedure and the client-triggered or VAL server-triggered location reporting procedure for CoAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR0044r0 (in C1-223450) and CR0046r1 (in C1-224056) added the CoAP description of the event-triggered location reporting procedure and the client-triggered or VAL server-triggered location reporting procedure. However, those CRs added a wrong definition of one attribute which does not follow the necessary naming convention, quote: B) a "minimum-interval-length" attribute specifying the minimum time between consecutive reports. The value is given in seconds; and Additionally, the CR0113r1 (in C1-246981) failed to correct the “LocationReporConfiguration” object for the case when the SU_LocationReporting API is provided by SLM-C. This leaves stage 3 not aligned with stage-2 requirements in TS 23.434 as some necessary attributes in the “LocationReportConfiguration” object are not provided. Changes from the agreed version of the CR at CT1#155: Note that CR0149r1 (in C1-253932) was agreed by CT1 but after the end of the meeting, MCC discovered that the CR number on the cover sheet was not correct. Hence, the only change from the agreed version of the CR is to increase the CR number on the cover sheet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) Naming of the "minimum-interval-length" attribute is corrected to "minimumIntervalLength"; and (2) the “LocationReporConfiguration” object for the case when the SU_LocationReporting API is provided by SLM-C is corrected by adding missing attributes immediate report indicator and the endpoint information of the requesting VAL server to which the location report notification has to be sent. Backwards compatibility analysis: The CR is backwards compatible as new attributes are added as optional. Also, the naming convention is aligned within the specification (i.e., the CoAP-based procedures description is aligned with the Annex B on CoAP definition). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong naming convention for CoAP attribute remains in the specification. Stage-2 requirements on immediate report indicator and the endpoint information of the requesting VAL server to which the location report notification has to be sent are missing for the case the SU_LocationReporting API is provided by SLM-C. Hence, that functionality cannot be implemented but required by stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.4.1, 6.2.2.5.1, 6.2.4.3, B.4.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: correction of the naming convention also under clause 6.2.2.5.1. Adding clause B.3.1.5.2. V2: the only difference from the agreed CR at CT1#155 is the correction of the CR number on the cover sheet. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.2.4.1	Fetching location reporting configuration

In order to fetch trigger configuration, the SLM-C shall send a CoAP GET request message to the SLM-S according to procedures specified in IETF RFC 7252 [21]. In the CoAP GET request, the SLM-C:

a)	shall set the CoAP URI identifying the trigger configuration to be fetched according to the resource definition in Annex B.3.1.2.2;

1)	the "apiRoot" is set to the SLM-S URI;

2)	the "valServiceId" is set to specific VAL service; and

3)	the "val-tgt-ue" query option is set to either the VAL user identity or VAL UE identity for which the trigger configuration is applicable;

b)	shall include an Accept option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report-configuration"; and

c)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [6].

Upon receiving a CoAP 2.05 (Content) response from the SLM-S containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report-configuration"; and

b)	including a "LocationReportConfiguration" object,

the SLM-C:

a)	shall store the content of the "LocationReportConfiguration" object;

b)	shall set the location reporting triggers accordingly; and

c)	shall start the minimum report interval timer.

* * * Next Change * * * *

##### 6.2.2.5.1	Fetching location reporting configuration

Upon receiving of a CoAP GET request where the CoAP URI of the CoAP GET request identifies a trigger configuration as specified in Annex B.3.1.2.2.3.1, the SLM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP GET request is not authorized to fetch requested trigger configuration, shall respond with a CoAP 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall generate a CoAP 2.05 (Content) response according to IETF RFC 7252 [21]. In the CoAP 2.05 (Content) response message, the SLM-S:

1)	shall include a Content-Format option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report-configuration"; and

2)	shall include a "LocationReportConfiguration" object:

i)	shall include a "locationType" attribute which is requested; and

ii)	shall include at least one of the followings:

A)	a "triggeringCriteria" object which provides the triggers for the SLM-C to request a location report;

B)	a "minimumIntervalLength" attribute specifying the minimum time between consecutive reports. The value is given in seconds; and

C)	an "immediateReportInd" attribute which indicates that an immediate location report is required and if the "immediateReportInd" attribute indicates the immediate location report is required:

I)	an "endpointId" attribute containing the endpoint information of the requesting VAL server to which the location report notification has to be sent; and

iii)	may include a "requestedLocAccessType" attribute specifying the location access type for which the location information is requested; and

iv)	may include a "requestedPosMethod" attribute specifying the positioning method for which the location information is requested; and

c)	shall send the CoAP 2.05 (Content) response towards the SLM-C.

* * * Next Change * * * *

#### 6.2.4.3	SLM client CoAP procedure

Upon receiving a request from a VAL user to obtain the location information of another VAL user, the SLM-C shall:

a)	if trigger configuration is provided, send a CoAP FETCH request according to procedures specified in IETF RFC 8132 [24] to SLM-S to observe the location information of another VAL user; and

b)	otherwise, send a CoAP GET request according to procedure specified in in IETF RFC 7252 [21] to SLM-S to retrieve the location information of another VAL user.

In the CoAP FETCH request, the SLM-C shall:

a)	set the CoAP URI identifying the location information to be observed according to the resource definition in Annex B.3.1.2.4.3.1;

1)	the "apiRoot" is set to the SLM-S URI;

b)	include an Accept option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report";

c)	set an Observe option to 0 (Register);

d)	set a Content-Format option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report-configuration";

e)	include a "LocationReportConfiguration" object:

1)	shall include a "valTgtUes" object set to the identity of the observed VAL users;

2) shall include a "locationType" attribute which is requested; and

3) shall include at least one of the following:

i)	a "triggeringCriteria" object which provides the triggers for the SLM-C to request a location report as described in Annex B.3;

ii)	a "minimumIntervalLength" attribute specifying the minimum time between consecutive reports. The value is given in seconds;

iii)	an "immediateReportInd" attribute which indicates that an immediate location report is required and if the "immediateReportInd" attribute indicates the immediate location report is required:

A)	an "endpointId" attribute containing the endpoint information of the requesting VAL server to which the location report notification has to be sent; and

4)	may include a "requestedLocAccessType"object set to the identifies of the location access type for which the location information is requested; and

5)	may include a "requestedPosMethod" object set to the identifies the positioning method for which the location information is requested; and

f)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [6].

In the CoAP GET request, the SLM-C shall:

a)	set the CoAP URI identifying the location information to be fetched according to the resource definition in Annex B.3.1.2.4.3.2;

1)	the "apiRoot" is set to the SLM-S URI; and

2)	the "val-tgt-ue" query option is set to either the VAL user identity or VAL UE identity for which the location is requested;

b)	include an Accept option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report"; and

c)	send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [6].

Upon receiving a CoAP 2.05 (Content) response from the SLM-S containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-location-info+cbor;modeltype=location-report"; and

b)	including one or more "LocationReport" objects,

the SLM-C:

a)	shall store the content of the received "LocationReport" object(s).

* * * Next Change * * * *

#### B.3.1.5.2	CDDL document

;;; LocationAreaQuery

LocationAreaQuery = {

geoArea: GeographicArea

}

;;; LocationAreaInfo

LocationAreaInfo = {

? valReqUe: ValTargetUe

? ueList: [* UeInfo]

}

;;; UeInfo

UeInfo = {

? ueId: ValTargetUe

? ueLoc: LocationInfo

}

;;; LocationReportConfiguration

;;+ Represents Location reporting configuration information.

LocationReportConfiguration = {

valTgtUes: [* ValTargetUe]

locationType: Accuracy

? triggeringCriteria: [* TriggeringCriteriaType]

? minimumIntervalLength: Uinteger

? immediateReportInd: boolean

? endpointId: EndpointId

? AccessType: [* AccessTypeType]

? PositioningMethod: [* PositioningMethodType]

}

;;; Accuracy

Accuracy = "CURRENT_SERVING_NCGI" / "NEIGHBOURING_NCGI" / "MBMS_SA" / "MBSFN_AREA" / "CURRENT_GEOGRAPHICAL_COORDINATE"

;;; TriggeringCriteriaType

TriggeringCriteriaType = {

? cellChange: CellChange

? trackingAreaChange: TrackingAreaChange

? plmnChange: PlmnChange

? mbmsSaChange: MbmsSaChange

? mbsfnAreaChange: MbsfnAreaChange

? periodicReport: PeriodicReport

? travelledDistance: TravelledDistance

? verticalAppEvent: VerticalAppEvent

? geographicalAreaChange: GeographicalAreaChange

? validPeriod: [+ ScheduledCommunicationTime]; The list of the scheduled time intervals for the reporting in form of day of the week and/or time period.

}

;;; CellChange

CellChange = {

? anyCellChange: BaseTrigger

? enterSpecificCells: SpecificCells

? exitSpecificCells: SpecificCells

}

;;; SpecificCells

SpecificCells = {

triggerId: TriggerId

cells: [* CellId]

}

;;; TrackingAreaChange

TrackingAreaChange = {

? anyTrackingAreaChange: BaseTrigger

? enterSpecificTrackingAreas: SpecificTrackingAreas

? exitSpecificTrackingAreas: SpecificTrackingAreas

}

;;; SpecificTrackingAreas

SpecificTrackingAreas = {

triggerId: TriggerId

trackingAreas: [* TaId]

}

;;; PlmnChange

PlmnChange = {

? AnyPlmnChange: BaseTrigger

? EnterSpecificPlmns: SpecificPlmns

? ExitSpecificPlmns: SpecificPlmns

}

;;; SpecificPlmns

SpecificPlmns = {

triggerId: TriggerId

plmns: [* PlmnId]

}

;;; MbmsSaChange

MbmsSaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificPlmns: SpecificMbmsSas

? exitSpecificPlmns: SpecificMbmsSas

}

;;; SpecificMbmsSas

SpecificMbmsSas = {

triggerId: TriggerId

mbmsSas: [* MbmsSaId]

}

;;; MbsfnAreaChange

MbsfnAreaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificMbsfnAreas: SpecificMbsfnAreas

? exitSpecificPlmn: SpecificMbsfnAreas

}

;;; SpecificMbsfnAreas

SpecificMbsfnAreas = {

triggerId: TriggerId

mbsfnAreas: [* MbsfnAreaId]

}

;;; PeriodicReport

PeriodicReport = {

triggerId: TriggerId

interval: Uinteger

}

;;; TravelledDistance

TravelledDistance = {

triggerId: TriggerId

distance: Uinteger

}

;;; VerticalAppEvent

VerticalAppEvent = {

? initialLogOn: BaseTrigger

? locConfigReceived: BaseTrigger

? anyOtherEvent: BaseTrigger

}

;;; GeographicalAreaChange

GeographicalAreaChange = {

? AnyGeoAreaChange: BaseTrigger

? EnterSpecificGeoAreas: SpecificGeoAreas

? ExitSpecificGeoAreas: SpecificGeoAreas

}

;;; SpecificGeoAreas

SpecificGeoAreas = {

triggerId: TriggerId

geoAreas: [* GeographicArea]

}

;;; EndpointId

;;+ Unique identifier of an endpoint of the selected VAL server.

EndpointId = text

;;; ScheduledCommunicationTime

;;+ Represents the scheduled time interval.

ScheduledCommunicationTime = {

? daysOfWeek: [1*6 DayOfWeek]   ; Identifies the day(s) of the week. If absent, it indicates every day of the week.

? timeOfDayStart: TimeOfDay

? timeOfDayEnd: TimeOfDay

}

;;; DayOfWeek

;;+ Integer between and including 1 and 7 denoting a weekday. Value 1 shall indicate Monday, and the subsequent weekdays shall be indicated with the next higher numbers, so value 7 shall indicate Sunday.

DayOfWeek = 1..7

;;; TimeOfDay

;;+ String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

TimeOfDay = text

;;; RequestedLocation

RequestedLocation = {

valTgtUes: [* ValTargetUe]

? locationType: Accuracy

? requestedLocAccessType: [* LocationAccessTypeType]

? requestedPosMethod: [* PositioningMethodType]

}

;;; LocationReport

LocationReport = {

valTgtUe: ValTargetUe

triggerIds: [* TriggerId]

locInfo: LocationInfo

}

;;; LocationInfo

LocationInfo = {

? cellId: CellId

? neighbouringCellIds: [* CellId]

? mbmsSaId: MbmsSaId

? mbsfnAreaId: MbsfnAreaId

? currentCoordinate: GeographicalCoordinates

}

;;; BaseTrigger

BaseTrigger = {

triggerId: TriggerId

}

;;; TriggerId

;;+ Unique identifier of a trigger.

TriggerId = text

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; GeographicArea

;;+ Geographic area specified by different shape.

GeographicArea = Point / PointUncertaintyCircle / PointUncertaintyEllipse / Polygon / PointAltitude / PointAltitudeUncertainty / EllipsoidArc

;;; GADShape

;;+ Common base type for GAD shapes.

GADShape = {

shape: SupportedGADShapes

}

;;; Point

;;+ Ellipsoid Point.

Point = {

~GADShape

point: GeographicalCoordinates

}

;;; PointUncertaintyCircle

;;+ Ellipsoid point with uncertainty circle.

PointUncertaintyCircle = {

~GADShape

point: GeographicalCoordinates

uncertainty: Uncertainty

}

;;; PointUncertaintyEllipse

;;+ Ellipsoid point with uncertainty ellipse.

PointUncertaintyEllipse = {

~GADShape

point: GeographicalCoordinates

uncertaintyEllipse: UncertaintyEllipse

confidence: Confidence

}

;;; Polygon

;;+ Polygon.

Polygon = {

~GADShape

pointList: PointList

}

;;; PointAltitude

;;+ Ellipsoid point with altitude.

PointAltitude = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

}

;;; PointAltitudeUncertainty

;;+ Ellipsoid point with altitude and uncertainty ellipsoid.

PointAltitudeUncertainty = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

uncertaintyEllipse: UncertaintyEllipse

uncertaintyAltitude: Uncertainty

confidence: Confidence

}

;;; EllipsoidArc

;;+ Ellipsoid Arc.

EllipsoidArc = {

~GADShape

point: GeographicalCoordinates

innerRadius: InnerRadius

uncertaintyRadius: Uncertainty

offsetAngle: Angle

includedAngle: Angle

confidence: Confidence

}

;;; GeographicalCoordinates

;;+ Geographical coordinates.

GeographicalCoordinates = {

lon: -180.0..180.0

lat: -90.0..90.0

}

;;; UncertaintyEllipse

;;+ Ellipse with uncertainty.

UncertaintyEllipse = {

semiMajor: Uncertainty

semiMinor: Uncertainty

orientationMajor: Orientation

}

;;; PointList

;;+ List of points.

PointList = [3*15 GeographicalCoordinates]

;;; Altitude

;;+ Indicates value of altitude.

Altitude = -32767.0..32767.0

;;; Angle

;;+ Indicates value of angle.

Angle = 0..360

;;; Uncertainty

;;+ Indicates value of uncertainty.

Uncertainty = float32 .ge 0

;;; Orientation

;;+ Indicates value of orientation angle.

Orientation = 0..180

;;; Confidence

;;+ Indicates value of confidence.

Confidence = 0..100

;;; InnerRadius

;;+ Indicates value of the inner radius.

InnerRadius = (0..327675) .and int32

;;; SupportedGADShapes

;;+ Indicates supported GAD shapes.

SupportedGADShapes = "POINT" / "POINT_UNCERTAINTY_CIRCLE" / "POINT_UNCERTAINTY_ELLIPSE" / "POLYGON" / "POINT_ALTITUDE" / "POINT_ALTITUDE_UNCERTAINTY" / "ELLIPSOID_ARC" / "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE" / "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID" / text

;;; CellId

;;+ Unique identifier of a cell.

CellId = text

;;; TaId

;;+ Unique identifier of a tracking area.

TaId = text

;;; PlmnId

;;+ Unique identifier of a PLMN.

PlmnId = text

;;; MbmsSaId

;;+ Unique identifier of a MBMS serving area.

MbmsSaId = text

;;; MbsfnAreaId

;;+ Unique identifier of a MBSFN area.

MbsfnAreaId = text

* * * Next Change * * * *

#### B.4.1.5.2	CDDL document

;;; LocationReportConfiguration

;;+ Represents Location reporting configuration information.

LocationReportConfiguration = {

valTgtUes: [* ValTargetUe]

locationType: Accuracy

? triggeringCriteria: [* TriggeringCriteriaType]

? minimumIntervalLength: Uinteger

? immediateReportInd: boolean

? endpointId: EndpointId

}

;;; Accuracy

Accuracy = "CURRENT_SERVING_NCGI" / "NEIGHBOURING_NCGI" / "MBMS_SA" / "MBSFN_AREA" / "CURRENT_GEOGRAPHICAL_COORDINATE"

;;; TriggeringCriteriaType

TriggeringCriteriaType = {

? cellChange: CellChange

? trackingAreaChange: TrackingAreaChange

? plmnChange: PlmnChange

? mbmsSaChange: MbmsSaChange

? mbsfnAreaChange: MbsfnAreaChange

? periodicReport: PeriodicReport

? travelledDistance: TravelledDistance

? verticalAppEvent: VerticalAppEvent

? geographicalAreaChange: GeographicalAreaChange

? validPeriod: [+ ScheduledCommunicationTime]; The list of the scheduled time intervals for the reporting in form of day of the week and/or time period.

}

;;; CellChange

CellChange = {

? anyCellChange: BaseTrigger

? enterSpecificCells: SpecificCells

? exitSpecificCells: SpecificCells

}

;;; SpecificCells

SpecificCells = {

triggerId: TriggerId

cells: [* CellId]

}

;;; TrackingAreaChange

TrackingAreaChange = {

? anyTrackingAreaChange: BaseTrigger

? enterSpecificTrackingAreas: SpecificTrackingAreas

? exitSpecificTrackingAreas: SpecificTrackingAreas

}

;;; SpecificTrackingAreas

SpecificTrackingAreas = {

triggerId: TriggerId

trackingAreas: [* TaId]

}

;;; PlmnChange

PlmnChange = {

? AnyPlmnChange: BaseTrigger

? EnterSpecificPlmns: SpecificPlmns

? ExitSpecificPlmns: SpecificPlmns

}

;;; SpecificPlmns

SpecificPlmns = {

triggerId: TriggerId

plmns: [* PlmnId]

}

;;; MbmsSaChange

MbmsSaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificPlmns: SpecificMbmsSas

? exitSpecificPlmns: SpecificMbmsSas

}

;;; SpecificMbmsSas

SpecificMbmsSas = {

triggerId: TriggerId

mbmsSas: [* MbmsSaId]

}

;;; MbsfnAreaChange

MbsfnAreaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificMbsfnAreas: SpecificMbsfnAreas

? exitSpecificPlmn: SpecificMbsfnAreas

}

;;; SpecificMbsfnAreas

SpecificMbsfnAreas = {

triggerId: TriggerId

mbsfnAreas: [* MbsfnAreaId]

}

;;; PeriodicReport

PeriodicReport = {

triggerId: TriggerId

interval: Uinteger

}

;;; TravelledDistance

TravelledDistance = {

triggerId: TriggerId

distance: Uinteger

}

;;; VerticalAppEvent

VerticalAppEvent = {

? initialLogOn: BaseTrigger

? locConfigReceived: BaseTrigger

? anyOtherEvent: BaseTrigger

}

;;; GeographicalAreaChange

GeographicalAreaChange = {

? AnyGeoAreaChange: BaseTrigger

? EnterSpecificGeoAreas: SpecificGeoAreas

? ExitSpecificGeoAreas: SpecificGeoAreas

}

;;; SpecificGeoAreas

SpecificGeoAreas = {

triggerId: TriggerId

geoAreas: [* GeographicArea]

}

;;; ScheduledCommunicationTime

;;+ Represents the scheduled time interval.

ScheduledCommunicationTime = {

? daysOfWeek: [1*6 DayOfWeek]   ; Identifies the day(s) of the week. If absent, it indicates every day of the week.

? timeOfDayStart: TimeOfDay

? timeOfDayEnd: TimeOfDay

}

;;; DayOfWeek

;;+ Integer between and including 1 and 7 denoting a weekday. Value 1 shall indicate Monday, and the subsequent weekdays shall be indicated with the next higher numbers, so value 7 shall indicate Sunday.

DayOfWeek = 1..7

;;; TimeOfDay

;;+ String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

TimeOfDay = text

;;;AccessTypeType

AccessType = "3GPP_ACCESS" / "NON_3GPP_ACCESS" / text

;;;PositioningMethodType

AccessType = "CELLID" / "ECID" / "OTDOA" / "BAROMETRIC_PRESSURE" / "WLAN" / "BLUETOOTH" / "MBS" / "MOTION_SENSOR" / "DL_TDOA" / "DL_AOD" / "MULTI-RTT" / "NR_ECID" / "UL_TDOA" / "UL_AOA" / "NETWORK_SPECIFIC" / text

;;; LocationReport

LocationReport = {

valTgtUe: ValTargetUe

triggerIds: [* TriggerId]

locInfo: LocationInfo

}

;;; LocationInfo

LocationInfo = {

? cellId: CellId

? neighbouringCellIds: [* CellId]

? mbmsSaId: MbmsSaId

? mbsfnAreaId: MbsfnAreaId

? currentCoordinate: GeographicalCoordinates

}

;;; BaseTrigger

BaseTrigger = {

triggerId: TriggerId

}

;;; TriggerId

;;+ Unique identifier of a trigger.

TriggerId = text

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; GeographicArea

;;+ Geographic area specified by different shape.

GeographicArea = Point / PointUncertaintyCircle / PointUncertaintyEllipse / Polygon / PointAltitude / PointAltitudeUncertainty / EllipsoidArc

;;; GADShape

;;+ Common base type for GAD shapes.

GADShape = {

shape: SupportedGADShapes

}

;;; Point

;;+ Ellipsoid Point.

Point = {

~GADShape

point: GeographicalCoordinates

}

;;; PointUncertaintyCircle

;;+ Ellipsoid point with uncertainty circle.

PointUncertaintyCircle = {

~GADShape

point: GeographicalCoordinates

uncertainty: Uncertainty

}

;;; PointUncertaintyEllipse

;;+ Ellipsoid point with uncertainty ellipse.

PointUncertaintyEllipse = {

~GADShape

point: GeographicalCoordinates

uncertaintyEllipse: UncertaintyEllipse

confidence: Confidence

}

;;; Polygon

;;+ Polygon.

objecv5 = {

pointList: PointList

}

Polygon = {

~GADShape

pointList: PointList

}

;;; PointAltitude

;;+ Ellipsoid point with altitude.

PointAltitude = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

}

;;; PointAltitudeUncertainty

;;+ Ellipsoid point with altitude and uncertainty ellipsoid.

PointAltitudeUncertainty = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

uncertaintyEllipse: UncertaintyEllipse

uncertaintyAltitude: Uncertainty

confidence: Confidence

}

;;; EllipsoidArc

;;+ Ellipsoid Arc.

EllipsoidArc = {

~GADShape

point: GeographicalCoordinates

innerRadius: InnerRadius

uncertaintyRadius: Uncertainty

offsetAngle: Angle

includedAngle: Angle

confidence: Confidence

}

;;; GeographicalCoordinates

;;+ Geographical coordinates.

GeographicalCoordinates = {

lon: -180.0..180.0

lat: -90.0..90.0

}

;;; UncertaintyEllipse

;;+ Ellipse with uncertainty.

UncertaintyEllipse = {

semiMajor: Uncertainty

semiMinor: Uncertainty

orientationMajor: Orientation

}

;;; PointList

;;+ List of points.

PointList = [3*15 GeographicalCoordinates]

;;; Altitude

;;+ Indicates value of altitude.

Altitude = -32767.0..32767.0

;;; Angle

;;+ Indicates value of angle.

Angle = 0..360

;;; Uncertainty

;;+ Indicates value of uncertainty.

Uncertainty = float32 .ge 0

;;; Orientation

;;+ Indicates value of orientation angle.

Orientation = 0..180

;;; Confidence

;;+ Indicates value of confidence.

Confidence = 0..100

;;; InnerRadius

;;+ Indicates value of the inner radius.

InnerRadius = (0..327675)

;;; SupportedGADShapes

;;+ Indicates supported GAD shapes.

SupportedGADShapes = "POINT" / "POINT_UNCERTAINTY_CIRCLE" / "POINT_UNCERTAINTY_ELLIPSE" / "POLYGON" / "POINT_ALTITUDE" / "POINT_ALTITUDE_UNCERTAINTY" / "ELLIPSOID_ARC" / "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE" / "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID" / text

;;; CellId

;;+ Unique identifier of a cell.

CellId = text

;;; TaId

;;+ Unique identifier of a tracking area.

TaId = text

;;; PlmnId

;;+ Unique identifier of a PLMN.

PlmnId = text

;;; MbmsSaId

;;+ Unique identifier of a MBMS serving area.

MbmsSaId = text

;;; MbsfnAreaId

;;+ Unique identifier of a MBSFN area.

MbsfnAreaId = text

* * * End of Changes * * * *