3GPP TSG-CT WG1 Meeting #154	C1-251601

# Wuhan, P.R. China; 07th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0726 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on changes of 5G ProSe multi-hop U2N relay UE's PDU session address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If the UE acting as a 5G ProSe multi-hop layer-3 UE-to-network relay UE established the PDU session for relay services in SSC mode 3, and obtains a new IPv6 prefix by performing the UE-requested PDU session establishment procedure upon receiving the 5GSM cause IE set to #39 "reactivation requested", the UE needs to update the IPv6 prefix of the remote UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE shall advertise the new IPv6 prefix to be used by the5G ProSe multi-hop layer-3 remote UE in the case mentioned as above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IPv6 packet of the remote UE is not able to be relayed by the relay UE if the IPv6 prefix is not updated accordingly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8b.2.7	IP address allocation in 5G ProSe multi-hop layer-3 UE-to-network relay procedure

For the UE acting as the 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop layer-3 remote UE receiving the IP address configuration IE in the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message from the UE's 5G ProSe multi-hop parent UE-to-network relay UE,

a)	if the IP address configuration IE indicates "DHCPv4 Server", the UE shall initiate the IPv4 address configuration with DHCPv4 procedure acting as a DHCP client, according to IETF RFC 2131 [23] and IETF RFC 4039 [24];

b)	if the IP address configuration IE indicates "IPv6 Router", the UE shall initiate the IPv6 address configuration with IPv6 stateless address auto-configuration acting as an IPv6 host based on IETF RFC 4862 [25].

The 5G ProSe layer-3 intermediate UE-to-network relay UE(s) are responsible for relaying router solicitation messages and router advertisement messages between the UE acting as the 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop layer-3 remote UE and the 5G ProSe multi-hop UE-to-network relay UE; and

c)	if the IP address configuration IE indicates "DHCPv4 Server & IPv6 Router", the UE shall choose the IP version and initiate the corresponding IP address configuration procedure as a client or host.

For the UE acting as a 5G ProSe multi-hop layer-3 UE-to-network relay UE, if

a)	the SSC mode of the PDU session is SSC mode 3; and

b)	the UE obtains a new IPv6 prefix by performing the UE-requested PDU session establishment procedure upon receiving the 5GSM cause IE set to #39 "reactivation requested" as specified in clause 6.3.2.3 of 3GPP TS 24.501 [11],

the UE shall advertise the new IPv6 prefix to be used by the5G ProSe multi-hop layer-3 remote UE as defined in RFC 4862 [25].

NOTE:	If the UE receives a new IPv4 address, the UE does not need to update the IPv4 address of the 5G ProSe multi-hop layer-3 remote UE since the IPv4 address of the 5G ProSe multi-hop layer-3 remote UE will be NAT translated by the UE.

Editor's note (5G_ProSe_Ph3, CR#0698):	Whether IPv6 prefix delegation via DHCPv6 and stateless DHCPv6 for additional parameter configuration are supported in 5G ProSe multi-hop layer-3 UE-to-network relay procedure is FFS.

* * * End of Changes * * * *