3GPP TSG-CT Meeting #108	CP-251234

# Prague, Czech Republic; 09th – 10th June 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.504 | CR | 0311 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Event Exposure subscription context resynchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, ReP_UDR |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Event Exposure subscription context resynchronization is not covered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New optional indication “Data to be resynchronized” is defined for the Data Restoration Notification. This is required since the NEF may potentially be able to re-create different data types (e.g. Event Exposure, Parameter Provisioning, PFDs…), then it has to be clearly indicated which one is required. New enumeration to define possible data to be resynchronized is reused from definition in 29.503. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Event Exposure subscription context is not possible to be recovered/resynchronized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.5a.1, 6.1.5a.2.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CT4 #129 agreed CR Rev 1 in C4-252387 is revised to change the attribute name to udmDataToResync in accordance with the naming conventions defined in TS 29.501. In addition, corrected clause identity for DataRestorationNotification data type in table 6.1.5a.1-1 and table identity in clause 6.1.5a.2.2. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[3]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository Service for Policy Data, Structured Data for Exposure and Application Data; Stage 3".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[7]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[8]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[9]	IETF RFC 6901(April 2013): "JavaScript Object Notation (JSON) Pointer".

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[15]	IETF RFC 9110: "HTTP Semantics ".

[16]	IETF RFC 9111: "HTTP Caching".

[17]	IETF RFC 9457: "Problem Details for HTTP APIs".

[18]	IETF RFC 7396: "JSON Merge Patch".

[19]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[20]	3GPP TR 21.900: "Technical Specification Group working methods".

[21]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0

[22]	IETF RFC 9113: "HTTP/2".

[23]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[24]	3GPP TS 23.527: "5G System; Restoration procedures".

[XX]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

* * * Next Change * * * *

#### 6.1.5a.1	General

This clause specifies the application data model supported by the API.

Table 6.1.5a.1-1 specifies the data types defined for the Nudr_DataRepository API.

Table 6.1.5a.1-1: Nudr_DataRepository specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| DataRestorationNotification | 6.1.5a.2.2 | Contains identities representing those UEs potentially affected by a data-loss event at the UDR |

Table 6.1.5a.1-2 specifies data types re-used by the Nudr_DataRepository service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository service-based interface.

Table 6.1.5a.1-2: Nudr_DataRepository re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [10] | Data Network Name with Network Identifier only. |
| Snssai | 3GPP TS 29.571 [10] | Single NSSAI |
| NfGroupId | 3GPP TS 29.571 [10] | NF Group ID |
| IdentityRange | 3GPP TS 29.510 [14] | Identity Range |
| SupiRange | 3GPP TS 29.510 [14] | SUPI Range |
| UdmDataToResynchronize | 3GPP TS 29.503 [XX] | Data that is required to resynchronize from UDM consumers. |

* * * Next Change * * * *

##### 6.1.5a.2.2	Type: DataRestorationNotification

Table 6.1.5a.2.2-1: Definition of type DataRestorationNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supiRanges | array(SupiRange) | O | 1..N | If present, it contains the list of SUPIs potentially subject to a data-loss event, or subject to a Subscriber Data Migration procedure, at the UDR. |  |
| gpsiRanges | array(IdentityRange) | O | 1..N | If present, it contains the list of GPSIs potentially subject to a data-loss event, or subject to a Subscriber Data Migration procedure, at the UDR. |  |
| resetIds | array(string) | O | 1..N | If present, it contains the list of Reset-IDs of those UEs potentially subject to a data-loss event at the UDR. |  |
| sNssaiList | array(Snssai) | O | 1..N | If present, it contains the list of slices (S-NSSAIs) potentially subject to a data-loss event at the UDR. |  |
| dnnList | array(Dnn) | O | 1..N | If present, it contains the list of DNNs potentially subject to a data-loss event at the UDR. |  |
| lastReplicationTime | DateTime | O | 0..1 | If present, it contains the timestamp of the most recent instant when the data was assumed to be consistent at UDR (i.e. the potential data loss event at UDR did not occur before this instant). In the case of subscriber data migration procedure, this value indicates the time when a UDR started subscriber data migration. |  |
| recoveryTime | DateTime | O | 0..1 | If present, it contains the timestamp of the instant when the potential data loss event was recovered at UDR (i.e. all data records stored by UDR after this time are assumed to be consistent). In the case of subscriber data migration procedure, this value indicates the time when traffic switch was enforced for a subscriber data migration procedure (e.g. the NFProfile update into NRF). |  |
| udrGroupId | NfGroupId | O | 0..1 | If present, it contains the ID of the UDR Group whose UEs have been potentially subject to a data loss event at the UDR. |  |
| rediscoveryInd | boolean | O | 0..1 | If present with value true, UDR rediscovery is required due to subscriber reallocation in a different UDR group Id. Default: false. | SUBDMIG |
| noResynchronizationRequired | Boolean | C | 0..1 | If present, it indicates whether data resynchronization is required. It shall only be present when rediscoveryInd is present and set to true. - true: data resynchronization is not required - false (default): data resynchronization is required | SUBDMIG |
| udmDataToResync | array(UdmDataToResynchronize) | O | 1..N | If present, it indicates the data that is required to resynchronize from UDM consumers. |  |

* * * Next Change * * * *

# A.2	Nudr_DataRepository API

(... text not shown for clarity ...)

schemas:

DataRestorationNotification:

description: >

Contains identities representing those UEs potentially affected by a data-loss event,

or a subscriber data migration event, at the UDR

type: object

properties:

supiRanges:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/SupiRange'

minItems: 1

gpsiRanges:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IdentityRange'

minItems: 1

resetIds:

type: array

items:

type: string

minItems: 1

sNssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

udrGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

rediscoveryInd:

type: boolean

default: false

noResynchronizationRequired:

type: boolean

default: false

udmDataToResync:

type: array

items:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/UdmDataToResynchronize'

minItems: 1

* * * End of Changes * * * *