3GPP TSG-CT Meeting #108	CP-251242

Prague, Czech Republic; 09th – 10th June 2025	(Revision of C3-252307)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1627 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to OpenAPI file of UAVFlightAssistance API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PUT and DELETE methods were wrongly referring TS 29.571 response codes which is not aligned with NBI OpenAPI file definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the PUT and DELETE methods response codes referring from TS 29.571 to TS 29.122. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with NBI OpenAPI file definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible correction to the following API: TS29522_UAVFlightAssistance.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 provides additional update: Removed the HTTP 501 and 502 response codes which are not defined in TS 29.122 common data API and not applied in NBI API yet. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

# A.37	UAVFlightAssistance API

openapi: 3.0.0

info:

title: 3gpp-uav-flight-assistance

version: 1.0.0-alpha.2

description: |

API for UAV Flight Assistance.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.2.0; 5G System; Network Exposure Function Northbound APIs.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/

servers:

- url: '{apiRoot}/3gpp-uav-flight-assistance/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{afId}/configurations:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF.

required: true

schema:

type: string

get:

summary: Request to retrieve all the active UAV Flight Assistance Configurations at the NEF.

operationId: GetUavFlightAssistConfig

tags:

- UAV Flight Assistance Configurations (Collection)

responses:

'200':

description: >

OK. All the Individual UAV Flight Assistance Configuration resources managed by the NEF

are returned.

If there are no existing Individual UAV Flight Assistance Configuration resources

managed at the NEF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/UavFlightAssistance'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new UAV Flight Assistance Configuration.

operationId: CreateUavFlightAssistConfig

tags:

- UAV Flight Assistance Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

responses:

'201':

description: >

Created. The UAV Flight Assistance Configuration is successfully created and a

representation of the created Individual UAV Flight Assistance Configuration resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

headers:

Location:

description: >

Contains the URI of the created Individual UAV Flight Assistance Configuration

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UavFlightAssistNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UAVFlightAssistNotif'

responses:

'200':

description: >

OK. The notification is successfully received and acknowledged, and notification

related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UAVFlightAssistNotifResp'

'204':

description: >

No Content. The notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/configurations/{configurationId}:

parameters:

- name: afId

description: Represents the identifier of the AF.

in: path

required: true

schema:

type: string

- name: configurationId

description: >

Represents the identifier of the Individual UAV Flight Assistance Configuration resource.

in: path

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual UAV Flight Assistance Configuration resource.

operationId: GetIndUavFlightAssistConfig

tags:

- Individual UAV Flight Assistance Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual UAV Flight Assistance Configuration resource is returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request to update an existing Individual UAV Flight Assistance Configuration.

operationId: UpdateIndUavFlightAssistConfig

tags:

- Individual UAV Flight Assistance Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

responses:

'200':

description: >

OK. The Individual UAV Flight Assistance Configuration resource is successfully

updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

'204':

description: >

No Content. The Individual UAV Flight Assistance Configuration resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual UAV Flight Assistance Configuration resource.

tags:

- Individual Group Parameters Provisioning (Document)

operationId: ModifyIndUavFlightAssistConfig

requestBody:

description: >

Contains the parameters to request the modification of the Individual UAV Flight

Assistance Configuration resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UavFlightAssistancePatch'

responses:

'200':

description: >

OK. The Individual UAV Flight Assistance Configuration resource is successfully modified

and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UavFlightAssistance'

'204':

description: >

No Content. The Individual UAV Flight Assistance Configuration resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual UAV Flight Assistance Configuration.

operationId: DeleteIndUavFlightAssistConfig

tags:

- Individual UAV Flight Assistance Configuration (Document)

responses:

'204':

description: >

No Content. The Individual UAV Flight Assistance Configuration resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

UavFlightAssistance:

description: Represents the UAV Flight Assistance Configuration.

type: object

properties:

uavId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

reqPurpose:

$ref: '#/components/schemas/RequestPurpose'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

flightPath:

$ref: '#/components/schemas/FlightPath'

altReports:

type: array

items:

$ref: '#/components/schemas/AltitudeReport'

minItems: 1

description: Indicates the UAV altitude report.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- uavId

- reqPurpose

- notifUri

FlightPath:

description: Represents a flight path.

type: object

properties:

pathSegInfo:

type: array

items:

$ref: '#/components/schemas/FlightPathSegInfo'

minItems: 1

required:

- pathSegInfo

FlightPathSegInfo:

description: Represents a flight path segment.

type: object

properties:

flightSeg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

uavSpeed:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

timeSchedule:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeDuration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- flightSeg

not:

required: [timeSchedule, timeDuration]

UAVFlightAssistNotif:

description: Notify the UAV status.

type: object

properties:

subscriptionId:

type: string

uavStatus:

$ref: '#/components/schemas/UavStatus'

borderPoint:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

altReportReqs:

$ref: '#/components/schemas/AltReportReqs'

required:

- subscriptionId

- uavStatus

UAVFlightAssistNotifResp:

description: Represents the response to the notification.

type: object

properties:

ussChgInd:

type: boolean

description: >

This attribute is used to indicate whether the USS changeover is completed successfully.

true indicates that USS changeover is completed

successfully; false (default): USS changeover failed.

required:

- ussChgInd

UavFlightAssistancePatch:

description: Represents the requested modifications to a UAV Flight Assistance Configuration.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

flightPath:

$ref: '#/components/schemas/FlightPath'

AltReportReqs:

description: Represents the UAV altitude reporting requirements.

type: object

properties:

altThreshold:

$ref: '#/components/schemas/AltRange'

altReportPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

altReportEvent:

$ref: '#/components/schemas/AltReportEvent'

AltitudeReport:

description: Represents the UAV altitude report.

type: object

properties:

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AltRange:

description: Represents the UAV altitude range for reporting.

type: object

properties:

lowerBound:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

upperBound:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

#

# ENUMERATIONS

#

UavStatus:

anyOf:

- type: string

enum:

- LEAVING

- ENTERING

- DEVIATING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the status of the UAV.

Possible values are:

- LEAVING: Indicates that the UAV status is leaving an area which is part of the planned

flight path.

- ENTERING: Indicates that the UAV status is entering an area which is part of the planned

flight path.

- DEVIATING: Indicates that the UAV status is deviating from the planned flight path.

RequestPurpose:

anyOf:

- type: string

enum:

- USS_CHANGEOVER

- PRE_FLIGHT_PLANNING

- IN_FLIGHT_MONITORING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the UAV Flight Assitance request purpose.

Possible values are:

- USS_CHANGEOVER: Indicates that the purpose of the request is pre-flight planning.

- PRE_FLIGHT_PLANNING: Indicates that the purpose of the request is USS changeover.

- IN_FLIGHT_MONITORING:Indicates that the purpose of the request is in-flight monitoring.

AltReportEvent:

anyOf:

- type: string

enum:

- TA_CHANGE

- RAN_NODE_CHANGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the Altitude reporting events.

Possible values are:

- TA_CHANGE: Indicates the change of UAV’s serving TA.

- RAN_NODE_CHANGE: Indicates the change of UAV’s serving RAN node.

*** End of Changes ***