3GPP TSG-CT Meeting #108	CP-251246

# Prague, Czech Republic; 09th – 10th June 2025								(revision of C3-252545)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 1075 | rev | 2 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Define OpenAPI for Nnwdaf_VFLTraining API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Nokia, Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-04-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Nnwdaf_VFLTrainingTraining was defined in TS 23.288 for VFL model training. This CR proposes to define OpenAPI for Nnwdaf_VFLTrainingTraining API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the OpenAPI for Nnwdaf_VFLTrainingTraining API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.10(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new OpenAPI for the following API: TS29520_Nnwdaf_VFLTraining.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: - add the missing definition of VflIntermedTrainInfo data type. - add the missing "type" for "vflFeatIds" and "interopInfo" attributes. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

A.10	Nnwdaf_VFLTraining API

openapi: 3.0.0

info:

title: Nnwdaf_VFLTraining

version: 1.0.0-alpha.1

description: |

Nnwdaf_VFLTraining API Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.520 V19.2.0; 5G System; Network Data Analytics Services.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.520/

servers:

- url: '{apiRoot}/nnwdaf-vfltraining/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnwdaf-vfltraining

paths:

/subscriptions:

post:

summary: Create a new Individual VFL Training Subscription resource.

operationId: CreateNWDAFVFLTrainingSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

responses:

'201':

description: Create a new Individual VFL Training Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nnwdaf-vfltraining/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

nwdafVflTrainingNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingNotify'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: update an existing Individual VFL Training Subscription

operationId: UpdateNWDAFVFLTrainingSubcription

tags:

- Individual VFL Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Nnwdaf_VFLTraining Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual VFL Training Subscription resource was modified successfully and a

representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'204':

description: >

The Individual VFL Training Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual VFL Training Subscription.

operationId: DeleteNWDAFVFLTrainingSubcription

tags:

- Individual VFL Training Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Nnwdaf_VFLTraining Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual VFL Training Subscription matching the subscriptionId was

deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnwdaf-vfltraining: Access to the Nnwdaf_VFLTraining API

schemas:

VflTrainingSubs:

description: Represents VFL Training subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

vflTrainSub:

type: array

items:

$ref: '#/components/schemas/VflTrainingSub'

minItems: 1

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- notifCorrId

- vflTrainSub

VflTrainingSub:

description: Represents VFL Training subscription for a analytics ID.

type: object

properties:

event:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

vflCorrId:

type: string

description: Represents the VFL Correlation ID.

interopInfo:

$ref: '#/components/schemas/VflInteropInfo'

maxRspTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

trainFilter:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventFilter'

intermediateInfo:

$ref: '#/components/schemas/VflIntermedTrainInfo'

chkFlg:

type: boolean

description: >

Set to "true" to indicate that the ML model accuracy monitoring information is

requested.The default value is "false" if omitted.

sampIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

description: Represents the initial sample list provided by the VFL server.

selectedSampIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

description: Represents the samples which are selected from the initial sample list.

chkPoint:

$ref: '#/components/schemas/VflCheckpointInfo'

vflFeatIds:

type: array

items:

type: string

minItems: 1

minNumSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeWindows:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

required:

- event

- vflCorrId

VflCheckpointInfo:

description: Represents VFL Training check point information.

type: object

properties:

saveCheckpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

resRoundInd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

oneOf:

- required: [saveCheckpoint]

- required: [resRoundInd]

VflInteropInfo:

description: Represents VFL interoperability information.

type: object

properties:

interopInfo:

type: array

items:

type: string

minItems: 1

required:

- interopInfo

VflTrainingNotify:

description: Represents a ML Model Training notification.

type: object

properties:

notifCorrId:

type: string

description: Notification correlation identifier.

intermediateInfo:

$ref: '#/components/schemas/VflIntermedTrainInfo'

modelMetric:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/MLModelMetric'

accMlModel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- notifCorrId

VflIntermedTrainInfo:

description: Represents the intermediate model training information.

type: object

properties:

intermedTrainInfo:

type: array

items:

type: string

minItems: 1

required:

- intermedTrainInfo

# *** End of Changes ***