3GPP TSG-CT Meeting #108	CP-251258

# Prague, Czech Republic; 09th – 10th June 2025						revision of C4-252340

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.581 | CR | 0052 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of In-session Unicast Repair of MBS Object Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED-CT |  |  |  |  |  | Date: |  |  | 2025-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA4 has defined in-session unicast repair of MBS Object Distribution and accordingy, related attributes are newly added in clause 4.5.6 of TS 26.502. Table 4.5.6 2 of TS 26.502 Parameter; Cardinality; Assigner; Description. …. Object repair exposure base URL; 0..1; MBSTF; An endpoint on the MBSTF from which objects are additionally made available to the MBS AS at reference point MBS-12 (NOTE 2). Present only when object repair is provisioned for this MBS Distribution Session with pull-based ingest by the MBS AS. This parameter is therefore mutually exclusive with Object repair ingest base URL.. Object repair ingest base URL; 0..1; MBS AS; An endpoint on the MBS AS to which objects ingested by the MBSTF are additionally pushed (NOTE 2). Present only when object repair is provisioned for this MBS Distribution Session with push-based ingest by the MBS AS. This parameter is therefore mutually exclusive with Object repair exposure base URL.. Thus, it is necessary to add the new parameters in TS 29.581. |
| Summary of change: |  | New attributes, objRepairExpBaseUrl and objRepairIngestBaseUrl, are added in Table 6.1.6.2.5-1: Definition of type ObjDistributionData. Nmbstf_DistSession API updated accrodingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not allignment with stage2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.5, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29581_Nmbstf_DistSession.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1. - update of API to add the "mutually exclusive" aspect Rev2 (CT#108). - Update of API: yaml correction of errors detected during CR implementation |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.5	Type: ObjDistributionData

Table 6.1.6.2.5-1: Definition of type ObjDistributionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objDistributionOperatingMode | ObjDistributionOperatingMode | M | 1 | Operating Mode for the Object distribution method |  |
| objAcquisitionMethod | ObjAcquisitionMethod | M | 1 | Indicates whether the objects(s) are to be pushed into the MBSTF by the MBS Application Provider or whether they are to be pulled from the MBS Application Provider by the MBSTF |  |
| objAcquisitionIdsPull | array(Uri) | C | 1..N | Represents the URL(s) (expressed as path(es) relative to the object ingest base URL provided in the "objIngestBaseUrl" attribute) pointing to the root object(s) to be pulled by the MBSTF and then distributed during this MBS Distribution Session. This attribute shall be provided, when available. See clause 6.1 of 3GPP TS 26.502 [17] (NOTE 2, NOTE 5) |  |
| objAcquisitionIdPush | Uri | C | 0..1 | Represents the URL (expressed as a path relative to the object ingest base URL provided in the "objIngestBaseUrl" attribute) pointing to the root object(s) to be pushed to the MBSTF and then distributed during this MBS Distribution Session. This attribute shall be provided, when available. See clause 6.1 of 3GPP TS 26.502 [17] (NOTE 2, NOTE 5) |  |
| objIngestBaseUrl | Uri | C | 0..1 | Represents the object ingest base URL. It contains a URL prefix that is replaced by the object distribution base URL by the MBSTF to derive the object distribution URI prior to the distribution of ingested objects. Shall be present if the object Distribution base URL (within the "objDistributionBaseUrl") is present. (NOTE 3, NOTE 4) |  |
| objDistributionBaseUrl | Uri | O | 0..1 | Represents the object distribution base URL. It contains a URL prefix with which the MBSTF replaces the object ingest base URL to derive the object distribution URL prior to the distribution of ingested objects. (NOTE 4) |  |
| objRepairExpBaseUrl | Uri | O | 0..1 | Represents the object repair exposure base URL. It contains an endpoint on the MBSTF from which objects are additionally made available to the MBS AS. This attribute may only be present when object repair is provisioned for this MBS Distribution Session with pull-based ingest by the MBS AS. (NOTE 6) |  |
| objRepairIngestBaseUrl | Uri | O | 0..1 | Represents the object repair ingest base URL. It contains an endpoint on the MBS AS to which objects ingested by the MBSTF are additionally pushed. This attribute may only be present when object repair is provisioned for this MBS Distribution Session with push-based ingest by the MBS AS. (NOTE 6) |  |
| NOTE 1: void. NOTE 2: Either the "objAcquisitionIdsPull" attribute or the "objAcquisitionIdsPush" attribute shall be provided, when available. NOTE 3: When the "objAcquisitionMethod" attribute is set to "PULL", this attribute may be provided by the MBSF during the creation and/or update/modification of the MBS Distribution Session (if received from the AF). When the "objAcquisitionMethod" attribute is set to "PUSH", this attribute may be provided by the MBSTF in the response to the creation and/or update/modification request of the MBS Distribution Session. NOTE 4: When the "objDistributionBaseUrl" attribute is omitted, nothing is replaced/removed from the object ingest URL when deriving the object distribution URL. NOTE 5: For group message delivery, the NEF transforms the group message payload into a file, so as an object to be ingested and distributed by the MBSTF using PULL or PUSH Method as specified in 3GPP TS 23.247 [15]. NOTE 6: "objRepairExpBaseUrl" attribute and "objRepairIngestBaseUrl" attribute are mutually exclusive as specified in 3GPP TS 26.502 [17]. |  |  |  |  |  |

* * * Next Change * * * *

# A.2	Nmbstf_DistSession API

openapi: 3.0.0

info:

title: Nmbstf-distsession

version: 1.1.0

description: |

MBSTF Distribution Session Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

ObjDistributionData:

description: Info for Object Distribution Method

type: object

properties:

objDistributionOperatingMode:

$ref: '#/components/schemas/ObjDistributionOperatingMode'

objAcquisitionMethod:

$ref: '#/components/schemas/ObjAcquisitionMethod'

objAcquisitionIdsPull:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

objAcquisitionIdPush:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objIngestBaseUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objDistributionBaseUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objRepairExpBaseUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objRepairIngestBaseUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- objDistributionOperatingMode

- objAcquisitionMethod

allOf:

- oneOf:

- required: [ objAcquisitionIdsPull ]

- required: [ objAcquisitionIdPush ]

- not:

required: [ objRepairExpBaseUrl, objRepairIngestBaseUrl ]

[…]

* * * End of Changes * * * *