3GPP TSG-CT Meeting #108	CP-251270

# Prague, Czech Republic; 09th – 10th June 2025

3GPP TSG-CT WG1 Meeting #155	C1-253720

# Bratislava, Slovakia, 19-23 May 2025

# Title:	Reply LS on Next Generation eCall

# Response to:	LS (C1-251558) on Next Generation eCall from CEN/TC 278/WG 15

# Release:	Release 19

# Work Item:	eCallCEN

Source:	CT1

To:	CEN/TC 278/WG 15

Cc:	SA, CT, SA2, SA1, ETSI TC MSG

Contact Person:

Name:	Osama Lotfallah

E-mail Address:	osamal@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C1-253772.zip

1. Overall Description:

3GPP CT1 thanks CEN/TC 278/WG 15 for their LS on Next Generation eCall regarding the alignment status of CEN cases 6 to 9. CT1 has re-analysed the differences between the requirements in the CEN specifications and the 3GPP specifications for these cases and would like to provide the following feedback:

- For last resort eCall triggered in a network that supports IMS emergency calls without ECL=T (case 6), the usage of a URI without an sos sub-type as specified in CEN specifications might result in emergency call setup failure in some countries (see Note 2 in clause 5.1.6.8.1 and several other clauses in TS 24.229). Therefore, an existing defined URI with an eCall sos sub-type in 3GPP specification is preferred;

- The detailed requirements for UE sending of MSD using audio media stream (e.g. cases 6 and 7) were not defined in 3GPP specifications previously. To avoid misinterpretation, we clarified this requirement and referred to CEN specifications for further details;

- 3GPP specifications do not fully define all possible redial attempts for pending voice, emergency or eCall (case 8). Switching from PS to CS domain as defined in 3GPP specifications is useful for implementation if the CS domain is available. To help facilitate UE redial requirements defined in CEN specifications, a note for implementation is added in 3GPP specification; and

- For case 9, any redial is left up to implementation – allowing an IVS to follow the CEN requirement as an option.

The attached CR shows the agreed changes.

2. Actions:

To CEN/TC 278/WG 15 group.

ACTION: 	CT1 asks CEN/TC 278/WG 15 to take the above information into account.

3. Date of Next CT1 Meetings:

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India