3GPP TSG-CT Meeting #108	CP-251288

# Prague, Czech Republic; 09th – 10th June 2025                                    (Revision of CP-251237)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0410 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ADRF UE RAT Connectivity Analytics - OpenAPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the SA6#64 meeting (S6-245596), UE RAT connectivity analytics for satellite has been agreed as part of TS 23.436. Hence, APIs provided by A-ADRF related to satellite access needs to be implemented in TS 29.549. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR has updated OpenAPI defintions to support SS_AADRF_UE_RAT_Connectivity_Analytics API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements are not implemented in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI. - TS29549_SS_AADRF_DataManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C3-252407 (Rev 2) is revised to fix the following Open API errors: - In UERatAnalyticsReq data type - Fix the Typo "obejct" - Correct the "area" attribute reference - Correct the "timeVal" attribute reference - In EventSubscription, include the missing "/" in reference. - In UERatAnalyticsInfo, include the correct reference. CP-251237 (Rev 3) is revised to fix the following in Open API: - add missing space for the required field of the UERatAnalyticsReq and UERatAnalyticsInfo data types |  |  |  |  |  |  |  |  |

# *** First Change ***

# A.22	SS_AADRF_DataManagement API

openapi: 3.0.0

info:

title: SS_AADRF_DataManagement

description: |

API for A-ADRF Data Management Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.1"

externalDocs:

description: >

3GPP TS 29.549 V18.7.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-aadrf-datamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create Individual A-ADRF Data Management Subscription.

operationId: CreateAADRFEventSubscription

tags:

- A-ADRF Data Management Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageSub'

responses:

'201':

description: >

The requested Individual A-ADRF Data Management Subscription is successfully created and

a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageNotification'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual A-ADRF Data Management Subscription resource.

required: true

schema:

type: string

delete:

summary: Remove the Individual A-ADRF Data Management Subscription.

operationId: UnsubscribeAADRFEventSubscription

tags:

- Individual A-ADRF Data Management Subscription (Document).

responses:

'204':

description: >

The individual A-ADRF Data Management Subscription resource matching the subscriptionId

is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DataManageSub:

description: Represents the event subscriptions.

type: object

properties:

eventSubscriptions:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubscriptions

- notifUri

EventSubscription:

description: Represents the event subscription.

type: object

properties:

event:

$ref: '#/components/schemas/AadrfEvent'

dataCollectReq:

type: string

dataProducerIds:

type: array

items:

type: string

minItems: 1

description: The list of Data Producer IDs.

valUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: The target VAL UE(s) identifiers.

valServerId:

type: string

description: The target VAL server ID.

valServiceId:

type: string

description: The VAL service ID.

profileCriteria:

type: string

validConds:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice(s) to which the subscription applies.

edgeReq:

$ref: '#/components/schemas/EdgeReq'

apiLogReq:

$ref: '#/components/schemas/ApiLogReq'

ueRatAnaReq:

$ref: '#/components/schemas/UERatAnalyticsReq'

required:

- event

EdgeReq:

description: Represents the EDGE data request requirement.

type: object

properties:

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

description: Identifiers the DN Access Identifier.

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

description: Identifiers the DN Access Identifier.

destEasId:

type: string

description: Identifier for the destination EAS.

destEesId:

type: string

description: Identifier for the destination EES.

anyOf:

- required: [dnais]

- required: [dnns]

- required: [destEasId]

- required: [destEesId]

ApiLogReq:

description: Represents the API log request requirement.

type: object

properties:

apiRequestorInfo:

type: string

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: Name of the API which was invoked.

apiVersion:

type: string

description: Version of the API which was invoked.

inputParameters:

type: string

description: List of input parameters.

result:

type: string

description: For HTTP protocol, it contains HTTP status code of the invocation.

apiInvokerId:

type: string

description: Identity of the API invoker which invoked the service API.

exposureLvl:

$ref: '#/components/schemas/ExposureLevel'

apiAggreInfo:

type: string

required:

- apiRequestorInfo

UERatAnalyticsReq:

description: Represents the UE RAT connectivity analytics requirements.

type: object

properties:

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

valUeAddrInfo:

type: array

items:

$ref: 'TS29549_SS_NetworkResourceAdaptation.yaml#/components/schemas/ValUeAddrInfo'

minItems: 1

required:

- valUeAddrInfo

DataManageNotification:

description: Represents the notification to the consumer.

type: object

properties:

event:

$ref: '#/components/schemas/AadrfEvent'

notifCorrId:

type: string

description: Notification correlation identifier.

valServerId:

type: string

description: The target VAL server ID.

valServiceId:

type: string

description: The VAL service ID.

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: The target VAL UE(s) identifiers.

locAccuracy:

type: string

validConds:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

apiLogs:

type: array

items:

$ref: '#/components/schemas/ApiLogInfo'

minItems: 1

description: The API logs information.

rttDeviation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice(s) to which the subscription applies.

edgeInfo:

$ref: '#/components/schemas/EdgeInfo'

ueRatAnaInfo:

$ref: '#/components/schemas/UERatAnalyticsInfo'

required:

- event

ApiLogInfo:

description: Represents the API log data.

type: object

properties:

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: Name of the API which was invoked.

failNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiAvailableInd:

type: boolean

description: Indicates the API availability.

apiVerChgNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiLocChg:

type: boolean

description: Indicates the API location changes for the target API.

apiThrottlingEvents:

type: string

invokeNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiVersion:

type: string

description: Version of the API which was invoked.

required:

- apiId

EdgeInfo:

description: Represents the EDGE related data.

type: object

properties:

edgeReq:

$ref: '#/components/schemas/EdgeReq'

output:

type: string

UERatAnalyticsInfo:

description: Represents the UE RAT Connectivity Analytics data information.

type: object

properties:

analyticsOut:

type: array

items:

$ref: 'TS29549_SS_ADAE_UeRatConnectivityAnalytics.yaml#/components/schemas/UeRatAnalyticsOutput'

minItems: 1

required:

- analyticsOut

# Simple data types and Enumerations

AadrfEvent:

anyOf:

- type: string

enum:

- HISTORICAL_SERVICEAPI

- NETWORK_SLICE

- EDGE_DATA

- LOCATION_ACCURACY

- UE_RAT_ANALYTICS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the subscribed events.

Possible values are:

- HISTORICAL_SERVICEAPI: The event for historical service API logs.

- NETWORK_SLICE: The event for the network slice data.

- EDGE_DATA: The event for the EDGE related data.

- LOCATION_ACCURACY: The event for the location accuracy data.

- UE_RAT_ANALYTICS: The event for the UE RAT Connectivity analytics data.

ExposureLevel:

anyOf:

- type: string

enum:

- READ

- WRITE

- DELETE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the exposure level.

Possible values are:

- READ: The exposure level for the logs to be exposed is read.

- WRITE: The exposure level for the logs to be exposed is write.

- DELETE: The exposure level for the logs to be exposed is delete.

# *** End of Changes ***