3GPP TSG-CT Meeting #109	CP-252213

# Beijing, P.R.China, 15-16 September 2025	(was C1-255687)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 6942 | rev | 4 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Collision of registration and MT SM procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If a registration procedure is ongoing, no SM procedure shall be initiated. While for the UE this is defined (see clause 6.1.1), a similar requirement is missing for the network. Rev-4 Update to CT #109: In the previous version (rev-3) at CT1 #156, in C1-255687, an incorrect CR was included. Instead of the 24.501 version of the CR (#6942), erroneously the 24.301 version of the CR (#4441 from C1-255686) was included. Thus, the tdoc numbers C1-255686 and C1-255687 were swapped at CT1 #156. Instead of C1-255686 including the 24.301 version of the CR, it included the 24.501 version…and C1-255687 ended up including the 24.301 version of the CR, instead of including the 24.501 version. The error has since been corrected in this revision to CT #109. There has been no change to the technical content of the CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a handover or inter-system handover to N1 is ongoing and the TA of the target cell is not included in the current registration area, the AMF shall suspend the transmission of 5GSM messages until the registration procedure has been successfully completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk that MT SM procedures which are initiated while a registration procedure is ongoing are not handled at all. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-4 Update to CT #109: In the previous version (rev-3) at CT1 #156, in C1-255687, an incorrect CR was included. Instead of the 24.501 version of the CR (#6942), erroneously the 24.301 version of the CR (#4441 from C1-255686) was included. Thus, the tdoc numbers C1-255686 and C1-255687 were swapped at CT1 #156. Instead of C1-255686 including the 24.301 version of the CR, it included the 24.501 version…and C1-255687 ended up including the 24.301 version of the CR, instead of including the 24.501 version. The error has since been corrected in this revision to CT #109. There has been no change to the technical content of the CR. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.2	Coordination between the protocols for 5GS mobility management and 5GS session management

A 5GS session management (5GSM) message is piggybacked in specific 5GS mobility management (5GMM) transport messages. To this purpose, the 5GSM messages can be transmitted in an information element in the 5GMM transport messages. In this case, the UE, the AMF and the SMF execute the 5GMM procedure and the 5GSM procedure in parallel. The success of the 5GMM procedure is not dependent on the success of the piggybacked 5GSM procedure.

The UE can only initiate the 5GSM procedure when there is a 5GMM context established at the UE.

During 5GMM procedures, the UE and the AMF shall suspend the transmission of 5GSM messages, except when:

a)	the 5GMM procedure is piggybacking 5GSM messages; or

b)	the UE is in 5GMM-CONNECTED mode and a service request procedure for re-establishing user-plane resources of PDU session(s) is initiated without including PDU session status IE or Allowed PDU session status IE. In this case, the UE and the AMF need not suspend the transmission of 5GSM messages related to other PDU session(s) than the one(s) for which the user- plane resources re-establishment is requested.

If a N1 mode to N1 mode handover or inter-system handover to N1 mode is ongoing and the TA of the target cell is not included in the current registration area, the AMF shall suspend the transmission of 5GSM messages until the registration procedure has been completed.

If the UE determines to locally release the N1 NAS signalling connection upon receiving an SOR transparent container during a registration procedure as specified in 3GPP TS 23.122 [5] subclause C.2, the UE shall suspend the transmission of 5GSM messages after sending the REGISTRATION COMPLETE message and until the N1 NAS signalling connection is released to obtain service on a higher priority PLMN, with the exception of the case when the UE has an emergency PDU session.

A 5GMM message piggybacking a 5GSM message for a PDU session shall be delivered via the access associated with the PDU session, if any, with the following exceptions:

a)	the AMF shall send, via 3GPP access, a DL NAS TRANSPORT message piggybacking a downlink 5GSM message of a network-requested 5GSM procedure for a PDU session associated with non-3GPP access if the conditions specified in subclause 5.5.1.3.4 or subclause 5.6.1.4 are met;

b)	the UE shall send an UL NAS TRANSPORT message piggybacking a response message to the 5GSM message described in a) via either:

1)	3GPP access; or

2)	non-3GPP access if the UE is in 5GMM-CONNECTED mode over non-3GPP access; and

NOTE:	The interaction between the 5GMM sublayer and the 5GSM sublayer to enable the UE to send the UL NAS TRANSPORT message containing the response message via 3GPP access is required. This is achieved via UE implementation.

c)	the UE shall send, via the target access, an UL NAS TRANSPORT message piggybacking a 5GSM message associated with a request type set to "existing PDU session" or "existing emergency PDU session" for handover of an existing PDU session between 3GPP access and non-3GPP access.

A 5GMM message piggybacking a 5GSM message as a response message to a request message associated with an MA PDU session, shall be delivered via the same access that the initial message was received.

* * * End of Changes * * * *