3GPP TSG CT WG3 Meeting #143	C3-254581

# Sophia-Antipolis, FR, 13th – 17th October, 2025											was C3-254391

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.569 | CR | 0006 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Various additional updates and corrections to the Naiotf_AIoT API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues were identified: - The AIoT Area related terminology needs to be aligned with the latest terms defined in clause 3.1 of TS 23.369, i.e., "AIoT Area", "External Area Identifier", "External Target Area" and "Target Area". Same for the "AIoT Device" terminology that needs to be made consistent everywhere within the specification as well. - The AIoT Inventory/Command request rejection cases are not fully addressed with regards to the stage 2 E2E procedures defined in clauses 6.2.2 and 6.2.3 of TS 23.369. Hereinafter some extracts from clause 6.2.2, since there are similar provisions in clause 6.2.3. 4. The AIOTF receives the Naiotf_AIoT_Inventory request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped. The AIOTF generates a Correlation ID corresponding to this AIoT service operation request, and is used for the AIOTF to correlate the service operation responses received from NG-RAN to the AIOTF request. The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier. The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code. The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request. - The AIOTF may perform AMF selection as described in clause 5.3.4. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of the related Ambient IoT requirements/provisions is not corrected/completed. |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.3.2, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.4, 6.1.6.2.6, 6.1.6.2.7, 6.1.6.2.8, 6.1.7.3, A.2 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces backwards compatible new feature to the OpenAPI descriptions of the following APIs: - TS29569_Naiotf_AIoT.yaml - TS29522_AIoT.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

##### 5.2.2.2.2	AIoT Inventory Request

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer requests to perform an AIoT Inventory operation to the AIOTF (see also clause 6.2.2 of 3GPP TS 23.369 [14]).

Figure 5.2.2.2.2-1: AIoT Inventory Request

1.	In order to request to perform an AIoT Inventory operation, the NF service consumer shall send an HTTP POST request message to the AIOTF targeting the URI of the corresponding custom operation (i.e., "InventoryRequest"), with the request body containing the InventoryReq data structure.

2a.	Upon reception of the Inventory request from the NF service consumer:

-	the AIOTF may perform the AF authorization for AIoT Services procedure as defined in clauses 5.6 and 6.2.2 of 3GPP TS 23.369 [14]; and

-	if the AF authorization for AIoT Services procedure is successful and upon successful processing of the request, the AIOTF shall respond to the NF service consumer with an HTTP "200 OK" status code to indicate that the AIoT Inventory request is successfully received and processed, with the response body containing AIoT Inventory related information within the InventoryResp data structure.

2b.	On failure, the AIOTF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code. In particular:

-	if AF authorization for AIoT Services procedure is not successful, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error;

-	if the target(s) of the AIoT Inventory request (e.g., target AIoT Device(s), filtering information) is/are not supported and/or not allowed, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AIOT_TARGETS_ERROR" application error;

-	if the provided time interval for results aggregation is invalid (e.g., the provided time interval is shorter than a locally configured minimum interval at the AIOTF as defined in clause 5.9 of 3GPP TS 23.369 [14]), the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "INVALID_AGGR_TIME_INVERTAVAL" application error; and

-	if the AIOTF fails to process the AIoT Inventory request (e.g., failure to select the NG-RAN(s) or RAN Reader(s) to handle the request), the AIOTF shall reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

* * * * Next changes * * * *

##### 5.2.2.3.2	AIoT Command Request

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer requests to perform an AIoT Command operation to the AIOTF (see also clause 6.2.3 of 3GPP TS 23.369 [14]).

Figure 5.2.2.3.2-1: AIoT Command Request

1.	In order to request to perform an AIoT Command operation, the NF service consumer shall send an HTTP POST request message to the AIOTF targeting the URI of the corresponding custom operation (i.e., "CommandRequest"), with the request body containing the CommandReq data structure.

2a.	Upon reception of the Command request from the NF service consumer:

-	the AIOTF may perform the AF authorization for AIoT Services procedure as defined in clauses 5.6 and 6.2.2 of 3GPP TS 23.369 [14]; and

-	if the AF authorization for AIoT Services procedure is successful and upon successful processing of the request, the AIOTF shall respond to the NF service consumer with an HTTP "200 OK" status code to indicate that the AIoT Command request is successfully received and processed, with the response body containing AIoT Command related information within the CommandResp data structure.

2b.	On failure, the AIOTF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code. In particular:

-	if AF authorization for AIoT Services procedure is not successful, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error;

-	if the target(s) of the AIoT Command request (e.g., target AIoT Device(s), filtering information) is/are not supported and/or not allowed, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AIOT_TARGETS_ERROR" application error; and

-	if the AIOTF fails to process the AIoT Command request (e.g., failure to select the NG-RAN(s) or RAN Reader(s) to handle the request), the AIOTF shall reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

* * * * Next changes * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Naiotf_AIoT service-based interface protocol.

Table 6.1.6.1-1: Naiotf_AIoT specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AIoTNotif | 6.1.6.2.6 | Represents the AIoT Operations Notification. |  |
| CommandReq | 6.1.6.2.4 | Represents the AIoT Command request. |  |
| CommandResp | 6.1.6.2.5 | Represents the AIoT Command response. |  |
| DevicesRepInfo | 6.1.6.2.8 | Represents the AIoT Device(s) related reporting information. |  |
| InventoryReq | 6.1.6.2.2 | Represents the AIoT Inventory request. |  |
| InventoryResp | 6.1.6.2.3 | Represents the AIoT Inventory response. |  |
| AIoTDevices | 6.1.6.2.7 | Represents the AIoT Device(s) related information. |  |

Table 6.1.6.1-2 specifies data types re-used by the Naiotf_AIoT service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naiotf_AIoT service-based interface.

Table 6.1.6.1-2: Naiotf_AIoT re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AiotArea | 3GPP TS 29.571 [16] | Represents the Target Area for AIoT. |  |
| AiotDevPermId | 3GPP TS 29.571 [16] | Represents the permanent identifier of the AIoT Device. |  |
| AiotFilteringInformation | 3GPP TS 29.571 [16] | Represents the the filtering information used for identifying the target AIoT Device(s). |  |
| Bytes | 3GPP TS 29.571 [16] | Represents a sequence of bytes. |  |
| CommandType | 3GPP TS 29.522 [15] | Represents the type of AIoT Command. |  |
| DurationSec | 3GPP TS 29.571 [16] | Represents a time duration in units of seconds. |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Represents error related information. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |

* * * * Next changes * * * *

##### 6.1.6.2.2	Type: InventoryReq

Table 6.1.6.2.2-1: Definition of type InventoryReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| targetArea | AiotArea | C | 0..1 | Contains the Target Area within which the requested Inventory operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT Device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT Device(s). |  |
| timeInterval | DurationSec | O | 0..1 | Contains the time interval to be used for results aggregation. |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Inventory operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.4	CommandReq

Table 6.1.6.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| targetArea | AiotArea | C | 0..1 | Contains the Target Area within which the requested Command operation shall apply. (NOTE) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT Device(s) related information. (NOTE) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT Device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.6	Type: AIoTNotif

Table 6.1.6.2.6-1: Definition of type AIoTNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transId | string | M | 1 | Contains the identifier of the AIoT transaction to which the notification is related. |  |
| devicesRepData | array(DevicesRepInfo) | C | 1..N | Contains the AIoT Device(s) related reporting information. (NOTE) |  |
| lastRepInd | boolean | C | 0..1 | Indicates that this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. - "true" indicates that this is the last report. This attribute shall be present only when this is the last reporting from the NF service consumer for the AIoT service operation identified by the "transId" attribute. When present, this attribute shall be set to "true". The presence of this attribute set to the value "false" is forbidden. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.7	Type: AIoTDevices

Table 6.1.6.2.7-1: Definition of type AIoTDevices

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| devices | array(AiotDevPermId) | C | 1..N | Contains the list of the permanent identifier(s) of the target AIoT Device(s). (NOTE) |  |
| filteringInfo | AiotFilteringInformation | C | 0..1 | Contains the filtering information used for identifying the target AIoT Device(s). (NOTE) |  |
| NOTE: These attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.8	Type: DevicesRepInfo

Table 6.1.6.2.8-1: Definition of type DevicesRepInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | AiotDevPermId | M | 1 | Contains the identifier of the AIoT Device to which the reporting information is related. |  |
| readCmdRep | Bytes | O | 0..1 | Contains the Read command specific report information for the AIoT Device identified by the "deviceId" attribute. This attribute may be present only if the reporting information is related to a Read command operation, i.e., the "commandType" attribute is set to "READ" in the corresponding AIoT Command service operation. |  |

* * * * Next changes * * * *

#### 6.1.7.3	Application Errors

The application errors defined for the Naiotf_AIoT service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| AF_NOT_AUTHORIZED | 403 Forbidden | The request for AIoT services is rejected because the AF is not authorized for the requested AIoT Services. |  |
| AIOT_TARGETS_ERROR | 403 Forbidden | The request for AIoT services is rejected because the target(s) of the AIoT request (e.g., target AIoT Device(s), filtering information) is/are not supported and/or not allowed. |  |
| INVALID_AGGR_TIME_INVERTAVAL | 403 Forbidden | The AIoT Inventory request is rejected because the provided aggregation time interval is invalid. |  |
| UNSPECIFIED_FAILURE | 500 Internal Server Error | The request for AIoT services is rejected because the AIOTF failed to process it for an unspecified reason. |  |

* * * * Next changes * * * *

# A.2	Naiotf_AIoT API

openapi: 3.0.0

info:

title: AIOTF Ambient IoT Service

version: 1.0.0-alpha.5

description: |

API for the Naiotf_AIoT Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.569 V2.0.0; Ambient IoT Function (AIOTF) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.569/

servers:

- url: '{apiRoot}/naiotf-aiot/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

paths:

/request-inv:

post:

summary: Request to perform an AIoT Inventory operation.

operationId: InventoryRequest

tags:

- AIoT Inventory Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:inventory

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryReq'

responses:

'200':

description: >

OK. The AIoT Inventory request is successfully received and processed, and

AIoT Inventory related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-cmd:

post:

summary: Request to perform an AIoT Command operation.

operationId: CommandRequest

tags:

- AIoT Command Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:command

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommandReq'

responses:

'200':

description: >

OK. The AIoT Command request is successfully received and processed, and

AIoT Command related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommandResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naiotf-aiot: >

Enables to access all the resources and custom operations of the Naiotf_AIoT API.

naiotf-aiot:inventory: >

Enables to access only the InventoryRequest custom operation (Naiotf_AIoT_Inventory

service operation) of the Naiotf_AIoT API.

naiotf-aiot:command: >

Enables to access only the CommandRequest custom operation (Naiotf_AIoT_Command

service operation) of the Naiotf_AIoT API.

schemas:

#

# STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: 'TS29522_AIoT.yaml#/components/schemas/CommandType'

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

transId:

type: string

devicesRepData:

type: array

items:

$ref: '#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

enum:

- true

description: >

Indicates that this is the last reporting from the AIOTF for the AIoT service

operation identified by the "transId" attribute.

true indicates that this is the last report.

This attribute shall be present only when this is the last reporting from the NF service

consumer for the AIoT service operation identified by the transId attribute.

When present, this attribute shall be set to true. The presence of this attribute set

to the value false is forbidden.

required:

- transId

anyOf:

- required: [devicesRepData]

- required: [lastRepInd]

AIoTDevices:

description: Represents the AIoT Device(s) related information.

type: object

properties:

devices:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

minItems: 1

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

oneOf:

- required: [devices]

- required: [filteringInfo]

DevicesRepInfo:

description: Represents the AIoT Device(s) related reporting information.

type: object

properties:

deviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

readCmdRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- deviceId

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

#

# Data types describing alternative data types or combinations of data types

#

* * * * End of changes * * * *