3GPP TSG-CT WG1 Meeting #157	C1-256786

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0830 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment for the information element containing the List of user info IDs and layer-2 IDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3, 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the reply LS S2-2508102 from SA2 and the corresponding stage-2 agreed CR S2-2508008, there is a need to add the list of Layer-2 IDs (associated with the corresponding User info IDs) in the Path information of the multi-hop U2N relay discovery and communication procedures based on model B discovery. Instead of introducing a new IE type for this purpose, there is already an existing IE type (since Rel-18, called "List of candidates U2U relay UE layer-2 IDs") that carries the List of Layer-2 IDs and the associated User info IDs of the UEs. Hence it is proposed to re-use this existing IE for the same purpose after making that existing IE more generic. Actually the name of the existing IE "List of candidates U2U relay UE layer-2 IDs" is misleading since the IE contains both User info IDs and Layer-2 IDs of the UEs, and not only the Layer-2 IDs. Additionally, the IE name should describe the basic building blocks of the IE (User Info IDs and Layer-2 IDs) rather than describing the functionality it is used for. Also having a single IE with the contents "User Info IDs and Layer-2 IDs" simplifies the protocol and enables future extendibility. Moreover, some corrections are needed in that IE, e.g. the minimum length value, which are also included in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1- Adjusting the "List of candidates U2U relay UE layer-2 IDs IE" to make it generic and reusable for all procedures. 2- Correcting the wrong information (the minimum length value of the IE). Backward compatibility analysis: The CR is not backward compatible because a UE that has implemented an older version of this specification may implement the minimum length of this IE as 5 which cannot be understood by the new version of UE that has implemented this CR. However this correction is mandatory since the minimum length of the IE practically can't be set to 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1- IE is not reusable, leading to complex specifications. 2- Wrong IE information (minimum length) stays in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.6.1, 10.3.6.11, 11.3.48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 10.3.6.1	Message definition

This message is sent by the UE to another peer UE to initiate the direct link modification procedure. See table 10.3.6.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION REQUEST

Significance:	dual

Direction:	UE to peer UE

Table 10.3.6.1.1: PROSE DIRECT LINK MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Link modification operation code | Link modification operation code 11.3.19 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 5-65537 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| 66 | Source end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 67 | Target end UE info | Application layer ID 11.3.4 | O | TLV | 3-257 |
| 2D | Target end UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4 |
| 33 | Source 5G ProSe layer-3 end UE MAC address | MAC address 11.3.45 | O | TLV | 8 |
| 9- | Relay reselection indication | Relay reselection indication 11.3.46 | O | TV | 1 |
| 2F | List of candidates U2U relay UE info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 30 | Source end UE IP address | UE IP address 11.3.49 | O | TLV | 7-19 |
| 31 | List of target end UE IP addresses | List of UE IP addresses 11.3.50 | O | TLV | 7-257 |
| 32 | List of candidates U2U relay UE layer-2 IDs | List of user info IDs and layer-2 IDs 11.3.48 | O | TLV | 12-257 |
| 50 | New MSBs of KNRP ID | MSBs of KNRP ID 2 11.3.52 | O | TLV | 4 |
| 70 | List of multi-hop remote UE parameters | List of multi-hop remote UE parameters 11.3.58 | O | TLV-E | 22-65538 |
| 10 | Child UEs multi-hop user info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 11 | End-to-end QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 12 | Multi-hop path info | List of user info ID 11.3.47 | O | TLV | 8-257 |

***** Next change *****

#### 10.3.6.11	List of candidates U2U relay UE layer-2 IDs

The UE may include this IE if the relay reselection is indicated to indicate a list of the candidates 5G ProSe UE-to-UE relay UE user info IDs and the associated layer-2 IDs.

***** Next change *****

### 11.3.48	List of user info IDs and layer-2 IDs

The list of user info IDs and layer-2 IDs information element contains the user info ID and the associated layer-2 ID of one or more UEs.

The list of user info IDs and layer-2 IDs information element is a type 4 information element with the minimum length of 12 octets and the maximum length of 257 length.

The list of user info IDs and layer-2 IDs information element is coded as shown in figure 11.3.48.1 and table 11.3.48.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of user info IDs and layer-2 IDs IEI |  |  |  |  |  |  |  | octet 1 |
| Length of list of user info IDs and layer-2 IDs contents |  |  |  |  |  |  |  | octet 2 |
| User info ID and layer-2 ID #1 |  |  |  |  |  |  |  | octet 3 octet u |
| User info ID and layer-2 ID #2 |  |  |  |  |  |  |  | octet (u+1)* octet v* |
| ... |  |  |  |  |  |  |  | octet (v+1)* octet w* |
| User info ID and layer-2 ID #n |  |  |  |  |  |  |  | octet (w+1)* octet x* |

Figure 11.3.48.1: List of user info IDs and  layer-2 IDs information element

Table 11.3.48.1: List of user info IDs and layer-2 IDs information element

User info ID and layer-2 ID (octet 3 to u) The user info ID and layer-2 ID field contains the user info ID of the UE with its associated layer-2 ID and is coded as in figure 11.3.48.2 and in table 11.3.48.2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of user info ID and layer-2 ID contents |  |  |  |  |  |  |  | octet 1 |
| User info ID |  |  |  |  |  |  |  | octet 2 octet 7 |
| Layer-2 ID |  |  |  |  |  |  |  | octet 8 octet 10 |

Figure 11.3.48.2: User info ID and layer-2 ID

Table 11.3.48.2: User info ID and layer-2 ID

User info ID (octet 2 to octet 7) The user info ID field contains the value of the user info ID of the UE and is coded as specified in figure 11.3.39.1 from the octet 3 and in table 11.3.39.1.

Layer-2 ID (octet 8 to 10) The layer-2 ID field contains the layer-2 ID of the UE and is coded as in figure 11.3.25.1 (octets 2 to 4) and in table 11.3.25.1.

***** End of changes *****