3GPP TSG-CT WG1 Meeting #157	C1-256790

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0842 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of EN for Additional Parameters Announcement Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As indicated in LS S2-2507872/C1-256069, SA2 decided to remove the RSC and Root Relay Info from the Additional Parameters Announcement Request message sent by the Remote UE and agreed S2-2507871 with following clarification: all the Layer-2 links between Remote UE and Intermediate U2N Relay and between Intermediate U2N Relay and U2N Relay are established per RSC. Therefore, the Additional Parameters Announcement Request sent by the Remote UE can be routed up to the U2N Relay through each Layer-2 link associated with the RSC without the RSC and Root Relay Info. Corresponding EN in clause 8b.2.8.2 is obsolete, and it is proposed to remove it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of EN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obsolete EN remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8b.2.8.2	5G ProSe additional parameters announcement procedure initiation by the 5G ProSe multi-hop layer-3 remote UE

Before initiating the 5G ProSe additional parameters announcement procedure for 5G ProSe multi-hop UE-to-network relay, a direct link shall be successfully established between the 5G ProSe multi-hop layer-3 remote UE and the 5G ProSe multi-hop layer-3 UE-to-network relay UE through zero or more 5G ProSe intermediate UE-to-network relay UE(s).

The 5G ProSe multi-hop layer-3 remote UE shall initiate a 5G ProSe additional parameters announcement procedure:

a)	when the 5G ProSe multi-hop layer-3 remote UE is triggered by an upper layer application to report NCGI or TAI of the serving cell, and:

1)	if the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information from the 5G ProSe multi-hop layer-3 UE-to-network relay UE cannot be received;

2)	if the NCGI or TAI is not included in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information from the 5G ProSe multi-hop layer-3 UE-to-network relay UE; or

3)	if the NCGI or TAI is not included in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response because the 5G ProSe multi-hop layer-3 UE-to-network relay discovery with model B is used for the direct link; or

b)	when the additional parameters announcement request refresh timer T5106 expires and the 5G ProSe multi-hop layer-3 remote UE still needs to obtain NCGI or TAI of the cell serving the 5G ProSe multi-hop layer-3 UE-to-network relay.

The 5G ProSe multi-hop layer-3 remote UE shall generate a PROSE ADDITIONAL PARAMETERS ANNOUNCEMENT REQUEST message and pass this message to the lower layers for transmission along with the 5G ProSe multi-hop layer-3 remote UE's layer-2 ID (for unicast communication) and the layer-2 ID of the next hop i.e., 5G ProSe intermediate UE-to-network relay UE or 5G ProSe multi-hop layer-3 UE-to-network relay UE for unicast communication.

Figure 8b.2.8.2.1: 5G ProSe additional parameters announcement procedure for 5G ProSe multi-hop layer-3 UE-to-network relay

NOTE 1:	The timer T5106 is started by the 5G ProSe multi-hop layer-3 remote UE.

NOTE 2:	The timer T5107 is started by the 5G ProSe layer-3 multi-hop UE-to-network relay UE.

* * * End of Changes * * * *