3GPP TSG-CT WG1 Meeting #157	C1-256789

# Sophia Antipolis, France 13 – 17 October 2025	was C1-256283

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0848 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of source layer-2 ID of discoverer end UE in path information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the explanation in reply LS S2-2508102, "For Model B discovery, similar to the single hop 5G ProSe UE-to-UE Relay operations (as specified in TS 23.304 clause 5.8.4), the User Info of the 5G ProSe End UE is expected to be protected in the Direct Discovery set. Thus, as the User Info ID of the source end UE would not be visible to the U2U relays, source end UE’s first 5G Prose layer-3 multi-hop UE-to-UE relay need the source-layer 2 ID in order to direct the response message to that particular source end UE in a unicast manner. This was discussed and approved in 23.304 CR 0508 S2-2412612.", source layer-2 ID of discoverer end UE needs to be included in path information. Both clause 10.2.19 and clause 10.2.20 specify List of multi-hop UE-to-UE relay paths IE. And clause 10.2.20 is less complete than clause 10.2.19. Hence, it proposes to remove clause 10.2.20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add source layer-2 ID of the discoverer end UE in the List of multi-hop UE-to-UE relay paths IE. Void clause 10.2.20. Remove the relevant editor's note in clause 11.2.24. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification with unresolved editor's note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8c.2.1.3.3.3, 10.2.20, 11.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 8c.2.1.3.3.3	Discoverer relay UE procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery

For Ethernet and unstructured data unit type, the procedure specified in clause 8c.2.1.2.3.3 applies with following differences and clarification:

a)	if the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type is the same as the relay service code parameter configured as specified in clause 5.2.9 for the connectivity service, the discoverer relay UE determines to send the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type upon receipt of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type from the source 5G ProSe multi-hop layer-3 end UE or other 5G ProSe multi-hop layer-3 UE-to-UE relay UE.

For the received List of multi-hop UE-to-UE relay paths IE, if:

1)	the value of the hop count field is less than the hop limit as described in clause 6.3.2.6.3.2 of 3GPP TS 23.304 [2]; and

2)	the user info ID of the discoverer relay UE is not listed in the "Path of relays" field, if any,

the UE shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type and set the PROSE PC5 DISCOVERY message as specified in clause 8c.2.1.3.3.2.2 with following differences:

i)	shall update received List of multi-hop UE-to-UE relay paths IE as follows:

-	the Hop count field is increased by 1;

-	add the user info ID of the discoverer relay UE to the "Path of relays" as described in table 11.2.24.2;

-	update the value of the length of the path of relays field accordingly;

-	if the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type is from the source 5G ProSe multi-hop layer-3 end UE, the UE shall set the SL2I bit to "Source layer-2 ID of the discoverer end UE is present" and include the source layer-2 ID of the the source 5G ProSe multi-hop layer-3 end UE in the "Source layer-2 ID of the discoverer end UE" field; and

ii)	shall include the updated List of multi-hop UE-to-UE relay paths IE in the PROSE PC5 DISCOVERY message.

otherwise, UE shall drop the received PROSE PC5 DISCOVERY message;

b)	if the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type, and the user info ID of the discoverer relay UE is listed in the "Path of relays" field, the UE shall forward PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type as specified in 6.3.2.6.3.2 of 3GPP TS 23.304 [2].

The UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages for multi-hop layer-3 UE-to-UE relay discovery response from the discoveree end UE or other discoverer relay UE.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type, for the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF, the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type.

If the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit,

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type. In the PROSE PC5 DISCOVERY message, the UE:

1)	shall set the List of multi-hop UE-to-UE relay paths IE to the List of multi-hop UE-to-UE relay paths as included in the received PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response;

2)	shall set the Discoveree info to the configured user info ID for the 5G ProSe multi-hop layer-3 UE-to-UE relay discovery, as specified in clause 5.2.9;

3)	shall set the Discoverer info to the Discoverer info as included in the received PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response;

4)	shall set the relay service code parameter to the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type;

5)	shall include the MIC field computed as described in 3GPP TS 33.503 [34];

6)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter; and

7)	shall set the ProSe direct discovery PC5 message type parameter and the ProSe direct discovery PC5 message content type extensions as specified in table 10.2.1.24;

c)	shall apply the DUIK, DUSK, or DUCK with the associated encrypted bitmask for the relay service code for 5G ProSe multi-hop layer-3 UE-to-UE relay discovery, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more of the above parameters, as specified in 3GPP TS 33.503 [34]; and

d)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

* * * Next Change * * * *

### 10.2.20	void

* * * Next Change * * * *

### 11.2.24	List of multi-hop UE-to-UE relay paths

The purpose of the list of multi-hop UE-to-UE relay paths information element is to carry the direct discovery set as specified in 3GPP TS 33.503 [34].

The list of multi-hop UE-to-UE relay paths information element is a type 6 information element.

The list of multi-hop UE-to-UE relay paths information element is coded as shown in figure 11.2.24.1 and table 11.2.24.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of multi-hop UE-to-UE relay paths IEI |  |  |  |  |  |  |  | octet 1 |
| Length of list of multi-hop UE-to-UE relay paths contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Path info #1 |  |  |  |  |  |  |  | octet 4 octet a |
| Path info #2 |  |  |  |  |  |  |  | octet (a+1)* octet b |
| …… |  |  |  |  |  |  |  | octet (b+1)* octet c |
| Path info #n |  |  |  |  |  |  |  | octet (c+1)* octet d |

Figure 11.2.24.1: List of multi-hop UE-to-UE relay paths information element

Table 11.2.24.1: List of multi-hop UE-to-UE relay paths information element

Path info (octet 3 to a) The path info is coded as in figure 11.2.24.2 and in table 11.2.24.2. Each path info represents the path information to one or more 5G ProSe multi-hop layer-3 end UE(s).

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hop count |  |  |  | Spare |  |  | SL2I | HL | octet 4 |
| Hop limit |  |  |  |  |  |  |  |  | octet 5* |
| Source layer-2 ID of the discoverer end UE |  |  |  |  |  |  |  |  | octet 6* octet 8* |
| Length of the list of protected user infos |  |  |  |  |  |  |  |  | octet 9 octet 10 |
| List of protected user infos |  |  |  |  |  |  |  |  | octet 11* octet m* |
| Length of the path of relays |  |  |  |  |  |  |  |  | octet m+1* octet m+2* |
| Path of relays |  |  |  |  |  |  |  |  | octet m+3* octet n* |

Figure 11.2.24.2: Path info

Table 11.2.24.2: Path info

| HL (bit 1 of octet 4) |  |
| --- | --- |
| Bit 1 |  |
| 0 | Hop limit field is not present |
| 1 | Hop limit field is present |
| SL2I (bit 2 of octet 4) |  |
| 0 | Source layer-2 ID of the discoverer end UE is not present |
| 1 | Source layer-2 ID of the discoverer end UE is present |
| Hop count (bit 5 to 8 of octet 4) The hop count field indicates the number of hops that the message is already relayed. |  |
| Source layer-2 ID of the discoverer UE (octet 6 to 8) |  |
| This field indicates the source layer-2 ID of the discoverer end UE. |  |
| List of protected user infos (octet 8 to m) The list of protected user infos field contains the protected user info(s) of the 5G ProSe multi-hop layer-3 end UE(s) that can be reached via the path of relays as indicated in "Path of relays" field (octet m+1 to n) and is coded as in figure 11.2.17.1 starting from octet 4. |  |
| Path of relays (octet m+3 to n) The path of relays field indicates is a list of intermediate 5G ProSe multi-hop layer-3 UE-to-UE relays that need to be traversed to reach the 5G ProSe multi-hop layer-3 end UE(s) as indicated in list of protected user infos field (octet 8 to m) and is coded as in figure 11.3.47.1 starting from octet 3. The order of the list starts from the current 5G ProSe multi-hop layer-3 UE-to-UE relay and ends with the last hop before reaching the 5G ProSe multi-hop layer-3 end UE(s). |  |

* * * End of Changes * * * *