3GPP TSG-CT WG1 Meeting #157	C1-256288

# Sophia Antipolis, France 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0853 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of multi-hop U2N relay and U2U relay in PKMF address request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 5G PKMF address request procedure description is lack of support for 5G ProSe multi-hop UE-to-network relay and the 5G ProSe multi-hop layer-3 UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of 5G ProSe multi-hop UE-to-network relay and the 5G ProSe multi-hop layer-3 UE-to-UE relay in 5G PKMF address request procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.17.1, 6.2.17.3, 6.2.17.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.17.1	General

The purpose of the 5G PKMF address request procedure is to allow a UE to obtain a 5G PKMF address from the 5G DDNMF in HPLMN or subscribed SNPN over PC3a interface as specified in 3GPP TS 33.503 [34] for the security procedure over user plane. A UE may initiate a 5G PKMF address request procedure:

a)	when the UE needs to obtain a 5G PKMF address from the 5G DDNMF in HPLMN or subscribed SNPN on demand; or

b)	when the UE can not access the 5G PKMF using the 5G PKMF address provisioned by the network as specified in clause 5.2.5.

To initiate a 5G PKMF address request procedure, the UE shall be configured with the authorized parameters for 5G ProSe UE-to-network relay as specified in clause 5.2.5, the authorized parameters for 5G ProSe UE-to-UE relay as specified in clause 5.2.7, the authorized parameters for 5G ProSe multi-hop UE-to-network relay as specified in clause 5.2.8, or the authorized parameters for 5G ProSe multi-hop layer-3 UE-to-UE relay as specified in clause 5.2.9. Both the 5G ProSe remote UE and the 5G ProSe UE-to-network relay UE are allowed to initiate the 5G PKMF address request procedure for 5G ProSe UE-to-network relay. Both the 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE are allowed to initiate the 5G PKMF address request procedure for 5G ProSe UE-to-UE relay. Both the 5G ProSe multi-hop remote UE and the 5G ProSe multi-hop UE-to-network relay UE are allowed to initiate the 5G PKMF address request procedure for 5G ProSe multi-hop UE-to-network relay. Both the 5G ProSe end UE and the 5G ProSe multi-hop layer-3 UE-to-UE relay UE are allowed to initiate the 5G PKMF address request procedure for 5G ProSe multi-hop layer-3 UE-to-UE relay.

NOTE:	The UE is allowed to access the 5G PKMF directly without initiating a 5G PKMF address request procedure if it is pre-configured with the 5G PKMF address as specified in clause 5.2.5, clause 5.2.7, clause 5.2.8 and clause 5.2.9.

* * * Next Change * * * *

#### 6.2.17.3	5G PKMF address request procedure accepted by the 5G DDNMF

Upon receiving a PROSE_5GPKMF_ADDRESS_REQUEST message, the 5G DDNMF shall check whether the UE is authorized for 5G ProSe UE-to-network relay, 5G ProSe UE-to-UE relay, 5G ProSe multi-hop UE-to-network relay, or 5G ProSe multi-hop layer-3 UE-to-UE relay. If authorized, the 5G DDNMF shall then send a PROSE_5GPKMF_ADDRESS_RESPONSE message with the <PKMF-address-response> element. In the <PKMF-address-response> element, the 5G DDNMF shall include:

a)	the transaction ID set to the value of the transaction ID received in the PROSE_5GPKMF_ADDRESS_REQUEST message from the UE; and

b)	the PKMF address set to the value of the 5G PMKF address in the HPLMN or subscribed SNPN of the UE.

* * * Next Change * * * *

#### 6.2.17.5	5G PKMF address request procedure not accepted by the 5G DDNMF

If the PROSE_5GPKMF_ADDRESS_REQUEST message cannot be accepted by the 5G DDNMF, the 5G DDNMF sends a PROSE_5GPKMF_ADDRESS_RESPONSE message containing a <PKMF-address-reject> element to the UE including an appropriate PC3a control protocol cause value and including the transaction ID set to the value of the transaction ID received in the PROSE_5GPKMF_ADDRESS_REQUEST message.

Upon receipt of the PROSE_5GPKMF_ADDRESS_RESPONSE message with the <PKMF-address-reject> element, if the transaction ID matches the value sent by the UE in a PROSE_5GPKMF_ADDRESS_REQUEST message, the UE shall consider the 5G PKMF address request procedure as rejected.

If the UE is not authorized for 5G ProSe UE-to-network relay, 5G ProSe UE-to-UE relay, 5G ProSe multi-hop UE-to-network relay, or 5G ProSe multi-hop layer-3 UE-to-UE relay, the 5G DDNMF shall send the PROSE_5GPKMF_ADDRESS_RESPONSE message containing a <PKMF-address-reject> element with PC3a control protocol cause value #3 "UE authorization failure".

* * * End of Changes * * * *