3GPP TSG-CT WG1 Meeting #157	C1-256706

Sophia-Antipolis, France 13-17 October 2025	was C1-xxxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6752 | rev | 1 | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the parameter for satellite id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to CR1656(S2-2507635),Npcf_PolicyAuthorization_Notify and Nsmf_EventExposure_Notify include only the DNAI in UP path change notifications, which is derived from the satellite identifier, rather than sending the satellite identifier redundantly. P-CSCF derives the satellite ID from the DNAI. DNAI is not defined in TS 24.229 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the term of DNAI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing defination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.1	Terms

For the purposes of the present document, the following terms apply.

3GPP PS data off status: indicates state of usage of the 3GPP PS data off. 3GPP PS data off status at the UE can be either "active" or "inactive".

Country: For the purposes of emergency service URNs in the present document, i.e. a service URN with a top-level service type of "sos" as specified in RFC 5031 [69], an ISO 3166-1 alpha-2 code as specified in ISO 3166-1 [207] is used to identify a region or a country.

Entry point: In the case that "border control concepts", as specified in 3GPP TS 23.228 [7], are to be applied in an IM CN subsystem, then these are to be provided by capabilities within the IBCF, and the IBCF acts as an entry point for this network (instead of the I-CSCF). In this case the IBCF and the I-CSCF can be co-located as a single physical node. If "border control concepts" are not applied, then the I-CSCF is considered as an entry point of a network. If the P-CSCF is in the home network, then the I-CSCF is considered as an entry point for this document. Similary, in case that "border control concepts", as specified in 3GPP TS 23.218 [5], are to be applied in an ISC interface, then these are to be provided by capabilities within the ISC gateway function, and the ISC gateway function acts as an entry point for this network.

Exit point: If operator preference requires the application of "border control concepts" as specified in 3GPP TS 23.228 [7], then these are to be provided by capabilities within the IBCF, and requests sent towards another network are routed via a local network exit point (IBCF), which will then forward the request to the other network (discovering the entry point if necessary). Similary, in case that "border control concepts", as specified in 3GPP TS 23.218 [5], are to be applied in an ISC interface, then these are to be provided by capabilities within the ISC gateway function, and requests sent towards another network are routed via a local network exit point (ISC gateway function).

Geo-local number: Either a geo-local service number as specified in 3GPP TS 23.228 [7] or a number in non-international format according to an addressing plan used at the current physical location of the user.

Home-local number: Either a home local service number as specified in 3GPP TS 23.228 [7] or a number in non-international format according to an addressing plan used in the home network of the user.

Main URI: In the case that the UE supports RFC 6140 [191] and performs the functions of an external attached network, the main URI is the URI which is used for the registration procedures in the To header of the REGISTER request as specified in RFC 6140 [191]; it represents the public user identities associated to that UE.

Newly established set of security associations: Two pairs of IPsec security associations that have been created at the UE and/or the P-CSCF after the 200 (OK) response to a REGISTER request was received.

Old set of security associations: Two pairs of IPsec security associations still in existence after another set of security associations has been established due to a successful authentication procedure.

Temporary set of security associations: Two pairs of IPsec security associations that have been created at the UE and/or the P-CSCF, after an authentication challenge within a 401 (Unauthorized) response to a REGISTER request was received. The SIP level lifetime of such created security associations will be equal to the value of reg-await-auth timer.

Integrity protected: See 3GPP TS 33.203 [19]. Where a requirement exists to send information "integrity-protected" the mechanisms specified in 3GPP TS 33.203 [19] are used for sending the information. Where a requirement exists to check that information was received "integrity-protected", then the information received is checked for compliance with the procedures as specified in 3GPP TS 33.203 [19].

Instance ID: An URN generated by the device that uniquely identifies a specific device amongst all other devices, and does not contain any information pertaining to the user (e.g., in GPRS instance ID applies to the Mobile Equipment rather than the UICC). The public user identity together with the instance ID uniquely identifies a specific UA instance. If the device has an IMEI available, it generates an instance ID based on its IMEI as defined in 3GPP TS 23.003 [3] clause 13. If the device has an MEID as defined in 3GPP2 S.R0048-A [86F] available, it generates an instance ID based on its MEID as defined in RFC 8464 [187]. If the device does not have an IMEI available and does not have an MEID available, the instance ID is generated as a string representation of a UUID as a URN as defined in RFC 9562 [154].

Resource reservation: Mechanism for reserving bearer resources that is required for certain access technologies.

Local preconditions: The indication of segmented status preconditions for the local reservation of resources as specified in RFC 3312 [30].

Alias URI, Alias SIP URI: A URI is an alias of another URI if the treatment of both URIs is identical, i.e. both URIs belong to the same set of implicitly registered public user identities, and are linked to the same service profile, and are considered to have the exact same service configuration for each and every service.

NOTE:	The S-CSCF recognizes that a given URI is an alias of another URI using the grouping sent from the HSS (see 3GPP TS 29.228 [14]).

Globally Routeable SIP URI: a SIP URI of which the hostname part can be resolved to the IP address of the entry entity of the network reponsible for the identity represented by the userpart.

Initial registration: The registration procedure for a public user identity initiated by the UE in the absence of any valid registration.

Registration expiration interval: An indication on how long a registration is valid, indicated using the Expires header field, or the "expires" header field parameter within the Contact header field, according to the procedures specified in RFC 3261 [26].

Reregistration: The registration procedure initiated by the UE to refresh or update an already existing registration for a public user identity.

Registration of an additional public user identity: The registration procedure initiated by the UE to explicitly register an additional public user identity during the life time of the registration of another registered public user identity, where both public user identities have the same contact address and P-CSCF.

Emergency registration: A special registration that relates to binding of a public user identity to a contact address used for emergency service.

Initial emergency registration: An emergency registration that is also an initial registration.

Emergency reregistration: An emergency registration that is also a reregistration.

Back-to-Back User Agent (B2BUA): As given in RFC 3261 [26]. In addition, for the usage in the IM CN subsystem, a SIP element being able to handle a collection of "n" User Agents (behaving each one as UAC and UAS, according to SIP rules), which are linked by some application logic that is fully independent of the SIP rules.

UE private IP address: It is assumed that the NAT device performs network address translation between a private and a public network with the UE located in the private network and the IM CN subsystem in the public network. The UE is assumed to be configured with a private IP address. This address will be denoted as UE private IP address.

UE public IP address: The NAT device is assumed to be configured with one (or perhaps more) public address(es). When the UE sends a request towards the public network, the NAT replaces the source address in the IP header of the packet, which contains the UE private IP address, with a public IP addressed assigned to the NAT. This address will be denoted as UE public IP address.

Encapsulating UDP header: For the purpose of performing UDP encapsulation according to RFC 3948 [63A] each IPsec ESP packet is wrapped into an additional UDP header. This header is denoted as Encapsulating UDP header.

Port_Uenc: In most residential scenarios, when the NAT device performs address translation, it also performs translation of the source port found in the transport layer (TCP/UDP) headers. Following RFC 3948 [63A], the UE will use port 4500 as source port in the encapsulating UDP header when sending a packet. This port is translated by the NAT into an arbitrarily chosen port number which is denoted as port_Uenc.

Multiple registrations: An additional capability of the UE, P-CSCF and S-CSCF, such that the UE (as identified by the private user identity and instance-id), can create multiple simultaneous registration bindings (flows), associated with one or more contact addresses, to any public user identity, Without this capability, a new registration from the UE for a public user identity replaces the existing registration binding, rather than merely creating an additional binding.

IMS flow set: An IMS flow set is a set of flows as defined in RFC 5626 [92]. The flows in an IMS flow set are determined by a combination of transport protocol, IP addresses, and ports. An IMS flow set is established by a successful IMS registration procedure.

NOTE:	For IPsec, the ports associated with the flow set include protected client ports and protected server ports as defined in 3GPP TS 33.203 [19] and an IMS flow set is made up of the following four flows:

-	Flow 1: (IP address UE, port_uc) <--> (IP address P-CSCF, port_ps) over TCP;

-	Flow 2: (IP address UE, port_uc) <--> (IP address P-CSCF, port_ps) over UDP;

-	Flow 3: (IP address UE, port_us) <--> (IP address P-CSCF, port_pc) over TCP; and

-	Flow 4: (IP address UE, port_us) <--> (IP address P-CSCF, port_pc) over UDP.

NOTE 3:	For IPsec, according to 3GPP TS 33.203 [19], the P-CSCF can only select among flows 3 or 4 when forwarding requests towards the UE. According to 3GPP TS 33.203 [19], flow 2 is only used for UE generated requests and responses. The P-CSCF uses flow 2 to identify the correct IMS flow set.

NOTE 4:	An IMS flow set can be considered as a realisation of a logical flow as used in RFC 5626 [92]. But this definition does not depend on any particular definition of a logical flow.

NOTE 5:	For TLS, the ports associated with the flow set include a protected client port and a protected server port and an IMS flow set is made up of the following flow:

-	(IP address UE, port) <--> (IP address P-CSCF, port) over TCP.

NOTE 6:	For SIP digest without TLS, an IMS flow set is as defined in RFC 5626 [92].

IMS flow token: A IMS flow token is uniquely associated with a IMS flow set. When forwarding a request destined towards the UE, the P-CSCF selects the flow from the IMS flow set denoted by the IMS flow token as appropriate according to 3GPP TS 33.203 [19] and RFC 3261 [26].

IMS registration related signalling: Signalling carrying the SIP messages sent by the UE and the network during the IMS registration procedure, i.e., SIP REGISTER request, SIP SUBSCRIBE request, and their responses.

IP Association: A mapping at the P-CSCF of a UE's packet source IP address, the "sent-by" parameter in the Via header field, and, conditionally, the port with the identities of the UE. This association corresponds to the IP address check table specified in 3GPP TS 33.203 [19].

Authorised Resource-Priority header field: a Resource-Priority header field that is either received from another entity in the trust domain relating to the Resource-Priority header field, or which has been identified as generated by a subscriber known to have such priority privileges for the resource priority namespace and level of priority used within that namespace.

Temporarily authorised Resource-Priority header field: a Resource Priority header field that has been temporarily approved by the P-CSCF, the S-CSCF, or an IBCF. Temporarily authorised Resource-Priority heaer field appears in an INVITE request only, and is applied only in the direction P-CSCF to S-CSCF to AS, S-CSCF to AS, or IBCF to S-CSCF to AS, for the request, and the reverse direction for 1xx responses to that request. Subsequent requests in the same dialog will require an authorised Resource-Priority header field in order to obtain priority privileges. It is only valid when all entities are in the same trust domain for the Resource-Priority header field.

Network-initiated resource reservation: A mechanism of resource reservation where the IP-CAN on the behalf of network initiates the resources to the UE.

Trace depth: When SIP signalling is logged for debugging purposes, trace depth is the level of detail of what is logged.

P-CSCF restoration procedures: the procedures for the IP-CAN and the UE to handle P-CSCF service interruption scenarios (see 3GPP TS 23.380 [7D]).

HSS based P-CSCF restoration procedures: the procedures for the IP-CAN, the IM CN subsystem, the HSS and the UE to handle P-CSCF service interruption scenarios (see 3GPP TS 23.380 [7D]). In 5GS the procedure is called UDM/HSS based P-CSCF restoration (see 3GPP TS 23.380 [7D]) since the UDM participates in the procedure.

PCRF based P-CSCF restoration procedures: the procedures for the IP-CAN, the IM CN subsystem, the PCRF and the UE to handle P-CSCF service interruption scenarios (see 3GPP TS 23.380 [7D]). In 5GS the procedure is called PCF based P-CSCF restoration (see 3GPP TS 23.380 [7D]) since the PCF takes the role of the PCRF.

Public network traffic: traffic sent to the IM CN subsystem for processing according to normal rules of the NGN. This type of traffic is known as public network traffic.

Private network traffic: traffic sent to the IM CN subsystem for processing according to an agreed set of rules specific to an enterprise. This type of traffic is known as private network traffic. Private network traffic is normally within a single enterprise, but private network traffic can also exist between two different enterprises if not precluded for regulatory reasons.

NOTE 7:	An IP-PBX or application functionality within the IM CN subsystem can change private network traffic to public network traffic and vice versa, by functionality known as "breakout" or "breakin" to the private network. As such a SIP transaction can be variously private network traffic and public network traffic on different hops across a SIP network.

Privileged sender: A privileged sender is allowed to send SIP messages where the identities in P-Asserted-Identity will be passed on in the P-CSCF and are not subject to further processing in the P-CSCF.

S-CSCF restoration procedures: the procedures for the IM CN subsystem and the UE to handle S-CSCF service interruption scenarios (see 3GPP TS 23.380 [7D]).

Loopback routeing: A method of routeing a SIP request back to the visited network for local breakout according to the roaming architecture for voice over IMS with local breakout as specified in 3GPP TS 23.228 [7].

UE performing the functions of an external attached network: an independent network connected to an IMS network over the Gm interface, through a single point and which is seen by the IMS network as a specific UE; e.g. an IP-PBX.

Static Mode of Operation: a mode of operation where the UE performing the functions of an external attached network does not initiate any IMS level registration procedures towards the operator IMS.

Canonical form of a SIP URI: Canoncial form of a SIP URI takes the form "sip:username@domain" as specified in RFC 3261 [26] subclause 10.3. SIP URI comparisons are performed as defined in RFC 3261 [26] subclause 19.1.4.

Originating home network: the home network of a user originating a transaction, and if applicable, the associated dialog.

Originating visited network: the visited network of a user originating a transaction, and if applicable, the associated dialog.

Terminating home network: the home network of a user terminating a transaction, and if applicable, the associated dialog.

Terminating visited network: the visited network of a user terminating a transaction, and if applicable, the associated dialog.

Type of emergency service: The type of emergency service is either an emergency call type standardized by 3GPP (see 3GPP TS 22.101 [8] subclause 10.1) or a similar capability not standardised by 3GPP and defined by national regulatory requirements. The generic (sos) service, identified by urn:service:sos, does not have a type of emergency service (even though usage of the generic (sos) service in the emergency call is defined).

Resource sharing: one dedicated EPS bearer is sharing resources among several ongoing sessions such that the highest GBR (and optionally MBR) to be shared for the set of PCC/QoS rules bound to the same bearer is used as input for the calculation of the GBR (and optionally MBR) of that bearer among the sessions sharing the resources.

Fully-Qualified Domain Name (FQDN): the syntax of the FQDN used in this specification is defined in RFC 3261 [26] subclause 25.1.

Trusted WLAN: A trusted non-3GPP access, where the non-3GPP access is a WLAN IP access.

Untrusted WLAN: An untrusted non-3GPP access, where the non-3GPP access is a WLAN IP access.

Calling number verification status determination: A feature which enables the terminating UE to determine whether number has been verified by the network as specified in RFC 8224 [252].

Calling number verification using signature verification and attestation information: A feature which enables a calling identity validation as specified in RFC 8224 [252] and uses an attestation information to vouch for the accuracy of the source of origin of the call. Attestation information consists of an attestation level and an origination identifier and may be included in the Identity header field as defined in RFC 8588 [261] and in the Attestation-Info and Origination-Id header fields as defined in subclauses 7.2.18 and 7.2.19.

Rich Call Data (RCD) info: It refers to the RCD information and RCD URL.

RCD verification using assertion of RCD info: A functionality which enables validation of RCD info as specified in RFC 9795 [302]. The RCD info may be provided in the Call-Info header field as specified in RFC 9796 [303]. As specified in RFC 9795 [302] the Identity header field is used for the purpose of authentication of the Call-info header field containing RCD info. The RCD verification using assertion of RCD info functionality can be implemented in two ways:

1)	within the Calling number verification using signature verification and attestation information feature which can be enhanced to authenticate and verify RCD info using the PASSporT SHAKEN JSON Web Token specified in RFC 8588 [261] with rcd PASSporT claims in accordance with RFC 9795 [302]; or

2)	if the Calling number verification using signature verification and attestation information feature is not supported, a standalone feature.

Priority verification using assertion of priority information: A feature which enables validation of a priority level provided in the Resource-Priority header field as specified in RFC 8443 [279] and, by extension for emergency sessions, the header field value "psap-callback" provided in the Priority header field as specified in RFC 9027 [278]. As specified in RFC 8443 [279] the Identity header field is used for the purpose of authentication of the Resource-Priority header field and, by extension for emergency sessions, the Priority header field value "psap-callback".

For the purposes of the present document, the following terms and definitions given in RFC 3261 [26] apply (unless otherwise specified see clause 6).

Client

Dialog

Final response

Header

Header field

Loose routeing

Method

Option-tag (see RFC 3261 [26] subclause 19.2)

Provisional response

Proxy, proxy server

Recursion

Redirect server

Registrar

Request

Response

Server

Session

(SIP) transaction

Stateful proxy

Stateless proxy

Status-code (see RFC 3261 [26] subclause 7.2)

Tag (see RFC 3261 [26] subclause 19.3)

Target Refresh Request

User agent client (UAC)

User agent server (UAS)

User agent (UA)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.002 [2] subclause 4.1.1.1 and subclause 4a.7 apply:

3GPP AAA proxy

3GPP AAA server

Breakout Gateway Control Function (BGCF)

Call Session Control Function (CSCF)

Home Subscriber Server (HSS)

Location Retrieval Function (LRF)

Media Gateway Control Function (MGCF)

MSC Server enhanced for IMS centralized services

Multimedia Resource Function Processor (MRFP)

Packet Data Gateway (PDG)

Subscription Locator Function (SLF)

WLAN UE

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.122 [4C] apply:

Equivalent Home PLMN (EHPLMN)

Home PLMN (HPLMN)

Visited PLMN (VPLMN)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.218 [5] subclauses 3.1, 8 and 13 apply:

Filter criteria

Initial filter criteria

Initial request

ISC gateway function

Media Resource Broker (MRB)

Multimedia Resource Function Controller (MRFC)

Standalone transaction

Subsequent request

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.228 [7] subclauses 3.1, 4.3.3.1, 4.3.6, 4.6, 4.13, 4.15a, 5.2, 5.4.12.1, 5.10, annex U, and annex W apply:

Border control concepts

Geo-local service number

Home local service number

Implicit registration set

Interconnection Border Control Function (IBCF)

Interrogating-CSCF (I-CSCF)

IMS Application Level Gateway (IMS-ALG)

IMS application reference

IMS Application Reference Identifier (IARI)

IMS communication service

IMS Communication Service Identifier (ICSI)

IMS Services for roaming users in deployments without IMS-level roaming interfaces

Local service number

IP-Connectivity Access Network (IP-CAN)

MPS-subscribed UE

P-CSCF enhanced for WebRTC (eP-CSCF)

Policy and Charging Rule Function (PCRF)

Private user identity

Proxy-CSCF (P-CSCF)

Public Service Identity (PSI)

Public user identity

RCD server

RCD server address

Rich Call Data (RCD) information

Rich Call Data (RCD) URL

Roaming Architecture for Voice over IMS with Local Breakout

Serving-CSCF (S-CSCF)

Statically pre-configured PSI

WebRTC IMS Client (WIC)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.292 [7C] apply:

ICS UE

SCC AS

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.167 [4B] apply:

eCall over IMS

Emergency-CSCF (E-CSCF)

Geographical location information

Location identifier

Location information

For the purposes of the present document, the following terms and definitions given in 3GPP TR 33.203 [19] apply:

GPRS-IMS-Bundled Authentication (GIBA)

Port_pc

Port_ps

Port_uc

Port_us

Protected server port

Protected client port

spi_uc

spi_us

For the purposes of the present document, the following terms and definitions given in 3GPP TR 21.905 [1] apply:

IMS Credentials (IMC)

International Mobile Equipment Identity (IMEI)

IMS SIM (ISIM)

Serial NumbeR (SNR)

Type Approval Code (TAC)

Universal Integrated Circuit Card (UICC)

Universal Subscriber Identity Module (USIM)

User Equipment (UE)

For the purposes of the present document, the following terms and definitions given in RFC 2401 [20A] Appendix A apply:

Security association

A number of different security associations exist within the IM CN subsystem and within the underlying access transport. Within this document this term specifically applies to either:

i)	the security association that exists between the UE and the P-CSCF. For this usage of the term, the term "security association" only applies to IPsec. This is the only security association that has direct impact on SIP; or

ii)	the security association that exists between the WLAN UE and the PDG. This is the security association that is relevant to the discussion of Interworking WLAN as the underlying IP-CAN.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.234 [7A] apply.

Interworking WLAN

For the purposes of the present document, the following terms and definitions given in ITU-T E.164 [57] apply:

International public telecommunication number

For the purposes of the present document, the following terms and definitions given in RFC 5012 [91] apply:

Emergency service identifier

Emergency service URN

Public Safety Answering Point (PSAP)

PSAP URI

For the purposes of the present document, the following terms and definitions given in RFC 5627 [93] apply:

Globally Routable User Agent URI (GRUU)

For the purposes of the present document, the following terms and definitions given in RFC 5626 [92] apply:

Flow

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.310 [19D] annex E and documents referenced therein:

TLS session

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.292 [8O] apply:

CS media

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.301 [8J] apply:

IMS Voice over PS Session (IMSVoPS) indicator

Persistent EPS bearer context

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.328 [19C] apply:

End-to-access edge security

For the purposes of the present document, the following terms and definitions given in 3GPP2 S.R0048-A v4.0 [86F] apply:

Mobile Equipment Identity (MEID)

Manufacturer code

Serial number

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.302 [8U] apply:

Restrictive non-3GPP access network

S2a

S2b

S2c

Trusted non-3GPP access

Untrusted non-3GPP access

Unauthenticated IMSI

Firewall traversal tunnel

For the purposes of the present document, the following terms and definitions given in 3GPP TS 32.240 [16] apply:

Charging Data Function (CDF);

Charging Data Record (CDR)

Online Charging Function (OCF)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 32.260 [17] apply:

IM CN subsystem Charging Identifier (ICID)

For the purposes of the present document, the following terms and definitions given in RFC 8119 [230] apply:

Service access number

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.101 [1A] apply:

eCall

Minimum Set of Data (MSD)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.011 [1C] apply:

3GPP PS data off

3GPP PS data off exempt services

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.402 [7E] apply.

TWAN

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.604 [8ZG] apply.

Diverting user

Diverted-to party

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.221 [272] apply:

Restricted Local Operator Services

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [257] apply:

DN Access Identifier (DNAI)

NR RedCap

Stand-alone Non-Public Network

UE-Satellite-UE (UE-SAT-UE) communication

* * * Next Change * * * *

3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

1xx	A status-code in the range 101 through 199, and excluding 100

18x	A status-code in the range 180 through 189

2xx	A status-code in the range 200 through 299

3xx	A status-code in the range 300 through 399

4xx	A status-code in the range 400 through 499

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

5xx	A status-code in the range 500 through 599

6xx	A status-code in the range 600 through 699

AAA	Authentication, Authorization and Accounting

ANBR	Access Network Bitrate Recommendation

APN	Access Point

APN	Access Point Name

AS	Application Server

ATCF	Access Transfer Control Function

AUTN	Authentication TokeN

AVP	Attribute-Value Pair

B2BUA	Back-to-Back User Agent

BFCP	Binary Floor Control Protocol

BGCF	Breakout Gateway Control Function

c	conditional

BRAS	Broadband Remote Access Server

BSSID	Basic Service Set Identifier

CCF	Charging Collection Function

CDF	Charging Data Function

CDR	Charging Data Record

CH	Credentials Holder

CK	Ciphering Key

CN	Core Network

CPC	Calling Party's Category

CLF	Connectivity session Location and repository Function

CSCF	Call Session Control Function

DHCP	Dynamic Host Configuration Protocol

DNAI	DN Access Identifier

DNN	Data Network Name

DNS	Domain Name System

DOCSIS	Data Over Cable Service Interface Specification

DRVCC	Dual Radio Voice Call Continuity

DTD	Document Type Definition

DTLS	Datagram Transport Layer Security

DTMF	Dual Tone Multi Frequency

DVB	Digital Video Broadcast

DVB-RCS2	Second Generation DVB Interactive Satellite System

e2ae-security	End-to-access edge security

EATF	Emergency Access Transfer Function

EC	Emergency Centre

ECF	Event Charging Function

ECI	E-UTRAN Cell Identity

ECN	Explicit Congestion Notification

E-CSCF	Emergency CSCF

EF	Elementary File

eP-CSCF	P-CSCF enhanced for WebRTC

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

FAP	cdma2000® 1x Femtocell Access Point

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GBR	Guaranteed Bit Rate

GCID	GPRS Charging Identifier

GGSN	Gateway GPRS Support Node

GPON	Gigabit-capable Passive Optical Networks

GPRS	General Packet Radio Service

GRUU	Globally Routable User agent URI

GSTN	General Switched Telephone Network

HPLMN	Home PLMN

HSS	Home Subscriber Server

HTTP	HyperText Transfer Protocol

i	irrelevant

IARI	IMS Application Reference Identifier

IBCF	Interconnection Border Control Function

ICE	Interactive Connectivity Establishment

I-CSCF	Interrogating CSCF

ICS	Implementation Conformance Statement

ICID	IM CN subsystem Charging Identifier

ICSI	IMS Communication Service Identifier

ID	Identifier

IK	Integrity Key

IKEv2	Internet Key Exchange Protocol Version 2

IM	IP Multimedia

IMC	IMS Credentials

IMEI	International Mobile Equipment Identity

IMS	IP Multimedia core network Subsystem

IMS-AGW	IMS Access Gateway

IMS-ALG	IMS Application Level Gateway

IMSI	International Mobile Subscriber Identity

IMSVoPS	IMS Voice over PS Session

IOI	Inter Operator Identifier

IP	Internet Protocol

IP-CAN	IP-Connectivity Access Network

IPsec	IP security

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

ISC	IP Multimedia Subsystem Service Control

ISIM	IM Subscriber Identity Module

I-WLAN	Interworking – WLAN

IWF	Interworking Function

KMS	Key Management Service

LEO	Low Earth Orbit

LRF	Location Retrieval Function

m	mandatory

MAC	Message Authentication Code

MBR	Maximum guaranteed Bit Rate

MCC	Mobile Country Code

MCPTT	Mission Critical Push To Talk

MEID	Mobile Equipment Identity

MEO	Medium Earth Orbit

MGCF	Media Gateway Control Function

MGW	Media Gateway

MNC	Mobile Network Code

MF	Media Function

MRB	Media Resource Broker

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

MSC	Mobile-services Switching Centre

MSD	Minimum Set of emergency related Data

MSRP	Message Session Relay Protocol

n/a	not applicable

NAI	Network Access Identifier

NA(P)T	Network Address (and Port) Translation

NASS	Network Attachment Subsystem

NAT	Network Address Translation

NCC	Network Control Center

NCC_ID	Network Control Center Identifier

NEF	Network Exposure Function

NID	Network Identifier

NP	Number Portability

o	optional

OCF	Online Charging Function

OLI	Originating Line Information

OMR	Optimal Media Routeing

PCC	Policy and Charging Control

PCF	Policy Control Function

PCO	Protocol Configuration Options

PCRF	Policy and Charging Rules Function

P-CSCF	Proxy CSCF

PDG	Packet Data Gateway

PDN	Packet Data Network

PDP	Packet Data Protocol

PDU	Protocol Data Unit

P-GW	PDN Gateway

PICS	Protocol Implementation Conformance Statement

PIDF-LO	Presence Information Data Format Location Object

PLMN	Public Land Mobile Network

PSAP	Public Safety Answering Point

PSI	Public Service Identity

PSTN	Public Switched Telephone Network

QCI	QoS Class Identifier

QoS	Quality of Service

RAND	RANDom challenge

RCD	Rich Call Data

RCS	Return Channel via Satellite

RCST	Return Channel via Satellite Terminal

RES	RESponse

RLOS	Restricted Local Operator Services

RTCP	Real-time Transport Control Protocol

RTP	Real-time Transport Protocol

SAC	Service Area Code

SAI	Service Area Identifier

SBA	Service Based Architecture

SBI	Service Based Interface

S-CSCF	Serving CSCF

SCTP	Stream Control Transmission Protocol

SDES	Session Description Protocol Security Descriptions for Media Streams

SDP	Session Description Protocol

SDU	Service Data Unit

SIP	Session Initiation Protocol

SLF	Subscription Locator Function

SNPN	Stand-alone Non-Public Network

SNR	Serial Number

SQN	SeQuence Number

SRVCC	Single Radio Voice Call Continuity

STUN	Session Traversal Utilities for NAT

SVN	Satellite Virtual Network

SVN-MAC	SVN Medium Access Control label

TAC	Type Approval Code

TFT	Traffic Flow Template

TP	Telepresence

TLS	Transport Layer Security

TRF	Transit and Roaming Function

TURN	Traversal Using Relay NAT

TWAG	Trusted WLAN Access Gateway

TWAN	Trusted WLAN

UA	User Agent

UAC	User Agent Client

UAS	User Agent Server

UDM	Unified Data Management

UDPTL	UDP Transport Layer

UDVM	Universal Decompressor Virtual Machine

UE	User Equipment

UICC	Universal Integrated Circuit Card

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

URN	Uniform Resource Name

USAT	Universal Subscriber Identity Module Application Toolkit

USIM	Universal Subscriber Identity Module

VPLMN	Visited PLMN

WebRTC	Web Real-Time Communication

WIC	WebRTC IMS Client

WLAN	Wireless Local Area Network

x	prohibited

xDSL	Digital Subscriber Line (all types)

XGPON1	10 Gigabit-capable Passive Optical Networks

XMAC	expected MAC

XML	eXtensible Markup Language

* * * End of Changes * * * *