3GPP TSG-CT WG1 Meeting #157	C1-256714

# Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6754 | rev | 1 | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the case of satellite change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Steps 9 and 10 in clause AE.5.2.1 in TS 23.228 state the following: 9. P-CSCF sends a SIP MESSAGE to IMS AS to request it to send SIP re-INVITE to the terminating network (i.e. Step 10) and then towards the UE in the originating network after receiving the SDP answer from the terminating network (i.e. Step 16). This SIP message contains the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP message also contains the satellite ID of the target satellite. 10. IMS AS sends SIP re-INVITE to the terminating network. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP re-INVITE also contains a SIP header for conveying the satellite ID of the target satellite. The above indicates that in case of satellite change, the SDP offer is not constructed by the P-CSCF. Instead, the P-CSCF uses a SIP MESSAGE to trigger the IMS AS to send a SIP re-INVITE. Then, the SDP offer is constructed and sent by the IMS AS when sending the SIP re-INVITE. The current stage-3 text in clause Y.2.2.3 is not aligned with the stage-2 requirements above, as the current stage-3 text specifies that the SDP offer is prepared and sent by the P-CSCF as follows: “If optimized media routing can continue, the P-CSCF shall select a new IMS-AGW on the target satellite and send the "satellite-id" parameter set to the target satellite identifier in the P-Access-Network-Info header field, with an SDP offer to the terminating side to trigger media negotiation.” Hence, the stage-3 text needs to be aligned to stage-2 text specifying that the P-CSCF uses a SIP MESSAGE for triggering the IMS-AGW to send a SIP re-INVITE message with an SDP offer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying that the P-CSCF uses a SIP MESSAGE for triggering the IMS-AGW to send a SIP re-INVITE message with an SDP offer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 specifications are not aligned with stage-2 requirements and specifications leading to ambiguity and possible interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### Y.2.2.3	Handling of satellite change

When the P-CSCF receives an Access Network Information notification with a target satellite identifier from the PCF, the P-CSCF shall determine whether optimized media routing can continue.

If optimized media routing can continue, the P-CSCF shall select a new IMS-AGW on the target satellite and shall trigger, using a SIP MESSAGE, the IMS AS to send a SIP re-INVITE with an SDP offer to the terminating side to trigger media negotiation. Both SIP MESSAGE and SIP re-INVITE include the "satellite-id" parameter set to the target satellite identifier in the P-Access-Network-Info header field.

If optimized media routing cannot continue, the P-CSCF shall select an IMS-AGW on the ground and activates ground fallback routing.

***** End of changes *****