3GPP TSG-CT WG1 Meeting #157	C1-256271

# Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6755 | rev | - | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNAI usage in case of satellite change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Steps 2 and 3 in clause AE.5.2.1 in TS 23.228 has been updated, where the text highlighted in greed is added and the text highlighted in red is removed, as follows: 2. P-CSCF receives the early notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic from PCF as defined in clause 4.3.6.3 of TS 23.502 [94]. This early notification contains DNAI corresponding to the satellite ID of a target satellite that has a gNB to which the UE gets connected and an indication being set "EARLY", indicating that 5GC is prepared to change the user plane path for optimized media routing to the one through this target satellite. P-CSCF derives the satellite ID from the DNAI. NOTE X: The derivation of the satellite ID from the DNAI is based on the operator's policy and implementation. 3. P-CSCF determines that optimized media routing continues to be possible based on the satellite ID received derived in step 2 for the originating network and the satellite ID stored for the terminating network. P-CSCF determines to continue activating optimized media routing. The above indicates that clause Y.2.2.3 needs to be aligned with the latest SA2 updates where, in case of satellite change, DNAI instead of satellite ID is received by the P-CSCF and the P-CSCF needs to derive the satellite ID from the received DNAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that, in case of satellite change, the P-CSCF receives the DNAI corresponding to a target satellite and that the P-CSCF shall derive the target satellite Identifier from the received DNAI. Also clarify that the derivation is based on the operator's policy and implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 specifications are not aligned with stage-2 requirements and specifications leading to ambiguity and possible interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### Y.2.2.3	Handling of satellite change

When the P-CSCF receives an Access Network Information notification with a DNAI corresponding to a target satellite from the PCF, the P-CSCF shall derive the target satellite identifier from the received DNAI and use the derived target satellite identifier in determining whether optimized media routing can continue.

NOTE 1:	Derivation of the satellite identifier from the DNAI is based on the operator's policy and implementation.

If optimized media routing can continue, the P-CSCF shall select a new IMS-AGW on the target satellite and send the  "satellite-id" parameter set to the target satellite identifier in the P-Access-Network-Info header field, with an SDP offer to the terminating side to trigger media negotiation.

If optimized media routing cannot continue, the P-CSCF shall select an IMS-AGW on the ground and activates ground fallback routing.

***** End of changes *****