3GPP TSG-CT WG1 Meeting #157	C1-256607

# Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4547 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stopping T3451 upon receiving paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Timer T3451 needs to be stopped when the UE receives a paging. Hence to align with the terminology used in existing specification, the term “receives a paging” needs to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying that when the UE receives paging, the UE supporting S&F satellite operation shall stop timer T3451, if running. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the specification regarding the used terms, this might cause confusions to readers of specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 4.11.5.2	Handling of timers

The UE supporting S&F satellite operation shall have no more than one timer T3451 running at a time.

The UE maintains timer T3451, which is started with the value of the S&F wait time duration received from the network during an attach procedure, a tracking area updating procedure or a service request procedure.

NOTE:	The S&F Wait Timer value takes into account the time taken to synchronize the UE’s context between MME(s) as specified in subclause O.2 of 3GPP TS 23.401 [10]).

The UE shall stop the timer T3451, if running, when the UE:

1)	successfully performs an attach procedure or a tracking area updating procedure:

a)	via a cell which is not a S&F satellite E-UTRAN cell;

b) 	via a cell being in a TA which is not in the list of TAs; or

c)	in a new PLMN;

2)	receives a paging; or

3)	successfully performs intersystem change from S1 mode to N1 mode or A/Gb mode or Iu mode.

The UE stores the S&F wait time duration in the non-volatile memory of the ME as described in annex C.

***** End of changes *****