3GPP TSG-CT WG1 Meeting #157	C1-256509

# Sophia-Antipolis, France 13-17 Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0913 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the need to update +CSTFOR to further enable IoT device energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sateliot, Novamint, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At the moment, the +CSTFOR contains NAS-level information which is available after a UE has triggered a NAS procedure (e.g., ATTACH, TAU). For the application layer to determine whether to initiate any MO data or MO signalling, it is important for the application layer to know the type of a detected cell. However, +CSTFOR currently provides whether the network operates in S&F mode or not upon request but not as part of the unsolicited result code. This limitations can easily lead to suboptimal operation especially in terms of conserving energy of an IoT device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the store and forward operation mode and the satellite ID of the currently serving cell as part of unsolcited result code of +CSTFOR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the store and forward operation mode and the satellite ID of the currently serving cell, the application layer/terminal cannot determine whether MO data or MO signalling over NTN can be initiated or not when the UE supports S&F satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.91 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### *********** First change **********

## 8.91	Store and Forward operation +CSTFOR

Table 8.91-1: +CSTFOR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSTFOR=[<n>] | +CME ERROR: <err> |
| +CSTFOR? | +CSTFOR: <n>, [<indicator>], [<S&F_wait_time_duration>],[<estimated_uplink_delivery_time_duration>],[<servingSATID>], [<number_of_SATID>[,<SATID1>[,<SATID2>,…[,<SATIDn>]]]],[<sfOperationMode>],[<modeSwitchDuration>] +CME ERROR: <err> |
| +CSTFOR=? | +CSTFOR: (list of supported <n>s) +CME ERROR: <err> |

Description

This command allows control of the store and forward operation parameters. The set command controls the presentation of unsolicited result code +CSTFOR: [<indicator>],[<S&F_wait_time_duration>], [<estimated_uplink_delivery_time_duration>],[<servingSATID>],[<number_of_SATID>[,<SATID1>[,<SATID2>,…[,<SATIDn>]]]],[<sfOperationMode>],[<modeSwitchDuration>].

Refer clause 9.2 for possible <err> values.

Read command returns the current settings of <n> and the S&F satellite operation parameters if sent by network for store and forward operation.

Test command returns supported <n>s values as a compound value.

Defined values

<n>: integer type.

0	Disable presentation of the unsolicited result code +CSTFOR: [<indicator>], [<S&F_wait_time_duration>], [<estimated_uplink_delivery_time_duration>], [<servingSATID>], [<number_of_SATID>[,<SATID1>[,<SATID2>,…[,<SATIDn>]]]],[<sfOperationMode>],[<modeSwitchDuration>]

1	Enable presentation of the unsolicited result code +CSTFORU: [<indicator>], [<S&F_wait_time_duration>], [<estimated_uplink_delivery_time_duration], [<servingSATID>], [<number_of_SATID>[,<SATID1>[,<SATID2>,…[,<SATIDn>]]]],[<sfOperationMode>],[<modeSwitchDuration>]

<indicator>: integer type. Indicates the store and forward operation mode of the currently serving cell.

0	indicates that the  serving cell is operating in normal mode, see 3GPP TS 36.331 [86], section 6.2.2.

1	indicates that the serving cell is operating in store and forward mode, see 3GPP TS 36.331 [86], section 6.2.2.

<S&F_wait_time_duration>: string type; Indicates the store and forward wait time duration in seconds. The <S&F_wait_time_duration> is coded as three bytes (octets 3 to 5) of the Time duration IE as defined in 3GPP TS 24.301 [83].

<estimated_uplink_delivery_time_duration>: string type; Indicates the estimated time required to deliver data to the ground network element from the time the data is sent by the UE to the current serving satellite indicated in <servingSATID>. The <estimated_uplink_delivery_time_duration> value is coded as three bytes (octets 3 to 5) of the Time duration information element in 3GPP TS 24.301 [83].

<servingSATID>: integer type; Indicates the satellite ID of the serving satellite, see 3GPP TS 36.331 [86], clause 6.3.6. The serving satellite may provide the <estimated_uplink_delivery_time_duration>, if operating in the S&F mode, see 3GPP TS 24.301 [83].

<number_of_SATID>:integer type; Indicates the number of satellite IDs present in the S&F monitoring list, see 3GPP TS 24.301 [83], clause 9.9.3.73.

<SATID>:integer type; Indicates the satellite ID in the S&F monitoring list, see 3GPP TS 24.301 [83], clause 9.9.3.73.

<sfOperationMode>:integer type; Indicates the mode switch information about satellite operation

0	indicates the time when the network is going to switch from S&F mode to normal mode as specified by <modeSwitchDuration>.

1	indicates the time when the network is going to switch from normal mode to S&F mode as specified by <modeSwitchDuration>.

<modeSwitchDuration>:string type; Indicates the UTC time information when the network is going to switch modes. When <sfOperationMode> is set to 0 indicates the UTC time when the network is going to switch from S&F mode to normal mode. When <sfOperationMode> is set to 1 indicates the UTC time when the network is going to switch from normal mode to S&F mode. See 3GPP TS 36.331 [86].

Implementation

Optional.

This command is only applicable to UEs supporting satellite E-UTRAN access.

##### *********** End of changes **********