3GPP TSG-CT Meeting #102	CP-233075

Edinburgh, GB, 11th – 12th December 2023	was C4-235159

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0932 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional User Plane Interfaces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2023-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in 3GPP TS 29.244, clause 8.2.118, the following user plane interfaces are defined: Table 8.2.118-1: Interface Type value Interface value; Values (Decimal). S1-U; 0. S5 /S8-U (NOTE 1); 1. S4-U; 2. S11-U; 3. S12; 4. Gn/Gp-U (NOTE 2); 5. S2a-U; 6. S2b-U; 7. eNodeB GTP-U interface for DL data forwarding; 8. eNodeB GTP-U interface for UL data forwarding; 9. SGW/UPF GTP-U interface for DL data forwarding; 10. N3 3GPP Access; 11. N3 Trusted Non-3GPP Access; 12. N3 Untrusted Non-3GPP Access; 13. N3 for data forwarding; 14. N9 (or N9 for non-roaming, see NOTE 3); 15. SGi; 16. N6; 17. N19; 18. S8-U; 19. Gp-U; 20. N9 for roaming; 21. Iu-U; 22. N9 for data forwarding; 23. Sxa-U; 24. Sxb-U; 25. Sxc-U; 26. N4-U; 27. SGW/UPF GTP-U interface for UL data forwarding; 28. N6mb/Nmb9; 29. N3mb; 30. N19mb; 31. Spare; 32 to 63. NOTE 1: If separation of roaming and non-roaming traffic is desired this value should only be used for the S5-U interface and "S8-U" (decimal 19) should be used for the S8-U interface. NOTE 2: If separation of roaming and non-roaming traffic is desired this value should only be used for the Gn-U interface and "Gp-U" (decimal 20) should be used for the Gp-U interface. NOTE 3: If separation of roaming and non-roaming traffic is desired, this value should only be used for N9 non-roaming interfaces and (decimal value "21") should be used for N9 roaming interfaces.. To be able to provision a proper PDR/FAR, the SMF or combined SMF/PGW-C needs to have knowledge of the configured Network Instances on the UPF for different user plane interface. This can be achieved either using local configuration in the SMF via OAM method or using an NRF. However, 3GPP TS 29.510 specifies only a subset of user plane interface types (of the ones specified in 3GPP TS 29.244, this needs to be enhanced. It is proposed to extend UP Interface Type, so that the SMF can solely rely on the provisioning/registration of the UPF NF Profile in the NRF. |
| Summary of change: |  | Extend user plane interface type to include, S1-U, S5-U, S8-U, S11-U, S12, S2a-U, S2b-U, N3 3GPP Access, N3 Trusted Non-3GPP access, N3 Untrusted Non-3GPP Access, N9 for roaming, SGi, N19, Sxa-U, Sxb-U, N4-U. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Extra administrative and operational cost to configure proper network instances for different user plane interfaces in the SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.3.9, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction on Nnrf_NFManagement API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: missing value for N19 in openAPI file |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 6.1.6.3.9	Enumeration: UPInterfaceType

Table 6.1.6.3.9-1: Enumeration UPInterfaceType

| Enumeration value | Description |
| --- | --- |
| "N3" | User Plane Interface: N3 (or N3 for 3GPP Access, see NOTE X) |
| "N6" | User Plane Interface: N6 |
| "N9" | User Plane Interface: N9 (or N9 for non-roaming, see NOTE Y) |
| "DATA_FORWARDING" | User Plane Interface for indirect data forwarding. (NOTE 1) |
| "N6MB" | User Plane Interface: N6mb |
| "N19MB" | User Plane Interface: N19mb |
| "N3MB" | User Plane Interface: N3mb |
| "NMB9" | User Plane Interface: Nmb9 |
| "S1U" | User Plane Interface: S1-U |
| "S5U" | User Plane Interface: S5-U |
| "S8U" | User Plane Interface: S8-U |
| "S11U" | User Plane Interface: S11-U |
| "S12" | User Plane Interface: S12 |
| "S2AU" | User Plane Interface: S2AU |
| "S2BU" | User Plane Interface: S2BU |
| "N3TRUSTEDN3GPP" | User Plane Interface: N3 Trusted Non-3GPP access |
| "N3UNTRUSTEDN3GPP" | User Plane Interface: N3 Untrusted Non-3GPP access |
| "N9ROAMING" | User Plane Interface: N9 for roaming interface |
| "SGI" | User Plane Interface: SGI |
| "N19" | User Plane Interface: N19 |
| "SXAU" | User Plane Interface: Sxa-U |
| "SXBU" | User Plane Interface: Sxb-U |
| "N4U" | User Plane Interface: N4-U |
| NOTE 1: This interface type may be used when a dedicated network instance is deployed for data forwarding. NOTE X: If separation of N3 traffic from 3GPP access and non-3GPP access is required for a PLMN, this value should only be used for the N3 for 3GPP access. NOTE Y: If separation of roaming and non-roaming traffic is desired over N9, this value should only be used for N9 non-roaming interfaces. |  |

# *** Next Change ***

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

info:

version: '1.3.0-alpha.5'

title: 'NRF NFManagement Service'

description: |

NRF NFManagement Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V18.4.0; 5G System; Network Function Repository Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

servers:

- url: '{apiRoot}/nnrf-nfm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

******skipped for clarity******

UPInterfaceType:

description: Types of User-Plane interfaces of the UPF

anyOf:

- type: string

enum:

- N3

- N6

- N9

- DATA_FORWARDING

- N3MB

- N6MB

- N19MB

- NMB9

- S1U

- S5U

- S8U

- S11U

- S12

- S2AU

- S2BU

- N3TRUSTEDN3GPP

- N3UNTRUSTEDN3GPP

- N9ROAMING

- SGI

- N19

- SXAU

- SXBU

- N4U

- type: string

******skipped for clarity******

# *** End of Change ***

# *** For Information Change ***

##### 6.1.6.2.24	Type: InterfaceUpfInfoItem

Table 6.1.6.2.24-1: Definition of type InterfaceUpfInfoItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| interfaceType | UPInterfaceType | M | 1 | User Plane interface type |
| ipv4EndpointAddresses | array(Ipv4Addr) | C | 1..N | Available endpoint IPv4 address(es) of the User Plane interface (NOTE 1) (NOTE 2) |
| ipv6EndpointAddresses | array(Ipv6Addr) | C | 1..N | Available endpoint IPv6 address(es) of the User Plane interface (NOTE 1) (NOTE 2) |
| endpointFqdn | Fqdn | C | 0..1 | FQDN of available endpoint of the User Plane interface (NOTE 1) (NOTE 2) |
| networkInstance | string | O | 0..1 | Network Instance (See 3GPP TS 29.244 [21]) associated to the User Plane interface |
| NOTE 1: At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the InterfaceUpfInfoItem. NOTE 2: When interfaceType is "DATA_FORWARDING", the SMF shall ignore these IEs. The UPF shall register a dummy FQDN or IP address for interfaceType "DATA_FORWARDING" (for backward compatibility reason). |  |  |  |  |