3GPP TSG-CT WG Meeting #86	CP-193079

# Itges, SPAIN; 09th – 10th December 2019       Revision of C4-195193, C4-195328, C4-195530

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0269 | rev | 3 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Secondary RAT Data Usage Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Secondary RAT Data Usage Report Transfer IE needs to be sent from the source AMF to the taget AMF during the N2 based handover. N2InfoContent included in the N2InfoNotificationRspData is used to transfer the Secondary RAT Data Usage Report Transfer IE, however the NgapIeType has not been extended to support the Secondary RAT Data Usage Report Transfer IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Essential corrections: - Add type for Secondary RAT Data Usage Report Transfer IE in N2InfoContent; - Update the N2 Information content for class SM table to support the Secondary RAT Data Usage Report Transfer IE and the related message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Secondary RAT data usage reporting will always fail during the handover procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.54, 6.1.6.3.17, 6.1.6.4.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new corrections to the OpenAPI file for Namf_Communication API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Cover sheet updated. - 6.1.6.2.27 moved from the info part into the actual changes and an extra dot removed in Table 6.1.6.2.27-1. - 6.1.6.2.54 moved from the info part into the actual changes and 'x' in the Table 6.1.6.2.x-1 was replaced by '54'. Rev2: Table number in 6.1.6.3.17 is also fixed Rev3 (for plenary): the first change to clause 6.1.6.2.27 was removed altogether, because (a) it was done against spec v15.1.0, instead of v15.1.1 and (b) the change is editorial and therefore irrelevant for this essential correcton CR |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

##### 6.1.6.2.54	Type: N2InfoNotificationRspData

Table 6.1.6.2.54-1: Definition of type N2InfoNotificationRspData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2Info | N2InfoContent | C | 0..1 | This IE shall be present in the N2InfoNotify response sent by the source AMF to the target AMF during an Inter NG-RAN node N2 based handover procedure (see clause 5.2.2.3.6.2), if Secondary Rat Usage Data are available at the source AMF. When present, this IE shall contain N2 Information. |

* * * * 2nd Change * * * *

##### 6.1.6.3.17	Enumeration: NgapIeType

Table 6.1.6.3.17-1: Enumeration NgapIeType

| Enumeration value | Description |
| --- | --- |
| "PDU_RES_SETUP_REQ" | PDU Session Resource Setup Request Transfer |
| "PDU_RES_REL_CMD" | PDU Session Resource Release Command Transfer |
| "PDU_RES_MOD_REQ" | PDU Session Resource Modify Request Transfer |
| "HANDOVER_CMD" | Handover Command Transfer |
| "HANDOVER_REQUIRED" | Handover Required Transfer |
| "HANDOVER_PREP_FAIL" | Handover Preparation Unsuccessful Transfer |
| "SRC_TO_TAR_CONTAINER" | Source to Target Transparent Container |
| "TAR_TO_SRC_CONTAINER" | Target to Source Transparent Container |
| "RAN_STATUS_TRANS_CONTAINER" | RAN Status Transfer Transparent Container |
| "SON_CONFIG_TRANSFER" | SON Configuration Transfer |
| "NRPPA_PDU" | NRPPa-PDU |
| "UE_RADIO_CAPABILITY" | UE Radio Capability |
| "SECONDARY_RAT_USAGE" | Secondary RAT Data Usage Report Transfer |

* * * * 3rd Change * * * *

###### 6.1.6.4.3.2	NGAP IEs

For N2 information class SM, N2 Information may encode following NGAP SMF related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-1.

Table 6.1.6.4.3-1: N2 Information content for class SM

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PDU Session Resource Setup Request Transfer | 9.3.4.1 | PDU SESSION RESOURCE SETUP REQUEST |
| PDU Session Resource Release Command Transfer | 9.3.4.3 | PDU SESSION RESOURCE RELEASE COMMAND |
| PDU Session Resource Modify Request Transfer | 9.3.4.5 | PDU SESSION RESOURCE MODIFY REQUEST |
| Handover Command Transfer | 9.3.4.12 | HANDOVER COMMAND |
| Handover Required Transfer | 9.3.4.14 | HANDOVER REQUIRED |
| Handover Preparation Unsuccessful Transfer | 9.3.4.18 | HANDOVER COMMAND |
| Secondary RAT Data Usage Report Transfer | 9.3.4.23 | SECONDARY RAT DATA USAGE REPORT |

For N2 information class RAN, N2 Information may encode one of the following NGAP Transparent Container IEs specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-2.

Table 6.1.6.4.3-2: N2 Information content for class RAN

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| Source to Target Transparent Container | 9.3.1.20 | HANDOVER REQUIRED, HANDOVER REQUEST |
| Target to Source Transparent Container | 9.3.1.21 | HANDOVER COMMAND, HANDOVER REQUEST ACKNOWLEDGE |
| UE Radio Capability | 9.3.1.74 | UE CAPABILITY INFO INDICATION. (NOTE 1). |
| SON Configuration Transfer | 9.3.3.6 | UPLINK RAN CONFIGURATION TRANSFER, DOWNLINK RAN CONFIGURATION TRANSFER |
| RAN Status Transfer Transparent Container | 9.2.3.13, 9.2.3.14 | UPLINK RAN STATUS TRANSFER, DOWNLINK RAN STATUS TRANSFER |
| NOTE 1: The AMF receives the UE Radio Capability within a UE CAPABILITY INFO INDICATION message and then the AMF shall store the UE Radio Capability information and transfer it to the target AMF during an inter AMF mobility procedure. |  |  |

For N2 information class NRPPa, N2 Information may encode the following NGAP NRPPA Transport related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-3

Table 6.1.6.4.3-3: N2 Information content for class NRPPa

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| NRPPa-PDU | 9.3.3.14 | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT UPLINK UE ASSOCIATED NRPPA TRANSPORT DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |

* * * * 4th Change * * * *

## A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.0.3

title: Namf_Communication

description: |

AMF Communication Service

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- namf-comm

[…]

NgapIeType:

anyOf:

- type: string

enum:

- PDU_RES_SETUP_REQ

- PDU_RES_REL_CMD

- PDU_RES_MOD_REQ

- HANDOVER_CMD

- HANDOVER_REQUIRED

- HANDOVER_PREP_FAIL

- SRC_TO_TAR_CONTAINER

- TAR_TO_SRC_CONTAINER

- RAN_STATUS_TRANS_CONTAINER

- SON_CONFIG_TRANSFER

- NRPPA_PDU

- UE_RADIO_CAPABILITY

- SECONDARY_RAT_USAGE

- type: string

[…]

* * * * End of Changes * * * *