3GPP TSG-CT WG1 Meeting #121	C1-198424

# Reno (NV), USA, 11-15 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | 3208 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the length of two octets support indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., ZTE, Huawei, Hisilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the current statements in TS 24.008: Note: If the container ID is 0023H or 0024H, then the octet z+3 and octet z+4 indicate the length of container ID contents. and in 24.301: If the UE supporting N1 mode supports receiving QoS rules with the length of two octets or QoS flow descriptions with the length of two octets via the extended protocol configuration options IE, the UE shall include the QoS rules with the length of two octets support indicator or the QoS flow descriptions with the length of two octets support indicator, respectively, in the protocol configuration options IE or the extended protocol configuration options IE. those statements imply that the “QoS rules with the length of two octets support indicator” (0023H) and the “QoS flow descriptions with the length of two octets support indicator” (0024H) sent from the UE to the network included in either PCO IE or ePCO IE are also with the length of container identifier contents field containing two octets. However, if the network could not support the parameters identified by the container identifiers 0023 and 0024, the corresponding unit is ignored and then decoding error will happen on the following parameters. I.e., the parameter sent by the UE includes 4 octets (2 octets ID and 2 octets length) but the network decodes only the first 3 octets (2 octets ID and 1 octet length) which are ignored when the network cannot support it. Therefore, there’s 1 octet left which may incur decoding error on the next parameter and thus impact the procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The length of container identifier contents field is always one octet for both the “QoS rules with the length of two octets support indicator” (0023H) and the “QoS flow descriptions with the length of two octets support indicator” (0024H) sent from the UE to the network. Interoperability impact analysis: This CR is not backward compatible. 1) UE compliant to 24.008 baseline interworking with network compliant to this CR. - The network cannot correctly decode the parameters identified by 0023 and 0024 sent by the UE (the UE sends 3 octets while the network decodes 4 octets). 2) Network compliant to 24.008 baseline interworking with UE compliant to this CR. - The network cannot correctly decode the parameters identified by 0023 and 0024 sent by the UE (the UE sends 4 octets while the network only decodes 3 octets). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Decoding error happens in the network side when the network could not suppport the parameters identified by the container identifiers 0023 and 0024. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

##### 10.5.6.3.1	General

The purpose of the protocol configuration options information element is to:

-	transfer external network protocol options associated with a PDP context activation, and

-	transfer additional (protocol) data (e.g. configuration parameters, error codes or messages/events) associated with an external protocol or an application.

The protocol configuration options is a type 4 information element with a minimum length of 3 octets and a maximum length of 253 octets.

The protocol configuration options information element is coded as shown in figure 10.5.136/3GPP TS 24.008 and table 10.5.154/3GPP TS 24.008.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol configuration options IEI |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of protocol config. options contents |  |  |  |  |  |  |  |  | octet 2 |  |
| 1 ext | 0 0 0 0 Spare |  |  |  | Configuration protocol |  |  |  | octet 3 |  |
| Protocol ID 1 |  |  |  |  |  |  |  |  | octet 4 octet 5 |  |
| Length of protocol ID 1 contents |  |  |  |  |  |  |  |  | octet 6 |  |
| Protocol ID 1 contents |  |  |  |  |  |  |  |  | octet 7 octet m |  |
| Protocol ID 2 |  |  |  |  |  |  |  |  | octet m+1 octet m+2 |  |
| Length of protocol ID 2 contents |  |  |  |  |  |  |  |  | octet m+3 |  |
| Protocol ID 2 contents |  |  |  |  |  |  |  |  | octet m+4 octet n |  |
| . . . |  |  |  |  |  |  |  |  | octet n+1 octet u |  |
| Protocol ID n-1 |  |  |  |  |  |  |  |  | octet u+1 octet u+2 |  |
| Length of protocol ID n-1 contents |  |  |  |  |  |  |  |  | octet u+3 |  |
| Protocol ID n-1 contents |  |  |  |  |  |  |  |  | octet u+4 octet v |  |
| Protocol ID n |  |  |  |  |  |  |  |  | octet v+1 octet v+2 |  |
| Length of protocol ID n contents |  |  |  |  |  |  |  |  | octet v+3 |  |
| Protocol ID n contents |  |  |  |  |  |  |  |  | octet v+4 octet w |  |
| Container ID 1 |  |  |  |  |  |  |  |  | octet w+1 octet w+2 |  |
| Length of container ID 1 contents |  |  |  |  |  |  |  |  | octet w+3 |  |
| Container ID 1 contents |  |  |  |  |  |  |  |  | octet w+4 octet x |  |
| . . . |  |  |  |  |  |  |  |  | octet x+1 octet y |  |
| Container ID n |  |  |  |  |  |  |  |  | octet y+1 octet y+2 |  |
| Length of container ID n contents |  |  |  |  |  |  |  |  | octet y+3 |  |
| Container ID n contents |  |  |  |  |  |  |  |  | octet y+4 octet z |  |
| Container ID n+1 |  |  |  |  |  |  |  |  | octet z+1 octet z+2 |  |
| Length of container ID n+1 contents (see NOTE) |  |  |  |  |  |  |  |  | octet z+3 octet z+4 |  |
| Container ID n+1 contents |  |  |  |  |  |  |  |  | octet z+5 octet za |  |
| NOTE: If the container ID is 0023H or 0024H for network to MS direction, then the octet z+3 and octet z+4 indicate the length of container ID contents. |  |  |  |  |  |  |  |  |  |  |

Figure 10.5.136/3GPP TS 24.008: Protocol configuration options information element

Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element

Configuration protocol (octet 3) Bits 3 2 1 0 0 0 PPP for use with IP PDP type or IP PDN type (see 3GPP TS 24.301 [120]) All other values are interpreted as PPP in this version of the protocol. After octet 3, i.e. from octet 4 to octet z, two logical lists are defined: - the Configuration protocol options list (octets 4 to w), and - the Additional parameters list (octets w+1 to z). Configuration protocol options list (octets 4 to w) The configuration protocol options list contains a variable number of logical units, they may occur in an arbitrary order within the configuration protocol options list. Each unit is of variable length and consists of a: - protocol identifier (2 octets); - the length of the protocol identifier contents of the unit (1 octet); and - the protocol identifier contents itself (n octets). The protocol identifier field contains the hexadecimal coding of the configuration protocol identifier. Bit 8 of the first octet of the protocol identifier field contains the most significant bit and bit 1 of the second octet of the protocol identifier field contains the least significant bit. If the configuration protocol options list contains a protocol identifier that is not supported by the receiving entity the corresponding unit shall be ignored. The length of the protocol identifier contents field contains the binary coded representation of the length of the protocol identifier contents field of a unit. The first bit in transmission order is the most significant bit. The protocol identifier contents field of each unit contains information specific to the configuration protocol specified by the protocol identifier. At least the following protocol identifiers (as defined in RFC 3232 [103]) shall be supported in this version of the protocol: - C021H (LCP); - C023H (PAP); - C223H (CHAP); and - 8021H (IPCP). The support of other protocol identifiers is implementation dependent and outside the scope of the present document. The protocol identifier contents field of each unit corresponds to a “Packet” as defined in RFC 1661 [102] that is stripped off the “Protocol” and the “Padding” octets. The detailed coding of the protocol identifier contents field is specified in the RFC that is associated with the protocol identifier of that unit. Additional parameters list (octets w+1 to z) The additional parameters list is included when special parameters and/or requests (associated with a PDP context) need to be transferred between the MS and the network. These parameters and/or requests are not related to a specific configuration protocol (e.g. PPP), and therefore are not encoded as the "Packets" contained in the configuration protocol options list. The additional parameters list contains a list of special parameters, each one in a separate container. The type of the parameter carried in a container is identified by a specific container identifier. In this version of the protocol, the following container identifiers are specified: MS to network direction: - 0001H (P-CSCF IPv6 Address Request); - 0002H (IM CN Subsystem Signaling Flag); - 0003H (DNS Server IPv6 Address Request); - 0004H (Not Supported); - 0005H (MS Support of Network Requested Bearer Control indicator); - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address Request); - 0008H (DSMIPv6 Home Network Prefix Request); - 0009H (DSMIPv6 IPv4 Home Agent Address Request); - 000AH (IP address allocation via NAS signalling); - 000BH (IPv4 address allocation via DHCPv4); - 000CH (P-CSCF IPv4 Address Request); - 000DH (DNS Server IPv4 Address Request); - 000EH (MSISDN Request); - 000FH (IFOM-Support-Request); - 0010H (IPv4 Link MTU Request); - 0011H (MS support of Local address in TFT indicator); - 0012H (P-CSCF Re-selection support); - 0013H (NBIFOM request indicator); - 0014H (NBIFOM mode); - 0015H (Non-IP Link MTU Request); - 0016H (APN rate control support indicator); - 0017H (3GPP PS data off UE status); - 0018H (Reliable Data Service request indicator); - 0019H (Additional APN rate control for exception data support indicator); - 001AH (PDU session ID); - 001BH (reserved); - 001CH (Reserved); - 001DH (Reserved); - 001EH (Reserved); - 001FH (Reserved); - 0020H (Ethernet Frame Payload MTU Request); - 0021H (Unstructured Link MTU Request); - 0022H (5GSM cause value); - 0023H (QoS rules with the length of two octets support indicator); - 0024H (QoS flow descriptions with the length of two octets support indicator); and - FF00H to FFFFH reserved for operator specific use. Network to MS direction: - 0001H (P-CSCF IPv6 Address); - 0002H (IM CN Subsystem Signaling Flag); - 0003H (DNS Server IPv6 Address); - 0004H (Policy Control rejection code); - 0005H (Selected Bearer Control Mode); - 0006H (Reserved); - 0007H (DSMIPv6 Home Agent Address) ; - 0008H (DSMIPv6 Home Network Prefix); - 0009H (DSMIPv6 IPv4 Home Agent Address); - 000AH (Reserved); - 000BH (Reserved); - 000CH (P-CSCF IPv4 Address); - 000DH (DNS Server IPv4 Address); - 000EH (MSISDN); - 000FH (IFOM-Support); - 0010H (IPv4 Link MTU); - 0011H (Network support of Local address in TFT indicator); - 0012H (Reserved); - 0013H (NBIFOM accepted indicator); - 0014H (NBIFOM mode); - 0015H (Non-IP Link MTU); - 0016H (APN rate control parameters); - 0017H (3GPP PS data off support indication); - 0018H (Reliable Data Service accepted indicator); - 0019H (Additional APN rate control for exception data parameters); - 001AH (reserved); - 001BH (S-NSSAI); - 001CH (QoS rules); - 001DH (Session-AMBR); - 001EH (PDU session address lifetime); - 001FH (QoS flow descriptions); - 0020H (Ethernet Frame Payload MTU); - 0021H (Unstructured Link MTU); - 0022H (Reserved); - 0023H (QoS rules with the length of two octets); - 0024H (QoS flow descriptions with the length of two octets); and - FF00H to FFFFH reserved for operator specific use. If the additional parameters list contains a container identifier that is not supported by the receiving entity the corresponding unit shall be ignored. The container identifier field is encoded as the protocol identifier field and the length of container identifier contents field is encoded as the length of the protocol identifier contents field. When the container identifier indicates P-CSCF IPv6 Address Request, DNS Server IPv6 Address Request, or MSISDN Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IM CN Subsystem Signaling Flag (see 3GPP TS 24.229 [95]), the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. In Network to MS direction this information may be used by the MS to indicate to the user whether the requested dedicated signalling PDP context was successfully established. When the container identifier indicates P-CSCF IPv6 Address, the container identifier contents field contains one IPv6 address corresponding to a P-CSCF address (see 3GPP TS 24.229 [95]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 4291 [99]. When there is a need to include more than one P-CSCF IPv6 address, then more logical units with the container identifier indicating P-CSCF IPv6 Address are used. If more than 3 instances of the P-CSCF IPv6 Address logical unit are received by the MS then the MS may ignore all but the first 3 instances of the P-CSCF IPv6 Address logical unit received. When the container identifier indicates DNS Server IPv6 Address, the container identifier contents field contains one IPv6 DNS server address (see 3GPP TS 27.060 [36a]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 4291 [99]. When there is a need to include more than one DNS Server IPv6 address, then more logical units with the container identifier indicating DNS Server IPv6 Address are used. When the container identifier indicates Policy Control rejection code, the container identifier contents field contains a Go interface related cause code from the GGSN to the MS (see 3GPP TS 29.207 [100]). The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates MS Support of Network Requested Bearer Control indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates Selected Bearer Control Mode, the container identifier contents field contains the selected bearer control mode, where ‘01H’ indicates that ‘MS only’ mode has been selected and ‘02H’ indicates that ‘MS/NW’ mode has been selected. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates DSMIPv6 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Network Prefix Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DSMIPv6 Home Agent Address, the container identifier contents field contains one IPv6 address corresponding to a DSMIPv6 HA address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 4291 [99]. When the container identifier indicates DSMIPv6 Home Network Prefix, the container identifier contents field contains one IPv6 Home Network Prefix (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). This IPv6 prefix is encoded as an IPv6 address according to IETF RFC 4291 [99] followed by 8 bits which specifies the prefix length. When the container identifier indicates DSMIPv6 IPv4 Home Agent Address, the container identifier contents field contains one IPv4 address corresponding to a DSMIPv6 IPv4 Home Agent address (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]). When the container identifier indicates P-CSCF IPv4 Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates DNS Server IPv4 Address Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates P-CSCF IPv4 Address, the container identifier contents field contains one IPv4 address corresponding to the P-CSCF address to be used. When there is a need to include more than one P-CSCF IPv4 address, then more logical units with the container identifier indicating P-CSCF IPv4 Address are used. If more than 3 instances of the P-CSCF IPv4 Address logical unit are received by the MS then the MS may ignore all but the first 3 instances of the P-CSCF IPv4 Address logical unit received. When the container identifier indicates DNS Server IPv4 Address, the container identifier contents field contains one IPv4 address corresponding to the DNS server address to be used. When there is a need to include more than one DNS Server IPv4 address, then more logical units with the container identifier indicating DNS Server IPv4 Address are used. P-CSCF IPv4 Address Request, P-CSCF IPv4 Address, DNS Server IPv4 Address Request and DNS Server IPv4 Address are applicable only in S1-mode. When the container identifier indicates IP address allocation via NAS signalling, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IP address allocation via DHCPv4, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates MSISDN, the container identifier contents field contains the MSISDN (see 3GPP TS 23.003 [10]) assigned to the MS. Use of the MSISDN provided is defined in subclause 6.4. When the container identifier indicates IFOM Support Request (see 3GPP TS 24.303 [124] and 3GPP TS 24.327 [125]), the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IFOM Support, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the Home Agent supports IFOM. When the container identifier indicates IPv4 Link MTU Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. When the container identifier indicates IPv4 Link MTU, the length of container identifier contents indicates a length equal to two. The container identifier contents field contains the binary coded representation of the IPv4 link MTU size in octets. Bit 8 of the first octet of the container identifier contents field contains the most significant bit and bit 1 of the second octet of the container identifier contents field contains the least significant bit. If the length of container identifier contents is different from two octets, then it shall be ignored by the receiver. When the container identifier indicates MS support of Local address in TFT, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS supports Local address in TFTs. When the container identifier indicates Network support of Local address in TFT, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the network supports Local address in TFTs. When the container identifier indicates P-CSCF Re-selection support, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This PCO parameter may be present only if a container with P-CSCF IPv4 Address Request or P-CSCF IPv6 Address Request is present. This information indicates that the UE supports P-CSCF re-selection based on procedures specified in 3GPP TS 24.229 [95] subclauses B.2.2.1C, L.2.2.1C and R.2.2.1C. When the container identifier indicates NBIFOM request indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS requests the NBIFOM usage. When the container identifier indicates NBIFOM accepted indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the network accepts UE's request of the NBIFOM usage. When the container identifier indicates NBIFOM mode, the length of container identifier contents indicates a length equal to one. If the length of container identifier contents indicates length different to one, it shall be ignored. The container identifier contents field containing value 00H indicates the UE-initiated NBIFOM mode. The container identifier contents field containing value 01H indicates the network-initiated NBIFOM mode. The container identifier contents field containing a value other than 00H and other than 01H shall be ignored. When the container identifier indicates Non-IP Link MTU Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS requests link MTU for "non-IP" PDN connection. When the container identifier indicates Non-IP Link MTU, the length of container identifier contents indicates a length equal to two. The container identifier contents field contains the binary coded representation of the link MTU size for non-IP PDN connection in octets which is at least 128 octets. Bit 8 of the first octet of the container identifier contents field contains the most significant bit and bit 1 of the second octet of the container identifier contents field contains the least significant bit. If the length of container identifier contents is different from two octets, then it shall be ignored by the receiver. When the container identifier indicates APN rate control support indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS supports APN rate control functionality. When the container identifier indicates APN rate control parameters, the container identifier contents field contains parameters for APN rate control functionality. The container contents are coded as described in subclause 10.5.6.3.2. When the container identifier indicates 3GPP PS data off UE status, the container identifier contents field contains information of the status of 3GPP PS data off in the UE for a PDN connection where "01H" indicates ’deactivated’ and "02H" indicates ‘activated’. The length of container identifier contents indicates a length equal to one. If the container identifier contents field is empty or its actual length is greater than one octet, then it shall be ignored by the receiver. When the container identifier indicates 3GPP PS data off support indication, the container identifier contents field is empty. The length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, then it shall be ignored by the receiver. When the container identifier indicates Reliable Data Service request indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS requests the Reliable Data Service usage as specified in 3GPP TS 24.250 [162]. When the container identifier indicates Reliable Data Service accepted indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the network accepts UE's request of the Reliable Data Service usage as specified in 3GPP TS 24.250 [162]. When the container identifier indicates Additional APN rate control for exception data support indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS supports additional APN rate control for exception data functionality. When the container identifier indicates Additional APN rate control for exception data parameters, the container identifier contents field contains parameters for additional APN rate control for exception data functionality. The container contents are coded as described in subclause 10.5.6.3.3. When the container identifier indicates PDU session identity, the container identifier contents field contains the PDU session identity assigned by the MS. The encoding of the PDU session identity and its usage are defined in 3GPP TS 24.007 [20]. When the container identifier indicates S-NSSAI, the container identifier contents field contains one S-NSSAI value followed by one PLMN ID that the S-NSSAI relates to. The S-NSSAI value is coded as the value part of S-NSSAI information element as specified in subclause 9.11.2.8 of 3GPP TS 24.501 [167]. The PLMN ID is encoded as the value of the PLMN identity of the CN operator IE in subclause 10.5.5.36. The usage of the S-NSSAI and the associated PLMN ID is defined in 3GPP TS 24.501 [167]. When the container identifier indicates QoS rules, the container identifier contents field contains the QoS rules for the QoS flow corresponding to the EPS bearer of the PDN connection. The QoS rules is coded as the value part of QoS rules information element as specified in subclause 9.11.4.13 of 3GPP TS 24.501 [167]. The usage of the QoS rules is specified in 3GPP TS 24.501 [167]. When the container identifier indicates Session-AMBR, the container identifier contents field contains the Session-AMBR for the PDU session corresponding to the PDN connection. The Session-AMBR is coded as the value part of Session-AMBR information element as specified in subclause 9.11.4.14 of 3GPP TS 24.501 [167]. The usage of the Session-AMBR is specified in 3GPP TS 24.501 [167]. When the container identifier indicates PDU session address lifetime, the length of container identifier contents indicates a length equal to two. The container identifier contents field contains the binary coded representation of how long the network is willing to maintain the PDU session in units of seconds. Bit 8 of the first octet of the container identifier contents field contains the most significant bit and bit 1 of the second octet of the container identifier contents field contains the least significant bit. If the length of container identifier contents is different from two octets, then it shall be ignored by the receiver. When the container identifier indicates QoS flow descriptions, the container identifier contents field contains the QoS flow descriptions for the QoS flow corresponding to the EPS bearer of the PDN connection. The QoS flow descriptions is coded as the value part of QoS flow descriptions information element as specified in subclause 9.11.4.12 of 3GPP TS 24.501 [167]. The usage of the QoS flow descriptions is specified in 3GPP TS 24.501 [167]. When the container identifier indicates Ethernet Frame Payload MTU Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS requests link MTU for an Ethernet PDU session. When the container identifier indicates Ethernet Frame Payload MTU, the length of container identifier contents indicates a length equal to two. The container identifier contents field contains the binary coded representation of Ethernet frame payload MTU size, i.e. the maximum size of a payload of an Ethernet frame which can be sent via an Ethernet PDU session in octets. Bit 8 of the first octet of the container identifier contents field contains the most significant bit and bit 1 of the second octet of the container identifier contents field contains the least significant bit. If the length of container identifier contents is different from two octets, then it shall be ignored by the receiver. When the container identifier indicates Unstructured Link MTU Request, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the MS requests link MTU for an Unstructured PDU session. When the container identifier indicates Unstructured Link MTU, the length of container identifier contents indicates a length equal to two. The container identifier contents field contains the binary coded representation of unstructured link MTU size, i.e. the maximum size of a message which can be sent via an Unstructured PDU session in octets. Bit 8 of the first octet of the container identifier contents field contains the most significant bit and bit 1 of the second octet of the container identifier contents field contains the least significant bit. If the length of container identifier contents is different from two octets, then it shall be ignored by the receiver. When the container identifier indicates 5GSM cause value, the container identifier contents field contains a 5GSM cause value. The encoding of the 5GSM cause value and its usage are specified in 3GPP TS 24.501 [167]. When the container identifier indicates QoS rules with the length of two octets support indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. The length of container identifier contents field consists of one octet. This information indicates that the MS supports receiving QoS rules with the length of two octets. When the container identifier indicates QoS flow descriptions with the length of two octets support indicator, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. The length of container identifier contents field consists of one octet. This information indicates that the MS supports receiving QoS flow descriptions with the length of two octets. When the container identifier indicates QoS rules with the length of two octets, the container identifier contents field contains the QoS rules for the QoS flow corresponding to the EPS bearer of the PDN connection if the MS has indicated the support of receiving QoS rules with the length of two octets. The QoS rules with the length of two octets is coded as the value part of QoS rules information element as specified in subclause 9.11.4.13 of 3GPP TS 24.501 [167]. The usage of the QoS rules is specified in 3GPP TS 24.501 [167]. See NOTE 2. When the container identifier indicates QoS flow descriptions with the length of two octets, the container identifier contents field contains the QoS flow descriptions for the QoS flow corresponding to the EPS bearer of the PDN connection if the MS has indicated the support of receiving QoS flow descriptions with the length of two octets. The QoS flow descriptions with the length of two octets is coded as the value part of QoS flow descriptions information element as specified in subclause 9.11.4.12 of 3GPP TS 24.501 [167]. The usage of the QoS flow descriptions is specified in 3GPP TS 24.501 [167]. See NOTE 2. When the container identifier indicates operator specific use, the Container contents starts with MCC and MNC of the operator providing the relevant application and can be followed by further application specific information. The coding of MCC and MNC is as in octet 2 to 4 of the Location Area Identification information element in subclause 10.5.1.3. NOTE 1: The additional parameters list and the configuration protocol options list are logically separated since they carry different type of information. The beginning of the additional parameters list is marked by a logical unit, which has an identifier (i.e. the first two octets) equal to a container identifier (i.e. it is not a protocol identifier). NOTE 2: If the QoS rules with the length of two octets or the QoS flow descriptions with the length of two octets is included, then extended protocol configuration options as specified in the subclause 10.5.6.3A shall be used.