3GPP TSG-CT WG1 Meeting #121	C1-198915

# Reno (NV), USA, 11-15 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1737 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the coding of EPS bearer identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Huawei, HiSilicon, ZTE, CATT, Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc16 |  |  |  |  |  | Date: |  |  | 2019-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on TS 24.301 sub-clause 9.3.2 “EPS bearer identity”: 9.3.2 EPS bearer identity Bits 5 to 8 of the first octet of every EPS Session Management (ESM) message contain the EPS bearer identity. The EPS bearer identity and its use to identify a message flow are defined in 3GPP TS 24.007 [12]. Also, based on TS 24.007 subclause 11.2.3.1.5 “EPS bearer identity”: 11.2.3.1.5 EPS bearer identity A L3 protocol may define that bits 5 to 8 of octet 1 of a standard L3 message of the protocol contain the EPS bearer identity. The EPS bearer identity is used to identify a message flow. It is clear that bits 5 to 8 of the octet is used to contain the EPS bearer identity in both the mapped EPS bearer context IE and the QoS flow descriptions IE. However, CR#1340 (Rel-16; 5GProtoc16 CR) incorrectly specified that the bits 1 to 4 of the octet is used for the EPS bearer identity. Furthermore, CR#1340 causes inter-operability issues, i.e., - Rel-15 UE (bits 5-8 for EBI) will not work with Rel-16 network (bits 1-4 for EBI) - Rel-16 UE (bits 1-4 for EBI) will not work with Rel-15 network (bits 5-8 for EBI) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the mapped EPS bearer context IE, the bits 5 to 8 of octet 4 are used to contain the EPS bearer identity. - For the QoS flow description IE, bits 5 to 8 of the parameter contents contain the EPS bearer identity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The EBI coding is incorrect. - The current requirement (i.e., bits 1-4 for EBI) causes serious inter-operablility issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.4.8, 9.11.4.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

#### 9.11.4.8	Mapped EPS bearer contexts

The purpose of the mapped EPS bearer contexts information element is to indicate a set of EPS bearer contexts for a PDU session, as described in subclause 6.1.4.1.

The mapped EPS bearer contexts information element is a type 6 information element with a minimum length of 7 octet and a maximum length of 65538 octets.

The mapped EPS bearer contexts information element is coded as shown in figure 9.11.4.8.1, figure 9.11.4.8.2, figure 9.11.4.8.3 and table 9.11.4.8.1.

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mapped EPS bearer contexts IEI |  |  |  |  |  |  |  | octet 1 |  |
|  | Length of Mapped EPS bearer contexts contents |  |  |  |  |  |  |  | octet 2 |  |
|  |  |  |  |  |  |  |  |  | octet 3 |  |
|  | Mapped EPS bearer context 1 |  |  |  |  |  |  |  | octet 4 octet u |  |
|  | Mapped EPS bearer context 2 |  |  |  |  |  |  |  | octet u+1 octet v |  |
|  | … |  |  |  |  |  |  |  | octet v+1 octet w |  |
|  | Mapped EPS bearer context n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 9.11.4.8.1: Mapped EPS bearer contexts

|  | 8 | 7 |  | 6 | 5 |  | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EPS bearer identity |  |  |  |  |  |  |  |  |  |  | octet 4 |
|  | Length of Mapped EPS bearer context |  |  |  |  |  |  |  |  |  |  | octet 5 octet 6 |
|  | Operation code |  | 0 Spare |  |  | E bit |  | Number of EPS parameters |  |  |  | octet 7 |
|  | EPS parameters list |  |  |  |  |  |  |  |  |  |  | octet 8 octet u |

Figure 9.11.4.8.2: Mapped EPS bearer context

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EPS parameter identifier 1 |  |  |  |  |  |  |  | octet 8 |
|  | Length of EPS parameter contents 1 |  |  |  |  |  |  |  | octet 9 |
|  | EPS parameter contents 1 |  |  |  |  |  |  |  | octet 10 octet h |
|  | EPS parameter identifier 2 |  |  |  |  |  |  |  | octet h+1 |
|  | Length of EPS parameter contents 2 |  |  |  |  |  |  |  | octet h+2 |
|  | EPS parameter contents 2 |  |  |  |  |  |  |  | octet h+3 octet i |
|  | … |  |  |  |  |  |  |  | octet i+1 octet j |
|  | EPS parameter identifier N |  |  |  |  |  |  |  | octet j+1 |
|  | Length of EPS parameter contents N |  |  |  |  |  |  |  | octet j+2 |
|  | EPS parameter contents N |  |  |  |  |  |  |  | octet j+3 octet u |

Figure 9.11.4.8.3: EPS parameters list

Table 9.11.4.8.1: Mapped EPS bearer contexts information element

EPS bearer identity (octet 4) Bits 5 to 8 contain the EPS bearer identity, and are coded as specified in subclause 9.3.2 of 3GPP TS 24.301 [15]. Bits 1 to 4 are spare and shall be coded as zero. Operation code (bits 8 to 7 of octet 7) Bits 8 7 0 0 Reserved 0 1 Create new EPS bearer 1 0 Delete existing EPS bearer 1 1 Modify existing EPS bearer Bit 6 of octet 7 is spare and shall be coded as zero. E bit (bit 5 of octet 7) For the "create new EPS bearer" operation, the E bit is encoded as follows: Bit 5 0 parameters list is not included 1 parameters list is included For the "modify existing EPS bearer" operation, the E bit is encoded as follows: Bit 5 0 previously provided parameters list extension 1 previously provided parameters list replacement If the E bit is set to "parameters list is not included", the number of EPS parameters field has zero value. If the E bit is set to "parameters list is included", the number of EPS parameters field has non-zero value. If the E bit is set to "previously provided parameters list extension" or "previously provided parameters list replacement", the number of parameters field can have zero or non-zero value. For the "create new EPS bearer" operation and "delete existing EPS bearer" operation, bit 5 of octet 7 is ignored. Number of EPS parameters (bits 4 to 1 of octet 7) The number of EPS parameters contains the binary coding for the number of EPS parameters in the EPS parameters list field. The number of EPS parameters field is encoded in bits 4 through 1 of octet x+1 where bit 4 is the most significant and bit 1 is the least significant bit. EPS parameters list (octets 8 to u) The EPS parameters list contains a variable number of EPS parameters. Each EPS parameter included in the EPS parameters list is of variable length and consists of: - an EPS parameter identifier (1 octet); - the length of the EPS parameter contents (1 octet); and - the EPS parameter contents itself (variable amount of octets). The EPS parameter identifier field is used to identify each EPS parameter included in the EPS parameters list and it contains the hexadecimal coding of the EPS parameter identifier. Bit 8 of the EPS parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following EPS parameter identifiers are specified: - 01H (Mapped EPS QoS parameters); - 02H (Mapped extended EPS QoS parameters); and - 03H (Traffic flow template). - 04H (APN-AMBR). - 05H (extended APN-AMBR). If the EPS parameters list contains an EPS parameter identifier that is not supported by the receiving entity the corresponding EPS parameter shall be discarded. The length of EPS parameter contents field contains the binary coded representation of the length of the EPS parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates mapped EPS QoS parameters, the length and parameter contents field are coded as specified in subclause 9.9.4.3 of 3GPP TS 24.301 [15]. When the parameter identifier indicates mapped extended EPS QoS parameters, the length and parameter contents field are coded as specified in subclause 9.9.4.30 of 3GPP TS 24.301 [15]. When the parameter identifier indicates traffic flow template, the length and parameter contents field are coded from octet 2 as shown figure 10.5.144 and table 10.5.162 of 3GPP TS 24.008 [12]. When the parameter identifier indicates APN-AMBR, the length and parameter contents field are coded as specified in subclause 9.9.4.2 of 3GPP TS 24.301 [15]. When the parameter identifier indicates Extended APN-AMBR, the length and parameter contents field are coded as specified in subclause 9.9.4.29 of 3GPP TS 24.301 [15].

***** Next change *****

#### 9.11.4.12	QoS flow descriptions

The purpose of the QoS flow descriptions information element is to indicate a set of QoS flow descriptions to be used by the UE, where each QoS flow description is a set of parameters as described in subclause 6.2.5.1.1.4.

The QoS flow descriptions information element is a type 6 information element with a minimum length of 6 octets. The maximum length for the information element is 65538 octets.

The QoS flow descriptions information element is coded as shown in figure 9.11.4.12.1, figure 9.11.4.12.2, figure 9.11.4.12.3, figure 9.11.4.12.4, and table 9.11.4.12.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QoS flow descriptions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of QoS flow descriptions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| QoS flow description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| QoS flow description 2 |  |  |  |  |  |  |  | octet u+1 octet v |
| ... |  |  |  |  |  |  |  | octet v+1 octet w |
| QoS flow description n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 9.11.4.12.1: QoS flow descriptions information element

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare | QFI |  |  |  |  |  |  |  |  |  | octet 4 |
| Operation code |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare | 0 Spare |  | octet 5 |
| 0 Spare |  | E | Number of parameters |  |  |  |  |  |  |  |  |  | octet 6 |
| Parameters list |  |  |  |  |  |  |  |  |  |  |  |  | octet 7* octet u* |

Figure 9.11.4.12.2: QoS flow description

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter 1 |  |  |  |  |  |  |  | octet 7 octet m |
| Parameter 2 |  |  |  |  |  |  |  | octet m+1 octet n |
| ... |  |  |  |  |  |  |  | octet n+1 octet o |
| Parameter n |  |  |  |  |  |  |  | octet o+1 octet u |

Figure 9.11.4.12.3: Parameters list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter identifier |  |  |  |  |  |  |  | octet 7 |
| Length of parameter contents |  |  |  |  |  |  |  | octet 8 |
| Parameter contents |  |  |  |  |  |  |  | octet 9 octet m |

Figure 9.11.4.12.4: Parameter

Table 9.11.4.12.1: QoS flow descriptions information element

QoS flow identifier (QFI) (bits 6 to 1 of octet 4) QFI field contains the QoS flow identifier. Bits 6 5 4 3 2 1 0 0 0 0 0 0 no QoS flow identifier assigned 0 0 0 0 0 1 QFI 1 to 1 1 1 1 1 1 QFI 63 The network shall not set the QFI value to 0.

Operation code (bits 8 to 6 of octet 5) Bits 8 7 6 0 0 1 Create new QoS flow description 0 1 0 Delete existing QoS flow description 0 1 1 Modify existing QoS flow description All other values are reserved.

E bit (bit 7 of octet 6) For the "create new QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 reserved 1 parameters list is included For the "Delete existing QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 parameters list is not included 1 reserved For the "modify existing QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 extension of previously provided parameters 1 replacement of all previously provided parameters If the E bit is set to "parameters list is not included", the number of parameters field has zero value. If the E bit is set to "parameters list is included", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" or "replacement of all previously provided parameters", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" and one of the parameters in the new parameters list already exists in the previously provided parameters, the parameter shall be set to the new value. Number of parameters (bits 6 to 1 of octet 6) The number of parameters field contains the binary coding for the number of parameters in the parameters list field. The number of parameters field is encoded in bits 6 through 1 of octet 6 where bit 6 is the most significant and bit 1 is the least significant bit. Parameters list (octets 7 to u) The parameters list contains a variable number of parameters. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (variable amount of octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (5QI); - 02H (GFBR uplink); - 03H (GFBR downlink); - 04H (MFBR uplink); - 05H (MFBR downlink); - 06H (Averaging window); and - 07H (EPS bearer identity). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates 5QI, the parameter contents field contains the binary representation of 5G QoS identifier (5QI) that is one octet in length. 5QI: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 5QI 1 0 0 0 0 0 0 1 0 5QI 2 0 0 0 0 0 0 1 1 5QI 3 0 0 0 0 0 1 0 0 5QI 4 0 0 0 0 0 1 0 1 5QI 5 0 0 0 0 0 1 1 0 5QI 6 0 0 0 0 0 1 1 1 5QI 7 0 0 0 0 1 0 0 0 5QI 8 0 0 0 0 1 0 0 1 5QI 9 0 0 0 0 1 0 1 0 to Spare 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 1 5QI 65 0 1 0 0 0 0 1 0 5QI 66 0 1 0 0 0 0 1 1 5QI 67 0 1 0 0 0 1 0 0 Spare 0 1 0 0 0 1 0 1 5QI 69 0 1 0 0 0 1 1 0 5QI 70 0 1 0 0 0 1 1 1 5QI 71 0 1 0 0 1 0 0 0 5QI 72 0 1 0 0 1 0 0 1 5QI 73 0 1 0 0 1 0 1 0 5QI 74 0 1 0 0 1 0 1 1 5QI 75 0 1 0 0 1 1 0 0 5QI 76 0 1 0 0 1 1 0 1 to Spare 0 1 0 0 1 1 1 0 0 1 0 0 1 1 1 1 5QI 79 0 1 0 1 0 0 0 0 5QI 80 0 1 0 1 0 0 0 1 Spare 0 1 0 1 0 0 1 0 5QI 82 0 1 0 1 0 0 1 1 5QI 83 0 1 0 1 0 1 0 0 5QI 84 0 1 0 1 0 1 0 1 5QI 85 0 1 0 1 0 1 1 0 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific 5QIs 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved The network shall consider all other values not explicitly defined in this version of the protocol as unsupported. If the UE receives a 5QI value (excluding the reserved 5QI values) that it does not understand, the UE shall choose a 5QI value from the set of 5QI values defined in this version of the protocol (see 3GPP TS 23.501 [8]) and associated with: - GBR QoS flows, if the QoS flow includes a GFBR uplink parameter and a GFBR downlink parameter; and - non-GBR QoS flows, if the QoS flow does not include a GFBR uplink parameter or does not include a GFBR downlink parameter. The UE shall use this chosen 5QI value for internal operations only. The UE shall use the received 5QI value in subsequent NAS signalling procedures. When the parameter identifier indicates "GFBR uplink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for uplink followed by two octets containing the value of the guaranteed flow bit rate for uplink. Unit of the guaranteed flow bit rate for uplink (octet 1) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for uplink in units defined by the unit of the guaranteed flow bit rate for uplink. When the parameter identifier indicates "GFBR downlink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for downlink followed by two octets containing the value of the guaranteed flow bit rate for downlink. Unit of the guaranteed flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the guaranteed flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate for downlink in units defined by the unit of the guaranteed flow bit rate for downlink. When the parameter identifier indicates "MFBR uplink", the parameter contents field contains the one octet indicating the unit of the maximum flow bit rate for uplink followed by two octets containing the value of maximum flow bit rate for uplink. Unit of the maximum flow bit rate for uplink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for uplink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for uplink in units defined by the unit of the maximum flow bit rate for uplink. When the parameter identifier indicates "MFBR downlink", the parameter contents field contains one octet indicating the unit of the maximum flow bit rate for downlink followed by two octets containing the value of the maximum flow bit rate for downlink. Unit of the maximum flow bit rate for downlink (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate for uplink. Value of the maximum flow bit rate for downlink (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate for downlink in units defined by the unit of the maximum flow bit rate for downlink. When the parameter identifier indicates "averaging window", the parameter contents field contains the binary representation of the averaging window for both uplink and downlink in milliseconds and the parameter contents field is two octets in length. When the parameter identifier indicates EPS bearer identity, the length of EPS bearer identity is one octet, bits 5 to 8 of the parameter contents contain the EPS bearer identity as specified in subclause 9.3.2 of 3GPP TS 24.301 [15] (see NOTE) and bits 1 to 4 of the parameter contents are spare and shall be coded as zero. The UE shall not include the EPS bearer identity parameter in any mobile originated 5GSM messages.

NOTE: The total number of EPS bearer identities included in all QoS flow descriptions of a UE cannot exceed fifteen.