3GPP TSG-CT WG Meeting #86	CP-193122

# Itges, SPAIN; 09th – 10th December 2019												   was C4-195336

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0266 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF / NF Service Set ID in UE Context Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the PDU session establishment, the AMF may discover the SMF Set ID and SMF Service ID from the NRF, and/or the AMF may receive binding indication from the SMF (for the created SM context resource) including the binding level and binding ID values (e.g. SMF Set ID, SMF Service Set ID, SMF Instance ID) The SMF Set ID, SMF Service Set ID and Binding Level shall be transferred to the target AMF during AMF changes, if available, e.g. to enable the target AMF to reselect another SMF (service) instance if the previous SMF (service) instance is not reachable, or to enable the target AMF to include a server binding indication header in the HTTP requests targeting the PDU session resource when using indirect communication. The same principles also apply to the PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New attributes are defined to enable to signal the SMF Set ID, SMF Service Set ID, SMF Binding Level, and the PCF Set ID, PCF Service Set ID, PCF Binding Level in UE Context Transfer procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target AMF does not receive the NF SET ID, NF Service Set ID and the Binding Level of the SMF and PCF from the source AMF, during AMF changes; the target AMF (or SCP) may not be able to select another SMF / PCF (service) instance to serve the UE e.g. if the previous SMF or PCF is no longer reachable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.25, 6.1.6.2.37, 6.1.6.3.xx, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features in the OpenAPI file TS29518_Namf_Communication. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: Correct the baseline of 6.1.6.1 to TS29.518, previously the change is implemented to clause 6.1.6.1 of TS29.502. |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Information within AMFStatusChangeSubscribe |  |
| AmfStatusChangeNotification | 6.1.6.2.3 | Information within AMFStatusChangeNotify |  |
| AmfStatusInfo | 6.1.6.2.4 | Information within AMFStatusChangeNotify |  |
| AssignEbiData | 6.1.6.2.5 | Represents information needed for AMF to assign EBIs. |  |
| AssignedEbiData | 6.1.6.2.6 | Represents successful assignment of EBI(s). |  |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |  |
| UEContextRelease | 6.1.6.2.8 | Information within ReleaseUeContext |  |
| N2InformationTransferReqData | 6.1.6.2.9 | N2 information requested to be transferred to 5G AN. |  |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Subscription information for non UE specific N2 information notification. |  |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | The created subscription for non UE specific N2 information notification. |  |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Subscription information for UE specific N1 and/or N2 information notification. |  |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | The created subscription for UE specific N1 and/or N2 information notification. |  |
| N2InformationNotification | 6.1.6.2.14 | N2 information for notification. |  |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |  |
| N1MessageNotification | 6.1.6.2.16 | N1 message notification data structure. |  |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container |  |
| N1N2MessageTransferReqData | 6.1.6.2.18 | N1/N2 message container |  |
| N1N2MessageTransferRspData | 6.1.6.2.19 | N1/N2 message transfer response |  |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |  |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |  |
| UeContextTransferReqData | 6.1.6.2.23 | Represents to start transferring of an individual ueContext resource from old AMF to new AMF. |  |
| UeContextTransferRspData | 6.1.6.2.24 | Indicates the transferring of the individual ueContext resource is started successfully. |  |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |  |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |  |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |  |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |  |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |  |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | N1/N2 Message Transfer Failure Notification |  |
| N1N2MessageTransferError | 6.1.6.2.31 | N1/N2 Message Transfer Error |  |
| N2N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |  |
| N2InformationTransferRspData | 6.1.6.2.33 | Indicates a successful delivery of N2 Information to the AN. |  |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |  |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |  |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |  |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |  |
| NssaiMapping | 6.1.6.2.38 | Represents a map of a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |  |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Provides information on the UE registration completion at a target AMF. |  |
| AssignEbiError | 6.1.6.3.40 | Represents the details regarding EBI assignment failure. |  |
| UeContextCreateData | 6.1.6.2.41 | Indicates a request to create an individual ueContext resource |  |
| UeContextCreatedData | 6.1.6.2.42 | Indicates a successful creation of an individual ueContext resource |  |
| UeContextCreateError | 6.1.6.2.43 | Represents an error when creating a UE context |  |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |  |
| N2InformationTransferError | 6.1.6.2.45 | Error within NonUeN2MessageTransfer response |  |
| PWSResponseData | 6.1.6.2.46 | Represents the type of PWS |  |
| PWSErrorData | 6.1.6.2.47 | Represents the type of PWS error |  |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |  |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |  |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |  |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Provides the status of UE context transfer status update at a source AMF. |  |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |  |
| N2InfoNotificationRspData | 6.1.6.2.54 | N2 information notification response data |  |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |  |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |  |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |  |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |  |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |  |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |  |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |  |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |  |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |  |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |  |
| StatusChange | 6.1.6.3.3 |  |  |
| N2InformationClass | 6.1.6.3.4 |  |  |
| N1MessageClass | 6.1.6.3.5 |  |  |
| N1N2MessageTransferCause | 6.1.6.3.6 |  |  |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |  |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |  |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |  |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |  |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |  |
| ScType | 6.1.6.3.12 | Indicates the security context type. |  |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |  |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |  |
| AMPolicyReqTrigger | 6.1.6.3.15 | AM Policy Request Triggers |  |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |  |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |  |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |  |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF, change or removal, or the V-SMF change | DTSSA |
| SbiBindingLevel | 6.1.6.3.xx | SBI Binding Level |  |

Table 6.1.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.1.6.1-2: Namf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |  |
| Arp | 3GPP TS 29.571 [6] |  |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |  |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |  |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |  |
| IndicationFlags | 3GPP TS 29.502 [16] | Indication Flags |  |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |  |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |  |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |  |
| TimeZone | 3GPP TS 29.571 [6] |  |  |
| UserLocation | 3GPP TS 29.571 [6] |  |  |
| AccessType | 3GPP TS 29.571 [6] |  |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |  |
| Uri | 3GPP TS 29.571 [6] |  |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |  |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |  |
| Uint16 | 3GPP TS 29.571 [6] |  |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |  |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |  |
| Pei | 3GPP TS 29.571 [6] |  |  |
| Dnn | 3GPP TS 29.571 [6] |  |  |
| Gpsi | 3GPP TS 29.571 [6] |  |  |
| GroupId | 3GPP TS 29.571 [6] |  |  |
| PlmnId | 3GPP TS 29.571 [6] |  |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |  |
| NsiId | 3GPP TS 29.531 [18] |  |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |  |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |  |
| Area | 3GPP TS 29.571 [6] |  |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |  |
| Ambr | 3GPP TS 29.571 [6] |  |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |  |
| DurationSec | 3GPP TS 29.571 [6] |  |  |
| StnSr | 3GPP TS 29.571 [6] | Session Transfer Number for SRVCC |  |
| CMsisdn | 3GPP TS 29.571 [6] | Correlation MSISDN |  |
| DateTime | 3GPP TS 29.571 [6] |  |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |  |
| NfSetId | 3GPP TS 29.571 [6] | NF Set ID |  |
| NfServiceSetId | 3GPP TS 29.571 [6] | NF Service Set ID |  |

NEXT CHANGE

6.1.6.2.25	Type: UeContext

Table 6.1.6.2.25-1: Definition of type UeContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present if available. When present, this IE contains SUPI of the UE. |
| supiUnauthInd | boolean | C | 0..1 | This IE shall be present if SUPI is present. When present, it shall indicate whether the SUPI is unauthenticated. |
| gpsiList | array(Gpsi) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain the GPSI(s) of the UE. |
| pei | Pei | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain Mobile Equipment Identity of the UE. |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM Group serving the UE. |
| ausfGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the AUSF Group serving the UE. |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |
| groupList | array(GroupId) | C | 1..N | This IE shall be present if the UE belongs to any subscribed internal group(s) and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall list the subscribed internal group(s) to which the UE belongs to. |
| drxParameter | DrxParameter | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain the DRX parameter of the UE. |
| subRfsp | RfspIndex | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the subscribed RFSP Index of the UE. |
| usedRfsp | RfspIndex | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the used RFSP Index of the UE. |
| subUeAmbr | Ambr | C | 0..1 | This IE shall be present if subscribed UE-AMBR has been retrieved from UDM and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall indicate the value of subscribed UE AMBR of the UE. |
| smsSupport | SmsSupport | C | 0..1 | and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. Indicates whether the UE supports SMS delivery over NAS via 3GPP access, or via non-3GPP access, or via both the 3GPP and non-3GPP access. |
| smsfId | NfInstanceId | C | 0..1 | This IE shall be present if the SMS service for UE is activated and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it indicates the identifier of the SMSF network function instance serving the UE. The NF service consumer (e.g. target AMF) may use this information to identify the SMSF NF service profile from among the SMSF NF service profiles it received from the NRF. |
| seafData | SeafData | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the security data derived from data received from AUSF of the UE. |
| 5gMmCapability | 5GMmCapability | C | 0..1 | This IE shall be present if the UE had provided this IE during Registration Procedure and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain 5G MM capability of the UE. |
| pcfId | NfInstanceId | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE indicates the identity of the PCF for AM Policy and/or UE Policy. |
| pcfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, it shall contain the NF Set ID of the PCF for AM Policy and/or UE Policy. |
| pcfAmpServiceSetId | NfServiceSetId | C | 0..1 | This shall be present, if available. When present, it shall contain the NF Service Set ID of the PCF's AM Policy service. |
| pcfUepServiceSetId | NfServiceSetId | C | 0..1 | This shall be present, if available. When present, it shall contain the NF Service Set ID of the PCF's UE Policy service. |
| pcfBindingLevel | SbiBindingLevel | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the SBI binding level of the PCF's AM policy and UE Policy association resources. |
| pcfAmPolicyUri | Uri | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall contain the URI of the individual AM policy resource (see 3GPP TS 29.507 [32] clause 5.3.3.2) used by the AMF. |
| amPolicyReqTriggerList | array(PolicyReqTrigger) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall indicate the AM policy request triggers towards the PCF. The NF Service Consumer (e.g. target AMF) shall use these triggers to request AM policy from the PCF whenever these triggers are met. The possible AM policy control request triggers are specified in clause 6.1.2.5 of 3GPP TS 23.503 [7]. |
| pcfUePolicyUri | Uri | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall contain the URI of the individual UE policy resource (see 3GPP TS 29.507 [32] clause 5.3.3.2) used by the AMF. |
| uePolicyReqTriggerList | array(PolicyReqTrigger) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall indicate the UE policy request triggers towards the PCF. The NF Service Consumer (e.g. target AMF) shall use these triggers to request UE policy from the PCF whenever these triggers are met. The possible UE policy control request triggers are specified in clause 6.1.2.5 of 3GPP TS 23.503 [31]. |
| hpcfId | NfInstanceId | O | 0..1 | This IE indicates the identity of PCF for AM Policy and/or UE Policy in home PLMN, when the UE is roaming. |
| restrictedRatList | array(RatType) | O | 1..N | When present, this IE shall indicate the list of RAT types that are restricted for the UE; see 3GPP TS 29.571 [6] |
| forbiddenAreaList | array(Area) | O | 1..N | When present, this IE shall indicate the list of forbidden areas of the UE. |
| serviceAreaRestriction | ServiceAreaRestriction | O | 0..1 | When present, this IE shall indicate Service Area Restriction for the UE. |
| restrictedCnList | array(CoreNetworkType) | O | 1..N | When present, this IE shall indicate the list of Core Network Types that are restricted for the UE. |
| eventSubscriptionList | array(AmfEventSubscription) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the event subscription(s) targeting the UE or the group the UE is part of. |
| mmContextList | array(MmContext) | C | 1..2 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the MM Contexts of the UE. |
| sessionContextList | array(PduSessionContext) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the PDU Session Contexts of the UE. |
| traceData | TraceData | C | 0..1 | This IE shall be present if signalling based trace has been activated (see 3GPP TS 32.422 [30]) and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. |
| serviceGapExpiryTime | DateTime | C | 0..1 | This IE shall be present if Service Gap Control is enabled and if the AMF has started a Service Gap Timer which has not expired yet (see clause 5.31.16 of 3GPP TS 23.501 [2]). The value of the IE shall indicate the expiry time of the active Service Gap Timer for the UE. |
| stnSr | StnSr | O | 0..1 | This IE shall be present if available, for UE supporting 5G-SRVCC (see clause 5.2.2.2.11 of 3GPP TS 23.502 [3]). When present, this IE contains STN-SR of the UE. |
| cMsisdn | CMsisdn | O | 0..1 | This IE shall be present if available, for UE supporting 5G-SRVCC (see clause 5.2.2.2.11 of 3GPP TS 23.502 [3]). When present, this IE contains C-MSISDN of the UE. |
| msClassmark2 | MSClassmark2 | O | 0..1 | This IE shall be present if available, for UE supporting 5G-SRVCC (see clause 5.2.2.2.11 of 3GPP TS 23.502 [3]). When present, this IE contains Mobile Station Classmark 2 of the UE. |
| supportedCodecList | array(SupportedCodec) | O | 1..N | This IE shall be present if available, for UE supporting 5G-SRVCC (see clause 5.2.2.2.11 of 3GPP TS 23.502 [3]). When present, this IE shall indicate the list of speech codecs supported by the UE. |
| smallDataRateStatusInfos | array(SmallDataRateStatusInfo) | O | 1..N | List of Small Data Rate Control Statuses for released PDU Sessions, see clause 5.31.14.3 of TS 23.501 [2]. |

NEXT CHANGE

6.1.6.2.37	Type: PduSessionContext

Table 6.1.6.2.37-1: Definition of type PduSessionContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pduSessionId | PduSessionId | M | 1 | Indicates the identifier of the PDU Session. |
| smContextRef | Uri | M | 1 | Indicates the URI of the SM context, including the apiRoot (see clause 6.1.3.3.2 of 3GPP TS 29.502 [16]). When present, it shall carry the URI of SM Context of: - I-SMF, for a PDU session with I-SMF; or - V-SMF, for HR PDU session; or - SMF, for non-roaming PDU session without I-SMF, or LBO roaming PDU session; |
| sNssai | Snssai | M | 1 | Indicates the associated S-NSSAI for the PDU Session. |
| dnn | Dnn | M | 1 | Indicates the Data Network Name. |
| accessType | AccessType | M | 1 | Indicates the access type of the PDU session. |
| allocatedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present when at least one EBI is allocated to the PDU session. When present, this IE shall contain the EBIs currently allocated to the PDU session. |
| hsmfId | NfInstanceId | C | 0..1 | This IE shall be present for non-roaming and home-routed PDU sessions. When present, it shall indicate the associated home SMF for the PDU Session. |
| hsmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the home SMF. |
| hsmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the home SMF's PDUSession service instance. |
| smfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present if available, for a non-roaming PDU session. When present, this IE shall contain the SBI binding level of the SMF's SM context resource. |
| vsmfId | NfInstanceId | C | 0..1 | This IE shall be present for roaming PDU sessions. When present, it shall indicate the associated visit SMF for home-routed the PDU Session, or the SMF for the local-breakout PDU session. |
| vsmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the V-SMF. |
| vsmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the V-SMF's PDUSession service instance. |
| vsmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the V-SMF's SM context resource. |
| ismfId | NfInstanceId | C | 0..1 | This IE shall be present if I-SMF is involved in the PDU session. When present, it shall indicate the associated I-SMF for the PDU Session. |
| ismfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the I-SMF. |
| ismfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the I-SMF's PDUSession service instance. |
| ismfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the SBI binding level of the I-SMF's SM Context resource. |
| nsInstance | NsiId | C | 1 | This IE shall be present if available. When present, this IE shall Indicate Network Slice Instance for the PDU Session |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session Context. This IE may be used by the AMF to identify PDU session contexts affected by a failure or restart of the SMF service instance (see clause 6.2 of 3GPP TS 23.527 [33]). |
| maPduSession | boolean | C | 0..1 | This IE shall be present if available. When present, this IE shall indicate whether it is an MA PDU session. true: indicates the PDU session is MA PDU session; false (default): the PDU session is not MA PDU session. |

NEXT CHANGE

6.1.6.3.xx	Enumeration: SbiBindingLevel

Table 6.1.6.3.xx-1: Enumeration SbiBindingLevel

| Enumeration value | Description |
| --- | --- |
| "NF_INSTANCE_BINDING" | Indicates binding to NF instance |
| "NF_SET_BINDING" | Indicates binding to NF Set |
| "NF_SERVICE_SET_BINDING" | Indicates binding to NF Service Set |

NEXT CHANGE

A.2	Namf_Communication API

********TEST SKIPPED********

UeContext:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supiUnauthInd:

type: boolean

gpsiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

ausfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

groupList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

drxParameter:

$ref: '#/components/schemas/DrxParameter'

subRfsp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndex'

usedRfsp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndex'

subUeAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

smsSupport:

$ref: '#/components/schemas/SmsSupport'

smsfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

seafData:

$ref: '#/components/schemas/SeafData'

5gMmCapability:

$ref: '#/components/schemas/5GMmCapability'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

pcfAmpServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pcfUepServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pcfBinding:

$ref: '#/components/schemas/SbiBindlingLevel'

pcfAmPolicyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

amPolicyReqTriggerList:

type: array

items:

$ref: '#/components/schemas/PolicyReqTrigger'

minItems: 1

pcfUePolicyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

uePolicyReqTriggerList:

type: array

items:

$ref: '#/components/schemas/PolicyReqTrigger'

minItems: 1

hpcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

restrictedRatList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

forbiddenAreaList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

minItems: 1

serviceAreaRestriction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

restrictedCoreNwTypeList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

minItems: 1

eventSubscriptionList:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/AmfEventSubscription'

minItems: 1

mmContextList:

type: array

items:

$ref: '#/components/schemas/MmContext'

minItems: 1

maxItems: 2

sessionContextList:

type: array

items:

$ref: '#/components/schemas/PduSessionContext'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

serviceGapExpiryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stnSr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StnSr'

cMsisdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CMsisdn'

msClassmark2:

$ref: '#/components/schemas/MSClassmark2'

supportedCodecList:

type: array

items:

$ref: '#/components/schemas/SupportedCodec'

minItems: 1

smallDataRateStatusInfos:

type: array

items:

$ref: '#/components/schemas/SmallDataRateStatusInfo'

minItems: 1

********TEST SKIPPED********

********TEST SKIPPED********

PduSessionContext:

type: object

properties:

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

smContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

allocatedEbiList:

type: array

items:

$ref: 'TS29502_Nsmf_PDUSession.yaml#/components/schemas/EbiArpMapping'

minItems: 1

hsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

hsmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

hsmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSesrviceSetId'

smfBinding:

$ref: '#/components/schemas/SbiBindlingLevel'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

vsmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

vsmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

vsmfBinding:

$ref: '#/components/schemas/SbiBindlingLevel'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ismfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

ismfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

ismfBinding:

$ref: '#/components/schemas/SbiBindlingLevel'

nsInstance:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/NsiId'

smfServiceInstanceId:

type: string

maPduSession:

type: boolean

default: false

required:

- pduSessionId

- smContextRef

- sNssai

- dnn

- accessType

********TEST SKIPPED********

********TEST SKIPPED********

#

# ENUMERATIONS

#

StatusChange:

anyOf:

- type: string

enum:

- AMF_UNAVAILABLE

- AMF_AVAILABLE

- type: string

N2InformationClass:

anyOf:

- type: string

enum:

- SM

- NRPPa

- PWS

- PWS-BCAL

- PWS-RF

- RAN

- type: string

N1MessageClass:

anyOf:

- type: string

enum:

- 5GMM

- SM

- LPP

- SMS

- UPDP

- type: string

N1N2MessageTransferCause:

anyOf:

- type: string

enum:

- ATTEMPTING_TO_REACH_UE

- N1_N2_TRANSFER_INITIATED

- WAITING_FOR_ASYNCHRONOUS_TRANSFER

- UE_NOT_RESPONDING

- N1_MSG_NOT_TRANSFERRED

- UE_NOT_REACHABLE_FOR_SESSION

- type: string

UeContextTransferStatus:

anyOf:

- type: string

enum:

- TRANSFERRED

- NOT_TRANSFERRED

- type: string

N2InformationTransferResult:

anyOf:

- type: string

enum:

- N2_INFO_TRANSFER_INITIATED

- type: string

CipheringAlgorithm:

anyOf:

- type: string

enum:

- NEA0

- NEA1

- NEA2

- NEA3

- type: string

IntegrityAlgorithm:

anyOf:

- type: string

enum:

- NIA0

- NIA1

- NIA2

- NIA3

- type: string

SmsSupport:

anyOf:

- type: string

enum:

- 3GPP

- NON_3GPP

- BOTH

- NONE

- type: string

ScType:

anyOf:

- type: string

enum:

- NATIVE

- MAPPED

- type: string

KeyAmfType:

anyOf:

- type: string

enum:

- KAMF

- KPRIMEAMF

- type: string

TransferReason:

anyOf:

- type: string

enum:

- INIT_REG

- MOBI_REG

- MOBI_REG_UE_VALIDATED

- type: string

PolicyReqTrigger:

anyOf:

- type: string

enum:

- LOCATION_CHANGE

- PRA_CHANGE

- SARI_CHANGE

- RFSP_INDEX_CHANGE

- ALLOWED_NSSAI_CHANGE

- type: string

RatSelector:

anyOf:

- type: string

enum:

- E-UTRA

- NR

- type: string

NgapIeType:

anyOf:

- type: string

enum:

- PDU_RES_SETUP_REQ

- PDU_RES_REL_CMD

- PDU_RES_MOD_REQ

- HANDOVER_CMD

- HANDOVER_REQUIRED

- HANDOVER_PREP_FAIL

- SRC_TO_TAR_CONTAINER

- TAR_TO_SRC_CONTAINER

- RAN_STATUS_TRANS_CONTAINER

- SON_CONFIG_TRANSFER

- NRPPA_PDU

- UE_RADIO_CAPABILITY

- type: string

N2InfoNotifyReason:

anyOf:

- type: string

enum:

- HANDOVER_COMPLETED

- type: string

SmfChangeIndication:

anyOf:

- type: string

enum:

- CHANGED

- REMOVED

- type: string

SbiBindingLevel:

anyOf:

- type: string

enum:

- NF_INSTANCE_BINDING

- NF_SET_BINDING

- NF_SERVICE_SET_BINDING

- type: string

********TEST SKIPPED********

END OF CHANGES