3GPP TSG-CT Meeting #86	CP-193169

# Sitges, SPAIN; 09th – 10th December 2019												rev of C1-198733

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1676 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Equivalent SNPNs not supported for stand-alone non-public networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | OPPO |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current 24.501, there is the following description: The functions and procedures of NAS described in the present document are applicable to an SNPN and an SNPN enabled UE unless indicated otherwise. And here is the description that UE should provide the GUTI when registers to the equivalent PLMN in subclause 5.5.1.2.2. c) if the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by an equivalent PLMN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE; However, the mentioned description is not applicable to SNPN since there is there is no equivalent SNPN concept. |  |  |  |  |  |  |  |  |
| Summary of change: |  | One difference between PLMN and SNPN is added as there is no equivalent SNPN is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether equivalent SNPN is supported or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev1: Move the change to general subclause 4.14.2 for SNPN based on the online agreement that only differences should be captured. rev2: The only change is to the title of the CR to reflect the reason for change. |  |  |  |  |  |  |  |  |

***** 1st change *****

### 4.14.2	Stand-alone non-public network

The functions and procedures of NAS described in the present document are applicable to an SNPN and an SNPN enabled UE unless indicated otherwise. The key differences brought by the SNPN to the NAS layer are as follows:

a)	instead of the PLMN selection process, the SNPN selection process is performed by a UE operating in SNPN access mode (see 3GPP TS 23.122 [5] for further details on the SNPN selection);

b)	a "permanently forbidden SNPNs" list and a "temporarily forbidden SNPNs" list are managed by a UE operating in SNPN access mode instead of forbidden PLMN lists;

c)	inter-system change to and from S1 mode is not supported;

d)	emergency services are not supported in SNPN access mode;

e)	with respect to the 5GMM cause values:

1)	5GMM cause values #74 "Temporarily not authorized for this SNPN" and #75 "Permanently not authorized for this SNPN" are supported whereas these 5GMM cause values cannot be used in a PLMN; and

2)	5GMM cause value #73 "Serving network not authorized" is not supported whereas this 5GMM cause value can be used in a PLMN;

Editor's note [WI: Vertical_LAN, CR#1286]:	It is FFS whether 5GMM cause values #31 "Redirection to EPC required" and # 72 "Non-3GPP access to 5GCN not allowed" are supported in an SNPN.

f)	a list of "5GS forbidden tracking areas for roaming" and a list of "5GS forbidden tracking areas for regional provision of service" are managed per SNPN (see 3GPP TS 23.122 [5]); and

x)	equivalent SNPN is not supported.

*** End of changes ***