3GPP TSG-CT3 Meeting #107	C3-195269

# Reno, USA, 11th – 15th  November 2019                                            (Revision of C3-195139)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0112 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker details update – Service Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.222 (Rel-16), clause 8.26 specifies the procedure and information flows related to update of API invoker’s API list on CAPIF core function. This procedure defines new CAPIF services that API invoker consmes to update the API list that the API invoker wishes to consume. Stage 3 aspects of this needs to be specified in TS 29.222 (Rel-16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Details of new service operations for CAPIF_API_Invoker_Management_API. - API invoker update includes API List (as per stage 2) and as per stage 3, API invoker information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Details of service definition for CAPIF_API_Invoker_Management_API not aligned with stage 2 aspects in TS 23.222. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.5.1.1, 5.5.2.1, 5.5.2.X (new), 5.5.2.X.1 (new), 5.5.2.X.2 (new), 5.5.2.Y (new), 5.5.2.Y.1 (new), 5.5.2.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact any Open API specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 5.1	Introduction of Services

The table 5.1-1 lists the CAPIF Core Function APIs below the service name. A service description subclause for each API gives a general description of the related API.

Table 5.1-1: List of CAPIF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| CAPIF_Discover_Service_API | Discover_Service_API | Request/ Response | AP Invoker |
|  | Event operations (NOTE) | (NOTE) | API Invoker |
| CAPIF_Publish_Service_API | Publish_Service_API | Request/ Response | API Publishing Function |
|  | Unpublish_Service_API | Request/ Response | API Publishing Function |
|  | Update_Service_API | Request/ Response | API Publishing Function |
|  | Get_Service_API | Request/ Response | API Publishing Function |
| CAPIF_Events_API | Subscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Notify_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Unsubscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
| CAPIF_API_Invoker_Management_API | Onboard_API_Invoker | Request/ Response | API Invoker |
|  | Offboard_API_Invoker | Request/ Response | API Invoker |
|  | Notify_Onboarding_Completion | Subscribe/Notify | API Invoker |
|  | Update_API_Invoker_Details | Request/Response | API Invoker |
|  | Notify_Update_Completion | Subscribe/Notify | API Invoker |
| CAPIF_Security_API | Obtain_Security_Method | Request/ Response | API Invoker |
|  | Obtain_Authorization | Request/ Response | API Invoker |
|  | Obtain_API_Invoker_Info | Request/ Response | API exposing function |
|  | Revoke_Authorization | Request/ Response | API exposing function |
| CAPIF_Monitoring_API | Event operations (NOTE) | (NOTE) | API Management Function |
| CAPIF_Logging_API_Invocation_API | Log_API_Invocation | Request/ Response | API exposing function |
| CAPIF_Auditing_API | Query_API_Invocation_Log | Request/ Response | API management function |
| CAPIF_Access_Control_Policy_API | Obtain_Access_Control_Policy | Request/Response | API Exposing Function |
| NOTE: The service operations of CAPIF Events API are reused by the CAPIF_Discover_Service_API, CAPIF_Publish_Service_API and CAPIF_Monitoring_API for events related services. |  |  |  |

2nd change

#### 5.5.1.1	Overview

The CAPIF API invoker management APIs, as defined in 3GPP TS 23.222 [2], allow API invokers via CAPIF-1 and CAPIF-1e reference point to on-board and off-board itself as a recognized user of the CAPIF or update the API invoker’s details on the CAPIF core function.

3rd change

#### 5.5.2.1	Introduction

The service operations defined for the CAPIF API Invoker Management API are shown in table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of the CAPIF_API_Invoker_Management_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Onboard_API_Invoker | This service operation is used by an API invoker to on-board itself as a recognized user of CAPIF | API invoker |
| Offboard_API_Invoker | This service operation is used by an API invoker to off-board itself as a recognized user of CAPIF | API invoker |
| Notify_Onboarding_Completion | This service operation is used by CAPIF core function to send an on-boarding notification to the API invoker. | CAPIF core function |
| Update_API_Invoker_Details | This service operation is used by an API invoker to update API invoker’s details in the CAPIF core function. | API Invoker |
| Notify_Update_Completion | This service operation is used by CAPIF core function to send an update notification to the API invoker | CAPIF core function |

4th change

#### 5.5.2.X	Update_API_Invoker_Details

##### 5.5.2.X.1	General

This service operation is used by an API invoker to update the API invoker’s profile details on the CAPIF core function.

##### 5.5.2.X.2	API invoker updating its details on CAPIF using Update_API_Invoker_Details service operation

To update the API invoker’s profile details on the CAPIF core function, the API invoker shall send a HTTP PUT message to the CAPIF core function to its resource representation, requesting to replace all properties in the existing resource, addressed by the URI received in the response to the request that has created the API invoker profile resource. The properties "apiInvokerId” and “onboardingInformation” shall remain unchanged from the previously provided values. The body of the HTTP PUT message shall include API invoker identity information, API invoker details that need to be updated and and a Notification Destination URI for update notification as specified in clause 8.4.2.3.3.X. API invoker details may include API invoker information and API List. Upon receiving the above described HTTP PUT message:

1.	if the CAPIF core function decides to update the API list of the API invoker without validation by CAPIF administrator, then the CAPIF core function:

a.	shall determine if the request in the HTTP PUT message by the API invoker is authorized or not.

b.   verify that the "apiInvokerId” and “onboardingInformation” properties are same as in API invoker resource on CAPIF core function.

c.	if the API invoker's request is authorized and the properties "apiInvokerId” and “onboardingInformation” match, the CAPIF core function shall:

i.	if the request contains an API list:

-	create a list of APIs the API invoker is allowed to access; and

-	update the resource identified by the CAPIF Resource URI of the API invoker’s HTTP PUT request with the updated information in the request and the API list created in step A;

ii.	if the request does not contain an API list, update the resource identified by the CAPIF Resource URI of the API invoker’s HTTP PUT request with the updated information in the request; and

iii.	return the updated API invoker details;

2.	otherwise, the CAPIF core function:

a.	shall acknowledge the receipt of the update API invoker details request to the API invoker.

b.   verify that the "apiInvokerId” and “onboardingInformation” properties are same as in API invoker resource on CAPIF core function;

c.	if the properties "apiInvokerId” and “onboardingInformation” match, then shall request the CAPIF administrator to validate the request or the API management to validate the request by sharing the API invoker identity information and the updated information received in the HTTP PUT message;

d.	on receiving confirmation of successful validation of the request from the CAPIF administrator or the API management, the CAPIF core function shall:

i.	update the resource identified by the CAPIF Resource URI of the API invoker’s HTTP PUT request, with validated information; and

ii.	return the updated API invoker details.

NOTE 1:	How the CAPIF core function determines that the CAPIF core function can process the request and update the API list of the API invoker automatically is out-of-scope of this specification.

NOTE 2:	Interactions between the CAPIF core function and the CAPIF administrator or the API management is out-of-scope of this specification.

NOTE 3:	After the operation is completed the API Invoker no longer needs to maintain the Notification Destination URI and may delete it.

5th change

#### 5.5.2.Y	Notify_Update_Completion

##### 5.5.2.Y.1	General

This service operation is used by the CAPIF core function to send a notification about the completion of the update of API invoker’s details.

##### 5.5.2.Y.2	Notifying API invoker update completion using Notify_Update_Completion service operation

When the CAPIF core function cannot immediately grant the update request (see subclause 5.5.2.X.2) it will send a response acknowledging the request and begin processing it. After completion, the CAPIF core function shall send an HTTP POST message using the Notification Destination URI received in the update details request. The body of the HTTP POST message shall include the updated API Invoker details.

Upon receiving the HTTP POST message, the API invoker shall process the message in the same manner it would have processed an immediate response to the update the details of the API invoker request, and respond to the HTTP POST message with HTTP response 204 No content.

End of changes