3GPP TSG-CT3 Meeting #107	C3-195271

# Reno, USA, 11th – 15th  November 2019                                              (Revsion of C3-195141)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0114 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API Provider Registration and Update – Service Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.222 (Rel-16), clauses 8.28 and 8.29 specify the procedure and information flows related to registration and update of API provider domain functions on CAPIF core function. These procedures define new CAPIF services that API management function consmes to register and update the registration of API provider domain functions. Stage 3 aspects of this needs to be specified in TS 29.222 (Rel-16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introducing new CAPIF API (CAPIF_API_Provider_Management_API) and its service operations. - Service definition for CAPIF_API_Provider_Management_API API, defining all the service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 details of CAPIF_API_Provider_Management_API not specified as per stage 2 aspects in TS 23.222. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.X (new), 5.X.1 (new), 5.X.1.1 (new), 5.X.2 (new), 5.X.2.1 (new), 5.X.2.2 (new), 5.X.2.2.1 (new), 5.X.2.2.2 (new), 5.X.2.3 (new), 5.X.2.3.1 (new), 5.X.2.3.2 (new), 5.X.2.4 (new), 5.X.2.4.1 (new), 5.X.2.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact any Open API specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 5.1	Introduction of Services

The table 5.1-1 lists the CAPIF Core Function APIs below the service name. A service description subclause for each API gives a general description of the related API.

Table 5.1-1: List of CAPIF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| CAPIF_Discover_Service_API | Discover_Service_API | Request/ Response | AP Invoker |
|  | Event operations (NOTE) | (NOTE) | API Invoker |
| CAPIF_Publish_Service_API | Publish_Service_API | Request/ Response | API Publishing Function |
|  | Unpublish_Service_API | Request/ Response | API Publishing Function |
|  | Update_Service_API | Request/ Response | API Publishing Function |
|  | Get_Service_API | Request/ Response | API Publishing Function |
| CAPIF_Events_API | Subscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Notify_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Unsubscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
| CAPIF_API_Invoker_Management_API | Onboard_API_Invoker | Request/ Response | API Invoker |
|  | Offboard_API_Invoker | Request/ Response | API Invoker |
|  | Notify_Onboarding_Completion | Subscribe/Notify | API Invoker |
| CAPIF_Security_API | Obtain_Security_Method | Request/ Response | API Invoker |
|  | Obtain_Authorization | Request/ Response | API Invoker |
|  | Obtain_API_Invoker_Info | Request/ Response | API exposing function |
|  | Revoke_Authorization | Request/ Response | API exposing function |
| CAPIF_Monitoring_API | Event operations (NOTE) | (NOTE) | API Management Function |
| CAPIF_Logging_API_Invocation_API | Log_API_Invocation | Request/ Response | API exposing function |
| CAPIF_Auditing_API | Query_API_Invocation_Log | Request/ Response | API management function |
| CAPIF_Access_Control_Policy_API | Obtain_Access_Control_Policy | Request/Response | API Exposing Function |
| CAPIF_API_Provider_Management_API | Register_API_Provider | Request/Response | API Management Function |
|  | Update_API_Provider | Request/Response | API Management Function |
|  | Deregister_API_Provider | Request/Response | API Management Function |
| NOTE: The service operations of CAPIF Events API are reused by the CAPIF_Discover_Service_API, CAPIF_Publish_Service_API and CAPIF_Monitoring_API for events related services. |  |  |  |

2nd change

## 5.X	CAPIF_API_Provider_Management_API

### 5.X.1	Service Description

#### 5.X.1.1	Overview

The CAPIF API provider management APIs, as defined in 3GPP TS 23.222 [2], allow API management functions via CAPIF-5 and CAPIF-5e reference points to register, deregister and update registration information of API provider domain functions (API Exposing Function, API Publishing Function, API management Function) as a recognized API provider domain of the CAPIF domain.

### 5.X.2	Service Operations

#### 5.X.2.1	Introduction

The service operations defined for the CAPIF API Provider Management API are shown in table 5.X.2.1-1.

Table 5.X.2.1-1: Operations of the CAPIF_API_Invoker_Management_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Register_API_Provider | This service operation is used by an API management function to register API provider domain functions as a recognized API provider domain of the CAPIF domain. | API Management Function |
| Update_API_Provider | This service operation is used by an API management function to update the API provider domain functions details in the CAPIF domain. | API Management Function |
| Deregister_API_Provider | This service operation is used by an API management function to deregister API provider domain functions as a recognized API provider domain of the CAPIF domain. | API Management Function |

#### 5.X.2.2	Register_API_Provider

##### 5.X.2.2.1	General

This service operation is used by an API management function to register API provider domain functions as a recognized API provider of CAPIF domain.

##### 5.X.2.2.2	API provider domain functions registering as a recognized API provider domain function of CAPIF using Register_API_Provider service operation

To register API provider domain as a recognized API provider of the CAPIF, the API management function shall send a HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include API provider Enrolment Details, consisting of details of all API provider domain functions and security information for CAPIF core function to validate the registration request.

Upon receiving the above described HTTP POST message, the CAPIF core function validates the security information and determine if the request sent by API management function is authorized or not. If the API management function is authorized, CAPIF core function shall:

a.	create the API provider domain profile consisting of API provider domain ID, API provider domain functions profiles as per the request. CAPIF core function shall assign the identities for the API provider domain functions.;

b.   create a new resource as defined in clause 8.X.2.2.3.1; and

c.   return the API provider domain profile, the CAPIF Resource URI in the response message and registration failure information specific to individual API provider domain functions.

#### 5.X.2.3	Update_API_Provider

##### 5.X.2.3.1	General

This service operation is used by an API management function to update API provider domain function details on the CAPIF domain.

##### 5.X.2.3.2	API management function updating API provider domain function details on CAPIF using Update_API_Provider service operation

To update the API provider domain profile and its individual functions details on CAPIF domain, the API management function shall send a HTTP PUT message to its resource representation in the CAPIF core function as specified in clause 8.X.2.3.3.1, requesting to replace all properties in the existing resource, addressed by the URI received in the response to the request that has created the API provider domain profile resource. The property “apiProviderDomainId”, shall remain unchanged from the previously provided values. The body of the HTTP PUT message shall include the API provider domain profile that need to be updated. Upon receiving the described HTTP PUT message:

1. the CAPIF core function shall process the updates received in the HTTP PUT message and determine if the request sent by API management function is authorized or not;

2. verify that the “apiProviderDomainId” property is same as in the API provider domain resource on CAPIF Core Function.

3.  if the API management function is authorized and the property “apiProviderDomainId” matches, then the CAPIF core function shall

a. replace the representation of the resource identified by the CAPIF Resource URI of the API management function’s HTTP PUT request with updated information in the request.

b. update the individual API provider domain function profiles as per the request. CAPIF core function shall create new API provider domain function profiles along with assignment of identities, if the API provider domain functions profiles in the request do not exist in CAPIF.

c. return the updated API provider domain profile resource and update failure information specific to individual API provider domain functions.

#### 5.X.2.4	Deregister_API_Provider

##### 5.X.2.4.1	General

This service operation is used by an API management function to deregister the API provider domain function as a recognized API provider of the CAPIF domain.

##### 5.X.2.4.2	API provider domain functions deregistering as a recognized API provider domain function of CAPIF using Deregister_API_Provider service operation

To deregister API provider domain as a recognized API provider of the CAPIF domain, the API management function shall send an HTTP DELETE message to its resource representation in the CAPIF core function as specified in clause 8.X.2.3.3.2.

Upon receiving the HTTP DELETE message, the CAPIF core function shall:

1.	determine if the request sent by the API management functions is authorized or not;

2.	if the API management function’s request is authorized, the CAPIF core function shall:

a.	delete the resource representation pointed by the CAPIF Resource Identifier; and

b.   delete the related API provider domain profile.

End of changes