3GPP TSG-CT3 Meeting #107	C3-195396

# Reno, USA, 11th – 15th  November 2019                (Revision of C3-195371, was C3-195272, was C3-195142)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0115 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API Provider Registration and Update – API Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.222 (Rel-16), clauses 8.28 and 8.29 specify the procedure and information flows related to registration and update of API provider domain functions on CAPIF core function. These procedures define new CAPIF services that API management function consmes to register and update the registration of API provider domain functions. Definition of API provider mangement API needs to be specified in TS 29.222 (Rel-16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Definition of CAPIF API (CAPIF_API_Provider_Management_API). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | API definition of CAPIF_API_Provider_Management_API is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following new clauses added: 8.X, 8.X.1, 8.X.2, 8.X.2.1, 8.X.2.2, 8.X.2.2.1, 8.X.2.2.2, 8.X.2.2.3, 8.X.2.2.3.1, 8.X.2.2.4, 8.X.2.3, 8.X.2.3.1, 8.X.2.3.2, 8.X.2.3.3, 8.X.2.3.3.1, 8.X.2.3.3.2, 8.X.2.3.4,8.X.3, 8.X.4, 8.X.4.1, 8.X.4.2, 8.X.4.2.1, 8.X.4.2.2, 8.X.4.2.3, 8.X.4.2.4, 8.X.4.3, 8.X.4.3.1, 8.X.4.3.2, 8.X.4.3.3, 8.X.5, 8.X.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact any Open API specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - C3-195272 was wrong document uploaded. Hence, new tdoc C3-195371 was allocated, but the revision of CR 0115 is 1. |  |  |  |  |  |  |  |  |

1st change

## 8.X	CAPIF_API_Provider_Management_API

### 8.X.1	API URI

The request URI used in each HTTP request from the API management function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "api-provider-management".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in clause 8.X.2.

### 8.X.2	Resources

#### 8.X.2.1	Overview

Figure 8.X.2.1-1: Resource URI structure of the CAPIF_API_Provider_Management_API

Table 8.X.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.X.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| All API Provider Domains Registrations | {apiRoot} /api-provider-management/v1 /registrations | POST | Registers new API provider domain by creating API provider domain with API provider domain functions profiles. |
| Individual API Provider Domain Registration | {apiRoot} /api-provider-management/v1 /registrations/{registrationId} | PUT | Updates an individual API provider domain identified by {registrationId} |
|  |  | DELETE | Deregisters API provider domain by deleting API provider domain and functions, identified by {registrationId}. |

#### 8.X.2.2	Resource: All API Provider Domains Registrations

##### 8.X.2.2.1	Description

The All API provider domains registrations resource represents all the API provider domains that are registered at a given CAPIF core function.

##### 8.X.2.2.2	Resource Definition

Resource URI: {apiRoot}/api-provider-management/v1/registrations

This resource shall support the resource URI variables defined in table 8.X.2.2.2-1.

Table 8.X.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |

##### 8.X.2.2.3	Resource Standard Methods

###### 8.X.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.X.2.2.3.1-1.

Table 8.X.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.X.2.2.3.1-2 and the response data structures and response codes specified in table 8.X.2.2.3.1-3.

Table 8.X.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | Enrolment details of the API provider domain including individual API provider domain function details. |

Table 8.X.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | 201 Created | API provider domain registered successfully The URI of the created resource shall be returned in the "Location" HTTP header. The list of successfully registered individual API provider domain functions, registration specific failure information of failed API provider domain function registrations, are included in APIProviderEnrolmentDetails which is provided in the response body. |

##### 8.X.2.2.4	Resource Custom Operations

None.

#### 8.X.2.3	Resource: Individual API Provider Domain Registration

##### 8.X.2.3.1	Description

The Individual API Provide Domain Registration resource represents an individual API provider domain that is registered at a given CAPIF core function.

##### 8.X.2.3.2	Resource Definition

Resource URI: {apiRoot}/api-provider-management/v1/registrations/{registrationId}

This resource shall support the resource URI variables defined in table 8.4.2.3.2-1.

Table 8.X.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 7.5 |
| registrationId | String identifying an individual registered API Provider domain resource |

##### 8.X.2.3.3	Resource Standard Methods

###### 8.X.2.3.3.1	PUT

The PUT method allows updating the registered API provider domain’s detail. The properties "apiProviderDomainId”, and “supportedFeatures” shall remain unchanged from previously provided values. This method shall support the URI query parameters specified in table 8.X.2.3.3.1-1.

Table 8.X.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in the table 8.X.2.3.3.1-2 and the response data structures and response codes specified in the table 8.X.2.3.3.1-3.

Table 8.X.2.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | Updated details of the API provider domain. |

Table 8.X.2.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | 200 OK | API provider domain’s information updated successfully. Updated details of the API provider domain is part of the APIProviderEnrolmentDetails, which is provided in the response body. The list of successfully updated individual API provider domain functions, registration update specific failure information of failed API provider domain function registration updates, are included in APIProviderEnrolmentDetails which is provided in the response body. |

###### 8.X.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 8.X.2.3.3.2-1.

Table 8.X.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the response codes specified in table 8.X.2.3.3.2-2 and the response data structures and response codes specified in table 8.X.2.3.3.2-3.

Table 8.X.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.X.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual registered API provider domain matching the registrationId is deleted. All the individual API provider domain functions of the API provider domain are deleted. |

##### 8.X.2.3.4	Resource Custom Operations

None.

### 8.X.3	Notifications

None.

### 8.X.4	Data Model

#### 8.X.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 8.X.4.1-1 specifies the data types defined specifically for the CAPIF_API_Provider_Management_API service.

Table 8.X.4.1-1: CAPIF_API_Provider_Management_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| APIProviderEnrolmentDetails | 8.X.4.2.2 | API provider domain’s enrolment details. |  |
| APIProviderFunctionDetails | 8.X.4.2.3 | API Provider domain function’s details. |  |
| RegistrationInformation | 8.X.4.2.4 | Registration information of the individual API provider domain function. |  |

Table 8.X.4.1-2 specifies data types re-used by the CAPIF_API_Provider_Management_API service.

Table 8.X.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [19] | Used to negotiate the applicability of optional features defined in table 8.4.6-1. |  |

#### 8.X.4.2	Structured data types

##### 8.X.4.2.1	Introduction

##### 8.X.4.2.2	Type: APIProviderEnrolmentDetails

Table 8.X.4.2.2-1: Definition of type APIProviderEnrolmentDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiProviderDomainId | string | O | 0..1 | API provider domain ID assigned by the CAPIF core function to the API management function while registering the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses. |  |
| registrationSecret | string | M | 1 | Security information necessary for the CAPIF core function to validate the registration of the API provider domain. Shall be present in HTTP POST request from API management function to CAPIF core function for API provider domain registration. |  |
| apiProviderFunctionsList | array(APIProviderFunctionDetails) | O | 1..N | A list of individual API provider domain functions details. When included by the API management function in the HTTP request message, it lists the API provider domain functions that the API management function intends to register/update in registration or update registration procedure. When included by the CAPIF core function in the HTTP response message, it lists the API domain functions details that are registered or updated successfully. |  |
| apiProviderDomainInformation | string | O | 0..1 | Generic information related to the API provider domain such as details of the API provider applications. |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in subclause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |
| failureReason | string | O | 0..1 | Registration or update specific failure information of failed API provider domain function registrations. Shall be present in the HTTP response body if atleast one of the API provider domain function registration or update registration fails. |  |

##### 8.X.4.2.3	Type: APIProviderFunctionDetails

Table 8.X.4.2.3-1: Definition of type APIProviderFunctionDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiProviderFuncId | string | O | 0..1 | API provider domain functionID assigned by the CAPIF core function to the API provider domain function while registering/updating the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to register itself. Shall be present in all other HTTP requests and responses. |  |
| registraionInformation | RegistrationInformation | M | 1 | Information necessary for the CAPIF core function to register the API provider domain function. This information shall be present in HTTP POST/PUT request from API management function to CAPIF core function for API provider domain registration. In the HTTP response message from CAPIF core function, shall include the updated registration information for API provider domain function. |  |
| apiProviderFuncRole | ApiProviderFuncRole | M | 1 | Role of API provider domain function. The role shall be present in the HTTP POST/PUT request that the API management function intends to register/update the API provider domain function as. CAPIF core function shall register the role of API provider domain function as per the request. |  |
| apiProviderFunctionInformation | string | O | 0..1 | Generic information related to the API provider domain function such as details of the API provider applications. |  |

##### 8.X.4.2.4	Type: RegistrationInformation

Table 8.X.4.2.4-1: Definition of type RegistrationInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiProviderPublicKey | string | M | 1 | Public Key of API Provider domain function. |  |
| apiProviderCertificate | string | O | 0..1 | API provider domain function’s generic client certificate |  |

#### 8.X.4.3	Simple data types and enumerations

##### 8.X.4.3.1	Introduction

This subclause defines simple data types and enumerations that will be referenced from data structures defined in the previous subclauses.

##### 8.X.4.3.2	Simple data types

The simple data types defined in table 8.X.4.3.2-1 shall be supported.

Table 8.X.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 8.X.4.3.3	Enumeration: ApiProviderFuncRole

Table 8.X.4.3.3-1: Enumeration ApiProviderFuncRole

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AEF | API provider function is API Exposing Function. |  |
| APF | API provider function is API Publishing Function. |  |
| AMF | API provider function is API Management Function. |  |

### 8.X.5	Error Handling

General error responses are defined in clause 7.7 of this specification.

### 8.X.6	Feature negotiation

General feature negotiation procedures are defined in clause 7.8 of this specification.

End of changes